3GPP TSG RAN WG1 #108-e			    R1-2205185

e-Meeting, February 21st – March 3rd, 2022

Agenda item:	8.8.2

Source: 	Moderator (Qualcomm)

Title: 	FL summary #1 of maintenance on PUCCH enhancements

Document for:	Discussion/Decision

# Introduction

In RAN1 109e preparation phase, the following issues are identified in R1-2203651, “Summary of preparation phase for Rel-17 NR coverage enhancements”.

| Issues | Related contributions | Initial assessment |
| --- | --- | --- |
| Issue#1: Correction/Clarification on frequency hopping for DMRS bundling for PUCCH and PUSCH. | R1-2204991, R1-2204872, R1-2203096, R1-2203522, R1-2204776, R1-2203440, R1-2203193 | [High priority] Suggest to be discussed in RAN1#109-e. |
| Issue#2: Clarification on inter-slot frequency hopping for Msg3 when DMRS bundling is enabled. | R1-2204513 | [Low priority] Suggest to be discussed in RAN1#109-e. |
| Issue#3: Clarification on Rel-17 inter-slot frequency hopping when frequency hopping interval is configured but DMRS bundling is not enabled. | R1-2204872, R1-2204873 | [Low priority] Suggest to be discussed in RAN1#109-e. |
| Issue#4: Clarification on the interrelationship between the length of hopping interval and that of nominal TDW. | R1-2204549 | Suggest NOT to be discussed in RAN1#109-e. Based on the agreement in RAN1 #107, the hopping interval and nominal TDW are separately configured, while the nominal TDW was determined followed by the determination of hopping interval. It seems this issue is clear. |
| Issue#5: Clarification on dynamic PUCCH repetition factor indication for HARQ-ACK of first SPS PDSCH associated with the activation DCI and SPS release DCI. | R1-2203309, R1-2204957, R1-2203193 | [Medium priority] Suggest to be discussed in RAN1#109-e. |

According to Chairman’s guidance, as listed below, it was agreed to discuss issue #1, #2, #3 and #5 in RAN1 109e.

[109-e-R17_CovEnh-03] Email discussion under 8.8.2 for maintenance on PUCCH enhancements for proposals in the FL summary of the preparation phase discussion, including Issue#1, Issue#2, Issue#3 and Issue# 5

In the reminder of this summary, the issues are addressed in following sections.

# Issue #1: frequency hopping for DMRS bundling for PUCCH/PUSCH

Regarding the inter-slot hopping for PUSCH with DMRS bundling enabled, based on companies input, there are three different options on the details of the RB offset determination for each hop, as listed below.

- Option 1: using physical slot index only

- Option 2: using SFN and physical slot index

- Option 3: using relative SFN and physical slot index.

Specifically, with option 1 where RB offset is determined based on physical slot index only, the following TP is proposed

### 6.3.1 Frequency hopping for PUSCH repetition Type A and for TB processing over multiple slots < Unchanged text omitted > In case of inter-slot frequency hopping and when PUSCH-DMRS-Bundling is enabled, the starting RB during slot is given by: [ RBstartns,fμ=RBstart RBstart+RBoffsetmodNBWPsize ns,fμ+nf∙Nslotframe,μNFH mod 2=0ns,fμ+nf∙Nslotframe,μNFH mod 2=1 ] where ns,fμ is the current slot number within a system radio frame, nf is the number of the system radio frame containing the current slot, Nslotframe,μ is the number of slots per frame for subcarrier spacing configuration μof the UL BWP that the PUSCH is transmitted on, NFH is the value of the higher layer parameter PUSCH-Frequencyhopping-Interval, is the starting RB within the UL BWP, as calculated from the resource block assignment information of resource allocation type 1 (described in Clause 6.1.2.2.2) andis the frequency offset in RBs between the two frequency hops. In case of inter-slot frequency hopping and when PUSCH-DMRS-Bundling is enabled, the starting RB during slot nsμ is given by: RBstartnsμ=RBstart RBstart+RBoffsetmodNBWPsize nsμNFH mod 2=0nsμNFH mod 2=1 where nsμ is the current slot number within a system radio frame, NFH is the value of the higher layer parameter PUSCH-Frequencyhopping-Interval, RBstart is the starting RB within the UL BWP, as calculated from the resource block assignment information of resource allocation type 1 (described in Clause 6.1.2.2.2) and RBstart is the frequency offset in RBs between the two frequency hops. < Unchanged text omitted >

With option 2 where RB offset is determined based on physical slot index and SFN, the following TP is proposed.

### 6.3.1 Frequency hopping for PUSCH repetition Type A and for TB processing over multiple slots < Unchanged text omitted > In case of inter-slot frequency hopping and when PUSCH-DMRS-Bundling is enabled, the starting RB during slot is given by: [RBstartns,fμ=RBstart RBstart+RBoffsetmodNBWPsize ns,fμ+nf∙Nslotframe,μNFH mod 2=0ns,fμ+nf∙Nslotframe,μNFH mod 2=1] where ns,fμ is the current slot number within a system radio frame, nf is the number of the system radio frame containing the current slot, Nslotframe,μ is the number of slots per frame for subcarrier spacing configuration μof the UL BWP that the PUSCH is transmitted on, NFH is the value of the higher layer parameter PUSCH-Frequencyhopping-Interval, is the starting RB within the UL BWP, as calculated from the resource block assignment information of resource allocation type 1 (described in Clause 6.1.2.2.2) andis the frequency offset in RBs between the two frequency hops. < Unchanged text omitted >

With option 3 where RB offset is determined based on physical slot index and relative SFN, the following TP is proposed.

### 6.3.1 Frequency hopping for PUSCH repetition Type A and for TB processing over multiple slots < Unchanged text omitted > In case of inter-slot frequency hopping and when PUSCH-DMRS-Bundling is enabled, the starting RB during slot nsμ is given by: [ RBstartns,fμ=RBstart RBstart+RBoffsetmodNBWPsize ns,fμ+nf∙Nslotframe,μNFH mod 2=0ns,fμ+nf∙Nslotframe,μNFH mod 2=1 ] where ns,fμ is the current slot number within a system radio frame, nf is the number of the system radio frame containing the current slot, has value 0 for the system radio frame containing the first slot determined for the PUSCH transmission, and is increased by 1 for each subsequent system radio frame, Nslotframe,μ is the number of slots per frame for subcarrier spacing configuration μ of the UL BWP that the PUSCH is transmitted on, NFH is the value of the higher layer parameter PUSCH-Frequencyhopping-Interval, if provided; otherwise, NFH is the value of PUSCH-TimeDomainWindowLength, RBstart is the starting RB within the UL BWP, as calculated from the resource block assignment information of resource allocation type 1 (described in Clause 6.1.2.2.2) and RBoffsetis the frequency offset in RBs between the two frequency hops. < Unchanged text omitted >

In R1-2204776, the following example is provided to compare the pros and cons of the three options.

“Figure 1 illustrates comparison of these options for inter-slot frequency hopping pattern. In the examples, it is assumed FDD system with 15kHz SCS, frequency hopping interval of 4 slots and 16 PUSCH repetitions, where the repetitions span within SFN = 1 and 2. Further, the starting physical slot for repetition is slot 0 in SFN 1.

From the figure, it can be observed that Option 1 using physical slot only for the determination of hopping pattern may lead to unbalanced frequency hopping pattern when repetitions cross the frame boundary. Similarly, for Option 2, unbalanced frequency hopping pattern may be observed at the beginning and end of the PUSCH repetitions. On the contrary, for Option 3, balanced frequency hopping pattern can be achieved when relative SFN and physical slot index are employed for the determination of hopping pattern.”

Figure 1. Frequency hopping pattern determination

FL would like to collect companies’ views on the above three options, before making a recommendation to take one option. Please provide answers to FL questions in the table below.

FL question 1: which one of the above three options should be adopted and why it should be adopted? What are your views on the pros and cons of each option?

| Company name | Prefer which option, and why? | Pros and cons of each option |  |
| --- | --- | --- | --- |
| vivo | Option 1 | Pros: there’s no need to consider the SFN to determine the hop, which is consistent with legacy mechanism for hop determination. Cons: No hopping will be performed when NFH>Nslotframe,μ, which depends on NW configuration and should be fine. |  |
| Panasonic | Prefer Option 2 Option 2 have more chance to perform inter-slot frequency hopping especially for TDD. In our view, Option 3 violates the previous agreement that inter-slot frequency hopping pattern for PUSCH repetitions with DMRS bundling is determined based on physical slot index. | Option 1: - Pros: No involvement of SFN to determine the hopping pattern. - Cons: Unbalanced hopping pattern when repetition cross the SFN boundary. Option 2: - Pros: Balanced hopping pattern when repetition cross the SFN boundary, especially for TDD - Cons: Unbalanced hopping pattern when the number of repetitions is larger than Nslotframe,μ. |  |
| Apple | Opt1 (based on what previously agreed Opt3 seems to be out) |  |  |
| Nokia/NSB | Option 2. We don’t think the illustration in Figure 1 above is correct for Option 2. | Option 2 should provide a more flexible and balanced FH pattern since all slots from all frames can be considered together for planning FH pattern. |  |
| Intel | Option 3. | As illustrated in the figure, it is very clear that Option 1 and 2 result in unbalanced frequency hopping pattern, which is not desirable in terms of performance with DMRS bundling. Option 3 provides the balanced hopping pattern, which should be considered. We do not think Option 3 violates the agreement, which only stated the physical slot index is for hopping pattern determination. Agreement Inter-slot frequency hopping pattern for PUSCH repetitions with DMRS bundling is determined based on physical slot index To Nokia, figure for Option 2 is when the starting physical slot for repetition is slot 0 in SFN 1. |  |
| QC | Option 1 | The other two options requiring over-turning a previous agreement: Agreement Inter-slot frequency hopping pattern for PUSCH repetitions with DMRS bundling is determined based on physical slot index Unless the spec is completely broken, we don’t see a strong need to revise this in the maintenance phase. It’s not a good precedent. |  |
| Samsung | Option 1 | It seems to be sufficient considering the existing agreements. |  |
| InterDigital | Option 2 | Option 1 would indeed generate imbalanced patterns in average (over all possible values of K). Option 2 seems simpler than Option 3, and it is not clear that Option 3 would generate more balanced patterns in average. |  |
| Sharp | Option 2 | Option 1 has the drawback that hopping gain cannot be fully obtained, because of the unbalancing between hop1 and hop2 when the repetitions across frame boundaries. Option 3 would lead to some restriction on UE scheduling, because the hopping pattern differs depending on the starting slot of the PUSCH repetitions. Option 2 does not have the above performance degradation or the scheduling restriction. |  |
| Ericsson | Option 2 | Agree with other companies that option 3 violates the agreement. We also note that the figures above do not reflect the benefit of Options 1 & 2 vs. 3 that the hopping pattern is common over a cell, and that Options 1 and 2 follow Rel-15 principles. Option 1 does avoid using the SFN, but the current spec supports Option 2 and is in our view not excessively complex. Option 2 allows a more balanced hopping pattern, and so some potential for better performance than Option 1. |  |
| CATT | Option 2 | For Option 1, in addition to the unbalanced frequency hopping pattern, another drawback is that no hopping can be actually achieved if the hopping interval NFH is configured larger than 10 slots in FDD with 15kHz SCS. For Option 2, frequency hopping pattern is relatively balanced without the issue in Option 1. A minor modification is also suggested in the TP for Option 2 to align with the subsequent specification as following: In case of inter-slot frequency hopping and when PUSCH-DMRS-Bundling is enabled, the starting RB during slot ns,fμ is given by… where ns,fμ is the current slot number within a system radio frame,. Don’t see the need to pursue Option 3 which is a variant of relative slot counting in our view. |
| NTT DOCOMO | Option 2 | Option 3 provides balanced hopping pattern, on the other hands, Option 3 does not align with the previous agreement. |  |
| ZTE | Prefer Option 3 which provides a good balance between UE multiplexing and balanced hopping pattern. | Option 1: - Better UE multiplexing - Cons: No FH when NFH>Nslotframe,μ or unbalanced hopping pattern when NFH>Nslotframe,μ/2. Option 2: - Pros: Better UE multiplexing - Cons: Unbalanced hopping pattern may happen when the PUSCH repetitions start from SFN index larger than 1. Option 3: - Pros: Balanced hopping pattern - Cons: May impact UE multiplexing when repetitions across the SFN boundary in some cases which may not be very typical. |  |
| CMCC | Option 1 and open to option 2 | Option 2 and 3 are conflict with the previous agreement: Agreement Inter-slot frequency hopping pattern for PUSCH repetitions with DMRS bundling is determined based on physical slot index Although option 1 cannot solve this issue, but it is aligned with previous agreement. We are open to option 2. But option 2 requires a not trivial spec updates. |  |
| CTC | Option 2 | Option 2 provides a more balanced FH pattern than Option 1. |  |
| Huawei, HiSilicon | Option 2 |  | As discussed in our contribution: Physical slot indexing has been agreed for PUSCH. Option 3 is relative indexing and thus not inline with the agreement. Option 1 can never support a hop in length of NFH=12, 14, 16, and, 20 if NFH is larger than the number of slots per frame Nslotframe,μ. Additionally, it has issues when a frequency hop is across the boundary between two adjacent system radio frames, especially in the case of that Nslotframe,μ is not divisible by NFH, such as Nslotframe,μ=10 and NFH=4, 6, 8, 12, 14, 16, and, 20. Option 2 has issues when a frequency hop is across the boundary between system radio frame with nf=1023 and system radio frame with nf=0, where SFN is counted from 0 to 1023. Option is not perfect but better than Option 1 and 3. |  |
| OPPO | Option 2 |  | Option 1: - Pros: No need to consider the SFN to determine the hop. - Cons: Unbalanced hopping pattern when repetitions across frame boundaries. Option 2: - Pros: More balanced hopping pattern (vs option1) when repetitions across frame boundaries, especially for TDD. - Cons: Unbalanced hopping pattern in same case. Option 3: - Pros: More balanced hopping pattern (vs option2). - Cons: May conflict with the previous agreement. In our view, the performance of option 2 is close to that of option 3. |  |

## 2nd round discussion

Based on companies input, the following is summarized.

- Option 1: using physical slot index only

  - Pros: No need to consider the SFN to determine the hop.

  - Cons: Unbalanced hopping pattern whenever repetitions across frame boundaries.

  - Supported by (5): VIVO, Apple, QC, Samsung, CMCC

- Option 2: using SFN and physical slot index

  - Pros: More balanced hopping pattern (vs option1) when repetitions across frame boundaries, especially for TDD.

  - Cons: Unbalanced hopping pattern at the boundary of SFN 1023 and 0.

  - Supported by (10): Panasonic, Nokia/NSB, Interdigital, Sharp, Ericsson, CATT, DOCOMO, CTC, Huawei/HiSi, OPPO

- Option 3: using relative SFN and physical slot index.

  - Pros: a few companies claim option 3 has more balanced hopping pattern (vs option2) in some scenarios, while others claim option 2 and 3 has similar performance consider all scenarios.

  - Cons: Conflict with the previous agreement. It is unfriendly to UE multiplexing as well.

  - Supported by: Intel, ZTE

It is clear that option 3 has only minority support. Its benefit is not agreed by all companies. Furthermore, many companies think it violates a previous agreement. Based on these reasons, FL suggest to remove option 3 and focus on option 1 and 2 for further discussion.

It seems obvious that option 2 can have better performance at SFB boundary than option 1. The main problem of option 2, claimed by several companies is that option 2 violate a previous agreement.

Agreement

Inter-slot frequency hopping pattern for PUSCH repetitions with DMRS bundling is determined based on physical slot index

According to the slot definition in TS38.211, it is FL’s understanding that, strictly speaking, physical slot index is defined within a frame. But FL would like to have a round of discussion on this to make sure we are on the same page.

“For subcarrier spacing configuration , slots are numbered nsμ∈0, …,Nslotsubframe,μ-1  in increasing order within a subframe and ns,fμ∈0, …,Nslotframe,μ-1  in increasing order within a frame.”

FL question 2: In your opinion, does option 2 overturns the following agreement?

Agreement

Inter-slot frequency hopping pattern for PUSCH repetitions with DMRS bundling is determined based on physical slot index

FL question 3: If the answer is YES to question 2, can you accept option 2 given its performance benefit?

| Company name | Answer to FL question 2 | Answer to FL question 3 |
| --- | --- | --- |
| CATT | No | Numbering of SFN itself is also physical. We do not think Option 2 (using SFN) violates the previous agreement, whose critical point is ‘numbering the slot in physical manner’ rather than ‘relative manner’. |
| China Telecom | No | Share the similar view with CATT. |
| Panasonic | No | We share the same view as CATT. |
| vivo | Yes | Not necessary. We do not see obvious performance gain that can be achieved via different FH patterns compared to legacy FH. When frequency hopping is more important, e.g. for large delay spread, JCE should be disabled and legacy FH can be enabled instead in our view. |
| InterDigital | No | Same view as CATT. |
| Intel | Yes | If Option 3 violates the previous agreement, we strongly believe that Option 2 also violates the agreement. Similar issue for unbalanced frequency hopping pattern can also exist for Option 2, which is resolved by Option 3. We share similar view as FL that in this specification, physical slot index is for the slot index within a frame. |
| QC | Yes | There was in fact a long debate on this in RAN1-108e. Huawei specifically asked about frame boundaries and we even excluded certain entries. Please see the following agreement: Agreement - Values for PUSCH-Frequencyhopping-Interval are “{2, 4, 5, 6, 8, 10, 12, 14, 16, 20}” - Note: For unpaired spectrum, UE is not expected to be configured the value of 6, 8, 12, 14, 16 Can companies clarify why they thought certain entries are being excluded? Also, the agreement in question was made a compromise --- we prioritized legacy operation for PUCCH and PUSCH repetition hopping and went with relative slot index for PUCCH and physical slot index for PUSCH. There should have been no doubt as to how this was supposed to operate. |
| OPPO | No | Same view as CATT. |
| Ericsson | No | Similar view as CATT. Also, the agreement says ‘based on’, not ‘using only’ physical slot index. The agreement requires that the slot number is used, but does not require that frame number is not used. Regarding the performance benefit, we expect gain, but expect the main benefit is that the gNB receiver and scheduler need to be less concerned about frame number boundaries. |
| Sharp | No | Same view as CATT. |
| CMCC | Yes | To solve the problem about FH across frame boundaries, the option 2 i s acceptable for us. |
| Nokia/NSB | No | Share the view as CATT & Ericsson. |
| ZTE | Yes | Share the view as Intel. Option 2 is not physical slot index while frame index + physical slot index. On other other hand, we can also accept Option 2 if Option 3 is not adopted. |
| Huawei, HiSilicon |  | @QC, for your question, we raised two issues before the agreement was made. One is the 20ms restriction of TDD UL/DL configuration, the other is the frame boundary issue. The former one led to adding the note because those values cannot always divide 20ms. The latter one is that those values cannot always divide frame duration, so to keep physical indexing, SFN should be involved. We asked in GTW whether companies were on the same page before to agree the new values. In my recollection, ZTE asked me why the note could be motivated by any SFN. I got a change to answer that we had identified two issues rather than only the 20ms restriction. Our comments about these issues can also be found in the summary of last meeting. |

## 3rd	round discussion

Background of this discussion is that how to decide the details on FH pattern based on the following agreement.

Agreement (made in RAN1 #108e)

Inter-slot frequency hopping pattern for PUSCH repetitions with DMRS bundling is determined based on physical slot index

Based on companies input in the 1st round discussion, the following is summarized.

- Option 1: using physical slot index only

  - Pros: No need to consider the SFN to determine the hop.

  - Cons: Unbalanced hopping pattern whenever repetitions across frame boundaries.

  - Supported by (5): VIVO, Apple, QC, Samsung, CMCC

- Option 2: using SFN and physical slot index

  - Pros: More balanced hopping pattern (vs option1) when repetitions across frame boundaries, especially for TDD.

  - Cons: Unbalanced hopping pattern at the boundary of SFN 1023 and 0.

  - Supported by (10): Panasonic, Nokia/NSB, Interdigital, Sharp, Ericsson, CATT, DOCOMO, CTC, Huawei/HiSi, OPPO

- Option 3: using relative SFN and physical slot index.

  - Pros: a few companies claim option 3 has more balanced hopping pattern (vs option2) in some scenarios, while others claim option 2 and 3 has similar performance consider all scenarios.

  - Cons: Conflict with the previous agreement. It is unfriendly to UE multiplexing as well.

  - Supported by (2): Intel, ZTE

Summary of companies input to FL question 2 & 3 in 2nd round discussion are listed as below.

Option 2 does not overturn previous agreements “Inter-slot frequency hopping pattern for PUSCH repetitions with DMRS bundling is determined based on physical slot index”.

- Supporting companies: CATT, CT, Panasonic, IDC, OPPO, Ericsson, Sharp, Nokia/NSB, Huawei/HiSi

Option 2 overturns previous agreements “Inter-slot frequency hopping pattern for PUSCH repetitions with DMRS bundling is determined based on physical slot index”.

- Supporting companies: VIVO, Intel, QC, CMCC (can accept option 2), ZTE (can accept option 2)

This is a tough situation. Based on majority support, FL recommend take option 2.

FL proposal 1a (majority view): Update the following agreement (made in RAN1 #108e) as the following.

Agreement

Inter-slot frequency hopping pattern for PUSCH repetitions with DMRS bundling is determined based on physical slot index and SFN.

If the above is not agreeable, then FL recommend the following backup plan to close this issue.

FL proposal 1b (backup plan): Update the following agreement (made in RAN1 #108e) as the following.

Agreement

Inter-slot frequency hopping pattern for PUSCH repetitions with DMRS bundling is determined based on physical slot index.

Note: following definition in TS 38.211, physical slot index does not take SFN into account.

Companies, please add comments on the above two proposals, if any, to the table below.

| Company name | Comment |
| --- | --- |

# Issue #2: frequency hopping for Msg3

In R1-2204513, it is identified that the highlighted part of the following agreement is not correctly captured in current specification.

Agreement For inter-slot FH for Msg3 PUSCH repetition, adopt the following legacy rules. - The Rel-16 RB offset determination mechanism defined in Table 8.3-1 of TS 38.213 for intra-slot FH for Msg3 PUSCH is reused. - The Rel-16 additional DMRS configuration defined in Clause 6.2.2 of TS 38.214 for Msg3 PUSCH in case intra-slot FH is disabled is reused. - The Rel-16 inter-slot FH pattern defined in Clause 6.3.1 of TS 38.214 for PUSCH repetition type A is reused.

The agreement states that Rel-16 based hopping should be applied for inter-slot PH for Msg3 PUSCH. While in current spec, it does not distinguish between Msg3 PUSCH and non-Msg3 PUSCH, thus Rel-17 based hopping is applied to both of them, when PUSCH repetition is enabled.

Based on FL’s initial assessment, the proposals in R1-2204513 are reasonable. Therefore, they are recommended as FL proposals in the following.

FL proposal 2: Based on the agreement made in 107e that “For inter-slot FH for Msg3 PUSCH repetition, adopt the following legacy rules. The Rel-16 RB offset determination mechanism defined in Table 8.3-1 of TS 38.213 for intra-slot FH for Msg3 PUSCH is reused”, the following TP is adopted for TS 38.214.

In case of inter-slot frequency hopping and when PUSCH-DMRS-Bundling is not enabled, or for inter-slot frequency hopping for a PUSCH scheduled by RAR UL grant or DCI format 0_0 with CRC scrambled by TC-RNTI, the starting RB during slot is given by: , where is the current slot number within a system radio frame, where a multi-slot PUSCH transmission can take place, is the starting RB within the UL BWP, as calculated from the resource block assignment information of resource allocation type 1 (described in Clause 6.1.2.2.2) and is the frequency offset in RBs between the two frequency hops. In case of inter-slot frequency hopping and when PUSCH-DMRS-Bundling is enabled, and when a PUSCH is not scheduled by RAR UL grant or DCI format 0_0 with CRC scrambled by TC-RNTI, the starting RB during slot is given by: [RBstartns,fμ=RBstart RBstart+RBoffsetmodNBWPsize ns,fμ+nf∙Nslotframe,μNFH mod 2=0ns,fμ+nf∙Nslotframe,μNFH mod 2=1] where ns,fμ is the current slot number within a system radio frame, nf is the number of the system radio frame containing the current slot, Nslotframe,μ is the number of slots per frame for subcarrier spacing configuration μof the UL BWP that the PUSCH is transmitted on, NFH is the value of the higher layer parameter PUSCH-Frequencyhopping-Interval, is the starting RB within the UL BWP, as calculated from the resource block assignment information of resource allocation type 1 (described in Clause 6.1.2.2.2) andis the frequency offset in RBs between the two frequency hops.

Please provide comments to the above FL proposal, if any, in the following table.

| Company name | Comment |
| --- | --- |
| vivo | The TP seems fine given Msg3 repetition is always based on available slot and does not depend on the configuration of PUSCH-DMRS-Bundling. |
| Panasonic | We are fine with FL proposal 2. |
| Apple | Support |
| Nokia/NSB | We don’t understand the title of this issue. DM-RS bundling is not supported for Msg3 repetition. The TP seems to be fine though. |
| Intel | It seems not needed. The cases for applying inter-slot frequency hopping and inter-slot frequency hopping are clearly described in the spec as follows: - Intra-slot frequency hopping, applicable to single slot and multi-slot configured PUSCH transmission, multi-slot PUSCH transmission scheduled by DCI format 0_1 or 0_2, each of multiple PUSCH transmissions scheduled by a DCI if the higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH is configured and each of multiple configured grant PUSCH transmissions in a configuration where the higher layer parameters cg-nrofSlots and cg-nrofPUSCH-InSlot are provided. - Inter-slot frequency hopping, applicable to multi-slot PUSCH transmission.. |
| QC | The TP seems fine to us. |
| Samsung | OK |
| InterDigital | Support |
| Sharp | Support the proposal. |
| Ericsson | Regarding the first change, "when PUSCH-DMRS-Bundling is not enabled" actually means the RRC parameter PUSCH-DMRS-Bundling is absent according to its definition in 38.331 v17.0.0. This is the case for Msg3 repetition because section 6.1.7 of 38.214 lists the kinds of PUSCH transmission when PUSCH-DMRS-Bundling is enabled, but Msg3 is not one of them. If more clarity is needed, we advise the following change. “In case of inter-slot frequency hopping and when PUSCH-DMRS-Bundling is not enabled configured". Regarding the second change, as “when PUSCH-DMRS-Bundling is enabled” doesn’t happen to Msg3 PUSCH, we don’t see the need to especially highlight it with the added text. If the proposal of Issue#3 below is agreed, adding the text to exclude Msg3 is needed. pusch-DMRS-Bundling-r17 ENUMERATED {enabled} OPTIONAL, -- Need S pusch-DMRS-Bundling Indicates whether DMRS bundling and time domain window for PUSCH are jointly enabled. If the field is absent, DMRS bundling and time domain window for PUSCH are jointly disabled. ### 6.1.7 UE procedure for determining time domain windows for bundling DM-RS For PUSCH transmissions of PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2, PUSCH repetition Type A with a configured grant, PUSCH repetition Type B and TB processing over multiple slots, when PUSCH-DMRS-Bundling is enabled, and for PUCCH transmissions of PUCCH repetition, when PUCCH-DMRS-Bundling is enabled, the UE determines one or multiple nominal TDWs, as follows:. |
| CATT | OK |
| NTT DOCOMO | Support |
| ZTE | Ok with the proposal. |
| CMCC | Support. |
| CTC | Support. |
| Huawei, HiSilicon | Support. DMRS bundling is not supported for Msg3 repetition. |
| OPPO | Support. |
| FL | @Intel: Thanks for your input. But in the text you cited, regarding inter-slot FH, it only has this sentence “Inter-slot frequency hopping, applicable to multi-slot PUSCH transmission”, which seems does not clarify the ambiguity on msg3 PUSCH inter-slot hopping pattern should follow Rel-16 or R-17 rule. It seems to me that the TP is still needed. |
| Intel | Thanks for the clarification. We are fine with the TP. |

## 2nd and 3rd round discussion

Given that most of the companies are fine with the TP in general. Let’s try to agree the TP, with the minor update suggested by Ericsson to change “enabled” to “configured”.

Further update FL proposal 2: The following TP is adopted for Subclause 6.3.1 in TS 38.214.

In case of inter-slot frequency hopping and when PUSCH-DMRS-Bundling is not enabled configured, or for inter-slot frequency hopping for a PUSCH scheduled by RAR UL grant or DCI format 0_0 with CRC scrambled by TC-RNTI, the starting RB during slot is given by: , where is the current slot number within a system radio frame, where a multi-slot PUSCH transmission can take place, is the starting RB within the UL BWP, as calculated from the resource block assignment information of resource allocation type 1 (described in Clause 6.1.2.2.2) and is the frequency offset in RBs between the two frequency hops. In case of inter-slot frequency hopping and when PUSCH-DMRS-Bundling is enabled configured, and when a PUSCH is not scheduled by RAR UL grant or DCI format 0_0 with CRC scrambled by TC-RNTI, the starting RB during slot is given by: [RBstartns,fμ=RBstart RBstart+RBoffsetmodNBWPsize ns,fμ+nf∙Nslotframe,μNFH mod 2=0ns,fμ+nf∙Nslotframe,μNFH mod 2=1] where ns,fμ is the current slot number within a system radio frame, nf is the number of the system radio frame containing the current slot, Nslotframe,μ is the number of slots per frame for subcarrier spacing configuration μof the UL BWP that the PUSCH is transmitted on, NFH is the value of the higher layer parameter PUSCH-Frequencyhopping-Interval, is the starting RB within the UL BWP, as calculated from the resource block assignment information of resource allocation type 1 (described in Clause 6.1.2.2.2) andis the frequency offset in RBs between the two frequency hops.

Since majority are support FL proposal 2. For the second round, if you are supporting the proposal, you don’t need to provide further input. The following table is provided to collect objections.

| Objecting companies: |  |
| --- | --- |
| Company name | Reason for objection |
| Ericsson | Thanks for FL's update. It is clear that DMRS bundling doesn't apply to Msg3 repetition. Our interpretation is that "when PUSCH-DMRS-Bundling is not configured" includes Msg3 repetition, and "when PUSCH-DMRS-Bundling is enabled" doesn't include Msg3 repetition. The new text looks redundant to us. Anyway, we can accept the proposal for better clarity. |
| Nokia/NSB | We think that using mixing between “enabled” and “configured” leads to confusion. The original wording is better. In addition, it’s unclear how can this parameter be configured but not enabled and vice versa (how can it be enabled but not configured)? |
| FL | Thank Nokia for catching the mixed usage of “enabled” and “configured”. Now I changed both to configured. Hope it is acceptable to everyone now. @all, please check the above Further update FL proposal 2, and express your concern/objection, if any, in this table. |

# Issue #3: frequency hopping when frequency hopping interval is configured but DMRS bundling is not enabled

In R1-2204872 and R1-2204873, it is observed that “However, the current specifications do not support configuration of the Rel-17 inter-slot frequency hopping pattern when DMRS bundling is not configured.  This is an artificial restriction in our understanding, since the UE hopping pattern is independent of if DMRS bundling is configured.”

According to R1-2204872 and R1-2204873, the benefits to allow UEs follow Rel-17 frequency hopping pattern even when DMRS bounding not enabled are summarized as below

- Not all Ues may benefit from, or support, DMRS bundling, but such Ues should be able to hop with the same patterns used by DMRS bundling Ues in the same cell in order to maintain spectral efficiency when frequency hopping is used in the cell.

  - Gain tradeoffs from joint channel estimation and frequency hopping can vary e.g. with speed, or on channel conditions for a given UE.

- As shown in R1-2204873, the Rel-17 inter-slot hopping pattern provides significant (e.g. ~0.5 dB) gains over the Rel-15 hopping pattern even when DMRS bundling is not configured.

Based on the above analysis, In R1-2204872, the following proposal is made.

Proposal in R1-2204872:

- The Rel-17 frequency hopping pattern can be configured independently from DMRS bundling

FL would like to collect companies’ views on the above proposal, before making a recommendation to accept it or not. Please provide comments to the above proposal, if any, in the following table.

| Company name | Comment |
| --- | --- |
| vivo | We can understand the intention of this proposal. However, this would then be purely FH enhancement when it’s independent from DMRS bundling feature. We would like to hear more views from other companies on supporting this enhancement. |
| Panasonic | I share the vivo’s view. |
| Apple | Do not support (R16 pattern will be applied). This is an enhancement which was not evaluated before. |
| Nokia/NSB | We do not see a clear motivation/advantage for using inter-slot FH with inter-slot bunding without DM-RS bundling. |
| Intel | It is not clear to us whether this is in the scope for NR coverage enhancement WI. In the meeting, RAN1 only considered inter-slot frequency hopping with DMRS bundling. |
| Samsung | In this WI inter-slot FH has been considered only when DM-RS bundling is enabled. Thus, no need to discuss inter-slot FH enhancements without DM-RS bundling. |
| Sharp | OK with the proposal. That would enable UE multiplexing between Ues capable of DM-RS bundling and Ues not capable of DM-RS bundling. |
| Ericsson | Support the proposal. Appreciate the feedback from other companies. There seem to be three questions: if inter-slot FH requires DMRS bundling to function, if there is a benefit to inter-slot FH if DMRS bundling is not configured, and if Rel-17 FH without DMRS bundling is in the scope of the WID. - If inter-slot FH requires DMRS bundling to function As can be seen from the 38.214 excerpt above, the frequency hopping offset is set by the frequency hopping interval length N_FH and the index of the slot in which the PUSCH is transmitted. It does not depend on the nominal or actual TDWs. It should therefore be clear that there is no dependence of the specified Rel-17 hopping behavior on DMRS bundling. - If there is a benefit to inter-slot FH if DMRS bundling is not configured We first would like to point out that configuring inter-slot FH without DMRS bundling has been suggested for a number of meetings, so this is nothing new (see e.g. R1-2107561 from August 2021). One of the benefits identified early was that Ues not configured with DMRS bundling could hop along with Ues that are configured with DMRS bundling and hopping, improving spectral efficiency. Another benefit is that there is also a link budget gain (e.g. ~0.5 dB), as discussed in R1-2204873. So there are both spectral efficiency benefits and link budget gains from Rel-17 inter-slot FH when DMRS bundling is not configured. - If the Rel-17 hopping pattern without DMRS bundling is in the WI scope The benefit of a new hopping pattern for DMRS bundling motivated inclusion in the WID. However, how a new feature combines with other features is normally discussed during the UE capabilities phase, or perhaps earlier in the WI. What RAN1 have specified is a frequency hopping pattern, not hopping as a function of DMRS bundling, so combining hopping with other features is quite natural in our view. Then if there are concerns about the scope within the WID, the WID does not identify PUCCH for frequency hopping enhancement, although we have specified it for PUCCH. To then say inter-slot FH can’t operate without inter-slot bundling is inconsistent with specifying it for PUCCH. PUCCH hopping is implemented differently than PUSCH, whereas support without DMRS bundling does not affect the hopping pattern at all. - Specification of PUSCH enhancements [RAN1, RAN4] - … - Specify mechanism(s) to enable joint channel estimation [RAN1, RAN4] - … - Inter-slot frequency hopping with inter-slot bundling to enable joint channel estimation [RAN1]. Therefore it is inconsistent to say that Rel-17 FH pattern without DMRS bundling is not in scope of the WID, given that PUCCH hopping was specified and was not explicitly included in the WID. Lastly, we think the spec change to clarify that DMRS bundling is not required for Rel-17 inter-slot hopping is quite simple, e.g. 38.214 section 6.3.1: In case of inter-slot frequency hopping and when PUSCH-DMRS-Bundling is enabled or PUSCH-Frequencyhopping-Interval is configured, the starting RB during slot is given by: [RBstartns,fμ=RBstart RBstart+RBoffsetmodNBWPsize ns,fμ+nf∙Nslotframe,μNFH mod 2=0ns,fμ+nf∙Nslotframe,μNFH mod 2=1]. |
| CATT | We do not see a need. As described in the WID that inter-slot frequency hopping with inter-slot bundling to enable joint channel estimation, enabled DMRS bundling should be a prerequisite for the enhanced frequency hopping pattern. |
| NTT DOCOMO | We understand the intention of the proposal, on the other hands, the proposals seems for improvement of HF and it is not included in the scope. |
| ZTE | No strong preference from our side. It could be ok to take it considering the potential gain while it may not be within the scope. |
| CMCC | Do not support this proposal. The motivation for introducing FH interval is to enable the FH under DMRS-Bundling. If DMRS-bundling is not configured, FH interval should not be configured. |
| CTC | We don’t support. We think this issue was discussed during the past meetings, and we think it is out of the scope. |
| Huawei, HiSilicon | No. It is not in line with the following agreement. Agreement: - For Rel-17 PUSCH repetition Type A without joint channel estimation, no new inter-slot frequency hopping mechanism is introduced. |  |
| OPPO | There is no need to enhance FH without DMRS-Bundling. |  |

## 2nd round discussion

Based on the input in the first-round discussion, majority companies don’t support this proposal. Some companies acknowledge the potential gain, while question whether it is included in the scope of the WID. Given the current situation, FL recommend to stop the discussion on this issue.

# Issue #5: dynamic PUCCH repetition factor indication for HARQ-ACK of first SPS PDSCH

Till now, the conclusion on whether or not supporting dynamic PUCCH repetition factor indication for HARQ-ACK for the first SPS PDSCH is still pending, because the ambiguity whether its PUCCH resource follow PRI in activation DCI or configured n1PUCCH-AN or sps-PUCCH-AN-List-r16 in RRC. Fortunately, in RAN1 108e, it was clarified that the PUCCH resource for the HARQ-ACK of the first SPS-PDSCH is not determined by the PRI in the activation DCI. It is determined by n1PUCCH-AN or sps-PUCCH-AN-List-r16 in RRC instead, based on the following agreement.

Agreement The TP for SPS PDSCH activation and PUCCH resource selection for the 1st SPS PDSCH is endorsed in R1-2202898 (TS38.213, Rel-16, CR#0286, Cat. F), R1-2202899 (TS38.213, Rel-17, CR#0287, Cat. A). - Note: Qualcomm, LG, DOCOMO, and Apple have concern that the above text proposal removes the functionality of PRI and DAI of DL SPS activation DCI. But these companies can accept the proposal for the sake of RAN1 progress, to avoid the consequence of losing the functionality of DL SPS in NR, given this proposal is the only solution due to other company’s objection to other proposal(s).

Based on the above clarifications, it was proposed in R1-2203193, R1-2203309, and R1-2204957 that the dynamic PUCCH repetition factor indication mechanism agreed in RAN1 106e is not applied to HARQ-ACK for the first SPS PDSCH associated with the activation DCI, while it should still be applied to HARQ-ACK for SPS release.

R1-2203193 has the following proposal.

Proposal 1: In NR Rel-17, for HARQ-ACK for SPS PDSCH, it is clarified that the dynamic PUCCH repetition factor indication mechanism agreed in RAN1 106e applies to HARQ-ACK corresponding to the SPS release DCI.

- Note: dynamic PUCCH repetition factor indication mechanism is not applied to HARQ-ACK for the first SPS PDSCH associated with the activation DCI.

- No specification impact is expected.

R1-2203309 has the following proposal.

- In NR Rel-17, the dynamic PUCCH repetition factor indication mechanism agreed in RAN1 106e does not apply to HARQ-ACK for SPS PDSCH including the first SPS PDSCH associated with the activation DCI except for the HARQ-ACK corresponding to the SPS release DCI

R1-2204957 has the following proposal.

Proposal 1: RAN1 concludes that the dynamic PUCCH repetition factor indication mechanism of NR R17 is not applicable to HARQ-ACK for the first SPS PDSCH associated with the activation DCI.

Proposal 2: RAN1 concludes that the dynamic PUCCH repetition factor indication mechanism of NR R17 is applicable to HARQ-ACK corresponding to the SPS release DCI.

FL’s initial assessment is that the above proposals are reasonable and straightforward. Although there is no specification impact as mentioned in the proposals, it is still beneficial to have a conclusion to close this discussion officially. Therefore, the following conclusion is proposed.

FL proposed conclusion: RAN1 conclude the following.

- The dynamic PUCCH repetition factor indication mechanism of NR R17 is not applicable to HARQ-ACK for the first SPS PDSCH associated with the activation DCI

- The dynamic PUCCH repetition factor indication mechanism of NR R17 is applicable to HARQ-ACK corresponding to the SPS release DCI

Note: no specification impact with the above conclusions.

Please provide comments to the above FL proposed conclusion, if any, in the following table.

| Company name | Comment |
| --- | --- |
| vivo | Fine with FL proposal. |
| Panasonic | We are fine with FL proposed conclusion. |
| Nokia/NSB | We don’t think a conclusion is needed. However, we are fine with the proposed conclusion if it is majority view. |
| Intel | We are fine with the conclusion. |
| Samsung | The specs are clear. No need for the conclusion. |
| InterDigital | Fine with FL proposal. |
| Sharp | Fine with the proposal. |
| Ericsson | We are OK with the conclusion to make it easier to follow the decisions made in the WI. However, it is not essential in our view, since the spec is already clear. |
| CATT | Fine with the proposal. |
| NTT DOCOMO | We are fine with FL proposal. |
| ZTE | Support |
| CMCC | Fine with FL conclusion. |
| CTC | Fine. |
| Huawei, HiSilicon | Since the specs are clear, we don’t feel a conclusion is needed. But if is majority view, we are fine with it. |
| OPPO | Fine with FL proposal. |

## 2nd and 3rd round discussion

Based on companies input from 1st round of discussion, majority are either supportive or fine to accept the above proposed conclusion. Given this issue has been discussed in RAN1 meeting for quite a few meetings, it is still beneficial to officially document a conclusion for the issue. Given this has no spec impact, FL just propose this as a conclusion to be captured in Chair’s notes.

FL proposed conclusion: RAN1 conclude the following.

- The dynamic PUCCH repetition factor indication mechanism of NR R17 is not applicable to HARQ-ACK for the first SPS PDSCH associated with the activation DCI

- The dynamic PUCCH repetition factor indication mechanism of NR R17 is applicable to HARQ-ACK corresponding to the SPS release DCI

Note: no specification impact with the above conclusions.

Since majority are either supportive or fine to accept the above proposed conclusion. For the second round, if you support or can accept the proposal, you don’t need to provide further input. The following table is provided to collect objections.

| Objecting companies: |  |
| --- | --- |
| Company name | Reason for objection |

# References

| R1-2203096 | Discussion on joint channel estimation for PUSCH and PUCCH | Huawei, HiSilicon |
| --- | --- | --- |
| R1-2203192 | Discussion on remaining issues for joint channel estimation | ZTE |
| R1-2203309 | Discussion on joint channel estimation for PUSCH&PUCCH | Spreadtrum Communications |
| R1-2203402 | Discussion on joint channel estimation for PUSCH and PUCCH | Panasonic |
| R1-2203440 | Remaining issues on joint channel estimation in Rel-17 | CATT |
| R1-2203522 | Remaining issues on joint channel estimation | vivo |
| R1-2203611 | Remaining issues on joint channel estimation for PUSCH and PUCCH | Nokia, Nokia Shanghai Bell |
| R1-2203652 | Remaining issues on joint channel estimation for PUSCH and PUCCH | China Telecom |
| R1-2203870 | Joint channel estimation for PUSCH and PUCCH | Samsung |
| R1-2204090 | Joint channel estimation for PUSCH and PUCCH | InterDigital, Inc. |
| R1-2204213 | Remaining issues on cross-slot channel estimation for PUSCH | Apple |
| R1-2204279 | Discussion on the remaining issues of joint channel estimation for PUSCH and PUCCH | CMCC |
| R1-2204350 | Remaining issues on joint channel estimation for PUSCH and PUCCH for coverage enhancement | NTT DOCOMO, INC. |
| R1-2204455 | Discussion on joint channel estimation for PUSCH&PUCCH | Spreadtrum Communications |
| R1-2204513 | Joint channel estimation for PUSCH and PUCCH | Sharp |
| R1-2204549 | Remaining issues on Joint channel estimation for PUCCH and PUSCH | WILUS Inc. |
| R1-2204776 | Remaining issues on joint channel estimation for PUSCH and PUCCH | Intel Corporation |
| R1-2204872 | Maintenance of Joint Channel Estimation for PUSCH and PUCCH | Ericsson |
| R1-2204991 | Joint channel estimation for PUSCH and PUCCH | Qualcomm Incorporated |