3GPP TSG RAN WG1 #109-e	R1-2205186

# e-Meeting, May 9th – 20th, 2022

Agenda item:	9.5.1.2

Source: 	Samsung

Title: 	Discussion on Evaluation for SL Positioning

Document for:	Discussion and Decision

- Introduction

In this contribution, we discuss about SL positioning evaluation for Rel-18 study on expanded and improved NR positioning and provide our view.

- Discussion

In the SID [1], the following objective is included regarding the SL positioning evaluation:

- Define evaluation methodology with which to evaluate SL positioning for the uses cases and coverage scenarios, reusing existing methodologies from sidelink communication and from positioning as much as possible [RAN1].

In the following subsections, we discuss about evaluation methodology for SL positioning and provide performance evaluation results.

  - Evaluation methodology

At first, as proposed in our companion contribution [2], any scenario can be considered for SL positioning. However, the technical solutions for SL positioning should be agnostic over scenarios as long as the requirements can be met. In this aspect, we do not need to perform SL positioning evaluation work for all SL coverage scenarios (i.e. IC, PC, and OoC) and all use cases of V2X, public safety, commercial, and IIOT. This is very important to reduce work load for SL performance evaluation.

Observation 1: In order to reduce work load, SL positioning evaluation work does not need to perform on all SL coverage scenarios and all uses cases captured in the SID.

Then, we need to discuss about SL positioning scenarios which should be taken into consideration when the evaluation methodology is defined for SL positioning. Unlike Uu positioning, SL positioning is typically performed between UE pair and a UE cannot utilize positioning signals from base stations in OoC scenario. In this regard, OoC scenario would be a key difference in SL positioning which did not evaluate before. Also, depending on SL environments, the number of available measurement sources for positioning can be limited and its quality can be deteriorating from both reference and measurement UEs’ moving. Therefore, the evaluation methodology for OoC scenario should be defined at first with consideration of realistic SL environments. Specifically, we need to take into account for V2X and public safety use cases. On the other hand, for IC scenario we can reuse the existing evaluation methodology as much as possible. For example, UMa, UMi, indoor office models [3] can be considered for commercial use case and InF-SH and InF-DH models [4] can be considered for IIOT use case. Lastly, we can deprioritize the PC scenario since its necessary is not clear yet.

Observation 2: For SL positioning evaluation, we can make observations for the following:

- For OoC scenario, the evaluation methodology should be defined at first with consideration of realistic SL environments.

- For IC scenario, the existing evaluation methodology in TR38.855 and TR38.857 can be reused as much as possible.

- PC scenario can be deprioritize if its necessity is not identified.

| - Square model | - Double square model 1 | - Double square model 2 |
| --- | --- | --- |

Figure 1 Square models for OoC scenario for SL positioning.

Based on the Observation 2, the evaluation methodology for SL positioning needs to be defined for OoC scenario at first with consideration of realistic SL environments. In our understanding, SL positioning should be able to work in stand-alone manner in OoC scenario while in IC scenario SL positioning can be used in supplementary manner to assist Uu positioning (Also, it can be used in stand-alone manner). Therefore, the performance evaluation for SL positioning in OoC scenario is important to study potential solutions for SL positioning. Also, we need to evaluate the performance of SL positioning in IC scenario to study its benefits with Uu positioning and potential issues. For OoC scenario, we can consider square models as shown in Figure 1 where UE-A is a measurement UE to perform SL positioning and UE-Bs are reference UEs. UE-Bs becomes transmission point (TP) for transmitting SL positioning signals and information and thus the distance between UE-Bs is defined as inter-TP distance.  The benefit of square model is in its simplicity and we can observe the impact of inter-TP distance easily. In SL positioning, high accuracy is essential when UEs are in close proximity whereas the accuracy constraint can be relaxed for further away UEs. Therefore, we need to evaluate the SL performance in various inter-TP distance. In addition, the movement both from reference UE (UE-B in Figure 1) and measurement UE (UE-A in Figure 1) needs to be considered in evaluation to reflect realistic SL environments. Therefore, we propose:

Proposal 1: The following evaluation scenarios are defined for NR SL positioning studies

- Scenario 1. Out-of-coverage,

  - SL positioning should be able to work in stand-alone manner

  - Square model can be used and various inter-TP distance used for evaluation.

  - The movement from both reference UEs and measurement UE is considered in evaluation.

- Scenario 2. In-coverage,

  - SL positioning can be used to assist Uu positioning.

  - SL positioning can be used in stand-alone manner

  - UMi/Uma/Indoor models (TR38.885) and InF-SH/InF-DH models (TS38.857) can be reused.

- FFS: Specific parameters for the evaluation scenarios

Next, we need to define a performance metric for SL positioning evaluation. As we did in Rel-16 and Rel-17 positioning evaluation work [3][4], we can use CDFs of horizontal and vertical (vertical error not necessarily applicable to all solutions and/or scenarios) positioning errors as performance metrics in NR SL positioning evaluations.

Proposal 2: At least CDFs of horizontal and vertical (vertical error not necessarily applicable to all solutions and/or scenarios) positioning errors are used as performance metrics in NR SL positioning evaluations

In addition, we need to make clear about a role of road side unit (RSU) if it is included in SL positioning evaluation. Simply, we can consider following two directions as:

- Direction 1: Tx/Rx UE (including UE type RSU), gNB/TRP (including gNB type RSU)

- Direction 2: Tx/Rx UE, gNB/TRP, RSU(PRU)

The direction 1 is that RSU can be treated as both UE type and gNB type. On the other hand, in direction 2, RSU is treated as a separate device. To avoid additional discussion to define RSU and evaluation methodology, we prefer the direction 1.

Proposal 3: If RSU is considered for SL positioning evaluation, it can be treated as both UE type and gNB type.

| - An effect from the reference UEs’ reliability | - An effect from the link quality between reference UE and measurement UE |
| --- | --- |

Figure 2 The performance of SL positioning.

  - Evaluation results

In this section, we provide initial evaluation results for SL positioning based on evaluation methodology suggested in the previous section. In Figure 2, we plot the CDFs of horizontal positioning errors in OoC scenario by using square model in Figure 1. More details on evaluation assumptions are given in Appendix.

At first, in Figure 2(a), we evaluate the performance of SL positioning accuracy depending on reference UEs’ reliability. We assume that the reference UEs (UE-Bs in Figure 1) have different reliability for the known location information and they are moving in an opposite direction from the measurement UE (UE-A in Figure 1) at speed of 70km/h. The measurement UE selects 4 or 6 reliable reference UEs to calculate absolute position and we compare this with no selection. It shows that selecting high reliable reference UEs can provide best positioning accuracy. In other way, in Figure 2(b), we evaluate the SL positioning accuracy depending on the link quality between reference UE and measurement UE. The measurement UE selects 4 reference UEs to calculate absolute position where all channel between reference UE is in LOS (TDL-D with desired delay spread in 10ns) or in NLOS (TDL-C with desired delay spread in 10ns and 100ns, respectively). It shows that LOS channel can provide best positioning accuracy. Therefore, the following observation and proposal can be drawn:

Observation 3: In SL positioning, measurement UE’s selection of reference UE (i.e. measurement source) is important for achieving high positioning accuracy.

Proposal 4: Study for measurement UE to decide measurement source(s) for SL positioning with proper criteria.

- Conclusions

This contribution discusses about evaluation methodology for SL positioning and provide initial evaluation results. Based on the discussion, the following observations and proposals are provided:

Observation 1: In order to reduce work load, SL positioning evaluation work does not need to perform on all SL coverage scenarios and all uses cases captured in the SID.

Observation 2: For SL positioning evaluation, we can make observations for the following:

- For OoC scenario, the evaluation methodology should be defined at first with consideration of realistic SL environments.

- For IC scenario, the existing evaluation methodology in TR38.855 and TR38.857 can be reused as much as possible.

- PC scenario can be deprioritize if its necessity is not identified.

Proposal 1: The following evaluation scenarios are defined for NR SL positioning studies

- Scenario 1. Out-of-coverage,

  - SL positioning should be able to work in stand-alone manner

  - Square model can be used and various inter-TP distance used for evaluation.

  - The movement from both reference UEs and measurement UE is considered in evaluation.

- Scenario 2. In-coverage,

  - SL positioning can be used to assist Uu positioning.

  - SL positioning can be used in stand-alone manner

  - UMi/Uma/Indoor models (TR38.885) and InF-SH/InF-DH models (TS38.857) can be reused.

- FFS: Specific parameters for the evaluation scenarios

Proposal 2: At least CDFs of horizontal and vertical (vertical error not necessarily applicable to all solutions and/or scenarios) positioning errors are used as performance metrics in NR SL positioning evaluations

Proposal 3: If RSU is considered for SL positioning evaluation, it can be treated as both UE type and gNB type.

Observation 3: In SL positioning, measurement UE’s selection of reference UE (i.e. measurement source) is important for achieving high positioning accuracy.

Proposal 4: Study for measurement UE to decide measurement source(s) for SL positioning with proper criteria.

References

RP-213588, “Study on expanded and improved NR positioning,” RAN#94-e, Dec., 2021.

R1-2203909, “On SL Positioning Scenarios and Requirements,” Samsung.

TR38.855, “Study on NR Positioning Support,” V16.0.0.

TR38.857, “Study on NR Positioning Enhancements,” V17.0.0.

Appendix: Evaluation assumptions

Table 1 Parameters for SL positioning evaluation in Figure 2.

| Parameter | Figure 2(a) | Figure 2(b) |
| --- | --- | --- |
| Channel Model | TDL-C with desired delay spread in 10ns | TDL-C with desired delay spread in 10ns and 100ns. TDL-D with desired delay spread in 10ns |
| Carrier Frequency | 4 GHz |  |
| Subcarrier Spacing | 30 kHz |  |
| Positioning Reference Signal Transmission Bandwidth | 100 MHz |  |
| Positioning Reference Signal Information | NR DL PRS (Comb-2, 2-symbols), 1 port |  |
| Number of reference UEs | 4 or 6 |  |
| UE power | 23 dBm |  |
| Inter-TP distance | 20 m |  |
| Positioning methods (OTDOA, Multi-RTT, …) | Multi-RTT based absolute positioning |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Threshold based peak detection (max Correlation) |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, taylor series, etc) | Taylor series |  |