3GPP TSG RAN WG1 #109-e	R1-2205189

e-Meeting, May 9th – May 20th, 2022

Source:	Moderator (OPPO)

Title:	Preparation phase summary of maintenance on Rel-17 Intra-UE Multiplexing/Prioritization – B

Agenda Item:	8.3.2

Document for:	Discussion and Decision

- Introduction

In this paper, Tdocs submitted to RAN1#109-e on this issue are summarized. The discussion plan for this meeting is proposed in Section 5 where companies can input comments.

- Remaining issues on multiplexing UCIs of different priorities in a PUCCH

Issue#1: HP PUCCH with HARQ-ACK and SR collides LP HARQ-ACK (HW, ZTE, Nokia, Spreadtrum, E///, CATT, vivo, Samsung, OPPO, DOCOMO, ITRI, WILUS, Intel, Qualcomm, LG)

Discussion status in last meeting:

When a PUCCH carrying explicit/implicit HP SR and HP HARQ-ACK with PUCCH format 0/1 overlaps with a PUCCH carrying LP HARQ-ACK, where the original PUCCH carrying the HP HARQ-ACK before Step 1 overlaps with K HP SRs which are all negative or include at least one positive,

- In Step 1, 1-bit HP SR is multiplexed to HP HARQ-ACK bits (based on Rel-15 rules) for determining the resultant PUCCH resource.

  - Note: The description of Step 1 here is only for information purpose, which has no spec impact.

- In Step 2, down-select from the two options:

  - Option 0: LP HARQ-ACK is dropped.

  - Option 2: 1-bit HP SR is appended to HP HARQ-ACK bits. The number of HP UCI bits is OUCI=OACK,1+1

    - The resultant PUCCH resource for multiplexing HP SR + HP HARQ-ACK + LP HARQ-ACK is either of PF2, PF3, or PF4.

HW proposal:

Proposal 2: When a PUCCH carrying HP SR and HP HARQ-ACK with PF0/1 overlaps with a PUCCH carrying LP HARQ-ACK:

- Option 2 is adopted, i.e., if HP HARQ-ACK is dynamic HARQ-ACK or SPS HARQ-ACK with sps-PUCCH-AN-List, 1-bit HP SR is appended to HP HARQ-ACK bits. The number of HP UCI bits is OUCI=OACK,1+1.

Proposal 3: For the case of HP SR with PF0 dropped due to collision with HP HARQ-ACK PF1 in Step 1, the resultant HP PUCCH after Step 1 is regarded as HP PUCCH with HARQ-ACK only when resolving the overlapping with LP HARQ-ACK in Step 2.

ZTE proposal:

Proposal 1: When a PUCCH carrying HP SR and HP HARQ-ACK with PUCCH format 0/1 overlaps with a PUCCH carrying LP HARQ-ACK, where the original PUCCH carrying the HP HARQ-ACK before Step 1 overlaps with K HP SRs,

  - If the total number of LP and HP HARQ-ACK bits is 2, the multiplexing follows the previous agreement for multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, i.e., treat the two bits as HARQ-ACK bits with high priority and the resultant PUCCH format is 0/1.

  - If the total number of LP and HP HARQ-ACK bits is larger than 2,

    - If the resultant PUCCH after multiplexing between HP SR and HP HARQ-ACK doesn't use the PUCCH resource with n1PUCCH-AN or SR resource with PF0/1, 1-bit HP SR is appended to HP HARQ-ACK bits. The number of HP UCI bits is OUCI=OACK,1+1. The resultant PUCCH resource for multiplexing HP SR + HP HARQ-ACK + LP HARQ-ACK is either of PF2, PF3, or PF4.

    - Otherwise, LP HARQ-ACK is dropped.

Nokia proposal:

Proposal 2.1: To resolve overlapping between a HP PUCCH carrying SR and HP HARQ-ACK with PUCCH format 0/1 and a LP PUCCH carrying HARQ-ACK and CSI/SR, only LP HARQ-ACK is multiplexed with HP UCI (i.e., SR and HARQ-ACK) and LP CSI/SR are dropped.

Proposal 3.1: When a PUCCH carrying explicit/implicit HP SR and HP HARQ-ACK with PUCCH format 0/1 overlaps with a PUCCH carrying LP HARQ-ACK, where the original PUCCH carrying the HP HARQ-ACK before Step 1 overlaps with K HP SRs which are all negative or include at least one positive,

- In Step 1, 1-bit HP SR is multiplexed to HP HARQ-ACK bits (based on Rel-15 rules) for determining the resultant PUCCH resource.

  - Note: The description of Step 1 here is only for information purpose, which has no spec impact.

- In Step 2,

  - if the HP HARQ-ACK is using a PUCCH resource provided by PUCCH-ResourceSet or SPS-PUCCH-AN-List, 1-bit HP SR is appended to HP HARQ-ACK bits. The number of HP UCI bits is OUCI=OACK,1+1

    - The resultant PUCCH resource for multiplexing HP SR + HP HARQ-ACK + LP HARQ-ACK is either of PF2, PF3, or PF4.

    - Reuse the (agreed) procedures for multiplexing of LP HARQ-ACK and HP HARQ-ACK on PUCCH resource with PF 2/3/4, i.e., PUCCH resource selection, separate coding, PRB determination, rate matching, power control, etc.

  - if HP HARQ-ACK is using a PUCCH resource provided by n1PUCCH-AN, the LP HARQ-ACK is dropped.

Spreadtrum proposal:

- Clarify that when a UE would transmit SR in a resource using PUCCH format 0 and HARQ-ACK information bits in a resource using PUCCH format 1 in a slot, the resultant PUCCH format 1 is regarded as negative SR or no SR.

E/// proposal:

- When a PUCCH carrying HP SR and HP HARQ-ACK with PF 0/1, the number of SR bits is K=1.

When a HP PUCCH carrying SR and HARQ-ACK with PF0/1 overlaps with a LP PUCCH carrying HARQ-ACK,

- 1-bit HP SR is appended to 1 or 2-bit HP HARQ-ACK, and treated together as a 2 or 3-bit HP HARQ-ACK;

- Reuse other procedures for multiplexing LP HARQ-ACK and HP HARQ-ACK onto a PUCCH resource with PF 2/3/4, i.e. separate coding, PRB determination, rate matching and power control.

- If the HP HARQ-ACK is a dynamic HARQ-ACK, a PUCCH resource indicated by PRI is used for multiplexing.

- If the HP HARQ-ACK is an SPS HARQ-ACK, a PUCCH resource determined from the PUCCH resource(s) provided by sps-PUCCH-AN-List is used for multiplexing.

When a HP PUCCH carrying SR and HARQ-ACK with PF0/1 overlaps with a LP PUCCH carrying HARQ-ACK and CSI/SR, only LP HARQ-ACK is multiplexed with HP UCI (i.e., SR and HARQ-ACK) and LP CSI/SR are dropped.

CATT proposal:

Proposal 1: If a PUCCH carrying HP SR and HP HARQ-ACK with PUCCH format 0/1 overlaps with a PUCCH carrying LP HARQ-ACK, LP HARQ-ACK is multiplexed with HP SR and HP HARQ-ACK where 1 bit HP SR is appended to HP HARQ-ACK bits and treated as HP UCI.

Proposal 2: If a PUCCH carrying HP SR and HP HARQ-ACK with PUCCH format 0/1 overlaps with a PUCCH carrying LP HARQ-ACK and LP CSI/SR, only LP HARQ-ACK is multiplexed with HP UCI and LP CSI/SR are dropped.

vivo proposal:

Proposal 1: When a PUCCH carrying explicit/implicit HP SR and HP HARQ-ACK with PUCCH format 0/1 overlaps with a PUCCH carrying LP HARQ-ACK, where the original PUCCH carrying the HP HARQ-ACK before Step 1 overlaps with K HP SRs which are all negative or include at least one positive,

- In Step 1, 1-bit HP SR is multiplexed to HP HARQ-ACK bits (based on Rel-15 rules) for determining the resultant PUCCH resource.

  - Note: The description of Step 1 here is only for information purpose, which has no spec impact.

- In Step 2,

  - Option 2a:

    - If HP HARQ-ACK is dynamic HARQ-ACK or SPS HARQ-ACK with SPS-PUCCH-AN-List, 1-bit HP SR is appended to HP HARQ-ACK bits. The number of HP UCI bits is OUCI=OACK,1+1

    - If HP HARQ-ACK is SPS HARQ-ACK with n1PUCCH-AN, LP HARQ-ACK is dropped.

Samsung proposal:

Proposal 1: For resolving collision of PUCCHs and/or PUSCHs of different priorities, Step 2 only considers the resulting PUCCHs and/or PUSCHs in Step 1.

Proposal 2: For resolving collision of PUCCHs of different priorities in step 2.1, if a LP PUCCH with LP HARQ-ACK overlaps with a HP PUCCH PF 0/1 with HP HARQ-ACK and positive HP SR, the LP PUCCH is dropped.

OPPO proposal:

Proposal 2: When a PUCCH carrying HP SR and HP HARQ-ACK with PUCCH format 0/1 overlaps with a PUCCH carrying LP HARQ-ACK, where the original PUCCH carrying the HP HARQ-ACK before Step 1 overlaps with K HP SRs, in step 2, 1-bit HP SR is appended to HP HARQ-ACK bits. The number of HP UCI bits is OUCI=OACK,1+1.

DOCOMO proposal:

- Summary of change

Add the corresponding changes to make the specification complete.

- Consequences if not approved

The spec is incomplete.

- Text proposal 1:

-------------------------- Start of Text Proposal for TS 38.213 -------------------------- <Unchanged parts omitted> 9 UE procedure for reporting control information <Unchanged parts omitted> if // this is for cases the UE supports multiplexing information of different priorities in a PUCCH/PUSCH transmission - a PUCCH transmission with HARQ-ACK information, without repetitions, with smaller priority index overlaps with a PUCCH transmission only with HARQ-ACK information, without repetitions, with larger priority index, or - a PUCCH transmission without repetitions that includes HARQ-ACK information of smaller priority index overlaps with a PUCCH transmission without repetitions using a PUCCH resource with PUCCH format 2/3/4 with that includes HARQ-ACK information and SR of larger priority index, or - a PUCCH transmission with HARQ-ACK information, without repetitions, with smaller or larger priority index overlaps, respectively, with a PUSCH transmission with larger or smaller priority index the UE - multiplexes HARQ-ACK information of different priority indexes and SR information of larger priority index, if any, in a same PUCCH transmission of larger priority index, or multiplexes HARQ-ACK information the UE would provide in a PUCCH transmission of smaller or larger priority index in a PUSCH transmission of larger or smaller priority index, respectively, and applies the procedures in clause 9.2.5.3 or 9.3, respectively, and - drops CSI and/or SR carried in the PUCCH of smaller priority index, if any - drops HARQ-ACK information of smaller priority index if the UE would multiplex the HARQ-ACK information of smaller priority index in a PUSCH where the UE multiplexes Part 1 CSI reports and Part 2 CSI reports of larger priority index - drops Part 2 CSI reports of smaller priority index if the UE would multiplex the HARQ-ACK information of smaller and larger priority indexes in a PUSCH where the UE multiplexes Part 1 CSI reports and Part 2 CSI reports of smaller priority index <Unchanged parts omitted> -------------------------- End of Text Proposal for TS 38.213 --------------------------

ITRI proposal:

Proposal 7:

To resolve overlapping between a HP PUCCH carrying SR and HP HARQ-ACK with PUCCH format 0/1 and a LP PUCCH, only LP HARQ-ACK is multiplexed with HP UCI. And the number of HP UCI bits is .

WILUS proposal:

- Proposal 1: For resolve overlapping of HP SR in a PF0/1 and LP HARQ-ACK/SR in PF0/1, the LP HARQ-ACK/SR in PF0/1 is dropped.

- Proposal 2: For resolve overlapping of HP HARQ-ACK/SR in a PF0/1 and LP HARQ-ACK, we propose to allow the multiplexing of HP HARQ-ACK/SR and LP HARQ-ACK in the resultant PUCCH resource for multiplexing of HP PUCCH and LP PUCCH by appending 1-bit HP SR to HP HARQ-ACK bits.

- Proposal 3: For resolve overlapping of HP HARQ-ACK/SR in a PF0/1 and LP HARQ-ACK/SR, we propose to allow the multiplexing of HP HARQ-ACK/SR and LP HARQ-ACK in the resultant PUCCH resource for multiplexing of HP PUCCH and LP PUCCH by appending only 1-bit HP SR to HP HARQ-ACK bits after dropping LP SR.

LG proposal:

Proposal #1: Agree the following proposal made in RAN1#108-e by adopting Option 4 or Option 7 for Step 2 below.

When a PUCCH carrying HP SR and HP HARQ-ACK with PUCCH format 0/1 overlaps with a PUCCH carrying LP HARQ-ACK, where the original PUCCH carrying the HP HARQ-ACK before Step 1 overlaps with K HP SRs which are all negative or include at least one positive, - In Step 1, 1-bit HP SR is multiplexed to HP HARQ-ACK bits (based on Rel-15 rules) for determining the resultant PUCCH resource. - Note: The description of Step 1 here is only for information purpose, which has no spec impact. - In Step 2, adopt one of the following two options. - Option 4: - If there is 1-bit HP HARQ-ACK and 1 bit LP HARQ-ACK, - If HP SR and HP HARQ-ACK are on PUCCH format 1, LP HARQ-ACK bit can be simply treated as HP HARQ-ACK bit, and the two HP HARQ-ACK bits are multiplexed with the HP SR using Rel-16/Rel-15 rules. - If HP SR and HP HARQ-ACK are on PUCCH format 0, LP HARQ-ACK is dropped. - If at least one of the HP HARQ-ACK payload size or LP HARQ-ACK payload size is greater than or equal to 2, - If HP HARQ-ACK is dynamic HARQ-ACK or SPS HARQ-ACK with sps-PUCCH-AN-List, 1-bit HP SR is appended to HP HARQ-ACK bits. The number of HP UCI bits is OUCI=OACK,1+1 (i.e. OUCI=OACK,1+log2K+1). - If HP HARQ-ACK is SPS HARQ-ACK with n1PUCCH-AN, LP HARQ-ACK is dropped. - Option 7: - If there is 1-bit HP HARQ-ACK and 1 bit LP HARQ-ACK, LP HARQ-ACK is dropped. - If at least one of the HP HARQ-ACK payload size or LP HARQ-ACK payload size is greater than or equal to 2, - If HP HARQ-ACK is dynamic HARQ-ACK or SPS HARQ-ACK with sps-PUCCH-AN-List, 1-bit HP SR is appended to HP HARQ-ACK bits. The number of HP UCI bits is OUCI=OACK,1+1. - If HP HARQ-ACK is SPS HARQ-ACK with n1PUCCH-AN, LP HARQ-ACK is dropped.

Intel proposal:

Proposal 1: When a PUCCH carrying positive HP SR and HP HARQ-ACK by PUCCH format 0/1 overlaps with a PUCCH carrying LP HARQ-ACK, the PUCCH carrying LP HARQ-ACK is drpped.

Qualcomm proposal:

Proposal 4: When a HP positive SR in PUCCH format 0/1 overlaps with LP HARQ-ACK and SR in PUCCH format 0/1, LP HARQ-ACK and SR are dropped.

Proposal 5: When a PUCCH carrying HP SR and HP HARQ-ACK with PUCCH format 0/1 overlaps with a PUCCH carrying LP HARQ-ACK, option 2 is adopted.

- Option 2: 1-bit HP SR is appended to HP HARQ-ACK bits. The number of HP UCI bits is OUCI=OACK,1+1. The HP UCI is then multiplexed with LP HARQ-ACK

  - A UE expect that the resultant PUCCH resource for multiplexing HP SR + HP HARQ-ACK + LP HARQ-ACK is in PF2, PF3, or PF4.

Proposal 6: When a PUCCH carrying HP SR and HP HARQ-ACK with PUCCH format 0/1 overlaps with a PUCCH carrying LP HARQ-ACK and SR.

- 1-bit HP SR is appended to HP HARQ-ACK bits. The number of HP UCI bits is OUCI=OACK,1+1. The HP UCI is then multiplexed with LP HARQ-ACK with the LP SR dropped.

  - A UE expect that the resultant PUCCH resource for multiplexing HP SR + HP HARQ-ACK + LP HARQ-ACK is in PF2, PF3, or PF4.

Issue#2: Multiplexing for SPS HARQ-ACK (HW, Samsung)

This issue was discussed in some company’ contribution in last meeting. In this meeting, more companies addressed this issue.

Status in running specification:

UE determines the PUCCH resource from the second sps-PUCCH-AN-List using OUCI=OACK,0+OACK,1

HW proposal:

Proposal 1: When a PUCCH carrying HP HARQ-ACK provided by n1PUCCH-AN overlaps with a PUCCH carrying LP HARQ-ACK with total number of HARQ-ACK bits more than 2 bits, the LP HARQ-ACK is dropped.

Samsung proposal:

Proposal 4: For resolving collision of two overlapping PUCCHs with different priorities in R17, if a HP PUCCH with SPS PDSCH HARQ-ACK overlaps with a LP PUCCH with HARQ-ACK and sps-PUCCH-AN-List is not provided in the second PUCCH-config, the LP PUCCH is dropped.

Issue#3: Interlace number adjustment for PUCCH (Apple)

Apple proposal:

Proposal 3-1: If a UE is provided a first interlace of MInterlace,0PUCCH PRBs by interlace0 in InterlaceAllocation, the UE has HARQ-ACK, SR and wideband or sub-band CSI reports to transmit, and the UE determines a PUCCH resource with PUCCH format 2, or the UE has HARQ-ACK, SR and wideband CSI reports to transmit and the UE determines a PUCCH resource with PUCCH format 3, or the UE has HARQ-ACK, SR and sub-band CSI reports to transmit, and the UE determines a PUCCH resource with PUCCH format 3, where

- the UE determines the PUCCH resource using the PUCCH resource indicator field in a last of a number of DCI formats with a value of a PDSCH-to-HARQ_feedback timing indicator field indicating a same slot for the PUCCH transmission, from a PUCCH resource set provided to the UE for HARQ-ACK transmission, and after the UE determines the PUCCH resource set

- if

n=1NUCI-part2totalOUCI-part2,n+OCRC,UCI-part2/Qm⋅r2+⌈n=1NUCI-part1totalOUCI-part1,n+OCRC,UCI-part1/Qm⋅r1⌉≤MInterlace,0PUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH,

the UE transmits the HARQ-ACK, SR, and CSI reports bits in a PUCCH over the first interlace

- else, if the UE is provided a second interlace of MInterlace,1PUCCH PRBs by interlace1 and if

n=1NUCI-part2totalOUCI-part2,n+OCRC,UCI-part2/Qm⋅r2+⌈n=1NUCI-part1totalOUCI-part1,n+OCRC,UCI-part1/Qm⋅r1⌉≤MInterlace,0PUCCH+MInterlace,1PUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH,

the UE transmits the HARQ-ACK, SR, and CSI reports bits in a PUCCH over both the first and second interlaces

- else, the UCI omission procedure is same as the corresponding one when the UE is provided PUCCH-ResourceSet by replacing MRBPUCCH with MInterlace,0PUCCH, or, if the UE is provided interlace1, by MInterlace,0PUCCH+MInterlace,1PUCCH.

Issue#4: Power control enhancement for PUCCH (Apple)

Apple proposal:

Proposal 5-1:  For PUCCH formats 2/3/4, the delta factor ΔTF,b,f,ci is determined from UCI part 1:

- The number of resource elements for UCI part 1 NRE-part1(i)=EUCI-part1EtotNRE(i),  where   EUCI-part1 is the number of coded bits for UCI part 1

- If  n=1NUCI-part1total‍OUCI-part1,n  is smaller or equal to 11,

  - ΔTF,b,f,c,part1i=10⋅log10(K1⋅n=1NUCI-part1total‍OUCI-part1,n/NRE-part1i)

  - If a HARQ-ACK codebook with OUCI-part1,n'  bits is included in UCI part 1,  is used instead of OUCI-part1,n'  for the HARQ-ACK codebook:

    - ΔTF,b,f,c,part1i=10⋅log10(K1⋅n=1NUCI-part1total‍OUCI-part1,n-OUCI-part1,n' +nHARQ-ACK(i)/NRE-part1i)

  - If more than one HARQ-ACK codebooks are included in UCI part 1 (e.g. one due to SPS HARQ deferral, another for HARQ feedback for dynamic grant PDSCH(s)), then replacement of the number of HARQ-ACK codebook size by the associated nHARQ-ACK can be applied to each HARQ-ACK codebook.

- otherwise

  - ΔTF,b,f,c,part1i=10log10(2K2⋅BPREi-1)  where

    - and

    - BPREi=n=1NUCI-part1total‍OUCI-part1,n+OCRC,UCI-part1/NRE-part1i

- And ΔTF,b,f,ci = ΔTF,b,f,c,part1i is applied to both UCI parts.

Proposal 5-2: the issue for PUCCH format 4 needs to be tackled. Proposal 5-1 above provides a solution for that.

Proposal 5-3: For Rel-17 intra-UE multiplexing, retain the difference between  and OUCI,1 .

Issue#5: Clarification on PUCCH resouce determination in spec (Qualcomm)

Qualcomm proposal:

Proposal 1: for multiplexing HP HARQ-ACKs and LP HARQ-ACKs, it is confirmed that current specification is interpreted as the following.

- The PRI in the lastly received DCI (if exist), which schedules a HP HARQ-ACK involved in the multiplexing, is used to select the PUCCH resource to transmit the multiplexed UCI payload.

- Note: no spec update is needed

- Remaining issues on multiplexing UCIs of different priorities in a PUCCH

Issue#6: Insufficient resources of HP PUSCH multiplexed with LP HARQ-ACK (HW, ZTE, Spreadtrum, CATT, Samsung, OPPO, InterDigital)

Discussion status in last meeting:

For the scenarios where multiplexing low-priority HARQ-ACK onto high-priority PUSCH, down-select from the options:

  - Option 1: In case of insufficient resource for LP HARQ-ACK, LP HARQ-ACK is entirely dropped

  - Option 2: LP HARQ-ACKs are mapped to the rest REs of the PUSCH based on the rate matching equation, if HP HARQ-ACK and/or HP CSI have been mapped in prior on the PUSCH.

  - Option 3: UE does not expect insufficient resource for multiplexing low-priority HARQ-ACK.

HW proposal:

Proposal 4: For the scenario of multiplexing low-priority HARQ-ACK onto high-priority PUSCH, support Option 2, i.e. LP HARQ-ACKs are mapped to the rest REs of the PUSCH based on the rate matching equation, if HP HARQ-ACK and/or HP CSI have been mapped in prior on the PUSCH.

ZTE proposal:

Proposal 4: LP HARQ-ACKs are mapped to the rest REs of the PUSCH based on the rate matching equation, if HP HARQ-ACK and/or HP CSI have been mapped in prior on the PUSCH.

Spreadtrum proposal:

- For the scenarios where multiplexing low-priority HARQ-ACK onto high-priority PUSCH using rate matching and RE mapping of CSI part 2,

  - Option 2: LP HARQ-ACKs are mapped to the rest REs of the PUSCH based on the rate matching equation, if HP HARQ-ACK and/or HP CSI have been mapped in prior on the PUSCH.

CATT proposal:

Proposal 3: If the amount of REs available for LP HARQ-ACK mapping on the HP PUSCH is not sufficient to satisfy the coding rate of the LP HARQ-ACK based on the corresponding beta offset, LP HARQ-ACK is mapped to the rest REs after mapping HP HARQ-ACK or HP CSI .

Samsung proposal:

Proposed conclusion: When multiplexing a LP HARQ-ACK in a HP PUSCH, LP HARQ-ACKs are mapped to the rest of PUSCH REs based on the rate matching equation, if HP HARQ-ACK and/or HP CSI, if any, were mapped on the PUSCH.

OPPO proposal:

Proposal 3: In case of insufficient resource for LP HARQ-ACK, LP HARQ-ACK is entirely dropped.

InterDigital proposal:

Proposal 3: In case HARQ-ACK of priority 0 would be multiplexed in PUSCH of priority 1 and insufficient resource is available to accommodate HARQ-ACK, the HARQ-ACK is dropped.

Issue#7: Multiplexing of CG-UCI on PUSCH of a different priority (HW, ZTE, New H3C, Nokia, Spreadtrum, E///, Apple, DCM, LG, Intel, Qualcomm)

Agreement

If LP HARQ-ACK without HP HARQ-ACK would be transmitted on HP PUSCH, down-select from the options:

- Option 2: UE follows the same behaviour as that in case of PUSCH with HP HARQ-ACK assuming two bits

- FFS for CG-UCI PUSCH.

HW proposal:

Proposal 5: Adopt the following rules for multiplexing CG-UCI with HARQ-ACK of a different priority:

- For CG-UCI on HP PUSCH,

- LP HARQ-ACK without HP HARQ-ACK: CG-UCI is regarded as HP HARQ-ACK to reuse the rule of HP HARQ-ACK and LP HARQ-ACK on HP PUSCH that has been agreed.

- LP HARQ-ACK with HP HARQ-ACK: CG-UCI is jointly encoded with HP HARQ-ACK to reuse the rule of HP HARQ-ACK and LP HARQ-ACK on HP PUSCH that has been agreed.

- For CG-UCI on LP PUSCH,

- HP HARQ-ACK without LP HARQ-ACK: CG-UCI is regarded as LP HARQ-ACK to reuse the rule of HP HARQ-ACK and LP HARQ-ACK on LP PUSCH that has been agreed.

- HP HARQ-ACK with LP HARQ-ACK: CG-UCI is jointly encoded with LP HARQ-ACK to reuse the rule of HP HARQ-ACK and LP HARQ-ACK on LP PUSCH that has been agreed.

ZTE proposal:

Proposal 2: For HARQ-ACK multiplex on the PUSCH with CG UCI, if the CG-UCI could be concatenated with the HARQ-ACK with the same priority, the resultant concatenated UCI should be treated as HARQ-ACK to be multiplexed on the PUSCH by legacy rules.

Proposal 3: For HARQ-ACK multiplex on the PUSCH with CG UCI, if the CG-UCI could not be concatenated with the HARQ-ACK with the same priority, the CG-UCI should be treated as HARQ-ACK to be multiplexed on the PUSCH by legacy rules.

New H3C proposal:

Proposal 1: If LP HARQ-ACK with CG-UCI would be transmitted on HP PUSCH, CG-UCI and LP HARQ-ACK should be multiplexed together based on current legacy scheme as mentioned in section 6.3.2 of TS 38.212.

Nokia proposal:

Proposal 3.2: For multiplexing a LP HARQ-ACK without HP HARQ-ACK into a HP CG PUSCH carrying CG-UCI, UE follows the same behaviour as in case of PUSCH with HP HARQ-ACK assuming two bits.

- If the assumed (2-bit) HP HARQ-ACK, LP HARQ-ACK, HP CSI including a single part, and CG-UCI would be transmitted on the HP CG PUSCH,

  - Reuse Rel-16 procedure for handing HP HARQ-ACK and CG-UCI: joint coding, rate matching and RE mapping for jointly coded HP HARQ-ACK and CG-UCI.

  - Reuse Rel-15 CSI part 1 rate matching and RE mapping for the single part of HP CSI.

  - Reuse Rel-15 CSI part 2 rate matching and RE mapping for LP HARQ-ACK.

- If the assumed (2-bit) HP HARQ-ACK, LP HARQ-ACK and HP CSI including two parts would be transmitted on the HP CG PUSCH,

  - LP HARQ-ACK is dropped.

  - Reuse Rel-16 procedure for handling HP HARQ-ACK, CG-UCI and two parts HP CSI on HP CG PUSCH.

Spreadtrum proposal:

- CG-UCI is with the same priority of the CG-PUSCH.

- If there are same priority of CG-UCI and HARQ-ACK, Rel-16 NR-U joint coding of HARQ-ACK and CG-UCI is done in the step 1, and then UCI types with different priorities colliding with HARQ-ACK and CG-UCI on PUSCH are done according to Rel-17 intra-UE multiplexing rules.

- If there are no same priority of CG-UCI and HARQ-ACK, the rate matching/puncturing and RE mapping listed in Table 2 is applied.

  - If LP HARQ-ACK would be transmitted on HP PUSCH with CG-UCI, CG-UCI is treated as HP HARQ-ACK assuming the existence, i.e. by reusing the rate matching/puncturing and RE mapping for HP HARQ-ACK.

  - If HP HARQ-ACK would be transmitted on LP PUSCH with CG-UCI, CG-UCI is treated as LP HARQ-ACK assuming the existence, i.e. by reusing the rate matching/puncturing and RE mapping for LP HARQ-ACK.

E/// proposal:

When a HP (or LP) CG-PUSCH carrying CG-UCI overlaps with a LP (or HP) PUCCH carrying HARQ-ACK, treat the CG-PUSCH with CG-UCI as a CG-PUSCH with a HARQ-ACK sequence of the same priority. Then apply the existing procedure for multiplexing a HP (or LP) PUSCH carrying HARQ-ACK with a LP (or HP) PUCCH carrying HARQ-ACK, including the procedure for handling CSI/SR (if any).

Apple proposal:

- Observation 4: agreement from Rel-17 IIoT/URLLC unlicensed spectrum access session is only about the support of UCI types in CG PUSCH, not about how that is supported, which belongs to the intra-UE multiplexing discussion.

- Proposal 4: For Rel-17 intra-UE multiplexing, reuse R15 HARQ-ACK rate matching/puncturing and RE mapping for CG-UCI (in the absence of HP HARQ-ACK) or CG-UCI/HP-HARQ-ACK (in the presence of HP HARQ-ACK). LP-HARQ-ACK is treated as if CG-UCI is absent.

DOCOMO proposal:

- Summary of change

Add the corresponding changes to make the specification complete.

- Consequences if not approved

The spec is incomplete.

- Text proposal 2:

-------------------------- Start of Text Proposal for TS 38.212 -------------------------- <Unchanged parts omitted> 6.3.2.1.5 UCI with different priority indexes If UCI-MuxWithDifferentPriority is configured, and HARQ-ACK bits associated with priority index 0, and CSI part 1 if any are transmitted on a PUSCH associated with priority index 1, the following UCI bit sequences are generated, a0(1), a1(1), a2(1), a3(1), …,aA(1)-11, and a0(2), a1(2), a2(2), a3(2), …,aA(2)-12 if any, according to the following: - If CSI part 1 is also transmitted on the PUSCH, - Set ai(1) for i=0,1,…,A(1)-1 as the bit sequence of CSI part 1, where the CSI fields of all CSI reports, in the order from upper part to lower part in Table 6.3.2.1.2-6, are mapped to the UCI bit sequence a0(1), a1(1), a2(1), a3(1), …,aA(1)-11 starting with a0(1). - Set ai(2)=oiACK-LP for i=0,1,…,OACK-LP-1 and A(2)=OACK-LP, where the HARQ-ACK bit sequence o0ACK-LP, o1ACK-LP, …, oOACK-LP-1ACK-LP associated with priority index 0 is given by Clause 9.1 of [5, TS 38.213]. - Otherwise, set ai(1)=oiACK-LP for i=0,1,…,OACK-LP-1 and A(1)=OACK-LP, where the HARQ-ACK bit sequence o0ACK-LP, o1ACK-LP, …, oOACK-LP-1ACK-LP associated with priority index 0 is given by Clause 9.1 of [5, TS 38.213]. If UCI-MuxWithDifferentPriority and cg-UCI-Multiplexing are configured, and HARQ-ACK bits associated with priority index 0, and CG-UCI are transmitted on a PUSCH associated with priority index 1, the following UCI bit sequences are generated, a0(1), a1(1), a2(1), a3(1), …,aA(1)-11, and a0(2), a1(2), a2(2), a3(2), …,aA(2)-12 if any, according to the following: - Set ai(1)=oiCG-UCI for i=0,1,…,OCG-UCI-1 and A(1)=OCG-UCI, where the CG-UCI bit sequence o0CG-UCI, o1CG-UCI, …, oOCG-UCI-1CG-UCI is given by Table 6.3.2.1.3-1, mapped in the order from upper part to lower part.. - Set ai(2)=oiACK-LP for i=0,1,…,OACK-LP-1 and A(2)=OACK-LP, where the HARQ-ACK bit sequence o0ACK-LP, o1ACK-LP, …, oOACK-LP-1ACK-LP associated with priority index 0 is given by Clause 9.1 of [5, TS 38.213]. <Unchanged parts omitted> 6.3.2.4.1.6 UCI with different priority indexes If UCI-MuxWithDifferentPriority is configured, and HARQ-ACK bits associated with priority index 0, and CSI part 1 if any are transmitted on a PUSCH associated with priority index 1: - If CSI part 1 is also transmitted on the PUSCH, - Perform rate matching for CSI part 1 according to clause 6.3.2.4.1.2, by assuming the number of HARQ-ACK information bits to be transmitted on PUSCH in clause 6.3.2.4.1.2 is 0 bit. - Perform rate matching for HARQ-ACK with priority index 0 according to clause 6.3.2.4.1.3, by taking HARQ-ACK with priority index 0 as CSI part 2 and replacing βoffsetPUSCH by βoffsetHARQ-ACK-LP, and assuming the number of HARQ-ACK information bits to be transmitted on PUSCH in clause 6.3.2.4.1.3 is 0 bit. - Otherwise, perform rate matching for HARQ-ACK with priority index 0 according to clause 6.3.2.4.1.2, by taking HARQ-ACK with priority index 0 as CSI-part 1 and replacing βoffsetPUSCH by βoffsetHARQ-ACK-LP, and assuming the number of HARQ-ACK information bits to be transmitted on PUSCH in clause 6.3.2.4.1.2 is 0 bit. If UCI-MuxWithDifferentPriority and cg-UCI-Multiplexing are configured, and HARQ-ACK bits associated with priority index 0, and CG-UCI are transmitted on a PUSCH associated with priority index 1: - Perform rate matching for CG-UCI according to clause 6.3.2.4.1.1, by taking CG-UCI as HARQ-ACK and replacing βoffsetPUSCH by βoffsetCG-UCI. - Perform rate matching for HARQ-ACK with priority index 0 according to clause 6.3.2.4.1.2, by taking HARQ-ACK with priority index 0 as CSI part 1 and replacing βoffsetPUSCH by βoffsetHARQ-ACK-LP. <Unchanged parts omitted> -------------------------- End of Text Proposal for TS 38.212 --------------------------

LG proposal:

Observation #1: In case when the multiplexing of UCIs with different priories on PUCCH/PUSCH is enabled, there seems no reason to disable cg-UCI-Multiplexing since any CSI would not be multiplexed on CG PUSCH (thus, the number of UCI encodings on CG PUSCH would not be larger than 3).

Observation #2: In case when the multiplexing of UCIs with different priories on PUCCH/PUSCH is enabled and also cg-UCI-Multiplexing is enabled, joint encoding between CG-UCI and HARQ-ACK may not be required (even for the same priority) due to absence of the CSI on CG PUSCH.

Proposal #2: Consider the following cases and UE behaviours in case when the multiplexing of UCIs with different priories on PUCCH/PUSCH is enabled and also cg-UCI-Multiplexing is enabled.

- Case H1 - {HP CG-UCI, HP HARQ-ACK, LP HARQ-ACK} would be multiplexed on HP CG PUSCH - Option 1: HP CG-UCI and HP HARQ-ACK are jointly encoded, and LP HARQ-ACK is separately encoded from the jointly-encoded HP UCIs. - The jointly-encoded HP UCIs reuse the encoding/rate-matching/RE mapping of Rel-15 HARQ-ACK, and the LP HARQ-ACK reuses the encoding/rate-matching/RE mapping of Rel-15 CSI part 1. - Option 2: HP HARQ-ACK, HP CG-UCI, and LP HARQ-ACK are separately encoded. - The HP HARQ-ACK, HP CG-UCI, LP HARQ-ACK reuse the encoding/rate-matching/RE mapping of Rel-15 HARQ-ACK, CSI part 1, CSI part 2, respectively. - Case H2 - {HP CG-UCI, LP HARQ-ACK} would be multiplexed on HP CG PUSCH - HP CG-UCI and LP HARQ-ACK are separately encoded. - The HP CG-UCI reuses the encoding/rate-matching/RE mapping of Rel-15 HARQ-ACK, and the LP HARQ-ACK reuses the encoding/rate-matching/RE mapping of Rel-15 CSI part 1. - Case L1 - {HP HARQ-ACK, LP CG-UCI, LP HARQ-ACK} would be multiplexed on LP CG PUSCH - Option 1: LP CG-UCI and LP HARQ-ACK are jointly encoded, and HP HARQ-ACK is separately encoded from the jointly-encoded LP UCIs. - The HP HARQ-ACK reuses the encoding/rate-matching/RE mapping of Rel-15 HARQ-ACK, and the jointly-encoded LP UCIs reuse the encoding/rate-matching/RE mapping of Rel-15 CSI part 1. - Option 2: HP HARQ-ACK, LP HARQ-ACK, and LP CG-UCI are separately encoded. - The HP HARQ-ACK, LP HARQ-ACK, and LP CG-UCI reuse the encoding/rate-matching/RE mapping of Rel-15 HARQ-ACK, CSI part 1, CSI part 2, respectively. - Case L2 - {HP HARQ-ACK, LP CG-UCI} would be multiplexed on LP CG PUSCH - HP HARQ-ACK and LP CG-UCI are separately encoded. - The HP HARQ-ACK reuses the encoding/rate-matching/RE mapping of Rel-15 HARQ-ACK, and the LP CG-UCI reuses the encoding/rate-matching/RE mapping of Rel-15 CSI part 1.

Intel proposal:

- Proposal 2: When When the cg-RetransmissionTimer is configured and cg-UCI-Multiplexing is enabled, CG-UCI is jointly encoded with HP HARQ-ACK with beta offset for the HP HARQ-ACK.

Qualcomm proposal:

Proposal 3: In Rel-17 intra-UE multiplexing, treat CG-UCI of a certain priority as if a HARQ-ACK of the same priority, joint encode CG-UCI with HARQ-ACK of the same priority (if exist).

Issue#8: Bitwidth of Beta-offset indicator (CATT)

CATT proposal:

The bit widths of beta_offset indicator in DCI format 0_1 or DCI format 0_2 based on different sets of beta-offset values can be different.

As a regular practice, a number of most significant bits with value set to '0' can be inserted to smaller beta_offset indicator until the bit width of the beta_offset indicator in DCI format for the HARQ-ACKs with different priorities multiplexing on one PUSCH are the same.

Proposal 4: Adopt the following text proposal for section 7.3.1.1.2 and 7.3.1.1.3 of TS38.212.

----------------------------------------------------- Start of text proposal ------------------------------------------------------

7.3.1.1.2	Format 0_1

DCI format 0_1 is used for the scheduling of one or multiple PUSCH in one cell, or indicating CG downlink feedback information (CG-DFI) to a UE.

The following information is transmitted by means of the DCI format 0_1 with CRC scrambled by C-RNTI or CS-RNTI or SP-CSI-RNTI or MCS-C-RNTI:

-	Identifier for DCI formats – 1 bit

-	The value of this bit field is always set to 0, indicating an UL DCI format

*** Unchanged text is omitted ***

-	Second PTRS-DMRS association – 2 bits if PTRS-DMRS association field and SRS resource set indicator field are present and maxRank>2; 0 bit otherwise. Table 7.3.1.1.2-25 and 7.3.1.1.2-26 are used to indicate the association between PTRS port(s) and DMRS port(s) corresponding to Second SRS resource indicator field and/or Second precoding information field when one PT-RS port and two PT-RS ports are configured by maxNrofPorts in PTRS-UplinkConfig respectively, and the DMRS ports are indicated by the Antenna ports field.

-	beta_offset indicator – 0 if the higher layer parameter betaOffsets = semiStatic; otherwise 2 bits as defined by Table 9.3-3 in [5, TS 38.213].

When two HARQ-ACK codebooks are configured for the same serving cell and if higher layer parameter priorityIndicatorDCI-0-1 is configured, if the bit width of the beta_offset indicator in DCI format 0_1 for one HARQ-ACK codebook is not equal to that of the beta_offset indicator in DCI format 0_1 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller beta_offset indicator until the bit width of the beta_offset indicator in DCI format 0_1 for the two HARQ-ACK codebooks are the same.

When higher layer parameter UCI-MuxWithDifferentPriority and priorityIndicatorDCI-0-1 are both configured, the bit width of the beta_offset indicator in DCI format 0_1 is determined by the maximum bit width of the beta_offset indicator in DCI format 0_1 for HARQ-ACK of larger priority index on PUSCH of smaller priority index, HARQ-ACK of smaller priority index on PUSCH of smaller priority index, HARQ-ACK of smaller priority index on PUSCH of higher priority index, HARQ-ACK of higher priority index on PUSCH of higher priority index. A number of most significant bits with value set to '0' are inserted to smaller beta_offset indicator until the bit width of the beta_offset indicator in DCI format 0_1 for HARQ-ACKs with different priorities multiplexing on one PUSCH with larger or smaller priority index are the same.

-	DMRS sequence initialization – 0 bit if transform precoder is enabled; 1 bit if transform precoder is disabled.

*** Unchanged text is omitted ***

7.3.1.1.3	Format 0_2

DCI format 0_2 is used for the scheduling of PUSCH in one cell.

The following information is transmitted by means of the DCI format 0_2 with CRC scrambled by C-RNTI or CS-RNTI or SP-CSI-RNTI or MCS-C-RNTI:

-	Identifier for DCI formats – 1 bit

-	The value of this bit field is always set to 0, indicating an UL DCI format

*** Unchanged text is omitted ***

-	Second PTRS-DMRS association – 2 bits if PTRS-DMRS association field and SRS resource set indicator field are present and maxRankDCI-0-2>2; 0 bit otherwise. Table 7.3.1.1.2-25 and 7.3.1.1.2-26 are used to indicate the association between PTRS port(s) and DMRS port(s) corresponding to Second SRS resource indicator field and/or Second precoding information field when one PT-RS port and two PT-RS ports are configured by maxNrofPorts in PTRS-UplinkConfig respectively, and the DMRS ports are indicated by the Antenna ports field.

-	beta_offset indicator – 0 bit if the higher layer parameter betaOffsets = semiStatic; otherwise 1 bit if 2 offset indexes are configured by higher layer parameter dynamicDCI-0-2 as defined by Table 9.3-3A in [5, TS 38.213], and 2 bits if 4 offset indexes are configured by higher layer parameter dynamicDCI-0-2 as defined by Table 9.3-3 in [5, TS 38.213].

When two HARQ-ACK codebooks are configured for the same serving cell and if higher layer parameter priorityIndicatorDCI-0-2 is configured, if the bit width of the beta_offset indicator in DCI format 0_2 for one HARQ-ACK codebook is not equal to that of the beta_offset indicator in DCI format 0_2 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller beta_offset indicator until the bit width of the beta_offset indicator in DCI format 0_2 for the two HARQ-ACK codebooks are the same.

When higher layer parameter UCI-MuxWithDifferentPriority and priorityIndicatorDCI-0-2 are both configured, the bit width of the beta_offset indicator in DCI format 0_2 is determined by the maximum bit width of the beta_offset indicator in DCI format 0_2 for HARQ-ACK of larger priority index on PUSCH of smaller priority index, HARQ-ACK of smaller priority index on PUSCH of smaller priority index, HARQ-ACK of smaller priority index on PUSCH of higher priority index, HARQ-ACK of higher priority index on PUSCH of higher priority index. A number of most significant bits with value set to '0' are inserted to smaller beta_offset indicator until the bit width of the beta_offset indicator in DCI format 0_2 for HARQ-ACKs with different priorities multiplexing on one PUSCH with larger or smaller priority index are the same.

-	DMRS sequence initialization – 0 or 1 bit

-	0 bit if the higher layer parameter dmrs-SequenceInitializationDCI-0-2 is not configured or if transform precoder is enabled;

-	1 bit if transform precoder is disabled and the higher layer parameter dmrs-SequenceInitializationDCI-0-2 is configured.

*** Unchanged text is omitted ***

------------------------------------------------------ End of text proposal ------------------------------------------------------

Issue#9: Indicate the presence of LP HARQ-ACK and/or HP HARQ-ACK to be multiplexed in PUSCH (WILUS)

WILUS proposal:

- Proposal 4: In case of HP-PUSCH or LP-PUSCH contains LP HARQ-ACK and HP HARQ-ACK, it should be discussed how to indicate the presence of LP HARQ-ACK and/or HP HARQ-ACK to be multiplexed.

- Remaining issues on PHY prioritization between DG and CG PUSCHs with different priorities

Issue#10: Spec clarification reflecting the agreement on DG-CG PUSCH prioritization (ZTE)

ZTE proposal:

Proposal 5: A TP for 38.214 is proposed to update the section 6.1 on collision between CG PUSCH and DG PUSCH with different priorities.

Reason for change: In Rel-17, when DG PUSCH overlaps with CG PUSCH with different priority, the UE can cancel the PUSCH with lower priority and then transmit the PUSCH with higher priority. The timeline defined in Rel-15 is not applicable to this case.

Summary of the change: The DG PUSCH overlapping with CG PUSCH with different priority in Rel-17 is excluded.

Consequences if not approved: The UE may determine the Rel-17 overlapping is error case if the timeline is not satisfied.

# 6.1 UE procedure for transmitting the physical uplink shared channel < Unchanged part is omitted > A UE is not expected to be scheduled by a PDCCH ending in symbol i to transmit a first PUSCH on a given serving cell overlapping in time with a transmission occasion, where the UE is allowed to transmit a second PUSCH with configured grant according to [10, TS38.321], starting in a symbol j on the same serving cell if the end of symbol i is not at least N2 symbols before the beginning of symbol j unless the first PUSCH and the second PUSCH have the different priority index [9, TS38.213] and the UE is provided prioritizationBetweenLP-DG-PUSCHandHP-CG-PUSCH or prioritizationBetweenHP-DG-PUSCHandLP-CG-PUSCH. The value N2 in symbols is determined according to the UE processing capability defined in Clause 6.4, and N2 and the symbol duration are based on the minimum of the subcarrier spacing corresponding to the PUSCH with configured grant and the subcarrier spacing of the PDCCH scheduling the PUSCH. < Unchanged part is omitted >

- Discussion plan in RAN1#109-e

1st round discussion in preparation phase

Discussion plan suggested by the moderator:

|  | Topic | Companies addressing this issue in Tdoc | Discussion plan |
| --- | --- | --- | --- |
| Issue#1 | HP PUCCH with HARQ-ACK and SR collides LP HARQ-ACK | HW, ZTE, Nokia, Spreadtrum, E///, CATT, vivo, Samsung, OPPO, DOCOMO, ITRI, WILUS, Intel, Qualcomm, LG | High priority (Try GTW or email approval) |
| Issue#2 | Multiplexing for SPS HARQ-ACK | HW, Samsung | Low priority (Try email approval only) |
| Issue#3 | Interlace number adjustment for PUCCH | Apple | Low priority (Try email approval only) |
| Issue#4 | Power control enhancement for PUCCH | Apple | Low priority (Try email approval only) |
| Issue#5 | Clarification on PUCCH resouce determination in spec | Qualcomm | Low priority (Try email approval only) |
| Issue#6 | Insufficient resources of HP PUSCH multiplexed with LP HARQ-ACK | HW, ZTE, Spreadtrum, CATT, Samsung, OPPO, InterDigital | High priority (Try GTW or email approval) |
| Issue#7 | Multiplexing of CG-UCI on PUSCH of a different priority | HW, ZTE, New H3C, Nokia, Spreadtrum, E///, Apple, DCM, LG, Intel, Qualcomm | High priority (Try GTW or email approval) |
| Issue#8 | Bitwidth of Beta-offset indicator | CATT | Low priority (Try email approval only) |
| Issue#9 | Indicate the presence of LP HARQ-ACK and/or HP HARQ-ACK to be multiplexed in PUSCH | WILUS | Low priority (Try email approval only) |
| Issue#10 | Spec clarification reflecting the agreement on DG-CG PUSCH prioritization | ZTE | Low priority (Try email approval only) |

Please input your comments to the plan, and explain why you think a topic should be treated with a higher or lower priority.

| Company | Comments |
| --- | --- |
| Huawei/Hisi | Basically fine with the plan, except for the following two issues: For Issue#8: Not clear of the motivation; is the intention to align beta-offset field to the two different DCIs, i.e., 0_1 and 0_2? There seems to be no need to align the DCI payload size for different DCI formats. Clarifications would be good. For Issue#9: We think there is no strong need to repeatedly discuss the DAI issue as it has been discussed in the last meeting with no consensus. |
| vivo | We basically agree with FL’a plan except that we think Issue 6 can be low priority. It has been discussed in previous meetings without consensus. The current spec is workable without any new agreement. |
| QC | We in general agree with FL’s plan. In addition, we have a few comments as below: Issue 2: this should be an error case. gNB configuration should avoid this. Issue 4: the power control issues (at least some of them) are valid. It is good to discuss them. Issue 5: It is critical to align the understanding of companies on this, as it is a key step for different priorities HARQ-ACKs multiplexing. The effort RAN1 need to spend on this seems very small. Hopefully, all the companies have the have understanding of the spec on this issue. |
| Intel | We agree with FL’s plan for High priority issues. For low priority issues, issue 2 and 5, we’re fine for discussion to make a conclusion. We also support discussion for issue 3 to support URLLC over unlicensed band. For other issues, we think they’re not essential issues to be discussed. |
| OPPO | We agree with FL’s plan for High priority issues. |
| DOCOMO | We agree with FL’s plan with the following comment: For Issue#9, we don’t think it is reasonable to reopen the discussion since it has been discussed in the past several meetings without consensus. |
| LG | We also agree with FL’s plan for high priority issues (and add LG as the company addressing Issue #1). For Issue #2, it seems to need some discussion/decision considering Rel-17 UEs not supporting sps-PUCCH-AN-List. |
| New H3C | We agree with FL’s plan for High priority issues |
| Nokia / NSB | We agree with FL’s assessment in principle, except on Issue #9: We have a similar view as HW/HiSi & DCM that just by using some email approval will not be able to change companies opinions, considering we spent online time on it in the past (without consensus). |
| Spreadtrum | We agree with FL’s plan for High priority issues |
| CATT | We are generally fine with FL’s plan expect for Issue #9. We share the comments from other companies that there is no need to reopen the discussion in the maintenance phase. For the question from Huawei on issue #8, we would like to clarify that the intention is not to align the bit width between DCI format 0_1 and DCI format 0_2, but the align the bit width for each DCI format due to potentially different number of beta offset values configured for different combinations across PHY priorities. |
| Samsung | Issue 2, a conclusion would be helpful to align companies’ understanding. Issue 6, agree with vivo, it can be treated as low priority, the current spec is clear, optimization is not needed. |
| InterDigital | Agree with moderator assessment. |
| ZTE | Agree with FL’s plan for High priority issues. For Issue#10, it is easy to be identified and the possible proposal is not hard to understand. Fine to email discuss this issue. For issue 2, we’re fine for discussion to make a conclusion. |
| Ericsson | Agree with FL view on the high priority issues. For the low priority issues marked by FL: - Issue 9: no need to discuss considering conclusion at last meeting. - OK to discuss in this meeting: Issue 1, 2, 10. |

Discussion status after 1st round:

|  | Topic | Companies suggesting to discuss this issue | Discussion plan |
| --- | --- | --- | --- |
| Issue#1 | HP PUCCH with HARQ-ACK and SR collides LP HARQ-ACK | HW, ZTE, Nokia, Spreadtrum, E///, CATT, vivo, Samsung, OPPO, DOCOMO, ITRI, WILUS, Intel, Qualcomm, LG | High priority but will be discussed after relevant agreement in intra-UE A (Try GTW or email approval) |
| Issue#2 | Multiplexing for SPS HARQ-ACK | HW, Samsung (Qualcomm questions the necessity) | Low priority (Try email approval only) |
| Issue#3 | Interlace number adjustment for PUCCH | Apple | Low priority (Try email approval only) |
| Issue#4 | Power control enhancement for PUCCH | Apple | Low priority (Try email approval only) |
| Issue#5 | Clarification on PUCCH resouce determination in spec | Qualcomm | Low priority (Try email approval only) |
| Issue#6 | Insufficient resources of HP PUSCH multiplexed with LP HARQ-ACK | HW, ZTE, Spreadtrum, CATT, Samsung, OPPO, InterDigital (vivo, Samsung questions the necessity) | High priority (Try GTW or email approval) |
| Issue#7 | Multiplexing of CG-UCI on PUSCH of a different priority | HW, ZTE, New H3C, Nokia, Spreadtrum, E///, Apple, DCM, LG, Intel, Qualcomm | High priority (Try GTW or email approval) |
| Issue#8 | Bitwidth of Beta-offset indicator | CATT (Huawei questions the necessity) | Low priority (Try email approval only) |
| Issue#9 | Indicate the presence of LP HARQ-ACK and/or HP HARQ-ACK to be multiplexed in PUSCH | WILUS (Huawei, DOCOMO, Nokia, CATT, Ericsson question the necessity) | Low priority (Try email approval only) |
| Issue#10 | Spec clarification reflecting the agreement on DG-CG PUSCH prioritization | ZTE | Low priority (Try email approval only) |

2nd round discussion in preparation phase

Based on the 1st round discussion,

- Issue#9 is removed from the list as 5 companies suggested.

- As coordinated with Intra-UE MUX A moderator, discussion on issue#1 will not be triggered until the discussion on “Step 2 only considers the resulting PUCCHs and/or PUSCHs in Step 1” outputs a conclusion from Intra-UE MUX A.

|  | Topic | Companies suggesting to discuss this issue | Discussion plan |
| --- | --- | --- | --- |
| Issue#1 | HP PUCCH with HARQ-ACK and SR collides LP HARQ-ACK | HW, ZTE, Nokia, Spreadtrum, E///, CATT, vivo, Samsung, OPPO, DOCOMO, ITRI, WILUS, Intel, Qualcomm, LG | High priority but will be discussed after relevant agreement in intra-UE A (Try GTW or email approval) |
| Issue#2 | Multiplexing for SPS HARQ-ACK | HW, Samsung (Qualcomm questions the necessity) | Low priority (Try email approval only) |
| Issue#3 | Interlace number adjustment for PUCCH | Apple | Low priority (Try email approval only) |
| Issue#4 | Power control enhancement for PUCCH | Apple | Low priority (Try email approval only) |
| Issue#5 | Clarification on PUCCH resouce determination in spec | Qualcomm | Low priority (Try email approval only) |
| Issue#6 | Insufficient resources of HP PUSCH multiplexed with LP HARQ-ACK | HW, ZTE, Spreadtrum, CATT, Samsung, OPPO, InterDigital (vivo questions the necessity) | High priority (Try GTW or email approval) |
| Issue#7 | Multiplexing of CG-UCI on PUSCH of a different priority | HW, ZTE, New H3C, Nokia, Spreadtrum, E///, Apple, DCM, LG, Intel, Qualcomm | High priority (Try GTW or email approval) |
| Issue#8 | Bitwidth of Beta-offset indicator | CATT (Huawei questions the necessity) | Low priority (Try email approval only) |
| Issue#9 | Indicate the presence of LP HARQ-ACK and/or HP HARQ-ACK to be multiplexed in PUSCH | WILUS (Huawei, DOCOMO, Nokia, CATT, Ericsson question the necessity) | Low priority (Try email approval only) |
| Issue#10 | Spec clarification reflecting the agreement on DG-CG PUSCH prioritization | ZTE | Low priority (Try email approval only) |

Please input if you have further comments.

| Company | Comments |
| --- | --- |

- References

- R1-2203073	Intra-UE multiplexing enhancements	Huawei, HiSilicon

- R1-2203189	Discussion on enhanced intra-UE multiplexing	ZTE

- R1-2203213	Remaining issue on intra-UE multiplexing and prioritization	New H3C Technologies Co., Ltd.

- R1-2203274	Maintenance of Rel-17 URLLC / IIoT Intra-UE Multiplexing/Prioritization	Nokia, Nokia Shanghai Bell

- R1-2203305	Discussion on intra-UE multiplexing/prioritization	Spreadtrum Communications

- R1-2203398	Remaining Issues of Intra-UE Multiplexing/Prioritization for IIoT/URLLC	Ericsson

- R1-2203435	Maintenance on intra-UE multiplexing and prioritization	CATT

- R1-2203514	Remaining issues on intra-UE Multiplexing/Prioritization for Rel-17 URLLC	vivo

- R1-2203863	Maintenance on Intra-UE Multiplexing/Prioritization	Samsung

- R1-2204003	Remaining issues on intra-UE multiplexing/prioritization	OPPO

- R1-2204091	Remaining issues of Intra-UE Multiplexing/Prioritization	InterDigital, Inc.

- R1-2204206	Remaining issues in intra-UE multiplexing/prioritization	Apple

- R1-2204344	Remaining issues on intra-UE multiplexing/prioritization for Rel.17 URLLC	NTT DOCOMO, INC.

- R1-2204439	Remaining details on intra-UE multiplexing and prioritization	ITRI

- R1-2204547	Remaining issues on intra-UE multiplexing/prioritization for URLLC/IIoT	WILUS Inc.

- R1-2204617	Remaining issues on Intra-UE multiplexing enhancements	LG Electronics

- R1-2204647	Intra-UE Multiplexing/Prioritization	ETRI

- R1-2204770	Remaining issue for Intra-UE Uplink Channel Multiplexing and Prioritization	Intel Corporation

- R1-2204983	Remaining issues on Intra-UE multiplexing and prioritization for IOT and URLLC	Qualcomm Incorporated