3GPP TSG RAN WG1 #109-e	R1-2205190

e-Meeting, May 9th – May 20th, 2022

Source:	Moderator (OPPO)

Title:	Summary#1 of email thread [109-e-R17-IIoT-URLLC-03]

Agenda Item:	8.3.2

Document for:	Discussion and Decision

- Introduction

In this paper, discussions under the following email thread in RAN1#108 are summarized.

[109-e-R17-IIoT-URLLC-03] Email discussion on intra-UE multiplexing/prioritization B by May 18 – Jia (OPPO)

- Issue#1 HP PUCCH with HARQ-ACK and SR collides LP HARQ-ACK

- Issue#2 Multiplexing for SPS HARQ-ACK

- Issue#3 Interlace number adjustment for PUCCH

- Issue#4 Power control enhancement for PUCCH

- Issue#5 Clarification on PUCCH resouce determination in spec

- Issue#6 Insufficient resources of HP PUSCH multiplexed with LP HARQ-ACK

- Issue#7 Multiplexing of CG-UCI on PUSCH of a different priority

- Issue#8 Bitwidth of Beta-offset indicator

- Issue#10 Spec clarification reflecting the agreement on DG-CG PUSCH prioritization

- 1st check point on May 12 and final check point on May 18

- Remaining issues on multiplexing UCIs of different priorities in a PUCCH

Issue#1: HP PUCCH with HARQ-ACK and SR collides LP HARQ-ACK

Discussion status in last meeting:

When a PUCCH carrying explicit/implicit HP SR and HP HARQ-ACK with PUCCH format 0/1 overlaps with a PUCCH carrying LP HARQ-ACK, where the original PUCCH carrying the HP HARQ-ACK before Step 1 overlaps with K HP SRs which are all negative or include at least one positive,

- In Step 1, 1-bit HP SR is multiplexed to HP HARQ-ACK bits (based on Rel-15 rules) for determining the resultant PUCCH resource.

  - Note: The description of Step 1 here is only for information purpose, which has no spec impact.

- In Step 2, down-select from the two options:

  - Option 0: LP HARQ-ACK is dropped.

  - Option 2: 1-bit HP SR is appended to HP HARQ-ACK bits. The number of HP UCI bits is OUCI=OACK,1+1

    - The resultant PUCCH resource for multiplexing HP SR + HP HARQ-ACK + LP HARQ-ACK is either of PF2, PF3, or PF4.

HW proposal:

Proposal 2: When a PUCCH carrying HP SR and HP HARQ-ACK with PF0/1 overlaps with a PUCCH carrying LP HARQ-ACK:

- Option 2 is adopted, i.e., if HP HARQ-ACK is dynamic HARQ-ACK or SPS HARQ-ACK with sps-PUCCH-AN-List, 1-bit HP SR is appended to HP HARQ-ACK bits. The number of HP UCI bits is OUCI=OACK,1+1.

Proposal 3: For the case of HP SR with PF0 dropped due to collision with HP HARQ-ACK PF1 in Step 1, the resultant HP PUCCH after Step 1 is regarded as HP PUCCH with HARQ-ACK only when resolving the overlapping with LP HARQ-ACK in Step 2.

ZTE proposal:

Proposal 1: When a PUCCH carrying HP SR and HP HARQ-ACK with PUCCH format 0/1 overlaps with a PUCCH carrying LP HARQ-ACK, where the original PUCCH carrying the HP HARQ-ACK before Step 1 overlaps with K HP SRs,

  - If the total number of LP and HP HARQ-ACK bits is 2, the multiplexing follows the previous agreement for multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, i.e., treat the two bits as HARQ-ACK bits with high priority and the resultant PUCCH format is 0/1.

  - If the total number of LP and HP HARQ-ACK bits is larger than 2,

    - If the resultant PUCCH after multiplexing between HP SR and HP HARQ-ACK doesn't use the PUCCH resource with n1PUCCH-AN or SR resource with PF0/1, 1-bit HP SR is appended to HP HARQ-ACK bits. The number of HP UCI bits is OUCI=OACK,1+1. The resultant PUCCH resource for multiplexing HP SR + HP HARQ-ACK + LP HARQ-ACK is either of PF2, PF3, or PF4.

    - Otherwise, LP HARQ-ACK is dropped.

Nokia proposal:

Proposal 2.1: To resolve overlapping between a HP PUCCH carrying SR and HP HARQ-ACK with PUCCH format 0/1 and a LP PUCCH carrying HARQ-ACK and CSI/SR, only LP HARQ-ACK is multiplexed with HP UCI (i.e., SR and HARQ-ACK) and LP CSI/SR are dropped.

Proposal 3.1: When a PUCCH carrying explicit/implicit HP SR and HP HARQ-ACK with PUCCH format 0/1 overlaps with a PUCCH carrying LP HARQ-ACK, where the original PUCCH carrying the HP HARQ-ACK before Step 1 overlaps with K HP SRs which are all negative or include at least one positive,

- In Step 1, 1-bit HP SR is multiplexed to HP HARQ-ACK bits (based on Rel-15 rules) for determining the resultant PUCCH resource.

  - Note: The description of Step 1 here is only for information purpose, which has no spec impact.

- In Step 2,

  - if the HP HARQ-ACK is using a PUCCH resource provided by PUCCH-ResourceSet or SPS-PUCCH-AN-List, 1-bit HP SR is appended to HP HARQ-ACK bits. The number of HP UCI bits is OUCI=OACK,1+1

    - The resultant PUCCH resource for multiplexing HP SR + HP HARQ-ACK + LP HARQ-ACK is either of PF2, PF3, or PF4.

    - Reuse the (agreed) procedures for multiplexing of LP HARQ-ACK and HP HARQ-ACK on PUCCH resource with PF 2/3/4, i.e., PUCCH resource selection, separate coding, PRB determination, rate matching, power control, etc.

  - if HP HARQ-ACK is using a PUCCH resource provided by n1PUCCH-AN, the LP HARQ-ACK is dropped.

Spreadtrum proposal:

- Clarify that when a UE would transmit SR in a resource using PUCCH format 0 and HARQ-ACK information bits in a resource using PUCCH format 1 in a slot, the resultant PUCCH format 1 is regarded as negative SR or no SR.

E/// proposal:

- When a PUCCH carrying HP SR and HP HARQ-ACK with PF 0/1, the number of SR bits is K=1.

When a HP PUCCH carrying SR and HARQ-ACK with PF0/1 overlaps with a LP PUCCH carrying HARQ-ACK,

- 1-bit HP SR is appended to 1 or 2-bit HP HARQ-ACK, and treated together as a 2 or 3-bit HP HARQ-ACK;

- Reuse other procedures for multiplexing LP HARQ-ACK and HP HARQ-ACK onto a PUCCH resource with PF 2/3/4, i.e. separate coding, PRB determination, rate matching and power control.

- If the HP HARQ-ACK is a dynamic HARQ-ACK, a PUCCH resource indicated by PRI is used for multiplexing.

- If the HP HARQ-ACK is an SPS HARQ-ACK, a PUCCH resource determined from the PUCCH resource(s) provided by sps-PUCCH-AN-List is used for multiplexing.

When a HP PUCCH carrying SR and HARQ-ACK with PF0/1 overlaps with a LP PUCCH carrying HARQ-ACK and CSI/SR, only LP HARQ-ACK is multiplexed with HP UCI (i.e., SR and HARQ-ACK) and LP CSI/SR are dropped.

CATT proposal:

Proposal 1: If a PUCCH carrying HP SR and HP HARQ-ACK with PUCCH format 0/1 overlaps with a PUCCH carrying LP HARQ-ACK, LP HARQ-ACK is multiplexed with HP SR and HP HARQ-ACK where 1 bit HP SR is appended to HP HARQ-ACK bits and treated as HP UCI.

Proposal 2: If a PUCCH carrying HP SR and HP HARQ-ACK with PUCCH format 0/1 overlaps with a PUCCH carrying LP HARQ-ACK and LP CSI/SR, only LP HARQ-ACK is multiplexed with HP UCI and LP CSI/SR are dropped.

vivo proposal:

Proposal 1: When a PUCCH carrying explicit/implicit HP SR and HP HARQ-ACK with PUCCH format 0/1 overlaps with a PUCCH carrying LP HARQ-ACK, where the original PUCCH carrying the HP HARQ-ACK before Step 1 overlaps with K HP SRs which are all negative or include at least one positive,

- In Step 1, 1-bit HP SR is multiplexed to HP HARQ-ACK bits (based on Rel-15 rules) for determining the resultant PUCCH resource.

  - Note: The description of Step 1 here is only for information purpose, which has no spec impact.

- In Step 2,

  - Option 2a:

    - If HP HARQ-ACK is dynamic HARQ-ACK or SPS HARQ-ACK with SPS-PUCCH-AN-List, 1-bit HP SR is appended to HP HARQ-ACK bits. The number of HP UCI bits is OUCI=OACK,1+1

    - If HP HARQ-ACK is SPS HARQ-ACK with n1PUCCH-AN, LP HARQ-ACK is dropped.

Samsung proposal:

Proposal 1: For resolving collision of PUCCHs and/or PUSCHs of different priorities, Step 2 only considers the resulting PUCCHs and/or PUSCHs in Step 1.

Proposal 2: For resolving collision of PUCCHs of different priorities in step 2.1, if a LP PUCCH with LP HARQ-ACK overlaps with a HP PUCCH PF 0/1 with HP HARQ-ACK and positive HP SR, the LP PUCCH is dropped.

OPPO proposal:

Proposal 2: When a PUCCH carrying HP SR and HP HARQ-ACK with PUCCH format 0/1 overlaps with a PUCCH carrying LP HARQ-ACK, where the original PUCCH carrying the HP HARQ-ACK before Step 1 overlaps with K HP SRs, in step 2, 1-bit HP SR is appended to HP HARQ-ACK bits. The number of HP UCI bits is OUCI=OACK,1+1.

DOCOMO proposal:

- Summary of change

Add the corresponding changes to make the specification complete.

- Consequences if not approved

The spec is incomplete.

- Text proposal 1:

-------------------------- Start of Text Proposal for TS 38.213 -------------------------- <Unchanged parts omitted> 9 UE procedure for reporting control information <Unchanged parts omitted> if // this is for cases the UE supports multiplexing information of different priorities in a PUCCH/PUSCH transmission - a PUCCH transmission with HARQ-ACK information, without repetitions, with smaller priority index overlaps with a PUCCH transmission only with HARQ-ACK information, without repetitions, with larger priority index, or - a PUCCH transmission without repetitions that includes HARQ-ACK information of smaller priority index overlaps with a PUCCH transmission without repetitions using a PUCCH resource with PUCCH format 2/3/4 with that includes HARQ-ACK information and SR of larger priority index, or - a PUCCH transmission with HARQ-ACK information, without repetitions, with smaller or larger priority index overlaps, respectively, with a PUSCH transmission with larger or smaller priority index the UE - multiplexes HARQ-ACK information of different priority indexes and SR information of larger priority index, if any, in a same PUCCH transmission of larger priority index, or multiplexes HARQ-ACK information the UE would provide in a PUCCH transmission of smaller or larger priority index in a PUSCH transmission of larger or smaller priority index, respectively, and applies the procedures in clause 9.2.5.3 or 9.3, respectively, and - drops CSI and/or SR carried in the PUCCH of smaller priority index, if any - drops HARQ-ACK information of smaller priority index if the UE would multiplex the HARQ-ACK information of smaller priority index in a PUSCH where the UE multiplexes Part 1 CSI reports and Part 2 CSI reports of larger priority index - drops Part 2 CSI reports of smaller priority index if the UE would multiplex the HARQ-ACK information of smaller and larger priority indexes in a PUSCH where the UE multiplexes Part 1 CSI reports and Part 2 CSI reports of smaller priority index <Unchanged parts omitted> -------------------------- End of Text Proposal for TS 38.213 --------------------------

ITRI proposal:

Proposal 7:

To resolve overlapping between a HP PUCCH carrying SR and HP HARQ-ACK with PUCCH format 0/1 and a LP PUCCH, only LP HARQ-ACK is multiplexed with HP UCI. And the number of HP UCI bits is .

WILUS proposal:

- Proposal 1: For resolve overlapping of HP SR in a PF0/1 and LP HARQ-ACK/SR in PF0/1, the LP HARQ-ACK/SR in PF0/1 is dropped.

- Proposal 2: For resolve overlapping of HP HARQ-ACK/SR in a PF0/1 and LP HARQ-ACK, we propose to allow the multiplexing of HP HARQ-ACK/SR and LP HARQ-ACK in the resultant PUCCH resource for multiplexing of HP PUCCH and LP PUCCH by appending 1-bit HP SR to HP HARQ-ACK bits.

- Proposal 3: For resolve overlapping of HP HARQ-ACK/SR in a PF0/1 and LP HARQ-ACK/SR, we propose to allow the multiplexing of HP HARQ-ACK/SR and LP HARQ-ACK in the resultant PUCCH resource for multiplexing of HP PUCCH and LP PUCCH by appending only 1-bit HP SR to HP HARQ-ACK bits after dropping LP SR.

Proposal #1: Agree the following proposal made in RAN1#108-e by adopting Option 4 or Option 7 for Step 2 below.

When a PUCCH carrying HP SR and HP HARQ-ACK with PUCCH format 0/1 overlaps with a PUCCH carrying LP HARQ-ACK, where the original PUCCH carrying the HP HARQ-ACK before Step 1 overlaps with K HP SRs which are all negative or include at least one positive, - In Step 1, 1-bit HP SR is multiplexed to HP HARQ-ACK bits (based on Rel-15 rules) for determining the resultant PUCCH resource. - Note: The description of Step 1 here is only for information purpose, which has no spec impact. - In Step 2, adopt one of the following two options. - Option 4: - If there is 1-bit HP HARQ-ACK and 1 bit LP HARQ-ACK, - If HP SR and HP HARQ-ACK are on PUCCH format 1, LP HARQ-ACK bit can be simply treated as HP HARQ-ACK bit, and the two HP HARQ-ACK bits are multiplexed with the HP SR using Rel-16/Rel-15 rules. - If HP SR and HP HARQ-ACK are on PUCCH format 0, LP HARQ-ACK is dropped. - If at least one of the HP HARQ-ACK payload size or LP HARQ-ACK payload size is greater than or equal to 2, - If HP HARQ-ACK is dynamic HARQ-ACK or SPS HARQ-ACK with sps-PUCCH-AN-List, 1-bit HP SR is appended to HP HARQ-ACK bits. The number of HP UCI bits is OUCI=OACK,1+1 (i.e. OUCI=OACK,1+log2K+1). - If HP HARQ-ACK is SPS HARQ-ACK with n1PUCCH-AN, LP HARQ-ACK is dropped. - Option 7: - If there is 1-bit HP HARQ-ACK and 1 bit LP HARQ-ACK, LP HARQ-ACK is dropped. - If at least one of the HP HARQ-ACK payload size or LP HARQ-ACK payload size is greater than or equal to 2, - If HP HARQ-ACK is dynamic HARQ-ACK or SPS HARQ-ACK with sps-PUCCH-AN-List, 1-bit HP SR is appended to HP HARQ-ACK bits. The number of HP UCI bits is OUCI=OACK,1+1. - If HP HARQ-ACK is SPS HARQ-ACK with n1PUCCH-AN, LP HARQ-ACK is dropped.

Intel proposal:

Proposal 1: When a PUCCH carrying positive HP SR and HP HARQ-ACK by PUCCH format 0/1 overlaps with a PUCCH carrying LP HARQ-ACK, the PUCCH carrying LP HARQ-ACK is drpped.

Qualcomm proposal:

Proposal 4: When a HP positive SR in PUCCH format 0/1 overlaps with LP HARQ-ACK and SR in PUCCH format 0/1, LP HARQ-ACK and SR are dropped.

Proposal 5: When a PUCCH carrying HP SR and HP HARQ-ACK with PUCCH format 0/1 overlaps with a PUCCH carrying LP HARQ-ACK, option 2 is adopted.

- Option 2: 1-bit HP SR is appended to HP HARQ-ACK bits. The number of HP UCI bits is OUCI=OACK,1+1. The HP UCI is then multiplexed with LP HARQ-ACK

  - A UE expect that the resultant PUCCH resource for multiplexing HP SR + HP HARQ-ACK + LP HARQ-ACK is in PF2, PF3, or PF4.

Proposal 6: When a PUCCH carrying HP SR and HP HARQ-ACK with PUCCH format 0/1 overlaps with a PUCCH carrying LP HARQ-ACK and SR.

- 1-bit HP SR is appended to HP HARQ-ACK bits. The number of HP UCI bits is OUCI=OACK,1+1. The HP UCI is then multiplexed with LP HARQ-ACK with the LP SR dropped.

  - A UE expect that the resultant PUCCH resource for multiplexing HP SR + HP HARQ-ACK + LP HARQ-ACK is in PF2, PF3, or PF4.

    - 1st round discussion

Void (Wait for relative agreement in Intra-UE MUX-A).

    - 2nd round discussion

Void (Wait for relative agreement in Intra-UE MUX-A).

Issue#2: Multiplexing for SPS HARQ-ACK (HW, Samsung)

This issue was discussed in some company’ contribution in last meeting. In this meeting, more companies addressed this issue.

Status in running specification:

UE determines the PUCCH resource from the second sps-PUCCH-AN-List using OUCI=OACK,0+OACK,1

HW proposal:

Proposal 1: When a PUCCH carrying HP HARQ-ACK provided by n1PUCCH-AN overlaps with a PUCCH carrying LP HARQ-ACK with total number of HARQ-ACK bits more than 2 bits, the LP HARQ-ACK is dropped.

Samsung proposal:

Proposal 4: For resolving collision of two overlapping PUCCHs with different priorities in R17, if a HP PUCCH with SPS PDSCH HARQ-ACK overlaps with a LP PUCCH with HARQ-ACK and sps-PUCCH-AN-List is not provided in the second PUCCH-config, the LP PUCCH is dropped.

    - 1st round discussion

Proposal for 1st round discussion:

For resolving collision of two overlapping PUCCHs with different priorities in R17, if a HP PUCCH with SPS PDSCH HARQ-ACK overlaps with a LP PUCCH with HARQ-ACK and sps-PUCCH-AN-List is not provided in the second PUCCH-config, the LP PUCCH is dropped.

| Company | Comments |
| --- | --- |
| OPPO | Agree with proposal |
| ZTE | Agree |
| Intel | We agree with the proposal. |
| vivo | ok with the proposal. |
| ITRI | Support the proposal. |
| DOCOMO | Agree with the proposal. |
| CATT | It is not quite clear to us why gNB does not configure sps-PUCCH-AN-List for a UE configured with Rel-17 intra-UE multiplexing. But if majority companies see the need, we are fine with the proposal. |
| Huawei/Hisi | Support |
| New H3C | We are fine with this proposal |
| Spreadtrum | Support the proposal. |
| Nokia/NSB | Support the proposal. |
| LG | Support the proposal. |
| InterDigital | Support |
| Samsung | Support |
| QC | Same view as CATT, not sure why gNB would not configure sps-PUCCH-AN-List in this case. |

    - 2nd round discussion (for email approval)

All companies showed supports to this proposal in the 1st round. So we try email approval for it in the 2nd round.

Proposal for email approval:

For resolving collision of two overlapping PUCCHs with different priorities in R17, if a HP PUCCH with SPS PDSCH HARQ-ACK overlaps with a LP PUCCH with HARQ-ACK and sps-PUCCH-AN-List is not provided in the second PUCCH-config, the LP PUCCH is dropped.

| Supporting companies: | LG, ZTE, OPPO,vivo, ITRI, DOCOMO |
| --- | --- |
| Objecting companies: | QC |
| Company | Reason for objection |
| QC | We apologize for input our comment late. But for this one, it is quite puzzling that why gNB would not configure sps-PUCCH-AN-List, given everything is static for SPS. |
| LG | @QC: Considering at least R17 UEs without capability of sps-PUCCH-AN-List, the relevant UE behavior (as in the above proposal) needs to be specified. |
| ZTE | Understand the concern from QC, to solve QC’s concern, maybe alternative way like “When a PUCCH carrying HP SPS PDSCH HARQ-ACK overlaps with a PUCCH carrying LP HARQ-ACK with total number of HARQ-ACK bits more than 2 bits, it is expected the sps-PUCCH-AN-List should be configured” can be considered. |
| OPPO | Share view as LG. The related capability with sps-PUCCH-AN-List, i.e. FG12-2, is independent from R17 intra UE multiplexing. And there is no reason to always bundle these two features. |
| Ericsson | We are fine with the proposal in principle. On the other hand, our understanding is, this refers to HP PUCCH with 1 bit SPS HARQ-ACK only, where the PUCCH resource is provided by n1PUCCH-AN. - That is, an HP PUCCH may carry dynamic HARQ-ACK with the SPS HARQ-ACK appended, where the PUCCH resource is provided by PRI. For this case, the proposal does not apply. Recommend adding ‘only’ and/or adding ‘PUCCH resource is provided by n1PUCCH-AN’: For resolving collision of two overlapping PUCCHs with different priorities in R17, if a HP PUCCH with SPS PDSCH HARQ-ACK only overlaps with a LP PUCCH with HARQ-ACK and sps-PUCCH-AN-List is not provided in the second PUCCH-config, the LP PUCCH is dropped. |
| QC2 | @LG: if a UE does not support sps-PUCCH-AN-List, which means it can only support one DL SPS per PHY priority. Then there is only 1-bit HP SPS A/N and 1-bit LP SPS A/N. When they overlap, the total payload is 2 bits and can be transmit by n1-SPS-AN resource. I don’t see why LP SPS A/N needs to be dropped. @all, another comment is that if the LP PUCCH with HARQ-ACK is dynamically scheduled A/N, it is an error case in Rel-16 and in current Rel-17 spec, if the DG LP A/N overlap with a HP SPS A/N. The proposal would turn it into a valid case and define UE behaviour for it, which we cannot agree on. |

Issue#3: Interlace number adjustment for PUCCH (Apple)

Apple proposal:

Proposal 3-1: If a UE is provided a first interlace of MInterlace,0PUCCH PRBs by interlace0 in InterlaceAllocation, the UE has HARQ-ACK, SR and wideband or sub-band CSI reports to transmit, and the UE determines a PUCCH resource with PUCCH format 2, or the UE has HARQ-ACK, SR and wideband CSI reports to transmit and the UE determines a PUCCH resource with PUCCH format 3, or the UE has HARQ-ACK, SR and sub-band CSI reports to transmit, and the UE determines a PUCCH resource with PUCCH format 3, where

- the UE determines the PUCCH resource using the PUCCH resource indicator field in a last of a number of DCI formats with a value of a PDSCH-to-HARQ_feedback timing indicator field indicating a same slot for the PUCCH transmission, from a PUCCH resource set provided to the UE for HARQ-ACK transmission, and after the UE determines the PUCCH resource set

- if

n=1NUCI-part2totalOUCI-part2,n+OCRC,UCI-part2/Qm⋅r2+⌈n=1NUCI-part1totalOUCI-part1,n+OCRC,UCI-part1/Qm⋅r1⌉≤MInterlace,0PUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH,

the UE transmits the HARQ-ACK, SR, and CSI reports bits in a PUCCH over the first interlace

- else, if the UE is provided a second interlace of MInterlace,1PUCCH PRBs by interlace1 and if

n=1NUCI-part2totalOUCI-part2,n+OCRC,UCI-part2/Qm⋅r2+⌈n=1NUCI-part1totalOUCI-part1,n+OCRC,UCI-part1/Qm⋅r1⌉≤MInterlace,0PUCCH+MInterlace,1PUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH,

the UE transmits the HARQ-ACK, SR, and CSI reports bits in a PUCCH over both the first and second interlaces

- else, the UCI omission procedure is same as the corresponding one when the UE is provided PUCCH-ResourceSet by replacing MRBPUCCH with MInterlace,0PUCCH, or, if the UE is provided interlace1, by MInterlace,0PUCCH+MInterlace,1PUCCH.

    - 1st round discussion

Proposal for 1st round discussion:

If a UE is provided a first interlace of MInterlace,0PUCCH PRBs by interlace0 in InterlaceAllocation, the UE has HARQ-ACK, SR and wideband or sub-band CSI reports to transmit, and the UE determines a PUCCH resource with PUCCH format 2, or the UE has HARQ-ACK, SR and wideband CSI reports to transmit and the UE determines a PUCCH resource with PUCCH format 3, or the UE has HARQ-ACK, SR and sub-band CSI reports to transmit, and the UE determines a PUCCH resource with PUCCH format 3, where

- the UE determines the PUCCH resource using the PUCCH resource indicator field in a last of a number of DCI formats with a value of a PDSCH-to-HARQ_feedback timing indicator field indicating a same slot for the PUCCH transmission, from a PUCCH resource set provided to the UE for HARQ-ACK transmission, and after the UE determines the PUCCH resource set

- if

n=1NUCI-part2totalOUCI-part2,n+OCRC,UCI-part2/Qm⋅r2+⌈n=1NUCI-part1totalOUCI-part1,n+OCRC,UCI-part1/Qm⋅r1⌉≤MInterlace,0PUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH,

the UE transmits the HARQ-ACK, SR, and CSI reports bits in a PUCCH over the first interlace

- else, if the UE is provided a second interlace of MInterlace,1PUCCH PRBs by interlace1 and if

n=1NUCI-part2totalOUCI-part2,n+OCRC,UCI-part2/Qm⋅r2+⌈n=1NUCI-part1totalOUCI-part1,n+OCRC,UCI-part1/Qm⋅r1⌉≤MInterlace,0PUCCH+MInterlace,1PUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH,

the UE transmits the HARQ-ACK, SR, and CSI reports bits in a PUCCH over both the first and second interlaces

- else, the UCI omission procedure is same as the corresponding one when the UE is provided PUCCH-ResourceSet by replacing MRBPUCCH with MInterlace,0PUCCH, or, if the UE is provided interlace1, by MInterlace,0PUCCH+MInterlace,1PUCCH.

| Company | Comments |
| --- | --- |
| OPPO | Before we go to details, we’d better to clarify whether to support R17 intra UE multiplexing in unlicensed band. |
| Apple | The support of CG-UCI multiplexing with HP/LP in R17 was agreed in the unlicensed band agreement. So our understanding R17 intra UE multiplexing unlicensed band is supported. |
| Intel | We share same understanding with Apple that R17 intra UE multiplexing is supported in unlicensed band. The group for Rel-17 URLLC over unlicensed band already discussed and agreed to support CG-UCI multiplexing for R17 intra UE multiplexing. For unlicensed band, at least for regions with PSD and OCB requirement, interlace based PUSCH is needed. Therefore, we tend to agree with Apple that extension of PRB determination to interlace determination is desirable. |
| vivo | We agree that R17 intra UE multiplexing should be supported in unlicensed band and it is needed to extend PRB determination to interlace determination. |
| DOCOMO | Share the same understanding that CG-UCI multiplexing of different priorities is supported in Rel-17 intra-UE multiplexing, and we are fine with the proposal. |
| Nokia/NSB | Since the support of joint operation was agreed under Unlicensed Band, then probably better to discuss any open points regarding this joint operation there. If the majority prefers to discuss this issue here, we are fine with the intention of the proposal. |
| Samsung | Share similar view as Nokia. |

    - 2nd round discussion (for email approval)

It has been agreed to support CG-UCI multiplexing of different priorities in the unlicensed band agreement in Rel-17. So the relative remaining issues can be discussed. Here companies can first check the following proposal for email approval.

Proposal for email approval:

If a UE is provided a first interlace of MInterlace,0PUCCH PRBs by interlace0 in InterlaceAllocation, the UE has HARQ-ACK, SR and wideband or sub-band CSI reports to transmit, and the UE determines a PUCCH resource with PUCCH format 2, or the UE has HARQ-ACK, SR and wideband CSI reports to transmit and the UE determines a PUCCH resource with PUCCH format 3, or the UE has HARQ-ACK, SR and sub-band CSI reports to transmit, and the UE determines a PUCCH resource with PUCCH format 3, where

- the UE determines the PUCCH resource using the PUCCH resource indicator field in a last of a number of DCI formats with a value of a PDSCH-to-HARQ_feedback timing indicator field indicating a same slot for the PUCCH transmission, from a PUCCH resource set provided to the UE for HARQ-ACK transmission, and after the UE determines the PUCCH resource set

- if

n=1NUCI-part2totalOUCI-part2,n+OCRC,UCI-part2/Qm⋅r2+⌈n=1NUCI-part1totalOUCI-part1,n+OCRC,UCI-part1/Qm⋅r1⌉≤MInterlace,0PUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH,

the UE transmits the HARQ-ACK, SR, and CSI reports bits in a PUCCH over the first interlace

- else, if the UE is provided a second interlace of MInterlace,1PUCCH PRBs by interlace1 and if

n=1NUCI-part2totalOUCI-part2,n+OCRC,UCI-part2/Qm⋅r2+⌈n=1NUCI-part1totalOUCI-part1,n+OCRC,UCI-part1/Qm⋅r1⌉≤MInterlace,0PUCCH+MInterlace,1PUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH,

the UE transmits the HARQ-ACK, SR, and CSI reports bits in a PUCCH over both the first and second interlaces

- else, the UCI omission procedure is same as the corresponding one when the UE is provided PUCCH-ResourceSet by replacing MRBPUCCH with MInterlace,0PUCCH, or, if the UE is provided interlace1, by MInterlace,0PUCCH+MInterlace,1PUCCH.

Note: It has been agreed to support CG-UCI multiplexing of different priorities in the unlicensed band agreement in Rel-17.

| Supporting companies: |  |
| --- | --- |
| Objecting companies: | QC (not real object. But we need see a real TP before we can agree on this), Nokia/NSB (same as QC) |
| Company | Reason for objection |
| QC | Again, we apologize for our late input. We actually are not against the principle/intention of this proposal. But there a lot of details in this proposal. This proposal is written like a TP but not following TP format. Can the proponents of this proposal please kindly provide exact TP to show what are the changes on top of current spec? For example, in current spec, there is no definition of NUCI-part2total, nor OUCI-part2,n, nor UCI-part2, nor NUCI-part1total, nor OUCI-part1,n, nor UCI-part1. I am not sure how editor can put the text and equations in this proposal together with current spec. Another minor question: in current spec, for the last else, it is captured as “else, the procedure is same as the corresponding one…”, why we want to say “UCI omission procedure” in this TP? |
| Nokia/NSB | Agree with QC. If this is such a long & very specific ‘agreement’ why not discussing & agreeing on a TP directly (especially as it seems to be written as a ‘pseudo TP’ already. |
| Ericsson | Agree with QC and Nokia that it’s better to work on TP text directly. Also: The formulation should follow the existing way which has been adopted earlier in the spec. In the current spec the similar formulas are multiplied by Qm * r1 * r2 to avoid divisions. Then, there seems to be no need for the ceiling (as in the spec), which is different from the proposal. |
| Apple | @QC, @Nokia, @Ericsson @FL, at RAN1 106-e agreement, we have the following Agreement For determining the PUCCH resource to carry the multiplexed high-priority and low-priority HARQ-ACKs, - The number of RBs for multiplexing HP HARQ-ACK and LP HARQ-ACK on a PUCCH format 3 is determined as following: - If OHP_UCI+OCRC, HP_UCIrHP_UCI∙Qm+OLP_UCI+OCRC, LP_UCIrLP_UCI∙Qm ≤MRBPUCCH∙Nsc, ctrlRB∙Nsymb-UCIPUCCH, the minimum number of RBs is determined as the number of MRB, minPUCCH, satisfying OHP_UCI+OCRC, HP_UCIrHP_UCI∙Qm+OLP_UCI+OCRC, LP_UCIrLP_UCI∙Qm≤MRB,minPUCCH∙Nsc, ctrlRB∙Nsymb-UCIPUCCH and OHP_UCI+OCRC, HP_UCIrHP_UCI∙Qm+OLP_UCI+OCRC, LP_UCIrLP_UCI∙Qm>MRB,minPUCCH-1∙Nsc, ctrlRB∙Nsymb-UCIPUCCH. - Note: rHP_UCI∙rLP_UCI∙Qm is multiplied at both sides to avoid mismatch between gNB and UE due to floating point operation. Editor to capture as suggested. - Otherwise, - Alt1: the number of RBs is MRBPUCCH. FFS: Whether/How LP HARQ-ACK is dropped. - Alt2: the number of RBs is determined by HP ACK payload size. LP HARQ-ACK is fully dropped. - Other alternatives are not precluded. - r_HP_UCI is maxCodeRate configured for HP bits and r_LP_UCI is maxCodeRate configured for LP bits in the second PUCCH-Config (the PUCCH-config containing the PUCCH resource of the HP HARQ-ACK). - FFS whether more than one maxCodeRate can be configured for one priority. - If MRB, minPUCCH is not equal to according to [4, TS 38.211], MRB, minPUCCH is increased to the nearest allowed value of nrofPRBs for PUCCH-format3 provided by the second PUCCH-Config [12, TS 38.331]. - HP coded bits and LP coded bits are not transmitted using the same RE(s) - FFS for PUCCH format 2. It make it simpler for companies to review the proposal, and also take into consideration ERcisson’s comment’s on ceiling, can we check the proposal below (detla is marked in red for interlace ? Hope the formulation is easy to follow. Thanks. Proposal for interlace number adjustment: If the UE is provided a first interlace of MInterlace,0PUCCH PRBs by interlace0 and a second interlace of MInterlace,1PUCCH PRBs by interlace1: For determining the PUCCH resource to carry the multiplexed high-priority and low-priority HARQ-ACKs, - The number of interlaces for multiplexing HP HARQ-ACK and LP HARQ-ACK on a PUCCH format 2/3 is determined as following: - If OHP_UCI+OCRC, HP_UCIrHP_UCI∙Qm+OLP_UCI+OCRC, LP_UCIrLP_UCI∙Qm ≤MInterlace,0PUCCH ∙Nsc, ctrlRB∙Nsymb-UCIPUCCH, then the UE transmits the UCIs in a PUCCH over the first interlace; otherwise the UE transmits the UCIs in a PUCCH over both interlaces. Note: rHP_UCI∙rLP_UCI∙Qm is multiplied at both sides to avoid mismatch between gNB and UE due to floating point operation. Editor to capture as suggested. - r_HP_UCI is maxCodeRate configured for HP bits and r_LP_UCI is maxCodeRate configured for LP bits in the second PUCCH-Config (the PUCCH-config containing the PUCCH resource of the HP HARQ-ACK). |

Issue#4: Power control enhancement for PUCCH (Apple)

Apple proposal:

Proposal 5-1:  For PUCCH formats 2/3/4, the delta factor ΔTF,b,f,ci is determined from UCI part 1:

- The number of resource elements for UCI part 1 NRE-part1(i)=EUCI-part1EtotNRE(i),  where   EUCI-part1 is the number of coded bits for UCI part 1

- If  n=1NUCI-part1total‍OUCI-part1,n  is smaller or equal to 11,

  - ΔTF,b,f,c,part1i=10⋅log10(K1⋅n=1NUCI-part1total‍OUCI-part1,n/NRE-part1i)

  - If a HARQ-ACK codebook with OUCI-part1,n'  bits is included in UCI part 1,  is used instead of OUCI-part1,n'  for the HARQ-ACK codebook:

    - ΔTF,b,f,c,part1i=10⋅log10(K1⋅n=1NUCI-part1total‍OUCI-part1,n-OUCI-part1,n' +nHARQ-ACK(i)/NRE-part1i)

  - If more than one HARQ-ACK codebooks are included in UCI part 1 (e.g. one due to SPS HARQ deferral, another for HARQ feedback for dynamic grant PDSCH(s)), then replacement of the number of HARQ-ACK codebook size by the associated nHARQ-ACK can be applied to each HARQ-ACK codebook.

- otherwise

  - ΔTF,b,f,c,part1i=10log10(2K2⋅BPREi-1)  where

    - and

    - BPREi=n=1NUCI-part1total‍OUCI-part1,n+OCRC,UCI-part1/NRE-part1i

- And ΔTF,b,f,ci = ΔTF,b,f,c,part1i is applied to both UCI parts.

Proposal 5-2: the issue for PUCCH format 4 needs to be tackled. Proposal 5-1 above provides a solution for that.

Proposal 5-3: For Rel-17 intra-UE multiplexing, retain the difference between  and OUCI,1 .

    - 1st round discussion

Proposal for 1st round discussion:

For PUCCH formats 2/3/4, the delta factor ΔTF,b,f,ci is determined from UCI part 1:

- The number of resource elements for UCI part 1 NRE-part1(i)=EUCI-part1EtotNRE(i),  where   EUCI-part1 is the number of coded bits for UCI part 1

- If  n=1NUCI-part1total‍OUCI-part1,n  is smaller or equal to 11,

  - ΔTF,b,f,c,part1i=10⋅log10(K1⋅n=1NUCI-part1total‍OUCI-part1,n/NRE-part1i)

  - If a HARQ-ACK codebook with OUCI-part1,n'  bits is included in UCI part 1,  is used instead of OUCI-part1,n'  for the HARQ-ACK codebook:

    - ΔTF,b,f,c,part1i=10⋅log10(K1⋅n=1NUCI-part1total‍OUCI-part1,n-OUCI-part1,n' +nHARQ-ACK(i)/NRE-part1i)

  - If more than one HARQ-ACK codebooks are included in UCI part 1 (e.g. one due to SPS HARQ deferral, another for HARQ feedback for dynamic grant PDSCH(s)), then replacement of the number of HARQ-ACK codebook size by the associated nHARQ-ACK can be applied to each HARQ-ACK codebook.

- otherwise

  - ΔTF,b,f,c,part1i=10log10(2K2⋅BPREi-1)  where

    - and

    - BPREi=n=1NUCI-part1total‍OUCI-part1,n+OCRC,UCI-part1/NRE-part1i

- And ΔTF,b,f,ci = ΔTF,b,f,c,part1i is applied to both UCI parts.

| Company | Comments |
| --- | --- |
| Apple | There are two issues: 1) the difference between and O_UCI should be reflected 2). For PUCCH format 4, the calculation of REs needs to be corrected. |
| vivo | Before we go to agree the proposal, we’d better to clarify what’s the difference between this proposal and current specification. We already agreed in previous meeting to calculate the delta factor ΔTF,b,f,ci based on high priority UCI. |
| Huawei/Hisi | Not clear about the intention. Is it a new proposal or a TP? |
| Nokia/NSB | We have a similar comment as vivo. |
| Samsung | The issue was discussed before. We don’t we should reopen the discussion and revert previous agreement. |
| Ericsson | It seems like the proposal addresses two different issues. The first is the difference between n_HARQ and O_ACK that is used for power control. The other is the number of REs for PF4. It seems reasonable to discuss these issues separately in two different proposals. In principle we agree with the intention of the proposal. |

    - 2nd round discussion

In the first round, companies asked:

- What’s the difference between this proposal and current specification? We already agreed in previous meeting to calculate the delta factor ΔTF,b,f,ci based on high priority UCI.

Apple is suggested to provide further explanation for the proposal in the 2nd round. Then other companies can further check their positions.

| Company | Comments |
| --- | --- |
| Apple | First we are not to revert a previous agreement. We agreed the calculation is according to HP UCI, yet due to the way that is captured in the specification, two issues arise: - Note for Type 1 codebook, when there is a small number of PDSCHs received by the UE, most of the entries in the Type 1 codebook are populated with NACKs as they don’t correspond to actual PDSCH reception. That is why in Rel-15, the parameter n_HARQ was introduced. In this case, the HP HARQ-ACK’s n_HARQ should be used. - The other one is the number of REs for PF4, the treatment following the specification text is incorrect. |

Issue#5: Clarification on PUCCH resouce determination in spec (Qualcomm)

Qualcomm proposal:

Proposal 1: for multiplexing HP HARQ-ACKs and LP HARQ-ACKs, it is confirmed that current specification is interpreted as the following.

- The PRI in the lastly received DCI (if exist), which schedules a HP HARQ-ACK involved in the multiplexing, is used to select the PUCCH resource to transmit the multiplexed UCI payload.

- Note: no spec update is needed

    - 1st round discussion

Proposal for 1st round discussion:

For multiplexing HP HARQ-ACKs and LP HARQ-ACKs, it is confirmed that current specification is interpreted as the following.

- The PRI in the lastly received DCI (if exist), which schedules a HP HARQ-ACK involved in the multiplexing, is used to select the PUCCH resource to transmit the multiplexed UCI payload.

- Note: No spec update is needed

| Company | Comments |
| --- | --- |
| OPPO | Agree with proposal |
| Sony | Agree |
| Apple | Agree |
| ZTE | Agree |
| Intel | We agree with the proposal |
| vivo | Agree |
| ITRI | Support the proposal. |
| DOCOMO | Agree |
| CATT | Agree |
| Huawei/Hisi | Support |
| New H3C | support |
| Spreadtrum | Support the proposal. |
| Nokia/NSB | Support |
| LG | Support |
| InterDigital | Support |
| Samsung | Agree |

    - 2nd round discussion (for email approval)

All companies showed supports to this proposal in the 1st round. So we try email approval for it in the 2nd round.

Proposal for email approval:

For multiplexing HP HARQ-ACKs and LP HARQ-ACKs, it is confirmed that current specification is interpreted as the following.

- The PRI in the lastly received DCI (if exist), which schedules a HP HARQ-ACK involved in the multiplexing, is used to select the PUCCH resource to transmit the multiplexed UCI payload.

- Note: No spec update is needed

| Supporting companies: | QC, LG, ZTE, Nokia/NSB (as a conclusion), OPPO,vivo, ITRI, Sony, Ericsson, DOCOMO |
| --- | --- |
| Objecting companies: |  |
| Company | Reason for objection |
| Nokia/NSB | Should be only a conclusion as (a) we only ‘confirm’ and (b) no specs impact. |

- Remaining issues on multiplexing UCIs of different priorities in a PUCCH

Issue#6: Insufficient resources of HP PUSCH multiplexed with LP HARQ-ACK

Discussion status in last meeting:

For the scenarios where multiplexing low-priority HARQ-ACK onto high-priority PUSCH, down-select from the options:

  - Option 1: In case of insufficient resource for LP HARQ-ACK, LP HARQ-ACK is entirely dropped

  - Option 2: LP HARQ-ACKs are mapped to the rest REs of the PUSCH based on the rate matching equation, if HP HARQ-ACK and/or HP CSI have been mapped in prior on the PUSCH.

  - Option 3: UE does not expect insufficient resource for multiplexing low-priority HARQ-ACK.

HW proposal:

Proposal 4: For the scenario of multiplexing low-priority HARQ-ACK onto high-priority PUSCH, support Option 2, i.e. LP HARQ-ACKs are mapped to the rest REs of the PUSCH based on the rate matching equation, if HP HARQ-ACK and/or HP CSI have been mapped in prior on the PUSCH.

ZTE proposal:

Proposal 4: LP HARQ-ACKs are mapped to the rest REs of the PUSCH based on the rate matching equation, if HP HARQ-ACK and/or HP CSI have been mapped in prior on the PUSCH.

Spreadtrum proposal:

- For the scenarios where multiplexing low-priority HARQ-ACK onto high-priority PUSCH using rate matching and RE mapping of CSI part 2,

  - Option 2: LP HARQ-ACKs are mapped to the rest REs of the PUSCH based on the rate matching equation, if HP HARQ-ACK and/or HP CSI have been mapped in prior on the PUSCH.

CATT proposal:

Proposal 3: If the amount of REs available for LP HARQ-ACK mapping on the HP PUSCH is not sufficient to satisfy the coding rate of the LP HARQ-ACK based on the corresponding beta offset, LP HARQ-ACK is mapped to the rest REs after mapping HP HARQ-ACK or HP CSI .

Samsung proposal:

Proposed conclusion: When multiplexing a LP HARQ-ACK in a HP PUSCH, LP HARQ-ACKs are mapped to the rest of PUSCH REs based on the rate matching equation, if HP HARQ-ACK and/or HP CSI, if any, were mapped on the PUSCH.

OPPO proposal:

Proposal 3: In case of insufficient resource for LP HARQ-ACK, LP HARQ-ACK is entirely dropped.

InterDigital proposal:

Proposal 3: In case HARQ-ACK of priority 0 would be multiplexed in PUSCH of priority 1 and insufficient resource is available to accommodate HARQ-ACK, the HARQ-ACK is dropped.

    - 1st round discussion

Proposal for 1st round discussion:

For the scenarios where multiplexing low-priority HARQ-ACK onto high-priority PUSCH, down-select from the options:

  - Option 1: In case of insufficient resource for LP HARQ-ACK, LP HARQ-ACK is entirely dropped

  - Option 2: LP HARQ-ACKs are mapped to the rest REs of the PUSCH based on the rate matching equation, if HP HARQ-ACK and/or HP CSI have been mapped in prior on the PUSCH.

  - Option 3: UE does not expect insufficient resource for multiplexing low-priority HARQ-ACK.

| Option 1 | OPPO, Sony, Intel, ITRI, LG, InterDigital, Nokia/NSB (can accept) |
| --- | --- |
| Option 2 | ZTE, Intel (can live with), vivo, DOCOMO, CATT, Huawei/Hisi, New H3C, Spreadtrum, Samsung, QC |
| Option 3 | Nokia/NSB |

| Company | Comments |
| --- | --- |
| OPPO | Agree with proposal and prefer to option 1 |
| Sony | Option 1 |
| ZTE | Option 2. This option is more natural to fully use the rest REs and is similar as the multiplexing in PUCCH. |
| Intel | We prefer option 1. We can live with option 2. |
| vivo | Option 2. Same rule as multiplexing on PUCCH. |
| ITRI | Prefer option 1. |
| DOCOMO | Option 2 is preferred. |
| CATT | We support Option 2 which is the same as for PUCCH. |
| Huawei/Hisi | Option 2 with the same rule as PUCCH. Basically we may not need to do over-optimization with either more gNB burden (as Option 3) or UE burden (as Option 1) for such corner case. |
| New H3C | We are fine with Option 2. |
| Spreadtrum | Option 2, with unified solution of PUCCH |
| Nokia, NSB | Option 3 is preferred. If Option 3 is not agreeable, then we can be fine with Option 1. |
| LG | Option 1 is preferred since LP HARQ-ACK multiplexing would be useless due to use of small scaling factor (alpha) for HP PUSCH. |
| InterDigital | Option1. The situation does not seem the same as for PUCCH since here the HP PUSCH would be impacted? |
| Samsung | Option 2. In our understanding, Option 2 is the current behavior according to TS 38.212 h10. Option 1 is optimization and it is not essential for the CR phase. Option 3 has restriction on scheduling and is not preferred. In addition, similar issue was discussed for PUCCH and Option 2 is adopted, same rule should apply to PUCCH and PUSCH. |
| Ericsson | We prefer option 1. There are issues with using the remaining REs for mapping the LP UCI. The biggest concern is, there are error cases introduced by too high code rates, e.g. code rate higher than 1, for LP UCI when the number of REs available is too small. This is not possible to encode with the current spec. The other issues is, it is not useful to transmit information with too high code rate that the gNB is not able to decode, when using arbitrary remaining REs. This is wasting energy and these REs could be better used to transmit useful PUSCH data. |

    - 2nd round discussion

Void (The proposal will be submitted to GTW session).

Issue#7: Multiplexing of CG-UCI on PUSCH of a different priority (HW, ZTE, New H3C, Nokia, Spreadtrum, E///, Apple, DCM, LG, Intel, Qualcomm)

Agreement

If LP HARQ-ACK without HP HARQ-ACK would be transmitted on HP PUSCH, down-select from the options:

- Option 2: UE follows the same behaviour as that in case of PUSCH with HP HARQ-ACK assuming two bits

- FFS for CG-UCI PUSCH.

HW proposal:

Proposal 5: Adopt the following rules for multiplexing CG-UCI with HARQ-ACK of a different priority:

- For CG-UCI on HP PUSCH,

- LP HARQ-ACK without HP HARQ-ACK: CG-UCI is regarded as HP HARQ-ACK to reuse the rule of HP HARQ-ACK and LP HARQ-ACK on HP PUSCH that has been agreed.

- LP HARQ-ACK with HP HARQ-ACK: CG-UCI is jointly encoded with HP HARQ-ACK to reuse the rule of HP HARQ-ACK and LP HARQ-ACK on HP PUSCH that has been agreed.

- For CG-UCI on LP PUSCH,

- HP HARQ-ACK without LP HARQ-ACK: CG-UCI is regarded as LP HARQ-ACK to reuse the rule of HP HARQ-ACK and LP HARQ-ACK on LP PUSCH that has been agreed.

- HP HARQ-ACK with LP HARQ-ACK: CG-UCI is jointly encoded with LP HARQ-ACK to reuse the rule of HP HARQ-ACK and LP HARQ-ACK on LP PUSCH that has been agreed.

ZTE proposal:

Proposal 2: For HARQ-ACK multiplex on the PUSCH with CG UCI, if the CG-UCI could be concatenated with the HARQ-ACK with the same priority, the resultant concatenated UCI should be treated as HARQ-ACK to be multiplexed on the PUSCH by legacy rules.

Proposal 3: For HARQ-ACK multiplex on the PUSCH with CG UCI, if the CG-UCI could not be concatenated with the HARQ-ACK with the same priority, the CG-UCI should be treated as HARQ-ACK to be multiplexed on the PUSCH by legacy rules.

New H3C proposal:

Proposal 1: If LP HARQ-ACK with CG-UCI would be transmitted on HP PUSCH, CG-UCI and LP HARQ-ACK should be multiplexed together based on current legacy scheme as mentioned in section 6.3.2 of TS 38.212.

Nokia proposal:

Proposal 3.2: For multiplexing a LP HARQ-ACK without HP HARQ-ACK into a HP CG PUSCH carrying CG-UCI, UE follows the same behaviour as in case of PUSCH with HP HARQ-ACK assuming two bits.

- If the assumed (2-bit) HP HARQ-ACK, LP HARQ-ACK, HP CSI including a single part, and CG-UCI would be transmitted on the HP CG PUSCH,

  - Reuse Rel-16 procedure for handing HP HARQ-ACK and CG-UCI: joint coding, rate matching and RE mapping for jointly coded HP HARQ-ACK and CG-UCI.

  - Reuse Rel-15 CSI part 1 rate matching and RE mapping for the single part of HP CSI.

  - Reuse Rel-15 CSI part 2 rate matching and RE mapping for LP HARQ-ACK.

- If the assumed (2-bit) HP HARQ-ACK, LP HARQ-ACK and HP CSI including two parts would be transmitted on the HP CG PUSCH,

  - LP HARQ-ACK is dropped.

  - Reuse Rel-16 procedure for handling HP HARQ-ACK, CG-UCI and two parts HP CSI on HP CG PUSCH.

Spreadtrum proposal:

- CG-UCI is with the same priority of the CG-PUSCH.

- If there are same priority of CG-UCI and HARQ-ACK, Rel-16 NR-U joint coding of HARQ-ACK and CG-UCI is done in the step 1, and then UCI types with different priorities colliding with HARQ-ACK and CG-UCI on PUSCH are done according to Rel-17 intra-UE multiplexing rules.

- If there are no same priority of CG-UCI and HARQ-ACK, the rate matching/puncturing and RE mapping listed in Table 2 is applied.

  - If LP HARQ-ACK would be transmitted on HP PUSCH with CG-UCI, CG-UCI is treated as HP HARQ-ACK assuming the existence, i.e. by reusing the rate matching/puncturing and RE mapping for HP HARQ-ACK.

  - If HP HARQ-ACK would be transmitted on LP PUSCH with CG-UCI, CG-UCI is treated as LP HARQ-ACK assuming the existence, i.e. by reusing the rate matching/puncturing and RE mapping for LP HARQ-ACK.

E/// proposal:

When a HP (or LP) CG-PUSCH carrying CG-UCI overlaps with a LP (or HP) PUCCH carrying HARQ-ACK, treat the CG-PUSCH with CG-UCI as a CG-PUSCH with a HARQ-ACK sequence of the same priority. Then apply the existing procedure for multiplexing a HP (or LP) PUSCH carrying HARQ-ACK with a LP (or HP) PUCCH carrying HARQ-ACK, including the procedure for handling CSI/SR (if any).

Apple proposal:

- Observation 4: agreement from Rel-17 IIoT/URLLC unlicensed spectrum access session is only about the support of UCI types in CG PUSCH, not about how that is supported, which belongs to the intra-UE multiplexing discussion.

- Proposal 4: For Rel-17 intra-UE multiplexing, reuse R15 HARQ-ACK rate matching/puncturing and RE mapping for CG-UCI (in the absence of HP HARQ-ACK) or CG-UCI/HP-HARQ-ACK (in the presence of HP HARQ-ACK). LP-HARQ-ACK is treated as if CG-UCI is absent.

DOCOMO proposal:

- Summary of change

Add the corresponding changes to make the specification complete.

- Consequences if not approved

The spec is incomplete.

- Text proposal 2:

-------------------------- Start of Text Proposal for TS 38.212 -------------------------- <Unchanged parts omitted> 6.3.2.1.5 UCI with different priority indexes If UCI-MuxWithDifferentPriority is configured, and HARQ-ACK bits associated with priority index 0, and CSI part 1 if any are transmitted on a PUSCH associated with priority index 1, the following UCI bit sequences are generated, a0(1), a1(1), a2(1), a3(1), …,aA(1)-11, and a0(2), a1(2), a2(2), a3(2), …,aA(2)-12 if any, according to the following: - If CSI part 1 is also transmitted on the PUSCH, - Set ai(1) for i=0,1,…,A(1)-1 as the bit sequence of CSI part 1, where the CSI fields of all CSI reports, in the order from upper part to lower part in Table 6.3.2.1.2-6, are mapped to the UCI bit sequence a0(1), a1(1), a2(1), a3(1), …,aA(1)-11 starting with a0(1). - Set ai(2)=oiACK-LP for i=0,1,…,OACK-LP-1 and A(2)=OACK-LP, where the HARQ-ACK bit sequence o0ACK-LP, o1ACK-LP, …, oOACK-LP-1ACK-LP associated with priority index 0 is given by Clause 9.1 of [5, TS 38.213]. - Otherwise, set ai(1)=oiACK-LP for i=0,1,…,OACK-LP-1 and A(1)=OACK-LP, where the HARQ-ACK bit sequence o0ACK-LP, o1ACK-LP, …, oOACK-LP-1ACK-LP associated with priority index 0 is given by Clause 9.1 of [5, TS 38.213]. If UCI-MuxWithDifferentPriority and cg-UCI-Multiplexing are configured, and HARQ-ACK bits associated with priority index 0, and CG-UCI are transmitted on a PUSCH associated with priority index 1, the following UCI bit sequences are generated, a0(1), a1(1), a2(1), a3(1), …,aA(1)-11, and a0(2), a1(2), a2(2), a3(2), …,aA(2)-12 if any, according to the following: - Set ai(1)=oiCG-UCI for i=0,1,…,OCG-UCI-1 and A(1)=OCG-UCI, where the CG-UCI bit sequence o0CG-UCI, o1CG-UCI, …, oOCG-UCI-1CG-UCI is given by Table 6.3.2.1.3-1, mapped in the order from upper part to lower part.. - Set ai(2)=oiACK-LP for i=0,1,…,OACK-LP-1 and A(2)=OACK-LP, where the HARQ-ACK bit sequence o0ACK-LP, o1ACK-LP, …, oOACK-LP-1ACK-LP associated with priority index 0 is given by Clause 9.1 of [5, TS 38.213]. <Unchanged parts omitted> 6.3.2.4.1.6 UCI with different priority indexes If UCI-MuxWithDifferentPriority is configured, and HARQ-ACK bits associated with priority index 0, and CSI part 1 if any are transmitted on a PUSCH associated with priority index 1: - If CSI part 1 is also transmitted on the PUSCH, - Perform rate matching for CSI part 1 according to clause 6.3.2.4.1.2, by assuming the number of HARQ-ACK information bits to be transmitted on PUSCH in clause 6.3.2.4.1.2 is 0 bit. - Perform rate matching for HARQ-ACK with priority index 0 according to clause 6.3.2.4.1.3, by taking HARQ-ACK with priority index 0 as CSI part 2 and replacing βoffsetPUSCH by βoffsetHARQ-ACK-LP, and assuming the number of HARQ-ACK information bits to be transmitted on PUSCH in clause 6.3.2.4.1.3 is 0 bit. - Otherwise, perform rate matching for HARQ-ACK with priority index 0 according to clause 6.3.2.4.1.2, by taking HARQ-ACK with priority index 0 as CSI-part 1 and replacing βoffsetPUSCH by βoffsetHARQ-ACK-LP, and assuming the number of HARQ-ACK information bits to be transmitted on PUSCH in clause 6.3.2.4.1.2 is 0 bit. If UCI-MuxWithDifferentPriority and cg-UCI-Multiplexing are configured, and HARQ-ACK bits associated with priority index 0, and CG-UCI are transmitted on a PUSCH associated with priority index 1: - Perform rate matching for CG-UCI according to clause 6.3.2.4.1.1, by taking CG-UCI as HARQ-ACK and replacing βoffsetPUSCH by βoffsetCG-UCI. - Perform rate matching for HARQ-ACK with priority index 0 according to clause 6.3.2.4.1.2, by taking HARQ-ACK with priority index 0 as CSI part 1 and replacing βoffsetPUSCH by βoffsetHARQ-ACK-LP. <Unchanged parts omitted> -------------------------- End of Text Proposal for TS 38.212 --------------------------

LG proposal:

Observation #1: In case when the multiplexing of UCIs with different priories on PUCCH/PUSCH is enabled, there seems no reason to disable cg-UCI-Multiplexing since any CSI would not be multiplexed on CG PUSCH (thus, the number of UCI encodings on CG PUSCH would not be larger than 3).

Observation #2: In case when the multiplexing of UCIs with different priories on PUCCH/PUSCH is enabled and also cg-UCI-Multiplexing is enabled, joint encoding between CG-UCI and HARQ-ACK may not be required (even for the same priority) due to absence of the CSI on CG PUSCH.

Proposal #2: Consider the following cases and UE behaviours in case when the multiplexing of UCIs with different priories on PUCCH/PUSCH is enabled and also cg-UCI-Multiplexing is enabled.

- Case H1 - {HP CG-UCI, HP HARQ-ACK, LP HARQ-ACK} would be multiplexed on HP CG PUSCH - Option 1: HP CG-UCI and HP HARQ-ACK are jointly encoded, and LP HARQ-ACK is separately encoded from the jointly-encoded HP UCIs. - The jointly-encoded HP UCIs reuse the encoding/rate-matching/RE mapping of Rel-15 HARQ-ACK, and the LP HARQ-ACK reuses the encoding/rate-matching/RE mapping of Rel-15 CSI part 1. - Option 2: HP HARQ-ACK, HP CG-UCI, and LP HARQ-ACK are separately encoded. - The HP HARQ-ACK, HP CG-UCI, LP HARQ-ACK reuse the encoding/rate-matching/RE mapping of Rel-15 HARQ-ACK, CSI part 1, CSI part 2, respectively. - Case H2 - {HP CG-UCI, LP HARQ-ACK} would be multiplexed on HP CG PUSCH - HP CG-UCI and LP HARQ-ACK are separately encoded. - The HP CG-UCI reuses the encoding/rate-matching/RE mapping of Rel-15 HARQ-ACK, and the LP HARQ-ACK reuses the encoding/rate-matching/RE mapping of Rel-15 CSI part 1. - Case L1 - {HP HARQ-ACK, LP CG-UCI, LP HARQ-ACK} would be multiplexed on LP CG PUSCH - Option 1: LP CG-UCI and LP HARQ-ACK are jointly encoded, and HP HARQ-ACK is separately encoded from the jointly-encoded LP UCIs. - The HP HARQ-ACK reuses the encoding/rate-matching/RE mapping of Rel-15 HARQ-ACK, and the jointly-encoded LP UCIs reuse the encoding/rate-matching/RE mapping of Rel-15 CSI part 1. - Option 2: HP HARQ-ACK, LP HARQ-ACK, and LP CG-UCI are separately encoded. - The HP HARQ-ACK, LP HARQ-ACK, and LP CG-UCI reuse the encoding/rate-matching/RE mapping of Rel-15 HARQ-ACK, CSI part 1, CSI part 2, respectively. - Case L2 - {HP HARQ-ACK, LP CG-UCI} would be multiplexed on LP CG PUSCH - HP HARQ-ACK and LP CG-UCI are separately encoded. - The HP HARQ-ACK reuses the encoding/rate-matching/RE mapping of Rel-15 HARQ-ACK, and the LP CG-UCI reuses the encoding/rate-matching/RE mapping of Rel-15 CSI part 1.

Intel proposal:

- Proposal 2: When When the cg-RetransmissionTimer is configured and cg-UCI-Multiplexing is enabled, CG-UCI is jointly encoded with HP HARQ-ACK with beta offset for the HP HARQ-ACK.

Qualcomm proposal:

Proposal 3: In Rel-17 intra-UE multiplexing, treat CG-UCI of a certain priority as if a HARQ-ACK of the same priority, joint encode CG-UCI with HARQ-ACK of the same priority (if exist).

    - 1st round discussion

Proposal for 1st round discussion:

For Rel-17 intra-UE multiplexing, treat CG-UCI of a certain priority as if a HARQ-ACK of the same priority.

  - Joint encode CG-UCI with HARQ-ACK of the same priority if it exists.

  - Then reuse the existing multiplexing rules.

  - Support: ZTE, vivo, ITRI, CATT, DOCOMO, Huawei/Hisi, New H3C, Spreadtrum, Nokia/NSB, InterDigital, QC

  - Not support: Intel (proposed another option), Samsung (proposed another option)

| Company | Comments |
| --- | --- |
| OPPO | Before we go to details, we’d better to clarify whether to support R17 intra UE multiplexing in unlicensed band. |
| ZTE | Agree the proposal. |
| Intel | As explained in section 2.3, it was already agreed to support R17 intra UE multiplexing in unlicensed band, though details for how to multiplex is missing. And please note, the spec already captures the case, though companies think the spec may not correctly reflect the situation. For the proposal, we don’t support the proposal. We prefer to always treat CG-UCI as HP UCI, because CG-UCI is critical for unlicensed operation, not only for PUSCH itself but also for DL. For example, CG-UCI includes COT sharing information, which could enable no LBT or Cat-2 LBT by gNB for DL transmission. The COT sharing is irrelevant to priority, i.e., no matter UL is HP or LP, as long as UE shares the COT, both HP and LP DL can be transmitted. |
| vivo | ok |
| ITRI | Support the proposal. |
| DOCOMO | Agree with the proposal. |
| CATT | Agree with the proposal. |
| Huawei/Hisi | Support. As the CG-UCI does not include the resource/MCS information, the position and resources of HP UCI is deterministic, and there is no chicken-egg problem for the UE to decode HP UCI before decoding CG-UCI. |
| New H3C | Agree with this proposal. |
| Spreadtrum | Agree with the proposal. |
| Nokia, NSB | Support the proposal. |
| LG | I’d like to ask FL for clarification on whether the consequence of the above proposal would be the followings. - If {HP CG-UCI, HP HARQ-ACK, LP HARQ-ACK} would be multiplexed on HP CG PUSCH, HP CG-UCI and HP HARQ-ACK are jointly encoded, and LP HARQ-ACK is separately encoded from the jointly-encoded HP UCIs. - The jointly-encoded HP UCIs reuse the encoding/rate-matching/RE mapping of Rel-15 HARQ-ACK, and the LP HARQ-ACK reuses the encoding/rate-matching/RE mapping of Rel-15 CSI part 1. - If {HP CG-UCI, LP HARQ-ACK} would be multiplexed on HP CG PUSCH, HP CG-UCI and LP HARQ-ACK are separately encoded. - The HP CG-UCI reuses the encoding/rate-matching/RE mapping of Rel-15 HARQ-ACK, and the LP HARQ-ACK reuses the encoding/rate-matching/RE mapping of Rel-15 CSI part 1. - {HP HARQ-ACK, LP CG-UCI, LP HARQ-ACK} would be multiplexed on LP CG PUSCH, LP CG-UCI and LP HARQ-ACK are jointly encoded, and HP HARQ-ACK is separately encoded from the jointly-encoded LP UCIs. - The HP HARQ-ACK reuses the encoding/rate-matching/RE mapping of Rel-15 HARQ-ACK, and the jointly-encoded LP UCIs reuse the encoding/rate-matching/RE mapping of Rel-15 CSI part 1. - If {HP HARQ-ACK, LP CG-UCI} would be multiplexed on LP CG PUSCH, HP HARQ-ACK and LP CG-UCI are separately encoded. - The HP HARQ-ACK reuses the encoding/rate-matching/RE mapping of Rel-15 HARQ-ACK, and the LP CG-UCI reuses the encoding/rate-matching/RE mapping of Rel-15 CSI part 1. |
| InterDigital | Support proposal. |
| Samsung | Not support. We share similar view as Intel. |
| QC | We support FL proposal |

    - 2nd round discussion

In the 2nd round, we can check companies’ positions/comments to Option 2 proposed by Intel and Samsung.

To LG, I think the current formulation is better and simpler for Option 1. You can check your position.

Proposal for 2nd round discussion:

For Rel-17 intra-UE multiplexing,

  - Down-select from the two options:

    - Option 1: Treat CG-UCI of a certain priority as if a HARQ-ACK of the same priority.

    - Option 2: Always treat CG-UCI as high priority.

  - Joint encode CG-UCI with HARQ-ACK of the same priority if it exists.

  - Then reuse the existing multiplexing rules.

| Company | Comments |
| --- | --- |
| QC | We support this proposal. Between the two options, we support option 1. |
| LG | We also support the proposal. @FL: I didn’t intend to reformulate the current Option 1, rather I’d like to clarify the consequence (in terms of detailed UE behaviors) of the Option 1. |
| ZTE | Support. Option 1 is preferred. CG-UCI is carried by the corresponding PUSCH, common understanding is the priority of CG-UCI shares the priority of PUSCH. If CG-UCI always has high priority, it means we need to revisit the current common understanding? For the consequence list by LG, I think the consequence of the four cases are also my understanding. |
| Nokia/NSB | Support. And we prefer Option 1. |
| OPPO | Support and prefer to option 1 |
| vivo | Support and prefer Option 1. For the consequence list by LG, we have the same understanding. |
| ITRI | Support and prefer Option 1. |
| Sony | Support Option 1. However, Option 1 sounds weird, i.e.: - Option 1: Treat CG-UCI of a certain priority as if a HARQ-ACK of the same priority. Where does this HARQ-ACK come from? If the PUSCH does not carry HARQ-ACK then the CG-UCI has no priority? Shouldn’t it simply be the same priority as the PUSCH? |
| Ericsson | Support option 1. |
| DOCOMO | Support Option 1. |

Issue#8: Bitwidth of Beta-offset indicator (CATT)

CATT proposal:

The bit widths of beta_offset indicator in DCI format 0_1 or DCI format 0_2 based on different sets of beta-offset values can be different.

As a regular practice, a number of most significant bits with value set to '0' can be inserted to smaller beta_offset indicator until the bit width of the beta_offset indicator in DCI format for the HARQ-ACKs with different priorities multiplexing on one PUSCH are the same.

Proposal 4: Adopt the following text proposal for section 7.3.1.1.2 and 7.3.1.1.3 of TS38.212.

----------------------------------------------------- Start of text proposal ------------------------------------------------------

7.3.1.1.2	Format 0_1

DCI format 0_1 is used for the scheduling of one or multiple PUSCH in one cell, or indicating CG downlink feedback information (CG-DFI) to a UE.

The following information is transmitted by means of the DCI format 0_1 with CRC scrambled by C-RNTI or CS-RNTI or SP-CSI-RNTI or MCS-C-RNTI:

-	Identifier for DCI formats – 1 bit

-	The value of this bit field is always set to 0, indicating an UL DCI format

*** Unchanged text is omitted ***

-	Second PTRS-DMRS association – 2 bits if PTRS-DMRS association field and SRS resource set indicator field are present and maxRank>2; 0 bit otherwise. Table 7.3.1.1.2-25 and 7.3.1.1.2-26 are used to indicate the association between PTRS port(s) and DMRS port(s) corresponding to Second SRS resource indicator field and/or Second precoding information field when one PT-RS port and two PT-RS ports are configured by maxNrofPorts in PTRS-UplinkConfig respectively, and the DMRS ports are indicated by the Antenna ports field.

-	beta_offset indicator – 0 if the higher layer parameter betaOffsets = semiStatic; otherwise 2 bits as defined by Table 9.3-3 in [5, TS 38.213].

When two HARQ-ACK codebooks are configured for the same serving cell and if higher layer parameter priorityIndicatorDCI-0-1 is configured, if the bit width of the beta_offset indicator in DCI format 0_1 for one HARQ-ACK codebook is not equal to that of the beta_offset indicator in DCI format 0_1 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller beta_offset indicator until the bit width of the beta_offset indicator in DCI format 0_1 for the two HARQ-ACK codebooks are the same.

When higher layer parameter UCI-MuxWithDifferentPriority and priorityIndicatorDCI-0-1 are both configured, the bit width of the beta_offset indicator in DCI format 0_1 is determined by the maximum bit width of the beta_offset indicator in DCI format 0_1 for HARQ-ACK of larger priority index on PUSCH of smaller priority index, HARQ-ACK of smaller priority index on PUSCH of smaller priority index, HARQ-ACK of smaller priority index on PUSCH of higher priority index, HARQ-ACK of higher priority index on PUSCH of higher priority index. A number of most significant bits with value set to '0' are inserted to smaller beta_offset indicator until the bit width of the beta_offset indicator in DCI format 0_1 for HARQ-ACKs with different priorities multiplexing on one PUSCH with larger or smaller priority index are the same.

-	DMRS sequence initialization – 0 bit if transform precoder is enabled; 1 bit if transform precoder is disabled.

*** Unchanged text is omitted ***

7.3.1.1.3	Format 0_2

DCI format 0_2 is used for the scheduling of PUSCH in one cell.

The following information is transmitted by means of the DCI format 0_2 with CRC scrambled by C-RNTI or CS-RNTI or SP-CSI-RNTI or MCS-C-RNTI:

-	Identifier for DCI formats – 1 bit

-	The value of this bit field is always set to 0, indicating an UL DCI format

*** Unchanged text is omitted ***

-	Second PTRS-DMRS association – 2 bits if PTRS-DMRS association field and SRS resource set indicator field are present and maxRankDCI-0-2>2; 0 bit otherwise. Table 7.3.1.1.2-25 and 7.3.1.1.2-26 are used to indicate the association between PTRS port(s) and DMRS port(s) corresponding to Second SRS resource indicator field and/or Second precoding information field when one PT-RS port and two PT-RS ports are configured by maxNrofPorts in PTRS-UplinkConfig respectively, and the DMRS ports are indicated by the Antenna ports field.

-	beta_offset indicator – 0 bit if the higher layer parameter betaOffsets = semiStatic; otherwise 1 bit if 2 offset indexes are configured by higher layer parameter dynamicDCI-0-2 as defined by Table 9.3-3A in [5, TS 38.213], and 2 bits if 4 offset indexes are configured by higher layer parameter dynamicDCI-0-2 as defined by Table 9.3-3 in [5, TS 38.213].

When two HARQ-ACK codebooks are configured for the same serving cell and if higher layer parameter priorityIndicatorDCI-0-2 is configured, if the bit width of the beta_offset indicator in DCI format 0_2 for one HARQ-ACK codebook is not equal to that of the beta_offset indicator in DCI format 0_2 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller beta_offset indicator until the bit width of the beta_offset indicator in DCI format 0_2 for the two HARQ-ACK codebooks are the same.

When higher layer parameter UCI-MuxWithDifferentPriority and priorityIndicatorDCI-0-2 are both configured, the bit width of the beta_offset indicator in DCI format 0_2 is determined by the maximum bit width of the beta_offset indicator in DCI format 0_2 for HARQ-ACK of larger priority index on PUSCH of smaller priority index, HARQ-ACK of smaller priority index on PUSCH of smaller priority index, HARQ-ACK of smaller priority index on PUSCH of higher priority index, HARQ-ACK of higher priority index on PUSCH of higher priority index. A number of most significant bits with value set to '0' are inserted to smaller beta_offset indicator until the bit width of the beta_offset indicator in DCI format 0_2 for HARQ-ACKs with different priorities multiplexing on one PUSCH with larger or smaller priority index are the same.

-	DMRS sequence initialization – 0 or 1 bit

-	0 bit if the higher layer parameter dmrs-SequenceInitializationDCI-0-2 is not configured or if transform precoder is enabled;

-	1 bit if transform precoder is disabled and the higher layer parameter dmrs-SequenceInitializationDCI-0-2 is configured.

*** Unchanged text is omitted ***

------------------------------------------------------ End of text proposal ------------------------------------------------------

    - 1st round discussion

Proposal for 1st round discussion:

Adopt the following text proposal for section 7.3.1.1.2 and 7.3.1.1.3 of TS38.212.

----------------------------------------------------- Start of text proposal ------------------------------------------------------

7.3.1.1.2	Format 0_1

DCI format 0_1 is used for the scheduling of one or multiple PUSCH in one cell, or indicating CG downlink feedback information (CG-DFI) to a UE.

The following information is transmitted by means of the DCI format 0_1 with CRC scrambled by C-RNTI or CS-RNTI or SP-CSI-RNTI or MCS-C-RNTI:

-	Identifier for DCI formats – 1 bit

-	The value of this bit field is always set to 0, indicating an UL DCI format

*** Unchanged text is omitted ***

-	Second PTRS-DMRS association – 2 bits if PTRS-DMRS association field and SRS resource set indicator field are present and maxRank>2; 0 bit otherwise. Table 7.3.1.1.2-25 and 7.3.1.1.2-26 are used to indicate the association between PTRS port(s) and DMRS port(s) corresponding to Second SRS resource indicator field and/or Second precoding information field when one PT-RS port and two PT-RS ports are configured by maxNrofPorts in PTRS-UplinkConfig respectively, and the DMRS ports are indicated by the Antenna ports field.

-	beta_offset indicator – 0 if the higher layer parameter betaOffsets = semiStatic; otherwise 2 bits as defined by Table 9.3-3 in [5, TS 38.213].

When two HARQ-ACK codebooks are configured for the same serving cell and if higher layer parameter priorityIndicatorDCI-0-1 is configured, if the bit width of the beta_offset indicator in DCI format 0_1 for one HARQ-ACK codebook is not equal to that of the beta_offset indicator in DCI format 0_1 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller beta_offset indicator until the bit width of the beta_offset indicator in DCI format 0_1 for the two HARQ-ACK codebooks are the same.

When higher layer parameter UCI-MuxWithDifferentPriority and priorityIndicatorDCI-0-1 are both configured, the bit width of the beta_offset indicator in DCI format 0_1 is determined by the maximum bit width of the beta_offset indicator in DCI format 0_1 for HARQ-ACK of larger priority index on PUSCH of smaller priority index, HARQ-ACK of smaller priority index on PUSCH of smaller priority index, HARQ-ACK of smaller priority index on PUSCH of higher priority index, HARQ-ACK of higher priority index on PUSCH of higher priority index. A number of most significant bits with value set to '0' are inserted to smaller beta_offset indicator until the bit width of the beta_offset indicator in DCI format 0_1 for HARQ-ACKs with different priorities multiplexing on one PUSCH with larger or smaller priority index are the same.

-	DMRS sequence initialization – 0 bit if transform precoder is enabled; 1 bit if transform precoder is disabled.

*** Unchanged text is omitted ***

7.3.1.1.3	Format 0_2

DCI format 0_2 is used for the scheduling of PUSCH in one cell.

The following information is transmitted by means of the DCI format 0_2 with CRC scrambled by C-RNTI or CS-RNTI or SP-CSI-RNTI or MCS-C-RNTI:

-	Identifier for DCI formats – 1 bit

-	The value of this bit field is always set to 0, indicating an UL DCI format

*** Unchanged text is omitted ***

-	Second PTRS-DMRS association – 2 bits if PTRS-DMRS association field and SRS resource set indicator field are present and maxRankDCI-0-2>2; 0 bit otherwise. Table 7.3.1.1.2-25 and 7.3.1.1.2-26 are used to indicate the association between PTRS port(s) and DMRS port(s) corresponding to Second SRS resource indicator field and/or Second precoding information field when one PT-RS port and two PT-RS ports are configured by maxNrofPorts in PTRS-UplinkConfig respectively, and the DMRS ports are indicated by the Antenna ports field.

-	beta_offset indicator – 0 bit if the higher layer parameter betaOffsets = semiStatic; otherwise 1 bit if 2 offset indexes are configured by higher layer parameter dynamicDCI-0-2 as defined by Table 9.3-3A in [5, TS 38.213], and 2 bits if 4 offset indexes are configured by higher layer parameter dynamicDCI-0-2 as defined by Table 9.3-3 in [5, TS 38.213].

When two HARQ-ACK codebooks are configured for the same serving cell and if higher layer parameter priorityIndicatorDCI-0-2 is configured, if the bit width of the beta_offset indicator in DCI format 0_2 for one HARQ-ACK codebook is not equal to that of the beta_offset indicator in DCI format 0_2 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller beta_offset indicator until the bit width of the beta_offset indicator in DCI format 0_2 for the two HARQ-ACK codebooks are the same.

When higher layer parameter UCI-MuxWithDifferentPriority and priorityIndicatorDCI-0-2 are both configured, the bit width of the beta_offset indicator in DCI format 0_2 is determined by the maximum bit width of the beta_offset indicator in DCI format 0_2 for HARQ-ACK of larger priority index on PUSCH of smaller priority index, HARQ-ACK of smaller priority index on PUSCH of smaller priority index, HARQ-ACK of smaller priority index on PUSCH of higher priority index, HARQ-ACK of higher priority index on PUSCH of higher priority index. A number of most significant bits with value set to '0' are inserted to smaller beta_offset indicator until the bit width of the beta_offset indicator in DCI format 0_2 for HARQ-ACKs with different priorities multiplexing on one PUSCH with larger or smaller priority index are the same.

-	DMRS sequence initialization – 0 or 1 bit

-	0 bit if the higher layer parameter dmrs-SequenceInitializationDCI-0-2 is not configured or if transform precoder is enabled;

-	1 bit if transform precoder is disabled and the higher layer parameter dmrs-SequenceInitializationDCI-0-2 is configured.

*** Unchanged text is omitted ***

------------------------------------------------------ End of text proposal ------------------------------------------------------

| Company | Comments |
| --- | --- |
| OPPO | The intention to configure different number of beta offset for different priority is not clear. Before we go to support TP, we’d better to clarify whether it is allowed to configure different number of beta offset for different priority. |
| vivo | Similar view as OPPO, the indication in DCI for different priority is jointly, there seems no motivation to configure different number of beta offset for different priority. |
| DOCOMO | Share similar view to OPPO and vivo. It should be firstly clarified whether different number of beta offset is supported for different priority. |
| CATT | The TP follows the Rel-16 principle where it is possible that different number of beta offsets are configured for different priorities. If companies would like to discuss the possible restriction on configuration, we are also open to discuss. |
| Huawei/Hisi | Not clear of the motivation: is the intention to align beta-offset field to the two different priorities of a DCI format? We do not see a clear motivation on supporting that. In R16, the beta offset field is fixed 2 bits regardless of the HP/LP priority; if the bit widths anyhow need to be aligned, what is the end of wasting that beta-offset bit as padding instead of using it as valid code point? |
| Nokia, NSB | No support. Share similar view as other companies, the motivation of such proposal is not clear. |
| LG | Similar view with other companies, the motivation to make the proposal necessary doesn’t seem to be clear. |
| Samsung | Seems not essential. It can be avoided by proper gNB configuration. |
| Ericsson | Do not support. There is no need of determining bit width of beta_offset. The most obvious example is: - The size of the bit field in format 0_1 is fixed to 0 or 2 bits depending on if semi-static or dynamic beta factors are used. It can be further discussed if zero padding of beta_offset is needed when mixed priority is configured. But this change, if needed, is editorial in nature. |

    - 2nd round discussion

Void (Stop discussion at least for this meeting. Companies can further consider this issue in next meeting.).

- Remaining issues on PHY prioritization between DG and CG PUSCHs with different priorities

Issue#10: Spec clarification reflecting the agreement on DG-CG PUSCH prioritization (ZTE)

ZTE proposal:

Proposal 5: A TP for 38.214 is proposed to update the section 6.1 on collision between CG PUSCH and DG PUSCH with different priorities.

Reason for change: In Rel-17, when DG PUSCH overlaps with CG PUSCH with different priority, the UE can cancel the PUSCH with lower priority and then transmit the PUSCH with higher priority. The timeline defined in Rel-15 is not applicable to this case.

Summary of the change: The DG PUSCH overlapping with CG PUSCH with different priority in Rel-17 is excluded.

Consequences if not approved: The UE may determine the Rel-17 overlapping is error case if the timeline is not satisfied.

# 6.1 UE procedure for transmitting the physical uplink shared channel < Unchanged part is omitted > A UE is not expected to be scheduled by a PDCCH ending in symbol i to transmit a first PUSCH on a given serving cell overlapping in time with a transmission occasion, where the UE is allowed to transmit a second PUSCH with configured grant according to [10, TS38.321], starting in a symbol j on the same serving cell if the end of symbol i is not at least N2 symbols before the beginning of symbol j unless the first PUSCH and the second PUSCH have the different priority index [9, TS38.213] and the UE is provided prioritizationBetweenLP-DG-PUSCHandHP-CG-PUSCH or prioritizationBetweenHP-DG-PUSCHandLP-CG-PUSCH. The value N2 in symbols is determined according to the UE processing capability defined in Clause 6.4, and N2 and the symbol duration are based on the minimum of the subcarrier spacing corresponding to the PUSCH with configured grant and the subcarrier spacing of the PDCCH scheduling the PUSCH. < Unchanged part is omitted >

    - 1st round discussion

Proposal for 1st round discussion:

Adopt the following text proposal for Section 6.1 of TS 38.214.

# 6.1	UE procedure for transmitting the physical uplink shared channel

< Unchanged part is omitted >

A UE is not expected to be scheduled by a PDCCH ending in symbol i to transmit a first PUSCH on a given serving cell overlapping in time with a transmission occasion, where the UE is allowed to transmit a second PUSCH with configured grant according to [10, TS38.321], starting in a symbol j on the same serving cell if the end of symbol i is not at least N2 symbols before the beginning of symbol j unless the first PUSCH and the second PUSCH have the different priority index [9, TS38.213] and the UE is provided prioritizationBetweenLP-DG-PUSCHandHP-CG-PUSCH or prioritizationBetweenHP-DG-PUSCHandLP-CG-PUSCH. The value N2 in symbols is determined according to the UE processing capability defined in Clause 6.4, and N2 and the symbol duration are based on the minimum of the subcarrier spacing corresponding to the PUSCH with configured grant and the subcarrier spacing of the PDCCH scheduling the PUSCH.

< Unchanged part is omitted >

| Company | Comments |
| --- | --- |
| OPPO | Fine with TP |
| ZTE | Agree |
| vivo | Fine with the TP in general. |
| ITRI | Fine with the TP. |
| DOCOMO | Fine with the TP. |
| CATT | Fine with the TP. |
| Huawei/Hisi | OK with the TP. |
| New H3C | Fine with TP. |
| Nokia/NSB | Fine with the TP in general. We suggest the following editorial changes: … unless the first PUSCH and the second PUSCH have the different priority indexes … |
| InterDigital | Fine with TP and Nokia/NSB editorial changes. |
| Samsung | We think it only needs to consider the case where prioritizationBetweenHP-DG-PUSCHandLP-CG-PUSCH is configured. We suggest the following update A UE is not expected to be scheduled by a PDCCH ending in symbol i to transmit a first PUSCH on a given serving cell overlapping in time with a transmission occasion, where the UE is allowed to transmit a second PUSCH with configured grant according to [10, TS38.321], starting in a symbol j on the same serving cell if the end of symbol i is not at least N2 symbols before the beginning of symbol j unless the first PUSCH and the second PUSCHs have the different priority indexes [9, TS38.213] and the UE is provided prioritizationBetweenLP-DG-PUSCHandHP-CG-PUSCH or prioritizationBetweenHP-DG-PUSCHandLP-CG-PUSCH. The value N2 in symbols is determined according to the UE processing capability defined in Clause 6.4, and N2 and the symbol duration are based on the minimum of the subcarrier spacing corresponding to the PUSCH with configured grant and the subcarrier spacing of the PDCCH scheduling the PUSCH. |
| QC | The issue identified by ZTE is valid. But the TP is not accurate. It could create a confusion that there are no timeline requirements for HP DG canceling LP CG. Therefore, it is better to clarify the above is only for DG and CG with same priority. For CG vs DG with different priorities, see 38.214 other section. |

    - 2nd round discussion

Proposal for 2nd round discussion:

Adopt the following text proposal for Section 6.1 of TS 38.214.

# 6.1	UE procedure for transmitting the physical uplink shared channel

< Unchanged part is omitted >

A UE is not expected to be scheduled by a PDCCH ending in symbol i to transmit a first PUSCH on a given serving cell overlapping in time with a transmission occasion, where the UE is allowed to transmit a second PUSCH with configured grant according to [10, TS38.321], starting in a symbol j on the same serving cell if the end of symbol i is not at least N2 symbols before the beginning of symbol j unless the first PUSCH and the second PUSCHs have the different priority indexes [9, TS38.213] and the UE is provided prioritizationBetweenLP-DG-PUSCHandHP-CG-PUSCH or prioritizationBetweenHP-DG-PUSCHandLP-CG-PUSCH. The value N2 in symbols is determined according to the UE processing capability defined in Clause 6.4, and N2 and the symbol duration are based on the minimum of the subcarrier spacing corresponding to the PUSCH with configured grant and the subcarrier spacing of the PDCCH scheduling the PUSCH.

< Unchanged part is omitted >

| Company | Comments |
| --- | --- |
| QC | The issue identified by ZTE is valid, and we agree with the intention of the TP. But the current TP is not accurate. It could create a confusion that there are no timeline requirements for HP DG canceling LP CG, because the updated spec could be read as the timeline requirements cannot be violated unless the two PUSCHs have different priority. Therefore, it is better to clarify the above section is only for DG and CG with same priority. For CG vs DG with different priorities, UE should follow other sections in 214. |
| ZTE | Thanks for Nokia, Samsung and QC’s comments. @ Samsung, I am not sure the reason that only case of prioritization between HP-DG-PUSCH and LP-CG-PUSCH is left. Would you like to elaborate more? For QC’s suggestion, we don’t think there is confusion which you worried about since here only just the timeline between the DCI for DG PUSCH and CG PUSCH is stated and the cancellation timeline has been defined in TS38.213. In addition, this section is also applied for DG and CG with different priorities in Rel-16. Therefore, the proposed update has the smallest spec change compared with your suggestion. |
| Nokia/NSB | Cannot accept, as this is not a TP on top of the current specs (mix of track changes between ZTE and Samsung version) – as commented by Email. As by the comment ZTE: Why would this only apply if configured with prioritizationBetweenHP-DG-PUSCHandLP-CG-PUSCH and not prioritizationBetweenLP-DG-PUSCHandHP-CG-PUSCH? (compared to the earlier ZTE version)?? |
| Ericsson | We agree with the intention of the TP. We have similar understanding as ZTE that Rel-16 allows DG override CG even when they have different priorities, and this section in 36.214 should apply. |
| DOCOMO | We share the same view as ZTE/Nokia/Ericsson. We are not clear why only HP DG vs. LP CG is considered for the exception. We prefer the previous TP proposed by ZTE. |

- Proposals for GTW discussion

GTW session on May 12th Thursday

Proposal 1:

For the scenarios where multiplexing low-priority HARQ-ACK onto high-priority PUSCH, down-select from the options:

  - Option 1: In case of insufficient resource for LP HARQ-ACK, LP HARQ-ACK is entirely dropped

  - Option 2: LP HARQ-ACKs are mapped to the rest REs of the PUSCH based on the rate matching equation, if HP HARQ-ACK and/or HP CSI have been mapped in prior on the PUSCH.

  - Option 3: UE does not expect insufficient resource for multiplexing low-priority HARQ-ACK.

| Option 1 | OPPO, Sony, Intel, ITRI, LG, InterDigital, Nokia/NSB (can accept) |
| --- | --- |
| Option 2 | ZTE, Intel (can live with), vivo, DOCOMO, CATT, Huawei/Hisi, New H3C, Spreadtrum, Samsung, QC |
| Option 3 | Nokia/NSB |

Proposal 2:

For HARQ-ACK multiplexing on the PUSCH with CG UCI,

  - Down-select from the two options:

    - Option 1: Treat CG-UCI of a certain priority as if a HARQ-ACK of the same priority.

    - Option 2: Always treat CG-UCI as high priority.

  - Joint encode CG-UCI with HARQ-ACK of the same priority if it exists.

  - Then reuse the existing multiplexing rules.

| Option 1 | QC, ZTE, Nokia/NSB, OPPO, vivo, ITRI, Sony (improve wording), E///, DOCOMO, [Huawei/Hisi, New H3C, Spreadtrum, InterDigital] |
| --- | --- |
| Option 2 | [Intel, Samsung] |

- References

- R1-2203073	Intra-UE multiplexing enhancements	Huawei, HiSilicon

- R1-2203189	Discussion on enhanced intra-UE multiplexing	ZTE

- R1-2203213	Remaining issue on intra-UE multiplexing and prioritization	New H3C Technologies Co., Ltd.

- R1-2203274	Maintenance of Rel-17 URLLC / IIoT Intra-UE Multiplexing/Prioritization	Nokia, Nokia Shanghai Bell

- R1-2203305	Discussion on intra-UE multiplexing/prioritization	Spreadtrum Communications

- R1-2203398	Remaining Issues of Intra-UE Multiplexing/Prioritization for IIoT/URLLC	Ericsson

- R1-2203435	Maintenance on intra-UE multiplexing and prioritization	CATT

- R1-2203514	Remaining issues on intra-UE Multiplexing/Prioritization for Rel-17 URLLC	vivo

- R1-2203863	Maintenance on Intra-UE Multiplexing/Prioritization	Samsung

- R1-2204003	Remaining issues on intra-UE multiplexing/prioritization	OPPO

- R1-2204091	Remaining issues of Intra-UE Multiplexing/Prioritization	InterDigital, Inc.

- R1-2204206	Remaining issues in intra-UE multiplexing/prioritization	Apple

- R1-2204344	Remaining issues on intra-UE multiplexing/prioritization for Rel.17 URLLC	NTT DOCOMO, INC.

- R1-2204439	Remaining details on intra-UE multiplexing and prioritization	ITRI

- R1-2204547	Remaining issues on intra-UE multiplexing/prioritization for URLLC/IIoT	WILUS Inc.

- R1-2204617	Remaining issues on Intra-UE multiplexing enhancements	LG Electronics

- R1-2204647	Intra-UE Multiplexing/Prioritization	ETRI

- R1-2204770	Remaining issue for Intra-UE Uplink Channel Multiplexing and Prioritization	Intel Corporation

- R1-2204983	Remaining issues on Intra-UE multiplexing and prioritization for IOT and URLLC	Qualcomm Incorporated