3GPP TSG RAN WG1 #109-e			R1-2205191

e-Meeting, May 9th – 20th, 2022

Agenda Item:	8.2.3

Source: 	Moderator (LG Electronics)

Title: 	Summary #1 of PDSCH/PUSCH enhancements (Scheduling/HARQ)

Document for:	Discussion and decision

Introduction

This is the summary document for 8.2.3 on PDSCH/PUSCH enhancements (especially for scheduling and HARQ) for NR above 52.6 GHz, based on the contributions listed in reference section.

The following email thread is assigned for discussion of this topic:

[109-e-R17-FR2-2-04] Email discussion under 8.2.3 for maintenance on scheduling and HARQ, for issues 4-1, 4-2, 4-3, 4-5, 4-10, 4-11, 4-12, 4-13, 4-16, 4-18, 4-21 in R1-2205124 – Seonwook (LGE)

- 1st check point: May 13 (any RRC impact by May 12)

- Final check point: May 18

Issue#4-1: Clarification on type-1 HARQ CB that time domain bundling is applied across PDSCHs scheduled by “the same DCI”

| Company | Views |
| --- | --- |
| [1] Huawei | Proposal 1: It should be clarified that logical AND operation is applied across all valid PDSCHs scheduled by the same DCI associated with a determined candidate PDSCH reception occasion when time bundling is applied for type 1 HARQ codebook generation. |
| [8] Fujitsu | Observation 1: According to the agreement made in RAN#107b-e meeting for type-1 HARQ-ACK codebook with time domain bundling, logical AND operation may be applied to PDSCHs scheduled by different DCIs (up to gNB’s scheduling). However, the current specification (TS 38.213 h10) seems not aligned with the agreement (the specification seems to limit logical AND operation to the case where the associated valid PDSCHs are scheduled by a single DCI). Proposal 3: For Type-1 HARQ-ACK codebook with time domain bundling, clarify the following in TS 38.213: - Logical AND operation is applied across all valid PDSCHs associated with a determined candidate PDSCH reception occasion, and the valid PDSCHs may be scheduled by one or more DCIs. |

Agreement (RAN1#107bis-e)

- Update the previous agreement made in RAN1#107-e, as follows:

Agreement (RAN1#107-e)

For multi-PDSCH scheduling with a single DCI

- Introduce a new RRC parameter, e.g., enableTimeDomainHARQ-Bundling, to enable time domain bundling operation for type-1 HARQ-ACK codebook per serving cell.

  - If the RRC parameter enables time domain bundling operation,

    - To determine the set of candidate PDSCH reception occasions,

      - A row index is removed if at least one symbol of every PDSCH associated with the row index is configured as semi-static UL. (NOTE: This is similar to the case of slot aggregated PDSCH in Rel-16)

      - Pruning procedure in Rel-16 is performed based on the last configured SLIV of each row index.

    - Logical AND operation is applied across all valid PDSCHs associated with a determined candidate PDSCH reception occasion, at least for 1-TB case.

    - FFS: UE does not expect the last scheduled SLIV overlaps with a semi-static UL symbol when parameter enableTimeDomainHARQ-Bundling is configured

- (Closed) [Moderator’s note#1] There could be two interpretations on the highlighted part above in the agreement made in RAN1#107bis-e.

- Interpretation 1: Logical AND operation is applied across all valid PDSCHs scheduled by the same DCI associated with a determined candidate PDSCH reception occasion when time bundling is applied for type-1 HARQ codebook generation.

- Interpretation 2: Logical AND operation is applied across all valid PDSCHs associated with a determined candidate PDSCH reception occasion, and the valid PDSCHs may be scheduled by one or more DCIs.

Companies are encouraged to express which interpretation is consistent with current specifications.

| Company | Views |
| --- | --- |
| Samsung | Interpretation 1. It is clear from the following text in 213. if the PDSCH is associated with the last SLIV in the TDRA row ojACK = binary AND operation of the HARQ-ACK information bits corresponding to first transport blocks in PDSCH receptions, that do not overlap with an uplink symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, scheduled by the DCI format on serving cell c; j=j+1; ojACK = binary AND operation of the HARQ-ACK information bits corresponding to second transport blocks in PDSCH receptions, that do not overlap with an uplink symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, scheduled by the DCI format on serving cell c; else ojACK= NACK; j=j+1; ojACK= NACK; end if |
| ZTE, Sanechips | We understand that the current specification is consistent with the Interpretation 1. |
| Intel | Interpretation 1 is the correct understanding based on early RAN1 discussions. Limiting time bundling to the PDSCHs scheduled by the same DCI avoids any error case caused by missing DCI(s). As a comparison, with interpretation 2, if PDSCHs scheduled by two DCIs are to be bundled from gNB point of view, and if UE misses one of the two PDCCHs, UE will simply generate the bundled HARQ-ACK based on the PDSCHs scheduled by only one PDCCH. This may result in wrong HARQ-ACK feedback. |
| Apple | Our understanding is that it is Interpretation 1 should be the correct interpretation. |
| InterDigital | Interpretation 1 |
| Nokia, NSB | We see that interpretation 1 is the correct one. This is also indicated by agreement’s text “For multi-PDSCH scheduling with a single DCI” limiting the following bullets to that case. Further, the second interpretation reduces the reliability of HARQ.ACK feedback in the case when UE fails to detect DCI scheduling PDSCH. |
| Panasonic | Our understanding is the Interpretation 1. |
| Qualcomm | We agree with Interpretation 1 |
| Huawei, HiSilicon | Our understanding is interpretation 1. Similar as the time bundling for type 2 HARQ codebook, the time bundling for type 1 HARQ codebook should also follow the same concept that AND operation is applied across PDSCH scheduled by single DCI. This is also the original intention when RAN1 achieved the agreement for type 1 HARQ codebook. If allowing HARQ-ACK corresponding to PDSCH scheduled by different DCI, it may cause unnecessary retransmission already scheduled with high reliability. |
| Fujitsu | While we still think the agreement itself is not clear to be interpretation 1, we are okey to follow the majority view that interpretation 1 is assumed for Type 1 HARQ-ACK codebook with time domain bundling. |
| vivo | As we understand, interpretation 1 is aligned with current spec and interpretation 2 seems to be more aligned with the description in the agreement. We are fine with both interpretations. |
| CATT1 | Our understanding is Interpretation 1 and it is aligned with current spec |
| OPPO | Interpretation 1. |
| Moderator | It seems that all companies agree with Interpretation 1. As pointed out by Samsung, since current specification is clear enough, we can close the issue at this meeting. |

In addition, [8] Fujitsu brought up the following issue.

Issue 2: How to support SPS PDSCH According to discussion so far, when time domain bundling is supported, it is possible that for a determined candidate PDSCH reception occasion, the associated PDSCHs include both PDSCH scheduled by DCI and PDSCH scheduled by SPS (SPS PDSCH). For example, as shown in Figure 2, for occasion 1, one associated valid PDSCH is scheduled by a DCI, and one associated valid PDSCH is scheduled by SPS. Figure 2. example of one valid PDSCH scheduled by one DCI and one valid PDSCH scheduled by SPS for an occasion However, so far, there is still no discussion about how to handle the case where the PDSCHs associated with a determined candidate PDSCH reception occasion include both PDSCH scheduled by DCI and PDSCH scheduled by SPS (SPS PDSCH). In our view, for such case, the following options can be considered. - Option 1: Logical AND operation is applied across all valid PDSCHs associated with a determined candidate PDSCH reception occasion, and the valid PDSCHs may be scheduled by DCI and SPS. - Option 2: For a determined candidate PDSCH reception occasion, if UE receives a DCI which schedule a PDSCH associated with the occasion, UE will not receive SPS PDSCH (if any) associated with the same occasion and not report HARQ-ACK information corresponding to the SPS PDSCH. Observation 2: For Type-1 HARQ-ACK codebook with time domain bundling, it has not been discussed how to support SPS PDSCH. Proposal 4: For Type-1 HARQ-ACK codebook with time domain bundling, discuss how to support SPS PDSCH and consider the following two options. - Option 1: Logical AND operation is applied across all valid PDSCHs associated with a determined candidate PDSCH reception occasion, and the valid PDSCHs may be scheduled by DCI and SPS. - Option 2: For a determined candidate PDSCH reception occasion, if the UE receives a DCI which schedules a PDSCH associated with the occasion, the UE would not receive SPS PDSCH (if any) associated with the same occasion and not report HARQ-ACK information corresponding to the SPS PDSCH.

- [Moderator’s note#2] Companies are encouraged to express which option in the above Proposal 4 is to be supported.

| Company | Views |
| --- | --- |
| Samsung | SPS PDSCH is not triggered by a DCI format scheduling multi-PDSCH. No time domain bundling is applicable. So, Option 1 is incorrect. The example in Fujitsu’s tdoc is treated as an error case. Recall that Rel-15 PDSCH repetition, where the last PDSCH repetition overlaps with UL symbol in a slot, but there is SPS PDSCH reception not overlapping with UL symbol in the slot. The situation is exactly same. This case is an error case by the following statement for 213. (Basically, the statement was introduced in Rel-15 to capture that there are no two PDSCH receptions for the same HARQ-ACK bit position) If the UE indicates a capability to receive more than one PDSCH per slot, for occasions of candidate PDSCH receptions corresponding to rows of R associated with a same value of br,k,nD, where br,k,nD∈B, the UE does not expect to receive more than one PDSCH in a same DL slot associated with a same coresetPoolIndex value if provided, or if coresetPoolIndex is not provided. We don’t see any specification update for this issue. |
| DOCOMO | Agree with Samsung. |
| ZTE, Sanechips | The similar case has been discussed in RAN1 #107-e as shown in the following figure. We think that this issue can be discussed after Issue #4-2. |
| Intel | We don’t support Option 1 since it has confusion for bundling in case missing PDCCH. Option 2 is aligned with existing behavior when a PDSCH scheduled by single-slot DCI is overlapped with a SPS PDSCH. In this case, SPS PDSCH is dropped and there is no valid HARQ-ACK feedback for SPS PDSCH. |
| Apple | Agree with Samsung’s interpretation. |
| InterDigital | Agree with Samsung that no update is needed. |
| Panasonic | Agree with Samsung’s comment. |
| Qualcomm | We agree with Samsung’s comment |
| Huawei, HiSilicon | As the transmission of scheduled PDSCH and SPS PDSCH are all controlled by gNB, gNB should avoid the case which might cause ambiguity when UE generate HARQ codebook. Thus, our understanding is option 2. |
| Fujitsu | Given interpretation 1 is the common understanding issue #4-1, option 2 is straight forward. Furthermore, we think it is necessary to update the spec. The spec. mentioned by Samsung as below is about restriction on number of PDSCHs per DL slot. It does not cover this case as shown in the example figure, where the SPS PDSCH and PDSCH scheduled by a DCI are in different DL slots. If the UE indicates a capability to receive more than one PDSCH per slot, for occasions of candidate PDSCH receptions corresponding to rows of R associated with a same value of br,k,nD, where br,k,nD∈B, the UE does not expect to receive more than one PDSCH in a same DL slot associated with a same coresetPoolIndex value if provided, or if coresetPoolIndex is not provided. |
| vivo | We share the similar view as ZTE. |
| CATT1 | No specification is needed. |
| OPPO | Agree with Samsung. |
| Moderator | It seems that all companies support Option 2 (i.e., For a determined candidate PDSCH reception occasion, if the UE receives a DCI which schedules a PDSCH associated with the occasion, the UE would not receive SPS PDSCH (if any) associated with the same occasion and not report HARQ-ACK information corresponding to the SPS PDSCH.). @ Fujitsu and Samsung, From my understanding, Samsung’s point is not about (valid) PDSCH in slot n-3 and SPS PDSCH in slot n-1, but about (invalid) PDSCH in slot n-1 and SPS PDSCH in slot n-1. Strictly speaking, the sentence cited by Samsung may have a problem due to the term “not expect to receive” since UE will not receive (invalid) PDSCH in slot n-1. In that sense, could we modify the sentence cited by Samsung, as follows (shown in red text)? I’m not sure if it works… It would be appreciated if you and other companies as well could share views on the below text. If the UE indicates a capability to receive more than one PDSCH per slot, for occasions of candidate PDSCH receptions corresponding to rows of R associated with a same value of br,k,nD, where br,k,nD∈B, the UE does not expect to receive or be scheduled with more than one PDSCH in a same DL slot associated with a same coresetPoolIndex value if provided, or if coresetPoolIndex is not provided. |
| Xiaomi | Maybe we miss something, from our understanding, the option2 is very similar as Samsung’s view(the example listed is an error case). We can accept Option 2. |
| Samsung | Regarding the Moderator’s suggestion, we think the current specification is clear from - Based on the text I copied in the previous round, a UE only receives one PDSCH among PDSCHs for a value of br,k,nD. In other words, even there are many PDSCHs corresponding to a value of br,k,nD, gNB makes sure only one PDSCH is received. - The type-1 CB is constructed the received one PDSCH for each br,k,nD. Therefore, gNB would not schedule the PDSCHs if the corresponding br,k,nD is used for other PDSCH reception. If it is deemed a problem, it should be discussed in Rel-15 maintenance. It is because the same scheduling situation can occur in Rel-15, i.e., SPS and multi-slot PDSCH. |
| Fujitsu | Thanks for moderator’s comments and guidance. Firstly, we would like to confirmed that previously we are talking about the valid PDSCH in slot n-3 and SPS PDSCH in slot n-1, not about the invalid PDSCH in slot n-1 and SPS PDSCH slot n-1. And UE behavior of whether/how to receive the valid PDSCH in slot n-3 and SPS PDSCH in slot n-1 is not clear and needs to be clarified in spec. Regarding adding “or be scheduled with” in the sentence, we agree that might resolve the issue. But we still have one question for clarification. According to the highlights as below, when the UE is capable of receiving more than one PDSCH per slot, it seems UE may receive two PDSCHs per slot if the two PDSCHs are associated with different coresetPoolIndex. If it is possible that the valid PDSCH in slot n-3 and SPS PDSCH in slot n-1 are associated with different coresetPoolIndex, just adding “or be scheduled with” would be not sufficient. For now, we assume that case is invalid. But still, we would like to check if it is the common understanding. If the UE indicates a capability to receive more than one PDSCH per slot, for occasions of candidate PDSCH receptions corresponding to rows of R associated with a same value of br,k,nD, where br,k,nD∈B, the UE does not expect to receive or be scheduled with more than one PDSCH in a same DL slot associated with a same coresetPoolIndex value if provided, or if coresetPoolIndex is not provided. To Samsung, It seems we have different understanding on the sentence “the UE does not expect to receive more than one PDSCH in a same DL slot”. In the legacy, br,k,nD is mapping to one DL slot. So, based on the sentence, it can be inferred that "a UE only receives one PDSCH among PDSCHs for a value of br,k,nD". It can cover the case of repetition because all repetitions are treated as one PDSCH. However, for case of multi-PDSCH scheduling, a br,k,nD is mapping to multiple DL slots in which different PDSCHs can be received, we do not think we can say “a UE only receives one PDSCH among PDSCHs for a value of br,k,nD". Based on the sentence, it is not clear how to receive the valid PDSCH in slot n-3 and SPS PDSCH in slot n-1. So, it is necessary to update the specification for option 2. |
| Moderator2 | @ Fujitsu, Seeing Samsung’s comment, I think Samsung’s analysis is correct in that the issue pointed out by Fujitsu exactly same as Rel-15 case where SPS PDSCH and slot-aggregated PDSCH are overlapped on the same PDSCH reception occasion br,k,nD. We are talking about the case when time domain bundling is configured for type-1 HARQ-ACK CB. Then, each PDSCH reception occasion is determined based on the last SLIV. In that sense, we don’t need to consider slot n-3 in Fujitsu’s figure (Perhaps this kind of confusion comes from the misunderstanding on current pseudo code, so please see my note also for Issue#4-2). Focusing on slot n-1, based on Samsung’s explanation, gNB would not make UE confused by scheduling PDSCH and activating SPS PDSCH in the same slot (or in the same PDSCH reception occasion). |
| Samsung | @Fujitsu. Thanks for the explanation. We cannot agree with your comment on “In the legacy, br,k,nD is mapping to one DL slot.” For multi-slot PDSCH reception, br,k,nD corresponds to all PDSCH repetitions. br,k,nD does not include slot index (note that nD is to cover different numerology of DL cell and UL cell). The only different thing is that the same TB is repeated in Rel-15 PDSCH repetition over slots, but the different TBs are mapped to each PDSCH reception in Rel-17. The definition of br,k,nD is not changed. For both cases, br,k,nD corresponds to all PDSCH receptions (whatever the same TB or the different TB) If you have still concern on this issue, could you kindly provide the TP to address your concern? We can further make a common understanding on whether your TP is essential or not. Thanks |
| Fujitsu | @Samsung For the time being, the TP suggested by moderator looks fine to us. C&P as below. If the UE indicates a capability to receive more than one PDSCH per slot, for occasions of candidate PDSCH receptions corresponding to rows of R associated with a same value of br,k,nD, where br,k,nD∈B, the UE does not expect to receive or be scheduled with more than one PDSCH in a same DL slot associated with a same coresetPoolIndex value if provided, or if coresetPoolIndex is not provided. And based on the above comments, we share the view with moderator that clarification of current pseudo code may help to figure out whether/how we consider slot n-3 in our figure for this issue. |
| Intel | We share the views from Samsung. The specification already captures the operation of Option 2 by br,k,nD. A br,k,nD determined by the last PDSCH of multi-PDSCH transmission is associated with all PDSCHs of the multi-PDSCH transmission. Based on TDRA table and semi-static TDD UL-DL configuration, after removing some invalid TDRA rows, each remaining valid row will be assign with a value br,k,nD. - Note: for multi-slot PDSCH or multi-PDSCH with time bundling, the TDRA row in the last slot is considered as valid if at least one SLIV is not overlapped with semi-static UL symbol. Consequently, a br,k,nD is assigned to the multi-PDSCH transmission The number of different values of br,k,nD in a slot is equal to the maximum non-overlap TDRA rows among the remaining TDRA rows in the slot. In other words, two or multiple remaining TDRA row can have the same value of br,k,nD. This is the desired outcome of the pseudo code. Finally, we have two cases - If br,k,nD for the TDRA row of the last PDSCH of the multi-PDSCH transmission is same as the br,k,nD of the SPS PDSCH, it will be an error case, based on the following specification. If the UE indicates a capability to receive more than one PDSCH per slot, for occasions of candidate PDSCH receptions corresponding to rows of R associated with a same value of br,k,nD, where br,k,nD∈B, the UE does not expect to receive more than one PDSCH in a same DL slot associated with a same coresetPoolIndex value if provided, or if coresetPoolIndex is not provided. - If br,k,nD for the TDRA row of the last PDSCH of the multi-PDSCH transmission is different as the br,k,nD of the SPS PDSCH, there is no collision. Therefore, UE can separately report the bundled HARQ-ACK for multi-PDSCH and the HARQ-ACK for the SPS PDSCH. I think this case is irrelevant to the current discussion As a conclusion, we think the current specification already cover the intended Option 2 behaviour. |
| Fujitsu | In our understanding, according to the discussions, companies are OK with Option 2. But it is not clear whether/how to update the spec., because it depends on how to interpretate the pseudo code for Type-1 HARQ-ACK codebook generation with time domain bundling. Considering the situation, we suggest a proposal as below. Hope it can be agreed in this meeting. And then we can further discuss specification impact in the future, based on the common understanding on pseudo code. Proposal： For Type-1 HARQ-ACK codebook with time domain bundling, - For a determined candidate PDSCH reception occasion, if the UE receives a DCI which schedules a PDSCH associated with the occasion, the UE would not receive SPS PDSCH (if any) associated with the same occasion and not report HARQ-ACK information corresponding to the SPS PDSCH. (Option 2) - FFS: specification impact |
| Samsung | Thanks Fujitsu for sharing your TP and proposal. Regarding the TP, it changes a UE behavior for single PDSCH scheduling defined in Rel-15. For example, it potentially blocks the SPS PDSCH cancelation by DG PDSCH scheduling. So, we need more time to check it. Regarding the proposal, we appreciate Fujitsu’ detail explanations so far. Unfortunately, we feel the proposal is not necessary because the specification is clear to us from Rel-15. Again, if there is a concern on br,k,nD, Rel-15 UE behaviour should be clarified first. |
| Moderator | @ Fujitsu and Samsung, I share the view from Samsung. If Fujitsu indeed wants to capture our discussion here, the following would be my proposal. Proposed conclusion: UE does not expect to be scheduled with a PDSCH (scheduled by multi-PDSCH DCI) and to receive SPS PDSCH for a PDSCH reception occasion within a slot. |
| Fujitsu | Thanks for moderator’s proposed conclusion. We are fine with the conclusion. |

- Proposed Conclusion #4-1 (OOO):

If type-1 HARQ-ACK codebook is configured, UE does not expect to be scheduled with a PDSCH (scheduled by multi-PDSCH DCI) and to receive SPS PDSCH for a PDSCH reception occasion within a slot.

Companies are encouraged to provide views on Proposed Conclusion #4-1.

| Company | Views |
| --- | --- |
| Moderator | Based on the discussion with Fujitsu, the above conclusion can be drawn, which is basically the same behavior for the case of one PDSCH occasion (among slot-aggregated PDSCH repetitions) and SPS PDSCH in the same slot, from Rel-15. Please let us know if the proposed conclusion is acceptable or not. |
| Intel | We support the FL proposal |
| ZTE, Sanechips | We are fine with the proposed conclusion#4-1 |
| Ericsson | Support proposed conclusion #4-1 |
| Samsung | A clarification question is can DG PDSCH cancel SPS PDSCH of two PDSCHs are overlapped? The proposed conclusion (not expected to be scheduled) seems it is not allowed. |
| Xiaomi | OK with the conclusion |

- Proposed Conclusion #4-1a (SPS PDSCH):

If type-1 HARQ-ACK codebook with time domain bundling is configured, UE does not expect to be scheduled with a PDSCH (scheduled by multi-PDSCH DCI) and to receive SPS PDSCH for a PDSCH reception occasion within a slot.

- Note: This applies only when the PDSCH (scheduled by multi-PDSCH DCI) is associated with the last SLIV and invalid due to collision with semi-static UL symbol(s).

Companies are encouraged to provide views on Proposed Conclusion #4-1a.

| Company | Views |
| --- | --- |
| Moderator | To address Samsung’s comment that DG PDSCH can override SPS PDSCH, I imposed further restrictions that the proposal is for the case of type-1 CB with time domain bundling configured, and DG PDSCH is invalid. @ Samsung, Would this work? |
| Huawei, HiSilicon | We are fine with the main sentence of the conclusion. However, we are not quite catching up what is the note for. Is it trying to restrict the scheduled PDSCH in the main sentence is invalid? |
| vivo | Fine with the proposed conclusion. |
| Moderator | @ Huawei, Yes, that is the intention. The case described in Fujitsu’s Tdoc is treated as error case, similar to Rel-15. |
| Fujitsu | We agree that when the DG PDSCH is valid, the DG PDSCH can drop the SPS PDSCH. We are fine with the Note. |
| Samsung | Thanks Moderator. Regarding the note, minor revision is “Note: This applies only when the PDSCH (scheduled by multi-PDSCH DCI) is associated with the last SLIV andis invalid due to collision with semi-static UL symbol(s). Also, can we capture that “no specification updated is expected” since we believe the current specification already includes this restriction. |
| Xiaomi | Ok with the conclusion. |

Issue#4-2: DCI-to-PDSCH OOO (case 5) and timeline of NNK1 (case 6) are based on configured SLIV vs. valid SLIV

| Company | Views |
| --- | --- |
| [1] Huawei | Proposal 2: Support to clarify that “scheduled” PDSCH/PUSCH is the “configured” PDSCH/PUSCH in DCI-to-PDSCH OOO (case 5) and timeline of NNK1 (case 6). |
| [2] ZTE | Proposal 1: For out-of-order scheduling, the rule for OOO scheduling determined is determined based on valid SLIVs. Proposal 2: For a first DCI scheduling multiple PDSCHs and providing an inapplicable value of k1 in its PDSCH-to-HARQ_feedback timing indicator filed, to multiplex the corresponding HARQ-ACK information in a PUCCH or PUSCH in a slot indicated by the PDSCH-to-HARQ_feedback timing indicator filed in a second DCI, only the valid PDSCHs scheduled by the first DCI are considered for definition of the corresponding timeline requirements. |
| [3] InterDigital | Proposal 1: Out-of-order scheduling determination should be based on valid SLIVs. Proposal 2: When a first DCI schedules multiple PDSCHs and provides an inapplicable value of k1 in its PDSCH-to-HARQ_feedback timing indicator filed to multiplex the corresponding HARQ-ACK information in a PUCCH or PUSCH in a slot indicated by the PDSCH-to-HARQ_feedback timing indicator filed in a second DCI, only the valid SLIVs scheduled by the first DCI are considered for definition of the corresponding timeline requirements. |
| [6] vivo | Proposal 1: For multi-PDSCH/PUSCH scheduling, OoO scheduling rules are applied only to valid SLIV(s) in the TDRA row indicated by a scheduling DCI. Proposal 2: For multi-PDSCH scheduling, and for case(s) where Type-2 codebook/enhanced Type-2 codebook is configured and an inapplicable value is provided by a PDSCH-to-HARQ_feedback timing indicator field included in a first DCI scheduling more than one PDSCH, only valid PDSCH(s) scheduled by the first DCI is considered to decide if an SPS PDSCH reception is received after the PDSCH(s) or not. |
| [8] Fujitsu | Observation 3：According to TS38.213 h10, for Type-1 HARQ-ACK codebook with time domain bundling, for a candidate PDSCH reception occasion associated with a single valid PDSCH scheduled by a DCI which indicates a TDRA row including more than one SLIV, the corresponding HARQ-ACK information bit is not correctly set. Proposal 5: For Type-1 HARQ-ACK codebook with time domain bundling, update the pseudo-code in TS 38.213 to clarify the following: - For a candidate PDSCH reception occasion associated with a single valid PDSCH scheduled by a DCI which indicates a TDRA row with more than one SLIV, the corresponding HARQ-ACK information bit is equal to the HARQ-ACK information bit for the single valid PDSCH. Proposal 6: For a first DCI scheduling multiple PDSCHs and providing an inapplicable value of k1 in its PDSCH-to-HARQ_feedback timing indicator filed, to multiplex the corresponding HARQ-ACK information in a PUCCH or PUSCH in a slot indicated by the PDSCH-to-HARQ_feedback timing indicator filed in a second DCI, only the valid PDSCHs scheduled by the first DCI are considered for definition of the corresponding timeline requirements. - TP#1 can be considered. |
| [11] OPPO | Proposal 1 For OOO scheduling, the scheduled PDSCHs/PUSCHs or received PDSCHs/PUSCHs are determined based on valid SLIV. Proposal 2 For a first DCI scheduling multiple PDSCHs and providing an inapplicable value of k1 in its PDSCH-to-HARQ_feedback timing indicator filed, to multiplex the corresponding HARQ-ACK information in a PUCCH or PUSCH in a slot indicated by the PDSCH-to-HARQ_feedback timing indicator filed in a second DCI, only the valid PDSCHs scheduled by the first DCI are considered for definition of the corresponding timeline requirements. |
| [12] Ericsson | Proposal 1 For multi-PDSCH/multi-PUSCH scheduling in Rel-17, out-of-order scheduling is determined based on configured SLIVs. Proposal 2 For multi-PDSCH scheduling in Rel-17, the evaluation of the time-line requirement for application of NN-K1 is based on configured SLIVs. |
| [14] Apple | Proposal 1: The following table indicates our preferences on “configured” vs “valid” for the existing agreements: Case; Issue; Agreement: Configured vs Valid; Apple’s Proposals. Case 5; Out-of-order behavior; Pending; Valid. Case 6; NN-K1; Pending; Valid. |
| [15] NTT DOCOMO | Proposal 1: For OoO scheduling judgement, support to consider “configured SLIV” - No TP is needed |
| [16] Nokia | Proposal 1: Determination of OOO scheduling and NN-K1 timeline requirements are based on the valid SLIVs, without further modification on type-1 codebook determination. Adopt TP#1~#3 in the Appendix. |
| [17] LG Electronics | Proposal #3: Do not consider any invalid PDSCH or PUSCH (which is collided with semi-static UL or DL symbol(s)) to check out-of-order scheduling. Proposal #4: If type-1 HARQ-ACK codebook is configured with time domain bundling and OOO scheduling is determined based on valid PDSCHs, - If there are more than one scheduled PDSCH for a PDSCH reception occasion m, then bundled HARQ-ACK information is generated for the PDSCH associated with the last SLIV. |
| [18] MediaTek | Proposal 2: For out-of-order scheduling, the rule for scheduling is determined based on configured SLIVs indicated by the TDRA information field. |
| [19] Intel | Proposal 2: - Prefer to apply unified handling for Case 5 and Case 6 - Prefer to confirm Case 5 OOO handling is based on configured SLIVs - Otherwise, if OOO handling is defined by the valid SLIVs, it is preferred to apply valid SLIVs in - PUSCH preparation time - PDSCH reception preparation time with cross carrier scheduling with different SCSs for PDCCH and PDSCH - Activated TCI states within multi-PDSCH transmission |
| [20] Qualcomm | Proposal 3: The out-of-order rules should be applied on the valid allocations only. Proposal 4: For a first DCI scheduling multiple PDSCHs and providing an inapplicable value of k1 in its PDSCH-to-HARQ_feedback timing indicator field, to multiplex the corresponding HARQ-ACK information in a PUCCH or PUSCH in a slot indicated by the PDSCH-to-HARQ_feedback timing indicator field in a second DCI, only the valid PDSCHs scheduled by the first DCI are considered for definition of the corresponding timeline requirements. |

Agreement: (RAN1#106bis-e)

For two multi-PDSCH (or two multi-PUSCH) scheduling DCIs, UE does not expect any of the scheduled PDSCHs (or PUSCHs) and the scheduling DCI to lead to out-of-order scheduling.

Conclusion (RAN1#107bis-e)

- UE does not expect any of the scheduled PDSCHs (or PUSCHs) and the scheduling DCIs to lead to out-of-order scheduling, also for the case of one multi-PDSCH (or multi-PUSCH) scheduling DCI and one single-PDSCH (or single-PUSCH) scheduling DCI, where multi-PDSCH (or multi-PUSCH) scheduling DCI schedules more than one PDSCH (or PUSCH).

  - This may not have specification impact.

- Note: It is separately discussed whether the scheduled PDSCHs (or PUSCHs or SLIV) is based on configured SLIV or valid SLIV.

Agreement (RAN1#108-e)

- The case where two multi-PDSCH (or multi-PUSCH) scheduling DCIs end in the same symbol but two multi-PDSCH (or multi-PUSCH) schedulings have overlapping spans, where the span is defined from the beginning of the first scheduled SLIV till the end of the last scheduled SLIV, is considered as out-of-order scheduling and is not expected by UE.

  - This applies also when one of two DCIs is single-PDSCH (or single-PUSCH) scheduling DCI, including the case that one DCI schedules multi-slot PDSCH (or PUSCH repetition type A or B).

  - Note: This doesn’t apply when each of two DCIs schedules multi-slot PDSCH (or PUSCH repetition type A or B) as in Rel-15/Rel-16

  - Note: This doesn’t apply when each of the two DCIs schedules single PDSCH (or single PUSCH) as in Rel-15/Rel-16

- Note: It is separately discussed whether the scheduled SLIV is based on configured SLIV or valid SLIV.

TS 38.214 For any two HARQ process IDs in a given scheduled cell, if the UE is scheduled to start receiving a first PDSCH starting in symbol j by a PDCCH ending in symbol i, the UE is not expected to be scheduled to receive a PDSCH starting earlier than the end of the first PDSCH with a PDCCH that ends later than symbol i. For any two HARQ process IDs in a given scheduled cell, if the UE is scheduled to start a first PUSCH transmission starting in symbol j by a PDCCH ending in symbol i, the UE is not expected to be scheduled to transmit a PUSCH starting earlier than the end of the first PUSCH by a PDCCH that ends later than symbol i. If a UE is configured with pdsch-TimeDomainAllocationListForMultiPDSCH-r17 in which one or more rows contain multiple SLIVs for PDSCH on a DL BWP of a serving cell, when any two DL DCIs end in the same symbol and at least one of the DCIs schedules multiple PDSCHs, the UE does not expect that the scheduled PDSCH(s) by the two DCIs have overlapping spans, where the span associated with a DCI is defined from the beginning of the first scheduled PDSCH or up to the end of the last scheduled PDSCH. If a UE is configured with pusch-TimeDomainAllocationListForMultiPUSCH-r17 in which one or more rows contain multiple SLIVs for PUSCH on a UL BWP of a serving cell, when any two UL DCIs end in the same symbol and at least one of the DCIs scheduling multiple PUSCHs, the UE does not expect that the any scheduled multiple PUSCHs have overlapping spans, where the span associated with a DCI is defined from the beginning of the first scheduled PUSCH till the end of the last scheduled PUSCH. TS 38.213 If a UE receives a first DCI format that the UE detects in a first PDCCH monitoring occasion and includes a PDSCH-to-HARQ_feedback timing indicator field providing an inapplicable value from dl-DataToUL-ACK-r16, - if the UE detects a second DCI format, the UE multiplexes the corresponding HARQ-ACK information in a PUCCH or PUSCH transmission in a slot that is indicated by a value of a PDSCH-to-HARQ_feedback timing indicator field in the second DCI format, where - if the UE is not provided pdsch-HARQ-ACK-Codebook-r16, the UE detects the second DCI format in any PDCCH monitoring occasion after the first one, and where the slot indicated by the value of the PDSCH-to-HARQ_feedback timing indicator field in the second DCI format is no later than a slot for HARQ-ACK information in response to a SPS PDSCH reception, if any, received after the PDSCH scheduled by the first DCI format. - if the UE is provided pdsch-HARQ-ACK-Codebook-r16, the UE detects the second DCI format in any PDCCH monitoring occasion after the first one, and the second DCI format indicates a HARQ-ACK information report for a same PDSCH group index as indicated by the first DCI format as described in clause 9.1.3.3, and where the slot indicated by the value of the PDSCH-to-HARQ_feedback timing indicator field in the second DCI format is no later than a slot for HARQ-ACK information in response to a SPS PDSCH reception, if any, received after the PDSCH scheduled by the first DCI format. - if the UE is provided pdsch-HARQ-ACK-Codebook-r16, the UE receives the second DCI format later than the slot for HARQ-ACK information in response to a SPS PDSCH reception received after the PDSCH scheduled by the first DCI format, and the second DCI format indicates a HARQ-ACK information report for a same PDSCH group index as indicated by the first DCI format as described in clause 9.1.3.3. - if the UE is provided pdsch-HARQ-ACK-OneShotFeedback, the first DCI format does not have associated HARQ-ACK information without scheduling a PDSCH reception or TCI state update, the UE detects the second DCI format in any PDCCH monitoring occasion after the first one, and the second DCI format includes a One-shot HARQ-ACK request field with value 1, the UE includes the HARQ-ACK information in a Type-3 HARQ-ACK codebook, as described in clause 9.1.4, and where the slot indicated by the value of the PDSCH-to-HARQ_feedback timing indicator field in the second DCI format is no later than a slot for HARQ-ACK information in response to a SPS PDSCH reception, if any, received after the PDSCH scheduled by the first DCI format. - if the UE is provided pdsch-HARQ-ACK-OneShotFeedback-r16, the first DCI format does not have associated HARQ-ACK information without scheduling a PDSCH reception or TCI state update, and the UE receives the second DCI format later than the slot for HARQ-ACK information in response to a SPS PDSCH reception received after the PDSCH scheduled by the first DCI format, and the second DCI format includes a One-shot HARQ-ACK request field with value 1, the UE includes the HARQ-ACK information in a Type-3 HARQ-ACK codebook, as described in clause 9.1.4. - otherwise, the UE does not multiplex the corresponding HARQ-ACK information in a PUCCH or PUSCH transmission.

Company views on the highlighted parts above in previous agreements/conclusion and specifications:

- Case 5: For out-of-order scheduling, is the rule for OOO scheduling determined based on configured SLIVs or valid SLIVs?

  - Based on configured SLIVs: Huawei, Ericsson, NTT DOCOMO, MediaTek, Intel

  - Based on valid SLIVs: ZTE, InterDigital, vivo, OPPO, Apple, Nokia, LG Electronics, Qualcomm

- Case 6: For a first DCI scheduling multiple PDSCHs and providing an inapplicable value of k1 in its PDSCH-to-HARQ_feedback timing indicator filed, to multiplex the corresponding HARQ-ACK information in a PUCCH or PUSCH in a slot indicated by the PDSCH-to-HARQ_feedback timing indicator filed in a second DCI, only the [valid] PDSCHs scheduled by the first DCI are considered for definition of the corresponding timeline requirements.

  - Based on configured SLIVs: Huawei, Ericsson, NTT DOCOMO, Intel

  - Based on valid SLIVs: ZTE, InterDigital, vivo, Fujitsu, OPPO, Apple, Nokia, Qualcomm

- [Moderator’s note] It can be observed that

Most companies agree that Case 5 and Case 6 should have a common solution; and

Slightly more companies prefer “based on valid SLIVs”; and

For Case 5, current specification can be interpreted as “based on configured SLIVs”; and

For Case 6, current specification requires further clarification no matter which option is chosen; and

For Case 5, proponents of “based on configured SLIVs” have a concern on the additional impact on type-1 HARQ-ACK CB with time domain bundling configured, while proponents of “based on valid SLIVs” suggested not to further modify type-1 HARQ-ACK CB.

- With those observations, it is suggested to go with one of the following two options:

Option 1: Based on configured SLIVs for Case 5 and Case 6

Option 2: Based on valid SLIVs for Case 5 and Case 6, without further modification on type-1 HARQ-ACK codebook generation

Companies are encouraged to provide views on which option is preferable or is NOT acceptable.

| Company | Views |
| --- | --- |
| Samsung | Option 2 is preferred but open to option 1 for the sake of progress. |
| DOCOMO | Thought we can understand the benefit of Option 2 in terms of scheduling flexibility, we prefer option 1 considering that we are now in the very late stage of maintenance. |
| ZTE, Sanechips | Our 1st preference is option 2. Considering the limited time and progress, we can also go for option1. |
| Intel | This issue is already discussed for several meetings and the pros/cons for each option should be clear now. We continuously support Option 1 for its simplicity and robustness. On the other hand, as we commented several times, if Option 2 is to be supported, we prefer to conclude valid SLIV is also used in some other cases for timeline checking - PUSCH preparation time - PDSCH reception preparation time with cross carrier scheduling with different SCSs for PDCCH and PDSCH - Activated TCI states within multi-PDSCH transmission Taking PUSCH preparation time as example, how to interpret the following figure? If OOO is based on valid SLIV, it doesn’t violate the OOO checking, hence is valid. On the other hand, if PUSCH preparation time is based on configured SLIV, it is invalid. Such a result comes from using different principle to check the two timelines As to activated TCI states within multi-PDSCH transmission, it was agreed that the activated TCI states are derived from the first PDSCH slot and keep unchanged in the span of the multiple PDSCHs. However, if the first PDSCH slot is invalid and if the second PDSCH slot is valid and right after the change of the activated TCI states, gNB has enough processing time to indicate a more appropriate TCI state from the new activated TCI states. |
| Apple | We can accept Option 1 for progress. |
| MediaTek | We support option 1 to avoid further discussion on Type-1 HARQvcodebook design. |
| Ericsson | Support Option 1 for simplicity. We prefer to avoid follow-on work. |
| InterDigital | We prefer Option 2. However, we are open to option 1 for the sake of progress. |
| Nokia, NSB | We prefer Option 2 over Option 1. However, we note that both options are acceptable. |
| Panasonic | We support Option 1. |
| Qualcomm | We support Option 2. |
| Huawei, HiSilicon | As the former proponent for valid SLIV, we acknowledged the benefit. However, considering the late stage of WI and relative more standard change required, we support option 1. As for option2, we are not quite clear how to avoid further modification on type 1 HARQ codebook generation. At least the interpretation on issue #4-1 should be resolved. |
| Fujitsu | We support Option 2 without “without further modification on type-1 HARQ-ACK codebook generation”. (Please note that our intention is not to reverse interpretation 1 discussed in issue #4-1. Interpretation 1 is fine to us.) Regarding Type-1 HARQ-ACK codebook with time domain bundling, no matter we adopt Option 1 or Option 2, it is necessary to further discuss how to update the spec. That is because, the current pseudo-code in spec. as below does not support HARQ-ACK codebook generation for the following case: - For occasion m , a single valid PDSCH is scheduled by a DCI indicating a TDRA row that includes more than one SLIV entry. Detail discussions can be found in our contribution (Issue 3). while m<Mc if enableTimeDomainHARQ-Bundling is provided for serving cell c and a PDSCH associated with occasion m is scheduled by a DCI format indicating a TDRA row that includes more than one SLIV entry if harq-ACK-SpatialBundlingPUCCH is not provided and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for the active DL BWP of serving cell c if the PDSCH is associated with the last SLIV in the TDRA row ojACK = binary AND operation of the HARQ-ACK information bits corresponding to first transport blocks in PDSCH receptions, that do not overlap with an uplink symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, scheduled by the DCI format on serving cell c; j=j+1; ojACK = binary AND operation of the HARQ-ACK information bits corresponding to second transport blocks in PDSCH receptions, that do not overlap with an uplink symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, scheduled by the DCI format on serving cell c; else ojACK= NACK; j=j+1; ojACK= NACK; end if j=j+1; elseif harq-ACK-SpatialBundlingPUCCH is provided and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for the active DL BWP of serving cell c if the PDSCH is associated with the last SLIV in the TDRA row; ojACK = binary AND operation of the HARQ-ACK information bits corresponding to all transport blocks in PDSCHs, that do not overlap with an uplink symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, scheduled by the DCI format of serving cell c if the UE receives one transport block, the UE assumes ACK for the second transport block; else ojACK = NACK; end if j=j+1; else if the PDSCH is associated with the last SLIV in the TDRA row; ojACK =binary AND operation of the HARQ-ACK information bits corresponding to all transport blocks in PDSCHs, that do not overlap with an uplink symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, scheduled by the DCI format of serving cell c else ojACK = NACK; end if j=j+1; end if else …. |
| vivo | Option 2 is preferred for more scheduling flexibility. However, we can accept option 1 for sake of progress. |
| CATT1 | We support Option 2. |
| OPPO | We support Option 2. |
| Moderator | Thanks a lot for being flexible. Majority companies seem to be able to accept Option 1 although this is not their first preference. So I would suggest to go with Option 1 and suggest following proposals. By the way, @ Fujitsu, I have different understanding with you in terms of interpretation of current pseudo code. “a PDSCH associated with occasion m” is a PDSCH scheduled by the DCI, regardless of whether it is invalid or valid. |
| Fujitsu | Many thanks to moderator for sharing your understanding. And it would be appreciated if you can elaborate more.😊 I was wondering, how do you understand the cyan part, e.g. what kinds of PDSCH scheduling would jump to “else”? Let me take the figure below as an example. For the valid PDSCH in slot n-3, associated with occasion 1, how to generate the corresponding HARQ-ACK bit according to the current pseudo code, esp. the cyan part? In generally, in our understanding, even if we assume ““a PDSCH associated with occasion m” is a PDSCH scheduled by the DCI, regardless of whether it is invalid or valid.”, the pseudo code does not support codebook generation for the case and needs to be udpated. And we do suggest to have a separate discussion about it among companies. |
| Moderator2 | @ Fujitsu, Let me elaborate my understanding on the cyan part. For the simplicity, let’s assume there is only one SLIV consisting of SLIV1_0/1_1/1_2 in your figure, and K1={1,2}. Since PDSCH reception occasions are determined only by the last SLIV, there are two PDSCH reception occasions where one is slot n-2 and the other is slot n-1. Now, assuming a DCI schedules slot n-3/n-2/n-1 (but other DCI cannot schedule slot n-4/n-3/n-2 at the same time if we agree OOO based on configured SLIV), First PDSCH reception occasion corresponds to slot n-2, then there is a PDSCH scheduled by the DCI. However, it is not associated with the last SLIV. So, in this case, else part in cyan is applied. Since this is not associated with last SLIV, based on the pseudo code, UE generates NACK which effectively corresponds to slot n-4/n-3/n-2 PDSCHs that cannot be scheduled. Second PDSCH reception occasion corresponds to slot n-1, then there is also a PDSCH scheduled by the DCI. Since the PDSCH is associated with the last SLIV, UE generates bundled HARQ-ACK corresponding to slot n-3 PDSCH (which is only one valid PDSCH in your figure). - Hope this clarifies my understanding. |
| Fujitsu | Thanks for moderator’s detailed clarification. Here is my further thinking. Firstly, it seems there may be different interpretations on “a PDSCH associated with occasion m” in the pseudo code. Based on moderator’s assumption of SLIVs and PDSCH scheduling, I drew a figure as below. I am not sure whether it correctly reflects moderator’s understanding. (Please correct me if I am wrong). For now, let me take it as Interpretation 1. Interpretation 1: “a PDSCH associated with occasion m”is a PDSCH scheduled in the corresponding DL slot of occasion m, and the corresponding DL slot of occasion m is the DL slot where the last SLIV locates for determining occasion m. Previously, we saw another possible interpretation of “a PDSCH associated with occasion m”. Interpretation 2: “a PDSCH associated with occasion m”is a PDSCH of which the corresponding HARQ-ACK information is mapping to occasion m. (Note that, based on Interpretation 2, it would never go to else part in cyan) Then, consider the case where there is only one valid PDSCH scheduled by a DCI indicating a TDRA row with more than one SLIV and the valid PDSCH is not associated with the last SLIV. For example, assume a DCI schedules slot n-3/n-2/n-1 but only PDSCH in slot n-3 is valid, as shown in the following figures. Note that occasion m=0 and m=1 is determined due to valid last SLIV of other TDRA rows, which are omitted for simplify. No matter we follow Interpretation 1 or 2 above, for occasion m=1, we cannot generate HARQ-ACK bit based on the green below in if part in cyan), because it is for the case of more than one valid PDSCHs scheduled by the DCI. ojACK = binary AND operation of the HARQ-ACK information bits corresponding to all transport blocks in PDSCHs, that do not overlap with an uplink symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, scheduled by the DCI format of serving cell c Based on Interpretation 1: Based on Interpretation 2: To sum up, we may need to clarify which interpretation is the common understanding (this is also related to discussion in [Moderator’s note#2]. And both interpretations have problem in generating codebook as explained above. |
| Moderator3 | @ Fujitsu, My understanding seems to be well-captured as Interpretation 1, appreciate it! If I correctly understand your point, we can modify the current specification as follows. ojACK = binary AND operation of the HARQ-ACK information bit(s) corresponding to all transport block(s) in PDSCH(s), that do not overlap with an uplink symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, scheduled by the DCI format of serving cell c Please let me know if it can figure out the issue you raised. |
| OPPO | We support Option 2. Some companies assume to take a unified solution (either valid SLIV or configured SLIV) for all cases, e.g., OOO, Type-1 CB generation, PUSCH preparation time. We think this is not necessary, and we should take best solution for different cases. In the case of OOO, we think Option 2 is the best option. With Option 1, the scheduling should avoid OOO for all the configured SLIVs which put tighter restriction than Option 2, and this restriction is not necessary, as from the UE side, the OOO issue only occurs for valid SLIVs. To Intel’s comments on PUSCH preparation time: We understand the current specification requires UE to check the PUSCH preparation time for each of the valid SLIV in case of single DCI scheduling multi-PUSCH rather than only check the first configured SLIV. Therefore we don’t think the PUSCH preparation is based on configured SLIV. To HW’s comments on Type-1 CB generation: It is not required to use a unified solution for both OOO and Type-1 CB generation. For OOO, valid SLIV can avoid unnecessary scheduling restriction. But it doesn’t mean that Type-1 CB generation should also take valid SLIV option. |
| Fujitsu | Thanks for moderator’s follow-up. I guess now we are on the same page about the issue. However, according to comments from Samsung and Intel in [Moderator’s note#2], It seems to me that their interpretation is Interpretation 2. And how to update the specification depends on which interpretation we assume for the current pseudo code. Either interpretation is fine to us. To make our point clear, we provide TP#1 assuming Interpretation 1 and TP #2 assuming Interpretation 2. TP #1 (assume Interpretation 1): while m<Mc if enableTimeDomainHARQ-Bundling is provided for serving cell c and a PDSCH associated with occasion m is scheduled by a DCI format indicating a TDRA row that includes more than one SLIV entry if harq-ACK-SpatialBundlingPUCCH is not provided and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for the active DL BWP of serving cell c if the PDSCH is associated with the last SLIV in the TDRA row if two or more PDSCH receptions do not overlap with an uplink symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, scheduled by the DCI format on serving cell c ojACK = binary AND operation of the HARQ-ACK information bits corresponding to first transport blocks in PDSCH receptions, that do not overlap with an uplink symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, scheduled by the DCI format on serving cell c; j=j+1; ojACK = binary AND operation of the HARQ-ACK information bits corresponding to second transport blocks in PDSCH receptions, that do not overlap with an uplink symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, scheduled by the DCI format on serving cell c; else ojACK = HARQ-ACK information bit corresponding to a first transport block of a single PDSCH reception, that does not overlap with an uplink symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, scheduled by the DCI format on serving cell c; j=j+1; ojACK = HARQ-ACK information bit corresponding to a second transport block of a single PDSCH reception, that does not overlap with an uplink symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, scheduled by the DCI format on serving cell c; else ojACK= NACK; j=j+1; ojACK= NACK; end if j=j+1; elseif harq-ACK-SpatialBundlingPUCCH is provided and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for the active DL BWP of serving cell c if the PDSCH is associated with the last SLIV in the TDRA row; if two or more PDSCH receptions do not overlap with an uplink symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, scheduled by the DCI format on serving cell c ojACK = binary AND operation of the HARQ-ACK information bits corresponding to all transport blocks in PDSCHs, that do not overlap with an uplink symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, scheduled by the DCI format of serving cell c if the UE receives one transport block, the UE assumes ACK for the second transport block; else ojACK = binary AND operation of the HARQ-ACK information bits corresponding to a first transport block and a second transport block of a single PDSCH reception, that does not overlap with an uplink symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, scheduled by the DCI format on serving cell c; if the UE receives one transport block, the UE assumes ACK for the second transport block; else ojACK = NACK; end if j=j+1; else if the PDSCH is associated with the last SLIV in the TDRA row; if two or more PDSCH receptions do not overlap with an uplink symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, scheduled by the DCI format on serving cell c ojACK =binary AND operation of the HARQ-ACK information bits corresponding to all transport blocks in PDSCHs, that do not overlap with an uplink symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, scheduled by the DCI format of serving cell c else ojACK = HARQ-ACK information bit corresponding to a transport block of a single PDSCH reception, that does not overlap with an uplink symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, scheduled by the DCI format on serving cell c; else ojACK = NACK; end if j=j+1; end if else …. TP #2 (assume Interpretation 2): while m<Mc if enableTimeDomainHARQ-Bundling is provided for serving cell c and a PDSCH associated with occasion m is scheduled by a DCI format indicating a TDRA row that includes more than one SLIV entry if harq-ACK-SpatialBundlingPUCCH is not provided and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for the active DL BWP of serving cell c if the PDSCH is associated with the last SLIV in the TDRA row if two or more PDSCH receptions do not overlap with an uplink symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, scheduled by the DCI format on serving cell c ojACK = binary AND operation of the HARQ-ACK information bits corresponding to first transport blocks in PDSCH receptions, that do not overlap with an uplink symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, scheduled by the DCI format on serving cell c; j=j+1; ojACK = binary AND operation of the HARQ-ACK information bits corresponding to second transport blocks in PDSCH receptions, that do not overlap with an uplink symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, scheduled by the DCI format on serving cell c; else ojACK= NACK; j=j+1; ojACK= NACK; ojACK = HARQ-ACK information bit corresponding to a first transport block of a single PDSCH reception, that does not overlap with an uplink symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, scheduled by the DCI format on serving cell c; j=j+1; ojACK = HARQ-ACK information bit corresponding to a second transport block of a single PDSCH reception, that does not overlap with an uplink symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, scheduled by the DCI format on serving cell c; end if j=j+1; elseif harq-ACK-SpatialBundlingPUCCH is provided and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for the active DL BWP of serving cell c if the PDSCH is associated with the last SLIV in the TDRA row; if two or more PDSCH receptions do not overlap with an uplink symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, scheduled by the DCI format on serving cell c ojACK = binary AND operation of the HARQ-ACK information bits corresponding to all transport blocks in PDSCHs, that do not overlap with an uplink symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, scheduled by the DCI format of serving cell c if the UE receives one transport block, the UE assumes ACK for the second transport block; else ojACK = NACK; ojACK = binary AND operation of the HARQ-ACK information bits corresponding to a first transport block and a second transport block of a single PDSCH reception, that does not overlap with an uplink symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, scheduled by the DCI format on serving cell c; if the UE receives one transport block, the UE assumes ACK for the second transport block; end if j=j+1; else if the PDSCH is associated with the last SLIV in the TDRA row; if two or more PDSCH receptions do not overlap with an uplink symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, scheduled by the DCI format on serving cell c ojACK =binary AND operation of the HARQ-ACK information bits corresponding to all transport blocks in PDSCHs, that do not overlap with an uplink symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, scheduled by the DCI format of serving cell c else ojACK = NACK; ojACK = HARQ-ACK information bit corresponding to a transport block of a single PDSCH reception, that does not overlap with an uplink symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, scheduled by the DCI format on serving cell c; end if j=j+1; end if else …. |
| Huawei, HiSilicon | To OPPO’s comment: I think the issue of type 1 CB is due to there might be PDSCHs scheduled by different DCI mapping to the same PDSCH reception occasions. It is a mismatch between PDSCH reception occasion determination based on the last configured SLIV of TDRA table and OOO determination based on valid SLIV. |
| OPPO | To HW’s comment: As discussed in our contribution, if configured SLIV is used, UE is not expected to be scheduled with this case since it would be taken as “configured SLIV OOO”. Therefore, this case should be avoided by gNB scheduling anyway no matter valid SLIV or configured SLIV is used, hence no impact on Type-1 CB generation is expected if valid SLIV is used. |
| Moderator4 | @ Fujitsu, Thank you very much for providing TP based on 2 interpretations. I agree that we are on the same page. Although I understood what the problem is, that problem is not in the scope of this email discussion. I recommend to discuss that problem in the next meeting. By the way, one comment for your TP from my side is that we need to consider not only the case where only one PDSCH is validated due to collision with semi-static UL, but also the case where TDRA row index associated with the PDSCH reception occasion includes a single SLIV. @ OPPO, I understand your point. For type-1 HARQ-ACK CB configured with time domain bundling, gNB has a scheduling restriction to avoid OOO, regardless of OOO rule is determined based on configured SLIV or based on valid SLIV. However, considering the additional specification impact for OOO based on valid SLIV, clear majority companies start accepting Option 1. So, it would be highly appreciated if you could reconsider this situation to move forward. |
| Fujitsu | Thanks for Moderator’s comments. 😊 Regarding “the case where TDRA row index associated with the PDSCH reception occasion includes a single SLIV”, my understanding is, it is covered by other parts of the pseudo code, but not shown in the TPs. Anyway, we are open to further discussing that case as well. |
| Samsung | Regarding two TPs from Fujitsu, we don’t need to change the pseudo-code structure if it works. For the single valid PDSCH case, we understood what is the problem and we are ok to further discuss the issue in the next meeting. |
| OPPO | @Moderator, For sake of progress, we can accept Option 1. 😊 |

- Proposed Conclusion #4-2-1 (OOO):

For a DCI that can schedule multiple PDSCHs or multiple PUSCHs,

- It is clarified that the scheduled PDSCHs (or PUSCHs or SLIVs) in the following previous conclusion/agreement correspond to scheduled PDSCHs (or PUSCHs or SLIVs) indicated by the TDRA information field.

  - NOTE: No spec change is needed.

Conclusion (RAN1#107bis-e) - UE does not expect any of the scheduled PDSCHs (or PUSCHs) and the scheduling DCIs to lead to out-of-order scheduling, also for the case of one multi-PDSCH (or multi-PUSCH) scheduling DCI and one single-PDSCH (or single-PUSCH) scheduling DCI, where multi-PDSCH (or multi-PUSCH) scheduling DCI schedules more than one PDSCH (or PUSCH). - This may not have specification impact. - Note: It is separately discussed whether the scheduled PDSCHs (or PUSCHs or SLIV) is based on configured SLIV or valid SLIV. Agreement (RAN1#108-e) - The case where two multi-PDSCH (or multi-PUSCH) scheduling DCIs end in the same symbol but two multi-PDSCH (or multi-PUSCH) schedulings have overlapping spans, where the span is defined from the beginning of the first scheduled SLIV till the end of the last scheduled SLIV, is considered as out-of-order scheduling and is not expected by UE. - This applies also when one of two DCIs is single-PDSCH (or single-PUSCH) scheduling DCI, including the case that one DCI schedules multi-slot PDSCH (or PUSCH repetition type A or B). - Note: This doesn’t apply when each of two DCIs schedules multi-slot PDSCH (or PUSCH repetition type A or B) as in Rel-15/Rel-16 - Note: This doesn’t apply when each of the two DCIs schedules single PDSCH (or single PUSCH) as in Rel-15/Rel-16 - Note: It is separately discussed whether the scheduled SLIV is based on configured SLIV or valid SLIV.

Companies are encouraged to provide views on Proposed Conclusion #4-2-1.

| Company | Views |
| --- | --- |
| Xiaomi | OK with the conclusion. |
| Samsung | Ok with the conclusion |
| Fujitsu | OK with the conclusion. |
| Intel | OK with the conclusion. |
| Ericsson | Support Conclusion #4-2-1 |
| Qualcomm | We are okay with the conclusion |
| Huawei, HiSilicon | Ok with the conclusion |
| Apple | Fine with conclusion. |
| vivo | Fine with the conclusion. |
| ZTE, Sanechips | OK with the conclusion. |
| Moderator | This proposal seems stable, so it can be suggested for email approval. |

- Proposal #4-2-2 (NN-K1):

Adopt the following text proposal to TS38.213 v17.1.0 Clause 9.1.3.

- Reason for change

  - UE behaviour for type-2 HARQ-ACK codebook generation is unclear when multi-PDSCH scheduling DCI provides an inapplicable value for HARQ feedback timing.

- Summary of change

  - For a first DCI scheduling multiple PDSCHs and providing an inapplicable value of k1 in its PDSCH-to-HARQ_feedback timing indicator filed, to multiplex the corresponding HARQ-ACK information in a PUCCH or PUSCH in a slot indicated by the PDSCH-to-HARQ_feedback timing indicator filed in a second DCI, the last PDSCHs scheduled by the first DCI are considered for definition of the corresponding timeline requirements.

- Consequences if not approved

  - Unclear UE behaviour for type-2 HARQ-ACK codebook generation when multi-PDSCH scheduling DCI provides an inapplicable value for HARQ feedback timing

--------------------------------------------Start of text proposal to TS38.213 v17.1.0 -------------------------------------------

9.1.3 Type-2 HARQ-ACK codebook determination

< Unchanged parts are omitted >

If a UE receives a first DCI format that the UE detects in a first PDCCH monitoring occasion and includes a PDSCH-to-HARQ_feedback timing indicator field providing an inapplicable value from dl-DataToUL-ACK-r16,

-	if the UE detects a second DCI format, the UE multiplexes the corresponding HARQ-ACK information in a PUCCH or PUSCH transmission in a slot that is indicated by a value of a PDSCH-to-HARQ_feedback timing indicator field in the second DCI format, where

-	if the UE is not provided pdsch-HARQ-ACK-Codebook-r16, the UE detects the second DCI format in any PDCCH monitoring occasion after the first one, and where the slot indicated by the value of the PDSCH-to-HARQ_feedback timing indicator field in the second DCI format is no later than a slot for HARQ-ACK information in response to a SPS PDSCH reception, if any, received after the PDSCH(s) scheduled by the first DCI format.

-	if the UE is provided pdsch-HARQ-ACK-Codebook-r16, the UE detects the second DCI format in any PDCCH monitoring occasion after the first one, and the second DCI format indicates a HARQ-ACK information report for a same PDSCH group index as indicated by the first DCI format as described in clause 9.1.3.3, and where the slot indicated by the value of the PDSCH-to-HARQ_feedback timing indicator field in the second DCI format is no later than a slot for HARQ-ACK information in response to a SPS PDSCH reception, if any, received after the PDSCH(s) scheduled by the first DCI format.

-	if the UE is provided pdsch-HARQ-ACK-Codebook-r16, the UE receives the second DCI format later than the slot for HARQ-ACK information in response to a SPS PDSCH reception received after the PDSCH(s) scheduled by the first DCI format, and the second DCI format indicates a HARQ-ACK information report for a same PDSCH group index as indicated by the first DCI format as described in clause 9.1.3.3.

-	if the UE is provided pdsch-HARQ-ACK-OneShotFeedback, the first DCI format does not  have associated HARQ-ACK information without scheduling a PDSCH reception or TCI state update, the UE detects the second DCI format in any PDCCH monitoring occasion after the first one, and the second DCI format includes a One-shot HARQ-ACK request field with value 1, the UE includes the HARQ-ACK information in a Type-3 HARQ-ACK codebook, as described in clause 9.1.4, and where the slot indicated by the value of the PDSCH-to-HARQ_feedback timing indicator field in the second DCI format is no later than a slot for HARQ-ACK information in response to a SPS PDSCH reception, if any, received after the PDSCH(s) scheduled by the first DCI format.

-	if the UE is provided pdsch-HARQ-ACK-OneShotFeedback-r16, the first DCI format does not have associated HARQ-ACK information without scheduling a PDSCH reception or TCI state update, and the UE receives the second DCI format later than the slot for HARQ-ACK information in response to a SPS PDSCH reception received after the PDSCH(s) scheduled by the first DCI format, and the second DCI format includes a One-shot HARQ-ACK request field with value 1, the UE includes the HARQ-ACK information in a Type-3 HARQ-ACK codebook, as described in clause 9.1.4.

-	otherwise, the UE does not multiplex the corresponding HARQ-ACK information in a PUCCH or PUSCH transmission.

--------------------------------------------End of Text proposal to TS38.213 v17.1.0 --------------------------------------------

Companies are encouraged to provide views on Proposal #4-2-2.

| Company | Views |
| --- | --- |
| Xiaomi | OK with the TP. |
| Samsung | Ok with the TP |
| Fujitsu | OK with the TP |
| Intel | OK with the TP. |
| ASUSTeK | OK with the TP. |
| Ericsson | Support Proposal #4-2-2 |
| Qualcomm | We are okay with the proposal |
| Huawei, HiSilicon | Support the proposal and TP |
| Apple | Fine with TP |
| vivo | OK with the TP. |
| ZTE, Sanechips | OK with the TP. |
| Moderator | This proposal seems stable, so it can be suggested for email approval. |

Issue#4-3: Type-1 HARQ CB for multiple PDSCHs scheduled by single DCI 1-1 and slot-aggregated PDSCH scheduled by DCI 1-2

| Company | Views |
| --- | --- |
| [1] Huawei | Proposal 4: Support configuring enableTimeDomainHARQ-Bundling for type 1 HARQ codebook feedback for multiple PDSCHs scheduled by single DCI 1-1 and pdsch-AggregationFactor for PDSCH scheduled by DCI 1-2 at same time. TP#1 is preferred. ---------------------------start of TP#1 for TS 38.213 Clause 9.1.2.1------------------------------------- 9.1.2.1 Type-1 HARQ-ACK codebook in physical uplink control channel ****Unchanged Text Omitted**** “elseif the UE is provided enableTimeDomainHARQ-Bundling and tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot n0,k+nD- ∆K0,r(d), at least one symbol of the PDSCH time resource derived by row r of set R' is configured as UL, where d = 0,1,…,C∆K0,r-1, ∆K0,r=maxK0K0,r-K0,r, and C∆K0,r is the cardinality of ∆K0,r and for each slot from n0,k+nD-NPDSCHrepeat,max+1 to slot n0,k+nD, at least one symbol of the PDSCH time resource derived by row r of set R is configured as UL if the row r of set R belongs to time domain resource allocation table configured for DCI format 1_2” ****Unchanged Text Omitted**** ----------------------------end of TP#1 for TS 38.213 Clause 9.1.2.1-------------------------------------- |
| [5] CATT | Proposal 3：Further optimization is not preferred for type-1 HARQ-ACK generation when both NPDSCHrepeat,max are more than 1 and enableTimeDomainHARQ-Bundling is provided. Approve TP-1 in Appendix. ------------------------------Start of TP#1 for TS 38.213 Clause 9.1.2.1 ---------------------------------- - 9.1.2.1 Type-1 HARQ-ACK codebook in physical uplink control channel ======================== Unchanged Text Omitted ======================== else if the UE is provided enableTimeDomainHARQ-Bundling and tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot nU-K1,k⋅2μDL-μUL+nD- ∆K0,r(d), at least one symbol of each PDSCH time resource derived by row r of set R' is configured as UL, where d = 0,1,…,C∆K0,r-1, ∆K0,r=maxK0K0,r-K0,r, and C∆K0,r is the cardinality of ∆K0,r. and for each slot from n0,k+nD-NPDSCHrepeat,max+1 to slotn0,k+nD, at least one symbol of the PDSCH time resource derived by row r of set R is configured as UL - -------------------------------------------------End of TP#1---------------------------------------------------- |
| [10] Samsung | Proposal 5: To support multi-PDSCH scheduling by DCI format 1_1 and PDSCH repetition by DCI format 1_2 in type-1 HARQ-ACK CB, a row r in the set R’ and the set R is removed when the both conditions are met - Condition 1 for multi-PDSCH scheduling) each SLIVs of the TDRA row r overlapped with a semi-static UL symbol - Condition 2 for PDSCH repetition) the last SLIV of the TDRA row r over K slots overlapped with a semi-static UL symbol. - Adopt TP#3 in Appendix for TS38.213 ======================= Start of TP #3 for TS 38.213 ======================== 9.1.2.1 Type-1 HARQ-ACK codebook in physical uplink control channel ======================== Unchanged Text Omitted ========================= while r<CR if the UE is not provided enableTimeDomainHARQ-Bundling and is provided tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot from slot n0,k+nD-NPDSCHrepeat,max+1 to slot n0,k+nD, at least one symbol of the PDSCH time resource derived by row r is configured as UL where K1,k is the k-th slot timing value in set K1, where n0,k is a DL slot with a smallest index among DL slots overlapping with UL slot nU-K1,k, or subslotLengthForPUCCH is provided for the HARQ-ACK codebook and the end of the PDSCH time resource for row r is not within any UL slot nU-K1,l, 0≤l<CK1, or if PDSCH-TimeDomainResourceAllocationListForMultiPDSCH is provided and HARQ-ACK information for PDSCH time resource derived by row r in slot n0,knU-K1,k⋅2μDL-μUL+nD cannot be provided in slot nU R=R\r; elseif the UE is provided enableTimeDomainHARQ-Bundling and tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot n0,knU-K1,k⋅2μDL-μUL+nD- ∆K0,r(d), at least one symbol of each PDSCH time resource derived by row r of set R' is configured as UL, where d = 0,1,…,C∆K0,r-1, ∆K0,r=maxK0K0,r-K0,r, and C∆K0,r is the cardinality of ∆K0,r and for each slot from n0,k+nD-NPDSCHrepeat,max+1 to slot n0,k+nD, at least one symbol of the PDSCH time resource derived by row r of set R is configured as UL R=R\r; R'=R'\r; else r=r+1; end if end while ======================= End of TP #3 for TS 38.213 ========================= |
| [12] Ericsson | Proposal 4 Simultaneous configuration of time domain bundling for multi-PDSCH scheduled by DCI 1_1 and repetition for PDSCH scheduled by DCI 1_2 is allowed. Proposal 5 Adopt TP-1 for Type-1 HARQ-ACK codebook in physical uplink control channel (TS 38.213, Section 9.1.2.1) for multi-PDSCH scheduling. -------------------------------Start of TP-1 for TS 38.213 Clause 9.1.2.1 ---------------------------------- 9.1.2.1 Type-1 HARQ-ACK codebook in physical uplink control channel ======================== Unchanged Text Omitted ========================= while r<CR if the UE is not provided enableTimeDomainHARQ-Bundling and is provided tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot from slot n0,k+nD-NPDSCHrepeat,max+1 to slot n0,k+nD, at least one symbol of the PDSCH time resource derived by row r is configured as UL if row r of set R belongs to time domain resource allocation table that only includes single SLIV rows, or at least one symbol of the PDSCH time resource derived by row r in slot n0,k+nD is configured as UL if row r of set R belongs to time domain resource allocation table that includes at least one row indicating multiple SLIVs, where K1,k is the k-th slot timing value in set K1, where n0,k is a DL slot with a smallest index among DL slots overlapping with UL slot nU-K1,k, or subslotLengthForPUCCH is provided for the HARQ-ACK codebook and the end of the PDSCH time resource for row r is not within any UL slot nU-K1,l, 0≤l<CK1, or if HARQ-ACK information for PDSCH time resource derived by row r in slot n0,knU-K1,k⋅2μDL-μUL+nD cannot be provided in slot nU R=R\r; elseif the UE is provided enableTimeDomainHARQ-Bundling and tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot n0,knU-K1,k⋅2μDL-μUL+nD- ∆K0,r(d), at least one symbol of the PDSCH time resource derived by row r of set R' is configured as UL, where d = 0,1,…,C∆K0,r-1, ∆K0,r=maxK0K0,r-K0,r, and C∆K0,r is the cardinality of ∆K0,r and for each slot from n0,k+nD-NPDSCHrepeat,max+1 to slot n0,k+nD, at least one symbol of the PDSCH time resource derived by row r of set R is configured as UL if the row r of set R belongs to time domain resource allocation table configured for DCI format 1_2 R=R\r; R'=R'\r; else r=r+1; end if end while -------------------------------------------------End of TP-1----------------------------------------------------- |
| [16] Nokia | Proposal 3: Type-1 codebook determination is modified to support PDSCH repetitions also when the HARQ-ACK time domain bundling is configured. Adopt the TP#4 in Appendix. TP#4: TP#G from [4] for type-1 codebook supporting PDSCH repetitions also with HARQ-ACK time domain bundling -----------------------------Start of TP#G for TS 38.213 Clause 9.1.2.1 ---------------------------------- 9.1.2.1 Type-1 HARQ-ACK codebook in physical uplink control channel ======================== Unchanged Text Omitted ========================= while r<CR if the UE is not provided enableTimeDomainHARQ-Bundling and is provided tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot from slot n0,k+nD-NPDSCHrepeat,max+1 to slot n0,k+nD, at least one symbol of the PDSCH time resource derived by row r is configured as UL where K1,k is the k-th slot timing value in set K1, where n0,k is a DL slot with a smallest index among DL slots overlapping with UL slot nU-K1,k, or subslotLengthForPUCCH is provided for the HARQ-ACK codebook and the end of the PDSCH time resource for row r is not within any UL slot nU-K1,l, 0≤l<CK1, or if HARQ-ACK information for PDSCH time resource derived by row r in slot n0,knU-K1,k⋅2μDL-μUL+nD cannot be provided in slot nU R=R\r; elseif the UE is provided enableTimeDomainHARQ-Bundling and tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot n0,knU-K1,k⋅2μDL-μUL+nD- ∆K0,r(d), at least one symbol of the PDSCH time resource derived by row r of set R' is configured as UL, where d = 0,1,…,C∆K0,r-1, ∆K0,r=maxK0K0,r-K0,r, and C∆K0,r is the cardinality of ∆K0,r and for each slot from n0,k+nD-NPDSCHrepeat,max+1 to slot n0,k+nD, at least one symbol of the PDSCH time resource derived by row r of set R is configured as UL R=R\r; R'=R'\r; else r=r+1; end if end while ------------------------------------------End of TP#G--------------------------------------------- |

- [Moderator’s note] It seems that all companies agree to support multi-PDSCH scheduling by DCI format 1_1 and PDSCH repetition by DCI format 1_2 in type-1 HARQ-ACK codebook, but have slightly different views on how to implement it in the specification. Companies are encourage to provide views on which TP is preferable or is NOT acceptable.

| Company | Views |
| --- | --- |
| Samsung | Regarding Ericsson’s TP on “if row r of set R belongs to time domain resource allocation table that only includes single SLIV rows, or at least one symbol of the PDSCH time resource derived by row r in slot n0,k+nD is configured as UL if row r of set R belongs to time domain resource allocation table that includes at least one row indicating multiple SLIVs”, it is not needed since the set R has single SLIV rows only due to the K1 set extension. Regarding the following two options, both options work. Option 2 can provide less overhead but additional complexity to check TDRA table for DCI format 1_2. We would like to simpler solution at this late stage of Rel-17, i.e., prefer Option 1. But we are also fine with Option 2 if majority companies support Option 2. - Option 1: and for each slot from n0,k+nD-NPDSCHrepeat,max+1 to slot n0,k+nD, at least one symbol of the PDSCH time resource derived by row r of set R is configured as UL - Option 2: and for each slot from n0,k+nD-NPDSCHrepeat,max+1 to slot n0,k+nD, at least one symbol of the PDSCH time resource derived by row r of set R is configured as UL if the row r of set R belongs to time domain resource allocation table configured for DCI format 1_2 |
| ZTE, Sanechips | Compared with other TPs, we slightly prefer TP1 from HW. |
| Intel | For the simultaneous configuration of multi-PDSCH scheduled by DCI 1_1 and PDSCH repetitions scheduled by DCI 1_2, we share the views from other companies that the time domain bundling for the multi-PDSCH transmission must be configured. Further, it would be helpful to make a clear conclusion whether such limitation is also applicable to Type2 or Type3 HARQ-ACK codebook |
| Apple | Prefer Option 1. |
| Nokia, NSB | Three companies (CATT, Samsung, Nokia) have made essentially similar text proposals which is also preferred by us. |
| Huawei, HiSilicon | We observed the major difference of TPs from companies is whether to explicitly mention row r is from TDRA of DCI 1-2. To our understanding, it is necessary because the whole paragraph is under the condition of “elseif the UE is provided enableTimeDomainHARQ-Bundling”, which is specifically configured for multiple PDSCH scheduling. If there is no explicit wording about DCI 1-2, it may cause confusion that PDSCH scheduled by DCI 1-1 with single SLIV can also use slot aggregation while there are entries of multi SLIV configured as well. |
| vivo | From the two options provided by Samsung, Option 2 is slightly preferred for potential overhead reduction. |
| CATT | We share the view from Nokia |
| OPPO | We slightly prefer Option 2 provided by Samsung. |
| Moderator | Between Option 1 and Option 2 provided by Samsung, I don’t see any clear majority. In my view, Option 2 seems to be a better approach in that it can avoid potential confusion. @ Intel, Could you elaborate what kind of clarification is needed for type-2 or type-3 HARQ-ACK codebook? |

- Proposal #4-3 (Type-1 CB):

Adopt the following text proposal to TS38.213 v17.1.0 Clause 9.1.2.1.

- Reason for change

  - Type-1 HARQ-ACK codebook generation mechanism is missing for PDSCHs scheduled by multi-PDSCH scheduling DCI (i.e., DCI format 1_1) and slot-aggregated PDSCH scheduled by DCI format 1_2.

- Summary of change

  - Add the condition in the pseudo code that slot aggregated PDSCH scheduled by DCI format 1_2 can be excluded if all corresponding PDSCH repetitions are invalid due to collision with semi-static UL symbols, in case time domain bundling is configured.

- Consequences if not approved

  - Undefined type-1 HARQ-ACK codebook generation mechanism for PDSCHs scheduled by multi-PDSCH scheduling DCI (i.e., DCI format 1_1) and slot-aggregated PDSCH scheduled by DCI format 1_2.

--------------------------------------------Start of text proposal to TS38.213 v17.1.0 -------------------------------------------

9.1.2.1 Type-1 HARQ-ACK codebook in physical uplink control channel

< Unchanged parts are omitted >

while k<CK1

if modnU-K1,k+1,max2μUL-μDL,1=0 or subslotLengthForPUCCH is provided for the HARQ-ACK codebook

Set nD=0 – index of a DL slot overlapping with an UL slot

Set Nk to a number of DL slots overlapping with UL slot nU-K1,k if subslotLengthForPUCCH is provided for the HARQ-ACK codebook; otherwise, Nk=max2μDL-μUL,1

while nD<Nk

if PDSCH-TimeDomainResourceAllocationListForMultiPDSCH and enableTimeDomainHARQ-Bundling are provided for serving cell c

R=RT;

R'=RT';

elseif PDSCH-TimeDomainResourceAllocationListForMultiPDSCH is provided and enableTimeDomainHARQ-Bundling is not provided for serving cell c

R=RT;

else

Set R to the set of rows

end if

Set CR to the cardinality of R

Set r=0 – index of row in set R

if slot nU starts at a same time as or after a slot for an active DL BWP change on serving cell c or an active UL BWP change on the PCell and slot n0,k+nD is before the slot for the active DL BWP change on serving cell c or the active UL BWP change on the PCell, or subslotLengthForPUCCH is provided for the HARQ-ACK codebook and slot n0,k+nD overlaps with UL slot nU-K1,k-1, k>0, where n0,k is a DL slot with a smallest index among DL slots overlapping with UL slot nU-K1,k,

nD=nD+1;

else

while r<CR

if the UE is not provided enableTimeDomainHARQ-Bundling and is provided tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot from slot n0,k+nD-NPDSCHrepeat,max+1 to slot n0,k+nD, at least one symbol of the PDSCH time resource derived by row r is configured as UL where K1,k is the k-th slot timing value in set K1, where n0,k is a DL slot with a smallest index among DL slots overlapping with UL slot nU-K1,k, or subslotLengthForPUCCH is provided for the HARQ-ACK codebook and the end of the PDSCH time resource for row r is not within any UL slot nU-K1,l, 0≤l<CK1, or if PDSCH-TimeDomainResourceAllocationListForMultiPDSCH is provided and HARQ-ACK information for PDSCH time resource derived by row r in slot n0,knU-K1,k⋅2μDL-μUL+nD cannot be provided in slot nU

R=R\r;

elseif the UE is provided enableTimeDomainHARQ-Bundling and tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot n0,knU-K1,k⋅2μDL-μUL+nD- ∆K0,r(d), at least one symbol of each PDSCH time resource derived by row r of set R' is configured as UL and for each slot from n0,k+nD-NPDSCHrepeat,max+1 to slot n0,k+nD, at least one symbol of the PDSCH time resource derived by row r of set R is configured as UL if the row r of set R belongs to time domain resource allocation table configured for DCI format 1_2, where d = 0,1,…,C∆K0,r-1, ∆K0,r=maxK0K0,r-K0,r, and C∆K0,r is the cardinality of ∆K0,r.

R=R\r;

R'=R'\r;

else

r=r+1;

end if

end while

< Unchanged parts are omitted >

while k<CK1

if modnU-K1,k+(Nslot,offsetUL2μoffset,UL-Nslot,offset,cDL2μoffset,DL,c)∙2μUL+1,max⁡(2μUL-μDL,1)=0 or subslotLengthForPUCCH is provided for the HARQ-ACK codebook

Set nD=0 – index of a DL slot overlapping with an UL slot

Set Nk to a number of DL slots overlapping with UL slot nU-K1,k if subslotLengthForPUCCH is provided for the HARQ-ACK codebook; otherwise, Nk=max2μDL-μUL,1

while nD<Nk

if PDSCH-TimeDomainResourceAllocationListForMultiPDSCH and enableTimeDomainHARQ-Bundling are provided for serving cell c

R=RT;

R'=RT';

elseif PDSCH-TimeDomainResourceAllocationListForMultiPDSCH is provided and enableTimeDomainHARQ-Bundling is not provided for serving cell c

R=RT;

else

Set R to the set of rows

end if

Set CR to the cardinality of R

Set r=0 – index of row in set R

if slot nU starts at a same time as or after a slot for an active DL BWP change on serving cell c or an active UL BWP change on the PCell and slot  n0,k+nD is before the slot for the active DL BWP change on serving cell c or the active UL BWP change on the PCell where n0,k is a DL slot with a smallest index among DL slots overlapping with UL slot nU-K1,k, or subslotLengthForPUCCH is provided for the HARQ-ACK codebook and slot n0,k+nD overlaps with UL slot nU-K1,k-1, k>0,

nD=nD+1;

else

while r<CR

if the UE is not provided enableTimeDomainHARQ-Bundling and is provided tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot from slot n0,k+nD-NPDSCHrepeat,max+1 to slot n0,k+nD, at least one symbol of the PDSCH time resource derived by row r is configured as UL where K1,k is the k-th slot timing value in set K1, where n0,k is a DL slot with a smallest index among DL slots overlapping with UL slot nU-K1,k, or subslotLengthForPUCCH is provided for the HARQ-ACK codebook and the end of the PDSCH time resource for row r is not within any UL slot nU-K1,l, 0≤l<CK1 or if PDSCH-TimeDomainResourceAllocationListForMultiPDSCH is provided and HARQ-ACK information for PDSCH time resource derived by row r in slot n0,knU-K1,k⋅2μDL-μUL+Nslot,offsetUL2μoffset,UL-Nslot,offset,cDL2μoffset,DL,c∙2μDL+nD cannot be provided in slot nU

R=R\r;

elseif the UE is provided enableTimeDomainHARQ-Bundling and tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot n0,knU-K1,k⋅2μDL-μUL+Nslot,offsetUL2μoffset,UL-Nslot,offset,cDL2μoffset,DL,c∙2μDL+nD- ∆K0,r(d), at least one symbol of each PDSCH time resource derived by row r of set R' is configured as UL and for each slot from n0,k+nD-NPDSCHrepeat,max+1 to slot n0,k+nD, at least one symbol of the PDSCH time resource derived by row r of set R is configured as UL if the row r of set R belongs to time domain resource allocation table configured for DCI format 1_2, where d = 0,1,…,C∆K0,r-1.

R=R\r;

R'=R'\r;

else

r=r+1;

end if

end while

--------------------------------------------End of Text proposal to TS38.213 v17.1.0 --------------------------------------------

Companies are encouraged to provide views on Proposal #4-3.

| Company | Views |
| --- | --- |
| Samsung | For the sake of progress, we are ok to the TP. |
| Intel | We are OK with the TP |
| Ericsson | Support Proposal #4-3 |
| Qualcomm | We are okay with the proposal |
| Huawei, HiSilicon | Support the proposal and TP |
| Apple | Fine with TP |
| vivo | We are fine with the TP. One minor comment for the summary of change part: “Corresponding PDSCHs” should be “Corresponding PDSCH repetitions” since slot-aggregated PDSCH is actually a single PDSCH? |
| Moderator | @ vivo, Thanks for spotting that point. I revised it accordingly. |
| ZTE, Sanechips | We are fine with the updated TP |
| Moderator | This proposal seems stable, so it can be suggested for email approval. |

(Closed) Issue#4-5: Collision handling between PUSCH and CORESET#0

| Company | Views |
| --- | --- |
| [4] Panasonic | Proposal 1: Support to update the previous agreement with the following change For multiple PDSCHs (or PUSCHs) scheduled by a single DCI, - Rel-15/16 behavior that is described in TS 38.213 Clauses 11 and 11.1 for a PDSCH (or PUSCH) indicated by DCI also applies for multiple PDSCHs (or PUSCHs) schedule by a single DCI. - If one of multiple PDSCHs (or PUSCHs) scheduled by the DCI collides with a flexible symbol (indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated), - If that PUSCH is collided with SSB symbols indicated by ssb-PositionsInBurst [or symbol(s) indicated by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set], the HARQ process number increment is skipped for the PUSCH. - Otherwise, the HARQ process number increment is not skipped for that PDSCH (or PUSCH). |
| [5] CATT | Proposal 2：If a PUSCH is collided with symbol(s) indicated by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set, the PUSCH transmission is skipped and the HARQ process number increment is skipped. |
| [14] Apple | Proposal 2: HARQ Process Number is NOT skipped if PUSCH collides with CORESET #0. |
| [19] Intel | Proposal 1 - If a PUSCH is collided with symbol(s) indicated by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set, the HARQ process number increment is not skipped for the PUSCH. - No TP is needed for HARQ process number increment for invalid PUSCH. |

Agreement: (RAN1#106bis-e)

For multiple PDSCHs (or PUSCHs) scheduled by a single DCI,

- Rel-15/16 behavior that is described in TS 38.213 Clauses 11 and 11.1 for a PDSCH (or PUSCH) indicated by DCI also applies for multiple PDSCHs (or PUSCHs) schedule by a single DCI.

- If one of multiple PDSCHs (or PUSCHs) scheduled by the DCI collides with a flexible symbol (indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated),

  - If that PUSCH is collided with SSB symbols indicated by ssb-PositionsInBurst [or symbol(s) indicated by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set], the HARQ process number increment is skipped for the PUSCH.

  - Otherwise, the HARQ process number increment is not skipped for that PDSCH (or PUSCH).

Company views on highlighted part above:

- If that PUSCH is collided with SSB symbols indicated by ssb-PositionsInBurst or symbol(s) indicated by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set, the HARQ process number increment is skipped for the PUSCH.

  - Supported by CATT

  - Objected by Panasonic, Apple, Intel

- [Moderator’s note] Even though the number of inputs is quite small, it is suggested to go with the direction that is supported by more companies and does not require additional specification impact.

- Proposal #4-5 (Collision btw PUSCH and CORESET#0):

- Update the previous agreement made in RAN1#106bis-e, as follows:

  - NOTE: No spec change is needed.

Agreement: (RAN1#106bis-e)

For multiple PDSCHs (or PUSCHs) scheduled by a single DCI,

- Rel-15/16 behavior that is described in TS 38.213 Clauses 11 and 11.1 for a PDSCH (or PUSCH) indicated by DCI also applies for multiple PDSCHs (or PUSCHs) schedule by a single DCI.

- If one of multiple PDSCHs (or PUSCHs) scheduled by the DCI collides with a flexible symbol (indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated),

  - If that PUSCH is collided with SSB symbols indicated by ssb-PositionsInBurst [or symbol(s) indicated by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set], the HARQ process number increment is skipped for the PUSCH.

  - Otherwise, the HARQ process number increment is not skipped for that PDSCH (or PUSCH).

Companies are encouraged to provide views on Proposal #4-5.

| Company | Views |
| --- | --- |
| Samsung | We can conclude the discussion by supporting Proposal #4-5. |
| ZTE, Sanechips | Support proposal #4-5. |
| Intel | We are fine with the proposal. |
| Apple | We support the proposal. |
| Ericsson | Support proposal #4-5 |
| InterDigital | Support the proposal. |
| Nokia, NSB | We support Proposal #4-5. |
| Panasonic | We support the proposal #4-5. |
| Qualcomm | We support the proposal |
| Huawei, HiSilicon | We support the proposal |
| Fujitsu | We support the proposal |
| vivo | Support proposal #4-5. |
| CATT1 | OK if this is the majority view. However, is the group’s understanding that this case is avoided by gNB scheduling ? |
| OPPO | We support the proposal. |
| Moderator | This proposal seems stable. I will recommend it for email approval. @ CATT, Yes, my understanding is that gNB scheduler needs to avoid the collision between PUSCH (scheduled by multi-PUSCH scheduling DCI) and CORESET#0, for semi-static flexible symbols. |
| Xiaomi | We can accept the Proposal#4-5. |
| Moderator | Proposal#4-5 is agreed by email, so this issue can be closed. |

Issue#4-10: Clarification on type-2 CB generation when both of spatial bundling and time domain bundling are configured

| Company | Views |
| --- | --- |
| [6] vivo | In [1], time domain bundling operation for Type-2 codebook is divided further into two cases, i.e. the case when harq-ACK-SpatialBundlingPUCCH is not provided and the case when harq-ACK-SpatialBundlingPUCCH is provided. For the latter case, the corresponding description is referred as below. If a UE is provided numberOfHARQ-BundlingGroups and harq-ACK-SpatialBundlingPUCCH for a serving cell c, the UE generates HARQ-ACK information over PDSCH reception groups for PDSCH receptions scheduled by a DCI format on the serving cell c where a maximum number of PDSCH reception groups, NHARQ-ACK,cTBG,max, is provided by numberOfHARQ-BundlingGroups. If the UE detects a DCI format scheduling NPDSCH,c PDSCH receptions on the serving cell c, the UE generates NHARQ-ACK,cTBG,max HARQ-ACK information bits for the NPDSCH,c PDSCH receptions as described in clause 9.1.1 by setting NHARQ-ACKCBG/TB,max=NHARQ-ACK,cTBG,max and C=NPDSCH,c. For a PDSCH reception group associated with at least one PDSCH that does not overlap with an UL symbol indicated by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated if provided, the UE assumes that TBs provided by a PDSCH that overlaps with an UL symbol indicated by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated if provided, are correctly received. For a PDSCH reception group associated only with PDSCHs that overlap with UL symbols indicated by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated if provided, the UE generates a NACK value for the PDSCH reception group.. Based on the above description, the UE refers to the description in clause 9.1.1 by setting NHARQ-ACKCBG/TB,max=NHARQ-ACK,cTBG,max and C=NPDSCH,c to generate NHARQ-ACK,cTBG,max HARQ-ACK information bits for the DL scheduling DCI. However, in clause 9.1.1 there is no description about spatial bundling, e.g. binary AND operation of the HARQ-ACK information bits corresponding to first and second transport blocks of all PDSCH receptions in a PDSCH reception group. Therefore, there may be ambiguity on how to perform spatial bundling when time domain bundling is configured for Type-2 codebook. Proposal 7: For time domain bundling of Type-2 codebook, clarify the corresponding UE behaviour to generate NHARQ-ACK,cTBG,max HARQ-ACK information bits for a DL scheduling DCI when spatial bundling is also configured. |

- [Moderator’s note] Companies are encouraged to comment for above Proposal 7 (e.g., spatial domain bundling first then time domain bundling second). In addition, please provide a TP if needed.

| Company | Views |
| --- | --- |
| Samsung | Not necessary. It is already captured in type-2 CB pseudo-code in 213. Since C=NPDSCH,c (the number of codeblocks is equal to the scheduled PDSCHs), each HARQ-ACK bit is considered as HARQ-ACK bit after binary AND operation as in the pseudo-code. elseif harq-ACK-SpatialBundlingPUCCH is provided to the UE and m is a monitoring occasion for PDCCH with a DCI format that supports PDSCH reception with two transport blocks and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks in at least one configured DL BWP of a serving cell, oTD⋅j+VC-DAI,c,mDL-1ACK = binary AND operation of the HARQ-ACK information bits corresponding to the first and second transport blocks of this cell Vs=Vs∪TD⋅j+VC-DAI,c,mDL-1 else |
| Intel | We share the observation from vivo that spatial bundling is not considered in clause 9.1.1, therefore, one fix should be to add behavior on spatial bundling outside then refer to clause 9.1.1 for HARQ-ACK bits generation of NHARQ-ACK,cTBG,max TBGs. |
| Apple | Support Intel’s proposed solution. |
| Nokia, NSB | We see that there might be ambiguity and specification would benefit if the bundling operation is clarified. |
| Qualcomm | We agree with Nokia |
| Huawei, HiSilicon | We share similar view as intel. |
| vivo | Support Intel’s proposal, i.e., to add behavior on spatial bundling outside then refer to clause 9.1.1 for HARQ-ACK bits generation of NHARQ-ACK,cTBG,max TBGs. |
| CATT1 | We agree with Nokia, Qualcom |
| CATT1 | We agree with Nokia, Qualcom |
| Moderator | Majority companies seems to agree with vivo’s observation that spatial bundling operation is missing in the corresponding paragraph for type-2 HARQ CB. So, I prepared for the following TP which can reflect Intel’s proposal. Please share your views. |

- Proposal #4-10 (Type-2 CB and spatial bundling):

Adopt the following text proposal to TS38.213 v17.1.0 Clause 9.1.3.1.

- Reason for change

  - How to perform spatial domain bundling for type-2 HARQ-ACK codebook is unclear when UE is configured with both of spatial domain bundling and time domain bundling.

- Summary of change

  - Clarify that UE first performs spatial domain bundling and then time domain bundling.

- Consequences if not approved

  - Unclear UE behavior for how to perform spatial domain bundling for type-2 HARQ-ACK codebook when UE is configured with both of spatial domain bundling and time domain bundling.

--------------------------------------------Start of text proposal to TS38.213 v17.1.0 -------------------------------------------

9.1.3.1 Type-2 HARQ-ACK codebook in physical uplink control channel

< Unchanged parts are omitted >

If a UE is provided numberOfHARQ-BundlingGroups and harq-ACK-SpatialBundlingPUCCH for a serving cell c, the UE generates HARQ-ACK information over PDSCH reception groups for PDSCH receptions scheduled by a DCI format on the serving cell c where a maximum number of PDSCH reception groups, NHARQ-ACK,cTBG,max, is provided by numberOfHARQ-BundlingGroups. If the UE detects a DCI format scheduling NPDSCH,c PDSCH receptions on the serving cell c, the UE generates NHARQ-ACK,cTBG,max HARQ-ACK information bits for the NPDSCH,c PDSCH receptions as described in clause 9.1.1 by setting NHARQ-ACKCBG/TB,max=NHARQ-ACK,cTBG,max and C=NPDSCH,c  after binary AND operation of the HARQ-ACK information bits corresponding to the first and second transport blocks of each PDSCH reception, if applicable. For a PDSCH reception group associated with at least one PDSCH that does not overlap with an UL symbol indicated by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated if provided, the UE assumes that TBs provided by a PDSCH that overlaps with an UL symbol indicated by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated if provided, are correctly received. For a PDSCH reception group associated only with PDSCHs that overlap with UL symbols indicated by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated if provided, the UE generates a NACK value for the PDSCH reception group.

--------------------------------------------End of Text proposal to TS38.213 v17.1.0 --------------------------------------------

Companies are encouraged to provide views on Proposal #4-10.

| Company | Views |
| --- | --- |
| Samsung | No support. As we commented, the pseudo code for type-2 CB already captures binary AND operation. Our understanding is that a UE generates type-2 CB based on the pseudo code. If spatial bundling is configured, then, as specified, the UE performs binary AND operation per each PDSCH. Based on the text copied above, the UE can perform time domain bundling with the spatial bundled HARQ-ACK bits. |
| Qualcomm | We are okay with the proposal |
| vivo | We are fine with the proposed TP. @Samsung: The pseudo code you mentioned is for legacy type-2 CB generation. We don’t see any connection between it and type-2 CB for multi-PDSCH scheduling. That’s why the ambiguity exists. |
| Samsung | @vivo, the legacy pseudo-code for type-2 CB generation is also applied to type-2 CB for multi-PDSCH scheduling. On top of the legacy pseudo-code, the new time domain bundling is applied. It would be better to share our understanding on this issue. - If multi-PDSCH scheduling is configured, one HARQ-ACK information bit per transport block is replaced by multi-bits for multi-PDSCHs. It is from the following text …, the UE determines the o0ACK, o1ACK,⋯,oOACK-1ACK according to the previous pseudo-code with the following modifications … - instead of generating one HARQ-ACK information bit per transport block for serving cell c, the UE generates NHARQ-ACKmax HARQ-ACK information bits where … - According to the pseudo code, binary AND operation is applied for two TBs as specified in the legacy pseudo-code. oTD⋅j+VC-DAI,c,mDL-1ACK = binary AND operation of the HARQ-ACK information bits corresponding to the first and second transport blocks of this cell Please let me know if I miss something. |
| Xiaomi | Ok with Proposal. |
| Moderator | @ Samsung, I’m just following Samsung’s logic. But, the first part you referred to is for the case when time domain bundling is not configured… isn’t it? |
| Samsung | @Moderator. Sorry, the correct texts for the first part is below. If a UE - is provided PDSCH-TimeDomainResourceAllocationListForMultiPDSCH and, if provided, numberOfHARQ-BundlingGroups with value NHARQ-ACKTBG,max>1 for NcellsDL,TBG serving cells; and - is not provided PDSCH-TimeDomainResourceAllocationListForMultiPDSCH or is provided numberOfHARQ-BundlingGroups with value NHARQ-ACKTBG,max=1, for NcellsDL,TB serving cells where NcellsDL,TB+NcellsDL,TBG=NcellsDL the UE determines the o0ACK, o1ACK,⋯,oOACK-1ACK according to the previous pseudo-code with the following modifications - […] - instead of generating one HARQ-ACK information bit per transport block for a serving cell from the NcellsDL,TBG serving cells, the UE generates NHARQ-ACK,maxTBG,max HARQ-ACK information bits, … |
| Huawei, HiSilicon | By comparing the text under CBG based feedback, there is a sentence “the pseudo-code operation when harq-ACK-SpatialBundlingPUCCH is provided is not applicable”. Does it imply that pseudo-code quoted by Samsung should be applied if it is not explicitly disabled? |
| Nokia, NSB | We tend to interpret the spec according to Samsung’s logic where the legacy pseudo-code is used for the generation of the 2nd sub-codebook with “instead of generating one HARQ-ACK information bit per transport block for a serving cell from the NcellsDL,TBG serving cells, the UE generates NHARQ-ACK,maxTBG,max HARQ-ACK information bits” In the paragraph “If a UE is provided numberOfHARQ-BundlingGroups and harq-ACK-SpatialBundlingPUCCH for a serving cell c, the UE generates HARQ-ACK information over PDSCH reception groups for PDSCH receptions scheduled by a DCI format on the serving cell c where a maximum number of PDSCH reception groups, NHARQ-ACK,cTBG,max, is provided by numberOfHARQ-BundlingGroups. If the UE detects a DCI format scheduling NPDSCH,c PDSCH receptions on the serving cell c, the UE generates NHARQ-ACK,cTBG,max HARQ-ACK information bits for the NPDSCH,c PDSCH receptions as described in clause 9.1.1 by setting NHARQ-ACKCBG/TB,max=NHARQ-ACK,cTBG,max and C=NPDSCH,c.” we find the highlighted text confusing: For the “UE generates NHARQ-ACK,cTBG,max HARQ-ACK information bits for the NPDSCH,c PDSCH receptions” only clause 9.1.1 is referenced, not the pseudo-code where spatial bundling is performed. As this is separate paragraph for condition “If a UE is provided numberOfHARQ-BundlingGroups and harq-ACK-SpatialBundlingPUCCH”, reader expects that the generated HARQ-ACK information bits are really for the PDSCH receptions, not for the first or second TBs of the PDSCHs, as that is discussed in the previous section. So in our view the “UE generates NHARQ-ACK,cTBG,max HARQ-ACK information bits for the NPDSCH,c PDSCH receptions” should refer both to clause 9.1.1 and spatial bundling carried out within the pseudo-code. |
| Moderator | @ Huawei, The quoted sentence implies that a UE configured with CBG will not expect to be configured with spatial domain bundling at the same time, which seems to be a separate issue. @ Samsung and Nokia, According to my spec reading, as Samsung commented, the legacy pseudo-code is used. However, before going into specification details, I would like to check if we are on the same page. Let me take an example. Assuming numberOfHARQ-BundlingGroups is configured as 4, 8 PDSCHs are scheduled, and two transport blocks for each PDSCH are enabled. - Case 1: If a UE is not provided harq-ACK-SpatialBundlingPUCCH - The UE will generate 4 bits for the first TB and 4 bits for the second TB, thus in total 8 bits. - Case 2: If a UE is provided harq-ACK-SpatialBundlingPUCCH - The UE will generate 4 bits in total. Is the above is the common understanding? Then, for the Case 2, I don’t think current specification is clear in terms of how UE generates 4 bits in total. If a UE is provided numberOfHARQ-BundlingGroups and harq-ACK-SpatialBundlingPUCCH for a serving cell c, the UE generates HARQ-ACK information over PDSCH reception groups for PDSCH receptions scheduled by a DCI format on the serving cell c where a maximum number of PDSCH reception groups, NHARQ-ACK,cTBG,max, is provided by numberOfHARQ-BundlingGroups. If the UE detects a DCI format scheduling NPDSCH,c PDSCH receptions on the serving cell c, the UE generates NHARQ-ACK,cTBG,max HARQ-ACK information bits for the NPDSCH,c PDSCH receptions as described in clause 9.1.1 by setting NHARQ-ACKCBG/TB,max=NHARQ-ACK,cTBG,max and C=NPDSCH,c. For a PDSCH reception group associated with at least one PDSCH that does not overlap with an UL symbol indicated by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated if provided, the UE assumes that TBs provided by a PDSCH that overlaps with an UL symbol indicated by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated if provided, are correctly received. For a PDSCH reception group associated only with PDSCHs that overlap with UL symbols indicated by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated if provided, the UE generates a NACK value for the PDSCH reception group.. Could Samsung, Nokia or anybody clarify? |
| Huawei, HiSilicon | I think the ambiguity comes from the relationship between the paragraph of “If a UE is provided numberOfHARQ-BundlingGroups and harq-ACK-SpatialBundlingPUCCH for a serving cell c, …” and the sentence of “the UE determines the o0ACK, o1ACK,⋯,oOACK-1ACK according to the previous pseudo-code with the following modifications…”. If the paragraph could be regarded as one of modifications to in the pseudo-code, similar as the text of “instead of generating one HARQ-ACK information bit per transport block for a serving cell from the NcellsDL,TBG serving cells,…”, then the original pseudo-code for spatial domain bundling can be replaced with the paragraph and clarification of AND operation should be added. However, the current writing method make it looks not like an modification of the pseudo-code but an additional handling, especially if the “condition 1” as discussed in issue 4-11 is deleted. |
| Intel | We share the view from moderator. |
| ZTE, Sanechips | We have similar understanding with Samsung. According to the TS text copy by Samsung, only half bits of HARQ-ACK information with harq-ACK-SpatialBundlingPUCCH generates vs without harq-ACK-SpatialBundlingPUCCH when maxNrofCodeWordsScheduledByDCI configured with 2. That is, the current spec implies spatial domain bundling first then time domain bundling second. If majority companies tend to modify the current spec, we are also fine since it can make spec more clearer. But from our point of view, we don’t think that there’s any problem without change spec. |
| Samsung | @Moderator. Regarding your first question, yes, 4 bits are generated when spatial bundling is configured. The current spec says “… the UE generates NHARQ-ACK,cTBG,max HARQ-ACK information bits for the NPDSCH,c PDSCH receptions…” Regarding the question on how to generate 4 bits in total, we are not clear what is the problem. HARQ-ACK bits for multi-PDSCHs are bundled by the CBG grouping rule in 9.1.1. Probably, the concern on the paragraph “If a UE is provided numberOfHARQ-BundlingGroups and harq-ACK-SpatialBundlingPUCCH for a serving cell c, the UE generates HARQ-ACK information over PDSCH reception groups for PDSCH receptions scheduled by a DCI format on the serving cell c,…” is there is no explicit description of spatial bundling operation in the paragraph. But, comparing with the previous paragraph where “If a UE is provided numberOfHARQ-BundlingGroups and is not provided harq-ACK-SpatialBundlingPUCCH for a serving cell c, the UE generates HARQ-ACK information over transport block groups (TBGs) for PDSCH receptions” states HARQ-ACK information over transport block groups (i.e., the HARQ-ACK bits for first TBs are bundled and the HARQ-ACK bits for second TBs are bundled separately), the paragraph says HARQ-ACK information over PDSCH reception groups which means already HARQ-ACK information of two TBs in a PDSCH are bundled. |
| Moderator | @ All, With my further thought, the current specification seems not to have a problem. As Samsung pointed out, the spec says like below and correspond to 4 bits in total for Case 2 in my example. “… the UE generates NHARQ-ACK,cTBG,max HARQ-ACK information bits for the NPDSCH,c PDSCH receptions…” Then, the generated 4 bits corresponding to a DCI will be carried over the pseudo-code, based on the following yellow-highlighted texts. “ If a UE - is provided PDSCH-TimeDomainResourceAllocationListForMultiPDSCH and, if provided, numberOfHARQ-BundlingGroups with value NHARQ-ACKTBG,max>1 for NcellsDL,TBG serving cells; and - is not provided PDSCH-TimeDomainResourceAllocationListForMultiPDSCH or is provided numberOfHARQ-BundlingGroups with value NHARQ-ACKTBG,max=1, for NcellsDL,TB serving cells where NcellsDL,TB+NcellsDL,TBG=NcellsDL the UE determines the o0ACK, o1ACK,⋯,oOACK-1ACK according to the previous pseudo-code with the following modifications - NcellsDL is used for the determination of a first HARQ-ACK sub-codebook for - SPS PDSCH reception, - any DCI format having associated HARQ-ACK information without scheduling PDSCH reception, and - PDSCH reception scheduled by a DCI format scheduling one PDSCH - PDSCH reception with NHARQ-ACKTBG,max=1 for TBG-based HARQ-ACK information on the NcellsDL,TB serving cells, - NcellsDL is replaced by NcellsDL,TBG for the determination of a second HARQ-ACK sub-codebook corresponding to the NcellsDL,TBG serving cells for TBG-based HARQ-ACK information, or for TB-based HARQ-ACK information corresponding to multiple PDSCH receptions scheduled by a single DCI format, and - if, for an active DL BWP of a serving cell, the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for one or more first CORESETs and is provided coresetPoolIndex with value 1 for one or more second CORESETs, and is provided ackNackFeedbackMode = joint, the serving cell is counted as two times where the first time corresponds to the first CORESETs and the second time corresponds to the second CORESETs, and - instead of generating one HARQ-ACK information bit per transport block for a serving cell from the NcellsDL,TBG serving cells, the UE generates NHARQ-ACK,maxTBG,max HARQ-ACK information bits, where NHARQ-ACK,maxTBG,max is the maximum value between NTB,cDL⋅NHARQ-ACK,cTBG,max across all NcellsDL,TBG serving cells if the UE is provided numberOfHARQ-BundlingGroups, and NTB,cDL⋅NPDSCH,cmax across all NcellsDL,TBG serving cells where the UE is not provided numberOfHARQ-BundlingGroups, and NTB,cDL is the value of maxNrofCodeWordsScheduledByDCI for serving cell c if harq-ACK-SpatialBundlingPUCCH is not provided; else, NTB,cDL=1. If for a serving cell c where the UE is provided numberOfHARQ-BundlingGroups, it is NTB,cDL⋅NHARQ-ACK,cTBG,max<NHARQ-ACK,maxTBG,max, the UE generates NACK for the last NHARQ-ACK,maxTBG,max-NTB,cDL⋅NHARQ-ACK,cTBG,max HARQ-ACK information bits for serving cell c. If for a serving cell c where the UE is not provided numberOfHARQ-BundlingGroups, it is NTB,cDL⋅NPDSCH,cmax <NHARQ-ACK,maxTBG,max, the UE generates NACK for the last NHARQ-ACK,maxTBG,max-NTB,cDL⋅NPDSCH,cmax HARQ-ACK information bits for serving cell c. ” Going to the pseudo code, from my understanding, oTD⋅j+VC-DAI,c,mDL-1ACK to oTD⋅j+VC-DAI,c,mDL-1+3ACK are filled with the 4 bits for Case 2 in my example. “ elseif harq-ACK-SpatialBundlingPUCCH is provided to the UE and m is a monitoring occasion for PDCCH with a DCI format that supports PDSCH reception with two transport blocks and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks in at least one configured DL BWP of a serving cell, oTD⋅j+VC-DAI,c,mDL-1ACK = binary AND operation of the HARQ-ACK information bits corresponding to the first and second transport blocks of this cell Vs=Vs∪TD⋅j+VC-DAI,c,mDL-1 else oTD⋅j+VC-DAI,c,mDL-1ACK = HARQ-ACK information bit of this cell Vs=Vs∪TD⋅j+VC-DAI,c,mDL-1 ” The remaining point is either of spatial bundling first or time bundling first. Based on Samsung’s interpretation, current specification gives a guide for spatial bundling first. From my view, either way is OK since the result of spatial bundling first would be equivalent with that of time bundling first. Having said that, we may conclude that the current specification does not have a problem when UE generates type-2 HARQ-ACK codebook with spatial and time domaing bundling operations configured. |
| Huawei, HiSilicon | The moderator seems copy the text from CBG-based feedback instead of time domain bundling cased. in CBG based feedback, a simple replacement from HARQ-ACK information for a TB to CBG is straightforward because spatial bundling is not supported. Then for time domain bundling, if we make similar replacement from TB to TBG, the AND operation should be performed after time domain bundling. To us, the AND operation is like an element wise AND between two vectors. |
| vivo | To Moderator: Agree with HW, the referenced description from TS38.213 is for CBG transmission instead of time domain bundling. We share the same view as Nokia that “in our view the ‘UE generates NHARQ-ACK,cTBG,max HARQ-ACK information bits for the NPDSCH,c PDSCH receptions’ should refer both to clause 9.1.1 and spatial bundling carried out within the pseudo-code”. The following TP can be considered. --------------------------------------------Start of text proposal to TS38.213 v17.1.0 ------------------------------------------- 9.1.3.1 Type-2 HARQ-ACK codebook in physical uplink control channel < Unchanged parts are omitted > If a UE is provided numberOfHARQ-BundlingGroups and harq-ACK-SpatialBundlingPUCCH for a serving cell c, the UE generates HARQ-ACK information over PDSCH reception groups for PDSCH receptions scheduled by a DCI format on the serving cell c where a maximum number of PDSCH reception groups, NHARQ-ACK,cTBG,max, is provided by numberOfHARQ-BundlingGroups. If the UE detects a DCI format scheduling NPDSCH,c PDSCH receptions on the serving cell c, the UE generates NHARQ-ACK,cTBG,max HARQ-ACK information bits for the NPDSCH,c PDSCH receptions as described in clause 9.1.1 by setting NHARQ-ACKCBG/TB,max=NHARQ-ACK,cTBG,max and C=NPDSCH,c , after binary AND operation described in clause 9.2.3.1. For a PDSCH reception group associated with at least one PDSCH that does not overlap with an UL symbol indicated by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated if provided, the UE assumes that TBs provided by a PDSCH that overlaps with an UL symbol indicated by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated if provided, are correctly received. For a PDSCH reception group associated only with PDSCHs that overlap with UL symbols indicated by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated if provided, the UE generates a NACK value for the PDSCH reception group. --------------------------------------------End of Text proposal to TS38.213 v17.1.0 -------------------------------------------- |
| Moderator | @ Huawei and vivo, Sorry for the confusion. Now I fixed my previous comment by quoting the correct paragraph… From my understanding, the cyan part below is not valid for the case when UE is configured with multi-PDSCH scheduling DCI and with spatial domain bundling. This is because before going into the pseudo code, UE generates NHARQ-ACK,maxTBG,max bits (instead of instead of generating one HARQ-ACK information bit per transport block for the serving cell) where spatial bundling is already applied (that is, 4 bits for Case 2 in my example). elseif harq-ACK-SpatialBundlingPUCCH is provided to the UE and m is a monitoring occasion for PDCCH with a DCI format that supports PDSCH reception with two transport blocks and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks in at least one configured DL BWP of a serving cell, oTD⋅j+VC-DAI,c,mDL-1ACK = binary AND operation of the HARQ-ACK information bits corresponding to the first and second transport blocks of this cell Vs=Vs∪TD⋅j+VC-DAI,c,mDL-1 else oTD⋅j+VC-DAI,c,mDL-1ACK = HARQ-ACK information bit of this cell Vs=Vs∪TD⋅j+VC-DAI,c,mDL-1 end if This is similar with CBG case where a UE generates NHARQ-ACK,maxCBG/TB,max bits even when 2-TB is enabled. “ If a UE - is provided PDSCH-CodeBlockGroupTransmission for NcellsDL,CBG serving cells; and - is not provided PDSCH-CodeBlockGroupTransmission, for NcellsDL,TB serving cells where NcellsDL,TB+NcellsDL,CBG=NcellsDL the UE determines the o0ACK, o1ACK,⋯,oOACK-1ACK according to the previous pseudo-code with the following modifications - NcellsDL is used for the determination of a first HARQ-ACK sub-codebook for - SPS PDSCH reception, - a DCI format having associated HARQ-ACK information without scheduling PDSCH reception, - TCI state update, and - TB-based PDSCH receptions on the NcellsDL,CBG serving cells and on the NcellsDL,TB serving cells, - NcellsDL is replaced by NcellsDL,CBG for the determination of a second HARQ-ACK sub-codebook corresponding to the NcellsDL,CBG serving cells for CBG-based PDSCH receptions, and - if, for an active DL BWP of a serving cell, the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for one or more first CORESETs and is provided coresetPoolIndex with value 1 for one or more second CORESETs, and is provided ackNackFeedbackMode = joint, the serving cell is counted as two times where the first time corresponds to the first CORESETs and the second time corresponds to the second CORESETs, and - instead of generating one HARQ-ACK information bit per transport block for a serving cell from the NcellsDL,CBG serving cells, the UE generates NHARQ-ACK,maxCBG/TB,max HARQ-ACK information bits, where NHARQ-ACK,maxCBG/TB,max is the maximum value of NTB,cDL⋅NHARQ-ACK,cCBG/TB,max across all NcellsDL,CBG serving cells and NTB,cDL is the value of maxNrofCodeWordsScheduledByDCI for serving cell c. If for a serving cell c it is NTB,cDL⋅NHARQ-ACK,cCBG/TB,max<NHARQ-ACK,maxCBG/TB,max, the UE generates NACK for the last NHARQ-ACK,maxCBG/TB,max-NTB,cDL⋅NHARQ-ACK,cCBG/TB,max HARQ-ACK information bits for serving cell c ” Then, in the pseudo code, instead of cyan part below, just oTD⋅j+VC-DAI,c,mDL-1ACK to oTD⋅j+VC-DAI,c,mDL-1+NHARQ-ACK,maxCBG/TB,max-1ACK are generated. if harq-ACK-SpatialBundlingPUCCH is not provided and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for at least one configured DL BWP of at least one serving cell, o2⋅TD⋅j+2VC-DAI,c,mDL-1ACK = HARQ-ACK information bit corresponding to the first transport block of this cell o2⋅TD⋅j+2VC-DAI,c,mDL-1+1ACK = HARQ-ACK information bit corresponding to the second transport block of this cell @ Huawei, Could you elaborate your statement “the AND operation is like an element wise AND between two vectors” To be specific, I wonder what two vectors imply. @ vivo, I guess you wanted to refer to pseudo code in Clause 9.1.3.1 (rather than 9.2.3.1). However, I’m not sure whether after applying pseudo code then UE applies 9.1.1 to generate NHARQ-ACKCBG/TB,max bits, or after generating NHARQ-ACKCBG/TB,max bits as in 9.1.1 then UE applies pseudo code... I thought the latter interpretation is correct… |

Issue#4-11: Introduce an independent nHARQ-ACK formula for type-2 HARQ CB when time domain is not configured but multi-PDSCH scheduling DCI is configured

| Company | Views |
| --- | --- |
| [6] vivo | ---------------------------------------------------Start TP2------------------------------------------------------ Reason for change: The defined calculation formula nHARQ-ACK=nHARQ-ACK,TB+nHARQ-ACK,TBG is not applicable for the case when the UE is not provided numberOfHARQ-BundlingGroups and two sub-codebooks are required. Summary of change: Define a calculation formula nHARQ-ACK=nHARQ-ACK,TB+nHARQ-ACK,multi for the case when the UE is not provided numberOfHARQ-BundlingGroups and two sub-codebooks are required. Consequences if not approved: There is no calculation formula of nHARQ-ACK applicable for the case when the UE is not provided numberOfHARQ-BundlingGroups and two sub-codebooks are required. 9.1.3.1 Type-2 HARQ-ACK codebook in physical uplink control channel … If a UE is not provided numberOfHARQ-BundlingGroups, detects a first DCI format scheduling one PDSCH reception or having associated HARQ-ACK information without scheduling a PDSCH reception, if any, and a second DCI format scheduling more than one PDSCH reception on a serving cell from the NcellsDL serving cells, if any, and the UE would provide corresponding HARQ-ACK information in a same PUCCH, the UE determines the o0ACK, o1ACK,⋯,oOACK-1ACK according to the previous pseudo-code with the following modifications - the UE determines a first HARQ-ACK sub-codebook based on each detected DCI format scheduling one PDSCH reception or having associated HARQ-ACK information without scheduling a PDSCH reception, or SPS PDSCH receptions, if any, and - the UE determines a second HARQ-ACK sub-codebook based on each detected DCI format scheduling more than one PDSCH reception, and - instead of generating one HARQ-ACK information bit per transport block for serving cell c, the UE generates NHARQ-ACKmax HARQ-ACK information bits where NHARQ-ACKmax is the maximum value of NTB,cDL⋅NPDSCH,cmax across all NcellsDL serving cells, NPDSCH,cmax is a maximum number of PDSCH receptions that can be scheduled by a DCI format on serving cell c as described in [6, TS 38.214], and NTB,cDL is the value of maxNrofCodeWordsScheduledByDCI for serving cell c if harq-ACK-SpatialBundlingPUCCH is not provided; else, NTB,cDL=1. The UE generates the NHARQ-ACKmax HARQ-ACK information bits in ascending order of the NPDSCH,cmax PDSCHs, including any PDSCH that overlaps with UL symbols indicated by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated if provided. If, for serving cell c, the UE detects a DCI format that schedules NPDSCH,c PDSCH receptions and NTB,cDL⋅NPDSCH,c<NHARQ-ACKmax , the UE generates NACK for the last NHARQ-ACKmax -NTB,cDL⋅NPDSCH,c HARQ-ACK information bits - The pseudo-code operation when PDSCH-CodeBlockGroupTransmission is provided is not applicable. - The counter DAI value and the total DAI value apply separately for each HARQ-ACK sub-codebook. - The UE generates the HARQ-ACK codebook by appending the second HARQ-ACK sub-codebook to the first HARQ-ACK sub-codebook. If the UE is not provided numberOfHARQ-BundlingGroups and OACK+OSR+OCSI≤11 and NcellsDL,TBG>0, the UE also determines nHARQ-ACK=nHARQ-ACK,TB+nHARQ-ACK,TBGmulti for obtaining a PUCCH transmission power, as described in clause 7.2.1, with nHARQ-ACK,TBGmulti=VDAI,mlastDL-c=0NcellsDL,TBGmulti-1UDAI,cmultiTBGmodTDNHARQ-ACK,maxTBG,max+c=0NcellsDL,multiTBG-1m=0M-1Nm,creceived,multiTBG where - NcellsDL,multi is the number of serving cells each of which is provided PDSCH-TimeDomainResourceAllocationListForMultiPDSCH in NcellsDL serving cells - if NcellsDL=1, VDAI,mlastDL is the value of the counter DAI in the last DCI format scheduling more than one PDSCH receptions for any serving cell c from the NcellsDL,multiTBG serving cells with TBG-based HARQ-ACK information or with TB-based HARQ-ACK information that the UE detects within the M PDCCH monitoring occasions - if NcellsDL>1, VDAI,mlastDL is the value of the total DAI in the last DCI format scheduling more than one PDSCH receptions with TBG-based HARQ-ACK information or with TB-based HARQ-ACK information for any serving cell c from the NcellsDL,multiTBG serving cells that the UE detects within the M PDCCH monitoring occasions - VDAI,mlastDL=0, if the UE does not detect any DCI format scheduling more than one PDSCH receptions with TBG-based HARQ-ACK information or with TB-based HARQ-ACK information for any serving cell c from the NcellsDL,multiTBG serving cells in any of the M PDCCH monitoring occasions - UDAI,cmultiTBG is the total number of DCI formats scheduling more than one PDSCH receptions with TBG-based HARQ-ACK information or with TB-based HARQ-ACK information for any serving cell c from the NcellsDL,multiTBG serving cells that the UE detects within the M PDCCH monitoring occasions for serving cell c. UDAI,cTBG=0 if the UE does not detect any DCI format scheduling more than one PDSCH receptions for serving cell c in any of the M PDCCH monitoring occasions - if harq-ACK-SpatialBundlingPUCCH is provided, - if numberOfHARQ-BundlingGroups is provided, Nm,creceived,TBG is the number of PDSCH groups that include at least one PDSCH not overlapping with a UL symbol indicated by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated if provided, that the UE receives in serving cell c from the NcellsDL,TBG serving cells in PDCCH monitoring occasion m and the UE reports corresponding HARQ-ACK information in the PUCCH - if numberOfHARQ-BundlingGroups is not provided, Nm,creceived,multiTBG is the number of PDSCHs that the UE receives in serving cell c from the NcellsDL,multiTBG serving cells in PDCCH monitoring occasion m and the UE reports corresponding HARQ-ACK information in the PUCCH - if harq-ACK-SpatialBundlingPUCCH is not provided, - if numberOfHARQ-BundlingGroups is provided, Nm,creceived,TBG is the number of TBGs including at least one PDSCH not overlapping with an UL symbol indicated by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated if provided, that the UE receives in serving cell c from the NcellsDL,TBG serving cells in PDCCH monitoring occasion m and the UE reports corresponding HARQ-ACK information in the PUCCH - if numberOfHARQ-BundlingGroups is not provided, Nm,creceived,multiTBG is the number of transport blocks in PDSCHs that the UE receives in serving cell c from the NcellsDL,multiTBG serving cells in PDCCH monitoring occasion m and the UE reports corresponding HARQ-ACK information in the PUCCH. If a UE is provided numberOfHARQ-BundlingGroups and is not provided harq-ACK-SpatialBundlingPUCCH for a serving cell c, the UE generates HARQ-ACK information over transport block groups (TBGs) for PDSCH receptions where, for a maximum number of NPDSCHmax PDSCH receptions scheduled by a DCI format on the serving cell, a maximum number of TBGs NHARQ-ACK,cTBG,max is provided by numberOfHARQ-BundlingGroups. If the UE detects a DCI format scheduling NPDSCH,c PDSCH receptions on the serving cell c, the UE generates NHARQ-ACK,cTBG,max HARQ-ACK information bits for first TBs and, if applicable, generates NHARQ-ACK,cTBG,max HARQ-ACK information bits for second TBs in the NPDSCH,c PDSCH receptions as described in clause 9.1.1 by setting NHARQ-ACKCBG/TB,max=NHARQ-ACK,cTBG,max and C=NPDSCH,c. For a TBG associated with at least one PDSCH that does not overlap with an UL symbol indicated by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated if provided, the UE assumes that TB(s) provided by a PDSCH that overlaps with an UL symbol indicated by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated if provided, are correctly received. For a TBG associated only with PDSCHs that overlap with UL symbols indicated by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated if provided, the UE generates a NACK value for the TBG. … --------------------------------------------------End TP2------------------------------------------------------- Proposal 8: For Type-2 codebook of multi-PDSCH scheduling, introduce an independent description for calculation formula of nHARQ-ACK defined for the case when the UE is not provided numberOfHARQ-BundlingGroups and two sub-codebooks are required, where the corresponding variables are used; the TP2 can be considered in TS38.213 as a starting point. |

- [Moderator’s note] Companies are encouraged to comment for above Proposal 8 and TP2.

| Company | Views |
| --- | --- |
| Samsung | Not support. The nHARQ-ACK,TBG already includes the case where the UE is not provided numberOfHARQ-BundlingGroups since the NcellsDL,TBG includes a cell where PDSCH-TimeDomainResourceAllocationListForMultiPDSCH is provided but numberOfHARQ-BundlingGroups is not provided (see below text and “if provided” is for this) If a UE - is provided PDSCH-TimeDomainResourceAllocationListForMultiPDSCH and, if provided, numberOfHARQ-BundlingGroups with value NHARQ-ACKTBG,max>1 for NcellsDL,TBG serving cells; and - is not provided PDSCH-TimeDomainResourceAllocationListForMultiPDSCH or is provided numberOfHARQ-BundlingGroups with value NHARQ-ACKTBG,max=1, for NcellsDL,TB serving cells where NcellsDL,TB+NcellsDL,TBG=NcellsDL vivo’s TP is incorrect since the nHARQ-ACK,TBG also needs for the case where numberOfHARQ-BundlingGroups is configured but the TP removes this case. The ambiguity might come from the superscript “TBG”. To avoid this ambiguity, we are ok to replace the superscript “TBG” with “multi” as proposed by vivo. |
| Intel | We tend to think the revision is not necessary. Based the definition of NcellsDL,TBG, it should also cover a cell that time domain bundling is not configured. - NcellsDL is replaced by NcellsDL,TBG for the determination of a second HARQ-ACK sub-codebook corresponding to the NcellsDL,TBG serving cells for TBG-based HARQ-ACK information, or for TB-based HARQ-ACK information corresponding to multiple PDSCH receptions scheduled by a single DCI format, and We are open to hear other companies’ views |
| Nokia, NSB | We do not see this as an essential change, although there is room to improve the clarity of specification on this aspect. |
| Huawei, HiSilicon | In current spec, the following formula for nHARQ-ACK,TBGis shared by multiple PDSCH scheduling with and without time bundling. In the variable description, the definition for TBG based (with time bundling) and TB based (without time bundling) is also separate. The only ambiguous variable is NHARQ-ACK,maxTBG,max, which should be considered as NHARQ-ACKmax when time bundling is not configured. A simple change can be resolved the confusion from our perspective. “- UDAI,cTBG is the total number of DCI formats scheduling more than one PDSCH receptions with TBG-based HARQ-ACK information or with TB-based HARQ-ACK information for any serving cell c from the NcellsDL,TBG serving cells that the UE detects within the M PDCCH monitoring occasions for serving cell c. UDAI,cTBG=0 if the UE does not detect any DCI format scheduling more than one PDSCH receptions for serving cell c in any of the M PDCCH monitoring occasions - NHARQ-ACK,maxTBG,max equals to NHARQ-ACKmax if numberOfHARQ-BundlingGroups is not provided” |
| vivo | In TS 38.213 v17.1.0 separate descriptions for HARQ-ACK sub-codebook setting are introduced for the case when the UE is not provided numberOfHARQ-BundlingGroups and the case when the UE is provided numberOfHARQ-BundlingGroups, respectively. However, in TS 38.213 v17.1.0 only one nHARQ-ACK calculation formula is introduced, which is intended to accommodate the above two cases at the same time. When the nHARQ-ACK calculation formula is used for the case that the UE is not provided numberOfHARQ-BundlingGroups, there may be some ambiguity. For example, the variable NcellsDL,TBG is only defined in the description for the case that the UE is provided numberOfHARQ-BundlingGroups. Besides, the variable NHARQ-ACK,maxTBG,max is defined dedicatedly for the case that the UE is provided numberOfHARQ-BundlingGroups, other than NHARQ-ACKmax which is defined for the case that the UE is not provided numberOfHARQ-BundlingGroups. We disagree with “vivo’s TP is incorrect since the nHARQ-ACK,TBG also needs for the case where numberOfHARQ-BundlingGroups is configured but the TP removes this case.” Commented by Samsung. The TP is to provide consistent description between HARQ-ACK sub-codebook settings and nHARQ-ACK calculation formulas, for the above two cases to avoid potential ambiguity. The description for nHARQ-ACK,TBG in TS 38.213 v17.1.0 is kept as it is for the case that the UE is provided numberOfHARQ-BundlingGroups, and an additional description is introduced dedicatedly for the case that the UE is not provided numberOfHARQ-BundlingGroups, based on our TP. |
| CATT1 | We are fine to clarify this. |
| Moderator1 | I couldn’t see if there is a consensus on the necessity of this TP. Meanwhile, I found 3 issues are discussing here. - Subscript of TBG: It seems to be a trivial issue whether the subscript should be “TBG” or “multi”. - Definition of NcellsDL,TBG: According to the highlighted part below, I think NcellsDL,TBG includes the cell configured with time domain bundling or without time domain bundling. If a UE - is provided PDSCH-TimeDomainResourceAllocationListForMultiPDSCH and, if provided, numberOfHARQ-BundlingGroups with value NHARQ-ACKTBG,max>1 for NcellsDL,TBG serving cells; and - is not provided PDSCH-TimeDomainResourceAllocationListForMultiPDSCH or is provided numberOfHARQ-BundlingGroups with value NHARQ-ACKTBG,max=1, for NcellsDL,TB serving cells where NcellsDL,TB+NcellsDL,TBG=NcellsDL. - Definition of NHARQ-ACK,maxTBG,max: According to the highlighted part below, it seems that NHARQ-ACK,maxTBG,max is also applicable when every cell in the PUCCH group is not configured with time domain bundling. - if, for an active DL BWP of a serving cell, the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for one or more first CORESETs and is provided coresetPoolIndex with value 1 for one or more second CORESETs, and is provided ackNackFeedbackMode = joint, the serving cell is counted as two times where the first time corresponds to the first CORESETs and the second time corresponds to the second CORESETs, and - instead of generating one HARQ-ACK information bit per transport block for a serving cell from the NcellsDL,TBG serving cells, the UE generates NHARQ-ACK,maxTBG,max HARQ-ACK information bits, where NHARQ-ACK,maxTBG,max is the maximum value between NTB,cDL⋅NHARQ-ACK,cTBG,max across all NcellsDL,TBG serving cells if the UE is provided numberOfHARQ-BundlingGroups, and NTB,cDL⋅NPDSCH,cmax across all NcellsDL,TBG serving cells where the UE is not provided numberOfHARQ-BundlingGroups, and NTB,cDL is the value of maxNrofCodeWordsScheduledByDCI for serving cell c if harq-ACK-SpatialBundlingPUCCH is not provided; else, NTB,cDL=1. If for a serving cell c where the UE is provided numberOfHARQ-BundlingGroups, it is NTB,cDL⋅NHARQ-ACK,cTBG,max<NHARQ-ACK,maxTBG,max, the UE generates NACK for the last NHARQ-ACK,maxTBG,max-NTB,cDL⋅NHARQ-ACK,cTBG,max HARQ-ACK information bits for serving cell c. If for a serving cell c where the UE is not provided numberOfHARQ-BundlingGroups, it is NTB,cDL⋅NPDSCH,cmax <NHARQ-ACK,maxTBG,max, the UE generates NACK for the last NHARQ-ACK,maxTBG,max-NTB,cDL⋅NPDSCH,cmax HARQ-ACK information bits for serving cell c.. If those points are correct, we may not need any further correction. But, please let me know if I’m wrong. |
| Samsung | We agree with the Moderator’s three points. Probably, the required specification update is to change “TBG” to “multi” in the superscript. |
| Huawei, HiSilicon | As for the issue 3 mentioned by moderator, the referred the text is under the condition of “If a UE - is provided PDSCH-TimeDomainResourceAllocationListForMultiPDSCH and, if provided, numberOfHARQ-BundlingGroups with value NHARQ-ACKTBG,max>1 for NcellsDL,TBG serving cells; and - is not provided PDSCH-TimeDomainResourceAllocationListForMultiPDSCH or is provided numberOfHARQ-BundlingGroups with value NHARQ-ACKTBG,max=1, for NcellsDL,TB serving cells where NcellsDL,TB+NcellsDL,TBG=NcellsDL” However, the scenario of “If a UE is not provided numberOfHARQ-BundlingGroups, detects a first DCI format scheduling one PDSCH reception or having associated HARQ-ACK information without scheduling a PDSCH reception, if any, and a second DCI format scheduling more than one PDSCH reception on a serving cell from the NcellsDL serving cells, …” appeared in the early paragraphs is not covered. In the scenario, the HARQ-ACK bit sis generate according to following text “- instead of generating one HARQ-ACK information bit per transport block for serving cell c, the UE generates NHARQ-ACKmax HARQ-ACK information bits where NHARQ-ACKmax is the maximum value of NTB,cDL⋅NPDSCH,cmax across all NcellsDL serving cells, NPDSCH,cmax is a maximum number of PDSCH receptions that can be scheduled by a DCI format on serving cell c as described in [6, TS 38.214], and NTB,cDL is the value of maxNrofCodeWordsScheduledByDCI for serving cell c if harq-ACK-SpatialBundlingPUCCH is not provided; else, NTB,cDL=1. The UE generates the NHARQ-ACKmax HARQ-ACK information bits in ascending order of the NPDSCH,cmax PDSCHs, including any PDSCH that overlaps with UL symbols indicated by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated if provided. If, for serving cell c, the UE detects a DCI format that schedules NPDSCH,c PDSCH receptions and NTB,cDL⋅NPDSCH,c<NHARQ-ACKmax , the UE generates NACK for the last NHARQ-ACKmax -NTB,cDL⋅NPDSCH,c HARQ-ACK information bits” If the nHARQ-ACK,TBG calculation formula is going to be used for this scenario, the link between NHARQ-ACKmax and NHARQ-ACK,maxTBG,max should be set up. |
| Moderator2 | @ Huawei, I disagree that the following paragraph doesn’t cover the case where a UE is not provided numberOfHARQ-BundlingGroups. “If a UE - is provided PDSCH-TimeDomainResourceAllocationListForMultiPDSCH and, if provided, numberOfHARQ-BundlingGroups with value NHARQ-ACKTBG,max>1 for NcellsDL,TBG serving cells; and - is not provided PDSCH-TimeDomainResourceAllocationListForMultiPDSCH or is provided numberOfHARQ-BundlingGroups with value NHARQ-ACKTBG,max=1, for NcellsDL,TB serving cells where NcellsDL,TB+NcellsDL,TBG=NcellsDL” According to the above highlighted “if provided”, numberOfHARQ-BundlingGroups may not be configured. I understand that NHARQ-ACKmax is being used only for a UE not configured with numberOfHARQ-BundlingGroups. However, for calculating nHARQ-ACK,TBG, NHARQ-ACK,maxTBG,max is derived by the paragraph I cited, and is also applicable to a UE not configured with numberOfHARQ-BundlingGroups. Therefore, I still don’t see any problem in the current specification. |
| Huawei, HiSilicon | @Moderator I have different understanding on the text. The HARQ codebook generation for multi PDSCH scheduling are structured under following 4 conditions. And these 4 conditions are mutual exclusive. Condition 1: If a UE is not provided numberOfHARQ-BundlingGroups, detects a first DCI format scheduling one PDSCH reception or having associated HARQ-ACK information without scheduling a PDSCH reception, if any, and a second DCI format scheduling more than one PDSCH reception on a serving cell from the NcellsDL serving cells - Condition 2: - If a UE is provided numberOfHARQ-BundlingGroups and is not provided harq-ACK-SpatialBundlingPUCCH for a serving cell c, - Condition 3: - If a UE is provided numberOfHARQ-BundlingGroups and harq-ACK-SpatialBundlingPUCCH for a serving cell c, - Condition 4: - is provided PDSCH-TimeDomainResourceAllocationListForMultiPDSCH and, if provided, numberOfHARQ-BundlingGroups with value NHARQ-ACKTBG,max>1 for NcellsDL,TBG serving cells; and - is not provided PDSCH-TimeDomainResourceAllocationListForMultiPDSCH or is provided numberOfHARQ-BundlingGroups with value NHARQ-ACKTBG,max=1, for NcellsDL,TB serving cells where NcellsDL,TB+NcellsDL,TBG=NcellsDL - The NHARQ-ACK,maxTBG,max is defined under condition 2,3,4 but not 1. - Finally the nHARQ-ACK,TBG will be applied for all 4 conditions. - The text moderator quoted is under condition 4 where multiple PDSCH scheduling and single PDSCH scheduling are on different CCs while the condition 1 is on the same CC. |
| Moderator3 | @ Huawei, Seeing Huawei’s comments, now I’m thinking of why Condition 1 is needed… From my understanding, Condition 4 is the superset of Conditions 1/2/3. I don’t understand your last statement that for Condition 4, multiple PDSCH scheduling and single PDSCH scheduling are on different CCs. Based on highlighted part below, UE can receive single PDSCH scheduling DCI from any of serving cells. If a UE - is provided PDSCH-TimeDomainResourceAllocationListForMultiPDSCH and, if provided, numberOfHARQ-BundlingGroups with value NHARQ-ACKTBG,max>1 for NcellsDL,TBG serving cells; and - is not provided PDSCH-TimeDomainResourceAllocationListForMultiPDSCH or is provided numberOfHARQ-BundlingGroups with value NHARQ-ACKTBG,max=1, for NcellsDL,TB serving cells where NcellsDL,TB+NcellsDL,TBG=NcellsDL the UE determines the o0ACK, o1ACK,⋯,oOACK-1ACK according to the previous pseudo-code with the following modifications - NcellsDL is used for the determination of a first HARQ-ACK sub-codebook for - SPS PDSCH reception, - any DCI format having associated HARQ-ACK information without scheduling PDSCH reception, and - PDSCH reception scheduled by a DCI format scheduling one PDSCH - PDSCH reception with NHARQ-ACKTBG,max=1 for TBG-based HARQ-ACK information on the NcellsDL,TB serving cells,. In that sense, I think the whole following part seems totally redundant, since two sub-codebook generation is sufficiently described under Condition 4… What do you think? If a UE is not provided numberOfHARQ-BundlingGroups, detects a first DCI format scheduling one PDSCH reception or having associated HARQ-ACK information without scheduling a PDSCH reception, if any, and a second DCI format scheduling more than one PDSCH reception on a serving cell from the NcellsDL serving cells, if any, and the UE would provide corresponding HARQ-ACK information in a same PUCCH, the UE determines the o0ACK, o1ACK,⋯,oOACK-1ACK according to the previous pseudo-code with the following modifications - the UE determines a first HARQ-ACK sub-codebook based on each detected DCI format scheduling one PDSCH reception or having associated HARQ-ACK information without scheduling a PDSCH reception, or SPS PDSCH receptions, if any, and - the UE determines a second HARQ-ACK sub-codebook based on each detected DCI format scheduling more than one PDSCH reception, and - instead of generating one HARQ-ACK information bit per transport block for serving cell c, the UE generates NHARQ-ACKmax HARQ-ACK information bits where NHARQ-ACKmax is the maximum value of NTB,cDL⋅NPDSCH,cmax across all NcellsDL serving cells, NPDSCH,cmax is a maximum number of PDSCH receptions that can be scheduled by a DCI format on serving cell c as described in [6, TS 38.214], and NTB,cDL is the value of maxNrofCodeWordsScheduledByDCI for serving cell c if harq-ACK-SpatialBundlingPUCCH is not provided; else, NTB,cDL=1. The UE generates the NHARQ-ACKmax HARQ-ACK information bits in ascending order of the NPDSCH,cmax PDSCHs, including any PDSCH that overlaps with UL symbols indicated by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated if provided. If, for serving cell c, the UE detects a DCI format that schedules NPDSCH,c PDSCH receptions and NTB,cDL⋅NPDSCH,c<NHARQ-ACKmax , the UE generates NACK for the last NHARQ-ACKmax -NTB,cDL⋅NPDSCH,c HARQ-ACK information bits - The pseudo-code operation when PDSCH-CodeBlockGroupTransmission is provided is not applicable. - The counter DAI value and the total DAI value apply separately for each HARQ-ACK sub-codebook. - The UE generates the HARQ-ACK codebook by appending the second HARQ-ACK sub-codebook to the first HARQ-ACK sub-codebook.. |
| Huawei, HiSilicon | Sorry for the confusion. My understanding of condition 4 is that some of CCs (NcellsDL,TBG) are configured with PDSCH-TimeDomainResourceAllocationListForMultiPDSCH while the other are not (NcellsDL,TB). For condition 1, it is written from the perspective of the scheduling UE received. It can be interpret as either all CCs or part of CCs are configured with PDSCH-TimeDomainResourceAllocationListForMultiPDSCH. Thus, I agree it is to some extend overlapped with condition 4. However, it is also the basis of condition 2 and 3 according to my reading which also provide information not mentioned in condition 4. If condition 4 is the superset of condition 1/2/3, maybe we should consider the procedure to merge condition 2/3 into condition 4 and remove the text for condition 1. |
| Moderator4 | @ Huawei, Thanks a lot for follow-up. In my view, I don’t see any problem if the above green texts are deleted at all. Please let me know if there is any problem. @ all, It would be highly appreciated if you could answer my question whether it is possible to remove the green text above. (since it seem redundant). |
| Intel | We share moderator’s view that the part of condition 1 from Huawei’s comments can be covered by part of condition 4 |
| ZTE, Sanechips | We would like to ask Moderator what the green text to be removed corresponds to. But if our understanding is correct, we share the same views with Moderator and Intel. |
| Moderator5 | @ ZTE, and all, Sorry for making confusion for green text. Now I marked green text in “Moderator3” above. |
| Samsung | We agree with Moderator. Condition 1 is included in Condition 4 and that is the reason why we said the specification update is not needed. Regarding whether the spec text for condition 1 is removed or kept, we are fine with both ways. |
| Huawei, HiSilicon | Thanks moderator for the clarification. We also agree that condition 1 is covered by condition 4. Our main concern to remove condition 1 individually is that it will make two paragraphs of condition 2/3 split from the rest pseudo code. If my understanding is correct, the paragraph of condition 2/3 is going to replace the following paragraph under condition 1, i.e. “ - instead of generating one HARQ-ACK information bit per transport block for serving cell c, the UE generates NHARQ-ACKmax HARQ-ACK information bits where NHARQ-ACKmax is the maximum value of NTB,cDL⋅NPDSCH,cmax across all NcellsDL serving cells, …” If finally, companies are fine to remove the whole text of condition 1, I would suggest to move the two paragraphs for condition 2/3 after text of condition 4, or even merge into the paragraph of “instead of generating one HARQ-ACK information bit per transport block for a serving cell from the NcellsDL,TBG serving cells, the UE generates NHARQ-ACK,maxTBG,max HARQ-ACK information bits, where NHARQ-ACK,maxTBG,max is the maximum value between NTB,cDL⋅NHARQ-ACK,cTBG,max across all NcellsDL,TBG serving cells if the UE is provided numberOfHARQ-BundlingGroups,…” in condition 4. |

- Proposal #4-11 (Type-2 CB):

Adopt the following text proposal to TS38.213 v17.1.0 Clause 9.1.3.1.

- Reason for change

  - Type-2 HARQ-ACK codebook generation when numberOfHARQ-BundlingGroups is not configured, is duplicated.

- Summary of change

  - Duplicated paragraph for type-2 HARQ-ACK codebook generation is deleted.

- Consequences if not approved

  - Incompact description in specification since type-2 HARQ-ACK codebook generation is duplicated when numberOfHARQ-BundlingGroups is not configured.

--------------------------------------------Start of text proposal to TS38.213 v17.1.0 -------------------------------------------

9.1.3.1 Type-2 HARQ-ACK codebook in physical uplink control channel

< Unchanged parts are omitted >

NcellsDLo0ACK, o1ACK,⋯,oOACK-1ACK

cNHARQ-ACKmaxNHARQ-ACKmaxNTB,cDL⋅NPDSCH,cmaxNcellsDLNPDSCH,cmaxcNTB,cDLcNTB,cDL=1NHARQ-ACKmaxNPDSCH,cmaxcNPDSCH,cNTB,cDL⋅NPDSCH,c<NHARQ-ACKmax NHARQ-ACKmax -NTB,cDL⋅NPDSCH,c

If a UE is provided numberOfHARQ-BundlingGroups and is not provided harq-ACK-SpatialBundlingPUCCH for a serving cell c, the UE generates HARQ-ACK information over transport block groups (TBGs) for PDSCH receptions where, for a maximum number of NPDSCHmax PDSCH receptions scheduled by a DCI format on the serving cell, a maximum number of TBGs NHARQ-ACK,cTBG,max is provided by numberOfHARQ-BundlingGroups. If the UE detects a DCI format scheduling NPDSCH,c PDSCH receptions on the serving cell c, the UE generates NHARQ-ACK,cTBG,max HARQ-ACK information bits for first TBs and, if applicable, generates NHARQ-ACK,cTBG,max HARQ-ACK information bits for second TBs in the NPDSCH,c PDSCH receptions as described in clause 9.1.1 by setting NHARQ-ACKCBG/TB,max=NHARQ-ACK,cTBG,max and C=NPDSCH,c. For a TBG associated with at least one PDSCH that does not overlap with an UL symbol indicated by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated if provided, the UE assumes that TB(s) provided by a PDSCH that overlaps with an UL symbol indicated by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated if provided, are correctly received. For a TBG associated only with PDSCHs that overlap with UL symbols indicated by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated if provided, the UE generates a NACK value for the TBG.

If a UE is provided numberOfHARQ-BundlingGroups and harq-ACK-SpatialBundlingPUCCH for a serving cell c, the UE generates HARQ-ACK information over PDSCH reception groups for PDSCH receptions scheduled by a DCI format on the serving cell c where a maximum number of PDSCH reception groups, NHARQ-ACK,cTBG,max, is provided by numberOfHARQ-BundlingGroups. If the UE detects a DCI format scheduling NPDSCH,c PDSCH receptions on the serving cell c, the UE generates NHARQ-ACK,cTBG,max HARQ-ACK information bits for the NPDSCH,c PDSCH receptions as described in clause 9.1.1 by setting NHARQ-ACKCBG/TB,max=NHARQ-ACK,cTBG,max and C=NPDSCH,c. For a PDSCH reception group associated with at least one PDSCH that does not overlap with an UL symbol indicated by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated if provided, the UE assumes that TBs provided by a PDSCH that overlaps with an UL symbol indicated by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated if provided, are correctly received. For a PDSCH reception group associated only with PDSCHs that overlap with UL symbols indicated by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated if provided, the UE generates a NACK value for the PDSCH reception group.

If a UE

-	is provided PDSCH-TimeDomainResourceAllocationListForMultiPDSCH and, if provided, numberOfHARQ-BundlingGroups with value NHARQ-ACKTBG,max>1 for NcellsDL,TBG serving cells; and

-	is not provided PDSCH-TimeDomainResourceAllocationListForMultiPDSCH or is provided numberOfHARQ-BundlingGroups with value NHARQ-ACKTBG,max=1, for NcellsDL,TB serving cells where NcellsDL,TB+NcellsDL,TBG=NcellsDL

the UE determines the o0ACK, o1ACK,⋯,oOACK-1ACK according to the previous pseudo-code with the following modifications

-	NcellsDL is used for the determination of a first HARQ-ACK sub-codebook for

-	SPS PDSCH reception,

-	any DCI format having associated HARQ-ACK information without scheduling PDSCH reception, and

-	PDSCH reception scheduled by a DCI format scheduling one PDSCH

-	PDSCH reception with NHARQ-ACKTBG,max=1 for TBG-based HARQ-ACK information on the NcellsDL,TB serving cells,

-	NcellsDL is replaced by NcellsDL,TBG for the determination of a second HARQ-ACK sub-codebook corresponding to the NcellsDL,TBG serving cells for TBG-based HARQ-ACK information, or for TB-based HARQ-ACK information corresponding to multiple PDSCH receptions scheduled by a single DCI format, and

-	if, for an active DL BWP of a serving cell, the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for one or more first CORESETs and is provided coresetPoolIndex with value 1 for one or more second CORESETs, and is provided ackNackFeedbackMode = joint, the serving cell is counted as two times where the first time corresponds to the first CORESETs and the second time corresponds to the second CORESETs, and

-	instead of generating one HARQ-ACK information bit per transport block for a serving cell from the NcellsDL,TBG serving cells, the UE generates NHARQ-ACK,maxTBG,max HARQ-ACK information bits, where NHARQ-ACK,maxTBG,max is the maximum value between NTB,cDL⋅NHARQ-ACK,cTBG,max across all NcellsDL,TBG serving cells if the UE is provided numberOfHARQ-BundlingGroups, and NTB,cDL⋅NPDSCH,cmax across all NcellsDL,TBG serving cells where the UE is not provided numberOfHARQ-BundlingGroups, and NTB,cDL is the value of maxNrofCodeWordsScheduledByDCI for serving cell c if harq-ACK-SpatialBundlingPUCCH is not provided; else, NTB,cDL=1. If for a serving cell c where the UE is provided numberOfHARQ-BundlingGroups, it is NTB,cDL⋅NHARQ-ACK,cTBG,max<NHARQ-ACK,maxTBG,max, the UE generates NACK for the last NHARQ-ACK,maxTBG,max-NTB,cDL⋅NHARQ-ACK,cTBG,max HARQ-ACK information bits for serving cell c. If for a serving cell c where the UE is not provided numberOfHARQ-BundlingGroups, it is NTB,cDL⋅NPDSCH,cmax <NHARQ-ACK,maxTBG,max, the UE generates NACK for the last NHARQ-ACK,maxTBG,max-NTB,cDL⋅NPDSCH,cmax  HARQ-ACK information bits for serving cell c.

-	The pseudo-code operation when PDSCH-CodeBlockGroupTransmission is provided is not applicable.

-	The counter DAI value and the total DAI value apply separately for each HARQ-ACK sub-codebook.

-	The UE generates the HARQ-ACK codebook by appending the second HARQ-ACK sub-codebook to the first HARQ-ACK sub-codebook.

--------------------------------------------End of Text proposal to TS38.213 v17.1.0 --------------------------------------------

_

Companies are encouraged to provide views on Proposal #4-11.

| Company | Views |
| --- | --- |
| vivo | We are fine with Proposal #4-11. |
| Xiaomi | OK with the TP |

(Closed) (E) Issue#4-12: Clarification on nHARQ-ACK,TB when it is used for the first sub-codebook out of two sub-codebooks

| Company | Views |
| --- | --- |
| [6] vivo | ---------------------------------------------------Start TP3------------------------------------------------------ Reason for change: When nHARQ-ACK,TB is used as a component to calculate other nHARQ-ACK (than nHARQ-ACK=nHARQ-ACK,TB), identified cases applied only for the equation nHARQ-ACK=nHARQ-ACK,TB may not be applicable anymore. Summary of change: Clarify the purpose or covered HARQ-ACK case(s) by nHARQ-ACK,TB when it is used as a component to calculate other nHARQ-ACK (than nHARQ-ACK=nHARQ-ACK,TB). Consequences if not approved: There may be ambiguity on whether the identified cases applied for the equation nHARQ-ACK=nHARQ-ACK,TB is also applicable for nHARQ-ACK,TB when it is used as a component to calculate other nHARQ-ACK (than nHARQ-ACK=nHARQ-ACK,TB). 9.1.3.1 Type-2 HARQ-ACK codebook in physical uplink control channel … If OACK+OSR+OCSI≤11 and NcellsDL,TBG>0, the UE also determines nHARQ-ACK=nHARQ-ACK,TB+nHARQ-ACK,TBG for obtaining a PUCCH transmission power, as described in clause 7.2.1, where nHARQ-ACK,TB corresponds to HARQ-ACK information contained in the first HARQ-ACK sub-codebook, and nHARQ-ACK,TBG corresponds to HARQ-ACK information contained in the second HARQ-ACK sub-codebook, with nHARQ-ACK,TBG=VDAI,mlastDL-c=0NcellsDL,TBG-1UDAI,cTBGmodTDNHARQ-ACK,maxTBG,max+c=0NcellsDL,TBG-1m=0M-1Nm,creceived,TBG where … --------------------------------------------------End TP3------------------------------------------------------- Proposal 9: Clarify the purpose or applicable case(s) of nHARQ-ACK,TB when it is used as a component to calculate other nHARQ-ACK (than nHARQ-ACK=nHARQ-ACK,TB) to avid ambiguity, and the TP3 can be considered in TS38.213. |

- [Moderator’s note] Do you agree on the above TP3? If not, please provide a reason or alternative TP.

| Company | Views |
| --- | --- |
| Samsung | Not necessary. The second sub-codebook corresponds to the TBG-based HARQ-ACK information or TB-based HARQ-ACK information corresponding to multiple PDSCH receptions by the following sentence in 213 NcellsDL is replaced by NcellsDL,TBG for the determination of a second HARQ-ACK sub-codebook corresponding to the NcellsDL,TBG serving cells for TBG-based HARQ-ACK information, or for TB-based HARQ-ACK information corresponding to multiple PDSCH receptions scheduled by a single DCI format |
| ZTE, Sanechips | Not necessary. According to the description of TS 38.213, the applicable cases of nHARQ,TB and nHARQ,TBG are clear. |
| Intel | It seems questionable whether the revision is necessary since the default understanding must be TB-based HARQ-ACK is in the first sub-codebook, while others in the second sub-codebook. |
| Ericsson | Seems not necessary |
| Nokia, NSB | We can accept the text proposal. |
| Huawei, HiSilicon | We are fine with TP. At the same time the corresponding text for CBG based HARQ-ACK should be changed accordingly considering the same situation. “If OACK+OSR+OCSI≤11, the UE also determines nHARQ-ACK=nHARQ-ACK,TB+nHARQ-ACK,CBG for obtaining a PUCCH transmission power, as described in clause 7.2.1, where nHARQ-ACK,TB corresponds to HARQ-ACK information contained in the first HARQ-ACK sub-codebook, and nHARQ-ACK,CBG corresponds to HARQ-ACK information contained in the second HARQ-ACK sub-codebook, with” |
| vivo | We prefer to explicitly clarify the understanding in the spec to avoid ambiguity. |
| CATT | We are fine with the correction |
| OPPO | Seems not necessary. |
| Moderator | I couldn’t see if there is a consensus on the necessity of this TP. However, based on Huawei’s comments, this issue also occurs from Rel-15 when two sub-codebooks are generated with CBG configured. If this is the case, we may not need a further correction. |
| Samsung | Agree with Moderator’s comment |
| Xiaomi | Ok with Moderator’s note |
| Moderator | This issue can be closed in this meeting. |

(Closed) (E) Issue#4-13: Reflect the agreement that priority indicator indicated in a multi-PXSCH scheduling DCI is applied to all of scheduled PXSCHs

| Company | Views |
| --- | --- |
| [7] NEC | Proposal 1: - Adopt following text change for clause 9 in TS 38.213. 9 UE procedure for reporting control information [….] If in an active DL BWP a UE monitors PDCCH for detection of DCI format that includes a priority indicator field, a priority index can be provided by the priority indicator field. If a UE indicates a capability to monitor, in an active DL BWP, PDCCH for detection of DCI format that includes a priority indicator field, the DCI format can schedule PUSCH transmission(s) of any priority, or PDSCH reception(s) and/or trigger a PUCCH transmission with corresponding HARQ-ACK information of any priority, and DCI format 1_1 or DCI format 1_2 can indicate a TCI state update and trigger a PUCCH transmission with corresponding HARQ-ACK information of any priority. A DCI format indicating a SPS PDSCH release, or SCell dormancy without scheduling a PDSCH reception, or indicating a TCI state update without scheduling PDSCH reception, is referred to as a DCI format having associated HARQ-ACK information without scheduling a PDSCH reception. [….]. |

- [Moderator’s note] Do you agree on the above TP? If not, please provide a reason or alternative TP.

| Company | Views |
| --- | --- |
| Samsung | Fine for the TP |
| ZTE, Sanechips | Support. |
| Intel | We are fine with the proposal. |
| Apple | Fine with proposal |
| Ericsson | Fine with the TP |
| InterDigital | We are fine with the TP |
| Panasonic | We are fine with the TP. |
| Qualcomm | We are fine with the TP |
| Huawei, HiSilicon | Support the TP |
| Fujitsu | Fine with the proposal |
| vivo | Fine with the TP. |
| CATT1 | Fine with the TP. |
| OPPO | Fine with the TP. |
| Moderator | This TP seems stable. So, I will recommend it for the email approval. |

- Proposal #4-13 (Priority indicator):

Adopt the following text proposal to TS38.213 v17.1.0 Clause 9.

- Reason for change

  - The following agreement is not captured in current specification.

Agreement: (RAN1#106-e) - For a DCI that can schedule multiple PUSCHs, - Priority indicator and open loop power control parameter set indication fields are applied to all of scheduled PUSCHs. - For a DCI that can schedule multiple PDSCHs, - Priority indicator field is applied to all of scheduled PDSCHs.

- Summary of change

  - Apply priority indicator to multiple PUSCHs or PDSCHs which are scheduling by a single DCI.

- Consequences if not approved

  - Missing agreement in specification

----------------------------------------------Start of text proposal to TS38.213 v17.1.0 --------------------------------------------

9 UE procedure for reporting control information

< Unchanged parts are omitted >

If in an active DL BWP a UE monitors PDCCH for detection of DCI format that includes a priority indicator field, a priority index can be provided by the priority indicator field. If a UE indicates a capability to monitor, in an active DL BWP, PDCCH for detection of DCI format that includes a priority indicator field, the DCI format can schedule PUSCH transmission(s) of any priority, or PDSCH reception(s) and/or trigger a PUCCH transmission with corresponding HARQ-ACK information of any priority, and DCI format 1_1 or DCI format 1_2 can indicate a TCI state update and trigger a PUCCH transmission with corresponding HARQ-ACK information of any priority.

--------------------------------------------End of Text proposal to TS38.213 v17.1.0 --------------------------------------------

Companies are encouraged to provide views on Proposal #4-13.

| Company | Views |
| --- | --- |
| Xiaomi | Support. |
| Samsung | Support |
| Intel | We support the TP. |
| Qualcomm | We support the proposal |
| Huawei, HiSilicon | Support the proposal |
| vivo | Support the proposal. |
| Moderator | The TP is agreed by email, so this issue can be closed. |

(Closed) Issue#4-16: Restriction on the number of PDSCH receptions/PUSCH transmissions in a slot

| Company | Views |
| --- | --- |
| [10] Samsung | We have the following agreements for 480/960 kHz SCS scenario. Agreement: (RAN1#106-e) - For single TRP operation, for 480/960 kHz SCS, - A UE does not expect to be scheduled with more than one unicast PDSCH in a slot, by a single DCI or multiple DCIs. - A UE does not expect to be scheduled with more than one PUSCH in a slot, by a single DCI or multiple DCIs. Agreement (RAN1#107-e) - For multi-TRP operation, for 480/960 kHz SCS, - A UE does not expect to be scheduled with more than one unicast PDSCH in a slot, by a single DCI or multiple DCIs, from the same TRP. - A UE does not expect to be scheduled with more than one PUSCH in a slot, by a single DCI or multiple DCIs, from the same TRP. - Note: This does not preclude a UE being scheduled with two PDSCHs (or two PUSCHs) in the same slot from two different TRPs for multi-DCI based multi-TRP mechanism.. If there is SPS PDSCH/CG PUSCH in the slot, UE behaviour is not clear. For example, whether UE can receive a unicast PDSCH and a SPS PDSCH in a slot? The intention of the agreement is to restrict that there is up to 1 PDSCH/PUSCH per slot. Same rule should apply to SPS PDSCH/CG PUSCH. In addition, whether the scheduled PDSCH/PUSCH is a valid one should be clarified. Proposal 1: For single TRP or multi-TRP operation, for 480/960 kHz SCS, - A UE does not expect to receive more than one unicast PDSCH in a slot on a serving cell from the same TRP. - A UE does not expect to transmit more than one PUSCH in a slot on a serving cell from the same TRP. |

- [Moderator’s note] Companies are encouraged to express whether above Proposal 1 is agreeable or not.

| Company | Views |
| --- | --- |
| Samsung | We support the proposal as a proponent |
| ZTE, Sanechips | Support |
| Intel | We are fine with the proposal. |
| Apple | Support the proposal |
| Ericsson | Fine with the proposal |
| InterDigital | We are fine with the proposal. |
| Nokia, NSB | We are fine with the proposal in principle. Good to add note for clarification. (note: this is extending the former agreements for scheduled by DCI to the case when scheduling without DCI (SPS/CG)) |
| Panasonic | We are fine with the proposal. |
| Qualcomm | We support the proposal |
| Huawei, HiSilicon | Support the proposal |
| Fujitsu | Fine with the proposal |
| vivo | We are fine with the proposal. |
| CATT1 | Fine with the proposal |
| OPPO | Fine with the proposal. |

- Proposal #4-16 (TDMed PXSCHs in a slot):

For single TRP or multi-TRP operation, for 480/960 kHz SCS,

- A UE does not expect to receive more than one unicast PDSCH in a slot on a serving cell from the same TRP.

- A UE does not expect to transmit more than one PUSCH in a slot on a serving cell from the same TRP.

Companies are encouraged to provide views on Proposal #4-16.

| Company | Views |
| --- | --- |
| Moderator | It seems that Samsung’s proposal is agreeable to all companies. Therefore, I will recommend this proposal for email approval. Please let us know if you have a concern. By the way, @ Nokia, In my view, this proposal has nothing to do with single SLIV restriction of SPS/CG/TCI state update. Rather, it may have a specification impact to the following sentences. 38.214 When configured with SCS = 5 or = 6, the UE does not expect to be scheduled with more than one unicast PDSCH in a slot, by a single DCI scheduling multiple PDSCHs or by multiple DCIs. … When configured with = 5 or 6 the UE does not expect to be scheduled with more than one PUSCH in a slot, by a single DCI or multiple DCIs. |
| Xiaomi | OK with the Proposal. |
| Samsung | Support |
| Intel | We are fine with the proposal. |
| ASUSTeK | Support |
| Qualcomm | We support the proposal |
| Huawei, HiSilicon | Support the proposal |
| Apple | Support |
| vivo | Support the proposal. |
| Moderator | This issue is agreed by email, so it can be closed in this meeting. |

(E) Issue#4-18: Reflect the agreement that UE does not apply pusch-AggregationFactor to DCI format 0_1 if multi-PUSCH scheduling DCI is configured

| Company | Views |
| --- | --- |
| [13] ASUSTeK | Observation 1: According to current standard, when the UE is configured with pusch-AggregationFactor and pusch-TimeDomainAllocationListForMultiPUSCH-r17, the number of repetitions K for PUSCH scheduled by DCI format 0_1 is not clear. Proposal 1: RAN1 clarify that the UE does not apply pusch-AggregationFactor to DCI format 0_1 is equivalent to that the number of repetitions K is 1. Proposal 2: Adopt the Text proposal 1 or Text proposal 2 in updating of TS38.214 section 6.1.2.1, for determining number of repetitions K. Text proposal 1 ### 6.1.2 Resource allocation #### 6.1.2.1 Resource allocation in time domain <Omitted> For PUSCH repetition Type A, when transmitting PUSCH scheduled by DCI format 0_1 or 0_2 in PDCCH with CRC scrambled with C-RNTI, MCS-C-RNTI, or CS-RNTI with NDI=1, the number of repetitions K is determined as - if numberOfRepetitions is present in the resource allocation table, the number of repetitions K is equal to numberOfRepetitions; - elseif the UE is configured with pusch-AggregationFactor and the transmitting PUSCH is scheduled by DCI format 0_2, the number of repetitions K is equal to pusch-AggregationFactor; - elseif the UE is configured with pusch-AggregationFactor, (and the transmitting PUSCH is scheduled by DCI format 0_1) and not configured with pusch-TimeDomainAllocationListForMultiPUSCH-r17, the number of repetitions K is equal to puschAggregationFactor; - otherwise K=1. <Omitted> If a UE is configured with pusch-TimeDomainAllocationListForMultiPUSCH-r17 in which one or more rows contain multiple SLIVs for PUSCH on a UL BWP of a serving cell, the UE does not apply pusch-AggregationFactor, if configured, to DCI format 0_1 on the UL BWP of the serving cell and the UE does not expect to be configured with numberOfRepetitions in pusch-TimeDomainAllocationListForMultiPDSCH-r17. <Omitted>. Text proposal 2 ### 6.1.2 Resource allocation #### 6.1.2.1 Resource allocation in time domain <Omitted> For PUSCH repetition Type A, when transmitting PUSCH scheduled by DCI format 0_1 or 0_2 in PDCCH with CRC scrambled with C-RNTI, MCS-C-RNTI, or CS-RNTI with NDI=1, the number of repetitions K is determined as - if numberOfRepetitions is present in the resource allocation table, the number of repetitions K is equal to numberOfRepetitions; - elseif the UE is configured with pusch-AggregationFactor and not configured with pusch-TimeDomainAllocationListForMultiPUSCH-r17 for corresponding DCI format, the number of repetitions K is equal to pusch-AggregationFactor; - otherwise K=1. <Omitted> If a UE is configured with pusch-TimeDomainAllocationListForMultiPUSCH-r17 in which one or more rows contain multiple SLIVs for PUSCH on a UL BWP of a serving cell, the UE does not apply pusch-AggregationFactor, if configured, to DCI format 0_1 on the UL BWP of the serving cell and the UE does not expect to be configured with numberOfRepetitions in pusch-TimeDomainAllocationListForMultiPDSCH-r17. <Omitted>. |

- [Moderator’s note] Companies are encouraged to express which TP between above Text proposals 1 and 2 is preferable.

| Company | Views |
| --- | --- |
| Samsung | No needed. It was discussed in several meeting and as we commented, the specification is not broken from the text in 214 If a UE is configured with pusch-TimeDomainAllocationListForMultiPUSCH-r17 in which one or more rows contain multiple SLIVs for PUSCH on a UL BWP of a serving cell, the UE does not apply pusch-AggregationFactor, if configured, to DCI format 0_1 on the UL BWP of the serving cell and the UE does not expect to be configured with numberOfRepetitions in pusch-TimeDomainAllocationListForMultiPUSCH-r17. |
| Intel | It is not clear whether this is needed. This was already captured as follows, e.g., for PUSCH. If a UE is configured with pusch-TimeDomainAllocationListForMultiPUSCH-r17 in which one or more rows contain multiple SLIVs for PUSCH on a UL BWP of a serving cell, the UE does not apply pusch-AggregationFactor, if configured, to DCI format 0_1 on the UL BWP of the serving cell and the UE does not expect to be configured with numberOfRepetitions in pusch-TimeDomainAllocationListForMultiPUSCH-r17.. |
| Ericsson | Agree with Samsung and Intel |
| ASUSTeK | We don’t think “the UE does not apply pusch-AggregationFactor, if configured, to DCI format 0_1” is enough for the issue. Because, in Sec. 6.1.2.1 in current TS 38.214 for determination of K is equal to pusch-AggregationFactor once the UE is configured with pusch-AggregationFactor. There is no further condition check for determination of K according to “apply or not apply pusch-AggregationFactor”. Therefore, we think it would be better to capture the change. If majority company prefer to keep the current procedural text, we propose to fix this issue in the paragraph texts as below. If a UE is configured with pusch-TimeDomainAllocationListForMultiPDSCH-r17 in which one or more rows contain multiple SLIVs for PUSCH on a UL BWP of a serving cell, the UE does not apply pusch-AggregationFactor, if configured, to DCI format 0_1 on the UL BWP of the serving cell (the number of repetition K=1) and the UE does not expect to be configured with numberOfRepetitions in pusch-TimeDomainAllocationListForMultiPDSCH-r17. If majority company think there’s no need to have spec impact, can we make a conclusion to ensure there is no misunderstanding. Conclusion: The UE does not apply pusch-AggregationFactor, if configured, to DCI format 0_1 means the number of repetitions K is 1. |
| Nokia, NSB | We think that the need for this TP is unclear (the current spec seems to be clear enough) |
| Panasonic | Agree with Samsung and Intel. |
| Qualcomm | We do not see a need for this TP as illustrated by Samsung and Intel |
| Huawei, HiSilicon | Agree with intel and Samsung |
| vivo | To avoid potential inconsistent or ambiguity, ASUSTeK’ proposal to fix this issue by adding “(the number of repetition K=1)” in the spec or making a conclusion can be considered. |
| CATT1 | We share the view with vivo. |
| OPPO | Agree with Samsung and Intel. |
| Moderator | Although majority companies opposed to TPs, considering that we have discussed this issue for several meetings, it would be better to make a conclusion as ASUSTeK suggested. |

- Proposed Conclusion #4-18 (Slot aggregation):

- It is clarified that “it does not apply to DCI format 1_1 or 0_1” in the previous agreement implies the number of repetition K is equal to 1.

Agreement (RAN1#107-e) - If a UE is configured with a TDRA table in which one or more rows contain multiple SLIVs for PDSCH for DCI format 1_1, the UE does not expect to be configured with repetitionNumber for the TDRA table, and if pdsch-AggregationFactor is configued in PDSCH-config, it does not apply to DCI format 1_1. - Note: repetitionNumber cannot be configured with pdsch-TimeDomainAllocationListDCI-1-2 as in Rel-16. - Note: Under agenda item 8.2.4, in RAN1#106-bis, it was already agreed that within the TDRA table for multi-PDSCH scheduling, the UE does not expect to be configured with the higher layer parameter repetitionNumber. - Note: These does not preclude pdsch-AggregationFactor can be configured and applies to DCI format 1_2 - If a UE is configured with a TDRA table in which one or more rows contain multiple SLIVs for PUSCH for DCI format 0_1, the UE does not expect to be configured with numberOfRepetitions for the TDRA table, and if pusch-AggregationFactor is configued in PUSCH-config, it does not apply to DCI format 0_1. - Note: These does not preclude numberOfRepetitions is configured for TDRA table corresponding to DCI format 0_2 - Note: These does not preclude pusch-AggregationFactor can be configured and applies to DCI format 0_2

Companies are encouraged to provide views on Proposed Conclusion #4-18.

| Company | Views |
| --- | --- |
| Xiaomi | OK with the Conclusion |
| Samsung | Fine with the proposed conclusion |
| Intel | We are fine with the proposed conclusion |
| ASUSTeK | Fine with FL’s conclusion. |
| Ericsson | Support Conclusion #4-18 |
| Qualcomm | We are okay with the conclusion |
| Huawei, HiSilicon | Support conclusion |
| vivo | Fine with the proposed conclusion. |
| ZTE, Sanechips | Fine with the proposed conclusion. |
| Moderator | This proposal seems stable, so it can be suggested for email approval. |

Issue#4-21: Clarification on the applicability of K1 set extension for K1 values for DCI 1_0

| Company | Views |
| --- | --- |
| [19] Intel | ## 3.2 DCI format 1_0 in Type-1 HARQ-ACK codebook In the existing Type-1 HARQ-ACK transmission in NR, a DCI format 1_0 can only be used to schedule a PDSCH transmission if the indicated K1 value belongs to the intersection of the configured set of K1 values for DCI format 1_1/1_2 and the predefined set of K1 values for DCI format 1_0, i.e. {1,2,3,4,5,6,7,8}. The Type-1 HARQ-ACK transmission with time bundling for FR2-2 can be considered as generation of an extended set of K1 values then allocate occasions for candidate PDSCH reception for each of the extended K1 values. The usage of K1 in the current specification in section 9.1.2.1 in 38.213 is organized with the following structure. - in the beginning of the section, the definition of K1 is provided, e.g., by RRC configuration - Then, in the beginning of the pseudo code, the RRC configured K1 set is extended to account the multiple SLIVs of each row in TDRA table - K1=K1,T, which replaces RRC configured K1 set with the extended K1 set - in the following pseudo code, it works based on the extended K1 set - finally, after the pseudo code, the term ‘K1’ is used again for one time, which is related to the determination of applicable slot timing values for DCI 1_0 Based on the above analysis, the current specification is already in a logic to handle DCI 1_0 with the extended K1 set, though there is explicit discussion yet. It is worth noting that such behaviour is beneficial. That is, the applicable slot timing values for DCI 1_0 can be from the extended K1 set instead of the RRC configured K1 set. In other words, even when the intersection of the predefined K1 values for DCI 1_0 and the RRC configured K1 set for DCI 1_1/1_2 is empty, it is still possible to do schedule based on DCI 1_0, which provides large flexibility for gNB configuration. Therefore, we prefer to clarify that the intersection of predefined K1 values for DCI 1_0 and the extended K1 set for DCI 1_1/1_2 is applicable to DCI 1_0. With the clarification, no change to the current specification is needed. Otherwise, the update to the specification is necessary. - 9.1.2.1 Type-1 HARQ-ACK codebook in physical uplink control channel … [Intel: definition of K1 and other parameters] set K1,T=∅ while r<CR set Pr to the set of entries for row r set K0,r to the set of K0 values of entries for row r set ∆K0,r=maxK0K0,r-K0,r set CPr to the cardinality of Pr set C∆K0,r to the cardinality of ∆K0,r set p=0 – index of element in Prset d=0 – index of element in ∆K0,r while p<CPr RT=RT∪Prp; p=p+1; end while while d<C∆K0,r K1,T=K1,T∪K1+∆K0,r(d)⋅2μUL-μDL∪K1+∆K0,r(d)⋅2μUL-μDL; d=d+1; end while r=r+1; end while K1=K1,T; … [Intel: other pseudo code for Type-1 HARQ-ACK codebook generation] If a UE is provided dl-DataToUL-ACK or dl-DataToUL-ACK-DCI-1-2, the UE does not expect to be indicated by DCI format 1_0 a slot timing value for transmission of HARQ-ACK information that does not belong to the intersection of the set of slot timing values {1, 2, 3, 4, 5, 6, 7, 8} for SCS configuration of PUCCH transmission μ≤3, {7, 8, 12, 16, 20, 24, 28, 32} for μ=5, and {13, 16, 24, 32, 40, 48, 56, 64} for μ=6, and the set of slot timing values provided by K1for the active DL BWP of a corresponding serving cell. …. Proposal 4 - The intersection of predefined K1 values for DCI 1_0 and the extended K1 set for DCI 1_1/1_2 is applicable to DCI 1_0. - The current specification already covers the behavior. Hence no specification change is necessary |

- [Moderator’s note] Companies are encouraged to express whether above Proposal 4 is agreeable or not.

| Company | Views |
| --- | --- |
| Samsung | No need to make a conclusion if the specification already supports it. |
| Intel | Our understanding is the current specification is already in logic to allow intersection with extended K1 set for fallback DCI format 1_0. Since it is also beneficial for scheduling flexibility, we prefer to confirm the behavior is supported and spec changes are also avoided. |
| Qualcomm | Agree with Samsung |
| vivo | Share the same view as Intel. |
| CATT | We are fine as long as the group reach a common understanding regarding the current specification. |
| Moderator | Even though the number of inputs is small and company views are split, I don’t think companies have different understanding from Intel’s conclusion. So, I suggest to agree the following as a conclusion to have a common understanding within the group. |

- Proposed Conclusion #4-21 (K1 set extension):

- If type-1 HARQ-ACK codebook is configured, the intersection of predefined K1 values for DCI 1_0 and the extended K1 set for DCI 1_1/1_2 is applicable to DCI 1_0.

- The current specification already covers the behavior. Hence no specification change is necessary

Companies are encouraged to provide views on Proposed Conclusion #4-21.

| Company | Views |
| --- | --- |
| Xiaomi | OK with the Proposal. |
| Samsung | Although still we are no reasons to make a conclusion, we can accept the proposed conclusion. Minor updates are below - If type-1 HARQ-ACK CB is configured, Tthe intersection of predefined K1 values for DCI 1_0 and the extended K1 set for DCI 1_1/1_2 is applicable to DCI 1_0. - The current specification already covers the behavior. Hence no specification change is necessary |
| Intel | We support the proposal and revision from Samsung |
| ASUSTeK | Fine with FL’s conclusion or Samsung’s revision. |
| Ericsson | Support Conclusion #4-21 |
| Qualcomm | We are okay with the conclusion |
| Huawei, HiSilicon | Support the conclusion |
| vivo | We support the proposal and revision from Samsung. |
| Moderator | @ Samsung, and all, I reflected Samsung’s comment. |
| ZTE, Sanechips | We are fine with the updated conclusion. |
| Moderator | This proposal seems stable, so it can be suggested for email approval. |

Reference

- R1-2203081	Remaining issues of PDSCH/PUSCH enhancement for 52-71GHz spectrum	Huawei, HiSilicon

- R1-2203292	Remaining issues on data channel enhancements for 52.6 to 71GHz	ZTE, Sanechips

- R1-2203371	Remaining issues for PDSCH/PUSCH enhancements to supporting 52.6-71 GHz band in NR	InterDigital, Inc.

- R1-2203401	Discussion on PDSCH/PUSCH enhancements for NR 52.6-71 GHz	Panasonic

- R1-2203432	Remaining issues on PDSCH/PUSCH enhancements for up to 71GHz operation	CATT

- R1-2203510	Maintenance on PDSCH/PUSCH enhancements for NR operation from 52.6GHz to 71GHz	vivo

- R1-2203678	Remaining issues of PDSCH/PUSCH enhancements for 52.6 to 71GHz	NEC

- R1-2203708	Remaining issues of multi-PDSCH/PUSCH scheduling via a single DCI	Fujitsu Limited

- R1-2203784	Remaining issues on PDSCH and PUSCH enhancements  for NR 52.6-71GHz	xiaomi

- R1-2203860	Maintenance on PDSCH/PUSCH enhancements for NR from 52.6 GHz to 71 GHz	Samsung

- R1-2203988	Discussion on remaining issue for PDSCH/PUSCH enhancements	OPPO

- R1-2204112	PDSCH-PUSCH Enhancements	Ericsson

- R1-2204190	Discussion on multi-PXSCH scheduling	ASUSTeK

- R1-2204203	On remaining issues for PDSCH PUSCH Enhancements	Apple

- R1-2204340	Remaining issues on PDSCH/PUSCH enhancements for NR in FR2-2	NTT DOCOMO, INC.

- R1-2204601	Remaining issues on PDSCH/PUSCH enhancements	Nokia, Nokia Shanghai Bell

- R1-2204613	Remaining issues of PDSCH/PUSCH enhancements to support NR above 52.6 GHz	LG Electronics

- R1-2204707	Remaining discussion on multi-PDSCH scheduling design for 52.6-71 GHz NR operation	MediaTek Inc.

- R1-2204768	Discussion on PDSCH/PUSCH enhancements for extending NR up to 71 GHz	Intel Corporation

- R1-2204980	PDSCH and PUSCH enhancements	Qualcomm Incorporated

- Appendix: Previous agreements

- RAN1#104-e

Agreement:

- For a UE and for a serving cell, scheduling multiple PDSCHs by single DL DCI and scheduling multiple PUSCHs by single UL DCI are supported.

  - Each PDSCH or PUSCH has individual/separate TB(s) and each PDSCH/PUSCH is confined within a slot.

  - FFS: The maximum number of PDSCHs or PUSCHs that can be scheduled with a single DCI

  - FFS: Whether multiple PDSCH scheduling applies to 120 kHz in addition to 480 and 960 kHz

  - At least for 120 kHz SCS, single-slot scheduling with slot-based monitoring will still be supported as specified in Rel-15/Rel-16

- The followings will not be considered in this WI.

  - Single DCI to schedule both PDSCH(s) and PUSCH(s)

  - Single DCI to schedule one or multiple TBs where any single TB can be mapped over multiple slots, where mapping is not by repetition

  - Single DCI to schedule N TBs (N>1) where a TB can be repeated over multiple slots (or mini-slots)

- Note: This does not imply that existing slot aggregation and/or repetition for PDSCH and PUSCH by single DCI is precluded for the serving cell.

Agreement:

- For a DCI scheduling multiple PDSCHs, HARQ-ACK information corresponding to PDSCHs scheduled by the DCI is multiplexed with a single PUCCH in a slot that is determined based on K1,

  - where K1 (indicated by the PDSCH-to-HARQ_feedback timing indicator field in the DCI or provided by dl-DataToUL-ACK if the PDSCH-to-HARQ_feedback timing indicator field is not present in the DCI) indicates the slot offset between the slot of the last PDSCH scheduled by the DCI and the slot carrying the HARQ-ACK information corresponding to the scheduled PDSCHs.

    - It is noted that granularity of K1 can be separately discussed.

- FFS: If needed, further discuss whether or not HARQ-ACK information corresponding to different PDSCHs scheduled by the DCI can be carried by different PUCCH(s)

Agreement:

For generating type-2 HARQ-ACK codebook corresponding to DCI that can schedule multiple PDSCHs, the following alternatives can be considered to DAI counting and will be down-selected in RAN1#104bis-e.

- Alt 1: C-DAI/T-DAI is counted per DCI.

- Alt 2: C-DAI/T-DAI is counted per PDSCH.

- Alt 3: C-DAI/T-DAI is counted per M scheduled PDSCH(s), where M is configurable (e.g., 1, 2, 4, …).

- FFS: Codebook generation details

- FFS: How to signal DAI values (e.g., increase of DAI bits for Alt 2 and Alt 3)

- FFS: Whether to apply time domain bundling of HARQ-ACK feedback

Agreement:

The multi-PUSCH scheduling defined in Rel-16 NR-U is the baseline for multi-PUSCH scheduling in Rel-17.

- FFS: Applicability to multi-PDSCH scheduling.

Agreement:

- For the multi-PUSCH scheduling in Rel-17, study the enhancement of the following in addition to Rel-16 multi-PUSCH scheduling.

  - CBGTI: Whether or not CBG (re)transmission is supported when more than one PUSCHs are scheduled (Already supported when only one PUSCH is scheduled).

  - CSI-request: Whether to apply same or different rule compared to Rel-16 (e.g., the PUSCH that carries the AP-CSI feedback is the first PUSCH that satisfies the multiplexing timeline).

  - TDRA: Down-select among

    - Alt 1: TDRA table is extended such that each row indicates up to [X, FFS for X] multiple PUSCHs (continuous in time-domain). Each PUSCH has a separate SLIV and mapping type. The number of scheduled PUSCHs is signalled by the number of indicated valid SLIVs in the row of the TDRA table signalled in DCI.

    - Alt 2: TDRA table is extended such that each row indicates up to [X, FFS for X] multiple PUSCHs (that can be non-continuous in time-domain). Each PUSCH has a separate SLIV and mapping type. The number of scheduled PUSCHs is signalled by the number of indicated valid SLIVs in the row of the TDRA table signalled in DCI.

    - Alt 3: TDRA table is extended such that each row indicates up to 8 multiple PUSCH groups (that can be non-continuous between PUSCH groups). Each PUSCH group has a separate SLIV, mapping type and number of slots/PUSCHs N. Within each PUSCH group, N PUSCHs occupy the same OFDM symbols indicated by the SLIV and mapping type. The number of scheduled PUSCHs is the sum of number of PUSCHs in all PUSCH groups in the row of the TDRA table signalled in DCI.

  - FDRA: Whether/how to enhance FDRA e.g., by increasing RBG size or changing allocation granularity

  - Frequency hopping: Whether/how to support frequency hopping for scheduled PUSCHs, e.g., inter-PUSCH/intra-PUSCH hopping

  - URLLC related fields such as priority indicator and open-loop power control parameter set indication: Whether/how to apply URLLC related fields for scheduled PUSCHs

  - Applicability to multi-PDSCH scheduling in Rel-17.

  - Note: Other enhancements are not precluded.

- RAN1#104bis-e

Agreement:

- The maximum number of PDSCHs that can be scheduled with a single DCI in Rel-17 is 8 for SCS of 480 and 960 kHz.

  - FFS: Further restrictions for 480 kHz to 4

  - FFS: A UE capability to select between 4 and 8 for 480 kHz SCS

  - Note: Multi-PDSCH scheduling for the case of 120 kHz SCS is still FFS as per prior agreement. This case can be addressed after this FFS has been decided.

- The maximum number of PUSCHs that can be scheduled with a single DCI in Rel-17 is 8.

  - FFS: Further restrictions for 120 kHz and 480 kHz SCS

  - FFS: A UE capability to select between different values for 120 kHz and 480 kHz SCS

Agreement:

For a DCI that can schedule multiple PDSCHs,

- MCS for the 1st TB: This appears only once in the DCI and applies commonly to the first TB of each PDSCH

- NDI for the 1st TB: This is signaled per PDSCH and applies to the first TB of each PDSCH

- RV for the 1st TB: This is signaled per PDSCH, with 2 bits if only a single PDSCH is scheduled or 1 bit for each PDSCH otherwise and applies to the first TB of each PDSCH

- HARQ process number: This applies to the first scheduled PDSCH and is incremented by 1 for subsequent PDSCHs (with modulo operation, if needed)

- FFS:

  - MCS/NDI/RV for the 2nd TB for each PDSCH, including whether scheduling of the 2nd TB for each PDSCH can be supported or not

  - Details of resource allocation related fields such as VRB-to-PRB mapping, PRB bundling size indicator, rate matching indicator, and ZP CSI-RS trigger

  - Whether/how to signal CBGFI/CBGTI if CBGFI/CBGTI is supported for multi-PDSCH scheduling

  - Details of fields that are common with multi-PUSCH scheduling, e.g., TDRA, FDRA, priority indicator, including potential enhancements

Agreement:

- For a DCI that can schedule multiple PUSCHs,

  - TDRA: Alt 2 (TDRA table is extended such that each row indicates up to 8 multiple PUSCHs (that can be non-continuous in time-domain). Each PUSCH has a separate SLIV and mapping type. The number of scheduled PUSCHs is implicitly indicated by the number of indicated valid SLIVs in the row of the TDRA table signalled in DCI.), as per agreement made in RAN1#104-e

    - FFS: signaling details

  - Note: Alt 2 does not preclude continuous resource allocation in time-domain.

- For a DCI that can schedule multiple PDSCHs,

  - TDRA: TDRA table is extended such that each row indicates up to 8 multiple PDSCHs (that can be non-continuous in time-domain). Each PDSCH has a separate SLIV and mapping type. The number of scheduled PDSCHs is implicitly indicated by the number of indicated valid SLIVs in the row of the TDRA table signalled in DCI.

    - FFS: signaling details

  - Note: This does not preclude continuous resource allocation in time-domain.

  - Note: Multi-PDSCH scheduling for the case of 120 kHz SCS is still FFS as per prior agreement. This case can be addressed after this FFS has been decided.

Agreement:

For enhancements of generating type-1 HARQ-ACK codebook corresponding to DCI that can schedule multiple PDSCHs, the following options can be considered,

- Option 1: The set of candidate PDSCH reception occasions is determined according to each SLIV of each row in the TDRA table and based on extension of K1 set

- Option 1a: The set of candidate PDSCH reception occasions is determined according to each SLIV of each row in the TDRA table

- Option 2: The set of candidate PDSCH reception occasions is determined according to the last SLIV of each row in the TDRA table

- FFS: Codebook generation details, including how to handle the collision with TDD DL/UL configuration and whether/how to extend K1 set based on K1 and slot offset between last PDSCH and other PDSCHs in a row in the TDRA table

Conclusion:

The following is observed for alternative 1 from prior agreement.

- For Alt 1 (C-DAI/T-DAI is counted per DCI) of generating type-2 HARQ-ACK codebook corresponding to DCI that can schedule multiple PDSCHs,

  - C-DAI/T-DAI in DL DCI: Same DAI overhead with Rel-16 single-PDSCH DCI

  - T-DAI in UL DCI:

    - In case of single codebook handling feedback for both single and multi-PDSCH scheduling, same DAI overhead with Rel-16 UL DCI

    - In case of separate sub-codebooks, need additional DAI field (with same bit-width of DAI with Rel-16 UL DCI), in UL DCI for all serving cells including a serving cell not configured with multi-PDSCH DCI

      - Note that DAI field increment for this case is similar for the case in Rel-15 where CBG is configured

  - HARQ-ACK codebook generation:

    - A separate sub-codebook can be generated when multi-PDSCH DCI is configured for a serving cell, similar to the way as 2nd sub-codebook is defined to handle CBG-based scheduling

      - FFS: whether single codebook or separate sub-codebooks is(are) generated when multi-PDSCH DCI is configured for a serving cell

      - FFS: how many sub-codebooks are generated when multi-PDSCH DCI is configured for a serving cell and CBG is configured for the serving cell and/or the other serving cell(s)

    - HARQ-ACK payload size is increased compared to single PDSCH scheduling only, since the number of HARQ-ACK bits corresponding to each DAI of the (sub-)codebook for multi-PDSCH DCI in case of separate sub-codebooks (or for all DL DCIs in case of single codebook) depends on the maximum configured number of PDSCHs for multi-PDSCH DCI across serving cells belonging to the same PUCCH cell group.

    - The number of HARQ-ACK bits for multi-PDSCH DCI in case of separate sub-codebooks, or for all DL DCIs in case of single codebook, does not depend on the number of actually scheduled PDSCHs, rather, it is fixed as the maximum configured number of PDSCHs.

    - FFS: time domain bundling of HARQ-ACK feedback, as per agreement in RAN1#104-e

  - Note that multi-PDSCH DCI refers to a DL DCI where at least one entry of the TDRA table allows scheduling more than one PDSCH

Conclusion:

The following is observed for alternative 2 from prior agreement.

- For Alt 2a (C-DAI/T-DAI is counted per PDSCH with a single codebook) of generating type-2 HARQ-ACK codebook corresponding to DCI that can schedule multiple PDSCHs,

  - C-DAI/T-DAI in DL DCI: Bit-width can be increased (FFS: by how much), in DL DCI not only for multi-PDSCH DCI but also for single-PDSCH DCI for all serving cells including a serving cell not configured with multi-PDSCH DCI

  - T-DAI in UL DCI: Bit-width can be increased (FFS: by how much), in UL DCI for all serving cells including a serving cell not configured with multi-PDSCH DCI

  - C-DAI/T-DAI in DL DCI and T-DAI in UL DCI shall be designed such that at most 3 consecutive DCI missing can be resolved, same as in Rel-15/16 NR.

    - FFS: details on increment of DAI field size

    - FFS: whether/how to handle the case where different DCI formats (e.g., DCI format 1_0 and DCI format 1_1) have different field sizes for C-DAI/T-DAI

  - HARQ-ACK codebook generation:

    - The number of HARQ-ACK bits depends on the number of scheduled PDSCHs.

    - FFS: ordering of the PDSCHs for DAI counting

    - FFS: time domain bundling of HARQ-ACK feedback, as per agreement in RAN1#104-e

  - Note that multi-PDSCH DCI refers to a DL DCI where at least one entry of the TDRA table allows scheduling more than one PDSCH

Conclusion:

The following is observed for alternative 3 from prior agreement.

- For Alt 3 (C-DAI/T-DAI is counted per M scheduled PDSCH(s), where M is configurable) of generating type-2 HARQ-ACK codebook corresponding to DCI that can schedule multiple PDSCHs,

  - If M equals to the maximum configured number of PDSCHs, Alt 3 is the same with Alt 1, if the same number of codebooks is assumed.

  - Else if M equals to 1, Alt 3 is the same with Alt 2.

  - Otherwise (i.e., 1<M<the maximum configured number of PDSCHs), Alt 3 is similar to Alt 2, except that

    - The number of HARQ-ACK bits corresponding to each DAI increases by M times.

    - NACK bits may be padded if the number of scheduled PDSCHs is not an integer multiple of M.

    - FFS: details on DAI field size

    - FFS: whether single codebook or separate sub-codebooks is(are) generated when multi-PDSCH DCI is configured for a serving cell

  - In addition, new RRC parameter to configure M needs to be introduced.

  - Note that multi-PDSCH DCI refers to a DL DCI where at least one entry of the TDRA table allows scheduling more than one PDSCH

- RAN1#105-e

Agreement:

- Do not use fallback DCI (i.e., DCI formats 0_0 and 1_0) for multi-PDSCH/PUSCH scheduling.

- Use DCI format 0_1 to schedule multiple PUSCHs with a single DCI.

- Use DCI format 1_1 to schedule multiple PDSCHs with a single DCI.

Conclusion:

For a DCI that can schedule multiple PUSCHs,

- CSI-request: When the DCI schedules M PUSCHs, the PUSCH that carries the aperiodic CSI feedback is M-th scheduled PUSCH for M <= 2, or (M-1)-th scheduled PUSCH for M > 2.

Agreement:

- If a PDSCH among multiple PDSCHs that are scheduled by a single DCI is collided with uplink symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, the UE does not receive the PDSCH.

  - FFS on how to handle HARQ-related issue for the PDSCH (e.g., HARQ process numbering)

- The UE does not expect to be scheduled with multiple PDSCHs by a single DCI, where every PDSCH is collided with uplink symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated.

- If a PUSCH among multiple PUSCHs that are scheduled by a single DCI is collided with downlink symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, the UE does not transmit the PUSCH.

  - FFS on how to handle HARQ-related issue for the PUSCH (e.g., HARQ process numbering)

- The UE does not expect to be scheduled with multiple PUSCHs by a single DCI, where every PUSCH is collided with downlink symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated.

Agreement:

For TDRA in a DCI that can schedule multiple PDSCHs (or PUSCHs),

- A row of the TDRA table can indicate PDSCHs (or PUSCHs) that are in consecutive or non-consecutive slots.

  - FFS: The maximum value of the gap between two consecutively scheduled PDSCHs or between two consecutively scheduled PUSCHs

  - FFS: The maximum value of the gap between the first scheduled PDSCH and the last scheduled PDSCH or between the first scheduled PUSCH and the last scheduled PUSCH

  - FFS: Details to introduce the gap between PDSCHs or between PUSCHs

Agreement:

For enhancements of generating type-1 HARQ-ACK codebook corresponding to DCI that can schedule multiple PDSCHs, the set of candidate PDSCH reception occasions corresponding to a UL slot with HARQ-ACK transmission is determined based on a set of DL slots and a set of SLIVs corresponding to each DL slot belonging to the set of DL slots.

- The set of DL slots includes all the unique DL slots that can be scheduled by any row index r of TDRA table in DCI indicating the UL slot as HARQ-ACK feedback timing.

- The set of SLIVs corresponding to a DL slot (belonging to the set of DL slots) at least include all the SLIVs that can be scheduled within the DL slot by any row index r of TDRA table in DCI indicating the UL slot as HARQ-ACK feedback timing.

  - FFS: details of further pruning of the set of SLIVs

  - FFS: impact if receiving more than one PDSCH in a slot is allowed, e.g., handling of overlapped SLIVs from different rows in the same and different DL slot

  - FFS impact of time domain bundling, if supported

Agreement:

- At least for 120 kHz SCS, for a DCI that can schedule multiple PUSCHs and is configured with the TDRA table containing at least one row with multiple SLIVs,

  - If CBG-based (re)transmission is configured, CBGTI field is not present when more than one PUSCHs are scheduled, but is present when a single PUSCH is scheduled, as in Rel-16.

- FFS:

  - For 480/960 kHz SCS, whether to apply the same behavior with 120 kHz SCS or not to support CBGTI field configuration in the DCI that can schedule multiple PUSCHs

  - For a DCI that can schedule multiple PDSCHs and is configured with the TDRA table containing at least one row with multiple SLIVs, whether/how to configure CBGTI/CBGFI fields

Agreement:

If Alt 1 (C-DAI/T-DAI is counted per DCI) is adopted for generating type-2 HARQ-ACK codebook corresponding to a DCI that can schedule multiple PDSCHs,

- At least two sub-codebooks are generated for a PUCCH cell group where

  - The first sub-codebook is for the following cases:

    - Any DCI that is not configured with CBG-based scheduling and is configured with TDRA table containing rows each with a single SLIV

    - Any DCI that is not configured with CBG-based scheduling and is configured with TDRA table containing at least one row with multiple SLIVs and schedules only a single PDSCH

  - The second sub-codebook is for the following case:

    - Any DCI that is configured with TDRA table containing at least one row with multiple SLIVs and schedules multiple PDSCHs

      - FFS: Methods (if needed) to align the size of HARQ-ACK feedback corresponding to different DCIs

      - FFS: Whether HARQ-ACK bits for 2 PDSCHs scheduled by this DCI can be included in the first sub-codebook in some cases

  - FFS: SPS PDSCH release, SCell dormancy indication without scheduled PDSCH

- FFS: 2 or 3 sub-codebooks if CBG is configured for a serving cell in the PUCCH cell group

- FFS: impact of time domain bundling, if supported, e.g., the number of sub-codebooks including single codebook if all A/N bits are bundled into a single bit per DCI

Agreement:

If Alt 2 (C-DAI/T-DAI is counted per PDSCH) is adopted for generating type-2 HARQ-ACK codebook corresponding to a DCI that can schedule multiple PDSCHs,

- PDSCH(s) scheduled by a single DCI is counted firstly, serving cell(s) in the same PUCCH cell group and same PDCCH monitoring occasion is counted secondly, and PDCCH monitoring occasion(s) is counted thirdly.

- The bit width of counter DAI field in fallback DCI (i.e., DCI formats 0_0 and 1_0) remains the same as in Rel-15 NR.

- Note: The DAI bit width and number of sub-codebooks shall ensure that at most 3 consecutive missed DCIs can be resolved, same as in Rel-15/16 NR

  - This shall not impose additional gNB’s scheduling restriction.

- In case where CBG retransmission is not configured for any serving cell in a same PUCCH cell group, the number of bits for each of counter DAI and total DAI in non-fallback DCI is extended (if needed) at least based on

  - The number of SLIVs associated with the row indexes in TDRA table

    - FFS: details

- FFS: the case with configuration of CBG retransmission

- FFS: the number of sub-codebooks

- FFS: for the UE indicating by type2-HARQ-ACK-Codebook support for more than one PDSCH reception on a serving cell that are scheduled from a same PDCCH monitoring occasion

- RAN1#106-e

Working assumption:

Scheduling multiple PDSCHs by single DL DCI applies to 120 kHz in addition to 480 and 960 kHz at least in FR2-2.

- FFS: Further limitations on maximum number of PDSCHs

Agreement:

Adopt Alt 1 (C-DAI/T-DAI is counted per DCI) for generating type-2 HARQ-ACK codebook corresponding to a DCI that can schedule multiple PDSCHs.

Agreement:

- The maximum number of PDSCHs/PUSCHs that can be scheduled with a single DCI in Rel-17 is 8 for SCS of 120, 480 and 960 kHz.

- FFS: Whether UE capability is introduced for restricting the maximum number of PDSCHs or PUSCHs that can be scheduled with a single DCI

Agreement:

If a scheduled PDSCH/PUSCH is dropped due to collision with UL/DL symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, HARQ process number increment is skipped for the PDSCH/PUSCH and applied only for valid PDSCH(s)/PUSCH(s).

- FFS: HARQ process number determination for the case where a scheduled PDSCH/PUSCH collides with a flexible symbol (indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated) if the UE is configured to monitor DCI format 2_0.

Agreement:

- For a DCI that can schedule multiple PUSCHs,

  - Priority indicator and open loop power control parameter set indication fields are applied to all of scheduled PUSCHs.

- For a DCI that can schedule multiple PDSCHs,

  - Priority indicator field is applied to all of scheduled PDSCHs.

Agreement:

For TDRA in a DCI that can schedule multiple PDSCHs (or PUSCHs),

- A row of the TDRA table can indicate PDSCHs (or PUSCHs) that are in consecutive or non-consecutive slots, by configuring {SLIV, mapping type, scheduling offset K0 (or K2)} for each PDSCH (or PUSCH) in the row of TDRA table.

- Note: Whether and how to reduce RRC overhead is left to RAN2.

Agreement:

For a DCI that can schedule multiple PDSCHs,

- Each of VRB-to-PRB mapping, PRB bundling size indicator, ZP-CSI-RS trigger, and rate matching indicator fields appears only once in the DCI.

- VRB-to-PRB mapping and PRB bundling size indicator fields are applied to all the PDSCHs scheduled by the DCI.

- For ZP-CSI-RS trigger field, the triggered aperiodic ZP CSI-RS is applied to all the slot(s) in which the PDSCH(s) scheduled by the DCI are contained.

- When receiving a PDSCH scheduled by the DCI, the REs corresponding to configured resources in rateMatchPatternGroup1 or rateMatchPatternGroup2 (according to indication of rate matching indicator field) are not available for the scheduled PDSCH.

Working assumption:

For NR FR2-2, two codeword transmission is supported, subject to UE capability.

- RRC parameter configures whether two codeword transmission is enabled or disabled.

  - FFS: Details on signaling of MCS/NDI/RV for the second TB in a DCI that can schedule multiple PDSCHs when two codeword transmission is enabled

  - FFS: Whether unified or separate parameter to enable/disable 2-TB for single and for multiple PDSCH scheduling

  - Strive to minimize the increase in the number of bits in the DCI needed to support this feature

Agreement:

- For single TRP operation, for 480/960 kHz SCS,

  - FFS: A UE does not expect to be scheduled with more than one PDSCH in a slot, by a single DCI or multiple DCIs.

  - FFS: A UE does not expect to be scheduled with more than one PUSCH in a slot, by a single DCI or multiple DCIs.

- For single TRP operation, for 120 kHz SCS (same as current specification for FR2-1 for PUSCH),

  - Subject to UE capability, a UE can be scheduled with more than one PDSCH in a slot, by a single DCI or multiple DCIs.

  - Subject to UE capability, a UE can be scheduled with more than one PUSCH in a slot, by a single DCI or multiple DCIs.

- FFS for multi-TRP operation

- Note: The optimization of HARQ codebook size for Type 1 or Type 2 codebook design is considered as a low priority in Rel-17 (this does not preclude HARQ ACK bundling in time domain).

- The agreement made in RAN1#105-e is revised as follows.

Agreement: (RAN1#105-e) For enhancements of generating type-1 HARQ-ACK codebook corresponding to DCI that can schedule multiple PDSCHs, the set of candidate PDSCH reception occasions corresponding to a UL slot with HARQ-ACK transmission is determined based on a set of DL slots and a set of SLIVs corresponding to each DL slot belonging to the set of DL slots. - The set of DL slots contains all the unique DL slots determined by considering all combinations of the configured K1 values and the configured rows of the TDRA table. - The set of SLIVs corresponding to a DL slot (belonging to the set of DL slots) contains all the SLIVs for that slot determined by considering all combinations of the configured K1 values and the configured rows of the TDRA table. - The Rel-16 procedure is reused for determining the candidate PDSCH reception occasions for the set of SLIVs corresponding to each DL slot belonging to the set of DL slots - Note: The Rel-16 procedure already handles pruning of multiple SLIVs corresponding to a DL slot, for both UEs that are and are not capable of receiving multiple PDSCHs per slot - FFS impact of time domain bundling, if supported

Agreement:

Consider the following options to construct type-2 HARQ-ACK codebook when CBG operation is configured, and down-select to one of the following options in RAN1#106bis-e.

- Option 1: HARQ-ACK bits corresponding to CBG-based PDSCH reception and multi-PDSCH reception are merged into the same sub-codebook.

- Option 2: HARQ-ACK bits corresponding to CBG-based PDSCH reception and HARQ-ACK bits corresponding to multi-PDSCH reception are contained in separate sub-codebooks.

- Option 3: UE does not expect to be configured with both of CBG operation and multi-PDSCH scheduling in the same PUCCH cell group.

- Note: Multi-PDSCH reception refers to the case where multiple PDSCHs are scheduled by a DCI that is configured with TDRA table containing at least one row with multiple SLIVs.

Agreement:

For NR FR2-2 at least for 480/960 kHz SCS, support 32 as the maximum number of HARQ processes for DL and UL, subject to UE capability.

- Note: Up to 32 maximal supported HARQ process number is already agreed in Rel-17 NTN WI.

- Working assumption: The same solution to support up to 32 HARQ process number in Rel-17 NTN WI is reused for NR FR2-2.

- RAN1#106bis-e

Agreement:

Confirm the working assumption from RAN1#106-e with the following modification.

Working assumption: (RAN1#106-e)

Scheduling multiple PDSCHs by single DL DCI applies to 120 kHz in addition to 480 and 960 kHz at least in FR2-2.

- FFS: Further limitations on maximum number of PDSCHs

- Note: Further limitations (in addition to what was agreed earlier) on the maximum number of PDSCHs or PUSCHs can be separately discussed for all SCSs.

Working assumption:

UE does not expect to be configured with both of CBG operation and multi-PDSCH scheduling in the same PUCCH cell group with a Type 2 codebook.

- If time bundling operation is supported, this working assumption can be revisited

Agreement:

For a PDSCH that is scheduled by multi-PDSCH scheduling DCI and is skipped due to collision with semi-static UL symbol(s),

- For Type-1 HARQ-ACK codebook generation, the PDSCH is not considered and the HARQ-ACK bit corresponding to the PDSCH is not reported by UE.

  - Note: Rel-16 procedure can be reused to handle this case.

- For Type-2 HARQ-ACK codebook generation, UE reports NACK for the PDSCH.

  - FFS on HARQ-ACK bit ordering

- Note: Codebook generation in case time domain bundling is enabled can be separately discussed if time domain bundling is supported.

Agreement:

For generating type-2 HARQ-ACK codebook corresponding to a DCI that can schedule multiple PDSCHs,

- HARQ-ACK bit corresponding to SPS PDSCH release or SCell dormancy indication without scheduled PDSCH, belongs to the first sub-codebook (which is defined in the previous agreement made in RAN1#105-e)

Agreement:

For two multi-PDSCH (or two multi-PUSCH) scheduling DCIs, UE does not expect any of the scheduled PDSCHs (or PUSCHs) and the scheduling DCI to lead to out-of-order scheduling.

- FFS: whether to allow OOO scheduling for the following two cases:

  - for the case of one multi-PDSCH (or multi-PUSCH) scheduling DCI and one single-PDSCH (or single-PUSCH) scheduling DCI, where multi-PDSCH (or multi-PUSCH) scheduling DCI schedules more than one PDSCH (or PUSCH)

  - for the case where two multi-PDSCH (or multi-PUSCH) scheduling DCIs end in the same symbol but two multi-PDSCH (or multi-PUSCH) scheduling DCIs have overlapping spans, where the span is defined from the beginning of the first scheduled SLIV till the end of the last scheduled SLIV

- Note: The above FFS aspect applies only to multi-PDSCH and multi-PUSCH scheduling with single DCI

Agreement:

For multiple PDSCHs (or PUSCHs) scheduled by a single DCI,

- Rel-15/16 behavior that is described in TS 38.213 Clauses 11 and 11.1 for a PDSCH (or PUSCH) indicated by DCI also applies for multiple PDSCHs (or PUSCHs) schedule by a single DCI.

- If one of multiple PDSCHs (or PUSCHs) scheduled by the DCI collides with a flexible symbol (indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated),

  - If that PUSCH is collided with SSB symbols indicated by ssb-PositionsInBurst [or symbol(s) indicated by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set], the HARQ process number increment is skipped for the PUSCH.

  - Otherwise, the HARQ process number increment is not skipped for that PDSCH (or PUSCH).

Conclusion:

For a DCI that can scheduled multiple PDSCHs (or PUSCHs), HARQ process number indicated in the DCI is applied to the first valid PDSCH (or PUSCH).

- Note: This is the consequence of previous agreements.

Agreement:

For single TRP operation, for 480/960 kHz SCS,

- A UE does not expect to be scheduled with more than one unicast PDSCH in a slot, by a single DCI or multiple DCIs.

- A UE does not expect to be scheduled with more than one PUSCH in a slot, by a single DCI or multiple DCIs.

Agreement:

For a DCI that can schedule multiple PDSCHs, and if RRC parameter configures that two codeword transmission is enabled,

- MCS for the 2nd TB: This appears only once in the DCI and applies commonly to the 2nd TB of each PDSCH

- NDI for the 2nd TB: This is signaled per PDSCH and applies to the 2nd TB of each PDSCH

- RV for the 2nd TB: This is signaled per PDSCH, with 2 bits if only a single PDSCH is scheduled or 1 bit for each PDSCH otherwise and applies to the 2nd TB of each PDSCH

- FFS: the maximum number of PDSCHs when 2 TB is enabled or when 2 TB is scheduled

- RAN1#107-e

Agreement

- For multi-PDSCH or multi-PUSCH scheduling DCI, FDRA enhancement is deprioritized in Rel-17.

Agreement

- For multi-TRP operation, for 480/960 kHz SCS,

  - A UE does not expect to be scheduled with more than one unicast PDSCH in a slot, by a single DCI or multiple DCIs, from the same TRP.

  - A UE does not expect to be scheduled with more than one PUSCH in a slot, by a single DCI or multiple DCIs, from the same TRP.

  - Note: This does not preclude a UE being scheduled with two PDSCHs (or two PUSCHs) in the same slot from two different TRPs for multi-DCI based multi-TRP mechanism.

Agreement

- For a DCI that can schedule multiple PDSCHs, CBGTI and CBGFI fields are not present in the DCI.

- UE does not expect to be configured with both of CBG operation and multi-PDSCH scheduling in the serving cell with a Type 1 codebook.

- Confirm the working assumption from RAN1#106bis-e with the following modification.

Working assumption: (RAN1#106bis-e)

- UE does not expect to be configured with both of CBG operation and multi-PDSCH scheduling in the same PUCCH cell group with a Type 2 codebook.

  - If time bundling operation is supported, this working assumption can be revisited

Agreement

For 480/960 kHz SCS, CBG-based HARQ cannot be configured for uplink and downlink.

Agreement

- The maximum number of PDSCHs that can be scheduled with a single DCI in Rel-17 is also 8 when 2 TB is enabled or when 2 TB is scheduled, for SCS of 120, 480 and 960 kHz.

  - Note: This is to handle FFS (the maximum number of PDSCHs when 2 TB is enabled or when 2 TB is scheduled) in previous agreement in RAN1#106bis-e.

Agreement

For multi-PUSCH scheduling DCI in Rel-17, support intra-slot frequency hopping which is applicable to each of multiple PUSCH transmissions scheduled by the DCI, and do not support inter-slot frequency hopping.

Agreement

For multi-PDSCH scheduling with a single DCI

- Introduce a new RRC parameter, e.g., enableTimeDomainHARQ-Bundling, to enable time domain bundling operation for type-1 HARQ-ACK codebook per serving cell.

  - If the RRC parameter enables time domain bundling operation,

    - To determine the set of candidate PDSCH reception occasions,

      - A row index is removed if at least one symbol of every PDSCH associated with the row index is configured as semi-static UL. (NOTE: This is similar to the case of slot aggregated PDSCH in Rel-16)

      - Pruning procedure in Rel-16 is performed based on the last configured SLIV of each row index.

    - Logical AND operation is applied across all valid PDSCHs associated with a determined candidate PDSCH reception occasion, at least for 1-TB case.

    - FFS: UE does not expect the last scheduled SLIV overlaps with a semi-static UL symbol when parameter enableTimeDomainHARQ-Bundling is configured

Agreement

- If a UE is configured with a TDRA table in which one or more rows contain multiple SLIVs for PDSCH for DCI format 1_1, the UE does not expect to be configured with repetitionNumber for the TDRA table, and if pdsch-AggregationFactor is configued in PDSCH-config, it does not apply to DCI format 1_1.

  - Note: repetitionNumber cannot be configured with pdsch-TimeDomainAllocationListDCI-1-2 as in Rel-16.

  - Note: Under agenda item 8.2.4, in RAN1#106-bis, it was already agreed that within the TDRA table for multi-PDSCH scheduling, the UE does not expect to be configured with the higher layer parameter repetitionNumber.

  - Note: These does not preclude pdsch-AggregationFactor can be configured and applies to DCI format 1_2

- If a UE is configured with a TDRA table in which one or more rows contain multiple SLIVs for PUSCH for DCI format 0_1, the UE does not expect to be configured with numberOfRepetitions for the TDRA table, and if pusch-AggregationFactor is configued in PUSCH-config, it does not apply to DCI format 0_1.

  - Note: These does not preclude numberOfRepetitions is configured for TDRA table corresponding to DCI format 0_2

  - Note: These does not preclude pusch-AggregationFactor can be configured and applies to DCI format 0_2

Agreement

- For type-2 HARQ-ACK codebook generation, HARQ-ACK bit ordering is based on configured SLIV position in the indicated TDRA row index, regardless of the validity of each scheduled PDSCH.

Agreement

- There is no consensus in RAN1 to support that HARQ-ACK information corresponding to different PDSCHs scheduled by a single DCI is carried over multiple PUCCHs in Rel-17.

Agreement

For multi-PDSCH scheduling with a single DCI

- Introduce a new RRC parameter, e.g., numberOfHARQ-BundlingGroups, to configure the number of HARQ bundling groups with value range {1, 2, 4} for type-2 HARQ-ACK codebook per serving cell.

  - If the RRC parameter is not configured for a serving cell, time domain bundling for type-2 HARQ-ACK codebook is not enabled for the serving cell.

  - The maximum number of PDSCHs allocated to each bundling group is ceil(NPDSCH,MAX/NHBG) where NHBG is the number of bundling groups configured by numberOfHARQ-BundlingGroups for a serving cell and NPDSCH,MAX is the maximum configured number of PDSCHs for the serving cell.

  - The PDSCHs corresponding to [configured or valid] SLIVs in a TDRA row index indicated by multi-PDSCH scheduling DCI are allocated to the bundling groups, e.g., if NHBG =4, NPDSCH,MAX =8, and 5 PDSCHs are scheduled, then 2/1/1/1 PDSCHs are assigned to each group, by reusing CBG grouping method.

    - For a group that is empty or is filled with only invalid PDSCH(s), HARQ-ACK bits for the bundling group is set to NACK (same principle as when no time bundling configured)

    - Logical AND operation is applied to across all valid PDSCHs within the same bundling group to generate 1 HARQ-ACK bit per group, at least for 1-TB case

  - If the number of HARQ bundling groups is configured as 1 for a serving cell, HARQ-ACK bits corresponding to any DCI for the serving cell belong to the first sub-codebook.

  - At least for 1-TB case, if the number of HARQ bundling groups is configured as larger than 1 for a serving cell, HARQ-ACK bits corresponding to multi-PDSCH scheduling case (which implies a multi-PDSCH DCI schedules more than one PDSCH) for the serving cell belong to the second sub-codebook,

    - Where the number of HARQ-ACK bits corresponding to a multi-PDSCH DCI is determined based on the maximum of Q value across all serving cells within the same PUCCH cell group, and Q=maximum configured number of PDSCHs for a cell without numberOfHARQ-BundlingGroups configured or Q=number of configured HARQ bundling groups for a cell with numberOfHARQ-BundlingGroups configured

- RAN1#107bis-e

Agreement

- In NR FR2-2, a UE supporting 32 maximum number of HARQ processes for 480/960 kHz SCS for DL (or for UL) shall support 32 as the maximum number of HARQ processes for 120 kHz SCS for DL (or UL), subject to UE capability.

Agreement

- If the higher layer parameter maxNrofCodeWordsScheduledByDCI indicates that two codeword transmission is enabled and more than one PDSCH is scheduled by a multi-PDSCH scheduling DCI,

  - Either the first or the second transport block of all scheduled PDSCHs is disabled by the DCI if IMCS = 26 and if RV bits are set to ‘1’ for the corresponding transport block of all scheduled PDSCHs (i.e. irrespective of whether this is a valid PDSCH).

Conclusion

For multi-PDSCH or multi-PUSCH scheduling DCI, the following maximum value of a gap is not specified in Rel-17 and up to gNB scheduler.

- The maximum value of the gap between two consecutively scheduled PDSCHs or between two consecutively scheduled PUSCHs

- The maximum value of the gap between the first scheduled PDSCH and the last scheduled PDSCH or between the first scheduled PUSCH and the last scheduled PUSCH

Conclusion

HARQ process number configured for SPS PDSCH (or CG PUSCH) can be allocated to a PDSCH (or PUSCH) of multi-PDSCH (or multi-PUSCH) scheduling, as long as the timeline condition defined in Rel-15/16 is met.

- Note: It is up to gNB implementation whether/how to avoid UL data retransmission due to HARQ process index collision and flushed HARQ transmit buffer.

Agreement

- Update the previous agreement made in RAN1#107-e, as follows:

Agreement (RAN1#107-e)

For multi-PDSCH scheduling with a single DCI

- Introduce a new RRC parameter, e.g., enableTimeDomainHARQ-Bundling, to enable time domain bundling operation for type-1 HARQ-ACK codebook per serving cell.

  - If the RRC parameter enables time domain bundling operation,

    - To determine the set of candidate PDSCH reception occasions,

      - A row index is removed if at least one symbol of every PDSCH associated with the row index is configured as semi-static UL. (NOTE: This is similar to the case of slot aggregated PDSCH in Rel-16)

      - Pruning procedure in Rel-16 is performed based on the last configured SLIV of each row index.

    - Logical AND operation is applied across all valid PDSCHs associated with a determined candidate PDSCH reception occasion, at least for 1-TB case.

    - FFS: UE does not expect the last scheduled SLIV overlaps with a semi-static UL symbol when parameter enableTimeDomainHARQ-Bundling is configured

Agreement

For multi-PDSCH scheduling with a single DCI and for type-2 HARQ-ACK codebook generation,

- Time domain bundling and spatial bundling can be independently configured.

Conclusion

- UE does not expect any of the scheduled PDSCHs (or PUSCHs) and the scheduling DCIs to lead to out-of-order scheduling, also for the case of one multi-PDSCH (or multi-PUSCH) scheduling DCI and one single-PDSCH (or single-PUSCH) scheduling DCI, where multi-PDSCH (or multi-PUSCH) scheduling DCI schedules more than one PDSCH (or PUSCH).

  - This may not have specification impact.

- Note: It is separately discussed whether the scheduled PDSCHs (or PUSCHs or SLIV) is based on configured SLIV or valid SLIV.

Conclusion

UE does not expect any of the received PDSCHs (including SPS PDSCH) and the resource for the HARQ-ACK transmission to lead to out-of-order scheduling, for any scheduling DCIs (including multi-PDSCH scheduling DCI).

Agreement

For a DCI that can schedule multiple PDSCHs or multiple PUSCHs,

- It is clarified that NDI/RV fields in the following previous agreements correspond to scheduled PDSCHs indicated by the TDRA information field.

Agreement: (RAN1#104-bis) For a DCI that can schedule multiple PDSCHs, - NDI for the 1st TB: This is signaled per PDSCH and applies to the first TB of each PDSCH - RV for the 1st TB: This is signaled per PDSCH, with 2 bits if only a single PDSCH is scheduled or 1 bit for each PDSCH otherwise and applies to the first TB of each PDSCH Agreement: (RAN1#106bis-e) For a DCI that can schedule multiple PDSCHs, and if RRC parameter configures that two codeword transmission is enabled, - NDI for the 2nd TB: This is signalled per PDSCH and applies to the 2nd TB of each PDSCH - RV for the 2nd TB: This is signalled per PDSCH, with 2 bits if only a single PDSCH is scheduled or 1 bit for each PDSCH otherwise and applies to the 2nd TB of each PDSCH

- Above clarification also applies to the DCI scheduling multiple PUSCHs, i.e., NDI/RV fields in the DCI correspond to scheduled PUSCHs indicated by the TDRA information field.

- The following example change to 38.214 Sections 5.1.3 and 6.1.4 can be recommended to the editor of 38.214 to use at the editor’s discretion

---------------------------Start of TP for TS 38.214 Clause 5.1.3 -----------------------------------------------

5.1.3	Modulation order, target code rate, redundancy version and transport block size determination

================ Unchanged Text Omitted =======================

When the UE is scheduled with multiple PDSCHs by a DCI, as described in clause 5.1.2.1, the bits of rv field and NDI field, respectively, in the DCI are one-to-one mapped to the scheduled PDSCH(s) indicated by the TDRA information field with the corresponding transport block(s) in the scheduled order, where the LSB bits of the rv field and NDI field, respectively, correspond to the last scheduled PDSCH indicated by the TDRA information field.

---------------------------------------------- End of TP --------------------------------------------------------------

---------------------------Start of TP for TS 38.214 Clause 6.1.4 -----------------------------------------------

6.1.4	Modulation order, redundancy version and transport block size determination

================ Unchanged Text Omitted =======================

When the UE is scheduled with multiple PUSCHs by a DCI, as described in clause 6.1.2.1, the bits of rv field and NDI field, respectively, in the DCI are one to one mapped to the scheduled PUSCH(s) indicated by the TDRA information field with the corresponding transport block(s) in the scheduled order where the LSB bits of the rv field and NDI field, respectively, correspond to the last scheduled PUSCH indicated by the TDRA information field.

---------------------------------------------- End of TP --------------------------------------------------------------

Conclusion

It is clarified that the absence or presence of CBGTI field in the following previous agreement is determined based on scheduled PUSCHs indicated by the TDRA information field (i.e. irrespective of whether this is a valid PUSCH).

Agreement: (RAN1#105-e) - At least for 120 kHz SCS, for a DCI that can schedule multiple PUSCHs and is configured with the TDRA table containing at least one row with multiple SLIVs, - If CBG-based (re)transmission is configured, CBGTI field is not present when more than one PUSCHs are scheduled, but is present when a single PUSCH is scheduled, as in Rel-16.

Agreement

A UE validates, for scheduling activation or scheduling release, a DL SPS assignment PDCCH or a configured UL grant Type 2 PDCCH if the PDCCH indicates a TDRA row index including only one SLIV.

Agreement

For type-1 HARQ-ACK codebook, if OACK+OSR+OCSI≤11, the UE determines a number of HARQ-ACK information bits nHARQ-ACK for obtaining a transmission power for a PUCCH, as follows.

- For a serving cell c configured with enableTimeDomainHARQ-Bundling, and for a DCI format indicating a TDRA row that includes more than one SLIV entry on the serving cell c, the UE considers a PDSCH (which carries one or two transport blocks enabled by the DCI format irrespective of whether the PDSCH is valid or not) only associated with the last SLIV as received, to determine Nm,creceived.

- RAN1#108-e

Agreement

- The case where two multi-PDSCH (or multi-PUSCH) scheduling DCIs end in the same symbol but two multi-PDSCH (or multi-PUSCH) schedulings have overlapping spans, where the span is defined from the beginning of the first scheduled SLIV till the end of the last scheduled SLIV, is considered as out-of-order scheduling and is not expected by UE.

  - This applies also when one of two DCIs is single-PDSCH (or single-PUSCH) scheduling DCI, including the case that one DCI schedules multi-slot PDSCH (or PUSCH repetition type A or B).

  - Note: This doesn’t apply when each of two DCIs schedules multi-slot PDSCH (or PUSCH repetition type A or B) as in Rel-15/Rel-16

  - Note: This doesn’t apply when each of the two DCIs schedules single PDSCH (or single PUSCH) as in Rel-15/Rel-16

- Note: It is separately discussed whether the scheduled SLIV is based on configured SLIV or valid SLIV.

Working assumption

The TP#1 below for TS 38.213 section 9.1.3.1 is endorsed.

- TP#1 (Construction of bundling group based on “configured” SLIVs)

If a UE is provided numberOfHARQ-BundlingGroups and is not provided harq-ACK-SpatialBundlingPUCCH for a serving cell c, the UE generates HARQ-ACK information over transport block groups (TBGs) for PDSCH receptions where, for a maximum number of NPDSCHmax PDSCH receptions scheduled by a DCI format on the serving cell, a maximum number of TBGs NHARQ-ACK,cTBG,max is provided by numberOfHARQ-BundlingGroups. If the UE detects a DCI format scheduling NPDSCH,c PDSCH receptions on the serving cell c, the UE generates NHARQ-ACK,cTBG,max HARQ-ACK information bits for first TBs and, if applicable, generates NHARQ-ACK,cTBG,max HARQ-ACK information bits for second TBs in the NPDSCH,c PDSCH receptions as described in clause 9.1.1 by setting NHARQ-ACKCBG/TB,max=NHARQ-ACK,cTBG,max and C=NPDSCH,c. For a TBG with at least one PDSCH not overlapping with an UL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, if provided, the UE assumes that a PDSCH in the TBG is correctly received if the PDSCH overlaps with an UL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated. For a TBG that includes only PDSCH(s) overlapping with UL symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, if provided, NACK is generated for the TBG.

Agreement

- When re-transmission of DL SPS is indicated by DCI format 1_1, the PDCCH indicates a TDRA row index including only one SLIV.

- When re-transmission of UL CG is indicated by DCI format 0_1, the PDCCH indicates a TDRA row index including only one SLIV.

Text Proposal TP#B (for 38.213, Clause 9.1.2.1) in section 4.2 of R1-2202679 is endorsed.

Text Proposal TP#C (for 38.214, Clause 6.1) in section 4.3 of R1-2202679 is endorsed.

Text Proposal TP#E (for 38.213, Clause 10.3) in section 4.5 of R1-2202679 is endorsed.

Text Proposal TP#H (for 38.214, Clauses 5.1 and 6.1) in section 4.8 of R1-2202679 is endorsed.

Conclusion

For a DCI that can schedule multiple PUSCHs, it is clarified that the following M is counted based on scheduled PUSCHs indicated by the TDRA information field.

- CSI-request: When the DCI schedules M PUSCHs, the PUSCH that carries the aperiodic CSI feedback is M-th scheduled PUSCH for M <= 2, or (M-1)-th scheduled PUSCH for M > 2.

Note: Aperiodic CSI feedback will be dropped if M-th scheduled PUSCH for M <= 2 (or (M-1)-th scheduled PUSCH for M > 2) is collided with semi-static DL or SSB symbols.

Agreement

TP#F1 below for TS 38.214 Clause 6.1.2.1 is endorsed

-------------------------------------------Start of TP#F1 for TS 38.214 Clause 6.1.2.1-----------------------------------------------

6.1.2.1	Resource allocation in time domain

If a UE is configured with pusch-TimeDomainAllocationListForMultiPUSCH-r17 in which one or more rows contain multiple SLIVs for PUSCH on a UL BWP of a serving cell, the UE does not apply pusch-AggregationFactor, if configured, to DCI format 0_1 on the UL BWP of the serving cell and the UE does not expect to be configured with numberOfRepetitions in pusch-TimeDomainAllocationListForMultiPUSCH-r17.

------------------------------------------------------------End of TP#F1----------------------------------------------------------------

Agreement

TP#J1 below for TS 38.214 Clause 5.1.3.2 is endorsed

-------------------------------------------Start of TP#J1 for TS 38.214 Clause 5.1.3.2 -----------------------------------------------

5.1.3.2	Transport block size determination

In case the higher layer parameter maxNrofCodeWordsScheduledByDCI indicates that two codeword transmission is enabled, then one of the two transport blocks is disabled by DCI format 1_1 if IMCS = 26 and if rvid = 1 for the corresponding transport block. When the UE is configured with higher layer parameter [pdsch-TimeDomainAllocationListForMultiPDSCH-r17], either the first or the second transport block of all scheduled PDSCHs is disabled by the DCI format 1_1 if IMCS = 26 and if rvid = 2 for the corresponding transport block of all scheduled PDSCHs. If both transport blocks are enabled, transport block 1 and 2 are mapped to codeword 0 and 1 respectively. If only one transport block is enabled, then the enabled transport block is always mapped to the first codeword.

------------------------------------------------------------End of TP#J1----------------------------------------------------------------

Agreement

For multi-PDSCH scheduling via a single DCI with 'tdmSchemeA' for single DCI based multi-TRP mechanism,

- If at least one of the repetitions of the PDSCH collides with semi-static UL symbols, the corresponding PDSCH (i.e., both repetitions) is considered as invalid.

  - Note: No specification impact on Type-1 HARQ-ACK codebook construction is expected, as a consequence of this agreement.

  - Note: This is not applied for the case when the multi-PDSCH DCI schedules only a single PDSCH.

Agreement

When a DCI format indicates TCI state update without scheduling PDSCH reception, the PDCCH indicates a TDRA row index including only one SLIV.

Agreement

For generating type-2 HARQ-ACK codebook corresponding to a DCI that can schedule multiple PDSCHs, if OACK+OSR+OCSI≤11, the UE determines a number of HARQ-ACK information bits nHARQ-ACK for obtaining a transmission power for a PUCCH, considering at least the followings.

- For a serving cell c configured with numberOfHARQ-BundlingGroups with NHARQ-ACKTBG,max=1, Nm,creceived (in nHARQ-ACK,TB formula) corresponding to the DCI in PDCCH monitoring occasion m is given by 2 if harq-ACK-SpatialBundlingPUCCH is not provided and the DCI schedules two codewords, or given by 1 otherwise.

- If the UE is configured with numberOfHARQ-BundlingGroups with NHARQ-ACKTBG,max>1 for a serving cell or not configured with numberOfHARQ-BundlingGroups but configured with PDSCH-TimeDomainResourceAllocationListForMultiPDSCH for a serving cell, nHARQ-ACK=nHARQ-ACK,TB+nHARQ-ACK,multi.

  - For a serving cell c configured with numberOfHARQ-BundlingGroups with NHARQ-ACKTBG,max>1, Nm,creceived (in nHARQ-ACK,multi formula) corresponding to the DCI in PDCCH monitoring occasion m is given by 2*X (X= the number of TBGs, each including at least one valid PDSCH which are constructed based on the PDSCH(s) scheduled by the DCI) if harq-ACK-SpatialBundlingPUCCH is not provided and the DCI schedules two codewords, or given by X otherwise.

  - For a serving cell c not configured with numberOfHARQ-BundlingGroups but configured with PDSCH-TimeDomainResourceAllocationListForMultiPDSCH, Nm,creceived (in nHARQ-ACK,multi formula) corresponding to the DCI in PDCCH monitoring occasion m is given by 2*Y (Y= the number of valid PDSCH receptions scheduled by the DCI) if harq-ACK-SpatialBundlingPUCCH is not provided and the DCI schedules two codewords, or given by Y otherwise.

  - Note: “valid PDSCH” implies a PDSCH not overlapping with an UL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, if provided