3GPP TSG RAN WG1 #109-e			R1-2205195

e-Meeting, May 9th –  20th, 2022

Agenda item:	8.1.3

Source: 	Moderator (ZTE)

Title: 	Moderator Summary #1 for Maintenance on Rel-17 SRS

Document for:	Discussion and Decision

- Introduction

The following email thread is assigned for email discussion on maintenance of Rel-17 SRS, please provide your comments in corresponding sections below.

[109-e-R17-MIMO-07] Maintenance on beam management (description of issues in R1-220XXXX) – Yang (ZTE)

- Issues #1-5, #2-1, #2-3, #2-4, #3-6 by May 18

- Editorial Issues #1-1, #1-2, #2-5, #3-2, #3-3, #3-4 by May 11

- Summary of High priority (H) issues

Issue#1-5: Available slot for flexible SRS triggering

| Issue | Companies’ views |
| --- | --- |
| TP#1-5: available slot for flexible SRS triggering (R1-2203423) <Start of TP for TS38.214> 6.2.1 UE sounding procedure <Unrelated part omitted> - t is configured via higher layer parameter availableSlotOffset with up to four different values for each triggered SRS resources set and is based on the subcarrier spacing of the triggered SRS transmission. When one or more SRS resource sets across all configured BWPs in a component carrier are configured, and at least one resource set is configured with availableSlotOffset parameter of more than one values, the indicated value of availableSlotOffset is indicated by SOI field in DCI scheduling PUSCH/PDSCH and DCI 0_1/0_2 without data and without CSI request described in [5, TS 38.212]. The UE shall apply indicated value of availableSlotOffset set specifically for those sets with configured availableSlotOffset parameter. When one or more SRS resource sets across all configured BWPs in a component carrier are configured and at least one resource set is configured with availableSlotOffset parameter, and the availableSlotOffset parameter for each SRS resource set has only one value, the UE shall apply the configured value of availableSlotOffset specifically for those sets with configured availableSlotOffset parameter. For SRS resource set configured with availableSlotOffset parameter, each of resource set is configured with K same number of values of availableSlotOffset parameter. For SRS resource set configured without availableSlotOffset parameter, t = 0 is applied for each of resource set. <Unrelated part omitted> <End of TP for TS38.214> FL Note: Basically, the motivation for restricting the same number of values of availableSlotOffset parameter across aperiodic SRS resource sets should be clarified first. If so, this TP may could be taken as way forward. | TP#1-5: - Support/Fine: LGE - Not support: |

Additional inputs of issue#1-5

| Company | Input |
| --- | --- |
| Mod V0 | - Check and update your view in the table above - Share more inputs here if needed |
| Samsung | Since there is no agreement on using same number of values of availableSlotOffset parameter, we think the above TP is not aligned with the previous agreement. |
| LGE | Support the TP since we already have the following agreement. We think there is no need to support different number of configured slot offset for each SRS resource sets, because some codepoint(s) of SRS triggering field can be ambiguous for the SRS resource set with less configured slot offset. Agreement Bit width of SOI depends on the maximum number of “t” values configured for any of the aperiodic SRS resource sets (FFS: across all CCs or across a CC/BWP) - The SOI field is 0 bit if the maximum number of ‘t’ values is one - If at least one resource set has “t” configured - For the resource sets with “t” value configured, each of them is configured with K values of “t”, where 1<=K<=4 - t=0 applies for the resource set(s) without “t” configured in RRC - If none of the resource sets is configured with “t” values, follow Rel-15 approach to determine slot offset |
| Apple | We are fine with either way |
| DOCOMO | We agree with Samsung. For the agreement raised by LGE above, we think it doesn’t necessarily mean K is always common across SRS-ResourceSets. |
| OPPO | In our understanding the original intention of the agreement cited by LGE is to ensure the same number of “t” values. That is why we use the single notation “K” for all resource sets. Thus, we support the TP. |
| ZTE | Don’t support this TP. In addition according to following agreement and following description in 38.212 and 38.214, we propose following TP 1-6 Agreement Bit width of SOI depends on the maximum number of “t” values configured for any of the aperiodic SRS resource sets (FFS: across all CCs or across a CC/BWP) - The SOI field is 0 bit if the maximum number of ‘t’ values is one - If at least one resource set has “t” configured - For the resource sets with “t” value configured, each of them is configured with K values of “t”, where 1<=K<=4 - t=0 applies for the resource set(s) without “t” configured in RRC - If none of the resource sets is configured with “t” values, follow Rel-15 approach to determine slot offset (38.212 section 7.3.1.1.2) - SRS offset indicator – 0, 1 or 2 bits. otherwise, log2(K) bits are used to indicate available slot offset according to Table 7.3.1.1.2-37 and Clause 6.2.1 of [6, TS 38.214], where K is the maximum number of entries of AvailableSlotOffset configured for all aperiodic SRS resource set(s) in the scheduled cell;. 38.214- section 6.2.1 For an SRS-ResourceSet configured with higher layer parameter resourceType set to 'aperiodic', a list of zero up to four different available slot offset values from the reference slot n + k to the slot where the aperiodic SRS resource set is transmitted where n is the slot with triggering DCI and k is SlotOffset is defined by the higher layer parameter AvailableSlotOffset. TP 1-6 For SRS resource set configured with availableSlotOffset parameter, each of resource set is configured with up to four values of availableSlotOffset parameter |
| Huawei, HiSilicon | Support the TP. According to both the agreement attached by other companies and the email discussion progress, we think each of resource set should be configured with the same number (i.e., K) of values of availableSlotOffset parameter. Following is the clarification by Hao to Bingchao and Afshin on 12th Oct. 2021 during the 106b email discussion: Is the 1st sub-bullet of the 2nd bullet implying a same K for all the resource sets? [Hao] For the resource sets with explicit t value configured, it should be a same K for these sets. For the other sets, apply t=0. |
| Intel | Ok with the TP. Same view as LGE/OPPO/Huawei. As indicated by the agreement, the number of ‘t’ values should be the same. |
| CATT | Support the TP. It is our interpretation that the previous agreement means same number of availableSlotOffset is configured for all SRS resource sets configured with availableSlotOffset. |
| vivo | We are fine either way |
| QC | The current specification is clear and reflect RAN1 agreements. When available slot is configured for more than one sets, then all sets have K values of ‘t’. This TP is not needed. |
| Ericsson | We are fine with either way |
| Lenovo | Fine with the TP. |

Issue#2-1: Inter-set GP for antenna switching

| Issue | Companies’ views |
| --- | --- |
| Remaining issue of inter-set GP for antenna switching (R1-2203507, R1-2203857, R1-2204139, R1-2204337, R1-2204537, R1-2204765, R1-2204978) FL proposal 2-1.A: Down-select one of the Alts to handle the case when the interval between SRS resource sets is larger than Y: - Alt 1: UL/DL signals are allowed to be transmitted in the interval between SRS resource sets for antenna switching, i.e., no scheduling restriction - Alt 2: Y consecutive symbols in the interval is reserved for scheduling restriction. - Alt 2-1: the position of guard period for scheduling restriction is the last Y symbols of the interval. - Alt 2-2: the position of guard period for scheduling restriction is the first Y symbols of the interval. - Alt 2-3: the position of guard period for scheduling restriction is configured by signaling. FL proposal 2-1.B: Down-select one of the Alts to handle the case when the interval between SRS resource sets is larger than 2Y: - Alt 1: UL/DL signals are allowed to be transmitted in the interval between SRS resource sets for antenna switching, i.e., no scheduling restriction. - Alt 2: the first Y symbols and the last Y symbols in the interval is reserved for scheduling restriction. TP#2-1: inter-set Guard period (R1-2204337) <Start of TP for TS38.214> 6.2.1 UE sounding procedure <Unrelated part omitted> When the UE is configured with the higher layer parameter usage in SRS-ResourceSet set to 'antennaSwitching', and a guard period of Y symbols is configured between SRS transmissions associated with the same SRS-ResourceSet according to Clause 6.2.1.2, the UE shall use the same priority rules as defined above during the guard period as if SRS was configured. When the UE is configured with the higher layer parameter usage in SRS-ResourceSet set to 'antennaSwitching', and a guard period of Y symbols is configured between SRS resources associated with different SRS-ResourceSet according to Clause 6.2.1.2, and if the SRS transmission before and after the guard period of Y symbols are not dropped, the UE does not transmit any other signal above during the guard period as if SRS was configured. <Unrelated part omitted> <End of TP for TS38.214> FL Note: For clarification, FL proposals 2.1-A aims for the case when the interval between SRS resource sets is larger than Y, and FL proposals 2.1-B aims for the case when the interval between SRS resource sets is larger than 2Y. Besides, TP#2-1 aims to emphasize that Y-symbol inter-set GP is not available for any other signal, including PUCCH, as long as the corresponding SRS resources are actually transmitted. | Proposal 2-1.A: Alt 1: - Support/Fine: - Not support: Alt 2-1: - Support/Fine: LGE - Not support: Alt 2-2: - Support/Fine: LGE - Not support: Alt 2-3: - Support/Fine: LGE - Not support: Proposal 2-1.B: Alt 1: - Support/Fine: - Not support: Alt 2: - Support/Fine: LGE, QC - Not support: TP#2-1: - Support/Fine: - Not support: |

Additional inputs of issue#2-1

| Company | Input |
| --- | --- |
| Mod V0 | - Check and update your view in the table above - Share more inputs here if needed |
| Samsung | Support Alt 2 in FL proposal 2-1.B. We think that the DL/UL transmission in the interval between sets is possible only when there are Y symbols after the last SRS transmission of the first set and also Y symbols before the first SRS transmission of the second set (total 2Y symbols the interval between two sets), and the first Y symbols and the last Y symbols in the interval between sets is reserved for scheduling restriction. |
| LGE | Support Alt 2 in both FL proposal 2-1.A and B to preserve transient period of SRS antenna switching. |
| Apple | For 2-1.A and 2-1.B, we do not support any further clarification when configured gap is larger than Y, i.e., all gap symbols will be impacted. If we need discussion, it should be discussed in RAN4. TP#2-1 looks okay to us. |
| DOCOMO | For FL proposal 2-1.A, our best preference is Alt 1, but we would also be ok with Alt 2. Among Alt 2-1 to 2-3, we would be open to discuss. Alt 2-1 or 2-2 can avoid additional configuration impact in RRC in this later stage of the WI, while Alt 2-3 can achieve a bit more flexibility on scheduling. For FL proposal 2-1.B, we are not sure why the alternatives capture only “no scheduling restriction” or “2Y symbols reservation”. Why would the same solution in FL proposal 2-1.A not be considered here? We are not sure why different solution has to be considered because of the number of symbols between SRS transmissions which is anyway larger than Y. We support TP#2-1. We believe it resolves the inconsistency between 6.2.1 and 6.2.1.2 when Y-symbol exists between inter-set SRS resources. |
| OPPO | We are open to Alt2-1, Alt.2-2 of 2-1.A, and Alt.2 of 2-1.B |
| ZTE | For 2-1.A and 2-1.B, we support Alt 2. further we propose to compromise these Alts to following Alt 3． In addition, the two SRS resource sets should be triggered by same signaling and includes different antennas. In the case two SRS sets for antenna switching are triggered by two DCIs(or are triggered by different type of signaling, such as one is triggered by DCI and one is triggered by RRC/MAC-CE) and includes same antenna, the gap is unnecessary. Alt3: If the interval between two SRS resource sets for antenna switching and including different antennas is larger than Y symbols, and the antennas between the last SRS resource in the first of the two SRS resource sets and other UL channels/signals transmitted in first symbol of the interval are different, the first Y symbols of the interval should be reserved as guard period for scheduling restriction. If the interval between two SRS resource sets for antenna switching including different antennas is larger than Y symbols, and the antennas between the first SRS resource in the second of the two SRS resource sets and other UL channels/signals transmitted in the last symbol of the interval are different, the last Y symbols of the interval should be reserved as guard period for scheduling restriction. |
| Huawei, HiSilicon | Support Alt 1. Not support TP#2-1. Since similar logic is applied for R15 1T4R and works well, we don’t think extra limitation should be added in maintenance phase. In terms of TP#2-1, it should be treated after we have a consensus on FL proposal 2-1.A/B. |
| Intel | Regarding FL proposal 2-1.A, fine with Alt 2-1 or Alt 2-2. The gap symbol position should be determined since the gap symbol is used for collision handling. Regarding FL proposal 2-1.B, it is not necessary. FL proposal 2-1.A is sufficient. For TP#2-1, we don’t think it’s needed. |
| CATT | We are fine with either Alt 2-1 or Alt 2-2 for 2-1.A. In TS 38.214, it is specified that “the UE shall use the same priority rules as defined above during the guard period as if SRS was configured”. If the position of GP is not predefined, which symbol(s) with the same priority as SRS resource is not clear. We think 2-1.B is not needed since it is a special case of 2-1.A. |
| vivo | On FL proposal 2-1.A, fine with Alt 2-1 or Alt 2-2. FL proposal 2-1.B may not be necessary |
| QC | The cleanest solution is Alt2 of proposal 2-1B. |
| Lenovo | On FL proposal 2-1.A, support Alt 1. On FL proposal 2-1.B, support Alt 1. |

Issue#2-3&2-4: Antenna switching for 1T2R/2T4R

| Issue | Companies’ views |
| --- | --- |
| TP#2-3: antenna switching for 1T2R/2T4R (R1-2203507) <Start of TP for TS38.214> 6.2.1.2 UE sounding procedure for DL CSI acquisition <Unrelated part omitted> - For 1T2R, if the UE is indicating a capability for [maximum 2 semi-persistent and maximum 1 periodic SRS resource sets] and/or a capability for [extension of aperiodic antenna switching SRS configuration]: - up to 3 SRS resource sets configured across different value combinations for the higher layer parameter resourceType in SRS-ResourceSet if the UE is indicating a capability for [maximum 2 semi-persistent and maximum 1 periodic SRS resource sets] or a capability for [extension of aperiodic antenna switching SRS configuration], and <Unrelated part omitted> - For 2T4R, if the UE is indicating a capability for [maximum 2 semi-persistent and maximum 1 periodic SRS resource sets] and/or a capability for [extension of aperiodic antenna switching SRS configuration]: - up to 3 SRS resource sets configured across different value combinations for the higher layer parameter resourceType in SRS-ResourceSet if the UE is indicating a capability for [maximum 2 semi-persistent and maximum 1 periodic SRS resource sets] or a capability for [extension of aperiodic antenna switching SRS configuration], and <Unrelated part omitted> <End of TP for TS38.214> TP#2-4: antenna switching for 1T2R/2T4R (R1-2203950) <Start of TP for TS38.214> 6.2.1.2 UE sounding procedure for DL CSI acquisition <Unrelated part omitted> - For 1T2R, if the UE is indicating a capability for [maximum 2 semi-persistent and maximum 1 periodic SRS resource sets] and/or a capability for [extension of aperiodic antenna switching SRS configuration]: - zero or one or two SRS resource sets configured with different value of resourceType in SRS-ResourceSet set to ‘periodic’ or ‘semi-persistent’ if the UE is not indicating a capability for [maximum 2 semi-persistent and maximum 1 periodic SRS resource sets], or up to two SRS resource sets configured with resourceType in SRS-ResourceSet set to ‘semi-persistent’ and up to one SRS resource set configured with resourceType in SRS-ResourceSet set to ‘periodic’ if the UE is indicating a capability for [maximum 2 semi-persistent and maximum 1 periodic SRS resource sets], where the two SRS resource sets configured with ‘semi-persistent’ are not activated at the same time. An SRS resource set has two SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a single SRS port, and the SRS port of the second resource in the set is associated with a different UE antenna port than the SRS port of the first resource in the same set, and - zero or one SRS resource set with resourceType in SRS-ResourceSet set to ‘aperiodic’ if the UE is not indicating a capability for [extension of aperiodic antenna switching SRS configuration], or up to two SRS resource sets configured with resourceType in SRS-ResourceSet set to ‘aperiodic’ if the UE is indicating a capability for [extension of aperiodic antenna switching SRS configuration] and indicating a capability for [maximum 2 semi-persistent and maximum 1 periodic SRS resource sets], or up to two SRS resource sets configured with resourceType in SRS-ResourceSet set to ‘aperiodic’ if the UE is indicating a capability for [extension of aperiodic antenna switching SRS configuration] but not indicating a capability for [maximum 2 semi-persistent and maximum 1 periodic SRS resource sets] and the UE is configured zero or one SRS resource set configured with resourceType in SRS-ResourceSet set to ‘periodic’ or ‘semi-persistent’ for 1T2R, where in the case of one resource set, a total of two SRS resources transmitted in different symbols of one slot and where the SRS port of the second resource in the given set is associated with a different UE antenna port than the SRS port of the first resource in the same set. In the case of two resource sets, a total of two SRS resources transmitted in different symbols of two different slots, and where the SRS port of each SRS resource in the given two sets is associated with a different UE antenna port. The two sets are each configured with one SRS resource, or <Unrelated part omitted> - For 2T4R, if the UE is indicating a capability for [maximum 2 semi-persistent and maximum 1 periodic SRS resource sets] and/or a capability for [extension of aperiodic antenna switching SRS configuration]: - zero or one or two SRS resource sets configured with a different value for the higher layer parameter resourceType in SRS-ResourceSet set to ‘periodic’ or ‘semi-persistent’ if the UE is not indicating a capability for [maximum 2 semi-persistent and maximum 1 periodic SRS resource sets], or up to two SRS resource sets configured with resourceType in SRS-ResourceSet set to ‘semi-persistent’ and up to one SRS resource set configured with resourceType in SRS-ResourceSet set to ‘periodic’ if the UE is indicating a capability for [maximum 2 semi-persistent and maximum 1 periodic SRS resource sets], where the two SRS resource sets configured with ‘semi-persistent’ are not activated at the same time. Each SRS resource set has two SRS resources transmitted in different symbols, each SRS resource in a given set consisting of two SRS ports, and the SRS port pair of the second resource is associated with a different UE antenna port pair than the SRS port pair of the first resource, and, - zero or one SRS resource set configured with resourceType in SRS-ResourceSet set to ‘aperiodic’ if the UE is not indicating a capability for [extension of aperiodic antenna switching SRS configuration], or up to two SRS resource sets configured with resourceType in SRS-ResourceSet set to ‘aperiodic’ if the UE is indicating a capability for [extension of aperiodic antenna switching SRS configuration] and a capability for [maximum 2 semi-persistent and maximum 1 periodic SRS resource sets], or up to two SRS resource sets configured with resourceType in SRS-ResourceSet set to ‘aperiodic’ if the UE is indicating a capability for [extension of aperiodic antenna switching SRS configuration] but not indicating a capability for [maximum 2 semi-persistent and maximum 1 periodic SRS resource sets] and the UE is configured zero or one SRS resource set configured with resourceType in SRS-ResourceSet set to ‘periodic’ or ‘semi-persistent’ for 2T4R, where in the case of one resource set, a total of two SRS resources transmitted in different symbols in the same slot, each SRS resource in a given set consisting of two SRS ports, and the SRS port pair of the second resource is associated with a different UE antenna port pair than the SRS port pair of the first resource. In the case of two resource sets, a total of two SRS resources transmitted in different symbols of two different slots, and where the SRS port pair of each SRS resource in the given two sets is associated with a different UE antenna port pair. The two sets are each configured with one SRS resource, or <Unrelated part omitted> <End of TP for TS38.214> FL Note: It’s worth taking the following (ref R1-2203507) as a reference for the discussion of this issue. In principle, the combination of SRS resource sets for 1T2R/2T4R antenna switching in the case of i. “Feature A + non Feature B” and ii. “non Feature A + Feature B” may need to be clarified in the current specification, as long as it makes sense to limit the total maximum number of SRS resource sets is 3. Consequently, both TP#2-3 and TP#2-4 aim to address this issue by taking this limitation. By comparison, TP#2-3 captures both case I and case ii, TP#2-3 only captures case i. Feature A: UE capability for [extension of aperiodic antenna switching SRS configuration] Feature B: UE capability for [maximum 2 semi-persistent and maximum 1 periodic SRS resource sets] - Table 1 Combinations of different features for 1T2R and 2T4R The number of max sets of 1T2R/2T4R; Feature B (SP/P SRS sets). Not support; Support. Feature A (AP SRS sets); Not support; 1P+1SP or 1P+1AP or 1SP+1AP Combination 1; Option 1: 1P+2SP+0AP Option 2: 1P+2SP+1AP Combination 4. Support; Option 1: 1P+1SP+2AP Option 2: 1P/1SP+2AP Option 3: 0P/SP+2AP Combination 3; 2SP + 1P + 2AP Combination 2. | TP#2-3: - Support/Fine: - Not support: TP#2-4: - Support/Fine: - Not support: |

Additional inputs of issue#2-3&2-4

| Company | Input |
| --- | --- |
| Mod V0 | - Check and update your view in the table above - Share more inputs here if needed |
| Samsung | We are fine with further clarification on the cases when either one of two new features is configured, and TP 2-3 can be a good starting point for the discussion. We wonder whether the intention of TP 2-3 is to add red part on top of the current specification (it means that 3 sub-bullets (2 existing + 1 new) is under both 1T2R and 2T4R) or to change existing 2 sub-bullets into a proposed red part (it means that only a new single sub-bullet is under both 1T2R and 2T4R). |
| Apple | We are fine with the discussion |
| DOCOMO | We think RAN1 needs to decide whether to limit the total number of SRS-ResourceSets to 3 or 2 for combination 3 and 3 for combination 2 at first. If such limitation is deemed necessary, TP can be discussed based on the proposal. |
| OPPO | To FL: TP#2-4 is also a complete solution since some case(s) has be captured by the current spec. For a UE not supporting Feature A and not supporting Feature B (i.e., legacy UE), it can be only configured with two different SRS configuration (e.g., 1P+1SP, 1P+1AP, 1SP+1AP). For a UE supporting Feature A but not supporting Feature B, it should follow the same restriction as legacy UE, but the 1AP SRS configuration can consist of up to SRS resource set. However, if we follow TP#2-3, the UE can be configured with 1P+1SP+1AP, which is not aligned with previous agreement. Thus, TP#2-4 is aligned with our agreement. |
| ZTE | Not needed. The TP will lead unnecessary restriction of scheduling of gNB and doesn’t provide any benefit for UE. |
| Huawei, HiSilicon | We are OK to limit the total number of resource set to 3 when either one of two new features is supported. |
| Intel | We are open for discussion on this issue. One question is since we are discussing Rel-17 UE behavior, and we don’t have agreement on the total number of SRS resource sets including all the time behaviors, why not allow the max number of SRS resource sets to be 4 for more flexibility, i.e., Option 1 for Combination 3 and Option 2 for Combination 4? |
| CATT | We are OK to limit the total number of SRS resource set to 3 when either one or both of two new futures are supported. |
| Vivo | We are OK to limit the total number of resource set to 3 when either one of two new features is supported. |
| QC | We are open to discussion. We should still use legacy rule and allow extension of #sets based on which feature is supported - When UE supports feature B only (extension of SP sets), we support option 1 (2SP + 1P) to configured or 2 sets (1P+1SP, 1P+1AP, 1SP+1AP) similar to legacy. - When UE supports feature A only (extension of AP sets), then UE can be configured with the extended AP sets plus one addition set (P or SP) or can be configured with legacy 2 sets (1P+1SP, 1P+1AP, 1SP+1AP) |
| Lenovo | We are open to discuss this issue. And we share similar view with QC that the legacy rule should adopted as much as possible. |

Issue#3-6: Maximum value of available slot offset ‘t’

| Issue | Companies’ views |
| --- | --- |
| Maximum value of available slot offset ‘t’ (R1-2204978) FL proposal proposal 3-6: UE drops the triggered A-SRS if the available slot offset is not reached within a specific span of time. FL Note: The problem of this issue seems existent and critical, companies are encouraged to share views on whether this issue can be addressed by proposal 3-6 and please provide the corresponding TP, if any. | Proposal 3-6: - Support/Fine: - Not support: |

Additional inputs of issue#3-6

| Company | Input |
| --- | --- |
| Mod V0 | - Check and update your view in the table above - Share more inputs here if needed |
| Samsung | Support the proposal, and the exact value of the span of time (e.g., number of slots [per SCS]), how the UE/gNB knows the span of time (e.g., by fixed value in spec, or by RRC configuration based on the corresponding UE capability) can be further discussed. Our initial thinking is that 32 slots which is the maximum value of legacy slot offset by RRC configuration can be a starting point of the discussion, but it can be different for each UE based on UE’s capability on bookkeeping. |
| LGE | We are not sure that it is critical issue at this maintenance stage. It can be treated as corner case or error case. |
| NEC | Support the proposal. |
| Apple | We are fine with the proposal |
| DOCOMO | We are ok to discuss this issue. However, we do not think it would be good to have dependency on UE capability. As supported in legacy slotOffset, 32 slots as a single limitation for actual offset can be straightforward. |
| OPPO | Seem an optimization |
| ZTE | We support the proposal. In addition, we think that the starting of the specific span and the length of the a specific span can be further studied. So we propose following updated proposal: Proposal 3-6-1 UE drops the triggered A-SRS if the available slot offset is not reached within a specific span of time which starts form the last symbol of DCI triggering the A-SRS and has a length of 32 slots |
| Huawei, HiSilicon | Agree with LGE and OPPO. We don’t think this is a critical issue that must be treated in maintenance phase. |
| Intel | Open for discussion |
| CATT | Similar view as LGE, OPPO, and Huawei. |
| Vivo | Agree with CATT, Huawei/HiSilicon, LGE, OPPO |
| QC | We support the proposal and fine with ZTE edit. It is not optimization step as other companies noted. It has an impact on UE timeline. |
| Ericsson | Ok with the proposal at the limit of 32 slots for all UE that support the feature. No UE capability. |
| Lenovo | Agree with vivo, CATT, Huawei/HiSilicon, LGE, OPPO. |

- Summary of Editorial (E) issues

Issue#1-1: Collision handling for aperiodic SRSs

| Issue | Companies’ views |
| --- | --- |
| TP#1-1: collision handling for aperiodic SRSs (R1-2203303) <Start of TP for TS38.214> 6.2.1 UE sounding procedure <Unrelated part omitted> In case a SRS resource with resourceType set as 'aperiodic' is triggered on the OFDM symbol(s) configured with periodic/semi-persistent SRS transmission, the UE shall transmit the aperiodic SRS resource and only the periodic/semi-persistent SRS symbol(s) overlapping within the symbol(s) are dropped, while the periodic/semi-persistent SRS symbol(s) that are not overlapped with the aperiodic SRS resource are transmitted. In case a SRS resource with resourceType set as 'semi-persistent' is triggered on the OFDM symbol(s) configured with periodic SRS transmission, the UE shall transmit the semi-persistent SRS resource and only the periodic SRS symbol(s) overlapping within the symbol(s) are dropped, while the periodic SRS symbol(s) that are not overlapped with the semi-persistent SRS resource are transmitted. For the overlapping SRS resources with the higher layer parameter resourceType in SRS-Resource set to 'aperiodic', and at least one resource set is configured with parameter availableSlotOffset across all configured BWPs in a component carrier except when SRS is configured with the higher layer parameter SRS-PosResource, a UE can transmit one or more SRS resources subject to UE's capability, where - the SRS resource(s) correspond lowest SRS-ID(s) with lowest CC-ID(s). <Unrelated part omitted> <End of TP for TS38.214> FL Note: In general, this TP aims to execute the dropping rule to handle the case of “multiple SRS resource sets with overlapping symbols and/or triggered by a same DCI” based on SRS resource set ID and its associated CC ID. Companies are encouraged to share views on whether this issue can be addressed by this TP. | TP#1-1: - Support/Fine: - Not support: |

Additional inputs of issue#1-1

| Company | Input |
| --- | --- |
| Mod V0 | - Check and update your view in the table above - Share more inputs here if needed |
| Samsung | Not support. We are not sure why this issue has been captured as “E”, since the result of preparation phase for this issue was 4 for “H” and 5 for “N”. We think since this can be handled by gNB implementation, there is no need to define an additional collision handling rule. |
| Apple | Not support |
| DOCOMO | Though we are supportive to discuss this issue, we agree that it is not E. |
| OPPO | Not support as it seems to introduce a new functionality in the maintenance phase. |
| ZTE | Not support. It is unnecessary because the gNB can avoid such scheduling. |
| Huawei, HiSilicon | Not support. This can be solved by gNB implementation. |
| Intel | Although we are generally fine with the TP, this issue should not be “E”. |
| CATT | We are fine to introduce collision rule. For the TP, we wonder whether “one or more SRS resources subject to UE's capability” means a new UE capability is needed or not? |
| Mod V14 | The status so far is: Support as “H”: DOCOMO, Intel, CATT Do NOT support: Samsung, Apple, OPPO, ZTE, Huawei/HiSilicon, QC Due to the similar situation as preparation phase, the discussion of this issue will be closed. |
| Mod V19 | No further discussion. |

Issue#1-2:  SRS configuration for T=R antenna switching

| Issue | Companies’ views |
| --- | --- |
| TP#1-2.A: SRS configuration for T=R antenna switching (R1-2203423) <Start of TP for TS38.214> 6.2.1.2 UE sounding procedure for DL CSI acquisition <Unrelated part omitted> - For 1T=1R, or 2T=2R, or 4T=4R, up to two SRS resource sets each with one SRS resource, where the number of SRS ports for each resource is equal to 1, 2, or 4 if the UE is not indicating a capability for [maximum 2 semi-persistent and maximum 1 periodic SRS resource sets], or up to two SRS resource sets configured with resourceType in SRS-ResourceSet set to 'semi-persistent' and up to one SRS resource set configured with resourceType in SRS-ResourceSet set to 'periodic' if the UE is indicating a capability for [maximum 2 semi-persistent and maximum 1 periodic SRS resource sets], where the two SRS resource sets configured with 'semi-persistent' are not activated at the same time,, or <Unrelated part omitted> <End of TP for TS38.214> TP#1-2.B: SRS configuration for T=R antenna switching (R1-2204904) <Start of TP for TS38.214> 6.2.1.2 UE sounding procedure for DL CSI acquisition <Unrelated part omitted> For 1T=1R, or 2T=2R, or 4T=4R, up to two SRS resource sets each with one SRS resource can be configured, where the number of SRS ports for each resource is equal to 1, 2, or 4, if the UE is not indicating a capability for [maximum 2 semi-persistent and maximum 1 periodic SRS resource sets]., or up to tTwo SRS resource sets configured with resourceType in SRS-ResourceSet set to 'semi-persistent' and up to one SRS resource set configured with resourceType in SRS-ResourceSet set to 'periodic' can be configured and the two SRS resource sets configured with 'semi-persistent' are not activated at the same time, or up to two SRS resource sets can be configured, if the UE is indicating a capability for [maximum 2 semi-persistent and maximum 1 periodic SRS resource sets], where each SRS resource set has one SRS resource, the number of SRS ports for each resource is equal to 1, 2, or 4 where the two SRS resource sets configured with 'semi-persistent' are not activated at the same time, or <Unrelated part omitted> <End of TP for TS38.214> FL Note: According to previous discussions of this issue in RAN#108-e meeting, TP#1.2-B is indeed in line with the output in Round 4 discussion (ref. R1-2202904), which is a compromise for handling companies’ concerns. To avoid any unnecessarily fallback discussions, it will be proper to adopt TP#1.2-B to make progress. | TP#1-2.A: - Support/Fine: - Not support: TP#1-2.B: - Support/Fine: LGE - Not support: |

Additional inputs of issue#1-2

| Company | Input |
| --- | --- |
| Mod V0 | - Check and update your view in the table above - Share more inputs here if needed |
| LGE | OK with 1-2.B. |
| Apple | Fine with 1-2B |
| DOCOMO | Ok with 1-2.B. |
| OPPO | Support 1-2.B |
| ZTE | We think up to two period SRS sets and up to one period SRS set can be configured if the UE is indicating a capability for [maximum 2 semi-persistent and maximum 1 periodic SRS resource sets] as captured in current specification. So we propose following TP #1-2.C TP #1-2.C For 1T=1R, or 2T=2R, or 4T=4R, up to two SRS resource sets each with one SRS resource can be configured with a different value for the higher layer parameter resourceType in SRS-ResourceSet set to 'periodic' or 'semi-persistent' , where each SRS resource set has one SRS resource and the number of SRS ports for each resource is equal to 1, 2, or 4 if the UE is not indicating a capability for [maximum 2 semi-persistent and maximum 1 periodic SRS resource sets], or up to two SRS resource sets configured with resourceType in SRS-ResourceSet set to 'semi-persistent' and up to one SRS resource set configured with resourceType in SRS-ResourceSet set to 'periodic' if the UE is indicating a capability for [maximum 2 semi-persistent and maximum 1 periodic SRS resource sets], where the two SRS resource sets configured with 'semi-persistent' are not activated at the same time, each SRS resource set has one SRS resource and the number of SRS ports for each resource is equal to 1, 2, or 4 |
| Huawei, HiSilicon | Support 1-2.B. |
| Intel | Generally fine with TP #1-2.B. But we slightly prefer to have clear agreement on the SRS configuration for xTyR with x=y instead of TP only. The reason is that in the corresponding UE feature (FG 23-8-4), it captures that the feature applies for all xTyR. Note1: - Applies for all supported xTyR where y<=8 - For xTyR where y>4, if UE does NOT support this feature, support maximum one SRS resource set for periodic SRS and maximum one SRS resource set for semi-persistent SRS - For xTyR where y<=4, if UE does not support this feature, follow Rel-15 on the number of resource sets for periodic and semi-persistent SRS - The two SP-SRS resource sets are not activated at the same time With this TP, the UE feature description and the spec is not aligned. Having a clear agreement can avoid unnecessary discussion in UE feature session. |
| CATT | Fine with 1-2.B. |
| vivo | This issue is also related to max number of supported SRS resource set, generally fine with TP#1-2B in principle, need more discussion |
| Mod V14 | @ZTE, vivo: please note that issue#1-2 aims to guarantee the legacy rule that up to two SRS resource sets can be configured for antenna switching T=R, which is consistent with previous agreements and the consensus of companies in the last meeting. From the perspective of FL, it will not be workable to introduce the optimization, e.g., the maximum number of SRS resource sets can be more than two, in the maintenance phase. @Intel: based on my explanation to ZTE and vivo, there is no problem in discussing this TP and FG 23-8-4 separately. Regarding your concern of the description of the FG above, it could be further discussed in UE feature session. @All: as my previous clarification, TP#1-2.B has been fully discussed so far (up to four rounds in the last meeting) and this is a hard compromise reached by companies. Moderator hopes that companies could take this situation into account and can be flexible to make progress on this issue. If no more input of concern before UTC 04:00AM on May 11 (Wednesday), TP#1-2.B will be assumed to be approved. |
| QC | Fine with the TP#1-2B. |
| Ericsson | Fine with the TP#1-2B. |
| ZTE | We are fine with TP#1-2B for progress |
| Lenovo | Fine with the TP#1-2B. |
| Mod V19 | No further concern of TP#1.2-B. Switch to RAN1 reflector for approval. |

Issue#2-5:  SRS for antenna switching

| Issue | Companies’ views |
| --- | --- |
| TP#2-5: SRS for antenna switching (R1-2203423) <Start of TP for TS38.214> 6.2.1.2 UE sounding procedure for DL CSI acquisition <Unrelated part omitted> When the UE is configured with the higher layer parameter usage in SRS-ResourceSet set as 'antennaSwitching', the UE may be configured with only one of the following configurations depending on the indicated UE capability supportedSRS-TxPortSwitch ('t1r2' for 1T2R, 't1r1-t1r2' for 1T=1R/1T2R, 't2r4' for 2T4R, 't1r4' for 1T4R, 't1r6' for 1T6R, '1t8r' 't1r8' for 1T8R, '2t6r' 't2r6' for 2T6R, '2t8r' 't2r8' for 2T8R, '4t8r' 't4r8' for 4T8R, 't1r1-t1r2-t1r4' for 1T=1R/1T2R/1T4R, 't1r4-t2r4' for 1T4R/2T4R, 't1r1-t1r2-t2r2-t2r4' for 1T=1R/1T2R/2T=2R/2T4R, 't1r1-t1r2-t2r2-t1r4-t2r4' for 1T=1R/1T2R/2T=2R/1T4R/2T4R, 't1r1' for 1T=1R, 't2r2' for 2T=2R, 't1r1-t2r2' for 1T=1R/2T=2R, 't4r4' for 4T=4R, or 't1r1-t2r2-t4r4' for 1T=1R/2T=2R/4T=4R): <Unrelated part omitted> <End of TP for TS38.214> | TP#2-5: - Support/Fine: LGE - Not support: |

Additional inputs of issue#2-5

| Company | Input |
| --- | --- |
| Mod V0 | - Check and update your view in the table above - Share more inputs here if needed |
| Samsung | Support the TP. |
| LGE | OK |
| NEC | Support |
| Apple | Fine with TP |
| DOCOMO | Support |
| OPPO | Support |
| ZTE | Support |
| Huawei, HiSilicon | Support. |
| Intel | Fine with the TP |
| CATT | Support |
| vivo | fine |
| Mod V14 | Acceptable to all companies so far. |
| QC | Support |
| Ericsson | ok |
| Lenovo | Support |
| Mod V19 | Acceptable to all companies. Switch to RAN1 reflector for approval. |

Issue#3-2:  Partial frequency hopping

| Issue | Companies’ views |
| --- | --- |
| TP#3-2: Partial frequency hopping (R1-2204171) <Start of TP for TS38.211> 6.4.1.4.3 Mapping to physical resources <Unrelated part omitted> When SRS is transmitted on a given SRS resource, the sequence rpi(n,l') for each OFDM symbol l' and for each of the antenna ports of the SRS resource shall be multiplied with the amplitude scaling factor in order to conform to the transmit power specified in [5, 38.213] and mapped in sequence starting with to resource elements in a slot for each of the antenna ports according to The length of the sounding reference signal sequence is given by Msc,bSRS=mSRS,bNscRBKTCPF where mSRS,b is given by a selected row of Table 6.4.1.4.3-1 with where is given by the field b-SRS contained in the higher-layer parameter freqHopping if configured, otherwise BSRS=0. The row of the table is selected according to the index given by the field c-SRS contained in the higher-layer parameter freqHopping. The quantity PF∈{2,4} is given by the higher-layer parameter FreqScalingFactor if configured, otherwise PF=1. When FreqScalingFactor is configured, the UE expects the length of the SRS sequence to be a multiple of 6. <Unrelated part omitted> <End of TP for TS38.211> | TP#3-2: - Support/Fine: LGE - Not support: |

Additional inputs of issue#3-2

| Company | Input |
| --- | --- |
| Mod V0 | - Check and update your view in the table above - Share more inputs here if needed |
| Sansung | Support the TP. |
| LGE | OK |
| Apple | Fine with TP |
| DOCOMO | Support |
| OPPO | Open to it although we think the TP is not necessary |
| ZTE | Support |
| Huawei, HiSilicon | Support. |
| Intel | Fine with the TP |
| CATT | OK with the TP. |
| vivo | fine |
| Mod V14 | Acceptable to all companies so far. |
| QC | Support |
| Ericsson | ok |
| Lenovo | Support |
| Mod V19 | Acceptable to all companies. Switch to RAN1 reflector for approval. |

Issue#3-3:  SRS frequency hopping

| Issue | Companies’ views |
| --- | --- |
| TP#3-3: SRS frequency hopping (R1-2203950) <Start of TP for TS38.214> 6.2.1.1 UE SRS frequency hopping procedure <Unrelated part omitted> For a given SRS resource, the UE is configured with repetition factor R∈{1,2,4} or R∈{1,2,3,4,5,6,7,8,10,12,14} by higher layer parameter resourceMapping in SRS-Resource where R≤Ns. When frequency hopping within an SRS resource in each slot is not configured (R=Ns), each of the antenna ports of the SRS resource in each slot is mapped in all the symbols to the same set of subcarriers in the same set of PRBs. When frequency hopping within an SRS resource in each slot is configured without repetition (R=1), according to the SRS hopping parameters , and defined in clause 6.4.1.4 of [4, TS 38.211], each of the antenna ports of the SRS resource in each slot is mapped to different sets of subcarriers in each OFDM symbol, where the same transmission comb value is assumed for different sets of subcarriers. When both frequency hopping and repetition within an SRS resource in each slot are configured (Ns ≥ 4, R ≥ 2), each of the antenna ports of the SRS resource in each slot is mapped to the same set of subcarriers within each pair set of R adjacent OFDM symbols, and frequency hopping across the pairs sets is according to the SRS hopping parameters , and , where should be divisible by . For operation with shared spectrum channel access, the UE does not expect that multiple hops of an SRS resource transmission are in different RB sets. A UE may be configured adjacent symbol aperiodic SRS resource with intra-slot frequency hopping within a bandwidth part, where the full hopping bandwidth is sounded with an equal-size subband across symbols when frequency hopping is configured with R=1. A UE may be configured Ns ≥ 4 adjacent symbols aperiodic SRS resource with intra-slot frequency hopping within a bandwidth part, where the full hopping bandwidth is sounded with an equal-size subband across pairs sets of R adjacent OFDM symbols, when frequency hopping is configured with R ≥ 2, Ns ≥ R and Ns should be divisible by R. Each of the antenna ports of the SRS resource is mapped to the same set of subcarriers within each pair set of R adjacent OFDM symbols of the resource. A UE may be configured symbol periodic or semi-persistent SRS resource with inter-slot hopping within a bandwidth part, where the SRS resource occupies the same symbol location in each slot. A UE may be configured symbol periodic or semi-persistent SRS resource with intra-slot and inter-slot hopping within a bandwidth part, where the N-symbol SRS resource occupies the same symbol location(s) in each slot. For Ns ≥ 4, when frequency hopping is configured with R ≥ 2, intra-slot and inter-slot hopping is supported with each of the antenna ports of the SRS resource mapped to different sets of subcarriers across pairs sets of R adjacent OFDM symbol(s) of the resource in each slot, where should be divisible by R. Each of the antenna ports of the SRS resource is mapped to the same set of subcarriers within each pair set of R adjacent OFDM symbols of the resource in each slot. For Ns= R, when frequency hopping is configured, inter-slot frequency hopping is supported with each of the antenna ports of the SRS resource mapped to the same set of subcarriers in R adjacent OFDM symbol(s) of the resource in each slot. <Unrelated part omitted> <End of TP for TS38.214> | TP#3-3: - Support/Fine: - Not support: |

Additional inputs of issue#3-3

| Company | Input |
| --- | --- |
| Mod V0 | - Check and update your view in the table above - Share more inputs here if needed |
| Samsung | Support the TP. |
| Apple | Fine with TP |
| DOCOMO | Support |
| OPPO | Support |
| ZTE | Support |
| Huawei, HiSilicon | Support. |
| Intel | Fine with the TP |
| CATT | Support |
| vivo | fine |
| Mod V14 | Acceptable to all companies so far. |
| QC | Support |
| Ericsson | ok |
| Lenovo | Support |
| Mod V19 | Acceptable to all companies. Switch to RAN1 reflector for approval. |

Issue#3-4:  Parameter alignment for start RB hopping

| Issue | Companies’ views |
| --- | --- |
| TP#3-4: Parameter alignment for start RB hopping (R1-2204904) <Start of TP for TS38.214> 6.2.1 UE sounding procedure <Unrelated part omitted> - Defining start RB index hopping for partial frequency sounding in different SRS frequency hopping periods for aperiodic/periodic/semi-persistent SRS based on the hopping pattern khopkhopping as described in clause 6.4.1.4.3 in [4, TS 38.211. If not configured, then start RB hopping is not enabled and khopkhopping is fixed to be 0 for all SRS symbols. <Unrelated part omitted> <End of TP for TS38.214> | TP#3-4: - Support/Fine: LGE - Not support: |

Additional inputs of issue#3-4

| Company | Input |
| --- | --- |
| Mod V0 | - Check and update your view in the table above - Share more inputs here if needed |
| Samsung | Support the TP. |
| LGE | OK |
| NEC | Support |
| Apple | Fine |
| DOCOMO | Support |
| OPPO | Support |
| ZTE | Support |
| Huawei, HiSilicon | Support. |
| Intel | Fine with the TP |
| CATT | Support |
| vivo | fine |
| Mod V14 | Acceptable to all companies so far. |
| QC | Support |
| Ericsson | ok |
| Lenovo | Support |
| Mod V19 | Acceptable to all companies. Switch to RAN1 reflector for approval. |

- References

| 1 | R1-2203260 | Remaining issues on SRS and CSI enhancements | ZTE |
| --- | --- | --- | --- |
| 2 | R1-2203303 | Remaining issues on SRS in Rel-17 | Spreadtrum Communications |
| 3 | R1-2203423 | Maintenance issues on SRS | CATT |
| 4 | R1-2203507 | Miscellaneous correction on Rel-17 MIMO | vivo |
| 5 | R1-2203675 | Discussion on remaining issues on SRS enhancement | NEC |
| 6 | R1-2203773 | Remaining issues on M-TRP beam management, SRS and CSI enhancement | xiaomi |
| 7 | R1-2203857 | Maintenance on Rel-17 SRS and CSI | Samsung |
| 8 | R1-2203950 | Other maintenance issues on further MIMO enhancements | OPPO |
| 9 | R1-2204030 | Remaining issues on CSI enhancements for Multi-TRP | Ericsson |
| 10 | R1-2204139 | Text proposal on SRS enhancement | LG Electronics |
| 11 | R1-2204171 | Remaining issues on SRS enhancements | Lenovo |
| 12 | R1-2204337 | Remaining issues on SRS enhancements | NTT DOCOMO, INC. |
| 13 | R1-2204537 | Maintenance of misceleneus Rel-17 MIMO issues | Nokia, Nokia Shanghai Bell |
| 14 | R1-2204765 | Remaining details on SRS enhancement | Intel Corporation |
| 15 | R1-2204904 | Remaining issues on SRS enhancement in Rel-17 | Huawei, HiSilicon |
| 16 | R1-2204978 | Remaining issues for SRS enhancement | Qualcomm Incorporated |
| 17 | R1-2203261 | R17 FeMIMO preparation phase 1 - Item 3 SRS | Moderator (ZTE) |