3GPP TSG RAN WG1 #109-e	                                               	R1-2205199

e-Meeting, May 9th – 20th, 2022

Agenda Item: 8.14

Source:	Moderator (Sony)

Title:	FL summary 1 of AI 8.14: Maintenance on Timing Relationships for IoT-NTN

Document for:	Discussion

Table of Contents

1	Introduction	1

2	Overview of Main Issues from company contributions	2

2.1	Issues# 2.1 & 2.4: NPDCCH Monitoring restrictions (case 1-6)	2

2.1.1	Companies Views	3

2.1.2	FIRST ROUND Discussion Issues# 2.1 & 2.4 NPDCCH Monitoring restrictions (case 1-6)	5

2.2	Issue# 2-2 & 2-3: NPDCCH Monitoring restrictions (cases 7-10)	7

2.2.1	Companies Views	7

2.2.2	FIRST ROUND Discussion of Issue# 2-2 & 2-3: NPDCCH Monitoring restrictions (case 7-10)	11

2.3	Issue# 2-5: Inclusion of Calculation of UE-eNB RTT in Spec	12

2.3.1	Companies Views	12

2.3.2	FIRST ROUND Discussion of Issue# 2-5: Inclusion of calculation UE-eNB RTT in spec	13

2.4	Issue# 2-6: Maintain Legacy behaviour in spec for TDD mode	14

2.4.1	Companies Views	14

2.4.2	FIRST ROUND Discussion on Issue# 2-6: Maintain Legacy behaviour in spec for TDD mode	17

3	Referenced Documents	20

# Introduction

This is the feature lead (FL) summary of contributions for the following discussion:

[109-e-R17-IoT-NTN-02] Email discussion for maintenance on timing relationship enhancements for issues 2-1, 2-2, 2-3, 2-4, 2-5 and 2-6 from R1-2205110 – Sam (Sony)

- 1st check point: May 13 (any RRC impact by May 12)

- Final check point: May 18

The issues to be tackled in this discussion were identified during the preparatory phase and are summarised in R1-2205110. The table below is reproduced from there:

| Issue# | Issue | References | FL initial assessment |
| --- | --- | --- | --- |
| 2-1 | NPDCCH Monitoring restrictions (case 1-6) FL: this issue has been discussed in at least 3 meetings. The spec could be clearer but all companies seemed to agree at last meeting that the spec is not wrong. | R1-2203089, R1-2203386, R1-2203991 | E |
| 2-2 | NPDCCH Monitoring restrictions (case 7-10) for misaligned UL and DL frame timing FL: there seems to be a particular issue warranting a second look when UL and DL frame timing is not aligned. | R1-2203089, R1-2203386, R1-2203722 | H |
| 2-3 | Address NPDCCH monitoring restrictions for subframes post-NPUSCH FL: this is the case in Rel16 and so RAN1 should make sure it is maintained in Rel17 | R1-2204997 | E |
| 2-4 | Consistency of use between logical time and physical time in timing relationship descriptions FL: this will help make the spec clearer with respect to Issue#2-1 for example. | R1-2204997 | E |
| 2-5 | Capture RAN1#108e agreement on calculation of UE-eNB RTT FL: this agreement was made in the last meeting and should be incorporated in the spec. | R1-2203386 | E |
| 2-6 | IoT NTN WI has dealt only with FDD. Maintain legacy behaviour in specs for TDD TS36.213 clauses 16.4.2, 16.5.1, 10.2 FL: Makes sense not to change spec in the case of TDD | R1- 2203632 | E |

# Overview of Main Issues from company contributions

From the preparatory phase, 6 issues have already been identified as outlined in Table 1. In this section, FL shall summarise company contributions on each issue and make proposals.

## Issues# 2.1 & 2.4: NPDCCH Monitoring restrictions (case 1-6)

In RAN1 #106bis-e, restrictions on NPDCCH monitoring were discussed and the different cases needing study for enhancement were designated in the following agreement.

Agreement: NPDCCH monitoring restrictions have been identified for further checking to see if changes for NB-IoT need to be made for the following cases: - case 1: MTBG NPUSCH - case 2: 2 NPUSCH HARQ processes scheduled - case 3: long single NPUSCH when MTBG or 2HARQ configured - case 4: single NPUSCH scheduled by DCI format N0 or RAR - case 5: NPUSCH format 2 in response to DCI format N1 - case 6: NPRACH in response to PDCCH order - case 7: NPUSCH with same HARQ process when 2 HARQ configured - case 8: subframes after NPUSCH processing - case 9: subframes after NPUSCH carrying Msg3 - case 10: NPRACH for SR for long NPRACH transmissions - case 11: NPRACH for SR for short NPRACH transmissions - FFS: the changes in each case - FFS: additional cases

At RAN1#107e, RAN1 agreed as follows:

Agreement

Modification of the designation of subframes with NPDCCH monitoring restrictions is needed for at least Cases 1 to 6.

Following a further discussion at RAN1#107e, companies could not reach a consensus as to the preferred way to describe in the specifications the DL subframes that the UE does not have to monitor for NPDCCH.  The following was concluded on cases 1-6:

Conclusion:

Leave it to spec editor to formulate in the specs the NPDCCH monitoring restrictions for Cases 1 to 6.

Explanatory Note for editor

When the UE changes from receiving on the DL to transmitting on the UL (or vice versa), immediately before/after the DL/UL switch the UE is not required to monitor an NPDCCH candidate in some DL subframes. The designation of these subframes in the spec needs to take the “effect” of the TA into consideration. There may be multiple ways to capture this in the specifications for (at least) Cases 1 to 6. Two options (in principle) are described below, to guide the spec editor to capture this as best he/she sees it. Examples of where the changes may apply for cases 1 to 6 can be found as examples in appendix A in R1-2112554.

Option 1: The DL subframes during which the UE is not required to monitor an NPDCCH candidate are described in terms of downlink subframe timing. This would typically involve inserting a “-TA” term in their indexing.

Option 2: The DL subframes during which the UE is not required to monitor an NPDCCH candidate are described in terms of uplink subframe timing using the indexing of the UL subframes that coincide in time with the DL subframes in question.

Taking Clause 16.6 of TS 36.213 Rel17 as an example, the specification editor uses the form:

… if the NB-IoT UE has a NPUSCH transmission ending in subframe n (accounting for uplink transmission timing), the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+3.

in describing the subframes designated for restricted NPDCCH monitoring. It therefore seems that the spec editor leans towards Option 2 in the way these subframes are described in the spec. The spec editor has not however clarified what “accounting for uplink transmission timing” means and how it is to be achieved in this context. Some companies think this could lead to misinterpretations and would like the language to be clearer. Other companies think the description in the current spec is fine.

### Companies Views

| Huawei | Observation 1: The description on the timing relationships is typically based on logical timing and there is no need to introduce a physical timing of TA. Proposal 2: For description on the NPDCCH monitoring of case1~6, option 2 is preferable, and the current specification text is enough. |
| --- | --- |
| MediaTek | Proposal 9: It is preferable to utilize Option 1 for NPDCCH monitoring restrictions for Cases 1 to 11 in the specifications. - The DL subframes during which the UE is not required to monitor an NPDCCH candidate are described in terms of downlink subframe timing. This would typically involve inserting a “-TA” term in their indexing. Observation 4: For Option 2 for NPDCCH monitoring restrictions for Cases 1 to 6 in the specifications, it is necessary to clarify the UE behaviour in the specifications where eNB schedules the NPUSCH to start in UL subframe n+k, where the ‘k’ variable is a generic subframe index corresponding to “k0+ K_offset –TA”. |
| OPPO | Proposal 1: For NPDCCH monitoring restrictions. - Adopt following TP#1 for TP 36.213 V17.1.0 *** < Beginning of TP#1 for TP 36.213 V17.1.0> *** 16.6 Narrowband physical downlink control channel related procedures *** < Unchanged parts are ommitted> *** For a NPDCCH UE-specific search space, if a NB-IoT UE is configured with higher layer parameter twoHARQ-ProcessesConfig or npusch-MultiTB-Config and if the NB-IoT UE detects NPDCCH with DCI Format N0 ending in subframe n, and if the corresponding NPUSCH format 1 transmission starts from n+k (accounting for uplink transmission timing), - if the corresponding NPDCCH with DCI format N0 with CRC scrambled by C-RNTI schedules two transport blocks as determined by the Number of scheduled TB for Unicast field if present, the UE is not required to monitor an NPDCCH candidate in any subframe starting from subframe n+1 to downlink subframe overlapping with uplink subframe n+k-1, otherwise the UE is not required to monitor an NPDCCH candidate in any subframe starting from downlink subframe overlapping with uplink subframe n+k-2 to downlink subframe overlapping with uplink subframe n+k-1; and - the UE does not expect to receive a DCI Format N0 before downlink subframe overlapping with uplink subframe n+k-2 for which the corresponding NPUSCH format 1 transmission ends later than subframe n+k+255 if the corresponding NPDCCH with DCI format N0 schedules one transport block. - for TDD, and if the corresponding NPUSCH format1 transmission ends in subframe n+m, the UE is not required to monitor NPDCCH in any subframe starting from subframe n+ k to subframe n+m-1. otherwise - if the NB-IoT UE detects NPDCCH with DCI Format N0 ending in subframe n or receives a NPDSCH carrying a random access response grant ending in subframe n, and if the corresponding NPUSCH format 1 transmission starts from n+k (accounting for uplink transmission timing), the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to downlink subframe overlapping with uplink subframe n+k-1. - for TDD, if the NB-IoT UE detects NPDCCH with DCI Format N0 ending in subframe n or receives a NPDSCH carrying a random access response grant ending in subframe n, and if the corresponding NPUSCH format 1 transmission ends in n+k, the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+k. *** < Unchanged parts are ommitted> *** If a NB-IoT UE detects NPDCCH with DCI Format N1 ending in subframe n, and if the corresponding NPDSCH transmission starts from n+k, and - for FDD, if the corresponding NPUSCH format 2 transmission starts from subframe n+m (accounting for uplink transmission timing), the UE is not required to monitor NPDCCH in any subframe starting from subframe n+ k to downlink subframe overlapping with uplink subframe n+m-1. - for TDD, if the corresponding NPUSCH format 2 transmission ends in subframe n+m the UE is not required to monitor NPDCCH in any subframe starting from subframe n+ k to subframe n+m-1. If a NB-IoT UE detects NPDCCH with DCI Format N1 for "PDCCH order" ending in subframe n, and - for FDD, if the corresponding NPRACH transmission starts from subframe n+k (accounting for uplink transmission timing), the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to downlink subframe overlapping with uplink subframe n+k-1. - for TDD, if the corresponding NPRACH transmission ends in subframe n+k, the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+k-1. *** < Unchanged parts are ommitted> *** *** < End of TP#1 for TP 36.213 V17.1.0> *** |
| Qualcomm | Proposal 3: Clarify the use of logical/physical time in different places of the specifications. - Option 1: For half-duplex monitoring restrictions (such as Example 1.2 in this contribution), use logical time to index uplink transmissions with UL indices, DL reception with DL indices, and use a TA term to link the two. - Option 2: Include a table in the specifications to state which relationships use logical time, and which use physical time. - Currently, only the half-duplex monitoring restrictions appear to use physical time, while other timing relationships use logical time. |

### FIRST ROUND Discussion Issues# 2.1 & 2.4 NPDCCH Monitoring restrictions (case 1-6)

The UE is not expected to monitor DL subframes for NPDCCH just before, during and immediately after the time when it transmits on the UL. Around these times, UL subframe indexing and/or UL absolute time that incorporates the TA are both clear for the UE. Therefore, what the specification has to describe is what DL subframes coincide or overlap with the given UL subframe, as OPPO puts it. The spec editor’s leaning towards Option 2 (using logical time instead of absolute time) is understandable because Rel16 already describes these subframes using logical subframe indexing. As suggested by OPPO, FL does not think that the spec editor’s approach should be changed too drastically. FL proposes the following TP with respect to cases 1 – 6. In this TP, when the original text indexes a DL subframe, for example:

“NB-IoT UE detects NPDCCH with DCI Format N0 ending in subframe n”,

The coincident UL subframe is described with respect to this index as:

“uplink subframe which, after accounting for uplink transmission timing, overlaps with downlink subframe n”

This allows the indexing in the rest of the text to be consistently describe DL or UL subframes as the case may be. In this TP, k or m are generic subframe indices.

*** < Beginning of TP#1 for TS 36.213 V17.1.0> ***

16.6	Narrowband physical downlink control channel related procedures

*** < Unchanged parts are omitted> ***

For a NPDCCH UE-specific search space, if a NB-IoT UE is configured with higher layer parameter twoHARQ-ProcessesConfig or npusch-MultiTB-Config and if the NB-IoT UE detects NPDCCH with DCI Format N0 ending in subframe n, and if the corresponding NPUSCH format 1 transmission starts from an uplink subframe which, after accounting for uplink transmission timing, overlaps with downlink subframe n+k (accounting for uplink transmission timing),

-	if the corresponding NPDCCH with DCI format N0 with CRC scrambled by C-RNTI schedules two transport blocks as determined by the Number of scheduled TB for Unicast field if present, the UE is not required to monitor an NPDCCH candidate in any subframe starting from subframe n+1 to subframe n+k-1, otherwise the UE is not required to monitor an NPDCCH candidate in any subframe starting from subframe n+k-2 to subframe n+k-1; and

- the UE does not expect to receive a DCI Format N0 before subframe n+k-2 for which the corresponding NPUSCH format 1 transmission ends later than subframe n+k+255 if the corresponding NPDCCH with DCI format N0 schedules one transport block.

-	for TDD, and if the corresponding NPUSCH format1 transmission ends in subframe n+m, the UE is not required to monitor NPDCCH in any subframe starting from subframe n+ k to subframe n+m-1.

otherwise

-	if the NB-IoT UE detects NPDCCH with DCI Format N0 ending in subframe n or receives a NPDSCH carrying a random access response grant ending in subframe n, and if the corresponding NPUSCH format 1 transmission starts from an uplink subframe which, after accounting for uplink transmission timing, overlaps with downlink subframe n+k (accounting for uplink transmission timing), the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+k-1.

-	for TDD, if the NB-IoT UE detects NPDCCH with DCI Format N0 ending in subframe n or receives a NPDSCH carrying a random access response grant ending in subframe n, and if the corresponding NPUSCH format 1 transmission ends in n+k, the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+k.

*** < Unchanged parts are omitted> ***

If a NB-IoT UE detects NPDCCH with DCI Format N1 ending in subframe n, and if the corresponding NPDSCH transmission starts from n+k, and

-	for FDD, if the corresponding NPUSCH format 2 transmission starts from an uplink subframe which, after accounting for uplink transmission, timing overlaps with downlink subframe n+m (accounting for uplink transmission timing), the UE is not required to monitor NPDCCH in any subframe starting from subframe n+ k to subframe n+m-1.

-	for TDD, if the corresponding NPUSCH format 2 transmission ends in subframe n+m the UE is not required to monitor NPDCCH in any subframe starting from subframe n+ k to subframe n+m-1.

If a NB-IoT UE detects NPDCCH with DCI Format N1 for "PDCCH order" ending in subframe n, and

-	for FDD, if the corresponding NPRACH transmission starts from an uplink subframe which, after accounting for uplink transmission timing, overlaps with downlink subframe n+k (accounting for uplink transmission timing), the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+k-1.

-	for TDD, if the corresponding NPRACH transmission ends in subframe n+k, the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+k-1.

*** < Unchanged parts are omitted> ***

*** < End of TP#1 for TS 36.213 V17.1.0> ***

FL Proposal 2.1-1:

Suggest to spec editor the above TP#1 to section 16.6 of TS 36.213

| Company | Support/Not Support FL Proposal 2.1-1 | Comments and Proposal |
| --- | --- | --- |

## Issue# 2-2 & 2-3: NPDCCH Monitoring restrictions (cases 7-10)

There were discussions on cases 7 – 10 at previous RAN1 meetings but no conclusion was reached on whether to modify the designation of subframes with restrictions on NPDCCH monitoring. Accordingly, the spec editor has not modified these designations either. Furthermore, cases 7 – 10 cover Issue# 2-3 – NPDCCH monitoring post NPUSCH transmission. So Issues# 2-2 and 2-3 will be treated together.

### Companies Views

| Huawei | Proposal 3: For case 7~11, adopt TP#1 for Clause 16.6 of TS36.213 to ensure that the NPDCCH monitoring takes the timing offset between the UL and DL frame at the eNB into consideration to keep the current timing relationship of scheduling otherwise unnecessary NPDCCH monitoring at the UE side would be introduced. We provide the following TP#1 for the description in Clause 16.6 of TS36.213 v17.0.0 for case7~11. TP#1 for Clause 16.6 of TS36.213 ==============Unchanged Text Omitted ============================== If a NB-IoT UE is configured with higher layer parameter twoHARQ-ProcessesConfig - and if the UE has a NPUSCH transmission ending in subframe n, - the UE is not required to receive transmissions in the Type B half-duplex guard periods as specified in [3]for FDD ; and - the UE is not expected to receive an NPDCCH with DCI format N0/N1 for the same HARQ process ID as the NPUSCH transmission in any subframe starting from subframe n+1 to subframe n+kmac+3; else if the UE is not using higher layer parameter edt-Parameters or if the UE is using higher layer parameter edt- and - if the NB-IoT UE has a NPUSCH transmission ending in subframe n , the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+kmac+3. otherwise, - If the NB-IoT UE has a NPUSCH transmission for Msg3 ending in subframe n'with transport block size , whereas if would have been selected the NPUSCH transmission would have ended in subframe n, the UE is not required to monitor NPDCCH in any subframe starting from subframe n'+1 to subframe n+kmac+3. ===========Unchanged Text Omitted ========================== For an NB-IoT UE configured with higher layer parameter sr-WithoutHARQ-ACK-Config, if the transmission of a narrowband random access preamble for SR ends on subframe n, - in case of frame structure type 1 with NPRACH format 0 and 1 when the number of NPRACH repetitions is greater than or equal to 64, or NPRACH format 2 when the number of NPRACH repetitions is greater than or equal to 16, the UE is not required to monitor NPDCCH UE-specific search space from subframe n to subframe n+kmac+40, - otherwise, the UE is not required to monitor NPDCCH UE-specific search space from subframe n to subframe n+kmac+3. |
| --- | --- |
| Qualcomm | Proposal 2: Modify the specification for DL monitoring restrictions after NPUSCH transmission to account for uplink transmission timing. |
| MediaTek | Proposal 11: Agree on introduction of K_mac for NPDCCH monitoring cases 7-11 in TP#1 to TS 36.213 Section 16.6. Proposal 12: Agree on introduction of term S(TA) equals to ceil (TA) for NPDCCH monitoring cases 1-11 in TP#1 to TS 36.213 Section 16.6. -------------------- Start of TP #1 for 3GPP TS 36.213 --------------------------------- 36.213 Section 16.6 Narrowband physical downlink control channel related procedures <Unchanged Text Omitted> For a NPDCCH UE-specific search space, if a NB-IoT UE is configured with higher layer parameter twoHARQ-ProcessesConfig or npusch-MultiTB-Config and if the NB-IoT UE detects NPDCCH with DCI Format N0 ending in subframe n, and if the corresponding NPUSCH format 1 transmission starts from n+k(accounting for uplink transmission timing), - [case 1: MTBG NPUSCH] if the corresponding NPDCCH with DCI format N0 with CRC scrambled by C-RNTI schedules two transport blocks as determined by the Number of scheduled TB for Unicast field if present, the UE is not required to monitor an NPDCCH candidate in any subframe starting from subframe n+1 to subframe n+k-S(TA)-1, where S(TA) equals to TA; otherwise [case 2: 2 NPUSCH HARQ processes scheduled] the UE is not required to monitor an NPDCCH candidate in any subframe starting from subframe n+k-S(TA)-2 to subframe n+k-S(TA)-1, where S(TA) equals to TA; and - [case 3: long single NPUSCH when MTBG or 2HARQ configured] the UE does not expect to receive a DCI Format N0 before subframe n+k-S(TA)-2 for which the corresponding NPUSCH format 1 transmission ends later than subframe n+k-S(TA)+255 if the corresponding NPDCCH with DCI format N0 schedules one transport block, where S(TA) equals toTA. - for TDD, and if the corresponding NPUSCH format1 transmission ends in subframe n+m, the UE is not required to monitor NPDCCH in any subframe starting from subframe n+ k to subframe n+m-1. otherwise - [case 4: single NPUSCH scheduled by DCI format N0 or RAR]if the NB-IoT UE detects NPDCCH with DCI Format N0 ending in subframe n or receives a NPDSCH carrying a random access response grant ending in subframe n, and if the corresponding NPUSCH format 1 transmission starts from n+k(accounting for uplink transmission timing), the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+k-S(TA)-1, where S(TA) equals to TA. - for TDD, if the NB-IoT UE detects NPDCCH with DCI Format N0 ending in subframe n or receives a NPDSCH carrying a random access response grant ending in subframe n, and if the corresponding NPUSCH format 1 transmission ends in n+k, the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+k. For a NPDCCH UE-specific search space, if a NB-IoT UE is configured with higher layer parameter twoHARQ-ProcessesConfig or npdsch-MultiTB-Config - and if the NB-IoT UE detects NPDCCH with DCI Format N1 ending in subframe n, and if a NPDSCH transmission starts from n+k, - if the corresponding NPDCCH with DCI format N1 with CRC scrambled by C-RNTI schedules two transport blocks as determined by the Number of scheduled TB for Unicast field if present, the UE is not required to monitor an NPDCCH candidate in any subframe starting from subframe n+1 to subframe n+k-1; - otherwise, the UE is not required to monitor an NPDCCH candidate in any subframe starting from subframe n+k-2 to subframe n+k-1; otherwise - if the NB-IoT UE detects NPDCCH with DCI Format N1 or N2 ending in subframe n, and if the corresponding NPDSCH transmission starts from n+k, the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+k-1. If a NB-IoT UE detects NPDCCH with DCI Format N1 ending in subframe n, and if the corresponding NPDSCH transmission starts from n+k, and - [case 5: NPUSCH format 2 in response to DCI format N1] for FDD, if the corresponding NPUSCH format 2 transmission starts from subframe n+m(accounting for uplink transmission timing) the UE is not required to monitor NPDCCH in any subframe starting from subframe n+ k to subframe n+m-S(TA)-1, where S(TA) equals to TA. - for TDD, if the corresponding NPUSCH format 2 transmission ends in subframe n+m the UE is not required to monitor NPDCCH in any subframe starting from subframe n+ k to subframe n+m-1. If a NB-IoT UE detects NPDCCH with DCI Format N1 for "PDCCH order" ending in subframe n, and - [case 6: NPRACH in response to PDCCH order] for FDD, if the corresponding NPRACH transmission starts from subframe n+k(accounting for uplink transmission timing), the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+k-S(TA)-1, where S(TA) equals to TA. - for TDD, if the corresponding NPRACH transmission ends in subframe n+k, the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+k-1. If a NB-IoT UE is configured with higher layer parameter twoHARQ-ProcessesConfig - and if the UE has a NPUSCH transmission ending in subframe n, - the UE is not required to receive transmissions in the Type B half-duplex guard periods as specified in [3]for FDD ; and - [case 7: NPUSCH with same HARQ process when 2 HARQ configured] the UE is not expected to receive an NPDCCH with DCI format N0/N1 for the same HARQ process ID as the NPUSCH transmission in any subframe starting from subframe n-S(TA)+1 to subframe n+ Kmac +3, where S(TA) equals to TA; else if the UE is not using higher layer parameter edt-Parameters or if the UE is using higher layer parameter edt-Parameters and - [case 8: subframes after NPUSCH processing] if the NB-IoT UE has a NPUSCH transmission ending in subframe n , the UE is not required to monitor NPDCCH in any subframe starting from subframe n-S(TA)+1 to subframe n+ Kmac +3, where S(TA) equals to TA. otherwise, - [case 9: subframes after NPUSCH carrying Msg3] If the NB-IoT UE has a NPUSCH transmission for Msg3 ending in subframe n'with transport block size , whereas if would have been selected the NPUSCH transmission would have ended in subframe n, the UE is not required to monitor NPDCCH in any subframe starting from subframe n'-S(TA)+1 to subframe n + Kmac +3, where S(TA) equals to TA. If a NB-IoT UE receives a NPDSCH transmission ending in subframe n, and if the UE is not required to transmit a corresponding NPUSCH format 2, the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+12. If a NB-IoT UE is configured with higher layer parameter twoHARQ-ProcessesConfig - the UE is not required to monitor an NPDCCH candidate of an NPDCCH search space if the candidate ends in subframe n, and if the UE is configured to monitor NPDCCH candidates of another NPDCCH search space having starting subframe k0 before subframe n+5 otherwise - the UE is not required to monitor NPDCCH candidates of an NPDCCH search space if an NPDCCH candidate of the NPDCCH search space ends in subframe n, and if the UE is configured to monitor NPDCCH candidates of another NPDCCH search space having starting subframe k0 before subframe n+5. An NB-IoT UE is not required to monitor NPDCCH candidates of an NPDCCH search space during an NPUSCH UL gap. An NB-IoT UE is not required to monitor NPDCCH candidates of a Type2A-NPDCCH common search space during the scheduling gap or the processing gap. For an NB-IoT UE configured with higher layer parameter sr-WithoutHARQ-ACK-Config, if the transmission of a narrowband random access preamble for SR ends on subframe n, - [case 10: NPRACH for SR for long NPRACH transmissions] in case of frame structure type 1 with NPRACH format 0 and 1 when the number of NPRACH repetitions is greater than or equal to 64, or NPRACH format 2 when the number of NPRACH repetitions is greater than or equal to 16, the UE is not required to monitor NPDCCH UE-specific search space from subframe n-S(TA) to subframe n+ Kmac+40, where S(TA) equals to TA - otherwise, [case 11: NPRACH for SR for short NPRACH transmissions] the UE is not required to monitor NPDCCH UE-specific search space from subframe n-S(TA) to subframe n+ Kmac +3, where S(TA) equals to TA. <Unchanged Text Omitted> ---------------------------------------- End of TP #1 for 3GPP TS 36.213 ----------------------------------------- |
| Sony | Observation: No specification changes are required for cases 7-11 of the “NPDCCH monitoring restrictions”. |

### FIRST ROUND Discussion of Issue# 2-2 & 2-3: NPDCCH Monitoring restrictions (case 7-10)

The companies who think there is a need to modify the designation of subframes for restricted NPDCCH monitoring in cases 7 – 11 raise the issue mostly to ensure that the UE does not monitor subframes for NPDCCH unnecessarily especially in the case when UL and DL frames are not aligned. There is also the added issue of whether to adopt the logical time description approach in describing such subframes. Given the proposal in Issue 2-1, FL will also modify the TPs to be consistent with this logical time description.

In this TP, when the original text indexes a UL subframe, for example:

“if the UE has a NPUSCH transmission ending in subframe n”,

The coincident DL subframe is described with respect to this index as:

“downlink subframe that overlaps with uplink subframe n”

This allows the indexing in the rest of the text to be logical always and refer generally to DL subframes since it is these that need monitoring. In this TP, k or n’ are generic subframe indices.

*** < Beginning of TP#2 for Clause 16.6 of TS 36.213 V17.1.0> ***

==============Unchanged Text Omitted ==============================

If a NB-IoT UE is configured with higher layer parameter twoHARQ-ProcessesConfig

-	and if the UE has a NPUSCH transmission ending in subframe n,

-	the UE is not required to receive transmissions in the Type B half-duplex guard periods as specified in [3]for FDD ; and

-	the UE is not expected to receive an NPDCCH with DCI format N0/N1 for the same HARQ process ID as the NPUSCH transmission in any downlink subframe starting from that overlaps with uplink subframe n+1 to subframe n+ Kmac +3;

else if the UE is not using higher layer parameter edt-Parameters or if the UE is using higher layer parameter edt- and

-	if the NB-IoT UE has a NPUSCH transmission ending in subframe n , the UE is not required to monitor NPDCCH in any downlink subframe starting from that overlaps with uplink subframes n+1 to subframe n+ Kmac +3.

otherwise,

-	If the NB-IoT UE has a NPUSCH transmission for Msg3 ending in subframe n'with transport block size , whereas if would have been selected the NPUSCH transmission would have ended in subframe n, the UE is not required to monitor NPDCCH in any downlink subframe starting from that overlaps with uplink subframe n'+1 to subframe n+ Kmac +3.

===========Unchanged Text Omitted ==========================

For an NB-IoT UE configured with higher layer parameter sr-WithoutHARQ-ACK-Config, if the transmission of a narrowband random access preamble for SR ends on subframe n,

-	in case of frame structure type 1 with NPRACH format 0 and 1 when the number of NPRACH repetitions is greater than or equal to 64, or NPRACH format 2 when the number of NPRACH repetitions is greater than or equal to 16, the UE is not required to monitor NPDCCH UE-specific search space in the downline subframes that overlap with uplink from subframe n to subframe n+ Kmac +40,

-	otherwise, the UE is not required to monitor NPDCCH UE-specific search space in the downline subframes that overlap with uplink from subframe n to subframe n+ Kmac +3.

NOTE: if higher layer parameter K-Mac is configured, Kmac = K-Mac otherwise, Kmac =  0.

*** < End of TP#2 for Clause 16.6 of TS 36.213 V17.1.0> ***

FL Proposal 2.2-1:

Suggest to spec editor the above TP#2 to section 16.6 of TS 36.213

| Company | Support/Not Support FL Proposal 2.2-1 | Comments and Proposal |
| --- | --- | --- |

## Issue# 2-5: Inclusion of Calculation of UE-eNB RTT in Spec

At RAN1#108e, the following agreement was made:

Agreement

For IoT NTN, calculate UE-eNB RTT using the following equation: RTTUEeNB=TTATf+Kmac

where Tf = subframe duration (1ms).

Companies contributing on this issue assert that this agreement be reflected in the specification.

### Companies Views

| MediaTek | Proposal 13: Agree on the capturing RAN1#108e agreement on RTTUEeNB=TTATf+Kmac in TP #2 and TP #3 to TS 36.213 Sections 6.1, 16.3.1. ---------------------------Start of TP #2 for 3GPP TS 36.213 ----------------------------- 36.213 Section 6.1 <Unchanged Text Omitted> - For BL/CE UEs, detection of a MPDCCH with DCI scrambled by RA-RNTI is attempted during a window controlled by higher layers (see [8], Clause 5.1.4), where UE-eNB RTT is calculated as floor(TTATf)+Kmac subframes, where TTA is specified in [TS 36.211, Clause 8.1], Tf is the subframe duration (1ms), and Kmac is provided by the higher layer parameter K-Mac in unit of 1 ms or Kmac=0 if K-Mac is not provided. If detected, the corresponding DL-SCH transport block is passed to higher layers. The higher layers parse the transport block and indicate the Nr-bit uplink grant to the physical layer, which is processed according to Clause 6.2. <Unchanged Text Omitted> ------------------------------End of TP #2 for 3GPP TS 36.213---------------------------- ------------------------------Start of TP #3 for 3GPP TS 36.213 -------------------------- 36.213 section 16.3.1 <Unchanged Text Omitted> - Detection of a NPDCCH with DCI scrambled by RA-RNTI is attempted during a window controlled by higher layers (see [8], Clause 5.1.4), where UE-eNB RTT is calculated as floor(TTATf)+Kmac subframes, where TTA is specified in [TS 36.211, Clause 8.1], Tf is the subframe duration (1ms), and Kmac is provided by the higher layer parameter K-Mac in unit of 1 ms or Kmac=0 if K-Mac is not provided. If detected, the corresponding DL-SCH transport block is passed to higher layers. The higher layers parse the transport block and indicate the Nr-bit uplink grant to the physical layer, which is processed according to Clause 16.3.3. <Unchanged Text Omitted> ----------------End of TP #3 for 3GPP TS 36.213----------------------------------- |
| --- | --- |

### FIRST ROUND Discussion of Issue# 2-5: Inclusion of calculation UE-eNB RTT in spec

FL makes the following proposal based on the MediaTek TPs:

---------------------------Start of TP #3 for 3GPP TS 36.213 -----------------------------

36.213 Section 6.1

<Unchanged Text Omitted>

-     For BL/CE UEs, detection of a MPDCCH with DCI scrambled by RA-RNTI is attempted during a window controlled by higher layers (see [8], Clause 5.1.4), where UE-eNB RTT is calculated as floor(TTATf)+Kmac subframes, where TTA is specified in [TS 36.211, Clause 8.1], Tf is the subframe duration (1ms), and Kmac is provided by the higher layer parameter K-Mac in unit of 1 ms or Kmac=0 if K-Mac is not provided. If detected, the corresponding DL-SCH transport block is passed to higher layers. The higher layers parse the transport block and indicate the Nr-bit uplink grant to the physical layer, which is processed according to Clause 6.2.

<Unchanged Text Omitted>

------------------------------End of TP #3 for 3GPP TS 36.213----------------------------

------------------------------Start of TP #4 for 3GPP TS 36.213 --------------------------

36.213 section 16.3.1

<Unchanged Text Omitted>

-     Detection of a NPDCCH with DCI scrambled by RA-RNTI is attempted during a window controlled by higher layers (see [8], Clause 5.1.4), where UE-eNB RTT is calculated as floor(TTATf)+Kmac subframes, where TTA is specified in [TS 36.211, Clause 8.1], Tf is the subframe duration (1ms), and Kmac is provided by the higher layer parameter K-Mac in unit of 1 ms or Kmac=0 if K-Mac is not provided. If detected, the corresponding DL-SCH transport block is passed to higher layers. The higher layers parse the transport block and indicate the Nr-bit uplink grant to the physical layer, which is processed according to Clause 16.3.3.

<Unchanged Text Omitted>

----------------End of TP #4 for 3GPP TS 36.213-----------------------------------

FL Proposal 2.3-1:

Suggest to spec editor the above TP#3 to section 6.1 and TP#4 to section 16.3.1 of TS 36.213

| Company | Support/Not Support FL Proposal 2.3-1: | Comments and Proposal |
| --- | --- | --- |

## Issue# 2-6: Maintain Legacy behaviour in spec for TDD mode

Ericsson argues that since Rel17 IoT NTN WID was focused only on FDD, legacy TDD clauses of the specification should not be affected by changes to the specification to support IoT NTN.

### Companies Views

| Ericsson | Observation 2: Since the Rel-17 IoT NTN WI has focused exclusively on FDD, the legacy specification text for TDD should not be changed. Proposal 8: Adopt the following TP for TS 36.213 Clause 16.4.2: -------------------- Start of TP for 3GPP TS 36.213 -------------------- 16.4.2 UE procedure for reporting ACK/NACK <Unchanged Text Omitted> The UE shall upon detection of a NPDSCH transmission ending in NB-IoT subframe n intended for the UE and for which an ACK/NACK shall be provided, start, after the end of - n+k0'+Koffset-1 DL subframe for FDD if Koffset is not configured, - n+k0'+Koffset-1 UL subframe for FDD if Koffset is configured, - k0'-1 NB-IoT UL subframes following the end of n+12+Koffset subframe for TDD, transmission of the NPUSCH carrying ACK/NACK response, and SR (if any) if the serving cell is FDD and the UE is configured with higher layer parameter sr-with-HARQ-ACK-Config, using NPUSCH format 2 in N consecutive NB-IoT UL slots, … -------------------- End of TP for 3GPP TS 36.213 -------------------- Proposal 9: Adopt the following proposals for TS 36.213 Clause 10.2: --------------------------------------------3GPP TS 36.213 -------------------------------------------- 16.5.1 UE procedure for transmitting format 1 narrowband physical uplink shared channel <Unchanged Text Omitted> A UE shall upon detection on a given serving cell of a NPDCCH with DCI format N0 ending in NB-IoT DL subframe n scheduling NPUSCH intended for the UE, perform, at the end of - n+k0 + Koffset DL subframe for FDD if Koffset is not configured, - n+k0+Koffset UL subframe for FDD if Koffset is configured, - k0 NB-IoT UL subframes following the end of n+8+ Koffset subframe for TDD, a corresponding NPUSCH transmission using NPUSCH format 1 in N consecutive NB-IoT UL slots ni with i = 0, 1, …, N-1 according to the NPDCCH information where - subframe n is the last subframe in which the NPDCCH is transmitted and is determined from the starting subframe of NPDCCH transmission and the DCI subframe repetition number field in the corresponding DCI; and - , where the value of is determined by the repetition number field in the corresponding DCI (see Clause 16.5.1.1), the value of is determined by the resource assignment field in the corresponding DCI (see Clause 16.5.1.1), the value of is the number of NB-IoT UL slots of the resource unit (defined in clause 10.1.2.3 of [3]) corresponding to the allocated number of subcarriers (as determined in Clause 16.5.1.1) in the corresponding DCI, and the value of is determined by the Number of scheduled TB for Unicast field, if present, in the corresponding DCI, otherwise - n0 is the first NB-IoT UL slot starting after the end of subframe n+k0+ Koffset for FDD if Koffset is not configured - n0 is the first NB-IoT UL slot starting after the end of UL subframe n+k0+Koffset for FDD if Koffset is configured - n0 is the first NB-IoT UL slot starting after k0 NB-IoT UL subframes following the end of n+8+ Koffset subframe for TDD --------------------------------------------3GPP TS 36.213 -------------------------------------------- Adopt the following proposals for TS 36.213 Clause 10.2: --------------------------------------------3GPP TS 36.213 -------------------------------------------- 10.2 Uplink HARQ-ACK timing <Unchanged Text Omitted> For TDD, a BL/CE UE shall upon detection of a PDSCH within subframe(s) n-k+ Koffset , where and is defined in Table 10.1.3.1-1 intended for the UE and for which HARQ-ACK response shall be provided, transmit the HARQ-ACK response using the same derived according to Clause 10.1.3.1 in subframe(s) n+ki with i =0,1, …, N-1, where - subframe n-k- Koffset is the last subframe in which the PDSCH is transmitted; and - 0≤k0<k1<…,kN-1 and the value of and is provided by higher layers parameter pucch-NumRepetitionCE-format1, if configured, otherwise it is provided by higher layer parameter pucch-NumRepetitionCE-Msg4-Level0-r13, pucch-NumRepetitionCE-Msg4-Level1-r13, pucch-NumRepetitionCE-Msg4-Level2-r13 or pucch-NumRepetitionCE-Msg4-Level3-r13 depending on whether the most recent PRACH coverage enhancement level for the UE is 0, 1, 2 or 3, respectively; and if N>1 - subframe(s) n+ki with i=0,1,…,N-1 are N consecutive BL/CE UL subframe(s) immediately after subframe n-1, and the set of BL/CE UL subframes are configured by higher layers; otherwise - k0 =0 except if the UE is configured with higher layer parameter ce-PDSCH-MultiTB-Config and multiple TB are scheduled in the corresponding DCI. --------------------------------------------3GPP TS 36.213 -------------------------------------------- --------------------------------------------3GPP TS 36.213 -------------------------------------------- 10.2 Uplink HARQ-ACK timing <Unchanged Text Omitted> For TDD, if a BL/CE UE is configured with higher layer parameter ce-PDSCH-MultiTB-Config and multiple TBs are scheduled in the corresponding DCI, the BL/CE UE shall upon detection of a PDSCH intended for the UE and for which HARQ-ACK response shall be provided, transmit the HARQ-ACK response using the same derived according to Clause 10.1.3.1 in subframe(s) sb+ki+ Koffset with b=0,1,…,B-1, i =0,1, …, N-1, where - B is the number of TB bundles - if the UE is not configured with higher layer parameter harq-AckBundling in ce-PDSCH-MultiTB-Config, B=NTB with bundle b consisting of only TB b. - Else, the value of B and the corresponding TBs in each bundle is determined according to clause 7.3 - is the number of scheduled TB determined in the corresponding DCI; - s0=n0+4, sb=max {nb+4, sb-1+Nb-1}, b≠0 - nb is the last subframe in which the PDSCH containing TB bundle b is transmitted; - Nb denotes the number of consecutive subframes including subframes that are not BL/CE UL subframes where the PUCCH with HARQ ACK for TB bundle b with repetition number of N is transmitted; and - 0≤k0<k1<…,kN-1 and the value of and is provided by higher layers parameter pucch-NumRepetitionCE-format1, if configured, otherwise it is provided by higher layer parameter pucch-NumRepetitionCE-Msg4-Level0-r13, pucch-NumRepetitionCE-Msg4-Level1-r13, pucch-NumRepetitionCE-Msg4-Level2-r13 or pucch-NumRepetitionCE-Msg4-Level3-r13 depending on whether the most recent PRACH coverage enhancement level for the UE is 0, 1, 2 or 3, respectively; and - subframe(s) sb+ki+ Koffset with i=0,1,…,N-1 for TB bundle b are N consecutive BL/CE UL subframe(s) immediately after subframe sb-1+ Koffset , and the set of BL/CE UL subframes are configured by higher layers. --------------------------------------------3GPP TS 36.213 -------------------------------------------- |
| --- | --- |

### FIRST ROUND Discussion on Issue# 2-6: Maintain Legacy behaviour in spec for TDD mode

It seems to FL that the Ericsson position is perfectly reasonable. TDD was not considered for the IoT NTN WI and so TDD should not be specified for IoT NTN. Secondly, changes to the specification to support IoT NTN should not inadvertently change the specification for terrestrial NB-IoT and eMTC. The mechanism Ericsson has used to differentiate between an NTN and TN in the proposed TPs, is to test whether Koffset is configured – which it would typically be for NTN but not for TN.

FL proposes 3 surveys:

- A survey to find out company views regarding the non-support of TDD for IoT NTN in Rel17 specifications

- Adequate means to differentiate between NTN and TN in the specification

- Adequacy of TPs to achieve the first two aims.

FL Survey 2.4-1a:

Do you agree that TDD for IoT NTN should not be supported in the specifications?

| Company | Yes/No FL Survey 2.4-1a: | Comments and Proposal |
| --- | --- | --- |

FL Survey 2.4-1b:

Do you agree that the presence/absence of configuration for Koffset is a suitable and adequate means for differentiating between NTN and TN in the specification?

| Company | Yes/No FL Survey 2.4-1b: | Comments and Proposal |
| --- | --- | --- |

Here are the TPs adapted from Ericsson contribution.

-------------------- Start of TP#5 for 3GPP TS 36.213 --------------------

16.4.2	UE procedure for reporting ACK/NACK

<Unchanged Text Omitted>

The UE shall upon detection of a NPDSCH transmission ending in NB-IoT subframe n intended for the UE and for which an ACK/NACK shall be provided, start, after the end of

-	n+k0'+Koffset-1 DL subframe for FDD if Koffset is not configured,

-	n+k0'+Koffset-1 UL subframe for FDD if Koffset is configured,

-	k0'-1 NB-IoT UL subframes following the end of n+12+Koffset subframe for TDD,

transmission of the NPUSCH carrying ACK/NACK response, and SR (if any) if the serving cell is FDD and the UE is configured with higher layer parameter sr-with-HARQ-ACK-Config, using NPUSCH format 2 in N consecutive NB-IoT UL slots, …

-------------------- End of TP#5 for 3GPP TS 36.213 --------------------

-------------------- Start of TP#6 for 3GPP TS 36.213 --------------------

16.4.2	UE procedure for reporting ACK/NACK

<Unchanged Text Omitted>

The UE shall upon detection of a NPDSCH transmission ending in NB-IoT subframe n intended for the UE and for which an ACK/NACK shall be provided, start, after the end of

-	n+k0'+Koffset-1 DL subframe for FDD if Koffset is not configured,

-	n+k0'+Koffset-1 UL subframe for FDD if Koffset is configured,

-	k0'-1 NB-IoT UL subframes following the end of n+12+Koffset subframe for TDD,

transmission of the NPUSCH carrying ACK/NACK response, and SR (if any) if the serving cell is FDD and the UE is configured with higher layer parameter sr-with-HARQ-ACK-Config, using NPUSCH format 2 in N consecutive NB-IoT UL slots, …

-------------------- End of TP#6 for 3GPP TS 36.213 --------------------

-------------------- Start of TP#7 for 3GPP TS 36.213 --------------------

16.5.1	UE procedure for transmitting format 1 narrowband physical uplink shared channel

<Unchanged Text Omitted>

A UE shall upon detection on a given serving cell of a NPDCCH with DCI format N0 ending in NB-IoT DL subframe n scheduling NPUSCH intended for the UE, perform, at the end of

-	n+k0 + Koffset DL subframe for FDD if Koffset is not configured,

-	n+k0+Koffset UL subframe for FDD if Koffset is configured,

-	k0 NB-IoT UL subframes following the end of n+8+ Koffset subframe for TDD,

a corresponding NPUSCH transmission using NPUSCH format 1 in N consecutive NB-IoT UL slots ni with i = 0, 1, …, N-1 according to the NPDCCH information where

-	subframe n is the last subframe in which the NPDCCH is transmitted and is determined from the starting subframe of NPDCCH transmission and the DCI subframe repetition number field in the corresponding DCI; and

-	, where the value of is determined by the repetition number field in the corresponding DCI (see Clause 16.5.1.1), the value of is determined by the resource assignment field in the corresponding DCI (see Clause 16.5.1.1), the value of  is the number of NB-IoT UL slots of the resource unit (defined in clause 10.1.2.3 of [3]) corresponding to the  allocated number of subcarriers (as determined in Clause 16.5.1.1) in the corresponding DCI, and the value of is determined by the Number of scheduled TB for Unicast field, if present, in the corresponding DCI,  otherwise

-	n0 is the first NB-IoT UL slot starting after the end of subframe n+k0+ Koffset for FDD if Koffset is not configured

-	n0 is the first NB-IoT UL slot starting after the end of UL subframe n+k0+Koffset for FDD if Koffset is configured

-	n0 is the first NB-IoT UL slot starting after k0 NB-IoT UL subframes following the end of n+8+ Koffset subframe for TDD

-------------------- End of TP#7 for 3GPP TS 36.213 --------------------

-------------------- Start of TP#8 for 3GPP TS 36.213 --------------------

10.2	Uplink HARQ-ACK timing

<Unchanged Text Omitted>

For TDD, a BL/CE UE shall upon detection of a PDSCH within subframe(s) n-k+ Koffset , where  and  is defined in Table 10.1.3.1-1 intended for the UE and for which HARQ-ACK response shall be provided, transmit the HARQ-ACK response using the same  derived according to Clause 10.1.3.1 in subframe(s) n+ki with i =0,1, …, N-1, where

-	subframe n-k- Koffset is the last subframe in which the PDSCH is transmitted; and

-	0≤k0<k1<…,kN-1 and the value of and  is provided by higher layers parameter pucch-NumRepetitionCE-format1, if configured, otherwise it is provided by higher layer parameter pucch-NumRepetitionCE-Msg4-Level0-r13, pucch-NumRepetitionCE-Msg4-Level1-r13, pucch-NumRepetitionCE-Msg4-Level2-r13 or pucch-NumRepetitionCE-Msg4-Level3-r13 depending on whether the most recent PRACH coverage enhancement level for the UE is 0, 1, 2 or 3, respectively; and

if N>1

-	subframe(s) n+ki with i=0,1,…,N-1 are N consecutive BL/CE UL subframe(s) immediately after subframe n-1, and the set of BL/CE UL subframes are configured by higher layers;

otherwise

-	k0 =0

except if the UE is configured with higher layer parameter ce-PDSCH-MultiTB-Config and multiple TB are scheduled in the corresponding DCI.

-------------------- End of TP#8 for 3GPP TS 36.213 --------------------

-------------------- Start of TP#9 for 3GPP TS 36.213 --------------------

10.2	Uplink HARQ-ACK timing

<Unchanged Text Omitted>

For TDD, if a BL/CE UE is configured with higher layer parameter ce-PDSCH-MultiTB-Config and multiple TBs are scheduled in the corresponding DCI, the BL/CE UE shall upon detection of a PDSCH intended for the UE and for which HARQ-ACK response shall be provided, transmit the HARQ-ACK response using the same  derived according to Clause 10.1.3.1 in subframe(s) sb+ki+ Koffset  with b=0,1,…,B-1,  i =0,1, …, N-1, where

-	B is the number of TB bundles

-	if the UE is not configured with higher layer parameter harq-AckBundling in ce-PDSCH-MultiTB-Config, B=NTB with bundle b consisting of only TB b.

-	Else, the value of B and the corresponding TBs in each bundle is determined according to clause 7.3

-	is the number of scheduled TB determined in the corresponding DCI;

-	s0=n0+4, sb=max {nb+4,  sb-1+Nb-1}, b≠0

-	nb is the last subframe in which the PDSCH containing TB bundle b is transmitted;

-	Nb denotes the number of consecutive subframes including subframes that are not BL/CE UL subframes where the PUCCH with HARQ ACK for TB bundle b with repetition number of N is transmitted;

and

-	0≤k0<k1<…,kN-1 and the value of and  is provided by higher layers parameter pucch-NumRepetitionCE-format1, if configured, otherwise it is provided by higher layer parameter pucch-NumRepetitionCE-Msg4-Level0-r13, pucch-NumRepetitionCE-Msg4-Level1-r13, pucch-NumRepetitionCE-Msg4-Level2-r13 or pucch-NumRepetitionCE-Msg4-Level3-r13 depending on whether the most recent PRACH coverage enhancement level for the UE is 0, 1, 2 or 3, respectively; and

-	subframe(s) sb+ki+ Koffset  with i=0,1,…,N-1 for TB bundle b are N consecutive BL/CE UL subframe(s) immediately after subframe sb-1+ Koffset , and the set of BL/CE UL subframes are configured by higher layers.

-------------------- End of TP#9 for 3GPP TS 36.213 --------------------

FL Survey 2.4-1c:

Do you support TP#5 to TP#9 meant to achieve the above?

| Company | Yes/No FL Survey 2.4-1c: | Comments and Proposal |
| --- | --- | --- |

# Referenced Documents

R1-2203089	Maintenance on NB-IoT/eMTC support for Non-Terrestrial Network	Huawei, HiSilicon

R1-2203386	Maintenance on NB-IoT/eMTC to support NTN	MediaTek Inc.

R1-2205110	" Moderator Summary for preparation phase on maintenance of Rel-17 WI on NB-IoT/eMTC support for Non-Terrestrial Network ", Moderator (MediaTek Inc.)

R1-2203632	On IoT NTN maintenance issues	Ericsson Limited

R1-2203722	Maintenance of IoT-NTN	Sony

R1-2203991	Discussion on remaining issues for NB-IOT/eMTC NTN	OPPO

R1-2204997	Maintenance on IoT-NTN	Qualcomm Incorporated