3GPP TSG RAN WG1 #109-e                                                   R1-2205201

e-Meeting, May 9th – May 20th, 2022

Source: 	Moderator (Nokia)

Title: 	Feature Lead Summary #1 for Maintenance of multipath/NLOS mitigation

Agenda item:	8.5.1

Document for: 	Discussion and Decision

# Introduction

In the WID, [1], for ePos the following objective was added at RAN#91 and was completed:

- Study and specify, if agreed, the enhancements of information reporting from UE and gNB for multipath/NLOS mitigation [RAN1, RAN2, RAN3]

In this contribution, we provide a summary of the maintenance for information reporting from UE and gNB for multipath/NLOS mitigation proposed by companies in contributions [2]-[4]. We also make some initial proposals to facilitate RAN1 discussion. This document also provides the summary of the following email discussion in RAN1#109-e:

[109-e-R17-ePos-04] Email discussion under 8.5.1 for maintenance on enhancements of information reporting from UE and gNB for multipath/NLOS mitigation, for issue 4-1 in R1-2205097 – Ryan (Nokia)

- 1st check point: May 13 (any RRC impact by May 12)

- Final check point: May 18

Issues for discussion

## Issue #4-1: LoS/NLoS Indicator Component Values

One issue discussed companies as summarized in [2] is:

- LOS/NLOS indicator component values for UE capability signaling.

Individual companies bringing proposals on this topic are:

- [3]:

  - Proposal 5: For UE’s capability to support reporting LoS/NLoS indicator

    - Support component 1 candidate value {hard value, both hard value and soft value}

- [4]:

  - Proposal 4: A UE capability supporting both soft and hard value reporting for LoS/NLoS indication is not supported

From the WI phase the relevant agreement was:

Agreement

- Support the following two options of values for LoS/NLoS indicator reporting from UE/TRP:

  - Soft values: [0, 0.1, …, 0.9, 1] (in steps of 0.1)

  - Hard values: [0, 1]

- The values correspond to the likelihood of LoS, with a value of 1 corresponding to LoS and a value of 0 corresponding to NLoS

### Round #1 Discussion (CLOSED)

Feature Lead View

This topic was discussed quite extensively for the last few meetings. Initial proposal for discussion

Proposal 4.1-A

Select one of the following alternatives to support for UE capability reporting of LOS/NLOS indicator values:

- Alt 1: Support candidate values of {hard value, both hard value and soft value}

- Alt 2: Support candidate values of {hard value, soft value}

Companies views:

| Company Name | Comments |
| --- | --- |
| Qualcomm | - Alt. 1 |
| vivo | Alt. 1 |
| ZTE | Alt. 1 BTW, the tdoc agenda should be 8.5.1. |
| Huawei, HiSilicon | Alt. 1 |
| Nokia/NSB | Alt. 1 |
| CATT | Alt. 1 |
| InterDigital | Alt. 2, but if the majority supports Alt. 1, can accept Alt. 1. For Alt. 1, it may be good to add the following note for clarification for “both hard value and soft value”. The note was added in the proposal by the feature lead in RAN1#108e. - Note: In Alt. 1., “both hard value and soft value” does not imply that the UE can report both hard and soft values for the same association (e.g., PRS resource) in the same time stamp. |
| FL | This issue was concluded during the UE features GTW session on ePos so we can close this issue. Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown 27. NR_pos_enh; 27-4-1; LOS/NLOS Indicator for UE-assisted positioning; 1. Support reporting LoS/NLoS indicator type to LMF 2. LOS/NLOS indicator granularity; one of 13-5,13-6, or 13-11; No; LOS/NLOS Indicator for UE-assisted positioning is not supported; Per UE; n/a No; n/a No; n/a No; [Component 1 candidate values: {hard value, hard+soft value[, both]}] Component 2 candidate values: {trpSpecific, resourceSpecific[, both]} [Note: a single value is reported when both multi-RTT and DL-TDOA are supported] FFS: signalling per method Need for location server to know if the feature is supported; Optional with capability signaling. |

Conclusion

In this contribution, we provided a review of the submitted contributions for NR Positioning on maintenance of information reporting from UE and gNB for multipath/NLOS mitigation and prepared an initial set of proposals to facilitate further discussion/decision by RAN1 during the RAN1#109–e meeting.

Outcome:

During the ePos UE features session the following agreement was reached:

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-4-1 | LOS/NLOS Indicator for UE-assisted positioning | 1. Support reporting LoS/NLoS indicator type to LMF 2. LOS/NLOS indicator granularity | one of 13-5,13-6, or 13-11 | No |  | LOS/NLOS Indicator for UE-assisted positioning is not supported | Per UE | n/a No | n/a No | n/a No | [Component 1 candidate values: {hard value, hard+soft value[, both]}] Component 2 candidate values: {trpSpecific, resourceSpecific[, both]} [Note: a single value is reported when both multi-RTT and DL-TDOA are supported] FFS: signalling per method Need for location server to know if the feature is supported | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

References

- RP-210903, Revised WID on NR Positioning Enhancements, CATT, Intel Corporation, Ericsson.

- R1-2205097, Moderator Summary for preparation phase on maintenance of Rel-17 WI on NR positioning enhancements, Moderator (CATT).

- R1-2203515	 Maintenance on accuracy improvements for NR positioning enhancements, vivo

- R1-2204127 Remaining issues for accuracy enhancements for NR positioning, InterDigital, Inc.