3GPP TSG RAN WG1 #109-e			     R1-2205211

e-Meeting, May 9th – 20th, 2022

Source:	Moderator (NTT DOCOMO, INC.)

Title:	Summary #2 on 9.12.1 Coverage enhancement for NR NTN

Agenda Item:	9.12.1

Document for: 	Discussion and Decision

- Introduction

[109-e-R18-NTN-01] Email discussion on coverage enhancement for NR NTN by May 20 – Shohei (Docomo) - Check points: May 16, May 20

Based on the above guidance from chairman, this documents are made for discussion on coverage enhancement for NR NTN. Schedule for discussion is below. FL requests companies to input based on the schedule (but might be updated based on progress and GTW session).

- 1st GTW (presentation / Q&A): May 9th Monday

- 1st round: ~ UTC 23:59 May 10th

- 2nd round: ~ UTC 16:59: May 12th

- (Request to declare agreements, if possible)

- Quiet period: UTC 23:59 May 13th – UTC 23:59 May 15th

- 3rd round: ~ UTC 07:00 23:59 May 16th

- 1st check point (checked by chair): UTC 16:59 May 16th

- 2nd GTW: May 17th Tuesday

- 4th round: ~ UTC 23:59 May 18th

- Email discussion: UTC 4:00 – UTC 16:59 May 20th

- 3rd GTW: May 20th Friday

- Final check point (checked by chair): UTC 16:59 May 20th

This topic is mentioned in Rel-18 NR NTN WID as captured in Appendix-1. In RAN1#109-e and RAN1#110, coverage performance under Rel-15/16/17 specifications is evaluated and whether NTN-specific techniques for coverage enhancement is necessary or not is concluded. FL assumes the following study plan for these purposes. It is noted that any perspective of technical enhancement will be discussed after the above study, if the necessity is clarified, though this perspective is briefly summarized in 6.13.

- RAN1#109-e: Agreements to have the same assumption for simulation evaluation

  - Target data rate

  - Evaluation methodology / Performance metrics

  - Link budget calculation

  - Satellite orbit

  - UE characteristics

  - Band / Bandwidth

  - Parameter set

  - Study cases

  - Targeted channels

  - Other assumptions

- RAN1#110: Agreements to list simulation results and have a conclusion on this study phase

  - Baseline performance: evaluation results list / analysis

  - Conclusion: whether NTN-specific technique is specified or not

FL requests companies to aim to agree all assumptions for simulation evaluation in this meeting. Otherwise, simulation evaluations by companies before RAN1#110 meeting are performed based on different assumptions and as the result, it would be the case that RAN1 cannot have appropriate conclusion in RAN1#110. In other words, we need to have sufficient agreements via email since GTW sessions for this topic might not be so many. This is why the above schedule is a bit tight.

In addition, FL requests companies to input contact information in section 5. The box is prepared based on chair. This meeting is the first one for Rel-18; it would be helpful for companies to know who join this topic, e.g. in order to have offline discussion.

- Collections of agreements/conclusions in RAN1#109-e

Email, May 12th

Agreement

For NR NTN coverage enhancement, evaluate only handset terminals as UE type.

- i.e., VSAT is not considered.

Email, May 17th

Agreement

Coverage performance in NR NTN is evaluated according to the following steps.

- Step 1: CNR is calculated as defined in 6.1.3.1 of TR38.821

  - For polarization loss,

    - 3 dB polarization loss is assumed as baseline, and companies are encouraged to report the value and corresponding justification if other value is used

- Step 2: Required SNR of target service is evaluated by LLS

- Step 3: The CNR and the required SNR are compared

Agreement

Coverage performance in NR NTN is evaluated for GEO/LEO-1200/LEO-600 scenarios.

- Note: Service type for each scenario is discussed separately

- Note: Parameter set (Set-1/2) is discussed separately

- Note: MEO can be evaluated optionally

GTW session, May 17th

Agreement

For evaluation of coverage performance in NR NTN,

- It is assumed that carrier bandwidth is sufficiently large to transmit each channel.

- Companies are encouraged to report BWP bandwidth, when necessary (e.g. for frequency hopping).

- Note: each channel bandwidth is discussed separately.

Agreement

For VoIP, AMR 4.75 kbps (TBS of 184 bits without CRC in physical layer) with 20 ms data arriving interval is used in the evaluations.

- Each packet is transmitted within 20 ms, if packet combining is not used.

  - Companies are encouraged to evaluate at least packet transmission without combining

  - Companies are encouraged to report how to apply packet combining, if used.

  - Note: in packet combining, two packets can be combined into a single packet at TX side

    - Companies should report the impact on E2E latency

- VoIP is evaluated only in LEO scenario.

- Note 1: PRB/MCS/TBS determinations are discussed separately

- Note 2: companies should report if HARQ is used in the evaluations, and if evaluations depart from the assumption that each packet is transmitted within 20 ms

Agreement

Reuse Set-1/2 satellite parameters as in table 6.1.1.1-1/2 of TR38.821 for GEO/LEO-1200/LEO-600 and S-band, and as in table 6.1.1.1-1/2 of RP-220590 for MEO and S-band.

- In addition, evaluations assuming relevant ITU regulatory limitations on power flux density can be reported in the study phase.

  - Companies should report which value of EIRP density is used and corresponding justification.

Email, May 19th

Agreement

For link budget calculation, parameters in the following table is assumed.

| Parameters | Notes |
| --- | --- |
| Carrier frequency | 2 GHz for DL and UL (S-band) |
| Channel bandwidth | FFS |
| Satellite altitude | 600 km, 1200 km, 10000 km, 35786 km |
| Target elevation angle | [30 (LEO), 12.5 (GEO-Set 1) , 20° (GEO –Set 2), 30° (MEO)] |
| Atmospheric loss | Equation (6.6-8) in [2] |
| Shadowing margin | 3 dB |
| Scintillation loss | Section 6.6.6 in [2] Ionospheric loss: = 2.2 dB (note 1) Tropospheric loss: Table 6.6.6.2.1-1 of [2] |
| Additional loss | 0 dB |
| Clear sky conditions | Yes |
| Satellite antenna polarization | Circular polarization |
| Terminal type | [S band: (M, N, P) = (1,1,2)] |
| Free space path loss | Equation (6.6-2) in [2] |
| Terminal RF parameters | FFS |
| Satellite RF parameters | FFS |
| Polarization loss | As agreed separately |
| Outcome | CNR |
| - NOTE 1: Based on P3 curve for 1% of time from Figure 6.6.6.1.4-1 of [2] after frequency scaling. - dB - NOTE 2: [2] in this table is 3GPP TR 38.811 v15.2.0: "Study on New Radio (NR) to support non-terrestrial networks (Release 15)" |  |

Agreement

If corresponding channel (including SCS) is agreed as evaluation target channel, the following features introduced in Rel-17 Coverage enhancement WI can be applied in coverage evaluation of NR NTN.

- For VoIP, max 20 PUSCH repetitions if SCS = 15 kHz and packet combining/HARQ are not applied; otherwise, max 32 PUSCH repetitions with consideration of the impact on E2E latency

- For low-data rate service, max 32 PUSCH repetitions

- TBoMS

- Joint channel estimation (DMRS bundling)

  - Companies are encouraged to report how to apply

- Max 16 Msg.3 PUSCH repetitions

- Proposals for agreements/conclusions

Proposal 1-2_v6

For low-data rate service, the following target data rate is assumed.

- Option 1:

  - For DL, 3 kbps if satellite EIRP density lower than values in table 6.1.1.1-1/2 of TR38.821 for GEO/LEO-1200/LEO-600 and S-band, or values in table 6.1.1.1-1/2 of RP-220590 for MEO and S-band due to ITU regulatory limitations on power flux density is considered; otherwise, 1 Mbps

  - For UL, 3 kbps and 100 kbps

  - Companies reporting that 1 Mbps for DL or 100 kbps for UL cannot be achieved in current spec are encouraged to report whether 1 Mbps for DL or 100 kbps for UL is to be achievable in valid assumption in each scenario.

- Option 2:

  - For DL, 1 Mbps

  - For UL, 100 kbps

  - Companies reporting that 1 Mbps for DL or 100 kbps for UL cannot be achieved in current spec are encouraged to report whether 1 Mbps for DL or 100 kbps for UL is to be achievable in valid assumption in each scenario.

- Option 3:

  - For DL, 3 kbps if satellite EIRP density lower than values in table 6.1.1.1-1/2 of TR38.821 for GEO/LEO-1200/LEO-600 and S-band, or values in table 6.1.1.1-1/2 of RP-220590 for MEO and S-band due to ITU regulatory limitations on power flux density is considered; otherwise, 1 Mbps

  - For UL, 3 kbps

Proposal 5-2_v6

Evaluate coverage performance for the following UE characteristics as in Table 6.1.1.1-3 of TR38.821 with update of polarization, Tx/Rx antenna gain, and antenna type and configuration.

| Characteristics | Handheld |
| --- | --- |
| Frequency band | S band (i.e. 2 GHz) |
| Antenna type and configuration | 1 TX, 2TX (optional) / 2 RX of (1,1,2) with omni-directional antenna element |
| Polarisation | Linear |
| Rx Antenna gain | [X] dBi per element |
| Antenna temperature | 290 K |
| Noise figure | 7 dB |
| Tx transmit power | 200 mW (23 dBm) |
| Tx antenna gain | [X] dBi per element |

- X = -5 as working assumption

  - Send an LS to RAN4 to ask whether above antenna gain is valid and if invalid, appropriate value.

Proposal 8-2_v2

For coverage performance evaluation, the following elevation angle is assumed.

- 30 deg for LEO, 12.5 deg for GEO-Set 1, 20 deg for GEO-Set 2, as in in Table 6.1.3.2-1 of TR38.821

  - Note: For GEO-Set 1, channel parameters for 10 deg is used in LLS.

- 30 deg for MEO

- Other elevation angles can be evaluated as optional

- Note: these values are elevation angles at the edge of the edge beam.

Proposal 8-3_v2

For NR NTN coverage enhancement, evaluate the following cases.

| Case | Satellite orbit | Satellite parameter set | Elevation angle (deg) | Terminal | Frequency band | Service type |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | GEO | 1 | [12.5] | Handset | S-band | Low-data rate service |
| 2 | GEO | 2 | [20] | Handset | S-band | Low-data rate service |
| 3 (Optional) | LEO-1200 | 1 | [30] | Handset | S-band | VoIP |
| 4 (Optional) | LEO-1200 | 1 | [30] | Handset | S-band | Low-data rate service |
| 5 | LEO-1200 | 2 | [30] | Handset | S-band | VoIP |
| 6 (Optional) | LEO-1200 | 2 | [30] | Handset | S-band | Low-data rate service |
| 7 (Optional) | LEO-600 | 1 | [30] | Handset | S-band | VoIP |
| 8 (Optional) | LEO-600 | 1 | [30] | Handset | S-band | Low-data rate service |
| 9 | LEO-600 | 2 | [30] | Handset | S-band | VoIP |
| 10 (Optional) | LEO-600 | 2 | [30] | Handset | S-band | Low-data rate service |
| 11 (Optional) | MEO | 1 | [30] | Handset | S-band | Low-data rate service |
| 12 (Optional) | MEO | 2 | [30] | Handset | S-band | Low-data rate service |

Proposal 9-1_v2

For NR NTN coverage enhancement, the following channels/signals are evaluated.

- PUSCH for VoIP

- PUSCH for low data rate service

- PUCCH format 1 with 2 bits (optional)

- PUCCH format 3 with 11 bits (optional)

- PRACH format 0

- PRACH format B4 (optional)

- PUSCH Msg.3

- PUCCH for Msg.4 HARQ-ACK (optional)

- SSB

- PDSCH for VoIP

- PDSCH for low data rate service

- PDSCH Msg.2 (optional)

- PDSCH Msg.4 (optional)

- PDCCH (optional)

- Broadcast PDCCH (PDCCH of Msg.2) (optional)

Proposal 10-2_v3

For coverage performance evaluation, the following are assumed for all channels/signals

- Channel model/Delay spread

  - Channel model as in Table 6.1.2-4 of TR38.821, assuming NTN-TDL-A (NLOS) and NTN-TDL-C (LOS)

- Evaluation scenario

  - Rural (LOS/NLOS)

  - Sub-urban (LOS/NLOS) (optional)

- Channel estimation: Realistic estimation

  - Companies are encouraged to report channel estimation method.

- SCS

  - 15 kHz only

- UE speed: 3 km/h

- Frequency drift: Not assumed

- Frequency offset: 0.1 ppm

Proposal 10-3_v1

For coverage evaluation of PUSCH in NR NTN, the following table is assumed.

| Parameter | Value |
| --- | --- |
| Frequency hopping | w/ or w/o frequency hopping |
| BLER | For low data rate service, w/ HARQ, 10% iBLER; w/o HARQ, 10% iBLER. For VoIP, 2% rBLER. |
| Number of UE transmit chains | 1, 2 (optional) |
| DMRS configuration | For 3km/h: Type I, 1 or 2 DMRS symbol, no multiplexing with data. For frequency hopping: Type I, 1 or 2 DMRS symbol for each hop, no multiplexing with data. PUSCH mapping Type, the number of DMRS symbols and DMRS position(s) are reported by companies. |
| Waveform | DFT-s-OFDM, CP-OFDM (optional) |
| PUSCH duration | 14 OS |
| Repetitions | w/ type A repetition, optional for type B repetition. The actual number of repetitions is reported by companies. |
| HARQ configuration | Whether/How HARQ is adopted is reported by companies. |
| PRBs/TBS/MCS for low data rate service | Any value of PRBs, and corresponding MCS index, reported by companies will be considered in the discussion. TBS can be calculated based on e.g. the number of PRBs, target data rate, frame structure and overhead. |
| PRBs/MCS for VoIP | Any value of PRBs reported by companies will be considered in the discussion. QPSK, pi/2 BPSK (optional) |

Proposal 10-4_v1

For coverage evaluation of PUCCH in NR NTN, the following table is assumed.

| Parameter | Value |
| --- | --- |
| PUCCH format | Format 1, 2bits UCI. Format 3, 11 bits UCI |
| Frequency hopping | w/ frequency hopping |
| BLER | - For PUCCH format 1: DTX to ACK probability: 1%. NACK to ACK probability: 0.1%. ACK missed detection probability: 1%. - For PUCCH format 3: BLER for Ack/Nack, SR: 1% BLER for CSI: 1%, optional for 10%. |
| Number of UE transmit chains | 1 |
| DMRS configuration | Number of DMRS symbols for PUCCH Format 3: Reported by companies |
| Repetitions | w/ repetition. The maximum number of repetitions is 8. |
| PUCCH duration | 14 OS |
| Number of PRBs | 1 PRB |

Proposal 10-5_v1

For coverage evaluation of PRACH in NR NTN, the following table is assumed.

| Parameter | Value |
| --- | --- |
| Format | Format 0, Format B4 |
| SCS | Reported by companies. |
| Performance metric | 1% missed detection at 0.1% false alarm probability 10% missed detection: reported by companies if this value is used |
| Number of UE transmit chains | 1, 2 (optional) |
| Other parameters | Reported by companies. |

Proposal 10-6_v1

For coverage evaluation of PUSCH Msg.3 in NR NTN, the following table is assumed.

| Parameter | Value |
| --- | --- |
| Frequency hopping | w/ or w/o frequency hopping |
| Number of UE transmit chains | 1, 2 (optional) |
| Number of DMRS symbol | w/o frequency hopping: 3, w/ frequency hopping: 2 for each hop |
| Waveform | DFT-s-OFDM |
| HARQ configuration | Whether/How is adopted is reported by companies. |
| PUSCH duration | 14 OS |
| Number of PRBs | 2 |
| TBS | 56 bits |
| Other parameters | Reported by companies. |

Proposal 10-7_v1

For coverage evaluation of SSB in NR NTN, the following table is assumed.

| Parameter | Value |
| --- | --- |
| Number of UE receive chains | 2 for 2GHz |
| Periodicity | 20ms |
| Performance metric | Combination of 4 SSBs in 80ms. Note: UE is not assumed to know the SS/PBCH block index |
| Other parameters | Reported by companies. |

Proposal 10-8_v2

For coverage evaluation of PDSCH in NR NTN, the following table is assumed.

| Parameter | Value |
| --- | --- |
| BLER | For low data rate service, w/ HARQ, 10% iBLER; w/o HARQ, 10% iBLER. For VoIP, 2% rBLER. |
| Waveform | CP-OFDM |
| Number of UE receive chains | 2 for 2GHz |
| HARQ configuration | Whether/How HARQ is adopted is reported by companies. |
| DMRS configuration | [3 DMRS symbols is used for PDSCH of Msg.2.] For 3km/h: Type I, 1 or 2 DMRS symbol, no multiplexing with data. PDSCH mapping Type, the number of DMRS symbols and DMRS position(s) are reported by companies. |
| PRBs/MCS/TBS | Reported by companies. |
| PDSCH duration | 12 OS |
| [Payload size for PDSCH of Msg.4] | [1040 bits] |
| Other parameters | Reported by companies. |

Proposal 10-9_v0

For coverage evaluation of PDCCH in NR NTN, the following table is assumed.

| Parameter | Value |
| --- | --- |
| Number of UE receive chains | 2 for 2GHz |
| Aggregation level | 16 |
| Payload | 40 bits |
| CORESET size | 2 symbols, 48 PRBs |
| Tx Diversity | Reported by companies |
| BLER | 1% BLER optional for 10% BLER |
| Number of SSB for broadcast PDCCH of Msg.2 | Reported by companies |
| Other parameters | Reported by companies |

- Email discussions

  - [Email] Topic #1: Target data rate

In WID, it is stated that target service for NR NTN coverage enhancement is VoIP and low-data rate services for commercial handset terminals. Meanwhile, the data rate is unclear there. RAN1 needs to agree target data rate for this topic.

For VoIP,

- Three companies [6/Pana] [9/Xiaomi] [22/Ericsson] propose to reuse ‘packet size for VoIP’ in Table A.1-1 of TR38.830. 320 bits with 20 ms data arrival interval is assumed; i.e., data rate would be 16 kbps if FL understands correctly. Two companies [5/vivo] [22/Ericsson] propose 4.75 kbps coming from the lowest AMR data rate. One company [1/HW, HiSi] proposes 9.1 kbps according to the codec for Enhanced Voice Services (EVS), successor of the voice codec AMR-WB, standardized by the 3GPP SA4 in Release 12. One company [20/THALES] proposes 17.2 kbps to consider the Enhanced Voice Services (EVS) codec with some overhead.

- There is a text of ‘... taking into account the studies in TR38.830 ...’ in WID. FL thinks that study of NR NTN coverage enhancement should reuse discussion outcome of NR TN coverage enhancement as much as possible. Without a certain reason to select different assumption, it would be reasonable that the same value is reused. FL would like to ask whether the same data rate as in TR38.830 can be selected or not, and if the answer is NO, the reason and value that should be adopted.

- Besides, at least one company [11/Samsung] mentions that latency requirement for VoIP should be discussed sufficiently in consideration of large propagation delay. In addition, it seems that several companies consider this aspect in their simulation evaluation, e.g. the max number of repetitions is not 32 but 20 in order to meet the latency requirement. FL would like to ask whether the latency requirement should clearly be agreed or not, and if the answer is YES, which value should be agreed.

For low data rate,

- For DL, two companies [9/Xiaomi] [20/THALES] propose 1 Mbps while one company [15/Apple] propose 100 kbps. For UL, two companies [9/Xiaomi] [20/THALES] propose 100 kbps one company [15/Apple] propose 50 kbps. It is noted that Rel-17 TN CovEnh assumed 1 Mbps for DL and 100 kbps for UL in rural scenario (see Appendix-3 for details).

- Four companies [1/HW, HiSi] [5/vivo] [11/Samsung] [22/Ericsson] suggest to have sufficient discussion on the target data rate. [1/HW, HiSi] [5/vivo] think that 100 kbps for UL is not reasonable value and lower value should be considered.

- FL would like to ask whether the same data rate as in TR38.830 can be selected or not. Also, please share the reason of YES/NO.

    - 1st round

Proposal 1-1_v0

For VoIP, a packet size of 320 bits with 20 ms data arriving interval is adopted as in Table A.1-1 of TR38.830.

Q: Do you agree the above proposal? If the answer is NO, please share the reason and which value should be adopted instead.

| Company | YES/NO | Comment (e.g. reason of NO) |
| --- | --- | --- |
| QC | No | It’s clear that the link budget is not enough to support even the lowest code rate in some cases. The lowest codec rate should be considered, i.e., AMR4.75 kbps or EVS 5.9 kbps. |
| Lenovo | No | There is large path loss for the satellite network. The scenario is different from the TN network. We also thinks a smallest codec rate is adopted for the evaluation, such as AMR 4.75kbps. |
| Apple | Yes | If target data rate for low-data rate service is low enough (e.g., comparable with 320bit /20ms VoIP), then we can consider lower codec rate for VoIP. |
| Xiaomi | Yes | Prefer to adopt the data rate assumption in TR 38.830 for NTN coverage enhancement. |
| vivo | No | There would be a big performance gap required for LEO-1200km set 2 case, if a packet size of 320 bits with 20ms data arriving interval is adopted. Furthermore, for NTN, there’s no need to require a higher quality voice service, the basic low data rate voice would be enough. We propose to choose either AMR4.75 kbps or EVS 5.9 kbps which have similar TB size in physical layer, i.e. 184 bits without CRC. |
| Samsung | [Yes] | As another baseline, we are okay with considering 4.75 kbps based on lowest AMR data rate. (e.g., 95bits with 20ms data arriving interval) |
| Nokia, Nokia Shanghai Bell | Yes |  |
| OPPO | No | The codec rate of 320bits with 20ms data arriving interval is larger than AMR4.75 kbps or EVS 5.9 kbps. Considering NTN link suffers from larger path loss than TN, AMR4.75 kbps or EVS 5.9 kbps can be considered for coverage performance evaluation in NR NTN. However, it is more intuitive for us to determine the TBS for the LLS assumption. |
| Huawei, HiSilicon | No | For VoIP, the target data rate should be aligned rather than the TBSS. EVS 5.9kbps could be used and it would be better to also align the PHY data rate among companies considering higher layer overhead. |
| LG | No | Considering limited link budget in NTN environment, it would be better to set lowest codec rate as a target. For WID description, it is desirable to re-use evaluation methodologies and some parameters which is not related to TN/NTN difference. However, it is not necessary to follow TR38.830 at least for scenarios and target data rate. We are also fine with considering 4.75 kbps based on lowest bitrate AMR codec. |
| NTT DOCOMO | Yes | We are open to discuss whether to consider lower codec rate. |
| ZTE | Yes | We are fine to use 320 bits packet size as baseline for evaluation. And we are also open to evaluate lower codec rate. |
| CATT | No | Due to limited link budget, lower code rate can be considered. |
| Ericsson | Yes | We are also ok with a lower codec rate, e.g. AMR 4.75 or EVS 5.9. |
| Thales | Yes | We support Moderator proposal. We agree with QC in some scenarios (e.g. LEO 1200km orbit with Set2 parameters) it will be challenging to close the link budget as the gap is large in case of VoNR packet size of 344 bits every 20ms. However we may need to consider the SIP invite message. Indeed, even with lower codec (EVS 5.9 kbps) we think a certain data rate should be guaranteed during the voice call establishment. If payload of 1500 bytes is considered for the SIP invite message. Depending on the average call setup time we expect that UL throughput required to transmit SIP signaling successfully could be at least 8 kbps. Which means that SIP invite message may require higher throughput than the bit data rate generated by the codec (EVS 5.9 kbps or AMR4.75 kbps). |
| MediaTek | No | AMR 4.75 kbps can be the baseline for NRNTN coverage enhancements for VoIP for the lowest link budget cases. Lower EVS codec 5.9 kbps may also be considered |
| Ligado | No | Agree with Qualcomm’s comments above. Closing the link to a GEO satellite will be challenging. AMR 4.75 kbps can be the baseline. |
| InterDigital | Yes | We are open to discuss on a lower codec rate |
| Sony | No | The link budget is very challenging, and a lower codec rate should be assumed. AMR4.75 seems appropriate. |
| Lockheed | Yes |  |
| Panasonic | Yes | We are ok to have the lowest codec rate as the optional evaluation. |
| Hughes/EchoStar | Yes | Agree with the moderator’s proposal |

Q: Do you think latency requirement for VoIP should be agreed clearly? If the answer is YES, please share the value.

| Company | YES/NO | Comment (e.g. latency requirement value) |
| --- | --- | --- |
| QC |  | Needs further discussion. For the purpose of current WID, it’s possible not to have a clear latency requirement rather a BLER requirement assuming 20 ms per frame or 40 ms per frame. |
| Lenovo | Yes | We think the latency requirement for VoIP should be agreed. We are fine to 20ms. |
| Apple | Yes | The latency requirement for VoIP is important, and we prefer to clarify it. |
| Xiaomi | Yes | We are fine with 20 ms as data arriving interval. |
| vivo | Yes | As mentioned in our contribution, the maximum end-to-end latency requirement equals 200ms as is used in LTE VoIP evaluation. It can be observed that at least MEO and GEO scenarios cannot satisfy the latency requirement. For LEO case, 20ms is allowed for transmission. Thus, we propose to support VoNR only on LEO scenarios and assume the maximum transmission time allowed is 20ms. |
| Nokia, Nokia Shanghai Bell | Yes | At least some aspects of latency requirements for time critical services such as VoIP need further discussion. In case continuous or near-continuous transmission is expected in the UL, the physical limitations of the transmission channel will naturally limit the tolerable latency, since there is no room for HARQ retransmissions. Under the condition that UE is not requiring continuous or near-continuous transmissions, there may be capacity for HARQ retransmissions and for that case it might be sufficient or adequate to address one HARQ retransmission, meaning that we could allow for a latency requirement of ~60 ms. |
| Huawei, HiSilicon | Yes | It is good to clarify the latency requirement clearly. However, it is open to discuss what latency value is required. |
| NEC | Yes | VoIP latency requirement should be discussed considering different satellite deployment scenarios (e.g. GEO/LEO). However, it is not intuitive to specify latency requirement wrt the occasion of initial PUSCH/PDSCH repetition because propagation delay would still be the major component for packet latencies. As an example for GEO, latency caused due to extra number of PUSCH repetitions shall not be significant as compared to the propagation delay in order of 140ms, but the same cannot be said for LEO-600. Hence, the VoIP latency values should be specified as radio access latency based on the NTN cell type (i.e. GEO/LEO-1200/LEO-600) |
| ZTE | Yes | The 20ms latency for VoIP is preferred. |
| CATT | Yes | For NTN case, the latency requirement can be defined differently from the TN case. |
| Ericsson | Yes | 200 ms end-to-end latency for LEO. We should also specify how much of this is available for the Uu interface. For MEO/GEO it could be considered to have a relaxed latency requirement or no requirement at all. |
| Thales | Yes | Latency requirement for VoNR in NTN should be further discussed. From our perspective, based on Table 5.7.4-1: Standardized 5QI to QoS characteristics mapping in TS 23.501: - For Voice (including repetitions ≤ 20 ) : 5QI-7 can accommodate the worst case PDB for LEO satellite type. FFS: for GEO. - 5QI-10 can accommodate the worst case PDB for GEO satellite type. But not appropriate for voice. |
| Ligado | No | Not relevant for GEO and potentially difficult for LEO due to RTT. |
| InterDigital | Yes | We are fine with 20ms latency |
| Sony | Yes | Latency target needs to be discussed. |
| Lockheed | Yes | The latency should be further discussed. |
| Panasonic | Yes | OK with 20ms as the value, but we are not sure the meaning of “latency” in this context. With the usual usage of “latency”, i.e. delay between gNB and UE modem, the propagation delay influences a lot in NTN. One possible approach can be "propagation delay + 20ms" as the latency requirement. |

Q: Do you think the following target data rate for low-data rate service as in Table A.1-1 of TR38.830 can be reused? Also, please share the reason of YES/NO and which value should be adopted instead.

- DL 1 Mbps, UL 100 kbps

| Company | YES/NO | Comment (e.g. reason of YES/NO) |
| --- | --- | --- |
| QC | No | The objective of the WID is to enable voice and low-date rate services for smart phones. Given the link budget issue, we should consider SMS services that will be the typical applications for smart phones over NTN. |
| Lenovo | No | We think a least the value 1Mbps for DL is a bit high. We think we should consider the smallest possible date rates which can be used by smart phone. |
| Apple | No | The DL of 1 Mbps is assumed for rural scenario. For NTN, we think a lower data rate can be considered. In TS 22.261 Section 6.16, it is mentioned that for “markets requiring minimal service levels”, 100 kbps for more than 100 km cell coverage can be considered. |
| Xiaomi | Yes | Adopt the performance requirements for satellite access specified in TR 22.261. |
| vivo | No | Considering the link budget results, both 1Mbps for DL and 100kbps for UL are too high to reach for NTN scenarios. In our view, the low data rate service is mainly targeted for GEO case in which voice cannot be supported , and it would be good enough to support some short message communication for GEO case. A feasible target data rate for low-data rate service could be determined based on evaluations for GEO scenario to find an achievable data rate via some potential coverage enhancement techniques that we can foresee. According to our initial simulations, we prefer to study a range of 0.5 to 10kbps as candidate target data rates so that a small and reasonable coverage enhancement gap would be required for GEO scenario. |
| Nokia, Nokia Shanghai Bell | Yes | Deployment scenarios are substantially different under NTN operation, since the gNB density is expected to be uniform (as much as can be) over the sky. Also, in Urban areas the propagation conditions are much more severe compared to Rural, and it does not make any sense to have higher target data rates for Urban scenarios – quite the contrary. For simplicity we would suggest that only the “Rural with long distance” is maintained here: Rural with long distance: DL 1Mbps, UL 100kbps, 30kbps (optional) |
| Huawei, HiSilicon | No | 100kbps for uplink is challenging for NTN considering the link budget, especially for GEO scenario. We should discuss proper low data rate target which could be supported by NTN link budget. |
| LG | Yes | We think “Rural with long distance” could be starting point. We can also set 100 kbps as optional instead of 30 kbps. |
| ZTE | No | The data rate is too high and may not able to be satisfied. We think the possible achievable data rate in NTN should be first evaluated and then consider the target rate based on what can be supported. |
| CATT | No | For NTN case, the data rate can be assumed as 100kbps or lower. |
| Ericsson | Yes | It can be used as a starting point. Could be revisited if it turns out it is too challenging. |
| Thales | Yes | We need to support MBB services which require a data rate at least few Mbps/few hundreds of kbps (DL/UL) as per Table 7.4.2-1 in [3, TS 22.261] : for pedestrian user, the required data rate is 1 Mbit/s in the downlink and 100 kbit/s in the uplink. |
| Ligado | No | Considering the link budget, the UL data rate value proposed is too high. A range of values should be considered. |
| Sony | No | We think these data rates are not supported by the link budget (though this depends on further link budget / evaluation assumptions). |
| Lockheed | Yes | This is sufficient for a low data rate service |
| Panasonic | Yes |  |
| Hughes/EchoStar | yes | Need to support MBB services |

Q: Please input only if you think other aspect on target data rate should be discussed.

| Company | Comment |
| --- | --- |
| Nokia, Nokia Shanghai Bell | Lower data rates for Urban scenarios (NLOS conditions) may be considered. |
| Thales | We may need to consider the SIP invite message. Indeed, even with lower codec (EVS 5.9 kbps) we think a certain data rate (maybe higher than bit data rate generated by the codec) should be guaranteed during the voice call establishment. |
| Ligado | GEO and LEO should be treated differently. |
| Lockheed | Yes |

    - Summary of 1st round

On Proposal 1-1_v0, 11 companies are OK with this proposal, but 9 companies think lower code rate should be considered. The rationale is that the link budget is not sufficient for the code rate of the proposal, and AMR 4.75 kbps or EVS 5.9 kbps should be used instead. Besides, for NTN / coverage limitation scenario, higher quality voice service might not be necessary. In addition, 7 companies supporting this proposal can also be OK to consider lower code rate. Based on the situation, FL suggests to use the lowest coding rate, i.e. AMR 4.75 kbps.

Regarding latency requirement for VoIP, most companies think that latency requirement should be considered. As the value, it seems that 20 ms and 200 ms are proposed. FL’s understanding based on reading [5/vivo] is that 20 ms is data arriving interval and 200 ms is end-to-end latency. Although 200 ms E2E latency is allowed, why 20 ms is proposed is that basically each packet should be transmitted successfully within 20 ms since all/most slots are used for repetitions and there is no/little room to perform HARQ retransmission. FL would like to ask whether this understanding is correct or not. In addition, Panasonic’s comment is that 20 ms is not ‘latency’. FL has the same view; 20 ms is time duration where transmission of each packet is allowed. Besides, some companies think that the 200 ms E2E latency cannot be met in GEO/MEO and thus VoIP should be evaluated only for LEO scenario. This argument seems valid but FL would like to ask companies’ views.

For low-data rate service, 7 companies are OK with the data rate from TR38.830 while 9 companies propose lower data rate. A bit more companies prefer different data rate. As commented by several companies, main target would be quite low-data rate service like SMS services. Some companies from 9 companies think the main issue is UL data rate, but other companies from 9 companies think DL data rate is also challenging. For DL, based on Apple’s comment, 100 kbps is considered for more than 100 km cell coverage in TS22.261. FL would like to reuse this value considering the current situation. For UL, Nokia/LGE suggests to consider value from ‘rural with long distance’. 30 kbps is assumed as optional, FL would like to reuse this value for NTN (not as optional). In addition, based on discussion in section 4.4, which scenario this low-data rate service should be evaluated in is an important topic. The proposal includes this aspect, and FL would like to see companies’ views.

    - 2nd round

Proposal 1-1_v1

For VoIP, AMR 4.75 kbps with 20 ms data arriving interval is adopted.

- Each packet is transmitted within 20 ms.

- VoIP is evaluated only in LEO scenario.

Q: Do you agree the above proposal? If the answer is NO, please share the reason and how to modify.

| Company | YES/NO | Comment (e.g. reason of NO) |
| --- | --- | --- |
| vivo | Yes |  |
| MediaTek | Yes |  |
| Lockheed | Yes |  |
| Apple | Yes |  |
| Lenovo | Yes |  |
| Samsung | Yes |  |
| Panasonic | Yes |  |
| Xiaomi | Yes |  |
| Spreadtrum | Yes |  |
| LG | Yes |  |
| NTT DOCOMO | Yes |  |
| ZTE | Yes |  |
| OPPO | Yes | Based on our discussion, we need to make a conclusion that GEO scenario does not support VoIP due to long RTT time (lager than 200ms E2E VoIP latency requirement) and sent it to SA2. |
| Thales | NO | We are fine to use a voice codec generating a lower bite rate. But we prefer a EVS (Enhanced voice services) codec. This is the current recommendation for 5G. And already used in 4G. We may consider a Wideband EVS codec with a bit rate of 5.9 kbps. We may need to capture in the proposal the corresponding TBS with 20ms data arriving interval. The 20ms given in the proposal is related to packet arrival. Indeed, speech packets are generated once every 20ms during period of speech activity. Furthermore, we need to discuss the SIP invite message issue. Indeed, if payload of 1500 bytes is considered for the SIP invite message. Depending on the average call setup time we expect that UL throughput required to transmit SIP signaling successfully could be at least 8 kbps. Which means that SIP invite message may require higher throughput than the bit data rate generated by the codec (EVS 5.9 kbps or AMR4.75 kbps). Regarding latency requirement for Voice: The latency to be considered is given in Table 5.7.4-1: Standardized 5QI to QoS characteristics mapping, TS 23.501 The packet delay budget that applies to the radio interface = 100ms – 20ms = 80ms. Indeed, according to TS 23.501 a static value for the CN PDB of 20 ms for the delay between a UPF terminating N6 and a 5G-AN should be subtracted from a given PDB to derive the packet delay budget that applies to the radio interface The Packet Delay budget (PDB) repsents the max permitted delay between the UE and the UPF . if both originating and terminating voice calls have VoNR connections, then the total end to end PDB will include 2 x 100ms = 200ms. The ITU-T recommends a maximum delay of 200ms to good QoS (Refer to ITU-T Rec G.114) |
| ETRI | Yes | We are fine with the proposal, and would like to suggest a clarification on typical packet size corresponding to AMR 4.75 kbps (like, “A packet size of X bits with 20ms data arriving interval”) And it would be helpful to check if X=160 or not. [160=96(AMR 4.75kbps Payload)+16(MAC)+8(RLC)+16(PDCP)+24(IP)] |
| Huawei, HiSilicon | Yes | We are either fine with EVS 5.9kbps. |
| Nokia, Nokia Shanghai Bell | Yes | We could also be OK with 5.9 kbps as well. |
| Ericsson | Partly | The first sub-bullet (Each packet is transmitted within 20 ms) is not necessary. HARQ with a small number of retransmissions is supported in Rel-17 and may be considered for LEO to evaluate if it can improve baseline coverage. |
| NEC | Yes |  |
| Ligado | Yes |  |
| QC | Partly | We agree with the main statement. The first sub-bullet is not needed. Voice packet combining, e.g., two voice frames per 40 ms, should be allowed if it provides performance gain. |

Proposal 1-2_v0

For low-data rate service, the target data rate is 100 kbps for DL and 30 kbps for UL.

- Low-data rate service is evaluated only in GEO scenario.

Q: Do you agree/accept the above proposal? If the answer is NO, please share the reason and how to modify/converge.

| Company | YES/NO | Comment (e.g. reason of NO) |
| --- | --- | --- |
| vivo | No | We’re fine to study GEO SMS services. However, we do not think the 30kbps target data rate for UL can be achievable based on our initial evaluation results and the potential coverage enhancement techniques we have in mind. In our evaluations for VoIP with 4.75kbps, 20 repetitions have been used. For AMR 4.75kbs VoIP, the corresponding data rate from physical layer is 10kbps. Without the limitation of 20ms data arriving interval, low-data rate service can use up to 32 repetitions according to Rel-17. However, if other assumptions are the same as VoIP, there would be less than 3dB gain. It can be predicated that target 30kbps for UL low-data rate service would be challenging even if some additional repetitions can be applied for low data rate transmission. In our study for low data rate service, with 1 PRB and MCS 9 corresponding to 5.47kbps data rate, with 32repetitions, target SNR derived is -10.94dB, required CNR calculated from link budget analysis is -19.73dB, meaning that around 9dB coverage enhancement is required for Set-2 GEO satellite with 90-degree elevation angle, and this gap is already quite large. According to above, for UL, we would prefer to study the data rate less than 10kbps given also that the SMS data rate should be no larger than VoIP. As for DL target data rate, without considering ITU PFD limitation, higher data rate could be achievable via compared to uplink. However, we’re wondering whether it necessary to have different data rate for UL and DL while we only consider the SMS service in GEO. In our view, the UL data rate could be assumed for DL as well. |
| Lenovo |  | We are fine with the date rate for DL and UL. Regarding the scenario, we think both LEO and GEO should be considered. |
| Apple |  | We are fine with the main bullet. We think the low-data rate service should be evaluated in both GEO and LEO (600, 1200) scenarios. |
| Samsung |  | Why low-data rate service is only evaluated for GEO? We think that evaluating LEO case is also needed since WID doesn’t separate use cases between LEO and GEO. |
| Panasonic | Yes |  |
| Xiaomi |  | We would like to support DL 1Mbps and UL 100 kbps for LEO case. And it is okay to relax the data rate requirement for GEO scenario, e.g., 100 kbps for DL and 30 kbps for UL. |
| LG |  | We are fine with the main bullet. Regarding main bullet, we think both LEO and GEO can be considered. |
| ZTE |  | We think both GEO and LEO should be evaluated. For UL data rate, 30kbps seems still high since AMR 4.75 kbps is proposed to be evaluated for VoIP in proposal 1-1. For DL data rate, since the DL performance is affected by the satellite EIRP, we may first consider how to handle the PFD limit. If the limit can be avoided by frequency management, high DL data rate can be achieved. Otherwise, low data rate should be assumed. |
| OPPO |  | The target data rate should apply all satellite scenario, so we can accept the proposal with the following modification: Proposal 1-2_v1 For low-data rate service, the target data rate is 100 kbps for DL and 30 kbps for UL for all satellite scenarios. - Low-data rate service is evaluated only in GEO scenario. |
| Thales |  | We need to consider LEO deployment as well. |
| ETRI | Yes | We are fine with the proposal, and open to the evaluations in not only GEO but also LEO. |
| Huawei, HiSilicon | No | We agree with vivo that 100 kbps for DL and 30 kbps for UL are still quite challenging. If we cannot converge on the data rate of low data rate, we can firstly prioritize the VOIP for the time being. |
| Nokia, Nokia Shanghai Bell | Yes with comment | It might be beneficial to also evaluate the low-data rate service for non-GEO scenarios. |
| Ericsson | Yes |  |
| MediaTek |  | LEO needs to be considered. A lower data rate targets for UL could be considered. VoIP could be prioritized. |
| NEC |  | We have similar view as Lenovo and Apple |
| Ligado | No | Agree with Vivo. |
| Hughes/EchoStar |  | Need to evaluate NGSO deployment as well. |
| QC | No | LEO should be evaluated too. Also, the motivation of 30 kbps for the UL is unclear given that it is clear it cannot be supported in most of the deployment scenarios. Instead, we need to focus on SMS that should not require data rate higher than VOIP. |

    - Summary of 2nd round

Majority support proposal 1-1_v1. FL suggests to agree this proposal with small modification based on comments. For some comments,

- To Thales, FL understands your preference; EVS codec and SIP signaling. But at the same time, now we are talking about service around coverage limited. Firstly we should focus on enabling VoIP service itself, not specific VoIP service. This would be majority companies’ view. In consideration of this view, what we can agree would be the current proposal. Hopefully you understand this situation.

- To ETRI, let’ try to ask the packet size can be agreed.

- To Ericsson, exactly initial TX can use less repetition number and only when failed, HARQ retransmission would be possible. Let’s check other companies’ views.

- To QC, let’s to ask packet combining can be considered.

For proposal 1-2_v0, it seems that majority think low-data rate service should be evaluated in LEO scenario as well as GEO scenario. Let’s remove the sub-bullet. In addition, several companies (not so small) believe that 30 kbps is still challenging for UL. Although focusing on SMS is suggested, exact value is not proposed from any companies. FL would like to ask what data rate should be evaluated. For DL, let’s wait for discussion progress of ITU regulatory limitations on power flux density.

    - 3rd round

Proposal 1-1_v2

For VoIP, AMR 4.75 kbps (a packet size of 160 bits) with 20 ms data arriving interval is adopted.

- Each packet is transmitted within 20 ms, if HARQ retransmission is not used.

  - Packet combining (e.g. two voice frames are transmitted within 40 ms) can be considered.

- VoIP is evaluated only in LEO scenario.

Q. Only if you are not OK with this proposal, please share view.

| Company | Comment |
| --- | --- |
| vivo | We are not OK with the proposal. Firstly, the packet size should be 184 bits which can be found in our contribution as listed below. - Table 6. AMR 4.75kbps voice packet CRC; 16 bits. MAC; 16 bits (with 12 bits SN size). RLC; 8 bits (with 6 bits SN size). PDCP; 16 bits. RTP/UDP/IP; 24 bits (with RoHC). Payload; 120 bits.. The number of bits for AMR 4.75kbps equals 95, and AMR header includes at least 19 bits. In addition, the total length of AMR codec should be byte aligned. Thus, the payload size is 120 bits. Then the packet size from the perspective of physical layer should be 184 bits. Furthermore, we are unclear about the meaning or motivation of “if HARQ retransmission is not used”. In our understanding, no matter whether HARQ retransmission is used or not, 20ms arriving interval requires that the transmission of each packet should be completed within 20ms. We’re fine to further discuss whether or how to do packet combining across different 20ms periods. According to above, we propose the modification below: Proposal 1-1_v2 For VoIP, AMR 4.75 kbps (a packet size of 160184 bits) with 20 ms data arriving interval is adopted. - Each packet is transmitted within 20 ms, if packet combining is not used. - FFS whether and how Ppacket combining (e.g. two voice frames are transmitted within 40 ms) can should be considered. - VoIP is evaluated only in LEO scenario. |
| OPPO | We do not support the proposal. We are not clear about the intention of “HARQ retransmission” and “packet combining” in the proposal. Does it mean that the transmission time of each packet can be larger than 20ms? If so, why Max 20 PUSCH repetitions for VoIP with SCS 15 kHz is required in Proposal 10-1? |
| MediaTek | It will be helpful to align company understanding on the packet size assumption for AMR 4.75 kbps with 20 ms data arriving interval. With 4.75 kbps it is 95 bits every 20 ms. Adding CRC/MAC/RLC/PDCP/ RTP/UDP/IP is another 80 bits. Should it be then one packet = 95 bits + 80 bits = 175 bits every 20 ms? |
| Apple | It is unclear about the necessity of introducing “packet combining” in the proposal. |
| QC | “HARQ retransmission if not applied” is not needed. In addition, we need to agree on the packet size. |
| Panasonic | Basically, OK with the Proposal 1-1_v2, but “if HARQ retransmission is not used” should be replaced with “if packet combining is not used” as mentioned by vivo. |
| Lenovo | We don’t think “HARQ retransmission is not used” is necessary. Also we would like more clarification on packet combining. |
| Xiaomi | It seems the first sub-bullet is not needed as 20ms data arriving interval is assumed in the main bullet. Not clear how packet combining be evaluated in the simulation. |
| LG | We are basically fine with the main bullet of the proposal. However, it should be clarified the transport block size including. For packet combining, we share vivo’s view. 20ms latency requirement should be met regardless of HARQ retransmission or TB repetition. |
| ETRI | We are fine with the vivo’s justification (160184) on the packet size (TB size). |
| Thales | We can accept the AMR 4.75 kbps as this is agreeable to the majority. We do not think the first sub-bullet is needed. Also we need to agree on the TBS. Even though we prefer to consider EVS codec with a bit rate of 5.9 kbps. The TBS assumption in case of EVS codec with a bit rate of 5.9 kbps and header compression should be = 190 bits every 20ms. Regarding the issue related to SIP invite, the FL’s answer is not clear us. For VoIP SIP is needed anyway as it is used for signaling procedures between the UE and IMS. SIP invite could be a concern on uplink is transmission vulnerability to poor radio condition; as pointed out in TR 36.754: - Size of SIP message used in VoLTE call setup is about 2KB. When UE is in poor radio condition (e.g. RSRP < -120dBm) and/or high interference (e.g. uplink IoT = 10dB), it has been observed in practical network that a SIP message is segmented into 200 RLC pieces, thus average call setup time and call drop rate are increased. Therefore a large SIP message size becomes a problem. Based on this, as we commented in 2nd round, we still think that PUSCH with SIP signaling message should be considered. And further discussion of what data rates should be used considering the SIP invite is needed. |
| ZTE | There is no need to mention “HARQ retransmission” when discussing packet. And the motivation of “packet combining” is not clear. |

| Nokia, Nokia Shanghai Bell | OK |
| --- | --- |

| Samsung | Partially agree. For bit size, it should be revised according to vivo’s comment. Regarding “packet combining”, we are not sure whether this solution is feasible or not since it hasn’t been discussed even in coverage enhancement study. So, we prefer to remove the corresponding sub-bullet. |
| --- | --- |
| Huawei, HiSilicon | We have a question for clarification. If packet combining is allowed, does it mean that a packet could be allowed to be transmitted in 40ms rather than in 20ms? The two bullet seems a little bit contradictory. Considering the diverged comments on the first bullet and its sub-bullet, an easier way could be at least first agreeing on basic things, e.g. For VoIP, AMR 4.75 kbps (a packet size of [160/184] bits) with 20 ms data arriving interval is adopted. - Each packet is transmitted within 20 ms, if HARQ retransmission is not used. - Packet combining (e.g. two voice frames are transmitted within 40 ms) can be considered. - VoIP is evaluated only in LEO scenario. |
| Ericsson | We think the first sub-bullet should be removed. |

Proposal 1-2_v1

For low-data rate service, the target data rate is X kbps for DL and Y kbps for UL.

Q. What value should be adopted for the above Y lower than 30 kbps? Note that X is pending due to PFD limitation.

| Company | Comment |
| --- | --- |
| vivo | According to our evaluations for GEO scenario, as we’ve commented in last round, in our study for low data rate service, with 1 PRB and MCS 9 corresponding to 5.47kbps data rate, with 32 repetitions, 2 RX, target SNR derived is -10.94dB, required CNR calculated from link budget analysis is -19.73dB, meaning that around 9dB coverage enhancement is required for Set-2 GEO satellite with 90-degree elevation angle, and this gap is already quite large. We propose we can set a set of candidate rates{1kbps, 3kbps, 5kbps} to study for GEO scenario. |
| OPPO | In R17 CovEnh, companies are encouraged to use 30 PRBs for 1Mbps, 4 PRBs for 100kbps, 1 PRB for 30kbps as a starting point for coverage evaluation. Therefore, we propose to firstly determine a set of PRBs to further study the target data rate. |
| MediaTek | Y=5 kbps would be consistent with AMR 4.75 kbps. |
| QC | Y=3 or 5 kbps can be considered. |
| Xiaomi | Y=3 can be considered. |
| Nokia, Nokia Shanghai Bell | X=100 kbps and Y=30kbps would be OK for us. |
| Samsung | It’s okay to consider similar level of VoIP data rate if main use case of low-data rate service is to support short message service (SMS) |

    - Summary of 3rd round

For VoIP

Most/majority companies think there is no need to consider HARQ retransmission; it is removed.

Regarding packet combining, it is commented that the definition is unclear. In FL’s understanding, even when each packet is arrived with 20 ms data interval, e.g. 2 packets can be combined into a new single packet at TX side and the packet can be transmitted within 40 ms with more than 20 repetitions. It seems that there are three positions: 1) OK with packet combining, 2) Not OK with packet combining, 3) Firstly clarify the definition. FL lists the 1st and 2nd options in the proposal with a note to clarify the definition. Down-selection is done in the GTW session.

As the packet size, 184 bits, 175 bits are proposed from vivo/ETRI/HW, MTK, respectively. FL thinks that vivo’s analysis is valid; then let’s use 184 bits instead.

LGE/Thales think TBS should be discussed, but FL will trigger discussion for the purpose later. This is clarified as a note.

To Thales, even though you think EVS codec and SIP invite should be considered, majority think the minimum data rate for VoIP, i.e. AMR codec, should be assumed and higher code rate is not valid. FL does not EVC code and SIP invite, but you can argue to include them in the GTW session.

For low-data rate service

As Y value (UL data rate), vivo suggests to assume 1/3/5 kbps. MTK thinks 5 kbps can be considered, Xiaomi prefer 3 kbps, and QC think 3 or 5 kbps. It seems Samsung is OK with these values, while Nokia prefer to use the values used in Rel-17 CovEnh WI, i.e. 30 kbps. FL finds that more companies believe smaller value similar to VoIP is valid. Among 1/3/5 kbps, FL believe that more candidate values are not helpful to reduce simulation burden; therefore FL recommends to assume the middle value = 3 kbps.

For X value (DL data rate), still issue on PFD limitation is not converged as discussed in section 4.7. In 1st/2nd rounds, there were comments as data rate would dependent on whether PFD limitation is considered. Then FL would like to try different values for each case to have progress. When PFD limitation is not considered, it seems that some companies prefer to larger data rate. If considered, some companies state the same data rate as UL is sufficient e.g. assuming SMS. But further update on the value would be necessary.

Proposal 1-1_v3

For VoIP, AMR 4.75 kbps (a packet size of 184 bits) with 20 ms data arriving interval is adopted.

- Option 1: Each packet is transmitted within 20 ms

- Option 2: Each packet is transmitted within 20 ms, if packet combining is not used.

  - Companies are encouraged to report how to apply packet combining, if used.

  - Note: In packet combining, multiple of 20 ms data can be combined into a single packet at TX side, and the packet is transmitted within more than 20 ms such that 200 ms E2E latency is satisfied

- VoIP is evaluated only in LEO scenario.

- Note: PRB/MCS/TBS determinations are discussed separately

Proposal 1-2_v2

For low-data rate service, the following the target data rate is assumed.

- For DL, 100 kbps if ITU regulatory limitations on power flux density is not considered; otherwise, 3 kbps

- For UL, 3 kbps

    - 4th round

In GTW session, data rate for VoIP was agreed. We need to decide for low-date rate service; FL would like to suggest to have discussion via reflector.

Proposal 1-2_v2

For low-data rate service, the following the target data rate is assumed.

- For DL, 100 kbps if ITU regulatory limitations on power flux density is not considered; otherwise, 3 kbps

- For UL, 3 kbps

Email discussion

  - [Closed] Topic #2: Evaluation methodology / Performance metrics

For coverage evaluation, the evaluation methodology / performance metrics should be aligned among companies. It seems that at least 7 companies [1/HW, HiSi] [5/vivo] [6/Pana] [9/Xiaomi] [17/DCM] [20/THALES] [22/Ericsson] propose to reuse evaluation methodology defined in Rel-16 NTN (i.e. TR38.821). Meanwhile, three companies [11/Samsung] [15/Apple] [19/LGE] consider evaluation methodology defined in Rel-17 coverage enhancement (i.e. TR38.830). [15/Apple] [19/LGE] propose to use not MPL but MIL.

In TR38.830, three performance metrics are defined: MPL / MIL / MCL. MPL is an absolute value calculated based on SINR, and compared to target MPL. Target MPL is derived from ISD (Inter-Site distance) value and formula. MIL / MCL do not include some aspects from MPL, i.e. they are relative values. Thus MCL / MPL were used to determine which channel/signal should be enhanced, though were not used to decide necessity of enhancement.

FL’s understanding is that at least MIL / MCL are not applicable in order to evaluate certain coverage performance since they are relative values as abovementioned. To use MPL, formula to decide target MPL is necessary; however, it seems that the formula defined in TR38.830 is not matched with NTN scenario. On the other hand, FL thinks that CNR calculation is well-defined in section 6.1.3.1 of TR38.821 and it can be reused for this study. CNR value is calculated based on parameters clearly defined in this study, and the value is compared to required SINR of target service, as several companies mentioned.

The above FL’s understanding might be incorrect, thus FL would like to ask whether or not this study can reuse link budget calculation defined in TR38.821 and compare with required SINR. In addition, FL would like to ask whether also CIR should be considered or not.

    - 1st round

Proposal 2-1_v0

Coverage performance in NR NTN is evaluated according to the following steps.

- Step 1: CNR is calculated as defined in 6.1.3.1 of TR38.821

  - FFS: whether to consider additional loss

- Step 2: Required SINR of target service is evaluated by LLS

- Step 3: The CNR and the required SINR are compared

Q: Do you agree the above proposal? If the answer is NO, please share the reason. Note that the FFS is added to discuss e.g. polarization loss, in the next section.

| Company | YES/NO | Comment (e.g. reason of NO, whether also CIR should be considered) |
| --- | --- | --- |
| QC | Yes |  |
| Lenovo | Yes |  |
| Apple | Yes |  |
| Xiaomi | Yes |  |
| vivo | Yes |  |
| Samsung |  | Considering MPL framework from TR38.830 seems complex and may not be relevant since some of parameters have not been defined in TR38.811/821. In this sense, simplifying MPL framework (which is explained in our Tdoc 2203929) can resolve this issue and it can provide the exact coverage range without considering 1) distance between satellite and UE and 2) elevation angle because the required SNR is input parameter of the framework and maximum distance is output parameter of the framework. On the other hand, comparing CNR and the required SINR requires to discuss on which candidate elevation angles and altitudes should be considered for deriving CNR. |
| Nokia, Nokia Shanghai Bell | No | Since satellite antennas will have overlapping coverage areas, it is crucial that these effects are taken into account. As SINR is used in step 2, it is important that the interference is also accounted for in step 1 and step 3. Therefore we suggest to calculate CNIR in step 1 rather than the simpler CNR based on single link conditions. |
| OPPO | Yes |  |
| Huawei, HiSilicon | Yes |  |
| LG |  | The reason why we propose to use MIL is to save discussion time by re-using existing methodology and MIL could be suitable to apply additional UE antenna gain/loss for bottleneck identification. Moreover, since MPL is basically allowed loss for given SNR target, we think there is no fundamental difference between MPL and CNR calculation. We understand both evaluation method are feasible. If majority view supports the proposal, we can support the proposal for our main goal; saving time to discuss. |
| NEC | Yes |  |
| NTT DOCOMO | Yes |  |
| ZTE | Yes | We are generally fine with this proposal. But for LLS, SNR may be evaluated instead of SINR since the interference is unknown. |
| CATT | Yes | One additional comment is that we need to first guarantee the performance of initial access. If the service target is reached, but the initial access including synchronization, system information reception or msg-3 transmission is not successful, then this evaluation is meaningless. |
| Ericsson | Yes |  |
| Thales | Yes | For NTN coverage enhancement study, the evaluation methodology can be based on link-level simulation but as system level simulation provides realistic results for coverage analysis we are open to conduct also system-level simulation for this study. |
| Spreadtrum | Yes |  |
| MediaTek | Yes | We are mainly fine with the proposal. For LLS, SNR may be evaluated as mentioned by ZTE. |
| InterDigital | Yes |  |
| Sony | Yes |  |
| Lockheed | Yes |  |
| ETRI | Yes | we are fine with the proposal |
| Panasonic | Yes | We are basically fine with the proposed steps. Although to use CINR as described in Table 6.1.3.2-1 of TR38.821 would be more aligned with previous evaluation, for the evaluation of the coverage limit, i.e beam edge or far away UEs, CNR should be acceptable direction for this evaluation. |
| Hughes/EchoStar | Yes | Agreed with Thales |
| QC | Yes |  |

    - Summary of 1st round

Most companies are OK with the evaluation steps based on TR38.821. Regarding SINR/SNR and CNIR/CNR, FL’s understanding is that ‘required’ something should use not SNR but SINR, and ‘achievable’ something can use CNR rather than CNIR. Using CNR means that zero interference is assumed as ideal situation. This assumption is valid if frequency/polarization reuse factor > 1, as discussed in section 4.8. However, this understanding might be incorrect. If so, please suggest to update SINR to SNR or CNR to CINR.

Basically most companies are OK with this proposal. FL suggests to discuss this proposal via reflector directly. It is noted that the proposal is updated with red color based on discussion in section 4.3.

    - 2nd round

Proposal 2-1_v1

Coverage performance in NR NTN is evaluated according to the following steps.

- Step 1: CNR is calculated as defined in 6.1.3.1 of TR38.821

  - 3 dB polarization loss is assumed

- Step 2: Required SINR of target service is evaluated by LLS

- Step 3: The CNR and the required SINR are compared

It seems that high-level concept is OK, but there are two controversial parts, which were discussed via reflector but not converged. FL would like to ask the following questions to solve this situation, with the updated proposal based on the email discussion. Frequency reuse factor/polarization reuse factor would be related to this proposal, so let’s discuss this aspect here.

Proposal 2-1_v5

Coverage performance in NR NTN is evaluated according to the following steps.

- Step 1: [CNR or CNIR] is calculated as defined in 6.1.3.1 of TR38.821

  - For polarization loss,

    - Option 1: 3 dB polarization loss is assumed

    - Option 2: 3 dB polarization loss is assumed as baseline, and companies are encouraged to report the value and corresponding justification if other value is used

    - Option 3: UE uses linear polarized antenna and satellite uses circularly polarized antenna, i.e., polarization loss is assumed. Companies are encouraged to report the value and corresponding justification

- Step 2: Required [SNR or SINR] of target service is evaluated by LLS

- Step 3: The [CNR or CNIR] and the required [SNR or SINR] are compared

Q: For proposal 2-1_v5, which option of polarization loss should be supported? Also please share the reason.

| Company | Option | Comment (e.g. reason) |
| --- | --- | --- |
| Samsung | Option 2 | Baseline should be necessary to calibrate link budget results. In that sense, 3 dB might be good candidate as a baseline. Since some companies want to consider other values than 3dB, it’s okay to report the value. |
| vivo | Option 1 or 2 | Due to misalignment on polarization between commercial handheld UE and satellite, 3 dB polarization loss could be usual in practical scenarios. And to find the bottleneck channel, the worst case should also consider this kind of necessary and valid loss. |
| Panasonic | Option 2 |  |
| Xiaomi | Option 2 | Option 2 is preferred as a baseline value is provided for calibration. |
| Spreadtrum | Option 2 |  |
| LG | Option 2 | We don’t have strong view on wording between Option 2 and 3. However, it is beneficial to have baseline assumption, to comparing results each other. |
| ZTE | Option 1 or 2 | In previous TR 38.821, 3dB polarization loss has already been assumed in evaluation and should be at least considered at start point. If there is different values reported by companies, clarification and justification is needed. |
| OPPO | Option 2 | 3dB polarization loss should be assumed as baseline for calibration. |
| Apple | Option 1 or 2 |  |
| Thales | Option 2 | Given the limited TU for the CE study we may consider 3 dB polarization loss as starting point but we think companies are encouraged to report the value and corresponding justification |
| Huawei, HiSilicon | Option 2 | As we commented in the email, we think 3dB should be baseline, and other values can be reported by companies. |
| Nokia, Nokia Shanghai Bell | Option 2 | Similar view as Thales. |
| Ericsson | Option 2 |  |
| Omnispace | Option 2 |  |
| MediaTek | Option 2 |  |
| NEC | Option 2 |  |
| Hughes/EchoStar | Option 2 |  |
| QC | Option 2 |  |

Q: For proposal 2-1_v5, do you think CIR should be considered? The following is FL’s assumption based on email discussion. If you have different view, also please share it.

- If CIR is considered, CIR is calculated according to table 6.1.3.2-1 of TR38.821. This calculation needs SLS.

- If CIR is considered, CNIR and SINR are used in the proposal. Otherwise, CNR and SNR.

| Company | YES/NO | Comment (e.g. reason) |
| --- | --- | --- |
| Samsung |  | We think that comparing CNR and SNR should be baseline for simplicity and comparing CINR and SINR could be considered as alternative approaches. |
| vivo |  | Agree with Samsung. |
| Panasonic |  | We prefer CNR as noise limited situation assuming interference mitigation by frequency/polarization reuse and/or interference coordination. We can use the CNR in Table 6.1.3.3-1 of 38.821. |
| Spreadtrum |  | CNR and SNR is baseline. |
| ZTE |  | We think LLS should be focused. Hence CNR and SNR can be baseline. |
| OPPO |  | We prefer to use CNR and SNR as baseline for coverage evaluation. When frequency/polarization reuse factor >1, the interference can be effectively mitigated. |
| Apple |  | We think CNR and SNR are baseline for coverage evaluation. |
| Thales |  | We are fine to consider only LLS for this study. So, CIR may not be needed. |
| ETRI |  | we prefer CNR and SNR as baseline |
| Huawei, HiSilicon |  | As commented in the email, we prefer to consider the interference considering the increased work seems not complicated. |
| Nokia, Nokia Shanghai Bell | Yes with comments | If CNR+SNR is used, the evaluations will not reflect reality. After all, there will be interference between cells – should we just ignore this? For the moment our preference is to have CNIR+SINR based evaluations. |
| Ericsson | No | Use CNR and SNR. System level simulations are not feasible within the short time frame of the study phase. |
| MediaTek | No | CNR and SNR can be used. There seems to be not enough time for SLS simulation feasibility within short time frame. |
| QC |  | With frequency reuse 3 or higher, the difference between CNR and CINR should not be large. To save time, we can have CNR as the baseline and make adjustment when needed. |

Proposal 8-1_v0

For NR NTN coverage enhancement, assume frequency reuse factor = 3 and polarization reuse factor = 1.

Q. If CIR is considered in proposal 2-1, do you think FRF(frequency reuse factor) / PRF (polarization reuse factor) should be determined for evaluation?

Q’: If YES, which value should be used? Also please share the reason.

| Company | YES/NO for Q | FRF for Q’ | PRF for Q’ | Comment |
| --- | --- | --- | --- | --- |
| vivo | Yes | 3 | 1 |  |
| Panasonic | Yes |  |  | If CIR is considered, FRF/PRF need to be determined because FRF/PRF impacts on CIR. We prefer larger FRF/PRF if CIR is considered. |
| OPPO | Yes | 3 | 1 | We also prefer lager FRF to reduce the interference impact if CIR is considered. |
| Thales | Yes | 3 | 1 |  |
| Huawei, HiSilicon | Yes | 3 and 1 | 1 | For frequency reuse factor=1, this is the most challenging cases for UL coverage. So, if we consider CIR, frequency reuse factor = 1 is preferred. |
| Nokia, Nokia Shanghai Bell | Yes | 3 and 1 | 1 | Probably it could be justified to have FRF of 3, but it will cause a severe drop in system capacity due to the limited resources available per cell. |
| Ericsson | Yes |  |  |  |
| MediaTek | Yes | 3 |  | Our understanding is that FRF=3 is typical value is legacy satellite constellations. Assuming FRF 3, we do not expect significant bias for CNR and SNR if interference is not modelled based on evaluation in Rel-16 NTN SI. |
| Hughes/EchoStar | Yes | 3 | 1 | Fine with us |
| QC | Yes | 3 | 1 |  |

    - Summary of 2nd round

For polarization loss, all companies are OK with option 2, so let’s take option 2.

For CIR, it seems that majority prefer CNR+SNR to save simulation efforts. At the same time, CIR is not so large when FRF = 3 is assumed as commented by QC. Regarding FRF/PRF, many companies prefer to use FRF=3 and PRF=1. FL believes that we do not have much time for this coverage evaluation with the current spec, and it is true that a lot of companies prefer not to perform SLS though HW thinks SLS to consider CIR is not so hard. Based on this, FL recommends to adopt CNR/SNR without agreeing specific FRF/PRF.

    - 3rd round

Proposal 2-1_v6

Coverage performance in NR NTN is evaluated according to the following steps.

- Step 1: CNR is calculated as defined in 6.1.3.1 of TR38.821

  - For polarization loss,

    - Option 2: 3 dB polarization loss is assumed as baseline, and companies are encouraged to report the value and corresponding justification if other value is used

- Step 2: Required SNR of target service is evaluated by LLS

- Step 3: The CNR and the required SNR are compared

Email discussion outcome: Agreed

Then now we can close this section.

  - [Closed] Topic #3: Link budget calculation

For details of link budget calculation, at least four companies [6/Pana] [8/CATT] [15/Apple] [22/Ericsson] seem to assume to reuse section 6.1.3.2 of TR38.821 as baseline. In addition, 7 companies [1/HW, HiSi] [6/Pana] [9/Xiaomi] [11/Samsung] [13/OPPO] [15/Apple] [16/CMCC] refer to polarization loss. [6/Pana] thinks that there is a typical scenario where satellite uses circular polarization but UE can transmit/receive only with linear polarization.

Detailed parameters from Table 6.1.3.2-1 of TR38.821 are dependent on discussions in other sections (e.g. evaluation methodology, shadowing margin, CIR, etc.). FL will prepare question / proposal to agree the detailed parameters for link budget calculation after having progress. For the 1st round, FL would like to ask whether polarization loss should be considered or not, and if the answer is YES, how to consider polarization loss.

    - 1st round

Q: Do you think polarization loss should be considered?

- Q’: If the answer of above Q is YES, how does polarization loss is considered?

  - Option 1: Update formula of link budget calculation

  - Option 2: Consider polarization loss in LLS

| Company | YES/NO | Option | Comment |
| --- | --- | --- | --- |
| QC | Yes |  | Both options are OK. |
| Lenovo | Yes |  | We prefer Option 1. |
| Apple | Yes | Option 1 | In NTN, we could consider polarization loss in link budget calculation. |
| Xiaomi | Yes | Option 1 |  |
| vivo | Yes | Option 1 | Option 1 is preferred. |
| Samsung | Yes | Option 1 | Simpler approach |
| Nokia, Nokia Shanghai Bell | Yes | Option 1 | If UE and satellite is not using same polarization configuration, it is an artifact of the link, which could be accounted for in the link budget. |
| OPPO | Yes | Option 1 | We support Option 1 since the polarization loss has already been included in the link budget calculation in TR38.821. |
| Huawei, HiSilicon | Yes | Option 1 |  |
| LG | Yes | Option 1 |  |
| NEC | Yes | Option 1 | Polarization loss should be taken into account considering different UE capabilities to support polarization (i.e. linear vs circular polarization). |
| ZTE | Yes | Option 1 | We are supportive of option 1. |
| CATT | Yes | Option 1 |  |
| Ericsson | Yes | Option 1 |  |
| Thales | Yes | Option 1 or Option 2 | Both are Ok |
| Spreadtrum | Yes | Option 1 |  |
| MediaTek | Yes |  | Both options can be considered. |
| Ligado | Yes | Option 1 |  |
| Sony | Yes | Option 1 or Option 2 | We think both options are okay. |
| Lockheed | Yes | 2 |  |
| ETRI | Yes | Option 1 or Option 2 | both are okay. |
| Panasonic | Yes | 1 |  |
| Hughes/EchoStar | Yes | 3 | 1 |
| QC | Yes |  | Both are OK. |

    - Summary of 1st round

Most companies think polarization loss should be considered and Option 1 is preferred. There is no objection, therefore, this aspect is reflected in Proposal 2-1.

    - 2nd round

Email discussion with proposal 2-1

    - 3rd round

Now FL would like to trigger discussion on detailed parameter for link budget calculation. As proposed by some companies, basically Table 6.1.3.2-1 of TR38.821 would be baseline but some update might be necessary. Let’s discuss which part should be updated. FL prepares table with some update with red color from the above table based on discussions so far. Rows with yellow color are dependent on other discussions.

Table: Parameter configuration for link budget analysis

| Parameters | Notes |
| --- | --- |
| Carrier frequency | 2 GHz for DL and UL (S-band), 20 GHz for DL and 30 GHz for UL (Ka-band) |
| System bandwidth | 30 MHz (S-band), 400 MHz (Ka-band) |
| Channel bandwidth | DL: system bandwidth/ frequency reuse factor UL: UL in S-band (handheld UE): 360 kHz Otherwise: system bandwidth/ frequency reuse factor Note: The UL bandwidth may be a challenge. FFS |
| Satellite altitude | 600 km, 1200 km, 35786 km |
| Target elevation angle | [30 (LEO), 12.5 (GEO-Set 1) , 20° (GEO –Set 2)] |
| Atmospheric loss | Equation (6.6-8) in [2] |
| Shadowing margin | 0 dB for VSAT as terminal and 3 dB for others |
| Scintillation loss | Section 6.6.6 in [2] Ionospheric loss: = 2.2 dB (note 1) Tropospheric loss: Table 6.6.6.2.1-1 of [2] |
| Additional loss | 0 dB |
| Clear sky conditions | Yes |
| Frequency reuse factor | 1, 2, 3 |
| Average CIR within a satellite beam based on logarithmic mean | Based on single satellite system-level calibration methodology, statistics for average CIR are only collected for the UEs located in the central beam of the 19-beamlayout. The central beam boresight direction is computed based on the target elevation angle assumption. When the generated beam has a partial or full coverage outside the earth, it is discarded. For DL calibration, CIR is computed by averaging CIR over UEs randomly distributed over the reference beam (UE distribution assumption of Table 6.1.1.1-5). (See Figure 6.1.3.2-1 for UE bandwidth allocation, and Figure 6.1.1.1-1 and Figure 6.1.1.1-2 for beam deployment). For UL calibration, For Handheld device, the channel bandwidth is 360 kHz. For VSAT, the channel bandwidth equals the system bandwidth allocated to each beam divided by 10. The devices in one beam are allocated on adjacent frequency resources. The same resource allocation is assumed for all the beams. CIR is computed by averaging over 10 simultaneously transmitting UEs randomly distributed over the reference beam (UE distribution assumption of Table 6.1.1.1-5). (See Figure 6.1.3.2-2 for UE bandwidth allocation, and Figure 6.1.1.1-1 and Figure 6.1.1.1-2 for beam deployment) The averaging should be performed over multiple realizations. |
| Satellite antenna polarization | Circular polarization |
| Polarization reuse | Enable if frequency reuse factor = 2 is considered. |
| Terminal type | [S band: (M, N, P) = (1,1,2)] |
| Free space path loss | Equation (6.6-2) in [2] |
| Terminal RF parameters | Table 6.1.1-3 FFS |
| Satellite RF parameters | Set-1 in Table 6.1.1-1 and Set-2 in Table 6.1.1-2 |
| Polarization loss | The considerations of Section 6.1.1.1 on Polarization loss apply. FFS |
| Outcome | [CNIR] |
| - NOTE 1: Based on P3 curve for 1% of time from Figure 6.6.6.1.4-1 of [2] after frequency scaling. - dB |  |

Note: [2] in this table is 3GPP TR 38.811 v15.2.0: "Study on New Radio (NR) to support non-terrestrial networks (Release 15)"

Q: Do you think the above table can be used with update of rows with yellow color according to outcome of other discussions? If you think further update for other part is necessary, please share it.

| Company | YES/NO | Comment |
| --- | --- | --- |
| vivo | Yes | In addition, if Proposal 2-1 is agreed, the parameter “Average CIR within a satellite beam based on logarithmic mean” could be ignored or deleted. |
| OPPO | Yes |  |
| Hughes/EchoStar | Partial | MEO should be included in the discussion. Section 4.1 of the WID RP-220953 “aims at specifying enhancements for NG-RAN based NTN (non-terrestrial networks) according to the following assumptions, to include: • GSO and NGSO (LEO and MEO) with transparent payload.” |
| MediaTek |  | We support inclusion of MEO as proposed by Echostar. The impact on simulations and evaluation should be reasonable. The row with satellite altitude and target elevation angles should be revised accordingly. |
| Novamint |  | We support Echostar’s proposal to include MEO |
| Lockheed | Partial | We agree MEO should be included in the discussion. |
| Apple |  | The row of “satellite RF parameters” depends on the discussions of Topic #7. We suggest changing it to “FFS”. |
| ESA |  | We support the MEO inclusion |
| QC | Yes |  |
| Panasonic | Yes | Agree with vivo’s comment. |
| Lenovo | Yes |  |
| Xiaomi | Yes |  |
| LG | Yes | Support |
| GateHouse | Yes, but | We agree that MEO should be included. |
| ETRI | Yes |  |
| Thales |  | MEO should be included in the discussion as proposed by Hughes/EchoStar. |
| ZTE | Yes |  |
| Nokia, Nokia Shanghai Bell | Yes |  |
| Samsung | Yes |  |
| Huawei, HiSilicon | Yes |  |
| Ericsson | Yes with comment | We agree that parameters for MEO should be added to facilitate study of (optional) MEO cases. |

    - Summary of 3rd round

Basically, companies are fine to reuse Table 6.1.3.2-1 of TR38.821 to determine detailed parameters for link budget calculation, with appropriate update based on other discussion/agreement.

Now proposal 2-1 is to be agreed, thus ‘Average CIR within a satellite beam based on logarithmic mean’ can be removed as commented by vivo. At the same time, ‘outcome’ becomes CNR.

Proposal 4-1 is also to be agreed, where MEO can be evaluated optionally. As mentioned by several companies, MEO needs to be added in this table.

As commented by Apple, satellite RF parameter is still under discussion. This row should be FFS.

Proposal 3-1_v0

For link budget calculation, parameters in the following table is assumed.

| Parameters | Notes |
| --- | --- |
| Carrier frequency | 2 GHz for DL and UL (S-band) |
| Channel bandwidth | FFS |
| Satellite altitude | 600 km, 1200 km, 35786 km |
| Target elevation angle | [30 (LEO), 12.5 (GEO-Set 1) , 20° (GEO –Set 2), X° (MEO)] |
| Atmospheric loss | Equation (6.6-8) in [2] |
| Shadowing margin | 3 dB |
| Scintillation loss | Section 6.6.6 in [2] Ionospheric loss: = 2.2 dB (note 1) Tropospheric loss: Table 6.6.6.2.1-1 of [2] |
| Additional loss | 0 dB |
| Clear sky conditions | Yes |
| Satellite antenna polarization | Circular polarization |
| Terminal type | [S band: (M, N, P) = (1,1,2)] |
| Free space path loss | Equation (6.6-2) in [2] |
| Terminal RF parameters | FFS |
| Satellite RF parameters | FFS |
| Polarization loss | FFS |
| Outcome | CNR |
| - NOTE 1: Based on P3 curve for 1% of time from Figure 6.6.6.1.4-1 of [2] after frequency scaling. - dB - NOTE 2: [2] in this table is 3GPP TR 38.811 v15.2.0: "Study on New Radio (NR) to support non-terrestrial networks (Release 15)" |  |

    - 4th round

FL think this proposal is a kind of stable, but FL added MEO altitude and updated target elevation angle. FL would like to suggest to have discussion via reflector.

Proposal 3-1_v1

For link budget calculation, parameters in the following table is assumed.

| Parameters | Notes |
| --- | --- |
| Carrier frequency | 2 GHz for DL and UL (S-band) |
| Channel bandwidth | FFS |
| Satellite altitude | 600 km, 1200 km, 10000 km, 35786 km |
| Target elevation angle | [30 (LEO), 12.5 (GEO-Set 1) , 20° (GEO –Set 2), 30° (MEO)] |
| Atmospheric loss | Equation (6.6-8) in [2] |
| Shadowing margin | 3 dB |
| Scintillation loss | Section 6.6.6 in [2] Ionospheric loss: = 2.2 dB (note 1) Tropospheric loss: Table 6.6.6.2.1-1 of [2] |
| Additional loss | 0 dB |
| Clear sky conditions | Yes |
| Satellite antenna polarization | Circular polarization |
| Terminal type | [S band: (M, N, P) = (1,1,2)] |
| Free space path loss | Equation (6.6-2) in [2] |
| Terminal RF parameters | FFS |
| Satellite RF parameters | FFS |
| Polarization loss | FFS |
| Outcome | CNR |
| - NOTE 1: Based on P3 curve for 1% of time from Figure 6.6.6.1.4-1 of [2] after frequency scaling. - dB - NOTE 2: [2] in this table is 3GPP TR 38.811 v15.2.0: "Study on New Radio (NR) to support non-terrestrial networks (Release 15)" |  |

Email discussion outcome: Agreed

Then now we can close this section.

  - [Closed] Topic #4: Satellite orbit

In TR38.821, GEO/LEO-1200/LEO-600 are assumed as satellite orbit. In addition, assumptions of MEO scenario was proposed in RAN#95-e meeting (RP-220590) and endorsed as a useful reference for future consideration. WID states GSO and NGSO (LEO and MEO) with transparent payload, but at the same time, several companies point out that limited scenario should be considered to reduce work for simulation evaluations.

For GEO, 5 companies [1/HW, HiSi] [13/OPPO] [15/Apple] [17/DCM] [20/THALES] propose to evaluate GEO scenario while three companies [6/Pana] [8/CATT] [19/LGE] suggest to deprioritize GEO scenario e.g. since performance is worse and it would not be main target for handset.

For LEO-1200, 8 companies [1/HW, HiSi] [6/Pana] [8/CATT] [13/OPPO] [15/Apple] [17/DCM] [19/LGE] [20/THALES] propose to evaluate LEO-1200 scenario and there is no observation/proposal of different view in any contribution.

For LEO-600, 6 companies [1/HW, HiSi] [8/CATT] [13/OPPO] [15/Apple] [19/LGE] [20/THALES] propose to evaluate LEO-600 scenario while one company [6/Pana] suggest to deprioritize this scenario since basically LEO-600 can achieve better coverage performance than LEO-1200.

For MEO, one company [22/Ericsson] mentions MEO scenario while there is no opinion on MEO in any other contribution.

    - 1st round

Q: Which satellite orbit should be evaluated in this study? Also please share the reason.

- Option 1: GEO

- Option 2: LEO-1200

- Option 3: LEO-600

- Option 4: MEO

| Company | Option | Comment (e.g. why each option should be considered or not be considered) |
| --- | --- | --- |
| QC | 1-3 |  |
| Lenovo | First priority: 2,3. Second priority:1. |  |
| Apple | Options 1, 2, 3 |  |
| Xiaomi | Option 1-3 |  |
| vivo | Option 2 & 3 for VoIP; Option 1 for low data rate service | As mentioned in our contribution, considering latency requirements, VoIP in GEO cannot be supported due to long propagation delay, and VoIP can only be supported in LEO scenario. So VoIP should be studied only in option 2 and option 3. Low data rate service does not require tight latency, and it should be targeted for GEO (and MEO) . A proper low data rate should be determined based on the link budget analysis. Study on GEO may be enough as data rates supported in GEO will anyway be able to supported in MEO, so option 4 may be not necessary. |
| Samsung | Option 2/3 | Considering limited handheld capabilities and VoIP’s general latency requirement (100~200ms), it might be better to focus on LEO cases. However, with our responses in Topic#2, we can consider more general LEO coverage between 300km ~ 2000km, and then we will be able to observe how much coverage current coverage schemes (or enhanced coverage schemes) can achieve. We believe that such observations could provide more insights instead of just considering 600/1200km cases only. Please note that all parameters are same for LEO-1200 and LEO-600 in case of UL transmission from Set-1 and Set-2 satellite parameters as shown in TR38.821. |
| Nokia, Nokia Shanghai Bell | Option 1+3 | LEO-1200 and MEO are already implicitly covered by options 1+3. The GEO case takes care of the high path loss (and low elevation angle), while the LEO-600 showcases the issues of a highly fluctuating channel (due to the high Doppler). |
| OPPO | Option 1,2,3 | The set-1 and set-2 satellite parameters in TR38.821 should be reused in this study. |
| Huawei, HiSilicon | Option 1-3 prioritized | 1-3 can be prioritized. It can be FFS on whether MEO should be evaluated, which could be decided based on the study of cases of option 1-3. For example, if GEO can be enhanced to support the target data rate then MEO is not need to be evaluated. |
| LG | Option 2, 3 |  |
| NEC | Option 1 and Option 2 | The objective of the work item is to support connectivity for commercial UEs and the main use case is that commercial UEs mainly access the NTN in absence of coverage of any TN. To achieve the above objectives, it is preferable to study connectivity to GEO and LEO-1200 which are expected to cause most of the issues for UE’s UL coverage. |
| NTT DOCOMO | Option 1,2,3 |  |
| ZTE | First priority: 2, 3 Second priority: 1 | In LEO, the performance is much worse than LEO. The performance gap may not able to be mitigated even with coverage enhancements. Hence, LEO scenarios should be first priority. |
| CATT | Option 1-3 |  |
| Ericsson | 1,2,3,4 | Evaluate baseline coverage of all to get the full picture. Then down-select if needed. |
| Thales | Option 1, 2, 3 | The coverage study cases: LEO-600 based on Set 2 Satellite parameter set LEO-1200 based on Set 2 Satellite parameter set GEO based on Set 1 Satellite parameter set |
| Spreadtrum | Options 1, 2, 3 |  |
| MediaTek | Option 3 | LEO-600 can be first priority. |
| Ligado | Option 1, 2 and 3 | GEO has some performance challenges but wide area coverage advantages. |
| InterDigital | Option 1-3 |  |
| Sony | Options 1,2,3 | As per previous NTN studies, once a spreadsheet is agreed, the extra work required for multiple scenarios is not significant. This evaluation will help the group understand which satellite orbits are feasible for the VOIP / low data rate services of the WID |
| Lockheed | 1,2,3 | We believe the GEO and LEO scenarios are most relevant. There are important use cases for GEO other than for a handset. |
| ETRI | Option 1,3 | the option {1, 3} might cover the other options {2,4}. |
| Panasonic | First priority: 2 Second priority: 1, 2 | LEO should be prioritized as the main target for handheld terminal. LEO-600 can be de-prioritized because LEO-1200 is more severe condition for coverage. |
| Hughes/EchoStar | Option 1, 2,4. Option 3 (0ptional) | Important to in included MEO. Assumptions of MEO scenario was proposed in RAN#95-e meeting (RP-220590) and endorsed as a useful reference for future consideration. This should be included to complete assessment for NGSO. |
| GateHouse | Option 1, 2 and 3 | The GEO and LEO scenarios should be used. |

    - Summary of 1st round

21 companies think GEO should be evaluated. 21 companies believe LEO-1200 should be evaluated. 22 companies think LEO-600 should be considered. 1 company suggests to include MEO in this study. Although several companies prefer not to include some options, FL thinks based on these inputs that GEO/LEO-1200/LEO-600 are assumed in this study.

Besides, several companies state that a scenario evaluation would be covered by another scenario evaluation and VoIP is not considerable in GEO scenario. FL thinks this comment is valid and it is beneficial to restrict evaluation scenario in order to reduce simulation efforts and subsequent discussions. This aspect would be related to discussion in section 4.1. Let’s discuss there. In addition, parameter set perspective is mentioned in the 1st round. FL assumes it is discussed in section 4.7/4.8.

    - 2nd round

Proposal 4-1_v0

Coverage performance in NR NTN is evaluated for GEO/LEO-1200/LEO-600 scenarios.

Q: Do you agree the above proposal? If the answer is NO, please share the reason and how to modify.

| Company | YES/NO | Comment (e.g. reason of NO) |
| --- | --- | --- |
| vivo | Yes | Fine with the proposal. Furthermore, as our comment in 1st round, we can focus coverage performance for VoIP on LEO-1200 and LEO-600, and focus coverage performance for low-data rate service on GEO. |
| MediaTek | Yes |  |
| Lockheed | Yes |  |
| Apple | Yes |  |
| Lenovo | Yes |  |
| Samsung | YES |  |
| Panasonic | Yes | In addition, it would be necessary to decide parameter set for each scenario. LEO-600/1200 based on Set 2 and GEO based on Set 1 would be a good candidate. |
| Xiaomi | Yes |  |
| Spreadtrum | Yes |  |
| LG | Yes |  |
| NTT DOCOMO | Yes |  |
| ZTE | Yes (Partly) | LEO can be prioritized over GEO. |
| OPPO | Yes |  |
| Thales | Yes | In our view the following coverage study cases should be considered: - LEO-600 based on Set 2 Satellite parameter - LEO-1200 based on Set 2 Satellite parameter - GEO based on Set 1 Satellite parameter |
| ETRI | Yes |  |
| Huawei, HiSilicon | Yes | Besides the satellite orbit attitude, we should also decide which scenario should be evaluated. Sub-urban scenario can be a candidate. And it is not clear from this proposal whether both set1 and set2 are considered. |
| Nokia, Nokia Shanghai Bell | No with comment | It should be noted that the GEO case would not be able to fulfill the latency and processing requirements of VoIP. Maximum orbital height for VoIP would most likely be LEO-1200 (for path loss) and LEO-600 (for channel dynamics) |
| Ericsson | Yes |  |
| Omnispace | Yes |  |
| NEC | Yes |  |
| Hughes/EchoStar |  | In our view important to evaluate for MEO as well - LEO-600 based on Set 2 Satellite parameter - MEO based on Set 1 Satellite parameter GEO based on Set 1 Satellite parameter |
| Lockheed | Yes (updated) | We believe the GEO and LEO scenarios are most relevant, however we also support Hughes/Echostar for the inclusion of MEO. |
| QC | Yes |  |
| MediaTek |  | MEO should be included as proposed by Echostar |

    - Summary of 2nd round

Most companies are OK with proposal 4-1. There are several comments for service type and parameter set, but as FL said above, these aspects are discussed separately. Let’s update the proposal with small clarification.

    - 3rd round

Proposal 4-1_v1

Coverage performance in NR NTN is evaluated for GEO/LEO-1200/LEO-600 scenarios.

- Note: Service type for each scenario is discussed separately

- Note: Parameter set (Set-1/2) is discussed separately

Email discussion outcome:

Proposal 4-1_v3

Coverage performance in NR NTN is evaluated for GEO/LEO-1200/LEO-600 scenarios.

- Note: Service type for each scenario is discussed separately

- Note: Parameter set (Set-1/2) is discussed separately

- Note: MEO can be evaluated optionally

Email discussion outcome: Agreed

Then now we can close this section.

  - [Email] Topic #5: UE characteristics including antenna gain

As UE type, it seems that most companies including 9 companies [1/HW, HiSi] [6/Pana] [8/CATT] [9/Xiaomi] [13/OPPO] [15/Apple] [16/CMCC] [19/LGE] [20/THALES] assume handset terminals in this study. Meanwhile, one company [22/Ericsson] proposes to evaluate also VSAT.

In WID, it is stated that target service for NR NTN coverage enhancement is VoIP and low-data rate services for commercial handset terminals. FL thinks this WID text means handset terminals are the UE characteristics to be evaluated in this study and thus coverage enhancement for VSAT UE is not assumed. FL would like to ask whether this understanding is correct or not.

For details of UE characteristics, three companies [1/HW, HiSi] [6/Pana (?)] [20/THALES] propose to reuse ‘handheld’ row in Table 6.1.1.1-3 of TR38.821 except for TX/RX antenna gain.

On antenna gain, it is mentioned in WID that more realistic assumptions on antenna gains instead of 0dBi should be determined in WG. One company [8/CATT] proposes -3 dBi, 6 companies [5/vivo] [6/Pana] [17/DCM] [19/LGE] [20/THALES] [22/Ericsson] propose to apply -5 dBi, and one company [9/Xiaomi] proposes -6 dBi. In addition, [22/Ericsson] thinks that RAN4 should determine the realistic assumptions on antenna gains. FL does not find any absolute reason to use each value in any contribution, but as abovementioned, we need to decide a certain antenna gain in this meeting. It might be doubtful whether RAN1 can ask it to RAN4.

    - 1st round

Proposal 5-1_v0

For NR NTN coverage enhancement, evaluate only handset terminals as UE type.

- i.e., VSAT is not considered.

Q: Do you agree the above proposal? If the answer is NO, please share the reason.

| Company | YES/NO | Comment (e.g. reason of NO) |
| --- | --- | --- |
| QC | Yes | VSAT does not need coverage enhancement. |
| Lenovo | Yes |  |
| Apple | Yes |  |
| Xiaomi | Yes |  |
| vivo | Yes |  |
| Samsung | Yes |  |
| Nokia, Nokia Shanghai Bell | Yes | Basically, if a UE operating as VSAT, it would most likely have quite good conditions for connecting to the NTN system and would hence be out of scope for these studies. |
| OPPO | Yes |  |
| Huawei, HiSilicon | Yes | As the link budget of VSAT is much better than handheld UEs, there is no need to evaluate it, considering the enhancements introduced for handheld can be applied to VAST directly if needed. |
| LG | Yes |  |
| NEC | Yes | We do not expect to see coverage issues with VSAT UEs because they are expected to have careful placement considering direct LoS. The coverage related issues are expected to be apparent mainly for commercial or handset UEs with low transmission power capability. |
| NTT DOCOMO | Yes | Since the objective of this AI is to evaluate coverage when using smartphones with more realistic assumptions, VSAT should not be considered. |
| ZTE | Yes |  |
| CATT | Yes |  |
| Ericsson | Yes | The WID objectives are a bit unclear since it is stated in the general objectives in section 4.1 in the WID that "Both “VSAT” devices with directive antenna (including fixed and moving platform mounted devices and commercial handset terminals (e.g. Power class 3) are supported in FR1" while the objectives specific to coverage enhancements in 4.1.1 state that "The work needs to cover the use case of voice and low-data rate services using commercial smartphones with more realistic assumptions on antenna gains instead of 0dBi currently assumed for link budget analysis for non-terrestrial networks.". The latter doesn't explicitly exclude that also other use cases (i.e., VSAT) are covered in the work. But if it is the common understanding that VSAT is not to be considered, then we are fine with that. |
| Thales | Yes | Smartphones with their ubiquity are the most important handheld devices. The study should focus on smartphones with more realistic antenna gain (yet to be defined). |
| Spreadtrum | Yes |  |
| MediaTek | Yes |  |
| InterDigital | Yes |  |
| Sony | Yes |  |
| Lockheed | Yes | We believe the antenna gain considered should be -5 dBi. We are not sure it is necessary to ask RAN4 at this time. |
| Panasonic | Yes |  |
| Hughes/EchoStar | Yes |  |
| QC | Yes |  |

Q: Which option should be agreed for UE TX/RX antenna gain? Also please share the reason.

- Option 1: -3 dBi

- Option 2: -5 dBi

- Option 3: -6 dBi

- Option 4: Send an LS to RAN4 to ask the value

| Company | Option | Comment (e.g. why the option is the most valid one) |
| --- | --- | --- |
| QC | Option 2 | Option 2 reflects better the reality. |
| Lenovo | Option 2 |  |
| Xiaomi | Option 3 | Option 2 is too restrictive for handheld, we better relax the requirement especially when calculating the link budget. |
| vivo | Option 2 | Option 2 is more close to the actual antenna gain. |
| Nokia, Nokia Shanghai Bell | Option 4 | RAN4 should be the experts on this topic. |
| Huawei, HiSilicon | Option 2 |  |
| LG | Option 2 |  |
| NTT DOCOMO | Option 4 | Since it concerns RF devices, it would be better to ask RAN4 for the exact value. |
| ZTE | Option 2 | We are also open for option 4 |
| CATT | Option 4 |  |
| Ericsson | Option 2, Option 4 | RAN1 can take -5 dBi as working assumption and ask RAN4 to either confirm or propose a different value. In general, it would be good to have more justification for the proposed values. |
| Thales | Option 2 (for now) And Option 4 | As mentioned by Moderator we need to decide a certain antenna gain in this meeting. The assumption on antenna gain could be -5dBi (as this reflects better actual antenna gain: see QC and vivo comments) but this does not preclude to send an LS to RAN4. |
| Spreadtrum | Option 4 |  |
| MediaTek | Option 2 | Option 2 can be the baseline. Open to Option 4 “Send an LS to RAN4 to ask the value” |
| InterDigital | Option 2 |  |
| Sony | Option 2 | We don’t see a need to send an LS to RAN4. The link budget is challenging enough and a 1dB difference in antenna gain assumption is going to make little difference to the feasibility of this work. If the majority of companies want to involve RAN4, we would not oppose, but involving RAN4 would seem to incur unnecessary delay and extra work for RAN4. |
| Lockheed | 2 | It is a good compromise. |
| ETRI | Option 4 | Same view as Nokia |
| Panasonic | Option 2 |  |
| Hughes/EchoStar | Option 2 and 4 |  |

Proposal 5-2_v0

Evaluate coverage performance for the following UE characteristics as in Table 6.1.1.1-3 of TR38.821 except for Tx/Rx antenna gain.

| Characteristics | Handheld |
| --- | --- |
| Frequency band | S band (i.e. 2 GHz) |
| Antenna type and configuration | (1, 1, 2) with omni-directional antenna element |
| Polarisation | Linear: +/-45°X-pol |
| Rx Antenna gain | [X] dBi per element |
| Antenna temperature | 290 K |
| Noise figure | 7 dB |
| Tx transmit power | 200 mW (23 dBm) |
| Tx antenna gain | [X] dBi per element |

Q: If proposal 4-1 5-1 is agreed, do you agree the above proposal? If the answer is NO, please share the reason.

| Company | YES/NO | Comment (e.g. which column should be modified) |
| --- | --- | --- |
| QC |  | Need further discussion on antenna polarization. |
| Lenovo | Yes |  |
| Apple |  | Not sure what is proposal 4-1. |
| Xiaomi | Yes |  |
| vivo | Yes |  |
| Nokia, Nokia Shanghai Bell | Yes | We cannot find any proposal 4-1, but assume that this is related to the exclusion of VSAT operation. |
| Huawei, HiSilicon | Yes | We are fine with the above proposal assuming the “proposal 4-1” should be “proposal 5-1”. Also, 1Tx/2Rx should be assumed for UE for link level simulation. |
| LG |  | Should it be related to proposal 5-1? |
| NTT DOCOMO | Yes |  |
| ZTE |  | Need further discussion |
| CATT |  | Not sure if the UE polarization mode is common understanding. |
| Ericsson | Yes |  |
| Thales | Yes | Antenna polarization maybe further discussed |
| Spreadtrum | Yes |  |
| MediaTek |  | It can be further discussed |
| Ligado | Yes with amendment | It is unclear what 45 degrees linear polarization means in a hand held terminal. The orientation of the polarization will depend on the orientation of the hand set with respect to the ground (the usual reference for defining polarization). Furthermore, the effect of the hand and the body will alter the orientation of the linear polarization. Suggest removing +/- 45 degree qualifier to linear polarization. |
| Sony |  | The antenna polarization assumption needs to be further discussed. The antennas in commercial devices (e.g., smart phones) in FR1 usually do not support dual linear polarizations. The antenna type and configuration should be consistent with commercial devices not supporting dual linear polarizations. |
| Lockheed | Yes | Antenna polarization may be further discussed. |
| Panasonic | Yes |  |
| Hughes/EchoStar | OK |  |

    - Summary of 1st round

All companies are OK to evaluate coverage performance of only handset terminals.

On UE TX/RX antenna gain, 13 companies think -5 dBi is the most appropriate value or can be used as working assumption. One company believes -6 dBi should be used instead. 7 companies including some of above 13 companies suggest to ask RAN4. RAN1 has only one additional meeting to conclude this study phase. Option 4 would be the most ideal direction, but we need to decide a certain value in this meeting. Therefore, as Ericsson suggests, it would be good to use -5 dBi for subsequent discussion as working assumption and at the same time to ask to RAN4 whether this value is valid or not.

Regarding detailed UE characteristics except antenna gain, basically companies are OK to reuse the table, but there are several comments for polarization aspect. Based on comments from Ligado/Sony, it seems that ‘+/-45°X-pol’ should be removed. Companies might have different preference on how to update polarization row; FL would like to here companies’ views.

    - 2nd round

Proposal 5-2_v1

Evaluate coverage performance for the following UE characteristics as in Table 6.1.1.1-3 of TR38.821 with update of polarization and Tx/Rx antenna gain.

| Characteristics | Handheld |
| --- | --- |
| Frequency band | S band (i.e. 2 GHz) |
| Antenna type and configuration | (1, 1, 2) with omni-directional antenna element |
| Polarisation | Linear |
| Rx Antenna gain | -5 dBi per element |
| Antenna temperature | 290 K |
| Noise figure | 7 dB |
| Tx transmit power | 200 mW (23 dBm) |
| Tx antenna gain | -5 dBi per element |

- Send an LS to RAN4 to ask whether above antenna gain is valid and if invalid, appropriate value.

Q: Do you agree the above proposal? If the answer is NO, please share the reason and how to modify.

| Company | YES/NO | Comment (e.g. reason of NO) |
| --- | --- | --- |
| vivo | Yes |  |
| MediaTek | Yes (Partly) | Diversity techniques with 2TX should not be precluded at this stage |
| Lockheed | Yes (Partially) | We agree with MediaTek, 2x2 polarization should not be excluded. |
| Lenovo | Yes |  |
| Samsung | Yes |  |
| Panasonic | Yes |  |
| Xiaomi | Yes (Partly) | We think the -5dBi antenna gain is too strict for UE. As the note implicit that RAN4’s input is necessary, we suggest to put brackets to show tentative value in the LS. We agree the other part for UE characteristics. Characteristics; Handheld. Frequency band; S band (i.e. 2 GHz). Antenna type and configuration; (1, 1, 2) with omni-directional antenna element. Polarisation; Linear. Rx Antenna gain; [-5] dBi per element. Antenna temperature; 290 K. Noise figure; 7 dB. Tx transmit power; 200 mW (23 dBm). Tx antenna gain; [-5] dBi per element. |
| Spreadtrum | Yes |  |
| LG | Yes |  |
| NTT DOCOMO | Yes |  |
| ZTE | Yes (Partly) | 2 Tx should not be precluded at this stage. |
| OPPO | Yes |  |
| Thales | Yes |  |
| ETRI | Yes |  |
| Huawei, HiSilicon | Yes |  |
| Nokia, Nokia Shanghai Bell | Yes with comments | It could be OK to use -5 dBi per element as working assumption while RAN4 is consulted. |
| Ericsson | Yes | We support Xiaomi's proposal to put the antenna gain in square brackets. |
| Omnispace | Yes |  |
| Hughes/EchoStar | OK |  |
| QC | Partly OK with comments | On UE antenna, (1,1,2) means 2 cross-polarized antennas that is not typical for 2 GHz. Suggest change to 2 Rx and 2 Tx. |

    - Summary of 2nd round

Companies are OK in high-level, while there are comments on two aspects.

- Regarding antenna gain, it should be clarified that -5 dBi is working assumption and might be updated based on RAN4’s input. FL thinks this comment is valid.

- For UE antenna, 2 TX / 2 RX should be used instead of (1,1,2) and thus diversity techniques with 2 TX should be allowed. FL understands it and let’s reflect this comment.

    - 3rd round

Proposal 5-2_v2

Evaluate coverage performance for the following UE characteristics as in Table 6.1.1.1-3 of TR38.821 with update of polarization, Tx/Rx antenna gain, and antenna type and configuration.

| Characteristics | Handheld |
| --- | --- |
| Frequency band | S band (i.e. 2 GHz) |
| Antenna type and configuration | 2 TX/RX with omni-directional antenna element |
| Polarisation | Linear |
| Rx Antenna gain | [X] dBi per element |
| Antenna temperature | 290 K |
| Noise figure | 7 dB |
| Tx transmit power | 200 mW (23 dBm) |
| Tx antenna gain | [X] dBi per element |

- X = -5 as working assumption

  - Send an LS to RAN4 to ask whether above antenna gain is valid and if invalid, appropriate value.

- 2 TX diversity techniques can be considered.

Q. Only if you are not OK with this proposal, please share view.

| Company | Comment |
| --- | --- |
| vivo | Support the proposal. We assume “2 TX/RX with omni-directional antenna element” means 2 TX and 2 RX. |
| Lockheed | We support the proposal. |
| OPPO | The antenna configuration is necessary for LLS, but 2Tx/2Rx is not clear for us, e.g., it can be configured as (1,1,2) or (1,2,1). |
| MediaTek | We support the proposal. |
| QC | Support |
| Lenovo | Support. |
| Xiaomi | Support |
| LG | Yes |
| Thales | Support |
| ZTE | Support the proposal |
| Nokia, Nokia Shanghai Bell | If UE and chipset manufacturers are OK with having mandatory support for 2 TX antennas, we would be OK. |
| Samsung | Fine with proposal. Considering future meetings (i.e., no meeting until August) for RAN1 and RAN4, it seems not feasible to get information from RAN4 on time. |
| Huawei, HiSilicon | - We share similar comment with OPPO. 2Tx/2Rx is not clear, it can be (1,1,2) / (2,1,1) / (1,2,1). We prefer to keep the original assumption (1, 1, 2) with omni-directional antenna element reusing the assumption in Table 6.1.1.1-3 of TR38.821. - We are discussing UE assumption here for evaluation, and why should we add a bullet for coverage enhancement techniques. Propose to remove the last bullet: “2 TX diversity techniques can be considered”. |

    - Summary of 3rd round

Most companies are OK with proposal, but there are two comments on antenna configuration. QC commented in 2nd round that (1,1,2) is not typical for 2 GHz. But at the same time, TR38.821 assumes this configuration as commented by HW, and clarification of 2 TX/RX is recommended by OPPO. FL lists two options of antenna configuration, and recommend to select one option in GTW session.

Regarding 2 TX diversity techniques, FL’s understanding is that this is only mechanisms already specified in Rel-15/16/17. New mechanism is not intended. This aspect is clarified.

Proposal 5-2_v3

Evaluate coverage performance for the following UE characteristics as in Table 6.1.1.1-3 of TR38.821 with update of polarization, Tx/Rx antenna gain, and antenna type and configuration.

| Characteristics | Handheld |
| --- | --- |
| Frequency band | S band (i.e. 2 GHz) |
| Antenna type and configuration | 2 TX/ 2 RX of Option 1: (1,1,2) Option 2: (1,2,1) with omni-directional antenna element |
| Polarisation | Linear |
| Rx Antenna gain | [X] dBi per element |
| Antenna temperature | 290 K |
| Noise figure | 7 dB |
| Tx transmit power | 200 mW (23 dBm) |
| Tx antenna gain | [X] dBi per element |

- X = -5 as working assumption

  - Send an LS to RAN4 to ask whether above antenna gain is valid and if invalid, appropriate value.

- 2 TX diversity techniques that are specified in Rel-15/16/17 can be considered.

    - 4th round

Based on short email discussion before GTW session, concern on antenna configuration and TX diversity techniques were raised.

For antenna configuration, HW thinks that 1 TX is mandatory and 2 TX should be optional. In addition, we still have options of detailed antenna configuration. FL prepares questions for further update.

On TX diversity techniques, FL thought that this intends existing mechanism in Rel-15/16/17, but LGE pointed out that there is no TX diversity mechanism in current spec. If this is correct, then FL is not sure how TX diversity techniques are applied. In this study, whether coverage performance with existing mechanism in Rel-15/16/17 is evaluated. New mechanism with future spec impact is not evaluated. Only if TX diversity mechanism has no spec impact (i.e. up to UE implementation), then the application is allowed. To clarify these perspectives, FL updates the last bullet as two options, and would like to ask what is the correct intention of the bullet. At least some companies suggest to remove the bullet already.

Proposal 5-2_v5

Evaluate coverage performance for the following UE characteristics as in Table 6.1.1.1-3 of TR38.821 with update of polarization, Tx/Rx antenna gain, and antenna type and configuration.

| Characteristics | Handheld |
| --- | --- |
| Frequency band | S band (i.e. 2 GHz) |
| Antenna type and configuration | Option I: 2 TX / 2 RX Option II: 1 TX, 2TX (optional) / 2 RX of Option 1: (1,1,2) Option 2: (1,2,1) with omni-directional antenna element |
| Polarisation | Linear |
| Rx Antenna gain | [X] dBi per element |
| Antenna temperature | 290 K |
| Noise figure | 7 dB |
| Tx transmit power | 200 mW (23 dBm) |
| Tx antenna gain | [X] dBi per element |

- X = -5 as working assumption

  - Send an LS to RAN4 to ask whether above antenna gain is valid and if invalid, appropriate value.

- Option A: 2 TX diversity techniques that are specified in Rel-15/16/17 can be considered.

- Option B: 2 TX diversity techniques that are performed by UE implementation without any spec impact can be considered.

- Option C: (Remove any bullet for 2 TX diversity techniques)

Q: In the above proposal, which option is preferred?

| Company | Option |  |  | Comment (e.g. why the option should be selected) |
| --- | --- | --- | --- | --- |
|  | I or II | 1 or 2 | A or B or C |  |
| Panasonic | II | 1 (as in 38.821) | C | Baseline performance should be evaluated in the study phase. |
| Xiaomi | II | 1 | C | We think only the (1,1,2) should be kept in UE Characteristic table. How many Tx chains can be discussed in channel-specific parameters. |
| Nokia, Nokia Shanghai Bell | I | No particular preference | C | 1 TX with 2 TX as optional to estimate the coverage gap would be needed to establish the worst case scenario from propagation point of view. If 2 TX is optional, there is no need to discuss any TX diversity techniques. |
| ZTE | I | 1 | C | Since 2 TX diversity may provide gain for evaluation, we prefer option I for alignment. And option 1 (1,1,2) can be reused from TR 38.821. For the bullet of diversity, we support option C, i.e., remove the bullet, since no TX diversity techniques has been specified. It is more of implementation issue and does not need to be stated here. |
| Thales | I | 1 | C |  |
| Apple | II | 1 | C | We think the Tx diversity techniques are not considered to align evaluation results from companies. |
| Ericsson | I or II | 1 | C | Option 1 since it is the assumption in TR 38.821. Option C since there is no need to state this. It is obvious that if 2 TX is assumed, TX diversity techniques can be used. And for baseline performance evaluation, only techniques that have no spec impact (compared to Rel-17) are allowed. |
| ETRI | II | 1 | C | Option II) As provided in 38.821, we think that at least 1 TX should be included. We are open to adding other TX antenna options. Option 1) As provided in 38.821, we slightly prefer (1,1,2) |
| Huawei, HiSilicon | II | 1 | C | Option II more aligns with existing assumption. |

    - Summary of 4th round

Option I vs Option II

Option I: Nokia/ZTE/Thales/Ericsson (4)

Option II: Pana/Xiaomi/Apple/Ericsson/ETRI/HW (6)

Number of supporting companies are not so different; in this case, FL recommends to take Option II since 1 TX is aligned with TR38.821. Of course 2 TX can additionally be reported if certain gain is obtained. Conclusion of this study might be determined based on the optional case.

Option 1 vs Option 2

Majority are OK to go with Option 1. This option is aligned with TR38.821. Then let’s take Option 1.

Option A vs Option B vs Option C

All companies think text for TX diversity is unnecessary. Let’s remove this text.

    - 5th round

Proposal 5-2_v6

Evaluate coverage performance for the following UE characteristics as in Table 6.1.1.1-3 of TR38.821 with update of polarization, Tx/Rx antenna gain, and antenna type and configuration.

| Characteristics | Handheld |
| --- | --- |
| Frequency band | S band (i.e. 2 GHz) |
| Antenna type and configuration | 1 TX, 2TX (optional) / 2 RX of (1,1,2) with omni-directional antenna element |
| Polarisation | Linear |
| Rx Antenna gain | [X] dBi per element |
| Antenna temperature | 290 K |
| Noise figure | 7 dB |
| Tx transmit power | 200 mW (23 dBm) |
| Tx antenna gain | [X] dBi per element |

- X = -5 as working assumption

  - Send an LS to RAN4 to ask whether above antenna gain is valid and if invalid, appropriate value.

- Option A: 2 TX diversity techniques that are specified in Rel-15/16/17 can be considered.

- Option B: 2 TX diversity techniques that are performed by UE implementation without any spec impact can be considered.

- Option C: (Remove any bullet for 2 TX diversity techniques)

Email discussion

  - [Closed] Topic #6: Band / Bandwidth

On carrier frequency for simulation evaluation, 7 companies [1/HW, HiSi] [6/Pana] [8/CATT] [9/Xiaomi] [13/OPPO] [15/Apple] [20/THALES] propose S-band (i.e. 2 GHz). In table 6.1.1.1-3 of TR38.821, only S-band is assumed for handheld UE; therefore, FL think that to adopt only S-band as proposed by companies is straightforward if only handheld UE is considered in this topic. However, UE type is not determined yet as asking in section 4.4 above. Question for carrier frequency is included in proposal 4-2 of section 4.4 or separately prepared after the decision of proposal 4-1, if needed.

For bandwidth, two companies [1/HW, HiSi] [6/Pana] propose 30 MHz for DL and 360 kHz for UL according to TR38.821 while one company [15/Apple] proposes 20 MHz based on TR38.830. FL would like to ask which assumption should be used for this study.

    - 1st round

Q: Which bandwidth should be assumed in this study? Also please share the reason.

- Option 1: 30 MHz for DL and 360 kHz for UL (i.e. aligned with TR38.821)

- Option 2: 20 MHz (i.e. aligned with TR38.830)

- Option 3: others

| Company | Option | Comment (e.g. why the option should be selected) |
| --- | --- | --- |
| QC |  | Not sure if an assumption of the BW is important. |
| Lenovo | Slightly prefer Option 2 | 20MHz is more popular for smart phone. |
| Apple | Option 2 |  |
| Xiaomi | Option 2 | The actual UL bandwidth used for UL transmission should be used for link budget calculation |
| vivo | Option 3 | In our understanding, UL bandwidth would influence the evaluation results. In link budget calculation, doubling UL bandwidth would result in 3dB loss. Although doubling UL bandwidth could decrease the code rate, the BLER performance would not always gain 3dB improvement, generally lower, especially when code rate is very low. Depending on the target data rate, a feasible RB allocation or UL bandwidth could be decided. For VoIP, we propose 4.75kbps, 15kHz SCS and 360 kHz band allocation (i.e. 2 PRBs). For low data rate service, we could start with 1 or 2 PRBs, and try different MCS indexes with modulation order no larger than 2. A final combination of {PRB allocation, MCS index} selected is the one with which best link budget can be achieved for GEO scenario. |
| Samsung | Option 3 (FFS) | It should be discussed together how much target rate should be achieved for VoIP and low-data rate service. Also, it should be discussed together about which sub-carrier spacing is considered since it affects the number of PRBs. |
| Nokia, Nokia Shanghai Bell | Option 3 | For system bandwidth, 20 MHz should be used to ensure that we can support UE multiplexing. For DL operation the used bandwidth should be equal to the needed bandwidth for the given channel (SSB, PDCCH, PDSCH), but the spectral density of the DL signal should remain constant. For UL operation should be the target UE bandwidth, which could be 360 kHz and this would be the value to use for the link budget calculations. |
| OPPO | Option 3 | It should be clarified that the bandwidth is the UL/DL transmission bandwidth in the link-level simulation, which is channel-dependent, e.g., PRACH. Therefore, it is impartial to use one value to cover all channels in section 4.9. In our opinion, the channel-specific bandwidth should be used in the link budget analysis. |
| Huawei, HiSilicon |  | Maybe it is better to firstly clarify what bandwidth refers to here. The carrier bandwidth or BWP bandwidth may impact the evaluation of frequency hopping. |
| LG | Option 1 or 2 | Fine with either but not both. |
| NTT DOCOMO | Option 1 | We interpret this proposal as referring to channel bandwidth, not system bandwidth. UE transmit power is limited, so UL channel bandwidth should be minimized to increase PSD. |
| ZTE |  | For LLS, the bandwidth is not important. There is no need to define such parameter in UL and DL LLS. For UL, further evaluation can be conducted for link budget analysis. |
| CATT | Option 1 |  |
| Ericsson | Option 3 | Leave it open for now. Different bandwidths could be evaluated to compare coverage (especially if Power Flux Density limitation is to be considered). |
| Thales | Option 3 | For uplink in case of PRACH long format 2: 1.048 MHz is required. So, option 1 is not appropriate. Option 2 may be ok. But we do not think it is needed to agree on bandwidth assumption. Overall, required bandwidth will depend on the channel. |
| Spreadtrum |  | We share the similar views with HW. |
| MediaTek | Option 3 | Depending on VoIP requirements and data rate requirements, 180 kHz for UL can be considered to improve UL link budget? For DL, the BW assumption can also be discussed. |
| Ligado | Option 3 | 5 MHz DL and 180 kHz UL should be considered. |
| Sony | Option 3 | The bandwidth used should be whatever is required to support the data rate. An UL bandwidth of 360kbps seems to be too high for supporting a 10kbps UL low data rate service (or low rate VOIP codec). |
| Lockheed | 2 | This is more consistent with NR. |
| Panasonic | Option 1 | Our understanding is the intention of 360kHz for uplink is PUSCH BW. |
| Hughes/EchoStar | Other | If referring to NR channel bandwidth, should consider 5 and 10 MHz DL |

    - Summary of 1st round

For bandwidth, Option 1 is supported by 4 companies; Option 2 is supported by 4 companies. On the other hand, a lot of companies think bandwidth assumption is not important and it is unclear whether this ‘bandwidth’ means carrier bandwidth or BWP bandwidth or each channel bandwidth. FL’s understanding was that carrier bandwidth is not important in this study, BWP bandwidth might have impact on FH as commented by HW, and channel bandwidth would be dependent on each channel / data rate / MCS / format / etc. Based on the inputs, FL would like to ask whether each bandwidth needs to be agreed or not.

In addition, as discussed in section 5, to evaluate only handset terminals seems agreeable. FL assumes that S-band (i.e. 2 GHz) is the carrier frequency that should be evaluated for the UE type. As 7 companies propose S-band in their contribution, it would be straightforward to agree S-band. This aspect is already included in Proposal 5-2, so discussion on this perspective in this section is skipped.

    - 2nd round

Q: Do you think carrier bandwidth should be agreed? If the answer is YES, please share the reason and the value.

| Company | YES/NO | Comment (e.g. reason of NO) |
| --- | --- | --- |
| vivo | No |  |
| MediaTek |  | For UL, the BW is important parameter to close UL budget. Using 180 kHz instead of 300 kHz, gives 3 dB link gain. This is particularly helpful to support QoS for VoIP For VoIP, 180 kHz and 360 kHz UL could be considered For data, 180 kHz 360 kHz UL could be assumption as in TR 38.821 |
| Lockheed | Partially | 20 MHz should not be precluded. |
| Apple | Yes | Carrier bandwidth could be needed in LLS, which will limit the upper bound of the PRB number. In order to achieve the target data rate, different combination of MCS and PRB number will be applied. |
| Samsung | No | For coverage study, we think that carrier bandwidth is not necessary. Only considering both carrier frequency and transmission bandwidth is enough for evaluation. |
| Panasonic | No | Carrier bandwidth (system bandwidth) is not required to be agreed. |
| Xiaomi | No | The actual UL bandwidth for UL transmission should be used for link budget calculation. |
| LG | No |  |
| NTT DOCOMO | No | We think carrier bandwidth is of little importance to coverage evaluation. |
| ZTE | No |  |
| OPPO | No | Carrier bandwidth is not necessary for coverage evaluation. |
| Huawei, HiSilicon | Conditional Yes | If we can agree the bandwidth of BWP, we are fine not to align the carrier bandwidth. If the BWP bandwidth is not agreed, we think the carrier bandwidth is needed which is at least the upper bound of the frequency hopping. |
| Nokia, Nokia Shanghai Bell | No | As starting point we agree with FL assessment that there seem to be different definitions of the term “Bandwidth”. The only important aspect for the link budget analysis is that carrier bandwidth is sufficiently large to carry the configured bandwidth of the default bandwidth part (which contains CORESET#0). So, the relevant parameters for link budget analysis would be (a) subcarrier spacing (SCS), (b) PRB allocation for the considered channels, and (c) power spectral density – which may change for the UL. At least for UL, it should be considered to use configurations with just a few PRBs (e.g., 4) and 15 kHz / 30 kHz SCS for the evaluations. |
| Ericsson | No | Due to power limitation and (possibly) PFD limitation, coverage should be evaluated with different carrier bandwidths. |
| Ligado | No | Best to keep carrier bandwidth open at this stage. |
| Hughes/EchoStar | No | Best to keep carrier bandwidth open at this stage. |
| QC | No |  |

Q: Do you think BWP bandwidth should be agreed? If the answer is YES, please share the reason and the value.

| Company | YES/NO | Comment (e.g. reason of NO) |
| --- | --- | --- |
| vivo | No |  |
| Apple | No |  |
| Samsung | No | It is not necessary. It is better to up to companies’ choice. |
| Panasonic | Yes/No | If company see the need of the evaluation for FH and PDCCH, it needs to be agreed. If RAN1 can conclude no need of the evaluation for them, no need of the agreement. |
| LG |  | We are not sure it should be mandated or not. However, it seems good to define baseline bandwidth, in order to have same setup and limitation of evaluation for each channel. |
| ZTE | No |  |
| OPPO | Yes | BWP bandwidth is at least required for the evaluation of FH. |
| Thales | No |  |
| Huawei, HiSilicon | Yes | As we commented, we see a number of companies evaluate the frequency hopping. We think this should be aligned among companies. |
| Nokia, Nokia Shanghai Bell | Yes | Since the BWP bandwidth is setting the upper limit for the DL and UL channels in terms of resource allocations, it may be beneficial to define this. |
| MediaTek | No |  |
| Hughes/EchoStar | No | For evaluation purpose yes but actual channel bandwidth should be up to deployment |
| QC | No |  |

Q: For each channel bandwidth (i.e. number of PRBs), FL’s intention is that this aspect can be discussed/agreed in section 4.10 by using tables A.1-X of TR38.830 as baseline. Only if you think different work plan is better, please comment below.

| Company | YES/NO | Comment (e.g. reason of NO) |
| --- | --- | --- |
| Apple |  | The data channel bandwidth is depending on target data rate and the MCS/PRB combination. MCS and PRB number can be reported by each company. |
| Panasonic | Yes | For PUSCH, candidates would be 360kHz (as in 38.830) or 180kHz, according to the companies’ comments. Either is ok to us, but we prefer one value as baseline and the other value as optional. |
| Huawei, HiSilicon |  | This maybe relates with the data rate supported. For VoIP,360kHz seems enough in uplink. However, for low data rate, it is not clear on the requirement. We can leave to companies to report the combination of MCS, RBs repetitions for the optimal performance. |
| Nokia, Nokia Shanghai Bell | Yes | OK to define channel bandwidth of each channel (and tx power) later. |
| Ericsson |  | Agree with Apple |
| Hughes/EchoStar |  | Support Apple’s view |
| QC |  | Agree with Apple |

    - Summary of 2nd round

For carrier bandwidth, most companies believe that this parameter is unnecessary for LLS and link budget analysis, which is aligned with FL’s understanding.

For BWP bandwidth, more companies think it is unnecessary, but at the same time some companies still suggest to agree it since this parameter is necessary for upper limit of FH/PDCCH/resource allocation. Now FL thinks that the assessment to agree this parameter is understandable, but in consideration of majority’s view, it might be difficult to agree a specific value. Then reporting by companies if necessary would be maximum what we can do.

For channel bandwidth, it seems that it is OK to discuss it in section 4.10, including option of reporting by companies.

    - 3rd round

Proposal 4-6_v0

For evaluation of coverage performance in NR NTN,

- It is assumed that carrier bandwidth is sufficiently large to transmit each channel.

- Companies are encouraged to report BWP bandwidth, when necessary (e.g. for frequency hopping).

- Note: each channel bandwidth is discussed separately.

Q: Do you agree the above proposal? If the answer is NO, please share the reason and how to modify.

| Company | YES/NO | Comment |
| --- | --- | --- |
| vivo | Yes |  |
| Lockheed | Yes |  |
| OPPO | Yes |  |
| Hughes/EchoStar | Yes |  |
| MediaTek | Yes |  |
| Novamint | Yes |  |
| Apple | Yes |  |
| QC | Yes |  |
| Panasonic | Yes |  |
| Lenovo | Yes |  |
| Xiaomi | Yes |  |
| LG | Yes |  |
| Thales | Yes |  |
| ZTE | Yes |  |
| Nokia, Nokia Shanghai Bell | Yes |  |
| Samsung | Yes |  |
| Huawei, HiSilicon | Yes |  |
| Ericsson | Yes |  |

    - Summary of 3rd round

No concern is raised. The proposal can be agreed as it is.

Proposal 4-6_v0

For evaluation of coverage performance in NR NTN,

- It is assumed that carrier bandwidth is sufficiently large to transmit each channel.

- Companies are encouraged to report BWP bandwidth, when necessary (e.g. for frequency hopping).

- Note: each channel bandwidth is discussed separately.

GTW session outcome: Agreed

Then now we can close this section.

  - [Closed] Topic #7: Parameter set

In TR38.821, two satellite parameters sets are defined. The details can be found in Appendix-2 of this document. It seems that four companies [1/HW, HiSi] [8/CATT] [15/Apple] [20/THALES] propose to reuse Table 6.1.1.1-1/2 of TR38.821 as baseline. Meanwhile, at the same time, four companies [1/HW, HiSi] [2/ZTE] [15/Apple] [22/Ericsson] think that ITU regulatory limitations on power flux density should be considered. If considered, satellite EIRP density would be smaller than the values indicated in the tables of TR38.821. One company [15/Apple] assumes the modified values as 44 dBW/MHz, 20 dBW/MHz and 14 dBW/MHz in GEO, LEO-1200, LEO-600, respectively.

    - 1st round

Q: Do you think ITU regulatory limitations on power flux density should be considered, and why? If the answer is YES, please share satellite EIRP density that should be assumed.

| Company | YES/NO | Comment (e.g. reason of YES/NO, satellite EIRP density) |
| --- | --- | --- |
| QC |  | We need to understand when and where the limitations apply. |
| Lenovo |  | We think the ITU requirement on power flex density should be considered. However, more clarification and discussion on power flex density are necessary. |
| Apple | Yes | The ITU regulatory limitation on power flux density needs to be considered. Based on the power flux density, our calculation shows the satellite EIRP density are 44 dBW/MHz, 20 dBW/MHz and 14 dBW/MHz for GEO, LEO-1200 and LEO-600, respectively. |
| Xiaomi |  | We think RAN4’s input on co-existence needed to decide whether ITU requirement should be considered. |
| vivo |  | ITU requirement on power flex density should be further studied and clarified. In our understanding, maybe RAN1 cannot decide whether to consider ITU regulatory limitations on power flux density in 3GPP. |
| Samsung |  | Generally, we are fine to consider PFD requirements since it is in WID. However, we think that “free-space path loss” could be considered to compare the PFD requirement and current satellite EIPR density assumption. For example, it considers -113+32.45+20log10fC+20log10d instead of -113+10*log10(4πd2) since “free-space path loss” model had been used well in order to calculate link budget analysis in TR 38.821. |
| Nokia, Nokia Shanghai Bell | Yes | ITU regulatory limitations should be followed. |
| OPPO | Yes | ITU regulatory limitations on power flux density should be considered and more discussion is needed. |
| Huawei, HiSilicon |  | It is fine to take ITU regulatory limitations on power flux density into consideration. However, it is reasonable to clarify and understand where the limitations apply, as commented by QC. In our understanding, it only happens when NTN and TN share the same band. |
| LG |  | Agree with Lenovo. Further discussions can be necessary. |
| ZTE | Yes | Since ITU has given the limit of power flux density regulation, we should consider it in investigation. Otherwise, the system may not be available for commercial use. The PFD constraint affects EIRP of satellite. Therefore, it should be considered when evaluating DL performance. And before determining satellite EIRP value, we should first consider how to treat the constraint, e.g., avoid the bands with PFD constraint, or send LS to ITU, or investigate DL coverage enhancement to mitigate the performance gap. There is no need to discuss the details before determining how to handle the PFD limit. |
| CATT |  | Need to discuss when and where the limitations apply. |
| Ericsson | Yes | ITU regulations should be followed. Which levels that apply can be further discussed. |
| MediaTek |  | Generally fine, but it can be further discuss to align company understanding |
| Ligado | No | Some GEO configurations can deploy higher power in 1 MHz than claimed for ITU regulations. |
| Thales | NO | The appendix 5 section 1.2.3.1 within the ITU Radio Regulations Appendices, Edition of 2020 is on the determination of the need for coordination between MSS and RDSS space stations (space-to-Earth) and terrestrial stations. This is not a requirement as we do not have to meet these values in areas where we provide service. This is only to trigger coordination discussions with administrations which want to protect their terrestrial service (if they have). |
| Sony | Yes | ITU regulations should be followed. |
| Lockheed | No | This should be resolved with the ITU. |
| ETRI | Yes | ITU regulatory limitations should be considered |
| Hughes/EchoStar | No |  |

Q: For agreed satellite orbit/altitude and carrier frequency, do you agree to reuse set-1/2 satellite parameters as in Table 6.1.1.1-3 of TR38.821 except for satellite EIRP density? If NO, please share the reason.

| Company | YES/NO | Comment (e.g. which column should be modified) |
| --- | --- | --- |
| QC | Yes |  |
| Lenovo | Yes |  |
| Apple | Yes |  |
| Xiaomi | Yes |  |
| vivo | Yes |  |
| Nokia, Nokia Shanghai Bell | Yes |  |
| Huawei, HiSilicon | Yes |  |
| NTT DOCOMO | Yes |  |
| ZTE | No | The determination of other satellite parameters are related to the satellite EIRP density. Reusing other parameters with modified EIRP may not be straightforward. Moreover, Table 6.1.1.1-3 lists the UE characteristics for SLS. It seems that the moderator aimed to cite other tables. |
| CATT | Yes |  |
| Ericsson | Yes |  |
| Thales | Yes |  |
| Spreadtrum | Yes |  |
| MediaTek | No | It is not clear why EIRP density in Set-1 or Set-2 is not re-used. We have preference to prioritize Set-1 with parameters as in TR 38.821. |
| Ligado | No | Noise figure assumption should be changed from 7 to 4 dB. This is possible to achieve in handheld satphones today. |
| Sony | Yes |  |
| Lockheed | Yes |  |
| ETRI | Yes |  |
| Panasonic | Yes |  |

    - Summary of 1st round

From companies’ inputs, 11 companies think that ITU regulatory limitations on power flux density should be considered while 3 companies are negative. At least 6 companies state that further discussions necessary. QC/HW point out that when/where this regulation is applied is unclear. HW/Thales point out that this regulation is applied in case where NTN shares the same band with TN, and Thales thinks there is no need to consider this situation in this study. Although many companies support to consider the regulation, FL feels that comment from QC/HW/Thales is valid and clarification on this aspect is necessary. In addition, ZTE point out that how to consider the regulation should be discussed; e.g. adjust EIRP value / avoid the bands with PFD constraint / send LS to ITU. LS sending to RAN4 is suggested by Xiaomi, but as abovementioned, sending LS to somewhere is not fine since simulation results are necessary in the next meeting. To solve this issue, FL prepares two further questions below.

For parameter set, to reuse set-1/2 satellite parameters from TR38.821 seems agreeable among most companies. On MTK’s comment, the intention is that the value might be updated based on ITU regulatory limitations on power flux density as discussed above. If the regulation is not considered, FL assumes the same value can be reused also for EIRP density. On comments from ZTE/Ligado, FL wonders whether such an update is really necessary, since other companies are OK with the current proposal. Now GEO/LEO-1200/LEO-600 and S-band are proposed, so corresponding proposal is prepared below.

    - 2nd round

Q-a: Do you agree that ITU regulatory limitations on power flux density needs to be considered only when NTN/TN share the same band?

- Q-b: If the answer is YES, do you think the situation should be assumed in this study? Also please share the reason.

- Q-c: If the answer is NO, when/where the regulation is applied?

| Company | YES/NO for Q-a | YES/NO for Q-b | Comment (e.g. reason for Q-b, answer for Q-c) |
| --- | --- | --- | --- |
| vivo | No |  | TN/NTN bands overlapping could be avoided by proper cell planning though we agree that the NTN cell may be quite large compared to TN cell. Such assumption could be up to network planning. Not sure whether we need any regulation. |
| MediaTek | Yes | No | TN/NTN sharing same band can be de-prioritized in Rel-18 |
| Lockheed | Yes |  | This issue needs further study. |
| Apple | Yes | Yes | ITU regulation needs to be followed and non-NTN service should be protected in the shared band. We consider the NTN study should cover all deployment scenarios, not just the selected limited scenarios. Considering the large coverage of NTN beam/cell (e.g., up to 3500 km beam footprint size in GEO), it is infeasible to avoid TN/NTN band overlap via proper cell planning. Hence, the PFD constraints need to be considered in the evaluation assumptions. |
| Lenovo |  |  | We think we can take Q-b as an assumption for now as power requirement is always related to some kind of coexistence/sharing of frequency domain resource. On the other hand, we think further study/discussion/clarification on the requirement is necessary. |
| Samsung | Yes |  | Two scenarios may be considered: 1) NTN/TN have different bands, and 2) NTN/TN have same band. For 1) case, we can reuse current satellite EIPR density specified in set-1/2 satellite parameters from TR38.821. For 2) case, we should discuss whether or not it is possible to reuse current satellite EIPR density considering ITU regulation requirement, and if not possible, which value should be assumed as satellite EIRP density. |
| Xiaomi | Yes |  |  |
| LG | Yes | No | Agree with MediaTek. The case when NTN/TN have same band can be de-prioritized in Rel-18. |
| ZTE | Yes |  | We can consider to deprioritize case where NTN/TN sharing the same band. However, since ITU regulation should be followed, we should clarify it in an agreement or a note to clearly explain how to treat the ITU regulation about PFD. |
| OPPO | Yes | Yes | As long as ITU regulatory limitations are followed in NTN, it should be considered in the evaluation. |
| Thales | NO | NO | In addition to our 1st round comment on this issue we would like emphases that in APPENDIX 5 ((REV.WRC-19) in ITU Radio Regulations in NOTE 3 it is noted that the coordination thresholds in the band 2 160-2 170 MHz (Region 2) and 2 170-2 200 MHz (all Regions) to protect other terrestrial services do not apply to International Mobile Telecommunications (IMT) systems, as the satellite and the terrestrial components are not intended to operate in the same area or on common frequencies within these bands. (WRC-12) |
| ETRI | Yes |  | Agree with ZTE |
| Huawei, HiSilicon | Yes |  | As mentioned by Samsung, we can give separate assumptions for the two different cases. For the case of TN and NTN in different bands, we can take the assumption in TR38.821. For the case of TN and NTN in the same band, we can see whether the group can be aligned regarding the detailed assumption on EIRP etc. However, we feel that maybe some guidance from RAN4 or RAN plenary is needed regarding this ITU regulation. We would be fine to keep it in square brackets for the time being and ask RAN4 or RAN plenary for confirmation. |
| Nokia, Nokia Shanghai Bell | Yes | Yes | Same view as OPPO |
| Ericsson |  |  | We think an LS should be sent to RAN4 to ask for their view. |
| Omnispace | NO |  | Agree with Thales. |
| Ligado | NO |  | Agree with Thales |
| Hughes/EchoStar | No | No | Agree with Thales |
| QC |  |  | Based on Thales comments, further clarification on the applicability of ITU limit needs further clarified. |

Q: If ITU regulatory limitations on power flux density is considered, how does simulation evaluation consider it?

- Option 1: Smaller satellite EIRP density (If you did not propose the value in 1st round, please share it)

- Option 2: Others

| Company | Option | Comment |
| --- | --- | --- |
| vivo |  | We can wait until whether ITU PFD limitation should be considered in NTN is decided. For coverage performance, we can perform the simulation evaluation without considering the limitation first. |
| Lockheed |  | This requires further discussion. |
| Lenovo | Option 1 |  |
| LG |  | Further discussions can be necessary. |
| ZTE |  | We can first conclude how to treat ITU PFD limit. If the group decides to avoid the limit (by frequency management or whatever), the legacy set-1/set-2 can be reused. If not, how to adjust the satellite parameters can be further discussed. |
| OPPO | Option 1 |  |
| Apple | Option 1 |  |
| Thales |  | ITU regulatory limitations on power flux density should not be considered |
| ETRI |  | Agree with vivo |
| Nokia, Nokia Shanghai Bell | Option 1 | Further discussion may be needed on this topic. |
| Ericsson |  | We think an LS should be sent to RAN4 to ask for their view. |
| Omnispace |  | ITU regulatory limitations on EIRP should not be considered. This requires further analysis. |
| MediaTek |  | Further discussions needed |
| Ligado | No | Compliance with ITU regulations on power flux density should be resolved individually between the satellite owner and ITU, as suggested by Lockheed. Also, agree with vivo about the flexibility that exists in achieving TN/NTN coexistence through cell planning (including beam shaping) and FR greater than 1. |
| Hughes/EchoStar |  | ITU regulatory limitations on EIRP should not be considered. This requires further analysis |
| QC | Option 1 |  |

Proposal 7-1_v0

Reuse Set-1/2 satellite parameters as in table 6.1.1.1-3 of TR38.821 for GEO/LEO-1200/LEO-600 and S-band.

- FFS: whether to modify satellite EIRP density based on ITU regulatory limitations on power flux density

Q: Do you agree the above proposal? If the answer is NO, please share the reason and how to modify.

| Company | YES/NO | Comment (e.g. reason of NO) |
| --- | --- | --- |
| vivo | Yes |  |
| Lockheed | Yes |  |
| Lenovo | Yes |  |
| Panasonic | Yes |  |
| LG | Yes |  |
| NTT DOCOMO | Yes |  |
| ZTE | Yes | Moreover, Table 6.1.1.1-3 lists the UE characteristics for SLS instead of satellite parameters. |
| OPPO | Yes |  |
| Apple | No | We still think the modified satellite EIRP density based on ITU regulatory limitations on PFD should be used to replace the corresponding parameters in Set 1 and Set 2. However, as a compromise, we think at least the modified satellite EIRP density based on ITU regulatory limitations on PFD should be put together with Set 1 and 2 satellite parameters in the evaluation. |
| Thales | Yes | The FFS is not needed. Note that all coexistence studies done by RAN4 do not take into account this PFD limitation. |
| ETRI | Yes |  |
| Huawei, HiSilicon | Yes | For the same band usage of TN and NTN, we are open to discuss the satellite assumption. |
| Nokia, Nokia Shanghai Bell | Yes |  |
| Ericsson | Yes |  |
| MediaTek | Yes |  |
| Ligado | Yes | Agree with Vivo. |
| Hughes/EchoStar |  | FFS not neeeded |
| QC | Yes |  |

    - Summary of 2nd round

On ITU regulatory limitations on PFD, although a bit more companies think the answer of Q-a is YES, Thales points out that ITU regulation is not applied as satellite and the terrestrial communications are not intended to operate in the same area or on common frequencies within band 2160-2170 MHz or 2170-2200 MHz. Based on this text from ITU regulation, FL thinks that it is doubtful 3GPP needs to consider this ITU regulation. Besides, some companies suggest to ask this aspect RAN plenary and/or RAN4. Considering the current situation, what RAN1 can do would be this approach; therefore, FL recommends not to consider this ITU regulation in the next meeting, but at the same time to send an LS to RAN plenary and/or RAN4 whether this ITU regulation should be considered in coverage evaluation in RAN1.

For Set-1/2 satellite parameters, it seems that all companies are OK with the proposal. For Apple’s comment, the issue on EIRP density value is covered by FFS. If we agree to consider PFD limitation, then naturally we can discuss the parameter; thus proposal 7-1_v0 should be OK for Apple.

    - 3rd round

(No update)

Proposal 7-1_v0

Reuse Set-1/2 satellite parameters as in table 6.1.1.1-3 of TR38.821 for GEO/LEO-1200/LEO-600 and S-band.

- FFS: whether to modify satellite EIRP density based on ITU regulatory limitations on power flux density

Email discussion

Proposal 7-2_v0

As working assumption, RAN1 does not consider ITU regulatory limitations on power flux density for coverage evaluation in NR NTN.

- Send an LS to [RAN plenary or RAN4] to ask whether this regulation should be considered or not.

Q: Considering Thales’s comment in 2nd round, do you agree the above proposal? If the answer is NO, please share the reason.

| Company | YES/NO | Comment |
| --- | --- | --- |
| vivo | Yes | Support the proposal. |
| Lockheed | Yes |  |
| Hughes/EchoStar | Yes |  |
| MediaTek | Yes |  |
| Novamint | Yes |  |
| Apple | No | Regarding Thales’ comment in 2nd round, we have the following considerations: 1. For the S-band (2170-2200 MHz DL, 2160 -2170 MHz UL), though the protection of terrestrial services of IMT system is not considered, there are other terrestrial services other than IMT system to be protected. 2. Another 3GPP agreed NTN band is L band (1525 – 1559 MHz DL, 1626.5-1660.5 UL) also has PFD limitation by ITU regulation. The terrestrial services need to be protected. Given these considerations, we do not agree the ITU limitation on PFD should be ignored. From our side, we do hope the R18 NTN coverage enhancements to provide a more general framework/tool that will benefit broader (or even future) scenarios, rather than limited to one band. We are not claiming that PFD limits will be universally applied, but only applicable to some areas where related services coexist. However, as long as there are such cases/locations exist, we should not completely ignore the problem. It is much desirable that 3GPP can provide the corresponding enhancements/tools to handle these scenarios. It is true that this ITU EIRP limitation is NOT hard limitation (i.e. threshold doesn’t have to be exactly the same as listed in the documents), and potentially there could be relaxation depending on the negotiation with each individual service provider or operator, case by case. However, we can’t assume that ITU regulation can be completely ignored, and mostly likely it is only partially relaxed. Although an LS to RAN4 or RAN plenary is a way of justification, we think the timeline in RAN1 is tight. According to the work plan, the evaluation assumptions need to be made in this RAN1 meeting. Hence, we suggest taking the ITU limitation on PFD into account, and make the corresponding evaluation assumptions (e.g., adjusted EIRP density based on PFD limitation) in this meeting. Companies may decide whether this adjusted EIRP density is used in justifying their evaluation results. |
| Lockheed | No | After checking on ITU regulations on the PFD limits we agree with Apple that that this issue should be considered. We feel that this issue should be considered in the current evaluation of NTN because we cannot avoid it everywhere. We should target a unified design so that NTN can be widely deployed without restrictions. Further we feel that RAN1 should consider this issue for the evaluation assumption in the interest of expediting a resolution to this. |
| FL |  | Considering current situation, I think 7-1/7-2 are discussed together. Both are discussed via reflector. Please see there. No more input for here is unnecessary. |
| Nokia, Nokia Shanghai Bell | Yes | We should consider whether the LS should be directed to RAN plenary and ITU instead. |

Email discussion outcome:

Proposal 7-1_v6

Reuse Set-1/2 satellite parameters as in table 6.1.1.1-1/2 of TR38.821 for GEO/LEO-1200/LEO-600 and S-band.

- Send an LS to [RAN plenary or RAN4] to ask whether ITU regulatory limitations on power flux density should be considered or not. Confirmation from [RAN plenary or RAN4] is needed for deciding work in the next work item phase (if endorsed)

- Before the confirmation from [RAN plenary or RAN4], evaluations assuming ITU regulatory limitations on power flux density can be reported optionally in the study phase.

  - If reported, EIRP density is [49] dBW/MHz, [20] dBW/MHz and [14] dBW/MHz in GEO, LEO-1200, LEO-600, respectively. Ccompanies are encouraged to report which value of EIRP density is used and corresponding justification.

    - Summary of 3rd round

For PFD limitation, Thales believe as commented in 2nd round that ITU regulation is not applied as satellite and the terrestrial communications are not intended to operate in the same area or on common frequencies within band 2160-2170 MHz or 2170-2200 MHz. However, Apple believe that the ITU regulation should be considered to protect other terrestrial services other than IMT system in S-band, and to protect IMT system in L-band.

FL is not sure whether this Apple’s argument is valid or not since now only S-band is to be assumed in this study and whether to consider other terrestrial system is quite unclear.

In email discussion, a compromise was suggested, where RAN1 can discuss both case with PFD limitation and case without PFD limitation, and whether PFD should be considered or not is asked to RAN plenary or RAN4. However, still Apple/OPPO cannot live with it since they believe that WID states PFD limitation should be considered in evaluation, this perspective is not said as ‘optional’, and thus sending corresponding LS is not valid. Regarding EIRP density for the consideration, companies think which value should be used is unclear; it would be OK to report the value by companies.

WID text is,

The evaluation should also take into account any related regulatory requirements, e.g., ITU limitation of power flux density.

FL is not sure whether this text means the PFD limitation ‘shall’ be considered in simulation evaluation, or can be ignored based on evaluation by this discussion. Discussion in GTW session would be necessary.

One more note is, now MEO scenario is to be agreed. Reference parameters are summarized in RP-220590, which was noted in RAN plenary #95 meeting. It would be straightforward to add this aspect in proposal 7-1.

Proposal 7-1_v7

Reuse Set-1/2 satellite parameters as in table 6.1.1.1-1/2 of TR38.821 for GEO/LEO-1200/LEO-600 and S-band, and as in table 6.1.1.1-1/2 of RP-220590 for MEO and S-band.

- Option 1

  - Send an LS to [RAN plenary or RAN4] to ask whether ITU regulatory limitations on power flux density should be considered or not. Confirmation from [RAN plenary or RAN4] is needed for deciding work in the next work item phase (if endorsed)

  - Before the confirmation from [RAN plenary or RAN4], evaluations assuming ITU regulatory limitations on power flux density can be reported optionally in the study phase.

    - If reported, companies are encouraged to report which value of EIRP density is used and corresponding justification.

- Option 2

  - Evaluations assuming ITU regulatory limitations on power flux density should be reported in the study phase.

    - Companies are encouraged to report which value of EIRP density is used and corresponding justification.

GTW session outcome: Agreed

Then now we can close this section.

  - [Email] Topic #8: Study cases

Table 6.1.1.1-9 of TR38.821 presents a list of study cases, where each case is defined as a combination of satellite orbit / satellite parameter set / central beam elevation / terminal / frequency band / frequency reuse / polarization reuse. Similarly, it would be necessary to agree study cases for coverage evaluation of NR NTN.

For GEO, two companies [2/ZTE] [20/THALES] propose to use set 1 while three companies [1/HW, HiSi] [9/Xiaomi] [13/OPPO] propose to assume both set 1 and set 2. On elevation angle, one company [1/HW, HiSi] proposes 90/2.3 deg for set 1 and 90/11 deg for set 2 (Nadir/edge); two companies [13/OPPO (for set 1)] [20/THALES] propose 12.5 deg; one company [13/OPPO (for set 2)] proposes 20 deg.

For LEO-1200, one company [20/THALES] proposes to use set 2 while four companies [1/HW, HiSi] [9/Xiaomi] [6/Pana] [13/OPPO] propose to assume both set 1 and set 2. On elevation angle, one company [1/HW, HiSi] proposes 90/26.3 deg for set 1 and 90/22.2 deg for set 2 (Nadir/edge); three companies [6/Pana] [13/OPPO] [20/THALES] propose 30 deg.

For LEO-600, one company [20/THALES] proposes to use set 2 while three companies [1/HW, HiSi] [9/Xiaomi] [13/OPPO] propose to assume both set 1 and set 2. On elevation angle, one company [1/HW, HiSi] proposes 90/27 deg for set 1 and 90/23.8 deg for set 2 (Nadir/edge); two companies [13/OPPO] [20/THALES] propose 30 deg.

Besides, [2/ZTE] refers to elevation angle like more than 30 degree for all cases; [5/vivo] considers 10 to 90 degrees as elevation angle. For frequency reuse / polarization reuse, one company [6/Pana] proposes option 1 while one company [13/OPPO] proposes option 1 and option 2. [5/vivo] proposes to VoIP only for LEO scenario.

One important note would be that Table 6.1.1.1-9 of TR38.821 includes central beam elevation and the value is 45 deg for GEO and 90 deg for LEO-600/LEO-1200, but they would not correspond to a certain elevation angle from perspective of each UE that is evaluated in this study. Meanwhile, Table 6.1.3.2-1 of TR38.821 includes target elevation angle as 30 deg for LEO, 12.5 deg for GEO-Set 1, 20 deg for GEO-Set 2. It seems that several companies propose to reuse these values.

For satellite orbit / satellite parameter set / terminal / frequency band, there are related questions in previous sections. After fixed, corresponding question / proposal would be prepared. Therefore, FL would like to ask questions on elevation angle / frequency reuse / polarization reuse in the 1st round.

    - 1st round

Q: Do you think the following elevation angle as in Table 6.1.3.2-1 of TR38.821 can be reused? If the answer is NO, please share the reason and which value should be adopted instead.

- 30 deg for LEO, 12.5 deg for GEO-Set 1, 20 deg for GEO-Set 2

| Company | YES/NO | Comment (e.g. reason of YES/NO) |
| --- | --- | --- |
| Lenovo |  | We think a value range rather than a single value should be evaluated for each scenario. |
| vivo |  | We need further clarification on the elevation angle values in the question. Does it mean that 30 deg would be the minimum elevation angle for LEO to be studied, 12.5 deg and 20 deg are the minimum elevation angles for GEO-Set 1 and GEO-Set 2, respectively? Will the final target elevation angles be determined when the link budget and the gap required to meet the target SNR is clear? It would also be good to determine the elevation angles that are supposed to support the service being studied according to the gap required after we get link budget for the elevation angles being studied. Note that if we still can not make the target service work for some elevation angels with all techniques that we can use, then we have to increase the target elevation angle in the end. |
| Samsung |  | Although this value is specified in TR 38.821, we slightly prefer to consider more general evaluation angles (e.g., only specifying minimum elevation angle) with MPL approach as explained Topic#2. |
| Nokia, Nokia Shanghai Bell | Yes |  |
| OPPO | Yes |  |
| Huawei, HiSilicon | No | The elevation angles are the center of the edge beam, which are not the worst cases with a satellite’s coverage. It is more reasonable to take the edge of the edge beam as the elevation angle. |
| NTT DOCOMO | Yes |  |
| ZTE |  | The elevation angles should be selected according to application and commercial deployment. It is preferred to evaluate a range of elevation angles instead of a single one. For LEO, 30 deg can be considered as the minimum elevation angle for evaluation. For GEO, the values should be further checked since the performance may not satisfy requirements under such low elevation angles. |
| CATT | Yes |  |
| Ericsson | Yes |  |
| Thales | Yes |  |
| MediaTek |  | TR 38.821 assumption for elevation angles can be re-used as baseline for lowest elevation angle. A range of (higher) elevation angles can be considered in the evaluation. |
| Ligado |  | Agree with MediaTek to consider a range of higher elevation angles. |
| InterDigital | Yes |  |
| Sony |  | The study only needs to consider the worst-case elevation angle. Can’t we assume that the coverage would be better for non-worst case elevation angles? |
| Lockheed | Yes |  |
| Panasonic | Yes | We should reuse the value defined in Table 6.1.3.2-1 (for link budget) of TR38.821 as much as possible. |
| Huhes/EchoStar | Yes | Please consider MEO Set 1 in RP-220590 |
| QC |  | The objective is to support elevation angle as much lower as possible as long as it is reasonable. |

Q: Which option of frequency/polarization reuse should be assumed in this study? Also please share the reason.

| Company | Option | Comment (e.g. why the option is selected) |
| --- | --- | --- |
| QC | Frequency reuse 3 |  |
| Lenovo | Frequency reuse 1 |  |
| Xiaomi | Frequency reuse 2/3 | Don’t consider interference in frequency reuse 2/3. |
| vivo | Frequency reuse 1/3 | If there would no SLS, frequency reuse factor would be not the key point and Frequency reuse 1 and reuse 3 could be selected. |
| Nokia, Nokia Shanghai Bell |  | The frequency and/or polarization reuse is a bit coupled to the entire cell layout. For simplicity we prefer to not introduce any special boosting mechanisms to make conditions artificially better for the link. Hence we would propose to use frequency reuse 1 and polarization reuse 1. |
| ZTE |  | We are not clear about the motivation of selecting frequency/polarization reuse option. If the frequency reuse is to avoid interference, FRF=3 should be assumed. |
| CATT | Frequency reuse 3 | In realistic scenarios, inter-cell or inter-satellite interference should be avoid. |
| Ericsson | Frequency reuse 1 | It should first be agreed if interference is to be taken into account in the link budget calculations. Otherwise, reuse does not matter. |
| Thales | FR 3 |  |
| MediaTek | FR3 | Assuming frequency reuse, motivation is to avoid interference. |
| Ligado | FR3 |  |
| Sony | FR3 | To avoid the interference. |
| Panasonic | Frequency reuse 3 | Although frequency reuse 1 should be the baseline operation in usual case, we are ok to assume frequency reuse 3 and use CNR (instead of CINR) for the link budget evaluation (in Proposal 2-1_v0) in order to focus noise limited coverage evaluation. |
| Hughes/EchoStar | FR3 |  |

    - Summary of 1st round

For elevation angle, 9 companies are OK to reuse values from Table 6.1.3.2-1 of TR38.821. Meanwhile, 6 companies think that more (higher) elevation angles should be considered as well. FL’s understanding was that for coverage evaluation, it is OK to perform simulation only for the worst case and thus there is no need to consider higher elevation angles also in order to reduce simulation efforts and subsequent discussions. However this understanding might be incorrect, so FL would like to ask corresponding question below. In addition, HW points out that the values in table 6.1.3.2-1 are the center of the edge beam and thus elevation angle at the edge of the edge beam should be used. FL would like to see companies’ views on this perspective.

Regarding frequency/polarization reuse, most companies are OK to apply frequency reuse factor = 3 (and thus polarization reuse factor = 1).

    - 2nd round

Q: Do you think elevation angles other than the worst case should be evaluated?

| Company | YES/NO | Comment |
| --- | --- | --- |
| vivo | Yes | Actually, we have not decide the target data rate yet and not clear whether the minimum elevation angle defined in TR38.821, would correspond to the worst case. Once the target data rate is determined, we should further check whether the worst elevation angle is high enough to achieve the target data rate. If the target data rate still cannot be achieved with the worst elevation angle even if all coverage enhancements are applied, we can evaluate other higher elevation angles (compared to those defined in 38.821) to find a target minimum elevation angle to support the target data rate. Thus, the evaluation on other elevation angles should not be precluded. |
| MediaTek | Yes | In case of VoIP, some data services there may be some trade off to support these services with adequate QoS and coverage. A more mature satellite constellation may allow larger edge elevation angles with higher QoS supported. |
| Lockheed | Yes |  |
| Apple | Yes | We are fine with examining the performance at various elevation angles. This could help to understand the coverage holes as well as make the design target in a later stage. |
| Lenovo | Yes | We think coverage enhancement may be also necessary for higher elevation angles. Meanwhile, based on the simulation, we can also identify a suitable range of elevation angles for actual operation. |
| Samsung | [No] | We are open to consider other values than the worst case. But, in order to align simulation setting between different companies, baseline should be necessary. We are not sure how many elevation angle could be considered in this simulation. |
| Panasonic | No | Elevation angle defined in 38.821 would be sufficient. Other higher elevation angle can be evaluated as optional. |
| LG | Yes | We are ok with evaluating various elevation angels, but in order to reduce simulation burden, the number of angles should be minimized. |
| ZTE | Yes | Without evaluation on various elevation angles, we can not figure out the range that can be supported by the system. It is possible that the target can be satisfied under high elevation angle but not satisfied under low elevation angle. |
| OPPO | No | We agree with Samsung and Panasonic that the elevation angles of the worst case should be adopted as baseline for calibration, and other elevation angles should be optional. |
| Thales | No | But we are open to consider other elevation angles than the worst case. |
| Huawei, HiSilicon | Yes | According to the discussion here and comments from companies, we think maybe we can investigate more elevation angles, e.g. 10 deg, 20 deg, ...., 90 deg for evaluaton. |
| Nokia, Nokia Shanghai Bell | No | For coverage enhancement evaluations, the worst case (but realistic) in channel conditions should be assumed. |
| Ericsson | No | Worst case elevation angle is sufficient. |
| MediaTek | Yes | More elevation angles can be considered to determine range that can be supported with different target data rates and QoS |
| NEC |  | We have similar view as Vivo |
| Ligado | Yes | It is necessary to characterize both the worst case and median performances. |
| Hughes/EchoStar | No | Open to consider other elevation angles than the worst case. |
| QC | Yes |  |

Q-a: Do you think the values of elevation angle in Table 6.1.3.2-1 of TR38.821 is the center of the edge beam?

- Q-b: If the answer is YES, which option is preferred?

  - Option 1: Agree certain values of elevation angle at the edge of the edge beam (please share the value as well)

  - Option 2: Agree elevation angle at the center of the edge beam and calculate elevation angle at the edge of the edge beam by each simulation

| Company | YES/NO for Q-a | Option for Q-b | Comment |
| --- | --- | --- | --- |
| vivo | Yes | Option 1 | In our understanding, a feasible target minimum elevation angle for the worst case could cover this elevation angle at the edge of the edge beam. |
| Lenovo |  | Option b |  |
| Panasonic | No |  | In our understanding, the “Target elevation angle” in Table 6.1.3.2-1 is elevation angle of the target UE for link budget calculation. The propagation loss can be calculated based on this value. If we use different value as the worst case, the following values in TR38.821 Table 4.4-2 can be used. GEO; LEO. Min Elevation angle for both sat-gateway and user equipment; 10° for service link and 10° for feeder link; 10° for service link and 10° for feeder link. Max distance between satellite and user equipment at min elevation angle; 40,581 km; 1,932 km (600 km altitude) 3,131 km (1,200 km altitude). |
| OPPO | No |  | We support to use the elevation angles defined in 38.821 for link budget calculation, and smaller elevation angles are not necessary. |
| Huawei, HiSilicon |  |  | We think 10 deg, 20 deg, ...., 90 deg is acceptable for us. |
| Nokia, Nokia Shanghai Bell | No |  | Similar view as Panasonic |
| Ericsson | Don't know | Option 1 |  |
| MediaTek |  |  | To our understanding, it is the beam edge elevation. If multiple beams per satellite, it is the edge beam edge elevation, so it is the minimum elevation. |
| Hughes/EchoStar |  |  | Too early to determine |
| QC |  |  | Not sure if it is important. |

Proposal 8-1_v0

For NR NTN coverage enhancement, assume frequency reuse factor = 3 and polarization reuse factor = 1.

The above proposal is discussed with proposal 2-1.

    - Summary of 2nd round

For elevation angle, more companies prefer to evaluate elevation angles other than the worst case. One rational is that elevation angle should be changed according to the target code rate and QoS. However, FL is not sure whether this discussion direction is valid, since firstly desirable coverage for each scenario of GEO/LEO is decided, and then whether service with target code rate that is discussed above can be provided for the area or not is discussed. Otherwise, some companies evaluate e.g. 10 deg and say ‘not sufficient’, but other companies evaluate e.g. 45 deg and say ‘sufficient’. This means that the discussion will become not whether coverage is sufficient or not, but rather which elevation angle should be assumed in real deployment. FL is not sure companies would like to go with this kind of discussions in this study.

FL can find recommendation to evaluate elevation angle in table 6.1.3.2-1 mainly and other elevation angles as optional. This would be good compromise, so the corresponding proposal is prepared.

In addition, FL thinks that now is good timing to discuss details of study cases. FL prepares corresponding question below, based on existing proposals/agreement.

    - 3rd round

Proposal 8-2_v0

The following elevation angle is evaluated.

- 30 deg for LEO, 12.5 deg for GEO-Set 1, 20 deg for GEO-Set 2, as in in Table 6.1.3.2-1 of TR38.821

- Other elevation angles can be evaluated as optional

Q: Do you agree the above proposal?

- Q’: If the answer is NO, do you think which elevation angle should be covered to meet target data rate should be discussed in this study?

| Company | YES/NO for Q | YES/NO for Q’ | Comment |
| --- | --- | --- | --- |
| vivo | Yes |  | We are OK with the proposal. |
| Lockheed | Yes |  |  |
| OPPO | Yes |  |  |
| Hughes/EchoStar | Partial | Partial | Propose to add MEO scenario with 30 deg elevation angle. |
| MediaTek |  |  | Values are fine, but should also include MEO as proposed by EchoStar. |
| Novamint |  |  | Agre plus support Echostar proposal to include MEO scenario with 30 deg elevation angle |
| Lockheed | Partial | Partial | We are ok with the proposal but would also like to see MEO included. |
| Apple | Yes |  |  |
| ESA |  |  | We are fine with MEO scenario and 30 deg elevation angle |
| Panasonic | Yes |  |  |
| Lenovo | Yes |  |  |
| Xiaomi | Yes |  |  |
| LG | Yes |  |  |
| GateHouse | Partial |  | Agree but MEO should also be included. |
| Thales | [Yes] |  | We would like to see MEO included as proposed by Hughes/EchoStar. |
| ZTE | Yes |  |  |
| Nokia, Nokia Shanghai Bell | Yes |  |  |
| Samsung | Yes |  |  |
| Huawei, HiSilicon | Yes |  | Yes, considering the assessment of the situation by moderator |
| Ericsson | Partial |  | For LLS, the NTN-CDL and NTN-TDL channel models are defined in TR 38.811 for elevation angles [10,20,30,…,90] degrees only. We suggest to change the elevation angle for GEO-Set 1 from 12.5 degrees to 10 (or 20) degrees to align assumptions between link budget calculation and LLS. Elevation angle for MEO is needed. |

Table: List of study cases

| Case | Satellite orbit | Satellite parameter set | Elevation angle (deg) | Terminal | Frequency band | Service type |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | GEO | 1 | [12.5] | Handset | S-band | Low-data rate service |
| 2 | GEO | 2 | [20] | Handset | S-band | Low-data rate service |
| 3 | LEO-1200 | 1 | [30] | Handset | S-band | VoIP |
| 4 | LEO-1200 | 1 | [30] | Handset | S-band | Low-data rate service |
| 5 | LEO-1200 | 2 | [30] | Handset | S-band | VoIP |
| 6 | LEO-1200 | 2 | [30] | Handset | S-band | Low-data rate service |
| 7 | LEO-600 | 1 | [30] | Handset | S-band | VoIP |
| 8 | LEO-600 | 1 | [30] | Handset | S-band | Low-data rate service |
| 9 | LEO-600 | 2 | [30] | Handset | S-band | VoIP |
| 10 | LEO-600 | 2 | [30] | Handset | S-band | Low-data rate service |

Q: Which case from the above table should be evaluated? If you think modification of this table is necessary (e.g. adding new column), also please share it.

| Company | Case | Comment |
| --- | --- | --- |
| vivo | Mandatory: 1/2/5/9; Optional: 3/7 | Considering low data rate service in GEO scenario is enough and more important. VoIP in LEO case is enough. So 4/6/8/10 can be removed. Considering the evaluation effort reduction, Case 3/7 could be optional. Because the main difference between Set-1 and Set-2 is EIRP. That is, a fixed performance gap could be considered due to EIRP, i.e. 6dB for LEO. |
| OPPO | Case 1-10 |  |
| Hughes/EchoStar |  | Add two cases for MEO for Set-1 and Set-2 described in RP-220590 using the same elevation angel as LEO-1200. |
| Lockheed |  | Agree with Hughes/Echostar. |
| Apple |  | The satellite parameter set depends on the discussions on Topic #7. |
| Panasonic | 1,5,9 | Either set 1 or set 2 for each orbit would be sufficient. Minimum set would be case 1,5,9 in the above table. Case 3, 7 can be evaluated as optional. |
| Lenovo | 1 2 5 9 |  |
| Thales |  | Agree with Hughes/Echostar. Handset = Smartphone |
| ZTE | Case 1-10 |  |
| Nokia, Nokia Shanghai Bell | 1,2,3,7,9 | To avoid too big a load on evaluations, we should consider down-selecting the table to a set of representative configurations. This would be achieved through the proposed set. |
| Huawei, HiSilicon | 3, 7 and 1, 4, 8 (if we could align on the low data rate) | Agree with Panasonic. We think only Set 1 is enough. |
| Ericsson | 1-10 | Cover all cases at least for link budget calculations. Possibly down-select for LLS. Add two optional cases for MEO (with Set-1 and Set-2) |

    - Summary of 3rd round

On elevation angle, most companies are OK with the proposal. As commented by several companies, now MEO is to be agreed; thereby elevation angle for MEO should be added. As the value, 30 deg is suggested.

Ericsson shares that for NTN-CDL/NTN-TDL channel models are defined in TR 38.811 for elevation angles [10,20,30,…,90] degrees only. Although FL finds that elevation angle in some tables is only 10*n deg (e.g. table 6.6.1-1 LOS probability), at the same time other values can also be found in the TR. In addition, table 6.1.3.2-1 in TR38.821 uses 12.5 deg already. FL believes that this value can be used also for this study.

For study cases, companies’ preferences can be summarized as below:

- Case 1: vivo, OPPO, Pana, Lenovo, ZTE, Nokia, HW, Ericsson (8)

- Case 2: vivo, OPPO, Lenovo, ZTE, Nokia, Ericsson (6)

- Case 3: vivo (optional), OPPO, Pana (Optional), ZTE, Nokia, HW, Ericsson (7)

- Case 4: OPPO, ZTE, HW, Ericsson (4)

- Case 5: vivo, OPPO, Pana, Lenovo, ZTE, Ericsson (6)

- Case 6: OPPO, ZTE, Ericsson (3)

- Case 7: vivo (optional), OPPO, Pana (Optional), ZTE, Nokia, HW, Ericsson (7)

- Case 8: OPPO, ZTE, HW, Ericsson (4)

- Case 9: vivo, OPPO, Pana, Lenovo, ZTE, Nokia, Ericsson (7)

- Case 10: OPPO, ZTE, Ericsson (3)

Although some companies prefer to consider all cases, multiple companies suggest to do down-selection to reduce simulation burden. FL thinks that less cases would be better from the perspective and reducing discussion burden in the next meeting. FL suggests to set cases with less supporters as optional and drop cases with minimum supporters.

One note is that additional cases for MEO need to be added.

Proposal 8-2_v1

The following elevation angle is evaluated.

- 30 deg for LEO, 12.5 deg for GEO-Set 1, 20 deg for GEO-Set 2, as in in Table 6.1.3.2-1 of TR38.821

- 30 deg for MEO

- Other elevation angles can be evaluated as optional

Proposal 8-3_v0

For NR NTN coverage enhancement, evaluate the following cases.

| Case | Satellite orbit | Satellite parameter set | Elevation angle (deg) | Terminal | Frequency band | Service type |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | GEO | 1 | [12.5] | Handset | S-band | Low-data rate service |
| 2 | GEO | 2 | [20] | Handset | S-band | Low-data rate service |
| 3 | LEO-1200 | 1 | [30] | Handset | S-band | VoIP |
| 4 (Optional) | LEO-1200 | 1 | [30] | Handset | S-band | Low-data rate service |
| 5 | LEO-1200 | 2 | [30] | Handset | S-band | VoIP |
| 6 | LEO-1200 | 2 | [30] | Handset | S-band | Low-data rate service |
| 7 | LEO-600 | 1 | [30] | Handset | S-band | VoIP |
| 8 (Optional) | LEO-600 | 1 | [30] | Handset | S-band | Low-data rate service |
| 9 | LEO-600 | 2 | [30] | Handset | S-band | VoIP |
| 10 | LEO-600 | 2 | [30] | Handset | S-band | Low-data rate service |
| 11 (Optional) | MEO | 1 | [30] | Handset | S-band | Low-data rate service |
| 12 (Optional) | MEO | 2 | [30] | Handset | S-band | Low-data rate service |

    - 4th round

FL would like to suggest to have discussion for the above proposals via reflector.

  - [Email] Topic #9: Evaluation target channel/signal

Section 4.2 of TR38.830 includes a list of evaluated channels/signals. [17/DCM] proposes to reuse this list with exception of TDD; [11/Samsung] proposes to evaluate PUSCH; [19/LGE] proposes to evaluate PUSCH/PRACH; [15/Apple] proposes PDCCH, PDSCH, Msg 2 PDSCH, Msg 3 PUSCH, Msg 4 PDSCH and Msg 4 HARQ-ACK for evaluation. FL would like to ask which channel/signal should be evaluated in this study.

    - 1st round

Q: Which channel/signal should be evaluated in this study? Also please share the reason. Note that FDD will be assumed for NR NTN according to TR38.821.

- A: PUSCH for eMBB

- B: PUSCH for VoIP

- C: PUCCH Format 1 with 2bits

- D: PUCCH Format 3 with 11bits

- D E: PUCCH Format 3 with 22bits

- F: SSB

- G: PRACH format 0

- H: PRACH format B4

- I: Broadcast PDCCH (PDCCH of Msg.2)

- J: PDSCH for Msg.2

- K: PUSCH of Msg.3

- L: PDSCH of Msg.4

- M: Unicast PDCCH

- N: PDSCH for eMBB

- O: PDSCH for VoIP

- P: PUCCH with HARQ-ACK for Msg.4

- Q: PUSCH with SIP invite

- R: PUSCH for CSI 11bit

- S: PUSCH for CSI 22bit

- T: Others

| Company | Channel/signal (e.g. A/B/K) | Comment (e.g. reason of the selection) |
| --- | --- | --- |
| QC |  | We don’t see the need to evaluate DL channels unless ITU limitations apply. PRACH format 2 and Msg3 should be evaluated. |
| Lenovo |  | We think we should firstly focus on PUSCH, PUCCH and PRACH. Evaluation for DL channel can be considered further after clarification/discussion of power flex density of ITU requirement. |
| Apple |  | With ITU limitation on PFD, we think downlink channels need to be examined. Also, we think RACH procedure signals need to be examined. We are fine with all the items, except that R/S/Q can be optional or remove. |
| vivo |  | We can focus on UL channels in RRC connected state first. For msg3, the TB size is small, it can already support retransmission/repetition without data rate requirement, so we do not see the need to study such UL channels. For downlink channels, it depends on the discussions on PFD and could be discussed later stage if needed. |
| Samsung | A/B | It is noted that we only have 0.25 TU per meeting, and 2 meetings. Also, noted that Rel-17 coverage enhancement study had considered A~S scenarios with 2 TUs. However, only few simulation results are submitted in most of scenarios. That’s why representative conclusion/observations were not made. Since Rel-18 NTN is focusing on VoIP and low-data service in connected mode, it is reasonable to consider data rate such as PUSCH/PDSCH. Then, PUSCH is the most challenging link rather than PDSCH. Focusing on PUSCH is reasonable considering very limited TU. |
| Nokia, Nokia Shanghai Bell | A, B, C, D, F, G, I, K, P, R | Main focus should be on the initial access and UL channels in general. No need to investigate short PRACH formats since the coverage enhancement solution to this specific problem is simple: Use a longer PRACH format with better coverage. |
| OPPO | A-P | For the item I, we suggest to evaluate PRACH format2, which has better coverage performance. |
| Huawei, HiSilicon |  | At least PUSCH, PRACH, PUCCH should be prioritized. The downlink channel can be considered when the group aligns on the PFD limitation assumption. |
| LG |  | We think it is desirable to focus on UL channel (i.e., PUSCH, PUCCH, and PRACH) first. |
| NEC | G/H/K | We consider initial access to be the main bottleneck in supporting connectivity of commercial UEs because for other UL and DL channels enhanced coverage solutions can be deployed dedicatedly based on UE capability exchange after initial access. Specifically, we would like to consider coexistence of commercial UEs and VSAT UEs in the same NTN cell. |
| NTT DOCOMO |  | UL channels should be prioritized. Longer RACH sequences would be better for coverage. Also, we think the main use case in NTN would be VoIP. So, especially B, C, D, E, G, K and P should be prioritized. |
| ZTE |  | We think the items B, F, G, and K should be prioritized. For DL, if PFD should be considered in evaluation, SSB(F) should be considered first. For UL, we think PUSCH(B), long PRACH preamble(G), and PUSCH in Msg3(K) should be considered with higher priority. Moreover, we are not clear about item A (PUSCH for eMBB). Firstly, A and B are both PUSCH but with different configuration. There is no need to differentiate them. Moreover, we are considering voice and low data rate services instead of eMBB in this study. |
| CATT |  | UL channels should be prioritized. |
| Ericsson |  | Difficult to limit what channels/signals to include before parameters and assumptions are agreed. R and S may be excluded. |
| Thales |  | Priority 1: Uplink Channels: PUSCH Msg3, PUSCH (A, Q), PUSCH VoNR (B), PUCCH (C, D, P) , PRACH long format 2 (H), Priority 2: Downlink Channels: PDCCH (M, I) PDSCH MBB (N, J) PDSCH VoNR (O) |
| Spreadtrum |  | Focus on initial access and UL channels |
| MediaTek | A, B | UL channels with PUSCH (A/B) can be the priority for the study. For PRACH, there may be overlap with Rel-18 (cellular) NR Coverage Enhancement WI. Duplication of discussions on PRACH coverage enhancements should be avoided. |
| InterDigital | A/B | Agree with Samsung. Given TU allocated, it is better to focus on most challenging link to support the target use case. |
| Sony |  | It is too early to preclude DL channels from the study. It will become clearer whether DL channels are a coverage bottleneck once a preliminary link budget is determined. |
| Lockheed | B, G, J, K, L, N, P |  |
| Panasonic | A/B/G/K | PUSCH for VoIP, eMBB, msg.3 and PRACH format 0 would be higher priority as bottleneck channels. |

    - Summary of 1st round

Summary of inputs is the following:

- Prioritize UL channels or support only UL channels: QC, Lenovo, vivo, Samsung, Nokia, HW, LGE, DCM, CATT, Spreadtrum, MTK, IDC, Pana (13)

- Both UL channels and DL channels: Apple, OPPO, Sony, Thales, Lockheed (5)

- A: PUSCH for eMBB: Lenovo, Apple, Samsung, Nokia, OPPO, HW, LGE, Thales, MTK, IDC, Pana (11)

- B: PUSCH for VoIP: Lenovo, Apple, Samsung, Nokia, OPPO, HW, LGE, DCM, ZTE, Thales, MTK, IDC, Lockheed, Pana (14)

- C: PUCCH Format 1 with 2bits: Apple, Nokia, OPPO, HW, LGE, DCM, Thales (7)

- D: PUCCH Format 3 with 11bits: Apple, Nokia, OPPO, HW, LGE, DCM, Thales (7)

- E: PUCCH Format 3 with 22bits: Apple, OPPO, HW, LGE, DCM (5)

- F: SSB: Apple, Nokia, OPPO, ZTE (4)

- G: PRACH format 0: Lenovo, Apple, Nokia, OPPO, HW, LGE, NEC, DCM, ZTE, Lockheed, Pana (11)

- H: PRACH format B4: Lenovo, Apple, OPPO, HW, LGE, NEC, Thales (7)

- I: Broadcast PDCCH (PDCCH of Msg.2): Apple, Nokia, OPPO, Thales (4)

- J: PDSCH for Msg.2: Apple, OPPO, Thales, Lockheed (4)

- K: PUSCH of Msg.3: QC, Apple, Nokia, OPPO, NEC, DCM, ZTE, Thales, Lockheed, Pana (10)

- L: PDSCH of Msg.4: Apple, OPPO, Lockheed (3)

- M: Unicast PDCCH: Apple, OPPO, Thales (3)

- N: PDSCH for eMBB: Apple, OPPO, Thales, Lockheed (4)

- O: PDSCH for VoIP: Apple, OPPO, Thales (3)

- P: PUCCH with HARQ-ACK for Msg.4: Apple, Nokia, OPPO, DCM, Thales, Lockheed (6)

- Q: PUSCH with SIP invite: Thales (1)

- R: PUSCH for CSI 11bit: Nokia (1)

- S: PUSCH for CSI 22bit: (0)

- T: Others

  - PRACH format 2: QC, OPPO (2)

Firstly it would be better to emphasize that ‘eMBB’ here means low-data rate service. This terminology is used in TR38.830, but seems to lead misunderstanding. Let us use ‘low-data rate service’ instead of ‘eMBB’ in this document.

There are two comments as time for this study is very limited and thus RAN1 should focus on small number of channels. FL agrees with this comment. We have only one meeting to discuss simulation results in this study, so a lot of target channels cannot be evaluated/discussed sufficiently.

On DL channels, it seems that whether DL channel should be evaluated or not is dependent on outcome of discussion in section 4.3; FL recommends to wait for progress of the discussion since which UL channels can be evaluated is also dependent on whether DL channel is evaluated or not.

    - 2nd round

Skipped to wait for progress of section 4.3.

    - 3rd round

Skipped to wait for progress of section 4.3.

    - 4th round

Which channels/signals are evaluated in this topic is one important topic that should be agreed in this meeting. Although we do not conclude issue on PFD limitation issue now (hopefully solved in 2nd GTW), FL would like to trigger this discussion in 4th round.

From companies’ inputs in 1st round, FL recommends to evaluate the following channels/signals. Regarding DL channels/signals, they might be dependent on outcome of discussion for PFD limitation issue. Brackets are added in this stage and to be updated later based on the discussion. we agreed that PFD limitation can be considered. This would imply that DL channel/signal would be target channel for evaluation.

Proposal 9-1_v0

For NR NTN coverage enhancement, the following channels/signals are evaluated.

- PUSCH for VoIP

- PUSCH for low data rate service

- PUCCH format 1 with 2 bits

- PUCCH format 3 with 11 bits

- PRACH format 0

- PRACH format B4

- PUSCH Msg.3

- SSB

- PDSCH for VoIP

- PDSCH for low data rate service

Q: Do you agree the above proposal? If the answer is NO, please share the reason and what should be added/modified/removed.

| Company | YES/NO | Comment |
| --- | --- | --- |
| Panasonic | Yes |  |
| Nokia, Nokia Shanghai Bell | No | We would suggest to remove PUSCH Msg.3 from the list. This was already considered for Rel-17 coverage enhancements so we do not need to further evaluate it here. The PUSCH performance would anyway be evaluated for the two other cases. We may also consider to remove PUCCH format 3 with 11 bits since this may not be representative for the coverage enhancements in the S-band in FR1, where MIMO operation is less relevant. We may also consider dropping evaluations for PRACH format B4 since it is having substantially shorter CP (and is hence more susceptible to multipath and timing inaccuracies – where the latter will mainly be visible for poor modeling of Common TA through the Common TA parameters). |
| ZTE | Basically fine but | We think some of the evaluations can be removed or optional to reduce workload since they may be less important. On PRACH evaluation, long PRACH format can be focused so that PRACH format B4 can be removed or optional. Moreover, compared with PUCCH, PUSCH is the bottleneck. We may focus on evaluation of PUSCH and remove PUCCH related bullets or set them as optional. |
| Thales | [YES] | Add PUSCH for SIP Invite: We think that PUSCH with SIP signaling message should be considered. PUSCH SIP Invite might be different from PUSCH for VoIP. We need to further discuss what data rates should be used considering the SIP invite is needed. |
| Apple | No | When PFD limitation is considered, we think at least PDCCH (both unicast and broadcast) needs to be evaluated. Additionally, we think RACH related channels need to be evaluated, including PUCCH with HARQ-ACK for Msg.4, PUSCH for Msg3, PDSCH for Msg.2 and Msg.4. |
| Ericsson | Yes |  |
| Huawei, HiSilicon | Basically yes. | If we can not agree on low data rate, it can be optional. |

    - Summary of 4th round

OK: Pana/Ericsson/HW

NO/Update: Nokia/ZTE/Thales/Apple

- Nokia: Remove Msg.3/PF3/PRACH format B4

  - Msg.3: Rel-17 CovEnh is enough

  - PF3: MIMO is less relevant

  - PRACH format B4: poor performance from multipath and timing inaccuracies

- ZTE: Remove PRACH format B4/PUCCH or set to optional

  - PRACH format B4: focus on longer format

  - PUCCH: focus on PUSCH

- Thales: Add PUSCH for SIP invite

- Apple: Add PDCCH/Msg2/Msg3/Msg4/PUCCH for Msg4

  - PDCCH: to consider PFD limitation

Firstly, FL thinks that more channels/signals cannot be evaluated since we have only one meeting for corresponding discussion. If added, it would be true that we cannot reach conclusion in the next meeting. PUSCH SIP invite is not added since SIP invite message is not agreed as we discussed for VoIP data rate in GTW. PDCCH/Msg2/Msg4/PUCCH for Msg4 have less supporters as summarized in 1st round. Addition of them would be impossible.

To reduce workload, FL recommends to set PUCCH and PRACH format B4 as optional, in consideration of the above input and the summary of 1st round. On Msg3, FL thinks there are more supporters as summarized in 1st round. Probably keeping it would be better.

    - 5th round

Proposal 9-1_v1

For NR NTN coverage enhancement, the following channels/signals are evaluated.

- PUSCH for VoIP

- PUSCH for low data rate service

- PUCCH format 1 with 2 bits (optional)

- PUCCH format 3 with 11 bits (optional)

- PRACH format 0

- PRACH format B4 (optional)

- PUSCH Msg.3

- SSB

- PDSCH for VoIP

- PDSCH for low data rate service

Email discussion

  - [Email] Topic #10: Link level assumptions, including Rel-17 CovEnh features

For detailed link level assumptions, it seems that companies think table A.1-X of TR38.830 can be reused as baseline. Meanwhile, a few modifications are proposed for NR NTN study. Companies’ proposals can be found in section 6.11 of this document. Assumptions will be discussed per channel/signal and which channel/signal is evaluated in this study is under discussion in the last section. Therefore, FL thinks questions / proposals for link level assumptions from TR38.830 can/should be prepared after concluding the last section.

In addition, several companies refer to features introduced in Rel-17 coverage enhancement WI.

- Four companies [2/ZTE] [10/Nokia, NSB] [13/OPPO] [20/THALES] think that enhanced PUSCH repetition can/should be considered. [13/OPPO] [20/THALES] propose 32 repetitions while [2/ZTE] proposes 20 repetitions.

- [10/Nokia, NSB] thinks Msg3 PUSCH repetition is applicable to NTN.

- Four companies [6/Pana] [10/Nokia, NSB] [17/DCM] [19/LGE] think that TBoMS can/should be considered.

- Three companies [2/ZTE] [6/Pana] [17/DCM] propose to use joint channel estimation (i.e. DMRS bundling) in simulation evaluation. Meanwhile, [10/Nokia, NSB] [18/Lenovo] [19/LGE] point out that joint channel estimation (DMRS bundling) has issue in NTN: Joint channel estimation is applicable only for transmissions with the same TA value.

FL would like to ask which feature should be used in this study and if YES, how.

    - 1st round

Q: Which feature from Rel-17 coverage enhancement should be used in this study? Also please share details (e.g. how).

- A: Enhanced PUSCH repetition

  - A-1: 32 repetitions

  - A-2: 20 repetitions

  - A-3: Others

- B: TBoMS

- C: Joint channel estimation (DMRS bundling)

- D: Msg3 PUSCH repetition

| Company | Feature (e.g. A-1/C) | Comment (e.g. reason of the selection, how to use) |
| --- | --- | --- |
| QC | A-3/B/C/D | For voice, without frame combining, more 20 repetitions are not allowed. For data, 32 repetitions can be considered. |
| Lenovo | A-2/B/C/D |  |
| Apple | A-1/A-2/B/C/D | For PUSCH with VoIP, 20 repetitions are used. For PUSCH with low-rate data, 32 repetition are used. |
| Xiaomi | A-1/A-2 | For PUSCH with VoIP, 20 repetitions are used. For PUSCH with low-rate data, 32 repetition are used. |
| vivo | A-3/C | For VoIP, if 15kHz SCS is considered, at most 20 repetitions are allowed. However, as for other numerology for VoIP or low data rate service, 32 repetitions could be also supported. For low data rate, up to 32 can be used. We do not see the need to use TBoMS which does not have much gain compared to repetition. There’s no need to study Msg3 as we’ve commented in topic 9. |
| Samsung | A-1/A-2 | A: The maximum number of repetitions specified in Rel-17 (32) should be used in NTN evaluation to understand the performance with already specified solutions. Two values could be used: 16 and 32. The latency with 32 repetitions should be further discussed (e.g. for VoIP). B and C: these are Rel-17 solutions that are applicable to NTN (with some constraints). They can be evaluated for NTN, although relative gains are expected to be similar to what found in TN. If RAN1 agrees to perform evaluation, it would be good to agree simulation assumptions for a limited number of cases for easy comparison of companies’ results. Otherwise, it is better to consider gains (e.g., up to 2dB gain for 10% iBLER for eMBB, and 1 dB gain for 2% rBLER for VoIP) reported in TN for link-budget calculation. D: Msg3 PUSCH repetition is another Rel-17 solution applicable to NTN. It can be used 4 as number of repetitions. |
| Nokia, Nokia Shanghai Bell | (A-1)/(B)/C/D | A-1 may not be needed, but if included, we would prefer to go for the 32 repetitions and evaluate compared to 16 repetitions at least for eMBB (32 repetitions may not be feasible for VoIP due to channel overflow). C/D may provide direct benefit in link performance (e.g. BLER). B (TBoMS) is not expected to need any modifications to be directly applicable for NR over NTN. Hence we do not see any need to perform further evaluations. If companies insist on performing TBoMS evaluations, we are fine with that as well. |
| OPPO | A-1/A-2/B/C/D | For VoIP service, 20 repetitions are used. For low data rate service, 32 repetitions are used. |
| Huawei, HiSilicon | A/B/C/D | For feature A, all the repetition number supported by Rel-17 coverage enhancement can be used. There is no need to define the specific number of repetitions as different scenarios can have different repetition number. The method of real channel estimation should be aligned first before applying feature C. |
| LG | A-1/B/C/D | For A-1/B/D, we are fine to study and evaluate. However, we don’t see the problem to support these in NTN scenarios. For Joint channel estimation (DMRS bundling), it is necessary to investigate how to support this feature for NTN scenarios. |
| NEC | A-2/B/D | Although, 32 repetitions is expected to provide the best performance but we need to consider the bottleneck use case of VoIP service for commercial UEs for coverage enhancement. In this case, PUSCH repetitions of 20 times should be considered as baseline considering 20ms packet arrival duration for VoIP packets for simplified scheduling operations. Also, as indicated in 4.9.1, idle mode procedures and Msg3 PUSCH performance needs to be evaluated for commercial UEs. |
| NTT DOCOMO | A-1/A-2/B/C/D | Considering that the basic packet generation periodicity for VoIP is 20ms, up to 20 repetitions could be used for 15kHz SCS, and up to 32 repetitions for 30kHz SCS. |
| ZTE | A/C/D | We are fine with A/C/D. For VoIP, the repetition may be limited to 20 due to the packet size, i.e., A-2 should be used. While for low data rate service, A-1 and A-3 can be considered. |
| CATT | A/B/C/D | The repetition number can be further discussed. |
| Ericsson | A-1/A-2/B/C/D | For VoIP, 32 repetitions per 20 ms is possible with SCS=30 kHz. |
| Thales | A-1(for data)/A-2 (for Voice)/B/C/D |  |
| Spreadtrum | A/B/C/D |  |
| MediaTek | A-1, C | For VoIP, up to 32 repetitions can be considered depending on assumption for voice codec, latency, and bandwidth. For data, it depends on assumption for data rates. DM RS bundling can improved channel estimation at low SNR |
| Ligado | A-3 | Adaptive repetitions, not considered in Release 17, should be considered here. |
| Sony | A-D | Release-17 coverage enhancement features should be the baseline. Hence, A-D should be considered. |
| Lockheed | A-1, C, D |  |
| Panasonic | A-2/B/C/D |  |

    - Summary of 1st round

Summary of inputs is the following:

- A: Enhanced PUSCH repetition: HW (without agreeing specific number), CATT (discuss repetition number), Sony (3)

  - A-1: 32 repetitions: QC (for data), Apple (for data), Xiaomi (for data), vivo (for data and VoIP with SCS 30 kHz), Samsung, (Nokia (for data)), OPPO (for data), LGE, NEC (for data), DCM (for data and VoIP with SCS 30 kHz), ZTE (for data), E (for data and VoIP with SCS 30 kHz), Thales (for data), MTK, Lockheed (15)

  - A-2: 20 repetitions: QC (for VoIP), Lenovo, Apple (for VoIP), Xiaomi (for VoIP), vivo (for VoIP with SCS 15 kHz), OPPO (for VoIP), NEC (for VoIP), DCM (for VoIP with SCS 15 kHz), ZTE (for VoIP), E (for VoIP with SCS 15 kHz), Thales (for VoIP), Pana (12)

  - A-3: Others: Ligado (1)

- B: TBoMS: QC, Lenovo, Apple, (Samsung), (Nokia), OPPO, HW, LGE, NEC, DCM, CATT, E, Thales, Sony, Pana (15)

- C: Joint channel estimation (DMRS bundling): QC, Lenovo, Apple, (Samsung), Nokia, OPPO, HW, LGE, DCM, ZTE, CATT, E, Thales, MTK, Sony, Lockheed, Pana (17)

- D: Msg3 PUSCH repetition: QC, Lenovo, Apple, (Samsung), Nokia, OPPO, HW, LGE, NEC, DCM, ZTE, CATT, E, Thales, Sony, Lockheed, Pana (17)

Based on the above discussion, all Rel-17 CovEnh mechanisms would be agreeable for this study while there are a few objections. On repetition number, companies’ views are highly aligned as 32 can be used for data and 20 should be assumed for VoIP. At the same time, there are comments that 32 can be used also for VoIP with SCS 30 kHz. FL thinks this comment is valid. In addition, FL assumes max number of Msg.3 PUSCH repetitions is straightforward to use in evaluation. On HW’s comment, FL’s understanding is that what should be evaluated is performance at the worst channel quality with all applicable features with the best way. FL does not see any need to evaluate less number of repetitions.

Meanwhile, as pointed out in [10/Nokia, NSB] [18/Lenovo] [19/LGE], how to apply joint channel estimation (DMRS bundling) in NTN is an issue. FL would like to ask whether/how to clarify this aspect for this study.

    - 2nd round

Proposal 10-1_v0

The following features introduced in Rel-17 Coverage enhancement WI are applied in coverage evaluation of NR NTN.

- 32 PUSCH repetitions for VoIP with SCS 30 kHz and low-data rate service

- 20 PUSCH repetitions for VoIP with SCS 15 kHz

- TBoMS

- [Joint channel estimation (DMRS bundling)]

- 8 Msg.3 PUSCH repetitions

Q: If proposal 1-1 is agreed (for 1st/2nd bullets), and if corresponding channel is agreed as evaluation target channel, do you agree the above proposal? If the answer is NO, please share the reason and how to modify.

| Company | YES/NO | Comment (e.g. reason of NO) |
| --- | --- | --- |
| vivo | No | Support other features except TBoMS. We do not see the need to use TBoMS which does not have much gain compared to repetition. Although we do not see the need to study Msg3 PUSCH, the maximum number of repetitions agreed in Rel-17 coverage enhancement topic is 16 instead of 8. Agreement For the number of repetitions configured by numberOfMsg3Repetitions, support {1, 2, 3, 4, 7, 8, 12, 16}. |
| MediaTek | No | Gains of TBoMS are not clear compared to repetitions. For VoIP, up to 32 repetitions may be considered with lower codec rates Joint channel estimation (DMRS bundling) seems desirable enhancements for channel estimation at low SNR. Up to 16 Msg3 PUSCH repetitions can be assumption |
| Lockheed | No | We do not see the need to study TBoMS. |
| Apple |  | We are generally fine with the proposal. The Msg.3 PUSCH has 16 repetitions can be considered. |
| Lenovo | Yes |  |
| Samsung | Yes |  |
| Panasonic | Yes | For Msg3 PUSCH repetitions, up to 16 repetitions should be assumed. |
| LG | Yes |  |
| NTT DOCOMO | Yes |  |
| ZTE | No | We do not seed the need to study TBoMS as the gain is not clear compared to repetitions. |
| OPPO | Yes |  |
| Thales | Yes | We are fine with the proposal |
| Huawei, HiSilicon | Yes | Some comments on the concern for TBoMS. The proposal here is regarding the baseline. The intention is not to study TBoMS. We should consider TBoMS adopted in Rel-17 as the baseline for the study of any future enhancement. |
| Nokia, Nokia Shanghai Bell | Yes | According to our understanding the current coverage enhancements for Rel-17 supports up to 16 Msg3 PUSCH repetitions. So we suggest to change from 8 to 16 Msg.3 PUSCH repetitions as feature to be applied in coverage evaluations in NR NTN. Further, it worth mentioning that it seems that RAN2 is considering to include support for Msg3 PUSCH repetitions already with Rel-17 maintenance, so it may be that this particular enhancement is not needed for Rel-18 (since it may already be present). |
| Ericsson | Yes with comment | Up to 16 Msg3 repetitions can be used |
| NEC | Yes |  |
| QC |  | SCS 30 kHz can be deprioritized. For voice, packet combing with more repetitions can be evaluated. |

Q: Do you think clarification on how to apply joint channel estimation (DMRS bundling) for coverage evaluation is necessary?

| Company | YES/NO | Comment |
| --- | --- | --- |
| vivo | Yes | Note that RAN1 needs to discuss potential NTN specific updates required to support DMRS bundling in NTN. Considering that the implementation of frequency offset pre-compensation and post-compensation could be assumed, DMRS bundling could be a potential feature applicable for NTN coverage enhancement. |
| Apple | Yes | The performance gain from joint channel estimation needs to be considered in NTN coverage enhancement. |
| Lenovo |  | It can be assumed and reported by each company for the simulation. |
| Panasonic | No | Not necessary in evaluation phase. How to enhance would be topic for later phase. |
| LG | Yes | In our view, it is necessary to discuss how to support and enable DMRS bundling in NTN environment, having larger RTT and specific TA adaptation. |
| OPPO | Yes | It can be assumed and reported by companies. |
| Huawei, HiSilicon | Yes | Companies can report how they do the joint channel estimation. |
| Nokia, Nokia Shanghai Bell |  | This might need further discussion. |
| Ericsson | Yes | This needs to be discussed further. |
| MediaTek | Yes | Assumption for channel estimation used in evaluation can be mentioned by company |
| QC | Yes |  |
| Ligado | Yes | Joint channel estimation (DMRS bundling) across 2 or 3 slots would provide more accurate channel estimation compared with conventional single slot based approach, especially in the case of LMS channel fading or blockage scenario. Since joint channel estimation does not need to increase the DMRS overhead, it is more about processing implementation at eNB, and it would be promising in PUSCH (or NPUSCH) coverage enhancement under certain poor channel conditions. |

    - Summary of 2nd round

Firstly FL would like to emphasize that now we are not talking about how NTN-specific enhancement is specified. Just listing Rel-17 features to be used in simulation evaluation to see whether current coverage performance is enough or not.

It seems that majority are OK with proposal 10-1_v0 (if we have corresponding agreement). Regarding TBoMS, FL’s intention is what HW commented kindly. Probably some modification to clarify the intention including repetition bullets would be better. On Msg.3 repetitions, ‘8’ is FL’s mistake. Thank you for pointing out. For QC’s comment, this proposal will be asked after solving ‘If ...’ part in question for proposal 10-1_v0. SCS aspect will be discussed separately.

For joint channel estimation (DMRS bundling), it seems that companies think some clarification is necessary and it can be reported by companies with simulation results. Also FL thinks that discussions of details for each mechanism is unnecessary/undesirable in consideration of time limitation.

In addition, as commented in section 4.11, channel model/channel estimation/evaluation scenario/UE speed/doppler spread and other aspects can be discussed without any conclusion of section 4.9. FL prepares corresponding questions below.

    - 3rd round

Proposal 10-1_v1

If proposal 1-1 is agreed (for 1st/2nd bullets), and if corresponding channel (including SCS) is agreed as evaluation target channel, the following features introduced in Rel-17 Coverage enhancement WI can be applied in coverage evaluation of NR NTN.

- Max 32 PUSCH repetitions for VoIP with SCS 30 kHz and low-data rate service

- Max 20 PUSCH repetitions for VoIP with SCS 15 kHz

- TBoMS

- Joint channel estimation (DMRS bundling)

  - Companies are encouraged to report how to apply

- Max 16 Msg.3 PUSCH repetitions

Q. Only if you are not OK with this proposal, please share view. Note that this proposal is tried for agreement after solving ‘if’ part.

| Company | Comment |
| --- | --- |
| vivo | Support the proposal. |
| Lockheed | Support the proposal |
| Apple | Support the proposal. |
| Lenovo | Support. |
| Xiaomi | Support |
| Thales | Support FL proposal |
| Nokia, Nokia Shanghai Bell | We are OK with this. |
| Samsung | Actually, listed features are related to PUSCH repetition. Does it implicitly mean that NTN coverage enhancements are mainly focusing on PUSCH? This is just a clarification question although this is not related to moderator’s question. |
| Huawei, HiSilicon | OK with it. However, even without the agreement of proposal 1-1, the baseline assumption is also valid. Not sure whether we should have “If proposal 1-1 is agreed (for 1st/2nd bullets), and if corresponding channel (including SCS) is agreed as evaluation target channel,”. |

Proposal 10-2_v0

For LLS, the following are assumed for all channels/signals

- Channel model/Delay spread

  - Option 1: Channel model as in Table 6.1.2-4 of TR38.821

  - Option 2: Others

- Evaluation scenario

  - Option A: Rural

  - Option B: Sub-urban

  - Option C: Urban

  - Option D: Dense urban

- Channel estimation: Realistic estimation

- SCS

  - Option X: 15 kHz, 30 kHz

  - Option Y: 15 kHz only

- UE speed: 3 km/h

- Frequency drift: Frequency drift as in Table 6.1.2-4 of TR38.821

Q: For channel model/delay spread, which option should be selected?

| Company | Option 1 or 2 | Comment |
| --- | --- | --- |
| vivo | Option 1 |  |
| OPPO | Option 1 | There are different channel models defined in TR38.811, including NTN-CDL-A/B(NLOS), NTN-CDL-A/B(LOS), NTN-TDL-A/B(NLOS), and NTN-TDL-A/B(LOS). Given that we have agreed to evaluate only handset terminal in NTN, NTN-TDL-A and NTN-TDL-C can be further selected for simplicity. |
| Apple | Option 1 |  |
| QC | Option 1 |  |
| Panasonic | Option 1 |  |
| Lenovo | Option 1 |  |
| LG | Option 1 |  |
| ETRI | Option 1 |  |
| Thales | Option 1 |  |
| ZTE | Option 1 |  |
| Nokia, Nokia Shanghai Bell | Option 1 |  |
| Huawei, HiSilicon | Option1 | We agree with OPPO that it is better to further align with the used channel model in 38.811. |
| Ericsson | Option 1 | Agree with OPPO. NTN-TDL-A and NTN-TDL-C is fine. |

Q: For evaluation scenario, which option should be selected?

| Company | Option A to D | Comment |
| --- | --- | --- |
| vivo | Option A/B |  |
| OPPO | Option A/C | Rural and urban scenarios can be selected as evaluation scenarios in NTN which are aligned with R17 CovEnh. Furthermore, the propagation condition of rural and urban scenario can be LOS and NLOS respectively. |
| Apple | Option A/B |  |
| Panasonic | Option A | Rural (LOS) should have higher priority. |
| Lenovo | Option A/C | We think Option A should have higher priority. We also think Option C may be necessary in case of some emergency case when TN is not available. |
| LG | Option A | Option C can be considered optionally for NLOS condition. |
| ETRI | Option A/C | same view as Lenovo |
| ZTE | Option A/B |  |
| Nokia, Nokia Shanghai Bell | Option A | This is the most like scenario for NTN (considering that GNSS is also assumed) |
| Huawei, HiSilicon | Option A/B |  |
| Ericsson | Option B | In TR 38.821, suburban was selected for LLS for PRACH (Table 6.1.2-1) and DL sync (Table 6.1.2-2). We can reuse this assumption. |

Q: For SCS, which option should be selected?

| Company | Option X or Y | Comment |
| --- | --- | --- |
| vivo | Option Y |  |
| OPPO | Option Y |  |
| Panasonic | Option Y |  |
| Lenovo | Option X | We have already list repetition number 32 for SCS=30KHz, so SCS=30KHz should be simulated for some cases. |
| Thales | Option X |  |
| ZTE | Option Y |  |
| Nokia, Nokia Shanghai Bell | Option Y |  |
| Huawei, HiSilicon | Option Y |  |
| Ericsson | Option X | 30 kHz can be used to enable evaluation of 32 repetitions for VoIP. |

Q: For others, do you agree the proposal? If the answer is NO, please share which should be updated.

| Company | YES/NO | Comment |
| --- | --- | --- |
| vivo | Yes |  |
| OPPO | Yes |  |
| Lenovo | Yes |  |
| ETRI | Yes |  |
| ZTE | Yes |  |
| Nokia, Nokia Shanghai Bell | No | If we include frequency drift we also need to have accurate modeling of when the UE will re-read the serving satellite ephemeris such that the frequency (Doppler) drift is reset. Since we do not have any commitment from UE side on this aspect we cannot have accurate modeling. If any frequency drift/error is to be modelled, we would prefer to have a static offset to model the average error. |
| Huawei, HiSilicon | Yes, with some further alignment | For realistic estimation, it would be useful to align the channel estimation method, e.g. LS channel estimation and linear interpolation. |
| Ericsson | No | Agree with Nokia. We do not have a model for frequency drift after UL frequency pre-compensation. A fixed frequency offset of e.g. 0.1 ppm would be preferred. |

    - Summary of 3rd round

For Rel-17 features

Most companies are OK with the proposal. On Samsung’s comment, intention of this proposal is not to focus on PUSCH. Such a discussion will be triggered in 4th round. In this proposal, just to confirm applying Rel-17 features when possible.

To HW’s comment, FL will suggest to agree proposal 1-1 before discussing proposal 10-1. Thus ‘if proposal 1-1 is agreed (for 1st/2nd bullets)’ will be removed accordingly. On ‘if corresponding channel (including SCS) is agreed as evaluation target channel’, such a discussion will be triggered in 4th round. Whether PUSCH/PUSCH Msg3 with 15 kHz/30kHz are evaluated or not is dependent on the subsequent discussion. Thus that part in proposal 10-1 is necessary in current stage.

For detailed parameters to be used in LLS for all channels/signals

Companies are OK to reuse channel model defined in Table 6.1.2-4 of TR38.821, and OPPO/HW/Ericsson suggest to clarify which channel model is assumed. NTN-TDL-A (NLOS) / NTN-TDL-C (LOS) is suggested by OPPO/Ericsson.

For evaluation scenario, most companies (10) think Option A = rural scenario should be assumed. Several companies (5) prefer to include Option B = sub-urban scenario, and some companies (4) suggest to consider Option C = urban scenario. Ericsson suggest to assume Option B as in table 6.1.2-1 of TR38.821. OPPO think Option C can be assumed as R17 CovEnh, and Lenovo/ETRI argues that Option C might be necessary for some emergency case. In addition, LOS/NLOS is discussed by some companies. FL feels that it is straightforward to assume rural scenario based on these inputs. Regarding other scenarios, FL suggests to drop them to reduce simulation burden.

For SCS, Option X = both 15 kHz and 30 kHz is preferred by 3 companies while Option Y = only 15 kHz is suggested by 6 companies. Then FL recommends to go with Option Y.

For other part, Nokia/Ericsson comment that the frequency model cannot be used and fixed frequency and fixed offset like 0.1 ppm should be assumed instead. Meanwhile other companies think the model is OK; thus FL would like to ask which is more reasonable way. One note is that frequency offset for each channel/signal will be discussed in 4th round. HW suggest to agree a certain channel estimation mechanism, but FL does not fine such an assumption in TR38.821/TR38.830. With report of details from companies, further alignment would be unnecessary.

Proposal 10-1_v1

If proposal 1-1 is agreed (for 1st/2nd bullets), and if corresponding channel (including SCS) is agreed as evaluation target channel, the following features introduced in Rel-17 Coverage enhancement WI can be applied in coverage evaluation of NR NTN.

- Max 32 PUSCH repetitions for VoIP with SCS 30 kHz and low-data rate service

- Max 20 PUSCH repetitions for VoIP with SCS 15 kHz

- TBoMS

- Joint channel estimation (DMRS bundling)

  - Companies are encouraged to report how to apply

- Max 16 Msg.3 PUSCH repetitions

Proposal 10-2_v1

For LLS, the following are assumed for all channels/signals

- Channel model/Delay spread

  - Option 1: Channel model as in Table 6.1.2-4 of TR38.821, assuming NTN-TDL-A (NLOS) and NTN-TDL-C (LOS)

  - Option 2: Others

- Evaluation scenario

  - Option A: Rural (LOS/NLOS)

  - Option B: Sub-urban

  - Option C: Urban

  - Option D: Dense urban

- Channel estimation: Realistic estimation

  - Companies are encouraged to report channel estimation method.

- SCS

  - Option X: 15 kHz, 30 kHz

  - Option Y: 15 kHz only

- UE speed: 3 km/h

- Frequency drift

  - Option P: Frequency drift as in Table 6.1.2-4 of TR38.821

  - Option Q: Not assumed

  - Note: Frequency offset for each channel/signal is discussed separately, based on table 6.1.2-1/6.1.2-2/6.1.2-4 of TR38.821

    - 4th round

For Rel-17 features, FL would like to suggest to have discussion for the proposal below via reflector. Note that FL updates a bit according to agreement we had.

Proposal 10-1_v2

If proposal 1-1 is agreed (for 1st/2nd bullets), and iIf corresponding channel (including SCS) is agreed as evaluation target channel, the following features introduced in Rel-17 Coverage enhancement WI can be applied in coverage evaluation of NR NTN.

- Max 32 PUSCH repetitions for VoIP with SCS 30 kHz and low-data rate service

- Max 20 PUSCH repetitions for VoIP with SCS 15 kHz

- For VoIP, max 20 PUSCH repetitions if SCS = 15 kHz and packet combining/HARQ are not applied; otherwise, max 32 PUSCH repetitions with consideration of the impact on E2E latency

- For low-data rate service, max 32 PUSCH repetitions

- TBoMS

- Joint channel estimation (DMRS bundling)

  - Companies are encouraged to report how to apply

- Max 16 Msg.3 PUSCH repetitions

Email discussion outcome: Agreed

For detailed parameters for all channels/signals, FL would like to trigger further discussion here.

Proposal 10-2_v2

For coverage performance evaluation, the following are assumed for all channels/signals

- Channel model/Delay spread

  - Option 1: Channel model as in Table 6.1.2-4 of TR38.821, assuming NTN-TDL-A (NLOS) and NTN-TDL-C (LOS)

  - Option 2: Others

- Evaluation scenario

  - Option A: Rural (LOS/NLOS)

  - Option B: Sub-urban

  - Option C: Urban

  - Option D: Dense urban

- Channel estimation: Realistic estimation

  - Companies are encouraged to report channel estimation method.

- SCS

  - Option X: 15 kHz, 30 kHz

  - Option Y: 15 kHz only

- UE speed: 3 km/h

- Frequency drift

  - Option P: Frequency drift as in Table 6.1.2-4 of TR38.821

  - Option Q: Not assumed

  - Note: Frequency offset for each channel/signal is discussed separately, based on table 6.1.2-1/6.1.2-2/6.1.2-4 of TR38.821

Q: For frequency drift, which option is preferred?

| Company | Option | Comment |
| --- | --- | --- |
| Panasonic |  | As mentioned by Nokia and Ericsson, a fixed offset like 0.1ppm is preferred. |
| Nokia, Nokia Shanghai Bell | Q | The frequency drift modeling causes continuous accumulation of frequency shift, so if no reset period is agreed, the reported results from companies would significantly change. For clarification, LLS with continuous X frames transmission with no reset of cumulative frequency shift due to drift would have worse results than another LLS with Y<X frames, because the cumulative frequency shift in the last X-Y frames of 1st case is higher than the maximum in Y consecutive frames (last frame) and leads to significant difference in the average BLER. So either we don't assume frequency drift (preferred) and use fixed residual offset of 0.1ppm or higher, or we should agree on the reset period for cumulative frequency shift due to drift. |
| ZTE | Q | Fixed frequency offset is preferred to align the evaluation. |
| Apple | Q | Easy to align the evaluation results. |
| Ericsson | Q |  |
| Huawei, HiSilicon | Q |  |

Q: For other part, do you agree the above proposal? If the answer is NO, please share the reason and what should be added/modified/removed.

| Company | Option | Comment |
| --- | --- | --- |
| ZTE | Yes |  |
| Ericsson | Yes |  |
| Huawei, HiSilicon |  | We don’t see the problem to make sub-urban as optional, which is not mandatory work for all companies. |

For each target channel/signal, detailed parameters must be agreed for simulation evaluation. As summarized in section 4.10, basically table A.1-X of TR38.830 can be reused; at the same time, NTN-specific assumptions can be found in table 6.1.2-1/6.1.2-2/6.1.2-4. FL prepares tables merged based on the tables in TR38.830/TR38.821 as below. FL would like to ask what should be updated/modified/added.

Proposal 10-3_v0

For coverage evaluation of PUSCH in NR NTN, the following table is assumed.

| Parameter | Value |
| --- | --- |
| Frequency hopping | w/ or w/o frequency hopping |
| BLER | For eMBB low data rate service, w/ HARQ, 10% iBLER; w/o HARQ, 10% iBLER. For VoIP, 2% rBLER. |
| Number of UE transmit chains | 1, 2 (optional) |
| DMRS configuration | For 3km/h: Type I, 1 or 2 DMRS symbol, no multiplexing with data. For 120km/h, (Optional: 30km/h): Type I, 2 or 3 DMRS symbol, no multiplexing with data. For frequency hopping: Type I, 1 or 2 DMRS symbol for each hop, no multiplexing with data. PUSCH mapping Type, the number of DMRS symbols and DMRS position(s) are reported by companies. |
| Waveform | DFT-s-OFDM, CP-OFDM (optional) |
| SCS | 30kHz for TDD, 15kHz for FDD. |
| PUSCH duration | 14 OS |
| Repetitions | For eMBB, w/o repetition as baseline, w/ repetition (optional). For VoIP, w/ type A repetition, optional for type B repetition. The actual number of repetitions is reported by companies. |
| HARQ configuration | For eMBB, whether HARQ is adopted is reported by companies. For VoIP, w/ HARQ. The maximum number of HARQ transmission (limited by frame structure and latency requirements) can be reported by companies. |
| PRBs/TBS/MCS for eMBB low data rate service | Any value of PRBs, and corresponding MCS index, reported by companies will be considered in the discussion. Companies are encouraged to use [30 PRBs for 1Mbps, 4 PRBs for 100kbps, 1 PRB for 30kbps] as a starting point. TBS can be calculated based on e.g. the number of PRBs, target data rate, frame structure and overhead. |
| PRBs/MCS for VoIP | [4] PRBs for VoIP as starting point. Other values of PRBs can be reported by companies. QPSK, pi/2 BPSK (optional) |
| Frequency offset | Frequency offset defined in table 6.1.2-4 of TR38.821 |

Q: If PUSCH is agreed as evaluation target, do you agree the above proposal? If the answer is NO, please share the reason and what should be added/modified. In addition, please share how many PRBs should be used.

| Company | YES/NO | Comment |
| --- | --- | --- |
| Panasonic |  | In HARQ configuration, “Whether HARQ is adopted and the maximum number of HARQ transmission (limited by latency requirement) can be reported by companies.” Frequency offset should be based on outcome of Proposal 10-2_v2. |
| Nokia, Nokia Shanghai Bell | Mostly yes | For the low data rate service we may consider the lower data rate of 3 kbps depending on the outcome of the direct email discussion. However, such decision should preferably be discussed with SA2 as well. Additionally, the frequency offset should be aligned with frequency offset assumptions in Proposal 10-2. |
| ZTE | Yes but | For VoIP, we may consider 2 PRBs as starting point. The frequency offset depends on the the discussion result of proposal 10-2. |
| Thales |  | Waveform: DFT-s-OFDM, CP-OFDM (optional) Repetitions: The following value set for repetition factor: {1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32} |
| Ericsson | Mostly yes | Recommendation on number of PRBs can be removed as discussed in Topic#6, 2nd round. Each company should report their assumptions. Assume fixed frequency offset of e.g. 0.1 ppm. |
| Huawei, HiSilicon |  | No need to recommend number of PRBs used. |

Proposal 10-4_v0

For coverage evaluation of PUCCH in NR NTN, the following table is assumed.

| Parameter | Value |
| --- | --- |
| PUCCH format | Format 1, 2bits UCI. Format 3, 4bits (3 bits A/N + 1 bit SR)/11/22 bits UCI |
| Frequency hopping | w/ frequency hopping |
| BLER | - For PUCCH format 1: DTX to ACK probability: 1%. NACK to ACK probability: 0.1%. ACK missed detection probability: 1%. - For PUCCH format 3: BLER for Ack/Nack, SR: 1% BLER for CSI: 1%, optional for 10%. |
| Number of UE transmit chains | 1 |
| DMRS configuration | Number of DMRS symbols for PUCCH Format 3: Reported by companies |
| SCS | 30kHz for TDD, 15kHz for FDD. |
| Repetitions | w/ repetition (optional), w/o repetition for PUCCH. The maximum number of repetitions is 8. |
| PUCCH duration | 14 OS |
| Number of PRBs | 1 PRB |
| Frequency offset | Frequency offset defined in table 6.1.2-4 of TR38.821 |

Q: If PUCCH is agreed as evaluation target, do you agree the above proposal? If the answer is NO, please share the reason and what should be added/modified.

| Company | YES/NO | Comment |
| --- | --- | --- |
| Panasonic |  | As agreed, “Companies are encouraged to report BWP bandwidth” should be added in frequency hopping. Frequency offset should be based on outcome of Proposal 10-2_v2. |
| Ericsson | Mostly yes | Assume fixed frequency offset of e.g. 0.1 ppm. |

Proposal 10-5_v0

For coverage evaluation of PRACH in NR NTN, the following table is assumed.

| Parameter | Value |
| --- | --- |
| Format | Format 0, Format B4 |
| SCS | Reported by companies. |
| Performance metric | 1% missed detection at 0.1% false alarm probability 10% missed detection: reported by companies if this value is used |
| Number of UE transmit chains | 1, 2 (optional) |
| Frequency offset | Frequency offset defined in table 6.1.2-1 of TR38.821 |
| Timing offset | Timing offset defined in table 6.1.2-2 of TR38.821 |
| Other parameters | Reported by companies. |

Q: If PRACH is agreed as evaluation target, do you agree the above proposal? If the answer is NO, please share the reason and what should be added/modified.

| Company | YES/NO | Comment |
| --- | --- | --- |
| Panasonic |  | Frequency offset should be based on outcome of Proposal 10-2_v2. Timing offset would not be needed because performance metric is not timing error. |
| ZTE | Yes but | We can only focus on long preamble format, which provides better coverage performance, to reduce work load. The frequency offset depends on the the discussion result of proposal 10-2. |
| Ericsson | Mostly yes | Assume fixed frequency offset of e.g. 0.1 ppm. Timing offset defined in table 6.1.2-2 of TR38.821 seems not applicable since it does not assume that TA pre-compensation is used by the UE. |

Proposal 10-6_v0

For coverage evaluation of PUSCH Msg.3 in NR NTN, the following table is assumed.

| Parameter | Value |
| --- | --- |
| Frequency hopping | w/ or w/o frequency hopping |
| Number of UE transmit chains | 1, 2 (optional) |
| Number of DMRS symbol | w/o frequency hopping: 3, w/ frequency hopping: 2 for each hop |
| Waveform | DFT-s-OFDM |
| SCS | 30kHz for TDD, 15kHz for FDD. |
| HARQ configuration | For eMBB low data rate service, whether HARQ is adopted is reported by companies. For VoIP, w/ HARQ. The maximum number of HARQ transmission (limited by frame structure and latency requirements) can be reported by companies. |
| PUSCH duration | 14 OS |
| Number of PRBs | 2 |
| TBS | 56 bits |
| Other parameters | Reported by companies. |
| Frequency offset | Frequency offset defined in table 6.1.2-4 of TR38.821 |

Q: If PUSCH Msg.3 is agreed as evaluation target, do you agree the above proposal? If the answer is NO, please share the reason and what should be added/modified.

| Company | YES/NO | Comment |
| --- | --- | --- |
| Panasonic |  | Frequency offset should be based on outcome of Proposal 10-2_v2. |
| ZTE | Yes | The frequency offset depends on the the discussion result of proposal 10-2. |
| Ericsson | Mostly yes | Assume fixed frequency offset of e.g. 0.1 ppm. |

Proposal 10-7_v0

For coverage evaluation of SSB in NR NTN, the following table is assumed.

| Parameter | Value |
| --- | --- |
| Number of UE receive chains | 4 for 4GHz/2.6GHz, 2 or 4 for 2GHz, 2 for 700MHz |
| SCS | 30kHz for TDD, 15kHz for FDD. |
| Periodicity | 20ms |
| Performance metric | Combination of 4 SSBs in 80ms. Note: UE is not assumed to know the SS/PBCH block index |
| Frequency offset | Frequency offset defined in table 6.1.2-1 of TR38.821 |
| Other parameters | Reported by companies. |

Q: If SSB is agreed as evaluation target, do you agree the above proposal? If the answer is NO, please share the reason and what should be added/modified.

| Company | YES/NO | Comment |
| --- | --- | --- |
| ZTE | Yes | The frequency offset depends on the the discussion result of proposal 10-2. |
| Ericsson | Yes |  |

Proposal 10-8_v0

For coverage evaluation of PDSCH in NR NTN, the following table is assumed.

| Parameter | Value |
| --- | --- |
| BLER | For eMBB low data rate service, w/ HARQ, 10% iBLER; w/o HARQ, 10% iBLER. For VoIP, 2% rBLER. |
| Waveform | CP-OFDM |
| Number of UE receive chains | 4 for 4GHz/2.6GHz, 2 or 4 for 2GHz, 2 for 700MHz |
| SCS | 30kHz for TDD, 15kHz for FDD. |
| HARQ configuration | For eMBB, whether HARQ is adopted is reported by companies. For VoIP, w/ HARQ. The maximum number of HARQ transmission (limited by frame structure and latency requirements) can be reported by companies. |
| DMRS configuration | 3 DMRS symbols is used for PDSCH of Msg.2. For 3km/h: Type I, 1 or 2 DMRS symbol, no multiplexing with data. For 120km/h, (Optional: 30km/h): Type I, 2 or 3 DMRS symbol, no multiplexing with data. For frequency hopping: Type I, 1 or 2 DMRS symbol for each hop, no multiplexing with data. PDSCH mapping Type, the number of DMRS symbols and DMRS position(s) are reported by companies. |
| PRBs/MCS/TBS | Reported by companies. |
| PDSCH duration | 12 OS For PDSCH of Msg.4, 12 OS |
| Payload size for PDSCH of Msg.4 | 1040 bits |
| Frequency offset | Frequency offset defined in table 6.1.2-4 of TR38.821 |
| Other parameters | Reported by companies. |

Q: If PDSCH is agreed as evaluation target, do you agree the above proposal? If the answer is NO, please share the reason and what should be added/modified.

| Company | YES/NO | Comment |
| --- | --- | --- |
| Panasonic |  | In HARQ configuration, “frame structure” in “(limited by frame structure and latency requirements)” should be removed because there would be no limitation by frame structure in FDD. Frequency offset should be based on outcome of Proposal 10-2_v2. |
| Ericsson | Yes |  |

    - Summary of 4th round

For common parameters

Regarding frequency drift/offset, FL finds that companies think frequency drift should not be assumed and a fixed offset of 0.1 ppm for any channel/signal is fine. The thinking is aligned among companies; FL reflects this direction.

On evaluation scenario, HW recommends to allow sub-urban as optional. Although FL still believes that only rural is better/more realistic as summarized in the last round, FL tries to add sub-urban as optional. If concern is raised, FL will remove it again.

For other part, there is no concern; they are maintained as they are.

For PUSCH parameters

It seems that alignment of number of PRBs is quite difficult. As some companies suggest, FL removes that part and companies should report which number is used in their simulation. Repetition factor is to be agreed in different proposal. Frequency offset row is removed since now it is captured as a common parameter. On HARQ, FL updates as ‘whether/how HARQ is adapted’ to include how many number of HARQ retransmissions is used.

For PUCCH parameters

Frequency offset row is removed since now it is captured as a common parameter. BWP bandwidth is already covered in the agreement. FL assumes no need to repeat the same thing here.

For PRACH parameters

Frequency offset row is removed since now it is captured as a common parameter. Timing offset row is removed based on comments from Panasonic/Ericsson.

For PUSCH Msg.3 parameters

Frequency offset row is removed since now it is captured as a common parameter. ‘whether/how HARQ is adapted’ is used as abovementioned.

For SSB

Frequency offset row is removed since now it is captured as a common parameter. Other part seems to be OK.

For PDSCH

Frequency offset row is removed since now it is captured as a common parameter. Text of HARQ row for PUSCH is copied here.

    - 5th round

Proposal 10-2_v3

For coverage performance evaluation, the following are assumed for all channels/signals

- Channel model/Delay spread

  - Channel model as in Table 6.1.2-4 of TR38.821, assuming NTN-TDL-A (NLOS) and NTN-TDL-C (LOS)

- Evaluation scenario

  - Rural (LOS/NLOS)

  - Sub-urban (LOS/NLOS) (optional)

- Channel estimation: Realistic estimation

  - Companies are encouraged to report channel estimation method.

- SCS

  - 15 kHz only

- UE speed: 3 km/h

- Frequency drift: Not assumed

- Frequency offset: 0.1 ppm

Proposal 10-3_v1

For coverage evaluation of PUSCH in NR NTN, the following table is assumed.

| Parameter | Value |
| --- | --- |
| Frequency hopping | w/ or w/o frequency hopping |
| BLER | For low data rate service, w/ HARQ, 10% iBLER; w/o HARQ, 10% iBLER. For VoIP, 2% rBLER. |
| Number of UE transmit chains | 1, 2 (optional) |
| DMRS configuration | For 3km/h: Type I, 1 or 2 DMRS symbol, no multiplexing with data. For frequency hopping: Type I, 1 or 2 DMRS symbol for each hop, no multiplexing with data. PUSCH mapping Type, the number of DMRS symbols and DMRS position(s) are reported by companies. |
| Waveform | DFT-s-OFDM, CP-OFDM (optional) |
| PUSCH duration | 14 OS |
| Repetitions | w/ type A repetition, optional for type B repetition. The actual number of repetitions is reported by companies. |
| HARQ configuration | Whether/How HARQ is adopted is reported by companies. |
| PRBs/TBS/MCS for low data rate service | Any value of PRBs, and corresponding MCS index, reported by companies will be considered in the discussion. TBS can be calculated based on e.g. the number of PRBs, target data rate, frame structure and overhead. |
| PRBs/MCS for VoIP | Any value of PRBs reported by companies will be considered in the discussion. QPSK, pi/2 BPSK (optional) |
| Frequency offset | Frequency offset defined in table 6.1.2-4 of TR38.821 |

Proposal 10-4_v1

For coverage evaluation of PUCCH in NR NTN, the following table is assumed.

| Parameter | Value |
| --- | --- |
| PUCCH format | Format 1, 2bits UCI. Format 3, 11 bits UCI |
| Frequency hopping | w/ frequency hopping |
| BLER | - For PUCCH format 1: DTX to ACK probability: 1%. NACK to ACK probability: 0.1%. ACK missed detection probability: 1%. - For PUCCH format 3: BLER for Ack/Nack, SR: 1% BLER for CSI: 1%, optional for 10%. |
| Number of UE transmit chains | 1 |
| DMRS configuration | Number of DMRS symbols for PUCCH Format 3: Reported by companies |
| Repetitions | w/ repetition. The maximum number of repetitions is 8. |
| PUCCH duration | 14 OS |
| Number of PRBs | 1 PRB |
| Frequency offset | Frequency offset defined in table 6.1.2-4 of TR38.821 |

Proposal 10-5_v1

For coverage evaluation of PRACH in NR NTN, the following table is assumed.

| Parameter | Value |
| --- | --- |
| Format | Format 0, Format B4 |
| SCS | Reported by companies. |
| Performance metric | 1% missed detection at 0.1% false alarm probability 10% missed detection: reported by companies if this value is used |
| Number of UE transmit chains | 1, 2 (optional) |
| Frequency offset | Frequency offset defined in table 6.1.2-1 of TR38.821 |
| Timing offset | Timing offset defined in table 6.1.2-2 of TR38.821 |
| Other parameters | Reported by companies. |

Proposal 10-6_v1

For coverage evaluation of PUSCH Msg.3 in NR NTN, the following table is assumed.

| Parameter | Value |
| --- | --- |
| Frequency hopping | w/ or w/o frequency hopping |
| Number of UE transmit chains | 1, 2 (optional) |
| Number of DMRS symbol | w/o frequency hopping: 3, w/ frequency hopping: 2 for each hop |
| Waveform | DFT-s-OFDM |
| HARQ configuration | Whether/How is adopted is reported by companies. |
| PUSCH duration | 14 OS |
| Number of PRBs | 2 |
| TBS | 56 bits |
| Other parameters | Reported by companies. |
| Frequency offset | Frequency offset defined in table 6.1.2-4 of TR38.821 |

Proposal 10-7_v1

For coverage evaluation of SSB in NR NTN, the following table is assumed.

| Parameter | Value |
| --- | --- |
| Number of UE receive chains | 2 for 2GHz |
| Periodicity | 20ms |
| Performance metric | Combination of 4 SSBs in 80ms. Note: UE is not assumed to know the SS/PBCH block index |
| Frequency offset | Frequency offset defined in table 6.1.2-1 of TR38.821 |
| Other parameters | Reported by companies. |

Proposal 10-8_v1

For coverage evaluation of PDSCH in NR NTN, the following table is assumed.

| Parameter | Value |
| --- | --- |
| BLER | For low data rate service, w/ HARQ, 10% iBLER; w/o HARQ, 10% iBLER. For VoIP, 2% rBLER. |
| Waveform | CP-OFDM |
| Number of UE receive chains | 2 for 2GHz |
| HARQ configuration | Whether/How HARQ is adopted is reported by companies. |
| DMRS configuration | For 3km/h: Type I, 1 or 2 DMRS symbol, no multiplexing with data. PDSCH mapping Type, the number of DMRS symbols and DMRS position(s) are reported by companies. |
| PRBs/MCS/TBS | Reported by companies. |
| PDSCH duration | 12 OS |
| Frequency offset | Frequency offset defined in table 6.1.2-4 of TR38.821 |
| Other parameters | Reported by companies. |

Proposal 10-2: Email discussion

Proposal 10-3/4/5/6/7/8: They are dependent on outcome of proposal 9-1. Waiting for the conclusion.

  - [Closed] Topic #11: Others

    - 1st round

Q: Please input only if you think other aspect for this topic should be discussed. Note that further question / proposal will be prepared corresponding to texts with bule color above.

| Company | Comment |
| --- | --- |
| QC | Diversity techniques should be discussed. |
| Lenovo | Polarization density should be considered. |
| vivo | Considering NTN specific enhancement, diversity based on polarization could be considered. |
| Huawei, HiSilicon | 1. NTN channel model should be aligned for link level simulation with the channels defined in 38.811. 2. MCS table used should be aligned or clearly reported by companies for evaluation. (MCS index table which support lower code rate can be selected, e.g. Table 6.1.4.1-2 in 38.214 for PUSCH and Table 5.1.3.1-3 in 38.214 for PDSCH) 3. If real channel estimation is adopted, it would be better to align on the channel estimation method and interpolation method, e.g. LS channel estimation and linear interpolation. |
| LG | PRACH repetition techniques should be discussed. |
| NEC | We need to study coexistence of VSAT UEs and commercial UEs in the same cell specifically with respect to radio resource efficiency for different channels (e.g. PRACH). |
| ZTE | Polarization enhancement should considered. Larger UE transmission power should be considered. |
| CATT | PRACH repetition techniques should be discussed. |
| MediaTek | Diversity techniques should be discussed |
| Ligado | Diversity techniques should be discussed, especially in the UL due to link margin constraints. |
| Sony | Polarization and diversity techniques should be considered. |
| Lockheed | Polarization and 2x2 precoding should be considered. |
| Hughes/EchoStar | RAN1 should strongly consider MEO analysis to ensure the completion of all orbits for NGSO deployment. |
| QC | Diversity techniques should be discussed |

    - Summary of 1st round

It seems that inputs for both coverage evaluation and enhancement techniques are mixed. The 1st question would be not appropriate; Let me ask again with the modified question text below.

    - 2nd round

Q: Please input only if you think other aspect for study of coverage evaluation by the next meeting should be discussed. That is, please avoid comment on enhancement mechanism perspective. Note that further question / proposal will be prepared corresponding to texts with bule color above.

| Company | Comment |
| --- | --- |
| MediaTek | Diversity techniques should be prioritized as shown to provide significant gains by several companies. This topic is also relevant to Topic#5 |
| Lockheed | Agree with Mediatek, diversity techniques should be prioritized. |
| Lenovo | Agree with MTK. |
| Panasonic | It would be good to align the channel model (NTN TDL-C, D etc). NTN TDL-D (LOS) can be a good candidate. |
| LG | PRACH repetition techniques should be discussed. |
| Thales | Performance enhancement for feedback-disabled-HARQ process. Support of larger aggregation factor is considered to be beneficial for NTN also for PDSCH. To our understanding the [X] in the Meeting #106-e agreement copied below it is still FFS. Also it is FFS whether different configurations are considered for the transmission via feedback-enabled or feedback-disabled-HARQ process RAN1#106-e Agreement: The maximum number of supported aggregation factor (i.e., pdsch-AggregationFactor) for DL PDSCH is [X] - FFS: X = 8, 16 or 32 |
| Huawei, HiSilicon | 1. NTN channel model should be aligned for link level simulation with the channels defined in 38.811. 2. MCS table used should be aligned or clearly reported by companies for evaluation. (MCS index table which support lower code rate can be selected, e.g. Table 6.1.4.1-2 in 38.214 for PUSCH and Table 5.1.3.1-3 in 38.214 for PDSCH) 3. If real channel estimation is adopted, it would be better to align on the channel estimation method and interpolation method, e.g. LS channel estimation and linear interpolation. 4. the evaluation scenario, e.g. sub-urban or rural etc. should be also aligned. |
| Nokia, Nokia Shanghai Bell | The main difference between NTN NGSO and TN is the doppler shift/drift and there is no assumption in the evaluation related to this point that makes the evaluation not too specific for NTN case. For that reason it may be worth considering: - Residual frequency error in the coverage evaluation after pre/post compensation using ephemeris, 0.2 ppm or higher could be used as RAN4 current assumption and discussion. - Agree on UE speed or doppler spread mainly. |
| Omnispace | Diversity techniques should be prioritized. |
| Ligado | Agree with MediaTek, Omnispace and others that diversity should be prioritized. |
| Hughes/EchoStar | RAN1 should strongly consider adding MEO analysis to ensure the completion of all orbits for NGSO deployment. |
| QC | Diversity techniques should be prioritized |

    - Summary of 2nd round

Firstly FL would like to emphasize that FL did not ask enhancement technique, which can be discussed in later meeting if conclusion is that enhancement is necessary.

A lot of companies think diversity aspect should be considered. This is reflected in proposal 5-2.

Regarding channel model/channel estimation/evaluation scenario/UE speed/doppler spread, although FL’s intention was to discuss these aspects after concluding section 9, exactly common aspects among channels can be discussed separately. Considering time limitation in this meeting, FL prepares corresponding questions under section 4.10.

For MCS/PRB and frequency offset, FL thinks that it should be discussed after concluding target data rate and section 4.10.

For MEO, it was discussed in section 4 (see first round), and FL cannot find strong support. FL recommends to focus on GEO/LEO or try to get full/most supports of MEO.

    - 3rd round

Q: Please input only if you think other aspect for study of coverage evaluation by the next meeting should be discussed.

| Company |  |  |
| --- | --- | --- |
| Lockheed | Yes | We believe MEO should be considered. |
| Hughes/EchoStar | Yes | MEO should be included in the discussion. Section 4.1 of the work item RP-220953 aims at specifying enhancements for NG-RAN based NTN (non-terrestrial networks) according to the following assumptions, to include: GSO and NGSO (LEO and MEO). MEO is one of the important orbits in NGSO constellations, its altitude will be a crucial aspect to NTN coverage enhancement. MEO characteristics need to be included in the evaluation to verify the validity of specifications. Its characteristics described in RP-220590 and endorsed by RAN#95-e to be used for future reference. |
| MediaTek | Yes | MEO should be included in study of coverage evaluation |
| Novamint | Yes | We believe MEO should be included in the study of coverage evaluation |
| Intelsat | Yes | Yes, MEO should be assessed an additional aspect for evaluation. |
| ESA | Yes | MEO should be included |
| LG | Yes | If PRACH is considered as an evaluation target in the discussion 4.9, PRACH repetition techniques should be discussed as an LLS assumption. . |
| GateHouse | Yes | We believe MEO should be considered in the study of coverage evaluation |
| Thales | Yes | Include MEO in the study of coverage evaluation |

    - Summary of 3rd round

MEO aspect is already covered in different sections. Regarding LGE’s comment, FL’s understanding is that PRACH repletion is new feature and to be discussed in R18 CovEnh. There would be no need to discuss now.

    - 4th round

Now full set of proposals are prepared from FL side.

Q: Please input only if you think other aspect for study of coverage evaluation by the next meeting should be discussed.

| Company | Comment |
| --- | --- |

    - Summary of 4th round

No input is found in 4th round. This section is now closed.

- Contact information

FL requests companies to input contact information in this section. The box is prepared based on chair. This meeting is the first one for Rel-18; it would be helpful for companies to know who join this topic, e.g. in order to have offline discussion.

| Company | Name | Email |
| --- | --- | --- |
| FL (NTT DOCOMO) | Shohei Yoshioka | shohei.yoshioka@docomo-lab.com |
| Lenovo | Hongmei Liu | Liuhm6@lenovo.com |
| Apple | Chunxuan Ye | Chunxuan_ye@apple.com |
| Apple | Chunhai Yao | Chunhai_yao@apple.com |
| Xiaomi | Min Liu | Liumin10@xiaomi.com |
| Xiaomi | Yajun Zhu | zhuyajun@xiaomi.com |
| vivo | Zhipeng Lin | zhipeng.lin@vivo.com |
| vivo | Yong Wang | wy.wang.5g@vivo.com |
| Nokia | Frank Frederiksen | Frank.frederiksen@nokia.com |
| OPPO | Hao LIN | lin.hao@oppo.com |
| OPPO | Zuomin WU | wuzuomin@oppo.com |
| OPPO | Nande Zhao | zhaonande@oppo.com |
| Huawei, HiSilicon | Xiaolei TIE | tiexiaiolei@huawei.com |
| Huawei, HiSilicon | Ying Chen | chenying18@huawei.com |
| Huawei, HiSilicon | Xinghua Song | songxinghua@huawei.com |
| ZTE | Fangyu Cui | cui.fangyu@zte.com.cn |
| CATT | Deshan Miao | miaodeshan@catt.cn |
| Ericsson | Stefan Eriksson Löwenmark | stefan.g.eriksson@ericsson.com |
| Thales | Mohamed EL JAAFARI | mohamed.el-jaafari@thalesaleniaspace.com |
| Spreadtrum | Zhenzhu Lei | reven.lei@unisoc.com |
| MediaTek | Gilles Charbit | Gilles.charbit@mediatek.com |
| InterDigital | Moon-il Lee | Moonil.lee@interdigital.com |
| Sony | Samuel Atungsiri | Sam.Atungsiri@sony.com |
| Lockheed | Robert Olesen | robert.l.olesen@lmco.com |
| ETRI | Dukhyun You | dhyou@etri.re.kr |
| Panasonic | Akihiko Nishio | nishio.akihiko@jp.panasonic.com |
| Samsung | Sungjin Park | sj100.park@samsung.com |
| Omnispace | Ron Olexa | rolexa@omnispace.com |
| NEC | Pravjyot Singh Deogun | pravjyot.deogun@emea.nec.com |
| Ligado | Clive Packer | clive@ligado.com |
| Hughes/EchoStar | Munira Jaffar | Munira.Jaffar@EchoStar.com; munirajaffar@hughes.com |
| Qualcomm | Xiao Feng Wang | wangxiao@qti.qualcomm.com |
| Qualcomm | LiangPing Ma | lpma@qti.qualcomm.com |
| Novamint | Thierry Bérisot | tberisot@novamint.com |
| GateHouse | Robert van der Pool | rvp@gatehouse.com |
| FGI | YenHua Li | danielli@fginnov.com |

- Contribution summary

  - Target data rate

- VoIP

  - 4.75 kbps: [5/vivo] [22/Ericsson]

    - [5/vivo]: The voice data rates supported for variable AMR codec mode [5] are listed in table 2. Normally AMR is used in voice communication, and to have best link budget, it would be preferred to select the lowest AMR data rate 4.75kbps.

  - 9.1 kbps: [1/HW, HiSi]

    - [1/HW, HiSi]: For the throughput requirement of VoIP and low-data rate services for commercial handset terminals, the codec for Enhanced Voice Services (EVS), successor of the voice codec AMR-WB, standardized by the 3GPP SA4 in Release 12, should be chosen as the starting point.

  - 17.2 kbps: [20/THALES]

    - [20/THALES]: we propose to consider the Enhanced Voice Services (EVS) codec which is the current recommendation for voice over NR: The EVS 13.2 kbps can be used in this study which corresponds to data rate of 17.2 kbps at layer 1 or a packet size of 344 bits every 20ms as depicted in  Figure 1.

  - Reuse ‘packet size for VoIP’ in Table A.1-1 of TR38.830 (i.e. 320 bits with 20 ms data arriving interval) : [6/Pana] [9/Xiaomi] [22/Ericsson]

  - Discuss latency requirement: [11/Samsung]

    - [11/Samsung] Since NR NTN has long RTD (Round Trip Delay) channel characteristics, it should be discussed what latency requirement for VoIP is considered.

- Low data rate

  - DL 1 Mbps: [9/Xiaomi] [20/THALES]

  - DL 100 kbps: [15/Apple]

  - UL 100 kbps: [9/Xiaomi] [20/THALES]

  - UL 50 kbps: [15/Apple]

  - Study: [1/HW, HiSi] [5/vivo] [11/Samsung] [22/Ericsson]

    - [1/HW, HiSi] for the GEO Nadir scenarios, this 100 kbps UL throughput could not be met under GEO nadir scenario based on our simulation results presented in Section 2.3.

    - [5/vivo] The data rate 100kbps assumed in Rel-17 coverage enhancement topic is too high for NTN scenario.

    - [11/Samsung] without having exact parameters such as packet size and latency for low-data rate service, it is difficult to check whether further coverage enhancement is necessary or not.

  - Evaluation methodology / Performance metrics

- Rel-16 NTN-like: [1/HW, HiSi] [5/vivo] [6/Pana] [9/Xiaomi] [17/DCM] [20/THALES] [22/Ericsson]

  - Link level: [1/HW, HiSi] [5/vivo] [6/Pana] [9/Xiaomi] [17/DCM] [20/THALES]

    - [1/HW, HiSi] [5/vivo] [6/Pana] [9/Xiaomi] [17/DCM] [20/THALES]

      - Link budget calculation for the CNR

      - LLS for required SINR of the target service

      - Compare the CNR and required SINR

  - Link level + System level: [20/THALES]

    - [20/THALES] Obtain the required SINR for the given target data rate based on link-level simulation.  Obtain the target performance based on system-level simulation (i.e. the 5th percentile downlink or uplink SINR value in CDF curve).

- Rel-17 CovEnh-like: [11/Samsung] [15/Apple] [19/LGE]

  - Revised MPL: [11/Samsung]

    - [11/Samsung] In [2], the concept of MPL (Maximum Path Loss) is used to derive coverage performance based on certain path loss model. Similar concept can be reused with simplifying some assumptions because main assumptions/parameters/approaches for [2] and [3] are quite different.

  - MIL: [15/Apple] [19/LGE]

    - [15/Apple] In the study of Release 17 NR coverage enhancement, three different metrics are used to identify the bottleneck physical layer channels: MCL (maximum coupling loss), MIL (maximum isotropic loss), MPL (maximum path loss). Since MCL only includes part of the gNB antenna gain (i.e., the number of TxRU over the number of RF chains), it does not fit with satellite EIRP density. On the other hand, the MPL includes pathloss and base station selection gain. This is unnecessary for NR NTN. Hence, we think only MIL is used to identify the bottleneck physical layer channel.

    - [19/LGE] In Rel-17 CE WI, three of metrics had been used; MIL, MCL and MPL. Each metric are defined in TR 38.830. MCL and MIL can be used to identify the coverage bottleneck among candidate physical radio channels. Some of candidate physical radio channels can be exempted from coverage bottleneck, based on a value of MPL, which can define deployment dependent target in given scenarios. Therefore, it is proposed to re-use this method for coverage bottleneck identification in NTN enhancement. Considering objectives of NR NTN enhancement, MIL can be considered mainly for reflecting impact of different UE antenna gain and polarization loss.

  - Link budget calculation

- Reuse 6.3.1 of TR38.821 as baseline: [22/Ericsson]

- Reuse Table 6.1.3.2-1 of TR38.821: [8/CATT]

  - Note: Only parameters involved with agreed satellite orbit and band

- Shadowing

  - 3 dB: [6/Pana] [15/Apple]

- Atmospheric path loss

  - 0.07 dB for LEO-1200 [6/Pana]

  - 0.2 dB for GEO, 0.1 dB for LEO-1200 and LEO-600: [15/Apple]

- Scintillation loss

  - 2.2 dB: [6/Pana] [15/Apple]

- Polarization loss

  - Include 3 dB polarization loss in the link budget analysis and assume polarization loss is fully compensated in LLS: [1/HW, HiSi]

    - [1/HW, HiSi]: The polarization loss could be handled by the receiver by implementation, e.g. by using coherent combination to composite the polarization loss, as shown in Figure 6 and Figure 7 in Appendix. Regarding how to capture the impact of polarization loss in the evaluation, in our evaluation, no polarization compensation loss will be assumed in the performance of LLS (e.g. BLER), and correspondingly the polarization loss is counted in the CNR of link budget.

  - 3 dB polarization loss should be considered: [6/Pana] [9/Xiaomi] [11/Samsung] [13/OPPO] [15/Apple] [16/CMCC]

    - [6/Pana]: Satellite with circular polarization and UE with linear polarization should be assumed as a typical scenario

    - [9/Xiaomi] The polarization loss need to be considered for link budget calculation assuming UE is linear polarization while the satellite is circular polarization. Based on TR 38.821, for downlink transmission, if the UE is dual-polarized with both horizontal and vertical polarization, a combination of the two Rx branches allows to prevent depolarization. While if the UE is single-polarization antenna, a 3dB depolarization loss need to be considered. For uplink transmission, a 3dB depolarization loss should be taken into account.

    - [11/Samsung] it should be considered for realistic NTN scenario although it hadn’t been considered in [2].

- Half power beam width

  - 3dB additional loss due to beamwidth defined by HPBW at the beam edge: [13/OPPO]

- Others

  - Rain and cloud attenuation: [16/CMCC]

  - Building entry loss, if indoor deployment is considered: [16/CMCC]

  - Carriage and container penetration loss for logistics application: [16/CMCC]

  - Vegetation loss for outdoor application: [16/CMCC]

  - Satellite orbit

- GEO: [1/HW, HiSi] [13/OPPO] [15/Apple] [17/DCM] [20/THALES]

  - [8/CATT] There are bad CNR results in the scenarios of GEO, especially set-2 scenarios, and the uplink link budget is worse than downlink link budget. Deprioritize GEO scenarios for NTN handset case.

  - [19/LGE] In terms of deployment scenarios, LEO-600/1200 and GEO can be considered basically. Considering that VoIP and low-data rate services for commercial handset terminals, the LEO can be considered as a baseline.

- LEO-1200: [1/HW, HiSi] [6/Pana] [8/CATT] [13/OPPO] [15/Apple] [17/DCM] [19/LGE] [20/THALES]

  - [6/Pana] In order to timely finalization of WID, target scenarios for the coverage evaluation should be limited. LEO would be the main target for smartphones while GEO would not be main target although the solution can be applied to GEO as well. We propose to limit the scenarios for the evaluation to LEO 1200 km because LEO 1200 km is more severe in terms of link budget than LEO 600 km and the evaluation result of LEO 1200 km could be applied to LEO 600 km. The main target frequency band should be S-band (2GHz) because target usage is VoIP and low data rate service i.e. narrow bands and lower frequency would be more suitable.

- LEO-600: [1/HW, HiSi] [8/CATT] [13/OPPO] [15/Apple] [17/DCM] [19/LGE] [20/THALES]

- MEO: [22/Ericsson]

  - UE characteristics

- Handset: [1/HW, HiSi] [6/Pana] [8/CATT] [9/Xiaomi] [13/OPPO] [15/Apple] [16/CMCC] [19/LGE] [20/THALES]

- VSAT: [22/Ericsson]

- Reuse Table 6.1.1.1-3 of TR38.821: [1/HW, HiSi] [6/Pana (?)] [20/THALES]

  - Note: Only parameters involved with agreed band and UE type

  - Note: discussion on Tx/Rx antenna gain is captured separately

  - Antenna gain

- -3 dBi: [8/CATT]

  - [8/CATT] due to the shell cover and body absorption, especially the metal shell, the signal will be attenuated; thus, antenna gain is possibly less than 0dBi. Now the metal shell of handsets will be individually designed with plastic dividing lines to ensure the signal emission, so we propose that -3dBi antenna gain in the LLS parameters.

- -5 dBi: [5/vivo] [6/Pana] [17/DCM] [19/LGE] [20/THALES] [22/Ericsson]

  - [5/vivo] UE TX antenna gain for commercial smartphones may be less than 0dBi due to conductivity loss after a signal is transmitted from a UE, and an antenna gain varying from -3dBi to -5dBi may be more realistic. So -5dBi, instead of 0dBi, should be selected.

  - [22/Ericsson] RAN1 needs input from RAN4 on realistic assumptions on antenna gains for commercial smartphones. While waiting for RAN4 input, -5 dBi can be assumed.

- -6 dBi: [9/Xiaomi]

- Discuss: [11/Samsung]

  - Band / Bandwidth

- Band

  - S band: [1/HW, HiSi] [6/Pana] [8/CATT] [9/Xiaomi] [13/OPPO] [15/Apple] [20/THALES]

- Bandwidth

  - 30 MHz for DL: [1/HW, HiSi] [6/Pana]

  - 360 kHz for UL: [1/HW, HiSi] [6/Pana]

  - 20 MHz: [15/Apple]

  - Parameter set

- Reuse Table 6.1.1.1-1 of TR38.821 (i.e. set-1): [8/CATT] [15/Apple] [20/THALES]

  - With the following updates: [1/HW, HiSi]

    - Removal of equivalent satellite antenna aperture/3dB beamwidth/satellite beam diameter [1/HW, HiSi]

  - Note: Only parameters involved with agreed satellite orbit and band

- Reuse Table 6.1.1.1-2 of TR38.821 (i.e. set-2): [8/CATT] [15/Apple] [20/THALES]

  - Note: Only parameters involved with agreed satellite orbit and band

- Power flux density

  - Discuss and align the view on whether the ITU regulatory limitations on power flux density is assumed for the DL NTN coverage enhancement study.: [1/HW, HiSi] [2/ZTE] [22/Ericsson]

    - [1/HW, HiSi]: However, the ITU regulation of limiting power flux density leads to a lower link budget [7], which might be lower than the PDSCH required decoding SNR. This needs to be discussed and make a conclusion whether the limitation of power flux density is assumed in NTN coverage study.

    - [2/ZTE] As can be found in Figure 5, the calculated DL power flux density exceeds the ITU regulation restriction with the maximum gap of 32dB. For the case of Set-1 LEO-600km satellite, the DL CNR will be severely reduced with PFD gap of 32 dB if corresponding DL EIRP complied with PFD limit. Thus, the DL coverage performance will be severely restricted when the PFD limit is followed in NTN scenarios. Based on the DL synchronization performance in [4], the DL coverage performance is provided in Table 4. When the downlink PFD limitation is not considered in NR over NTN, the detection of PSS can be successful for Set-1 and Set-3 satellite parameters even if the antenna gain of -5dBi in smart phones is assumed. However, the PSS detection cannot be guaranteed since the huge coverage gap is introduced if PFD limitation is followed. What’s more, all DL physical channels will be seriously affected. In order to resolve such huge coverage gap, potential enhancement on NTN DL coverage should be further studied. For example, planning frequency bans on S-band for NTN to avoid interfering with terrestrial services, and only follow PFD limit in specific services or geographical areas, etc.

  - the satellite EIRP density is adjusted to satisfy ITU regulation of PFD limitation: [15/Apple]

    - [15/Apple] With the consideration of ITU regulation of PFD limitation, the satellite EIRP density is adjusted to 44 dBW/MHz, 20 dBW/MHz and 14 dBW/MHz in GEO, LEO-1200, LEO-600, respectively. In case of LEO-600 at elevation angle of 90 degrees, the satellite EIRP density is only 14 dBW/MHz. In other words, to meet the power flux density limitation, the satellite EIRP density has to be reduced from 34 dBW/MHz in set 1 satellite parameters to 14 dBW/MHz. This power flux density limitation will result in a reduction of 20 dB on satellite EIRP. Similarly, in case of LEO-1200 or GEO at elevation angle of 90 degrees, the satellite EIRP density is only 20 dBW/MHz or 44 dBW/MHz.

  - Study cases

- For GEO

  - Set 1, 90/2.3 deg, S-band: [1/HW, HiSi]

  - Set 1, 12.5 deg, S-band: [13/OPPO] [20/THALES]

  - Set 1, S-band: [9/Xiaomi]

  - Set 2, 90/11 deg, S-band: [1/HW, HiSi]

  - Set 2, 20 deg, S-band: [13/OPPO]

  - Set 2, S-band: [9/Xiaomi]

  - Not consider Set 2: [2/ZTE]

- For LEO-1200

  - Set 1, 90/26.3 deg, S-band: [1/HW, HiSi]

  - Set 1, 30 deg, S-band: [6/Pana] [13/OPPO]

  - Set 1, S-band: [9/Xiaomi]

  - Set 2, 90/22.2 deg, S-band: [1/HW, HiSi]

  - Set 2, 30 deg, S-band: [6/Pana] [13/OPPO] [20/THALES]

  - Set 2, S-band: [9/Xiaomi]

- For LEO-600

  - Set 1, 90/27 deg, S-band: [1/HW, HiSi]

  - Set 1, 30 deg, S-band: [13/OPPO]

  - Set 1, S-band: [9/Xiaomi]

  - Set 2, 90/23.8 deg, S-band: [1/HW, HiSi]

  - Set 2, 30 deg, S-band: [13/OPPO] [20/THALES]

  - Set 2, S-band: [9/Xiaomi]

- Appropriate setting of minimum elevation angle, e.g., more than 30 degree for all cases: [2/ZTE]

- A wide range of elevation angles varying from 10 degrees to 90 degrees: [5/vivo]

- Option 1/2 of frequency/polarization reuse: [13/OPPO]

- Option 1 of frequency/polarization reuse: [6/Pana]

- VoIP is discussed only for LEO: [5/vivo]

  - [5/vivo]: Assuming transparent payload satellite is used, if both the transmitter and the receiver are in the NTN network, the round-trip time would be considered as the propagation delay to calculate the allowed transmission time, as is shown in table 1. VoIP in GEO and MEO cannot be supported due to long propagation delay, and VoIP can only be supported in LEO scenario. VoIP in LEO can use 20 repetitions transmission within 20ms interval, subject to numerology

- Note: ‘X deg’ means elevation angle

  - Evaluation target channel/signal

- PUSCH: [11/Samsung]

  - [11/Samsung] Although NTN channel environment might be different to TN, the study of NR NTN coverage performance should start from the PUSCH which is well-known to be the bottleneck channel for FR1 as concluded in [2]. Besides, since main use cases for NR NTN enhancement are VoIP and low-data rate service for commercial handset terminals (a.k.a. smartphone), focusing on PUSCH for NR NTN coverage evaluation is reasonable.

- PDCCH, PDSCH, Msg 2 PDSCH, Msg 3 PUSCH, Msg 4 PDSCH and Msg 4 HARQ-ACK: [15/Apple]

  - [15/Apple] In Release 17 study of NR coverage enhancement, downlink channels show good coverage performance. Hence, the work item of Release 17 NR coverage enhancement focuses only on uplink channels, i.e., PUSCH, PUCCH and Msg 3 PUSCH. However, in the context of NTN, with the consideration of ITU limitation on PFD, the downlink channels may need to be examined. Hence, we propose to examine the downlink physical channels like PDCCH and PDSCH. Also, the message transmission in RACH procedure involves the wider beam, which may suffer from the power degradation. Hence, Msg 2 PDSCH, Msg 3 PUSCH, Msg 4 PDSCH and HARQ-ACK for Msg 4 need to be examined for coverage.

- The list of physical channels in TR 38.830: [17/DCM]

  - i.e. PUSCH for eMBB (FDD), PUSCH for VoIP (FDD), PUCCH Format 1 with 2bits, PUCCH Format 3 with 11bits, PUCCH Format 3 with 22bits, SSB, PRACH format 0, PRACH format B4, Broadcast PDCCH (PDCCH of Msg.2), PDSCH for Msg.2, PUSCH of Msg.3, PDSCH of Msg.4, Unicast PDCCH, PDSCH for eMBB (FDD), PUCCH with HARQ-ACK for Msg.4, PUSCH with SIP invite, PUSCH for CSI 11bit, PUSCH for CSI 22bit

  - [17/DCM] Considering the difference in transmit power between UE and satellite, uplink channels would be a potential bottleneck. Uplink channels should be prioritized.

- PUSCH/PRACH: [19/LGE]

  - [19/LGE] For target channel of coverage enhancement, we can refer to evaluation result of Rel-17 CE. Though the evaluation was conducted with different assumption, PUSCH and PRACH were identified as bottleneck channels that have coverage issues, in most of scenarios. In other words, which scenario is assumed may have no significant impacts for identifying bottleneck channel.

  - Link level assumptions

- For S-SSB

  - Reuse the following columns from Table 6.1.2-1 of TR38.821

    - Carrier frequency: [1/HW, HiSi] [20/THALES]

    - Channel model: [20/THALES]

    - SCS: [1/HW, HiSi] [20/THALES]

    - Antenna configuration at the TRP (satellite): [1/HW, HiSi] [20/THALES]

    - Antenna configuration at the UE: [1/HW, HiSi] [20/THALES]

    - UE speed: [20/THALES]

    - UE elevation angle: [20/THALES]

    - Frequency offset:

    - Frequency drift:

    - Phase noise model: [20/THALES]

    - Metrics: [20/THALES]

    - Note: Only parameters involved with agreed band

  - Reuse the following columns from Table A.1-8 of TR38.830

    - Periodicity: [20/THALES]

    - Number of UE receive chains: [6/Pana]

    - SCS: [6/Pana]

    - Performance metric: [6/Pana]

    - Other parameters: [6/Pana]

- For PRACH

  - Reuse the following columns from Table 6.1.2-2 of TR38.821

    - Configurations: [1/HW, HiSi] [20/THALES]

    - Carrier frequency: [1/HW, HiSi] [20/THALES]

    - Channel model: [20/THALES]

    - Antenna configuration at the TRP (satellite): [1/HW, HiSi (1 RX)] [20/THALES (1 RX)]

    - Antenna configuration at the UE: [20/THALES]

    - Frequency offset:

    - UE speed: [20/THALES]

    - Timing offset:

    - Phase noise model:

    - PRACH design:

    - Metric:

    - Receiver:

    - Note: Only parameters involved with agreed band

  - Reuse the following columns from Table A.1-4 of TR38.830

    - Format: [6/Pana]

    - SCS: [6/Pana]

    - Performance metric: [1/HW, HiSi (1st one)] [6/Pana] [20/THALES (1st one)]

    - Number of UE transmit chains: [6/Pana]

    - Other parameters: [6/Pana]

  - PRACH format

    - Format 2: [20/THALES]

    - Format B4: [20/THALES]

  - UE antenna configuration

    - (1, 1, 2) with omni-directional antenna element: [1/HW, HiSi]

- For PUCCH/PUSCH

  - Reuse the following columns from Table 6.1.2-4 of TR38.821

    - Parameters: [1/HW, HiSi] [20/THALES]

    - Carrier frequency: [1/HW, HiSi] [20/THALES]

    - Channel coding: [20/THALES]

    - SCS: [1/HW, HiSi] [20/THALES]

    - Channel estimation:

    - Frequency offset: [20/THALES]

    - Frequency drift: [20/THALES]

    - Frequency tracking: [20/THALES (Option 1)]

    - UE speed: [20/THALES]

    - Channel model: [20/THALES]

    - Satellite antenna configuration: [1/HW, HiSi] [20/THALES]

    - UE antenna configuration: [1/HW, HiSi] [20/THALES]

    - Phase noise model: [20/THALES]

    - Metrics:

    - Note: Only parameters involved with agreed band

- For PUSCH

  - Reuse the following columns from Table A.1-2 of TR38.830

    - Frequency hopping: [6/Pana]

    - Performance metric (BLER): [1/HW, HiSi] [6/Pana (For VoIP)] [20/THALES]

    - Number of UE transmit chains: [6/Pana] [20/THALES (only 1)]

    - DMRS configuration: [6/Pana]

    - Waveform: [6/Pana] [20/THALES (DFT-s-OFDM)]

    - PUSCH duration: [20/THALES]

    - Repetitions: [6/Pana (For VoIP)]

    - HARQ configuration: [6/Pana (For VoIP)]

    - PRBs/TBS/MCS for eMBB: [20/THALES (except for ‘any value of ...’)]

    - PRBs/MCS for VoIP: [6/Pana] [20/THALES (except for ‘Other values of ...’)]

  - 30 kHz SCS: [6/Pana]

  - w/ repetition w/ max repetition factor of

    - 32: [13/OPPO] [20/THALES]

    - 20: [2/ZTE]

  - w/ and w/o HARQ: [20/THALES]

  - w/ and w/o joint channel estimation (i.e. DMRS bundling): [2/ZTE] [6/Pana] [17/DCM]

  - w/ and w/o TBoMS: [6/Pana] [17/DCM] [19/LGE]

- For PUCCH

  - Reuse the following columns from Table A.1-3 of TR38.830

    - PUCCH format: [6/Pana] [20/THALES (except for 4 bits of PF3)]

    - FH: [6/Pana]

    - Performance metric (BLER): [1/HW, HiSi] [6/Pana] [20/THALES]

    - Number of UE transmit chains: [6/Pana] [20/THALES]

    - DMRS configuration: [6/Pana]

    - Repetitions: [6/Pana]

    - PUCCH duration: [6/Pana] [20/THALES]

    - Number of PRBs: [6/Pana] [20/THALES]

  - w/ repetition w/ max repetition factor of

    - 8: [13/OPPO] [20/THALES]

  - 30 kHz SCS: [6/Pana]

- For Msg.3 PUSCH

  - Reuse the following columns from Table A.1-5 of TR38.830

    - FH: [6/Pana]

    - Number of UE transmit chains: [6/Pana] [20/THALES (only 1)]

    - Number of DMRS symbol: [6/Pana] [20/THALES (only 3)]

    - Waveform: [6/Pana] [20/THALES]

    - SCS: [6/Pana] [20/THALES (FDD)]

    - HARQ configuration: [6/Pana (For VoIP)]

    - PUSCH duration: [6/Pana] [20/THALES]

    - Number of PRBs: [6/Pana] [20/THALES]

    - TBS: [6/Pana] [20/THALES]

    - Other parameters: [6/Pana]

  - w/o HARQ: [20/THALES]

  - w/ repetition w/ max repetition factor of

    - 16: [13/OPPO] [19/LGE (?)]

- For PDCCH/PDSCH

  - Reuse the following columns from Table 6.1.2-4 of TR38.821

    - Parameters: [1/HW, HiSi] [20/THALES]

    - Carrier frequency: [1/HW, HiSi] [20/THALES]

    - Channel coding: [1/HW, HiSi] [20/THALES]

    - SCS: [1/HW, HiSi] [20/THALES]

    - Channel estimation: [20/THALES]

    - Frequency offset: [20/THALES]

    - Frequency drift: [20/THALES]

    - Frequency tracking: [20/THALES (Option 1)]

    - UE speed: [20/THALES]

    - Channel model: [20/THALES]

    - Satellite antenna configuration: [1/HW, HiSi] [20/THALES]

    - UE antenna configuration: [1/HW, HiSi] [20/THALES]

    - Phase noise model: [20/THALES]

    - Metrics:

    - Note: Only parameters involved with agreed band

- For PDSCH

  - Reuse the following columns from Table A.1-6 of TR38.830

    - Performance metric (BLER): [1/HW, HiSi] [6/Pana (For VoIP)] [20/THALES]

    - Waveform: [6/Pana] [20/THALES]

    - Number of UE receive chains: [6/Pana]

    - SCS: [6/Pana]

    - HARQ configuration: [6/Pana (For VoIP)]

    - DMRS configuration: [6/Pana] [20/THALES (Msg.2, For 3km/h)]

    - PRBs/MCS/TBS: [6/Pana]

    - PDSCH duration: [6/Pana] [20/THALES]

    - Payload size for PDSCH of Msg.4: [6/Pana] [20/THALES]

    - Other parameters: [6/Pana]

  - Number of UE receive chains = 1: [20/THALES]

  - w/ and w/o HARQ: [20/THALES]

- For PDCCH

  - Reuse the following columns from Table A.1-7 of TR38.830 with the following updates

    - Number of UE receive chains: [6/Pana]

    - SCS: [6/Pana]

    - Aggregation level: [6/Pana] [20/THALES]

    - Payload: [6/Pana] [20/THALES]

    - CORESET size: [6/Pana] [20/THALES]

    - TX Diversity: [6/Pana]

    - Performance metric (BLER) : [1/HW, HiSi] [6/Pana] [20/THALES]

    - Number of SSB for broadcast PDCCH of Msg.2: [6/Pana]

    - Other parameters: [6/Pana]

  - Number of UE receive chains = 1: [20/THALES]

- 6.1.2 of TR38.821 as baseline: [22/Ericsson]

- Tables in Annex A of TR38.830 are baseline: [9/Xiaomi] [15/Apple]

  - [9/Xiaomi] Companies are encouraged to provide the detailed parameters if different channel-specific parameters assumption are assumed in evaluation.

- For all

  - Channel model

    - NTN-TDL-C: [15/Apple]

    - 100 ns delay spread: [15/Apple]

    - [22/Ericsson]: The performance with the NTN TDL-D channel model can be very sensitive to the choice of elevation angle. It can be seen that at 2% BLER, there is a 6 dB difference between 10° and 30° elevation angle, and almost 11 dB difference between 10° and 90°. A possible reason for this is that the channel model has a significantly more dominant LoS component at very low elevation angles.

  - UE velocity

    - 3 km/h: [15/Apple]

  - Applicability of Rel-17 CovEnh features

- PUSCH repetition enh

  - [10/Nokia, NSB] Application of an increased number of PUSCH repetitions to NTN is straightforward and does not require specific adaptations. Counting of PUSCH repetitions based on available slots is currently not relevant for NTN. Do not study further methods for application of increased number of PUSCH repetitions and available slot counting to NTN.

- Joint channel estimation (DMRS bundling)

  - [10/Nokia, NSB] Application of DMRS bundling to NTN is not straightforward and methods to enable utilization of such feature in NTN should be further investigated. Study further methods for application of DMRS bundling to NTN.

  - [18/Lenovo] There is no impact of large propagation delay to joint channel estimation window. A TA update can be considered as an event to terminate the joint channel estimation window. Application of updated K-offset is only at the start of the joint channel estimation window.

    - Regarding the impact of satellite movement, there may be TA update within the window. As pre R17 agreement, TA update should be considered to be an event which terminates the joint channel estimation window. So in R18, when there is a TA updated due to satellite movement, it should also be considered as an event to terminate the window.

  - [19/LGE] It is necessary to discuss whether to support the DMRS bundling in Rel-18 NTN for joint channel estimation. In order to perform the joint channel estimation at the network, the UE should transmit the UL signal/channel to a fixed UL TA value (i.e., Total TA including the UE specific TA and common TA). But, in NTN system, especially for LEO case, since UL TA value depends heavily on the movement of the satellite, the UE behavior of maintaining the UL TA during UL transmission may be undesirable. It is necessary to discuss how to support changing open loop TA (e.g., UE specific TA and/or common TA) during repeated transmission of UL signal/channel in Rel-18 NTN.

- TBoMS

  - [10/Nokia, NSB] TBoMS does not seem to require specific adaptations for NTN. Do not study further methods for application of TBoMS to NTN.

- PUCCH repetition enh

  - [10/Nokia, NSB] The dynamic indication of the repetition factor for scheduled PUCCH does not seem to require specific adaptations for NTN. Study methods for dynamic indication of the repetition factor for unscheduled PUCCH.

- Msg3 PUSCH repetition

  - [10/Nokia, NSB] The basic framework of Msg3 repetitions is directly applicable for NR over NTN. RAN1 to further investigate if NTN specific enhancements are needed for Msg3 repetitions.

  - Simulation results / Observations / Potential techniques

- Simulation results

  - [1/HW, HiSi]

    - For UL, based on the results in Figure 3 and Figure 4, the existing enhancement schemes provided by Rel-17 specification can provide voice services for commercial smartphones with -5 dBi antenna gain assumption for UL LEO600-SET1-edge scenario if the polarization loss could be compensated. However, there is a large gap (~5 dB) between the CNR from link budget analysis and required SNR of PUSCH even assuming 0 dBi antenna gain and no polarization loss reception for UL GEO-SET1-edge scenario. For DL, based on the results of Figure 5, the required decoding SNR of PDSCH under DL GEO-SET1-edge (– 9.89 dB wrt. 10% iBLER) is 0.96 dB lower than the CNR in the link budget analysis for GT of -5 dBi with a throughput of 1072.8 Kbps, which fully meets the VoIP requirement.

    - The existing enhancement schemes provided by the Rel-17 specification can provide UL voice service for commercial smartphones under LEO 600 scenario including the edge UEs if satellite reception implementation compensates the polarization loss. The existing enhancement schemes provided by the Rel-17 specification cannot provide UL voice service for commercial smartphones under the GEO scenario even for nadir UEs with 0 dBi antenna gain. The required decoding SNR of PDSCH (-9.89 dB) is lower than the CNR of the link budget (-8.04) with much higher throughput than the typical EVS codec requirement to deliver VoIP traffic, even for GEO edge UEs.

  - [2/ZTE]

    - It can be found that the coverage gap is larger for LEO-1200 and GEO with the maximum coverage gap as 17 dB in case that 10 degree elevation angle in Set-2 GEO satellite is assumed. In contrast, the voice service can be satisfied when the elevation angle varies from 30 to 90 degree for Set-1 LEO-600km satellite.

    - The small antenna gain in smart phone will lead to huge coverage gap for NTN scenarios. For the study of UL coverage, identification on the target deployment scenarios and satellite parameters should be concluded firstly.

    - Moreover, as the results shown in Figure 1, it can be found that for LEO-1200 and LEO-600 scenarios, the coverage gap at different elevation angles changes dramatically since the distance between UE and satellite varies rapidly. However, in GEO scenarios, the coverage gap is basically the same at different elevation angles because of small CNR variation caused by high orbit altitude.

  - [3/Spreadtrum]

    - According to the link budget calculation formula and the antenna gain of the smart phone (i.e., -5dBm), we calculate the CNR of the uplink and downlink channels for smart phone in NTN. For DL channel, it can be observed that the worst CNR for the GEO/LEO-1200/LEO-600 satellites are given by -10.2 dB/-3.8 dB/-1.4dB. For GEO, the worst CNR values are lower than the SNR requirements for downlink physical channels. For UL channel, it can be observed that the worst CNR for the GEO/LEO-1200/LEO-600 satellites are given by -20.7 dB/-13.6 dB/-8.2dB. These CNR values are lower than the SNR requirements for uplink physical channels with the Rel-17 coverage enhancement.

    - For GEO, the worst CNR values are lower than the SNR requirements for downlink physical channels. For uplink physical channels, the worst CNR values of the GEO/LEO-1200/LEO-600 satellites are lower than the SNR requirements with the Rel-17 coverage enhancement.

  - [4/MTK]

    - There is only one very low UL CNR of -15.7 dB for Set-2 GEO, and UL CNR=-10.9 dB for Set-1 GEO in S band assuming handheld UE without TX/RX antenna gain loss. For UL: Set-1 Lowest SNR = -10.9 dB (Cases 4, 5, S band, GEO); second lowest SNR = -2.6 dB (Cases 14, 15, S band, LEO-1200); Set-2 Lowest SNR = -15.7 dB (Cases 19, 20, S band, GEO); second lowest SNR = -8.6 dB (Cases 29, 30, S band, LEO-1200); For DL: Set-1 Lowest SNR = 0.0 dB (Cases 4, 5, S band, GEO); second lowest SNR = 6.6 dB (Cases 14, 15, S band, LEO-600); Set-2 Lowest SNR = -5.2 dB (Cases 19, 20, S band, GEO); second lowest SNR =0.5 dB (Cases 21-23, S band, LEO-600);

    - With Rel-17 NR coverage enhancements with up to 32 repetitions on UL and 8 slot aggregations on DL, it is expected that most UL and DL link budget cases in TR 38.821 can be closed with some margins.

    - DL SNR of -10 dB and -12 dB in TDL-A (NLOS), and DL SNR of -12 dB and -13 dB in TDL-C (LOS) can be achieved with DL PDSCH slot aggregation factor 8 and 16 respectively at 10% BLER. Rel-15 DL aggregation factor 8 is sufficient to close the DL link budget in NR NTN in smart phone assuming at least 5 dB margin or larger for RX antenna gain loss in smart phone.

    - Precoding with 2Tx-2Rx with precoding can give 3 dB SNR gain compare to no precoding with 90% UTP achieved at SNR of -7 dB without repetitions. Rel-15 UL repetition of up to 32 is sufficient to close the UL link budget in NR NTN in smart phone assuming at least 5 dB margin or larger for RX antenna gain loss in smart phone.

  - [5/vivo]

    - For PUSCH with AMR 4.75kbps, TBS=184bits (120+64), lowest required SNR@1%BLER = -9.53dB. For Set-1, LEO-1200 elevation angle 10~30 degrees cannot reach the required SNR. For Set-2, LEO-600 elevation angle 10~30 degrees and LEO-1200 elevation angle 10~90 degrees cannot reach the required SNR. VoIP cannot work in GEO and MEO due to large coverage gap as well. Under the channel model of NTN TDL-D with delay spread 300ns, frequency hopping can provide around 2dB gain. Focus on the LEO satellite to support VoIP in NTN.

    - With 1 PRB and MCS0 corresponding to 0.56kbps data rate, target SNR derived is -15.32dB, a 4.4dB gain is required with some coverage enhancement techniques for Set-2 GEO satellite with high elevation angles. With 1 PRB and MCS 9 corresponding to 5.47kbps data rate, target SNR derived is -10.94dB, a 3.8dB gain is required with some coverage enhancement techniques for Set-1 GEO satellite with high elevation angles.

    - Little gain can be seen for AMR 4.75kbps voice packet transmission on PUSCH when up to 4 bytes of the MAC/RLC/PDCP headers are reduced. Send an LS to RAN2 to ask the maximum RAN protocol overhead that can be reduced for voice packet transmission in NR NTN with a reasonable complexity.

    - For VoIP, as is discussed in section 2.1, only up to 20ms is allowed, i.e. for 15kHz SCS, up to 20 repetitions are allowed for PUSCH transmission which can already support up to 32 repetitions even based on available slot introduced in NR Rel-17 coverage enhancement work item. There’s no need to increase the number of repetitions supported in NR Rel-17 for VoIP use case.

    - For low rate data, the larger the number of repetitions is, the lower the data rate will be, with a given number of PRBs allocated. Furthermore, the larger the number of PRBs are allocated, the worse the link budget derived would be as the power density of resource element will be decreased when the number of PRBs are increased. The number of repetitions allowed for low data rate transmission depends on the latency allowed in GEO scenario.

    - Circular polarization can be regard as another spatial domain on top of antenna ports. Circular polarization enhancement on DL Tx diversity could be further studied.

  - [8/CATT]

    - We pick up the Format A3 for performance evaluation. When the frequency offset is configured with 1/10 SCS, 1/6 SCS, 1/3SCS, 1/2SCS, the related results are shown in the figure 1. Based on the result of figure 1, the performance of PRACH gradually decreases as the frequency offset increases, and PRACH Format of length-139 sequence with large FO doesn’t meet uplink link budget. PRACH format of sequence-139 doesn’t match uplink link budget when there is large frequency offset of NTN scenario.

    - If NTN system works in low SINR range, the sequence length would be a dominated factor. For sake of performance robustness, the sequence-839 is desired to resist low SINR. Figure 5 shows the performance comparison of sequence-839 and sequence-139. The sequence-839 is configured as PRACH Format3 with 5kHz frequency offset, and its SCS is 5kHz.The sequence-139 applies PRACH Format A3 with 5kHz frequency offset, and its SCS is 30kHz. As can be seen from the simulation results, the PRACH performance cannot meet the GEO Set-1 and Set-2, and LEO1200 Set-2 scenarios. PRACH should be enhanced on GEO Set-1 and Set-2, and LEO1200 Set-2 scenarios. PRACH sequence with length-839 is better than the sequence with length-139 in low SNR case. PRACH sequence-839 is not suitable for the SNR conditions of GEO Set-1 and Set-2, and LEO1200 Set-2 scenarios. Suggest PRACH enhancement based on the sequence-839 used in NTN handset UE.

  - [9/Xiaomi]

    - The UL channel is more challenging than DL channel, the UL coverage is limited in all the scenarios while the DL coverage is limited in GEO Set2 scenario. The GEO scenario is more challenging than LEO scenario, the average UL gap for GEO is 20 dB for Set 1 and 25 dB for Set 2, while the average UL gap is 5dB for LEO-600km Set1 and 10dB for LEO-1200km Set1. For GEO Set 2, all the channels have the coverage issues.

  - [11/Samsung]

    - For LEO case, maximum achievable coverage of PUSCH for VoIP using legacy coverage enhancement schemes is 1686km for 2Rx and 4921km for 16Rx assuming that transmit antenna gain is 0dBi. For LEO case, maximum achievable coverage of PUSCH for VoIP using legacy coverage enhancement schemes is 949km for 2Rx and 2768km for 16Rx assuming that transmit antenna gain is -5dBi.

    - For GEO case, maximum achievable coverage of PUSCH for VoIP using legacy coverage enhancement schemes is 14193km for 2Rx and 41407km for 16Rx assuming that transmit antenna gain is 0dBi. For GEO case, maximum achievable coverage of PUSCH for VoIP using legacy coverage enhancement schemes is 7981km for 2Rx and 23285km for 16Rx assuming that transmit antenna gain is -5dBi.

  - [12/NEC]

    - Commercial UEs UL transmissions are expected to experience much lower CINR values as compared to UL transmission from VSAT UEs with higher Tx EIRP values

    - But for initial random-access procedure, coverage enhancement solutions (e.g. number of RACH repetitions) can only be configured in cell specific manner. Note that for RACH coverage enhancement for commercial UEs, network can potentially configure RACH formats with larger number of sequence repetition number (e.g. format A3 or B4) to account for lower CINR values. However, this should be noted that higher number of repetitions reduces radio resources available for other UL transmissions. Networks may need to configure RACH formats with high number of sequence repetitions (e.g. A3 or B4) in NTN cells for commercial UEs in order to overcome low CINR values

    - Configuring cell specific RACH resources with larger number of repetitions leads to poor radio resource efficiency specially when only a small number of commercial UEs are trying to connect to NTN cell

  - [13/OPPO]

    - To further analyze the coverage performance in NR NTN, we provide the gaps between the required SNRs for different physical channels and target SNRs for different satellite parameters in Table 9 and Table 10 in the Appendix. However, not all channels are scheduled with the maximum number of repetitions in our simulation. To identify the candidate physical channels that have coverage issues, the required SNRs for different channels are further compensated with the maximum number of repetitions and the updated gaps are provided as follows: To cover the use case of commercial smartphone, the antenna gains of 0dBi and -5dBi are both considered to evaluate the coverage performance in NR NTN. In addition, we note that PUCCH repetition is not supported for initial access, so the gap for PUCCH Format1 without repetition is also provided in Table 2.

  - [17/DCM]

    - Combined with the SINR simulation results, the following observations were obtained: 25% GEO/ 100% LEO-1200 users can meet the required BLER of PUSCH-VoIP with Rep.8. 91% GEO/ 100% LEO-1200 users can meet the required BLER of PUSCH-VoIP with Rep.32. 91% GEO/ 100% LEO-1200 users can meet the required BLER of PUSCH-VoIP with TBoMS 8, Rep.4. 96% GEO/ 100% LEO-1200 users can meet the required BLER of PUSCH-VoIP with Rep.32, JCE. 96% GEO/ 100% LEO-1200 users can meet the required BLER of PUSCH-VoIP with TBoMS 8, Rep.4, JCE. Even when the antenna gain of UE is -5 dBi, in NTN in S-band, 96% GEO/ 100% LEO-1200 users can meet the required BLER of PUSCH-VoIP with Rel-17 features.

    - Combined with the SINR simulation results, the following observations were obtained: 55% GEO/ 100% LEO-1200 users can meet the required NACK to ACK probability of PUCCH format 1 with Rep4. 95% GEO/ 100% LEO-1200 users can meet the required NACK to ACK probability of PUCCH format 1 with Rep8. 97% GEO/ 100% LEO-1200 users can meet the required NACK to ACK probability of PUCCH format 1 with Rep8 & JCE4. 93% GEO/ 100% LEO-1200 users can meet the required ACK missed detection probability of PUCCH format 1 with Rep4. 99% GEO/ 100% LEO-1200 users can meet the required ACK missed detection probability of PUCCH format 1 with Rep8. 99% GEO/ 100% LEO-1200 users can meet the required ACK missed detection probability of PUCCH format 1 with Rep8 & JCE4. Even when the antenna gain of UE is -5 dBi, in NTN in S-band, 97% GEO/ 100% LEO-1200 users can meet the required error probability of PUCCH format 1 with Rel-17 features.

  - [20/THALES]

    - VoNR with 720 kHz bandwidth, and satellite receive antennas with 24 dB gain, the achievable UL SNR is -16,62dB. PUSCH msg3 might be also one of the coverage bottlenecks as the SNR is -13,61dB. The achievable UL SNR for PRACH long format 2 is -18,25dB. Repetition number of PRACH transmission may need to be increased.

    - To support smartphones in NTN, a significantly lower link budget needs to be supported by considering the following: Only one satellite receive antenna instead of two. Smaller satellite antenna gains and smaller elevation angles. When elevation angle decreases from 30 to 10 degrees, the path loss increases by about 4 to 5 dB for satellite orbits in 600km to 1200 km. Smaller UE antenna gains. Internal antennas of smartphones are typically negative (e.g. -5dBi).

  - [22/Ericsson]

    - The UL target SNR ranges from -23.63 dB to -5.22 dB with handheld devices in S-band and from 9.75 dB to 33.85 dB with VSAT devices in Ka-band.

    - Table 6 summarizes the required aggregation factor in different network scenarios, assuming a target residual BLER of 2%. It can be seen that only the LEO scenarios with Set-1 satellite parameters are supported by the simulated configurations. With handheld devices with realistic assumptions on antenna gain, VoIP coverage is a challenge with the evaluated configuration (up to 20 PUSCH repetitions and cross-slot channel estimation) except in LEO scenarios with satellite parameter set 1.

  - [23/QC]

    - For the AMR 4.75kbps voice codec and a satellite with the Set-2 parameters, with TBoMS, DMRS bundling, antenna switching and 16 repetitions for PUSCH, a commercial smart phone and a satellite with the Set-2 parameters can support elevation angle of 40 degrees or higher for 600kM satellite altitude. For higher satellite altitudes, the minimum supported elevation angles can be much higher. For example, for satellite altitude 1000km, the minimum supported elevation angle is 70 degrees, and that requires a higher-density constellation than the case of 600km satellite altitude. For 1200km, no elevation angle can be supported. Therefore, additional coverage enhancement techniques are needed to support a broader range of elevation angles for higher satellite altitudes.

    - With DMRS bundling, antenna switching, frequency hopping and 2 repetitions for PUCCH format 3 with payload size 8 bits, a commercial smart phone can support elevation angle of 30 degrees or higher for a 600kM LEO satellite.

    - PRACH is a bottleneck channel for supporting low-data rate services over commercial smart phones in NTN. As can be seen from Figure 10, antenna diversity provides about 1 dB gain at 1% miss detection rate. Since the bandwidth of PRACH format 2 is about 1 MHz, a X dB SNR for PRACH format 2 corresponds to  a X+7.6 dB SNR for 1 PRB PUSCH transmission with 15 kHz subcarrier spacing. Without antenna switching and assuming a frequency error of 200 Hz, the minimum SNR that can be supported with 1% miss detection rate is -16.4 dB, which corresponds to an SNR of -8.8 dB for a 180 kHz BW transmission.

    - Msg3 is a bottleneck channel for supporting low-data rate services over commercial smart phones in NTN. Rel-17 supports a maximum of 16 repetitions for both the initial Msg3 transmission and any Msg3 retransmission. Although the reliability of Msg3 can be improved via many retransmissions, such a scheme will lead to large delay given the large propagation delay in NTN. Therefore, it is important to enhance the performance of the initial Msg3 transission for low-data rate services. Figure 11 shows the Msg3 performance for a payload of 56 bits and a freuqnecy-domain resource allocation of 1RB. It is seen that for 16 repetitions and a target BLER 0.1, a minimum -9.3 dB SNR is required. The supported elevation angels are shown in Table 4. It is seen that the current Msg3 design in Rel-17 can support a very small range of elvevation angles (from 80 to 90 degrees) for satellite altitude 1200km.

- Potential techniques

  - [2/ZTE]

    - For VoIP service, interleaved multiple TBs can provide about 0.8 dB gain. Interleaved multiple TBs should not be supported with consideration on date rate and delay for VoIP.

    - Solutions to achieve the transmission/reception via the circular polarization should be supported for smart phones.

    - For VoIP service, higher transmission power should be supported for GEO and LEO-1200 scenarios with greater minimum elevation angle.

  - [3/Spreadtrum]

    - Repetitions enhancements for 2-Step RACH should be considered in R18 for smart phones in NTN.

    - Larger aggregation factor for PDSCH transmission should be considered in R18 for smart phones in NTN.

  - [4/MTK]

    - RAN1 can further study 2Tx diversity techniques including assumption for the number of receive antennas in the satellite for at least 2Tx antenna switching, 2 Tx precoding

  - [5/vivo]

    - Try to reuse the coverage enhancement techniques introduced in NR Rel-17 coverage enhancement topic for coverage enhancement in NTN to minimize the work load in NTN, and study whether any NTN specific changes are needed.

  - [7/Sony]

    - RAN1 should study configuration of UL waveform switching from CP-OFDM to DFT-s-OFDM for UEs in need of coverage enhancement.

    - Study more accurate UE beamforming for coverage enhancement of the NTN UL.

    - Study the impact of configuration of small transmission bandwidths for enhancement of the NTN UL coverage.

    - Study the use of repeat transmissions and impact of incremental redundancy for the low code rates typical of NTN in the enhancement of the NTN UL coverage.

    - Study the adoption of DFT-s-OFDM configurability for coverage enhancement of the NTN DL.

    - RAN1 should enhance the polarization support for Rel-18 to improve the coverage of NTN.

    - RAN1 can study how network can obtain the UE capabilities on supported polarization modes in Rel-18.

    - RAN1 should study inter-user multiplexing over the polarization domain in Rel-18.

    - RAN1 should study polarization indication per beam in Rel-18.

  - [9/Xiaomi]

    - The mechanism to improve the PDCCH coverage can be further studied in Rel-18 NTN.

    - The mechanism to compensate the polarization loss can be further studied in Rel-18 NTN.

  - [12/NEC]

    - At least two sets of RACH resources are required to be configured with different RACH formats or repetitions, to allow both VSAT UEs and commercial UEs gain access to NTN cell

    - Discuss how to configure multiple RACH resource sets per cell for initial random-access procedure. Option-1: Configure two separate UL cells (similar to SUL and NUL) to allow configuring different set of RACH resources for commercial UEs and VSAT UEs. Option-2: Configure two sets of RACH resources with different RACH formats/repetitions within initial active BWP

  - [13/OPPO]

    - Based on the evaluated results in NR NTN, the following channels have coverage issues, and enhancements should be considered: PRACH, PUSCH of Msg3, PUSCH, PUCCH, PUCCH during initial access

  - [14/Lockheed]

    - Using DFT-s-OFDM for the downlink waveform reduces the PAPR, which enables higher power and or more efficient transmissions to overcome increased path losses in NTN. Consider use of DFT-s-OFDM for downlink transmissions.

  - [18/Lenovo]

    - (Updated K-offset MAC CE is applied at the start of the first repetition of an uplink channel.)

    - Study the scenario where gNB adopts different polarization modes to serve UEs with different polarizations modes at different time instances.

    - Study the association between polarization mode and RS.

    - Study diversity scheme based on polarization in R18.

    - Study the impact of CSI prediction on system performance especially in NLOS channel conditions.

  - [19/LGE]

    - It can be desirable to consider introducing both PRACH preamble repetition and Msg. 3 PUSCH repetition in Rel-18 NTN.

    - If repeated transmission of PRACH preamble is supported in Rel-18 NTN, RAN1 should at least consider the following discussion points. Maximum repetition number, RA-RNTI, RAR window starting point

  - [20/THALES]

    - PUSCH: Same as considered for NR coverage enhancements with larger number of repetitions/aggregation slots.

    - PUCCH: Same as considered for NR coverage enhancements with larger number of repetitions/aggregation slots.

    - PUSCH Msg3: Enabling PUSCH coverage enhancements during RACH. Unlike in terrestrial network, re-scheduling of Msg3 is not desirable due to large round-trip delay in NTN.

    - PRACH: PRACH format 2. Evaluation is needed to determine the  number of repetitions that may be required

  - [21/Sharp]

    - With considerations on long NTN propagation distance and commercial smartphones with more realistic assumptions on antenna gains, uplink channels including PUCCH, PUSCH and PRACH could have coverage issues.

    - Large numbers of repetitions should be supported for NTN-specific uplink coverage enhancements in Rel-18.

    - Repetitions in both time domain and frequency domain should be supported for NTN-specific uplink coverage enhancements in Rel-18.

    - Joint consideration of repetitions and satellite movement/switch should be studied for NTN-specific uplink coverage enhancements in Rel-18.

  - [23/QC]

    - To support voice over commertical smart phones in NTN, RAN protocol overhead reduction is needed.

    - Consider enhancements to enable transmit antenna switching within a single-slot and multi-slot transmission.

    - Consider enhancements to enable transmit precoding in NTN including proper coherent MIMO requirements.

# References

- R1-2203159	Discussion on coverage enhancement for NR NTN	Huawei, HiSilicon

- R1-2203240	Discussion on coverage enhancement for NTN	ZTE

- R1-2203350	Consideration on coverage enhancement for NR NTN	Spreadtrum Communications

- R1-2203389	Coverage enhancement for NR NTN	MediaTek Inc.

- R1-2203588	Discussions on coverage enhancement for NR NTN	vivo

- R1-2203669	Discussion on NTN coverage enhancement	Panasonic

- R1-2203746	Considerations on improving NR NTN Coverage	Sony

- R1-2203757	Discussion on coverage enhancement for NR NTN	CATT

- R1-2203804	Discussion on coverage enhancement for NR-NTN	xiaomi

- R1-2203844	Solutions for coverage enhancements in NR over NTN	Nokia, Nokia Shanghai Bell

- R1-2203929	On coverage enhancement for NR NTN	Samsung

- R1-2203946	Discussion on coverage enhancements aspects for NR NTN	NEC

- R1-2204011	Discussion on coverage enhancement for NR NTN	OPPO

- R1-2204079	Consideration on coverage enhancement for NR NTN	Lockheed Martin

- R1-2204267	On coverage enhancement for NR NTN	Apple

- R1-2204328	Discussion on coverage enhancement for NR NTN	CMCC

- R1-2204402	Discussions on coverage enhancement for NR NTN	NTT DOCOMO, INC.

- R1-2204515	Discussion on coverage enhancement for NR NTN	Lenovo

- R1-2204521	Discussion on coverage enhancement for NR NTN	LG Electronics

- R1-2204545	Discussion on NR NTN coverage enhancement	THALES

- R1-2204645	Discussions on Coverage enhancement for NR NTN	Sharp

- R1-2204662	On coverage enhancement for NR NTN	Ericsson

- R1-2205058	Coverage enhancements for NR NTN	Qualcomm Incorporated

- Appendix-1 (Copy from WID)

### 4.1 Objective of SI or Core part WI or Testing part WI The work item aims at specifying enhancements for NG-RAN based NTN (non-terrestrial networks) according to the following assumptions with implicit compatibility to support HAPS (high altitude platform station) and ATG (air to ground) scenarios: - GSO and NGSO (LEO and MEO) with transparent payload. - Earth fixed tracking area. Earth fixed & Earth moving cells for NGSO - FDD mode - UEs with GNSS capabilities - Both “VSAT” devices with directive antenna (including fixed and moving platform mounted devices and commercial handset terminals (e.g. Power class 3) are supported in FR1 - Only “VSAT” devices with directive antenna (including fixed and moving platform mounted devices) are supported in above 10 GHz bands. Note: In Rel-17 WID, “VSAT” device with external antenna on moving platform is equivalent to a device that operate on platforms in motion, and this is referred to as ESIM. The detailed objectives are to specify enhancing features to Rel-15, 16 & 17’s NR radio interface & NG-RAN as follows: 4.1.1 Coverage enhancement The Rel-18 NTN objectives are focused on the applicability of the solutions developed by general NR coverage enhancement to NTN, and identifying potential issues and enhancements if necessary, considering the NTN characteristics including large propagation delay and satellite movement. Only NTN-specific characteristics are to be included in this coverage enhancement work, otherwise it should be part of another WI (e.g., UL enhancement of coverage). The work needs to cover the use case of voice and low-data rate services using commercial smartphones with more realistic assumptions on antenna gains instead of 0dBi currently assumed for link budget analysis for non-terrestrial networks. The specific realistic antenna gain assumption will be determined at the working group level. The evaluation should also take into account any related regulatory requirements, e.g., ITU limitation of power flux density. Have a 1-TU 6-month study phase focusing on the following (to derive clear & limited scope): - Evaluate the coverage performance and identify the candidate physical radio channels that have coverage issues specific to NTN with following target services taking into account the studies in TR38.830 where appropriate, as well as general coverage enhancement techniques specified in Rel-18 [RAN1,RAN2,RAN4] - VoIP and low-data rate services for commercial handset terminals The following items are shown as examples of areas to consider in the next step of the study. The actual items for study will be based on the evaluation of coverage issues specific to NTN identified above. - NTN-specific repetitions enhancements beyond techniques covered in Rel-17 CovEnh WI for the relevant channels - NTN-specific techniques for improved diversity and/or reduced polarization loss - Improved performance of low-rate codecs in link budget limited situation including reducing RAN protocol overhead for VoNR - NOTE: Intent is not to introduce a new codec. RAN to determine by RAN#97 (for RAN1 items) and RAN#98 (for RAN2 items) whether the study phase has identified any need for NTN-specific coverage enhancements in Rel-18. If needed, the set of NTN-specific work item objectives will be updated.

- Appendix-2 (Tables from TR38.821)

Table 6.1.1.1-1: Set-1 satellite parameters for system level simulator calibration

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Satellite antenna pattern |  | Section 6.4.1 in [2] | Section 6.4.1 in [2] | Section 6.4.1 in [2] |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note 1) | S-band (i.e. 2 GHz) | 22 m | 2 m | 2 m |
| Satellite EIRP density |  | 59 dBW/MHz | 40 dBW/MHz | 34 dBW/MHz |
| Satellite Tx max Gain |  | 51 dBi | 30 dBi | 30 dBi |
| 3dB beamwidth |  | 0.4011 deg | 4.4127 deg | 4.4127 deg |
| Satellite beam diameter (Note 2) |  | 250 km | 90 km | 50 km |
| Equivalent satellite antenna aperture (Note 1) | Ka-band (i.e. 20 GHz for DL) | 5 m | 0.5 m | 0.5 m |
| Satellite EIRP density |  | 40 dBW/MHz | 10 dBW/MHz | 4 dBW/MHz |
| Satellite Tx max Gain |  | 58.5 dBi | 38.5 dBi | 38.5 dBi |
| 3dB beamwidth |  | 0.1765 deg | 1.7647 deg | 1.7647 deg |
| Satellite beam diameter (Note 2) |  | 110 km | 40 km | 20 km |
| Payload characteristics for UL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note1) | S-band (i.e. 2 GHz) | 22 m | 2 m | 2 m |
| G/T |  | 19 dB K-1 | 1.1 dB K-1 | 1.1 dB K-1 |
| Satellite Rx max Gain |  | 51 dBi | 30 dBi | 30 dBi |
| Equivalent satellite antenna aperture (Note1) | Ka-band (i.e. 30 GHz for UL) | 3.33 m | 0.33 m | 0.33 m |
| G/T |  | 28 dB K-1 | 13 dB K-1 | 13 dB K-1 |
| Satellite RX max Gain |  | 58.5 dBi | 38.5 dBi | 38.5 dBi |
| NOTE 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of [2]. NOTE 2: This beam size refers to the Nadir pointing of the satellite NOTE 3: All these satellite parameters are applied per beam. NOTE 4: The EIRP density values are considered identical for all frequency re-use factor options. NOTE 5: The EIRP density values are provided assuming the satellite HPA is operated with a back-off of [5] dB. |  |  |  |  |

Table 6.1.1.1-2: Set-2 satellite parameters for system level simulator calibration

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Satellite antenna pattern |  | Section 6.4.1 in [2] | Section 6.4.1 in [2] | Section 6.4.1 in [2] |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note 1) | S-band (i.e. 2 GHz) | 12 m | 1 m | 1 m |
| Satellite EIRP density |  | 53.5 dBW/MHz | 34 dBW/MHz | 28 dBW/MHz |
| Satellite Tx max Gain |  | 45.5 dBi | 24 dBi | 24 dBi |
| 3dB beamwidth |  | 0.7353 deg | 8.8320 deg | 8.8320 deg |
| Satellite beam diameter (Note 2) |  | 450 km | 190 km | 90 km |
| Equivalent satellite antenna aperture (Note 1) | Ka-band (i.e. 20 GHz for DL) | 2 m | 0.2 m | 0.2 m |
| Satellite EIRP density |  | 32 dBW/MHz | 2 dBW/MHz | -4 dBW/MHz |
| Satellite Tx max Gain |  | 50.5 dBi | 30.5 dBi | 30.5 dBi |
| 3dB beamwidth |  | 0.4412 deg | 4.4127 deg | 4.4127 deg |
| Satellite beam diameter (Note 2) |  | 280 km | 90 km | 50 km |
| Payload characteristics for UL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note1) | S-band (i.e. 2 GHz) | 12 m | 1 m | 1 m |
| G/T |  | 14 dB K-1 | -4.9 dB K-1 | -4.9 dB K-1 |
| Satellite Rx max Gain |  | 45.5 dBi | 24 dBi | 24 dBi |
| Equivalent satellite antenna aperture (Note1) | Ka-band (i.e. 30 GHz for UL) | 1.33 m | 0.13 m | 0.13 m |
| G/T |  | 20 dB K-1 | 5 dB K-1 | 5 dB K-1 |
| Satellite Rx max Gain |  | 50.5 dBi | 30.5 dBi | 30.5 dBi |
| NOTE 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of [2]. NOTE 2: This beam size refers to the Nadir pointing of the satellite NOTE 3: All these satellite parameters are applied per beam. NOTE 4: The EIRP density values are considered identical for all frequency re-use factor options. |  |  |  |  |

Table 6.1.1.1-3: UE characteristics for system level simulations

| Characteristics | VSAT (Note 2) | Handheld | Other (Note 1) |
| --- | --- | --- | --- |
| Frequency band | Ka band(i.e. 30 GHz UL and 20 GHz DL) | S band (i.e. 2 GHz) | Ka band(i.e. 30 GHz UL and 20 GHz DL) |
| Antenna type and configuration | Directional Section 6.4.1 of [2] with 60 cm equivalent aperture diameter | (1, 1, 2) with omni-directional antenna element | Directional (M,N,P,Mg,Ng) = (TBD,TBD,2,1,1); (dV,dH) = (TBD, TBD)λ with directional antenna element (HPBW=65 deg) |
| Polarisation | circular | Linear: +/-45°X-pol | Linear: +/-45°X-pol |
| Rx Antenna gain | 39.7 dBi | 0 dBi per element | TBD dBi per element |
| Antenna temperature | 150 K | 290 K | TBD K |
| Noise figure | 1.2 dB | 7 dB | TBD dB |
| Tx transmit power | 2 W (33 dBm) | 200 mW (23 dBm) | [TBD W (TBD dBm)] |
| Tx antenna gain | 43.2 dBi | 0 dBi per element | TBD dBi per element |
| NOTE 1: Moving platforms (e.g., aircrafts, vessels), building mounted devices. These values are provided for information. NOTE 2: VSAT terminal characteristics could be implemented with phased array antenna |  |  |  |

Table 6.1.1.1-9: List of calibration study cases

| Case | Satellite orbit | Satellite parameter set | Central beam elevation | Terminal | Frequency Band | Frequency/ Polarization Reuse |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | GEO | Set 1 | 45 deg | VSAT | Ka-band | Option 1 |
| 2 | GEO | Set 1 | 45 deg | VSAT | Ka-band | Option 2 |
| 3* | GEO | Set 1 | 45 deg | VSAT | Ka-band | Option 3 |
| 4* | GEO | Set 1 | 45 deg | Handheld | S-band | Option 1 |
| 5* | GEO | Set 1 | 45 deg | Handheld | S-band | Option 2 |
| 6 | LEO-600 | Set 1 | 90 deg | VSAT | Ka-band | Option 1 |
| 7 | LEO-600 | Set 1 | 90 deg | VSAT | Ka-band | Option 2 |
| 8* | LEO-600 | Set 1 | 90 deg | VSAT | Ka-band | Option 3 |
| 9 | LEO-600 | Set 1 | 90 deg | Handheld | S-band | Option 1 |
| 10 | LEO-600 | Set 1 | 90 deg | Handheld | S-band | Option 2 |
| 11* | LEO-1200 | Set 1 | 90 deg | VSAT | Ka-band | Option 1 |
| 12* | LEO-1200 | Set 1 | 90 deg | VSAT | Ka-band | Option 2 |
| 13* | LEO-1200 | Set 1 | 90 deg | VSAT | Ka-band | Option 3 |
| 14 | LEO-1200 | Set 1 | 90 deg | Handheld | S-band | Option 1 |
| 15 | LEO-1200 | Set 1 | 90 deg | Handheld | S-band | Option 2 |
| 16** | GEO | Set 2 | 45 deg | VSAT | Ka-band | Option 1 |
| 17** | GEO | Set 2 | 45 deg | VSAT | Ka-band | Option 2 |
| 18** | GEO | Set 2 | 45 deg | VSAT | Ka-band | Option 3 |
| 19** | GEO | Set 2 | 45 deg | Handheld | S-band | Option 1 |
| 20** | GEO | Set 2 | 45 deg | Handheld | S-band | Option 2 |
| 21** | LEO-600 | Set 2 | 90 deg | VSAT | Ka-band | Option 1 |
| 22** | LEO-600 | Set 2 | 90 deg | VSAT | Ka-band | Option 2 |
| 23** | LEO-600 | Set 2 | 90 deg | VSAT | Ka-band | Option 3 |
| 24** | LEO-600 | Set 2 | 90 deg | Handheld | S-band | Option 1 |
| 25** | LEO-600 | Set 2 | 90 deg | Handheld | S-band | Option 2 |
| 26** | LEO-1200 | Set 2 | 90 deg | VSAT | Ka-band | Option 1 |
| 27** | LEO-1200 | Set 2 | 90 deg | VSAT | Ka-band | Option 2 |
| 28** | LEO-1200 | Set 2 | 90 deg | VSAT | Ka-band | Option 3 |
| 29** | LEO-1200 | Set 2 | 90 deg | Handheld | S-band | Option 1 |
| 30** | LEO-1200 | Set 2 | 90 deg | Handheld | S-band | Option 2 |
| NOTE 1: no star = 1st priority, * = second priority scenario, ** = third priority scenario NOTE 2: Only 1st priority cases will be considered for calibration phase 1 |  |  |  |  |  |  |

Table 6.1.2-1: LLS parameters for DL synchronization evaluation

|  | S-band | Ka-band |
| --- | --- | --- |
| Carrier Frequency | 2 GHz | 20 GHz |
| Channel Model | For GEO (optional): Baseline TDL/CDL model in [2], with delay/angular scaling factors equals to the mean delay/angular spread and mean K factor for suburban LOS elevation angle 10 deg For LEO: Baseline TDL/CDL model in [2], with delay/angular scaling factors equals to the mean delay/angular spread and mean K factor for suburban LOS elevation angle [30] deg |  |
| Subcarrier Spacing(s) | 15kHz, 30kHz | 120kHz, 240kHz |
| DL RS | SSB |  |
| Antenna Configuration at the TRP (satellite) | 1Tx | 1Tx |
| Antenna Configuration at the UE | (1, 1, 2) with omni-directional antenna element | VSAT with 60 cm equivalent aperture diameter |
| UE speed | 3 km/h | 0 km/h, 1200 km/h |
| UE elevation angle | For GEO (optional): 10°, For LEO: 30° |  |
| Frequency Offset | UE crystal accuracy: 10 ppm Satellite: oscillator accuracy values provided in Table 6.1.1.1-8 Doppler shift in channel due to satellite movement: max. Doppler shift values provided in Table 6.1.1.1-8 Doppler shift in channel due to UE movement: max. value to be computed based on the UE speed and the elevation angle Note 1: The final frequency offset is computed as follows where: denotes the final frequency offset in Hz denotes the UE crystal accuracy in ppm denotes Doppler shift due to satellite movement in ppm. Pre/post Doppler shift compensation can be assumed. denotes the Doppler shift due to UE movement in ppm denotes the carrier frequency used on the service Down Link in Hz A uniform distribution in [ - FO max value, + FO max value] shall be assumed. Note 2: Doppler spectrum on Rayleigh fading taps based on Jake model should be considered in addition to Doppler shift (see section 6.9.2 in [2]) Note 3: For a Rayleigh fading tap a minimum Doppler of 1 Hz should be considered. |  |
| Frequency drift | [Doppler rate values provided in Table 6.1.1.1-8] |  |
| Phase noise model | S-band phase noise modelling (optional) Ka-band phase noise modelling: phase noise profile according to TR38.803. |  |
| Metrics | One-shot initial cell detection accuracy of PCID; CDF of timing and frequency residual offset at SNIR point corresponding to 90% likelihood for one-shot detection accuracy of cell ID. Note 4: FAR of PCID detection requirement = 1% |  |
| NOTE: The SNR range to be evaluated should be based on the link budget analysis for each channel |  |  |

Table 6.1.2-2: LLS parameters for PRACH performance evaluation

| Configurations | S-band | Ka-band |
| --- | --- | --- |
| Carrier Frequency | 2 GHz | 30 GHz |
| Channel Model | Baseline TDL/CDL-D model in [2], with delay/angular scaling factors equals to the mean delay/angular spread and mean K factor for suburban at corresponding elevation angle for each case |  |
| Antenna Configuration at the TRP (satellite) | 1 Rx 2 Rx optional | 1 Rx 2 Rx optional |
| Antenna Configuration at the UE | Omni-directional antenna with single linearly polarized antenna element | VSAT with 60 cm equivalent aperture diameter |
| Frequency Offset | Doppler shift in channel due to satellite movement; max. Doppler shift values provided in Table 6.1.1.1-8 Doppler shift in channel due to UE movement; max. value to be computed based on the UE speed and the elevation angle Residual frequency offset after synchronization: [0.1] ppm Note 1: In case the network performs both pre and post common Doppler shift compensation, the final frequency offset is computed as follow: where denotes the final frequency offset in Hz denotes the residual frequency offset after synchronization in ppm denotes the residual Doppler shift due to satellite movement in ppm after common Doppler compensation denotes the Doppler shift due to UE movement in ppm denotes the central frequency used on the service Up Link in Hz A uniform distribution in [ - FO max value, + FO max value] shall be assumed Note 2: Doppler spectrum on Rayleigh fading taps based on Jake model should be considered in addition to Doppler shift (see section 6.9.2 in [2]) Note 3: For a Rayleigh fading tap a minimum Doppler of 1 Hz should be considered. |  |
| UE speed | 3 km/h | 0 km/h, 1000 km/h |
| Timing Offset | A uniform distribution in [0 max differential delay] shall be assumed. Note 1: Ideal common delay compensation is assumed. Note 2: The maximal differential delay values that should be supported for NTN are provided in Table 4.2-2. The max differential delays expected for specific cases can be computed based on the half power beam width and the target elevation angle. |  |
| Phase noise model | S-band phase noise modelling (optional) Ka-band phase noise modelling: phase noise profile according to in TR38.803. |  |
| PRACH design | Each company should provide details on configuration (i.e. format, SCS, N_CS, …). New formats are not precluded. |  |
| Metric | PRACH detection rate, FAR (Based on the preamble pool size is not less than 64), CDF of estimation error for frequency/timing, |  |
| Receiver | Companies are encouraged to report the receiver for PRACH detection. |  |

Table 6.1.2-3: PRACH study cases

|  | Elevation angle | Differential delay | UL Frequency offset (Both S- and Ka-band) (with compensation of common Doppler) | Beam Set at satellite |
| --- | --- | --- | --- | --- |
| Case 1 | 90 degree for LEO | Small | Large | Set-2 |
| Case 2 | 45 degree for LEO | Medium | Medium | Set-2 |
| Case 3 | 10 degree for GEO and 30 degree for LEO | Large | Small | Set-2 |
| Case 4 | With both open loop timing and frequency compensation | Small | Small | Set-2 |
| NOTE 1: As the baseline, the number of UEs that simultaneously access the network in a single random access occasion (RO) is 2. The two UEs may have different timing offsets/Doppler, which are randomly picked within the [0 Max_differential_delay]/[-max_UL_frequency_offset max_UL_frequency_offset] per case; NOTE 2: Fixed power offset between UEs is 3dB. NOTE 3: The SINR of the stronger UE for simulation is based on the SNR from link budget (with bandwidth for UL = 1MHz for VSAT in Ka, and Handheld for S) with additional offset of [-6-10 log10(Bandwidth [MHz])] dB) per case where the -6 dB degradation is introduced as additional margin. |  |  |  |  |

Table 6.1.2-4: LLS parameters for data transmission performance evaluation

| Parameters | S-band |  | Ka-band |
| --- | --- | --- | --- |
| Carrier frequency | 2 GHz |  | DL 20 GHz, UL 30 GHz |
| Channel coding scheme | NR channel coding |  |  |
| Subcarrier spacing | 15 kHz, 30 kHz |  | 60 kHz, 120 kHz |
| Channel estimation | Realistic estimation |  |  |
| Frequency offset | Residual Frequency error after DL synchronisation: [0.1] ppm assuming UL pre-compensation |  |  |
| Frequency drift | [Doppler rate values provided in Table 6.1.1.1-8] |  |  |
| Frequency tracking | Option 1: drift pre-compensation is assumed Option 2: no pre-compensation is assumed |  |  |
| UE speed | 3 km/h | 0 km/h, 1000 km/h |  |
| Channel model | For GEO (optional): Baseline TDL/CDL model in [2], with delay/angular scaling factors equals to the mean delay/angular spread and mean K factor based on the selected channel conditions. These parameters should be provided by the companies. For LEO: Baseline TDL/CDL model in [2], with delay/angular scaling factors equals to the mean delay/angular spread and mean K factor based on the selected channel conditions. These parameters should be provided by the companies. |  |  |
| Satellite antenna configuration | 1Tx/Rx |  | 1Tx/Rx |
| UE antenna configuration | (1, 1, 2) with omni-directional antenna element |  | VSAT with 60 cm equivalent aperture diameter |
| Phase noise Model | S-band phase noise modelling (optional) Ka-band phase noise modelling: phase noise profile according to TR 38.803 |  |  |
| Metrics | BLER, Throughput |  |  |

Table 6.1.3.2-1: Parameter configuration for link budget analysis

| Parameters | Notes |
| --- | --- |
| Carrier frequency | 2 GHz for DL and UL (S-band), 20 GHz for DL and 30 GHz for UL (Ka-band) |
| System bandwidth | 30 MHz (S-band), 400 MHz (Ka-band) |
| Channel bandwidth | DL: system bandwidth/ frequency reuse factor UL: UL in S-band (handheld UE): 360 kHz Otherwise: system bandwidth/ frequency reuse factor Note: The UL bandwidth may be a challenge. |
| Satellite altitude | 600 km, 1200 km, 35786 km |
| Target elevation angle | 30 (LEO), 12.5 (GEO-Set 1) , 20° (GEO –Set 2) |
| Atmospheric loss | Equation (6.6-8) in [2] |
| Shadowing margin | 0 dB for VSAT as terminal and 3 dB for others |
| Scintillation loss | Section 6.6.6 in [2] Ionospheric loss: = 2.2 dB (note 1) Tropospheric loss: Table 6.6.6.2.1-1 of [2] |
| Additional loss | 0 dB |
| Clear sky conditions | Yes |
| Frequency reuse factor | 1, 2, 3 |
| Average CIR within a satellite beam based on logarithmic mean | Based on single satellite system-level calibration methodology, statistics for average CIR are only collected for the UEs located in the central beam of the 19-beamlayout. The central beam boresight direction is computed based on the target elevation angle assumption. When the generated beam has a partial or full coverage outside the earth, it is discarded. For DL calibration, CIR is computed by averaging CIR over UEs randomly distributed over the reference beam (UE distribution assumption of Table 6.1.1.1-5). (See Figure 6.1.3.2-1 for UE bandwidth allocation, and Figure 6.1.1.1-1 and Figure 6.1.1.1-2 for beam deployment). For UL calibration, For Handheld device, the channel bandwidth is 360 kHz. For VSAT, the channel bandwidth equals the system bandwidth allocated to each beam divided by 10. The devices in one beam are allocated on adjacent frequency resources. The same resource allocation is assumed for all the beams. CIR is computed by averaging over 10 simultaneously transmitting UEs randomly distributed over the reference beam (UE distribution assumption of Table 6.1.1.1-5). (See Figure 6.1.3.2-2 for UE bandwidth allocation, and Figure 6.1.1.1-1 and Figure 6.1.1.1-2 for beam deployment) The averaging should be performed over multiple realizations. |
| Satellite antenna polarization | Circular polarization |
| Polarization reuse | Enable if frequency reuse factor = 2 is considered. |
| Terminal type | Ka-band: VSAT S band: (M, N, P) = (1,1,2) |
| Free space path loss | Equation (6.6-2) in [2] |
| Terminal RF parameters | Table 6.1.1-3 |
| Satellite RF parameters | Set-1 in Table 6.1.1-1 and Set-2 in Table 6.1.1-2 |
| Polarization loss | The considerations of Section 6.1.1.1 on Polarization loss apply. |
| Outcome | CNIR |
| NOTE 1: Based on P3 curve for 1% of time from Figure 6.6.6.1.4-1 of [2] after frequency scaling. dB |  |

- Appendix-3 (Tables from TR38.830)

Table A.1-1: General parameters for FR1

| Parameter | Value |
| --- | --- |
| Scenario and frequency | Urban: 4GHz (TDD), 2.6GHz (TDD) Rural: 4GHz (TDD), 2.6GHz (TDD), 2GHz (FDD), 700MHz (FDD) Rural with long distance: 700MHz (FDD), 4GHz (TDD) |
| Frame structure for TDD | DDDSU (S: 10D:2G:2U) only for 4GHz DDDSUDDSUU (S: 10D:2G:2U) only for 4GHz DDDDDDDSUU (S: 6D:4G:4U) only for 2.6GHz Other frame structures can be reported by companies. |
| Target data rates for eMBB | Urban: DL 10Mbps, UL 1Mbps Rural: DL 1Mbps, UL 100kbps Rural with long distance: DL 1Mbps, UL 100kbps, 30kbps (optional) |
| Packet size for VoIP | A packet size of 320 bits with 20ms data arriving interval is adopted. Size (bits). Payload; 256. CRC; 16 (TBS size lower than 3824 bits). MAC; 16 (with 12 bits SN size). RLC; 8 (with 6 bits SN size). PDCP; 16. RTP/UDP/IP; 24 (w RoHC). If applicable, companies report TB size assumed in evaluation. For SIP invite message - Payload of 1500 bytes can be a starting point. - The assumptions (TB size, time period etc.) are reported by companies. - Contributions R1-2003464 and R1-2005259 are taken into account for the evaluation - In addition, 1 second time period can also be considered. |
| Latency requirements for VoIP | Latency requirements assumed in VoIP evaluation for TDD and FDD are reported by companies. |
| Pathloss model (select from LoS or NLoS) | Urban: NLoS Rural: NLoS and LoS |
| BWP | 100MHz for 4GHz and 2.6GHz. 20MHz for 2GHz (FDD) 20MHz (optional for 10MHz) for 700MHz. (FDD) |
| Channel model for link-level simulation | TDL-C for NLOS, TDL-D for LOS. |
| Delay spread | Urban: 300ns Rural: 300ns Rural with long distance: 30ns |
| UE velocity | Urban: 3km/h for indoor Rural: 3km/h for indoor, 120km/h (optional 30km/h) for outdoor |
| Number of antenna elements for BS | - Urban: 192 antenna elements for 4GHz and 2.6GHz, (M,N,P,Mg,Ng) = (12,8,2,1,1) (optional) 128 antenna elements for 4GHz, (M,N,P,Mg,Ng) = (8,8,2,1,1) - Rural: 64 antenna elements for 4GHz and 2.6GHz (M,N,P,Mg,Ng) = (8,4,2,1,1) 32 antenna elements for 2GHz (M,N,P,Mg,Ng) = (8,2,2,1,1) 16 antenna elements for 700MHz (M,N,P,Mg,Ng) = (4,2,2,1,1) |
| Number of TxRUs for BS | gNB architectures to study: - 2 or 4 TXRUs for 2GHz, 700 MHz - 64TxRUs for 2.6 and 4 GHz. - Optional: 32 TXRUs at 2 GHz gNB modeling in LLS for TDL: - Option 1: 2 or 4 gNB RF chains in LLS. - Option 2 (Optional): Number of gNB RF chains = number of TXRUs in LLS. - Companies can report if and how correlation is modelled. |

Table A.1-2: Channel-specific parameters for PUSCH for FR1

| Parameter | Value |
| --- | --- |
| Frequency hopping | w/ or w/o frequency hopping |
| BLER | For eMBB, w/ HARQ, 10% iBLER; w/o HARQ, 10% iBLER. For VoIP, 2% rBLER. |
| Number of UE transmit chains | 1, 2 (optional) |
| DMRS configuration | For 3km/h: Type I, 1 or 2 DMRS symbol, no multiplexing with data. For 120km/h, (Optional: 30km/h): Type I, 2 or 3 DMRS symbol, no multiplexing with data. For frequency hopping: Type I, 1 or 2 DMRS symbol for each hop, no multiplexing with data. PUSCH mapping Type, the number of DMRS symbols and DMRS position(s) are reported by companies. |
| Waveform | DFT-s-OFDM, CP-OFDM (optional) |
| SCS | 30kHz for TDD, 15kHz for FDD. |
| PUSCH duration | 14 OS |
| Repetitions | For eMBB, w/o repetition as baseline, w/ repetition (optional). For VoIP, w/ type A repetition, optional for type B repetition. The actual number of repetitions is reported by companies. |
| HARQ configuration | For eMBB, whether HARQ is adopted is reported by companies. For VoIP, w/ HARQ. The maximum number of HARQ transmission (limited by frame structure and latency requirements) can be reported by companies. |
| PRBs/TBS/MCS for eMBB | Any value of PRBs, and corresponding MCS index, reported by companies will be considered in the discussion. Companies are encouraged to use 30 PRBs for 1Mbps, 4 PRBs for 100kbps, 1 PRB for 30kbps as a starting point. TBS can be calculated based on e.g. the number of PRBs, target data rate, frame structure and overhead. |
| PRBs/MCS for VoIP | 4 PRBs for VoIP as starting point. Other values of PRBs can be reported by companies. QPSK, pi/2 BPSK (optional) |

Table A.1-3: Channel-specific parameters for PUCCH for FR1

| Parameter | Value |
| --- | --- |
| PUCCH format | Format 1, 2bits UCI. Format 3, 4bits (3 bits A/N + 1 bit SR)/11/22 bits UCI |
| Frequency hopping | w/ frequency hopping |
| BLER | - For PUCCH format 1: DTX to ACK probability: 1%. NACK to ACK probability: 0.1%. ACK missed detection probability: 1%. - For PUCCH format 3: BLER for Ack/Nack, SR: 1% BLER for CSI: 1%, optional for 10%. |
| Number of UE transmit chains | 1 |
| DMRS configuration | Number of DMRS symbols for PUCCH Format 3: Reported by companies |
| SCS | 30kHz for TDD, 15kHz for FDD. |
| Repetitions | w/ repetition (optional), w/o repetition for PUCCH. The maximum number of repetitions is 8. |
| PUCCH duration | 14 OS |
| Number of PRBs | 1 PRB |

Table A.1-4: Channel-specific parameters for PRACH for FR1

| Parameter | Value |
| --- | --- |
| Format | Format 0, Format B4, or Format C2 |
| SCS | Reported by companies. |
| Performance metric | 1% missed detection at 0.1% false alarm probability 10% missed detection: reported by companies if this value is used |
| Number of UE transmit chains | 1, 2 (optional) |
| Other parameters | Reported by companies. |

Table A.1-5: Channel-specific parameters for PUSCH of Msg.3 for FR1

| Parameter | Value |
| --- | --- |
| Frequency hopping | w/ or w/o frequency hopping |
| Number of UE transmit chains | 1, 2 (optional) |
| Number of DMRS symbol | w/o frequency hopping: 3, w/ frequency hopping: 2 for each hop |
| Waveform | DFT-s-OFDM |
| SCS | 30kHz for TDD, 15kHz for FDD. |
| HARQ configuration | For eMBB, whether HARQ is adopted is reported by companies. For VoIP, w/ HARQ. The maximum number of HARQ transmission (limited by frame structure and latency requirements) can be reported by companies. |
| PUSCH duration | 14 OS |
| Number of PRBs | 2 |
| TBS | 56 bits |
| Other parameters | Reported by companies. |

Table A.1-6: Channel-specific parameters for PDSCH for FR1

| Parameter | Value |
| --- | --- |
| BLER | For eMBB, w/ HARQ, 10% iBLER; w/o HARQ, 10% iBLER. For VoIP, 2% rBLER. |
| Waveform | CP-OFDM |
| Number of UE receive chains | 4 for 4GHz/2.6GHz, 2 or 4 for 2GHz, 2 for 700MHz |
| SCS | 30kHz for TDD, 15kHz for FDD. |
| HARQ configuration | For eMBB, whether HARQ is adopted is reported by companies. For VoIP, w/ HARQ. The maximum number of HARQ transmission (limited by frame structure and latency requirements) can be reported by companies. |
| DMRS configuration | 3 DMRS symbols is used for PDSCH of Msg.2. For 3km/h: Type I, 1 or 2 DMRS symbol, no multiplexing with data. For 120km/h, (Optional: 30km/h): Type I, 2 or 3 DMRS symbol, no multiplexing with data. For frequency hopping: Type I, 1 or 2 DMRS symbol for each hop, no multiplexing with data. PDSCH mapping Type, the number of DMRS symbols and DMRS position(s) are reported by companies. |
| PRBs/MCS/TBS | Reported by companies. |
| PDSCH duration | 12 OS For PDSCH of Msg.4, 12 OS |
| Payload size for PDSCH of Msg.4 | 1040 bits |
| Other parameters | Reported by companies. |

Table A.1-7: Channel-specific parameters for PDCCH for FR1

| Parameter | Value |
| --- | --- |
| Number of UE receive chains | 4 for 4GHz/2.6GHz, 2 or 4 for 2GHz, 2 for 700MHz |
| SCS | 30kHz for TDD, 15kHz for FDD. |
| Aggregation level | 16 |
| Payload | 40 bits |
| CORESET size | 2 symbols, 48 PRBs |
| Tx Diversity | Reported by companies |
| BLER | 1% BLER optional for 10% BLER |
| Number of SSB for broadcast PDCCH of Msg.2 | Reported by companies |
| Other parameters | Reported by companies |

Table A.1-8: Channel-specific parameters for SSB for FR1

| Parameter | Value |
| --- | --- |
| Number of UE receive chains | 4 for 4GHz/2.6GHz, 2 or 4 for 2GHz, 2 for 700MHz |
| SCS | 30kHz for TDD, 15kHz for FDD. |
| Periodicity | 20ms |
| Performance metric | Combination of 4 SSBs in 80ms. Note: UE is not assumed to know the SS/PBCH block index |
| Other parameters | Reported by companies. |

- Appendix-4 (Outcomes of post meetings)

N/A