3GPP TSG RAN WG1 Meeting #109-e											          R1-2205220

e-Meeting, May 9th – 20th, 2022

Agenda Item:		9.1.4.2

Source:				Moderator (InterDigital, Inc.)

Title:					FL Summary on SRI/TPMI Enhancements; First Round

Document for:	Discussion and Decision

# Background

In RAN plenary #94, the WID for Rel-18 MIMO enhancements was finalized [1]. According to the WID, some enhancements for SRI/TPMI are necessary to enable 8 TX UE transmission.

Objective 5: Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices - Note: Potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will be identified as part of the study.

To accomplish the objective, the scope of this agenda item centers on codebook design for 8TX, CW to layer mapping, SRS enhancements to support 8 ports, impacts resulted from coherency characteristics of such UEs as well as UE operation with full power.

# High Priority Topics:

As it has been discussed by many companies, different variations of antennas placement can be considered where each could differently influence codebook design and coherency characteristics of a UE. Furthermore, to decide on the number of codewords, the number of layers, etc., extensive evaluations are required. And lastly, there are some levels of dependency of other sub-agendas on this work. Having said that it is important to establish a common view on UE antenna layout, evaluation methodologies and related assumptions in meeting WG1 # 109e.

# Antenna Layout for 8TX UE

According to companies’ contributions, different antenna layouts can be considered for 8TX UEs. Since antenna layout has a direct impact on codebook design, it is important to have a decision on this aspect before moving forward (ZTE, vivo, OPPO). Some companies have stated that various antenna layouts should be supported for uplink transmission of an 8TX UE (Huawei, IDC, Samsung, NTT, CATT). However, to save effort and reflect real-life use case, another group of companies have proposed to first identify/prioritize the most-likely device type (Samsung, Ericsson), and/or also identify/prioritize the most-likely antenna layouts (OPPO, Lenovo, NTT, MediaTek, Ericsson, CMCC).  Further, some companies have suggested that UE capability reporting should include some information about 8TX UE antenna layout (NTT, IDC). From (NEC) perspective, UE coherency capability reporting is sufficient and can bear related required information. Based on the views expressed by companies,

FL Proposal 2.1: For 8TX UE, down-select one of the following alternatives for UE antenna layout,

- Alt1: Support linear array (1D/2D) of single-polarized configuration

- Alt2: Support linear array (1D/2D) of cross-polarized configuration

- Alt3: Support either Alt1 or Alt2 according to a UE indication

Table 1

| Company | Views |
| --- | --- |
| Apple | We support Alt 2. Regarding single-polarized configuration, we are open to discuss further. We would like to understand better the scenarios we are targeting at and why cross-pol configurations may not be applicable for these scenarios. |
| LG | We prefer Alt 2 as cross pol is assumed in 2Tx and 4Tx UL. For Alt1, we are open for further discussion. |
| Samsung | Support Alt2 as baseline. In our view, to support various antenna layouts, we can consider Ng>=1 antenna groups, each group comprises coherent antennae, and across groups, antennae can be non-coherent/coherent depending on device types and scenarios. |
| OPPO | We prefer Alt 2 which is assumed for legacy codebook design. For an 8-port UE, the antenna size would be an issue with single-polarized configuration. |
| NTT DOCOMO | We support Alt2 and we share similar view as SS on antenna groups. |
| CMCC | Support Alt2. From our understanding, cross-polarized configuration is the most-likely antenna layout for 8T UE, so we prefer to prioritize the discussion based on cross-polarization assumption. |
| Lenovo | We prefer Alt 2 and share similar view as Samsung and DOCOMO on antenna grouping method. |
| NEC | We prefer Alt 2. And we share similar view as Samsung, DoCoMo and Lenovo that antenna groups can be considered. |
| vivo | We are fine with the proposal. In our view, this is somehow also related to coherency discussion in section 2.3, the UE antenna layout in this proposal is for full-coherent and partial-coherent. |
| Spreadtrum | We prefer Alt 2 as baseline. |
| ZTE | We share the above super-majority views that Alt-2 is sufficient. |
| Huawei, HiSilicon | We prefer Alt2 considering the cross-polarization configuration. And we share similar view with Samsung and others that there can be different antenna groups depending on UE implementation. Our understanding is that the intention of the proposal is for the antennas with coherence, as for antennas without coherence the antenna layout is not relevant. Therefore, we propose the following update: FL Proposal 2.1: For 8TX UE, down-select one of the following alternatives for UE antenna layout of the antenna group with coherent antennas - Alt1: Support linear array (1D/2D) of single-polarized configuration - Alt2: Support linear array (1D/2D) of cross-polarized configuration - Alt3: Support either Alt1 or Alt2 according to a UE indication |
| Intel | Fine with FL proposal, and Alt-2 is preferred. |
| CATT | We prefer Alt2. We are open to discuss whether other configurations also can be considered. |
| InterDigital | We support Alt2 as baseline, and share similar view as SS, DOCOMO, Lenovo, NEC, and Huawei for the antenna grouping. |
| Xiaomi | We prefer alt.2, but other configurations are not precluded for discussion and evaluation. |
| MediaTek | Alt2, as we are considering CPE/FWA devices, we understand that the devices are capable of cross-polarized configuration and can be considered as baseline. However, if there are more details and use cases on single polarized vs cross polarized configuration from 8TX perspective, we are open to the discussion. Further, we assume uniform antenna spacing in the antenna layout. |
| QC | Thank FL for the proposal. We prefer Alt 2. And we share the same view as Huawei and Samsung on the assumption of antenna coherence for this proposal. |
| FL | @All: Thanks for your valuable inputs. Please find below the updated proposal based on the provided inputs by majority. FL Proposal 2.1: For 8TX UE, down-select one of the following alternatives for UE antenna layout, - Alt2: Support linear array (1D/2D) of cross-polarized configuration |
| Ericsson | A ULA/UPA assumption excludes industrial or FWA devices that have spherical coverage. The ULA/UPA assumption is also inconsistent with Rel-15 designs that allow antenna selection in the sense that antenna selection PMIs work well with directional antennas. We suggest the following layout for 8 Tx as one of the setups to be evaluated: Alt 4: 4 sided directional cross pol array (4 dBi, 110° BW). Regarding Alt3, this seems like a specification issue rather than a simulation setup. Existing CSI reporting supports both Alt 1 and Alt 2 in my understanding. Shouldn’t we evaluate a codebook to see how well it works with different antenna configurations before discussing what is reported? Regarding Alt 1 vs. Alt 2: Alt 2 is a good candidate for outdoor mounted CPEs, and so we think it should be included. Alt 1 is also possible, but the use case is not so clear for us. So our proposal is to include Alt 2 and Alt 4 as primary configurations, so that precoding designs can address antenna systems with spherical coverage and those with only sectorized coverage. Alt 1 can be optional. |
| Nokia, NSB | Support Alt 2, as x-pol configuration is used in DL codebook design. We are open to Alt 1 or other possible UE antenna layouts. If we really want to consider UE antenna layout in 8Tx codebook design, we shall address different “CPE/FWA/vehicle/Industrial devices”. This large variant UE devices may have totally different antenna configurations with various antenna spacing and polarization configs. Any single assumption won’t be ideal, but we need to have one 8Tx codebook design. DL Type-I codebook is a good starting point. |
| Sharp | We support FL proposal 2.1. We believe that the use of cross-polarized antennas as the basis will reduce the effort of specifying the codebook. |
| CMCC | Support the updated FL proposal 2.1. |
| KDDI | We prefer Alt.2 as a baseline. On top of this alternatives, as indicated by other companies, antenna grouping needs to be considered for the type of devices and scenarios. |
| OPPO | We think the antenna layout is only used for evaluation of different solutions, and should be part of EVM. The codebook/non-codebook design can be applied to any antenna layout with 8 Tx in practical, though the performance may not be optimal. |
| ZTE | Fine with the updated FL proposal 2.1. But if only one Alt2, no need “down-select one of the following alternatives” part. |

# Evaluation Methodologies

Several aspects of 8TX UE operation rely on simulation studies to justify proposed enhancements. For example, determining potential benefits of 2 CW for UL, extending to more than 4 layers transmission, identifying proper codebook design, etc., they all require a baseline evaluation methodology to allow an accurate and fair comparison.

For this meeting, several companies have provided their simulation results. Some system level evaluation results are provided by (vivo, Samsung, ZTE, Ericsson, Intel, NTT, Nokia, Qualcomm, MediaTek), while other companies (vivo, Xiaomi, OPPO, Huawei, Lenovo) have shared their link level findings.

# SLS EVM

There are some differences in EVM assumptions used by companies in their conducted SLS simulation studies. For example, companies have assumed different assumptions for carrier frequency (2GHz, 3.5 GHz, 4GHz, etc.), SCS (15KHz, 30KHz), ISD (200m, 500m, 700m), etc. Therefore, to have a fair and an accurate comparison of provided results, it is important to have a baseline set of EVM parameters.  According to the EVM assumptions reported in companies’ contributions, the following is proposed as the reference for SLS evaluation.

FL Proposal 2.2.1: Adopt the following Table as the reference EVM for SLS evaluation.

Table 2

| Parameter | Value |
| --- | --- |
| Frequency range | 3.5 GHz |
| Multiple access | OFDMA |
| Numerology | 14 OFDM symbol slot SCS 15 KHz |
| Scenario | UMi (ISD = 200 m) UMa (ISD = 500 m) 80% Indoor, 20% Outdoor |
| System bandwidth | 52 RBs |
| gNB RX antenna setup and port layouts (𝑀,𝑁,𝑃,𝑀𝑔,𝑁𝑔,𝑀𝑝,𝑁𝑝) | (8,8,2,1,1,4,8) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 (2,2,2,1,1,1,2) with (dH, dV) = (0.5, 0.8)λ |
| gNB receiver noise figure | 5dB |
| gNB receiver | MMSE-IRC |
| gNB scheduler | Single user with proportional fair |
| Modulation | Up to 64QAM |
| MIMO scheme | SU-MIMO with rank adaptation |
| UE speed | 3 Km/h |
| UE TX antenna configuration | TBD |
| Traffic model | Full buffer |
| Codebook configurations | Non-codebook: eigenvector-based Rel-15 UL codebook: full/partial/non coherent Rel-15 DL Type 1 codebook: L=1 |
| Precoder granularity | Wideband |
| Power control | Open loop, P0=-50, alpha = 0.8 |
| Metric | UL mean-user throughput |

Table 3

| Company | Views |
| --- | --- |
| LG | We are generally ok with Table 2. But, indoor environment seems to be missed. |
| Samsung | We prefer reusing legacy EVM assumptions as much as possible. Therefore, we suggest the following: - Scenario: similar to DL, we propose to include Dense urban (38.901) - gNB antenna: 32 ports are too many. In practice, gNB will use smaller than 32 RF chains for reception. We think the following is reasonable: - 2Rx/4Rx/8Rx depending on max rank value - Traffic model: we usually simulate non-full-buffer (FTP1) in MIMO, since it is more realistic. So, we prefer FTP1 with 0.1Mbytes packet size. - Metric: UPT (avg. and 5% ) vs TPMI overhead |
| OPPO | 1. We think codebook configurations are not needed to be specified. 2. For gNB Rx antennae, (2,2,2,1,1,2,2) can be a better choice than (2,2,2,1,1,1,2), considering there would be 8 ports at UE. 3. we think 30kHz should be used instead of 15kHz, similar what we had for SRS in Rel-17. |
| NTT DOCOMO | We think dense urban scenario is important for 8TX UL. Hence, we propose to include ‘Dense urban, 1 layer macro (ISD = 200m)’. For other parameters, we’re generally OK. |
| CMCC | We prefer to use 2.6GHz as frequency range for EVM. |
| Lenovo | We prefer the second gNB Rx antenna layout considering fc@3.5GHz. |
| ZTE | For SCS 15 KHz: we prefer 30 KHz, which is a more typical value from perspective of NR product. For System bandwidth 52 RBs: we slightly prefer a value of e.g., 20 MHz, which is clearer than number of RBs. For gNB RX antenna setup and port layouts (𝑀,𝑁,𝑃,𝑀𝑔,𝑁𝑔,𝑀𝑝,𝑁𝑝): - (8,8,2,1,1,4,8) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 should be replaced by (8,8,2,1,1,2,8), which is more typical. - (2,2,2,1,1,1,2) with (dH, dV) = (0.5, 0.8)λ : no need so less # ports at gNB for 8Tx UE, should be removed. - Then, (4,4,2,1,1,4,4) which is also a typical configuration should be added. For UE TX antenna configuration: we suggest the following 2 types: - (M, N, P, Mg, Ng, Mp, Np) =(2,2,2,1,1,2,2) and (dH, dV) = (0.5, 0.5), and - (M, N, P, Mg, Ng, Mp, Np) = (4,1,2,1,1,4,1) and (dH, dV) = (0.5, 0.5) For Codebook configurations, - Non-codebook: eigenvector-based: non codebook based precoder depends on UE implementation, and, to be honest, the corresponding simulation should be deprioritized. - Rel-15 UL codebook: full/partial/non coherent: no need simulation for partial and non coherent cases, or low priority - Rel-15 DL Type 1 codebook: L=1: R15 UL 4-Tx based can also be evaluated, depending on outcome of codebook design. For Metric, besides UL mean-user throughput, at least 95%-ile UPT should be evaluated. Therefore, we suggest the following changes: Parameter; Value. Frequency range; 3.5 GHz. Multiple access; OFDMA. Numerology; 14 OFDM symbol slot SCS 15 30 KHz. Scenario; UMi (ISD = 200 m) UMa (ISD = 500 m) 80% Indoor, 20% Outdoor. System bandwidth; 52 RBs 20 MHz. gNB RX antenna setup and port layouts (𝑀,𝑁,𝑃,𝑀𝑔,𝑁𝑔,𝑀𝑝,𝑁𝑝); (8,8,2,1,1,42,8) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 (2,2,2,1,1,1,2) with (dH, dV) = (0.5, 0.8)λ (4,4,2,1,1,4,4) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆. gNB receiver noise figure; 5dB. gNB receiver; MMSE-IRC. gNB scheduler; Single user with proportional fair. Modulation; Up to 64QAM. MIMO scheme; SU-MIMO with rank adaptation. UE speed; 3 Km/h. UE TX antenna configuration; TBD (M, N, P, Mg, Ng, Mp, Np) =(2,2,2,1,1,2,2) with (dH, dV) = (0.5, 0.5) or (M, N, P, Mg, Ng, Mp, Np) = (4,1,2,1,1,4,1) with (dH, dV) = (0.5, 0.5). Traffic model; Full buffer. Codebook configurations; [Non-codebook: eigenvector-based] Rel-15 UL codebook: full[/partial/non coherent] Rel-15 DL Type 1 codebook: L=1 R15 UL 4-Tx based codebook. Precoder granularity; Wideband. Power control; Open loop, P0=-50, alpha = 0.8. Metric; UL mean-user throughput and 95%-ile UPT. |
| Huawei, HiSilicon | For system bandwidth, we prefer to use more typical values for network deployment, such as 40MHz/100MHz. For gNB antenna configuration, 32 ports and 8 ports seem to be more practical than 64 ports and 4 ports. |
| Intel | Generally fine with FL proposal. For the codebook configuration, we think Rel-15 UL 4Tx codebook should be the baseline since the simulation is for 8Tx UL. Suggest the following change: Rel-15 UL 4Tx codebook as baseline: full/partial/non coherent |
| CATT | Since 3.5GHz is considered, SCS of 30 kHz is preferred. For codebook configuration, Rel-15 UL codebook should be removed since 8 Tx is not supported in Rel-15. It is our view that Rel-15 DL Type 1 codebook for 8Tx can be considered as baseline for full-coherent. 2/4 coherent groups with Rel-15 4Tx/2Tx precoding matrices for each coherent group can be considered as baseline for partial-coherent. Codebook with antenna selection precoding matrices can be considered as baseline for non-coherent. |
| MediaTek | In general, we are fine. However, we would like to add few methodologies here - FTP traffic with RU 50% so as evaluate at low interference scenarios. - 8 Rx (2,2,2,1,1,2,2) at gNB considered for baseline or fine with Samsung proposal. 64 Rx ports is not needed and max 32 Rx ports is more practical. - UE speed for Indoor 3Km/hr and for outdoor 30Km/hr Depending on the agreed baseline CB design, we may need to carry out performance evaluation with fixed rank setting also. @FL: Please provide the clarification on the “L=1” definition under R15 DL Type 1 CB. |
| QC | We have a few comments on the SLS EVM assumptions. - Modulation order cap at 64QAM is an inappropriate/unfair assumption. In this simulation, 256QAM should be allowed, and the following is why. For a UE sees high UL SNR, there are two ways to boost UL throughput. One way is increase # layers to go beyond 4 layers. Another way is keeping max 4 layers while increase the modulation order to 256 QAM. Capping modulation order at 64QAM introduces an artificial restriction which is in favor of >4 layer Tx. Whatever gain showed with this artificial restriction is not valid/fair comparison between max 4 layers vs max 8 layers PUSCH. A fair comparison between should allow 256QAM and see whether increase mod order or increase # layers is more efficient to improve 8 Tx PUSCH throughput. At least based on our simulation result below, max 4 layers with max mod order 256QAM is better than max 8 layers with max mod order of 64QAM. Please notice that in the figure, legend “MCS 256-QAM” means max MCS is 256QAM, “MCS 64-QAM” means max MCS is 64QAM. Key observation from the above result. - 4-layer PUSCH with max 256 QAM can yield larger throughput than 8-layer PUSCH with max 64 QAM. - With max 256 QAM, the gain of 8-layer PUSCH over 4-layer PUSCH is marginal. - With artificial cap at max 64QAM, yes, there is noticeable gain of 8-layer PUSCH over 4-layer PUSCH. But again, the gain is based on inappropriate cap at max 64QAM. In summary, based on above analysis, we should do SLS with modulation order up to 256QAM. We also have some other comments as below. - SCS should be 30Khz, as other companies commented. - System bandwidth should be 20Mhz, as other companies commented. - gNB antenna: (2,2,2,1,1,1,2) with (dH, dV) = (0.5, 0.8)λ is not needed. Not sure why a gNB needs to support many 8 Tx UEs would have less than 8 Rx. - gNB scheduler: MU scheduling should be supported. Only support SU is oversimplification. - MIMO scheme: MU-MIMO should be evaluated. SU-MIMO is oversimplification. - Codebook: eigen vector based NCB is fine. For CB, it is up to each company to evaluate its codebook proposal. There is no need to force companies to evaluate which codebooks. - Power control: P0=-50 is too high. We can do a simple calculation assume the following gNB receiver NoiseFigure = 5 dB NoiseFloordBmPerHz = -174 dBm System Bandwidth = 25MHz With that, even assuming a PUSCH allocation take the whole system bandwidth (which is worst case for UL Tx), the noise power is -174+5+10*log10(25*10^6) = -105dB. If we set P0 to -50dB, which means target SNR is 55dB! We don’t think this is an appropriate SNR operation range for UL transmission. Large P0 will cause large IOT (interference over thermal) and hurt cell edge UE throughput and average throughput. We propose to set P0 in the range of [-90dBm, -80dBm], which translate to target SNR [15dB, 25dB], with full system bandwidth allocation for a PUSCH. One might think this SNR range is a little low. But please notice that, with smaller # RBs allocation, say ~6 RBs, the target SNR is boosted by ~10dB to reach [25dB, 35dB]. We think these are more reasonable operation SNR range for PUSCH, because full system bandwidth assign to a single UE is very rare. |
| FL | @All: Thanks for your valuable inputs. Please find below the updated table based on the provided inputs by majority. Updated FL Proposal 2.2.1: Adopt the following Table as the reference EVM for SLS evaluation. - Companies may provide additional evaluation results per their case of interest. Parameter; Value. Frequency range; 3.5 GHz. Multiple access; OFDMA. Numerology; 14 OFDM symbol slot SCS 30 KHz. Scenario; UMi (ISD = 200 m) UMa (ISD = 500 m) 80% Indoor, 20% Outdoor Dense urban, 1 layer macro (ISD = 200m). System bandwidth; 20 MHz. gNB RX antenna setup and port layouts (𝑀,𝑁,𝑃,𝑀𝑔,𝑁𝑔,𝑀𝑝,𝑁𝑝); (2,2,2,1,1,2,2) with (dH, dV) = (0.5, 0.8)λ (4,4,2,1,1,4,4) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆. gNB receiver noise figure; 5dB. gNB receiver; MMSE-IRC. gNB scheduler; Single user with proportional fair. Modulation; Up to 256QAM. MIMO scheme; SU-MIMO with rank adaptation. UE speed; 3 Km/h. UE TX antenna configuration; To be according to outcome of Proposal 2.1. Traffic model; Full buffer FTP traffic (RU 50%). Codebook for benchmarking; R15 UL 4-Tx based codebook Eigen-based (Non-codebook). Precoder granularity; Wideband. Power control; Open loop, P0=-50, -80, alpha = 0.8. Metric; UL mean-user throughput and 95%-ile UPT. |
|  | Some initial comments: Regarding the scenarios, as discussed in R1-220487 and R1-2204876, we think 3 scenarios should be defined for 8 Tx evaluations: Use-case-dependent system-level simulation parameters. Use case; Outdoor FWA; Indoor FWA; Industrial. UE distribution; 100% outdoor; 100% indoor; 100% indoor. Scenario; UMa (500 m ISD); UMi (200 m ISD); Indoor Office. Power control; [P0=-50, α = 0.8]; [P0=-80, α = 0.8]; [P0=-80, α = 0.8]. UE antenna configuration; ULA or UPA , pointing towards gNB; Pointing in different directions; Pointing in different directions. UE antenna pattern; Directional/Isotropic; Directional/Isotropic; Directional/Isotropic. UE antenna height; TBD; According to 36.873; According to 38.901. UE transmit power; [31] dBm; 23 dBm; 23 dBm. UE speed; 3 kmph; 3 kmph; 3 kmph. - For the outdoor FWA model - Operator installed CPEs are assumed, and so devices may be larger/more complex than typical UEs - CPEs can be mounted on the side or roof of the building, and the panel boresight can be pointed at the gNB during installation. - If one panel/sector is used, CPEs can be pointed at the gNB - If the CPEs have spherical coverage, no UEs are randomly oriented - All CPEs are outdoor - UE antenna height is TBD - UE power class can be [31 dBm], since it is mounted outside. 31 dBm is a starting point, and should be verified. - For the Indoor FWA model - Consumer deployed FWA or mobile hotspot devices are assumed, which may be more cost sensitive than operator deployed CPEs. - A smaller ISD seems appropriate given the greater pathloss - All UEs are indoor - UEs are randomly oriented - UE power class 3 can be used, since these are more similar to consumer devices. - For the Industrial model - Enterprise or factory indoor deployments are important to capture, since these can be strong scenarios case for higher throughput applications and since they are of high commercial interest. - All UEs are indoor, and the InH model seems a good starting point. Note that this is also being discussed for the STxMP evaluations in AI 9.1.4.1 - UEs are randomly oriented - UE power class 3 seems appropriate given that gNBs are indoor. Regarding the system simulation assumptions in general: - Channel model should be according to 38.901 - System bandwidth should be 273 PRBs (100 MHz). - The (M,N,P=2,2,2) antenna configuration is better suited for lower frequencies or for indoor gNBs. Since 8 Tx UEs at e.g. 700 MHz may not be so likely, we suggest it be used for indoor, to limit the number of simulation configurations. The gNB antenna models should be from 38.901. - We may wish to include SU/MU-MIMO switching, but can further discuss. - Full buffer is not appropriate to evaluate system level throughput in the general case. 8 layer transmission is sensitive to interference, and full buffer will have unrealistic interference behaviors, especially at the low loads where high order MIMO is likely to be beneficial. Non-full buffer models should be used. We suggest FTP model 1. A packet size of [500 kBs] seems appropriate. - Codebook configurations do not need to be identified as part of the simulation assumptions. This should be up to proponents. - Power control with P0=-50, α = 0.8 should be identified as a starting point. The power control parameters may need to be further optimized for the scenarios. - The metrics should be with {20, 50, 70}% RU and mean, median, and cell edge throughput. - It is important to consider realistic implementations as the antenna systems get more complex. Inter-element phase error should be considered for fully/partially coherent designs. 38.101 has ±40° phase error for 2 Tx. We can use a model with various phase errors to establish the sensitivity of designs. Suggest uniformly distributed X∈{±40°, ±20°,±5°}, phase error as a starting point. |
| Nokia, NSB | Generally okay, with some suggestions: System bandwidth shall be 100 MHz. Likely 8Tx shall be used for high-end UE for high data rate. gNB scheduler shall handle multiple UEs per TRP Channel model: shall be based on 38.901 Traffic model: could at least include bursty traffic model (RU 50% is okay) Precoder granularity: for Type-I codebook there is a subband component. We need to address subband granularity Metric: should include 5% throughput as well. |
| OPPO | We are general fine with the EVM. |
| CATT | In order avoid misleading, we prefer to change “R15 UL 4-Tx based codebook” to “R15 UL 4-Tx codebook”. |

# LLS EVM

There are some differences in EVM assumptions used by companies in their conducted LLS simulation studies. For example, companies have assumed different assumptions for modulation-coding (QPSK and fixed code-rate, AMC, etc.), SCS (15KHz, 30KHz), rank (fixed, adaptive, etc), etc. Therefore, to have a reasonable and an accurate comparison of provided results, it is important to have a baseline set of EVM parameters.  According to the EVM assumptions reported in companies’ contributions, the following is proposed for as the reference for LLS evaluation.

FL Proposal 2.2.2: Adopt the following Table as the reference EVM for LLS evaluation.

Table 4

| Parameter | Value |
| --- | --- |
| Carrier Frequency | 3.5 GHz |
| Waveform | CP-OFDM |
| SCS | 15 KHz |
| Scheduled bandwidth | 52 RBs |
| gNB RX antenna setup and port layouts (𝑀,𝑁,𝑃,𝑀𝑔,𝑁𝑔,𝑀𝑝,𝑁𝑝) | (8,8,2,1,1,4,8) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 (2,2,2,1,1,1,2) with (dH, dV) = (0.5, 0.8)λ |
| UE TX antenna configuration | TBD |
| UE speed | 3 Km/h |
| Layer | Adaptive rank |
| AMC | On |
| DMRS configuration | Type 1 |
| Channel estimation | Real |
| Channel Model | TDL-A (30ns), TDL-C (300ns), CDL-A (30ns), CDL-C (300ns) |

Table 5

| Company | Views |
| --- | --- |
| LG | Ok with Table 4. One clarification is whether both LLS and SLS are mandatory or not. We thinks it is up to each company. |
| Samsung | We are OK to discuss LLS assumptions, we need to clarify what it will be used for. For instance, LLS should not be used for 8Tx CB evaluations. |
| OPPO | 1. For gNB Rx antennae, (2,2,2,1,1,2,2) can be a better choice than (2,2,2,1,1,1,2), considering there would be 8 ports at UE. 2. we think 30kHz should be used instead of 15kHz, similar what we had for SRS in Rel-17. 3. BLER with fixed MCS can also be used as a metric. 4. Can Samsung clarify why LLS cannot be used for CB evaluation? In LTE and NR Rel-15, only LLS is used for evaluation of UL codebook design. |
| NTT DOCOMO | Similar comment with LG. |
| CMCC | We prefer to use 2.6GHz as frequency range for EVM. |
| Lenovo | We prefer the second gNB Rx antenna layout considering fc@3.5GHz. |
| ZTE | In our views, it is less essential over SLS. But, anyway, we are open to make a consensus for the simulation assumption. The final agreed parameters of SLS can be reconsidered for LLS, such as SCS, bandwidth... |
| Huawei, HiSilicon | For scheduled bandwidth, we prefer to have a larger value, such as 104 RB for 10MHz. For gNB antennas, 32 ports and 8 ports would be more practical. In addition, we prefer to add CDL-B channel model to cover more scenarios. |
| Intel | We think LLS simulation could be optional for 8Tx UL. |
| MediaTek | We also believe LLS simulations should be optional. |
| QC | Same view as Intel. LLS should be optional for 8 Tx study. Besides, we have some detailed comments as below. - SCS should be 30Khz, as other companies commented. - System bandwidth should be 20Mhz, as other companies commented. - gNB antenna: (2,2,2,1,1,1,2) with (dH, dV) = (0.5, 0.8)λ is not needed. Not sure why a gNB needs to support many 8 Tx UEs would have less than 8 Rx. |
| FL | @All: Thanks for your valuable inputs. Please find below the updated table based on the provided inputs by majority. FL Proposal 2.2.2: Adopt the following Table as the reference EVM for LLS evaluation, if required. - Companies may provide additional evaluation results per their case of interest. Parameter; Value. Carrier Frequency; 3.5 GHz. Waveform; CP-OFDM. SCS; 30 KHz. System bandwidth; 20 MHz. gNB RX antenna setup and port layouts (𝑀,𝑁,𝑃,𝑀𝑔,𝑁𝑔,𝑀𝑝,𝑁𝑝); (4,4,2,1,1,4,4) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 (2,2,2,1,1,2,2) with (dH, dV) = (0.5, 0.8)λ. UE TX antenna configuration; To be according to outcome of Proposal 2.1. UE speed; 3 Km/h. Layer; Adaptive rank. AMC; On. DMRS configuration; Type 1. Channel estimation; Real. Channel Model; TDL-A (30ns), TDL-C (300ns), CDL-A (30ns), CDL-C (300ns). |
| Ericsson | Some initial comments: - Channel model should be according to 38.901 - System bandwidth should be 273 PRBs (100 MHz). We think both a small and large number of PUSCH PRBs could be studied to account for frequency diversity or its lack. So [4, 273] PRBs is one possibility. No strong view on the exact numbers. - The (M,N,P=2,2,2) antenna configuration is better suited for lower frequencies or for indoor gNBs. Since 8 Tx UEs at e.g. 700 MHz may not be so likely, we suggest it be used for indoor, to limit the number of simulation configurations. The gNB antenna models should be from 38.901. - Both fixed and adaptive rank should be considered. For example, BLER curves may be useful to consider reliability at different error rates. - Inter-element phase error should be considered for fully/partially coherent designs. 38.101 has ±40° phase error for 2 Tx. We can use a model with various phase errors to establish the sensitivity of designs. Suggest uniformly distributed X∈{±40°, ±20°,±5°}, phase error as a starting point. - TDL models should not be used for MIMO design, especially if they are uncorrelated. Spatial channel models, including CDL models should be used instead. |
| Nokia, NSB | Agree with companies on the motivations of using LLS for 8Tx codebook design. We shall mainly use system-level simulation for 8Tx codebook design. LLS could be optional. |
| KDDI | The scope includes a vehicle use case, so there is no need to exclude the assumptions about UE speed assuming a vehicle. |
| OPPO | For LLS, we think 3km/h is sufficient. We are fine with the EVM. |

# Coherency of 8TX UEs

Many companies (Ericsson, MediaTek, ZTE, OPPO, Lenovo, Apple, Qualcomm, LG, Intel) have acknowledged the importance of considering the coherency characteristics of UE into the general scope of the work and aspects related to codebook design. The main difference in companies’ views is whether it is relevant to consider all three levels of coherency, i.e., full coherent, partially coherent and non-coherent, or only a subset of the three possible capabilities. In other words, some companies believe that for considered use cases and applications, consideration of a fully coherent or non-coherent 8TX UE may be too optimistic or pessimistic, respectively. For example, from hardware cost perspective, it may not be reasonable to assume a fully coherent 8 TX UE for most applications, as maintaining coherency over 8 antennas may be more difficult and more expensive than the case of a 4TX UE. On the other hand, maintaining coherency over a set of two or 4 antennas may be well within the current cost structure of a 4 TX UE, therefore it would not be likely not to have some coherent pairs in an 8 TX UE. Having said that based on the current expressed views, it seems that the necessity of support of at least partially coherent UEs is shared by most companies.

FL Proposal 2.3: At least partial-coherent Ues are considered for uplink transmission with 8TX Ues.

- FFS applicability of fully-coherency and non-coherency conditions for 8TX UE

Table 6

| Company | Views |
| --- | --- |
| Apple | We are fine with the proposal. On the full coherent configurations, we think it is possible at least for FWA devices, which is why we suggested to include it in the design. On the non-coherent configurations, we would like to understand the practical use cases where all the 8Tx are non-coherent. |
| LG | Fine with proposal. |
| Samsung | For CPE/FWA, the antenna structure is likely to support both full- and partial-coherency. So, we think at least full and partial-coherency should be supported. Re non-coherency, we can consider it if RAN1 wants to support devices with all 8 non-coherent antennae. |
| OPPO | We agree with apple that full coherence is possible, while 8Tx non-coherent may be a corner case. |
| NTT DOCOMO | Our first preference is to consider all three levels of coherency. If priority consideration is needed, we think at least partial-coherency and full-coherency should be supported. |
| CMCC | We could not predict the industrial maturity, maybe there exists non-coherent 8T UE, so we prefer to consider all the coherence type. |
| Lenovo | In our opinion, coherence assumptions should be carefully studied first, and a framework with ≥2 coherence group is studied, with antennas per coherence group being fully coherent and antennas across groups partially/non-coherent. Agreeing on partial coherence without further detail may not be very insightful for the study. |
| NEC | Fine with the proposal. And for full coherency and non coherency, we also think they are also possible. |
| Vivo | In our view, non-coherent antennas should also be supported, from spec point of view this is straight forward extension of non-coherent codebook subset. Besides, non-codebook is to be supported, which could support non-coherent antennas. For full-coherent, it could be limited to antenna layout in section 2.1. |
| Spreadtrum | Agree with FL proposal and full coherent and non-coherent should also be supported. |
| ZTE | Agree with Apple that full coherent configuration should be supported by at least high-cost FWA devices. That would be shining point of UL 8-Tx in Rel-18. So we strongly suggest to include full coherent configuration in the main bullet as follows. FL Proposal 2.3: At least partial-coherent Ues and fully-coherent Ues are considered for uplink transmission with 8TX Ues. FFS applicability of fully-coherency and non-coherency conditions for 8TX UE |
| Huawei, HiSilicon | We prefer to consider all the coherence types to accommodate different UE antenna layouts. |
| Intel | We are open to discussion on the coherence types including full coherent/partial coherent/non-coherent. |
| CATT | We prefer to consider all of coherent types. |
| InterDigital | Support the FL proposal at least as a solid common ground among companies, which is also in line with the antenna grouping, mentioned in Seciton2.1. We are supportive to study the fully-coherency and non-coherency conditions as well. |
| Xiaomi | Same view with Apple. Generally fine with the FL’s proposal. |
| MediaTek | We share similar views as Samsung. |
| QC | We are fine to prioritize partial coherent Tx. We have similar view as VIVO non-coherent should be supported, due to trivial spec impact and easy UE implementation. For full coherent UE, we can start the discussion on nailing down the assumption of antenna layout. |
| FL | @All: Thanks for your valuable inputs. Please find below the updated proposal based on majority view. FL Proposal 2.3: Support coherent and partially-coherent Ues for uplink transmission with 8TX Ues. - FFS applicability of non-coherency conditions for 8TX UE. |
| Ericsson | We think all 3 non/partial/fully coherent capabilities should be studied, given the CPE/FWA/vehicle/Industrial devices that are in scope. These represent a wide range of cost/complexity as well as form factors, and so excluding any one of these designs should not be done without some investigation or refinement of scope. Regarding use cases for non-coherent, one example configuration would be a 4 panel UE with directional cross pol antennas. |
| Nokia, NSB | The proposal is okay. But there are various configurations or coherent levels for 8Tx partial-coherent antennas. Unlike 4Tx partial-coherence, 8Tx partial-coherence would have multiple meanings. We may clarify/define 8Tx coherence level before we can decide which coherent levels are supported. |
| Sharp | We prefer to consider all coherent types. |
| CMCC | We still prefer to consider all the coherent types. We could not see the reasonable point to preclude non-coherent type at this early stage. |
| KDDI | We are fine with the updated FL’s proposal. |
| OPPO | We are fine with the updated proposal. |
| ZTE | Fine with the updated proposal 2.3. |

# Detailed Topics

In this section, detailed topics related to operation of 8TX UE are discussed. Final decisions on these topics may hinge on evaluation results using an agreed set of EVM.

# Number of Layers and CWs

In NR Rel-17, uplink transmission is restricted to single codeword and a maximum of 4 layers for transmission. However, for 8TX UEs, some companies are arguing for increasing the number of codewords and layers.

- Number of Layers

Companies have discussed benefits of utilizing more than 4 layers for uplink transmission (ZTE, vivo, Xiaomi, CMCC, NTT, MediaTek, Ericsson), and further, some companies have shared their evaluation results indicating a higher achieved throughput when 8 layers are used. However, another group of companies seem to be skeptical about the promised gain, and they also have expressed their concern about the workload related to codebook design for transmission with 8 layers. From their perspectives, support of 4 layers should be prioritized, and then upon further evaluation, if justified by use case and the observed performance gain, transmission with more than 4 layers can be supported (Samsung, Lenovo, MediaTek, Qualcomm, Intel). Given that the main concern expressed by companies for transmission with 8 layers is related to potential complications for codebook design, the following is proposed,

FL Proposal 3.1a: For 8TX UE uplink transmission,

- for codebook-based, prioritize transmission with 4 layers

  - FFS support of transmission with 8 layers, i.e., if justified by further evaluation

- for non-codebook-based, support transmission with 8 layers

Table 7

| Company | Views |
| --- | --- |
| Apple | Although we understand the thinking behind the proposal, it is a bit strange to support different # of layers for codebook-based and non-codebook-based schemes. Whether to support 4 or 8 layers should be decided based on the performance gain, and it is not clear to us why this is different for the two transmission schemes. |
| LG | Agree with Apple that it is unclear to have different the maximum # of supported layer for CB and NCB. |
| Samsung | We are OK to prioritize up to 4 layers for both CB and NCB-based UL, and are open to further study >4 layers for both CB/NCB. |
| OPPO | We also prefer the same number of max layer for CB and NCB. And We think maximal 8 layers can be beneficial for both CB and NCB. |
| NTT DOCOMO | As pointed out by Apple/LG, it is unclear to have different maximum number of supported layers for CB based NCB. In addition, based on our simulation results, both CB-based and NCB-based PUSCH transmission with up to 8 layers could achieve significant performance gain. |
| CMCC | To consider the performance, some companies have shared their evaluation results indicating a higher achieved throughput when 8 layers are used, we propose to support 8 layers UL transmission. To consider different use cases and different channel conditions of 8 TX UL transmission and consider the specification integrity and forward compatibility, we also propose to support 8 layers UL transmission for both codebook and non-codebook-based transmission. |
| Lenovo | First, we understand the supported number of layers should be determined based on performance evaluation. Second, we prefer to have unified consideration for both schemes on this issue since we fail to see the motivation to treat both schemes separately. |
| NEC | We also prefer same number of max number of layers for CB and NCB. |
| Vivo | In our view, codebook based and non-codebook based can be on equal footing, start with 4 layers, main discussion points for >4 layers is DCI design/overhead, CW layer mapping etc which are same for both codebook and non-codebook based transmission schemes |
| Spreadtrum | Same number of max layer number should be supported |
| ZTE | UL 8-Tx could support up to 8 layers, anyway we need to design codebook for 8 port in Rel-18. The main workload comes from 4Tx to 8Tx. We don’t think it is a big burden for supporting 8Tx + up to 8 layers on basis of 8Tx + up to 4 layers, assuming a common framework can be used. Some companies, including ZTE, have provided evaluation results which show a considerable benefit to support up to 8 layers for 8Tx. We all know that we may always go further than commercialization. So supporting up to 8 layers is very important for UL 8Tx in Rel-18 and we suggest the following changes: FL Proposal 3.1a: For 8TX UE uplink transmission, - for codebook-based, prioritize transmission with up to 8 4 layers - FFS support of transmission with 8 layers, i.e., if justified by further evaluation - for non-codebook-based, support transmission with up to 8 layers |
| Huawei, HiSilicon | We prefer to support UL transmission with up to 8 layers for both CB and NCB. As shown in our contribution (R1-2203152) for DMRS, there are a large number of UE using rank>4, which can be illustrated by the following figure. |
| Intel | Fine with FL proposal. 4 layers is preferred as starting point. |
| CATT | Similar view as the majority that same max number of layers for CB and NCB. We think up to 8 layers shall be supported. |
| InterDigital | We are fine with ZTE’s revision (with changing ‘prioritize’ to ‘support’), based on the current observations from evaluations including the one Huawei mentioned. Details on precoding mechanism supporting up to 8 layers for CB-UL and non-CB-UL can be separately discussed further. |
| Xiaomi | we support up to 8 layers for both CB and NCB. |
| MediaTek | We prefer to have consistency between NCB and CB based transmission. Furthermore, we would like to prioritize 4 layers for both NCB and CB transmission. |
| QC | We thank FL for the proposal. But we disagree with the way to decide supported max layers for 8Tx based on NCB and CB. The decision should be based on performance. If 8-layer has no gain over 4-layer PUSCH, then we don’t support it for both CB and NCB. Based on our simulation result below, 8-layer PUSCH has only marginal gain over 4-layer PUSCH, with 256QAM. Please notice that in the figure, legend “MCS 256-QAM” means max MCS is 256QAM, “MCS 64-QAM” means max MCS is 64QAM. Key observation from the below result. - 4-layer PUSCH with max 256 QAM can yield larger throughput than 8-layer PUSCH with max 64 QAM. - With max 256 QAM, the gain of 8-layer PUSCH over 4-layer PUSCH is marginal. - With artificial cap at max 64QAM, yes, there is noticeable gain of 8-layer PUSCH over 4-layer PUSCH. But again, the gain is based on inappropriate cap at max 64QAM. Based on our understanding of other companies simulation results, all the results showing gain of 8-layer PUSCH over 4-layer PUSCH are capping mod order at 64QAM, which is an unfair assumption, because it forces UE to use available power to transmit >4 layers, while UE has another choice to use available power to transmit with 256QAM but keep 4 layers. A fair comparison should allow 256QAM and the gain of 8-layer PUSCH would diminish, like we show in above figure. Based on the above observations, we don’t support max 8 layers for both CB and NCB. 4 layer is sufficient enough for 8 Tx PUSCH. |
| FL | @All: Thanks for your valuable inputs. Please find below the updated proposal based on majority view. FL Proposal 3.1a: For 8TX UE uplink transmission, - prioritize specification for transmission with 4 layers - support of transmission with 8 layers, if justified by evaluation (decision to be made in WG1 #110) |
| Ericsson | We do not agree to prioritize to 4 layers at this time. Given that companies already have simulation results showing the benefit of 8 layers, and since this is the first meeting, such a proposal is premature. That being said, we are also concerned about workload. The different design alternatives: codebook, multi-TPMI/SRS resource, and non-codebook each can have different design complexity depending on the approaches. So we should address the workload and specification impact as part of the investigation into these solutions. |
| Nokia, NSB | What does the “prioritize” mean in this context? For “UL targeting CPE/FWA/vehicle/Industrial devices”, we are targeting high-end Ues that will support higher data rate and a higher peak rate. There is no need to “prioritize” 4 layers at this stage. |
| Sharp | We prefer to support up to 8 layer. |
| CMCC | Some companies have already shared their evaluation results indicating a higher achieved throughput when 8 layers are used, why do we need to make decision in WG1#110? We do not agree to prioritize 4 layers, and we propose to support 8 layers UL transmission in this meeting. |
| KDDI | We think that should not decide to prioritize to 4 layers at this time. Some companies have already shown the benefit of 8 layers obtained in the case of non-codebook-based. On the other hand, considering the workload, in the case of codebook-based, the performance gain may not be sufficient for the specification impact. So, we need to discuss whether specify codebook-based or not. |
| OPPO | We think further evaluation is needed before we make this decision. RAN1 can evaluate and make a decision in the next meeting. |
| CATT | We also prefer not to set priority. We support up to 8 layers. |
| ZTE | We still believe Rel-18 should support up to 8 layers. But we agree with FL that we can see more evaluations. Regarding to 256QAM mentioned by QC, we don’t think all UEs could support such high modulation order. For some UE which could support up to 64, we should consider up to 8 layers for enhancement. As shown by QC’s results, it is worthy of introducing 8 layers for the UE with up to 64QAM. |

- Number of CWs

Several Companies have argued in favor of supporting 2 CW for at least when more than 4 layers are used for transmission (ZTE, vivo, Xiaomi, CMCC, NTT, MediaTek). According to presented arguments by companies, use of more than 1 CW can be useful to address potential difference in signal quality experienced by different layers. Furthermore, utilization of 2 CW may be very well suited for 8 TX UEs with two panels where each panel may become involved in transmission of a different CW. If 2 CW is supported, to reduce the workload, some companies have proposed to reuse the NR DL codeword to layer mapping (MediaTek, Qualcomm, LG). And lastly, (IDC) has proposed to link usage of 2 CW according to coherence capability of a UE. Based on expressed views by companies, utilization of 2 CW can be supported for at least transmission with more than 4 layers.

FL Proposal 3.1b: For 8TX UE uplink transmission, support 2 CW transmission for when more than 4 layers are used.

- reuse DL CW to layer mapping for more than 4 layers

Table 8

| Company | Views |
| --- | --- |
| Apple | To clarify, based on the formulation of the proposals, does it mean that we already agree to support 8Tx (and more than 4 layers, which is part of P3.1a)? As this objective has a study phase, it would be good to be clear about what we are trying to agree on exactly. Maybe we could add a qualifier such as “if supported”. We are generally fine with the proposal if more than 4 layers are agreed to be supported. |
| Samsung | We are open to study it after some progress on 8Tx CB and max supported #layers are made. |
| OPPO | We are fine with the proposal. |
| NTT DOCOMO | Support the FL proposal and fine with adding ‘if supported’. |
| Lenovo | Support in principle. Suggesting minor wording change to address Apple’s concerns: FL Proposal 3.1b: For 8TX UE uplink transmission with >4 layers (if supported), support 2 CW transmission for when more than 4 layers are used. - reuse DL CW to layer mapping for more than 4 layers (if supported) |
| NEC | We are fine with the proposal. |
| Vivo | In our view, first we take proposal 3.1a and further evaluate/justify support for 8 layers. Hence, this proposal is not needed in this meeting |
| Spreadtrum | Support FL proposal |
| ZTE | To our understanding, 2CWs should be considered in following cases: - Case 1: >4 layers. Similar to DL >4-layer case, it should be straightforward to support 2CWs. - Case 2: STxMP. If UL transmissions are simultaneously transmitted via multiple panels, towards multiple TRPs, it is very reasonable to support separate CW for each panel-TRP link. - Case 3: 2~4 layers. As discussed in R1-2110955 (TEI-17 proposal on NR codeword mapping), 2 CWs provide more flexibility which can benefit for alleviating performance loss due to large difference among layers which was observed in reality by our product team. Furthermore, in Rel-18, on the basis of supporting 2CWs for case 1 and case 2, it would not bring much workload for case 3, and we can obtain benefit of flexibility for future product, therefore it should be supported from perspective of product and we suggest the following changes: FL Proposal 3.1b: For 8TX UE uplink transmission, support 2 CW transmission for the following cases: - Case 1: when more than 4 layers are used., e.g., reuse DL CW to layer mapping for more than 4 layers - Case 2: STxMP at least for corresponding to multiple TRPs. - Case 3: when 2~4 layers are used. |
| Huawei, HiSilicon | We are fine with the proposal. |
| Intel | Same view as vivo. The proposal depends on whether more than 4 layers is supported, and it may not be needed in this meeting. |
| CATT | Fine with the proposal. |
| InterDigital | We are fine with Lenovo’s modified version. We are open to further study the cases for 2 CW Tx mentioned by ZTE. |
| Xiaomi | support |
| MediaTek | We share similar views as vivo, the outcome of 3.1a is needed before discussing layer mapping issue. And we would like to emphasise that there is different work item on multi panel UL with max of 4-layer support. We assume 8 TX discussed here is based on single panel transmission. |
| QC | We support Lenovo’s modified version. We need study >4 layers PUSCH performance before agreeing to support it. Regarding ZTE’s proposed case 2 and 3. We think STxMP should be discussed in other agenda. 2CW for 2-4 layers are out of scope. If RAN1 want to redesign layer mapping for 2~4 layers, fine, we have another proposal: for 2~4 layers, keep a single CW, while allowing different MCS for different layers of the single CW. |
| FL | @All: Thanks for your valuable inputs. Please find below the updated proposal based on majority view. FL Proposal 3.1b: For 8TX UE uplink transmission with more than 4 layers, (if supported), - support 2 CW transmission - reuse DL CW to layer mapping |
| Ericsson | Multiple codewords could have a high specification impact. We are open to studying it, and think that reusing DL CW to layer mapping is a sensible starting point if 2 CWs are to be supported, but it is far too early in the work to agree to specify in our view. |
| Nokia, NSB | Don’t think we can make a decision at the first meeting. We shall study more to evaluate the benefits of 2CW or more codewords for higher layer Tx. |
| Intel2 | Agree with Ericsson that multiple codewords may have big spec impact. In our understanding, one codeword can also work. But we are open to study the support of multiple codewords. Since this is the first meeting, we suggest the following change to the FL proposal. FL Proposal 3.1b: For 8TX UE uplink transmission with more than 4 layers, (if supported), - study whether to support 2 CW transmission - reuse DL CW to layer mapping if 2 CWs are used |
| Sharp | We support the FL proposal 3.1b in principle. |
| CMCC | Support the updated FL proposal 3.1b. |
| OPPO | We support FL proposal 3.1b. |
| CATT | OK with FL proposal 3.1b. |
| ZTE | Regarding to MediaTek and QC’s comments, in our minds there is no need to discuss 2CWs schemes in both AI 9.1.4.1 and 9.1.4.2, the spec impacts may be very aligned for the two cases. We suggest to discuss 2CWs for case 2 as above together with at least case 1 in this AI 9.1.4.2. |

# Codebook Design

Most companies believe that both codebook and non-codebook uplink transmissions should be supported for 8TX UEs (ZTE, Xiaomi, Samsung, LG, Lenovo, Apple, Sharp, MediaTek, Qualcomm, Ericsson). However, from the perspective of codebook design there seem to be two main lines of thought. A first group of companies believe that the codebook for 8 TX UE transmission should be based on legacy 2TX/4TX codebooks where by their concatenation and potential co-phasing, an 8TX codebook can be resulted (Huawei, ZTE, Apple, vivo, CATT, Xiaomi, Lenovo, Qualcomm). Alternatively, given the larger number of antennas in an 8 TX UE that may be place in a cross-polarized format, a second group of companies propose to reuse DL Type-I codebook (ZTE, OPPO, Spreadtrum, Apple, vivo, CATT, Xiaomi, Lenovo, NTT, NEC, Samsung, Nokia, MediaTek). Based on presented discussions and proposals by companies,

FL Proposal 3.2: For 8TX UE codebook-based uplink transmission, down-select one of the following alternatives,

- Alt1: Concatenation of legacy NR Rel-17 2TX/4TX codebooks

- Alt2: Reuse NR Rel-17 Type I codebook

- Alt3: Support either Alt1 or Alt2 according to a UE indication

Table 9

| Company | Views |
| --- | --- |
| Apple | Our understanding is that the codebook design also depends on the coherence assumptions on the 8Tx antenna configurations. In case of full coherency, either Alt 1 or Alt 2 could be candidate alternatives. But for partial coherency, Alt 2 may not be so suitable any more. Therefore, we think it is good to have separate considerations depending on coherency assumptions. Editorial suggestion: add “UL” in Alt 1, and add “DL” for Alt 2 for clarification. |
| LG | In our view, it is closely related to UE coherency. For example, Alt 1 is preferred for partial coherent codebook subset, while Alt2 is preferred for fully coherent codebook subset. |
| Samsung | Suggest to revise the wording, i.e., the design can be based on legacy codebooks. The exact codebook can include a subset of DL precoders or/and include additional components. Also, several companies have proposed a design based on selection of antenna groups. So, suggest to add two Alts 1-1 and 2-1 on the antenna groups. FL Proposal 3.2: For 8TX UE codebook-based uplink transmission, down-select one of the following alternatives, - Alt1: Based on Concatenation of legacy NR Rel-157 UL 2TX/4TX codebooks - Alt1-1: selection of Ng>=1 antenna groups + based on NR Rel.15 UL 2Tx/4Tx CB for the selected groups - Alt2: Based on Reuse NR Rel-157 DL Type I codebook - Alt2-1: selection of Ng>=1 antenna groups + based on NR Rel.15 DL Type I CB for the selected groups - Alt3: Support either Alt1 or Alt2 according to a UE indication |
| OPPO | We are fine with the proposal. |
| OPPO | We think Alt2 can be supported at least for full coherent codebook. If detail is needed, we propose Alt2-2 below: Alt 2-2: Reuse NR Rel-15 DL 8Tx Type I codebook with possibly reduced over-sampling factor. |
| NTT DOCOMO | We share similar view with Apple and LG that the codebook design is related to the UE antenna coherency. For example, Alt2 is suitable for full coherency UE as a starting point. Then, we suggest listing candidate alternatives for each coherency assumption. |
| CMCC | Support |
| Lenovo | Our preference is to list alternatives for codebook design in this meeting, and make decision on selection/down-selection of codebook design in the next meetings. As Apple pointed that the codebook design is more related with coherent assumption. So, it’s better to discuss the codebook design for different coherent assumptions. Also, since multiple companies proposed supporting frequency-selective precoding which impacts codebook design, we prefer it is emphasized as part of this proposal, e.g., in terms of FFS. Please find below a suggested updated proposal FL Proposal 3.2: For 8TX UE codebook-based uplink transmission, down-select (at least) one of the following alternatives for full-coherent UE in RAN1#110 meeting, - Alt1: Concatenation of legacy NR Rel-17 2TX/4TX codebooks - Alt2: Reuse NR Rel-17 Type I codebook - Alt3: Support either Alt1 or Alt2 according to a UE indication FFS: Support of sub-band granularity for codebook design |
| NEC | We prefer updated Alt 2 and Alt 2-1 from Samsung. In addition, as we discussed in our contribution, we think from WID wording, it’s a little ambiguous that whether codebook enhancement for 1~3 layers is needed for 8Tx, or only focused on >=4 layers? Do we need to clarify this or it’s common understanding that 1~3 layers should be enhanced? |
| Vivo | We are fine with the proposal, similar to comment from Apple alt1 and alt2 could be for different antenna coherent assumptions |
| Spreadtrum | Support FL proposal and we prefer Alt. 1 |
| ZTE | Support. Full coherent 8Tx could have the above 2 choices, concatenating legacy UL 2/4 Tx codebooks, or reusing DL 8Tx Type 1 codebook. And CSI codebook design for partial coherent 8Tx can also have the 2 choices, using legacy UL 2/4 Tx codebooks, or using DL 8Tx Type 1 codebook with zero/non-zero element patterns. |
| Huawei, HiSilicon | We prefer Alt 1 to design a unified 8Tx codebook structure for different coherence types. We agree that the codebook may depends on antenna configurations. For partial coherent antenna layout, Alt-1 is more suitable. And for full coherent antenna layout, further evaluations may be needed to compare between Alt-1 and Alt-2. |
| Intel | Fine with FL proposal in principle. We think Alt-1 is more generic solution. Details can be further discussed. |
| CATT | We are wondering whether the proposal is targeting to full-coherence, partial-coherence or non-coherence. We prefer to list candidate codebook designs for each coherent capability first, and then make the down selection. On codebook design for full-coherent, besides the listed alternatives, codebook designed by computer search also can be considered as a candidate, which is similar as the approach for codebook design for DFT-s-OFDM wave in Rel-15. On codebook design for partial-coherent, how many coherent groups are supported should be discussed. For each coherent group, the precoding matrices for 2Tx/4Tx in Rel-15 can be considered as a start. On codebook design for non-coherent, down selection of port-selection precoding matrices can be considered to avoid too large overhead of TPMI indication. |
| InterDigital | Support FL proposal, and we are also open to discuss other formulations mentioned by some other companies. We can put ‘at least’ (before ‘one of’) on the main sentence, as we may decide depending on the coherency condition. |
| Xiaomi | Same view with Samsung, Alt 1 is preferred for partial coherent codebook subset, while Alt2 is preferred for fully coherent codebook subset. |
| MediaTek | We are fine the Samsung modification for now. Our preference is to start with Alt2 as baseline. Alt2 is well studied, show consistent gains and has been used in the DL. So, a complete or subset CBs of Alt2 would reduce the burden of the standardization compared to Alt1. |
| QC | Similar view as other companies. The codebook design for 8 Tx needs to be separately discussed for full coherent and partial coherent. We suggest FL to reformulate the proposal to separate these two cases. Another comment on Alt 3: this is a direction we want to avoid. This basically double the standardization effort by specify two schemes with same functionality. We don’t want to take that route, given the workload of this AI is already very heavy. We prefer to design only one new codebook for full coherent 8 Tx. For Partial coherent 8 Tx, we prefer to reuse existing Rel-15 UL 2-Tx/4-Tx codebook and concatenate them to build partial coherent 8 Tx codebook. |
| FL | @All: Thanks for your valuable inputs. Please find below the updated proposal based on majority view. FL Proposal 3.2: For 8TX UE codebook-based uplink transmission, - down-select one of the following alternatives, - Alt1: Concatenation of legacy NR Rel-15 2TX/4TX UL codebooks - Alt2: Reuse of NR Rel-15 DL Type I codebook - FFS support of various antenna grouping based on UE coherency |
| Ericsson | We think a multiple TPMI design is also possible, and this should not be classified as concatenating existing codebooks. Having only Type 1 DL based codebooks can limit the ability to support different levels of partial coherence. Furthermore, Type 1 based TPMIs can be in a codebook with non-coherent or partially coherent TPMIs, as was done in the Rel-15 UL codebook. Lastly, we are not ready to specify any one type of scheme without further study. Overall, we prefer something more like: Study the following alternatives to support 8 Tx: - 8 port UL MIMO codebooks, including fully/partially/non-coherent operation and all full power modes. - Extending Rel-15 NCB-based operation to 8 layers - Indicating multiple Rel-15/16 TPMIs, each corresponding to an SRS resource |
| Nokia, NSB | We should start with a baseline design, such as Type-I codebook, but not necessarily “reuse” the whole thing. Because Type-I 8Tx codebook has different configurations, some of which may not fit into uplink scenario. We should have a baseline to start with (such as Type-I codebook with L=1); through detailed work we may pick/design certain configurations for uplink Tx. |
| Sharp | We share same view with Ericsson. Multiple TPMI design is possible. |
| OPPO | We propose to use DL type 1 codebook as the starting point/baseline. Other design, e.g. multiple TPMIs, is also possible if gain over type 1 codebook is observed. |
| CATT | We are open to discuss whether one or multiple codebooks are supported in Rel-18. The proposal is ambiguous. As commented by many companies, the codebook design for 8 Tx needs to be separately discussed for full-coherent, partial-coherent and non-coherent. |
| ZTE | Fine with the updated FL proposal. |

# SRS Enhancements

Support of 8-port SRS and some preliminary details related to its operation and configuration for both codebook- and non-codebook-based transmission have been discussed by several companies (Lenovo, Qualcomm, MediaTek, ZTE, Samsung, OPPO, LG, Apple, Intel, NTT, Intel, Lenovo,Xiaomi). One of the main issues that has been brought up by companies is related to the configuration of 8 SRS resources. In some views, SRS resource set configuration should be enhanced to support up to 8 resources. While in other proposed solutions, 8 SRS resources may be spread over several SRS resource sets. The deciding factor between these two approaches somehow may lie in UE antenna layout. Therefore, based on different companies’ views and proposals,

FL Proposal 3.3: For 8TX UE, at least for non-codebook-based uplink transmission, support of total 8-ports of SRS may be done by at least one of

- Alt1: SRS resource set with 8 single-port SRS resources

- Alt2: Multiple SRS resource sets with a total number of single-port SRS resources of 8

- Alt3: Support either Alt1 or Alt2 according to a UE indication

Table 10

| Company | Views |
| --- | --- |
| Apple | We are fine with the proposal on the high level. But for Alt 2, we feel this has some overlap with AI 9.1.4.1 (UL precoding indication for multi-panel transmission) assuming the intention is to allow simultaneous transmissions from the multiple panels. We would like some clarification on the logistics regarding which AI will handle such a design. |
| LG | Before discussing this issue, Proposal 3.1a should be determined first. |
| Samsung | We are OK to list alts, but we need to discuss the max #supported layers first (Proposal 3.1a). |
| OPPO | We prefer Alt1 which has minimal specification impact. For Alt2, the compatibility with other features may be an issue, e.g. uplink multiple panels transmission with two SRS sets, Rel-17 PUSCH repetition with two SRS sets. |
| NTT DOCOMO | Support the FL proposal and fine with adding ‘if up to 8 layers PUSCH transmission is supported’. |
| CMCC | We are fine with the proposal. But this issue is also discussed in agenda 9.1.3.2 (SRS), we are wondering which AI will handle this issue. |
| Lenovo | It seems this issue overlaps with 8Tx SRS in AI 9.1.3.2, however we see the moderator’s point that SRS enhancement discussion are key to moving forward with 8Tx UE discussion. We prefer if this is brought up to the chairman’s attention to decide how this overlap can be resolved |
| NEC | We prefer Alt 1. |
| Vivo | We are fine with the proposal, there could be overlapping discussion in STxMP agenda |
| Spreadtrum | We prefer Alt 1. |
| ZTE | SRS enhancements should comprise both codebook based and non codebook based transmission. This proposal only discusses non codebook case. We are generally fine with it for non codebook case. We also agree with Apple that it should be clarified which AI should cover the SRS design, it seems AI 9.1.3.2 SRS enhancement targeting TDD CJT and 8 TX operation is much more suitable in our initial thought. |
| Huawei, HiSilicon | We prefer Alt1 for minimal specification impact, and the benefit of Alt2 is unclear to us. In addition, we are open to discuss whether to allow max 2 SRS ports per SRS resource. |
| Intel | Generally fine with FL proposal. Since it’s about SRS configuration, we think it’s suitable to discuss in this AI. |
| CATT | We prefer Alt 1. Maybe this issue should be discussed in AI 9.1.3.2. |
| InterDigital | Support the FL proposal with adding the condition mentioned by DOCOMO. |
| Xiaomi | Support the FL proposal |
| MediaTek | We are fine with proposal in principle. |
| QC | We are fine with FL proposal. |
| FL | @All: Thanks for your valuable inputs. - As noted by some of colleagues, this topic may have an overlap with AI 9.1.3.2. - This is being discussed with FL for AI 9.1.3.2. Please stay tuned for the update. |
| Ericsson | It is too early to agree to support any one of the alternatives. We are OK to further study Alt 1 and Alt 2. Alt 3 seems to assume that both Alt 1 and Alt 2 are needed and are separate modes of operation. So we don’t see the need to list Alt 3 at this time. |
| Nokia, NSB | Before we agree on this proposal, we need to have a discussion on whether to extend non-codebook based UL to 8Tx. Need to find some scenarios that could provide better performance for 8Tx codebook-based UL Tx. |
| Sharp | We think this issue should be discussed after Proposal 3.1a. |
| OPPO | We are fine to discuss it in 9.1.3.2 firstly and then decide whether enhancement on SRI is needed. |

# TPMI/SRI Indication

TPMI/SRI indication for codebook-based transmission requires additional details that will become available after completing codebook design. Hence, it has been proposed to visit this discussion after finalizing the design of the 8TX codebook (CATT). As for SRI indication for non-codebook-based transmission, several proposals have been put forward. A first group of companies propose to reuse the recently introduced two SRI fields for uplink multi-TRP PUSCH transmission (vivo, LG, Intel). However, other companies (Samsung, Xiaomi) propose to use a bitmap approach for SRI indication. Further, an extension of existing SRI field is proposed by (CATT). Further discussion on this topic will be beneficial.

FL Proposal 3.4: TBD based on collecting further inputs from companies.

Table 11

| Company | Views |
| --- | --- |
| Apple | For TPMI, it definitely depends on the codebook design, and it will be better to wait until we get some clarity on the codebook (or this can be discussed together with codebook design). For SRI, it is related to P3.3, and the design would be different depending on which alternatives we go with. It seems difficult to have completely independent discussions. |
| LG | It can be discussed with codebook design. |
| Samsung | Agree, this can be discussed later. But, we can perhaps list alternatives for further study |
| OPPO | It can be depended on codebook design. |
| NTT DOCOMO | For CB-based PUSCH, the TPMI and SRI indication depends on the codebook design. For non-CB-based PUSCH, the SRI indication depends on the selected option for 3.3. |
| CMCC | TPMI can be discussed after the codebook design. SRI for non-codebook can be discussed after the decision of number of SRS resource sets for non-codebook. |
| Lenovo | Agree with Apple, discussion on TPMI is strongly related to codebook design. Similarly for SRI indication for nCB PUSCH, discussion is also dependent on 8Tx SRS enhancement. Prefer to defer discussion on this proposal to future meetings |
| NEC | This can be discussed later. |
| Vivo | It can be discussed after some clarity in codebook design, for non-codebook 2 SRI fields is straight forward with less spec impact, and the design can be unified with STxMP as well. |
| Spreadtrum | 8 Tx codebook design should be discussed firstly. |
| ZTE | Details may depend on codebook design, and SRS enhancement. TPMI/SRI scheme can be either one indication based, or multiple indication based. For one indication based scheme, one TPMI indication could indicate one or more precoder, e.g., joint TPMIs indication. For multiple indication based scheme, each TPMI indication indicates one precoder. |
| Huawei, HISilicon | For CB, TPMI overhead is an important metric in 4TX codebook design. It’s even more important for 8TX considering the enlarged codebook, potentially more CWs. We think we can consider other indication scheme with low overhead and high resolution, such as beamformed CSI-RS. Specifically, TRP transmits a beamformed CSI-RS with precoder WDL, then UE can receive the CSI-RS as a vector of PUL as PUL=HWDL By choosing the precoder WDL of CSI-RS, the selected codeword PUL can be indicated to UE with high precision. Because the CSI-RS overhead is not related to the number of codewords, we can have a high-resolution codebook with less overhead. For SRI in NCB, the benefit of bitmap approach is unclear to us, and we think SRI overhead for 8TX should be considered for further enhancement. |
| Intel | Similar view as Apple. The TPMI indication depends on the codebook design. |
| CATT | The TPMI/SRI indication of UL 8 Tx for codebook based PUSCH is highly related to the design of codebook and SRS, as well as the presentation way of the codebook. Therefore we prefer to discuss the issue after the codebook and SRS design are available. |
| Xiaomi | This discussion can be deferred. |
| MediaTek | TPMI and SRI can be discussed after the codebook design and number of SRS resource sets is discussed. |
| Ericsson | SRI design enhancement for non-codebook based transmission is related to the SRS resource design alternatives as Apple points out. Our general feeling is extensions of NCB to 8 layers should not be over optimized with respect to SRI overhead. Scenarios where Ues support more than 4 layers are not likely to be PDCCH limited. |
| Nokia, NSB | This issue should be discussed after 8Tx CB and 8Tx non-codebook scheme is defined. |
| Sharp | This issue should be discussed after clarification of codebook design. |

# Full Power Transmission

Several companies have expressed their support for full power uplink transmission by extending Rel-16 mechanism (IDC, CATT, CMCC, Ericsson, Qualcomm, NTT, ZTE, Samsung, LG, vivo). Clearly, the actual specifications discussion will be held after completion of codebook design. Two companies (Apple, Intel) have stated that support of full power operation should be based on using advance Ues that are equipped with full-rated PAs on all TX chains. Therefore, based on different companies’ views and proposals,

FL Proposal 3.5: Support full power transmission for 8TX Ues.

- Details are FFS upon completion of codebook design

Table 12

| Company | Views |
| --- | --- |
| Apple | Yes, this can be discussed after the codebook design. But more importantly it depends on the targeted scenarios/UE devices. Instead of having such a high level proposal, it would be good to understand the different use cases and the types of UE devices, in order to understand which types of full power Ues may be more practical to consider. We should avoid unnecessarily defining multiple types of full power Ues just because certain modes exist already. This discussion does not have dependency on the codebook design and can be carried out in parallel. |
| LG | Support |
| Samsung | We prefer to discuss this issue after 8Tx codebook is designed. |
| OPPO | Support. |
| NTT DOCOMO | Support. |
| CMCC | Support. |
| Lenovo | Support to discuss this feature when codebook design is ready since it’s more related with the codebook and PA architecture. |
| NEC | This can be discussed later. |
| Vivo | Support the proposal |
| ZTE | Agree. Full power mode 0, 1, and 2 can all be considered similar to Rel-16. But for full power mode 2, if legacy way is reused, the report from UE to gNB on which TPMI supports full power may lead to a big workload, e.g., defining a even bigger table to reflect several hundreds TPMIs for 8Tx. This issue may need to be considered. |
| Huawei, HiSilicon | We are fine with the proposal. |
| Intel | Fine with FL proposal. |
| CATT | Support the proposal in principle. We think not all details are upon completion of codebook design. For example, whether full power mode 1 should be supported or not may be not related to codebook design. We are ok to FFS which full power transmission mode(s) are supported and the details of the design. |
| InterDigital | Support the FL proposal. |
| Xiaomi | Fine with the proposal |
| MediaTek | Support |
| QC | Support FL proposal. |
| Ericsson | We agree that full power transmission is an essential aspect of UL MIMO designs. Our hope is that existing full power schemes can be reused or slightly extended to support 8 Tx. On the other hand, it seems too early to agree to support them, since we do not have a corresponding precoding design. Our suggestion would be: - Include full power transmission in the evaluation of 8 Tx TPMI/SRI enhancements. |
| Nokia, NSB | The proposal is fine because full Tx power should be supported. |
| Intel2 | For full power Mode 0, it doesn’t have any dependency on the codebook and TPMI design. Maybe we can start with full power Mode 0? |
| Sharp | We support the FL proposal. |

# Other Issues

Some other issues have also brought up by companies, including support of frequency selective precoding and details related to PTRS-DMRS association indication when transmission rank is greater than 4.

Table 13

| Issue | Supporting Companies |
| --- | --- |
| Frequency selective precoding for 8 TX UE | Huawei/HiSilicon, Lenovo, Samsung, Nokia, NSB |
| PTRS-DMRS association indication for 8TX UE | vivo, NTT, Qualcomm, ZTE,Xiaomi |

# Feature-lead Proposals for Approval

# Round1

FL Proposal 2.2.1: Adopt the following Table as the reference EVM for SLS evaluation. - Companies may provide additional evaluation results per their case of interest. Parameter; Value. Frequency range; 3.5 GHz. Multiple access; OFDMA. Numerology; 14 OFDM symbol slot SCS 15 KHz , 30 KHz. Scenario; 38.901 -UMi (ISD = 200 m) -UMa (ISD = 500 m) -80% Indoor (3Km/h), 20% Outdoor (30km/h) -Indoor industrial 38.802 -Dense urban (ISD =200m), (per Table A.2.1-1). Channel model; 38.901. System bandwidth; 20 MHz, 100 MHz. gNB RX antenna setup and port layouts (𝑀,𝑁,𝑃,𝑀𝑔,𝑁𝑔,𝑀𝑝,𝑁𝑝); (2,2,2,1,1,2,2) with (dH , dV ) = (0.5, 0.5)λ (4,4,2,1,1,4,4) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆. gNB receiver noise figure; 5dB. gNB receiver; MMSE-IRC. gNB scheduler; Single user with proportional fair. Modulation; Up to 64 QAM , Up to 256QAM. MIMO scheme; SU-MIMO with rank adaptation. UE speed; 3 Km/h, 30Km/h. UE TX antenna configuration; To be defined according to outcome of Proposal 2.1. Traffic model; FTP model 1, packet size 500KB/s, (RU 20%, 50%, 70%) Full buffer (optional). Codebook for benchmarking; R15 UL 4-Tx codebook , Eigen-based (Non-codebook ), companies report PRG assumption. Precoder granularity; Wideband. Power control; Open loop, -alpha = 0.8 -P0 = -50, -80 dBm to be selected according to the deployment scenario. UE power rating; 23 dBm (indoor) 32 dBm (outdoor). Metric; UL mean-user throughput, 5%-ile and 95%-ile UPT. FL Proposal 2.2.2: Adopt the following Table as a starting point for the reference EVM for LLS evaluation - Companies may provide additional evaluation results per their case of interest - Further revisions may be considered in RAN1#109, if needed Parameter; Value. Carrier Frequency; 3.5 GHz. Waveform; CP-OFDM. SCS; 15 KHz , 30 KHz. System bandwidth; 20 MHz, 100 MHz. Scheduled PRBs; 5, 25, 50, 260 PRBs. gNB RX antenna setup and port layouts (𝑀,𝑁,𝑃,𝑀𝑔,𝑁𝑔,𝑀𝑝,𝑁𝑝); (4,4,2,1,1,4,4) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 (2,2,2,1,1,2,2) with (dH , dV ) = (0.5, 0.5)λ. UE TX antenna configuration; To be defined according to outcome of Proposal 2.1. UE speed; 3 Km/h. Layer; Fixed, Adaptive rank. AMC; Fixed, Adaptive On. DMRS configuration; Type 1. Channel estimation; Real. Channel Model; TDL -A (30ns), TDL -C (300ns), CDL -A (30ns), CDL -C (300ns). FL Proposal 2.3: Study fully-coherent, partially-coherent and non-coherent UEs for uplink transmission with 8TX UEs . FL Proposal 3.5: Study full power transmission for 8TX UEs . - Details are FFS upon completion of codebook design

# List of Companies’ Proposals

| Huawei | Proposal 1: SRI enhancement with overhead reduction to enable 8TX NCB based UL transmission should be supported. Proposal 2: UL 8TX codebook design should consider various antenna layouts so as to accommodate different UE implementations. Proposal 3：Frequency selective precoding should be supported for 8TX UL transmission. Proposal 4：Block-wise codebook based on legacy 2TX or 4TX UL codebook should be supported to enable 8TX in CB based UL transmission. Proposal 5: The beamformed CSI-RS should be considered to indicate UL precoders to UE. |
| --- | --- |
| ZTE | Proposal 1: Regarding 8 Tx-UL operation in Rel-18, support 8-TX and more than 4 layers UL transmission. Proposal 2: Both codebook based and non-codebook based UL transmission should be supported in Rel-18 for UL MIMO. Proposal 3: Full coherent and partial coherent capabilities should be supported, and non coherent capability can be precluded or deprioritized in Rel-18 for UL MIMO. Proposal 4: On 8-Tx UL transmission enhancement, 2 CWs for UL transmission should be supported. - Condition of enabling >1 CWs for UL transmission can be further studied in RAN1, e.g., if the number of TX(s) and the number of UL layers exceed threshold(s). Proposal 5: The legacy full power mode 1 and 2 should be supported in Rel-18 for UL MIMO, but details can be discussed after codebook design is clear. Proposal 6: Antenna layout assumption for partial coherent codebook should be determined before codebook design. Proposal 7: Regarding 8-Tx full coherent codebook design, the following schemes can be considered - Scheme 1: reusing DL type I 8Tx codebook scheme - Scheme 2: UL 4Tx codebook + additional phase Proposal 8: Regarding 8-Tx partial coherent codebook design, the following schemes can be considered - Scheme 1: 8-port full coherent codebook indication + puncture pattern indication (indicating zero-power elements in a matrix) - Scheme 2: Multiple port groups, with common or separate precoding information indication Proposal 9: Regarding non codebook based transmission design for 8-Tx, - The number of SRS resources in an SRS set can be up to 8 - Potential optimization for SRI re-design considering DCI overhead, e.g., 8 bits or less Proposal 10: Regarding CW-to-layer mapping, support up to 2 CWs for UL 8-Tx transmission, and then further study the following aspects: - Scrambling, layer mapping, and DCI format to support second TB - Code rate and TB size when UCI is transmitted on PUSCH |
| Spreadtrum | Proposal 1：Study the potential methods to reduce DCI overhead, e.g. limit the selection of SRS resource for part of layers. Proposal 2: The following codebook design principle for 8 antenna ports can be considered: - Option 1: based on LTE householder matrix - Option 2: based on NR DL codebook for 8-layer transmission - Option 3: other possible combination of existing precoding matrices |
| InterDigital | Proposal 1: For enabling 8 Tx UL for UEs with many antennas, it should be discussed to use single or dual codeword based on considered scenarios, UE types, coherency types, and so forth. Proposal 2: Consider UE to report its capabilities on a supported type of antenna/panel structure or virtualization capability across UE antenna ports such as SRS antenna ports, enabling up to 8 Tx UL. Proposal 3: Support to retain the full power transmission mode of operation with necessary enhancements to be also applicable for the new enhanced UL-MIMO transmission case supporting up to 8-Tx UL. |
| CATT | Proposal 1: Support UL 8Tx with up to 8 layers in Rel-18. Proposal 2: Codebook designs for non-coherent, partial-coherent and full-coherent devices are considered for UL 8Tx. Proposal 3: For UL 8Tx with DFT-s-OFDM, precoding matrix in Table 1 is included in non-coherent codebook. Proposal 4: For PUSCH transmission with CP-OFDM, down selection of port-selection precoding matrix shall be considered for non-coherent codebook for 8Tx . Proposal 5: On the design of partial-coherent codebook(s) for UL 8Tx, one or more typical partial-coherent relationships of antennas are considered -Two coherent groups with four coherent antennas per group, and four coherent groups with two coherent antennas per group can be considered with high priority. Proposal 6: On the design of precoding matrices for partial-coherent transmission for UL 8Tx, the precoding matrices for 2Tx/4Tx in Rel-15 can be considered for each coherent antenna group as a baseline. Proposal 7: Full-coherent precoding matrix for UL 8Tx for DFT-s-OFDM can be designed by computer search with good distance properties. Proposal 8: For the design of full-coherent precoding matrices for UL 8Tx for CP-OFDM, down selecting precoding matrices from DL Type I codebook can be considered as a start. Proposal 9: On full-coherent codebook design for UL 8Tx for CP-OFDM, supporting multiple codebooks for various types of antenna structures can be considered. Proposal 10: For UL 8Tx, the size of codebook should be carefully determined. Proposal 11: UL full power transmission is supported for UL 8Tx, -Whether all of UL full power transmission schemes in Rel-16 are supported can be considered. Proposal 12: For UL 8Tx for codebook based PUSCH, TPMI/SRI indication is designed after the codebook and SRS design are available. Proposal 13: For UL 8Tx for non-codebook based PUSCH, extending the bit width of current SRI field to enable indicating up to 8 SRS resources. |
| vivo | Proposal 1: The supported scenarios and antenna assumptions should be settled in the very first meeting for the group to work in focused manner. Proposal 2: Two SRI fields corresponding to two SRS resource sets for non-codebook transmission can be considered to simplify the SRI indication. Proposal 3: Codebook constructed by two 4Tx precoders indicated by two TPMI fields can be considered for partial and none coherent antenna assumption. Proposal 4: DL type1 codebook can be considered for fully coherent antenna assumption. Proposal 5: Two CWs can be considered for rank>4. Proposal 6: Following issues should be further discussed: -PTRS-DMRS association indication when rank>4 - Impact on full power modes |
| NEC | Proposal 1: Clarifying whether 1-3 layer transmission is based on enhancements with 8Tx or not. 2 Proposal 2: From UE perspective, reporting capability of full, partial and non coherent is sufficient. Proposal 3: Type I single-panel codebook is reused for uplink full coherent codebook enhancement. Proposal 4: Overhead reduction for partial and non coherent codebook should be studied. |
| Sony | Proposal 1: Enhanced TPMI indication with finer precoding information can be considered for UE UL transmission enhancements. Proposal 2: Enhanced TPMI indication with finer precoding information can also be considered for UE UL transmission enhancements in multi TRP scenario. |
| Xiaomi | Proposal 1: Support 8Tx with more than 4 transmission layers for the UL. Proposal 2: For the PUSCH, the number of antenna ports should be extended to 8 accordingly. Proposal 3: Support both codebook based UL transmission and non-codebook based UL transmission. Proposal 4: 2 codewords should be supported for up to 8 layers of uplink transmission. Proposal 5: The supported 2 codeword transmission scheme can be enabled when more than X transmission layers is configured, X is up to UE capability. Proposal 6: For non-codebook based PUSCH transmission with 8Tx, SRI indicated by the bitmap of the SRS resources or the ports of the SRS resource configured for the SRS resource set is preferred for the simplicity without the effort on the design of new SRI tables. Proposal 7: Three coherence types of codewords should be considered in Rel-18 UL 8Tx codebook design, i.e., full-coherent, partial-coherent, and non-coherent codewords. Proposal 8: 8Tx full-coherent codewords can be designed on the basis of Rel-15 DL Type I single-panel 8Tx codebook with selected parameters. Proposal 9: For partial-coherent codewords, the antenna ports can be divided into two or more antenna port groups. Proposal 10: 8Tx partial-coherent codewords can be designed on the basis of 8Tx full-coherent codewords. Proposal 11: 8Tx partial-coherent codewords can be designed on the basis of Rel-15 UL 4Tx full-coherent codewords. Proposal 12: The scaling factor can be computed as sqrt(1⁄K) where K denotes the number of non-zero elements in 8Tx codewords. 5/8 Proposal 13: The research on DFT-based multi-panel codebook can be left for FFS. Some other pre-designed orthogonal codebooks other than DFT-based codebook can also be left for FFS. |
| Samsung | Proposal 1: for the study of 8 Tx UL operations, -Prioritize one device type for efficient study and discussions - -The one device should be the one which is ‘most-likely’ to have 8 Tx antennae for UL operations - -1st priority: CPE - -2nd priority: FWA Proposal 2: Regarding the transmission scheme for 8Tx UL operations, -Support both CB and NCB based UL transmission - -For RAN1 work, adopt a serialized approach, -prioritize CB based discussion until RAN1#110 -NCB based discussion starts after RAN1#110 Proposal 3: regarding the 8 Tx antenna structure, study the following aspects -Polarization: co-pol or dual-pol -Antenna groups: one group or multiple groups - -Antenna panels and coherence types: -Single panel: one antenna group and full-coherence -Multi-panel: multiple antenna groups and partial-/non-coherence Proposal 4: support a single unified 8Tx codebook structure for different coherence types (i.e. FC, PC, and NC) based on antenna groups -Antennae within a group are coherent -Antennae across multiple groups are non-coherent Proposal 5: regarding the 8Tx UL codebook, -The baseline is Rel. 15 NR 8Tx DL Type I codebook - -Support the following parameters for 8Tx UL codebook -𝑃= number of antenna polarization; 𝑃=1 for co-pol, and 𝑃=2 for dual-pol. -𝑀𝑔= number of antenna groups. -𝑁1= Number of antennae in 1st dimension. -𝑁2= Number of antennae in 2nd dimension. Proposal 6: investigate both single-stage and dual-stage codebook structures Proposal 7: Discussion on full power modes can be start after the 8Tx codebook is designed Proposal 8: regarding 8Tx NCB based UL transmission, -Support number of SRS resources (𝑁𝑆𝑅𝑆) up to 8 -Support both one SRS resource set and two SRS resource sets -When 𝑁𝑆𝑅𝑆≤4, the SRI indication follows legacy (Rel.15) scheme, and -When 𝑁𝑆𝑅𝑆>4, -Study the need for 𝐿𝑚𝑎𝑥>4 -study the following SRI indication schemes -Alt1: combinatorial index scheme -Alt2: bitmap based scheme Proposal 9: for STx2P, support both -Case 1 (1 PUSCH): one SRI indicating a pair of SRS resources (e.g. STx2P to sTRP) -Case 2 (2 PUSCHs): two SRIs, each indicating a SRS resource for a TRP (e.g. STx2P to mTRP) Proposal 10: regarding max number of layers -prioritize the RAN1 work for max 4 layers - >4 layers can be supported, if its need and use cases can be identified Proposal 11: for the study of 8Tx UL operations, support EVM assumptions provided in Table 1. |
| OPPO | Proposal 1: Limited antenna pattern(s) should be confirmed for evaluation of 8Tx uplink transmission. Proposal 2: Support full coherent and partial coherent codebook for 8 Tx uplink. - - For full-coherent codebook with uniform linear antenna array, DL 8Tx Type 1 CB (wideband beam and co-phasing) as baseline with possible beam reduction. o FFS for other antenna array, e.g. clustered or four-sided antenna array - - For partial-coherent codebook, only consider intra-polarization coherence (2Tx) similar to Rel-15. - - Non-coherent codebook has low priority considering practical antenna pattern and signaling overhead. Proposal 3: Consider separate indication of TRI and TPMI for 8 Tx uplink. Proposal 4: Introduce SRI enhancement to indicate up to 8 SRS resources for non-codebook uplink transmission, considering signaling overhead and standardization complexity. |
| LG | Proposal 1: Support both codebook and non-codebook based 8Tx UL transmission in Rel-18 MIMO. Proposal 2: Support up to 8 layers for 8Tx UL transmission in Rel-18 MIMO, and reuse the same codeword-to-layer mapping used in DL transmission. Proposal 3: Support full power 8Tx UL Transmission, and the details can be further discussed after finishing 8 Tx UL codebook design. Proposal 4: Support two-level partial coherency for codebook based 8Tx UL transmission. -Level-1: 4-pair 2-Tx coherency - Level-2: 2-pair 4-Tx coherency Proposal 5: Adopt Table 4 for rank 1 8 Tx codebook for CP-OFDM. Proposal 6: Adopt Table 5 for rank 1 8 Tx codebook for DFT-s-OFDM. Proposal 7: Consider following alternatives for enabling 8Tx non-codebook based UL Transmission. -Alt1. Increase # of SRS resource from 4 to 8. - Alt2. Allow max 2 SRS ports per SRS resource - Alt3. Reuse Rel-17 S-DCI based M-TRP PUSCH mechanism |
| Lenovo | Proposal 1: Both codebook and non-codebook based PUSCH should be supported for 8Tx UL operation Proposal 2: Evaluate 8Tx UL operation for both Rank = 4 and Rank > 4. Rank > 4 is supported only if the potential gains exceed the complexity corresponding to rank upgrade. Proposal 3: Support of less than rank 4 is not precluded for 8Tx UL operation. Proposal 4: Study codeword-to-layer mapping for 8Tx UL operation if Rank>4 is supported Proposal 5: To support 8Tx UL transmission, on the SRS configuration, -One or two SRS resources with 8 SRS ports can be configured in the SRS resource set for CB when codebook based UL transmission is configured, and -Up to 8 SRS resources with single port can be configured in the SRS resource set for nCB when non-codebook based UL transmission is configured. Proposal 6: Prioritize full coherence and partial coherent UE capability for 8Tx UL operation. Proposal 7: For full coherent single panel UE, the antenna configurations illustrated in Figure 1 are considered for evaluation of 8Tx UL operation. Proposal 8: For multi-panel UE, the antenna configurations illustrated in Figure 2 are considered for evaluation of 8Tx UL operation. Proposal 9: Study different codebook design alternatives for 8Tx UL operation. Proposal 10: Dual-stage codebook should be adopted for 8Tx UE and Rel-15 DL Type-I CSI codebook for 8 ports can be adopted as a starting point. Proposal 11: TPMI signaling overhead is considered as a performance metric when studying different alternatives for 8Tx UL codebook design. Proposal 12: Study the performance benefits, signaling overhead and specification impact of supporting frequency-selective precoding for 8Tx UE. |
| Apple | Proposal 1: Consider at least full coherent and partial coherent antenna configurations, and use the antenna configurations in Figure 1 as the baseline. -FFS non-coherent antenna configurations Proposal 2: Consider both codebook based and non-codebook based UL transmission schemes for 8 Tx UL operation. Proposal 3: For codebook based transmission scheme with full coherent antenna configuration, use the DL Type I codebook design for 8 Tx as the starting point for 8 Tx UL. Proposal 4: If separate beam indication is to be considered for the partial coherent antenna configurations, clarify whether this should be further discussed in this AI or merged into the AI on simultaneous multi-panel UL transmission. Proposal 5: For codebook based transmission with partial coherent antenna configuration, use the legacy codebook for 4 Tx UL as the starting point for the per-panel precoding. Proposal 6: If full power mode is to be considered, advanced UEs similar to a UE that supports ul-FullPwrMode-r16 in Rel-16 (with all full-rated PAs) should be assumed to simplify the design. Proposal 7: For non-codebook based transmission, consider SRI enhancements to support up to 8 layers. Potential overhead reduction for SRI field can be considered. |
| CMCC | Proposal 1: All the 1-8 layers UL transmission should be considered for 8 TX UL transmission. Proposal 2: SRI field in Rel-15 can be reused for codebook based 8 TX UL transmission. Proposal 3: How to indicate up to 8 transmission rank and corresponding PUSCH precoder without increasing the SRI overhead for non-codebook UL transmission can be further studied. Proposal 4: The supported configurations of (N1, N2) for 8 TX UE can be N1=N2=2 or N1=4, N2=1 with the consideration of dual polarization, and the supported configurations of over sampling factor (O1, O2) can be further discussed. Proposal 5: Enable 2 CWs with individual MCS, RV and NDI for 8 TX UL transmission can be further studied. Proposal 6: Full TX power enhancement for enabling 8 TX UL transmission should be discussed for fullpower, fullpowerMode1, and fullpowerMode2. |
| NTT DOCOMO | Proposal 1: Support 8TX UL transmission with up to 8 layers per UE. Proposal 2: -Discuss 8TX UE antenna layout and identify the important UE antenna layout in commercial NW. -Support UE capability reporting of 8TX UE antenna layout. -Rel-18 design should be able to applicable to any 8TX UE antenna layout. Proposal 3: -Support two codewords for PUSCH transmission for more than 4 layers. Following enhancements can be further discussed. - codeword-to-layer mapping for more than 4 layers for spatial multiplexing - DCI enhancement with codeword-specific indications of MCS, NDI, and RV - UCI multiplexing on two codewords PUSCH -Support two codewords for simultaneous multi-panel UL transmission for less than 4 layers. The design on support of two codewords PUSCH is common and discussed jointly for 8TX UL transmission and multi-panel UE transmission. Proposal 4: -Support antenna port indication enhancement for UL rank from 5 to 8. -Support enhanced PTRS-DMRS association for more than 4 scheduled DMRS ports. Proposal 5: -Support codebook-based PUSCH transmission for more than 4 layers. -Support to define new UL 8TX codebook, which should consider non-coherent, partial-coherent, and full-coherent UEs, and applicable to any 8TX UE antenna layout. -Support UE capability reporting of coherent ports for partial-coherent UE. -Support NR DL Type 1 single-panel codebook as starting point for UL full-coherent precoders. -The number of supported non-coherent, partial-coherent, and full-coherent precoders should be carefully considered to reduce the DCI indication overhead. -Support full power transmission enhancements for non-coherent and partial-coherent precoders. Proposal 6: Support 8-port SRS resource with usage of ‘codebook’. Proposal 7: -Support non-codebook-based PUSCH transmission for more than 4 layers. -Support configuration of eight 1-port SRS resources in an SRS resource set with usage of ‘noncodebook’. -Support SRI indication enhancement for non-codebook-based PUSCH for 8 SRS resources configured with usage of ‘noncodebook’ for maximum UL rank from 1 to 8. Proposal 8: Support to discuss SRS enhancement for antenna switching for 8T8R. |
| Sharp | Proposal 1: 8 Tx UL operation should be introduced in Rel-18. Proposal 2: More than 4 layers should be supported in Rel-18. Proposal 3: The number of layers should be 5, 6, 7 and 8 for study purpose in Rel-18. |
| Nokia | Proposal 1: Study NR DL 8Tx codebooks for uplink transmission, with consideration of uplink antenna implementations for CPE/FWA/vehicle/Industrial devices. Proposal 2: Consider 8Tx codebook design with azimuth/elevation antenna port configuration (𝑵𝟏,)=(𝟒,𝟏) of Type-I 8Tx codebook. Proposal 3: RAN1 shall discuss on whether 8Tx multi-panel codebook should be studied in Rel-18 under Objective 5. Proposal 4: RAN1 shall study system level performance for 8Tx codebook design with frequency selective precoding. Proposal 5: RAN1 shall agree on simulation assumptions for system-level evaluation for 8Tx performance. Proposal 6: Consider a common DCI format for sub-band precoder indication for various UL resource allocation sizes. Proposal 7: Consider to group coherence antenna ports to support the indication of 8Tx coherence levels. |
| Mediatek | Proposal 1: Support both Codebook and Non-Codebook-based UL Transmission precoding for 8 TX UL. Proposal 2: Study and if justified, support more than 4 transmission layers. Proposal 3: Study, and if justified, support two codewords for more than 4-layer transmission, where codeword-to-layer mapping follows its DL equivalent defined in 3GPP TS 38.211 Section 7.3.1.3. Proposal 4: For the evaluations of 8 TX UL, 8 Rx antennas should be considered for the baseline evaluation. Proposal 5: Support uniform antenna arrays for UL 8 TX codebook, with 4x1 and 2 antenna configurations. Proposal 6: Study the effect of precoding accuracy as a function of the transmission rank, and if justified, design the precoder CB with variable accuracy at different ranks. Proposal 7: Specify the 8 TX UL precoder codebook as a subset of (or identical to) the 8-TX DL Type 1-SP precoder codebook. Proposal 8: With Release 15 Type I SP CB as a baseline design for 8 TX UL CB, subsets of Type I CB can be chosen to help reduce PMI overhead through the following techniques: -Lower DFT oversampling factors (which can be rank-dependent). -Smaller number of beam co-phasing choices. -FFS: Other schemes Proposal 9: For Type I SP based UL precoder, study and if justified specify a two-stage PMI feedback to limit feedback overhead, where -Slow-changing PMI elements are grouped in one stage with low feedback frequency -Fast-changing PMI elements are grouped in the other stage with higher feedback frequency -Information from both PMI feedback stages construct a complete PMI. Proposal 10: Study and if justified specify an 8 TX UL precoder codebook using linear combination coefficients of DFT beams. -Example: A subset of the Type II DL CB. Proposal 11: For 8 TX UEs, partially coherent antenna groups should be uniformly divided into one or more of the following: -Two groups of 4 coherent ports each -Four groups of 2 coherent groups each Proposal 12: For Partially coherent 8TX antenna ports, each antenna group of size M should be designed such that: -The port group exclusively transmits up to M layers, which are not transmitted by the other port groups. -The precoding weights for those port in the focus group constitute a sub-matrix of the full precoder matrix with size M (where L is the number of layers and ) -The submatrix is populated using wither (i) the legacy coherent precoders for M TX ports with L layers, or (ii) alternative precoder design. Proposal 13: Support non-coherent antenna ports for 8 TX UEs. Proposal 14: For SRS Resource Sets configured for non-codebook usage, support up to 8 SRS Resources. Proposal 15: UL 8 TX to be evaluated under low to moderate UE velocity scenarios as 8 TX is aimed for relatively static CPE devices. Study Dense Urban with ISD=200m and UMa with ISD=500m to evaluate scenarios under larger ISD. |
| Intel | Proposal 1: RAN1 to consider full coherence UE as starting point for UL transmission with 8Tx Proposal 2: RAN1 to consider full power Mode 0 as starting point for codebook-based UL transmission with 8Tx Proposal 3: RAN1 to discuss the codebook construction for codebook-based UL transmission with 8Tx Proposal 4: RAN1 to consider 4 layers as starting point for codebook-based UL transmission with 8Tx Proposal 5: RAN1 to discuss the SRS configuration for codebook-based UL transmission with 8Tx Proposal 6: RAN1 to discuss the TPMI indication enhancement for codebook-based UL transmission with 8Tx Proposal 7: For non-codebook based transmission in Rel-18, RAN1 to discuss the SRS configuration, i.e., one SRS resource set vs. two SRS resource sets Proposal 8: For non-codebook based transmission in Rel-18, RAN1 to discuss the SRI field enhancement, i.e., one SRI field vs. two SRI fields |
| Ericsson | Proposal 1: Define a limited set of deployment scenarios for Rel-18 UL MIMO relevant to market needs. -Use the scenarios given in [2] as a starting point. -Down select among FWA/CPE, vehicular, and industrial scenarios if possible. -Use realistic UE transmit chains and antenna implementations. Proposal 2: Techniques are compared primarily using system-level simulation studies. -Net benefits of Rel-18 UL MIMO designs are evaluated considering UE and gNB antenna directionality and SINR conditions over the cells. -Link level simulations can additionally be used e.g. for impairments such as channel estimation -Evaluations are used to determine which Rel-15/16/17 UL MIMO modes should be enhanced. Proposal 3: Study the following alternatives to support 8 Tx -8 port UL MIMO codebooks, including fully/partially/non-coherent operation and all full power modes. -Extending Rel-15 NCB-based operation to 8 layers -Indicating multiple Rel-15/16 TPMIs, each corresponding to an SRS resource |
| Qualcomm | Proposal 1: Study the performance of 8 Tx PUSCH via SLS. Prioritize 8Tx CPE/FWA devices in this study. Proposal 2: Adopt the following SLS assumptions as a starting point to study the performance of 8 Tx PUSCH. Parameter; Value. Metric; UL throughput. Baseline; Rel-16 UL NCB and CB based PUSCH with 4Tx. Carrier frequency; 3.5GHz. SCS; 30KHz. System bandwidth; 20MHz/40MHz/100MHz. UE antennas layout; (M, N, P) = (1,4,2) for ULA; (M, N, P) = (2,2,2) for UPA, (dH,dV) = (0.5, 0.5)λ. UE speed; 3km/h. Scenario; Umi/Uma. Proposal 3: Rel-18 prioritize the specification for 8 Tx PUSCH with up to 4 layers. Proposal 4: For codebook based 8 Tx PUSCH in Rel-18, prioritize the specification for non-coherent and partial coherent 8 Tx precoders. Proposal 5: Rel-18 support both codebook and non-codebook based 8 Tx PUSCH. Proposal 6: NR Rel-18 reuse and concatenate existing Rel-15 2 Tx and/or 4 Tx PUSCH precoders to support 8 Tx PUSCH precoders with partial coherent or noncoherent 8 Tx. - -Prioritize the specification of the following two cases. - -Concatenate two 4 Tx precoders to build an 8 Tx precoder. - -Concatenate four 2 Tx precoders to build an 8 Tx precoder. - -FFS details on signalling to reuse and concatenate existing Rel-15 precoders. Proposal 7: As a starting point, Rel-18 study the new precoder codebook for PUSCH with fully coherent 8 Tx based on DFT matrix or Householder matrix. Proposal 8: Rel-18 specify SRS and SRI enhancement to support non-codebook based PUSCH with 8 Tx. -FFS details on SRI enhancement for 8 SRS ports sounding via a single SRS resource set. -FFS details on SRI enhancement for 8 SRS ports sounding via multiple SRS resource set, each sounding less than 8 ports. Proposal 9: For 2 CWs PUSCH with 8 layers in Rel-18, reuse Rel-15 2 CWs PDSCH CW to layer mapping procedure. Proposal 10: Study, and if necessary, specify HARQ enhancement to support two codewords PUSCH with 8 Tx including at least the following aspects -NDI, RV, MCS signaling for the second CW -CBG based PUSCH with 2 CWs -Dynamic switch between 2 CW and single CW PUSCH Proposal 11: Study, if necessary, specify the UCI-multiplexing enhancement to support UCI multiplexing on two codewords PUSCH with 8 Tx including at least the following aspects -Multiplex UCI only on one of the CWs or both CWs -Whether allowing different beta offset values for the two CWs Proposal 12: Study, if necessary, specify the signaling on DMRS port indication in DCI to support PUSCH with more than 4 layers. Proposal 13: Support reusing Rel-16 full power transmission schemes for PUSCH with 8 Tx, with necessary enhancement if identified. -Strive to minimize spec impact. |

# References

- RP-213598, “New WI: MIMO evolution for Downlink and Uplink”, Samsung, 3GPP RAN Meeting #94e, Dec.6-17, 2021

- R1-2203155, Discussion on SRI/TPMI enhancement for enabling 8 TX UL transmission, Huawei, HiSilicon

- R1-2203269, SRI/TPMI enhancement for enabling 8 TX UL transmission, ZTE

- R1-2203326, Discussion on SRI/TPMI enhancement for enabling 8 TX UL transmission, Spreadtrum Communications

- R1-2203384, On SRI/TPMI Enhancement, InterDigital, Inc.

- R1-2203447, On SRI/TPMI enhancement for UL 8 TX, CATT

- R1-2203547, Views on enabling 8 TX UL transmission, vivo

- R1-2203687, Discussion on SRI/TPMI enhancement, NEC

- R1-2203727, Considerations on TPMI enhancement for UL transmission, Sony

- R1-2203799, Enhancements on 8Tx uplink transmission, xiaomi

- R1-2203894, Views on TPMI/SRI enhancements for 8Tx UL transmission, Samsung

- R1-2203959, SRI TPMI enhancement for 8 TX UL transmission, OPPO

- R1-2204147, SRI/TPMI enhancement for enabling 8 TX UL transmission, LG Electronics

- R1-2204168, SRI/TPMI enhancement for enabling 8TX UL transmission, Lenovo

- R1-2204235	Views on SRI/TPMI enhancement for enabling 8 TX UL transmission, Apple

- R1-2204293, Discussion on SRI/TPMI enhancement for enabling 8 TX UL transmission, CMCC

- R1-220437, Discussion on 8 TX UL transmission, NTT DOCOMO, INC.

- R1-2204512, Views on 8 TX UL transmission, Sharp

- R1-2204544, UL enhancements for enabling 8Tx UL transmission, Nokia, Nokia Shanghai Bell

- R1-2204692, SRI/TPMI enhancment for enabling 8 TX UL Tranmission, MediaTek Inc.

- R1-2204791, Discussion on enhancement for 8Tx UL transmission, Intel Corporation

- R1-2204876, SRI/TPMI enhancement for enabling 8 TX UL transmission, Ericsson

- R1-2205020, Enhancements for 8 Tx UL transmissions, Qualcomm Incorporated