3GPP TSG RAN WG1 Meeting #109-e											          R1-2205221

e-Meeting, May 9th – 20th, 2022

Agenda Item:		9.1.4.2

Source:				Moderator (InterDigital, Inc.)

Title:					FL Summary on SRI/TPMI Enhancements; Second Round

Document for:	Discussion and Decision

# Background

In RAN plenary #94, the WID for Rel-18 MIMO enhancements was finalized [1]. According to the WID, some enhancements for SRI/TPMI are necessary to enable 8 TX UE transmission.

Objective 5: Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices - Note: Potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will be identified as part of the study.

To accomplish the objective, the scope of this agenda item centers on codebook design for 8TX, CW to layer mapping, SRS enhancements to support 8 ports, impacts resulted from coherency characteristics of such UEs as well as UE operation with full power.

# High Priority Topics:

As it has been discussed by many companies, different variations of antennas placement can be considered where each could differently influence codebook design and coherency characteristics of a UE. Furthermore, to decide on the number of codewords, the number of layers, etc., extensive evaluations are required. And lastly, there are some levels of dependency of other sub-agendas on this work. Having said that it is important to establish a common view on UE antenna layout, evaluation methodologies and related assumptions in meeting WG1 # 109e.

# Antenna Layout for 8TX UE

According to companies’ contributions, different antenna layouts can be considered for 8TX UEs. Since antenna layout has a direct impact on codebook design, it is important to have a decision on this aspect before moving forward (ZTE, vivo, OPPO). Some companies have stated that various antenna layouts should be supported for uplink transmission of an 8TX UE (Huawei, IDC, Samsung, NTT, CATT). However, to save effort and reflect real-life use case, another group of companies have proposed to first identify/prioritize the most-likely device type (Samsung, Ericsson), and/or also identify/prioritize the most-likely antenna layouts (OPPO, Lenovo, NTT, MediaTek, Ericsson, CMCC).  Further, some companies have suggested that UE capability reporting should include some information about 8TX UE antenna layout (NTT, IDC). From (NEC) perspective, UE coherency capability reporting is sufficient and can bear related required information.

For Phase I discussion, 4 alternatives were identified based on companies’ views for possible down selection. Table 1 show companies position for each alternative.

Table 1

| Proposed Alternative | Supporting Companies |
| --- | --- |
| Alt1: Support linear array (1D/2D) of single-polarized configuration | Could discuss: Apple, LG, vivo, Intel, CATT, Xiaomi, MediaTek, Ericsson, Nokia |
| Alt2: Support linear array (1D/2D) of cross-polarized configuration | Apple, LG, Samsung, OPPO, NTT, CMCC, Lenovo, NEC, Spreadtrum, ZTE, Huawei, HiSilicon, Intel, CATT, InterDigital, Xiaomi, Qualcomm, Ericsson (outdoor), Nokia, NSB, Sharp, KDDI, vivo |
| Alt3: Support either Alt1 or Alt2 according to a UE indication | - |
| Alt 4: 4-sided directional cross pol array (4 dBi, 110° BW) | Ericsson |

Based on the recent agreement that all three levels of UE coherency are to be considered for 8TX UEs, the following is proposed,

FL Proposal 2.1: For 8TX UE, study the following UE antenna layout,

- Alt1: For non-coherent UEs, study linear array (1D/2D) of single-polarized antenna configuration

- Alt2: For fully/partial-coherent UEs, study linear array (1D/2D) of cross-polarized antenna configuration

  - Ng>=1 antenna groups can be considered where each group comprises coherent antennas, and across groups, antennas can be non-coherent/coherent depending on device types

Table 2

| Company | Views |
| --- | --- |
| OPPO | If antennae across groups can be coherent, it means antennae in the same or different groups are both coherent. Then why to define antenna group for this case? We think definition of antenna group is not mandatory and RAN1 can design partial coherent codebook in the same way as Rel-15. Hence, we suggest to delete the sub-bullet for Alt2. We think the antenna pattern proposed by Ericsson has been widely applied in CPE at least for DL reception. It is expected to be also applied to uplink potentially. Hence, we think 4-side x-pol array can be at least optional for evaluation. |
| Ericsson | Firstly, for Alts 1 and 2, our understanding is that panel(s) containing a ULA or UPA of directional antennas is proposed. The panels would have the 8 dBi elements with antenna patterns assumed according to Table 7.3-1 of 38.901, right? If so, then do proponents of Alt 1 think that the UE contains 3 panels? This seems possible to us for e.g. CPEs mounted above the rooftop, but this is a less likely commercial implementation, since it seems to presume 24 total elements, and probably 24 Tx chains with only 8 being active at a time. We suggest that ULAs/UPAs have a single panel, and to assume that the panel is mounted outdoor and pointed toward a gNB at the time of installation. This seems a more likely commercial implementation for at least outdoor mounted FWA. An alternative to an 8 element ULA/UPA is to use subarrays with different boresights. Alt 4 above is one way to do it. Such an array would only contain 8 total elements and Tx chains. This seems suitable as an FWA device that looks more like a UE, such as a mobile hotspot or an indoor FWA device. Alternatively, it could be mounted on a rooftop. Given the array configurations above, coherence can be applied across subarrays having a same or different polarization, across elements with the same or different boresights, and within or across panels. We would prefer to study different configurations to see the performance is under different coherence assumptions. As such, we do not see the need to already restrict the study of non/partial/full coherence that we just agreed last Friday, and so don’t see the benefit of narrowing to non-coherent vs. full/partial-coherent UEs as in the proposal. Regarding antenna groups and coherence, we think some clear modeling would be quite helpful. If multiple panels facing the same direction are used and some imperfect control of relative phase is assumed across these panels, it is important to understand what that phase error could be for this kind of multi-panel antenna to make any sense. Similarly, as we have already pointed out, coherent UL MIMO designs can have significant amounts of relative phase error. For 2 Tx, 38.101 presently has ±40° relative phase error among the Tx chains. We expect operator installed CPE devices may be able to tolerate higher complexity and support lower amounts of relative phase error. However, we think we should not continue to neglect RF complexity aspects such as relative phase error in RAN1 designs, given what we have learned from the RAN4 requirements and that the only commercially used UL MIMO designs today are non-coherent in our understanding. So our suggestion is to have some measure of the tolerance to phase error as part of the evaluations of UL MIMO, so that we can better establish performance/complexity tradeoffs. Therefore we propose that relative phase error be modelled, using a uniformly distributed phase error X∈{±40°, ±20°,±5°}, as a starting point. The exact values should be further studied. Overall, our proposal is: UE arrays to study include at least: - A panel containing a linear/planar array of 2D cross-polarized 8 dBi elements having 65° beamwidth - The UE is mounted outdoor and element boresights of the panel are toward a gNB - A [4] sided directional array of [4 dBi] cross-polarized elements each element having [110°] beamwidth - The UE is indoor [or roof mounted outside] and is randomly oriented - Other UE array configurations are not precluded - Relative phase error is modeled among coherent Tx chain pairs in UE arrays - A uniformly distributed phase error X∈{[±40°, ±20°,±5°]}, is used as a starting point |
| ZTE | Support the FL’s proposal. Generally speaking, we think that the popular antenna layout should be specified. Meanwhile, we can support the FL’s description for antenna group, which is aligned with typical definition of antenna group as in Rel-15/16. Then, we may have a note for Alt-4, like other antenna layout is not precluded in the EVM. |
| Spreadtrum | Support the FL’s proposal, but the necessity of introducing the concept of “antenna group” needs further discussion. |
| Intel | Generally fine with FL proposal. But we are open for discussion. |
| NTT DOCOMO | Support FL proposal. |
| Sharp | Support the FL Proposal 2.1 in principle. However, we prefer to prioritize cross-polarized antenna because the configuration will reduce the effort of specifying the codebook. |
| Huawei, HiSilicon | We are fine with the FL’s proposal. In the future specification work, both Alt 1&2 can be considered. And from our understanding, Alt-4 can be a special case of Alt-2, with 4 antenna groups each consisting of 2 antennas. |
| Samsung | @OPPO: for multiple-panel UEs, when antenna group = panel, whether antenna across groups can be coherent or non-coherent depends on panel related information such as location of panels, inter-panel spacing etc. The sub-bullet covers different assumptions regarding multiple groups (such as panels). Beside, whether non-coherent/coherent depends on device types and scenarios. So, we prefer to keep it. BTW, Alt4 is also an example of antenna groups where we have 4 groups each with 2 ports. |
| Nokia, NSB | Okay with the proposal. The motivation for the proposal is for the 8Tx CB design. If this is the case, we should focus on Alt 2 with coherent antenna in mind for a new 8Tx CB, which is still lacking in NR spec. For this text “Ng>=1 antenna groups can be considered where each group comprises coherent antennas, and across groups, antennas can be non-coherent/coherent depending on device types” under Alt-2, this statement justifies an independent proposal to define coherent group. |
| Lenovo | Support the proposal in general. - We see the two antenna layouts as options rather than alternatives (both should be discussed with prioritizing cross-polarized UE antennas). - We agree with Nokia, antenna grouping would be better discussed as a standalone proposal. |
| MTK | Our preference is with Alt2. @FL: We would like to correct in Table 1 that our position is in favor of Alt2 (in Table 1). However, we are open to discuss on Alt1. |
| InterDigital | Support the FL proposal. Alt 4 is not precluded for study and covered under an example of Alt 2. So, we think this proposal seems the way forward as a good starting point of discussion. |
| LG | OK with FL proposal. |
| Apple | Two alternatives are listed in the proposal. Is the intention to down-select at a certain point? If yes, our preference is Alt 2. Otherwise, we are fine to have both given that we already agreed to support FC/PC/NC configurations. |
| QC | We fully support FL proposal. At this early stage, it is good to study both alternatives. |
| OPPO | Though we think Alt.1 is not needed, we can accept the proposal. |

# Evaluation Methodologies

Several aspects of 8TX UE operation rely on simulation studies to justify proposed enhancements. For example, determining potential benefits of 2 CW for UL, extending to more than 4 layers transmission, identifying proper codebook design, etc., they all require a baseline evaluation methodology to allow an accurate and fair comparison.

For this meeting, several companies have provided their simulation results. Some system level evaluation results are provided by (vivo, Samsung, ZTE, Ericsson, Intel, NTT, Nokia, Qualcomm, MediaTek), while other companies (vivo, Xiaomi, OPPO, Huawei, Lenovo) have shared their link level findings.

# SLS EVM

There are some differences in EVM assumptions used by companies in their conducted SLS simulation studies. For example, companies have assumed different assumptions for carrier frequency (2GHz, 3.5 GHz, 4GHz, etc.), SCS (15KHz, 30KHz), ISD (200m, 500m, 700m), etc. Therefore, to have a fair and an accurate comparison of provided results, it is important to have a baseline set of EVM parameters.  According to the EVM assumptions reported in companies’ contributions, the following is proposed as the reference for SLS evaluation.

During the discussion in Phase I, companies discussed various aspects of SLS evaluation and narrowed down  the extent of the simulations. At a high level, 4 different deployment scenarios were proposed by companies. Based on the discussion in Phase I, the following is proposed as the baseline EVM for LLS

FL Proposal 2.2.1: Adopt the following Table as the reference EVM for SLS evaluation.

- Companies may provide additional evaluation results per their case of interest.

Table 3

| Parameter | Value |
| --- | --- |
| Frequency range | 3.5 GHz |
| Multiple access | OFDMA |
| Numerology | 14 OFDM symbol slot SCS 15 KHz, 30 KHz |
| Scenario | 38.901 - UMi (ISD = 200 m), 80% Indoor (3Km/h), 20% Outdoor (30km/h) - Uma (ISD = 500 m), 80% Indoor (3Km/h), 20% Outdoor (30km/h) - InF, InF type to be selected/reported by company 38.802 - Dense urban (ISD=200m), (per Table A.2.1-1) |
| Channel model | 38.901 |
| System bandwidth | 20 MHz, 100 MHz |
| gNB RX antenna setup and port layouts (𝑀,𝑁,𝑃,𝑀𝑔,𝑁𝑔,𝑀𝑝,𝑁𝑝) | (8,8,2,1,1,4,8) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 (4,4,2,1,1,4,4) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 (2,2,2,1,1,2,2) with (dH, dV) = (0.5, 0.5)λ |
| Antenna radiation pattern parameters | 38.901 |
| gNB receiver noise figure | 5dB |
| gNB receiver | MMSE-IRC |
| gNB scheduler | Single user with proportional fair |
| Modulation | Up to 64 QAM, Up to 256QAM |
| MIMO scheme | SU-MIMO with rank adaptation |
| UE speed | 3 Km/h, 30Km/h |
| UE TX antenna configuration | To be defined according to outcome of Proposal 2.1 |
| Traffic model | FTP model 1, packet size 500KB/s, (RU 20%, 50%, 70%) Full buffer (optional) |
| Suggested benchmarking | R15 UL 4-Tx codebook, Eigen-based, companies report PRG assumption |
| Precoder granularity | Wideband |
| Power control | Open loop, - alpha = 0.8 - P0 = -50, -80 dBm to be selected according to the deployment scenario |
| UE power rating | 23 dBm (suggested for indoor) 32 dBm (suggested for outdoor) |
| Metric | UL mean-user throughput, 5%-ile and 95%-ile UPT |

Table 4

| Company | Views |
| --- | --- |
| OPPO | If the simulation parameters e.g. scenario, antenna array, SCS, Max modulation are all selected by each companies, it may be difficult to align the performance. |
|  | Some general comments: - Regarding the scenarios, there are 5 InF scenarios in 38.901, and we should then have to pick among them; otherwise companies’ simulations will not be so well aligned. Our thinking is instead to at least have Indoor Office / InH, for the indoor gNB scenario since this is a better established model and will allow more alignment among companies. Note that the 9.1.4.1 evaluations for STxMP, the latest proposal for indoor is more consistent with InH than InF: - Indoor (TR 38.901/802) - While 30kmph can be an interesting speed, we don’t understand the scenario. The 80/20 indoor/outdoor ratio is for eMBB which is not part of this work item in my understanding. 30 kmph is clearly not for UEs mounted on the side of a building, nor is it indoor fixed access. I can see that an outdoor gNB may serve a mixture of FWA and outdoor mobile UEs, but then we should define what those mobile UEs are. Can we have some clarification on what these UEs look like, including their antenna setups, and what the scenario is where UEs will be at 30 kmph? - The (M,N,P=2,2,2) antenna configuration is better suited for lower frequencies or for indoor gNBs. Since 8 Tx UEs at e.g. 700 MHz may not be so likely, we suggest it be used for indoor, to limit the number of simulation configurations. - We may wish to include SU/MU-MIMO switching. Can we also list SU/MU-MIMO in square brackets? - Thanks for listing low, median, and high Rus. Can we have the low and high Rus in square brackets, i.e. {[20], 50, [70]}% RU, or say that they are starting points so that we can adjust these if needed according to simulation outcomes? - Antenna radiation patterns come from different places; please see suggestions in the scenario table below. - To reiterate our suggestion from section 2.1: we should have some measure of the tolerance to phase error as part of the evaluations of UL MIMO, so that we can better establish design tradeoffs. Therefore we propose: - Relative phase error is modeled among coherent Tx chain pairs of UE arrays - A uniformly distributed phase error X∈{[±40°, ±20°,±5°]}, is used as a starting point Regarding the scenarios, as discussed in R1-220487 and R1-2204876, we think 3 scenarios should be defined for 8 Tx evaluations in order to have an understanding of the use cases and to try to limit the number of parameter combinations. For example, indoor gNBs should be smaller than outdoor gNBs, outdoor fixed Ues should point to a gNB, be at higher power, and can be a ULA/UPA, smaller ISDs seem more suitable for indoor Ues, etc. Use-case-dependent system-level simulation parameters. Use case; Outdoor FWA; Indoor FWA; Industrial. UE distribution; 100% outdoor; 100% indoor; 100% indoor. Scenario; Uma (500 m ISD); Umi (200 m ISD); Indoor Office (InH). Power control; [P0=-50, α = 0.8]; [P0=-80, α = 0.8]; [P0=-80, α = 0.8]. UE antenna configuration; ULA or UPA: pointing towards gNB Multi-sided array: Pointing in different directions; Multi-sided array: Pointing in different directions; Multi-sided array: Pointing in different directions. UE antenna height; TBD; According to 36.873; According to 38.901. UE transmit power; [32] dBm; 23 dBm; 23 dBm. UE speed; 3 kmph; 3 kmph; 3 kmph. gNB antenna configuration; (8,8,2,1,1,4,8) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 (4,4,2,1,1,4,4) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆; (8,8,2,1,1,4,8) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 (4,4,2,1,1,4,4) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆; (2,2,2,1,1,2,2) with (dH, dV) = (0.5, 0.5)λ. gNB antenna element gain, HPBW; 8 dBi, 65° 38.901 Table 7.3-1; 8 dBi, 65° 38.901 Table 7.3-1; 5 dBi, 90° IMT.2412 Table 10. - For the outdoor FWA model - Operator provided CPEs are assumed, and so devices may be larger/more complex than typical UEs - CPEs can be mounted on the side or roof of the building, and the panel boresight can be pointed at the gNB during installation when they are on the side of the building. - If one panel/sector is used, CPEs can be pointed at the gNB - If the CPEs have spherical coverage, they are randomly oriented, presumably above rooftop. - All CPEs are outdoor - UE antenna height is TBD - While our initial proposal was [31] dBm for outdoor mounted UEs, we can be OK with [32 dBm] as a starting point. Again, we would like to confirm this value. - For the Indoor FWA model - Consumer deployed FWA or mobile hotspot devices are assumed, which may be more cost sensitive than operator deployed CPEs. - A smaller ISD seems appropriate given the greater pathloss - All UEs are indoor - UEs are randomly oriented - UE power class 3 can be used, since these are more similar to consumer devices. - For the Industrial model - Enterprise or factory indoor deployments are important to capture, since these can be strong scenarios case for higher throughput applications and since they are of high commercial interest. - All UEs are indoor, and the Indoor Office / InH model seems a good starting point. Note that this is also being discussed for the STxMP evaluations in AI 9.1.4.1 - UEs are randomly oriented - UE power class 3 seems appropriate given that gNBs are indoor. Overall, we propose the 3 scenarios and the corresponding parameters as described in the ‘Use-case-dependent system-level simulation parameters’ table above. |
| ZTE | Thanks so much for FL’s efforts. Please review our following comments: For UE speed, we are also wondering the case of ‘30Km/h’ in this enhancement. Alternatively, ‘3-Km/h’ should be optional. Then, after reviewing some companies’ input, we do not think that MU-MIMO should be considered herein for saving simulation efforts. After that, it seems that we are on the same page that high RU may be a typical case for 8TX operation, we think that RU=70% should be considered with higher priority. Then, the ‘optional’ for full buffer may not be needed. FTP model 1, packet size 500KB/s, (RU 20%, 50%, 70%) Full buffer (optional). Finally, regarding UE antenna layout, we share the same views with OPPO that some baseline need to be defined for EVM at least. In AI 8.1.3.2 SRS, the following has been agreed. How about just reusing this architecture. Agreement For 8 Tx SRS, a starting point of UE antenna configurations can be: - (M, N, P; Mg,Ng; Mp, Np) = (2,2,2; 1,1; 2,2), (dH, dV) = (0.5, 0.5)λ, or - (M, N, P; Mg,Ng; Mp, Np) = (1,4,2; 1,1; 1,4), (dH, dV) = (0.5, 0.5)λ. - FFS other 8 Tx UE antenna configuration and alignment with outcomes from other agenda items. |
| Intel | Generally fine with FL proposal. Regarding ZTE’s suggestion on the UE antenna configuration, we think the antenna configuration of SRS for 8Tx should be aligned with the UE antenna configuration in this AI. |
| NTT DOCOMO | For UE speed, we do not understand the use case of ‘30Km/h’. For Outdoor FWA (urban macro scenario), it is more possible that we deploy the 8TX CPEs in dense urban scenario, instead of a normal urban macro scenario. For SU-MIMO vs. MU-MIMO, we think SU-MIMO should be baseline. For traffic model, we think full buffer should be also baseline in addition to FTP model 1. For UE antenna layout, agree with ZTE’s suggestion. |
| Huawei, HiSilicon | We are fine with the FL’s proposal. |
| CATT | We prefer to prioritize scenarios of outdoor FWA and indoor FWA, and define SLS parameters for the two scenarios separately. One suggestion is as follows: Parameter; Outdoor FWA; Indoor FWA. Frequency range; 3.5 GHz; 3.5 GHz. Multiple access; OFDMA; OFDMA. Numerology; 14 OFDM symbol slot; 14 OFDM symbol slot. SCS 30 KHz; SCS 30 KHz. Scenario; 38.901; 38.901. - Uma (ISD = 500 m); - Umi (ISD = 200 m). UE distribution; 100% outdoor; 100% indoor. System bandwidth; 100 MHz; 20 MHz. gNB RX antenna setup and port layouts (𝑀,𝑁,𝑃,𝑀𝑔,𝑁𝑔,𝑀𝑝,𝑁𝑝); (4,4,2,1,1,4,4) with (dH, dV) = (0.5, 0.8)λ; (2,2,2,1,1,2,2) with (dH, dV) = (0.5, 0.5)λ. Antenna radiation pattern parameters; 38.901; 38.901. gNB receiver noise figure; 5dB; 5dB. gNB receiver; MMSE-IRC; MMSE-IRC. gNB scheduler; Single user with proportional fair; Single user with proportional fair. Modulation; Up to 256QAM; Up to 256QAM. MIMO scheme; SU-MIMO with rank adaptation; SU-MIMO with rank adaptation. UE speed; 3 Km/h; 3 Km/h. UE TX antenna configuration; To be defined according to outcome of Proposal 2.1; To be defined according to outcome of Proposal 2.1. Traffic model; FTP model 1, packet size 500KB/s, (RU 20%, 50%, 70%); FTP model 1, packet size 500KB/s, (RU 20%, 50%, 70%). Suggested benchmarking; R15 UL 4-Tx codebook,; R15 UL 4-Tx codebook,. Eigen-based, companies report PRG assumption.; Eigen-based, companies report PRG assumption. Precoder granularity; Wideband; Wideband. Power control; Open loop,; Open loop,. - alpha = 0.8; - alpha = 0.8. - P0 = -50 dBm; - P0 = -80 dBm. UE power rating; 32 dBm; 23 dBm. Metric; UL mean-user throughput, 5%-ile and 95%-ile UPT; UL mean-user throughput, 5%-ile and 95%-ile UPT. |
| Samsung | Support |
| Nokia, NSB | Generally okay. However, there are too many multiple options in some parameters: subcarrier spacing: 2 options; system bandwidth: 2 options; modulation: 2 options; UE speed: 2 options; traffic model: 2 options, etc. The resulting simulation scenarios would be quite large due to these choices. This is a collection of proposals without much consolidation. Suggest to limit options. |
| Lenovo | Agree with Nokia, prefer reducing the simulation parameter variation for meaningful comparison across companies. |
| MTK | We also prefer to reduce the number of simulation parameters, single SCS, bandwidth and modulation should be agreed on, to reduce the simulation efforts. Regarding the modulation order we prefer up to 256QAM. We do not want to restrict the modulation order to see the gains from increased layers only. 64 Rx ports at gNB is not relevant for this study. With a greater number of Rx ports, it is hard to differentiate among the transmit precoder performances as the precoders get close to SVD. |
| FL | Thank you all! I think this is the best we can get at this time, companies’ support to wrap this proposal is appreciated. Updated proposal based on companies’ suggestion: FL Proposal 2.2.1: Adopt the following Table as the reference EVM for SLS evaluation. - Companies may provide additional evaluation results per their case of interest. Parameter; Value. Frequency range; 3.5 GHz. Multiple access; OFDMA. Numerology; 14 OFDM symbol slot SCS 15 KHz, 30 KHz. Scenario; Outdoor FWA (38.901): Uma (ISD = 500 m), 100% Outdoor, 3Km/h. Indoor FWA (38.901): Umi (ISD = 200 m), 100% Indoor, 3Km/h. Industrial (38.901): Indoor Office (Inh), 3Km/h. Dense urban (38.802): (ISD=200m), (per Table A.2.1-1). Channel model; 38.901. System bandwidth; 20 MHz, 100 MHz. gNB RX antenna setup and port layouts (𝑀,𝑁,𝑃,𝑀𝑔,𝑁𝑔,𝑀𝑝,𝑁𝑝); Outdoor FWA: (8,8,2,1,1,4,8) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 (4,4,2,1,1,4,4) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 Indoor FWA: (8,8,2,1,1,4,8) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 (4,4,2,1,1,4,4) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 Industrial: (2,2,2,1,1,2,2) with (dH, dV) = (0.5, 0.5)λ. Antenna radiation pattern parameters; 38.901. gNB receiver noise figure; 5dB. gNB receiver; MMSE-IRC. gNB scheduler; Single user with proportional fair. Modulation; - Up to 64 QAM - Up to 256QAM. MIMO scheme; SU-MIMO with rank adaptation. UE speed; 3 Km/h, 30Km/h. UE TX antenna configuration; To be defined according to outcome of Proposal 2.1. Traffic model; - FTP model 1, packet size 500KB/s, RU= 50% and suggested low/high RU of values of 20% and 70% - Full buffer (optional). Suggested benchmarking; R15 UL 4-Tx codebook, Eigen-based, companies report PRG assumption. Precoder granularity; Wideband. Power control; Open loop, - alpha = 0.8 - P0 = -50, -80 dBm to be selected according to the deployment scenario. UE power rating; 23 dBm (suggested for indoor) 32 dBm (suggested for outdoor). Metric; UL mean-user throughput, 5%-ile and 95%-ile UPT. |
| LG | OK with FL proposal. |
| Apple | OK |
| QC | Although we still think restricting modulation order up to 64 QAM is a biased assumption to show artificial gain of 8-layer over 4-layer PUSCH, we understand it is up to companies to simulate whatever they like. For the sake of RAN1 progress, we are fine with FL proposal. We will still question the gain showed with this biased assumption though. |
| OPPO | Though we think 8Rx and 32Rx are sufficient for evaluation, we can accept the proposal. |

# LLS EVM

There are some differences in EVM assumptions used by companies in their conducted LLS simulation studies. For example, companies have assumed different assumptions for modulation-coding (QPSK and fixed code-rate, AMC, etc.), SCS (15KHz, 30KHz), rank (fixed, adaptive, etc), etc. Therefore, to have a reasonable and an accurate comparison of provided results, it is important to have a baseline set of EVM parameters.  According to the EVM assumptions reported in companies’ contributions, the following is proposed for as the reference for LLS evaluation.

During the discussion in Phase I, companies discussed various aspects of LLS evaluation. Based on the discussion in Phase I, the following is proposed as the baseline EVM for LLS evaluation.

FL Proposal 2.2.2: Adopt the following Table as the reference EVM for LLS evaluation

- Companies may provide additional evaluation results per their case of interest

- LLS is optionally used for 8Tx UL evaluation

Table 5

| Parameter | Value |
| --- | --- |
| Carrier Frequency | 3.5 GHz |
| Waveform | CP-OFDM |
| SCS | 15 KHz , 30 KHz |
| System bandwidth | 20 MHz, 100 MHz |
| Scheduled PRBs | 5, 25, 50, 260 PRBs |
| gNB RX antenna setup and port layouts (𝑀,𝑁,𝑃,𝑀𝑔,𝑁𝑔,𝑀𝑝,𝑁𝑝) | (8,8,2,1,1,4,8) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 (4,4,2,1,1,4,4) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 (2,2,2,1,1,2,2) with (dH , dV ) = (0.5, 0.5)λ |
| UE TX antenna configuration | To be defined according to outcome of Proposal 2.1 |
| UE speed | 3 Km/h |
| Layer | Adaptive, Fixed (reported by company) |
| AMC | Adaptive, Fixed (reported by company) |
| DMRS configuration | Type 1 |
| Channel estimation | Real |
| Channel Model | CDL -A (30ns), CDL -C (300ns) |

Table 6

| Company | Views |
| --- | --- |
| OPPO | Fine with the proposal. |
| Ericsson | ‘Layer’ should be clarified as ‘Number of layers’ Can it be clarified that the Type 1 DMRS configuration is required only for baseline simulations, since we may define different DMRS port Types for more than 4 ports? We might want to add that RVs are reported by companies (depends on if HARQ is used) To reiterate our suggestion from section 2.1: we should have some measure of the tolerance to phase error as part of the evaluations of UL MIMO, so that we can better establish design tradeoffs. Therefore we propose to add the following to the LLS assumptions: - Relative phase error is modeled among coherent Tx chain pairs in UE arrays - A uniformly distributed phase error X∈{[±40°, ±20°,±5°]}, is used as a starting point |
| ZTE | Firstly, for aligning EVM, how about removing 15KHz of SCS. In our initial thoughts, the necessity of two types of SCS may not be needed. Then, for UE antenna layout, as we mentioned in Section 2.2.1, we suggest to reuse what we have for SRS. Agreement For 8 Tx SRS, a starting point of UE antenna configurations can be: - (M, N, P; Mg,Ng; Mp, Np) = (2,2,2; 1,1; 2,2), (dH, dV) = (0.5, 0.5)λ, or - (M, N, P; Mg,Ng; Mp, Np) = (1,4,2; 1,1; 1,4), (dH, dV) = (0.5, 0.5)λ. - FFS other 8 Tx UE antenna configuration and alignment with outcomes from other agenda items. Finally, regarding to relative phase errors, we are open to have some further justification. But for EVM, we are not convinced to have it herein. |
| Intel | Fine with FL proposal. |
| NTT DOCOMO | For UE antenna layout, agree with ZTE’s suggestion. |
| Huawei, HiSilicon | It is unclear for us to preclude CDL-B. In our understanding, CDL-A/B/C have different properties and target for different scenarios. We prefer to add CDL-B channel model to cover more scenarios. Companies can bring the results of interested scenarios. We are fine with other parameters. Channel Model; CDL -A (30ns), CDL-B (100ns), CDL -C (300ns). |
| Samsung | OK, but prefer to clarify that LLS can be used to provide additional results. SLS results are preferable, at least codebook evaluations |
| Nokia, NSB | We don’t think this LLS table is needed for CB evaluation. Need to address the motivation of using LLS for 8Tx codebook design. |
| FL | Thank you all! I think this is the best we can get at this time, companies’ support to wrap this proposal is appreciated. Updated proposal based on companies’ suggestion: FL Proposal 2.2.2: Adopt the following Table as the reference EVM for LLS evaluation - Companies may provide additional evaluation results per their case of interest - LLS is optionally used for 8Tx UL evaluation, if needed Table 5 Parameter; Value. Carrier Frequency; 3.5 GHz. Waveform; CP-OFDM. SCS; 15 KHz , 30 KHz. System bandwidth; 20 MHz, 100 MHz. Scheduled PRBs; 5, 25, 50, 260 PRBs. gNB RX antenna setup and port layouts (𝑀,𝑁,𝑃,𝑀𝑔,𝑁𝑔,𝑀𝑝,𝑁𝑝); (8,8,2,1,1,4,8) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 (4,4,2,1,1,4,4) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 (2,2,2,1,1,2,2) with (dH , dV ) = (0.5, 0.5)λ. UE TX antenna configuration; To be defined according to outcome of Proposal 2.1. UE speed; 3 Km/h. Number of Layers; Adaptive, Fixed (reported by company). AMC; Adaptive, Fixed (reported by company). DMRS configuration; Type 1. Channel estimation; Real. Channel Model; CDL -A (30ns), CDL-B (100ns), CDL -C (300ns). |
| LG | OK with FL proposal. As we expressed in the email, DFT-s-OFDM should be considered. But, we are ok to prioritize for CP-OFDM at this stage. |
| Apple | OK |
| OPPO | We think LLS is also important and support the proposal. |

# Detailed Topics

In this section, detailed topics related to operation of 8TX UE are discussed. Final decisions on these topics may hinge on evaluation results using an agreed set of EVM.

# Number of Layers

In NR Rel-17, uplink transmission is restricted to single codeword and a maximum of 4 layers for transmission. However, for 8TX UEs, some companies are arguing for increasing the number of codewords and layers.

Companies have discussed benefits of utilizing more than 4 layers for uplink transmission (ZTE, vivo, Xiaomi, CMCC, NTT, MediaTek, Ericsson), and further, some companies have shared their evaluation results indicating a higher achieved throughput when 8 layers are used. However, another group of companies seem to be skeptical about the promised gain, and they also have expressed their concern about the workload related to codebook design for transmission with 8 layers. From their perspectives, support of 4 layers should be prioritized, and then upon further evaluation, if justified by use case and the observed performance gain, transmission with more than 4 layers can be supported (Samsung, Lenovo, MediaTek, Qualcomm, Intel). Given that the main concern expressed by companies for transmission with 8 layers is related to potential complications for codebook design, the following is proposed,

During the discussion in Phase I, companies weighed in on whether uplink transmission with 8 layers should be supported. From the 14 supporting companies for transmission with 8 layers, 6 companies have shared their evaluation results in this meeting. The reported results exhibit a significant gain by using 8 layers for transmission. On the other hand, the minority of companies that are asking for further evaluation, argue that the potential gain achieved by using 8 layers may be marginal and not worth the specification work. Qualcomm is against adoption of 8 layers. According to Qualcomm, by allowing maximum modulation of 256QAM instead of restricting it to 64QAM, there is not much difference between TP achieved between 4- and 8-layer transmission.

Table 7

| Proposed Alternative | Supporting Companies |
| --- | --- |
| Support CB/NCB-based transmission with 8 layers | 14 companies: - With simulations: NTT, ZTE, Huawei, HiSilicon, vivo, Ericsson - Without simulations: OPPO, CMCC, CATT, InterDigital, Xiaomi, Nokia, Sharp, CMCC, |
| Support CB/NCB-based transmission with more than 4 layers after further study its benefits | 6 companies: - With simulations: Intel, MediaTek - Without simulations: Apple, Samsung, Lenovo, OPPO |
| Do not support CB/NCB-based transmission with 8 layers | - With simulations: Qualcomm |

Based on the views expressed by majority of companies, who are in support of transmission with 8 layers, the following is proposed,

FL Proposal 3.1a: For 8TX UE uplink transmission, study codebook- and non-codebook-based transmission with both 4 and 8 layers.

Table 8

| Company | Views |
| --- | --- |
| OPPO | We showed our simulation result in AI 9.1.3.1 that support of more than 4 layers can provide significant gain. We are fine to further study and evaluate this issue. For the proposal, we think it should be “with maximal layer number of both 4 and 8 layers” |
| Ericsson | Support with OPPO’s clarification. |
| ZTE | Support the updated version from OPPO. After reviewing the WID, we are wondering whether we can have an agreement to support 8-Tx operation firstly, e.g., as a main bullet, like ‘Enhancement on 8-Tx UL operation to support 4 and more layers is supported in Rel-18.’, considering super majority views. At least, we do not see that any companies object the enhancement for 8-TX UL operation. - Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices - Note: Potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will be identified as part of the study. |
| Spreadtrum | Support the updated version from OPPO |
| Intel | Ok with OPPO’s suggestion. |
| NTT DOCOMO | Support OPPO’s update. |
| Sharp | Support with OPPO’s clarification. |
| Huawei, HiSilicon | Support the updated version from OPPO. We also support ZTE’s views to have an agreement to support 8-TX firstly, although the proposal implies the support of 8 layers, perhaps an explicit agreement is more clear. |
| CATT | It seems the proposal means both up to 4 and up to 8 layers. We suggest to refining the proposal as follows. FL Proposal 3.1a: For 8TX UE uplink transmission, study codebook- and non-codebook-based transmission with both up to 4 layers and up to 8 layers. |
| Samsung | We are open to study >4 layers. However, since the scope of the study for up to 4 layers and > 4 layers are different, in particular, up to 4 layers has smaller scope and is more important from deployment perspective, we can perhaps prioritize up to 4 layers while we study > 4 layers. |
| Nokia, NSB | The proposal is good. It is just a reflection of the WID “enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices” |
| Lenovo | Support OPPO’s updated wording |
| MTK | We are still with same view that more than 4-layer support must be studied and justified through simulations. As presented in our Tdoc(R1-2204692) initial set of simulations that show marginal benefits between 4-layer and 8-layer transmission. |
| InterDigital | OK with OPPO’s update. Also, share with ZTE’s view on the interpretation of the WID. So, RAN1’s task is clearly given by the WID for study to support 4 and more layers to enable 8 Tx UL operation. It is desired RAN1 to pursue the highest gain achievable for 8 Tx, where the direction for pursuing that should be based on a positive assessment as a peak throughput performance for 8 Tx UL. |
| FL | Thank you all! Updated proposal based on companies’ suggestion: FL Proposal 3.1a: For 8TX UE uplink transmission, study codebook- and non-codebook-based transmission with maximal layer number of both 4 and 8 layers. |
| LG | OK with FL proposal. |
| Apple | OK with the proposal |
| QC | At this stage, it is fine to study both. But we just want to point out that max of 4 layers is baseline, which can be prioritized in terms of study and specification. Supporting of max of 8-layer is optimization, which must be justified with meaningful gain with unbiased simulation assumptions. Similar to MTK results, our simulation results also show only marginal gain for 8-layer over 4-layer PUSCH. |
| OPPO | Fine with the updated proposal. |
| vivo | Yes, it should be For 8TX UE uplink transmission, study codebook- and non-codebook-based transmission with both max 4 and max 8 layers |
| Xiaomi | Fine with the proposal 3.1a |

# Codebook Design

Most companies believe that both codebook and non-codebook uplink transmissions should be supported for 8TX UEs (ZTE, Xiaomi, Samsung, LG, Lenovo, Apple, Sharp, MediaTek, Qualcomm, Ericsson). However, from the perspective of codebook design there seem to be two main lines of thought. A first group of companies believe that the codebook for 8 TX UE transmission should be based on legacy 2TX/4TX codebooks where by their concatenation and potential co-phasing, an 8TX codebook can be resulted (Huawei, ZTE, Apple, vivo, CATT, Xiaomi, Lenovo, Qualcomm). Alternatively, given the larger number of antennas in an 8 TX UE that may be place in a cross-polarized format, a second group of companies propose to reuse DL Type-I codebook (ZTE, OPPO, Spreadtrum, Apple, vivo, CATT, Xiaomi, Lenovo, NTT, NEC, Samsung, Nokia, MediaTek).

During the discussion in Phase I, companies further discussed different directions for design of the codebook for 8TX UE. The summary of the discussion and position of each company is summarized here,

Table 9

| Proposed Alternative | Supporting Companies |
| --- | --- |
| Based on NR Rel-15 UL 2TX/4TX codebooks | Spreadtrum, Huawei, Hiilicon, Intel, |
| Based on NR Rel-15 DL Type I codebook | OPPO, NEC, MediaTek, Nokia |
| Select codebook according to UE coherency assumption | Apple, LG, NTT, Lenovo, vivo, Huawei, Hiilicon, CATT, InterDigital, Xiaomi, Qualcomm |

Based on the recent agreement that all three levels of UE coherency are to be considered for 8TX UEs, and also according to companies’ views that the codebook design should be based on coherency characteristics of a UE, the following is proposed.

FL Proposal 3.2: For 8TX UE codebook-based uplink transmission, down-select one of

- Alt1:

  - Study NR Rel-15 UL 2TX/4TX codebooks as the starting point for design of codebook for non-coherent UEs

  - Study NR Rel-15 DL Type I codebook as the starting point for design of codebook for fully/partially-coherent UEs

- Alt2:

  - Study NR Rel-15 UL 2TX/4TX codebooks as the starting point for design of codebook for fully/partially/non-coherent UEs

- Alt3:

  - Study NR Rel-15 DL Type I codebook as the starting point for design of codebook for fully/partially/non-coherent UEs

Table 10

| Company | Views |
| --- | --- |
| OPPO | We prefer Alt3 except for non-coherent UEs. We don’t think Rel-15 UL 2Tx/4Tx CB is needed for 8Tx non-coherent UE. For non-coherent CB, selection of 1 Tx from 8 Tx (with 8 CWs) is the straightforward way. Neither Rel-15 DL CB nor Rel-15 UL CB is needed to be reused. |
| Ericsson | We think a clarification of Alt 2 makes. Please note that the Rel-15 codebook includes Type 1 DL precoding matrices as well as those supporting non- and partially-coherent precoders. In order to have codebooks that support UEs with the same amount of flexibility as the Rel-15 design, we should start with the Rel-15 design, rather than having a codebook that only works with gNB like antenna configurations. Therefore we propose: - Alt2: - Study NR Rel-15 UL 2TX/4TX codebooks in combination with those based on NR Rel-15 DL Type I codebooks as the starting point for design of codebook for fully/partially/non-coherent UEs |
| ZTE | In our initial thought, either Rel-15 DL or UL codebook can work, and then the most of efforts may be paid on the details of achieving the corresponding codebook. Maybe, some examples will be helpful for moving forward this issues in the future. Regarding Rel-15 DL Type I codebook, the following should be studied. - Whether all or subset of the DL Type-I 8-Tx codebook is used for UL can be further studied, e.g., candidate values of N1, N2, O1, O2, i1,1, i1,2, i1,3, i2, codebookMode, etc.. - How to indicate such codebook for UL can be further studied, a uniform codebook scheme for full coherent and partial coherent is preferred. Regarding Rel-15 UL 2Tx/4Tx codebook, the following should be studied: - FFS: co-phase information across different matrixes/port groups, each of which is generated by Rel-15 UL 2Tx/4Tx codebook. - For example, then rank 1-8 8-port codebook can be determined by a list of existing UL precoding matrix (e.g., with R (rank value) columns and X rows (e.g., X=4)) and then combined by co-phase information. - E.g. W0 is a rank-4 4-port full coherent codebook, which can be extended to rank 8 8-port codebook , shown as below. |
| Spreadtrum | We prefer Alt.2, especially 4Tx based. Further, we need to study the combination methods and possible co-phasing information |
| Intel | Generally fine with the original FL Proposal 3.2. The suggestion from Ericsson could be listed as another alternative, i.e., Alt 4. |
| NTT DOCOMO | In our understanding, NR Rel-15 DL Type I codebook is more suitable for fully-coherent UEs, and is not suitable for non-coherent UEs. On the other hand, if the motivation is to list all the possible options, it is possible to select different directions for fully-coherent and partially-coherent UEs. |
| Sharp | Support Alt3 but open to considering Alt2. Alt3 would reduce specification effort, because as mentioned by Ericsson, all or part of the DL Type I codebooks support fully/partially/non-coherent. On the other hand, we believe that Alt2 could be studied for partial coherent antennas because of the possibility of better performance. |
| Huawei, HiSilicon | We prefer Alt 2 to design a unified 8Tx codebook structure for different coherence types. We agree that both Rel-15 DL Type I and UL 2TX/4TX codebook can support 8TX with some enhancements. However, the needed workload to achieve such enhancements may be vastly different. In our understanding, for Alt 2, concatenation of multiple Rel-15 2TX/4TX UL codewords with co-phasing coefficients can build an 8 Tx codeword, supporting all three coherence types. And the legacy TPMI indication table can be reused to minimize the specification impact. The specification effort mainly focuses on the co-phase information as ZTE commented and multiple TPMIs to enable precoder indication with more than 4 layers. On the contrary, for Alt 3, the specification effort may consist of codebook design for partial/no coherence, new TPMI indication table, candidate values of N1, N2, O1, O2, i1,1, i1,2, i1,3, i2, codebookMode as ZTE commented. Intuitively, the specification effort of Alt 3 is far larger than that of Alt 2. For above reasons, we think Alt 2 is preferred. |
| CATT | We prefer to consider different baselines for the three coherent codebooks respectively. For non-coherent codebook, there is only 1 non-zero element for each column and row. Realize 8 ports codebook with combinations of precoders in NR Rel-15 UL 2TX/4TX codebooks is not needed. Besides, The total number of non-coherent precoding matrices is 255 for up to 8 layers. Whether and how to limit the number of precoders in the codebook can be considered. For partial-coherent codebook, multiple coherent groups can be considered, with the precoding matrices for NR Rel-15 UL 2TX/4TX for each coherent group as a start. For fully-coherent codebook, we prefer to consider down selecting precoders in NR Rel-15 DL Type I codebook as a start. Therefore, we propose to add Alt4 in the proposal as follows. - Alt4: - Study NR Rel-15 DL Type I codebook as the starting point for design of codebook for fully-coherent UEs. - Study NR Rel-15 UL 2TX/4TX codebooks for each coherent group as the starting point for design of codebook for partial-coherent UEs. - Study whether and how to restrict the number of antenna selection precoding matrices in the codebook for non-coherent UEs as the starting point for design of codebook for non-coherent UEs. |
| Samsung | Support Alt3 for full- and partial-coherent precoders. Also, several companies have proposed a design based on selection of antenna groups, which for example is needed for multi-panel UEs where a panel = antenna group. So, we suggest to add a bullet as follows: - Study selection of Ng>=1 antenna groups from N>1 antenna groups in Alt1-3 E/// suggestion regarding Alt 2 is also fine |
| Ericsson2 | To further elaborate our earlier comment above. We think Alt 2 should be clarified that Type 1 Rel-15 DL precoders are not precluded, since the other alternatives list Type 1, but Alt 2 does not, and since the Rel-15 codebook includes precoders based on Type 1 DL precoding matrices. A second clarification is that one or multiple TPMIs should be considered, as was proposed by multiple companies. Then our preference is to start with Rel-15 UL MIMO, since it supports a variety of UE implementations and antenna configurations, whereas the Type 1 codebook is only fully coherent. Therefore we revise our proposal above as follows: - Alt2: - Study NR Rel-15 UL 2TX/4TX codebooks as the starting point for design of codebook for fully/partially/non-coherent UEs - Rel-17 precoders studied may include those based on NR Rel-15 DL Type I codebooks - One or multiple TPMIs corresponding to one or multiple SRS resources can be studied |
| Nokia, NSB | Support Alt3. We shall prioritize the 8Tx CB design first and then address the non-coherent/partial coherent cases. Alt 3 would be a good starting choice. |
| Lenovo | We believe further clarity of the codebook design approach would be available once the coherence assumptions for UE antennas are finalized. For instance, for fully-coherent 8Tx, reusing Rel. 15 UL 2/4Tx codebook is not reasonable. We also prefer considering frequency-selective precoding as part of the codebook design. The following FFS is suggested: FFS: whether UL Tx codebook supports frequency-selective precoding |
| MTK | In principle, it is fine. Our preference Alt3 because of its limited standardization workload. |
| InterDigital | We share similar view as Huawei, in that Alt2 can be a good starting point of the work at least for fully and partially-coherent UEs. As many companies support a versatile form of codebook according to UE coherency assumption including us, we can further study proper concatenation using the UL 2TX/4TX codebooks with co-phase information, also mentioned by ZTE. |
| LG | According to coherency, different principle can be applied as we did in Rel-15. For, fully coherent TPMIs, DL Type I codebook can be a starting point, while non/partial coherent TPMIs, Rel-15 UL 2TX/4TX codebooks can be a starting point. So, we slightly modified CATT’s suggestion as below. - Alt4: - Study NR Rel-15 DL Type I codebook as the starting point for design of codebook for fully-coherent UEs. - Study NR Rel-15 UL 2TX/4TX codebooks for each coherent group as the starting point for design of codebook for partial/non-coherent UEs. |
| Apple | We are not sure we agree with how the alternatives are formulated. For non-coherent CB, it can follow the principle of the R15 NC CB, but doesn’t necessarily have to use the exact CB as the starting point. For partial-coherent/non-coherent CB, our understanding is that NR Rel-15 DL Type I codebook is not really applicable. |
| QC | I am wondering if there is a typo in Alt 1? At least in our understanding, Alt 1 should be the following. For non-coherent/partial-coherent UEs, it is suitable to reuse Rel-15 2Tx/4Tx UL CBs can concatenate them to build 8 Tx CB. For full coherent UE, it is appropriate to reuse DL type 1 codebook. We are not sure how partial coherent UE reuse DL type 1 codebook. - Alt1: - Study NR Rel-15 UL 2TX/4TX codebooks as the starting point for design of codebook for non-coherent/partial-coherent Ues - Study NR Rel-15 DL Type I codebook as the starting point for design of codebook for fully/partially-coherent Ues We can also support LG’s Alt 4, which is the same as the modified Alt 1 by us. |
| OPPO | Based on the discussion, we intend to agree with the Alt4 proposed by CATT, which can address our concerns. |
| vivo | In our view, alt 1 and alt 2 can be merged. Basic idea is that the codebooks specified in Rel-17 spec for UL and DL single panel Type I are to begin with. Regarding relationship with coherence and corresponding UE capability can be discussed as we progress. Many companies proposed to consider DL single panel Type I for full-coherent scenario, which we believe also valid for particular deployment. As well all know designing completely new codebooks takes tremendous effort and time consuming, hence we are fine to take legacy codebooks (UL and DL single panel type I) as baseline. Merged Alt 1 and alt 2, the reason for remove 2Tx is that current 4Tx partial-coherent codebook can support coherence assumption of 2+2+2+2. We use 2Tx codebooks then we may need 4 TPMI fields - Merged Alt1&Alt2: - Study NR Rel-15 UL 2TX/4TX codebooks as the starting point for design of codebook for fully/partially/non-coherent UEs - Study NR Rel-15 DL Type I codebook as the starting point for design of codebook for fully/partially-coherent UEs |
| Xiaomi | We can support Alt.3 for fully /partially coherent UEs, and are open to Alt.2 for the potentially less spec effort. |

# Feature-lead Proposals for GTW Approval

(Background Info: Table 1)

| Proposed Alternative | Supporting Companies |
| --- | --- |
| Alt1: Support linear array (1D/2D) of single-polarized configuration | Could discuss: Apple, LG, vivo, Intel, CATT, Xiaomi, MediaTek, Ericsson, Nokia |
| Alt2: Support linear array (1D/2D) of cross-polarized configuration | Apple, LG, Samsung, OPPO, NTT, CMCC, Lenovo, NEC, Spreadtrum, ZTE, Huawei, HiSilicon, Intel, CATT, InterDigital, Xiaomi, Qualcomm, Ericsson (outdoor), Nokia, NSB, Sharp, KDDI, vivo |
| Alt3: Support either Alt1 or Alt2 according to a UE indication | - |
| Alt 4: 4-sided directional cross pol array (4 dBi, 110° BW) | Ericsson |

FL Proposal 2.1: For 8TX UE, study the following UE antenna layout,

- Alt1: For non-coherent UEs, study linear array (1D/2D) of single-polarized antenna configuration

- Alt2: For fully/partial-coherent UEs, study linear array (1D/2D) of cross-polarized antenna configuration

  - Ng>=1 antenna groups can be considered where each group comprises coherent antennas, and across groups, antennas can be non-coherent/coherent depending on device types

FL Proposal 2.2.1: Adopt the following Table as the reference EVM for SLS evaluation.

-          Companies may provide additional evaluation results per their case of interest.

(Table 3)

| Parameter | Value |
| --- | --- |
| Frequency range | 3.5 GHz |
| Multiple access | OFDMA |
| Numerology | 14 OFDM symbol slot SCS, 30 KHz |
| Scenario | Outdoor FWA (38.901): UMa (ISD = 500 m), 100% Outdoor, 3Km/h |
|  | Indoor FWA (38.901): UMi (ISD = 200 m), 100% Indoor, 3Km/h |
|  | Industrial (38.901): Indoor Office (Inh), 3Km/h |
| Channel model | 38.901 |
| System bandwidth | 20 MHz, 100 MHz |
| gNB RX antenna setup and port layouts (𝑀,𝑁,𝑃,𝑀𝑔,𝑁𝑔,𝑀𝑝,𝑁𝑝) | Outdoor FWA: (8,8,2,1,1,4,8) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 (4,4,2,1,1,4,4) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 Indoor FWA: (8,8,2,1,1,4,8) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 (4,4,2,1,1,4,4) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 Industrial: (2,2,2,1,1,2,2) with (dH, dV) = (0.5, 0.5)λ |
| Antenna radiation pattern parameters | Outdoor/Indoor FWA: 38.901 Table 7.3-1, 8 dBi, 65° HPBW Industrial: IMT.2412 Table 10,5 dBi, 90° HPBW |
| gNB receiver noise figure | 5dB |
| gNB receiver | MMSE-IRC |
| gNB scheduler | Single user with proportional fair |
| Modulation | - Up to 64 QAM - Up to 256QAM |
| MIMO scheme | SU-MIMO with rank adaptation |
| UE speed | 3 Km/h |
| UE TX antenna configuration | To be defined according to outcome of Proposal 2.1 |
| Traffic model | - FTP model 1: Packet size 500KB/s, RU= 50% and suggested low/high RU of values of 20% and 70% - [Full buffer] |
| Suggested benchmarking | R15 UL 4-Tx codebook, Eigen-based, companies report PRG assumption |
| Precoder granularity | [Wideband] |
| Power control | Open loop, - alpha = 0.8 - P0 = -50, -80 dBm to be selected according to the deployment scenario |
| UE power rating | 23 dBm (suggested for indoor) 32 dBm (suggested for outdoor) |
| Metric | UL mean-user throughput, 5%-ile and 95%-ile UPT |

| Proposed Alternative | Supporting Companies |
| --- | --- |
| Based on NR Rel-15 UL 2TX/4TX codebooks | Spreadtrum, Huawei, HiSilicon, Intel, |
| Based on NR Rel-15 DL Type I codebook | OPPO, NEC, MediaTek, Nokia |
| Select codebook according to UE coherency assumption | Apple, LG, NTT, Lenovo, vivo, Huawei, Hiilicon, CATT, InterDigital, Xiaomi, Qualcomm |

(Background Info: Table 9)

FL Proposal 3.2: For 8TX UE codebook-based uplink transmission, down-select one of

- Alt1-a:

  - Study NR Rel-15 UL 2TX/4TX codebooks as the starting point for design of the codebook for non-coherent UEs

  - Study NR Rel-15 DL Type I codebook as the starting point for design of the codebook for fully/partially-coherent UEs

- Alt1-b:

  - Study NR Rel-15 UL 2TX/4TX codebooks as the starting point for design of the codebook for partially/non-coherent UEs

  - Study NR Rel-15 DL Type I codebook as the starting point for design of the codebook for fully-coherent UEs

- Alt2:

  - Study NR Rel-15 UL 2TX/4TX codebooks as the starting point for design of codebook for fully/partially/non-coherent UEs

- Alt3:

  - Study NR Rel-15 DL Type I codebook as the starting point for design of codebook for fully/partially/non-coherent UEs

# Agreements

Agreement Study fully-coherent, partially-coherent and non-coherent UEs for uplink transmission with 8TX UEs. Agreement Study full power transmission for 8TX UEs. - Details are FFS upon completion of codebook design Agreement Adopt the following Table as the reference EVM for LLS evaluation - Companies may provide additional evaluation results per their case of interest - LLS is optionally used for 8Tx UL evaluation, if needed Parameter; Value. Carrier Frequency; 3.5 GHz. Waveform; CP-OFDM. SCS; 30 KHz. System bandwidth; 20 MHz, 100 MHz. Scheduled PRBs; 5, 25, 50, 260 PRBs. gNB RX antenna setup and port layouts (𝑀,𝑁,𝑃,𝑀𝑔,𝑁𝑔,𝑀𝑝,𝑁𝑝); (8,8,2,1,1,4,8) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 (4,4,2,1,1,4,4) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 (2,2,2,1,1,2,2) with (dH , dV ) = (0.5, 0.5)λ. UE TX antenna configuration; To be defined according to outcome of Proposal 2.1. UE speed; 3 Km/h. Number of Layers; Adaptive, Fixed (reported by company). AMC; Adaptive, Fixed (reported by company). DMRS configuration; Type 1; 1 front loaded + 1 additional symbol. Channel estimation; Real. Channel Model; CDL-A (30ns), CDL-B (100ns), CDL-C (300ns). Agreement For 8TX UE uplink transmission, study codebook- and non-codebook-based transmission with maximal layer number of both 4 and 8 layers.

# List of Companies’ Proposals

| Huawei | Proposal 1: SRI enhancement with overhead reduction to enable 8TX NCB based UL transmission should be supported. Proposal 2: UL 8TX codebook design should consider various antenna layouts so as to accommodate different UE implementations. Proposal 3：Frequency selective precoding should be supported for 8TX UL transmission. Proposal 4：Block-wise codebook based on legacy 2TX or 4TX UL codebook should be supported to enable 8TX in CB based UL transmission. Proposal 5: The beamformed CSI-RS should be considered to indicate UL precoders to UE. |
| --- | --- |
| ZTE | Proposal 1: Regarding 8 Tx-UL operation in Rel-18, support 8-TX and more than 4 layers UL transmission. Proposal 2: Both codebook based and non-codebook based UL transmission should be supported in Rel-18 for UL MIMO. Proposal 3: Full coherent and partial coherent capabilities should be supported, and non coherent capability can be precluded or deprioritized in Rel-18 for UL MIMO. Proposal 4: On 8-Tx UL transmission enhancement, 2 CWs for UL transmission should be supported. - Condition of enabling >1 CWs for UL transmission can be further studied in RAN1, e.g., if the number of TX(s) and the number of UL layers exceed threshold(s). Proposal 5: The legacy full power mode 1 and 2 should be supported in Rel-18 for UL MIMO, but details can be discussed after codebook design is clear. Proposal 6: Antenna layout assumption for partial coherent codebook should be determined before codebook design. Proposal 7: Regarding 8-Tx full coherent codebook design, the following schemes can be considered - Scheme 1: reusing DL type I 8Tx codebook scheme - Scheme 2: UL 4Tx codebook + additional phase Proposal 8: Regarding 8-Tx partial coherent codebook design, the following schemes can be considered - Scheme 1: 8-port full coherent codebook indication + puncture pattern indication (indicating zero-power elements in a matrix) - Scheme 2: Multiple port groups, with common or separate precoding information indication Proposal 9: Regarding non codebook based transmission design for 8-Tx, - The number of SRS resources in an SRS set can be up to 8 - Potential optimization for SRI re-design considering DCI overhead, e.g., 8 bits or less Proposal 10: Regarding CW-to-layer mapping, support up to 2 CWs for UL 8-Tx transmission, and then further study the following aspects: - Scrambling, layer mapping, and DCI format to support second TB - Code rate and TB size when UCI is transmitted on PUSCH |
| Spreadtrum | Proposal 1：Study the potential methods to reduce DCI overhead, e.g. limit the selection of SRS resource for part of layers. Proposal 2: The following codebook design principle for 8 antenna ports can be considered: - Option 1: based on LTE householder matrix - Option 2: based on NR DL codebook for 8-layer transmission - Option 3: other possible combination of existing precoding matrices |
| InterDigital | Proposal 1: For enabling 8 Tx UL for UEs with many antennas, it should be discussed to use single or dual codeword based on considered scenarios, UE types, coherency types, and so forth. Proposal 2: Consider UE to report its capabilities on a supported type of antenna/panel structure or virtualization capability across UE antenna ports such as SRS antenna ports, enabling up to 8 Tx UL. Proposal 3: Support to retain the full power transmission mode of operation with necessary enhancements to be also applicable for the new enhanced UL-MIMO transmission case supporting up to 8-Tx UL. |
| CATT | Proposal 1: Support UL 8Tx with up to 8 layers in Rel-18. Proposal 2: Codebook designs for non-coherent, partial-coherent and full-coherent devices are considered for UL 8Tx. Proposal 3: For UL 8Tx with DFT-s-OFDM, precoding matrix in Table 1 is included in non-coherent codebook. Proposal 4: For PUSCH transmission with CP-OFDM, down selection of port-selection precoding matrix shall be considered for non-coherent codebook for 8Tx . Proposal 5: On the design of partial-coherent codebook(s) for UL 8Tx, one or more typical partial-coherent relationships of antennas are considered -Two coherent groups with four coherent antennas per group, and four coherent groups with two coherent antennas per group can be considered with high priority. Proposal 6: On the design of precoding matrices for partial-coherent transmission for UL 8Tx, the precoding matrices for 2Tx/4Tx in Rel-15 can be considered for each coherent antenna group as a baseline. Proposal 7: Full-coherent precoding matrix for UL 8Tx for DFT-s-OFDM can be designed by computer search with good distance properties. Proposal 8: For the design of full-coherent precoding matrices for UL 8Tx for CP-OFDM, down selecting precoding matrices from DL Type I codebook can be considered as a start. Proposal 9: On full-coherent codebook design for UL 8Tx for CP-OFDM, supporting multiple codebooks for various types of antenna structures can be considered. Proposal 10: For UL 8Tx, the size of codebook should be carefully determined. Proposal 11: UL full power transmission is supported for UL 8Tx, -Whether all of UL full power transmission schemes in Rel-16 are supported can be considered. Proposal 12: For UL 8Tx for codebook based PUSCH, TPMI/SRI indication is designed after the codebook and SRS design are available. Proposal 13: For UL 8Tx for non-codebook based PUSCH, extending the bit width of current SRI field to enable indicating up to 8 SRS resources. |
| vivo | Proposal 1: The supported scenarios and antenna assumptions should be settled in the very first meeting for the group to work in focused manner. Proposal 2: Two SRI fields corresponding to two SRS resource sets for non-codebook transmission can be considered to simplify the SRI indication. Proposal 3: Codebook constructed by two 4Tx precoders indicated by two TPMI fields can be considered for partial and none coherent antenna assumption. Proposal 4: DL type1 codebook can be considered for fully coherent antenna assumption. Proposal 5: Two CWs can be considered for rank>4. Proposal 6: Following issues should be further discussed: -PTRS-DMRS association indication when rank>4 - Impact on full power modes |
| NEC | Proposal 1: Clarifying whether 1-3 layer transmission is based on enhancements with 8Tx or not. 2 Proposal 2: From UE perspective, reporting capability of full, partial and non coherent is sufficient. Proposal 3: Type I single-panel codebook is reused for uplink full coherent codebook enhancement. Proposal 4: Overhead reduction for partial and non coherent codebook should be studied. |
| Sony | Proposal 1: Enhanced TPMI indication with finer precoding information can be considered for UE UL transmission enhancements. Proposal 2: Enhanced TPMI indication with finer precoding information can also be considered for UE UL transmission enhancements in multi TRP scenario. |
| Xiaomi | Proposal 1: Support 8Tx with more than 4 transmission layers for the UL. Proposal 2: For the PUSCH, the number of antenna ports should be extended to 8 accordingly. Proposal 3: Support both codebook based UL transmission and non-codebook based UL transmission. Proposal 4: 2 codewords should be supported for up to 8 layers of uplink transmission. Proposal 5: The supported 2 codeword transmission scheme can be enabled when more than X transmission layers is configured, X is up to UE capability. Proposal 6: For non-codebook based PUSCH transmission with 8Tx, SRI indicated by the bitmap of the SRS resources or the ports of the SRS resource configured for the SRS resource set is preferred for the simplicity without the effort on the design of new SRI tables. Proposal 7: Three coherence types of codewords should be considered in Rel-18 UL 8Tx codebook design, i.e., full-coherent, partial-coherent, and non-coherent codewords. Proposal 8: 8Tx full-coherent codewords can be designed on the basis of Rel-15 DL Type I single-panel 8Tx codebook with selected parameters. Proposal 9: For partial-coherent codewords, the antenna ports can be divided into two or more antenna port groups. Proposal 10: 8Tx partial-coherent codewords can be designed on the basis of 8Tx full-coherent codewords. Proposal 11: 8Tx partial-coherent codewords can be designed on the basis of Rel-15 UL 4Tx full-coherent codewords. Proposal 12: The scaling factor can be computed as sqrt(1⁄K) where K denotes the number of non-zero elements in 8Tx codewords. 5/8 Proposal 13: The research on DFT-based multi-panel codebook can be left for FFS. Some other pre-designed orthogonal codebooks other than DFT-based codebook can also be left for FFS. |
| Samsung | Proposal 1: for the study of 8 Tx UL operations, -Prioritize one device type for efficient study and discussions - -The one device should be the one which is ‘most-likely’ to have 8 Tx antennae for UL operations - -1st priority: CPE - -2nd priority: FWA Proposal 2: Regarding the transmission scheme for 8Tx UL operations, -Support both CB and NCB based UL transmission - -For RAN1 work, adopt a serialized approach, -prioritize CB based discussion until RAN1#110 -NCB based discussion starts after RAN1#110 Proposal 3: regarding the 8 Tx antenna structure, study the following aspects -Polarization: co-pol or dual-pol -Antenna groups: one group or multiple groups - -Antenna panels and coherence types: -Single panel: one antenna group and full-coherence -Multi-panel: multiple antenna groups and partial-/non-coherence Proposal 4: support a single unified 8Tx codebook structure for different coherence types (i.e. FC, PC, and NC) based on antenna groups -Antennae within a group are coherent -Antennae across multiple groups are non-coherent Proposal 5: regarding the 8Tx UL codebook, -The baseline is Rel. 15 NR 8Tx DL Type I codebook - -Support the following parameters for 8Tx UL codebook -𝑃= number of antenna polarization; 𝑃=1 for co-pol, and 𝑃=2 for dual-pol. -𝑀𝑔= number of antenna groups. -𝑁1= Number of antennae in 1st dimension. -𝑁2= Number of antennae in 2nd dimension. Proposal 6: investigate both single-stage and dual-stage codebook structures Proposal 7: Discussion on full power modes can be start after the 8Tx codebook is designed Proposal 8: regarding 8Tx NCB based UL transmission, -Support number of SRS resources (𝑁𝑆𝑅𝑆) up to 8 -Support both one SRS resource set and two SRS resource sets -When 𝑁𝑆𝑅𝑆≤4, the SRI indication follows legacy (Rel.15) scheme, and -When 𝑁𝑆𝑅𝑆>4, -Study the need for 𝐿𝑚𝑎𝑥>4 -study the following SRI indication schemes -Alt1: combinatorial index scheme -Alt2: bitmap based scheme Proposal 9: for STx2P, support both -Case 1 (1 PUSCH): one SRI indicating a pair of SRS resources (e.g. STx2P to sTRP) -Case 2 (2 PUSCHs): two SRIs, each indicating a SRS resource for a TRP (e.g. STx2P to mTRP) Proposal 10: regarding max number of layers -prioritize the RAN1 work for max 4 layers - >4 layers can be supported, if its need and use cases can be identified Proposal 11: for the study of 8Tx UL operations, support EVM assumptions provided in Table 1. |
| OPPO | Proposal 1: Limited antenna pattern(s) should be confirmed for evaluation of 8Tx uplink transmission. Proposal 2: Support full coherent and partial coherent codebook for 8 Tx uplink. - - For full-coherent codebook with uniform linear antenna array, DL 8Tx Type 1 CB (wideband beam and co-phasing) as baseline with possible beam reduction. o FFS for other antenna array, e.g. clustered or four-sided antenna array - - For partial-coherent codebook, only consider intra-polarization coherence (2Tx) similar to Rel-15. - - Non-coherent codebook has low priority considering practical antenna pattern and signaling overhead. Proposal 3: Consider separate indication of TRI and TPMI for 8 Tx uplink. Proposal 4: Introduce SRI enhancement to indicate up to 8 SRS resources for non-codebook uplink transmission, considering signaling overhead and standardization complexity. |
| LG | Proposal 1: Support both codebook and non-codebook based 8Tx UL transmission in Rel-18 MIMO. Proposal 2: Support up to 8 layers for 8Tx UL transmission in Rel-18 MIMO, and reuse the same codeword-to-layer mapping used in DL transmission. Proposal 3: Support full power 8Tx UL Transmission, and the details can be further discussed after finishing 8 Tx UL codebook design. Proposal 4: Support two-level partial coherency for codebook based 8Tx UL transmission. -Level-1: 4-pair 2-Tx coherency - Level-2: 2-pair 4-Tx coherency Proposal 5: Adopt Table 4 for rank 1 8 Tx codebook for CP-OFDM. Proposal 6: Adopt Table 5 for rank 1 8 Tx codebook for DFT-s-OFDM. Proposal 7: Consider following alternatives for enabling 8Tx non-codebook based UL Transmission. -Alt1. Increase # of SRS resource from 4 to 8. - Alt2. Allow max 2 SRS ports per SRS resource - Alt3. Reuse Rel-17 S-DCI based M-TRP PUSCH mechanism |
| Lenovo | Proposal 1: Both codebook and non-codebook based PUSCH should be supported for 8Tx UL operation Proposal 2: Evaluate 8Tx UL operation for both Rank = 4 and Rank > 4. Rank > 4 is supported only if the potential gains exceed the complexity corresponding to rank upgrade. Proposal 3: Support of less than rank 4 is not precluded for 8Tx UL operation. Proposal 4: Study codeword-to-layer mapping for 8Tx UL operation if Rank>4 is supported Proposal 5: To support 8Tx UL transmission, on the SRS configuration, -One or two SRS resources with 8 SRS ports can be configured in the SRS resource set for CB when codebook based UL transmission is configured, and -Up to 8 SRS resources with single port can be configured in the SRS resource set for nCB when non-codebook based UL transmission is configured. Proposal 6: Prioritize full coherence and partial coherent UE capability for 8Tx UL operation. Proposal 7: For full coherent single panel UE, the antenna configurations illustrated in Figure 1 are considered for evaluation of 8Tx UL operation. Proposal 8: For multi-panel UE, the antenna configurations illustrated in Figure 2 are considered for evaluation of 8Tx UL operation. Proposal 9: Study different codebook design alternatives for 8Tx UL operation. Proposal 10: Dual-stage codebook should be adopted for 8Tx UE and Rel-15 DL Type-I CSI codebook for 8 ports can be adopted as a starting point. Proposal 11: TPMI signaling overhead is considered as a performance metric when studying different alternatives for 8Tx UL codebook design. Proposal 12: Study the performance benefits, signaling overhead and specification impact of supporting frequency-selective precoding for 8Tx UE. |
| Apple | Proposal 1: Consider at least full coherent and partial coherent antenna configurations, and use the antenna configurations in Figure 1 as the baseline. -FFS non-coherent antenna configurations Proposal 2: Consider both codebook based and non-codebook based UL transmission schemes for 8 Tx UL operation. Proposal 3: For codebook based transmission scheme with full coherent antenna configuration, use the DL Type I codebook design for 8 Tx as the starting point for 8 Tx UL. Proposal 4: If separate beam indication is to be considered for the partial coherent antenna configurations, clarify whether this should be further discussed in this AI or merged into the AI on simultaneous multi-panel UL transmission. Proposal 5: For codebook based transmission with partial coherent antenna configuration, use the legacy codebook for 4 Tx UL as the starting point for the per-panel precoding. Proposal 6: If full power mode is to be considered, advanced UEs similar to a UE that supports ul-FullPwrMode-r16 in Rel-16 (with all full-rated PAs) should be assumed to simplify the design. Proposal 7: For non-codebook based transmission, consider SRI enhancements to support up to 8 layers. Potential overhead reduction for SRI field can be considered. |
| CMCC | Proposal 1: All the 1-8 layers UL transmission should be considered for 8 TX UL transmission. Proposal 2: SRI field in Rel-15 can be reused for codebook based 8 TX UL transmission. Proposal 3: How to indicate up to 8 transmission rank and corresponding PUSCH precoder without increasing the SRI overhead for non-codebook UL transmission can be further studied. Proposal 4: The supported configurations of (N1, N2) for 8 TX UE can be N1=N2=2 or N1=4, N2=1 with the consideration of dual polarization, and the supported configurations of over sampling factor (O1, O2) can be further discussed. Proposal 5: Enable 2 CWs with individual MCS, RV and NDI for 8 TX UL transmission can be further studied. Proposal 6: Full TX power enhancement for enabling 8 TX UL transmission should be discussed for fullpower, fullpowerMode1, and fullpowerMode2. |
| NTT DOCOMO | Proposal 1: Support 8TX UL transmission with up to 8 layers per UE. Proposal 2: -Discuss 8TX UE antenna layout and identify the important UE antenna layout in commercial NW. -Support UE capability reporting of 8TX UE antenna layout. -Rel-18 design should be able to applicable to any 8TX UE antenna layout. Proposal 3: -Support two codewords for PUSCH transmission for more than 4 layers. Following enhancements can be further discussed. - codeword-to-layer mapping for more than 4 layers for spatial multiplexing - DCI enhancement with codeword-specific indications of MCS, NDI, and RV - UCI multiplexing on two codewords PUSCH -Support two codewords for simultaneous multi-panel UL transmission for less than 4 layers. The design on support of two codewords PUSCH is common and discussed jointly for 8TX UL transmission and multi-panel UE transmission. Proposal 4: -Support antenna port indication enhancement for UL rank from 5 to 8. -Support enhanced PTRS-DMRS association for more than 4 scheduled DMRS ports. Proposal 5: -Support codebook-based PUSCH transmission for more than 4 layers. -Support to define new UL 8TX codebook, which should consider non-coherent, partial-coherent, and full-coherent UEs, and applicable to any 8TX UE antenna layout. -Support UE capability reporting of coherent ports for partial-coherent UE. -Support NR DL Type 1 single-panel codebook as starting point for UL full-coherent precoders. -The number of supported non-coherent, partial-coherent, and full-coherent precoders should be carefully considered to reduce the DCI indication overhead. -Support full power transmission enhancements for non-coherent and partial-coherent precoders. Proposal 6: Support 8-port SRS resource with usage of ‘codebook’. Proposal 7: -Support non-codebook-based PUSCH transmission for more than 4 layers. -Support configuration of eight 1-port SRS resources in an SRS resource set with usage of ‘noncodebook’. -Support SRI indication enhancement for non-codebook-based PUSCH for 8 SRS resources configured with usage of ‘noncodebook’ for maximum UL rank from 1 to 8. Proposal 8: Support to discuss SRS enhancement for antenna switching for 8T8R. |
| Sharp | Proposal 1: 8 Tx UL operation should be introduced in Rel-18. Proposal 2: More than 4 layers should be supported in Rel-18. Proposal 3: The number of layers should be 5, 6, 7 and 8 for study purpose in Rel-18. |
| Nokia | Proposal 1: Study NR DL 8Tx codebooks for uplink transmission, with consideration of uplink antenna implementations for CPE/FWA/vehicle/Industrial devices. Proposal 2: Consider 8Tx codebook design with azimuth/elevation antenna port configuration (𝑵𝟏,)=(𝟒,𝟏) of Type-I 8Tx codebook. Proposal 3: RAN1 shall discuss on whether 8Tx multi-panel codebook should be studied in Rel-18 under Objective 5. Proposal 4: RAN1 shall study system level performance for 8Tx codebook design with frequency selective precoding. Proposal 5: RAN1 shall agree on simulation assumptions for system-level evaluation for 8Tx performance. Proposal 6: Consider a common DCI format for sub-band precoder indication for various UL resource allocation sizes. Proposal 7: Consider to group coherence antenna ports to support the indication of 8Tx coherence levels. |
| Mediatek | Proposal 1: Support both Codebook and Non-Codebook-based UL Transmission precoding for 8 TX UL. Proposal 2: Study and if justified, support more than 4 transmission layers. Proposal 3: Study, and if justified, support two codewords for more than 4-layer transmission, where codeword-to-layer mapping follows its DL equivalent defined in 3GPP TS 38.211 Section 7.3.1.3. Proposal 4: For the evaluations of 8 TX UL, 8 Rx antennas should be considered for the baseline evaluation. Proposal 5: Support uniform antenna arrays for UL 8 TX codebook, with 4x1 and 2 antenna configurations. Proposal 6: Study the effect of precoding accuracy as a function of the transmission rank, and if justified, design the precoder CB with variable accuracy at different ranks. Proposal 7: Specify the 8 TX UL precoder codebook as a subset of (or identical to) the 8-TX DL Type 1-SP precoder codebook. Proposal 8: With Release 15 Type I SP CB as a baseline design for 8 TX UL CB, subsets of Type I CB can be chosen to help reduce PMI overhead through the following techniques: -Lower DFT oversampling factors (which can be rank-dependent). -Smaller number of beam co-phasing choices. -FFS: Other schemes Proposal 9: For Type I SP based UL precoder, study and if justified specify a two-stage PMI feedback to limit feedback overhead, where -Slow-changing PMI elements are grouped in one stage with low feedback frequency -Fast-changing PMI elements are grouped in the other stage with higher feedback frequency -Information from both PMI feedback stages construct a complete PMI. Proposal 10: Study and if justified specify an 8 TX UL precoder codebook using linear combination coefficients of DFT beams. -Example: A subset of the Type II DL CB. Proposal 11: For 8 TX UEs, partially coherent antenna groups should be uniformly divided into one or more of the following: -Two groups of 4 coherent ports each -Four groups of 2 coherent groups each Proposal 12: For Partially coherent 8TX antenna ports, each antenna group of size M should be designed such that: -The port group exclusively transmits up to M layers, which are not transmitted by the other port groups. -The precoding weights for those port in the focus group constitute a sub-matrix of the full precoder matrix with size M (where L is the number of layers and ) -The submatrix is populated using wither (i) the legacy coherent precoders for M TX ports with L layers, or (ii) alternative precoder design. Proposal 13: Support non-coherent antenna ports for 8 TX UEs. Proposal 14: For SRS Resource Sets configured for non-codebook usage, support up to 8 SRS Resources. Proposal 15: UL 8 TX to be evaluated under low to moderate UE velocity scenarios as 8 TX is aimed for relatively static CPE devices. Study Dense Urban with ISD=200m and UMa with ISD=500m to evaluate scenarios under larger ISD. |
| Intel | Proposal 1: RAN1 to consider full coherence UE as starting point for UL transmission with 8Tx Proposal 2: RAN1 to consider full power Mode 0 as starting point for codebook-based UL transmission with 8Tx Proposal 3: RAN1 to discuss the codebook construction for codebook-based UL transmission with 8Tx Proposal 4: RAN1 to consider 4 layers as starting point for codebook-based UL transmission with 8Tx Proposal 5: RAN1 to discuss the SRS configuration for codebook-based UL transmission with 8Tx Proposal 6: RAN1 to discuss the TPMI indication enhancement for codebook-based UL transmission with 8Tx Proposal 7: For non-codebook based transmission in Rel-18, RAN1 to discuss the SRS configuration, i.e., one SRS resource set vs. two SRS resource sets Proposal 8: For non-codebook based transmission in Rel-18, RAN1 to discuss the SRI field enhancement, i.e., one SRI field vs. two SRI fields |
| Ericsson | Proposal 1: Define a limited set of deployment scenarios for Rel-18 UL MIMO relevant to market needs. -Use the scenarios given in [2] as a starting point. -Down select among FWA/CPE, vehicular, and industrial scenarios if possible. -Use realistic UE transmit chains and antenna implementations. Proposal 2: Techniques are compared primarily using system-level simulation studies. -Net benefits of Rel-18 UL MIMO designs are evaluated considering UE and gNB antenna directionality and SINR conditions over the cells. -Link level simulations can additionally be used e.g. for impairments such as channel estimation -Evaluations are used to determine which Rel-15/16/17 UL MIMO modes should be enhanced. Proposal 3: Study the following alternatives to support 8 Tx -8 port UL MIMO codebooks, including fully/partially/non-coherent operation and all full power modes. -Extending Rel-15 NCB-based operation to 8 layers -Indicating multiple Rel-15/16 TPMIs, each corresponding to an SRS resource |
| Qualcomm | Proposal 1: Study the performance of 8 Tx PUSCH via SLS. Prioritize 8Tx CPE/FWA devices in this study. Proposal 2: Adopt the following SLS assumptions as a starting point to study the performance of 8 Tx PUSCH. Parameter; Value. Metric; UL throughput. Baseline; Rel-16 UL NCB and CB based PUSCH with 4Tx. Carrier frequency; 3.5GHz. SCS; 30KHz. System bandwidth; 20MHz/40MHz/100MHz. UE antennas layout; (M, N, P) = (1,4,2) for ULA; (M, N, P) = (2,2,2) for UPA, (dH,dV) = (0.5, 0.5)λ. UE speed; 3km/h. Scenario; Umi/Uma. Proposal 3: Rel-18 prioritize the specification for 8 Tx PUSCH with up to 4 layers. Proposal 4: For codebook based 8 Tx PUSCH in Rel-18, prioritize the specification for non-coherent and partial coherent 8 Tx precoders. Proposal 5: Rel-18 support both codebook and non-codebook based 8 Tx PUSCH. Proposal 6: NR Rel-18 reuse and concatenate existing Rel-15 2 Tx and/or 4 Tx PUSCH precoders to support 8 Tx PUSCH precoders with partial coherent or noncoherent 8 Tx. - -Prioritize the specification of the following two cases. - -Concatenate two 4 Tx precoders to build an 8 Tx precoder. - -Concatenate four 2 Tx precoders to build an 8 Tx precoder. - -FFS details on signalling to reuse and concatenate existing Rel-15 precoders. Proposal 7: As a starting point, Rel-18 study the new precoder codebook for PUSCH with fully coherent 8 Tx based on DFT matrix or Householder matrix. Proposal 8: Rel-18 specify SRS and SRI enhancement to support non-codebook based PUSCH with 8 Tx. -FFS details on SRI enhancement for 8 SRS ports sounding via a single SRS resource set. -FFS details on SRI enhancement for 8 SRS ports sounding via multiple SRS resource set, each sounding less than 8 ports. Proposal 9: For 2 CWs PUSCH with 8 layers in Rel-18, reuse Rel-15 2 CWs PDSCH CW to layer mapping procedure. Proposal 10: Study, and if necessary, specify HARQ enhancement to support two codewords PUSCH with 8 Tx including at least the following aspects -NDI, RV, MCS signaling for the second CW -CBG based PUSCH with 2 CWs -Dynamic switch between 2 CW and single CW PUSCH Proposal 11: Study, if necessary, specify the UCI-multiplexing enhancement to support UCI multiplexing on two codewords PUSCH with 8 Tx including at least the following aspects -Multiplex UCI only on one of the CWs or both CWs -Whether allowing different beta offset values for the two CWs Proposal 12: Study, if necessary, specify the signaling on DMRS port indication in DCI to support PUSCH with more than 4 layers. Proposal 13: Support reusing Rel-16 full power transmission schemes for PUSCH with 8 Tx, with necessary enhancement if identified. -Strive to minimize spec impact. |

# References

- RP-213598, “New WI: MIMO evolution for Downlink and Uplink”, Samsung, 3GPP RAN Meeting #94e, Dec.6-17, 2021

- R1-2203155, Discussion on SRI/TPMI enhancement for enabling 8 TX UL transmission, Huawei, HiSilicon

- R1-2203269, SRI/TPMI enhancement for enabling 8 TX UL transmission, ZTE

- R1-2203326, Discussion on SRI/TPMI enhancement for enabling 8 TX UL transmission, Spreadtrum Communications

- R1-2203384, On SRI/TPMI Enhancement, InterDigital, Inc.

- R1-2203447, On SRI/TPMI enhancement for UL 8 TX, CATT

- R1-2203547, Views on enabling 8 TX UL transmission, vivo

- R1-2203687, Discussion on SRI/TPMI enhancement, NEC

- R1-2203727, Considerations on TPMI enhancement for UL transmission, Sony

- R1-2203799, Enhancements on 8Tx uplink transmission, xiaomi

- R1-2203894, Views on TPMI/SRI enhancements for 8Tx UL transmission, Samsung

- R1-2203959, SRI TPMI enhancement for 8 TX UL transmission, OPPO

- R1-2204147, SRI/TPMI enhancement for enabling 8 TX UL transmission, LG Electronics

- R1-2204168, SRI/TPMI enhancement for enabling 8TX UL transmission, Lenovo

- R1-2204235	Views on SRI/TPMI enhancement for enabling 8 TX UL transmission, Apple

- R1-2204293, Discussion on SRI/TPMI enhancement for enabling 8 TX UL transmission, CMCC

- R1-220437, Discussion on 8 TX UL transmission, NTT DOCOMO, INC.

- R1-2204512, Views on 8 TX UL transmission, Sharp

- R1-2204544, UL enhancements for enabling 8Tx UL transmission, Nokia, Nokia Shanghai Bell

- R1-2204692, SRI/TPMI enhancment for enabling 8 TX UL Tranmission, MediaTek Inc.

- R1-2204791, Discussion on enhancement for 8Tx UL transmission, Intel Corporation

- R1-2204876, SRI/TPMI enhancement for enabling 8 TX UL transmission, Ericsson

- R1-2205020, Enhancements for 8 Tx UL transmissions, Qualcomm Incorporated