3GPP TSG RAN WG1 #109-e			R1-2205244

e-Meeting, May 9th – 20th, 2022

Agenda Item:	8.2.5

Source:	Moderator (InterDigital, Inc.)

Title:	Discussion Summary #1 of Beam Management for new SCSs

Document for:	Discussion and Decision

# Introduction

In this contribution, we summarize all issues discussed on beam management and timings associated with beam-based operation for new SCSs to support NR from 52.6 GHz to 71 GHz for the following email discussion in RAN#109-e.

[109-e-R17-FR2-2-07] Email discussion for maintenance on beam management for new SCSs, for issue 7-1 (introduction of beam switching gap) in R1-2205124, until May 13 – Youngwoo (InterDigital)

# Remaining issues

## Introduction of beam switching gap and scheduling restrictions

### Observations and Proposals from Contributions

| ###### Company | ###### Observations and Proposals from Contributions |
| --- | --- |
| ###### [ZTE/Sanechips, 1] | Proposal 3: It is proposed to introduce a UE capability for supporting the UE reporting the beam switching time. |
| ###### [NTT Docomo, 5] | Proposal 1: - Support a UE capability for Rx and Tx beam switching time between adjacent DL signals/channels and adjacent UL signals/channels, respectively. - Number of symbols for the beam switching time is up to UE feature discussion, and possibly RAN4 progress. - The beam switching time should be considered for two adjacent signals/channels with different QCL Type-D assumptions. |
| ###### [LGE, 6] | Proposal #1: Support a UE capability for Rx and Tx beam switching time between adjacent DL signals/channels and adjacent UL signals/channels, respectively. |
| ###### [Huawei/HiSi, 7] | Proposal 2: Regarding beam switch time, support the following for 960k Hz and 480 kHz SCSs - UE is not expected to be scheduled/configured with a signal/channel on one symbol before to and one symbol after of another signal/channel if the signals/channels have two different QCL-D assumptions. |

### Summary of views

In [LGE, 6], it is pointed out that UE may require beam switching time which is longer than gNB beam switching time 59 ns which is tentatively agreed in RAN4.

Different from Rel-15/16 beam management operation, beam switching time (i.e., required time to change phase shifters) should be considered at least for 960 kHz SCS, and in addition, for 480 kHz SCS when applicable. Even though RAN4 tentatively agreed on 59 ns beam switching time at gNB side [1], UE may require beam switching time longer than gNB or have different beam switching time capabilities depending on how to implement. Therefore, it should be discussed how to handle beam switching time or how to allocate symbol-level gap between DL/UL signals/channels. One possible approach could be to define UE capabilities for beam switching time and corresponding UE behaviour when receiving adjacent DL signals/channels (or transmitting adjacent UL signals/channels) considering the reported beam switching time.

In addition, [ZTE/Sanechips, 1], [NTT Docomo, 5] and [Huawei/HiSi, 7] also proposed to support UE capability signaling to report beam switching time and corresponding scheduling restrctions.

### Discussion

Given the support from multiple companies, the proposal from [LGE, 6] is copied in the below for further discussion.

#### Proposal 2.1-a

Support a UE capability for Rx and Tx beam switching time between adjacent DL signals/channels and adjacent UL signals/channels, respectively.

| Company | Input |
| --- | --- |
| Ericsson | RAN1 has still not received feedback from RAN4 on UE beam switching time. In our view it is premature for RAN1 to make agreements related to beam switching gaps and associated capabilities. If there is a need for beam switching gaps, it places quite heavy scheduling restrictions on the gNB, and we do not want to make agreements on this without knowledge of concrete numbers. Hence, we do not support Proposal 2.1-a and prefer to wait until RAN4 provides feedback. |
| ZTE, Sanechips | We support Proposal 2.1-a. We are also fine to wait for RAN4’s reply. |
| LG Electronics | We are also OK to wait for RAN4’s response, since we may not need new UE capability in the end if only one value for RX/TX beam switching time is defined for each SCS. However, what is more important thing in our view is corresponding UE behavior when RX/TX beam switching time is longer than CP duration. To be specific, as illustrated in our Tdoc R1-2204615, we need to define UE behavior (e.g., drop CG-PUSCH or drop DG-PUSCH depending on PUSCH cancellation time) when CG-PUSCH and DG-PUSCH transmissions are consecutively assigned and the reported beam switching time between CG-PUSCH and DG-PUSCH is not guaranteed. |
| Samsung | In our view, if RAN4 decides beam switching gap is required, one symbol should be reserved to switch transmission beams, including data beam at least for the two signlas/channels with different QCL type-D assumptions. However, we are not sure which SCS is applied or how many symbols are reserved. Therefore, we prefer to hold the discussion until RAN4’s response. |
| vivo | We prefer to wait for RAN4’s agreement before decide a UE capability for it in RAN1. Based on our understanding of related discussion in RAN4, the switching time for UE may be more than CP length for SCS 480/960 kHz but within one symbol. If that’s the case, we don’t really see strong need to define a UE capability if just one value. |
| InterDigital | We support the proposal, but fine to wait until RAN4’s response on UE beam switching time. |
| CATT | We prefer to wait for RAN4 feedback. It is our understanding that ran1 may need to further discuss depending ran4 feedback but we may not need to introduce UE capabilities. |
| QC | We are fine to wait for RAN4 feedback. In addition, if beam switch gap is to be introduced, we prefer to clarify the scenarios for the gap to be applied. Otherwise, it is likely to have beam misalignment at both sides. |
| DOCOMO | Ok with wating for RAN4 feedback. Sending LS to ask RAN4 feedback could be considered as we did for SRS antenna switching GP. For the need of UE capability, it is of course dependent on RAN4. Our understanding is that RAN4 defines requirements to consier any type of UEs. If the need of guard period for beam switching is observed, we assume RAN4 is going to define it for UEs with minimum capability. In this case, operators would be very happy if some vendors can implement UE which doesn’t need such guard period between different beams. In this case, differentiation between UEs capable of that and not capable of that will be necessary. |

### Conclusion

Based on the inputs from multiple companies, the moderator suggests to hold the discussion until RAN4 feedback.

# References

- R1-2203294, Remaining issues on beam management for 52.6 to 71GHz, ZTE, Sanechips,

- R1-2203373, Remaining issues for beam management for new SCSs, InterDigital, Inc.,

- R1-2203512, Maintenance on beam management, vivo,

- R1-2204114, Remaining issues for beam management, Ericsson,

- R1-2204342, Remaining issues on beam based operation for new SCSs for NR from 52.6 to 71 GHz, NTT DOCOMO, INC.,

- R1-2204603, Supported values for SSSG switching delay for 480 kHz and 960 kHz SCS, Nokia, Nokia Shanghai Bell,

- R1-2204615, Remaining issues of beam management to support NR above 52.6 GHz, LG Electronics,

- R1-2204896, Remaining issues of PUCCH enhancement and beam management enhancement for 52-71GHz spectrum, Huawei, HiSilicon.