3GPP TSG RAN WG1 #109-e			R1-2205261

e-Meeting, May 9th –  20th, 2022

Agenda item:	8.1.3

Source: 	Moderator (ZTE)

Title: 	Endorsed TPs for Maintenance on Rel-17 SRS

Document for:	Discussion and Decision

- Introduction

To endorse the following TPs (i.e., TP#1-2.B, TP#2-5, TP#3-2, TP#3-3 and TP#3-4) in Moderator summary #1 for Maintenance on Rel-17 SRS [1].

- Summary of endorsed TPs

TP#1-2.B in TS38.214

Reason of change:

For antenna switching T=R case, in Rel-15, up to two SRS resource sets are supported, including 2 periodic, or two aperiodic, or 1 periodic+1 aperiodic, etc. In Rel-17, when UE has the capability of “Maximum 2 SP and 1 periodic SRS sets for antenna switching”, the legacy configurations should be maintained, i.e., the configurations of 2 periodic, or 2 aperiodic, or 1 periodic+1 aperiodic should be supported. However, the mentioned legacy SRS configurations are missing for Rel-17 UE in the current wording of spec.

Summary of change:

Clarify in TS38.214 that the configurations of 2 periodic, or 2 aperiodic, or 1 periodic+1 aperiodic should be supported when the UE has the capability of “Maximum 2 SP and 1 periodic SRS sets for antenna switching”.

Consequence if not approved:

For T=R, the legacy configurations of 2 periodic, or 2 aperiodic, or 1 periodic+1 aperiodic are missing for Rel-17 UE with the capability of “Maximum 2 SP and 1 periodic SRS sets for antenna switching”.

TP#1-2.B: SRS configuration for T=R antenna switching

<Start of TP for TS38.214> 6.2.1.2 UE sounding procedure for DL CSI acquisition <Unrelated part omitted> For 1T=1R, or 2T=2R, or 4T=4R, up to two SRS resource sets each with one SRS resource can be configured, where the number of SRS ports for each resource is equal to 1, 2, or 4, if the UE is not indicating a capability for [maximum 2 semi-persistent and maximum 1 periodic SRS resource sets]., or up to tTwo SRS resource sets configured with resourceType in SRS-ResourceSet set to 'semi-persistent' and up to one SRS resource set configured with resourceType in SRS-ResourceSet set to 'periodic' can be configured and the two SRS resource sets configured with 'semi-persistent' are not activated at the same time, or up to two SRS resource sets can be configured, if the UE is indicating a capability for [maximum 2 semi-persistent and maximum 1 periodic SRS resource sets], where each SRS resource set has one SRS resource, the number of SRS ports for each resource is equal to 1, 2, or 4 where the two SRS resource sets configured with 'semi-persistent' are not activated at the same time, or <Unrelated part omitted> <End of TP for TS38.214>

TP#2-5 in TS38.214

Reason of change:

The capability for antenna switching with xTyR is denoted as “txry” in Rel-16. In Rel-17, '1t8r' for 1T8R, '2t6r' for 2T6R, '2t8r' for 2T8R, '4t8r' for 4T8R are used in the spec, which is not aligned with that for xTyR in Rel-16.

Summary of change:

Change “ '1t8r' for 1T8R, '2t6r' for 2T6R, '2t8r' for 2T8R, '4t8r' for 4T8R” to “ 't1r8' for 1T8R, 't2r6' for 2T6R, 't2r8' for 2T8R, 't4r8' for 4T8R” in TS38.214.

Consequence if not approved:

The parameter of UE’s capability in TS38.214 for 1T8R, 2T6R, 2T8R, and 4T8R would not be aligned with the specs for UE’s capability.

TP#2-5: SRS for antenna switching

<Start of TP for TS38.214> 6.2.1.2 UE sounding procedure for DL CSI acquisition <Unrelated part omitted> When the UE is configured with the higher layer parameter usage in SRS-ResourceSet set as 'antennaSwitching', the UE may be configured with only one of the following configurations depending on the indicated UE capability supportedSRS-TxPortSwitch ('t1r2' for 1T2R, 't1r1-t1r2' for 1T=1R/1T2R, 't2r4' for 2T4R, 't1r4' for 1T4R, 't1r6' for 1T6R, '1t8r' 't1r8' for 1T8R, '2t6r' 't2r6' for 2T6R, '2t8r' 't2r8' for 2T8R, '4t8r' 't4r8' for 4T8R, 't1r1-t1r2-t1r4' for 1T=1R/1T2R/1T4R, 't1r4-t2r4' for 1T4R/2T4R, 't1r1-t1r2-t2r2-t2r4' for 1T=1R/1T2R/2T=2R/2T4R, 't1r1-t1r2-t2r2-t1r4-t2r4' for 1T=1R/1T2R/2T=2R/1T4R/2T4R, 't1r1' for 1T=1R, 't2r2' for 2T=2R, 't1r1-t2r2' for 1T=1R/2T=2R, 't4r4' for 4T=4R, or 't1r1-t2r2-t4r4' for 1T=1R/2T=2R/4T=4R): <Unrelated part omitted> <End of TP for TS38.214>

TP#3-2 in TS38.211

Reason of change:

It has been agreed that the candidate value of PF including 2 and 4, however, the candidate values are not well captured in TS38.211 as the other parameters.

Summary of change:

Clarify in TS38.211 that the candidate value of PF including 2 and 4.

Consequence if not approved:

The candidate value of PF will be ambiguous in TS38.211.

TP#3-2: Partial frequency hopping

<Start of TP for TS38.211> 6.4.1.4.3 Mapping to physical resources <Unrelated part omitted> When SRS is transmitted on a given SRS resource, the sequence rpi(n,l') for each OFDM symbol l' and for each of the antenna ports of the SRS resource shall be multiplied with the amplitude scaling factor in order to conform to the transmit power specified in [5, 38.213] and mapped in sequence starting with to resource elements in a slot for each of the antenna ports according to The length of the sounding reference signal sequence is given by Msc,bSRS=mSRS,bNscRBKTCPF where mSRS,b is given by a selected row of Table 6.4.1.4.3-1 with where is given by the field b-SRS contained in the higher-layer parameter freqHopping if configured, otherwise BSRS=0. The row of the table is selected according to the index given by the field c-SRS contained in the higher-layer parameter freqHopping. The quantity PF∈{2,4} is given by the higher-layer parameter FreqScalingFactor if configured, otherwise PF=1. When FreqScalingFactor is configured, the UE expects the length of the SRS sequence to be a multiple of 6. <Unrelated part omitted> <End of TP for TS38.211>

TP#3-3 in TS38.214

Reason of change:

When R is larger than 2, the wording “pair” is conflicting with R>2 adjacent OFDM symbols since a pair of symbols indicates two symbols.

Summary of change:

Change “pair” to “set” to avoid the confliction between “pair” and R adjacent symbols in TS38.214.

Consequence if not approved:

The wording is conflicting with itself in TS38.214.

TP#3-3: SRS frequency hopping

<Start of TP for TS38.214> 6.2.1.1 UE SRS frequency hopping procedure <Unrelated part omitted> For a given SRS resource, the UE is configured with repetition factor R∈{1,2,4} or R∈{1,2,3,4,5,6,7,8,10,12,14} by higher layer parameter resourceMapping in SRS-Resource where R≤Ns. When frequency hopping within an SRS resource in each slot is not configured (R=Ns), each of the antenna ports of the SRS resource in each slot is mapped in all the symbols to the same set of subcarriers in the same set of PRBs. When frequency hopping within an SRS resource in each slot is configured without repetition (R=1), according to the SRS hopping parameters , and defined in clause 6.4.1.4 of [4, TS 38.211], each of the antenna ports of the SRS resource in each slot is mapped to different sets of subcarriers in each OFDM symbol, where the same transmission comb value is assumed for different sets of subcarriers. When both frequency hopping and repetition within an SRS resource in each slot are configured (Ns ≥ 4, R ≥ 2), each of the antenna ports of the SRS resource in each slot is mapped to the same set of subcarriers within each pair set of R adjacent OFDM symbols, and frequency hopping across the pairs sets is according to the SRS hopping parameters , and , where should be divisible by . For operation with shared spectrum channel access, the UE does not expect that multiple hops of an SRS resource transmission are in different RB sets. A UE may be configured adjacent symbol aperiodic SRS resource with intra-slot frequency hopping within a bandwidth part, where the full hopping bandwidth is sounded with an equal-size subband across symbols when frequency hopping is configured with R=1. A UE may be configured Ns ≥ 4 adjacent symbols aperiodic SRS resource with intra-slot frequency hopping within a bandwidth part, where the full hopping bandwidth is sounded with an equal-size subband across pairs sets of R adjacent OFDM symbols, when frequency hopping is configured with R ≥ 2, Ns ≥ R and Ns should be divisible by R. Each of the antenna ports of the SRS resource is mapped to the same set of subcarriers within each pair set of R adjacent OFDM symbols of the resource. A UE may be configured symbol periodic or semi-persistent SRS resource with inter-slot hopping within a bandwidth part, where the SRS resource occupies the same symbol location in each slot. A UE may be configured symbol periodic or semi-persistent SRS resource with intra-slot and inter-slot hopping within a bandwidth part, where the N-symbol SRS resource occupies the same symbol location(s) in each slot. For Ns ≥ 4, when frequency hopping is configured with R ≥ 2, intra-slot and inter-slot hopping is supported with each of the antenna ports of the SRS resource mapped to different sets of subcarriers across pairs sets of R adjacent OFDM symbol(s) of the resource in each slot, where should be divisible by R. Each of the antenna ports of the SRS resource is mapped to the same set of subcarriers within each pair set of R adjacent OFDM symbols of the resource in each slot. For Ns= R, when frequency hopping is configured, inter-slot frequency hopping is supported with each of the antenna ports of the SRS resource mapped to the same set of subcarriers in R adjacent OFDM symbol(s) of the resource in each slot. <Unrelated part omitted> <End of TP for TS38.214>

TP#3-4 in TS38.214

Reason of change:

In Rel-17, start RB hopping was supported for partial sounding. Accordingly, a new parameter was introduced to define the hopping pattern. In 38.211, the hopping pattern is denoted as khop, while in 38.214, the hopping pattern is denoted as khopping. So it’s necessary to align it in the different spec.

Summary of change:

Clarify in TS38.214 that the hopping pattern is denoted as khop to keep alignment with TS38.211.

Consequence if not approved:

The parameter name for start RB hopping pattern is not aligned in 38.211 and 38.214, which may lead to the misunderstanding.

TP#3-4: Parameter alignment for start RB hopping

<Start of TP for TS38.214> 6.2.1 UE sounding procedure <Unrelated part omitted> - Defining start RB index hopping for partial frequency sounding in different SRS frequency hopping periods for aperiodic/periodic/semi-persistent SRS based on the hopping pattern khopkhopping as described in clause 6.4.1.4.3 in [4, TS 38.211. If not configured, then start RB hopping is not enabled and khopkhopping is fixed to be 0 for all SRS symbols. <Unrelated part omitted> <End of TP for TS38.214>

- References

- R1-2205195 Moderator summary #1 for Maintenance on Rel-17 SRS, Moderator (ZTE).