3GPP TSG-RAN WG1 Meeting #109-e	R1- 2205264

e-Meeting, May 9th – 20th, 2022

Agenda Item:	8.3.3

Source:	Moderator (Ericsson)

Title:	Summary#2: [109-e-R17-IIoT-URLLC-04] Maintenance of URLLC/IIoT operation on unlicensed band

WI code:	NR_IIOT_URLLC_enh

Document for:	Discussion, Decision

# Introduction

This document provides the summary of the discussions during RAN1#109-e under the following email thread assigned by RAN1 Chair:

[109-e-R17-IIoT-URLLC-04] Email discussion on unlicensed band URLLC/IIoT (issues captured in R1-2105103) by May 18 – Sorour (Ericsson)

- Issue#1: COT association for PUSCH scheduled via RAR (R1-2204618)

- Issue#2: Correction of maximum value of RRC parameter offsetUE (R1-2204893) for IIoT/URLLC RRC parameters Email discussion during RAN1#109-e if it is not resolved by RAN2 by start of RAN1#109-e.

- Issue#3: Editorial TPs for 37.213 (R1-2204893) conditioned that are quickly supported without controversy. Otherwise drop Issue#3 for discussion.

- 1st check point on May 12 and final check point on May 18

This document is a revision of R1-2205263.

# Discussion

## Issue#1 & Issue#3 – First round

Issue#1: COT association for PUSCH scheduled via RAR (R1-2204618)

Issue#3: Editorial TPs for 37.213 (R1-2204893) conditioned that are quickly supported without controversy. Otherwise drop Issue#3 for discussion

Moderator has provided a draft CR for both Issue#1 and Issue#3. Please note that some of the editorial suggestions (adding “,” in few occasions) are not implemented. Moreover, “u” is changed to “µ” for the time unit.

Moderator’s recommendation:

Please review the draft CR uploaded as V000 for endorsement by chair and provide your comments, including any information and description on the cover page as well as the TPs.

| Company | Comment |
| --- | --- |
| Huawei, HiSilicon | We support the TP for Issue#1 in R1-2204618 We also support the editorial TPs in R1-2204893 as the proponent for clarity and consistency of the respective clauses in the spec. We are fine with not adding the suggested “,” in few occasions. For the draft CR, as per the following guidance from Chairman, we think that the group cannot agree a draft CR, but can only agree on the TPs. So, our understanding is that if the TPs are agreeable, our Editor would use the prepared draft as the Alignment CR. Only text proposals are to be submitted (no individual draft CRs, please!). - For each text proposal, companies are to provide relevant information (e.g. reason for change, summary of change, consequences if not approved) in a clear and concise manner - Editors to prepare final CRs |
| Intel | We are OK to support TPs for issue#1 in R1-2204618, but we still believe some of the editorials in R1-2204893 are not really needed. However, if other companies believe those may help readability of the spec, we are fine with them. As for the CR, we also have same understanding as Huawei. |
| Futurewei | We are OK with TP for Issue#1 in R1-2204618; we are fine with the editorial TPs in R1-2204893. |
| vivo | We are fine with the TP for issue#1: COT association for PUSCH scheduled via RAR (R1-2204618). For editorials in R1-2204893, we do not think it is needed. But we are fine with it. |
| ZTE | We are OK with TP for Issue#1 in R1-2204618; As for the editorial changes, although they seem not essential, we can live with them. |
| Apple | We are OK with TP for Issue#1 in R1-2204618. For editorials in R1-2204893, we think most of them are not needed except the change of 16uμs. We can accept it if majority companies are fine. |
| Samsung | We are OK with TP for Issue#1 in R1-2204618. |
| WILUS | We are OK with the TP for Issue#1 in R1-2204618. For the editorial changes in R1-2204893, the draft CR uploaded by moderator is OK to us. |
| New H3C | We are fine with TP for Issue#1 in R1-2204618. For the editorial changes in R1-2204893, we are fine with it |
| LG | We are fine with both TP for Issue#1 in R1-2204618 and the TPs in R1-2204893. |
| Moderator | Thank you all for reviewing the YPs and also your flexibility in considering Editorial changes. I will ask Chair for email approval of the TP. Thanks for bringing to my attention that the draft CR was not needed. I guess Editor can enjoy it |

## Issue#2 – First round

Issue#2: Correction of maximum value of RRC parameter offsetUE (R1-2204893) for IIoT/URLLC RRC parameters Email discussion during RAN1#109-e if it is not resolved by RAN2 by start of RAN1#109-e.

Regarding this issue, Moderator’s understanding of RAN2 situation is as the following:

- Issue#2 is addressed by Rapporteur of RAN2 Running CR for IIOT/URLLC RRC parameters.

- Please see the Rapporteur summary in R2-2205506 and the proposed CR for Approval in R2-2205507.

- According to RAN2 procedures, a single CR covering all RRC issues would be approved only after the RAN2 meeting (and hence after the RAN1 meeting as well).

Moderator’s recommendation: From Modoerator’s perspective, it seems this issue would be resolved with or without RAN1 LS. If the group agrees, Moderator recommends to include the suggested correction in the overall IioT/URLLC LS to send to RAN2. For the LS, the proposal is as the following:

Proposal:

Include in the IioT/URLLC RRC parameters LS to RAN2, the following clarification:

- The proper maximum values for the parameter offsetUE in SemiStaticChannelAccessConfigUE are 139/279/559 for 15/30/60kHz SCS rather than 279/559/1119, and the proper value range is thus 559 rather than 1119.

Please share your view regarding the recommendation and proposal above.

| Company | Comment |
| --- | --- |
| Huawei, HiSilicon | We agree with Moderator’s recommendation and we support the proposal as the proponent. |
| Sony | Agree with the proposal |
| Intel | We are OK with the Moderator’s recommendation, and with the proposal. |
| Futurewei | We are OK with Moderator’s recommendation and proposal. |
| Vivo | We agree with Moderator’s recommendation and the proposal. |
| ZTE | We are OK with Moderator’s recommendation and proposal. |
| Apple | OK |
| Samsung | We are fine with Moderator’s recommendation and proposal. |
| WILUS | We are OK with Moderator’s recommendation and proposal. |
| New H3C | We are fine with it. |
| LG | We are fine with Moderator’s recommendation and proposal. |
| Moderator | Thank you all for supporting the proposal. Moderator requests Chair for email approval of the proposal. |

# Email Approval

Based on the discussions in Section 2, the following proposals are acceptable by the group.

Proposal 1:

Include in the IIoT/URLLC RRC parameters LS to RAN2, the following clarification:

- The proper maximum values for the parameter offsetUE in SemiStaticChannelAccessConfigUE are 139/279/559 for 15/30/60kHz SCS rather than 279/559/1119, and the proper value range is thus 559 rather than 1119.

Proposal 2:

- Adopt the following text proposal for Clause 4.3 and Clause 4.3.1 of TS 37.213 V 17.1.0.

| Reason for change: | 1) For semi-static channel access procedures when a UE can initiate a channel occupancy, the UE behaviour to determine the associated channel occupancy for a scheduled UL transmission in Clause 4.3.1.2.4 are specified according to whether the scheduled UL transmission and the corresponding scheduling DCI are confined within same gNB FFP or within different gNB FFP. The current description in the specification does not cover the case where the scheduled UL transmission is a PUSCH transmission that is not scheduled by a DCI, but it is scheduled via the corresponding RAR (during RACH procedure). 2) Some editorial corrections for clarity, alignment with similar clauses, and accuracy of the specifications |
| --- | --- |
| Summary of change: | 1) Add the description to include the case that the schedule UL transmission is scheduled by a RAR message in Clause 4.3.1.2.4 and follow-up clauses 4.3.1.2.4.1 and 4.3.1.2.4.2. 2) Improve accuracy and readability of the specification by editorial changes |
| Consequences if not approved: | In case of semi-static channel access procedures when a UE can initiate a channel occupancy, the UE behaviour to determine the channel occupancy association for a UL transmission that is scheduled by a RAR message would be unspecified. |

==================START of Text Proposal for TS 37.213 V 17.1.0 ============================

4.3	Channel access procedures for semi-static channel occupancy

Channel access procedures based on semi-static channel occupancy as described in this Clause, are intended for environments where the absence of other technologies is guaranteed e.g., by level of regulations, private premises policies, etc.

If a gNB provides UE(s) with higher layer parameters ChannelAccessMode-r16 ='semiStatic' by SIB1 or dedicated configuration for a serving cell, a periodic channel occupancy can be initiated by the gNB on a channel(s) within the bandwidth of the serving cell every Tx within every two consecutive radio frames, starting from the even indexed radio frame at i∙Txx∙Tx with a maximum channel occupancy time Ty=0.95Tx, where Tx= period Tx=Periodin ms, is a higher layer parameter provided in SemiStaticChannelAccessConfig and i∈0,1,…,20Tx-1. A duration of  Tz=max0.05Tx , 100uμs at the end of a period is referred to as the idle duration of that period.

If the gNB additionally configures a UE with higher layer parameter ue-SemiStaticChannelAccessConfig consisting of ue-Period and ue-Offset, the UE can initiate a channel occupancy on a channel(s) within the bandwidth of the serving cell every Tu  = ue-Period in  ms with corresponding maximum channel occupancy time Tv=0.95Tu. The offset of the periodic channel occupancy is determined by To= ue-Offset as the number of symbols from the beginning of an even indexed radio frame to the start of the first period in that radio frame in which the UE can initiate a channel occupancy. A duration of Tw=max0.05Tu , 100μus  at the end of a period is referred to as the idle duration of that period.

For determining a Channel Occupancy Time based on semi-static channel access procedures, duration of any transmission gap within a period excluding the corresponding idle duration is counted in the channel occupancy time.In the following procedures in this clause, when a gNB or UE performs sensing for evaluating a channel availability, the sensing is performed at least during a sensing slot duration Tsl=9μus, unless longer sensing duration is required (e.g. by level of regulation), in which case sensing is performed within a duration of Tsl=16μus. When sensing is performed within a duration of Tsl=16μus, the channel is considered to be idle if the channel is sensed to be idle for total of at least 5μus with at least 4μus of sensing occurring in the last 9μus time interval in the sensing duration. The corresponding  XThresh adjustment for performing sensing by a gNB or a UE is described in clauses 4.1.5 and 4.2.3, respectively.

4.3.1	Channel access procedures to initiate a channel occupancy

For semi-static channel occupancy, the procedures in Clause 4.3.1.1 are followed if ue-SemiStaticChannelAccessConfig is absent. Otherwise, the procedures in Clause 4.3.1.2 are applicable.

If a UE fails to access the channel(s) prior to an intended UL transmission to a gNB, Layer 1 notifies higher layers about the channel access failure.

4.3.1.1	Channel occupancy initiated only by gNB

A channel occupancy that is initiated by a gNB and is shared with UE(s), satisfies the following:

-	The gNB shall transmit a DL transmission burst starting at the beginning of a period of duration Tx in which the channel occupancy is initiated immediately after sensing the channel to be idle for at least a sensing slot duration Tsl. If the channel is sensed to be busy, the gNB shall not perform any transmission during the current period.

-	The gNB may transmit a DL transmission burst(s) within the channel occupancy time immediately after sensing the channel to be idle for at least a sensing slot duration Tsl if the gap between the DL transmission burst(s) and any previous transmission burst is more than 16uμs.

-	The gNB may transmit DL transmission burst(s) after UL transmission burst(s) within the channel occupancy time without sensing the channel if the gap between the DL and UL transmission bursts is at most 16μus.

-	A UE may transmit UL transmission burst(s) after detection of a DL transmission burst(s) within the channel occupancy time as follows:

-	If the gap between the UL and DL transmission bursts is at most 16μus,  the UE may transmit UL transmission burst(s) after a DL transmission burst(s) within the channel occupancy time without sensing the channel.

-	If the gap between the UL and DL transmission bursts is more than 16μus,  the UE may transmit UL transmission burst(s) after a DL transmission burst(s) within the channel occupancy time after sensing the channel to be idle for at least a sensing slot duration Tsl within a 25μus interval ending immediately before transmission.

-	A UE may be indicated by the gNB to transmit UL transmission burst(s) within the channel occupancy time without sensing the channel or after sensing the channel to be idle for at least a sensing slot duration Tsl within a 25μus interval ending immediately before transmission.

-	The gNB and UEs shall not transmit any transmissions in a set of consecutive symbols for a duration of at least Tz=max0.05Tx , 100μus before the start of the next period.

4.3.1.2		Channel occupancy initiated by gNB or UE

4.3.1.2.1	Channel occupancy initiated by gNB and sensing procedures

The gNB initiates a channel occupancy in a period of duration  Tx if the gNB transmits a DL transmission burst starting at the beginning of the period immediately after sensing the channel to be idle for at least a sensing slot duration Tsl and ends the transmission of the DL transmission burst before the start of the idle duration of that period. When the gNB initiates a channel occupancy in that period, the gNB shall not transmit any transmission(s) within the idle duration of that period. A UE determines that the gNB has initiated the channel occupancy in that period by detection of a DL transmission burst(s) in that period or an indication by a DCI as described in Clause 4.3.1.2.4.

When a UL or DL transmission burst(s) is associated with the channel occupancy that is initiated in that period by the gNB, the following are applicable:

-	The UL or DL transmission burst(s) is confined within that period and ends before the start of the idle duration of that period.

-	If the gap between the DL transmission burst(s) and any previous transmission burst in that period is more than 16uμs, the DL transmission burst(s) may be transmitted if the channel is sensed to be idle for at least a sensing slot duration Tsl=9usTsl immediately before the DL transmission.

-	If the gap between the DL transmission burst(s) and any previous UL transmission burst in that period is at most 16uμs, the DL transmission burst(s) may be transmitted without sensing.

-	If the gap between the UL transmission burst(s) and any previous DL transmission burst in that period is more than 16uμs, the UL transmission burst(s) may be transmitted if the channel is sensed to be idle for at least a sensing slot duration Tsl=9usTsl within a 25µs 25usinterval ending immediately before the UL transmission burst(s).

-	If the gap between the UL transmission burst(s) and any previous DL transmission burst in that period is at most 16uμs, the UL transmission burst(s) may be transmitted without sensing.

4.3.1.2.2	Channel occupancy initiated by UE and sensing procedures

A UE initiates a channel occupancy in a period of duration Tu if the UE transmits a UL transmission burst starting at the beginning of the period immediately after sensing the channel to be idle for at least a sensing slot duration Tsl=9usTsl and ends the transmission of the UL transmission burst before the start of the idle duration of that period. When the UE initiates a channel occupancy in that period, the UE shall not transmit any transmission(s) within the idle duration of that period.

When a UL or DL transmission burst(s) is associated with the channel occupancy that is initiated in that period by the UE, the following are applicable:

-	The UL or DL transmission burst(s) is confined within that period and ends before the start of the idle duration of that period.

-	If the gap between the UL transmission burst(s) and any previous transmission burst in that period is more than 16uμs, the UL transmission burst(s) may be transmitted if the channel is sensed to be idle for at least a sensing slot duration Tsl=9usTsl immediately before the UL transmission burst(s).

-	If the gap between the UL transmission burst(s) and any previous DL transmission burst in that period is at most 16uμs, the UL transmission burst(s) may be transmitted without sensing.

-	If the gap between the DL transmission burst(s) and any previous UL transmission burst in that period is more than 16uμs, the DL transmission burst(s) may be transmitted if the channel is sensed to be idle for at least a sensing slot duration Tsl=9usTsl within a 25µs interval ending immediately before the DL transmission.

-	If the gap between the DL transmission burst(s) and any previous UL transmission burst in that period is at most 16uμs, the DL transmission burst(s) may be transmitted without sensing.

When a DL transmission burst(s) is associated with a channel occupancy that is initiated in a period of duration Tu by a UE, the DL transmission burst(s) shall include unicast user plane data or control information intended for the UE that has initiated the channel occupancy in that period. The gNB may include in the DL transmission burst(s) an additional transmission(s) intended to other UEs than the UE that has initiated the channel occupancy in that period or broadcast transmission(s), only if the gNB satisfies the condition that the detection of the additional DL transmission(s) at any UE will not be associated with a channel occupancy that is initiated by gNB following the procedures described in Clause 4.3.1.2.3 and 4.3.1.2.4.

*** Unchanged text is omitted ***

4.3.1.2.3	Association with initiated channel occupancy for configured UL transmissions

When a UE is configured with a UL transmission, the UE follows the following procedures to determine if the configured UL transmission is associated with a channel occupancy that is initiated by the gNB or the UE.

-	If the configured UL transmission would occur at the beginning of a period of duration Tu  and would end before the idle duration corresponding to that period, the following is applied:

-	If the configured UL transmission would occur within a period of duration Tx and would end before the idle duration corresponding to that period and if the UE has already determined that the gNB has initiated a channel occupancy in that period as described in Clause 4.3.1.2.1, the UE assumes that the configured UL transmission is associated with the channel occupancy that is initiated by the gNB.

-	Otherwise, the UE assumes that the configured UL transmission is associated with a channel occupancy to be initiated by the UE.

*** Unchanged text is omitted ***

4.3.1.2.4	Association with initiated channel occupancy for scheduled UL transmissions

When a UL transmission(s) is scheduled by a DCI or a RAR message, the scheduling DCI or the RAR message indicates the channel access parameters for the UL transmission(s) as described in [10]. Based on the DCI or the RAR message, the UE determines if the scheduled UL transmission(s) is associated with a channel occupancy that is initiated by the gNB or the UE, and whether sensing and CP extension are applicable.

4.3.1.2.4.1	Intra-period scheduled UL transmissions

The procedures in this clause are applicable when a scheduled UL transmission and the corresponding scheduling DCI or the corresponding RAR transmission are confined within the same period of duration Tx corresponding to the carrier within which the UL transmission is scheduled and regardless of whether they are transmitted on the same carrier or different carriers.

*** Unchanged text is omitted ***

4.3.1.2.4.2	Cross-period scheduled UL transmissions

The procedures in this clause are applicable when a scheduled UL transmission and the corresponding scheduling DCI or the corresponding RAR transmission are confined within different periods of duration Tx corresponding to the carrier within which the UL transmission is scheduled and regardless of whether they are transmitted on the same carrier or different carriers.

*** Unchanged text is omitted ***

==================END of Text Proposal for TS 37.213 V 17.1.0 ============================

# Conclusion

The following has been agreed during this email discussion.

Agreement

Include in the IIoT/URLLC RRC parameters LS to RAN2, the following clarification:

- The proper maximum values for the parameter offsetUE in SemiStaticChannelAccessConfigUE are 139/279/559 for 15/30/60kHz SCS rather than 279/559/1119, and the proper value range is thus 559 rather than 1119.

Agreement

The text proposal in Section 3 (Proposal 2) of R1-2205263 is endorsed for the editor’s CRs on TS 37.213 V 17.1.0 (Clause 4.3 and Clause 4.3.1).