3GPP TSG RAN WG1 #109-e                                                              R1-2205286

e-Meeting, May 9th – 20th, 2022

Source:	Moderator (Intel Corporation)

Title:	Endorsed TPs in Rel.17 NR FeMIMO maintenance on HST

Agenda item:	8.1.2

Document for:	Discussion and Decision

# Summary of Endorsed TPs

The following editorial TP for TS 38.214 was discussed in the email thread [109-e-R17-MIMO-06] Maintenance on HST (description of issues in R1-2205145) – Avik (Intel).

The text proposal is available in the summary of first round discussions for maintenance of HST-SFN in R1-2205287 and was endorsed by RAN1 Chairman over email.

TP#5 (for TS 38.214):

Reason for change: In clause 5.2.1.5.1, the text description for specifying the TCI state for the reception of the aperiodic CSI-RS when the CORESET with lowest ID is indicated with two TCSI states has extra “if” wording which is not required

Summary of change: The word “if” after the else condition should be deleted.

Consequences if not approved: If there is no other DL signal with indicated TCI state in the same symbols as the CSI-RS, the UE should apply the first one of two TCI states indicated for the CORESET with the lowest CORESET ID. If the word “if” is not deleted, the case when UE applies the first TCI state of the CORESET with lowest ID is not clear in specification.

TP for 38.214 Section 5.2.1.5.1 ----------------Unchanged part omitted------------------------ - else if a UE is configured with sfnSchemePdcch set to 'sfnSchemeA', it is not configured with enableTwoDefaultTCI-States, and the two TCI states are activated for the CORESET by the activation command as described in clause 6.1.3.x of [10, TS 38.321] - if there is any other DL signal with an indicated TCI state in the same symbols as the CSI-RS, the UE applies the QCL assumption of the other DL signal also when receiving the aperiodic CSI-RS. The other DL signal refers to PDSCH scheduled with an offset larger than or equal to the threshold timeDurationForQCL, as defined in [13, TS 38.306], periodic CSI-RS, semi-persistent CSI-RS, aperiodic CSI-RS in a NZP-CSI-RS-ResourceSet scheduled with offset larger than or equal to the UE reported threshold beamSwitchTiming when the reported value is one of the values {14,28,48} and when enableBeamSwitchTiming is not provided or the NZP-CSI-RS-ResourceSet is configured with the higher layer parameter trs-Info , aperiodic CSI-RS in a NZP-CSI-RS-ResourceSet configured with the higher layer parameter repetition set to 'off' or configured without the higher layer parameters repetition and trs-Info scheduled with offset larger than or equal to 48 when the UE provides beamSwitchTiming-r16 and enableBeamSwitchTiming is provided, aperiodic CSI-RS in a NZP-CSI-RS-ResourceSet configured with the higher layer parameter repetition set to 'on' scheduled with offset larger than or equal to the UE reported threshold beamSwitchTiming-r16 and enableBeamSwitchTiming is provided; - else if, the UE applies the first one of two TCI states indicated for the CORESET with the lowest CORESET ID in the latest slot within the active BWP of the cell in which the CSI-RS is to be received when receiving the aperiodic CSI-RS, ----------------Unchanged part omitted------------------------