3GPP TSG RAN WG1 #109-e															       R1-2205287

e-Meeting, May 9th – 20th, 2022

Source:	Moderator (Intel Corporation)

Title:	Moderator Summary of Round 1 for Rel. 17 NR FeMIMO

Maintenance on HST

Agenda item:	8.1.2

Document for:	Discussion and Decision

# Introduction

The document contains summary of maintenance issues and text proposals (TPs) on enhancements for HST-SFN deployment based on the following email thread assignment:

[109-e-R17-MIMO-06] Maintenance on HST (description of issues in R1-2205145) – Avik (Intel)

- Issues 1, 2, 8 and 9 by May 18

- Editorial Issue 11 by May 11

The summary of the preparation phase with description of issues can be found in R1-2205145. The following is the summary of high priority issues 1, 2, 8 and 9 and editorial 11 identified during the preparation phase.

# High Priority Issues

Issue 1

This issue is for capturing agreements stating combination of SFN PDCCH scheme 1 and single-TRP PDSCH is supported, and the combination of SFN PDCCH TRP-based pre-compensation and single-TRP PDSCH is not supported. The following is the summary if issue 1 from the preparation phase.

Table 1 Summary of Issue 1

| # | Issue (summary of CR proposal) | Company inputs (if any) |
| --- | --- | --- |
|  | In 38.214 Section 5.1, capture agreements stating combination of SFN PDCCH scheme 1 and single-TRP PDSCH is supported, and the combination of SFN PDCCH TRP-based pre-compensation and single-TRP PDSCH is not supported - Alt-1: TP to capture related behavior in R1-2203506 (vivo), R1-2203302 (Spreadtrum) - If a UE is configured with sfnSchemePdcch set to 'sfnSchemeA' for a DL BWP and activated with two TCI states by MAC CE, and the UE does not report its capability of [nonSfnPdsch-sfnPdcch], the UE does not expect to be indicated with one TCI state in a codepoint of the DCI field 'Transmission Configuration Indication' in DCI format 1_1/1_2. - If a UE is configured with sfnSchemePdcch set to 'sfnSchemeB' for a DL BWP and activated with two TCI states by MAC CE, the UE does not expect to be indicated with one TCI state in a codepoint of the DCI field 'Transmission Configuration Indication' in DCI format 1_1/1_2. - Alt-2: TP to capture related behavior in R1-2204977 (Qualcomm) - If a UE is configured with sfnSchemePdcch set to 'sfnSchemeA' for a DL BWP and CORESET is activated with two TCI states by MAC CE, and the UE does not report its capability of [nonSfnPdsch-sfnPdcch], the UE shall be configured with sfnSchemePdsch set to 'sfnSchemeA'. - If a UE is configured with sfnSchemePdcch set to 'sfnSchemeB' for a DL BWP and CORESET is activated with two TCI states by MAC CE, the UE shall be configured with sfnSchemePdsch set to 'sfnSchemeB'. FL Note: This issue was extensively discussed in the last meeting. Since the agreements do not seem to be reflected in current specification, it seems to be a valid issue. Companies can provide further inputs along with preference of TPs proposed in the alternatives. | Alt-1: - Support: vivo, Spreadtrum, Apple, OPPO, Huawei/HiSi, Ericsson, Xiaomi, ZTE, CATT, Samsung, DOCOMO, Lenovo - Not Support: Alt-2: - Support: Qualcomm - Not Support: |

### Round-1

Given the summary from the preparation phase, Alt-1 seems to be the super-majority view. Therefore, the following is the FL proposal for this round based on the TP provided in R1-2203506:

TP#1:

TS 38.214 5.1 UE procedure for receiving the physical downlink shared channel < Unchanged parts are omitted > When a UE is configured with higher layer parameter sfnSchemePdsch set to either 'sfnSchemeA' or 'sfnSchemeB' for a DL BWP and - if the UE reports its capability of [dynamicSFN], the UE is indicated with one or two TCI state(s) in a codepoint of the DCI field 'Transmission Configuration Indication' in DCI format 1_1/1_2, or - otherwise, the UE is not expected to be indicated with one TCI state per any of TCI codepoint by MAC CE, and the UE is indicated with two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication' in DCI format 1_1/1_2, and the UE procedure for receiving the PDSCH upon detection of a PDCCH follows clause 5.1 and the QCL assumption for the PDSCH as defined in clause 5.1.5. When a UE is configured with both sfnSchemePdsch and sfnSchemePdcch, the UE shall expect that sfnSchemePdsch and sfnSchemePdcch are set to the same scheme, either 'sfnSchemeA' or 'sfnSchemeB'. If a UE is configured with sfnSchemePdcch set to 'sfnSchemeA' for a DL BWP and activated with two TCI states by MAC CE, and the UE does not report its capability of [nonSfnPdsch-sfnPdcch], the UE does not expect to be indicated with one TCI state in a codepoint of the DCI field 'Transmission Configuration Indication' in DCI format 1_1/1_2. If a UE is configured with sfnSchemePdcch set to 'sfnSchemeB' for a DL BWP and activated with two TCI states by MAC CE, the UE does not expect to be indicated with one TCI state in a codepoint of the DCI field 'Transmission Configuration Indication' in DCI format 1_1/1_2. < Unchanged parts are omitted >

| Company | Comment |
| --- | --- |
| Moderator | This issue has been extensively discussed in the previous meeting and there was no consensus. The situation is clear in this meeting i.e., Alt-1 has super majority view, therefore the FL proposal is to agree to TP#1. Companies can provide views and further comments on TP#1 |
| OPPO | Support Alt-1. |
| Samsung | Support Alt-1. |
| Apple | Support Alt-1 |
| Lenovo | Support Alt-1 and proposed TP1. |
| Xiaomi | Support Alt-1 |
| ZTE | Agree with TP#1. |
| Nokia/NSB | Support TP#1 |
| DOCOMO | Support TP#1 |
| CATT | Support TP#1 |
| vivo | Support TP#1 |
| Huawei, HiSilicon | Fine with proposed TP. |
| QC | Don’t support TP. We would like the supporting companies to provide response and clarification to these points/questions: - Provide RAN1 agreement that support this TP language. - When UE is configured with SFN PDCCH and doesn’t support ‘nonSfnPdsch-sfnPdcch’, then what is the PDSCH scheme? Please consider the table below reflecting the supported SFN schemes. - Clarify how this TP apply to the UE supporting dynamic switching between SFN PDSCH and single-TRP PDSCH. PDSCH. Rel-15; SFN scheme A; SFN scheme B. PDCCH; Rel-15; N/A; Supported; Supported. SFN scheme A; Supported; Supported; No supported. SFN scheme B; Not support; Not supported; Supported. |
| LGE | Fine with the TP |
| Mod | TP#1 seems to have super-majority support, but Qualcomm has raised some concerns and despite extensive discussions in the last meeting, we should try to address these concerns. |

Issue 2

The issue is for support of dynamic switching of sTRP and SFN scheme for PDSCH when PDCCH is not configured with SFN. The summary of Issue 2 from preparation phase is provided in Table 2.

Table 2 Summary of Issue 2

| # | Issue (summary of CR proposal) | Company inputs (if any) |
| --- | --- | --- |
|  | UE not capable of sTRP / SFN dynamic switching - Alt-1: When SFN is configured for PDSCH and not configured for PDCCH, dynamic switching between single TRP and SFN should be supported - Alt-2: When SFN is configured for PDSCH and not configured for PDCCH, TCI field may be present in DCI formats 1_1 and 1_2 for a UE that is not capable of dynamic switching FL Note: This issue was extensively discussed in the last meeting with no conclusion. Initial FL assessment is that this may be an essential issue for Rel-17. Companies can provide input if they think otherwise. | Alt-1 - Support: ZTE, Huawei/HiSi, Ericsson, ZTE, DOCOMO - Not Support: QC, Samsung Alt-2 - Support: Lenovo, Apple, OPPO, Xiaomi, LGE, CATT, vivo, Spreadtrum - Not Support: QC, Samsung |

### Round-1

Based on the inputs in the preparation phase, the following alternatives are proposed and inputs from companies are invited for down selection of the alternatives. Alt-1 and Alt-2 are based on the alternatives in Table 2 and Alt-3 is based on inputs from Qualcomm and Samsung who did not agree with either alternative.

Proposal#2

When SFN is configured for PDSCH and not configured for PDCCH

- Alt-1: Dynamic switching between single-TRP and SFN should be supported

- Alt-2: Support of dynamic switching between single-TRP and SFN is based on UE capability and TCI field may be present in DCI formats 1_1 and 1_2 for a UE that is not capable of dynamic switching

- Alt-3: Support of dynamic switching between single-TRP and SFN is based on UE capability and is independent of SFN configuration of PDCCH

| Company | Comment |
| --- | --- |
| Moderator | Alt-1 and Alt-2 are based on the alternatives in Table 2 and Alt-3 is based on inputs from Qualcomm and Samsung who did not agree with either alternative and suggested that dynamic switching between sTRP and SFN should not depend on the SFN configuration of PDCCH. This issue can benefit from further discussion. Once the alternatives are down-selected, an appropriate TP will be further discussed. |
| OPPO | Support Alt-2. |
| Samsung | Support Alt-3. We think that discussing a further condition on dynamic switching is not appropriate in the maintenance phase. [Mod: Based on current inputs, since SFN PDSCH + single TRP PDCCH is agreed and dynamic switching between single TRP and SFN as UE capability is also agreed, my understanding is that Alt-3 will be the default option and up to gNB implementation if no consensus can be reached. Check reply from DOCOMO]. |
| Apple | Support Alt-3 |
| Lenovo | Prefer Alt-2. |
| Xiaomi | Prefer Alt-2. Not clear about the meaning of Alt -3. |
| ZTE | Support Alt-1. For Alt-2, if UE is not capable of dynamic switching, maybe only one codepoint is activated with two TCI states, in such case there is no TCI field in DCI based on Rel-16 rule, so we think no need to make such restriction that TCI field should be present in DCI format 1_1 and 1_2 in such case. [Mod: this is a valid point. We can update the condition of Alt-2 to state that TCI field should be present in the DCI for the case when more that one TCI codepoint is activated with two TCI states] For Alt-3, if UE is indicated by DCI format 1_0, single-TRP based PDSCH transmission should be supported for Rel-16, so we think dynamic switching should always be supported for PDSCH scheduled in DCI format 1_0 by default. For DCI format 1_1 and 1_2, dynamic switching should be supported based on UE capability. [Mod: Check my response to Samsung and response from DOCOMO below] |
| Nokia/NSB | Support Alt1. Please clarify Alt-3, we cannot understand the operation. [Mod: Alt-3 was meant to capture input from Samsung and Qualcomm in initial rounds, but based on my reply to Samsung, input from DOCOMO and my assessment at the end of the table, it may not be required] |
| DOCOMO | Support Alt1. For Alt.2, we agree with ZTE. For Alt.3, for UE not capable of dynamic switching, is it the intention that gNB should configure TCI state field for DCI format 1_1/1_2, where all TCI codepoints have the two TCI states. If so, it should be up to gNB implementation, and we don’t need to agree Alt.3 in RAN1. [Mod: This is a valid point. Alt-3 may not be needed. In my understanding, Alt-3 is a default option if no consensus can be reached.] |
| CATT | We prefer Alt 2. For Alt 1, if UE not capable of sTRP / SFN dynamic switching, does it mean that the combination of SFN-ed PDSCH and non-SFN PDCCH always cannot be configured? Based the agreement in the previous meetings, the combination of SFN-ed PDSCH and non-SFN PDCCH was already supported without UE capability of sTRP / SFN dynamic switching. If we misunderstanding this alternative, please clarify it. [Mod: This is a valid point. My understanding also the same. Revised Proposal#2 reflects this] |
| vivo | We prefer Alt 2. Additionally, it seems the condition “the scheduling offset is equal or larger than threshold timeDurationForQCL if applicable” is missed in the main bullet. [Mod: This condition has been included in revised Proposal#2] |
| Huawei, HiSilicon | We prefer Alt 1. As mentioned by ZTE, Alt 2 missed the case if only one codepoint is configured. Alt 3 would requires some complicated discussion on default beams, which should be avoided at this stage. |
| QC | Support Alt 3. It was agreed that for sfnPDSCH + nonSFNpdcch that TCI field is always present in the DCI. If UE supports dynamic switching, the TCI codepoint could indicate either single TCI states or two TCI states. If UE doesn’t support dynamic switching, then UE doesn’t expect single TCI state codepoint. Alt 3 is the default behavior based on current specification that describes UE dynamic switching only based on SFN PDSCH configuration. [Mod: This is also my understanding, hence if we do not reach consensus, Alt-3 is default and may not need any agreement] 38.214 – clause 5.1 ….. When a UE is configured with higher layer parameter sfnSchemePdsch set to either 'sfnSchemeA' or 'sfnSchemeB' for a DL BWP and - if the UE reports its capability of [dynamicSFN], the UE is indicated with one or two TCI state(s) in a codepoint of the DCI field 'Transmission Configuration Indication' in DCI format 1_1/1_2, or - otherwise, the UE is not expected to be indicated with one TCI state per any of TCI codepoint by MAC CE, and the UE is indicated with two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication' in DCI format 1_1/1_2, and 38.214 – clause 5.1.5 …. When a UE is configured with sfnSchemePdsch and sfnSchemePdcch is not configured, when scheduled by DCI format 1_1/1_2, if the time offset between the reception of the DL DCI and the corresponding PDSCH of a serving cell is equal to or greater than a threshold timeDurationForQCL if applicable, the UE shall expect TCI field present.. |
| LGE | Alt2 or Alt3 is preferred. |
| Moderator | Based on feedback, the current situation is as follows: Alt-1 (6): ZTE, Nokia/NSB, DOCOMO, Huawei/HiSi Alt-2 (6): vivo, OPPO, Lenovo, Xiaomi, CATT, LGE Alt-3 (4): Apple, Samsung, QC, LGE Alt-1: since dynamic switching between sTRP and SFN is agreed as UE optional and support of SFN PDSCH + single TRP PDDCH is also agreed, Alt-1 implies that UEs not capable of supporting dynamic switching would not be able to support SFN PDSCH + single TRP PDCCH. The comment from CATT is valid and proponents of Alt-1 are encouraged to clarify if this understanding is correct i.e., Alt-1 places an additional restriction for UEs not capable of dynamic switching. Alt-2: Comment from ZTE is valid and TCI field in DCI 1_1/1_2 should be present for the case when more than one TCI codepoint is activated with two TCI states. Alt-3: This was meant to capture the comment from Samsung and Qualcomm in the first round. However, comment from DOCOMO is valid i.e., for UEs not capable of dynamic switching, it is up to gNB to ensure all TCI codepoints have two TCI states. Since dynamic switching has already been agreed, Alt-3 in current form does not need to be agreed. Considering the inputs, it seems better to revert the formulation of Proposal#2 to the original one from preparation phase i.e., the case under discussion should be for UEs not capable of dynamic switching between sTRP and SFN and the following modified proposal is provided for further comments Proposal#2-1 For scheduling offset equal to or larger than threshold timeDurationForQCL, if applicable, for a UE that is not capable of dynamic switching between single TRP and SFN - Alt-1: SFN PDSCH + single TRP PDCCH is not supported - Alt-2: For SFN PDSCH + single TRP PDCCH, the TCI field should be present in DCI formats 1_1/1_2 when more than one TCI codepoint is activated with two TCI states - Note: for UE not capable of dynamic switching between sTRP and SFN, it does not expect to be activated by MAC-CE with single TCI state for any TCI codepoint |

Issue 8

Issue 8 is for TCI state assumption for CORESETs associates with CSS Type 0/0A/1/2. The summary from preparation phase is provided in Table 3.

Table 3 Summary of Issue 8

| # | Issue (summary of CR proposal) | Company inputs (if any) |
| --- | --- | --- |
| 8. | SFN for CORESETS associated with CSS Type 0/0A/1/2 - Alt-1: UE doesn’t expect PDCCH candidates in CSS type 0/0A/1/2 to be associated with CORESET activated with two TCI states. - Alt-2: Apply only the first TCI state for PDCCH reception - Alt-3: Apply both TCI states for PDCCH reception FL Note: This issue was extensively discussed in previous meetings with no consensus. Companies can provide further inputs. Please also provide inputs on alternatives listed | Alt-1: - Support: Qualcomm, OPPO - Not Support: Alt-2: - Support: vivo, Nokia/NSB, Lenovo, OPPO, Huawei/HiSi, LGE, Samsung, DOCOMO - Not Support: Alt-3: - Support: ZTE - Not Support: |

### Round-1

Based on the discussion in the preparation phase, the majority view is to support Alt-2, i.e., apply only the first TCI state for PDCCH reception and the first round FL proposal is as follows:

Proposal#3

A CORESET activated with two TCI states can be associated with CSS Type 0/0A/1/2 and the first TCI state is applied for PDCCH reception.

| Company | Comment |
| --- | --- |
| Moderator | This issue was extensively discussed in the last meeting without consensus so the FL proposal for the first round is to adopt the majority view. Companies are encouraged to provide their views and a TP can be discussed in later rounds |
| OPPO | We prefer Alt-2 which is majority. |
| Samsung | Support Alt-2. |
| Apple | Support Alt-3, this is assuming UE already supports SFN PDCCH But we do not think we need to discuss this issue at all. TCI is configured per CORESET, there is no need to discuss TCI configuration based on Search Space. If we would like to introduce Search Space based TCI configuration, we need to consider a clean/explicit design instead of the proposed default/implicit design. |
| Lenovo | Support Proposal#3 |
| Xiaomi | Fine with Proposal#3 |
| ZTE | Support Alt-3. According to proposal#3, it restricts only one beam can be received for CSS if the CCE of USS and CSS type 0/0A/2 are overlapped, and then the robustness of CSS will be lower than USS, which is not in line with Rel-16 related design and also deviates from Rel-17 FeMIMO WID. Basically, if CORESET#0 is configured with two TCI states, it is natural that the UE can receive the two beams for both USS and CSS. |
| Nokia/NSB | Support Alt 2. |
| DOCOMO | Support Alt.2. |
| CATT | Support Alt-3. |
| vivo | Support proposal #3. |
| Huawei, HiSilicon | Support the proposal. |
| QC | Although Alt 3 makes more sense when SFN PDCCH is configured, however, there is no guarantee that gNB will activate CORESET#0 with two TCI states. This means that UE needs to support two different behaviors for common PDCCH based on the corresponding CORESET state(s). Then, to keep a consistent UE behavior, Alt 1 or Alt 2 makes sense. |
| LGE | Support the proposal. |
| Mod | Based on current inputs, the current proposal still seems to be majority opinion. Additionally, one reason for treating CORESET#0 separately is that RAN2 LS reply in the last meeting stated that CORESET#0 can be configured with two TCI states, but this has not been agreed/clarified for CORESETs other than CORESET#0 which are associated with CSS Type 0/0A/1/2. To make the proposal clearer, the following modification is proposed Modified Proposal#3 For a CORESET, other than CORESET#0, activated with two TCI states and associated with CSS Type 0/0A/1/2, the first TCI state is applied for PDCCH reception. |

Issue 9

This issue is for TCI state assumption of CORESET#0 when configured with two TCI states. The summary of the preparation phase discussion is provided in Table 4.

Table 4 Summary of Issue 9

| # | Issue (summary of CR proposal) | Company inputs (if any) |
| --- | --- | --- |
| 9. | CORESET#0 configured with two TCI states - Alt-1: Only one TCI state should be used for CSS reception - Alt-2: Both TCI states should be used for CSS reception FL Note: This issue is based on reply-LS to RAN2 in the last meeting. Further inputs from companies are needed. | Alt-1: - Support: OPPO, QC, ZTE, DOCOMO, vivo, Lenovo - Not Support: Alt-2: - Support:, LGE - Not Support: |

### Round-1

Based on the discussion in the preparation phase, it seems that Alt-1 has majority view and is therefore proposed in the first round of discussions.

Proposal#4

If CORESET#0 is configured with two TCI states, only one TCI state should be used for CSS reception.

| Company | Comment |
| --- | --- |
| Moderator | Companies can provide further views on the initial FL proposal and an appropriate TP can be discussed in later rounds. |
| OPPO | Support Alt-1. |
| Samsung | This issue can be discussed with Issue 8. |
| Apple | Support Alt-3, this is assuming UE already supports SFN PDCCH But we do not think we need to discuss this issue at all. TCI is configured per CORESET, there is no need to discuss TCI configuration based on Search Space. If we would like to introduce Search Space based TCI configuration, we need to consider a clean/explicit design instead of the proposed default/implicit design. |
| Lenovo | Support Proposal#4 |
| Xiaomi | Fine with Proposal#4 |
| ZTE | Support Alt-2. Basically, it shall be common understanding that the CORESET#0 acts more important role in communication because the UE gets common PDCCH in CORESET#0. In current specification, only one monitoring occasion associated with one SSB is used for CSS, the reliability of CSS cannot be guaranteed at same level as USS which can be transmitted by multiple CORESETs. If two monitoring occasions of CSS determined by two SSB of the two TCI states, the reliability of CSS can be enhanced. |
| Nokia/NSB | Support Alt-1. |
| DOCOMO | Support Alt-1. |
| CATT | Support Alt-2. |
| vivo | Support Proposal#4. When CORESET#0 is configured with two TCI states, - if both TCI states of the CORESET#0 are applied for PDCCH reception when Type0/0A/2-PDCCH CSS is configured as search space#0, UE would monitor PDCCH candidates at monitoring occasions associated with two SS/PBCH blocks QCL-ed with CSI-RS resources in two TCI states, leading to a TDM-based PDCCH monitoring rather than SFN-based. - if both TCI states of the CORESET#0 are applied for PDCCH reception in Type1-PDCCH CSS, in the case that the PDCCH with RA-RNTI is transmitted in a CFRA procedure triggered by a PDCCH order, both the PDCCH and PDCCH order would be transmitted in SFN manner. - if the network transmits the broadcast information in SFN manner, the performance of reception of the broadcast information would be degraded for legacy UE, which has been shown in our tdoc R1-2203506. |
| Huawei, HiSilicon | It seems this issue is also related to issue#8. In issue#8, we are discussing the use of two TCI states depending on CSS type, and in this issue on CORESET ID, then what if CORESET#0 is associated with different CSS types? [Mod: Yes this is valid, Issue#8 has been modified to exclude CORESET#0. This is because while RAN1 has confirmed that CORESET#0 can be configured with two TCI states, such agreement has not been made for other CORESETs associated with CSS. Hence the two proposals. From FL perspective, it would be good to have aligned solutions for both Issue#8 and Issue#9] |
| QC | Similar views as issue 8. |
| LGE | Support the proposal. |
| Mod | Based on current inputs, Proposal#4 still seems to be majority opinion. To align with Issue#8, the following modification is proposed: Modified Proposal#4 If CORESET#0 is configured with two TCI states, the first TCI state is applied for PDCCH reception |

# Editorial Issue

Issue 11 (Closed)

This issue was brought up in [20] and all companies agreed in the preparation phase that this is an editorial issue.

Table 5 Summary of Issue 11

| # | Issue (summary of CR proposal) | Company inputs (if any) |
| --- | --- | --- |
| 9. | In clause 5.2.1.5.1, the text description for specifying the TCI state for the reception of the aperiodic CSI-RS when the CORESET with lowest ID is indicated with two TCSI states has extra “if” wording that should be deleted. FL Note: This seems to be an editorial update. | Agree with E: Apple, Oppo, Huawei/HiSilicon, Ericsson, Qualcomm, ZTE, LGE, CATT, vivo, Spreadtrum, Samsung, DOCOMO, Lenovo |

Based on the input from preparation phase, the following TP from [20] is proposed

TP#5:

TP for 38.214 Section 5.2.1.5.1 ----------------Unchanged part omitted------------------------ - else if a UE is configured with sfnSchemePdcch set to 'sfnSchemeA', it is not configured with enableTwoDefaultTCI-States, and the two TCI states are activated for the CORESET by the activation command as described in clause 6.1.3.x of [10, TS 38.321] - if there is any other DL signal with an indicated TCI state in the same symbols as the CSI-RS, the UE applies the QCL assumption of the other DL signal also when receiving the aperiodic CSI-RS. The other DL signal refers to PDSCH scheduled with an offset larger than or equal to the threshold timeDurationForQCL, as defined in [13, TS 38.306], periodic CSI-RS, semi-persistent CSI-RS, aperiodic CSI-RS in a NZP-CSI-RS-ResourceSet scheduled with offset larger than or equal to the UE reported threshold beamSwitchTiming when the reported value is one of the values {14,28,48} and when enableBeamSwitchTiming is not provided or the NZP-CSI-RS-ResourceSet is configured with the higher layer parameter trs-Info , aperiodic CSI-RS in a NZP-CSI-RS-ResourceSet configured with the higher layer parameter repetition set to 'off' or configured without the higher layer parameters repetition and trs-Info scheduled with offset larger than or equal to 48 when the UE provides beamSwitchTiming-r16 and enableBeamSwitchTiming is provided, aperiodic CSI-RS in a NZP-CSI-RS-ResourceSet configured with the higher layer parameter repetition set to 'on' scheduled with offset larger than or equal to the UE reported threshold beamSwitchTiming-r16 and enableBeamSwitchTiming is provided; - else if, the UE applies the first one of two TCI states indicated for the CORESET with the lowest CORESET ID in the latest slot within the active BWP of the cell in which the CSI-RS is to be received when receiving the aperiodic CSI-RS, ----------------Unchanged part omitted------------------------

| Company | Comment |
| --- | --- |
| Moderator | Companies are encouraged to indicate if there is any issue with TP#5. |
| OPPO | Support. |
| Samsung | Support the TP. |
| Apple | Fine with TP |
| Lenovo | Support |
| Xiaomi | Support |
| ZTE | Support. |
| Nokia/NSB | Support. |
| DOCOMO | Support |
| CATT | Support |
| vivo | Support |
| Huawei, HiSilicon | Support |
| QC | Support |
| LGE | Support |
| Mod | This proposal was endorsed via email. |

# References

|  | R1-2203106 | Remaining issues on enhancements for Multi-TRP deployment | Huawei, HiSilicon |
| --- | --- | --- | --- |
|  | R1-2203259 | Remaining issues on multi-TRP deployment | ZTE |
|  | R1-2203302 | Remaining issues on multi-TRP deployment | Spreadtrum Communications |
|  | R1-2203422 | Maintenance issues on Rel-17 M-TRP enhancements | CATT |
|  | R1-2203506 | Maintenance on enhancements for multi-TRP Deployment | vivo |
|  | R1-2203644 | Remaining issues for Multi-TRP Deployment | Ericsson |
|  | R1-2203674 | Discussion on remaining issues for multi-TRP deployment | NEC |
|  | R1-2203765 | Maintenance of Enhancements for Multi-TRP Deployment | Langbo |
|  | R1-2203772 | Remaining issues on HST-SFN deployment enhancement | xiaomi |
|  | R1-2203856 | Maintenance on Rel-17 multi-TRP and HST-SFN | Samsung |
|  | R1-2203949 | Maintenance on enhancements for mTRP deployment | OPPO |
|  | R1-2204138 | Text proposal on multi-TRP deployment | LG Electronics |
|  | R1-2204170 | Remaining issues on multi-TRP deployment | Lenovo |
|  | R1-2204193 | Remaining issues for mTRP PUSCH | ASUSTeK |
|  | R1-2204200 | Remaining Issues on Multi-TRP Enhancement | Apple |
|  | R1-2204336 | Remaining issues on enhancements for Multi-TRP Deployment | NTT DOCOMO, INC. |
|  | R1-2204536 | Maintenance of enhancements for Multi-TRP Deployment | Nokia, Nokia Shanghai Bell |
|  | R1-2204683 | Maintenance of Rel-17 beam management for multi-TRP | MediaTek Inc. |
|  | R1-2204764 | MIMO Enhancements for Multi-TRP Deployment | Intel Corporation |
|  | R1-2204977 | Remaining Details for Multi-TRP Operation | Qualcomm Incorporated |