3GPP TSG RAN WG1 #109-e			R1-2205289

e-Meeting, May 9th – 20th, 2022

Agenda item:	9.1.2

Source: 	Moderator (Samsung)

Title: 	EVM for Rel-18 MIMO CSI enhancements

Document for:	Discussion and Decision

## Introduction

The scope given in the Rel-19 NR Evolved MIMO WID pertaining to CSI enhancement is as follows:

- Study, and if justified, specify CSI reporting enhancement for high/medium UE velocities by exploiting time-domain correlation/Doppler-domain information to assist DL precoding, targeting FR1, as follows: - Rel-16/17 Type-II codebook refinement, without modification to the spatial and frequency domain basis - UE reporting of time-domain channel properties measured via CSI-RS for tracking - Study, and if justified, specify enhancements of CSI acquisition for Coherent-JT targeting FR1 and up to 4 TRPs, assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs, as follows: - Rel-16/17 Type-II codebook refinement for CJT mTRP targeting FDD and its associated CSI reporting, taking into account throughput-overhead trade-off

## EVM: SLS

On Rel-18 CSI enhancement EVM for SLS, use the attached excel spreadsheet “EVM CSI V03” (in /tsg_ran/WG1_RL1/TSGR1_109-e/Inbox/drafts/9.1.2/ROUND 1).

The excel spreadsheet is included as an attachment with this document.

## EVM: LLS

On Rel-18 CSI enhancement EVM for LLS (only for TRS-based TDCP), companies can use the following simulation assumptions:

- For mTRP 120kmph and over, use Rel-17 HST assumptions (cf. section 2.1 in R1-2007201)

- For sTRP up to 120km/h:

| Parameter | Value |
| --- | --- |
| Carrier frequency and subcarrier spacing | 3.5 GHz with 30 kHz SCS |
| System bandwidth | 20MHz, 100MHz |
| TRS bandwidth | 20MHz, 100MHz |
| Channel model | Alt. 1: TDL channels with uncorrelated antenna elements with first priority on TDL-A while the use of other TDL channels isn’t precluded Alt. 2: CDL channels with first priority on CDL-A while the use of other CDL channels isn’t precluded |
| Delay spread | 10ns, 30ns, 100ns, 300ns, and 1000ns |
| UE velocity | 3km/h, 10km/h, 20km/h, 30km/h, 60km/h, 120km/h |
| Antennas at UE | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for rank > 2 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) For TRS based Doppler accuracy evaluations a single UE antenna may also be used Other configurations are not precluded. |
| Antennas at gNB | 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ For TRS based Doppler accuracy evaluations a single gNB port may also be used. Other configurations are not precluded. |
| Link adaptation | For TRS based Doppler accuracy: Not applicable For mode selection performance: Adaptation of both MCS and rank. |
| Evaluation metrics for measurement accuracies | RMS error, Standard deviation, Bias |
| Evaluation metric for Doppler based mode selection | User throughput |