3GPP TSG RAN WG1 #109-e		           R1-2205296

e-Meeting, May 9st –20th , 2022

Source:	Moderator (CATT)

Title:	Moderator summary #1 on enhancements of beam management for multi-TRP

Agenda Item:	8.1.2

Document for:	Discussion and Decision

Background

Based on the following email thread assignment, this document summarizes the remaining issues with category H on enhancements of beam management for multi-TRP.

[109-e-R17-MIMO-05] Maintenance on beam management for mTRP (description of issues in R1-220XXXX) by May 18 – Xin (CATT)

- Issues #1, #2, #3, #4&5 (4&5 can be discussed together), #7, #15

High priotiy issues

Issue 1: Whether CRI report is needed when the corresponding resource set is configured with repetition “on”

In RAN1#108-e, it was agreed that UE can be configured with the same or different value(s) of repetition in different CMR sets, and FFS UE reports CRI regardless of whether repetition is set to ‘on’ or not.

- R1-2203259 (ZTE) proposes that UE shall report CRI for each of corresponding CMR set regardless whether repetition in the set is set to 'on' or not.

- R1-2203506(vivo), R1-2204170(lennovo), R1-2204200 (Apple) and R1-2204336(DOCOMO) propose that CRI(s) are not needed when the corresponding resource set is configured with repetition “on”.

- R1-2203856(Samsung) proposes the higher layer parameter repetition is set to the same value for both CSI resource sets.

### Round 1

Based on views of companies showed in preparation phase, the following proposed for discussion.

FL Proposal 1: CRI(s) are not reported when the corresponding resource set is configured with repetition “on”.

Companies are invited to provide their preferences and comments in the table below.

| Company | Comments |
| --- | --- |
| Apple | Support |
| NTT DOCOMO | Support. And the complete proposal should be: - When groupBasedBeamReporting-r17 in CSI-ReportConfig is configured, CRI(s) are not reported when the corresponding resource set is configured with repetition “on”. |
| QC | Support |
| Lenovo | Support. |
| Xiaomi | Support |
| Samsung | Support – fine with DOCOMO’s version |
| Nokia/NSB | We don’t see strong need for this agreement. We prefer less specification impact for this limited case (e.g. 38.212 encoding) . UE can report any CRI of the largest RSRP according to its RX beam. |
| OPPO | Support |
| ZTE | Not support. We share the similar views with Nokia. It is because, if the parameter repetition in one resource set is set to ‘on’, and that in the other resource set is set to ‘off’ , whether to refine its Rx beam or scan the Tx beam is unclear from the perspective of gNB side. So we support report CRI regardless of whether repetition is set to ‘on’. |
| vivo | Fine with DOCOMO’s version. |
| CATT | Support |
| Mod | Based on suggestion from DOCOMO and views from majority, the following revision is proposed. FL Proposal 1: When groupBasedBeamReporting-r17 in CSI-ReportConfig is configured, CRI(s) are not reported when the corresponding resource set is configured with repetition “on”. |
| Huawei, HiSilicon | We can support FL Proposal 1. |
| QC | Support latest FL proposal 1 |
| Apple | Support latest FL proposal 1 |
| Lenovo | Support the latest FL proposal 1. |
| Mod | Views on FL Proposal 1 are summarized as follows: Support by: Apple, DOCOMO, QC, Lenovo, Xiaomi, Samsunng, OPPO, vivo, CATT Concerned by: ZTE, [Nokia/NSB] |

Issue 2: Update of beam/power control parameters for PUCCH after receiving gNB response

In this meeting, R1-2203259(ZTE), R1-2204336(DOCOMO), R1-2204536(Nokia), R1-2204764(Intel) and R1-2203773(Xiaomi) propose to introduce an association between PUCCH and BFD-RS set (TRP). It’s noted that this issue has been discussed extensively in previous meetings. In the last meeting, the following has been proposed and discussed without any agreement/conclusion.

FL proposal 2 (FL proposal 2.3 of #107e): After 28 symbols from receiving the BFR response, if a BFD –RS set on a spCell/PUCCH –Scell fails, the UL spatial filter for PUCCH resource(s) associated with the failed BFD –RS set on the spCell/PUCCH –Scell  is updated by the RS resource associated with the latest reported new candidate beam for the failed BFD –RS set (if found) , and the power control parameter for the corresponding PUCCH transmission is set to  [qu=0], qd= qnew , and  [l=0], where qnew  is the RS resource associated with the latest reported new candidate beam for the failed BFD –RS set, if

1. the PUCCH resource is provided with spatial relation info,

[2. SR is not transmitted or is transmitted on PUCCH not associated with this failed BFR –RS set, and]

3. the failed BFD –RS set on the spCell/PUCCH –Scell is indicated in the TRP –specific BFR MAC-CE

The association between the PUCCH resource(s) and the failed BFD –RS set can be done by the PUCCH group for group-based spatial relation info update associated with each BFD –RS set.

### Round 1

Regarding this issue, companies are invited to provide their answer to the following questions.

Q: For mTRP MB, do you think it’s necessary to introduce the mechanism of beam/power update for PUCCH after receiving gNB response?

If the answer is yes, please provide your comments to FL proposal 2.

| Company | Comments |
| --- | --- |
| Apple | We are open to introduce this mechanism. |
| NTT DOCOMO | We think beam/power update for PUCCH after receiving gNB response should be supported. |
| QC | Support beam/power update for PUCCH |
| Lenovo | We support beam/power update for PUCCH in principle. However, for the association between PUCCH resource and BFD-RS set, we think MAC CE based indication is a good alternative since the beam of a PUCCH resource is activated by a MAC CE which can be reuse to indicate the association. |
| Xiaomi | Prefer to introduce this mechanism. For FL proposal 2, if the PUCCH is configured by schedulingRequestIDForMTRPBFR, the beam update will be wrong. Thus it is better to introduce association between PUCCH and CORESETPoolIndex. And the FL proposal can be revised as below. With the revised proposal, it is possible that PUCCH-SR#0 with beam#0 is configured with CORESETPoolIndex#0 and associated with BFD-RS set#1 (beam#1), and PUCCH-SR#1 with beam#1 is configured with CORESETPoolIndex#1 and associated with BFD-RS set#0 (beam#0).When BFD-RS set#1 fails, i.e., beam#1 fails, UE will transmit SR on PUCCH-SR#0 with beam#0 and update the beam of PUCCH-SR#1. While for PUCCH not for SR, PUCCH#1 will be configured with CORESETPoolIndex#1. When BFD-RS set#1 fails, i.e., beam#1 fails, UE will update the beam of PUCCH-SR#1. FL proposal 2 (FL proposal 2.3 of #107e): After 28 symbols from receiving the BFR response, if a BFD –RS set on a spCell/PUCCH –Scell fails, the UL spatial filter for PUCCH resource(s)/ PUCCH-SR resource configured associated with the failed BFD –RS set on the spCell/PUCCH –Scell CORESETPoolIndex k (k=0,1) is updated by the RS resource associated with the latest reported new candidate beam (if found) associated with the failed BFD -RS set k (k=0,1) is updated by the RS resource associated with the latest reported new candidate beam for the failed BFD –RS set (if found) , and the power control parameter for the corresponding PUCCH transmission is set to [qu=0], qd= qnew , and [l=0], where qnew is the RS resource associated with the latest reported new candidate beam associated with the failed BFD -RS set k (k=0,1)for the failed BFD –RS set, if 1. the PUCCH resource/ PUCCH-SR resource is provided with spatial relation info, [2. SR is not transmitted or is transmitted on PUCCH not associated with this failed BFR –RS set, and] 3. the failed BFD –RS set on the spCell/PUCCH –Scell is indicated in the TRP –specific BFR MAC-CE The association between the PUCCH resource(s) and the failed BFD –RS set can be done by the PUCCH group for group-based spatial relation info update associated with each BFD –RS set. Introduce CORESETPoolIndex configured for each PUCCH resource/ PUCCH-SR resource/SR configurations (Maybe not needed since there was an agreement in RAN1-98b meeting). Above is applicable for mDCI based mTRP operation. For reference RAN1-98b meeting Agreement For M-DCI NCJT transmission, each PUCCH resource may be associated with a value of higher layer index per CORESET - FFS: Additional restriction such as TDM PUCCH transmission across different higher layer index per CORESET - FFS: Details on association |
| LGE | Support. Regarding Xiaomi’s suggestion, in our view, the association between PUCCH-SR resource and BFD-RS set for BFRQ PUCCH selection is different/independent from the association between PUCCH resource group and BFD-RS set for PUCCH beam update. In your example, PUCCH-SR#0 can be configured with PUCCH resource group#0 associated with BFD-RS set#0 for PUCCH beam update in the original proposal, then if BFD-RS set#0 fails, i.e., beam#0 fails, UE will transmit SR on PUCCH-SR#1 with beam#1 and update the beam of PUCCH-SR#0 (if new beam is found) which is configured with PUCCH resource group#0 for beam update. So, we don’t think Xiaomi’s revision is needed. |
| Samsung | Fine with PUCCH beam update at least. |
| Nokia/NSB | Support beam/power update for PUCCH after receiving gNB response. |
| OPPO | Yes, in our view, it’s necessary to reset the spatial relation for PUCCH per failed TRP link after BFRR. We think the current version of FL proposal seems fine to us. To avoid triggering lengthy discussion in maintainence phase, the association between PUCCH resource(s) and BFD RS set should reuse what has been specified as much as possible. In this case, we could reuse the Rel.16 PUCCH resource group for group-based spatial relation info update. Via NW’s proper configuration, these PUCCH group can be associated with BFD-RS set and can be recovered after BFRR. |
| ZTE | Yes. To facilitate TRP-specific uplink recovery, PUCCH-related beam and power control update according to the association between PUCCH and BFD-RS set should be supported. |
| vivo | Seems too late to introduce new features for PUCCH grouping. |
| CATT | It would be better if we introduce the mechanism of beam/power update for PUCCH. The association between PUCCH and TRP can be established. FL proposal 2 is acceptable to us. |
| Mod | @Xiaomi: Thanks for your revision. However, I have similar understanding as LGE that the association between PUCCH-SR resource and BFD-RS set for BFRQ PUCCH selection is different/independent from the association between PUCCH resource group and BFD-RS set for PUCCH beam update. According to the feedback above, FL Proposal 2 is updated as follows. Please further show your opinion on updated FL Proposal 2 and especially the parts marked in red. FL proposal 2: After 28 symbols from receiving the BFR response, if a BFD –RS set on a spCell/PUCCH –Scell fails, the UL spatial filter for PUCCH resource(s) associated with the failed BFD –RS set on the spCell/PUCCH –Scell is updated by the RS resource associated with the latest reported new candidate beam for the failed BFD –RS set (if found) , and the power control parameter for the corresponding PUCCH transmission is set to [qu=0], qd= qnew , and [l=0], where qnew is the RS resource associated with the latest reported new candidate beam for the failed BFD –RS set, if 1. the PUCCH resource is provided with spatial relation info, [2. SR is not transmitted or is transmitted on PUCCH not associated with this failed BFR –RS set, and] 3. the failed BFD –RS set on the spCell/PUCCH –Scell is indicated in the TRP –specific BFR MAC-CE The association between the PUCCH resource(s) and the failed BFD –RS set can be done by - Alt-1: PUCCH group for group-based spatial relation info update associated with each BFD –RS set. - Alt-2: MAC CE based indication. |
| Huawei, HiSilicon | We think FL proposal 2 is an enhancement which seems unnecessary at this point. We think it is a bit late to opening a discussion regarding PUCCH association mechanism at this stage of maintenance phase. |
| QC | Support latest FL proposal 2 with Alt1, which reuses existing signaling |
| Apple | In our view, the proposal should be completed since it is maintenance phase. Since both Alt1 and Alt2 could have RAN2 spec impact, we suggest not to take this proposal. |
| Lenovo | Support latest FL proposal 2 with Alt2. |
| CATT | Views of companies on FL Proposal 2 are summarized as follows: Supported by: DOCOMO, QC, Lenovo, [Xiaomi], LGE, Samsung, Nokia/NSB, OPPO, ZTE, CATT Concerned by: vivo, HW, Apple |

Issue 3: Beam update of CORESETs for S-DCI

For S-DCI, to enable beam update of CORESETs after UE receives gNB’s response, R1-2204336(DOCOMO), R1-2204536(Nokia)， R1-2204764(Intel) and R1-2203773(Xiaomi) propose to establish association between CORESETs and BFD-RS sets.

It’s noted that this issue has been discussed extensively in previous meetings. Based on views of proponent companies, the following proposal is listed for discussion.

FL Proposal 3: For S-DCI, after receiving gNB response, the UE assumes that

- The QCL assumption of CORESETs associated with the failed BFD-RS sets is updated to the latest reported qnew.

- To associate BFD-RS set with CORSET

  - Alt-1: UE can determine the association between CORESET and BFD-RS set based on the indicated TCI state for each CORESET

  - Alt-2: explicitly associate CORESET(s) and explicitly configured BFD-RS set index by RRC

### Round 1

Regarding this issue, companies are invited to provide their answer to the following questions.

Q: For S-DCI, do you think it’s necessary to introduce the mechanism of beam update for CORESETs after receiving gNB response?

If the answer is yes, please provide your comments to FL proposal 3.

| Company | Comments |
| --- | --- |
| Apple | It seems unnecessary, since usually sDCI works in ideal-backhaul and normal beam indication can be used |
| NTT DOCOMO | We think the beam update for CORESETs after receiving gNB response for S-DCI should be supported. Otherwise, the benefit of configuring two BFD-RS sets for S-DCI is small. And we prefer Alt-1. |
| QC | Support. The reported new beam may not be in any configured TCI, so gNB may not always be able to indicate TCI for the new beam without reconfig for the TCI |
| Lenovo | Support the beam update for CORESTETs after receiving gNB’s confirm of TRP-specific BFR which can increase the link robustness. And we support Alt 2 for the two alternatives which is more simple. |
| Xiaomi | Support and prefer Alt-2. |
| LGE | Support. And prefer Alt-2 for the clarity of configuration. |
| Samsung | Seems a bit late to introduce RRC linkage for Alt-2 – impact on RAN2 |
| Nokia/NSB | Support. We support Alt 1. |
| OPPO | It doesn’t seem necessary to associate CORESET(s) and BFD RS set for sDCI. The reason lies in the fact that for sDCI, one CORESET may be not always used by one particular TRP. It’s up to NW to switch CORESET(s) from one TRP to another TRP, by using MAC CE activating TCI state for CORESET. So with TRP-specific BFRQ, NW is able to reset the beam of CORESET (transmitted by failed TRP link) via the same MAC CE. |
| ZTE | Yes, and we prefer Alt-2. In our views, Alt-1 is incomplete solution: as in a typical case, if BFD-RS corresponds to CSI-RS for BM, but the reference RS in TCI state applied to CORESET refers to TRS, what we shall do? But we think the association between PUCCH and BFD-RS sets in Issue 2 should be considered together. |
| vivo | It seems unnecessary. Introducing association between CORESET and BFD-RS set would complicate the signalling design and decrease the scheduling flexibility. Contrasted with the affects mentioned above, the benefits of the association is insignificant, since gNB can always update the beam of CORESETs by legacy methods (e.g., MAC CE). |
| CATT | It would be better if we introduce beam update mechanism for S-DCI. Alt-2 is preferred. |
| Mod | Regarding Issue 3, views from companies are summarized as follows. - Support beam update of CORESETs for S-DCI: DCM(Alt-1), QC, Lenovo(Alt-2), Xiaomi(Alt-2), LGE(Alt-2), Nokia(Alt-1), ZTE(Alt-2), CATT(Alt-2) - Not support beam update of CORESETs for S-DCI: Apple, OPPO, vivo Based on the summary of views above, the following revision is listed for further discussion. FL Proposal 3: For S-DCI, after receiving gNB response, the UE assumes that - The QCL assumption of CORESETs associated with the failed BFD-RS sets is updated to the latest reported qnew. - To associate BFD-RS set with CORSET, support explicitly associate CORESET(s) and explicitly configured BFD-RS set index by RRC. |
| Huawei, HiSilicon | It is an enhancement which does not seem necessary but if it has a strong support we are open to discuss it. |
| QC | Support latest FL proposal 3. The sDCI should have same performance as mDCI |
| Apple | We should avoid adding new RRC parameter for optimization. We do not support this proposal. |
| Lenovo | Support the latest FL proposal 3. |
| Mod | @HW: thanks to be flexible. Views of companies on FL Proposal 3 are summarized as follows: Supported by: DOCOMO, QC, Lenovo, Xiaomi, LGE, [Samsung], [Nokia/NSB], ZTE, CATT Concerned by: Apple, OPPO, vivo Let’s continue this discussion over email. |

Issue 4&5: Configuration/update of explicit BFD-RS set

It was agreed to support to configure/update explicit BFD-RS set by RRC signaling and MAC CE signaling. However, as raised by R1-2203422(CATT), R1-2204170(Lenovo) and R1-2204536(Nokia), RRC parameters related to BFD-RS sets have not been captured in spec yet.

With regard to MAC-CE based BFD-RS set update,

- R1-2203259(ZTE) proposes to activate up to 2 BFD-RS(s) per set and send LS to RAN2.

- R1-2204138(LGE) proposes that no MAC-CE activation is required when the number of RRC configured BFD-RS(s) in a set is equal to or less than NBFD.

- R1-2204536(Nokia) proposes that MAC-CE operation is also applied if only one BFD-RS set is configured.

Based on the proposals from those companies, we have the following proposals for further discussion.

FL Proposal 4-1: TP to section 6 of 213

A UE can be provided, for each BWP of a serving cell, a set q0 of periodic CSI-RS resource configuration indexes by failureDetectionResourcesToAddModList and a set q1 of periodic CSI-RS resource configuration indexes and/or SS/PBCH block indexes by candidateBeamRSList or candidateBeamRSListExt or candidateBeamRSSCellList for radio link quality measurements on the BWP of the serving cell. Instead of the sets q0 and q1, for each BWP of a serving cell, the UE can be provided respective two sets q0,0 and q0,1 of periodic CSI-RS resource configuration indexes by failureDetectionResourcesToAddModList1 and failureDetectionResourcesToAddModList2 that can be activated by a MAC CE [11 TS 38.321] and corresponding two sets q1,0 and q1,1 of periodic CSI-RS resource configuration indexes and/or SS/PBCH block indexes by candidateBeamRSList1 and candidateBeamRSList2, respectively, for radio link quality measurements on the BWP of the serving cell. If the number of resources provided in each set q0,0 and q0,1 is equal to or less than NBFD, UE considers that the resources in each set q0,0 and q0,1 are activated. The set q0,0 is associated with the set q1,0 and the set q0,1 is associated with the set q1,1. The [set q0 or] two sets q0,0 and q0,1 can be updated by a MAC-CE.

FL Proposal 4-2: Send a LS to RAN2 with the following:

- For a given BWP in a CC, TRP-specific MAC-CE for BFD-RS activation can activate up to 2 BFD-RS(s) from respective RRC BFD-RS pools (including up to 64 BFD-RSs) for a TRP-ID (e.g. CORESETPoolIndex, BFD-RS set index).

### Round 1

Companies are invited to provide their preferences and comments in the table below.

| Company | Comments |
| --- | --- |
| Apple | Support |
| NTT DOCOMO | Support |
| QC | Support |
| Xiaomi | Support in prcinple. It will be much better to add “based on UE capability” FL Proposal 4-2: Send a LS to RAN2 with the following: - For a given BWP in a CC, TRP-specific MAC-CE for BFD-RS activation can activate up to 2 BFD-RS(s) from respective RRC BFD-RS pools (including up to 64 BFD-RSs) for a TRP-ID (e.g. CORESETPoolIndex, BFD-RS set index) based on UE capability. |
| LGE | Support the TP and FL proposal 4-2. |
| Samsung | OK |
| Nokia/NSB | We need further clarification of “activation”. We have agreed update of BFD-RS, but activation is somewhat different. We can consider simple update rather than activation. Because RRC configuration is only including RS-ID, so just replacing this RS-ID in MAC-CE should be easier. With FL proposal, some issues should be clarified. First, whether the activation is the mandatory feature and what is the UE operation before activation. Second, whether to support explicit activation and deactivation or just only newly activated BFD-RSs are valid. Third, whether only one BFD-RS set can be activated. |
| OPPO | Fine with FL proposal |
| ZTE | Support. |
| vivo | The details for mentioned by Nokia would be resolved by RAN2 signaling. We can wait for RAN2 signalling design. And corresponding RAN1 specification would potentially need further update with the new signaling. |
| CATT | Support |
| Mod | @Nokia: hopefully vivo’s response can address your concern. Based on discussion above, the following update is given for further discussion. Please note the highlighted parts. FL Proposal 4-1: TP to section 6 of 213 A UE can be provided, for each BWP of a serving cell, a set q0 of periodic CSI-RS resource configuration indexes by failureDetectionResourcesToAddModList and a set q1 of periodic CSI-RS resource configuration indexes and/or SS/PBCH block indexes by candidateBeamRSList or candidateBeamRSListExt or candidateBeamRSSCellList for radio link quality measurements on the BWP of the serving cell. Instead of the sets q0 and q1, for each BWP of a serving cell, the UE can be provided respective two sets q0,0 and q0,1 of periodic CSI-RS resource configuration indexes by failureDetectionResourcesToAddModList1 and failureDetectionResourcesToAddModList2 that can be activated by a MAC CE [11 TS 38.321] and corresponding two sets q1,0 and q1,1 of periodic CSI-RS resource configuration indexes and/or SS/PBCH block indexes by candidateBeamRSList1 and candidateBeamRSList2, respectively, for radio link quality measurements on the BWP of the serving cell. If the number of resources provided in each set q0,0 and q0,1 is equal to or less than NBFD, UE considers that the resources in each set q0,0 and q0,1 are activated. The set q0,0 is associated with the set q1,0 and the set q0,1 is associated with the set q1,1. The set q0 or two sets q0,0 and q0,1 can be updated by a MAC-CE. FL Proposal 4-2: Send a LS to RAN2 with the following: - For a given BWP in a CC, TRP-specific MAC-CE for BFD-RS activation can activate up to 2 BFD-RS(s) from respective RRC BFD-RS pools (including up to 64 BFD-RSs) for a TRP-ID (e.g. CORESETPoolIndex, BFD-RS set index) based on UE capability. |
| Huawei, HiSilicon | We can support FL Proposal 4-2 |
| QC | Fine for latest FL proposal 4-2. For FL proposal 4-1, suppose N_BFD is 2, and gNB may still want to configure 2 candidate RS while only activating one of them, right? So FL proposal 4-1 may restrict such flexibility. But we are also open to other views. |
| Apple | We think q_0 should be removed in the TP, as it is related to R15/R16 BFR. We are not sure whether the LS is still needed, since RAN1 has already replied LS R1-2205168. |
| Lenovo | We share the same view that think q_0 should be removed in the TP since it is related to R15/16 BFR but R17 TRP-specific BFR. |
| Mod | @QC: regarding your question, to my understanding, the answer is yes. @Xiaomi: thanks for your suggestion on FL Proposal 4-2. @Apple and Lenovo: thanks for the suggestion, the following TP is revised accordingly. @All: we just noticed that the RRC parameter names captured in the latest version of 331 are failureDetectionSet1-r17 and failureDetectionSet2-r17 now. failureDetectionSet1-r17 BeamFailureDetectionSet-r17 OPTIONAL, -- Need R failureDetectionSet2-r17 BeamFailureDetectionSet-r17 OPTIONAL -- Need R So, the following TP in FL Proposal 4-1 is revised accordinally: FL Proposal 4-1: TP to section 6 of 213 A UE can be provided, for each BWP of a serving cell, a set q0 of periodic CSI-RS resource configuration indexes by failureDetectionResourcesToAddModList and a set q1 of periodic CSI-RS resource configuration indexes and/or SS/PBCH block indexes by candidateBeamRSList or candidateBeamRSListExt or candidateBeamRSSCellList for radio link quality measurements on the BWP of the serving cell. Instead of the sets q0 and q1, for each BWP of a serving cell, the UE can be provided respective two sets q0,0 and q0,1 of periodic CSI-RS resource configuration indexes by failureDetectionSet1-r17 and failureDetectionSet2-r17 that can be activated by a MAC CE [11 TS 38.321] and corresponding two sets q1,0 and q1,1 of periodic CSI-RS resource configuration indexes and/or SS/PBCH block indexes by candidateBeamRSList1 and candidateBeamRSList2, respectively, for radio link quality measurements on the BWP of the serving cell. If the number of resources provided in each set q0,0 and q0,1 is equal to or less than NBFD, UE considers that the resources in each set q0,0 and q0,1 are activated. The set q0,0 is associated with the set q1,0 and the set q0,1 is associated with the set q1,1. The two sets q0,0 and q0,1 can be updated by a MAC-CE. FL Proposal 4-2: Send a LS to RAN2 with the following: - For a given BWP in a CC, TRP-specific MAC-CE for BFD-RS activation can activate up to 2 BFD-RS(s) from respective RRC BFD-RS pools (including up to 64 BFD-RSs) for a TRP-ID (e.g. CORESETPoolIndex, BFD-RS set index) based on UE capability. Let’s continue our discussion over email. If we all think a LS to RAN2 is still needed, it should be done as soon as possible. |

Issue 7: Report of cell-specific BFR and TRP-specific BFR in enhanced BFR MAC-CE

R1-2203259(ZTE) points out that cell-specific BFR and TRP-specific BFR can be reported in a same enhanced BFR MAC-CE in 38.321. However, this behavior is not specified in 38.213. To keep consistency between 321 and 213, it can be clarified that the second PUSCH MAC CE can also include cell-specific BFR information.

FL Proposal 5: BFR information of a first serving cell with cell-specific BFR and a second serving cell with TRP-specific BFR can be reported in a same enhanced MAC-CE.

### Round 1

Companies are invited to provide their preferences and comments in the table below.

| Company | Comments |
| --- | --- |
| Apple | We think this is a RAN2 issue, as it is related to BFR MAC CE design. |
| NTT DOCOMO | Not needed. We support it in principle, but we think it is a RAN2 issue. And current BFR MAC CE design in RAN2 seems to be aligned with such principle already. |
| QC | Support. It seems beneficial/flexible for both cell-level and TRP-level BFR reported in a single MAC-CE |
| Lenovo | Same view with Apple which is not needed to be discussed here. |
| LGE | We are not sure that RAN1 spec should include all RAN2 related procedure. |
| Samsung | Same view. Can leave it to RAN2 |
| Nokia/NSB | Agree in principle. Also fine with left this up to RAN2. |
| OPPO | In our view, if RAN1’s specification doesn’t conflict with the BFR MAC CE enhancement made by RAN2, then it’s not necessary to agree more and adjust TS 38.213 accordingly. |
| ZTE | Support. We can NOT agree that it is just relevant to RAN2. If the above is our RAN1 concensus, the following TP should be endorsed. # 6. Link recovery procedures <Unchanged part omitted> If there is at least one serving cell associated with sets q0,0 and q1,0, and with sets q0,1 and q1,1, the UE can provide in a second PUSCH MAC CE index(es) for cell(s) with q0, or with at least one of q0,0 and q0,1 having radio link quality worse than Qout,LR, the index(es) of those q0,0 and/or q0,1, and indication(s) of presence of qnew and of index(es) qnew, if any, from corresponding sets q1,0 and/or q1,1 for the serving cells. <Unchanged part omitted> |
| vivo | Support. In current spec version of TS38.321, a MAC PDU shall contain at most one MAC CE for BFR, which means the BFR information of CC#1 and TRP#1 in CC#2 should be reported in the same BFR MAC CE when CC#1 and TRP#1 in CC#2 are detected with beam failure simultaneously. To maintain the specification consistency, the corresponding description in TS38.213 should be revised as following. # 6. Link recovery procedures <Unchanged part omitted> If there is at least one serving cell associated with sets q0,0 and q1,0, and with sets q0,1 and q1,1, the UE can provide in a second PUSCH MAC CE index(es) for cell(s) with q0, or with at least one of q0,0 and q0,1 having radio link quality worse than Qout,LR, and/or the index(es) of those q0,0 and/or q0,1, and indication(s) of presence of qnew and of index(es) qnew, if any, from q1, or corresponding sets q1,0 and/or q1,1 for the serving cells. <Unchanged part omitted> |
| CATT | Support. We think “and/or” added by Vivo is unnecessary. If at least one of q0,0 and q0,1 having radio link quality worse than Qout,LR, the index(es) of those q0,0 and/or q0,1 should be reported. The following modificaiton can be considered. # 6. Link recovery procedures <Unchanged part omitted> If there is at least one serving cell associated with sets q0,0 and q1,0, and with sets q0,1 and q1,1, the UE can provide in a second PUSCH MAC CE index(es) for cell(s) with q0, or with at least one of q0,0 and q0,1 having radio link quality worse than Qout,LR the index(es) of those q0,0 and/or q0,1, and indication(s) of presence of qnew and of index(es) qnew, if any, from q1, or corresponding sets q1,0 and/or q1,1 for the serving cells. <Unchanged part omitted> |
| Mod | Suggest to further discuss based on the following revision. FL Proposal 5: TP to section 6 of 213 # 6. Link recovery procedures <Unchanged part omitted> If there is at least one serving cell associated with sets q0,0 and q1,0, and with sets q0,1 and q1,1, the UE can provide in a second PUSCH MAC CE index(es) for cell(s) with q0, or with at least one of q0,0 and q0,1 having radio link quality worse than Qout,LR the index(es) of those q0,0 and/or q0,1, and indication(s) of presence of qnew and of index(es) qnew, if any, from q1, or corresponding sets q1,0 and/or q1,1 for the serving cells. <Unchanged part omitted> |
| Huawei, HiSilicon | We tend to agree with some other companies that it is a RAN2 issue and the TP is not required |
| QC | Support latest FL proposal 5. It is RAN1 issue to match spec with RAN2 design |
| Apple | We do not think the TP is needed. |
| Lenovo | We do not think the TP is needed since it is a RAN2 issue. |
| Mod | Tend to agree with QC and some other companies that align RAN 1 spec with RAN2 design is an issue of RAN1. |

Issue 15: Determination of per-cell BFR or per-TRP BFR

As pointed out by R1-2204336 (DOCOMO), for implicit configuration of BFD-RS sets, UE does not know whether it should determine one BFD-RS set for per-cell BFR or two BFD-RS sets for per-TRP BFR. Therefore the following TP is proposed:

TP for section 6 of 213:

If the UE is not provided q0 by failureDetectionResourcesToAddModList, but provided q1, for a BWP of the serving cell, the UE determines the set q0 to include periodic CSI-RS resource configuration indexes with same values as the RS indexes in the RS sets indicated by TCI-State for respective CORESETs that the UE uses for monitoring PDCCH. If the UE is not provided q0,0 or q0,1, but provided q1,0 and q1,1, for a BWP of the serving cell, the UE determines the set q0,0 or q0,1 to include periodic CSI-RS resource configuration indexes with same values as the RS indexes in the RS sets indicated by TCI-State for first and second CORESETs that the UE uses for monitoring PDCCH, where the UE is provided two coresetPoolIndex values 0 and 1 for the first and second CORESETs, or is not provided coresetPoolIndex value for the first CORESETs and is provided coresetPoolIndex value of 1 for the second CORESETs, respectively.

### Round 1

Regarding the TP from R1-2204336 (DOCOMO), companies are invited to provide their preferences and comments in the table below.

| Company | Comments |
| --- | --- |
| Apple | We failed to see the necessity for this change |
| NTT DOCOMO | We think it is no harm clarifying the spec. to make the spec. more clear. |
| QC | Seems not needed for such addition |
| Lenovo | We are OK with the TP which can make the spec more clear. |
| Samsung | OK with the TP. |
| Nokia/NSB | We are fine with/without the TP. |
| OPPO | For implicit BFD RS determination, it is not necessary to deliberately mention whether the mandatory NBI RS set or sets is (are) configured. UE would not be confused. |
| ZTE | In our initial thought, it seems unnecessary. |
| vivo | It seems unnecessary. |
| CATT | Support. An explicit RRC parameter is needed to indicate UE to determine one implicit BFD-RS set or two implicit BFD-RS sets. If the TP is not supported, other RRC parameter (e.g., RRC parameter to indicate “cell-specific BFR” or “TRP-specific BFR”) is still needed. |
| Mod | Views of companies are summarized as follows: - Support the above TP (from R1-2204336): DCM, Samsung, CATT - Above TP (from R1-2204336) is not needed: Apple, QC, OPPO, ZTE, vivo To opponent of this TP, I’d like to understand how does a UE know whether it should determine one BFD-RS set for per-cell BFR or two BFD-RS sets for per-TRP BFR? |
| Huawei, Hisilicon | The TP seems not necessary. UE can determine whether it is going to determine one or two BFD-RS sets according to the number of CORESTPoolIndex configured. |
| QC | Our understanding is that UE can determine 2 BFD sets if the following IE is configured. The IE BeamFailureRecoveryServingCellConfig is used to configure the UE with candidate beams for beam failure recovery in case of beam failure detection in a serving cell when two BFD sets are configured. See also TS 38.321 [3], clause xxx. Editor’s note: how to refer BFD sets will depend how those will be implemented. Same reason candidatebeamlists are not yet associated to BFD sets in field description - BeamFailureRecoveryServingCellConfig information element -- ASN1START -- TAG-BEAMFAILURERECOVERYSERVINGCELLCONFIG-START BeamFailureRecoveryServingCellConfig-r17 ::= SEQUENCE { rsrp-ThresholdBFR-r17 RSRP-Range OPTIONAL, -- Need M candidateBeamRSList-r17 SEQUENCE (SIZE(1..maxNrofCandidateBeams-r17)) OF CandidateBeamRS-r16 OPTIONAL, -- Need M candidateBeamRSList2-r17 SEQUENCE (SIZE(1..maxNrofCandidateBeams-r17)) OF CandidateBeamRS-r16 OPTIONAL, -- Need M ... } |
| Apple | Regarding the BFD set determination, we share the same view with QC. |
| Mod | Views of companies are summarized as follows: - Support the above TP (from R1-2204336): DCM, Samsung, CATT - Above TP (from R1-2204336) is not needed: Apple, QC, OPPO, ZTE, vivo, HW, QC Seems no consensus can be reached on this issue. Suggest to close this discussion. |

Reference

| 1 | R1-2203259 | Remaining issues on multi-TRP deployment | ZTE |
| --- | --- | --- | --- |
| 2 | R1-2203422 | Maintenance issues on Rel-17 M-TRP enhancements | CATT |
| 3 | R1-2203506 | Maintenance on enhancements for multi-TRP Deployment | Vivo |
| 4 | R1-2203856 | Maintenance on Rel-17 multi-TRP and HST-SFN | Samsung |
| 5 | R1-2204138 | Text proposal on multi-TRP deployment | LG Electronics |
| 6 | R1-2204170 | Remaining issues on multi-TRP deployment | Lenovo |
| 7 | R1-2204200 | Remaining Issues on Multi-TRP Enhancement | Apple |
| 8 | R1-2204336 | Remaining issues on enhancements for Multi-TRP Deployment | NTT DOCOMO |
| 9 | R1-2204536 | Maintenance of enhancements for Multi-TRP Deployment | Nokia, Nokia Shanghai Bell |
| 10 | R1-2204683 | Maintenance of Rel-17 beam management for multi-TRP | MediaTek |
| 11 | R1-2204764 | MIMO Enhancements for Multi-TRP Deployment | Intel |
| 12 | R1-2203773 | Remaining issues on M-TRP beam management, SRS and CSI enhancement | Xiaomi |