3GPP TSG RAN WG1 #109-e                                                                                                  R1-2205340

- e-Meeting, May 9th – 20th, 2022

Source:	Moderator (ZTE)

Title:	Summary#2 of Email discussion on side control information

Agenda Item:	9.8.1

Document for:     Discussion and Decision

- Introduction

In RAN1#109e meeting, the SI on NCR is initialized. According to the companies’ inputs, discussion on following essential aspects are summarized as below:

- Modelling and terminology of NCR

- Side control information: Beam information

- Side control information: ON-OFF information

- Side control information: TDD information

- Side control information: Timing information

- Side control information: PC information

Companies are encouraged to provide the inputs for corresponding topics.

- Topic-1: Modelling and terminology of NCR

  - Company view (Round-1)

As mentioned by companies that the terminology along with the reference modelling of NCR should be defined for this SI to facilitate the discussion. Based on the views shared by many companies e.g., contribution from [Huawei, ZTE, vivo, Sony, Samsung, Lenovo, CMCC, DoCoMo, LG, Ericsson, ETRI, QC] with corresponding illustration, compared to the legacy RF repeater, additional component to enable the communication between gNB and NCR to receive the control information is also needed.  Regarding the protocol structure of this component, since it’s mainly determined by the design of side control information and others, it’s better to consolidate it later and according to the scope of this SI, supports of L1/L2 functionality can be the baseline. Then, the following including the modelling of network controlled repeater, definition of functionality and terminology are proposed by FL:

Proposal 1-1: Recommend to capture the following model of network-controlled repeater in TR 38.867.

- The NCR-MT is defined as a component to maintain the Control link (C-link) between gNB and NCR to enable the information exchanges (e.g., side control information). The C-link is based on NR Uu interface and in same frequency band as Forwarding-link (F-link as described below).

- The NCR-RU is defined as a component to perform the analog pass-through of all data between gNB and UE via F-link for backhaul (FLB) and F-link for service (FLS). The behavior of F-link will be controlled according to the received side control information from gNB.

- Note: As the baseline, the L1/L2 functionality is assumed be supported by the NCR-MT and the detailed protocol structure of NCR-MT will be further discussed.

Companies are encouraged to share your views and if there is concern, please directly propose the corresponding updates.

| Companies | Comments and Views |
| --- | --- |
| ETRI | We are considering two groups of antennas (or antenna panels), one for NCR-MT and the other one for NCR-RU. In this assumption, the antennas (or antenna panels) for NCR-MT can be utilized for both SCI reception and forwarding the RF signals. The figure in proposal 1-1 may be aligned with our thoughts from protocol-stack perspectives. However, from antenna configuration perspectives, do you assume three groups of antennas (or antenna panels), i.e., one for NCR-MT and one for NCR-RU to the gNB side and one for NCR-RU to UE side? We think the system model should be able to cover the possible implementations including above examples. Maybe we could start from assuming two different protocol stacks for control plane and user plane, respectively. With this approach, we may not need to struggle to newly define C-link, FLB, FLS, etc. |
| vivo | For the 2nd bullet, the F-link may behave based on indication of legacy signalling, e.g., SIB may inform semi-static TDD. It is not precise to restrict that the F-link is only controlled by side control information. - The NCR-RU is defined as a component to perform the analog pass-through of all data between gNB and UE via F-link for backhaul (FLB) and F-link for service (FLS). The behavior of F-link will be controlled according to the received side control information from gNB. For the note, we assume that L3 functionality is also supported by MT, the beam management of FLB and C-link can reuse legacy procedure, i.e., DCI MAC CE and RRC will be used as the beam management signalling. - Note: As the baseline, the L1/L2/L3 functionality is assumed be supported by the NCR-MT and the detailed protocol structure of NCR-MT will be further discussed. |
| Ericsson | We should properly differentiate between the antennas used for donor/backhaul and service/access links, respectively, e.g., BS-side and UE-side antennas. Contrary to ETRI, we don’t think separate antennas are needed/motivated for the MT and forwarding functions towards the gNB. Since it is not apparent how the MT and forwarding will be separated, we think it is more appropriate to use MT function and forwarding function instead of components. In addition, also the MT can include an RU, we think that term should be avoided for the forwarding function. Among other things, this would require an extensive specification of forwarding antenna configuration whereas a joint antenna would eliminate that need. Considering inband operation, we don’t see any need for separate MT and forwarding antennas towards the gNB. Assuming shared BS-side antennas, we provide one version of suitable terms in our contribution and other companies provide others. In particular, we think that we should properly differentiate between the different repeater links and base repeater terminology on terminology from similar fields (e.g., Rel-17 repeaters, NTN) where the terms donor and service links are used. Other companies have used backhaul and access links related to IAB. Either is fine by us. If companies think a separate control link is needed, it can also be included. Using the term forwarding for both access and backhaul links opens up for misunderstandings. Finally, it is not RAN1’s responsibility to define the protocol structure of the NCR-MT. RAN1 specifies RAN1 functionality within the RAN1 realm. The necessary protocols will result from that. |
| Nokia | It may help clarify in this case that side control information here includes, L1/L2 signaling, higher layer dedicated config, and potentially system information. Additionally, it is unclear if the diagram is intended to indicate that NCR C-link is bi-directional. We believe more discussion is necessary regarding whether the NCR control interface must be bi-directional, or whether the NCR can be controlled via DL signaling exclusively. |
| Apple | - In general, our preference is to use ‘function entity’ as in current high-layer specification (e.g., MAC spec) to define the ‘NCR-MT’ and ‘NCR-RU’, instead of ‘component’. The reason is to avoid misinterpretation on the reference model for real implementation. - On the exact terms, we slightly prefer to use ‘backhaul’ link and ‘access link’. For each ‘backhaul’ link or ‘access link’, there are still two directions, one is ‘reception’ and the other is ‘forwarding’. For example, ‘forwarding’ and ‘reception’ for ‘backhaul’ link and ‘access link’ can be termed as ‘F-B’ and ‘R-B’, ‘F-A’ and ‘R-A’ respectively. |
| CATT | 1.regarding the note, we suggest to change it to: As the baseline, at least L1/L2 functionality is assumed be supported by the NCR-MT and the detailed protocol structure of NCR-MT will be further discussed. - It may not be necessary to have the drawing of internal of the NCR (MT RU antenna etc), since the understanding/implementation of each companies may be different. The important and necessary part is the interations (links) between NCR/gNB/UE |
| Lenovo | We are generally fine with the model. We have two suggestions as shown below. The reason for the first change is that not only data but also control information will be forwarded by repeater. The reason for the second change is that backhaul is always paired with access. - The NCR-RU is defined as a component to perform the analog pass-through of all information between gNB and UE via F-link for backhaul (FLB) and F-link for access (FLA). The behavior of F-link will be controlled according to the received side control information from gNB. |
| NEC | We agree in general. And there are some suggestions on the first bullet: 1. For the first bullet, in-band or out-band is not discuss yet, it’s too early to limit the resource carried the exchanging information. 2. Capability information for repeater is important for gNB to determine the side control information, we think them should be one of the exchanged information too. The NCR-MT is defined as a component to maintain the Control link (C-link) between gNB and NCR to enable the information exchanges (e.g., side control information, capability information). The C-link is based on NR Uu interface and in same frequency band as Forwarding-link (F-link as described below). |
| NTT DOCOMO | We support the proposal. We also assume that NCR has NCR-MT to receive side control information and NCR-UR for amplify-and-forward the signals from gNB/UE. |
| AT&T | We prefer to use access, backhaul, and control links for simplicity. We are open to considering both cases where the MT shares spectrum/antennas with the RU or has a separate antenna and operates out-of-band from the access and backhaul links. |
| Intel | We support the proposal. For the note, we support CATT’s suggestion that ‘at least’ L1/L2 functionality is assumed be supported by the NCR-MT. |
| CMCC | We support to have a clear definition for the links among gNB, NCR and UEs, which will align the language and facilitate the further discussions. Whether those definition would be captured in the spec depends on the future discussion and normative works. From our understanding, the control link of MT and the forwarding link for backhaul could share the same antenna or antenna panels to communicate with gNB. And the function of NCR-RU part only contains forwarding and amplifying. Only radio component exists in the NCR-RU part without any protocol stack. And the NCR-MT which has a protocol stack will control the work of the NCR-RU. We share a similar view that the link between gNB and NCR and the link between NCR to UE for the data forwarding should be defined or identified clearly. For the naming issue, we slightly preferred the backhaul link and access link for the data forwarding. Or we could just change the forwarding link for service (FLS) to forwarding link for access (FLA) to align with IAB. To fully reflect the content of WID, the note in the 3rd bullet could be updated as below. This is similar as vivo’s proposal but with some editorial updates. - Note: As the baseline, the L1/L2 functionality and the associated configurations are is assumed be supported by the NCR-MT and the detailed protocol structure of NCR-MT will be further discussed. |
| ZTE | We support this proposal to model the general functionality of the repeater. - Regarding the detailed assumption and clarification on the RF configuration, e.g., antenna, it’s covered by the further clarification via the proposal below. And the whole model is done from functionality perspective if we change the component to functionality. - For the terminology, it’s also fine to update it as forwarding for access if preferred by majority, but keep the different as C-link and FLB is certainly needed to for different functionality. Meanwhile, as stated in the scope, the NCR, e.g., the forwarding behavior will be under the control of the network via side control information, we need to clarify it in the model. Similar comments to the inband part, it has already defined in the scope as general assumption. - For the terminology, to address the potential ambiguity, e.g., also the MT can include an RU, we may can update the NCR-RU as NCR-FU (forwarding unit). For the L1/L2 functionality, taking it as the baseline is aligned with the scope. It can be further updated based on the progress in future. |
| Rakuten | We have similar view as ZTE. Regardless of the agreement on capturing in TR, for the future discussion, it would be good to have agreements on some terminology definition. |
| Sony | For the first bullet, the information exchanged between the gNB and the NCR-MR component can be described more accurately. From the SID, only side control information and information for smart repeater management, including identification and authorization, needs to be exchanged. In our view, NCR-RU is controlled via side control information or OAM procedures, only. The side control information, however, might be implicitly extracted from, e.g., SIB information intended for the NCR-MT, such as UL/DL TDD configuration and timing advance information. The sentence “The behavior of F-link will be controlled according to the received control side information from gNB,” in the second bullet, can be kept. Regarding the note, we prefer the formulation “Note: As the baseline, at least the L1/L2 functionality …” We think that supporting higher layers at the NCR-MT can significantly reduce the integration effort. |
| MediaTek | We share a similar view that the control link of MT and the forwarding link for backhaul could share the same antenna or antenna panels to communicate with gNB. We think the intention of FL’s proposal is more like to define functionality but not ‘a component’, as Apple proposed. We also agree with CATT to add “at least” because RRC configuration is necessary. |
| Spreadtrum | We think a clear definition for the links among gNB, NCR and UEs considering the reception and transmission is necessary, Apple’s suggestion is fine. |
| Fujitsu | We generally support the proposal. We think some clarifications on the terminology ‘side control information’ may be necessary. |
| LG | In terms of structure of NCR, we have similar view with the proposal. However, for the first bullet, there is a concern about the restriction that C-link and F-link should be in the same frequency band. We have not yet discussed the possibility of multi-carrier operation of NCR, and if QCL can be assumed between C-link and F-link, it is not necessarily limited to the same frequency band. For example, it may be considered that C-link and F-link operate on different carriers, and it may also be discussed that C-link operates as a single carrier but F-link operates as a multi-carrier. For the second bullet, it is also discussed that the operation of the RU is controlled by the configuration given to the MT. For example, the DL/UL of the RU may be determined by the TDD configuration provided by the MT. It seems that the current expression can be interpreted as that the F-link is controlled only through side control information. Therefore, it is preferred to express it in such a way that side control information is used to control the behavior of F-link. |
| CEWiT | Support the proposal |
| Huawei, HiSilicon | We suggest to reuse the terminology from SID as much as possible. It is more accurate to replace “analog” by “amplify-and-forward”. The term “service” seems like an application layer terminology, and we suggest use “access”. Moreover, the note is not needed. As a baseline assumption for 9.8.2, L1/L2/L3 are supported. We propose to revise as: Proposal 1-1: Recommend to capture the following model of network-controlled repeater in TR 38.867. - The NCR-MT is defined as a component to maintain the Control link (C-link) between gNB and NCR to enable the information exchanges (e.g., side control information). The C-link is based on NR Uu interface and in same frequency band as Forwarding-link (F-link as described below). - The NCR-RU is defined as a component to perform the analog pass-through of all data amplify-and-forwarding of the signals between gNB and UE via F-link for backhaul (FLB) and F-link for service (FLS) access (FLA). The behavior of F-link will be controlled according to the received side control information from gNB. - Note: As the baseline, the L1/L2 (and its configuration) functionality is assumed be supported by the NCR-MT and the detailed protocol structure of NCR-MT will be further discussed. |
| Samsung | We think that NCR-RU is a layer-1 component, the mentioning of “all data” is not appropriate. Also, the term “link for backhaul” and “link for service” may imply a higher layer functionality of NCR-RU. Hence, we suggest the following changes. Proposal 1-1: Recommend to capture the following model of network-controlled repeater in TR 38.867. - The NCR-MT is defined as a component to communicate with a gNB via Control link (C-link) to enable the information exchanges (e.g., side control information). The C-link is based on NR Uu interface and in same frequency band as Forwarding-link (F-link as described below). - The NCR-RU is defined as a component to perform the analog pass-through of UL/DL RF signals between the gNB and UEs via the F-link at gNB-side and the F-link at UE-side. The behavior of the NCR-RU is controlled by the side control information. Note: As the baseline, the L1/L2 functionality is assumed be supported by the NCR-MT and the detailed protocol structure of NCR-MT will be further discussed. |
| IIT-K | Regarding the 2nd bullet, we agree with Samsung and would suggest not to restrict the NCR-RU to “analog pass through of all data” at this stage. We propose to further study the gains of having limited L1 capability at RU. The option of having such capability is that certain common signals such as SSB for FLS can be generated at L1 layer, reducing the overhead for forwarding of such signals as mentioned in our contribution (R1-2204758). |
| Philips | We support the proposal, noting CATT’s suggestion that ‘at least’ L1/L2 functionality is assumed be supported by the NCR-MT. In addition, we support AT&T’s suggestion about link terminology. |
| Qualcomm | We generally support this proposal with some comments (especially we have concerns about the note). Aligned with other companies’ suggestions, we propose the following changes for more clarity. - The NCR-RU is defined as a component to perform the analog pass-through of all data the signals between gNB and UE via F-link for backhaul (FLB) and F-link for service (FLS). The behavior of F-link will be controlled according to the received side control information from gNB. - Note: As the baseline, at least the L1/L2 functionality is assumed be supported by the NCR-MT and the detailed protocol structure of NCR-MT will be further discussed. Regarding the first change (all data the signals): - It is not just data, but control and reference signals can also be forwarded by the repeater. - Not necessarily “all” the signals are forwarded by the repeater. In an implementation, the UE may have other alternative paths to the network. Regarding the second change (at least the L1/L2): - The note in its current form suggests that RAN1 already decided a baseline architecture for NCR-UE (which only support L1/L2 functionality), and more details of such a baseline architecture will be worked out later. We, in RAN1, indeed are not in the position of deciding about the NCR-UE’s architecture. However, it is safe to assume NCR-UE at least supports L1/L2 functionality. |
| Charter | We are in agreement with the architecture shown in proposal 1-1 but also share the same concern as LG regarding the statement of restricting C-link and F-link in the same frequency band as we think support of multi-carrier operation in NCR may be a viable deployment scenario. We are of the view to use as much legacy work as possible for NCR-MT while keeping the cost down. We are fine with the assumption to start with L1/L2 protocol stack but will like to evaluate if some low feature version of L1/L2/L3 stack can be considered to reduce the implementation cost. |
| CableLabs | We prefer a simplified definition such as backhaul link, access link and control link. For backhaul and access linkw, we are supportive on Apple proposal on further defining receiving and forwarding separately for each link. |
| Sharp | We support the proposal in general. Same as many companies above, the forward link for service may also include other channels, such as SSBs, RACH, etc. Also, the last bullet, we support “at least” L1/L2 signalling. Higher layer signaling may also be used for side information, e.g. on the SSB configuration at NCR, TDD UL/DL configuration etc. |

Meanwhile, regarding the reference structure (e.g., RF structure) for the NCR, as mentioned by companies, in general, either same or different RF component can be used at NCR side for C-link and FLB. And as highlighted by others, e.g., vivo, it’s better to ensure the QCL-relationship between C-link and FLB. Then, the following is proposed:

Proposal 1-2: Recommend to capture the following assumption of network-controlled repeater in TR 38.867.

- As the baseline, same RF component at NCR side is used for C-link and FLB.

  - If dedicated RF components are used (e.g., antenna) at NCR side for C-link and FLB, respectively, these RF components should be co-located to ensure the quasi collocation relationships between C-link and FLB.

- Different RF components are used at NCR side for the gNB-NCR link (including C-link and FLB) and NCR-UE link (i.e., FLS).

Companies are encouraged to share your views and if there is concern, please directly propose the corresponding updates.

| Companies | Comments and Views |
| --- | --- |
| ETRI | Support in principle. We have similar comments with proposal 1-1 for the terminology aspects. |
| vivo | Agree in principle However, We wonder whether RF architecture needs to be clarified or not. It is sufficient to make assumption on QCL relationship. |
| Ericsson | Partly agree. We think that the alternative with different BS-side antennas for control and forwarding can be omitted already now. If it isn’t, it would require RAN1 to specify new frameworks for beam configuration, timing, power control, etc. for a function that is supposed to have no intelligence. This ambition goes beyond the scope of the SI and such specification work is not included in the given time budget for the SI/WI. |
| Nokia | We could agree that independent control management framework C-link and FLB beam management and timing is not supported. |
| Apple | Given the current situation, one possible way forward is to agree the first bullet and put ‘FFS’ on the second sub-bullet (i.e., ‘separate antenna panel’). If we have time in this meeting, we can still discuss it. On the other hand, as commented by vivo already, we may focus on the QCL assumptions between DL/UL in backhaul links, instead of discussing the RF structure up to vendor-specific implementations. This is the methodology we used since Rel-15 for FR2 related designs. |
| CATT | Suggest to de-prioritize dedicated RF component option |
| Lenovo | Fine with the proposal. |
| NEC | Support it in principle We wonder whether need to descript the QCL type for the two links. If yes, which QCL type is reasonable. |
| NTT DOCOMO | We are fine with the proposal. We think sharing the components or co-located components is reasonable assumption for NCR configuration. |
| Intel | We also prefer to deprioritize separate RF component. For separate RF component, as mentioned by NEC, we needs to discuss the QCL type. |
| CMCC | Fine with the proposal if the terminologies are aligned with the previous proposals. It is fine to us that the C-link and FLB at NCR share a same set of RF components (including antennas) or have a QCL assumption, which means the control link and FLB could share or partially share the procedure of beam management. |
| ZTE | We are fine to support this proposal. Regarding the QCL relationship for dedicated configuration, all QCL (QCL A + D) types should be considered to minimize the efforts. |
| Rakuten | Agree. Good to minimize the cost of NCR. |
| Sony | As commented by some other companies, we do not see a need to ban implementations with dedicated NCR-MT antennas and RF chains. It is so far sufficient to say that “the quasi-colocation relationships may be assumed between signals C-link signals and FLB signals.” Whether same RF component is shared by C-link and FSB or dedicated RF components are used can be left to implementations. |
| MediaTek | We prefer to de-prioritize dedicated RF component option and focus on the case that C-link and FLB using same RF component or antenna sets. We are open to discuss the dedicated RF component case if the functionality for baseline case, i.e., C-link and FLB using same RF component, is completed. |
| Spreadtrum | Agree in principle. |
| KDDI | We support this proposal in principle. Regarding the dedicated RF components, to share the same RF components is beneficial in terms of deployment. |
| Fujitsu | Regarding the first bullet, we agree same RF component for C-link and FLB is the baseline. In our view, dedicated RF components could be allowed for implementation, only if it does not require extra standardization effort. Therefore, we suggest following changes. Proposal 1-2: Recommend to capture the following assumption of network-controlled repeater in TR 38.867. - As the baseline, same RF component at NCR side is used for C-link and FLB. - If dedicated RF components are used (e.g., antenna) at NCR side for C-link and FLB, respectively, these RF components should be co-located to ensure the quasi collocation relationships between C-link and FLB. It is up to implementation. No extra standardization effort for this case is needed. - Different RF components are used at NCR side for the gNB-NCR link (including C-link and FLB) and NCR-UE link (i.e., FLS). |
| LG | We would like to consider same RF component and dedicated RF component with the same priority. Also, we have the similar view with vivo. Assumption on QCL relationship seems enough, rather than to limit the RF architecture for NCR. |
| CEWiT | Support the proposal |
| Huawei, HiSilicon | It seems undesirable for the NCR to have different RF components (e.g., antenna) for C-link and FLB. A shared structure has the benefit of lower hardware cost and also has smaller specification impact. Thus, we prefer to delete the assumption of different RF components: Proposal 1-2: Recommend to capture the following assumption of network-controlled repeater in TR 38.867. - As the baseline, same RF component at NCR side is used for C-link and FLB. - If dedicated RF components are used (e.g., antenna) at NCR side for C-link and FLB, respectively, these RF components should be co-located to ensure the quasi collocation relationships between C-link and FLB. - Different RF components are used at NCR side for the gNB-NCR link (including C-link and FLB) and NCR-UE link (i.e., FLS). |
| Samsung | The intention of this proposal is understandable but the wording needs to be refined. For example, the case in the first sub-bullet is actually in parallel with the case in the first bullet. Also, the wording of “NCR side” is confusing. |
| IIT-K | We support the proposal. |
| Philips | Agree in principle with the remark that, at this point, there seems no reason to rule out implementations that use dedicated RF components for C-link and FLB. We share Sony’s view that it is sufficient to say that a “quasi collocation relationships between C-link and FLB” may be assumed. |
| Qualcomm | We support this proposal and agree with other companies that we should keep the scope of study/work manageable. Having said that, we are also OK with Apple’s suggestion to put an “FFS” on the sub-bullet. |
| Charter | Wording needs to be updated. Our assumption is that NCR-MT and NCR-RU in the FLB share the RF components (especially antennas), while the NCR-RU has another set of RF components (e.g., antennas) in FLS. |
| CableLabs | We are supportive on first main bullet. For a sub-bullet of first main bullet, we prefer to either remove or put FFS. The case seems require significant impact/discussions where benefits of the case are a bit unclear. If we consider different RF between two, we prefer to deprioritize the case and focus same RF first. Second main bullet is okay with us. |
| Sharp | We support the first bullet. For the second bullet, both same RF and different RF can be considered. |

In addition, some companies highlights that clarification on the operation between different links with different functionality and direction (e.g., gNB and NCR) is needed.  Then, from FL’s perspective, following assumption seems to be reasonable:

Proposal 1-3: Recommend to capture the following assumption of network-controlled repeater in TR 38.867.

- The C-link and FLB in DL (from gNB to NCR) can be performed simultaneously or in TDM way.

- As the baseline, the C-link and FLB in UL (from NCR to gNB) is performed in TDM way.

Companies are encouraged to share your views and if there is concern, please directly propose the corresponding updates.

| Companies | Comments and Views |
| --- | --- |
| ETRI | Not sure whether we need the second bullet or not (DL and UL should be TDMed in this SI without any new agreement). |
| vivo | In our understanding, whether or not MT and RU can transmit in C-link and in FLB simultaneously, should be subject to repeater’s capability. If we only assume the TDM capability, do we need any rule to prioritize a given link, i.e., C-link or FLB. To simplify the discussion, we propose the following direction. If there is MT UL transmission, NCR-MT always transmit the MT UL. It is up to NCR capability whether to forward a UL transmission simultaneously by NCR-RU. |
| Ericsson | Partly agree. We don’t see a need to include the restrictive DL TDM operation since, from the gNB perspective, the repeater and UE will be collocated and hence easily co-scheduled. Furthermore, for UL, we can leave for the gNB scheduler to resolve potential prioritization. |
| Nokia | In general we are fine with the proposal, but strict TDM may be spectrally inefficient since expected bandwidth for control plane could be considerably smaller than FL. It would be helpful to understand the motivation for strict TDM. |
| Apple | This proposal is not clear for us. First, in our understanding, the ‘C-link’ in FIG is a logic tunnel to carry SCI and we do not see the need to introduce limitation between ‘C-link’ and FW link. One potential restriction is the link between ‘FLB’ from NCR to gNB and ‘R-B’ from gNB to NCR if the antenna panel at BS side is shared for DL/UL on backhaul link. Then, the following is the intended behaviour assuming a shared antenna panel? - As the baseline, the C-link reception backhaul link (R-B) in DL (from gNB to NCR) and FLB in UL (from NCR to gNB) is performed in TDM way. |
| CATT | Agree with the first bullet. |
| Lenovo | We prefer also to include simultaneous uplink of C-link and FLB at the first stage. Priority of simultaneous operation and TDM operation can be discussed further later. - As the baseline, the C-link and FLB in UL (from NCR to gNB) is performed in TDM way. Additionally, the C-link and FLB in UL (from NCR to gNB) can be performed simultaneously if interference between baseband and RF can be solved. |
| NEC | For the first bullet, does “simultaneously” means FDMed C-link and FLB in DL? If yes, we think it can’t be applied to UL too. |
| NTT DOCOMO | We support the proposal. And we are also open for the discussion whether TDM operation is necessary or not for DL and UL (it may be up to the structure of NCR). |
| AT&T | Ok for TDM as a baseline, but should consider the feasibility to support other multiplexing options for the backhaul and control links. |
| Intel | In general, we support the proposal. For DL, it can be up to gNB whether TDM or FDM for both links. For UL, for shared antenna, it is unclear whether/how a repeater can support FDM, and any other standard impact. We don’t think we have sufficient time to figure out all details. Therefore, we prefer to focus on TDM case. |
| CMCC | From our understanding, in DL, the C-link and FLB could work simultaneously. But it is not necessary to let the C-link and FLB always work simultaneously. We could be a bit more aggressive to delete “or in TDM way”, if it is acceptable to the majority. For the 2nd bullet, it is not clear to us if there is any implementation issue to integrate C-link and FLB link into one transmission. If there could be a common understanding about that, we have no problem update the 2nd bullet. Currently, the 2nd bullet is fine to us. |
| ZTE | In general, we support this proposal to take the basic assumption to clarify scheduling between DL of C-link and DL of FLB, UL of C-link and UL of FLB. Regarding the detailed scheduling mechanism/priority, it’s purely determined by the network’s implementation. |
| Xiaomi | Support the first bullet for DL. For UL, we think it is up to gNB that whether schedule the NCR-MT and UE simultaneously, which is also subject to the NCR’s frequency related capability. |
| Sony | We agree with vivo and others that whether NCR-MT and FLB can transmit simultaneously, in the UL, should depend on the repeater’s capability. From the point of view of standardization, there is no extra complexity in allowing NCR-MT and FLB to transmit simultaneously. We are open to discuss whether a prioritization rule is needed for NCRs that are not capable of simultaneous NCR-MT and FLB transmissions. |
| MediaTek | We agree with the first bullet. TDM or simultaneous transmission can be discussed later; it is related to repeater’s capability and gNB’s scheduling behaviour. |
| Spreadtrum | We think the restriction on TDM is unnecessary, the choice of transmission scheme is affected by on NCR’s capability. |
| KDDI | For DL, we support this proposal. For UL, we are fine with this proposal as a baseline. We also think that it is helpful to consider the feasibility to support other multiplexing option such as FDM. |
| Fujitsu | For the 1st bullet, we are confused about the typical application scenario of DL ‘TDM’. Simultaneous DL should be the baseline. The DL TDM depends on implementation. No extra STD effort is required. For the 2nd bullet, we are fine with it. |
| LG | For UL of C-link and FLB, we don’t support to make TDM operation as baseline. We would like to consider NCR with the capability of simultaneous uplink transmission for C-link and FLB together, since simultaneous operation also beneficial in latency perspective. |
| CEWiT | Support the proposal |
| Huawei, HiSilicon | Support. Agree with some companies that the FDM/TDM operation between MT and RU can be left to implementation. |
| Samsung | Ok with the proposal in general. |
| IIT-K | Support the proposal. |
| Philips | We agree with LG. |
| Qualcomm | We support this proposal. Regarding the second bullet, we think simultaneous UL transmission of C-link and FLB (NCR to gNB) is subject to (a) the capability of the NCR to support such an operation – i.e., receive UE’s signals, amplify it, combine it with its own generate signal, and forward it, and (b) interference and the required repeater’s power setting. Given such considerations, it is appropriate to assume a simpler TDM scheme as the baseline. |
| Charter | We support the C-link (control only) and Forward links (data in backhaul and service links) should be TDMed. This will enable the repeater to decode the side control information and set up respective paths as per the control information. Also, it will enable the feedback from NCR-MT to gNB easy rather than multiplexing the UL data and control together (if feedback from NCR-MT is agreed). |
| CableLabs | In general, we are fine to study both cases for control and backhaul reception, which may require further discussions on repeater capability and expected behavior. |
| Sharp | We are fine with the first DL proposal although it does not exclude anything. For UL, TDM as the baseline is a simple approach to avoid multiplexing with forwarded data from UE. However, FDM may be supported with NCR capabilities and scheduling constraints. |

    - Summary of the 1st round

According to the further discussion, the following agreements have been achieved for Topic-1.

Agreement

Capture the following model of network-controlled repeater in TR 38.867.

- The NCR-MT is defined as a function entity to communicate with a gNB via Control link (C-link) to enable the information exchanges (e.g. side control information). The C-link is based on NR Uu interface.

  - Note: Side control information is at least for the control of NCR-FU

- The NCR-FU is defined as a function entity to perform the amplify-and-forwarding of UL/DL RF signal between gNB and UE via backhaul link and access link. The behavior of the NCR-FU will be controlled according to the received side control information from gNB.

  - Company view (Round-2)

For the [Proposal 1-2], the main concern is whether to support the case that dedicated RF components are used at NCR for C-link and FLB as commented by [Ericsson, Apple, CATT, Intel, MTK, Huawei, CableLabs, QC] But as [Sony, LG, Philips] replies, we only need to clarify the QCL and whether it’s achieved by same or dedicated RF component is based on implementation. In addition, regarding the QCL type, in order to ensure the same timing relationship between FLB and C-link, in case of dedicated RF component, both Type-A and D should be kept.

Then, from moderator’s perspective, the following updated version is proposed for GTW:

Updated Proposal 1-2: Recommend to capture the following assumption of network-controlled repeater in TR 38.867.

- As the baseline, same RF component of NCR is used for C-link and FLB.

  - If dedicated RF components are used (e.g., antenna) at NCR for C-link and backhaul link, respectively, these RF components should be co-located to ensure the quasi collocation relationships between C-link and backhaul link for both Type-A and Type-D

    - Note: It is up to implementation without extra standardization effort.

- Different RF components are used at NCR for the backhaul link and access link.

Companies are encouraged to share your views and if there is concern, please directly propose the corresponding updates.

| Companies |  | Comments and Views |  |
| --- | --- | --- | --- |
| Lenovo |  | Fine with the proposal. We are wondering there should be “if applicable” for QCL type D as in legacy NR specification. Updated Proposal 1-2: Recommend to capture the following assumption of network-controlled repeater in TR 38.867. - As the baseline, same RF component of NCR is used for C-link and FLB. - If dedicated RF components are used (e.g., antenna) at NCR for C-link and backhaul link, respectively, these RF components should be co-located to ensure the quasi collocation relationships between C-link and backhaul link for both Type-A and Type-D(if applicable) - Note: It is up to implementation without extra standardization effort. - Different RF components are used at NCR for the backhaul link and access link. |  |
| CMCC |  | Fine with the proposal. And Lenovo’s updates seem more accurate. From our understanding, the MT part should have the knowledge of channel state of the backhaul link. And then it can facilitate the control or the beamforming of the BH link. |  |
| CEWiT |  | Support the proposal |  |
| vivo |  | The proposal should clarify that, the assumption is applied only if C-link and F-link are operating in the same frequency band. |  |
| IIT-K |  | Support. |  |
| Fujitsu |  | We are fine with the proposal. |  |
| NEC |  | Support this proposal. |  |
| Ericsson |  | Support with modifications. It is not possible to refer to QCL in this context since QCL relates to different DL RSs and neither MT nor FU are capable of that. The proposal relates to the BS-side (towards the gNB) of repeater operation, which cannot be associated with a reference sequence, since for that case the repeater would correspond to a UE. For that reason, we propose to delete the sub-bullet about dedicated RF components. |  |
| AT&T |  | Support the comments from vivo |  |
| Nokia |  | We find the term “RF components” vague and would like to better understand the implications of this proposal with specific regard to RAN1 impact. Does this imply that the NCR-MT and NCR-FU backhaul use a common set of beam configurations (this seems to be the primary implication as indicated by the sub-bullet)? Must they be on the same band and/or carrier? With a similar consideration, does the second sub-bullet imply that NCR-FU backhaul and NCR-FU access link could be on a separate band and/or carrier. |  |
| Qualcomm |  | Support this proposal. We also acknowledge Ericsson’s comment about the definition of different QCL types. In our view, the sub-bullet can still remain, but the reference to QCL types should be reworded to something more generic. |  |
| Sony |  | We support this proposal. Ok to include Lenovo´s suggestion. Ok also to include vivo’s clarification. |  |
| Charter |  | We share the view of Nokia regarding the use of the term “RF components”. What does “Co-location” mean in terms of RF components? Is it to specify in-band for NCR-MT and NCR-FU? Suggest that this proposal be reworded to make it consistent with the assumptions of using the same beams for NCR-MT and NCR-FU (backhaul). |  |
| NTT DOCOMO |  | We support the proposal. |  |
| Apple |  | We are generally ok with FL proposal with the following comments - On the first sub-bullet, if it is kept, it is more proper to put it under the 2nd bullet as it talks about the restriction if the ‘Different RF components is used’. - On the exact wording of sub-bullet, the QCL was defined in clause 5.1.5 of TS 38.214, which establishes the relation between channel transmission/reception and a DL/UL reference signal. For NCR, our understanding is that backhaul link is sort of transparent to NCR-FU and NCR-MT performs as legacy UE to acquire the transmission and reception beams by using the ‘typeA’ and ‘typeD’ quasi co-location. The intention of the proposal seems to ensure the ‘typeA’ and ‘typeD’ quasi co-location used by C-link at NCR-MU is maintained for channel/signals in backhaul link by NCR-FU. With the above reasoning, the following modification was suggested: - As the baseline, same RF component of NCR is used for C-link and FLB. - If dedicated RF components are used (e.g., antenna) at NCR for C-link and backhaul link, respectively, these RF components should be co-located to ensure the quasi collocation relationships between C-link and backhaul link for both Type-A and Type-D - Note: It is up to implementation without extra standardization effort. - Different RF components are used at NCR for the backhaul link and access link. - If dedicated different RF components are used (e.g., antenna) at NCR for C-link and backhaul link, respectively, these RF components should be co-located to ensure the Type-A and Type-D quasi collocation relationships used by between C-link and are applied for the channels/signals of backhaul link for both Type-A and Type-D - Note: It is up to implementation without extra standardization effort. |  |
| Intel |  | We’re generally fine with the proposal. Regarding the sub-bullets, the wording suggested by Apple seems better, but it should be under 1st bullet rathe than 2nd bullet, because 2nd bullet is talking about access and backhaul link, not C-link and backhaul link. |  |
| Samsung |  | Ok with the updated proposal in general. |  |
| Xiaomi |  | Support |  |
| ZTE |  | Support the proposal. Regarding the updates from Apple, Two parts are discussed in this proposal and in this first bullet, we mainly focus on the RF assumption at NCR side towards to BS. But in the 2nd bullet, we prefer to highlight whether same RF will be used for both backhaul and access at NCR side towards BS and UE, respectively. |  |
| LG |  | We are fine with the proposal. Following modification seems more clear in our perspective. If dedicated RF components are used (e.g., antenna) at NCR for C-link and backhaul link, respectively, these RF components should be co-located to ensure the quasi collocation relationships between C-link and backhaul link withfor both QCL Type-A and QCL Type-D |  |
| Huawei, HiSilicon |  | We share similar view with some other companies and prefer to delete the sub-bullet for dedicated RF components. |  |
| Sharp |  | Support the proposal. |  |
| CATT2 | Support with improved wording to describe QCL relationship. |  |

For the [Proposal 1-3], some companies think the current bullets may be restrictive and also highlight that the scheduling is purely up to gNB’s implementation. From moderator’s perspective, it’s true that the corresponding action is belong to gNB’s decision. But as the compromise, it’s still possible to propose some examples to clarify the multiplexing between C-link and backhaul link. Then, the updated version is proposed:

Updated Proposal 1-3: Recommend to capture the following examples of the scheduling of C-link and FLB by gNB in TR 38.867.

- The DL of C-link and DL of backhaul link can be performed simultaneously or in TDM way.

- The UL of C-link and UL of backhaul link can be performed in TDM way.

- Note: The scheduling of C-link and backhaul link is up to gNB’s implementation

Companies are encouraged to share your views and if there is concern, please directly propose the corresponding updates.

| Companies |  | Comments and Views |  |
| --- | --- | --- | --- |
| Lenovo |  | We still prefer that simultaneous UL of C-link and UL of backhaul link is also captured as an example. At least if the C-link and UL of backhaul link are in different bands, it can be performed simultaneously. Updated Proposal 1-3: Recommend to capture the following examples of the scheduling of C-link and FLB by gNB in TR 38.867. - The DL of C-link and DL of backhaul link can be performed simultaneously or in TDM way. - The UL of C-link and UL of backhaul link can be performed in TDM way. - The UL of C-link and UL of backhaul link can be performed simultaneously at least when in different frequency bands. - Note: The scheduling of C-link and backhaul link is up to gNB’s implementation |  |
| CMCC |  | Fine with Fl’s proposal. For the updates from Lenovo, although we think the it is workable for C-link and UL backhaul transmitting simultaneously in different bands. But this C-link (Band A) and BH link (Bank B) work in different bands may bring additional assumptions or issues to be clarified, such as - whether MT could have the information from BH link, - whether the TDD configuration in Band A is same as Bank B - whether Band A and Band B have a same numerology, e.g. subcarrier spacings, or share the same SSB patterns and other cell specific configurations ? - if the C-link and BH link works in the different band, single or separate antennas should be assumed, and whether the QCL assumptions are applicable. Before capturing the updates, we should solve the issues above first. |  |
| CEWiT |  | Based on third bullet scheduling is up to gNB implementation, and in the e.g., only few options are mentioned. We would like to know whether other options are precluded or not, for e.g., scheduling UL using FDM Also, would like to clarify whether “simultaneously” means just FDM or all options |  |
| vivo |  | 1.The F-link cannot be scheduled, since repeater (even gNB) does not precisely know whether transmission will be via repeater or not. However, we can say from repeater transmission perspective, we assume either FDM or TDM transmission between F-link and C-link. Updated Proposal 1-3: Recommend to capture the following examples of the transmission/reception of C-link and backhaul link scheduling of C-link and FLB by gNB in TR 38.867. - The DL of C-link and DL of backhaul link can be performed simultaneously or in TDM way. - The UL of C-link and UL of backhaul link can be performed in TDM way. - Note: The scheduling of C-link and backhaul link is up to gNB’s implementation 2.Moreover, FDMed UL multiplexing can be supported subject to repeater’s capability, assuming that C-link and F-link may transmitted in different CC with dedicated RF. - The UL of C-link and UL of backhaul link can be performed simultaneously. |  |
| Fujitsu |  | Regarding the 1st bullet, for DL, “can be performed simultaneously or in TDM way” seems to imply it is up to NCR’s capability to support simultaneously performing DL of C-link and DL of backhaul link or in TDM way. However, in our view, NCRs should support simultaneous DL receptions by default, and the gNB performs scheduling assuming NCRs can simultaneously receive DL of C-link and DL of backhaul link. If there is no clarification on necessity of TDM up to NCR’s capability, “or in TDM way” should be removed. Regarding the 2nd bullet, for UL, it is not clear whether simultaneously scheduling of UL of C-link and UL of backhaul link is still open for discussion. If it is not intended to support simultaneously scheduling as well per NCR capability, we prefer to clarify that “the UL of C-link and UL of backhaul link cannot be performed simultaneously”. Regarding the Note, we think the intention is not clear. Is the intention to clarify the scheduling is NOT depending on NCR’s capability? For example, for DL, can gNB always simultaneously schedule DL of C-link and DL of backhaul link, without any capability report from the NCR? If that is the intention, we prefer to clarify that it is without capability report from NCR. Based the above, we suggest the following modifications: Updated Proposal 1-3: Recommend to capture the following examples of the scheduling of C-link and FLB backhaul link by gNB in TR 38.867. - The DL of C-link and DL of backhaul link can be performed simultaneously or in TDM way. - The UL of C-link and UL of backhaul link can be performed in TDM way. - The UL of C-link and UL of backhaul link cannot be performed simultaneously Note: The scheduling of C-link and backhaul link is up to gNB’s implementation, without the need of capability report from NCR |  |
| NEC |  | We still confused about the reason for defining a limitation on the multiplexing method. If hardware complexity or feasibility of combining two IF/RF signals is the main reason, we think it should be captured in the TR at least. |  |
| Ericsson |  | Support the proposal. Although operation on different band for MT and FU functions is not excluded, we think it will require more extensive specification work if the FU cannot utilize the MT link towards the gNB. Essentially, it would require a second MT-like function for FU configuration and that should clearly be avoided. For that reason, we prefer the FL’s proposal before Leonovo’s modification. |  |
| Nokia |  | Support in principle. The note does not seem necessary since scheduling is always a matter of implementation, but multiplexing discussion can still focus on what channels will carry NCR-MT C-link signals, and what channels will carry NCR-FU transmissions. |  |
| Qualcomm |  | We do not support the proposal in its current form. As pointed out by some other companies, and discussed before, the NCR’s capability for simultaneous MT and FU operation should be discussed. It should be clarified what is the minimum capability of NCR (e.g., simultaneous MT RX and FU forwarding DL?), and whether there are some optional capabilities (e.g., simultaneous MT TX and FU forwarding UL?). Accordingly, the note “The scheduling of C-link and backhaul link is up to gNB’s implementation” may not be accurate, if the NCR’s capability should also be taken into account. |  |
| Sony |  | The proposal is unclear as to whether simultaneous UL operation of MT and FU is to be further discussed or not. As mentioned by others, we believe that this topic needs further discussion. Moreover, as pointed out by CEWiT and Fujitsu, it is not clear whether “simultaneous” means FDM and/or something else. We therefore propose the following modificaitons: Updated Proposal 1-3: Recommend to capture the following examples of the scheduling of C-link and FLB by gNB in TR 38.867. - The DL of C-link and DL of backhaul link can be performed simultaneously in FDM or in TDM way. - The UL of C-link and UL of backhaul link can be performed in FDM or in TDM way. Note: The scheduling of C-link and backhaul link is up to gNB’s implementation |  |
| Charter |  | In principle we support to utilize MT link for FU-link towards the gNB but also would like to not limit the NCR repeater capability of not supporting multiband operations as legacy repeater supports multiband operations. |  |
| NTT DOCOMO |  | We support the proposal. The restriction of the scheduling is up to NCR capability or structure, and if the proposal is based on the possible NCR capability or structure, we agree with the proposal. |  |
| Apple |  | We are generally supportive on moderator’s proposal with the following comments: - For DL, we support TDM or FDM multiplexing between C-link of NCR-MT and backhaul link of NCR-FU, e.g., if the side-control information is carried by means of DCI over PDCCH. - For UL, the TDM-based operation may simplify NCR and the FDM multiplexing can be additionally support. - For FDM-based operation in both DL/UL, corresponding NCR capabilities can be introduced to provide NCR implementation flexibility. Regarding the different band of C-link and backhaul/access link, our thought is that it is out of scope based on the ‘in-band’ in the following sentence: - Network-controlled repeaters are inband RF repeaters used for extension of network coverage on FR1 and FR2 bands |  |
| Intel |  | We generally support the proposal, except the note. Currently, it is unclear whether gNB can ensure TDM for UL C-link and UL backhaul link purely by scheduling, e.g., if NCR-MT may transmit PRACH when NCR-FU would forward UL signals, gNB may not know when NCR-MT would transmit UL. Therefore, we prefer to delete the note for now. |  |
| Samsung |  | Ok with the updated proposal. |  |
| Xiaomi |  | We share similar view with some other companies that capture simultaneous UL transmission in TR 38.867. We suggest adding a note to clarify that simultaneous UL transmission subjects to NCR’s capability. Updated Proposal 1-3: Recommend to capture the following examples of the scheduling of C-link and FLB by gNB in TR 38.867. - The DL of C-link and DL of backhaul link can be performed simultaneously or in TDM way. - The UL of C-link and UL of backhaul link can be performed in TDM way. - Note: The scheduling of C-link and backhaul link is up to gNB’s implementation - Simultaneous UL transmission, e.g., in FDM way is subject to NCR’s capability. |  |
| ZTE |  | We are fine with the proposal. |  |
| MediaTek |  | Not clear to us whether “The UL of C-link and UL of backhaul link can be performed simultaneously” is precluded or not, as commented by CEWiT and Fujitsu. We prefer to keep this possibility. Agree with QC that the NCR’s capability should also be taken into account. |  |
| LG |  | We don’t agree with limiting UL of C-link and UL of backhaul link to only TDM way. It potentially depends on the NCR’s capability. Considering NCR’s limited capability and latency perspective, simultaneous UL operation, e.g. FDM, would be beneficial. According to the previous agreement, FLB in the main bullet should be modified to backhaul link. |  |
| Huawei, HiSilicon |  | Support |  |
| Sharp |  | Fine with the proposal. But would like to clarify the UL of C-link and UL of backhaul can be in TDM. Does it mean FDM can be further considered or supported? |  |
| CATT2 | We are fine with the proposal. |  |

Meanwhile, during the GTW discussion, it seems that companies share different understandings on the operation band of MT and FU based on the terminology “in-band” listed in the scope. To achieve the common understanding, following clarification are made:

- Both NCR-MT and NCR-FU works in the same frequency range, the case as NCR-MT in FR1 and NCR-FU in FR2 should be precluded in this SI in Rel-18.

- The frequency range of MT should be fully within the passband of NCR-FU for both backhaul and access link regardless of whether same or different RF components are used.

- As the typical operation of in-band mode, the operation of NCR-MT and NCR-FU is expected in same carrier.

Then, following proposal is recommended from moderator’s side:

Proposal 1-4: Recommend to capture the following assumption of network-controlled repeater in TR 38.867.

- The frequency band of NCR-MT should be within the passband of NCR-FU.

  - As the baseline, the NCR-MT and NCR-FU are in same carrier.

Companies are encouraged to share your views and if there is concern, please directly propose the corresponding updates.

| Companies |  | Comments and Views |  |
| --- | --- | --- | --- |
| Lenovo |  | Fine with the proposal. |  |
| CMCC |  | Fine with the proposal. In our view, the inband repeater means both the C-link and BH/AC link work in the same band. Or at least the band of C-link/MT should be within the band of NCR-FU. If the MT works in a band outside the band of NCR-FU, I wonder if this requires normative work. |  |
| CEWiT |  | Support the proposal |  |
| vivo |  | It is reasonable to assume that BH and AL are in-band, which is similar as IAB discussion. However, for the C-link, we can consider C-link is out of band, e.g., in FR1, or in both FR1 and FR2 depending on UE’s hardware. Regarding the multiple CC operation, we assume that FU can perform amplifying in multiple CCs, while the control information is transmitted from a given CC. Proposal 1-4: Recommend to capture the following assumption of network-controlled repeater in TR 38.867. - Option 1: The frequency band of NCR-MT should be C-link is within the same frequency band passband of NCR-FU of F-link. - As the baseline, the NCR-MT and NCR-FU are in same carrier. - Option 2: The frequency band of C-link is in different frequency band of F-link. |  |
| IIT-K |  | Fine with the proposal. |  |
| Fujitsu |  | We are generally fine with the proposal. In the legacy, RF repeater may support one or multiple passbands. For NCR, it should also be considered that the NCR-FU may have multiple passbands and across multiple carriers, so we suggest the following modification: Proposal 1-4: Recommend to capture the following assumption of network-controlled repeater in TR 38.867. - The frequency band of NCR-MT should be within the one of passband(s) of NCR-FU. As the baseline, the NCR-MT and NCR-FU are in shares a same carrier. |  |
| NEC |  | Agree with the proposal. |  |
| Ericsson |  | Support with modifications to better align it with RAN1 terminology. Furthermore, we don’t want to exclude the possibility of NCR multicarrier operation. Our proposal is then: The NCR-MT’s carrier is expected to be comprised within the set of carriers forwarded by the NCR-FU. The baseline configuration is that the MT and FU share a carrier. |  |
| Nokia |  | Support Ericsson’s proposed modification. |  |
| Qualcomm |  | We support this proposal, also agree with Fujitsu’s point (regarding multiple passbands) as well as Ericsson’s suggestion. |  |
| Sony |  | We support vivo’s formulation. We can also support Ericsson’s suggestion, but then we believe that further clarifications are needed: are all the carriers forwarded by the NCR-FU in the same frequency band? |  |
| Charter |  | We agree with Fujitsu’s and Ericsson’s point of supporting multi-carriers and we are fine with the proposal recommended by Ericsson |  |
| NTT DOCOMO |  | We support the proposal. |  |
| Apple |  | Support the proposal and Fujitsu’s modification |  |
| Intel |  | We share similar view with other companies to support multi-carrier case, and we support the proposal updated by Fujitsu. |  |
| Samsung |  | Support the proposal. |  |
| Xiaomi |  | Our understanding of in-band is that the frequency range of MT should be fully within the passband of NCR-FU. The main bullet is clear enough and we think the baseline description in the sub-bullet is not necessary. |  |
| ZTE |  | We support this proposal and fine to use the terminology in RAN1 instead of RAN4. |  |
| MediaTek |  | Support and ok with Fujitsu/Ericsson’s update. |  |
| LG |  | We are fine with the baseline that NCR-MT and NCR-FU are in same carrier. However, we don’t think it is not necessary to restrict that the frequency band of NCR-MT should be within the passband of NCR-FU. In our view, in order to reduce interference between UEs, especially between UE within the coverage of gNB and UEs within the coverage of NCR, it can be considered that NCR-MT and NCR-FU operates in different frequency resources within a same band. For example, different carriers within a band can be used for NCT-MT and NCR-FU. When multi-carriers are used, it can be assumed that one carrier is used for a UE within normal coverage served by gNB directly and the other carrier is used for a UE outside of the normal coverage therefore served by repeater. In this scenario, NCT-MT can operate in the first carrier, and NCR-FR can operate in the second carrier. In addition, in order to guarantee the throughput of UEs served by NCR-FU, it is also necessary to consider that the NCR-FU is operated in multi-carrier. |  |
| Huawei, HiSilicon |  | Support. We think the current proposal covers the points mentioned by Fujitsu and Ericsson. In our understanding, NCR-MT can be in any passband of NCR FU and the passband(s) also covers multicarrier. |  |
| Sharp |  | Support the proposal. |  |
| CATT2 | Fine with the principle of proposal |  |

    - Summary of the 2nd round

For the [Updated Proposal 1-2], following comments are highlighted:

- Regarding [Lenovo, CMCC, Ericsson, QC, Apple, Charter, Nokia]’s comments on QCL type, from moderator’s perspective, it’s fine to add ‘if applicable’ similar to legacy spec.  Meanwhile, as one generic wording, the description is updated to highlight the large-scale properties of the channel.

- Regarding [vivo, AT&T]’s comments for clarification,

  - Only if C-link and F-link are operating in the same frequency band.

- Regarding [Nokia]’s comments for clarification, this proposal is to address the assumption on RF including antenna, whether on the same band and/or carrier will be clarified in later proposal, although the same band/carrier is expected as baseline.

- Moreover, the 2nd bullet is for the assumption on the RF for backhaul and access link, which is different as the 1st bullet including the sub-bullet.

Then, according to the discussion above, the following proposal is provided:

Proposal 1-2-1: Recommend to capture the following assumption of network-controlled repeater in TR 38.867.

- As the baseline, same RF component of NCR is used for C-link and backhaul link.

  - If dedicated RF components are used (e.g., antenna) at NCR for C-link and backhaul link, respectively, same large-scale properties of the channel, i.e., channel properties included in Type-A and Type-D (if applicable), are expected to be experienced by these links.

    - Note: It is up to implementation without extra standardization effort.

- Different RF components are used at NCR for the backhaul link and access link.

For the [Updated Proposal 1-3], it seems that the main essential point is whether the NCR’s capability should be considered for scheduling. On this aspect, from moderator’s views, it’s fine to taking the NCR’s capability into account if there is one possibility in implementation to preclude the scheduling, e.g., simultaneous MT RX and FU forwarding DL. Then, the proposal is updated as:

Proposal 1-3-1: Recommend to capture the following examples of the scheduling of C-link and backhaul link by gNB in TR 38.867.

- The DL of C-link and DL of backhaul link can be performed simultaneously or in TDM way.

- The UL of C-link and UL of backhaul link can be performed in TDM way.

- Note: The scheduling of C-link and backhaul link is up to gNB’s with consideration on NCR capability.

For the [Proposal 1-4], it seems that majority share the same understanding on the frequency allocation that at least the C-link and backhaul/access should be in same frequency range, but whether in same carrier is still pending. For this issue, from moderator’s perspective, it’s possible that the CA (intra-band or inter-band) can be used to the UE served by the repeater. In this case, the NCR-MT can only be partial of carries (i.e., within the pass-band). Moreover, as commented by [Ericsson], the wording should be updated with RAN1 terminology. Then, the following proposal is listed:

Proposal 1-4-1: Recommend to capture the following assumption of network-controlled repeater in TR 38.867.

- The NCR-MT’s carrier should be within the set of carries forwarded by the NCR-FU in same frequency range.

  - As the baseline, the NCR-MT and NCR-FU are in same carrier.

- Topic-2 Beam information

  - Company view (Round-1)

For the beam information, as mentioned by majority that introduction of beam information as one side control information is beneficial for NCR operation and [HW, ZTE, vivo, Apple, LG, ETRI, MTK, Intel, DoCoMo] highlights that it is at least to control the beam at NCR side for FLS. In addition, the performance improvement after the introduction of the beam information is also further justified by the simulation results from [ZTE, vivo, Samsung, ETRI, QC] as below:

- [Source-1, ZTE] shows that the NCRs with beam information can improve the SINR performance, especially for the UE at 5%-tile, 50%-tile of CDF. Meanwhile, compared to the legacy RF repeater, the additional interference can be mitigated for the UE above 95%-tile of CDFs.

- [Source-2, vivo] shows that when the RU beam is fixed to set towards the cell edge, the SINR performance of the UEs is improved compared with the case when there is no repeater. Especially for the cell edge UE, SINR gain is 2.3 dB for the 10% UE with the worst SINR. When the RU beam is set dynamically towards the serving UE, the SINR performance of the UEs is further improved compared with the case of the fixed RU beam. Especially for the cell edge UE, SINR gain is about 6.3 dB for the 10% UE with the worst SINR.

- [Source-3, Samsung] shows that by introducing repeaters applying beamforming, 2.34 dB, 6.15 dB, and 6.53 dB gain can be achieved at 5%-tile, 50%-tile, and 95%-tile CDFs of the SINR compared to the NR system without repeaters, respectively. In addition, 2.03 dB, 5.18 dB, and 6.53 dB gains at 5%-tile, 50%-tile, and 95%-tile CDFs of the SINR can be achieved compared to the NR system with legacy repeaters, respectively.

- [Source-4, ETRI] shows that performance gain on SINR can be achieved by introducing semi-static repeater gain/power configuration, and additional performance gain can be achieved by introducing dynamic repeater gain/power configuration. More than 5 dB gain can be further achieved by using large SCI payload for beam control for large repeater-RU antenna configuration.

- [Source-5, QC] shows that Adaptive access-link (UE-side) beamforming will offer significant performance gain by providing a larger beamforming gain and reducing the interference (due to use of narrower beams), e.g., the median SINR can improve by 11dB.

[Charter Communication] also proposed to indicate the null information along with beam information.

Then, followings are proposed from FL’s perspective:

Proposal 2-1 Beam information is beneficial and recommended as the side control information for network-controlled repeater to control the behaviour of NCR-RU

- FFS: Detailed mechanism of indication.

Companies are encouraged to share your views and if there is concern, please directly propose the corresponding updates.

| Companies | Comments and Views |
| --- | --- |
| ETRI | Support. |
| Vivo | Support |
| Ericsson | Support UE-side beamforming control. |
| Nokia | In general, we are supportive, but the proposal currently reads as a conclusion to us. Perhaps it would clarify to propose: Proposal 2-1 NCR is capable of receiving side control information for network-controlled repeater to control the beamforming behaviour of NCR-RU - FFS: Detailed mechanism of indication. |
| Apple | Support. |
| CATT | OK |
| Lenovo | Support the proposal. |
| NEC | Support |
| NTT DOCOMO | We support. |
| AT&T | Support |
| Intel | Is the proposal for the beam information for FLS only? If yes, we support the proposal, but we suggest to clearly capture FLS in the proposal. |
| CMCC | Support the proposal. |
| ZTE | We support it. And the behaviour of NCR-RU refers to the FLS. Further updates can be considered if there is ambiguity. |
| Xiaomi | Support. We think both beam selection and beam indication mechanism should be further studied for beam control. |
| Sony | Support. |
| MediaTek | Support |
| Spreadtrum | Support. |
| KDDI | We support this proposal. |
| Fujitsu | Support the proposal. |
| LG | Support |
| CEWiT | Support the proposal |
| CableLabs | Support |
| Huawei, HiSilicon | The evaluation results are for FR2, and it is better to reflect this. Proposal 2-1 For FR2, beam Beam information is beneficial and recommended as the side control information for network-controlled repeater to control the behaviour of NCR-RU • FFS: Detailed mechanism of indication. |
| Samsung | Support the proposal. |
| IIT-K | Support |
| Philips | We support this proposal. |
| Qualcomm | We support this proposal. |
| Charter | We support the proposal |
| InterDigital | Support Proposal 2-1. |
| Sharp | Support |

More specifically, to enable the proper indication of beam information in the side control information, details, e.g., beam management procedure for each links, are also highlighted:

- gNB-NCR link (including control link and FLB)

As mentioned by companies that clarification on the beam assumption at NCR side for the gNB-NCR link (i.e., FLB and C-link) is needed, e.g., static beam only or configurable beam. In addition, supports on the adaptive beam is highlighted by [Ericsson, CMCC, QC] with consideration on the potential beneficial for the low-height deployment of NCR [CMCC, QC].

Regarding the determination/indication of the beam information at NCR side for FLB and C-link, most companies prefer to assume same beam for these two links, but dedicated indication via side control information is also preferred by others.

Then, following proposal is recommended from FL:

Proposal 2-2: Both fixed beam and adaptive beam can be considered at NCR side for gNB-NCR link (i.e., C-link and FLB)

Companies are encouraged to share your views and if there is concern, please directly propose the corresponding updates.

| Companies | Comments and Views |
| --- | --- |
| vivo | We agree the principle. However, the fixed beam seems no difference as the legacy repeater, which does not require spec. work. We prefer only adaptive beam is further considered. |
| Ericsson | We support both alternatives for BS-side beamforming. Additionally, the adaptive beamforming should be based on legacy UE beamforming framework. |
| Nokia | Agree with vivo that fixed beam can be considered as baseline. We would like to discuss the possibility of adapting legacy beamforming for FLS beam management. Given proposal 2-3, it may make sense to consider C-link beam management exclusively in this proposal. |
| Apple | Support both alternatives for NCR. On the other hand, the fixed beam option has no 3GPP specification impacts. For adaptive beamforming, we share the view from Ericsson to make it clearer and concrete: - Modified Proposal 2-2: Both fixed beam and adaptive beam by reusing the existing beamforming management procedures defined for UE can be considered at NCR side for gNB-NCR link (i.e., C-link and FLB) |
| CATT | Agree that fixed beam can be considered as baseline |
| Lenovo | Fine with the proposal. We think the beam used for repeater initial access can be used as a default beam, and there can also be signaling to update the beam for gNB-NCR link. For different cases, same or dedicated signal may be applicable. |
| NEC | Does the “fix” mean that the beam direction of NCR for gNB-NCR link is determined in network planning? Or does it mean the beam direction is determined by initial access with the legacy procedure and doesn’t change for the further transmission and forwarding? If it means the letter, we think adaptive indication with a long switching period or trigger commend is sufficient. |
| NTT DOCOMO | We support the proposal. We assume that NCR-MT may have capability of adaptive beam management, but we also understand the fix beam for simple NCR structure. |
| AT&T | We support reusing the existing UE beam management framework for adaptive beamforming. |
| Intel | In our view, fixed beam without standard impact can be considered as baseline. Adaptive beam reusing existing beam management can be additionally considered. |
| CMCC | Support the proposal. |
| ZTE | We are fine with this proposal and it’s to cover the different implementation of beams. Regarding the mechanism for beam management, it’s covered in the proposal below. |
| Xiaomi | Support. |
| Sony | We prefer only the adaptive link mechanism, but are open to discuss both options. |
| MediaTek | Support |
| Spreadtrum | Agree. However, how to determine (or define) the fixed beam need to be justified. Whether fixed beam is configured by network or obtained by initial beam training or determined by other way? |
| KDDI | We support this proposal. |
| Fujitsu | Fine with the proposal. Considering various deployment scenarios/manners, both fixed beam and adaptive beam should be supported for gNB-NCR link. To use which one is up to implementation. For adaptive beam, at least semi-statically setting the beam should be supported, e.g., the beam can be set by procedures like initial access of the legacy UEs. And more dynamic beam management can be further studied. |
| LG | We agree the proposal in principle. We’d like to clarify the proposal. Is it suggest that Rel-18 NCR supports both fixed beam and adaptive beam for gNB-NCR link? Or is the proposal to study and support one or both for Rel-18 NCR? |
| CEWiT | We think that a fixed beam should be enough for gNB-NCR link since the NCR is static and the gNB-NCR link will be mostly LoS. |
| CableLabs | Support the proposal. For adaptive case, as mentioned by a few companies, we can put legacy procedure as a baseline. With fixed repeater, repeater capability on beam management support could be reduced compared to normal UE. |
| Huawei, HiSilicon | We think the C-link and FLB should reuse the Rel-15/16/17 beam management procedure, and no further optimization is expected. |
| Samsung | Support adaptive beam rather than fixed beam. We also think that adaptive beam is useful in low-height deployment of NCR. Besides, adaptive beam is useful for NCR to couple with the beam adjustment from gNB. |
| IIT-K | Support |
| Philips | We share similar views with Sony and Samsung and prefer to support adaptive rather than fixed beam. |
| Qualcomm | We support this proposal. |
| Charter | We support the proposal but prefer adaptive beam to be considered for gNB-NCR link to allow for beam failure recovery in case of blockages etc especially in FR2. We believe legacy beamforming/failure recovery procedures can be used for gNB-NCR link |
| InterDigital | Support Proposal 2-2. We think fixed and adaptive beam configuration should be both supported for gNB-NCR link, where the adaptive beam configuration can be prioritized. |
| Sharp | Support in general. Since only fixed NCR is considered, fixed beam can be used as baseline. Of course, the gNB can change the beam sweeping pattern and beam width, and adaptive beam is necessary in some cases. |

Proposal 2-3: Following options can be considered to determine the beam at NCR side for FLB:

- Option-1: The beam information is indicated via the side control information from gNB.

- Option-2: The same beam as the C-link is assumed.

Companies are encouraged to share your views and if there is concern, please directly propose the corresponding updates.

| Companies | Comments and Views |
| --- | --- |
| vivo | We support in principle. Our preference is option 1, since beam information for C-link may not available for all the time duration, the beam indication for C-link is available only for the time resource of MT scheduling. |
| Ericsson | Disagree. We think Option-2 can already now be selected. Option-1 is not feasible without a new beam management framework and the benefit of that is very limited and should be considered beyond the scope of the SI. |
| Nokia | We have a similar view with Ericsson that beam management for C-link and FLB should not be performed independently. |
| Apple | Again, our interpretation on ‘C-link’ is a logic tunnel to carry the SCI. This proposal may refer to the reception of backhaul link, instead of ‘C-link’. With this assumption, Opt.2 is feasible at least for FR2 in relevant designs i.e., reciprocity, such as ‘The same beam as the C-link reception of backhaul link is assumed for FLB’ |
| CATT | Support the proposal. At this time no need to exclude option 1. |
| Lenovo | Fine with the proposal. We support both and prefer different options to be applied at different circumstances. |
| NEC | Maybe we need to define the signaling layer of the side control information first. Due to the stable NCR, the beam at NCR side for FLB doesn’t need to update frequently. Proposal 2-3: Following options can be considered to determine the beam at NCR side for FLB: - Option-1: The beam information is indicated via the side control information from gNB. Option-2: The same beam as the C-link is assumed. |
| NTT DOCOMO | We support the proposal. We think Option-2 seems straight forward especially if we consider the assumption in Proposal 1-2 (sharing/co-located components or co-components). |
| AT&T | Option-2 is reasonable for the shared HW scenario, but Option-1 may be additionally supported |
| Intel | We share similar view with E/// and NOKIA that option -2 is sufficient. New procedure/signalling for beam management dedicated for FLB link requires great standard effort without clear benefit. |
| CMCC | Fine with the proposal. For the beam management of BH link, the legacy BM schemes could be reused. But the beam information could be carried through side control information and under the control of gNB. And whether the FLB always share the same beam of C-link needs more discussion. |
| ZTE | We support this proposal. Regarding whether prioritization is needed or not can be further discussed. |
| Xiaomi | We support option 2 as baseline. For option 1, if the fixed beam is considered at NCR side for gNB-NCR link, it seems the side control information is not needed. |
| Sony | We are open to discuss both options. |
| MediaTek | Support |
| Spreadtrum | We prefer option 1, but option 2 is not precluded. If the beam information is provided, the NCR should apply the indicated beam, but if the beam information is not indicated, the C-link beam should be used. |
| KDDI | We support this proposal, and think that option 2 could be baseline. |
| LG | We support to discuss both options. We think further clarification is necessary for Option-2. The beam used by the MT may not be configured to one, but may be multiple. It seems further study is needed to determine which beam among the beams of MT is applied for RU. |
| CEWiT | Support the proposal |
| CableLabs | We share similar views with a few companies that Option-2 should be the baseline at least simultaneous C-link and backhaul reception occur. In case of TDM, it’s not clear why same or different beams are important. |
| Huawei, HiSilicon | We support Option 2. We suggest to discuss this after Proposal 1-2. |
| Samsung | The formulation of this proposal is a little bit confusing. It is better to clarify whether this proposal is for the definition of source beam for NCR-RU beam indication (for gNB-side/FLB). If so, option-2 is equivalent to using the same spatial filter for C-link for the transmission/reception of NCR-RU; while option-1 means to use logic beam ID for the beam indication (which is independent to the beam indication of C-link). In that sense, option-2 is preferred. |
| IIT-K | We have similar views as Ericsson and Samsung and prefer Option-2 |
| Philips | We agree with AT&T. |
| Qualcomm | We support this proposal and prefer Option-1. We would like to also echo CMCC’s comment that the legacy beam management procedure of NCR-MT can be reused to find/track the BH beam(s). However, beam indication can be separate. Let’s assume two BH beams could be identified by NCR-MT. We would like to support the flexibility of indicating which beam can be used for the forwarding operation. Even in the legacy, the TCI state can change between PDCCH and PDSCH, and it is natural to support such a feature for NCR too – e.g., side control sent to NCR-MT using beam1, indicating an upcoming forwarding operation using beam2. |
| Charter | We prefer Option-2 |
| InterDigital | Support Proposal 2-3. While C-link beam may be used to determine the beam at NCR side for FLB, the beam information may be different for the C-link and FLB (e.g., due to different numerologies). Therefore, gNB should be able to indicate beam information independent of the C-link beam. |
| Sharp | Support in principle, with slight preference on Option 2. |

- NCR-UE link (i.e., FLS)

The benefits to enable the beam information for the FLS have been justified by majority companies. Regarding the detailed mechanism on the determination and indication of this information, as mentioned by companies that solutions to indicate the beam to forward the cell-specific signal and UE-specified signal should be further discussed. In addition, at least, the dynamic indication is also highlighted companies for the FLS to forward the UE-specific signal [ZTE, vivo, QC, Apple].

Then, from FL’s side, following is proposed:

Proposal 2-4: At least the dynamic indication can be considered for the beam of FLS at NCR side

Companies are encouraged to share your views and if there is concern, please directly propose the corresponding updates.

| Companies | Comments and Views |
| --- | --- |
| vivo | We provide simulation for both semi-static and dynamic beam indication, and shows gain for both cases. In our understanding, semi-static beam is suitable to serve a coverage hole, where beam direction does not always change. We provide the following modification… - Semi-static and dynamic indication can be considered for the beam of FLS at NCR side |
| Ericsson | Partly agree. We think that there are strong reasons why both semi-statical and dynamic beam configuration should be implemented. In particular, both repeater resource efficiency and energy efficiency may benefit from more independent operation as enabled by semi-static configuration. |
| Nokia | Agree with Ericsson and vivo that semi-static and dynamic beam management should be considered. |
| Apple | We also support both semi-static and dynamic indication for beamforming information on access link. Main motivation is, similar as slot format indication, to minimize the signalling overhead. |
| CATT | Agree the wording from vivo. |
| Lenovo | In principle, we are fine with the proposal. However, we think semi-static beam indication for FLS could be beneficial in some deployment scenarios. Therefore, we prefer to explicitly mentioning both semi-static and dynamic beam indication to be considered. |
| NEC | Support |
| NTT DOCOMO | We prefer to consider both semi-static and dynamic indications for cell-specific signal and UE-specified signal, since NCR should amplify-and-forward both signals. |
| AT&T | Agree with NTT DOCOMO |
| Intel | We share similar view with other companies that both semi-static and dynamic indication can be consider. |
| CMCC | We are fine to support both semi-static and dynamic indications for the beam of FLS at NCR side. |
| ZTE | We are fine to this proposal and in general open to the semi-static indication, especially with consideration on the forwarding for different channel. |
| Xiaomi | We support that both semi-static and dynamic beam indication should be considered for NCR. |
| Sony | We support the proposal with the understanding that other indication mechanism, such as semi-static indication, can also be discussed. |
| MediaTek | Both semi-static and dynamic indication can be considered. |
| Spreadtrum | Agree with vivo, both semi-static and dynamic beam management should be considered. |
| KDDI | As indicated by other companies, both semi-static and dynamic beam indication for FLS at NCR side should be considered. |
| Fujitsu | Dynamic or semi-static indication seems more like a signaling. The similar proposal is discussed in 9.8.2, as proposal 2-1-1. |
| LG | We think semi-static beam indication may be useful, but ok to the Proposal. |
| CEWiT | Support the proposal |
| CableLabs | Support. Also okay to include semi-static indication as well. |
| Huawei, HiSilicon | We suggest address the basic design principle first, then details can be carried under the assumptions on the design principle. For NCR-UE link beam, the number of SSB/CSI-RS beams impacts on the gNB overhead. It is better that the gNB can have the flexibility to configure the number of beams used for NCR-UE link. We propose the revise as: Proposal 2-4: The number of SSB/CSI-RS access beams that can be used by the NCR are indicated by the gNB - At least FFS the dynamic / semi-static indication can be considered for the beam of FLS at NCR side |
| Samsung | Support the proposal. We also think that semi-static beam indication needs to be considered. |
| IIT-K | We are fine to support both dynamic and semi-static beam indication |
| Philips | We agree that semi-static and dynamic indications can both be considered. |
| Qualcomm | We agree that both dynamic and semi-static can be considered. |
| Charter | We also like to support both dynamic and semi-static beam indication. We think semi-static updates for cell-specific signal and dynamic updates for UE-specific signals will be beneficial |
| InterDigital | We agree with other companies on that both semi-static and dynamic beam indication should be considered for the FLS beam at NCR side. |
| Sharp | We agree that both dynamic and semi-static indications should be considered. |

- Others

Additional aspects related to the determination and indication of beam information are also proposed by companies. For example:

- Regarding how to represent the beam information, potential solutions, e.g., introduce a new beam ID, reference signal ID, enhanced TCI states or time domain resource identifier, are proposed by companies [HW, ZTE, Samsung, vivo, Spreadtrum, LG, Fujitsu, Apple]. In these way, mapping between notation and NCR beam should be indicated by gNB to NCR. Also, different ways to map the SSB to NCR’s beam with potential indication are also mentioned [CMCC, ZTE, CableLabs, HW].

For this point, it’s valid issue from FL’s perspective and we can initialize the discussion by taking the above options as starting point.

- Regarding the NCR capability, e.g., the supported number of beam [CATT, HW], boresight of antenna array and beam direction of NCR’s antenna for FLS link [ETRI] or antenna array configuration (e.g., using <N1, N2, O1, O2> structure) to the gNB [ETRI]. Meanwhile, the corresponding report of beam related capability is also proposed.

For this point, from FL’s perspective, the decision of NCR capability including the details on report can be handled in the normative phase later.

- To avoid the auto-excitation of NCR, companies [vivo, CMCC] propose to study the necessity to introduce some methods, e.g., beam restriction, to alleviate the potential interference.

For this point, from FL’s perspective, this issue is addressed by implementation for legacy RF repeater and similar way is also applicable for NCR. But companies are also encouraged to check if additional solutions are needed.

- Some companies also mentions that the study on the beam management to optimize the NCR beam for either FLB or FLS is needed [ETRI, Intel] with consideration on the signaling overhead, latency and complexity. From FL’s perspective, companies are encouraged to check if additional solution should be defined.

Companies are encouraged to share your views and if there is concern, please directly propose the corresponding updates.

| Companies | Comments and Views |
| --- | --- |
| Ericsson | We think that some assumptions about beam reciprocity/correspondence may need to be discussed for DL/UL beam matching, e.g., at initial access. Additionally, and similar to Bullet 1 above, beam arrangement should be included in the beam signalling framework for reduced broadcast signal overhead and increased dedicated signal throughput. |
| CMCC | For issue #3, not only the auto-excitation should be deal with, but also the self-interference from the transmitting part to the receiving part of the NCR should be considered. Since the RU part the NCR will receive the signals from AC/BH link, amplify and forward them to the BH/AC links. The process delay could be around tens of nano seconds. The receiving beam could be interfered by the transmission beam due to the signal reflections from the surrounding environments. Beam restriction could be realized through beam management. But the gNB should have the knowledge that the self-interference happens between some transmission and reception directions first. Then it can solve this issue. |
| IIT-K | We share similar views with Ericsson about beam reciprocity/correspondence to be discussed. Additionally, how a gNB determines the identities of UEs connected through NCR should also be discussed. |
| Charter | We support 1 and agree that this discussion should be the starting point. We also support 2 and think it should be decided in this phase as NCR capability reported to gNB will enable the gNB to set up the beams properly We do not support 3 and agree with FL that it can be addressed as implementation We believe 4 should be part of 1 |
| InterDigital | Beam association between NCR FLB beams and NCR FLS beams can be included in beam signalling configurations, where the association can result in lower control signalling overhead and latency. |

    - Summary of the 1st round

According to the further discussion, the following agreements have been achieved for Topic-2.

Agreement

At least for FR2, beam information is beneficial and recommended as the side control information for network-controlled repeater to control the behaviour of NCR at least for access link

- FFS: Detailed mechanism of indication.

- Note: There are no supporting evaluation results on FR1 at this point to reach similar conclusion

Agreement

Both fixed beam and adaptive beam can be considered at NCR for both C-link and backhaul-link.

- FFS: the mechanism for indication and determination of beam.

- Note: Fixed beam refers to the case that beam at NCR for both C-link and backhaul-link cannot be changed.

Agreement

Both the dynamic indication and semi-static indication can be considered for the beam of access link for NCR-FU.

- FFS: the details of each indication

- FFS: the maximum number of beams configured for NCR-FU access link.

  - Company view (Round-2)

For the [Proposal 2-3], the proposal is general supported with comments to take the Option-2 as the default method. However, it's still possible to further consider additional solution as commented by other. Then, following updated version is proposed:

[Updated Proposal 2-3]: The same beam as C-link is assumed for beam at NCR for backhaul link.

- FFS: additional indication from gNB to determine the beam at NCR side for backhaul link if adaptive beam is supported by NCR.

Companies are encouraged to share your views and if there is concern, please directly propose the corresponding updates.

| Companies |  | Comments and Views |  |
| --- | --- | --- | --- |
| Lenovo |  | Support. |  |
| CMCC |  | Our view is that the beam of C-link could be reused for the NCR BH link but the BH link and the C-link do not need to use the same beam all the time. Since the beam management of MT or the C-link could reuse the legacy procedure, the beam of C-link could be very flexible depending gNB’s scheduling, based on DCI or MAC CE based indication or RRC configurations. As the C-link is the communication between gNB and MT/UE, it could be very flexible. But the BH may not need to change in a such dynamic way and always follows the beams of the C-link. Our preference is to update the main bullet as, The same beam as C-link is can be assumed for beam at NCR for backhaul link. For the 2nd bullet, we support the additional indication from gNB to determine the beam at NCR for BH link. And the adaptive beams for both C-link and BH links are supported according to the agreed proposal 2-2 v2, if my understanding is right. |  |
| CEWiT |  | Support the proposal |  |
| vivo |  | F-link beam and C-link beam may not always the same, since sometimes there is no C-link beam information due to lack of C-link transmission/reception. [Updated Proposal 2-3]: The same beam as C-link is assumed for The following options are considered to determine the beam at NCR for backhaul link. - The beam at NCR is determined based on beam information of NCR-MT - FFS: additional indication from gNB to determine the beam at NCR side for backhaul link if adaptive beam is supported by NCR. |  |
| IIT-K |  | Support the proposal. |  |
| Fujitsu |  | Support. |  |
| NEC |  | Support this proposal. |  |
| Ericsson |  | Support the intention. Beams are not defined in RAN1, instead RAN1 defines TCI states for defining which beam combinations to use for a certain transmission. For that reason, we propose to reformulate according to: The same TCI states as defined for the MT’s control link can be assumed to be applied to the FU’s backhaul link. We don’t think adaptive beamforming is the only requirement for providing additional information for beam determination. Another requirement is separate MT and FU antennas on the BS side. That modifies the FFS to: • FFS: additional indication from gNB to determine the beam at NCR side for backhaul link if adaptive beamforming is supported by NCR and MT and FU use separate antennas. |  |
| AT&T |  | Support the proposal from vivo and the additional FFS from Ericsson |  |
| Nokia |  | Agree with proposal, but also can support modification to use TCI states for clarity. To our understanding, FFS point can be removed if the assumption is that TCI states for NCR-MT are re-used for NCR-FU. |  |
| Qualcomm |  | Partially support. We support the suggested change by CMCC. We also support the sub-bullet as an optional feature. Our proposed modification: The same beam as C-link is can be assumed for beam at NCR for backhaul link. FFS: support additional optional indication from gNB to determine the beam at NCR side for backhaul link if adaptive beam is supported by NCR. |  |
| Sony |  | We support the proposal as formulated by Ericsson. |  |
| Charter |  | We support the proposal |  |
| NTT DOCOMO |  | We support the proposal. |  |
| Apple |  | As commented on the previous ‘Updated Proposal 1-2’, it is desirable to use ‘TCI-state’ since ‘beam’ is invisible in L1 specification. Therefore, we support the following proposal from Ericsson: - ‘The same TCI states as defined for the MT’s control link can be assumed to be applied to the FU’s backhaul link.’ On the sub-bullet, our understanding is that it intends to support adaptive beam indication for backhaul link, instead of C-link, as PDSCH TCI-state adaption for legacy UE. |  |
| KDDI |  | We support the proposal, but we can also agree with the modification from Ericsson for further clarification. |  |
| Intel |  | We support the main bullet updated by CMCC. Regarding FFS point, we’re fine with additional indication for backhaul link, but we think the indicated beam should be from the set of beams configured for NCR-MT, otherwise, a separate beam management procedure for the backhaul link is needed, which requires additional standard effort and complexity without clear benefit. |  |
| Samsung |  | Support the proposal in general. |  |
| Xiaomi |  | As legacy beam management mechanism is assumed for MT, there might be different TCIs configured for different channels in the C-link, and even multiple TCIs for one specific channel in case of multi-TRP transmission. It is not clear which beam is assumed for backhaul link. The modification from vivo and Ericsson are fine to us. |  |
| ZTE |  | We are fine with the updates from CMCC. The beam can be same or different compared between C-link and backhaul link. But as baseline, same mechanism is preferred. |  |
| MediaTek |  | We prefer vivo’s proposal. |  |
| LG |  | We support the intention of the proposal as a baseline, but have similar view with CMCC. As we mentioned Round-1 previously, if adaptive beam is supported for C-link, configured beams for C-link will not always be only one, may be configured with multiple beams. So, we need to study about which beam(s) of the backhaul link would be determined and used among the configured beam(s) from the C-link. We suggest to update the proposal as follow. [Updated Proposal 2-3]: The same beam as C-link is assumed for beam at NCR for backhaul link. • FFS: how to determine beam of C-link assumed for beam at NCR FFS: additional indication from gNB to determine the beam at NCR side for backhaul link if adaptive beam is supported by NCR. |  |
| Huawei, HiSilicon |  | We think the “additional” in the FFS should be removed. No new beam indication mechanism should be introduced for the C-link, neither for the NCR FU backhaul link. We may revise the proposal as: [Updated Proposal 2-3]: The same beam as C-link is assumed for beam at NCR for backhaul link. - Legacy mechanism can be reused. - FFS: additional indication from gNB to determine the beam at NCR side for backhaul link if adaptive beam is supported by NCR. |  |
| Sharp |  | Support |  |
| CATT2 | We share similar view with CMCC and others that same or different beam can be used for C-link and backhaul link. |  |

Meanwhile, as commented by [Ericsson, IIT-K], the assumption on the beam correspondence for DL/UL should be clarified at NCR side. From moderator’s perspective, it’s reasonable to discuss it by following proposal:

Proposal 2-5: The beam correspondence/reciprocity is assumed for the DL/UL operation of NCR-MT and NCR-FU.

Companies are encouraged to share your views and if there is concern, please directly propose the corresponding updates.

| Companies |  | Comments and Views |  |
| --- | --- | --- | --- |
| Lenovo |  | Regarding beam correspondence/reciprocity, our view is that there is beam correspondence for the DL/UL of the link between gNB and NCR-MT, for the DL/UL of the link between gNB and NCR-FU. And whether there is beam correspondence for DL(UL) of link between gNB and NCR-MT and UL(DL) of link between gNB and NCR-FU should be further discussed. |  |
| CMCC |  | Lenovo raise a good point. The proposal could be updated as Proposal 2-5: The beam correspondence/reciprocity is assumed for the DL/UL operation of NCR-MT and NCR-FU. - the DL/UL of the link between gNB and NCR-MT - the DL/UL of the link between gNB and NCR-FU |  |
| CEWiT |  | Support the proposal |  |
| vivo |  | This assumption is applied for FR2, let us clarify this point. |  |
| IIT-K |  | Support the proposal. |  |
| Fujitsu |  | Support the proposal in principle. |  |
| NEC |  | It’s OK for us. |  |
| Ericsson |  | Support in general but it may be sensible to limit the proposal to the FU’s access link since the MT’s beam management is assumed to mimic a UE’s beam management and there no such assumptions are made. The FU’s backhaul link is further expected to rely on the MT’s beam management. |  |
| AT&T |  | Ok with this as an assumption, but support the comments from Lenovo/CMCC |  |
| Nokia |  | The proposal seems to be overly restrictive since a beam correspondence/reciprocity assumption is not necessary for a standard Uu interface. The purpose of the proposal seems to be to avoid separate beam identifiers for NCR access link for DL and UL, and for that reason we would support Ericsson’s proposal to limit beam correspondence/reciprocity assumption to NCR-FU access link. |  |
| Qualcomm |  | Agree with Ericsson. This should be limited to the NCR-FU’s TX/RX on the access link. |  |
| Sony |  | We can support CMCC’s suggestion and Ericsson’s suggestion. |  |
| Charter |  | Agree with Ericsson’s proposal |  |
| NTT DOCOMO |  | We support the proposal. |  |
| Apple |  | It may be necessary to clarify the proposal applicability, i.e., does it refer to backhaul link or access link or both? We support to apply both considering the FR2 use cases. |  |
| Intel |  | Agree with E/// that the limitation is applied for access link. |  |
| Samsung |  | Support the proposal and also fine with the update provided by E///. |  |
| Xiaomi |  | Support the intention of the proposal. |  |
| ZTE |  | Support and fine with the clarification. We can focus on the access link. |  |
| MediaTek |  | We would like to clarify “beam correspondence” mentioned here. 38.306: beamCorrespondenceWithoutUL-BeamSweeping Indicates how UE supports FR2 beam correspondence as specified in TS 38.101-2 [3], clause 6.6. The UE that fulfils the beam correspondence requirement without the uplink beam sweeping (as specified in TS 38.101-2 [3], clause 6.6) shall set the field to supported. The UE that fulfils the beam correspondence requirement with the uplink beam sweeping (as specified in TS 38.101-2 [3], clause 6.6) shall not report this field. Does this proposal aim to mandate “beamCorrespondenceWithoutUL-BeamSweeping” for access link of NCR-FU? |  |
| LG |  | We support the proposal for NCR-MT. For NCR-FU, it seems necessary to clarify whether the proposal is applied to either backhaul link or access link or both. |  |
| Huawei, HiSilicon |  | The definition of beam reciprocity should be clarified. For example in 38.101-2, the beam correspondence is defined as the ability of the UE to select a suitable beam for UL transmission based on DL measurements with or without relying on UL beam sweeping. The same definition can be adopted for NCR-MT and NCR-FU backhaul. For NCR-FU access, we can also assume beam correspondence, and the tolerance can be up to implementation. In NR only beam correspondence of UE is defined. We prefer to delete the “reciprocity”: Proposal 2-5: The beam correspondence/reciprocity is assumed for the DL/UL operation of NCR-MT and NCR-FU. |  |
| Sharp |  | Support |  |
| CATT2 | OK with the proposal |  |

    - Summary of the 2nd round

For the [Updated Proposal 2-3], the essential part is whether to mandate the same beam for C-link and backhaul link always. Based on the inputs, it’s clear that even same mechanism for beam indication and determination is used for these two links, there is still possibility to take different beam for C-link and backhaul link at NCR, especially when these two links are in TDMed manner or with separately antenna at NCR side for MT and NCR-FU. Then, the following updated proposal is proposed:

[Proposal 2-3-1]: The same beam as C-link can be assumed for beam at NCR for backhaul link.

- FFS: additional indication from gNB to determine the beam at NCR side for backhaul link if adaptive beamforming is supported by NCR

Note: Legacy mechanism is the baseline for beam indication and determination.

For the [Proposal 2-5], the views are aligned within majority with further clarification on the applicable links of this feature. And from implementation’s perspective, we only need to clarify the assumption for NCR-FU and MT and the legacy assumption for the involved gNB and UE is still valid. Then, from moderator’s perspective, the following updated version is listed:

Proposal 2-5-1: The beam correspondence is assumed for:

- the DL/UL of the access link at NCR-FU

- the DL/UL of the backhaul link at NCR-FU

- FFS: the DL/UL of the C-link at NCR-MT

- Topic-3 ON-OFF information

  - Company view (Round-1)

For the ON-OFF information, as mentioned by majority that introduction of ON-OFF information for NCR is beneficial. In addition, the performance improvement after the introduction of the ON-OFF information is also further justified by the simulation results from [ZTE, vivo, Samsung, QC] as listed below:

- [Source-1, ZTE] shows that NCRs with ON-OFF information can mitigate the interference for high SINR UEs while maintain the performance of low SINR UEs, and also ON-OFF information can provide efficient interference management in FR1.

- [Source-2, vivo] shows that about 9.8dB gain can be achieved for the 10% tile UEs on the SINR performance after introducing ON-OFF indication.

- [Source-3, Samsung] shows that additional gain is observed for the repeater by both applying beamforming and on/off management compare to the NR system with the repeater only applying beamforming.

- [Source-4, QC] shows that about 2 dB gains on median SINR can be achieved by introducing dynamic on-off information.

Then, followings are proposed from FL’s perspective:

Proposal 3-1 ON-OFF information is beneficial and recommended as the side control information for network-controlled repeater to control the behaviour of NCR-RU.

- FFS: Detailed mechanism of indication.

Companies are encouraged to share your views and if there is concern, please directly propose the corresponding updates.

| Companies | Comments and Views |
| --- | --- |
| Vivo | Support the proposal |
| Ericsson | Support |
| Nokia | Support. |
| Apple | Support |
| CATT | OK |
| Lenovo | Support the proposal. |
| NEC | Support |
| NTT DOCOMO | We support. |
| AT&T | Support |
| Intel | Support |
| CMCC | support |
| ZTE | Support |
| Sony | Support |
| Spreadtrum | Support. |
| KDDI | We support this proposal. |
| Fujitsu | Support the proposal. |
| LG | Support the proposal |
| CEWiT | Support the proposal |
| CableLabs | Support |
| Huawei, HiSilicon | NCR RU OFF at broadcast channel forwarding time may lead to coverage loss / hole, and this should be avoided. In addition, the signalling part is discussed in proposal 3-2, it is better not start to make recommendations already in this proposal. Proposal 3-1 ON-OFF information is beneficial and recommended as the side control information for network-controlled repeater to control the behaviour of NCR-RU. - FFS: Detailed mechanism of indication. Note: the coverage of broadcast channels should not be impacted by NCR RU OFF. |
| Samsung | Support the proposal. |
| IIT-K | Support |
| Philips | We support this proposal. |
| Qualcomm | Support. |
| Charter | Support the proposal |
| InterDigital | Support Proposal 3-1 |
| Sharp | Support |

More specifically, to enable the indication of this information and how to perform the on/off by NCR-RU/MT, detailed solutions in either explicit (e.g., directly indication of ON/OFF state by dynamic [LG, QC, CMCC] or pattern-based signal [Sony, ZTE, vivo]) or implicit way (e.g., coupled with PC information or time domain resource allocation for NCR) is proposed.

Meanwhile, introducing the additional rule/criteria [Panasonics, Charter Communication] are also mentioned, e.g., taking the OFF as default state or the coverage of broadcast channels should not be impacted by NCR RU OFF [HW].  Potential extension of DRX procedure to control the on-off state of NCR-RU is also mentioned by [vivo, Spreadtrum, Apple, Ericsson, Fujitsu].

From FL’s perspective, we can start with the discussion by taking the above solution as potential options with following proposal:

Proposal 3-2: The ON-OFF information can be indicated to NCR by gNB with following options:

- Option 1: Explicit indication with the dynamic signalling or semi-static signal

- Option 2: Explicit indication with the on-off pattern

- Option 2: Implicit indication with the signalling of beam or PC information

- Option 4: Implicit indication with the extension of DRX procedure to NCR RU

Companies are encouraged to share your views and if there is concern, please directly propose the corresponding updates.

| Companies | Comments and Views |
| --- | --- |
| vivo | We prefer to make several parallel points for further discussion, instead of the options, e.g., Periodic or aperiodic on-duration, or Dynamic or semi-static signaling |
| Ericsson | Support in general. We assume the proposal only concerns the repeater forwarding/RU function. Furthermore, we think that a combination of the above proposals can be envisioned, e.g., both explicit and implicit signaling. |
| Nokia | Similar view to Ericsson that explicit vs implicit on/off should be considered first with additional details FFS. |
| Apple | We agree with vivo to formulate the discussion from different perspectives. One formulation example is as follows: - Periodic or Aperiodic or both? - Implicit or explicit way or both? - If explicit, semi-static or dynamic or both? |
| CATT | Support the proposal |
| Lenovo | Support the proposal. |
| NEC | We prefer Option-1. Some typos is suggested - Option 1: Explicit indication with the dynamic signalling or semi-static signaling - Option 2: Explicit indication with the on-off pattern - Option 23: Implicit indication with the signalling of beam or PC information - Option 4: Implicit indication with the extension of DRX procedure to NCR RU |
| NTT DOCOMO | We support. |
| AT&T | Support Option 2 and 3 which aligns with existing support for resource coordination in IAB (i.e. Hard and Not Available resource indications for the access link) |
| Intel | We’d like to clarify, is proposal 3-2 only for RU, or both RU and MT? If the intention is to also include MT, in our view, we’d better fist discuss whether on/off operation for MT is supported, whether existing mechanism such as DRX as legacy UE or new mechanism is needed. Assuming here we discuss on/off for RU, we think it would be sufficient to first discuss from perspective of explicit and implicit, dynamic and semi-static at this stage. |
| CMCC | Fine with the proposal. And option 1 is preferred. Our thinking is to indicate or allocate the time domain resources which will be used for NCR for data forwarding. For the resources indicated for the data forwarding, the corresponding time or period could be considered as “on”. The resources that are not indicated could be considered as “off”. We think this could be considered as one kind of explicit indications. |
| ZTE | We are fine with this proposal. Regarding the solution, anyway, we need to organize it in certain way and starting with the detailed options based on companies inputs are reasonable. Also others including potential combination are not precluded. |
| Rakuten | Support |
| Sony | We prefer to consider both options 1 and option 2. Option 2, on-off patterns, can be configured semi-statically. Option 1, dynamic signaling, can be use to override specific instances. |
| Spreadtrum | Support the proposal. |
| KDDI | We are fine with this proposal. Also, other combinations could be considered. |
| Fujitsu | We are fine with the intention of the proposal. But the proposal is not clear. There seems cross between options. For example, option 1 is constructed from a perspective which is different with other options. |
| LG | We think that there is not enough discussion to organize the options for the ON-OFF indication method. In addition to the listed implicit indication methods, it also can be considered to implicitly determine ON-OFF using D/U information. For example, if D/U indication for F-link is supported for NCR, the resource for which D/U information is not indicated may be determined to be OFF. In addition, it may be considered that both the explicit indication and the implicit indication are applied to determine ON-OFF. For example, if dynamic D/U indication is not supported, semi-static flexible resource can be implicitly determined to be OFF, and additionally, ON-OFF information may be dynamically indicated to a semi-static DL/UL resource. Therefore, like Vivo said, rather than making options now, it would be nice to be able to organize the points that need discussion first. |
| CEWiT | Support the proposal |
| CableLabs | We prefer a bit generalized options (for example, Option 1 and 2 (or 3, 4) can be used jointly). Whether to use on off pattern, DRX, power, for us, detailed signaling mechanisms, and we propose to include SFI as a mechanism where reserved resources can be used as an indication to off. |
| Huawei, HiSilicon | Option 1 and option 2 seems to be the same, and it is unnecessary since NCR RU ON indication should always accompany beam or PC information. For Option 4, we are wondering if DRX impacts the forwarding of broadcast channels (such as SSB, PRACH, …). We think that the NCR RU OFF should not reduce the network coverage or leads to coverage hole. |
| Samsung | Share similar view apple and the combination different aspects need to be considered as well. |
| IIT-K | Support |
| Philips | We share LG’s view. |
| Qualcomm | Support. |
| Charter | We support 1 and 3 (listed as 2). We don’t understand the difference between options 1 and 2 and think option 2 should be part of option 1. We also want to emphasize that some criteria of interference determination/threshold need to be decided to use OFF (either whole receiver or just particular beams). |
| InterDigital | Support Proposal 3-2. It is a good starting point. |
| Sharp | We share LG’s views. More discussion is needed to list the options. |

    - Summary of the 1st round

According to the further discussion, the following agreement have been achieved for Topic-3:

Agreement

ON-OFF information is beneficial and recommended for network-controlled repeater to control the behaviour of NCR-FU.

- FFS: Detailed mechanism of ON-OFF indication and determination.

- FFS: explicit indication or implicit indication of ON-OFF information

  - Company view (Round-2)

For the [Proposal 3-2], companies express the views that potential solution can be re-organized in other structured way with more candidates. From moderator’s perspective, it’s fine to collect the potential consideration in this stage and others are not precluded. Moreover, the difference between Option-1 and Option-2 is that in Option-1, the information is indicated individually but in Option-2, a pattern based way is considered to present the on-off state in the defined manner.

Then, the updated version is proposed:

Updated Proposal 3-2: The ON-OFF information can be indicated to NCR by gNB with following options:

- Option 1: Explicit indication with the single or multiple on-off state

- Option 2: Explicit indication with the on-off pattern

- Option 3: Implicit indication with the signalling of beam or PC information

- Option 4: Implicit indication with the extension of DRX procedure to NCR RU

- Other solutions (e.g., potential combination of explicit and implication solution) are not precluded.

Companies are encouraged to share your views and if there is concern, please directly propose the corresponding updates.

| Companies | Comments and Views |
| --- | --- |
| Lenovo | Support. |
| CMCC | General fine with the proposal. Could the FL clarify that for the option-1, the information is indicated individually. Does it mean that the on-off state is indicated per a time domain unit individually or per some other dimension ? |
| CEWiT | Support the proposal |
| vivo | Option 1 and option 2 seem the same. We prefer to keep option 2. The implicit indication seems not well discussed in the contributions, we prefer to FFS the implicit indication instead of “supporting” Updated Proposal 3-2: The ON-OFF information can be indicated to NCR by gNB with following options: - Option 1: Explicit indication with the single or multiple on-off state - Option 2: Explicit indication with the on-off pattern - FFS: Implicit indication with the signalling of beam or PC information - Option 4: Implicit indication with the extension of DRX procedure to NCR RU - Other solutions (e.g., potential combination of explicit and implication solution) are not precluded. |
| IIT-K | Support the proposal. |
| Fujitsu | Support |
| NEC | Agree in general. And we think it’s better to merge the option 1 and option 2 in a sub-bullet for now. Or the detailed methods can be listed as examples for explicit and implicit indication respectively. |
| Ericsson | Support |
| Nokia | We have a similar view as vivo with respect to option 1 and option 2. In our view Option 1 and 2 are differentiated only by specific signaling details which are inappropriate to discuss at this stage of the study. Dynamic/semi-static indication, time resolution (e.g., symbol-/slot-level), applicable channels and signals are all more important and primary issues to consider over the details of how the indication is provided. We prefer to combine options 1 and 2 with FFS for details of explicit indication. |
| Qualcomm | Support. It may make sense to clarify that RAN1 should down-select one or multiple of these options. (similar to the note added to Updated Proposal 6-2) |
| Sony | Support. All the various options can be studied. |
| Charter | We support the proposal but agree with Nokia and Vivo that option 1 and 2 are same |
| NTT DOCOMO | We support the proposal. |
| Apple | Support moderator’s proposal in general. Some clarification maybe needed e.g., the difference between Opt.2 vs. Opt.4 as DRX function is explicitly configured and is a sort of on-off pattern. |
| KDDI | Basically, we support the proposal, but as several other companies have pointed out, option 1 and 2 represent specific signaling differences, so they can be treated as one option at this stage. |
| Intel | For option 1&2, we share same view with vivo and Nokia to combine these two options. For option 3, in our view, there is no need to restrict the implicit indication to only beam or PC information, there are also other ways to derive on/off. For example, for TDD, if it indicates UL symbol, it implies DL is off. For another example, flexible symbol can also indicate off. Therefore, we suggestion to revise option 3 as: Implicit indication with other side control information |
| Samsung | Support the proposal. |
| ZTE | Support the proposal. As mentioned by the summary above, Option-1 and Option-2 is different, e.g., individually or pattern based. More specifically, in Option-1, the on-off of NCR-FU will be changed per indication or per indication with multiples. But in Option-2, the on-off will be repeated in pattern based. Moreover, we also need to clearly list the solution via implicit indication, which have already mentioned by companies with details as summarized by moderator. |
| MediaTek | We only need to list two options (explicit and implicit) and leave details for explicit/implicit indication FFS. Patterns, on-off states are not well defined yet in current proposal. |
| LG | We support the proposal in general. In our view, another considerable example for implicit indication of ON-OFF is that to determine flexible time resource or time resource without DL/UL indication as OFF resource. So, we would link to include DL/UL information to the Option 3 as follow. Option 3: Implicit indication with the signalling of beam, DL/UL or PC information |
| Huawei, HiSilicon | We share similar view with Nokia. Options 1 and 2 can be combined for FFS. Moreover, “RU” should be revised to “FU” in option 4. |
| Spreadtrum | We share similar view with MediaTek. |
| Sharp | Support the proposal. |
| CATT2 | It seems the difference between option1 and option 2 is one is dynamic while the other is semi-static. Also we think the bullet should be change to (since it’s too early to agree to support all these options) The following options can be considered for indication of ON-OFF information to NCR by gNB |

    - Summary of the 2nd round

For the Updated Proposal 3-2, this proposal is fine for majority except for the clarification on the difference between Option-1 and Option-2. Regarding this issue, as mentioned in the summary before, in Option-1, the on-off state may be indicated or controlled per indication, i.e., one state change per indication.  However, in Option-2, one pattern is predefined.

Regarding the difference between Option-2 and Option-4, in Option-4, it means that the legacy DRX of MT can be extended to control the ON-OFF state of NCR as additional functionality. But in Option-2, the defined pattern is only to control the forwarding.

Meanwhile, regarding the implicitly solution, potential solutions have already been highlighted by companies and it’s fair to capture it.

Then, the following updated version is listed:

Proposal 3-2-1: The following options can be considered to indicate the ON-OFF information to NCR by gNB

- Option 1: Explicit indication with the single or multiple on-off state per indication

- Option 2: Explicit indication with the on-off pattern

- Option 3: Implicit indication with signalling for other information (e.g., beam, DL/UL configuration, or PC information)

- Option 4: Implicit indication with the extension of DRX procedure to NCR-FU

- Other solutions (e.g., potential combination of explicit and implication solution) are not precluded.

- Topic-4 TDD configuration

  - Company view (Round-1)

For the TDD configuration, many companies share the views that it’s necessary to let the NCR be aware of TDD UL/DL configuration. And this information is required to determine proper DL and UL forwarding in the similar way as legacy RF repeater [Nokia, ZTE, CMCC, CAICT, Qualcomm, Samsung, Intel]. Then, FL think it is necessary to identify TDD UL/DL configuration is needed for NCR in the TR.

Meanwhile, regarding details of TDD configuration, especially for the granularity of this information in time domain, following views are shared by companies:

- Option 1: Static DL/UL configuration [ZTE, CATT, Intel, Qualcomm]

- Option 2: Only semi-static DL/UL configuration [Huawei/HiSilicon, ZTE, Nokia/Nokia Shanghai Bell, Xiaomi, Lenovo, Samsung, CMCC, MediaTek, Intel, Ericsson, CATT, KDDI] with following reasons:

- In practical networks, dynamic TDD is rarely used and it would increase the chance of cross-link interference. [Huawei, ZTE, CMCC, Lenovo, Xiaomi, Intel, Ericsson, KDDI]

- For NCR, the coverage area is much smaller and hence a smaller number of covered UEs. Therefore, only a small portion of the overall time resources are expected to be required by the UEs connected via NCR. [Huawei]

- An NCR has higher energy efficiency and lower overhead if it is only ON in the semi-static UL and DL time, due to less dynamic UL-DL indication. [Huawei]

- Dynamic TDD is not strictly necessary although it can increase network flexibility at the cost of increased NCR signalling overhead, complexity and cost. [ZTE, Nokia, vivo]

- With consideration on the additional processing time and potential DL-UL switching time, the benefits for dynamic TDD is meaningless for the resource usage. [ZTE, Intel]

- To investigate the additional benefit due to the dynamic SFI, a model is needed for self-interference coupling across different DL/UL directions. Given the limited time for SI phase, suggest to not consider L1-signaling for dynamic TDD configuration for NCR in Rel-18.[MediaTek]

- Option 3: Both dynamic and semi-static DL/UL configuration [LGE, Qualcomm, vivo] with following reasons:

- Supporting dynamic TDD for Rel-18 NCR would be beneficial in terms of efficient resource utilization and short latency. [LGE, vivo]

- Dynamic change of the UL-DL information can better adapt to the traffic pattern while causing complicated interference problem. [Xiaomi]

In addition, according to the simulation results from [QC], about 4.5 dB gain on median SINR can be achieved by introducing TDD-awareness as listed below:

- [Source-1, QC] shows that a TDD-aware repeater (i.e., with full information about UL-DL TDD configuration) has a superior performance compared to a repeater that forwards signals blindly in both DL/UL directions with increased coverage (e.g., by ~10%), and improved the throughput (e.g., a median SINR improvement of ~4.5dB).

Then, from FL’s perspective, followings are proposed:

Proposal 4-1: At least semi-static TDD UL/DL configuration is needed for network-controlled repeater to ensure the proper DL/UL behavior.

Companies are encouraged to share your views and if there is concern, please directly propose the corresponding updates.

| Companies | Comments and Views |
| --- | --- |
| ETRI | We think dynamic TDD should be further studied as well, at least for future-proof design of NCR. |
| vivo | Support the proposal in principle |
| Ericsson | Support. We don’t think dynamic TDD is motivated based on network interference and the very limited deployment of such networks. Besides, time does not allow such specification in this release. |
| Nokia | Support |
| CATT | Support |
| Lenovo | Support the proposal. |
| NEC | We think dynamic TDD should be studied too. |
| NTT DOCOMO | We support. |
| AT&T | Open to supporting dynamic TDD as well (at least to ensure dynamic TDD can be operated at the gNB) |
| Intel | Support |
| CMCC | Support the proposal. |
| ZTE | Support |
| Xiaomi | Support |
| Rakuten | Support |
| Sony | We support this proposal. |
| Spreadtrum | Support. |
| KDDI | We support this proposal. |
| Fujitsu | Support the proposal. But there seems a cross with 9.8.2 as well. |
| LG | It seems more appropriate to say that the TDD UL/DL information applied to the F-link is at least semi-statically determined rather than the current proposal. Semi-static TDD UL/DL configuration in the proposal can be interpreted differently by each company. It may be interpreted as the semi-static TDD UL/DL configuration received by the MT is applied to F-link, or it may be interpreted as separately configuring the semi-static TDD UL/DL configuration applied to the F-link. Therefore, in order to use the current proposal as it is, clarification of the semi-static TDD UL/DL configuration seems required. |
| CEWiT | Support the proposal |
| CableLabs | Support |
| Huawei, HiSilicon | Support |
| Samsung | Support the proposal. |
| IIT-K | Support |
| Philips | We support the proposal but, like AT&T, are open to consider dynamic TDD as well. |
| Qualcomm | Partially support. We think some clarification is really needed. It is essential to make sure the repeater can properly operate within the current NR framework, while it is nice to also guarantee some level of futureproof-ness. We agree that the semi-static TDD UL/DL configuration should at least be provided to the repeater. However, let’s assume (for whatever reason) some slots are labelled as “flexible” in the semi-static TDD UL/DL configuration. The question is about the expected behaviour of the repeater, when it is instructed to forward a signal within such resources. We should discuss this case. Without any further information about the forwarding direction (UL vs DL), will the repeater assume this is an error case and ignore the instruction? Will it assume a default direction (e.g., DL)? Will it (if has the capability) blindly forward signals in both directions? We think the side-control info that primarily provides dynamic beam information (e.g., sent via DCI) could also optionally indicate whether this is for DL or UL, with minimal extra overhead. However, before discussing possible solutions, we need to agree that there could be such an ambiguity problem that should be discussed. Hence, we propose the following modification. Proposal 4-1: At least semi-static TDD UL/DL configuration is needed for network-controlled repeater to ensure the proper DL/UL behavior. FFS: how to handle the symbols/slots labeled as “flexible” in the semi-static TDD DL/UL configuration. |
| Charter | We also think that dynamic TDD should be studied too. |
| InterDigital | Dynamic and semi-static TDD signalling should both be supported. Full duplex operation in NCR is an important point of differentiation compared with legacy RF repeaters and IAB. Therefore, both semi-static and dynamic TDD should be studied. |
| Sharp | Support |

In addition, regarding whether to introduce additional TDD DL/UL configuration into side control information, different views are shared by companies as below:

- Side control information: [Spreadtrum, Sony, Charter, CableLabs, CEWiT/IIT-K, Sharp, Qualcomm].

In this way, as commented by companies, some benefits can still be achieved, e.g., introduction of NCR-specific TDD pattern as side control information to avoid limiting the deployment [Charter].

- Reuse the legacy configuration for UE in Rel-17:[Intel, ZTE, Ericsson, Fujitsu]

In this way, companies think that it seems sufficient that NCR obtains TDD UL/DL configuration simply by receiving cell broadcast information. NCR switches to DL/UL chain for DL and UL symbols and NCR could stop forwarding and prepare for switch in flexible symbol. Therefore, there is no motivation to transmit TDD UL/DL information by side control information.

From FL’s perspective, this aspect related to the detailed signalling design can be handled in AI 9.8.2 by taking above two as candidate solutions.

Companies are encouraged to share your views and if there is concern, please directly propose the corresponding updates.

| Companies | Comments and Views |
| --- | --- |
| vivo | legacy configuration for UE in Rel-17 should be the baseline, we can further discuss whether side control information also convey TDD conf. or not. |
| Ericsson | Regarding whether to introduce repeater specific side-control information, e.g., NCR-specific TDD patterns, the repeater-MT would be configured with legacy methods. Having the forwarding function configured with a TDD patter that is not a subset of the repeater-MT TDD pattern would be problematic, at best. |
| NTT DOCOMO | We are fine that this point can be discussed in AI 9.8.2. We think NCR-MT may have TDD pattern information, so that indicating TDD configuration via side control information seems not necessary. |
| CMCC | We are fine to move the discussion to the AI 9.8.2. We are open for both options. But if the side control information is used, the TDD configuration indicated in the SCI should not conflict with the one broadcasted by the gNB. |
| Sony | We think that whether to introduce additional TDD DL/UL configuration into side control information should be discussed further. |
| LG | Applying the legacy TDD configuration for UE to determine DL/UL of F-link can be baseline. Regarding this, it seems necessary to discuss whether to apply only TDD-UL-DL-ConfigurationCommon or TDD-UL-DL-ConfigDedicated as well. Additionally, dynamic DL/UL indication by side control information can be considered for flexible resource utilization. |
| CEWiT | Support |
| Huawei, HiSilicon | We think additional TDD DL/UL configuration is not needed. In practical deployment, only semi-static UL-DL TDD configuration are used to avoid unnecessary cross-link interference among gNBs as well as the UEs. And the semi-static UL-DL configuration for NCR RU can be obtained by NCR MT, e.g., SIB1 or dedicated RRC configuration. |
| Samsung | Share similar view with vivo. |
| IIT-K | Support |
| Philips | Okay to move to AI 9.8.2. |
| Qualcomm | Repeater-MT’s TDD configuration (acquired via any of TDD-UL-DL-ConfigCommon, TDD-UL-DL-ConfigDedciated, or SFI) should be the baseline (and in many cases, sufficient) for the forwarding operation (by NCR-RU). However, as commented above, the question is what if some symbols/slots are still labelled as “flexible” in the Repeater-MT’s TDD configuration. We need to discuss and have a working solution. |
| Charter | We support side control information as this will make it easy for NCR to decode UE specific TDD patterns on a particular beam |
| Sharp | Support |

    - Summary of the 1st round

According to the further discussion, the following agreement have been achieved for Topic-4:

Agreement

For the TDD UL/DL configuration of network controller repeater:

- At least semi-static TDD UL/DL configuration is needed for network-controlled repeater for links including C-link, backhaul link and access link.

  - FFS: handling of flexible symbols

- Note1: The same TDD UL/DL configuration is always assumed for backhaul link and access link

- Note2: The same TDD UL/DL configuration is assumed for C-link and backhaul link and access link if NCR-MT and NCR-FU are in the same frequency band.

- Topic-5 Timing information

  - Company view (Round-1)

For this topic, according to the views of most companies, regarding the timing alignment for DL/UL transmission of NCR for both C-link and forwarding link (including FLB and FLS), following three assumptions are proposed:

- Option 1: The UL and DL timings of the NCR-RU are aligned with the DL timing of the NCR-MT [Samsung]

- Option 2: The DL timing of the NCR-RU is aligned with the DL timing of the NCR-MT. The UL timing of the NCR-RU is aligned with the UL timing of the NCR-MT. [Huawei/HiSilicon, ZTE, Nokia/Nokia Shanghai Bell, Intel, Ericsson, Samsung, CMCC]

Meanwhile, if the internal delay of NCR-RU (e.g., group delay) is not negligible, the corresponding impacts should also be considered to determine the boundary, i.e., the DL Tx timing of NCR-RU is based on the the DL Rx Timing and internal delay of NCR-RU; The UL Rx timing of NCR-RU is based on the UL Tx timing of NCR-MT/RU and internal delay of NCR-RU [LGE, Qualcomm]

However, two companies [Ericsson, ZTE] think any possible internal delay can be handled by implementation, more companies think it should be taken into consideration.

- Option 3: The UL and DL timings of NCR-RU are aligned with the DL timing of gNB [Samsung]

For these solution,

- In Option 1, to achieve the alignment of UL and DL timing of NCR-RU, it seems that the UE need to compensate the TA between NCR and UE. However, since the NCR is transparent to the UE, the UE can neither know a propagation delay between itself and the NCR nor a TA to be compensated for its UL transmission to the NCR. The above assumption may not be applicable and the UE can only maintain one global TA. Meanwhile, all of timings of the NCR-RU are aligned with the DL Rx timing of the NCR-MT. The DL Tx timing and the UL Rx timing at the gNB side is not aligned. Thus the timing of the NCR-RU UL Tx signals arriving at the gNB side cannot be aligned with the timing of other UL signals from the NCR-MT and other gNB-served UEs. It will cause resource collision and interference.

- In Option 2, the timing of NCR-RU mainly follows the timing of NCR-MT. And regarding the DL/UL of NCR-RU part, it’s aligned with the assumption of legacy RF repeater.

Regarding the impacts of internal delay of RU, it can also be handled in the similar way as legacy repeater without spec impact.

- In Option 3, it has similar problems as Option 1. The misalignment of received UL signals at the gNB side from NCR-RU, NCR-MT and other gNB-served UEs will cause interference. It is difficult to ensure that NCR-RU UL Rx is aligned with NCR-RU DL Tx as the NCR is transparent to the UE. In addition, Option 3 also requires the NCR to have the function of caching and can control the forwarding time of NCR RU, which will increase delay, complexity and cost.

Then, from FL’s perspective, Option-2 is proposed:

Proposal 5-1: For the timing of NCR, the following assumption is considered as baseline:

- The DL timing of the NCR-RU is aligned with the DL timing of the NCR-MT.

- The UL timing of the NCR-RU is aligned with the UL timing of the NCR-MT.

FFS: the impact of internal delay of NCR-RU.

Companies are encouraged to share your views and if there is concern, please directly propose the corresponding updates.

| Companies | Comments and Views |
| --- | --- |
| ETRI | We think the Rel-17 RF repeater should be the baseline for this topic. |
| Vivo | We support the proposal |
| Ericsson | Support |
| Nokia | Proposal may require some further clarification. For DL: We agree that NCR-RU DL Rx timing should be aligned NCR-MT DL Rx timing, but DL Tx time has a deterministic offset relative to DL Rx timing. For UL: We agree that NCR-RU UL Tx timing should be aligned with NCR-MT UL Tx timing, but here NCR-RU UL Tx timing has a deterministic offset relative to NCR-RU UL Rx timing. |
| Apple | Support. |
| CATT | support |
| Lenovo | We are generally fine with option 2, however, we think the internal delay should be considered firstly. |
| NEC | Support in general. And we have a little modification: - The DL receiving timing of the NCR-RU is aligned with the DL receiving timing of the NCR-MT. - The UL transmitting timing of the NCR-RU is aligned with the UL transmitting timing of the NCR-MT. FFS: the impact of internal delay of NCR-RU. |
| NTT DOCOMO | We support. |
| AT&T | Support |
| Intel | Support |
| CMCC | Support and fine with the updates from NEC. |
| ZTE | We support the Proposal. In fact, the timing of NCR-RU in option 2 is the same as the legacy RF repeater. If considering the concerns from ETRI/Nokia/Lenovo, we are also open to define the timing of NCR-RU according to the timing of the legacy RF repeater (handling internal delay belongs to implementation and has no spec impact on timing). |
| Xiaomi | Support. For the internal delay of NCR, we think the UL-DL switching time in the backhaul link should be considered, which is different from the legacy RF repeater. And the delay can be compensated by the UE to maintain UL timing alignment between NCR-RU and NCR-MT. |
| Sony | We support this proposal for the DL. For the UL, however, we believe that some more discussions are needed. In particular, some UEs might be served by a direct link to the BS and by a link via the NCR, simultaneously. In this case, aligning the UL timing of the NCR-RU with that of the NCR-MT will most likely lead to UL transmissions of such UEs being incompletely forwarded by the NCR. This is because the timing advance of these UEs will likely be set to align transmissions according to the earliest of the two links, i.e., the direct link to the BS. A simple remedy exists, which is to allow the NCR-RU, in the UL, to start transmissions earlier than the NCR-MT, according to some NCR-RU timing advance value set by the gNB. |
| Spreadtrum | Support. |
| KDDI | We support this proposal. |
| Fujitsu | Support the proposal. |
| LG | We are ok to the principle, but we’d like to discuss DL-Rx/UL-Tx timing and DL-Rx/UL-Rx timing of the NCR-RU separately. In our view, the DL-Rx timing for the FLB link can be aligned with the DL timing of the NCR-MT and the UL-Tx timing for the FLB link can be aligned with the UL timing of the NCR-MT. However, regarding DL-Tx timing and UL-Rx timing for FLS link, whether these timings can be aligned with the DL and UL timing of NCR-MT or the effect of delay should be considered, it would be good to leave it as a further study. |
| CEWiT | Support the proposal |
| CableLabs | We are okay with the proposal with the understanding additional offset between two timings would be further discussed under FFS |
| Huawei, HiSilicon | Support |
| Samsung | Support the proposal. |
| IIT-K | Support |
| Philips | We support this proposal. |
| Qualcomm | We support this proposal with the modifications suggested by NEC. |
| Charter | We are fine with the proposal |
| InterDigital | Support Proposal 5-1. |
| Sharp | Support |

In addition, regarding the DL/UL timing determination of the NCR-MT, following options are summarized

- Option 1: the same as normal UE, e.g. obtained by SSB measurement [Huawei, ZTE, vivo, Lenovo, Samsung, Nokia/Nokia Shanghai Bell, CATT, Intel, KDDI, Spreadtrum]

- Option 2: timing information is included in side control information or derived from the channel carrying the side control information [Spreadtrum, Sony]

- Option 3: Based on the implementation during the deployment [Spreadtrum]

According to the above summary, it’s clear that majority prefer to reuse legacy timing mechanisms for NCR-MT. And from FL’s perspective, since the NCR-MT at least has the capability to process the L1/L2 signalling, it’s reasonable to reuse the legacy mechanism and the following proposal is recommended:

Proposal 5-2: The determination of DL/UL timing for NCR-MT follows the legacy timing mechanism in Rel-17.

Companies are encouraged to share your views and if there is concern, please directly propose the corresponding updates.

| Companies | Comments and Views |
| --- | --- |
| ETRI | Is it proposed as a conclusion? |
| vivo | Support |
| Ericsson | Support |
| Nokia | More discussion is necessary. It may be that DL and UL timing synch could be managed by implementation for a stationary deployment. |
| Apple | Support |
| CATT | support |
| Lenovo | Support the proposal. |
| NEC | Support |
| NTT DOCOMO | We support. |
| AT&T | Support |
| Intel | Support |
| CMCC | Support |
| ZTE | We support the Proposal. |
| Xiaomi | Support |
| Sony | We support this proposal. In addition, we propose that the NCR-MT is considered the timing reference of the NCR-RU. (The timing of the NCR-RU can then be further adjusted by the gNB by means of a newly introduced NCR-RU timing advance parameter. The purpose of this further adjustment is explained in our comment to Proposal 5-1.) |
| Spreadtrum | Support. |
| KDDI | We support this proposal. |
| Fujitsu | Support the proposal. |
| LG | Support |
| CEWiT | Support the proposal |
| CableLabs | We are not very clear on the proposal. Does it mean repeater would use initial access based TA adjustment and need to handle TA adjustment MAC CEs? As a main objective of NCR is low complexity/cost, we like to further discuss what the implication of the proposal is. |
| Huawei, HiSilicon | Support |
| Samsung | Support the proposal. |
| IIT-K | Support |
| Philips | We support the proposal. |
| Qualcomm | Support. |
| Charter | We support the proposal |
| InterDigital | Support Proposal 5-2. |
| Sharp | Support. Please clarify that the timings here are for synchronization and TA only. |

    - Summary of the 1st round

According to the further discussion, the following agreement have been achieved for Topic-5:

Agreement

For the timing of NCR, the following assumption is considered as baseline:

- The DL receiving timing of the NCR-FU is aligned with the DL receiving timing of the NCR-MT.

- The UL transmitting timing of the NCR-FU is aligned with the UL transmitting timing of the NCR-MT.

- FFS: the impact of internal delay on the following timing relationships:

  - The DL receiving timing and DL transmitting timing of the NCR-FU

  - The UL transmitting timing and UL receiving timing of the NCR-FU

For the [Proposal 5-2], details can be further discussed in AI 9.8.2.

- Topic-6 Power control information

  - Company view (Round-1)

For the power control information, although it’s proposed as the 2nd priority in this SI, many companies [Huawei, ZTE, Spreadtrum, vivo, NEC, Lenovo, Charter, DCM, CMCC, Panasonic, ETRI, CEWiT, AT&T, Rakuten Moible] still prefer to support this feature for NCR. [Ericsson] also highlights that study on gain control and associated signaling is needed for self-interference management due to repeater oscillation. The benefits of this information is also justified by the simulation results from [Huawei, vivo, ETRI] as listed below:

- [Source-1, Huawei] shows that for the uplink transmission via NCR, a fixed NCR amplifying gain may lead to interference to the gNB or NCR UL coverage loss. For the downlink transmission via NCR, a fixed NCR amplifying gain may lead to NCR RU saturation or NCR DL coverage loss.

- [Source-2, vivo] shows that the optimal system performance can be achieved when repeater’s gain is set to a proper value.

- [Source-3, ETRI] shows that dynamic repeater gain/power control can provide additional SINR gain over semi-static repeater gain/power configuration.

However, others [CATT, LG, Samsung, CAICT, MTK, KDDI, Qualcomm, Fujitsu] prefer to deprioritize the discussion.

Then, from FL’s perspective, following is proposed:

Proposal 6-1 Power control information is beneficial for network-controlled repeater to control the behaviour of NCR-RU.

- FFS: Detailed mechanism of indication.

Companies are encouraged to share your views and if there is concern, please directly propose the corresponding updates.

| Companies | Comments and Views |
| --- | --- |
| ETRI | Support the proposal. |
| vivo | Support the proposal |
| Ericsson | Do not support the proposal. While we agree that power/gain control can be used for coverage configuration, we are not sure if such configuration should be specified by RAN1 but can be left to OAM. Additionally, repeater power/gain control must not interfere with UE power/gain control, limiting the applicability of power/gain control, in particular to longer time constants than UE power/gain control. Another matter is that the repeater requirements will need to change with to advanced power/gain control, e.g., analog components will face stricter linearity requirements, driving up cost and power consumption. For that reason, power/gain control should be limited to situation where it is necessary for operation, i.e., repeater self-oscillation. |
| Nokia | It is unclear if this is intended as a proposal or a conclusion. Power control information may be beneficial, but the question should be whether we consider enhancements to support power control enhancements or whether this should be down-prioritized. Our understanding is that power control is an optimization and should only be considered after progress is made on essential functionality. |
| CATT | At this stage it is hard to make the conclusion |
| Lenovo | Support the proposal. |
| NEC | Support |
| NTT DOCOMO | We would like to clarify the power control is for UL or DL or both UL and DL. We support especially for UL power control. For DL power control, we are open for the discussion, on the other hands, we are not sure dose gNB have sufficient information (whether interference happens or not between NCR and gNB, and sufficient NCR DL power) to control NCR DL power. |
| AT&T | Support the proposal – the time scale of the power control should be further considered as mentioned by Ericsson |
| Intel | We prefer to focus on essential functionalities, deprioritize the discussion for power control at this stage. |
| CMCC | Support the proposal. Details could be further studied. |
| ZTE | We support this proposal and the details of controlling including time scale, or gain only can be further discussed. |
| Rakuten | We share the same view as NTT DOCOMO. |
| Sony | Support. |
| Spreadtrum | Support. |
| Fujitsu | Support the proposal. @ Ericsson Regarding “For that reason, power/gain control should be limited to situation where it is necessary for operation, i.e., repeater self-oscillation”, could you please elaborate more about how to resolve the issue in your mind? Would it require side control information, or any other information exchanged between gNB and NCR? |
| LG | We’d like to discuss further. |
| CEWiT | Support the proposal |
| CableLabs | We share a similar view with Intel, like to deprioritize this topic |
| Huawei, HiSilicon | Support for both UL and DL. As discussed in our contribution there are some major problems to use a fixed gain for uplink: - Amplified noise leads to interference for UL multi-user multiplexing (e.g., FDM) if the fixed gain is high. - Uplink transmission failure for a UE far away from the NCR if the fixed gain is low. - Even worse, an over-amplified signal may block the gNB reception. For downlink, more severe problems exist if a fixed gain is adopted: - Signal distortion if the fixed gain make the NCR saturated (due to a large NCR received power). - Reduced NCR coverage leads to coverage hole at a lower NCR received power (as a result of a lower NCR DL transmission power). - Unstable coverage performance due to the time changing NCR received power. The received power depends on backhaul link condition, such as channel fading or blockage. To address some of the comments, for both DL and UL, there will be broadcast transmissions e.g. SSB/SIB1/PRACH and unicast transmissions, e.g. CSI-RS/PDSCH/PUSCH. Typically they will use different beams, i.e. wide beam for broadcast channels and narrow beam for unicast transmissions. Therefore, the PSD received at the NCR will be different across different channels/signals. If the same amplifying gain is used, there will be a coverage mismatch between broadcast channels and dedicated channels. |
| Samsung | A fixed gain is preferred for NCR-RU, and no power control information is preferred. |
| IIT-K | Support |
| Philips | We agree with Nokia. |
| Qualcomm | What is the implication of this proposal? Is it to agree to “consider” power control for the repeater? If so, then we are OK to support only if it is clarified that this aspect still has a secondary priority and should not be treated at the same priority as the other identified aspects. |
| Charter | Support the proposal |
| Sharp | Power control can be deprioritized. As mentioned in the SI description A fixed maximum power can be used as baseline. |

In addition, regarding on the detailed solution for PC information indication, as mentioned by [CMCC, QC, ZTE, ETRI, HW], controlling of either amplifying gain or transmission power can be considered. Then, from FL’s perspective, following is proposed as starting point.

Proposal 6-2: The following options can be considered to enable the power control of NCR

- Option-1: Amplifying gain control

- Option-2: Transmission power control

Companies are encouraged to share your views and if there is concern, please directly propose the corresponding updates.

| Companies | Comments and Views |
| --- | --- |
| ETRI | Fine with the proposal. Also OK to add that other options are not precluded. |
| Vivo | Agree |
| Ericsson | Disagree. We don’t understand how Option 2 can work with variable input power. Furthermore, it would not be transparent to the UE since UE PC would definitely need to change due to the repeater interfering in amplification. |
| Nokia | In our view this could not be agreed unless proposal 6-1 was agreed as it seems to be part of the FFS related to indication details. |
| CATT | Need more discussion |
| Lenovo | Support the proposal. |
| NEC | Support |
| NTT DOCOMO | We would like to clarify the proposal is for UL or DL or both UL and DL. For UL power control of NCR-RU, we may consider another option, such as following UL power of NCR-MT (and UL power of NCR-MT can be controlled with legacy power control mechanism). |
| AT&T | Support the proposal |
| CMCC | Support the proposal. |
| ZTE | We support the Proposal. And Option-1 is preferred due to its simplicity if down-selection is needed. |
| Xiaomi | If power control is supported, we prefer option 1. |
| Sony | We support discussing both options. |
| Spreadtrum | Support the proposal. |
| Fujitsu | We are fine with the proposal. |
| LG | We’d like to discuss further. |
| CEWiT | Support the proposal |
| Huawei, HiSilicon | Support the proposal and will be fine to limit to option 1. |
| Samsung | It is preferred to deprioritize the discussion. |
| IIT-K | Support |
| Philips | If power control is supported then option 1 seems the better choice. |
| Qualcomm | We agree with Nokia. This proposal depends on Proposal 6-1. And we should settle the earlier proposal first. |
| Charter | We are fine with the proposal |
| Sharp | Fine with the proposal if power control is not de-prioritized. |

  - Company view (Round-2)

For the [Proposal 6-1], the key point from companies is still on whether the PC is needed or not. In our view, according to the current inputs, it’s reasonable to make the observation that the PC is beneficial (e.g., including both DL and UL according to the evaluation and also to address repeater self-oscillation). Then, updated version is preferred.

Proposal 6-1 Recommend to capture the following observation in TR 38.867.

- Power control information is beneficial for network-controlled repeater to control the behavior of NCR-FU.

  - FFS: Detailed mechanism of indication.

Companies are encouraged to share your views and if there is concern, please directly propose the corresponding updates.

| Companies | Comments and Views |
| --- | --- |
| Lenovo | Support. |
| CMCC | Support |
| CEWiT | Support the proposal |
| Vivo | Support |
| IIT-K | Support. |
| Fujitsu | Support |
| NEC | Agree. |
| Ericsson | Do not support. The general topic of power control is down prioritized in the SID and should stay that way. Additionally, there are disadvantages with PC in terms of cost, repeater efficiency and UE operation being affected. For that reason, we think a formulation regarding power control should be restricted to power control enabling repeater operation. Considering self-oscillation is the real threat to repeater operation, we propose the following: Power control is beneficial for the NCR to enable the functionality of the NCR-FU. |
| AT&T | Support |
| Nokia | We still have some concern about the clarity of the proposal. The benefits noted by Huawei in the previous round seem to be more clearly a benefit of gain control rather than power control. It would also help to identify the challenges we intend to address. It seems the primary benefit is minimizing non-linear distortion and self-interference. Both problems may be solved by implementation, while other issues like CLI may require more complicated solutions. Additionally, the benefits/costs to our understanding are not equal in DL and UL. It seems that UL gain control may be beneficial, but the motivation for DL gain control is less clear. We would prefer to limit the scope of study to side control information for UL gain control for the purpose of limiting non-linear distortion and oscillation/self-interference. |
| Qualcomm | We support the proposal in its current form. |
| Sony | Support. |
| Charter | We are fine with the proposal |
| NTT DOCOMO | We assume that the power control is for both UL and DL, and would like to clarify it. And we think UL power control is more beneficial for mitigating interference. |
| Samsung | It is preferred to deprioritize the discussion. |
| ZTE | We are fine with this proposal. Whether it’s for gain or power, can be considered in proposal 6-2. |
| LG | We’d like to understand if this proposal applies to either DL Tx power control or UL Tx power control, or both. Furthermore, we would like to ask supporters of power control why they are supporting this proposal at this stage. We want to understand the benefit of it. |
| Huawei, HiSilicon | Support. To Ericsson: We think power control can improve the network performance in terms of reducing interference, avoiding saturation, and guaranteeing the NCR coverage, as clarified in the previous round. The repeater efficiency can also be improved if the NCR power / gain is matched with backhaul and access link, since a lower gain can be adopted for a good backhaul and access link condition, rather than always the largest / fixed gain. The repeater gain is already adjustable, and we don’t think cost will be impacted by power control. To Nokia: we think the gain control is one approach to achieve power control. The NCR DL/UL transmission power can be derived by amplifying gain, the received signal and noise in the passband, such as: PNCR, Tx=GNCR+10log1010PNCR, Rx/10 +10PNCR, Noise/10 [dBm] |
| Spreadtrum | Support. |
| Rakuten | Support |
| Sharp | Support the proposal. |
| CATT2 | It is too early to reach this conclusion. |

For the [Proposal 6-2], with consideration on the comments, e.g., whether to support PC or additional solutions, following updates is made:

Updated Proposal 6-2: The following options can be considered to enable the power control of NCR-FU

- Option-1: Amplifying gain control

- Option-2: Transmission power control

- Others solutions are not preclude.

Note: Down-selection is expected in SI.

Companies are encouraged to share your views and if there is concern, please directly propose the corresponding updates.

| Companies | Comments and Views |
| --- | --- |
| Lenovo | Support. |
| CMCC | Fine with the proposal. |
| CEWiT | Support the proposal |
| vivo | What is other solutions, is there any example, if we have no idea on the other solution, we prefer to remove the bullet. One/multiple of the solution should be selected after more discussion. Updated Proposal 6-2: The following options can be considered to enable the power control of NCR-FU - Option-1: Amplifying gain control - Option-2: Transmission power control - Others solutions are not preclude. Note: Down-selection is expected in SI one/multiple of the options can be down-selected. |
| IIT-K | Support. |
| Fujitsu | Support. |
| NEC | Support. |
| Ericsson | Do not support the proposal. Unless companies can motivate how Option-2 can work with legacy UE power control, we propose to already now narrow down to Option-1. |
| AT&T | Support studying both options. |
| Qualcomm | Support. |
| Sony | Support with vivo’s modifications. |
| Charter | Agree with AT&T |
| NTT DOCOMO | We would like to discuss this point after we reach agreement of Proposal 6-1. At least we would like to clarify the proposal is for UL or DL or both UL and DL, since different solution can be applied for UL and DL power control, e.g. for DL power control Option-1/2 can be applied and for UL power control, Option-1/2 or UL Tx power of NCR-FU may follow that of NCR-MT. |
| Samsung | It is preferred to deprioritize the discussion. |
| ZTE | Support to study both options and clarify the details of Option-2 in future. |
| MediaTek | We are not clear on option 2. As explained by E///, it is not possible to control Tx power due to variable input power. Or is the intention for option 2 to control max. Tx power of NCR? |
| LG | We are not sure if Option 2 makes sense. We prefer Option 1. |
| Huawei, HiSilicon | As commented earlier, we are fine to make down-selection to option 1 if majority feel this is the way to go. |
| Rakuten | Support |
| Sharp | Support the proposal. |
| CATT2 | Prefer to defer this after Proposal 6-1. |

    - Summary of the 2nd round

For the Proposal 6-1, the current proposal is fine for majority based on the inputs with following comments:

- Regarding [Ericsson]’s comment to restrict the applicable scenario of power/gain control, from moderator’s perspective, once the functionality is enabled in future, how to use is up to gNB’s decision. And the generic solution is applied for all. Meanwhile, we can further discuss the details of signalling including granularity.

- Regarding [Nokia, Docomo]’s comment to clarify the applicable link for this feature, based on the inputs, it seems that enabling this feature is beneficial for both DL and UL.

Then, with consideration on the comment, from moderator’s perspective, following updated version is proposed as:

Proposal 6-1-1: Recommend to capture the following observation in TR 38.867.

- Power control information is beneficial for network-controlled repeater to control the behavior of NCR-FU for the DL of service link and UL of backhaul link.

  - FFS: Detailed mechanism of indication.

For the Updated Proposal 6-2, it seems that more concentrated discussion is preferred and Option-1 is more preferred as baseline. Then, the following updated version is proposed:

Proposal 6-2-1: As the baseline, the controlling of the amplifying gain of NCR-FU is considered to enable the power control of NCR-FU

- FFS: Controlling of the transmission power of NCR-FU

- Proposals for discussion at GTW sessions

Proposal 3-2-1: The following options can be considered to indicate the ON-OFF information to NCR by gNB

- Option 1: Explicit indication with the single or multiple on-off state per indication

- Option 2: Explicit indication with the on-off pattern

- Option 3: Implicit indication with signalling for other information (e.g., beam, DL/UL configuration, or PC information)

- Option 4: Implicit indication with the extension of DRX procedure to NCR-FU

- Other solutions (e.g., potential combination of explicit and implication solution) are not precluded.

Proposal 2-5-1: The beam correspondence is assumed for:

- the DL/UL of the access link at NCR-FU

- the DL/UL of the backhaul link at NCR-FU

- FFS: the DL/UL of the C-link at NCR-MT

Proposal 1-4-1: Recommend to capture the following assumption of network-controlled repeater in TR 38.867.

- The NCR-MT’s carrier should be within the set of carries forwarded by the NCR-FU in same frequency range.

  - As the baseline, the NCR-MT and NCR-FU are in same carrier.

Proposal 6-1-1: Recommend to capture the following observation in TR 38.867.

- Power control information is beneficial for network-controlled repeater to control the behavior of NCR-FU for the DL of service link and UL of backhaul link.

  - FFS: Detailed mechanism of indication.

Proposal 6-2-1: As the baseline, the controlling of the amplifying gain of NCR-FU is considered to enable the power control of NCR-FU

- FFS: Controlling of the transmission power of NCR-FU

Proposal 1-2-1: Recommend to capture the following assumption of network-controlled repeater in TR 38.867.

- As the baseline, same RF component of NCR is used for C-link and backhaul link.

  - If dedicated RF components are used (e.g., antenna) at NCR for C-link and backhaul link, respectively, same large-scale properties of the channel, i.e., channel properties included in Type-A and Type-D (if applicable), are expected to be experienced by these links.

    - Note: It is up to implementation without extra standardization effort.

- Different RF components are used at NCR for the backhaul link and access link.

Proposal 1-3-1: Recommend to capture the following examples of the scheduling of C-link and backhaul link by gNB in TR 38.867.

- The DL of C-link and DL of backhaul link can be performed simultaneously or in TDM way.

- The UL of C-link and UL of backhaul link can be performed in TDM way.

- Note: The scheduling of C-link and backhaul link is up to gNB’s with consideration on NCR capability.

[Proposal 2-3-1]: The same beam as C-link can be assumed for beam at NCR for backhaul link.

- FFS: additional indication from gNB to determine the beam at NCR side for backhaul link if adaptive beamforming is supported by NCR

Note: Legacy mechanism is the baseline for beam indication and determination.

- Conclusion

Appendix

R1-2203133	Discussion on side control information to enable NR network-controlled repeaters	Huawei, HiSilicon

R1-2203237	Discussion on side control information to enable NR network-controlled repeaters	ZTE

R1-2203343	Discussion on side control information to enable NR network-controlled repeaters	Spreadtrum Communications

R1-2203354	Discussion on side control information to enable NR network-controlled repeaters	Nokia, Nokia Shanghai Bell

R1-2203376	Discussions on side control information to enable NR network-controlled repeaters	InterDigital, Inc.

R1-2203476	Discussion on side control information for NR network-controlled repeaters	CATT

R1-2203578	Discussion on side control information to enable NR network-controlled repeaters	vivo

R1-2203697	Discussion on side control information for network-controlled repeaters	NEC

R1-2203741	Considerations on side control information to enable NR network-controlled repeaters	Sony

R1-2203832	Discussion on side control information to enable NR network-controlled repeaters	xiaomi

R1-2203921	Side control information to enable NR network-controlled repeaters	Samsung

R1-2204064	Discussion on side control information for network controllable repeater	Lenovo

R1-2204066	Side Control Information for Network-Controlled Repeaters	Charter Communications, Inc

R1-2204086	Side Control Information for supporting initial access	CableLabs

R1-2204119	Side control information for network-controlled repeaters	SHARP Corporation

R1-2204258	Potential side control information for NW-controlled repeater	Apple

R1-2204321	Discussion on Side control information to enable NR network-controlled repeaters	CMCC

R1-2204393	Discussion on side control information to enable NR network-controlled repeaters	NTT DOCOMO, INC.

R1-2204532	Discussion on side control information for NCR	LG Electronics

R1-2204565	Discussion on side control signals for network-controlled repeaters	Panasonic

R1-2204610	Discussion on side control information to enable NR network-controlled repeaters	CAICT

R1-2204642	Control information for enabling NW-controlled repeaters	Ericsson

R1-2204653	Discussions on side control information for network-controlled repeater	ETRI

R1-2204688	Side control information for network-controlled repeaters	MediaTek Inc.

R1-2204757	Discussion on Side control information to enable NR network-controlled repeaters	CEWiT, IITK

R1-2204813	Disucssion on Side control information to enable NR network-controlled repeater	Intel Corporation

R1-2204847	Discussion on side control information to enable NR network-controlled repeaters	KDDI Corporation

R1-2204864	Side control information to enable NR network-controlled repeaters	AT&T

R1-2205047	On side control information for network controlled repeaters (NCR)	Qualcomm Incorporated

R1-2205068	Initial view on Network-controlled repeater	Rakuten Moible

R1-2205085   Side control information for NR network-controlled repeaters Fujitsu