3GPP TSG RAN WG1 Meeting #109-e	R1-2205350

e-Meeting, May 9th – 20th, 2022

Agenda item:	7.2.5

Source: 	Moderator(ZTE)

Title: 	Summary of [109-e-R16-URLLC-05] Issue#7: Correction on PDCCH reception with last symbol aligning with the first symbol of UL CI for Rel-16

Document for:	Discussion and Decision

Introduction

This document provides the summary of the following email discussion on the issue of PDCCH reception with last symbol aligning with the first symbol of UL CI proposed by [1].

[109-e-R16-URLLC-05] Issue#7: Correction on PDCCH reception with last symbol aligning with the first symbol of UL CI for Rel-16 by May 13 – Junfeng (ZTE)

Discussion

Issue description

According to the following description in clause 11.2A of TS 38.213, UL cancelation of a PUSCH/SRS is applicable only if the last symbol of a PDCCH for the PUSCH/SRS transmission is earlier than the first symbol of PDCCH carrying the UL CI.

On other hand, if a UE has already cancelled a PUSCH/SRS by a UL CI, and a PDCCH can schedule another PUSCH/SRS transmission, as long as 1) the last symbol of the PDCCH is later than the first symbol of PDCCH carrying the UL CI, and 2) the scheduled PUSCH/SRS isn’t overlapped with the symbols of the cancelled PUSCH/SRS transmission.

- 11.2A Cancellation indication ... An indication by a DCI format 2_4 for a serving cell is applicable to a PUSCH transmission or an SRS transmission on the serving cell. If the PUSCH transmission or the SRS transmission is scheduled by a DCI format, the indication by the DCI format 2_4 is applicable to the PUSCH transmission or SRS transmission only if the last symbol of the PDCCH reception providing the DCI format is earlier than the first symbol of the PDCCH reception providing the DCI format 2_4. For the serving cell, the UE determines the first symbol of the TCI symbols to be the first symbol that is after T'proc,2 from the end of a PDCCH reception where the UE detects the DCI format 2_4, where T'proc,2 is obtained from Tproc,2 for PUSCH processing capability 2 [6, TS 38.214] assuming d2,1=doffset⋅2-μUL2-μ where doffset is provided by delta_Offset, μ being the smallest SCS configuration between the SCS configuration of the PDCCH and the smallest SCS configuration μUL provided in scs-SpecificCarrierList of FrequencyInfoUL or FrequencyInfoUL-SIB. The UE does not expect to cancel the PUSCH transmission or the SRS transmission before a corresponding symbol that is Tproc,2 assuming that d2,1=0 after a last symbol of a CORESET where the UE detects the DCI format 2_4. ... If, based on an indication by a DCI format 2_4, a UE cancels a PUSCH transmission or an SRS transmission, the UE does not expect to be scheduled by a second DCI format to transmit a PUSCH or an SRS over symbols that include symbols of the cancelled PUSCH transmission or SRS transmission, where the last symbol of the PDCCH reception providing the second DCI format is later than the first symbol of the PDCCH reception providing the DCI format 2_4.

There is a missing case that if a UE cancels a first PUSCH/SRS transmission based on an indication by a DCI format 2_4, and the last symbol of a PDCCH for a second PUSCH/SRS transmission is aligning with the first symbol of a PDCCH carrying the DCI format 2_4. Then, the second PUSCH/SRS can be scheduled without any restriction according to the current spec. For example, the second PUSCH/SRS can be scheduled on the symbols of the first PUSCH/SRS transmission, which was cancelled previously.

However, this conflicts with the agreement below.

Agreement(RAN1#101-e) (Alt 1) If UE has to cancel a PUSCH transmission or an SRS transmission based on the detected UL CI, another DG-PUSCH2 CAN be scheduled on the resource indicated by the UL CI but not overlapping with cancelled symbols, if the ending symbol of the PDCCH carrying the 2nd UL grant is no earlier than the first symbol of the PDCCH carrying the UL CI. - The above applies regardless whether RRC parameter applicabilityforCI is configured or not - No additional spec impact expected.

Thus, the following TP in R1-2203184 [1] is provided to clarify the PDCCH reception with last symbol aligning with the first symbol of UL CI and to remedy the conflicts with the agreement.

- 11.2A Cancellation indication <Unchanged parts are omitted> If, based on an indication by a DCI format 2_4, a UE cancels a PUSCH transmission or an SRS transmission, the UE does not expect to be scheduled by a second DCI format to transmit a PUSCH or an SRS over symbols that include symbols of the cancelled PUSCH transmission or SRS transmission, where the last symbol of the PDCCH reception providing the second DCI format is no earlier later than the first symbol of the PDCCH reception providing the DCI format 2_4. <Unchanged parts are omitted>

1st round discussion

Question 1: Please provide your views on whether the proposed change in R1-2203184 [1] for Rel-16 can be supported. Or any other suggestion?

| Company name | Support or not | Comments |
| --- | --- | --- |
| CATT | Support |  |
| DOCOMO | Support |  |
| Nokia/NSB | Support |  |
| HW/HiSi | Support |  |
| Qualcomm | Support |  |
| Intel | Support |  |
| ZTE | Support |  |

Question 2: Please provide your views whether the potential mirror CR for Rel-17 is needed.

| Company name | Need or not | Comments |
| --- | --- | --- |
| CATT | Needed |  |
| DOCOMO | Needed |  |
| Nokia/NSB | Needed | Any Rel-16 CR where the feature is still present in R17 requires a mirror CR. |
| HW/HiSi | Needed |  |
| Qualcomm | Needed |  |
| Intel | Needed. |  |
| ZTE | Needed |  |

Second Round

Based on companies’ comments during the first round discussion, the situation was summarized below.

| Q1 | Capture the following TP in the specification TS 38.213 in Section 11.2A < Unchanged parts are omitted > If, based on an indication by a DCI format 2_4, a UE cancels a PUSCH transmission or an SRS transmission, the UE does not expect to be scheduled by a second DCI format to transmit a PUSCH or an SRS over symbols that include symbols of the cancelled PUSCH transmission or SRS transmission, where the last symbol of the PDCCH reception providing the second DCI format is no earlier later than the first symbol of the PDCCH reception providing the DCI format 2_4. < Unchanged parts are omitted > |
| --- | --- |
| Support | (9) CATT, DOCOMO, Nokia/NSB, HW/HiSi, Qualcomm, Intel, ZTE |
| Object | - |

| Q2 | Whether the potential mirror CR for Rel-17 is needed. |
| --- | --- |
| Support | (9) CATT, DOCOMO, Nokia/NSB, HW/HiSi, Qualcomm, Intel, ZTE |
| Object | - |

9 companies support and no objections have been raised against the TP and the need for mirror CR for Rel-17 in Round 1 discussion. In the moderators’ view, the text proposal is stable and can be recommended for endorsement.

Proposal 1: Capture the following TP in the specification TS 38.213 in Section 11.2A

< Unchanged parts are omitted > If, based on an indication by a DCI format 2_4, a UE cancels a PUSCH transmission or an SRS transmission, the UE does not expect to be scheduled by a second DCI format to transmit a PUSCH or an SRS over symbols that include symbols of the cancelled PUSCH transmission or SRS transmission, where the last symbol of the PDCCH reception providing the second DCI format is no earlier later than the first symbol of the PDCCH reception providing the DCI format 2_4. < Unchanged parts are omitted >

Two draft CRs for Rel-16 and Rel-17 are now available in the subfolder [Draft CR] of [109-e-R16-URLLC-05]. Companies are encouraged to further check them including the cover page.

Conclusion

Based on the discussions above, the following is endorsed.

Agreement

The following text proposal for TS38.213 (Section 11.2A) is endorsed in R1-2205348 (TS38.213, Rel-16, CR#0308, Cat. F) and R1-2205349 (TS38.213, Rel-17, CR#0309, Cat. A)

< Unchanged parts are omitted > If, based on an indication by a DCI format 2_4, a UE cancels a PUSCH transmission or an SRS transmission, the UE does not expect to be scheduled by a second DCI format to transmit a PUSCH or an SRS over symbols that include symbols of the cancelled PUSCH transmission or SRS transmission, where the last symbol of the PDCCH reception providing the second DCI format is no earlier later than the first symbol of the PDCCH reception providing the DCI format 2_4. < Unchanged parts are omitted >

Reference

R1-2203184	Correction on PDCCH reception with last symbol aligning with the first symbol of UL CI for Rel-16	ZTE