3GPP TSG RAN WG1 #109-e			           R1-2205392

e-Meeting, May 9th – 20th, 2022

Agenda Item: 8.7

Source: Moderator (MediaTek)

Title: Preparation phase summary of maintenance on idle/inactive UE power saving enhancements

Document for: Discussion and Decision

# Introduction

This preparation summary is for identifying and checking maintenance issues for idle/inactive UE power saving enhancements to be formally discussed (via emails) in RAN1#109-e. Companies’ views and support levels are collected in the following Sections 2 and 2, and final selected topics by session chair (CMCC/Xiaodong) are listed in Section 4.

# Preparation for Maintenance on Paging Enhancements

## Alignment with RAN2 agreement

In RAN2#117-e meeting, the following on bundled functionalities of PEI and UE subgrouping are agreed:

PEI + UEID subgrouping is one capability A PEI-capable UE must support at least UEID-based subgrouping method. RAN2 confirms that “PEI without subgrouping” can be implemented by configuring PEI plus UEID subgrouping with one subgroup. “PEI without subgrouping” can be configured by only one method.

In the following table, there collect companies’ related proposals for alignment with the RAN2 agreements:

| Company name | Company’s views/comments |
| --- | --- |
| Huawei, HiSilicon |  |
| Spreadtrum |  |
| CATT | Proposal 4: Adopt the following TP in TS 38.212 and TS 38.213. ============================ start of TP========================================== ## <Unchanged text omitted> ## 7.3.1.3.8 Format 2_7 DCI format 2_7 is used for notifying the paging early indication and TRS availability indication for one or more UEs. The following information is transmitted by means of the DCI format 2_7 with CRC scrambled by PEI-RNTI: - Paging indication field – NPOPEINSGPO bit(s), where - NPOPEI is the number of paging occasions configured by higher layer parameter PONumPerPEI as defined in Clause 10.4A in [5, TS 38.213]; - NSGPOis the number of sub-groups of a paging occasion configured by higher layer parameter subgroupsNumPerPO, if subgroupsNumPerPO is configured; otherwise NSGPO is set to 1. - Each bit in the field indicates one UE subgroup of a paging occasion if subgroupsNumPerPO is greater than 1configured; otherwise each bit in the field indicates the UE group of a paging occasion. - TRS availability indication – 1, 2, 3, 4, 5, or 6 bits if TRS-ResourceSetConfig is configured; 0 bits otherwise. The size of DCI format 2_7 is indicated by the higher layer parameter payloadSizeDCI_format2_7, according to Clause 10.4A of [5, TS 38.213]. The number of information bits in format 2_7 shall be equal to or less than the payload size of format 2_7. If the number of information bits in format 2_7 is less than the size of format 2_7, the remaining bits are reserved. ## <Unchanged text omitted> ============================= end of TP========================================== ============================ start of TP========================================== ## 10.4A PDCCH monitoring for early indication of paging ## <Unchanged text omitted> A paging indication field of DCI format 2_7 includes NPOPEI segments of K bits, where K=NSGPO if NSGPO>0 and K=1 if NSGPO is not provided. For a subgroup index iSG, 0≤iSG<K, a UE determines a value for the iPO⋅K+iSG bit in the paging indication field, where iPO=UE_IDmodN⋅NS+iSmodNPOPEI is a paging occasion index, and UE_ID, N, NS, iSG, and iS are defined in [17, TS 38.304]. When the value is '1', the UE monitors a paging occasion determined according to [17, TS 38.304]; otherwise, the UE is not required to monitor the paging occasion. If NPOPEI<NS, the number of symbols from the start of the frame to the start of the first PDCCH monitoring occasion for DCI format 2_7 that is associated with paging occasion index iPO is the iSNPOPEI+1-th value from the NSNPOPEI values provided by firstPDCCH-MonitoringOccasionOfPEI-O. ## <Unchanged text omitted> ============================= end of TP==========================================. |
| vivo | Observation 1: For the case of PEI without subgrouping, as per RAN2 TS 38.331 CR, the value of subgroupsNumPerPO-r17 will be configured as ‘one’, while in RAN1 TS 38.213, we have agreed subgroupsNumPerPO-r17 will not be provided in this case. Observation 2: There is inconsistence between RAN1 and RAN2 specifications on the description of PEI without subgrouping. Proposal 1: Adopt the following TPs. --------------------------------------Start of Draft TP for TS 38.213 V17.1.0 (2022-03) -------------------- <Unchanged parts omitted> ## 10.4A PDCCH monitoring for early indication of paging A UE can be provided the following for detection of a DCI format 2_7 in RRC_IDLE state or in RRC_INACTIVE state [12, TS 38.331] - a search space set, by peiSearchSpace, to monitor PDCCH for detection of DCI format 2_7 according to a Type2A-PDCCH CSS set as described in clause 10.1 - a number of frames, by PEI-F_offset, from the start of a first paging frame of paging frames associated with a number of PDCCH monitoring occasions for DCI format 2_7 [17, TS 38.304] to the start of a frame - a number of symbols, by firstPDCCH-MonitoringOccasionOfPEI-O, from the start of the frame to the start of the first PDCCH monitoring occasion for DCI format 2_7 - a size, by payloadSizeDCI_format2_7 - a number of subgroups per paging occasion, NSGPO, by subgroupsNumPerPO - a number of paging occasions associated with the number of PDCCH monitoring occasions for DCI format 2_7, NPOPEI, by PONumPerPEI A paging indication field of DCI format 2_7 includes NPOPEI segments of K bits, where K=NSGPO if NSGPO>0 and K=1 if NSGPO is not provided. For a subgroup index iSG, 0≤iSG<K, a UE determines a value for the iPO⋅K+iSG bit in the paging indication field, where iPO=UE_IDmodN⋅NS+iSmodNPOPEI is a paging occasion index, and UE_ID, N, NS, iSG, and iS are defined in [17, TS 38.304]. When the value is '1', the UE monitors a paging occasion determined according to [17, TS 38.304]; otherwise, the UE is not required to monitor the paging occasion. <Unchanged parts omitted> ------------------------------------- Start of Draft TP for TS 38.213 V17.1.0 (2022-03) --------------------. ---------------------------------------- Start of Draft TP for TS 38.212 V17.1.0 (2022-03) ----------------- <Unchanged parts omitted> ##### 7.3.1.3.8 Format 2_7 DCI format 2_7 is used for notifying the paging early indication and TRS availability indication for one or more UEs. The following information is transmitted by means of the DCI format 2_7 with CRC scrambled by PEI-RNTI: - Paging indication field – NPOPEINSGPO bit(s), where - NPOPEI is the number of paging occasions configured by higher layer parameter PONumPerPEI as defined in Clause 10.4A in [5, TS 38.213]; - NSGPOis the number of sub-groups of a paging occasion configured by higher layer parameter subgroupsNumPerPO, if subgroupsNumPerPO is configured; otherwise NSGPO is set to 1. - Each bit in the field indicates one UE subgroup of a paging occasion if subgroupsNumPerPO is configured; otherwise each bit in the field indicates the UE group of a paging occasion. - TRS availability indication – 1, 2, 3, 4, 5, or 6 bits if TRS-ResourceSetConfig is configured; 0 bits otherwise. The size of DCI format 2_7 is indicated by the higher layer parameter payloadSizeDCI_format2_7, according to Clause 10.4A of [5, TS 38.213]. The number of information bits in format 2_7 shall be equal to or less than the payload size of format 2_7. If the number of information bits in format 2_7 is less than the size of format 2_7, the remaining bits are reserved. <Unchanged parts omitted> --------------------------------------- Start of Draft TP for TS 38.212 V17.1.0 (2022-03) --------------------. |
| ZTE, Sanechips | - Proposal 1: Adopt the following TP. ........................................................... Text Proposal for TS38.213.............................................................................. 10.4 A PDCCH monitoring for early indication of paging *** Unchanged text is omitted *** A paging indication field of DCI format 2_7 includes NPOPEI segments of K bits, where K=NSGPO if NSGPO>0 and K=1 if NSGPO is not provided. For a subgroup ID index iSG, 0≤iSG<K, a UE determines a value for the iPO⋅K+iSG bit in the paging indication field, where iPO=UE_IDmodN⋅NS+iSmodNPOPEI is a paging occasion index, and UE_ID, N, NS, iSG, and iS are defined in [17, TS 38.304]. When the value is '1', the UE monitors a paging occasion determined according to [17, TS 38.304]; otherwise, the UE is not required to monitor the paging occasion. ........................................................... Text Proposal for TS38.213.............................................................................. |
| xiaomi | TP#3 for TS 38.212 Clause 7.3.1.3.8 ============================= Unchanged part omitted ============================== 7.3.1.3.8 Format 2_7 DCI format 2_7 is used for notifying the paging early indication and TRS availability indication for one or more UEs. The following information is transmitted by means of the DCI format 2_7 with CRC scrambled by PEI-RNTI: - Paging indication field – NPOPEINSGPO bit(s), where - NPOPEI is the number of paging occasions configured by higher layer parameter PONumPerPEI as defined in Clause 10.4A in [5, TS 38.213]; - NSGPOis the number of sub-groups of a paging occasion configured by higher layer parameter subgroupsNumPerPONSGPO. - Each bit in the field indicates one UE subgroup of a paging occasion. - TRS availability indication – 1, 2, 3, 4, 5, or 6 bitsif TRS-ResourceSetConfig is configured; 0 bits otherwise. The size of DCI format 2_7 is indicated by the higher layer parameter payloadSizeDCI_format2_7, according to Clause 10.4A of [5, TS 38.213]. The number of information bits in format 2_7 shall be equal to or less than the payload size of format 2_7. If the number of information bits in format 2_7 is less than the size of format 2_7, the remaining bits are reserved. ============================= Unchanged part omitted ============================== TP#4 for TS 38.212 Clause 7.3.1.3.8 ============================= Unchanged part omitted ============================== 7.3.1.3.8 Format 2_7 DCI format 2_7 is used for notifying the paging early indication and TRS availability indication for one or more UEs. The following information is transmitted by means of the DCI format 2_7 with CRC scrambled by PEI-RNTI: - Paging indication field – NPOPEINSGPO bit(s), where - NPOPEI is the number of paging occasions configured by higher layer parameter PONumPerPEI as defined in Clause 10.4A in [5, TS 38.213]; - NSGPOis the number of sub-groups of a paging occasion configured by higher layer parameter subgroupsNumPerPO, if subgroupsNumPerPO is configured; otherwise NSGPO is set to 1. - Each bit in the field indicates one UE subgroup of a paging occasion if subgroupsNumPerPO is configured; otherwise each bit in the field indicates the all the UE associated to a paging occasion. - TRS availability indication – 1, 2, 3, 4, 5, or 6 bitsif TRS-ResourceSetConfig is configured; 0 bits otherwise. The size of DCI format 2_7 is indicated by the higher layer parameter payloadSizeDCI_format2_7, according to Clause 10.4A of [5, TS 38.213]. The number of information bits in format 2_7 shall be equal to or less than the payload size of format 2_7. If the number of information bits in format 2_7 is less than the size of format 2_7, the remaining bits are reserved. ============================= Unchanged part omitted ============================== |
| Samsung |  |
| Panasonic |  |
| Sharp |  |
| Apple |  |
| Nokia | Proposal: Adopt following TP to TS38.213 Section 10.4: ## 10.4A PDCCH monitoring for early indication of paging A UE can be provided the following for detection of a DCI format 2_7 in RRC_IDLE state or in RRC_INACTIVE state [12, TS 38.331] - a search space set, by peiSearchSpace, to monitor PDCCH for detection of DCI format 2_7 according to a Type2A-PDCCH CSS set as described in clause 10.1 - a number of frames, by PEI-F_offset, from the start of a first paging frame of paging frames associated with a number of PDCCH monitoring occasions for DCI format 2_7 [17, TS 38.304] to the start of a frame - a number of symbols, by firstPDCCH-MonitoringOccasionOfPEI-O, from the start of the frame to the start of the first PDCCH monitoring occasion for DCI format 2_7 - a size, by payloadSizeDCI_format2_7 - a number of subgroups per paging occasion, NSGPO, by subgroupsNumPerPO - a number of paging occasions associated with the number of PDCCH monitoring occasions for DCI format 2_7, NPOPEI, by PONumPerPEI A paging indication field of DCI format 2_7 includes NPOPEI segments of K bits, where K=NSGPO if NSGPO>0 and K=1 if NSGPO is not provided. For a subgroup index iSG, 0≤iSG<K, a UE determines a value for the iPO⋅K+iSG bit in the paging indication field, where iPO=UE_IDmodN⋅NS+iSmodNPOPEI is a paging occasion index, and UE_ID, N, NS, iSG, and iS are defined in [17, TS 38.304]. When the value is '1', the UE monitors a paging occasion determined according to [17, TS 38.304]; otherwise, the UE is not required to monitor the paging occasion. If NPOPEI<NS, the number of symbols from the start of the frame to the start of the first PDCCH monitoring occasion for DCI format 2_7 that is associated with paging occasion index iPO is the iSNPOPEI+1-th value from the NSNPOPEI values provided by firstPDCCH-MonitoringOccasionOfPEI-O.. Proposal: Adopt following TP to TS38.212 Section 7.3.1.3.8: ##### 7.3.1.3.8 Format 2_7 DCI format 2_7 is used for notifying the paging early indication and TRS availability indication for one or more UEs. The following information is transmitted by means of the DCI format 2_7 with CRC scrambled by PEI-RNTI: - Paging indication field – NPOPEINSGPO bit(s), where - NPOPEI is the number of paging occasions configured by higher layer parameter PONumPerPEI as defined in Clause 10.4A in [5, TS 38.213]; - NSGPOis the number of sub-groups of a paging occasion configured by higher layer parameter subgroupsNumPerPO, if subgroupsNumPerPO is configured; otherwise NSGPO is set to 1. - Each bit in the field indicates one UE subgroup of a paging occasion if subgroupsNumPerPO is configured; otherwise each bit in the field indicates the UE group of a paging occasion. - TRS availability indication – 1, 2, 3, 4, 5, or 6 bits if TRS-ResourceSetConfig is configured; 0 bits otherwise. The size of DCI format 2_7 is indicated by the higher layer parameter payloadSizeDCI_format2_7, according to Clause 10.4A of [5, TS 38.213]. The number of information bits in format 2_7 shall be equal to or less than the payload size of format 2_7. If the number of information bits in format 2_7 is less than the size of format 2_7, the remaining bits are reserved.. |
| LGE |  |
| MediaTek | Proposal 1: Adopt the following TPs to TS 38.212 and TS 38.213 for removing the case PEI is applied without subgroupsNumPerPO provided, as per the RAN2 agreements in RAN2#117-e. ##### Section 7.3.1.3.8 (Format 2_7) of TS 38.212 <Unchanged text omitted> DCI format 2_7 is used for notifying the paging early indication and TRS availability indication for one or more UEs. The following information is transmitted by means of the DCI format 2_7 with CRC scrambled by PEI-RNTI: - Paging indication field – NPOPEINSGPO bit(s), where - NPOPEI is the number of paging occasions configured by higher layer parameter PONumPerPEI as defined in Clause 10.4A in [5, TS 38.213]; - NSGPOis the number of sub-groups of a paging occasion configured by higher layer parameter subgroupsNumPerPO, if subgroupsNumPerPO is configured; otherwise NSGPO is set to 1. - Each bit in the field indicates one UE subgroup of a paging occasion if subgroupsNumPerPO > 1 is configured; otherwise each bit in the field indicates the UE group of a paging occasion. <Unchanged text omitted>. Section 10.4A of TS 38.213 <Unchanged text omitted> A paging indication field of DCI format 2_7 includes NPOPEI segments of K bits, where K=NSGPO if NSGPO>0 and K=1 if NSGPO is not provided. For a subgroup index iSG, 0≤iSG<K, a UE determines a value for the iPO⋅K+iSG bit in the paging indication field, where iPO=UE_IDmodN⋅NS+iSmodNPOPEI is a paging occasion index, and UE_ID, N, NS, iSG, and iS are defined in [17, TS 38.304]. When the value is '1', the UE monitors a paging occasion determined according to [17, TS 38.304]; otherwise, the UE is not required to monitor the paging occasion. <Unchanged text omitted>. |
| Nordic Semi. |  |
| Intel |  |
| Ericsson | Proposal 2: Adopt TP2 for 38.213-h10, subclause 10.4A and TP3 for 38.212-h10 subclause 7.3.1.3.8 to align with RRC spec on subgroup number configuration. <begin TP2 for 38.213-h10, subclause 10.4A> A paging indication field of DCI format 2_7 includes NPOPEI segments of K bits, where K=NSGPO if NSGPO>0 and K=1 if NSGPO is not provided. For a subgroup index iSG, 0≤iSG<K, a UE determines a value for the iPO⋅K+iSG bit in the paging indication field, where iPO=UE_IDmodN⋅NS+iSmodNPOPEI is a paging occasion index, and UE_ID, N, NS, iSG, and iS are defined in [17, TS 38.304]. When the value is '1', the UE monitors a paging occasion determined according to [17, TS 38.304]; otherwise, the UE is not required to monitor the paging occasion. <end TP2> <begin TP3 for 38.212-h10, subclause 7.3.1.3.8> The following information is transmitted by means of the DCI format 2_7 with CRC scrambled by PEI-RNTI: - Paging indication field – NPOPEINSGPO bit(s), where - NPOPEI is the number of paging occasions configured by higher layer parameter PONumPerPEI as defined in Clause 10.4A in [5, TS 38.213]; - NSGPOis the number of sub-groups of a paging occasion configured by higher layer parameter subgroupsNumPerPO, if subgroupsNumPerPO is configured; otherwise NSGPO is set to 1. - Each bit in the field indicates one UE subgroup of a paging occasion if subgroupsNumPerPO is configured; otherwise each bit in the field indicates the UE group of a paging occasion. - TRS availability indication – 1, 2, 3, 4, 5, or 6 bits if TRS-ResourceSetConfig is configured; 0 bits otherwise. <end TP3> |
| Qualcomm |  |

Given the necessity and the consistent proposals, moderator suggests the following proposal for companies to decide wither to discuss it in the following email discussion.

Proposal 2.1-1: For alignment with RAN2 agreements, adopt the following TPs for TS 38.212 and TS 38.213, respectively

============================ start of TP========================================== ## <Unchanged text omitted> ## 7.3.1.3.8 Format 2_7 DCI format 2_7 is used for notifying the paging early indication and TRS availability indication for one or more UEs. The following information is transmitted by means of the DCI format 2_7 with CRC scrambled by PEI-RNTI: - Paging indication field – NPOPEINSGPO bit(s), where - NPOPEI is the number of paging occasions configured by higher layer parameter PONumPerPEI as defined in Clause 10.4A in [5, TS 38.213]; - NSGPOis the number of sub-groups of a paging occasion configured by higher layer parameter subgroupsNumPerPO, if subgroupsNumPerPO is configured; otherwise NSGPO is set to 1. - Each bit in the field indicates one UE subgroup of a paging occasion if subgroupsNumPerPO is greater than 1configured; otherwise each bit in the field indicates the UE group of a paging occasion. - TRS availability indication – 1, 2, 3, 4, 5, or 6 bits if TRS-ResourceSetConfig is configured; 0 bits otherwise. The size of DCI format 2_7 is indicated by the higher layer parameter payloadSizeDCI_format2_7, according to Clause 10.4A of [5, TS 38.213]. The number of information bits in format 2_7 shall be equal to or less than the payload size of format 2_7. If the number of information bits in format 2_7 is less than the size of format 2_7, the remaining bits are reserved. ## <Unchanged text omitted> ============================= end of TP========================================== ============================ start of TP========================================== ## 10.4A PDCCH monitoring for early indication of paging ## <Unchanged text omitted> A paging indication field of DCI format 2_7 includes NPOPEI segments of K bits, where K=NSGPO if NSGPO>0 and K=1 if NSGPO is not provided. For a subgroup index iSG, 0≤iSG<K, a UE determines a value for the iPO⋅K+iSG bit in the paging indication field, where iPO=UE_IDmodN⋅NS+iSmodNPOPEI is a paging occasion index, and UE_ID, N, NS, iSG, and iS are defined in [17, TS 38.304]. When the value is '1', the UE monitors a paging occasion determined according to [17, TS 38.304]; otherwise, the UE is not required to monitor the paging occasion. If NPOPEI<NS, the number of symbols from the start of the frame to the start of the first PDCCH monitoring occasion for DCI format 2_7 that is associated with paging occasion index iPO is the iSNPOPEI+1-th value from the NSNPOPEI values provided by firstPDCCH-MonitoringOccasionOfPEI-O. ## <Unchanged text omitted> ============================= end of TP==========================================

Companies please provide your views on whether to discuss Proposal 2.1-1 in RAN1#109-e email discussion

Table 1: Companies’ views on whether to discuss Proposal 2.1-1 in the above

| Company name | Whether to discuss the proposal (Y/N)? | Additional comment(s), if available |
| --- | --- | --- |
| Nordic | Y | “greater than 1” is not needed |
| MediaTek | Y | Support to discuss the TPs |
| Nokia | Y |  |
| ZTE, Sanechips | Y |  |
| LGE | Y |  |
| Spreadtrum | Y |  |
| CATT | Y |  |
| vivo | Y | Support to solve the inconsistence with RAN2 spec. |
| Qualcomm | Y |  |
| Huawei, HiSilicon | Y |  |
| Intel | Y |  |
| Ericsson | Y |  |
| Apple | Y |  |
| Ratio of Y | 100% (13/13) | Moderator: Recommended to be 1st priority topic |

## Whether/how to clarify joint PDCCH monitoring behaviour when both SDT and PEI are applied

In RAN1#108-e meeting, there is discussion on whether/how to include TP for clarifying UE monitoring behaviour when both SDT and PEI are applied. The necessity depends on whether the same CSS is utilized for the PDCCH of STD and PEI. In the following table, there collect companies’ related views:

| Company name | Company’s views/comments |
| --- | --- |
| Huawei, HiSilicon | … it is unnecessary to introduce additional restriction on the monitoring occasions for PDCCH-candidates in a Type2A-PDCCH CSS set, and we don’t see any necessity of clarification on current specification, which seems to be nothing to do with paging enhancements in Rel-17. Observation 1: No further clarification on monitoring occasion determination of PEI is needed. |
| Spreadtrum |  |
| CATT |  |
| vivo |  |
| ZTE, Sanechips |  |
| xiaomi |  |
| Samsung |  |
| Panasonic |  |
| Sharp |  |
| Apple |  |
| Nokia | Observation: It does not appear necessary to add reference to Type2A-PCCCH CSS to the text describing the UE behaviour when is configured with C-RNTI and is provided with SS ID=0 for searchSpaceID in PDCCH-ConfigCommon text in 38.213. Proposal: Reference to Type2A-PCCCH CSS is not added to the text describing the UE behaviour when is configured with C-RNTI and is provided with SS ID=0 for searchSpaceID in PDCCH-ConfigCommon text in 38.213. Observation: It does not appear necessary to add reference to Type2A-PCCCH CSS to the text describing the UE behaviour when is configured with C-RNTI and is provided non-zero SS ID for searchSpaceID in PDCCH-ConfigCommon text in 38.213. Proposal: Reference to Type2A-PCCCH CSS is not added to the text describing the UE behaviour when is configured with C-RNTI and is provided non-zero SS ID for searchSpaceID in PDCCH-ConfigCommon text in 38.213. |
| LGE |  |
| MediaTek | Observation 1: The common search space sets for SDT and PEI are different. Conclusion 1: No TP is needed for clarifying the joint PDCCH monitoring for both SDT and PEI. |
| Nordic Semi. |  |
| Intel |  |
| Ericsson |  |
| Qualcomm |  |

From companies’ feedback, moderator would like to suggest the following conclusion for companies to decide whether to discuss it further:

Conclusion 2.2-1: Reference to Type2A-PCCCH CSS is not added to the text describing the UE behaviour when is configured with C-RNTI.

Companies please provide your views on whether to discuss Conclusion 2.2-1 in RAN1#109-e email discussion

Table 2: Companies’ views on whether to discuss Conclusion 2.2-1 in the above

| Company name | Whether to discuss the conclusion (Y/N)? | Additional comment(s), if available |
| --- | --- | --- |
| Nordic | Y | Agree with proposal |
| MediaTek | Y | Support to discuss and also agree with the proposal |
| Nokia | Y | Alternatively, not discussing results also no change, which is also fine, but probably it is good to conclude something. |
| ZTE, Sanechips |  | Agree with Nokia. But also okay to clarify. |
| LGE | Y | We support the proposal. |
| Spreadtrum |  | OK for clarification |
| CATT | Y | This issue needs to be clarified when UE receives paging information, such as PTWS, with C-RNTI. |
| Qualcomm |  | The proposal suggests no change to spec. Then there is no need to discuss the issue. |
| Huawei, HiSilicon |  | Agree with Nokia, ZTE and Qualcomm. It seems there is no need to discuss the issue considering the conclusion is there is no spec impact. |
| intel |  | Proposal suggests no specification impact. So this issue can be dropped. |
| Ericsson | Y | OK to discuss. |
| Apple | Y | OK to discuss |
| Ratio of Y | 58% (7/12) | Moderator: Recommended to be 2nd priority topic |

## Minimum time gap between PEI-O and PO

In the following table, there collect companies’ views related to minimum time gap between PEI-O and PO:

| Company name | Company’s views/comments |
| --- | --- |
| Huawei, HiSilicon |  |
| Spreadtrum | Proposal 1: The positive minimum gap between SSB and the associated PEI-PDCCH or paging PDCCH can be defined or concluded as the information/reference for network configuring PEI. Proposal 2: The minimum gap between SSB and the associated PEI-PDCCH or paging PDCCH reuses the minimum time gap value 1 for DCI format 2_6, i.e. 1/1/1/2 slots for 15/30/60/120kHz respectively. |
| CATT |  |
| vivo |  |
| ZTE, Sanechips |  |
| xiaomi |  |
| Samsung |  |
| Panasonic |  |
| Sharp |  |
| Apple |  |
| Nokia | Observation: Minimum time gap between PO and PEI should enable UE to acquire synchronisation for paging message reception. Observation: From receiver preparation perspective minimum gap of 3ms could be considered. |
| LGE |  |
| MediaTek |  |
| Nordic Semi. |  |
| Intel |  |
| Ericsson |  |
| Qualcomm |  |

From companies’ feedback, moderator would like to suggest the following proposal for companies to decide whether to discuss it in the upcoming email discussion:

Proposal 2.3-1: Minimum time gap between PEI-O and PO is supported. Decide the value from one of the following two alternatives:

- Alt 1: 1/1/1/2 slots for 15/30/60/120kHz respectively

- Alt 2: 3 ms

Companies please provide your views on whether to discuss Proposal 2.3-1 in RAN1#109-e email discussion

Table 3: Companies’ views on whether to discuss Proposal 2.3-1 in the above

| Company name | Whether to discuss the proposal (Y/N)? | Additional comment(s), if available |
| --- | --- | --- |
| Nordic | N | This is not essential, since it was agreed that UE may fall back to PO monitoring if gNB configuration is bad. |
| MediaTek | Y | Either of the time gap requirement is small and can ensure minimum impact to system flexibility. It is also useful to set such requirement so that network can avoid bad configuration that cannot effectively utilize PEI resource for UE power saving. Note: Without such gap, idle/inactive UE supporting PEI can be required a shorter warm-up time than a connected-mode UE supporting DCP, while we somehow expect the idle/inactive UE should be always more power saving. |
| ZTE, Sanechips | N | Whether to monitor PEI depends on UE implementation, without any indication to NW. No more discussion is needed at this late phase. |
| LGE | N | Similar view with Nordic and ZTE. |
| Spreadtrum | Y | It is useful for gNB to configure proper PEI |
| CATT | N | This is not an essential correction. |
| vivo | Y partially | We agree to discuss the minimum time gap between PEI-O and PO. However, if we genuinely want to open the discussion, from our understanding, it will be more significant to consider another alternative i.e., Alt 3: the minimum time gap should contain M SSB, where the value of M can be 1, 2, 3 etc. Since from power saving perspective, if no SSB contained within the minimum offset/gap between the PEI-O and PO, there will be no power saving gain by configuring PEI. And if the intention of Proposal 2.3-1 is only for aligning with the minimum time gap defined for DCI format 2_6, then we think it is not necessary considering the agreed conclusion i.e., it is up to UE implementation whether the UE monitors the MO(s) for a PEI. |
| Huawei, HiSilicon | N | There is already a conclusion in the previous meetings that UE can decide whether to monitor PEI occasions up to UE implementation. It seems this is not essential considering this conclusion in CR phase. |
| Intel | N | Not essential. Current procedure is fine. |
| Ericsson | N | This is not essential correction. |
| Apple | N | This is not essential as the UE can determine whether to monitor PEI or not autonomously. |
| Ratio of Y | 27% (3/11) | Moderator: Not recommended for discussion due to lack of support/consensus |

## Additional Clarifications and Others

In the following, there collect “single-company” proposals for additional clarification or other purposes not included in the above subsections.

| Company name | Company’s views/comments |
| --- | --- |
| Huawei, HiSilicon |  |
| Spreadtrum |  |
| CATT | For a UE supporting sidelink, the UE does not expect to detect different DCIs for scheduling respective PDSCH and PSSCH in a same PDCCH monitoring occasion. Following the same principle, the UE also does not expect to detect PDCCH-based PEI in the PDCCH monitoring occasion for PSSCH scheduling. Proposal 2: Adopt the following TP in TS 38.213. ============================ start of TP========================================== ## 10.1 UE procedure for determining physical downlink control channel assignment ## <Unchanged text omitted> A UE does not expect to detect, in a same PDCCH monitoring occasion, a DCI format with CRC scrambled by a SI-RNTI, RA-RNTI, MsgB-RNTI, TC-RNTI, P-RNTI, PEI-RNTI, C-RNTI, CS-RNTI, MCS-RNTI, MCCH-RNTI, G-RNTI, or G-CS-RNTI and a DCI format with CRC scrambled by a SL-RNTI or a SL-CS-RNTI for scheduling respective PDSCH reception and PSSCH transmission on a same serving cell. ## <Unchanged text omitted> ============================= end of TP==========================================. … In RAN1#107bis, it is agreed to use PEI-F_offset with the range of {0,1,2,…,16} to determine the frame of PEI. So the definition of PEI-F_offset in TS 38.213 should be further modified as the description in TP. Proposal 1: Adopt the following TP in TS 38.213. ============================= start of TP========================================== ## 10.4A PDCCH monitoring for early indication of paging A UE can be provided the following for detection of a DCI format 2_7 in RRC_IDLE state or in RRC_INACTIVE state [12, TS 38.331] - a search space set, by peiSearchSpace, to monitor PDCCH for detection of DCI format 2_7 according to a Type2A-PDCCH CSS set as described in clause 10.1 - a number of frames, by PEI-F_offset, from the start of a frame to the start of a first paging frame of paging frames associated with a number of PDCCH monitoring occasions for DCI format 2_7 [17, TS 38.304] to the start of a frame - a number of symbols, by firstPDCCH-MonitoringOccasionOfPEI-O, from the start of the frame to the start of the first PDCCH monitoring occasion for DCI format 2_7 - a size, by payloadSizeDCI_format2_7 - a number of subgroups per paging occasion, NSGPO, by subgroupsNumPerPO - a number of paging occasions associated with the number of PDCCH monitoring occasions for DCI format 2_7, NPOPEI, by PONumPerPEI ## <Unchanged text omitted> ============================= end of TP==========================================. … when searchSpaceID is configured with a zero value for a Type0/0A/2-PDCCH CSS, the determination of monitoring occasions for PDCCH candidates as described in clause 13 is limited to CONNECTED UE in TS 38.213-17.1.0. The whole sentence is not clear with 3 main sentences without proper conjunction and leads to the lack of relevant description for IDLE/INACTIVE UE. Thus, when Type0/0A/2-PDCCH CSS is SS#0, the following TP is to capture the same PDCCH monitoring occasions determination based on SearchSpaceZero for both CONNECTED and IDLE/INACTIVE UE with additional condition for CONNECTED UE. Proposal 3: Adopt the following TP in TS 38.213. ============================ start of TP========================================== ## 10.1 UE procedure for determining physical downlink control channel assignment ## <Unchanged text omitted> If a UE is provided a zero value for searchSpaceID in PDCCH-ConfigCommon for a Type0/0A/2-PDCCH CSS set, or is not provided searchSpaceBroadcast, the UE determines monitoring occasions for PDCCH candidates of the Type0/0A/2-PDCCH CSS set as described in clause 13, and if the UE is provided a C-RNTI, the UE monitors PDCCH candidates only at monitoring occasions associated with a SS/PBCH block, where the SS/PBCH block is determined by the most recent of - a MAC CE activation command indicating a TCI state of the active BWP that includes a CORESET with index 0, as described in [6, TS 38.214], where the TCI-state includes a CSI-RS which is quasi-co-located with the SS/PBCH block, or - a random access procedure that is not initiated by a PDCCH order that triggers a contention-free random access procedure ## <Unchanged text omitted> ============================= end of TP==========================================. |
| vivo | Based on TS 38.304 [4], the following clarification is given for paging reception in multi-beam operations. In multi-beam operations, the UE assumes that the same paging message and the same Short Message are repeated in all transmitted beams and thus the selection of the beam(s) for the reception of the paging message and Short Message is up to UE implementation. The paging message is same for both RAN initiated paging and CN initiated paging.. Similarly, it is natural to make a clarification for PEI reception in multi-beam operations since the very similar configuration for PEI occasion and paging occasion i.e., same as that of PO, “A PEI occasion (PEI-O) is a set of PDCCH monitoring occasions (MOs) and can consist of multiple time slots (e.g. subframe or OFDM symbol) where PEI can be sent (TS 38.213 [4]).” To avoid unnecessary ambiguity, we suggest to include the following description for PEI reception in multi-beam operations. Proposal 2: In multi-beam operations, the UE assumes that the same PEI is repeated in all transmitted beams and thus the selection of the beam(s) for the reception of the PEI is up to UE implementation. |
| ZTE, Sanechips |  |
| xiaomi |  |
| Samsung |  |
| Panasonic |  |
| Sharp |  |
| Apple |  |
| Nokia |  |
| LGE |  |
| MediaTek |  |
| Nordic Semi. |  |
| Intel |  |
| Ericsson | When SearchSpace ID is non-zero, the ALs/BDs for PEI are configured by the corresponding search space configuration. In case search space zero is used for PEI, RAN1 spec update is needed for identifying the number of BDs/Als for PEI. It can simply follow the legacy SearchSpaceZero BDs/ALs, i.e. fixed values specified (as in Table 10.1-1 of 38.213). Below proposal is made to reflect this. Proposal 1: Adopt TP1 for 38.213-h10, subclause 10.1 to reflect BDs/CCEs when searchSpaceZero is used for PEI. <begin TP1 for 38.213-h10, subclause 10.1> If a UE is not provided peiSearchSpace for Type2A-PDCCH CSS set, the UE does not monitor PDCCH for Type2A-PDCCH CSS set on the DL BWP. The CCE aggregation levels and the maximum number of PDCCH candidates per CCE aggregation level for Type2A-PDCCH CSS set are given in Table 10.1-1 when the UE is provided a non-zero value for searchSpaceID for the Type2A-PDCCH CSS set. If the UE is provided peiSearchSpace with zero value for the Type2A-PDCCH CSS set index, and for the SS/PBCH block and CORESET multiplexing patterns 2 and 3, the UE determines PDCCH monitoring occasions as described in clause 13 and the CCE aggregation levels and the number of PDCCH candidates per CCE aggregation level are given in Table 10.1-1. <end TP1> |
| Qualcomm |  |

Companies please provide your views on whether to discuss any of the above proposals, and please specify which proposal(s) is identified:

Table 4: Companies’ views on whether to discuss any proposal in the above table

| Company name | Whether to discuss any proposal (Y/N)? | Which proposal to discuss, if identified |
| --- | --- | --- |
| Nordic | Ericsson and CATT proposal could be discussed |  |
| MediaTek | Y | Support to discuss Proposal 2 by CATT and the proposal 1 by Ericsson. Not support discussing others. |
| Spreadtrum | Y | We would like to discuss “the minimum gap between SSB and the associated PEI-PDCCH” which is useful for gNB to configure proper PEI-PDCCH |
| CATT | Y | CATT and Ericsson’s proposals |
| Huawei, HiSilicon |  | CATT’s proposal 1 and Ericsson’s proposal seems OK to be discussed. |
| Intel | Y | CATT and E/// proposals |
| Ericsson | Y | CATT and Ericsson proposals. |
| Apple | Y | (Moderator: No information for being counted in the statistics below) |
| Ratio of Y for Ericsson proposal 1 | 85% (6/7) | Moderator: Recommended to be 1st priority topic (as Proposal 2.4-1) |
| Ratio of Y for CATT proposal 1 | 71% (5/7) | Moderator: Recommended to be 2nd priority topic (as Proposal 2.4-2) |
| Ratio of Y for CATT proposal 2 | 71% (5/7) | Moderator: Recommended to be 2nd priority topic (as Proposal 2.4-3) |
| Ratio of Y for CATT proposal 3 | 57% (4/7) | Moderator: Recommended to be 2nd priority topic (as Proposal 2.4-4) |

# Preparation for Maintenance on TRS Information for Idle/Inactive UEs

## Clarification on the reference point for TRS availability indication

In RAN1#108-e meeting, there is one remaining issue:

Agreement Down-select one of the TPs to clarify the current default DRX cycle used for determining the reference point for start of validity duration for an availability indication - Alt1: Adopt TP#1 - Alt2: Adopt TP#2 - Note: TRS availability indication transmitted in all PDCCH monitoring occasions of the same PEI-O or PO is same. - Note: Reference point for TRS availability indication in a given DCI is identical for all UEs regardless of UE specific DRX value. ============================= start of TP#1========================================== ## 10.4B Indication of TRS resources ## <Unchanged text omitted> A value of '1' for a bit of the bitmap indicates presence of associated TRS resource sets for the multiple of the number of frames, starting from a SFN determined from SFN+PF_offsetmodT=0 [17, TS 38.304] that corresponds to the frame within the DRX cycle that includes a the PDCCH providing the DCI format 2_7, or the DCI format 1_0 with CRC scrambled by P-RNTI, with the TRS availability indication field indicating the TRS resource sets, where T is provided by defaultPagingCycle. A value of '0' for a bit of the bitmap is ignored by the UE. A UE can receive first and second PDCCHs that provide DCI format 2_7 or DCI format 1_0 with CRC scrambled by P-RNTI that indicate presence of TRS resource sets for the multiple of the number of frames, where the second PDCCH reception after the first PDCCH reception by a time that is smaller than the multiple of the number of frames. ## <Unchanged text omitted> ============================= end of TP#1========================================== ============================= start of TP#2========================================== ## 10.4B Indication of TRS resources ## <Unchanged text omitted> A value of '1' for a bit of the bitmap indicates presence of associated TRS resource sets for the multiple of the number of frames, starting from a SFN determined from SFN+PF_offsetmodT=0 [17, TS 38.304] that corresponds to the frame of the DRX cycle associated with the first PDCCH MO of the PEI-O or PO that includes a the PDCCH providing the DCI format 2_7, or the DCI format 1_0 with CRC scrambled by P-RNTI, with the TRS availability indication field indicating the TRS resource sets, where T is provided by defaultPagingCycle. A value of '0' for a bit of the bitmap is ignored by the UE. A UE can receive first and second PDCCHs that provide DCI format 2_7 or DCI format 1_0 with CRC scrambled by P-RNTI that indicate presence of TRS resource sets for the multiple of the number of frames, where the second PDCCH reception after the first PDCCH reception by a time that is smaller than the multiple of the number of frames. ## <Unchanged text omitted> ============================= end of TP#2==========================================.

In the following table, companies’ views are collected:

| Company name | Company’s views/comments |
| --- | --- |
| Huawei, HiSilicon | Proposal 1: Adopt TP#2 of the agreements on reference point made in RAN1 #108-e to define the reference point of validity duration based on the first MO of a PO/PEI-O where UE receives the availability indication. |
| Spreadtrum |  |
| CATT | The PO/PEI-O is located within a single default DRX cycle. The frame of PEI-O should belong to the same DRX cycle as that of PO regardless the actual location of PEI-O frame when PEI-O spanning cross two DRX cycles [2]. There is no ambiguity of reference point for the TRS indication between gNB and UE. Proposal 5: TP#1 should be adopted to determine the reference point of validity duration for availability indication. =========================== start of TP#1========================================== ## 10.4B Indication of TRS resources ## <Unchanged text omitted> A value of '1' for a bit of the bitmap indicates presence of associated TRS resource sets for the multiple of the number of frames, starting from a SFN determined from SFN+PF_offsetmodT=0 [17, TS 38.304] that corresponds to the frame within the DRX cycle that includes a the PDCCH providing the DCI format 2_7, or the DCI format 1_0 with CRC scrambled by P-RNTI, with the TRS availability indication field indicating the TRS resource sets, where T is provided by defaultPagingCycle. A value of '0' for a bit of the bitmap is ignored by the UE. A UE can receive first and second PDCCHs that provide DCI format 2_7 or DCI format 1_0 with CRC scrambled by P-RNTI that indicate presence of TRS resource sets for the multiple of the number of frames, where the second PDCCH reception after the first PDCCH reception by a time that is smaller than the multiple of the number of frames. ## <Unchanged text omitted> =========================== end of TP#1==========================================. |
| vivo |  |
| ZTE, Sanechips | - Observation 1: For PEI based availability indication, the reference time of the start of validity duration has no ambiguity issue according to current specification. - Proposal 2: TP#1 should be adopted to clarify the current default DRX cycle used for determining the reference point for start of validity duration for an availability indication. |
| xiaomi |  |
| Samsung | Proposal 1: Down-select Alt1/TP#1 to clarify the current default DRX cycle used for determining the reference point for start of validity duration for an availability indication. |
| Panasonic | Proposal 2: Adopt the TP#1 |
| Sharp | Proposal 1: support TP#2 as listed for down-selection in RAN1-108-e meeting to determine the reference point for the start of the validity duration |
| Apple | Proposal: For determining the reference point for start of validity duration for an TRS availability indication, adopt Alt 1/TP#1. |
| Nokia | Proposal: Adopt TP#1 for clarifying the reference point definition for current default DRX cycle. |
| LGE | Proposal 1: Adopt TP#2 for determining the reference point for start of TRS validity duration for an availability indication. |
| MediaTek | Proposal 2: According to the agreement for determining the reference point for the indicated validity duration, adopt TP#2 to resolve the ambiguity issue when PO or PEI-O spreads over two frames that correspond to different paging cycles. |
| Nordic Semi. | Proposal-1: Adopt TP#2 as clarification for the iTRS validity reference point. |
| Intel | Observation 1: There is no critical need to change the existing agreement regarding reference point definition. Proposal 1: Support TP # 1 for clarification of current default DRX cycle. |
| Ericsson | Proposal 3: For clarifying the current DRX cycle for TRS occasions reference point, adopt TP alt#2 from RAN1#108-e agreement. |
| Qualcomm |  |

From companies’ inputs, the following statistics can be identified:

TP#1 (7 companies): CATT, ZTE, Samsung, Panasonic, Apple, Nokia, Intel

TP#2 (6 companies): HW, Sharp, LGE, MTK, Nordic, Ericsson

By the above, moderator suggest the following proposal as starting point for companies to decide whether to discuss it further in upcoming email discussion:

Proposal 3.1-1: To clarify reference point of TRS availability indication, adopt the following TP to TS 38.213

============================= start of TP#1========================================== ## 10.4B Indication of TRS resources ## <Unchanged text omitted> A value of '1' for a bit of the bitmap indicates presence of associated TRS resource sets for the multiple of the number of frames, starting from a SFN determined from SFN+PF_offsetmodT=0 [17, TS 38.304] that corresponds to the frame within the DRX cycle that includes a the PDCCH providing the DCI format 2_7, or the DCI format 1_0 with CRC scrambled by P-RNTI, with the TRS availability indication field indicating the TRS resource sets, where T is provided by defaultPagingCycle. A value of '0' for a bit of the bitmap is ignored by the UE. A UE can receive first and second PDCCHs that provide DCI format 2_7 or DCI format 1_0 with CRC scrambled by P-RNTI that indicate presence of TRS resource sets for the multiple of the number of frames, where the second PDCCH reception after the first PDCCH reception by a time that is smaller than the multiple of the number of frames. ## <Unchanged text omitted> ============================= end of TP#1==========================================

Companies please provide your views on whether to discuss Proposal 3.1-1 in RAN1#109-e email discussion

Table 5: Companies’ views on whether to discuss Proposal 3.1-1 in the above

| Company name | Whether to discuss the proposal (Y/N)? | Additional comment(s), if available |
| --- | --- | --- |
| Nordic | Y | Current proposal not our preference, but will not object |
| MediaTek | Y | Support to discuss the proposal (although current version is not our preference) |
| Nokia | Y |  |
| ZTE, Sanechips | Y |  |
| LGE | Y | Fine with discussion, but current proposal is not our preference. |
| Spreadtrum | Y |  |
| CATT | Y |  |
| vivo | Y | Support Proposal 3.1-1. |
| Qualcomm | Y |  |
| Huawei, HiSilicon | Y | The issue should be discussed, but the two TPs should be both considered as starting point, considering almost the same number of support. We prefer the TP#2. |
| Intel | Y |  |
| Apple | Y |  |
| Ratio of Y | 100% (12/12) | Moderator: Recommended to be 1st priority topic |

## How do CONNECTED UEs handle the TRS resources

Agreement - TRS-ResourceSetConfig IE is not accounted in determination of available resource elements for PDSCH.

In the following table, there collect companies’ proposal for capturing the above agreement in the specification:

| Company name | Company’s views/comments |
| --- | --- |
| Huawei, HiSilicon | Proposal 2: Adopt the following TP2 not to impact connected mode UE’s rate matching behavior due to TRS resource set(s) configured in SIB. ------------------------------------------ Start of Text Proposal 2------------------------------------------ < Unchanged parts are omitted > #### 7.3.1.5 Mapping to virtual resource blocks The UE shall, for each of the antenna ports used for transmission of the physical channel, assume the block of complex-valued symbols yp0, …, yp(Msymbap-1) conform to the downlink power allocation specified in [6, TS 38.214] and are mapped in sequence starting with yp0 to resource elements k',lp,μ in the virtual resource blocks assigned for transmission which meet all of the following criteria: - they are in the virtual resource blocks assigned for transmission; - the corresponding physical resource blocks are declared as available for PDSCH according to clause 5.1.4 of [6, TS 38.214]; - the corresponding resource elements in the corresponding physical resource blocks are - not used for transmission of the associated DM-RS or DM-RS intended for other co-scheduled UEs as described in clause 7.4.1.1.2; - not used for non-zero-power CSI-RS according to clause 7.4.1.5 if the corresponding physical resource blocks are for a PDSCH scheduled by a PDCCH with the CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, G-RNTI, G-CS-RNTI, MCCH-RNTI, or a PDSCH with SPS, except if the non-zero-power CSI-RS is a CSI-RS configured by the higher-layer parameter CSI-RS-Resource-Mobility in the MeasObjectNR IE or except if the non-zero-power CSI-RS is an aperiodic non-zero-power CSI-RS resource or except if the non-zero-power CSI-RS is TRS resources configured via [TRS-ResourceSetConfig]; - not used for PT-RS according to clause 7.4.1.2; - not declared as 'not available for PDSCH according to clause 5.1.4 of [6, TS 38.214]. The mapping to resource elements (k',l)p,μ allocated for PDSCH according to [6, TS 38.214] and not reserved for other purposes shall be in increasing order of first the index k' over the assigned virtual resource blocks, where k'=0 is the first subcarrier in the lowest-numbered virtual resource block assigned for transmission, and then the index l. ------------------------------------------ End of Text Proposal 2------------------------------------------. |
| Spreadtrum |  |
| CATT | However, the TRS-ResourceSetConfig for IDLE/Inactive UE is not part of non-zero-power CSI-RS which is configured by NZP-CSI-RS-ResourceSet IE. Even CONNECTED mode UE had to read the SIB occasionally, it would not store the TRS configuration for IDLE/Inactive UE from SIB-X and obtain the L1 signalling indication of TRS availability indication from DCI format 1_0 or 2_7. It is not needed to include TRS resources configured via TRS-ResourceSetConfig into non-zero-power CSI-RS resources. Thus, no TP is needed to include the exception case for TRS-ResourceSetConfig in section 7.3.1.5 of TS 38.211. Observation1: It is not needed to include TRS resources configured via TRS-ResourceSetConfig into non-zero-power CSI-RS resources configured by NZP-CSI-RS-ResourceSet. |
| vivo |  |
| ZTE, Sanechips |  |
| xiaomi |  |
| Samsung |  |
| Panasonic |  |
| Sharp | Proposal 2: adopt TP1 in this document for PDSCH power control ---------------------<begin TP1 for TS 38.214>-------------------------------------------------------------------- 4.1 Power allocation for downlink <omit unchanged text> For downlink DM-RS associated with PDSCH, the UE may assume the ratio of PDSCH EPRE to DM-RS EPRE ( [dB]) is given by Table 4.1-1 according to the number of DM-RS CDM groups without data as described in Clause 5.1.6.2 except when the REs are occupied by resources configured by the TRS-ResourceSetConfig for idle/inactive UE where the PDSCH EPRE is 0 . The DM-RS scaling factor specified in Clause 7.4.1.1.2 of [4, TS 38.211] is given by . -----------------<end TP1>------------------------------------------------------------------------------- |
| Apple |  |
| Nokia | Proposal: TRS-ResourceSetConfig IE does not affect the UE behaviour in CONNECTED mode and can be ignored by UE in CONNECTED mode. If it is seen necessary to have the rate matching behaviour specifically accounted, following TP could be considered: #### 7.3.1.5 Mapping to virtual resource blocks The UE shall, for each of the antenna ports used for transmission of the physical channel, assume the block of complex-valued symbols yp0, …, yp(Msymbap-1) conform to the downlink power allocation specified in [6, TS 38.214] and are mapped in sequence starting with yp0 to resource elements k',lp,μ in the virtual resource blocks assigned for transmission which meet all of the following criteria: - they are in the virtual resource blocks assigned for transmission; - the corresponding physical resource blocks are declared as available for PDSCH according to clause 5.1.4 of [6, TS 38.214]; - the corresponding resource elements in the corresponding physical resource blocks are - not used for transmission of the associated DM-RS or DM-RS intended for other co-scheduled UEs as described in clause 7.4.1.1.2; - not used for non-zero-power CSI-RS according to clause 7.4.1.5 if the corresponding physical resource blocks are for a PDSCH scheduled by a PDCCH with the CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, G-RNTI for multicast, G-CS-RNTI, or a PDSCH with SPS, except if the non-zero-power CSI-RS is a CSI-RS configured by the higher-layer parameter CSI-RS-Resource-Mobility in the MeasObjectNR IE or except if the non-zero-power CSI-RS is an aperiodic non-zero-power CSI-RS resource or except if the non-zero-power CSI-RS is TRS resources configured only via TRS-ResourceSetConfig in SIB; - not used for PT-RS according to clause 7.4.1.2; - not declared as 'not available for PDSCH according to clause 5.1.4 of [6, TS 38.214].. |
| LGE |  |
| MediaTek |  |
| Nordic Semi. | Porposal-2: Adopt TP for TS38.211 sub-clause 7.3.1.5 to clarify idle TRS rate-matching. #### 7.3.1.5 Mapping to virtual resource blocks The UE shall, for each of the antenna ports used for transmission of the physical channel, assume the block of complex-valued symbols yp0, …, yp(Msymbap-1) conform to the downlink power allocation specified in [6, TS 38.214] and are mapped in sequence starting with yp0 to resource elements k',lp,μ in the virtual resource blocks assigned for transmission which meet all of the following criteria: - they are in the virtual resource blocks assigned for transmission; - the corresponding physical resource blocks are declared as available for PDSCH according to clause 5.1.4 of [6, TS 38.214]; - the corresponding resource elements in the corresponding physical resource blocks are - not used for transmission of the associated DM-RS or DM-RS intended for other co-scheduled UEs as described in clause 7.4.1.1.2; - not used for non-zero-power CSI-RS according to clause 7.4.1.5 if the corresponding physical resource blocks are for a PDSCH scheduled by a PDCCH with the CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, G-RNTI for multicast, G-CS-RNTI, or a PDSCH with SPS, except if the non-zero-power CSI-RS is a CSI-RS configured by the higher-layer parameter CSI-RS-Resource-Mobility in the MeasObjectNR IE, or CSI-RS configured by the higher-layer parameter TRS-ResourceSetConfig, or except if the non-zero-power CSI-RS is an aperiodic non-zero-power CSI-RS resource; - not used for PT-RS according to clause 7.4.1.2; - not declared as 'not available for PDSCH according to clause 5.1.4 of [6, TS 38.214]. The mapping to resource elements (k',l)p,μ allocated for PDSCH according to [6, TS 38.214] and not reserved for other purposes shall be in increasing order of first the index k' over the assigned virtual resource blocks, where k'=0 is the first subcarrier in the lowest-numbered virtual resource block assigned for transmission, and then the index l.. |
| Intel |  |
| Ericsson |  |
| Qualcomm |  |

By the above, moderator would like to suggest the following proposal for companies to check whether to discuss it further in the upcoming email discussion:

Proposal 3.2-1: Adopt the following TP to TS 38.211:

#### 7.3.1.5 Mapping to virtual resource blocks The UE shall, for each of the antenna ports used for transmission of the physical channel, assume the block of complex-valued symbols yp0, …, yp(Msymbap-1) conform to the downlink power allocation specified in [6, TS 38.214] and are mapped in sequence starting with yp0 to resource elements k',lp,μ in the virtual resource blocks assigned for transmission which meet all of the following criteria: - they are in the virtual resource blocks assigned for transmission; - the corresponding physical resource blocks are declared as available for PDSCH according to clause 5.1.4 of [6, TS 38.214]; - the corresponding resource elements in the corresponding physical resource blocks are - not used for transmission of the associated DM-RS or DM-RS intended for other co-scheduled UEs as described in clause 7.4.1.1.2; - not used for non-zero-power CSI-RS according to clause 7.4.1.5 if the corresponding physical resource blocks are for a PDSCH scheduled by a PDCCH with the CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, G-RNTI for multicast, G-CS-RNTI, or a PDSCH with SPS, except if the non-zero-power CSI-RS is a CSI-RS configured by the higher-layer parameter CSI-RS-Resource-Mobility in the MeasObjectNR IE or except if the non-zero-power CSI-RS is an aperiodic non-zero-power CSI-RS resource or except if the non-zero-power CSI-RS is TRS resources configured only via TRS-ResourceSetConfig in SIB; - not used for PT-RS according to clause 7.4.1.2; - not declared as 'not available for PDSCH according to clause 5.1.4 of [6, TS 38.214].

Companies please provide your views on whether to discuss Proposal 3.2-1 in RAN1#109-e email discussion

Table 6: Companies’ views on whether to discuss Proposal 3.2-1 in the above

| Company name | Whether to discuss the proposal (Y/N)? | Additional comment(s), if available |
| --- | --- | --- |
| Nordic | Y | If somehow iTRS resources/resource-sets can be linked in dedicated RRC then Nokia TP should be adopted, if such resources/resource-sets would be reconfigured by dedicated RRC, then we prefer our TP. |
| MediaTek | Y | Support to further discuss it. |
| Nokia | Y | We would also propose to agree: Proposal: TRS-ResourceSetConfig IE does not affect the UE behaviour in CONNECTED mode and can be ignored by UE in CONNECTED mode. |
| ZTE, Sanechips | Y |  |
| LGE | Y | We prefer Sharp’s proposal. |
| Spreadtrum | N | Connected-mode UE should ignore the additional TRS. It has large impact on the connected-mode UE for T/F tracking loop |
| CATT | N | CONNECTED mode UE does not read the SIB-X for TRS |
| vivo | Y |  |
| Qualcomm | Y |  |
| Huawei, HiSilicon | Y | The agreements was made in RAN1#108 and we should discuss how to capture it in the specification. |
| Intel | Y | Agree with Nokia’s view |
| Ericsson | Y |  |
| Apple | Y | We can fine to discuss, and we support Nokia’s proposal. |
| Ratio of Y | 85% (11/13) | Moderator: Recommended to be 1st priority topic |

## Maximum number of configurable TRS resource sets

Agreement in RAN1#107 For the maximum number of TRS resource sets configured by higher layer, X, - X = 64 - FFS: the number of configured TRS resource sets is not larger than the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1

In the following table, there collect companies’ proposals related to the above FFS:

| Company name | Company’s views/comments |
| --- | --- |
| Huawei, HiSilicon | Observation 4: It is beneficial to keep the maximum number of configurable TRS resource sets to 16 for FR1 to reduce UE memory size for low cost and power consumption, especially for RedCap UEs. Proposal 4: Number of configured TRS resource sets in SIB for idle/inactive UEs is not larger than 16 for FR1. |
| Spreadtrum |  |
| CATT |  |
| vivo |  |
| ZTE, Sanechips |  |
| xiaomi |  |
| Samsung |  |
| Panasonic |  |
| Sharp |  |
| Apple |  |
| Nokia |  |
| LGE |  |
| MediaTek |  |
| Nordic Semi. | Proposal-3: The maximum number of TRS resource sets per cell within a band is function of Lmax and upper-bounded by 64. |
| Intel |  |
| Ericsson |  |
| Qualcomm |  |

By the above, moderator would like to suggest the following proposal for companies to decide whether to discuss it further in the upcoming email discussion:

Proposal 3.3-1: Number of configured TRS resource sets in SIB for idle/inactive UEs is not larger than 16 for FR1.

Companies please provide your views on whether to discuss Proposal 3.3-1 in RAN1#109-e email discussion

Table 7: Companies’ views on whether to discuss Proposal 3.3-1 in the above

| Company name | Whether to discuss the proposal (Y/N)? | Additional comment(s), if available |
| --- | --- | --- |
| Nordic | Y |  |
| MediaTek | Y | The number limitation can make this feature more practical for FR1 deployment |
| ZTE, Sanechips | N |  |
| LGE | N |  |
| Spreadtrum | N | Not a critical issue. Idle/inactive UE only cares about the SSB and the corresponding additional TRS |
| CATT | N | This was discussed before. We don’t see the need for further discussion |
| vivo | Y |  |
| Qualcomm | Y | Ok to discuss, but agreed that this is not critical. |
| Huawei, HiSilicon | Y | Agree with MTK. |
| Intel | Y |  |
| Ericsson | N | This is not an essential correction. |
| Apple | Y | We support the proposal. |
| Ratio of Y | 58% (7/12) | Moderator: Recommended to be 2nd priority topic |

## Additional Clarification and Others

In the following, there collect “single-company” proposals for additional clarification or other purposes not included in the above subsections.

| Company name | Company’s views/comments |
| --- | --- |
| Huawei, HiSilicon | Proposal 5: Considering there is no explicit parameter to configure the TRS availability field size, the number of bits of TRS availability indication field is determined implicitly based on indBitID in all the configured TRS resource set(s) in SIB-X. Proposal 6: Adopt the TP3 to determine TRS availability indication field length. ------------------------------------- Start of Text Proposal 3------------------------------------------ < Unchanged parts are omitted > #### 7.3.1.2.1 Format 1_0 The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by P-RNTI: < Unchanged parts are omitted > - TRS availability indication – 1, 2, 3, 4, 5, or 6 bits according to indBitID(s) provided by the TRS-ResourceSetConfig if TRS-ResourceSetConfig is configured; 0 bits otherwise. - Reserved bits – (8 – M) bits for operation in a cell with shared spectrum channel access in frequency range 1 or for operation in a cell in frequency range 2-2; (6 – M) bits for operation in a cell without shared spectrum channel access, where the value of M is the number of bits for the field of 'TRS availability indication' as defined above. #### 7.3.1.3.8 Format 2_7 The following information is transmitted by means of the DCI format 2_7 with CRC scrambled by PEI-RNTI: < Unchanged parts are omitted > - TRS availability indication – 1, 2, 3, 4, 5, or 6 bits according to indBitID(s) provided by the TRS-ResourceSetConfig if TRS-ResourceSetConfig is; 0 bits otherwise. ------------------------------------- End of Text Proposal 3------------------------------------------. Observation 5: Allowing paging DCI with short message indicator of value ‘00’ to indicate TRS availability violates the motivation of introducing paging DCI based TRS availability indication and validity duration of TRS availability. Proposal 7: Paging DCI with short message indicator of value ‘00’ to indicate TRS availability is not allowed. Proposal 8: Adopt TP#4 to align with the agreements. ------------------------------------------ Start of Text Proposal 4------------------------------------------ < Unchanged parts are omitted > ## 10.4B Indication of TRS resources A UE in RRC_IDLE state or RRC_INACTIVE state can be provided by TRS-ResourceSetConfig a set of TRS occasions [6, TS 38.214]. If TRS-ResourceSetConfig is provided, a DCI format 2_7, if peiSearchSpace is provided, and a DCI format 1_0 with CRC scrambled by P-RNTI includes a TRS availability indication field [4, TS 38.212] that provides a bitmap to groups of TRS resource sets where the configuration of each TRS resource set includes an association to a bit of the bitmap. The UE can be additionally provided a multiple, by validityDuration, for a number of frames provided by defaultPagingCycle for TRS resource sets with indicated presence; if validityDuration is not provided, the multiple is equal to 2. A value of '1' for a bit of the bitmap indicates presence of TRS on the associated TRS resource sets for the multiple of the number of frames, starting from a SFN determined from SFN+PF_offsetmodT=0 [17, TS 38.304] that corresponds to the frame that includes a PDCCH providing the DCI format 2_7, or the DCI format 1_0 with CRC scrambled by P-RNTI, with the TRS availability indication field indicating the presence of TRS on the TRS resource sets, where T is provided by defaultPagingCycle. A value of '0' for a bit of the bitmap indicates no change to a current assumption for the availability, or unavailability if the UE is not in a validity duration, of the TRS on the associated TRS resource sets. A UE can receive first and second PDCCHs that provide DCI format 2_7 or DCI format 1_0 with CRC scrambled by P-RNTI that indicate presence of TRS resource sets for the multiple of the number of frames, where the second PDCCH reception after the first PDCCH reception by a time that is smaller than the multiple of the number of frames. ------------------------------------------ Start of Text Proposal 4------------------------------------------. |
| Spreadtrum |  |
| CATT |  |
| vivo |  |
| ZTE, Sanechips | In TS 38.331 [4], the “scrambling ID” for TRS resource set is provided by ‘scramblingID-Info’ in SIB12. Hence, the scramblingID in TS 38.214 should be modified to keep consistent with that in TS 38.331. Therefore, the TP for scrambling ID for TRS resource set in TS 38.214 should be adopted hereafter. TS 38.331 h00[4] scramblingID-Info One or more scrambling IDs are configured for a TRS resource set. If a common scrambling ID is configured, it applies to all the TRS resources within the TRS resource set. Otherwise, each TRS resource within the TRS resource set is provided with a scrambling ID. If the number of TRS resources for the TRS resource set is 2, scramblingIDperResourceListWith2-r17 is configured, while scramblingIDperResourceListWith4-r17 is configured for the case that the number of TRS resources for the TRS resource set is 4.. - Proposal 3: The following TP for scrambling ID for TRS resource set in TS 38.214 should be adopted. ......................................... Text Proposal for TS38.214.............................................................................. 5.1.6.1.1 CSI-RS for tracking *** Unchanged text is omitted *** Each NZP CSI-RS resource, defined in Clause 7.4.1.5.3 of [4, TS 38.211], is configured by the higher layer parameter TRS-ResourceSet with the following restrictions for a UE in RRC_IDLE or RRC_INACTIVE: *** Unchanged text is omitted *** - One or more scramblingIDs scrambling IDs where if a single scramblingID scrambling ID is configured by scramblingID-Info, it applies to all NZP-CSI-RS resources in the resource set, otherwise, each NZP-CSI-RS resource is provided with a scramblingID scrambling ID. *** Unchanged text is omitted *** ....................................... End of Text Proposal for TS38.214........................................................................ |
| xiaomi | Proposal 1: If TRS resource for idle/inactive UE is configured, and during the validity time, the TRS resource configuration is changed by SI update procedure, before UE detects a following L1 TRS availability indication, all TRS resources are assumed as unavailable. |
| Samsung |  |
| Panasonic | Proposal 1: The UE can assume the configured group(s) of TRS resource set(s) are unavailable in below cases: - C1: before UE receives L1 availability indication after camping on a cell - C2: the TRS resource configuration is changed by SI update procedure - C3: During the system modification period and before the new SIBs become valid |
| Sharp |  |
| Apple |  |
| Nokia | … the TRS configuration for IDLE/Inactive UEs is done by TRS-ResourceSet-r17 IE (in SIBx) and not via NZP-CSI-RS-Resource IE. Hence, the description in TS38.211 appears not to cover the definition of TRS configuration for IDLE/Inactive UEs. Hence, it could be considered to account this also in the Section 7.4.1.5 of TS38.211. Proposal: Adopt following TP to Section 7.4.1.5.1 of 38.211: ##### 7.4.1.5.1 General Zero-power (ZP) and non-zero-power (NZP) CSI-RS are defined - for a non-zero-power CSI-RS configured by the NZP-CSI-RS-Resource IE or by the CSI-RS-Resource-Mobility field in the CSI-RS-ResourceConfigMobility IE or by the TRS-ResourceSet IE, the sequence shall be generated according to clause 7.4.1.5.2 and mapped to resource elements according to clause 7.4.1.5.3 - for a zero-power CSI-RS configured by the ZP-CSI-RS-Resource IE, the UE shall assume that the resource elements defined in clause 7.4.1.5.3 are not used for PDSCH transmission subject to clause 5.1.4.2 of [6, TS 38.214]. The UE performs the same measurement/reception on channels/signals except PDSCH regardless of whether they collide with ZP CSI-RS or not.. as can be seen from the running CR for TS38.331 not all the parameters needed to complete the NZP-CSI-RS configuration are provided, but can be derived e.g. from TS38.214. To align TS38.211, following changes could be considered. Proposal: Adopt following TP to Section 7.4.1.5.3 of TS38.211: ##### 7.4.1.5.3 Mapping to physical resources [text omitted] The value of is given by the higher-layer parameter density in the CSI-RS-ResourceMapping IE or the CSI-RS-CellMobility IE and the number of ports X is given by the higher-layer parameter nrofPorts. For NZP CSI-RS configured by the TRS-ResourceSet IE, the density =3 and number of ports X =1. The UE is not expected to receive CSI-RS and DM-RS on the same resource elements. The UE shall assume for a non-zero-power CSI-RS where is selected such that the power offset specified by the higher-layer parameter powerControlOffsetSS in the NZP-CSI-RS-Resource IE or in the TRS-ResourceSet IE, if provided, is fulfilled. The quantities k', l', , and are given by Tables 7.4.1.5.3-1 to 7.4.1.5.3-5 where each k,l in a given row of Table 7.4.1.5.3-1 corresponds to a CDM group of size 1 (no CDM) or size 2, 4, or 8. The CDM type is provided by the higher layer parameter cdm-Type in the CSI-RS-ResourceMapping IE. For NZP CSI-RS configured by the TRS-ResourceSet IE, the CDM type is ‘noCDM’. The indices k' and l' index resource elements within a CDM group. The time-domain locations l0∈0,1, …, 13 and l1∈2, 3, …, 12 are provided by the higher-layer parameters firstOFDMSymbolInTimeDomain and firstOFDMSymbolInTimeDomain2, respectively, in the CSI-RS-ResourceMapping IE or the CSI-RS-ResourceConfigMobility IE and defined relative to the start of a slot. For NZP CSI-RS configured by TRS-ResourceSet IE, the time-domain locations l0∈0,1, …, 13 and l1∈2, 3, …, 12 are provided by the higher-layer parameter firstOFDMSymbolInTimeDomain and firstOFDMSymbolInTimeDomain+4. The frequency-domain location is given by a bitmap provided by the higher-layer parameter frequencyDomainAllocation in the CSI-RS-ResourceMapping IE or, the CSI-RS-ResourceConfigMobility IE or the TRS-ResourceSet IE with the bitmap and value of ki in Table 7.4.1.5.3-1 given by - , ki-1=fi for row 1 of Table 7.4.1.5.3-1 - , ki-1=fi for row 2 of Table 7.4.1.5.3-1 - , ki-1=4fi for row 4 of Table 7.4.1.5.3-1 - , ki-1=2fi for all other cases where is the bit number of the bit in the bitmap set to one, repeated across every 1ρ of the resource blocks configured for CSI-RS reception by the UE. The starting position and number of the resource blocks in which the UE shall assume that CSI-RS is transmitted are given by the higher-layer parameters freqBand and density in the CSI-RS-ResourceMapping IE for the bandwidth part given by the higher-layer parameter BWP-Id in the CSI-ResourceConfig IE or given by the higher-layer parameters nrofPRBs in the CSI-RS-CellMobility IE where the the startPRB given by csi-rs-MeasurementBW is relative to common resource block 0. For NZP CSI-RS configured by TRS-ResourceSet IE, the starting position and number of the resource blocks in which the CSI-RS can be transmitted are given by the higher-layer parameters nrofRBs, and startingRB in the TRS-ResourceSet IE, where startingRB is relative to common resource block 0 and density =3.. It is good to note that, with density set to 3 and number of ports limited to 1, CDM type could be derived implicitely. |
| LGE |  |
| MediaTek |  |
| Nordic Semi. |  |
| Intel | Proposal 2: If L1-based TRS availability indication is enabled, before UE receives a L1 availability indication after camping on a cell or after receiving an updated TRS configuration, the UE can assume the configured group(s) of TRS resource set(s) are unavailable. |
| Ericsson |  |
| Qualcomm | Observation 1: Excessive transmissions of paging PDCCH with short message indicator equal to ‘00’ causes higher operating cost of a legacy UE. Proposal 1: If network configures the Rel-17 TRS availability indicator in paging PDCCH, UE does not expect TRS availability indication field contain all 0 bits in the paging PDCCH if the short message indicator is ‘00’. |

Companies please provide your views on whether to discuss any of the above proposals, and please specify which proposal(s) is identified:

Table 8: Companies’ views on whether to discuss any proposal in the above table

| Company name | Whether to discuss any proposal (Y/N)? | Which proposal to discuss, if identified |
| --- | --- | --- |
| Nordic | Generally, clarifications to sub-clause 5.1.6.1.1 CSI-RS for tracking could be discussed Huawei P6 can be discussed Nokia 211 clarification can be discussed | Intel proposal is not needed, because TRS is validated only when 1 is received QC proposal and Huawei P7: we do not understand why it should be precluded. UEs must be able to handle reception of reserved field, as it can be used for future UEs for other reason. |
| MediaTek | Y | Support to discuss the following proposals: - Proposal 7 of Huawei: It looks a good solution to avoid dummy all-zero indication in paging PDCCH when PEI is already used for TRS availability indication. Those bits can be used for other future feature. - TPs to TS 38.211 proposed by Nokia |
| Nokia | Y | Evidently, I think Nokia proposal should be discussed (to close the loop on the parameters). Also P6 from Huawei could be discussed For P1 from xiaomi/Panasonic; RAN2 made following agreement, thus not sure if RAN1 needs more discussion: A UE which acquired SIB-X with a TRS/CSI-RS configuration but didn’t yet receive an associated L1-based availability indication considers the configured TRS/CSI-RS as “unavailable”. For P3 from ZTE; I also though about this but concluded that as scramblingID-Info refers/provides IEs of ScramblingId there was no need to change, but fine to disucss |
| ZTE, Sanechips | Y | Huawei: P6 ZTE:P3 Nokia: clarification on TS38.211 |
| CATT | Y | Nokia and ZTE’s proposals |
| vivo | Y | Support to discuss Huawei P6 and Nokia’s proposals. Other proposals are not necessary. |
| Qualcomm | Y | For QC proposal and Huawei P7, we are fine to have a conclusion that for Rel-17, there is no need for network to configure all 0’s to TRS availability and short message indicator to ‘00’ as this does not provide any information to UE. |
| Huawei, HiSilicon | Y | HW’s P6 and P7, as commented by MTK, Qulacomm and other companies. HW’s P8, Intel’s P2, Panasonic’s P1: these proposals are related and discussing how to capture the assumption of unavailability. Otherwise the spec is not clear. Nokia’s proposal: this should be also discussed. |
| Ericsson | Y | Huawei’s P6, P8 (at least parts of it), Nokia TPs for 211. |
| Apple | Y | Huawei P6 Nokia’s |
| Ratio of Y for Nokia TPs | 90% (9/10) | Moderator: Recommended to be 1st priority topics (as Proposals 3.4-1 and 3.4-2) |
| Ratio of Y for Huawei P6 | 70% (7/10) | Moderator: Recommended to be 2nd priority topic (as Proposal 3.4-3) |
| Ratio of Y for Huawei P7 | 30% (3/10) | Moderator: Recommended for companies’ further check and potential discussion in next meeting |
| Ratio of Y for ZTE P3 | 30% (3/10) | Moderator: Recommended for companies’ further check and potential discussion in next meeting |

# Summary

Moderator would like to thank companies’ valuable inputs. Based on the statistics appended in Tables 1-8, the following list of topics are suggested for email discussion:

| 1st priority topics (Ratio of Y >= 80%) | Proposal 2.1-1, Proposal 2.4-1, Proposal 3.1-1, Proposal 3.2-1, Proposal 3.4-1, Proposal 3.4-2 |
| --- | --- |
| 2nd priority topics (Ratio of Y < 80% and >= 50%) | Conclusion 2.2-1, Proposal 2.4-2, Proposal 2.4-3, Proposal 2.4-4, Proposal 3.3-1, Proposal 3.4-3 |

After session chair’s joint consideration, the following topics (including 1st priority topics and part of 2nd priority topics) are selected for formal email discussion:

Proposal 2.1-1, Proposal 2.4-1, Proposal 3.1-1, Proposal 3.2-1, Proposal 3.4-1, Proposal 3.4-2, Conclusion 2.2-1, Proposal 2.4-2, Proposal 2.4-3, Proposal 3.4-3

For ease of reference, moderator quoted the selected proposals and conclusion in the table below:

Proposal 2.1-1: For alignment with RAN2 agreements, adopt the following TPs for TS 38.212 and TS 38.213, respectively ============================ start of TP========================================== ## <Unchanged text omitted> ## 7.3.1.3.8 Format 2_7 DCI format 2_7 is used for notifying the paging early indication and TRS availability indication for one or more UEs. The following information is transmitted by means of the DCI format 2_7 with CRC scrambled by PEI-RNTI: - Paging indication field – NPOPEINSGPO bit(s), where - NPOPEI is the number of paging occasions configured by higher layer parameter PONumPerPEI as defined in Clause 10.4A in [5, TS 38.213]; - NSGPOis the number of sub-groups of a paging occasion configured by higher layer parameter subgroupsNumPerPO, if subgroupsNumPerPO is configured; otherwise NSGPO is set to 1. - Each bit in the field indicates one UE subgroup of a paging occasion if subgroupsNumPerPO is greater than 1configured; otherwise each bit in the field indicates the UE group of a paging occasion. - TRS availability indication – 1, 2, 3, 4, 5, or 6 bits if TRS-ResourceSetConfig is configured; 0 bits otherwise. The size of DCI format 2_7 is indicated by the higher layer parameter payloadSizeDCI_format2_7, according to Clause 10.4A of [5, TS 38.213]. The number of information bits in format 2_7 shall be equal to or less than the payload size of format 2_7. If the number of information bits in format 2_7 is less than the size of format 2_7, the remaining bits are reserved. ## <Unchanged text omitted> ============================= end of TP========================================== ============================ start of TP========================================== ## 10.4A PDCCH monitoring for early indication of paging ## <Unchanged text omitted> A paging indication field of DCI format 2_7 includes NPOPEI segments of K bits, where K=NSGPO if NSGPO>0 and K=1 if NSGPO is not provided. For a subgroup index iSG, 0≤iSG<K, a UE determines a value for the iPO⋅K+iSG bit in the paging indication field, where iPO=UE_IDmodN⋅NS+iSmodNPOPEI is a paging occasion index, and UE_ID, N, NS, iSG, and iS are defined in [17, TS 38.304]. When the value is '1', the UE monitors a paging occasion determined according to [17, TS 38.304]; otherwise, the UE is not required to monitor the paging occasion. If NPOPEI<NS, the number of symbols from the start of the frame to the start of the first PDCCH monitoring occasion for DCI format 2_7 that is associated with paging occasion index iPO is the iSNPOPEI+1-th value from the NSNPOPEI values provided by firstPDCCH-MonitoringOccasionOfPEI-O. ## <Unchanged text omitted> ============================= end of TP==========================================.

Proposal 2.4-1: Adopt TP1 for 38.213-h10, subclause 10.1 to reflect BDs/CCEs when searchSpaceZero is used for PEI. <begin TP1 for 38.213-h10, subclause 10.1> If a UE is not provided peiSearchSpace for Type2A-PDCCH CSS set, the UE does not monitor PDCCH for Type2A-PDCCH CSS set on the DL BWP. The CCE aggregation levels and the maximum number of PDCCH candidates per CCE aggregation level for Type2A-PDCCH CSS set are given in Table 10.1-1 when the UE is provided a non-zero value for searchSpaceID for the Type2A-PDCCH CSS set. If the UE is provided peiSearchSpace with zero value for the Type2A-PDCCH CSS set index, and for the SS/PBCH block and CORESET multiplexing patterns 2 and 3, the UE determines PDCCH monitoring occasions as described in clause 13 and the CCE aggregation levels and the number of PDCCH candidates per CCE aggregation level are given in Table 10.1-1. <end TP1>

Proposal 3.1-1: To clarify reference point of TRS availability indication, adopt the following TP to TS 38.213 ============================= start of TP#1========================================== ## 10.4B Indication of TRS resources ## <Unchanged text omitted> A value of '1' for a bit of the bitmap indicates presence of associated TRS resource sets for the multiple of the number of frames, starting from a SFN determined from SFN+PF_offsetmodT=0 [17, TS 38.304] that corresponds to the frame within the DRX cycle that includes a the PDCCH providing the DCI format 2_7, or the DCI format 1_0 with CRC scrambled by P-RNTI, with the TRS availability indication field indicating the TRS resource sets, where T is provided by defaultPagingCycle. A value of '0' for a bit of the bitmap is ignored by the UE. A UE can receive first and second PDCCHs that provide DCI format 2_7 or DCI format 1_0 with CRC scrambled by P-RNTI that indicate presence of TRS resource sets for the multiple of the number of frames, where the second PDCCH reception after the first PDCCH reception by a time that is smaller than the multiple of the number of frames. ## <Unchanged text omitted> ============================= end of TP#1==========================================.

Proposal 3.2-1: Adopt the following TP to TS 38.211: #### 7.3.1.5 Mapping to virtual resource blocks The UE shall, for each of the antenna ports used for transmission of the physical channel, assume the block of complex-valued symbols yp0, …, yp(Msymbap-1) conform to the downlink power allocation specified in [6, TS 38.214] and are mapped in sequence starting with yp0 to resource elements k',lp,μ in the virtual resource blocks assigned for transmission which meet all of the following criteria: - they are in the virtual resource blocks assigned for transmission; - the corresponding physical resource blocks are declared as available for PDSCH according to clause 5.1.4 of [6, TS 38.214]; - the corresponding resource elements in the corresponding physical resource blocks are - not used for transmission of the associated DM-RS or DM-RS intended for other co-scheduled UEs as described in clause 7.4.1.1.2; - not used for non-zero-power CSI-RS according to clause 7.4.1.5 if the corresponding physical resource blocks are for a PDSCH scheduled by a PDCCH with the CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, G-RNTI for multicast, G-CS-RNTI, or a PDSCH with SPS, except if the non-zero-power CSI-RS is a CSI-RS configured by the higher-layer parameter CSI-RS-Resource-Mobility in the MeasObjectNR IE or except if the non-zero-power CSI-RS is an aperiodic non-zero-power CSI-RS resource or except if the non-zero-power CSI-RS is TRS resources configured only via TRS-ResourceSetConfig in SIB; - not used for PT-RS according to clause 7.4.1.2; - not declared as 'not available for PDSCH according to clause 5.1.4 of [6, TS 38.214]..

Proposal 3.4-1: Adopt following TP to Section 7.4.1.5.1 of 38.211: ##### 7.4.1.5.1 General Zero-power (ZP) and non-zero-power (NZP) CSI-RS are defined - for a non-zero-power CSI-RS configured by the NZP-CSI-RS-Resource IE or by the CSI-RS-Resource-Mobility field in the CSI-RS-ResourceConfigMobility IE or by the TRS-ResourceSet IE, the sequence shall be generated according to clause 7.4.1.5.2 and mapped to resource elements according to clause 7.4.1.5.3 - for a zero-power CSI-RS configured by the ZP-CSI-RS-Resource IE, the UE shall assume that the resource elements defined in clause 7.4.1.5.3 are not used for PDSCH transmission subject to clause 5.1.4.2 of [6, TS 38.214]. The UE performs the same measurement/reception on channels/signals except PDSCH regardless of whether they collide with ZP CSI-RS or not..

Proposal 3.4-2: Adopt following TP to Section 7.4.1.5.3 of TS38.211: ##### 7.4.1.5.3 Mapping to physical resources [text omitted] The value of is given by the higher-layer parameter density in the CSI-RS-ResourceMapping IE or the CSI-RS-CellMobility IE and the number of ports X is given by the higher-layer parameter nrofPorts. For NZP CSI-RS configured by the TRS-ResourceSet IE, the density =3 and number of ports X =1. The UE is not expected to receive CSI-RS and DM-RS on the same resource elements. The UE shall assume for a non-zero-power CSI-RS where is selected such that the power offset specified by the higher-layer parameter powerControlOffsetSS in the NZP-CSI-RS-Resource IE or in the TRS-ResourceSet IE, if provided, is fulfilled. The quantities k', l', , and are given by Tables 7.4.1.5.3-1 to 7.4.1.5.3-5 where each k,l in a given row of Table 7.4.1.5.3-1 corresponds to a CDM group of size 1 (no CDM) or size 2, 4, or 8. The CDM type is provided by the higher layer parameter cdm-Type in the CSI-RS-ResourceMapping IE. For NZP CSI-RS configured by the TRS-ResourceSet IE, the CDM type is ‘noCDM’. The indices k' and l' index resource elements within a CDM group. The time-domain locations l0∈0,1, …, 13 and l1∈2, 3, …, 12 are provided by the higher-layer parameters firstOFDMSymbolInTimeDomain and firstOFDMSymbolInTimeDomain2, respectively, in the CSI-RS-ResourceMapping IE or the CSI-RS-ResourceConfigMobility IE and defined relative to the start of a slot. For NZP CSI-RS configured by TRS-ResourceSet IE, the time-domain locations l0∈0,1, …, 13 and l1∈2, 3, …, 12 are provided by the higher-layer parameter firstOFDMSymbolInTimeDomain and firstOFDMSymbolInTimeDomain+4. The frequency-domain location is given by a bitmap provided by the higher-layer parameter frequencyDomainAllocation in the CSI-RS-ResourceMapping IE or, the CSI-RS-ResourceConfigMobility IE or the TRS-ResourceSet IE with the bitmap and value of ki in Table 7.4.1.5.3-1 given by - , ki-1=fi for row 1 of Table 7.4.1.5.3-1 - , ki-1=fi for row 2 of Table 7.4.1.5.3-1 - , ki-1=4fi for row 4 of Table 7.4.1.5.3-1 - , ki-1=2fi for all other cases where is the bit number of the bit in the bitmap set to one, repeated across every 1ρ of the resource blocks configured for CSI-RS reception by the UE. The starting position and number of the resource blocks in which the UE shall assume that CSI-RS is transmitted are given by the higher-layer parameters freqBand and density in the CSI-RS-ResourceMapping IE for the bandwidth part given by the higher-layer parameter BWP-Id in the CSI-ResourceConfig IE or given by the higher-layer parameters nrofPRBs in the CSI-RS-CellMobility IE where the the startPRB given by csi-rs-MeasurementBW is relative to common resource block 0. For NZP CSI-RS configured by TRS-ResourceSet IE, the starting position and number of the resource blocks in which the CSI-RS can be transmitted are given by the higher-layer parameters nrofRBs, and startingRB in the TRS-ResourceSet IE, where startingRB is relative to common resource block 0 and density =3..

Conclusion 2.2-1: Reference to Type2A-PCCCH CSS is not added to the text describing the UE behaviour when is configured with C-RNTI.

Proposal 2.4-2: Adopt the following TP in TS 38.213. ============================ start of TP========================================== ## 10.1 UE procedure for determining physical downlink control channel assignment ## <Unchanged text omitted> A UE does not expect to detect, in a same PDCCH monitoring occasion, a DCI format with CRC scrambled by a SI-RNTI, RA-RNTI, MsgB-RNTI, TC-RNTI, P-RNTI, PEI-RNTI, C-RNTI, CS-RNTI, MCS-RNTI, MCCH-RNTI, G-RNTI, or G-CS-RNTI and a DCI format with CRC scrambled by a SL-RNTI or a SL-CS-RNTI for scheduling respective PDSCH reception and PSSCH transmission on a same serving cell. ## <Unchanged text omitted> ============================= end of TP==========================================.

Proposal 2.4-3: Adopt the following TP in TS 38.213. ============================= start of TP========================================== ## 10.4A PDCCH monitoring for early indication of paging A UE can be provided the following for detection of a DCI format 2_7 in RRC_IDLE state or in RRC_INACTIVE state [12, TS 38.331] - a search space set, by peiSearchSpace, to monitor PDCCH for detection of DCI format 2_7 according to a Type2A-PDCCH CSS set as described in clause 10.1 - a number of frames, by PEI-F_offset, from the start of a frame to the start of a first paging frame of paging frames associated with a number of PDCCH monitoring occasions for DCI format 2_7 [17, TS 38.304] to the start of a frame - a number of symbols, by firstPDCCH-MonitoringOccasionOfPEI-O, from the start of the frame to the start of the first PDCCH monitoring occasion for DCI format 2_7 - a size, by payloadSizeDCI_format2_7 - a number of subgroups per paging occasion, NSGPO, by subgroupsNumPerPO - a number of paging occasions associated with the number of PDCCH monitoring occasions for DCI format 2_7, NPOPEI, by PONumPerPEI ## <Unchanged text omitted> ============================= end of TP==========================================.

Proposal 3.4-3: Adopt the TP3 to determine TRS availability indication field length. ------------------------------------- Start of Text Proposal 3------------------------------------------ < Unchanged parts are omitted > #### 7.3.1.2.1 Format 1_0 The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by P-RNTI: < Unchanged parts are omitted > - TRS availability indication – 1, 2, 3, 4, 5, or 6 bits according to indBitID(s) provided by the TRS-ResourceSetConfig if TRS-ResourceSetConfig is configured; 0 bits otherwise. - Reserved bits – (8 – M) bits for operation in a cell with shared spectrum channel access in frequency range 1 or for operation in a cell in frequency range 2-2; (6 – M) bits for operation in a cell without shared spectrum channel access, where the value of M is the number of bits for the field of 'TRS availability indication' as defined above. #### 7.3.1.3.8 Format 2_7 The following information is transmitted by means of the DCI format 2_7 with CRC scrambled by PEI-RNTI: < Unchanged parts are omitted > - TRS availability indication – 1, 2, 3, 4, 5, or 6 bits according to indBitID(s) provided by the TRS-ResourceSetConfig if TRS-ResourceSetConfig is; 0 bits otherwise. ------------------------------------- End of Text Proposal 3------------------------------------------.

# Reference

R1-2203085, “Remaining issues on UE power saving in IDLE/inactive mode”, Huawei, HiSilicon

R1-2203310, “Remaining issues on enhancements for idle/inactive-mode UE power saving”, Spreadtrum Communications

R1-2203478, “Remaining issues of Paging enhancement and TRS/CSI-RS”, CATT

R1-2203519, “Remaining issues on idle/inactive-mode UE power saving”, vivo

R1-2203595, “Remaining issues of idle/inactive-mode UE power saving”, ZTE, Sanechips

R1-2203789, “Remaining issues on power saving enhancement for idle/inactive UEs”, xiaomi

R1-2203867, “Maintenance on enhancements for idle/inactive-mode UE power saving”, Samsung

R1-2204071, “Remaining issues on Enhancements for idle/inactive-mode UE power saving”, Panasonic

R1-2204175, “Remaining issues on idle/inactive-mode UE power saving”, Sharp

R1-2204210, “Remaining issues on idle/inactive-mode UE power saving”, Apple

R1-2204604, “Open issues on Idle/Inactive mode power saving”, Nokia, Nokia Shanghai Bell

R1-2204620, “Remaining issue on TRS/CSI-RS occasion(s) for idle/inactive UEs”, LG Electronics

R1-2204704, “Maintenance on idle/inactive-mode UE power saving”, MediaTek Inc.

R1-2204745, “On remaining issues of TRS design for idle/inactive UEs”, Nordic Semiconductor ASA

R1-2204773, “Discussion on remaining aspects of UE Power saving in idle/inactive mode”, Intel Corporation

R1-2204959, “Maintenance for Rel-17 UE PS : Idle/Inactive mode”, Ericsson

R1-2204988, “Maintenance issues on idle and inactive mode UE power saving”, Qualcomm Incorporated