3GPP TSG RAN WG1#109-e																															  R1-2205393

e-Meeting, May 9th – 20th, 2022

Source:	Moderator

Title:		Preparation Phase Summary of Maintenance on PDCCH Monitoring Adaptation

Agenda Item:	8.7

Document for:	Discussion and Decision

# Preparation phase

In the following table, there list the issues from the submitted contributions (thanks to vivo/Xiadong). Together with the lists of quoted proposals and TPs in Appendix, companies can quickly understand the support level and the possible proposals, which will help companies to decide which issue(s) should be discussed in upcoming email discussion.

## Categorization and check of companies proposals:

| Issue ID | Brief summary | Relevant proposals | Text proposals for spec |
| --- | --- | --- | --- |
| 1 | PDCCH Skipping for skipping C-RNTI/... in Type 0/0A/1/2 CSS | [1,HW, P1] [4, vivo, P1] [5, ZTE, P1] [7, SS, Conclusion 1] [9, Pana, P1/P2] [13, LGE, P1] [19, QC, P5] | [1, HW, P2] [7, SS, TP1] |
| 2-1 | How to capture the PDCCH skipping for SR and RACH procedure in the spec. | [1,HW, P10/P11] [4, vivo, P3/P4] [5, ZTE, P2] [7, SS, P1/P2] [8, OPPO, P2] [9, Pana, P4] [12, Nokia, P2/P3] [14, MTK, P3] [15, Nordic, P1] [16, Intel, P1] [18, Ericsson, P5] [18, Ericsson, P6] [19, QC, P1/P2/P3/P4] | [4, vivo, P3/P4] [5, ZTE, P2] [8, OPPO, TP2] [12, Nokia, P2/P3] [14, MTK, P5] [15, Nordic, P1] [16, Intel, P1] |
| 2-2 | SSSG switching and SR/RACH interaction | [15, Nordic, P3] | [15, Nordic, P3] |
| 2-3 | Default R17 SSSG upon configuration/reconfiguration | [15, Nordic, P2] | [15, Nordic, P2] |
| 3-1 | Application delay, whether to confirm WA? - Modertor: Suggest to include 3-2 for email discussion | [1,HW, P3] [3, CATT, P6] [7, SS, P4] [14, MTK, P11] [18, Ericsson, P2] |  |
| 3-2 | Application delay - FFS: a minimum applicable scheduling offset is configured in the BWP - Moderator: Suggest to merge to the discussion on 3-1 | [1,HW, P4] [3,CATT, P7] [5, ZTE, P5] [7, SS, P3] [12, Nokia, P4] [14, MTK, P1] [18, Ericsson, P3] | [1,HW, P5, TP2] [5, ZTE, P5] |
| 3-3 | Application delay Interaction with HARQ | [1,HW, P5] [2, SPRD, P1/P2] [3,CATT, P8] [5, ZTE, P4] [10, Apple, P1/P2] [11, DCM, P2] [12, Nokia, P6/P7] [13, LGE, P2] [14, MTK, P2] [16, Intel, P3] [17, InterDigital, P1] [18, Ericsson, P1] [19, QC, P8] | [1,HW, P6, TP3] |
| 3-4 | Application delay, P_Switch values for 480kHz and 960kHz not included in the spec | [12, Nokia, P5] [15, Nordic, P5] | [15, Nordic, P5] |
| 4 | SSSG timer: whether to confirm the WA? - Resets the timer after a slot of the active DL BWP of the serving cell when the UE detects a DCI format in a PDCCH reception in the slot with CRC scrambled by - Working Assumption: C-RNTI/CS-RNTI/MCS-C-RNTI | [3, CATT, P4] [4, vivo, P2] [11, DCM, P1] [12, Nokia, P1] [14, MTK, P9] [19, QC, P7] | [3, CATT, P5, TP2] |
| 5 | TP from ZTE | [5, ZTE, P8] |  |
| 6-1 | In case of a scheduling DCI indicating PDCCH monitoring adaptation and BWP switching simultaneously, | [1,HW, P9] [5, ZTE, P6] [6, Xiaomi, P1] [14, MTK, P7] | [14, MTK, P8] |
| 6-2 | If the active BWP is switched due to the expiration of bwp-InactivityTimer before the end of the PDCCH skipping duration, what is the default behavior on the new active BWP | [1,HW, P8] [5, ZTE, P7] [6, Xiaomi, P1] [8, OPPO, P1] [14, MTK, P6] | [8, OPPO, TP1] [14, MTK, P8] |
| 6-3 | When the source BWP is not configured with PDCCH monitoring adaptation and the target BWP is configured with PDCCH monitoring adaptation When the source BWP is configured with PDCCH monitoring adaptation and the target BWP is not configured with PDCCH monitoring adaptation | [8, OPPO, P1] | [8, OPPO, TP1] |
| 7 | Support of PDCCH monitoring case 5 |  |  |
| 8 | Handling of indication change, need to confirm the WA | [3, CATT, P10] [4, vivo, P7] [14, MTK, P10] |  |
| 9 | The following is discussed this meeting and is not converged. [High] Proposal 7-3a (v1):  The PDCCH skipping applies only in active time  Down-select from the following options:  Option 1a: the PDCCH skipping duration is considered irrespective the slot(s) are in active time or not,  Option 1b: PDCCH skipping is terminated when UE enters outside active time | [1,HW, P12] [3, CATT, P3] [4, vivo, P5] [5, ZTE, P3] [10, Apple, P3] [13, Nokia, P10] [13, LGE, P3] [14, MTK, P4] [15, Nordic, P4] [16, Intel, P2] [18, Ericsson, P4] [19, QC, P6] | [13, Nokia, P11] [14, MTK, P5] [15, Nordic, P4] |
| 10 | Skipping the indicated interval until UE receives a new skipping indication | [3, CATT, P1] | [3, CATT, P2, TP1] |
| 11 | Confirm the WA: The bit length of the candidate skipping values and SSSG switching initial timer values in slots for 480kHz and 960kHz SCS are assumed to be the same as that for 120KHz SCS. | [3, CATT, P9] |  |
| 12 | PDCCH skipping and MBS, If a UE is indicated to skip PDCH for a duration, the UE skips monitoring PDCCH according to Type3-PDCCH CSS sets or USS sets on the serving cell, except for monitoring a DCI format with CRC scrambled by G-RNTI, or G-CS-RNTI, or MCCH-RNTI | [4, vivo, P6] |  |
| 13 | It needs to be clarified when PDCCH skipping is applied, whether SPS/CG is affected. For simpler UE behavior, active SPS/CG is not affected. | [9, Pana, P3] |  |
| 14 | Adding restriction to Rel-17 PDCCH monitoring adaptaion which can be only used when C-DRX is configured. | [13, Nokia, P8] | [13, Nokia, P9] |

Please provide your view below. “Y” to acknowledge it is an issue and need to discuss, “N” to disagree it is an issue and not need to discuss.

| Company | 1 | 2-1 | 2-2 | 2-3 | 3-1 | 3-2 | 3-3 | 3-4 | 4 | 5 | 6-1 | 6-2 | 6-3 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Huawei | Y | Y | Y | Y | Y | Y | Y | Y | Y |  | Y | Y | Y | N | Y | Y | N |  |  |  | N |
| Spreadtrum | Y | Y | N | N | Y | Y | Y | Y | Y |  | N | N | N | N | N | Y | N | Y | N | N | N |
| CATT | N | N | N | N | Y | Y | Y | Y | Y | N | N | N | N | N | Y | Y | Y | Y | N | N | N |
| vivo | Y | Y | N | N | Y | N | Y | Y | Y | Y | Y | Y | N | N | Y | Y | N | Y | Y | N | N |
| ZTE | Y | Y | N | Y | Y | Y | Y | Y | Y | Y | Y | Y |  | N | Y | Y | N | Y | N | N | N |
| xiaomi |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Samsung |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OPPO |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Panasonic |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Apple |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DOCOMO | Y | Y |  |  | Y | Y | Y |  | Y |  | Y | Y |  |  | Y | Y |  |  |  |  |  |
| Nokia |  | Y |  |  | Y | Y | Y | Y | Y |  | Y | Y | Y | Y |  | Y |  |  |  |  | Y |
| LG | Y | Y | Y | N | N | N | Y | N | Y | N | N | N | N |  |  | Y | N |  | N | N | N |
| MediaTek | Y | Y |  |  | Y | N | Y |  | Y |  | Y | Y |  |  | Y | Y |  |  |  |  |  |
| Nordic | N | Y | Y | Y | N | N | N | Y | N | N | Y | Y | N | N | N | Y | N | N | N | N | N |
| Intel | N | Y | N | N | Y | Y | Y | Y | Y | N | Y | Y | Y | N | N | Y | N | N | N | N | N |
| InterDigital, |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ericsson |  | Y | Y | Y | Y | Y | Y | Y | Y |  | Y | Y | Y |  |  | Y |  |  |  |  |  |
| Qualcomm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Companies’ further comments/suggestions for selected issue(s)

| Company | Comment(s)/suggestion(s) for selected issue(s) (e.g., suggested TP(s), initial proposal(s), reason(s) for discussion or no discussion) |
| --- | --- |
| Huawei |  |
| Spreadtrum |  |
| CATT |  |
| vivo | The WA can be confirmed unless there is critical issues |
| ZTE | - For issue 1, 2-1, 3-1,3-2,3-3,3-4, 5, 6-1, 6-2, 9, if these issues have no conclusion, the implementation of the power saving schemes may be affected. And these issues have been discussed a lot at previous RAN1 meetings. So we think these issues should be discussed and make a conclusion. - For issue 4, 8, 11, these issues are about confirm the WA, we are okay if these WA are automatically confirmed. - For 3-4, since the searchSpaceSwitchDelay is a R16 IE, whether a new RRC signal is needed should be discussed firstly. 38.331-h00 SearchSpaceSwitchConfig-r16 ::= SEQUENCE { cellGroupsForSwitchList-r16 SEQUENCE(SIZE (1..4)) OF CellGroupForSwitch-r16 OPTIONAL, -- Need R searchSpaceSwitchDelay-r16 INTEGER (10..52) OPTIONAL -- Need R }. - For 2-2, we think it is difficult to specify whether a SSSG is sparse or dense. And even in a sparse SSSG UE can monitor PDCCH. Hence we think this issue should be lower priority. - For 6-3, agree with HW’s opinion in [R1-2203086], it can be solved reusing the same as R15 rule in cross-BWP scheduling and has no spec impact. - For issue 7, 10, the power saving schemes can work well without these issues, therefore, we think these issues should be lower priority. - For issue 13, there is no spec impact. - For issue 14, it can be discussed together with issue 9 - For the new issue (2-3) added by Nordic, we think it can be discussed together with issue6-2, similar case when there is no indication received in the new BWP or after reconfiguration |
| xiaomi |  |
| Samsung |  |
| OPPO |  |
| Panasonic |  |
| Apple |  |
| DOCOMO |  |
| Nokia | On issue 3-4, our discussion relates to the signalled range of searchSpaceSwitchDelay, which is currently lower than min. delay. Also the C-DRX behaviour needs to be captured in RAN1 spesification. |
| LG |  |
| MediaTek | General: For those issues not marked Y or N, we think more check may be needed Issue 1: This issue needs to be decided for specific UE implementation. If no consensus, we suggest to not to add any additional CSS for PDCCH skipping. Issue 2: There looks consensus on “PDCCH is terminated”. To assist the convergence of the corrections, one TP for SR and one TP for RACH can be separately discussed. Issue 3-1: From UE implementation, minimum applicable scheduling offset enables UE power saving by PDSCH/PUSCH related power-saving design. There is no need to have application delay of PDCCH monitoring adaptation depend on minimum scheduling offset. Therefore, confirming the working assumption without the FFS looks logical for us. Issue 3-2: Suggest to confirm current working assumption and close the FFS (no dependency on minimum applicable scheduling offset) Issue 3-3: Our first preference is to complete R17 PDCCH monitoring adaptation, while optimization of retransmission handling can be discussed in Rel-18 XR. However, if majority would like to include retransmission handling, a simplified design, i.e., PDCCH skipping is “terminated” whenever ReTX timer (of any HARQ process) is started, may be one compromise solution to consider. Issue 4: Existing WA can be confirmed without any fatal issue identified Issues 6-1 and 6-2: Existing convention for concurrent indications with BWP switching indication can be reused. Issue 8: Looks no issue to confirm the WA Issue 9: It suggested to terminate PDCCH skipping outside Active Time so as to avoid duplicated functionality with Rel-16 DCP |
| Nordic | In general, for WA confirmations we prefer not to discuss (waste time), unless some technical issue has been identified. We added Issue 2-3 which was missed. |
| Intel |  |
| InterDigital, |  |
| Ericsson |  |
| Qualcomm |  |

## Recommended topics for email discussion

Moderator would like to thanks companies’ valuable inputs. The following please find the statistics calculated:

| Issue ID | 1 | 2-1 | 2-2 | 2-3 | 3-1 | 3-2 | 3-3 | 3-4 | 4 | 5 | 6-1 | 6-2 | 6-3 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Total #Y | 7 | 11 | 4 | 4 | 10 | 8 | 11 | 9 | 11 | 2 | 9 | 9 | 4 | 1 | 6 | 12 | 1 | 4 | 1 | 0 | 1 |
| Total #N | 3 | 1 | 5 | 5 | 2 | 4 | 1 | 1 | 1 | 4 | 3 | 3 | 5 | 7 | 3 | 0 | 7 | 2 | 6 | 7 | 8 |
| Total #inputs | 10 | 12 | 9 | 9 | 12 | 12 | 12 | 10 | 12 | 6 | 12 | 12 | 9 | 8 | 9 | 12 | 8 | 6 | 7 | 7 | 9 |
| Ratio of Y (%) | 70 | 92 | 44 | 44 | 83 | 67 | 92 | 90 | 92 | 33 | 75 | 75 | 44 | 13 | 67 | 100 | 13 | 67 | 14 | 0 | 11 |

Based on the above statistics, moderator would like to suggest those issues with Ratio of Y >= 80% as 1st priority topics and those issues with Ratio of Y < 80% and >= 50% as 2nd priority topics:

- 1st priority topics (Ratio of Y >= 80%)

| Issue ID | Brief summary |
| --- | --- |
| 2-1 | How to capture the PDCCH skipping for SR and RACH procedure in the spec. |
| 3-1 | Application delay, whether to confirm WA? - Modertor: Suggest to include 3-2 for email discussion (i.e. whether to depend on minimum applicable scheduling offset, if configured) |
| 3-3 | Application delay Interaction with HARQ |
| 3-4 | Application delay, P_Switch values for 480kHz and 960kHz not included in the spec |
| 4 | SSSG timer: whether to confirm the WA? - Resets the timer after a slot of the active DL BWP of the serving cell when the UE detects a DCI format in a PDCCH reception in the slot with CRC scrambled by - Working Assumption: C-RNTI/CS-RNTI/MCS-C-RNTI |
| 9 | The following is discussed this meeting and is not converged. [High] Proposal 7-3a (v1):  The PDCCH skipping applies only in active time  Down-select from the following options:  Option 1a: the PDCCH skipping duration is considered irrespective the slot(s) are in active time or not,  Option 1b: PDCCH skipping is terminated when UE enters outside active time |

- 2nd priority topics (Ratio of Y < 80% and >= 50%)

| Issue ID | Brief summary |
| --- | --- |
| 1 | PDCCH Skipping for skipping C-RNTI/... in Type 0/0A/1/2 CSS |
| 3-2 | Application delay - FFS: a minimum applicable scheduling offset is configured in the BWP - Moderator: Suggest to merge to the discussion on 3-1 |
| 6-1 | In case of a scheduling DCI indicating PDCCH monitoring adaptation and BWP switching simultaneously, |
| 6-2 | If the active BWP is switched due to the expiration of bwp-InactivityTimer before the end of the PDCCH skipping duration, what is the default behavior on the new active BWP |
| 8 | Handling of indication change, need to confirm the WA |
| 11 | Confirm the WA: The bit length of the candidate skipping values and SSSG switching initial timer values in slots for 480kHz and 960kHz SCS are assumed to be the same as that for 120KHz SCS. |

# Summary

With the above information and statistics in Section 1.2, session chair (CMCC/Xiaodong) finally select the following list of topics for the formal email discussion of agenda item 8.7.2:

[109-e-R17_PowSav-02] Email discussion under 8.7.2 for maintenance on PDCCH monitoring adaptation, for issues in the FL summary of the preparation phase discussion, including Issue 2-1, Issue 3-1 (merging Issue 3-2), Issue 3-3, Issue 3-4, Issue 4, Issue 9, Issue 1 - Xiaodong(vivo)

·         Discussion and decision by 5/20

# Reference

The following contributions are submitted in RAN1#109-E in AI 8.7.2,

- R1-2203086	Remaining issues on the extensions to Rel-16 DCI-based power saving adaptation for an active BWP	Huawei, HiSilicon

- R1-2203311	Remaining issues on enhancements for power saving adaptation during active time	Spreadtrum Communications

- R1-2203479	Remaining issues of PDCCH monitoring adaptation	CATT

- R1-2203520	Remaining issues on DCI-based power saving adaptation in connected mode	vivo

- R1-2203596	Remaining issues of PDCCH monitoring adaptation	ZTE, Sanechips

- R1-2203790	Remaining issues on PDCCH monitoring adaptation for power saving	xiaomi

- R1-2203868	Maintenance on DCI-based power saving techniques	Samsung

- R1-2203993	Remaining issues for DCI-based power saving adaptation	OPPO

- R1-2204072	Remaining issues for extensions to Rel-16 DCI-based power saving adaptation during DRX Active Time	Panasonic

- R1-2204211	Remaining details on enhanced DCI-based power saving adaptation	Apple

- R1-2204348	Discussion on extension to DCI-based power saving adaptation	NTT DOCOMO, INC.

- R1-2204605	Open issues on PDCCH monitoring adaptation for UE power saving	Nokia, Nokia Shanghai Bell

- R1-2204621	Remaining issues on DCI-based power saving adaptation during DRX ActiveTime	LG Electronics

- R1-2204705	Maintenance on DCI-based PDCCH Monitoring Adaptation	MediaTek Inc.

- R1-2204746	On remaining issues of PDCCH monitoring adaptation	Nordic Semiconductor ASA

- R1-2204774	Discussion on remaining issues of DCI-based PDCCH monitoring adaptation in active time	Intel Corporation

- R1-2204828	Remaining issues on DCI-based power saving adaptation	InterDigital, Inc.

- R1-2204960	Maintenance for Rel-17 UE PS : PDCCH monitoring adaptation	Ericsson

- R1-2205006	DCI-based power saving adaptation during DRX ActiveTime	Qualcomm Incorporated

Other references:

- RP-200938, “Revised WID: UE Power Saving Enhancements for NR”, MediaTek Inc., RAN#88-e

- R1-2202969	Corrections on UE power saving enhancements in 38.212	Huawei

- R1-2202953	Corrections further to the introduction of UE power savings enhancements in NR	Samsung

- R1-2202964	Corrections further to the introduction of UE power savings enhancements in NR	Nokia

- R1-2202760, “LS on updated Rel-17 LTE and NR higher-layers parameter list”, RAN1, RAN1#108-e

# Appendix: Proposals from companies’ submitted contributions

## Huawei, HiSilicon

- R1-2203086	Remaining issues on the extensions to Rel-16 DCI-based power saving adaptation for an active BWP	Huawei, HiSilicon

Observation 1: If PDCCH skipping/SSSG switching and BWP switching are indicated simultaneously by the same scheduling DCI, the PDCCH monitoring adaptation field in the DCI indicates the PDCCH monitoring adaptation on the target BWP, which has no specification impact reusing the same as R15 rule in cross-BWP scheduling.

Proposal 1: Do not monitor DCI format 0_0 and DCI format 1_0 with CRC scrambled by the C-RNTI, MCS-C-RNTI, or the CS-RNTI in Type0/0A/1 or 2 PDCCH CSS during the PDCCH skipping duration.

Proposal 2: adopt the TP#1 to capture that monitoring of DCI format 0_0 and DCI format 1_0 with CRC scrambled by the C-RNTI, MCS-C-RNTI, or the CS-RNTI in Type0/0A/1 or 2 PDCCH CSS is skipped during the PDCCH skipping duration.

Proposal 3: confirm the working assumption of application delay of SSSG switching in RAN1#108:

Working Assumption - Upon detecting a scheduling DCI format 1-1/1-2/0-1/0-2 indicating SSSG switching (i.e., Beh 2/2A/2B), - the UE applies SSSG switching on an active BWP of the serving cell at a first slot that is at least Pswitch symbols after the last symbol of the PDCCH reception - FFS: a minimum applicable scheduling offset is configured in the BWP Note: Pswitch is defined in Table 10.4-1 in TS38.213

Proposal 4: If minimum scheduling offset is configured in the active BWP, the application delay of PDCCH skipping or SSSG switching is at least max (applicable K0min, Z), where K0min is the current applied minimum scheduling offset in the active BWP.

Proposal 5: Adopt the Text Proposal 2 for the application delay of PDCCH monitoring adaptation.

Proposal 6: Adopt the DL retransmission timer in Alt.1 to handle the retransmission of PDCCH skipping, i.e. the UE still performs PDCCH monitoring for HARQ retransmission when drx-RetransmissionTimerDL is running.

Proposal 7: Adopt the Text Proposal 3 on the handling of HARQ retransmission.

Proposal 8: If the active BWP is switched due to expiration of bwp-InactivityTimer before the end of a PDCCH skipping duration, the default behavior on the new active BWP is

- UE starts monitoring PDCCH according to the configured SS sets if SSSG is not configured on the new active BWP;

- UE starts monitoring PDCCH according to the SS set(s) associated with SSSG#0 if SSSG is configured on the new active BWP.

Proposal 9: In case of a scheduling DCI indicating PDCCH monitoring adaptation and BWP switching simultaneously,

- If PDCCH skipping is indicated, the skipping duration starts from the slot indicated by the slot offset value of the time domain resource assignment field in the DCI format;

- If SSSG switching is indicated, the indicated SSSG takes effect from the slot indicated by the slot offset value of the time domain resource assignment field in the DCI format.

Proposal 10: If the UE is skipping PDCCH monitoring for a duration and at the first slot after the last OFDM symbol of a positive SR transmission in PUCCH, the UE stops PDCCH skipping in the duration.

Proposal 11: Support SSSG switching to a SSSG with shorter PDCCH monitoring periodicity when RAR window/MsgB window/contention resolution timer is running or at the first slot after the last OFDM symbol of a positive SR transmission in PUCCH .

Proposal 12: Continue counting the PDCCH skipping duration outside active time.

## Spreadtrum Communications

- R1-2203311	Remaining issues on enhancements for power saving adaptation during active time	Spreadtrum Communications

Proposal 1: Rel-17 SSSG switching does not interact with the DRX HARQ-ACK timer or retransmission timer.

Proposal 2: For UL grant, UE may monitor PDCCH in PDCCH skipping duration during running of DRX HARQ-ACK timer or retransmission timer.

## CATT

- R1-2203479	Remaining issues of PDCCH monitoring adaptation	CATT

Proposal 1：If UE is indicated in skipping an interval of the PDCCH monitoring, UE would keep skipping the interval of PDCCH monitoring and monitor the PDCCH at the next PDCCH monitoring occasion after the skipping interval until UE receives a new PDCCH skipping indication.

Proposal 2: The Text Proposal 1 below is adopted.

--------------------------------- Start of Text Proposal 1 for TS 38.213------------------------------------ < Unchanged parts are omitted > 10.4 Search space set group switching and skipping of PDCCH monitoring < Unchanged parts are omitted > A UE can be provided a set of durations by PDCCHSkippingDurationList for PDCCH monitoring on an active DL BWP of a serving cell and, if the UE is not provided searchSpaceGroupIdList-r17 on the active DL BWP of the serving cell, a DCI format 0_1 and a DCI format 0_2 that schedule PUSCH transmission, and a DCI format 1_1 and a DCI format 1_2 that schedule PDSCH receptions can include a PDCCH monitoring adaptation field of 1 bit or of 2 bits. If the field has 1 bit and for PDCCH monitoring by the UE according to Type3-PDCCH CSS sets or USS sets on the active DL BWP of the serving cell - a '0' value for the bit indicates no skipping in PDCCH monitoring - a '1' value for the bit indicates skipping PDCCH monitoring for a duration provided by the first value in the set of durations If the field has 2 bits and for PDCCH monitoring by the UE according to Type3-PDCCH CSS sets or USS sets on the active DL BWP of the serving cell - a '00' value for the bits indicates no skipping in PDCCH monitoring - a '01' value for the bits indicates skipping PDCCH monitoring for a duration provided by the first value in the set of durations - a '10' value for the bits indicates skipping PDCCH monitoring for a duration provided by the second value in the set of durations - a '11' value for the bits indicates skipping PDCCH monitoring for a duration provided by the third value in the set of durations, if any; otherwise, if the set of durations includes two values, a use of the '11' value is reserved When UE is indicated skipping PDCCH monitoring for a duration in a DCI format 0_1, DCI format 1_1, DCI format 0_2, and/or DCI format 1_2, UE would keep skipping the indicated interval until UE receives a new PDCCH skipping indication. < Unchanged parts are omitted > ----------------------------------- End of Text Proposal 1 for TS 38.213--------------------------------------------------

Proposal 3: PDCCH skipping duration would not be applies to outside active time, i.e. the PDCCH skipping duration would be stopped outside the DRX active time.

Proposal 4: UE resets the timer after a slot of the active DL BWP of the serving cell when the UE detects a DCI format in a PDCCH reception in the slot with CRC scrambled by C-RNTI/CS-RNTI/MCS-C-RNTI/INT-RNTI/SFI-RNTI/TPC-PUSCH-RNTI/TPC-PUCCH-RNTI/TPC-SRS-RNTI/CI-RNTI/SP-CSI-RNTI.

Proposal 5: The TP 2 below is adopted.

--------------------------------- Start of Text Proposal 2 for TS 38.213------------------------------------ < Unchanged parts are omitted > 10.4 Search space set group switching and skipping of PDCCH monitoring < Unchanged parts are omitted > If a UE is provided group indexes for a Type3-PDCCH CSS set or a USS set by searchSpaceGroupIdList-r17 and a timer value by searchSpaceSwitchTimer-r17 for PDCCH monitoring on an active DL BWP of a serving cell and the timer is running, the UE - decrements the timer after a slot of the active DL BWP of the serving cell when the UE does not detect a DCI format in a PDCCH reception in the slot - resets the timer after a slot of the active DL BWP of the serving cell when the UE detects a DCI format in a PDCCH reception in the slot with CRC scrambled by C-RNTI/CS-RNTI/MCS-C-RNTI/INT-RNTI/SFI-RNTI/TPC-PUSCH-RNTI/TPC-PUCCH-RNTI/TPC-SRS-RNTI/CI-RNTI/SP-CSI-RNTI. < Unchanged parts are omitted > ----------------------------------- End of Text Proposal 2 for TS 38.213--------------------------------------------------

Proposal 6: Confirm the working assumption:

- Upon detecting a scheduling DCI format 1-1/1-2/0-1/0-2 indicating SSSG switching (i.e., Beh 2/2A/2B),

  - the UE applies SSSG switching on an active BWP of the serving cell at a first slot that is at least Pswitch symbols after the last symbol of the PDCCH reception

  - Note: Pswitch is defined in Table 10.4-1 in TS38.213

Proposal 7: The minimum applicable scheduling offset would not be considered in the application delay for either the SSSG switching or the PDCCH skipping after the monitoring adaptation indication received.

Proposal 8: The PDCCH skipping would not be impacted when DRX Retransmission timer is running.

Proposal 9: Confirm the Working Assumption:

The bit length of the candidate skipping values and SSSG switching initial timer values in slots for 480kHz and 960kHz SCS are assumed to be the same as that for 120KHz SCS.

Proposal 10: Confirm the Working Assumptions:

- When the UE receives DCI format 0_1 /1_1/0_2/1_2 with a ‘PDCCH monitoring adaptation indication' field in slot n, the UE does not expect to receive DCI format 0_1 /1_1/0_2/1_2 with a ‘PDCCH monitoring adaptation indication’field indicating change to another SSSG or skipping for the same active BWP of the scheduling cell before slot n+X of the scheduling cell, where X is the value of the application delay.

- If SSSG timer expires in slot n, the UE dos not expect to receive DCI format 0_1 /1_1/0_2/1_2 with a ‘PDCCH monitoring adaptation indication’ field indicating change to another SSSG or skipping for the same active BWP of the scheduling cell before slot n+X of the scheduling cell, where X is the value of the application delay.

## Vivo

- R1-2203520	Remaining issues on DCI-based power saving adaptation in connected mode	vivo

Proposal 1: Type0/0A/1/2-PDCCH CSS monitoring is not impacted by Rel-17 PDCCH monitoring adaptation

Proposal 2:

Confirm the following working assumption, and no specification impact is needed.

Agreement

If a UE is provided group indexes for a Type3-PDCCH CSS set or a USS set by searchSpaceGroupIdList-r17 and a timer value by searchSpaceSwitchTimer-r17 for PDCCH monitoring on a serving cell and the timer is running, the UE

- Resets the timer after a slot of the active DL BWP of the serving cell when the UE detects a DCI format in a PDCCH reception in the slot with CRC scrambled by

- Working Assumption: C-RNTI/CS-RNTI/MCS-C-RNTI

- Otherwise, decrease the timer value by one after each slot.

- Note: There is no timer for SSSG switching is running for UE in SSSG#0

Proposal 3: Agree the following conclusions and capture it in the TS38.213.

-  If the UE is skipping PDCCH monitoring for a duration and at the first slot after the last OFDM symbol of a positive SR transmission in PUCCH, the UE stops PDCCH skipping (i.e., PDCCH skipping is not activated).

Proposal 4: Agree the following conclusions and capture it in the TS38.213.

If a UE received a DCI format indicating skipping PDCCH monitoring for a duration that overlaps with at least one symbol of a window provided by ra-ResponseWindow or msgB-ResponseWindow or the duration where ra-ContentionResolutionTimer is running which is controlled by higher layers [11, TS 38.321], the UE skips monitoring PDCCH according to Type3-PDCCH CSS sets or USS sets on the serving cell, except for monitoring a DCI format in response to

- a PRACH transmission, or

- a transmission of a PRACH and a PUSCH, or to a transmission of only a PRACH if the PRACH preamble is mapped to a valid PUSCH occasion, or

- a PUSCH transmission scheduled by a RAR UL grant

Proposal 5: Agree the following conclusions and capture it in the TS38.213

The PDCCH skipping duration is considered irrespective the slot(s) are in active time or not.

Proposal 6: Agree the following conclusions and capture it in the TS38.213.

If a UE is indicated to skip PDCH for a duration, the UE skips monitoring PDCCH according to Type3-PDCCH CSS sets or USS sets on the serving cell, except for monitoring a DCI format with CRC scrambled by G-RNTI, or G-CS-RNTI, or MCCH-RNTI.

Proposal 7: confirm the following WA and no specification impact is needed.

Working assumptions:

- When the UE receives DCI format 0_1 /1_1/0_2/1_2 with a ‘PDCCH monitoring adaptation indication' field in slot n, the UE does not expect to receive DCI format  0_1 /1_1/0_2/1_2 with a ‘PDCCH monitoring adaptation indication’ field indicating change to another SSSG or skipping for the same active BWP of the scheduling cell before slot n+X of the scheduling cell, where X is the value of the application delay.

- If SSSG timer expires in slot n, the UE does not expect to receive DCI format  0_1 /1_1/0_2/1_2 with a ‘PDCCH monitoring adaptation indication’ field indicating change to another SSSG or skipping for the same active BWP of the scheduling cell before slot n+X of the scheduling cell, where X is the value of the application delay.

## ZTE, Sanechips

- R1-2203596	Remaining issues of PDCCH monitoring adaptation	ZTE, Sanechips

Proposal 1: When the UE is indicated skipping PDCCH monitoring for a duration on a serving cell, the UE skips monitoring of PDCCH candidates for DCI with CRC scrambled by the C-RNTI, the MCS-C-RNTI, or the CS-RNTI in any CSS and USS.

Proposal 2: Adopt the following Text Proposal on PDCCH skipping for SR and RACH procedure

<Unchanged parts are omitted> A UE does not expect to receive in a second slot a PDCCH on an active DL BWP that provides a DCI format indicating skipping PDCCH monitoring, or start of PDCCH monitoring according to search space sets with group index 1 or 2 for the active DL BWP, if the second slot is not at least Pswitch symbols after a first slot where the timer expires. PDCCH skipping is not applied(i.e., stop PDCCH skipping), - when UE monitors PDCCH within a ra-ResponseWindow, as described in clause 8.2, - when UE monitors PDCCH within a MsgB-ResponseWindow, as described in clause 8.2A, - if contention resolution timer is running [38.321]. If a UE is skipping PDCCH monitoring for a duration and transmits a positive SR in PUCCH, the UE stops PDCCH skipping at the first slot after the last OFDM symbol of a positive SR transmission in PUCCH. <Unchanged parts are omitted>

Proposal 3: The PDCCH skipping duration is considered irrespective the slot(s) are in active time or not.

Proposal 4: An additional UE behavior is not needed to handle the case of PDCCH skipping interaction with HARQ retransmission.

Proposal 5: Upon detecting a scheduling DCI format 1-1/1-2/0-1/0-2 indicating PDCCH skipping,

• UE applies PDCCH skipping on an active BWP of the serving cell at the first slot after the application delay of minimum applicable scheduling offset if a minimum applicable scheduling offset is configured in the BWP,

• otherwise, UE applies PDCCH skipping on the BWP of the serving cell at the first slot after the last OFDM symbol of the PDCCH transmission.

Upon detecting a scheduling DCI format 1-1/1-2/0-1/0-2 indicating SSSG switching,

• UE applies SSSG switching on an active BWP of the serving cell at the first slot after the application delay of minimum applicable scheduling offset if a minimum applicable scheduling offset is configured in the BWP,

• otherwise, UE applies SSSG switching on the BWP of the serving cell at the first slot that is at least Pswitch symbol after the last OFDM symbol of the PDCCH transmission.

Proposal 6: When UE receives a DCI indicating a PDCCH monitoring adaptation and a BWP switching, UE applies the PDCCH monitoring adaptation after BWP switching.

Proposal 7: If the active BWP is switched due to the expiration of bwp-InactivityTimer before the end of the PDCCH skipping duration,

UE does not perform PDCCH skipping before receiving a DCI indicates PDCCH skipping, if only PDCCH skipping duration list is configured in the new BWP.

UE performs PDCCH monitoring behavior according to search space sets with group index 0, if SSSG list is configured in the new BWP.

Proposal 8: The following text proposal is proposed:

TS 38.213 <Unchanged parts are omitted> 10.4 Search space set group switching and skipping of PDCCH monitoring A UE can be provided a group index for a respective Type3-PDCCH CSS set or USS set by searchSpaceGroupIdList or searchSpaceGroupIdList_r17 for PDCCH monitoring on a serving cell. And a UE can be provided a set of durations by PDCCHSkippingDurationList for PDCCH monitoring on a serving cell. If the UE is not provided searchSpaceGroupIdList or searchSpaceGroupIdList_r17 for a search space set, and the UE is not provided PDCCHSkippingDurationList for PDCCH monitoring on a serving cell, the following procedures are not applicable for PDCCH monitoring according to the search space set. <Unchanged parts are omitted>

## Xiaomi

- R1-2203790	Remaining issues on PDCCH monitoring adaptation for power saving	xiaomi

Proposal 1:

For the case that PDCCH monitoring adaptation and BWP switching are indicated in the same DCI, the PDCCH monitoring adaptation can be applied for the new BWP.

For the case that BWP switching is indicated after PDCCH monitoring adaptation indication, but before the expiration of PDCCH skipping timer or SSSG switching timer, clear the PDCCH monitoring adaptation indication and all the related running timer associated to the original BWP and go back to a default PDCCH monitoring behaviour in the new BWP.

## Samsung

- R1-2203868	Maintenance on DCI-based power saving techniques	Samsung

Conclusion #1:  Monitoring of PDCCH candidates in Type0/0A/1 or 2 PDCCH CSS is not affected by PDCCH monitoring adaptation.

- Adopt proposed TP #1 for TS 38.213

Proposal #1: UE cancels PDCCH skipping adaptation after transmission of a postive SR.

Proposal #2: UE cancels PDCCH skipping adaptation after transmission of PRACH.

Proposal #3: Upon detecting a scheduling DCI format 1-1/1-2/0-1/0-2 indicating PDCCH monitoring adaptation, the UE applies PDCCH monitoring adaptation on an active BWP of the serving cell at the first slot that is at least minK0 slots after the PDCCH reception if a minimum applicable scheduling offset is configured

- minK0 is applied minimum applicable scheduling offset for K0

Proposal #4: Confirm the following working assumption

Working assumptions:

- When the UE receives DCI format 0_1 /1_1/0_2/1_2 with a ‘PDCCH monitoring adaptation indication' field in slot n, the UE does not expect to receive DCI format  0_1 /1_1/0_2/1_2 with a ‘PDCCH monitoring adaptation indication’ field indicating change to another SSSG or skipping for the same active BWP of the scheduling cell before slot n+X of the scheduling cell, where X is the value of the application delay.

- If SSSG timer expires in slot n, the UE does not expect to receive DCI format  0_1 /1_1/0_2/1_2 with a ‘PDCCH monitoring adaptation indication’ field indicating change to another SSSG or skipping for the same active BWP of the scheduling cell before slot n+X of the scheduling cell, where X is the value of the application delay.

## OPPO

- R1-2203993	Remaining issues for DCI-based power saving adaptation	OPPO

Proposal 1: RAN1 specification clarify the time unit applied for different BWPs. RAN1 specification defined the first SSSG applied for newly activated BWP.

The text proposals can be captured as:

------------------------------ TP#1: TS 38.213 ----------------------------------- ## 10.4 Search space set group switching and skipping of PDCCH monitoring *** Unchanged text is omitted *** When the PDCCH monitoring adaptation field indicates to a UE to skip PDCCH monitoring for a duration on the active DL BWP of a serving cell, the UE starts skipping of PDCCH monitoring at the beginning of a first slot that is after the last symbol of the PDCCH reception providing the DCI format with the PDCCH monitoring adaptation field. A skipping duration for PDCCH monitoring is a time based on the SCS configured in the BWP indicating the PDCCH monitoring adaptation. For an activated BWPs, UE will start of PDCCH monitoring according to search space sets with group index 0, if configured, before receiving a DCI indicating adaptation of PDCCH monitoring. *** Unchanged text is omitted ***

In those regards, we prefer to treat RACH procedures and SR procedures differently. For SR, it will terminate both skipping duration and SSSG timer.

The text proposals can be captured as:

------------------------------ TP#2: TS 38.213 ----------------------------------- ## 10.4 Search space set group switching and skipping of PDCCH monitoring *** Unchanged text is omitted *** A UE does not expect to receive in a second slot a PDCCH on an active DL BWP that provides a DCI format indicating skipping PDCCH monitoring, or start of PDCCH monitoring according to search space sets with group index 1 or 2 for the active DL BWP, if the second slot is not at least Pswitch symbols after a first slot where the timer expires. UE’s skipping PDCCH monitoring does not apply for the PDCCH monitoring during RAR window, during MsgB window, or when the contention resolution timer is running. At the first slot after the last OFDM symbol of a positive SR transmission in PUCCH, the UE terminate the ongoing PDCCH skipping and any running timer triggered by searchSpaceSwitchTimer-r17. *** Unchanged text is omitted ***

## Panasonic

- R1-2204072	Remaining issues for extensions to Rel-16 DCI-based power saving adaptation during DRX Active Time	Panasonic

Proposal 1: PDCCH skipping is not applied to Type 0/0A/1/2 CSS.

Proposal 2: PDCCH skipping applies to C-RNTI, MCS-C-RNTI and CS-RNTI in Type 3 CSS and USS.

Proposal 3: It needs to be clarified when PDCCH skipping is applied, whether SPS/CG is affected. For simpler UE behavior, active SPS/CG is not affected.

Proposal 4: If the UE is skipping PDCCH monitoring for a duration and at the first slot after the last OFDM symbol of a positive SR transmission in PUCCH, the UE stops PDCCH skipping  (i.e., PDCCH skipping is not activated deactivated).

## Apple

- R1-2204211	Remaining details on enhanced DCI-based power saving adaptation	Apple

Proposal 1: HARQ retransmission should be taken into consideration for PDCCH.

Proposal 2: UE is expected to perform PDCCH monitoring when drx_RetransmissionTimerDL is running.

Proposal 3: PDCCH skipping duration can apply to outside active time. No need to cancel the PDCCH skipping when DRX inactive timer expires.

Proposal 4: Confirm the working assumptions.

## NTT DOCOMO, INC.

- R1-2204348	Discussion on extension to DCI-based power saving adaptation	NTT DOCOMO, INC.

Proposal 1:  For how to reset SSSG switching timer, Working assumption should be comfirmed.

Proposal 2:  UE should monitor PDCCH when DRX Retransmission timer is running.

## Nokia, Nokia Shanghai Bell

- R1-2204605	Open issues on PDCCH monitoring adaptation for UE power saving	Nokia, Nokia Shanghai Bell

In Section 2 we discuss the looked the timer related behaviour and concluded as follows:

Proposal: Confirm the working assumption in agreement:

Agreement If a UE is provided group indexes for a Type3-PDCCH CSS set or a USS set by searchSpaceGroupIdList-r17 and a timer value by searchSpaceSwitchTimer-r17 for PDCCH monitoring on a serving cell and the timer is running, the UE - Resets the timer after a slot of the active DL BWP of the serving cell when the UE detects a DCI format in a PDCCH reception in the slot with CRC scrambled by - Working Assumption: C-RNTI/CS-RNTI/MCS-C-RNTI - Otherwise, decrease the timer value by one after each slot. - Note: There is no timer for SSSG switching is running for UE in SSSG#0

In Section 3 we considered other open aspects related to the PDCCH monitoring adaptation for certain MO originated behaviours, in light of the RAN2 LS [2], we observe and propose in Section 2.2:

Proposal: Capture following text to TS38.213 Section 10.4:

If UE has pending SR(s) in a serving cell [14, TS38.321], the UE is not allowed apply PDCCH monitoring skipping in cells belonging to the cell group of the serving cell.

Observation: It could be considered, for simplicity and system robustness that PDCCH skipping is cancelled when RACH procedure is triggered and PDCCH monitoring adaptation field indicating skipping is ignored when ra-ResponseWindow, msgB-ResponseWindow or ra-ContentionResolutionTimer is running.

Observation: In case of BFR via CFRA, PDCCH skipping should not affect the PDCCH monitoring in recovery search space to enable completion of procedure. Also, to enable further beam management actions, UE should resume normal monitoring after BFR.

Proposal: Agree that when RACH procedure is triggered or running, UE should to stop the PDCCH skipping and not apply the skipping if indication is received while RACH procedure is running and adopt following TP to TS38.213 Section 10.4:

## 10.4 Search space set group switching and skipping of PDCCH monitoring [text omitted]. If random access procedure is initiated as described in Section 8, and UE is skipping the PDCCH monitoring for a duration, the UE shall stop the skipping of PDCCH monitoring. iWhen ra-ResponseWindow, msgB-ResponseWindow or ra-ContentionResolutionTimer is running [14, TS38.321], UE shall ignore the PDCCH monitoring adaptation field in DCI indicating PDCCH monitoring skipping for a duration.

Observation: If it is not acceptable to cancel skipping or ignore skipping indications upon/during RACH procedure, stopping the skipping for the duration of the timers can be considered as per following TP to TS38.213 Section 10.4:

## 10.4 Search space set group switching and skipping of PDCCH monitoring [text omitted]. If ra-ResponseWindow, msgB-ResponseWindow or ra-ContentionResolutionTimer is running [14, TS38.321], the UE shall not apply skipping in PDCCH monitoring.

In Section 4, for the application delay we make following proposal:-

Proposal: The SSSG switching delay is not affected by minimum scheduling slot offset restriction.

Proposal: Agree that the supported values for SSSG switching delay, searchSpaceSwitchDelay, for 480kHz and 960kHz, are scaled by factor the supported values for 120kHz. The scaling factor is selected among {[4x and 8x]} or by {[8x and 16x]} for 480kHz and 960kHz respectively.

Observation: New parameter may be needed for Rel-17 SSSG switching delay to differentiate the configuration from Rel-16 SSSG switching delay.

The operation in relation to HARQ feedback and pending UL transmissions is discussed in Section 5 and we observe and propose as follows:-

Proposal: If PDCCH skipping duration overlaps with drx-RetransmissionTimerDL/UL, UE should stop skipping and resume normal PDCCH monitoring for the duration of drx-RetransmissionTimerDL/UL.

Proposal: Clarify the UE behaviour, in case of PDCCH monitoring during the drx-RetransmissionTimerDL/UL during PDCCH skipping duration, is determined by the PDCCH monitoring adaptation indication received in DCI during the re-transmission timer.

To conclude the interaction with C-DRX operation in Section 6 we make following observations and proposals:-

Proposal: As the work item determines the scope to be DRX Active Time, it should be confirmed that Rel-17 PDCCH monitoring adaptation is configured only together with C-DRX.

Proposal: Adopt following TP to Section 10.4 of TS38.213 to reflect the agreement on PDCCH monitoring skipping outside Active Time:

## 10.4 Search space set group switching and skipping of PDCCH monitoring [text omitted] When the PDCCH monitoring adaptation field indicates to a UE to skip PDCCH monitoring for a duration on the active DL BWP of a serving cell, the UE starts skipping of PDCCH monitoring at the beginning of a first slot that is after the last symbol of the PDCCH reception providing the DCI format with the PDCCH monitoring adaptation field. The PDCCH monitoring skipping can be applied only during Active Time [11, TS 38.321].

Proposal: Agree either that PDCCH skipping duration expires upon expiry of inactivity timer (drx-InactivityTimer) or that PDCCH skipping duration timer is decremented independently of C-DRX Active time.

Proposal: If it is agreed that PDCCH skipping duration expires upon expiry of inactivity timer (drx-InactivityTimer), adopt following TP to Section 10.4 of TS38.213:

## 10.4 Search space set group switching and skipping of PDCCH monitoring [text omitted] When the PDCCH monitoring adaptation field indicates to a UE to skip PDCCH monitoring for a duration on the active DL BWP of a serving cell, the UE starts skipping of PDCCH monitoring at the beginning of a first slot that is after the last symbol of the PDCCH reception providing the DCI format with the PDCCH monitoring adaptation field. The duration for PDCCH monitoring skipping expires when drx-InactivityTimer expires [11, TS 38.321].

## LG Electronics

- R1-2204621	Remaining issues on DCI-based power saving adaptation during DRX ActiveTime	LG Electronics

Observation 1: Based on TS38.213 and TS38.321, legacy UE monitors PDCCH candidates for a DCI with CRC scrambled by C-RNTI in a Type0/0A/1/2-PDCCH CSS set for a duration.

Observation 2: If monitoring behavior of Observation 1 is applied without modification to a UE indicated to skip for a duration, it will decrease power saving efficiency of monitoring adaptation.

Observation 3: If a UE does not monitor Type0/0A/1/2-PDCCH CSS set, a UE has the opportunities for turning off RF chains which gives benefits of power saving.

Proposal 1: Support that a UE skips monitoring of PDCCH candidates for DCI format 0_0 and DCI format 1_0 with CRC scrambled by C-RNTI, MCS-C-RNTI, or CS-RNTI in any CSS and USS when UE is indicated skipping PDCCH monitoring for a duration.

Proposal 2: Support that a UE monitor PDCCH for retransmission scheduling while DRX Retransmission timer is running even if a UE is in skipping duration.

Proposal 3: PDCCH monitoring adaptation should be terminated upon expiry of drx-InactivityTimer (or when UE enters outside DRX Active Time).

## MediaTek Inc.

- R1-2204705	Maintenance on DCI-based PDCCH Monitoring Adaptation	MediaTek Inc.

Observation 1: UE power saving gain with non-zero minK0/minK2 is from 1) allowing data processing unit(s) to stay in power saving mode and 2) saving RF reception of PDSCH by knowing no data scheduled earlier, and neither is related to PDCCH monitoring or processing.

Proposal 1: Application delay of R17 PDCCH monitoring adaptation scheme, i.e., SSSG switching or PDCCH skipping triggered by scheduling DCI(s), doesn’t depend on any minimum applicable scheduling offset.

Observation 2: Retransmission handling issue can be avoided by network implementation, e.g., applying lower MCS for the last TB or triggering skipping by a (dummy) UL scheduling after confirming all DL TBs are correctly received by UE. Although either of the above network solutions can be further optimized for reduced system overhead, such optimization can be left to Rel-18.

Proposal 2: No additional UE behavior for retransmission handling is included in Rel-17.

Proposal 3: PDCCH skipping is terminated when the RAR window/MsgB window/contention resolution timer is started or when SR is pending.

- Note: UE can be indicated to perform PDCCH skipping after RACH or SR is handled by network.

Proposal 4: PDCCH skipping is terminated when UE enters outside active time (i.e., Option 1b is supported)

- Note: skipping PDCCH monitoring of DRX on-duration is already achieved by Rel-16 DCI format 2_6

Proposal 5: Adopt the following TP to Section 10.4 of TS 38.213:

<Unchanged parts are omitted> When the PDCCH monitoring adaptation field indicates to a UE to skip PDCCH monitoring for a duration on the active DL BWP of a serving cell, the UE starts skipping of PDCCH monitoring at the beginning of a first slot that is after the last symbol of the PDCCH reception providing the DCI format with the PDCCH monitoring adaptation field. The UE terminates skipping of PDCCH monitoring whenever any of the following condition holds [11, TS 38.321]: - RAR window/MsgB window/contention resolution timer is started - SR is pending - Outside Active Time of a DRX group configured for the serving cell If a UE is provided group indexes for a Type3-PDCCH CSS set or a USS set by searchSpaceGroupIdList-r17 and a timer value by searchSpaceSwitchTimer-r17 for PDCCH monitoring on an active DL BWP of a serving cell and the timer is running, the UE <Unchanged parts are omitted>

Proposal 6: After active BWP change triggered by BWP timer expiration, UE applies the default PDCCH monitoring behavior and resets every SSSG timer according to the related configuration in the target BWP, if available

Proposal 7: If UE is indicated to change active BWP and PDCCH monitoring behavior at the same time,

- UE applies the indicated PDCCH monitoring behavior according to the configuration in the target BWP after the active BWP change

- UE counts the SSSG timer or PDCCH skipping duration after the active BWP change

Proposal 8: Adopt the following text proposal as a new paragraph in the end of Section 10.4 of TS 38.213:

<Unchanged parts are omitted> After active BWP change triggered by BWP timer expiration, UE applies the default PDCCH monitoring behavior and resets every SSSG timer according to the related configuration in the target BWP, if available. If UE is indicated to change active BWP and PDCCH monitoring behavior at the same time, the UE • applies the indicated PDCCH monitoring behavior according to the configuration in the target BWP after the active BWP change • counts the SSSG timer or PDCCH skipping duration after the active BWP change When the PDCCH monitoring adaptation field indicates to a UE to skip PDCCH monitoring for a duration on the active DL BWP of a serving cell, the UE starts skipping of PDCCH monitoring at the beginning of a first slot that is after the last symbol of the PDCCH reception providing the DCI format with the PDCCH monitoring adaptation field. <Unchanged parts are omitted>

Proposal 9: Confirm the following working assumption:

Agreement If a UE is provided group indexes for a Type3-PDCCH CSS set or a USS set by searchSpaceGroupIdList-r17 and a timer value by searchSpaceSwitchTimer-r17 for PDCCH monitoring on a serving cell and the timer is running, the UE - Resets the timer after a slot of the active DL BWP of the serving cell when the UE detects a DCI format in a PDCCH reception in the slot with CRC scrambled by - Working Assumption: C-RNTI/CS-RNTI/MCS-C-RNTI - Otherwise, decrease the timer value by one after each slot. - Note: There is no timer for SSSG switching is running for UE in SSSG#0

Proposal 10: Confirm the following working assumption:

Working assumptions: - When the UE receives DCI format 0_1 /1_1/0_2/1_2 with a ‘PDCCH monitoring adaptation indication' field in slot n, the UE does not expect to receive DCI format 0_1 /1_1/0_2/1_2 with a ‘PDCCH monitoring adaptation indication’ field indicating change to another SSSG or skipping for the same active BWP of the scheduling cell before slot n+X of the scheduling cell, where X is the value of the application delay. - If SSSG timer expires in slot n, the UE does not expect to receive DCI format 0_1 /1_1/0_2/1_2 with a ‘PDCCH monitoring adaptation indication’ field indicating change to another SSSG or skipping for the same active BWP of the scheduling cell before slot n+X of the scheduling cell, where X is the value of the application delay.

Proposal 11: Confirm the working assumption without FFS:

Working Assumption - Upon detecting a scheduling DCI format 1-1/1-2/0-1/0-2 indicating SSSG switching (i.e., Beh 2/2A/2B), - the UE applies SSSG switching on an active BWP of the serving cell at a first slot that is at least Pswitch symbols after the last symbol of the PDCCH reception - FFS: a minimum applicable scheduling offset is configured in the BWP - Note: Pswitch is defined in Table 10.4-1 in TS38.213

## Nordic Semiconductor ASA

- R1-2204746	On remaining issues of PDCCH monitoring adaptation	Nordic Semiconductor ASA

Proposal-1: Adopt the following TP in 38.213 sub-clause 10.4

A UE cancels PDCCH monitoring skipping, if any, and starts PDCCH monitoring at the beginning of a first slot or slot group that is after the last transmitted symbol of PRACH preamble or last transmitted symbol of PUCCH containing Scheduling request.

Proposal-2: Adopt the following TP in 38.213 sub-clause 10.4

When a UE is provided searchSpaceGroupIdList-R17, the UE resets PDCCH monitoring according to search space sets with group index 0.

Proposal-3: Adopt the following “blue addition” for TP in 38.213 sub-clause 10.4

When a UE is provided searchSpaceGroupIdList-R17 or when UE transmits PRACH preamble or PUCCH containing Scheduling request, the UE resets PDCCH monitoring according to search space sets with group index 0.

Proposal-4: Adopt the following “blue addition” for TP in 38.213 sub-clause 10.4

A UE cancels PDCCH monitoring skipping, if any, and starts PDCCH monitoring at the beginning of a first slot or slot group that is after the last transmitted symbol of PRACH preamble, last transmitted symbol of PUCCH containing Scheduling request or slot where DRX inactivity timer expires.

Proposal-5: Adopt the following TP for 38.213 sub-clause 10.4

Table 10.4-1: Minimum value of 𝑷𝒔𝒘𝒊𝒕𝒄𝒉 [symbols]

| 𝝁 | Minimum 𝐏𝐬𝐰𝐢𝐭𝐜𝐡 value for UE processing capability 1 [symbols] | Minimum 𝐏𝐬𝐰𝐢𝐭𝐜𝐡 value for UE processing capability 2 [symbols] |
| --- | --- | --- |
| 0 | 25 | 10 |
| 1 | 25 | 12 |
| 2 | 25 | 22 |
| 3 | 40 | 36 |
| 5 | 160 | 144 |
| 6 | 320 | 288 |

## Intel Corporation

- R1-2204774	Discussion on remaining issues of DCI-based PDCCH monitoring adaptation in active time	Intel Corporation

Observation 1: If PDCCH skipping duration is counted irrespective of active time or outside active time, it may potentially overlap with DRX ON of next DRX cycle, and motivation/benefit of this scenario is not clear.

Observation 2: Natural consequence of PDCCH skipping being applicable to active time “only” implies it should not apply when UE enters outside active time.

Proposal 1: Adopt the following TP in connection to capturing the RAN2 agreements on PDCCH skipping in RAN1 specification.

10.4 Search space set group switching and skipping of PDCCH monitoring *** Unchanged text is omitted *** When the PDCCH monitoring adaptation field indicates to a UE to skip PDCCH monitoring for a duration on the active DL BWP of a serving cell, the UE starts skipping of PDCCH monitoring at the beginning of a first slot that is after the last symbol of the PDCCH reception providing the DCI format with the PDCCH monitoring adaptation field, except during ra-ResponseWindow msgB-ResponseWindow ra-ContentionResolutionTimer or when SR is pending [11, 38.321] *** Unchanged text is omitted ***

Proposal 2: PDCCH skipping is terminated when UE enters outside active time.

Proposal 3: Interaction between application delay of PDCCH skipping and retransmissions is not supported in Rel-17.

## InterDigital, Inc.

- R1-2204828	Remaining issues on DCI-based power saving adaptation	InterDigital, Inc.

Proposal 1:

Case 1: Upon detecting a scheduling DCI format 1-1/1-2 indicating PDCCH skipping (i.e., Beh 1A), the UE performs PDCCH monitoring (i.e., Beh 1) when drx-RetransmissionTimerDL is running.

Case 2: Upon detecting a scheduling DCI format 0-1/0-2 indicating PDCCH skipping (i.e., Beh 1A), the UE performs PDCCH monitoring (i.e., Beh 1) when drx-RetransmissionTimerUL is running.

## Ericsson

- R1-2204960	Maintenance for Rel-17 UE PS : PDCCH monitoring adaptation	Ericsson

Proposal 1	UE will monitor PDCCH (i.e., PDCCH skipping is cancelled) when DRX Retransmission timer DL is running.

Proposal 2	Confirm that RRC parameter searchSpaceSwitchDelay is used to configure the Pswitch value for Rel-17 SSSG switching.

Proposal 3	For SSSG-switching, minimumSchedulingOffset can be considered as a factor in the SSSG-switching application delay.

Proposal 4	Select from below alternatives for application of PDCCH skipping duration outside active time

  - Alt1:

- PDCCH skipping is applied for active time overlapping the indicated skipping duration. PDCCH skipping duration decrements by slot irrespective it is in active time or not.

  - Alt 2:

- When DCI format 2_6 is configured,  PDCCH skipping is stopped when UE goes into DRX (i.e. into outside active time).

- When DCI format 2_6 is not configured, PDCCH skipping is applied for active time overlapping the indicated skipping duration and PDCCH skipping duration decrements by slot irrespective whether UE is in active time or not.

Proposal 5	At the first slot after the last OFDM symbol of a positive SR transmission in PUCCH, UE stops PDCCH skipping  (i.e., PDCCH skipping is not activated).

Proposal 6	UE stops PDCCH skipping during the RAR/MsgB window, and while contention resolution timer is running.

## Qualcomm Incorporated

- R1-2205006	DCI-based power saving adaptation during DRX ActiveTime	Qualcomm Incorporated

Proposal 1: If a UE transmits a positive SR or a Regular BSR during a PDCCH skip duration, the UE cancels PDCCH skipping.

Proposal 2: If a UE transmits a RA preamble for a CFRA or CBRA procedure during a PDCCH skip duration, the UE cancels PDCCH skipping.

Proposal 3: If a positive SR or a Regular BSR transmission cancels PDCCH skipping, the cancellation is applied only to the allowed serving cells (allowedServingCells ) for the logical channel(s) associated with the SR or BSR.

Proposal 4: If a RACH procedure cancels PDCCH skipping, the cancellation is applied only to the SpCell.

Proposal 5: During an indicated PDCCH skip duration, a UE does not monitor any PDCCH candidates for DCI formats with CRC scrambled by C-RNTI, MCS-C-RNTI, or CS-RNTI, in any CSS and USS.

Proposal 6: PDCCH skipping duration is counted irrespective of whether the slots are in DRX Active Time or not.

Proposal 7: During an indicated PDCCH skip duration, PDCCH monitoring activity only for DCI formats with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, CI-RNTI, INT-RNTI, SFI-RNTI, SP-CSI-RNTI, TPC-PUCCH-RNTI, TPC-PUSCH-RNTI, TPC-SRS-RNTI, or AI-RNTI in any CSS and USS is impacted.

Proposal 8: During a PDCCH skip duration, the UE will discontinuously monitor PDCCH according to drx-HARQ-RTT-TimerDL/UL and drx-RetransmissionTimerDL/UL.