| 3GPP TR 38.859 V0.0.0 (2022-05) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on Expanded and Improved NR Positioning; (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	8

4	General Descriptions of Expanded NR Positioning Enhancements	8

5	Sidelink Positioning	9

5.1	Sidelink Positioning Scenarios and Requirements	9

5.2	Potential Solutions for Sidelink Positioning	9

5.2.1	Physical Layer aspects for SL Positioning Solutions	9

5.2.2	Potential Architecture and Signalling Procedures for Sidelink Positioning	10

5.3	Summary of Sidelink Positioning Evaluations	10

5.5.1	Evaluation of Bandwidth Requirements to meet Identified Accuracy Requirements	10

5.5.2	Evaluation of Absolute Positioning, Relative Positioning, and Ranging Methods	10

6	Positioning Enhancements for Improved Integrity, accuracy, and power efficiency	11

6.1	Integrity for RAT-Dependent Positioning Techniques	11

6.1.1	Identification of error sources	11

6.1.2	Methodologies, procedures and signalling for determination of positioning integrity	11

6.1.3	Summary of Evaluation Results for Integrity for RAT-Dependent Positioning Techniques	11

6.2	PRS / SRS Bandwidth Aggregation	11

6.2.1	Potential Solutions Based on PRS / SRS Bandwidth Aggregation	11

6.2.2	Summary of Evaluations for PRS/SRS Bandwidth Aggregation	11

6.3	Carrier Phase Measurements	11

6.3.1	Potential Solutions for NR Carrier Phase Positioning	11

6.3.2	Summary of Evaluations for NR Carrier Phase Positioning	11

6.4	Low Power High Accuracy Positioning	11

6.4.1	Target use cases and requirements for Low Power High Accuracy Positioning	11

6.4.2	Summary of Evaluations for Low Power High Accuracy Positioning	11

6.5	Positioning of UEs with Reduced Capabilities	11

6.5.1	Potential Solutions for Positioning for RedCap UEs	11

6.5.2	Summary of Evaluations for Positioning for RedCap UEs	11

7	Conclusions	12

Annex A.1: Evaluation Methodology for Sidelink Positioning	12

Annex A.2: Evaluation Methodology for PRS/SRS Bandwidth Aggregation	12

Annex A.3: Evaluation Methodology for NR Carrier Phase Positioning	12

Annex A.4: Evaluation Methodology for Low Power High Accuracy Positioning	12

Annex A.5: Evaluation Methodology for Positioning for RedCap UEs	12

Annex B.1: Evaluation Results for Sidelink Positioning	12

Annex B.2: Evaluation Results for Integrity for RAT-Dependent Positioning Techniques	12

Annex B.3: Evaluation Results for PRS/SRS Bandwidth Aggregation	12

Annex B.4: Evaluation Results for NR Carrier Phase Positioning	13

Annex B.5: Evaluation Results for Low Power High Accuracy Positioning	13

Annex B.6: Evaluation Results for Positioning for RedCap UEs	13

Annex X: Change history	13

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document captures the findings of the study item "Study on Expanded and Improved NR Positioning" [7]. The purpose of this technical report is to document the requirements, additional scenarios, evaluations and technical proposals treated during the study and provide a way forward toward normative work on expanded enhancements to NR positioning in TSG RAN WGs.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	GPP TR 38.857: "Study on NR positioning enhancements".

[3]	3GPP TR 38.845: "Study on scenarios and requirements of in-coverage, partial coverage, and out-of-coverage NR positioning use cases".

[4]	3GPP TS 22.261: "Service requirements for the 5G system".

[5]	3GPP TR 22.855: "Study on ranging-based services".

[6]	3GPP TS 22.104: "Service requirements for cyber-physical control applications in vertical domains".

[7]	RP-213588: “New SID on Study on expanded and improved NR positioning”.

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [6] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [6].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [6] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [6].

AGV	Automated Guided Vehicle

BW	Bandwidth

DL	Downlink

GNSS	Global Navigation Satellite System

IIoT	Industrial Internet of Things

IoT	Internet of Things

ITS	Intelligent Transportation Systems

LPHAP	Low Power High Accuracy Positioning

NR	New Radio

PRS	Positioning Reference Signal

RAN	Radio Access Network

RAT	Radio Access Technology

RedCap	Reduced Capability

RTK	Real Time Kinematic

SI	Study Item

SID	Study Item Description

SL	Sidelink

SRS	Sounding Reference Signals

TR	Technical Report

TS	Technical Specification

UE	User Equipment

UL	Uplink

V2X	Vehicle to Everything

WI	Work Item

# 4	General Descriptions of Expanded NR Positioning Enhancements

In Release 17, 3GPP RAN conducted studies on "NR positioning enhancements" TR 38.857 [2] and "Scenarios and requirements of in-coverage, partial coverage, and out-of-coverage NR positioning use cases" TR 38.845 [3].

The study on "Scenarios and requirements of in-coverage, partial coverage, and out-of-coverage NR positioning use cases" focussed on V2X and public safety use cases with the outcome being captured in TR 38.845 [3]. Additionally, SA1 has developed requirements in TS 22.261 [4] for "Ranging based services” TR 22.855 [5] and has developed positioning accuracy requirements in TS 22.104 [6] for IIoT use cases in out-of-coverage scenarios. There is a need for 3GPP to study and develop sidelink positioning solutions that can support the use cases, scenarios and requirements identified during these activities.

The study on "NR positioning enhancements" TR 38.857 [2] investigated higher accuracy, and lower latency location, high integrity and reliability requirements resulting from new applications and industry verticals for 5G. Some of the enhancements identified during that work have been specified during the Release 17 Work Item on "NR positioning enhancements", but there remain a number of opportunities for enhancement that have not yet been incorporated into the specifications.

Regarding higher accuracy, two promising techniques identified in earlier studies will be considered in Release 18: one is to take the advantage of the rich 5G spectrum to increase the bandwidth for the transmission and reception of the positioning reference signals based on PRS/SRS bandwidth aggregation for intra-band carriers, and the other is to use the NR carrier phase measurements. GNSS carrier phase positioning has been used very successfully for centimetre-level positioning but is limited to outdoor applications. NR carrier phase positioning has the potential for significant performance improvements for indoor and outdoor deployments in comparison with the existing NR positioning methods, as well as shorter latency and lower UE power consumption in comparison with RTK-GNSS outdoors.

Positioning integrity is a measure of the trust in the accuracy of the position-related data and the ability to provide timely warnings based on assistance data provided by the network. The focus in Release 17 work was on GNSS integrity, and for Release 18 it is natural to extend this to address other positioning techniques as well as there are relevant integrity aspects of mission critical use cases that rely on positioning estimates and the corresponding uncertainty estimate. Integrity enables applications to make the correct decisions based on the reported position, e.g., when monitoring a robotic arm to decide whether its arm movement are within allowed limits to ensure safety distances to humans and other objects.

SA1 has introduced requirements for LPHAP (Low Power High Accuracy Positioning) for industrial IoT scenarios including use cases such as massive asset tracking, AGV tracking in industrial factory and person localization in danger zones. The SA1 requirements are for high accuracy and extreme low power consumption with battery life sustainable up to one or more years. A typical scenario of interest is use case 6 as defined TS 22.104 [6], which corresponds to tracking of workpiece (in- and outdoor) in assembly area and warehouse with a target accuracy of <1m, a positioning interval of 15-30 seconds, and a battery life of 6-12 months. While Release 17 NR positioning has introduced support for positioning in RRC_INACTIVE state, there is a need to evaluate whether the current system allows LPHAP requirements to be met.

Release 17 has specified support for RedCap UEs with reduced bandwidth support and reduced complexity including reduced number of receive chains. Such UEs could support NR positioning functionality but there is a gap in that the core and performance requirements have not been specified for the positioning related measurements performed by RedCap UEs, and no evaluation was performed to see how the reduced capabilities of RedCap UEs might impact eventual position accuracy. This gap is to be investigated by the present SI.

# 5	Sidelink Positioning

## 5.1	Sidelink Positioning Scenarios and Requirements

The following objectives were captured in SID [7] for study of sidelink positioning solutions:

Coverage scenarios to cover:

In-coverage, partial-coverage and out-of-coverage.

Requirements:

Based on requirements identified in TR 38.845 [2] and TS 22.261 [3] and TS 22.104 [5].

Use cases:

V2X (TR 38.845) [2], public safety (TR 38.845) [2], commercial (TS 22.261) [3], IIOT (TS 22.104) [5].

Spectrum:

ITS, licensed

## 5.2	Potential Solutions for Sidelink Positioning

### 5.2.1	Physical Layer aspects for SL Positioning Solutions

### 5.2.2	Potential Architecture and Signalling Procedures for Sidelink Positioning

## 5.3	Summary of Sidelink Positioning Evaluations

### 5.3.1	Evaluation of Bandwidth Requirements to meet Identified Accuracy Requirements

### 5.3.2	Evaluation of Absolute Positioning, Relative Positioning, and Ranging Methods

## 5.4	Potential specification impact for Sidelink Positioning

# 6	Positioning Enhancements for Improved Integrity, accuracy, and power efficiency

## 6.1	Integrity for RAT-Dependent Positioning Techniques

### 6.1.1	Identification of error sources

### 6.1.2	Methodologies, procedures and signalling for determination of positioning integrity

### 6.1.3	Summary of Evaluation Results for Integrity for RAT-Dependent Positioning Techniques

### 6.1.4	Potential Specification Impact for Integrity for RAT-Dependent Positioning Techniques

## 6.2	PRS / SRS Bandwidth Aggregation

### 6.2.1	Potential Solutions Based on PRS / SRS Bandwidth Aggregation

### 6.2.2	Summary of Evaluations for PRS/SRS Bandwidth Aggregation

### 6.2.3	Potential Specification Impact for PRS/SRS Bandwidth Aggregation

## 6.3	NR Carrier Phase Positioning

### 6.3.1	Potential Solutions for NR Carrier Phase Positioning

### 6.3.2	Summary of Evaluations for NR Carrier Phase Positioning

### 6.3.3	Potential Specification Impact for NR Carrier Phase Positioning

## 6.4	Low Power High Accuracy Positioning

### 6.4.1	Target use cases and requirements for Low Power High Accuracy Positioning

### 6.4.2	Summary of Evaluations for Low Power High Accuracy Positioning

### 6.4.3	Potential Specification Impact for Low Power High Accuracy Positioning

## 6.5	Positioning of UEs with Reduced Capabilities

### 6.5.1	Potential Solutions for Positioning for RedCap UEs

### 6.5.2	Summary of Evaluations for Positioning for RedCap UEs

### 6.5.3	Potential Specification Impact for Positioning for RedCap UEs

# 7	Conclusions

Annex A.1: Evaluation Methodology for Sidelink Positioning

Annex A.2: Evaluation Methodology for PRS/SRS Bandwidth Aggregation

Annex A.3: Evaluation Methodology for NR Carrier Phase Positioning

Annex A.4: Evaluation Methodology for Low Power High Accuracy Positioning

Annex A.5: Evaluation Methodology for Positioning for RedCap UEs

Annex B.1: Evaluation Results for Sidelink Positioning

Annex B.2: Evaluation Results for Integrity for RAT-Dependent Positioning Techniques

Annex B.3: Evaluation Results for PRS/SRS Bandwidth Aggregation

Annex B.4: Evaluation Results for NR Carrier Phase Positioning

Annex B.5: Evaluation Results for Low Power High Accuracy Positioning

Annex B.6: Evaluation Results for Positioning for RedCap UEs

Annex X: Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-05 | RAN1#109-e | R1-2205398 |  |  |  | Baseline TR skeleton. | 0.0.0 |