3GPP TSG RAN WG1 #109-e		R1-2205407

e-Meeting, May 9th – 20th, 2022

Source:	Moderator (OPPO)

Title:	Summary on UL precoding indication for multi-panel transmission

Agenda Item:	9.1.4.1

Document for:	Discussion and Decision

- Proposal for EVM

Proposal A: For the EVM of STxMP of Rel-18:

- Reuse the SLS assumption of BM/Multi-panel UE in R1-2007151 with necessary update, as shown in Table A1

- Reuse the LLS assumption of Rel-17 mTRP UL repetition transmission with necessary update, as shown in Table A2

- Note: company can evaluate FR1 and explain the details of EVM assumptions for that.

Table A1: SLS assumption for STxMP of Rel-18

| Parameters | Values |
| --- | --- |
| Frequency Range | FR2 @ 30 GHz, SCS: 120 kHz, BW: 80 MHz, |
| Scenarios | - Dense urban (macro-layer only, TR 38.913) @FR2, 200m ISD, 2-tier model with wrap-around (7 sites, 3 sectors/cells per cell), 100% outdoor - Indoor (TR 38.901/802) |
| UE speed | Option 1: Stationary UEs Option 2: 3 km/hr for all UEs Option 3: Dense Urban 100% outdoor UE with 30km/hr (optional) |
| Maximum UE Tx Power | - Option 1: Max TRP of 23dBm and max EIRP 43 dBm of two panels - Option 2: Max TRP of 23 dBm and max EIRP 43 dBm per panel Note 1: Companies to state their assumptions on this. Note 2: In Option 2, the max TRP of two panels might exceed the limit of PC2 in one band based on existing RAN4 power class definitions (which is currently per band). Companies to provide details whether Option 2 results in exceeding such limit in actual simulations. Note 3: In Option 1, companies to explain how max EIRP across two panels transmitting beams in different directions is determined. Note 4: In case that Option 2 is used and max TRP of two panels exceeds the limit of PC2, companies to provide the excess value of the TRP of STxMP transmission over the TRP of the single panel transmission baseline. |
| BS receiver Noise Figure | 7 dB |
| BS Antenna Configuration | For dense urban: (M, N, P, Mg, Ng) = (4, 8, 2, 2, 2). (dV, dH) = (0.5, 0.5) λ. (dg,V, dg,H) = (2.0, 4.0) λ For Indoor: (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1). (dV, dH) = (0.5, 0.5) λ Note: Other structure are optional and reported by company. Note: Companies to explain TXRU weights mapping. Note: Companies to explain beam selection. Note: Companies to explain number of BS beams |
| BS Antenna radiation pattern | TR 38.802 Table A.2.1-6, Table A.2.1-7 |
| UE antenna configuration | Option 1: Panel structure: 1x4x2 or (M, N, P) = (1, 4, 2), dH = 0.5 λ. Number of panels: 2, 3 or 4 Note: Companies to explain the number and locations of panels. Option 2: (M, N, P, Mg, Ng) = (2, 4, 2, 1, 2); (dV, dH) = (0.5, 0.5)λ. (dg,V, dg,H) = (0, 0)λ. * Θmg,ng=90°; Ω0,1=Ω0,0+180. The polarization angles are 0 and 90 Note: Other panel structure is optional and to be reported by company |
| UE Antenna radiation pattern | TR 38.802 Table A.2.1-8 |
| UE dropping | Random |
| UE and panel orientation | Vertical but random in azimuth |
| Traffic Model | Option 1: FTP model 1 with packet size 0.5Mbytes (other value is not precluded). Option 2: FTP mode 3 Other traffic models including the full buffer are optional and can be reported by the company |
| Control and RS overhead | Companies explain details of the assumptions |
| BF/Precoder scheme | Companies explain what scheme is used |
| UE Antenna height | 1.5 m |
| UL MIMO Mode, rank | UL SU-MIMO/MU-MIMO Up to rank 4 for STxMP with 2 panels. |
| Per panel power control and other issues that are affected by RF transmission chain architecture | Companies can explain the details of transmission chain architecture for supporting STxMP |
| Cross-link interference between 2 panels | Companies to explain whether/how the interference is assumed. |
| Baseline scheme | - Option 1: single panel transmission with panel selection. - Option 2: Rel-17 mTRP Uplink TDM repetition scheme with single panel per occasion. |

Table A2: LLS assumption for STxMP of Rel-18

| Parameters | Values |
| --- | --- |
| Frequency Range | FR2 @ 30 GHz, SCS: 120 kHz, |
| Channel model | CDL for FR2 (TDL for FR2 can be optionally used) |
| BS antenna configuration | 2 TRP and 2 Rx ports per TRP |
| UE antenna configuration | 2 panels and 2 Tx ports on each panel |
| Path-loss modeling | {0,3,6, 9, 12} dB gap between panels/TRPs |
| Blockage | Blockage model from Rel-16 (x dB power offset with probability p): Companies to report x and p, and other assumptions, if any. |
| Target BLER | [10^-3, 10^-4, 10^-5]: BLER values shown in plots should be based on enough number of samples, e.g., ~100/BLER samples |
| Baseline scheme | - Option 1: single panel transmission with panel selection. - Option 2: Rel-17 mTRP Uplink TDM repetition scheme with single panel per occasion. |
| # of RBs/symbols | Companies to Report. |
| DMRS pattern | - DM-RS configuration type 1 for PUSCH - DM-RS configuration type 2 for PUSCH (optional) - DM-RS according to PUCCH formats (companies to report the details for PUCCH DM-RS) |
| Code rates | Low (<0.2) and moderate (<0.4) |
| Frequency hopping | Reported by companies |
| UL transmission scheme | Codebook based UL transmission is baseline. Non-codebook based can be optional. |
| Redundancy Version | Reported by companies |
| Schemes | FDM-based, SDM-based and SFN scheme. |
| Cross-link interference between 2 panels | Companies to explain whether/how the interference is assumed. |
| Receiver assumption | Reported by companies |

- Proposal for mDCI-based STxMP PUSCH

Proposal 2: For multi-DCI based STxMP PUSCH+PUSCH transmission, study and evaluate the following aspects:

- Two PUSCHs are associated with different TRPs and transmitted from different UE panels. The total number of layers of these two PUSCHs is up to 4.

- Study STxMP of PUSCH+PUSCH transmission is applicable to which one(s) of DG-PUSCH, CG-PUSCH and msg3/msgA PUSCH.

- The overlapping type(s) of fully/partially in time domain and fully/partially/non-overlapping in frequency domain are to be studied and justified for PUSCH+PUSCH.

- The study shall take into the UE implementation and RF considerations.

- Study the conditions required for STxMP PUSCH+PUSCH.

- Proposals for STxMP PUCCH

Proposal 3.1: Study and evaluate the following schemes for STxMP PUCCH transmission in single-DCI based mTRP:

- PUCCH FDM-A scheme: different frequency domain parts of one PUCCH resource are transmitted by two different UE panels.

- PUCCH FDM-B scheme: two FDM-ed PUCCH transmission occasion of the same UCI with the same PUCCH format are transmitted from two different UE panels

- PUCCH SFN scheme: The same PUCCH/PUCCH-DMRS is transmitted from two different UE panels simultaneously.

- FFS: which one(s) PUCCH formats can support the PUCCH FDM-A, FDM-B, and SFN schemes

Concerns: Ericsson (out of scope), Huawei (lower priority than PUSCH, be ok with “out of scope” too)

Proposal 3.2: For multi-DCI-based mTRP system, study and evaluate transmitting two PUCCH resources to different TRPs with different UE panels that are fully or partially overlapping in time domain:

- Study which overlapping type(s) of fully/partially in time domain and fully/partially/non-overlapping in frequency domain are supported for PUCCH+PUCCH.

  - The study shall take into the UE implementation and RF considerations.

  - Study the conditions required for STxMP of PUCCH+PUCCH.

- Study which one(s) of PUCCH formats can support the PUCCH+PUCCH.

- Study the enhancement of UCI multiplexing/dropping rule for STxMP in multi-DCI based mTRP system if STxMP is supported.

  - FFS: whether/how to support UCI multiplexing/dropping for time-domain overlapped PUCCH and PUSCH transmitted from different panels.

Concerns: Ericsson (out of scope), Huawei (lower priority than PUSCH, be ok with “out of scope” too)

For easy reference, here is the objective 6 in Rel-18 MIMO WID:

- Study, and if needed, specify the following items to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable) - UL precoding indication for PUSCH, where no new codebook is introduced for multi-panel simultaneous transmission - The total number of layers is up to four across all panels and total number of codewords is up to two across all panels, considering single DCI and multi-DCI based multi-TRP operation. - UL beam indication for PUCCH/PUSCH, where unified TCI framework extension in objective 2 is assumed, considering single DCI and multi-DCI based multi-TRP operation - For the case of multi-DCI based multi-TRP operation, only PUSCH+PUSCH, or PUCCH+PUCCH is transmitted across two panels in a same CC.

- Proposals for sDCI-based PUSCH

Proposal 1.2: Study the enhancement of SRS resource set configuration and SRI/TPMI indication for STxMP PUSCH scheme:

- The configuration of two SRS resource sets, SRS resource set indicator field, two SRI fields and two TPMI fields of Rel-17 mTRP PUSCH TDM repetition is the starting point.

Proposal 1.3: Study to support the layer combinations of {1+1, 1+2, 2+1, 2+2} for the SDM scheme (if supported) of STxMP PUSCH,

- This is for 1 CW at least.

- The layer combination for the SDM scheme can be further studied for 2 CW if  2 CW in SDM scheme is supported.