3GPP TSG RAN WG1 #109-e	R1-2205447

e-Meeting, 09 – 20 May, 2022

Agenda item:	7.2.10

Source:	Moderator (Nokia)

Title:	[109-e-R16-MRDC-CA-01] Email discussion on NR DC in FR2

WI:	LTE_NR_DC_CA_enh-Core

Document for:	Discussion and Decision

# 1	Introduction

This document facilitates and summarizes the RAN1#109-e agenda item 7.2.10 Maintenance on Multi-RAT Dual-Connectivity and Carrier Aggregation enhancements (LTE, NR) discussion thread [109-e-R16-MRDC-CA-01].

[109-e-R16-MRDC-CA-01] Email discussion on NR DC in FR2, considering RAN4 LS in R1-2203030, by May 13 – Karri (Nokia) - Discuss whether/how to specify FR2 NR-DC power control - Intra-band case and inter-band case can be separately discussed - Discuss if/how to update the UE power sharing capability definitions

The related documents are listed below, a Liaison Statement received from RAN4 in R1-2203030 in response to RAN1 LS to RAN4 in R1-2104018 triggered three submissions to 7.2.10 as well as two submissions to AI5. A pre-meeting summary can be found in R1-2205127.

| TDoc | Title | Source | Agenda item |
| --- | --- | --- | --- |
| R1-2203030 | Reply LS on power control for NR-DC | RAN4, OPPO, vivo | 5 (incoming LS) |
| R1-2203187 | Discussion on power control for NR-DC for FR2 | ZTE | 7.2.10 |
| R1-2204198 | On remaining issues for NR-NR DC Power Control | Apple | 7.2.10 |
| R1-2204334 | Discussion on power control for NR-DC in FR2 | NTT DOCOMO, INC. | 7.2.10 |
| R1-2203496 | Discussion on power control for NR-DC in FR2 | vivo | 5 |
| R1-2204965 | Discussion on Reply LS from RAN4 on power control for NR-DC | Ericsson | 5 |
| R1-2205127 | [109-e-Prep-AI7.2.10 MR-DC/CA] Prep phase summary | Moderator (Nokia) | 7.2.10 |

# 2	Discussion

## 2.1	Whether/how to specify FR2 NR-DC power control

Three contributions propose defining UE behaviour for FR2 NR-DC power control when p-NR-FR2 is not configured (3187, 4198, 4334). One contribution proposes to define that NR-DC power control is not supported for FR2 in Rel-16/17 (3496), and by implication NR-DC on FR2 is not supported.

### 2.1.1	Round 1 (CLOSED)

Two broad alternatives exist, specify support, or specify that the configuration is not supported. There are different options how to realize each alternative.

Alt1: define UE behaviour for FR2 NR-DC power control when p-NR-FR2 is not configured.

- Opt1 (3187, 4198): Add the following text in 38.213 7.6.2

If a UE is configured with an MCG using NR radio access in FR2 and with a SCG using NR radio access in FR2 and is not configured with p-NR-FR2, the UE performs transmission power control as described in clauses 7.1 through 7.5 for each cell group and as described in [8-2, TS 38.101-2] across cell groups.

- Opt2 (4334): Add the following text in 38.213 7.6.2

Note: The UE does not expect to be configured with p-NR-FR2 in this release of the specification.

If a UE is configured with an MCG using NR radio access in FR1 or in FR2 and with a SCG using NR radio access in FR2 or in FR1, respectively, or if a UE is configured with an MCG using NR radio access in a FR2 band and with a SCG using NR radio access in another FR2 band and is not configured with p-NR-FR2, the UE performs transmission power control independently per cell group as described in clauses 7.1 through 7.5.

Alt2: define that FR2 NR-DC power control is not supported. 3496 outlines three different alternatives on this

- Opt1: make a conclusion that power control for NR-DC in FR2 is not supported

- Opt2: add a Note in TS.38.213 7.6.2 to indicate that power control for NR-DC in FR2 is not supported

Note: if a UE is configured with an MCG using NR radio access in FR2 and with a SCG using NR radio access in FR2, NR DC power control is not supported in this release of the specification.

- Opt3: remove the relevant descriptions on supporting power control for NR-DC in FR2 in TS.38.213 7.6.2

Please provide your company views on whether you prefer supporting NR-DC power control for FR2 (Alt1), or prefer defining that this is not supported in Rel-16/17 (Alt2), and if you have a view on which option under the preferred alternative should be taken.

| Company | Alt1/2 | Company comment |
| --- | --- | --- |
| Qualcomm | Alt.1 or neither | First of all, there is no reason not to support FR2 inter-band NR-DC. Alt.2 is too restrictive. Alt.1 maybe OK but further clarifications are necessary. We wonder whether we have the common understanding on the following aspects. - Both p-NR-FR1 and p-NR-FR2 are optional parameters that can cap the total power per cell-group of the FR. Therefore, NR-DC power control should work for FR1 (or for FR2) even when p-NR-FR1 (or p-NR-FR2) is not configured. This is common as for p-MaxEUTRA and p-NR-FR1 for EN-DC (to be discussed in [109-e-AI5-LSs-03]). - RAN4 LS indicates that they referred to FR2+FR2 UL-CA. According to RAN4 approved CR for FR2+FR2 UL-CA (R4-2206600), they have specified following. - Intra-band UL-CA: sum of EIRPs for the carriers in the band is capped by PUMAX (<= EIRPmax), sum of TRPs for the carriers in the band is capped by PTMAX (<= TRPmax). In other words, EIRP and TRP are shared by carriers in the band for intra-band UL-CA. - Inter-band UL-CA: there is no sum cap for EIRP and TRP across bands. However, as specified in 6.2A.2.2.3 and 6.2A.2.6, there is MPR for inter-band UL-CA. The value of MPR may depend on RB location, MCS, etc. A-MPR/P-MPR may also depend on the dynamic factors. As such, there is no EIRP/TRP sharing between the bands – however, max power determination would depend on decoding results of DCIs scheduling UL transmissions on the bands. With the above understanding, our proposal is following: - Conclude that RAN1 spec does not mandate both parameters p-NR-FR1 and p-NR-FR2. - RAN4 decided not to use p-NR-FR2. If they do not use it in their spec, then max power for a CG in FR2 is not capped by p-NR-FR2. RAN1 does not need to explicitly capture that p-NR-FR2 is not configured or not used. - Conclude that Toffset is necessary for FR2 inter-band NR-DC in Rel-16 - Although there is no power sharing between the CGs for FR2 inter-band NR-DC, RAN4 indicated that it is not “independent power control”, since MPR and so on would depends on RBs and/or MCS for the UL transmissions. To make sure that the UE can compute MPR and so on dynamically, similar handling as dynamic power sharing (ensuring the time gap, Toffset, for MCG scheduling) is necessary. We think if the above two points are concluded, perhaps almost no change is necessary in RAN1 spec. |
| ZTE | Alt.1 | Based on RAN4’s response LS, RAN4 doesn’t say FR2 NR-DC is not supported. Thus, we suggest to go with Alt.1. Within Alt.1, Opt 1 is more aligned with RAN4’s response since RAN4 mentioned power sharing for intra-band CA (NR-DC) and possible independent power control for inter-band CA (NR-DC). |
| CATT | Neither or Alt 1 | Alt 1 is good intention. However, we discussed in power control that NR-DC for FR2 does not have to have specification text since there is no SAR requirements for FR2. |
| vivo | Alt.2 | According to the latest LS from RAN4 (R4-2206566): … Up to now RAN4 hasn’t introduce NR-DC band combination for FR2, therefore UL CA is referred here and it is RAN4 understanding that same conclusion can be applied to NR-DC. … In addition, RAN4 do not plan to discuss p-NR-FR2 or p-UE-FR2 in Rel-17. … It is clearly that RAN4 does not support NR-DC in FR2 in Rel-16 and Rel-17. Define new UE behaviour is meaningless. RAN1 should accept the fact that NR-DC in FR2 does not work in Rel-16 and Rel-17. We prefer to make the specification clearer. In the meanwhile, we are open to discuss this issue in future release to check whether we need to update specification or not. |
| NTT DOCOMO | Alt 1 (Opt.1) or QCM proposal | We also prefer Alt.1 direction i.e., define UE behaviour for FR2 NR-DC power control when p-NR-FR2 is not configured. According to the contributions and above comments from companies, we think Opt.1 or QCM proposal would be good direction. |
| Intel | Alt. 2 | It is not clear how Alt. 1 works. Can proponent clarify if Alt. 1 is independent power sharing or not. If not, how UE can determine the TX power of each cell group when both CGs are simultaneously scheduled? How to report PHR for each CG so that gNB has a reference for UL scheduling for each CG? If the behavior cannot be clarified clearly, we think Alt. 2 is the right direction since RAN4 replies to RAN1 that RAN4 hasn’t introduce NR-DC band combination for FR2 till now. If a clear operation can be defined, we are fine to go with Alt. 1. |
| Ericsson1 |  | Given current status, the only update to RAN1 spec that reflects current status accurately at this stage seems to be to capture following note in 38.213 7.6.2 and leave rest of specification as is. Note: The UE does not expect to be configured with p-NR-FR2 in this release of the specification. With above clarification to spec, do not see strong need to make a further RAN1 conclusion that that power control for NR-DC in FR2 is not supported |
| Samsung | Alt. 2 | There would be ambiguities with the descriptions in 7.6.2 of 38.213 for NR-DC in FR2 and with p-NR-FR2 not being provided (and in other cases as mentioned by Intel). Given that for Rel-17 there is no NR-DC band combination defined in FR2, it should be sufficient to go with Alt.2 in RAN1 specs and avoid ambiguous specifications. |
| Huawei, HiSilicon |  | Since RRC p-NR-FR2 is not supported by RAN4 and is removed from RAN2 spec, any RAN1 text associated with the RRC parameter should be removed. Therefore, neither Opt. 1 nor Opt. 2 in Alt 1 is acceptable. If the concerned RAN1 text has been removed, we are open to discuss how to support of FR2-FR2 inter-band NR-DC. For any discussion of support of FR2-FR2 inter-band NR-DC, we suggest to discuss the following first, - Whether the cross-CG power control of FR2-FR2 inter-band NR-DC is specified or it is up to UE implementation. - If any solution to be specified, what is the maximum cap to determine when power sharing is needed, considering that RAN4 has never clearly defined the maximum power limitation in the sentence “when the maximum power limitation was reached” of the LS. - If no solution to be specified, how a gNB can send appropriate TPC commands for CGs to UEs? |
| Apple | Alt.1 | The intention of Alt.1 is to leave cross-CG power control/sharing for UE implementation, given the fact that RAN4 has no intention to define the p-NR-FR2. It should be noted that the per-UE upper bound has been defined for FR2 based on power class and can be used for FR2-FR2 NR-NR DC as well. Regarding the PHR issue raised by Intel, it is unclear for us. Note that the PHR is reported on a per CC basis and there is always per-CC maximum power configured by gNB and then P_c,max,c would be computed using the equation in TS 38.101. With the computed P_c,max,c, the PHR can be computed on a per CC basis and reported based on the scheduled PUSCH or reference format for virtual PHR for that CC. |

Summary after Round #1

- There appears to be no disagreement that in Rel-16/17 the UE cannot be configured with p-NR-FR2

- There is no consensus on whether this prevents RAN1 specifications from supporting FR2 NR-DC

  - 5 companies indicating that there is no issue from RAN1 perspective

  - 2 companies indicating that RAN1 specifications don’t need to be fixed when RAN4 doesn’t offer such band combinations and RAN4 is not supporting inter-band FR2 NR-DC

  - 1 company indicates that RAN1 specifications are not clear, although a rebuttal to the raised issue was brought up

### 2.1.2	Round 2 (CLOSED)

It appears that one thing that p-NR-FR2 cannot be configured, and what seems to be the contentious point is what does this really mean. Following two alternatives are put forward, that eliminate the p-NR-FR2 from TS38.213 7.6.2. The intention is not to state anything on whether the FR2 NR-DC is supported or is not supported by the RAN1 specifications. There would be no need for RAN1 to debate that.

Moderator proposal: Modify TS38.213 7.6.2:

- Alt 1: add: The UE does not expect to be configured with p-NR-FR2 in this release of the specification.

- Alt 2: remove all references to p-NR-FR2 as below

<Unchanged part omitted> If a UE is configured with an MCG and a SCG using NR radio access in FR1 and/or in FR2, the UE is configured a maximum power PMCG for transmissions on the MCG by p-NR-FR1 and/or by p-NR-FR2 and a maximum power PSCG for transmissions on the SCG by p-NR-FR1 and/or by p-NR-FR2 and with an inter-CG power sharing mode by nrdc-PCmode-FR1 for FR1 and/or by nrdc-PCmode-FR2 for FR2. The UE determines a transmission power on the MCG and a transmission power on the SCG per frequency range. If a UE is provided semi-static-mode1 for nrdc-PCmode-FR1 or for nrdc-PCmode-FR2, or semi-static-mode2 for nrdc-PCmode-FR1 or for nrdc-PCmode-FR2, the UE does not expect PMCG and PSCG to be configured such that PMCG+PSCG>PTotalNR-DC, where PMCG is the linear value of PMCG, PSCG is the linear value of PSCG, and PTotalNR-DC is the linear value of a configured maximum transmission power for NR-DC operation in FR1 or FR2 as defined in [8-3, TS 38.101-3]. If a UE is provided semi-static-mode1 for nrdc-PCmode-FR1 or for nrdc-PCmode-FR2, the UE determines a transmission power for the MCG or for the SCG as described in clauses 7.1 through 7.5 using PMCG or PSCG as the maximum transmission power, respectively. If a UE is provided semi-static-mode2 for nrdc-PCmode-FR1 or for nrdc-PCmode-FR2 - if the UE is not provided tdd-UL-DL-ConfigurationCommon for the MCG or SCG, the UE determines a transmission power for the MCG or for the SCG as described in clauses 7.1 through 7.5 using PMCG or PSCG as the maximum transmission power, respectively - if at least one symbol of slot i1 of the MCG or of the SCG that is indicated as uplink or flexible to a UE by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated, if provided, overlaps with a symbol for any ongoing transmission overlapping with slot i2 of the SCG or of the MCG, respectively, the UE determines a power for the transmission on the SCG or the MCG overlapping with slot i2 as described in clauses 7.1 through 7.5 using PSCG or PMCG, respectively, as the maximum transmission power - otherwise, the UE determines a power for the transmission on SCG or the MCG overlapping with slot i2, as described in [8-3, TS 38.101-3] and in clauses 7.1 through 7.5 without considering PSCG or PMCG respectively The UE expects to be provided semi-static-mode2 for nrdc-PCmode-FR1 or for nrdc-PCmode-FR2 only for synchronous NR-DC operation [10, TS 38.133]. If a UE - is provided dynamic for nrdc-PCmode-FR1 or for nrdc-PCmode-FR2, and - indicates a capability to support dynamic power sharing for intra-FR NR DC, <Unchanged part omitted>

Please provide your company view on whether you can accept eliminating p-NR-FR2 related configurations from TS38.213 7.6.2, and if you’d be OK with Alt1 or Alt2 (or some other solution), and if you have a clear preference over which way to do this.

| Company | Company comment |
| --- | --- |
| CATT | We can leave RAN1 spec as it is and don’t see the need of agreeing the proposal. |
| vivo | We prefer Alt 2. In our understanding, a similar description to Alt 1 can be added to the IE field description in 38.331, rather than 38.213. In addition, same as p-NR-FR1 is not configured for NR-DC in FR1, whether to define new UE behaviour in the case of p-NR-FR2 is not configured for NR-DC in FR2 is still open. If we adopt Alt 1, it seems we also have to address this issue, which is undesired. |
| Qualcomm | We share the view with CATT. Anyway, p-NR-FR1 and p-NR-FR2 are optional parameters (assume this is a common understanding). Then, it should be sufficient if p-NR-FR2 is not used in the RAN4 spec. |
| ZTE | The way suggested by CATT and Qualcomm is OK for us. We can also live with Alt.1. |
| NTT DOCOMO | We are fine with Alt.1 or suggestion from CATT. |
| Ericsson2 | We prefer Alt1. Regarding CATT proposal (i.e., leave RAN1 spec as is), if that is done, perhaps RAN1 can request RAN2 to update field description in RRC spec to reflect that the UE does not expect to be configured with p-NR-FR2 in this release of the specification (e.g., like below). Reflecting this aspect in the specifications avoids confusion. p-NR-FR2 The maximum total transmit power to be used by the UE in this NR cell group across all serving cells in frequency range 2 (FR2). The maximum transmit power that the UE may use may be additionally limited by p-Max (configured in FrequencyInfoUL) and by p-UE-FR2 (configured total for all serving cells operating on FR2). This field is only used in NR-DC. UE does not expect to be configured with this parameter in this release of the specification. Do not prefer Alt2 (e.g., no need to pre-emptively remove nrdc-PCmode-FR2 . RAN1can revisit if this is needed/not after RAN4 eventually finalizes power limits for FR2) |
| Apple | Support Ericsson proposal to make it clear by sending LS to RAN2 for field description update. RAN1 spec can be left without change. |

Summary after Round #2 (copied to section 3.1)

- There appears to be no disagreement that in Rel-16/17 the UE cannot be configured with p-NR-FR2

- There is no to modify RAN1 specification to reflect this

- A proposal to request RAN2 to add a note to the p-NR-FR2 field description “UE does not expect to be configured with this parameter in this release of the specification” could be agreeable.

## 2.2	If/how to update the UE power sharing capability definitions

Two contributions raise the point that the current UE capability has an issue with DC configurations where both FR1 and FR2 carriers are present (4334, 4965).

Current capability definitions from 38.306

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| intraFR-NR-DC-PwrSharingMode1-r16 Indicates whether the UE supports intra-FR NR DC with semi-static power sharing mode1 between MCG and SCG cells of same frequency range as defined in TS 38.213 [11]. If this field is absent, the UE does not support intra-FR NR DC. | BC | No | No | No |
| intraFR-NR-DC-PwrSharingMode2-r16 Indicates whether the UE supports semi-static power sharing mode2 between MCG and SCG cells of same frequency range for synchronous intra-FR NR DC as defined in TS 38.213 [11]. The UE indicating the support of this also indicates the support of intraFR-NR-DC-PwrSharingMode1-r16. | BC | No | No | No |
| intraFR-NR-DC-DynamicPwrSharing-r16 Indicates the UE support of dynamic power sharing for intra-FR NR DC between MCG and SCG cells of same frequency range with long or short offset as specified in TS 38.213 [11]. The UE indicating the support of this also indicates the support of intraFR-NR-DC-PwrSharingMode1-r16. | BC | No | No | No |

### 2.2.1	Round 1 (CLOSED)

It appears evident that the three power sharing modes the 38-306 capabilities are related to are not supported on FR2 cells, the capabilities should be modified to reflect this.

Moderator proposal: Send an LS to RAN2 asking the three capability definitions to be updated as follows:

- This capability is applicable for FR1 only. In case MCG and/or SCG have cells in different frequency ranges, this capability indicates the capability of the power sharing only between those MCG and SCG cells with UL in FR1.

Please provide your company views on the moderator proposal

| Company | Company comment |
| --- | --- |
| Qualcomm | We are OK with the moderator’s proposal. Then handling of FR2 NR-DC power-control depends on the outcome of discussion in Section 2.1. |
| ZTE | Ok to limit the existing UE capabilities to FR1. Then, do we need to add another three new UE capabilities for FR2 NR-DC? |
| CATT | We are OK with moderator’s proposal to limit the UE capability to FR1. |
| Vivo | Fine with the proposal. There is no need to discuss this capability for FR2 at this stage. |
| NTT DOCOMO | We are fine with moderator’s proposal for existing DC power sharing FGs. |
| Intel | We are OK with the moderator’s proposal. |
| Ericsson1 | Support Moderator Proposal. |
| Samsung | OK with the proposal. |
| Huawei, HiSilicon | OK with the proposal. No need to discuss new UE capability for FR2-FR2 NR-DC before a consensus is reached for S2.1. |
| Apple | Ok with the proposal. |

Summary after Round #1

- There is a consensus on updating the existing UE capability to limit the power sharing modes to FR1 only

As the existing UE capabilities are power about power sharing modes and there is no power sharing in FR2, these capabilities would seem to not be needed for FR2 no matter what the outcome is for FR2 NR-DC support.

### 2.2.2	Round 2 (CLOSED)

Moderator proposal: Liaise the following to RAN2

RAN1 Rel-16/17 specifications do not support any NR-DC power sharing between cell groups on FR2. RAN1 requests RAN2 to modify the NR-DC power sharing mode in TS38.306 (and potentially in TR38.822) as follows: - For capabilities intraFR-NR-DC-PwrSharingMode1-r16, intraFR-NR-DC-PwrSharingMode2-r16 and intraFR-NR-DC-DynamicPwrSharing-r16 (FGs 18-1/1a/1b): - In case MCG and/or SCG have cells in different frequency ranges, this FG indicates the capability of the power sharing only between those MCG and SCG cells with UL in FR1. - Note: above clarification for FG18-1/1a/1b does not mean that Rel-16 Ues are mandated to support power sharing mechanisms like FG18-1/1a/1b for FR2-FR2 DC.

Please provide your company views on the moderator proposal

| Company | Company comment |
| --- | --- |
| vivo | OK with this proposal. |
| Qualcomm | The first sentence may cause confusion and starting from the second sentence should suffice. Suggest to delete “RAN1 Rel-16/17 specifications do not support any NR-DC power sharing between cell groups on FR2”. |
| ZTE | Similar view as Qualcomm, The first sentence can removed. |
| NTT DOCOMO | We are fine with the proposal and removing the first sentence as suggested by Qualcomm. |
| Ericsson2 | We suggest below updates to limit scope of LS to requesting capability updates. RAN1 Rel-16/17 specifications do not support any NR-DC power sharing between cell groups on FR2. RAN1 requests RAN2 to modify the NR-DC power sharing mode capabilities in TS38.306 (and potentially in TR38.822) as follows: - For capabilities intraFR-NR-DC-PwrSharingMode1-r16, intraFR-NR-DC-PwrSharingMode2-r16 and intraFR-NR-DC-DynamicPwrSharing-r16 (FGs 18-1/1a/1b): - In case MCG and/or SCG have cells in different frequency ranges, this FG indicates the capability of the power sharing only between those MCG and SCG cells with UL in FR1. Note: above clarification for FG18-1/1a/1b does not mean that Rel-16 Ues are mandated to support power sharing mechanisms like FG18-1/1a/1b for FR2-FR2 DC. |
| Apple | Support to delete first sentence, which should be fact already with the updated UE capabilities and field description as discussed in issue 1. |

Summary after Round #2 (copied to section 3.2)

- Consensus to send the LS to RAN2 with the deletion of the 1st sentence of the moderator proposal for Round #2

# 3	Conclusion

## 3.1 Whether/how to specify FR2 NR-DC power control

Summary after Round #2 (copied from section 2.1.2)

- There appears to be no disagreement that in Rel-16/17 the UE cannot be configured with p-NR-FR2

- There is no to modify RAN1 specification to reflect this

- A proposal to request RAN2 to add a note to the p-NR-FR2 field description “UE does not expect to be configured with this parameter in this release of the specification” could be agreeable.

Moderator proposal for conclusion:

- Request RAN2 to add a note to the p-NR-FR2 field description “UE does not expect to be configured with this parameter in this release of the specification” could be agreeable

LS to RAN2, RAN4 sent out in R1-2205448

## 3.2 If/how to update the UE power sharing capability definitions

Summary after Round #2 (copied to section 2.2.2)

- Consensus to send the LS to RAN2 with the deletion of the 1st sentence of the moderator proposal for Round #2

Moderator proposal for conclusion: (note the added text on top of the round#2 proposal to make the sentence more sensible)

Moderator proposal: Liaise the following to RAN2

RAN1 requests RAN2 to modify the NR-DC power sharing mode related capabilities in TS38.306 (and potentially in TR38.822) as follows: - For capabilities intraFR-NR-DC-PwrSharingMode1-r16, intraFR-NR-DC-PwrSharingMode2-r16 and intraFR-NR-DC-DynamicPwrSharing-r16 (FGs 18-1/1a/1b): - In case MCG and/or SCG have cells in different frequency ranges, this FG indicates the capability of the power sharing only between those MCG and SCG cells with UL in FR1. - Note: above clarification for FG18-1/1a/1b does not mean that Rel-16 Ues are mandated to support power sharing mechanisms like FG18-1/1a/1b for FR2-FR2 DC.

LS to RAN2, RAN4 sent out in R1-2205448