3GPP TSG RAN WG1 #109-e                                                                                   R1- 2205479

e-Meeting, May 9th – 20th, 2022

Source:	Moderator (CATT)

Title:	Endorsed TPs in Rel.17 NR FeMIMO maintenance: Item 2c mTRP BM

Agenda Item:	8.1.2

Document for:	Decision

Summary of endorsed TPs

## TP for TS 38.213 (Issue #7: Report of cell-specific BFR and TRP-specific BFR in enhanced BFR MAC-CE)

Reason of change: The following agreement was achieved in RAN1#104b-e meeting: Agreement Support simultaneous configuration of cell-specific BFR and TRP-specific BFR in different CCs. In current 38.321, the BFR information of a first serving cell with cell-specific BFR and a second serving cell with TRP-specific BFR can be reported in a same enhanced MAC-CE, but it can’t be reported according to 38.213. Summary of change: It should be clarified that the second PUSCH MAC CE can also include BFR information of a serving cell with cell-specific BFR configuration. In addition, the condition to use the second PUSCH MAC-CE is that there is at least one serving cell configured with TRP-Specific BFR. Consequence if not approved: BFR information for a first serving cell with cell-specific BFR configuration and a second serving cell with TRP-specific BFR configuration has to be reported in different MAC-CEs. It leads to unnecessary overhead of MAC-CE. In addition, it doesn’t match with the BFR reporting method in 38.321.

================= Start of the TP for TS 38.213================================== - 6. Link recovery procedures <unchanged parts omitted> If there is at least one serving cells associated with sets q0,0 and q1,0, and with sets q0,1 and q1,1, the UE can provide in a second PUSCH MAC CE index(es) for cell(s) with q0, and/or with at least one of q0,0 and q0,1 having radio link quality worse than Qout,LR, the index(es) of those q0,0 and/or q0,1, and indication(s) of presence of qnew and of index(es) qnew, if any, from q1, and/or corresponding sets q1,0 and/or q1,1 for the serving cells. <unchanged parts omitted> ================= End of the TP for TS 38.213==================================

## TP for TS 38.213 (Issue #4&5: Configuration/update of explicit BFD-RS set)

Reason of change: In RAN1#108-e meeting, the following agreement was achieved to support to configure/update explicit BFD-RS set by RRC signaling and MAC CE signaling. Agreement Support to configure/update explicit BFD -RS set by RRC signaling and MAC CE signaling.. However, in current specification, RRC parameters related to BFD-RS sets have not been captured yet. Summary of change: Add RRC parameters failureDetectionSet1-r17 and failureDetectionSet2-r17. Consequence if not approved: RRC parameters related to BFD-RS sets are absent.

================= Start of the TP for TS 38.213================================== - 6. Link recovery procedures <unchanged parts omitted> A UE can be provided, for each BWP of a serving cell, a set q0 of periodic CSI-RS resource configuration indexes by failureDetectionResourcesToAddModList and a set q1 of periodic CSI-RS resource configuration indexes and/or SS/PBCH block indexes by candidateBeamRSList or candidateBeamRSListExt or candidateBeamRSSCellList for radio link quality measurements on the BWP of the serving cell. Instead of the sets q0 and q1, for each BWP of a serving cell, the UE can be provided respective two sets q0,0 and q0,1 of periodic CSI-RS resource configuration indexes by failureDetectionSet1-r17 and failureDetectionSet2-r17 that can be activated by a MAC CE [11 TS 38.321] and corresponding two sets q1,0 and q1,1 of periodic CSI-RS resource configuration indexes and/or SS/PBCH block indexes by candidateBeamRSList1 and candidateBeamRSList2, respectively, for radio link quality measurements on the BWP of the serving cell. The set q0,0 is associated with the set q1,0 and the set q0,1 is associated with the set q1,1. <unchanged parts omitted> ================= End of the TP for TS 38.213==================================