3GPP TSG RAN WG1 Meeting #109-e	  R1-22xxxxx

e-Meeting, May 9 – 20, 2022

Agenda Item:	9.2.2.1

Source:	Moderator (Huawei)

Title:	Summary#5 of [109-e-R18-AI/ML-03]

Document for:	Discussion and Decision

Introduction

In RAN#94-e plenary meeting, a new SID on AI/ML for air-interface was approved for Rel-18 [1], where the use cases include CSI feedback enhancement, and evaluations would be performed for this use case:

Study the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact. Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels Note: the selection of use cases for this study solely targets the formulation of a framework to apply AI/ML to the air-interface for these and other use cases. The selection itself does not intend to provide any indication of the prospects of any future normative project. …… For the use cases under consideration: - Evaluate performance benefits of AI/ML based algorithms for the agreed use cases in the final representative set: - Methodology based on statistical models (from TR 38.901 and TR 38.857 [positioning]), for link and system level simulations. - Extensions of 3GPP evaluation methodology for better suitability to AI/ML based techniques should be considered as needed. - Whether field data are optionally needed to further assess the performance and robustness in real-world environments should be discussed as part of the study. - Need for common assumptions in dataset construction for training, validation and test for the selected use cases. - Consider adequate model training strategy, collaboration levels and associated implications - Consider agreed-upon base AI model(s) for calibration - AI model description and training methodology used for evaluation should be reported for information and cross-checking purposes - KPIs: Determine the common KPIs and corresponding requirements for the AI/ML operations. Determine the use-case specific KPIs and benchmarks of the selected use-cases. - Performance, inference latency and computational complexity of AI/ML based algorithms should be compared to that of a state-of-the-art baseline - Overhead, power consumption (including computational), memory storage, and hardware requirements (including for given processing delays) associated with enabling respective AI/ML scheme, as well as generalization capability should be considered. …… Note 1: specific AI/ML models are not expected to be specified and are left to implementation. User data privacy needs to be preserved. - Note 2: The study on AI/ML for air interface is based on the current RAN architecture and new interfaces shall not be introduced.

This document summarizes the key issues discussed under agenda item 9.2.2.1 based on the views in [2]-[26], and aims to discuss a set of issues for the evaluation of the AI/ML-based CSI feedback enhancements in RAN1#109-e.

Generic issues on evaluation methodology

Summary of views from companies

### 2.1-1: High-level suggestions to the CSI feedback evaluation

Sharing of the AI/ML details for evaluation: Ericsson, Nokia

- Ericsson: Along with evaluation results, the proponent should provide sufficient details about the experiment (e.g., data generation, feature extraction, AI/ML model design, training, validation, and testing) so that the whole experiment can be reproduced by persons skilled in the art of AI/ML and 3GPP CSI evaluations.

  - Share a high-level, academic-paper style, description of the AI/ML model architecture. Sufficient details should be provided so that person(s) skilled in AI/ML can reimplement the AI/ML model.

  - Share simulation parameters used to generate synthetic datasets for training, validation, and testing. Sufficient detail should be provided to enable persons skilled in 3GPP CSI evaluations to reproduce the most important characteristics of dataset(s).

  - Share important data preprocessing / feature extraction steps.

  - Share per-sample loss functions and loss functions (the overall objective function that was optimized). This can include, for example, regularization terms in the loss function.

- Nokia: Companies to specify their CSI feedback/prediction training parameters, ML algorithm configuration, associated pre-processing, output processing, and control mechanisms

  - Companies should provide information about the training, structure, and usage of the ML model.  The purpose for providing this detailed information about the ML algorithm is to not only understand how the ML algorithm is used in the system, but also to be able to reproduce and compare performance and complexity of different ML approaches as well as to compare to baseline non-ML methods.

| Parameter | Example values |
| --- | --- |
| Training data | Actual and estimated channel response, actual and estimated channel eigenvectors Source and size of training dataset (simulated channels, field data) |
| ML algorithm | Model topology, such as autoencoder, variational autoencoder Encoder/decoder layers Activation function, such as LeakyReLu Quantization method and feedback overhead Training and validation procedures, including epochs, batch size, and dataset split between training and validation. Loss function used during training (MSE, cosine similarity) |
| Required pre-processing | Estimate downlink channel, estimate strongest eigenvectors of the transmit covariance matrix |
| Output data-processing | Calculation of channel eigenvectors from an uncompressed channel, calculation of precoding weights from channel eigenvectors |
| Control mechanisms | Communication of encoder and decoder configurations and where control of the configuration is originated (e.g., at the UE or the gNB) |

Separate phases for the evaluation work plan: Samsung, CATT

- Samsung: Consider a two-phased approach for evaluation. Phase I to compare various AI/ML models and their gain for representative sub-use case selection by using non-conventional performance metrics such as NMSE, cosine similarity, etc., and Phase II to evaluate the gain of AI/ML schemes as compared to conventional benchmark schemes by using traditional metrics such as UPT, feedback overhead

- CATT: For the evaluation on AI/ML based CSI feedback, the following two phases are considered. Phase 1: Intermediate evaluation for AI model selection, with performance, model size and complexity of AI model considered. Phase 2: Final evaluation for system level performance verification.

Down-selection of sub use cases: LG

- LG: According to the sub-use cases for CSI feedback enhancement, AI/ML model, input/output data and KPI/metric are diverge. In order to reduce RAN1 workload, clarification and/or down-scoping of sub-use cases are needed.

### 2.1-2 Sub use cases that are evaluated/considered

The sub use cases that have been raised/evaluated by companies can be categorized into 4 classes, where for the CSI compression sub use case, it further include CSI compression in spatial-frequency domain, CSI compression in spatial-frequency-time domain, and joint CSI compression and CSI prediction, which all adopt a two-sided AI/ML structure. For the CSI prediction, it further include CSI prediction in spatial domain and CSI prediction in time domain.

- Sub use case 1: CSI compression. Huawei, Hisilicon, Apple, vivo, Ericsson, Nokia, OPPO, Samsung, Qualcomm, CATT, LG, Xiaomi, CMCC, NTT DOCOMO, Beijing Jiaotong University, Charter, FUTUREWEI, NVIDIA, Lenovo, InterDigital, MediaTek Fujitsu

- Sub use case 2: CSI prediction. vivo, Nokia, Samsung, LG, FUTUREWEI, MediaTek

- Sub use case 3: CSI estimation enhancement. LG

  - LG: Normally, L-MMSE channel estimator is assumed where it uses values obtained from CSI-RS in time-frequency grid. If there is some mismatch in channel and noise covariance matrix, the performance in the L-MMSE estimator will be degraded. Instead of L-MMSE channel estimator, AI/ML can be applied to further improve the estimation performance.

- Sub use case 4: CSI interpolation. Fujitsu

  - Fujitsu: UE is configured with a portion of the sub-bands for CSI feedback. An example is selecting every second sub-band. At the BS side, AI/ML is used to recover the CSI for the whole band. 50% overhead reduction can be achieved compared to the type-II codebook-based method. The performance gain is 7% in terms of cosine similarity.

### 2.1-3: Generic evaluation methodology

Simulation approach

Option 1: System level simulation is adopted/can be considered: Huawei, Hisilicon, Ericsson, vivo, Apple, Nokia, Intel, OPPO, Samsung, ZTE, Qualcomm, CATT, CMCC, Xiaomi, AT&T, Beijing Jiaotong University, Spreadtrum, NVIDIA, Fujitsu, MediaTek

Option 2: Link level simulation is adopted/can be considered: Xiaomi, CMCC, NTT DOCOMO, Charter, NVIDIA, InterDigital, Fujitsu, MediaTek

Option 3: Other approaches: NVIDIA

- NVIDIA: Additional simulation methodology for generating synthetic data, such as digital twins, should be explored for the development and evaluation of CSI feedback enhancement

EVM table

In particular, for system level simulation method, there can be two ways to establish the EVM as a baseline:

Option 1: Take EVM of R17 CSI enhancement as a baseline [27]: Huawei, HiSilicon, Ericsson, OPPO, LG

Option 2: Take EVM of R16 CSI enhancement as a baseline [28]: Nokia, OPPO, Intel, ZTE, Spreadtrum, FUTUREWEI

Some parameters that are differently assumed from the values in the R16/R17 EVM table

- Mix of multiple configurations for dataset construction: Ericsson

- Frequency range: ZTE

  - ZTE: 2GHz (on top of R16 EVM)

- UE speed

  - Ericsson: If the AI/ML model is trained to perform CSI prediction, then consider revising the UE speed parameter from section 4 of [27], e.g. use a parameter sweep X=3,10,20,30 km/h where all UEs use the same speed X and all UEs are outdoor

  - Apple: 3km/h is adopted

- UE distribution

  - Nokia: Multiple indoor/outdoor UE ratios for training (Indoor vs Outdoor=100%vs0%, 80%vs20%, 50%vs50%, 20%vs80%, 0%vs100%) and inference (Indoor vs Outdoor=100%vs0%, 50%vs50%, 0%vs100%) are evaluated.

- Baseline for performance evaluation

  - Huawei, HiSilicon: Take R17 TypeII codebook as a baseline

- Traffic model (as well as traffic load)

  - Full buffer is assumed in preliminary assumptions/as a starting point: Huawei, HiSilicon, vivo, Ericsson, AT&T

- Channel estimation

Some companies mentioned that the channel estimation can be ideal as a starting point for evaluation, while some other company underlined the imperfect CSI due to channel estimation error needs to be considered.

  - Ideal downlink channel estimation is raised (as the starting point/ assumed in the EVM table). vivo, ZTE, Xiaomi, InterDigital

  - Imperfect CSI could be considered as performance KPIs for evaluation for CSI compression. Beijing Jiaotong University

- Feedback assumption

  - vivo: Ideal UCI feedback (no CSI bits are failed or wrong both for AI method and non-AI method)

### 2.1-4: Source to generate the dataset

The following summary on the source of the dataset includes both the explicit proposals from the contributions on the source to generate the dataset, and the actually adopted method for evaluations from the contributions though there is no explicit proposal.

Synthetic dataset for AI/ML training/validation/testing generated from TR38.901: Huawei, HiSilicon, vivo, Apple, OPPO, Samsung, Qualcomm, CATT, Xiaomi, CMCC, AT&T, NTT DOCOMO, Beijing Jiaotong University, Charter, NVIDIA, Lenovo, InterDigital, Fujitsu

- UMa as a basline: OPPO, CAICT, Fujitsu

- UMa, UMi channel models: Beijing Jiaotong University

- UMa, UMi, InH channel models: Huawei, HiSilicon, vivo, AT&T

- UMa, UMi, InH, RMa channel models: Lenovo

- CDL channels (CDL-A, CDL-B, CDL-C) for link level simulation: vivo, OPPO, Xiaomi, CMCC, NTT DOCOMO, Charter, InterDigital, Fujitsu

Dataset from field is adopted/can be considered in the evaluation: Nokia, OPPO, CAICT, AT&T, NVIDIA, Lenovo

- Nokia: Field data is used as training input in the evaluation.

- OPPO: Field test-based dataset can also be considered after its quality is examined

- CAICT: Field test data could be used for AI/ML model inference accuracy and generalization ability verification and will not be used for throughput comparison in SLS.

- AT&T: Depending on the sub-use case, testbed data can be shared or a public data set can be used for training and algorithm evaluation.

- NVIDIA: Companies are encouraged to contribute real data to develop and evaluate AI/ML based algorithms for CSI feedback enhancement.

- Lenovo: FFS: whether/how to collect field data that provides good representation of the environment

### 2.1-5: Calibration

Options for Calibration of the dataset and/or AI/ML model

- Option 1: Align the setup of the dataset construction phase and parameters for channel generation in the simulation: Huawei, HiSilicon, Samsung, ZTE, Xiaomi, AT&T Fujitsu

  - Huawei, HiSilicon: The calibration of the AI/ML model and dataset can be achieved by aligning the setup of the dataset construction phase and parameters for channel generation in the simulation, and calibrating the intermediate results such as generalized cosine similarity

  - Samsung: For a fair comparison, common assumptions can be taken in regards to determining scenarios, values, distribution of parameters, and simulation settings. Other meta properties of datasets, including dataset size, portioning for training, test and validation can also be agreed upon

  - ZTE: For ease of comparison, the data type & data size of the training data input to AI models are encouraged to be discussed, and aligned if possible.

  - Xiaomi: If open data set is not available, companies should provide the assumption of training set, validation set and test set respectively in the evaluation.

  - AT&T: Depending on the sub-use case, a data set can be constructed using common assumptions for training and algorithm evaluation

  - Fujitsu: Common simulation assumptions and parameters should be discussed and agreed for calibration purposes.

For Option 1, Metrics for calibration:

  - Cosine similarity: Huawei, HiSilicon, vivo, ZTE

  - Rank number

    - Rank distribution (regarding rank adaption): vivo

    - RI is fixed to 1 for calibration: Xiaomi

- Option 2: Share the dataset to public for cross check/calibration over companies. vivo, CAICT

  - vivo: Companies are encouraged to share the dataset and model files to public for cross check purposes

  - CAICT: The cross check for dataset is hard to operate by purely simulation parameter alignment. It is a straight way to directly provide the dataset online for double check. The upload and download of large size datasets for crosscheck should be considered on some public platforms where the datasets could be well maintained.

  - Company that feels no need for Option 2: Qualcomm

    - Qualcomm: For data generated using agreed 3GPP channel models, there is no need for calibration of the dataset across companies. For data from other sources, some statistics derived from the data may be reported along with the results.

- Option 3: Generate a common/open/shared dataset: Intel, Xiaomi, InterDigital, MediaTek

  - Intel: Consider a common data-set construction such that calibration or alignment of evaluation results across companies can be achieved. For this purpose a simplified KPI of precoder error metric (e.g. cosine similarity) can be used

  - Xiaomi: For AI data set, using an open data set of channel matrix (H) is proposed for calibration

  - InterDigital: Consider to use a common dataset for model training; similarly, use a common dataset for model performance evaluation

  - MediaTek: Construct the shared data set for channel samples to evaluate the performance of AI/ML-based MIMO CSI compression

- Option 4: Align the AI/ML model, e.g., for calibration purpose or taken as a reference. Samsung, CAICT, Xiaomi, Spreadtrum, NVIDIA

  - Samsung: Only for the model calibration in CSI compression, aligned loss function, hyper-parameter values, and details of the AI model are considered together

  - CAICT: Transformer based AI/ML model could be considered as reference model

  - Xiaomi: For AI model, a reference AI model is proposed as a baseline for calibration.

  - NVIDIA: Baseline AI model(s) should be identified for the purpose of calibration in the study of AI/ML based algorithms for CSI feedback enhancement

Non-ideal assumptions for real network

Ground-truth labels

- Ideal ground-truth label as a starting point for evaluation: Huawei, Hisilicon, ZTE

  - ZTE: In the initial evaluation, realistic sub-band eigenvectors under ideal channel estimation can be applied as the input of AI models for training.

- Further study training inputs fed back from UEs are quantized: Huawei, Hisilicon

  - Huawei, Hisilicon: Study the non-ideal aspects where training inputs (ground-truth labels of original CSI) fed back from UEs are quantized.

Dataset/model updating

- No AI/ML dataset/model updating (as a starting point): Huawei, Hisilicon, Lenovo

- Further study AI/ML model updating/fine-tuning after model deployment: Huawei, Hisilicon, Lenovo

  - Huawei, Hisilicon: Study the non-ideal aspects where AI/ML model updating or fine-tuning after model deployment

  - Lenovo: Each proposed CSI-feedback AI/ML model should be accompanied with a data collection procedure which can be used during the model update. Simulation assumptions should include definition of event(s) modelling changes in network settings or channel statistics during the simulation for evaluating AI Model adaptiveness to new conditions

### 2.1-6: Metrics

Intermediate KPIs

Cosine similarity, also named as generalized cosine similarity (GCS), has been widely considered over companies. The similarity can be also represented with its squared formula, i.e., Squared GCS (SGCS), which are also recommended by some companies. Equivalent MSE is also raised by one company which could be regarded as a formula operation of GCS from Moderator’s view.

An alternative is Mean square error (MSE) / normalized mean square error (NMSE), which evaluates the error/difference between two results (which can either be vectors or matrixes).

As mentioned by some company, the GCS is more beneficial on reflecting the relationship between vectors, so would be a good option for eigenvector compression. On the other hand, MSE/NMSE may be beneficial for matrix compression.

- Option 1: GCS/SGCS: Huawei, HiSilicon, vivo, Apple, Nokia, OPPO, Intel, Samsung, ZTE, CATT, LG, CAICT, Xiaomi, CMCC, Beijing Jiaotong University, Spreadtrum, Charter, FUTUREWEI, Lenovo, Fujitsu, MediaTek

  - GCS/SGCS is adopted/evaluated for CSI compression sub use case: Huawei, HiSilicon, vivo, Apple, Nokia, OPPO, Samsung, ZTE, CATT, LG, Xiaomi, CMCC, Spreadtrum, Charter, FUTUREWEI, Fujitsu, MediaTek

  - Cosine similarity is adopted/evaluated for CSI prediction sub use case: Nokia, vivo, Samsung, CATT, FUTUREWEI

  - Subband-level Cosine similarity: Huawei, HiSilicon, vivo, Apple, Nokia, OPPO, Samsung, ZTE, Xiaomi, CMCC, Charter, MediaTek

    - GCS: Apple, vivo, Huawei, HiSilicon, Nokia, Samsung, ZTE, FUTUREWEI, Fujitsu, MediaTek. wi is the original eigenvector of subband i, and wi is the reconstructed eigenvector of subband i at the decoder output. N is the total number of subband for PMI feedback. E{∙} denotes the average operation over multiple samples.

ρ=E1Ni=1N||wiHwi ||||wi||  ||wi||

    - SGCS: OPPO, vivo, ZTE, CAICT, Xiaomi, CMCC. wk is the real channel eigenvector for the k-th sub-band and wk' is the recovered channel eigenvector for the k-th sub-band. Nsp is the number of samples in testing set, Nsb is the number of sub-band.

ρ2= 1Nspnsp=0Nsp-11Nsbk=0Nsb-1wkHwk'wkwk'2

Xiaomi: If the AI-based CSI feedback is full channel feedback, SGCS is given in the following fomula, where K is the sub-band number, Hk is the original channel matrix of sub-band k and Hk' is the recovered channel matrix of sub-band k by AI

GCS2=E1Kk=1KHkH∙Hk'Hk2Hk'22

  - PRB-level Cosine similarity (GCS or SGCS): OPPO, ZTE

    - wk,j denotes the real channel eigenvector for the j-th RB in the k-th sub-band, and NRB is the number of RB in one sub-band.

ν2= 1Nspnsp=0Nsp-11Nsbk=0Nsb-11NRBj=0NRB-1wk,jHwk'wk,jwk'2

  - Subcarrier-level Cosine similarity: MediaTek

- Cosine similarity for multiple ranks: Apple, Huawei, HiSilicon

  - Option 1: average over all ranks, where  wji is the rank-j eigen-vector of subband i, and wji is the reconstructed rank-j eigenvector of subband i at the decoder output: Apple

ρ=E1NsNcj=1Nsi=1Nc||wjiHwji ||||wji||  ||wji||

  - Option 2: weighted average over all ranks: Apple

ρ=E1NsNcj=1Nsλji=1Nc||wjiHwji ||||wji||  ||wji||

  - Option 3: separately show the Cosine similarity for each rank: Huawei, HiSilicon

- Equivalent MSE is evaluated for CSI compression sub use case: Qualcomm

  - The equivalent MSE (per-RB level) is directly related to the cosine loss and defined as follows

Equivalent MSE v, v=2-2Cos(θ(v, v)).

where the cosine loss between vectors v (ideal singular vector) and v (reconstructed singular vector) is used as used as the training loss function and defined as follows

Cosine Loss v, v=-Cos(θ(v, v)).

- Option 2: Mean square error (MSE) / normalized mean square error (NMSE): vivo, Nokia, Samsung, LG, CAICT, Beijing Jiaotong University, FUTUREWEI

  - MSE/NMSE is adopted/evaluated for CSI compression sub use case：CATT, LG, FUTUREWEI

  - MSE/NMSE is adopted/evaluated for CSI prediction sub use case: Nokia, vivo, Samsung, CATT, LG, FUTUREWEI

    - L is the number of samples, Hl is the results derived by AI model and Hl is the corresponding ground truth results

MSE=1Ll=1LHl-Hl2=1Ll=1Ln=0N-1m=0M-1Hl,n,m-Hl,n,m2

NMSE=1Ll=1LHl-Hl2Hl2=1Ll=1Ln=0N-1m=0M-1Hl,n,m-Hl,n,m2n=0N-1m=0M-1Hl,n,m2

Sensitivity of inference accuracy

One company raises to consider the impact of noise to the accuracy of the CSI reconstruction accuracy, e.g., the ground truth labels.

- Lenovo: The inference accuracy should be reported for a range of noisy training CSI and also noisy input data for the proposed CSI-feedback mechanism

Label for comparing GCS/NMSE

One company raises if the channel estimation is realistic, then the GCS should be calculated based on the eigenvector form the ideal channel (instead of the UE estimated one).

- Fujitsu: The vectors from the ideal channel matrix should be used in the calculation of the cosine similarity

Eventual KPIs

- Throughput/spectral efficiency (via SLS): Huawei, HiSilicon, Ericsson, vivo, Apple, Nokia, OPPO, Intel, Samsung, ZTE, Qualcomm, CATT, LG, CAICT, Xiaomi, AT&T, NTT DOCOMO, Beijing Jiaotong University, Spreadtrum, NVIDIA, Lenovo, Fujitsu

  - Nokia, AT&T: Mean and cell edge throughput

  - Samsung: Average UPT, 5% UPT, 50% UPT, 95% UPT

  - NVIDIA: e.g., average and 5-percentile throughput

- BLER (for LLS): Xiaomi, InterDigital

- Overhead

  - CSI feedback overhead: Ericsson, Nokia, OPPO, Intel, ZTE, Qualcomm, CATT, LG, AT&T, NTT DOCOMO

  - Reference signal overhead: Nokia

    - Nokia: For CSI prediction, overhead also includes both reference signal (in terms of Transmissions per second)

    - MediaTek: AI/ML-based solutions can enable the reduction of DMRS overhead.

  - Overhead in LCM: Intel, CATT, AT&T, Lenovo

    - Intel: Air-interface overhead due to data-collection, performance monitoring and feedback, model adaptation

    - CATT: Overhead of exchanging AI-specific signaling

    - AT&T: The training overhead and latency as well as the inference latency should be included as a KPI

    - Lenovo: Each proposed CSI-feedback AI/ML model should be accompanied with a data collection procedure which can be used during the model update. FFS: communication overhead. It is essential to quantify how often the model has to go through the model update procedure.

Capability/complexity related KPIs

- Processing complexity Huawei, HiSilicon, vivo, Nokia, Intel, Samsung, ZTE, CATT, CAICT, NTT DOCOMO, Beijing Jiaotong University, Spreadtrum, FUTUREWEI, Lenovo

  - Floating point operations (FLOPs): Huawei, HiSilicon, vivo, Nokia, Samsung, ZTE, NTT DOCOMO, Beijing Jiaotong University, Spreadtrum, FUTUREWEI

Except for FLOPs, the processing complexity is raised to be evaluated in other perspectives

  - Pre-processing complexity (e.g. SVD) for inference data: Intel

  - Model pruning and quantization (AI-ML model implementation related aspects): Intel

  - Power consumption: Huawei, HiSilicon, vivo, Samsung, Spreadtrum

- Memory storage Huawei, HiSilicon, vivo, Nokia, Intel, Samsung, ZTE, CATT, CAICT, Beijing Jiaotong University, Spreadtrum, FUTUREWEI

  - AI/ML model size/ Number of quantization bits: Huawei, HiSilicon, vivo, Nokia, Intel, Samsung, ZTE, CATT, Spreadtrum, FUTUREWEI

    - vivo: KPI for model transmission

    - FUTUREWEI: For training phase, it includes memory used to store outputs of the intermediate layers (forward propagation), parameters, error signals, gradient of parameters, additional memory used by the chosen optimizer, e.g., for momentum. For inference phase, it includes memory used to store outputs of the intermediate layers, and parameters.

  - Number of AI parameters/weights, quantization of weights/layer structure: vivo, Nokia, Intel, Spreadtrum, FUTUREWEI

    - vivo: KPI for model transmission

One company mentioned that the memory storage is also correlated with the generalization capability of the AI/ML model.

  - Spreadtrum: The storage requirement also depends on the generalization of AI/ML model. For example, if the generalization is poor, more AI/ML model should be storage.

One company mentioned that the complexity/latency may be impacted by tangled memory storage and FLOPs.

  - Samsung: Due to various levels of the suitability of an AI/ML models, e.g. LSTM, CNN, etc., for parallel processing, the inference latency does not have a direct relationship with computational complexity (number of FLOPs). To this end, a unified capability-related KPI which indicates the various aspects such as computational complexity [in a unit of FLOPs] and the model size [in a unit of bytes]) have to be studied.

- Performance impact of model size/FLOPs vivo, Samsung

Some companies have evaluated the impact of the model size to the model accuracy/performance. It shows that the AI/ML model with smaller size can largely relieve the burden of computation/memory but with the sacrifice of performance loss to some extent.

  - vivo: The small AI model saves more than 90% AI model size and at the cost of about 5% performance loss.

  - Samsung: In terms of the CS, the ViT-based AE outperforms the other AEs. However, in terms of the FLOPs, the 2D-CNN-based AE is quite efficient to perform the AI processing compared to other AEs

|  | CS performance | FLOPs |
| --- | --- | --- |
| ViT-based AE | 0.921 | 4 MFLOPs |
| bi-LSTM-based AE | 0.911 | 10 MFLOPs |
| 2D-CNN-based AE | 0.906 | 1 MFOPs |

The metric of latency is mentioned by a couple of companies. The latency normally means the inference latency, but two companies also raise to adopt training time/latency as a metric.

- Latency: Huawei, HiSilicon, Ericsson, vivo, Nokia, CATT, CAICT, AT&T, Beijing Jiaotong University, NVIDIA, Lenovo

  - Inference latency

    - Nokia: For CSI prediction, inference latency between CSI-RS transmission to inference of the predicted CSI

    - Ericsson: CSI reporting latency reduction

    - AT&T: The training latency as well as the inference latency should be included as a KPI

    - NVIDIA: It is important to report the KPIs together with the used computing platform (such as the GPU model)

  - Training/Model updating time: AT&T, Lenovo

    - Lenovo: If the proposed methods are not universal and need to be retrained based on some changes in the environment, discussion is needed on the approximate time required to update the model.

  - Object latency as a metric: Qualcomm

    - Qualcomm: Inference latency may be difficult to quantify as it would be implementation dependent

Other KPIs

- For CSI compression sub use case

  - Ability of a dual-side AI models for multi-vendor case: Ericsson

- For CSI prediction sub use case

  - Observation time: Nokia

Benchmark for evaluation results comparison

- TypeII: Huawei, HiSilicon, Ericsson, vivo, Apple, Nokia, OPPO, Intel, Samsung, ZTE, CATT, LG, Xiaomi, CMCC, NTT DOCOMO, Beijing Jiaotong University, Spreadtrum, InterDigital Fujitsu

  - R16 TypeII: Huawei, HiSilicon, Ericsson, vivo, Apple, Nokia, OPPO, Intel, Samsung, ZTE, CATT, LG, NTT DOCOMO, Beijing Jiaotong University, Spreadtrum Fujitsu

  - R17 TypeII: Huawei, HiSilicon, Samsung, CATT, LG

- Other benchmarks

  - Rel-16 Type I: OPPO, CMCC, InterDigital

  - Reference AI/ML model: Spreadtrum

- For CSI prediction sub use case

  - Option 1: Nearest historical CSI (without prediction): vivo, Samsung

  - Option 2: Kalman filtering: Nokia, Samsung

  - Option 3: R18 MIMO: MediaTek

  - Other options (extrapolation, etc.): Samsung

### 2.1-7: Generalization

As raised by some companies, dataset used for training an AI/ML model can be close to the deployment scenario for which the model will be used, and data / observations used in the testing phase is also drawn from the same distribution as the training data. On the other hand, with overfitting, the model is memorizing the features of the training set, rather than learning how to generalize to unseen samples, so always using the same dataset statistics for training, validation, and testing would cripple the generalization performance. Thus it would be beneficial if the performances of the scenario-specific AI/ML model and the generalized AI/ML model can be evaluated, to study the trade-off between scenario-specific and generalization.

- FUTUREWEI: For each (sub) use case, discuss whether a scenario-based modelling/training approach is expected, or a generalized modelling/training approach is preferred, or both should be supported

- InterDigital: Several scenarios are considered for the training datasets, including Single-channel synthetic training dataset (CDL-A), and Multi-channel synthetic training dataset (combination of CDL-A, CDL-B, CDL-C). They are separately applied to various channel types (CDL-A, CDL-B, and CDL-C) for testing/comparison.

- vivo: The baseline dataset can be made up of samples from all scenarios with the same number of samples. The performance of different combination ratio can be evaluated to compare with the baseline. Also, the dataset composed of samples from each single scenario is need as the upper bound.

Some specific methods to assess the generalization performance in evaluations are brought up by companies. Two principles are raised by companies, including:

Principle 1: The training data set is constructed by mixing data from different configurations/ scenarios

Principle 2: Training set and testing data set are from different configurations/scenarios

The first issue under Principle 1 and Principle 2 is what mixed configuration/scenarios and what configuration/scenario, respectively, should be assumed for constructing the dataset.

Issue 1: Assumptions on build a mixed dataset

A number of companies support generating mixed dataset and raise methods on how to achieve the mixed dataset for assessing the generalization.

Dataset generated from various/mixed configurations/scenarios: Huawei, HiSilicon, Ericsson, vivo, Nokia, OPPO, Intel, Samsung, InterDigital,

- Method 1: Same input dimension across different scenarios: vivo, Huawei, HiSilicon, Nokia, OPPO, Intel, Samsung, InterDigital,

- Method 2: Different input dimensions across different scenarios: Ericsson, vivo

  - Note that for the different input dimensions, some pre-processing may be used to resize the input data dimensions as raised by vivo.

Company views on mixed configurations/scenarios:

- Huawei, HiSilicon: The dataset for training the AI/ML model can be generated based mixed scenarios, and the dataset for testing is generated based on one of the mixed scenarios or another scenario.

- Ericsson: The dataset for AI model training should represent the large space of MIMO channels in cells where massive MIMO is likely to be deployed and where MU-MIMO is commonly used. A starting point can be to create a reference dataset training mix using these parameters:

  - A mix of 32 and 16 port CSI-RS

  - A mix of Dense Urban (Macro only) and Urban Macro and Urban Micro

  - A mix of 2 GHz and 4 GHz carrier frequencies using 15 and 30 kHz SCS respectively

  - A mix of 200m and 500m ISD

  - A mix of Indoor users (3 km/h), outdoor users (3 km/h) and outdoor users (30 km/h)

- vivo: Consider both cases with same or different input data dimensions for data set construction to verify generalization performance. An example is given as a mix of UMa/UMi and InH. Evaluation results shows a reasonable mixing ratio can provide good performance for each scenario.

- Nokia, Intel: Different indoor/outdoor UE ratios separately for training (e.g., Indoor vs Outdoor=100%vs0%, 80%vs20%, 50%vs50%, 20%vs80%, 0%vs100%) and inference (e.g., Indoor vs Outdoor=100%vs0%, 50%vs50%, 0%vs100%) are evaluated.

- Samsung: Consider datasets from mixed scenarios or different distributions of channel parameters in a single scenario. AI/ML model trained from outdoor dataset and applied for outdoor/indoor inference set, or AI/ML model trained from mixed outdoor&indoor dataset and applied for outdoor inference set.

- OPPO: The system-level UMa and UMi mixed channel dataset should be considered. Evaluation results shows AI/ML model trained on the mixed channel dataset with sufficient samples is comparable to that trained on the single channel dataset.

- Intel: Data-set should target a variety of data (statistical channel model data, measured or ray-tracing based channel model data), and target various channel model types (indoor, outdoor, UMa, UMi etc.)

- InterDigital: Mixed dataset with multi-channel synthetic training dataset (combination of CDL-A, CDL-B, CDL-C).

Then the second issue is what specific inference configuration/scenario should be assumed for verification.

Issue 2: Assumptions on scenarios/configurations to verify generalization

A number of companies support verify the generalization under varying scenarios/configurations and raise the candidate various scenarios/configurations.

Verify the generalization under varying scenarios/configurations: Ericsson, vivo, CATT, CAICT, Beijing Jiaotong University, Spreadtrum, Lenovo, InterDigital, Fujitsu

Company views on varying scenarios for evaluating generalization:

- vivo: generalization can be evaluated by

  - 1) changing the channel model (e.g., UMi, UMa, and Indoor for SLS, CDL-A, CDL-B, CDL-C, CDL-D, CDL-E for LLS),

  - 2) changing the parameters of the channel model (e.g., number of ports, bandwidth, antenna configuration, carrier frequency, delay spread, speed of UE, component of multipath),

  - 3) changing the resources on which the AI model applied (e.g., paths, subbands, PRBs, antennas, angles)

- Ericsson: The ability of the ML model to generalize to varying assumptions on gNB and UE implementation such as subarray size, directional UE antennas. User throughput difference compared to UE speed X=3 km/h at UE speeds X=10,20,30 km/h if the AI/ML model is trained to perform CSI prediction.

- CATT: When evaluating the generalization ability for AI/ML based CSI feedback compression, scenarios with different frequency selectivity shall be considered. When evaluating the generalization ability for AI/ML based channel prediction in time domain, scenarios with different time selectivity shall be considered

- CAICT: The generalization ability of AI/ML model for CSI feedback could be verified by using same AI/ML model structure to multiple scenarios

- Beijing Jiaotong University: Model generalization can be measured by verifying the performance of AI models in different test sets

- Spreadtrum: To reflect the generalization, scenario should be diverse as far as possibly, i.e., Urban Macro, UMi, and indoor should be included

- Lenovo: The inference accuracy of the model (without model updates) should be reported for different channel models including UMa, UMi, CDL (with various parameters). Evaluate the scalability of the model for different parameter settings, e.g., different number of antenna ports available at the UE or gNBs, subcarrier spacing, or channel bandwidth.

- InterDigital: AI/ML models are trained respectively by Single-channel dataset (CDL-A), and Multi-channel dataset (combination of CDL-A, CDL-B, CDL-C). The test is performed by applying Single-channel/ Multi-channel dataset to the CDL-A channel with the same parameter for inference, and to another channel with a different parameter of delay spread, channel type (CDL-B/C), or UE speed.

- Fujitsu: For AI/ML testing, it is preferable to test the robustness of the trained AI/ML model by applying it to various channels

Other issues on generalization

Some companies raise issues on the training complexity or performance if the dataset is too large or too inclusive.

- Issue 1: Challenges on model training

  - CAICT: With dataset construction based on multiple simulation scenarios, the generalization ability of AI/ML model could be enhanced. However, the expanding dataset will cause great challenges to AI/ML model training. Considering the trained AI/ML model should be verified in system level simulation, the dataset construction for performance evaluation could be per scenario base.

- Issue 2: Performance impact by mixed training dataset

As observed by some preliminary results, compared to the scenario specific AI/ML model where the dataset for training is subject to a single scenario, some degradations are observed when the dataset for training is subject to mixed scenarios.

  - Samsung: Compared to the AE trained with only outdoor dataset, the CS performance of AE trained with mixed dataset is degraded by around 6%.

  - InterDigital: The Mixed multi-channel dataset exhibits a small performance degradation compared to Single-channel dataset (CDLA) when tested in the CDL-A channel (possibly due the generalization loss). However, mixed multi-channel dataset outperforms Single-channel dataset for inference under a CDL-A channel of a different delay spread, and outperforms Single-channel dataset for inference under a CDL-B/C channel.

On the other hand, as observed by some other companies, a mixed model can achieve good performance for each scenarios of inference, as long as the size for each specific scenario in the dataset is large enough.

  - vivo: The models trained by dataset constructed with mixed InH-based and UMi-based data behave well for both scenarios, even not as good as with the dataset from one entire scenario. The increasing number of correct samples in a mixed dataset can improve the performance and the wrong samples do not influence the performance.

  - OPPO: The SGCS performance of AI model trained on the mixed channel dataset with sufficient samples is comparable to that trained on the single channel dataset.

1st round email discussions

### 2.2-1: Generic evaluation methodology

Simulation approach

Aligning the simulation approach (system level, link level, etc.) is crucial for calibrating the results over companies and provide consistent evaluation results as the outcome of the SI. As summarized from the contributions, 18 companies have adopted/recommended the system level simulation approach, while 8 companies adopted/recommended the link level simulation approach.

Option 1: System level simulation is adopted/can be considered (18 companies) Option 2: Link level simulation is adopted/can be considered (8 companies) Option 3: Other approaches (1 company)

From the Moderator’s understanding, using system level simulation approach is beneficial to provide comprehensive results of, e.g., throughput, etc., in a multi-cell perspective, which is more close to realistic network environment. But let’s see if we can align the simulation approach in the 1st round.

Question 2.2.1: For the evaluation of the AI/ML based CSI feedback enhancement, which of the following simulation approach should be adopted:

- Option 1: System level simulation

- Option 2: Link level simulation

- Option 3: Others

Please provide you input to the following two tables, 1st table your company name and 2nd table for additional comments. Please provide the detailed methodology if you prefer Option 3.

| Option 1 | Supporting companies | Xiaomi, vivo Huawei/Hisi, CAICT, MediaTek, Ericsson, CMCC, Nokia/NSB, NVIDIA, InterDigital, Fraunhofer, Charter, Futurewei, Lenovo, LG, Panasonic, CATT, ZTE Samsung Fujitsu Qualcomm Apple OPPO, Intel, Beijing Jiaotong University, AT&T |
| --- | --- | --- |
|  | Objecting companies |  |
| Option 2 | Supporting companies | Xiaomi, InterDigital, Fraunhofer, Charter, LG, Panasonic Samsung Fujitsu OPPO, Intel, Beijing Jiaotong University |
|  | Objecting companies |  |
| Option 3 | Supporting companies |  |
|  | Objecting companies |  |

| Company | View |
| --- | --- |
| Xiaomi | Both SLS and LLS for throughput performance comparison are okay. |
| vivo | The channel generated by LLS, in general, behaves less characteristics, such as Tx/Rx antenna field patterns, Tx/Rx Ant location vector, and long-term fading. Compared to the system performance simulated by SLS, LLS simulating for the AI-based CSI feedback enhancement usually appears a better performance. Therefore, SLS should be adopted as a mandatory approach. |
| Huawei/Hisi | System level simulation has been widely applied in NR MIMO enhancements of legacy releases, so it will be easier to find a benchmark and calibrate the performance, and the legacy EVM table can be easily borrowed as a starting point. |
| CAICT | As pointed out by moderator, SLS is beneficial to provide comprehensive results of throughput, which is more close to realistic network environment. |
| MediaTek | We also think SLS is more than enough to evaluate the performance of AI/ML based CSI feedback. |
| Ericsson | LLS is optional and can be used for intermediate studies. SLS using FTP traffic model must be used to draw conclusions in the SI. |
| CMCC | SLS should be used to evaluate the performance of AI/ML approach. For SLS, we can use system throughput, SE or UPT as the eventual KPI. LLS can be adopted as an optional approach. |
| Nokia/NSB | SLS have been used consistently in the past for CSI feedback evaluations, hence we can borrow the EVM assumptions from Rel-16 eType-II CSI and compare performance to a legacy CSI feedback mechanism |
| NVIDIA | - Option 1: System level simulation should be the baseline. - Option 2: Link level simulation can complement system level simulation and may provide more insights. - Option 3: This is a study item – the first time for 3GPP to conduct comprehensive evaluation on AI/ML for air interface. It may be beneficial to be open minded to also explore new evaluation methodology to perform more physically accurate simulation, e.g., a virtual world simulation that can take both data and the governing physics into account to train a neural network that creates an AI surrogate model for digital twins. |
| InterDigital | We don’t see any reason to preclude any one of SLS and LLS for evaluation. To align the results across companies, we are ok to use SLS and make LLS as optional. |
| Fraunhofer | Both SLS and LLS are suitable for performance measurement and comparison. |
| Charter | Both SLS and LLS can help to identify advantages and disadvantages of ML techniques. LLS can be used to quantify/report the initial gain in proposed sub use cases and SLS can be used for more comprehensive evaluation of agreed sub use cases. |
| Lenovo | Final performance benefits evaluations in the SI should be based on SLS. LLS can be used for intermediate relative performance comparison between models. |
| LG | It may depend on the sub-use cases and metric. If we want to see performance benefit of MU performance, SLS should be selected. Therefore, for codebook compression, SLS seems suitable as usually did in NR MIMO. For CSI prediction, if we want to see the predication performance only via GCS or NMSE, LLS seems sufficient. |
| CATT | Achieving obvious performance gain with acceptable complexity increasing of devices is a basic precondition of specifying a solution in 3GPP. Therefore for AI/ML based CSI feedback, at least system level simulation for selected sub use cases is needed. However, system level simulation is time consuming, and the performance of the simulation results would be affected by lots of factors besides AI/ML model(s). Therefore it is not a good way to evaluate large amount of AI models through system level evaluation directly. We suggest to consider evaluations with two phases: - Phase 1: Intermediate evaluation for preliminary AI model selection; - Phase 2: Final evaluation for system level performance verification. For phase 1, the AI model is not actually deployed in the system, and the performance of AI model is evaluated outside the system. For phase 2, the AI model is deployed in the system, and system level performance would be provided. |
| ZTE | We agree with the system level simulation evaluation, which is more close to realistic network environment, and SLS in Rel-16 and Rel-17 has been aligned and calibrated among companies. |
| Samsung | We want to clarify whether this is for evaluation on selected representative sub-use cases. We believe that link-level simulations can be considered for Phase I of the evaluation process. In particular, LLS can be used to generate dataset for AI/ML model training/validation and testing. System-level simulations can be considered for Phase II of the evaluation process. |
| Fujitsu | We think both LLS and SLS are necessary to assess the performance gain of AI method over non-AI method. |
| Qualcomm | We are open to LLS but prefer SLS since the channel models cover a broader range of scenarios. Also, past MIMO CSI studies used SLS, and a similar methodology can be used. Standalone evaluation of the ML model may also be useful to understand ML performance in isolation. |
| OPPO | The final conclusion should be obtained based on the evaluation on Option 1. The Option 2 can be considered as complementary for intermediate KPI calibration. |
| Intel | In general both SLS and LLS are reasonable and should be allowed, especially because AI-ML is a new study in RAN1 and we should be more open and curious – we don’t intend to prioritize one over the other without looking at results. |
| Beijing Jiaotong University | Both SLS and LLS should be considered for performance measurement and comparison. |

Baseline SLS EVM table-overall

If Option 1 in Question 2.2.1 is considered as the simulation approach, then the follow-up issue is what baseline EVM assumptions we should adopt at least for generic scenarios/parameters not couple with AI/ML. As summarized from the contributions, 4 companies recommend to adopt the R17 CSI enhancement EVM as a baseline, while 6 companies recommend to adopt the R16 CSI enhancement EVM as a baseline.

Option 1: Take EVM of R17 CSI enhancement as a baseline [27] (4 companies) Option 2: Take EVM of R16 CSI enhancement as a baseline [28] (6 companies)

The R16/R17 EVM parameters are pasted in the following table as a reference, where the highlighted entries are different parameters between R16 and R17:

Table 1 EVM for generic parameters

| Parameter |  | R16 Value | R17 Value |
| --- | --- | --- | --- |
| Duplex, Waveform |  | FDD (TDD is not precluded), OFDM |  |
| Multiple access |  | OFDMA |  |
| Scenario |  | Dense Urban (Macro only) is a baseline. Other scenarios (e.g. UMi@4GHz 2GHz, Urban Macro) are not precluded. |  |
| Frequency Range |  | FR1 only, 4GHz. | FR1 only, 2GHz with duplexing gap of 200MHz between DL and UL, optional for 4GHz |
| Inter-BS distance |  | 200m |  |
| Channel model |  | According to TR 38.901 | N/A |
| Antenna setup and port layouts at gNB |  | Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ - 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ Other configurations are not precluded. |  |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for rank > 2 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) Other configuration is not precluded. |  |
| BS Tx power |  | 41 dBm for 10MHz, 44dBm for 20MHz, 47dBm for 40MHz |  |
| BS antenna height |  | 25m |  |
| UE antenna height & gain |  | Follow TR36.873 |  |
| UE receiver noise figure |  | 9dB |  |
| Modulation |  | Up to 256QAM |  |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |  |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |  |
|  | SCS | 15kHz |  |
| Number of RBs |  | 52 for 15 kHz SCS | N/A |
| Simulation bandwidth |  | 10 MHz for 15kHz as a baseline, and configurations which emulate larger BW, e.g., same sub-band size as 40/100 MHz with 30kHz, may be optionally considered. | 20 MHz for 15kHz as a baseline (optional for 10 MHz with 15KHz), and configurations which emulate larger BW, e.g., same sub-band size as 40/100 MHz with 30kHz, may be optionally considered |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |  |
| MIMO scheme |  | SU/MU-MIMO with rank adaptation is a baseline for overhead reduction. For low RU, SU-MIMO or SU/MU-MIMO with rank adaptation are assumed for higher rank extension. For medium/high RU, SU/MU-MIMO with rank adaptation is assumed` for higher rank extension. | R17: For low RU, SU-MIMO with rank adaptation are assumed For medium/high RU, SU/MU-MIMO with rank adaptation is assumed |
| MIMO layers |  | For all evaluation, companies to provide the assumption on the maximum MU layers (e.g. 8 or 12) |  |
| CSI feedback |  | Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback) : 5 ms, - Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms |  |
| Overhead |  | Companies shall provide the downlink overhead assumption |  |
| Traffic model |  | FTP model 1 with packet size 0.5 Mbytes Other FTP model is not precluded. |  |
| Traffic load (Resource utilization) |  | - 50/70 % for CSI overhead reduction - 20/50 % for high rank extension Companies are encouraged to report the MU-MIMO utilization. | - 70% for SU/MU-MIMO with rank adaptation - 20% for SU-MIMO with rank adaptation Companies are encouraged to report the MU-MIMO utilization. |
| UE distribution |  | - 80% indoor (3km/h), 20% outdoor (30km/h) |  |
| UE receiver |  | MMSE-IRC as the baseline receiver |  |
| Feedback assumption |  | Realistic |  |
| Channel estimation |  | Realistic |  |
| Evaluation Metric |  | Throughput and CSI feedback overhead as baseline metrics. Additional metrics, e.g., ratio between throughput and CSI feedback overhead, can be used. Maximum overhead (payload size for CSI feedback)for each rank at one feedback instance is the baseline metric for CSI feedback overhead, and companies can provide other metrics. |  |
| Baseline for performance evaluation |  | Rel-15 Type II Codebook is the baseline for performance and overhead evaluation for overhead reduction. (Type I Codebook can be considered at least for performance evaluation) - Companies are encouraged to compare the proposed overhead reduction scheme with Rel-15 overhead reduction scheme, Rel-15 Type I Codebook is the baseline for performance and overhead evaluation for higher rank codebook. | Rel-16 PS eTypeII Codebook is the baseline for performance and overhead evaluation. (Type I Codebook can be considered at least for performance evaluation) - Note that it is encouraged to disclose further details of beamforming mechanism/ordering over CSI-RS ports/resources. |

Some specific entries can be marked as FFS due to two reasons:

- The following entries are different with parameter values between R16 EVM and R17 EVM, so they can be marked as FFS, and separately discussed in later questions/proposals.

- ‘Frequency Range’

- ‘Channel model’

- ‘Number of RBs’

- ‘Simulation bandwidth’

- ‘MIMO scheme’

- ‘Traffic load (Resource utilization)’

- ‘Baseline for performance evaluation’

- For the following two entries, a number of companies raise different assumptions with R16/17 EVM, so the R16/17 EVM can be taken as baseline, and whether a different value is needed will be separately discussed in later questions/proposals.

- ‘Traffic model’: Full buffer is assumed in preliminary assumptions/as a starting point (4 companies)

- ‘Channel estimation’: Ideal downlink channel estimation (4 companies)

Based on the above thinking, the following question is raised, and let’s see the views from companies.

Question 2.2.2: For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, do you agree the following table is taken as a baseline of EVM?

- Note: the following table captures the common parts of the R16 CSI enhancement EVM table and the R17 CSI enhancement EVM table, while the different parts are FFS.

- Note: the baseline EVM is used to compare the performance with the benchmark release, while the AI/ML related parameters (e.g., dataset construction, generalization verification, and AI/ML related metrics) can be of additional/different assumptions.

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD (TDD is not precluded), OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) is a baseline. Other scenarios (e.g. UMi@4GHz 2GHz, Urban Macro) are not precluded. |
| Frequency Range |  | FR1 only, FFS 2GHz or 4GHz as a baseline |
| Inter-BS distance |  | 200m |
| Channel model |  | FFS |
| Antenna setup and port layouts at gNB |  | Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ - 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ Other configurations are not precluded. |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for rank > 2 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) Other configuration is not precluded. |
| BS Tx power |  | 41 dBm for 10MHz, 44dBm for 20MHz, 47dBm for 40MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz |
| Number of RBs |  | FFS |
| Simulation bandwidth |  | FFS |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | FFS |
| MIMO layers |  | For all evaluation, companies to provide the assumption on the maximum MU layers (e.g. 8 or 12) |
| CSI feedback |  | Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback) : 5 ms, - Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms |
| Overhead |  | Companies shall provide the downlink overhead assumption |
| Traffic model |  | FTP model 1 with packet size 0.5 Mbytes as a baseline Other FTP model is not precluded. FFS full buffer |
| Traffic load (Resource utilization) |  | FFS |
| UE distribution |  | - 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic as a baseline FFS ideal channel estimation |
| Evaluation Metric |  | Throughput and CSI feedback overhead as baseline metrics. Additional metrics, e.g., ratio between throughput and CSI feedback overhead, can be used. Maximum overhead (payload size for CSI feedback)for each rank at one feedback instance is the baseline metric for CSI feedback overhead, and companies can provide other metrics. |
| Baseline for performance evaluation |  | FFS |

Please provide you input to the following two tables, 1st table your company name and 2nd table for additional comments.

| Supporting companies | Xiaomi, vivo Huawei/Hisi, CAICT, Ericsson, CMCC, Nokia/NSB, NVIDIA, InterDigital, Fraunhofer, Charter, Futurewei, Panasonic, CATT, ZTE Samsung Fujitsu Qualcomm (some changes below) OPPO, Beijing Jiaotong University |
| --- | --- |
| Objecting companies | Lenovo |

| Company | View |
| --- | --- |
| Xiaomi | We prefer R16 EVM parameters, but the current proposal is acceptable for progress. |
| vivo | In terms of the traffic model, we believe that a full buffer traffic should be considered as a baseline. To simplify the simulation procedure, an ideal channel estimation should be taken as a baseline. FFS for realistic channel estimation with some additional definition of estimation error. |
| Huawei/Hisi | Basically in our understanding, R17 EVM can be a starting point, and AI/ML specific parameters, such as AI/ML specific metrics, more/mixed scenarios/configurations for generalization, etc., can be added on top of that. |
| CAICT | We support moderator’s proposal. |
| Ericsson | Note that the Rel.17 EVM was modified from the Rel.16 EVM to better suit the special case of Rel.17 FDD reciprocity studies with gNB pre-compensation. Hence, the Rel.16 EVM is the de facto baseline for CSI evaluations in RAN1 MIMO. |
| CMCC | Regarding to traffic model, we think full buffer can be considered as a baseline if we take the throughput or spectral efficiency as the eventual KPI. For channel estimation, we think ideal estimation can make evaluation work simpler. Ideal channel estimation should be taken as a baseline. |
| Nokia/NSB | We think Rel-16 EVM should be taken as reference as the baseline for comparison should be Rel-16 eType-II CSI. Rel-17 specified a port selection version of Type-II suitable for partial reciprocity in FDD operations, so it is not suitable as baseline because it assumes that CSI-RS are precoded in both spatial and frequency domain. This means that comparing to Rel-17 FeType-II PS may require generating datasets that include this CSI-RS precoding and are UE specific. |
| NVIDIA | We support moderator’s proposal. |
| Fraunhofer | Realistic channel estimation should be assumed at the UEs. Full buffer traffic can be considered for simplicity for the traffic model. |
| Lenovo | As we highlighted in R1-2204417, AI/ML models may learn a particular scenario really well while have inferior performance in environment with slight change in parameter assumptions. Also the AI/ML model might need a very specific input sizes that limits the scalability of the model. So, it is very important that the baseline EVM include scenarios and cases that can evaluate the generalization and scalability of the proposed model. Thus, we propose to consider and define a set of multiple scenarios and configurations that should be used for evaluation of the proposed scheme. For example we can consider baseline for the evaluation: 1. Two Scenarios - UMa and UMi 2. UE 4RX and 2RX antenna setup (for 4GHz can only consider 4RX) 3. gNB antenna setup 32 ports and 16 ports Two simulation bandwidths (10MHz and 20MHz) and 2 subcarrier spacings (15kHz and 30kHz) also should be considered. The results can be reported in different cases: - One model is trained based on a training dataset and evaluated against all above scenario - One model is trained based on a training dataset, then fine-tuned for each of the above scenarios, and then evaluated against that scenario. The overhead and complexity of the fine tuning should be reported in this case - A separate model is trained for each of the scenarios and evaluated again that particular scenario. The complexity of having separate model for each case should be discussed in this case |
| LG | Ok for the proposal |
| CATT | Generally fine with the table as baseline. But SCS should depend on the target frequency band, e.g., 30kHz for 4GHz, 15kHz for 2GHz. For evaluation of CSI, trade-off between throughput and overhead is used as the metric in Rel-16 and Rel-17 evaluation. Therefore, it would be better to use the same metric. |
| ZTE | We think both Rel-16 and Rel-17 SLS assumptions can be reused as two baselines for different sub use cases. For example, for AI/ML-based CSI compression in spatial&frequency domain, Rel-16 SLS assumptions can be reused for performance evaluation. When considering enhancements on UL-DL reciprocity, we can apply Rel-17 SLS assumptions for simulation. FFS: Any modifications to existing Rel-16/Rel-17 SLS assumptions can be discussed and determined. |
| Samsung | If this is for final-stage evaluation on selected representative sub-use cases, we agree on starting with Rel-16 and Rel-17 EVMs. Furthermore, we want to mark these as FFS: UE distribution: Scenario: FFS baseline Mobility Modeling: FFS Here, baseline scenario should be FFS, before the agreement for methodology for generalization is endorsed. |
| Qualcomm | Regarding feedback assumption being realistic, it would be useful to clarify what this means. Ideal UCI feedback can be the starting point (i.e., CSI is delivered without errors). Regarding downlink overhead, companies should clarify whether the CSI-RS transmission is UE-specific or not and take that into account for overhead computation. Regarding SCS, we prefer to add 30 KHz as an option. |
| OPPO | For the baseline of EVM table, we agree with the most of assumptions in the table above. For the FFS item, we propose to use the following assumptions as follows: Parameter; Value. Frequency Range; FR1 only, 4GHz as a baseline. Channel model; According to TR 38.901. Antenna setup and port layouts at UE; 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) Other configuration is not precluded.. Number of RBs; 52RB, 13 sub-bands. Simulation bandwidth; 10MHz with 15kHz SCS. MIMO scheme; For low RU, SU-MIMO with rank adaptation are assumed For medium/high RU, SU/MU-MIMO with rank adaptation is assumed. Traffic model; FTP model 1 with packet size 0.5 Mbytes as a baseline Full buffer should also be considered. Other FTP model is not precluded.. Traffic load (Resource utilization); - 70% for MU-MIMO with rank adaptation - 20% for SU-MIMO with rank adaptation. Channel estimation; Both realistic and ideal can be used as a baseline Companies should provide their assumptions. Baseline for performance evaluation; Rel-16 PS eTypeII Codebook is the baseline for performance and overhead evaluation. (Type I Codebook can be considered at least for performance evaluation). |
| Intel | In general we prefer R16 EVM due to reason from Ericsson. We think full-buffer should be baseline – bursty traffic results differ much more across companies. We can use bursty after the simulation results mature. Ideal channel estimation can be baseline – if not then good to discuss what non-ideal model is used. The important thing here is to try to reduce some variability of modeling. |
| Beijing Jiaotong University | We support moderator’s proposal. |
| AT&T | Rel.16 EVM should be considered as baseline. Full buffer traffic should also be baseline. |

As mentioned in Question 2.2.2, the following parameters may be separately discussed.

As per Moderator’s understanding, for ‘Channel model’, and ‘Number of RBs’ which are missed by R17 EVM, reusing R16 EVM may be a simple way, with the change of adding 106 PRBs for 20MHz case (which may be necessary since 20MHz bandwidth is typically configured for commercial NR FDD networks).

For ‘Frequency Range’, ‘Simulation bandwidth’, ‘MIMO scheme’, and ‘Traffic load (Resource utilization)’, reusing R17 EVM which is state-of-art may be a reasonable way.

Let’s hear the views of companies.

Question 2.2.3: For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, do you agree the following parameters are taken into the baseline of EVM?

- Note: the baseline EVM is used to compare the performance with the benchmark release, while the AI/ML related parameters (e.g., dataset construction, generalization verification, and AI/ML related metrics) can be of additional/different assumptions.

- Note: the entries of the table in below are changed on top of the corresponding entries of the EVM table in Question 2.2.2.

| Parameter | Value |
| --- | --- |
| Frequency Range | FR1 only, FFS 2GHz or 4GHz as a baseline 2GHz with duplexing gap of 200MHz between DL and UL, optional for 4GHz |
| Channel model | FFS According to TR 38.901 |
| Number of RBs | FFS 52 for 10MHz bandwidth and 15 kHz SCS, 106 for 20 MHz bandwidth and 15 kHz SCS |
| Simulation bandwidth | FFS 20 MHz for 15kHz as a baseline (optional for 10 MHz with 15KHz), and configurations which emulate larger BW, e.g., same sub-band size as 40/100 MHz with 30kHz, may be optionally considered |
| MIMO scheme | FFS For low RU, SU-MIMO with rank adaptation are assumed For medium/high RU, SU/MU-MIMO with rank adaptation is assumed |
| Traffic load (Resource utilization) | FFS - 70% for SU/MU-MIMO with rank adaptation - 20% for SU-MIMO with rank adaptation Companies are encouraged to report the MU-MIMO utilization. |

Please provide you input to the following two tables, 1st table your company name and 2nd table for additional comments.

| Supporting companies | vivo Huawei/Hisi, CAICT, NVIDIA, Fraunhofer, Charter, Futurewei, Lenovo, Panasonic, CATT, ZTE Qualcomm (one comment below) OPPO Beijing Jiaotong University |
| --- | --- |
| Objecting companies | Ericsson (partly, see below), Nokia/NSB (in part, details below) |

| Company | View |
| --- | --- |
| vivo | In the assumption of the frequency range, we believe that 200MHz frequency gap is unnecessary because the majority channel scenarios in the simulation does not behave the channel reciprocity. According to our evaluation results, moreover, the generalized AI-model deployed for the different frequency ranges can offer the similar performance (e.g., spectral efficiency) as long as the frequency range belongs to FR1. |
| Huawei/Hisi | R17 EVM can be a starting point |
| CAICT | Support moderator’s proposal. |
| MediaTek | We think R17 EVM has already taken FDD reciprocity and port selection codebook into account. We would suggest to first determine the benchmark CSI feedback scheme. To compare with R16 eType II, R16 EVM can be used as baseline. To compare with R17 FeType II PS, R17 EVM can be as baseline. If needed, some changes can be applied. |
| Ericsson | R16 EVM should be the starting point, as R17 EVM was specifically tailored for a special use case (FDD based reciprocity) OK with channel model. For frequency range, no need to state duplexing gap. Not sure we need to agree on number of RBs, it is implicitly given by BW and SCS. For 4 GHz, 30 kHz SCS should be used. For MIMO scheme and traffic load, there is no reason to exclude MU-MIMO at low RU, it is a scheduler issue. It’s better to say rank adaption and SU/MU based on scheduler decision for any RU. Traffic load points should be 20,50 and 70%. |
| Nokia/NSB | Rel16 EVM should be the starting point. We do not think R17 Type-II is relevant as benchmark for this study because it is a port selection codebook that assumes a more complicated setup for SLS, with SRS estimation, gNB calculation of dominant angles and delays, UE-specific precoding of CSI-RS in both spatial and frequency domain. Frequency Range: assumption on duplexing gap is not needed as partial reciprocity scenario is too complicated for an initial ML study Simulation bandwidth: we suggest reusing Rel-16 eType-II assumptions for direct comparison, copied below: “10 MHz for 15kHz as a baseline, and configurations which emulate larger BW, e.g., same sub-band size as 40/100 MHz with 30kHz, may be optionally considered.” |
| NVIDIA | The assumption of 200 MHz duplexing gap does not appear necessary. |
| Fraunhofer | Rel 16 EVM may be a good starting point as well. |
| Lenovo | Generally, we agree with the assumptions. Noting that we should have multiple scenarios and parameter configurations for the evaluations as mentioned for Q2.2.2. Two simulation bandwidths (10MHz and 20MHz) and 2 subcarrier spacings (15kHz and 30kHz) should be considered. |
| LG | We thinks differentiation of SU/MU-MIMO dynamic switching according to RU is not necessary. Simply, SU/MU-MIMO dynamic switching can be a baseline. |
| CATT | Regarding system bandwidth, we prefer to have 10MHz with 15kHz SCS as baseline. If 20MHz bandwidth is to be evaluated, e.g., for 4GHz, we would use 30kHz SCS. |
| ZTE | We think both Rel-16 and Rel-17 SLS assumptions can be reused as two baselines for different sub use cases. Different simulation parameters e.g. carrier frequency and bandwidth, are available for us to perform simulations for different sub use cases. FFS: Any modifications to existing Rel-16/Rel-17 SLS assumptions can be discussed and determined. |
| Samsung | We want to add these as FFS: To evaluate the performance of aspects related to temporal-domain compression, we want to add a more diverse UE distribution. The values UE distribution: - X% indoor (3km/h), Y1% outdoor (10km/h), Y2% outdoor (20km/h), Y3% outdoor (30km/h), FFS: X, Y1,Y2,Y3 Mobility Modeling: FFS |
| Qualcomm | Regarding SCS, we prefer to add 30 KHz as an option |
| OPPO | We support the ‘Frequency Range’, ‘Channel Model’, ‘MIMO schemes’, ‘Traffic load’ with parameters above should be taken into the baseline of EVM. As for the ‘Simulation bandwidth’ and ‘Number of RBs’, we suggest to use 10MHz with 15KHz SCS, and 52 RBs as the baseline. |
| Intel | Prefer R16 as baseline – no need for duplexing gap, full-buffer can be used, no need for changing scheduler based on RU |
| Beijing Jiaotong University | Support moderator’s proposal. |
| AT&T | We agree with Nokia, Ericsson and Intel that Rel. 16 should be the baseline. Full buffer should be used, and no need to specify the duplexing gap, or changing scheduler based on RU. |

Baseline SLS EVM table-channel estimation

4 companies raise to consider ideal downlink channel estimation for ‘Channel estimation’. To the understanding of Moderator, it may be optionally added to this entry if needed. Let’s see if it is acceptable for companies.

Question 2.2.4: For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, for ‘Channel estimation’, whether ideal DL channel estimation is optionally taken into the baseline of EVM?

| Supporting companies | Xiaomi, vivo Huawei/Hisi, CAICT, MediaTek, CMCC, Nokia/NSB (not for SLS results), NVIDIA, Charter, Futurewei, Lenovo, CATT, ZTE Samsung Qualcomm OPPO, Beijing Jiaotong University |
| --- | --- |
| Objecting companies | Ericsson, InterDigital, LG |

| Company | View |
| --- | --- |
| Xiaomi | DL channel can be estimated through CSI-RS and DMRS. It needs to clarify whether ideal DL channel estimation is applied to both CSI-RS and DMRR or not. For calibrating simulation results, it is fine that ideal DL channel estimation is regarded as baseline. |
| vivo | As we mentioned previously, an ideal channel estimation should be taken as a baseline. This is because our goal in the simulation evaluation is to find out the performance gain by state-of-the-art AI-based technologies as long as the evaluation assumptions are kept being the same. Moreover, involving the channel estimation error in the simultion could make the simulation works more complicated. |
| Huawei/Hisi | Ideal channel estimation can be optionally considered for preliminary results for simplicity. In addition, for dataset generation, it can be considered if we assume the simulation based dataset can also be applied to the realistic network. |
| CAICT | Ideal channel estimation could simplify the simulation process and not affect the relative performance for CSI compression. |
| MediaTek | To remove the other impacts, we think ideal channel estimation can be used at least for performance evaluation. |
| Ericsson | We don’t see the value of performing simulation with idealities other than for calibration purpose. RAN1 has made SLS with realistic modeling of non-idealities for many years now. To make a conclusion in the SI, we need to be as realistic as possible. One goal of the SI is to see whether AI can enhance over existing releases. |
| CMCC | The motivation of the agenda is to evaluation the permeance gain of AI based method over traditional methods, we think taking ideal channel estimation as a baseline is more straightforward. Channel estimation error might make the gain analysis more complicated. |
| Nokia/NSB | This can be included, for example, for dataset generation and/or verification, butnot for generating SLS results |
| NVIDIA | There may be two interpretations of “ideal channel estimation”: - Interpretation 1: Ideal channel estimation for dataset construction - Interpretation 2: Ideal channel estimation for system performance evaluation (such as throughput). It’s worth first clarifying if the proponents target Interpretation 1 or Interpretation 2 or both. |
| InterDigital | Tend to agree with Ericsson. Other than calibration, realistic modeling should be considered to conclude the SI especially for CSI enhancement aspect. It is often observed that a sophisticated enhancement scheme doesn’t show the gain in the realistic scenarios and sensitive to impairments. |
| Charter | Both ideal and practical channel estimation should be evaluated for better understanding of the ML algorithms behavior. |
| Lenovo | Ideal channel estimation can be considered for intermediate results to simplify the simulation assumptions. |
| LG | We agree with Ericsson and InterDigital, ideal assumption only applied for calibration purpose. |
| CATT | Channel estimation error shall be properly modeled to access the performance gain of AI/ML algorithm. But we are ok if ideal channel estimation is adopted optionally. |
| ZTE | For ease of AI/ML GCS performance calibration, ideal DL channel estimation can be initially adopted. However, non-ideal DL channel estimation with error modeling should be further considered in the final system simulation. |
| Samsung | Ideal channel can be considered for intermediate results. |
| Qualcomm | Agree with using ideal DL channel estimation as an option; it would be useful to clarify whether this applies to both CSI-RS and DMRS. |
| OPPO | We support that ideal channel estimation can be optionally taken into the baseline of EVM. |
| Intel | Ideal is okay – for non-ideal good to discuss a model. |
| Beijing Jiaotong University | The channel capacity with imperfect CSI can be measured with the known feedback CSI and CSI error. |

Baseline SLS EVM table-Traffic model

4 companies raise to consider full buffer for ‘Traffic model’. To the understanding of Moderator, it may be optionally added to this entry if needed. Let’s see if it is acceptable for companies.

Question 2.2.5: For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, for ‘Traffic model’, whether full buffer traffic is optionally taken into the baseline of EVM?

| Supporting companies | Support, vivo Huawei/Hisi, CAICT, MediaTek, CMCC, NVIDIA, Fraunhofer, Charter, Futurewei, Lenovo, CATT Samsung Qualcomm OPPO, Intel, Beijing Jiaotong University, AT&T |
| --- | --- |
| Objecting companies | Ericsson, InterDigital, LG |

| Company | View |
| --- | --- |
| Xiaomi | Support to use full buffer as the optional baseline traffic model. |
| vivo | Full buffer traffic can be considered as a baseline and FTP1 can be optional. |
| Huawei/Hisi | It can be considered at the starting phase of SI for simplicity. |
| CAICT | Full buffer could be used as baseline. |
| MediaTek | Full buffer traffic can be used as a baseline. |
| Ericsson | Full buffer was abandoned several years ago in MIMO evaluations in RAN1, we don’t see why it needs to come back, it is unrealistic and results are meaningless except for calibration purpose. Full buffer cannot be used to make a prediction of the usefulness of a feature and hence doesn’t provide any insights that assist in drawing SI conclusions. We object to using full buffer to make any conclusions in this SI. |
| CMCC | Full buffer can be taken as the baseline. |
| Nokia/NSB | Ok to consider only as optional. We don’t see a reason to change the assumption from Rel-16: “FTP model 1 with packet size 0.5 Mbytes. Other FTP model is not precluded.” |
| NVIDIA | Full buffer can be an optional baseline. |
| InterDigital | Similar to the ideal channel estimation, full buffer is unrealistic and it shouldn’t be used to make any conclusion. It is often observed that a scheme provides significant gain in full buffer as optimal MU-pairing is always available at the scheduler but the gain disappears when non-full buffer is used. |
| Fraunhofer | The full buffer traffic load for the starting point is supported. |
| Charter | Full buffer can be optionally considered and evaluated. |
| Lenovo | Full buffer can be optional for intermediate studies. |
| LG | FTP model 1 is preferred as it reflect more realistic environment. |
| CATT | Full buffer results can be optionally provided for calibration purpose. |
| ZTE | We think we can continue to adopt SLS assumptions in Rel-16/Rel-17, e.g. FTP model 1 with packet size 0.5 Mbytes, which is closer to real environment. Additionally, full-buffer can be further discussed for initial AI/ML model calibration. |
| Samsung | Ok to adopt full buffer traffic as optional traffic model. |
| Qualcomm | Agree with using full-buffer traffic as an option. |
| OPPO | We agree that the full buffer can be optionally taken into the baseline of EVM. |
| Intel | Agree to use full-buffer traffic |
| Beijing Jiaotong University | Full buffer could be used as baseline. |
| AT&T | Full buffer should be considered. It is anyway not precluded in Rel. 16 EVM, and is useful for evaluating the performance of some of the sub-use cases in CSI enhancements |

Baseline SLS EVM table -Baseline for performance evaluation

As per the understanding of Moderator, it is a general way to select the state-of-art release as the baseline to study/evaluate the performance of a new feature. But as a majority of companies initially adopt R16 Type II CB as the baseline in the contributions, Moderator lists the two options for down selection. Please provide your views on your preferred benchmark scheme.

Question 2.2.6: For the evaluation of the AI/ML based CSI feedback enhancement, which of the following benchmark CSI feedback scheme should be adopted as the baseline for performance evaluation

- Option 1: R16 Type II CB

- Option 2: R17 Type II CB

- Option 3: Other

| Option 1 | Supporting companies | Xiaomi, vivo Huawei/Hisi, CAICT, MediaTek, Ericsson, CMCC, Nokia/NSB, NVIDIA, InterDigital(with clarification), Fraunhofer, Charter, Futurewei, lenovo, LG, Panasonic, CATT, ZTE Samsung Fujitsu Qualcomm OPPO Intel, Beijing Jiaotong University, AT&T |
| --- | --- | --- |
|  | Objecting companies |  |
| Option 2 | Supporting companies | Huawei/Hisi, CAICT, CATT OPPO (2nd step after Option 1), Beijing Jiaotong University |
|  | Objecting companies |  |
| Option 3 | Supporting companies |  |
|  | Objecting companies |  |

| Company | View |
| --- | --- |
| Xiaomi | Option2 needs additional SRS transmission to obtain uplink channel, whereas Option 1 does not. CSI feedback based on AI/ML does not require SRS transmission as well. Hence, Option 1 is preferred. |
| vivo | R16 Type II CB should be considered as a baseline, while R17 Type II CB could be optional only when the channel reciprocity is needed. |
| Huawei/Hisi | For the baseline of performance evaluation, both Rel-16 Type II codebook and Rel-17 Type II codebook should be considered as the state-of-the-art non-AI CSI feedback scheme for performance comparison. |
| CAICT | We are fine with both option 1 and 2. |
| MediaTek | Agree with Xiaomi. We also prefer option 1. |
| Ericsson | Rel.17 Type-II is a special solution for FDD reciprocity that relies on extensive gNB pre-compensation, the use of both SRS and CSI-RS. The performance of the R17 scheme is heavily implementation dependent and requires detailed knowledge and fine tuning. Hence, it is not a good baseline to compare against. It is better to use the “stable”and straightforward Rel.16 Type II CB (normal) (option 1). |
| CMCC | R16 Type-II CB should be the baseline. R17 Type-II CB is more about Port selection codebook when channel reciprocity exists. |
| Nokia/NSB | We do not think R17 Type-II is relevant as benchmark for this study because it is a port selection codebook that assumes a more complicated setup for SLS, with SRS estimation, gNB calculation of dominant angles and delays, UE-specific precoding of CSI-RS in both spatial and frequency domain. |
| NVIDIA | R16 Type II CB is a more proper baseline. |
| InterDigital | This baseline will be used to conclude SI that whether AIML based enhancement justifies standards impacts in the following releases? If yes, implementation based AIML based solution needs to be also considered as baseline? We understand that Type II codebook should be a baseline but it is not necessarily the baseline to justify the AIML based specification impact. BTW, between Rel-16 and Rel-17 Type II codebooks, agree with companies it should be Rel-16 Type II since Rel-17 Type II codebook is mainly for FDD reciprocity. |
| Fraunhofer | The AI-based approaches is enough to outperform option 1 in order to be a candidate for replacement. |
| Lenovo | Prefer using Rel-16 Type-II CB as the baseline for study. Rel-17 Type-II PS codebook as based on beamformed CSI-RS transmission with the beamforming based on network implementation, which would provide an additional complexity to comparing evaluation results by different companies. Additionally, Rel-17 Type-II PS codebook was intended to be used for systems with partial UL-DL channel reciprocity, i.e., small duplexing distance, which may be too restrictive for the study |
| LG | Prefer option 1. |
| CATT | R17 Type II CB can be considered as benchmark for the scenarios with DL/UL reciprocity of angle and delay, and R16 Type II CB can be used as benchmark for other scenarios. |
| ZTE | We think R16 Type II CB can initially be adopted as the baseline for performance evaluation and calibration among companies, and R17 Type II CB can be an option for some specific use cases. We think both R16 Type II CB and R17 Type II CB can be adopted as the baselines for performance evaluation and calibration among companies. For example, for AI/ML-based CSI compression in spatial&frequency domain, Rel-16 Type II CB can be used for performance comparison. When considering enhancements on UL-DL reciprocity, we can apply Rel-17 Type II CB as a baseline for evaluation. |
| Samsung | Option 1: Rel-16 Type II CB (regular) as first choice. FFS: Rel-17 Type II (PS) CB, if angle-delay reciprocity is exploited |
| Fujitsu | We think Rel-16 Type II codebook is sufficient for evaluation purpose of AI encoder/decoder. |
| Qualcomm | We believe it is sufficient to use R16 Type II CB as the benchmark. If R17 is used as a benchmark, companies should clarify whether the CSI-RS transmission is UE-specific or not and take that into account for overhead computation. |
| OPPO | We think that in the initial stage of this SID, Option 1 with Rel-16 Type II codebook should be adopted as the baseline. After Option 1 has been well evaluated, the Option 2 with R17 Type II codebook can also be considered in the second stage. Other options for baseline performance evaluation are not required. |
| Intel | R16 Type II CB |
| Beijing Jiaotong University | We are okay with both option 1 and 2. |
| AT&T | Rel. 16 type II CB should be sufficient |

Baseline LLS EVM

If Option 2 in Question 2.2.1 is considered as the simulation approach, then the follow-up issue is we’d better find a legacy EVM table to be taken as a reference, so that the parameters can be updated on top of that to save efforts/time. The proponents are welcome to provide some EVM reference, or directly provide the parameter list, if you think the LLS is necessary.

Question 2.2.7: For the evaluation of the AI/ML based CSI feedback enhancement, if LLS is preferred, what is the baseline EVM from your view?

- Note: the baseline EVM is used to compare the performance with the benchmark release, while the AI/ML related parameters (e.g., dataset construction, generalization verification, and AI/ML related metrics) can be of additional/different assumptions.

| Company | View |
| --- | --- |
| Xiaomi | The following parameters can be assumed as baseline. Parameter; Value. Carrier frequency; 3.5GHz. Bandwidth; 10MHz. Subcarrier spacing; 15KHz. RB number; 48. Subband number K; 12. Nt; 32. Nr; 4. Channel model; CDL-A300. UE speed; 3km/h. Delay spread; 300ns. AI content; Eigenvector. Channel estimation; Ideal. RI; 1. |
| vivo | The CDL-C channel with 300ns delay spread can be considered as baseline, in conjunction of the additional parameters as The number of transmit antenna; 32. The number of receive antenna; 2. Carrier frequency; 3GHz. Subcarrier spacing; 30kHz. PRB number; 52. speed; 30km/h. LLS parameters; CDL-C, delay spread=300ns. Length of one sample in time domain; 200 slot (100ms). The number of historical CSI inputs; 12. The slot indices of historical CSI inputs; [71, 81, 91, …, 181]-th slots (spacing is 10 slots). The number of future CSIs to be predicted; 1~5. The slot indices of the predicted CSI; [183, 185, 187, 189, 191]-th slots (i.e., [+1ms, +2ms, +3ms, +4ms, +5ms]). The type of CSI; Raw channel, Eigenvector (or singular vector). |
| Nokia/NSB | There is no need for a new set of EVM assumptions for LLS, as we can reuse existing SLS EVM assumptions from legacy Rel-16 regular Type-II codebook. |
| Fraunhofer | Parameter; Value. Carrier frequency; 3.5GHz. Bandwidth; 10MHz. Subcarrier spacing; 15KHz. RB number; 48. Subband number; 12. Transmit antenna; 32. Receive antennas; 4. UE speed; 3 or 30 km/h. Delay spread; 50 or 300ns. Channel estimation; realistic. AI input; Eigenvectors and raw channel. |
| Charter | We agree with similar tables as suggested by other companies; however, AI input does not need to be strictly specified; any preprocessing can be used depending on the AI model but should be reported by companies. |
| CATT | For the evaluation, intermediate evaluation and SLS (eventual evaluation) are sufficient, LLS simulation is not needed. |
| ZTE | We think whether LLS is appropriate for being a baseline needs discussing and determining at first. Then, we need further discussion on this issue. |
| Samsung | In addition, CDL extension with different distribution of channel parameters should be considered for the generalization verification |
| Fujitsu | Parameter; Value. RB Number; 24. Sub-Carrier Spacing; 30 KHz. SNR; 0 dB, 5 dB, 10 dB. Sub-Band Number; 12. Channel Model; According to TR 38.901. CDC-C model describes the diversity well, and hence is the first choice. CDC-A model can also be used.. |
| OPPO | We think that LLS can be used as a complementary dataset for calibration and AI model performance evaluation. We suggest to use CDL-A, CDL-C with delay spread 30ns,300ns and UE speed with 3km/h, 30km/h as the baseline LLS EVM. Other necessary parameters such as gNB/UE antenna configuration, frequency band, number of RBs can be same to the SLS EVM table. |
| Intel | Agree with Nokia, same baseline as SLS can be used |

### 2.2-2: Source to generate the dataset

By the summary of the contributions, 17 companies raise that synthetic dataset can be adopted, while 6 companies think dataset from field is also helpful, but so far by Moderator’s review, there is no clear suggestion on where/how to collect plentiful field dataset as the input to 3GPP evaluation. So let’s see the views from companies on the source to contribute the dataset in the CSI feedback evaluation.

Synthetic dataset for AI/ML training/validation/testing generated from TR38.901 (17 companies) Dataset from field is adopted/can be considered in the evaluation (6 companies)

Question 2.2.8: For the evaluation of the AI/ML based CSI feedback enhancement, do you agree that the dataset for AI/ML training/validation/testing is generated by TR 38.901 channel models as a starting point?

- FFS whether/how to generate the dataset from field test.

| Supporting companies | Xiaomi, vivo Huawei/Hisi, CAICT, MediaTek, Ericsson, CMCC, Nokia/NSB, NVIDIA, InterDigital, Fraunhofer, Charter, Futurewei, Lenovo, LG, Panasonic, CATT, ZTE Samsung Fujitsu Qualcomm Apple OPPO, Beijing Jiaotong University, AT&T |
| --- | --- |
| Objecting companies |  |

| Company | View |
| --- | --- |
| Xiaomi | A common data set generated by TR 38.901 is preferred. |
| vivo | Dataset generated from TR 38.901 can be a start point. FFS for field dataset in terms of construction of the synthetic field dataset. |
| Huawei/Hisi | The dataset from field test would be beneficial in the long run, but considering it would be time consuming to collect various and diverse training inputs from the realistic network, so it will be time efficient to start with synthetic dataset. |
| CAICT | Agree with the proposal. |
| MediaTek | As we already explained in our Tdoc (R1-2205101), there are some issues to collect the field data. At least for evaluation, synthetic data can be used. |
| CMCC | The dataset generated by TR 38.901 can be the starting point. |
| Nokia/NSB | Dataset can be generated by reusing the EVM assumptions from Rel-16 eType-II |
| NVIDIA | Support the proposal. |
| Fraunhofer | A common dataset from TR 38.901 can be assumed as a starting point. However, dataset generated from field set should be eventually used for final evaluation. |
| Lenovo | Agree that the dataset for AI/ML training/validation/testing is generated by TR 38.901, at least for spatial/frequency CSI compression use case. We also support initiation of discussion on how we can produce a common field data which is a good representative of the environment. We believe evaluations with such data is very valuable in the next phase of the AI/ML. |
| LG | Support the proposal. |
| CATT | Synthetic dataset generated by TR 38.901 can be considered as baseline. We are open to optionally use dataset from field, if solid field dataset are available. |
| ZTE | We think synthetic dataset generated from TR38.901 can be more convenient for companies to align and calibrate because TR38.901 has been used for channel modeling since NR. So it is a good starting point for evaluating AI/ML models using synthetic dataset generated by calibrated channel models. FFS: Any modifications to existing channel model ,e.g. spatial consistency, can be discussed and determined. |
| Samsung | In addition, CDL extension with different distribution of channel parameters should be considered for the generalization verification |
| Fujitsu | Yes, TR38.901 should be the starting point for the evaluation, and no additional new model needs to be used unless it is demonstrated that 38.901 is not sufficient to cover typical scenarios of interest. As to field data, we don’t think it is necessary to be considered in this SI. |
| OPPO | We agree that the dataset for AI/ML training/validation/testing is generated by TR 38.901 channel models as a starting point. Moreover, if field test dataset with can be provided after its quality is examined, we also support to construct the field test dataset for evaluation. |
| Intel | We support construction of a common data-set where companies can contribute some data, this is because there is still a lot of variability on how companies generate channels and is customary in the literature to use a common data-set for AI/ML evaluation |
| Beijing Jiaotong University | Agree with the proposal. |

### 2.2-3: Calibration of dataset and/or AI/ML model

As the dataset and AI/ML models over companies may be different, the issue on how to calibrate the dataset and/or AI/ML models over companies have been raised by a number of companies. The views from contributions can be summarized into 4 options. On the other hand, one company thinks there is no need to perform calibration as long as the dataset is generated from 3GPP simulated channel.

- Option 1: Align the parameters for channel generation in the simulation (5 companies) - Option 2: Share the dataset to public for cross check/calibration over companies (2 companies) - Option 3: Generate a common/open/shared dataset (4 companies) - Option 4: Align the AI/ML model, e.g., for calibration purpose or taken as a reference. (5 companies) - Option 5: No need to perform calibration (1 company) - Option 6: Other

Question 2.2.9: For the evaluation of the AI/ML based CSI feedback enhancement, if the dataset for AI/ML training/validation/testing is generated by TR 38.901 channel models, which of the following option(s) do you prefer for the calibration on the dataset and/or AI/ML model over companies?

- Option 1: Align the parameters (e.g., for scenarios/channels) for generating the dataset in the simulation

- Option 2: Share the dataset and/or model files to public for cross check over companies

- Option 3: Generate a common dataset

- Option 4: Align the AI/ML model (e.g., loss function, hyper-parameter, etc.)

- Option 5: No need to perform calibration

- Option 6: Other

| Option 1 | Supporting companies | Xiaomi, vivo Huawei/Hisi, CAICT, Ericsson, CMCC, Nokia/NSB, NVIDIA, InterDigital, F, Charter, Futurewei, Lenovo, LG, Panasonic, CATT, ZTE Samsung Fujitsu Qualcomm Apple OPPO, Beijing Jiaotong University, AT&T |
| --- | --- | --- |
|  | Objecting companies |  |
| Option 2 | Supporting companies | Vivo, CAICT, CMCC, NVIDIA, Panasonic, CATT OPPO, Intel, Beijing Jiaotong University |
|  | Objecting companies | Ericsson, Lenovo Qualcomm |
| Option 3 | Supporting companies | Xiaomi, MediaTek, NVIDIA, Futurewei, Lenovo, Panasonic, CATT OPPO, Intel, Beijing Jiaotong University, AT&T |
|  | Objecting companies | Ericsson Samsung Qualcomm |
| Option 4 | Supporting companies | Xiaomi, CMCC, NVIDIA, Lenovo (only for calibration) OPPO, Intel, Beijing Jiaotong University |
|  | Objecting companies | Ericsson Qualcomm |
| Option 5 | Supporting companies | Qualcomm |
|  | Objecting companies | Xiaomi, Lenovo Samsung |
| Option 6 | Supporting companies |  |
|  | Objecting companies |  |

| Company | View |
| --- | --- |
| Xiaomi | AI model and dataset calibration is necessary to align companies’ understanding on the baseline performance of AI based CSI feedback, and it is necessary for comparing the performance of different AI models in the future. A common dataset would be helpful to align the calibration results. |
| vivo | We prefer both option 1 and option 2. In the dataset construction, there are two alternatives; either to align the parameters or to share the datasets with the parameters in use. By the way, our initial data set file for CSI compression and CSI prediction can be downloaded from the link ([5] and [6] in R1-2203550). |
| Huawei/Hisi | Option 1 is the simplest way for 3GPP, and as the parameters are aligned, the dataset will more or less be aligned. The EVM table in Question 2.2.2 can be a reference for parameter alignment. For other options, more efforts may be needed in 3GPP level, and anyhow Option 1 may need to be a starting point for other options. |
| CAICT | We prefer both option 1 and option 2 is considered for performance calibrations. |
| MediaTek | Common dataset is preferred due to easy calibration. |
| Ericsson | We don’t see a need for 3GPP to agree on a baseline performance of AI. The Rel-16 Type-II (normal) can be used as the baseline and companies can compare the benefits against this classical baseline. |
| CMCC | The dataset and AI model should be aligned so that 3GPP could have a common understanding on the performance of AI/ML based approach. For the dataset, we think at least option 1 is needed. Besides, companies can share their dataset voluntarily as option 2. And maybe option 3 is not needed if option 2 is adopted. For the AI model, we think option 4 is needed so that people could align their evaluation results more efficiently. |
| Nokia/NSB | Agreeing to a common set of EVM assumptions should be enough to ensure that the datasets used by companies are consistent. Willing companies may share their models but this should be on a voluntary basis |
| NVIDIA | In general, we feel the mobile industry should evolve towards a more open ecosystem. Sharing data set and model files in a public accessible way are good initiatives in line with progressing towards openness. |
| Fraunhofer | Option 1 is enough to generate similar data set for fair comparison and calibration. |
| Charter | Aligning the channel/scenario parameters for LLS and SLS simulations is sufficient for synthetic data set generation. For field data set, if required in later evaluation stages, a common data set should be provided. No alignment is needed for AI model, however; the AI model design/parameters need to be reported by companies for better understanding the provided gains. |
| Futurewei | As AI/ML model parameters are determined by training data, ideally, common datasets should be used for apple-to-apple comparison; thus, our first choice is Option 3. However, for convenience of the study, alignment on simulation parameters in generating datasets and intermediate evaluation metrics is a compromise; thus, Option 1 is our second choice. If option 1 is adopted, results from multiple runs, e.g., 10 with random sampling (where applicable) should be reported in distributions/cdf plots when companies share their results. |
|  | Option1: We agree. The parameters for the evaluation dataset should be agreed upon. The training dataset might be different between companies, but for each proposal it is still needed to present what training data parameter assumptions is used Option2: If it is not a common model/data it may be very complicated and time consuming to run the model and use the data of other companies. So as it is not practical, we are not in favor of this option. Option3: It helps on evaluation of different proposals if we can have a common dataset (at least for testing and calibration). So we are in favor of this option conditioned on that the dataset is a good representative of different scenarios. Option4: As there could be very different methods, alignment of different companies are not needed. However, it is useful for calibration purpose. Though the performance of all of proposed models should be evaluated against a set of complete KPIs (e.g. inference accuracy, complexity, overhead, generalization and ….) Option 5: We need to have some level of agreement at least on the evaluation scenarios otherwise the evaluation results are not comparable. As it is a new area for RAN1, some initial calibration phase would be useful. |
| LG | Option 1 is preferred in order for easy of calibration. |
| CATT | We are open to Option 1-3. Regarding the AI/ML model, companies are encouraged to report their design, but it is not necessary to align AI/ML model among companies. |
| ZTE | We think it is prior to choose Option 1 as a baseline, because channel model has been calibrated in previous release and we only need to align the parameters for generating the dataset in the simulation. Option 2 and Option 3 can be further discussed for preliminary KPI calibration, however, they mostly depend on the sub use cases for various dataset generation. Therefore, it may cost much time to align the dataset generation, which may bring extra workload. |
| Samsung | Share details of AI/ML models for reproducibility. Details may include model architecture, loss-functions, hyper-parameters, etc. |
| Fujitsu | First of all, we strongly support to have the calibration since AI/ML model itself can be treated as a black box, so the input dataset and several common parameters have to be aligned for performance comparison among companies. We support to have a set of common parameters for scenarios/channels so that the features of the generated dataset will be similar. It is not necessary to unify the dataset itself, because it can be dependent on different AI/ML model implementation by companies. Moreover, it is not suggested sharing multiple models and datasets by different companies since it will increase the workload for analyzing at this stage. |
| Qualcomm | We prefer option 1 and 5. Generating data based on TR 38.901 and an agreed set of EVM assumptions is sufficient. There should be no need for further calibration of the data. While R16 Type II can be agreed as the baseline scheme, there is no need to align the AI model used by companies. |
| OPPO | From our view, a common dataset is the best option for calibrating. Or at least, the parameters for generating the dataset should be aligned. Furthermore, to better calibrate the AI performance, we also suggest to have a common reference AI model for cross check between companies. |
| Beijing Jiaotong University | We prefer option 1-4 are considered for performance calibrations. |

### 2.2-4: Metrics

Intermediate KPIs

Generalized cosine similarity (GCS), Squared GCS (SGCS), Mean square error (MSE), normalized mean square error (NMSE), and equivalent MSE have been raised in the contributions as the intermediate KPIs to reflect the accuracy of the AI/ML model. 20 companies adopt/evaluate GCS/SGCS as the intermediate KPI in their contributions, 7 companies adopt/evaluate MSE/NMSE as the intermediate KPI, and 1 company evaluate with equivalent MSE. Let’s see if we need to/can align on one of them as the intermediate KPI for CSI feedback evaluation.

Question 2.2.10: For the evaluation of the AI/ML based CSI feedback enhancement, do you agree that the intermediate KPI should be adopted as part of the ‘Evaluation Metric’ to compare the similarity/error between the input original CSI of the AI/ML model and the output CSI (e.g., recovered/predicted CSI) of the AI/ML model? If so, which option(s) do you prefer?

- Option 1: Generalized cosine similarity (GCS) or Squared GCS (SGCS)

  - Note: wi is the input original CSI vector of frequency unit i, and wi is the output CSI vector of frequency unit i. N is the total number of frequency units. E{∙} denotes the average operation over multiple samples.

GCS=E1Ni=1N||wiHwi ||||wi||  ||wi||,  SGCS=E1Ni=1N||wiHwi ||||wi||  ||wi||2

- Option 2: Normalized mean square error (NMSE)

  - Note: H is the original CSI vector/matrix, and H is the output CSI vector/matrix. E{∙} denotes the average operation over multiple samples.

NMSE=E{H-H2H2}

- Option 3: Both Option 1 and Option 2 can be adopted up to company’s report

- Option 4: Other

Please provide your company name on whether to support the main text of the question to the 1st table, and your preferred option to the 2nd table.

| Supporting companies | Xiaomi, vivo Huawei/Hisi, CAICT, MediaTek, CMCC, Nokia/NSB, Ericsson, InterDigital, Fraunhofer, Charter, Lenovo, Panasonic, CATT, ZTE Samsung Fujitsu Apple OPPO, Beijing Jiaotong University |
| --- | --- |
| Objecting companies | Qualcomm |

| Option 1 | Supporting companies | Xiaomi, vivo Huawei/Hisi, CAICT, MediaTek, CMCC, Fraunhofer, CATT Fujitsu Apple OPPO, Intel (only for single rank/layer) , Beijing Jiaotong University |
| --- | --- | --- |
|  | Objecting companies | Ericsson, Lenovo (it is good in conjunction with another metric) |
| Option 2 | Supporting companies | Xiaomi, vivo, CATT Fujitsu, Beijing Jiaotong University |
|  | Objecting companies | Ericsson, Lenovo |
| Option 3 | Supporting companies | Nokia/NSB, NVIDIA, InterDigital, Charter, Futurewei, CATT, ZTE Samsung Intel, Beijing Jiaotong University |
|  | Objecting companies | Ericsson, Lenovo |
| Option 4 | Supporting companies | Ericsson (see comment below), Fraunhofer, Lenovo |
|  | Objecting companies |  |

| Company | View |
| --- | --- |
| Xiaomi | It is better to align the metric. GCS/SGCS can be used for eigenvector based CSI compression, NMSE can be used for full channel matrix based CSI compression. For Option 1, wi can also be ideal CSI vector. It needs to clarify why wi is original CSI vector of frequency unit i |
| vivo | In option 1, we prefer SGCS which is more realistic to represent the correlation of the channel. |
| Huawei/Hisi | At least for the CSI compression sub use case, GCS reflects the accuracy of the compressed eigenvector, which is more similar to the R16/17 PMI based mechanism. But we are open to hear more voices from Option 2 companies. |
| CAICT | Our first preference is option 1 and also fine with option 2. |
| MediaTek | Don’t need to use both metrics. GCS/SGCS is enough. |
| CMCC | We prefer Option 1 to reflect the accuracy of CSI. We are also fine with Option 2. |
| Nokia/NSB | Different metric may be used for CSI compression and CSI-prediction sub-use case, for example. Either metric may also be preferable depending on whether the channel or its eigenvectors are compressed |
| NVIDIA | Agree with Xiaomi: GCS/SGCS can be used for eigenvector based CSI compression, NMSE can be used for full channel matrix based CSI compression. |
| Ericsson | We see serious issues with the use of CS and GCS as discussed in Section 2.2.2 of our tdoc R1-2203282. - Problems with loss functions using cosine similarity and generalized cosine similarity (Option 1) - They are not defined multi-layer transmissions (i.e., rank > 1). - They do not map well to the objective of maximizing downlink throughput. - For example, imagine a single-layer transmission over a channel with two distinct main clusters for which the corresponding precoding vectors are approximately orthogonal (i.e., the strongest and second strongest eigenvectors). If the pathloss of both clusters is approximately the same, then in SU-MIMO it does not matter which eigenvector is used. However, the cosine similarity of the reconstructed precoding vector with respect to the true strongest eigenvector will heavily (and unnecessarily) punish selecting the second strongest eigenvector as the precoding vector. - Problems with loss functions using squared error and normalized squared error (Option 2) These per-sample loss functions can require the UE AI and NW AI to compress and reconstruct aspects of the channel that are irrelevant for SU or MU-MIMO performance, such as the channel subspace represented by the weakest eigenvector. Using such loss functions may thus lead to unnecessary large uplink overhead as the CSI contains information that is not useful for the scheduler. Hence, we consider a more elaborate KPI that better reflect the true performance for SU or MU-MIMO, and that takes received SNR into consideration, see Section 2.2.2 of our tdoc R1-2203282. |
| Fraunhofer | AI-based methods use explicit CSI feedback while Type II CSI is based on explicit CSI. So NMSE is not a good metric for comparison. GCS/SGCS can be used instead. On the other hand spectral efficiency for multiuser scenarios is more realistic metric since it mimics the total throughput. In LLS, the performance gain e.g., BLER is also a proper metric. |
| Lenovo | Option 1: This metrics only consider the first Eigen vector which might be not enough information about the channel state. So we need this metric as well as another metric to show how well other eigenvectors can be estimated by the gNB. Please look at the metric defined below. Option 2: As finding the best beamforming vector(s) may not need all details of the CSI, this metric is not needed. Option 3: As we do not need to all details of the CSI for finding the best beamforming vector(s), option 2 is not needed. Option 4: We propose to have the following metrics as the intermediate performance metrics. GCS1 is what defined in Option 1 and GCSfull captures the similarity of all eigenvectors. GCS1=E1Ni=1N¦¦wi1Hwi1 ¦¦¦¦wi1¦¦ ¦¦wi1¦¦, GCSfull=E1Ni=1NjKλij¦¦wijHwij ¦¦¦¦wij¦¦ ¦¦wij¦¦, Where wijand λij are the jtheigenvector and its associated eigenvalue at frequency unit i and k is the rank of the channel. |
| LG | Option 1 seems sufficient as it is closely related to throughput performance. |
| CATT | We prefer to capture the metric to the 1st table. We think both option 1 and option 2 can be considered. Option 1 can be used for sub use cases with eigenvector as input, and option 2 can be used for sub use cases with full channel as input. We are also open to other new KPIs. |
| ZTE | We think the intermediate KPI is necessary for performance evaluation and calibration among companies. GCS/SGCS is a good metric to evaluate the accuracy of recovered eigenvectors and NMSE can be used to evaluate the accuracy of recovered channel matrix. |
| Fujitsu | We support to have the cosine similarity and NMSE as the intermediate KPIs for evaluating the CSI feedback. These two metrics are broadly adopted in related research and can be regarded as good reflector of similarities between two vectors. |
| Qualcomm | The similarity metric should be defined between the recovered output CSI and the desired output CSI. Note that in general, the desired output CSI may be different from the input or the AI/ML model. For example, the encoder input could be the raw channel matrix and the decoder output could be the eigen vector of the channel. What input to use – whether/how to perform SVD on the channel H to derive an eigenvector v, or to directly feed H into an encoder, is up to company’s decision. The group only needs to agree on the output and define a metric between the ideal output (i.e., SVD of H) and the decoder output. As for the metric, we think we can use GCS in case the output is the eigenvector v and NMSE in case the output is the channel H. Having said that, we can define an “equivalent MSE” based on GCS, as MSE=2-2*GCS, as explained in Qualcomm contribution. This is a unified way that allows the group to compare the performance of v output and H output using the same metric. So, we propose to use the equivalent MSE that covers both option 1 and option 2. For rank > 1, discussion is needed. |
| Apple | For higher rank, consider eigen-value weighted cosine similarity. NMSE can be used for CSI prediction. NMSE does not provide much insight comparing to type II CSI baseline. |
| OPPO | We prefer option 1 with SGCS as the intermediate KPI. |
| Beijing Jiaotong University | We prefer Option 1 to reflect the accuracy of CSI and also fine with Option 2. |

Capability/complexity related KPIs

For processing complexity, 9 companies raise to adopt Floating point operations (FLOPs) as the KPI for processing complexity.

For memory storage, 9 companies raise to adopt AI/ML model size as the KPI for processing complexity, while 5 companies raise to adopt the number of AI/ML parameters to reflect the storage requirement.

As per Moderator’s understanding, companies may report such capability/complexity related KPIs as part of the ‘Evaluation Metric’ added on top of the EVM table in Question 2.2.2.

Question 2.2.11: For the evaluation of the AI/ML based CSI feedback enhancement, do you agree that Floating point operations (FLOPs) should be adopted as part of the ‘Evaluation Metric’, and reported by companies?

| Supporting companies | Xiaomi, vivo Huawei/Hisi, CAICT, MediaTek, Ericsson, CMCC, Nokia/NSB, NVIDIA, InterDigital, Fraunhofer, Charter, Futurewei, Lenovo (with a note), LG, Panasonic, CATT, ZTE Samsung Fujitsu Qualcomm Apple OPPO, Intel, Beijing Jiaotong University, AT&T |
| --- | --- |
| Objecting companies |  |

| Company | View |
| --- | --- |
| Xiaomi | Besides the FLOPs, we think the power consumption should also be considered. |
| vivo | FLOPs can be used to measure the computation complexity and delay. However, this should be discussed in the general discussions on AIML framework. |
| Huawei/Hisi | It can be reported by companies to reflect the model complexity, and reflect the relationship between complexity and performance. |
| CAICT | FLOPs Should be the main KPI for AI model complexity. |
| MediaTek | FLOPs can be used for complexity comparison. |
| CMCC | FLOPs can be adopted as a metric to evaluate the complexity of AI model. |
| NVIDIA | FLOPs is an important metric for complexity characterization. |
| Fraunhofer | FLOPs can be considered for measuring the complexity. Also, the total data storage should be considered |
| Lenovo | Complexity of an AI/Ml model is different in different stages: Complexity of training phase: We should report the complexity if the model is going to be trained using online field data, but if the model will be trained using offline field data or simulated data we do not need to report training complexity. Complexity of inference phase: it should be reported Complexity of the model update phase: Should be reported if it happens online. |
| LG | FLOPs can be considered as a good metric to compare AI models. |
| CATT | We think the computation complexity of AI model should be taken into consideration as the KPI, which can be represented by FLOPs. Therefore FLOPs should be adopted as part of‘Evaluation Metric’. |
| ZTE | We think FLOPs can be initially adopted as a evaluation metric for AI/ML model comparison. However, different inference latency may emerge even with the same Flops because of different hardware configurations. Therefore, whether some other metrics are needed for complexity evaluation should be further discussed. |
| Fujitsu | It is always necessary to have a proper evaluation on the complexity of the AI/ML model, and FLOPs is one commonly recognized metric which can reflect the computational complexity of the AI/ML model. It will be helpful for evaluating the trade-off between the performance and the cost of an AI/ML model. |
| Qualcomm | Agree with the proposal. |
| Apple | Suggest change “should” to “can” We would like to clarify whether the detailed NN structure and parameters will be shared together with FLOPs or just FLOPs number is proposed? |
| OPPO | We agree that FLOPs can be used for AI model complexity evaluation. |
| Beijing Jiaotong University | The model complexity can be measured by FLOP, and the hardware requirement can be measured by FLOPS. |

Question 2.2.12: For the evaluation of the AI/ML based CSI feedback enhancement, do you agree AI/ML memory storage should be adopted as part of the ‘Evaluation Metric’, and reported by companies? If so, which of the following format(s) do you prefer?

- Option 1: AI/ML model size, i.e., number of bytes occupied by the AI/ML model

- Option 2: Number of AI/ML parameters

- Option 3: Other

Please provide your company name on whether to support the main text of the question to the 1st table, and your preferred option to the 2nd table.

| Supporting companies | Xiaomi Huawei/Hisi, CAICT, MediaTek, Ericsson, CMCC, Nokia/NSB, NVIDIA, InterDigital, Fraunhofer, Charter, Futurewei, Lenovo, LG, Panasonic, CATT, ZTE Samsung Qualcomm Apple OPPO , Beijing Jiaotong University |
| --- | --- |
| Objecting companies |  |

| Option 1 | Supporting companies | Xiaomi Huawei/Hisi, CAICT, CMCC, NVIDIA, InterDigital, Lenovo, LG, CATT Samsung Fujitsu OPPO, Beijing Jiaotong University |
| --- | --- | --- |
|  | Objecting companies |  |
| Option 2 | Supporting companies | vivo Huawei/Hisi, MediaTek, CMCC, Nokia/NSB, NVIDIA, Futurewei, Lenovo Samsung Qualcomm Apple OPPO, Intel |
|  | Objecting companies |  |
| Option 3 | Supporting companies |  |
|  | Objecting companies |  |

| Company | View |
| --- | --- |
| Xiaomi | Even though the number of AI/ML parameters are same, different platforms may lead to different model sizes. Hence, Option 2 does not show the actual model size. Option 1 is direct and objective. Besides the memory size, we think the power consumption should also be considered |
| vivo | In our view, AI model size is determined by the number of AI parameters, the number of quantification bits and the data format. Hence, the AI model size is not suitable for fair memory storage comparison. We believe that the number of float AI parameters can be used to represent the memory storage as a starting point and the number of quantification bits can be further studied for memory storage and computation complexity. We prefer to use the number of AI/ML parameters and the number of quantification bits. However, this should be discussed in the general discussions on AIML framework. |
| Huawei/Hisi | It can be reported by companies to reflect the model complexity, and reflect the relationship between model size and performance. For the specific parameter, we think both AI/ML model size in bytes and the AI parameter number can be reported. |
| CAICT | Model size is enough to measure the required memory storage. |
| MediaTek | Prefer option 2 |
| Ericsson | Proponent need to explain whether a counted parameter is real or complex valued. |
| CMCC | Both AI model size and number of AI parameters can be the reference metrics to show AI model memory storage. |
| NVIDIA | Both options are useful information for companies to understand each other’s model size. |
| Fraunhofer | The total required memory storage of the AI-based method can be determined by the number of parameters. Then, the overall model size can be calculated directly which depends on the allocated bits to each parameter. |
| Charter | In the case of autoencoders, it is better to report number of parameters/model size separately for encoder and decoder, since each part reside in different entity (gNB or UE) |
| Futurewei | Number of parameters in the model and model size are highly correlated even though they may not be exactly the same as model size also includes additional space for the model architecture information. Number of model parameters is adopted as the common indicator for model size in most industry and academic articles, and it is readily available from the development environment / model summary. Given all these, we support using number of AI/ML model parameters. |
| Lenovo | Both option 1 and option 2 provide relevant information. |
| CATT | This can be discussed after we gain more understanding on the employed AI/ML model. |
| ZTE | We think AI/ML memory storage should be adopted as a evaluation metric, because high model complexity will increase storage overhead. The number of parameters may be improper because the quantization of float numbers may not be calculated. Besides, we are also not sure how to evaluate the trade-off between system performance and AI/ML model size and it is hard to define a upper bound of AI model size, which is needed for further study. |
| Samsung | Both option 1 and 2 can be supported. |
| Fujitsu | We support to use model file size as the storage KPI since it is straightforward and easy to understand, it is necessary to avoid potential confusion introduced by ambiguous expressions such as “Number of AI/ML parameters” which need extra workload to first clarify related concepts. |
| Qualcomm | We prefer using the number of parameters as the metric. |
| OPPO | We think that both Option 1 and Option 2 can be accepted. Relative information should be provided when a company propose an AI/ML algorithm. |
| Beijing Jiaotong University | We prefer option1 than option2. |

### 2.2-5: Methodology for generalization

Methodology of verifying generalization performance

8 companies raise to adopt mixed the training dataset from various scenarios/configurations to verify the generalization performance. 9 companies discuss in their contributions to verify the AI/ML model under various testing/inference scenarios/configurations. As comparison baselines, training dataset from a single scenario/configuration can be adopted, which can be applied to the same scenario/configuration (overfitting may occur) and to a different scenario/configuration. So the following question is raised to collect the views from companies on the methodology of verifying generalization of the dataset or AI/ML model.

Question 2.2.13: For the evaluation of the AI/ML based CSI feedback enhancement, do you agree that it should be considered to verify the generalization of the AI/ML model? If so, which of the option(s) do you prefer for the verification?

- Option 1: The training dataset is constructed by mixing training inputs from multiple configurations/scenarios, and the testing/inference is performed for a single configuration/ scenario

  - FFS: if the single configuration/ scenario belongs to the multiple configurations/scenarios

- Option 2: The training dataset is constructed by training inputs from a single configuration#A /scenario#A, and the testing/inference is performed for a different single configuration#B/scenario#B

- Option 3: The training dataset is constructed by training inputs from a single configuration#A /scenario#A, and the testing/inference is performed for the same configuration#A/scenario#A

- Option 4: Other

Please provide your company name on whether to support the main text of the question to the 1st table, and your preferred option to the 2nd table.

| Supporting companies | Xiaomi Huawei/Hisi, CAICT, MediaTek, Ericsson, CMCC, Nokia/NSB, NVIDIA, InterDigital, Fraunhofer, Charter, Futurewei, Lenovo, LG, Panasonic, CATT, ZTE Samsung Fujitsu Qualcomm OPPO, Beijing Jiaotong University, AT&T |
| --- | --- |
| Objecting companies |  |

| Option 1 | Supporting companies | Xiaomi (Second preference), vivo Huawei/Hisi, MediaTek, Ericsson, CMCC, Nokia/NSB, NVIDIA, InterDigital, Fraunhofer, Charter, LG, CATT Samsung Fujitsu Qualcomm Apple OPPO, Intel, Beijing Jiaotong University, AT&T |
| --- | --- | --- |
|  | Objecting companies | Lenovo |
| Option 2 | Supporting companies | vivo Huawei/Hisi, CMCC, Nokia/NSB, Fraunhofer, Charter，CATT Fujitsu Intel |
|  | Objecting companies | Lenovo OPPO |
| Option 3 | Supporting companies | Xiaomi (First preference) Huawei/Hisi, CAICT, Nokia/NSB, NVIDIA, ZTE Qualcomm Apple OPPO, Intel, AT&T |
|  | Objecting companies | Ericsson, InterDigital, Lenovo |
| Option 4 | Supporting companies | Futurewei, Lenovo |
|  | Objecting companies |  |

| Company | View |
| --- | --- |
| Xiaomi | Option 3 is preferred. Clarification needed for the same configuration#A/scenario#A, in our understanding, same scenario can even have different detailed parameter assumptions, then the generalization of AI model can be evaluated by option 3. Option 1 can also be considered if there is no much performance loss. |
| vivo | We prefer both option 1 and option 2. We believe that option 2 should be the baseline to verify the generalization ability of the AI model. Option 1 can be considered as an enhanced dataset construction way, which may improve the generalization ability. As for option3, it can be seen as a particular case of option 1. |
| Huawei/Hisi | Option 2 and Option 3 can be considered as baselines, e.g., performance degradation may be suffered under Option 2 due to weak generalization, while the performance under Option 3 will be optimistic due to overfitting. Option 1 performance may stay in between, or close to Option 3. Option 2 can also be used to verify the similarity between different scenarios/configurations. E.g., if the AI/ML model trained under one scenario performs also good under another scenario, it means the two scenarios are similar enough to be applied with the same model. Option 3 can be considered as an example of scenario-specific model, while Option 1 can be considered as a generalized model. |
| CAICT | We observe that with the same AI/ML model structure could achieve good performance under different SLS scenarios. Therefore, the benefit of option 1 over option 3 to verify generalization of AI/ML model is limited. The performance of Option 2 is expected to be low and option 2 is not proposed. |
| MediaTek | Option 1 is preferred |
| Ericsson | Option 3 doesn’t’ make sense at all, it produces overfitting models. Results will produce an overestimate of the benefits of the AI-CSI compared to reality and is irrelevant for the SI conclusion. |
| CMCC | We think Option 2 can be the baseline to verify generalization performance. Using Option 1, the AI model might perform better in generalization ability. |
| Nokia/NSB | All 3 options seem valid to test the robustness of an ML model against variations of the configuration/scenario between training and inference. Option 3 can be used as upper bound reference |
| NVIDIA | - Option 1 may mimic the construction of a AI/ML model that is as generic as possible and then its application to a specific scenario. - Option 3 may mimic the scenario specific use of AI/ML for wireless. |
| InterDigital | Agree with Ericsson that Option 3 will show too optimistic performance results from AI model which is not realistic unless we will have AI models for all possible individual scenarios/configurations and switching them based on the scenarios/configurations. |
| Fraunhofer | In our opinion, options 1 and 2 should be studied and implemented. The performance of the network should be compared in all three cases. It is not obvious which option leads to the better performance. Option 1 converges to a global optimum network for all scenarios. Option 2 measure the robustness of the trained network whether works well for other scenarios. But option 3 trains the AI for unrealistic scenario which is working only for a specific scenario. |
| Futurewei | Please see the details of our feedback regarding model generalization in the response for 2.2.16. At high-level, we should first discuss what solution deployment options need to be supported and the associated assumptions and expectations before directly jumping into the discussion regarding how to mix data from different scenarios/configurations: Solution deployment options include: - Scenario (with associated configurations) based solution - This option should be used as baseline / default solution deployment option. - For this option, the goal of model generalization verification is to make sure the trained model can generalize to data samples unseen during the training phase while the test samples are generated from the same scenario. - Regarding data generation strategy for this option, please see our response for 2.2.16. Note: companies should strive to support different configurations (e.g., # antenna ports/configurations) in the same scenario using one main AI/ML model. This means the main architecture of the AI/ML model remains unchanged while pre/post processing and adaptation (internal or external to the AI/ML model) techniques can be leveraged to support various configurations. - Generalized solution that supports multiple scenarios (each with associated configurations) - This solution deployment option can be optionally supported. - There are various possible data availability situations under this option. The associated assumptions and expectations/goal may be different from generalization perspective. The corresponding data generation strategy for training and testing may also be different; thus, they should be separately discussed. Please see more detailed discussion on this topic in our response for 2.2.16. |
| Lenovo | To test the generalization, we should test the model in different scenarios. Each company train their model using their preferred methods and preferred training dataset, but all should be evaluated against the same KPIs. Of course, we can discuss what is possibly a good training dataset, but we believe the training dataset can be different between different companies. The important part is to show the proposed model have high performance in different test scenarios/configurations. So no coordination needed for the training data and for the test dataset we propose to have a set of scenarios that we report the result for each case. We have tried to further clarify that in the following two comments and our response to Question 2.2.2. |
| LG | Option 1 is preferred |
| CATT | Either option 1 or option 2 seems fine. We wonder how to verify the generalization of the AI/ML model by option 3. |
| ZTE | We think Option 3 can be adopted as a baseline to evaluate performance compared to the legacy method. In addition, verification of other cases needs to be compared with Option 3. we are not clear different configurations mean, e.g. large-scale parameters or small-scale parameters, which should be further discussed. |
| Fujitsu | We think option 1 is the first choice, and option 2 can be considered for further study. A perfect dataset should incorporate multiple scenarios with diverse features which can be learnt by the model. A fine selection for the scenarios will help the model to converge. Option 2 also deserves some study because it will save the overall overhead for deploying the AI/ML model in different scenarios, but this work should be left for further study and it is not the target at this stage. |
| Qualcomm | Option 3 can be used for initial verification and option 1 can be used for final evaluation. |
| OPPO | We support that Option 1 and Option 3 should be considered. Specifically, Option 3 can be used to evaluate the performance of AI model and TypeII baseline. Option 1 can be used to evaluated the generalization performance of AI model. Specifically, we think that the single configuration/scenario should belong to the multiple configurations/scenarios. Moreover, we object Option 2 for verification since it is meaningless to evaluation AI performance when training dataset and testing dataset are unmatched. |
| Beijing Jiaotong University | Option 1 is preferred |

Assumptions on scenario(s)/configuration(s)

If the preference of Question 2.2.13 is Option 1/2/3, then the next question is, what specific scenario(s)/configuration(s) should be assumed in the EVM to generate/mix the dataset. Note that changing the scenario may not necessarily mean the change of the dimension of the AI/ML input/output, while changing the configurations may lead to the change of the dimension of the AI/ML input/output. So the scenarios and configurations are asked in two separate questions.

Question 2.2.14: For the evaluation of the AI/ML based CSI feedback enhancement, which of the following scenario(s) can be considered for verifying the generalization of the AI/ML model?

- Scenario: (e.g. UMa, UMi, FFS InH)

  - Supporting companies: Huawei/Hisi (2nd), Samsung Qualcomm

- Outdoor/Indoor UE distribution (e.g., Outdoor/Indoor UE ratio)

  - Supporting companies: Huawei/Hisi (1st), Nokia/NSB Samsung

- Outdoor UE speed (e.g., 3km/h, 30km/h, etc.)

  - Supporting companies: Huawei/Hisi (1st), Nokia/NSB Samsung

- Frequency range (e.g., 2GHz, 4GHz, etc.)

  - Supporting companies: Huawei/Hisi (2nd)

- Numerology (e.g., 15kHz, 30kHz, etc.)

  - Supporting companies: Huawei/Hisi (2nd)

- Inter-BS distance (e.g., ISD=200m, 500m)

  - Supporting companies: Huawei/Hisi (2nd)

- Other

  - Supporting companies:

| Company | View |
| --- | --- |
| Moderator | If you prefer a mixed dataset in Question 2.2.13, please elaborate your suggested method/ratio of mixing different scenarios/configurations into one dataset for the specific bullet that you prefer in this question. |
| vivo | We believe that each scenario and the relevant scenario combination should be taken into account in the simulation. In some specific scenarios, Uma, Umi and InH could be considered together. In terms of the frequency range, as we indicated previously, a specific frequency range is not necessarily limited as long as the frequency range is in FR1. |
| Huawei/Hisi | For a cell-specific AI/ML model, it needs to be generalized enough to all UEs in this cell, including different UE speeds, and indoor/outdoor UEs (with potentially varying ratio). Therefore, these two scenarios should be considered with slightly higher priority. Other scenarios can be also considered for a more universal AI/ML model. |
| Ericsson | The mix provided by the FL looks good to us. Mixing ratio can be 1/n per parameter if there are n alternatives per parameters. |
| NVIDIA | The list provides a good set of mixed scenarios. |
| Futurewei | Please see the details of our feedback regarding model generalization in the response for 2.2.16. At high-level, we should first discuss what solution deployment options need to be supported and the associated assumptions and expectations before directly jumping into the discussion regarding how to mix data from different scenarios/configurations: Solution deployment options include: - Scenario (with associated configurations) based solution - This option should be used as baseline / default solution deployment option. - For this option, the goal of model generalization verification is to make sure the trained model can generalize to data samples unseen during the training phase while the test samples are generated from the same scenario. - Regarding data generation strategy for this option, please see our response for 2.2.16. Note: companies should strive to support different configurations (e.g., # antenna ports/configurations) in the same scenario using one main AI/ML model. This means the main architecture of the AI/ML model remains unchanged while pre/post processing and adaptation (internal or external to the AI/ML model) techniques can be leveraged to support various configurations. - Generalized solution that supports multiple scenarios (each with associated configurations) - This solution deployment option can be optionally supported. - There are various possible data availability situations under this option. The associated assumptions and expectations/goal may be different from generalization perspective. The corresponding data generation strategy for training and testing may also be different; thus, they should be separately discussed. Please see more detailed discussion on this topic in our response for 2.2.16. |
| Lenovo | No coordination is needed for the training dataset. For the test dataset we propose to have a set of scenarios that we report the result for each scenario as mentioned in our response to Q2.2.2. |
| CATT | We support to consider all about scenarios. Details can be further discussed. |
| ZTE | We tentatively agree on mixing the data from multiple scenarios, e.g. UMa and UMi, while other parameter configurations for data mixing needs further discussion. It seems that there is much workload for companies to align and generate dataset. |
| Samsung | Outdoor UE speed (Y1% 10km/h, Y2% 20km/h, Y3% 30km/h) Outdoor/Indoor (Y1% Outdoor, Y2% Indoor) FFS : Y1,Y2, Y3 |
| Qualcomm | We support studying generalization across different layout scenarios, e.g., UMa, UMi, etc. |
| Apple | The generalization study needs to consider evaluation effort. Further reduce the combinations are preferred. |
| OPPO | We prefer the mixed UMa and UMi dataset with proportion 50% and 50%. Since UMa and UMi include both 80% indoor UEs with speed 3km/h and 20% outdoor UEs with speed 30km/h, we think that outdoor/indoor mixing and outdoor UE speed mixing are not very necessary. |
| Intel | We are supportive of such generalizations |
| Beijing Jiaotong University | We support to consider all about scenarios and details can be further discussed. |

Question 2.2.15: For the evaluation of the AI/ML based CSI feedback enhancement, which of the following configuration(s) can be considered for verifying the generalization of the AI/ML model:

- Antenna port number (e.g., 32 ports, 16 ports)

  - Supporting companies: Huawei/Hisi, Nokia/NSB

- Antenna configurations (M,N,P,Mg,Ng,Mp,Np)

  - Supporting companies:

- Bandwidth: (e.g., 10MHz, 20MHz)

  - Supporting companies: Huawei/Hisi,

- CSI feedback payload

  - Supporting companies: Huawei/Hisi, Nokia/NSB

- Other

  - Supporting companies:

| Company | View |
| --- | --- |
| Moderator | If you prefer a mixed dataset in Question 2.2.13, please elaborate your suggested method/ratio of mixing different scenarios/configurations into one dataset for the specific bullet that you prefer in this question. |
| vivo | All configurations are mandated. In the antenna configuration, as shown in our evaluation results, the AI model trained with 0.8 wavelength slightly degrades as opposed to the configuration of 0.5 wavelength. Hence, it is necessary to study the generalization. |
| Huawei/Hisi | Antenna port number, bandwidth, and payload are all configurable to different UEs, so the generalization to these configurations needs to be evaluated. |
| Ericsson | Only gNB antennas relevant for midband MU-MIMO operations is of relevance, e.g. 32 ports. (massive MIMO deployment) |
| NVIDIA | The list provides a good set of mixed configurations. |
| Futurewei | Please see the details of our feedback regarding model generalization in the response for 2.2.16. At high-level, we should first discuss what solution deployment options need to be supported and the associated assumptions and expectations before directly jumping into the discussion regarding how to mix data from different scenarios/configurations: Solution deployment options include: - Scenario (with associated configurations) based solution - This option should be used as baseline / default solution deployment option. - For this option, the goal of model generalization verification is to make sure the trained model can generalize to data samples unseen during the training phase while the test samples are generated from the same scenario. - Regarding data generation strategy for this option, please see our response for 2.2.16. Note: companies should strive to support different configurations (e.g., # antenna ports/configurations) in the same scenario using one main AI/ML model. This means the main architecture of the AI/ML model remains unchanged while pre/post processing and adaptation (internal or external to the AI/ML model) techniques can be leveraged to support various configurations. - Generalized solution that supports multiple scenarios (each with associated configurations) - This solution deployment option can be optionally supported. - There are various possible data availability situations under this option. The associated assumptions and expectations/goal may be different from generalization perspective. The corresponding data generation strategy for training and testing may also be different; thus, they should be separately discussed. Please see more detailed discussion on this topic in our response for 2.2.16. |
| Lenovo | We believe we should consider a set of different scenarios/configurations for the evaluation purposes as mentioned in our response to Q2.2.2. The results can be reported in different cases: - One model is trained based on a training dataset and evaluated against all the scenarios/configurations - One model is trained based on a training dataset, then fine-tuned for each of the scenarios/configurations, and then evaluated against that scenario/configurations. The overhead and complexity of the fine tuning should be reported in this case - A separate model is trained for each of the scenarios/configuration and evaluated again that particular scenario/configuration. The complexity of having separate model for each case should be discussed in this case |
| CATT | We support to consider all about scenarios. Details can be further discussed. |
| ZTE | We think we can initially verify the mixed dataset with different bandwidths, e.g. 10M and 20M, while other parameter configurations for data mixing needs further discussion. It seems that there is much workload for companies to align and generate dataset. |
| Samsung | Better to stick with single value for the above parameters |
| Apple | The generalization study needs to consider evaluation effort. Consider to limit to 32 ports. |
| OPPO | We think that in the initial stage of this SID, to verify the generalization performance with different dimension of AI/ML input/output is not in top priority. This can be remained for FFS. |
| Beijing Jiaotong University | We support to consider all about scenarios and details can be further discussed. |

### 2.2-6: Others

Question 2.2.16: Do you think there are additional high priority EVM issues/parameters which are generic to all sub use cases and have not been discussed/captured in previus sections?

| Company | View |
| --- | --- |
| Futurewei | Regarding questions related to generalization, before we start discussing how to mix data from different scenarios/configurations, we need to first consider the following: - Scenario (with associated configurations) based solution should be used as baseline / default option. Note: companies should strive to support different configurations (e.g., # antenna ports/configurations) in the same scenario using one main AI/ML model. This means the main architecture of the AI/ML model remains unchanged while pre/post processing and adaptation (internal or external to the AI/ML model) techniques can be leveraged to support various configurations. - Generalized solution that supports multiple scenarios (each with associated configurations) can be optionally supported. AI/ML model generalization needs to be verified for both solution options but in different ways. For scenario-based solution option: - In this option, the AI/ML model is designed for a particular scenario and is expected to perform well with any new data samples from that scenario. - The goal of model generalization verification is to make sure the trained model can generalize to test data samples unseen during the training phase while the test samples are generated from the same scenario. - Data generation: - Training data: generated from many realizations/drops belong to the scenario that the AI/ML model is designed for. - Testing data: generated from a separate set of realizations/drops belong to the same scenario. For generalized solution option: - In this option, the AI/ML is designed for multiple scenarios (each associated configurations), e.g., scenarios 1-10. - There are different situations in terms of data availability in this option. Some typical situations include: - Sufficient representative data is available from all the scenarios that the AI/ML model is intended to be deployed to. - Sufficient representative data is available from a subset of the scenarios (e.g., scenarios 1, 3-5, 8-10) while a small number of samples is available for the remaining scenarios (e.g., scenarios 2, 4, 6, 7, 9). - Sufficient representative data is available from a subset of the scenarios (e.g., scenarios 1, 3-5, 8-10) while NO sample data is available for the remaining scenarios (e.g., scenarios 2, 4, 6, 7, 9). - The expectation of generalization for each situation: - For this situation, the model may be trained using a subset of the available data from all the scenarios and the expectation is for the trained model to perform well with new/unseen data samples from any of target scenarios (e.g., scenarios 1-10). - For this situation, the model may be trained initially using a subset of the available data from scenarios 1, 3-5, 8-10 then applied transfer or domain generalization techniques before / during / after the model training. After this tuning/adaptation procedure, the model is expected to work well with new/unseen data samples from scenarios 1, 3-5, 8-10 and data samples from scenarios 2, 4, 6, 7, 9 as well. - For this situation, if NO data (labeled or unlabeled) is available from scenarios 2, 4, 6, 7, 9, we should NOT expect the trained model (using data from scenarios 1, 3-5, 8-10) to perform as well with samples from scenarios 2, 4, 6, 7, 9. However, some meta learning techniques may be leveraged to allow the model to have decent performance. - The goal of model generalization verification for these situations: - For this situation, the goal is to verify that the trained model (using data from scenarios 1-10) can generalize to data samples unseen during the training phase from any of the 1-10 scenarios. - For this situation, the goal is to verify that the final model (either via meta/transfer learning or domain generalization) can generalize to data samples unseen during the training phase from any of the 1-10 scenarios. - For this situation, the goal is like situation b) by leveraging meta-learning techniques. - Data generation: - Situation a) - Training data: generated from many realizations of scenarios 1-10. - Testing data: generated from a separate set of realizations for scenarios 1-10. - Situation b) - Training data: generated from many realizations of scenarios 1, 3-5, 8-10 and a small number of realizations of scenarios 2, 4, 6, 7, 9. - Testing data: generated from a separate set of realizations for scenarios 10. - Situation c) - Training data: generated from many realizations of scenarios 1, 3-5, 8-10. - Testing data: generated from a separate set of realizations for scenarios 1, 3-5, 8-10 and a set of realizations for scenarios 2, 4, 6, 7, 9. For CSI feedback enhancement use case, we should discuss what model deployment options should be considered as the corresponding data availability situations before directly jumping into discussing what data to generate and how to mix them. Once we are aligned with deployment options that should be supported, the discussion regarding data generation should be much easier. For study purpose, it is recommended to only focus on data availability situations a and b for the generalized deployment option, at least initially. |

2nd round email discussions

Some high priority questions/proposals are given in the 2nd round.

### 2.3-1: Generic evaluation methodology

Simulation approach

A proposal is given based on Question 2.2.1 on the simulation approach. It looks majority companies think it is a good direction to take SLS as a baseline, while also optionally consider LLS as complement.

One additional view from NVIDIA is also to consider other open approaches such as virtual world simulation for this brand new feature. From Moderator’s view, it may be time consuming for discussion such brand new simulation methods at 3GPP, but let’s see if other companies can accept a FFS on whether/how other approaches are open for discussions.

Question 2.2.1: For the evaluation of the AI/ML based CSI feedback enhancement, which of the following simulation approach should be adopted: - Option 1: System level simulation - (24 companies) Xiaomi, vivo Huawei/Hisi, CAICT, MediaTek, Ericsson, CMCC, Nokia/NSB, NVIDIA, InterDigital, Fraunhofer, Charter, Futurewei, Lenovo, LG, Panasonic, CATT, ZTE Samsung Fujitsu Qualcomm Apple OPPO, Intel - Option 2: Link level simulation - (10 companies) Xiaomi, InterDigital, Fraunhofer, Charter, LG, Panasonic Samsung Fujitsu OPPO, Intel - Option 3: Others

The proposal is given as (Proposal 2.3.1 is editorially changed on top of Question 2.2.1):

Proposal 2.3.1:Question 2.2.1: For the evaluation of the AI/ML based CSI feedback enhancement, which of the following system level simulation approach should be is adopted as baseline:

- Link level simulation is optionally adopted

- [FFS: whether/how other simulation approach can be considered]

- Option 1: System level simulation

- Option 2: Link level simulation

- Option 3: Others

It looks the [FFS] part in the last version causes some confusions. Moderator updates the Proposal 2.3.1 by removing that part as in below. @Companies please provide your preference/comments based on the updated proposal (changed on top of Ericsson’s clean version):

Proposal 2.3.1: For the evaluation of the AI/ML based CSI feedback enhancement, system level simulation approach is adopted as baseline

- Link level simulation is optionally adopted

- [FFS: whether/how other simulation approach can be considered]

| Supporting companies | Apple, Ericsson, OPPO, Samsung (with modification) , Beijing Jiaotong University, LG Huawei/Hisi, Spreadtrum, CMCC, vivo, CAICT, Futurewei, Fujitsu, Xiaomi, NVIDIA, ZTE, MediaTek, Nokia/NSB, Lenovo, Qualcomm, AT&T, InterDigital, DCM |
| --- | --- |
| Objecting companies |  |

| Company | View |
| --- | --- |
| Moderator (replies to 1st round comments) | @CATT: to Moderator, it is up to companies to provide what kind of results (intermediate KPIs, or eventual KPIs), during the SI phase. For SLS, companies may also use intermediate KPIs if the results are convincing. @ Samsung: to Moderator, yes, the eventual performance is based on SLS, so we need to align SLS EVM with higher priority. For representative sub use case selection, it is encouraged to adopt SLS also (may be intermediate KPI can be used for simplicity) to better align companies and compare with R16/17 benchmark which are SLS based, but as said in the 1st bullet of the proposal, companies may optionally use LLS if the results are convincing. |
| Ericsson | The proposal is hard to read. I copy it here for readibility: Proposal 2.3.1: For the evaluation of the AI/ML based CSI feedback enhancement, system level simulation approach is adopted as baseline - Link level simulation is optionally adopted - [FFS: whether/how other simulation approach can be considered] I have a question what is the meaning of the brackets around an FFS? |
| OPPO | We agree with this proposal. |
| Samsung | Thank you FL for your consideration. We support this proposal in principle. We agree the eventual performance evaluation is better to be via SLS (not precluding LLS). However, for representative sub-use case selection, we may rely on simpler evaluations on the AI/ML models themselves. We support dataset generation via SLS or LLS, but the first-stage evaluation (until sub-use case selection) can be simplified by considering simpler metrics such as NMSE and GCS. To highlight that this proposal is for the eventual evaluation, the following modification is proposed from our side. Proposal 2.3.1: For the final phase of evaluation of the AI/ML based CSI feedback enhancement, system level simulation approach is adopted as baseline - Link level simulation is optionally adopted - For Phase I, i.e., representative sub-use case selection, simplified evaluation is adopted. - [FFS: whether/how other simulation approach can be considered] FFS: Details of simplified evaluation. |
| Beijing Jiaotong University | We agree with this proposal. |
| LG | We are fine with the modified proposal. One question is what are other simulation approaches other than LLS and SLS? Since there is no supporting company for other simulation approach, can we just delete FFS? |
| Huawei/Hisi | Support the two bullets, and for the bracket [FFS], suggest to remove it as long as there is no clear interpretation and corresponding evaluation results. |
| Moderator | @Samsung Generally we should not limit companies to only evaluate with ‘simplified evaluation’ even at a starting phase rather than a comprehensive one. Which option (‘simplified evaluation’/’ comprehensive evaluation’) or which KPI (intermediate/eventual) to adopt should be up to companies selection. @ Apple @OPPO @ Samsung @Beijing Jiaotong University please check if the updated version is good for you. |
| Ericsson | Support to remove the bracketed sentence. |
| Spreadtrum | We agree with the updated version. |
| CMCC | Support the updated proposal. |
| vivo | We are supportive of this proposal. |
| CATT | While we are OK that SLS should be evaluated, we have the same understanding with Samsung. Other than SLS and LLS, intermedia results (e.g. GCS, MMSE of resumed/estimated channel) can also be reported, at least optionally for fast cross check. In fact it is already parallel discussed in 2.3-4 (intermediate KPI). But the current proposal seems precluding such intermediate evaluation? We suggest the following update incorporating Samsung’s view: Proposal 2.3.1: For the evaluation of the AI/ML based CSI feedback enhancement, system level simulation approach is adopted as baseline - Link level simulation is optionally adopted - Simplified evaluation, e.g. intermediate evaluation on AI/ML output, is optionally adopted. |
| CAICT | Support the latest version. |
| Xiaomi | We think the modification from Samsung make sense. The intermediate KPI can be optional adopted for cross check. The CATT’s updated version is fine to us. |
| NVIDIA | Fine with the latest version. |
| ZTE | We agree with the proposal that system level simulation evaluation is adopted as baseline. |
| MediaTek | Support the proposal in principle. We prefer Samsung or CATT’s version. |
| Nokia/NSB | We support the latest version from the moderator. However, we agree that intermediate results can be reported, but the final performance evaluation is based on SLS results. |
| Lenovo | We agree with the updated version. |
| Intel | Not sure about the wording - same view as Samsung – also it is unclear what is the meaning of baseline and optional? |
| Qualcomm | We agree with the proposal. Intermediate KPIs should also be included in the metrics to evaluate. |
| AT&T | We agree with the current version of the proposal |
| InterDigital | We are ok with the proposal |

Baseline SLS EVM table-overall

For the baseline EVM, it looks most companies are fine/can accept the following table except for the FFS parts. So let’s see if we can support this proposal (Proposal 2.3.2 is editorially changed on top of Question 2.2.2), with a ‘FFS’ added for better clarifying in the text, an optional 30kHz for 4GHz in ‘SCS’, and an explanation added in ‘Overhead’ in the table

Proposal 2.3.2:Question 2.2.2: For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, do you agree the following table is taken as a baseline of EVM?

- Note: the following table captures the common parts of the R16 CSI enhancement EVM table and the R17 CSI enhancement EVM table, while the different parts are FFS.

- Note: the baseline EVM is used to compare the performance with the benchmark release, while the AI/ML related parameters (e.g., dataset construction, generalization verification, and AI/ML related metrics) can be of additional/different assumptions.

- FFS: modifications on top of the following table for the purpose of AI/ML related evaluations.

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD (TDD is not precluded), OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) is a baseline. Other scenarios (e.g. UMi@4GHz 2GHz, Urban Macro) are not precluded. |
| Frequency Range |  | FR1 only, FFS 2GHz or 4GHz as a baseline |
| Inter-BS distance |  | 200m |
| Channel model |  | FFS |
| Antenna setup and port layouts at gNB |  | Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ - 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ Other configurations are not precluded. |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for rank > 2 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) Other configuration is not precluded. |
| BS Tx power |  | 41 dBm for 10MHz, 44dBm for 20MHz, 47dBm for 40MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz, optionally 30kHz for 4GHz |
| Number of RBs |  | FFS |
| Simulation bandwidth |  | FFS |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | FFS |
| MIMO layers |  | For all evaluation, companies to provide the assumption on the maximum MU layers (e.g. 8 or 12) |
| CSI feedback |  | Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback) : 5 ms, - Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms |
| Overhead |  | Companies shall provide the downlink overhead assumption (i.e., whether the CSI-RS transmission is UE-specific or not and take that into account for overhead computation) |
| Traffic model |  | FTP model 1 with packet size 0.5 Mbytes as a baseline Other FTP model is not precluded. FFS full buffer |
| Traffic load (Resource utilization) |  | FFS |
| UE distribution |  | - 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic as a baseline FFS ideal channel estimation |
| Evaluation Metric |  | Throughput and CSI feedback overhead as baseline metrics. Additional metrics, e.g., ratio between throughput and CSI feedback overhead, can be used. Maximum overhead (payload size for CSI feedback)for each rank at one feedback instance is the baseline metric for CSI feedback overhead, and companies can provide other metrics. |
| Baseline for performance evaluation |  | FFS |

| Supporting companies | Apple, OPPO, Samsung, Beijing Jiaotong University, LG Huawei/Hisi, Spreadtrum, CMCC, vivo, CATT, CAICT, Futurewei, Fujitsu, Xiaomi, NVIDIA, ZTE, MediaTek, Nokia/NSB, Lenovo (with a slight change on note2), Qualcomm, AT&T, InterDigital, DCM |
| --- | --- |
| Objecting companies |  |

| Company | View |
| --- | --- |
| Moderator (replies to 1st round comments) | @ Lenovo @Samsung: Please see in the 2nd note, that ‘Note: the baseline EVM is used to compare the performance with the benchmark release, while the AI/ML related parameters (e.g., dataset construction, generalization verification, and AI/ML related metrics) can be of additional/different assumptions’. Generalization issues will be discussed in later sections. To make it clearer and self-explanatory, Moderator adds a FFS in the end (from ZTE’s suggestion). Hope that can address your concerns. @ Qualcomm: The feedback assumption is reused from R16/17 EVM, so the meaning is inherited. For the CSI-RS overhead issue, the explanation has been added in ‘Overhead’ entry. For SCS, optional 30kHz is now added. @CMCC @vivo for the channel estimation and traffic model issues, as some companies are not supporting even the optional parameter, Moderator is thinking to make it optional may be a middle ground. |
| OPPO | We agree with most parameters in this EVE table. However, we also suggest that 4RX antenna setup at UE side can be used for rank = 1 configuration. Whether 2RX and 4RX is adopted should be reported by companies. |
| Samsung | Thank you FL for clarification. We support this. |
| Beijing Jiaotong University | We agree with those parameters. |
| LG | Ok. One comment is SCS has dependency on carrier frequency, so maybe putting FFS on 30GHz is better. |
| Huawei/Hisi | Support. |
| Moderator | @LG Please consider that in ‘Frequency Range’ we only adopt FR1. |
| CMCC | Support the proposal in principle. Full buffer and ideal channel estimation can be optional. |
| vivo | We prefer to add InH in the scenario, and realistic/ideal in feedback assumption. |
| CATT | Agree with CMCC to optionally consider ideal channel estimation. Above all, we have concern for the SCS in 4GHz. we should remove ‘optionally’ for 30kHz. For 4GHz carrier frequency, SCS=30 kHz is commonly assumed and should be the baseline SCS. Companies can to check with their research team. |
| CAICT | Support this proposal. |
| Xiaomi | We agree with CMCC ideal channel estimation as an option. |
| NVIDIA | Support this proposal to move forward. |
| ZTE | We generally agree with the table as baseline. From our view, we also suggest that 4RX antenna configuration at UE side can be used for rank = 1 configuration as well. Additionally, ideal channel estimation can be adopted as baseline for calibration. |
| MediaTek | Support |
| Nokia/NSB | Support. In some cases, scenarios other than dense urban macro may be needed, for example for model generalisation testing, but the current formulation does not preclude this possibility |
| Lenovo | @Moderator: Thanks a lot for the clarification. We still have concerns on the proposal. To clarify, if only one simulation configuration is supported in the baseline EVM, the model could “overfit” to that configuration and can show good results compared to non-ML method, although it may not achieve the similar gains for other simulation configurations. Therefore, even for comparison with the benchmark release, we prefer to have an evaluation dataset which captures multiple simulation configurations. So we suggest to update Note2 as below: Note2: the baseline EVM is used to compare the performance with the benchmark release for one scenario/configuration. The conclusions for the use-case in the SI should only be drawn after comparing performance using EVM with multiple scenarios/configurations. The AI/ML related parameters (e.g., dataset construction, generalization verification, and AI/ML related metrics) can be of additional/different assumptions. |
| Intel | If we are doing time-domain prediction, it would be good to have scenario with all outdoors with different speeds. Full-buffer should be included as well because bursty result gains are much harder to reproduce because packet schedulers are very different in different companies |
| Qualcomm | Thank you for making the clarifications. We support the proposal. |
| AT&T | We agree with the current version of the proposal |
| InterDigital | Ok with the proposal |

For the R16/R17 difference entries, as per the suggestion from ZTE, these parameters are separated into two tables for R16 and R17 respectively, so companies can choose and report which table to use. Let’s see if this can be accepted or not. The R16 EVM and R17 EVM are pasted in the following table with changes on top of them. ‘Number of RBs’ entry is removed as per the suggestion of Ericsson. ‘Frequency range’ changing 2GHz as baseline as it operates for FDD mainly. 30kHz for 4GHz is added per the suggestion of QC. ‘MIMO scheme’ and ‘Traffic load’ changed as per suggestion from LG and Ericsson. ‘FFS’ added for the text same as Proposal 2.3.2.

Proposal 2.3.3: For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, the following parameters are taken into the baseline of EVM

- Note: The 2nd column applies if R16 TypeII codebook is selected as baseline, and the 3rd column applies if R17 TypeII codebook is selected as baseline.

- Note: the baseline EVM is used to compare the performance with the benchmark release, while the AI/ML related parameters (e.g., dataset construction, generalization verification, and AI/ML related metrics) can be of additional/different assumptions.

- FFS: modifications on top of the following table for the purpose of AI/ML related evaluations.

| Parameter | Value (if R16 as baseline) | Value (if R17 as baseline) |
| --- | --- | --- |
| Frequency Range | FR1 only, 2GHz as baseline, optional for 4GHz. | FR1 only, 2GHz with duplexing gap of 200MHz between DL and UL, optional for 4GHz |
| Channel model | According to TR 38.901 | According to TR 38.901 |
| Number of RBs |  |  |
| Simulation bandwidth | 10 MHz for 15kHz as a baseline, and configurations which emulate larger BW, e.g., same sub-band size as 40/100 MHz with 30kHz, may be optionally considered. Above 15kHz is replaced with 30kHz SCS for 4GHz. | 20 MHz for 15kHz as a baseline (optional for 10 MHz with 15KHz), and configurations which emulate larger BW, e.g., same sub-band size as 40/100 MHz with 30kHz, may be optionally considered. Above 15kHz is replaced with 30kHz SCS for 4GHz |
| MIMO scheme | SU/MU-MIMO with rank adaptation is a baseline for overhead reduction. For low RU, SU-MIMO or SU/MU-MIMO with rank adaptation are assumed for higher rank extension. For medium/high RU, SU/MU-MIMO with rank adaptation is assumed for higher rank extension. SU/MU-MIMO with rank adaptation. Companies are encouraged to report the SU/MU-MIMO with RU | For low RU, SU-MIMO with rank adaptation are assumed For medium/high RU, SU/MU-MIMO with rank adaptation is assumed SU/MU-MIMO with rank adaptation. Companies are encouraged to report the SU/MU-MIMO with RU |
| Traffic load (Resource utilization) | - 50/70 % for CSI overhead reduction - 20/50 % for high rank extension 20/50/70% Companies are encouraged to report the MU-MIMO utilization. | - 70% for SU/MU-MIMO with rank adaptation - 20% for SU-MIMO with rank adaptation 20/50/70% Companies are encouraged to report the MU-MIMO utilization. |

| Supporting companies | Apple, OPPO, Samsung, Beijing Jiaotong University, LG Huawei/Hisi, Spreadtrum, CMCC, vivo, CATT, CAICT, Futurewei, Fujitsu, Xiaomi, NVIDIA, ZTE, MediaTek, Nokia/NSB, Lenovo, Qualcomm, AT&T, InterDigital |
| --- | --- |
| Objecting companies |  |

| Company | View |
| --- | --- |
| OPPO | We agree with this proposal. |
| Samsung | We support in principle. |
| Beijing Jiaotong University | We agree with this proposal. |
| LG | We are generally fine with proposal, but it should be aligned with proposal 2.3.2, e.g., frequency range. |
| Huawei/Hisi | Support |
| CMCC | We support in principle. |
| vivo | We are supportive of this proposal. |
| CATT | Support. |
| CAICT | Support |
| NVIDIA | Support this proposal to move forward. |
| ZTE | We agree with this proposal. |
| MediaTek | Support |
| Nokia/NSB | We support in principle. It is our understood that companies may not (and most likely will not) be able to simulate all the combination cases in the EVM |
| Lenovo | We agree as the first scenario/configuration. |
| Qualcomm | We agree with this proposal. |
| AT&T | support |
| InterDigital | Ok with the proposal |

Baseline SLS EVM table-channel estimation

For the channel estimation, it seems different companies have different understandings, but let’s see if we can add a note to ensure the purpose of the ideal CE. In addition, as mentioned by NVIDIA, channel estimation applicable phase may be discussed as FFS.

(Proposal 2.3.4 is changed on top of Question 2.2.4)

Proposal 2.3.4: For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, for ‘Channel estimation’, whether ideal DL channel estimation is optionally taken into the baseline of EVM for the purpose of calibration?

- Note: Realistic DL channel estimation with error modeling should be used for performance comparison with the benchmark release and drawing SI conclusions.

- FFS: the ideal channel estimation is applied for dataset construction, or performance evaluation/inference.

| Supporting companies | OPPO, Samsung (with removing FFS), Beijing Jiaotong University, LG Huawei/Hisi, Spreadtrum, CMCC, vivo (with removing for the purpose of calibration), CATT, CAICT, Futurewei, Xiaomi, NVIDIA, ZTE, MediaTek, Nokia/NSB, Lenovo, Qualcomm (comments below), AT&T, InterDigital, DCM |
| --- | --- |
| Objecting companies |  |

| Company | View |
| --- | --- |
| OPPO | We think that ideal channel estimation can be added to the baseline. |
| Samsung | In order to start early evaluation on candidate sub-use cases, companies should agree on dataset generation as early as possible. That said, we propose to remove the FFS in the above proposal. Proposal 2.3.4: For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, for ‘Channel estimation’, whether ideal DL channel estimation is optionally taken into the baseline of EVM for the purpose of calibration? - Note: Realistic DL channel estimation with error modeling should be used for performance comparison with the benchmark release and drawing SI conclusions. - FFS: t The ideal channel estimation is applied for dataset construction, or performance evaluation/inference. |
| Beijing Jiaotong University | We agree with this proposal. But realistic DL channel estimation with error modeling should be considered for performance comparison. And Channel capacity (bit/s) with imperfect CSI could be considered as performance KPIs for evaluation for CSI compression. |
| Huawei/Hisi | Support. Realistic DL channel estimation should be used for final throughput evaluation. |
| CMCC | We agree with this proposal. Ideal channel estimation can be an optional baseline for the purpose of calibration. Realistic channel estimation can be adopted for performance comparison. |
| vivo | We believe that ideal DL channel estimation is not only used for calibration purpose, but also for performance evaluation purpose. Companies are encouraged to provide the evaluation results using both ideal and realistic channel estimation. |
| CAICT | Ideal channel estimation could be used as baseline. |
| Xiaomi | Ideal channel estimation should be considered to generate dataset for aligning simulation results. |
| NVIDIA | Support this proposal to move forward. |
| ZTE | We think ideal channel estimation should be taken as a baseline for ease of initial simulation calibration among companies. While in the evaluation phase, realistic channel estimation with error modeling should be adopted for performance comparison with the benchmark release. |
| MediaTek | Better to use ideal channel estimation for calibration. |
| Nokia/NSB | The input to the encoder will be based on an estimated channel so it would be best to include the effect of channel estimation when training. However, early results could be provided with ideal channel estimation. |
| Lenovo | We support. |
| Intel | Suggest ideal channel estimation for calibration |
| Qualcomm | We support the proposal and have the following comments. Comment on the wording: doesn’t “Realistic DL channel estimation” already imply “with error modeling”? If yes, then “with error modeling” could be removed to avoid confusion. If realistic channel estimation is to be modeled, then for intermediate KPI evaluation, it should be clarified whether the reference should be based on the ideal channel or the realistic estimated channel. We prefer to use a reference based on the ideal channel. |
| AT&T | Support ideal channel estimation as optional |
| InterDigital | Support the proposal. |

Baseline SLS EVM table-Traffic model

Similar to Proposal 2.3.4, a note is added to ensure the purpose.

(Proposal 2.3.5 is changed on top of Question 2.2.5)

Proposal 2.3.5 Question 2.2.5: For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, for ‘Traffic model’, whether full buffer traffic is optionally taken into the baseline of EVM for the purpose of calibration?

- Note: FTP traffic should be used for performance comparison with the benchmark release and drawing SI conclusions.

| Supporting companies | OPPO, Samsung, Beijing Jiaotong University, LG Huawei/Hisi, Spreadtrum, CMCC, vivo (with removing for the purpose of calibration), CATT, CAICT, Futurewei, Xiaomi, NVIDIA, MediaTek, Nokia/NSB, Lenovo, Qualcomm, AT&T, InterDigital |
| --- | --- |
| Objecting companies |  |

| Company | View |
| --- | --- |
| OPPO | We think that full buffer can be optionally taken into the baseline of EVM for calibration. |
| Samsung | Fine with the proposal. |
| Beijing Jiaotong University | We agree with this proposal. |
| Huawei/Hisi | Support. FTP model 1 should be used for final throughput evaluation. |
| CMCC | We agree with this proposal. Full buffer can be an optional baseline, at least for the purpose of calibration. |
| vivo | We believe that full buffer traffic is not only used for calibration purpose, but also for performance evaluation purpose. Companies are encouraged to provide the evaluation results using both FTP and full buffer traffics. |
| Xiaomi | We support using full buffer as optional baseline for calibration at least for intermediate metric, e.g., GCS comparison. |
| NVIDIA | Support this proposal to move forward. |
| ZTE | We think we can continue to use FTP traffic model for performance evaluation, which is closer to real environment. Note that other FTP models can also be good options for performance evaluation. Additionally, full buffer traffic may be optionally used for calibration purpose. |
| MediaTek | Support |
| Nokia/NSB | We agree with full buffer being optional |
| Lenovo | We support the proposal. |
| Intel | agree with Vivo, suggest full-buffer for calibration and evaluation |
| Qualcomm | We agree with this proposal. |
| AT&T | Support the proposal on calibration and agree with Vivo and intel that full buffer is used for evaluation as well |
| InterDigital | Support the proposal |

### 2.3-2: Source to generate the dataset

Based on the discussions of the 1st round, it seems Question 2.2.8 is clear and stable, so Moderator makes editorial changes, and to wait for discussion at Fri. GTW. No discussion needed in this round.

Proposal 2.3.6 Question 2.2.8: For the evaluation of the AI/ML based CSI feedback enhancement, do you agree that the dataset for AI/ML training/validation/testing is generated by TR 38.901 channel models as a starting point?

- FFS whether/how to generate the dataset from field test.

### 2.3-3: Calibration of dataset and/or AI/ML model

Based on the 1st round discussion of Question 2.2.9, Option 1 is supported by most companies, and there seems to objection received. For Option 2-4, there are also some companies supporting, while at least one company objecting. So Moderator tries to first take Option 1 as a starting point (as it is anyhow needed even for Option2-4), and trigger the detailed solutions for carrying out Option 2-4 (if supported) in the next round.

Proposal 2.3.7: For the evaluation of the AI/ML based CSI feedback enhancement, if the dataset for AI/ML training/validation/testing is generated by TR 38.901 channel models, for the calibration purpose on the dataset and/or AI/ML model over companies, consider to align the parameters (e.g., for scenarios/channels) for generating the dataset in the simulation as a starting point.

| Supporting companies | Apple, Ericsson, OPPO, Samsung, Beijing Jiaotong University, LG Huawei/Hisi, Spreadtrum, CMCC, vivo, CATT, CAICT, Futurewei, Fujitsu, Xiaomi, NVIDA, ZTE, MediaTek, Nokia/NSB, Lenovo, Qualcomm, AT&T, InterDigital, DCM |
| --- | --- |
| Objecting companies |  |

| Company | View |
| --- | --- |
| OPPO | We agree with that the parameters for generating dataset should be aligned. |
| Samsung | Support the proposal. |
| Beijing Jiaotong University | We agree with this proposal. |
| Huawei/Hisi | Support. The baseline EVM table can provide a well guideline for aligning the parameters. |
| CMCC | We agree with this proposal. |
| vivo | We are supportive of this proposal. |
| CAICT | Support |
| Xiaomi | Even the parameters are aligned, the channel data may different between companies. We think a common data set generated by 38.901 is more suitable for calibration purpose. |
| NVIDIA | Support this proposal to move forward. |
| ZTE | We agree with the proposal of aligning the parameters for generating the dataset. |
| MediaTek | Support |
| Lenovo | Looks good to us. |
| Intel | Agree with Xiaomi, channel generation varies widely from company to company |
| Qualcomm | We agree with the proposal. |
| AT&T | Support the proposal |
| InterDigital | Support the proposal |

### 2.3-4: Metrics

Intermediate KPIs

It looks most companies prefer Option 1 or Option 2 or both, and the preference between Option 1 and Option 2 may depends on the specific sub use case/input channel type. But still some companies have a different view on the intermediate KPI. Moderator tries to take both Option 1 and Option 2 as a starting point, while leave FFS for other methods.

Proposal 2.3.8: For the evaluation of the AI/ML based CSI feedback enhancement, as a starting point, take the intermediate KPIs of GCS/SGCS and NMSE as part of the ‘Evaluation Metric’ to compare the similarity/error between the input original CSI of the AI/ML model and the output CSI (e.g., recovered/predicted CSI) of the AI/ML model

- For GCS/SGCS,

  - FFS: how to calculate GCS/SGCS for rank>1

  - FFS: whether GCS or SGCS is adopted

- FFS other metrics, e.g., equivalent MSE, or received SNR.

| Supporting companies | Apple, OPPO, Samsung, Beijing Jiaotong University, LG Huawei/Hisi, Spreadtrum, CMCC, vivo, CATT, CAICT, Futurewei, Fujitsu, Xiaomi, NVIDIA, ZTE, MediaTek, Nokia/NSB, Lenovo, InterDigital, DCM |
| --- | --- |
| Objecting companies | Ericsson, Qualcomm |

| Company | View |
| --- | --- |
| Moderator (replies to 1st round comments) | @Ericsson: to Moderator, the received SNR is somehow eventual performance which is related also with precoding methods, MU paring methods, so how it can directly reflect the AI/ML model performance if the paring/precoding is not part of the AI/ML model? More elaborations are appreciated. @Qualcomm: can you elaborate how equivalent MSE=2-2*GCS is different with GCS? To moderator it is calculated based on a linear formula from GCS? @ Lenovo Your option 4 is still kind of GCS (of multiple ranks), right? |
| Ericsson | No need to make an agreement before we have developed a metric that works for rank>1 and that is relevant for MU-MIMO operation in a massive MIMO system where channels are NLOS. Looking at rank=1 metric doesn’t make for MIMO use case (but maybe good enough in academic papers?). We risk starting off in a direction in this SI that doesn’t give any benefit in SLS later on. We need to resolve the issues with CGS and NMSE pointed out in our tdoc: - Problems with loss functions using cosine similarity and generalized cosine similarity - They are not defined multi-layer transmissions (i.e., rank > 1). - They do not map well to the objective of maximizing downlink throughput. - For example, imagine a single-layer transmission over a channel with two distinct main clusters for which the corresponding precoding vectors are approximately orthogonal (i.e., the strongest and second strongest eigenvectors). If the pathloss of both clusters is approximately the same, then in SU-MIMO it does not matter which eigenvector is used. However, the cosine similarity of the reconstructed precoding vector with respect to the true strongest eigenvector will heavily (and unnecessarily) punish selecting the second strongest eigenvector as the precoding vector. - Problems with loss functions using squared error and normalized squared error These per-sample loss functions can require the UE AI and NW AI to compress and reconstruct aspects of the channel that are irrelevant for SU or MU-MIMO performance, such as the channel subspace represented by the weakest eigenvector. Using such loss functions may thus lead to unnecessary large uplink overhead as the CSI contains information that is not useful for the scheduler. |
| OPPO | From our view, we prefer to use SGCS as the baseline. The SGCS for rank>1 can be FFS. However, whether other metrics such as MSE or received SNR is necessary can be further discussed. |
| Samsung | Support. For first FFS, Lenevo’s proposal in Round 1 for multi-rank GCS is a good starting point. |
| Beijing Jiaotong University | In our view, both GCS and SGCS can be considered for performance measurement and comparison, they have no difference. And other metrics such as MSE ,received SNR or system capcity is necessary can be discussed. |
| Huawei/Hisi | Support. The GCS is the most related metric to the accuracy of the recovered eigenvector, which is the performance of AI model. The calculation of GCS for rank1 is clear. We only need to discuss and align on how to calculate GCS for higher rank, which is easier than to adopt a brand new metric. |
| CMCC | Support. We prefer SGCS as the intermediate KPI. For the SGCS for Rank >1, it can be further studied. |
| vivo | We are supportive this proposal. |
| CAICT | We prefer SGCS as starting point. |
| NVIDIA | Support this proposal to move forward. |
| ZTE | We agree with this proposal. From our view, SGCS can be adopted as baseline for comparing the similarity among companies, because SGCS can improve the resolution of performance gap for clear comparison. In addition, we think rank=1 can be the prior option for companies to evaluate intermediate performance, which is easier for companies to align and calibrate. For GCS/SGCS for rank>1, it also needs an aligned calculation approach to evaluating GCS/SGCS performance to lay a foundation for SLS. |
| MediaTek | GCS or SGCS is enough as an intermediate KPI |
| Nokia/NSB | We prefer to avoid using the term “original CSI” as it is not well defined and use instead input and output Proposal 2.3.8: For the evaluation of the AI/ML based CSI feedback enhancement, as a starting point, take the intermediate KPIs of GCS/SGCS and NMSE as part of the ‘Evaluation Metric’ to compare the similarity/error between the input of the AI/ML model and the output (e.g., recovered/predicted CSI) of the AI/ML model - For GCS/SGCS, - FFS: how to calculate GCS/SGCS for rank>1 - FFS: whether GCS or SGCS is adopted FFS other metrics, e.g., equivalent MSE, or received SNR. |
| Lenovo | Yes, indeed it is based on GCS which consider both rank 1 and higher ranks. |
| Qualcomm | It seems that our previous comment has not been correctly captured. So, we would like to reemphasize it: The reference to compute the similarity metric should not be ‘input original CSI’. Instead, the similarity metric should be defined between the recovered output CSI and the target output CSI. This is related to the wording change we had proposed in Proposal 6.1.3 (now 3.3.1). The reason is that the target output CSI may be different from the input or the AI/ML model. For example, the encoder input could be the raw channel matrix and the decoder output could be the eigen vector of the channel. What input to use – whether/how to perform SVD on the channel H to derive an eigenvector v, or to directly feed H into an encoder, is up to company’s decision. The group only needs to agree on the target output and define a metric between the ideal target output and the decoder output. If realistic channel estimation is modeled, then the target output should be based on the ideal channel. Regarding equivalent MSE vs. GCS, we agree they are linearly related. The benefit of using the equivalent MSE version is it gives a unified way to compare the case of raw channel target output and eigenvector target output using the same type of metric. |

As per Moderator’s view, between GCS and SGCS, we may choose one of them as an aligned metric to avoid comparing based on different basis, so let’s see if we can align on any one of them.

Question 2.3.1: If you prefer Option 1: GCS/SGCS as the metric, which of GCS and SGCS do you prefer as the metric?

- Option 1: GCS

- Option 2: SGCS

- Note: wi is the input original CSI vector of frequency unit i, and wi is the output CSI vector of frequency unit i. N is the total number of frequency units. E{∙} denotes the average operation over multiple samples.

GCS=E1Ni=1N||wiHwi ||||wi||  ||wi||,  SGCS=E1Ni=1N||wiHwi ||||wi||  ||wi||2

| Option 1 Supporting companies | Apple, LG Huawei/Hisi, Spreadtrum, CATT, Futurewei, Fujitsu, Xiaomi, NVIDIA, MediaTek, Lenovo, InterDigital |
| --- | --- |
| Option 2 Supporting companies | OPPO, , Beijing Jiaotong University , CMCC, vivo, CAICT, Xiaomi, NVIDIA, ZTE, MediaTek, Qualcomm, InterDigital |
| Option 3 Supporting companies |  |

| Company | View |
| --- | --- |
| Samsung | Is option 3 both? |
| Beijing Jiaotong University | Option 2 is preferred |
| Huawei/Hisi | Either option 1 and 2 is fine for us. We slightly prefer option 1. |
| CMCC | We prefer SGCS as the intermediate KPI. |
| Moderator | @Samsung Wrongly copy-paste. Now crossed out. |
| NVIDIA | Either option is fine. |
| ZTE | We prefer SGCS to be a baseline, because when the GCS performance is so good that GCS gap becomes so small that it is hard to tell the difference between AI models. However, SGCS can improve the resolution of performance gap for clear comparison. |
| MediaTek | Both options are fine. |
| Nokia/NSB | Either is fine |
| Lenovo | We prefer Option1. Option2 and other variants can be FFS. |
| Qualcomm | Either option is fine. We slightly prefer option 2. |
| InterDigital | Either option should be fine |

Lenovo raises that for rank>1, and if GCS is selected as the metric, we need to consider how to express the GCS value/values.

Question 2.3.2: If you prefer Option 1: GCS/SGCS as the metric, how to evaluate the rank>1 cases by using GCS/SGCS (take GCS for example)?

- Option 1: Average over all ranks

  - Note: wij is the jtheigenvector at frequency unit i and K is the rank of the channel

GCS=E1Ni=1NjK||wijHwij ||||wij||  ||wij||

- Option 2: Weighted average over all ranks

  - Note: wijand λij are the jtheigenvector and its associated eigenvalue at frequency unit i and K is the rank of the channel

GCS=E1Ni=1NjKλij||wijHwij ||||wij||  ||wij||

- Option 3: GCS/SGCS is separately calculated for each rank (e.g., for K ranks, K GCS values are derived)

- Option 4: Other

| Option 1 Supporting companies | Apple, OPPO, Beijing Jiaotong University, LG Huawei/Hisi, Spreadtrum, CMCC, vivo, CATT, Futurewei (ok as well) , Xiaomi, NVIDIA, ZTE, MediaTek, Qualcomm |
| --- | --- |
| Option 2 Supporting companies | Apple, OPPO, Samsung (see the extension for SGCS) , Beijing Jiaotong University, Spreadtrum, CMCC, CATT, Futurewei (preferred) , Fujitsu (the GCS formula should be updated) , Xiaomi, NVIDIA, MediaTek, Lenovo, Qualcomm |
| Option 3 Supporting companies | OPPO, Beijing Jiaotong University Huawei/Hisi, Spreadtrum, CMCC, CATT, Futurewei (ok as well) , Xiaomi, NVIDIA, Qualcomm |
| Option 4 Supporting companies |  |

| Company | View |
| --- | --- |
| OPPO | We think that both Option 1, Option 2 and Option 3 can be considered for FFS. |
| Samsung | Support the proposal with the following extension for SGCS - Option 2: For GCS, weighted average over all ranks - Note: wijand λij are the jtheigenvector and its associated eigenvalue at frequency unit i and K is the rank of the channel GCS=E1Ni=1NjKλij¦¦wijHwij ¦¦¦¦wij¦¦ ¦¦wij¦¦ For SGCS, weighted average over all ranks - Note: wijand λij are the jtheigenvector and its associated eigenvalue at frequency unit i and K is the rank of the channel |
| Beijing Jiaotong University | Either option 1-3 seems fine. |
| Huawei/Hisi | For Option 3, the performance between legacy and R18 AI/ML can be per rank compared. Linear combination or weighted combination may not straightforwardly reflect the per rank accuracy, e.g. part of rank(s) is severely degraded, but other rank(s) is quite high, the average GCS may still high. But we can also accept Option 1 as a 2nd preference, considering its simplicity. |
| CMCC | Option 1~3 can be further studied. |
| CATT | Either would be fine. All of them can represent the performance difference. If there is no consensus, it can be left to companies report. |
| CAICT | We think all options are fine. |
| Futurewei | We prefer the weighted average while options 1 (simpler) and 3 are also ok. |
| Fujitsu | Our view is that the GCS should depend on the specific eigenvalues in the multi-rank scenario. For this reason, we prefer Option 2 to Option 1. However, the GCS formular in Option 2 needs to be revised for the following reason: - Normalization is needed to restrict the range of the GCS to be within [0,1]. We propose the following GCS formulae. One option is GCS=E1Ni=1NjKλij2j=1Kλij2¦¦wijHwij ¦¦¦¦wij¦¦ ¦¦wij¦¦. Another option is GCS=E1Ni=1NjKλijj=1Kλij¦¦wijHwij ¦¦¦¦wij¦¦ ¦¦wij¦¦. |
| Xiaomi | These options can be further studied and evaluated. |
| NVIDIA | We support to report all these statistics to provide full information, given any of the three is not more difficult to compute than the other two. |
| ZTE | We agree with the evaluation of rank>1 cases, and we initially prefer Option 1 as GCS/SGCS baseline. We think Option 1 considers inter-layer interference and emphasizes the fairness of reconstruction at all layers. However, Option 2 is better to protect the accuracy of strongest layer, which may mask the poor recovered performance of other layers. While Option 3 may not jointly consider the influence between layers. |
| MediaTek | Prefer option 1 and 2 |
| Lenovo | Option 1 probably is not a good metric since it assumes same importance between all eigenvectors which is not correct. For option 3, we are unclear how GCS/SGCS vectors (value for each rank) will be compared between two models. @ Fujitsu: We assume that averaged power of the channel realizations are normalized. By dividing with the sum of eigenvalues each instantaneous channel will be normalized which is not preferred. |
| Intel | It seems that one issue with option-1 above is that it is not a subspace metrics (or rotation invariant) meaning that if we use [w1 w2] vs Q[w1 w2] where Q is unitary and w1, w2 are layer-1, layer-2 precoding vectors, we will get not get the same results – Option-2 may be more relevant for explicit feedback |
| Qualcomm | All options 1 – 3 can be considered. |

Capability/complexity related KPIs

For the processing complexity KPI, based on the discussions of the 1st round, it seems Question 2.2.11 is clear and stable, so Moderator makes editorial changes, and to wait for discussion at Fri. GTW. No discussion needed in this round.

Proposal 2.3.9 Question 2.2.11: For the evaluation of the AI/ML based CSI feedback enhancement, do you agree that Floating point operations (FLOPs) is should be adopted as part of the ‘Evaluation Metric’, and reported by companies?

For the Memory storage KPI, it looks the preference between Option 1 and Option 2 is half-half. So let’s see if we may support both based on report of companies. As to the format of the AI/ML parameters (e.g., real or complex valued), proponent companies may provide views.

Proposal 2.3.10: For the evaluation of the AI/ML based CSI feedback enhancement, AI/ML memory storage in terms of AI/ML model size and number of AI/ML parameters is adopted as part of the ‘Evaluation Metric’, and reported by companies.

- FFS: the format of the AI/ML parameters

| Supporting companies | Apple, OPPO, Samsung, Beijing Jiaotong University, LG Huawei/Hisi, Spreadtrum, CMCC, vivo, CATT, CAICT, Futurewei, Fujitsu, Xiaomi, NVIDIA, ZTE, MediaTek, Nokia/NSB, Lenovo, InterDigital |
| --- | --- |
| Objecting companies |  |

| Company | View |
| --- | --- |
| OPPO | We agree with this proposal. Relative information should be provided when a company propose an AI/ML algorithm. |
| Beijing Jiaotong University | We agree with this proposal. |
| Huawei/Hisi | Support. |
| CMCC | We agree with the proposal. |
| Futurewei | Using either one is ok, but we prefer not requiring both. |
| Fujitsu | We support to use model file size as the storage KPI since it is straightforward and easy to understand, it is necessary to avoid potential confusion introduced by ambiguous expressions such as “Number of AI/ML parameters” which need extra workload to first clarify related concepts. |
| NVIDIA | Support this proposal to move forward. |
| ZTE | We generally agree with this proposal. We think AI/ML model size or number of AI/ML parameters can be used for evaluation initially. However, it should be clarified for companies how AI/ML model size or number of AI/ML parameters is calculated because different model formats are provided. |
| Lenovo | Support |
| Qualcomm | We support using the number of AI/ML parameters. From the current wording, it is not clear if both need to be reported, or if it is left to companies to choose one. We do not support AI/ML model size since it is dependent on the model format and implementation aspects. |

Besides, as raised by Xiaomi, power consumption should be considered as a complexity. Let’s see the views from companies.

Question 2.3.3: For the evaluation of the AI/ML based CSI feedback enhancement, whether/how the power consumption is adopted as part of the ‘Evaluation Metric’, and reported by companies?

| Company | View |
| --- | --- |
| Apple | Highly implementation dependent value. |
| OPPO | We are not very clear how to evaluate the power consumption. And it seems relys on the hardware implementation. |
| Samsung | Power consumption is in a way related to the number of parameters or floating-point operations FLOPs in the AI/ML model. Companies may study, if power consumption can be indirectly inferred by reporting based on Proposal 2.3.2. |
| Beijing Jiaotong University | Power consumption is related to many factors, and the hardware also needs to be aligned, which is difficult to use as a metric. |
| LG | Agree with Apple, it depends on the implementation. |
| Huawei/Hisi | No need to take it as metric at the moment since it quite depends on specific hardware/firmware, but can keep studying. |
| Spreadtrum | We also think no need to take power consumption as a metric now. If other companies have reasonable methods to evaluate it, we can also accept it. |
| CMCC | It seems power consumption is related to many factors and hardware implementation. Not easy to evaluate power consumption. |
| vivo | Indeed, it is hard to measure the power consumption exactly and the power consumption level can equivalently be represented by AI/ML model size or the number of AI/ML parameters. Therefore, there is no need to adopt an extra metric for power consumption. |
| CATT | We are OK to consider, but if power assumption is adopted as part of ‘Evaluation Metric’, more detail on how to model and represent power consumption is needed. This may require more work. |
| CAICT | We agree with Huawei’s view. |
| Futurewei | We do not see the need to have UE power consumption as a KPI here. And it requires a lot of extra effort and more discussions across companies. |
| Fujitsu | We do not support that power consumption is adopted as part of the evaluation metric. The reason is that it is difficult to be used as a measure for a simulation-based performance assessment (highly depends on GPU/CPU for the simulation). |
| Xiaomi | Power consumption is important for terminal or chip vendor. For the same AI/ML model, the power consumption may be different for different terminals. Some terminals cannot adopt the AI/ML model due to unafforded power consumption. Therefore, power consumption should be further studied although it depends on many factors. |
| ZTE | We think power consumption is dependent on implementation, and we are not clear how to evaluate it, which may have a lower priority. |
| MediaTek | We believe power consumption is important but for evaluation stage, we don’t need to discuss it since it is hard to evaluate. |
| Nokia/NSB | We don’t think this needs to be adopted in the EVM as it heavily depends on specific hardware choices. Model size and FLOP count should be enough as a starting point. |
| Lenovo | Power consumption evaluation is more important for the model (or part of the model) running at the UE (as it operates on battery power). It can be considered as a KPI even for gNB but with lower priority. Another complexity is that measuring power consumption very much depended on the actual hardware that the AI/ML model running on, and usually should be performed in a controlled environment. These complexities also exist for some other metrics like inference latency that was discussed during the GTW. There could be some possible work around for that as: - Try to execute two models on a same device and environment so the power consumption values can be comparable (the actual values still depends on the HW setup but we can compare the two models as they have been executed on same setting). - As the power consumption is related to computational complexity, we may use parameters like number of operations needed, the frequency that the model should complete an inference operation. We believe this is the more straight forward method. |
| Qualcomm | It is implementation dependent and difficult to evaluate. |
| InterDigital | Tend to agree with other companies that it is difficult to evaluate as we don’t even have the power consumption model for this case. |

### 2.2-5: Methodology for generalization

The views from companies are a bit diverse. This issue will be discussed in the next round.

3rd round email approval

To reduce the work load and save time for GTW discussions, some relatively stable proposals (No objection received from the 2nd round email discussions and delivered for May 13 GTW) are provided in this section for email approval.

### 2.4-1: Generic evaluation methodology: Baseline SLS EVM table-overall

For the baseline EVM, following changes are made on top of Proposal 6.2.2.

- For ‘Antenna setup and port layouts at UE’, as commented by OPPO, ZTE, and Ericsson that rank=1 should not be precluded (rank value should be freely scheduled by gNB), the ‘rank=1-4’ has been added accordingly for 4Rx.

- For ‘UE distribution’, other indoor/outdoor UE distributions and other UE speeds may be needed for the sub use case of CSI prediction in time domain as per comments from Intel and Ericsson. But as there are a couple of companies in Sec.4 have concerns on the study priority of CSI prediction, a ‘FFS whether/what other indoor/outdoor distribution and/or UE speeds for outdoor UEs needed’ is added.

- For ‘SCS’, the ‘optional 30kHz SCS for 4GHz’ is replaced with a more open ‘15kHz for 2GHz, 30kHz for 4GHz’ as per the comments from CATT and Ericsson.

So, the new proposal is updated on top of Proposal 6.2.2 with the changes above captured:

Proposal 2.4.1 Proposal 6.2.2: For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, the following table is taken as a baseline of EVM

- Note: the following table captures the common parts of the R16 CSI enhancement EVM table and the R17 CSI enhancement EVM table, while the different parts are FFS.

- Note: the baseline EVM is used to compare the performance with the benchmark release, while the AI/ML related parameters (e.g., dataset construction, generalization verification, and AI/ML related metrics) can be of additional/different assumptions.

  - The conclusions for the use cases in the SI should be drawn based on generalization verification over potentially multiple scenarios/configurations.

- FFS: modifications on top of the following table for the purpose of AI/ML related evaluations.

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD (TDD is not precluded), OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) is a baseline. Other scenarios (e.g. UMi@4GHz 2GHz, Urban Macro) are not precluded. |
| Frequency Range |  | FR1 only, FFS 2GHz or 4GHz as a baseline |
| Inter-BS distance |  | 200m |
| Channel model |  | FFS According to TR 38.901 |
| Antenna setup and port layouts at gNB |  | Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ - 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ Other configurations are not precluded. |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for rank > 2 (rank 1-4) 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) Other configuration is not precluded. |
| BS Tx power |  | 41 dBm for 10MHz, 44dBm for 20MHz, 47dBm for 40MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz for 2GHz, 30kHz for 4GHz, optionally 30kHz for 4GHz |
| Number of RBs |  | FFS |
| Simulation bandwidth |  | FFS |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | FFS |
| MIMO layers |  | For all evaluation, companies to provide the assumption on the maximum MU layers (e.g. 8 or 12) |
| CSI feedback |  | Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback) : 5 ms, - Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms |
| Overhead |  | Companies shall provide the downlink overhead assumption (i.e., whether the CSI-RS transmission is UE-specific or not and take that into account for overhead computation) |
| Traffic model |  | FTP model 1 with packet size 0.5 Mbytes as a baseline Other FTP model is not precluded. FFS full buffer |
| Traffic load (Resource utilization) |  | FFS |
| UE distribution |  | - 80% indoor (3km/h), 20% outdoor (30km/h) FFS whether/what other indoor/outdoor distribution and/or UE speeds for outdoor UEs needed |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic as a baseline FFS ideal channel estimation |
| Evaluation Metric |  | Throughput and CSI feedback overhead as baseline metrics. Additional metrics, e.g., ratio between throughput and CSI feedback overhead, can be used. Maximum overhead (payload size for CSI feedback)for each rank at one feedback instance is the baseline metric for CSI feedback overhead, and companies can provide other metrics. |
| Baseline for performance evaluation |  | FFS |

| Supporting companies | Apple, vivo (with comments), LG, Ericsson, CAICT, ZTE, Nokia/NSB, mediatek, Futurewei, NVIDIA, DCM, Samsung, OPPO, InterDigital, CATT, Fujitsu, Xiaomi, Huawei/Hisi, Qualcomm (one comment below), Spreadtrum, Lenovo, CMCC |
| --- | --- |
| Objecting companies |  |

| Company | View |
| --- | --- |
| Moderator | @Companies please check if the changed parts in the EVM table is OK for you. Note: If no objection received till the email approval deadline of UTC 8:00 May 17, this proposal will be delivered for email approval by Mr. Chair. |
| vivo | The agreement should be made after the decision of section 2.5. For feedback Assumption: Whether we need to define the mechanism to generate the error rate on the feedback channel? At least, there are two alternatives to model such an error. Alt-1 is to simply set an error as a hyper-parameter, say, 1%. It is worthwhile noting that 1% is a reasonable value for control channel experienced by UCI. Alt-2 is to model an UL link, in addition to the DL (assuming DL is used to generate the channel for CSI feedback). Based on the SINR received from the UL, a BLER on UCI can be calculated. If an error occurs in UCI reception, moreover, a HARQ process will be taken on the DL and UL. Alt-1 is simple, but with low accuracy. We believe, 1% error rate if setting perhaps may not affect the final system performance. A similar consequence may be made if we simply assume an ideal feedback channel. Alt-2, on the contrast, ensures a much high accuracy, but burns a lot of time to complete the system level simulation. If the agreement goes to Alt-1, we may need to change the wording as, “CSI-feedback with error”. Otherwise, the original wording “realistic” is fine to us. For simplicity, nevertheless, we are supportive of either ideal CSI feedback assumption or Alt-1, other than Alt-2. For Channel Estimation: If we assume “Channel Estimation” to be realistic, at least, there are two alternatives to model such an error. Alt-1 is to directly generate the DL channel response matrix with an error, i.e., H’=H+e(SINR), where e(SINR) is the error term as a function of SINR received from the DL. Alt-2 is to estimate the channel based on the estimation algorithm, either MMSE or ZF. For simplicity, nevertheless, we are supportive of Alt-1. For Traffic Model: FTP and full buffer traffic models are idiomatically utilized in the pass EVM, e.g., in TR 36.897. Both should be supported in the Rel-18 EVM. FTP traffic model can offer the evaluated performance which is close to what a realistic system can achieve, while full buffer traffic model can demonstrate how far the throughput performance the AI-based CSI-feedback mechanism can go. If we only limit the traffic model to be FTP, we are never aware of the performance ceiling the AI-based CSI-feedback mechanism can approach. |
| LG | For more progress, as agreed in framework section in the last week, TR 38.901 can be used rather than FFS in channel model. |
| Ericsson | To vivo: Note that the non full buffer doesn’t asymptotically approach the full buffer results when resource utilization increases since the system reaches instability at high RU. The full buffer results is some other value that has no real interpretation in real life. (Recall the discussion on this topic at the RAN1 meeting in Malmö, Sweden) Agree with LG on the use of 38.901 |
| CAICT | Ideal feedback channel could also be considered. |
| ZTE | We agree with the table as baseline. Initially, ideal channel estimation can be also adopted as baseline for calibration. |
| Nokia/NSB | - Channel model. We also prefer to agree on the channel model of TR 38.901. In fact, these EVM assumptions were originally agreed in RAN1#94bis as additional updates on the existing EVM of Table A.2.1-1 of TR38.802 which uses 38.901 channel model - Feedback assumption. In our understanding, “realistic” simply means that for DL transmission SLS we do not assume ideal channel knowledge at the gNB. This is the same assumption used in Table A.2.1-1 of TR38.802. We don’t see any need to model the decoding error on PUSCH for the purpose of this study. |
| Moderator | Note: for the ‘Channel model’ entry, ‘According to TR 38.901’ is added according to the 9.2.1 agreement. @vivo The detailed channel estimation methods should be a further issue after the realistic channel estimation rule endorsed, right? We are not supposed to wait for all details determined/clarified before endorsing on a quite high level agreement. |
| Intel | We prefer to include full-buffer traffic, the reason is that FB schedulers are much more standard than bursty. The gains due to CSI becomes very much dependent on the FTP traffic scheduler and it will be hard to have comparable results (especially at low load). We have also had both FB and FTP in RAN1 in the past with no problems. Pls. see section 7.2 in TR36.897 how gains vary in bursty case (one example here :-)): |
| Samsung | We support the proposed Table. In our view, these can be decided early - Frequency Range: FR1 only, 2GHz baseline 4GHz optional - UE distribution: Option 1: 80% indoor (3km/h), 20% outdoor (30km/h) Option 2: 100% outdoor (variable speed, e.g., x1% 10km/h, x2% 20km/h and x3% 30km/h to be reported by companies) |
| OPPO | We agree with most of parameters in this EVE table. Moreover, we also support to consider SCS 15KHz for 4GHz in numerology. |
| CATT | We are generally fine with the EVM. But for evaluation metric, we prefer to remove one of the example of additional metrics, i.e. “e.g., ratio between throughput and CSI feedback overhead”. This is not a valid metric, which is rarely (or never?) used. It does not perform additional information than the conventional ‘Throughput vs. CSI feedback overhead’ curve. |
| Qualcomm | For the traffic model, FTP model 3 should also be included as a baseline case. |
| CMCC | We agree with this table. Ideal channel estimation can be used as a baseline at least for calibration. |
| Moderator | @ CATT the ‘example’ is just an optional metric, so I guess leave it there does no really impact the mandatory throughput/overhead metric, but just keeps more flexibility for companies as an optional choice. @Qualcomm ‘Other FTP model is not precluded’ that means FTP 3 can still be chosen by companies. @ OPPO But isn’t 4GHz TDD spectrum with 30kHz widely commercialized? @ Intel as the door is still open for full buffer as discussed in Proposal 2.5.2, so if that proposal is agreed, companies can still use full buffer for initial evaluations. Would that be acceptable for you? |

### 2.4-2: Generic evaluation methodology: Baseline SLS EVM table-FFS part

The FFS parts of the overall baseline EVM table seem to be ok in general except one comment from Ericsson on the ‘frequency range’ entry. But it is still not fully convincing to Moderator to reverse the baseline/optional between 2GHz/4GHz, so the original Proposal 6.2.3 is provided in below.

Proposal 2.4.2 Proposal 6.2.3: For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, the following parameters are taken into the baseline of EVM

- Note: The 2nd column applies if R16 TypeII codebook is selected as baseline, and the 3rd column applies if R17 TypeII codebook is selected as baseline.

  - FFS baseline release for potential sub use cases involving CSI enhancement on time domain

- Note: the baseline EVM is used to compare the performance with the benchmark release, while the AI/ML related parameters (e.g., dataset construction, generalization verification, and AI/ML related metrics) can be of additional/different assumptions.

  - The conclusions for the use cases in the SI should be drawn based on generalization verification over potentially multiple scenarios/configurations.

- FFS: modifications on top of the following table for the purpose of AI/ML related evaluations.

| Parameter | Value (if R16 as baseline) | Value (if R17 as baseline) |
| --- | --- | --- |
| Frequency Range | FR1 only, 2GHz as baseline, optional for 4GHz. | FR1 only, 2GHz with duplexing gap of 200MHz between DL and UL, optional for 4GHz |
| Channel model | According to TR 38.901 | According to TR 38.901 |
| Number of RBs |  |  |
| Simulation bandwidth | 10 MHz for 15kHz as a baseline, and configurations which emulate larger BW, e.g., same sub-band size as 40/100 MHz with 30kHz, may be optionally considered. Above 15kHz is replaced with 30kHz SCS for 4GHz. | 20 MHz for 15kHz as a baseline (optional for 10 MHz with 15KHz), and configurations which emulate larger BW, e.g., same sub-band size as 40/100 MHz with 30kHz, may be optionally considered. Above 15kHz is replaced with 30kHz SCS for 4GHz |
| MIMO scheme | SU/MU-MIMO with rank adaptation is a baseline for overhead reduction. For low RU, SU-MIMO or SU/MU-MIMO with rank adaptation are assumed for higher rank extension. For medium/high RU, SU/MU-MIMO with rank adaptation is assumed for higher rank extension. SU/MU-MIMO with rank adaptation. Companies are encouraged to report the SU/MU-MIMO with RU | For low RU, SU-MIMO with rank adaptation are assumed For medium/high RU, SU/MU-MIMO with rank adaptation is assumed SU/MU-MIMO with rank adaptation. Companies are encouraged to report the SU/MU-MIMO with RU |
| Traffic load (Resource utilization) | - 50/70 % for CSI overhead reduction - 20/50 % for high rank extension 20/50/70% Companies are encouraged to report the MU-MIMO utilization. | - 70% for SU/MU-MIMO with rank adaptation - 20% for SU-MIMO with rank adaptation 20/50/70% Companies are encouraged to report the MU-MIMO utilization. |

| Supporting companies | Vivo, Ericsson, CAICT, ZTE, MediaTek, Futurewei, NVIDIA, Samsung, OPPO, InterDigital, CATT, Fujitsu, Xiaomi Huawei/Hisi, Qualcomm, Spreadtrum, CMCC |
| --- | --- |
| Objecting companies |  |

| Company | View |
| --- | --- |
| Moderator | @Ericsson: As the ‘Duplex’ entry in the overall EVM table is FDD mainly (reusing R16/17 EVM), would it be acceptable for you to keep ‘Frequency Range’ as it is (to keep consistent with the ‘Duplex’ entry)? (As far as Moderator’s knowledge, 2GHz FDD MM has also been commercially deployed. BTW please correct me if I understood incorrectly, as from R1-2203281, it seems 2GHz is also adopted.) Note: If no objection received till the email approval deadline of UTC 8:00 May 17, this proposal will be delivered for email approval by Mr. Chair. |
| Apple | Would like to clarify the understanding. “Note: The 2nd column applies if R16 TypeII codebook is selected as baseline, and the 3rd column applies if R17 TypeII codebook is selected as baseline.” Our previous understanding of this baseline is proposed for spatial-frequency domain AI based CSI compression. However based on email discussion for email thread -04, it seems the intention here is to use R16 or R17 baseline for both spatial-frequency, and time-spatial-freq domain AI based CSI compression. If this is the case, we object to the proposal. Comparing time-spatial-freq domain AI based CSI compression to R16/R17 baseline is not a fair comparison to show AI performance gain. |
| Moderator | @Apple A ‘FFS’ added for sub use case involving time domain (including both CSI compression and CSI prediction). BTW, companies are welcome to provide your view on the baseline release for comparing CSI enhancement involving time domain (compression & prediction). Please also refer to Question 4.2.4 in the 1st round discussions which is related with benchmark of time domain. |
| LG | We are ok if this is for spatial-freq domain compression. For time-spatial-freq domain AI based CSI, we agree with Apple. |
| ZTE | We agree with this proposal for spatial-freq domain sub use cases. |
| Nokia/NSB | We are ok in principle. It is our understood that companies may not (and most likely will not) be able to simulate all the combination cases in the EVM. However, it is not clear how Rel-17 Type-II port selection codebook can be used as baseline for this AI/ML study. Rel-17 Type-II assumes that CSI-RS ports are precoded by the gNB in both spatial and frequency domain in a UE-specific manner. Does this mean that comparing to Rel-17 FeType-II PS requires UE-specific datasets datasets that include this CSI-RS precoding? |
| Intel | Prefer Rel-16 Type II as baseline, agree with Nokia that precoded CSI-RS brings questions as to how we compare - whether the AI-ML model/training is UE specific ? |
| NTT DOCOMO | Regarding SCS, we prefer 30kHz as a baseline at 4GHz to capture the AI/ML gain in the popular deployed scenarios. |
| Samsung | @Nokia/NSB. We have the same understanding. If Rel-17 Type II port selection codebook is used, the input for the two-sided model is from precoded CSI-RS ports. |
| Moderator | @ Nokia/NSB @Samsung @Intel As per Moderator’s understanding, the baseline is used to compare the eventual performance under the same scenario, as said in the Note. Note: the baseline EVM is used to compare the performance with the benchmark release,… So, R17 is included here for comparing throughput, since generally the latest release is adopted as the benchmark for justification of AI/ML. On the other hand, it is decoupled with how to generate the AI/ML dataset or train the AI/ML model. E.g., the AI/ML dataset can be still constructed based on normal CSI-RS (i.e., unified over UEs/cell-specific). |
| Lenovo | Agree with Nokia that Rel-17 Type-II PS with precoded CSI-RS ports may require UE-specific AI/ML model and datasets and thus prefer only Rel-16 Type II codebook as baseline. |
| CMCC | Basically, we are fine with this proposal. |

### 2.4-3: Calibration of dataset and/or AI/ML model

For Proposal 6.2.6 which was delivered to May 13 GTW, it looks the views are converged. Considering the adopting 38.901 for dataset generation has been agreed in Agenda 9.2.1, the sentence is crossed out from the original proposal.

Proposal 2.4.3 Proposal 6.2.6: For the evaluation of the AI/ML based CSI feedback enhancement, if the dataset for AI/ML training/validation/testing is generated by TR 38.901 channel models, for the calibration purpose on the dataset and/or AI/ML model over companies, consider to align the parameters (e.g., for scenarios/channels) for generating the dataset in the simulation as a starting point.

| Supporting companies | Apple, vivo, LG, Ericsson, CAICT, ZTE, Nokia/NSB, MediaTek, Futurewei, NVIDIA, DCM, Samsung, OPPO, InterDigital, CATT, Fujitsu, Xiaomi (with clarification) Huawei/Hisi, Qualcomm, Spreadtrum, Lenovo, CMCC |
| --- | --- |
| Objecting companies |  |

| Company | View |
| --- | --- |
| Moderator | Note: If no objection received till the email approval deadline of UTC 8:00 May 17, this proposal will be delivered for email approval by Mr. Chair. |
| Intel | We prefer common data-set, so that variability across companies due to data-set is removed. Channel model according to TR38.901 can have various implementation flavors. It can be observed from R1-143469 that even marginal statistics of SCM parameters have a good variation across companies. |
| Xiaomi | We think common data-set generated by TR 38.901 with aligned parameters is not precluded, if so, we support this proposal. |
| Moderator | @Xiaomi Your understanding is correct. Aligning parameters is taken ‘as a starting point’. @Intel even using common dataset in future, we need to align the parameters to build that common dataset, right? |

3rd round email discussions

Some high priority questions/proposals are given in the 3rd round.

### 2.5-1: Generic evaluation methodology

Baseline SLS EVM table-channel estimation

For the channel estimation, there are still some different voices on: 1) ideal channel can be used for generating dataset (from Samsung), 2) ideal channel can be used for intermediate results (from QC), 3) ideal channel can be used for performance evaluation (from vivo). So Moderator tries to add in the purpose that ‘ideal CE can be used for comparing intermediate results’, and hope that can be a middle ground.

Upd Proposal 2.5.1 Proposal 6.2.4: For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, for ‘Channel estimation’, ideal DL channel estimation is optionally taken into the baseline of EVM for the purpose of calibration and/or comparing intermediate results (e.g., accuracy of AI/ML output CSI, etc.)

- Note: Realistic DL channel estimation with error modeling should be used for at least eventual performance comparison with the benchmark release and drawing SI conclusions.

- FFS: the ideal channel estimation is applied for dataset construction, or performance evaluation/inference.

- FFS: how to model the realistic channel estimation

- FFS: ideal channel used for intermediate results comparison with AI/ML output CSI from realistic channel estimation

| Supporting companies | Apple, LG, CAICT, ZTE, MediaTek, Futurewei, NVIDIA, DCM, Samsung, OPPO, Panasonic, CATT, Fujitsu, Xiaomi, Huawei/Hisi, Qualcomm (comment below) , Spreadtrum, Lenovo, CMCC, Ericsson |
| --- | --- |
| Objecting companies |  |

| Company | View |
| --- | --- |
| vivo | We are fine to consider ideal DL channel estimation as optional. Regarding the realistic channel estimation, at least, there are two alternatives to model such an error. Alt-1 is to directly generate the DL channel response matrix with an error, i.e., H’=H+e(SINR), where e(SINR) is the error term as a function of SINR received from the DL. Alt-2 is to estimate the channel based on the estimation algorithm, either MMSE or ZF. For simplicity, nevertheless, we are supportive of Alt-1. |
| CAICT | For intermediate results comparison, we are also fine with ideal DL channel estimation as baseline or optional. |
| ZTE | We agree with ideal DL channel estimation is taken into the baseline for calibration and comparing the intermediate results. However, if we need to compare the accuracy of AI/ML model under realistic DL channel estimation, how to deal with error modeling should be clarified and further discussed. |
| Futurewei | We are ok with the proposal. “Error modeling” for realistic DL channel estimation may need further discussion. |
| OPPO | We also support to use ideal channel estimation for dataset construction and performance evaluation. |
| CATT | Almost fine. Just a little contradictory to see ‘optionally’ and ‘baseline’ together in ‘optionally taken into the baseline of EVM’. Does it simply mean ‘optionally used in the EVM table’. |
| Qualcomm | We support the proposal, but feel the wording is not clear. Even for the simulations used for eventual performance comparison, companies may report and compare intermediate results/KPIs. In this case, the above wording becomes ambiguous. Ideal DL channel estimation for “comparing intermediate results” can have two interpretations: - When running simulations solely for the purpose of comparing intermediate results, ideal DL channel estimation is assumed at the UEs. - Irrespective of whether the simulation assumes realistic or DL channel estimation at the UEs, while calculating intermediate KPIs (e.g., SGCS), the target output CSI in the calculation is chosen based on the ideal DL channel. It would be important to clarify which of these two is meant by the proposal. Our response to the corresponding question in the earlier round is aligned with the second interpretation above. |
| CMCC | We support this proposal. We agree to use ideal channel estimation for calibration and some intermediate results comparison. |
| Moderator | @ CATT ‘baseline of EVM’ means the EVM for evaluating the eventual performance compared with the legacy release (used also in other proposals). Your understanding is correct, i.e., it means the EVM table. @QC For your 1st comment: ‘Even for the simulations used for eventual performance comparison, companies may report and compare intermediate results/KPIs’ – that is why ‘at least eventual performance comparison’ is used in the first bullet. For your 2nd comment: my straightforward understanding is your 1st interpretation, since the text says ‘ideal channel estimation’ instead of ‘ideal channel’ for intermediate result comparison. A ‘FFS’ added as per your 2nd comment. @vivo a ‘FFS’ added here for detailed error modeling method. |

Baseline SLS EVM table-Traffic model

Similar situation as ‘channel estimation’, that for ‘traffic model’, some companies (vivo, Intel, AT&T) hold the view that full buffer can be also used for performance evaluation. So, same as Proposal 2.5.1 that ‘comparing intermediate results’ is added as a purpose.

Proposal 2.5.2 Proposal 6.2.5: For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, for ‘Traffic model’, full buffer traffic is optionally taken into the baseline of EVM for the purpose of calibration and/or comparing intermediate results (e.g., accuracy of AI/ML output CSI, etc.)

- Note: FTP traffic should be used for at least eventual performance comparison with the benchmark release and drawing SI conclusions.

| Supporting companies | Apple, LG, CAICT, ZTE, MediaTek, Futurewei, NVIDIA, Samsung, OPPO, Panasonic, CATT, Xiaomi Huawei/Hisi, Qualcomm, Spreadtrum, Lenovo, CMCC, Ericsson |
| --- | --- |
| Objecting companies |  |

| Company | View |
| --- | --- |
| vivo | FTP and full buffer traffic models are idiomatically utilized in the pass EVM, e.g., in TR 36.897. Both should be supported in the Rel-18 EVM. FTP traffic model can offer the evaluated performance which is close to what a realistic system can achieve, while full buffer traffic model can demonstrate how far the throughput performance the AI-based CSI-feedback mechanism can go. If we only limit the traffic model to be FTP, we are never aware of the performance ceiling the AI-based CSI-feedback mechanism can approach. |
| ZTE | We basically agree with this proposal. Note that other FTP models can also be taken into baseline for intermediate or eventual performance evaluation. |
| OPPO | We agree with this proposal that full buffer can be optionally taken into the baseline of EVM. However, from our understanding, we think that the ‘Traffic model’ will not influence the intermediate results (e.g., accuracy of AI/ML output CSI). |
| CATT | Same question with the previous comment. A little contradictory to see ‘optionally’ and ‘baseline’ together in ‘optionally taken into the baseline of EVM’. Does it simply mean ‘optionally used in the EVM table’. |
| Qualcomm | We support the proposal, but feel the wording is not clear. Even for the simulations used for eventual performance comparison, companies may report and compare intermediate results/KPIs. In this case, the above wording becomes ambiguous. |
| CMCC | We agree with this proposal. Full buffer can be used for calibration, at least. |

Baseline LLS EVM

For the LLS EVM table, 4 companies have provided the inputs of the LLS table in the 1st round. Moderator only capture the generic part in the following in this section, as the AI/ML related parameters will be discussed later in a separate clause.

| Parameter | Value (Xiaomi) | Value (vivo) | Value (Fraunhofer) | Value (Fujitsu) |
| --- | --- | --- | --- | --- |
| Carrier frequency | 3.5GHz | 3GHz | 3.5GHz |  |
| Bandwidth | 10MHz | [20MHz] | 10MHz |  |
| Subcarrier spacing | 15KHz | 30kHz | 15KHz | 30 KHz |
| RB number | 48 | 52 | 48 | 24 |
| Subband number | 12 |  | 12 |  |
| Nt | 32 | 32 | 32 |  |
| Nr | 4 | 2 | 4 |  |
| Channel model | CDL-A300 | CDL-C |  | CDC-C |
| UE speed | 3km/h | 30km/h | 3 or 30 km/h |  |
| Delay spread | 300ns | 300ns |  |  |
| Channel estimation | Ideal |  | Realistic |  |
| RI | 1 |  |  |  |

As there are still diverse from the inputs of LLS proponents, and some of the parameters are not aligned with the SLS EVM, so the view from Moderator is to align with the almost stable SLS EVM (as also suggested by OPPO), while for the channel model part, a LLS specific channel model of CDL-C is adopted as baseline, and CDL-A as optional (as suggested by Fujitsu). In addition, the same ‘Note’ and ‘FFS’ are pasted which is the same as the SLS EVM. As it is a quite initial suggestion, a question is made accordingly to collect views from companies.

Question 2.5.1: For the evaluation of the AI/ML based CSI feedback enhancement, if LLS is preferred, the following table is taken as a baseline of EVM

- Note: the baseline EVM is used to compare the performance with the benchmark release, while the AI/ML related parameters (e.g., dataset construction, generalization verification, and AI/ML related metrics) can be of additional/different assumptions.

  - The conclusions for the use cases in the SI should be drawn based on generalization verification over potentially multiple scenarios/configurations.

- FFS: modifications on top of the following table for the purpose of AI/ML related evaluations.

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | FDD (TDD is not precluded), OFDM |
| Carrier frequency | 2GHz as baseline, optional for 4GHz |
| Bandwidth | 10MHz or 20MHz |
| Subcarrier spacing | 15kHz for 2GHz, 30kHz for 4GHz |
| Nt | 32 |
| Nr | 4 |
| Channel model | CDL-C as baseline, CDL-A as optional |
| UE speed | 3 km/h or 30 km/h |
| Delay spread | 30ns or 300ns |
| Channel estimation | FFS (same as SLS EVM) |
| RI | Rank 1-4 |

| Supporting companies | vivo, LG, CAICT, Samsung, OPPO, Panasonic, InterDigital, Fujitsu, Xiaomi |
| --- | --- |
| Objecting companies |  |

| Company | View |
| --- | --- |
| Moderator | Please LLS proponents provide more details to make the table as complete as possible, as Moderator observes the LLS EVM tables provided in the 1st round are not sufficient as the SLS table to achieve full alignment over companies. |
| vivo | We are supportive of this proposal. |
| LG | Editorial comment: RI -> Rank per UE |
| Samsung | UE speed: 3kmhr, 10km/h, 20km/h or 30km/h to be reported by companies |
| OPPO | For this carrier frequency, the ‘optional’ can be deleted. For SCS, we also support to consider SCS 15KHz for 4GHz as a baseline. While the specific antenna layout configuration for Nt and Nr should also be listed same as SLS EVM. |
| Fujitsu | Channel estimation: Use the practical channel estimation algorithms, e.g. LS or MMSE. |
| Huawei/Hisi | More parameters are lacking: Antenna configuration at gNB/UE, Link adaptation, Eventual KPI (BLER?UPT?), etc. |

### 2.5-2: Source to generate the dataset [void]

As the source of generating the dataset has been agreed in 9.2.1, the following proposal is not needed in 9.2.2.1.

Proposal 2.3.6 Question 2.2.8: For the evaluation of the AI/ML based CSI feedback enhancement, do you agree that the dataset for AI/ML training/validation/testing is generated by TR 38.901 channel models as a starting point?

- FFS whether/how to generate the dataset from field test.

### 2.5-3: Calibration of dataset and/or AI/ML model

As the principle for calibrating based on aligning the parameters for generating dataset has been delivered for 2.4-3 email approval, a next question is: which metric(s) can be used for calibration over companies.

Question 2.5.2: For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, besides the eventual KPI (e.g., throughput), what other metric(s) do you think can be used for the calibration purpose?

- Option 1: Accuracy of AI/ML output CSI

- Option 2: Received SNR

- Option 3: Other

| Option 1 | Supporting companies | Apple, vivo, LG, CAICT, ZTE, MediaTek, Futurewei, NVIDIA, DCM, Samsung, OPPO, Panasonic, InterDigital, CATT, Fujitsu, Xiaomi, Huawei/Hisi, Spreadtrum, Lenovo, CMCC |
| --- | --- | --- |
|  | Objecting companies |  |
| Option 2 | Supporting companies |  |
|  | Objecting companies |  |
| Option 3 | Supporting companies | Ericsson |
|  | Objecting companies |  |

| Company | View |
| --- | --- |
| Moderator | From Moderator’s view, here ‘other metric(s)’ may specifically mean the intermediate KPIs, but let’s hear the voices from companies first. Note: ‘Accuracy of AI/ML output CSI’ is used as an inclusive expression for addressing similarity/error of CSI reconstruction/prediction. Please elaborate the specific metric if you prefer Option 3. |
| vivo | We are supportive of Option-1 as a baseline, but optionally for Option-2 if it is necessary. |
| ZTE | We think the accuracy of AI/ML output CSI has higher priority to be taken into baseline for calibration and performance comparison. |
| OPPO | We support to use SGCS to calibrate the accuracy of AI/ML output CSI. |
| Qualcomm | Once the parameters have been aligned, there should be no need for further calibration of the data. |
| Lenovo | We believe, in the calibrating phase “intermediate KPIs” such as inference accuracy (option 1) should be used. |

### 2.5-4: Metrics

Intermediate KPIs

As discussed in the May 13 GTW, some companies believe it will be effort saving to separate the evaluation into two phases, with the initial phase of using a simplified evaluation with the purpose of AI/ML model/solution selection, and the final phase of delivering the eventual KPI for comparing with the benchmark.

Question 2.5.3: For the evaluation of the AI/ML based CSI feedback enhancement, do you agree that companies are encouraged to perform simplified evaluation to evaluate intermediate KPI(s) for the purpose of AI/ML solution and/or sub use case comparison?

- FFS: Parameter assumptions for the simplified evaluation

- Note: the simplified evaluation is not used for eventual performance comparison with the benchmark release and drawing SI conclusions.

| Supporting companies | Apple, vivo, CAICT, ZTE, MediaTek, Futurewei, NVIDIA, DCM, Samsung, OPPO, Panasonic, CATT, Xiaomi Huawei/Hisi, Qualcomm (comment below) , Spreadtrum, Lenovo, CMCC, Ericsson |
| --- | --- |
| Objecting companies |  |

| Company | View |
| --- | --- |
| vivo | We are supportive of this proposal. |
| LG | We are not sure on sub use case comparison. In order to finalize sub-use cases, we think performance evaluation/comparison seems more suitable. |
| ZTE | We generally agree with this proposal. Simplified evaluation is adopted for initial intermediate KPIs evaluation, while it is more crucial to evaluate the eventual system performance compared with the benchmark release. |
| NTT DOCOMO | We support the idea to consider the gain from simplified evaluation as an intermediate KPI. However, it is not necessary to encourage all companies to do it in case that some companies can evaluate the eventual KPIs. |
| Samsung | We are supportive of this proposal. |
| OPPO | We agree with this proposal. For the FFS item, we suggest to use the same parameter assumptions to the eventual system-level evaluation. |
| CATT | As commented in GTW, simplified evaluation is more like ‘direct AI/ML evaluation’. Evaluation on the AI/ML models themselves is helpful on fast calibration, fast AI/ML model selection and sub use case comparison. |
| Fujitsu | The details of simplified evaluation should be clarified. |
| Qualcomm | We suggest the term “standalone evaluation of AI/ML model performance” instead of “simplified evaluation” for clarity. We prefer that the note be removed, since the intermediate KPIs are still under discussion, and we feel it is premature to determine how the intermediate KPIs may impact the study and conclusions. |
| CMCC | We support this proposal. In the initial stage, intermediate KPIs can be used. However, it is more important to use eventual KPIs for sub use case comparison. |

For Proposal 2.3.8 in the 2nd round email discussions, some companies raise a different view on the metric except GCS/SGCS and NMSE, as well as the expression of CSI accuracy.

For the expression of CSI accuracy, as per the comments from QC and Nokia, it is replaced with ‘Accuracy of AI/ML output CSI’ to be more inclusive.

Proposal 2.5.3 Proposal 2.3.8: For the evaluation of the AI/ML based CSI feedback enhancement, as a starting point, take the intermediate KPIs of GCS/SGCS and NMSE as part of the ‘Evaluation Metric’ to compare the similarity/error between the input original CSI of the AI/ML model and the output CSI (e.g., recovered/predicted CSI) of the AI/ML model evaluate the accuracy of the AI/ML output CSI

- For GCS/SGCS,

  - FFS: how to calculate GCS/SGCS for rank>1

  - FFS: whether GCS or SGCS is adopted

- FFS other metrics, e.g., equivalent MSE, or received SNR.

| Supporting companies | Apple, vivo, LG, CAICT, ZTE, MediaTek, Futurewei, DCM, Samsung, OPPO, Panasonic, CATT, Fujitsu, Xiaomi Huawei/Hisi, Qualcomm (comment below) , Spreadtrum, Lenovo (comment below) , CMCC |
| --- | --- |
| Objecting companies |  |

| Company | View |
| --- | --- |
| Moderator | @Ericsson For your comments on the GCS part, ‘the cosine similarity of the reconstructed precoding vector with respect to the true strongest eigenvector will heavily (and unnecessarily) punish selecting the second strongest eigenvector as the precoding vector’ – isn’t that the UE should report multiple CSIs/PMIs separately for each rank for rank>1 situation (which is the same as legacy, with the only difference that the compressed CSIs/PMIs are AI/ML output without explicit physical meaning)? For your comments on the NMSE part, ‘Using such loss functions may thus lead to unnecessary large uplink overhead as the CSI contains information that is not useful for the scheduler’ – if it is for CSI prediction, then there seems no overhead issue; if it is for CSI compression, the overhead would be reflected to the eventual performance (as a potential drawback)? More clarifications are appreciated. |
| vivo | We are supportive of this proposal. |
| ZTE | We agree with this proposal. We think GCS and SGCS express the same meaning of similarity and only one metric is okay. We prefer using SGCS because SGCS can improve the resolution of performance gap for clear comparison. |
| OPPO | We agree with this proposal. From our opinion, we also prefer SGCS as the intermediate KPI. And how to calculate SGCS for rank>1 can be FFS. |
| Qualcomm | We thank the moderator for taking our feedback into account. Regarding the metric, we share the concern that GCS or SGCS may not reflect the impact on the final throughput. While we agree that this could be a starting point, other metrics should also be studied, especially for rank > 1 case. For example, the gap between the spectral efficiency achievable using the target eigen vector and the spectral efficiency achievable using the output eigen vector could be considered as a measure of accuracy. Another concern is: NMSE is well-suited to cases where the CSI output at the gNB is the raw channel, while GCS/SGCS is well-suited to cases where the output is the eigenvector. Based on this, we propose the following version: For the evaluation of the AI/ML based CSI feedback enhancement, as a starting point, take the intermediate KPIs of GCS/SGCS and/or NMSE as part of the ‘Evaluation Metric’ to evaluate the accuracy of the AI/ML output CSI - For GCS/SGCS, - FFS: how to calculate GCS/SGCS for rank>1 - FFS: whether GCS or SGCS is adopted - FFS other metrics, e.g., equivalent MSE, or received SNR, or spectral efficiency gap. |
|  | We believe the goal of the CSI feedback is better aligned with GSC [not NMSE], so we prefer to keep that one as the intermediate KPI. Of course, each company can optionally report other KPIs that they feel suitable. So, we suggest the following change For the evaluation of the AI/ML based CSI feedback enhancement, as a starting point, take the intermediate KPIs of GCS/SGCS and NMSE as part of the ‘Evaluation Metric’ - For GCS/SGCS, - FFS: how to calculate GCS/SGCS for rank>1 - FFS: whether GCS or SGCS is adopted - Companies may optionally report other intermediate KPIs as NMSE, equivalent MSE, … FFS other metrics, e.g., equivalent MSE, or received SNR. |
| Ericsson | The proposal is ok, but companies can also provide additional, more relevant KPI that better reflect the purpose of this use case. |

For Question 2.3.1 in the 2nd round discussion, between GCS and SGCS, it looks most companies are fine with both, while some companies raise a minor merit of SGCS that the resolution/gap between solutions will be better reflected (as pointed out by ZTE). So let’s see if SGCS will be adopted.

Proposal 2.5.4: For the evaluation of the AI/ML based CSI feedback enhancement, if the GCS/SGCS is adopted as the intermediate KPI as part of the ‘Evaluation Metric’, SGCS should be chosen.

- Note: wi is the target input original CSI vector of frequency unit i, and wi is the output CSI vector of frequency unit i. N is the total number of frequency units. E{∙} denotes the average operation over multiple samples.

SGCS=E1Ni=1N||wiHwi ||||wi||  ||wi||2

| Supporting companies | Vivo, CAICT, ZTE, MediaTek, Futurewei, DCM, Samsung, OPPO, Panasonic, Fujitsu, Xiaomi(with modification) Huawei/Hisi, Qualcomm, Spreadtrum, Lenovo(with a modification) , CMCC |
| --- | --- |
| Objecting companies |  |

| Company | View |
| --- | --- |
| vivo | We are supportive of this proposal. |
| ZTE | We agree with this proposal. |
| Samsung | A small modification to accommodate time-domain compression too. Proposal 2.5.4: For the evaluation of the AI/ML based CSI feedback enhancement, if the GCS/SGCS is adopted as the intermediate KPI as part of the ‘Evaluation Metric’, SGCS should be chosen. - Note: wi is the target input original CSI vector of frequency unit i, and wi is the output CSI vector of frequency resource unit i. N is the total number of time/frequency resource units. E{∙} denotes the average operation over multiple samples. |
| Fujitsu | We think both GCS and SGCS are OK. |
| Xiaomi | We generally agree with this proposal, but the target CSI vector is not clear, for example, it could be the eigenvector based on ideal channel matrix, and it could be eigenvector based on realistic channel estimation. We suggest to add a note for companies to report the target CSI they assumed in the simulation. Proposal 2.5.4: For the evaluation of the AI/ML based CSI feedback enhancement, if the GCS/SGCS is adopted as the intermediate KPI as part of the ‘Evaluation Metric’, SGCS should be chosen. - Note: wi is the target input original CSI vector of frequency unit i, and wi is the output CSI vector of frequency unit i. N is the total number of frequency units. E{∙} denotes the average operation over multiple samples. Companies are encouraged to report the detail information of target CSI vector assumed in the evaluation. SGCS=E1Ni=1N¦¦wiHwi ¦¦¦¦wi¦¦ ¦¦wi¦¦2 |
| Qualcomm | We agree with this proposal. On the wording, we prefer to replace “CSI vector” with “CSI” throughout the proposal for clarity. |
| Lenovo | We generally agree with GCS but we prefer GCS rather than SGSC. GCS=E1Ni=1N¦¦wiHwi ¦¦¦¦wi¦¦ ¦¦wi¦¦2 In SGSC we are penalizing the smaller ¦¦wiHwi ¦¦¦¦wi¦¦ ¦¦wi¦¦due to the power of 2 which we could not find a good reason to do such squaring. So. We prefer simple averaging of them as above. SGSC can be still optionally reported. |

For Question 2.3.2 in the 2nd round discussion, among the 3 candidate options for expressing GCS/SGCS, it is still controversial. Let’s see if we can first keep an open list and agree on a further study/down-select for the 3 options. In addition, as the calculations of all the 3 metrics may not introduce significant additional efforts, companies are encouraged to study and compare the pros/cons of the 3 options.

In addition, for Option 2, a normalized GCS/SGCS is changed as suggested by Fujitsu. For Option 3, as there are comments on how it works, some clarifications are added.

Proposal 2.5.5: For the evaluation of the AI/ML based CSI feedback enhancement, if the GCS/SGCS is adopted as the intermediate KPI as part of the ‘Evaluation Metric’, further study and down select at least one of the following options as GCS/SGCS for rank>1 cases

- Option 1: Average over all ranks

  - Note: wij is the jtheigenvector of the target CSI at frequency unit i and K is the rank of the channel, wij is the jth output CSI vector of frequency unit i.

GCS=E1Ni=1NjK||wijHwij ||||wij||  ||wij||,  SGCS=E1Ni=1NjK||wijHwij ||||wij||  ||wij||2

- Option 2: Weighted average over all ranks

  - Note: wijand λij are the jtheigenvector of the target CSI and its associated eigenvalue at frequency unit i and K is the rank of the channel, wij is the jth output CSI vector of frequency unit i.

GCS=E1Ni=1NjKλij||wijHwij ||||wij||  ||wij||

GCS=E1Ni=1NjKλijj=1Kλij||wijHwij ||||wij||  ||wij||,  SGCS=E1Ni=1NjKλijj=1Kλij||wijHwij ||||wij||  ||wij||2

- Option 3: GCS/SGCS is separately calculated for each rank (e.g., for K ranks, K GCS/SGCS values are derived separately, and the GCS/SGCS values for per rank are compared over solutions)

| Supporting companies | Apple, CAICT, MediaTek, Futurewei, Samsung, OPPO, Panasonic, CATT, Fujitsu, Xiaomi(with modification) Huawei/Hisi, Qualcomm (comment below) , Spreadtrum, CMCC |
| --- | --- |
| Objecting companies | Lenovo (agree with a modification) |

| Company | View |
| --- | --- |
| vivo | In general, GCS/SGCS should be separately calculated for each rank in case of evaluating the system performance. Therefore, we are supportive of Option 3 as a baseline. For the different purposes such as the calculation of loss function, however, Option-1 may be taken into account only as an intermediate KPI. Towards Option-2, there is no clear evidence to ensure the benefits by introducing a weight parameter in the formula. On the contrary, it may incur somewaht complicated issue in our performance analysis. |
| LG | We are generally fine with this proposal, but it can be discussed after agreeing on Proposal 2.5.3. |
| CAICT | We support option 1 should be used as loss function and intermediate KPI. |
| ZTE | We think Option 1 can be used as GCS/SGCS baseline, because Option 1 considers inter-layer interference and emphasizes the fairness of reconstruction for all layers. However, Option 2 is better to protect the accuracy of strongest layer, which may mask the poor recovered performance of other layers. While Option 3 may not jointly consider the influence between layers. |
| Samsung | Weighting the layers with their corresponding power coefficient is reasonable and emulates the actual performance. Therefore, we support Option 2. We are also open to take Option 1 if it simplifies things. |
| OPPO | We are both okay for Option 1 and Option 2. The advantage of Option 2 compared with Option 1 requires further clarification by companies. |
| Fujitsu | We prefer Option 2 to the other two. @Lenovo Thank you for your comments. We understand that the channel matrix may be normalized when it is generated. However, this ideal channel matrix is perturbed by noise and interference, and hence the estimated channel matrix is no longer properly normalized after the realistic channel estimation. As a result, the “unnormalized GCS/SGCS” may be outside the range of [0,1]. So, it is necessary to normalize the GCS/SGCS in the way we proposed. Specifically, the normalized GCS is GCS=E1Ni=1NjKλijj=1Kλij¦¦wijHwij ¦¦wij wij, where λij is an eigenvalue of the squared matrix GGH, and G is the channel matrix, the superscript H is the Hermitian operation. |
| Xiaomi | Similar to our comments on Proposal 2.5.4, we think companies need to provide the detailed information of target CSI in the evaluation. |
| Qualcomm | We have a concern that GCS or SGCS may not reflect the impact on the final throughput. While we agree that this could be a starting point, other metrics should also be studied, especially for rank > 1 case. For example, the gap between the spectral efficiency achievable using the target eigen vector and the spectral efficiency achievable using the output eigen vector could be considered as a measure of accuracy. For the SGCS formula in Option 2, if the output CSI equals the target CSI, the SGCS should be 1. This does not appear to be the case in the formula listed in the proposal. Therefore, we suggest the following version: SGCS=E1Ni=1NjK(λij)2j=1K(λij)2¦¦wijHwij ¦¦¦¦wij¦¦ ¦¦wij¦¦2 On the wording, for clarity, we suggest replacing “jth output CSI vector” with “estimate of the jth eigenvector reconstructed from the received CSI feedback”. |
| Lenovo | Option 1 is not a good metric since it gives same importance to all eigenvectors which is not correct and in practice eigenvectors corresponding to small eigne-values are not useful for scheduling purposes. For option 3, we are unclear how GCS/SGCS vectors (one value for each rank) will be compared between two models. For option2, we are in favor of the previous version of weighted GCS, i.e., GCS=E1Ni=1NjKλij¦¦wijHwij ¦¦¦¦wij¦¦ ¦¦wij¦¦ Note that when we compute eigenvectors we assume that averaged power of the channel realizations are normalized (not each individual realization of H). By dividing λij by the sum of eigenvalues, results in instantaneous channel normalization which is not preferred. @ Fujitsu: Thanks a lot for further explanation. You are correct that after realistic channel estimation the estimated channel is not perfectly normalized, however assuming a white noise the “Expectation” operation should solve this point. Moreover. Consider a case of ideal channel estimation. We believe that you agree in this case, GCS=E1Ni=1NjKλij¦¦wijHwij ¦¦¦¦wij¦¦ ¦¦wij¦¦ will be between [0,1]. If you agree with this, then, for idea channel estimation, SGCS=E1Ni=1NjK(λij)2j=1K(λij)2¦¦wijHwij ¦¦¦¦wij¦¦ ¦¦wij¦¦2 Will be strictly smaller than one (due to the j=1K(λij)2 term) which we do not want In fact, since we take expectation over multiple samples the simple the expectation of weighted GCS will be in [0,1] @Qualcom: The issue that you mention is due to the added normalization term. If we use GCS=E1Ni=1NjKλij¦¦wijHwij ¦¦¦¦wij¦¦ ¦¦wij¦¦ Then, if the output CSI equals the target CSI, the GCS should be 1 as you expected. |
| CMCC | We are fine with either option 1 or option 2. |

For NMSE in Proposal 2.5.3, it is not clear to Moderator whether there are open issues on the expression of the NMSE, so an open question is raised to see if more issues need to be discussed/clarified for NMSE.

Question 2.5.4: For the evaluation of the AI/ML based CSI feedback enhancement, if the NMSE is adopted as the intermediate KPI as part of the ‘Evaluation Metric’, do you think there is any additional issue that needs discussion/clarification?

| Company | View |
| --- | --- |
| vivo | No additional issue. |
| Qualcomm | NMSE is well-suited to cases where the CSI output at the gNB is the raw channel. |

Capability/complexity related KPIs

The following two proposals from the 1st round and the 2nd round discussions are mostly stable, but as they may be common over all use cases, Moderator will not treat them in the 3rd round, and see if they would be agreed in the framework part.

Proposal 2.3.9 Question 2.2.11: For the evaluation of the AI/ML based CSI feedback enhancement, do you agree that Floating point operations (FLOPs) is should be adopted as part of the ‘Evaluation Metric’, and reported by companies?

Proposal 2.3.10: For the evaluation of the AI/ML based CSI feedback enhancement, AI/ML memory storage in terms of AI/ML model size and number of AI/ML parameters is adopted as part of the ‘Evaluation Metric’, and reported by companies.

- FFS: the format of the AI/ML parameters

### 2.5-5: Methodology for generalization

Methodology of verifying generalization performance

For how to verify the generalization performance in the 1st round discussion, the views are quite diverse. But Moderator tries to at least make a progress on Option 1, i.e., making a mixed dataset, since it seems fewest companies are objecting Option 1 (only one company), so Moderator tries to put Option 1 as the option that should be at least supported, and put Option 2/3 as FFS.

Proposal 2.5.6: For the evaluation of the AI/ML based CSI feedback enhancement, from the dataset and testing/inference set construction perspective, the generalization of the AI/ML model is verified by at least adopting Option 1. FFS Option 2 and Option 3.

- Option 1: The training dataset is constructed by mixing training inputs from multiple configurations/scenarios, and the testing/inference is performed for a single configuration/ scenario

  - FFS: if the single configuration/ scenario belongs to the multiple configurations/scenarios

- FFS Option 2: The training dataset is constructed by training inputs from a single configuration#A/scenario#A, and the testing/inference is performed for a different single configuration#B/scenario#B

- FFS Option 3: The training dataset is constructed by training inputs from a single configuration#A/scenario#A, and the testing/inference is performed for the same configuration#A/scenario#A

- FFS: the detailed combination/priority of scenarios and configurations for verifying generalization.

- FFS: the generalization verification considering the model updating/selection

| Supporting companies | vivo (without FFS for Opiton 2 and 3), CAICT, MediaTek, Samsung, OPPO, Panasonic, InterDigital, CATT, Fujitsu, Xiaomi (with modification) Huawei/Hisi, Qualcomm (see comment below) , Spreadtrum, CMCC |
| --- | --- |
| Objecting companies | Lenovo |

| Company | View |
| --- | --- |
| Moderator | @OPPO The intention of Option 2 is to provide a baseline to observe/study how the performance will degrade when the AI/ML model is applied to an unseen scenario. @Ericsson @ InterDigital The intention of Option 3 is to provide an upper bound reference, as the AI/ML model may be scenario-specific. E.g., for a cell-specific AI/ML model deployed at a Macro site, it may not need to consider InH dataset. @Lenovo @ Futurewei This proposal mainly discuss the generalization verification issue from the dataset and testing/inference set construction perspective. @ Futurewei The priority of scenarios and configurations will be discussed in a separate question later. Will adding a ‘FFS’ as the second last bullet be acceptable for you? @Lenovo The generalization from the model updating/selection perspective will be discussed in a separate question later. Will adding a ‘FFS’ as the last bullet be acceptable for you? |
| vivo | The dataset constructed in Option 1 is utilized for the generalization of the AI/ML model. In contrast, the dataset constructed in Option 2 offers the worst performance while the dataset constructed in Option 3 offers the best performance. Therefore, Option 2 and Option 3 are still necessary in the system evaluation, but only for the comparison purpose. Note that Option 3 is the special case of Option 1 in case of single configuration/ scenario. |
| CAICT | Option 1 should be baseline. |
| ZTE | We think Option 3 should also be adopted as a baseline, because verification of other cases, including the legacy CB, needs to be compared with Option 3, which is a performance upper bound. In addition, some mixed details, e.g. an appropriate mixed ratio, need further discussion for Option 1. We are not sure whether AI model can achieve good generalization for Option 2, which can be further studied. |
| Futurewei | We are ok with the proposal in general. However, we think Option 3 should always be verified as default (scenario-based deployment option) while Option 1 and Option 2 can be optionally verified. |
| NVIDIA | Option 3 should be baseline – scenario specific models are of high interest in practice. |
| OPPO | We agree with this proposal. Option 1 and Option 3 should be considered as a baseline to evaluate the generalization performance. Option 2 can be optional. |
| CATT | In our understanding, Option 3 should be the upper bound performance for comparison. Option 1(or 2) will be compared to Option 3 to verify the generalization. |
| Xiaomi | We think all of the options can be used to verify the generalization of the AI/ML model, but option 1, 3 should be the baseline. Proposal 2.5.6: For the evaluation of the AI/ML based CSI feedback enhancement, from the dataset and testing/inference set construction perspective, the generalization of the AI/ML model is verified by at least adopting Option 1 and option 3. FFS Option 2 and Option 3. - Option 1: The training dataset is constructed by mixing training inputs from multiple configurations/scenarios, and the testing/inference is performed for a single configuration/ scenario - FFS: if the single configuration/ scenario belongs to the multiple configurations/scenarios - FFS Option 2: The training dataset is constructed by training inputs from a single configuration#A/scenario#A, and the testing/inference is performed for a different single configuration#B/scenario#B - FFS Option 3: The training dataset is constructed by training inputs from a single configuration#A/scenario#A, and the testing/inference is performed for the same configuration#A/scenario#A - FFS: the detailed combination/priority of scenarios and configurations for verifying generalization. - FFS: the generalization verification considering the model updating/selection |
| Qualcomm | Option 3 can be useful for initial verification or upper bound and option 1 can be used for final evaluation. Another approach could be the following. Instead of discussing and agreeing on how the training dataset is constructed, the generalization capability can be studied by specifying several test scenarios and evaluating the performance of a single common AI/ML model across all those test scenarios. The training dataset to train such a model can be selected and reported by companies. The performance of such a common model can be compared with scenario-specific models custom-trained for each test scenario. We feel this approach may be easier as companies are free to determine the training data mix, as long as different drops and/or different UEs are used compared to the test dataset. |
| Lenovo | We believe “the generalization of the AI/ML model is verified by checking how well the proposed scheme performs on a good representative test dataset.” So we mainly need to focus on the test dataset; therefore, we think there is no need for three separate discussions for analysis of the generalizability. We have provided our views in response to the next question. |
| CMCC | We are generally fine with this proposal. Option 1 can be the baseline for generalization verification. Option 2 maybe shows the worst performance on generalization and Option 3 shows the upper bound of generalization. |
| Samsung | Fine with proposal. |

This question is raised by Futurewei in the 1st round, on the priority of scenarios and configurations for verifying generalization.

Question 2.5.5: For the evaluation of the AI/ML based CSI feedback enhancement, for how to consider the combination/priority of various scenarios and configurations for constructing dataset and testing/inference set, what is your view?

- Option 1 (scenario based): Fix the scenario, and verify the generalization performance of an AI/ML model, which is trained by dataset from this specific scenario with one/mixed configuration(s), and applied to the inference/testing set of the same scenario with a same/different configuration

  - Note: the scenario can be one of the set {‘Scenario’, ‘Frequency range’, ‘Numerology’, ‘Inter-BS distance’, Outdoor/Indoor UE distribution, UE speed} from the baseline EVM

  - Note: The configurations can be one or a mix of the set {Antenna port number, Antenna configurations, bandwidth, CSI feedback payload} from the baseline EVM.

- Option 2 (scenario generalized): Verify the generalization performance of an AI/ML model, which is trained by dataset from this one/mixed scenario(s) with one/mixed configuration(s), and applied to the inference/testing set of the same/different scenario with a same/different configuration

  - Note: the scenario can be one or a mix of the set {‘Scenario’, ‘Frequency range’, ‘Numerology’, ‘Inter-BS distance’, Outdoor/Indoor UE distribution, UE speed} from the baseline EVM

  - Note: The configurations can be one or a mix of the set {Antenna port number, Antenna configurations, bandwidth, CSI feedback payload} from the baseline EVM.

- Option 3: Other

| Company | View |
| --- | --- |
| Moderator | It is a good direction to consider the detailed priority ordering of generalization verification, since there are numerous combination of different scenarios/configurations. But considering the work load of this meeting, it is not expected to draw any conclusion on this point during the 109-e meeting. Companies are encouraged to provide open views for this question. @ Futurewei please elaborate your proposal and make corrections to Moderator’s understanding if needed. |
| vivo | The scenarios and configurations are equally important in our performance evaluation. However, we may leverage the relevant parameters in EVM, dependent on the sub use cases. This means that, in the construction of data set, the combination/priority is a use case basis, and we don’t need to (de)periodize them without consideration of use case. In the notes, we prefer to have some minor rewording as follows: - Note: the scenario can be one of the set {‘Channel Scenario’, ‘Frequency range’, ‘Numerology’, ‘Inter-BS distance’, Outdoor/Indoor UE distribution, UE speed, etc.} from the baseline EVM Note: The configurations can be one or a mix of the set {Antenna port number, Antenna configurations, bandwidth, CSI feedback payload, etc.} from the baseline EVM. |
| CAICT | We believe both option 1 and 2 could be considered and option 2 is preferred. |
| ZTE | We think many details need to be discussed and determined for dataset generation with different scenarios/configurations, which brings much workload for evaluation. We suggest generalization verification can be deferred for further study. In addition, we are not clear that trained by dataset from this one/mixed scenario(s) with one/mixed configuration(s), and applied to the inference/testing set of the same scenario with a same/different configuration in Option 2. We think it may be the same as Option 1, which needs further clarification. |
| Futurewei | We are generally ok. However, we suggest separating Option 2 into 2 may be clearer: - Atl1: AI/ML model is trained by dataset from one scenario with one or mixed configuration(s), and applied to the inference/testing set of different scenario(s) with same or different configuration(s) - Alt2: AI/ML model is trained by dataset from mixed scenarios, each with one or mixed configuration(s), and applied to the inference/testing set of the same or different scenarios with same or different configuration(s) |
| OPPO | We think that this classification is not a good way to discuss generalization problem. From our view, whether ‘scenario based’ or ‘scenario generalized’ is not the key issue, and the definitions of ‘scenario’ and ‘configuration seem not very proper. We think that the generalization problem can be described as: To evaluate the performance of AI/ML model trained on dataset#1 and tested/inferenced on dataset#2, where dataset#1 and dataset#2 are generated under different parameter assumptions. While the key question is how the ‘different parameter assumptions’ affects the dataset and AI/ML model utilization. Therefore, we have two cases, which should be discussed separately: Case1. Different parameter assumptions, such as ‘Scenario’, ‘Frequency range’, ‘Numerology’, ‘Inter-BS distance’, Outdoor/Indoor UE distribution, UE speed, only lead to different datasets with the same AI/ML model input/output dimension, so that the AI/ML model trained on dataset#1 can be directly used on dataset#2. Case2. Different parameter assumptions, such as ‘Antenna port number’, ’Sub-band number’, ’CSI feedback payload’ lead to different datasets and AI/ML model input/output dimensions, so that the AI/ML model trained on dataset#1 cannot be directly used on dataset#2. Some AI/ML model interface should be changed and re-designed. From our view, Case 1 should be firstly studied in the initial stage of this SID since it is simpler than Case 2 for evaluation. Case 2 should also be considered in the future. |
| CATT | Both option 1 and option 2 can be considered. Option 1 can be used to verify configuration generalization and Option 2 can be used to verify scenario generalization. |
| Fujitsu | We prefer Option 2 to Option 1. |
| Qualcomm | Option 2 is preferred but both could be considered. The approach outlined by our response to Proposal 2.5.6 above can also be considered. |
| Lenovo | One of the main evaluation criteria of an AI/ML model is its Generalization capability to verify if the proposed scheme can work properly in different situations that may happen in the real world. If a proposed scheme performs appropriately over all possible network scenarios/configurations, we may say that the proposed scheme is Generalizable across different scenarios/configurations. If the model has satisfactory results only for the configuration parameter setting of the same scenario, the proposed scheme is Generalizable within a specific scenario. Bases on that we suggest the following proposal: The generalization capability of an AI/ML based CSI feedback can be categorized into the following levels: - Level 1: Generalizable within a scenario: Verify the generalization capability of an AI/ML model by evaluating its performance for each configuration over a set of multiple configurations of a single scenario. - The set of multiple configurations of a single scenario (testing dataset) should be a good representation of all different possible configurations for that single scenario. - For this case, training dataset may contains samples from that specific scenario with one/mixed configuration(s). - Level 2: Generalizable across multiple scenarios: Verify the generalization capability of an AI/ML model by evaluating its performance for each scenario/configuration over a set of multiple scenarios/configurations. - The set of multiple scenarios/configurations (testing dataset) should be a good representation of all different possible scenarios and configurations of the network. - For this case, training dataset may contains samples from one/mixed scenarios with one/mixed configuration(s). Note: the scenario can be one of the set {‘Scenario’, ‘Frequency range’, ‘Numerology’, ‘Inter-BS distance’, Outdoor/Indoor UE distribution, UE speed} from the baseline EVM Note: The configurations can be one or a mix of the set {Antenna port number, Antenna configurations, bandwidth, CSI feedback payload} from the baseline EVM. |
| CMCC | We prefer Option 2. Besides, the definitions of ‘scenario’ and ‘configuration’ seems not very clear. We think the two cases raised by OPPO should be discussed. |
| Samsung | First, we may need to differentiate general “scenario” and “deployment scenario”. The Notes can be modified as follows: - Note: the scenario can be one of the set {‘Deployment Sscenario’, ‘Frequency range’, ‘Numerology’, ‘Inter-BS distance’, Outdoor/Indoor UE distribution, UE speed} from the baseline EVM In Option 2, is it FL’s intention that “same/different scenario” apply only when the model is trained with dataset for single scenario? |

This question is raised by Lenovo in the 1st round, on the generalization verification from the model updating/selection perspective. Companies are encouraged to provide open views for Options.

Question 2.5.6: For the evaluation of the AI/ML based CSI feedback enhancement, from the AI/ML model updating/selection perspective, for how to verify the generalization, what is your view?

- Option 1: A single AI/ML model is trained based on scenario#A/configuration#A, and applied to the inference/testing to a same scenario#A/configuration#A or a different scenario#B /configuration#B.

- Option 2: An AI/ML model is trained based on scenario#A/configuration#A, fine-tuned with additional dataset from scenario#B/configuration#B, and applied to the inference/testing to scenario#B/configuration#B.

- Option 3: Multiple AI/ML models are trained based on scenario#A/configuration#A, scenario#B/configuration#B, …, respectively, and separately applied to the inference/testing to scenario#A/configuration#A, scenario#B/configuration#B, respectively.

- Option 4: Other

| Company | View |
| --- | --- |
| Moderator | It is a good direction to consider the generalization verification from the AI/ML model updating/selection perspective. But considering the work load of this meeting, it is not expected to draw any conclusion on this point during the 109-e meeting. Companies are encouraged to provide open views for this question. @Lenovo please elaborate your proposal and make corrections to Moderator’s understanding if needed. |
| vivo | The data set should be constructed based on the predetermined scenario/configuration, individually. As discussed in Proposal 2.5.6, an mixing data set will be synthesized for AI-model training, and an mixing data set will be synthesized for testing/validation, dependent on the sub use case. From the AI/ML model updating/selection perspective, similarly, we can collect the data samples (generated from either the same scenario/configuration or different scenarios/configurations) in a batch or a mini-batch, and update/select the AI-model. |
| CAICT | We share the same view from vivo. |
| ZTE | This question can be discussed after AI/ML model updating/selection is finalized in 9.2.1. Initially we think Option 3 may be efficient for adapting to different scenarios with no need for online training and may show a good generalization performance, but extra storage overhead needs to be considered. In addition, we are open to discuss Option 1 and Option 2. |
| OPPO | It seems that only Option 1 is relevant to generalization issue, Option 2 with AI/ML model fine-tuning may need to online training in practical deployment. Option 3 with AI/ML model selection with matched training set and testing set can give the upper bound of the AI/ML model performance. |
| CATT | Open to study Option 1 to 3. |
| Fujitsu | We do not think it is necessary to be considered for evaluation. |
| Lenovo | We believe we could combine the generalizability discussions under one proposal. We have provided our views in response to the previous question. |
| Samsung | All, Option 1 to Option 3, could be relevant cases. We agree with ZTE that this can be discussed under 9.2.1. |

Assumptions on scenario(s)/configuration(s)

For the assumptions on the scenario(s), it looks the ‘Scenario’, Outdoor/Indoor UE distribution and Outdoor UE speed receive most support. While other items also suggested by some companies to adopt all of them for verifying the generalization. Regarding the work load, the first items are proposed, while other are for FFS.

Proposal 2.5.7: For the evaluation of the AI/ML based CSI feedback enhancement, the following scenario(s) can be considered as a starting point for verifying the generalization of the AI/ML model

- Scenario: (e.g. UMa, UMi, FFS InH)

- Outdoor/Indoor UE distribution (e.g., Outdoor/Indoor UE ratio)

- Outdoor UE speed (e.g., 3km/h, 30km/h, etc.)

- FFS Frequency range (e.g., 2GHz, 4GHz, etc.)

- FFS Numerology (e.g., 15kHz, 30kHz, etc.)

- FFS Inter-BS distance (e.g., ISD=200m, 500m)

- FFS: other scenario(s) are not precluded

- FFS: the exact value and mixing ratio

- Note: companies are encouraged to report which scenario/set of mixed scenarios are adopted for constructing dataset, and which scenario is adopted for testing/inference

| Supporting companies | LG, CAICT, MediaTek, Futurewei, OPPO, InterDigital, CMCC |
| --- | --- |
| Objecting companies |  |

| Company | View |
| --- | --- |
| Apple | Do not see 30km/h speed plays a role for spatial/freq domain CSI compression data set study. It is for CSI prediction use case. |
| vivo | For the generalization purpose, InH should be included in the scenario. According to our simulation analysis, the generalization between UMa and UMi is comparably simple, while the generalization between UMa/UMi and InH is quite tricky, strongly relying on the synthetic data set. For instance, the ratio of data samples generated from different channel scenarios is one of key factor needed to be manipulated. |
| OPPO | We support this proposal. The mixed ‘Scenario’, ‘Outdoor/Indoor UE distribution’ and ‘Outdoor UE speed’ should be considered for training dataset. For testing/inference, constructing multiple datasets with only single configuration instead of mixed dataset is suggested. |
| CATT | Generalization verification for scenarios (e.g. UMa, UMi, FFS InH) can be considered as a starting point. Other parameters for given scenario(s) can be FFS. |
| Qualcomm | Generalization across scenario (UMa, UMi, InH, RMa, etc.) is preferred since it already includes several aspects including inter-BS distance, outdoor/indoor distribution, etc. |
| Samsung | In one realization, a single model could be trained and applied for all deployment scenarios. In this case, one may require to evaluate generalization capability of a model across multiple deployment scenarios. In a yet another case, a UE and/or gNB may keep distinct models trained to be applied to distinct deployment scenarios. In this case, it may be beneficial to validate the generalization capability over different distribution of channel parameters within a single deployment scenario. To this end, we consider different distributions of channel conditions in a single deployment scenario. As commented in the previous question We suggest modification as the followings. - Deployment Sscenario: (e.g. UMa, UMi, FFS InH) - Outdoor/Indoor UE distribution (e.g., Outdoor/Indoor UE ratio) - Outdoor UE speed (e.g., 3km/h, 30km/h, etc.) - FFS Frequency range (e.g., 2GHz, 4GHz, etc.) - FFS Numerology (e.g., 15kHz, 30kHz, etc.) - FFS Inter-BS distance (e.g., ISD=200m, 500m) - FFS: other scenario(s) are not precluded - FFS: the exact value and mixing ratio - FFS Different distributions/set of the general channel parameters (e.g., delay spread, angle spread, gNB heights, Inter-BS distance, etc.) - FFS: channel propagation (LOS/NLOS) |

For the assumptions on the configuration(s), a similar proposal is provided.

Proposal 2.5.8: For the evaluation of the AI/ML based CSI feedback enhancement, the following configuration(s) can be considered as a starting point for verifying the generalization of the AI/ML model

- Antenna port number (e.g., 32 ports, 16 ports)

- Antenna configurations (e.g., wavelength)

- Bandwidth: (e.g., 10MHz, 20MHz)

- CSI feedback payload

- FFS: other configuration(s) are not precluded

- FFS: the exact value and mixing ratio

- Note: companies are encouraged to report which configuration/set of mixed configurations are adopted for constructing dataset, and which configurationis adopted for testing/inference

| Supporting companies | Vivo, LG, CAICT, MediaTek, InterDigital, CMCC |
| --- | --- |
| Objecting companies |  |

| Company | View |
| --- | --- |
| vivo | We are fine with this proposal. “Antenna configurations (e.g., wavelength)” needs to be changed to “Antenna configurations (e.g., antenna spacing)”. |
| Futurewei | As the feedback we provided previously, we think companies should strive to use one main model architecture to support various configurations and apply adaptation techniques to support different configurations while keeping the main model architecture unchanged. |
| OPPO | From our view, the ‘Antenna port number’ and ‘CSI feedback payload’ mix may lead to different AI model input/output dimensions. We prefer to study this issue in the later stage. |
| CATT | Support vivo’s update. |
| Lenovo | We believe we need to have separate “Antenna port number” and “Antenna configurations” for the UE and gNB. The definition “CSI feedback payload” is also not clear for us |
| Samsung | Share the same view as OPPO. “Antenna port number” may lead to different input dimension and different model. Instead, a model may have variable performance for different ranks, i.e., rank restriction (configuration). Proposal 2.5.8: For the evaluation of the AI/ML based CSI feedback enhancement, the following configuration(s) can be considered as a starting point for verifying the generalization of the AI/ML model - Antenna port number (e.g., 32 ports, 16 ports) - Antenna configurations (e.g., wavelength) - Bandwidth: (e.g., 10MHz, 20MHz) - CSI feedback payload - Rank restriction (configuration) - FFS: other configuration(s) are not precluded - FFS: the exact value and mixing ratio - Note: companies are encouraged to report which configuration/set of mixed configurations are adopted for constructing dataset, and which configuration is adopted for testing/inference |

4th round email approval

### 2.6-1: Generic evaluation methodology: Baseline SLS EVM table-overall

As there is still one company has concern on the the ftp model as baseline, so the overall table is put back to the 4th round email approval, the only two changes: 1) changing ‘traffic model’ to ‘FFS’ as there is indeed no consensus on the last round; 2) editorial change of removing ‘Number of RBs’ as it is also removed in Proposal 2.6.2 in a previous round.

So companies please check if changes will be good for you.

Proposal 2.6.1 Proposal 2.4.1: For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, the following table is taken as a baseline of EVM

- Note: the following table captures the common parts of the R16 CSI enhancement EVM table and the R17 CSI enhancement EVM table, while the different parts are FFS.

- Note: the baseline EVM is used to compare the performance with the benchmark release, while the AI/ML related parameters (e.g., dataset construction, generalization verification, and AI/ML related metrics) can be of additional/different assumptions.

  - The conclusions for the use cases in the SI should be drawn based on generalization verification over potentially multiple scenarios/configurations.

- FFS: modifications on top of the following table for the purpose of AI/ML related evaluations.

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD (TDD is not precluded), OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) is a baseline. Other scenarios (e.g. UMi@4GHz 2GHz, Urban Macro) are not precluded. |
| Frequency Range |  | FR1 only, FFS 2GHz or 4GHz as a baseline |
| Inter-BS distance |  | 200m |
| Channel model |  | According to TR 38.901 |
| Antenna setup and port layouts at gNB |  | Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ - 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ Other configurations are not precluded. |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) Other configuration is not precluded. |
| BS Tx power |  | 41 dBm for 10MHz, 44dBm for 20MHz, 47dBm for 40MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz for 2GHz, 30kHz for 4GHz |
| Number of RBs |  | FFS |
| Simulation bandwidth |  | FFS |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | FFS |
| MIMO layers |  | For all evaluation, companies to provide the assumption on the maximum MU layers (e.g. 8 or 12) |
| CSI feedback |  | Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback) : 5 ms, - Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms |
| Overhead |  | Companies shall provide the downlink overhead assumption (i.e., whether the CSI-RS transmission is UE-specific or not and take that into account for overhead computation) |
| Traffic model |  | FFS FTP model 1 with packet size 0.5 Mbytes as a baseline Other FTP model is not precluded. FFS full buffer |
| Traffic load (Resource utilization) |  | FFS |
| UE distribution |  | - 80% indoor (3km/h), 20% outdoor (30km/h) FFS whether/what other indoor/outdoor distribution and/or UE speeds for outdoor UEs needed |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic as a baseline FFS ideal channel estimation |
| Evaluation Metric |  | Throughput and CSI feedback overhead as baseline metrics. Additional metrics, e.g., ratio between throughput and CSI feedback overhead, can be used. Maximum overhead (payload size for CSI feedback)for each rank at one feedback instance is the baseline metric for CSI feedback overhead, and companies can provide other metrics. |
| Baseline for performance evaluation |  | FFS |

| Supporting companies | Apple, vivo (with comments), LG, Ericsson, CAICT, ZTE, Nokia/NSB, mediatek, Futurewei, NVIDIA, DCM, Samsung, OPPO, InterDigital, CATT, Fujitsu, Xiaomi, Huawei/Hisi, Qualcomm, Spreadtrum, Lenovo, CMCC, DCM, Panasonic |
| --- | --- |
| Objecting companies |  |

| Company | View |
| --- | --- |
| LG | Regarding UE antenna height and gain, we can directly refer TR 38.901 instead of TR 36.873. The difference btw two TRs is whether modeling of co-pol is included or not. |
| vivo | In scenario, we prefer to include the “InH”. Dense Urban (Macro only) is a baseline. Other scenarios (e.g. UMi@4GHz 2GHz, Urban Macro, InH) are not precluded. |
| Qualcomm | We agree with the proposal |
| Ericsson | Support |
| Moderator | @LG As I am concerned whether other companies have time for double check on 901 and 873, could you live with the original version? @vivo as ‘while the AI/ML related parameters (e.g., dataset construction, generalization verification, and AI/ML related metrics) can be of additional/different assumptions’, we can make a separate discussion on generalization issues. |
| CATT | We still think the metric of ‘ratio between throughput and CSI feedback overhead’ is unnecessary. But with the understanding that this is just an optional metric, we are fine with the table. |
| ZTE | We agree with the proposal. |
| Samsung | Reiterating our comment for Round 3. We support the proposed Table. In our view, these can be decided early - Frequency Range: FR1 only, 2GHz baseline 4GHz optional - UE distribution: Option 1: 80% indoor (3km/h), 20% outdoor (30km/h) Option 2: 100% outdoor (variable speed, e.g., x1% 10km/h, x2% 20km/h and x3% 30km/h to be reported by companies) |

### 2.6-2: Generic evaluation methodology: Baseline SLS EVM table-FFS part

As some companies still have some comments on the dataset construction for Proposal 2.4.2 in the 3rd round email approval, it is brought up again with some clarifications.

Proposal 2.6.2 Proposal 2.4.2: For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, the following parameters are taken into the baseline of EVM

- Note: The 2nd column applies if R16 TypeII codebook is selected as baseline, and the 3rd column applies if R17 TypeII codebook is selected as baseline.

  - The baseline for performance evaluation is not related with R18 dataset construction or AI/ML model training

  - FFS baseline for potential sub use cases involving CSI enhancement on time domain

- Note: the baseline EVM is used to compare the performance with the benchmark release, while the AI/ML related parameters (e.g., dataset construction, generalization verification, and AI/ML related metrics) can be of additional/different assumptions.

  - The conclusions for the use cases in the SI should be drawn based on generalization verification over potentially multiple scenarios/configurations.

- FFS: modifications on top of the following table for the purpose of AI/ML related evaluations.

| Parameter | Value (if R16 as baseline) | Value (if R17 as baseline) |
| --- | --- | --- |
| Frequency Range | FR1 only, 2GHz as baseline, optional for 4GHz. | FR1 only, 2GHz with duplexing gap of 200MHz between DL and UL, optional for 4GHz |
| Simulation bandwidth | 10 MHz for 15kHz as a baseline, and configurations which emulate larger BW, e.g., same sub-band size as 40/100 MHz with 30kHz, may be optionally considered. Above 15kHz is replaced with 30kHz SCS for 4GHz. | 20 MHz for 15kHz as a baseline (optional for 10 MHz with 15KHz), and configurations which emulate larger BW, e.g., same sub-band size as 40/100 MHz with 30kHz, may be optionally considered. Above 15kHz is replaced with 30kHz SCS for 4GHz |
| MIMO scheme | SU/MU-MIMO with rank adaptation. Companies are encouraged to report the SU/MU-MIMO with RU | SU/MU-MIMO with rank adaptation. Companies are encouraged to report the SU/MU-MIMO with RU |
| Traffic load (Resource utilization) | 20/50/70% Companies are encouraged to report the MU-MIMO utilization. | 20/50/70% Companies are encouraged to report the MU-MIMO utilization. |

| Supporting companies | Vivo, Ericsson, CAICT, ZTE, MediaTek, Futurewei, NVIDIA, Samsung, OPPO, InterDigital, CATT, Fujitsu, Xiaomi Huawei/Hisi, Qualcomm, Spreadtrum, CMCC,Apple, Lenovo, DCM, Panasonic |
| --- | --- |
| Objecting companies |  |

| Company | View |
| --- | --- |
| Moderator | @ Nokia/NSB @Samsung @Intel @Lenovo As per Moderator’s understanding, the baseline is used to compare the eventual performance under the same scenario, as said in the Note. Note: the baseline EVM is used to compare the performance with the benchmark release,… E.g., if a company evaluate R17 as baseline, it is used for comparing throughput (since generally the latest release is adopted as the benchmark for justification of AI/ML), but has nothing to do with how to generate the AI/ML dataset or train the AI/ML model. In that sense, the AI/ML dataset can be still constructed based on normal CSI-RS (i.e., unified over UEs/cell-specific). A Note ‘The baseline for performance evaluation is not related with R18 dataset construction or AI/ML model training’ is added as a sub-bullet to clarify. |
| Qualcomm | We agree with the proposal. |
| Ericsson | Support |
| ZTE | We agree with the proposal. |
| Samsung | Thank you, moderator for the clarification. In our view, Rel-17 Type II PS CB is a fair baseline scheme to compare eventual results only when the angle-delay reciprocity is exploited for AI/ML based solution too. With this assumption, we are fine with the considering both Rel-16 and Rel-17 CBs. Note: For fairness, the same assumption on the exploitation of angle-delay reciprocity should be taken by the AI/ML based CSI feedback scheme and by the selected baseline scheme. |

### 2.6-3: Generic evaluation methodology: Baseline SLS EVM table-Channel estimation

It looks in the 3rd round discussion, the channel estimation part is relatively converged, so brought up to email approval in the 4th round, with wording changes on the 1st Note (as per QC’s comment) and two newly added FFS (from vivo and QC respectively). Updates basedon QC’s comment.

Upd Proposal 2.6.3 Upd Proposal 2.5.1: For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, for ‘Channel estimation’, ideal DL channel estimation is optionally taken into the baseline of EVM for the purpose of calibration and/or comparing intermediate results (e.g., accuracy of AI/ML output CSI, etc.)

- Note: Realistic DL channel estimation should be used for at least Eeventual performance comparison with the benchmark release and drawing SI conclusions should be based on realistic DL channel estimation.

- FFS: the ideal channel estimation is applied for dataset construction, or performance evaluation/inference.

- FFS: how to model the realistic channel estimation

- FFS: Whether ideal channel is used as target CSI for intermediate results calculation comparison with AI/ML output CSI from realistic channel estimation

| Supporting companies | Apple, LG, CAICT, ZTE, MediaTek, Futurewei, NVIDIA, DCM, Samsung, OPPO, Panasonic, CATT, Fujitsu, Xiaomi, Huawei/Hisi, Qualcomm (comment below) , Spreadtrum, Lenovo, CMCC, Ericsson |
| --- | --- |
| Objecting companies |  |

| Company | View |
| --- | --- |
| Moderator | @ CATT ‘baseline of EVM’ means the EVM for evaluating the eventual performance compared with the legacy release (used also in other proposals). Your understanding is correct, i.e., it means the EVM table. @QC For your 1st comment: See the change on the 1st Note. For your 2nd comment: my straightforward understanding is your 1st interpretation, since the text says ‘ideal channel estimation’ instead of ‘ideal channel’ for intermediate result comparison. A ‘FFS’ added as per your 2nd comment. @vivo a ‘FFS’ added here for detailed error modeling method. |
| Qualcomm | We thank the moderator for the changes and agree with the proposal with the following edit. For the last FFS, we would like to update the wording for clarity: FFS: Whether ideal channel is used as target CSI for intermediate results calculation comparison with AI/ML output CSI from realistic channel estimation |
| Ericsson | Support the proposal |
| Moderator | Editorial updates based on QC’s comment |
| CATT | @FL, thanks for the explanation. We are OK with the proposal. |
| ZTE | We agree with this proposal. |
| Xiaomi | We think the latest version for the last FFS is fine. In Proposal 2.7.6, we captured a note saying that encourage companies to report the detail information of the vector of target CSI assumed in the evaluation. We think these information can be helpful to further decide whether use ideal channel as target CSI for metric calculation, the FFS can be decided in the next meeting based on the evaluation results and assumption reported by companies. |
| Samsung | We support this in principle. The phrase “if SLS is adopted,” can be deleted, as this proposal can be applied to LLS-based evaluation too. |

4th round email discussions

### 2.7-1: Generic evaluation methodology: Baseline SLS EVM table-Traffic model

Combine the Proposal 2.5.2 into the ‘Traffic model’ entry of the baseline EVM table as follows. Two options are provided. The difference is: for Option 2, if the full buffer is selected, it can be also used for eventual performance evaluation rather than used for only comparing intermediate results in Option 1.

Proposal 2.7.1 For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, the ‘Traffic model’ in the baseline of EVM is captured as follows:

- Option 1 (combination of Proposal 2.5.2 with the original EVM):

| Traffic model | FTP model as a baseline FTP model 1 with packet size 0.5 Mbytes as a baseline Other FTP model is not precluded. FFS full buffer Full buffer can be optionally taken for the purpose of calibration and/or comparing intermediate results (e.g., accuracy of AI/ML output CSI, etc.) |
| --- | --- |

- Option 2 (combination of Proposal 2.5.2 with Intel’s proposal):

| Traffic model | FTP model as a baseline FTP model 1 with packet size 0.5 Mbytes as a baseline Other FTP model is not precluded. FFS full buffer Full buffer model is not precluded |
| --- | --- |

| Option 1 | Supporting companies | NVIDIA, InterDigital, LG, CAICT, OPPO, vivo, Ericsson, CMCC, DCM, CATT, ZTE, Xiaomi |
| --- | --- | --- |
|  | Objecting companies |  |
| Option 2 | Supporting companies | Futurewei, Qualcomm (comment below), Apple |
|  | Objecting companies |  |

| Company | View |
| --- | --- |
| OPPO | We prefer to use full buffer model for calibration, and the final conclusion should be drawn from FTP baseline. |
| vivo | We prefer, “Full buffer is considered as optional traffic model”. |
| Apple | What is exactly the difference between option 1 and 2 listed above? We support using FB as an alternative traffic model. |
| Qualcomm | We support option 2. To capture what is mentioned in the explanation just before the proposal above, we suggest the following change in the last line of option 2: Full buffer model is not precluded for eventual performance evaluation |
| Moderator | @Apple The difference is: for Option 2, if the full buffer is selected, it can be also used for eventual performance evaluation rather than used for only comparing intermediate results in Option 1. |
| ZTE | We prefer to agree with Option 1. Full buffer model can be optionally used for calibration, and the eventual evaluation should be performed from FTP model. |
| Xiaomi | We prefer Option 1 and also support option 2 with QC’s modification. |
| Samsung | Option 2 is reasonable. We concur with Intel and do not want to preclude FB from the eventual evaluation. |

### 2.7-2: LLS EVM table

A number of companies provided inputs to the LLS EVM table, so let’s see if it is OK for the following proposal:

Proposal 2.7.2 Question 2.5.1: For the evaluation of the AI/ML based CSI feedback enhancement, if LLS is preferred, the following table is taken as a baseline of EVM

- Note: the baseline EVM is used to compare the performance with the benchmark release, while the AI/ML related parameters (e.g., dataset construction, generalization verification, and AI/ML related metrics) can be of additional/different assumptions.

  - The conclusions for the use cases in the SI should be drawn based on generalization verification over potentially multiple scenarios/configurations.

- FFS: modifications on top of the following table for the purpose of AI/ML related evaluations.

- FFS: other parameters and values if needed

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | FDD (TDD is not precluded), OFDM |
| Carrier frequency | 2GHz as baseline, optional for 4GHz |
| Bandwidth | 10MHz or 20MHz |
| Subcarrier spacing | 15kHz for 2GHz, 30kHz for 4GHz |
| Nt | 32: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Nr | 4: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ |
| Channel model | CDL-C as baseline, CDL-A as optional |
| UE speed | 3 km/h or 30 km/h 3kmhr, 10km/h, 20km/h or 30km/h to be reported by companies |
| Delay spread | 30ns or 300ns |
| Channel estimation | FFS (same as SLS EVM) Realistic channel estimation algorithms (e.g. LS or MMSE) |
| RI Rank per UE | Rank 1-4 |

| Supporting companies | vivo, LG, CAICT, Samsung, OPPO, Panasonic, InterDigital, Fujitsu, Xiaomi, OPPO, Qualcomm, Ericsson, CMCC, DCM |
| --- | --- |
| Objecting companies |  |

| Company | View |
| --- | --- |
| OPPO | We are okay for this LLS table. From our view, we suggest to use 3km/h as a baseline for UE speed. And for rank per UE, we also prefer to firstly study Rank 1 for initial evaluation. |
| Qualcomm | We agree with the proposal. |
| Xiaomi | We prefer fixe RI=1 for initial evaluation. |
| Samsung | Thank you Moderator. For channel estimation we can consider the same approach as SLS and set it as - Realistic as a baseline - FFS ideal channel estimation |

### 2.7-3: Calibration of dataset and/or AI/ML model

Making Question 2.5.2 as the following proposal.

Proposal 2.7.3: For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, besides the eventual KPI (e.g., throughput), the intermediate KPI of the accuracy of AI/ML output CSI can be used for the calibration purpose

- FFS: Other metrics for calibration purpose

| Supporting companies | NVIDIA, Futurewei, caict, OPPO, MediaTek, vivo,Apple, Ericsson, CMCC, Lenovo, DCM, Fujitsu, Panasonic, CATT, ZTE, Xiaomi, Samsung |
| --- | --- |
| Objecting companies |  |

| Company | View |
| --- | --- |
| Moderator | @QC the calibration can be also understood as sort of AI/ML model/solution comparison [over companies]. |
| OPPO | We agree with this proposal. However, we also have a concern on the calibration. Considering the eType II performance reported by companies may be different, by whether the absolute intermediate KPI (e.g. SGCS) or the relative performance gap compared to eType II baseline is for calibration should be discussed. |
| Qualcomm | @Moderator: In our understanding, intermediate evaluation for AI/ML model performance comparison is different from calibration. Intermediate evaluation may not involve LLS or SLS and may instead only test the AI/ML model in a standalone manner to study and compare the performance using intermediate KPI such as SGCS. In contrast, the proposal above involves SLS. While we are in support of standalone AI/ML performance evaluation using intermediate KPIs (i.e., Proposal 2.7.4), we think calibration of the dataset across companies is not needed if the evaluation assumptions and parameters are agreed. |

### 2.7-4: Metrics

Intermediate KPIs

The following proposal is generated from Question 2.5.3, based on comments of the last round:

Proposal 2.7.4: For the evaluation of the AI/ML based CSI feedback enhancement, companies are encouraged can consider to perform standalone evaluation of AI/ML model simplified evaluation to evaluate intermediate KPI(s) for the purpose of AI/ML solution and/or sub use case comparison

- FFS: Parameter assumptions for the simplified evaluation standalone evaluation of AI/ML model

- Note: the simplified evaluation is not used for eventual performance comparison with the benchmark release and drawing SI conclusions should be based on eventual KPI(s).

A clean updated version based on QC and FUTUREWEI comments.

Upd Proposal 2.7.4: For the evaluation of the AI/ML based CSI feedback enhancement, companies can consider performing intermediate evaluation on AI/ML model performance to derive the intermediate KPI(s) for the purpose of AI/ML solution and/or sub use case comparison

- FFS: Parameter assumptions for the intermediate evaluation of AI/ML model performance

- Note: intermediate KPI(s) includes accuracy of AI/ML output CSI

- Note: the simplified evaluation is not used for eventual performance comparison with the benchmark release and drawing SI conclusions should be based on eventual KPI(s).

| Supporting companies | NVIDIA, Futurewei (with modification), CAICT, OPPO, MediaTek, vivo, Apple, Qualcomm (comment below), Ericsson, CMCC, Lenovo, DCM, Fujitsu, Panasonic, CATT, ZTE, Xiaomi, Samsung |
| --- | --- |
| Objecting companies |  |

| Company | View |
| --- | --- |
| Futurewei | In general, we are ok with proposal 2.7-4. We suggest rephrasing the main sentence to: For the evaluation of the AI/ML based CSI feedback enhancement, companies are encouraged can consider performing intermediate evaluation on AI/ML model performance simplified evaluation using the intermediate KPI(s) for the purpose of AI/ML solution and/or sub use case comparison. |
| OPPO | We agree with this proposal. |
| Qualcomm | We agree with the proposal with a few changes. We agree with Futurewei’s wording of the main sentence with one more small edit. Also, we prefer to remove the note. Intermediate KPIs may also provide useful insights and it is premature to determine how the intermediate KPIs may impact the study and conclusions. Hence, we propose the following version: “For the evaluation of the AI/ML based CSI feedback enhancement, companies can consider performing intermediate evaluation on AI/ML model performance to derive the intermediate KPI(s) for the purpose of AI/ML solution comparison. • FFS: Parameter assumptions for the intermediate evaluation on AI/ML model performance - Note: performance comparison with the benchmark release and drawing SI conclusions should be based on eventual KPI(s)” |
| Ericsson | What is the meaning of “eventual KPIs” in the note? The note is unclear. |
| Moderator | An updated version provided, based on QC and FUTUREWEI comments. @Ericsson: “eventual KPIs” means e.g., throughput as in proposal 2.7.3. A ‘Note: intermediate KPI(s) includes accuracy of AI/ML output CSI’ also added to interpret intermediate KPI |
| CATT | We are fine with the updated proposal. |
| ZTE | We generally agree with this proposal. However, we are not clear what Parameter assumptions in FFS means, please give a clear clarification. |
| Sasmung | Agree |

The metrics of intermediate KPI is changed based on QC’s comments.

Proposal 2.7.5 Proposal 2.5.3: For the evaluation of the AI/ML based CSI feedback enhancement, as a starting point, take the intermediate KPIs of GCS/SGCS and/or NMSE as part of the ‘Evaluation Metric’ to evaluate the accuracy of the AI/ML output CSI

- For GCS/SGCS,

  - FFS: how to calculate GCS/SGCS for rank>1

  - FFS: whether GCS or SGCS is adopted

- FFS other metrics, e.g., equivalent MSE, or received SNR, or spectral efficiency gap.

| Supporting companies | Apple, vivo, LG, CAICT, ZTE, MediaTek, Futurewei, DCM, Samsung, OPPO, Panasonic, CATT, Fujitsu, Xiaomi Huawei/Hisi, Qualcomm, Spreadtrum, Lenovo (comment below) , CMCC, NVIDIA, Samsung |
| --- | --- |
| Objecting companies |  |

| Company | View |
| --- | --- |
| Moderator | @ Lenovo Please refer to the 1st round Question 2.2.10 that a number of companies are choosing NMSE; and NMSE is well-suited for raw channel input processing |
| Qualcomm | We agree with the proposal. |
| Lenovo | @Moderator: Thanks for the explanation. We are actually okay that companies report NMSE as other metric but as you mentioned NMSE is a good metric for evaluating how well the complete CSI has been reconstructed. Based on the eventual goal of CSI feedback (e.g., throughout), the intermediate KPI should be more within the same line. So, our slightly modified text is: For the evaluation of the AI/ML based CSI feedback enhancement, as a starting point, take the intermediate KPIs of GCS/SGCS and optionally NMSE as part of the ‘Evaluation Metric’ to evaluate the accuracy of the AI/ML output CSI - For GCS/SGCS, - FFS: how to calculate GCS/SGCS for rank>1 - FFS: whether GCS or SGCS is adopted - FFS other metrics, e.g., equivalent MSE, or received SNR, or link throughput gap. |
| Samsung | Support |

Updates based on QC’s comment.

Upd Proposal 2.7.6 Proposal 2.5.4: For the evaluation of the AI/ML based CSI feedback enhancement, if the GCS/SGCS is adopted as the intermediate KPI as part of the ‘Evaluation Metric’, SGCS should be chosen.

- Note: wi is the vector of target CSI vector of frequency resource unit i, and wi is the vector of output CSI vector of frequency resource unit i. N is the total number of frequency resource units. E{∙} denotes the average operation over multiple samples.

- Note: Companies are encouraged to report the detail information of the vector of target CSI assumed in the evaluation.

SGCS=E1Ni=1N||wiHwi ||||wi||  ||wi||2

| Supporting companies | Vivo, CAICT, ZTE, MediaTek, Futurewei, DCM, Samsung, OPPO, Panasonic, Fujitsu, Xiaomi Huawei/Hisi, Qualcomm, Spreadtrum, Lenovo(with a modification) , CMCC, NVIDIA, Apple |
| --- | --- |
| Objecting companies |  |

| Company | View |
| --- | --- |
| Moderator | @ Lenovo I think you are actually choosing GCS which is the opposite option rather than making a modification. Would you reconsider to accept SGCS as the option? Please refer to ZTE’s comments that your ‘penalizing the smaller ¦¦wiHwi ¦¦¦¦wi¦¦ ¦¦wi¦¦due to the power of 2’ is actually the merit of SGCS (larger resolution of performance gap for clear comparison) |
| Qualcomm | We support the proposal but still have a concern with the wording. It would be useful to clarify whether this proposal applies only to rank 1 case with the eigen vector as output CSI type. For rank > 1, SGCS formula may need some discussion. The use of the term “vector” can be confusing. We prefer to remove “vector of” in the wording. |
| Moderator | @QC rank>1 case has been covered by Proposal 2.7.5. |
| Lenovo | @Moderator: Thanks for the explanation. We actually do not see how SGCS would result in higher resolution as we are adding all of ¦¦wiHwi ¦¦¦¦wi¦¦ ¦¦wi¦¦2, with some terms being penalized more than others. To us GCS is a more common and a more natural metric to select unless we see a good benefit of the SGCS. If we are missing some good property of SGCS, we would be happy to discuss that. We are okay to keep both metrics as well. Since this way we have the first moment E[X] and the second moment E[X2] of a random variable X=¦¦wiHwi ¦¦¦¦wi¦¦ ¦¦wi¦¦. But just keeping the second moment and disregarding the first moment of a non-zero mean X is not well justified in our opinion. Later, we still need to discuss the definition of intermediate KPI for rank>1 |
| CATT | We share similar views with Lenovo and do not see why SGCS results in higher resolution than GCS. But if the majority would like to use SCGS, we can live with it for easy comparison. |
| Samsung | Support. Support keeping both SGCS and GCS too. |

### 2.7-5: Methodology for generalization

Regarding the detailed methodology for generalization is still controversial over companies, a more generic principle is then provided as in the following proposal. Updates based on FUTUREWEI comment.

Upd Proposal 2.7.7: For the evaluation of the AI/ML based CSI feedback enhancement, companies are encouraged to report how to they verify the generalization of the AI/ML model, including:

- The training dataset of configuration(s)/ scenario(s), including potentially the mixed training dataset from multiple configurations/scenarios

- The testing/inference set of configuration(s)/ scenario(s)

- The detailed list of configuration(s) and/or scenario(s)

- Other details are not precluded

| Supporting companies | NVIDIA, Futurewei (with comments), InterDigital, LG, CAICT, OPPO, MediaTek, vivo, Apple, Qualcomm, Ericsson, CMCC, Lenovo, DCM, CATT, ZTE, Xiaomi(with comments), |
| --- | --- |
| Objecting companies |  |

| Company | View |
| --- | --- |
| Futurewei | In general, we are ok with proposal 2.7.7, which give companies the flexibility of verifying generalization. We suggest some wording changes in the main sentence: “For the evaluation of the AI/ML based CSI feedback enhancement, companies are encouraged to report how they verify the generalization of the AI/ML model, including:” In addition, we believe model generalization outcome for scenario-based solution deployment option should be included in the result as a base for the ease of understanding the generalization performance in the case results for other generalization options are also reported. Generalization for scenario-based option is to verify how a trained AI/ML model Futurewei performs using data unseen during the training phase from the same scenario as input. Note that scenario can mean a family of scenarios. |
| OPPO | We agree with this proposal. A general principle is a good starting point. |
| Qualcomm | We agree with the proposal. |
| ZTE | We think a generic principle is fine and agree with this proposal. |
| Xiaomi | We support this proposal with modification. Proposal 2.7.7: For the evaluation of the AI/ML based CSI feedback enhancement, companies are encouraged to report how to verify the generalization of the AI/ML model, including: - The training dataset of configuration(s)/ scenario(s), including potentially the mixed training dataset from multiple configurations/scenarios - The testing/inference dataset of configuration(s)/ scenario(s) - The detailed list of configuration(s) and/or scenario(s) - Other details are not precluded |
| Samsung | As it is laid out in Proposal 2.6.1 verification of generalization capability of an AI/ML model is essential part of the eventual evaluation. To highlight this point, we want to modify this proposal as: Proposal 2.7.7: For the evaluation of the AI/ML based CSI feedback enhancement, companies are encouraged to report study how to verify the generalization capability of the AI/ML model, including: - The training dataset of configuration(s)/ scenario(s), including potentially the mixed training dataset from multiple configurations/scenarios - The testing/inference set of configuration(s)/ scenario(s) - The detailed list of configuration(s) and/or scenario(s) - Other details are not precluded |

5th round email approval

### 2.8-1: Generic evaluation methodology: Baseline SLS EVM table-FFS part

With regard to the comments from Samsung, the 1st sub-bullet is changed accordingly. Please check if it is OK for companies.

Editorial updates based on FUTUREWEI comments.

Upd Proposal 2.8.1 Proposal 2.6.2: For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, the following parameters are taken into the baseline of EVM

- Note: The 2nd column applies if R16 TypeII codebook is selected as baseline, and the 3rd column applies if R17 TypeII codebook is selected as baseline.

  - FFS: Whether the selected baseline for performance evaluation is not related with impacts the R18 dataset construction and/or AI/ML model training

  - Same consideration with respect to utilizing angle-delay reciprocity should be taken for the AI/ML based CSI feedback and the baseline scheme if R17 TypeII codebook is selected as baseline

  - FFS baseline for potential sub use cases involving CSI enhancement on time domain

- Note: the baseline EVM is used to compare the performance with the benchmark release, while the AI/ML related parameters (e.g., dataset construction, generalization verification, and AI/ML related metrics) can be of additional/different assumptions.

  - The conclusions for the use cases in the SI should be drawn based on generalization verification over potentially multiple scenarios/configurations.

- FFS: modifications on top of the following table for the purpose of AI/ML related evaluations.

| Parameter | Value (if R16 as baseline) | Value (if R17 as baseline) |
| --- | --- | --- |
| Frequency Range | FR1 only, 2GHz as baseline, optional for 4GHz. | FR1 only, 2GHz with duplexing gap of 200MHz between DL and UL, optional for 4GHz |
| Simulation bandwidth | 10 MHz for 15kHz as a baseline, and configurations which emulate larger BW, e.g., same sub-band size as 40/100 MHz with 30kHz, may be optionally considered. Above 15kHz is replaced with 30kHz SCS for 4GHz. | 20 MHz for 15kHz as a baseline (optional for 10 MHz with 15KHz), and configurations which emulate larger BW, e.g., same sub-band size as 40/100 MHz with 30kHz, may be optionally considered. Above 15kHz is replaced with 30kHz SCS for 4GHz |
| MIMO scheme | SU/MU-MIMO with rank adaptation. Companies are encouraged to report the SU/MU-MIMO with RU | SU/MU-MIMO with rank adaptation. Companies are encouraged to report the SU/MU-MIMO with RU |
| Traffic load (Resource utilization) | 20/50/70% Companies are encouraged to report the MU-MIMO utilization. | 20/50/70% Companies are encouraged to report the MU-MIMO utilization. |

| Supporting companies | Vivo, Ericsson, CAICT, ZTE, MediaTek, Futurewei, NVIDIA, Samsung, OPPO, InterDigital, CATT, Fujitsu, Xiaomi Huawei/Hisi, Qualcomm, Spreadtrum, CMCC,Apple, Lenovo, DCM, Panasonic, Intel, LG, Nokia/NSB |
| --- | --- |
| Objecting companies |  |

| Company | View |
| --- | --- |
| Moderator | A Note ‘Same consideration with respect to utilizing angle-delay reciprocity should be taken for the AI/ML based CSI feedback and the baseline scheme’ added under the 1st bullet. |
| Futurewei | We suggest adding “if R17 TypeII codebook is selected as baseline” to the note to make it clear. - Same consideration with respect to utilizing angle-delay reciprocity should be taken for the AI/ML based CSI feedback and the baseline scheme if R17 TypeII codebook is selected as baseline. |
| Intel | Ok with update from Futurewei |
| CAICT | Fine with FL’s proposal. |
| OPPO | We are okay for this update. |
| vivo | We are fine with FL’s proposal |
| LG | Fine with FL’s propsal |
| Qualcomm | We agree with the proposal. |
| CATT | Fine with the proposal capturing Futurewei’s update. |
| CMCC | OK with the updated Proposal. |
| ZTE | We are fine with FL’s proposal. |
| Samsung | Support |
| Nokia/NSB | Ok with the proposal. The wording of the added yellow part can be clarified as - Additional assumptions from R17 TypeII EVM, with respect to utilizing angle-delay reciprocity, should be considered for the AI/ML based CSI feedback and the baseline scheme if R17 TypeII codebook is selected as baseline It may be worth capturing in Appendix II, for reference the other EVM assumptions that were agreed in Rel-17 CSI for partial reciprocity, copied below for reference: Agreement For EVM for FDD CSI enhancement in Rel-17, cluster delays and angles resulting from fast fading channel generation are the same for FDD DL and UL channels. Agreement For EVM for FDD CSI enhancement in Rel-17, use following Alt 1 as the baseline and Alt 2 as the optional - Alt 1: Based on Section 5.3 of TR 36.897, to generate FDD DL and UL channels. - Alt 2: Based on Section 7.6.5 of TR 38.901, to generate FDD DL and UL channels with following modifications: - Different per-cluster shadowing is generated for DL and UL, and DL (or UL) angles are generated based on DL (or UL) cluster powers. Then UL (or DL) uses the same angles and its own cluster powers to generate the channel matrix. - XPR is generated independently for DL and UL. Agreement For EVM for FDD CSI enhancement in Rel-17, using SRS error model in Table A.1-2 in 36.897 with Δ=9 dB. - Companies are encouraged to disclose SRS configuration parameters, if differently Agreement For EVM for FDD CSI enhancement in Rel-17, using the following calibration error model HUL=HUL⋅E=HUL⋅a1ejθ1⋯0⋮⋱⋮0⋯aNejθN - HUL is the spatial UL channel at gNB side with calibration error - HUL is the ideal spatial UL channel without calibration error - E represents the mismatch of transmission and reception circuits of gNB - ai is the amplitude error - θi is the phase error - N is the number of antennas at gNB side With amplitude error (expressed in decibel of x=20log10a) and phase error are normal distribution with 0.7dB and 5 degrees standard deviation, respectively. Both amplitude/phase errors are assumed to be constant during a simulation drop at time, and constant either across whole simulation bandwidth or per 4 PRB at frequency. Companies shall report the assumption of error modelling at frequency. Agreement For EVM for FDD CSI enhancement in Rel-17, companies are encouraged to describe general procedure with regarding to UE/UE group/cell-specific beamforming bases applied to CSI-RS ports/resources, spatial and/or Frequency domain precoding, CSI measurement behavior over beamformed CSI-RS, etc. for the sake of RAN1 discussion. - Note that whether there is spec impact is up to further RAN1 discussion. Agreement For EVM for FDD CSI enhancement in Rel-17, companies are encouraged to consider AP beamformed CSI-RS overhead used in both baseline and Rel-17 enhancement evaluation, either to be the same, or to be reported as averaged X CSI-RS ports per Y ms per cell during simulation runs if differently. |

### 2.8-2: Generic evaluation methodology: Baseline SLS EVM table-Channel estimation

Proposal 2.8.2 Proposal 2.6.3: For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, for ‘Channel estimation’, ideal DL channel estimation is optionally taken into the baseline of EVM for the purpose of calibration and/or comparing intermediate results (e.g., accuracy of AI/ML output CSI, etc.)

- Note: Eventual performance comparison with the benchmark release and drawing SI conclusions should be based on realistic DL channel estimation.

- FFS: the ideal channel estimation is applied for dataset construction, or performance evaluation/inference.

- FFS: How to model the realistic channel estimation

- FFS: Whether ideal channel is used as target CSI for intermediate results calculation with AI/ML output CSI from realistic channel estimation

| Supporting companies | Apple, LG, CAICT, ZTE, MediaTek, Futurewei, NVIDIA, DCM, Samsung, OPPO, Panasonic, CATT, Fujitsu, Xiaomi, Huawei/Hisi, Qualcomm, Spreadtrum, Lenovo, CMCC, Ericsson, Intel, Nokia/NSB |
| --- | --- |
| Objecting companies |  |

| Company | View |
| --- | --- |
| Moderator | ‘if SLS is adopted’ removed to be consistent over SLS/LLS, as per Samsung comment |
| Samsung | Support |

### 2.8-3: Baseline LLS EVM table

Some updates based on OPPO, Xiaomi, Samsung comments.

Upd Proposal 2.8.3 Proposal 2.7.2: For the evaluation of the AI/ML based CSI feedback enhancement, if LLS is preferred, the following table is taken as a baseline of EVM

- Note: the baseline EVM is used to compare the performance with the benchmark release, while the AI/ML related parameters (e.g., dataset construction, generalization verification, and AI/ML related metrics) can be of additional/different assumptions.

  - The conclusions for the use cases in the SI should be drawn based on generalization verification over potentially multiple scenarios/configurations.

- FFS: modifications on top of the following table for the purpose of AI/ML related evaluations.

- FFS: other parameters and values if needed

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | FDD (TDD is not precluded), OFDM |
| Carrier frequency | 2GHz as baseline, optional for 4GHz |
| Bandwidth | 10MHz or 20MHz |
| Subcarrier spacing | 15kHz for 2GHz, 30kHz for 4GHz |
| Nt | 32: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Nr | 4: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ |
| Channel model | CDL-C as baseline, CDL-A as optional |
| UE speed | 3kmhr, 10km/h, 20km/h or 30km/h to be reported by companies |
| Delay spread | 30ns or 300ns |
| Channel estimation | Realistic channel estimation algorithms (e.g. LS or MMSE) as a baseline, FFS ideal channel estimation |
| RI Rank per UE | Rank 1-4. Companies are encouraged to report the Rank number, and whether/how rank adaptation is applied |

| Supporting companies | vivo, LG, CAICT, Samsung, OPPO, Panasonic, InterDigital, Fujitsu, Xiaomi, OPPO, Qualcomm, Ericsson, CMCC, DCM, Nokia/NSB |
| --- | --- |
| Objecting companies |  |

| Company | View |
| --- | --- |
| Moderator | @OPPO @Xiaomi ‘Companies are encouraged to report the Rank number’ Generally we should not restrict the rank number in EVM. |
| Intel | “Rank 1-4” – does it mean rank adaptation is done or does it mean rank adaptation may or may not be done in LLS ? |
| Moderator | ‘Companies are encouraged to report the Rank number’ – In my understanding, whether rank adaptation or fixed rank is per companies report. Updates to clarify. |
| Samsung | Support |

### 2.8-4: Metrics – Intermediate KPIs

Proposal 2.8.4 Proposal 6.3.8: For the evaluation of the AI/ML based CSI feedback enhancement, companies can consider performing intermediate evaluation on AI/ML model performance to derive the intermediate KPI(s) (e.g., accuracy of AI/ML output CSI) for the purpose of AI/ML solution comparison

- FFS: Parameter assumptions for the intermediate evaluation of AI/ML model performance

- Note: intermediate KPI(s) includes accuracy of AI/ML output CSI

- Note: eventual performance comparison with the benchmark release and drawing SI conclusions should be based on eventual KPI(s).

| Supporting companies | NVIDIA, Futurewei (with modification), CAICT, OPPO, MediaTek, vivo, Apple, Qualcomm, Ericsson, CMCC, Lenovo, DCM, Fujitsu, Panasonic, CATT, ZTE, Xiaomi, Samsung, Intel, LG, Nokia/NSB |
| --- | --- |
| Objecting companies |  |

| Company | View |
| --- | --- |
| Moderator | Please see the updated text |
| Qualcomm | We agree with the proposal. |
| Samsung | Support |

Proposal 2.8.5 Proposal 2.7.5: For the evaluation of the AI/ML based CSI feedback enhancement, as a starting point, take the intermediate KPIs of GCS/SGCS and/or NMSE as part of the ‘Evaluation Metric’ to evaluate the accuracy of the AI/ML output CSI

- For GCS/SGCS,

  - FFS: how to calculate GCS/SGCS for rank>1

  - FFS: whether GCS or SGCS is adopted

- FFS other metrics, e.g., equivalent MSE, received SNR, or throughput/spectral efficiency gap.

| Supporting companies | Apple, vivo, LG, CAICT, ZTE, MediaTek, Futurewei, DCM, Samsung, OPPO, Panasonic, CATT, Fujitsu, Xiaomi Huawei/Hisi, Qualcomm, Spreadtrum, Lenovo , CMCC, NVIDIA, Samsung, Intel |
| --- | --- |
| Objecting companies |  |

| Company | View |
| --- | --- |
| Moderator | @ Lenovo The ‘throughput’ part captured. For the ‘Optionally NMSE’ part, however, I believe companies can freely choose the metric based on/matched with their preferred AI/ML model and CSI input, so the intention here is no to restrict which metric is mandatory while the other is optional – we could be more inclusive at the start of the SI. Could you live with that? |
| CATT | Not sure why ‘throughput/spectral efficiency gap’ is part of intermediate KPIs to evaluate the accuracy of the AI/ML output CSI. It seems to be a further comparison based on eventual KPI. But since it is just a possible example under ‘e.g.,…’, we are OK with the proposal to save some time. |
| ZTE | We are not sure why ‘throughput/spectral efficiency gap’ is part of intermediate KPIs to evaluate the accuracy of the AI/ML output CSI. It seems to be included in eventual KPIs. |
| Qualcomm | On the above comment from CATT, ZTE: We agree that throughput should be an eventual KPI and not an intermediate KPI, as it depends on several aspects beyond the AI/ML model output (e.g., scheduling). However, the spectral efficiency (SE) achievable can be computed based on the AI/ML model’s output CSI and the true raw channel. SE gap between SE of the output CSI and the ideal SE (of the target CSI itself) could then be considered as a measure of AI/ML model accuracy similar to SGCS and is therefore an intermediate KPI: SE gap = SE(wi, Hi) – SE(wi, Hi), averaged across resource units. where, for resource unit i, wi is the target CSI (e.g., eigenvectors), and wi is the output CSI, and Hi is the raw channel matrix. In fact, such a metric could address some of the concerns regarding SGCS not reflecting the actual performance. Also, it naturally extends to the case of rank > 1. Hence we propose the following change (i.e., remove “throughput”): For the evaluation of the AI/ML based CSI feedback enhancement, as a starting point, take the intermediate KPIs of GCS/SGCS and/or NMSE as part of the ‘Evaluation Metric’ to evaluate the accuracy of the AI/ML output CSI - For GCS/SGCS, - FFS: how to calculate GCS/SGCS for rank>1 - FFS: whether GCS or SGCS is adopted FFS other metrics, e.g., equivalent MSE, received SNR, or throughput/spectral efficiency gap. |
| Lenovo | We agree with the proposal. @Moderator: by throughput we actually meant link throughput (as intermediate KPIs) which can better show the performance (compared with GCS) and it is also straight forward to derive. It should resolve the concern that other companies have. |
| Samsung | Support |
|  | Generally fine with the proposal, but we agree that throughput and actual SE (measured from SLS) are not really intermediate KPIs. We suggest capturing QC’s suggestion as follows, for clarity: FFS other intermediate metrics, e.g., MSE, SNR, estimated spectral efficiency, etc. |

Some companies are still not happy with SGCS prioritize GCS. Then let’s try to see if supporting both based on companies’ report can be acceptable.

Proposal 2.8.6 Proposal 6.3.10: For the evaluation of the AI/ML based CSI feedback enhancement, if the GCS/SGCS is adopted as the intermediate KPI as part of the ‘Evaluation Metric’, companies to report whether SGCS or GCS is adopted SGCS should be chosen.

- Note: wi is the vector of target CSI of resource unit i, and wi is the vector of output CSI of resource unit i. N is the total number of resource units. E{∙} denotes the average operation over multiple samples.

- Note: Companies are encouraged to report the detail information of the vector of target CSI assumed in the evaluation.

GCS=E1Ni=1N||wiHwi ||||wi||  ||wi||,  SGCS=E1Ni=1N||wiHwi ||||wi||  ||wi||2

| Supporting companies | Intel, vivo, Qualcomm, CATT, CMCC, Fujitsu, Lenovo, Samsung, Nokia/NSB |
| --- | --- |
| Objecting companies |  |

| Company | View |
| --- | --- |
| OPPO | From our view, if GCS/SGCS are both included, the intermediate KPI calibration may be more complicated since the transformation between GCS and SGCS is not very direct. |
| ZTE | We are fine with either or both for progress. However, we prefer using a common KPI for evaluation since it is convenient for calibration. |
| Lenovo | We agree with the proposal. |

### 2.8-5 Capability/complexity related KPIs

The following proposals have been mostly stable in 1st/2nd round, but as 9.2.1 has not treated such issues, and we may not have GTW to endorse it, so try to also bring them for email approval here.

Proposal 2.8.7 Proposal 6.3.12: For the evaluation of the AI/ML based CSI feedback enhancement, Floating point operations (FLOPs) is adopted as part of the ‘Evaluation Metric’, and reported by companies.

| Supporting companies | Xiaomi, vivo Huawei/Hisi, CAICT, MediaTek, Ericsson, CMCC, Nokia/NSB, NVIDIA, InterDigital, Fraunhofer, Charter, Futurewei, Lenovo, LG, Panasonic, CATT, ZTE, Samsung Fujitsu Qualcomm Apple OPPO, Intel, Beijing Jiaotong University, AT&T, Intel |
| --- | --- |
| Objecting companies |  |

| Company | View |
| --- | --- |

Proposal 2.8.8 Proposal 6.3.13: For the evaluation of the AI/ML based CSI feedback enhancement, AI/ML memory storage in terms of AI/ML model size and number of AI/ML parameters is adopted as part of the ‘Evaluation Metric’, and reported by companies who may select either or both.

- FFS: the format of the AI/ML parameters

| Supporting companies | Apple, OPPO, Samsung, Beijing Jiaotong University, LG Huawei/Hisi, Spreadtrum, CMCC, vivo, CATT, CAICT, Futurewei, Fujitsu, Xiaomi, NVIDIA, ZTE, MediaTek, Nokia/NSB, Lenovo, InterDigital, Intel, Qualcomm |
| --- | --- |
| Objecting companies |  |

| Company | View |
| --- | --- |
| Qualcomm | We agree with the proposal. |
| Samsung | Support |

### 2.8-6: Methodology for generalization

Change the first sentence by Samsung’s suggestion. I guess to study the verification is the common understanding.

Upd Proposal 2.8.9 Proposal 6.3.7: For the evaluation of the AI/ML based CSI feedback enhancement, study the verification of generalization. Ccompanies are encouraged to report how they verify the generalization of the AI/ML model, including:

- The training dataset of configuration(s)/ scenario(s), including potentially the mixed training dataset from multiple configurations/scenarios

- The testing/inference set of configuration(s)/ scenario(s) for testing/inference

- The detailed list of configuration(s) and/or scenario(s)

- Other details are not precluded

| Supporting companies | NVIDIA, Futurewei (with comments), InterDigital, LG, CAICT, OPPO, MediaTek, vivo, Apple, Qualcomm, Ericsson, CMCC, Lenovo, DCM, CATT, ZTE, Xiaomi(with comments),Intel, Nokia/NSB |
| --- | --- |
| Objecting companies |  |

| Company | View |
| --- | --- |
| Moderator | @Xiaomi for your comment, ‘The testing/inference dataset’ – the testing/inference is a set of scenario(s) for applying the AI/ML model but NOT a dataset of samples/data. The original text is kept. |
| Xiaomi | Thanks for Moderator’s response. Now we understand your intention is a set of scenarios, but it is still not clear to us what’s the different between training and testing in the evaluation, both of them need data/samples in our understanding. Could you please clarify that? We suggest to have an aligned description for training and testing/inference, e.g., The testing/inference dataset of configuration(s)/ scenario(s), including potentially the mixed testing/inference dataset from multiple configurations/scenarios. |
| Moderator | @Xiaomi for testing, yes, it may be a dataset (independently generated with training dataset), but for inference, it is performed without collecting a dataset, right? Both ways can equally evaluate the generalization, so both are reserved in the proposal. In addition, not clear how the testing/inference can be with ‘mixed’ set – the AI/ML is applied once only for a single scenario/configuration, right? Updates as per your comments, hope that can avoid the terminology issue. |
| Xiaomi | Thanks for the follow up and explanation, we are fine with the update. |
| Samsung | Ok. |

5th round email discussion

### 2.9-1: Generic evaluation methodology: Baseline SLS EVM table-Traffic model

Proposal 2.7.1 For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, the ‘Traffic model’ in the baseline of EVM is captured as follows:

- Option 1 (combination of Proposal 2.5.2 with the original EVM):

| Traffic model | FTP model as a baseline FTP model 1 with packet size 0.5 Mbytes as a baseline Other FTP model is not precluded. FFS full buffer Full buffer can be optionally taken for the purpose of calibration and/or comparing intermediate results (e.g., accuracy of AI/ML output CSI, etc.) |
| --- | --- |

- Option 2 (combination of Proposal 2.5.2 with Intel’s proposal):

| Traffic model | FTP model as a baseline FTP model 1 with packet size 0.5 Mbytes as a baseline Other FTP model is not precluded. FFS full buffer Full buffer model is not precluded |
| --- | --- |

| Option 1 | Supporting companies | NVIDIA, InterDigital, LG, CAICT, OPPO, vivo, Ericsson, CMCC, DCM, CATT, ZTE, Xiaomi, Nokia/NSB, MediaTek |
| --- | --- | --- |
|  | Objecting companies |  |
| Option 2 | Supporting companies | Futurewei, Qualcomm, Apple, Intel, Samsung |
|  | Objecting companies |  |

| Company | View |
| --- | --- |
| OPPO | We prefer to use full buffer model for calibration, and the final conclusion should be drawn from FTP baseline. |
| vivo | We prefer, “Full buffer is considered as optional traffic model”. |
| Apple | What is exactly the difference between option 1 and 2 listed above? We support using FB as an alternative traffic model. |
| Qualcomm | We support option 2. To capture what is mentioned in the explanation just before the proposal above, we suggest the following change in the last line of option 2: Full buffer model is not precluded for eventual performance evaluation |
| Moderator | @Apple The difference is: for Option 2, if the full buffer is selected, it can be also used for eventual performance evaluation rather than used for only comparing intermediate results in Option 1. |
| ZTE | We prefer to agree with Option 1. Full buffer model can be optionally used for calibration, and the eventual evaluation should be performed from FTP model. |
| Xiaomi | We prefer Option 1 and also support option 2 with QC’s modification. |
| Samsung | Option 2 is reasonable. We concur with Intel and do not want to preclude FB from the eventual evaluation. |
| Moderator | @Companies Please keep inputing your views, and also type in your company to ‘Objecting companies’ if you have strong concern. I will based on number/comments of supporting companies and objecting companies to make a further proposal. Raising of any compromise solution is welcome! @QC The main text of the baseline EVM table has noted ‘Note: the baseline EVM is used to compare the performance with the benchmark release’ which already includes that. |
| Intel | Option 2 includes Option 1, but option 1 precludes FB. When option-1 says FB is optionally used for calibration, does it mean that FTP model is used for calibration? |
| Moderator | @Intel There is no restriction for FTP model – it can be used for any purpose (performance comparison, or calibration). |
| CAICT | We have no strong view on option 1 and 2 and the two options are not mutual excusive. In general, Option 1 is better for us. |
| Qualcomm | We support Option 2. @Moderator: Thank you for the clarification. |
| CMCC | We prefer Option 1 and Option 2 is also fine for us. |

### 2.9-2: Metrics – Intermediate KPIs

A remaining issue in the 2nd round discussion is raised again here, related with the GCS/SGCS calculation for rank>1 cases. As the preference of the following 3 Options are quite diverse, an open proposal is then given in below, to let companies report their preferred GCS/SGCS calculation methods.

Proposal 2.9.2 Question 2.3.2: For the evaluation of the AI/ML based CSI feedback enhancement, if the GCS/SGCS is adopted as the intermediate KPI as part of the ‘Evaluation Metric’, for rank>1 cases, companies to report the GCS/SGCS calculation methods, including:

- Option 1: Average over all ranks

  - Note: wij is the jtheigenvector at resource unit i and K is the rank of the channel

GCS=E1Ni=1NjK||wijHwij ||||wij||  ||wij||,  SGCS=E1Ni=1NjK||wijHwij ||||wij||  ||wij||2

- Option 2: Weighted average over all ranks

  - Companies to report the formula (e.g., whether normalization is applied for eigienvalues)

- Option 3: GCS/SGCS is separately calculated for each rank (e.g., for K ranks, K GCS values are derived respectively)

- Option 4: Other

| Supporting companies | Futurewei, CAICT, OPPO, Qualcomm, CATT, CMCC, ZTE, Fujitsu, MediaTek |
| --- | --- |
| Objecting companies |  |

| Company | View |
| --- | --- |
| Moderator | For Option 2: Weighted average over all ranks, an example of GCS calculated based on non-normalized eigienvalues is: GCS=E1Ni=1NjKλij¦¦wijHwij ¦¦¦¦wij¦¦ ¦¦wij¦¦ An example of GCS calculated based on normalized eigienvalues is: GCS=E1Ni=1NjKλijj=1Kλij¦¦wijHwij ¦¦¦¦wij¦¦ ¦¦wij¦¦. |
| OPPO | We agree with this proposal. We also suggest to use Option 1 as a baseline, which form of Option 2 is more effective to represent the performance of CSI recovery accuracy can be FFS. However, we prefer not to have multiple kinds of KPIs, so that companies are more difficult to calibrate. |
| vivo | We prefer to take Option-1 as a baseline because introducing new parameter leads more divergence. In addition, we are not sure of its benefit. |
| Qualcomm | We agree with the proposal. |
| CATT | OK. One small typo, ‘eigienvalues’ should be ‘Eigenvalues’. |
| CMCC | We support this proposal and prefer Option 1 as the baseline. |
| ZTE | We agree with this proposal and prefer to use Option 1 as a baseline. As intermediate evaluation metrics, we think there is no need to adopt multiple different KPIs to evaluate, since extra workload may be introduced for companies to calibrate. |
| Lenovo | We believe, Option 1 is not a good metric since it gives same importance to all eigenvectors which is not correct and in practice eigenvectors corresponding to small eigne-values are not useful for scheduling purposes. Note that by giving same importance even to less useful eigenvectors, we are limiting the possibility of CSI compression. For option 3, we are unclear how GCS/SGCS vectors (one value for each rank) will be compared between two models. For option2, we are in favor of the previous version of weighted GCS as GCS=E1Ni=1NjKλij¦¦wijHwij ¦¦¦¦wij¦¦ ¦¦wij¦¦ ** why not normalized eigenvalues: Note that when we compute eigenvectors we assume that averaged power of the channel realizations are normalized (not each individual realization of H which will be the case if we divide the eigne vectors by their sum). We may also point out that the “Expectation” operation in the above formulation will take care of variations the norm of different channel realizations and also estimation error. And, in case of ideal channel estimation GCS will be normalized as expected by Qualcomm since it will be reduced to: GCS=E1Ni=1NjKλij¦¦wij ¦​2 |
| Samsung | Option two (with normalized Eigenvalues) is reasonable as it immolates user throughput. |
| Nokia/NSB | We can live with this proposal. However, it would be preferable to consider these options for down-selection at the next RAN1 meeting. This would allow companies to compare the different options and see which seems to be the best indicator for higher rank. |
| MediaTek | Support in principle. Option 2 is preferred. |

6th round email discussion

### 2.10-1: Calibration of dataset and/or AI/ML model

In the 1st round discussions, besides Option1 (aligning parameters), some companies are also in favor of Option 2/3/4 for the calibration of dataset and/or AI/ML models in Question 2.2.9.

Question 2.10.1: For the evaluation of the AI/ML based CSI feedback, besides aligning parameters, do you think it is necessary to additionally adopt any of the following options to calibrate over companies? Please elaborate how to implement your preferred option in the 3GPP level.

- Option 2: Share the dataset and/or model files to public for cross check over companies

- Option 3: Generate a common dataset over companies

- Option 4: Align the AI/ML model over companies

| Option 2 | Support/Can accept | OPPO, vivo, CAICT |
| --- | --- | --- |
|  | Object/Concern | Ericsson, Nokia/NSB, Qualcomm |
| Option 3 | Support/Can accept | OPPO, CATT, LG, Xiaomi, vivo, CAICT, NVIDIA, Intel, Lenovo |
|  | Object/Concern | Ericsson, Nokia/NSB, Qualcomm, Samsung |
| Option 4 | Support/Can accept | OPPO, LG, vivo, CAICT, Intel |
|  | Object/Concern | Ericsson, Qualcomm, Samsung |
| Not necessary | Support/Can accept | Ericsson Huawei/Hisi, DCM, Fujitsu, Futurewei (can accept Option 3), Nokia/NSB, InterDigital, Lenovo, Qualcomm |
|  | Object/Concern | Futurewei (object Option 2 and Option 4) |

| Company | View |
| --- | --- |
| Moderator | Please elaborate how to implement your preferred option in the 3GPP level if you prefer Option 2/3/4. E.g., for Option 3/4, who/how to generate the common dataset/AI model; for Option 2/3/4, where the dataset/AI model is uploaded, and how the dataset/AI model is maintained, etc. |
| OPPO | We think this is good proposal. We support both Option 2/3/4. For option 2, companies are encouraged to provide their datasets and AI/ML models through papers and data platforms, and then FL can conclude these options for discussion. For option 3/4, we can accept these two methods, but a reasonable scheme should be further studied and discussed. |
| Ericsson | We don’t see the need for this additional work. Besides the questionable necessity, it will consume a huge amount of time in RAN1 to align the structure of representing data files. We would also need to set up a 3GPP server for this task as external servers cannot be used due to legal reasons. The benefit or value of proceeding with these suggested tasks are not clear to us. Aligning parameters as in Option 1 is sufficient. |
| CATT | As we already agreed to align the parameters, we consider all options here are optional. Among them, since AI model design of different companies is quite sensitive, perhaps we can at most try to build a common data set. |
| LG | We think option 3 and 4 seem sufficient for the calibration purpose. |
| Xiaomi | We believe a common dataset would be beneficial for cross check. For how to conduct a common dataset, we think each company can upload the channel dataset generated based on TR 38.901 as agreed for each scenario/configuration, e.g., 1 million channel matrix for UMa. A tool can be used to mix all the dataset uploaded by different companies for one specific scenario/configuration and generate e.g., 1 million channel matrix randomly. For different scenario/configuration, a tool can be used to mix the different scenario/configuration among mixed dataset of different companies. Whether align AI model can be decided in the next meeting based on companies’ evaluation results. |
| vivo | MIMO related evaluations are conducted without performance alignment, resulting in a large deviation between companies. To tackle such an issue, a dataset and AI/ML model should be commonly aligned, which does not cost too much burden. For research purpose, we share our dataset for AI-based CSI feedback and CSI prediction through the link [5] and [6] of R1-2203550, respectively. |
| ZTE | We think Option 1 is enough for dataset calibration and no need to adopt other options for calibration. Since we ultimately focus on the system performance, which relies on each company’s own SLS platform and other datasets may not match. In addition, these options depend on the sub use cases for various dataset generation, which may bring extra workload. To Option 4, we think it is not necessary for companies to align the AI model. |
| Huawei/Hisi | Option 1 is the simplest way for 3GPP. Option 3 and 4 will introduce addition work load. Option 2 can be voluntarily performed by companies, but no action is needed in 3GPP level. |
| NTT DOCOMO | Companies can report the Option2/3/4 information voluntarily. As some companies view AI/ML model information as an intellectual asset, it looks difficult to mandate Option2/3/4 for calibration. |
| Fujitsu | We think that it is helpful to align AI/ML models, however, it is difficult to do that. |
| CAICT | We support option 2/3/4. the progress of double check between different companies is necessary even with synthetic dataset. Companies could share their dataset and/or AI models on public platform for download and cross check and provide the download address. |
| Futurewei | By aligning parameters in generating dataset (Option 1) is easier for companies to conduct the study and is sufficient. We understand common dataset may be ideal given AI/ML is data-driven; thus, we are willing to consider generating a common dataset based on agreed scenarios/assumptions/parameters if companies reach consensus. This, however, this requires additional discussions. Option 2: we can accept sharing datasets but not sharing model files. Option 4: there is no need to align AI/ML model across companies. Each company should be given the flexibility of choosing their own model architecture and hyper-parameter settings. |
| CMCC | Option 2/3/4 can help people align their evaluation results more efficiently. Companies can volunteerly offer their dataset and AI model for cross check. |
| Intel | We think 3, 4 could be the starting point – for logistical aspects of constructing a common data-set we can take advice from MCC |
| Nokia/NSB | Similar concerns as expressed by Ericsson |
| Lenovo | We support Option4 In principle, we are okay with Option 3 especially if the dataset can be constructed from filed samples is very desirable. Sharing of such dataset could be through some public repositories that everyone can have access to. The main complexities which are FFS are how we can make sure that the dataset has sufficient statistics about different possibilities that could happened in the network. |
| Qualcomm | Since it has been agreed to align parameters and to use TR 38.901 models, we do not see a need for further calibration. Sharing the dataset or using a common dataset may not be easy in practice. Besides, for SLS-based evaluation, it is not clear how a common dataset generated for a certain drop and a certain time duration can be used in a different simulation for a different drop, duration, etc. Considering the workload to study the AI/ML performance and generalization aspects, defining and agreeing the procedure for such aspects seems unnecessary. Choice of the AI/ML model should be left to companies to make sure that different techniques are explored in the study. Intermediate evaluation of AI/ML model accuracy can be used to compare AI/ML model performance across companies. |
| Samsung | We share the same concern as Ericsson. |

### 2.10-2: Baseline SLS EVM table-Traffic model

From the 5th round discussions, it is still controversial between Option 1 and Option 2 for Proposal 2.7.1. Then Moderator tries in the 6th round to find a middle ground between the two options, with changes on top of the Option 2 in Proposal 2.7.1.

Proposal 2.10.1 Proposal 2.7.1 For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, the ‘Traffic model’ in the baseline of EVM is captured as follows:

| Traffic model | FTP model as a baseline FTP model 1 with packet size 0.5 Mbytes Other FTP model is not precluded. Full buffer model is not precluded. FFS: whether full buffer is taken only for the purpose of calibration and/or comparing intermediate results (e.g., accuracy of AI/ML output CSI, etc.), or also for eventual performance comparison with the benchmark release and drawing SI conclusions. |
| --- | --- |

| Support/Can accept | OPPO, CATT, LG, Xiaomi, vivo Huawei/Hisi, DCM, CAICT, NVIDIA, Futurewei, CMCC, Intel, Apple, Lenovo, Qualcomm, Samsung |
| --- | --- |
| Object/Concern | Ericsson, Nokia/NSB (FFS only), InterDigital |

| Company | View |
| --- | --- |
| Moderator | The ‘FFS’ part may be decided in the next meeting after companies analyze the pros/cons of the full buffer traffic model with potentially evaluation results for both. |
| OPPO | We agree to consider full buffer model for comparing eventual performance. From our understanding, the accuracy of AI/ML output CSI is only determined by the dataset and AI/ML model, whether FTP or full buffer model is utilized will not affect the intermediate results. |
| Ericsson | Full buffer, if used, can only be used for calibration purpose and intermediate results, it cannot be used for decision making on the feasibility of the feature and for making SI conclusions. |
| CATT | We think full buffer model can be optional supported. |
| LG | We agree with Ericsson, it can be used for calibration purpose. But, we can live with this proposal. |
| Xiaomi | For the FFS part, we prefer only use full buffer for intermediate calibration. |
| vivo | So far, both full buffer and FTP are commonly used in EVM (TR 36.897, TR 38.802). In general, they offer the performance evaluation results (e.g., throughput) from different angles. If we only take FTP as a traffic model, we are afraid that the final performance gain may be dependent on the traffic. Moreover, this is the first SI where the AI-based technologies are involved, and we are not aware of the exact behaviors of the AI occurring with CSI-feedback mechanism. The evaluation results based on the full buffer traffic could give us more information to judge the benefits and future direction according to company’s own purposes and willingness. |
| ZTE | We think full buffer model model can be optionally taken for the purpose of calibration as Option 1 in last round. Full buffer model is not precluded. Full buffer can be optionally taken for the purpose of calibration and/or comparing intermediate results (e.g., accuracy of AI/ML output CSI, etc.) Since in previous release, FTP models are adopted and we can reuse them as baseline, no matter FTP 1 or other FTP models, for eventual performance evaluation among companies. |
| Huawei/Hisi | Support. |
| CAICT | We think both FTP and full buffer traffic model could be used for performance justification. For simplicity, full buffer could be used for calibration. For eventual performance comparison with the benchmark release and drawing SI conclusions, we have no strong views. |
| NVIDIA | Full buffer traffic model may provide additional insights. |
| Intel | Good to have both FB and FTP captured in TR – FL proposal is not mandating FB results from companies while not limiting such contribution. |
| Nokia/NSB | Our preference is to keep full buffer for intermediate results only |
| InterDigital | Full buffer should be only for calibration/intermediate results only. It doesn’t provide any insight for the benefit of AI/ML in realistic scenarios. |
| Qualcomm | We support the approach that at this point full-buffer evaluations should not be precluded and can be agreed in the EVM table. Whether to use the full-buffer results in the eventual performance comparison can be discussed at a later stage. In our view, full-buffer results provide useful insights on the system capacity and the system performance at high load level. |
| Samsung | In our view, it is beneficial to consider full-buffer model as additional alternative to FTP Model 1 in the eventual evaluation and comparison of schemes. As some companies mentioned it, in some cases, evaluation based on FTP models may not give a clear picture of the gain/loss from AI/ML based CSI feedback. In this regard, full buffer model based evaluations and comparisons are good alternative. |

### 2.10-3: Baseline SLS EVM table-Channel estimation

In the 3rd the realistic channel estimation, as raised by vivo, there are at least two alternatives to model such an error, wherein one of them may be aligned by companies.

- Alt-1 is to directly generate the DL channel response matrix with an error, i.e., H’=H+e(SINR), where e(SINR) is the error term as a function of SINR received from the DL.

- Alt-2 is to estimate the channel based on the estimation algorithm, either MMSE or ZF.

Therefore, a question is raised in the 6th round on whether it is needed to align the channel estimation methods over companies, and if so, which method should be adopted.

Question 2.10-2: For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, for ‘Channel estimation’, do you think there is a need to align the error modeling method if realistic channel estimation is adopted?

- Option 1: Generate the DL channel response matrix with an error, i.e., H’=H+E(SINR), where E(SINR) is the error matrix as a function of DL SINR, H’ is estimated channel matrix and H is the real channel matrix

- Option 2: Generate the DL channel response matrix based on realistic channel estimation algorithm

  - FFS: detailed channel estimation algorithm, e.g., LMMSE, LS, etc.

- Option 3: No need to align the error modeling for realistic channel estimation

- Option 4: Other

| Option 1 | Support/Can accept | OPPO, CATT, LG, Xiaomi, vivo, ZTE Huawei/Hisi, DCM. CAICT, NVIDIA, CMCC, Intel, Apple, Lenovo |
| --- | --- | --- |
|  | Object/Concern |  |
| Option 2 | Support/Can accept | OPPO |
|  | Object/Concern |  |
| Option 3 | Support/Can accept | Ericsson Huawei/Hisi, Futurewei, Nokia/NSB, InterDigital, Apple, Lenovo, Qualcomm, Samsung |
|  | Object/Concern | OPPO |
| Option 4 | Support/Can accept |  |
|  | Object/Concern |  |

| Company | View |
| --- | --- |
| OPPO | We support Option 1 and Option 2 to align the error modeling method since it may greatly influence the eventual performance. |
| Ericsson | It seems RAN1 have not aligned error models in previous work item EVMs such as MIMO, eMIMO, feMIMO, so it is up to each company to model the channel estimation errors. No need to spend precious SI time to align on an “3GPP error model”. |
| CATT | Yes, and prefer Option 1. |
| LG | Option 1 is usually utilized for the channel estimation in SLS. So, we prefer to adopt simple approach such as Option 1.Also, in option 1, the error matrix can be reported by companies. |
| vivo | We need to define a simplified channel estimation mechanism to reduce the AI-based system level simulation burden. |
| ZTE | We think Option1 can be used as baseline for simplicity, while Option2 is hard to align the corresponding details. |
| Huawei/Hisi | We think this can be left for simulation implementation and not necessary to be aligned. If it is really needed, we prefer option 1. Modeling the link level algorithm of LMMSE/LS in system level is too complex and causes huge consumption of computation resources. |
| CAICT | Channel estimation algorithm used for AI/ML model training and SLS should be aligned. Option 1 is preferred. We share the same view from HW that modeling link level channel estimation in SL is difficult. |
| NVIDIA | Realistic channel estimation is typically up to companies. If the purpose is to align the error modeling, Option 1 appears to be more feasible. |
| Futurewei | We think this can be left to companies. |
| CMCC | We prefer Option 1. It is simpler and widely used in SLS. |
| Intel | Option 1 is okay – companies can report the exact function used to associate DL SINR and CE error |
| InterDigital | It can be left to the companies. |
| Apple | For option 1, how to come up with E(SINR) may be a problem. If E(SINR) is not agreed, then Option 3 is okay |
| Lenovo | We are okay with Option 1 and Option 3. |
| Qualcomm | Modeling realistic channel estimation is not new in this study item and is not unique to the study of AI/ML-based enhancements for CSI. Therefore, there is no need to align the error modeling and companies can report the method they adopted. |
| Samsung | As long as companies report how they generated/estimated their DL channel matrices, we prefer Option 3. |

### 2.10-4: Baseline SLS EVM table-Baseline for performance evaluation

The parameters for the R16 typeII CB (if selected as baseline) and R17 type II CB (if selected as baseline) have been agreed, so the ‘Baseline for performance evaluation’ entry of the overall SLS EVM table may therefore be determined as in the following question. The spirit is the baseline scheme is per company selection and report.

Question 2.10.3: For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, for ‘Baseline for performance evaluation’ in the baseline EVM table, do you agree with the following entry?

| Baseline for performance evaluation | Companies need to report which option is used between - Rel-16 TypeII Codebook as the baseline for performance and overhead evaluation. - Rel-17 TypeII Codebook as the baseline for performance and overhead evaluation. - Type I Codebook can be considered at least for performance evaluation |
| --- | --- |

As some companies have concern on the Type I CB as baseline, Moderator moves the 3rd bullet into the FFS. But from Moderator’s understanding, AI/ML may be a unified solution applicable to all CSI payload sizes, including those small payload sizes supported by only Type I CB (then the target is to achieve performance gain under the same payload size)?

Upd Question 2.10.3: For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, for ‘Baseline for performance evaluation’ in the baseline EVM table, do you agree with the following entry?

| Baseline for performance evaluation | Companies need to report which option is used between - Rel-16 TypeII Codebook as the baseline for performance and overhead evaluation. - Rel-17 TypeII Codebook as the baseline for performance and overhead evaluation. - FFS: Type I Codebook can be considered at least for performance evaluation |
| --- | --- |

| Support/Can accept | OPPO, Ericsson, CATT, LG (Partly) ,Xiaomi, ZTE(Partly) Huawei/Hisi, DCM, Fujitsu, CAICT, NVIDIA, Futurewei, CMCC, Intel, InterDigital, Apple, Lenovo (with some note), Qualcomm, Samsung |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| OPPO | We agree with this proposal. From our view, the Rel-16/Rel-17 Type II and Type I codebook performance should also be aligned. |
| CATT | Generally OK. But Type I codebook seems unnecessary here. |
| LG | We are not sure whether Type I CB needs to be evaluated as the main motivation of channel compression is to reduce / enhance the Type II CSI. We think using Type II CSI (especially Rel-16 Type II CSI) seems sufficient. |
| vivo | We prefer “Rel-16 TypeII Codebook as the baseline for performance and overhead evaluation”. |
| ZTE | We generally agree with this proposal. However, we think there is no need to consider Type I CB for performance evaluation and suggest removing it. - Type I Codebook can be considered at least for performance evaluation Since it may not make sense that AI approaches having gains over Type I Codebook can outperforms other legacy CBs. Therefore, we prefer Rel-16 Type II CB or Rel-17 Type II CB should be adopted as the baseline for performance comparison. |
| Huawei/Hisi | Support. It is more flexible and inclusive to let companies select the comparable baseline and report. |
| NTT DOCOMO | Since Rel-16 and Rel-17 type II Codebook as the baseline have pros and cons, it should be up to company which scheme is selected or whether both scheme are evaluated. |
| Fujitsu | We prefer Rel-16 Type-II codebook to be the baseline for performance and overhead evaluation. |
| Moderator | Please companies provide your views on whether Type I CB can be taken as baseline for comparison. |
| Ericsson | Type-I can optionally be included in addition to Type-II. For higher UE speeds, the Type-II report have less benefit over Type-I and can actually become worse than type-II, and it can be interesting to see the UE speeds when this occurs. So I think Type-I can be a baseline as well (optionally for the cases where it can be suspected that Type-I is better than Type-II), but one of the Type-II performances always needs to be reported. |
| CAICT | We would like to use Type I Codebook and R16 TypeII Codebook as baseline for lower overhead and higher overhead performance evaluation. |
| CMCC | We think Type-I Codebook can be optional. At least we can see the performance gain of the AI based approach over traditional codebook with small payload sizes. |
| Intel | We prefer Rel-16 Type II only but okay with proposal |
| Nokia/NSB | Type I cannot be used as an alternative baseline to Type-II because it would be difficult to draw common conclusions from two very different baselines |
| Lenovo | We are fine with the moderator’s proposal, however we prefer to prioritize Rel-16 TypeII Codebook. Rel-17 TypeII codebook is based on partial UL-DL channel reciprocity assuming FR1 with small duplexing distance, while Type-I codebook has significantly less resolution compared with Rel-16 TypeII codebook |
| Qualcomm | Comparing against Type I codebook may overestimate the benefit of AI/ML. Therefore, Type I should not be used as a baseline. We prefer to use Rel-16 Type II as the baseline. We are also fine with leaving it to companies to select between Rel-16 and Rel-17 Type II and report their choice. If Rel-17 Type II is used as a baseline and CSI-RS is assumed to be transmitted in a UE-specific manner, the downlink throughput must account for the resulting CSI-RS overhead correctly. |
| Samsung | Agree with Qualcomm above. |

### 2.10-5: Metrics

If Cosine similarity is selected as the metric, one remaining issue is: whether GCS or SGCS is selected as the metric. From Moderator’s view, it would be better to down select to a unified option to better align and capture the companies’ evaluation results in the TR. But as the preference of GCS and SGCS is controversial, we may need more time to conduct further analysis first, and to make down-selection maybe in the next meeting. A ‘FFS’ is then added accordingly

Proposal 2.10.2 Proposal 2.8.6: For the evaluation of the AI/ML based CSI feedback enhancement, if the GCS/SGCS is adopted as the intermediate KPI as part of the ‘Evaluation Metric’, companies to report whether SGCS or GCS is adopted.

- Note: wi is the target CSI of resource unit i, and wi is the output CSI of resource unit i. N is the total number of resource units. E{∙} denotes the average operation over multiple samples.

- Note: Companies are encouraged to report the detail information of the target CSI assumed in the evaluation.

- FFS: Further down-selection between GCS and SGCS.

GCS=E1Ni=1N||wiHwi ||||wi||  ||wi||,  SGCS=E1Ni=1N||wiHwi ||||wi||  ||wi||2

| Support/Can accept | OPPO, CATT , LG, Xiaomi, ZTE Huawei/Hisi, DCM, Fujitsu, CAICT, NVIDIA, Futurewei, CMCC, Intel,Apple, Lenovo, Qualcomm (comment below) |
| --- | --- |
| Object/Concern | Ericsson, |

| Company | View |
| --- | --- |
| OPPO | We agree with this proposal. We also suggest to down select one from GCS and SGCS for better and easier performance calibration between companies. |
| Ericsson | Since this is used for intermediate KPI only, it is up to each company to describe which metric is used. We have major concerns with these metrics, as discussed in our tdoc and the last two weeks. In addition, this is a rank 1 metric and we don’t see the relevance of a rank 1 metric as the UE has 4 RX and rank>1 is common. |
| CATT | Generally OK. While for the FFS, since the main bullet already says ‘companies to report whether SGCS or GCS is adopted’, we feel down selection is unnecessary. But we can accept if majority would like to. |
| vivo | We prefer SGCS. |
| ZTE | We agree with this proposal. As intermediate evaluation metrics, we think there is no need to adopt multiple different KPIs to evaluate the cosine similarity, since extra workload may be introduced for companies to calibrate. From our view, we prefer SGCS is selected for evaluation. |
| Huawei/Hisi | Support |
| NTT DOCOMO | We also find it beneficial to down-select GCS or SGCS. Since it is just the intermediate KPI, we are fine with either GCS or SGCS as long as they represent the performance of AI/ML based CSI feedback. |
| Fujitsu | We prefer GCS to be the intermediate KPI. |
| CAICT | SGCS is preferred. If GCS is also chosen for intermediated KPI, it is also acceptable. |
| NVIDIA | Support down-selection. |
| CMCC | We support down-selection. Using one metric can help better compare copanies’s results. We prefer SGCS. |
| Nokia/NSB | We agree with CATT’s comment and share Ericsson’s concern. We don’t need to spend effort on this FFS |
| Intel | Prefer to downselect |
| Lenovo | we are okay with the proposal with the FFS. |
| Qualcomm | There appears to be some overlap in the scenarios discussed in Proposals 2.10.2 and 2.10.3. If the intent is that 2.10.2 applies only to rank 1 case (the formulas suggest so), it would be better to add “for rank 1 case” after “Evaluation Metric”. Regarding GCS vs. SGCS, comparison across companies would be difficult if some companies report GCS and other companies report SGCS. Down-selection is one option to address this, but another alternative is to adopt both, i.e., companies can report both metrics. Based on this discussion, here is a proposed change: For the evaluation of the AI/ML based CSI feedback enhancement, if the GCS/SGCS is adopted as the intermediate KPI as part of the ‘Evaluation Metric’ for rank 1 case, companies to report whether both SGCS or and GCS is are adopted. - Note: wi is the target CSI of resource unit i, and wi is the output CSI of resource unit i. N is the total number of resource units. E{∙} denotes the average operation over multiple samples. - Note: Companies are encouraged to report the detail information of the target CSI assumed in the evaluation. - FFS: Further down-selection between GCS and SGCS. GCS=E1Ni=1N¦¦wiHwi ¦¦¦¦wi¦¦ ¦¦wi¦¦, SGCS=E1Ni=1N¦¦wiHwi ¦¦¦¦wi¦¦ ¦¦wi¦¦2 Also, for consistency and to avoid duplicate discussion, it might be good to merge Proposals 2.10.2 and 2.10.3 into a single general proposal that covers both rank 1 and rank > 1 cases. This could be achieved by removing “rank > 1” from Proposal 2.10.3 since the formulas in that proposal also apply to rank 1 when K is set to 1. |
| Samsung | Support. Agree with Qualcomm on merging Proposal 2.10.2 and 2.10.3 |

Another remaining issue is: how to calculate or extend GCS/SGCS for rank >1 cases. From Moderator’s view, it would be better to down select to a unified option to better align and capture the companies’ evaluation results in the TR. But as the preference of GCS and SGCS is controversial, we may conduct further analysis first, and to make down-selection maybe in the next meeting. A ‘FFS’ is then added accordingly.

Proposal 2.10.3 Proposal 2.9.2: For the evaluation of the AI/ML based CSI feedback enhancement, if the GCS/SGCS is adopted as the intermediate KPI as part of the ‘Evaluation Metric’, for rank>1 cases, companies to report the GCS/SGCS calculation/extension methods, including:

- Method 1: Average over all ranks

  - Note: wij is the jtheigenvector at resource unit i and K is the rank of the channel

GCS=E1Ni=1NjK||wijHwij ||||wij||  ||wij||,  SGCS=E1Ni=1NjK||wijHwij ||||wij||  ||wij||2

- Method 2: Weighted average over all ranks

  - Note: Companies to report the formula (e.g., whether normalization is applied for eigenvalues)

- Method 3: GCS/SGCS is separately calculated for each rank (e.g., for K ranks, K GCS values are derived respectively, and comparison is performed for per rank)

- Other methods are not precluded

- FFS: Further down-selection among the above options or take one/a subset of the above methods as baseline(s).

| Support/Can accept | OPPO, Ericsson, CATT, LG, Xiaomi, ZTE Huawei/Hisi, DCM, Fujitsu, CAICT, NVIDIA, Futurewei, CMCC, Intel, Nokia/NSB, Apple, Lenovo, Qualcomm, Samsung |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| OPPO | We agree with this proposal. We are both okay for method 1 and method 2, it would be better if companies can provide different performance evaluation results based on method 1 and method 2 so that we can find which KPI is more suitable when rank > 1. |
| Ericsson | The proposal is ok. No need for the FFS since we don’t need to align metric for intermediate KPIs so there will be no down-selection later. Each company can report which metric they have used, But we don’t object to have the FFS there in this proposal. |
| CATT | We are fine with Method 1~3. |
| Xiaomi | We can accept this proposal, but it is expected to do the down-selection among all methods at this meeting. We support Method 2 or 3. For option 1, it is not reasonable as each rank’s weight is same. For the terminology, we can understand that here per rank is a column of eigenvector for each layer, but it is different from our understanding on rank and layer. Could the FL please clarify whether the rank value indicate multiple layers or single layer, for example rank=2 means both layer 1 and layer 2 or rank=2 means layer 2 only. |
| vivo | We prefer Method 3. If GCS/SGCS is used to calculate loss-function, Method-1 is fine to us. |
| ZTE | We agree with this proposal. An unified calculation method should be down-selected for alignment and calibration among companies since so many different KPIs for intermediate performance evaluation may cause extra workload for calibration. From our view, we prefer Method 1 is adopted as the baseline. |
| Huawei/Hisi | Support the main text. Regarding to the different methods, we prefer Method 3 and we think Method 3 is the most simple and intuitive one without any misunderstanding. The merit of Method 1/2 is they derive a single cosine similarity over layers, which is easier for comparison. Method 1 is simpler than Method 2, while Method 2 can better differentiate the importance of different layers by introducing Eigen-values as weights. However, Method 2 may have multiple variants, so further discussions may be needed for a further down-selection under Method 2. |
| Fujitsu | As mentioned earlier, we prefer Method 2 with normalization. The normalized GCS is GCS=E1Ni=1NjKλijj=1Kλij¦¦wijHwij ¦¦wij wij, where λij is an eigenvalue of the squared matrix GGH, and G is the channel matrix, the superscript H is the Hermitian operator. |
| CAICT | We are fine with FL’s proposal and method 1 is preferred. |
| CMCC | We support this proposal and prefer Method 1 or Method 3. |
| Intel | Support down-selection otherwise it will be hard to compare |
| Lenovo | We believe, Method 1 is not a good metric since it gives same importance to all eigenvectors which is not correct and in practice eigenvectors corresponding to small eigne-values are not useful for scheduling purposes. Note that by giving same importance even to less useful eigenvectors, we are limiting the possibility of CSI compression. For Method 3, we are unclear how GCS/SGCS vectors (one value for each rank) will be compared between two models. For Method 2, we are in favor of the following version of weighted GCS as GCS=E1Ni=1Nj=1Kλij¦¦wijHwij ¦¦¦¦wij¦¦ ¦¦wij¦¦ ** why not normalized eigenvalues: Note that when we compute eigenvectors we assume that averaged power of the channel realizations are normalized (not each individual realization of H which will be the case if we divide the eigne vectors by their sum). We may also point out that the “Expectation” operation in the above formulation will take care of variations the norm of different channel realizations and also estimation error. And, in case of ideal channel estimation GCS will be normalized as expected by Qualcomm since it will be reduced to: |
| Qualcomm | Regarding the proposal, we share the concern regarding the layer index mismatch problem, and we prefer to adopt an intermediate KPI that accounts for this problem. We propose to use numerical spectral efficiency gap as the intermediate KPI for all ranks (explained in our response to Question 2.10.4). Regarding the wording, the comma before “for rank > 1” should be removed for clarity: “ … as part of the ‘Evaluation Metric’, for rank>1 cases …” Also, as mentioned in our comment in Proposal 2.10.2 above, it might be good to merge Proposals 2.10.2 and 2.10.3 into a single general proposal that covers both rank 1 and rank > 1 cases for consistency and to avoid duplicate discussion. |
| Samsung | Method2 is preferred. Normalizing the (S)GCS values for ranks with respect to their singular values better emulates throughput. |

In the reflector, Ericsson raised a comment on the issue of GSC/SGCS when extending to rank>1 cases, it imposes a strong order on the "true" vectors that the AE must follow, e.g.,

- The AI/ML output for layer 1 must try to reconstruct the true strongest eigenvector/DFT vector.

- The AI/ML output for layer 2 must try to reconstruct the second true strongest eigenvector/DFT vector.

- and so on...

If, for some reason, the AI/ML mixes the ordering (it outputs the second strongest vector for layer one), as in channel with two similar strength clusters, then the cosine similarity measure will be zero (orthogonal among eigenvectors).

So, an open question is raised by Moderator on whether this issue will occur, and if so, what could be a candidate solution.

Question 2.10-4: If GCS/SGCS is adopted as the intermediate KPI, for rank>1 cases, do you think using mismatched/mixed layer indexes between the target CSI and the output CSI would occur and cause a problem on the result of GCS/SGCS (e.g., the cosine similarity is calculated by using of the j-th layer eigenvector of the target CSI and the k-th layer eigenvector of the output CSI)? If so, what could be the solution to overcome this issue?

| Company | View |
| --- | --- |
| OPPO | We think this problem will not happen since AI/ML model has its own label for each layer. |
| Ericsson | If eigenvalue 1 and 2 are the same due to the channel, then eigenvector 1 and 2 have 50% risk of being interchanged. For OPPO’s comment, it seems they are proposing a solution to the issue, perhaps they can elaborate more? |
| CATT | This may be a relatively rare case that Eigenvalue 1 is equal to Eigenvalue 2. For other cases, the problem does not exist. |
| LG | We tend to agree with CATT, it seems rare case where 1st and 2nd eigenvalue are almost same. But, we are open for further discussion on this issue. |
| Xiaomi | We think the case explained by moderator is possible but we are not sure whether our understanding is aligned with moderator. Based on our assumption, for example, the strongest eigenvector is (1,1,0) corresponding eigenvalue 6, while the second strongest eigenvector is (0,0,1) corresponding eigenvalue 5 based on SVD of ideal channel. However, the SVD of realistic channel is (0,0,1) as strongest and (1,1,0) as second strongest due to some reasons, then the GCS between output CSI and ideal CSI is 0 for each rank, while from the throughput perspective, the performance is not impacted as both of the eigenvector are feedback to the gNB. If our assumption is correct, we think it is okay to allow this case happen, because actually we are comparing the AI performance with the baseline., e.g., eTypeII. The case of GCS equals to 0 will reduce the absolute GCS value of AI, while it also happens to eType II, therefore it does not impact the gap. We may have wrong assumption or understanding, if so, could moderator elaborate the case or give detailed example? |
| vivo | It seems that such a problem does not occur if the layer index is reported by UCI. |
| ZTE | From our view, it is not necessary to limit the layer ordering as long as the target CSI is fixed and the output CSI of AI model can be matched with the corresponding order of the target CSI for Rank>1. |
| Huawei/Hisi | We think this is related to the training strategy and should be part of the AI/ML model design. With a well-designed loss function and training strategy, the mismatched problem should be avoided by the design of its input/output. E.g., the input eigenvectors are mapped by the descending order of their associated Eigen-values, while the output eigenvectors follow the same order. Furthermore, even the mixed order issue happens, there can be some solutions to resolve it, e.g., for a single output eigenvector, its GCS/SGCS is determined by selecting the maximum cosine similarity value among its combinations with all target eigenvectors. Overall, whether this issue occurs and how to resolve it can be reported by companies as part of the AI/ML model description. |
| NTT DOCOMO | In our understanding, it depends on the target CSI. If the target CSI is ideal CSI and realistic channel estimation is used to estimate CSI on UE, the mismatched layer indexes occur between output of CSI and target CSI. However, if a) the target CSI derived ideal channel estimation or b) the target CSI is the input of the encoder, there is no chance to mismatch the layer indexes between output of CSI and target CSI. |
| CAICT | We think using the same AI/ML model for each layer could be considered and with reported layer number, the problem could be solved. |
| CMCC | It seems to be part of AI model implementation. We think AI model itself can handle this issue. Good AI model will pursue a high GCS/SGCS with proper loss function. |
| Intel | Agree with DOCOMO, if such issue is discovered, metrics defined on subspaces can be used: see “A. Barg and D. Y. Nogin, "Bounds on packings of spheres in the Grassmann manifold," in IEEE Transactions on Information Theory, vol. 48, no. 9, pp. 2450-2454, Sept. 2002” |
| Nokia/NSB | Yes, we think this is an issue that need to be addressed. We prefer to study this issue more and discuss it in the next meeting |
| Lenovo | It is a valid question that should be analyzed. Further study for RAN1#110 (fine-tuned details can be deferred to next meeting) |
| Qualcomm | We agree that this problem can indeed occur. In fact, it can even occur in the rank = 1 case if the UE-side AI/ML input is the raw channel and the target gNB-side AI/ML output is the eigen-vector. Realistic channel estimation modeling can also contribute to the problem. AI/ML models where the model operates on only one layer at a time may not have a reordering issue. However, more generally if the AI/ML model processes the raw channel or multiple layers together, then reordering can happen. The chosen KPI should be applicable to all types of AI/ML model operation. For a given rank K, the eventual performance depends only on the set of K eigenvectors, not on their order. Any AI/ML model accuracy KPI that is based on per-layer accuracy (such as GCS/SGCS) could have the issue that a layer reordering/mismatch can result in a low KPI value while the eventual performance may still be good. To address this concern, we propose the numerical spectral efficiency gap intermediate KPI. The spectral efficiency (SE) achievable can be computed based on the AI/ML model’s output CSI and the true raw channel. The gap between the SE of the output CSI and the ideal SE (of the target CSI itself) could then be considered as a measure of AI/ML model accuracy: SE gap = SE(wi, Hi) – SE(wi, Hi), averaged across resource units. where, for resource unit i, wi is the target CSI (e.g., the set of K eigenvectors), and wi is the output CSI, Hi is the raw channel matrix. The SE gap metric is naturally applicable to the case of rank > 1 and could address the concerns regarding GCS/SGCS. For example, if rank is 2 and the AI/ML model outputs the second strongest eigen vector for layer 1 and the strongest eigen vector for layer 2, the eventual performance will be the same as if there was no reordering. In this case, while the GCS or SGCS metric will be low, the SE gap metric will be the same as without reordering. In this way, SE gap reflects the eventual performance more accurately than GCS/SGCS. |
| Samsung | If seems the problem is in SVD based pre/post-processing. In that case isn’t this part of the model design? If the input of the AI/ML model is based the channel measurement in spatial-frequency or angle-delay domain rather than eigenvectors, then NMSE can be taken as intermediate KPI instead of (S)GCS. |

For the CSI enhancement metrics, we have agreed upon throughput, overhead (in the baseline EVM table), intermediate KPI (e.g., accuracy of AI/ML output CSI, with details FFS), capability/complexity related KPI (FLOPs, memory storage), etc. Except from the above agreed KPIs, Moderator wants to know if there is any other metric that needs to be captured. Please provide your views if any.

Question 2.10-5: Besides the metrics that have been agreed, do you think there is any other metric for AI/ML based CSI feedback enhancement which should be captured in the EVM but has not been discussed previously?

| Company | View |
| --- | --- |
| Ericsson | Not at the moment, but we’d like to keep the door open for adding additional metric later in the SI. |
| Xiaomi | We think power consumption should also be provided by companies. |
| Nokia/NSB | Agree with Ericsson. |
| InterDigital | No |
| Samsung | It is fine to consider a variety of intermediate KPIs. But we are afraid it would also make comparison difficult. So better to focus on the already agreed upon KPIs unless there is a significant advantage from adopting additional ones. |

### 2.10-6: Additional AI/ML related parameters/values for the EVM table

The baseline EVM tables for LLS and SLS have been agreed, yet AI/ML related parameters (e.g., dataset construction, generalization verification, and AI/ML related metrics) can be of additional/different assumptions.

E.g., during the previous discussions, some company has raised that the Deployment scenario can include InH besides Uma/UMi for the purpose of generalization verification, while some company held the view that UE distribution can include 100% outdoor UEs as an option with various UE speeds for CSI involving temporal domain. So, Moderator tries to provide the baseline EVM table that has been agreed, and collect the views from companies on what different/additional values can be added on top of the baseline EVM.

Note: The following baseline EVM table is provided by merging two previous agreements.

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD (TDD is not precluded), OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) is a baseline. Other scenarios (e.g. UMi@4GHz 2GHz, Urban Macro) are not precluded. |
| Frequency Range |  | FR1 only, FFS 2GHz or 4GHz as a baseline - If R16 is selected as baseline for performance evaluation: 2GHz as baseline, optional for 4GHz - If R17 is selected as baseline for performance evaluation: 2GHz with duplexing gap of 200MHz between DL and UL, optional for 4GHz |
| Inter-BS distance |  | 200m |
| Channel model |  | According to TR 38.901 |
| Antenna setup and port layouts at gNB |  | Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ - 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ Other configurations are not precluded. |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) Other configuration is not precluded. |
| BS Tx power |  | 41 dBm for 10MHz, 44dBm for 20MHz, 47dBm for 40MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz for 2GHz, 30kHz for 4GHz |
| Simulation bandwidth |  | FFS - If R16 is selected as baseline for performance evaluation: 10 MHz for 15kHz as a baseline, - If R17 is selected as baseline for performance evaluation: 20 MHz for 15kHz as a baseline (optional for 10 MHz with 15KHz), and configurations which emulate larger BW, e.g., same sub-band size as 40/100 MHz with 30kHz, may be optionally considered. Above 15kHz is replaced with 30kHz SCS for 4GHz. |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | FFS SU/MU-MIMO with rank adaptation. Companies are encouraged to report the SU/MU-MIMO with RU |
| MIMO layers |  | For all evaluation, companies to provide the assumption on the maximum MU layers (e.g. 8 or 12) |
| CSI feedback |  | Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback) : 5 ms, - Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms |
| Overhead |  | Companies shall provide the downlink overhead assumption (i.e., whether the CSI-RS transmission is UE-specific or not and take that into account for overhead computation) |
| Traffic model |  | FFS |
| Traffic load (Resource utilization) |  | FFS 20/50/70% Companies are encouraged to report the MU-MIMO utilization |
| UE distribution |  | - 80% indoor (3km/h), 20% outdoor (30km/h) FFS whether/what other indoor/outdoor distribution and/or UE speeds for outdoor UEs needed |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic as a baseline FFS ideal channel estimation |
| Evaluation Metric |  | Throughput and CSI feedback overhead as baseline metrics. Additional metrics, e.g., ratio between throughput and CSI feedback overhead, can be used. Maximum overhead (payload size for CSI feedback)for each rank at one feedback instance is the baseline metric for CSI feedback overhead, and companies can provide other metrics. |
| Baseline for performance evaluation |  | FFS |

Question 2.10-6: If SLS is adopted, for the purpose of AI/ML related evaluations (e.g., dataset construction, or generalization verification), what additional/different value(s) can be considered on top of the agreed baseline EVM table?

- Note: Under this question, no need to discuss the AI/ML related metrics which have been agreed.

| Company | View |
| --- | --- |
| Moderator | Please type in your preferred changes, if any, on top of the agreed baseline EVM table, and provide the purpose for the changes (e.g., verifying generalization, AI/ML related solution/sub use case, or others). As an example: Scenario; Dense Urban (Macro only) is a baseline. Other scenarios (e.g. UMi@4GHz 2GHz, Urban Macro) are not precluded. UMi, InH included for the purpose of verifying AI/ML generalization... UE distribution; - Option 1: 80% indoor (3km/h), 20% outdoor (30km/h) - Option 2: 100% outdoor (variable speed, e.g., x1% 10km/h, x2% 20km/h and x3% 30km/h to be reported by companies) for the purpose of temporal-domain compression FFS whether/what other indoor/outdoor distribution and/or UE speeds for outdoor UEs needed.. |
| Ericsson | If we are to study ML based channel prediction in this SI, the problem we see in network is the degrading performance of MU-MIMO at speeds around and above 10 km/h. Hence, we believe it is of interest for this sub use case to have somewhat more fine granularity in the UE speed, perhaps (3,10,15,20,30) km/h is a good set. For these data points we could see the MU-MIMO degradation and eventually how ML can improve. If we only use 3 and 30 km/h as in current SLS assumptions, we will not see the effect of ML for CSI prediction since at 30 km/h it may be much harder to make improvements compared to around 10-20 km/h. |
| Xiaomi | We think the following parameters related to analog beamforming can be considered, if analog BF is considered. TXRU mapping to antenna elements; One TXRU per vertical dimension per polarization. TXRU mapping weights; TXRU virtualization only in the vertical dimension (i.e. 1D virtualization) using DFT, i.e., 1D sub-array partition model defined in TR36.897. Criteria for beam selection for serving TRP; Select the best beam pair among the limited set of DFT beams, based on the criteria of maximizing receive power after beamforming.. |
| vivo | We prefer to include ‘InH’ into the e.g., although I know other scenarios (e.g. UMi@4GHz 2GHz, Urban Macro) are not precluded. Based on our simulation results, the generalization of AI model using a synthetic data set constructed by UMi and UMa is simple, but it cannot be always realized using a synthetic data set constructed by UMi (or UMa) and InH. Increasing the dimension of data set, therefore, is necessary from the perspective of model generalization. |
| ZTE | We generally agree with the above EVM table, however, we think the examples from Moderator may not be suitable. For the first example, we think ‘Other scenarios are not precluded’ can include UMi and InH scenarios, and they may not be mentioned again. For the second example, we may not need to consider Option 2 right now since temporal-domain compression is a pending sub use case for further discussion. We think we can reuse the statement in last round: FFS whether/what other indoor/outdoor distribution and/or UE speeds for outdoor UEs needed |
| Huawei/Hisi | Deployment Scenario: AI/ML model should be generalized over multiple deployment scenarios, including UMa, UMi, and InH. UE distribution-indoor/outdoor ratio: AI/ML model should be generalized over various indoor/outdoor ratios, including 100%:0, 20%:80% (already there), 50%:50%, 20%:80%, and 0:100%. UE distribution-outdoor UE speed: AI/ML model should be generalized over various UE speeds, including, e.g., 3km/h, 10km/h, 30km/h (already there), 60km/h, and 120km/h. How to combine the different indoor/outdoor ratio and different outdoor UE speeds can be FFS. Antenna setup and port layouts at gNB: the antenna configuration for indoor hotspot should be included for verifying generalization. Simulation bandwidth: 10MHz, 20MHz, 30MHz, 50MHz can be added for verifying generalization/scalability. |
| NTT DOCOMO | We are fine with the principle of proposal, especially the scenarios close to UMa and different from UMa can be considered for generalization. However, the proponent should bring the reason why specifically UMi and InH need to be captured in the above table. |
| Nokia/NSB | Generally agree with Ericsson. For CSI prediction, it may be necessary to consider different indoor/outdoor ratios in addition to additional UE speeds. |

Similarly, for the LLS baseline EVM table, please provide additional/different value(s) if any for the purpose of AI/ML related evaluations on top of the following LLS table which has been agreed.

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | FDD (TDD is not precluded), OFDM |
| Carrier frequency | 2GHz as baseline, optional for 4GHz |
| Bandwidth | 10MHz or 20MHz |
| Subcarrier spacing | 15kHz for 2GHz, 30kHz for 4GHz |
| Nt | 32: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Nr | 4: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ |
| Channel model | CDL-C as baseline, CDL-A as optional |
| UE speed | 3kmhr, 10km/h, 20km/h or 30km/h to be reported by companies |
| Delay spread | 30ns or 300ns |
| Channel estimation | Realistic channel estimation algorithms (e.g. LS or MMSE) as a baseline, FFS ideal channel estimation |
| Rank per UE | Rank 1-4. Companies are encouraged to report the Rank number, and whether/how rank adaptation is applied |

Question 2.10-7: If LLS is adopted, for the purpose of AI/ML related evaluations (e.g., dataset construction, or generalization verification), what additional/different value(s) can be considered on top of the agreed baseline EVM table?

- Note: Under this question, no need to discuss the AI/ML related metrics which have been agreed.

| Company | View |
| --- | --- |
| Moderator | Please type in your preferred changes, if any, on top of the agreed baseline EVM table, and provide the purpose for the changes (e.g., verifying generalization, AI/ML related solution/sub use case, or others). |

### 2.10-7: Methodology for generalization

In the 3rd round discussion, QC and Lenovo pointed out that the generalization capability can be studied by specifying several test scenarios and evaluating the performance of a single AI/ML model across all those test scenarios. On the other hand, how the single AI/ML model is trained (e.g., based on a scenario-specific dataset, or a mix of datasets) can be reported by companies. So an open question is raised in this round on your view: whether there is a need to align the testing/inference set in the EVM and what should be the set in EVM? And if so, what parameters/values to be reported?

Question 2.10-8: For the verification of the generalization under the AI/ML based CSI feedback enhancement, whether it is needed to align the set of configuration(s)/ scenario(s) for testing/ inference in the EVM? If needed, what parameters and values should be considered to generate the set of configuration(s)/ scenario(s) for testing/inference?

- Note: The generalization performance is verified by applying the trained AI/ML model across the set of configuration(s)/ scenario(s).

| Company | View |
| --- | --- |
| Moderator | It is not expected to achieve agreement on this question at this meeting. But companies are encouraged to share your views, which may be taken as a reference for the next meeting. |
| OPPO | From our view, in the first stage of generalization performance evaluation, we suggest not to consider the alignment, companies are encouraged to provide their results and configuration(s) / scenarios(s). In the future, it is better to selection several typical mixed configurations(s) / scenario(s) for conclusion. |
| Ericsson | Agree with OPPO |
| CATT | Agree with OPPO. At the first stage we can leave to companies’ report. |
| LG | We agree with OPPO. |
| vivo | The generalization performance (or test performance) of AI model is affected by the AI model structure, the variety of training data set and the training strategy. It is better to keep the training loss to be an accurate approximation of the generalization loss uniformly for all hypotheses. We believe, different levels of generalization may need to be verified. For the generalization of CSI enhancement, therefore, the following - The same and different input data dimensions, according to pre-processing and post-processing. - The different data set constructed in a way that data samples across different UEs, different cells, different drops, different scenarios. - The training data set and testing data set constructed by synthetic data samples. Different ratio of data mixture with the same total sample number for each dataset. |
| ZTE | We agree with OPPO. From our view, this discussion can be deferred to the next meeting after some general proposals are determined. |
| NTT DOCOMO | There is no need to align the set of configuration(s) / scenario(s) for testing / inference in the EVM. Same as AI/ML model training, companies could report it together with the evaluation results of generalization. |
| CAICT | Agree with OPPO’s view. |
| Futurewei | We think there is no need to align scenarios/configurations as each company may target different solution deployment scenarios which will determine how to verify model generalization. Companies can report what scenarios/configurations are used in training and testing datasets for model generalization performance evaluation. |
| CMCC | We agree with OPPO. |
| Nokia/NSB | Agree with OPPO |
| InterDigital | Agree with Oppo |
| Qualcomm | Agree with Oppo. We prefer to leave it for companies to report. |

7th round email approval

### 2.11-1: Baseline SLS EVM table-Traffic model

From the 6th round discussions, 3 companies cannot live with the proposal with view that full buffer can only be used for intermediate result comparison.

Supporting (16): OPPO, CATT, LG, Xiaomi, vivo Huawei/Hisi, DCM, CAICT, NVIDIA, Futurewei, CMCC, Intel, Apple, Lenovo, Qualcomm, Samsung

Objecting (3): Ericsson, Nokia/NSB (FFS only), InterDigital

Moderator cannot find a better wording to satisfy both camps, so try to provide two compromise versions, and companies can choose which can be acceptable. The proponent companies of full buffer may analyze the necessity of full buffer as an option to be captured into the TR in the next meeting with potential evaluation results.

Proposal 2.11.1: For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, the ‘Traffic model’ in the baseline of EVM is captured as follows:

- Option 1:

| Traffic model | FTP model as a baseline FTP model 1 with packet size 0.5 Mbytes Other FTP model is not precluded. Full buffer model is not precluded. FFS: whether full buffer is taken only for the purpose of calibration and/or comparing intermediate results (e.g., accuracy of AI/ML output CSI, etc.), or also for eventual performance comparison with the benchmark release and drawing SI conclusions. |
| --- | --- |

- Option 2:

| Traffic model | FTP model as a baseline FTP model 1 with packet size 0.5 Mbytes Other FTP model is not precluded. Full buffer model is not precluded at least for the purpose of calibration and/or comparing intermediate results (e.g., accuracy of AI/ML output CSI, etc.). FFS: whether full buffer is taken only for the purpose of calibration and/or comparing intermediate results (e.g., accuracy of AI/ML output CSI, etc.), or also for eventual performance comparison with the benchmark release and drawing SI conclusions. |
| --- | --- |

| Option 1 | Support/Can accept | DCM, OPPO, ZTE, LG, CATT, CAICT, Samsung, CMCC, vivo, NVIDIA, Intel |
| --- | --- | --- |
|  | Object/Concern | Ericsson, Nokia/NSB |
| Option 2 | Support/Can accept | DCM, Ericsson, CATT, CAICT, CMCC Huawei/Hisi, Qualcomm, NVIDIA, Futurewei, InterDigital, Nokia/NSB |
|  | Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | As this is the last round of email, companies are encouraged to provide the changes for potential compromise (on top of either Option 1 or Option 2) rather than simply saying yes or no. |
| OPPO | We prefer Option 1. But we are still not clear how the traffic model affects the intermediate KPI, which is only influenced by the dataset and AI model from our understanding. |
| ZTE | We prefer Option 1 to be a baseline, whether full buffer is taken for the purpose of calibration and comparing intermediate results can be further discussed. Additionally, if full buffer for eventual performance comparison with the benchmark release is not captured in SI conclusions, we are fine with Option 2 for progress. |
| LG | We prefer Option 1. But, we could live with Option 2 for the sake of progress. |
| Ericsson | Full buffer, if used, can only be used for calibration purpose and intermediate results, it cannot be used for decision making on the feasibility of the feature and for making SI conclusions. |
| Huawei/Hisi | FTP traffic has been widely applied as the major traffic model for NR evaluations, while full buffer may not well present the realistic traffics. Option 2 can be a good compromise to keep the door still open for full buffer by saying ‘full buffer .. at least for the purpose of calibration and/or comparing intermediate results’, as it should be full buffer proponents obligation to convince its applicable cases for AI/ML evaluation. |
| vivo | Option 1 is fine to us. For clarification of FFS, does it mean the companies are still encouraged to provide the simulation results based on Full Buffer Traffic? Whether adopting Full Buffer Traffic will be determined in the next meeting based on the simulation comparison between FTP and full buffer. |
| Nokia/NSB | FTP model should be used eventually for performance evaluation. It’s ok to use full buffer for intermediate results |
| Intel | Perhaps come back to this in the next meeting |

### 2.11-2: Baseline SLS EVM table-Channel estimation

As there a still a number of companies believe there is no need to align the error modeling method, thus Moderator tries to propose the following conclusion.

Proposed Conclusion 2.11-1: For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, for ‘Channel estimation’, it is up to companies to choose the error modeling method for realistic channel estimation and report by willingness.

| Support/Can accept | DCM, ZTE, LG, CATT, CAICT, Samsung, Ericsson, CMCC Huawei/Hisi, Fujitsu, Qualcomm, vivo, NVIDIA, Futurewei, InterDigital, Nokia/NSB, Lenovo |
| --- | --- |
| Object/Concern | OPPO |

| Company | View |
| --- | --- |
| Samsung | Ok. |
| Intel | Do we need to agree to this ? |

### 2.11-3: Baseline SLS EVM table-Baseline for performance evaluation

In the 6th round, it looks the views on whether to optionally adopt Type I CB as basline is controversial.

Supporting Type I as optional (3): Ericsson, CAICT, CMCC

Not supporting Type I (7): CATT, LG, ZTE, Nokia, Lenovo, Qualcomm, Samsung

So, Moderator moves the Type I CB in the FFS with changes, and see if this version is good to all companies.

Proposal 2.11.2: For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, the ‘Baseline for performance evaluation’ in the baseline of EVM is captured as follows

| Baseline for performance evaluation | Companies need to report which option is used between - Rel-16 TypeII Codebook as the baseline for performance and overhead evaluation. - Rel-17 TypeII Codebook as the baseline for performance and overhead evaluation. - FFS: Whether Type I Codebook can be optionally considered at least for performance evaluation |
| --- | --- |

| Support/Can accept | DCM,OPPO, ZTE, Ericsson, CATT, CAICT, CMCC Huawei/Hisi, Fujitsu, Qualcomm, vivo, NVIDIA, Futurewei, InterDigital, Nokia/NSB, Lenovo |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | Note that Type I CB, if optionally adopted, is not considered as an alternative baseline to Type II, but rather for drawing conclusion for a separate applicable case, as Type I and Type II can be applied to different payload sizes/scenarios. |
| ZTE | We agree with this proposal. We think if we compare the performance of AI/ML-based approach with Type I Codebook, performance gains of the AI/ML-based approach may be overestimated. Therefore, whether to take Type I Codebook as baseline needs further discussion. |
| Ericsson | OK. We only think Type-I is needed as a baseline for the KPIs and parameter settings (i.e. UE speed) where it outperforms Type-II |
| Samsung | Thank you FL. If it is adopted optionally, we agree with the proposal. |

### 2.11-4: Metrics

In the last round, some companies have concern on whether the FFS down-selection is needed. In addition, some companies think both SGCS and GCS and be reported. So Moderator tries to make the following changes to generate an updated proposal.

Proposal 2.11.3 Proposal 2.10.2: For the evaluation of the AI/ML based CSI feedback enhancement, if the GCS/SGCS is adopted as the intermediate KPI as part of the ‘Evaluation Metric’, companies to report whether SGCS, or GCS, or both is are adopted.

- Note: as an example of rank=1, wi is the target CSI of resource unit i, and wi is the output CSI of resource unit i. N is the total number of resource units. E{∙} denotes the average operation over multiple samples.

GCS=E1Ni=1N||wiHwi ||||wi||  ||wi||,  SGCS=E1Ni=1N||wiHwi ||||wi||  ||wi||2

- Note: Companies are encouraged to report the detail information of the target CSI assumed in the evaluation.

- FFS: Further down-selection between GCS and SGCS.

| Support/Can accept | DCM,OPPO, ZTE, LG, CATT, CAICT, Samsung, CMCC Huawei/Hisi, Fujitsu, Qualcomm, vivo, NVIDIA, Futurewei, Nokia/NSB, Lenovo |
| --- | --- |
| Object/Concern | Ericsson |

| Company | View |
| --- | --- |
| Moderator | @QC @Samsung the intention of this proposal is to address the issue 1) whether GCS or SGCS or both are adopted, while the next proposal is to address the issue of 2) how to express (S)GCS for the case of rank>1. As they are addressing different issues, it would be good to separate them in two proposals to ensure that companies against issue 1) would not object the issue 2) proposal. |
| Ericsson | We don’t see the need for this Proposal, it is conflicting with Proposal 2.11.4 |
| Samsung | OK. Thank you for the explanation. We are fine with adopting both. |
| Qualcomm | We agree with the proposal and thank the moderator for the clarification. |
| Nokia/NSB | We are ok with this proposal. In our understanding, it addresses the second FFS of a previous agreement (FFS: whether GCS or SGCS is adopted), for rank 1, so maybe to address Ericsson’s concern, we can clarify this in the main statement For the evaluation of the AI/ML based CSI feedback enhancement, if the GCS/SGCS is adopted as the intermediate KPI as part of the ‘Evaluation Metric’, for rank = 1, companies to report whether SGCS, or GCS, or both is are adopted. |

For multiple ranks, it looks the views are mostly stable. So put the following proposal with an editorial change and by adding some missed interpretations of the formula. Updates for correcting an editorial error.

Upd Proposal 2.11.4 Proposal 2.10.3: For the evaluation of the AI/ML based CSI feedback enhancement, if the GCS/SGCS is adopted as the intermediate KPI as part of the ‘Evaluation Metric’, for rank>1 cases, companies to report the GCS/SGCS calculation/extension methods, including:

- Method 1: Average over all ranks

  - Note: wij is the jtheigenvector of the target CSI at resource unit i and K is the rank of the channel. wij is the jth output vector of the output CSI of frequency resource unit i. N is the total number of resource units. E{∙} denotes the average operation over multiple samples.

GCS=E1Ni=1NjK||wijHwij ||||wij||  ||wij||,  SGCS=E1Ni=1NjK||wijHwij ||||wij||  ||wij||2

- Method 2: Weighted average over all ranks

  - Note: Companies to report the formula (e.g., whether normalization is applied for eigenvalues)

- Method 3: GCS/SGCS is separately calculated for each rank (e.g., for K ranks, K GCS values are derived respectively, and comparison is performed for per rank)

- Other methods are not precluded

- FFS: Further down-selection among the above options or take one/a subset of the above methods as baseline(s).

| Support/Can accept | DCM,OPPO, ZTE, LG, Ericsson, CATT, CAICT, Samsung, CMCC Huawei/Hisi, Fujitsu, Qualcomm, vivo, NVIDIA, Futurewei, Nokia/NSB, Lenovo |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| NTT DOCOMO | Fine with the proposal. We think the target CSI could be CSI estimated by UE or ideal CSI in the above equation. Could you clarify which one is meant by target CSI? |
| ZTE | We agree with this proposal. We think we can discuss the GCS/SGCS calculation methods for rank>1 in detail in next meeting after more simulation evaluations are performed by companies, and then down select a better subset of methods as baseline. |
| Ericsson | The “for rank>1 cases” should be removed |
| Samsung | OK. |
| Moderator | @ DOCOMO The meaning of the target CSI is still FFS as in the previous agreement. Agreement For the evaluation of the AI/ML based CSI feedback enhancement, for ‘Channel estimation’, ideal DL channel estimation is optionally taken into the baseline of EVM for the purpose of calibration and/or comparing intermediate results (e.g., accuracy of AI/ML output CSI, etc.) - Note: Eventual performance comparison with the benchmark release and drawing SI conclusions should be based on realistic DL channel estimation. - FFS: the ideal channel estimation is applied for dataset construction, or performance evaluation/inference. - FFS: How to model the realistic channel estimation FFS: Whether ideal channel is used as target CSI for intermediate results calculation with AI/ML output CSI from realistic channel estimation. @Ericsson: But if we remove ‘rank>1’, then the Method 1~3 are of the same result, right? The intention of this proposal is to list candidates to express multiple ranks, so keeping ‘rank>1’ there can better clarify? |
| Qualcomm | A small edit in the newly added wording: “frequency unit” should be replaced with “resource unit” for consistency. |
| Lenovo | We are okay with this proposal assuming that the down-selection will be discussed later. |

Specific evaluation methodology for CSI compression sub use case

Summary of views from companies

### 3.1-1: Companies that evaluated/considered CSI compression: Huawei, Hisilicon, Ericsson, vivo, Apple, Nokia, OPPO, Samsung, ZTE, Qualcomm, CATT, LG, CAICT, Xiaomi, CMCC, NTT DOCOMO, Beijing Jiaotong University, Charter, FUTUREWEI, NVIDIA, Lenovo, InterDigital, Fujitsu, MediaTek

CSI compression in spatial-frequency domain: Huawei, Hisilicon, Ericsson, vivo, Apple, Nokia, OPPO, Samsung, ZTE, Qualcomm, CATT, LG, CAICT, Xiaomi, CMCC, NTT DOCOMO, Beijing Jiaotong University, Charter, FUTUREWEI, InterDigital, Fujitsu, MediaTek

- Huawei, Hisilicon: For rank1 and rank2, AI/ML-based CSI compression on spatial-frequency domain can provide better performance with around 8%-11% over Rel-16 Type II codebook and outperforms Rel-17 Type II codebook. For achieving the same throughput, AI/ML-based CSI compression on spatial-frequency domain requires less feedback overhead, with an overhead reduction of about 60%

- Ericsson: Gains over Rel-16 Type II CB for cell edge (from -40.5% gap with ideal to -32.2% gap with ideal) and cell average (from -33.9% gap with ideal to -27.2% gap with ideal).

- vivo: For rank1, the AI model can achieve about 9% average cosine similarity gain and 11% average SE gain or 64% overhead reduction.

- Apple: For rank1, preliminary evaluation for rank-1 precoder shows around 45% overhead reduction based on cosine similarity.

- Nokia: For rank1, the cosine similarity remains above 83% for compression ratios as low as 1/26.

- OPPO: The AI/ML-based method can achieve better SGCS performance than Rel-16 eType II.

- Samsung: The AI/ML-based method can achieve Cosine similarity of >0.9 for various AI/ML algorithms.

- ZTE: For rank1, the Transformer based AI model has around 1% - 3% performance gain in GCS or reduces the feedback overhead by around 20% - 35% compared with the Rel-16 eType II

- Qualcomm: For the UMI/InH channel model, at a loss of -7.4 dB/-11.6 dB, ML based solution gives a 33%/58% overhead reduction over eType2, respectively.

- CATT: It can be seen from the results that under the same cosine similarity, compared with traditional codebook based CSI feedback, 40 ~ 150 bits can be saved by AI based PMI compression feedback

- CAICT: The SCGS of 48bits and 128 bits CSI feedback by 500 epoch training are 0.83 and 0.92

- Xiaomi: In terms of accurate improvement, AI-based feedback with the same cost has a significant performance improvement in terms of SGCS, which is about 15% gain compared with Type II feedback

- CMCC: With the same feedback bits, AI based approach could obtain 4%~40% performance gain over traditional codebook in the SGCS, or reduce 30%~60% feedback bits under the same SGCS

- NTT DOCOMO: By comparing to the Rel-16 Type II codebook, AI/ML method could improve the SE by around 10% for SU and 180% for 4-UE MU when the number of UCI bits is 108.

- Charter: The proposed NN can achieve ρ≈0.9 with only 60 bits of feedback, i.e., 70% decrease in overhead. To achieve the best performance, i.e., ρ≈0.98, the NN requires 169 bits which is 50% less than the required overhead for eType II.

- InterDigital: The multi-channel (combination of CDL-A, CDL-B and CDL-C) trained AI/ML CSI compression model tested in-distribution, outperforms CSI Type I, even with smaller feedback overhead. With the same feedback overhead, the AI/ML mixed model performs better than CSI Type I.

- Fujitsu: Compared to the Rel-16 codebook-based method, 20% gain in cosine similarity can be achieved by the AI/ML scheme 1, in which only 22% of the CSI feedback overhead is required

- MediaTek: AI/ML-based MIMO CSI compression shows about 8 to 10% better performance over eType II codebook in terms of cosine similarity

CSI compression in spatial-frequency-time domain: Huawei, Hisilicon, Apple

Note from Moderator: different from the CSI prediction in time domain which predicts a future CSI with an AI/ML model, the CSI compression involving time domain compresses CSI with an AI/ML model by taking into account the historical CSI information, and the feedback CSI does not include predicted future CSI.

- Huawei, Hisilicon: Two-sided structure of auto-encoder is adopted. AI/ML-based CSI compression on spatial-frequency-time domain can provide around 12%-14% performance gain over Rel-16 Type II codebook and outperforms AI/ML-based CSI compression on spatial-frequency domain.

Joint CSI prediction and CSI compression: Samsung

- Samsung: Two-sided structure of auto-encoder is adopted. Time domain correlation curves in terms of cosine similarity and selected best PMI over 100 time slots are simulated. It indicates three-dimensional CSI compression in the spatial, frequency, and time domains, may potentially achieve significant CSI feedback overhead reduction.

### 3.1-2: AI/ML structure for CSI compression

Two-sided structure including a CSI compression part and a CSI decompression part (auto-encoder as an example): Huawei, Hisilicon, Apple, vivo, Ericsson, Nokia, OPPO, Samsung, ZTE, Qualcomm, CATT, LG, CAICT, Xiaomi, CMCC, NTT DOCOMO, Beijing Jiaotong University, Charter, FUTUREWEI, InterDigital, Fujitsu, MediaTek

Based on the views from companies, there can be following approaches to obtain/align the CSI compression part at UE and the CSI decompression part at the gNB.

- Option 1: The model is preloaded with gNB/UE implementation with offline manner. Huawei, Hisilicon, OPPO, Samsung, Charter

  - Huawei, Hisilicon: Training is performed in an offline manner, where the network collects the training inputs and performs training at a separate training phase as described previously, and after the AI/ML model is deployed, it is not updated during the simulation. Evaluations on the AI/ML model updating or fine-tuning can be further studied.

  - OPPO: In the model downloading/uploading procedure does not need to be specified in the specifications (e.g. assuming the model has been preloaded with gNB/UE implementation), CSI feedback compression is with Collaboration level 1b: Signaling exchange for AI/ML inference without model exchange before and during AI/ML inference

  - Samsung: One possibility is an offline trained AI/Model can be shared/transferred from gNB to UE

  - Charter: In this contribution, we solely focus on the potential overhead reduction provided by autoencoders and assume the gNB and UE have access to NN’s coefficients after training.

- Option 2: The model is delivered to the other side via air-interface. OPPO

- Option 3: Separate training of the two-sided model: Huawei, Hisilicon, Ericsson

  - Huawei, Hisilicon: Study whether/how the CSI compression part and the CSI decompression part of a two-sided AI/ML model can be matched under separate training at gNB and UE without model exchange via air interface.

  - Ericsson: KPI includes the ability of a dual-side AI models to cope with the multi-vendor issue

AI/ML model settings for multiple ranks

Huawei, HiSilicon, vivo, and Xiaomi have discussed the issue on how to set up AI/ML models for multiple ranks situation. Huawei, HiSilicon, vivo analyzed detailed methods, which can be summarized as follows:

- Option1: Separated AI/ML models are trained per rank and applied for corresponding ranks to perform individual inference. Huawei, Hisilicon, vivo (per layer model)

  - vivo: Per-layer model can achieve similar cosine similarity with half model size compared with per-rank model, as observed from the evaluations results

- Option2: A unified AI/ML model is trained and applied for each rank to perform individual inference. Huawei, Hisilicon

- Option3: Separated AI/ML models are trained for a certain number of ranks and applied for the corresponding ranks to perform inference. vivo (per rank model)

- Option4: A unified AI/ML model is trained and applied for adaptive ranks to perform inference. Huawei, Hisilicon

### 3.1-3: AI/ML model adopted in evaluations

- Huawei, Hisilicon: Transformer

- Ericsson: CNN

- Apple: CNN

- Nokia: CSINET-like

- OPPO: EVCsiNet-T

- Samsung: ViT, 2D CNN, bi-LSTM

- ZTE: Both CNN (modified ResNet) and Transformer are used

- CATT: Transformer

- Xiaomi: Transformer

- CMCC: Both EVCsiNet and Transformer are used

- NTT DOCOMO: Both Transformer and CRNet are used

- Beijing Jiaotong University: CsiNet+

- Charter: CNN

- InterDigital: CSI-Net

### 3.1-4: Simulation approach

Option 1: System level simulation is adopted/can be considered: Huawei, Hisilicon, Ericsson, vivo, Apple, Nokia, Intel, OPPO, Samsung, ZTE, Qualcomm, CATT, CMCC, AT&T, Beijing Jiaotong University, MediaTek

Option 2: Link level simulation is adopted/can be considered: Xiaomi, LG, CMCC, NTT DOCOMO, Charter, InterDigital, MediaTek

### 3.1-5: Input CSI information of the AI/ML model

Input original CSI format

Option 1: Raw channel matrix. Nokia, LG, Xiaomi, NTT DOCOMO,

- Nokia: The channel matrix is estimated in each PRB at the UE and fed into the autoencoder and recovered at the gNB at the output of the autoencoder.

- LG: Raw channel information can be compressed and feed-backed to gNB. Thus, it may lead to explicit feedback and its specification impact seems larger compared to implicit feedback based scheme.

- Xiaomi: Both full channel feedback and eigenvector feedback should be considered

- NTT DOCOMO: Both the channel matrix and eigenvector are assumed for evaluating the performance of AI/ML-based compression/reconstruction.

Option 2: Eigenvector. Huawei, Hisilicon, Apple, Nokia, OPPO, Samsung, ZTE, Qualcomm, CATT, Xiaomi, CMCC, NTT DOCOMO, Charter

- Huawei, Hisilicon: Eigenvector should be used as the original CSI applied for training/inference under AI/ML-based CSI feedback.

- Apple: Using eigen vector as input enjoys much lower feedback overhead in low rank case, enable straightforward comparison with type II codebook design.

- Nokia: The PRB’s are grouped into a smaller number of Nsub subbands, then for each subband the covariance matrix snapshots CH,i∈CNt× Ntis constructed; then the eigenvalue-eigenvector decomposition of the CH,i matrix for each subband i is obtained; as the input of the CSI compression part, the joint eigenvector matrix V∈ CNt×Nsub  is generated corresponding to the set of eigenvectors for all of the subbands.

- ZTE: In our initial evaluations, sub-band eigenvectors are used as the training data for AI models.

- Qualcomm: We use the strongest singular vector “v” as the input to the ML model’s encoder。

- LG: Raw channel information can be compressed and feed-backed to gNB. Thus, it may lead to explicit feedback and its specification impact seems larger compared to implicit feedback based scheme

- Xiaomi: The input data is eigenvector  wk based on SVD of channel matrix generated by CDL-A300, after linear embedding and positional embedding, the input data are encoded in the encoder.

- NTT DOCOMO: Both the channel matrix and eigenvector are assumed for evaluating the performance of AI/ML-based compression/reconstruction.

- Charter: The UE compresses the best singular vector of channel matrix and sends it back to the gNB. In turn, the gNB can use the compressed PMI to calculate the beamforming vectors.

As a difference with R16/17 subband based CSI compression method, Apple raises to use channel state estimation from CSI-RS directly as the input to the AI/ML model to avoid calculating the per-subband eigenvector.

Option 3: Legacy PMI (e.g., Type I, Type II CSI). LG

- LG: In Rel-16/17 Type II CSI, the linear basis, i.e. DFT vector, is utilized for frequency domain compression in order to reduce payload of Rel-15 Type II CSI. So, AI/ML can be used to further reduce the payload of legacy CSI and increase the performance.

Pre-processing

Pre-processing to build a smaller AI/ML model

- vivo: Precoders of each subband can be transformed into angle-delay domain with selected beam and delay. Reduction of 90% AI model size and at the cost of about 5% performance loss is observed by evaluations results. In addition, for generalization performance, the small AI/ML model can achieve better generalization performance than the normal AI/ML model without pre-processing.

Pre-processing to adapt different input/output dimensions (e.g., CSI feedback payload)

- vivo: To generalize different payload without training a new AI model, payload truncation is used for different payloads so the length of encoder output can be fixed.

### 3.1-6: Output of the AI/ML model (compressed CSI)

Quantization method for the output of the encoder:

vivo: Study the performance loss caused by post-processing (e.g., quantization) of the compressed CSI

CMCC: Uniform quantization is used in the evaluation

InterDigital: Linear quantization

1st round discussions

### 3.2-1: AI/ML structure for CSI compression

A vast majority of 21 companies discuss to adopt a two-sided AI/ML model for CSI compression sub use cases, including CSI compression in spatial-frequency domain, CSI compression in spatial-frequency-time domain, and joint operation of CSI prediction and CSI compression. An example of the two-sided AI/ML model is auto-encoder structure, where the CSI compression part takes the functionality of the ‘encoder’, and the CSI decompression part takes the functionality of the ‘decoder’.

Note in the 1st round, we may only focus on the inference of the two-sided model in the evaluation. For the model training and model updating, how would the structure be like depends on the discussions on the framework agenda, so we may wait for a relatively clear/stable view in that part before triggering relative discussions in the evaluation agenda.

Let’s see if we can agree on this structure for evaluation of the CSI compression sub use cases.

Proposal 3.2.1: For the evaluation of the AI/ML based CSI compression sub use cases, a two-sided structure is assumed, including an AI/ML-based CSI compression part which is used to compress the original CSI into the compressed CSI for feedback and an AI/ML-based CSI decompression part which is used to decompress the received feedback CSI.

- At least for inference, the CSI compression part is located at the UE side, and the CSI decompression part is located at the gNB side.

| Supporting companies | Xiaomi, vivo Huawei/Hisi, CAICT, MediaTek, Ericsson, CMCC, Nokia/NSB, NVIDIA, InterDigital, Charter, Futurewei, Lenovo, LG, Panasonic, CATT, ZTE Samsung Fujitsu Qualcomm Apple OPPO, Beijing Jiaotong University |
| --- | --- |
| Objecting companies |  |

| Company | View |
| --- | --- |
| Lenovo | We agree to evaluate the two-sided structure but this should not preclude study of other structures |
| Fujitsu | It is the most natural way to deploy the Encoder-decoder structure into the network. We agree to have this configuration for the initial evaluation study. |
| OPPO | We agree that to use this structure for evaluation of the CSI compression sub use case. |
| Ericsson | I have a question whether this Proposal precludes maintaining (or even allowing to slightly increase) the CSI report size compared to legacy baseline. I’m thinking that CSI compression sub use case should also allow for a similar CSI report size as in legacy, but with a clear benefit in MU-MIMO performance due to AI. So “compression” in this proposal shouldn’t be interpreted as UCI overhead reduction, but more generally as a compression of channel information to a more rich/efficient CSI representation. Perhaps we could add a note saying “Compression refers to information and not the UCI payload” |
| Beijing Jiaotong University | Agree with the proposal. |

### 3.2-2: Input of the AI/ML-based CSI compression

5 companies adopt the raw channel matrix as the input of the CSI compression part (encoder), while 13 companies perform some pre-processing to obtain the eigenvector which is then sent as input to the CSI compression part. For both methods, various frequency granularities are assumed (e.g., RB level, subband level); in addition, for the eigenvector method, it may need to further discuss how to handle the multiple rank case.

It would be good for evaluation and calibration purpose if companies may align the input of the AI/ML model, or at least keep limited methods for companies’ report. Let’s see the companies’ views on the input of the CSI compression part.

Question 3.2.1: For the evaluation of the AI/ML based CSI compression sub use cases, if the two-sided structure with a CSI compression part and a CSI decompression part is adopted, do you think the input original CSI type of the AI/ML model should be taken into the EVM? If so, what should be the original CSI type of the CSI compression part?

- Option 1: Raw channel matrix estimated by UE

  - FFS the frequency granularity of the raw channel matrix

- Option 2: Eigenvector of the raw channel matrix estimated by UE

  - FFS the frequency granularity of the eigenvector

  - FFS multi-rank case

- Option 3: No need to capture the input of the CSI compression in the EVM

- Option 4: Other

| Option 1 Supporting companies | Vivo (as an optional), Ericsson, Nokia/NSB, NVIDIA, InterDigital, Charter, Futurewei, Panasonic, ZTE Fujitsu Apple, Beijing Jiaotong University |
| --- | --- |
| Option 2 Supporting companies | Xiaomi, Vivo (as a mandatory) Huawei/Hisi, CAICT, MediaTek, Ericsson (note that it should be eigenvectors, i.e. in plural, otherwise only rank 1 can be supported), CMCC, Nokia/NSB, NVIDIA, InterDigital, Fraunhofer, Charter, Futurewei, LG, Panasonic, CATT, ZTE Fujitsu Apple OPPO, Beijing Jiaotong University |
| Option 3 Supporting companies | Lenovo Samsung Qualcomm |
| Option 4 Supporting companies |  |

| Company | View |
| --- | --- |
| Xiaomi | The input CSI type should be specified, the eigenvector could be a starting point. For the FFS part, as many evaluation details need to be aligned when the number of eigenvector is more than 1. For example, in case of rank=2, whether the AI model parameters for layer 1 and layer 2 should be trained jointly or separately. Therefore, RI is recommended to be fixed to 1 for eigenvector generation. |
| vivo | Option 2 can be considered as a baseline and option 1 can be FFS. Eigenvector represents the channel characteristics, whereby Rel-16 Type II codebook has been designed. Moreover, the dimension of Eigenvector is usually smaller than that of raw channel fading matrices, resulting in less input data of AI models. Therefore, it can be used as a starting point of AI/ML models. |
| Huawei/Hisi | Compressing the raw channel may need more discussions on: whether the compression applies to original channel matrix (including large scale information) or normalized channel matrix (only small scale information); whether/how the CQI/RI needs to be fed back as in the legacy, etc. In addition, if the raw channel matrix is fed back to gNB, it will change the legacy precoder generation method, e.g., the SVD decomposition has to be performed at the gNB side to obtain the eigenvector. |
| CAICT | We prefer to focus on option 2 first for directly performance comparison with legacy schemes. |
| MediaTek | We prefer option 2. Don’t need to use multiple options for evaluation. |
| Ericsson | All options needs to be studied. Each company report the pre-processing method they used to obtain the results. (Option 2 note that it should be eigenvectors, i.e. in plural, otherwise only rank 1 can be supported) |
| CMCC | Option 2 should be adopted since the benchmark CSI feedback schemes usually take eigenvectors to characterize the channel. It is more straightforward. |
| NVIDIA | Option 1 is important for understanding the full benefit of compressing raw channel, compared to the existing setup of type I/II codebook based on eigenvectors. |
| InterDigital | Ok with either option for calibration. For performance evaluation, we shouldn’t limit any options though. |
| Fraunhofer | The pre-processing of the channel can be considered as the first part of the algorithm. Also, most benchmark methods consider eigenvectors as input, so it is easier for comparison. |
| Charter | Both Option 1 and 2 can be reported by companies. |
| Futurewei | Both options should be supported. Companies should be given the flexibility of choosing their own inputs to the AI/ML model; thus, option 3 is also ok. For convenience of the EVM, it is ok to prioritize one option while leave the other one as optional. |
| Lenovo | Option 3: The input of the model should be suitable to the proposed model and it can be different between proposals. Selecting one particular input type restrict exploration of different schemes. However, all proposals should be evaluated against the same set of KPIs. |
| LG | Option 2 is slightly preferred. |
| Panasonic | Either Option 1 or Option 2 or both could be considered. |
| CATT | We prefer to consider eigenvector as the original CSI type of the AI/ML model. In order to reduce the complexity of AI model, the frequency granularity of the eigenvector can be per sub band. We are open to discuss whether raw channel or other channel pre-processing mechanisms can be considered. |
| ZTE | We think both Option 1 and Option 2 are important sub use cases of CSI compression. Note that the raw channel in Option 1 can also be in time domain in addition to frequency domain, which is an option for further discussed. |
| Samsung | It is beneficial for companies to have flexibility in choosing the input type for their AI/ML modeled. However, companies should be encouraged to report the input type they have considered for reproducibility and better interpretation of their results. |
| Fujitsu | The raw channel matrix and its eigenvectors are two options. In particular, eigenvectors of the channel matrix is the source of the traditional codebook approach, which makes it easy for the performance comparison with the AI/ML approach. In addition, the flexibility introduced by the AI/ML allows us to choose the raw channel matrix as the AI/ML input as well. |
| Qualcomm | In R15/16/17 CSI, RAN1 only specifies the final form of the CSI feedback, for example the PMI, e.g., W or W1*W2 or W1*W2*Wf and does not discuss how to derive it. Similarly, only the output of the CSI decompression part needs to be agreed. In general, the input to the compression part (encoder) and output of the decompression part (decoder) need not be of the same type. For example, the encoder input could be the raw channel matrix and the decoder output could be the eigen vector of the channel. It would be beneficial to leave it to companies to study how to select the best input to the encoder model, and instead only agree on the output of the decoder model. |
| OPPO | We support Option 2 as the original CSI type in current stage of this SID. Specifically, we suggest that the frequency granularity of the eigenvector is sub-band level. Moreover, we also support to study the multi-rank case in this sub use case. |
| Intel | We think it should be the output of the study – not input |
| Beijing Jiaotong University | Both Option 1 and Option 2 should be considered |

Some company mention other pre-processing methods except for the SVD decomposition to generate the eigenvector. Let’s hear more voices from companies on whether/how to determine other pre-processing methods in the EVM.

Question 3.2.2: For the evaluation of the AI/ML based CSI compression sub use cases, except for the eigenvector generation if needed, what other pre-processing do you think is beneficial to be taken as EVM?

| Company | View |
| --- | --- |
| vivo | The main benefits of pre-processing are to reduce the model size and realize the generalization. In case of antenna port generalization, for instance, the dimension of input data may be different, e.g., 32 ports and 16 ports. By pre-processing, each subband can be transformed into angle-delay domain and 8 strongest beams can be extracted from 16 ports and 32 ports. This ensures the fixed size of input data in AI models. |
| CAICT | The eigenvector could be generated based on the output of channel estimation and no need for other pre-processing. |
| MediaTek | We support vivo’s suggestion to reduce the model size and feedback overhead. |
| Ericsson | Identifying these is part of the study. Companies should describe what they used so others can verify their results. |
| Nokia/NSB | We prefer not to capture detailed types of pre-processing in the EVM. Companies are to elaborate on the pre-processing used in their results |
| NVIDIA | Companies can present this as part of their respective AI/ML model evaluation. |
| Futurewei | In general, pre-processing (including data transformation) is considered as part of the data preparation or feature engineering task, and it is considered as implementation specific, e.g., some data transformations may help AI/ML model perform better. Besides raw input data, either CSI or eigenvectors, we believe pre-processing aspects should not be considered as part of the EVM. |
| Lenovo | Another option that can be considered is DFT processing of the raw channel matrix, based on legacy PMI reporting. This can also be used as a baseline for evaluation However, as discussed in Q3.2.1, proponents of a AI/ML model should provide details of the pre-processing functions used. Selecting one particular preprocessing function can restrict exploration of different schemes. |
| LG | Agree with Ericsson, it can be part of study. |
| ZTE | We think a eType II-like PMI is another pre-processing option for reference. That is, UE firstly conducts the compression from spatial and/or frequency domain to get the eType II-like PMI, which can be further discussed and determined. |
| Samsung | In our view, pre-processing schemes are up-to companies’ choices. Companies are encouraged to report them so that the associated computational complexity is considered. However, we do not see the need to specify/list pre-processing schemes in the EVM. |
| Qualcomm | The pre-processing should be left to companies and should not be specified in the EVM. We agree that identifying this is part of the study. |
| OPPO | We agree this is a good starting point. Other aspects can be further considered if needed in future study. |

3 companies raise the joint compression of spatial-frequency and time domain, including 2 companies mention the historical CSI can be involved in the compression, and 1 company mention the future (predicted) CSI can be involved in the compression. Let’s see if the time domain CSI is involved in the CSI compression, if there is something additional that can be captured into the EVM.

Question 3.2.3: For the evaluation of the AI/ML based CSI compression sub use cases, if time domain CSI is involved in the CSI compression, what additional information related to the time domain CSI do you think is beneficial to be taken as EVM?

| Company | View |
| --- | --- |
| Huawei/Hisi | The dataset for training AI/ML model should include time domain information, for example, the sample of dataset can be collected by N consecutive CSI measurement intervals. |
| CAICT | We think channel prediction and CSI compression are two sub use case and could be treaded separately. If we treat channel prediction and CSI compression separately, whether the input of eigenvector for compression is after channel prediction or not will not affect the design of AI encoder and decoder. |
| Ericsson | Identifying these is part of the study. Companies should describe what they used so others can verify their results. |
| Nokia/NSB | We prefer not to consider time-domain in CSI compression sub-use case as this is currently considered in Rel-18 MIMO WI, hence we do not have yet any legacy scheme to compare to |
| NVIDIA | Companies can present this as part of their respective AI/ML model evaluation. |
| Lenovo | Since a parallel discussion on time-domain CSI compression is ongoing in AI 9.1.2, we suggest reusing the EVM agreed in AI 9.1.2 for time domain CSI-specific configuration parameters to reduce the burden of simulation efforts |
| Samsung | It is beneficial to align the EVM considered for enhancement of Type II CSI in Rel-18 MIMO WI. As an example, the following additional considerations can be made: - The configuration of resources periodicity for the case of SP CSI-RS resources. The configuration of burst of CSI-RS resources for AP CSI-RS resources. |
| OPPO | From our view, we think that spatial-frequency domain CSI compression should be evaluated firstly in the initial stage of this SID. The time domain CSI involved for CSI compression can be remained for further study in the second phase. |

### 3.2-3: Output of the AI/ML-based CSI compression

3 companies mentioned the detailed method to quantize the output of the AI/ML-based encoder to a bit sequence, which is an example of post-processing. Some company mentioned other post-processing methods besides the quantization. Let’s hear more voices from companies on whether/how to determine post-processing methods in the EVM.

Question 3.2.4: For the evaluation of the AI/ML based CSI compression sub use cases, what post-processing do you think is beneficial to be taken as EVM?

| Company | View |
| --- | --- |
| vivo | The main benefit of post-processing is to generalize the payloads quantized to fit the UCI container. This requires one encoder and multiple corresponding decoders, and realizes the generalization for the adaption of different feedback payloads. For more details, the decoder may receive the truncated feedback payloads and reconstruct the CSI. In the training procedure, multiple decoders are jointly trained together with a single encoder, so as to optimize the loss function. |
| CAICT | The design of quantizer for AI/ML decoder could left for implementation. |
| Ericsson | Identifying these is part of the study. Companies should describe what they used so others can verify their results. |
| Nokia/NSB | We do not see a need to include post-processing in the EVM assumptions. Companies are to elaborate on the adopted pre- and post-processing used to obtain their results |
| NVIDIA | Companies can present this as part of their respective AI/ML model evaluation. |
| Fraunhofer | The loss caused by different quantization methods should be studied. |
| Futurewei | From EVM perspective, post processing needs depend on what the AI/ML model output is and what is eventually needed to derive the agreed performance metrics (e.g., cosine similarity or NMSE calculated using either eigenvectors or raw CSI). If companies choose to apply some data pre-processing/transformation, some post-processing may be needed. Like pre-processing, this can be left to company’s choice. |
| Lenovo | Proponents of a AI/ML model should provide details of the post-processing functions used for their proposal. Selecting one particular post-processing function can restrict exploration of different schemes. |
| LG | It can be part of study. |
| CATT | Scalar quantization can be adopted as baseline. Vector quantization of the output can be considered. AI/ML-based quantization is also worthy of exploration. |
| Samsung | Similar to preprocessing, post-processing schemes are up-to companies’ choices. Companies are encouraged to report them so that the associated computational complexity is considered. However, we do not see the need to specify/list pre-processing schemes in the EVM. |
| Qualcomm | The quantization techniques to derive a bit representation from the encoder output should be left to companies. |
| OPPO | We agree that different quantization schemes may have great effect on AI/ML performance for CSI compression. However, from our opinion, the quantization layer is part of encoder and dequantization layer is part of decoder, which should be jointly trained during AI/ML model training phase. It is not suitable to evaluate the quantization post-processing independently. We support that quantization/dequantization method should be reported by companies as part of AI/ML model. |

### 3.2-4: Others

Question 3.2.5: For the evaluation of the AI/ML based CSI compression sub use cases, what other aspects related with EVM do you think is necessary to be discussed?

| Company | View |
| --- | --- |
| Futurewei | When using NN-based AI/ML model, it usually comes with some randomness even if you set the random seed. To avoid any misleading evaluation result or misunderstanding, it is recommended to provide results from multiple runs (e.g., via CDF plot) by using the same data to train the same AI/ML model architecture multiple times and evaluate using the save-aside testing data, where the testing data can be fixed across runs. |
| Qualcomm | The final output of the CSI decompression part needs to be agreed in the EVM. |

2nd round email discussions

The discussions seem diverse in the 1st round. No high priority issues to be discussed in the 2nd round.

### 3.3-1: AI/ML structure for CSI compression

It looks the proposal 6.1.3 is still controversial on its language, so Moderator tries to bring it back to discussion in the 2nd round.

@Companies, the Qualcomm’s version is copied from email thread, which receives echo from a couple of companies.

Proposal 6.1.3: For the evaluation of the AI/ML based CSI compression sub use cases, a two-sided structure is assumed prioritized, including an AI/ML-based CSI compression generation part to generate the CSI feedback which is used to compress the original CSI into the compressed CSI for feedback CSI and an AI/ML-based CSI decompression reconstruction part which is used to decompress reconstruct the received feedback CSI feedback. - At least for inference, the CSI compression generation part is located at the UE side, and the CSI decompression reconstruction part is located at the gNB side.

For your easy review, a clean version is provided in below. Please provide your comments and suggested changes if any on top of Proposal 3.3.1.

Proposal 3.3.1 Proposal 6.1.3: For the evaluation of the AI/ML based CSI compression sub use cases, a two-sided structure is prioritized, including an AI/ML-based CSI generation part to generate the CSI feedback and an AI/ML-based CSI reconstruction part which is used to reconstruct the received CSI feedback.

- At least for inference, the CSI generation part is located at the UE side, and the CSI reconstruction part is located at the gNB side.

| Supporting companies | Ericsson, vivo, CATT, CAICT, Fujitsu (with a minor change), NVIDIA, Nokia/NSB, Lenovo (with added phrase for clarity), Qualcomm (minor edit for clarity), InterDigital |
| --- | --- |
| Objecting companies |  |

| Company | View |
| --- | --- |
| Ericsson | We support the current version even if it is not perfect. We understand that there is some confusion what is “CSI” in this new context. A backup (if consensus cannot be reached) could be to fall back to something like: For the evaluation of the AI/ML based CSI enhancement sub use cases, a two-sided structure is prioritized, which includes an AI/ML-based generation part and a corresponding AI/ML-based reconstruction part. - At least for inference, the generation part is located at the UE side, and the reconstruction part is located at the gNB side. |
| vivo | We supportive of this version. Only one rewording is, “generate the CSI feedback” should be “generate the CSI feedback information”, and “reconstruct the received CSI feedback” should be “reconstruct the received CSI feedback information”. |
| CATT | Generally fine. OK with vivo’s minor update. |
| CAICT | We are general fine. |
| Fujitsu | We suggest a minor change to Proposal 3.3.1. For the evaluation of the AI/ML based CSI compression sub use cases, a two-sided structure is prioritized, including an AI/ML-based CSI generation part to generate the CSI feedback and an paired AI/ML-based CSI reconstruction part which is used to reconstruct the received CSI feedback. |
| Xiaomi | Support in principle. For CSI generation part and CSI reconstruction part, they can be replaced with encoder and decoder according to our common understanding. |
| NVIDIA | Support this proposal to move forward, despite the wording is not perfect but acceptable. |
| MediaTek | We are mostly fine with the proposal, but what does “prioritized” mean? Do we have other option? |
| Nokia/NSB | It’s not clear if this proposal applies to all the CSI enhancement sub-use cases or just the “CSI compression” sub-use case We agree with Qualcomm and others that we should reuse well defined and established CSI-related terminology whenever possible. We also agree with Ericsson that by “CSI compression” we actually mean “CSI feedback calculation” and compression does not necessarily imply that we only aim at reducing the overhead of legacy CSI reporting, but rather at improving the performance-overhead trade-off. We suggest this rewording: Proposal 3.3.1: For the evaluation of the AI/ML based CSI compression sub-use case, a two-sided structure is prioritized, including an AI/ML-based CSI encoder to generate the CSI feedback and an AI/ML-based CSI decoder to reconstruct the received CSI feedback. - At least for inference, the CSI encoder is located at the UE side, and the CSI decoder is located at the gNB side. |
| Lenovo | For the evaluation of the AI/ML based CSI compression sub use cases, a two-sided structure is prioritized, including an AI/ML-based CSI generation part to generate the CSI feedback and an AI/ML-based CSI reconstruction part which is used to reconstruct the CSI from the received CSI feedback. - At least for inference, the CSI generation part is located at the UE side, and the CSI reconstruction part is located at the gNB side. |
| Qualcomm | We support the proposal above with a minor wording change for clarity: For the evaluation of the AI/ML based CSI compression sub use cases, a two-sided structure is prioritized, including an AI/ML-based CSI generation part to generate the CSI feedback and an AI/ML-based CSI reconstruction part which is used to reconstruct the CSI from the received CSI feedback. - At least for inference, the CSI generation part is located at the UE side, and the CSI reconstruction part is located at the gNB side. We would like to summarize here, the reason behind the wording changes we had proposed earlier. We believe it would be good to avoid introducing new terms, if possible, because doing so may cause confusion or may inadvertently restrict the scope of the study. In general, the input to the AI/ML model on the UE side may not be the same as the target output of the AI/ML model on the gNB side. For example, the UE-side input could be the raw channel and the gNB-side output could be the eigen vector. |

### 3.3-2 ~ 3.3-4: [Void]

3rd round email approval

To reduce the work load and save time for GTW discussions, the relatively stable proposal (delivered for May 13 GTW) is provided in this section for email approval.

So, the new proposal is updated on top of Proposal 6.2.7 with the changes from the comments of the 2nd round captured:

Proposal 3.4.1 Proposal 6.2.7: For the evaluation of the AI/ML based CSI compression sub use cases, a two-sided structure is prioritized, including an AI/ML-based CSI generation part to generate the CSI feedback information and a paired an AI/ML-based CSI reconstruction part which is used to reconstruct the CSI from the received CSI feedback information.

- At least for inference, the CSI generation part is located at the UE side, and the CSI reconstruction part is located at the gNB side.

| Supporting companies | Vivo, Nokia/NSB, MediaTek, NVIDIA, DCM, Samsung, OPPO, Panasonic, InterDigital, CATT, Fujitsu Huawei/Hisi, Qualcomm, Spreadtrum, Lenovo, CMCC |
| --- | --- |
| Objecting companies |  |

| Company | View |
| --- | --- |
| Moderator | Note: If no objection received till the email approval deadline of UTC 8:00 May 17, this proposal will be delivered for email approval by Mr. Chair. |
| Apple | Need further discussion. As discussed in our paper in agenda 4.2.2.2, it is not clear how CQI calculation at UE side. Based on current our paper, and discussion in email so far, we see different options: - UE perform decoder inferencing as well in order to calculate CQI. The CQI has the same definition as traditional CQI with PMI. - UE calculate CQI assuming perfect eigen-vector. - In case encoder input is channel, and decoder output is precoder matrix, as commented by QC in email, not sure whether UE follow (1) or just use wideband CQI for feedback. If (1) is used, the above definition of two side is not accurate. Instead, to explore all opens, we propose to reuse the two-side model definition which is proposed in general section (two-sided model: A pair of AI/ML models over which joint inference is performed). Proposal 3.4.1: For the evaluation of the AI/ML based CSI compression sub use cases, a two-sided model structure is prioritized, including an AI/ML-based CSI generation part to generate the CSI feedback information and an paired AI/ML-based CSI reconstruction part which is used to reconstruct the CSI from the received CSI feedback information. |
| Moderator | @Apple so in your case (1), the AI/ML model inference is performed at only UE (no AI model needed at gNB), right? Not sure other companies’ views, so still keep the original proposal as it is, except for a minor change on removing ‘paired’, as comments from QC in email reflector. |
| vivo | We are fine with this proposal. We believe, “paired” is a redundant wording, and should be removed. |
| LG | We are generally fine with proposal, but we suggest some modification as follow: Proposal 3.4.1: For the evaluation of the AI/ML based CSI compression sub use cases, a two-sided structure is prioritized, including an AI/ML-based CSI generation part to generate the CSI feedback information and an paired AI/ML-based CSI reconstruction part which is used to reconstruct the CSI from the received CSI feedback information. |
| ZTE | We are generally fine with this proposal and ‘paired’ may be redundant and can be removed. |
| Moderator | @LG what is the difference with the original version? |
| CATT | Support FL’s proposal. |
| Qualcomm | We support the proposal. Regarding Apple’s comment above, we believe CQI should be a separate topic. The current proposal says that the decoder is at the gNB for inference. The wording does not preclude other scenarios related to the CQI options. |
| Ericsson | Ok, prefer not to complicate the current discussion with CQI discussion |

3rd round email discussions

Some high priority questions/proposals are given in the 3rd round.

### 3.5-1: Input of the AI/ML-based CSI compression

In the 1st round, a number of companies hold the view that the input type of the AI/ML model should be a part of the study, and is encouraged to be reported by companies. So let’s see if this proposal is acceptable for companies.

Proposal 3.4.2: For the evaluation of the AI/ML based CSI compression sub use cases, if the two-sided structure with an AI/ML-based CSI generation part and a paired AI/ML-based CSI reconstruction part is adopted, companies are encouraged to report the input CSI type of the CSI generation part, including at least one of the following:

- Option 1: Raw channel matrix estimated by UE

  - Note: Companies to report the frequency granularity of the raw channel matrix

- Option 2: Eigenvector(s) of the raw channel matrix estimated by UE

  - Note: Companies to report the frequency granularity of the eigenvector

  - Note: Companies to report eigenvectors for multi-rank

- Other options are not precluded

| Supporting companies | Apple, LG, CAICT, ZTE, MediaTek, Futurewei, NVIDIA, DCM, Samsung, OPPO, Panasonic, CATT, Xiaomi Huawei/Hisi, Qualcomm, Spreadtrum, Lenovo (with a modification) , CMCC, Ericsson |
| --- | --- |
| Objecting companies |  |

| Company | View |
| --- | --- |
| vivo | We need to define what exactly the input CSI first is, i.e., prior or posterior to the pre-processing. The eigenvector(s) is just the representation of raw channel matrix through the pre-processing. |
| CAICT | Option 2 is proposed. |
| ZTE | We think there can be an Option 3 for the input CSI type of the CSI generation part, which is Raw channel matrix/eigenvectors compressed in spatial/frequency/time domain. Our proposed option can filter some unnecessary components, such as spatial/frequency vectors and layers, so that AI/ML-based compression may be more efficient. |
| OPPO | We support Option 2 since it can be directly compared with Type II baseline. |
| Fujitsu | We support both the two options. |
| Qualcomm | The word “paired” can be removed as it is implied by “two-sided structure”. |
| Lenovo | In general, we agree that companies are encouraged to optionally report the details of the model that they have used. We do not even need to limit this proposal to the case of two-sided structure and it can a general proposal (like what has been suggested in 9.2.1) Our modified proposal is: For the evaluation of the AI/ML based CSI compression sub use cases, companies are encouraged to report the details of their model such as, the structure, the input CSI type (e.g., raw estimated channel matrix, Eigenvector(s) of the estimated channel matrix), data pre-processing/post-processing , the loss function. |
| CMCC | We prefer Option 2. It is more straightforward to compare with the benchmark CSI methods. |
| Ericsson | In more detail, we suggest that Type-II based CSI processing (simplified, e.g. W1 projection), can be used for pre-processing. |

### 3.5-2: Pre/Post-processing of the AI/ML-based CSI compression

Similarly to the input type, the pre-processing/post-processing of the input CSI are to be reported by companies.

Proposal 3.4.3: For the evaluation of the AI/ML based CSI compression sub use cases, if the two-sided structure with an AI/ML-based CSI generation part and a paired AI/ML-based CSI reconstruction part is adopted, companies are encouraged to report the pre-processing/post-processing of the input CSI of the CSI generation part and/or the pre-processing/post-processing of the output CSI of the CSI reconstruction part.

| Supporting companies | Apple, vivo, LG, CAICT, MediaTek, Futurewei, NVIDIA, DCM, Samsung, OPPO, Panasonic, InterDigital, CATT, Xiaomi Huawei/Hisi, Qualcomm, Spreadtrum, Lenovo (can be combined with Proposal 3.4.2), CMCC, Ericsson |
| --- | --- |
| Objecting companies |  |

| Company | View |
| --- | --- |
| vivo | It is worth noting that pre-processing/post-processing methods could be applied not only to the input CSI of the CSI generation part and the output CSI of the CSI reconstruction part as mentioned in the proposal, but also the output of the CSI generation part and/or the input of the CSI reconstruction part. Therefore, we propose to re-wording the corresponding part as “…report the pre-processing/post-processing of the CSI generation part and/or the pre-processing/post-processing of the CSI reconstruction part”. |
| ZTE | From our perspective, pre-processing needs to be clarified whether some operations, e.g. transforming to other domain, are included. |
| Futurewei | We are ok with the proposal in general. Companies may provide pre/post processing if they wish. |
| OPPO | We agree with this proposal. However, from our opinion, we suggest that in the initial stage of this SID, companies are encouraged to use the basic AI/ML solution without extra pre/post-processing methods for calibration. |
| Qualcomm | The word “paired” can be removed as it is implied by “two-sided structure”. |
| Lenovo | It can be combined with Proposal 3.4.2 (shown in green in our response to Proposal 3.4.2) |
| Ericsson | It is also useful if the proponents report whether there is any side information about preprocessing, normalization etc that needs to be reported in UCi to the gNB. This needs to be taken into account in the UCI overhead. |

4th round email approval

Two changes based on inputs from companies: 1) two-sided structuretwo-sided model to be consistent with 9.2.1. 2) prioritized  considered as a starting point based on comments from ZTE, AT&T.

Proposal 3.6.1: For the evaluation of the AI/ML based CSI compression sub use cases, a two-sided structure model is considered as a starting point prioritized, including an AI/ML-based CSI generation part to generate the CSI feedback information and an AI/ML-based CSI reconstruction part which is used to reconstruct the CSI from the received CSI feedback information.

- At least for inference, the CSI generation part is located at the UE side, and the CSI reconstruction part is located at the gNB side.

| Supporting companies | Futurewei, InterDigital, LG, CAICT, OPPO, MediaTek, vivo,Apple, Qualcomm, CMCC, Lenovo, DCM, Fujitsu, Panasonic, CATT, ZTE, Xiaomi, Samsung |
| --- | --- |
| Objecting companies |  |

| Company | View |
| --- | --- |
| vivo | We prefer to keep the original working, “prioritized”. |
| Qualcomm | We agree with the proposal. |
| ZTE | We generally agree with this proposal for progress, and prefer this wording considered as a starting point. At this early stage, we don't think any scheme should be prioritized. So, we would like to suggest a separate proposal for single-sided sub use case, because it doesn’t have conflicts with two-sided model on evaluation KPIs. We think single-sided model has the same priority to be evaluated and studied. |
| Samsung | OK |

4th round email discussions

From the 3rd round discussion, Moderator tends to accept a more generic way from Lenovo, to provide a candidate list for companies to report.

Upd Proposal 3.7.1: For the evaluation of the AI/ML based CSI compression sub use cases, companies are encouraged to report the details of their model, including:

- The structure of the AI/ML model

- The input CSI type, e.g., raw channel matrix estimated by UE, eigenvector(s) of the raw channel matrix estimated by UE, etc.

- The out CSI type, e.g., reconstructed channel matrix, reconstructed eigenvector(s), etc.

- Data pre-processing/post-processing

- Loss function

- Others are not precluded

| Supporting companies | NVIDIA, Futurewei, InterDigital, LG, CAICT, OPPO, MediaTek, vivo, Apple, Qualcomm, Ericsson, CMCC, Lenovo, DCM, Fujitsu, Panasonic, CATT, ZTE, Xiaomi(with comments) |
| --- | --- |
| Objecting companies |  |

| Company | View |
| --- | --- |
| Futurewei | In general, we are ok with the proposal. Companies may share certain level of AI/ML model related information if they wish and decide what details are considered as implementation-specific or proprietary, which they don’t wish to share, e.g., customized loss function may be considered as implementation-specific or proprietary. |
| Qualcomm | We agree with the proposal. The output CSI type can also be added to the list since input and output CSI type may not be the same. |
| Ericsson | With structure of the model, our interpretation is the type (CNN, RNN,Transfomer, Inception, …) the number of layers, branches, real valued or complex valued parameters etc. It is also important to report whether the gNB needs side information about the pre-processing at the UE. |
| Moderator | Updates based on QC comment. |
| ZTE | We are fine with the proposal. Some AI/ML model related information can be shared for reference. |
| Xiaomi | We have one comment on the input CSI type. It is not clear how to define the raw channel in evaluation. If we analog BF is considered, whether the channel matrix generated by TR38.901 or channel matrix after analog BF is considered as raw channel. |
| Samsung | Support assuming this is optional. Typo: out CSI output CSI |

5th round email approval

Two changes based on inputs from companies: 1) two-sided structuretwo-sided model to be consistent with 9.2.1. 2) prioritized  considered as a starting point based on comments from ZTE, AT&T.

Proposal 3.8.1 Proposal 3.6.1: For the evaluation of the AI/ML based CSI compression sub use cases, a two-sided structure model is considered as a starting point prioritized, including an AI/ML-based CSI generation part to generate the CSI feedback information and an AI/ML-based CSI reconstruction part which is used to reconstruct the CSI from the received CSI feedback information.

- At least for inference, the CSI generation part is located at the UE side, and the CSI reconstruction part is located at the gNB side.

| Supporting companies | Futurewei, InterDigital, LG, CAICT, OPPO, MediaTek, vivo, Apple, Qualcomm, CMCC, Lenovo, DCM, Fujitsu, Panasonic, CATT, ZTE, Xiaomi, Samsung, Nokia/NSB |
| --- | --- |
| Objecting companies |  |

| Company | View |
| --- | --- |
| Moderator | @vivo As a number of companies are still in favor of the original wording, could you live with it? |
| vivo | We are fine with this wording. |
| Samsung | Fine. |

Upd Proposal 3.8.2 Proposal 6.3.6: For the evaluation of the AI/ML based CSI compression sub use cases, companies are encouraged to report the details of their models, including:

- The structure of the AI/ML model, e.g., type (CNN, RNN, Transfomer, Inception, …), the number of layers, branches, real valued or complex valued parameters, etc.

- The input CSI type, e.g., raw channel matrix estimated by UE, eigenvector(s) of the raw channel matrix estimated by UE, etc.

  - FFS: if raw channel is selected, the input CSI is obtained from the channel raw channel matrix is with or without analog BF

- The output CSI type, e.g., channel matrix, eigenvector(s), etc.

- Data pre-processing/post-processing

- Loss function

- Others are not precluded

| Supporting companies | NVIDIA, Futurewei, InterDigital, LG, CAICT, OPPO, MediaTek, vivo, Apple, Qualcomm, Ericsson, CMCC, Lenovo, DCM, Fujitsu, Panasonic, CATT, ZTE, Xiaomi(with comments), Nokia/NSB |
| --- | --- |
| Objecting companies |  |

| Company | View |
| --- | --- |
| Moderator | Updates based on QC, Xiaomi, Ericsson. |
| Xiaomi | Thanks for capturing our concern, for the FFS part, we think the ‘if raw channel is selected’ should be removed, as the eigenvector is also derived based on the raw channel. |
| Qualcomm | We agree with the proposal. |
| Moderator | Updates based on Xiaomi. |

Specific evaluation methodology for CSI prediction sub use case

Summary of views from companies

### 4.1-1: Companies that evaluated/considered CSI prediction: vivo, Nokia, Samsung, LG, FUTUREWEI, MediaTek

CSI prediction on time domain: vivo, Nokia, Samsung, MediaTek

- vivo: For CSI predicting of +5ms and the input CSI is raw channel, the NMSE/Cosine similarity of AI/ML-based prediction is ~-10dB/~0.93 while the baseline without CSI prediction is ~-1.3dB/~0.76.

- Nokia: It is observed that for nomadic users channel prediction horizons of 15 to 30 ms for a very low NMSE of -20 dB based on a limited number of channel observations; the baseline of Kalman filtering is 7~10dB higher.

- Samsung: AI/ML based CSI prediction achieves ~-8dB avg. NMSE with a comparison of ~4.8dB for baseline sample-and-hold predictor avg. NMSE.

- MediaTek: Depending on the requirements on CSI prediction, for example the required prediction length, AI/ML-based solutions may show superior performance compared to classical non-AI based methods as the prediction length increases.

CSI prediction on spatial domain: LG

- LG: UE measures CSI based on the transmitted CSI-RS, and estimate/predict the full CSI for configured BW based on the AI/ML. Thus, the overhead of CSI-RS can be reduced. Take a full 16 port CSI-RS for instance, based on gNB configuration and/or AI/ML, the actual transmitted CSI-RS can be reduced by half of full CSI-RS ports (i.e., 8 port) and can be transmitted to UE. Then, UE will measure 8-port CSI-RS and recover the 16-port channel via AI/ML.

### 4.1-2: AI/ML structure for CSI prediction

One-sided structure is adopted where the AI/ML is operated at either gNB or UE: vivo, Nokia, Samsung

### 4.1-3: AI/ML model adopted in evaluations

vivo: FCN

Nokia: RNN / LSTM

Samsung: 3D-CNN

### 4.1-4: Simulation approach

Option 1: Link level simulation is adopted: vivo

Option 2: System level simulation is adopted: Nokia, Samsung

For Option 1 and Option 2, the companies provide the generic EVM as follows.

- vivo: For Option 1, a set of basic evaluation parameters needs to be aligned as baseline

| The number of transmit antenna | 32 |
| --- | --- |
| The number of receive antenna | 2 |
| Carrier frequency | 3GHz |
| Subcarrier spacing | 30kHz |
| PRB number | 52 |
| speed | 30km/h |
| LLS parameters | CDL-C, delay spread=300ns |
| Length of one sample in time domain | 200 slot (100ms) |
| The number of historical CSI inputs | 12 |
| The slot indices of historical CSI inputs | [71, 81, 91, …, 181]-th slots (spacing is 10 slots) |
| The number of future CSIs to be predicted | 1~5 |
| The slot indices of the predicted CSI | [183, 185, 187, 189, 191]-th slots (i.e., [+1ms, +2ms, +3ms, +4ms, +5ms]) |
| The type of CSI | Raw channel, Eigenvector (or singular vector) |

- Nokia: For Option 2, the Rel-16 SLS assumptions for CSI feedback are an appropriate starting point for SLS assumptions for CSI prediction. The Rel-16 EVM can refer to [28].

- Samsung: For Option 2, following parameters are given as EVM.

- UE speed of 30 km/h

- 3GPP UMi channel model

- Carrier frequency of 2.1 GHz

- Channel bandwidth of 20 MHz

- gNB has Nt = 32 transmit antennas and Nr = 4 receive antennas

- K = 52 resource blocks

- CSI-RS periodicity of 5 ms.

### 4.1-5: Modeling of the UE mobility/trajectory

Option 1: No explicit trajectory modeling (Reflected by Doppler shift which can refer to Section 7.5 of TR 38.901). Samsung

- Samsung: The angular and power properties of multi-path clusters, and the rays (paths) stay the same between two-channel measurements

Option 2: Linear trajectory (refer to mobility modeling for intra-cell mobility scenarios for Rel-17 Multi-beam enhancement [29])

Option 3: Spatial consistency modeled in Section 7.6.3 of TR38.901. Nokia

- Nokia: Since users will be in motion, the channel will need to vary with position. We propose to use the spatial consistency procedure A defined in TR 38.901.

### 4.1-6: Input CSI information of the AI/ML model for CSI prediction

Three candidate options are raised by companies for analysis.

Option 1: Raw channel matrix vivo Samsung Nokia

Option 2: Eigenvector vivo

Option 3: CSI feedback information Nokia

Company views on the three options:

- vivo: As per the evaluation between Option 1 and Option 2, it is observed that Option 1 outperforms Option 2 in terms of NMSE and Cosine Similarity.

- Nokia: Number of pre-measured channels (observation time), feedback of CSI (e.g., when prediction is performed at the gNB)

- Samsung: Each raw CSI sample is a 3-D array with a dimension of (4, 32, 52)

### 4.1-7: Others

Number of historical CSI occasions

vivo evaluates 2, 4, 6, 12, 18 historical CSI occasions for inferring the future CSI. As per the evaluation results, it is observed that the prediction performance can be improved by increasing the number of historical CSI inputs, while the increased complexity is analyzed as marginal.

Prediction of multiple future CSIs

vivo evaluates the case where the AI/ML model outputs multiple future CSIs where the future time indices are fixed. The motivation of providing multiple future CSIs is the scheduling for transmission can be more flexible. Both extrapolation (1-step) and interpolation (2-steps) approaches are evaluated, observed with similar performance on NMSE and Cosine Similarity.

Generalization of future CSIs for CSI prediction

- Generalization over frequency PRBs

vivo evaluates the generalization of the AI/ML model for CSI prediction to assess AI/ML model for CSI prediction achieves similar performance on different frequency PRBs/subbands. The evaluation is performed by training the AI/ML model on 1PRB and apply it to another PRB with different frequency locations of the bandwidth, and it is observed that the generalization over frequency PRBs is good in terms of NMSE and Cosine Similarity.

- Generalization over channel parameters

vivo raised the generalization with respect to channel parameters should also be evaluated.

- Generalization of percell/UE model

vivo evaluates the generalization of the Per-UE AI/ML model and Per-cell AI/ML model. From the evaluation result, it is observed that Per-UE AI/ML model outperforms Per-cell AI/ML model in terms of NMSE and Cosine Similarity, with the disadvantage of higher overhead for training/delivering/storage.

- Fine-tuning

vivo evaluates the performance of fine-tuning to overcome the generalization issue, where compared to the direct training of multiple Per-UE models, the fine-tuning is achieved by using fewer labeled data based on a common model (e.g., the Per-cell model). For AI/ML-based CSI prediction, the label of data can be collected in real time by the CSI measurement. From the evaluations results, the predicted CSI with fine-tuning outperforms that without fine-tuning in terms of NMSE and Cosine Similarity.

1st round discussions

### 4.2-1: AI/ML structure for CSI prediction

For the AI/ML-based CSI prediction functionality, 6 companies mention/evaluate this sub use case, where 4 companies raise the CSI prediction in time domain, and 1 company raise the CSI prediction in spatial domain. Let’s see if we can converge on limited sub use cases to perform evaluation to save effort in future.

Question 4.2.1: If the AI/ML based CSI prediction sub use cases are to be evaluated, what prediction method do you think can be considered?

- Option 1: CSI prediction in time domain

  - Note: predict future CSI is based on historical CSI

- Option 2: CSI prediction in spatial domain

  - Note: predict CSI of a larger number of CSI ports based on CSI of a small number of CSI ports

- Option 3: Other

| Option 1 | Supporting companies | vivo Huawei/Hisi, CAICT, MediaTek, Ericsson, Nokia/NSB (with lower priority compared to CSI compression), NVIDIA, InterDigital, Lenovo, LG, Panasonic, CATT Samsung Fujitsu Apple OPPO, Intel |
| --- | --- | --- |
|  | Objecting companies |  |
| Option 2 | Supporting companies | InterDigital, LG |
|  | Objecting companies | Ericsson |
| Option 3 | Supporting companies | ZTE |
|  | Objecting companies |  |

| Company | View |
| --- | --- |
| vivo | For the legacy CSI feedback procedure, the CSI measurement, CSI feedback and DL transmission utilizing the CSI feedback for precoding are conducted at different time (slots). If the CSI feedback from previous time is directly used to generate DL precoding, the spectral efficiency will be reduced due to the channel aging, especially for high mobility scenarios. AI-based CSI prediction is a way to solve this problem. Various evaluations show that AI-based CSI prediction outperforms the non-prediction case and non-AI based CSI predictions. Furthermore, the AI-based CSI prediction is also an approach for reducing the RS overhead and feedback frequency. For the viewpoint of studying the life cycle management of CSI feedback enhancement, finally, CSI prediction could serve as a typical sub use case to study the performance and specification impact of finetuning. Therefore, we propose to study the sub use case of AI/ML for CSI prediction with high priority. |
| CAICT | Option 1 is preferred. Option 2 belongs to AI/ML for channel estimation. |
| MediaTek | Time prediction can combat CSI aging and we believe is worth studying. |
| Ericsson | Option 2 is not commercially interesting. The CSI-RS overhead is not an issue in current networks. |
| NVIDIA | CSI prediction in time domain is a use case of practical relevance. |
| InterDigital | Not sure if we need to preclude any sub use case at this point. |
| Lenovo | We support Option 1. In our understanding, prediction only applies to time domain. Inference of CSI of a larger no. of CSI ports based on CSI of a small number of CSI ports should be under CSI feedback compression |
| LG | Agree with InterDigital, it is premature to preclude any sub-use cases at this stage. |
| ZTE | Option 3: CSI prediction in spatial & frequency domain Different from Option 2, we think frequency domain can be taken into consideration in addition to spatial domain. With AI/ML models, we think AI/ML models are possible to recover and predict more channel information. |
| Qualcomm | Our preference is to focus on CSI compression sub-use-case. Regarding time domain CSI prediction, it would be beneficial to wait for the study on CSI reporting enhancements by exploiting time domain correlation in the R18 work item on MIMO evolution before studying ML-based channel prediction. Regarding spatial domain CSI prediction, we prefer to study it in a future release. |
| OPPO | We think that Option 1 with CSI prediction in time domain can be considered. Option 2 can be remained for FFS in the future. |

As per the description of majority companies, a one-sided AI/ML model is assumed, where it can be located at either UE or gNB. Let’s see if we can converge on the structure of the AI/ML model.

Proposal 4.2.1: If the AI/ML based CSI prediction sub use cases are to be evaluated, a one-sided structure is assumed, where the AI/ML inference is performed at either gNB or UE.

| Supporting companies | vivo Huawei/Hisi, CAICT, Ericsson, Nokia/NSB, InterDigital, Panasonic, CATT, ZTE Samsung Fujitsu OPPO, Intel |
| --- | --- |
| Objecting companies | MediaTek, NVIDIA, Lenovo Qualcomm |

| Company | View |
| --- | --- |
| vivo | We are fine with the proposal. |
| CAICT | We think AI/ML inference is performed at UE side is better. |
| MediaTek | We believe it is too early to limit the scope of CSI prediction to one-sided models only. For example, an autoencoder-like model can be used where the UE sends latent information about the CSI, and the gNB uses that information to predict future CSI. |
| Ericsson | The single sided solutions should be part of the study together with the dual sided solutions, and not excluded at this point. |
| Nokia/NSB | It makes sense to consider this one-sided structure first for simplicity and leave other more complicated structures for later studies |
| NVIDIA | It does not appear necessary to make such restriction at this early stage of the study item. |
| Lenovo | In our view a one-sided structure is a more probable solution, but at this stage of SI we should not limit the proposals and companies should be able to propose any method. Again of course, all methods should be evaluated against similar set of KPIs which include performance, overhead, complexity, generalization ability and others |
| CATT | If AI/ML based CSI prediction is supported, we are fine to focus on sub use cases with one-side structure. However, the sub use cases with no spec impact are not preferred. |
| ZTE | We think AI/ML inference can be performed at gNB because network is normally more powerful than UE in terms of storage, computation capability, power consumption. With AI/ML models, we think network are capable of recovering and predicting more channel information from UE reports. |
| Qualcomm | It is not clear why two-sided structure should be excluded at this point. |
| OPPO | From our view, we think AI/ML inference performed at either gNB or UE is acceptable, which should be further evaluated on common EVM in the future. |

### 4.2-2: Input of the AI/ML-based CSI prediction

Three candidate options are raised by companies as the input for CSI prediction. Let’s see the companies’ views on the input of the CSI prediction.

Question 4.2.2: If the AI/ML based CSI prediction sub use cases are to be evaluated, do you think the input type of the AI/ML model should be taken into the EVM? If so, what should be the input of the CSI prediction?

- Option 1: Raw channel matrix measured by UE (e.g., if the AI/ML model is located at UE side)

- Option 2: Eigenvector of the raw channel matrix (e.g., if the AI/ML model is located at UE side)

- Option 3: CSI feedback information (e.g., if the AI/ML model is located at gNB side)

- Option 4: No need to capture the input of the CSI prediction in the EVM

- Option 5: Other

| Option 1 Supporting companies | Vivo, CAICT, MediaTek, Ericsson, NVIDIA, InterDigital, LG, Panasonic Fujitsu Apple OPPO |
| --- | --- |
| Option 2 Supporting companies | Ericsson Fujitsu OPPO |
| Option 3 Supporting companies | Ericsson, NVIDIA, InterDigital, LG, Panasonic OPPO |
| Option 4 Supporting companies | Ericsson, Nokia/NSB, Lenovo Samsung Qualcomm |
| Option 5 Supporting companies |  |

| Company | View |
| --- | --- |
| vivo | To calibrate the performance of the AI-based CSI prediction, it is essential to align the input type of the model. AI-based CSI prediction can be conducted at both the gNB and UE while the suitable input type of model is different. It is worth noting that UE-based CSI prediction is more beneficial than the gNB-based one due to the availability to the labeled data and less information loss in the historical CSIs. Furthermore, UE is available for raw channel matrix (may be noisy). In our contribution, the AI-based CSI prediction based on the raw channel outperforms that based on eigenvector. The additional operation (also increases processing complexity) of eigen value decomposition may lead to irregular phase reversal and loss of time varying characteristic compared to the raw channel. Therefore, the UE based CSI prediction using the raw channel matrix as the input is suggested. |
| CAICT | We proposed to treat CSI prediction and CSI compression as two sub use cases and option 1 is preferred. |
| MediaTek | We prefer to evaluate CSI prediction based on raw CSI input since it contains the most information, without loss, quantization, or distortion. However, we are open to other options. |
| Ericsson | All options are on the table. It is up to each company to study different approaches and report along with their results whether they used Option 1,2,3 or 5. |
| Nokia/NSB | We do not think capturing the preprocessing in the EVM is needed at this early stage |
| NVIDIA | All options can be explored. Option 1 and Option 3 are baseline options if down selection is needed. |
| Lenovo | At this stage companies should be allowed to use any input data as long as they are evaluated against similar set of KPIs. |
| CATT | If the AI/ML model is located at UE side, we prefer to consider eigenvector as the original input type of the AI/ML model. If the AI/ML model is located at gNB side, we are OK to consider CSI feedback information as the original input type of the AI/ML model.We are open to discuss whether other information can be considered. |
| Fujitsu | Similar to our corresponding view in the CSI compression, we think that both raw channel matrix and its eigenvectors can be used as AI/ML input for the use case of CSI prediction. |
| Qualcomm | The input to the AI/ML model should be left to companies. |
| OPPO | We think that both Option 1, Option 2 and Option 3 can be considered in current stage, And which kind of CSI prediction solution can achieve the best performance requires further studies, and need to be down-selected for final evaluation. |
| Intel | This should be output of study |

### 4.2-3: UE trajectory modeling

2 companies raise the UE mobility/trajectory modeling. Let’s hear more voices from companies on whether/how to model UE trajectory.

Question 4.2.3: If the AI/ML based CSI prediction sub use cases are to be evaluated, whether/how to model UE trajectory?

- Option 1: No explicit trajectory modeling (UE mobility reflected by Doppler shift)

- Option 2: Linear trajectory reusing Rel-17 Multi-beam enhancement

- Option 3: Spatial consistency reusing Section 7.6.3 of TR38.901

- Option 4: Other

| Option 1 | Supporting companies | Vivo, Nokia/NSB, Lenovo, CATT Qualcomm Apple |
| --- | --- | --- |
|  | Objecting companies |  |
| Option 2 | Supporting companies |  |
|  | Objecting companies |  |
| Option 3 | Supporting companies | Nokia/NSB (procedure A only) |
|  | Objecting companies |  |
| Option 4 | Supporting companies | CAICT Samsung |
|  | Objecting companies |  |

| Company | View |
| --- | --- |
| vivo | The timescale for CSI prediction is typically on the order of slots, where the movement of UE is marginal, and the Doppler shift can be seen as constant. Therefore, the channel aging can be reflected by the doppler shift and the non-explicit trajectory modeling is enough. |
| Huawei/Hisi | Whether to model the spatial consistency can be discussed based on more input from companies, e.g., if some company performs the spatial consistency modeling and shows the performances are quite close between the channel models with and without spatial consistency modeling, then there is no need to model it; otherwise it may be needed. |
| CAICT | No strong view and if 5ms CSI feedback is used, there is no strong motivation to model UE trajectory. |
| MediaTek | Trajectory modeling is important only if the prediction horizon is long enough such that the UE considerably changes its position. Further study is required to identify whether trajectory modeling is needed. |
| Nokia/NSB | We don’t think spatial consistency model is needed as the targeted medium UE speed for this study should not be sufficiently high to cause a change in location during the CSI prediction event. Optionally, companies may use spatial consistency model A |
| NVIDIA | Further study is needed, e.g., check if the spatial consistency models in TR 38.901 are accurate or not. |
| Lenovo | We support option 1. A UE moving with speed of 100 km/h is expected to move <3m within 100ms. For an outdoor layout with >200m. ISD, the dominant paths are not expected to vary significantly within the 100ms, which is a reasonable window for CSI prediction. Given that, we prefer not to consider spatial consistency |
| LG | Further study is needed. |
| CATT | We prefer Option 1. Considering practical UE speed and CSI reporting interval, the UE location would not change much between two CSI reporting instance. UE mobility modelled by Doppler shift would be sufficient. |
| Samsung | In our view, the UE mobility (trajectory) modeling is dependent on whether link-level or system-level evaluation is considered. In our view, for beam-management consideration of spatial consistency is a must as beam dwelling time is relatively longer and the UE would cover relatively longer distance. For CSi enhancement with temporal-compression, however, we are interested in a much shorter duration, channel stationary time which is in the order of 100ms. For moderate mobility (up to 30kmhr), dataset can be generated via LLS without defining UE trajectory and by relying on Doppler components of channel modeling in Section 7.5 of TR 38.901. In particular, independent snap-shots of the channel over “channel stationary time” can be used as a dataset. For Phase II evaluation via SLS, we may consider spatial consistency as the simulation duration per each drop is usually in order of a few seconds. |
| OPPO | We are open to consider the UE trajectory modeling approaches. Evaluation complexity needs to be considered. |

### 4.2-4: Other additional EVMs for CSI prediction

Baseline CSI prediction scheme for performance comparison

3 companies discuss the baseline non-AI/ML based CSI prediction schemes as a performance comparison. Let’s see whether/how to consider the non-AI/ML CSI prediction scheme as a baseline in the CSI prediction sub use cases.

Question 4.2.4: If the AI/ML based CSI prediction sub use cases are to be evaluated, whether/how to take a baseline non-AI/ML based CSI prediction scheme for performance comparison?

- Option 1: Nearest historical CSI (no prediction)

- Option 2: Kalman filtering

- Option 3: CSI prediction scheme discussed in R18 MIMO WI

- Option 4: Other

| Option 1 | Supporting companies | vivo Huawei/Hisi, CAICT, Ericsson, Nokia/NSB, NVIDIA Samsung Apple |
| --- | --- | --- |
|  | Objecting companies |  |
| Option 2 | Supporting companies | MediaTek, Nokia/NSB (optional), LG Fujitsu OPPO |
|  | Objecting companies |  |
| Option 3 | Supporting companies | MediaTek, InterDigital, Lenovo, LG Samsung Qualcomm |
|  | Objecting companies | Ericsson |
| Option 4 | Supporting companies | Samsung, Fujitsu: ARIMA method |
|  | Objecting companies |  |

| Company | View |
| --- | --- |
| vivo | The nearest historical CSI (i.e., no prediction) can be used as the baseline, which can be aligned easily. It may be difficult to align on the non-AI predictors (e.g., Kalman filtering or extrapolation) since these approaches are not specified in the standards. Furthermore, the complexity comparison between the proposed AI/ML schemes and non-AI predictors is also difficult. As for the Option 3, strictly speaking, the case studied in R18 MIMO is a temporal compression rather than a time domain prediction. And the case studied in R18 MIMO is also a study item. We think it is no need to wait for the result of another study item. |
| Huawei/Hisi | As Option 2 performance quite depends on the specific algorithm, the baseline may hardly calibrated. For Option 3, as the R18 MIMO is a parallel ongoing topic, it will wait for long time after the results from there is stable (same view as vivo). |
| CAICT | We prefer option 1 as baseline. |
| MediaTek | Simply using the nearest historical CSI for benchmarking is unfortunately not sufficient to warrant the adoption of AI/ML-based CSI prediction. Instead, a fair assessment of AI/ML-based CSI prediction should include a comparison to a classical non-AI/ML solution that attempts to solve the same problem. Such classical solution could be based on techniques like Kalman Filters, Auto-Regression, etc. However, given that the R18 MIMO WI is currently discussing CSI prediction under the CSI Enhancement objective (AI 9.1.2), it is reasonable to assume that the best classical solution 3GPP should consider for benchmarking is the one it already adopts. Until the MIMO CSI enhancement objective is concluded, temporary solutions could be adopted for benchmarking, so that this AI is not stalled. |
| Ericsson | Option 3 does not make sense since we don’t want dependencies in this SI on work in other WIs |
| Nokia/NSB | We don’t think option 3 is viable as the schemes for Rel-18 MIMO WI may be finalised too late for this study item. Besides Type-II enhancement in time/Doppler may not focus on prediction as this is either UE or gNB implementation, but rather in time/Doppler domain compression |
| NVIDIA | Option 1 is a straightforward baseline that is easy be used. |
| InterDigital | Rel-18 MIMO will cover CSI prediction in time domain based on Type II codebook which should be a baseline for the group to make a conclusion whether AI based solution justifies specification impacts or not. Given that Rel-18 MIMO is ongoing WI, we can simply down-prioritize AI/ML based CSI prediction in time domain in Rel-18 and any baseline (Option 1 or Option 2) may be used just for evaluation. |
| Lenovo | Prefer reusing prospective Rel-18 MIMO baseline for AI-based CSI prediction to avoid duplicated discussions. We can prioritize CSI compression EVM and defer CSI prediction for one or two meetings, until discussions in Rel-18 MIMO CSI prediction are more mature. |
| LG | Agree with option 1 can be simple baseline, but comparison perspective, it seems very lower bound. So, this can be further studied including option 3. |
| CATT | The best schemes supported by Rel-17 and Rel-18 spec. shall be used as baseline including standard-transparent AI/ML based prediction. |
| Samsung | Additionally, a genie-aided CSI can be considered as an upper-bound. |
| Fujitsu | To have a fair comparison, we think that taking a non-AI method with prediction capability as a baseline is necessary. According to our initial study results, AI method cannot win non-AI method in all the evaluation scenarios. |
| Qualcomm | We propose to wait for the conclusion of the R18 MIMO WI before studying ML based CSI prediction. |
| OPPO | We think that Option 2 is an appropriate baseline for CSI prediction in current stage. Based on this, we are open to further consider Option 3 in the future. |

Time related assumptions

2 companies discuss the historical CSI window (in terms of slots) on which the CSIs are input to the AI/ML model, and the predict time which corresponds to the future CSI. Let’s see whether/what time related assumptions for CSI prediction (especially in time domain) sub use case are additionally needed to be taken into the EVM.

Question 4.2.5: If the AI/ML based CSI prediction sub use cases are to be evaluated, whether/what time related assumptions are needed to be taken into the EVM?

- Option 1: Historical time window

- Option 2: Predict time which is the future time slot corresponding to the predicted CSI

- Option 3: No need to capture the time related assumptions of the CSI prediction in the EVM

- Option 4: Other

| Option 1 Supporting companies | Vivo, MediaTek, NVIDIA Samsung |
| --- | --- |
| Option 2 Supporting companies | Vivo, CAICT, MediaTek, NVIDIA, CATT Samsung |
| Option 3 Supporting companies | Nokia/NSB Fujitsu |
| Option 4 Supporting companies | MediaTek |

| Company | View |
| --- | --- |
| vivo | In our contribution, the impact of the number of historical CSIs is evaluated, which shows that the prediction accuracy increases as the number of historical CSIs increases. Furthermore, the performance of AI-based prediction is also impacted by the choice of future time slots to be predicted. The accuracy reduces with the increase of time interval between the predicting time and time corresponding to historical CSIs. Therefore, to calibrate the performance of the AI-based CSI prediction, it is essential to align both the historical time window and the future time slots to be predicted. |
| CAICT | We think the prediction time should align with the time slot that CSI is used for scheduling. |
| MediaTek | Timing related assumptions are crucial for meaningful evaluation of CSI prediction models. The duration of the input CSI, as well as the CSI sampling period will determine the quality of CSI prediction. Prediction models should strive to extend their prediction time horizon for as long as possible. Tradeoffs between the input duration, sampling rate and output prediction length should be studied. Moreover, limits on inference time for CSI prediction should also be specified such that the predicted CSI is available in a timely manner. |
| Nokia/NSB | These assumptions can be elaborated upon by companies presenting their results |
| NVIDIA | Prediction is naturally based on a past time window and carried out for future time slot(s). |
| CATT | In our opinion, at least option 2 is needed. We are open to discuss whether option 1 and other information is needed. |
| Fujitsu | No need to have such restriction at this stage. |
| OPPO | Needs further study. We are open to discuss the options. |

### 4.2-5: Others

Question 4.2.6: For the evaluation of the AI/ML based CSI prediction sub use cases, what other aspects related with EVM do you think is necessary to be discussed?

| Company | View |
| --- | --- |
| vivo | 1. The generalization of AI-based CSI prediction should be studied. As shown in our contribution, the generalization capability of AI-based CSI prediction is different over frequency PRBs, channel parameters and scenarios. The generalization capability will influence the choice of model selection, model transfer, model inference, model updating and model storage. Therefore, we suggest studying the generalization performance of AI-based CSI prediction. 2. The finetuning process of AI-based CSI prediction should be studied, which is an important process of life cycle management. For AI-based CSI prediction, the label of data can be collected in real time by the CSI measurement. Therefore, if the predicted future CSI is on the time occasion of a CSI measurement, finetuning is available for the AI-based CSI prediction. This is significantly different from other use cases where the label is hard to be collected in real time. By finetuning, scenario-specific model can be derived using a few online collected data, which can further improve the prediction performance in the real system. |
| Futurewei | We suggest focusing on the CSI feedback compression/reconstruction sub use case in the study. We don’t support including the CSI prediction sub use case in Rel-18 study. |

2nd round email discussions

Some high priority questions/proposals are given in the 2nd round.

### 4.3-1: AI/ML structure for CSI prediction

For the candidate sub use case for evaluation/study, it seems the majority companies are in favor of the CSI prediction in time domain. So the proposal is raised to take time domain prediction as a starting point (if CSI prediction is to be valuated), while other potential sub use cases can be FFS.

Proposal 4.3.1: If the AI/ML based CSI prediction sub use cases are to be evaluated, consider CSI prediction in time domain (predict future CSI based on historical CSI) as a starting point

- FFS whether/how to evaluate other CSI prediction methods, e.g., CSI prediction in spatial domain/ frequency domain, etc.

| Supporting companies | Apple, Ericsson, OPPO, Samsung, LG Huawei/Hisi, CMCC, vivo, CATT, CAICT, Fujitsu, Xiaomi, NVIDIA, MediaTek, Nokia/NSB, Lenovo |
| --- | --- |
| Objecting companies |  |

| Company | View |
| --- | --- |
| Samsung | We are open to study prediction in spatial and frequency domains but this may have some overlap with CSI compression spatial-frequency domains. |
| Futurewei | We prefer only focusing on CSI feedback compression/reconstruction sub use case. |
| Fujitsu | Considering the evaluation workload, we prefer to focus on the sub use case of time domain CSI prediction only, if CSI prediction is chosen as a sub use case. |
| Xiaomi | We think CSI prediction in time domain is enough for study the sub use case of CSI prediction, other prediction methods is considered as low priority. |
| NVIDIA | Support this proposal to move forward. |
| ZTE | We basically agree with this proposal. From our perspective, CSI prediction in spatial or/and frequency domains is also an important use case, which can be considered as a starting point as well. For example, UE may be possible to only report parts of compressed spatial&frequency DFT vectors and corresponding weighting coefficients. Then, when network use AI/ML model to recover whole spatial&frequency DFT vectors and corresponding weighting coefficients. So, we suggest CSI prediction in spatial or/and frequency domains should be discussed and determined. |
| MediaTek | Support. We also think CSI prediction in time is enough. |
| Nokia/NSB | We are open to studying CSI prediction sub-use case but with lower priority compared to CSI compression |
| Lenovo | Support in general, however it is not clear what “CSI prediction in spatial domain/frequency domain” is. In our understanding, “prediction” is related to a future event. I guess what is meant by “CSI prediction in spatial domain/frequency domain” is inference of CSI at port x, sub-band y given CSI at port z, sub-band w. Such scenario should be categorized under CSI compression sub use-cae, in our opinion |
| Qualcomm | Our preference is to focus on CSI compression sub-use-case. Considering that the proposal wording says, “If the AI/ML based CSI prediction sub use cases are to be evaluated…”, it may be better to first discuss whether CSI prediction sub use cases are to be evaluated before discussing the specifics. |
| InterDigital | We also prefer to focus on CSI compression sub use case in Rel-18 considering that Rel-18 MIMO is working on CSI prediction based on Type-II codebook. |
| DCM | We prefer to prioritize CSI compression in the SI. To fairly compare the performance of AI/ML model, a reasonable baseline technique should be assumed. Since the CSI enhancement in time domain will be studied in Rel-18 MIMO WI, where CSI prediction will also be considered, it could be worth putting off time domain CSI prediction study until identifying the gain of scheme without AI/ML model to be agreed in the Rel-18 MIMO WI. |

### 4.3-2: Input of the AI/ML-based CSI prediction

No high priority issue to be discussed in this round.

### 4.3-3: UE trajectory modeling

It looks more companies are in favor of Option 1, i.e. no trajectory modeling, so let’s see if this can be accepted.

Proposal 4.3.2: If the AI/ML based CSI prediction sub use cases are to be evaluated, for the CSI prediction in time domain, no explicit trajectory modeling is considered as a starting point, i.e., UE mobility reflected by Doppler shift.

| Supporting companies | Apple, Ericsson, OPPO, Samsung, LG Huawei/Hisi, CMCC, vivo, CATT, CAICT, Fujitsu, Xiaomi, MediaTek, Nokia/NSB, Lenovo, DCM |
| --- | --- |
| Objecting companies |  |

| Company | View |
| --- | --- |
| Samsung | We want to clarify our intention here. For sub-use cases that consider temporal-correlation of CSI, i.e., CSI prediction, spatial-frequency-time compression, the training/testing dataset can be generated without explicit definition of UE trajectory. |
| CMCC | Doppler shift seems enough to reflect the CSI change in time domain. |
| Nokia/NSB | We don’t think a trajectory model is needed as the targeted UE speed for this study should not be sufficiently high to cause a change in location during the CSI prediction event. Optionally, companies may use spatial consistency model A/B |
| Qualcomm | Our preference is to focus on CSI compression sub-use-case. It may be better to first discuss whether CSI prediction sub use cases are to be evaluated before discussing the specifics. |

### 4.3-4: Other additional EVMs for CSI prediction

No high priority issue to be discussed in this round.

### 4.3-5: Others

No high priority issue to be discussed in this round.

3rd round email approval

To reduce the work load and save time for GTW discussions, the relatively stable proposal (No objection received in the 2nd round) is provided in this section for email approval.

### 4.4-1: AI/ML structure for CSI prediction

Note that as 1) there are still companies have concern on the CSI prediction sub use case, and 2) preliminary simulation results are limited, ‘If the AI/ML based CSI prediction sub use cases are to be evaluated’ is used here, which means whether to further study/evaluate the CSI prediction need more input from companies to make it convincing; but as long as we agree on studying/evaluating this sub use case in future, the CSI prediction in time domain would be a starting point.

Proposal 4.4.1 Proposal 4.3.1: If the AI/ML based CSI prediction sub use cases are to be evaluated, consider CSI prediction in time domain (predict future CSI based on historical CSI) as a starting point

- FFS whether/how to evaluate other CSI prediction methods, e.g., CSI prediction in spatial domain/ frequency domain, etc.

| Supporting companies | Apple, vivo, LG, Ericsson, CAICT, Nokia/NSB, MediaTek, NVIDIA, DCM, Samsung, OPPO, Panasonic, CATT, Fujitsu, Lenovo, CMCC, Ericsson |
| --- | --- |
| Objecting companies | InterDigital, Qualcomm |

| Company | View |
| --- | --- |
| Moderator | Note: If no objection received till the email approval deadline of UTC 8:00 May 17, this proposal will be delivered for email approval by Mr. Chair. |
| ZTE | We basically agree with this proposal. From our perspective, CSI prediction in spatial or/and frequency domains is also an important use case, which can be considered as a starting point as well. Similar to CSI prediction in time domain, we suggest another proposal can be listed in parallel for comments as follows: If the AI/ML based CSI prediction sub use cases are to be evaluated, consider CSI prediction in spatial and/or frequency domain as a starting point - FFS details for CSI prediction in spatial and/or frequency domain |
| NTT DOCOMO | It is recommend that companies give clear decription of assumption of baseline for comparison when they provide the simulation results for CSI prediction in time domain. |
| InterDigital | Not sure what is the intention/benefit of this proposal without reaching consensus to study CSI prediction in Rel-18. |
| Qualcomm | It would be best to first discuss and agree whether ML-based CSI prediction sub use case is to be studied before discussing the specific details for evaluation. Considering that Rel-18 MIMO WI will be studying aspects related to CSI prediction, it would be beneficial to wait for the outcome of that study before studying ML-based CSI prediction. |
| Ericsson | It can be up to each company to present solutions and results with prediction. These can be captured separately in the TR. |

### 4.4-2: UE trajectory modeling

The proposal 4.3.2 in the 2nd round discussion is pasted in below with only changing the proposal number.

Proposal 4.4.2 Proposal 4.3.2: If the AI/ML based CSI prediction sub use cases are to be evaluated, for the CSI prediction in time domain, no explicit trajectory modeling is considered as a starting point, i.e., UE mobility reflected by Doppler shift.

| Supporting companies | Apple, vivo, LG, CAICT, Nokia/NSB, MediaTek, DCM, Samsung, OPPO, Panasonic, CATT, Fujitsu, Lenovo, CMCC |
| --- | --- |
| Objecting companies | InterDigital, Qualcomm |

| Company | View |
| --- | --- |
| Moderator | Note: If no objection received till the email approval deadline of UTC 8:00 May 17, this proposal will be delivered for email approval by Mr. Chair. |
| InterDigital | Similar comment to the proposal 4.4.1. This seems to be the next step discussion after the group reaches a consensus to study CSI prediction in Rel-18. |
| Qualcomm | It would be best to first discuss and agree whether ML-based CSI prediction sub use case is to be studied before discussing the specific details for evaluation. Considering that Rel-18 MIMO WI will be studying aspects related to CSI prediction, it would be beneficial to wait for the outcome of that study before studying ML-based CSI prediction. |

3rd round email discussions

Some high priority questions/proposals are given in the 3rd round.

### Input of the AI/ML-based CSI prediction

Similar suggestion as for the CSI compression sub use case, that CSI prediction input is to be reported by companies.

Proposal 4.5.1: If the AI/ML based CSI prediction sub use cases are to be evaluated, companies are encouraged to report the input CSI type of the CSI generation part, including at least one of the following:

- Option 1: Raw channel matrix estimated by UE

- Option 2: Eigenvector(s) of the raw channel matrix estimated by UE

- Option 3: CSI feedback information

- Other options are not precluded

| Supporting companies | LG, MediaTek, Futurewei, NVIDIA, DCM, Samsung, OPPO, Panasonic, CATT, Fujitsu Huawei/Hisi, Lenovo (only the first part), CMCC |
| --- | --- |
| Objecting companies | Qualcomm |

| Company | View |
| --- | --- |
| vivo | We are supportive of Option 1 as baseline, but optionally for Option 2. The process of deriving eigenvector(s) can be seen as a pre-processing over raw channel matrix. |
| CAICT | Same view with vivo. |
| ZTE | We think raw channel matrix/eigenvectors compressed in spatial/frequency/time domain can be another option for the input CSI type of the CSI generation part, which needs further discussion. |
| MediaTek | Option 1 is preferred. |
| OPPO | We are both okay for Option 1, 2 and 3. And it requires further study to down-select one Option with better performance. |
| Qualcomm | It would be best to first discuss and agree whether ML-based CSI prediction sub use case is to be studied before discussing the specific details for evaluation. Considering that Rel-18 MIMO WI will be studying aspects related to CSI prediction, it would be beneficial to wait for the outcome of that study before studying ML-based CSI prediction. |
| Lenovo | As we discussed in Q3.4.2, we agree that the companies are encouraged to optionally report the details of the model that they have used. However, at this stage we cannot put restriction. Our modified proposal is: If the AI/ML based CSI prediction sub use cases are to be evaluated, companies are encouraged to report the details of their model such as, the structure, the input CSI type, data pre-processing/post-processing , the loss function. |
| CMCC | We prefer Option 1 while Option 2 is also OK for us. |

Specific evaluation methodology for other sub use cases

1st round discussions

Question 5.1.1: For the evaluation of the AI/ML based CSI feedback enhancement, is there any other sub use case that you think is necessary for EVM discussion and have not been discussed/captured in previous sections?

| Company | View |
| --- | --- |
| Ericsson | In dual sided solutions, such as encoder-decoder in UE and gNB respectively, there is a need to evaluate the performance of the sub use case when UE and gNB comes from different vendors and multi-vendor joint training may be less flexible compared to when a single vendor can train both encoder and decoder jointly. Companies are encouraged to develop an EVM (if any) until next meeting to capture the effect of the multi-vendor solution for this important case. |
| Futurewei | No other use cases should be considered in the study. |
| ZTE | - Enhancements on UL-DL reciprocity: With AI/ML models, we think that UE may be more efficient to recover the real DL channel by measurements from UL channel and measurements reported by UE(e.g. Type I/Type II/eType II/ FeType II PMI). That is, measurements from UL channel and measurements reported by UE are both fed into a AI/ML model, the expected output of the AI/ML model is a precise DL channel. It can be studied if the existing measurement reports and SRS transmission are sufficient in such case. |
| Beijing Jiaotong University | The use case Bit-level DL-based CSI feedback considers the CSI compression problem as a source coding problem, which is the mainstream in the classical separate source channel coding (SSCC) system. However, there are several drawbacks. Firstly, this SSCC scheme has been demonstrated inferior to the joint source-channel coding (JSCC) scheme in the finite block length regime in theory. Secondly, the SSCC scheme has “cliff effect” in the real wireless scenario. That means the reconstruction quality of the CSI drops drastically, if the real feedback channel condition is worse than expected, and beyond the capability of the applied channel coding scheme. In this case, the recovered CSI at the BS is useless for the subsequent process. The JSCC scheme can provide a graceful performance degradation even the real channel condition becomes worse than the expected channel condition, which makes the recovered CSI still valuable for the subsequent process. Lastly, even though the hybrid automatic repeat request (HARQ) mechanism can compensate for errors of channel decoding caused by channel condition mismatch, HARQ inevitably increases the additional feedback overhead and brings the latency problem for the CSI feedback task. All in all, the Symbol-level DL-based CSI feedback built upon the JSCC strategy should be considered. We compare the architecture of the symbol-level DL-based CSI feedback with that of the bit-level DL-based CSI feedback for a better explanation. |

2nd round discussions

No high priority issue to be discussed in this round.

Potential proposals for GTW

Proposals for May 11 GTW

### 6.1-1 Simulation approach

The first and foremost issue is the simulation approach to evaluate the CSI feedback enhancement. More companies are in favor of SLS, and some companies supporting LLS are also fine with SLS. On the other hand, for the companies supporting SLS, some think it will be also good to consider LLS as an optional/complementary approach. In addition, one company raised other open approaches such as virtual world simulation can also be considered for this brand new feature.

Question 2.2.1: For the evaluation of the AI/ML based CSI feedback enhancement, which of the following simulation approach should be adopted: - Option 1: System level simulation - (22 companies) Xiaomi, vivo Huawei/Hisi, CAICT, MediaTek, Ericsson, CMCC, Nokia/NSB, NVIDIA, InterDigital, Fraunhofer, Charter, Futurewei, Lenovo, LG, Panasonic, CATT, ZTE Samsung Fujitsu Qualcomm Apple - Option 2: Link level simulation - (8 companies) Xiaomi, InterDigital, Fraunhofer, Charter, LG, Panasonic Samsung Fujitsu - Option 3: Others

So the proposal is given as (Proposal 6.1.1 is editorially changed on top of Question 2.2.1):

Proposal 6.1.1:Question 2.2.1: For the evaluation of the AI/ML based CSI feedback enhancement, which of the following system level simulation approach should be is adopted as baseline:

- Link level simulation is optionally adopted

- [FFS: whether/how other simulation approach can be considered]

- Option 1: System level simulation

- Option 2: Link level simulation

- Option 3: Others

### 6.1-2 Source to generate the dataset

For the source to generate the dataset, a vast majority companies (if I do not miss) are supporting to adopt the TR 38.901 synthetic models to generate the training/validation/testing dataset as a starting point, while leaving a FFS whether/how to include field test dataset.

Question 2.2.8: For the evaluation of the AI/ML based CSI feedback enhancement, do you agree that the dataset for AI/ML training/validation/testing is generated by TR 38.901 channel models as a starting point? - FFS whether/how to generate the dataset from field test. (Supporting: 22 companies) Xiaomi, vivo Huawei/Hisi, CAICT, MediaTek, Ericsson, CMCC, Nokia/NSB, NVIDIA, InterDigital, Fraunhofer, Charter, Futurewei, Lenovo, LG, Panasonic, CATT, ZTE Samsung Fujitsu Qualcomm Apple (Objecting: 0)

So we may try a quick check if the proposal is agreeable (Proposal 6.1.2 is editorially changed on top of Question 2.2.8).

Proposal 6.1.2 Question 2.2.8: For the evaluation of the AI/ML based CSI feedback enhancement, do you agree that the dataset for AI/ML training/validation/testing is generated by TR 38.901 channel models as a starting point?

- FFS whether/how to generate the dataset from field test.

### 6.1-3 AI/ML structure for CSI compression

For the AI/ML structure for evaluating CSI compression, a vast majority companies (if I do not miss) are supporting the Proposal 3.2.1, which adopts a two-sided AI/ML structure, with an example of auto-encoder model. In the evaluation, we assume the inference parts of CSI compression and CSI decompression para are located at UE and gNB, respectively.

Proposal 3.2.1: For the evaluation of the AI/ML based CSI compression sub use cases, a two-sided structure is assumed, including an AI/ML-based CSI compression part which is used to compress the original CSI into the compressed CSI for feedback and an AI/ML-based CSI decompression part which is used to decompress the received feedback CSI. - At least for inference, the CSI compression part is located at the UE side, and the CSI decompression part is located at the gNB side. (Supporting: 21 companies) Xiaomi, vivo Huawei/Hisi, CAICT, MediaTek, Ericsson, CMCC, Nokia/NSB, NVIDIA, InterDigital, Charter, Futurewei, Lenovo, LG, Panasonic, CATT, ZTE Samsung Fujitsu Qualcomm Apple (Objecting: 0)

So we may try a quick check if the same proposal is agreeable (Proposal 6.1.3 is editorially changed on top of Proposal 3.2.1)

Proposal 6.1.3: Proposal 3.2.1: For the evaluation of the AI/ML based CSI compression sub use cases, a two-sided structure is assumed, including an AI/ML-based CSI compression part which is used to compress the original CSI into the compressed CSI for feedback and an AI/ML-based CSI decompression part which is used to decompress the received feedback CSI.

- At least for inference, the CSI compression part is located at the UE side, and the CSI decompression part is located at the gNB side.

Proposals for May 13 GTW

### 6.2-1 Simulation approach

In the 1st and 2nd round discussions, companies as basically aligned to adopt SLS as baseline, while LLS as optional/complementary approach.

Proposal 2.3.1: For the evaluation of the AI/ML based CSI feedback enhancement, system level simulation approach is adopted as baseline - Link level simulation is optionally adopted - [FFS: whether/how other simulation approach can be considered] Supporting: Apple, Ericsson, OPPO, Samsung (with modification) , Beijing Jiaotong University, LG Huawei/Hisi, Spreadtrum, CMCC, vivo, CAICT, Futurewei, Fujitsu, Xiaomi, NVIDIA, ZTE, MediaTek, Nokia/NSB, Lenovo, Qualcomm, AT&T, InterDigital, DCM Objecting:

On top of that, some companies still want to clarify some simplified evaluation, such as using intermediate KPI (e.g., GCS, NMSE, equivalent MSE, receive SNR, etc.) can be also considered. To Moderator’s understanding, intermediate KPI as discussed in the Metrics section will naturally be part of the EVM, but relatively decoupled with the evaluation approach (SLS/LLS). But if adding that part that can better clarify, we may try to see if it is OK for all.

Note: a new bullet is added:

======================================================================

Version 1 (original version):

Proposal 6.2.1 Proposal 2.3.1: For the evaluation of the AI/ML based CSI feedback enhancement, system level simulation approach is adopted as baseline

- Link level simulation is optionally adopted

Version 2 (FFS added as per CATT suggestion):

Proposal 6.2.1 Proposal 2.3.1: For the evaluation of the AI/ML based CSI feedback enhancement, system level simulation approach is adopted as baseline

- Link level simulation is optionally adopted

- FFS: Simplified evaluation, e.g. intermediate evaluation on AI/ML output, is optionally adopted.

======================================================================

### 6.2-2 Baseline SLS EVM table-overall

For the baseline EVM, it looks most companies are fine/can accept the following table.

Proposal 2.3.2:Question 2.2.2: For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, do you agree the following table is taken as a baseline of EVM? - Note: the following table captures the common parts of the R16 CSI enhancement EVM table and the R17 CSI enhancement EVM table, while the different parts are FFS. - Note: the baseline EVM is used to compare the performance with the benchmark release, while the AI/ML related parameters (e.g., dataset construction, generalization verification, and AI/ML related metrics) can be of additional/different assumptions. - FFS: modifications on top of the following table for the purpose of AI/ML related evaluations. Supporting: Apple, OPPO, Samsung, Beijing Jiaotong University, LG Huawei/Hisi, Spreadtrum, CMCC, vivo, CATT, CAICT, Futurewei, Fujitsu, Xiaomi, NVIDIA, ZTE, MediaTek, Nokia/NSB, Lenovo (with a slight change on note2), Qualcomm, AT&T, InterDigital, DCM Objecting:

Some comments/suggestions include 2 points

- Rank=1 is suggested for 4Rx as optional. – Moderator: ‘Other configuration is not precluded’ does not preclude Rank=1 for 4Rx

- Generalization verification requires more diverse scenarios/configurations, and conclusions should be drawn based on multiple scenarios/configurations. – Moderator: the meaning has been included by the 2nd Note in the original version, i.e., parameters for generalization can be of additional/different assumptions. But if adding this sentence may better clarify, let’s see if other companies can accept.

Lenovo: Note2: the baseline EVM is used to compare the performance with the benchmark release for one scenario/configuration. The conclusions for the use-case in the SI should only be drawn after comparing performance using EVM with multiple scenarios/configurations. The AI/ML related parameters (e.g., dataset construction, generalization verification, and AI/ML related metrics) can be of additional/different assumptions.

The proposal for GTW is therefore provided, including the suggestion from Lenovo which is editorially changed in below.

======================================================================

Proposal 6.2.2: Proposal 2.3.2: For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, the following table is taken as a baseline of EVM

- Note: the following table captures the common parts of the R16 CSI enhancement EVM table and the R17 CSI enhancement EVM table, while the different parts are FFS.

- Note: the baseline EVM is used to compare the performance with the benchmark release, while the AI/ML related parameters (e.g., dataset construction, generalization verification, and AI/ML related metrics) can be of additional/different assumptions.

  - The conclusions for the use cases in the SI should be drawn based on generalization verification over potentially multiple scenarios/configurations.

- FFS: modifications on top of the following table for the purpose of AI/ML related evaluations.

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD (TDD is not precluded), OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) is a baseline. Other scenarios (e.g. UMi@4GHz 2GHz, Urban Macro) are not precluded. |
| Frequency Range |  | FR1 only, FFS 2GHz or 4GHz as a baseline |
| Inter-BS distance |  | 200m |
| Channel model |  | FFS |
| Antenna setup and port layouts at gNB |  | Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ - 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ Other configurations are not precluded. |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for rank > 2 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) Other configuration is not precluded. |
| BS Tx power |  | 41 dBm for 10MHz, 44dBm for 20MHz, 47dBm for 40MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz, optionally 30kHz for 4GHz |
| Number of RBs |  | FFS |
| Simulation bandwidth |  | FFS |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | FFS |
| MIMO layers |  | For all evaluation, companies to provide the assumption on the maximum MU layers (e.g. 8 or 12) |
| CSI feedback |  | Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback) : 5 ms, - Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms |
| Overhead |  | Companies shall provide the downlink overhead assumption (i.e., whether the CSI-RS transmission is UE-specific or not and take that into account for overhead computation) |
| Traffic model |  | FTP model 1 with packet size 0.5 Mbytes as a baseline Other FTP model is not precluded. FFS full buffer |
| Traffic load (Resource utilization) |  | FFS |
| UE distribution |  | - 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic as a baseline FFS ideal channel estimation |
| Evaluation Metric |  | Throughput and CSI feedback overhead as baseline metrics. Additional metrics, e.g., ratio between throughput and CSI feedback overhead, can be used. Maximum overhead (payload size for CSI feedback)for each rank at one feedback instance is the baseline metric for CSI feedback overhead, and companies can provide other metrics. |
| Baseline for performance evaluation |  | FFS |

======================================================================

Next is some FFS parts of the overall EVM table. It looks most companies are fine with it, so it is directly pasted in below for GTW. To be consistent with Proposal 6.2.2, the cyan bullet is added.

======================================================================

Supporting: Apple, OPPO, Samsung, Beijing Jiaotong University, LG Huawei/Hisi, Spreadtrum, CMCC, vivo, CATT, CAICT, Futurewei, Fujitsu, Xiaomi, NVIDIA, ZTE, MediaTek, Nokia/NSB, Lenovo, Qualcomm, AT&T, InterDigital Objecting:

Proposal 6.2.3: Proposal 2.3.3: For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, the following parameters are taken into the baseline of EVM

- Note: The 2nd column applies if R16 TypeII codebook is selected as baseline, and the 3rd column applies if R17 TypeII codebook is selected as baseline.

- Note: the baseline EVM is used to compare the performance with the benchmark release, while the AI/ML related parameters (e.g., dataset construction, generalization verification, and AI/ML related metrics) can be of additional/different assumptions.

  - The conclusions for the use cases in the SI should be drawn based on generalization verification over potentially multiple scenarios/configurations.

- FFS: modifications on top of the following table for the purpose of AI/ML related evaluations.

| Parameter | Value (if R16 as baseline) | Value (if R17 as baseline) |
| --- | --- | --- |
| Frequency Range | FR1 only, 2GHz as baseline, optional for 4GHz. | FR1 only, 2GHz with duplexing gap of 200MHz between DL and UL, optional for 4GHz |
| Channel model | According to TR 38.901 | According to TR 38.901 |
| Number of RBs |  |  |
| Simulation bandwidth | 10 MHz for 15kHz as a baseline, and configurations which emulate larger BW, e.g., same sub-band size as 40/100 MHz with 30kHz, may be optionally considered. Above 15kHz is replaced with 30kHz SCS for 4GHz. | 20 MHz for 15kHz as a baseline (optional for 10 MHz with 15KHz), and configurations which emulate larger BW, e.g., same sub-band size as 40/100 MHz with 30kHz, may be optionally considered. Above 15kHz is replaced with 30kHz SCS for 4GHz |
| MIMO scheme | SU/MU-MIMO with rank adaptation is a baseline for overhead reduction. For low RU, SU-MIMO or SU/MU-MIMO with rank adaptation are assumed for higher rank extension. For medium/high RU, SU/MU-MIMO with rank adaptation is assumed for higher rank extension. SU/MU-MIMO with rank adaptation. Companies are encouraged to report the SU/MU-MIMO with RU | For low RU, SU-MIMO with rank adaptation are assumed For medium/high RU, SU/MU-MIMO with rank adaptation is assumed SU/MU-MIMO with rank adaptation. Companies are encouraged to report the SU/MU-MIMO with RU |
| Traffic load (Resource utilization) | - 50/70 % for CSI overhead reduction - 20/50 % for high rank extension 20/50/70% Companies are encouraged to report the MU-MIMO utilization. | - 70% for SU/MU-MIMO with rank adaptation - 20% for SU-MIMO with rank adaptation 20/50/70% Companies are encouraged to report the MU-MIMO utilization. |

======================================================================

Baseline SLS EVM table-channel estimation

The ‘channel estimation’ entry for the overall EVM.

Proposal 2.3.4: For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, for ‘Channel estimation’, whether ideal DL channel estimation is optionally taken into the baseline of EVM for the purpose of calibration? - Note: Realistic DL channel estimation with error modeling should be used for performance comparison with the benchmark release and drawing SI conclusions. - FFS: the ideal channel estimation is applied for dataset construction, or performance evaluation/inference. Supporting: OPPO, Samsung (with removing FFS), Beijing Jiaotong University, LG Huawei/Hisi, Spreadtrum, CMCC, vivo (with removing for the purpose of calibration), CATT, CAICT, Futurewei, Xiaomi, NVIDIA, ZTE, MediaTek, Nokia/NSB, Lenovo, Qualcomm (comments below), AT&T, InterDigital, DCM Objecting:

Still some views on whether ideal channel estimation is merely used for calibration or other evaluation purpose, and which (ideal/realistic) should be the baseline. So let’s see if we can somehow converge on GTW based on Proposal 6.2.4. Editorial changes are made as per companies’ comments.

======================================================================

Proposal 6.2.4: Proposal 2.3.4: For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, for ‘Channel estimation’, ideal DL channel estimation is optionally taken into the baseline of EVM for the purpose of calibration

- Note: Realistic DL channel estimation with error modeling should be used for performance comparison with the benchmark release and drawing SI conclusions.

- FFS: the ideal channel estimation is applied for dataset construction, or performance evaluation/inference.

======================================================================

Baseline SLS EVM table-Traffic model

The ‘Traffic model’ entry for the overall EVM.

Proposal 2.3.5 Question 2.2.5: For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, for ‘Traffic model’, whether full buffer traffic is optionally taken into the baseline of EVM for the purpose of calibration? - Note: FTP traffic should be used for performance comparison with the benchmark release and drawing SI conclusions. Supporting: OPPO, Samsung, Beijing Jiaotong University, LG Huawei/Hisi, Spreadtrum, CMCC, vivo (with removing for the purpose of calibration), CATT, CAICT, Futurewei, Xiaomi, NVIDIA, MediaTek, Nokia/NSB, Lenovo, Qualcomm, AT&T, InterDigital Objecting:

Similar to channel estimation, some views on whether full buffer is merely used for calibration or other evaluation purpose. So let’s see if we can somehow converge on GTW based on Proposal 6.2.5.

======================================================================

Proposal 6.2.5 Proposal 2.3.5: For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, for ‘Traffic model’, full buffer traffic is optionally taken into the baseline of EVM for the purpose of calibration

- Note: FTP traffic should be used for performance comparison with the benchmark release and drawing SI conclusions.

======================================================================

### 6.2-3: Calibration of dataset and/or AI/ML model

It looks most companies are fine with Proposal 2.3.7, so it is directly pasted in below for GTW.

======================================================================

Supporting: Apple, Ericsson, OPPO, Samsung, Beijing Jiaotong University, LG Huawei/Hisi, Spreadtrum, CMCC, vivo, CATT, CAICT, Futurewei, Fujitsu, Xiaomi, NVIDA, ZTE, MediaTek, Nokia/NSB, Lenovo, Qualcomm, AT&T, InterDigital, DCM Objecting:

Proposal 6.2.6 Proposal 2.3.7: For the evaluation of the AI/ML based CSI feedback enhancement, if the dataset for AI/ML training/validation/testing is generated by TR 38.901 channel models, for the calibration purpose on the dataset and/or AI/ML model over companies, consider to align the parameters (e.g., for scenarios/channels) for generating the dataset in the simulation as a starting point.

======================================================================

### 6.2-4: AI/ML structure for CSI compression

Proposal 3.3.1 is relatively stable in the 2nd round discussion. Some editorial changes in below based on suggestions from companies.

======================================================================

Supporting: Ericsson, vivo, CATT, CAICT, Fujitsu (with a minor change), NVIDIA, Nokia/NSB, Lenovo (with added phrase for clarity), Qualcomm (minor edit for clarity), InterDigital, MediaTek Objecting:

Proposal 6.2.7 Proposal 3.3.1: For the evaluation of the AI/ML based CSI compression sub use cases, a two-sided structure is prioritized, including an AI/ML-based CSI generation part to generate the CSI feedback and an paired AI/ML-based CSI reconstruction part which is used to reconstruct the CSI from the received CSI feedback.

- At least for inference, the CSI generation part is located at the UE side, and the CSI reconstruction part is located at the gNB side.

======================================================================

Proposals for May 18 GTW

### 6.3-1 Baseline SLS EVM table-overall

Proposal 6.3.1: For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, the following table is taken as a baseline of EVM

- Note: the following table captures the common parts of the R16 CSI enhancement EVM table and the R17 CSI enhancement EVM table, while the different parts are FFS.

- Note: the baseline EVM is used to compare the performance with the benchmark release, while the AI/ML related parameters (e.g., dataset construction, generalization verification, and AI/ML related metrics) can be of additional/different assumptions.

  - The conclusions for the use cases in the SI should be drawn based on generalization verification over potentially multiple scenarios/configurations.

- FFS: modifications on top of the following table for the purpose of AI/ML related evaluations.

Table 6.3-1 Baseline SLS EVM table

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD (TDD is not precluded), OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) is a baseline. Other scenarios (e.g. UMi@4GHz 2GHz, Urban Macro) are not precluded. |
| Frequency Range |  | FR1 only, FFS 2GHz or 4GHz as a baseline |
| Inter-BS distance |  | 200m |
| Channel model |  | According to TR 38.901 |
| Antenna setup and port layouts at gNB |  | Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ - 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ Other configurations are not precluded. |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) Other configuration is not precluded. |
| BS Tx power |  | 41 dBm for 10MHz, 44dBm for 20MHz, 47dBm for 40MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz for 2GHz, 30kHz for 4GHz |
| Simulation bandwidth |  | FFS |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | FFS |
| MIMO layers |  | For all evaluation, companies to provide the assumption on the maximum MU layers (e.g. 8 or 12) |
| CSI feedback |  | Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback) : 5 ms, - Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms |
| Overhead |  | Companies shall provide the downlink overhead assumption (i.e., whether the CSI-RS transmission is UE-specific or not and take that into account for overhead computation) |
| Traffic model |  | FFS |
| Traffic load (Resource utilization) |  | FFS |
| UE distribution |  | - 80% indoor (3km/h), 20% outdoor (30km/h) FFS whether/what other indoor/outdoor distribution and/or UE speeds for outdoor UEs needed |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic as a baseline FFS ideal channel estimation |
| Evaluation Metric |  | Throughput and CSI feedback overhead as baseline metrics. Additional metrics, e.g., ratio between throughput and CSI feedback overhead, can be used. Maximum overhead (payload size for CSI feedback)for each rank at one feedback instance is the baseline metric for CSI feedback overhead, and companies can provide other metrics. |
| Baseline for performance evaluation |  | FFS |

| Supporting companies | Apple, vivo (with comments), LG, Ericsson, CAICT, ZTE, Nokia/NSB, mediatek, Futurewei, NVIDIA, DCM, Samsung, OPPO, InterDigital, CATT, Fujitsu, Xiaomi, Huawei/Hisi, Qualcomm, Spreadtrum, Lenovo, CMCC, DCM, Panasonic |
| --- | --- |
| Objecting companies |  |

### 6.3-2 Baseline SLS EVM table-FFS part

Further updates based on Samsung’s comments.

Proposal 6.3.2: For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, the following parameters are taken into the baseline of EVM

- Note: The 2nd column applies if R16 TypeII codebook is selected as baseline, and the 3rd column applies if R17 TypeII codebook is selected as baseline.

  - FFS: Whether the selected baseline for performance evaluation is not related with impacts the R18 dataset construction and/or AI/ML model training

  - FFS baseline for potential sub use cases involving CSI enhancement on time domain

- Note: the baseline EVM is used to compare the performance with the benchmark release, while the AI/ML related parameters (e.g., dataset construction, generalization verification, and AI/ML related metrics) can be of additional/different assumptions.

  - The conclusions for the use cases in the SI should be drawn based on generalization verification over potentially multiple scenarios/configurations.

- FFS: modifications on top of the following table for the purpose of AI/ML related evaluations.

Table 6.3-2 Baseline SLS EVM table-FFS part

| Parameter | Value (if R16 as baseline) | Value (if R17 as baseline) |
| --- | --- | --- |
| Frequency Range | FR1 only, 2GHz as baseline, optional for 4GHz. | FR1 only, 2GHz with duplexing gap of 200MHz between DL and UL, optional for 4GHz |
| Simulation bandwidth | 10 MHz for 15kHz as a baseline, and configurations which emulate larger BW, e.g., same sub-band size as 40/100 MHz with 30kHz, may be optionally considered. Above 15kHz is replaced with 30kHz SCS for 4GHz. | 20 MHz for 15kHz as a baseline (optional for 10 MHz with 15KHz), and configurations which emulate larger BW, e.g., same sub-band size as 40/100 MHz with 30kHz, may be optionally considered. Above 15kHz is replaced with 30kHz SCS for 4GHz |
| MIMO scheme | SU/MU-MIMO with rank adaptation. Companies are encouraged to report the SU/MU-MIMO with RU | SU/MU-MIMO with rank adaptation. Companies are encouraged to report the SU/MU-MIMO with RU |
| Traffic load (Resource utilization) | 20/50/70% Companies are encouraged to report the MU-MIMO utilization. | 20/50/70% Companies are encouraged to report the MU-MIMO utilization. |

| Supporting companies | Vivo, Ericsson, CAICT, ZTE, MediaTek, Futurewei, NVIDIA, Samsung, OPPO, InterDigital, CATT, Fujitsu, Xiaomi Huawei/Hisi, Qualcomm, Spreadtrum, CMCC,Apple, Lenovo, DCM, Panasonic |
| --- | --- |
| Objecting companies |  |

### 6.3-3 Baseline SLS EVM table-Channel estimation

Proposal 6.3.3: For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, for ‘Channel estimation’, ideal DL channel estimation is optionally taken into the baseline of EVM for the purpose of calibration and/or comparing intermediate results (e.g., accuracy of AI/ML output CSI, etc.)

- Note: Eventual performance comparison with the benchmark release and drawing SI conclusions should be based on realistic DL channel estimation.

- FFS: the ideal channel estimation is applied for dataset construction, or performance evaluation/inference.

- FFS: how to model the realistic channel estimation

- FFS: Whether ideal channel is used as target CSI for intermediate results calculation with AI/ML output CSI from realistic channel estimation

| Supporting companies | Apple, LG, CAICT, ZTE, MediaTek, Futurewei, NVIDIA, DCM, Samsung, OPPO, Panasonic, CATT, Fujitsu, Xiaomi, Huawei/Hisi, Qualcomm (comment below) , Spreadtrum, Lenovo, CMCC, Ericsson |
| --- | --- |
| Objecting companies |  |

### 6.3-4 Baseline SLS EVM table-Traffic model

For ‘Traffic model’ entry, two options are provided. The difference is: for Option 2, if the full buffer is selected, it can be also used for eventual performance evaluation rather than used for only comparing intermediate results in Option 1.

Proposal 6.3.4: For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, the ‘Traffic model’ in the baseline of EVM is captured as follows:

- Option 1 (combination of Proposal 2.5.2 with the original EVM):

| Traffic model | FTP model as a baseline FTP model 1 with packet size 0.5 Mbytes as a baseline Other FTP model is not precluded. FFS full buffer Full buffer can be optionally taken for the purpose of calibration and/or comparing intermediate results (e.g., accuracy of AI/ML output CSI, etc.) |
| --- | --- |

- Option 2 (combination of Proposal 2.5.2 with Intel’s proposal):

| Traffic model | FTP model as a baseline FTP model 1 with packet size 0.5 Mbytes as a baseline Other FTP model is not precluded. FFS full buffer Full buffer model is not precluded |
| --- | --- |

| Option 1 | Supporting companies | NVIDIA, InterDigital, LG, CAICT, OPPO, vivo, Ericsson, CMCC, DCM, CATT, ZTE, Xiaomi |
| --- | --- | --- |
|  | Objecting companies |  |
| Option 2 | Supporting companies | Futurewei, Qualcomm (comment below), Apple |
|  | Objecting companies |  |

### 6.3-5 AI/ML structure for CSI compression

Proposal 6.3.5: For the evaluation of the AI/ML based CSI compression sub use cases, a two-sided structure model is considered as a starting point prioritized, including an AI/ML-based CSI generation part to generate the CSI feedback information and an AI/ML-based CSI reconstruction part which is used to reconstruct the CSI from the received CSI feedback information.

- At least for inference, the CSI generation part is located at the UE side, and the CSI reconstruction part is located at the gNB side.

| Supporting companies | Futurewei, InterDigital, LG, CAICT, OPPO, MediaTek, vivo,Apple, Qualcomm, CMCC, Lenovo, DCM, Fujitsu, Panasonic, CATT, ZTE |
| --- | --- |
| Objecting companies |  |

Further updates based on Xiaomi’s comments

Proposal 6.3.6: For the evaluation of the AI/ML based CSI compression sub use cases, companies are encouraged to report the details of their model, including:

- The structure of the AI/ML model

- The input CSI type, e.g., raw channel matrix estimated by UE, eigenvector(s) of the raw channel matrix estimated by UE, etc.

  - FFS: if raw channel is selected, the raw channel matrix is with or without analog BF

- The output CSI type, e.g., reconstructed channel matrix, reconstructed eigenvector(s), etc.

- Data pre-processing/post-processing

- Loss function

- Others are not precluded

| Supporting companies | NVIDIA, Futurewei, InterDigital, LG, CAICT, OPPO, MediaTek, vivo, Apple, Qualcomm, Ericsson, CMCC, Lenovo, DCM, Fujitsu, Panasonic, CATT, ZTE |
| --- | --- |
| Objecting companies |  |

### 6.3-6 Metrics – Generalization

Further updates based on Samsung’s comments

Proposal 6.3.7: For the evaluation of the AI/ML based CSI feedback enhancement, study the verification of generalization. Ccompanies are encouraged to report how they verify the generalization of the AI/ML model, including:

- The training dataset of configuration(s)/ scenario(s), including potentially the mixed training dataset from multiple configurations/scenarios

- The testing/inference set of configuration(s)/ scenario(s)

- The detailed list of configuration(s) and/or scenario(s)

- Other details are not precluded

| Supporting companies | NVIDIA, Futurewei (with comments), InterDigital, LG, CAICT, OPPO, MediaTek, vivo, Apple, Qualcomm, Ericsson, CMCC, Lenovo, DCM, CATT, ZTE |
| --- | --- |
| Objecting companies |  |

### 6.3-7 Metrics – Intermediate KPIs

For whether/how to use intermediate evaluation with intermediate KPI (such as accuracy of output CSI) for evaluation, most companies agree the principle on using it for the purpose of AI/ML solution comparison.

Some company points out the FFS part is not clear on what different parameters should be considered, so it is removed.

Some company holds the view that the intermediate evaluation can also be used for eventual performance evaluation, so the note in the original version was crossed out.

Proposal 6.3.8: For the evaluation of the AI/ML based CSI feedback enhancement, companies can consider performing intermediate evaluation on AI/ML model performance to derive the intermediate KPI(s) for the purpose of AI/ML solution comparison

- FFS: Parameter assumptions for the intermediate evaluation of AI/ML model performance

- Note: intermediate KPI(s) includes accuracy of AI/ML output CSI

- Note: eventual performance comparison with the benchmark release and drawing SI conclusions should be based on eventual KPI(s).

For intermediate KPI, the following proposal is given, by selection GCS/SGCS and/or NMSE as part of the metric. Further updates based on Lenovo’s comments.

Proposal 6.3.9: For the evaluation of the AI/ML based CSI feedback enhancement, as a starting point, take the intermediate KPIs of GCS/SGCS and/or [optionally] NMSE as part of the ‘Evaluation Metric’ to evaluate the accuracy of the AI/ML output CSI

- For GCS/SGCS,

  - FFS: how to calculate GCS/SGCS for rank>1

  - FFS: whether GCS or SGCS is adopted

- FFS other metrics, e.g., equivalent MSE, or received SNR, or throughput/spectral efficiency gap.

| Supporting companies | Apple, vivo, LG, CAICT, ZTE, MediaTek, Futurewei, DCM, Samsung, OPPO, Panasonic, CATT, Fujitsu, Xiaomi Huawei/Hisi, Qualcomm, Spreadtrum, Lenovo (comment below) , CMCC, NVIDIA |
| --- | --- |
| Objecting companies |  |

Proposal 6.3.10: For the evaluation of the AI/ML based CSI feedback enhancement, if the GCS/SGCS is adopted as the intermediate KPI as part of the ‘Evaluation Metric’, SGCS should be chosen.

- Note: wi is the vector of target CSI of resource unit i, and wi is the vector of output CSI of resource unit i. N is the total number of resource units. E{∙} denotes the average operation over multiple samples.

- Note: Companies are encouraged to report the detail information of the vector of target CSI assumed in the evaluation.

SGCS=E1Ni=1N||wiHwi ||||wi||  ||wi||2

Backup Proposal 6.3.10: For the evaluation of the AI/ML based CSI feedback enhancement, if the GCS/SGCS is adopted as the intermediate KPI as part of the ‘Evaluation Metric’, GCS should be chosen.

- Note: wi is the vector of target CSI of resource unit i, and wi is the vector of output CSI of resource unit i. N is the total number of resource units. E{∙} denotes the average operation over multiple samples.

- Note: Companies are encouraged to report the detail information of the vector of target CSI assumed in the evaluation.

GCS=E1Ni=1N||wiHwi ||||wi||  ||wi||

### 6.3-8 Baseline LLS EVM table

Proposal 6.3.11: For the evaluation of the AI/ML based CSI feedback enhancement, if LLS is preferred, the following table is taken as a baseline of EVM

- Note: the baseline EVM is used to compare the performance with the benchmark release, while the AI/ML related parameters (e.g., dataset construction, generalization verification, and AI/ML related metrics) can be of additional/different assumptions.

  - The conclusions for the use cases in the SI should be drawn based on generalization verification over potentially multiple scenarios/configurations.

- FFS: modifications on top of the following table for the purpose of AI/ML related evaluations.

- FFS: other parameters and values if needed

Table 6.3-3 Baseline LLS EVM table

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | FDD (TDD is not precluded), OFDM |
| Carrier frequency | 2GHz as baseline, optional for 4GHz |
| Bandwidth | 10MHz or 20MHz |
| Subcarrier spacing | 15kHz for 2GHz, 30kHz for 4GHz |
| Nt | 32: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Nr | 4: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ |
| Channel model | CDL-C as baseline, CDL-A as optional |
| UE speed | 3kmhr, 10km/h, 20km/h or 30km/h to be reported by companies |
| Delay spread | 30ns or 300ns |
| Channel estimation | Realistic channel estimation algorithms (e.g. LS or MMSE) |
| Rank per UE | Rank 1-4 |

| Supporting companies | vivo, LG, CAICT, Samsung, OPPO, Panasonic, InterDigital, Fujitsu, Xiaomi, OPPO, Qualcomm, Ericsson, CMCC, DCM |
| --- | --- |
| Objecting companies |  |

### 6.3-9 Capability/complexity related KPIs

Proposal 6.3.12: For the evaluation of the AI/ML based CSI feedback enhancement, Floating point operations (FLOPs) is adopted as part of the ‘Evaluation Metric’, and reported by companies.

| Supporting companies | Xiaomi, vivo Huawei/Hisi, CAICT, MediaTek, Ericsson, CMCC, Nokia/NSB, NVIDIA, InterDigital, Fraunhofer, Charter, Futurewei, Lenovo (with a note), LG, Panasonic, CATT, ZTE Samsung Fujitsu Qualcomm Apple OPPO, Intel, Beijing Jiaotong University, AT&T |
| --- | --- |
| Objecting companies |  |

Proposal 6.3.13: For the evaluation of the AI/ML based CSI feedback enhancement, AI/ML memory storage in terms of AI/ML model size and number of AI/ML parameters is adopted as part of the ‘Evaluation Metric’, and reported by companies who may select either or both.

- FFS: the format of the AI/ML parameters

| Supporting companies | Apple, OPPO, Samsung, Beijing Jiaotong University, LG Huawei/Hisi, Spreadtrum, CMCC, vivo, CATT, CAICT, Futurewei, Fujitsu, Xiaomi, NVIDIA, ZTE, MediaTek, Nokia/NSB, Lenovo, InterDigital |
| --- | --- |
| Objecting companies |  |

Proposals for May 20 GTW

### 6.4-1 Stable proposals (>24hrs) [for email endorsement]

Proposal 6.3.1: For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, the following table is taken as a baseline of EVM

- Note: the following table captures the common parts of the R16 CSI enhancement EVM table and the R17 CSI enhancement EVM table, while the different parts are FFS.

- Note: the baseline EVM is used to compare the performance with the benchmark release, while the AI/ML related parameters (e.g., dataset construction, generalization verification, and AI/ML related metrics) can be of additional/different assumptions.

  - The conclusions for the use cases in the SI should be drawn based on generalization verification over potentially multiple scenarios/configurations.

- FFS: modifications on top of the following table for the purpose of AI/ML related evaluations.

Table 6.3-1 Baseline SLS EVM table

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD (TDD is not precluded), OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) is a baseline. Other scenarios (e.g. UMi@4GHz 2GHz, Urban Macro) are not precluded. |
| Frequency Range |  | FR1 only, FFS 2GHz or 4GHz as a baseline |
| Inter-BS distance |  | 200m |
| Channel model |  | According to TR 38.901 |
| Antenna setup and port layouts at gNB |  | Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ - 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ Other configurations are not precluded. |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) Other configuration is not precluded. |
| BS Tx power |  | 41 dBm for 10MHz, 44dBm for 20MHz, 47dBm for 40MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz for 2GHz, 30kHz for 4GHz |
| Simulation bandwidth |  | FFS |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | FFS |
| MIMO layers |  | For all evaluation, companies to provide the assumption on the maximum MU layers (e.g. 8 or 12) |
| CSI feedback |  | Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback) : 5 ms, - Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms |
| Overhead |  | Companies shall provide the downlink overhead assumption (i.e., whether the CSI-RS transmission is UE-specific or not and take that into account for overhead computation) |
| Traffic model |  | FFS |
| Traffic load (Resource utilization) |  | FFS |
| UE distribution |  | - 80% indoor (3km/h), 20% outdoor (30km/h) FFS whether/what other indoor/outdoor distribution and/or UE speeds for outdoor UEs needed |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic as a baseline FFS ideal channel estimation |
| Evaluation Metric |  | Throughput and CSI feedback overhead as baseline metrics. Additional metrics, e.g., ratio between throughput and CSI feedback overhead, can be used. Maximum overhead (payload size for CSI feedback)for each rank at one feedback instance is the baseline metric for CSI feedback overhead, and companies can provide other metrics. |
| Baseline for performance evaluation |  | FFS |

| Supporting companies | Apple, vivo (with comments), LG, Ericsson, CAICT, ZTE, Nokia/NSB, mediatek, Futurewei, NVIDIA, DCM, Samsung, OPPO, InterDigital, CATT, Fujitsu, Xiaomi, Huawei/Hisi, Qualcomm, Spreadtrum, Lenovo, CMCC, DCM, Panasonic |
| --- | --- |
| Objecting companies |  |

Upd Proposal 2.8.3: For the evaluation of the AI/ML based CSI feedback enhancement, if LLS is preferred, the following table is taken as a baseline of EVM

- Note: the baseline EVM is used to compare the performance with the benchmark release, while the AI/ML related parameters (e.g., dataset construction, generalization verification, and AI/ML related metrics) can be of additional/different assumptions.

  - The conclusions for the use cases in the SI should be drawn based on generalization verification over potentially multiple scenarios/configurations.

- FFS: modifications on top of the following table for the purpose of AI/ML related evaluations.

- FFS: other parameters and values if needed

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | FDD (TDD is not precluded), OFDM |
| Carrier frequency | 2GHz as baseline, optional for 4GHz |
| Bandwidth | 10MHz or 20MHz |
| Subcarrier spacing | 15kHz for 2GHz, 30kHz for 4GHz |
| Nt | 32: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Nr | 4: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ |
| Channel model | CDL-C as baseline, CDL-A as optional |
| UE speed | 3kmhr, 10km/h, 20km/h or 30km/h to be reported by companies |
| Delay spread | 30ns or 300ns |
| Channel estimation | Realistic channel estimation algorithms (e.g. LS or MMSE) as a baseline, FFS ideal channel estimation |
| Rank per UE | Rank 1-4. Companies are encouraged to report the Rank number, and whether/how rank adaptation is applied |

| Supporting companies | vivo, LG, CAICT, Samsung, OPPO, Panasonic, InterDigital, Fujitsu, Xiaomi, OPPO, Qualcomm, Ericsson, CMCC, DCM, Nokia/NSB |
| --- | --- |
| Objecting companies |  |

Upd Proposal 2.8.9: For the evaluation of the AI/ML based CSI feedback enhancement, study the verification of generalization. Companies are encouraged to report how they verify the generalization of the AI/ML model, including:

- The training dataset of configuration(s)/ scenario(s), including potentially the mixed training dataset from multiple configurations/scenarios

- The configuration(s)/ scenario(s) for testing/inference

- The detailed list of configuration(s) and/or scenario(s)

- Other details are not precluded

| Supporting companies | NVIDIA, Futurewei (with comments), InterDigital, LG, CAICT, OPPO, MediaTek, vivo, Apple, Qualcomm, Ericsson, CMCC, Lenovo, DCM, CATT, ZTE, Xiaomi(with comments),Intel, Nokia/NSB |
| --- | --- |
| Objecting companies |  |

Upd Proposal 3.8.2: For the evaluation of the AI/ML based CSI compression sub use cases, companies are encouraged to report the details of their models, including:

- The structure of the AI/ML model, e.g., type (CNN, RNN, Transformer, Inception, …), the number of layers, branches, real valued or complex valued parameters, etc.

- The input CSI type, e.g., raw channel matrix estimated by UE, eigenvector(s) of the raw channel matrix estimated by UE, etc.

  - FFS: the input CSI is obtained from the channel with or without analog BF

- The output CSI type, e.g., channel matrix, eigenvector(s), etc.

- Data pre-processing/post-processing

- Loss function

- Others are not precluded

| Supporting companies | NVIDIA, Futurewei, InterDigital, LG, CAICT, OPPO, MediaTek, vivo, Apple, Qualcomm, Ericsson, CMCC, Lenovo, DCM, Fujitsu, Panasonic, CATT, ZTE, Xiaomi(with comments), Nokia/NSB |
| --- | --- |
| Objecting companies |  |

### 6.4-2 Almost stable proposals (>12hrs) [for email endorsement]

Upd Proposal 2.8.1: For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, the following parameters are taken into the baseline of EVM

- Note: The 2nd column applies if R16 TypeII codebook is selected as baseline, and the 3rd column applies if R17 TypeII codebook is selected as baseline.

  - Additional assumptions from R17 TypeII EVM Same consideration with respect to utilizing angle-delay reciprocity should be considered taken for the AI/ML based CSI feedback and the baseline scheme if R17 TypeII codebook is selected as baseline

  - FFS baseline for potential sub use cases involving CSI enhancement on time domain

- Note: the baseline EVM is used to compare the performance with the benchmark release, while the AI/ML related parameters (e.g., dataset construction, generalization verification, and AI/ML related metrics) can be of additional/different assumptions.

  - The conclusions for the use cases in the SI should be drawn based on generalization verification over potentially multiple scenarios/configurations.

- FFS: modifications on top of the following table for the purpose of AI/ML related evaluations.

| Parameter | Value (if R16 as baseline) | Value (if R17 as baseline) |
| --- | --- | --- |
| Frequency Range | FR1 only, 2GHz as baseline, optional for 4GHz. | FR1 only, 2GHz with duplexing gap of 200MHz between DL and UL, optional for 4GHz |
| Simulation bandwidth | 10 MHz for 15kHz as a baseline, and configurations which emulate larger BW, e.g., same sub-band size as 40/100 MHz with 30kHz, may be optionally considered. Above 15kHz is replaced with 30kHz SCS for 4GHz. | 20 MHz for 15kHz as a baseline (optional for 10 MHz with 15KHz), and configurations which emulate larger BW, e.g., same sub-band size as 40/100 MHz with 30kHz, may be optionally considered. Above 15kHz is replaced with 30kHz SCS for 4GHz |
| MIMO scheme | SU/MU-MIMO with rank adaptation. Companies are encouraged to report the SU/MU-MIMO with RU | SU/MU-MIMO with rank adaptation. Companies are encouraged to report the SU/MU-MIMO with RU |
| Traffic load (Resource utilization) | 20/50/70% Companies are encouraged to report the MU-MIMO utilization. | 20/50/70% Companies are encouraged to report the MU-MIMO utilization. |

| Supporting companies | Vivo, Ericsson, CAICT, ZTE, MediaTek, Futurewei, NVIDIA, Samsung, OPPO, InterDigital, CATT, Fujitsu, Xiaomi Huawei/Hisi, Qualcomm, Spreadtrum, CMCC,Apple, Lenovo, DCM, Panasonic, Intel, LG, Nokia/NSB |
| --- | --- |
| Objecting companies |  |

Proposal 2.8.5: For the evaluation of the AI/ML based CSI feedback enhancement, as a starting point, take the intermediate KPIs of GCS/SGCS and/or NMSE as part of the ‘Evaluation Metric’ to evaluate the accuracy of the AI/ML output CSI

- For GCS/SGCS,

  - FFS: how to calculate GCS/SGCS for rank>1

  - FFS: whether GCS or SGCS is adopted

- FFS other metrics, e.g., equivalent MSE, received SNR, or numerical spectral efficiency gap.

### 6.4-3 Metrics – Intermediate KPIs: Cosine similarity

An updated FFS added to reflect the views of some companies to further down-select between GCS/SGCS.

Proposal 2.8.6: For the evaluation of the AI/ML based CSI feedback enhancement, if the GCS/SGCS is adopted as the intermediate KPI as part of the ‘Evaluation Metric’, companies to report whether SGCS or GCS is adopted.

- Note: wi is the target CSI of resource unit i, and wi is the output CSI of resource unit i. N is the total number of resource units. E{∙} denotes the average operation over multiple samples.

- Note: Companies are encouraged to report the detail information of the target CSI assumed in the evaluation.

- FFS: Further down-selection between GCS and SGCS.

GCS=E1Ni=1N||wiHwi ||||wi||  ||wi||,  SGCS=E1Ni=1N||wiHwi ||||wi||  ||wi||2

| Supporting companies | Intel, vivo, Qualcomm, CATT, CMCC, Fujitsu, Lenovo, Samsung, Nokia/NSB |
| --- | --- |
| Objecting companies |  |

An updated FFS added to reflect the views of some companies to further down-select or select one Method as baseline.

Proposal 2.9.2: For the evaluation of the AI/ML based CSI feedback enhancement, if the GCS/SGCS is adopted as the intermediate KPI as part of the ‘Evaluation Metric’, for rank>1 cases, companies to report the GCS/SGCS calculation methods, including:

- Method Option 1: Average over all ranks

  - Note: wij is the jtheigenvector at resource unit i and K is the rank of the channel

GCS=E1Ni=1NjK||wijHwij ||||wij||  ||wij||,  SGCS=E1Ni=1NjK||wijHwij ||||wij||  ||wij||2

- Method Option 2: Weighted average over all ranks

  - Companies to report the formula (e.g., whether normalization is applied for eigienvalues)

- Method Option 3: GCS/SGCS is separately calculated for each rank (e.g., for K ranks, K GCS values are derived respectively)

- Option 4: Other

- FFS: Further down-selection among the above options or take one of the above options as baseline.

### 6.4-4 Baseline SLS EVM table-Traffic model

The ‘Traffic model’ entry for the FFS part in the overall SLS EVM table is stil controversial. Two options are provided. The difference is: for Option 2, if the full buffer is selected, it can be also used for eventual performance evaluation rather than used for only comparing intermediate results in Option 1.

Proposal 2.7.1 For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, the ‘Traffic model’ in the baseline of EVM is captured as follows:

- Option 1 (combination of Proposal 2.5.2 with the original EVM):

| Traffic model | FTP model as a baseline FTP model 1 with packet size 0.5 Mbytes Other FTP model is not precluded. Full buffer can be optionally taken for the purpose of calibration and/or comparing intermediate results (e.g., accuracy of AI/ML output CSI, etc.) |
| --- | --- |

- Option 2 (combination of Proposal 2.5.2 with Intel’s proposal):

| Traffic model | FTP model as a baseline FTP model 1 with packet size 0.5 Mbytes Other FTP model is not precluded. Full buffer model is not precluded |
| --- | --- |

| Option 1 | Supporting companies | NVIDIA, InterDigital, LG, CAICT, OPPO, vivo, Ericsson, CMCC, DCM, CATT, ZTE, Xiaomi, Nokia/NSB, MediaTek |
| --- | --- | --- |
|  | Objecting companies |  |
| Option 2 | Supporting companies | Futurewei, Qualcomm, Apple, Intel, Samsung |
|  | Objecting companies |  |

Agreements for 109-e

Agreement

For the performance evaluation of the AI/ML based CSI feedback enhancement, system level simulation approach is adopted as baseline

- Link level simulation is optionally adopted

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, for the calibration purpose on the dataset and/or AI/ML model over companies, consider to align the parameters (e.g., for scenarios/channels) for generating the dataset in the simulation as a starting point.

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, for ‘Channel estimation’, ideal DL channel estimation is optionally taken into the baseline of EVM for the purpose of calibration and/or comparing intermediate results (e.g., accuracy of AI/ML output CSI, etc.)

- Note: Eventual performance comparison with the benchmark release and drawing SI conclusions should be based on realistic DL channel estimation.

- FFS: the ideal channel estimation is applied for dataset construction, or performance evaluation/inference.

- FFS: How to model the realistic channel estimation

- FFS: Whether ideal channel is used as target CSI for intermediate results calculation with AI/ML output CSI from realistic channel estimation

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, companies can consider performing intermediate evaluation on AI/ML model performance to derive the intermediate KPI(s) (e.g., accuracy of AI/ML output CSI) for the purpose of AI/ML solution comparison.

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, Floating point operations (FLOPs) is adopted as part of the ‘Evaluation Metric’, and reported by companies.

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, AI/ML memory storage in terms of AI/ML model size and number of AI/ML parameters is adopted as part of the ‘Evaluation Metric’, and reported by companies who may select either or both.

- FFS: the format of the AI/ML parameters

Agreement

For the evaluation of the AI/ML based CSI compression sub use cases, a two-sided model is considered as a starting point, including an AI/ML-based CSI generation part to generate the CSI feedback information and an AI/ML-based CSI reconstruction part which is used to reconstruct the CSI from the received CSI feedback information.

- At least for inference, the CSI generation part is located at the UE side, and the CSI reconstruction part is located at the gNB side.

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, the following table is taken as a baseline of EVM

- Note: the following table captures the common parts of the R16 CSI enhancement EVM table and the R17 CSI enhancement EVM table, while the different parts are FFS.

- Note: the baseline EVM is used to compare the performance with the benchmark release, while the AI/ML related parameters (e.g., dataset construction, generalization verification, and AI/ML related metrics) can be of additional/different assumptions.

  - The conclusions for the use cases in the SI should be drawn based on generalization verification over potentially multiple scenarios/configurations.

- FFS: modifications on top of the following table for the purpose of AI/ML related evaluations.

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD (TDD is not precluded), OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) is a baseline. Other scenarios (e.g. UMi@4GHz 2GHz, Urban Macro) are not precluded. |
| Frequency Range |  | FR1 only, FFS 2GHz or 4GHz as a baseline |
| Inter-BS distance |  | 200m |
| Channel model |  | According to TR 38.901 |
| Antenna setup and port layouts at gNB |  | Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ - 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ Other configurations are not precluded. |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) Other configuration is not precluded. |
| BS Tx power |  | 41 dBm for 10MHz, 44dBm for 20MHz, 47dBm for 40MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz for 2GHz, 30kHz for 4GHz |
| Simulation bandwidth |  | FFS |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | FFS |
| MIMO layers |  | For all evaluation, companies to provide the assumption on the maximum MU layers (e.g. 8 or 12) |
| CSI feedback |  | Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback) : 5 ms, - Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms |
| Overhead |  | Companies shall provide the downlink overhead assumption (i.e., whether the CSI-RS transmission is UE-specific or not and take that into account for overhead computation) |
| Traffic model |  | FFS |
| Traffic load (Resource utilization) |  | FFS |
| UE distribution |  | - 80% indoor (3km/h), 20% outdoor (30km/h) FFS whether/what other indoor/outdoor distribution and/or UE speeds for outdoor UEs needed |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic as a baseline FFS ideal channel estimation |
| Evaluation Metric |  | Throughput and CSI feedback overhead as baseline metrics. Additional metrics, e.g., ratio between throughput and CSI feedback overhead, can be used. Maximum overhead (payload size for CSI feedback)for each rank at one feedback instance is the baseline metric for CSI feedback overhead, and companies can provide other metrics. |
| Baseline for performance evaluation |  | FFS |

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, as a starting point, take the intermediate KPIs of GCS/SGCS and/or NMSE as part of the ‘Evaluation Metric’ to evaluate the accuracy of the AI/ML output CSI

- For GCS/SGCS,

  - FFS: how to calculate GCS/SGCS for rank>1

  - FFS: whether GCS or SGCS is adopted

- FFS other metrics, e.g., equivalent MSE, received SNR, or numerical spectral efficiency gap.

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, if LLS is preferred, the following table is taken as a baseline of EVM

- Note: the baseline EVM is used to compare the performance with the benchmark release, while the AI/ML related parameters (e.g., dataset construction, generalization verification, and AI/ML related metrics) can be of additional/different assumptions.

  - The conclusions for the use cases in the SI should be drawn based on generalization verification over potentially multiple scenarios/configurations.

- FFS: modifications on top of the following table for the purpose of AI/ML related evaluations.

- FFS: other parameters and values if needed

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | FDD (TDD is not precluded), OFDM |
| Carrier frequency | 2GHz as baseline, optional for 4GHz |
| Bandwidth | 10MHz or 20MHz |
| Subcarrier spacing | 15kHz for 2GHz, 30kHz for 4GHz |
| Nt | 32: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Nr | 4: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ |
| Channel model | CDL-C as baseline, CDL-A as optional |
| UE speed | 3kmhr, 10km/h, 20km/h or 30km/h to be reported by companies |
| Delay spread | 30ns or 300ns |
| Channel estimation | Realistic channel estimation algorithms (e.g. LS or MMSE) as a baseline, FFS ideal channel estimation |
| Rank per UE | Rank 1-4. Companies are encouraged to report the Rank number, and whether/how rank adaptation is applied |

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, study the verification of generalization. Companies are encouraged to report how they verify the generalization of the AI/ML model, including:

- The configuration(s)/ scenario(s) for training dataset, including potentially the mixed training dataset from multiple configurations/scenarios

- The configuration(s)/ scenario(s) for testing/inference

- Other details are not precluded

Agreement

For the evaluation of the AI/ML based CSI compression sub use cases, companies are encouraged to report the details of their models, including:

- The structure of the AI/ML model, e.g., type (CNN, RNN, Transformer, Inception, …), the number of layers, branches, real valued or complex valued parameters, etc.

- The input CSI type, e.g., raw channel matrix estimated by UE, eigenvector(s) of the raw channel matrix estimated by UE, etc.

  - FFS: the input CSI is obtained from the channel with or without analog BF

- The output CSI type, e.g., channel matrix, eigenvector(s), etc.

- Data pre-processing/post-processing

- Loss function

- Others are not precluded

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, the following parameters are taken into the baseline of EVM

- Note: The 2nd column applies if R16 TypeII codebook is selected as baseline, and the 3rd column applies if R17 TypeII codebook is selected as baseline.

  - Additional assumptions from R17 TypeII EVM with respect to utilizing angle-delay reciprocity should be considered for the AI/ML based CSI feedback and the baseline scheme if R17 TypeII codebook is selected as baseline

  - FFS baseline for potential sub use cases involving CSI enhancement on time domain

- Note: the baseline EVM is used to compare the performance with the benchmark release, while the AI/ML related parameters (e.g., dataset construction, generalization verification, and AI/ML related metrics) can be of additional/different assumptions.

  - The conclusions for the use cases in the SI should be drawn based on generalization verification over potentially multiple scenarios/configurations.

- FFS: modifications on top of the following table for the purpose of AI/ML related evaluations.

| Parameter | Value (if R16 as baseline) | Value (if R17 as baseline) |
| --- | --- | --- |
| Frequency Range | FR1 only, 2GHz as baseline, optional for 4GHz. | FR1 only, 2GHz with duplexing gap of 200MHz between DL and UL, optional for 4GHz |
| Simulation bandwidth | 10 MHz for 15kHz as a baseline, and configurations which emulate larger BW, e.g., same sub-band size as 40/100 MHz with 30kHz, may be optionally considered. Above 15kHz is replaced with 30kHz SCS for 4GHz. | 20 MHz for 15kHz as a baseline (optional for 10 MHz with 15KHz), and configurations which emulate larger BW, e.g., same sub-band size as 40/100 MHz with 30kHz, may be optionally considered. Above 15kHz is replaced with 30kHz SCS for 4GHz |
| MIMO scheme | SU/MU-MIMO with rank adaptation. Companies are encouraged to report the SU/MU-MIMO with RU | SU/MU-MIMO with rank adaptation. Companies are encouraged to report the SU/MU-MIMO with RU |
| Traffic load (Resource utilization) | 20/50/70% Companies are encouraged to report the MU-MIMO utilization. | 20/50/70% Companies are encouraged to report the MU-MIMO utilization. |

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, the ‘Baseline for performance evaluation’ in the baseline of EVM is captured as follows

| Baseline for performance evaluation | Companies need to report which option is used between - Rel-16 TypeII Codebook as the baseline for performance and overhead evaluation. - Rel-17 TypeII Codebook as the baseline for performance and overhead evaluation. - FFS: Whether Type I Codebook can be optionally considered at least for performance evaluation |
| --- | --- |

For the evaluation of the AI/ML based CSI feedback enhancement, if the GCS/SGCS is adopted as the intermediate KPI as part of the ‘Evaluation Metric’ for rank>1 cases, companies to report the GCS/SGCS calculation/extension methods, including:

- Method 1: Average over all layers

  - Note: wij is the jtheigenvector of the target CSI at resource unit i and K is the rank. wij is the jth output vector of the output CSI of resource unit i. N is the total number of resource units. E{∙} denotes the average operation over multiple samples.

GCS=E1Ni=1NjK||wijHwij ||||wij||  ||wij||,  SGCS=E1Ni=1NjK||wijHwij ||||wij||  ||wij||2

- Method 2: Weighted average over all layers

  - Note: Companies to report the formula (e.g., whether normalization is applied for eigenvalues)

- Method 3: GCS/SGCS is separately calculated for each layer (e.g., for K layers, K GCS/SGCS values are derived respectively, and comparison is performed per layer)

- Other methods are not precluded

- FFS: Further down-selection among the above options or take one/a subset of the above methods as baseline(s).

FL Summary

Following progress has been achieved in the 109-e meeting:

- Simulation approach is determined: SLS as a baseline, while LLS as optional

- Baseline SLS EVM table is achieved, with most parameters determined.

- Baseline LLS EVM table is achieved, aligned with SLS EVM table with some different parameters of, e.g., channel models/UE speeds, etc.

- Channel estimation: realistic channel estimation as baseline, ideal channel estimation as optional.

- Calibration: Aligning parameters over companies as a starting point.

- AI/ML related KPIs

  - Eventual KPI: throughput, overhead as in the EVM table.

  - Intermediate KPI (e.g., accuracy of AI/ML output CSI) can be used for the purpose of AI/ML solution comparison.

  - Intermediate KPI: GCS/SGCS and/or NMSE as a starting point. Candidate options listed for GCS/SGCS with rank>1.

  - Capability/complexity related KPI: FLOPs, and memory storage (in terms of AI/ML model size and number of AI/ML parameters) are adopted as Metrics

- Generalization: Generalization performance to be studied. A general principle is provided that companies to report details, including scenarios/configurations for training and testing/inference.

- CSI compression: A two-sided model is considered as a starting point, including an AI/ML-based CSI generation part to generate the CSI feedback information and an AI/ML-based CSI reconstruction part which is used to reconstruct the CSI from the received CSI feedback information

  - A general principle that companies are encouraged to report the details of the AI/ML models, including AI/ML structure, input CSI type, output CSI type, data pre-processing/post-processing, loss function, etc.

Following aspects may be at least discussed in the next/future meetings:

- Baseline SLS EVM table

  - Some FFS parameter is still to be determined, including ‘traffic model’ (ftp model and/or full buffer).

  - Additional/different parameters/values are to be further determined based on AI/ML related evaluations.

- Baseline LLS EVM table: FFS additional parameters or values if needed.

- Channel estimation

  - FFS The ideal channel estimation is applied for dataset construction, or performance evaluation/inference

  - FFS: Whether ideal channel is used as target CSI for intermediate KPI calculation

- Generalization:

  - Whether/how to align the configuration(s)/scenario(s) for training dataset, including how to mix the dataset from multiple configurations/scenarios

  - How to consider the configuration(s)/scenario(s) for the testing/inference.

  - Other principles/guidance of how to perform generalization.

- Intermediate KPI:

  - Other intermediated KPIs, e.g., equivalent MSE, received SNR, or numerical spectral efficiency gap, and the analysis of pros/cons.

  - For GCS/SGCS, which single one to be adopted, or both considered based on companies’ report.

  - For GCS/SGCS, how to consider rank>1 cases.

  - Companies to analyze the pros/cons of the intermediate KPIs, so that a potential down-selection may be achieved in the next meeting.

- Capability/complexity related KPI: For the number of AI/ML parameters, FFS the format of the AI/ML parameters.

- CSI compression: Whether any other parameters need to be captured into the EVM. Companies to report the details of the adopted AI/ML model.

- CSI prediction: Proponents may provide more evaluation results and methodology inputs to convince the benefits and RAN1 efforts, for a further EVM discussion/determination.

- References

- RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, December 6-17, 2021.

- R1-2203068 Discussion on evaluation of AI/ML for CSI feedback enhancement use case  FUTUREWEI

- R1-2203140	Evaluation on AI/ML for CSI feedback enhancement  Huawei, HiSilicon

- R1-2203248	Evaluation assumptions on AI/ML for CSI feedback	ZTE

- R1-2203281	Evaluations on AI-CSI	Ericsson

- R1-2203451	Discussion on evaluation on AI/ML for CSI feedback  CATT

- R1-2203550	Evaluation on AI/ML for CSI feedback enhancement  vivo

- R1-2203808	Discussion on evaluation on AI/ML for CSI feedback enhancement  xiaomi

- R1-2203897	Evaluation on AI ML for CSI feedback enhancement	Samsung

- R1-2204015	Evaluation methodology and preliminary results on AI/ML for CSI feedback enhancement  OPPO

- R1-2204050	Evaluation on AI/ML for CSI feedback enhancement  InterDigital, Inc.

- R1-2204055	Evaluation of CSI compression with AI/ML  Beijing Jiaotong University

- R1-2204063	Performance evaluation of ML techniques for CSI feedback enhancement  Charter Communications, Inc

- R1-2204149  Evaluation on AI/ML for CSI feedback enhancement	LG Electronics

- R1-2204180  Some discussions on evaluation on AI-ML for CSI feedback	 CAICT

- R1-2204238  Initial evaluation on AI/ML for CSI feedback  Apple

- R1-2204295  Discussion on evaluation on AI/ML for CSI feedback enhancement  CMCC

- R1-2204375  Discussion on evaluation on AI/ML for CSI feedback enhancement  NTT DOCOMO, INC.

- R1-2204417  Evaluation on AI/ML for CSI feedback  Lenovo

- R1-2204499  Discussion on evaluation on AI/ML for CSI feedback enhancement  Spreadtrum Communications, BUPT

- R1-2204571  Evaluation on ML for CSI feedback enhancement  Nokia, Nokia Shanghai Bell

- R1-2204793  Evaluation for CSI feedback enhancements  Intel Corporation

- R1-2204840  On evaluation assumptions of AI and ML for CSI feedback enhancement  NVIDIA

- R1-2204860  Evaluation of AI/ML for CSI feedback enhancements  AT&T

- R1-2205024  Evaluation on AIML for CSI feedback enhancement  Qualcomm Incorporated

- R1-2205076  Evaluation on AI/ML for CSI feedback enhancement  Fujitsu Limited

- R1-2006973, “Discussion Summary for CSI enhancements MTRP and FR1 FDD reciprocity”, RAN1#102-e, August 17-28, 2020.

- R1-1812101, “Final Report of 3GPP TSG RAN WG1 #94bis v1.0.0 (Chengdu, China, 8th-12th October 2018),” MCC Support, 3GPP RAN1#95, 12-16 November 2018.

- R1-2007151, Samsung, “Moderator summary for multi-beam enhancement: EVM”, RAN1 #102-e, August 17-28, 2020.

- Appendix I: R16 EVM for CSI enhancement

R1-1811929	Feature Lead summary on Evaluation Methodologies for NR-eMIMO	Nokia, Nokia Shanghai Bell

Agreement

Existing EVM (Table A.2.1-1 of TS38.802 or NR phase 2 EVM) can be a starting point with additional updates.

Proposal 2-0: Use the table 2-1 with the following additional updates

Table 2-1. SLS assumptions for CSI enhancement

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD (TDD is not precluded), OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) is a baseline. Other scenarios (e.g. UMi@4GHz 2GHz, Urban Macro) are not precluded. |
| Frequency Range |  | FR1 only, 4GHz. |
| Inter-BS distance |  | 200m |
| Channel model |  | According to the TR 38.901 |
| Antenna setup and port layouts at gNB |  | Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ - 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ Other configurations are not precluded. |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for rank > 2 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) Type II overhead reduction Other configuration is not precluded. |
| BS Tx power |  | 41 dBm |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz |
| Number of RBs |  | 52 for 15 kHz SCS |
| Simulation bandwidth |  | 10 MHz for 15kHz as a baseline, and configurations which emulate larger BW, e.g., same sub-band size as 40/100 MHz with 30kHz, may be optionally considered. |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | SU/MU-MIMO with rank adaptation is a baseline for overhead reduction. For low RU, SU-MIMO or SU/MU-MIMO with rank adaptation are assumed for higher rank extension. For medium/high RU, SU/MU-MIMO with rank adaptation is assumed for higher rank extension. |
| MIMO layers |  | For all evaluation, companies to provide the assumption on the maximum MU layers (e.g. 8 or 12) |
| CSI feedback |  | Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback) : 5 ms, - Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms |
| Overhead |  | Companies shall provide the downlink overhead assumption |
| Traffic model |  | FTP model 1 with packet size 0.5 Mbytes Other FTP model is not precluded. |
| Traffic load (Resource utilization) |  | - 50/70 % for CSI overhead reduction - 20/50 % for high rank extension Companies are encouraged to report the MU-MIMO utilization. |
| UE distribution |  | - 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic |
| Evaluation Metric |  | Throughput and CSI feedback overhead as baseline metrics. Additional metrics, e.g., ratio between throughput and CSI feedback overhead, can be used. Maximum overhead (payload size for CSI feedback)for each rank at one feedback instance is the baseline metric for CSI feedback overhead, and companies can provide other metrics. |
| Baseline for performance evaluation |  | Rel-15 Type II Codebook is the baseline for performance and overhead evaluation for overhead reduction. (Type I Codebook can be considered at least for performance evaluation) - Companies are encouraged to compare the proposed overhead reduction scheme with Rel-15 overhead reduction scheme, Rel-15 Type I Codebook is the baseline for performance and overhead evaluation for higher rank codebook. |

- Appendix II: R17 EVM for CSI enhancement

Proposal:  For EVM for FDD CSI enhancement in Rel-17, following SLS parameter are used:

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD (TDD is not precluded), OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) is a baseline. Other scenarios (e.g. UMi@4GHz 2GHz, Urban Macro) are not precluded. |
| Frequency Range |  | FR1 only, 2GHz with duplexing gap of 200MHz between DL and UL, optional for 4GHz |
| Inter-BS distance |  | 200m |
| Antenna setup and port layouts at gNB |  | Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ - 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ Other configurations are not precluded. |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for rank > 2 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) Other configuration is not precluded. |
| BS Tx power |  | 41 dBm for 10MHz, 44dBm for 20MHz, 47dBm for 40MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz |
| Simulation bandwidth |  | 20 MHz for 15kHz as a baseline (optional for 10 MHz with 15KHz), and configurations which emulate larger BW, e.g., same sub-band size as 40/100 MHz with 30kHz, may be optionally considered |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | For low RU, SU-MIMO with rank adaptation are assumed For medium/high RU, SU/MU-MIMO with rank adaptation is assumed |
| MIMO layers |  | For all evaluation, companies to provide the assumption on the maximum MU layers (e.g. 8 or 12) |
| CSI feedback |  | Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback) : 5 ms, - Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms |
| Overhead |  | Companies shall provide the downlink overhead assumption |
| Traffic model |  | FTP model 1 with packet size 0.5 Mbytes Other FTP model is not precluded. |
| Traffic load (Resource utilization) |  | 70% for SU/MU-MIMO with rank adaptation 20% for SU-MIMO with rank adaptation Companies are encouraged to report the MU-MIMO utilization. |
| UE distribution |  | - 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic |
| Evaluation Metric |  | Throughput and CSI feedback overhead as baseline metrics. Additional metrics, e.g., ratio between throughput and CSI feedback overhead, can be used. Maximum overhead (payload size for CSI feedback)for each rank at one feedback instance is the baseline metric for CSI feedback overhead, and companies can provide other metrics. |
| Baseline for performance evaluation |  | Rel-16 PS eTypeII Codebook is the baseline for performance and overhead evaluation. (Type I Codebook can be considered at least for performance evaluation) - Note that it is encouraged to disclose further details of beamforming mechanism/ordering over CSI-RS ports/resources. |