3GPP TSG RAN WG1 #109-e                                                                                                  R1-2205395

- e-Meeting, May 9th – 20th, 2022

Source:	Moderator (ZTE)

Title:	Summary#3 of Email discussion on side control information

Agenda Item:	9.8.1

Document for:     Discussion and Decision

- Proposals for discussion at GTW sessions

[Updated proposal 3-2-1_v4]: The following options can be considered to indicate the ON-OFF information from gNB to NCR for controlling the behaviour of NCR-Fwd:

- Option 1: Explicit indication with single/multiple on-off state per indication/configuration (e.g., via dynamic or semi-static signalling) or on-off pattern (e.g., periodic/semi-static ON-OFF pattern or new DRX-like pattern for ON-OFF)

- Option 2: Implicit indication via the signalling for other information (e.g., beam, DL/UL configuration, or PC information)

  - Note: This example does not imply that PC information is necessary or not.

- Other solutions (e.g., potential combination of explicit and implication solution) can be further discussed.

[Proposal 1-4-1_v2]: Recommend to capture the following assumption of network-controlled repeater in TR 38.867.

- At least one of the NCR-MT’s carrier(s) should be within the set of carriers forwarded by the NCR-Fwd in same frequency range.

  - The NCR-MT and NCR-Fwd operating in the same carrier is prioritized for the study.

[Proposal 1-2-1_v2]: Recommend to capture the following assumption of network-controlled repeater in TR 38.867.

- As the baseline, same large-scale properties of the channel, i.e., channel properties in Type-A and Type-D (if applicable), are expected to be experienced by C-link and backhaul link (at least when the NCR-MT and NCR-Fwd operating in same carrier).

[Proposal 2-3-1_v4]: As the baseline, the same TCI states as C-link are assumed for beam at NCR for backhaul link if the NCR-MT’s carrier(s) is within the set of carries forwarded by the NCR-Fwd.

- FFS: additional indication from gNB to determine the beam at NCR side for backhaul link or implicit determination of the beam at NCR side for backhaul link if adaptive beamforming is supported by NCR-Fwd for backhaul.

Note: the same assumption of the beam correspondence is applied for DL/UL of the backhaul link at NCR-Fwd and the DL/UL of the C-link at NCR-MT.

[Updated Proposal 2-5-1_v3]: The beam correspondence is assumed for:

- the DL/UL of the access link at NCR-Fwd

[Updated Proposal 6-1-1_v5]: Recommend to capture the following observation in TR 38.867:

- Power control information is [may be] beneficial for network-controlled repeater to control the behavior of NCR-Fwd for the DL of access link and/or UL of backhaul link.

[Proposal 6-2-1_v1]: As the baseline, the controlling of the amplifying gain of NCR-Fwd is considered to enable the power control of NCR-Fwd if PC it’s recommended for NCR in Rel-18

- FFS: Controlling of the transmission power of NCR-Fwd

- Conclusion

Appendix

R1-2203133	Discussion on side control information to enable NR network-controlled repeaters	Huawei, HiSilicon

R1-2203237	Discussion on side control information to enable NR network-controlled repeaters	ZTE

R1-2203343	Discussion on side control information to enable NR network-controlled repeaters	Spreadtrum Communications

R1-2203354	Discussion on side control information to enable NR network-controlled repeaters	Nokia, Nokia Shanghai Bell

R1-2203376	Discussions on side control information to enable NR network-controlled repeaters	InterDigital, Inc.

R1-2203476	Discussion on side control information for NR network-controlled repeaters	CATT

R1-2203578	Discussion on side control information to enable NR network-controlled repeaters	vivo

R1-2203697	Discussion on side control information for network-controlled repeaters	NEC

R1-2203741	Considerations on side control information to enable NR network-controlled repeaters	Sony

R1-2203832	Discussion on side control information to enable NR network-controlled repeaters	xiaomi

R1-2203921	Side control information to enable NR network-controlled repeaters	Samsung

R1-2204064	Discussion on side control information for network controllable repeater	Lenovo

R1-2204066	Side Control Information for Network-Controlled Repeaters	Charter Communications, Inc

R1-2204086	Side Control Information for supporting initial access	CableLabs

R1-2204119	Side control information for network-controlled repeaters	SHARP Corporation

R1-2204258	Potential side control information for NW-controlled repeater	Apple

R1-2204321	Discussion on Side control information to enable NR network-controlled repeaters	CMCC

R1-2204393	Discussion on side control information to enable NR network-controlled repeaters	NTT DOCOMO, INC.

R1-2204532	Discussion on side control information for NCR	LG Electronics

R1-2204565	Discussion on side control signals for network-controlled repeaters	Panasonic

R1-2204610	Discussion on side control information to enable NR network-controlled repeaters	CAICT

R1-2204642	Control information for enabling NW-controlled repeaters	Ericsson

R1-2204653	Discussions on side control information for network-controlled repeater	ETRI

R1-2204688	Side control information for network-controlled repeaters	MediaTek Inc.

R1-2204757	Discussion on Side control information to enable NR network-controlled repeaters	CEWiT, IITK

R1-2204813	Disucssion on Side control information to enable NR network-controlled repeater	Intel Corporation

R1-2204847	Discussion on side control information to enable NR network-controlled repeaters	KDDI Corporation

R1-2204864	Side control information to enable NR network-controlled repeaters	AT&T

R1-2205047	On side control information for network controlled repeaters (NCR)	Qualcomm Incorporated

R1-2205068	Initial view on Network-controlled repeater	Rakuten Moible

R1-2205085   Side control information for NR network-controlled repeaters Fujitsu