| 3GPP TR 38.867 V0.1.0 (2022-05) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access network; Study on NR network-controlled repeaters; (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	8

4	Introduction	8

5	Modelling of Network-controlled repeater	8

6	Side control information	8

6.1	Beam information	8

6.2	Timing information	8

6.3	Information on UL-DL TDD configuration	9

6.4	ON-OFF information	9

6.5	Power control information	9

7	L1/L2 signalling for side control information	9

7.1	Signalling for side control information	9

7.2	Configuration of signalling	9

8	Repeater management	9

8.1	Solution on Repeater management	9

8.2	Specification impacts	9

9	Conclusion	9

Annex <A>: <Annex title for a Technical Report>	10

A.1	Heading levels in an annex	10

Annex <B>: Change history	11

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

[Editor’s Note: This clause is based on the text from the Objective parts in SID.]

The present document captures the findings from the study item “Study on NR Network-controlled Repeaters” [2].

The SI includes the study and identification of side control information (i.e., beamforming information, Timing information, information on UL-DL TDD configuration, ON-OFF information and power control information) for network-controlled repeaters and corresponding L1/L2 signaling (including its configuration) to carry the side control information. The scope of the study also includes the study on the aspects (i.e.,identification and authorization) of network-controlled repeater management

The study on NR network-controlled repeaters is to focus on the following scenarios and assumptions:

- Network-controlled repeaters are inband RF repeaters used for extension of network coverage on FR1 and FR2 bands, while during the study FR2 deployments may be prioritized for both outdoor and O2I scenarios.

- For only single hop stationary network-controlled repeaters

- Network-controlled repeaters are transparent to UEs

- Network-controlled repeater can maintain the gNB-repeater link and repeater-UE link simultaneously

NOTE1: Cost efficiency is a key consideration point for network-controlled repeaters.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP RP-213700: " New SI: Study on NR Network-controlled Repeaters ".

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

NCR	Network-controlled repeater

NCR-MT	NCR-Mobile termination

NCR-Fwd 	NCR-Forwarding

C-link	Control link

4	Introduction

[Editor’s Note: This clause reuses the text from the Justification parts in SID.]

Coverage is a fundamental aspect of cellular network deployments. Mobile operators rely on different types of network nodes to offer blanket coverage in their deployments. Deployment of regular full-stack cells is one option but it may not be always possible (e.g., no availability of backhaul) or economically viable.

As a result, new types of network nodes have been considered to increase mobile operators’ flexibility for their network deployments. For example, Integrated Access and Backhaul (IAB) was introduced in Rel-16 and enhanced in Rel-17 as a new type of network node not requiring a wired backhaul. Another type of network node is the RF repeater which simply amplify-and-forward any signal that they receive. RF repeaters have seen a wide range of deployments in 2G, 3G and 4G to supplement the coverage provided by regular full-stack cells. In Rel-17, RAN4 specified RF and EMC requirements for such RF repeaters for NR targeting both FR1 and FR2.

While an RF repeater presents a cost effective means of extending network coverage, it has its limitations. An RF repeater simply does an amplify-and-forward operation without being able to take into account various factors that could improve performance. Such factors may include information on semi-static and/or dynamic downlink/uplink configuration, adaptive transmitter/receiver spatial beamforming, ON-OFF status, etc.

A network-controlled repeater is an enhancement over conventional RF repeaters with the capability to receive and process side control information from the network. Side control information could allow a network-controlled repeater to perform its amplify-and-forward operation in a more efficient manner. Potential benefits could include mitigation of unnecessary noise amplification, transmissions and receptions with better spatial directivity, and simplified network integration.

5	Modelling of Network-controlled repeater

[Editor’s Note: This clause intent to capture the conceptual model of network-controlled repeater.]

The Network-controlled repeater is modelled as Figure 5-1, which includes the NCR-MT and NCR-Fwd. The NCR-MT is defined as a function entity to communicate with a gNB via Control link (C-link) to enable the information exchanges (e.g. side control information at least for the control of NCR-Fwd). The C-link is based on NR Uu interface.

The NCR-Fwd is defined as a function entity to perform the amplify-and-forwarding of UL/DL RF signal between gNB and UE via backhaul link and access link. The behavior of the NCR-Fwd will be controlled according to the received side control information from gNB.

Figure 5-1: Conceptual model of Network-controlled repeater

Additionally, at least one of the NCR-MT’s carrier(s) should be within the set of carriers forwarded by the NCR-Fwd in same frequency range. And the NCR-MT and NCR-Fwd operating in the same carrier is prioritized for the study.

As baseline, same large-scale properties of the channel, i.e., channel properties in Type-A and Type-D (if applicable), are expected to be experienced by C-link and backhaul link (at least when the NCR-MT and NCR-Fwd operating in same carrier).

6	Side control information

[Editor’s Note: This clause includes the progress for each side control information, which will be captured in sub-clause. Potential analysis for cost for each information can be captured based on the agreement.]

6.1	Beam information

For the backhaul link and C-link, both fixed beam and adaptive beam can be considered at NCR for both C-link and backhaul-link (FFS: the mechanism for indication and determination of beam), where the fixed beam refers to the case that beam at NCR for both C-link and backhaul-link cannot be changed.

As baseline, the same TCI states as C-link are assumed for beam at NCR-Fwd for backhaul link if the NCR-MT’s carrier(s) is within the set of carriers forwarded by the NCR-Fwd (FFS: additional indication from gNB to determine the beam at NCR-Fwd for backhaul link or implicit determination of the beam at NCR-Fwd for backhaul link). The same assumption of the beam correspondence is applied for DL/UL of the backhaul link at NCR-Fwd as the DL/UL of the C-link at NCR-MT.

For the access link, at least for FR2, beam information is beneficial and recommended as the side control information for network-controlled repeater to control the behaviour of NCR at least for access link (FFS: Detailed mechanism of indication. Note: There are no supporting evaluation results on FR1 at this point to reach similar conclusion). The beam correspondence is assumed for the DL/UL of the access link at NCR-Fwd.

In the access link beam indication, an access link beam can be indicated by (Note: The above does not imply that the NCR can or cannot generate and transmit reference signals to a UE or receive and process reference signals from a UE. RAN1 to select one of the two options, combine the two options, or select both options in RAN1#110):

- Option 1: A beam index (FFS: How to indicate the corresponding time domain resource of the beam. )

- Option 2: An index of a source RS (e.g. a TCI-like indicator) (FFS: The definition of the source RS. FFS: How to indicate the corresponding time domain resource of the beam. FFS: The definition of the association between the source RS and the beam.)

Both the dynamic indication and semi-static indication can be considered for the beam of access link for NCR-Fwd. (FFS: the details of each indication; FFS: the maximum number of beams configured for NCR-Fwd access link)

6.2	Timing information

For the timing of NCR, the following assumption is considered as baseline (FFS: the impact of internal delay on the following timing relationships: The DL receiving timing and DL transmitting timing of the NCR-Fwd/The UL transmitting timing and UL receiving timing of the NCR-Fwd):

- The DL receiving timing of the NCR-Fwd is aligned with the DL receiving timing of the NCR-MT.

- The UL transmitting timing of the NCR-Fwd is aligned with the UL transmitting timing of the NCR-MT.

It’s conclude that legacy UE mechanism is sufficient to achieve DL/UL timing for NCR-MT

6.3	Information on UL-DL TDD configuration

For the TDD UL/DL configuration of network controller repeater, at least semi-static TDD UL/DL configuration is needed for network-controlled repeater for links including C-link, backhaul link and access link(FFS: handling of flexible symbols)

The same TDD UL/DL configuration is always assumed for backhaul link and access link. Additional, the same TDD UL/DL configuration is assumed for C-link and backhaul link and access link if NCR-MT and NCR-Fwd are in the same frequency band.

6.4	ON-OFF information

ON-OFF information is beneficial and recommended for network-controlled repeater to control the behaviour of NCR-Fwd. (FFS: Detailed mechanism of ON-OFF indication and determination FFS: explicit indication or implicit indication of ON-OFF information)

The following options can be considered to indicate the ON-OFF information from gNB to NCR for controlling the behaviour of NCR-Fwd:

- Option 1: Explicit indication with on-off state (e.g., via dynamic or semi-static signalling) or on-off pattern (e.g., periodic/semi-static ON-OFF pattern or new DRX-like pattern for ON-OFF)

- Option 2: Implicit indication via the signalling for other information (e.g., beam, DL/UL configuration, or PC information)

  - Note: This example does not imply that PC information is necessary or not.

- Other solutions (e.g., potential combination of explicit and implication solution) can be further discussed.

6.5	Power control information

7	L1/L2 signalling for side control information

7.1	Signalling for side control information

[Editor’s Note: This clause includes the candidate signalling for each side control information including the required enhancement on other aspects, which will be captured in sub-clause.]

7.1.1	Signalling for beam information

From the perspective of signaling design, following mechanisms can be considered for the access link beamforming of the NCR-Fwd.

- Option #2-1: Dynamic beam indication only

- Option #2-2: Semi-static beam indication only

- Option #2-3: Dynamic beam indication and semi-static beam indication

7.1.2	Signalling for timing information

7.1.3	Signalling for UL-DL TDD configuration

    - Signalling for ON-OFF information

For indication of NCR-Fwd ON-OFF for efficient interference management and improved energy efficiency, both dynamic and semi-static indication can be considered. (FFS: RAN1 to consider whether/how to handle the forwarding of broadcast and cell-specific signals/channels.)

7.1.5	Signalling for power control information

7.2	Configuration of signalling

For the configuration of signalling, the NCR-MT can obtain the necessary configuration for receiving the L1/L2 signaling of the side control information.

- Option 1: The necessary configuration is from RRC.

- Option 2: The necessary configuration is from OAM or hard-coded.

- Option 3: The necessary configuration is partially configured by RRC and partially configured by OAM or hard-coded.

The necessary configurations from RRC and/or OAM(or hard-coded) contain:

- The configurations of PHY channels to carry the L1/L2 signaling including

  - The configurations for receiving PDCCH and PDSCH.

  - The configurations for transmitting PUCCH, if needed.

  - The configurations for transmitting PUSCH, if needed.

- The configurations of L1/L2 signaling including

  - The configurations for DCI.

  - The configurations for UCI, if needed.

  - The configurations for MAC CE, if needed.

For the parameters in the necessary configurations for L1/L2 signaling, the existing parameters for PDCCH, PDSCH, PUCCH, PUSCH, DCI, UCI and MAC CE in Rel-17 are the baseline for further discussion. (This does not imply that all Rel-17 parameters will be supported for the NCR-MT. Additionally, this does not imply that PUCCH, PUSCH, UCI and MAC CE are currently agreed to be supported. Further consideration is needed.)

8	Repeater management

[Editor’s Note: This clause includes the aspect related to repeater management (i.e., identification and authorization) including the required assistance on other aspects.]

8.1	Solution on Repeater management

8.2	Specification impacts

[Editor’s Note: This clause includes the identified specification impacts for each solution based on the inputs from RAN2 and RAN3, it will be captured in sub-clause.]

9	Performance evaluation

[Editor’s Note: This clause mainly includes the potential simulation results for each side control information based on the agreement.]

10	Conclusion

Annex <A>:
<Annex title for a Technical Report>

A.1	Heading levels in an annex

Annex <B>:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-05 | RAN1#109e | R1-2205231 |  |  |  | TR Skeleton | V0.0.0 |
| 2022-05 | RAN1#109e | R1-2205496 |  |  |  | Capture the agreement in RAN1#109e | V0.1.0 |