3GPP TSG RAN WG1 #109-e		                                   					       												                 R1-2205509

e-Meeting, May 9th – 20th, 2022

Source:	Ad-Hoc Chair (AT&T)

Title:	Session Notes of AI 8.16.13

Agenda Item:	8.16.13

Document for: 	Endorsement

#### 8.16.13	UE features for DSS

[109-e-R17-UE-features-DSS-01] Email discussion on UE features for DSS – Ralf (AT&T)

- 1st check point for LS to RAN2: May 13

- Final check point for any remaining issues: May 20

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 34. NR_DSS | 34-1 | Cross-carrier scheduling from SCell to PCell/PSCell with search space restrictions (Type A) | Support of Cross-carrier scheduling from sSCell to PCell/PSCell with search space restrictions (Type A) - Cross-carrier scheduling from sSCell to PCell/PSCell with CIF - Search space restrictions: sSCell USS set(s) (for CCS from sSCell to PCell/PSCell) and at least following search space sets on PCell/PSCell can only be configured such that UE does not monitor them in overlapping slot of PCell/PSCell and sSCell - USS sets for DCI formats 0_1,1_1,0_2,1_2 - USS sets for DCI formats 0_0,1_0 - Type3-CSS set(s) for DCI formats 1_0/0_0 with C-RNTI/CS-RNTI/MCS-C-RNTI - Configuration of scaling factor α for BD and CCE limit handling and PDCCH overbooking handling on P(S)Cell - FFS: # The number of unicast DCI limits for PCell/PSCell scheduling - Processing K1 unicast DCI scheduling DL on PCell/PSCell per PCell/PSCell slot and its aligned N consecutive sSCell slot(s) - Processing K2 unicast DCI scheduling UL on PCell/PSCell per PCell/PSCell slot and its aligned N consecutive sSCell slot(s) - FFS: N is based on pair of (PCell/PSCell SCS, sSCell SCS): N=1 for(15,15), (30,30), (60,60) and N=2 for (15,30), (30,60) and N=4 for (15, 60) - Same numerology between sSCell and P(S)Cell or sSCell SCS is larger than P(S)Cell SCS - USS set(s) for DCI format 0_1,1_1 configured on sSCell for CCS from sSCell to Pcell/PSCell and USS set(s) for DCI format 0_2,1_2 configured on sSCell for CCS from sSCell to PCell/PSCell if UE supports FG 11-1 (dci-Format1-2And0-2-r16) - sSCell USS set(s) (for CCS from sSCell to Pcell/PSCell) and Type0/0A/1/2 CSS sets on Pcell/PSCell can be configured so that the UE monitors them in overlapping slot of Pcell/PSCell and sSCell - no simultaneous monitoring between ‘USS sets (for P(S)Cell scheduling) on sSCell’ and ‘Type 0/0A/1/2/CSS sets on P(S)Cell for DCI formats with CRC scrambled by C-RNTI/MCS-C-RNTI/CS-RNTI’ - simultaneous monitoring of ‘USS sets (for P(S)Cell scheduling) on sSCell’ and ‘Type 0/0A/1/2/CSS sets on P(S)Cell for DCI formats with CRC not scrambled by C-RNTI/MCS-C-RNTI/CS-RNTI’ - FFS: Support of monitoring DCI formats 0_1,1_1,0_2 (if supported),1_2 (if supported) on PCell/PSCell USS set(s) - PDCCH monitoring occasion(s) on sSCell for cross-carrier scheduling to PCell/PSCell - FFS: frame boundary alignment between PCell/PSCell and sSCell | 6-5 | Yes | N/A | Cross-carrier scheduling from SCell to PCell/PSCell with search space restrictions (Type A) is not supported | Per BC | No | Applicable to FR1 only | No | Candidate value set: One or more of supported SCS combinations ({P(S)Cell SCS in kHz, sSCell SCS in kHz}) from following set are indicated by the UE: {15,15}, {15,30}, {(15, 60)}, [{30,30}, {30,60},{60,60}]) [Candidate value set 2: frequency band pair(s) for {PCell/PSCell, sSCell}] Component 4 candidate values: (K1, K2) = {(1,1) for FDD P(S)Cell; (K1, K2) = (1,2) for TDD P(S)Cell, [(K1, K2) = (2,2) for FDD P(S)Cell; (K1, K2) = (2,4) for TDD P(S)Cell]} Component 9 candidate values: [Value 1: PDCCH monitoring occasion(s) on Pcell/PSCell and on sSCell for cross-carrier scheduling to Pcell/PSCell is within the first 3 OFDM symbols of sSCell slot overlapping with the first 3 OFDM symbols of a Pcell/PSCell slot. Value 2: PDCCH monitoring occasion(s) on Pcell/PSCell and on sSCell for cross-carrier scheduling to Pcell/PSCell is not restricted to within the first 3 OFDM symbols of any sSCell slot overlapping with a Pcell/PSCell slot] Note: The CCS from sSCell to PCell is applicable to FR1 only but there can be other SCells in FR2 configured for the UE Note: The SCell configured with Cross-carrier scheduling to PCell/PSCell is referred to as ‘sSCell’ Note: Candidate value set 2 only applies for the following value sets of components 1: {30,30}, {30,60},{60,60} Note: A UE supporting this FG does not imply that the UE can be configured with sSCell in shared spectrum | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 34. NR_DSS | 34-1a | DCI formats on PCell/PSCell USS set(s) | Support of monitoring DCI formats 0_1,1_1,0_2 (if supported),1_2 (if supported) on PCell/PSCell USS set(s) | 34-1 | Yes | N/A | DCI formats on PCell/PSCell USS set(s) is not supported | Per BC | No | Applicable to FR1 only | No |  | Optional with capability signalling |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 34. NR_DSS | 34-2 | Cross-carrier scheduling from SCell to PCell/PSCell (Type B) | Support of Cross-carrier scheduling (CCS) from sSCell to PCell/PSCell (Type B) - Cross-carrier scheduling from sSCell to PCell/PSCell with CIF - sSCell USS set(s) (for CCS from sSCell to PCell/PSCell) and search space sets on PCell/PSCell can be configured so that the UE monitors them in overlapping slot of PCell/PSCell and sSCell - Configuration of scaling factor α for BD and CCE limit handling and PDCCH overbooking handling on P(S)Cell - The number of unicast DCI limits for PCell/PSCell scheduling - Processing K1 unicast DCI scheduling DL on PCell/PSCell per PCell/PSCell slot and its aligned N consecutive sSCell slot(s) - Processing K2 unicast DCI scheduling UL on PCell/PSCell per PCell/PSCell slot and its aligned N consecutive sSCell slot(s) - FFS: N is based on pair of (PCell/PSCell SCS, sSCell SCS): N=1 for(15,15), (30,30), (60,60) and N=2 for (15,30), (30,60) and N=4 for (15, 60) - Same numerology between sSCell and P(S)Cell or sSCell SCS is larger than P(S)Cell SCS - USS set(s) for DCI format 0_1,1_1 configured on sSCell for CCS from sSCell to PCell/PSCell and USS set(s) for DCI format 0_2,1_2 configured on sSCell for CCS from sSCell to PCell/PSCell if UE supports FG 11-1 (dci-Format1-2And0-2-r16) - PDCCH monitoring occasion(s) on sSCell for cross-carrier scheduling to Pcell/PSCell - FFS: frame boundary alignment between PCell/PSCell and sSCell | 6-5 | Yes | N/A | Cross-carrier scheduling from SCell to Pcell/PSCell (Type B) is not supported | Per BC | No | Applicable to FR1 only | No | Candidate value set: One or more of supported SCS combinations ({P(S)Cell SCS in kHz, sSCell SCS in kHz}) from following set are indicated by the UE: {15,15}, {15,30}, (15, 60), [{30,30}, {30,60},{60,60})] [Candidate value set 2: frequency band pair(s) for {Pcell/PSCell, sSCell}] Component 4 candidate values: (K1, K2) = {(1,1) for FDD P(S)Cell; (K1, K2) = (1,2) for TDD P(S)Cell} Component 7 candidate values: [Value 1: PDCCH monitoring occasion(s) on Pcell/PSCell and on sSCell for cross-carrier scheduling to Pcell/PSCell is within the first 3 OFDM symbols of sSCell slot overlapping with the first 3 OFDM symbols of a Pcell/PSCell slot. Value 2: PDCCH monitoring occasion(s) on Pcell/PSCell and on sSCell for cross-carrier scheduling to Pcell/PSCell is not restricted to within the first 3 OFDM symbols of any sSCell slot overlapping with a Pcell/PSCell slot] Note: The CCS from sSCell to Pcell is applicable to FR1 only but there can be other Scells in FR2 configured for the UE Note: The SCell configured with Cross-carrier scheduling to Pcell/PSCell is referred to as ‘sSCell’ Note: Candidate value set 2 only applies for the following value sets of components 1: {30,30}, {30,60},{60,60} Note: A UE supporting this FG does not imply that the UE can be configured with sSCell in shared spectrum | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 35. LTE_NR_DC_enh2 | 35-1 | Aperiodic CSI-RS for tracking for fast SCell activation | - Aperiodic CSI-RS for tracking for fast SCell activation is triggered by enhanced SCell activation/deactivation MAC CE - Aperiodic CSI-RS for tracking for fast SCell activation is triggered within the BWP indicated by firstActiveDownlinkBWP-Id for the SCell - Maximum number of aperiodic CSI-RS resource set configurations for tracking for fast SCell activation that can be configured to UE per CC in a reported band - Maximum number of aperiodic CSI-RS resource set configurations for tracking for fast SCell activation that can be configured to UE across CCs in a reported band | 6-5 | Yes | N/A | Aperiodic CSI-RS for tracking for fast SCell activation is not supported | Per band | N/A | N/A | N/A | Component 3 candidate values: {[1,] 8,16,32,48,64,128,255} Component 4 candidate values: {[1,] 8,16,32,64,128,256,512,1024} Note: In component 3, the candidate component values {FFS 8} does not apply to FR2 Note: In component 4, the candidate component values {FFS 8} does not apply to FR2 Note: component 3 and 4 candidate values refer to the number of RS configurations for fast SCell activation that can be indicated by the MAC CE The NZP-CSI-RS configured as RS for tracking for fast SCell activation are not considered when counting the maximum NZP-CSI-RS configurations of FG2-33 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

R1-2203103	Rel-17 UE features for DSS and MR-DC	Huawei, HiSilicon

R1-2203200	Discussion on Rel-17 UE features for DSS	ZTE

R1-2203539	Maintenance on UE features for DSS	vivo

R1-2203780	Discussion on UE features for NR DSS	xiaomi

R1-2203884	UE features for DSS	Samsung

R1-2204007	Discussion on UE features for DSS	OPPO

R1-2204226	Views on Rel-17 DSS UE features	Apple

R1-2204596	On UE features for DSS and LTE NR DC enhancements	Nokia, Nokia Shanghai Bell

R1-2204696	On UE features for DSS	MediaTek Inc.

R1-2204854	Summary of UE features for DSS	Moderator (AT&T)

R1-2204964	UE features for DSS	Ericsson

R1-2205009	UE features for DSS and LTE_NR_DC_enh2	Qualcomm Incorporated

R1-2205150	Views on Rel-17 eDSS UE features 	Apple

Revision of R1-2204226