3GPP TSG RAN WG1 #109-e		                                   					       												                 R1-2205512

e-Meeting, May 9th – 20th, 2022

Source:	Ad-Hoc Chair (AT&T)

Title:	Session Notes of AI 8.16.2

Agenda Item:	8.16.2

Document for: 	Endorsement

#### 8.16.2	UE features for supporting NR from 52.6 GHz to 71 GHz

[109-e-R17-UE-features-52-71GHz-01] Email discussion on UE features for supporting NR from 52.6 GHz to 71 GHz – Ralf (AT&T)

- 1st check point for LS to RAN2: May 13

- Final check point for any remaining issues: May 20

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 24. NR_ext_to_71GHz | 24-8 | 32 DL HARQ processes for FR 2-2 | Support 32 HARQ processes in DL for 120/480/960 kHz | 24-1 | Yes | N/A | 32 DL HARQ processes for FR 2-2 is not supported | FFS Per band | N/A | N/A | N/A | A UE supporting 32 maximum number of HARQ processes for 480/960 kHz SCS for DL shall support 32 as the maximum number of HARQ processes for 120 kHz SCS for DL in FR2-2 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 24. NR_ext_to_71GHz | 24-8b | 32 DL HARQ processes for FR 2-2 - maximum number of component carriers | Maximum number of component carriers that can be configured with 32 DL HARQ processes | 24-8 | Yes | N/A |  | Per BC | N/A | N/A | N/A | Candidate component values: {1,2,3,4,6,8,16,32} | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-9 | 32 UL HARQ processes for FR 2-2 | Support 32 HARQ processes in UL for 120/480/960 kHz | 24-1 | Yes | N/A | 32 DLUL HARQ processes for FR 2-2 is not supported | FFS Per band | N/A | N/A | N/A | A UE supporting 32 maximum number of HARQ processes for 480/960 kHz SCS for DLUL shall support 32 as the maximum number of HARQ processes for 120 kHz SCS for DLUL in FR2-2 | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-9b | 32 UL HARQ processes for FR 2-2 - maximum number of component carriers | Maximum number of component carriers that can be configured with 32 UL HARQ processes | 24-9 | Yes | N/A |  | Per BC | N/A | N/A | N/A | Candidate component values: {1,2,3,4,5,8,16,32} | Optional with capability signalling |

R1-2203083	Rel-17 UE features for extension to 71 GHz	Huawei, HiSilicon

R1-2203295	Rel-17 UE features for 52.6 to 71GHz	ZTE, Sanechips

R1-2203530	Remaining issues on UE features for NR operation from 52.6GHz to 71GHz	vivo

R1-2203878	Views on UE features for supporting NR from 52.6 GHz to 71 GHz	Samsung

R1-2204115	UE features for extending current NR operation to 71 GHz	Ericsson

R1-2204136	Discussion on UE features for FR2-2	OPPO

R1-2204219	Views on Rel-17 Beyond 52.6 GHz UE features	Apple

R1-2204357	Views on UE features for supporting NR in FR2-2	NTT DOCOMO, INC.

R1-2204587	On UE features for supporting NR from 52.6 GHz to 71 GHz	Nokia, Nokia Shanghai Bell

R1-2204625	Discussion on UE features for NR above 52.6 GHz	LG Electronics

R1-2204708	Views on UE features for supporting NR from 52.6 GHz to 71 GHz	MediaTek Inc.

R1-2204780	Discussion on UE capabilities for extending NR up to 71 GHz	Intel Corporation

R1-2204850	Summary of UE features for supporting NR from 52.6 GHz to 71 GHz	Moderator (AT&T)

R1-2204999	UE features for NR from 52p6 Ghz to 71 Ghz	Qualcomm Incorporated