3GPP TSG RAN WG1 #109-e			  R1-2205515

e-Meeting, May 9th – 20th, 2022

Agenda Item:	8.10.1

Source:	Moderator (AT&T)

Title:	TP for TS38.214 on CSI feedback for eIAB

Document for:	Discussion/Approval

# Introduction

This contribution provides a TP for TS38.214 based on agreements made in RAN1#109-e for the following email discussion:

[109-e-R17-eIAB-02] Issues #1, #2, #4, #5, #8 by May 13 – Thomas (AT&T)

- 1st check point: May 13

- Final check point: May 20

- RAN2 related issues to be finalized by 1st check point

# TP for TS38.214 Clause 2.3.1.1

The following agreement was made in RAN1#109e in agenda item 8.10:

Proposal 2.2.3’: The following TP (in R1-220XXXX) on CSI feedback accounting for provided power

adjustment is endorsed for editor’s CR for TS38.214.

2.3.1.1 CSI reference resource definition ***Unchanged parts are omitted *** If configured to report CQI index, in the CSI reference resource, the UE shall assume the following for the purpose of deriving the CQI index, and if also configured, for deriving PMI and RI: - The first 2 OFDM symbols are occupied by control signaling. - The number of PDSCH and DM-RS symbols is equal to 12. - The same bandwidth part subcarrier spacing configured as for the PDSCH reception - The bandwidth as configured for the corresponding CQI report. - The reference resource uses the CP length and subcarrier spacing configured for PDSCH reception - No resource elements used by primary or secondary synchronization signals or PBCH. - Redundancy Version 0. - The ratio of PDSCH EPRE to CSI-RS EPRE is as given in Clause 5.2.2.3.1. - In addition, the IAB-MT shall account for the provided DL TX power adjustment, if indicated for the slot of the CSI reference resource by DL Tx Power Adjustment MAC CE as described in [11, TS 38.321]. - Assume no REs allocated for NZP CSI-RS and ZP CSI-RS. *** Unchanged parts are omitted ***

# TP Documentation

| Reason for change: | In RAN1#108-e meeting, it was agreed that the IAB-MT shall consider the provided DL TX power adjustment in addition to Pc when deriving CSI feedback: Agreement The IAB-MT shall account for the provided DL TX power adjustment (as indicated by the parent-node) in addition to Pc when deriving CSI feedback.. However, in the list of UE assumptions on CSI calculation in TS38.214 Clause 2.3.1.1, the agreed IAB-MT behavior for CSI feedback with the provided DL TX power adjustment is not captured under the ratio of PDSCH EPRE to CSI-RS EPRE. |
| --- | --- |
| Summary of change: | It is explicitly stated that the IAB-MT shall account for the provided DL TX power adjustment (as indicated by the parent-node) when deriving CSI feedback in TS38.214 Clause 2.3.1.1. |
| Consequences if not approved: | Ambiguous IAB-MT behavior when deriving CSI feedback if a DL TX power adjustment is provided from the parent node in addition to Pc when deriving CSI feedback. |