3GPP TSG RAN WG1 #109-e		                                                      R1-2205520

e-Meeting, May 9th – 20th, 2022

Source:	Moderator (CATT)

Title:	Summary #2 of [109-e-R18-Duplex-03] Email discussion on subband non-overlapping full duplex

Agenda Item:	9.3.2

Document for:	Discussion and Decision

- Introduction

This document summarizes the discussions under the following email thread.

[109-e-R18-Duplex-03] Email discussion on subband non-overlapping full duplex by May 20 – Yanping (CATT)

- Check points: May 12, May 18, May 20

In this summary, SBFD is used to denote subband non-overlapping full duplex, which is used by majority companies in the input contributions [2]-[32] and TR skeleton in [33].

- UL/DL resource partition for SBFD operation

This section discusses UL/DL time and frequency resource partition for SBFD (subband non-overlapping full duplex) operation at gNB side. Note that whether/how UL/DL time and frequency resource partition is informed to UE is separately discussed in section 3.

  - SBFD operation within a carrier vs. across carriers

    - 1st round discussion

According to the justification section of the SID [1], SBFD operation is within a conventional TDD band at gNB side.

TDD is widely used in commercial NR deployments. In TDD, the time domain resource is split between downlink and uplink. Allocation of a limited time duration for the uplink in TDD would result in reduced coverage, increased latency and reduced capacity. As a possible enhancement on this limitation of the conventional TDD operation, it would be worth studying the feasibility of allowing the simultaneous existence of downlink and uplink, a.k.a. full duplex, or more specifically, subband non-overlapping full duplex at the gNB side within a conventional TDD band.

SBFD operation across different carriers are discussed in [3], [15] and [30].

ZTE proposes to consider SBFD operation for intra-band contiguous and non-contiguous frequency spectrum scenarios in Rel-18 and strive for a unified solution in [3]. Ericsson assumes SBFD operation within a carrier for a baseline SBFD solution and does not focus on simultaneous transmission/reception on different carriers in [15]. Intel proposes that RAN1 should prioritize the study of SBFD operation within a carrier while SBFD operation on multiple carriers (intra-band CA with different TDD UL/DL configurations) can be studied with lower priority in [30].

Figure 2-1: SBFD operation across different carriers within a TDD band [30]

Many other companies assume SBFD operation within a carrier although they did not explicitly compare SBFD operation within a carrier with SBFD operation across different carriers.

It seems that it is common understanding that SBFD operation within a carrier should be studied, while it is not clear whether companies are also interested to consider SBFD operation across different carriers in Rel-18 study.

Proposal 2-1:

Study subband non-overlapping full duplex operation within a TDD carrier.

- FFS subband non-overlapping full duplex operation across different carriers within a TDD band

|  | Company |
| --- | --- |
| Support | Sony, IDC, Sharp, QC, New H3C, vivo, NEC, Xiaomi, Ericsson, Spreadtrum, CATT, Panasonic, Intel, ITRI, Lenovo, ETRI, Samsung, CMCC, DOCOMO, WILUS, CEWiT, KDDI, Nokia, NSB, MediaTek, LG Electronics |
| Not support |  |

| Company | Comments |
| --- | --- |
| InterDigital | The terminology “subband” here is for discussion purpose only, and the considered DL region (subband) and UL region (subband) can be a “group of RBs” level which can be flexibly configured/indicated by gNB (e.g., by an extended form of SFI). Additionally, BWP-level SBFD can also be studied. These are all anyway within a TDD carrier. |
| QC | The study should focus mainly on SBFD within at TDD carrier. |
| vivo | We do not think “intra-band CA with different TDD UL/DL configurations” is within the scope of the current SI. |
| Xiaomi | We share the views that SBFD should be within a carrier. CA with different TDD configurations is not in the scope of SBFD. |
| Ericsson | Agree with Proposal 2-1 that SBFD within a carrier should be prioritized. SBFD across carriers can be treated as 2nd priority. SBFD across carriers comes with some built in assumptions which may not hold in practice. For example, in Figure 2-1 above, it would imply UL Carrier Aggregation across 3 carriers to make use of the UL-only slot, and it is not clear that all UEs support such functionality. Hence the baseline should be SBFD within a carrier. A minor editorial comment: The main bullet of Proposal 2-1 says "Study" and the sub-bullet says FFS, which is also means "study." We assume that the intention is to attach priorities. Maybe this can be clarified in the proposal (and other proposals too). |
| Spreadtrum | Subbands refer to some frequency regions that within a carrier, which divide the whole carrier bandwidth into several parts can have different UL/DL operations. From this understanding, we think SBFD means within a carrier. |
| CATT | Our understanding of SBFD across carriers is intra-band CA with different TDD UL/DL configurations. |
| ITRI | SBFD within at TDD carrier is the first priority |
| ZTE | We are not sure about the difference between “study” and “FFS”. From our perspective, both of them are to be further studied. However, if the intention of “FFS” is to make “FFS” as a second level priority, then we are not OK with this proposal. Companies even haven’t shown the detailed solution for both “within a TDD carrier” or “across different carriers”, RAN1 should at least discuss some details for different solutions first and try to compare different solutions technically. And then we can further down-select. We propose the following. Study subband non-overlapping full duplex operation within a TDD carrier. Study subband non-overlapping full duplex operation across different carriers within a TDD band |
| Samsung | Note to FL: [10] also discusses SBFD using single carrier vs. inter-band contiguous or non-contiguous TDD CA. In addition, it is also possible to consider the use of a SUL configuration for SBFD. We support the FL proposal that SBFD operation should be studied using a single TDD carrier. There are several immediate drawbacks when using intra-band CA for SBFD: SE penalty from inter-CC guard-bands compared to single carrier case, inter-CC scheduling restrictions due to simultaneous Tx/Rx not supported by UE and increased UE complexity. It is much preferable to avoid that intra-band CA support by the UE becomes a prerequisite to SBFD deployment by the operator. Therefore, single carrier for SBFD operation should be baseline for the R18 SI evaluation without later precluding the use of intra-band CA or SUL where meaningful and possible to do so, e.g., FR2. We propose to modify the FFS part of Proposal 2-1 as follows: “SBFD can be configured in the UE channel BW of a single NR carrier. Further study the potential benefits and impacts when using intra-band CA or configuring SUL for SBFD.” |
| CMCC | Considering the workload of this SI, SBFD in single carrier should be as high priority. |
| Nokia, NSB | The study should focus on SBFD within a TDD carrier, though the required changes in specifications and requirements may be applicable to both SBFD within at TDD carrier and intra-band CA across different carriers with different TDD configurations. |
| Huawei, HiSilicon | Fine to study SBFD within a carrier. SBFD across different CCs is also interesting given that this may simply the UL-DL TDD configurations, potential collision handling rules and may also ease the discussions on requirement in RAN4. |
| MediaTek | Agree with the FL proposal. |
| Apple | Do not agree. This is out of scope. |
| LG Electronics | We are fine with FL proposal. |

Companies are invited to share your views on the study of SBFD operation across different carriers within a TDD band in Rel-18.

Question 2-1:

Which option should be adopted for study of SBFD operation across different carriers within a TDD band?

- Option 1: Study subband non-overlapping full duplex operation across different carriers within a TDD band and subband non-overlapping full duplex operation within a TDD carrier with equal priority.

- Option 2: Study subband non-overlapping full duplex operation across different carriers within a TDD band with lower priority.

- Option 3: Subband non-overlapping full duplex operation across different carriers within a TDD band is not considered in Rel-18.

|  | Company |
| --- | --- |
| Option 1 | NEC, OPPO (if question is asked for the whole SI across all involved WGs), ZTE, KDDI |
| Option 2 | Sony, IDC, QC, New H3C, OPPO (if question is asked for RAN1 priority), Ericsson, Spreadtrum, CATT, Panasonic, Intel, ITRI, Lenovo, ETRI, Samsung, CMCC, DOCOMO, WILUS, Nokia, NSB, MediaTek, SK Telecom |
| Option 3 | Sharp, vivo, Xiaomi, LG Electronics |

| Company | Comments |
| --- | --- |
| InterDigital | The terminology “subband” here is for discussion purpose only, and the considered DL region (subband) and UL region (subband) can be a “group of RBs” level which can be flexibly configured/indicated by gNB (e.g., by an extended form of SFI). Additionally, BWP-level SBFD can also be studied. These are all anyway within a TDD carrier. |
| Sharp | SFBD with different carriers requires guard bands between subbands (i.e., carriers for this case) in all slots although the guard bands are not necessary for non-SFBD region. Here, SFBD region is a time region which potentially be able to be utilized for SFBD and non-SFBD region is a time region which is not utilized for SFBD. The unnecessary guard bands cause system capacity loss which should be one of the main targets for this SI. |
| QC | We believe that non-CA based SBFD (i.e. SBFD within a TDD carrier) should be fully reused to define the CA-based SBFD. |
| vivo | We do not think “intra-band CA with different TDD UL/DL configurations” is within the scope of the current SI. |
| NEC | For initial study it is not desirable to exclude the scenario of SBFD over different carriers. We need to study advantages and disadvantages of all 3 scenarios: - SBFD within single BWP - SBFD across different BWPs but within same carrier - SBFD across different carriers |
| Xiaomi | We share same views as vivo. We don’t think CA-based SBFD should be considered. It is not only out of scope but also introducing impacts on CA operation. |
| OPPO | If the question is about the whole SI including the one in RAN4, Option 1 should be selected. For example, RAN4 needs to assess whether the existing ACIR needs some change in case of SBFD, especially when the adjacent carriers belong to different operators. |
| Ericsson | We support Option 2. We want to clarify one thing. In Question 2-1, our understanding is that "SBFD across carriers" means that different carriers would have different TDD UL/DL patterns, e.g., CC1/3 would have D-D-D-D-U and CC2 would have D-U-U-U-U. This is in contrast to 3 carriers where all 3 would have the same TDD UL/DL pattern. We think the former should be lower priority, but for the latter there is no need to de-prioritize since it is just a straightforward extension of SBFD within a carrier. In that sense we agree with Qualcomm's statement if that is what they mean by "fully reused." |
| Spreadtrum | Proposal 2-1 is with higher priority, after that if the SBFD can be directly reused for carriers, we are also fine. |
| ITRI | SBFD within at TDD carrier is the first priority |
| Intel | We think it is reasonable to first focus on SBFD within a carrier. After that, if time permits, we can check whether the mechanism can be directly applied for CA-based SBFD case and any additional mechanism is needed. Further, it may be more appropriate that RAN4 performs evaluations for the intra-band-inter-carrier scenarios than RAN1. |
| Lenovo | Methods developed for SBFD within a carrier can be reused for SBFD across carriers later. |
| ZTE | At this stage, we don’t see any clear necessity to rule out any potential solutions. RAN1 should encourage companies to provide detailed solutions and then compare them technically. |
| Samsung | We think that completely ruling out SBFD using intra-band CA in the R18 SI (Option 3) would be premature, e.g., FR2 and UE multi-beam simultaneous reception and multiple Rx chains. On the other hand, SBFD operation using single carrier should have highest priority. It is also not meaningful to give single carrier vs. intra-band CA the same priority from the resulting additional workload to conduct the system level evaluation part. This is best avoided. What we are missing in the 1st round FL summary is some general functional requirements for SBFD operation. SBFD operation by the gNB should not be disruptive to existing deployments, e.g., when the TDD cell is operated with SFBD using single carrier, then it must still be able to serve as PCell, SCell or SpCell in FR1 inter-band and the FR1-FR2 CA scenarios for the legacy and newer release UEs. Another requirement that seems important to capture is that SBFD operation using single-carrier should be supported for UEs supporting a single BWP (FG6-0) and aligned DL-UL centre frequency. UE complexity in the RF should not increase when operating in the TDD cell with SBFD. We should try to capture such design requirements in 2nd round discussions. |
| CMCC | Half-duplex CA has been supported in Rel-16, some further enhancements on top of it can be studied in this SI with low priority if time permits. |
| Nokia, NSB | Agree with Qualcomm (see also our comments to Proposal 2-1) |
| Huawei, HiSilicon | As mentioned above, in the case of CA, the legacy UL/DL TDD configuration can be reused in each carrier, and different TDD UL/DL configurations can be used in different carriers. Most likely, the collision handling rules introduced in Rel-16 can also be reused. |
| MediaTek | The study can focus on subband non-overlapping full duplex operation within a TDD carrier at the start, and can be extended at later stage to CA-based SBFD. |
| Apple | Option 3. In our view the scope of SBFD is within a TDD carrier (and CA is out of scope) |
| LG Electronics | At this stage, it is not clear whether non CA-based SBFD can be fully re-used. But, we think if non CA-based SBFD can be fully re-used, study for CA-based SBFD is not necessary in RAN1. Maybe, study in RAN4 is necessary to support CA-based SBFD. We prefer to focus on only SBFD within a carrier. |

    - 2nd round discussion

In 1st round discussion, although companies support proposal 2-1 in general, companies have different views on SBFD across carriers (intra-band CA with different TDD UL/DL configurations). Majority companies think the study should focus on/prioritize SBFD within a TDD carrier and two companies think intra-band CA with different TDD UL/DL configurations is not within the scope of the SI. Three companies prefer to study both SBFD within a carrier and across carriers with same priority.

It is not clear whether companies have the same understanding on the carrier in the discussion and it may be good to align the understanding first.

As shown in Figure   2 -2, assuming there are three aggregated carriers each with 100MHz in legacy TDD system as shown in the left figure, there are two cases of SBFD across carriers (intra-band CA with different TDD UL/DL configurations).

- Case 1: SBFD operation across legacy TDD carriers without SBFD operation within a legacy TDD carrier

Case 1 shown in the middle enable SBFD across legacy TDD carriers while in a single TDD carrier, there is no SBFD operation.

- Case 2: SBFD operation within a legacy TDD carriers by CA-based SBFD scheme

Case 2 shown in the right figure enable SBFD within a TDD carrier which is Scheme #4 CA based SBFD in section 3.2.

Figure 2-2: SBFD across carriers case 1 & case2

The intention of the original proposal was to de-prioritize Case 1 only. In addition, based on the inputs, it seems that several companies are interested in studying CA-based SBFD scheme. So it is proposed that we first agree on whether to study Case 1.

Updated Proposal 2-1:

Study subband non-overlapping full duplex operation within a legacy TDD carrier.

- FFS whether subband non-overlapping full duplex operation across different legacy carriers within a TDD band is considered.

|  | Company |
| --- | --- |
| Support | New H3C, DOCOMO, NEC, vivo, Intel, Panasonic, Xiaomi, CATT, Sony, Spreadtrum, CMCC, ZTE, Lenovo, Ericsson (FFS bullet needs clarification), OPPO, Sharp, QC, Samsung (modified wording), ITRI, Nokia, NSB, IDC |
| Not support |  |

For the FFS part, please indicate whether you support to not consider SBFD across legacy TDD carriers or consider SBFD across legacy TDD carriers with low priority.

|  | Company |
| --- | --- |
| Not considered | Vivo, Sharp |
| Considered with low priority | New H3C, DOCOMO, NEC, Intel, Panasonic, Xiaomi, Sony, Spreadtrum, CMCC, ZTE, Lenovo, Ericsson (FFS bullet needs clarification), QC, SK Telecom, ITRI, Nokia, NSB, IDC |

| Company | Comments |
| --- | --- |
| New H3C | SBFD with across different legacy carriers within a TDD band should be considered after the group completed studying SBFD in a TDD carrier because SBFD current is in SI stage and should study possible proposed scenarios. |
| NEC | Similar view as New H3C. We would not like to rule out SBFD across different carriers at this point of study. In addition, it is not clear to us why we emphasize ‘legacy TDD carrier’ here. In our understanding, the essential point is whether we consider SBFD in single carrier or multiple carriers no matter it is a legacy carrier or not. In order to make it more general, we suggest removing ‘legacy’ in the main bullet and FFS. |
| Vivo | The intra-band CA with different TDD UL/DL configurations is not in the study scope, hence no need to consider it. We propose to delete the FFS. FFS whether subband non-overlapping full duplex operation across different legacy carriers within a TDD band is considered. |
| Intel | Case 1 can be studied after we finish the study of SBFD within a carrier, if time allows. Further, it may be more appropriate that RAN4 performs evaluations for case 1 than RAN1. Besides, in our understanding, case 2 is scheme #4 under section 3.2, which would be more challenge than case 1, and have larger impact on legacy UE. |
| Xiaomi | Agree with vivo. We don’t think we need to consider the FFS point. Intra-band CA with carriers configured different UL-DL TDD configurations should be discussed in HF-CA. It should not be handled in duplex enhancement. |
| Sony | I share similar views with New H3C and Intel that SBFD across different carriers within a TDD band should be only studied if there is time left after single carrier case is completed. |
| Spreadtrum | SBFD within a carrier is with higher priority, after that if the SBFD can be directly reused for carriers, we are also fine. |
| ZTE | We can first focus on the SBFD operation within one legacy TDD carrier. |
| Lenovo | For intra-band CA based SBFD, UE behaviour may be similar to SBFD within a carrier, since UE does not expect to transmit in one carrier and receive in another carrier simultaneously. Thus, SBFD within a carrier can be extended to SBFD across multiple carriers later. |
| Ericsson | Thank-you to the moderator for illustrating the different cases of “SBFD across carriers.” It is good to try to achieve common understanding. However, in our view Case 1 and Case 2 are the same – both have 3 CCs, and the TDD pattern is different on CC1/3 compared to CC2; the only difference is the CC bandwidths between Case 1 and 2. In Case 1, the CC bandwidths are all 100 MHz. In Case 2, the CC bandwidths are 40, 20, 40 MHz. In summary, we support at least the main bullet of Updated Proposal 2-1. If the intention of the FFS is to exclude Case 1, but FFS on Case 2, we don’t think the updated wording achieves that. In our read, both Case 1 and 2 would still be open for study based on our comment above. We are open to further discuss “SBFD across carriers”, but we think it should be with lower priority. |
| OPPO | In case of SBFD across legacy TDD carriers with no SBFD in any single carrier, current physical procedure in one carrier can be reused directly. And the key issue in this scenario is handling of adjacent carrier interference, which means the major related work are with RAN4. So SBFD across legacy TDD carriers can start in RAN4. RAN1 can trigger the work based on RAN4 input if any. |
| Sharp | We are fine with the updated Proposal 2-1. On the other hand, we have similar view with Ericsson that the updated Proposal 2-1 put both Case 1 and Case 2 as FFS. We don’t think 100 MHz as legacy carrier. Legacy carrier is a bandwidth in one component carrier. |
| QC | Both cases shall be studied. However, the study should focus first on SBFD within at TDD carrier. Then, the CA-based SBFD could be defined by fully reusing the non-CA based SBFD (i.e., SBFD within a TDD carrier). |
| SK Telecom | We are support both case studies. There are a lot of use cases for SBFB. The study should focus case 2 for extended coverage and real time services. |
| Samsung | We support the intent of the proposal but suggest different wording to align with existing 3GPP terminology. We see the focus of evaluation efforts in the R18 SID on SBFD for the single carrier case (Case 1). We don’t think it should be precluded to consider SBFD through CA (Case 2), but only as a second priority. We suggest slightly different wording. (Modified) Proposal 2-1: “Study subband non-overlapping full duplex operation when configured in the UE channel bandwidth. FFS: subband non-overlapping full duplex operation when configured in the Aggregated BS Channel Bandwidth in an NR operating band.” |
| Nokia, NSB | It could be clarified in the FFS that “subband non-overlapping full duplex operation across different legacy carriers within a TDD band” essentially corresponds to enable CA between carriers with different TDD configurations within a TDD band. |
| Huawei, HiSilicon | Our views is that the SBFD operation within one carrier and across different carriers can both be studied since both of them are within of the scope of the SI. It is not clear putting an FFS on the SBFD operation across different carriers means. |
| InterDigital | SBFD operation within a legacy TDD carrier should be focused first. |

    - 3rd round discussion

In previous discussion, Samsung proposed to discuss some general functional requirements for SBFD operation. Let’s check companies’ views on the following proposal.

Proposal 2-8:

SBFD operation is supported for UEs implementing only a single DL and UL BWP pair with aligned center frequency

|  | Company |
| --- | --- |
| Support | Samsung |
| Not support | Apple, Lenovo, ZTE, CEWiT, Sony, Sharp, Nokia, NSB, MediaTek Ericsson (can support with modifications), ITRI, IDC |

| Company | Comments |
| --- | --- |
| Spreadtrum | For SBFD within a TDD carrier, DL and UL BWP pair always have the aligned frequency centre. So if the proposal wants to reuse the current UL/DL BWP pair for SBFD operation, without change of DL/UL BWP frequency centre with SBFD symbols, we are fine with this intension. Second, according to “only a single DL and UL BWP pair”, it is too strict with “only a single”, because it is easy to misunderstanding that it only allows one BWP pair share the aligned centre frequency, but the other BWP pair cannot aligned with each other. Thus, “single” can be removed. So, small update for the proposal: SBFD operation is supported for UEs implementing still assuming only a single DL and UL BWP pair with aligned center frequency |
| Apple | SBFD at gNB is doable for some legacy UEs with legacy TDD BWP configuration (in which of course UL and DL BWP have the same SCS and center frequency). But, what’s the point to take such an agreement (even with modified version) while we don’t know yet the pros and cons of such SBFD operation! |
| Lenovo | With SBFD operation at gNB, enhanced UE capability supporting a DL and UL BWP pair with different center frequencies may be beneficial, together with fast BWP switching capability. We think more discussions are needed regarding UE capabilities. |
| New H3C | We are fine with FL proposal with spreadtrum’s modificaiton in general |
| DOCOMO | Support the intention that SBFD operation is on current DL/UL BWP pair. Fine with Spreadtrum’s modification. |
| ZTE | This requires further discussion. For example, if subband is configured as “BWP”, then this proposal may not be reasonable. Also, as commented by other companies, “aligned center frequency” may not always be guaranteed. |
| vivo | Share Apple’s view and the proposal would have impacts on the detailed SBFD schemes as discussed in section 3.2. |
| CEWiT | Is this proposal wrt a single UE or multiple UEs sharing the same centre frequency? If it is for a single UE, we need clarity on “ only a single DL and UL BWP pair”. Based on our understanding, different BWP pairs can have different centre frequencies. We also agree with Lenovo that UE capability might be enhanced to support different centre frequencies for a DL and UL BWP pair. |
| WILUS | It seems quite early to restrict DL/UL BWP pair with aligned center frequency. Different center frequency can be considered for UL subband. |
| Huawei, HiSilicon | Agree with other companies that it is too restrictive even for SBFD operation within one carrier. |
| NEC | The proposal is not clear to us. Does it mean that reuse legacy TDD BWP configuration for SBFD capable UE? If so, we are fine with the proposal in general. |
| Panasonic | We are fine with UL/DL BWP pair with aligned center frequency. We share Spreadtrum’s concern that “only a single DL and UL BWP pair” can be misunderstood. Then, we are fine with Spreadtrum’s modification. |
| CATT | We prefer to discuss this later. |
| Fujitsu | Support with Spreadtrum’s modification. |
| Sony | As commented by others, further clarifications are required on the intention of this proposal as some companies have different interpretations. If the intention is to enable SBFD in a single pair of BWP then this should be made clear. |
| Intel | It is unclear to us, whether it is legacy UE or new UE for this proposal. If it is for legacy UE, maybe better to discuss under section 3.1. If it is for new UE, we tend to agree SBFD operation in TDD carrier should be workable for UEs supporting aligned frequency centre for DL and UL BWP pair as well as UEs supporting non-aligned DL and UL BWP pair, if supported. |
| Xiaomi | We share the views the current proposal introduce unnecessary restrictions. We are OK to discuss this issue later. |
| Sharp | We don’t support it. It’s restrictive at this stage. For example, BWP-based SBFD with 1 DL and 1 UL subband configuration would be excluded by this proposal. |
| OPPO | When defining RRC IE of BWP, TS 38.331 specifies that “In case of TDD, a BWP-pair (UL BWP and DL BWP with the same bwp-Id) must have the same center frequency (see TS 38.213 [13], clause 12)”. So is the value of this proposal just the restriction of “only a single pair”? |
| Nokia, NSB | We have similar view as ZTE in this round. Additionally, it is not clear how UE with single BWP pair can support this. Actually there may be different way for UE with different number of BWP. Further study should be done in this early stage and it should not have this limitation now. If SBFD operation in the BWP of legacy UE if it only supports one BWP, there will be also impact on the legacy UE. |
| LG Electronics | We think it is premature to make an agreement. Further discussion regarding UE implementation for supporting DL and UL BWP pair which centre frequency could be non-aligned seems necessary. |
| Samsung | As background to the proposal: In current Rel-15 NR specifications (213, S11), “for unpaired spectrum operation, a UE does not expect to receive a configuration where the center frequency for a DL BWP is different than the center frequency for an UL BWP when the BWP-Id of the DL BWP is same as the BWP-Id of the UL BWP.” (Note: different configured BWP pairs can have different configured center frequencies). Rel-15 UE FG 6-1 (basic BWP operation with restriction) is the most widespread and commonly implemented TDD UE approach. When we refer to “single DL and UL BWP pair with aligned center frequency”, we mean a UE implementing FG 6-1, e.g., the UE can be configured by the network with 1 DL BWP and 1 UL BWP linked through the BWP-Id in RRC, and the DL and the UL BWP of the UE must be configured with the same center frequency as by 213. Only RRC re-configuration can be used to re-configure the DL and UL BWP for FG 6-1. Motivation: If we relax the legacy “same center frequency” assumption for DL + UL BWP inherent to TDD UE operation in the NR band, we de-factor force dual-LO RFFE and change the UE RF. We think this is very undesirable. Introduction of SBFD at the gNB and a R19 UE providing enhanced support to the UE should (ideally) not require changing anything fundamental in the UE RF design. If we keep the “same center frequency”, but design SFBD around the assumption that multiple BWPs (e.g., FGs 6-1a to 6-4) are available in the UE implementation, we also increase UE complexity (and that is one of the reasons why these FGs are not popular UE implementation choices as by now). We do not want to preclude enhanced SBFD support by UEs implementing > 1 BWP, but we think it is much preferable if multiple BWP support by the UE “just” for purpose of SBFD doesn’t become a prerequisite for the UE. This would then much raise the burden for UE design to implement any feature in support of gNB side SBFD. Some additional discussion points: (1) As some companies have pointed out, and we share that view, when only 1 DL and UL BWP is available in the UE, this creates constraints with respect to the BWP configuration depending on the SBFD UL SB frequency occupancy and the NR carrier BW. However, the BWP can always be configured to contain the UL SB. Currently, when SBFD is operation in the TDD cell with legacy UEs, the same constraints exist for the network. (2) BWP switching delays (using DCI or timer-based) exist, e.g., 5 (or 2) slots for 30 kHz depending on UE capability. We do not think these are likely to change, because they derive from UE decoding delays, BB and RF even if a BWP is “re-interpreted” in ASN.1, e.g., gets a transmission direction. We are fine with the proposed modification to the FL proposal by Spreadtrum to remove “single” if that can help to resolve ambiguity. |
| QC | If the intention of the proposal is legacy UE, then are fine. If so, the following should be clarified in the proposal. This means that SBFD is transparent to legacy UE. SBFD operation is supported for legacy UEs implementing only a single active DL and UL BWP pair with aligned center frequency |
| MediaTek | No need to have restriction on BWPs for SBFD-capable UEs. |
| Ericsson | We think it is reasonable to strive for an SBFD solution (even for new UEs) that will work with only a single BWP pair with aligned center frequencies. We think there would need to be some small spec changes in 38.213 to allow DL RBs in an UL BWP, but it would need to be made clear in the spec that that DL and UL cannot be received/transmitted simultaneously (from a UE perspective). We also think it is reasonable to strive for an SBFD solution that does not rely on BWP switching. In summary, we understand the intention of the proposal, but it would be too strict to say "only." Using the wording "at least" would be better. Since this is an SI, not a WI, the proposal should be rephrased to say "study" or "consider," not "support." |
| InterDigital | Premature and not clear. But, we are fine with the modified proposal by Spreadtrum. |

  - Subband time-frequency location

    - Subband time location

      - 1st round discussion

In conventional TDD systems, UL and DL within a TDD carrier are split in time domain and symbols are divided into DL, UL and flexible symbols, where flexible symbols are used as guard period for DL-UL switching [18].

Study of UL subband in legacy DL symbol(s) is widely supported by companies although companies have different views on whether UL subband is in every DL symbol or in a subset of the DL symbols, e.g. it is assumed in [15] that UL subband is configured in all DL symbols, i.e. there are no DL-only symbols. In addition, DL symbols/slots including SSBs are considered to be not available for SBFD operation due to UE backwards compatibility considerations in [10].

With UL subband in DL symbol(s), it seems that the UL subband in flexible symbol(s) is also feasible since DL-UL switching is not needed within UL subband.

Several companies [7][9][10][22][28][30] have some reservations on DL subband in legacy UL symbol(s) due to unclear motivation, UL resource fragmentation, potential CLI to UL reception for legacy gNB with conventional TDD etc.

Proposal 2-2:

Study subband non-overlapping full duplex operation (UL subband) in legacy DL and flexible symbol(s).

- FFS UL subband in all legacy DL and flexible symbols or in a subset of all legacy DL and flexible symbols

- FFS subband non-overlapping full duplex operation (DL subband) in legacy UL symbol(s)

|  | Company |
| --- | --- |
| Support | Sony, IDC, Sharp, New H3C, NEC, Xiaomi, Ericsson (with revision), Spreadtrum, CATT, Panasonic, Intel, ITRI, CMCC, DOCOMO, WILUS, KDDI, Nokia, NSB, MediaTek (with revision), LG Electronics |
| Not support | QC, vivo, OPPO, Lenovo, Samsung, CEWiT |

| Company | Comments |
| --- | --- |
| Sony | Unclear why we need to restrict that UL subband must be in all legacy DL & flexible symbols. This should be something that is configurable. |
| InterDigital | Similar view with Sony. It is flexible enough that gNB can configure multiple candidate new slot formats, each having flexible DL/UL regions (set of RBs), and then dynamically indicate/select one out of the multiple, e.g., by DCI. With this, gNB can control any intended UE behavior, meaning it is just an unnecessary restriction when preventing some legacy UL symbols, etc., as long as such configuration flexibility (selecting one out of the multiple) is given at the gNB side. |
| QC | That is a study item not a work item. It is too early to preclude DL subband configurations within legacy UL slot only based on statements without proper analysis. We believe that both schemes of subband in legacy DL slot and UL slots should be studied. Then based on the study outcome, proper conclusion should be made. In addition, the UL subband shouldn’t be applied in all legacy DL slots. Still some legacy DL slots are needed for legacy operation. |
| vivo | We would like to comment that at least the subband non-overlapping full duplex operation (DL subband) in legacy flexible symbol(s) can be supported. About whether subband non-overlapping full duplex operation (DL subband) can be supported in legacy UL symbol(s), we think it is beneficial to support it to improve the DL performance from throughput and latency perspective especially if all neighbor cells are deploying SBFD. If the DL/UL subband configuration (including the frequency and time domain range) is configurable and decided by the gNB, then there is no need to restrict it. And this is the first meeting of the SI, it is not clear what is the difference between study (main bullet) and FFS (in the sub-bullet), we suggest to not exclude any possible operation at current stage. Furthermore, the 1st sub-bullet seems not needed as it is not excluded by the main bullet. We propose the following update Updated Proposal 2-2: Study subband non-overlapping full duplex operation (UL subband) in legacy DL and flexible symbol(s). - FFS UL subband in all legacy DL and flexible symbols or in a subset of all legacy DL and flexible symbols Study subband non-overlapping full duplex operation (DL subband) in legacy UL symbol(s) |
| Xiaomi | We share same views with Sony and IDC. From our understanding, which DL symbols or flexible symbols can be occupied by UL subband is up to configuration. Regarding the argument that some DL slots are needed for legacy operation from Qualcomm, we think it can be handled by proper UL subband configuration. The principle of SBFD should be no impacts on legacy operation. For the second FFS, we don’t support it from the aspect of no motivation and more impacts on legacy UL transmission. Regarding the argument that it can improve the DL performance, we are not convinced as a DL dominated UL-DL TDD configuration can be configured at the first place to guarantee DL performance. I think this is the typical use case for UL subband. |
| OPPO | Our major concern is that the above proposal, if stated from UE perspective, unnecessarily makes the Rel-18 duplex non-transparent to UE. The UE in the current spec would not see any UL grant in legacy DL symbols. We can be ok to the following revision. Proposal 2-2 (revision): Study subband non-overlapping full duplex operation (UL subband) in legacy DL and flexible symbol(s) from gNB perspective. - FFS UL subband in all legacy DL and flexible symbols or in a subset of all legacy DL and flexible symbols - FFS subband non-overlapping full duplex operation (DL subband) in legacy UL and flexible symbol(s) from gNB perspective. |
| Ericsson | Comment #1: We think that 'D' subband in a legacy UL slot is not motivated. The intention of this study item (see Justification section in WID) is to provide more UL opportunities to improve UL coverage and latency. It is out-of-scope to consider increased DL opportunities, since DL is not the bottleneck according to the justification in the SID: TDD is widely used in commercial NR deployments. In TDD, the time domain resource is split between downlink and uplink. Allocation of a limited time duration for the uplink in TDD would result in reduced coverage, increased latency and reduced capacity. As a possible enhancement on this limitation of the conventional TDD operation, it would be worth studying the feasibility of allowing the simultaneous existence of downlink and uplink, a.k.a. full duplex, or more specifically, subband non-overlapping full duplex at the gNB side within a conventional TDD band. Furthermore, allowing 'D' subband in legacy UL slot creates gNB-gNB CLI toward a legacy (static TDD) operator, which is undesirable from a coexistence perspective. Comment #2: We think that it requires more discussion on how to handle 'F' symbols/slots. It is not clear that legacy UEs fully support or are fully tested to be able to operate with undetermined UL/DL direction (even though it is supported by the spec) for more than just a guard time. Most (if not all) deployments to date are static TDD where the UL/DL direction is known in advance. Hence as a first step, we should prioritize at least UL subband in a DL symbol, and then further discuss how 'F' symbols should be used. Based on the above comments we suggest the following revision: Proposal 2-2 (modified) Study subband non-overlapping full duplex operation (UL subband) in legacy DL and flexible symbol(s). - FFS UL subband in all legacy DL and flexible symbols or in a subset of all legacy DL and flexible symbols - FFS Whether/how SBFD operates in flexible symbols, considering legacy UE support of flexible symbols for more than just a guard period - FFS subband non-overlapping full duplex operation (DL subband) in legacy UL symbol(s) |
| Spreadtrum | If the intention is only study the semi-static D->UL, and flexible-> UL as the higher priority, we are fine. Because from the justification of the SID, the main benefit is to increase the UL coverage, UL delay etc. For the case of semi-static U->DL, it can be concluded into lower priority. |
| Intel | For DL subband in UL symbol, we fail to understand how such configuration can improve UL. The motivation of Rel-18 duplex SI is clearly captured in justification part, which is to improve UL, not to increase DL resource. “In TDD, the time domain resource is split between downlink and uplink. Allocation of a limited time duration for the uplink in TDD would result in reduced coverage, increased latency and reduced capacity. As a possible enhancement on this limitation of the conventional TDD operation, it would be worth studying the feasibility of allowing the simultaneous existence of downlink and uplink, a.k.a. full duplex, or more specifically, subband non-overlapping full duplex at the gNB side within a conventional TDD band.” Besides, we clearly see the degradation of legacy gNB UL reception caused by DL subband in UL symbol, even if legacy TDD configuration is aligned. Clear justification and performance evaluation are needed to demonstrate the benefit of supporting DL subband in UL symbols, which in our view, is not worthy considering in Rel-18. Regarding whether a subset of all DL symbols can be configured with SBFD operation, we’re fine to further study, considering potential impact on initial access or idle state. |
| ITRI | For the first FFS, it should be configurable. For the second FFS, both schemes of subband in legacy DL slot and UL slots should be studied. |
| Lenovo | We think both UL subband and DL subband should be studied. The suggested modifications are shown below: Proposal 2-2: Study subband non-overlapping full duplex operation (UL subband) in legacy DL and flexible symbol(s) and non-overlapping full duplex operation (DL subband) in legacy UL symbol(s). - FFS UL subband in all legacy DL and flexible symbols or in a subset of all legacy DL and flexible symbols - FFS subband non-overlapping full duplex operation (DL subband) in legacy UL symbol(s) |
| ZTE | The definition of “UL subband” should be clarified first to reach common understanding among companies. Currently, “UL subband” seems to refer to “subband non-overlapping full duplex operation”. But it is a little bit confusing from our perspective. Similar issue for DL subband. Also, agree with Sony, the time domain for FD operation should be configurable. |
| Samsung | In urban macro and suburban macro deployment cases, it is expected that due to high Tx power and/or frequent LoS conditions for wide and medium area base station classes, gNB-to-gNB interference dominates. SBFD using DL subbands in UL slots is therefore less attractive than for small cell deployments like InH or InF using local area base station classes. Also, legacy UE should use the UL slot at least for random access procedure. If DL subband is introduced in the UL slot, we need to study how to ensure the legacy UE transmits PRACH, msg3 PUSCH, and PUCCH for mag4 PDSCH. At this end, the legacy UE’s UL performance may be degraded due to limited frequency hopping bandwidth, which should be avoided. It is also clear that assuming “all” DL or F symbols/slots are available for SBFD is not possible when considering SSBs and legacy UEs (EPRE assumptions). SI evaluations will show and allow proper conclusion for which deployments and assumptions the use of SFBD in D, F or U is meaningful and possible. The symbols/slots in which the SBFD UL subband can be configured by the gNB should be flexible. This is however more a Stage 3 design question. We propose modified Proposal 2-2: Study subband non-overlapping full duplex operation (UL subband) in legacy DL and flexible symbol(s). - FFS UL subband in all legacy DL and flexible symbols or in a subset of all legacy DL and flexible symbols - FFS subband non-overlapping full duplex operation (DL subband) in legacy UL symbol(s) |
| CMCC | Since the Tx power of gNB is much higher than UE, the adjacent channel CLI in full DL subband in legacy UL symbol(s) is much severer than UL subband in legacy DL symbols, it may not feasible to deploy it in coexistence scenario. |
| WILUS | Rel-18 Duplex is motivated from reduced coverage, increased latency, and reduced capacity of uplink in TDD. Therefore, study on subband non-overlapping full duplex operation (UL subband) in legacy DL and flexible symbol(s) should be prioritized. |
| CEWiT | We should study both UL and DL sub-bands to allow flexibility at the gNB. Further, SBFD operation should be configurable across slots. |
| Nokia, NSB | Tend to agree with Sony and InterDigital. It is not clear what would be the reason to have a limitation on configuring UL sub-band in all DL symbols. Having DL-only symbols may facilitate operation with legacy UEs. Related to DL subband in UL symbols, our concern is that allowing DL transmissions in UL legacy symbols may be much more detrimental to legacy TDD operating in adjacent channel (as compared to allowing UL transmissions in DL legacy symbols). Also, as the main motivation for the SI is to improve UL capacity, coverage and latency, it makes sense to focus on enabling UL subband in DL legacy slots, though enabling DL subband in UL legacy slots may not be precluded. |
| Huawei, HiSilicon | There are two main benefit of SBFD: one is to improve the UL coverage by utilizing more UL resources, the other is the latency reduction. Allowing the configurability of subband non-overlapping full duplex operation (DL subband) in legacy UL symbol(s) could potentially provide DL scheduling opportunities in UL symbols hence should also be studied. Overall, there is no need to put any restrictions on subband configuration at this early stage of the SI. One need to look into the detailed performance potentials, co-existence possibilities and specification impact. |
| MediaTek | We agree with the principle of the proposal, but the wording need to be revised. Comment#1: Regarding this part of the proposal “in legacy DL and flexible symbol(s)”, in our view “DL symbols” or “DL slots” should mean these symbols are used as DL in another legacy cell/operator. For a gNB that wants to use these “DL symbols” or “DL slots” for SBFD (i.e., having UL subband in these symbols), it is expected to be indicate these symbols as flexible symbols. Otherwise, (i.e., if the gNB indicated these symbols as D), the gNB will not be able to use these symbols for SBFD. Hence, we suggest having proper definition of what “legacy DL” symbols means in the proposal. Something like the following could be considered for the definition: - DL symbols/slots: symbols/slots that are fully used for DL (by legacy gNB and SBFD gNB). - Legacy DL symbols/slots: symbols/slots that are used for DL in legacy TDD gNB, and can be used for SBFD scheme (i.e., having UL RBs in these symbols). - UL symbols/slots: symbols/slots that are fully used for UL (by legacy gNB and SBFD gNB). Comment#2: As the justification of the SID is to increase the UL opportunities, having DL subbands in UL slots doesn’t seem aligned with the SID objective. Secondly, due to the high inter-gNB CLI, having DL subband in UL slots will cause issues to cells/operators with legacy TDD operation. Thus, avoiding DL subband in UL slots will eliminate the inter-gNB CLI towards a victim legacy cell/operation, which helps with the feasibility of SBFD in real deployments. |
| Apple | Support the intention, although the second FFS should be removed. DL sub-bands within legacy UL symbols is not aligned with the objectives defined for this SID (all related to UL enhancement) and for some scenarios can be left to gNB implementation anyway. |
| LG Electronics | Allowed maximum Tx power for legacy UL slots/symbols should be taken into account for determination whether legacy UL slots/symbols can be used to allow DL transmission from gNB with higher power than that of UEs. |

Companies are invited to share your views on the study of SBFD operation (DL subband) in legacy UL symbol(s) in Rel-18.

Question 2-2:

Which option should be adopted for study of SBFD operation (DL subband) in legacy UL symbol(s)?

- Option 1: Study subband non-overlapping full duplex operation (DL subband) in legacy UL symbol(s) and subband non-overlapping full duplex operation (UL subband) in legacy DL and flexible symbol(s) with equal priority.

- Option 2: Study subband non-overlapping full duplex operation (DL subband) in legacy UL symbol(s) with lower priority.

- Option 3: Subband non-overlapping full duplex operation (DL subband) in legacy UL symbol(s) is not considered in Rel-18.

|  | Company |
| --- | --- |
| Option 1 | QC, New H3C, vivo, OPPO (wait for regulation study in RAN4), Panasonic, ITRI, Lenovo, ETRI, ZTE, DOCOMO (1st preference), CEWiT |
| Option 2 | Sony, IDC, NEC, Spreadtrum, DOCOMO (2nd preference) , Nokia, NSB |
| Option 3 | Sharp, Xiaomi, Ericsson, CATT, Intel, Samsung, CMCC, WILUS, MediaTek, LG Electronics |

| Company | Comments |
| --- | --- |
| InterDigital | Similarly for P2-2, it is flexible enough that gNB can configure multiple candidate new slot formats, each having flexible DL/UL regions (set of RBs), and then dynamically indicate/select one out of the multiple, e.g., by DCI, as gNB can have the full flexibility to control the intended network operation. This issue on having some restrictions seems not so critical/urgent, although we are okay with Option 2 here, in principle, at this early phase. |
| New H3C | The SBFD DL in legacy UL or flexible can use the same concept as the SBFD UL in legacy DL or Flexible. We prefer to have the equal priority. |
| vivo | At least for non-coexistence scenario, e.g. all neighbour cells deploying SBFD, study of SBFD operation (DL subband) in legacy UL symbol(s) should have equal priority as the SBFD operation (UL subband) in legacy DL symbol(s). |
| NEC | The main use case of introducing DL subband within UL slots is for the case when gNB is operating under UL dominant frame structure. This scenario is mainly applicable to the case of small cells deployment. Although this is a useful scenario, but it adds significant complications for legacy UEs whose UL transmissions would need to be robustly cancelled bv the gNB very frequently. Studying SBFD over DL and legacy slots can be more easily controlled from gNB. |
| Xiaomi | We don’t see the motivation. More importantly, having a DL subband in a UL slot introduce much more significant impacts on legacy operation compared with having a UL subband in a UL subband in a DL slot. The reason is the gNB-to-gNB CLI is much more serious than UE-to-UE CLI. |
| OPPO | We oppose all options if they mean from UE perspective. Besides, before RAN1/RAN4 get a clear picture of regulation in various regions, it seems premature to focus on digging a UL resource in DL symbols while giving lower priority to having DL resource in UL symbols. |
| Ericsson | We think that 'D' subband in a legacy UL slot is not motivated, and should not be further studied. The intention of this study item (see Justification section in WID) is to provide more UL opportunities to improve UL coverage and latency. It is out-of-scope to consider increased DL opportunities, since DL is not the bottleneck according to the justification in the SID: TDD is widely used in commercial NR deployments. In TDD, the time domain resource is split between downlink and uplink. Allocation of a limited time duration for the uplink in TDD would result in reduced coverage, increased latency and reduced capacity. As a possible enhancement on this limitation of the conventional TDD operation, it would be worth studying the feasibility of allowing the simultaneous existence of downlink and uplink, a.k.a. full duplex, or more specifically, subband non-overlapping full duplex at the gNB side within a conventional TDD band. Furthermore, allowing 'D' subband in legacy UL slot creates gNB-gNB CLI toward a legacy (static TDD) operator, which is undesirable from a coexistence perspective. |
| Panasonic | We think that it would be too early to deprioritize DL subband configurations within legacy UL slot at this moment. |
| Intel | As explained for proposal 2-2, we think supporting DL subband in legacy UL symbol cannot improve UL performance, and it is clearly out of scope for Rel-18 SI as commented by E///. |
| ITRI | Schemes of subband in legacy DL slot and UL slots should be studied. |
| ZTE | The definition of DL/UL subband should be clarified first. Is it within a BWP, equal to a BWP or considered as a cell? |
| Samsung | See our comment on proposal 2-2 |
| DOCOMO | It seems too early to preclude DL subband in legacy UL symbol at this stage. But we are fine to take it as low priority. |
| WILUS | As we commented in proposal 2-2, study on subband non-overlapping full duplex operation (UL subband) in legacy DL and flexible symbol(s) should be prioritized. |
| Huawei, HiSilicon | See our comments to the above question. |
| MediaTek | Same reasoning as explained in our response to proposal 2-2. |
| Apple | Option 3, same comment as P 2-2 (DL sub-band is not aligned with objectives defined in SID) |
| LG Electronics | Option 3 is preferred. |

      - 2nd round discussion

Companies’ views on whether to study SBFD operation in legacy UL symbols are quite divergent. It is therefore proposed to separately discuss SBFD operation in legacy UL symbols later. The updated proposal 2-2a only covers SBFD operation in DL and flexible symbols.

Updated2 Proposal 2-2:

Study schemes for subband non-overlapping full duplex operation (UL subband) in legacy DL and flexible symbol(s).

- FFS UL subband in all legacy DL and flexible  and flexible symbols or in a subset of all legacy DL and flexible symbols

- FFS subband non-overlapping full duplex operation (DL subband) in legacy UL symbol(s)

|  | Company |
| --- | --- |
| Support | New H3C( with modification), NEC, Intel (with revision), Panasonic Xiaomi, CATT, Sony, Spreadtrum, CMCC, Ericsson, Sharp, SK Telecom, Samsung, ITRI, Nokia, NSB, IDC |
| Not support | vivo, ZTE, Lenovo, OPPO, QC |

| Company | Comments |
| --- | --- |
| New H3C | Description of main bullet is already quite clear. Sub-bullet on FFS isn’t clear to us. It isn’t sure whether it is enough that we only consider schemes of UL suband for flexible symbols? So we suggest deleting sub-bullet related to FFS part. |
| DOCOMO | We are fine to separately discuss DL subband on UL symbols, and we are generally fine with the proposal. But we have question on “and flexible”. If “a subset of all flexible symbols” can be included, why “all legacy flexible symbols” should be excluded. |
| NEC | Fine with the proposal in general. For the FFS, it is not clear to us whether UL sub-band in flexible symbols only is allowed or not. |
| vivo | UL subband in legacy DL/flexible symbol(s) and DL subband in legacy UL/flexible symbol(s), both are SBFD schemes. There is no need to have separate discussion, we should not prioritize one over the other in the first SI meeting. We have following comments: - gNB-to-gNB inter-/intra-subband CLI cannot be worse than full band CLI in dynamic TDD case, if preclude study the DL subband in legacy UL/flexible symbol(s), what is the reason/logic to study dynamic TDD, the study of dynamic TDD operation also need to take co-existence with legacy gNB into account - In addition, what is the difficulty to support the DL subband in legacy UL/flexible symbol(s) especially when all neighbour cells deploying SBFD? - SID scope does not preclude to study SFBD scheme of DL subband in legacy UL/flexible symbol(s), justification of the SID should not limit the study. |
| Intel | We’d like to clarify the intension of ‘and flexible’. Does it imply SBFD can be configured in all flexible symbols, regardless of SSB presence in the flexible or not? We have not studied the impact on initial access yet, we suggest to keep it open for now, so we prefer “FFS UL subband in all legacy DL and flexible symbols or in a subset of all legacy DL and flexible symbols”. |
| Moderator | The proposal is updated based on the comments by adding back ‘and flexible’. |
| Xiaomi | We are OK to delete all the flexible symbols since company have concerns on the flexible operation or the other way around. Actually we are not sure whether we need to discuss duplex on flexible symbols as gNB side it is already support duplex operation, saying it depends on gNB decision to schedule UL or DL. Regarding the first question from vivo, dynamic TDD is already supported in the specification and surely we are working on it. We shouldn’t mix SBFD and dynamic TDD together. |
| Spreadtrum | We support to keep “and flexible”, since there are some restrictions that UL transmission cannot be configured or scheduled on some flexible symbols, such as the SSB symbols, the dynamic scheduling of PDSCH symbols, and dynamic SFI indicated by SFI, etc. So, at this early stage, only UL symbol is nature for UL transmission in Rel-15, DL and flexible symbols, all have some cases that not allowing UL during those symbols. |
| CMCC | We support to study UL subband in all or partial flexible slots, regarding Intel’s question, we think it can be further discussed in Proposal 4.1 on the UL/DL collision handling when SSB is transmitted in flexible slots. |
| ZTE | We are not sure what the sub-bullet means, especially for the “all”. The location of UL subband should be configurable from our perspective. |
| Lenovo | We think it would be beneficial for RAN1 to develop schemes applicable to both SBFD operation (UL subband) in legacy DL and flexible symbol(s) and SBFD operation (DL subband) in legacy UL and flexible symbol(s). RAN4 can further study potential band-specific restrictions on deploying a DL subband. |
| Ericsson | Support Updated2 Proposal 2-2 One point that we would like to raise is that while the spec allows symbols to be configured as flexible, it is not clear in commercial deployments whether or not all legacy UEs fully support configuration with undetermined UL/DL direction. This may not be fully tested. Hence, we think this point should be discussed further, since if RAN1 develops SBFD schemes that rely on legacy UEs to support 'flexible' symbols, then it may be a problem for a coexistence perspective. We could add a sub-bullet like Study should consider whether or not legacy UEs fully support undetermined UL/DL direction, i.e., cell-common configuration with 'flexible' symbols |
| OPPO | First, it is not clear to us whether the proposal is from gNB perspective or UE perspective. Please clarify. Secondly, if the proposal is from UE perspective, - For DL symbols, RAN1 should firstly discuss whether the REl-18 UE should be informed of UL transmission in DL symbols, instead of hiding this essential issue inside a weak proposal for “study”. - For flexible symbols, in current spec, the flexible symbol (if still flexible after DCI 2_0 indication) means the symbol being fully reserved and likely inaccessible to NR. So it needs to clarify whether the flexible symbol is the one after semi-static TDD UL/DL configuration or after DCI 2_0 indication. |
| QC | Too early to preclude without any study! The Study should include both schemes (UL subband in legacy UL slots and DL subband in legacy UL slot). |
| SK Telecom | We support to study UL subband in all or partial flexible slots. We would like to have many options. |
| Samsung | See our Round 1 comments. |
| Nokia, NSB | As we commented in first round, it is still unclear what would be the reason to impose the limitation with UL subband in all legacy DL and flexible symbols. |
| Huawei, HiSilicon | We are not sure what it implies by removing the second FFS bullet, it can still be studied, or? |
| InterDigital | We are fine to separately discuss DL subband on UL symbols, and see our 1st round comments. |

      - 3rd round discussion

Updated Proposal 2-2a:

Study schemes of UL subband in legacy DL and flexible symbols configured by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated.

- Note: Legacy DL and flexible symbols are from gNB’s perspective

|  | Company |
| --- | --- |
| Support | Lenovo, New H3C, DOCOMO, CEWiT, WILUS, NEC(support in principle), Panasonic, CATT, Fujitsu, Intel , Xiaomi, Sharp, LG Electronics, Samsung (modified wording) Ericsson (modified) |
| Not support | vivo, Sony, OPPO (some issues need to be discussed first) , Nokia, NSB, QC, ITRI |

| Company | Comments |
| --- | --- |
| Spreadtrum | For the flexible symbols, as commented before, we supported it. For example, there are maybe some DL scheduling for one UE in some frequency resources, and UL scheduling for another UE using other frequency resources in the same symbols, which is not supported before. These different DL/UL directions in flexible symbol can be achieved by UL subband in flexible symbol, and it is an important cases for study. For tdd-UL-DL-ConfigurationCommon, we supported it. Since a TDD carrier always has this cell-specified configuration, it is the fundamental slot format of UL/DL/flexible. However, we hesitate to agree with tdd-UL-DL-ConfigurationDedicated, which may or may not be configured since Rel-15, and it can be UE-specific slot format information, which can used as signaling for SBFD symbols. So, we propose to delete or FFS “tdd-UL-DL-ConfigurationDedicated”, and focus on DL and flexible symbols configured by tdd-UL-DL-ConfigurationCommon. |
| New H3C | We are fine with this proposal. |
| ZTE | The “schemes” in the proposal is not clear, does it refer to how to indicate the resource of subband to UE? |
| vivo | For the 1st SI meeting, no need to limit our study scope. In reality, we have not only DL heavy TDD configuration, there is UL heavy TDD config. for vertical industries and for such case, the DL may become the bottleneck for the overall performance. About the concern on the interference to legacy gNB, the subband CLI interference is more relaxed compared to the full band CLI in dynamic TDD that we are going to study, we do not see the logic to study dynamic TDD considering the co-existence scenario, but preclude study one SBFD scheme that DL subband in legacy UL/flexible symbol(s). We suggest following modification for study: Proposal 2-2a: Study schemes of UL subband in DL and flexible symbols configured by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated. Study schemes of DL subband in UL and flexible symbols configured by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated. |
| Huawei, HiSilicon | We tend to agree with vivo that there is no need to preclude the study of DL subband in UL/flexible symbols. The possible usage scenario from our point of view is indoor low latency URLLC traffic and it is preferred that a UE can transmit/receive |
| NEC | Fine with the proposal in general. We are not sure whether the tdd-UL-DL-ConfigurationDedicated will be also applied for SBFD capable UE to configure the transmission direction of the sub-band, if so, it is strange that SBFD capable UE will operate UL transmission in UL subband in DL symbol configured by tdd-UL-DL-ConfigurationDedicated. So we also suggest to remove “and tdd-UL-DL-ConfigurationDedicated” from the main bullet, we are aslo fine to have a FFS for tdd-UL-DL-ConfigurationDedicated if needed. |
| Sony | Do we really need the restriction that SBFD only applicable to semi-static configured DL & FL symbols? So what happens if a semi-static configured FL symbol is indicated as DL by SFI? Do we rule that out already in the 1st meeting? I think it is better not to impose any restrictions at this point, and perhaps make it palatable for vivo by not excluding DL subbands in UL/FL symbols i.e. we can make the proposal simple, i.e.: Study schemes for at least UL subband in legacy DL and flexible symbols. |
| Intel | We are fine with the proposal. A minor revision as below, considering it is possible only one of the parameters are provided by gNB. Study schemes of UL subband in legacy DL and flexible symbols configured by tdd-UL-DL-ConfigurationCommon and/or tdd-UL-DL-ConfigurationDedicated. |
| Xiaomi | We are fine with FL proposal. As only semi-static UL DL TDD configuration is included in the proposal, I am wondering how to handle the dynamically indicated SFI. |
| OPPO | We think the discussion of proposal depends on quite a few things first: - Whether UL subband should be non-transparent to UE (given it has to be transparent to legacy UE)? - Whether UL-subband is semi-statically or dynamically defined? (as in Proposal 2-6, this relates to why the proposal terminates at semi-static TDD configuration) - Whether the UL-subband is cell-specific or UE-specific (in case of non-transparency)? (this relates to why TDD-ULDL-Dedicated is needed on top of TDD_ULDL_Common). |
| Nokia, NSB | Not clear if the proposal is to be understood from UE perspective or from gNB perspective. Assuming this is from UE perspective, we think that only “schemes enabling UL subband in symbols configured as DL by tdd-UL-DL-ConfigurationCommon” should be studied. From UE perspective, it does not make sense to have UL subband in symbols configured as DL via tdd-UL-DL-ConfigurationDedicated. If this is from gNB perspective, then we are fine with the proposal (i.e. gNB should be able to configure UL subband in symbols that are configured as either flexible or DL by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated. But no need to define gNB implementation. |
| LG Electronics | As a starting point of discussion, we are fine with the proposal. |
| Samsung | We support the idea of the proposal but think the wording can be much improved. We also propose to keep the SBFD case using UL slots FFS (see our comments in Section 2.2.2) We propose: “At least study SBFD operation when the SBFD UL SB uses DL and/or F symbols configured by tdd-UL-DL-ConfigurationCommon and/or tdd-UL-DL-ConfigurationDedicated.” FFS: SBFD operation when the SBFD DL SB(s) use(s) symbols configured as U. |
| QC | This is a study item. Too early to preclude DL subband configurations within legacy UL slot without a study first. Some companies only think for SBFD deployment in brown spectrum where we need to consider the coexistence and impact to legacy operator. However, this is NOT the only deployment for SBFD. There are cases as greenfield deployment or isolated cell where coexistence is not an issue and then there shouldn't be any restriction on UL/DL subbands. In addition, this is a whole release study item for 5G advanced and could be the foundation for 6G. So, a proper study should be conducted, and proper conclusion should be drawn for both UL/DL subband configurations based on deployment scenario. One important thing to clarify, the amount of adjacent channel interference from DL transmission of legacy operator to UL subband in SBFD slots to SBFD gNB is similar to the amount of adjacent channel interference from DL subband in legacy UL slot to UL slot in legacy operator. If there are solutions by the SBFD gNB to handle adjacent channel interference from legacy operator, then there could be solutions that could be studied to enable DL subband configured in UL/F symbols while coexisting with legacy operator. It is counter intuitive that companies are willing to study schemes for UL subband without considering the feasibility of both adjacent channel and co-channel interference, while at same time they argue not to study DL subband highlighting the issues of adjacent channel interference with legacy. That is not consistent in our views. Also, if UL subband is excluded, how can this design be used for intra-band CA? Finally, one important question to the FL: AI 9.3.2 should address the following aspects according to the RAN1#109e agenda by the chair, however, we don’t see any proposal to address the feasibility study. We suggest a different version of the proposals above to focus on the feasibility of UL/DL subband schemes. The feasibility study should take into account the number of subbands, subband sizes (e.g., to accommodate common signalling), SBFD deployment scenario and coexistence with legacy operation co-channel and adjacent channel. Study the feasibility and schemes of UL and DL subbands configuration in legacy slots considering the SBFD deployment scenario and coexistence with legacy operation. |
| MediaTek | We support the intention of the proposal. However, the wording could be misleading. For legacy UEs, it is not clear how an UL transmission can be scheduled in symbols that are configured as DL in by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated. |
| Ericsson | We support the proposal, and we do not see a justification for 'D' subband in legacy 'U' symbols. Based on some of the comments regarding tdd-UL-DL-ConfigurationDedicated, probably better to modify the proposal as follows: Study schemes of UL subband in legacy DL and flexible symbols configured by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated. - Note: Legacy DL and flexible symbols are from gNB’s perspective We still think that SBFD solutions should not rely on configuration of 'F' symbols, since it is far from clear that legacy UEs will operate properly in a commercial network. Hence, we still suggest to add the following note: Note: Study should consider whether or not legacy UEs fully support operation with undetermined UL/DL direction, i.e., in flexible symbols |
| ITRI | First, DL subband configurations within legacy UL slot could be studied as well. Secondly, for the proposal, D/F symbols are common to all UEs, so configured by tdd-UL-DL-ConfigurationCommon. And UL subband is for R-18 UE, it could be configured by tdd-UL-DL-ConfigurationDedicated(UE-specific) as a starting point. |

      - 4th round discussion

Proposal 2-2b:

At least study SBFD operationschemes ofwith UL subband in DL and flexible symbols configured by tdd-UL-DL-ConfigurationCommon and/or tdd-UL-DL-ConfigurationDedicated.

|  | Company |
| --- | --- |
| Support | NEC (with modifications), Nokia, NSB (with modification), Lenovo, Ericsson (with modification), Samsung, CMCC, Xiaomi, Intel, WILUS, MediaTek, Fujtisu, CATT, KDDI |
| Not support | Spreadtrum, vivo, ITRI, CEWiT, Sony, QC, OPPO |

| Company | Comments |
| --- | --- |
| Spreadtrum | Still have concerns toward “tdd-UL-DL-ConfigurationDedicated”. First, there is always tdd-UL-DL-ConfigurationCommon for a TDD carrier, so there is no possiblity that without tdd-UL-DL-ConfigurationCommon but with tdd-UL-DL-ConfigurationDedicated. Thus the “or” in the bullet does not make sense. Secend, tdd-UL-DL-ConfigurationDedicated can be UE-specific slot format information, which can used as signaling including the SBFD/UL subband symbols for a UE. Thus, this SBFD scheme only use UL subband in DL/flexible configured by tdd-UL-DL-ConfigurationCommon. We think it also work and cannot preclude it now. Last, the DL and flexible symbols configured by tdd-UL-DL-ConfigurationCommon have the largest available symbol region for UL subband of SBFD, it can cover all the cases that can be configured by dedicated signal or dynamic SFI or DCI. Since DL configured by tdd-UL-DL-ConfigurationDedicated can be DL or flexible configured by tdd-UL-DL-ConfigurationCommon; and Flexible configured by tdd-UL-DL-ConfigurationDedicated can only be flexible configured by tdd-UL-DL-ConfigurationCommon. Similar rules apply for dynamic SFI or DCI. Therefore, the last round proposal from us is recommended, delete or FFS “tdd-UL-DL-ConfigurationDedicated”. |
| vivo | We do not support the proposal and adding “at least” does not address our concern. Without any studying to prioritize one scheme over the other is not fair, not reasonable. We support the proposal proposed by QC in 3rd Round. Study the feasibility and schemes of UL and DL subbands configuration in legacy slots considering the SBFD deployment scenario and coexistence with legacy operation. |
| ITRI | First, DL subband configurations within legacy UL slot could be studied as well. Secondly, for the proposal, D/F symbols are common to all UEs, so configured by tdd-UL-DL-ConfigurationCommon. And UL subband is for R-18 UE, it could be configured by tdd-UL-DL-ConfigurationDedicated(UE-specific) as a starting point. |
| CEWiT | We agree with the view from Spreadtrum. We do not want to limit the SBFD operation to resources configured by tdd-UL-DL-ConfigurationDedicated only. Hence, the term “or” should be removed. |
| NEC | We have similar argument as 3rd round and like other companies that applicability of tdd-UL-DL-ConfigurationDedicated is not clear. It would be preferable to keep the proposal in following format: At least study SBFD operation with UL subband in DL and flexible symbols configured by tdd-UL-DL-ConfigurationCommon FFS Indication by tdd-UL-DL-ConfigurationDedicated. |
| Huawei, HiSilicon | We think there is no need to down-prioritize SBDF operation with DL subband in UL and flexible symbols. |
| Nokia, NSB | We tend to agree with comments from Ericsson and propose to change to “At least study SBFD operation with UL subband in DL and flexible symbols configured by tdd-UL-DL-ConfigurationCommon and/or tdd-UL-DL-ConfigurationDedicated.”. Add a note that, for SBFD operation with UL subband in flexible symbols, the study should consider whether or not legacy UEs fully support operation with undetermined UL/DL direction in tdd-UL-DL-ConfigurationCommon. |
| Sony | The proposal suggest that ONLY DL symbols configured by tdd-UL-DL-ConfigurationCommon can be used for SBFD. That is if a Flexible symbol that is later converted to DL by SFI can never be used for SBFD. If this is the intention then we do not support this proposal. |
| Lenovo | The above proposal should not intend to de-prioritize SBFD operation with DL subband in UL and flexible symbols configured by tdd-UL-DL-ConfigurationCommon and/or tdd-UL-DL-ConfigurationDedicated. In the end, it is beneficial to make RAN1 specs flexible enough to support both cases (i.e. UL subband and DL subband). |
| Ericsson | Same view as 3rd round; share view with Nokia. We think a proposal that says "at least tdd-UL-DL-ConfigurationCommon is a step forward." We still think the note we suggested in the 3rd round is needed. |
| Samsung | We are fine with the FL proposal 2-2b. The “/or” can effectively be removed. 213 for example just uses “tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated if provided …” to deal with that case. It is not clear to us why would want to preclude tdd-UL-DL-ConfigurationDedicated from the proposal? Inter-op concerns w.r.t. UE implementation exist with respect to F symbols/slots, but not with respect to FG 5-1a. Let’s consider an example, where the followings are configured tdd-UL-DL-ConfigurationCommon: DFFFU tdd-UL-DL-ConfigurationDedicated: DDDUU Case 1) UL subband in DL and flexible symbols can be configured by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated - In this case, UL subband can be configured in the first 3 slots Case 2) UL subband in DL and flexible symbols can be configured by tdd-UL-DL-ConfigurationCommon - In this case, UL subband can be configured in the first 4 slots. However, since the forth slot is indicated as UL slot to a UE, gNB cannot configure UL subband in the forth slot. (it is because we do not agree DL subband in UL slot/symbol). Therefore, gNB only configure UL subband in the first 3 slots, which is same as Case 1 Based on two cases, we cannot find the motivation to remove tdd-UL-DL-ConfigurationDedicated. |
| DOCOMO | We share similar view as vivo that DL subband in UL/flexible symbols also in the scope. But we can live with the proposal considering it doesn’t preclude the possibility. |
| CMCC | We don’t see the reason to remove tdd-UL-DL-ConfigurationDedicated, the intention of this proposal is to say we study SBFD in DL/flexible symbols, what is DL/flexible symbols? The definition of tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated is much clear to answer this question. |
| ZTE | Thanks FL for the updates, we won’t object this proposal. But we also see some points to have some flexibility for further discussion, the proposal from QC in the 3rd round discussion seems to be a good middle ground between companies. |
| Xiaomi | We are generally fine with the proposal. It should be noted that tdd-UL-DL-ConfigurationDedicated may or may not be configured. From this perspective, it is not restrict the configuration to ‘and’. We suggest the following modification to cover different semi-static configurations: Proposal 2-2b: At least study SBFD operationschemes ofwith UL subband in DL and flexible symbols configured by: - tdd-UL-DL-ConfigurationCommon and/or tdd-UL-DL-ConfigurationDedicated. - Or, tdd-UL-DL-ConfigurationCommon |
| QC | We are disappointed with the direction of this discussion. Without technical discussion and proper study, we don’t see any reason to prioritize one UL subband scheme in legacy slots over the DL subband in legacy slot. With that said, we sustain a formal objection on the FL proposal. To move forward, we suggest a different version of the proposals above to focus on the feasibility of UL/DL subband schemes. The feasibility study should take into account the number of subbands, subband sizes (e.g., to accommodate common signalling), SBFD deployment scenario and coexistence with legacy operation co-channel and adjacent channel. Study the feasibility and schemes of UL and DL subbands configuration in legacy slots considering the SBFD deployment scenario and coexistence with legacy operation. |
| Intel | @ Spreadtrum, it is quite clearly captured in TS 38.213 and 331, both tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated can be optional for TDD carrier. Therefore, it is possible that gNB only configures one of them, that’s why we think ‘and/or’ would be more accurate. We’re also fine with the wording suggested by Samsung, ‘tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated if provided’, which is more aligned with the spec. tdd-UL-DL-ConfigurationCommon TDD-UL-DL-ConfigCommon OPTIONAL, -- Cond TDD tdd-UL-DL-ConfigurationDedicated TDD-UL-DL-ConfigDedicated OPTIONAL, -- Cond TDD Conditional Presence; Explanation. TDD; The field is optionally present, Need R, for TDD cells; otherwise it is absent.. @NOKIA, we don’t fully understand your comment in 3rd round ‘From UE perspective, it does not make sense to have UL subband in symbols configured as DL via tdd-UL-DL-ConfigurationDedicated.’. It seems you support UL subband in symbols configured as DL via tdd-UL-DL-ConfigurationCommon. Let’s say, via tdd-UL-DL-ConfigurationCommon is D D D F U, and UL subband is in 2nd and 3rd DL slot and Flexible symbol. Then, do you mean, if gNB provides tdd-UL-DL-Dedicated to UE, 2nd and 3rd slot can NOT be indicated as DL? In our view, it conflicts with legacy rule, i.e., cell-specific DL/UL can not be changed by UEs-specific DL/UL. Furthermore, cell-specific flexible symbols can be overridden by DL symbols for UE specific TDD UL/DL configurations. In this case, if DL symbols can be used for SBFD operation, why DL symbols configured by UE specific TDD UL/DL configurations cannot be used for SBFD operation? In summary, we don’t see any motivation/reason to remove tdd-UL-DL-ConfigurationDedicated. |
| OPPO | The situation is not quite improved from previous round. We suggest to discuss on the issues that this proposal depends on. |
| Panasonic | We are generally fine with the proposal. However, we also think that DL subband in UL/flexible symbols can be considered. Then, we are fine with Qualcomm’s proposal. |
| MediaTek | Support the proposal in principle. |
| Ericsson2 | After reconsideration we are fine with the wording suggested by Samsung: ‘tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated if provided’ |

    - Subband frequency location

      - 1st round discussion

Majority companies think that a subband consists of a set of consecutive RBs for the same transmission direction (either UL or DL) and RE level subband as shown below is considered in [29] for better accommodating wide-band RS.

Figure 2-3: RE level subband [29]

In addition, several companies discussed the number of subbands within a carrier and proposed to limit the number of subbands within a carrier e.g. to three [2][5][18][30].

Proposal 2-3:

For subband non-overlapping full duplex operation within a TDD carrier, up to three subbands are considered in Rel-18.

- A subband consists of a set of consecutive RBs for the same transmission direction.

|  | Company |
| --- | --- |
| Support | QC, NEC, Ericsson, Spreadtrum, CATT, Intel, ITRI, ETRI, CMCC, DOCOMO, WILUS, KDDI, Nokia, NSB, MediaTek (1 UL SB), LG Electronics |
| Not support | IDC (too early to restrict up to 3), Sharp, New H3C, Panasonic, Lenovo, Samsung (allow 1 UL subband), CEWiT |

| Company | Comments |
| --- | --- |
| InterDigital | It should be rather agreeable, in RAN1, to support at least two-subbands SBFD and three-subbands SBFD at this moment. But, strictly limiting ourselves up to three subbands are not desired and urgent. We support such a “consecutive RBs” wise subband, as RE-level seems not so needed and may be too complicated. |
| Sharp | We suggest rephrasing that “up to two DL subbands and up to one UL subband”. |
| New H3C | RB level or RBG level should be considered. |
| vivo | We can study further |
| Xiaomi | It is too early to support more than one subbands before it is justified. If we really want to agree on something about the number of subband, we think Sharp’s version can be a starting point. |
| OPPO | We thought it is one of the study targets to learn the impacts to partition any number of sub-bands, instead of deciding a max number of 3 without study. |
| Ericsson | We support Proposal 2-3 and this sets a reasonable scope for the SI. We can also accept Sharp's modification. |
| Spreadtrum | We support the proposal. Because with some restrictions of DL/UL switching points in the frequency, it can simplify the SBFD implementation. First, it can simplify the CLI analysis. Next, it can reduce the payload of signaling. In addition, it can lower UE implementation complexity of SBFD. Last, this can improve spectrum efficiency considering the unusable resources between DL/UL subbands. |
| Panasonic | We think that limiting the number of subbands would not be essential at this moment. We support “A subband consists of a set of consecutive RBs for the same transmission direction”. |
| Intel | We support the proposal and would also be fine with the version from Sharp, e.g., we can add ‘up to two DL subbands and up to one UL subband’ as sub-bullet. |
| Lenovo | Can study further. |
| ZTE | Similar view as InterDigital. It is too premature to limit the number of subbands. |
| Samsung | We are ok with the proposed definition of an (SBFD) subband that “a subband consists of a set of consecutive RBs for the same transmission direction”. We do not think that an RE-level design (as opposed to RB level) of SBFD operation currently makes sense when considering SIC implementation capabilities at the gNB. We are in principle okay with the proposal that up to 3 SBs can be configured but prefer to more explicitly capture that we mean 1 UL SB and up to 2 DL SBs. We propose to modify proposal 2-3: "For subband non-overlapping full duplex operation within a TDD carrier, up to three two DL subbands and 1 UL subband are considered in Rel-18.” |
| CMCC | Ok with sharp’s version |
| DOCOMO | Sharp’s modification is also fine for us. |
| CEWiT | As a starting point, 3 sub-bands can be considered but not restricted to this as it will reduce flexibility for gNB scheduling specially for higher bandwidths. Sub-band can be RB/RBG/BWP level. The RE level in Fig. 2-2 is a kind of rate matching only needed for wideband signals within a sub-band. The sub-band itself need not be defined at a RE level. |
| KDDI | We are fine with Sharp’s revision. |
| Nokia, NSB | We think no need to have this decision in this very beginning of the SI phase. The pros and cons of the UL at inner side of the DL or UL at edge of DL should be analyzed/evaluated and compared in detail in RAN1/RAN4 before any decision can be made. As allowing too much flexibility in subband configuration comes with additional signalling complexity, we should only enable configurations that are feasible for gNB implementation. This also requires input from RAN4. For example, depending on whether meeting the requirements for gNB self-interference and inter-subband gNB-2-gNB CLI requires the use of subband filters at the gNB. Also, we think RAN1 should ask RAN4 what is the real benefit (if any) to configure 3 subbands. While, as compared to 2 subbands, 3 subbands requires additional guard bands and may have larger specification impacts, the gain, in terms of reduced adjacent-channel interference, by placing the UL subband in the inner part of a TDD carrier is not fully clear due to the spectrum emission mask (both in-band and out-of-band) being relatively flat for both gNB and UE. |
| Huawei, HiSilicon | In our view, up to three subband (two UL subband and one DL subband, or one UL subband and two DL subband) may be sufficient in one frequency band. If the number of subband is too large, the design of BS and UE may become over complicated, e.g., more filters should be used at transmitter to suppress the leakage interference, and more filters should be used at receiver to suppress the blocking interferences. |
| MediaTek | Agree with the proposal with restriction of one UL subband. |
| Apple | Not sure why 3, one UL sub-band is sufficient (and as mentioned before, DL subband is not justified) |
| LG Electronics | We are also fine with modification from Sharp. |

Several companies discussed the possible UL subband frequency locations within a carrier considering inter-operator co-existence on adjacent channels [2][7][10][18][28][30]. Two cases where the UL subband is located in the inner part of a carrier between two DL subbands and on one edge of a carrier were identified. For the former case, there is proposal to define several candidate locations [2].

Figure 2-4: UL subband locations [18]

Proposal 2-4:

An UL subband within a carrier can be located in the inner part of the carrier or on the edge of the carrier.

- FFS candidate frequency locations of the UL subband

|  | Company |
| --- | --- |
| Support | Sony, IDC (in principle),QC, New H3C, NEC, Xiaomi, Ericsson (with minor revision), Spreadtrum, CATT, Intel , ITRI, ETRI, Samsung (with minor revision), CMCC, DOCOMO, WILUS, CEWiT, KDDI, MediaTek, LG Electronics |
| Not support | OPPO (the proposal does not say much, and with unclear terminology) , Nokia, NSB |

| Company | Comments |
| --- | --- |
| Sony | This proposal does not really say much since it isn’t clear what the alternative would be apart from the UL subband being in the inner or edge of the carrier. However, it is good to be open at this point. |
| InterDigital | Same view with Sony, but in principle the Proposal 2-4 should be agreeable. Flexible configurability on the DL/UL subbands (in an RB-group level) at gNB seems sufficient enough to support various demands on SBFD. Some particular selections (e.g., as Fig 2-3 above) can be adopted/used for evaluation. |
| Sharp | FL summary for AI9.3.1 describes two configuration options. We think both two can be supported. - SBFD Subband configuration#1: {BD, BU, BG}={2*ND, NU, 2*NG}, which means one SBFD slot consists of one UL subband at the center of the channel bandwidth and two DL subbands at two sides of the channel bandwidth. - SBFD Subband configuration#2: {BD, BU, BG}={ND, NU, NG}, which means one SBFD slot consists of one UL subband at one side of the channel bandwidth and one DL subband at the other side of the channel bandwidth. |
| QC | Not sure the essentiality of the proposal. Other than centre and edge, where else the UL subband can be located. |
| vivo | Same views as Sony. |
| NEC | For UL subband at the edge of the carrier RAN1/RAN4 need to also study applicability of guard band to reduce interference to carrier adjacent to the UL subband which may be deployed by a different operator. |
| Xiaomi | Flexible configuration for UL subband should be supported. |
| OPPO | This proposal does not say much and does not seem to exclude anything. It is also not clear to us what “inner part” means. Is the sub-band just one RB apart from the carrier edge still called “inner part”? |
| Ericsson | We are find to further study location and size of the UL subband. Since this is a study item, the proposal should include in the word "study," consistent with other proposals. Proposal 2-4 (modified) Study An UL subband location within a carrier including can be located in the inner part of the carrier or on the edge of the carrier. - FFS candidate frequency locations of the UL subband |
| Spreadtrum | Support. Inner and edge of the carrier, both have its own deployment scenarios. |
| Panasonic | Same views as Sony. |
| Intel | We agree that both cases, i.e., UL subband is in the inner part of carrier and one the edge of carrier can be considered, but the FFS point is not clear to us. Is this correct understanding that e.g., for the case of UL subband in the inner part of carrier, where is the starting PRB and where is the ending PRB? |
| ZTE | Similar view as Sony, InterDigital and QC. |
| Samsung | We think that it makes sense that the frequency occupancy of the SBFD UL subband can be configured flexibly anywhere within the NR carrier bandwidth. In this sense we are supportive of the intention of the FL proposal. It is not clear to us that the proposed wording is suitable to best capture the intention of the FL proposal. We propose to modify the FL proposal to: “The SBFD frequency-domain occupation, e.g., the location and bandwidth of the SBFD UL subband in the BS channel BW is configurable.” |
| CMCC | Support this proposal, the intention of this proposal is to clarify that in case that three operators coexist in three adjacent carriers in the same TDD band which the carrier in the middle is deployed with SBFD operation while the two adjacent carrier sat two sides are deployed with legacy TDD operation, the UL subband is configured in the middle as the left part in Figure 2-5 to mitigate the adjacent CLI to other operators. In two operator coexistence case, the UL subband can be configured at the edge as the right part in Figure 2-6. |
| WILUS | Support. Subbands should be flexibly configurable since location of an UL subband is related with deployment scenarios among operators. |
| Nokia, NSB | We think no need to have this decision in this very beginning of the SI phase. The pros and cons of the UL at inner side of the DL or UL at edge of DL should be analyzed/evaluated and compared in detail in RAN1/RAN4 before any decision can be made. |
| Huawei, HiSilicon | Support to study different configurations. The SBFD configuration may be dependent on operator’s spectrum allocations. |
| MediaTek | Agree with the proposal, we support the modification from Ericsson. |
| Apple | Do not support. No point to this proposal, UL sub-band shall be in the canter frequency of BWP (as the operation is transparent to UE and to avoid UE open up its UL BWP unnecessarily). |
| LG Electronics | Support the proposal. |

Regarding the bandwidth of subband(s), companies think that the bandwidth should not be fixed. Moreover, some companies discussed to predefine a few candidate bandwidth(s) for subband(s) while some other companies discussed to directly indicate the number of RBs.

Proposal 2-5:

The bandwidth a subband is not fixed.

- FFS granularity

|  | Company |
| --- | --- |
| Support | Sony (clarifications required), IDC (support Sony’s version with adding “RB-level”), New H3C, NEC, Xiaomi, Ericsson (with modification), CATT, Intel, ITRI, Lenovo, ETRI (fine with Ericsson’s version), CMCC (fine with Ericsson’s version), DOCOMO, WILUS (fine with Ericsson’s version), CEWiT, Nokia, NSB, LG Electronics |
| Not support | Sharp, OPPO (need more discussions), Samsung, MediaTek (proposal is not clear) |

| Company | Comments |
| --- | --- |
| Sony | The term “not fixed” is confusing, this could mean that the subband bandwidth can change dynamically or semi-statically. However, I believe the intention is really the configurable range/values of bandwidths for a subband. A suggested proposal: The subband bandwidth size is configurable. - FFS the range of values and granularity |
| InterDigital | Similar views as Sony. We can add one more clarification on it as follows: The subband bandwidth(in RB-level) size is configurable. - FFS the range of values and granularity |
| Sharp | Intention is not clear. “fixed” mean fixed in specification, or fixed in RRC configuration, or fixed in a band, or other interpretation? We suggest putting it for further study. |
| QC | It is not clear what is meant by fixed. |
| New H3C | A modified proposal based on sony’s version: The subband bandwidth size is configurable. - FFS the range of values and granularity (RB or RBG) |
| vivo | If the intention is as Sony mentioned, we are fine with Sony’s modification. |
| NEC | One of the main objectives of SBFD is to reduce latency. Hence, the scheduling behaviour for SBFD is expected to be dynamic in nature and both the time and frequency allocations for subband may vary based on cell requirements. |
| Xiaomi | Support Sony’s version. |
| OPPO | “not fixed” is unclear. But we are also not ok with Sony’s version, because in that case the discussion so far misses one important step: whether the subband should be made transparent to Rel-18 UE or not? |
| Ericsson | Support Interdigital and Sony's revisions. Furthermore, the proposal should be "study" consistent with other proposals for the SI.: Study The subband bandwidth size is configurationble. - FFS the range of values and granularity |
| Spreadtrum | Transparent or non-transparent SBFD can be discussed first. We prefer to postpone this proposal. Since if transparent SBFD is applied, UE does not know about SBFD information, gNB can decide its subband bandwidth. For the discussion, we prefer the proposal: The subband bandwidth is not fixed at least from gNB side. |
| Intel | We have similar question as other companies. Does ‘fixed bandwidth’ indicate the bandwidth is hard-coded, or only a set of values can be configured by RRC, etc. |
| ZTE | This subband size is highly related to the deployment scenario and guard band size. Also, this highly relates to the RAN4 study. For example, if RAN4 finds that analog filter is needed for gNB self-interference, then it may not be possible to change the subband size flexibly. Overall, we think it is too early to make this proposal at this stage. |
| Samsung | It is not clear to us what “fixed” means. We cannot support the FL proposal as currently shown. It makes sense to consider if the SBFD UL SB when configured in a set of symbols/slots must then have the same frequency occupancy in those time-domain resources (or not). Before that, it may first be necessary to reach a RAN1 agreement that the SBFD UL SB configuration is known to the UE. If not, an agreement about a same or (flexible) SB configuration does not impact the UE behavior, e.g., it would be gNB implementation only. |
| DOCOMO | Support Ericsson’s revision. |
| Nokia, NSB | Generally, we agree it can be studied. The scope of the proposal is not clear. We think that the subband size should be configurable, range and values are FFS. |
| Huawei, HiSilicon | Support the intention of the proposal and fine with Ericsson’s revision. |
| MediaTek | The proposal is not clear. Is this from UE perspective or gNB perspective? We would expect that gNBs that implement SBFD will have common/fixed size and location for the UL/DL subbands to avoid intra-subband CLIs. However, from UE perspective, the proposal is not clear. As an example, if the gNB want to implement 20MH UL subband in the middle of 100MHz carrier, the gNB can configure the UEs with flexible slots and dynamically schedule the UEs to transmit or receive on subset of the RBs (e.g., UE1 40MHz DL, UE2 20MHz UL, UE3 40MHz DL). Hence, the gNB instructs the UE to transmit UL on a subset of the RBs (e.g., 10MHz in the middle of the carrier) at a given time instant and transmit on other subset of the RBs at another time instant. Thus, in such case, UEs are not aware of what are the sizes of the DL/UL subbands. |
| Apple | The proposal is not clear. Is it the BW “of” of a sub-band is not fixed? Recall that anything related to subband is a concept at gNB and between gNBs. From UE perspective, it is legacy TDD BWP configuration. |
| LG Electronics | We are fine with the proposal in principle. Also, we are fine with modification from Sony. |

      - 2nd round discussion

Majority companies agree to consider up to three subbands and some companies would like to make it clear that they are up to two DL subbands and up to one UL subband.

Updated Proposal 2-3:

For subband non-overlapping full duplex operation within a TDD carrier in legacy DL and flexible symbol(s), up to one ULthree subbands isare considered in Rel-18.

- A subband consists of a set of consecutive RBs for the same transmission direction.

|  | Company |
| --- | --- |
| Support | New H3C, DOCOMO, NEC, Intel, Xiaomi, CATT , Sony, CMCC, Ericsson, Sharp, Samsung, ITRI, Nokia, NSB |
| Not support | vivo, ZTE, Lenovo, OPPO, QC, IDC |

| Company | Comments |
| --- | --- |
| New H3C | We are fine with this proposal in principal. For sub-bullet, our modification is as below: A subband consists of a set of consecutive RBs/RBG for the same transmission direction. |
| vivo | For evaluation purpose, we are fine to assume one UL subband or one DL subband in DL or UL slot. But this is the beginning of study item phase, no need to put the limitation now. Maximum subband number should be decided later after we have better understanding/analysis about the SI-/CLI-interference handling and the frequency location of the subband. |
| Panasonic | We still think that limiting the number of UL subbands is not essential at this moment because how to allocate UL subband is not clarified yet. However, if majority companies prefer the limitation, we can live with the proposal. |
| Spreadtrum | We prefer to use similar words as Proposal 3.2, such as with the update: For subband non-overlapping full duplex operation within a TDD carrier in legacy DL and flexible symbol(s), up to one UL subband that gNB would use for SBFD operation is considered in Rel-18. - A subband consists of a set of consecutive RBs for the same transmission direction. |
| ZTE | Before having clear definition and full picture of the subband, it is premature to discuss the exact number of subbands. |
| Lenovo | We don’t think this proposal is essential at this point. |
| Ericsson | We think it is good to clarify number of UL subbands. We don't see a motivation for more than 1. |
| OPPO | - Same concern from us as for updated Proposal 2-2, regarding to symbol types being “gNB perspective” vs. “UE perspective”. - Is “up to one UL subband” the limitation from gNB perspective? i.e., is it allowed for each UE to have up to one UL subband while for gNB to see more than one UL subband? - “Up to one” includes 0 and 1. So the proposal seems to name the SBFD operation for the case when there is no UL subband in DL/Flexible symbols. - The sub-bullet is a general rule, and should not be conditioned on what is confined by the main bullet. |
| QC | We think it is too early to discuss this proposal. |
| Samsung | We support the FL proposal 2-3. We think it is important to at least agree on the number of UL subbands for SFBD, e.g., 1. This part should not be too controversial given gNB implementation capabilities for SBFD. We see the SBFD UL SB as configured from the gNB perspective, e.g., where UL transmissions for any UE in an SBFD slot can be scheduled by the gNB. Whether a particular UE is then schedulable using the SBFD UL SB or only a part of it will depend on the UE BWP configuration and other factors. We think the RB-level definition (“a subband consists of a set of consecutive RBs…”) following the example of the frequency occupancy of the R15 CSI bandwidth is for now sufficient. Whether RBG-level configuration of the SBFD UL SB is meaningful can be considered and concluded upon later in the SI. |
| Huawei, HiSilicon | The proposal seems to suggest that subband consists of UL subband and DL subband, which implies the UL/DL subband are defined/configured separately. We would like to understand whether the intention of the proposal is for discussion purpose or for signaling framework design? |
| InterDigital | It is too early to limit the number of configurable UL/DL subbands. |

For proposal 2-5, several companies commented that fixed is not clear and prefer to change it to configurable. The original reason from moderator to use fixed instead of configurable is that the latter seems to imply that the bandwidth of the subband would be configured to the UEs but it is not yet concluded whether subband location is informed to the UE.

In addition, some companies commented that proposal 2-4 does not say much. Maybe we can include the subband location in the updated proposal 2-5.

Updated Proposal 2-5:

The location and bandwidth of a subband is not fixedconfigurable.

- FFS granularity

- Note that whether the location and bandwidth of a subband are informed to UE is separately discussed.

|  | Company |
| --- | --- |
| Support | New H3C, DOCOMO, NEC, Intel, CATT, Sony, ZTE, Lenovo, Ericsson, SK Telecom, ITRI, Nokia, NSB, IDC |
| Not support | OPPO (have concern with “location”) |

| Company | Comments |
| --- | --- |
| Ericsson | vivo raised a point in the 1st round about "location." Is this just frequency domain location, or also time domain? In our view both frequency and time domain location need to be configurable. |
| OPPO | For the sub-band location (we suppose it refers to location in freq-domain), we would like to know what kinds of freq-domain locations can be before agreeing it is configurable, because it may related to implementation complexity, i.e., whether the gNB may assume to apply a general band-pass filter (if the location/BW makes the subband in the middle of whole BW but in a asymmetric way) comparing to relatively simpler low-pass/high-pass filter (if the location/BW makes the subband symmetrically around center of frequency or at the very end of whole BW) in baseband processing. We are ok to the proposal if it talks about bandwidth only. |
| Nokia, NSB | We are ok with the proposal though we would still like to have a discussion in RAN1/RAN4 about the benefits (if any) to place the UL subband in the inner part of a TDD carrier. |
| InterDigital | We support the proposal in principle. However, we prefer to remove the Note, or change ‘whether’ to ‘how’, since the main bullet says they are configurable then they should be anyhow informed to the UE even in an implicit way. |

      - 3rd round discussion

Updated Proposal 2-3b:

For SBFD operation within a TDD carrier when configured in legacy downlink or flexible symbol, at least consider only one UL subband and up to two DL subbands within the carrier from gNB’s perspective.

- FFS more than one UL subband within the carrier

- FFS: Case of SBFD operation in uplink symbols

|  | Company |
| --- | --- |
| Support | Spreadtrum, New H3C, DOCOMO, vivo, CEWiT, NEC, CATT, Fujitsu, Sony, SK Telecom, Intel , Xiaomi, Sharp, LG Electronics, Samsung, MediaTek Ericsson (with modification), ITRI |
| Not support | Apple, Lenovo, WILUS, OPPO, Nokia, NSB, QC |

| Company | Comments |
| --- | --- |
| Spreadtrum | Although we do not think DL subband within UL symbols is with high priority, we are open to list it as FFS. |
| Apple | The second FFS should be removed (SBFD for DL is not aligned with SID’s objectives which is for UL enhancements) |
| Lenovo | Too early to decide the number of subbands to be supported. |
| New H3C | We are fine with this proposal in general |
| DOCOMO | We are fine with the proposal. We prefer to keep the FFS for the DL subbands in uplink symbols. It is not excluded in the SI objective part, and it is too early to drop the case in this stage. |
| ZTE | Maybe we don’t need to rush for this proposal since the definition of subband is not clear and whether DL subband is needed is not clear yet. |
| vivo | Fine with the proposal. |
| WILUS | The second sub-bullet seems unnecessary since the main bullet denotes ‘when configured in legacy downlink or flexible symbol’. As FL proposed to separately discuss SBFD operation in legacy UL symbols later in Section 2.2.1, it also should be discussed separately. |
| Huawei, HiSilicon | We are in general fine with the proposal. |
| CATT | We are in general fine with the proposal but would like to delete ‘when configured’. For ‘legacy downlink and flexible symbol’, the wording is better to be aligned with proposal 2-2a. |
| Intel | We’re fine with the intention of the proposal, but we think maybe it is more accurate to use similar wording as updated proposal 2-2a, i.e., downlink or flexible symbol configured by tdd-UL-DL-ConfigurationCommon and/or tdd-UL-DL-ConfigurationDedicated, And we think no need to keep 2nd FFS. |
| Xiaomi | Fine with the proposal. Agree with CATT that the wording should be aligned with proposal 2-2a. |
| OPPO | Our concern for Proposal 2-2a still applies here on “when configured in legacy downlink or flexible symbol”, which we suggest to remove here without impacting the proposal intention. It is also unclear to us how the “DL subband” in this proposal is created, since “how it is created” may relate to “how many”. Is this “DL subband” defined as “remaining consecutive RBs” after giving away UL subband or it is created on gNB side by configuration as well? A related question is whether {All UL subbands}+{ALL DL subbands} must be equal to or can be smaller than the whole BW? |
| Nokia, NSB | We would prefer to also include more than one DL subband as FFS. The fact is that the benefits of configuring the UL subband in the inner part of the TDD carrier should first be confirmed by RAN4, otherwise this may only complicate the physical layer design without providing any real benefit. |
| LG Electronics | We are fine with the proposal in general. But, we think 2nd sub-bullet is not necessary under Updated Proposal 2-3b:. |
| Samsung | We support the FL proposal. |
| QC | Too early to discuss this proposal. First, we need to study the subband configuration. Please see QC comments in section 2.2.1 Updated Proposal 2-2a. |
| Ericsson | Fine with Updated Proposal 2-3b But prefer to remove the 2nd FFS. |

Updated Proposal 2-5a:

The time and frequency location of subbands are configurable at gNB side (i.e. time and frequency location of subbands are not fixed or predefined in the specification).

- Note that whether the time and/or frequency location of subbands are informed to UE is separately discussed.

|  | Company |
| --- | --- |
| Support | Lenovo, New H3C, ZTE, NEC, Panasonic, CATT, Sony, Intel, LG Electronics, Samsung, Ericsson (with modification), ITRI |
| Not support | OPPO, Nokia, NSB |

| Company | Comments |
| --- | --- |
| Lenovo | Please see a couple of minor suggested edits: Updated Proposal 2-5a: The time and frequency location of subbands are configurable by gNB at gNB side (i.e. time and frequency location of subbands are not fixed or predefined in the specification). Note that whether/how the time and/or frequency location of subbands are informed to UE is separately discussed. |
| New H3C | Current we are in the SI. I wonder whether we need put this note on “time and frequency location of subbands are not fixed or predefined in the specification” or not. |
| ZTE | We are generally fine with this proposal. |
| Huawei, HiSilicon | It is still not clear what configurability means at gNB side. Maybe the intention is the clear in the i.e. part. Updated Proposal 2-5a: The time and frequency location of subbands are configurable at gNB side (i.e. time and frequency location of subbands are not fixed or predefined in the specification). - Note that whether the time and/or frequency location of subbands are informed to UE is separately discussed. |
| CATT | We are also fine with Huawei’s version. |
| Intel | To avoid confusion of ‘configurable’ (it is not about signalling, it is just about changeable location of subband from gNB’s perspective), we prefer to directly use the wording in the bracket as we suggested in 2nd round. The time and frequency location of subbands are configurable at gNB side (i.e. time and frequency location of subbands are not fixed or predefined in the specification). |
| OPPO | As we mentioned in earlier comments, frequency location of subband may relate to the subband filtering complexity if such filtering eventually turns out to be needed. We are open to the following configurability on gNB side (for simplicity, we leave time location out since we do not have concern on time domain): - There is only one UL subband and its center frequency aligns with center frequency of DL/UL BWP. The BW of the UL subband is configurable. - There is only one UL subband and it is located at one end of UL BWP, with either the center frequency or BW (but not both) of the UL subband configurable. We would have a concern for now on the configurability for the case where one UL subband does not locate at one end of of UL BWP and its center frequency does not align with center frequency of UL BWP; in this case both center frequency and BW of the subband are configurable. |
| Nokia, NSB | In our opinion, it does not make sense to discuss of configurability at gNB side. The discussion on subband location cannot be separated from the discussion on whether subband configuration needs to be provided to the UE. If the UE is not informed of the subband configuration, then where the gNB places the UL and DL subbands is gNB implementation specific (depending on the limitation imposed by DL and UL frequency domain resource allocation in the corresponding slot/symbol). |
| LG Electronics | We agree the intention of the proposal. We are fine with modification suggestion from Huawei. |
| Samsung | We agree with the intent of the proposal. |
| QC | gNB can configure the UL/DL subbands. However, we think RAN4 input or guidance may be needed on this proposal. There could be some RAN4 requirements on the minimum or maximum size of the subbands given the location of the subband and the requirements of inter-subband selectivity and interference leakage. Suggest to add subbullet below. The time and frequency location of subbands are configurable at gNB side (i.e. time and frequency location of subbands are not fixed or predefined in the specification). - Note that whether the time and/or frequency location of subbands are informed to UE is separately discussed. - Subject to any RAN4 guidance on minimum or maximum subband size and subband location within TDD carrier. |
| MediaTek | Similar view as Huawei. |
| Ericsson | Support Intel's wording which should adresss Huawei's comment. Also we agree with QC on RAN4 guidance, and this guidance should also include - Subject to any RAN4 guidance on minimum or maximum subband and guardband size and subband location within TDD carrier. |
| ITRI | We are generally fine with this proposal. |

      - 4th round discussion

Proposal 2-3c:

For SBFD operation within a TDD carrier when configured in legacy downlink andor flexible symbols, at least consider one UL subband and up to two DL subbands within the carrier from gNB’s perspective.

- FFS more than one UL subband within the carrier

- FFS: Case of SBFD operation in uplink symbols

|  | Company |
| --- | --- |
| Support | Spreadtrum, ITRI, CEWiT, NEC, Sony, Ericsson (with revision), Samsung, DOCOMO,CMCC, Xiaomi, Intel, MediaTek, Fujitsu, CATT |
| Not support | Nokia, NSB, Lenovo, QC, WILUS |

| Company | Comments |
| --- | --- |
| ITRI | Whether subband configuration needs to be provided to the UE should be discussed as well. |
| Huawei, HiSilicon | This proposal is related to proposal 2-2b, not sure whether the discussion can be separated out. |
| Nokia, NSB | We support Ericsson’s proposal in 3rd discussion round, i.e. use Intel’s wording with QC’s note on RAN4 guidance, e.g. something in these lines: “The time and frequency location of subbands in a TDD carrier are not fixed or predefined in the specification but may be subject to any RAN4 guidance on minimum or maximum subband and guardband size, and subband location within TDD carrier.” |
| Lenovo | “DL subband” on downlink and flexible symbols is a confusing terminology. SBFD operation in uplink symbols should not be FFS. |
| Ericsson | We support Proposal 2-3c without the 2nd sub-bullet. @Nokia: It seems your comment is related to Proposal 2-5b below |
| Samsung | We support FL proposal 2-3c. We do not see why it is necessary to add notes on RAN4 guidance. Whether the time & frequency location of SBs are fixed or predefined in the specs or if there are min/max SB sizes, or if GB is comprised in the SB or not, all that should be possible to discuss and agree on later in RAN1 & RAN4. Above proposed FL agreement is only about the number of UL SBs for DL and F slots when used for SBFD. Clearly, the question of the number of SBFD DL SBs when used in UL slots must be considered separately because different configuration constraints exist for such an SBFD scenario. |
| CMCC | We also don’t think it is necessary to add RAN4 notes, this proposal is just saying the UL/DL subband configurations in RAN1 perspective. |
| ZTE | We suggest to wait until we agree whether to introduce DL subband and then come back to this proposal. |
| QC | We don’t agree with the FL proposal. It should be discussed after resolving the earlier proposal Proposal 2-2b |
| Intel | Though we still think 2nd sub-bullet is unnecessary, we’re fine with the proposal. |
| OPPO | What is the definition of DL subband? In case one UL subband is at center part of BWP in DL symbol from gNB perspective, it cuts out two DL parts at two ends of BWP. Are these two parts called DL subbands? But given the whole symbol is DL anyway, is the proposal intended to define DL subband in DL symbols? If yes, this seems against the logic in people’s mind for UL subband, which is defined in non-UL symbols. |
| WILUS | The second sub-bullet seems unnecessary since the main bullet denotes ‘when configured in legacy downlink or flexible symbol’. It can be discussed after RAN1 agree to study DL subband in UL symbol. |
| MediaTek | Support the proposal. |

Proposal 2-5b:

The time and frequency location of subbands are configurable at gNB side (i.e. time and frequency location of subbands are not fixed or predefined in the specification).

- Subject to any RAN4 guidance on minimum or maximum subband and guardband size and subband location within TDD carrier.

- Note that whether the time and/or frequency location of subbands are informed to UE is separately discussed.

|  | Company |
| --- | --- |
| Support | Vivo, ITRI, Sony, Lenovo, Ericsson, ZTE, Panasonic, MediaTek, CATT |
| Not support |  |

| Company | Comments |
| --- | --- |
| Ericsson | Support Proposal 2-5b. Also suggest the following modification "subbands within a TDD carrier" for additional clarity, but this is not essential. |
| WILUS | We are fine with the proposal in principle. More clarification on ‘time location of subbands’ seems necessary. Does it mean other than DL and flexible symbols by tdd-UL-DL-ConfigurationCommon and/or tdd-UL-DL-ConfigurationDedicated? |
| MediaTek | Support the proposal in principle. |

    - Semi-static vs. dynamic subband location

It seems that companies have different assumptions on whether the subband location is semi-static or dynamic at gNB side. At least semi-static subband location should be considered in Rel-18 and more discussion would be needed for dynamic subband location.

Proposal 2-6:

Semi-static subband location is considered in Rel-18.

- FFS dynamic subband location

|  | Company |
| --- | --- |
| Support | Sony, IDC (in principle, but needs clarification), Sharp, QC, New H3C, NEC, Xiaomi, Ericsson (with revision), CATT, Panasonic, Intel, ITRI, Lenovo, ZTE, Samsung, CMCC, DOCOMO, WILUS, CEWiT, KDDI, Nokia, NSB, LG Electronics |
| Not support | vivo, OPPO (the proposal is not clear to us), MediaTek (proposal is not clear) |

| Company | Comments |
| --- | --- |
| Sony | Although we support a configurable subband. Whether the location is semi-static configured or not depends on whether we want the subband to be transparent to the UE or not. This Proposal assume that we want Rel-18 UE to know where the subbands are. NOTE that for legacy UEs, the subband has to be transparent and hence I wonder if we want to have the discussion whether subband is transparent or not before deciding on this. |
| InterDigital | The Proposal 2-6 seems too broad. Assuming the intention of it and based on so far discussions, we can consider some updates as follows: Modified Proposal 2-6: Semi-static subband location for each configurable SBFD format is considered in Rel-18. - Dynamic selection of one among multiple configured SBFD formats is considered - FFS max number of configurable SBFD formats - FFS dynamic subband location |
| vivo | About the “location”, does it refer to frequency domain only or it includes both frequency and time domain? We think above proposal talks about the signalling, better to phrase like Subband ‘location’ can be configured by semi-static signalling in Rel-18. Both semi-static and dynamic signalling should be studied, evaluation results are expected to justify the gain. No need to limit ourselves without study. |
| NEC | Dynamic subband location (at least subband bandwidth) should be studied to effectively utilize the radio resources based on real time cell requirements. If we use semi-static subband allocation then we will end up with lower system capacity which defeats the entire purpose of SBFD operation. |
| Xiaomi | Dynamic UL subband location may be not needed considering we already have dynamic TDD. We should firstly focus on semi-static UL subband location. If time permits, we are open to discuss dynamic UL subband location. |
| OPPO | We think there is one key point missing here: whether the semi-static information can tell the UE to prepare for both DL subband and UL subband at the same time. One of the reasons to have semi-static DL/UL indication for a UE is to allow UE (RF) to prepare beforehand for the transmission/reception on a specific direction. Such purpose is gone if the indication tells both. So we prefer the following revision: Modified Proposal 2-6: Semi-static subband location is considered in Rel-18. - FFS whether a UE can be semi-statically informed of subbands with opposite transmission directions at the same time. - FFS dynamic subband location |
| Ericsson | We support semi-static configuration of the subbands. We do not agree to dynamic size/location. While quite transpartent to the UE, this would create significant implementation challenges on the gNB side. Furthermore, we don't see the need for such dynamic changes. The subband size/location is a configuration choice that an operator would make for a given deployment. Proposal 2-6: Study Semi-static configuration of subband location is considered in Rel-18. FFS dynamic subband location |
| Intel | In our understanding, this proposal is regarding whether the subband location can be changed, and whether semi-static or dynamic change is supported, if the location can be changed. It is NOT relevant to the signalling design for subband indication, i.e., how to inform such change to UE. Even if SBFD is transparent to some UEs, flexible subband configuration should be still supported. We agree at least semi-static change can be supported. We’re also open to support dynamic change of subband for better adaption to dynamic traffic variation. |
| Lenovo | Can include further clarification as follows: Proposal 2-6: Semi-static subband location in time and frequency domain is considered in Rel-18. FFS dynamic subband location in time and frequency domain |
| ZTE | At this stage, we think both semi-static vs. dynamic subband location should be included. Further down-selection should be based on the study outcome of RAN4. |
| Samsung | We think that this question is best split in 2 parts. (1) Is the SBFD configuration known to the UE, e.g., symbols/slots in time-domain and the frequency occupancy of the SBFD UL SB, e.g., start RB and number of RBs in which UL transmissions using the SBFD UL SB are possible? (2) Whether the SBFD UL SB when configured can then be used only for UL transmissions or also for DL scheduling by the gNB. Regarding (1), we think that semi-static configuration is sufficient at least as baseline. SBFD UL reception is determined by gNB implementation. Regarding (2), we think that DL dynamic scheduling by the gNB in the time-/frequency resources of the configured SBFD UL SB must be possible. It is undesirable that the introduction of the SBFD feature results in hard UL-DL resource partitioning in the TDD cell. If needed, both could be captured by the following modified proposal 2-6: (1) “The UL subband configuration, e.g., symbols/slots and the frequency occupancy in which UL transmissions can be scheduled by the gNB, can be provided to the UE. Signaling details FFS.” (2) “For a UE, the transmission direction of a symbol in the SBFD UL subband is determined by gNB scheduling” |
| CMCC | We don’t see the motivation to support dynamically change the subband configuration, if it is allowed what’s the difference from dynamic TDD? |
| KDDI | We share the same view with CMCC. |
| Nokia, NSB | We think the support for dynamic subband location should also be based on input from RAN4 related to whether subband filters are needed to meet the requirements for gNB self-interference and inter-subband gNB-2-gNB CLI. |
| Huawei, HiSilicon | As commented by other companies, we should probably discuss whether subband configurations can be transparent to UE. If the subband is transparent to the UE, there is no need to discuss the configuration. |
| MediaTek | We have similar comment to what we provided in the response to Proposal 2-5. For the example we provided in our responses there, is that considered semi-static or dynamic subband location? |
| LG Electronics | Semi-static configuration is a default operation. Further study is necessary whether dynamic configuration is beneficial or not. |

Question 2-3:

Which option should be adopted for study of dynamic subband location?

- Option 1: Study dynamic subband location and semi-static subband location with equal priority.

- Option 2: Study dynamic subband location with lower priority.

- Option 3: Dynamic subband location is not considered in Rel-18.

|  | Company |
| --- | --- |
| Option 1 | Sony, IDC (in principle, but unclear), Sharp, QC, New H3C, vivo, NEC, Panasonic, Intel, Lenovo, ETRI, ZTE, DOCOMO, WILUS, CEWiT, Nokia, NSB |
| Option 2 | Xiaomi, OPPO, Spreadtrum, CATT, ITRI, Samsung, CMCC, KDDI, LG Electronics |
| Option 3 | Ericsson |

| Company | Comments |
| --- | --- |
| InterDigital | If “dynamic” means selecting one among multiple configured SBFD formats (as mentioned in the above Modified Proposal 2-6), this should be considered as a basic feature, since the existing SFI indication mechanism already supports this. But, if “dynamic” means here some proposals to determine SBFD format “on the fly”, it needs to be discussed further in more details. |
| QC | Study whether there are any needs or benefits for dynamic SBFD indication. |
| NEC | Dynamic subband location (at least subband bandwidth) should be studied to effectively utilize the radio resources based on real time cell requirements. If we use semi-static subband allocation then we will end up with lower system capacity which defeats the entire purpose of SBFD operation. |
| Xiaomi | Considering we already support dynamic TDD, the motivation of dynamic UL subband is weak. If majority companies want to have a study on it, we are OK to treat it with low priority. |
| OPPO | The gain from SBFD is coverage improvement. Semi-static SBFD is enough to improve coverage. In addition, it seems to us the following two setup may end up with the similar effects: - dynamic subband location - semi-static subband location (at gNB and transparent to UE) + dynamic scheduling/grant |
| Ericsson | Please see our comments regarding Proposal 2-3 |
| Samsung | It is too early to preclude dynamic subband adaptation in the first meeting of Rel-18 SI. As we commented in proposal 2-6, semi-static UL subband configuration is the baseline and the gNB can indicate by scheduling DCI the transmission direction of a symbol. In addition, dynamic UL subband adaption can be studied but it should be defined as a separate UE capability such as when SFI is used for dynamic indication. We think that RAN4 guidance and confirmation will be required if the gNB RF can adjust the UL subband frequency occupancy, e.g., the bandwidth and location of the SBFD UL SB. At least to train the SIC and for inter-cell CLI management, additional RF components/requirements are needed depending on the SBFD UL SB bandwidth and location. |
| Nokia, NSB | We would like to clarify difference between dynamic indication of subband location (i.e. dynamically changing the UL-DL frequency domain partitioning in SBFD symbols/slots) and dynamic indication of SBFD slots/symbols. To our understanding, this proposal/discussion is about the former. Based on this assumption, RAN1 should study the benefits for dynamic indication of subband location but only after confirmation from RAN4 that dynamic subband location is feasible from gNB implementation perspective. |
| Huawei, HiSilicon | Dynamic subband location location is much more challenging than semi-static subband location in particular for macro deployment, where the UL will potentially be impacted by strong gNB-gNB co-channel CLI. This is similar to deploy dynamic TDD in macro cells, we have not see the strong need to study dynamic subband location configuration with equal priority as semi-static subband configuration. For indoor deployment, we have not seen the strong need for this either. |
| MediaTek | Similar comment to what we provided in the response to Proposal 2-5. For the example we provided in our responses there, is that considered semi-static or dynamic subband location? |
| LG Electronics | The First step for study of SBFD is to make a basic functionality of SBFD. Dynamic configuration for SBFD could be a feature for performance enhancement. In this sense, we think dynamic should be second priority for study. |

  - Guard band between DL/UL subbands

Many companies see the potential need to have guard band between DL and UL subbands for filter rolloffs to mitigate the interference from DL subband to UL subband. It is expected that RAN4’s inputs would be required to decide the necessity and size of guard band.

If the guard band is confirmed to be needed, it needs to be discussed whether/how to inform UE the related information, e.g. size/location of guard band.

Explicit guard band configuration and scheduling-based guard band configuration are provided in [10] as shown in the following figure.

Figure 2-7: Guard band configurations: Explicit configuration vs Scheduling based [10]

Proposal 2-7:

Study whether/how to inform UE the guard band(s) between UL and DL subbands if any.

|  | Company |
| --- | --- |
| Support | Sony, IDC (Modified P2-7), QC, New H3C, NEC, Xiaomi, OPPO, Ericsson (with vivo's modification), Spreadtrum, CATT, Panasonic, Intel, ITRI, Lenovo, ETRI, ZTE, Samsung, CMCC, DOCOMO, WILUS, CEWiT, KDDI, Nokia, NSB, LG Electronics |
| Not support | MediaTek |

| Company | Comments |
| --- | --- |
| Sony | This proposal is related to Proposal 2-6 and we have the same comment, that is, we should firstly consider whether the subband is transparent or not to Rel-18 UE. If Rel-18 UE needs to know the locations of the subbands then it will also need to know where the guardbands are (if any). |
| InterDigital | This is an important design factor for SBFD, as in our contribution the initial LLS results showed the importance of the frequency gap between DL/UL subbands. But, “guard band” sounds like a fixed number of RBs that will be wasted in all cases (to mitigate negative CLI effects), which we believe is not the intention of the group. So, we should clarify that the negative CLI effects will not always exist, as they depend on traffic condition, beam-domain condition, and power-domain condition, etc. It means, in RAN1 spec point of view, the “guard band” should not be treated as a part of configurable component, but as a resulting consequence of a SBFD format (indicating the location of DL RBs and UL RBs) given to the UE. Thus, we can propose an update as follows: Modified Proposal 2-7: Study whether/how to inform UE the guard band(s) between UL and DL subbands if any. - Note the guard band(s) may not be a configuration parameter, but can be determined according to the indicated location of DL/UL subbands(RBs). |
| Sharp | Firstly, we need to define guard bands for further study. |
| QC | Open to study |
| New H3C | From our point of view, the guard band is needed. However, set the explicit guard band will introduce the signalling overhead in case of semi-static configuration. In the dynamic case, the guard band can be implemented by gNB’s scheduling |
| vivo | The study should include 1) the required guardband size 2) whether and how to make the guardband size known by the UE. Suggested below the updated proposal Study the required size of the guard band between UL and DL subbands and whether/how to inform UE the guard band(s). |
| OPPO | This topic could be a good example to seek RAN4 help, e.g., on adjacent sub-band interference ratio. |
| Ericsson | Support vivo's modification. |
| Intel | In our view, RAN1 needs to first determine whether and how large would the guard band be, and then, discuss whether/how to inform UE the guard band. vivo’s update looks good to us. |
| ZTE | Similar view as Sony, it would be good if companies can have some initial study on whether the subband configuration can be transparent to UE. |
| Samsung | We support the FL proposal. We see the need to provide the SBFD UL SB configuration to the UE as independent from the need to inform the UE about guard RBs/REs for purpose of Tx/Rx filtering. It is very desirable to provide the UE with the SBFD UL SB configuration. Newer release UEs can then be independently configured with the SBFD UL resources and are not constrained by SIB1 tdd-UL-DL-ConfigCommon for the legacy UEs, e.g., SBFD UL in slots configured as D for legacy UEs or possibility to set all SBFD slots to D for legacy UEs. |
| CMCC | Regarding the guard band size, input from RAN4 is needed. |
| DOCOMO | Support vivo’s revision. |
| CEWiT | We have a similar view with Sony. First, we need to decide whether subband is informed to UE or not. If yes, then guardband information also needs to be given. |
| Nokia, NSB | In our view, guard band configuration does not need to be distinguished from subband configuration. Resources that are not configured as part of the UL or DL subband are part of the guard band. gNB can control the guard band also by means of scheduling and/or configuration of radio resources. |
| Huawei, HiSilicon | Agree with Sony. Before we decide on this proposal, we should discuss whether subband is transparent for UE. |
| MediaTek | From RAN1 procedures perspective, it is not clear why the guard band need to be known by the UE! The following statement that is used as motivation for the proposal is not clear to us “If the guard band is confirmed to be needed, it needs to be discussed whether/how to inform UE the related information, e.g. size/location of guard band.” Why the UE needs to know about the guard band if it is needed from performance improvement perspective (to reduce the CLI or SI as an example)? Also, legacy UEs can’t be informed about such guard bands. We would like to see more solid reasoning on such need before agreeing on studying signaling mechanisms for guard bands. |
| LG Electronics | It could be studied whether to inform UE the guard band(s) between UL and DL subbands at this stage. |

- Indication of UL/DL resource partition

  - Legacy UE

    - 1st round discussion

In conventional TDD system, UL and DL are split in time domain. TDD slot format of a serving cell is semi-statically and/or dynamically indicated to the UE via SIB, dedicated RRC signaling, SFI etc. For SBFD operation, additional UL/DL partition in frequency domain is introduced and it is obvious that it is unknown to legacy UEs.

Companies share the same view that legacy NR UEs can be served in a cell operating SBFD without knowing the UL/DL resource partitioning in frequency domain. The following approaches were identified in companies’ contributions.

- Approach #1

One approach is to keep the common TDD UL-DL configuration as it is and UE does not expect an opposite transmission direction from that configured by the common TDD UL-DL configuration. An example is shown below where the symbols for SBFD operation from gNB’s perspective are still configured as DL symbols in TDD-UL-DL-ConfigCommon. From legacy UE’s perspective, UE does not know that some of the RBs in DL symbols are used for UL transmissions and does not expect any UL transmissions in DL symbols. gNB should not configure/schedule any UL transmissions in the DL symbols for legacy UEs.

Figure 3-8: time-frequency domain resource partitioning from gNB perspective [15]

- Approach #2

An alternative approach is to configure the symbols for SBFD operation as flexible in TDD-UL-DL-ConfigCommon. There are different variants of approach #2.

  - Approach #2-1

In this approach, different UE-specific TDD UL-DL configurations can be configured to different UEs as shown below. For a certain UE, only DL or UL transmission is expected in a symbol according to the cell-specific and UE-specific configurations. gNB should avoid configuring/scheduling DL transmissions in UL symbols or configuring/scheduling UL transmissions in DL symbols for a UE.

Note that support of UE specific RRC configuration of UL/DL assignment is subject to UE capability (i.e. FG 5-1a).

Figure 3-9: different UE specific TDD UL-DL configurations [5]

  - Approach #2-2

In this approach, dynamic SFI is used to dynamically indicate UL/DL assignment to a group of UEs. Note that support dynamic SFI is also subject to UE capability (i.e. FG 3-6).

  - Approach #2-3

In this approach, no additional indication of UL/DL assignment is provided and UE transmits or receives in the flexible symbol(s) based on gNB configuration/scheduling according to existing rules.

Proposal 3.1 (for conclusion):

Legacy UEs can be scheduled in UL-only symbols or in UL subband in UL or flexible symbols with existing RAN1 specification and UE behavior.

|  | Company |
| --- | --- |
| Support | Sony, IDC, Sharp, QC (w/ added DL-only slot), New H3C, NEC, Xiaomi, Spreadtrum, CATT, Panasonic, Intel (need clarification ), ITRI, Lenovo (prefer vivo’s wording), ETRI, ZTE, Samsung, DOCOMO, WILUS, CEWiT, KDDI, MediaTek (with modifications) |
| Not support | Ericsson (support with revision), CMCC (support Ericsson’s revision) , Nokia, NSB, LG Electronics (fine with modification from Ericsson) |

| Company | Comments |
| --- | --- |
| Sony | Legacy UE should be able to operate SBFD from the gNB’s prespective. The subbands are transparent to legacy UE. |
| InterDigital | This is the only way for legacy UEs, meaning legacy UE operations cannot be changed from existing specifications up to Rel-16. We support Approach #1 that Rel-18 SBFD should not touch TDD-UL-DL-ConfigCommon (within which legacy UEs are all aligned for the Tx direction with Rel-18 UEs). And, Rel-18 SBFD can be enhanced on TDD-UL-DL-ConfigDedicated. |
| Sharp | The proposal can be viewed differently from UE perspective and gNB perspective. This proposal is for gNB perspective, correct? If so, we prefer explicitly mentioning that “From gNB perspective”, on the top of the proposal. |
| QC | Legacy UEs can be scheduled in UL-only symbols or DL-only symbols or in UL subband in UL or flexible symbols with existing RAN1 specification and UE behavior |
| New H3C | Both approach #1 and approach #2 (2-1 and #2-2) should be considered. For Approach #2-1, it is aligned with TDD-UL-DL-ConfigDedicated, Approach #2-2 is aligned with SFI. |
| vivo | Legacy UEs cannot “see” the UL subband, and above proposal is for UL transmission, so we suggest following modification: The uplink transmission for legacy UEs can be scheduled/configured only in uplink or flexible symbols with existing RAN1 specification and UE behavior. |
| Xiaomi | If we understanding correctly, the intention of the proposal is to conclude the legacy UE behaviour on the RBs occupied by UL subband. We suggest following modification: From gNB perspective, legacy UEs can be scheduled on the RBs occupied by UL subband with existing RAN1 specification and UE behavior. We don’t need to mention where UL subband can be configured. |
| OPPO | It is a bit strong to claim the legacy UE “can be scheduled”, which seems to indicate no performance issue at all. We suggest to use following formulation: Proposal 3.1 (for conclusion, revision): Legacy UEs can operate with gNB-side SBFD resource allocation with existing RAN1 specification and UE behavior. |
| Ericsson | Same as our comment in Section 2.2.1: We think that it requires more discussion on how to handle 'F' symbols/slots. It is not clear that legacy UEs fully support or are fully tested to be able to operate with undetermined UL/DL direction (even though it is supported by the spec) for more than just a guard time. Most (if not all) deployments to date are static TDD where the UL/DL direction is known in advance. Hence as a first step, we should prioritize at least UL subband in a DL symbol, and then further discuss how 'F' symbols should be used. Proposal 3.1 (for conclusion): (modified) Legacy UEs can be scheduled UL in at least UL-only symbols or in UL subband in UL or flexible symbols with existing RAN1 specification and UE behavior. FFS: Further discuss whether or not legacy UEs fully support or are fully tested to operate with undetermined UL/DL direction, i.e., in flexible symbols |
| Spreadtrum | We support the conclusion. According to Approach#1, it can be applied for the UE with the basic UE capability, such as without FG5-1a, FG3-6 report. For the UE support FG5-1a, FG3-6, more resources in flexible can be used. Anyway, we agree that the legacy UE can work well in SBFD network. |
| Intel | We agree that all approaches provided by FL is workable for legacy UE, depending on UE capability. For proposal 3.1, it is unclear to us, - What is the difference between UL-only symbol and UL symbol? Does UL symbol actually contain DL and UL subband, and legacy UE is scheduled in UL subband in UL symbol? As we discussed in section 2.2.1, we don’t think configuring DL subband in UL symbol is a reasonable scenario for Rel-18 full duplex. Or, is it a typo and is meant to say “UL subband in DL or flexible symbols”? - Why legacy UE cannot work in DL-only symbol for DL reception? - Why legacy UE cannot work in DL subband in DL symbol for DL reception? DL symbol actually contains UL and DL subband, and legacy UE can be scheduled in DL subband in DL symbol. |
| Samsung | We agree with both observations that as far as legacy UE transmission and reception behavior is concerned, backwards compatibility of and possibility of SBFD operation is supported using existing RAN1 specifications and UE behavior, e.g., by configuring DDDSU or by configuring flexible symbols for the legacy UEs in the symbols/slots configured for UL transmissions using the SBFD UL SB when needed. |
| CMCC | Similar view as Ericsson, we support approach#1, we are not sure whether commercial UEs will encounter interoperation problems or not when the network changes the legacy cell specific TDD UL/DL configuration to a configuration with more flexible slots. |
| DOCOMO | Agree with the principle that the SBFD operation at gNB side is transparent to legacy UE. |
| KDDI | We share the similar view with vivo. Legacy UEs can be scheduled/configured only in uplink or flexible symbols with existing RAN1 specification and UE behavior. |
| Nokia, NSB | The proposal is confusing. The legacy UE can be scheduled in UL in UL-only symbols and in flexible symbols, and in DL in DL-only symbols and in flexible symbols. The question is rather whether we can assume all legacy UEs are able to operate with flexible symbols configured in TDD-UL-DL-ConfigCommonto, which is the main difference between approach #1 and approach #2. |
| Huawei, HiSilicon | From specification point of view, we agree with the observation but also agree with some companies that the operation on flexible symbols have not been fully tested. |
| MediaTek | We agree with the proposal with the following minor modifications: “Legacy UEs can be scheduled UL in UL-only symbols, or in UL subband in UL or DL in DL-only symbols, and UL or DL in flexible symbols with existing RAN1 specification and UE behavior.” |
| LG Electronics | At this stage, it should be studied whether UL subband within SBFD time resource can be allowed for legacy UE. |

    - 2nd round discussion

The proposal 3.1 was intended to cover UL transmission only. Sorry for the confusion. Now the proposal is updated.

Updated2 Proposal 3.1 (for conclusion):

Existing RAN1 specification supports to schedule legacy UEs can be scheduled to transmit UL in UL-only symbols or in UL subband in UL or flexible symbols with SBFD operation at gNB side and to receive DL in DL-only symbols or in DL subband in symbols with SBFD operation at gNB side with existing RAN1 specification and UE behavior.

|  | Company |
| --- | --- |
| Support | New H3C, DOCOMO, NEC, vivo, Panasonic, Xiaomi, CATT, Sony, Spreadtrum, CMCC, Lenovo, QC, SK Telecom, Samsung (only with modified wording), ITRI |
| Not support | Intel (need to clarify the difference between UL-only and UL symbol), OPPO (the proposed conclusion intends to introduce DL subband, which is new feature not in any other proposals for agreement) , Sharp, Nokia, NSB |

| Company | Comments |
| --- | --- |
| NEC | Fine with the proposal in general. Considering the legacy UE doesn’t know the UL sub-band location, we didn’t see the difference between UL-only symbols or UL symbols with UL subband from UE perspective. We suggest following update for clarification. Updated Proposal 3.1 (for conclusion): Legacy UEs can be scheduled to transmit UL in UL-only symbols or in UL subband in UL or flexible symbols with existing RAN1 specification and UE behavior. |
| Intel | It is quite confusing, what is UL-only symbols and UL subband in UL. In our understanding, from legacy UE’s perspective, UE can only be scheduled to transmit UL in UL symbol or flexible symbol (the UE does not know there is a UL subband or full UL). From gNB’s perspective, legacy UE can be scheduled to transmit UL in UL symbol or in UL subband in SBFD symbol. Therefore, we suggest updating the proposal as below: From a gNB’s perspective, legacy UEs can be scheduled to transmit UL in UL-only symbols or in UL subband in UL or flexible symbols with SBFD operation with existing RAN1 specification and UE behavior. Besides, we’d like to clarify, the proposal does not imply gNB can configure SBFD operation in legacy UL symbol. The proposal only says gNB can have an symbol with SBFD operation (that may be limited to DL and flexible symbols only, but not the focus of this proposal) in which it can schedule UL. |
| Moderator | The proposal is updated based on the comments. Given that some companies commented that it is not clear whether legacy UEs fully support or are fully tested to be able to operate with undetermined UL/DL direction (even though it is supported by the spec) for more than just a guard time, the proposal is now reworded in RAN1 specification point of view. In addition, DL reception is added. |
| Xiaomi | We are generally fine with the proposal. The intention of the proposal is to claim legacy UE behaviour is not impacted. Hence we are OK with the updated proposal. |
| Sony | We support the proposal but with minor editorial below (somehow it reads funny saying “Existing RAN1 specification supports to schedule…”): Existing RAN1 specification supports the gNB to schedule legacy UEs can be scheduled to transmit UL in UL-only symbols or in UL subband in UL or flexible symbols with SBFD operation at gNB side and to receive DL in DL-only symbols or in DL subband in symbols with SBFD operation at gNB side with existing RAN1 specification and UE behavior. |
| CMCC | We are fine to clarify this proposal is in RAN1 specification point of view, but whether legacy UEs fully support or are fully tested to be able to operate with undetermined UL/DL direction still needs to be studied which is a basial question and will impact on the DL/UL resource partition signalling design. |
| Ericsson | Share the same view as CMCC. If the intention is to clarify that the spec supports legacy UE operation with 'flexible' symbols, then it should be stated in the same proposal that the "reality" in terms of commercial UEs could be different, and this will affect RAN1 design. Hence, we would recommend the following update Existing RAN1 specification supports the gNB to schedule legacy UEs can be scheduled to transmit UL in UL-only symbols or in UL subband in UL or flexible symbols with SBFD operation at gNB side and to receive DL in DL-only symbols or in DL subband in symbols with SBFD operation at gNB side with existing RAN1 specification and UE behavior. Further study whether or not legacy UEs fully support UL scheduling in symbols with undetermined UL/DL direction (i.e., flexible symbols) |
| OPPO | This conclusion seems to assume there would be a concept of DL subband, which has never been shown up in earlier proposals. In other words, the proposal for conclusion potentially introduces new spec features. The true intention is not clear to us between “proposal for agreement” vs. “proposal for conclusion”. |
| Sharp | It is clear that the legacy UE operates with existing TDD configuration (i.e., tdd-UL-DL-ConfigurationCommon, tdd-UL-DLConfigurationDedicated, and dynamic SFI), and SBFD configuration is totally invisible from the legacy UE perspective. We shouldn’t agree on something which may impact on the legacy UE behaviour. If no change on the legacy UE behaviour is intended by this proposal, we don’t think it’s necessary to agree on this proposal. |
| QC | Part of feasibility study is to make sure that legacy UE scheduling and performance is not affected by SBFD operation at the gNB. |
| Samsung | We share the same view as CMCC and Ericsson that we cannot currently conclude whether UE behavior using existing specifications is “always” and “under all circumstances” sufficient, e.g., for F symbols/slots. This aspect is too premature to conclude and will require more study in the R18 SI. The wording proposed by Ericsson is better for us, e.g., at least for D and U symbols/slots, things are clear, and for F symbols/slots, there should be an FFS. |
| Nokia, NSB | Still confusing. Of course, legacy UEs can only transmit UL in UL-only symbols or in flexible symbols. But why “in UL subband in UL or flexible symbols”? First of all, it does not make sense to speak of UL subband in UL symbols. Also, even with SBFD enabled, the gNB may decide to configure a flexible symbol as an UL symbol, in which case a legacy UE should not be restricted to transmit in UL subband in flexible symbols. Similar issue for legacy UE to receive DL. |
| Huawei, HiSilicon | Fine with the updated proposal on the reflector. |

    - 3rd round discussion

Updated Proposal 3-1a:

Existing RAN1 specification supports gNB to schedule legacy UEs to transmit UL in UL-only symbols or in a time-frequency resource intended for UL subband in symbols with SBFD operation at gNB side and to receive DL in DL-only symbols or in a time-frequency resource intended for DL subband in symbols with SBFD operation at gNB side.

- Further study whether or notThat the legacy UEs may not fully support UL transmission and DL reception in symbols with undetermined UL/DL direction (i.e., flexible symbols) should be taken into account.

|  | Company |
| --- | --- |
| Support | Spreadtrum, Lenovo, New H3C, DOCOMO, vivo, CEWiT, Panasonic, CATT, Fujitsu, Sony, Nokia/NSB (clarification or update needed) , LG Electronics, Samsung (different proposed wording), Ericsson,, ITRI |
| Not support | Intel (Fine with main bullet, but we don’t support the sub-bullet), Sharp, , OPPO (the proposal should be an observation) |

| Company | Comments |
| --- | --- |
| Lenovo | Further suggested updates are: Updated Proposal 3-1a: Existing RAN1 specification supports gNB to schedule legacy UEs to transmit UL in UL-only symbols or in a time-frequency resource intended for UL subband in symbols with SBFD operation at gNB side and to receive DL in DL-only symbols or in a time-frequency resource intended for DL subband in symbols with SBFD operation at gNB side. |
| New H3C | We are fine with Lenovo’s further modification. |
| ZTE | We agree with the intention of this proposal. However, the wording may be confusing, especially if “at gNB side” is deleted. Thus, we prefer to keep “at gNB side”. |
| vivo | Also fine with Lenovo’s further modification. |
| CEWiT | We are fine with the main bullet. However, for the sub-bullet, if the UL BWP of the legacy UE extends beyond the UL sub-band (into the DL sub-band), it will still perform the UL transmission over the full BWP. So, it is not clear to us why the sub-bullet is being considered. |
| WILUS | Since legacy UE is not configured with SBFD, there is no impact on legacy UE. In other words, gNB may not schedule for legacy UE to transmit/receive UL/DL in UL/DL subband in legacy DL/UL symbols. For the flexible symbols, even legacy UE who is not configured with SBFD, can be indicated to transmit/receive according to the UE bahavior in the current RAN1 specification. |
| Huawei, HiSilicon | As commented on the email reflector, we agree with intention of the proposal but the current formulation is not accurate. It may be misunderstood that gNB can schedule legacy UEs to transmit UL in a time-frequency resource intended for UL subband at gNB side even when the time-frequency resource is configured as DL for the legacy UE. However, I would assume this is not the intention. |
| NEC | Fine with the intention of the proposal. It is not very clear to us that the proposal is to clarify the existing RAN1 specification or to make a new proposal for the impact of SBFD operation on legacy UE? Also, “time-frequency resource intended for UL subband” seems to imply SBFD operation within legacy UL slots/symbols which we are yet to agree on, so we prefer removing “to transmit UL in a time-frequency resource intended for UL subband at gNB side and” and put this as FFS |
| Sony | The formulation of the proposal in 2nd round actually is better. I think maybe it is best to spell out the whole thing, to avoid any confusion i.e.: Proposal 3-1 (Conclusion) Existing RAN1 specification supports gNB implementation of SBFD that is transparent to legacy UE: - gNB can schedule UL transmission in an UL subband of an OFDM symbol operating with SBFD to a legacy UE, where that OFDM symbol is configured or indicated as UL for that legacy UE - gNB can schedule DL transmission in a DL subband of an OFDM symbol operating with SBFD to a legacy UE, where that OFDM symbol is configured or indicated as DL for that legacy UE. |
| Intel | We don’t support the sub-bullet. In our understanding, it is mandatory for all UEs since Rel-15 to be able to handle “flexible” symbols by semi-static configurations. Please check UE feature 5-1, component 7 as shown below. UE supports UL transmission or DL reception without cell-specific TDD config (all symbols are treated as flexible symbol when gNB does not provide cell-specific TDD config). Therefore, there is no need to consider any special handling for SBFD to ensure UEs not supporting mandatory features to work properly! 5. Scheduling/HARQ operation; 5-1; Basic scheduling/HARQ operation; 7) Dynamic UL/DL determination based on L1 scheduling DCI with/without cell specific RRC configured UL/DL assignment; Yes; N.A.; N.A.; N.A.; RAN1; Mandatory without capability signaling; Mandatory without capability signaling. |
| Xiaomi | Similar views as Huawei/HiSilicon. |
| Sharp | Sub-bullet should be “further study” until it’s clear that what functionalities in legacy specification have not been tested in real deployment. |
| OPPO | We suppose this proposal is an observation. We think the comment from HW could be a over-concern, since every thing in the proposed text belongs to “Existing RAN1 specification supports...”. However, if the proposal is not an observation, HW’s comment is valid. |
| Nokia, NSB | Support if this means that “existing RAN1 specifications supports the gNB to schedule a legacy UE to transmit in UL in a slot/symbol with both UL and DL subbands, and using frequency resources contained in the UL subband”, and vice versa (i.e. “existing RAN1 specifications supports the gNB to schedule a legacy UE to receive in DL in a slot/symbol with both UL and DL subbands, and using frequency resources contained in the DL subband”. |
| LG Electronics | We are fine with main bullet of proposal. |
| Samsung | We are fine with the main bullet. For the sub-bullet, we can remove “undetermined UL/DL direction” and directly use flexible symbols. It is because there are only three type of symbols, DL, UL and flexible in RAN1 specification. No confusion is possible. We are not sure what happened to the sub-bullet. The proposed wording by Ericsson Round 2 comment may be clearer. We propose to use, “Further study whether or not legacy UEs fully support UL scheduling in symbols with undetermined UL/DL direction, e.g., symbols configured as F. One clarification question is: how does gNB know if the legacy UE fully supports UL transmission and DL transmission in the flexible symbols (based on the observed inconsistent UE implementation behavior in practice)? |
| QC | We think the proposal should be simplified to study legacy UE operation in SBFD deployment. Study how existing RAN1 specification supports legacy UEs operation in SBFD deployment. |
| Ericsson | Support the proposal. We prefer to keep the sub-bullet. @Intel. Just because a feature is mandatory, doesn't mean that it works. There have been notable examples in the past of mandatory features that have been activated in an operator's network where interoperability problems occurred. Most (if not all) commercial deployments of TDD so far are based on semi-static configuration of a TDD pattern. It seems like a risky path to design an SBFD solution that relies on legacy UEs to operate properly with all 'flexible' symbols. |

    - 4th round discussion

Proposal 3-1b:

Existing RAN1 specification supports gNB to schedule legacy UEs to transmit UL in a time-frequency resource intended for UL subband at gNB side and to receive DL in a time-frequency resource intended for DL subband at gNB side.

- A legacy UE is not expected to transmit UL in a DL symbol from UE perspective or to receive DL in an UL symbol from UE perspective according to existing RAN1 specification.

- Further study whether or not That the legacy UEs may not fully support UL transmission and DL reception in symbols with undetermined UL/DL direction (i.e., flexible symbols) should be taken into account.

|  | Company |
| --- | --- |
| Support | Spreadtrum, vivo (fine in principle), ITRI, New H3C, NEC (not sure about 2nd bullet), Sony, Lenovo, Ericsson, Samsung, DOCOMO, CMCC, Xiaomi, Panasonic, MediaTek, Fujitsu, CATT, LG Electronics, KDDI |
| Not support | Intel (not support 2nd bullet), WILUS |

| Company | Comments |
| --- | --- |
| Spreadtrum | For the second subbullet, legacy UE only do whatever gNB configure/schedule it to do. For dynamic UL transmission or DL reception, they have the highest priority among all the DL/UL derections, and gNB have no problem to avoid the potential problem for legacy UE. According to higher layer configured UL transmission or DL reception, we admit that there are may be some problems for perfectly remain the current UE behavious in flexible symbols. So we support it. |
| vivo | Support in principle. The 1st subbullet is not needed given the main bullet already says “Existing RAN1 specification”. For the 2nd subbullet, it is not sure and clear what actions we need to do and how we can do for studying the legacy UE behabvior. We prefer to modify the 2nd subbullet as following: The SBFD design should consider that the legacy UEs may not fully support UL transmission and DL reception in symbols with undetermined UL/DL direction (i.e., flexible symbols) in the existing commercial deployments. |
| New H3C | In general we are fine with this proposal. For 2nd subbullet, I wonder whether it already is enough for legacy UEs to keep behaviors of rel-17 in flexible symbols |
| NEC | Similar to Vivo, we are not sure what is the intention of the 2nd bullet and what is required to be studied for inconsistent legacy UE behavior. |
| Huawei, HiSilicon | Fine with the proposal in general. |
| Nokia, NSB | We also think it is better to simply mention Study how to schedule legacy UE in symbols with SBFD operation. |
| Sony | We support both sub-bullets. There was some confusion on the main bullet before, which may suggest legacy UE transmitting UL in DL symbols. The 1st sub-bullet in this new proposal clarified that. |
| Ericsson | Support Proposal 3-1b. We think vivo's suggested change to the 2nd sub-bullet is good too – it adds more clarity on what RAN1 should do: "The SBFD design should consider that the legacy UEs may not fully support UL transmission and DL reception in symbols with undetermined UL/DL direction (i.e., flexible symbols) in the existing commercial deployments." Regarding the 1st sub-bullet, we are okay with it, but it seems rather obvious. |
| Samsung | We are ok with the FL proposed wording in 3-1b. Regarding the second note, “fully support” is quite unclear, which a function in RAN1 specification is not supported in the legacy UE? As indicated by Intel in the last round comment, dynamic UL/DL determination is mandatory UE feature without signaling. |
| CMCC | We are fine with this proposal. |
| ZTE | Similar view as other companies, the second bullet is not very clear to us. |
| Xiaomi | We are fine with the proposal. For the second bullet, we prefer vivo’s version. However, we also think the ‘fully support UL transmission and DL reception in symbols with undetermined UL/DL direction’ is confusing. At least broadcast channel(e.g. SSB, Type0-CSS) and semi-configured UL/DL can be transmitted on the flexible symbols. We are not sure how and should be considered in the SBFD design. |
| QC | As commend in earlier round, we think the proposal should be simplified to study legacy UE operation in SBFD deployment. Study how existing RAN1 specification supports legacy UEs operation in SBFD deployment. |
| Intel | We don’t support 2nd sub-bullet. As we explained in previous rounds, there is no need to consider any special handling for SBFD to ensure UEs not supporting mandatory features to work properly! For the wording, what does ‘symbols with undetermined UL/DL direction’ mean? As we commented in 2nd round, if there is SSB in a flexible symbol, is it undetermined UL/DL direction or determined DL direction? If the intension is Flexible symbol without considering of any signals (e.g., SSB) which can be used to determine the UL/DL direction, let’s just say flexible symbol, rather than ‘symbols with undetermined UL/DL direction’. If companies really want to study whether a UE support flexible symbol, can anyone explain the procedure for such study? According to some comments, it seems some companies are unsure whether legacy UE can support flexible symbol, because this mandatory feature is not fully tested, thus companies suggest SBFD design should consider this possibility. If we follow this logic, it seems we should go through ALL mandatory features to check whether it is tested or not, and SBFD design should consider all the mandatory features which are not fully tested yet, rather than only naming one feature. Furthermore, we’re not sure which group should take responsibility to check and evaluate if UEs in the field are supporting mandatory requirements, but we believe it is not RAN1’s scope. RAN1 should wait for the response from the group which is responsible for this task, and then, study the impact, if necessary. It does not make sense that RAN1 now assumes UEs in the field cannot support this mandatory requirement and then study SBFD operation based on this unverified assumption. Moreover, if a UE can not support flexible symbol, do you think the UE can work under dynamic TDD? If no, why not suggest to study this for AI 9.3.3 too ? |
| OPPO | The last bullet is also not clear to us. Is the intention to identify the assumptions for gNB behaviors which can make the legacy UE not to see the symbols where its Tx/Rx is not fully supported? |
| WILUS | We share the similar view with Intel. For the flexible symbols, legacy UE can be indicated to transmit/receive according to the UE bahavior in the current RAN1 specification. |
| Panasonic | We support the proposal. |
| MediaTek | Fine with the proposal. |
| LG Electronics | Fine with the proposal |
| Ericsson | It appears as though some companies are confident that legacy UEs are fully functional under the following two scenarios and face no inter-operability issues in commercial networks - UE is provided with tdd-UL-DL-ConfigurationCommon in which some/all symbols are 'flexible' - UE is neither provided with tdd-UL-DL-ConfigurationCommon nor tdd-UL-DL-ConfigurationDedicated We think that that designing a working SBFD system under this assumption is risky for ensuring legacy UEs operate properly. However, if companies are so sure, then the correct path forward is to make the following working assumption (or conclusion) for the study item: Working assumption: For the purposes of the Rel-18 SI on full duplex, it is assumed that legacy UEs are fully functional and and face no inter-operability issues in commercial networks in both of the following scenarios: - UE is provided with tdd-UL-DL-ConfigurationCommon in which some or all symbols are 'flexible' - UE is neither provided with tdd-UL-DL-ConfigurationCommon nor tdd-UL-DL-ConfigurationDedicated If companies are not comfortable making this assumption, then the simple work around is to assume that symbols used for SBFD operation are symbols configured as 'D' by tdd-UL-DL-ConfigurationCommon, and not 'F' symbols. |

  - SBFD capable UE

Both approach #1 and approach #2 for legacy UE operation in section 3.1 are “UE transparent” approaches. UE is not aware of resource partitioning between UL and DL in frequency domain (i.e. subband location).

To adopt approach #1 for all UEs including SBFD capable UEs is not appropriate since UL subband cannot be used by any UE in that case. Approach #2 can be used for SBFD capable UEs so that the SBFD is operated in a UE transparent way for all the UEs. It is proposed in [12] that the subband allocation is transparent to UE and the study should not base on a prerequisite for UE to have the knowledge of simultaneous DL/UL allocation. However, many other companies think that there are potential restrictions and complexity at gNB side and therefore proposed to study the “UE non-transparent” approach. Some other companies directly discussed how to inform subband location to SBFD capable UEs. On a high-level, the following schemes were proposed by companies.

- Scheme #1: RB-set based SBFD

In this scheme, a subband includes a set of consecutive RBs within a BWP as shown in the upper figure in Figure   3 -10 and there may be both UL and DL resources in a same symbol within a BWP. The frequency location of the subband is indicated to the UE. The time location of the subband can be predefined (e.g. in all DL symbols [15]) or indicated to the UE.

Figure 3-10: SBFD operation [10]

- Scheme #2: SUL based SBFD

Samsung [10] discussed a possible scheme that UL subband is configured by means of SUL configuration as shown in the lower figure in Figure   3 -10.

- Scheme #3: BWP based SBFD

BWP based SBFD operation where each subband is defined as one BWP were discussed in [3][11][16][21][28]. In this case, there would be no UL and DL resources in the same symbol within a BWP.

Different frame structures can be provided for different BWPs as illustrated in Figure   3 -11, which is not supported in current specification. In addition, enhancements to reduce BWP switching delay were proposed by several companies.

Figure 3-11: BWP-based SBFD operation [3]

- Scheme #4: CA based SBFD

ZTE [3] and Samsung [10] discussed intra-band CA based SBFD where the frequency resources within a carrier are configured as different CCs to the UE.

According to the proposal in [3], different subbands are regarded as different cells and directional collision handling mechanism defined in Rel-16 half duplex CA can be applied to support CA-based SBFD operation.

Figure 3-12: Half duplex CA based SBFD [3]

The approach to use intra-band CA for SBFD in [10] is shown in the figure below, where either 2 or 3 CCs need to be configured for the UE in the aggregated BS channel BW.

Figure 3-13: SBFD using intra-band CA [10]

Proposal 3.2:

Study whether/how to inform UE time and/or frequency location of subband.

|  | Company |
| --- | --- |
| Support | Sony, IDC (Scheme#1 and #3), Sharp, QC, New H3C, NEC, Xiaomi, OPPO, Ericsson, Spreadtrum, CATT, Panasonic, Intel, ITRI, Lenovo, ETRI, ZTE, Samsung, CMCC, DOCOMO, WILUS, CEWiT, KDDI, Nokia, NSB, MediaTek, LG Electronics |
| Not support |  |

| Company | Comments |
| --- | --- |
| Sony | CA should not be a prerequisite to support SBFD, i.e. the UE should be able to support SBFD without needing to also support CA. |
| InterDigital | Agree with Sony that SBFD should be able to be supported within one CC. We support to study Scheme #1: RB-set based SBFD and Scheme #3: BWP based SBFD. |
| New H3C | We slightly prefer Scheme#1 RB-Set based SBFD and CA-based SBFD |
| vivo | We would like to clarify the relation or difference between this proposal and proposal 2-6. |
| Xiaomi | Agree with Sony. |
| OPPO | From our view, the following two issues are separate: - Whether a Rel-18 UE should observe a DL subband and a UL subband at the same time and in a same “BWP/cell”, regardless whether the DL or UL subband is actually used by the UE. - Whether a Rel-18 UE should know the SBFD assignments (at least for a given time instance and a given transmission direction). We think the first question should have answer NO, otherwise there would be too many spec changes. We are open to the second question, although we do not see strong reason for a YES. We do not think a UE-capability argument can dismiss CA-based SBFD in the SI phase. |
| Spreadtrum | We think it is the first issue to study. Our first preference is transparent SBFD, Scheme #3: BWP based SBFD is a suitable scheme, without many changes, including DL/UL collision handling, new resource mapping, etc. So at least it can be in the scope. We are open for non-transparent SBFD, although we do not found enough benefits until now. |
| Intel | In our view, we first need to study and agree whether non-transparent indication of SBFD configuration to UE can be considered. As such, both transparent and non-transparent indications for SBFD configurations should be considered and captured in the TR as part of the study. |
| ZTE | Some initial discussion on transparent and non-transparent is beneficial. |
| Samsung | See our comments on FL proposal 2-6 |
| CEWiT | We prefer scheme #1 and scheme #3. |
| KDDI | We prefer to study Scheme#3: BWP based SBFD because the smaller BWP operation can mitigate CLI impact by lower sampling rate than the BWP for the entire carrier. |
| Nokia, NSB | RAN1 should first agree on the need to inform SBFD capable UEs of the UL/DL time and frequency resource partitioning before discussing whether e.g. RB set based or BWP based scheme is to be used. This may also depend on e.g. what can be assumed at the UE in terms of adapting the digital filter bandwidth to the corresponding UL/DL subband. |
| Huawei, HiSilicon | Fine to study this further but it needs to be decided at some point based on whether this information will be beneficial for UE operation or co-existence among operators. |

Question 3.1:

Which scheme(s) do you support to further study for subband time-frequency location indication?

|  | Company |
| --- | --- |
| Scheme #1: RB-set based | Sony, IDC, Sharp, QC, New H3C, vivo, NEC, Xiaomi, Ericsson, Spreadtrum, CATT, Panasonic, ITRI, Lenovo, ETRI, ZTE, CMCC, DOCOMO, WILUS, CEWiT, KDDI, Nokia, NSB, LG Electronics |
| Scheme #2: SUL based | Sharp |
| Scheme #3: BWP based | Sony, IDC, Sharp, vivo, NEC, Xiaomi, OPPO, Spreadtrum, Panasonic, ITRI, Lenovo, ZTE, CMCC, CEWiT, KDDI, Nokia, NSB |
| Scheme #4: CA based | New H3C, NEC, OPPO, ZTE, Nokia, NSB |

| Company | Comments |
| --- | --- |
| Sony | Do we need to restrict to only 4 indications at this point? |
| Sharp | As commented in response to Question 2-1, we think Scheme#4 is should be excluded since CA-based solution losses system capacity by unnecessary guard bands. |
| QC | Consider scheme #1 as baseline, open to further study other schemes (e.g., scheme #3 and #4) considering pros/cons to UE implementation, timeline and current UE RF requirements. |
| NEC | We support that study of Scheme #1,#3,#4 needs to be studied. Advantages of Scheme#2 are not clear. |
| OPPO | We prefer #3 and #4, and open to #1. We have a concern on #2, which connects the full-duplex to a specific carrier type, i.e., open a SUL carrier (from RAN1 perspective) in a non-SUL carrier (from RAN4 perspective). This may cause bigger impacts in a higher level than in resource allocation scope. |
| Ericsson | Agree with Sony's comment in Proposal 3.2 – CA should not be a pre-requisite for SBFD operation, especially since it will require UL CA between 3 carriers, and it is not clear that this is well supported amongst UEs. BWP-based SBFD will most likely require BWP switching between SBFD symbols and UL-only symbols. We think RB set-based SBFD is simplest. The BWP can span multiple RB sets (may need some small spec updates to make it clear that the UE is not expected to receive UL and DL simultaneously within a BWP). With such an update, the gNB would ensure that only DL or UL is scheduled in an SBFD symbol for a given UE, but not both simultaneously. |
| Spreadtrum | Our first choice is Scheme #3: BWP based, it can be applied without a little changes. We are open for Scheme#1. And if the scheme can be directly extended to scheme#4, it would be OK, but we don’t think it is a good idea to study it a lot at early SI stage. |
| Intel | We think we don’t need to down-select in first meeting. We suggest to discuss all schemes (not only limited to 4 schemes above) according to the categories as follows: Type #1: Within a BWP, UE may see DL and UL subband within a symbol （RB-set based is just one possible solution for scheme#1, there can be other solutions.） Type #2: Within a carrier, UE may see both DL and UL subband within a symbol, but within a BWP, UE only sees one direction (DL or UL) Type#3: Among multiple carries, UE may see DL and UL subband within a symbol, but within a carrier, UE only sees one direction (DL or UL) - #3-1: SUL and non-SUL - #3-2: CA For #3-1, it is not clear to us whether it can work, if DL subband can be configured in UL symbol. |
| Lenovo | It should be possible to support SBFD without CA. |
| Samsung | Among the 4 schemes, we slightly prefer the RB-set based scheme #1 (but see our comments on FL proposals 2-1 and 2-2). We are open to all possible approaches at this stage. Their potential pros/cons and RAN1 specification impacts should be studied in this SI. |
| CMCC | Scheme#1 is the baseline and scheme#3 can be studied, but for scheme#3, companies should study whether to reduce/relax the BWP switching delay, otherwise, there will be much performance loss due to the BWP switching delay among different BWPs. |
| DOCOMO | We prefer scheme #1. BWP switching delay is an issue for Scheme #3. Regarding scheme #2 and #4, we think it is possible to support SBFD for a UE not capable of SUL or CA. |
| Nokia, NSB | Consider scheme #1 as baseline, study scheme #3 in relation to UE capability to perform fast BWP switch and/or adaptation of digital filter bandwidth. Scheme #4 may also be studied (this also relates to the discussion under Proposal 2-1). |
| Huawei, HiSilicon | All schemes should be studied. As mentioned earlier, for scheme #4, we think there may also be some merit. Firstly, in the case of CA, the UL/DL subband configuration can be obtained, i.e., a legacy UL-DL TDD configuration is configured in each carrier. Secondly, the legacy collision handling mechanism can be used in each carrier as much as possible. |
| MediaTek | We don’t see a strong need for down-selection or periodization at this stage. All schemes can be considered at the start. |

- Potential enhancements for SBFD

  - UE collision handling between DL and UL

    - 1st round discussion

Several companies proposed to study potential enhancements to UE behavior for collision handling between DL reception/UL transmission [2][6][7][15][18][19][21][30][32] for SBFD operation, e.g. collision handling of UL transmission in DL or SSB symbols, collision handling of configured UL transmission and DL receptions in the same symbol, collision of DL reception in PRACH symbols etc.

Proposal 4.1:

Study potential enhancements for UE collision handling between UL and DL.

|  | Company |
| --- | --- |
| Support | Sony, IDC, Sharp, QC, New H3C, NEC, Xiaomi, Ericsson (support in principle, but clarification required), Spreadtrum, CATT, Panasonic, Intel, ITRI, Lenovo, ETRI, ZTE, Samsung, CMCC, DOCOMO, CEWiT, Nokia, NSB, LG Electronics |
| Not support |  |

| Company | Comments |
| --- | --- |
| QC | Current 3GPP specification should be relaxed first to allow UL/DL multiplexing in same slot. |
| vivo | We would like to clarify following: - The UE should be Rel-18 SBFD capable UE. - The collision means the collision between the UL transmission in UL subband and DL reception in the DL subband in the same symbol in the same carrier, correct? |
| OPPO | In our view, the proposal, if agreed, does not necessarily mean the collision would happen for sure. So the following proposal modification is preferred. Proposal 4.1 (revision): Study potential enhancements for UE collision handling between UL and DL. - The study can include potential enhancements to avoid UE-side collision between UL and DL. - Note: This does not mean the UE-side collision between UL and DL is not avoidable for all UEs (e.g., based on existing specification). |
| Ericsson | Agree with QC's comment. We have the same clarification questions as vivo. |
| Spreadtrum | If non-transparent SBFD is applied, new DL/UL collisions may appear, we are open to discuss. |
| Samsung | We are open to considering the benefits (and potential drawbacks) of enhanced DL reception/UL transmission collisions rules for UEs with enhanced SBFD support in later releases. However, existing UE behavior with respect to the R16 HD CA collision handling rules should be baseline and first captured as such by agreement. |
| DOCOMO | We share same view as vivo’s first question. Regarding vivo’s second question, we think collision of UL transmission and DL subband (and DL reception and UL subband if DL subband on UL symbol is also supported) should also be in the scope. |
| Nokia, NSB | Without enhancements it may only be possible to operate with semi-static configuration of SBFD slots/symbols. |
| MediaTek | Same clarification questions as vivo. It is essential to distinguish between legacy UEs and SBFD UEs in the proposals/discussions. |

    - 2nd round discussion

Several companies proposed to clarify that UE in proposal 4.1 is SBFD capable UE.

The intention of the proposal is to be generic. The collision handling between UL and DL does not only include collision handling of both UL transmission and DL receptions in the same symbol, but also includes e.g. collision handling of UL transmission in DL or SSB symbols etc.

Updated Proposal 4.1:

Study potential enhancements for UE collision handling between UL and DL for SBFD capable UE.

|  | Company |
| --- | --- |
| Support | New H3C, DOCOMO, NEC, vivo, Intel, Panasonic, Xiaomi, CATT, Sony, CMCC, ZTE, Lenovo, Ericsson, Sharp,QC, SK Telecom, Samsung, ITRI, Nokia, NSB, IDC |
| Not support | OPPO |

| Company | Comments |
| --- | --- |
| Lenovo | SBFD is done by gNB, and Rel-18 UE may be aware of gNB SBFD operation. Thus, we propose the following modification: Updated Proposal 4.1: Study potential enhancements for UE collision handling between UL and DL for SBFD capable aware UE. |
| Ericsson | Support Updated Proposal 4.1 |
| OPPO | Updated proposal 2-5 says “whether the location and bandwidth of a subband are informed to UE is separately discussed” . So before RAN1 determines that the UE would be informed of UL transmission in a UE-awared DL symbol, it is premature to assume the UE would know there is a collision and perform the corresponding handling. We can accept the following: Updated Proposal 4.1: Study potential enhancements for UE collision handling between UL and DL for SBFD capable UE, if it is supported for UE to be made aware of such collision. |
| QC | Support as SBFD is transparent to legacy UE. |
| Samsung | We support the updated proposal 4.1 |
| Huawei, HiSilicon | Fine with the proposal |
| InterDigital | Support the proposal |

    - 3rd round discussion

Proposal 4-1:

Study potential enhancements for UE collision handling between UL and DL at UE side.

|  | Company |
| --- | --- |
| Support | Lenovo, New H3C, DOCOMO, ZTE, CEWiT, WILUS, NEC, Panasonic, CATT, Fujitsu, Sony, SK Telecom，Intel, Sharp, Nokia, NSB, LG Electronics, Samsung, QC, Ericsson (with modifications) |
| Not support | OPPO |

| Company | Comments |
| --- | --- |
| Spreadtrum | It is too urgent to discuss the collision handling at UE side. We can go back to this topic after transparent/non-transparent SBFD scheme have a clearer picture. Or the proposal can limit to non-transparent SBFD if it is supported, such as: Study potential enhancements for collision handling between UL and DL at UE side for non-transparent SBFD if it is agreed. |
| New H3C | We are fine with this proposal in general. |
| vivo | We prefer original Updated Proposal 4.1 for UEs supporting SBFD operation. If companies have concern on the wording of SBFD capable UE, we can use the wording proposed by Lenovo, that Study potential enhancements for collision handling between UL and DL for SBFD aware UE. |
| Huawei, HiSilicon | Support |
| NEC | We think that 2nd round proposal was more accurate as this seems to indicate some impact on legacy UEs as well. It is better to include a note like “This proposal does not imply any impact on legacy UEs” |
| SK Telecom | We are fine with this proposal in general. |
| Xiaomi | We prefer vivo’s version. Just mention ‘at UE side’ may not be sufficient as gNB may also take actions for the collision. |
| OPPO | Same comments as from Spreadtrum. Further, even if the SBFD is non-transparent to UE, whether there is a collision between DL and UL may still depend on what UL subband means for a UE when it is informed to UE: - if UL subband means the Tx/Rx prioritization (UL Tx is prioritized over DL Rx), there is no collision at all to handle. - if UL subband means just a resource planning but not Tx/Rx prioritization with real Tx or Rx, the collision handling is needed. |
| Nokia, NSB | Tend to agree with Spreadtrum and vivo that this only makes sense for SBFD aware UEs in case of non/transparent SBFD. |
| LG Electronics | We are fine with the proposal. |
| Samsung | We are generally fine with the proposal. However, we consider that relying on gNB scheduling is a basic solution to avoid such a collision. After the principles of SBFD configuration and the scheduling of UEs during SBFD operation are clearer, we should then look at the UE collision handling and corresponding UE transmission/reception behavior. |
| QC | Support. However, we think that legacy restriction should relaxed first. Also, this proposal is for Rel-19+ UE that are aware of SBFD and not for legacy UE. Study potential enhancements for UE collision handling between UL and DL at UE side with assumption of relaxed restriction to allow UL/DL multiplexing in same slot in different subbands. |
| MediaTek | No strong objection, but this seems more like WI discussion. |
| Ericsson | Prefer to keep the wording "for SBFD capable UEs" or "SBFD aware UEs" |

    - 4th round discussion

Proposal 4-1a:

Study potential enhancements for collision handling between UL and DL at UE side for SBFD aware UE.

|  | Company |
| --- | --- |
| Support | Vivo, ITRI, CEWIT, New H3C, NEC, Nokia, NSB, Sony (text suggestion), Lenovo, IDC, Ericsson, Samsung, DOCOMO,CMCC, QC, Intel, WILUS, Panasonic, Fujitsu, CATT, LG Electronics, KDDI |
| Not support | OPPO (modification proposed), MediaTek |

| Company | Comments |
| --- | --- |
| CEWiT | We agree in general. However, we have concerns on what is the definition of “SBFD aware UE”? It seems like the UE is being explicitly signalled about SBFD (e.g., subband) operation. However, there is no agreement regarding this explicit signalling. Hence, we prefer the proposal by QC in the previous round. “Study potential enhancements for UE collision handling between UL and DL at UE side with assumption of relaxed restriction to allow UL/DL multiplexing in same slot in different subbands.” |
| Huawei, HiSilicon | The “SBFD aware” is a bit ambiguous. We prefer the original wording which is more simple and generic. It is anyway would be coupled with a UE capability. |
| Nokia, NSB | We still think it only makes sense to study this in case on non-transparent SBFD operation for SBFD aware UE, i.e. if the subband configuration is signalled to the UE (Proposal 3-2). |
| Sony | The word “aware” may not be used often. Perhaps we use another term that is used more frequently, i.e. “support”. Suggested proposal: Proposal 4-1a: Study potential enhancements for collision handling between UL and DL at UE side for UE supporting SBFD. NOTE: A UE that supports SBFD may or may not requires explicit signalling to tell it where exactly the subbands. |
| Ericsson | We support Proposal 4-1a; however, we also agree that Sony's suggested wording is more clear ("for a UE supporting SBFD") which clearly differentiates from legacy UEs. |
| Samsung | We are fine with the intent of the proposal and prefer Sony’s revision. |
| DOCOMO | Support the intention of the proposal, and Sony’s version is fine for us. |
| CMCC | We also prefer Sony’s revision. |
| ZTE | Fine with Sony’s revision. |
| Xiaomi | Sony’s revision is fine to us. |
| QC | Support |
| OPPO | We think RAN1 needs to firstly study which of following interpretations is taken for e.g. a UL sub-band. - Interpretation #1: the UL subband is interpreted as resource direction planning, where the actual resource direction can be subject to collision handling. - Interpretation #2: the UL subband is interpreted as resource direction re-definition, where the actual resource direction is already redefined as UL and then UE even does not check any potential DL reception in symbols due to half-duplex limitation. No collision is perceivable to UE in this case. Because in our understanding the collision between DL and UL has not much to be studied if Interpretation #2 is taken, does this proposal imply the subband will be used just as a way for “resource direction planning” in Interpretation #1? The proposal also means the SBFD would be made non-transparent to UE, which should be discussed separately. In a word, this proposal contains two fundamental issues, both of which touch the prerequisite of collision handling itself. If the majority indeed want to jump to this proposal before solving the above two prerequisites, we suggest to consider the following proposal modification that can hopefully do the bypass: Proposal 4-1a: Study potential enhancements for collision handling between UL and DL at UE side for SBFD aware UE if the UE can be made aware of the collision. |
| MediaTek | We don’t really think this is an urgent topic for study. - For legacy UEs, the collision is expected to be avoided by the network, and the scheduling needs to rely on legacy collision handling rules. Without progress on other aspects for SBFD (e.g., transparent or non-transparent SBFD, how the subbands configured, how the new UEs scheduled), it will be infeasible to have meaningful discussion on collision handling rule for SBFD aware UEs. |
| Fujitsu | Support with Sony’s revision. |
| LG Electronics | Support the proposal in principle. Also, fine with modification from Sony. |

  - Resource allocation

Due to UL subband within DL symbols, DL resource allocation for both physical channels, e.g. PDSCH and physical signal e.g. CSI-RS may be impacted and potential enhancements may be studied [6][10][15].

It is proposed in [10] that multi-slot PUSCH, e.g., PUSCH repetition Type A, and PUCCH repetitions can be configured for transmission using only the SBFD slots/symbols. Frequency domain resource allocation for PDSCH repetition is discussed in [20].

Separate PUSCH/PUCCH configurations for PUSCH/PUCCH transmissions in an UL sub-band is proposed in [21].

Question 4.1:

Do you agree to study the impact/potential enhancements of resource allocation in symbols with subbands?

|  | Company |
| --- | --- |
| Agree | Sony, IDC, Sharp, QC, New H3C, vivo, NEC, Xiaomi, OPPO, Ericsson, Spreadtrum, CATT, Intel, ITRI, Lenovo, ETRI, ZTE, Samsung, CMCC, DOCOMO, WILUS, CEWiT, Nokia, NSB, MediaTek, LG Electronics, ITRI |
| Not agree |  |

| Company | Comments |
| --- | --- |
| InterDigital | Basically, any kinds of configured periodic/semi-persistent resources are to be checked, as limiting those resources to be only used on some fixed slot/symbols are practically impossible and severely restricts flexibility in resource utilization. |
| Ericsson | Agree that all signals/channels should be checked. Some enhancements can be envisioned already, e.g., non-contiguous frequency domain resource allocation for CSI-RS. |
| Samsung | Yes. Both time-domain and frequency domain resource allocation should be studied. |
| Nokia, NSB | We think this proposal is related to “enhancements for UE collision handling”, i.e. enhancements to resource allocation in symbols with subbands may be seen as one way to enhance UE collision handling. |

  - Initial access

Apple [17] thinks that it may be desired by a future release UE a signaling and procedure to indicate UE(s) about duplex operation at a cell before/during cell acquisition for cell (re)selection and proposed to study feasibility of a mechanism to indicate future release UE about cell duplex operation mode.

Samsung [10] and Nokia [22] see potential benefit for initial access in UL subband, e.g. PRACH and RRC/NAS signaling exchange during initial access in UL subband.

Figure 4-14: RACH occasion in UL subband [10]

Question 4.2:

Do you agree to study signaling and procedure to indicate subband non-overlapping full duplex operation at a cell for cell (re)selection?

|  | Company |
| --- | --- |
| Agree | Sony, QC, New H3C, Samsung (modified proposal), KDDI |
| Not agree | IDC (rather for study on BFR first), Sharp, NEC, Ericsson, Spreadtrum, CATT, Intel, ETRI, CMCC, Nokia, NSB, MediaTek, LG Electronics, ITRI |

| Company | Comments |
| --- | --- |
| InterDigital | We’re not so against on the study, but it seems better to be considered in a later phase or release, as cell (re)selection procedures happen not frequently. In a better step-by-step approach, we would suggest studying BFR procedure first which is designed for much dynamic recovery process based on Layer-1 measurements. |
| Sharp | Future compatibility is not in the scope of SID. |
| New H3C | The signalling to the SBFD capable UE is needed before the UE start the FD operation. For legacy UE, it should ignore such signaling. |
| vivo | We think it should be low priority. |
| NEC | Similar view as InterDigital |
| Xiaomi | We share the views that it can be deprioritized. |
| OPPO | Share the view with InterDigital. This study is not critical for SBFD operation. |
| Ericsson | We think that as a baseline, SBFD should not be visible to UEs in IDLE mode. SBFD operation in CONNECTED mode should be prioritized. |
| Spreadtrum | It can be studied in the late stage. |
| Intel | We don’t think this issue should be studied at this stage. |
| ZTE | Similar view as InterDigital. |
| Samsung | We see benefits in making it possible to use SBFD UL transmissions already during the Initial Access and NAS Attach procedure. We are not suggesting that existing cell (re-)selection behavior should be changed. Actual NR TDD 30 kHz deployments experience much worse (~2x) total procedure time when compared to FDD NR low-band with 15 kHz. It must be considered that about 8-10 DL and UL messages must be exchanged in DL and UL directions each by the network and UE during NAS initial attach after RACH msg 4. These signaling exchanges are mostly “atomic” and “in sequence”, e.g., successful completion of a NAS procedure such as Authentication Request/Response is required before a subsequent NAS procedure such as NAS Identity Request can be initiated. NR TDD 30 kHz when compared to NR FDD 15 kHz in the NR low band is penalized by the average UL frame alignment delay incurred for each of the UL RRC and NAS messages due to limited number of UL slots. We assume that baseline for SBFD operation in the TDD cell is using SBFD when the UE is in RRC_CONNECTED. The UE can be provided with a dedicated RRC configuration for the SBFD UL SB when needed. However, enabling SBFD already during Initial Access, similar to R17 REDCAP early indication would allow much to compress the timeline and bring TDD on par with FDD during Initial Access. For using RACH in the SBFD slots, we see this as an independent question from Initial Access. The use of RACH ROs in the UL SBs for SBFD slots can avoid resource fragmentation in the UL slot. More contiguous UL schedulable BW becomes available to the gNB scheduler. We propose the following modified wording, “RAN1 to study potential benefits and specification impacts of SBFD during the Initial Access and NAS Attach procedure.” |
| CMCC | We don’t see the motivation to study it, since the SBFD deployment will not impact on SSB transmission and the cell selection/re-selection measurement procedure is the same as previous releases. |
| DOCOMO | We are open to this aspect but it should be lower priority than study of CONNECTED mode. |
| WILUS | We share the similar view with InterDigital. |
| Nokia, NSB | This does not seem to be an essential feature of SBFD. The benefits of a SBFD capable UE using information on SBFD operation in a cell when performing cell (re)selection are not fully clear. This requires discussion in RAN2. |
| MediaTek | Not an essential issue at this stage of the SI. |

Question 4.3:

Do you agree to study potential benefits and specification impacts of random access and initial access using UL subband?

|  | Company |
| --- | --- |
| Agree | Sony, Sharp, QC, New H3C, CATT, Intel, Lenovo, ZTE, Samsung, CMCC, DOCOMO, KDDI, Nokia, NSB, MediaTek, LG Electronics, SK Telecom, ITRI |
| Not agree | IDC, NEC, OPPO, Ericsson, Spreadtrum, ETRI |

| Company | Comments |
| --- | --- |
| InterDigital | Similar to Q4.2, we’re not so against on the study, but it seems better to be considered in a later phase or release. In a better step-by-step approach, we would suggest studying BFR procedure first which is designed for much dynamic recovery process based on Layer-1 measurements. |
| Sharp | In Rel-17, msg3 has been identified as a potential bottleneck for coverage and a solution has been specified. From UL coverage point of view, study can be done in this SI. |
| vivo | Not a priority during SI. |
| NEC | Advantage of using SBFD for initial access are not very clear. Besides, introducing initial access (e.g. PRACH) enhancements can result in more complex network operation because it would require additional signaling mechanisms for idle UEs and can result in lower system efficiency. We prefer to study this at a later stage. |
| OPPO | The SBFD operation framework could be different with initial access procedure, in which the RRC configuration could get lost. The pre-requisite could be putting subband information into broadcast signaling, which is not required for basic SBFD operation and should be postponed to later release. |
| Ericsson | We don't think latency for connection setup is a bottleneck, hence in our view the latency and coverage improvements targeted in this study item should be focused on CONNECTED mode. |
| Intel | We’re open to study SBFD in initial access. During the study, we should consider the impact on legacy UE. |
| ZTE | We are open to consider SBFD for initial access. But maybe it should be treated as second priority. |
| Samsung | See our comments on question 4.2 |
| CMCC | We are open to discuss it. |
| WILUS | We’re open to study SBFD during initial access. However, SBFD operation on RRC_CONNECTED mode should be studied with higher priority. |
| Nokia, NSB | Allowing use of SBFD slots/symbol during initial access procedure can improve the latency and resource utilization. |
| Huawei, HiSilicon | Support. Compared with legacy TDD, SBFD has more UL resources in UL subband in SBFD slots. So it can be used to transmit PRACH to enhance the initial access. Of course, this may only be applicable for SBFD capable UEs. |
| MediaTek | Open to discuss. |

  - CSI measurement/feedback enhancement

CSI measurement and reporting enhancements are proposed in [7][14][27].

It is proposed in [7] to study potential enhancements for CSI measurement and reporting for SBFD considering potential collision with subband.

It is proposed in [14] to consider to apply measurement skipping on some SBFD slots/symbols and power adjustment in deriving CSI.

It is proposed in [27] to study CSI feedback enhancements considering that the frequency resources for the duplex operation mode and those for CSI feedback may or may not be identical to each other at a given time instance.

Question 4.4:

Do you agree to study CSI measurement and reporting enhancements for SBFD operation?

|  | Company |
| --- | --- |
| Agree | Sony, IDC, QC, New H3C, vivo, NEC, Xiaomi, Ericsson, Spreadtrum, CATT, Panasonic, Intel, Lenovo, ETRI, ZTE, Samsung, CMCC, DOCOMO, CEWiT, KDDI, Nokia, NSB, MediaTek, LG Electronics, ITRI |
| Not agree |  |

| Company | Comments |
| --- | --- |
| InterDigital | Basically, as some examples of various configurable periodic/semi-persistent resources, periodic/semi-persistent CSI-RS based measurements are to be studied in relation to SBFD, as limiting those resources to be only used on some fixed slot/symbols are practically impossible and severely restricts flexibility in resource utilization. |
| Ericsson | We are okay to study, but we point out that subband CSI reporting in Rel-15 already supports reporting of non-contiguous subbands, |
| Nokia, NSB | We see the discussion on CSI resources and measurement enhancement s as a part of the discussion on resource allocation in section 4.2. |
| LG Electronics | We think study of CSI measurement and reporting for supporting SBFD operation is required. Channel condition, Tx power, CSI-RS resource could not be identical between HD slot and FD slot. |

  - Others

There are some other enhancements proposed by companies, e.g.

- Timing advance offset to be configured as 0 [2]

- non-uniform MCS [8]

- time boundary alignment between UL and DL within a slot for SB-FD [24]

- impacts on reciprocity assumption due to separate Tx and Rx panels [29]

Question 4.5:

Do you see any other enhancements need to be discussed to enable SBFD operation?

| Company | Comments |
| --- | --- |
| Sony | We support non-uniform MCS allocation for a TB where the CB/CBG in a TB can have different MCS such that lower MCS can be used for frequency resource that have strong CLI and vice-versa. We can also consider non-uniform UL transmit power where different RBs can be transmitted with higher/lower power for a TB. |
| InterDigital | UL power control aspects need to be studied, as the UL subband allocation approaches to the DL subband allocation, the negative UE-to-UE CLI effects to DL performance increase (as shown in our initial evaluation results). |
| QC | Other enhancement related to timing (e.g., timing advance or DL/UL alignment) should be studied. |
| vivo | Agree with InterDigital that UL power control should be studied. |
| NEC | Effect of time alignment between UL and DL needs to be discussed, especially considering the inter-UE CLI caused by any misalignment. |
| Xiaomi | We also agree with that UL power control should be studied. Furthermore, the DL-to-UL switching period (between DL slot to UL subband from UE perspective) should also be considered. |
| OPPO | In our view, UL power control can be discussed in interference solution, but not here. |
| Ericsson | Not at this time |
| Intel | We are fine to further study UL power control, valid symbol/slot for PUSCH/PUCCH with repetition, etc |
| Lenovo | We think uplink power control for transmission in UL subband should be studied. |
| Samsung | We see limitations when using existing legacy UL power control with SBFD. For example, when the gNB configures higher open-loop power control parameters for a UE to overcome the inter-subband CLI and self-interference in the SBFD slot, this may result in higher UL interference in the full UL slot, where no inter-subband CLI and self-interference occurs. Similar considerations apply to the closed loop power control component. We also see need to limit the maximum configured UE output power in SBFD slots, e.g., not just per serving cell per UE (or for the cells in FR1). The max configured UE output power determines the interference range of the aggressor UE transmitting in the UL SB. QCL assumptions for transmissions using the SBFD UL subband should be considered. We are open to consider benefits and impacts of transmit timing and possibly inter-UE alignment and frequency-domain resource allocation in the SI. |
| CMCC | TA offset and time boundary alignment should be studied. |
| DOCOMO | We are open to further study potential aspects, especially for UL power control and timing issues. |
| Nokia, NSB | We think that the gains of non-uniform MCS (and transmit power) should be evaluated before agreeing on support. Non-uniform transmission power may also be difficult to realize in practice, RAN4 input is needed. Timing advance and time-boundary issues should be first clarified. We think it is important to discuss the implication on reciprocity (as well as on reduced massive MIMO gain for same gNB volume) due to separate Tx and Rx panels. In general, we think it is too early to preclude discussions on other enhancements. |

- Interference mitigation solutions

New interference types are introduced in SBFD operation, including self-interference, inter-subband CLI at both gNB and UE sides. There can be intra-subband CLI if subband configurations across cells are not aligned or when co-channel co-existence with legacy TDD cells.

Study of interference mitigation solutions is important to judge the feasibility and benefit of SBFD. Various interference mitigation solutions are proposed by companies where some of the solutions are RAN4-related, e.g. antenna isolation, analog/digital filter, RF cancellation, etc. Several companies discussed the need of RAN4’s inputs [2][6][7][22][30]. Given that interference modeling needs to be discussed in AI 9.3.1 for evaluation which also requires RAN4’s inputs, it is proposed to discuss later whether additional inputs from RAN4 are needed.

The proposals for UE-to-UE CLI mitigation and for gNB-to-gNB CLI mitigation are summarized in section 5.1 and 5.2 respectively. It seems that the discussion may overlap with the discussion in AI 9.3.3 and it is desirable that we first discuss how to organize the discussion to avoid repeating the same discussion in multiple agenda items.

Question 5.1:

How do you prefer to organize the discussion of CLI mitigation schemes which are applicable for both SBFD and dynamic/flexible TDD?

- Option 1: CLI mitigation schemes which can be applicable for both subband non-overlapping full duplex and dynamic/flexible TDD are discussed in agenda item 9.3.2.

- Option 2: CLI mitigation schemes which can be applicable for both subband non-overlapping full duplex and dynamic/flexible TDD are discussed in agenda item 9.3.3.

|  | Company |
| --- | --- |
| Option 1 | Sony, IDC (in principle), new H3C, OPPO, Ericsson |
| Option 2 | Sharp, QC, NEC, Xiaomi, Spreadtrum, CATT, Intel, Lenovo, CMCC, DOCOMO, CEWiT, LG, ITRI |

| Company | Comments |
| --- | --- |
| Sharp | Inter-subband CLI should be discussed under this AI, and Intra-subband CLI should be discussed under 9.3.3. |
| New H3C | The CLI should be considered whatever in which item. If we have to select one ,we slightly prefer option1. |
| vivo | We are fine as either option as long as it will be discussed in one agenda for efficiency. |
| NEC | It is preferable to have the common aspects for CLI mitigation for SBFD and flexible TDD should be discussed in 9.3.3. In 9.3.2, we should discuss the aspects only applicable to SBFD. |
| Ericsson | In our view, dynamic TDD can be viewed as a special case of SBFD where there are differing subband sizes within/between operators, and the subband size is either 0% or 100%. Hence, we think that CLI mitigation (if needed) can be discussed in 9.3.2 primarily. The discussion in 9.3.3 can simply be mitigation approaches that are specific to dynamic TDD. In fact in our 9.3.3 contribution, we discuss how a phased approach could be adopted for studying dynamic TDD starting with a two operator scenario (SBFD and legacy static TDD) in a first phase (baseline) followed by later phases where the static TDD operator is replaced by a SBFD operator and differing subband sizes are introduced. Finally full-blown dynamic TDD can be studied. We think that CLI mitigation schemes (if needed) that are identified in earlier phases will be common to both SBFD and dynamic TDD. |
| Panasonic | We are fine with either option. |
| Intel | While either option could work, and Option 2 is slightly preferred if we have to pick one, we are wondering if this categorization is sufficiently unambiguous in determining “applicability” to SBFD or dynamic/flexible TDD only or both, etc. Instead of categorizing from perspective of schemes/solutions, it may be better to categorize as suggested by Sharp – that inter-subband CLI be discussed under 9.3.2 and intra-subband CLI be considered under 9.3.3, |
| ZTE | It seems the chair and rapporteur’s guidance is to treat inter-subband CLI in 9.3.2 and intra-subband CLI in 9.3.3. |
| Samsung | We have no strong view and preference. |
| CMCC | Inter-subband CLI can be studied in AI 9.3.2 and intra-subband CLI can be studied in AI 9.3.3 |
| DOCOMO | We are fine with either option but slightly prefer option 2. |
| CEWiT | Interference scenarios in dynamic TDD will also be present in SBFD, but interference scenarios like inter-subband CLI, intra-cell UE-to-UE CLI etc. will be specific to only SBFD. Hence, in our opinion, the CLI handling schemes applicable for dynamic TDD will also be applicable to SBFD. Therefore, CLI handling schemes applicable for both dynamic TDD and SBFD should be studied in 9.3.3 whereas, CLI handling schemes specific to SBFD should be studied in 9.3.2. |
| Nokia, NSB | We have same view as Sharp. |
| Huawei, HiSilicon | As discussed during the GTW, it may be fine to study inter-suband CLI in 9.3.2 and intra-subband CLI in 9.3.3. |
| MediaTek | To manage the load in each agenda item, for SBFD and dynamic/flexible TDD, inter-UE CLI can be discussed in 9.3.2 and inter-gNB CLI in 9.3.3. |
| LG Electronics | We are fine with either option, and Option 2 is slightly preferred. |

  - UE-to-UE CLI mitigation

    - 1st round discussion

CLI-RSSI and SRS-RSRP reporting introduced in Rel-16 are based on L3 filtered measurements and L3 reporting is configured with support for both event-triggered and periodic reporting. L1/L2 based UE-to-UE CLI measurement/report is proposed to be studied in [2][5][8][16][24][30][32].

In addition, subband-based CLI measurement/report is proposed to be studied in [8][10][18][32].

Power domain enhancements e.g. to increase DL transmission power for victim UE or to reduce UL transmission power from aggressor UE are proposed in [8][10][14][16][17].

There are other proposals for UE-to-UE CLI mitigation, e.g.

- Consider timing alignment on measurement RS transmission [3]

- event-based CLI sensing behavior at the victim UE side [14]

- measure SRS before PUSCH transmission for aggressor [17]

- specify a resource in which UE does not perform DL reception and UL transmission [23]

- Support UE Rx beam (QCL-D) configuration and indication per CLI measurement resource [32]

Companies can comment on which schemes should be discussed for SBFD operation in this agenda item.

| Company | Comments |
| --- | --- |
| Sony | We should consider inter subband interference measurements since intra-cell UE to UE CLI is caused by inter-subband interference. We expect a DL subband to be wideband and so the inter subband interference measurements within a DL subband should be divided into mini-blocks (or mini-subbands). An example is shown below where the DL subband is divided into blocks {B#1, B#2, B#3, B#4} and the UE measures the inter subband interference in each block. This would benefit the scheduler as it knows how aggressive (or what MCS) it can schedule a PDSCH for a UE that is close to the UL subband (i.e. in block B#1). We should also consider CLI measurement reports in L1 especially for intra-cell CLI since this are used for scheduling at the gNB. |
| InterDigital | We consider the following (which may not be disjoint approaches, but can be applied together to better cope with CLI): - L1/L2 based UE-to-UE CLI measurement/report - subband-based CLI measurement/report - Power domain enhancements (gNB’s DL power backoff, UE’s UL power reduction) - Victim UE-side event-based CLI sensing and subband switching (not LBT at Tx) - measure SRS before PUSCH transmission for aggressor - specify a resource in which UE does not perform DL reception and UL transmission (as “muted RBs” indications) - Support UE Rx beam (QCL-D) (or aggressor UE’s SRS beam) configuration and indication per CLI measurement resource |
| QC | The study shouldn’t be limited to specific enhancement. This is the first meeting and all suggested schemes by the companies could be categorized and listed for further study. - Support L1/L2 based CLI reporting to increase flexibility and reduce reporting latency compared to Rel-16 L3 based framework - Support UE Rx beam (QCL-D) configuration and indication per CLI measurement resource for enabling CLI-aware beam management - Support subband-based CLI reporting for accurate measurement of CLI leakage in SBFD |
| vivo | About subband-based CLI measurement/report, from our understanding, it is already supported in Rel-16 by configuring appropriate nrofPRBs. We would like to ask proponent what additional enhancements are needed for subband-based CLI measurement/report for Rel-18 SBFD compared to Rel-16. We think it is beneficial to study the L1/L2 based UE-to-UE CLI measurement/report. |
| NEC | L1/L2 based CLI measurement and reporting should be studied. Also, beam specific enhancements can be considered for CLI mitigation. |
| OPPO | R16 UE CLI can be a starting point and following enhancements can be considered: - L1/L2 based UE-to-UE CLI measurement/report - subband-based CLI measurement/report |
| Ericsson | We are open to discussing enhancements of Rel-16 CLI-RSSI and SRS-RSRP such as L1/L2 measurement reporting and subband reporting. |
| Panasonic | We consider that at least the following enhancements need to be studied. However, we are open for other enhancements. - L1/L2 based UE-to-UE CLI measurement/report - subband-based CLI measurement/report - Power domain enhancements (reduction of UL transmission power from aggressor UE) |
| Intel | In our view, it is better to consider some higher-level categories for different CLI mechanism. We can discuss each category. For example, - L1/L2 based CLI procedure - Beam related enhancement for CLI - Timing enhancement for CLI - New Information to exchange, etc. … |
| Lenovo | We think L1/L2 CLI measurement and reporting, along with spatial differentiation (e.g. measurements with different Rx beams), and power-level solutions, i.e. power control, should be studied. |
| ZTE | Timing alignment for measurement RS transmission In Rel-16 CLI, the time offset is derived by UE implementation. However, UE2 cannot derive the time offset accurately by itself, especially in some of the typical deployments of Rel-18 full duplex. Therefore, potential enhancement on timing alignment should be reconsidered in Rel-18. |
| Samsung | We think that introducing a UE reportable CLI-SINR measurement using a known configured DL measurement signal, e.g., NZP CSI-RS resource(s) is a better way to measure and report intra-cell inter-subband UL-DL interference than using SRS (as by R16 CLI). Forcing the UEs to transmit SRS in the DL subband(s) of an SBFD slot is not only detrimental to DL throughput but is also questionable in terms of its usefulness to the gNB scheduler. When SRS is transmitted by a UE in the DL subband(s) of the SBFD slot, DL transmissions from the gNB in these measurement resources are muted. SIC is not active. UE reported signal strength measurements, e.g., SRS-RRSP are therefore not much representative of actual UL-DL inter-subband interference in the SBFD slot. |
| CMCC | As the discussion in question 5.1, we only need to focus on solutions for inter-subband CLI handling in this AI, and measurement and report of UE-to-UE inter-subband CLI should be studied. |
| DOCOMO | Agree with Intel’s suggestion for high-level category first. In our opinion, beam specific LCI report enhancement, L1/L2 CLI report enhancement, and subband CLI report enhancement are the most potential enhancements. |
| CEWiT | Same view as QC. However, we would like to add some more categories: - Support dynamic measurement and reporting of CLI - Support study of CLI handling schemes at aggressor UE (e.g., Tx beam related) - Support study of CLI handling schemes for improvement of measurement accuracy. |
| KDDI | We think that categorization and listing should be first. Regardless of whether the victim UE belongs to an adjacent carrier (i.e., other operator) or not, we believe that a mechanism to control the UL transmission of aggressor UE is necessary because the CLI impact is larger when the victim UE is in poor DL quality. |
| Nokia, NSB | We think it is too early to preclude any enhancement. We agree with Qualcomm that RAN1 should start by identifying, based on existing proposals, potential areas for enhancement, e.g.: - Advanced UE RF requirements - Power control enhancements - L1/L2 based CLI reporting - Schemes for accurate measurement of CLI leakage In particular, the focus should be on enhancements to handle intra-cell inter-subband UE-to-UE CLI. |
| Huawei, HiSilicon | As discussed in our contribution, L1/L2 based UE-to-UE CLI measurement solutions should be studied at least. |
| MediaTek | No restriction on CLI mitigation need to be considered at this stage. CLI and SI will be the main issues for SBFD, and we should be open to discuss any solution that addresses these interferences. |
| LG Electronics | We are fine with identification of potential area for enhancement at this stage. In our view, L1/L2 based UE-to-UE CLI measurement/report can be included. Also, subband-wise CLI measurement can be considered for SBFD case. |

    - 3rd round discussion

Given the guidance below and based on companies inputs, let’s discuss potential inter-gNB and inter-UE CLI handling schemes that are specific for SBFD.

Guideline for future meetings - Note: AI 9.3.3 handles the potential inter-gNB and inter-UE CLI handling schemes that are specific for dynamic TDD and schemes that are common for both SBFD and dynamic/flexible TDD. - Note: AI 9.3.2 handles the potential inter-gNB and inter-UE CLI handling schemes that are specific for SBFD.

Proposal 5-2:

Study subband based CLI measurement and report

|  | Company |
| --- | --- |
| Support | Spreadtrum, Lenovo, New H3C, DOCOMO, ZTE, vivo, CEWiT, WILUS, Huawei, HiSilicon, NEC, Panasonic, CATT, Fujitsu, Sony, SK Telecom, Xiaomi, OPPO, Nokia, NSB, Samsung, QC, Ericsson, ITRI |
| Not support | Intel , LG Electronics |

| Company | Comments |
| --- | --- |
| Intel | The wording is confusing. Rel-16 CLI already supports CLI measurement on configured PRBs, e.g., a UE can be configured with two CLI resource and each CLI resource consists of 40MHz respectively within a BWP of 100MHz, which is effectively subband based CLI. In our understanding, the intention of the proposal is to study CLI measurement and report for inter-subband CLI handling, rather than subband CLI. Therefore, we suggest revising the proposal as Study CLI measurement and report for inter-subband CLI handling. |
| OPPO | Our understanding is that the Proposal 5-2 has no intention to let UE to distinguish between gNB-to-UE interference and UE-to-UE CLI. |
| Nokia, NSB | We are ok to study subband based CLI measurements and reports, but we think it is too early to have an agreement/proposal that limits the study to subband based CLI measurements and reports. |
| LG Electronics | In Rel-16 CLI handling WI, L3 based subband wise UE-to-UE CLI measurement and reporting is already designed as CLI handling framework including CLI-RSSI and SRS-RSRP. Target of this proposal seems not to be clear. It needs to be clarify whether this ‘CLI measurement and reporting’ is targeting for gNB or for UE, and whether other ‘CLI measurement and reporting’ (e.g., L1/L2 based) is mentioned. |
| MediaTek | We should keep the proposal generic: “Study inter-gNB and inter-UE CLI handling schemes for SBFD” |
| Ericsson | Given that CLI measurement and reporting exists in Rel-16, maybe it's better to rephrase as "Study whether enhancements to Rel-16 CLI measurement and reporting are beneficial for SBFD operation" |

Question 5-2:

What other inter-UE CLI handling schemes that are specific for SBFD should be studied?

| Company | Comments |
| --- | --- |
| Lenovo | Power control based schemes, e.g., using separate power control for the UL transmission in the subband. |
| ZTE | From our perspective, power control based schemes may be SBFD specific, e.g., power control based schemes for DG-PUSCH and CG-PUSCH. |
| NEC | It is preferable not to make any hard agreement on the CLI handling schemes to be studied at this point (given that SBFD framework itself is not fixed). We can keep this discussion for the next meeting. |
| Sony | I think there is a huge list of possible schemes that companies would be interested it. I therefore same similar view with NEC that we do not start excluding any schemes that did not have a chance to be discussed properly yet. |
| Intel | We share similar view with Sony. |
| Nokia, NSB | We think that potential areas for further study with special focus on intra-cell inter-subband UE-to-UE CLI are: - Advanced UE RF requirements - Power control enhancements - L1/L2 based CLI reporting - Schemes for accurate measurement of CLI leakage |
| Samsung | We think that this discussion (and consolidating a list based on company inputs) can be deferred to the next RAN1 meeting. |
| Ericsson | Similar view as Samsung |

    - 4th round discussion

Proposal 5-3:

Study whether enhancements to Rel-16 CLI measurement and reporting are beneficial for SBFD operation

|  | Company |
| --- | --- |
| Support | Spreadtrum, vivo, CEWiT, New H3C, NEC, Huawei/HiSilicon, DOCOMO, OPPO, WILUS, Fujitsu, CATT |
| Not support | Nokia, NSB, Sony, IDC, Samsung, Intel, Panasonic, MediaTek, LG Electronics, KDDI |

| Company | Comments |
| --- | --- |
| Spreadtrum | For SBFD with or without dynamic TDD, the interference model is even more complex and rapidly varies. A faster CLI measurement/report mechanism can be beneficial to handle this issue. |
| Nokia, NSB | “Enhancements to Rel-16 CLI measurement and reporting” seems to suggest RAN1 should only focus on enhancements of the existing CLI measurement and reporting schemes. It is not clear if this would preclude studying other enhancements such as advanced UE RF requirements, power control enhancements, etc. We think it is too early to limit the study to enhancements of CLI measurement and reporting schemes. Therefore, we propose to change to “Study whether additional enhancements are beneficial to handle UE-2-UE intra-cell inter-subband CLI in SBFD operation”. |
| Sony | The original proposal, i.e. Proposal 5-2 was fine. We do share similar view with Nokia that this proposal seems to restrict us by tethering us to the past (i.e. Rel-16) and therefore limit the enhancements that we can introduce for Rel-18. The arguments made by Intel & LG seems to simply suggest that we do not re-invent the wheel and should introduce something that Rel-16 CLI cannot offer. Perhaps we should simply just address their concerns. A suggested proposal: Proposal 5-3: Study whether new measurements and reportings, other than those in Rel-16 CLI, are beneficial for SBFD operation. |
| Lenovo | We prefer the modified proposal 5-2: “Study CLI measurement and reporting for SBFD operation” |
| InterDigital | Share same view as Sony |
| Ericsson | Okay to defer to next meeting. Based on the guidelines agreed in the GTW earlier this week, schemes need to be categorized as generic (apply to dTDD and SBFD – discussin in 9.3.3) or specialized (only apply to SBFD – discuss in 9.3.2). Deferring to next meeting allows time to do this. |
| Samsung | We certainly agree with Proposal 5-3. We consider enhancements to CLI measurement and reporting as potentially beneficial for SBFD. But there are other areas for potential enhancements as well. We propose to remain with Round 3 Proposal 5-2 and to discuss and consolidate a list based on company inputs by the next RAN1 meeting. |
| CMCC | From our perspective, the Rel-16 CLI is only intra-subband (co-channel) CLI, if the intention of this proposal is further enhance intra-subband (co-channel) CLI, it should be studied in AI 9.3.3. But if the intention of this proposal is further study inter-subband CLI, we are fine with it. |
| ZTE | We are fine with Sony’s revision. |
| Xiaomi | We also think the current wording is a little bit restrictive. We are fine with Sony’s version or the suggestion from Samsung. |
| QC | We think the proposal text should be further clarified as it currently limits scope to only L3 based Rel-16 CLI framework. We suggest to update proposal language to study limitation of Rel-16 CLI framework and identify enhancement and new solutions for CLI measurement and reporting for SBFD operation. Proposal 5-3: Study limitation of Rel-16 CLI framework and identify enhancement and new solutions for CLI measurement and reporting for SBFD operation. |
| Intel | We share similar view with other companies, we should not limit the scope to enhancement based on Rel-16 CLI. We are fine with NOKIA’s revision. |
| Panasonic | We are OK either of Sony's revision or Qualcomm's revision. |
| MediaTek | No need to restrict the study to “measurement and reporting”. We propose having generic wording: “Study inter-gNB and inter-UE CLI handling schemes for SBFD”. @Nokia: we support your intention, but why you want to limit the study to “UE-2-UE”, “intra-cell” & “inter-subband” CLI? What about inter-gNB, inter-cell, and intra-band CLIs? |
| LG Electronics | Enhancement of Rel-16 CLI handling mechanism could be a one of candidate direction for supporting SBFD. But, in this stage, it is not desirable to limit the scope. |
| KDDI | We are fine with Sony’s revision. |

  - gNB-to-gNB CLI mitigation

    - 1st round discussion

Various gNB-to-gNB CLI mitigation schemes were proposed in [2] [3][5][7][18][22][32]. The proposals are quite divergent. Please refer to respective contributions for more details.

Companies can comment on which schemes should be discussed for SBFD operation in this agenda item.

| Company | Comments |
| --- | --- |
| QC | The study shouldn’t be limited to specific enhancement. This is the first meeting and all suggested schemes by the companies could be categorized and listed for further study. - Support inter-gNB coordination schemes to identify compatible inter-gNB beam pairs, which is enabled by inter-gNB CLI measurement and reporting per candidate DL/UL beam pair. - Support of inter-gNB CLI channel measurement and reporting to neighbouring gNBs for enabling Tx/Rx beamforming or nulling. |
| Ericsson | Our high level view is that gNB-gNB CLI mitigation (if needed) can be done by gNB implementation, and need not be specified. |
| Intel | In our view, the following gNB to gNB CLI mitigation mechanism can be considered for SBFD operation: - CLI measurement and reporting among gNBs - timing-synchronization assistance information exchange between gNBs - subband configuration exchange between gNBs for SBFD operation However, if a common solution is identified for both SBFD and dynamic TDD, RAN1 can further decide whether to continue the discussion under SBFD or dynamic TDD AI. |
| Lenovo | We think dedicated resources for inter-gNB CLI measurement should be studied. |
| ZTE | Regarding gNB-gNB CLI in Rel-18,  Framework for gNB-gNB CLI: e.g., using Rel-16 RIM Framework-1 as a baseline framework;  Measurement RS: the existing DL RS (e.g., SSB, CSI-RS) can be reused as measurement RS; UL rate matching/cancellation mechanism can be defined for more accurate measurement. |
| Samsung | (1) SBFD operation should not need to rely on inter-operator signaling (or inter-operator coordination). (2) We also note that many gNB-side CLI measurements are possible by implementation and without specification support. For purpose of OAM and network configuration, the gNB can create silent intervals where no DL and UL transmissions are scheduled (or configured) in the serving cell and measure the DL signals from co- and adjacent channel neighbor cells in the deployment using SSBs or NZP CSI-RS or the transmitted DL signals. In principle, gNB-to-gNB CLI interference is observable by the base station. One drawback is that such gNB-specific measurements in the inter-operator-case can’t rely on a priori knowledge of the DL signal and when it will be transmitted from the measured base station. (3) We think that RAN1 should study and evaluate the benefits of a new DL reference signal design to support intra-operator gNB-to-gNB (DL-to-UL) CLI measurements, because the existing R16 RIM-RS type 1 or 2 are not suitable for purpose of intra-operator gNB-to-gNB (DL-to-UL) CLI measurements in NR mid-band small deployments. Similar to R17 eIAB, providing desired/prohibited beam indications, but using Xn-AP to support intra-operator gNB-to-gNB (DL-to-UL) CLI mitigation could be a useful tool as CLI mitigation mechanism. |
| CMCC | As the discussion in question 5.1, we only need to focus on solutions for inter-subband CLI handling in this AI, For inter-cell gNB-to-gNB inter-subband CLI handling, the following fields can studied: - Subband analog filters Inter-gNB coordination in time-domain, frequency-domain, spatial-domain, and power domain. Backhaul signalling enhancement may be needed to support inter-vendor cooperation. |
| DOCOMO | We think inter-gNB CLI measurement and information exchange can be studied. |
| Nokia, NSB | We think it is too early to preclude any enhancement. Possible schemes for gNB-2-gNB CLI mitigation may include: - UL and DL power control - Advanced gNB receivers - Inter-gNB scheduling and beamform coordination |
| Huawei, HiSilicon | As discussed in our contribution, following should be studied by RAN1: - advanced IRC receiver - TX beamforming - Beam pairing for FR2 And following should be studied by RAN4, and the performance of these interference suppression and/or mitigation techniques should be provided to RAN1 to check the feasibility as well as the performance of SBFD: - Antenna isolation - Interference cancellation (analogue and digital domain) - RX analogue filter - Guard band |
| MediaTek | Inter-gNB CLI handling schemes should be studied given the high impact of this CLI on the system performance. Otherwise, it will be possible to assess the feasibility and performance of SBFD and DTDD schemes. |
| LG Electronics | For gNB-to-gNB CLI handling, gNB-to-gNB CLI measurement can be studied. And information exchange for supporting inter-gNB CLI measurement can be studied. |

    - 3rd round discussion

Question 5-3:

What inter-gNB CLI handling schemes that are specific for SBFD should be studied?

| Company | Comments |
| --- | --- |
| Spreadtrum | In RAN1, inter-gNB CLI measurement/report for SBFD/dynamic TDD needs further study with high priority, including more dynamic measurement and report, subband-level information sharing between gNBs. |
| New H3C | Inter-gNB CLI handling schemes on SBFD+dynamic/flexible TDD should be FFS. |
| DOCOMO | Since resource partitioning for SBFD is different from dynamic TDD case, at least SBFD specific inter-gNB information exchange enhancement should be studied. Moreover, inter-gNB CLI measurement/reporting is also potential. |
| ZTE | From our perspective, it seems most of the schemes are common for SBFD and dynamic TDD. |
| Huawei, HiSilicon | For SBFD, there are two major types of CLI - (inter-cell) gNB-gNB co-channel inter-subband CLI - (inter-cell) co-site inter-sector co-channel inter-subband CLI The inter-gNB CLI handling include advanced receivers based on potentially improve channel and interference covariance matrix estimation; Inter-gNB CLI measurement and report, etc. |
| NEC | For gNB-gNB handling, some schemes have been discussed in AI 9.3.3 for dynamic/flexible TDD. In this agenda, we should only focus on extra enhancements for SBFD. In our view, at least CLI measurement and report enhancements based on subband and scheduling info (e.g., SFBD symbols and subbands) sharing between gNBs should be investigated. |
| Sony | It is a bit too early to start excluding/including schemes that have not even got a chance to be discussed yet. I think companies should be able to judge whether a scheme belongs to SBFD or dynamic/flexible TDD. Hence it is good that we have a thorough discussion on them in the next meeting. |
| Intel | Regarding the SBFD specific inter-gNB CLI handling, subband configuration exchange between gNBs for SBFD operation and inter-gNB CLI measurement and reporting are to be studied. |
| Nokia, SNB | In general, we think that most of inter-gNB CLI handling schemes may be common to both dynamic TDD and SBFD. However, there could be solutions that are specific to SBFD, i.e. enhancements to exchange of intended TDD configuration to include gNB subband configuration. |
| LG Electronics | Enhancement of intended TDD configuration including subband configuration could be a candidate. Subband size adaptation considering on CLI could be a candidate for study. |
| Samsung | We think that this discussion can be deferred to the next RAN1 meeting. |
| Ericsson | Similar view as Samsung |

- Co-existence with legacy UE/ gNB

  - CLI with legacy UE

Intra-cell UE-to-UE CLI with legacy UE within a cell operating SBFD is discussed in [30] and [32].

Two options on Tx filtering for SBFD capable UEs (shown below) were discussed in [30], which may differ the CLI level to legacy UE. However, it is considered that the interference can be managed by gNB with proper user selection, scheduling and UL power control.

Figure 6-15: Inter-subband CLI between Rel-18 and legacy UEs [30]

It is considered in [32] that gNB should handle legacy UE by utilizing Rel-16 CLI framework and proper scheduling. In addition, it is proposed that UE RF requirements are not changed for coexistence study of legacy UE in RAN4.

For inter-cell UE-to-UE CLI, it is discussed in [30] that assuming same UL/DL resource partitioning across cells, legacy UE suffers inter-subband CLI same as intra-cell case. Otherwise if UL/DL resource partitioning across cells is not the same, there can be intra-subband CLI as well.

It is not clear what needs to be concluded/discussed at this point. Companies please provide your comments/suggestions if any in the following table.

| Company | Comments |
| --- | --- |
| QC | Similar thoughts as ones in proposal 3-1. |
| vivo | We do not think sub-band filter at the UE side should be assumed. Considering that legacy UEs can be scheduled in the sub-bands, we should assume the legacy UE implementation, i.e. wideband filter. |
| Ericsson | Clearly gNB/UEs in a legacy operator's network cannot be expected to support enhancements to be able to coexist with an SBFD network on an adjacent channel. Based on this, it is vital in this study item to evaluate a two operator scenario in which Operator A implements SBFD and Operator B (legacy) uses static TDD. Our view is that SBFD would be introduced in brownfield deployments (i.e., ones with legacy). In such a deployment, UEs/gNBs in Operator B's network should not suffer degraded performance. |
| Intel | In our view, it is important to analysis the impact to legacy UE, though it is not clear to us whether to discuss it in this agenda item or evaluation agenda item. When we evaluate co-existence with legacy UE without interference coordination, we should model the inter-subband interference based on existing filtering at legacy UE side, but the filtering at new UE side depends on RAN4’s study, e.g., subband based filtering. When we evaluate co-existence with legacy UE with interference coordination, we still need to assume existing filtering at legacy UE side. In addition, the interference can be measured and reported by legacy UE based on Rel-16 CLI mechanism, and new interference handling can be applied by new UE, and interference coordination based on result from legacy and new UE can be applied. |
| CEWiT | CLI schemes at aggressor UE (e.g., Tx beam related) can be considered to reduce impact on leagacy UEs. |
| Nokia, NSB | We think impact of SBFD operation on legacy performance is very important to evaluate. In this perspective, it is essential to assume a realistic model for both the in-band emissions and for the receiver selectivity for both gNB and UE. The model may depend on whether it can be assumed that the UE/gNB is using subband-specific digital and/or analogue filters. On the other hand, the gain of SBFD may also depend on how fast and dynamically the bandwidth of the subband-specific digital and/or analogue filters can be modified. All this requires feedback from RAN4 before RAN1 can conclude on the impact of SBFD on legacy operation. |
| MediaTek | It is essential to evaluate the impact to/from legacy UEs. Also, we need to distinguish between R16 UEs (that might support CLI measurement/reporting) and R15 UE which is not capable of CLI measurement/reporting. In addition to the CLI impact, the possible solutions targeted for SBFD capable UEs might need to consider the presence of legacy UEs. |
| LG Electronics | At this stage, it is not clear whether subband filter (Opt 1 in Figure 6.14) can be applied to SBFD capable UE or not, which needs to be studied in RAN4. Nevertheless, it can be assumed that wideband filter (Opt2 in Figure 6.14) is applied for both SBFD capable UE and legacy UE. Hence, we think that RAN1 can start a discussion based on the assumption that wideband filter is for both SBFD capable UE and legacy UE. |

  - CLI with legacy gNB

For co-channel co-existence of legacy TDD gNB and SBFD gNB, if same UL/DL assignment is applied to legacy TDD gNB and SBFD gNB, and SBFD is only operated in legacy DL symbols as shown in Figure   6 -16, it is expected that SBFD operation has no impact to the UL reception of legacy gNB as analyzed in [5] and [30].

Figure 6-16: Impact on legacy gNB for SBFD operation in legacy DL symbols [30]

Otherwise if SBFD operation is operated in legacy UL symbol(s) as shown in Figure   6 -17, legacy gNB suffers DL-to-UL interference in UL symbols as analyzed in [30].

Figure 6-17: Impact on legacy gNB for SBFD operation in legacy UL symbols [30]

It is proposed in [32] that co-channel co-existence with legacy gNB can be handled by gNB implementation technique such as subband muting (shown in Figure   6 -18), beamform nulling and interference cancellation.

Figure 6-18: Muting technique to handle coexistence with legacy gNB [32]

Intel [30] also analyzed the co-channel co-existence with legacy gNB with dynamic UL/DL assignment and legacy gNB or SBFD gNB may suffer DL-to-UL interference depending on UL/DL configuration combination, regardless of whether a UL or DL symbol can be configured with SBFD operation.

It is not clear what needs to be concluded/discussed at this point. Companies please provide your comments/suggestions if any in the following table.

| Company | Comments |
| --- | --- |
| QC | Subband muting could be efficient technique to handle coexistence with legacy. |
| Ericsson | Clearly gNB/UEs in a legacy operator's network cannot be expected to support enhancements to be able to coexist with an SBFD network on an adjacent channel. Based on this, it is vital in this study item to evaluate a two operator scenario in which Operator A implements SBFD and Operator B (legacy) uses static TDD. Our view is that SBFD would be introduced in brownfield deployments (i.e., ones with legacy). In such a deployment, UEs/gNBs in Operator B's network should not suffer degraded performance. |
| Intel | In our view, it is important to analysis the impact to legacy gNB, though it is not clear to us whether to discuss it in this agenda item or evaluation agenda item. We can discuss （1）at least for aligned legacy TDD configuration case, whether legacy gNB suffers CLI interference from SBFD gNB, e.g., - If SBFD is only allowed in DL and F symbol, no impact on legacy gNB reception. - If SBFD is supported in UL symbol, there is intra-subband DL-to-UL interference to legacy gNB. There may be coexistence issue. (2) if legacy gNB suffers CLI interference from SBFD gNB, how to reduce the interference, and how much performance degradation is expected, if any. - E.g., Muting the corresponding PRBs in legacy gNB. Then, the throughput is reduced in legacy gNB, etc. - E.g., Power adjustment |
| Nokia, NSB | We think that co-channel co-existence with legacy gNB (by e.g. subband muting) does not require specific discussions in RAN1 but can be handled as part of gNB-to-gNB CLI mitigation (section 5.2). |
| LG Electronics | We have same view with Nokia. Co-channel co-existence with legacy gNB can be handled as part of gNB-to-gNB CLI mitigation. |

- Email discussion before GTW session on May 16th

Moderator provided a set of updated proposals based on companies’ inputs before quiet time and the proposals were discussed over email with the following companies’ comments before GTW session on May 16th.

Proposal 2-1a:

Prioritize SBFD operation within a TDD carrier.

- SBFD operation across different carriers within a TDD band (intra-band CA with different TDD UL/DL configurations) can be considered with lower priority.

| Company | Comment |
| --- | --- |
| New H3C | We support this proposal |
| vivo | Do NOT support. The case that SBFD operation across legacy TDD carriers without SBFD operation within a legacy TDD carrier is not within the study scope. |
| Lenovo | Support |
| Huawei, HiSilicon | It is not clear what a lower priority means here. We could be okay if the intention is for performance evaluation purpose, but this should be clear spelled out in the proposal. |
| Ericsson | Support We think SBFD across carriers should be 2nd priority since it comes with a built in requirement for UE support of UL CA of 3 carriers. Suggest minor wording update “…(intra-band CA with different TDD UL/DL configurations on different CCs)…” |
| Intel | Support |
| InterDigital | Support |
| Xiaomi | Same views as vivo. We don’t think the subbullet is within the scope and hence no effort is needed for intra-band CA case under the umbrella of SBFD. |
| CMCC | Support |
| ZTE | Maybe one way to go is to say "study SBFD operation within a TDD carrier". Then we can delete the subbullet. |
| ASUSTeK | Fine with the proposal |
| Samsung | Support |
| Spreadtrum | Support. If SBFD operation within a TDD carrier can be directly applied for CA SBFD, we are also fine. However, at the early stage, it should be with low priority. |
| Sony | Support |

Proposal 2-2a:

Study schemes of UL subband in legacy DL and flexible symbols.

- Note: Legacy DL and flexible symbols are from gNB’s perspective

| Company | Comment |
| --- | --- |
| New H3C | We are fine with Study schemes of UL subband in legacy DL. Regarding flexible symbols, we should also study schemes of UL SB as well as DL SB. It isn’t clear to us why only to study schemes of UL subbands in flexible symbols. |
| vivo | Do NOT support. It is not necessary or fair to preclude DL subband in legacy UL/flexible symbol(s) without study in the first SI meeting. Both UL subband in legacy DL/flexible symbol(s) and DL subband in legacy UL/flexible symbol(s) are SBFD schemes that fit well with the SI scope, the SI justification is just justification, it should not limit our study scope. In reality, we have not only DL heavy TDD configuration, there is UL heavy TDD config. for vertical industries and for such case, the DL may become the bottleneck for the overall performance. In addition, the gNB-to-gNB subband CLI interference is more relaxed compared to the full band CLI in dynamic TDD that we are going to study, we do not see the logic to study dynamic TDD considering the co-existence scenario, but preclude study one SBFD scheme that DL subband in legacy UL/flexible symbol(s). We suggest following modification for study: Proposal 2-2a: Study schemes of UL subband in legacy DL and flexible symbols. Study schemes of DL subband in legacy UL and flexible symbols. - Note: Legacy DL, UL and flexible symbols are from gNB’s perspective |
| Nokia, NSB | It is not clear for the note. Why it is from gNB’s perspective? As the proposal to study UL subband (from UE perspective), we think the proposal is from UE perspective. |
| Lenovo | We think “legacy DL and flexible symbols” are from UE perspective. |
| Huawei, HiSilicon | We would like to clarify what the “legacy DL and flexible symbols” means in the proposal. Does it include legacy DL and flexible symbols configured by the gNB and legacy DL and flexible symbols determined dynamically by the gNB? |
| Ericsson | We support the intention of the proposal; however, as we raised in the email discussion it is necessary to consider legacy UE support (or lack thereof) for undetermined UL/DL direction. Therefore, we think the following should be added to the proposal: Study shall consider whether or not legacy UEs fully support undetermined UL/DL direction, i.e., cell-common configuration with 'flexible' symbols Similar to Nokia’s comment, we don’t understand why the note is added. Contrary to vivo’s comment, we think DL subband in legacy UL / flexible symbols is out of scope of the SID. The justification section of the SID explicitly states provision of additional UL opportunities, not additional DL opportunities. |
| Intel | We’re fine with the proposal. In our view, the main bullet concerns interpretation from not only gNB’s but from UE’s perspective as well. While the exact interpretation for a symbol may differ between gNB and UE -e.g., a symbol could be “flexible” for a UE, but “DL” for a gNB. Considering this, describing the proposal from a UE’s perspective (as long as no case is missed) is desirable to avoid any confusions down the road. In this regard, the “legacy DL and flexible symbols” should be those symbols identified by a UE based on semi-static configurations (or their absence, in which case all symbols are identified as semi-static flexible symbols). Accordingly, we propose the following text for the Note. Note: Legacy DL and flexible symbols are the symbols determined as such based on semi-static TDD DL-UL configurations. Regarding vivo’s comment, we share same view with E///. Regarding the legacy UE’s capability for flexible symbol mentioned by E///, in our understanding, it is mandatory for all UEs since Rel-15 to be able to handle “flexible” symbols by semi-static configurations (please check UE feature 5-1, component 7, it is optional for a gNB to provide semi-static TDD DL-UL configurations since Rel-15, but UE still has to work properly in this case), and thus, there is no need to revisit such considerations. Furthermore, it is certainly not in RAN1 scope to check and evaluate if UEs in the field are supporting mandatory requirements. Therefore, we think there is no need to add such sub-bullet. |
| Xiaomi | We are generally fine with the proposal. However, we are not sure why do we need to study UL subband in flexible symbols. Considering gNB can schedule a UE to transmit or receive on the flexible symbols up to itself, the current specification already support duplex operation in flexible symbols from gNB side. We are not sure what kind of enhancement is needed for flexible symbols. |
| CMCC | We are fine with this proposal, regarding the definition of “legacy DL and flexible symbols”, we think it can be clarified as “DL and flexible symbols configured by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated” |
| ZTE | Similar as other companies, the "flexible symbols" need to be calrified. |
| ASUSTeK | Though the final specification impact may be up to detailed signaling design, we think for flexible symbol UL subband and DL subband should be treated equally at this point (e.g. for flexible symbol those resource NOT part of UL subband are implicitly DL subband?). For DL symbol, Ericsson’s and Intel ‘s comments seem valid that it’s out of scope. Regarding the note, we also think it could be removed, probably the following edit could be considered: Study schemes of UL subband and/or DL subband in legacy DL and flexible symbols indicated by legacy RRC signaling and/or legacy DCI format 2_0. |
| Samsung | We support main bullet. Regarding the note, our understanding is the legacy DL and flexible symbols are from both gNB’s and UE’s perspective, not gNB perspective only. If it is gNB perspective only, does it allow DL subband in UE’s UL symbols? We does not support DL subband in UL symbols. |
| Spreadtrum | We support the proposal. DL subband in UL symbols should be with low priority, it is against the justification of UL coverage, delay etc. Agree with DL symbol and flexible symbol are from gNB’s perspective, because transparent or non-transparent SBFD have not been decided yet, the symbol directions can be known for gNB at least. |
| Sony | Don’t really quite get the NOTE. Are the DL & Flexible symbols also not known to the UE? |

Proposal 2-2b:

For discussion purpose, SBFD symbols is defined as symbols with subbands that gNB would use for SBFD operation.

- Note: it does not imply a new symbol type is introduced.

| Company | Comment |
| --- | --- |
| New H3C | We are fine with this proposal |
| vivo | Support |
| Lenovo | Can be a conclusion. |
| Huawei, HiSilicon | Support |
| Ericsson | Support However, we would like to clarify what is meant by “new symbol type”. Does it preclude the configuration of RB sets within such a symbol? Hopefully not. |
| Intel | Support |
| InterDigital | Support. The Note may not be needed and can be removed. |
| Xiaomi | Support |
| CMCC | Support |
| ZTE | Support |
| ASUSTeK | Fine with the proposal. Since it’s already clarify that it’s “for discussion purpose”, we think the note could be removed. |
| Samsung | Support |
| Spreadtrum | Support |
| Sony | Support |

Proposal 2-3a:

A subband consists of a set of consecutive RBs for the same transmission direction.

| Company | Comment |
| --- | --- |
| New H3C | We suggest adding RBG to resource granularity of subbands for further study. A subband consists of a set of consecutive RBs/RBGs for the same transmission direction. |
| vivo | Support |
| Lenovo | Support |
| Huawei, HiSilicon | We assume this proposal is also intended to facilitate discussion, it has nothing to do with how a subband is configured or indicated. Hence, we suggest to add “For discussion purpose”. |
| Ericsson | Support. Maybe it should be clarified that it is “within a TDD carrier” |
| Intel | Support |
| InterDigital | Support, but should add “For discussion purpose” at the beginning. |
| Xiaomi | Support Ericsson’s suggestion. |
| CMCC | Support, Ericsson’s version is also fine to us |
| ZTE | This is for discussion purpose, right? We propose to add "For discussion purpose". |
| ASUSTeK | In general the proposal is fine, while it seems not clear whether it’s from UE’s perspective or gNB’s perspective and whether they are always consistent? Prefer InterDigital’s edit. |
| Samsung | Agree with Huawei, HiSilicon. We need to separately discuss how to configure/indicate a subband and whether to allow different direction scheduling e.g., dynamic DL reception scheduling within UL subband. |
| Panasonic | We support the granularity of subband is RB-level and not RE-level. On the other hand, the current wording implies that the minimum unit of subband is 2 RBs and more because it says "a set of consecutive RBs" (e.g., RBG is used to configure subband size instead of RB). We propose to add sub-bullet: "FFS: the size of consecutive RBs" |
| Spreadtrum | Support, also fine with Ericsson’s update. |
| Sony | Support |

Proposal 2-3b:

For SBFD operation within a TDD carrier, at least consider only one UL subband and up to two DL subbands within the carrier from gNB’s perspective.

- FFS more than one UL subband within the carrier

| Company | Comment |
| --- | --- |
| New H3C | We are fine with this proposal. |
| vivo | Although we do not see the need to limit the number without any study, if majority support the proposal, we are fine with it. |
| Nokia, NSB | It should be FFS. No need to have this agreed in this early stage. We think it is still needed to have a discussion in RAN1/RAN4 about the benefits (if any) to place the UL subband in the inner part of a TDD carrier. |
| Lenovo | Do not support. Feasibility study is needed before determining the max number of subbands. |
| Huawei, HiSilicon | Okay |
| Ericsson | Support. Regarding Nokia’s comment, the proposal says “up to two DL subbands;” hence it seems both D-U-D and D-U configurations are considered. |
| Intel | Support |
| InterDigital | Support, but should remove ‘only’. Then, it reads like “at least consider one UL subbnand and up to two DL…” |
| Xiaomi | Support. |
| CMCC | Support |
| ASUSTeK | FFS, too early to make such decision. |
| Samsung | Support the intention. Regarding FFS, we are not sure the motivation to allow multiple UL subbands, but ok to discuss it. One clarification is does this proposal preclude different UL subbands across slots. For example, it is unclear whether the case where UL subband 1 is used for slot 0/1 and UL subband 2 (different location/bandwidth in frequency) is used for slot 2/3 is precluded or not? |
| Panasonic | We share the same view as vivo. |
| Spreadtrum | Support. It is good to simplify the inter-band CLI analysis, and resource allocation efficiency. |
| ZTE | OK |
| Sony | Support |

Proposal 2-5a:

The time and frequency location of subbands are configurable.

- Note that whether the time and/or frequency location of subbands are informed to UE is separately discussed.

| Company | Comment |
| --- | --- |
| New H3C | We are fine with this proposal. |
| Apple | The intention is not yet clear to us. If the purpose of this proposal it the configuration to be indicated to UE, then the whole proposal has to be discussed under the sub-bullet. If the configuration is a matter of gNB only, and transparent to UE, we don’t need to agree on something here. |
| vivo | Support |
| Lenovo | Support |
| Huawei, HiSilicon | We have the similar question as Apple. It is not entirely clear whether there is a configuration from gNB point of view is needed if it is not conveyed to the UE. |
| Ericsson | Support |
| Intel | As Apple points out, the wording for the main bullet is a bit confusing. If the intention is to say that they are not pre-defined/specified, we should just say that. For example, Proposal 2-5a: The time and frequency location of subbands are configurable not fixed or pre-defined in the specifications. - Note that whether the time and/or frequency location of subbands are informed to UE is separately discussed. |
| InterDigital | Support the proposal in principle. However, we prefer to remove the Note, or change ‘whether’ to ‘how’, since the main bullet says they are configurable then they should be anyhow informed to the UE even in an implicit way. |
| Xiaomi | Support. |
| CMCC | Support |
| ZTE | Similar question as Apple. |
| ASUSTeK | Think Intel’s edit is better, and the note could be removed after such formulation. Note that though we could understand the intention, while strictly speaking “subband” is a concept of frequency domain rather than time domain, so we could focus on “frequency location” in this proposal and remove “time” for now. (Also proposal 2-2a already implies SBFD symbols are not fixed/predefined). |
| Samsung | Support. |
| Panasonic | We share the same view as Apple. |
| Spreadtrum | Before the decision of transparent or non-transparent SBFD scheme, “configurable” can be replaced by “decided by gNB for SBFD operation” |
| Sony | Support |

Proposal 3-1a:

Existing RAN1 specification supports gNB to schedule legacy UEs to transmit UL in UL-only symbols or in UL subband in symbols with SBFD operation at gNB side and to receive DL in DL-only symbols or in DL subband in symbols with SBFD operation at gNB side.

- Further study whether or not legacy UEs fully support UL transmission and DL reception in symbols with undetermined UL/DL direction (i.e., flexible symbols)

| Company | Comment |
| --- | --- |
| New H3C | It isn’t clear to us how to support “A subband consists of a set of consecutive RBs for the same transmission direction” by Existing RAN1 specification. |
| Apple | Do not support. SBFD configuration is a concept at gNB only. Yes, even in current spec gNB may configure different UL-DL TDD configurations to different UEs, UE1 and UE2, so at a given symbol one is transmitting and another is receiving, but each UE follows its own TDD indication based on which a UE won't transmit in a D symbol or expect to receive in a U symbol. |
| vivo | Support the direction. For the sub-bullet, we understand the concern from companies that in reality, the legacy UE may not fully support or be tested for the flexible symbols. And this may impact on the signaling design. So is it better to modify the sub-bullet as below: - Further study whether or not That the legacy UEs may not fully support UL transmission and DL reception in symbols with undetermined UL/DL direction (i.e., flexible symbols) should be taken into account. |
| Nokia, NSB | Still confusing. Of course, legacy UEs can only transmit UL in UL-only symbols or in flexible symbols. But why “in UL subband in UL or flexible symbols”? First of all, it does not make sense to speak of UL subband in UL symbols. Also, even with SBFD enabled, the gNB may decide to configure a flexible symbol as an UL symbol, in which case a legacy UE should not be restricted to transmit in UL subband in flexible symbols. Similar issue for legacy UE to receive DL. |
| Huawei, HiSilicon | Even though we are fine with the intention of the proposal, we think it is not accurate to say that the gNB can schedule legacy UE in UL subband in SBFD symbols since the symbol may be configured as DL for legacy UEs. |
| Ericsson | Support. We are fine with vivo’s update to the sub-bullet |
| Intel | We support the intension of main bullet of the proposal, with the understanding the symbol (UL, or DL, or SBFD) and subband is from gNB's point of view while legacy UE can only see legacy UL/DL/F symbol without knowing which symbol is SBFD symbol and the frequency location of the subband. Regarding the sub-bullet, as commented for proposal 2-2b, no need of the sub-bullet. Furthermore, it is confusing, what does “in symbols with undetermined UL/DL direction” mean. For example, if in a semi-static F symbol, there is SSB, is it a symbol with undetermined UL/DL direction ? |
| Xiaomi | From our understanding, the purpose is to conclude that legacy UE behavior is not impacted due to the subband. We are OK with the intention. However, there is some confusions on the current wording. For example, the UL subband is typically configured in DL symbols. In this case, Gnb cannot schedule the legacy UE to transmit uplink within the UL subband. The main bullet can be rewording in a simplified formula, like: The legacy UE behavior on TDD carrier is not impacted by SBFD operation… |
| CMCC | We support this proposal. Regarding the sub-bullet, it is very important key issue to be considered in the SBFD configuration signalling design, e.g., whether the time/frequency resource configuration of subband is transparent to UE or not, whether a common signalling is used for both legacy UEs and SBFD capable UEs |
| ZTE | Similar view as Huawei. |
| ASUSTeK | We agree that legacy system supports some sort of “subband operation” in terms of “flexible symbols” and “network scheduling of frequency resource”. While it also true that this cannot be done in full flexibility as that is enjoyed by Rel-18 duplex enhanced UE, e.g. it’s for sure that legacy UE could not transmit on symbol indicated. If the main focus is on “flexible symbol” only, it’s unclear what needs to be further study given the fact disclosed by the main bullet. |
| Samsung | The intention of the proposal is, by gNB’s configuration/signaling, a legacy UE can transmit UL signal in a time-frequency resource intended to UL subband if the time-frequency resource overlaps with flexible symbol. No need to mention UL symbol and DL symbol since it is evident that the legacy UE cannot receive or transmit on UL symbol or DL symbol, respectively. Existing RAN1 specification supports gNB to schedule legacy UEs to transmit UL in UL-only symbols or in a time-frequency resource intended for UL subband in symbols with SBFD operation at gNB side and to receive DL in DL-only symbols or in a time-frequency resource intended for DL subband in symbols with SBFD operation at gNB side. We are ok to keep the sub-bullet. But, from RAN1 specification point of views, isn’t it clear? This sub-bullet should be for the issue on “test” in legacy UEs, not RAN1 specification. |
| Sony | Support |

Proposal 4-1:

Study potential enhancements for UE collision handling between UL and DL.

| Company | Comment |
| --- | --- |
| New H3C | We are fine with this proposal. |
| Apple | Tends to agree but the scope is large. Who resolves the collision, is it gNB or victim or aggressive UE? |
| vivo | For moderator’s following comments: “From moderator’s point of view, it should be clear that enhancements/new design do not apply to legacy UE so even without “SBFD capable UE”, there should be no confusion that the proposal may apply to legacy UE.” Does above proposal also apply to legacy UE? |
| Moderator | Sorry for the confusion. What I intended to say is that the proposal does not apply to legacy UE even without ‘SBFD capable UE’. Or in other word, the proposal should not be understood as applicable to legacy UE. |
| Nokia, NSB | Is it for SBFD capable UE or also for legacy UE? We support it to be studied for SBFD capable UE. |
| Lenovo | Support |
| Huawei, HiSilicon | Support and agree this can be applicable to both legacy UE (not necessarily other SBFD features) and SBFD capable UEs. |
| Ericsson | We think there would be no harm to clarify that the proposal applies to SBFD capable UEs. We also think there can be a collision between a scheduled/configured UL or DL signal with a guardband, hence this should also be considered in the study. Suggest the following update to capture this: Study potential enhancements for UE collision handling between UL and DL and between UL or DL and a guardband. |
| Intel | In our view, how to handle the collision is from UE’s perspective. Since legacy UE can not identify SBFD symbol, any enhancement for collision handling for SBFD is only for SBFD capable UE. With this understanding, we support the proposal. |
| InterDigital | Support the proposal |
| Xiaomi | Support. |
| CMCC | We also think this issue is for SBFD capable UE only |
| ZTE | OK |
| ASUSTeK | Support the proposal |
| Samsung | We support the proposal. At least for legacy UEs, gNB should take into account the potential DL/UL collision since the legacy UEs cannot know the presence of SBFD operations. For SBFD-capable UEs, we are fine to study further DL/UL collision handling rule. |
| Spreadtrum | We do not support. It can be postponed until non-transparent SBFD is agreed. |
| Sony | Support |

Proposal 5-1:

CLI handling schemes which are common for both SBFD and dynamic/flexible TDD are discussed in agenda item 9.3.3.

CLI handling schemes specific for SBFD are discussed in agenda item 9.3.2.

| Company | Comment |
| --- | --- |
| New H3C | We are fine with this proposal. |
| vivo | Support |
| Nokia, NSB | We suggest to update it as: Inter-subband CLI should be discussed under 9.3.2, and Intra-subband CLI should be discussed under 9.3.3. |
| Lenovo | Fine with the proposal. |
| Huawei, HiSilicon | Support |
| Ericsson | Support the proposal. We don’t think the proposal should be changed to specify inter- and intra-subband CLI, since when coexistence with legacy is considered (cochannel or adjacent channel), there are no subbands. |
| Intel | We share similar view with NOKIA. Instead of categorizing from perspective of schemes/solutions, it may be better to categorize CLI type. |
| Xiaomi | Support. |
| CMCC | We support this proposal which is aligned with the category of endorsed TR skeleton. 6.3 Inter-gNB and inter-UE CLI handling schemes Editor’s note: This section captures the potential inter-gNB and inter-UE CLI handling schemes that are specific for SBFD. 8.1 Inter-gNB and inter-UE CLI handling schemes Editor’s note: This section captures the potential inter-gNB and inter-UE CLI handling schemes that are specific for dynamic TDD and schemes that are common for both SBFD and dynamic/flexible TDD. |
| Samsung | We agree with CMCC. |
| Spreadtrum | Support |
| Sony | We would prefer that anything that has subband element and common to dynamic/flexible TDD should be studied in AI 9.3.2. |

- Proposals for GTW session on May 16th

Updated Proposal 2-1a:

Prioritize SBFD operation within a TDD carrier.

- SBFD operation across different carriers within a TDD band (intra-band CA with different TDD UL/DL configurations on different CCs) can be considered with lower priority.

Updated Proposal 2-2a:

Study schemes of UL subband in legacy DL and flexible symbols configured by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated.

- Note: Legacy DL and flexible symbols are from gNB’s perspective

Updated Proposal 2-2b (for conclusion):

For discussion purpose, SBFD symbols is defined as symbols with subbands that gNB would use for SBFD operation.

- Note: it does not imply a new symbol type (in addition to DL, UL and flexible) is introduced.

Updated Proposal 2-3a:

A subband consists of a set of consecutive RBs for the same transmission direction within a TDD carrier.

Updated Proposal 2-3b:

For SBFD operation within a TDD carrier, at least consider only one UL subband and up to two DL subbands within the carrier from gNB’s perspective.

- FFS more than one UL subband within the carrier

Updated Proposal 2-5a:

The time and frequency location of subbands are configurable at gNB side (i.e. time and frequency location of subbands are not fixed or predefined in the specification).

- Note that whether the time and/or frequency location of subbands are informed to UE is separately discussed.

Updated Proposal 3-1a:

Existing RAN1 specification supports gNB to schedule legacy UEs to transmit UL in UL-only symbols or in a time-frequency resource intended for UL subband in symbols with SBFD operation at gNB side and to receive DL in DL-only symbols or in a time-frequency resource intended for DL subband in symbols with SBFD operation at gNB side.

- Further study whether or notThat the legacy UEs may not fully support UL transmission and DL reception in symbols with undetermined UL/DL direction (i.e., flexible symbols) should be taken into account.

Proposal 4-1:

Study potential enhancements for UE collision handling between UL and DL at UE side.

Proposal 5-1:

CLI handling schemes which are common for both SBFD and dynamic/flexible TDD are discussed in agenda item 9.3.3.

CLI handling schemes specific for SBFD are discussed in agenda item 9.3.2.

- Proposals for GTW session on May 20th

Proposal 2-5c:

The time and frequency location of subbands within a TDD carrier are not fixed or predefined in the specification.

- Subject to any RAN4 guidance on minimum or maximum subband and guardband size and subband location within TDD carrier.

- Note that whether the time and/or frequency location of subbands are informed to UE is separately discussed.

Proposal 3-1c:

Existing RAN1 specification supports gNB to schedule legacy UEs to transmit UL in a time-frequency resource intended for UL subband at gNB side and to receive DL in a time-frequency resource intended for DL subband at gNB side.

- A legacy UE is not expected to transmit UL in a DL symbol from UE perspective or to receive DL in an UL symbol from UE perspective according to existing RAN1 specification.

- Further study whether or not legacy UEs fully support UL transmission and DL reception in symbols with undetermined UL/DL direction (i.e., flexible symbols). The SBFD design should consider that the legacy UEs may not fully support UL transmission and DL reception in symbols with undetermined UL/DL direction (i.e., flexible symbols) in the existing commercial deployments.

Proposal 4-1b:

Study potential enhancements for collision handling between UL and DL for SBFD aware UE supporting SBFD.

Proposal 2-2c:

At least study SBFD operation with UL subband in DL and flexible symbols configured by tdd-UL-DL-ConfigurationCommon and/or tdd-UL-DL-ConfigurationDedicated if provided.

Proposal 2-3c:

For SBFD operation within a TDD carrier in downlink and flexible symbols, at least consider one UL subband and up to two DL subbands within the carrier from gNB’s perspective.

- FFS more than one UL subband within the carrier

- FFS: Case of SBFD operation in uplink symbols

- Contact person

Please provide the information of the contact person in the following table to facilitate the discussions.

| Company | Name | Email address |
| --- | --- | --- |
| Sony | Shin Horng Wong | shinhorng.wong@sony.com |
| InterDigital | Jonghyun Park | jonghyun.park@interdigital.com |
| Sharp | Tomoki Yoshimura | yoshimurat@sharplabs.com |
| Qualcomm | Muhammad Abdelghaffar | mabdelgh@qti.qualcomm.com |
| New H3C | Lei Zhou | zhou.leih@h3c.com |
| New H3C | Lei Kong | Kong.lei@h3c.com |
| vivo | Lihui Wang | wanglihui@vivo.com |
| NEC | Pravjyot Singh Deogun | pravjyot.deogun@emea.nec.com |
| Xiaomi | Lei Wang | wanglei25@xiaomi.com |
| OPPO | Wenfeng Zhang | zhangwenfeng@oppo.com |
| Ericsson | Stephen Grant | stephen.grant@ericsson.com |
| Spreadtrum | Huan Zhou | Huan.Zhou@unisoc.com |
| CATT | Yanping Xing | xingyanping@catt.cn |
| Panasonic | Tomoya Nunome | nunome.tomoya@jp.panasonic.com |
| Intel | Yi Wang | yi5.wang@Intel.com |
| ITRI | Jen-Hsien Chen | itriA40175@itri.org.tw |
| Lenovo | Hyejung Jung | hyejung@motorola.com |
| ETRI | Hoondong Noh | hoondong.noh@etri.re.kr |
| ZTE | Xingguang WEI | wei.xingguang@zte.com.cn |
| Samsung | Marian Rudolf Kyungjun Choi | m.rudolf@partner.samsung.com kyungj.choi@samsung.com |
| CMCC | Tuo Yang Fei Wang | yangtuo@chinamobile.com wangfei@chinamobile.com |
| DOCOMO | Qiping Pi | piqp@docomolabs-beijing.com.cn |
| WILUS | David (Geunyoung) Seok | david.seok@wilusgroup.com |
| CEWiT | Priyanka Dey | priyanka@cewit.org.in |
| Nokia, NSB | Jingyuan Sun | Jingyuan.sun@nokia-sbell.com |
| Huawei, HiSilicon | Xinghua Song | songxinghua@huawei.com |
| MediaTek | Mohammed Al-Imari | Mohammed.Al-Imari@mediatek.com |
| LG Electronics | Hyunsoo Ko | hyunsoo.ko@lge.com |
| SK Telecom | Sanghoon Cho | seanc.cho@sk.com |
| KDDI | Masahito Umehara | ma-umehara@kddi.com |

- Reference

- RP-220633		Revised SID: Study on evolution of NR duplex operation	CMCC

- R1-2203157		Discussion on subband non-overlapping full duplex	Huawei, HiSilicon

- R1-2203204		Discussion of subband non-overlapping full duplex	ZTE

- R1-2203215		Discussion for subband non-overlapping full duplex	New H3C Technologies Co., Ltd.

- R1-2203328		Discussion on subband non-overlapping full duplex	Spreadtrum Communications

- R1-2203459		Discussion on subband non-overlapping full duplex	CATT

- R1-2203558		Discussion on subband non-overlapping full duplex	vivo

- R1-2203732		Adjacent Channel Interference in non-overlapping subband Full Duplex TDD operations	Sony

- R1-2203815		Discussion on subband non-overlapping full duplex	xiaomi

- R1-2203904		Subband non-overlapping full duplex for duplex evalution	Samsung

- R1-2203945		Discussion on subband non-overlapping full duplex	NEC

- R1-2204022		Discussion on subband non-overlapping full duplex	OPPO

- R1-2204054		Subband non-overlapping full duplex	SHARP Corporation

- R1-2204069		Discussion on subband non-overlapping full duplex	InterDigital, Inc.

- R1-2204107		Subband non-overlapping full duplex	Ericsson

- R1-2204156		Discussion on subband non-overlapping full duplex	Panasonic

- R1-2204245		Views on subband non-overlapping full duplex	Apple

- R1-2204304		Discussion on subband non-overlapping full duplex	CMCC

- R1-2204380		Discussion on subband non-overlapping full duplex	NTT DOCOMO, INC.

- R1-2204412		Discussion on sub band non-overlapping full duplex	CENC

- R1-2204423		Subband non-overlapping full duplex	Lenovo

- R1-2204431		On subband non-overlapping full duplex for NR	Nokia, Nokia Shanghai Bell

- R1-2204441		Discussion on sub-band non-overlapping full duplex	ITRI

- R1-2204530		Study on Subband non-overlapping full duplex	LG Electronics

- R1-2204550		Discussion on sub-band non overlapping full duplex	WILUS Inc.

- R1-2204637		Introduction of subband non-overlapping full duplex	ASUSTeK

- R1-2204651		Discussions on subband non-overlapping full duplex enhancements	ETRI

- R1-2204722		Discussion on subband non-overlapping full duplex for NR	MediaTek Inc.

- R1-2204751		Discussion on subband non-overlapping full duplex	CEWiT

- R1-2204800		Discussions on subband non-overlapping full duplex	Intel Corporation

- R1-2204866		Considerations for subband non-overlapping full duplex	Charter Communications

- R1-2205031		Feasibility and techniques for Subband non-overlapping full duplex	Qualcomm Incorporated

- R1-2205187		TR 38.858 skeleton for study on evolution of NR duplex operation	CMCC