3GPP TSG RAN WG1 #109-e								          R1-2205524

e-Meeting, May 9th – 20th, 2022

Agenda item:	8.5.1

Source: 	Moderator (Ericsson)

Title: 	Moderator Summary for [109-e-R17-ePos-03] maintenance on accuracy improvements for UL-AoA and DL-AoD positioning solutions

Document for:	Discussion and Decision

## Introduction

This summary documents the email discussion for the maintenance of accuracy improvements for UL-AoA and DL-AoD positioning solutions, as per the following chairman decision:

[109-e-R17-ePos-03] Email discussion under 8.5.1 for maintenance on accuracy improvements for UL-AoA and DL-AoD positioning solutions, for issues 2-2, 3-1, 3-2, 3-3, 3-5, 3-6, 3-8, 3-10 and 3-15 in R1-2205097 – Florent (Ericsson)

- 1st check point: May 13 (any RRC impact by May 12)

- Final check point: May 18

## Discussion

## Issue #2-2 LS response for Rx diversity of first path UL SRS RSRPP

Options for reporting of UL SRS RSRPP Rx diversity were agreed in RAN1 #108e as follow:

Agreement Apply the following changes to the definition for UL SRS-RSRPP in the previous agreement: Definition UL SRS reference signal received path power (UL SRS-RSRPP) is defined as the power of the received UL SRS signal configured for the measurement at the i-th path delay of the channel response, where UL SRS-RSRPP for 1st path delay is the power corresponding to the first detected path. - Working assumption: For frequency range 1, the reference point for the UL SRS-RSRPP shall be the antenna connector of the gNB. For frequency range 2, UL SRS-RSRPP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. - FFS: For frequency range 1 and 2, if receiver diversity is in use by the gNB, the reported UL SRS-RSRPP value shall not be lower than the corresponding UL SRS-RSRPP of any of the individual receiver branches - FFS: Note: First and additional paths RSRP, when provided in the same report, use the same RX branch(es) selected for the first arrival path and for the UL SRS-RSRP if the UL SRS-RSRP is reported - FFS: whether/how to capture the note in the specifications Note: The following two options are supported by gNB to LMF: - Option 1 (RX diversity for the first path UL SRS-RSRPP) - The same RX branch(es) as applied for the first path UL SRS-RSRPP measurements are used for the additional paths UL SRS-RSRPP measurements if those are provided together - For frequency range 1 and 2, if receiver diversity is in use by the gNB for UL SRS-RSRPP measurements, then reported UL SRS-RSRPP value for the first path shall not be lower than the corresponding UL SRS-RSRPP for the first path of any of the individual receiver branches - Option 2 (RX diversity for UL SRS-RSRP) - The same RX branch(es) as applied for UL SRS-RSRP measurements are used for UL SRS-RSRPP measurements (i.e., the first and additional paths UL SRS-RSRPP if those are provided)

In R1-2203040, RAN3 sent the following question for clarification:

RAN3 will define the value range of the UL-SRS-RSRPP measurement for the i-th path as an INTEGER (0..126). Some companies question whether the information on the RX Diversity options, captured as a note in the RAN1 agreement, need to be signalled to LMF along with the UL-SRS-RSRPP measurement. RAN1 to feedback if information on the Rx Diversity options needs to be signalled to LMF and to comment on the UL-SRS-RSRPP value range, if needed?

It is thus proposed to discuss whether the Rx diversity option for UL SRS RSRPP imply any signaling of the options.

## First round of discussion

Question 2.1.1: should the use of option 1 or option 2 be signaled to the LMF when reporting UL SRS RSRPP?

Companies are encouraged to respond in the table below:

Question 2.1.1:

| company | comment |
| --- | --- |
| vivo | In our view, there is no information on the Rx Diversity options needs to be signalled to LMF. If UL SRS-RSRP is not reported, only Option 1 can be selected; but if UL SRS-RSRP is reported, only Option 2 can be selected. In addition, regarding the issue of the UL-SRS-RSRPP measurement, we has no comments on the value range of the UL-SRS-RSRPP measurement for the i-th path. |
| ZTE | We don’t think explicitly signaling is needed. Option 2 is used when RSRP is reported. Otherwise, Option 1 is used. |
| Huawei, HiSilicon | We think it should be reported to allow flexibility at gNB. There should be no such restriction that when both are reported, the Rx branch of SRS-RSRPP should be the same as that of SRS-RSRP. |
| Nokia/NSB | We don’t see the necessity to introduce additional signaling as two options are clearly differentiated by the reporting content. |
| OPPO | We do not think it is needed to introduce signalling of those two options. Those two options are used for different measurement use case, not options that can be selected. |
| CATT | We prefer that no signaling is needed to LMF on the Rx Diversity options. Current Note seems to be clear on the selection of the Rx Diversity options when gNB reporting UL SRS RSRPP to LMF. |
| Qualcomm | We don’t see the necessity to introduce signaling |
| Ericsson | We don’t think the signalling is necessary. |

Since we have only one company supporting the introduction of the additional signalling, and 6 other commenting companies opposing it,  it seems unlikely introducing signalling would be agreeable.  We propose thus to respond to RAN3 that no signalling of the options is needed.

## Second  round of discussion

Proposal 2.1.2: in response to RAN3’s question in the LS R1-2203040:  information on the Rx Diversity options does not needs to be signalled to LMF.

Companies are encouraged to respond in the table below:

Question 2.1.2:

| company | comment |
| --- | --- |
| Huawei, HiSilicon | We disagee with the statement that when RSRP is reported, RSRPP should follow the branches of RSRP. There was no agreement substantiating the statement, which is limiting gNB implementation. We could accept it being without signaling, but it should be clear that when UL SRS-RSRP is reported, whether to use Option1 or Option 2 for UL SRS-RSRPP is up to gNB implementation. |
| Qualcomm | We are OK with saying that there is no signaling, but add a note that it is up to gNB implementation as Huawei, HiSilicon suggested |
| Nokia/NSB | Support. We also think it is up to gNB implementation and it is okay to add a note. |
| vivo | We are okay with FL proposal |
| Ericsson | Support, OK with the proposed note. |
| LGE | We are agree with FL’s proposal. |

## Third  round of discussion (closed)

The issue is stable, and Huawei  has provided a way forward that should be able to resolve the issue, let’s try the revised proposal as follow:

Proposal 2.1.3: in response to RAN3’s question in the LS R1-2203040:  information on the Rx Diversity options does not needs to be signalled to LMF.

- Note: when UL SRS-RSRP is reported, whether to use Option1 or Option 2 for UL SRS-RSRPP is up to gNB implementation.

Companies are encouraged to respond in the table below:

Proposal 2.1.3:

| company | comment |
| --- | --- |
| FL | The following agreement was captured in the chair notes: Agreement In response to RAN3’s question in the LS R1-2203040: information on the Rx Diversity options does not needs to be signalled to LMF. · Note: when UL SRS-RSRP is reported, whether to use Option1 or Option 2 for UL SRS-RSRPP is up to gNB implementation. |

## Issue #3-1 reporting of PRS RSRPP and PRS RSRP for the first path from different resources

In [1], proposal 5 suggests supporting reporting RSRPP and RSRP for the first path from different resources:

Proposal 5: For DL-AoD, introduce a separate DL PRS resource set ID and resource ID for the first RSRPP measurement when the resource is not same as that for the first RSRP measurement. ● The additional RSRPP measurement still takes (0..30) as the reporting range in reference to the first RSRPP measurement.

## First round of discussion

We can start by discussing the proposal as it is in [1] and see what update it may need:

Proposal 2.2.1: For DL-AoD, introduce a separate DL PRS resource set ID and resource ID for the first RSRPP measurement when the resource is not same as that for the first RSRP measurement.

- The additional RSRPP measurement still takes (0..30) as the reporting range in reference to the first RSRPP measurement.

Companies are encouraged to respond in the table below:

Proposal 2.2.1

| company | comment |
| --- | --- |
| vivo | Alternative, we can take (0…60) for double side differential RSRPP reporting (i.e. -30 to 30 with 1 dB), and then no additional set and resource ID that needs to be introduced |
| ZTE | Not necessary to introduce a new feature here. The differential PRS-RSRP from -30 dB to 30 dB can be used in such case. Here is our suggestion: Proposal 2.2.1: For DL-AoD, introduce a separate DL PRS resource set ID and resource ID for the first RSRPP measurement when the resource is not same as that for the first RSRP measurement. Tthe additional RSRPP measurement still takes from -30 dB to 30 dB (0..30) as the reporting range in reference to the first RSRPP measurement |
| Qualcomm | No support this enhancement. Our understanding is that the double sided PRSR RSRPP will be fine. |
| Huawei, HiSilicon | We support this proposal. Anyway if double-side differential RSRPP reporting is used, we should send LS to RAN2 and RAN4 to update their spec. |
| Nokia/NSB | We also think double sided RSRPP reporting might be okay. The modified proposal from ZTE is okay. |
| OPPO | Not support to agree on new differential RSRP range, which shall be decided by RAN4. If RAN1 decided that that differential RSRPP could be out of range of current differential RSRP definition in RAN4, we shall send the LS to notify RAN4 about that and then leave it to RAN4 to “fix” it. |
| CATT | This issue seems to be related to RAN4. We prefer to send an LS to RAN4 and let RAN4 to determine whether the double-side differential RSRPP reporting should be used. |
| vivo 2 | Based on the following LS in R4-2202780, we would like to notice that 10.1.24.3.2 includes 10.1.24.3.2-1 for single -side differential differential RSRP, and 10.1.24.3.2-2 for double-side differential RSRP. So, we think there are no additional work for RAN4. - PRS-RSRPP may be reported by reusing absolute and differential PRS-RSRP measurement report mapping tables in TS38.133 clause 10.1.24.3.1 and 10.1.24.3.2 respectively. - When differential reporting is used, PRS-RSRPP is reported as the difference in dB with respect to a reference measurement. RAN4 understands that it is up to RAN1/2 to decide what reference measurement would be for the PRS-RSRPP differential reporting.. |
| Huawei, HiSilicon2 | To vivo: If you look at TS 38.133 clause 10.1.24.3.2, we believe if we decided to have double sided RSRPP reporting for DL-AoD, we should kindly inform them that the table selection for RSRP in Rel-16 should not be reused for RSRPP in Rel-17. ##### 10.1.24.3.2 Differential Report Mapping for PRS-RSRP Measurement The reporting range of differential PRS-RSRP is defined from -30 dB to 0 dB with 1 dB resolution when nr-DL-AoD-RequestLocationInformation message is received. The mapping of measured quantity is defined in Table 10.1.24.3.2-1. The range in the signalling may be larger than the guaranteed accuracy range. The reporting range of differential PRS-RSRP is defined from -30 dB to 30 dB with 1 dB resolution when nr-DL-TDOA-RequestLocationInformation or nr-Multi-RTT-RequestLocationInformation is received. The mapping of measured quantity is defined in Table 10.1.24.3.2-2. The range in the signalling may be larger than the guaranteed accuracy range or the range supported by the UE receiver for differentail RSRP measured on different PRS resources in frequency domain at the same time. |

## Second round of discussion  (closed)

We can summarize the opinions are follow:

- Double-sided RSRPP (using +-30dB range centered at 0dB) range is used as the reporting range: ZTE, Nokia

- No agreement needed: Qualcomm, Vivo

- Agree with proposal: Huawei

- Request for changes to the range should be send in LS to RAN4: Oppo, CATT, Huawei,

Based on the last comment from Huawei, it seems that an agreement is needed. We do not have a formal agreement on double-sided range for RSRPP for AOD, and it is needed to be able to impact the ran4 specification. We can re-use the wording from the ZTE comment to revise the proposal:

Proposal 2.2.2: For DL-AoD, the additional RSRPP measurement takes from -30 dB to 30 dB as the reporting range in reference to the first RSRPP measurement

- Send an LS to RAN2 informing of the agreement.

Companies are encouraged to respond in the table below:

Proposal 2.2.2

| company | Comment |
| --- | --- |
| Huawei, HiSilicon | LS should also be sent to RAN2. |
| Qualcomm | Support. Indeed, it makes sense the LS to go to RAN2 also. |
| CATT | OK with the proposal and also fine with sending it to RAN2. |
| Nokia/NSB | Support with sending LS to RAN2. |
| vivo | Support |
| Ericsson | Support. Agree that the LS is better suited for RAN2. |
| FL | the proposal is updated to correct RAN4 to RAN2. |
| LGE | Agree. |
| Huawei, HiSilicon2 | Just to clarity, we think BOTH WGs should be informed.2 |
| FL | We can update the proposal to include RAN4: Proposal 2.2.2: For DL-AoD, the additional RSRPP measurement takes from -30 dB to 30 dB as the reporting range in reference to the first RSRPP measurement - Send an LS to RAN2 and RAN4 informing of the agreement. |
| FL | The following agreement was captured during the email discussion: Agreement For DL-AoD, the additional RSRPP measurement takes from -30 dB to 30 dB as the reporting range in reference to the first RSRPP measurement - Send an LS to RAN2 and RAN4 informing of the agreement. |

## Issue #3-2 TP clarifying reception of the DL PRS-RSRPP in 38.214

The following TP was proposed in [4] as a clarification for 38.214:

TP2.3:

----------------Start of TP for TS38.214--------------------- #### 5.1.6.5 PRS reception procedure …… The UE may be configured to measure and report, subject to UE capability, up to 24 DL PRS-RSRP measurements on different DL PRS resources associated with the same dl-PRS-ID. When the UE reports DL PRS-RSRP measurements from one DL PRS resource set, the UE may indicate which DL PRS-RSRP measurements associated with the same higher layer parameter nr-DL-PRS-RxBeamIndex [17, TS 37.355] have been performed using the same spatial domain filter for reception if for each nr-DL-PRS-RxBeamIndex reported there are at least 2 DL PRS-RSRP measurements associated with it within the DL PRS resource set. When the UE reports multiple DL PRS-RSRP measurements for a DL PRS resource, the multiple DL PRS-RSRP measurements, which can be associated with the same or different higher layer parameter nr-DL-PRS-RxBeamIndex, may have the same or different timestamps. The UE may be configured to measure and report, subject to UE capability, up to 24 DL PRS RSRPP for the first detected path on different DL PRS resources associated with the same dl-PRS-ID. …… ----------------End of TP for TS38.214---------------------

## First round of discussion  (closed)

Companies are encouraged to comment on the TP2.3 in the table below:

TP2.3

| company | comment |
| --- | --- |
| vivo | Not essential The added description doesn’t regulate anything, and we think it is a note that is used for explanation and may not necessarily be added to the specification |
| ZTE | It is not a critical issue. |
| Qualcomm | Seems unnecessary, the text is fine as is. |
| Nokia/NSB | It seems not essential issue, and we prefer to keep the current text. |
| CATT | Support the TP, in order to align the spec descriptions with RAN1 previous agreement as follows, In RAN1#106bis-e meeting, the following agreements were achieved for DL-AoD enhancement: Agreement: The agreement from RAN1#106-e on the number of DL PRS RSRP measurements per TRP is extended as follows: - For UE-A DL-AOD, support reporting up to N DL PRS RSRP measurements per TRP, where N is UE capability and candidate values include {16,24}. - For UE-A DL-AOD, support reporting up to M first path PRS RSRP measurements per TRP, where M is a UE capability - FFS: Values of M. Candidate values include {2,4,8,16,24}. - FFS: Whether M is always equal to N - Note: Multiple RSRPs corresponding to same or different Rx Beam index should be able to be reported for a given PRS resource for same or different timestamps. - Note: the maximum number of DL PRS RSRP associated with the same Rx beam index is up to the UE implementation According to the above agreement, for a given PRS resource, multiple DL PRS RSRPs could be reported, which may correspond to same or different Rx Beam index and same or different timestamps. This feature should be specified in TS 38.214 for a clear description on UE reporting. |
| Huawei, HiSilicon | We wonder whether this multiple RSRP reporting for the same target PRS resource is contradicting with the current RSRP receiver diversity condition. Definition; DL PRS reference signal received power (DL PRS-RSRP), is defined as the linear average over the power contributions (in [W]) of the resource elements that carry DL PRS reference signals configured for RSRP measurements within the considered measurement frequency bandwidth. For frequency range 1, the reference point for the DL PRS-RSRP shall be the antenna connector of the UE. For frequency range 2, DL PRS-RSRP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. For frequency range 1 and 2, if receiver diversity is in use by the UE, the reported DL PRS-RSRP value shall not be lower than the corresponding DL PRS-RSRP of any of the individual receiver branches.. Applicable for; RRC_CONNECTED, RRC_INACTIVE. |
| FL | Given the lack of support, we can close this proposal. |

## Issue #3-3 maximum number of PRS resources in a subset

In [6] the authors propose to limit the maximum number of PRS resources in a subset to 8. Previous discussion on the issue during RAN1#108e did not converge.

## First round of discussion   (closed)

We can start the discussion with the proposal in [6].  In order to respond to the questions asked during the UE feature discussion in RAN1#108e, we can also discuss how each PRS resource is identified. At the end of RAN1#108e, the latest version of the proposal was to include PRS resource set ID and resource ID.

Proposal 2.4.1:

- The maximum number of PRS resources in a PRS subset is 8.

- The number of resources within a subset transmitted to UE should not be larger than the maximum number of resources that UE reporting

- Each PRS resource in the PRS subset is identified with a PRS resource set ID and PRS resource ID

Companies are encouraged to comment on proposal 2.4.1 in the table below:

Proposal 2.4.1:

| company | comment |
| --- | --- |
| vivo | Okay |
| ZTE | We prefer not to restrict the maximum number of PRS resources in a PRS subset. That is 24 should be used by default. Actually, 24 is used in RAN2’s latest CR. Hence, RAN1 does not need to do anything. |
| Qualcomm | We think that it can 24, without a need of UE capability. |
| Huaawei, HiSilicon | Same view as ZTE. |
| Nokia/NSB | It is unclear why we need to restrict the maximum number. We respect the latest RAN2 CR. |
| OPPO | The proposal is not needed. |
| CATT | We share the same view with ZTE. It seems that it is not necessary to further restrict the maximum number of PRS resources in a PRS subset. |
| Ericsson | Agree with ZTE’s view. |
| FL | Based on the lack of support, we don’t think we will reach a compromise to accept the proposal. Therefore we can close this issue for this meeting. |

## Issue #3-5 Reference for additional path DL PRS RSRPP

In [9], the reference for additional paths DL PRS RSRPP is discussed.  It is proposed to support differential reporting of additional path DL PRS RSRPP using the DL PRS RSRPP of the first path for the same resource.  The proposal corresponds to alt.2 of the following proposal in RAN1#108:

Proposal 4.1-C - Support reporting differential RSRPP for the PRS-RSRPP measurement in DL-TdoA and multi-RTT for at least the additional paths. - Alt. 1: The reference for all the additional paths PRS-RSRPPs is the PRS-RSRPP of the first path associated with the nr-RSTD-r16 or nr-UE-RxTxTimeDiff-r16 measurement. - Alt. 2: The reference for the additional path PRS-RSRPP is the PRS-RSRPP of the first path of the same PRS resource as the additional path PRS-RSRPP.

## First round of discussion

We can continue the discussion from RAN1#108e. Both alternatives in the proposal from RAN1#108e can work. Looking at the current use of differential reporting in LPP, alt-1 is closer to what is done for additional path and additional measurements in 37.355.

Proposal 2.5.1: Support reporting differential RSRPP for the PRS-RSRPP measurement in DL-TdoA and multi-RTT for at least the additional paths.

  - Alt. 1: The reference for all the additional paths PRS-RSRPPs is the PRS-RSRPP of the first path associated with the nr-RSTD-r16 or nr-UE-RxTxTimeDiff-r16 measurement.

  - Alt. 2: The reference for the additional path PRS-RSRPP is the PRS-RSRPP of the first path of the same PRS resource as the additional path PRS-RSRPP.

Companies are encouraged to comment their preferred alternative in proposal 2.5.1 in the table below:

Proposal 2.5.1:

| company | comment |
| --- | --- |
| ZTE | Prefer Alt.2. |
| Qualcomm | We have concerns for Alt. 1 as we discussed before and we think that Alt. 2 is the right way of using differential RSRPP: We should be using the same PRS resource as reference. . If we cannot converge, we are fine with absolute RSRPP reporting for the additional paths. A solution that will still work without any problem. |
| Huawei, HiSilicon | We have concern for Alt.2. OK to accept absolute RSRPP reporting for DL-TDOA and Multi-RTT. |
| Nokia/NSB | Prefer Alt.1. okay with absolute RSRPP reporting. |
| OPPO | We prefer Alt.2 if differential RSRPP is to be supported here since that Alt.1 would introduce out-of-range differential RSRPP value, which would impair the benefit of differential RSRP. If we can not reach consensus, we are ok with absolute RSRPP. |
| InterDigital | We prefer Alt. 2. |
| CATT | During the Round #4 Discussion for this issue in AI8.5.5 in RAN1#108-e (can refer to section 3.4.4 in R1-2202559), we observed that the reference as the strongest path RSRPP had been supported by CATT, Qualcomm, Huawei, HiSilicon, OPPO, LGE, so we prefer the proposal to be updated as follows for further discussion: - Updated Proposal 2.5.1: Support reporting differential RSRPP for the PRS-RSRPP measurement in DL-TdoA and multi-RTT for at least the additional paths. - Alt. 1: The reference for all the additional paths PRS-RSRPPs is the PRS-RSRPP of the strongest path associated with the nr-RSTD-r16 or nr-UE-RxTxTimeDiff-r16 measurement. - Alt. 2: The reference for the additional path PRS-RSRPP is the PRS-RSRPP of the strongest path of the same PRS resource as the additional path PRS-RSRPP. |
| Qualcomm | With regards to the proposal from CATT, we still support Alt. 2 |
| Ericsson | OK with alt2 |

## second round of discussion

based on the received comments, the majority supports alt2.  We can check if it is possible to reach a compromise:

Proposal 2.5.2: Support reporting differential RSRPP for the PRS-RSRPP measurement in DL-TdoA and multi-RTT for at least the additional paths.

  - (Alt. 2) The reference for the additional path PRS-RSRPP is the PRS-RSRPP of the first path of the same PRS resource as the additional path PRS-RSRPP.

Companies are encouraged to comment their preferred alternative in proposal 2.5.1 in the table below:

Proposal 2.5.2:

| company | comment |
| --- | --- |
| Huawei, HiSilicon | Do not support the proposal. In this case, we do not support differential reporting. |
| Qualcomm | Support the proposal |
| CATT | We can live with this proposal, if “the first path” is changed into “the strongest path”, as we mentioned in our previous comments. |
| Ericsson | OK with the proposal. To CATT: we prefer to keep PRS RSRPP of “the first path” because it is clearly defined in 215. Additionally, if the UE reports a NLOS PDP, the earliest path will not be the strongest and in that case it would mean that one of the additional path would have to be identified as reference for the other additional paths. |
| LGE | We are sorry for late response. Regarding both options, we are supportive of Alt.1 with a modified table. The reason is that not only Alt.2 has vulnerabilities of error propagation since FAP for additional measurement is reported by using differential reporting but also Alt.1 has less specification impact. But, if we can not reach consensus, we are ok with absolute RSRPP. |

## third round of discussion  (closed)

we don’t seem to be able to break the deadlock between alt1 and alt2. Since some companies voiced interest for absolute reporting as a fall back, let’s check if that is an acceptable alternative:

Proposal 2.5.3: Support reporting absolute RSRPP for the PRS-RSRPP measurement in DL-TdoA and multi-RTT for at least the additional paths.

Companies are encouraged to comment on  proposal 2.5.3 in the table below:

Proposal 2.5.3:

| company | comment |
| --- | --- |
| Qualcomm | Fine to go with that |
| Huawei, HiSilicon | Yes. |
| vivo | We wonder whether the additional measurement for the first path is absolute or not, if it is, we prefer to remove “at least the additional path” |
| CATT | We can live with this proposal as a compromised solution. |
| FL | Based on vivo’s comment Let’s remove “at least”: Proposal 2.5.3a: Support reporting absolute RSRPP for the PRS-RSRPP measurement in DL-TdoA and multi-RTT for at least the additional paths. |
| FL | The following was captured in the chair notes: Agreement Support reporting absolute RSRPP for the PRS-RSRPP measurement in DL-TdoA and multi-RTT for at least the additional paths. |

## Issue #3-6 TP for 38.214 regarding the AOD window

The following TP was proposed in [13] as a clarification for 38.214:

TP2.6

TP for TS 38.214: Reason for change: The text specificaton on expected DL-AoD/AoA is not complete Summary of change: Add text to specify the method of calculating range of expected DL-AoD/AoA. Consequences if not approved: Ambiguity in UE behavior.

5.1.6.5 PRS reception procedure <Unchanged parts are omitted> The UE may request to be provided with either expected DL-AoD/ZoD and uncertainty range(s) of expected DL-AoD/ZoD, or expected DL-AoA/ZoA and uncertainty range(s) of the expected DL-AoA/ZoA. The UE may be provided with expected DL-AoD/ZoD and uncertainty range(s) of the expected DL-AoD/ZoD with granularity of 1 degree. The UE may be provided with expected DL-AoA/ZoA and uncertainty range(s) of the expected DL-AoA/ZoA with granularity of 1 degree. The uncertainty range(s) of the expected DL-AoD/DL-AoA may be configured within [0, 60]. The uncertainty range(s) of expected DL-ZoD/DL-ZoA may be configured within [0, 30]. The UE may calculate the range of expected DL-AoD/DL-ZoD as (expected DL-AoD/ZoD – uncertainty range of expected DL-AoD/ZoD/2, expected DL-AoD/ZoD + uncertainty range of expected DL-AoD/ZoD/2) and may calculate the range of expected DL-AoA/DL-ZoA as (expected DL-AoA/ZoA – uncertainty range of expected DL-AoA/ZoA/2, expected DL-AoA/ZoA + uncertainty range of expected DL-AoA/ZoA/2) <Unchanged parts are omitted>

## First round of discussion   (closed)

Companies are encouraged to comment on the TP2.6 in the table below:

TP2.6

| company | comment |
| --- | --- |
| vivo | we prefer not to modify RAN1 specification, and the related description can refer to ExpectedRSTD and be added to the RAN2 specification |
| ZTE | OK with the TP |
| QC | OK with the TP, but not really a critical issue. |
| Huawei, HiSilicon | Prefer not to make RAN1 specification duplicated with RAN2 ASN.1 field description. |
| Nokia/NSB | We don’t see a strong necessity to capture this to RAN1 spec. |
| OPPO | We support the TP |
| CATT | We can live with the TP in principle. However, there are multiple “/” symbols with different meansings in the following descriptions, we prefer to use a formula instead of text descrtptions in the TP: -Current text: (expected DL-AoD/ZoD – uncertainty range of expected DL-AoD/ZoD/2 -Preferred formula: (φAOA – ΔφAOA/2, φAOA + ΔφAOA/2). |
| FL | Since there is no consensus, the TP cannot be adopted at the first checkpoint. We can check again at the final checkpoint. |
| Ericsson | We don’t think the TP content is necessary in RAN1 specs, since it’s only spelling out the ASN1 content that will be captured in RAN2 specs. |
| LGE | OK with the TP. |
| FL | The TP was not agreed, based on the lack of consensus. |

## Issue #3-8 Proposed correction to the PRS RSRPP definition (closed)

[1] and [24] observe that the definition of DL PRS RSRPP does not correctly capture the fact that the measurement should be performed at the first path delay in the following sentence

38.215:

DL PRS reference signal received path power (DL PRS-RSRPP), is defined as the power of the linear average of the channel response at the i-th path delay of the resource elements that carry DL PRS signal configured for the measurement, where DL PRS-RSRPP for the 1st path delay is the power contribution corresponding to the first detected path in time.

[1] propose to add the following sentence to complete the definition:

The channel response at the i-th path delay on a resource element corresponds to the response on the resource element of a shifted version of the channel where the i-th path delay is shifted to 0.

[24] proposes to reword the definition

DL PRS reference signal received path power (DL PRS-RSRPP) is defined as the power of the PRS channel response at the ith path delay, where - The PRS channel response is obtained by a time-domain transform of the resource elements that carry the received DL PRS signal configured for the measurement - DL PRS-RSRPP for 1st path delay is the power contribution corresponding to the first detected path in time.

## First round of discussion  (closed)

It is proposed to start the discussion by checking whether any change is needed and if so, what change should be implemented.

Question 2.7.1: should the DL PRS RSRPP in 38.215 be modified, and if so, which of the following option for changes is preferred:

- Alt 1: proposal in [1]

- Alt2: proposal in [24]

- Alt3: other proposal (please specify in your comment)

Companies are encouraged to comment on question 2.7.1 in the table below:

Question 2.7.1

| company | comment |
| --- | --- |
| vivo | From our side, we prefer to not really try to change it again since we spent a lot of time confirming it. |
| ZTE | We prefer not to spend time on this discussion. We see no critical issue with the existing agreements. |
| Qualcomm | Alt. 3: OK as is now |
| Nokia/NSB | If needed, prefer to send an LS to RAN4 to fix this issue. |
| OPPO | No change is needed. We did spend much effort on the definition and so far what in spec might be the best we can do. |
| CATT | We worry that it may be difficult to converge on this issue, so we prefer to keep current descriptions in the specs. |
| Ericsson | Either of alt. 1 or alt 2 would be fine, in order to make the definition make sense. Right now, we think it is impossible to derive the right formula from the definition. |
| FL | It seem unlikely a correction is agreeable, thus we suggest to close the discussion. |

## Issue #3-10 DL PRS RSRPP and Rx Diversity:

The Rx diversity options for DL PRS RSRPP were discussed during RAN1#108e in the context of DL AOD. The captured definition of the measurement did not include Rx diversity consideration since, in the context of DL AOD, no additional paths was considered.

[1] proposes to introduce an Rx branch set ID reported with the measurement when the UE uses multiple rx branches set. In [24] it is proposed to mirror the definition for rx diversity for SRS RSRPP in the PRS RSRPP definition.

## First round of discussion

We can start by checking whether there is a preferred alternative between the two proposals from [1][24]:

Proposal 2.8.1: For DL PRS RSRPP measured with Rx diversity, select between the following (if any):

- Alt1: For frequency range 1 and 2, if receiver diversity is in use by the UE for DL PRS-RSRPP measurements:

- The reported DL PRS-RSRPP value for the first and additional paths shall be provided for the same receiver branch(es) as applied for DL PRS-RSRP measurements, or

- The reported DL PRS-RSRPP value for the first path shall not be lower than the corresponding DL PRS-RSRPP for the first path of any of the individual receiver branches and the reported DL PRS-RSRPP for the additional paths shall be provided for the same receiver branch(es) as applied DL PRS-RSRPP for the first path.

Alt2: Support the following Rx diversity scheme for RSRPP reporting for DL-TDOA and Multi-RTT

- For both frequency range 1 and frequency range 2, if receiver diversity is in use by the UE, the reported RSRPP may be associated with an Rx branch set ID.

- The RSRPP measurements associated with the same Rx branch set ID for a TRP correspond to the same set of Rx branches.

Companies are encouraged to comment on proposal 2.8.1 in the table below:

Proposal 2.8.1

| company | comment |
| --- | --- |
| vivo | We prefer to Alt 1 and remove the second sub-bullet of Alt 1 since if the first bullet (“- The reported DL PRS-RSRPP value for the first and additional paths shall be provided for the same receiver branch(es) as applied for DL PRS-RSRP measurements”) is supported, it is difficult to ensure the DL PRS-RSRPP value for the first path of the same Rx branches of RSRP will be not lower than any individual receiver branches. In addition, based on the following RAN4 agreement, the same Rx branches are agreed upon, we don’t find enough benefits to add complexity to choose the Rx branches for RSRPP first and then measure the timing, especially considering RSRPP for timing-based positioning is only used to assist. RAN4 Agreements: - Same Rx branches as applied for PRS-RSRP measurement are used for path PRS-RSRP measurement |
| ZTE | We are OK with Alt.1 as it is a symmetrical design from UL. |
| Huawei, HiSilicon | As we explained our paper, when RSRP/RSRPP reporting is associated with TEG feature, it becomes a mess if we restrict the Rx branches. It becomes possible that some RSTD measurement associated with a TEG may never qualify to have a RSRP/RSRPP reporting, because the RSRP/RSRPP is lower than that of another Rx branch (TEG). |
| OPPO | We prefer Alt.1. The concern on Alt.2 is that the Rx branch is part of UE implementation. Alt.1 is the normal way to define measurement when Rx diversity is used at the receiver. |
| CATT | Support Alt.1. Alt.1 matches the corresponding descriptions of UL RSRP/RSRPP. |
| Qualcomm | We prefer Alt. 1 |
| Huawei, HiSilicon2 | We have strong concern over Alt.1 when it is associated with TEG reporting. We hope everyone understand Alt.1 means when supporting Alt.1; it means that some Rx panels/Rx will not have any RSRPP reporting at all. We do not mind the settle the gNB RSRPP reporting criterion to align with UE side if we go with Alt.2. From our side, Alt.2 give sufficient freedom at UE when selecting the branch(es) of RSRPP reporting. |
| ZTE2 | We understand Huawei concern on Alt.1 if we considering multiple panels corresponding to multiple Rx TEGs. However, if we restrict the Alt.1 applicalbe within either the first measurement or an additional measurement, then different TEGs/RX branches can still be used between the first measurement and an additional measurement. Here is our suggestion: Proposal 2.8.1: For DL PRS RSRPP measured with Rx diversity, select between the following (if any): - Alt1: For frequency range 1 and 2, if receiver diversity is in use by the UE for a DL PRS-RSRPP measurement elements (a first measurement or an additional measurement): - The reported DL PRS-RSRPP value for the first and additional paths shall be provided for the same receiver branch(es) as applied for DL PRS-RSRP measurements, or - The reported DL PRS-RSRPP value for the first path shall not be lower than the corresponding DL PRS-RSRPP for the first path of any of the individual receiver branches and the reported DL PRS-RSRPP for the additional paths shall be provided for the same receiver branch(es) as applied DL PRS-RSRPP for the first path. |
| vivo 2 | Based on the concern from Huawei for some Rx panels/Rx will not have any RSRPP reporting at all, we suggest to reuse AoD agreement for Rx branches,: Proposal 2.8.1: For DL PRS RSRPP measured with Rx diversity, select between the following (if any): - Alt1: For frequency range 1 and 2, if receiver diversity is in use by the UE for DL PRS-RSRPP measurements: - The reported DL PRS-RSRPP value for the first and additional paths shall be provided for the same receiver branch(es) as applied for DL PRS-RSRP measurements, |
| Huawei, HiSilicon | Reply to ZTE2: Honestly speaking, we do not see much difference. A Rx panel (Rx TEG) that has smaller RSRPP for a target PRS resource than the other panel will still not be reported. On “The reported DL PRS-RSRPP value for the first path shall not be lower than the corresponding DL PRS-RSRPP for the first path of any of the individual receiver branches”, we would clarify that in a single measurement element (first or additional measurement), there would be only a single first path RSRPP, and multiple additional path RSRPP. Then for this single RSRPP, it should be corresponding to the maximum across all Rx branches given the above text, so still some Rx branches are not qualified for RSRPP reporting. Reply vivo: I do not understand how aligning with DL-AoD would resolve the case, and what makes it worse is that it forces UE to report RSRP in order to report RSRPP and both are received by the same Rx branch, and the branch for RSRP still follows the existing receiver diversity restriction. |
| FL | it looks like we will not have an agreement by the first checkpoint. We can continue the discussion until the next checkpoint, since I don’t think there is ASN1 impact. |
| Huawei, HiSilicon | Since there is no consensus to adopt the change, and given the current TS 38.215 does not specify any requirement on the RSRPP Rx branch, we think it is should be natural to conclude this. Proposed conclusion: With regards to DL PRS-RSRPP reporting for DL-TDOA and Multi-RTT, the Rx branch corresponding to the reported DL PRS-RSRPP if the Rx diversity is in use by the UE is up to UE implementation. Send an LS to RAN4 for them to take it into account when defining the DL PRS-RSRPP performance requirement. |
| LGE | We are supportive of Huawei's latest proposed conclusion. |

## second round of discussion  (closed)

since there is no consensus, we can try to adopt the proposed conclusion proposed by Huawei above:

proposal 2.8.2: Proposed conclusion:

With regards to DL PRS-RSRPP reporting for DL-TDOA and Multi-RTT, the Rx branch corresponding to the reported DL PRS-RSRPP if the Rx diversity is in use by the UE is up to UE implementation.

Companies are encouraged to comment on the proposed conclusion in the table below:

Proposal 2.8.2

| company | comment |
| --- | --- |
| FL | The following was captured in the chair notes: Conclusion With regards to DL PRS-RSRPP reporting for DL-TDOA and Multi-RTT, the Rx branch corresponding to the reported DL PRS-RSRPP if the Rx diversity is in use by the UE is up to UE implementation. |

## Issue #3-15 TP clarifying reception of the DL PRS-RSRPP in 38.214

The following TP was proposed in [1] as a clarification for 38.214:

TP2.9:

================== Start of TP ================== The UE may be configured to measure and report, subject to UE capability, up to 24 DL PRS-RSRP measurements on different DL PRS resources associated with the same dl-PRS-ID. The UE may be configured to measure and optionally report via higher layer signaling nr-DL-PRS-FirstPathRSRP-Result, subject to UE capability, up to 24 DL PRS RSRPP for the first detected path on different DL PRS resources associated with the same dl-PRS-ID. The UE may be configured to optionally report a differential DL RSRPP for a PRS resource with reference to [nr-DL-PRS-FirstPathRSRP-Result] and/or a differential DL PRS RSRP with reference to [nr-DL-PRS-RSRP-Result] via higher layer parameter [NR-DL-AoD-AdditionalMeasurementElement]. When the UE reports DL PRS-RSRP/DL PRS-RSRPP measurements, the UE may indicate which DL PRS-RSRP/DL PRS-RSRPP measurements associated with the same higher layer parameter nr-DL-PRS-RxBeamIndex [17, TS 37.355] have been performed using the same spatial domain filter for reception if for each nr-DL-PRS-RxBeamIndex reported there are at least 2 DL PRS-RSRP measurements or at least 2 DL PRS-RSRPP measurements associated with it within the DL PRS resource sets on a positioning frequency layer with the same dl-PRS-ID. ================== End of TP ==================

## First round of discussion  (closed)

Companies are encouraged to comment on the TP2.9 in the table below:

TP2.9

| company | comment |
| --- | --- |
| vivo | We don’t think “from one DL PRS resource set “ can be removed. Generally speaking, for AoD positioning, only multiple resources from one set can be used to calculate AoD angle. In addition, we don’t remember related agreement (“from different DL PRS resource sets”) has been achieved. |
| ZTE | Support this TP. @vivo We see benefit if resources within two sets can be jointly used for AOD positioning. It’s possible that two DL PRS resources from different DL PRS resource sets are received by the same spatial domain filter (i.e. the same DL PRS Rx beam index). With this information being reported to LMF, LMF can determine that whether the two different DL PRS-RSRP measurements associated with the two DL PRS resources are due to different TRP transmission beams or different UE Rx beams when conducting angle calculation. |
| Huawei, HiSilicon | First, the condition of RSRPP should be added for the Rx beam index reporting. Second, with subset association, it should be possible that UE reports only two RSRPP measurements corresponding to one PRS resource in two PRS resource sets of a TRP. |
| Qualcomm | We think it should be possible to report across 2 PRS resource sets. |
| FL | We can continue the discussion until the next checkpoint (I don’t think there is ASN.1 impact). |
| LGE | According to 37.355, Rx beam index can be presented under the assumption that at least two measurements from the same DL-PRS resource set. If we support this TP, we think it seems inconsistent with the current description if our understanding is right. nr-DL-PRS-RxBeamIndex This field provides an index of the target device receive beam used for DL-PRS measurements. If the value of the receive beam index for two or more DL PRS measurements is the same, it indicates that the target device receive beam for the two or more DL PRS measurements were made with the same RX beam. The field is mandatory present if at least two DL-PRS RSRP measurements from the same DL-PRS Resource Set have been made with the same RX beam by the target device; otherwise it is not present.. |
| Huawei, HiSilicon | To LGE/FL: We think LPP spec neesds to be updated if we agree to the TP, which is why we have the following proposal in our contribution R1-2203099. Unfortunately it was not included in the discussion. Proposal 9: Send an LS to RAN2 informing that the following change to the field description of nr-DL-PRS-RxBeamIndex is preferred from RAN1 perspective. nr-DL-PRS-RxBeamIndex This field provides an index of the target device receive beam used for DL-PRS measurements. If the value of the receive beam index for two or more DL PRS measurements is the same, it indicates that the target device receive beam for the two or more DL PRS measurements were made with the same RX beam. The field is mandatory present if at least two DL-PRS RSRP measurements or at least two DL-PRS RSRPP measurements from the DL-PRS Resource Sets on a positioning frequency layer of the TRP have been made with the same RX beam by the target device; otherwise it is not present.. |

## Conclusion

TBD

## References

- R1-2203099	Maintenance of Rel-17 positioning accuracy improvements	Huawei, HiSilicon

- R1- 2205095 Maintenance of NR Positioning Accuracy	Nokia, Nokia Shanghai Bell

- R1-2203176	Maintenance of Other NR positioning enhancements	Nokia, Nokia Shanghai Bell

- R1-2203436	Maintenance on enhancements of accuracy improvements for NR positioning	CATT

- R1-2203437	Maintenance on latency reduction for NR positioning	CATT

- R1-2203515	Maintenance on accuracy improvements for NR positioning enhancements	vivo

- R1-2203516	Discussion on other maintenance issues on NR positioning enhancements	vivo

- R1-2203619	Remaining issues on accuracy improvement for Rel-17 positioning	ZTE

- R1-2203620	Remaining issues other than accuracy improvement for Rel-17 Positioning	ZTE

- R1-2203786	Remaining issues on PRS collision detection	xiaomi

- R1-2203864	Maintenance on accuracy improvement related enhancement	Samsung

- R1-2203865	Maintenance on latency and efficiency improvement related enhancement	Samsung

- R1-2203960	Maintenance of Rel-17 Positioning Accuracy Enhancement	OPPO

- R1-2203961	Maintenance of Rel-17 Positioning enhancement other than accuracy enhancement	OPPO

- R1-2204127	Remaining issues for accuracy enhancements for NR positioning	InterDigital, Inc.

- R1-2204128	Remaining issues for NR positioning	InterDigital, Inc.

- R1-2204275	Remaining issues on accuracy improvements	CMCC

- R1-2204276	Remaining issues on latency improvements	CMCC

- R1-2204346	Remaining issue on accuracy improvements	NTT DOCOMO, INC.

- R1-2204522	Discussion on maintenance for NR positioning other enhancements	LG Electronics

- R1-2204903	Maintenance of Rel-17 positioning latency and efficiency improvements	Huawei, HiSilicon

- R1-2204942	Maintenance of accuracy improvements for NR positioning enhancements	Ericsson

- R1-2204943	Remaining issues for NR positioning enhancements	Ericsson

- R1-2204985	Maintenance on Accuracy Improvements	Qualcomm Incorporated

- R1-2204986	Maintenance on Other Issues in NR Positioning Enhancements	Qualcomm Incorporated

- R1-2203040 (R3-222721), “Questions concerning the implementation of RAN1 agreements in NRPPa,” RAN3 (Ericsson)

- R1- 2205095 (revision of R1- R1- 2205075)  Maintenance of NR Positioning Accuracy Nokia, Nokia Shanghai Bell