3GPP TSG RAN WG1 Meeting #109-e	R1-2205532

e-Meeting, May 09 – 20, 2022

Source: 	Moderator (Intel Corporation)

Title:	Summary #3 of email discussion on initial access aspect of NR extension up to 71 GHz

Agenda item:	8.2.1

Document for:	Discussion

# Introduction

In this contribution, moderator summarizes discussions on remaining issues related to initial access for extending NR up to 71 GHz based submitted contributions from RAN1 #109-e. Issues for discussion are based on outcome of the preparation phase on maintanence on supporting NR from 52.6 GHz to 71 GHz. An assessment on each of the maintenance issues is provided in Table 1.

[109-e-R17-FR2-2-01] Email discussion under 8.2.1 for maintenance on initial access aspects, for issues 1-1, 1-4, 1-5 and 1-6 in R1-2205124 – Daewon (Intel)

- 1st check point: May 13 (any RRC impact by May 12)

- Final check point: May 18

Table 1. Issue assessment for initial access aspects.

| Issue# | Issue | References | assessment |
| --- | --- | --- | --- |
| 1-1 | Additional RB offset for 96 PRB CORESET#0 | [1] [2] [4] [5] [6] [7] [8] [9] [10] [11] [13] | Discuss in [109-e-R17-FR2-2-01] |
| 1-2 | Use of PBCH DMRS for partial indication of Q | [3] |  |
| 1-3 | Confirmation of WA on signaling for Q | [4] [7] [9] [11] |  |
| 1-4 | CD-SSB frequency indication using NCD-SSB | [6] | Discuss in [109-e-R17-FR2-2-01] |
| 1-5 | Description update of longBitmap of SSB-ToMeasure | [6] | Discuss in [109-e-R17-FR2-2-01] |
| 1-6 | Removal of ‘-‘ sign from 24 RB offset for mux pattern 3 | [1] | Discuss in [109-e-R17-FR2-2-01] |

# Summary of issues

## 2.1 (Issue 1-1) Additional RB offset for 96 PRB CORESET#0

### Summary of Discussions

- For additional RB offset for 96 PRB COREST#0 with 480/960 kHz SCS

  - Support X = 76

    - Huawei/HiSilicon, ZTE, CATT, Samsung, vivo, OPPO, Ericsson, Apple,

  - Support X = 38

    - Intel

  - Do not support additional value

    - Nokia/NSB

  - Postpone decision until RAN4 concludes raster design

    - LGE

### 1st Round Discussion

Based on submitted contribution, a clear majority supports X=76. Based on this moderator suggests the following agreement.

Suggest for agreement:

- Agree to TP#1-1 for TS38.213

#### TP #1-1 (TS38.213)

Reasons for change: The specification is missing additional SSB and CORESET RB offset for FR2-2 when using 96 RB CORESET.

Summary of change: Add 76 RB offset for 96 RB CORESET with SSB/CORESET multiplexing pattern 1.

Consequence if not approved: Unable to support 96 RB CORESET for some channel location deployments in FR2-2

13 UE procedure for monitoring Type0-PDCCH CSS sets ============== Unchanged Text Omitted ============== Table 13-10A: Set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {120, 120} kHz, {480, 480} kHz, or {960, 960} kHz for FR2-2 Index; SS/PBCH block and CORESET multiplexing pattern; Number of RBs NRBCORESET; Number of Symbols NsymbCORESET; Offset (RBs). 0; 1; 24; 2; 0. 1; 1; 24; 2; 4. 2; 1; 48; 1; 0. 3; 1; 48; 1; 14. 4; 1; 48; 1; 28. 5; 1; 48; 2; 0. 6; 1; 48; 2; 14. 7; 1; 48; 2; 28. 8; 1; 96; 1; 0. 9; 1; 96; 1; 76. 10; 1; 96; 2; 0. 11; 1; 96; 2; 76. 12; 3; 24; 2; -20 if kSSB=0 -21 if kSSB>0. 13; 3; 24; 2; -24. 14; 3; 48; 2; -20 if kSSB=0 -21 if kSSB>0. 15; 3; 48; 2; -48. ============== Unchanged Text Omitted ==============

Please comment on the suggestion above.

| Company | Comments |
| --- | --- |
| DOCOMO | Ok with TP#1-1. |
| Nokia | We are fine to go with majority view (i.e. TP#1-1) |
| Intel | Willing to accept TP#1-1 and to go with majority view. |
| LG Electronics | Even though our first preference is not to define X, we can accept TP#1-1 if majority companies support it. |
| Samsung | We support the proposal and the associated TP#1-1. |
| Qualcomm | We are fine with TP #1-1 |
| Ericsson | Support TP #1-1 |
| Huawei, HiSilicon | Support TP#1-1 |
| Apple | Support TP #1-1 |
| OPPO | Support TP #1-1 |
| vivo | Support TP#1-1 |
| ZTE, Sanechips | Support TP #1-1 |
| CATT | Support TP #1-1 |
| InterDigital | Support TP #1-1 |

### <Summary of 1st Round Discussion>

TP#1-1 seems agreeable by all. Moderator will suggest for email approval of TP#1-1 (See section 3)

With this moderator assume this issue is now closed.

### [Discussion CLOSED]

## 2.2 (Issue 1-4) CD-SSB frequency indication using NCD-SSB

- From [6] Samsung:

  - For CD-SSB indication using NCD-SSB on synchronization raster, support the indicated GSCN of the indicated CD-SSB as NGSCNReference+NGSCNStep⋅NGSCNOffset, wherein NGSCNStep is the step size given in Table 5.4.3.3-2 of TS 38.101-2.

### Summary of Discussions

For Rel-15, when SSB does not have associated RMSI transmission, the SSB can indicate SSB frequency location that has an associated RMSI. The value range that can be indicated for the SSB frequency location is limited from -256 to +256. Table 13-17 of TS38.213 shows the range that can be used to indicate the SSB location.

Table 13-17: Mapping between the combination of kSSB and controlResourceSetZero and searchSpaceZero in pdcch-ConfigSIB1 to NGSCNOffset for FR2

| kSSB | 16×controlResourceSetZero +searchSpaceZero | NGSCNOffset |
| --- | --- | --- |
| 12 | 0, 1, …, 255 | 1, 2, …, 256 |
| 13 | 0, 1, …, 255 | -1, -2, …, -256 |
| 14 | 0, 1, …, 255 | Reserved, Reserved, …, Reserved |

The maximum number of GSCN entries that can fit for licensed spectrum in 60 GHz band can potentially exceed 256 steps in GSCN. Based on this Samsung proposes the update the indicated GSCN using Rel-15 feature by introducing a step size for the indication.

### 1st Round Discussion

Moderators suggest further discuss Samsung’s proposal, TP#2-1.

#### TP #2-1 (TS38.213)

Reasons for change: Number of GSCN entries that can be indicated by a non-cell defining SSB in FR2-2 is larger than what the specification can support for FR1 and FR2-1, which is -256 to +256.

Summary of change: Add multiplier step size for FR2-2 when determining the GSCN location indicated by a non-cell defining SSB.

Consequence if not approved: Cell defining SSB location that can be indicated by the network will be limited to a small frequency range scope and cannot be utilized in certain situations.

13 UE procedure for monitoring Type0-PDCCH CSS sets ============== Unchanged Text Omitted ============== If a UE detects a first SS/PBCH block and determines that a CORESET for Type0-PDCCH CSS set is not present, and for 24≤kSSB≤29 for FR1 or for 12≤kSSB≤13 for FR2, the UE may determine the nearest (in the corresponding frequency direction) global synchronization channel number (GSCN) of a second SS/PBCH block having a CORESET for an associated Type0-PDCCH CSS set as NGSCNReference+NGSCNSize⋅NGSCNOffset. NGSCNReference is the GSCN of the first SS/PBCH block, NGSCNSize=1 for FR1 and FR2-1, and NGSCNSize is the step size given by Table 5.4.3.3-2 of [8-2, TS 38.101-2] for FR2-2, and NGSCNOffset is a GSCN offset provided by Table 13-16 for FR1 and Table 13-17 for FR2. If the UE detects the second SS/PBCH block and the second SS/PBCH block does not provide a CORESET for Type0-PDCCH CSS set, as described in clause 4.1, the UE may ignore the information related to GSCN of SS/PBCH block locations for performing cell search. ============== Unchanged Text Omitted =================

| Company | Comments |
| --- | --- |
| DOCOMO | Ok with TP#2-1. |
| Nokia | Fine with TP#2-1. |
| Intel | Ok with TP#2-1. |
| LG Electronics | We understand the motivation that the number of GSCN can exceed 256. However, considering that synchronization raster for 60 GHz unlicensed band has not been finalized yet in RAN4, we can defer the decision on whether to adopt TP#2-1. |
| Samsung | We support the proposal and associated TP#2-1. |
| Qualcomm | Fine with TP #2-1 |
| Ericsson | We share a similar view as LGE. Since this is not RRC impacting, we can revisit once the sync raster design is complete. |
| Huawei, HiSilicon | We are general fine with the concept. It should be further clarified whether same scheme is applicable to both n263 and n264. Also, the table index should be updated to 5.4.3.3-1 in which step size is used. |
| Apple | Fine with proposal and the corresponding TP#2-1. |
| OPPO | Fine with TP#2-1. |
| vivo | We share the same view as LGE and Ericsson. |
| ZTE, Sanechips | We agree that this issue needs to be solved, but at present RAN4 only gives the GSCNs of n264 (66-71GHz) in FR2-2, and some frequency bands have not been defined. Therefore, we share same views as LGE and Ericsson. We can discuss this issue after the relevant work of RAN4 is completed. |
| Moderator | Please continue to provide comments. @Samsung Can you double check the table that step size should refer to? Moderator thinks it should be Table 5.4.2.3-1. Table 5.4.3.3-2 doesn’t exist in 101-2 specification as far as moderator can tell. |
| CATT | We share the view with some other companies that this discussion can be deferred. |
| InterDigital | Ok with TP#2-1. Fine with deferring the issue to after finalization of synch raster design for unlicensed band by RAN4. |
| Samsung | Response to moderator: Sorry we made a typo on the reference. It should be Table 5.4.3.3-2 in TS 38.104, and we believe it’s equivalent to use Table 5.4.2.3-1 in TS 38.101-2. The table update for FR2-2 has not been adopted in latest spec, and the RAN4 agreed TP is R4-2206582 (which is for TS 38.104). |
| Moderator | Just to respond to Huawei’s question on applicability for n263 and n264. The TP states the step size applies for FR2-2, therefore moderator assumes both n263 and n264 would apply. |

### <Summary of 1st Round Discussion>

From the comments, most companies seem to be ok in principle with TP#2-1. However, few companies commented they would like to wait for RAN4 to conclude the channel and sync raster before concluding, as the RAN4 conclusion can potentially impact the needs and requirement of the TP#2-1.

Moderator has update TP#2-1 to fix the table reference.

#### TP #2-1A (TS38.213)

Reasons for change: Number of GSCN entries that can be indicated by a non-cell defining SSB in FR2-2 is larger than what the specification can support for FR1 and FR2-1, which is -256 to +256.

Summary of change: Add multiplier step size for FR2-2 when determining the GSCN location indicated by a non-cell defining SSB.

Consequence if not approved: Cell defining SSB location that can be indicated by the network will be limited to a small frequency range scope and cannot be utilized in certain situations.

13 UE procedure for monitoring Type0-PDCCH CSS sets ============== Unchanged Text Omitted ============== If a UE detects a first SS/PBCH block and determines that a CORESET for Type0-PDCCH CSS set is not present, and for 24≤kSSB≤29 for FR1 or for 12≤kSSB≤13 for FR2, the UE may determine the nearest (in the corresponding frequency direction) global synchronization channel number (GSCN) of a second SS/PBCH block having a CORESET for an associated Type0-PDCCH CSS set as NGSCNReference+NGSCNSize⋅NGSCNOffset. NGSCNReference is the GSCN of the first SS/PBCH block, NGSCNSize=1 for FR1 and FR2-1, and NGSCNSize is the step size given by Table 5.4.3.3-2 5.4.3.3-1 of [8-2, TS 38.101-2] for FR2-2, and NGSCNOffset is a GSCN offset provided by Table 13-16 for FR1 and Table 13-17 for FR2. If the UE detects the second SS/PBCH block and the second SS/PBCH block does not provide a CORESET for Type0-PDCCH CSS set, as described in clause 4.1, the UE may ignore the information related to GSCN of SS/PBCH block locations for performing cell search. ============== Unchanged Text Omitted =================

From moderator’s checking with RAN4 progress, RAN4 has made the following agreements on Tuesday this week.

Agreement: - Channel Raster Option 2 (Intel, R4-2208046) - for 100 MHz channel bandwidth, NREF = {2564083 + 1680*N, N = 0:137} - for 400 MHz channel bandwidth, NREF = {2566603 + 6720*N, N = 0:33} - Further discuss the channel raster for wider channel bandwidth, i.e. 800MHz, 1600MHz, 2000MHz Agreement: - Have one GSCN entry corresponding to each channel with the smallest bandwidth for SCS. - The total number is [138] entries for 120KHz SCS. - Further discuss the GSCN for 480KHz and 960KHz SCS

RAN4 Chair also plans to come back to the channelization issue on May 17, which moderator assumes will finalize the rest of the channelization.

With this said, moderator thinks the comments from LGE, Ericsson, vivo, CATT, and Interdigital is valid. It might be safer to first review the RAN4 CRs before agreeing to TP that utilizes not yet defined values.

Based on comments, moderator suggest to deferring this discussion after May 17, in which RAN4 may conclude the issue.

### [CLOSED] 2nd Round Discussion

Postpone 2nd round discussion until RAN4 has concluded on the raster. Moderator will check RAN4 progress on May 17 and provide an update of the situation.

The following are latest agreements from RAN4.

Agreement - Channel Raster Option 2 (Intel, R4-2208046) - for 100 MHz channel bandwidth, NREF = {2564083 + 1680*N, N = 0:137} - for 400 MHz channel bandwidth, NREF = {2566603 + 6720*N, N = 0:33} - Further discuss the channel raster for wider channel bandwidth, i.e. 800MHz, 1600MHz, 2000MHz Agreement: - Have one GSCN entry corresponding to each channel with the smallest bandwidth for SCS. - The total number is [138] entries for 120KHz SCS. - Further discuss the GSCN for 480KHz and 960KHz SCS Agreement: Agree on Option 2. - channel raster for wider channel bandwidths (800MHz, 1600MHz, 2000MHz) - Option 2: - For 800 MHz: {2569963 + 6720*N, N = 0:32} - For 1600 MHz: {2576683 + 6720*N, N =0:30} - For 2000 MHz: {2580043 + 6720*N, N=0:29; 2585083, 2655643, 2692603, 2764843} Agreement: [Agree on Option 1] and further check the value in this meeting. - sync raster applicable to Pcell and PScell (cells that support initial access as per WID) - Option 1: - For 120 kHz SCS: 24156 + 6*N - 3*floor((N+5)/18), N=0:137 - For 480 kHz SCS: 24162 + N*24 - 12*floor((N+4)/18), N=0:33 Tentative agreement: - For sync raster applicable to non-initial access - Step size <3>, <12>, and <6> for 120, 480, and 960 kHz, respectively - Clarify the sync raster of 960kHz is not for initial access and [the usage of the 960KHz sync raster for the PSCell is not precluded]. - As per WID, initial access with 960khz is not supported

With the above agreements and tentative agreements in RAN4, are companies ready to review and conclude whether or not to accept TP#2-1A?

If companies need more time to review (after all agreements are concluded in RAN4), then moderator suggests revisiting either (1) next meeting or (2) after Thursday this week (when RAN4 could potentially conclude on the issues over the reflector by approving the WF R4-2210577). Please provide your preferences if you think RAN1 should wait.

| Company | Comments |
| --- | --- |
| LG Electronics | At this stage, we prefer to revisit this issue next meeting after related RAN4 CRs are endorsed. By the way, we have one question to understand current specification better. What happens if we do not change the spec? For example, when the first SSB corresponds to NGSCNReference=24674 (which is the first GSCN value defined for n264 licensed band) and NGSCNOffset=2 is indicated by MIB, then the second GSCN corresponds to 24674? Or 24675 (because 24675 is the nearest GSCN for n264 from indicated GSCN=24674)? In other words, we would like to clarify what “nearest GSCN” implies in the specification. “If a UE detects a first SS/PBCH block and determines that a CORESET for Type0-PDCCH CSS set is not present, and for 24≤kSSB≤29 for FR1 or for 12≤kSSB≤13 for FR2, the UE may determine the nearest (in the corresponding frequency direction) global synchronization channel number (GSCN) of a second SS/PBCH block having a CORESET for an associated Type0-PDCCH CSS set as NGSCNReference+NGSCNOffset. NGSCNReference is the GSCN of the first SS/PBCH block and NGSCNOffset is a GSCN offset provided by Table 13-16 for FR1 and Table 13-17 for FR2.” |
| ZTE, Sanechips | We have the same view with LGE, that it, tend to discuss this issue at the next meeting considering related agreements have not fully endorsed in RAN4. |
| vivo | Agree with LG and ZTE that it could be revisited in next meeting. |
| Huawei, HiSilicon | We would like to see the endorsed RAN4 CR at first. |
| OPPO | We agree with other companies that it can be revisited in next meeting. |
| Samsung | From the RAN4 agreement, the step size for 120 kHz sync raster is still 3 and the step size for 480 kHz sync raster is still 12, so the proposal in TP #2-1A can still work for unlicensed band. But we are ok to discuss this later if most companies want so. Response to LG: the wording “nearest” is understood to be combined with the description “a second SS/PBCH block having a CORESET for an associated Type0-PDCCH CSS set”. So it means the nearest GSCN such that there is a cell defining SSB on that GSCN. For instance, if there can be multiple cell defining SSBs, then the scheme will indicate the nearest one. |
| Ericsson | Fine to wait until next meeting once RAN4 CRs are in place |
| LG Electronics | Response to Samsung: Thanks Hongbo for answering the question. But I have follow-up questions. So, in my example, if NGSCNOffset=2 is indicated by MIB from NGSCNReference=24674 (which is the first GSCN value defined for n264 licensed band), UE won’t go to 24675 (which is not the same as the indicated GSCN but is the nearest SS raster) to detect the second SSB. Is it correct understanding? If this is the case, gNB has to indicate NGSCNOffset=3, right? In addition, if MIB shall indicate the exact location of GSCN where cell-defining SSB exists, how can there be multiple cell defining SSBs? |
| Samsung2 | Response to LG: For the first follow-up question, you are right that the UE will not search 24675 (I think there is a typo in your comment, and you would want to say NGSCNOffset=1 ^^), since it’s not a valid sync raster point (i.e., there cannot be a cell-defining SSB there). Just to clarify, 24675 is a valid GSCN, but not a valid SS raster for band n264 (so the comment in the bracket is a little bit inaccurate). So overall, it’s up to gNB’s implementation to indicate the correct GSCN value (e.g. in your example, an integer multiple of 3). Actually this happened in FR1/FR2-1 as well – for the bands with sync raster step size not as 1 (which implies there are cases of invalid indication), however, since the value range for sync raster is small, always taking the step size as 1 doesn’t cause the issue of indication out of range. For the additional follow-up question, the multiple cell-defining SSBs are for the wide operation case. The indication of cell-defining SSB using non-cell-defining SSB is per band, so it’s possible that network can implement multiple cell-defining SSB in different frequency locations in a wideband. Just to clarify, the scheme works even without the wording “nearest” (at least to us, whether or not restricting the indication to the nearest cell-defining SSB does not matter to much). |
| Moderator | RAN4 was not able to conclude the final CRs at the end of May 19 GTW session. RAN1 may not have sufficient time to review the RAN4 CRs on channelization by the time RAN4 concludes them in this meeting. Therefore, moderator suggests to close the discussion and discuss this issue (if needed) in the next meeting. |

### <Summary of 2nd Round Discussion>

RAN4 was not able to conclude the final CRs at the end of May 19 GTW session. RAN1 may not have sufficient time to review the RAN4 CRs on channelization by the time RAN4 concludes them in this meeting. Therefore, moderator suggests to close the discussion and discuss this issue (if needed) in the next meeting.

### [Discussion CLOSED]

## 2.3 (Issue 1-5, 1-6) Editorial aspects

### Summary of Discussions

- From [1] Huawei/HiSilicon:

  - Remove the negative sign from index 13 and 15 of the CORESET0 configuration table for 480 and 960 kHz (TP#3-1).

#### TP#3-1 (TS38.213)

Reasons for change: The sign offset RB value of index 13 and 15 of Table 13-10A is incorrect.

Summary of change: Remove the ‘-‘ negative sign for offset RB value of index 13 and 15 of Table 13-10A.

Consequence if not approved: Incorrect RB offset configuration for multiplexing pattern 3 and both network and UE are not able to utilize the configuration.

# 13 UE procedure for monitoring Type0-PDCCH CSS sets ## *** Unchanged text is omitted *** Table 13-10A: Set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {120, 120} kHz, {480, 480} kHz, or {960, 960} kHz for FR2-2 Index; SS/PBCH block and CORESET multiplexing pattern; Number of RBs NRBCORESET; Number of Symbols NsymbCORESET; Offset (RBs). 0; 1; 24; 2; 0. 1; 1; 24; 2; 4. 2; 1; 48; 1; 0. 3; 1; 48; 1; 14. 4; 1; 48; 1; 28. 5; 1; 48; 2; 0. 6; 1; 48; 2; 14. 7; 1; 48; 2; 28. 8; 1; 96; 1; 0. 9. 10; 1; 96; 2; 0. 11. 12; 3; 24; 2; -20 if kSSB=0 -21 if kSSB>0. 13; 3; 24; 2; 24. 14; 3; 48; 2; -20 if kSSB=0 -21 if kSSB>0. 15; 3; 48; 2; 48. ## *** Unchanged text is omitted ***

- From [6] Samsung:

  - Add description to longBitmap of SSB-ToMeasure (TP#3-2, requires LS to RAN2)

#### TP#3-2 (TS38.331)

SSB-ToMeasure field descriptions

longBitmap Bitmap when maximum number of SS/PBCH blocks per half frame equals to 64 as defined in TS 38.213 [13], clause 4.1. For operation with shared spectrum channel access, if the k-th bit is set to 1, the UE assumes that one or more SS/PBCH blocks within the SMTC measurement duration with candidate SS/PBCH block indexes corresponding to SS/PBCH block index equal to k – 1 may be measured; if the k-th bit is set to 0, the UE assumes that the corresponding SS/PBCH block(s) are not measured.

#### TP#3-2A (TS38.331)

SSB-ToMeasure field descriptions

longBitmap Bitmap when maximum number of SS/PBCH blocks per half frame equals to 64 as defined in TS 38.213 [13], clause 4.1. For operation with shared spectrum channel access, if the k-th bit is set to 1, the UE assumes that one or more SS/PBCH blocks within the SMTC measurement duration with candidate SS/PBCH block indexes corresponding to SS/PBCH block index equal to k – 1 may be transmitted measured; if the k-th bit is set to 0, the UE assumes that the corresponding SS/PBCH block(s) are not measured transmitted.

### 1st Round Discussion

Both issues 1-5 and 1-6 are editorial in nature but seems essential for completion of the specifications. Both proposals from Huawei and Samsung seem straightforward and suggest the following.

Suggest for agreement:

- Agree to TP#3-1 for TS38.213

- Agree in principle to TP#3-2 for TS38.331. Send LS to RAN2 to ask to update the description.

Please comment on the suggestion above.

| Company | Comments |
| --- | --- |
| DOCOMO | Agree with Moderator’s suggestion. |
| Nokia | Fine with the TP#3-1. On TP#3-2, there are some differences between the corresponding text in mediumBitmap and the proposed text. It appears that the proposed text attempts to align with the SSB-ToMeasure field description. I.e. proposed text refers to SMTC (similar to SSB-ToMeasure), while DBTW is used in the legacy description for mediumBitmap, and the proposed text talks of measurements while the corresponding text talks of transmission. In principle both ways are fine if group so agrees, while it might be simplest to align with earlier wording. If we choose to refer to measurement, rather than transmissions, the value 1 should not be interpreted as ‘may be measured’, but ‘is to be measured’ similarly as per SSB-ToMeasure. |
| Intel | Agree to TP#3-1. For TP#3-2, agree to Nokia’s suggestion to change ‘may be measured’ to ‘is measured’. Other than this, in principle ok with TP#3-2. |
| LG Electronics | Support TP#3-1. For TP#3-2, here is the excerpt from TS 38.331. SSB-ToMeasure field descriptions. longBitmap Bitmap when maximum number of SS/PBCH blocks per half frame equals to 64 as defined in TS 38.213 [13], clause 4.1.. mediumBitmap Bitmap when maximum number of SS/PBCH blocks per half frame equals to 8 as defined in TS 38.213 [13], clause 4.1. For operation with shared spectrum channel access, if the k-th bit is set to 1, the UE assumes that one or more SS/PBCH blocks within the SMTC measurement duration with candidate SS/PBCH block indexes corresponding to SS/PBCH block index equal to k – 1 may be transmitted; if the kt-th bit is set to 0, the UE assumes that the corresponding SS/PBCH block(s) are not transmitted. The k-th bit is set to 0, where k > ssb-PositionQCL-Common and the number of actually transmitted SS/PBCH blocks is not larger than the number of 1’s in the bitmap. If ssb-PositionQCL is configured with a value smaller than ssb-PositionQCL-Common, only the leftmost K bits (K = ssb-PositionQCL) are applicable for the corresponding cell.. shortBitmap Bitmap when maximum number of SS/PBCH blocks per half frame equals to 4 as defined in TS 38.213 [13], clause 4.1.. It seems that yellow text can be reused also for the case of longBitmap. It would be better if Samsung can clarify why “transmitted” in yellow text is changed to “measured” in TP#3-2. |
| Samsung | Agree with TP#3-1 (actually this is an inconsistency between the agreed CR and final specification, so can simply notify editor to fix it in the next update, but we are ok with an explicit agreement as well). For TP#3-2, actually we believe the RAN2 description on the mediumBitmap is a little different from RAN1 intention (should not be about SSB transmission, but which SSB to measure), and the description in TP#3-2 for longBitmap better reflects RAN1 intention. We are ok with Nokia’s suggestion on the wording “is to be measured”. |
| Qualcomm | Fine with TP #3-1. For TP #3-2, we are fine with Nokia’s suggestion. |
| Ericsson | Support TP #3-1 Regarding TP #3-2, if Nokia’s suggestion is adopted, is it really so that the UE should measure each candidate position with the same SSB index? Shouldn’t it first detect which candidate position corresponds to an actual SSB transmission and then measure on that one? Isn’t that why the original wording was “may be transmitted”? |
| Huawei, HiSilicon | Support TP#3-1 For TP#3-2, we prefer to the consistent description between longBitmap and mediumBitmap considering they are from the same motivation. Either “transmit” or “measure” should be fine but they should be same. As for the wording between “is” or “may”, we share similar view as Ericsson that not all candidate SSB position should be measured. If UE already measured the SSB at first candidate position with certain SSB index, UE can skip the rest. |
| Apple | Support TP#3-1. For TP#3-2, our preference is to keep consistency between longbitmap and mediumBitma field, especially they are associated with same function to avoid confusion. |
| OPPO | Support TP#3-1. For TP#3-2, our preference is to use the original wording “transmitted”. |
| vivo | Fine with TP #3-1. Agree with Nokia’s suggestion for TP #3-2. |
| ZTE, Sanechips | Fine with the TP#3-1. For TP #3-2, we disagree with neither TP#3-2 nor Nokia’s suggestion. We think the description of mediumBitmap can be directly reused for longBitmap. We see no need to revise or rephrase. |
| Moderator | Added TP#3-2a that used the description from mediumBitmap. |
| CATT | Fine with the TP#3-1. Ok with TP#3-2a |
| Ericsson | Support TP #3-1 Support TP #3-2a |
| InterDigital | Support TP#3-1. |
| Huawei, HiSilicon | Support TP#3-1 and TP#3-2a |
| LG Electronics | Support TP#3-1 and TP#3-2a |
| Nokia | We are fine with TP#3-2A (and TP#3-1 still) |
| Samsung2 | We still have concern with the wording “transmitted”, but we agree it can be handled within RAN2 and making wording consistent from RAN1 perspective. In this sense, we can be ok with TP#3-2a (but we should not agree TP#3-2a directly since it’s a RAN2 spec). |

### <Summary of 1st Round Discussion>

From the feedback TP#3-1 seems agreeable by all. Moderator will suggest for email approval (see section 3). TP#3-2a seem agreeable by all. As Samsung noted this is requested to change RAN2 specification, so wording for agreement should be:

- Agree in principle to TP#3-2A for TS38.331 from R1-2205138. Send LS to RAN2 asking to update the description.

#### TP#3-2A (TS38.331)

SSB-ToMeasure field descriptions

longBitmap Bitmap when maximum number of SS/PBCH blocks per half frame equals to 64 as defined in TS 38.213 [13], clause 4.1. For operation with shared spectrum channel access, if the k-th bit is set to 1, the UE assumes that one or more SS/PBCH blocks within the SMTC measurement duration with candidate SS/PBCH block indexes corresponding to SS/PBCH block index equal to k – 1 may be transmitted; if the k-th bit is set to 0, the UE assumes that the corresponding SS/PBCH block(s) are not transmitted.

With the above, moderator assume the two issues 1-5 and 1-6 are now closed.

### [Discussion CLOSED]

# Summary of Agreements/Conclusions from RAN1 #109-e

Agreement:

- Text Proposal #1-1 (for TS38.213 v17.1.0, clause 13) in section 3 of R1-2205138 is endorsed, without the empty row.

#### TP #1-1 (TS38.213)

Reasons for change: The specification is missing additional SSB and CORESET RB offset for FR2-2 when using 96 RB CORESET.

Summary of change: Add 76 RB offset for 96 RB CORESET with SSB/CORESET multiplexing pattern 1.

Consequence if not approved: Unable to support 96 RB CORESET for some channel location deployments in FR2-2

13 UE procedure for monitoring Type0-PDCCH CSS sets ============== Unchanged Text Omitted ============== Table 13-10A: Set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {120, 120} kHz, {480, 480} kHz, or {960, 960} kHz for FR2-2 Index; SS/PBCH block and CORESET multiplexing pattern; Number of RBs NRBCORESET; Number of Symbols NsymbCORESET; Offset (RBs). 0; 1; 24; 2; 0. 1; 1; 24; 2; 4. 2; 1; 48; 1; 0. 3; 1; 48; 1; 14. 4; 1; 48; 1; 28. 5; 1; 48; 2; 0. 6; 1; 48; 2; 14. 7; 1; 48; 2; 28. 8; 1; 96; 1; 0. 9; 1; 96; 1; 76. 10; 1; 96; 2; 0. 11; 1; 96; 2; 76. 12; 3; 24; 2; -20 if kSSB=0 -21 if kSSB>0. 13; 3; 24; 2; -24. 14; 3; 48; 2; -20 if kSSB=0 -21 if kSSB>0. 15; 3; 48; 2; -48. ============== Unchanged Text Omitted ==============

Agreement:

- Text Proposal #3-1 (for TS38.213 v17.1.0, clause 13) in section 3 of R1-2205138 is endorsed.

#### TP#3-1 (TS38.213)

Reasons for change: The sign offset RB value of index 13 and 15 of Table 13-10A is incorrect.

Summary of change: Remove the ‘-‘ negative sign for offset RB value of index 13 and 15 of Table 13-10A.

Consequence if not approved: Incorrect RB offset configuration for multiplexing pattern 3 and both network and UE are not able to utilize the configuration.

# 13 UE procedure for monitoring Type0-PDCCH CSS sets ## *** Unchanged text is omitted *** Table 13-10A: Set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {120, 120} kHz, {480, 480} kHz, or {960, 960} kHz for FR2-2 Index; SS/PBCH block and CORESET multiplexing pattern; Number of RBs NRBCORESET; Number of Symbols NsymbCORESET; Offset (RBs). 0; 1; 24; 2; 0. 1; 1; 24; 2; 4. 2; 1; 48; 1; 0. 3; 1; 48; 1; 14. 4; 1; 48; 1; 28. 5; 1; 48; 2; 0. 6; 1; 48; 2; 14. 7; 1; 48; 2; 28. 8; 1; 96; 1; 0. 9. 10; 1; 96; 2; 0. 11. 12; 3; 24; 2; -20 if kSSB=0 -21 if kSSB>0. 13; 3; 24; 2; 24. 14; 3; 48; 2; -20 if kSSB=0 -21 if kSSB>0. 15; 3; 48; 2; 48. ## *** Unchanged text is omitted ***

Agreement:

- Text Proposal #3-2A for TS38.331 in section 3 of R1-2205138 is endorsed and recommended to RAN2.

- Send LS to RAN2 asking to update the description.

#### TP#3-2A (TS38.331)

SSB-ToMeasure field descriptions

longBitmap Bitmap when maximum number of SS/PBCH blocks per half frame equals to 64 as defined in TS 38.213 [13], clause 4.1. For operation with shared spectrum channel access, if the k-th bit is set to 1, the UE assumes that one or more SS/PBCH blocks within the SMTC measurement duration with candidate SS/PBCH block indexes corresponding to SS/PBCH block index equal to k – 1 may be transmitted; if the k-th bit is set to 0, the UE assumes that the corresponding SS/PBCH block(s) are not transmitted.

# Reference

- R1-2203079, “Remaining issue of initial access signals and channels for 52-71GHz spectrum,” Huawei, HiSilicon, SIA

- R1-2203290, “Remaining issues on initial access aspects for 52.6 to 71GHz,” ZTE, Sanechips

- R1-2203369, “Remaining issues for initial access operation in 52.6-71GHz,” InterDigital, Inc.

- R1-2203430, “Remaining issues on Initial access aspects for up to 71GHz operation,” CATT

- R1-2203508, “Maintenance on initial access for NR operation from 52.6GHz to 71GHz,” vivo

- R1-2203858, “Maintenance on initial access aspects for NR from 52.6 GHz to 71 GHz,” Samsung

- R1-2203986, “Discussion on remaining issue for initial access aspects,” OPPO

- R1-2204110, “Initial Access Aspects,” Ericsson

- R1-2204201, “Remaining issues for initial access aspects,” Apple

- R1-2204338, “Remaining issues on initial access aspects for NR in FR2-2,” NTT DOCOMO, INC.

- R1-2204599, “Initial access aspects,” Nokia, Nokia Shanghai Bell

- R1-2204611, “Remaining issues of initial access aspects to support NR above 52.6 GHz,” LG Electronics

- R1-2204766, “Discussion on initial access aspects for extending NR up to 71 GHz,” Intel Corporation

# List of RAN1 Agreements on initial access

## RAN1 #104-e

R1-2102073	[Draft] LS on beam switching gap for 60 GHz band	Intel Corporation

Final LS endorsed in R1-2102202

Agreement:

Send an LS to RAN4 to get input on gap required for gNBs and UEs for beam switching and for UL/DL and DL/UL switching.

Agreement:

Whether or not to support 240 kHz, 480kHz and 960kHz SCS for SSB and the conditions under which SSB for 240 kHz, 480 kHz and 960 kHz may be supported will be decided no later than RAN1#104bis-e.

Agreement:

For an unlicensed band that requires LBT, further study whether/how to support discovery burst (DB) and discovery burst transmission window (DBTW) at least for 120 kHz SSB SCS

- If DB supported

  - FFS: What signals/channels are included in DB other than SS/PBCH block

- If DBTW is supported

  - Support mechanism to indicate or inform that DBTW is enabled/disabled for both IDLE and CONNECTED mode UEs

    - FFS: how to support UEs performing initial access that do not have any prior information on DBTW.

  - PBCH payload size is no greater than that for FR2

  - Duration of DBTW is no greater than 5 ms

  - Number of PBCH DMRS sequences is the same as for FR2

- The following points are additionally FFS:

  - How to indicate candidate SSB indices and QCL relation without exceeding limit on PBCH payload size

  - Details of the mechanism for enabling/disabling DBTW considering LBT exempt operation and overlapping licensed/unlicensed bands

  - Whether or not to support DBTW for SSB SCS(s) other than 120 kHz if other SSB SCS(s) are supported

Agreement:

For CORESET#0 and Type0-PDCCH search space configured in MIB:

- Support {SS/PBCH Block, CORESET#0 for Type0-PDCCH} SCS equal to {120, 120} kHz

  - Support at least SSB and CORESET#0 multiplexing patterns, number of RBs for CORESET#0, number of symbols (duration of CORESET#0) that are supported in Rel-15/16 for {SS/PBCH Block, CORESET#0 for Type0-PDCCH} SCS = {120, 120} kHz.

    - FFS: Supporting additional values

  - FFS: Supported values for SSB to CORESET#0 offset RBs

- If 480kHz SSB SCS that configures CORESET#0 and Type0-PDCCH CSS in MIB is agreed to be supported,

  - Support {SS/PBCH Block, CORESET#0 for Type0-PDCCH} SCS equal to {480, 480} kHz

- If 960 kHz SSB SCS that configures CORESET#0 and Type0-PDCCH CSS in MIB is agreed to be supported,

  - Support {SS/PBCH Block, CORESET#0 for Type0-PDCCH} SCS equal to {960, 960} kHz

- If 240 kHz SSB SCS is agreed to be supported,

  - Support {SS/PBCH Block, CORESET#0 for Type0-PDCCH} SCS equal to {240, 120} kHz

- FFS: any other combinations between one of SSB SCS (120, 240, 480, 960) and one of CORESET#0 SCS (120, 480, 960)

  - FFS: initial timing resolution based on low SCS (120 kHz) and its impact on the performance of higher SCS (480/960 kHz)

Agreement:

For 480 kHz and 960 kHz SSB SCS (if agreed)

- Study further on reserving symbol gap between SSB positions with different SSB index (and possibly between SSB position and other signal/channels)

  - FFS: whether symbol gap is needed for only 960 kHz or both 480 and 960 kHz.

- Study further on reserving gap for UL/DL switching within the pattern accounting possibility for reserving UL transmission occasions in the SSB pattern

- Study should account for inputs from RAN4

Agreement:

- For initial access and non-initial access use cases, support 120kHz PRACH SCS with sequence length L=571, 1151 (in addition to L=139) for PRACH Formats A1~A3, B1~B4, C0, and C2.

- For non-initial access use cases,

  - if 480kHz and/or 960 kHz SSB SCS is agreed to be supported, support 480 and/or 960 kHz PRACH SCS with sequence length L=139 for PRACH Formats A1~A3, B1~B4, C0, and C2, respectively.

    - FFS: support of sequence length L = 571, 1151

- FFS: Support of 480 and/or 960 kHz PRACH SCS for initial access use cases, if 480 and/or 960 kHz SSB SCS is agreed to be supported for initial access

Agreement:

If 480 and/or 960 kHz PRACH SCS is supported, RAN1 should study whether or not the current RA-RNTI calculation and PRACH identification in RAR correctly provides unique identification of PRACH.

## RAN1 #104-bis-e

Agreement:

For the case where SSB location and SCS are explicitly provided to the UE (non-initial access) and SSB does not configure Type-0 PDCCH, support 480 kHz and 960 kHz numerologies for the SSB

- Note: Strive to minimize specification impact due to the new SCS for SSB

Agreement:

- For operation with shared spectrum channel access of NR 52.6 – 71 GHz, support discovery burst (DB) and define the DB same as in Rel-16 37.213 Section 4.0

- FFS: Support discovery burst transmission window (DBTW) at least for SSB with 120 kHz SCS with the following requirements

  - PBCH payload size is no greater than that for FR2

  - Duration of DBTW is no greater than 5 ms

  - Number of PBCH DMRS sequences is the same as for FR2

  - FFS: applicability of DBTW design for 120kHz to SSB with 480kHz and 960kHz SCS

  - Support mechanism to indicate or inform that DBTW is enabled/disabled for both IDLE and CONNECTED mode UEs

    - FFS: how to support UEs performing initial access that do not have any prior information on DBTW.

    - FFS: details of the mechanism for enabling/disabling DBTW considering LBT exempt operation and overlapping licensed/unlicensed bands

    - FFS: details of how to inform UEs of the configuration of DBTW

Agreement:

For SSB with 120kHz SCS for NR 52.6 GHz to 71 GHz,

- 120 kHz SCS: the first symbols of the candidate SS/PBCH blocks have indexes {4, 8,16, 20} + 28×n, where index 0 corresponds to the first symbol of the first slot in a half-frame.

- For carrier frequencies within 52.6 GHz to 71GHz, support at least 𝑛 = 0, 1, 2, 3, 5, 6, 7, 8, 10, 11, 12, 13, 15, 16, 17, 18.

  - Other values of n (if any) are FFS, and support of additional n values are subject to support of DBTW for 120kHz SSB

Agreement:

- PRACH configuration for 480/960 kHz SCS (if agreed)

  - The minimum PRACH configuration period is 10 ms (as in FR2)

  - For RO configuration for PRACH with 480/960kHz SCS,

    - FFS: details of how to configure the 480/960 kHz PRACH ROs using [60 or 120 kHz] reference slot considering at least:

      - location of 480/960 kHz PRACH slot per reference slot

      - location of duration containing 480/960khz PRACH slot pattern within 10ms

      - potential impact to RA-RNTI calculation

## RAN1 #105-e

Agreement:

For 480kHz/960kHz SSB, select one of the following alternatives:

- ALT 1) First symbols of the candidate SSB have index {X, Y} + 14*n, where index 0 corresponds to the first symbol of the first slot in a half-frame

  - value of X and Y are identical for 480kHz and 960kHz

    - FFS: exact value of X and Y

- ALT 2) First symbols of the candidate SSB have index {4, 8, 16,20} + 28*n, where index 0 corresponds to the first symbol of the first slot in a half-frame

- Values of n for 480kHz and 960kHz for ALT 1 and 2

  - FFS: whether number of values for ‘n’ depend on LBT operation (i.e. LBT vs no-LBT)

  - FFS: exact values of ‘n’ for each SCS

  - Values of ‘n’ for one mode of operation shall be strictly a subset of values for another mode of operation, if two mode of operation exist for number of candidate SSBs

  - FFS: whether values of ‘n’ shall not be all consecutive integer values (i.e. non-candidate SSB slots are positioned every few candidate SSB slots)

Proposal:

In addition to 120kHz, support 480 kHz SSB for initial access with support of CORESET0/Type0-PDCCH configuration in the MIB with following constraints.

- Limited sync raster entry numbers

  - It is assumed that RAN4 supports a channelization design which results in the total number of synchronization raster entries considering both licensed and unlicensed operation in a 52.6 – 71 GHz band no larger than 665 (Note: the total number of synchronization raster entries in FR2 for band n259 + n261 is 602). If the assumption cannot be satisfied, it’s up to RAN4 to decide its applicability to bands in 52.6 – 71 GHz.

- only 480kHz CORESTE#0/Type0-PDCCH SCS supported for 480 kHz SSB SCS.

- SSB time domain candidate resource pattern (within a slot or pair of slots) for 480 and 960kHz SSB are identical

- Prioritize support SSB-CORESET0 multiplexing pattern 1. Other patterns discussed on a best effort basis.

- Note: Strive to minimize specification impact by reusing tables for CORESET#0 and type0-PDCCH CSS set configuration defined for FR2 in Rel-15, as much as possible

Formal objection sustained by: Huawei, MediaTek (would like to discuss at next meeting)

Proposal:

In addition to 120kHz, support both 480 and 960 kHz SSB for initial access with support of CORESET0/Type0-PDCCH configuration in the MIB with following constraints.

- Limited sync raster entry numbers

  - It is assumed that RAN4 supports a channelization design which results in the total number of synchronization raster entries considering both licensed and unlicensed operation in a 52.6 – 71 GHz band no larger than 665 (Note: the total number of synchronization raster entries in FR2 for band n259 + n261 is 602). If the assumption cannot be satisfied, it’s up to RAN4 to decide its applicability to bands in 52.6 – 71 GHz.

- only 1 CORESTE#0/Type0-PDCCH SCS supported for each SSB SCS i.e., (480,480) and (960,960).

- SSB time domain candidate resource pattern (within a slot or pair of slots) for 480 and 960kHz SSB are identical

- Prioritize support SSB-CORESET0 multiplexing pattern 1. Other patterns discussed on a best effort basis.

- Note: Strive to minimize specification impact by reusing tables for CORESET#0 and type0-PDCCH CSS set configuration defined for FR2 in Rel-15, as much as possible

Formal objection sustained by: Huawei, MediaTek (object to 960 kHz)

Proposal:

To support ANR and PCI confusion detection for 480/960kHz SCS based SSB, support CORESET#0/Type0-PDCCH configuration in MIB of 480 and 960kHz SSB

- FFS: additional method(s) to enable support to obtain neighbor cell PCI and SIB1 contents related to CGI reporting

- Only 1 CORESTE#0/Type0-PDCCH SCS supported for each SSB SCS, i.e., (480,480) and (960,960).

- Prioritize support SSB-CORESET0 multiplexing pattern 1. Other patterns discussed on a best effort basis.

- Note: Strive to minimize specification impact by reusing tables for CORESET#0 and type0-PDCCH CSS set configuration defined for FR2 in Rel-15, as much as possible

- Note: From UE perspective, ANR detection for 480/960kHz SCS based SSB is not supported if the UE does not support 480/960 SCS for SSB.

- Note: for ANR, when reading the MIB, the cell containing the SSB is known to the UE, as defined in 38.133 specification.

Formal objection sustained by: Huawei

Agreement:

For the case agreed in RAN1 #104bis-e where 480/960 kHz SSB location and SCS are explicitly provided to the UE (non-initial access)

- Support configuring CORESET#0/Type0-PDCCH for the purpose of ANR/PCI confusion detection by down selecting from the following two alternatives

  - Alt 1) Using dedicated signaling

  - Alt 2) Using configuration in MIB

    - Note: for ANR, when reading the MIB, the cell containing the SSB is known to the UE, as defined in 38.133 specification.

Agreement:

For 480kHz and 960kHz PRACH,

- Down-select among option 1 and 2

  - Option 1) The reference slot duration corresponds to 60 kHz SCS. A PRACH slot index, nslotRA , corresponds to one of the starting 480/960 kHz PRACH slots within the reference slot.

    - FFS: supported values of the starting PRACH slot index nslotRA  within reference slot and whether or not the ROs for a given PRACH configuration can span more than one PRACH slot if gaps between consecutive ROs are supported for LBT and/or beam switching purposes

  - Option 2) Each 120kHz RO corresponds to 4 and 8 candidate RO positions for 480kHz and 960kHz PRACH, respectively. Information about the number and locations of 480/960kHz candidate RO(s) are configured or pre-selected within each 120kHz RO. The reference 120kHz RO is determined by the current PRACH configuration method in Rel-15/16 specification.

- Following alternatives are considered on PRACH density

  - ALT 1) At least the same density (i.e. number of PRACH slots per reference slot) as for 120kHz PRACH in FR2 is supported

    - FFS: support for higher PRACH slot density (number of PRACH slots per reference slot)

  - ALT 2) at least the same RO density (i.e. number of RO per reference slot) as for 120kHz PRACH in FR2 is supported

    - FFS: support for higher RO density

  - An “example” illustration of PRACH slots for 480/960kHz is shown below:

- FFS: whether and how to account for LBT in RO configuration (if needed)

- FFS: whether and how to account for beam switching gap in RO configuration (if needed)

Agreement:

FFS: Support DBTW at least for 120kHz

- FFS whether DBTW will be applicable for 480/960 kHz SSB SCS

  - If DBTW is supported for 480/960kHz SSB:

    - For the case agreed in RAN1 #104bis-e where 480/960 kHz SSB location and SCS are explicitly provided to the UE (non-initial access), indication of DBTW configuration (e.g. enable/disable of DBTW,  NSSBQCL, and DBTW length) are supported by dedicated signaling.

- For 120kHz SSB, support mechanism to distinguish at least the following scenarios:

  - Case 1) (Unlicensed with LBT off) + DBTW disabled

  - Case 2) (Unlicensed with LBT on) + DBTW enabled

  - Case 3) (Unlicensed with LBT on) + DBTW disabled

  - Case 4) (Licensed) + DBTW disabled

  - FFS: Whether/how LBT on/off is indicated in MIB

    - If not indicated in MIB, then FFS whether/how the UE determines different sizes of DCI 1_0 with CRC scrambled by SI-RNTI

  - FFS: whether any case(s) can be combined for DBTW signaling design and how to handle implications to DCI 1_0 size ambiguity if is not distinguished in signaling

  - FFS: whether all above cases need an explicit indication

  - FFS: Whether a single indication can be used for combination of more than one cases

- For 120 kHz SSB, enable/disable of DBTW is indicated by one or more of the following methods:

  - Option 1) signaling in MIB

    - Option 1-1) disabling DBTW is jointly coded with NSSBQCL

    - Option 1-2) indicated by other bit fields in MIB

    - FFS: among options 1-1 and 1-2

  - Option 2) distinct GSCN used by the SSB

  - Option 3) By comparing the value of  NSSBQCL in MIB and DBTW length after UE reads SIB1 or by comparing the value of  NSSBQCL in MIB and default DBTW length of 5 ms before UE reads SIB1.

  - FFS: whether to support option 1, 2, 3, or any combination of the options.

  - Note: enable/disable signaling of DBTW by MIB or GSCN does not preclude other signaling methods

Agreement:

If DBTW is supported,

- Working assumption: MIB signaling to support

  - Alt A) indication of NSSBQCL at least for 120kHz SSB

    - In this case, the total number of values of NSSBQCL to not exceed 4

  - Alt B) Explicit indication of SSB index and/or SSB candidate location

    - FFS on the details of signaling

  - FFS between Alt A, or B, or supporting both

- Supported DBTW lengths

  - Alt 1) 0.5, 1, 2, 3, 4, 5 msec

    - Note: same as Rel-16 FR1 NR-U

  - Alt 2) maximum 5 msec

    - FFS other values

  - FFS between Alt 1 and 2

- Number of candidate positions when DBTW is enabled

  - For 120kHz SSB

    - FFS between 64 or 80

  - If DBTW is additionally supported for 480/960kHz SSB

    - FFS between 64 or 128

## RAN1 #106-e

Conclusion:

RAN1 will continue discussions to develop solutions for supporting DBTW

Agreement:

- For 480 and 960kHz PRACH:

  - The reference slot duration corresponds to 60 kHz SCS. A PRACH slot index, nslotRA , corresponds to one of the starting 480/960 kHz PRACH slots within the reference slot.

Agreement:

- For 480kHz and 960kHz sub-carrier spacing, first symbols of the candidate SSB have index {2, X} + 14*n, where index 0 corresponds to the first symbol of the first slot in a half-frame.

- Alt 1: X = 8

- Alt 2: X = 9

Agreement:

For 480kHz and 960kHz sub-carrier spacing, first symbols of the candidate SSB have index {2, 9} + 14*n, where index 0 corresponds to the first symbol of the first slot in a half-frame.

Working assumption:

For 120kHz SSB, the number of candidates SSBs in a half frame is 64.

Agreement:

For DBTW with 120kHz SCS (if supported), support DBTW lengths {0.5, 1, 2, 3, 4, 5} msec

- Note: this should be the same as Rel-16 NR-U DBTW lengths.

Agreement:

For ‘controlResourceSetZero’ configuration for {SSB, CORESET#0/Type0-PDCCH} = {480, 480} kHz and {960, 960} kHz,

- Support the following set of parameters.

| SS/PBCH block and CORESET multiplexing pattern | Number of RBs | Number of Symbols |
| --- | --- | --- |
| 1 | 24 | 2 |
| 1 | 48 | 1 |
| 1 | 48 | 2 |

  - Note: the number of entries corresponding the same {mux pattern, number of RB, number of symbol} tuple (listed above) will depend on required RB offsets that needs to be supported based on channel and sync raster design.

- FFS: addition other set of parameters

Agreement:

Do not support PRACH length L=571, 1151 for 960kHz PRACH and at least L =1151 for 480kHz PRACH.

Agreement:

For 480 and 960kHz PRACH:

- At least the same RO density in time domain (i.e. number of specified RO per reference slot according the PRACH configuration index) as for 120kHz PRACH in FR2 is supported

  - FFS: Support gap between consecutive ROs in time domain and the details to derive the gap

Agreement:

For 480 and 960kHz PRACH,

- When a PRACH slot can contain all time domain PRACH occasions corresponding to a PRACH Config. Index in Table 6.3.3.2-4 of 38.211 including gap(s) between consecutive PRACH occasions (if supported) to account for LBT and/or beam switching,

  - and when number of PRACH slots in a reference slot is 1,

    - nslotRA=[7] for 480kHz and nslotRA=[15] for 960kHz PRACH

  - and when the number of PRACH slots in a reference slot is 2,

    - nslotRA=[3,7] for 480kHz and nslotRA=[7,15] for 960kHz PRACH

- FFS: nslotRA values, when a PRACH slot cannot contain all time domain PRACH occasions, corresponding to a PRACH Config. Index in Table 6.3.3.2-4 of 38.211 including gap(s) between consecutive PRACH occasions (if supported) to account for LBT and/or beam switching.

- FFS: whether to allow for additional nslotRA values if the maximum that can be configured for the number of FD RO’s is less than 8 (due to BW limitation)

## RAN1 #106-bis-e

Working assumption:

Support DBTW for 120 kHz.

- FFS: Support for 480 kHz and 960 kHz

Conclusion:

Do not support gap between consecutive ROs for 480kHz and 960kHz

Agreement:

Same DCI size for DCI 1_0 in CSS regardless of channel access mode (i.e., LBT on/off).

- Existing DCI size alignment in TS38.212 applies to DCI 1_0 and 0_0 in CSS.

Agreement:

- Indication of licensed and unlicensed operation is not explicitly indicated in MIB or PBCH payload.

  - FFS: Whether or not to indicate licensed regime by different synchronization raster entries.

- Indication of use of LBT or no-LBT is not explicitly indicated in MIB or PBCH payload.

Agreement:

No other values of n other than agreed previously is supported for 120kHz SCS, where parameter ‘n’ is the set of values to determine the first symbols of the candidate SSB blocks for 120kHz SCS in agreement from RAN1 #104-bis-e.

Working assumption:

- For {SSB, CORESET#0/Type0-PDCCH} = {120, 120} kHz, support multiplexing pattern 1 with 96 PRB CORESET#0, and {1, 2} symbol durations

- Note: the working assumption can be confirmed once RAN1 agrees on the number of needed SSB-CORESET0 offsets for 24 and 48 RB CORESET0 based on RAN4 channelization design

Agreement:

Additionally, support PRACH length L=571 for 480kHz

Agreement:

Support 120 kHz and 480 kHz subcarrier spacing for initial UL BWP for PCell.

Working assumption:

For SCS that DBTW is supported, the following fields are used to indicate parameters related to operation of DBTW

- If only 1 bit is needed: subCarrierSpacingCommon

- If 2 bits is needed: subCarrierSpacingCommon, and 1 bit from pdcch-ConfigSIB1 (pending CORESET0 or search space design would allows for this bit), else, use the spare-bit (not the Msg Extension bit)

  - The design of CORESET0 and search space shall be done without any consideration to this proposal

  - If 2 bits are needed for both 120kHz and 480/960kHz cases, then use the same bit field combination (i.e. use pdcch-ConfigSIB1 bit for 120/480/960 kHz or spare-bit for 120/480.960 kHz)

  - Note: If pdcch-ConfigSIB1 bit is used, the use of controlResourceSetZero (searchSpaceZero) for 120 kHz and   searchSpaceZero (controlResourceSetZero) for 480/960 kHz is not precluded

- FFS: if 3 bits are required

- Note: the working assumption can be confirmed after RAN1 agrees on the number of needed SSB-CORESET0 offsets based on RAN4 channelization design

Agreement:

For 120kHz SCS, for  values:

- If 2 bits are available in MIB for , at least support {16, 32, 64}

- If 1 bit is available in MIB for , support {32, 64}

  - FFS: methods to indicate more  values without increasing used number of bits, e.g., {16, 32, 64}

- Note: value  < 64 indicates DBTW enabled/supported and operation with shared spectrum.

- Note: For operation without shared spectrum channel access, a UE expects to be configured with  = 64. Use of =64 in shared spectrum is not precluded.

- FFS: 1 bit or 2 bits used for

Agreement:

Supported value of n for 480/960kHz SSB slot pattern:

- ALT A) non-contiguous, N slot gap (slots that do not contain SSB) every M slots that contain SSB

  - same pattern will apply to 480kHz and 960kHz (i.e same N and M for 480 and 960 kHz)

  - N = 2, M = 8

  - FFS: starting position of n

- ALT B) non-contiguous, N slot gap (slots that do not contain SSB) every M slots that contain SSB

  - scaled version pattern will apply between 480 and 960 kHz (i.e. N and M for 480kHz, 2N and 2M for 960 kHz)

  - N = 2, M = 8

  - FFS: starting position of n

- ALT C) slots that do not contain SSB correspond to the slots that do not contain SSB in 120 kHz Case D.

  - Note: ALT 4 means that only slots 32-39 for 480 kHz SSB pattern are reserved for UL and 960 kHz SSB pattern is contiguous.

Agreement:

For ‘searchSpaceZero’ configuration for {SSB, CORESET#0/Type0-PDCCH} = {480, 480} kHz and {960, 960} kHz, use the following table for multiplexing pattern 1:

- FFS: The value of X (> 0)

- FFS: whether or not to use different X value depending on whether DBTW is ON/OFF

- FFS: whether or not to use same or different X value for 480 and 960 kHz

- FFS: whether Y = , or Y=, or whether to remove entries with Y

| Index |  | Number of search space sets per slot |  | First symbol index |
| --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 1 | 0 |
| 1 | 0 | 2 | 1/2 | {0, if is even}, {7, if is odd} |
| 2 | X | 1 | 1 | 0 |
| 3 | X | 2 | 1/2 | {0, if is even}, {7, if is odd} |
| 4 | 5 | 1 | 1 | 0 |
| 5 | 5 | 2 | 1/2 | {0, if is even}, {7, if is odd} |
| 6 | 0 | 2 | 1/2 | {0, if is even}, {Y, if is odd} |
| 7 | X | 2 | 1/2 | {0, if is even}, {Y, if is odd} |
| 8 | 5 | 2 | 1/2 | {0, if is even}, {Y, if is odd} |
| 9 | 5 + X | 1 | 1 | 0 |
| 10 | 5 + X | 2 | 1/2 | {0, if is even}, {7, if is odd} |
| 11 | 5 + X | 2 | 1/2 | {0, if is even}, {Y, if is odd} |
| 12 | 0 | 1 | 2 | 0 |
| 13 | 5 | 1 | 2 | 0 |
| 14 | Reserved |  |  |  |
| 15 | Reserved |  |  |  |

## RAN1 #107-e

Agreement

- Support DBTW with 480 and 960 kHz SCS.

- For licensed and unlicensed operation, support 64 candidate SSB positions in a half frame

- Working assumption: Use 2 bits for Q:

  - SubcarrierSpacingCommon

  - spare bit in MIB

- Send LS to RAN2 for confirming the use of the spare bit in MIB

  - The use of 2 bits for Q can be revisited if RAN2 tells RAN1 that the spare bit cannot be used

R1-2112614	[Draft] LS on initial access for 60 GHz	Intel Corporation

Final LS endorsed in R1-2112805.

Agreement

Confirm the following working assumptions:

- (From #106-bis-e) Support DBTW for 120 kHz.

- (From #106-e) For 120kHz SSB, the number of candidates SSBs in a half frame is 64.

Agreement

For SCS that support DBTW, UE derives the QCL relation between candidate SSBs by the value of i modNSSBQCL , where i is the candidate SSB index.

Conclusion

- The bit-width of ssb-PositionsInBurst in SIB1 and ServingCellConfigCommon is kept the same as in Rel-15 (i.e., 16-bits in SIB1 and 64-bits in ServingCellConfigCommon).

Agreement

If multiplexing pattern 3 for 480 and 960 kHz is supported, the TDRA allocation table C is updated as follows:

- Row index 6 (previously reserved) is set to

  - Dmrs-TypeA-Position: 2,3

  - PDSCH mapping type: Type B

  - K0 : 0

  - S = 11

  - L = 2

Agreement

Finalizing PRACH slot index for 480 and 960 kHz (removal of bracket of previous agreement)

- when number of PRACH slots in a reference slot is 1,

  - nslotRA=7 for 480kHz and nslotRA=15 for 960kHz PRACH

- when the number of PRACH slots in a reference slot is 2,

  - nslotRA=3,7 for 480kHz and nslotRA=7,15 for 960kHz PRACH

Agreement

Update the Table 8.1-2 in TS38.213 to indicate the Ngap (gap between valid RO and SS/PBCH) for 480 kHz and 960 kHz SCS as follows:

- Ngap=8 for 480 kHz

- Ngap=16 for 960 kHz;

Agreement

For single cell operation or for operation with carrier aggregation in a same frequency band, a UE does not transmit PRACH and PUSCH/PUCCH/SRS in a same slot or when a gap between the first or last symbol of a PRACH transmission in a first slot is separated by less than 𝑁 symbols from the last or first symbol, respectively, of a PUSCH/PUCCH/SRS transmission in a second slot where 𝑁=16 for 𝜇=5, 𝑁=32 for 𝜇=6, and 𝜇 is the SCS configuration for the active UL BWP. For a PUSCH transmission with repetition Type B, this applies to each actual repetition for PUSCH transmission [6, TS 38.214].

Conclusion:

ΔDelay as part of gap between last symbol of PDCCH order reception and first symbol of the PRACH transmission for FR2-2 uses the same value as FR2-1 (i.e. single value for FR2).

Agreement

- For 480 kHz, slot index, n, that contain SSB are:

  - n = {0,1,2,3,4,5,6,7, 8,9,10,11,12,13,14,15, 16,17,18,19,20,21,22,23, 24,25,26,27,28,29,30,31}

- For 960 kHz, slot index, n, that contain SSB are:

  - n = {0,1,2,3,4,5,6,7, 8,9,10,11,12,13,14,15, 16,17,18,19,20,21,22,23, 24,25,26,27,28,29,30,31}

Agreement

For ‘searchSpaceZero’ configuration for {SSB, CORESET#0/Type0-PDCCH} = {120, 120} kHz,

- use Table 13-12 in TS38.213 for multiplexing pattern 1,

- use Table 13-15 in TS38.213 for multiplexing pattern 3.

Agreement

For ‘searchSpaceZero’ configuration for {SSB, CORESET#0/Type0-PDCCH} = {480, 480} kHz and {960, 960} kHz, parameter X from previous RAN1 agreement is set to:

- X = 1.25 for 480 kHz

- X = 0.625 for 960 kHz

Conclusion:

For FR2-2, support the same mechanism as in Rel-16 for extended RAR window for both 4-step and 2-step RACH.

Agreement

For a Type-2 random access procedure, a UE transmits a PUSCH, when applicable, after transmitting a PRACH. The UE encodes a transport block provided for the PUSCH transmission using redundancy version number 0. The PUSCH transmission is after the PRACH transmission by at least N symbols where N=16 for μ=5 and N=32 for μ=6, and μ is the SCS configuration for the active UL BWP.

Agreement

For 480 and 960 kHz, supported DBTW lengths are:

- {1.25, 1, 0.75, 0.5, 0.25, 0.125, X} ms, where X = 0.0625 if Q=8 is supported and X is removed if Q=8 is not supported.

Agreement

SSB-PositionQCL-Relation IE to indicate QCL relationship between SSB positions for FR2-2 are same set of values supported for NSSBQCL in MIB.

Agreement

For operation with shared spectrum access, for SS/PBCH block and CORESET#0 multiplexing pattern 3, a UE monitors PDCCH in the Type0-PDCCH CSS set over slots that include Type0-PDCCH monitoring occasions associated with SS/PBCH blocks that are quasi co-located with the SS/PBCH block that provides a CORESET for Type0-PDCCH CSS set.

Agreement

For ‘searchSpaceZero’ configuration for {SSB, CORESET#0/Type0-PDCCH} = {480, 480} kHz and {960, 960} kHz, parameter Y from previous RAN1 agreement is Y = NsymbCORESET.

Agreement

- For 480kHz and 960kHz PRACH, reuse the RA-RNTI and MSGB-RNTI formula as FR2 and express the slot indexes t_id based on 120kHz SCS:

  - RA-RNTI =1+s_id+14×t_id+14×80×f_id +14×80×8×ul_carrier_id

  - MSGB-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id + 14 × 80 × 8 × 2

    - where the subcarrier spacing to determine t_id is based on the value of µ specified in clause 5.3.2 in TS 38.211 [8] for µ = {0, 1, 2, 3}

    - for µ = {5, 6}, t_id is the index of the 120 kHz slot in a system frame that contains the PRACH occasion (0 ≤ t_id < 80).

  - Note: As per previous RAN1 agreement, there is only one 480 or 960 kHz PRACH slot in a 120kHz slot, such that RA-RNTI and MSGB-RNTI does not result in ID collision.

- Send LS to RAN2 on the updates on RA-RNTI and MSGB-RNTI.

R1-2112734	[Draft] LS on RA-RNTI and MSGB-RNTI for 480 and 960 kHz	Intel Corporation

Final LS endorsed in R1-2112832 (with removal of “first” in text referring to the captured agreement)

Agreement

- Same NSSBQCL values using the same set of signaling bits are supported for 120, 480, and 960 kHz.

- Supported values of NSSBQCL: {16, 32, 64}

  - Note:

    - For operation with shared spectrum channel access, any supported value of NSSBQCL can be indicated and value < 64 indicates DBTW enabled

    - UE is expected to be configured with NSSBQCL=64 in licensed operations

    - For operation with and without shared spectrum channel access, NSSBQCL=64 indicates that the SS/PBCH block index and the candidate SS/PBCH block index have a one-to-one mapping relationship.

Working assumption

For ‘controlResourceSetZero’ configuration for {SSB, CORESET#0/Type0-PDCCH} = {480, 480} kHz and {960, 960} kHz,

- After supporting entries for multiplexing pattern 1 for the agreed pairs of (NRBCORESET, NsymbCORESET) ={(24, 2), (48, 1), (48,2)} (with required RB offsets), if additional entries are left, support multiplex pattern 3 with 24 PRB and 2 symbol duration, and multiplexing pattern 3 with 48 PRB and 2 symbol duration.

Working assumption

For ‘controlResourceSetZero’ configuration for {SSB, CORESET#0/Type0-PDCCH} = {480, 480} kHz and {960, 960} kHz,

- After supporting entries for multiplexing pattern 1 for the agreed pairs of (NRBCORESET, NsymbCORESET) ={(24, 2), (48, 1), (48,2)} (with required RB offsets) and multiplex pattern 3 with 24 and 48 PRB and 2 symbol duration (with required RB offsets), if additional entries are left, support multiplexing pattern 1 with 96 PRB and 2 symbol duration

  - Note: the working assumption can be confirmed once RAN1 agrees on the number of needed SSB-CORESET0 offsets for 24 and 48 RB CORESET0 based on RAN4 channelization design.

Agreement

- If NSSBQCL is indicated, the same interpretation of ssb-PositionsInBurst in SIB1 or ServingCellConfigCommon as in Rel-16 is supported, i.e.:

  - A bit set to 1 at position k∈{1..64} indicates SS/PBCH block index k-1

  - The UE assumes that a bit at position k > NSSBQCL is set to 0

    - For ssb-PositionsInBurst in SIB1, the UE assumes that a bit at groupPresence corresponding to a SS/PBCH block index ≥ NSSBQCL is set to 0

  - Note: for ssb-PositionsInBurst in SIB1, position k corresponds to the SS/PBCH block index indicated by a bit in inOneGroup and a bit in groupPresence

- In operation with shared spectrum in 60 GHz, for ssb-PositionsInBurst in ServingCellConfigCommonSIB,

  - for MSB k, k≥1, of inOneGroup and MSB m, m≥1, of groupPresense of ssb-PositionsInBurst:

    - if MSB k of inOneGroup and MSB m of groupPresense are set to 1, the UE assumes that SSB(s) within DBTW with ‘candidate SSB index(es)’ corresponding to ‘SSB index’ equal to k-1+(m-1)×8 may be transmitted;

    - if MSB k of inOneGroup or MSB m of groupPresense is set to 0, the UE assumes that SSB(s) within DBTW with ‘candidate SSB index(es)’ corresponding to ‘SSB index’ equal to k-1+(m-1)×8 is not transmitted;

- In operation with shared spectrum in 60 GHz, for ssb-PositionsInBurst in ServingCellConfigCommon,

  - ssb-PositionsInBurst bits correspond to supported ‘SSB indices’,

    - and UE assumes that SSB(s) within DBTW with ‘candidate SSB index(es)’ corresponding to indicated bit(s) may be transmitted;

    - and UE assumes that SSB(s) within DBTW with ‘candidate SSB index(es)’ corresponding to not indicated bit(s) are not transmitted

- Note to spec editor: The above three bullets maintain the same behavior as Rel-16 NR-U

Agreement

Update the Table 6.3.3.2-1 in TS 38.211 as follows:

- Table 6.3.3.2-1: Supported combinations of ΔfRA and  Δf, and the corresponding value of k.

| LRA | ΔfRA for PRACH | Δf for PUSCH | NRBRA, allocation expressed in number of RBs for PUSCH | k |
| --- | --- | --- | --- | --- |
| ... | ... | ... | ... | ... |
| 139 | 120 | 120 | 12 | 2 |
| 139 | 120 | 480 | 3 | 1 |
| 139 | 120 | 960 | 2 | 23 |
| 139 | 480 | 120 | 48 | 2 |
| 139 | 480 | 480 | 12 | 2 |
| 139 | 480 | 960 | 6 | 2 |
| 139 | 960 | 120 | 96 | 2 |
| 139 | 960 | 480 | 24 | 2 |
| 139 | 960 | 960 | 12 | 2 |
| 571 | 120 | 120 | 48 | 2 |
| 571 | 120 | 480 | 12 | 1 |
| 571 | 120 | 960 | 7 | 47 |
| 571 | 480 | 120 | 192 | 2 |
| 571 | 480 | 480 | 48 | 2 |
| 571 | 480 | 960 | 24 | 2 |
| 1151 | 120 | 120 | 97 | 6 |
| 1151 | 120 | 480 | 25 | 23 |
| 1151 | 120 | 960 | 13 | 45 |

## RAN1 #107-bis-e

Conclusion:

RRC parameters list to capture changes identified below:

- Add the following note to the comment section of discoveryBurstWindowLength-r17 row in RRC parameter list

  - “Note: This parameter is to be included in both SIB1 and the common serving cell configuration parameters”.

- Support adding “SSB-PositionQCL-Relation-r17” to RRC parameter list as both UE-specific and cell-specific parameter.

- Inform RAN2 (by adding notes to RRC parameter list) that the either the value range of the information element SSB-PositionQCL-Relation-r16 needs to be extended, or a new Rel-17 IE need to be defined to allow configuration of Q = 16, 32, or 64 in SIB2, SIB3, SIB4, MeasObjectNR, and ServingCellConfigCommon for RRM measurements when operating with shared spectrum channel access in FR2-2.

The TP below for TS38.213v17.0.0 is endorsed as a working assumption

=========== Unchanged Text Omitted =========== Table 13-15A: PDCCH monitoring occasions for Type0-PDCCH CSS set - SS/PBCH block and CORESET multiplexing pattern 3 and {SS/PBCH block, PDCCH} SCS {480, 480} kHz or {960, 960} kHz Index; PDCCH monitoring occasions (SFN and slot number); First symbol index (k = 0, 1, …, 31). 0; SFNc=SFNSSB,i nc=nSSB,i; 2, 9 in i=2k, i=2k+1. ============ Unchanged Text Omitted ============

The TP below for TS38.211v17.0.0 is endorsed

*** Unchanged text omitted *** 5.3.2 OFDM baseband signal generation for PRACH The time-continuous signal on antenna port p for PRACH is defined by sl(p,μ)t=k=0LRA-1ak(p,RA)ej2πk+Kk1+kΔfRAt-NCP,lRATc-tstartRA K=ΔfΔfRA k1=k0μ+NBWP,istart-Ngridstart,μNscRB-Ngridsize,μNscRB2+nRAstartNscRB+nRANRBRANscRBif LRA∈139, 839 or LRA∈571, 1151 in FR2-2NRB,UL,n0+nRAstart,μ-NRB,UL,n0start,μNscRBif LRA∈571, 1151 in FR1 k0μ=Ngridstart,μ+Ngridsize,μ2NscRB-Ngridstart,μ0+Ngridsize,μ02NscRB2μ0-μ *** Unchanged text omitted ***

The TP below for TS38.214v17.0.0 is endorsed

*** Unchanged text omitted *** 7 UE procedures for transmitting and receiving on a carrier with intra-cell guard bands For operation with shared spectrum channel access in FR1, when the UE is configured with any of IntraCellGuardBandsPerSCS for UL carrier and for DL carrier with SCS configuration μ, the UE is provided with NRB-set,x-1 intra-cell guard bands on a carrier with μ, each defined by start CRB and size in number of CRBs,GB s,xstart,μ andGB s,xsize,μ , provided by higher layer parameters startCRB and nrofCRBs, respectively, wheres∈0,1,…,NRB-set,x-2. The subscript x is set to DL and UL for the downlink and uplink, respectively. Where there is no risk of confusion, the subscript x can be dropped. The intra-cell guard bands separateNRB-set,x RB sets, each defined by start and end CRB,RB s,xstart,μ andRB s,xend,μ, respectively. The UE does not expect that nrofCRBs is configured with non-zero value smaller than the applicable intra-cell guard bands as specified in [8, TS 38.101-1] corresponding toμ and carrier sizeNgrid,xsize,μ. The UE determines the start and end CRB indices for s∈0,1,…,NRB-set,x-1 as *** Unchanged text omitted ***

R1-2200689	Summary #1 of email discussion on initial access aspect of NR extension up to 71 GHz	Moderator (Intel Corporation)

TP# 8-1a for TS38.213 in section 3 of R1-2200689 is endorsed.

Conclusion

RRC parameters list to capture changes identified below

- New parameter, ra-ResponseWindow-r17, under sub-feature group SSB and RACH

  - Value range {sl240, sl320, sl640, sl960, sl1280, sl1920, sl2560}

  - Based on previous conclusion:

    - For FR2-2, support the same mechanism as in Rel-16 for extended RAR window for both 4-step and 2-step RACH.

- New parameter, msgB-ResponseWindow-r17, under sub-feature group SSB and RACH

  - Value range { sl240, sl640, sl960, sl1280, sl1920, sl2560}

  - Based on previous conclusion:

    - For FR2-2, support the same mechanism as in Rel-16 for extended RAR window for both 4-step and 2-step RACH.

- Existing parameter, msgA-PRACH-RootSequenceIndex-r16, under sub-feature group SSB and RACH

  - Description:

    - May not need to change the IE, but need to add in the note on the limitation to be used with SCS. Field description requires updating to capture that L = 1151 is not supported for SCS 480 and 960 kHz and L = 571 is not supported for 960 kHz.

  - Value range:

    - CHOICE { l571 INTEGER {0..569}, l1151 INTEGER {0..1149}}

  - Cell-specific

## RAN1 #108-e

Working assumption

- Use 1 bit for Q in MIB

  - SubcarrierSpacingCommon field will be used to convey value of {32, 64} for operation with shared spectrum channel access

  - Note that this is revising the working assumption made in RAN1#107-e on “use 2 bits for Q, {SubcarrierSpacingCommon, spare bit in MIB}”

Conclusion

Update the ssb-PositionQCL in RRC to {32, 64} values.

- For reference, the following are list of RRC IEs that references ssb-PositionQCL in release 16.

  - SIB2:: ssb-PositionQCL-Common-r16

  - SIB3:: ssb-PositionQCL-r16

  - SIB4:: ssb-PositionQCL-Common-r16

  - SIB4:: ssb-PositionQCL-r16

  - MeasObjectNR:: ssb-PositionQCL-Common-r16

  - MeasObjectNR:: ssb-PositionQCL-r16

  - ServingCellConfigCommon:: ssb-PositionQCL-r16

Text Proposal #1-3 (for 38.213, Section 4.1) in section 3 of R1-2202503 is endorsed.

TP# 1-3 for TS38.213

4 Synchronization procedures 4.1 Cell search < Unchanged parts are omitted > For operation without shared spectrum channel access, an SS/PBCH block index is same as a candidate SS/PBCH block index. < Unchanged parts are omitted > For operation with shared spectrum channel access in FR2-2, a UE assumes that SS/PBCH blocks in a serving cell that are within a same discovery burst transmission window or across discovery burst transmission windows are quasi co-located with respect to average gain, quasi co-location 'typeA' and 'typeD' properties, when applicable, if a value of imodNSSBQCL is same among the SS/PBCH blocks, where i is the candidate SS/PBCH block index. NSSBQCL is either provided by ssb-PositionQCL or, if ssb-PositionQCL is not provided, obtained from a MIB provided by a SS/PBCH block according to Table 4.1-2. The UE can determine an SS/PBCH block index according to i modNSSBQCL. The UE assumes that within a discovery burst transmission window, a number of transmitted SS/PBCH blocks on a serving cell is not larger than NSSBQCL and a number of transmitted SS/PBCH blocks with a same SS/PBCH block index is not larger than one. For operation without shared spectrum channel access in FR2-2, a UE expects subCarrierSpacingCommon = ‘scs30or120’ from a MIB provided by a SS/PBCH block. Table 4.1-2: Mapping between the combination of subCarrierSpacingCommon and spare to NSSBQCL for operation with shared spectrum channel access in FR2-2 subCarrierSpacingCommon; spare; NSSBQCL. scs15or60; 0; 16. scs15or60; 1; 32. scs30or120; 0; 64. scs30or120; 1; reserved. < Unchanged parts are omitted >

Conclusion

- For operation with shared spectrum channel access, support default DBTW length of 5ms if discoveryBurstWindowLength is not provided in FR2-2.

- No change to specification is needed

Working Assumption

- The following table is used for set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {120, 120}, {480, 480}, and {960, 960} kHz for FR2-2.

  - FFS: whether/how to define X, if defined, candidate values {56, 76}

- Text Proposal #4-2D (for 38.213, Section 13) in section 3 of R1-2202503 is endorsed

- Note: this working assumption can be revisited once RAN4 finalizes the further details of the channelization.

| Index | SS/PBCH block and CORESET multiplexing pattern | Number of RBs | Number of Symbols | Offset (RBs) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 24 | 2 | 0 |
| 1 | 1 | 24 | 2 | 4 |
| 2 | 1 | 48 | 1 | 0 |
| 3 | 1 | 48 | 1 | 14 |
| 4 | 1 | 48 | 1 | 28 |
| 5 | 1 | 48 | 2 | 0 |
| 6 | 1 | 48 | 2 | 14 |
| 7 | 1 | 48 | 2 | 28 |
| 8 | 1 | 96 | 1 | 0 |
| 9 | 1 | 96 | 1 | X |
| 10 | 1 | 96 | 2 | 0 |
| 11 | 1 | 96 | 2 | X |
| 12 | 3 | 24 | 2 | -20 if -21 if > 0 |
| 13 | 3 | 24 | 2 | 24 |
| 14 | 3 | 48 | 2 | -20 if -21 if > 0 |
| 15 | 3 | 48 | 2 | 48 |

Text Proposal #5-1A (for 38.215, Section 5.1.3) in section 3 of R1-2202503 is endorsed.

TP# 5-1A for TS38.215

5.1.3 SS reference signal received quality (SS-RSRQ) ======================== Unchanged Text Omitted =========================== Table 5.1.3-1: NR Carrier RSSI measurement symbols OFDM signal indication endSymbol; Symbol indexes. 0; {0,1}. 1; For 480 kHz and 960 kHz {0,1,2,..,10,12}; otherwise {0,1,2,..,10,11}. 2; {0,1,2,…, 5}. 3; {0,1,2,…, 7}. ========================= Unchanged Text Omitted ==============================

Text Proposal #7-2B (for 38.211, Section 5.3.2) in section 3 of R1-2202503 is endorsed.

TP# 7-2B for TS38.211

5.3.2 OFDM baseband signal generation for PRACH < Unchanged parts are omitted > where - is given by the parameter "starting symbol" in Tables 6.3.3.2-2 to 6.3.3.2-4; - is the PRACH transmission occasion within the PRACH slot, numbered in increasing order from 0 to within a RACH slot where is given Tables 6.3.3.2-2 to 6.3.3.2-4 for LRA∈139,571,1151 and fixed to 1 for ; - is given by Tables 6.3.3.2-2 to 6.3.3.2-4; - is given by - if ∆fRA∈1.25, 5, 15, 60 kHz, then - if ∆fRA∈30, 120 kHz and either of "Number of PRACH slots within a subframe" in Tables 6.3.3.2-2 to 6.3.3.2-3 or "Number of PRACH slots within a 60 kHz slot" in Table 6.3.3.2-4 is equal to 1, then nslotRA=1, otherwise nslotRA∈0,1 {indent backwards here}- if ∆fRA∈480, 960 kHz and - the "Number of PRACH slots within a 60 kHz slot" in Table 6.3.3.2-4 is equal to 1, then nslotRA=7 for ∆fRA=480 kHz and nslotRA=15 for ∆fRA=960 kHz - the "Number of PRACH slots within a 60 kHz slot" in Table 6.3.3.2-4 is equal to 1, then nslotRA=7 for ∆fRA=480 kHz and nslotRA=15 for ∆fRA=960 kHz, or - the "Number of PRACH slots within a 60 kHz slot" in Table 6.3.3.2-4 is equal to 2, then nslotRA∈3,7 for ∆fRA=480 kHz and nslotRA∈7,15 for ∆fRA=960 kHz. < Unchanged parts are omitted >

Text Proposal #7-3B (for 38.211, Section 7.4.3.1) in section 3 of R1-2202503 is endorsed.

TP# 7-3B for TS38.211

#### 7.4.3.1 Time-frequency structure of an SS/PBCH block ============= Unchanged Text Omitted ============= - For operation with shared spectrum channel access in FR2-2 and fFor operation without shared spectrum channel access and for operation with shared spectrum channel access in FR2-2, the 4 least significant bits of are given by the higher-layer parameter ssb-SubcarrierOffset and for FR1 the most significant bit of is given by aA+5 in the PBCH payload as defined in clause 7.1.1 of [4, TS 38.212]. - For operation with shared spectrum channel access in FR1, the 4 least significant bits of kSSB are given by the higher-layer parameter ssb-SubcarrierOffset and the most significant bit of kSSB is given by aA+5 in the PBCH payload as defined in clause 7.1.1 of [4, TS 38.212]. If kSSB≥24, kSSB=kSSB ; otherwise, kSSB=2kSSB2, and if ssb-SubcarrierOffset is not provided, kSSB is derived from the frequency difference between the SS/PBCH block and Point A. ============= Unchanged Text Omitted =====================

Text Proposal #7-4B (for 38.213, Section 13) in section 3 of R1-2202503 is endorsed.

TP# 7-4B for TS38.213

13 UE procedure for monitoring Type0-PDCCH CSS sets ============= Unchanged Text Omitted ============= For operation with shared spectrum channel access in FR2-2 and fFor operation without shared spectrum channel access and for operation with shared spectrum channel access in FR2-2, a UE assumes that the offset in Tables 13-1 through 13-10C is defined with respect to the SCS of the CORESET for Type0-PDCCH CSS set from the smallest RB index of the CORESET for Type0-PDCCH CSS set to the smallest RB index of the common RB overlapping with the first RB of the corresponding SS/PBCH block. The SCS of the CORESET for Type0-PDCCH CSS set is provided by subCarrierSpacingCommon for FR1 and FR2-1 and same as the SCS of the corresponding SS/PBCH block for FR2-2. In Tables 13-7, 13-8, and 13-10 kSSB is defined in [4, TS 38.211]. ============= Unchanged Text Omitted =====================