3GPP TSG RAN WG1 #109-e                                                              R1-2205536

e-Meeting, May 9th – 20th, 2022

Source:	Moderator (Intel Corporation)

Title:	Endorsed TP for Rel.17 NR FeMIMO maintenance on HST-SFN

Agenda item:	8.1.2

Document for:	Discussion and Decision

# Summary of Endorsed TPs

The following TP for TS 38.214 was discussed in the email thread [109-e-R17-MIMO-06] Maintenance on HST (description of issues in R1-2205145) – Avik (Intel).

The text proposal was endorsed by RAN1 Chairman over email. The new additions to current specification are in red text.

TP for TS 38.214:

Reason for change: The combination of Rel-17 SFN PDCCH scheme 1 and single-TRP PDSCH is supported as an optional UE feature [nonSfnPdsch-sfnPdcch]. Meanwhile, the combination of Rel-17 SFN PDCCH TRP-based pre-compensation and single-TRP PDSCH is not supported. However, in the current TS 38.214, the description of these mentioned combinations has not been captured.

Summary of change: To describe the combination of SFN PDCCH scheme 1 and single-TRP PDSCH is supported, and the combination of SFN PDCCH TRP-based pre-compensation and single-TRP PDSCH is not supported.

Consequences if not approved: Current RAN1 agreement will not have been correctly reflected in specification and it would impact UE complexity and reception.

TP for TS 38.214 Section 5.1 5.1 UE procedure for receiving the physical downlink shared channel < Unchanged parts are omitted > When a UE is configured with higher layer parameter sfnSchemePdsch set to either 'sfnSchemeA' or 'sfnSchemeB' for a DL BWP and - if the UE reports its capability of [dynamicSFN], the UE is indicated with one or two TCI state(s) in a codepoint of the DCI field 'Transmission Configuration Indication' in DCI format 1_1/1_2, or - otherwise, the UE is not expected to be indicated with one TCI state per any of TCI codepoint by MAC CE, and the UE is indicated with two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication' in DCI format 1_1/1_2, and the UE procedure for receiving the PDSCH upon detection of a PDCCH follows clause 5.1 and the QCL assumption for the PDSCH as defined in clause 5.1.5. When a UE is configured with both sfnSchemePdsch and sfnSchemePdcch, the UE shall expect that sfnSchemePdsch and sfnSchemePdcch are set to the same scheme, either 'sfnSchemeA' or 'sfnSchemeB'. If a UE is configured with sfnSchemePdcch set to 'sfnSchemeA' for a DL BWP and activated with two TCI states by MAC CE, and the UE does not report its capability of [nonSfnPdsch-sfnPdcch], the UE is expected to be configured with sfnSchemePdsch set to 'sfnSchemeA' and indicated with two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication', if the PDSCH is scheduled by DCI format 1_1/1_2. If a UE is configured with sfnSchemePdcch set to 'sfnSchemeB' for a DL BWP and activated with two TCI states by MAC CE, the UE is expected to be configured with sfnSchemePdsch set to 'sfnSchemeB' and indicated with two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication', if the PDSCH is scheduled by DCI format 1_1/1_2. < Unchanged parts are omitted >