3GPP TSG RAN WG1 #109-e	   R1-2205540

e-Meeting, May 9th – 20th, 2022

Agenda item:	9.3.1

Source: 		Moderator (CMCC)

Title: 	Summary# of email discussion on evaluation of NR duplex evolution

Document for:	Discussion/decision

# 1 Introduction

The SI Study on evolution of NR duplex operation was approved in RAN plenary #94-e meeting [1], and the SID was revised in RAN plenary #95 e-meeting [2].

The following email thread for AI 9.3.1 Evaluation on NR duplex evolution is announced by chairman in RAN1#109-e:

[109-e-R18-Duplex-02] Email discussion on evaluation of NR duplex evolution by May 20 – Fei (CMCC)

- Check points: May 12, May 18, May 20

In this contribution, we summarized the related issues and proposals based on the contributions submitted in RAN1#109-e under the agenda item 9.3.1 [3]-[25].

The following sections are structured as follows. From section 2 to 5, we categorize the key issues raised by contributions into 4 kinds and some sections may cover more than one sub-issue. For each issue/sub-issue, we provide the background and related proposals, the summary of the proposals, and initial proposals/questions suggested by moderator in sub-sections. For each identified proposal/question, one table is provided to collect company views during the email discussion. The stable proposals will be moved to section 6 as discussion progresses. In section 7, some proposals will be selected for discussion in the GTW session.

If possible, please try to provide your replies within 24h. Moderator will try to update the proposals based on companies’ inputs on a daily basis.

# 2 Issue#1: Deployment scenarios for SBFD

## 2.1 Submitted proposal

| Company | Proposals |
| --- | --- |
| CMCC | Proposal 1: Focus on the following scenarios for SBFD evaluation: - Scenario A1 (FR1, FR2): Indoor hotspot with common UL/DL subband configuration - Scenario A2 (FR1, FR2): Urban Micro or Dense Urban micro layer with common UL/DL subband configuration - Scenario A3 (FR1): Urban Macro with common UL/DL subband configuration - Scenario C3 (FR1): Urban Macro considering co-channel co-existence with legacy TDD operation - Scenario D3 (FR1): Urban Macro considering adjacent-channel co-existence with legacy TDD operation Observation 3: For Scenario C3 (Urban Macro considering co-channel co-existence with legacy TDD operation), no system level simulation is needed, and the DL performance impact of SBFD operation on the legacy TDD can be simply analyzed based on the reduction of the available DL frequency resources due to resource muting for legacy TDD. Proposal 25: No system level simulation is needed for Scenario C3 (Urban Macro considering co-channel co-existence with legacy TDD operation). Proposal 26: It is not necessary for RAN1 to perform the performance evaluation for adjacent-channel co-existence between SBFD operation and legacy TDD operation. |
| Huawei | Observation 2: Low latency is the dominant benefit for subband non-overlapping full duplex under the factory/industry deployment scenario. Observation 3: Better uplink coverage is the dominant benefit for subband non-overlapping full duplex under the Macro deployment scenario. Proposal 3: Scenario 3-3 should be studied with a high priority for subband non-overlapping full duplex. - Scenario 3-3: Factory/industry Pico with subband non-overlapping full duplex (and potential Macro with DL dominant TDD configuration) (FR1 only). Proposal 4: Scenario 1-3 should be studied with a high priority for subband non-overlapping full duplex. - Scenario 1-3: Macro with subband non-overlapping full duplex with same resource configurations (FR1 and FR2). Proposal 5: The following deployment scenarios can be studied after the evaluation of other scenarios: - Scenarios 4: Macro with subband non-overlapping full duplex and Macro with legacy TDD configuration on the same carrier (co-channel co-existence with legacy base stations scenarios); - Scenarios 5: Macro with subband non-overlapping full duplex and Macro with legacy TDD configuration on the adjacent carriers (adjacent-channel co-existence or inter-operator co-existence scenarios). |
| ZTE | Proposal 1: Perform thorough analysis and study for sub-band non-overlapping duplex in Rel-18 to lay the foundation for future duplex study for both 5G and 6G. Proposal 3: Rel-18 duplex evolution considers the following 6 challenges of legacy TDD system and evaluate the potential gain of subband full duplex. - Challenge ①: Ensuring UL throughput + UL coverage simultaneously. - Challenge ②: Ensuring UL throughput + DL&UL Latency simultaneously. - Challenge ③: Ensuring UL coverage + DL&UL Latency simultaneously. - Challenge ④: Ensuring DL throughput + DL&UL Latency simultaneously. - Challenge ⑤: Ensuring DL throughput + UL throughput simultaneously. - Challenge ⑥: Ensuring DL throughput + UL coverage simultaneously. Proposal 4: RAN1 studies at least the following scenarios in Rel-18 duplex SI - For subband full duplex: urban macro, dense urban, indoor hotspot and urban macro + indoor hotspot (optional). - For dynamic/flexible TDD: dense urban, and urban macro + indoor hotspot Proposal 5: RAN1 studies the following potential deployment for dynamic TDD and subband full duplex. - Deployment#1 (1st priority): Dynamic TDD, all the gNBs are legacy TDD gNB in the same channel with two different TDD slot formats. - Deployment#2 (1st priority): Subband full duplex, all the gNBs are subband full duplex gNB in the same channel with the same time/frequency pattern without any co-existence co-channel or adjacent channel interference. - Deployment#3 (1st priority): Subband full duplex, all the gNBs in channel#1 are subband full duplex gNB with the same time/frequency pattern. There are also other legacy TDD gNBs in adjacent channel#2. - Deployment#4 (1st priority): Subband full duplex, some gNBs in channel#1 are subband full duplex gNB with the same time/frequency pattern. There are also other legacy TDD gNBs in channel#1. - Deployment#5 (2nd priority): Subband full duplex, gNBs in channel#1 are subband full duplex gNB with the different time/frequency patterns. - Deployment#6 (2nd priority): Isolated case. There is only one single subband full duplex gNB without any interference from other co-channel or adjacent channel gNBs. |
| New H3C | Proposal 4: The following duplex mode options are considered for on NR duplex evolution. - Case 1: Dynamic duplex mode Any resource in duplex manner can be changed dynamically between DL and UL. In the evaluation, depending on the incoming traffic and the scheduler decision, any suband in the slots/symbols for duplex mode can carry DL or UL traffic. Operation based on dynamic TDD is expected to cause so-called cross-link interference where the dominant interference for a transmission in one direction (e.g., downlink) is caused by another transmission in the other direction (e.g., uplink). - Case 2: Static duplex mode A coordinated scheme where the DL:UL ratio for the allocated duplex mode slots/symbols is fixed for some period of time and the same DL:UL ratio is used by all nodes in the network. This scheme for the allocated duplex mode slots/symbols is equivalent to the traditional legacy FDD. In other words, the DL:UL ratio for the allocated duplex mode slots/symbols is the same and synchronous across all the nodes in the network. Operation based on static duplex mode is immune to so-called cross-link interference while the DL to UL ratio for the allocated duplex mode slots follows a static or semi-static structure that is matched to the long-term statistics of the incoming DL to UL traffic ratio. |
| Spreadtrum Communications, BUPT | Proposal 1: For evolution of duplex operation, isolated/non-isolated indoor factory, Macro & indoor factory and urban Macro are recommended as evaluation scenarios. - For the first step, non-isolated factory and urban Macro with the same subband configuration are recommended as the highest priority candidate scenarios. |
| CATT | Proposal 1: The deployment scenarios for SBFD evaluation include indoor hotspot, dense urban and urban macro. Proposal 5: For SBFD evaluation, prioritize the case that subband configuration across all the cells operating SBFD is the same. Proposal 13: Co-channel co-existing of SBFD with legacy TDD is studied in RAN1 and adjacent channel co-existence is led by RAN4. |
| vivo | Proposal 1: For SBFD and dynamic TDD evaluation, system-level simulation assumptions in TR 38.828 can be used as a starting point. Proposal 2: For non-coexistence scenarios in SBFD or dynamic TDD evaluation, UMa, 1-layer DU and InH can be high priority. Proposal 3: 100% grid-shifted deployment of gNB can be used for multiple operators’ co-existence scenarios. Proposal 4: For co-coexistence scenarios in SBFD or dynamic TDD evaluation, UMa, 1-layer DU and InH, can be considered. Proposal 7: For fair comparison between semi-static SBFD and semi-static TDD, the DL-to-UL resource ratio is assumed to be the same. Proposal 8: For SBFD with dynamic frequency format, the baseline is case 2-1. |
| xiaomi | Proposal 1: For subband non-overlapping full duplex, performance evaluation should be done at least in the indoor/hotspot, dense urban and macro urban scenarios. Proposal 2: For evaluation of subband non-overlapping full duplex, co-existence between legacy gNB and duplex gNB in the same network should be considered. Proposal 4: For evaluation of SBFD, the following two cases should be considered for each identified scenarios to evaluate the uplink performance: - Indoor hotspot - Baseline: DL dominated TDD w/o CLI - Duplex case: common UL subband configuration - Dense urban - Baseline: DL dominated TDD w/o CLI - Duplex case: common UL subband configuration - Urban Macro - Baseline: DL dominated TDD w/o CLI - Duplex case: common UL subband configuration - Co-channel co-existence of duplex gNB and legacy gNB - Baseline: DL dominated TDD w/o CLI - Duplex case: common UL subband configuration + legacy TDD Proposal 5: For evaluation of SBFD, the following two cases should be considered for each identified scenarios to evaluate the downlink performance: - Indoor hotspot - Baseline: DL dominated TDD w/o CLI - Duplex case: common UL subband configuration - Dense urban - Baseline: DL dominated TDD w/o CLI - Duplex case: common UL subband configuration - Urban Macro - Baseline: DL dominated TDD w/o CLI - Duplex case: common UL subband configuration - Co-channel co-existence of duplex gNB and legacy gNB - Baseline: DL dominated TDD w/o CLI - Duplex case: common UL subband configuration + legacy TDD |
| Samsung | Proposal 1: For evaluation purpose, the following three categories of applicable scenarios can be evaluated with their priorities. - The non-coexistence applicable scenario with highest priority - The coexistence applicable scenario of static TDD and a new duplex operation with medium priority - The coexistence applicable scenario of two new duplex operations with low priority Proposal 2: For non-coexistence applicable scenario, RAN1 takes the following applicable scenarios as a baseline - Indoor hotspot (FR1/FR2) with SBFD-only deployment and dynamic/flexible TDD-only deployment - 1-layer Dense Urban (FR1/FR2) with SBFD-only deployment and dynamic/flexible TDD deployment - Urban Macro (FR1) with SBFD-only deployment and dynamic/flexible TDD-only deployment - Note: down-selection/prioritization of non-coexistence applicable scenarios can be done in Rel-18 Proposal 3: For the coexistence applicable scenario of static TDD and a new duplex operation, RAN1 takes the following applicable scenarios as a baseline - 1-layer/2-layer Dense Urban (FR1/FR2) with coexistence deployment of static TDD and SBFD or dynamic/flexible TDD. - For 1-layer, 1 gNB or 7 gNBs with capability of SBFD or dynamic/flexible TDD are surrounded by gNBs with static TDD - For 2-layer, Macro layer is for static TDD and Micro/indoor hotspot layer is for SBFD or dynamic/flexible TDD or Macro layer is for SBFD and Micro/indoor hotspot layer is for static TDD - Note: down-selection/prioritization of the coexistence applicable scenario of static TDD and a new duplex operation can be done in Rel-18 Proposal 4: For the coexistence applicable scenario of two new duplex operations, RAN1 takes the following applicable scenarios as a baseline - 2-layer Dense Urban (FR1/FR2) with coexistence deployment of SBFD and dynamic/flexible TDD deployment - For 2-layer, Macro layer is for SBFD and Micro/indoor hotspot layer is for dynamic/flexible TDD - Note: down-selection/prioritization of the coexistence applicable scenario of two new duplex operations can be done in Rel-18 Proposal 5: For evaluation purpose, RAN1 to prioritize the aligned subband configuration for SBFD operation. Proposal 6: To evaluate impacts on adjacent channel, consider the followings as a baseline - For Macro layer (in Urban macro or Dense urban), - Two operators’ gNBs are located at the same (0% grid shift) - The second operator’s gNBs are located at edge of the first operator’s gNB (100% grid shift) - FFS: indoor hotspot deployment - Adjacent channel uses static TDD operation. |
| OPPO | Observation 1: Subband non-overlapping full duplex at the gNB side is beneficial for macro cell and small cell. For macro cell, semi-static (from gNB perspective) subband non-overlapping full duplex can be sufficient; for small cell, dynamic (from gNB perspective) subband non-overlapping full duplex is more suitable to handle dynamic uplink-downlink traffic ratio. Proposal 1: The deployment assumptions for Macro and InH in TR 38.828 can be used as reference. |
| SHARP Corporation | Proposal 1: RAN1 further studies duplex evolutions at least with the following scenarios - Macro-cell scenario - Small-cell scenario Proposal 2: RAN1 further studies scenarios where one TRP serves DL resource and another TRP serves UL resource for duplex evolution. |
| InterDigital, Inc. | Observation 1. Scenarios on subband non-overlapping (as for inter-subband CLI), subband partial overlapping and subband overlapping (as for intra-subband CLI) may achieve different gains based on at least traffic and/or cell sizes. Proposal 1. Consider evaluating achieved gain and performance in subband non-overlapping scenario based on inter-subband CLI, and also in subband partial overlapping and subband overlapping scenarios based on intra-subband CLI. Proposal 2. Consider evaluating intra- and inter-subband CLI in both FR1 and FR2 frequency ranges. Proposal 3. Urban macro and indoor scenarios can be considered for evaluations in this study, and the indoor scenarios should be prioritized which have the most significant UE-to-UE CLI effects. |
| Ericsson | Proposal 8: Scenarios where SBFD performance improvements may be realistically possible and can be simulated/evaluated by participating entities should be prioritized. Proposal 9: Regarding evaluations, both FR1 and FR2 should be considered in the study. Proposal 12: Regarding evaluation scenarios, two-operator urban macro scenario should be considered as the baseline scenario. We do not preclude other scenarios but it is important to study at least one real-world deployment. Proposal 14: For multi operator scenarios, it is important to consider realistic grid shifts. This should also include lower grid shifts. 0% grid shift (co-sited operators) should also be considered with proper channel model assumed between the gNBs of different operators that may be deployed at different heights. Consider studying 0%, 10% and 100% grid shifts. Proposal 23: RAN1 to agree the system level simulation parameters listed in Annex B. |
| KT Corp. | Observation 2: No performance degradation was observed for Macro-to-Indoor and Indoor-to-Macro scenario Proposal 1: For the deployment scenario, prioritize indoor small cell based gNB for sub-band full duplex gNB |
| Panasonic | Proposal 1: The following assumptions can be considered for link-level simulation. - Scenarios used for coverage enhancement evaluation can be a starting point. - Typical subband allocation can be that edges of a band are for DL and the center of a band is for UL. - For DL evaluation, CLI due to time difference between DL and UL symbols needs to be taken into account if DL and UL are assumed as in-band. |
| Apple | Proposal 1: Full-duplex operation shall not be supported for macro-to-macro scenarios, at least for FR1. Observation1: For indoor scenario, UL Tx power has a big impact (positive or negative) on the UE-to-UE CLI. |
| NTT DOCOMO, INC. | Proposal 1: Rural and Urban scenarios for FR1, and Indoor and Urban scenarios for FR2 is considered for evaluation. Proposal 2: Evaluation assumptions is derived by both UL heavy traffic scenario (e.g. eMBB), and coverage enhancement scenario (e.g. VoIP). |
| Nokia, Nokia Shanghai Bell | Proposal 3: For evaluating the performance and identifying enhancements for SBFD, focus on co-channel scenarios with same UL-DL PRB partitioning in all the cells and on adjacent channel scenario with legacy TDD. Proposal 4: For evaluating the performance and identifying enhancements for full duplex on non-overlapping PRBs, focus on the following scenarios for system-level simulations: - Urban Macro or Dense urban macro, with focus on co-channel and adjacent channel coexistence. - Indoor small cell deployments, e.g. open office scenario from TR 38.901 or Indoor scenario from TR 38.828, or outdoor small cell deployment, e.g. by only considering the small-cell layer of HetNet Scenario 2A from TR 36.872. Small cell deployment evaluations shall focus on co-channel interference aspects. |
| LG Electronics | Proposal 1: For deployment scenarios of study on NR duplex evolution, deployment scenarios (i.e., Dense urban, Urban macro, and Indoor hotspot) which were applied for flexible duplex evaluation in Rel-14 NR SI are considered. Proposal 2: Evaluation for NR duplex evolution is performed at FR1 and FR2-1. Also, it can be discussed whether or not to consider FR 2-2 for study of NR duplex evolution. |
| MediaTek Inc. | Observation 5: The feasibility and the performance of SBFD and DTDD schemes highly depend on the deployment scenarios considered for these schemes. Proposal 4: For the evaluations of SBFD and DTDD schemes, RAN1 should consider the deployment scenarios listed in Table 1 and Table 2. Proposal 6: For deployment scenarios with two operators, as starting point, 0% and 100% grid shift are assumed between the two operators’ gNBs: - For the 100% grid shift in Macro deployment, gNBs of the second operator are shifted, relative to the gNBs’ locations of the first operator, by ±ISD/2 on one axis and ±ISD/(2*sqrt(3)) on the other axis. Table 1: Deployment scenarios and topologies for SBFD. Scenario No.; Deployment Scenario; Topology. 1; Single-operator SBFD Same SBFD pattern in all cells; Single Layer: Urban, Dens Urban, Indoor. 2; Single-operator SBFD Different SBFD patterns among cells; Single Layer: Urban, Dens Urban HetNet: Macro-Indoor. 3; Two operators: 1st operator SBFD 2nd operator legacy TDD; Single Layer: Urban, Dens Urban, Indoor HetNet: Macro-Indoor Grid shift: 0% and 100%. |
| CEWiT | Proposal 1: Sub-band partitioning for DL and UL resources based on fixed DL to UL ratio is supported for evaluations. |
| Intel Corporation | Proposal 1: For the evaluations of both NOFD, and dynamic/flexible TDD, - For FR-1, both indoor office and urban macro scenario could be considered: - For FR-2, in addition to indoor office and urban macro scenario, the dense urban scenario could be considered. Proposal 5:For system-level evaluations on NOFD - Simulation assumptions, including deployment scenarios, antenna configurations, and related assumptions, as agreed during Rel-16 CLI/RIM, can be considered as starting points for system-level evaluations for NOFD operation. - Consider Table 3-7 in the Appendix II for NOFD evaluations in FR1 and FR2. - Non-full buffer with FTP traffic model 3 should be considered for traffic modelling. - DL/UL UPT should be used as the primary performance metric for SLS evaluations. Proposal 6: RAN1 should consider appropriate NOFD configurations and modelling options for self- and cross-link interference for the evaluations for NOFD. - Send an LS to RAN4 during RAN1 #109-e to receive inputs from RAN4 on self- and cross-link interference for NOFD operation. |
| Qualcomm Incorporated | Observation 1: Deployment scenarios that could benefit from subband full duplex, e.g. reduced latency and improve coverage, should be considered as baseline deployment scenarios. Other deployment scenarios may be considered as optional deployment scenarios. Observation 2: Subband full duplex deployment for Massive MIMO macro cell deployment with large EIRP could benefit from UL coverage gain and latency improvement and at same time is challenging deployment due to large self-interference at gNB. For FR1, UMa with 500m ISD shall be considered and for FR2, UMa with 200m ISD shall be considered. Observation 3: Subband full duplex deployment Indoor deployment may reduce requirements on gNB for self-interference mitigation due to small Tx Power. However, at least for FR1, it may be challenging deployment for handling cross link-interference. Observation 4: Same UL/DL subband configurations across all cells is more practical from deployment scenario. Proposal 1: For FR1, support UMa with 500m ISD as baseline deployment scenario for subband non-overlapping full duplex evaluation. Proposal 2: For FR1, support UMi and InH as optional deployment scenarios. Proposal 3: For FR2, support UMa with 200m ISD for FR2-1 and InH for FR2-1 as baseline deployment scenarios for subband non-overlapping full duplex evaluation. Proposal 4: For FR2, support UMi with 100m ISD for FR2-1, InH for FR2-2 and IAB as optional deployment scenarios. Proposal 5: For FR1 and FR2, support configuration of same UL/DL subbands across cells for subband full duplex study for baseline evaluation. Proposal 6: Support SLS as main tool for the evaluation of subband full duplex study. Proposal 7: For subband full duplex deployment scenario, legacy TDD deployment scenario should be used as base for the evaluation. Proposal 12: Support the following slot format configurations for the evaluation of subband full duplex. - For subband full duplex deployment scenario, use same full duplex slot format XXXXX (X=FD=D+U). - Note: all slots are flexible from the UE perspective. - For legacy TDD deployment scenario, use DDDSU as defined in Table A.1-2 of 38.838. Proposal 20: RAN 1 shall consider simulation parameters in Tables 1, and 2 for FR1 full duplex evaluation. Proposal 21: RAN 1 shall consider simulation parameters in Tables 1, 3, and 4 for FR2 full duplex evaluation. |

## 2.2 Summary

The deployment cases for SBFD proposed by companies are summarized as below:

- Deployment Case 1 (Non-coexistence case with single SBFD subband configuration): One single operator using one single carrier is considered. All the cells belonging to the operator use SBFD operation with the same SBFD subband configuration.

  - CMCC, Huawei, ZTE, CATT, vivo, xiaomi, Samsung, Nokia, MediaTek, Qualcomm, New H3C

- Deployment Case 2 (Non-coexistence case with multiple SBFD subband configurations): One single operator using one single carrier is considered. All the cells belonging to the operator use SBFD operation, but different cells may use different SBFD subband configurations.

  - vivo, MediaTek, New H3C, ZTE (2nd priority)

- Deployment Case 3 (Co-channel co-existence case): One single operator using one single carrier is considered. Among the cells belonging to the operator, some of them use legacy TDD operation while the others use SBFD operation with the same SBFD subband configuration.

  - CMCC, ZTE, CATT, xiaomi, Samsung (medium priority), Huawei (2nd priority)

- Deployment Case 4 (Adjacent-channel co-existence case): Two operators each using one carrier are considered and the two carriers are adjacent carriers. One operator uses legacy TDD operation while the other operator uses SBFD operation with the same SBFD subband configuration.

  - ZTE, vivo, Ericsson, Nokia, MediaTek, Samsung (medium priority), Huawei (2nd priority)

In addition, two companies [CATT, CMCC] propose that the performance evaluation for Deployment Case 4 can be done in RAN4.

Moderator suggests Initial proposal 1-1 and 1-2.

The applicable deployment scenarios for SBFD proposed by companies are summarized as below:

- Indoor hotspots or factory (low level of DL Tx power)

  - CMCC (FR1, FR2), Huawei (FR1), ZTE (FR1), Spreadtrum, CATT, vivo, xiaomi, Samsung (FR1, FR2), OPPO, [SHARP], InterDigital (FR1, FR2), KT, Apple, DOCOMO (FR2), Nokia, LG (FR1, FR2-1), MediaTek, Intel (FR1, FR2), Qualcomm (FR2, 2nd priority for FR1)

- Urban Macro (high level of DL Tx power)

  - CMCC (FR1), Huawei (FR1, FR2), ZTE (FR1), Spreadtrum, CATT, vivo, xiaomi, Samsung (FR1), OPPO, SHARP, InterDigital (FR1, FR2), [DOCOMO], Nokia, Ericsson (FR1, FR2), LG (FR1, FR2-1), MediaTek, Intel (FR1, FR2), Qualcomm (FR1, FR2)

- Dense Urban (medium level of Tx power)

  - Two layers: Macro + Micro

    - ZTE (FR1), [xiaomi], [DOCOMO], [Nokia], [LG (FR1, FR2-1)], [Intel (FR2)], Samsung (2nd priority)

  - Single layer: Macro layer or Micro layer

    - CMCC (Micro layer, FR1, FR2), CATT, vivo, [xiaomi], Samsung (FR1, FR2), [DOCOMO], [Nokia], [LG (FR1, FR2-1)], MediaTek, Intel (FR2)

- HetNet (Urban Macro + Indoor hotspot)

  - MediaTek, ZTE (2nd priority), Samsung (2nd priority), Spreadtrum (2nd priority)

- Urban Micro (medium level of Tx power)

  - Qualcomm (2nd priority for FR1 and FR2-1), CMCC (FR1, FR2)

- Rural

  - DOCOMO (FR1)

- IAB

  - Qualcomm (2nd priority for FR2-2)

- Isolated scenario

  - ZTE (2nd priority), Spreadtrum (2nd priority)

The number of combinations of the above deployment cases and scenarios will be large, especially considering both FR1 and FR2 need to be considered for some scenarios, some companies propose to prioritize some of the combinations to reduce the simulation workload.

Moderator suggests Initial proposal 1-3 and 1-4.

## 2.3 1st Round Proposals (closed)

### Initial proposal 1-1:

For discussion purpose, define the following deployment cases for SBFD:

- Deployment Case 1 (Non-coexistence case with single SBFD subband configuration): One single operator using one single carrier is considered. All the cells belonging to the operator use SBFD operation with the same SBFD subband configuration.

- Deployment Case 2 (Non-coexistence case with multiple SBFD subband configurations): One single operator using one single carrier is considered. All the cells belonging to the operator use SBFD operation, but different cells may use different SBFD subband configurations.

- Deployment Case 3 (Co-channel co-existence case): One single operator using one single carrier is considered. Among the cells belonging to the operator, some of them use legacy TDD operation while the others use SBFD operation with the same SBFD subband configuration.

- Deployment Case 4 (Adjacent-channel co-existence case): Two operators each using one carrier are considered and the two carriers are adjacent carriers. One operator uses legacy TDD operation while the other operator uses SBFD operation with the same SBFD subband configuration.

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| QC | Support the deployment scenario classification. Deployment Case 2 should be deprioritized or studied at later stage as it is more challenging for handling intra-SB cross-link interference. |
| NTT DOCOMO | We support the proposal. |
| New H3C | We are fine with this proposal. |
| CATT | Support |
| Spreadtrum | Support deployment cases for SBFD. Deployment case 1 should be prioritized and evaluation on it should be started ASAP. |
| ZTE | We are general fine with the structure. However, we are not sure whether “carrier” is the most appropriate wording to use here. The detailed solution on support of SBFD is under discussion in 9.3.2, where subband-based solution, BWP-based solution and carrier based solution are still open. Thus, at this stage, we hope the proposal 1-1 itself has no intention to preclude any potential solutions in AI9.3.2. It would be good to add a note to clarify this. |
| NEC | Agree with the listed scenarios |
| vivo | All deployment cases should be studied. Case 2 can be used to study the potential benefit of applying different SBFD configuration (or even dynamic SFBD configuration) among different cells. The CLI issues in case 2 is not that different from dynamic TDD case. One question for clarification, is semi-static SBFD subband configuration assumed for all these deployment cases? |
| Sharp | We support the proposal. |
| Sony | Case 3 is a special case of Case 2, i.e. Case 3 is basically Case 2 where the UL subband (assuming we only have one UL subband) is zero. We can perhaps just select one of them rather than both of them. |
| MediaTek | Support the proposal. |
| Xiaomi | Support. For case 2, we have similar feeling as Qualcomm. If majority of companies support to prioritize this case, we would like to clarify how to realize the different SBFD subband across cells, e.g.: - How many subband configurations are allowed across gNBs? - For each gNB, its subband configuration is selected randomly or some rules need to be discussed firstly? Whether overlapped SBFD subband is allowed or not? |
| Huawei, HiSilicon | Fine with definition. |
| Ericsson | We think case 1 and case 4 are the baseline scenarios to evaluate for SBFD. It is necessary to align the system level results amongst companies assuming same SBFD configuration before going to a more flexible SBFD. Therefore, we can support single operator Case 1 and two operator deployment Case 4 as the baseline cases. |
| Nokia/NSB | Support the proposal in general. Agree with Ericsson that case 1 and case 4 should be the baseline. Similar view with Sony and QC, case 2 can be deprioritize or replaced by case 3. |
| Intel | Support Proposal 1-1. |
| OPPO | Support the deployment cases definition. The case 1 could be treated as higher priority. |
| InterDigital | Support all the listed scenarios. Case 2 should also be considered, which provides resource allocation flexibility among cells, considering that existing dynamic/flexible TDD already allows this flexibility. Study by limiting resource allocation flexibility for SBFD in Rel-18 compared to (worse than) existing dynamic/flexible TDD doesn’t make sense. |
| Samsung | We are ok for the classification and ok to study Deployment Case 1, 3, and 4. For Deployment Case 2, we do not see benefits to prioritize the study on different UL subband configurations across cells, which introduce intra-subband CLI. The performance degradation by intra-subband CLI can be evaluated in Deployment Case 3, where gNB with legacy TDD operation may interfere UL reception at gNB with SBFD operation. Hence, we prefer to deprioritize Deployment Case 2. |
| KDDI | We support the proposal. |
| LG | Fine with the proposal. |
| Moderator | Regarding initial proposal 1-1, most companies are OK regarding the definition. We will discuss the prioritization among these scenarios in separate proposal. @vivo, there is no intention to restrict how the SBFD subband is configured, e.g., semi-static or dynamic, but I think dynamic SBFD subband configuration is not needed for Deployment Case 1. |

### Initial proposal 1-2:

For SBFD evaluation, take Deployment Case 1 and Case 4 as high priority, and take Deployment Case 2 as low priority.

- FFS: the priority of Deployment Case 3.

- FFS: Deployment Case 4 should be evaluated in RAN1-only, or RAN4-only, or both RAN1 and RAN4.

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| QC | Support in principles. We think a phased approach for the study should be considered where Case 1 should highest priority and studied first as baseline. Then, case 4 should be studied as second step. Agree to deprioritize case 2. |
| NTT DOCOMO | We support the proposal. Case 3 can be middle priority, since mixed deployment of legacy and SBFD cells can be considered. |
| New H3C | We are fine with this proposal. |
| CATT | Support. Between Case 1 and Case 4, we also think Case 1 should be the highest priority at least for RAN1 evaluation. |
| Spreadtrum | We think case 1 has the highest priority and case 2 has the lowest priority. FFS: the priority of case 3 and case 4. |
| ZTE | We are fine to take Case 2 with lower priority. However, from our perspective, Case 3 should also be prioritized since it is clearly mentioned in the SID that co-channel interference should also be taken into account. Regarding Case 4, we think this case can be simulated in RAN1. But if any RAN4 information/assistance is needed, we can always send liaison to RAN4. |
| NEC | We don’t agree with this proposal. From our understanding, all cases including Case 2 and Case 3 should be considered as equally high priority. |
| Vivo | We agree that case 1 and case 4 can be baseline with higher priority, while case 2 and case 3 can be optional and they can be studied based on companies’ input. Both RAN1 and RAN4 should be involved for the study of case 4. |
| Sharp | We are OK with the proposal. Case 1 could be the highest priority. |
| Sony | Support the proposal |
| MediaTek | In addition to Case#1 and Case#4, Case#3 should be considered as high priority because none of the other two cases can capture the impact of the co-channel co-existence. |
| Xiaomi | Support. For case 3, we think it can be prioritized as it is a typical case wherein legacy gNB and duplex gNB co-exist in the network. For the second FFS point, we think it should be handled by RAN4. |
| Huawei, HiSilicon | Support to take Case 1 as high priority and Case 2 as low priority. Case 3 should be also be included in the study as indicated in the SID. The scenario for Case 3 we have in mind is the Macro base station use legacy TDD configurations and indoor Pico use SBFD. For Case 4, we are wondering whether there will be any difference compare to the Rel-16 co-existence study even through the SID indicates that it should be studied in RAN1 in Rel-18. |
| Ericsson | We support the proposal prioritizing Case 1 and Case 4. However, we have the following comments. Regarding the first FFS point, Case 3 is not well-motivated in our opinion. In real-world deployments, it is unusual that only a single network operates in a region. On the other hand, two operators sharing the same channel is also not realistic. A single operator that wants to introduce SBFD will probably have to upgrade all the gNBs to SBFD mode in order to avoid gNB-gNB CLI issues. - Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1). - Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4). Regarding the second FFS point, according to the SID the RAN1 and RAN4 objectives are different. RAN1 should study the performance in adjacent and cochannel scenarios, and RAN4 should study the feasibility and impact on RF requirements in cochannel and adjacent channel scenarios. Hence, we think Case 4 should be evaluated in RAN1, while RAN4 considers feasibility and impact on RF requirements. One other aspect we need to consider is the impact of adjacent legacy operator to SBFD operator in the adjacent channel coexistence studies which is enabled by case 4. |
| Nokia/NSB | Support the proposal. Case 4 should be evaluated at least in RAN1 to develop the potential schemes. |
| Intel | We agree that Case 4 should be studied in RAN4 and hence, we would prefer to discuss evaluation priorities for the first three cases here. Of the first three, we think Case 1 and 3 are of higher priority compared to Case 2. |
| OPPO | Support. |
| InterDigital | No need to deprioritize Case 2 at the beginning, as Case 2 is only representing flexible resource allocation/utilization among cells. It doesn’t make sense to study a new SBFD scenario by limiting its resource allocation flexibility worse than the existing dynamic/flexible TDD which already allows flexible resource allocation among cells. |
| Samsung | As we commented in 1-1, we are ok to deprioritize Deployment Case 2. For Deployment Case 3, we need to further consider two sub-deployments, one is 1-layer co-channel co-existence case and another is 2-layer co-channel co-existence case. Due to high penetration loss, 2-layer case can give less CLI compared to 1-layer deployment. So, if needed, 2-layer co-channel co-existence case can be higher priority than 1-layer. For Deployment Case 4, we should avoid the last option, i.e., both RAN1 and RAN4. Since the adjacent-channel co-existence case is usually RAN4’s work, but evaluation assumption and interference modelling for SBFD should be kept same in RAN1 evaluation and RAN4 evaluation. Thus, before decision, we need to send LS to RAN4 on RAN4’s preference/understanding on adjacent-channel co-existence study. |
| KDDI | We support the proposal. |
| LG | We support the proposal in general. We prefer to handle Case 4 by RAN4. If necessary, inputs from RAN1 on the considerations for adjacent-channel co-existence evaluation can be discussed. |
| Moderator | Regarding initial proposal 1-2: - All companies agree Deployment Case 1 as high priority. - All companies agree to deprioritize Deployment Case 2 except NEC and Interdigital. Vivo suggests to take Case 2 as optional. - Regarding Deployment Case 3, it seems 5 companies [ZTE, NEC, MTK, Xiaomi, Intel] think it can be prioritized. vivo suggests to take it as optional. Ericsson thinks it is not well-motivated. Two companies [Samsung, Huawei] think Deployment Case 3 with 2 layers can be higher priority than 1-layer. - Regarding Deployment Case 4, most companies agree it as high priority. Intel thinks it can be done in RAN4, Huawei thinks Rel-16 coexistence study seems enough, and Samsung thinks it should be avoided to evaluate it in both RAN1 and RAN4. |

### Initial proposal 1-3:

For SBFD Deployment Case 1, at least consider the following scenarios for evaluation:

- For FR1,

  - Indoor hotspot (use Indoor hotspot defined in TR38.802 as starting point)

  - Urban macro (use Urban macro defined in TR38.802 as starting point)

  - Select one from {Dense Urban Macro layer, Dense Urban Micro layer, Urban Micro}

    - FFS which one

    - use Dense Urban defined in TR38.802 as starting point for Dense Urban Macro layer and Dense Urban Micro layer

    - use Umi in section 6.2 of TR38.901 or 3D-Umi in section 6 of TR36.873 as the starting point for Urban Micro

  - FFS: Rural

- For FR2-1,

  - Indoor hotspot (use Indoor hotspot defined in TR38.802 as starting point)

  - Select one from {Dense Urban Macro layer, Dense Urban Micro layer, Urban Micro}

    - FFS which one

    - use Dense Urban defined in TR38.802 as starting point for Dense Urban Macro layer and Dense Urban Micro layer

    - use Umi in section 6.2 of TR38.901 or 3D-Umi in section 6 of TR36.873 as the starting point for Urban Micro

  - FFS: IAB

- FFS: Whether FR2-2 is considered or not in Rel-18.

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| QC | For FR1, the first two scenarios (Urban Macro and Indoor hotspot) are sufficient for the evaluation. Urban Macro should be considered as the baseline deployment scenario as most of the SBFD benefits of UL coverage gain and latency reduction can be leveraged in Macro deployment. For FR2, support InH and Dense Urban Macro (200m ISD) as baseline evaluation. We also support Umi (100m ISD) as additional evaluation. Additionally, it is important to consider the evaluation for InH for FR2-2. |
| NTT DOCOMO | We support the proposal. |
| New H3C | We are fine with this proposal. |
| CATT | Support |
| Spreadtrum | We are open about frequency range. About the scenarios, we think indoor hotspot and Urban Macro at least should be involved in the evaluation. And about indoor hotspot, we also want to make it clear that does it refer to indoor office or indoor factory? |
| ZTE | For FR1, we are also supportive to include Rural for SBFD simulation considering the request from operators. Besides, it seems only homogenous network is selected for SBFD simulation. We suggest to add at least one Hetnet scenario, e.g., two layers dense urban, urban macro + indoor hotspot. We prefer to add the following bullet. Two-layer dense urban (urban macro and urban micro as defined in 38.802). |
| NEC | This proposal should also be applied to SBFD Deployment Case 2. Also following scenario should be considered “HetNet with Urban Macro and Indoor hotspot deployed in the same carrier” |
| vivo | For FR1, we think InH, Urban Macro and Dense Urban with single Macro layer can be considered. For FR2, we think InH and Dense Urban with single Macro layer can be considered Definitions in TR 38.802 for these scenarios are used as starting point. |
| Sharp | We support the proposal. |
| Sony | Support the proposal |
| MediaTek | We are generally fine with the proposal as starting point. |
| Xiaomi | We are generally fine with the proposal. Our preference is to choose dense urban as the third scenario on top of indoor/hotspot and urban Macro. |
| Huawei, HiSilicon | For FR1, we are fine with the proposal. We would like also support the proposal from ZTE to include a Hetnet deployment for SBFD simulation. For FR2, we don’t see a strong need to include indoor hotspot since the expected benefit of SBFD for indoor scenario is low latency, which does not seems to be a critical issue for FR2 considering the larger SCS. We support to study Dense Urban Macro for FR2. We also suggest to drop IAB in this study. |
| Ericsson | We think that scenarios where SBFD performance improvements may be realistically possible and can be simulated by participating entities should be prioritized. Furthermore, one of the main potential benefits of SBFD is coverage enhancements. There are basically two reasons a UE can be in poor coverage, either it is very far away from the base station, or it is indoor. To study the benefits of SBFD, sufficiently many UEs should be in poor coverage. In our opinion, the most relevant scenario for this is an urban macro deployment with a high proportion of indoor users (e.g., 70 – 80% which is seen in many practical deployments) (similar to Urban Macro + Dense urban without any Micro cells and only 1 cluster per cell). This can occur when there are groups of indoor Ues clustered in buildings as shown in the figure below. The figure illustrates a 10% grid shift. |
| Nokia/NSB | We agree in general. For FR1, it could be sufficient and it will saves us time to prioritize InH and UMa . |
| Intel | We agree with the comment from QC that for FR1, UMa and InH are sufficient. Also, we second QC’s comment that, for FR2-1, InH and Dense Urban Macro should suffice. |
| OPPO | For FR1, Urban macro could be used as baseline case. For FR2, use Indoor hotspot as baseline case. |
| InterDigital | Agree with NEC that the proposal should be equally applied to SBFD Deployment Case 2. |
| Samsung | Generally fine with the proposal. For FR1 and FR2, we support the use Dense Urban Macro layer since we expect SBFD operation can be deployed in macro layer with fixed UL subband configuration to provide higher UL coverage and latency gain. We do not support to study IAB since it is not specified in SID. For FR2-2, it should be deprioritized at least for SBFD operation. Due to high pathloss, gNB with FR2-2 may support very small number of UEs, so that full duplex capability is not necessary. |
| KDDI | We support in principle. For FR1, in addition to InH and Urban Macro, we think Dense Urban with single Macro layer should be included. For FR2, we support the use of InH and Dense Urban Macro layer scenarios. |
| LG | We are fine with the proposal. |
| Moderator | The proposal was updated based on companies’ comments |

### Initial proposal 1-4:

For SBFD Deployment Case 4 (if evaluation is needed in RAN1), at least consider the following scenarios for evaluation:

- Urban Macro

- FFS: the grid shift between two networks

- FFS: Indoor hotspot, Dense Urban

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| QC | Support. |
| New H3C | We prefer to study Urban Macro and Indoor hotspot with high priority. |
| CATT | Support |
| Spreadtrum | Considering the network deployment in China, we’d like to prioritize 0% grid shift in co-existence cases. |
| ZTE | From our perspective, dense urban should also be considered for Case 4. Different operators may have different levels of deployments, e.g., one operator only deploys urban macros while another operator may deploy both urban macros and urban micros. We are open to consider grid shift in this scenario. |
| vivo | For co-coexistence scenarios in SBFD evaluation, UMa and InH can be considered 100% grid-shifted deployment of gNB from different operators can be used. Besides, the scenarios for FR1 and FR2 for case 4 need to be separately evaluated, similar to case 1. |
| Sharp | We support the proposal. |
| Sony | Support. Make sense to look into urban macro as this is the case where interference among operators is likely. However, it is unclear if inter operator interference is a issue for indoor hotspot. |
| MediaTek | We prefer to start with Urban Macro and Indoor hotspot. |
| Xiaomi | Support. |
| Huawei, HiSilicon | As mentioned before, we are not sure whether there will be any practical differences compared to the Rel-16 study. |
| Ericsson | We support the proposal in principle with slight modifications. As mentioned above in proposal 1-2, SID clearly states that Case 4 needs to be evaluated. Furthermore, it is worthy to note that in commercial network deployments, there are multiple operators, at least two that may be co-located or in the vicinity of one another. For multi operator scenarios, it is important to consider realistic grid shifts. This should also include lower grid shifts. 0% grid shift (co-sited operators) should also be considered with proper channel model assumed between the gNBs of different operators that may be deployed at different heights. In addition, for grid shifts more than 0%, proper channel model between BSs assuming LOS needs to be considered. We propose to consider studying 0%, 10% and 100% grid shifts. ### Initial proposal 1-4(modified by Ericsson): For SBFD Deployment Case 4 (if evaluation is needed in RAN1), at least consider the following scenarios for evaluation: - Urban Macro - FFS: the grid shift between two networks: 0%, 10% and 100% - FFS: Indoor hotspot, Dense Urban |
| Nokia/NSB | Support the proposal in principle. We can prioritize 0% grid-shift, and we are fine with 100%. Regarding to 10% grid shift, we need further clarification We think that uniform 10% shift doesn’t match to the real network deployment. Can we replace this by applying different coupling loss between two networks? We don’t think indoor scenario need to be evaluated for the case 4. |
| Intel | As such the proposal may be fine, but we prefer to defer to RAN4 on adjacent channel related evaluations. |
| OPPO | Support |
| InterDigital | Support the FL proposal |
| Samsung | Before discussing the detailed assumptions, RAN1 needs to send LS to RAN4 on 1) which WG will perform adjacent-channel co-existence evaluation and 2) how to define evaluation assumptions (e.g., if RAN4 is decided to perform the evaluation, RAN4 reuses RAN1’s evaluation assumption defined for Urban Macro, or RAN4 makes own evaluation assumptions) |
| KDDI | We support the proposal. |
| LG | We prefer to consider Dense urban and Indoor hotspot scenario as well. |
| Moderator | Regarding initial proposal 1-4, some companies [New H3C, vivo, MTK] also want to consider InH for Deployment Case 4, but some companies [Nokia, Sony] prefer not. ZTE wants to consider Dense Urban. In addition, Companies have different understandings on whether RAN1 or RAN4 to perform the evaluation, I think this can be discussed separately. |

## 2.4 2nd Round Proposals (closed)

### Updated proposal 1-1:

For discussion purpose, define the following deployment cases for SBFD:

- Deployment Case 1 (Non-coexistence case with single SBFD subband configuration): One single operator using one single carrier is considered. All the cells belonging to the operator use SBFD operation with the same SBFD subband configuration.

- Deployment Case 2 (Non-coexistence case with multiple SBFD subband configurations): One single operator using one single carrier is considered. All the cells belonging to the operator use SBFD operation, but different cells may use different SBFD subband configurations.

- Deployment Case 3 (Co-channel co-existence case): One single operator using one single carrier is considered. Among the cells belonging to the operator, some of them use legacy TDD operation while the others use SBFD operation with the same SBFD subband configuration.

  - Deployment Case 3-1: Only 1-layer is considered,

  - Deployment Case 3-2: 2-layer is considered, Macro+Indoor, or Macro

- Deployment Case 4 (Adjacent-channel co-existence case): Two operators each using one carrier are considered and the two carriers are adjacent carriers. One operator uses legacy TDD operation while the other operator uses SBFD operation with the same SBFD subband configuration.

Note: This definition has no intention to preclude any potential solutions for SBFD in AI9.3.2

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| Apple | Deprioritize case 2 and 3 to avoid intra-subband CLI |
| New H3C | We are fine with updated proposal 1-1 |
| Intel | We are OK with proposal, but we think that case 4 should be studied and considered in RAN4. Therefore, RAN1 should focus on the rest of deployment within higher emphasis on Case 1 and 3-1. |
| Samsung | We are fine with the updated proposal 1-1. For RAN1 evaluation, we need to deprioritize deployment case 2. For deployment case 4, we agree with intel that it can be studies and considered in RAN4. |
| vivo | We support the updated proposal 1-1 |
| CATT | Support |
| ZTE | For the following bullet, we propose to delete “Macro+Indoor, or Macro” for now since anyway we will discuss it in a separate proposal. From our perspective, two layer dense urban is also a interesting scenario. Deployment Case 3-2: 2-layer is considered, Macro+Indoor, or Macro |
| MediaTek | Support the proposal. |
| Sony | Similar views with Apple. We don’t think we need both Case 2 & Case 3. We can pick just one of them. |
| Nokia, NSB | We are ok with the proposal. We understand that the intention of the proposal is to have an aligned definition of the different deployment scenarios, but it does not necessarily mean that we will evaluate all of them. |
| NEC | We support the proposal. We have some concerns with deprioritization of Case 2 which we are indicating in Proposal 1-2. |
| Huawei, HiSilicon | Fine with the proposal except the typo on Deployment Case 3-2. Deployment Case 3-2: 2-layer is considered, Macro+Indoor, or Macro |
| Xiaomi | We are fine with the updated proposal 1-1. Besides, we share the same views as Sumsung and Intel. |
| Spreadtrum | Support the definition of scenarios. |
| Ericsson | We support case1 and case 4. Regarding Case2, handling intra-subband CLIs due to different SBFD configurations would be complicated and hence should be deprioritized. Regarding Case 3, we would like to note that it is unusual that only a single network operates in a region. Secondly, if coverage improvements are one of the potential benefits of SBFD, Case 3 with two layers scenario is not coverage limited due to the existence of the micro/pico BS. In general, we think case 3 could be deprioritized, or considered as an optional deployment case. |
| NTT DOCOMO | We support the proposal. |
| QC | Fine with deployments classifications for the discussion purpose only. We think Deployment Case 2 and case 3 should be treated with lower priority as compared to case 1 (highest priority) and case 4. |
| InterDigital | Agree with Intel that Case 4 should be studied and considered in RAN4. Then, RAN1 can consider Case 1, 2, 3 only. No need to deprioritize anything. Companies can consider their interested cases for study. If at least one case needs to be commonly assumed, Case 1 can be the default one and other cases can be freely chosen for focused study by each company. |
| Moderator | The following has been agreed in GTW: Updated proposal 1-1: For discussion purpose for evaluation, define the following deployment cases for SBFD: - Deployment Case 1 (Non-coexistence case with single SBFD subband configuration): One single operator using one single carrier is considered. All the cells belonging to the operator use SBFD operation with the same SBFD subband configuration. - Deployment Case 2 (Non-coexistence case with multiple SBFD subband configurations): One single operator using one single carrier is considered. All the cells belonging to the operator use SBFD operation, but different cells may use different SBFD subband configurations. - Deployment Case 3 (Co-channel co-existence case): One single operator using one single carrier is considered. Among the cells belonging to the operator, some of them use legacy TDD operation (static TDD operation) while the others use SBFD operation with the same SBFD subband configuration. - Deployment Case 3-1: Only 1-layer is considered - Deployment Case 3-2: 2-layer is considered - Deployment Case 4 (Adjacent-channel co-existence case): Two operators each using one carrier are considered and the two carriers are adjacent carriers. One operator uses legacy TDD operation (static TDD operation) while the other operator uses SBFD operation with the same SBFD subband configuration. Note: This definition has no intention to preclude any potential solutions for SBFD in AI9.3.2 Note: SBFD subband configuration is from gNB perspective. |

### Updated proposal 1-2:

For SBFD evaluation,

- Deployment Case 1 and Case 4 are high priority.

  - FFS: Deployment Case 4 should be evaluated in RAN1-only, or RAN4-only, or both RAN1 and RAN4.

- Deployment Case 3 with 2-layers (Macro + small cell) is high priority, in which one layer uses legacy TDD operation and the other layer uses SBFD operation.

  - FFS: the small cell is indoor or outdoor, and other details

- Deployment Case 3 with 1-layer (Macro or small cell) is low priority, in which some gNBs use legacy TDD operation and the other use SBFD operation.

- Deployment Case 2 is low priority

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| Apple | Do not support 2nd and 3rd bullet (only case 1 and 4 are high priority) |
| New H3C | We are fine with this proposal |
| Intel | Do not support 1st bullet as we think that from RAN1 perspective Case 4 should not be prioritized, but should be studied in RAN4 as this lies within their expertise. |
| Samsung | We are generally ok except deployment case 4, which is further coordicated with RAN4. One clarification question is on the meaning of “low priority” For example, we can consider two interpretations on low priority. Which interpretation is intended? - Interpretation 1) The deployment case is captured in TR and used to draw conclusion/recommendation but it is optional (it is up to each company to provide the result of the deployment case) - Interpretation 2) RAN1 focus on the deployment case with high priority in this SI. If time is allowed, RAN1 can discuss evaluation assumptions for the deployment case with low priority. It means that according to RAN1 progress, the deployment case with low priority may or may not captured in TR. |
| vivo | We suggest Deployment Case 1 and Case 4 are baseline, while other cases can be optional. Regarding Deployment Case 2, we wonder if there is much difference for handling cross-link interference between SBFD and dynamic/flexible TDD. According to the SID, dynamic/flexible TDD needs to be studied considering how to handle the CLI issue. We think there is no reason to preclude the study for SBFD for case 2 due to the CLI issue while to study dynamic/flexible TDD with the similar CLI issue. |
| CATT | We do not think Case 3 should be high priority. |
| ZTE | We support to mark Case 3 as high priority. The WID clearly asks RAN1 to study the impact on legacy operation assuming their co-existence in co-channel and adjacent channels. Case 3 is handling the co-channle case. - Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1). |
| MediaTek | We are fine with the proposal. We don’t agree with the argument that Case#4 is only within RAN4 expertise. All the UE’s/gNB’s RF/interference models (IBE, ACLR, ACS, in-channel selectivity, etc.) for SBFD and DTDD (dynamic TDD) are within RAN4 expertise, and we expect RAN4 to provide inputs on these models so RAN1 can conduct the evaluations (SLS, LLS, etc.). RAN1 will conduct the performance and feasibility analysis for the considered scenarios. |
| Sony | Support. |
| Nokia, NSB | We agree with Apple. We think it is sufficient to only have case 1 and 4 as high priority. While others can be low or medium priority. In our view, the coexistence problem in deployment case #3 with two layers is not very different than the one that would be experienced with dynamic TDD when macro cell layers uses e.g. DDDSU and small cell layer uses DDSUU or other UL-heavy TDD pattern. |
| NEC | We are ok with the proposal as long as a note can be added that deprioritization of Case 2 does not preclude potential solutions related to flexible or dynamic SBFD resource allocation among cells. |
| Huawei, HiSilicon | We support the proposal. There are clear use cases for Case 3 (e.g. smart factory) hence we also support to study this scenario with high priority. Case 4 is a co-existence study which is typically done in RAN4 but we can accept to study it in RAN1 with the understanding that the evaluation assumptions and performance metric may be different among different working groups. |
| Xiaomi | Share the same views with Intel. Regarding to the clarification raised by Samsung, our understanding is interpretation 2). |
| Spreadtrum | We are generally fine with this propose. Agree with Ericsson about case3, we don’t think it will happen in single operator. If majority think case 3 should be included. We prefer to 2-layers scenario with two operator which make this case more reasonable. |
| Ericsson | As commented earlier, we do not see the need for second bullet point. Furthermore, Case4 needs to be studied and evaluated both in RAN4 and RAN1. |
| NTT DOCOMO | We support the proposal. |
| QC | Support the first and last bullets only. Deployment case 3 with 2-layers should be treated with lower priority as well. As commented earlier, we think a phased approach for the study should be considered where Case 1 should highest priority and studied first as baseline. Then, case 4 should be studied as second step. |
| InterDigital | No need to make a detailed prioritization among the cases. Companies can consider their interested cases for study. If at least one case needs to be commonly assumed, Case 1 can be the default one and other cases can be freely chosen for focused study by each company. |
| Moderator | Updated based on comments. Let’s further check companies’ views. |

### Updated proposal 1-3:

For SBFD Deployment Case 1, at least consider the following scenarios for evaluation:

- For FR1,

  - Indoor office (use Indoor office defined in TR38.802/TR38.901 as starting point)

  - Urban macro (use Urban macro defined in TR38.802 as starting point)

  - Select one from {Dense Urban Macro layer, Dense Urban with 2-layer}

    - use Dense Urban defined in TR38.802 as starting point

  - FFS: Rural

- For FR2-1,

  - Indoor office (use Indoor office defined in TR38.802/TR38.901 as starting point)

  - Dense Urban Macro layer (use Dense Urban defined in TR38.802 as starting point)

  - FFS: IAB

- FFS: Whether FR2-2 is considered or not in Rel-18.

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| New H3C | We are fine with this proposal |
| Intel | We are generally OK with the proposal, but for FR-1 we do not think any additional deployments, beyondInH and UMA, are necessary for Deployment Case 1. |
| Samsung | We are fine with the proposal. For the three FFS (Rural, IAB, FR2-2), we do not support to study these in R18 SI. From SID, it is explicitly stated that duplex enhancement at the gNB side. We are not sure whether IAB is included in the gNB side. Thus, RAN-level guide/clarification should be required to study IAB with SBFD operation and so RAN1 discussion should not include the IAB scenario at this stage. |
| vivo | We are fine with InH and UMa for FR1, and InH for FR2. Regarding Dense urban, we suggest to adopt Dense Urban Macro layer with single layer as baseline for both FR1 and FR2, while Dense Urban with 2-layer can be optional for FR1. We would like to propose that the scenarios in proposal 1-3 should also be applied for Deployment Case 2, though case 2 is optional |
| CATT | Support |
| ZTE | Comment#1: 38.802 was generated when the 5G channel models for FR1 is not ready. In 38.802, the channel modes for FR1 use LTE 3D channel models. We don’t think companies still intend to use LTE 3D channel models. Thus, we propose to change all the 38.802 to 38.901 for FR1. Comment#2: Regarding “Select one from {Dense Urban Macro layer, Dense Urban with 2-layer}”, our preference is to select at least one Hetnet scenario for SBFD study. Thus, we support to select Dense Urban with 2-layer. |
| MediaTek | Fine with the proposal |
| Sony | Support |
| Nokia, NSB | Same as Intel. It is sufficient to have InH and Uma as the mandatory scenarios for FR1. |
| NEC | We are okay with this proposal |
| Huawei, HiSilicon | For FR1, we are fine with the proposal. For FR2, we still don’t see the need to include indoor office since the expected benefit of SBFD for indoor scenario is low latency, which does not seems to be critical for FR2 considering the larger SCS. We think IAB can be dropped from the study. |
| Xiaomi | We are fine with the updated proposal. |
| Spreadtrum | We think indoor factory should also be taken into account in FR1 and FR2. And agree with Interl about InH and urban Macro is sufficient for FR1. |
| Ericsson | We support the proposal. |
| NTT DOCOMO | We support the proposal. |
| QC | For FR1, support only first two bullets (InH and Uma). The third sub-bullet could be considered as optional deployment scenario. For FR2, we also support Umi (100m ISD) as additional evaluation. Suggest the following edits: For SBFD Deployment Case 1, at least consider the following scenarios for evaluation: - For FR1, - Indoor office (use Indoor office defined in TR38.802/TR38.901 as starting point) - Urban macro (use Urban macro defined in TR38.802 as starting point) - Optionally, Select one from {Dense Urban Macro layer, Dense Urban with 2-layer} - use Dense Urban defined in TR38.802 as starting point - FFS: Rural - For FR2-1, - Indoor office (use Indoor office defined in TR38.802/TR38.901 as starting point) - Dense Urban Macro layer (use Dense Urban defined in TR38.802 as starting point) - Optionally, Dense Urban Micro layer - FFS: IAB - FFS: Whether FR2-2 for Indoor Hotspot is considered or not in Rel-18. |
| Moderator | The following has been agreed in GTW: Updated proposal 1-3: For SBFD Deployment Case 1, at least consider the following scenarios for evaluation: - For FR1, - Indoor office (use Indoor office defined in TR38.802/TR38.901 as starting point) - Urban macro (use Urban macro defined in TR38.802/TR38.901 as starting point) - FFS: UE outdoor/indoor proportion, clustering, etc - Optional: Dense Urban with 1-layer or 2-layer (use Dense Urban defined in TR38.802/TR38.901 as starting point) - FFS: Rural - For FR2-1, - Indoor office (use Indoor office defined in TR38.802/TR38.901 as starting point) - Dense Urban Macro layer (use Dense Urban defined in TR38.802 as starting point) - FFS: UE outdoor/indoor proportion, clustering, etc - Optional: Dense Urban micro (use Dense Urban micro defined in TR38.802/TR38.901 as starting point) - FFS: Whether FR2-2 is considered or not in Rel-18. Note: For optional scenarios, they can be captured in TR and it is up to each company to provide the results. The results can be used to draw conclusion/recommendation depending on the number of companies providing the results. |

### Updated proposal 1-4:

For SBFD Deployment Case 4 (if evaluation is needed in RAN1), at least consider the following scenarios for evaluation:

- Urban Macro

- FFS: the grid shift between two networks, e.g., 0%, 100%

- FFS: Indoor hotspot, Dense Urban

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| New H3C | We are fine with this proposal |
| Intel | As indicated before, we prefer to first conclude on whether RAN1 needs to evaluate Case 4. Otherwise, assumptions should be left up to RAN4 to decide. |
| Samsung | If RAN1 do SLS on deployment case 4, we are ok with the proposal. A minor comment is the first FFS on the grid shift should be the sub-bullet of Urban Macro. |
| vivo | We are fine with this proposal |
| CATT | Support |
| ZTE | OK |
| MediaTek | Fine with the proposal |
| Sony | Support. We still don’t think indoor make sense for Case 4 but since it is FFS we are fine to support this. |
| Nokia, NSB | We support the proposal. |
| Huawei, HiSilicon | Fine. |
| Xiaomi | Ok with the proposal. |
| Spreadtrum | About adjacent-channel co-existence case agree with Huawei in Initial proposal 1-4. If reuse the interference modelling (ACIR) in R16, we don’t think there will be different compared with study in R16. We suggest firstly to send LS to RAN4 to ask whether the interference modelling per subcarrier/RB/subband is reasonable in SBFD. |
| Ericsson | We support the proposal. We do not support the text “ if evaluation is needed in RAN1” in brackets. As commented in the first round, we think that the SID explicitly states the study of adjacent channel coexistence studies in RAN1. Furthermore, in our system level simulations we see significant degradation in SBFD performance at medium/high loads when STDD coexists with SBFD due to gNB-gNB CLI. Therefore, we think it needs to be evaluated in RAN1. |
| NTT DOCOMO | We support the proposal. |
| QC | We are confused by the condition (if evaluation is needed in RAN1) as Case 4 is treated as high priority in Proposal 1-2. Besides, the scenarios for FR1 and FR2 for case 4 need to be separately evaluated, like case 1: - For FR1, urban macro. For FR2, dense urban and indoor hotspot can be considered. |
| InterDigital | Agree with Intel |
| Moderator | Let’s use the version after GTW discussion for further check. |

## 2.5 3rd Round Proposals (closed)

### Updated proposal 1-2a:

For SBFD evaluation from RAN1 perspective,

- High priority

  - Deployment Case 1

- Optional

  - Deployment Case 4 (SLS methodology used in RAN1 should be coordinated with RAN4)

  - Deployment Case 3-2 with 2-layer, in which one layer uses legacy TDD operation and the other layer uses SBFD operation

  - Deployment Case 3-1 with 1-layer

  - Deployment Case 2

Note: For deployment cases that are optional, they can be captured in TR and it is up to each company to provide the results. The results can be used to draw conclusion/recommendation depending on the number of companies providing the results.

| Company | Comment |
| --- | --- |
| New H3C | We are fine with this proposal. |
| ZTE | As we commented previously, deployment Case 3 and Case 4 is clearly listed in the SID. To complete this SI, we need to makr Case 3 and Case 4 as high priority. Also, we see some use cases for Case 3-1. We can not expect to upgrade all the gNBs in one area to full duplex at once. |
| Huawei, HiSilicon | We prefer to study “Deployment Case 3-2 with 2-layer, in which one layer uses legacy TDD operation and the other layer uses SBFD operation” with high priority. |
| Sony | Support. |
| MediaTek | The SID clear on that the impact on legacy operation assuming co-existence in co-channel and adjacent channels need to be studied. “Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1).” Case-1 doesn’t have co-existence with legacy operation, consequently, prioritizing only Case-1 leave a gap in the study item regarding co-existence with legacy operation. Case-3 covers the part of co-existence with legacy operation in co-channel. Case-4 cover the part of co-existence with legacy operation in adjacent channels. So, these two cases maps to the SID objective, and without studying these two cases the SI will be incomplete. |
| InterDigital | Support the FL proposal. No need to further consider detailed prioritization among the optional cases, as Case 1 can be the default one and other cases can be freely chosen for focused study by each company. Companies can consider their interested cases for study. |
| Apple | Support FL’s proposal. We are not supportive to add Case4. We see that SID tasks RAN1 to evaluate/design coexistence for adjacent channels, but definitely assumptions for such analysis like ACLR, ACS, etc shall be validated by RAN4 (possibly this part can be verisited in RANP), where same as R16 it should be RAN4 responsible for adjacent channel analysis. |
| Intel | We are fine with the current prioritization. |
| Nokia | We are ok with this proposal. On the case 4, because RAN1 focus can be different from RAN4, we prefer some flexibility should be supported as. Deployment Case 4 (Strive SLS methodology used in RAN1 to be coordinated with RAN4) |
| QC | Priotrization is definitely needed to manage the workload of the study item and achieves the objectives of the SID. In our views, a phased approach for the study should be considered where Case 1 should highest priority and studied first as baseline. Then, case 4 should be studied to identify any impact of the coexistence with legacy operators. Deployment case 3 with 2-layers should be treated with lower priority as compared to Deployment case 1 and case 4 since gNB can handle co-channel coexistence with gNB implementation-based schemes. Deployment case 2 is lowest priority as it is challenging for handling the CLI. |
| Fujitsu | Support |
| NTT DOCOMO | We support the proposal, and we prefer that case 4 has priority (e.g. 2nd priority). |
| LG | Fine with the proposal |
| vivo | We think case 2/3/4 are all needed to be evaluated and at least case 4 is baseline. But we are also OK with these cases as optional. |
| CATT | We support FL proposal. Based on previous discussion, we think it is sufficient to include Case 3 and Case 4 in optional cases and let companies to decide whether to provide the results in RAN1. |
| Ericsson | We still cannot support this proposal in its current form. In reality there are no single operator cases (Case 1) and Adjacent channel coexistence study is described in the SID (Case 4). Additionally, although SID requests the study of impact to legacy assuming coexistence in co-channel, we are not sure if it is a realistic case where two operators coexist in co-channel. Therefore, Case 4 needs to be the baseline in our opinion, but we are ok to support both Case1 and Case 4 as High Priority, and Case3 as optional. Therefore, we prefer the modified proposal below. ### Updated proposal 1-2a: (Modified by Ericsson) For SBFD evaluation from RAN1 perspective, - High priority - Deployment Case 1 - Deployment Case 4 (SLS methodology used in RAN1 should be coordinated with RAN4) - Optional - Deployment Case 4 (SLS methodology used in RAN1 should be coordinated with RAN4) - Deployment Case 3-2 with 2-layer, in which one layer uses legacy TDD operation and the other layer uses SBFD operation - Deployment Case 3-1 with 1-layer - Deployment Case 2 |
| Spreadtrum | We support this proposal. |
| Xiaomi | We are generally fine with the proposal. Maybe we should not debate which case should be priotized or not, as it is mentioned that all the evaluation results can be submitted by companies and can be captured in TR. For case 3-1, we don’t think it should be removed. It would a typical case partial gNB are updated to duplex gNB while the others are legacy. |
| Samsung | We are fine with the proposal. Companies can evaluate interested deployment cases. |
| Moderator | Let’s have a another try to see whether companies are OK with a phased approach as QC suggested. |

### Updated proposal 1-4a:

For SBFD Deployment Case 4, at least consider the following scenarios for evaluation:

- FR1: Urban Macro

  - FFS: UE outdoor/indoor proportion, clustering, etc

- FR2-1: Dense Urban Macro

- FFS: the grid shift between two networks, e.g., 0%, 100%

- FFS: Indoor hotspot

| Company | Comment |
| --- | --- |
| New H3C | We are fine with this proposal. |
| ZTE | OK |
| Huawei, HiSilicon | Fine with the proposal. |
| Sony | Support |
| MediaTek | Support |
| InterDigital | Fine with the FL proposal |
| Intel | Considering that based on prior proposal (Updated Proposal 1-2a) RAN1 should coordinate with RAN4 on the SLS methodology, the deployments for case 4 should be also be part of that coordination and require RAN4 inputs to decide. |
| Nokia, NSB | We support this proposal. |
| QC | Support |
| Fujitsu | Support |
| DOCOMO | We support. |
| LG | We’d like to include Dense urban scenario with FFS for both of FR1 and FR2-1. Adjacent chanel interference impact from micro layer to micro layer may be different from macro cell scenario. In our view, it would be meaningful to check the possibility of co-existence in the micro layer, which is expected to have relatively small interference. |
| vivo | We are fine with the proposal. |
| CATT | We support FL proposal. |
| Ericsson | We support the proposal. |
| Spreadtrum | Support |
| Xiaomi | Fine with the proposal. |
| Samsung | Fine with the proposal. Do we need to add “FFS: UE outdoor/indoor proportion, clustering, etc” under FR2-1 dense urban macro? |
| Moderator | Updated. @LG, is your intention to include Dense Urban Micro layer with FFS for both FR1 and FR2-1? |

## 2.6 4th Round Proposals (closed)

### Updated proposal 1-2b:

For SBFD evaluation from RAN1 perspective,

- High priority (using phased approach)

  - Phase 1:

    - Deployment Case 1

  - Phase 2:

    - Deployment Case 4 (strive for SLS methodology used in RAN1 to be coordinated with RAN4)

    - Deployment Case 3-2 with 2-layer, in which one layer uses legacy TDD operation and the other layer uses SBFD operation

- Optional:

  - Deployment Case 3-1 with 1-layer

  - Deployment Case 2

Note: For deployment cases that are optional, they can be captured in TR and it is up to each company to provide the results. The results can be used to draw conclusion/recommendation depending on the number of companies providing the results.

| Company | Comment |
| --- | --- |
| Intel | We prefer the earlier version; the current “Phased approach” is not very clear to us and may need some clarifications. What does phase 1 and phase 2 imply? Does it mean that Case 4 and Case 3-2 are evaluated after Case 1, or after RAN1 will reach out to RAN4? If the reply is the latter, we could accept the proposal, but this should be described clearly in the proposal, including the “Optional” categorization from previous round for the “Phase 2” items. However, if the understanding is that phase 1 and phase 2 is simply a temporal connotation, then we prefer to revert the proposal to prior text: in fact under this interpretation, more discussions may be needed, for instance to determine when Phase 1 is complete, and when phase 2 should start. |
| CATT | We also think the “phased approach” is not very clear and prefer the earlier version. To address the comments from other companies, a possible way forward is to add a note that deployment Case 4 and Case 3-2 can be revisited after coordination with RAN4. |
| Ericsson | We also do not support this version. It is not clear to us at what point it is declared Phase 1 ends and Phase 2 starts. Please see the following revisions. ### Updated proposal 1-2b: (Modified by Ericsson) For SBFD evaluation from RAN1 perspective, - High priority (using phased approach) - Phase 1: - Deployment Case 1 - Phase 2: - Deployment Case 4 (strive for SLS methodology used in RAN1 to be coordinated with RAN4) - Deployment Case 3-2 with 2-layer, in which one layer uses legacy TDD operation and the other layer uses SBFD operation - Optional: - Deployment Case 3-1 with 1-layer - Deployment Case 2 Note: For deployment cases that are optional, they can be captured in TR and it is up to each company to provide the results. The results can be used to draw conclusion/recommendation depending on the number of companies providing the results. |
| ZTE | Thanks FL for the great effort, we support this phased approach. From our perspective, phased approach means we will first discuss the simulation assumptions for scenarios in phase 1 and after that, we will discuss simulation assumpitons for scenarios in phase 2. The detailed arrangements can be decided by Chiar, Rapporteur and FL. |
| QC | In principle okay. However, for Deployment case 3 (co-channel coexistence), we believe Case 3-1 with 1-layer should be higher priority than case 3-2 with 2 layers. |
| Xiaomi | We share the view that phase 1 and phase 2 is not clear. We agree with CATT’s suggestion. |
| Samsung | Agree with Intel that the phased approach is suddenly suggested by FL and it is unclear what’s meaning of phase 1 and phase 2. Rather than the phased approach, we prefer the updated proposal 1-2a and further discuss whether case 4 and case 3-2 with 2-layer are high priority or not. Even though RAN1 accepts the phased approach, we need to make evaluation methodololy/assumptions and calibration for all deployment cases for two priorities at the same time. It means that there is not “phased” discussion in RAN1. |
| KDDI | From an operator perspective, in reality, thare is neither single operator case nor case where all legacy gNBs are updated to a practical gNB at once. Therefore, we think that case 3-1/3-2/4 should have a priority. Second, for the updated proposal, we share the same view with other companies that the meaning “phased approach” is not clear. |
| Huawei, HiSilicon | We prefer the previous version with the understanding that the conclusions/observations on feasiblity and performance will be drawn on all high-priority deployment cases. |
| vivo | We share the similar view as Intel and other companies that the phase is not clear. We prefer the updated proposal 1-2a in the 3rd round. |
| Spreadtrum | We also prefer the earlier version. It’s not very clear about “phase1””phase2”. |
| Nokia, NSB | We share view with Ericsson. Phased approach is unclear. |
| MediaTek | Similar view as Ericsson |
| Sony | Perhaps if the intention of the phases are clarified, it is fine. If the intention is to look into Deployment Case 1 first followed by the others, we are fine with it. |

### Updated proposal 1-4b:

For SBFD Deployment Case 4, at least consider the following scenarios for evaluation from RAN1 perspective:

- FR1: Urban Macro

- FR2-1: Dense Urban Macro layer

- FFS: UE outdoor/indoor proportion, clustering, etc

- FFS: the grid shift between two networks, e.g., 0%, 100%

- FFS: Indoor hotspot, Dense Urban Micro layer

| Company | Comment |
| --- | --- |
| Intel | Considering the text and comments related to prior proposal, and as previously mentioned, RAN1 should coordinate with RAN4 on the SLS methodology, and the deployments for case 4 should be also part of that coordination and require RAN4 inputs to decide the ownership and deployments to consider. |
| CATT | The comments from Intel are reasonable. |
| Ericsson | Support the proposal. |
| ZTE | OK |
| QC | Support. One comment, we think that FFS on UE distributions between outdoor/indoor and clustering should be applicable for FR1 only. |
| Xiaomi | OK |
| Samsung | Fine with the proposal. With this, further coordination with RAN4 is needed. |
| KDDI | Support the proposal. |
| Huawei, HiSilicon | Not sure why layer is added for FR2-1 and in FFS. |
| Vivo | We are fine with the proposal. |
| Spreadtrum | Support. |
| Nokia, NSB | Support the proposal. |
| MediaTek | Support |
| Sony | Support |

## 2.7 5th Round Proposals (open)

### Updated proposal 1-2c:

For SBFD evaluation from RAN1 perspective,

- High priority

  - Deployment Case 1

  - FFS: Deployment Case 4 (strive for SLS methodology used in RAN1 to be coordinated with RAN4)

  - FFS: Deployment Case 3-2 with 2-layer, in which one layer uses legacy TDD operation and the other layer uses SBFD operation

- Optional

  - Deployment Case 3-1 with 1-layer

  - Deployment Case 2

Note: For deployment cases that are optional, they can be captured in TR and it is up to each company to provide the results. The results can be used to draw conclusion/recommendation depending on the number of companies providing the results.

| Company | Comment |
| --- | --- |
| Moderator | Suggests that we just take this as agreement to give a list of scenarios that we need to make assumptions for, so that we can further discuss the detailed assumptions for all these scenarios. |
| ZTE | We propose the following way forward. Proposal. For SBFD evaluation from RAN1 perspective, - High priority - Deployment Case 1 - FFS: Deployment Case 4 (strive for SLS methodology used in RAN1 to be coordinated with RAN4) - FFS: Deployment Case 3-2 with 2-layer, in which one layer uses legacy TDD operation and the other layer uses SBFD operation - Note: Case 4 and Case 3-2 may need coordination between RAN1 and RAN4. - Optional - Deployment Case 3-1 with 1-layer - Deployment Case 2 Note: For deployment cases that are optional, they can be captured in TR and it is up to each company to provide the results. The results can be used to draw conclusion/recommendation depending on the number of companies providing the results. |
| New H3C | We are fine with this proposal with ZTE’s modification. |
| MediaTek | We don’t support the proposal. As the Chairman suggested in the last GTW session, we could skip the prioritization of the cases. At the end, each company is free to evaluate the case/scenario that they think it is essential for the study. We already have a list of the cases to be evaluated for SBFD. It is better to focus on finalizing the details for these cases. |
| Xiaomi | We are fine with FL’s suggestion. |
| Spreadtrum | Support. |
| KDDI | We think “FFS” should be removed from Case 4 and 3-2. For Case 3-1, it should have a priority because it would be typical from an operator perspective. |
| Huawei, HiSilicon | We think the two FFS bullets can be removed and focus on detailed simulation assumptions. Suggest proposal For SBFD evaluation from RAN1 perspective, - High priority - Deployment Case 1 - FFS: Deployment Case 4 (strive for SLS methodology used in RAN1 to be coordinated with RAN4) - FFS: Deployment Case 3-2 with 2-layer, in which one layer uses legacy TDD operation and the other layer uses SBFD operation - Optional - Deployment Case 3-1 with 1-layer - Deployment Case 2 Note: For deployment cases that are optional, they can be captured in TR and it is up to each company to provide the results. The results can be used to draw conclusion/recommendation depending on the number of companies providing the results. |
| Sony | Support. |
| Nokia, NSB | We support for ZTE’s or HW’s updated proposal. |
| vivo | We support the updated version from HW. |
| Ericsson | We agree with ZTE’s or HW’s proposed modification but we also acknowledge the comments from Mediatek about not having any priority scenarios. |
| QC | Similar views as MediaTek, we should follow Mr. chair guidelines w/o priotrization. Even if a deployment scearnio is listed as high priority or optiona, there is no mandate that companie must submit results. Each company can submit evalatuion results of scenario of interest. The note is good enough and provide good guidelines on how to capture results into TR and how condlucsion/recommendation are determined. For SBFD evaluation from RAN1 perspective, - High priority - Deployment Case 1 - FFS: Deployment Case 4 (strive for SLS methodology used in RAN1 to be coordinated with RAN4) - FFS: Deployment Case 3-2 with 2-layer, in which one layer uses legacy TDD operation and the other layer uses SBFD operation - Note: Case 4 and Case 3-2 may need coordination between RAN1 and RAN4. - Optional - Deployment Case 3-1 with 1-layer - Deployment Case 2 Note: For deployment cases that are optional, they can be captured in TR and it is up to each company to provide the results. The results can be captured in TR and used to draw conclusion/recommendation depending on the number of companies providing the results. |
| Intel | We support the FL’s proposal as is. |
| InterDigital | We share same view as MediaTek and Qualcomm to simply remove all the prioritization and having the list to be agreed, as this was Chairman’s clear guidance in the last GTW session. Qualcomm’s revision is the way forward and should be supported. |
| NTT DOCOMO | We are fine with the proposal, and prefer to remove FFS. |
| Samsung | We support the FL’s proposal. |
| CATT | We support the FL proposal. |

### Updated proposal 1-4b:

For SBFD Deployment Case 4, at least consider the following scenarios for evaluation from RAN1 perspective:

- FR1: Urban Macro

- FR2-1: Dense Urban Macro layer

- FFS: UE outdoor/indoor proportion, clustering, etc

- FFS: the grid shift between two networks, e.g., 0%, 100%

- FFS: Indoor hotspot, Dense Urban Micro layer

| Company | Comment |
| --- | --- |
| Moderator | Regarding ‘FFS: UE outdoor/indoor proportion, clustering, etc’, the ‘UE outdoor/indoor proportion’ may apply to FR1 only, and maybe only ourdoor is considered for FR2, but it’s not clear whether or not UE clustering also needs to be considered for FR2. Anyway, we will discuss these details later, so I put it general for FR1 and FR2-1. Let’s check last time if companies are fine with this proposal. I understand we haven’t decide whether Case 4 is high priority or optional, but just as chairman guided, we need to agree the detailed assumptions for these Cases, and further coordinate with RAN4, so Let’s make some progress. |
| ZTE | OK |
| New H3C | Support FL proposal. |
| MediaTek | Support |
| Xiaomi | OK |
| Spreadtrum | Support. |
| Huawei, HiSilicon | We are also fine with the proposal. |
| Sony | Support. |
| Nokia, NSB | Support the proposal. |
| vivo | We are fine with the proposal |
| Ericsson | We support the proposal. |
| QC | Thank for the FL clarification. If that is the common understanding that FFS on UE dropping will be discussed further for FR1 and FR2, we are okay with the FL proposal. |
| Intel | OK with the proposal. |
| InterDigital | OK |
| NTT DOCOMO | We are fine with the proposal. |
| Samsung | Fine with the proposal |
| CATT | We support the FL proposal. |

# 3 Issue#2: Deployment scenarios for dynamic/flexible TDD

## 3.1 Submitted proposal

| Company | Proposals |
| --- | --- |
| CMCC | Proposal 2: Focus on the following scenario for dynamic/flexible TDD evaluation: - HetNet scenario with Urban Macro using DL dominated fixed TDD frame structure and Indoor hotspot using UL dominated fixed TDD frame structure in FR1 - FFS: whether flexible TDD can be additionally considered for indoor hotspot Proposal 27: For dynamic/flexible TDD evaluation, consider channel model as following: - TRxP-UE - Macro-UE: UMa - InH TRxP-UE: InH-Open Office - TRxP-TRxP:. - Macro-Macro: UMa (hUE =25m) - InH TRxP- InH TRxP: InH-Open Office (hUE =3m) - Macro-InH TRxP: UMa (with O2I penetration loss and hUE =3m) - UE-UE: - If the two UEs are in the same Indoor open office: follow Scenario A1 in Table 9 in section 5.1.2. - Otherwise: follow Scenario A3 in Table 9 in section 5.1.2. Proposal 28: For dynamic/flexible TDD evaluation, consider TDD configuration as following: - Macro: DL heavy TDD UL/DL configuration, e.g., DDDSU, S (10DL : 2GP: 2UL) - Indoor: UL heavy TDD UL/DL configuration - Mandatory: Fixed TDD UL/DL configuration, e.g., DSUUU, S (10DL : 2GP : 2UL) - Optional: Dynamic TDD UL/DL configuration based on variable UL/DL traffic ratio Proposal 29: For dynamic/flexible TDD evaluation, at least include DL/UL UPT (5%, 50%, 95%, Average) as performance metric. |
| Huawei | Proposal 1: To choose the deployment scenarios for Rel-18 NR duplex evolution, the industrial interest is an important factor that should be taken into account. Proposal 2: Scenario 2-1 should be studied with a high priority for dynamic/flexible TDD enhancement. - Scenario 2-1: Macro with DL dominant TDD configuration and Pico with UL dominant TDD configuration at local area (FR1 only). |
| ZTE | Proposal 2: Perform thorough analysis and study for dynamic/flexible TDD with the same priority as sub-band non-overlapping duplex. Proposal 4: RAN1 studies at least the following scenarios in Rel-18 duplex SI - For subband full duplex: urban macro, dense urban, indoor hotspot and urban macro + indoor hotspot (optional). - For dynamic/flexible TDD: dense urban, and urban macro + indoor hotspot Proposal 5: RAN1 studies the following potential deployment for dynamic TDD and subband full duplex. - Deployment#1 (1st priority): Dynamic TDD, all the gNBs are legacy TDD gNB in the same channel with two different TDD slot formats. - Deployment#2 (1st priority): Subband full duplex, all the gNBs are subband full duplex gNB in the same channel with the same time/frequency pattern without any co-existence co-channel or adjacent channel interference. - Deployment#3 (1st priority): Subband full duplex, all the gNBs in channel#1 are subband full duplex gNB with the same time/frequency pattern. There are also other legacy TDD gNBs in adjacent channel#2. - Deployment#4 (1st priority): Subband full duplex, some gNBs in channel#1 are subband full duplex gNB with the same time/frequency pattern. There are also other legacy TDD gNBs in channel#1. - Deployment#5 (2nd priority): Subband full duplex, gNBs in channel#1 are subband full duplex gNB with the different time/frequency patterns. - Deployment#6 (2nd priority): Isolated case. There is only one single subband full duplex gNB without any interference from other co-channel or adjacent channel gNBs. Proposal 10: For system level simulation for subband full duplex and dynamic TDD, - DL and UL need to be simulated simultaneously in the same system - Performance metrics: Cell capacity (i.e., average number of supported users per cell) - Traffic model: XR or FTP3 Proposal 13: For Subband full duplex and dynamic TDD simulation, - Perform geometry calibration for subband full duplex at least for Urban macro, Indoor hotspot, Dense urban and urban macro + indoor hotspot (optional). - Perform geometry calibration for dynamic TDD at least for Dense urban and urban macro + indoor hotspot. - Consider the ACLR/ACIR model defined in TR38.828 for geometry calibration. - Consider two different self-interference capabilities for gNB, i.e., 90dB and 130dB. Proposal 14: Consider the simulation parameters in table 7-1 in Appendix for geometry calibration. |
| CATT | Proposal 2: The deployment scenarios for flexible/dynamic TDD evaluation include indoor hotspot and heterogeneous deployment with macro and indoor. Proposal 3: System-level evaluation is used for Rel-18 SBFD and flexible/dynamic TDD evaluation. Proposal 16: Performance of flexible/dynamic TDD with existing CLI handling schemes is the baseline to show potential performance gain of new CLI handling schemes. Proposal 17: Adopt simulation assumptions in Table 2 for flexible/dynamic TDD system evaluation. |
| vivo | Proposal 1: For SBFD and dynamic TDD evaluation, system-level simulation assumptions in TR 38.828 can be used as a starting point. Proposal 2: For non-coexistence scenarios in SBFD or dynamic TDD evaluation, UMa, 1-layer DU and InH can be high priority. Proposal 3: 100% grid-shifted deployment of gNB can be used for multiple operators’ co-existence scenarios. Proposal 4: For co-coexistence scenarios in SBFD or dynamic TDD evaluation, UMa, 1-layer DU and InH, can be considered. |
| xiaomi | Proposal 3: For flexible/dynamic TDD, evaluate and study the performance in HetNet scenario. |
| Samsung | Proposal 1: For evaluation purpose, the following three categories of applicable scenarios can be evaluated with their priorities. - The non-coexistence applicable scenario with highest priority - The coexistence applicable scenario of static TDD and a new duplex operation with medium priority - The coexistence applicable scenario of two new duplex operations with low priority Proposal 2: For non-coexistence applicable scenario, RAN1 takes the following applicable scenarios as a baseline - Indoor hotspot (FR1/FR2) with SBFD-only deployment and dynamic/flexible TDD-only deployment - 1-layer Dense Urban (FR1/FR2) with SBFD-only deployment and dynamic/flexible TDD deployment - Urban Macro (FR1) with SBFD-only deployment and dynamic/flexible TDD-only deployment - Note: down-selection/prioritization of non-coexistence applicable scenarios can be done in Rel-18 Proposal 3: For the coexistence applicable scenario of static TDD and a new duplex operation, RAN1 takes the following applicable scenarios as a baseline - 1-layer/2-layer Dense Urban (FR1/FR2) with coexistence deployment of static TDD and SBFD or dynamic/flexible TDD. - For 1-layer, 1 gNB or 7 gNBs with capability of SBFD or dynamic/flexible TDD are surrounded by gNBs with static TDD - For 2-layer, Macro layer is for static TDD and Micro/indoor hotspot layer is for SBFD or dynamic/flexible TDD or Macro layer is for SBFD and Micro/indoor hotspot layer is for static TDD - Note: down-selection/prioritization of the coexistence applicable scenario of static TDD and a new duplex operation can be done in Rel-18 Proposal 4: For the coexistence applicable scenario of two new duplex operations, RAN1 takes the following applicable scenarios as a baseline - 2-layer Dense Urban (FR1/FR2) with coexistence deployment of SBFD and dynamic/flexible TDD deployment - For 2-layer, Macro layer is for SBFD and Micro/indoor hotspot layer is for dynamic/flexible TDD - Note: down-selection/prioritization of the coexistence applicable scenario of two new duplex operations can be done in Rel-18 Proposal 5: For evaluation purpose, RAN1 to prioritize the aligned subband configuration for SBFD operation. Proposal 6: To evaluate impacts on adjacent channel, consider the followings as a baseline - For Macro layer (in Urban macro or Dense urban), - Two operators’ gNBs are located at the same (0% grid shift) - The second operator’s gNBs are located at edge of the first operator’s gNB (100% grid shift) - FFS: indoor hotspot deployment - Adjacent channel uses static TDD operation. |
| Nokia, Nokia Shanghai Bell | Proposal 1: The dynamic TDD study should focus on identifying enhancements and solutions that allow gNBs to more freely adjust their TDD radio frame configuration in line with their traffic demands by combating cross-link interference. Proposal 2: For evaluating the performance and identifying enhancements for dynamic TDD operation, focus on a two-layer HetNet scenarios with outdoor macro cells + outdoor or indoor small cells with cochannel deployment (e.g. based on Scenario 1 from TR 36.872). - Macro layer uses static DL-heavy TDD radio frame configuration (to ensure inter-operator coexistence) while the small cell layer operates with dynamic TDD in line with their UL and DL traffic demands. Proposal 8: For the traffic models and KPIs for dynamic TDD evaluations, assume the following: - FTP3 traffic model with large payload size, e.g. 0.5 MBytes. - Main KPI: User perceived throughput (UPT). - Offered load selected to meet a certain resource utilization (RU) target, e.g. 10% (low load) and 40% (medium load). - Both DL-heavy traffic, 4:1, and symmetric 1:1 DL:UL traffic can be considered. For the proposed HetNet scenario with dynamic TDD, it may be further considered to have DL-heavy traffic for the UEs connected to the macro-cells, while the small-cell UEs have balanced or UL-heavy traffic. |
| LG Electronics | Proposal 1: For deployment scenarios of study on NR duplex evolution, deployment scenarios (i.e., Dense urban, Urban macro, and Indoor hotspot) which were applied for flexible duplex evaluation in Rel-14 NR SI are considered. Proposal 2: Evaluation for NR duplex evolution is performed at FR1 and FR2-1. Also, it can be discussed whether or not to consider FR 2-2 for study of NR duplex evolution. |
| MediaTek | Observation 5: The feasibility and the performance of SBFD and DTDD schemes highly depend on the deployment scenarios considered for these schemes. Proposal 4: For the evaluations of SBFD and DTDD schemes, RAN1 should consider the deployment scenarios listed in Table 1 and Table 2. Proposal 6: For deployment scenarios with two operators, as starting point, 0% and 100% grid shift are assumed between the two operators’ gNBs: - For the 100% grid shift in Macro deployment, gNBs of the second operator are shifted, relative to the gNBs’ locations of the first operator, by ±ISD/2 on one axis and ±ISD/(2*sqrt(3)) on the other axis. Table 2: Deployment scenarios and topologies for DTDD. Scenario No.; Operator#1; Operator#2; Notes. 1; Macro; Macro; Grid shift: 0%, 100%. 2; HetNet: Macro-Small; Macro; Macro cells: aligned TDD pattern Indoor cells: misaligned with Macro cells. 3; Indoor; Indoor; Misaligned TDD patterns, Grid shift: between 0% and 100%. |
| Intel Corporation | Proposal 1: For the evaluations of both NOFD, and dynamic/flexible TDD, - For FR-1, both indoor office and urban macro scenario could be considered: - For FR-2, in addition to indoor office and urban macro scenario, the dense urban scenario could be considered. Proposal 7: For system-level evaluations on dynamic/flexible TDD - Simulation assumptions, including deployment scenarios, antenna configurations, and related assumptions, summarized in Table 3-7 in the Appendix II can be considered as starting points for system-level evaluations for dynamic/flexible TDD operation in FR1 and FR2. - Non-full buffer with FTP traffic model 3 should be used for traffic modelling. - DL/UL UPT should be used as the primary performance metric. - Specific dynamic/flexible TDD configuration choices could be reported by companies. |
| Qualcomm Incorporated | Proposal 22: For FR1, support the following deployment scenarios for study on potential enhancement on Dynamic/flexible TDD. - Baseline: UMa. - Optional: HetNet (Macro and Pico) Proposal 23: For FR2, support the following deployment scenarios for study on potential enhancement on Dynamic/flexible TDD. - Baseline: UMa (FR2-1) macro and InH (FR2-1) - Optional: UMi (FR2-1) and InH (FR2-2) Proposal 24: Support SLS as main tool for the evaluation of potential enhancement of dynamic/flexible TDD study. Proposal 25: For dynamic/flexible TDD, - Utilize the BS antenna configuration of legacy baseline TDD - Slot format is all flexible FFFFF (D or U direction is picked based on traffic) Proposal 26: All other simulation assumptions dynamic/flexible TDD evaluation can be the same as legacy TDD, e.g. antenna configuration could be the same as legacy TDD with single panel configuration for Tx or Rx. For FR2, the bandwidth configuration dynamic/flexible TDD evaluation could be either all for DL or all for UL. |

## 3.2 Summary

In Rel-14 NR SI, flexible TDD has been evaluated for indoor hotspot, urban macro and dense urban scenarios. Some companies suggest to take these deployment scenarios as a starting point, but some other companies suggest to avoid the repetition work that has been done before, and to focus on HetNet scenario with outdoor Macro cells and indoor small cells deployed in the same carrier which has not been studied sufficiently yet. The interested scenarios from companies are summarized as below:

- HetNet (Urban Macro + Indoor hotspot):

  - CMCC (FR1), Huawei (FR1), ZTE, CATT, xiaomi, Samsung (FR1, FR2), Nokia, [MediaTek], Qualcomm (2nd priority for FR1)

- Dense Urban:

  - ZTE, vivo (1-layer), Samsung (FR1, FR2), Nokia, LG, [MediaTek], Intel (FR2)

- Indoor hotspot:

  - vivo, Samsung (FR1, FR2), LG, MediaTek, Intel (FR1, FR2), Qualcomm (FR2-1)

- Urban Macro:

  - vivo, Samsung (FR1), LG, MediaTek, Intel (FR1, FR2), Qualcomm (FR1, FR2-1)

- Urban Micro:

  - Qualcomm (2nd priority for FR2-1)

For dynamic/flexible TDD evaluation in HetNet (Urban Macro + Indoor hotspot) or Dense urban with two layers, the following TDD configurations are proposed:

- Macro using DL dominant static TDD and small cell using UL dominant static TDD

  - CMCC, ZTE, Huawei

- Macro using DL dominant static TDD and small cell using dynamic TDD

  - Samsung, Nokia

Moderator suggests Initial proposal 2-1.

Regarding adjacent-channel coexistence between two operators with legacy TDD operation and dynamic TDD operation, it has been evaluated in Rel-16 CLI WI, some companies [Nokia, CMCC] suggest to avoid repeating such evaluation in Rel-18, while some other companies [Samsung] show interest in it. Moderator suggests Initial question 2-2.

## 3.3 1st Round Proposals (closed)

### Initial proposal 2-1:

For evaluation of dynamic/flexible TDD deployed by one single operator, at least consider the following scenarios for evaluation for FR1 and FR2:

- HetNet with Urban Macro and Indoor hotspot deployed in the same carrier:

  - Macro gNBs use DL dominant static TDD UL/DL configuration (e.g., DDDSU, S (10DL: 2GP: 2UL)), and Indoor gNBs use UL dominant static TDD UL/DL configuration (e.g., DSUUU, S (10DL: 2GP: 2UL))

    - (Optional) Indoor gNBs use dynamic TDD UL/DL configuration

- Dense Urban with two layers deployed in the same carrier:

  - Macro gNBs use DL dominant static TDD UL/DL configuration (e.g., DDDSU), and Micro gNBs use UL dominant static TDD UL/DL configuration (e.g., DSUUU)

    - (Optional) Micro gNBs use dynamic TDD UL/DL configuration

- FFS: Indoor hotspot, Urban Macro

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| QC | Don’t support. Urban Macro should be considered as baseline scenario for evaluation of dynamic TDD. It is surprising that both InH and Urban Macro listed as FFS while it has same support level as Dense urban! |
| New H3C | We firstly prefer to consider Macro and Indoor hotspot separately with high prioroty and then consider HetNet scenario with 2nd priority. Of course we are open to discuss about this proposal if majority views |
| CATT | We support the first scenario and would like to consider indoor hotspot with dynamic TDD as well. |
| Spreadtrum | We support this proposal and prefer option 1 of Hetnet with Macro + indoor scenario. Considering the outcome of eIMTA and CLI&RIM, the penetration loss between outdoor and indoor decrease the impact of interference and make it a possible scenario. Besides, it is a realistic deployment of industry. |
| ZTE | Now it seems only Hetnet is selected for dynamic TDD simulation. We understand that Hetnet should be prioritized for dynamic TDD. But homogenous network should also be considered in the simulation to fully understand the potential uses for dynamic TDD. Thus, we are supportive to add indoor hotspot and Urban Macro. In addition, we suggest to delete the example TDD configurations at this stage because the TDD configurations may be related to the frequency for simulation. For example, for 4GHz, DDDSUDDSUU may be adopted. |
| NEC | Agree with listed scenarios but we are not sure why dynamic TDD for indoor and micro gNB is considered optional in the above proposal. Without this feature, it might not be able to achieve the required performance gain. |
| vivo | The current proposal seems to prioritize different TDD configuration among different cells, such that it may make the study of dynamic TDD as optional. However, study of dynamic TDD is clearly in scope of the SI, which should not be considered as optional for study. For the scenario, we prefer to use the same scenarios for both SBFD and dynamic/flexible TDD evaluation, while the HetNet deployment can be optionally evaluated. |
| Sharp | At least InH and Urban Macro can be supported for FR1. We think some level of alignment between SFBD and dynamic/flexible TDD should be considered since those functionalities are interrelated. |
| Sony | Fine with proposal. |
| MediaTek | Generally fine with the proposal. Also, we are fine with including homogenous network Urban Macro with different TDD patterns. |
| Xiaomi | For dense Urban with two layers deployed in the same carrier, the gNB-to-gNB (especially Macro to Micro) is severe because of LOS between them. From this perspective, the simulation results would be quite pessimistic from the lesson we studied in Rel-16 CLI. We are not sure why this case is regarded as the baseline for dynamic TDD. Could proponent elaborates a bit? |
| Huawei | Support scenario 1 and scenario 2 since there are clear commercial use cases from the vertical market and it is more feasible as pointed out by Spreadtrum. Question to the proponent of applying dynamic TDD to urban macro, what is the expected use case for this?! In addition, the Rel-16 co-existence study has already conclude the following. What has been changed since Rel-16? #### 6.3.1.1 Macro-to-Macro scenario - Performance degradation was observed from the BS-to-BS interference for macro-macro scenario, which suggests that dynamic TDD should not be operated in such scenarios. |
| Ericsson | We think that two operator urban macro scenario need to be considered as the baseline. In real-world deployments, single operator may not operate over a region. Usually there are multiple operators in close vicinity to each other in a region. Furthermore, we propose that the study in RAN1 should take a phased approach based on SBFD with difference in UL subband size between gNBs for evaluating dynamic TDD where potential CLI issues and whether solutions are needed, are identified in each phase. Each phase introduces increasing sources of difference first between and then within each operator's network. We view full dynamic TDD as a special case of SBFD with UL subband size 0% and 100% of the carrier BW. In our proposal these are the following phases that could be considered : Phase 0 (Baseline: SBFD coexistence with static TDD) Phase 1 (SBFD coexistence) Phase 2 (Flexible SBFD coexistence) Phase 3 (Dynamic TDD with a protected UL-only slot) Phase 4 (Dynamic TDD): |
| Nokia/NSB | We prefer dynamic TDD for indoor gNB as a baseline. We think dynamic TDD with Macro-layer is difficult to be supported based on Rel-16 study, and SBFD can be an alternative solution. Instead, we think HetNet scenario static Macro and dynamic Indoor/pico cell should be the most important usecase. |
| Intel | Do not support. While we’re open to considering HetNet scenarios (e.g., UMa + Hotspot) and see their value, we agree with QC that UMa and InH should form the baseline scenarios, similar to SBFD. |
| OPPO | Urban macro [with single layer] can be used as baseline case for dynamic/flexible TDD evaluation. |
| InterDigital | Share similar views with Qualcomm, Intel, OPPO, and more. Urban macro and Indoor hotspot should be considered as baseline for dynamic TDD. |
| Samsung | For dynamic/flexible TDD, we support the same scenarios as we support for SBFD operation to compare which duplex operation is promising under each scenario. Also, we are ok to exclude Urban Macro layer for both FR1 and FR2 from the conclusion of Rel-16 CLI in TR38.828 below. FR1: “Performance degradation was observed from the BS-to-BS interference for macro-macro scenario, which suggests that dynamic TDD should not be operated in such scenarios” FR2 “Some performance degradation was observed from the BS-to-BS interference for macro-macro scenario. The differences in the simulation results imply that operating dynamic TDD in this scenario without impact to neighbor network may be deployment dependent and requires at least careful planning and collaboration between operators to avoid performance impact.” |
| LG | We are ok with the proposal. |
| Moderator | I added Indoor office based on comments. Although some companies prefer to consider Urban Macro, some other companies argue it should be excluded for both FR1 and FR2 based on the conclusion of Rel-16 CLI in TR38.828. |

### Initial question 2-2:

Whether the adjacent-channel coexistence case between dynamic TDD and legacy TDD defined as below needs to be considered for evaluation or not?

- Adjacent-channel co-existence between dynamic TDD and legacy TDD: Two operators each using one carrier are considered and the two carriers are adjacent carriers. One operator uses legacy TDD operation while the other operator uses dynamic TDD operation.

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| QC | Don’t support the study of coex between dynamic and legacy TDD. This means repeating of earlier R16 work on coexistence. What should be considered is the coexistence between SBFD and legacy TDD (Case 4). |
| NTT DOCOMO | We think this is important aspect for inter-operator scenario. On the other hands, we think SBFD is more realistic solution so that this point can be considered in SBFD evaluation. |
| New H3C | It is fine to consider this case. |
| CATT | We do not see the need given the previous study in Rel-16. |
| Spreadtrum | In R16 CLI&RIM, adjacent channel coexistence cases have been studied and they are exactly the same cases as described here. So we think the repetition of the same discussion should be avoided. |
| ZTE | We are open to consider this case. However, SBFD already covers the adjacent channel co-existence between SBFD and legacy TDD. To avoid duplicated effort, we can prioritize other cases first. |
| NEC | Yes, adjacent-channel coexistence needs to be studied as well |
| vivo | We agree the adjacent channel co-existence should be studied for dynamic TDD, as it is our understanding that adjacent-channel co-existence issue is the key problem that makes dynamic TDD unsuccessful in the commercial deployment. Therefore it should be studied with high priority. |
| Sharp | We have similar view with NTT DOCOMO that adjacent-channel co-existence between dynamic/flexible TDD and legacy TDD can be considered in SBFD. |
| Sony | Share similar view with QC & DOCOMO that it is sufficient to consider Case 4 of SBFD + legacy TDD. |
| MediaTek | Yes, we support studying this case given that the simulation assumptions may be different from R16 work, which could result in different findings. |
| Xiaomi | Same view as Qualcomm/CATT. |
| Huawei | Adjacent-channel co-existence between dynamic TDD and legacy TDD has been evaluated in Rel-16 NR study, and we should not repeat the evaluation again. |
| Ericsson | We think adjacent channel coexistence case needs to be considered as the baseline; hence, the answer to Question 2-2 is “yes”. Similar to SBFD study, the crucial aspect in SID is the impact to legacy and this is addressed by the two-operator scenario. It is important to understand the impact to legacy operators as this would determine the deployment feasibility of dynamic TDD network. In other words, no matter the gain from dynamic TDD if the impact to legacy is significant it may not be allowed. Our understanding is that the Rel 16 study was not comprehensive in the analysis of adjacent channel cases (0% grid shift was not simulated, as usually in reality one would find multiple operators co-sited), and hence there is a need to augment the study done in Rel 16 with new and realistic assumptions. |
| Nokia/NSB | We share view with Huawei. We don’t see need for co-existence study should be repeated in Rel-18. |
| Intel | Adjacent channel coexistence studies fall under RAN4 domain. Thus, we prefer not to repeat the same exercise. RAN4 did this once and it would not be necessary for them to repeat it again. |
| InterDigital | As mentioned by many companies, repeating same or similar evaluations/discussions from Rel-16 are to be avoided according to the SID. |
| Samsung | It should be decided in RAN4 since the R16 CLI evaluation was performed in RAN4. |
| LG | It is redundant since adjacent-channel co-existence study was performed in Rel-16 CLI. |
| Moderator | Majority [QC, DoCoMo, CATT, Spreadtrum, ZTE, Sharp, Sony, Xiaomi, Huawei, Nokia, Intel, Interdigital, Samsung] prefer to not repeat the evaluation since it has been done in Rel-16, or it is sufficient to consider SBFD Deployment Case 4 for co-existence between SBFD and legacy TDD, or such evaluation can be decided by RAN4 since the R16 CLI evaluation was performed in RAN4. Some other companies [NEC, vivo, MTK, Ericsson] prefer to consider this scenario as high priority. |

## 3.4 2nd Round Proposals (closed)

### Updated proposal 2-1:

For evaluation of dynamic/flexible TDD deployed by one single operator, at least consider the following scenarios for evaluation for FR1 and FR2:

- Scenario 1: HetNet with Urban Macro and Indoor office deployed in the same carrier, and Macro gNBs use DL dominant static TDD UL/DL configuration.

  - Option 1: Indoor gNBs use UL dominant static TDD UL/DL configuration

  - Option 2: Indoor gNBs use dynamic TDD UL/DL configuration

- Scenario 2: Dense Urban with two layers deployed in the same carrier, and Macro gNBs use DL dominant static TDD UL/DL configuration.

  - Option 1: Micro gNBs use UL dominant static TDD UL/DL configuration

  - Option 2: Micro gNBs use dynamic TDD UL/DL configuration

- Scenario 3: Indoor office

- FFS: Urban Macro

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| New H3C | We are fine with this proposal |
| Intel | We can accept dropping UMa for dynamic TDD, but prefer to at least down select between Scenarios 1 and 2. Alternatively, we suggest removing the “at least” in the main bullet. |
| Samsung | Generally fine with 3 scenarios. Further clarifications are needed. - For Indoor office, it is not clear that all gNBs have dynamic TDD or different static UL/DL configuration or comibination of them. - For the case where gNBs use dynamic SFI, some of UEs can support the dynamic SFI while other UEs cannot support. Is it assumed that all UEs have the dynamic SFI capability? |
| vivo | We are fine with this proposal |
| CATT | We are fine in general but would like to clarify whether the two options for scenario 1 and 2 would be further down-selected or not. |
| ZTE | OK with the proposal. |
| MediaTek | We are fine with the principle of the proposal, but we need to discuss the assumptions for UE distributions in these deployments. As we shown in our contribution R1-2204721, the assumption of uniformly and randomly distributed UEs doesn’t reflect real scenarios and it will not be possible to evaluate the impact of inter-UE CLI on the system performance. Hence, as proposed in R1-2204721 and R1-2204106, UEs clustering need to be considered. |
| Sony | Support. |
| Nokia, NSB | We generally support the proposal. For Indoor office, it could be clarified that all gNBs have dynamic (and indenpendently adjusted) TDD? |
| NEC | We support the proposal |
| Huawei, HiSilicon | We are fine with this proposal. |
| Xiaomi | We are fine with scenario 1-3. As commented in perious round, we don’t think urban Macro should be considered. |
| Spreadtrum | Prefer Scenario 1 with option1. Penetration loss make scenario 1 to be a more reasonable cast to get benefit. We suggest to study scenario 1 at the first stage. |
| Ericsson | We do not support the proposal. We think the same two operator Urban Macro could also be evaluated for dynamic TDD. |
| QC | We share similar views as Ericsson, we don’t support FL proposal. The scope of this SI is potential enhancement on dynamic/flexible TDD. Urban Macro is the main commercial deployment scenario. It should be the baseline deployment scenario. We don’t agree with the FL proposal on excluding Urban Macro from the study. Besides, the scenarios for FR1 and FR2 for dynamic TDD need to be separately evaluated, similar to proposal 1-3. - For FR1, urban macro and indoor hotspot should be considered. - For FR2, dense urban and indoor hotspot should be considered. |
| Moderator | Updated based on comments. Let’s further check. |

### Initial proposal 2-2:

In Rel-18, the adjacent-channel coexistence case between dynamic TDD and legacy TDD is low priority.

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| Apple | Support |
| New H3C | We are fine with this proposal |
| Intel | Support, from a RAN1 perspective. In any case, it should be left to RAN4. |
| Samsung | It is only for RAN1 evaluation scope, and should be confirmed in RAN4. So, we suggest the following modification In Rel-18, the adjacent-channel coexistence case between dynamic TDD and legacy TDD is low priority from RAN1 perspective. |
| vivo | We are not sure why deployment case 4 for SBFD with adjacent-channel coexistence between SBFD and legacy TDD is high priority, while the case for dynamic/flexible TDD with adjacent-channel coexistence between dynamic TDD and legacy TDD would be low priority. We suggest to give equal treatment to the study of SBFD and dynamic/flexible TDD. As we commented earlier, adjacent-channel co-existence issue is the key problem that makes dynamic TDD unsuccessful in the commercial deployment. Therefore, it should be studied with high priority |
| CATT | We agree Samsung’s version. |
| ZTE | If Case 4 is treated as high priority for SBFD, then we are ok to treat adjacent-channel coexistence case between dynamic TDD and legacy TDD as low priority |
| MediaTek | We don’t support. The impact of inter-UE and inter-gNB CLIs for this case needs to be evaluated as well. Unlike the case of the same operator, the CLI handling is challenging for the two operators case. |
| Sony | Support. |
| Nokia, NSB | We support this proposal. RAN4 already studied this in Rel-16, so we don’t see need for Samsung’s modification. |
| Huawei, HiSilicon | Fine with the proposal. |
| Xiaomi | Support Samsung’s modification. |
| Spreadtrum | Agree with this proposal. |
| Ericsson | We share the same view as Vivo. We do not support this proposal. Dynamic TDD network could be viewed as an SBFD network with either 0% or 100% resource allocation for UL. Therefore, if it is applicable to SBFD it must be for dynamic TDD as well. Adjacent channel coexistence is the realistic case that causes elevated gNB-gNB CLI problem that needs to be studied. |
| NTT DOCOMO | We support the proposal. |
| QC | Support. |
| Moderator | This proposal has been merged to updated proposal 2-1a. |

## 3.5 3rd Round Proposals (closed)

### Updated proposal 2-1a:

For evaluation of dynamic/flexible TDD, consider the following scenarios for evaluation:

- FR1

  - Indoor office with dynamic TDD UL/DL configuration

  - HetNet with Urban Macro and Indoor office deployed in the same carrier, and Macro gNBs use DL dominant static TDD UL/DL configuration.

    - Option 1: Indoor gNBs use UL dominant static TDD UL/DL configuration

    - Option 2: Indoor gNBs use dynamic TDD UL/DL configuration

  - Optional: Adjacent-channel coexistence case between dynamic TDD and legacy TDD

  - Optional: Urban Macro with dynamic TDD UL/DL configuration

- FR2

  - Indoor office with dynamic TDD UL/DL configuration

  - Dense Urban Macro layer with dynamic TDD UL/DL configuration

| Company | Comment |
| --- | --- |
| New H3C | We are fine with this proposal |
| ZTE | We noticed that lots of companies are interested in the scenarios 2 listed in the previous proposal. - Scenario 2: Dense Urban with two layers deployed in the same carrier, and Macro gNBs use DL dominant static TDD UL/DL configuration. - Option 1: Micro gNBs use UL dominant static TDD UL/DL configuration - Option 2: Micro gNBs use dynamic TDD UL/DL configuration We propose to add it back. |
| Huawei, HiSilicon | Fine with the proposal. |
| Sony | Support. |
| MediaTek | We support the proposal in general apart from having “Adjacent-channel coexistence case between dynamic TDD and legacy TDD” as optional. The word “optional” should be removed from the proposal for this case. As we discussed in our response to “Updated proposal 1-2a”, this case is part of the SID’s objectives. |
| InterDigital | Fine with the FL proposal |
| Intel | We are OK with the proposal. |
| Nokia, NSB | We support this proposal. |
| QC | The word optional should be removed for the subbullet on “Urban Macro with dynamic TDD UL/DL configuration”. |
| Fujitsu | Support |
| DOCOMO | We are fine with the proposal. |
| LG | We share the view with ZTE. We’d like to keep “Dense urban with two layers” scenario. |
| vivo | We are fine with the proposal |
| CATT | - “Adjacent-channel coexistence case between dynamic TDD and legacy TDD” is too broad and we think only HetNet with Urban Macro and Indoor office needs to be considered. - We suggest to change “dynamic TDD UL/DL configuration” to “dynamic TDD UL/DL assignment” since “dynamic TDD UL/DL configuration” seems to be dynamic SFI to us and we think dynamic TDD can be also achieved by e.g. dynamic scheduling in flexible symbols. It would be good to clarify whether Option 1 and Option 2 for FR1 will be further down-selected or both of the two options are considered. |
| Ericsson | We support the proposal in principle. We think that the adjacent channel coexistence case should not be optional. Few companies argue that we do not need this study because of the study done in Rel 16. However, the Rel 16 study is not comprehensive and had a set of assumptions that may have been too optimistic. For e.g., the study in Rel 16 assumed full buffer traffic and did not consider impact on dTDD network from legacy networks. Furthermore, the study postulated that 0% grid shift will not work without any studies being done. |
| Spreadtrum | Support |
| Xiaomi | We would like to make a clarification on the assumptions for ‘Urban Macro with dynamic TDD UL/DL configuration’: do we need to consider some interference mitigation scheme in the simulation? If not, we are not quite sure why do we need such kind of simulation as it is recommended in TR38.828 that dynamic TDD should not be operated in Macro scenario, especially in FR1. |
| Samsung | Fine with the proposal. FR2 should be FR2-1. |
| Moderator | Updated. Let’s have another try to check if companies are OK with a phased approach. |

## 3.6 4th Round Proposals (closed)

### Updated proposal 2-1b:

For evaluation of dynamic/flexible TDD, consider the following scenarios for evaluation:

- FR1 (using a phased approach)

  - Phase 1

    - Baseline scenarios

      - Indoor office with dynamic TDD UL/DL assignment

      - HetNet with Urban Macro and Indoor office deployed in the same carrier, and Macro gNBs use DL dominant static TDD UL/DL configuration. Both of the following options can be considered for this scenario.

        - Option 1: Indoor gNBs use UL dominant static TDD UL/DL configuration

        - Option 2: Indoor gNBs use dynamic TDD UL/DL assignment

    - Optional scenarios:

      - Urban Macro with dynamic TDD UL/DL configuration

      - Dense Urban with two layers deployed in the same carrier, and Macro gNBs use DL dominant static TDD UL/DL configuration. Both of the following options can be considered for this scenario.

        - Option 1: Micro gNBs use UL dominant static TDD UL/DL configuration

        - Option 2: Micro gNBs use dynamic TDD UL/DL assignment

  - Phase 2

    - Adjacent-channel coexistence case between dynamic TDD and legacy TDD

      - FFS: detailed scenario for adjacent-channel coexistence case

- FR2-1

  - Indoor office with dynamic TDD UL/DL assignment

  - Dense Urban Macro layer with dynamic TDD UL/DL assignment

| Company | Comment |
| --- | --- |
| Intel | We have same concerns as in proposal 1-2b for SBFD regarding the meaning of phase 1 and phase 2. In short, it is not clear how to determine completion of the first phase, etc. We think the earlier version is much clearer and suggest to classify “Adjacent-channel coexistence case” under Optional without any phases. We understand that the SID lists study of adjacent channel case also for RAN1, but it also has the following statement, and it is clear RAN4 already evaluated this case: Note: For potential enhancements on dynamic/flexible TDD, utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion. |
| CATT | Thanks FL for taking our comments into account. We are in general fine with the proposal except the phased approach as we commented for proposal 1-2b. |
| Ericsson | Same comment on the clarity of when the phase 1 ends and phase 2 starts. We propose to simply remove the phases. We support the proposal in principle. We think that the adjacent channel coexistence case should not be optional. Few companies argue that we do not need this study because of the study done in Rel 16. However, the Rel 16 study is not comprehensive and had a set of assumptions that may have been too optimistic. For e.g., the study in Rel 16 assumed full buffer traffic and did not consider impact on dTDD network from legacy networks. Furthermore, the study postulated that 0% grid shift will not work without any studies being done. ### Updated proposal 2-1b: (Modified by Ericsson) For evaluation of dynamic/flexible TDD, consider the following scenarios for evaluation: - FR1 (using a phased approach) - Phase 1 - Baseline scenarios - Indoor office with dynamic TDD UL/DL assignment - HetNet with Urban Macro and Indoor office deployed in the same carrier, and Macro gNBs use DL dominant static TDD UL/DL configuration. Both of the following options can be considered for this scenario. - Option 1: Indoor gNBs use UL dominant static TDD UL/DL configuration - Option 2: Indoor gNBs use dynamic TDD UL/DL assignment - Optional scenarios: - Urban Macro with dynamic TDD UL/DL configuration - Dense Urban with two layers deployed in the same carrier, and Macro gNBs use DL dominant static TDD UL/DL configuration. Both of the following options can be considered for this scenario. - Option 1: Micro gNBs use UL dominant static TDD UL/DL configuration - Option 2: Micro gNBs use dynamic TDD UL/DL assignment - Phase 2 - Adjacent-channel coexistence case between dynamic TDD and legacy TDD - FFS: detailed scenario for adjacent-channel coexistence case - FR2-1 - Indoor office with dynamic TDD UL/DL assignment - Dense Urban Macro layer with dynamic TDD UL/DL assignment |
| ZTE | Thanks FL for the great effort. We are ok with this phased approach. If there is no strong concern, we would suggest to list “Dense Urban with two layers deployed in the same carrier” in the baseline scenarios for the following reasonse. - In Rel-16, only homogenerous networks are simulated for CLI. It would be better if we can focus on Hetnet in Rel-18. Dense urban is one of the practical scenarios for Hetnet; - The interference situation for dense ubran is different from that for urban macro + indoor hotspot. |
| QC | Thanks FL for the great efforts! We think Urban macro for FR1 should be considered as baseline as well. The adjacent channel study for dynamic TDD should be optional given earlier study in Rel-16. Is phased approach for FR1 only? |
| Samsung | We share the view from Intel that the earlier version is clearer. |
| Huawei, HiSilicon | We prefer the previous version and have a similar view with Intel that the nessessity of redoing coexistence case between dynamic TDD and legacy TDD in Rel-18 is unclear. On the proposal to add Urban macro into the baseline, we don’t understand the reason given that the conclusion from Rel-16 co-existence study #### 6.3.1.1 Macro-to-Macro scenario - Performance degradation was observed from the BS-to-BS interference for macro-macro scenario, which suggests that dynamic TDD should not be operated in such scenarios. |
| vivo | It is not clear what the phase means. Suggest to remove it. Besides, we have the similar view as Ericsson that adjacent channel coexistence case should not be optional. According to the updated proposal 1-2b, case 4 is regarded as high priority. In that sense, adjacent channel coexistence case should also be studied with high priority. The modifications by Ericsson look good to us. |
| Spreadtrum | We are generally fine with this proposal. But prefer to add indoor factor scenario into HetNet scenariso as “HetNet with Urban Macro and Indoor office/factory deployed in the same carrier”. About adjacent channel, we agree with Intel and Huawei to avoid repetition. As to SBFD case, it is because that subband is introduced and interference may be reduced. |
| Nokia, NSB | As commented by other companies, we have preference for the earlier version |
| MediaTek | Similar view as Ericsson |
| Sony | Support if the intention of the phases are to work on Phase 1 first followed by Phase 2. |

## 3.7 5th Round Proposals (open)

### Updated proposal 2-1c:

For evaluation of dynamic/flexible TDD, consider the following scenarios for evaluation:

- FR1

  - High priority

    - Indoor office with dynamic TDD UL/DL assignment

    - HetNet with Urban Macro and Indoor office deployed in the same carrier, and Macro gNBs use DL dominant static TDD UL/DL configuration. Both of the following options can be considered for this scenario.

      - Option 1: Indoor gNBs use UL dominant static TDD UL/DL configuration

      - Option 2: Indoor gNBs use dynamic TDD UL/DL assignment

    - FFS: Adjacent-channel coexistence case between dynamic TDD and legacy TDD

      - FFS: detailed scenario for adjacent-channel coexistence case

  - Optional:

    - Urban Macro with dynamic TDD UL/DL assignment

    - Dense Urban with two layers deployed in the same carrier, and Macro gNBs use DL dominant static TDD UL/DL configuration. Both of the following options can be considered for this scenario.

      - Option 1: Micro gNBs use UL dominant static TDD UL/DL configuration

      - Option 2: Micro gNBs use dynamic TDD UL/DL assignment

- FR2-1

  - Indoor office with dynamic TDD UL/DL assignment

  - Dense Urban Macro layer with dynamic TDD UL/DL assignment

| Company | Comment |  |
| --- | --- | --- |
| Moderator | Suggests that we just take this as agreement to give a list of scenarios that we need to make assumptions for, so that we can further discuss the detailed assumptions for all these scenarios. |  |
| ZTE | Following the guidance from online yesterday, maybe we don’t need to have “High priority” or “optional”. We suggest the following way forward. Proposal: For evaluation of dynamic/flexible TDD, consider the following scenarios for evaluation: - FR1 - High priority - Indoor office with dynamic TDD UL/DL assignment - HetNet with Urban Macro and Indoor office deployed in the same carrier, and Macro gNBs use DL dominant static TDD UL/DL configuration. Both of the following options can be considered for this scenario. - Option 1: Indoor gNBs use UL dominant static TDD UL/DL configuration - Option 2: Indoor gNBs use dynamic TDD UL/DL assignment - FFS: Adjacent-channel coexistence case between dynamic TDD and legacy TDD - FFS: detailed scenario for adjacent-channel coexistence case - Optional: - Urban Macro with dynamic TDD UL/DL assignment - Dense Urban with two layers deployed in the same carrier, and Macro gNBs use DL dominant static TDD UL/DL configuration. Both of the following options can be considered for this scenario. - Option 1: Micro gNBs use UL dominant static TDD UL/DL configuration - Option 2: Micro gNBs use dynamic TDD UL/DL assignment - FR2-1 - Indoor office with dynamic TDD UL/DL assignment - Dense Urban Macro layer with dynamic TDD UL/DL assignment |  |
| New H3C | We are fine with this proposal with ZTE’s medication. |  |
| MediaTek | We don’t support the proposal. As the Chairman suggested in the last GTW session, we could skip the prioritization of the cases. At the end, each company is free to evaluate the case/scenario that they think it is essential for the study. We just need to have a list of the cases that considered for evaluations. |  |
| Xiaomi | We are fine with FL’s suggestion. One clarification on the TDD configuration for Micro layer: the two options is for further downselection or can be reported by companies? Different method to determine TDD configuration may lead to different interference situation. Hence it’s better to have a uniform assumption on it. |  |
| Spreadtrum | We prefer to put Adjacent-channel coexistence case into Optional. No simulation assumption is not changed, the necessary should be further clarified. |  |
| Huawei, HiSilicon | We can follow the same treatment as proposal 1-2c, i.e. remove the FFS bullet. To address the concerns on “Adjacent-channel coexistence case” , we suggest to add an FFS bullet to identify the detailed simulation assumptions that are different from the Rel-16 co-existence study. Suggested proposal For evaluation of dynamic/flexible TDD, consider the following scenarios for evaluation: - FR1 - High priority - Indoor office with dynamic TDD UL/DL assignment - HetNet with Urban Macro and Indoor office deployed in the same carrier, and Macro gNBs use DL dominant static TDD UL/DL configuration. Both of the following options can be considered for this scenario. - Option 1: Indoor gNBs use UL dominant static TDD UL/DL configuration - Option 2: Indoor gNBs use dynamic TDD UL/DL assignment - FFS: Adjacent-channel coexistence case between dynamic TDD and legacy TDD - FFS: detailed scenario for adjacent-channel coexistence case - FFS: detailed simulation assumptions that are different from the Rel-16 co-existence study. - Optional: - Urban Macro with dynamic TDD UL/DL assignment - Dense Urban with two layers deployed in the same carrier, and Macro gNBs use DL dominant static TDD UL/DL configuration. Both of the following options can be considered for this scenario. - Option 1: Micro gNBs use UL dominant static TDD UL/DL configuration - Option 2: Micro gNBs use dynamic TDD UL/DL assignment - FR2-1 - Indoor office with dynamic TDD UL/DL assignment - Dense Urban Macro layer with dynamic TDD UL/DL assignment |  |
| Sony | Support. |  |
| Nokia, NSB | Support FL proposal. Regarding to Adjacent-channel coexistence, based on the Rel-16 study, performance degaration is marginal and not observed with enough isolation. We don’t expect co-located gNB of other operator for indoor gNB, so in this case, we don’t need to consider adjacent channel interference. We are fine FFS, but do not agree with removing FFS. TR38.828 6.3 recommendation #### 6.3.1.1 Macro-to-Macro scenario - Performance degradation was observed from the BS-to-BS interference for macro-macro scenario, which suggests that dynamic TDD should not be operated in such scenarios. #### 6.3.1.2 Indoor scenarios (Indoor-to-Macro and Indoor-to-Indoor) - Performance degradations were not observed from operating dynamic TDD between an indoor network and a macro network and vice versa if there is sufficient isolation between them. No significant impact from operating dynamic TDD for the indoor scenario was observed as long as the BS and UE powers are similar and the operators co-ordinate so that basestation positions are offset. If higher BS power is assumed, some throughput degradation in the indoor scenario was observed due to BS to BS interference. The observations imply that dynamic TDD can be used in indoors as long as care is taken. |  |
| vivo | We support the updated version from HW. |  |
| Ericsson |  | We support the proposal with ZTE’s or HW’s modification. |  |
| QC |  | Similar comment as proposal 1-2c. We should follow Mr. chair guidelines. Even if a deployment scearnio is listed as high priority or optional, there is no mandate that companie must submit results. Each company can submit evalatuion results of scenario of interest. The note is good enough and provide good guidelines on how to capture results into TR and how condlucsion/recommendation are determined. We are fine with ZTE’s modification. In addition, we can add the same note in proposal 1-2c. For consistency between FR1 and FR2 adjacent channel scenario should be added as well. For evaluation of dynamic/flexible TDD, consider the following scenarios for evaluation: - FR1 - High priority - Indoor office with dynamic TDD UL/DL assignment - HetNet with Urban Macro and Indoor office deployed in the same carrier, and Macro gNBs use DL dominant static TDD UL/DL configuration. Both of the following options can be considered for this scenario. - Option 1: Indoor gNBs use UL dominant static TDD UL/DL configuration - Option 2: Indoor gNBs use dynamic TDD UL/DL assignment - FFS: Adjacent-channel coexistence case between dynamic TDD and legacy TDD - FFS: detailed scenario for adjacent-channel coexistence case - Optional: - Urban Macro with dynamic TDD UL/DL assignment - Dense Urban with two layers deployed in the same carrier, and Macro gNBs use DL dominant static TDD UL/DL configuration. Both of the following options can be considered for this scenario. - Option 1: Micro gNBs use UL dominant static TDD UL/DL configuration - Option 2: Micro gNBs use dynamic TDD UL/DL assignment - FR2-1 - Indoor office with dynamic TDD UL/DL assignment - Dense Urban Macro layer with dynamic TDD UL/DL assignment - Adjacent-channel coexistence case between dynamic TDD and legacy TDD - FFS: detailed scenario for adjacent-channel coexistence case Note: it is up to each company to provide the results. The results can be captured in TR and used to draw conclusion/recommendation depending on the number of companies providing the results. |  |
| Intel |  | Share same view with Nokia, and we are OK with the FL’s proposal and to keep the FFS. |  |
| Samsung |  | Support the FL’s proposal |  |
| CATT |  | We support the FL proposal. |  |

# 4 Issue #3: Interference modeling and LS to RAN4

## 4.1 Submitted proposals

- Interference haracter

| Company | Proposals |
| --- | --- |
| CMCC | Observation 1: There are self-interference (SI), co-channel inter-subband CLI (including co-channel inter-cell gNB-gNB inter-subband CLI and co-channel intra-cell/inter-cell UE-UE inter-subband CLI), co-channel intra-subband CLI (including co-channel inter-cell gNB-gNB intra-subband CLI and co-channel inter-cell UE-UE intra-subband CLI), and adjacent-channel CLI (including adjacent-channel gNB-gNB CLI and adjacent-channel UE-UE CLI) in SBFD network. Proposal 5: At least for RAN1’s discussion and evaluation for SBFD, define the following interference types: - Self-interference (SI) at gNB side: Interference caused by DL transmission in DL subband in a SBFD carrier to UL reception in UL subband in the same carrier at the gNB side. - Co-channel inter-cell gNB-gNB inter-subband CLI: CLI caused by DL transmission of the aggressor gNB in DL subband in a SBFD carrier to UL reception of the victim gNB in UL subband (different from the DL subband) in the same SBFD carrier. - Co-channel intra-cell/inter-cell UE-UE inter-subband CLI: CLI caused by UL transmission of the aggressor UE in UL subband in a SBFD carrier to DL reception of the victim UE in DL subband (different from the UL subband) in the same cell or neighbouring cell in the same SBFD carrier. - Co-channel inter-cell gNB-gNB intra-subband CLI: CLI caused by DL transmission of the aggressor gNB in a subband in one carrier to UL reception of the victim gNB in the same subband in the same carrier. - Co-channel inter-cell UE-UE intra-subband CLI: CLI caused by UL transmission of the aggressor UE in a subband in one carrier to DL reception of the victim UE in the same subband in the same carrier. - Adjacent-channel gNB-gNB CLI: CLI caused by DL transmission of the aggressor gNB in a legacy TDD carrier to UL reception of the victim gNB in another adjacent SBFD carrier. - Adjacent-channel UE-UE CLI: CLI caused by UL transmission of the aggressor UE in a SBFD carrier to DL reception of the victim UE in another adjacent legacy TDD carrier. Proposal 6: At least for RAN1’s discussion and evaluation for SBFD, consider the following for interference modelling: - Self-interference ratio (SIR): The ratio of the power transmitted by gNB on one frequency unit (e.g., one subband/RB/subcarrier) in a SBFD carrier to the total interference received by the same gNB on a different frequency unit (e.g., another subband/RB/subcarrier) in the same SBFD carrier. - The SIR can be described per subband, per RB, or per subcarrier depending on the granularity of the frequency unit. - For RAN1’s simulation purpose, it is preferred that RAN4 can provide the value or value range of per-subcarrier-SIR, αSIm,n, denoting the ratio of the power transmitted by gNB on subcarrier m to the interference received by the same gNB on a subcarrier n. - Inter-subband interference ratio (ISIR): The ratio of the power transmitted by the aggressor gNB/UE on one frequency unit (e.g., one subband/RB/subcarrier) in a SBFD carrier to the total interference received by the victim gNB/UE on a different frequency unit (e.g., another subband/RB/subcarrier) in the same SBFD carrier. - Note: The ISIR can be described per subband, per RB, or per subcarrier depending on the granularity of the frequency unit. - For RAN1’s simulation purpose, it is preferred that RAN4 can provide the value or value range of per-subcarrier-ISIR, βm,n, denoting the ratio of the power transmitted by the aggressor gNB/UE on subcarrier m to the interference received by the victim gNB/UE on a subcarrier n. - Adjacent-channel interference ratio (ACIR): RAN1 understands the ACIR in TR38.828 is defined as the ratio of the power transmitted by the aggressor gNB/UE on one carrier to the total interference received by the victim gNB/UE on the adjacent carrier, i.e., the ACIR in TR38.828 is described per carrier. - For RAN1’s simulation purpose, it is preferred that RAN4 can provide the value or value range of ACIR with finer granularity, e.g., the per-subcarrier-ACIR, γm,n , denoting the ratio of the power transmitted by the aggressor gNB/UE on subcarrier m in one carrier to the interference received by the victim gNB/UE on another subcarrier n in the adjacent carrier. Proposal 8: Take the values of interference ratios in Table 5 as the assumption for RAN1 calibration for SBFD evaluation. Table 5 RAN1 assumptions on interference ratios for Phase-1 calibration FR1; FR2. Scaling factor; dB(273 * 12) for 100MHz channel bandwidth with 30kHz SCS; dB(264*12) for 200MHz channel bandwidth with 60kHz SCS. gNB’s per-subcarrier-SIR: αSIm,n; Option 1: 138 dB Option 2: 163 dB; Option 1: 117 dB Option 2: 142 dB. gNB-gNB per-subcarrier-ISIR: βgNB-gNBm,n; 78 dB; 57 dB. UE-UE per-subcarrier-ISIR: βUE-UEm,n; 63 dB; 51 dB. gNB-gNB per-subcarrier-ACIR: γgNB-gNBm,n; 78 dB; 57 dB. UE-UE per-subcarrier-ACIR: γUE-UEm,n; 63 dB; 51 dB. |
| Huawei | Observation 4: The cross link interference can be classified as linear and non-linear interference, where downlink signals from own cell or other cells may lead to blocking, and BS-to-BS CLI, UE-to-UE CLI, and BS self CLI have impact on demodulation performance. Proposal 6: The following interferences will impact the system performance and should be studied in Rel-18 NR duplex evolution: - Linear interference - In-band BS-to-BS CLI - In-band UE-to-UE CLI - Blocking caused by DL/UL signal - Non-linear interference - Inter-subband BS-to-BS CLI - Inter-subband UE-to-UE CLI - Inter-subband BS self CLI Proposal 7: The interference models of in-band BS-to-BS CLI and in-band UE-to-UE CLI used in TR 38.802 can be reused for Rel-18 NR duplex evolution. Proposal 8: The following aspects for non-linear interference modeling should be studied in Rel-18 NR duplex evolution: - Strength of non-linear interference (RAN4) - Correlation of non-linear interference between antenna ports - Non-linear interference can be modeled as uncorrelated signals - Non-linear signal on each transmit antenna port can be modeled as Gaussian distribution, i.e., beamforming should not be considered in the modeling of non-linear interference - Channel for non-linear interference - Both large fading and fast fading should be modeled |
| ZTE | Proposal 7: Align understanding on the following terminologies. - Co-channel interference: The interference is from carrier#1 to carrier#2, where the centre frequency of carrier#1 from aggressor and carrier#2 from victim is the same. - Adjacent channel interference: The interference is from carrier#1 to carrier#2, where the centre frequency of carrier#1 from aggressor and carrier#2 from victim is different. - Subband: A subband is a number of continuous RBs within a BWP, including DL subband and UL subband. - Intra-subband interference: The interference is from subband#1 to subband#2, where frequency range of subband#1 from aggressor and subband#2 from victim is the same. - Inter-subband interference: The interference is from subband#1 to subband#2, where frequency range of subband#1 from aggressor and subband#2 from victim is different. - Co-channel intra-subband interference: The interference is from subband#1 of carrier#1 to subband#2 of carrier#2, where frequency range of subband#1 from aggressor and subband#2 from victim is the same and the centre frequency of carrier#1 and carrier#2 is the same. - Co-channel inter-subband interference: The interference is from subband#1 of carrier#1 to subband#2 of carrier#2, where frequency range of subband#1 from aggressor and subband#2 from victim is different and the centre frequency of carrier#1 and carrier#2 is the same. Proposal 8: Consider the following 9 interferences in RAN1 simulation. Among them, gNB self-interference (I1), gNB-gNB co-channel inter-subband interference (I3) and UE-UE co-channel inter-subband interference (I6) need RAN4 input. - gNB self-interference (I1): RAN4 input is needed. - gNB-gNB co-channel intra-subband interference (I2) - gNB-gNB co-channel inter-subband interference (I3): RAN4 input is needed. - gNB-gNB adjacent interference (I4) - UE-UE co-channel intra-subband interference (I5) - UE-UE co-channel inter-subband interference (I6): RAN4 input is needed. - UE-UE adjacent-channel interference (I7) - gNB-UE co-channel intra-subband interference (DL) (I8) - UE-gNB co-chanel intra-subband interference (UL) (I10) Proposal 9: Use the following interference model for subband full duplex simulation. - gNB self-interference per RB. Basic assumption for RAN1 simulation calibration: 90dB and 130dB. - gNB-gNB co-channel intra-subband interference per RB of this intra-subband per aggressor gNB - gNB-gNB co-channel inter-subband interference per RB per inter-subband per aggressor gNB - gNB-gNB adjacent interference per RB per aggressor gNB - UE-UE co-channel intra-subband interference per RB of this intra-subband per aggressor UE - UE-UE co-channel inter-subband interference per RB per inter-subband per aggressor UE - UE-UE adjacent interference per RB per aggressor UE - gNB-UE co-channel intra-subband interference (DL) per RB of this intra-subband per aggressor gNB - UE-gNB co-chanel intra-subband interference (UL) per RB of this intra-subband per aggressor UE |
| CATT | Proposal 4: Self-interference power for SBFD evaluation is modeled according to the following equation: - Pself-interference = BS Tx power + BS antenna gain – X - Value of X is FFS subject to RAN4’s inputs Proposal 5: For SBFD evaluation, prioritize the case that subband configuration across all the cells operating SBFD is the same. Proposal 6: Inter-sector inter-subband CLI power and inter-gNB inter-subband CLI power are modeled according to the following equations respectively: - Pinter-sector inter-SB CLI = aggressor sector Tx power + aggressor sector Tx antenna gain + victim sector Rx antenna gain– Y1 - Pinter-gNB inter-SB CLI = aggressor gNB Tx power + aggressor gNB Tx antenna gain + victim gNB Rx antenna gain– PL – Y2 - Values of Y1 and Y2 are FFS subject to RAN4’s inputs Proposal 7: UE-to-UE inter-subband CLI power is modeled according to the following equation: - Pinter UE inter-SB CLI = aggressor UE Tx power + aggressor UE Tx antenna gain + victim UE Rx antenna gain– PL – Z - Value of Z is FFS subject to RAN4’s inputs |
| vivo | Observation 1: New interference types e.g., self-interference and inter-subband CLI are introduced in full duplex operation, which may have impacts on the system performance. Proposal 10: Modeling of the interference types highlighted by yellow color in Table 3 should be investigated for SBFD evaluation. Table 3 interference types for full duplex operation Index; Aggressor/victim; Channel/subband; Details. ①; UE-to-BS; Co-channel intra-subband or Adjacent-channel; UL-to-UL interference, legacy. ②; BS-to-UE; Co-channel intra-subband or Adjacent-channel; DL-to-DL interference, legacy. ③; BS-to-BS; Co-channel inter-subband; Self-interference. ④; BS-to-BS; Co-channel inter-subband or Adjacent-channel; Inter-cell CLI. ⑤; BS-to-BS; Co-channel intra-subband; Inter-cell CLI. ⑥; UE-to-UE; Co-channel inter-subband; Intra-cell CLI. ⑦; UE-to-UE; Co-channel inter-subband or Adjacent-channel; Inter-cell CLI. ⑧; UE-to-UE; Co-channel intra-subband; Inter-cell CLI. |
| xiaomi | Proposal 9: Self-interference can be modeled by the following formula and the capability on self-interference suppression is up to RAN4. Iself=PTx+ Gantenna-Sup |
| Samsung | Proposal 12: To model the received power of gNB-to-gNB (DL-to-UL) interference at subcarrier k from all aggressor gNBs (Iinter-gNB/inter-subbandk), RAN1 takes the following model as a starting point. Iinter-gNB/inter-subbandk=all aggressor gNBsj∈DL subcarriersPCLI,gNBj→k - where PCLI,gNBj→k=DL Transmission power at subcarrier j of aggressor gNB – effective channel gain between the aggressor gNB and victim gNB – fj→k. - The suppression value fj→k should be asked to RAN4 - RAN1 further to study the simplified model for the suppression level fj→k and its feasibility should be asked to RAN4 Proposal 13: To model the residual self-interference power after self-interference cancelation at subcarrier k (Iselfk), the representative value for Iselfk is considered as additional noise level in addition to the original noise level. - The (worst-case) residual self-interference value should be asked to RAN4 Proposal 14: To model the received power of UE-to-UE (UL-to-DL) interference at subcarrier k from all aggressor Ues (Iinter-UE/inter-subbandk), RAN1 takes the following model as a starting point. Iinter-UE/inter-subbandj→k=all aggressor UEsj∈UL subcarriersPCLI,UEj→k - where PCLI,UEj→k=UL Transmission power at subcarrier j of aggressor UE – effective channel gain between the aggressor UE and victim UE – gj→k. - The suppression value gj→k should be asked to RAN4 - RAN1 further to study the simplified model for the suppression level gj→k and its feasibility should be asked to RAN4 |
| OPPO | Proposal 3: The following interference types should be considered in the evaluation: - gNB->gNB - gNB->UE - UE->UE - UE->gNB Proposal 4: ACIR listed in TR 38.828 could be reused for adjacent carriers, if needed. Proposal 5: Send LS to RAN4 to ask whether and how to model the interference ratio among subbands. |
| Ericsson | Proposal 1: Adopt a net effect model that captures the essential behaviours of a realistic DPD and PA combination with -45 dBc ACLR compliance. This requires input from RAN4. Proposal 2: Adopt a simple crest factor processing model, e.g., hard clipping, that captures the essential behaviours of a BS design to increase transmit power. This requires input from RAN4. Proposal 3: The self-interference channel should be haracte as a set of tapped delay lines directly from TX sub-array ports to RX sub-array ports. Proposal 4: Self-interference channel coefficients should be based on realistic setups supported by real measurements or high-fidelity electromagnetic (EM) evaluations. Figure 10: Illustration of self-interference channel haracter for panels with sub-arrays Proposal 5: The gNB receiver in an SBFD system needs to be carefully studied and modelled with respect to various aspects such as selectivity, linearity etc. Proposal 6: Considering there is no RAN4 requirement on inter-subband selectivity within the channel bandwidth, RAN1/4 needs to study whether the rejection of UL signal in the UL part of an SBFD carrier by a UE receiving DL signal in the DL part is the same as adjacent channel selecitivity requirement. Proposal 7: Send an LS to RAN4 requesting feedback on various radio and antenna modelling aspects that are required for RAN1 to establish evaluation assumptions for both system-level and link-level simulations. Proposal 11: RAN1 to agree on net effect relative PSD interference metrics for system level evaluations taking into account both transmitter and receiver. In general, different values are needed for different interference cases. |
| Nokia | Observation 4: On the evaluation methodology for full duplex on non-overlapping PRBs, the selected scenarios should focus on the impacts of the full-duplex-specific sources of interference, namely: - (intra-cell) gNB self-interference from in-band emissions - (inter-cell) gNB-to-gNB interference from in-band emissions - (intra-cell and inter-cell) UE-to-UE interference from in-band emissions - (inter-cell) gNB-to-gNB interference from adjacent-channel emissions - (inter-cell) UE-to-UE interference from adjacent-channel emissions Proposal 3: For evaluating the performance and identifying enhancements for SBFD, focus on co-channel scenarios with same UL-DL PRB partitioning in all the cells and on adjacent channel scenario with legacy TDD. Proposal 5: For the modelling of in-band emissions (IBE) for SBFD evaluations, adopt the UE IBE model as specified 3GPP TS 38.101-1, Section 6.4.2.3 including General, IQ Image and Carrier leakage components. Proposal 6: RAN1 and RAN4 to discuss and agree on models for gNB and UE ICS and gNB IBE. Proposal 7: For the modelling of inter-subband co-channel interference in SBFD: - IBE and ICS models are used to determine an inter-subband interference ratio (ISIR) in dB which determines the interference generated by an allocated (i.e. ‘aggressor’) PRB or resource element (RE) to a non-allocated (‘victim’) PRB/RE, relative to the normalized aggressor’s transmit power. - For gNB self-interference calculations, assume a self-interference suppression value α (in dB) expressing the gNB’s capability to suppress inter-subband interference (determined from ISIR) using a combination of Tx-Rx isolation and analog or digital cancellation. - Simulation can be run with different values of α, e.g. from 50 dB (conservative) to 110 dB (highly-optimistic). - For gNB-gNB or UE-UE links, the resulting co-channel inter-subband interference is only a function of the aggressor’s transmit power, ISIR and coupling gain between the aggressor and victim gNB or UE. No additional interference suppression is considered. |
| LG | Proposal 3: Apply Adjacent Subband Interference Ratio (ASIR) from BS to BS and from UE to UE for evaluation of subband non-overlapped full duplex. Discuss on exact values of ASIR for evaluation. Proposal 4: Apply residual Self-Interference (SI) after self-interference cancelation or mitigation for evaluation of subband non-overlapped full duplex. Discuss on definition and exact values of residual SI. |
| MediaTek | Observation 1: A model that is similar to in-channel selectivity need to be considered for the impact of inter-UE CLI on reception performance in the scenario when UE channel BWs/BWPs overlap across sub-bands. This model is within RAN4 expertise. Observation 2: The characteristics/requirements relating to Tx & Rx channel selectivity are within RAN4 expertise. Observation 3: Different RAN4 RF requirements exist for the different potential UE configuration approaches, and the different requirements may have different impacts on the ability of legacy UEs to be protected from interference in a SBFD operating scenario. Observation 4: RAN1 making “assumptions” on how these requirements should be characterized in the context of SBFD does not seem appropriate. Proposal 2: RAN1 to liaise RAN4 to request feedback on the characterization of the relevant UE Tx/Rx requirements for the different UE configuration scenarios: 1) UE channel BWs/BWPs overlap across sub-bands, 2) UE channel BW/BWP confined within a sub-band. Proposal 9: For DTDD and SBFD evaluations, RAN1 considers the following interference types: - At the gNB: Inter-gNB, adjacent channel CLI, Inter-gNB co-channel CLI, Inter-gNB intra-subband CLI, Inter-gNB inter-subband CLI, and Self-interference. - At the UE: Inter-UE adjacent channel CLI, Inter-UE co-channel CLI, Inter-UE intra-subband CLI, Inter-UE inter-subband CLI. |
| CEWiT | Proposal 7: Define the SI model to be used for SBFD evaluations. |
| Qualcomm | Self-interference modelling Observation 9: The amount of residual self-interference depends on gNB spatial isolation, subband frequency isolation, digital interference cancellation and beamform nulling/isolation. Proposal 15: The residual self-interference at gNB receiver is modelled as fixed value across the UL subband and is given by Pself-interfence = TxPowerDL-α - where α is the overall self-inference reduction capability of the gNB by means of spatial isolation, subband frequency isolation, digital interference cancellation and beamform nulling/isolation. - FFS: Frequency selective residual self-interference modelling. Clutter reflection Observation 10: There is no 3GPP model for clutter modelling. Observation 11: Exact clutter modelling is complicated and may drain RAN1 time and efforts. Observation 12: A statistical clutter model based on statistics of clutter strength and AoA is simple model. Proposal 16: At least for FR2, for subband full duplex deployment scenario, simplified statistical clutter modelling can be considered based on statistics of cluster power and AoA. Proposal 17: For subband full duplex deployment scenario, simplified statistical clutter modelling shall be intra-serving-gNB model and shall have no impact on other gNBs and Ues in the network. Cross-link interference modelling (inter-gNB and inter-UE) Observation 13: RAN1 needs to agree on the inter-subband cross-link interference model for subband full duplex evaluation. Proposal 18: Inter-gNB inter-subband CLI can be modelled using two steps - Step 1: gNB-to-gNB channel model based on TR 38.802 / 38.901 as gNB-to-UE channel and height adjustment. - Step 2: inter-SB leakage based on the frequency isolation between DL subband and UL subband. Proposal 19: Inter-UE inter-subband CLI can be modelled using two steps - Step 1: UE-to-UE channel model based on TR 38.802 / 38.901. - Step 2: inter-SB leakage based on the frequency isolation between UL subband and DL subband. |

- LS to RAN4

| Company | Proposals |
| --- | --- |
| CMCC | Proposal 7: Include the following in the LS to RAN4: - RAN1 would like to ask RAN4 about the values or value ranges of the following interference ratios used for SBFD evaluation. - gNB’s self-interference ratio (SIR) - In RAN1’s understanding, SIR is defined as the ratio of the power transmitted by gNB on one frequency unit (e.g., one subband/RB/subcarrier) in a SBFD carrier to the total interference received by the same gNB on a different frequency unit (e.g., another subband/RB/subcarrier) in the same SBFD carrier. - Note: It is up to RAN4 to describe the SIR per subband, per RB, or per subcarrier. For RAN1’s simulation purpose, it is preferred that RAN4 can provide the value or value range of per-subcarrier-SIR. - Inter-subband interference ratio (ISIR) including gNB-gNB ISIR and UE-UE ISIR - In RAN1’s understanding, ISIR is defined as the ratio of the power transmitted by the aggressor gNB/UE on one frequency unit (e.g., one subband/RB/subcarrier) in a SBFD carrier to the total interference received by the victim gNB/UE on a different frequency unit (e.g., another subband/RB/subcarrier) in the same SBFD carrier. - Note: It is up to RAN4 to describe the ISIR per subband, per RB, or per subcarrier. For RAN1’s simulation purpose, it is preferred that RAN4 can provide the value or value range of per-subcarrier-ISIR. - Adjacent-channel interference ratio (ACIR) including gNB-gNB ACIR and UE-UE ACIR - RAN1 understands the ACIR in TR38.828 is the ratio of the power transmitted by the aggressor gNB/UE on one carrier to the total interference received by the victim gNB/UE on the adjacent carrier, i.e., the ACIR in TR38.828 is described per carrier. - For RAN1’s simulation purpose, it is preferred that RAN4 can provide the value or value range of ACIR with finer granularity, e.g., the per-subcarrier-ACIR defined as the ratio of the power transmitted by the aggressor gNB/UE on one subcarrier in one carrier to the interference received by the victim gNB/UE on another subcarrier in the adjacent carrier. - If values or value ranges of SIR/ISIR/ACIR provided by RAN4 are defined with a coarser granularity, e.g., per carrier/subband/RB, it is preferred that a method can be provided by RAN4 to derive the SIR/ISIR/ACIR values with finer granularity from the SIR/ISIR/ACIR values with coarser granularity. - Note: At least for RAN1’s simulation purpose, the typical value of the bandwidth of the UL subband in subband non-overlapping full duplex is tens of MHz, e.g., 20MHz, 40MHz, 60MHz, etc… - Note: It’s up to RAN4 to consider the antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression. From RAN1 perspective, it is preferred that the SIR value under the assumption of separate-Tx/Rx antenna architecture can be provided by RAN4 as well as the corresponding separate-Tx/Rx antenna configuration including the placement of the Tx antennas and Rx antennas in the horizontal/vertical domain. |
| ZTE | Proposal 6: To progress RAN1 study on duplex smoothly, - RAN1 starts identifying information that requires RAN4 input and sends LS to RAN4 as soon as possible. - RAN1 firstly calibrates geometry based on some simplified interference model defined by RAN1 and secondly calibrates geometry based on RAN4’s input once it is available. |
| Xiaomi | Proposal 8: Send an LS to RAN4 to check the views on the following two issue: - How to model the inter-subband interference from a DL subband to a UL subband? - How to model the inter-subband interference from a UL subband to a UL subband? |
| OPPO | Proposal 5: Send LS to RAN4 to ask whether and how to model the interference ratio among subbands. |
| Ericsson | Proposal 1: Send an LS to RAN4 requesting feedback on various radio and antenna modelling aspects that are required for RAN1 to establish evaluation assumptions for both system-level and link-level simulations. Proposal 2: In the LS to RAN request feedback on the following gNB and UE aspects: - gNB - Realistic net effect model that captures the essential behavior of a realistic DPD and PA combination with -45 dBc ACLR compliance. - Simple model of creset factor reduction (CFR) processing, e.g., hard clipping with filtering, that captures the essential behaviors of a practical BS designed for PA efficiency and ACLR compliance. - Realistic models on UL receiver selectivity, dynamic range and nonlinearity behaviors - Realistic antenna aspects including - Self-interference isolation levels between each Tx sub-array port and each Rx sub-array port including - Frequency dependence - Any dependence on beam direction - Any impact on Tx or Rx beam patterns due to the antenna isolation design - Isolation levels between sectors within the same site for the same operator - Isolation levels between co-sited multi-sector antennas for different operators - Any other important modelling components characterizing typical gNB TX/RX functions. - UE - Realistic model for determining the inter-subband emission levels within the channel bandwidth and the emission levels in adjacent channels - Considering there is no RAN4 requirement on inter-subband selectivity within the channel bandwidth, feedback is needed on what rejection level of UL signal in the UL part of an SBFD carrier by a UE receiving DL signal in the DL part can be used |
| MediaTek | Proposal 1: RAN1 to work on the assumption that full SLS will be performed by RAN1 for SBFD, but dependent on information from RAN4 to haracterize CLI modelling at UE Tx and Rx. Confirm the way of working between RAN1 and RAN4 with a SID revision at RAN#96. Proposal 3: Include the above questions in the liaison to RAN4. - Please can RAN4 provide input to enable RAN1 to characterize CLI (Cross-Link Interference) between UEs in system-level simulations for the following UE operating scenarios for FR1 and FR2: - UE Channel BW and BWP in DL or UL is not confined within a sub-band for SBFD operation. - UE channel BW and BWP in DL and UL are confined within a sub-band for SBFD operation. Note: RAN4 should take into account that interference may vary across slots, and that Tx and Rx between UEs in different sub-bands may not be fully time-aligned. - Does RAN4 see any RAN4 specific pros and cons with either configuration a or b? |
| Intel | Proposal 6: RAN1 should consider appropriate NOFD configurations and modelling options for self- and cross-link interference for the evaluations for NOFD. - Send an LS to RAN4 during RAN1 #109-e to receive inputs from RAN4 on self- and cross-link interference for NOFD operation. |

## 4.2 Summary

Several companies [CMCC, Huawei, ZTE, CATT, vivo, Samsung, OPPO, Nokia, MediaTek, Qualcomm, etc.] raise that SBFD suffer from multiple types of interference including linear interferences and non-linear interferences, e.g.,

- gNB self-interference (SI): Non-linear interference

- gNB-UE co-channel intra-subband interference

- UE-gNB co-channel intra-subband interference

- (inter-cell) gNB-gNB co-channel intra-subband CLI

- (inter-cell) UE-UE co-channel intra-subband CLI

- (inter-cell) gNB-gNB co-channel inter-subband CLI: Non-linear interference

- (intra-cell/inter-cell) UE-UE co-channel inter-subband CLI: Non-linear interference

- gNB-gNB Adjacent-channel CLI: Non-linear interference

- UE-UE Adjacent-channel CLI: Non-linear interference

Moderator thinks it is important for companies to have a common understanding on the terminologies used to describe these interferences firstly, so moderator suggests Initial proposal 3-1 and Initial proposal 3-2.

Regarding these interferences, several companies propose different or similar modelling methods for gNB self-interference, gNB-gNB or UE-UE co-channel inter-subband CLI, and gNB-gNB or UE-UE adjacent-channel CLI. Some companies propose to define the following interference ratios which need RAN4’s input for interference modelling:

- Self-interference ratio (SIR): It is used to represent the overall self-interference suppression capability of gNB by means of spatial isolation, subband frequency isolation, digital interference cancellation and beamform nulling/isolation, etc. The SIR, denoted as αSIm,n,  can be defined as the ratio of the power transmitted by gNB on a frequency unit m (e.g., subband/RB/subcarrier m) in a SBFD carrier to the residual self-interference received by the same gNB on a different frequency unit n (e.g., another subband/RB/subcarrier n) in the same SBFD carrier.

- Inter-subband interference ratio (ISIR): denoted as βgNB-gNBm,n or βUE-UEm,n, it can be defined as the ratio of the power transmitted by the aggressor gNB/UE on a frequency unit m (e.g., subband/RB/subcarrier m) in a SBFD carrier to the interference received by the victim gNB/UE on a different frequency unit n (e.g., another subband/RB/subcarrier n) in the same SBFD carrier.

- Adjacent-channel interference ratio (ACIR): The ACIR in TR38.828 is defined as the ratio of the power transmitted by the aggressor gNB/UE on one carrier to the total interference received by the victim gNB/UE on the adjacent carrier, i.e., the ACIR in TR38.828 is described per carrier. For RAN1’s simulation purpose, it is preferred that RAN4 can provide the value or value range of ACIR with finer granularity, e.g., the per-subcarrier-ACIR, γgNB-gNBm,n or γUE-UEm,n, denoting the ratio of the power transmitted by the aggressor gNB/UE on subcarrier m in one carrier to the interference received by the victim gNB/UE on another subcarrier n in the adjacent carrier.

          - gNB per sub-carrier-SIR αSIm,n

(b) gNB-gNB per-subcarrier-ISIR βgNB-gNBm,n 	 (c) UE-UE per-subcarrier-ISIR βUE-UEm,n

(d) gNB-gNB per-subcarrier-ACIR γgNB-gNBm,n 			 I UE-UE per-subcarrier-ACIR γUE-UEm,n

Regarding interference modelling, moderator suggests Initial proposal 3-3, Initial proposal 3-4 and Initial proposal 3-5.

Several companies [Ericsson, CMCC, ZTE, xiaomi, OPPO, Intel, etc.] suggest to send an LS to RAN4 regarding the interference modelling as soon as possible.

CMCC suggests to include the following in the LS to RAN4:

- RAN1 would like to ask RAN4 about the values or value ranges of the following interference ratios used for SBFD evaluation.

- gNB’s self-interference ratio (SIR)

    - In RAN1’s understanding, SIR is defined as the ratio of the power transmitted by gNB on one frequency unit (e.g., one subband/RB/subcarrier) in a SBFD carrier to the total interference received by the same gNB on a different frequency unit (e.g., another subband/RB/subcarrier) in the same SBFD carrier.

    - Note: It is up to RAN4 to describe the SIR per subband, per RB, or per subcarrier. For RAN1’s simulation purpose, it is preferred that RAN4 can provide the value or value range of per-subcarrier-SIR.

- Inter-subband interference ratio (ISIR) including gNB-gNB ISIR and UE-UE ISIR

    - In RAN1’s understanding, ISIR is defined as the ratio of the power transmitted by the aggressor gNB/UE on one frequency unit (e.g., one subband/RB/subcarrier) in a SBFD carrier to the total interference received by the victim gNB/UE on a different frequency unit (e.g., another subband/RB/subcarrier) in the same SBFD carrier.

    - Note: It is up to RAN4 to describe the ISIR per subband, per RB, or per subcarrier. For RAN1’s simulation purpose, it is preferred that RAN4 can provide the value or value range of per-subcarrier-ISIR.

- Adjacent-channel interference ratio (ACIR) including gNB-gNB ACIR and UE-UE ACIR

    - RAN1 understands the ACIR in TR38.828 is the ratio of the power transmitted by the aggressor gNB/UE on one carrier to the total interference received by the victim gNB/UE on the adjacent carrier, i.e., the ACIR in TR38.828 is described per carrier.

    - For RAN1’s simulation purpose, it is preferred that RAN4 can provide the value or value range of ACIR with finer granularity, e.g., the per-subcarrier-ACIR defined as the ratio of the power transmitted by the aggressor gNB/UE on one subcarrier in one carrier to the interference received by the victim gNB/UE on another subcarrier in the adjacent carrier.

- If values or value ranges of SIR/ISIR/ACIR provided by RAN4 are defined with a coarser granularity, e.g., per carrier/subband/RB, it is preferred that a method can be provided by RAN4 to derive the SIR/ISIR/ACIR values with finer granularity from the SIR/ISIR/ACIR values with coarser granularity.

- Note: At least for RAN1’s simulation purpose, the typical value of the bandwidth of the UL subband in subband non-overlapping full duplex is tens of MHz, e.g., 20MHz, 40MHz, 60MHz, etc…

- Note: It’s up to RAN4 to consider the antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression. From RAN1 perspective, it is preferred that the SIR value under the assumption of separate-Tx/Rx antenna architecture can be provided by RAN4 as well as the corresponding separate-Tx/Rx antenna configuration including the placement of the Tx antennas and Rx antennas in the horizontal/vertical domain.

Ericsson suggests to send an LS to RAN4 requesting feedback on various radio and antenna modelling aspects that are required for RAN1 to establish evaluation assumptions for both system-level and link-level simulations. In the LS to RAN4 request feedback on the following gNB and UE aspects:

- gNB

  - Realistic net effect model that captures the essential behavior of a realistic DPD and PA combination with -45 dBc ACLR compliance.

  - Simple model of creset factor reduction (CFR) processing, e.g., hard clipping with filtering, that captures the essential behaviors of a practical BS designed for PA efficiency and ACLR compliance.

  - Realistic models on UL receiver selectivity, dynamic range and nonlinearity behaviors

  - Realistic antenna aspects including

    - Self-interference isolation levels between each Tx sub-array port and each Rx sub-array port including

      - Frequency dependence

      - Any dependence on beam direction

    - Any impact on Tx or Rx beam patterns due to the antenna isolation design

    - Isolation levels between sectors within the same site for the same operator

    - Isolation levels between co-sited multi-sector antennas for different operators

  - Any other important modelling components characterizing typical gNB TX/RX functions.

- UE

  - Realistic model for determining the inter-subband emission levels within the channel bandwidth and the emission levels in adjacent channels

  - Considering there is no RAN4 requirement on inter-subband selectivity within the channel bandwidth, feedback is needed on what rejection level of UL signal in the UL part of an SBFD carrier by a UE receiving DL signal in the DL part can be used

MediaTek suggests to include the following questions in LS to RAN4:

- Please can RAN4 provide input to enable RAN1 to characterize CLI (Cross-Link Interference) between UEs in system-level simulations for the following UE operating scenarios for FR1 and FR2:

  - UE Channel BW and BWP in DL or UL is not confined within a sub-band for SBFD operation.

  - UE channel BW and BWP in DL and UL are confined within a sub-band for SBFD operation.

  - Note: RAN4 should take into account that interference may vary across slots, and that Tx and Rx between UEs in different sub-bands may not be fully time-aligned.

- Does RAN4 see any RAN4 specific pros and cons with either configuration

Regarding LS to RAN4, moderator suggest Initial proposal 3-6 and Initial question 3-7.

## 4.3 1st Round Proposals (closed)

Based on the inputs from the contributions submitted to RAN1#109-e, some initials proposals are made as follows. Companies are encouraged to provide views on these proposals.

### Initial proposal 3-1:

For discussion purpose, consider the following as RAN1’s common understanding:

- Co-channel interference: The interference is from the aggressor in carrier#1 to the victim in carrier#2, where the carrier#1 and carrier#2 are the same carrier.

  - Co-channel intra-subband interference: The interference is caused by transmission of the aggressor in subband#1 in a carrier to reception of the victim in subband#2 in the same carrier, where the frequency range of subband#1 and subband#2 are the same.

  - Co-channel inter-subband interference: The interference is caused by transmission of the aggressor in subband#1 in a carrier to reception of the victim in subband#2 in the same carrier, where the frequency range of subband#1 and subband#2 are different.

- Adjacent channel interference: The interference is from the aggressor in carrier#1 to the victim in carrier#2, where the carrier#1 and carrier#2 are adjacent carriers.

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| QC | It is bit confusing to use carrier#1 and carrier#2 and then mention they are same carrier. Similarly for the subband #1 and subband#2. Also, the use of ‘frequency range’ maybe miss-understood as FR1/FR2. Suggest the following: For discussion purpose, consider the following as RAN1’s common understanding: - Co-channel interference: The interference is from the aggressor in carrier#1 to the victim in carrier#2, where the carrier#1 and carrier#2 are the same carrier. - Co-channel intra-subband interference: The interference is caused by transmission of the aggressor in a subband #1 in a carrier to reception of the victim in the same subband#2 in the same carrier, where the frequency range of subband#1 and subband#2 are the same. - Co-channel inter-subband interference: The interference is caused by transmission of the aggressor in subband#1 in a carrier to reception of the victim in subband#2 in the same carrier, where the frequency range of subband#1 and subband#2 are different. - Adjacent channel interference: The interference is from the aggressor in carrier#1 to the victim in carrier#2, where the carrier#1 and carrier#2 are adjacent carriers.. |
| NTT DOCOMO | We support. |
| New H3C | Support |
| CATT | For partially overlapping subbands, e.g. in case subband configurations are not aligned, what interference type would be according to the proposal? Our understanding is that the interference in overlapping RBs is also intra-subband interference and the interference in the non-overlapping RBs is inter-subband interference. |
| Spreadtrum | We are fine with this proposal. |
| ZTE | Our understanding is the same proposal 3-1. We are open to consider other descriptions as long as companies have the same understanding on these terminologies. |
| NEC | Agree with the understanding |
| vivo | We are generally fine with the understanding in proposal 3-1. For co-channel case, we have the similar concern with QC. The different carrier indexes but the same carrier frequency could be misleading. It is better to say the interference is within the same carrier. For Co-channel interference, it is not clear to us what is the definition of frequency range of subband. Does it mean for intra-subband interference case, the subband #1 and subband #2 are overlapping in frequency, while for inter-subband interference case the subband #1 and subband #2 are non-overlapping in frequency? |
| Sharp | We are OK with the original FL proposal or Qualcomm’s proposal. |
| Sony | I must admit it was confusing (similar to QC’s comment) reading carrier#1 and carrier#2 then say they are the same carrier. I prefer QC’s modifications. |
| MediaTek | The description is a bit confusing. - The wording of “Co-channel interference” seems to imply the interferences are within the same carrier. However, it could cause confusion when it is used to describe inter-subband interference because the interference is coming from adjacent subband rather than same subband (co-channel). - It says “carrier#1 to the victim in carrier#2”, yet carrier#1 and carrier#2 are the same carrier. We should make the definition of “co-channel” interference clear, i.e., it doesn’t imply the same resources/RBs. Otherwise, we the wording of “co-channel” could be removed as follows: - Co-channel interference: The interference is from the aggressor in carrier#1 to the victim in carrier#2, where the carrier#1 and carrier#2 are the same carrier. - Co-channel intra-subband interference: The interference is caused by transmission of the aggressor in subband#1 in a carrier to reception of the victim in subband#2 in the same carrier, where the frequency range of subband#1 and subband#2 are the same. - Co-channel inter-subband interference: The interference is caused by transmission of the aggressor in subband#1 in a carrier to reception of the victim in subband#2 in the same carrier, where the frequency range of subband#1 and subband#2 are different. - Adjacent channel interference: The interference is from the aggressor in carrier#1 to the victim in carrier#2, where the carrier#1 and carrier#2 are adjacent carriers.. |
| Xiaomi | Support. |
| Huawei, HiSlicon | Support the proposal. |
| Ericsson | We support the proposal. |
| Nokia/NSB | Support the proposal. |
| Intel | We support the proposal. |
| OPPO | Support the proposal |
| InterDigital | Revisions from Qualcomm and MTK are both valid. So, we support a modified proposal applied by merging comments from both Qualcomm and MTK. |
| Samsung | Fine for the proposal, even if it is already the common understanding on RAN1 based on contributions from companies. |
| KDDI | We support the proposal. |
| LG | Support the proposal in principle. We are ok with QC’s version. |
| Moderator | The proposal was updated based on companies’ comments. |

### Initial proposal 3-2:

Consider the following interference types in RAN1 SBFD simulation:

- gNB self-interference (SI): Interference caused by DL transmission in DL subband in a carrier to UL reception in UL subband in the same carrier at the gNB side, where the frequency range of the DL subband and the UL subband are different.

- gNB-UE co-channel intra-subband interference: This is the same as the legacy DL interference type in legacy TDD network with static TDD UL/DL configuration.

- UE-gNB co-channel intra-subband interference: This is the same as the legacy UL interference type in legacy TDD network with static TDD UL/DL configuration.

- (inter-cell) gNB-gNB co-channel intra-subband CLI: CLI caused by DL transmission of the aggressor gNB in a subband in one carrier to UL reception of the victim gNB in the same subband in the same carrier.

- (inter-cell) UE-UE co-channel intra-subband CLI: CLI caused by UL transmission of the aggressor UE in a subband in one carrier to DL reception of the victim UE in the same subband in the same carrier.

- (inter-cell) gNB-gNB co-channel inter-subband CLI: CLI caused by DL transmission of the aggressor gNB in DL subband in a carrier to UL reception of the victim gNB in UL subband in the same carrier, where the frequency range of the DL subband and the UL subband are different.

- (intra-cell/inter-cell) UE-UE co-channel inter-subband CLI: CLI caused by UL transmission of the aggressor UE in UL subband in a carrier to DL reception of the victim UE in DL subband in the same cell or neighboring cell in the same carrier, where the frequency range of the UL subband and the DL subband are different.

- gNB-gNB adjacent-channel CLI: CLI caused by DL transmission of the aggressor gNB in a carrier to UL reception of the victim gNB in another adjacent carrier.

- UE-UE adjacent-channel CLI: CLI caused by UL transmission of the aggressor UE in a carrier to DL reception of the victim UE in another adjacent carrier.

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| QC | We have few comments on the proposal: - The use of ‘frequency range’ may be confusing. Instead, suggest the following wording, “where the DL subband and the UL subband are non-overlapping in frequency”. - Since the deployment scenarios with the same UL/DL subbands are prioritized, then we suggest to only consider SI, (inter-cell) gNB-gNB co-channel inter-subband CLI, and (intra-cell/inter-cell) UE-UE co-channel inter-subband CLI as highest priority in SBFD simulation. - Then, the gNB-gNB adjacent-channel CLI and UE-UE adjacent-channel CLI should be studied as second step for Case 4 coexistence study. - Support to deprioritize (inter-cell) gNB-gNB co-channel and (inter-cell) UE-UE co-channel intra-subband CLI. |
| NTT DOCOMO | We are not quite sure to include intra-subband CLI, since for SBFD, the subband may not overlap. |
| New H3C | Support |
| CATT | Agree with the comment of ‘frequency range’ from QC. In addition, we would like to clarify whether co-site inter-sector CLI is included in (inter-cell) gNB-gNB co-channel inter-subband CLI. |
| Spreadtrum | We think it will be clearer to add potential (same/different) subband configuration of gNBs mentioned in every interference types. And agree with QC to modify “where the frequency range of the DL subband and the UL subband are different” to “where the DL subband and the UL subband are non-overlapping in frequency”. |
| ZTE | We are generally aligned with the FL proposal except for the following one. It seems we need to delete “in the same carrier” in this scenario. - (intra-cell/inter-cell) UE-UE co-channel inter-subband CLI: CLI caused by UL transmission of the aggressor UE in UL subband in a carrier to DL reception of the victim UE in DL subband in the same cell or neighboring cell in the same carrier, where the frequency range of the UL subband and the DL subband are different. |
| NEC | Agree with the listed types |
| vivo | We are generally fine with the interference types definition in proposal 3-2. These interference types can be the starting point and which interference type will be considered is depending on the scenario to be evaluated. We have two additional comments as follows. - We share the same view with QC about the usage of “frequency range”. The suggestion from QC looks good to us. - It seems the following are also inter-cell interference, if so, would be good to clarify. - gNB-UE co-channel intra-subband interference: This is the same as the legacy DL interference type in legacy TDD network with static TDD UL/DL configuration. - UE-gNB co-channel intra-subband interference: This is the same as the legacy UL interference type in legacy TDD network with static TDD UL/DL configuration. |
| Sharp | (inter-cell) gNB-gNB/UE-UE co-channel intra-subband CLI can be considered in dynamic/flexible TDD since they are common issues for SFBD and dynamic/flexible TDD. |
| Sony | Similar comment as DOCOMO on the intra-subband CLI: - gNB-UE co-channel intra-subband interference: This is the same as the legacy DL interference type in legacy TDD network with static TDD UL/DL configuration. - UE-gNB co-channel intra-subband interference: This is the same as the legacy UL interference type in legacy TDD network with static TDD UL/DL configuration. Is this supposed to be in the same cell, if yes, where is the interference? Why do we say this is same as legacy DL/UL interference when legacy does not have subbands? |
| MediaTek | Same view as in “Initial proposal 3-1” reading the use of “co-channel”. We should make the definition of “co-channel” interference clear, i.e., it doesn’t imply the same resources/RB. |
| Xiaomi | Our understanding on the proposal is to achieve common understanding on the potential interference types in the pending simulation. Which interference types are modelled in the simulation depends on the scenarios identified by proposal 1-1. With this understanding, we support this proposal. |
| Huawei, HiSlicon | Support the proposal. |
| Ericsson | We support the proposal 3.2 in principle with the following modifications. It is worthy to note that inter-cell, co-channel intra subband CLI for both gNB-gNB and UE-UE occurs only when the SBFD configurations are different for two cells. Because we want to down-prioritize those deployment cases, we need not consider them in the initial proposals. Leakage between sectors Furthermore, one needs to also consider inter-sector interference caused by DL transmission of the aggressor in DL subband in a carrier over a sector to the UL reception of the victim in UL subband in the same carrier over another sector that is co-sited as shown in the figure below. Therefore, we propose the following modifications to the original proposal. ### Initial proposal 3-2Modified by Ericsson) Consider the following interference types in RAN1 SBFD simulation: - gNB self-interference (SI): Interference caused by DL transmission in DL subband in a carrier to UL reception in UL subband in the same carrier at the gNB side, where the frequency range of the DL subband and the UL subband are non-overlappingdifferent. - gNB-UE co-channel intra-subband interference: This is the same as the legacy DL interference type in legacy TDD network with static TDD UL/DL configuration. - UE-gNB co-channel intra-subband interference: This is the same as the legacy UL interference type in legacy TDD network with static TDD UL/DL configuration. - (inter-cell) gNB-gNB co-channel intra-subband CLI: CLI caused by DL transmission of the aggressor gNB in a subband in one carrier to UL reception of the victim gNB in the same subband in the same carrier. - (inter-cell) UE-UE co-channel intra-subband CLI: CLI caused by UL transmission of the aggressor UE in a subband in one carrier to DL reception of the victim UE in the same subband in the same carrier. - (inter-cell) gNB-gNB co-channel inter-subband CLI: CLI caused by DL transmission of the aggressor gNB in DL subband in a carrier to UL reception of the victim gNB in UL subband in the same carrier, where the frequency range of the DL subband and the UL subband are different. - This includes co-channel inter-subband CLI between gNBs in different sectors of the same site i.e., inter-sector interference. - (intra-cell/inter-cell) UE-UE co-channel inter-subband CLI: CLI caused by UL transmission of the aggressor UE in UL subband in a carrier to DL reception of the victim UE in DL subband in the same cell or neighboring cell in the same carrier, where the frequency range of the UL subband and the DL subband are different. - gNB-gNB adjacent-channel CLI: CLI caused by DL transmission of the aggressor gNB in a carrier to UL reception of the victim gNB in another adjacent carrier that may be co-sited. - This includes adjacent-channel CLI between gNBs in the same and different sectors of the same site i.e, intra and inter-sector interference. - UE-UE adjacent-channel CLI: CLI caused by UL transmission of the aggressor UE in a carrier to DL reception of the victim UE in another adjacent carrier. |
| Nokia/NSB | Fine with the proposal. |
| Intel | For different scenarios, the set of interference types may be different. Since 4 deployments are provided in proposal 1-1, maybe it is better to also list the set of interference types for each scenario, respectively. |
| OPPO | Ok with the proposal. |
| InterDigital | Agree with Spreadtrum to add potential (same/different) subband configuration of gNBs mentioned in every interference types. Also, agree with Qualcomm to avoid using ‘frequency range’. |
| Samsung | Fine the proposal with the understanding that some of interferences are not used according to deployment scenarios. For example, (inter-cell) gNB-gNB co-channel intra-subband CLI and (inter-cell) UE-UE co-channel intra-subband CLI are not used in the same UL subband configuration across gNBs (Deployment Case 1). |
| KDDI | Agree with the direction provided by Intel. Some types of interferences are not used according to deployment scenarios. |
| LG | Ok with the proposal. |
| Moderator | @vivo, Sony, regarding the gNB-UE co-channel intra-subband interference and UE-gNB co-channel intra-subband interference, they are inter-cell interference if we do not consider MU-MIMO, and if we consider MU-MIMO, they could also be from the same cell. @Sony, ‘intra-subband’ here basically means the interference is from the aggressor in the same frequency resources, this is the legacy interference we usually modelled in SLS, and if you have better wording, we can also consider it. @Spreadtrum, Intel, Interdigital, I add a note to clarify that some of the interferences may not be used according to the deployment scenarios. I think companies understand that gNB-gNB co-channel intra-subband CLI and UE-UE co-channel intra-subband CLI do not need to be considered when the SBFD subband configurations are the same across gNBs. @DoCoMo, Ericsson, the prioritization of the deployment scenarios can be discussed separately. |

### Initial proposal 3-3:

For discussion of gNB self-interference modelling, consider introducing self-interference ratio (SIR) to represent the overall self-interference suppression capability of gNB by means of spatial isolation, subband frequency isolation, digital interference cancellation and beamform nulling/isolation, etc. The SIR, denoted as αSIm,n,  can be defined as the ratio of the power transmitted by gNB on a frequency unit m (e.g., subband/RB/subcarrier m) in a SBFD carrier to the residual self-interference received by the same gNB on a different frequency unit n (e.g., another subband/RB/subcarrier n) in the same SBFD carrier.

- The value or value range of SIR should be asked to RAN4. The SIR can be described per subband, per RB, or per subcarrier depending on the granularity of the frequency unit. For RAN1’s simulation purpose, it is preferred that RAN4 can provide the value or value range of per-subcarrier-SIR.

- FFS: details of gNB self-interference modelling, e.g., the gNB self-interference at UL subcarrier n can be modelled as

  - ISIn=m≠nm∈DL subbandISIm,n, wherein,

    - m is the DL subcarrier index.

    - dBISIm,n=Ptxm-dBαSIm,n

    - Ptxm is gNB’s DL transmission power at subcarrier m (in dBm).

    - αSIm,n is the per-subcarrier-SIR.

  - The feasibility of the gNB self-interference modelling should be asked to RAN4

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| QC | We have few comments on the proposal: - The description of self-interference should include the spatial description in addition to the frequency description. The SIR represents the ratio of the total transmit power by gNB across all transmit chains to the received power on single receive chain. - For FR2, the spatial isolation could depend on the Tx and Rx beam-pair especially for the clutter reflections. - There is no single value for the SIR as it depends on gNB implementation and the knobs it deploys to handle the SI. Also, this value range is different for FR1 and FR2. Preferred value is per-RB for SLS evaluation, not per RE - In addition to the direct leakage, there could be reflection from nearby reflectors (clutter). Support to add an FFS to study clutter impact Suggest the following: For discussion of gNB self-interference modelling, consider introducing self-interference ratio (SIR) to represent the overall self-interference suppression capability of gNB by means of spatial isolation, subband frequency isolation, digital interference cancellation and beamform nulling/isolation, etc. The SIR, denoted as αSIm,n, can be defined as the ratio of the total power transmitted by gNB across all transmit chains on a frequency unit m (e.g., subband/RB/subcarrier m) in a SBFD carrier to the residual self-interference received by the same gNB on a single receiver chain on a different frequency unit n (e.g., another subband/RB/subcarrier n) in the same SBFD carrier. - The value or value range of SIR should be asked to RAN4 for each frequency range. The SIR can be described per subband, per RB, or per subcarrier depending on the granularity of the frequency unit. For RAN1’s simulation purpose, it is preferred that RAN4 can provide the value or value range of per-RB-SIR. - FFS: details of gNB self-interference modelling, e.g., the gNB self-interference at UL subcarrier n can be modelled as - ISIn,rxi=j=1NTxm≠nm∈DL subbandISIm,n,txj, wherein, - m is the DL subcarrier index. - dBISIm,n=Ptxm-dBαSIm,n - Ptxm is gNB’s total DL transmission power at subcarrier m (in dBm). - αSIm,n is the per-RB-SIR. - ISIn,rxi is gNBs self-interference received power at receiver chain i at subcarrier n (in dBm). - FFS: the dependence of αSIm,non Tx/Rx beam-pair for FR2 especially for clutter echo. - FFS: consider a statistical clutter model based on statistics of clutter strength and AoA. - The feasibility of the gNB self-interference modelling should be asked to RAN4. |
| NTT DOCOMO | We support. |
| New H3C | Support |
| CATT | The granularity of SIR may be decided by RAN4. In addition, we think the Tx/Rx antenna gain should also be considered for self-interference modeling. |
| ZTE | From our perspective, per sub-carrier granularity is too small. We would prefer some per RB or per subband level model. However, this is anyway up to RAN4. If RAN1 can not reach consensus on this, we can just delete the following sentence. For RAN1’s simulation purpose, it is preferred that RAN4 can provide the value or value range of per-subcarrier-SIR. Besides, for the self-interference, we think that element gain should also be considered in the calculation. We propose to add element gain. |
| NEC | Agree that SIR range should be provided by RAN4. Also, assumptions (i.e. self-interference mitigation techniques) used by RAN4 for modelling of SIR should be asked which is expected to be useful for further study. |
| Vivo | The revisions from QC make sense. We have two comments. - We need to highlight to RAN4 that the key assumption that requires RAN4 input is αSIm,n and ask its dependency with the frequency distance between DL RB m and UL RB n. Besides, whether the self-interference strength has dependency with the DL subband size, the number of RBs used for DL transmission also need to be provided by RAN4. - When determining the self-interference, the gNB’s DL transmission power is considered. To our understanding, for SBFD the total bandwidth of DL is smaller than that in a full DL slot. It should be clarified that for SBFD whether the PSD of DL is kept the same, or the DL power is scaled depending the actual DL bandwidth. |
| Sharp | We have similar view with vivo that we should ask αSIm,n to RAN4. |
| Sony | Support |
| MediaTek | We generally fine with the proposal apart from the FFS. Given that the value ranges and granularity are expected to be provided by RAN4, all what RAN1 need to provide to RAN4 is the possible configurations, and RAN4 should be able to provide the model for the self-interference. Also, we agree on with QC point regarding mention “reflection from nearby reflectors” as a note to be considered by RAN4. |
| Xiaomi | We support FL’s proposal in principle. As mentioned in the main bullet, a frequency unit m can be subband/RB/subcarrier, it seems the explanation of m for the formula should be updated accordingly. - m is the RB index or subband index or DL subcarrier index. |
| Huawei, HiSilicon | Fine with the general direction. |
| Ericsson | We support the intention of the proposal 3-3. However, there are a few things to consider. - The acronym SIR is a bit misleading because it is the ratio of signal power in one subband to interference power on another subband, whereas SIR is usually signal to interference ratio on the same resources. - The ratio defined assumes that the DL signal at the receiver does not saturate the receiver or cause the AGC to change the receiver noise floor. - It is not clear to us to what extent would the ratio be dependent on the TX/RX beam directions. - Digital interference cancellation for self-interference is a wide area where we need to discuss various factors for cancellation in the digital domain. It seems to us that the intention here is to indicate only the net effect DPD +PA suppression and isolation model for self-interference in the UL subband. Therefore, we remove that aspect from the discussions here. However, it needs to be discussed separately. - This level of modelling is appropriate for system level simulations, but more detailed modelling needs to be considered for link level simulations. Therefore, we propose the following modifications- ### Initial proposal 3-3: For discussion of gNB self-interference modelling for the purposes of system level simulations (more detailed model needed for link level simulations), consider introducing Ratio of self-interference ratio (RSIR) to represent the overall self-interference suppression capability of gNB by means of spatial isolation, subband frequency isolation, digital interference cancellation and beamform nulling/isolation, etc. The RSISIR, denoted as αSIm,n, can be defined as the ratio of the power transmitted by gNB on a frequency unit m (e.g., subband/RB/subcarrier m) in a SBFD carrier to the residual self-interference received by the same gNB on a different frequency unit n (e.g., another subband/RB/subcarrier n) in the same SBFD carrier. - The value or value range of RSISIR should be asked to RAN4. The RSISIR can be described per subband, per RB, or per subcarrier depending on the granularity of the frequency unit. For RAN1’s simulation purpose, it is preferred that RAN4 can provide the value or value range of per-subcarrier- RSISIR. - FFS: details of gNB self-interference modelling, e.g., the gNB self-interference at UL subcarrier n can be modelled as - ISIn=m≠nm∈DL subbandISIm,n, wherein, - m is the DL subcarrier index. - dBISIm,n=Ptxm-dBαSIm,n - Ptxm is gNB’s DL transmission power at subcarrier m (in dBm). - αSIm,n is the per-subcarrier-RSI. - The feasibility of the gNB self-interference modelling including factors such as blocking, AGC, TX/RX beamforming etc. should be asked to RAN4 |
| Nokia/NSB | Overall support. On the FFS, we would like to raise our preference for simple models. Current FFS proposal from the moderator suggests to imply that the observed interference on UL RBs/SCs will depend on the amount of allocated DL RBs. This is not fully aligned with our views and may require more discussions. Our preferred model is: ISI[dBm]=Ptx[dBm]-αSI[dB]-10 log10BWDL/BWUL , where - ISI is the total residual interference across allocated UL subcarriers (in dBm) - Ptx is the total transmit power across allocated DL subcarriers (in dBm) - αSI is the self-interference suppression ratio (i.e. ratio of the power transmitted on a given frequency unit to the residual self-interference on a different frequency unit) - BWDL and BWUL are the DL and UL allocation bandwidths, respectively. Note that in case all DL subcarriers have equal transmit power per subcarrier, the model reduces to: ISI(n)[dBm]=Ptxm[dBm]-αSI[dB] , where - ISI(n) is the residual interference on UL subcarrier n (in dBm) - Ptxm is the transmit power per DL subcarrier (in dBm) αSI is the self-interference suppression ratio (i.e. ratio of the power transmitted on a given frequency unit to the residual self-interference on a different frequency unit). We can ask for RAN4 feedback are good, also we can consider potential enhancement with taking into account non-flat frequency response |
| Intel | We also think RB-level SIR value is more feasible for SLS evaluation. The accurate value for SIR is to be provided by RAN4. Before RAN4’s feedback, RAN1 can decide one or two values to be used for initial calibration first. |
| OPPO | The location of Tx/Tx antenna may need to be considered. |
| InterDigital | Support Qualcomm’s revision. The granularity of per-subcarrier is too small, and per-RB should be sufficient. Ericsson’s modification on RSI and removing the digital interference cancelation is fine to us. |
| Samsung | Few comments : - The SIC capability at gNB depends on gNB types. For example, gNBs in Macro layer require much powerful SIC capability due to higher BS transmission power and more TX antennas elements. So, multiple values for SIR can be asked to RAN4 according to deployment scenario, BS transmission power, or # of antenna elements and antenna structure. - Typically, a gNB vendor may have SIC design targets to enable SBFD operation for each deployment scenario, BS transmission power, or # of antenna elements and antenna structure. For example, one SIC design target is to allow up to [X] dB residual self-interference level over noise level, where X is design parameter and can be considered as worst-case residual self-interference level. We can directly take into account the value X in the evaluation. - For RAN1 evaluation, among per-subcarrier, per-RB and per-subband, any units can be used. We support to use per-subcarrier unit since it can capture the self-interference as precisely as possible. However, no need to require one unit for RAN4. It is up to RAN4 and RAN1 can implement any unit in evaluation. So, we suggest the following modification: For RAN1’s simulation purpose, it is preferred that RAN4 can provide the value or value range ofRAN1 will assume αSIm,n is per-subcarrier-SIR. |
| LG | Fine with the proposal, and prefer to use RB as frequency unit n. |
| Moderator | @CATT, ZTE, it is clarified that RSI also takes into account the impact of Tx/Rx antenna element gain on self-interference. @Nokia, I think the model provided by you is basically same as the proposal, except that you assume the αSI is constant. I think anyway we will ask RAN4 regarding the RSI. At least for simulation, using αSIm,n is more generic. @all, Regarding per-subcarrier or per-RB level, it can be up to RAN4 to provide. However, in SLS, the SINR is usually calculated per subcarrier (e.g., calculating the channel coefficient, signal power, interference power, noise), even RAN4 finally provide a per-RB-RSI, RAN1 still needs to translate it into per-subcarrier-RSI for simulation purpose. |

### Initial proposal 3-4:

For discussion of gNB-gNB or UE-UE co-channel inter-subband CLI modelling, consider introducing gNB-gNB or UE-UE inter-subband interference ratio (ISIR), denoted as βgNB-gNBm,n or βUE-UEm,n, defined as the ratio of the power transmitted by the aggressor gNB/UE on a frequency unit m (e.g., subband/RB/subcarrier m) in a SBFD carrier to the interference received by the victim gNB/UE on a different frequency unit n (e.g., another subband/RB/subcarrier n) in the same SBFD carrier.

- The value or value range of ISIR should be asked to RAN4. The ISIR can be described per subband, per RB, or per subcarrier depending on the granularity of the frequency unit. For RAN1’s simulation purpose, it is preferred that RAN4 can provide the value or value range of per-subcarrier-ISIR.

- FFS: details of gNB-gNB co-channel inter-subband CLI modelling, e.g.,

  - The gNB-gNB co-channel inter-subband CLI from aggressor gNB A' to victim gNB A at UL subcarrier n can be modelled as Iinter-SB-CLIn,A'→A=m≠nm∈DL subbandIinter-SB-CLIm,n,A'→A, wherein,

    - m is the DL subcarrier index of aggressor gNB A'.

    - dBIinter-SB-CLIm,n,A'→A=Ptxm,A'+Channel GainA'→A-dBβgNB-gNBm,n

    - Ptxm,A' is the DL transmission power of aggressor gNB A' at subcarrier m (in dBm).

    - Channel GainA'→A is the effective channel gain between the aggressor gNB A' and the victim gNB A, taking into account the pathloss, penetration loss, shadow fading, Tx/Rx antenna gain, beamforming gain and fast fading.

    - βgNB-gNBm,n is the gNB-gNB per-subcarrier-ISIR.

- FFS: details of UE-UE co-channel inter-subband CLI modelling, e.g.,

  - The UE-UE co-channel inter-subband CLI from aggressor UE B' to victim UE B at DL subcarrier n can be modelled as Iinter-SB-CLIn,B'→B=m≠nm∈UL subbandIinter-SB-CLIm,n,B'→B, wherein,

    - m is the UL subcarrier index of aggressor UE B'.

    - dBIinter-SB-CLIm,n,B'→B=Ptxm,B'+Channel GainB'→B-dBβUE-UEm,n

    - Ptxm,B' is the UL transmission power of aggressor UE B' at subcarrier m (in dBm).

    - Channel GainB'→B is the effective channel gain between the aggressor UE B' and the victim UE B, taking into account the pathloss, penetration loss, shadow fading, Tx/Rx antenna gain, beamforming gain and fast fading.

    - βUE-UEm,n is the UE-UE per-subcarrier-ISIR.

- FFS: Modelling of co-site inter-sector co-channel inter-subband CLI

- The feasibility of the gNB-gNB and UE-UE co-channel inter-subband CLI modelling should be asked to RAN4

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| QC | Support in principle. We have few comments on the proposal: - The value of the ISIR is different for FR1 and FR2. - Preferred value is per-RB for system level evaluation. - Channel model based on TR 38.802 and TR 38.901 with some adjustments on height and angular spread. |
| NTT DOCOMO | We support. |
| New H3C | Support |
| CATT | The granularity of SIR may be decided by RAN4. |
| Spreadtrum | Modelling of co-site inter-sector co-channel inter-subband CLI should be taken into account. We are open for ISIR per subband, per multi RB and per RB. |
| ZTE | Similar view as for proposal 3-3. From our perspective, per sub-carrier granularity is too small. We would prefer some per RB or per subband level model. However, this is anyway up to RAN4. If RAN1 can not reach consensus on this, we can just delete the following sentence. For RAN1’s simulation purpose, it is preferred that RAN4 can provide the value or value range of per-subcarrier-SIR. Regarding the two FFSs, we suggest to delete them for now. Anyway we need to further discuss them further. |
| NEC | Agree with the proposal |
| vivo | Similar comment as to proposal 3-3: We need to highlight to RAN4 that the key assumption that requires RAN4 input is βgNB-gNBm,n and βUE-UEm,n , and ask its dependency with the frequency distance between Tx RB m and the Rx RB n. |
| Sharp | We have similar view that we should ask is βgNB-gNBm,n and βUE-UEm,n to RAN4. |
| Sony | Support |
| MediaTek | Similar view as provided in response to “Initial proposal 3-3”. No need for the two FFSs. |
| Xiaomi | Support FL’s proposal in principle. Similar comments as above, the explanation for m should be updated as ‘m is the RB index or subband index or DL subcarrier index because of the definition of n is another subband/RB/subcarrier index. |
| Huawei, HiSilicon | We have a different view on the modelling method. In our understanding, our understanding it will be more proper to model the inter-subband interference signal and the gNB-gNB and UE-UE channel model. There is no need to introduce a concept of ISIR since this would effectively imply that the gNB-gNB or UE-UE CLI are only suppressed by frequency separation. According to our initial analysis in R1-2203156, for the homogeneous macro deployment, SBFD is infeasible due to the strong gNB-to-gNB CLI. |
| Ericsson | Like our comments for proposal 3-4, there is a need to consider saturation and AGC changes here as well. Furthermore, we think that modelling of co-site inter-sector co-channel inter-subband CLI is important and needs to be modelled. In general, we think that these detailed modelling can be done later but only assume certain high level aspects of the interference and perform system level simulations. |
| Nokia/NSB | We are partly supporting. We think that the detailed UE-UE and gNB-gNB channel gain modelling needs further discussion. If beamforming gain and fast fading are modelled, from simulations point of view, this could mean that the MMSE-IRC received would be able to suppress part of this interference. We are not sure this is realistic assumption considering that this is just spurious emissions so prefer to be marked as ‘FFS’. |
| Intel | We generally support the proposal. Similar to comments for proposal 3-3, we think RB-level ISIR value is more feasible for SLS evaluations. The values of the ISIR depends on Tx/Rx filtering. We prefer to consider multiple ISIR values, according to different combinations of assumptions on Tx/Rx filtering at aggressor and victim sides, respectively. |
| OPPO | Agree with the proposal. The value per RB or subband is preferred. |
| InterDigital | Support in principle. Suggest removing per-subcarrier granularity, which is too small unnecessarily. |
| Samsung | Generally fine with the proposal. As commented in 3-3, no need to require specific frequency unit to RAN4. Regarding on co-side inter-sector co-channel inter-subband CLI, we think it is one of self-interference with much larger spatial separation. So, it can be handled by a proper gNB’s SIC implementation so that we can ignore it (at least less than self-interference). Also, if we need to define the modelling of co-site inter-sector co-channel inter-subband CLI, the CLI type should be captured in proposal 3-2. |
| LG | Fine with the proposal, and prefer to use RB as frequency unit n. |
| Moderator | @Huawei, this proposal does not prohibit modeling the channel between gNB-gNB and UE-UE. I just update it to clarify that the power is transmitted by the aggressor gNB/UE on a single transmit chain and attenuated after the channel propagation (including large scale fading, small scale fading and Tx/Rx antenna gain). |

### Initial proposal 3-5:

Regarding gNB-gNB or UE-UE adjacent-channel inter-subband CLI modelling, the ACIR in TR38.828 can be used as the starting point.

- RAN1 understands the ACIR in TR38.828 is defined as the ratio of the power transmitted by the aggressor gNB/UE on one carrier to the total interference received by the victim gNB/UE on the adjacent carrier, i.e., the ACIR in TR38.828 is described per carrier.

- For RAN1’s simulation purpose, it is preferred that RAN4 can provide the value or value range of ACIR with finer granularity, e.g., the per-subcarrier-ACIR, γgNB-gNBm,n or γUE-UEm,n, denoting the ratio of the power transmitted by the aggressor gNB/UE on subcarrier m in one carrier to the interference received by the victim gNB/UE on another subcarrier n in the adjacent carrier.

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| QC | Support in principle, prefer value per RB value specified for each frequency range. In addition, the BS-BS and UE-UE adjacent channel should be modelled following the description of TR 38.828. |
| NTT DOCOMO | We support. |
| New H3C | Support |
| CATT | Given that we think co-channel scenarios are prioritized and adjacent-channel scenarios are deprioritized for RAN1 evaluation, maybe it is sufficient to ask RAN4 to provide appropriate assumptions directly. |
| Spreadtrum | Value or value range of ACIR per RB/per multi-RB/per subband are OK for us. The granularity depends on the evaluation of ACIR variation conducted by RAN4. |
| ZTE | Similar as our previous comments. We think per sub-carrier-ACLR is too small granularity. We suggest to have per-RB or per –subband granularity. |
| vivo | For γgNB-gNBm,n or γUE-UEm,n, also need to ask RAN4 about their dependency with the frequency distance between Tx RB m and the Rx RB n. |
| Sharp | We support the proposal. |
| Sony | Support |
| MediaTek | Given that RAN1 will send an LS to RAN4, we believe these models should be provided by RAN4 as well. |
| Xiaomi | We agree with the ACIR value used in TR38.828. We are not sure why finer granularity is needed. |
| Huawei, HiSilicon | As commented earlier, we are not sure about the necessity of the adjacent channel co-existence study. |
| Ericsson | We support the proposal in principle. ACIR in RAN4 is used with certain assumptions on adjacent channel selectivity and adjacent channel leakage ratio. We need to use a similar parameter for RAN1 system level simulations that RAN4 needs to provide the range. |
| Nokia/NSB | ACIR as in TR38.828 can be used as starting point and feedback from RAN4 good as usual |
| Intel | We support the proposal. Similar to comments for proposal 3-3, we think RB-level ACIR value is more feasible for SLS evaluations. |
| OPPO | Agree with the proposal. It is better for RAN4 to provide ACIR value on a per RB or subband basis. |
| InterDigital | Support in principle. Suggest replacing the per-subcarrier-ACIR with per-RB-ACIR, as the granularity of RB(s) is sufficient. |
| Samsung | Deprioritize the discussion till RAN4 starts to their adjacent-channel coexistence study. |
| LG | RB level ACIR value is preferred. In our understanding, ACIR value used by RAN4 is based on the assumption that frequency resource of victim is located right next to frequency resource of aggressor. However, in case of SBFD, subband of victim may be far from the frequency resource of aggressor. Thus, when RAN4 provides the ACIR value, it is necessary to consider the frequency resource separation between the victim and the aggressor. |
| Moderator | @Xiaomi, the reason that RAN1 use ACIR with finer granularity is that the ACIR may be different for different RBs in the victim carrier. |

### Initial proposal 3-6:

RAN1 sends an LS to RAN4 to ask about the values or value ranges of the following interference ratios used for SBFD evaluation in RAN1:

- gNB self-interference ratio (SIR)

- Inter-subband interference ratio (ISIR) including gNB-gNB ISIR and UE-UE ISIR

- Adjacent-channel interference ratio (ACIR) including gNB-gNB ACIR and UE-UE ACIR

- FFS: other details of the LS

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| QC | Support in principle. |
| NTT DOCOMO | We support. |
| New H3C | Support |
| CATT | Support |
| Spreadtrum | We think Inter-sector inter-subband CLI should also be considered in this proposal. |
| ZTE | Support in principle. |
| NEC | Agree |
| vivo | Is it the intention to ask RAN4 in more detail, the RB level interference modelling as proposed in 3-3/3-4/3-5? If so we would be OK. |
| Sharp | We support the proposal. |
| Sony | Support |
| MediaTek | In addition to these interferences, a model that is similar to in-channel selectivity need to be considered for the impact of inter-UE CLI on reception performance in the scenario when UE channel BWs/BWPs overlap across sub-bands. Unlike the BS, there are NO “in-channel selectivity” RF requirements defined for the UE today in 38.101-1/2. However, in considering any UE receiver “in-channel” impact the following would need to be considered: The impact to UE AGC operation due to large difference between UE2 Tx interferer power and UE1 Rx wanted signal power, and the impact of changes in interferer or interferer power over time on effective AGC operation in the UE. This model is within RAN4 expertise, and RAN1 should ask RAN4 to provide inputs on how to model the impact of “in-channel” interference. |
| Xiaomi | Not sure why the third sub-bullet is needed. From our understanding, the ACIR value used in TR38.828 can be reused. |
| Huawei, HiSilicon | No. We have a different view on the interference modelling for inter-subband gNB-gNB and UE-UE CLI. |
| Ericsson | We support the direction in general. |
| Nokia/NSB | We support the proposal in principle. - In addition to the SIR, ISIR, and ACIR models, RAN4 should be asked about realistic isolation of co-located gNBs in the scenarios of interest (Urban Macro, Dense Urban, Urban Micro, Indoor Hotspot). - RAN4 should also provide feedback on how the gNB implementation (Active Antenna Systems with multiple transceivers, passive systems with few transceivers) impacts the interference models or achievable isolation. - Both FR1 and FR2 should be covered. |
| Intel | Support the proposal |
| OPPO | Support the proposal |
| Samsung | Support for SIR and ISIR. Regarding ACIR, we think that RAN4 should perform the adjacent-channel coexistence study and then inform RAN1 of their finding on ACIR value |
| Ericsson 2 | We further agree with comments from Nokia. In addition to that, we think it is important to also consider how the antenna pattern and antenna gain is impacted by the isolation structure. |
| LG | As mentioned above, for ACIR, it is necessary to consider the frequency resource separation between the victim and the aggressor. |
| Moderator | This proposal basically has been covered by the previous proposals. We can further discuss more details after we have some progress regarding proposal 3-3~3-5. |

### Initial question 3-7:

Whether RAN1 needs to additionally include the following or any other contents in LS to RAN4?

- gNB

  - Realistic net effect model that captures the essential behavior of a realistic DPD and PA combination with -45 dBc ACLR compliance.

  - Simple model of creset factor reduction (CFR) processing, e.g., hard clipping with filtering, that captures the essential behaviors of a practical BS designed for PA efficiency and ACLR compliance.

  - Realistic models on UL receiver selectivity, dynamic range and nonlinearity behaviors

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| QC | For the SLS evaluation, no other input is needed from RAN4. If agreed to adopt LLS as addition evaluation methodology, some input from RAN4 may be needed for proper UE and gNB NL modelling. |
| New H3C | Support |
| ZTE | All these bullets seems to fall into RAN4’s scope. More clarification on the motivation to have them in RAN1 is needed. |
| NEC | As discussed in 3-3, RAN4 should also be asked of assumptions (e.g. self-interference mitigation techniques used by gNB) used for modelling of SIR. |
| MediaTek | In our view, RAN1 should provide the possible configurations for SBFD and dynamic TDD to RAN4, and RAN4 provide the details for the interference models. It is up to RAN4 what “effects” it will consider for such model. We are fine with providing some notes on what should be considered or not, but we think some of the notes might be misleading and it may consume RAN1 time to have consensus on these notes. |
| Huawei, HiSilicon | We would like to know whether the intention is to study the output interference signal property from a DL inter-subband. |
| Ericsson | We support the proposal in principle with some additions. For the purposes of link level simulations for self-interference cancellation a detailed model of the gNB Tx chain is needed. Otherwise, evaluations could be unrealistic. ### Initial question 3-7:(Modified by Ericsson) Whether RAN1 needs to additionally include the following or any other contents in LS to RAN4? - gNB - Realistic net effect model that captures the essential behavior of a realistic DPD and PA combination with -45 dBc ACLR compliance. - Simple model of crest factor reduction (CFR) processing, e.g., hard clipping with filtering, that captures the essential behaviors of a practical BS designed for PA efficiency and ACLR compliance. - Realistic models on UL receiver selectivity, dynamic range and nonlinearity behaviors - Any other important modelling components characterizing typical gNB TX/RX functions. - UE - Important modelling components or inputs characterizing typical UE TX/RX functions, for example, to help with determining inter-subband emission levels and inter-subband selectivity. |
| Nokia/NSB | We don’t see much necessity of the additional contents into RAN4 LS. |
| Intel | As we commented for proposal 3-4, it is preferred that RAN4 can provide multiple ISIR values, with different combinations of assumptions on Tx/Rx filtering at aggressor and victim side, e.g., the ISIR values could be different among gNBs. |
| Samsung | It is premature to ask these points since it is not clear whether any is needed in RAN1. RAN1 should have a clear understanding on these points before asking to RAN4. In our view, the negative impacts from the non-linearity can be already captured in SIR or ISIR. |
| LG | We share the view with Samsung. It can be assumed that impacts from the non-linearity are reflected in SIR/ISIR provided by RAN4. |
| Moderator | Companies’ views are divergent. We can further discuss more details after we have some progress regarding proposal 3-3~3-5. |

## 4.4 2nd Round Proposals (closed)

### Updated proposal 3-1:

For discussion purpose, consider the following as RAN1’s common understanding:

- Co-channel interference: The interference is from the aggressor to the victim in the same carrier.

  - Co-channel intra-subband interference: The interference is caused by transmission of the aggressor in a subband in a carrier to reception of the victim in the same subband in the same carrier.

  - Co-channel inter-subband interference: The interference is caused by transmission of the aggressor in subband#1 in a carrier to reception of the victim in subband#2 in the same carrier, where the subband#1 and subband#2 are non-overlapping in frequency.

  - Note: ‘Co-channel’ here doesn’t imply the same RB, it implies the same carrier.

- Adjacent channel interference: The interference is from the aggressor in carrier#1 to the victim in carrier#2, where the carrier#1 and carrier#2 are adjacent carriers.

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| New H3C | We are fine with this proposal |
| Intel | We are OK with the updated text of this proposal |
| Samsung | We are generally fine with the proposal It is allowed in deployment case 2 that different gNBs can have different UL subbands but some of UL subband is partially overlapped in frequency. The proposal seems only cover the UL subband is same across gNBs in definition of co-channel intra-subband interference. |
| vivo | We are fine with the updated proposal 3-1 |
| CATT | The proposal is fine if only same SBFD subband configuration is considered. But if different SBFD subband configurations (Deployment Case 2), it seems that the co-channel interference is not clear. |
| ZTE | After reading companies’ comments, we are not sure whether we need to change “carrier” to “channel” in the proposal. Especially with the note, it seems “carrier” equals to “channel”. |
| MediaTek | Fine with the updated proposal. |
| Sony | The following note is confusing: - Note: ‘Co-channel’ here doesn’t imply the same RB, it implies the same carrier. We have intra-subband interference defined as: Co-channel intra-subband interference: The interference is caused by transmission of the aggressor in a subband in a carrier to reception of the victim in the same subband in the same carrier What does this intra-subband means in the context of this note (interference with different RBs)? Is this the case where a subband say occupying RB1 to RB20 and has UE1 occupying RB1-RB10 whilst UE2 occupying RB11-RB20 and we are looking at interference between UE1 and UE2? Isn’t this just basic TDD stuff in Rel-15 and LTE TDD stuff? Why do we need to look into this? |
| Nokia, NSB | We are ok with the proposal |
| NEC | We are okay with the proposal |
| Huawei, HiSilicon | Fine. |
| Xiaomi | We are fine with the updated proposal. |
| Spreadtrum | Agree with Sony about intra-subband interference definition. The same resource in the subband should be noted in the definition. |
| Ericsson | We support this proposal. |
| NTT DOCOMO | We support the proposal. |
| QC | Support |
| Moderator | Updated proposal 3-1a was provided based on companies’ comments. A note was added to address Sony/CATT/Samsung/Spreadtrum’s comments. |

### Updated proposal 3-2:

Consider the following as the common understanding on potential interference types for SBFD operation:

- gNB self-interference (SI): Interference caused by DL transmission in DL subband in a carrier to UL reception in UL subband in the same carrier at the gNB side, where the DL subband and the UL subband are non-overlapping in frequency.

- gNB-UE co-channel intra-subband interference: This is the same as the legacy DL interference type in legacy TDD network with static TDD UL/DL configuration.

- UE-gNB co-channel intra-subband interference: This is the same as the legacy UL interference type in legacy TDD network with static TDD UL/DL configuration.

- (inter-cell) gNB-gNB co-channel intra-subband CLI: CLI caused by DL transmission of the aggressor gNB in a subband in one carrier to UL reception of the victim gNB in the same subband in the same carrier.

- (inter-cell) UE-UE co-channel intra-subband CLI: CLI caused by UL transmission of the aggressor UE in a subband in one carrier to DL reception of the victim UE in the same subband in the same carrier.

- (inter-cell) gNB-gNB co-channel inter-subband CLI: CLI caused by DL transmission of the aggressor gNB in DL subband in a carrier to UL reception of the victim gNB in UL subband in the same carrier, where the DL subband and the UL subband are non-overlapping in frequency.

  - This includes co-channel inter-subband CLI between gNBs in different sectors of the same site, i.e., co-site inter-sector co-channel inter-subband CLI.

- (intra-cell/inter-cell) UE-UE co-channel inter-subband CLI: CLI caused by UL transmission of the aggressor UE in UL subband in a carrier to DL reception of the victim UE in DL subband in the same cell or neighboring cell in the same carrier, where the UL subband and the DL subband are non-overlapping in frequency.

- gNB-gNB adjacent-channel CLI: CLI caused by DL transmission of the aggressor gNB in a carrier to UL reception of the victim gNB in another adjacent carrier.

  - This includes adjacent-channel CLI between gNBs in the same and different sectors of the same site, i.e., co-site intra and inter-sector adjacent-channel CLI.

- UE-UE adjacent-channel CLI: CLI caused by UL transmission of the aggressor UE in a carrier to DL reception of the victim UE in another adjacent carrier.

- Note: Some of the interferences may not be used according to the deployment scenarios, e.g, the SBFD subband configurations are the same or different across gNBs.

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| New H3C | We are fine with this proposal |
| Intel | We are OK with the proposal including the supporting note. |
| Samsung | We disagree gNB-gNB co-channel inter-subband CLI includes co-site inter-sector co-channel inter-subband CLI. Co-site inter-sector co-channel inter-subband CLI is far different interference characteristic and different cancelation techniques can be applied, compared to gNB-gNB co-channel inter-subband CLI. - Since these sector antennas are co-located and can be connected to a single baseband processing unit, the interference can be removed by means of digital interference cancelation, which is not applicable to gNB-gNB co-channel inter-subband CLI. So, we cannot reuse gNB-gNB co-channel inter-subband CLI modelling for co-site inter-sector co-channel inter-subband CLI. Futhermore, the inter-sector co-channel inter-subband CLI is similar as two-panel antennas in one sector and so the inter-sector co-channel inter-subband CLI is one type of self-interference between two-panel antennas - Inter-subband CLI from other gNB cannot be anticipated, but co-site inter-sector co-channel interference can be managed by a gNB. For example, gNB can use Tx beamforming to suppress inter-sector co-channel interference if needed. The Tx beamforming is hard to use gNB-gNB co-channel interference since gNB’s interface is required and there are some delay. With the observations, we suggest to define inter-sector co-channel inter-subband CLI separately. |
| vivo | We are fine with this proposal |
| CATT | We are fine with the proposal and also fine with Samsung’s suggestion to separately model inter-sector co-channel inter-subband CLI. |
| ZTE | We also prefer to separately model inter-sector co-channel interferences. |
| MediaTek | Fine with the updated proposal. |
| Sony | Thanks Moderator for the explanation. However, the intra subband interference is still confusing especially when we now have the note in Updated proposal 3-1 stating: - “Note: ‘Co-channel’ here doesn’t imply the same RB, it implies the same carrier. If two UEs use different RBs and both are receiving in the DL, where is this intra subband interference? Is this adjacent RB interference between two UEs in the same carrier? Shouldn’t OFDM air interfance sort this out already? Why is this now significant where we have this since LTE days and no one bothered with it? Would appreciate some clarification otherwise, I think we shouldn’t waste resource considering this type of interference since it has been there since the word TDD was invented. |
| Nokia, NSB | We are ok with the proposal |
| NEC | We are okay with the proposal |
| Huawei, HiSilicon | We are fine with this proposal. On the co-channel inter-sector intersubband CLI, our understanding is that the modeling of the CLI may be dependent on whether the co-site sectors are closely to each other. The definition as it is now could be fine but it can be studied in later feasibility and performance evaluations. |
| Xiaomi | We are OK with Samsung’s suggestion. Besides, inter-sector co-channel intra-subband CLI should also be included for completion. |
| Spreadtrum | We are generally fine with the proposal and suggest to consider using overlapping resource in intra-subband interference definition. |
| Ericsson | We support the proposal. |
| NTT DOCOMO | We support the proposal. |
| QC | We are fine with the updated proposal of interference types. One minor editorial change: - Note: Some of the interferences may not be used according to the deployment scenarios, e.g, whether the SBFD subband configurations are the same or different across gNBs. |
| Moderator | Updated proposal 3-2a was provided based on companies’ comments. |

### Updated proposal 3-3:

For discussion of gNB self-interference modelling for the purposes of system level simulations (more detailed model may be needed for link level simulations), consider introducing ratio of self-interference (RSI) to represent the overall self-interference suppression capability of gNB by means of spatial isolation, subband frequency isolation and beamform nulling/isolation, etc. SRI also takes into account the impact of Tx/Rx antenna element gain on self-interference. The RSI, denoted as αSIm,n,  can be defined as the ratio of the total power transmitted by gNB across all transmit chains on a frequency unit m (e.g., subband/RB/subcarrier m) in a SBFD carrier to the residual self-interference received by the same gNB on a single receiver chain on a different frequency unit n (e.g., another subband/RB/subcarrier n) in the same SBFD carrier.

- The following should also be asked to RAN4:

  - The value range of RSI for each frequency range. The RSI can be described per subband, per RB, or per subcarrier depending on the granularity of the frequency unit, and it is up to RAN4.

  - The dependency of RSI with the frequency distance between Tx frequency unit m and the Rx frequency unit n, and the dependency of RSI with the SBFD DL subband size, the number of RBs used for DL transmission.

  - The feasibility of the gNB self-interference modelling including factors such as blocking, AGC, TX/RX beamforming, etc.

- FFS: whether RSI takes into account digital interference cancellation or not.

- FFS: details of gNB self-interference modelling, e.g., the gNB self-interference on a single receiver chain at UL subcarrier n can be modelled as

  - ISIn=m≠nm∈DL subbandISIm,n, wherein,

    - m is the DL subcarrier index.

    - dBISIm,n=Ptxm-dBαSIm,n

    - Ptxm is gNB’s DL transmission power across all transmit chains at subcarrier m (in dBm).

    - αSIm,n is the per-subcarrier-SIR.

  - FFS: the dependence of αSIm,non Tx/Rx beam-pair for FR2 especially for clutter echo.

  - FFS: consider a statistical clutter model based on statistics of clutter strength and AoA.

  - FFS: whether/how to consider the impact of locations of Tx/Rx antenna elements on RSI.

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| New H3C | We are fine with this proposal |
| Intel | We are OK with the updated text and proposal. |
| Samsung | Generally fine with the proposal. Regarding FFS: whether RSI takes into account digital interference cancelation or not, we cannot agree this FFS. Digital interference cancelation is a key techniques to remove residual self-interference at baseband processing unit in digital domain. For high power gNB (in macro layer), the residual self-interference level without digital interference cancelation is still larger than noise level so that the digital interference cancelation could be implemented. If RSI does not include the digital interference cancelation in SLS, the evaluation results are too pessimistic and cannot capture realistic implementation. So, we suggest to the following modification in yellow: For discussion of gNB self-interference modelling for the purposes of system level simulations (more detailed model may be needed for link level simulations), consider introducing ratio of self-interference (RSI) to represent the overall self-interference suppression capability of gNB by means of spatial isolation, subband frequency isolation, digital interference cancellation and beamform nulling/isolation, etc. SRIRSI also takes into account the impact of Tx/Rx antenna element gain on self-interference. The RSI, denoted as αSIm,n, can be defined as the ratio of the total power transmitted by gNB across all transmit chains on a frequency unit m (e.g., subband/RB/subcarrier m) in a SBFD carrier to the residual self-interference received by the same gNB on a single receiver chain on a different frequency unit n (e.g., another subband/RB/subcarrier n) in the same SBFD carrier. - The following should also be asked to RAN4: - The value range of RSI for each frequency range. The RSI can be described per subband, per RB, or per subcarrier depending on the granularity of the frequency unit, and it is up to RAN4. - The dependency of RSI with the frequency distance between Tx frequency unit m and the Rx frequency unit n, and the dependency of RSI with the SBFD DL subband size, the number of RBs used for DL transmission. - The feasibility of the gNB self-interference modelling including factors such as blocking, AGC, TX/RX beamforming, etc. - FFS: whether RSI takes into account digital interference cancellation or not. |
| vivo | We are fine with the updated proposal. Just one typo in the first paragraph, “SRI” should be “RSI” |
| CATT | Support |
| MediaTek | We are generally fine with the proposal. For the LS to RAN4, RAN1 should provide the relevant SBFD configurations. |
| Sony | Support |
| Nokia, NSB | In principle agree with the proposal. Agree with Samsung on that the final RSI should include all cancellation aspects. In addition, as the number of transmit and receive chains may have an impact on the RSI, RAN4 should also take this into account when providing feedback on the cancellation. We suggest to add to the RAN4 list of items the following bullet point: - The following should also be asked to RAN4: … - The dependency of RSI with the number of transmit and receive chains. |
| NEC | We support this proposal |
| Huawei, HiSilicon | In principle, there is also wireless channel between the TX and RX antenna array including direct coupling and reflection. For direct coupling, it is hard to model the channel. For the reflection, the channel may be modelled based on the cluster reflection. On the other hand, the self-interference suppression/cancellation capability is closely related to implementations such as antenna isolation, interference cancellation, TX beamforming, RX analogue filter, Beam pairing (for FR2), etc. For SLS, the simplied model seems sufficient. |
| Xiaomi | The explanation of m under the formula of self-interference should be aligned with the main bullet, i.e. it may be subband/RB/subcarrier depending on RAN4’s decision. We are OK with the other parts. |
| Spreadtrum | Support |
| Ericsson | As stated in the proposal, a “more detailed model may be needed for link level simulations”, and we think this needs to be addressed in a concrete proposal. To elaborate, one of the important components for SBFD feasibility evaluation is the achievable performance of, e.g., digital self-interference cancellation (as stated in the FFS of this proposal). This requires detailed link-level simulations, and several companies have proposed schemes, e.g., based on Interference Rejection Combining (IRC), symbol-level successive interference cancellation, (SLIC), codeword-level successive interference cancellation (CWIC), etc. In order to have comparable results between companies, it is necessary that RAN1 agrees on a detailed model of the net effect of the gNB transmit chain, accounting e.g., for PA + PA linearization (DPD) + crest factor reduction, etc. For such a detailed model, input is needed from RAN4. Such a model would allow the generation of the gNB Tx signal on a sample-by-sample basis in the link simulator using the baseband OFDM signal (undistorted) as input. The self-interference model RSI has the following components and in order to study feasibility of SBFD, it is necessary to consider each of these. For example, Qualcomm lists the following components in the self-interference link budget in R1-2205031, and we think at least these components require feasibility analysis. - Feasibility study on how much antenna isolation is attainable - Feasibility study on subband frequency isolation - Feasibility study on beam nulling - Feasibility study on analog rejection - Feasibility study on interference cancellation algorithms on the RX side As discussed above, link level simulations are needed to model and verify attainable self-interference cancellation, and this requires input from RAN4 on a realistic net-effect (PA + DPD + crest factor reduction) model. Regarding the first FFS from the last section, we think that FR1 mid-band also needs to be included. |
| NTT DOCOMO | We support the proposal. |
| QC | We are generally fine with the general direction. However, we think it may better to ask RAN4 for separate numbers of spatial isolation, frequency isolation, beamform nulling/isolation, subband filter and digital cancellation instead of a single number. This will enable RAN1 to evaluate the impact of separate factors independently. Also, we think a simple model of the SI should be adpted that doesn’t depend on the amount of allocated DL RBs. As highlighted in our tdoc R1-2205031, the amount of residual self-interfence in the UL subbands for different DL allocations is almost the same. Figures below shows Lab measurements to capture the leakage power from the DL signal into the UL subband. The allocation of DL signal bandwidth was varied from 32 to 112 RBs per the DL subband starting from edge DL RB to UL subband. The measured leakage power ratio (i.e., frequency selectivity) per RB in the UL subband for the different DL signal bandwidth is shown in the figure. -45 dBc ACLR Figure 2-6: Per-RB adjacent subband leakage ratio (dBc) In addition, a first-order approach is to consider a flat model (non-frequency selective) for the frequency isolation. In the analysis below, the variation of residual self-interference across the UL SB are within few dBs. dBISI=Ptxm-αSI-spatial+αSI-frequency +αSI-beam+αSI-digtial In addition, the sublet of feasibility of gNB self-interference modelling should consider the Tx/Rx beam pair for the case of FR2 deployment. - The feasibility of the gNB self-interference modelling including factors such as blocking, AGC, TX/RX beamforming, Tx/Rx beam pair for FR2 etc. |
| Moderator | Updated proposal 3-3a was provided based on companies’ comments. The proposal is for SLS, I think it would be better and enough to use RSI to reflect the overall self-interference suppression capability including digital interference cancellation as Sumsung suggested. A more detailed model for link level simulations can be separately discussed. |

### Updated proposal 3-4:

For discussion of gNB-gNB or UE-UE co-channel inter-subband CLI modelling, consider introducing gNB-gNB or UE-UE inter-subband interference ratio (ISIR) to represent the interference suppression effect of subband frequency isolation and possible RF filtering. The ISIR, denoted as βgNB-gNBm,n or βUE-UEm,n, can be defined as the ratio of the power transmitted by the aggressor gNB/UE on a single transmit chain on a frequency unit m (e.g., subband/RB/subcarrier m) in a SBFD carrier and attenuated after the channel propagation (including large scale fading, small scale fading and Tx/Rx antenna gain) to the interference received by the victim gNB/UE on single receiver chain on a different frequency unit n (e.g., another subband/RB/subcarrier n) in the same SBFD carrier.

- The following should also be asked to RAN4:

  - The value range of ISIR for each frequency range. The ISIR can be described per subband, per RB, or per subcarrier depending on the granularity of the frequency unit, and it is up to RAN4.

  - The dependency of ISIR with the frequency distance between Tx frequency unit m and the Rx frequency unit n

  - The feasibility of the gNB-gNB and UE-UE co-channel inter-subband CLI modelling including factors such as blocking, AGC, etc. should be asked to RAN4

- FFS: details of gNB-gNB co-channel inter-subband CLI modelling, e.g.,

  - The gNB-gNB co-channel inter-subband CLI from aggressor gNB A' to victim gNB A at UL subcarrier n can be modelled as Iinter-SB-CLIn,A'→A=m≠nm∈DL subbandIinter-SB-CLIm,n,A'→A, wherein,

    - m is the DL subcarrier index of aggressor gNB A'.

    - dBIinter-SB-CLIm,n,A'→A=Ptxm,A'+Channel GainA'→A-dBβgNB-gNBm,n

    - Ptxm,A' is the DL transmission power of aggressor gNB A' at subcarrier m (in dBm).

    - Channel GainA'→A is the effective channel gain between the aggressor gNB A' and the victim gNB A, taking into account the pathloss, penetration loss, shadow fading, Tx/Rx antenna gain, beamforming gain and fast fading.

    - βgNB-gNBm,n is the gNB-gNB per-subcarrier-ISIR.

- FFS: details of UE-UE co-channel inter-subband CLI modelling, e.g.,

  - The UE-UE co-channel inter-subband CLI from aggressor UE B' to victim UE B at DL subcarrier n can be modelled as Iinter-SB-CLIn,B'→B=m≠nm∈UL subbandIinter-SB-CLIm,n,B'→B, wherein,

    - m is the UL subcarrier index of aggressor UE B'.

    - dBIinter-SB-CLIm,n,B'→B=Ptxm,B'+Channel GainB'→B-dBβUE-UEm,n

    - Ptxm,B' is the UL transmission power of aggressor UE B' at subcarrier m (in dBm).

    - Channel GainB'→B is the effective channel gain between the aggressor UE B' and the victim UE B, taking into account the pathloss, penetration loss, shadow fading, Tx/Rx antenna gain, beamforming gain and fast fading.

    - βUE-UEm,n is the UE-UE per-subcarrier-ISIR.

- FFS: Modelling of co-site inter-sector co-channel inter-subband CLI

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| New H3C | We are fine with this proposal |
| Intel | We are OK with the updated text and proposal. |
| Samsung | We are fine with the proposal. |
| vivo | We are fine with this proposal |
| CATT | Support |
| MediaTek | As explained in our contribution, it is essential to provide RAN4 with the assumptions on SBFD configurations. For example, is UE victim DL BWP spans the UL subband or is it confined to the DL subband? is the aggressor UE UL BWP spans the DL subbands or is it confined to the UL subband? The two assumptions illustrated in the figure below. a) UE channel and BWPs confined within sub-band b) BWPs overlap across sub-bands |
| Sony | Support |
| Nokia, NSB | We support the proposal, but suggest to add to the RAN4 list of items the following bullet point: - The dependency of ISIR with the number of transmit and receive chains. |
| Huawei, HiSilicon | We don’t support the proposal. To have a proper model of the co-channel inter-subband gNB-gNB or UE-UE CLI, it is important to reach some common understanding where the co-channel inter-subband CLI comes from. In general, the interference from DL subband at the aggressor to UL subband at the victim are contributed by two parts: one is the transmitter leakage due to the non-linearity of transmitter, and the other is the receiver selectivity due to non-linearity of receiver. - The first part depends on the signal transmitted on the DL subband, i.e. the interference received on UL subband is a function of the transmitted signal on the DL subband, and the transmitted interference signal on the UL RBs has nothing to do with the channel. It is a function of the DL to UL leakage ratio similar to ACLR, but not the same concept depending the DL subband and UL subband are in the same carrier or different carrier. The transmitted interference signal will propagate over wireless channel to the receiver. - For the interference due to selectivity, the interference is generated by selection of the signal on the DL subband at the victim. The power of the interference due to the selectivity depends on the received power of the signal on the DL RBs. However, the received power of the signal on the DL RB at the receiver depends on the transmit power of the DL signal, the precoding at transmitter and the channel between the transmitter and receiver. As an example of the above discussion, when the transmitted power of the aggressor is 53dBm, and the DL-UL leakage ratio is -45dB, then the transmitted interference power on the UL RB will be 8dBm assuming equal number of DL and UL RBs. And if the total attenuation including path loss and the antenna gain is -75dB, the leakage interference could be -82dBm. For the interference due to the receiver selectivity, due to the downlink precoding, path loss and the TRX antenna gain, the received power of the signal on the downlink PRB at the receiver is not fixed due to different precoding effect. For instance, the received power could be -30dBm, and assuming the selectivity ratio is -45dB, then the interference on the UL RBs could be -75dBm. The signal on the DL RBs and the leakage on the UL RBs may have different radiation patterns, hence requires different modelling. Based on the above analysis, the leakage interference and receiver selectivity interference should be modelled separately. It is difficult to combine the two factor into one. Therefore, we suggest to ask RAN4 for the two parts separately. Furthermore what RAN4 will provide is uncertain as it is now, e.g. one value or a value range. So we suggest just asking RAN4 for the modelling of the interference and how to proceed with the model provided by RAN4 can be for further discussed within RAN1. Proposal: For discussion of gNB-gNB or UE-UE co-channel inter-subband CLI modelling, consider the following aspects: - Transmitter leakage due to the non-linearity of the transmitter at the aggressor, - Receiver selectivity due to non-linearity of the receiver at the victim. The following questions should be asked to RAN4: - For a specific DL RB(s) and UL RB(s) pair in one carrier or in different carriers for both gNB and UE - How to model the DL TX to UL RX or UL TX to DL RX leakage interference at the transmitter - How to model the interference due to the receiver selectivity at the receiver - FFS: Usage of the above model provided by RAN4 in the evaluation - FFS: Modelling of co-site inter-sector co-channel inter-subband CLI In the following, we also provide some of our thinking on how to make use of the leakage interference and selectivity interference for RAN1 evaluations. - The leakage interference can be modeled as follows sinter-SB-CLI, leakage, ktxn=m≠nm∈DL subbandsinter-SB-CLI, leakage, ktxm,n is the leakage signal of the ktx-th transmit antenna of subcarrier n, which may be modeled as a Gaussian variable. rinter-SB-CLI, leakagen=Hnsinter-SB-CLI, leakagen and Hn is the wireless channel from the aggressor antenna array to the receiver antenna array, and rinter-SB-CLI, leakagen is the received leakage interference of the receiver antenna array. The received leakage power could be modeled as the diagonal element s of HnHnHP, and P is the leakage power of each transmit antenna on subcarrier n. - The selectivity interference at the receiver can be modeled as follows: The transmitted signal on subcarrier m is WmSm, and then the received signal on the DL subcarrier m at the receiver is HmWmSm, so the received power on subcarrier m at the receiver is the diagonal elements of (PHmWmSmHmWmSmH), P is the transmit power for subcarrier m. and the selectivity interference is the diagonal elements of (βm,nPHmWmSmHmWmSmH), where βm,n is the selectivity ratio between subcarrier m, and n. The total interference power is the sum of the leakage and selectivity. However, it should be noted that in the evaluation, the interference should be modeled as signal, not just interference power. |
| Xiaomi | The explanation of m under the formula of inter-subband CLI should be aligned with the main bullet, i.e. it may be subband/RB/subcarrier depending on RAN4’s decision. We are OK with the other parts. |
| Intel2 | We support the proposal, but we suggest adding within the second sub-bullet within the first bullet, similarly as for the self-interference modelling, the following: - The following should also be asked to RAN4: - The value range of ISIR for each frequency range. The ISIR can be described per subband, per RB, or per subcarrier depending on the granularity of the frequency unit, and it is up to RAN4. - The dependency of ISIR with the frequency distance between Tx frequency unit m and the Rx frequency unit n, and the dependency of ISIR with the SBFD DL subband size, the number of RBs used for DL transmission. - The feasibility of the gNB-gNB and UE-UE co-channel inter-subband CLI modelling including factors such as blocking, AGC, etc. should be asked to RAN4 |
| Spreadtrum | Support |
| Ericsson | We support the proposal in principle. It is worthy to note that, we need feedback from RAN4 regarding modelling of co-site co-channel inter-subband CLI since RAN4 regularly considered front to back ratio in their analysis. Therefore, we propose that the point be highlighted instead of an FFS. |
| NTT DOCOMO | We support the proposal. |
| QC | We are geneally okay with the proposal. However, we think a simpler approach is to separate channel modelling discussion (which is discussed in issue # 4-4 FL summary) and the required value of ISIR for gNB-gNB or UE-UE for both frequency ranges. In addition, a first order approximation is to assume that the inter-subband leakage is flat to simplify the SLS model. This proposal can focus on ISIR definition at the transmit side. For discussion of gNB-gNB or UE-UE co-channel inter-subband CLI modelling, consider introducing gNB-gNB or UE-UE inter-subband interference ratio (ISIR) to represent the interference suppression effect of subband frequency isolation and possible RF filtering. The ISIR, denoted as βgNB-gNB or βUE-UE , can be defined as the ratio of the power transmitted by the aggressor gNB/UE on a single transmit chain on a frequency unit m (e.g., subband/RB/subcarrier m) in a SBFD carrier and attenuated after the channel propagation (including large scale fading, small scale fading and Tx/Rx antenna gain) to the interference power at the same transmit chain received by the victim gNB/UE on single receiver chain on a different frequency unit n (e.g., another subband/RB/subcarrier n) in the same SBFD carrier. - The value range of ISIR for each frequency range. The ISIR can be described per subband or per RB, or per subcarrier depending on the granularity of the frequency unit, and it is up to RAN4. |
| Apple | Do not support the proposal. In our view, channel propagation should not be included (similar to ACLR calculation for adjucent channel). In addition, in-band emission can be the starting point for RAN4. Why here the transmit power is per Tx chain but for SIR at gNB it is across all Tx chains? |
| Moderator | Given Huawei explained a more accurate model using leakage ratio at transmitter side and receiver selectivity at receiver side, let’s see if companies are OK to ask RAN4 in a simpler way for both co-channel inter-subband CLI modelling and adjacent-channel CLI modelling as Huawei suggested. Updated proposal 3-4a was provided to cover both co-channel inter-subband CLI modelling and adjacent-channel CLI modelling. @Apple, regarding why here the transmit power is per Tx chain but for SIR at gNB it is across all Tx chains, the reason is we will further apply the MIMO channel model (including small scale) when we calculating the gNB-gNB inter-subband CLI, similar as Huawei explained regarding how to use the leakage ratio to model the leakage interference. |

### Updated proposal 3-5:

Regarding gNB-gNB or UE-UE adjacent-channel inter-subband CLI modelling, the ACIR in TR38.828 can be used as the starting point.

- RAN1 understands the ACIR in TR38.828 is defined as the ratio of the power transmitted by the aggressor gNB/UE on one carrier to the total interference received by the victim gNB/UE on the adjacent carrier, i.e., the ACIR in TR38.828 is described per carrier. The ACIR can be described with finer granularity, e.g., per subband, per RB. It is up to RAN4 to provide ACIR with finer granularity for RAN1 simulation.

- For RAN1’s simulation purpose, RAN1 assume γgNB-gNBm,n or γUE-UEm,n is the ratio of the power transmitted by the aggressor gNB/UE on a frequency unit m (e.g., subband/RB/subcarrier m) in one carrier to the interference received by the victim gNB/UE on another frequency unit n (e.g., another subband/RB/subcarrier n) in the adjacent carrier.

- The following need to be asked to RAN4:

  - The value range of gNB-gNB ACIR and UE-UE ACIR.

  - The dependency of ACIR with the frequency distance between Tx frequency unit m and the Rx frequency unit n

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| New H3C | We are fine with this proposal |
| Intel | We are OK with the updated text and proposal. |
| Samsung | We can revisit this proposal after resolving the below FFS point in Updated proposal 1-2. FFS: Deployment Case 4 should be evaluated in RAN1-only, or RAN4-only, or both RAN1 and RAN4. |
| vivo | We are fine with this proposal |
| CATT | Given that deployment case 4 may be evaluated in RAN4-only as in proposal 1-2, we do not think we need to make such agreements in this meeting and it is sufficient to let RAN4 provides inputs if needed. |
| Sony | Support |
| Nokia, NSB | We support the proposal, but suggest to add to the RAN4 list of items the following bullet point: - The dependency of ACIR with the number of transmit and receive chains. |
| Huawei, HiSilicon | From our point of view, this question is related to the proposal 3-4. It is related to the modelling provided by RAN4. Hence, there is no need to have this agreement right now. |
| Xiaomi | We are OK with the updated proposal and fine to check with RAN4. Just elaborate ourself a little bit: For legacy definition of ACIR, it consists of two component, i.e. ACLR and ACS - ACLR: the ratio of the filtered mean power centred on the assigned NR channel frequency to the filtered mean power centred on an adjacent NR channel frequency at nominal channel spacing. - ACS: the ratio of the receive filter attenuation on the assigned channel frequency to the receive filter attenuation on the adjacent channel(s). From our understanding, the difference from SBFD and legacy adjacent channel CLI is that the victim subband doesn’t occupied the whole channel. Hence at least the ACLR part can be reused. For the ACS part, from our understanding the current definition can be reused as well. But we are open to discuss if companies think it is better to check with RAN4. |
| Spreadtrum | Support |
| Ericsson | The proposal for adjacent channel CLI is incomplete in our opinion. We think that the adjacent channel CLI also needs to be modelled in the same way as the co-channel CLI described in 3-4. On that regard, we think that the excellent description of the adjacent channel CLI in the Moderator’s contribution could be used as the starting point. Furthermore, we are not sure if there is common understanding between RAN1 and RAN4 regarding the specific definition of ACIR. The RAN4 framework of adjacent channel leakage ratio (ACLR), adjacent channel selectivity (ACS) and adjacent channel interference ratio (ACIR) are all metrics that are tailored towards defining requirements, and in particular requirements that can be tested. For system simulations, in our view it would be preferable if RAN1 agrees on a simplified framework instead of trying to apply the RAN4 concepts directly. ACIR is used for estimating the interference ratio in the adjacent carrier, and it may cause confusion if we use the same term here, since the SBFD subband is not the whole adjacent carrier. Considering the above two points, please find modifications considering the text from R1-2204303. ### Updated proposal 3-5:(Modified by Ericsson) Regarding gNB-gNB or UE-UE adjacent-channel inter-subband CLI modelling, the ACIR in TR38.828 can be used as the starting point. - RAN1 understands the ACIR in TR38.828 is defined as the ratio of the power transmitted by the aggressor gNB/UE on one carrier to the total interference received by the victim gNB/UE on the adjacent carrier, i.e., the ACIR in TR38.828 is described per carrier. For SBFD operation a similar interference ratio as ACIR but in the subband of the adjacent carrier, with finer granularity ,e.g., per RB, per sub-carrier or per subband needs to be considered. We can call this gNB-gNB- adjacent channel-per-subcarrier interference ratio γgNB-gNBm,n and UE-UE- adjacent channel-per-subcarrier interference ratio γUE-UEm,n. - Details of gNB-gNB adjacent-channel CLI modelling, i.e, Iadjacent-channel-CLIn,A'''→A Iadjacent-channel-CLIn,A'''→A=m≠nm∈adjacent DL channelIadjacent-channel-CLIm,n,A'''→A dBIadjacent-channel-CLIm,n,A'''→A=PtxgNB+CLA'''→A-dBγgNB-gNBm,n wherein, - CLA'''→A is the coupling loss between the serving gNB A and interfering gNB A''' in adjacent channel, including pathloss, penetration loss, shadow fading and Tx/Rx antenna gain. - γgNB-gNBm,n is the gNB-gNB- adjacent channel-per-subcarrier interference ratio, - Details of UE-UE adjacent-channel CLI modelling, Iadjacent-channel-CLIn,B'''→B Iadjacent-channel-CLIn,B'''→B=m≠nm∈adjacent UL channelIadjacent-channel-CLIm,n,B'''→B dBIadjacent-channel-CLIm,n,B'''→B=PtxUE,B'''+CLB'''→B-dBγUE-UEm,n wherein, - PtxUE,B''' is the average UL transmission power per subcarrier of UE B''' in adjacent channel (in dBm). - CLB'''→B is the coupling loss between the target UE B and interfering UE B''' in adjacent channel, including pathloss, penetration loss, shadow fading and Tx/Rx antenna gain. - γUE-UEm,n is the UE-UE adjacent channel per-subcarrier-interference ratio - For RAN1’s simulation purpose, RAN1 assumes γgNB-gNBm,n or γUE-UEm,n is the ratio of the power transmitted by the aggressor gNB/UE on a frequency unit m (e.g., subband/RB/subcarrier m) in one carrier to the interference received by the victim gNB/UE on another frequency unit n (e.g., another subband/RB/subcarrier n) in the adjacent carrier. - The following need to be asked to RAN4: - The value range of gNB-gNB and UE-UEadjacent channel per-subcarrier-interference ratio as described above. - The value range for the interference ratio within and between sectores i.e., two operators on adjacent channels in the same sector and between gNBs in different sector. - The dependency of ACIR with the frequency distance between Tx frequency unit m and the Rx frequency unit n |
| NTT DOCOMO | We support the proposal. |
| QC | We are fine with this proposal, couple of comments - The values of ACIR should be reported different for FR1 and FR2. - A simple model is to consider a flat non-frequency selective of ACIR at least for the initial evaluation. - The value range of gNB-gNB ACIR and UE-UE ACIR per frequency range. |
| Moderator | The proposal has been merged into the updated proposal 3-4a. |

## 4.5 3rd Round Proposals (closed)

### Updated proposal 3-1a:

For discussion purpose, consider the following as RAN1’s common understanding:

- Co-channel interference: The interference is from the aggressor to the victim in the same carrier.

  - Co-channel intra-subband interference: The interference is caused by transmission of the aggressor on some RBs in a subband in a carrier to reception of the victim on the same RBs in the same subband in the same carrier.

  - Co-channel inter-subband interference: The interference is caused by transmission of the aggressor in subband#1 in a carrier to reception of the victim in subband#2 in the same carrier, where the subband#1 and subband#2 are non-overlapping in frequency.

- Adjacent channel interference: The interference is from the aggressor in carrier#1 to the victim in carrier#2, where the carrier#1 and carrier#2 are adjacent carriers.

Note: ‘Co-channel’ here means ‘co-carrier’. ‘Adjacent-channel’ here means ‘adjacent-carrier’. A subband here means a number of continuous RBs, it has not to be the whole SBFD UL subband or DL subband, and it can be just a part of the SBFD UL subband or DL subband.

| Company | Comment |
| --- | --- |
| New H3C | We are fine with this proposal |
| ZTE | We are ok with this propsaol in genernal. Just to clarify, the definition here is just for evulation purpose and has no implication for the solution discussion, e.g., subband definition. |
| Huawei, HiSilicon | Support. The definition of subband is also discussed under AI 9.3.2. It is better to align with terminology. |
| Sony | @Moderator, thanks for clarifying intra-subband interference. This is much clearer. For inter-subband interference, I take it subband#1 and subband#2 are in different link direction (e.g. subband#1 is UL and subband#2 is DL and vice-versa). Perhaps we should make that clear: - Co-channel inter-subband interference: The interference is caused by transmission of the aggressor in subband#1 in a carrier to reception of the victim in subband#2 in the same carrier, where the subband#1 and subband#2 are non-overlapping in frequency and in different transmission link directions. |
| MediaTek | Support |
| InterDigital | Support. But, the Note should be revised in line with the agreement made in the most recent GTW session on AI9.3.2. It seems better we can simply copy from the agreement as follows: Note: ‘Co-channel’ here means ‘co-carrier’. ‘Adjacent-channel’ here means ‘adjacent-carrier’. A subband consists of 1 RB or a set of consecutive RBs for the same transmission direction here means a number of continuous RBs, it has not to be the whole SBFD UL subband or DL subband, and it can be just a part of the SBFD UL subband or DL subband. |
| Intel | We are generally OK with the proposal, and for the definition of sub-band we agree with other companies that terminology across AIs should be aligned, and the definition discussed under AI 9.3.2 could be adopted here. |
| Nokia, NSB | We support this proposal. |
| QC | Support, one minor comment. A subband here means a number of continuous contiguous RBs |
| Fujitsu | Support |
| DOCOMO | We are fine with the proposal. |
| LG | Support In addition, we would like to propose to use these terminologies for the discussion in 9.3.2 and 9.3.3. |
| vivo | We are fine with this proposal |
| CATT | We assume “on some RBs” and “on the same RBs” are added to address the case of unaligned subband configuration case. However, “in the same subband” is not clear to us. For example, for the following case, which subband are we referring to for co-channel intra-subband interference? In addition, the subband in the proposal seems to be different from the subband we agreed in AI9.3.2 as per the note, which may lead to potential confusion. We provide our suggested modifcations as below. For discussion purpose, consider the following as RAN1’s common understanding: - Co-channel interference: The interference is from the aggressor to the victim in the same carrier. - Co-channel intra-subband interference: The interference is caused by transmission of the aggressor on some RBs in a subband in a carrier to reception of the victim on the same RBs in the sameoverlapping subbands for different transmission directions in the same carrier. - Co-channel inter-subband interference: The interference is caused by transmission of the aggressor on some RBs in subband#1 in a carrier to reception of the victim on non-overlapping RBs in subband#2 in the same carrier, where the subband#1 and subband#2 are for different transmission directions non-overlapping in frequency. - Adjacent channel interference: The interference is from the aggressor in carrier#1 to the victim in carrier#2, where the carrier#1 and carrier#2 are adjacent carriers. Note: ‘Co-channel’ here means ‘co-carrier’. ‘Adjacent-channel’ here means ‘adjacent-carrier’. A subband here means a number of continuous RBs, it has not to be the whole SBFD UL subband or DL subband, and it can be just a part of the SBFD UL subband or DL subband. |
| Ericsson | We support the proposal in principle. |
| Spreadtrum | Support |
| Xiaomi | Support. |
| Samsung | Agree with InterDigital. Same definition should be used if possible. |
| Moderator | @Sony, the proposal has saying that ‘The interference is caused by transmission of the aggressor in subband#1 in a carrier to reception of the victim in subband#2 in the same carrier’, I think it is clear that the subband#1 is for transmission from aggressor perspective, and the subband #2 is for reception from victim perspective. In general, it is not strictly correct to say “the subband#1 and subband#2 are in different transmission link directions”, for example, if we assume the aggressor is gNB1 (gNB1 transmit in DL subband#1) and the victim is UE1 (UE1 receives in DL subband#2 but UE1 is in another cell, e.g., served by gNB2), then you can see here both subband#1 and subband#2 are DL but non-overlapping in frequency, in this case there could be gNB-UE co-channel inter-subband interference (although this type of interference is not what we intend to discuss). In a summary, this proposal is just to give a definition for ‘Co-channel inter-subband/intra-subband interference’ and ‘Adjacent channel interference’ so that we are on the same page when we are discussing issues. @CATT/all, I still think a clarification in the note regarding the subband definition here (which is used only for the definition of ‘intra-subband interference’ and ‘inter-subband interference’) is helpful. The following figure 1 and figure 2 are the illustrations of ‘intra-subband interference’ and ‘inter-subband interference’ respectively, and the ‘subbands’ used for them are also illustrated. From these figures, you can see that a ‘subband’ here just means a number of consecutive RBs, it has not to be the whole SBFD UL subband or SBFD DL subband as we defined in AI9.3.2, and it can be just a part of the SBFD UL subband or SBFD DL subband. I understand having different definitions for ‘subband’ is confusing and should be avoided, but since the terminology ‘intra-subband interference’ and ‘inter-subband interference’ have already been used in the SID objective, and we also need to use them frequently in the subsequent discussions, I think we need to clarify the definition of them and to have a common understanding in RAN1 on them, so I further clarified in the second note that ‘The definition of the subband here is only used for the definition of ‘intra-subband interference’ and ‘inter-subband interference’, and it will not be used for other purposes, e.g., SBFD subband configuration’. Hope that can reduce the impact of the different definitions on ‘subband’. Figure 1: The illustration of ‘intra-subband interference’ and the ‘subband’ used for it Figure 2: The illustration of ‘inter-subband interference’ and the ‘subband’ used for it |

### Updated proposal 3-2a:

Consider the following as the common understanding on potential interference types for SBFD operation:

- gNB self-interference (SI): Interference caused by DL transmission in DL subband in a carrier to UL reception in UL subband in the same carrier at the gNB side, where the DL subband and the UL subband are non-overlapping in frequency.

- gNB-UE co-channel intra-subband interference: This is the same as the legacy DL interference type in legacy TDD network with static TDD UL/DL configuration.

- UE-gNB co-channel intra-subband interference: This is the same as the legacy UL interference type in legacy TDD network with static TDD UL/DL configuration.

- (inter-cell) inter-site gNB-gNB co-channel intra-subband CLI: CLI caused by DL transmission of the aggressor gNB on some RBs in a subband in one carrier to UL reception of the victim gNB in a different site on the same RBs in the same subband in the same carrier.

- (inter-cell) co-site inter-sector co-channel intra-subband CLI: CLI caused by DL transmission of the aggressor gNB on some RBs in a subband in one carrier to UL reception of the victim gNB in another sector of the same site on the same RBs in the same subband in the same carrier.

- (inter-cell) UE-UE co-channel intra-subband CLI: CLI caused by UL transmission of the aggressor UE on some RBs in a subband in one carrier to DL reception of the victim UE on the same RBs in the same subband in the same carrier.

- (inter-cell) inter-site gNB-gNB co-channel inter-subband CLI: CLI caused by DL transmission of the aggressor gNB in DL subband in a carrier to UL reception of the victim gNB in a different site in UL subband in the same carrier, where the DL subband and the UL subband are non-overlapping in frequency.

- (inter-cell) co-site inter-sector co-channel inter-subband CLI: CLI caused by DL transmission of the aggressor gNB in DL subband in a carrier to UL reception of the victim gNB in another sector of the same site in UL subband in the same carrier, where the DL subband and the UL subband are non-overlapping in frequency.

- (intra-cell/inter-cell) UE-UE co-channel inter-subband CLI: CLI caused by UL transmission of the aggressor UE in UL subband in a carrier to DL reception of the victim UE in DL subband in the same cell or neighboring cell in the same carrier, where the UL subband and the DL subband are non-overlapping in frequency.

- gNB-gNB adjacent-channel CLI: CLI caused by DL transmission of the aggressor gNB in a carrier to UL reception of the victim gNB in another adjacent carrier.

  - This includes adjacent-channel CLI between gNBs in the same and different sectors of the same site, i.e., co-site intra and inter-sector adjacent-channel CLI.

- UE-UE adjacent-channel CLI: CLI caused by UL transmission of the aggressor UE in a carrier to DL reception of the victim UE in another adjacent carrier.

Note: Some of the interferences may not be used according to the deployment scenarios, e.g, whether the SBFD subband configurations are the same or different across gNBs.

| Company | Comment |
| --- | --- |
| New H3C | We are fine with this proposal |
| ZTE | OK |
| Huawei, HiSilicon | Generally fine with the proposal. On (inter-cell) co-site inter-sector co-channel intra-subband CLI, it is not clear whether there is a strong motivation to use different SBFD eceivestyn within a site. |
| Sony | Support. |
| MediaTek | Support |
| Intel | We are generally OK with the proposal, but we also share same view with Huawei that the motivation to consider and the impact of the co-site inter-sector co-channel intra-subband CLI are not clear. |
| Nokia, NSB | We support this proposal. |
| QC | Support. |
| Fujitsu | Support. |
| DOCOMO | We are fine with the proposal. |
| LG | Support In addition, we would like to propose to use these terminologies for the discussion in 9.3.2 and 9.3.3. |
| vivo | We are fine with this proposal and agree with ZTE that this is for evaluation purpose |
| CATT | See our comment for proposal 3-1a. |
| Ericsson | We support the proposal in principle. |
| Spreadtrum | We are generally fine with the proposal but have the same doubt as Huawei. |
| Xiaomi | Support. |
| Samsung | Fine with the proposal. Agree with Huawei, Intel and Spreadtrum that we cannot see strong motivation to configure different UL ssuband at co-site sectors. |
| Moderator | Updated |

### Updated proposal 3-3a:

For discussion of gNB self-interference modelling for the purposes of system level simulations (more detailed model may be needed for link level simulations), consider introducing ratio of self-interference (RSI) to represent the overall self-interference suppression capability of gNB by means of spatial isolation, subband frequency isolation, digital interference cancellation and beamform nulling/isolation, etc. RSI also takes into account the impact of Tx/Rx antenna element gain on self-interference. The RSI, denoted as αSIm,n,  can be defined as the ratio of the total power transmitted by gNB across all transmit chains on a frequency unit m (e.g., subband/RB/subcarrier m) in a SBFD carrier to the residual self-interference received by the same gNB on a single receiver chain on a different frequency unit n (e.g., another subband/RB/subcarrier n) in the same SBFD carrier.

- The following should also be asked to RAN4:

  - The value range of RSI for each frequency range. The RSI can be described per subband, per RB, or per subcarrier depending on the granularity of the frequency unit, and it is up to RAN4.

  - The dependency of RSI with the frequency distance between Tx frequency unit m and the Rx frequency unit n, and the dependency of RSI with the SBFD DL subband size, the number of RBs used for DL transmission.

  - The feasibility of the gNB self-interference modelling including factors such as blocking, AGC, TX/RX beamforming, Tx/Rx beam pair for FR1/FR2, etc.

  - The dependency of RSI with the number of transmit and receive chains

  - The separate self-interference cancellation values or value ranges of different cancellation methods, e.g, spatial isolation, subband frequency isolation, digital interference cancellation and beamform nulling/isolation, etc.

  - RAN1 will also provide the assumption on the SBFD subband configuration, FFS details

- FFS: whether RSI takes into account digital interference cancellation or not.

- FFS: details of gNB self-interference modelling, e.g., the gNB self-interference on a single receiver chain at UL subcarrier n can be modelled as

  - ISIn=m≠nm∈DL subbandISIm,n, wherein,

    - m is the DL subcarrier index.

    - dBISIm,n=Ptxm-dBαSIm,n

    - Ptxm is gNB’s DL transmission power across all transmit chains at subcarrier m (in dBm).

    - αSIm,n is the per-subcarrier-RSI.

  - FFS: the dependence of αSIm,non Tx/Rx beam-pair for FR1/FR2 especially for clutter echo.

  - FFS: consider a statistical clutter model based on statistics of clutter strength and AoA.

  - FFS: whether/how to consider the impact of locations of Tx/Rx antenna elements on RSI.

| Company | Comment |
| --- | --- |
| New H3C | We are fine with this proposal |
| ZTE | OK |
| Huawei, HiSilicon | There seems to be some overlapping between the different sub-bullets that are supposed to ask RAN4. The first subbullet seems to imply that what RAN1 is asking for is one value after considering different factors of the other subbullet. On “The feasibility of the gNB self-interference modelling including factors such as blocking, AGC, TX/RX beamforming, Tx/Rx beam pair for FR1/FR2, etc.”, is the intention to ask RAN4 about the eceivesty of the modeling or eceivesty of SI handling in general? On “The separate self-interference cancellation values or value ranges of different cancellation methods, e.g, spatial isolation, subband frequency isolation, digital interference cancellation and beamform nulling/isolation, etc.”, it seems to be overlapped with sub-bullet 2 and sub-bullet 3. |
| Sony | Support |
| MediaTek | Fine with the proposal. |
| Intel | We are generally fine with the proposal. However, on the sub-bullet “RAN1 will also provide the assumption on the SBFD subband configuration, FFS details” it would be good to clarify the intention, including which aspects are “FFS”, and whether these are for this meeting or subsequent discussions. If the intention is to simply share relevant RAN1 agreements on SBFD configuration from this meeting then we can just do that once relevant agreements are made, e.g., related to those in Section 5.2 without the bullet as proposed right now under list of questions to RAN4 to avoid confusion. At the minimum, this should be a note and without FFS. |
| Nokia, NSB | We support this proposal. |
| QC | We think it may better to ask RAN4 to detail the values of the RSI as separate numbers of spatial isolation, frequency isolation, beamform nulling/isolation, subband filter and digital cancellation instead of a single number. This will enable RAN1 to evaluate the impact of separate factors independently. αSI(n,m)= αSI-spatial+αSI-frequency (n,m)+αSI-beam+αSI-digtial Also, it may be good to ask RAN4 if it is possible to have a simple flat (non-frequency selective) model for the RSI and under which conditions this assumption is valid. - The dependency of RSI with the frequency distance between Tx frequency unit m and the Rx frequency unit n, and the dependency of RSI with the SBFD DL subband size, the number of RBs used for DL transmission. - Whether it is possible to simplify the RSI as frequency flat model. - The condition(s), e.g., minimum guard band and/or number of RBs in DL allocation, under which the dependency of the RSI on frequency can be ignored. |
| Fujitsu | Support |
| DOCOMO | We are fine with the proposal. |
| LG | We are fine with the proposal. |
| vivo | We are fine with this proposal and agree with ZTE that this is for evaluation purpose |
| CATT | The unit of ISIm,n should be dBm. Maybe it is better to have a subbullet for ISIm,n for definition and unit, similar as for Ptxm. |
| Ericsson | We also support Qualcomm’s comment about splitting the self-interference into various components that could be cancelled. This provides a clear framework on how much self-interference cancellation is attainable. |
| Xiaomi | Support. |
| Samsung | Geneally fine with the proposal. A few comments are - “the dependency of RSI with the SBFD DL subband size, the number of RBs used for DL transmission” can be further considered only if per-subband granularity is adopted. For per-subcarrier or per-RB graunularities, the dependency may not exist. - “FFS: consider a statistical clutter model based on statistics of clutter strength and AoA.” This is for evaluation assumptions, not interference model. We can discuss this separately. - What is difference between TX/RX beamforming and Tx/Rx beam pair for FR1/FR2? TX/RX beamforming can be removed. - “FFS: whether/how to consider the impact of locations of Tx/Rx antenna elements on RSI.” may be included in spatial isolation. RAN4 will answer and the feedback should be taken into account RAN1 SLS. So, we can remove this FFS. |
| Moderator | Updated based on companies’ comments |

### Updated proposal 3-4a:

For discussion of gNB-gNB or UE-UE co-channel inter-subband CLI modelling and adjacent-channel CLI modelling, consider the following aspects:

- Transmitter leakage due to non-linearity of the transmitter at the aggressor.

- Receiver selectivity due to non-linearity of the receiver at the victim.

The following questions should be asked to RAN4:

- For a specific DL RB(s) and UL RB(s) pair of gNB-gNB link, where the DL RB(s) and the UL RB(s) may be in the same carrier or in eceives carriers, and assuming the aggressor gNB transmits on the DL RB(s) and the victim gNB eceives on the UL RB(s),

  - How to model the DL RB(s) to UL RB(s) leakage interference at the gNB transmitter?

  - How to model the DL RB(s) to UL RB(s) interference due to receiver selectivity at the gNB receiver?

  - How to model the above interferences due to leakage at the transmitter and receiver selectivity at the receiver when the gNBs are from different sectors of the same site in the same carrier, or when the gNBs are from the same/different sector(s) of the same site in adjacent carriers?

- For a specific DL RB(s) and UL RB(s) pair of UE-UE link, where the DL RB(s) and the UL RB(s) may be in the same carrier or in eceives carriers, and assuming the aggressor UE transmits on the UL RB(s) and the victim UE eceives on the DL RB(s),

  - How to model the UL RB(s) to DL RB(s) leakage interference at the UE transmitter?

  - How to model the UL RB(s) to DL RB(s) interference due to receiver selectivity at the UE receiver?

- FFS: Usage of the above model provided by RAN4 in the evaluation

| Company | Comment |
| --- | --- |
| New H3C | We are fine with this proposal |
| ZTE | OK |
| Huawei, HiSilicon | Support. There are two typos in bullet 3 and bullet 4, “adjacent”. |
| Sony | Support |
| MediaTek | Support |
| Intel | We are fine with the proposal. |
| Nokia, NSB | We support this proposal. “ in the proposal, “in the adjacent carrier” is turned into “in 102eceives carriers”, so please FL to check this error. |
| QC | Few comments: - It could be better to split proposal for co-channel and adjacent channels for further clarity. - There is no mention of DL subband or UL subbaand in the description of the interference leakage while main bullet metion ‘inter-subband CLI modeling’. - Receiver selectivity is a receiver's capability to received desired signal in the presence of other unwanted interfering signal, in that case is the cross-link interference. It is not clear what is meant by receiver selectivity due non-lineairty of the victim receiver. - The third subbulet of third bullet should be further clarified that RAN1 requires how to model inter-subband CLI for the following two cases (using definition from earlier proposal) - (inter-cell) inter-site gNB-gNB co-channel inter-subband CLI - (inter-cell) co-site inter-sector co-channel inter-subband CLI |
| Fujitsu | Support |
| DOCOMO | We are fine with the proposal. |
| LG | We are fine with the proposal. |
| vivo | We are fine with the proposal |
| CATT | We are fine with the proposal. Same as Nokia. |
| Ericsson | We are ok with the proposal in principle. However, we need clarification on the first two bullet points on non-linearities. Furthermore, we think that the adjacent channel CLI and co-channel CLI needs to be modelled in a similar fashon as self-interference in 3-1a. On that regard, we think that the excellent description of the co-channel and adjacent channel CLI in the Moderator’s contribution (updated proposal 3-4 and 3-5 in second round discussions) could be used as the starting point. |
| Xiaomi | Support. |
| Samsung | Several comments are below - Is it intedned for LLS evaluation or Link budget analysis or SLS evaluation? “FFS: Usage of the above model provided by RAN4 in the evaluation” could be first clarified before asking LS to RAN4. The proposal describes per-RB or per-RB set pair. But, as in SLS, it is up to RAN4 on frequency units. The same approach should be used to avoid any misleadings. |
| Moderator | Updated based on companies’ comments. Let’s first focus on inter-subband CLI. |

## 4.6 4th Round Proposals (closed)

### Updated proposal 3-1b:

For discussion purpose, consider the following as RAN1’s common understanding:

- Co-channel interference: The interference is from the aggressor to the victim in the same carrier.

  - Co-channel intra-subband interference: The interference is caused by transmission of the aggressor on some RBs in a subband in a carrier to reception of the victim on the same RBs in the same subband in the same carrier.

  - Co-channel inter-subband interference: The interference is caused by transmission of the aggressor in subband#1 in a carrier to reception of the victim in subband#2 in the same carrier, where the subband#1 and subband#2 are non-overlapping in frequency.

- Adjacent channel interference: The interference is from the aggressor in carrier#1 to the victim in carrier#2, where the carrier#1 and carrier#2 are adjacent carriers.

Note: ‘Co-channel’ here means ‘co-carrier’. ‘Adjacent-channel’ here means ‘adjacent-carrier’.

Note: A ‘subband’ here just means a number of consecutive RBs, it has not to be the whole SBFD UL subband or SBFD DL subband as we defined in AI9.3.2, and it can be just a part of the SBFD UL subband or SBFD DL subband. The definition of the subband here is only used for the definition of ‘intra-subband interference’ and ‘inter-subband interference’, and it will not be used for other purposes, e.g., SBFD subband configuration.

| Company | Comment |
| --- | --- |
| InterDigital | We now understood what the moderator’s original intension on the second Note (regarding the definition of “subband”) after looking at the Figures 1&2 provided by the moderator in the closed 3rd round proposal in s4.5. But, this new “second-level” subband definition is rather confusing and not needed, as that is just a temporal term here and is basically duplicated with the phrase of “the same RBs” in its meaning for intra-subband CLI. Then, we can simplify the wording by removing “in a subband” and “in the same subband” for the intra-subband bullet, and make the Note be exactly copied from what was agreed for ‘subband’ as follows: … - Co-channel intra-subband interference: The interference is caused by transmission of the aggressor on some RBs in a subband in a carrier to reception of the victim on the same RBs in the same subband in the same carrier. … Note: A ‘subband’ consists of 1 RB or a set of consecutive RBs for the same transmission direction here just means a number of consecutive RBs, it has not to be the whole SBFD UL subband or SBFD DL subband as we defined in AI9.3.2, and it can be just a part of the SBFD UL subband or SBFD DL subband. The definition of the subband here is only used for the definition of ‘intra-subband interference’ and ‘inter-subband interference’, and it will not be used for other purposes, e.g., SBFD subband configuration. |
| Intel | We are Ok with the proposal in general but think that the Note is unnecessarily complicated – the update from IDC is sufficient and preferred. |
| CATT | Thanks moderator for the clarification. We share the same view as IDC that “second-level” subband definition is confusing as we commented in 3rd round discussion and is better to be avoided. IDC’s updated may be fine for intra-subband interference, but “where the subband#1 and subband#2 are non-overlapping in frequency” for inter-subband interference still needs to be revised. Our proposal in 3rd round was as follows: - Co-channel intra-subband interference: The interference is caused by transmission of the aggressor on some RBs in a subband in a carrier to reception of the victim on the same RBs in the sameoverlapping subbands for different transmission directions in the same carrier. - Co-channel inter-subband interference: The interference is caused by transmission of the aggressor on some RBs in subband#1 in a carrier to reception of the victim on non-overlapping RBs in subband#2 in the same carrier, where the subband#1 and subband#2 are for different transmission directions non-overlapping in frequency. If the intention is to not only include CLI but also interference in the same link direction, we can consider: - Co-channel intra-subband interference: The interference is caused by transmission of the aggressor on some RBs in a subband in a carrier to reception of the victim on the same RBs in the sameoverlapping subbands in the same carrier. - Co-channel inter-subband interference: The interference is caused by transmission of the aggressor on some RBs in subband#1 in a carrier to reception of the victim on non-overlapping RBs in subband#2 in the same carrier, where the subband#1 and subband#2 are non-overlapping in frequency. We are also open to the current proposal as long as companies have the same understanding on the definition of subband. |
| Ericsson | We support the proposal in principle. However, we have the following clarification question. Question: In the following scenario (different size UL subband between aggressor and victim), is the illustrated CLI classified as inter- or intra-subband? If it is classified as "inter-subband", then the wording "where the subband#1 and subband#2 are non-overlapping in frequency" seems problematic, since clearly the 'D' subband of the aggressor and the 'U' subband of the victim are indeed overlapping in frequency. If it is classified as "intra-subband", then the wording "in the same subband" seems problematic, since the aggressor subband is 'D' and the victim subband is 'U', hence one can interpret these as different subbands |
| ZTE | We are generally fine with the proposal. To avoid any further confusion, it would be better to align the definition of subband. |
| QC | The figures from the FLs in the previous round are sufficient to clarify intra-subband CLI and inter-subband CLI and can replace the note. |
| Xiaomi | We are fine with FL proposal. Per our reading on the proposal, the case raised by Ericsson is intra-subband CLI. We think the note used to explain this case. |
| Samsung | We are generally fine with the proposal. Regarding the Erisson’s question, the interference is intra-subband interference. From the figure from the FL in the last comment, what we understand is the subband in the proposal is the physical frequency segment (set of RBs), which is not same as the subband we defined in AI9.3.2. So, in Ericsson’s figure, there are 5 segments and intra-subband interference can be defined in the second/forth segments. |
| Huawei, HiSilicon | We are fine with definition. |
| vivo | We are fine with the proposal, either with or without the last note. Regarding Ercisson’s question, our understanding is that the interference is intra-subband interference, according to the current definition in the proposal. |
| Spreadtrum | We are fine with the proposal. And agree with Xiaomi and Samsung about Ericsson’s case which is intra-subband CLI since CLI happened in the same RBs. |
| Nokia, NSB | We support the proposal. |
| MediaTek | Support the proposal in principle. |
| Sony | We share similar views with CATT and would be fine with the clarifications provided by CATT’s modified proposal with some minor editorials, i.e.: - Co-channel intra-subband interference: The interference is caused by transmission of the aggressor on some RBs in a subband in a carrier to reception of the victim on the same RBs in sameoverlapping subbands of different transmission directions in the same carrier. - Co-channel inter-subband interference: The interference is caused by transmission of the aggressor on some RBs in subband#1 in a carrier to reception of the victim on non-overlapping RBs in subband#2 in the same carrier, where the subband#1 and subband#2 are for different transmission directions non-overlapping in frequency. |

### Updated proposal 3-2b:

For discussion purpose, consider the following as the common understanding in RAN1 on the definition of interference types for SBFD operation:

- gNB self-interference (SI): Interference caused by DL transmission in DL subband in a carrier to UL reception in UL subband in the same carrier at the gNB side, where the DL subband and the UL subband are non-overlapping in frequency.

- gNB-UE co-channel intra-subband interference: This is the same as the legacy DL interference type in legacy TDD network with static TDD UL/DL configuration.

- UE-gNB co-channel intra-subband interference: This is the same as the legacy UL interference type in legacy TDD network with static TDD UL/DL configuration.

- (inter-cell) inter-site gNB-gNB co-channel intra-subband CLI: CLI caused by DL transmission of the aggressor gNB on some RBs in a subband in one carrier to UL reception of the victim gNB in a different site on the same RBs in the same subband in the same carrier.

- (inter-cell) co-site inter-sector co-channel intra-subband CLI: CLI caused by DL transmission of the aggressor gNB on some RBs in a subband in one carrier to UL reception of the victim gNB in another sector of the same site on the same RBs in the same subband in the same carrier.

- (inter-cell) UE-UE co-channel intra-subband CLI: CLI caused by UL transmission of the aggressor UE on some RBs in a subband in one carrier to DL reception of the victim UE on the same RBs in the same subband in the same carrier.

- (inter-cell) inter-site gNB-gNB co-channel inter-subband CLI: CLI caused by DL transmission of the aggressor gNB in DL subband in a carrier to UL reception of the victim gNB in a different site in UL subband in the same carrier, where the DL subband and the UL subband are non-overlapping in frequency.

- (inter-cell) co-site inter-sector co-channel inter-subband CLI: CLI caused by DL transmission of the aggressor gNB in DL subband in a carrier to UL reception of the victim gNB in another sector of the same site in UL subband in the same carrier, where the DL subband and the UL subband are non-overlapping in frequency.

- (intra-cell/inter-cell) UE-UE co-channel inter-subband CLI: CLI caused by UL transmission of the aggressor UE in UL subband in a carrier to DL reception of the victim UE in DL subband in the same cell or neighboring cell in the same carrier, where the UL subband and the DL subband are non-overlapping in frequency.

- gNB-gNB adjacent-channel CLI: CLI caused by DL transmission of the aggressor gNB in a carrier to UL reception of the victim gNB in another adjacent carrier.

  - This includes adjacent-channel CLI between gNBs in the same and different sectors of the same site, i.e., co-site intra and inter-sector adjacent-channel CLI.

- UE-UE adjacent-channel CLI: CLI caused by UL transmission of the aggressor UE in a carrier to DL reception of the victim UE in another adjacent carrier.

Note: Some of the interferences may not be used according to the deployment scenarios, e.g, whether the SBFD subband configurations are the same or different across gNBs.

Note: This does not imply we need to consider all the above interference types in evaluation for SBFD.

| Company | Comment |
| --- | --- |
| Intel | We are OK with the proposal. |
| CATT | We support the proposal in general except it is related to update for proposal 3-1b. If proposal 3-1b is further updated, this proposal needs to be updated accordingly. |
| Ericsson | We are ok with the proposal in principle. |
| ZTE | We are OK with the proposal. |
| QC | Support |
| Samsung | Fine with the proposal |
| Huawei, HiSilicon | We are fine with the proposal. |
| vivo | We are fine with the proposal. |
| Spreadtrum | Support. |
| Nokia, NSB | We support the proposal. |
| MediaTek | Support the proposal in principle. |
| Sony | Support. |

### Updated proposal 3-3b:

Regarding gNB self-interference modelling for system level simulation purpose (more detailed model may be needed for link level simulation), consider introducing ratio of self-interference (RSI) to represent the overall self-interference suppression capability of gNB by means of spatial isolation, subband frequency isolation, digital interference cancellation and beamform nulling/isolation, etc. RSI also takes into account the impact of Tx/Rx antenna element gain on self-interference. The RSI, denoted as αSIm,n,  can be defined as the ratio of the total power transmitted by gNB across all transmit chains on a frequency unit m (e.g., subband/RB/subcarrier m) in a SBFD carrier to the residual self-interference received by the same gNB on a single receiver chain on a different frequency unit n (e.g., another subband/RB/subcarrier n) in the same SBFD carrier.

- FFS: details of gNB self-interference modelling using RSI in SLS. As one example based on per-RB-RSI, the gNB self-interference on a single receiver chain at UL RB n can be modelled as

  - ISIn=m≠nm∈DL subbandISIm,n, wherein,

    - dBmISIm,n=Ptxm-dBαSIm,n

    - ISIm,nis the gNB self-interference on a single receiver chain at UL RB n caused by DL transmission on DL RB m.

    - m is the DL RB index in DL subbands.

    - Ptxm is gNB’s DL transmission power across all transmit chains at RB m (in dBm).

    - αSIm,n is the per-RB-RSI.

  - FFS: consider a statistical clutter model based on statistics of clutter strength and AoA.

- The following should be asked to RAN4:

  - What is the value range of RSI αSI(n,m) for each frequency range, and under what assumptions on the self-interference suppression means the value range of RSI is provided?

    - RAN1 understands the RSI can be described per subband, per RB, or per subcarrier depending on the granularity of the frequency unit, and it is up to RAN4 to provide the RSI in which granularity.

  - Whether it is possible for RAN4 to provide RAN1 the respective capabilities of different self-interference suppression means? e.g., is it possible to provide the separate numbers of spatial isolation, subband frequency isolation, beamform nulling/isolation and digital cancellation, etc., as below?

    - αSI(n,m)= αSI-spatial+αSI-frequency (n,m)+αSI-beam+αSI-digtial +…

      - αSI-spatial denotes the spatial isolation.

      - αSI-frequency (n,m) denotes the suband frequency isolation between the Tx frequency unit m and the Rx frequency unit n.

      - αSI-beam denotes the beamform nulling/isolation.

      - αSI-digtial denotes the digital cancellation capability.

  - Whether it is possible to simplify the RSI as frequency flat model, and under which condition(s) the dependency of the RSI on frequency can be ignored?

  - The feasibility of provided value range of RSI regarding factors such as blocking, AGC, etc.

  - Does RSI have any dependency with the following factors or any other factors? What are the dependencies?

    - gNB’s antenna aspects, e.g., the assumed antenna architecture, the number of transmit chains and receive chains, etc.

    - Frequency aspects, e.g., the frequency distance between the Tx frequency unit m and the Rx frequency unit n, the number of RBs allocated for DL transmission, etc.

    - Beam aspects, e.g., Tx/Rx beam-pair for FR1/FR2 especially for clutter echo, etc.

- Note: RAN1’s consideration on the frequency locations and sizes of SBFD DL subband and SBFD UL subband assumed in SBFD operation can be provided to RAN4.

### Initial proposal 3-3c (newly added):

For the purposes of link-level evaluations, an overall "net effect" model is needed that captures key components of the gNB transmit chain including crest-factor reduction (CFR) + digital pre-distortion (DPD) + power amplifier (PA), while satisfying the -45 dBc requirement on ACLR. For link-level simulations, the model should be sufficiently detailed to allow accurate generation of the self-interference signals to be cancelled.

The following should be asked to RAN4

- For accurate generation of the self-interference signals at the gNB for the purposes of link level evaluation of self-interference cancellation, recommend a model to be used for capturing the overall "net" effect of key gNB transmit chain components, i.e., CFR + DPD + PA.

| Company | Comment |
| --- | --- |
| Intel | We are OK with the proposal. |
| CATT | We are fine with the proposal. |
| Ericsson | We are ok with the proposal in principle. However, we would like to highlight one of the issues we raised during the 2nd round of discussions. One of the key aspects of feasibility evaluation for SBFD is the degree to which self-interference can be cancelled, and proper evaluation of this will require link simulations. This in turn requires a detailed model that captures the overall effect of key-components in the gNB transmit chain, i.e., crest-factor reduction (CFR) + DPD + PA. For CFR, we think a simple model can be used, e.g., hard clipping + filtering to capture the essential behavior. With such an overall model for CFR + DPD + PA, the self-interference can be generated in a link simulation accurately, and self-interference cancellation can be properly studied. This will require feedback from RAN4, and we suggest that the LS to RAN4 includes a question on this. So far, the above proposal only covers the requirements from a system simulation perspective; link level is missing. Hence, we suggest the following addition to the proposal to make sure both system level and link level are covered: Initial proposal 3-3b1: For the purposes of link-level evaluations, an overall "net effect" model is needed that captures key components of the gNB transmit chain including crest-factor reduction (CFR) + digital pre-distortion (DPD) + power amplifier (PA), while satisfying the -45 dBc requirement on ACLR. For link-level simulations, the model should be sufficiently detailed to allow accurate generation of the self-interference signals to be cancelled. - The following should be asked to RAN4 - For accurate generation of the self-interference signals at the gNB for the purposes of link level evaluation of self-interference cancellation, recommend a model to be used for capturing the overall "net" effect of key gNB transmit chain components, i.e., CFR + DPD + PA. |
| ZTE | We are OK with the proposal. |
| QC | Support the FL proposal. One minor editorial comment below - αSI-beam denotes the beamform nulling or beam isolation |
| Xiaomi | We are fine with th proposal. One modification suggestion: the last third subbulet already implies thiere is dependency on frequency for RSI. - ‘Whether it is possible to simplify the RSI as frequency flat model, and under which condition(s) the dependency of the RSI on frequency can be ignored?’ Accordingly, the last second sub-subbullet can be moved to the above last third subbullet for consisitent. |
| Samsung | Fine with the proposal |
| Moderator | I added a new proposal 3-3c based on Ericsson’s comment to collect companies’ views. |
| Huawei, HiSilicon | On 3-3b, we are fine with the proposal in principle. However, the following subbullet is not needed since there is another subbullet discussing the “frequency aspects”. Whether it is possible to simplify the RSI as frequency flat model, and under which condition(s) the dependency of the RSI on frequency can be ignored? On 3-3c, we would like to understand the reasoning for the “-45 dBc requirement on ACLR”. In addition, we would like to clarify how to make use of the model for LLS since our understanding is that self-interference will be largely dependent on implementation. |
| vivo | We are fine with the proposal 3-3b. |
| Nokia, NSB | We support the proposal 3-3b. The newly added part need further discussion. |
| MediaTek | Support the proposal 3-3b in principle. |
| Ericsson 2 | We support the proposal 3-3b and 3-3c. Response to Huawei: the -45 dBC is the ACLR requirement currently, and considering SBFD will coexist with legacy networks, this requirement needs to be met regardless. If self-interference is largely based on implementation, how can we guarantee that X dB of SIC is attainable? Company A may provide X dB SIC and company B may provide Y dB SIC amount which will cause coexistence issues, also to legacy networks. Therefore, we need a clear framework on how much SIC is attainable based on the factors we described in our comment for 2nd round discussion proposal 3-3. We need to perform link level simulations for this purpose and need RAN4 to help us with the net-effect model. |
| Sony | Support Proposal 3-3b. On Proposal 3-3c, isn’t the “net effect” for CFR + DPD + PA, already factored into the RSI that we are asking RAN4 to provide? Are these just additional details but really we are only interested in RSI in RAN1 for system level simulation purpose? |
| Moderator | Proposal 3-3c has been agreed as below: Updated proposal 3-3c: Regarding gNB self-interference modelling for system level simulation purpose, consider introducing ratio of self-interference (RSI) to represent the overall self-interference suppression capability of gNB by means of spatial isolation, subband frequency isolation, digital interference cancellation and beamform nulling/isolation, etc. RSI also takes into account the impact of Tx/Rx antenna element gain on self-interference. The RSI, denoted as αSIm,n, can be defined as the ratio of the total power transmitted by gNB across all transmit chains on a frequency unit m (e.g., subband/RB/subcarrier m) in a SBFD carrier to the residual self-interference received by the same gNB on a single receiver chain on a different frequency unit n (e.g., another subband/RB/subcarrier n) in the same SBFD carrier. - FFS: Model for link level simulations and relevant questions to ask RAN4 - FFS: details of gNB self-interference modelling using RSI in SLS. As one example based on per-RB-RSI, the gNB self-interference on a single receiver chain at UL RB n can be modelled as - ISIn=m≠nm∈DL subbandISIm,n, wherein, - dBmISIm,n=Ptxm-dBαSIm,n - ISIm,nis the gNB self-interference on a single receiver chain at UL RB n caused by DL transmission on DL RB m. - m is the DL RB index in DL subbands. - Ptxm is gNB’s DL transmission power across all transmit chains at RB m (in dBm). - αSIm,n is the per-RB-RSI. - FFS: consider a statistical clutter model based on statistics of clutter strength and AoA. - The following should be asked to RAN4: - What is the value range of RSI αSI(n,m) for each frequency range, and under what assumptions on the self-interference suppression means the value range of RSI is provided? - RAN1 understands the RSI can be described per subband, per RB, or per subcarrier depending on the granularity of the frequency unit, and it is up to RAN4 to provide the RSI in which granularity. - Whether it is possible for RAN4 to provide RAN1 the respective capabilities of different self-interference suppression means? e.g., is it possible to provide the separate estimates for spatial isolation, subband frequency isolation, beamform nulling/isolation, and digital cancellation, etc., as below? - dB(αSI(n,m))= dB(αSI-spatial)+dB(αSI-frequency (n,m))+dB(αSI-beam)+dB(αSI-digtial) +… (Fei will change) - αSI-spatial denotes the spatial isolation. - αSI-frequency (n,m) denotes the suband frequency isolation between the Tx frequency unit m and the Rx frequency unit n. - αSI-beam denotes the beamform nulling or beam isolation. - αSI-digtial denotes the digital cancellation capability. - Whether it is possible to simplify the RSI as frequency flat model, and under which condition(s) the dependency of the RSI on frequency can be ignored? - The feasibility of provided value range of RSI regarding factors such as blocking, AGC, etc. - Does RSI have any dependency with the following factors or any other factors? What are the dependencies? - gNB’s antenna aspects, e.g., the assumed antenna architecture, the number of transmit chains and receive chains, etc. - Frequency aspects, e.g., the frequency distance between the Tx frequency unit m and the Rx frequency unit n, the number of RBs allocated for DL transmission, etc. - Beam aspects, e.g., Tx/Rx beam-pair for FR1/FR2 especially for clutter echo, etc. - Note: RAN1’s consideration on the frequency locations and sizes of SBFD DL subband and SBFD UL subband assumed in SBFD operation can be provided to RAN4. |

### Updated proposal 3-4bc:

For discussion of gNB-gNB and UE-UE co-channel inter-subband CLI modelling in system level simulation, consider the following two aspects RAN1 understands at least the following two aspects need to be considered:

- Aspect 1: The unwanted emissions due to Tx non-linearity at the transmitter of the aggressor from the allocated RBs to the non-allocated RBs in the same carrier, similar as the In-Band Emission defined for UE.

- Aspect 2: The receiver selectivity at the victim to receive the desired signal in the allocated RBs in the presence of the unwanted signals at the non-allocated RBs. The unwanted signals received by the receiver of the victim from the non-allocated RBs when the receiever receives wanted signals in the allocated RBs, similar as the In-channel selectivity defined for BS.

The following questions should be asked to RAN4:

- Whether it is feasible to consider the above two aspects for gNB-gNB and UE-UE co-channel inter-subband CLI modelling in system level simulation? Are there any other aspects should also be taken into account?

- For a specific pair of DL frequency unit m (e.g., subband/RB m) and UL frequency unit n (e.g., subband/RB n) of gNB-gNB link, where the DL frequency unit m and UL frequency unit n are in the same carrier and non-overlapping in frequency, and assuming the aggressor gNB transmits on the DL frequency unit m and the victim gNB receives on the UL frequency unit n,

  - How to model the interference from DL frequency unit m to UL frequency unit n due to Aspect 1 (defined above) at the gNB transmitter?

  - How to model the interference from DL frequency unit m to UL frequency unit n due to Aspect 2 (defined above) at the gNB receiver?

  - How to model the above interferences for the following two cases:

    - inter-site gNB-gNB co-channel inter-subband CLI

    - co-site inter-sector co-channel inter-subband CLI

- For a specific pair of DL frequency unit m (e.g., subband/RB m) and UL frequency unit n (e.g., subband/RB n) of UE-UE link, where the DL frequency unit m and UL frequency unit n are in the same carrier and non-overlapping in frequency, and assuming the aggressor UE transmits on the UL frequency unit n and the victim UE receives on the DL frequency unit m,

  - How to model the interference from UL frequency unit n to DL frequency unit m due to Aspect 1 (defined above) at the UE transmitter?

  - How to model the interference from UL frequency unit n to DL frequency unit m due to Aspect 2 (defined above) at the UE receiver?

FFS: Usage of the above model provided by RAN4 in the evaluation

### Updated proposal 3-5b (newly added):

Regarding gNB-gNB and UE-UE adjacent-channel CLI modelling for system level simulation, the ACIR in TR38.828 can be used as the starting point.

- RAN1 understands the ACIR in TR38.828 is defined as the ratio of the power transmitted by the aggressor gNB/UE on one carrier to the total interference received by the victim gNB/UE on the adjacent carrier, i.e., the ACIR in TR38.828 is described per carrier.

- For SBFD operation a similar interference ratio as ACIR but in the subband of the adjacent carrier, with finer granularity (e.g., per RB or per subband) needs to be considered. For example, we can define gNB-gNB per-RB-ACIR (γgNB-gNBm,n ) and UE-UE per-RB-ACIR ( γUE-UEm,n) as the ratio of the power transmitted by the aggressor gNB/UE on RB m in one carrier to the interference received by the victim gNB/UE on another RB n in the adjacent carrier.

- Details of inter-site gNB-gNB adjacent-channel CLI modelling, i.e, the inter-site gNB-gNB adjacent-channel CLI from aggressor gNB A''' to victim gNB A on UL RB n can be modelled by Iadjacent-channel-CLIn,A'''→A as below:

Iadjacent-channel-CLIn,A'''→A=m∈DL RBs in adjacent channelIadjacent-channel-CLIm,n,A'''→A,

dBmIadjacent-channel-CLIm,n,A'''→A=Ptxm,A'''+CLA'''→A-dBγgNB-gNBm,n.

wherein,

  - Iadjacent-channel-CLIm,n,A'''→A is the gNB-gNB adjacent-channel CLI of victim gNB A on UL RB n caused by DL transmission of aggressor gNB A''' on RB m in adjacent carrier.

  - Ptxm,A''' is the DL transmission power of aggressor gNB A''' on RB m in adjacent channel (in dBm).

  - CLA'''→A is the coupling loss between aggressor gNB A''' and victim gNB A, including pathloss, penetration loss, shadow fading and Tx/Rx antenna gain.

  - γgNB-gNBm,n is the gNB-gNB per-RB-ACIR.

- Details of UE-UE adjacent-channel CLI modelling, i.e, the UE-UE adjacent-channel CLI from aggressor UE B''' to victim UE B on DL RB n can be modelled by Iadjacent-channel-CLIn,B'''→B as below:

Iadjacent-channel-CLIn,B'''→B=m∈UL RBs in adjacent channelIadjacent-channel-CLIm,n,B'''→B,

dBmIadjacent-channel-CLIm,n,B'''→B=Ptxm,B'''+CLB'''→B-dBγUE-UEm,n.

wherein,

  - Iadjacent-channel-CLIm,n,B'''→B is the UE-UE adjacent-channel CLI of victim UE B on UL RB n caused by UL transmission of aggressor UE B''' on RB m in adjacent carrier.

  - Ptxm,B''' is the UL transmission power of aggressor UE B''' on RB m in adjacent channel (in dBm).

  - CLB'''→B is the coupling loss between aggressor UE B''' and victim UE B, including pathloss, penetration loss, shadow fading and Tx/Rx antenna gain.

  - γUE-UEm,n is the UE-UE per-RB-ACIR.

- The following need to be asked to RAN4:

  - What is the value range of gNB-gNB per-RB-ACIR (γgNB-gNBm,n ) and UE-UE per-RB-ACIR ( γUE-UEm,n) as described above for each frequency range?

  - Does gNB-gNB and UE-UE per-RB-ACIR have any dependency with the following factors or any other factors? What are the dependencies?

    - the frequency distance between the Tx RB m and the Rx RB n, etc.

  - Whether it is feasible to use the inter-site gNB-gNB adjacent-channel CLI modelling as described above?

  - How to model gNB-gNB adjacent-channel CLI when the two gNBs are from the same sector of the same site in adjacent carriers?

  - How to model gNB-gNB adjacent-channel CLI when the two gNBs are from different sectors of the same site in adjacent carriers?

  - Whether it is feasible to use the UE-UE adjacent-channel CLI modelling as described above?

| Company | Comment |
| --- | --- |
| Intel | As for the aspect 1 and aspect 2, how to model the interference is yet not clear and this depends on, e.g., Tx/Rx filtering. Therefore, we believe that it is not accurate to say “similar as the In-Band Emission defined for UE”, or “similar as the In-channel selectivity defined for BS”, since this is based on wideband filtering. With that said, these aspects should be part of questions to RAN4. |
| CATT | Similar as Intel, we think whether aspect 1 and 2 are “similar as the In-Band Emission defined for UE”, and “similar as the In-channel selectivity defined for BS” should be discussed in RAN4. |
| Ericsson | We are not sure why the adjacent CLI modelling was removed from the updated proposal. When we send an LS to RAN4, we need to include the clarifications on adjacent channel CLI modelling as well. Therefore, although we can split the proposal to co-channel and adjacent channel CLI modelling, we need both to be discussed. Therefore, we propose to split the existing proposals as below. In the 3rd round discussions, we agreed wilth comments from other companies that it was not clear what was meant by "receiver selectivity due to non-linearity. Furthermore, both blocking and dynamic range are crucial factors that should not be overlooked. Hence, we suggest the following revisions. ### Updated proposal 3-4b: (Modified by Ericsson) For discussion of gNB-gNB and UE-UE co-channel inter-subband CLI modelling in system level simulation, consider the following two aspects: - Aspect 1: The unwanted emissions at the transmitter of the aggressor from the allocated RBs to the non-allocated RBs in the same carrier, similar as the In-Band Emission defined for UE. - Aspect 2: The unwanted signals received by the receiver of the victim from the non-allocated RBs when the receiever receives wanted signals in the allocated RBs, similar as the In-channel selectivity defined for BS. The following questions should be asked to RAN4: - For a specific pair of DL frequency unit m (e.g., subband/RB m) and UL frequency unit n (e.g., subband/RB n) of gNB-gNB link, where the DL frequency unit m and UL frequency unit n are in the same carrier and non-overlapping in frequency, and assuming the aggressor gNB transmits on the DL frequency unit m and the victim gNB receives on the UL frequency unit n, - How to model the interference from DL frequency unit m to UL frequency unit n due to Aspect 1 (defined above) at the gNB transmitter? - How to model the interference from DL frequency unit m to UL frequency unit n due to Aspect 2 (defined above) at the gNB receiver? - How to model the above interferences for the following two cases: - inter-site gNB-gNB co-channel inter-subband CLI - co-site inter-sector co-channel inter-subband CLI - For a specific pair of DL frequency unit m (e.g., subband/RB m) and UL frequency unit n (e.g., subband/RB n) of UE-UE link, where the DL frequency unit m and UL frequency unit n are in the same carrier and non-overlapping in frequency, and assuming the aggressor UE transmits on the UL frequency unit n and the victim UE receives on the DL frequency unit m, - How to model the interference from UL frequency unit n to DL frequency unit m due to Aspect 1 (defined above) at the UE transmitter? - How to model the interference from UL frequency unit n to DL frequency unit m due to Aspect 2 (defined above) at the UE receiver? FFS: Usage of the above model provided by RAN4 in the evaluation ### Updated proposal 3-5b: (Modified by Ericsson) For discussion of gNB-gNB or UE-UE co-channel inter-subband CLI modelling and adjacent-channel CLI modelling, consider the following aspects: - Transmitter leakage due to non-linearity of the transmitter at the aggressor. - Receiver selectivity, blocking, dynamic range, and due to non-linearity of the receiver at the victim. The following questions should be asked to RAN4: - For a specific DL RB(s) and UL RB(s) pair of gNB-gNB link, where the DL RB(s) and the UL RB(s) may be in the same carrier or in eceives the adjacent carrier, and assuming the aggressor gNB transmits on the DL RB(s) and the victim gNB receives on the UL RB(s), - How to model the DL RB(s) to UL RB(s) leakage interference at the gNB transmitter? - How to model the DL RB(s) to UL RB(s) interference due to receiver selectivity and blocking at the gNB receiver? - How to model non-linearity of the gNB receiver? - How to model the above interferences due to leakage at the transmitter and receiver selectivity at the receiver when the gNBs are from different sectors of the same site in the same carrier, or when the gNBs are from the same/different sector(s) of the same site in adjacent carriers? |
| ZTE | We prefer the previous version. We have the same concern for this proposal as Intel. |
| QC | We are generally fine with the proposal. The description of Aspect 1 and Aspect 2 could be further clarified, for example. - Aspect 1: The unwanted emissions due to Tx non-linearitiy at the transmitter of the aggressor from the allocated RBs to the non-allocated RBs in the same carrier, similar as the In-Band Emission defined for UE. - Aspect 2: The victim receiver selectivity to receive the desired signal in the allocated RBs in the presence of the unwanted signals at the non-allocated RBs. |
| Xiaomi | Share similar concern as Intel/CATT/ZTE. |
| Samsung | We are fine with the proposal with removing “, similar as the In-Band Emission defined for UE” and “, similar as the In-channel selectivity defined for BS” |
| Moderator | Based on comments so far, I updated proposal 3-4b to 3-4c. Regarding gNB-gNB and UE-UE adjacent-channel CLI modelling for system level simulation, I think we usually use ACIR in SLS, maybe we can directly ask RAN4 to provide the value or value range of ACIR with finer granularity (since in SBFD operation the UL subband with RB level size may be located in different frequency positions in a carrier). I provided proposal 3-5b to collect companies views. |
| Huawei, HiSilicon | We support proposal in 3-4c in general. On 3-5b, we think Ericsson’s proposal should be the right direction that the detailed questions should be sent to RAN4. We should not take it for granted that ACIR metrics will still be used in this SI. We can probably wait for RAN4’s guidance on this. |
| vivo | We are fine with the proposal. |
| Nokia, NSB | Fine to remove ”similar as..." From the description of Aspects 1 and 2. As Ericsson points out, adjacent channel CLI modeling was left out and should be included |
| MediaTek | We have the following comments: - It is not clear to us why the adjacent CLI modelling was removed from the updated proposal 3-4c. So, this needed to be re-added to the proposal. - To evaluate the impact to on the victim UE receiver (selectivity, blockading, dynamic range, etc.), RAN4 should take into consideration that interference may vary across slots, and that Tx and Rx between UEs in different sub-bands may not be fully time-aligned. Thus, we would like to suggest the following note to be added to the proposal. “Note: RAN4 should take into account that interference may vary across slots, and that Tx and Rx between UEs in different sub-bands may not be fully time-aligned.” We would like to suggest removing the “due to Tx non-linearity” from description of Aspect 1. |
| Ericsson 2 | Regarding proposal 3-4c: As mentioned previously, blocking, and dynamic range and non-linearity of the receiver at at the victim cannot be ignored. This is added as Aspect 3 in the proposed modification. ### Updated proposal 3-4bc: (Modified by Ericsson) For discussion of gNB-gNB and UE-UE co-channel inter-subband CLI modelling in system level simulation, consider the following two aspects RAN1 understands at least the following two aspects need to be considered: - Aspect 1: The unwanted emissions due to Tx non-linearity at the transmitter of the aggressor from the allocated RBs to the non-allocated RBs in the same carrier, similar as the In-Band Emission defined for UE. - Aspect 2: The receiver selectivity at the victim to receive the desired signal in the allocated RBs in the presence of the unwanted signals at the non-allocated RBs. The unwanted signals received by the receiver of the victim from the non-allocated RBs when the receiever receives wanted signals in the allocated RBs, similar as the In-channel selectivity defined for BS. - Aspect 3: the receiver blocking at the victim. i.e., overload of the receiver dynamic range - Aspect 4: non-linearity of the receiver at the victim The following questions should be asked to RAN4: - Whether it is feasible to consider the above two three aspects for gNB-gNB and UE-UE co-channel inter-subband CLI modelling in system level simulation? Are there any other aspects should also be taken into account? - For a specific pair of DL frequency unit m (e.g., subband/RB m) and UL frequency unit n (e.g., subband/RB n) of gNB-gNB link, where the DL frequency unit m and UL frequency unit n are in the same carrier and non-overlapping in frequency, and assuming the aggressor gNB transmits on the DL frequency unit m and the victim gNB receives on the UL frequency unit n, - How to model the interference from DL frequency unit m to UL frequency unit n due to Aspect 1 (defined above) at the gNB transmitter? - How to model the interference from DL frequency unit m to UL frequency unit n due to Aspect 2 and Aspect 3 (defined above) at the gNB receiver? - How to model Aspect 4 for the gNB receiver? - How to model the above interferences for the following two cases: - inter-site gNB-gNB co-channel inter-subband CLI - co-site inter-sector co-channel inter-subband CLI - For a specific pair of DL frequency unit m (e.g., subband/RB m) and UL frequency unit n (e.g., subband/RB n) of UE-UE link, where the DL frequency unit m and UL frequency unit n are in the same carrier and non-overlapping in frequency, and assuming the aggressor UE transmits on the UL frequency unit n and the victim UE receives on the DL frequency unit m, - How to model the interference from UL frequency unit n to DL frequency unit m due to Aspect 1 (defined above) at the UE transmitter? - How to model the interference from UL frequency unit n to DL frequency unit m due to Aspect 2 and Aspect 3 (defined above) at the UE receiver? FFS: Usage of the above model provided by RAN4 in the evaluation Regarding proposal 3-5b: We agree with Huawei’s comments and as previously suggested, the ACIR assumes a certain set of assumptions. We are not sure if there is common understanding between RAN1 and RAN4 regarding the specific definition of ACIR. ACIR is used for estimating the interference ratio in the adjacent carrier, and it may cause confusion if we use the same term here (even though we use an equivalent parameter in the SLS), since the SBFD subband is not the whole adjacent carrier. Therefore, we propose to remove the usage of ACIR in the description to RAN4. We also understand the need to provide a framework to RAN4 to get inpuits regarding various components (like what we are doing for Self-interference cancellation). Considering the above two points, perhaps, we can have a combined proposal with following changes. ### Updated proposal 3-5b (newly added): Regarding gNB-gNB and UE-UE adjacent-channel CLI modelling for system level simulation, the ACIR in TR38.828 can be used as the starting point. consider the following aspects: - Transmitter leakage due to non-linearity of the transmitter at the aggressor. - Receiver selectivity, blocking, dynamic range, and due to non-linearity of the receiver at the victim. - RAN1 understands the ACIR in TR38.828 is defined as the ratio of the power transmitted by the aggressor gNB/UE on one carrier to the total interference received by the victim gNB/UE on the adjacent carrier, i.e., the ACIR in TR38.828 is described per carrier. - For SBFD operation a similar interference ratio as ACIR but in the subband of the adjacent carrier, with finer granularity (e.g., per RB or per sub-carrier or per subband) needs to be considered. For example, we can define gNB-gNB- adjacent channel-per-subcarrier interference ratio γgNB-gNBm,n and UE-UE- adjacent channel-per-subcarrier interference ratio γUE-UEm,n. gNB-gNB per-RB-ACIR (γgNB-gNBm,n ) and UE-UE per-RB-ACIR ( γUE-UEm,n) as the ratio of the power transmitted by the aggressor γgNB-gNBm,n or γUE-UEm,n is the ratio of the power transmitted by the aggressor gNB/UE on a frequency unit m (e.g., RB/subcarrier m) in one carrier to the interference received by the victim gNB/UE on another frequency unit n (e.g., another RB/subcarrier n) in the adjacent carrier. - Details of inter-site gNB-gNB adjacent-channel CLI modelling, i.e, the inter-site gNB-gNB adjacent-channel CLI from aggressor gNB A''' to victim gNB A on UL RB/sub-carrier n can be modelled by Iadjacent-channel-CLIn,A'''→A as below: Iadjacent-channel-CLIn,A'''→A=m∈DL RBs in adjacent channelIadjacent-channel-CLIm,n,A'''→A, dBmIadjacent-channel-CLIm,n,A'''→A=Ptxm,A'''+CLA'''→A-dBγgNB-gNBm,n. wherein, - Iadjacent-channel-CLIm,n,A'''→A is the gNB-gNB adjacent-channel CLI of victim gNB A on UL RB/ sub-carrier n caused by DL transmission of aggressor gNB A''' on RB/ sub-carrier m in adjacent carrier. - Ptxm,A''' is the DL transmission power of aggressor gNB A''' on RB/ sub-carrier m in adjacent channel (in dBm). - CLA'''→A is the coupling loss between aggressor gNB A''' and victim gNB A, including pathloss, penetration loss, shadow fading and Tx/Rx antenna gain. - γgNB-gNBm,n is the gNB-gNB adjacent channel-per-subcarrier interference ratio. - Details of UE-UE adjacent-channel CLI modelling, i.e, the UE-UE adjacent-channel CLI from aggressor UE B''' to victim UE B on DL RB/ sub-carrier n can be modelled by Iadjacent-channel-CLIn,B'''→B as below: Iadjacent-channel-CLIn,B'''→B=m∈UL RBs in adjacent channelIadjacent-channel-CLIm,n,B'''→B, dBmIadjacent-channel-CLIm,n,B'''→B=Ptxm,B'''+CLB'''→B-dBγUE-UEm,n. wherein, - Iadjacent-channel-CLIm,n,B'''→B is the UE-UE adjacent-channel CLI of victim UE B on UL RB/ sub-carrier n caused by UL transmission of aggressor UE B''' on RB/ sub-carrier m in adjacent carrier. - Ptxm,B''' is the UL transmission power of aggressor UE B''' on RB/ sub-carrier m in adjacent channel (in dBm). - CLB'''→B is the coupling loss between aggressor UE B''' and victim UE B, including pathloss, penetration loss, shadow fading and Tx/Rx antenna gain. - γUE-UEm,n is the UE-UE adjacent channel-per-subcarrier interference ratio. The following need to be asked to RAN4: - For a specific DL RB(s) and UL RB(s) pair of gNB-gNB link, where the DL RB(s) and the UL RB(s) are in the adjacent carrier, and assuming the aggressor gNB transmits on the DL RB(s) and the victim gNB receives on the UL RB(s), - What is the value range of gNB-gNB adjacent channel-per-subcarrier interference ratio (γgNB-gNBm,n ) and UE-UE adjacent channel-per-subcarrier interference ratio ( γUE-UEm,n) as described above for each frequency range? In other words, - How to model the DL RB(s) to UL RB(s) leakage interference at the gNB transmitter? - How to model the DL RB(s) to UL RB(s) interference due to receiver selectivity and blocking at the gNB receiver? - How to model non-linearity of the gNB receiver? - How to model gNB-gNB adjacent-channel CLI when the two gNBs are from the same sector of the same site in adjacent carriers? - How to model gNB-gNB adjacent-channel CLI when the two gNBs are from different sectors of the same site in adjacent carriers? - Whether it is feasible to use the inter-site gNB-gNB adjacent-channel CLI modelling as described above? - Does gNB-gNB and UE-UE adjacent channel-per-subcarrier interference ratio have any dependency with the following factors or any other factors? What are the dependencies? - the frequency distance between the Tx RB m and the Rx RB n, etc. - Whether it is feasible to use the UE-UE adjacent-channel CLI modelling as described above? |
| Sony | We are fine with the modified proposal 3-4c. For Proposal 3-5b, I take it we just want an ACIR that has finer granularity from RAN4. We are fine with the intention. |
| Moderator | Proposal 3-4c has been agreed as below: Agreement For discussion of gNB-gNB and UE-UE co-channel inter-subband CLI modelling in system level simulation, RAN1 understands at least the following two aspects need to be considered: - Aspect 1: The unwanted emissions due to Tx non-linearity at the transmitter of the aggressor from the allocated RBs to the non-allocated RBs in the same carrier. - Aspect 2: The receiver selectivity at the victim to receive the desired signal in the allocated RBs in the presence of the unwanted signals at the non-allocated RBs. (e.g. receiver blocking at the victim, overload of the receiver dynamic range, etc) The following questions should be asked to RAN4: - Whether it is feasible to consider the above two aspects for gNB-gNB and UE-UE co-channel inter-subband CLI modelling in system level simulation? Are there any other aspects should also be taken into account? - For a specific pair of DL frequency unit m (e.g., subband/RB m) and UL frequency unit n (e.g., subband/RB n) of gNB-gNB link, where the DL frequency unit m and UL frequency unit n are in the same carrier and non-overlapping in frequency, and assuming the aggressor gNB transmits on the DL frequency unit m and the victim gNB receives on the UL frequency unit n, - How to model the interference from DL frequency unit m to UL frequency unit n due to Aspect 1 (defined above) at the gNB transmitter? - How to model the interference from DL frequency unit m to UL frequency unit n due to Aspect 2 (defined above) at the gNB receiver? - How to model the above interferences for the following two cases: - inter-site gNB-gNB co-channel inter-subband CLI - co-site inter-sector co-channel inter-subband CLI - For a specific pair of DL frequency unit m (e.g., subband/RB m) and UL frequency unit n (e.g., subband/RB n) of UE-UE link, where the DL frequency unit m and UL frequency unit n are in the same carrier and non-overlapping in frequency, and assuming the aggressor UE transmits on the UL frequency unit n and the victim UE receives on the DL frequency unit m, - How to model the interference from UL frequency unit n to DL frequency unit m due to Aspect 1 (defined above) at the UE transmitter? - How to model the interference from UL frequency unit n to DL frequency unit m due to Aspect 2 at the UE receiver? FFS: Usage of the above model provided by RAN4 in the evaluation |

## 4.7 5th Round Proposals (open)

### Updated proposal 3-1c(v1):

For discussion purpose, consider the following as RAN1’s common understanding:

- Co-channel interference: The interference is from the aggressor to the victim in the same carrier.

  - Co-channel intra-subband interference: The interference is caused by transmission of the aggressor on some RBs in a subband in a carrier to reception of the victim on the same RBs in the same subband in the same carrier.

  - Co-channel inter-subband interference: The interference is caused by transmission of the aggressor in subband#1 in a carrier to reception of the victim in subband#2 in the same carrier, where the subband#1 and subband#2 are non-overlapping in frequency.

- Adjacent channel interference: The interference is from the aggressor in carrier#1 to the victim in carrier#2, where the carrier#1 and carrier#2 are adjacent carriers.

Note 1: ‘Co-channel’ here means ‘co-carrier’. ‘Adjacent-channel’ here means ‘adjacent-carrier’.

Note 2: A ‘subband’ here just means a number of consecutive RBs, it has not to be the whole SBFD UL subband or SBFD DL subband as we defined in AI9.3.2, and it can be just a part of the SBFD UL subband or SBFD DL subband. The definition of the subband here is only used for the definition of ‘intra-subband interference’ and ‘inter-subband interference’, and it will not be used for other purposes, e.g., SBFD subband configuration. The following figures illustrate the ‘intra-subband interference’ and ‘inter-subband interference’ respectively and the ‘subband’ used for them.

Figure 1: The illustration of ‘intra-subband interference’ and the ‘subband’ used for it

Figure 2: The illustration of ‘inter-subband interference’ and the ‘subband’ used for it

### Updated proposal 3-2c(v1):

For discussion purpose, consider the following as the common understanding in RAN1 on the definition of interference types for SBFD operation:

- gNB self-interference (SI): Interference caused by DL transmission in DL subband in a carrier to UL reception in UL subband in the same carrier at the gNB side, where the DL subband and the UL subband are non-overlapping in frequency.

- gNB-UE co-channel intra-subband interference: This is the same as the legacy DL interference type in legacy TDD network with static TDD UL/DL configuration.

- UE-gNB co-channel intra-subband interference: This is the same as the legacy UL interference type in legacy TDD network with static TDD UL/DL configuration.

- (inter-cell) inter-site gNB-gNB co-channel intra-subband CLI: CLI caused by DL transmission of the aggressor gNB on some RBs in a subband in one carrier to UL reception of the victim gNB in a different site on the same RBs in the same subband in the same carrier.

- (inter-cell) co-site inter-sector co-channel intra-subband CLI: CLI caused by DL transmission of the aggressor gNB on some RBs in a subband in one carrier to UL reception of the victim gNB in another sector of the same site on the same RBs in the same subband in the same carrier.

- (inter-cell) UE-UE co-channel intra-subband CLI: CLI caused by UL transmission of the aggressor UE on some RBs in a subband in one carrier to DL reception of the victim UE on the same RBs in the same subband in the same carrier.

- (inter-cell) inter-site gNB-gNB co-channel inter-subband CLI: CLI caused by DL transmission of the aggressor gNB in DL subband in a carrier to UL reception of the victim gNB in a different site in UL subband in the same carrier, where the DL subband and the UL subband are non-overlapping in frequency.

- (inter-cell) co-site inter-sector co-channel inter-subband CLI: CLI caused by DL transmission of the aggressor gNB in DL subband in a carrier to UL reception of the victim gNB in another sector of the same site in UL subband in the same carrier, where the DL subband and the UL subband are non-overlapping in frequency.

- (intra-cell/inter-cell) UE-UE co-channel inter-subband CLI: CLI caused by UL transmission of the aggressor UE in UL subband in a carrier to DL reception of the victim UE in DL subband in the same cell or neighboring cell in the same carrier, where the UL subband and the DL subband are non-overlapping in frequency.

- gNB-gNB adjacent-channel CLI: CLI caused by DL transmission of the aggressor gNB in a carrier to UL reception of the victim gNB in another adjacent carrier.

  - This includes adjacent-channel CLI between gNBs in the same and different sectors of the same site, i.e., co-site intra and inter-sector adjacent-channel CLI.

- UE-UE adjacent-channel CLI: CLI caused by UL transmission of the aggressor UE in a carrier to DL reception of the victim UE in another adjacent carrier.

Note: Some of the interferences may not be used according to the deployment scenarios, e.g, whether the SBFD subband configurations are the same or different across gNBs.

Note: This does not imply we need to consider all the above interference types in evaluation for SBFD.

### Updated proposal 3-1c(v2):

For discussion purpose, consider the following as RAN1’s common understanding:

- Co-channel interference: The interference is from the aggressor to the victim in the same carrier.

  - Co-channel intra-subband interference: The interference is caused by transmission of the aggressor on a set of RBs in a carrier to reception of the victim on the same set of RBs in the same carrier.

  - Co-channel inter-subband interference: The interference is caused by transmission of the aggressor in a first set of RBs in a carrier to reception of the victim in a second set of RBs in the same carrier, where the two RB sets are non-overlapping in frequency.

- Adjacent channel interference: The interference is from the aggressor in carrier#1 to the victim in carrier#2, where the carrier#1 and carrier#2 are adjacent carriers.

Note 1: ‘Co-channel’ here means ‘co-carrier’. ‘Adjacent-channel’ here means ‘adjacent-carrier’.

### Updated proposal 3-2c(v2):

For discussion purpose, consider the following as the common understanding in RAN1 on the definition of interference types for SBFD operation:

- gNB self-interference (SI): Interference caused by DL transmission on a set of DL RBs in a carrier to UL reception on a set of UL RBs in the same carrier at the gNB side, where the two RB sets are non-overlapping in frequency.

- gNB-UE co-channel intra-subband interference: This is the same as the legacy DL interference type in legacy TDD network with static TDD UL/DL configuration.

- UE-gNB co-channel intra-subband interference: This is the same as the legacy UL interference type in legacy TDD network with static TDD UL/DL configuration.

- (inter-cell) inter-site gNB-gNB co-channel intra-subband CLI: CLI caused by DL transmission of the aggressor gNB on a set of RBs in one carrier to UL reception of the victim gNB in a different site on the same set of RBs in the same carrier.

- (inter-cell) co-site inter-sector co-channel intra-subband CLI: CLI caused by DL transmission of the aggressor gNB on a set of RBs in one carrier to UL reception of the victim gNB in another sector of the same site on the same set of RBs in the same carrier.

- (inter-cell) UE-UE co-channel intra-subband CLI: CLI caused by UL transmission of the aggressor UE on a set of RBs in one carrier to DL reception of the victim UE on the same set of RBs in the same carrier.

- (inter-cell) inter-site gNB-gNB co-channel inter-subband CLI: CLI caused by DL transmission of the aggressor gNB on a first set of RBs in a carrier to UL reception of the victim gNB in a different site on a second set of RBs in the same carrier, where the two RB sets are non-overlapping in frequency.

- (inter-cell) co-site inter-sector co-channel inter-subband CLI: CLI caused by DL transmission of the aggressor gNB on a first set of RBs in a carrier to UL reception of the victim gNB in another sector of the same site on a second set of RBs in the same carrier, where the two RB sets are non-overlapping in frequency.

- (intra-cell/inter-cell) UE-UE co-channel inter-subband CLI: CLI caused by UL transmission of the aggressor UE on a first set of RBs in a carrier to DL reception of the victim UE on a second set of RBs in the same cell or neighboring cell in the same carrier, where the two RB sets are non-overlapping in frequency.

- gNB-gNB adjacent-channel CLI: CLI caused by DL transmission of the aggressor gNB in a carrier to UL reception of the victim gNB in another adjacent carrier.

  - This includes adjacent-channel CLI between gNBs in the same and different sectors of the same site, i.e., co-site intra and inter-sector adjacent-channel CLI.

- UE-UE adjacent-channel CLI: CLI caused by UL transmission of the aggressor UE in a carrier to DL reception of the victim UE in another adjacent carrier.

Note: Some of the interferences may not be used according to the deployment scenarios, e.g, whether the SBFD subband configurations are the same or different across gNBs.

Note: This does not imply we need to consider all the above interference types in evaluation for SBFD.

|  | Company |
| --- | --- |
| Support 3-1c(v1)&3-2c(v1) | ZTE, QC |
| Support 3-1c(v2)&3-2c(v2) | Sony, Ericsson, IDC, CATT |

| Company |  | Comment |  |
| --- | --- | --- | --- |
| Moderator |  | I will invite companies to provide comments and views on the two sets of proposals: the first set is 3-1c(v1)&3-2c(v1), and the second set is 3-1c(v2)&3-2c(v2). Here I share some of my thinkings on the two set of proposals: - 3-1c(v1)&3-2c(v1): These two are the same as 4th round except two figures are included. ‘subband’ is used in the definition to clarify ‘intra-subband interference’ and ‘inter-subband interference’, but the definition of ‘subband’ here is a little different from the ‘SBFD subband’ we have in AI9.3.2. Strictly speaking, we may need to differentiate ‘subband’ and ‘SBFD subband’, and ‘SBFD subband’ is what we defined in AI9.3.2. A note and two figures are also given to clarify the difference of ‘subband’ used here and ‘SBFD subband’ defined in AI9.3.2. - 3-1c(v2)&3-2c(v2): ‘subband’ is not used in the definition of ‘intra-subband interference’ and ‘inter-subband interference’, instead, we just used ‘a set of RBs’ to clarify them. Some one may feel a little bit strange that ‘subband’ is not used in the definition of ‘intra-/inter-subband …’, but I think it should be clear although it may not be perfect. From my perspective, these proposals are just for defining the terminologies since we need to use them frequently in the subsequent discussions. I think both sets are fine and all companies are on the same page after we had so much discussions on them. Let’s collet companies’ views and just follow the majority. |  |
| ZTE |  | We slightly prefer 3-1c(v1)&3-2c(v1). Otherwise, RAN4 may be confused about the relationship between subband and RB sets. |  |
| New H3C |  | We are fine with 3-1c(v1)&3-2c(v1) based on below conclusion in AI9.3.2 Conclusion For discussion purpose, for SBFD operation within a TDD carrier, a SBFD subband consists of 1 RB or a set of consecutive RBs for the same transmission direction. |  |
| Xiaomi |  | No strong view, either is OK to us. |  |
| Spreadtrum |  | Both are OK to us. |  |
| Huawei, HiSilicon |  | We have a sligh preference to 3.1c(v2) and 3.2c(v2) but can also accept 3.1c(v1) and 3.2c(v1). |  |
| Sony |  | Object to the use of “same” subband, i.e. Object to Proposal 3-1c(v1) and Proposal 3-2c(v2). In Proposal 3-1c(v1): - Co-channel intra-subband interference: The interference is caused by transmission of the aggressor on some RBs in a subband in a carrier to reception of the victim on the same RBs in the same subband in the same carrier. The “same subband” used here is really misleading and even confusing. In the accompanied figures, there are no “same” subbands as the subbands in gNB1 and gNB2 are totally different. I really do not understand why we call it “same” where nothing about them is the same. If we must use the “subband” term to describe the interference, why can’t we call it “overlapping” subband or “another” subband, which were suggested by some companies (including Sony) for a few rounds already but were totally ignored and instead, insisted on a confusing term like “same” subband? It seems strange that people rather abandon the term “subband” than to abandon the misleading term “same”. Hence, if there are only two choices, we prefer Proposal 3-1c(v2) and Proposal 3-2c(v2), which provide a far more accurate description. |  |
| Nokia, NSB |  | We don’t have strong view, either option is fine. |  |
| Moderator |  | @Sony, I noticed CATT/Sony have provided some modified versions (I did not directly use that version since I think it is also not so clear and accurate), based on which I provided 3.1(v2)&3.2(v2) to collect companies’ views, and I did not ignore them. |  |
| vivo |  | We prefer 3.1c(v2) and 3.2c(v2), as they are clearer. |  |
| Sony |  | @Moderator: Thanks for Proposal 3.1(v2) and 3.2(v2). Actually, to be precise, the term “intra subband” is also not accurate. “Intra” means something happening within an object, e.g. intra-UE L1 prioritisation is prioritisation within a UE. Intra-subband would mean that something is happening within/inside a subband. For example, if we have the configuration below, gNB1 has Subband A, Subband B and Subband C whilst gNB2 has Subband D, Subband E and Subband F. If we focus on say Subband A, “intra subband” interference would simply mean interference within Subband A, i.e. solely inside of Subband A. The interference described in Figure 1 of Section 4.7 above, is really between two different subbands. If we use my figure here, it is basically interference from DL transmission in Subband A into overlapping RBs in Subband E. Subband A and Subband E have totally different configurations and belong to totally different gNBs. These are actually inter subbands that overlaps. We never use “intra UE” interference to mean interference between two different UEs, so it is rather surprising and confusing why we refer to “intra subband” interference to mean interference between two different subbands. Hence, it was really confusing and misleading when the term “same subband” was further used to describe “intra subband” are used to describe interference between two different subbands that have different configurations and belonged to different gNBs. It is really surprising that these terms were used over several rounds of email discussion and only in the 5th round with the figure used to describe the interference, does it become clear that the terms “same” and “intra” used are not accurate. If I were to describe the intented interferences I would say: - Co-channel overlapping inter-subband interference: The interference is caused by transmission of the aggressor on a set of RBs in a carrier to reception of the victim on the same set of RBs in the same carrier, where the aggressor and victim are in different gNBs/site. - Co-channel non-overlapping inter-subband interference: The interference is caused by transmission of the aggressor in a first set of RBs in a carrier to reception of the victim in a second set of RBs in the same carrier, where the two RB sets are non-overlapping in frequency. The aggressor and victim can be in the same gNB/site or in different gNB/site. |  |
| Ericsson | We agree with the comments from Sony regarding the usage of “same subband” for intre-subband interference, which causes confusion in our opinion. We prefer v2. We are also open to editing V1 to ensure there is no confusion. |  |
| QC | A subband is set or contiguous RBs. Both versions deliver the same message. However, we prefer V1 as it is aligned with study item terminology ‘subband’. |  |
| Intel | Slightly prefer prefer 3-1c(v2)&3-2c(v2) since the text is a bit more clear, and less ambiguous, but we do not have a strong preference. |  |
| InterDigital | Prefer 3.1c(v2) and 3.2c(v2), which are clear enough withough having “second-level” subband terms. |  |
| NTT DOCOMO | We don’t have strong preference for the options. |  |
| CATT | We prefer 3-1c(v2)&3-2c(v2) to avoid potential confusion. |  |

### Initial proposal 3-3c:

For the purposes of link-level evaluations, an overall "net effect" model is needed that captures key components of the gNB transmit chain including crest-factor reduction (CFR) + digital pre-distortion (DPD) + power amplifier (PA), while satisfying the -45 dBc requirement on ACLR. For link-level simulations, the model should be sufficiently detailed to allow accurate generation of the self-interference signals to be cancelled.

The following should be asked to RAN4

- For accurate generation of the self-interference signals at the gNB for the purposes of link level evaluation of self-interference cancellation, recommend a model to be used for capturing the overall "net" effect of key gNB transmit chain components, i.e., CFR + DPD + PA.

| Company | Comment |
| --- | --- |
| Moderator | Some companies have already commented on this proposal in 4th round. For convenience, I copied them below: - Huawei’ comment: we would like to understand the reasoning for the “-45 dBc requirement on ACLR”. In addition, we would like to clarify how to make use of the model for LLS since our understanding is that self-interference will be largely dependent on implementation. - Ericsson’s response: the -45 dBC is the ACLR requirement currently, and considering SBFD will coexist with legacy networks, this requirement needs to be met regardless. If self-interference is largely based on implementation, how can we guarantee that X dB of SIC is attainable? Company A may provide X dB SIC and company B may provide Y dB SIC amount which will cause coexistence issues, also to legacy networks. Therefore, we need a clear framework on how much SIC is attainable based on the factors we described in our comment for 2nd round discussion proposal 3-3. We need to perform link level simulations for this purpose and need RAN4 to help us with the net-effect model. - Sony’s comment: isn’t the “net effect” for CFR + DPD + PA, already factored into the RSI that we are asking RAN4 to provide? Are these just additional details but really we are only interested in RSI in RAN1 for system level simulation purpose? - Moderator’s response on Sony’s comment: This proposal is for LLS, not SLS. In LLS, we may need more detailed models for self-interference. Companies please continue comment and refine the proposal. |
| ZTE | Regarding the newly added Initial proposal 3-3c, it seems the intention is to ask for detailed modeling for self-inerfrence. We are not sure whether RAN4 could provide such detailed modeling. But we are open to consider this proposal. |
| New H3C | We are fine with asking RAN4 for "net effect" model |
| MediaTek | Fine with the proposal. |
| Xiaomi | We are fine with the proposal. We believe it can benefit RAN1 on both simulation and study on self-interference suppression. |
| Huawei, HiSilicon | We support the intention of the proposal appreciated Ericsson’s feedback. We would like to have some further clarifications. First, since the proposal is to ask RAN4 for a model which captures the overall "net" effect of gNB transmit chain components, we are wondering whether this has been covered by gNB-gNB inter-cell inter-subband interference model, where the following question is asked to RAN4. - How to model the interference from DL frequency unit m to UL frequency unit n due to Aspect 1 (defined above) at the gNB transmitter? Second, based on the RAN4 model if provided, one still need to discuss how to make use of model, e.g. the channel model between a given {Tx antenna, Rx antenna} pair. Is this the intention? |
| Sony | It doesn’t hurt to ask for more information. It would also be good to clarify how these extra parameters are to be used, as Huawei commented, the SIC or any advance cancellation methods are based on gNB implementation. |
| Nokia, NSB | We are fine to have good model, but still unclear how detail level we are asking for. It is also good to discuss in RAN1 and ask for its validity to RAN4. But, we can follow majority view at this moment. |
| vivo | We support the intention to ask RAN4. We have one comment. When asking RAN4 about the model to be used for capturing the overall "net" effect, it is better to ask RAN4 which components shall be excluded, e.g., spectrum shaping filter, outband filter and so on, rather than which elements shall be considered especially considering different implementation may have different components elements of transmitting chain. |
| Ericsson | We support the proposal Based on the company’s comments above, it seems it is not clear what level of detail for which RAN1 would be asking RAN4, and how this model would be used. - Huawei points out that RAN1 will ask RAN4 about the interference from DL frequency unit m to UL frequency unit n due to Aspect 1, and this is fine for SLS. - Nokia questions what level of detail is being requested of RAN4. - Sony and Huawei question how the model would be used We'd like to try to clarify those points. For link-leve simulation (LLS), the level of detail we have in mind would allow the actual time domain signal at the output of the PA to be generated at link level capturing the net effect of the DPD + PA. For example, one model that RAN4 has discussed in the past is a generalized memory poloynomial (GMP) for the PA, which for a given sequence of input samples x(n) allows the output y(n) to be calculated taking into account the non-linear effects of the PA. From a RAN1 perspective, we are interested in the net effect of the DPD + PA to generate the output signal, not just the PA. For example, the net effect could be captured with the following GMP model: ynetn=k∈Kanetl∈Lanetaklnetxn-lxn-l2k Which allows the generation of the PA output signal samples y_net(n) given the input signal samples to the DPD x(n). What RAN4 would need to discuss is what should the coefficients of the model be and how many terms should be include to model the output signal with sufficient accuracy? To answer Huawei's question, clearly this is different than what is needed for system simulations where it is not necessary to actually generate the time domain waveform at the output of the PA. In addition, since typically a gNB transmit chain includes some form of crest factor reduction (CFR) to limit the amplitude of the input signal to the DPD, and thus allow the PA to be operated efficiently without excessive backoff. CFR is inherently non-linear, and to constrain the spectrum, digital filtering is used. For RAN1 LLS purposes, we think that CFR could be modeled in a very simple way by hard limiting followed by digital filtering. However, the feasibility of this would need to be discussed in RAN4. So, in summary, we think RAN1 should ask RAN4 for two things: - Net effect model for DPD + PA to model the time domain output signal from the PA with sufficient accuracy. As we pointed out earlier, the gNB would need to be compliant with existing specifications on ACLR (-45 dBc), so the net effect model should account for that. - How to model CFR in a simple way and still capture the essential behavior of a practical gNB designed for PA efficiency and ACLR compliance. Regarding how this model would be used, the main purpose is to study what levels of self-interference cancellation are practically achievable (i.e., how many dBs of suppression) considering practical algorithms. Such numbers can then be used as practical/feasible inputs to system level simulation to establish the performance of SBFD. |
| QC | During RAN1 NR waveform study, RAN1 has sent an LS to RAN4 (R1-163890 / R4-163128) for realistic power amplifier model for 5G waveform evaluation. Based on RAN4 response, RAN1 adopted RAPP PA model [2] with specific parametrization for <6 GHz. This model considers DPD and CFR [1]. This PA model could be used as a first-order approximation. In addition, RAN1 may ask RAN4 on futher enhancement of this PA model (if needed). Also, RAN1 should ask RAN4 for both frequency ranges. [85-18] Mihai (Nokia) PA assumption for NR for email discussion and approval about power amplifier assumptions for NR DL waveform evaluations by utilizing R1-166004 and R1-166010 and also email approval to confirm WA until June 9th Done: According to Mr Chair’s email dated June 17th , the following is agreed: [UL part] …. [DL part] - For below 6 GHz DL, consider as a starting point the below Rapp model parametrization assuming 46 dBm total output power and 57.6 dBm saturation power (this model considers DPD and CFR) . Parameter for Rapp model < 6GHz DL. Target output power [dBm]; 46. Saturation output power [dBm]; 57.6. Smoothness factor p; 3. Smoothness factor q; 5. Fitting parameter A; -0.14. Fitting parameter B; 1.2. Note that the gain of the model can be chosen freely, as it depends on the input signal reference level. The expected output for LTE10 DL 64-QAM signal is 46 dBm, with ACLR 45 dB and meeting respective spectrum emission masks. - For above 6GHz DL, consider as a starting point a suitable Rapp model parametrization.. [1] R1-166056, Final Report of 3GPP TSG RAN WG1 #85 v1.0.0, Nanjing, China, 23rd – 27th May 2016, clause [85-18] Mihai (Nokia) [2] R1-167779, On DL Rapp PA model for waveform evaluation below 6 GHz, Nokia, Gothenburg, Sweden, 22 - 26 August 2016 We suggest including this history in the LS to RAN4 and ask them whether this model can be reused and any further enhancements from RAN4. ### Initial proposal 3-3c: For the purposes of link-level evaluations, an overall "net effect" model is needed that captures key components of the gNB transmit chain including crest-factor reduction (CFR) + digital pre-distortion (DPD) + power amplifier (PA), while satisfying the -45 dBc requirement on ACLR. For link-level simulations, the model should be sufficiently detailed to allow accurate generation of the self-interference signals to be cancelled. In Rel-15, RAN1 has asked RAN4 for realistic power amplifier model for 5G waveform evaluation (R1-163890 / R4-163128) and based on RAN4 response, RAN1 has adopted modified RAPP PA model (R1-167779) that considers DPD and CFR and 45 dB ACLR. The following should be asked to RAN4 - For accurate generation of the self-interference signals at the gNB for the purposes of link level evaluation of self-interference cancellation, recommend a model to be used for capturing the overall "net" effect of key gNB transmit chain components, i.e., CFR + DPD + PA for both FR1 and FR2. - Whether the RAPP PA model in R1-167779 can be used for the RAN1 evaluation and whether any further model enhancements is needed. |
| Intel | We are OK with the proposal, and definitely it won’t hurt to ask more information to RAN4 especially if this would benefit this study. |
| NTT DOCOMO | We are fine with the proposal. |
| Samsung | We are open to the proposal. Few comments are - If you understand correctly, the proposal is to accurately model self-interference for link-level-evaluation. To do this, a net effect model of CFR, DPD, and PA is suggested. From our understanding, it is also important to take into account channel propargation between Tx antanna chain and Rx antenna chain. - We cannot understand the reason why RAN1 explicitly requests a specific self-interference model such as the net effect model. It is up to RAN4’s works. Since RAN4’s works is not started, we are afraid to make too specific model without RAN4’s expertise. As the RSI in the SLS evaluation, we can ask high-level questions on how to model self-interference by considering TX non-linearity and also give the net effect model as a candidate method. - Even though RAN1 may have a specific and accurate self-interference model, the unclear part is how to use is and draw a conclusion. Probabily, RAN1 will not discuss self-interference cancellation algorithm and implementation. Due to very diverse algorithms and implementations across different companies, the results may be also diverse. In this case, how can we make a concensus? |
| CATT | We are fine with the proposal. |

### Updated proposal 3-5c:

Regarding gNB-gNB and UE-UE adjacent-channel CLI modelling for system level simulation, RAN1 understands at least the following aspects need to be considered:

- Aspect 1: The unwanted emissions due to Tx non-linearity at the transmitter of the aggressor from the allocated RBs in one carrier to the non-allocated RBs in the adjacent carrier.

- Aspect 2: The receiver selectivity at the victim to receive the desired signal in the allocated RBs in one carrier in the presence of the unwanted signals at the non-allocated RBs in the adjacent carrier. (e.g. receiver blocking at the victim, overload of the receiver dynamic range, etc)

The following questions should be asked to RAN4:

- Whether it is feasible to consider the above two aspects for gNB-gNB and UE-UE adjacent-channel CLI modelling in system level simulation? Are there any other aspects should also be taken into account?

- For a specific pair of DL frequency unit m (e.g., subband/RB m) and UL frequency unit n (e.g., subband/RB n) of gNB-gNB link, where the DL frequency unit m and UL frequency unit n are in adjacent carriers and non-overlapping in frequency, and assuming the aggressor gNB transmits on the DL frequency unit m and the victim gNB receives on the UL frequency unit n,

  - How to model the interference from DL frequency unit m to UL frequency unit n due to Aspect 1 (defined above) at the gNB transmitter?

  - How to model the interference from DL frequency unit m to UL frequency unit n due to Aspect 2 (defined above) at the gNB receiver?

  - How to model the above interferences for the following cases:

    - the two gNBs are from the same sector of the same site in adjacent carriers, i.e., co-site co-sector gNB-gNB adjacent-channel CLI

    - the two gNBs are from different sectors of the same site in adjacent carriers, i.e., co-site inter-sector gNB-gNB adjacent-channel CLI

    - the two gNBs are from different sites in adjacent carriers, i.e., inter-site gNB-gNB adjacent-channel CLI

- For a specific pair of DL frequency unit m (e.g., subband/RB m) and UL frequency unit n (e.g., subband/RB n) of UE-UE link, where the DL frequency unit m and UL frequency unit n are in adjacent carriers and non-overlapping in frequency, and assuming the aggressor UE transmits on the UL frequency unit n and the victim UE receives on the DL frequency unit m,

  - How to model the interference from UL frequency unit n to DL frequency unit m due to Aspect 1 (defined above) at the UE transmitter?

  - How to model the interference from UL frequency unit n to DL frequency unit m due to Aspect 2 at the UE receiver?

Specifically, the following is one example for gNB-gNB and UE-UE adjacent-channel CLI modelling in system level simulation from RAN1 perspective:

- RAN1 understands the ACIR in TR38.828 is used for gNB-gNB and UE-UE adjacent-channel CLI modelling in Rel-16 CLI study item, which is defined as the ratio of the power transmitted by the aggressor gNB/UE on one carrier to the total interference received by the victim gNB/UE on the adjacent carrier, i.e., the ACIR in TR38.828 is described per carrier. For system-level simulation of adjacent-channel co-existence between legacy TDD operation and SBFD operation, a similar interference ratio as ACIR but in the subband of the adjacent carrier, with finer granularity (e.g., per subband or per RB or per sub-carrier) needs to be considered.

- As one example, we can define gNB-gNB-adjacent-channel-per-RB interference ratio γgNB-gNBm,n and UE-UE-adjacent-channel-per-RB interference ratio γUE-UEm,n to represent the overall effect of the Aspect 1 and Aspect 2 described above. γgNB-gNBm,n or γUE-UEm,n can be defined the ratio of the power transmitted by the aggressor gNB/UE on RB m in one carrier to the interference received by the victim gNB/UE on another RB n in the adjacent carrier.

  - Details of inter-site gNB-gNB adjacent-channel CLI modelling: the inter-site gNB-gNB adjacent-channel CLI from aggressor gNB A''' to victim gNB A on UL RB n can be modelled by Iadjacent-channel-CLIn,A'''→A as below:

Iadjacent-channel-CLIn,A'''→A=m∈DL RBs in adjacent channelIadjacent-channel-CLIm,n,A'''→A,

dBmIadjacent-channel-CLIm,n,A'''→A=Ptxm,A'''+CLA'''→A-dBγgNB-gNBm,n.

Wherein,

    - Iadjacent-channel-CLIm,n,A'''→A is the gNB-gNB adjacent-channel CLI of victim gNB A on UL RB n caused by DL transmission of aggressor gNB A''' on RB m in adjacent carrier.

    - Ptxm,A''' is the DL transmission power of aggressor gNB A''' on RB m in adjacent channel (in dBm).

    - CLA'''→A is the coupling loss between aggressor gNB A''' and victim gNB A, including pathloss, penetration loss, shadow fading and Tx/Rx antenna gain.

    - γgNB-gNBm,n is the gNB-gNB-adjacent-channel-per-RB interference ratio.

  - Details of UE-UE adjacent-channel CLI modelling: the UE-UE adjacent-channel CLI from aggressor UE B''' to victim UE B on DL RB n can be modelled by Iadjacent-channel-CLIn,B'''→B as below:

Iadjacent-channel-CLIn,B'''→B=m∈UL RBs in adjacent channelIadjacent-channel-CLIm,n,B'''→B,

dBmIadjacent-channel-CLIm,n,B'''→B=Ptxm,B'''+CLB'''→B-dBγUE-UEm,n.

Wherein,

    - Iadjacent-channel-CLIm,n,B'''→B is the UE-UE adjacent-channel CLI of victim UE B on UL RB n caused by UL transmission of aggressor UE B''' on RB m in adjacent carrier.

    - Ptxm,B''' is the UL transmission power of aggressor UE B''' on RB m in adjacent channel (in dBm).

    - CLB'''→B is the coupling loss between aggressor UE B''' and victim UE B, including pathloss, penetration loss, shadow fading and Tx/Rx antenna gain.

    - γUE-UEm,n is the UE-UE-adjacent-channel-per-RB interference ratio.

- The following questions should be asked to RAN4 regarding the above example of gNB-gNB and UE-UE adjacent-channel CLI modelling in system level simulation:

  - Whether it is feasible to use the gNB-gNB-adjacent-channel-per-RB interference ratio (γgNB-gNBm,n) and UE-UE-adjacent-channel-per-RB interference ratio (γUE-UEm,n) as described above to represent the overall effect of the Aspect 1 and Aspect 2? If it is feasible, then what is the value range of the gNB-gNB-adjacent-channel-per-RB interference ratio (γgNB-gNBm,n) and UE-UE-adjacent-channel-per-RB interference ratio (γUE-UEm,n) for each frequency range?

  - Whether it is feasible to apply the detailed inter-site gNB-gNB adjacent-channel CLI modelling using gNB-gNB-adjacent-channel-per-RB interference ratio (γgNB-gNBm,n) in system-level simulation, i.e., the ‘Details of inter-site gNB-gNB adjacent-channel CLI modelling’ part?

  - Whether it is feasible to apply the detailed UE-UE adjacent-channel CLI modelling using UE-UE-adjacent-channel-per-RB interference ratio (γUE-UEm,n) in system-level simulation, i.e., the ‘Details of UE-UE adjacent-channel CLI modelling’ part?

  - How to model so-site co-sector gNB-gNB adjacent-channel CLI using gNB-gNB-adjacent-channel-per-RB interference ratio (γgNB-gNBm,n) in system-level simulation?

  - How to model so-site inter-sector gNB-gNB adjacent-channel CLI using gNB-gNB-adjacent-channel-per-RB interference ratio (γgNB-gNBm,n) in system-level simulation?

| Company | Comment |
| --- | --- |
| ZTE | Regarding the newly added propose Updated proposal 3-5b (newly added) , we think it is important to consider adjacent channel in RAN1 simulatoin thus we should agree and capture it in the LS to RAN4. |
| New H3C | In general, we are fine with this proposal. |
| MediaTek | Generally, we are supporting the proposal. However, we are not sure if this part of the proposal is neded: “Specifically, the following is one example for gNB-gNB and UE-UE adjacent-channel CLI…..”. |

| Xiaomi | We are fine with the proposal. There are two typos in the last two bullets, it should be co-site instead of so-site. |
| --- | --- |
| Huawei, HiSilicon | We think the example provided in the later part of the proposal needs to be discussed further. Basically, we think it is not roper to use one parameter, i.e. gNB-gNB-adjacent-channel-per-RB interference ratio γgNB-gNBm,n and UE-UE-adjacent-channel-per-RB interference ratio γUE-UEm,n, to represent the overall effect of the Aspect 1 and Aspect 2. Our understanding is that the overall interference from the aggressor to the vitim can be calculated based on a similar method as the co-channel inter-gNB inter-subband interference, i.e. sum of the interference due to Aspect 1 and Aspect 2. We would be fine if the description from “Specifically, the following is one example for gNB-gNB and UE-UE adjacent-channel CLI modelling in system level simulation from RAN1 perspective:” can be removed. ” |
| Sony | There was a comment regarding the function “dBm” and “dB” during the GTW which I believe is also applicable here. Shouldn’t we use a more traditional expression like “10 log10” or if for whatever reason, we MUST insist, on this unconventional notation of “dBm” and “dB”, at least describe what these two functions are as I do not believe they are generally used notably people outside of 3GPP (assuming these will also be captured in the TR which will be read by people outside of 3GPP). |
| Nokia, NSB | We are fine with the proposal. |
| vivo | We are fine with the proposal. |
| Ericsson | We support the proposal in principle. “co-site” is misspelled as so-site in some places. There are two sections focusing on questions to ask RAN4. Perhaps, these could be combined in a seamless manner? Furthermore, how to model co-site interference is asked at two places. |
| QC | Genearally fine with the proposals. However, the proposal seems to be more detailed in terms of describing the adjacent-channel modeling as compared to co-channel proposal. Suggest consistent structure for both proposals. The bullet on ACIR is important though. |
| Intel | We are generally OK with the proposal. |
| NTT DOCOMO | We are fine with the proposal. |
| Samsung | Generally fine with the proposal. If RAN4 provide a feedback on an adjacent channel CLI modeing for SLS, RAN1 will use this model for both SBFD evaluation and d-TDD evaluation. Is this correct understanding? The text “For system-level simulation of adjacent-channel co-existence between legacy TDD operation and SBFD operation, a similar interference ratio as ACIR but in the subband of the adjacent carrier, with finer granularity (e.g., per subband or per RB or per sub-carrier) needs to be considered.” may mislead the CLI model is only applicable to SBFD evaluation. |
| CATT | We are fine with the proposal. |

# 5 Issue #4: Other evaluation methodology and assumptions

## 5.1 Issue #4-1: Performance metrics and high-level evaluation methodology

### 5.1.1 Submitted proposal

- Performance Metrics and Traffic Model

| Company | Proposals |
| --- | --- |
| CMCC | Proposal 23: One or multiple of the following metrics can be used for Phase-1 calibration - Metric 1: CDF of coupling loss (serving cell) from port 0 - Metric 2: CDF of DL wideband SINR considering legacy gNB-UE interference only - Metric 3: CDF of SBFD DL SINR before receiver considering legacy gNB-UE interference and co-channel UE-UE inter-subband CLI - Metric 4: CDF of legacy UL SINR before receiver considering legacy UE-gNB interference only - Metric 5: CDF of SBFD UL SINR before receiver considering legacy UE-gNB interference, self-interference and co-channel gNB-gNB inter-subband CLI - FFS: definition of the above metrics |
| ZTE | Proposal 6: To progress RAN1 study on duplex smoothly, - RAN1 starts identifying information that requires RAN4 input and sends LS to RAN4 as soon as possible. - RAN1 firstly calibrates geometry based on some simplified interference model defined by RAN1 and secondly calibrates geometry based on RAN4’s input once it is available. Proposal 10: For system level simulation for subband full duplex and dynamic TDD, - DL and UL need to be simulated simultaneously in the same system - Performance metrics: Cell capacity (i.e., average number of supported users per cell) - Traffic model: XR or FTP3 |
| Spreadtrum, BUPT | Proposal 2: At least the following metrics should be considered for SBFD study: - Geometry - Coverage - Delay - {5%,50%,95%,average} UPTs Proposal 3: Simulation assumptions including baseline, SBFD configuration, FTP model and interference modeling are recommended to be calibrated ASAP. |
| CATT | Proposal 10: Adopt DL/UL UPT and user plane latency as performance metrics for SBFD evaluation. Proposal 11: Adopt DL/UL UPT as performance metric for flexible/dynamic TDD evaluation. Proposal 14: Consider low, medium and high RU ratios in SBFD system evaluation. Proposal 15: Adopt simulation assumptions in Table 1 for SBFD system evaluation. |
| Vivo | Proposal 5: For NR duplex evolution, traffic configuration in Table 1 can be considered as the starting point. Table 1. Traffic configuration for evaluation of NR duplex enhancement Traffic model; Packet size; arrival rate λ; Traffic load; Ratio of DL/UL traffic. FTP3; 0.1Mbytes; Based on traffic load; Low:25% Medium:50% [high:80%, optional]; {2:1} {4:1} [{1:1}, optional]. 0.5Mbytes. 1.5Kbytes; 100p/s; --; --. Proposal 7: For fair comparison between semi-static SBFD and semi-static TDD, the DL-to-UL resource ratio is assumed to be the same. Proposal 9: The following performance metrics can be considered for NR duplex evolution, - Latency (mean, 5, 50, 95 %) - User-perceived throughput (mean, 5, 50, 95 %) - Resource utilization - RU = number of occupied resources for a given link directiontotal resources - CDF of received SINR |
| xiaomi | Proposal 6: At least the following metrics should be considered for performance evaluation: - UL/DL UPT - Latency of uplink packet transmission |
| Samsung | Proposal 7: For evaluation, RAN1 takes DL/UL UPT CDF and Latency CDF as a performance metric to draw a conclusion and recommendation in TR. - DL/UL received SINR can be used for calibration - FFS: other performance metrics Proposal 8: For evaluation purpose, RAN1 takes the deployment related parameters in Tables 2-4 as a starting point. |
| OPPO | Proposal 6: KPIs should be clarified to analyze the evaluation, preferably including statistics (including distribution etc) of SINR, throughput, spectrum effectiveness and CLI. |
| Ericsson | Proposal 16: In system level simulation, for the initial phase of performance evaluation comparison is performed between different duplex modes based on the same amount of input traffic within each networks. Proposal 17: For co-existence evaluations (e.g. between two networks), in addition to the case of same input traffic in the two networks, also consider high input traffic in the aggressor and low in the victim. Proposal 18: For traffic load in system level simulations, it is based on an aggregated system resource utilization between UL and DL denoted as rDLuDL+rULuUL, where r is the ratio of DL/UL resource allocation and u is the DL/UL resource utilization, i.e., the average ratio of used versus available resources in the respective direction. Proposal 19: For performance metrics to be used in the system level simulations, the following are proposed: - Resource utilization: Resource utilization per transmission direction - Coverage: Target Maximum Pathloss (MPL) to achieve a certain average bit rate (10Mbps for DL and 1Mbps for UL). - User Throughput: Mean user throughput and 5%ile user throughput Resource utilization per direction Proposal 20: RAN1 studies SBFD’s effects on DL and UL resource utilization per direction to provide better understanding of SBFD operations. Coverage Proposal 21: RAN1 studies SBFD’s effects on DL and UL coverage under different traffic loads / resource utilization levels to provide better understanding of SBFD operations. User Throughput Proposal 22: RAN1 studies SBFD’s effects on DL and UL mean and cell-edge user throughputs under different traffic loads and grid shifts to provide better understanding of SBFD operations. Proposal 23: RAN1 to agree the system level simulation parameters listed in Annex B. |
| LG | Proposal 8: Discuss DL and UL packet size and packet arrival rate for evaluation of subband non-overlapping full duplex and dynamic/flexible TDD. Proposal 9: Discuss whether DL and UL traffic is simultaneously generated for evaluation of subband non-overlapping full duplex and dynamic/flexible TDD. Proposal 11: Average UE throughput and average UE latency can be adopted as the performance metric for evaluation for subband non-overlapping full duplex and dynamic/flexible TDD. Discuss further details of definition of average UE throughput and average UE latency. |
| CEWiT | Proposal 2: Baseline TDD system with same DL to UL ratio can be used as reference for comparing the performance of SBFD system. Proposal 3: Define the various parameters (e.g., inter packet delay) for FTP 3 to be used in the evaluations. Proposal 4: Define the use cases for SBFD evaluation, a. latency reduction, b. UL SINR improvement. Proposal 5: The following evaluation metrics are supported: - a. Average latency for DL and UL - b. Average SINR in UL Proposal 6: Evaluation metric “latency” is defined as the time taken from the generation of a packet to the complete transmission of the packet (including HARQ transmission). |
| Intel | Proposal 3: - Simulation assumptions and evaluation methodologies as agreed during Rel-17 NR coverage enhancement SI can be considered as starting points for evaluation of coverage performance for NOFD operation. - Consider Table 1 and Table 2 in the Appendix I for NOFD performance evaluation for FR1 and FR2, respectively. - Self-interference modelling should be considered for link-level simulation for NOFD. Proposal 4:For target reliability and packet size assumptions, User Plane (UP) latency for UL and DL may be evaluated using numerical analyses while using results from link- and system-level evaluations to determine details of UL/DL scheduling flows for PUSCH and PDSCH scheduling. Proposal 5: For system-level evaluations on NOFD - Simulation assumptions, including deployment scenarios, antenna configurations, and related assumptions, as agreed during Rel-16 CLI/RIM, can be considered as starting points for system-level evaluations for NOFD operation. - Consider Table 3-7 in the Appendix II for NOFD evaluations in FR1 and FR2. - Non-full buffer with FTP traffic model 3 should be considered for traffic modelling. - DL/UL UPT should be used as the primary performance metric for SLS evaluations. |
| Qualcomm | Proposal 8: For subband full duplex deployment scenario, support to use DL/UL UPT and DL/UL transfer time as evaluation metrics. |
| DOCOMO | Proposal 2: Evaluation assumptions is derived by both UL heavy traffic scenario (e.g. eMBB), and coverage enhancement scenario (e.g. VoIP). |
| Nokia | Proposal 9: For the traffic models and KPIs for SBFD evaluations, assume the following: - FTP3 traffic model with large payload size, e.g. 0.5 Mbytes. - Main KPI: User perceived throughput (UPT). - Offered load selected to meet a certain resource utilization (RU) target, e.g. 10% (low load) and 40% (medium load). - Both DL-heavy traffic, 4:1, and symmetric 1:1 DL:UL traffic can be considered. - FTP3 traffic model with small payload size, e.g. 32-1500 Bytes or other similar traffic models as discussed during Rel-16 URLLC/IioT Study Item in TR 38.824. - Main KPIs: CDF of the latency successfully delivered FTP3 packets and/or percentage of Ues meeting certain latency and reliability requirement. - Arrival rate per UE can be fixed, e.g. to 100 packet/second in both UL and DL. - Note: Companies to explicitly account for half-duplex UE limitations preventing the UE to receive and transmit at the same time. |

- High-level evaluation methodology

| Company | Proposals |
| --- | --- |
| Huawei | Proposal 10: At least the following evaluation methodologies should be considered to evaluate Rel-18 NR duplex evolution: - Link budget analysis - Interference strength evaluation for Scenario 2-1, Scenario 3-3, and Scenario 1-3 - Coverage evaluation for Scenario 1-3 - Link level evaluation - Coverage evaluation for Scenario 1-3 - Interference suppression evaluation for Scenario 2-1, Scenario 3-3, Scenario 1-3 - System level evaluation - Coverage and capacity evaluation for Scenario 2-1, Scenario 3-3, and Scenario 1-3 Proposal 11: Reuse the evaluation methodology, assumptions, and metrics of link budget analysis in IMT-2020 self-evaluation and/or TR 38.830 of Rel-17 NR coverage enhancement for Rel-18 NR duplex evolution. Proposal 12: Reuse the evaluation methodology of link level evaluation in TR 38.830 of Rel-17 coverage enhancement for Rel-18 NR duplex evolution and adopt the evaluation assumptions and metrics in Table 6. Proposal 13: Reuse the system level evaluation methodology in ITU-R M.2412 for Rel-18 NR duplex evolution and adopt the evaluation assumptions and metrics in Table 7 for system level evaluation. Proposal 16: Capture the link budget results in Table 8-9 and the following observations into TR 38.858: - In-band UE-to-UE CLI can be negligible in Scenario 2-1 - Further enhancements are required to suppress the in-band BS-to-BS CLI in Scenario 2-1 Proposal 17: Capture the link budget results in Table 10-13 and the following observations into TR 38.858: - Inter-subband BS-to-BS CLI from small cells to small cells and inter-subband UE-to-UE CLI in small cells can be negligible in Scenario 3-3 - Further enhancements are required to suppress inter-subband BS self CLI in Scenario 3-3 - Further enhancements are required to handle the BS-to-BS blocking and UE-to-UE blocking issue in Scenario 3-3 Proposal 18: Capture the link budget results in Table 14-16 and the following observations into TR 38.858: - Inter-subband UE-to-UE CLI in Macro cells can be negligible in Scenario 1-3 - Further enhancements are required to handle inter-subband BS-to-BS CLI from Macro cells to Macro cells in Scenario 1-3 - Further enhancements are required to handle inter-subband BS self CLI in Scenario 1-3 - Further enhancements are required to handle the BS-to-BS blocking and UE-to-UE blocking issue in Scenario 1-3 |
| Ericsson | Proposal 1: Adopt a net effect model that captures the essential behaviours of a realistic DPD and PA combination with -45 dBc ACLR compliance. This requires input from RAN4. Proposal 2: Adopt a simple crest factor processing model, e.g., hard clipping, that captures the essential behaviours of a BS design to increase transmit power. This requires input from RAN4. Proposal 3 The self-interference channel should be odelled as a set of tapped delay lines directly from TX sub-array ports to RX sub-array ports. Proposal 4 Self-interference channel coefficients should be based on realistic setups supported by real measurements or high-fidelity electromagnetic (EM) evaluations. Proposal 5 The gNB receiver in an SBFD system needs to be carefully studied and modelled with respect to various aspects such as selectivity, linearity etc. Proposal 6 Considering there is no RAN4 requirement on inter-subband selectivity within the channel bandwidth, RAN1/4 needs to study whether the rejection of UL signal in the UL part of an SBFD carrier by a UE receiving DL signal in the DL part is the same as adjacent channel selecitivity requirement. Proposal 7 Send an LS to RAN4 requesting feedback on various radio and antenna modelling aspects that are required for RAN1 to establish evaluation assumptions for both system-level and link-level simulations. |
| Intel | Proposal 3: - Simulation assumptions and evaluation methodologies as agreed during Rel-17 NR coverage enhancement SI can be considered as starting points for evaluation of coverage performance for NOFD operation. - Consider Table 1 and Table 2 in the Appendix I for NOFD performance evaluation for FR1 and FR2, respectively. - Self-interference modelling should be considered for link-level simulation for NOFD. Proposal 4: For target reliability and packet size assumptions, User Plane (UP) latency for UL and DL may be evaluated using numerical analyses while using results from link- and system-level evaluations to determine details of UL/DL scheduling flows for PUSCH and PDSCH scheduling. |
| DOCOMO | Proposal 4: Evaluate link level performance with LLS for study on the bandwidth of subband and the bandwidth of guard band for DL and UL subbands - Parameters of “power difference”, “bandwidth of interference channels/subbands”, and “bandwidth of guard band” need to be studied and defined for the evaluation Proposal 5: Study and define modeling of emissions of interference signal at gNB and UE, respectively for LLS and SLS evaluations. Observations : - eMBB FR1 : Degradation of 0.1 dB is observed when guard band is 5 RBs and PSD difference is 30 dB, and no degradation is observed when guard band is 25 RBs with 10, 20, 30 dB PSD difference. - PSD difference and guard bandwidth are key parameters for the evaluation and it should be studied and defined - eMBB FR2 : Degradation of 2.7 dB, 0.3 dB are observed for 1 PRB and 6 PRBs guard band with 30 dB PSD difference, respectively. - Since required SNR for the target BLER is higher, wider guard band or small PSD difference is required - VoIP : Degradation of 3.0 dB, 0.6 dB are observed for 5 PRBs and 25 PRBs guard band with 40 dB PSD difference for FR1. - Larger bandwith of interference signals is one of the reasons for the degradation |
| Qualcomm | Proposal 6: Support SLS as main tool for the evaluation of subband full duplex study. Proposal 27: RAN 1 shall consider simulation parameters Table 6 FR1 evaluation on Dynamic/flexible TDD Proposal 28: RAN 1 shall consider simulation parameters in Tables 1, 3, and 7 for FR2 dynamic/flexible TDD evaluation. The bandwidth configuration dynamic/flexible TDD evaluation could be either all for DL or all for UL at least for FR2. |
| CATT | Proposal 3: System-level evaluation is used for Rel-18 SBFD and flexible/dynamic TDD evaluation. Proposal 9: Perform system platform calibration for SBFD evaluation with SINR CDF. Proposal 13: Co-channel co-existing of SBFD with legacy TDD is studied in RAN1 and adjacent channel co-existence is led by RAN4. |
| CMCC | Proposal 3: No calibration phase is needed for the evaluation of dynamic/flexible TDD. Proposal 4: Regarding the evaluation work of SBFD, if majority think it is necessary to have a calibration phase before performance evaluation, the following can be considered: - Phase-1: Conduct calibration based on RAN1’s assumption on the interference modelling for self-interference, gNB-gNB and UE-UE inter-subband CLI, gNB-gNB and UE-UE inter-operator (i.e. adjacent-channel) CLI. - Phase-2: Conduct performance evaluation based on RAN4’s input on the interference modelling for self-interference, gNB-gNB and UE-UE inter-subband CLI, gNB-gNB and UE-UE inter-operator (i.e. adjacent-channel) CLI. Proposal 22: One or multiple of the following scenarios can be considered for SBFD calibration: - Scenario A1 (FR1, FR2): Indoor hotspot with common UL/DL subband configuration - Scenario A2 (FR1, FR2): Urban Micro or Dense Urban micro layer with common UL/DL subband configuration - Scenario A3 (FR1): Urban Macro with common UL/DL subband configuration Proposal 23: One or multiple of the following metrics can be used for Phase-1 calibration - Metric 1: CDF of coupling loss (serving cell) from port 0 - Metric 2: CDF of DL wideband SINR considering legacy gNB-UE interference only - Metric 3: CDF of SBFD DL SINR before receiver considering legacy gNB-UE interference and co-channel UE-UE inter-subband CLI - Metric 4: CDF of legacy UL SINR before receiver considering legacy UE-gNB interference only - Metric 5: CDF of SBFD UL SINR before receiver considering legacy UE-gNB interference, self-interference and co-channel gNB-gNB inter-subband CLI - FFS: definition of the above metrics Proposal 24: Scenario specific parameters as shown in Table 12 can be considered as a starting point for Phase-1 calibration. |
| New H3C | Proposal 1: The system level simulation is used for evaluation on NR duplex evolution. Proposal 2: Simulation assumption on flexible duplex in TR38.802 is used as starting point for evaluation on NR duplex evolution. Proposal 3: Above-mentioned table 1, table 2 and table 2 related Simulation parameters are made as baseline for evaluation on NR duplex evolution. |

### 5.1.2 Summary

The performance metrics for SBFD or dynamic/flexible TDD evaluation are summarized as below:

- DL/UL UPT or User Throughput

  - CMCC, Spreadtrum, BUPT, CATT, vivo, xiaomi, Samsung, Intel, Qualcomm, Ericsson, OPPO, LG

- DL/UL Latency

  - Spreadtrum, BUPT, vivo, xiaomi, Samsung, LG, CEWiT, Intel, Qualcomm

- DL/UL received SINR for calibration

  - CMCC, Spreadtrum, BUPT, vivo, Samsung, OPPO, CEWiT

- Coverage, e.g., MPL

  - Spreadtrum, Ericsson

- Resource utilization

  - vivo, Ericsson

- Cell capacity

  - ZTE

Regarding performance metrics, moderator suggests Initial proposal 4-1-1.

Regarding the traffic model, the following are proposed by companies:

- FTP3

  - ZTE, vivo, Nokia, CATT, etc.

- XR

  - ZTE

- VoIP

  - DOCOMO

Ericsson suggests that:

- In system level simulation, for the initial phase of performance evaluation comparison is performed between different duplex modes based on the same amount of input traffic within each network.

- For co-existence evaluations (e.g. between two networks), in addition to the case of same input traffic in the two networks, also consider high input traffic in the aggressor and low in the victim.

ZTE suggests that DL and UL need to be simulated simultaneously in the same system.

Regarding traffic model for SLS, moderator suggests Initial proposal 4-1-2.

Regarding the high-level evaluation methodologies, most companies think SLS should be used for DL/UL UPT and latency evaluation. In addition to SLS, some companies [Huawei, Intel, Ericsson] also suggest to reuse the link level evaluation methodology in TR 38.830 (i.e., LLS + Link budget analysis) as a start to evaluate the coverage performance (e.g., MPL, MCL, MIL) for SBFD. One company [DoCoMo] suggests to use LLS for study on the bandwidth of subband and the bandwidth of guard band for DL and UL subbands for SBFD.

Regarding high-level evaluation methodologies, moderator suggests Initial question 4-1-3.

Some companies [ZTE, CATT, CMCC] suggest RAN1 to conduct SLS calibration before performance evaluation for SBFD as below:

- Phase-1: Conduct calibration based on RAN1’s assumption on the interference modelling for self-interference, gNB-gNB and UE-UE inter-subband CLI, gNB-gNB and UE-UE inter-operator (i.e. adjacent-channel) CLI.

- Phase-2: Conduct performance evaluation based on RAN4’s input on the interference modelling for self-interference, gNB-gNB and UE-UE inter-subband CLI, gNB-gNB and UE-UE inter-operator (i.e. adjacent-channel) CLI.

CMCC suggests the following metrics for RAN1 calibration of SBFD SLS platform:

- Legacy metrics:

  - Metric 1: CDF of coupling loss (serving cell) from port 0

  - Metric 2: CDF of DL wideband SINR considering legacy gNB-UE interference only

- New metrics:

  - Metric 3: CDF of SBFD DL SINR before receiver considering legacy gNB-UE interference and co-channel UE-UE inter-subband CLI

  - Metric 4: CDF of legacy UL SINR before receiver considering legacy UE-gNB interference only

  - Metric 5: CDF of SBFD UL SINR before receiver considering legacy UE-gNB interference, self-interference and co-channel gNB-gNB inter-subband CLI

Regarding SLS calibration for SBFD, moderator suggests Initial question 4-1-4.

### 5.1.3 1st Round Proposals (closed)

#### Initial proposal 4-1-1:

At least the following metrics are considered for SLS for SBFD and dynamic/flexible TDD evaluation.

- DL/UL UPT or user throughput (CDF or {mean, 5%, 50%, 95%})

- Latency (CDF or {mean, 5%, 50%, 95%})

- Resource utilization

- FFS: DL/UL received SINR for SLS calibration, if RAN1 concludes SLS calibration is needed

- FFS: definitions of the above metrics

- FFS: other metrics

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| QC | Support |
| NTT DOCOMO | We support. |
| New H3C | Support |
| CATT | Support |
| Spreadtrum | We think the coverage parameters, e.g., MCL/MIL/MPL, could also be taken into account. For one aspect of the potential benefit of full duplex is the uplink coverage improvement. To better justify the full duplex operation, coverage parameters are necessary. |
| ZTE | One question for clarification, is the “user throughput” the same as UPT? We think calibration is essential otherwise it is impossible to achieve common understandings among companies. |
| NEC | Agree with above metrics |
| vivo | We should encourage company to report DL/UL received SINR, even though we may not perform calibration, but SINR cdf can provide useful information for companies to better understand the results from each other. For the RU definition, we think the total resources including both DL and UL resources need to be considered, e.g., RU = number of occupied resources for a given link directiontotal resources |
| Sharp | We support the proposal. |
| Sony | Support |
| Xiaomi | Support |
| Huawei, HiSilicon | Support |
| Ericsson | One of the potential benefits of SBFD is coverage. Hence, we think that a metric for Coverage needs to be added. Another claimed benefit of both SBFD and dynamic TDD is latency improvements. In our view these claimed latency improvements stem from two effects. The first one being that the probability that UL (or DL) resources are available when data arrives is higher and thus there is less time wasted waiting for an appropriate resource. The second being that the system has higher capacity, e.g. because more UL resources can be allocated if needed, power limited Ues can be given more time-domain resources instead of frequency-domain resources and for the case of dynamic TDD, UL and DL resources will better match the actual need. Here we can observe that the first effect is mostly present at low load when there is no queueing. Also, the potential improvements, will depend heavily on detailed physical layer aspects, such as scheduling, PDCCH/PUCCH capacity/periodicity, scheduling request periodicity and so forth. Once the load gets high enough, what really matters is the system capacity as queuing delays will dominate. Thus, in our view, many of the latency aspects can either be evaluated analytically (low load) or will follow the same trends as the throughput metrics discussed above (high load). In addition, we are also concerned that due to the required level of detail to capture the first aspect, aligning results among companies will be challenging. Based on this discussion, we do not preclude latency metrics per se, but we think they should be of lower priority compared to throughput metrics. Therefore, we propose to modify the proposal as follows. #### Initial proposal 4-1-1(modified by Ericsson): At least the following metrics are considered for SLS for SBFD and dynamic/flexible TDD evaluation. - DL/UL UPT or user throughput (CDF or {mean, 5%, 50%, 95%}) - Coverage metric: MPL to achieve a certain bit rate in UL and DL () - Latency (CDF or {mean, 5%, 50%, 95%}) - Resource utilization - FFS: DL/UL received SINR for SLS calibration, if RAN1 concludes SLS calibration is needed - FFS: definitions of the above metrics - FFS: other metrics |
| Nokia/NSB | Support |
| Intel | Support |
| OPPO | Support |
| InterDigital | Support the FL proposal |
| Samsung | Support DL/UL UPT or user throughput and Latency. The resource utilization can be one of evaluation parameter as used in TS38.802 and 828. |
| KDDI | Support. |
| Moderator | @ZTE, user throughput may be the average UPT for a UE based on some companies’ contributions. Anyway, these definitions need to be further discussed. @Ericsson, Spreadtrum, based on the discussion on question 4-1-3, I think more discussion are needed regarding whether to reuse coverage performance metrics (e.g., MPL, MCL, MIL) defined in TR 38.830 for SBFD coverage evaluation. |

#### Initial proposal 4-1-2:

Regarding traffic model for SBFD and dynamic/flexible TDD evaluation, at least FTP3 is considered.

- FFS: other traffic models, e.g., XR, VoIP

- FFS: Packet size, traffic load, ratio of DL/UL traffic

- Performance evaluation comparison between different duplex modes (e.g., legacy static TDD vs. SBFD) should be performed based on the same amount of input traffic.

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| QC | Support. The third sub-bullet should be merged into the main bullet. |
| NTT DOCOMO | We support. |
| New H3C | Support |
| CATT | Support |
| Spreadtrum | Support. |
| ZTE | Support in principle. |
| NEC | Agree with above. Also, different values of traffic load and ratio of UL/DL traffic should be considered for study because interference ratio and expected improvement would be significantly dependent on these factors. |
| Vivo | URLLC type of traffic (periodical small packet) can also be evaluated |
| Sharp | We support the proposal. |
| Sony | Support |
| Xiaomi | Support |
| Huawei, HiSilicon | Support |
| Nokia/NSB | Support |
| Intel | Support. Also, we are open to consider periodic traffic with small packets, e.g., for URLLC. |
| OPPO | Agree. |
| InterDigital | Support |
| Samsung | Support. |
| Ericsson | For the purpose of system level simulations, in order to evaluate the impact to victim network from an aggressor network, one should consider including different loads in aggressor and victim networks, typically high load in the aggressor and low/medium/high in the victim. One of the SID is to study the impact to legacy systems, which needs to be less in all loads of the aggressor network. Therefore, we propose the following modifications – #### Initial proposal 4-1-2: (Modified by Ericsson) Regarding traffic model for SBFD and dynamic/flexible TDD evaluation, at least FTP3 is considered. - FFS: other traffic models, e.g., XR, VoIP - FFS: Packet size, traffic load, ratio of DL/UL traffic - Performance evaluation comparison between different duplex modes (e.g., legacy static TDD vs. SBFD) should be performed based on the same amount of input traffic. - Different loads for aggressor network (typically high load) and victim network(low/medium/high) to study impact to legacy systems. |
| KDDI | Support. |
| Moderator | @Ericsson, we can discuss the details of traffic load later |

#### Initial question 4-1-3:

Whether to reuse the link level evaluation methodology in TR 38.830 (i.e., LLS + Link budget analysis) as a starting point to evaluate the coverage performance of SBFD? Whether the similar performance metrics (e.g., MPL, MCL, MIL) defined in TR 38.830 for Rel-17 coverage enhancement are considered as the performance metrics for SBFD evaluation?

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| QC | We think it is beneficial to support link-budget and LLS to evaluate the proper impact on UE DL performance in presence of inter-UE CLI and impact on gNB UL reception in presence of SI. |
| NTT DOCOMO | We support to perform LLS as well as SLS, to study the necessity of guard band and appropriate number of PRBs for aggressor and victim channel bandwidth. |
| New | Support |
| CATT | More discussion would be needed on how to evaluate the potential coverage improvement via SBFD. |
| ZTE | We feel that LLS is beneficial for determination of guard band size, impact of different interference. But it is not necessary directly related to coverage. |
| Vivo | 5% UPT performance can already be used as the metric for coverage evaluation. We are open to additional link-budget analysis. For LLS, we are ok to reuse the evaluation methodology in TR 38.830 for SBFD. |
| Sharp | We are OK to reuse the methodology in TR38.830 if LLS would be done. |
| Xiaomi | We are wondering how the complicated interference in SBFD scenario can be reflected in the LLS evaluation methodology in TR38.830. |
| Huawei, HiSilicon | Uplink coverage improvement is one important benefit for SBFD. So it should be evaluated in detail. In SI of Rel-17 coverage enhancement, LLS and link budget analysis are used to evaluate the coverage. So these methods and corresponding metrics should be reused to evaluate the coverage performance of SBFD as a starting point. |
| Ericsson | We support the second half of the proposal to use Coverage metrics such as MPL, MCL for system level evaluations. Regarding link level evaluation methodology, we think that link level simulation is needed to study the self- interference suppression and cancellation. |
| Intel | Yes, it would be necessary to evaluate the coverage performance for SBFD as it is one of the performance metrics of interest for SBFD operation. The framework defined in Rel-17 SI on CE is a suitable starting point for our studies. |
| Moderator | Observation: It seems 4 companies [New H3C, Huawei, Ericsson, Intel] are supportive to reuse the coverage performance metrics (e.g., MPL, MCL, MIL) defined in TR 38.830 for coverage evaluation of SBFD. Some other companies are not supportive. Some companies are supportive to perform LLS and/or link budget analysis, but the motivations are divergent. It seems more discussion are needed regarding whether to reuse coverage performance metrics (e.g., MPL, MCL, MIL) defined in TR 38.830. |

#### Initial question 4-1-4:

Whether RAN1 needs to conduct SLS calibration for SBFD evaluation or not? What’s your view regarding taking the following metrics for SBFD SLS calibration as a starting point?

- Legacy metrics:

  - Metric 1: CDF of coupling loss (serving cell) from port 0

  - Metric 2: CDF of DL wideband SINR considering legacy gNB-UE interference only

- New metrics:

  - Metric 3: CDF of SBFD DL SINR before receiver considering legacy gNB-UE interference and UE-UE co-channel inter-subband CLI

  - Metric 4: CDF of legacy UL SINR before receiver considering legacy UE-gNB interference only

  - Metric 5: CDF of SBFD UL SINR before receiver considering legacy UE-gNB interference, self-interference and gNB-gNB co-channel inter-subband CLI

- FFS: detailed definitions of these metrics

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| QC | It is helpful to conduct SLS calibration for SBFD to align results across the companies. However, we are concerned about the impact on the progress of the study. |
| New | Support |
| CATT | Give that new interference types are introduced for SBFD, we think SLS calibration is needed. |
| Spreadtrum | We prefer to do calibration for SBFD using Metric2-5. And we think inter-sector inter subband CLI should be taken into account in Metric 5. |
| ZTE | All the five metrics are fine for us. |
| Vivo | We may not need a formal calibration process, but the SINR cdf can be reported by companies together with other metrics for cross-check. For SBFD case, how to model the self-interference and gNB-gNB co-channel inter-subband CLI need to be decided first. |
| Sharp | Calibration process is preferred. |
| Xiaomi | We agree that SLS calibration is important. Regarding the metrics raised by FL, we think they are useful as we can quickly find out where the problem is once bias results are collected. However, considering the workload, i.e., both simulation workload and calibration workload, maybe we don’t need to provide all of them. From our perspective, metric 3 and metric 5 can be used for calibration purpose. |
| Huawei, HiSilicon | Fine if majority support. |
| Ericsson | Yes, we need to consider SLS calibration for SBFD evaluation. Regarding the metrics for calibration, we support the proposal. |
| Nokia/NSB | We think companies should be welcome to provide calibration-related statistics together with their performance results, but we do not think this should be a prerequisite for doing the actual performance evaluation (as this could slow down significantly the SI progress) |
| Intel | We agree that calibration efforts would be beneficial and agree with the proposal in principle. To confirm our understanding, is it correct that “SINR before receiver” corresponds to pre-processing SINR based on large scale channels without considering any scheduling – i.e., geometry-like snapshots for random selection of one UE and/or gNB in each cell for transmission for a given drop. |
| OPPO | The calibration across companies is helpful. |
| InterDigital | Same understanding with Nokia |
| Samsung | We are okay to conduct SLS calibration for SBFD. |
| LG | Ok to conduct SLS calibration for SBFD |
| Moderator | Observations: Most companies think calibration is helpful, but some companies have concern on the workload, and some companies think calibration should not be a prerequisite for doing the actual performance evaluation since this could slow down significantly the SI progress. Regarding the metrics used for calibration, some companies think the 5 metrics or a subset of them can be used, but some companies think we can use the same metrics as in proposal 4-1-1. We can discuss this issue later. |

### 5.1.4 2nd Round Proposals (closed)

#### Updated proposal 4-1-1:

At least the following metrics are considered for SLS for SBFD and dynamic/flexible TDD evaluation.

- DL/UL UPT or user throughput (CDF or {mean, 5%, 50%, 95%})

- Latency (CDF or {mean, 5%, 50%, 95%})

- Resource utilization

- FFS: DL/UL received SINR for SLS calibration, if RAN1 concludes SLS calibration is needed

- FFS: Coverage metric: MPL to achieve a certain bit rate in UL and DL similar as in TR38.830

- FFS: definitions of the above metrics

- FFS: other metrics

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| New H3C | We are fine with this proposal |
| Intel | We are Ok with the original proposal. We think coverage evaluations are typically associated with LLS and link budget studies. |
| Samsung | We are fine with the proposal. Detail definitions are further discussed on the agreed performance metrics. |
| Vivo | We are fine with the proposal except the FFS for coverage metric. We wonder how to obtain such coverage metric by SLS. |
| CATT | The proposal is about SLS metrics while we do not think MPL is an SLS metric. In addition, it is not clear to us how to show the coverage gain for SBFD via MPL. |
| Sony | Support |
| Nokia, NSB | We support the proposal. |
| NEC | We are okay with the proposal |
| Huawei, HiSilicon | Agree with Intel that the coverage can be studied based on link budget and LLS. |
| Xiaomi | Agree with Intel. |
| Spreadtrum | We are fine with this proposal, and think SINR for calibration is necessary. |
| Ericsson | We want to highlight that Coverage metric MPL to achieve a certain bit rate could be used as a KPI for the system level simulations. We did not propose to do link budget analysis of coverage as was done in TR 38.830. In addition, since coverage is a potential benefit, we think coverage metric should not be under FFS. #### Updated proposal 4-1-1: (Modified by Ericsson) At least the following metrics are considered for SLS for SBFD and dynamic/flexible TDD evaluation. - DL/UL UPT or user throughput (CDF or {mean, 5%, 50%, 95%}) - Latency (CDF or {mean, 5%, 50%, 95%}) - Resource utilization - Coverage metric: MPL to achieve a certain bit rate in UL and DL similar as in TR38.830 - FFS: DL/UL received SINR for SLS calibration, if RAN1 concludes SLS calibration is needed - FFS: definitions of the above metrics - FFS: other metrics |
| NTT DOCOMO | We agree with Intel. |
| QC | Support |
| Moderator | Updated proposal 4-1-1a was provided based on companies’ comments. It seems companies are not clear how to derive MPL based on SLS. |

#### Updated proposal 4-1-2:

Regarding traffic model for SBFD and dynamic/flexible TDD evaluation, at least FTP3 is considered. Performance evaluation comparison between different duplex modes (e.g., legacy static TDD vs. SBFD) should be performed based on the same amount of input traffic.

- FFS: other traffic models, e.g., XR, VoIP

- FFS: Packet size, traffic load, ratio of DL/UL traffic

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| Intel | We are Ok with the updated proposal. |
| Samsung | Support |
| vivo | We are fine with this proposal |
| CATT | Support |
| Sony | Support |
| Nokia, NSB | We support the proposal. |
| Huawei, HiSilicon | Fine. |
| Xiaomi | Support. |
| Spreadtrum | Support |
| Ericsson | We support the proposal in principle. We think that to study the impact to legacy static TDD, we need different amount of input traffic for the aggressor and victim networks. Typically, the aggressor network has more input traffic so that the impact to victim networks can be evaluated. Typically, in such studies, It is not acceptable that the aggressor network coexists well with victim at low loads at the aggressor network but destroys the victim network at high loads. Therefore, we need to consider different amount of input traffic at least for adjacent channel/co-channel coexistence studies. |
| QC | Support |
| Moderator | Updated proposal 4-1-2a was provided based on companies’ comments. |

### 5.1.5 3rd Round Proposals (closed)

#### Updated proposal 4-1-1a:

At least the following metrics are considered for SLS for SBFD and dynamic/flexible TDD evaluation.

- DL/UL UPT or user throughput (CDF or {mean, 5%, 50%, 95%})

- Latency (CDF or {mean, 5%, 50%, 95%})

- Resource utilization

- FFS: DL/UL received SINR for SLS calibration, if RAN1 concludes SLS calibration is needed

- FFS: Coverage metric: MPL to achieve a certain bit rate in UL and DL

  - FFS: how to derive MPL based on SLS

- FFS: definitions of the above metrics

- FFS: other metrics

| Company | Comment |
| --- | --- |
| New H3C | We are fine with this proposal |
| ZTE | OK |
| Huawei, HiSilicon | The FFS bullet on the coverage metric can be separated out since the main bullet is for SLS. For coverage evaluation, the MPL calculation should be based on LLS. |
| Sony | Support |
| MediaTek | Fine with this proposal |
| InterDigital | Fine with the FL proposal |
| Intel | For the FFS related to coverage metric, as mentioned in prior comments we think coverage evaluations are typically associated with LLS and link budget studies, and we agree with Huawei and we think this should be a separate bullet dedicated for discussion related to LLS. |
| Nokia, NSB | We support this proposal. |
| QC | Support. Even if RAN1 didn’t agree to have calibration phase, the UL/DL SINR could be very useful to align the results across companies. FFS is fine though if majority think otherwise. |
| Fujitsu | Support |
| DOCOMO | We are fine with the proposal. |
| LG | We share the view with Huawei. |
| Vivo | We are fine with the proposal except the FFS bullet for coverage metric. We share the view with Huawei that it should be based on LLS. |
| CATT | We are fine with the proposal and agree with Huawei and vivo. |
| Ericsson | We support the proposal in principle. However, we think that Coverage is an important metric for system level evaluations as one of the potential benefits of SBFD is coverage. In our contribution, we provided the results for MPL to achieve a certain data rate for UL (1Mbps) and DL(10 Mbps). This metric was based on coverage enhancements SI, where link budget analysis was use to arrive at the MPL. However, in our system level simulations we simply used the metric on the results obtained from Throughput vs MPL plot that we have shown in out contribution. In our opinion, this can be used as the starting point for Coverage metric. #### Updated proposal 4-1-1a: At least the following metrics are considered for SLS for SBFD and dynamic/flexible TDD evaluation. - DL/UL UPT or user throughput (CDF or {mean, 5%, 50%, 95%}) - Latency (CDF or {mean, 5%, 50%, 95%}) - Resource utilization - FFS: DL/UL received SINR for SLS calibration, if RAN1 concludes SLS calibration is needed - FFS: Coverage metric: MPL to achieve a certain bit rate in UL and DL - FFS: how to derive MPL based on SLS - FFS: definitions of the above metrics - FFS: other metrics |
| Spreadtrum | We are generally fine with the proposal but we think calibration is necessary and would like to remove the FFS before DL/UL received SINR. |
| Xiaomi | OK |
| Samsung | We are fine with the proposal |
| Sony | Support |
| Moderator | Agreement At least the following metrics are considered for SBFD and dynamic/flexible TDD evaluation. - DL/UL UPT or user throughput (CDF or {mean, 5%, 50%, 95%}) using SLS - Latency (CDF or {mean, 5%, 50%, 95%}) using SLS - Resource utilization using SLS - DL/UL received SINR using SLS - Coverage metric - FFS: MPL to achieve a certain bit rate in UL and DL - FFS: definitions of the above metrics - FFS: other metrics |

#### Updated proposal 4-1-2a:

Regarding traffic model for SBFD and dynamic/flexible TDD evaluation, at least FTP3 is considered. Performance evaluation comparison between different duplex modes (e.g., legacy static TDD vs. SBFD) should be performed based on the same amount of input traffic.

- FFS: other traffic models, e.g., XR, VoIP

- FFS: Packet size, traffic load, ratio of DL/UL traffic

- FFS: additionally consider different amount of input traffic at least for adjacent channel/co-channel coexistence studies

| Company | Comment |
| --- | --- |
| ZTE | OK |
| Huawei, HiSilicon | Support |
| Sony | Support |
| MediaTek | Fine with this proposal |
| InterDigital | Fine with the FL proposal |
| Intel | We are OK with the proposal |
| Nokia, NSB | We support this proposal. |
| QC | Support. Regarding the FFS bullet, we need to clarify why different input traffic is needed for co-channel coexistence scenario. We could understand the motivation for consideration of different amount of input traffic for adjacent channel coex case across two operators. |
| Fujitsu | Support |
| DOCOMO | We are fine with the proposal. |
| LG | We are fine with the proposal. |
| Vivo | We are fine with the proposal |
| CATT | We support the FL proposal. |
| Ericsson | We support the proposal in principle. |
| Spreadtrum | Support. |
| Xiaomi | Support. |
| Samsung | Fine with the proposal. |
| Sony | Support |
| Moderator | Agreement Regarding traffic model for SBFD and dynamic/flexible TDD evaluation, at least FTP3 is considered. Performance evaluation comparison between different duplex modes (e.g., legacy static TDD vs. SBFD) should be performed based on the same amount of input traffic. - FFS: other traffic models, e.g., XR, VoIP - FFS: Packet size, traffic load, ratio of DL/UL traffic - FFS: additionally consider different amount of input traffic at least for adjacent-channel/co-channel coexistence studies |

## 5.2 Issue #4-2: SBFD Subband configuration

### 5.2.1 Submitted proposal

| Company | Proposals |
| --- | --- |
| CMCC | Observation 2: Two SBFD configuration can be considered, i.e., - SBFD configuration#1: DXXXU, with one UL subband between two DL subbands in slot X. - SBFD configuration#2: DXXXU, with one DL subband and one UL subband in slot X. Proposal 9: SFBD configuration#1 as well as the resource allocation pattern in slot X as following can be used for Phase-1 calibration. - Top/bottom DL subband BW in RB (x/2): 107 RB for FR1 and 103 RB for FR2. - Centre UL subband BW in RB (y): 53 RB for FR1 and 52 RB for FR2. - Guard band BW at one side in RB (z): 3 RB for both FR1 and FR2. |
| CATT | Proposal 8: FFS the guard band required between UL and DL subbands. Proposal 12: Adopt semi-static TDD as baseline for SBFD evaluation. - FFS dynamic TDD |
| xiaomi | Observation: Scenarios wherein different UL subband are configured across gNBs can be deferred until we have progress in section 9.3.2. |
| Samsung | Proposal 11: RAN1 takes the parameters for duplex evolution in Table 7. SBFD: DXXXU * X slot contains BWUL MHz for UL at the center and remaining bandwidth for DL, where BWUL = [20] MHz for FR1 and [40] MHz for FR2. * Guard band can be further considered to suppress inter-subband interference |
| OPPO | Proposal 2: For evaluation purpose, there can be two types of uplink-downlink subband allocation in a carrier: - Type 1: one or more downlink subband and one or more uplink subband within one symbol/slot - Type 2: only one downlink subband and only one uplink subband within one symbol/slot |
| Panasonic | Proposal 1: The following assumptions can be considered for link-level simulation. - Scenarios used for coverage enhancement evaluation can be a starting point. - Typical subband allocation can be that edges of a band are for DL and the center of a band is for UL. - For DL evaluation, CLI due to time difference between DL and UL symbols needs to be taken into account if DL and UL are assumed as in-band. Observation 1: Guard band can mitigate inter-UE CLI from adjacent subband. Observation 2: CLI measurement/repot from UE would be useful for the adjustment of guard band. |
| DOCOMO | Proposal 3: Frequency allocation of UL subband at middle PRBs is prioritized for the study and evaluation. |
| LG | Proposal 6: Discuss the location and ratio of wideband half-duplex resource and sub-band full duplex resource for evaluation of subband non-overlapping full duplex. Proposal 7: Discuss DL/UL subband size, and guard band size for evaluation of subband non-overlapping full duplex. |
| MediaTek | Proposal 7: For SBFD, UL subbands are assumed in DL slots. The UL subbands can be in the centre of the channel bandwidth or at the edge channel bandwidth. Proposal 8: DL subbands in UL slots are not considered for SBFD scheme. |
| Qualcomm | Observation 8: For SLS evaluation, SBFD is transparent to the UE where all slots are flexible from UE perspective. gNB dynamically schedules the UE within the UL or DL subbands of the SBFD slot. - Full band CSI (SRS and CSI-RS) can be enabled at some non-SBFD symbols Proposal 12: Support the following slot format configurations for the evaluation of subband full duplex. - For subband full duplex deployment scenario, use same full duplex slot format XXXXX (X=FD=D+U). - Note: all slots are flexible from the UE perspective. - For legacy TDD deployment scenario, use DDDSU as defined in Table A.1-2 of 38.838. Proposal 13: For FR2, for legacy TDD deployment scenario and subband full duplex deployment scenario, - Support periodic reserved DL-only slots and UL-only slots for common control channels - E.g. 20 slots per 20 ms for SSB, 20 slots per 160 ms for PRACH Proposal 14: For subband full duplex deployment scenario, support configurable N1 RBs DL subbands, N2 RBs UL subbands and N3 RBs as the gap between the DL and UL subbands - Option 1: Support ~40% RBs for each of the two DL subbands (N1=2x~40% RBs) and ~20% RBs for UL subband in middle (N2=~20% RBs) and guard band in between - N or 0 RB for the gap between DL and UL subbands (N3=2xN or 0 RB) - Option 2: For FR2, support fully overlapping DL/UL band configuration (N1=N2=entire BW and N3=0) |
| Huawei | Proposal 14: Realistic deployment limitations should be considered for the baseline of the evaluation, such as the backhaul delay, antenna port radiation pattern, etc. Proposal 15: Realistic deployment configurations and fair configurations should be taken into account for the baseline selection, such as legacy TDD with frame structure as DDDSU and DDSUU. |
| Vivo | Proposal 6: For the study of performance gain by SBFD, following baseline schemes can be compared with - 1st baseline: TDD with semi-static UL-DL configuration. - 2nd baseline: dynamic TDD Proposal 7: For fair comparison between semi-static SBFD and semi-static TDD, the DL-to-UL resource ratio is assumed to be the same. Proposal 8: For SBFD with dynamic frequency format, the baseline is case 2-1. - Case 2-1: Dynamic TDD (Baseline 2) |

### 5.2.2 Summary

In time domain, the following frame structures for SBFD are suggested by companies [CMCC, Samsung, Qualcomm, etc.]:

- Frame structure#1: DXXXU, where, X is the SBFD slot with DL subband(s) and UL subband.

  - Regarding using DL-only for the first slot, one reason is that some DL-only slots may be needed for transmitting SSBs and it may not available for SBFD operation, and another reason is that it can be used to schedule wideband (e.g., full bandwidth) DL transmission or for legacy UE not supporting SBFD operation. The reason to use UL-only for the last slot is that DDDSU is commonly used in commercial static TDD network, and it is not desired to introduce DL subband in the last uplink slot since it will cause gNB-gNB inter-operator CLI which may degrade the network performance of the adjacent operator.

- Frame structure#2: XXXXU

- Frame structure#3: XXXXX

Regarding the frame structures for SBFD evaluation, moderator suggests Initial proposal 4-2-1.

In frequency domain, two types of SBFD subband configurations in SBFD slots are proposed by companies:

- Subband configuration#1: SBFD slot consists of one UL subband at the center of the channel bandwidth and two DL subbands at two sides of the channel bandwidth

  - CMCC, Samsung, OPPO, MediaTek

- Subband configuration#2: SBFD slot consists of one UL subband at one side of the channel bandwidth and one DL subband at the other side of the channel bandwidth

  - CMCC, OPPO, Panasonic, DOCOMO, MediaTek, Qualcomm

These two SBFD subband configurations can be described using {BD, BU, BG} as below:

- Subband configuration#1: {BD, BU, BG}={2*ND, NU, 2*NG}

- Subband configuration#2: {BD, BU, BG}={ND, NU, NG}

- BD: the total number of RBs in all DL subband(s)

- BU: the total number of RBs in all UL subband(s)

- BG: the total number of RBs in all the guard band(s) between UL and DL subbands

- ND: the number of RBs in one DL subband

- NU: the number of RBs in one UL subband

- NG: the number of RBs in one guard band between one UL subband and one DL subband

Regarding the SBFD subband configuration, moderator suggests Initial proposal 4-2-2.

Subband configuration#1						Subband configuration#2

Regarding the performance evaluation and comparison between legacy TDD operation and SBFD operation, the following two options are proposed by companies as the baseline for legacy TDD operation:

- Opt 1: semi-static TDD

  - Huawei, CATT, vivo, xiaomi, CEWiT

- Opt 2: dynamic TDD

  - vivo (2nd priority)

For Opt 1 (semi-static TDD), some companies [vivo, CEWiT] suggest to use the same DL-to-UL resource ratio between SBFD and semi-static TDD.

Moderator suggests Initial proposal 4-2-3.

### 5.2.3 1st Round Proposals (closed)

#### Initial proposal 4-2-1:

Consider at least one of the following TDD frame structures for SBFD evaluation:

- Frame structure#1: DXXXU, where X denotes the SBFD slot with DL subband(s) and UL subband.

- Frame structure#2: XXXXU, where X denotes the SBFD slot with DL subband(s) and UL subband.

- Frame structure#3: XXXXX, where X denotes the SBFD slot with DL subband(s) and UL subband.

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| QC | Support frame structure #3 as baseline. For FR2, for legacy TDD, SBFD and dynamic TDD, support periodic reserved D-only and U-only slots for common control channels, e.g. 20 slots per 20ms for SSB, 20 slots per 160ms for PRACH. |
| NTT DOCOMO | We support, and we prefer structure #2/#1. |
| New H3C | Support |
| CATT | We think we can start with frame structure #1. |
| Spreadtrum | Support Frame structure #1 as baseline, where legacy DL slot and UL slot should be reserved. Legacy downlink slot is convenient for cell specific configuration and scheduling. So we think frame structure with downlink slot should be adopted in evaluation. Besides, if we also allow the change of legacy UL slot into SBFD slot, the uplink performance might be affected considering half-duplex issue at UE side. |
| ZTE | We support Frame structure#1 and structure#2, which can provide backward compatibility for legacy UEs. |
| NEC | Agree with listed cases |
| vivo | all the three frame structures can be evaluated, company should report their assumption about frame structure. |
| Sharp | We support the proposal. |
| Sony | Do we need Frame structure#2, which does not seems to differ much from Frame structure#1 and Frame structure#3? |
| Xiaomi | Frame structure#1 and #2 are preferred, which are more realistic. |
| Huawei, HiSilicon | Support |
| Ericsson | For the purpose of SBFD evaluation, two operator scenario cases with same TDD configuration as the static TDD needs to be used for the SBFD network. On that regard, Frame structure 2 is appropriate. All the D slots are SBFD slots with an UL subband and an UL-only slot also allowed. As a second stage of evaluations, frame structure #1 could also be considered. Frame structure #3 brings gNB-gNB interference to legacy TDD systems and must be deprioritized. In addition, we think the traffic split also needs to be the same split as the time resource allocation. |
| Nokia/NSB | Support |
| Intel | We are OK with the proposal, and slightly prefer frame structure #1, since this may be a more typical structure with same reasoning captured by the FL. |
| OPPO | We would have a concern on defining a slot type with both DL and UL in it, and making it observable to UE. We wonder how this evaluation condition is applied when the evaluation contains legacy UE as well. We are ok it is made clear the above frame structures are all from gNB perspective. |
| InterDigital | All the listed three frame structures, for evaluation purposes, can be considered by companies for their evaluations. Companies can describe their assumed frame structures. |
| Samsung | We slightly prefer frame structure#1 since it is more balanced and additional UL resources can be available without degrading peak DL performance (using the first slot). As a second preference, frame structure#2 is acceptable. But, we do not support frame structure #3, which brings additional (inter-cell) gNB-gNB co-channel intra-subband CLI in the co-channel co-existence scenario (Deployment Case 3) and gNB-gNB adjacent-channel CLI in the adjacent channel co-existence scenario (Deployment Case 3). Also, UL performance of msg3 PUSCH or PUCCH of msg4 PDSCH is degraded due to limited frequency hopping bandwidth. |
| KDDI | We prefer frame structure #1 and #2. |
| Moderator | About 8 companies prefer #1, about 5 companies prefer #2, some companies think all three can be considered. Small update to address OPPO’s concern. |

#### Initial proposal 4-2-2:

Consider two SBFD subband configurations as below:

- SBFD Subband configuration#1: {BD, BU, BG}={2*ND, NU, 2*NG}, which means one SBFD slot consists of one UL subband at the center of the channel bandwidth and two DL subbands at two sides of the channel bandwidth.

- SBFD Subband configuration#2: {BD, BU, BG}={ND, NU, NG}, which means one SBFD slot consists of one UL subband at one side of the channel bandwidth and one DL subband at the other side of the channel bandwidth.

- BD: the total number of RBs in all DL subband(s)

- BU: the total number of RBs in all UL subband(s)

- BG: the total number of RBs in all the guard band(s) between UL and DL subbands

- ND: the number of RBs in one DL subband

- NU: the number of RBs in one UL subband

- NG: the number of RBs in one guard band between one UL subband and one DL subband

- FFS: which configuration(s) is(are) used for evaluation.

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| QC | For SLS evaluation, support subband configuration #1. For FR2, support fully overlapped full duplex configuration as optional evaluation. |
| New H3C | Support |
| CATT | Support and different configurations may be used for different scenarios. |
| Spreadtrum | Support FL’s proposal. |
| ZTE | We support both of the two subband configurations. |
| NEC | We would like to prioritize SBFD subband configuration-1 for the Rel-18 study item to reduce the evaluation efforts. If configuration#2 is agreed for study, then it needs to be clarified whether additional guard (apart from what is indicated above) band is required for this configuration. This is to mitigate interference cause by UL transmissions from the UL subband to adjacent carriers deployed by another operator. |
| Vivo | Agree Both configuration #1 and configuration #2 can be evaluated. Company shall report their assumption on the configuration and the size of each sub-band, including the guardband size. |
| Sharp | We support the proposal. |
| Sony | Support |
| Xiaomi | Support. Regarding which configuration(s) is (are) used for evaluation, our preference is configuration#1. If the inter-band interference is not considered in the SLS, the only difference between configuration#1 and configuration#2 is the guard band overhead, which can be reflected by quantitative calculation if either configuration is simulated. |
| Huawei, HiSilicon | Both configurations can be evaluated. |
| Ericsson | For purpose of system level evaluations., consider SBFD configuration #1 initially. |
| Nokia/NSB | Support |
| Intel | We support the proposal, and for evaluations both configurations could be considered. |
| OPPO | Fine with the proposal. |
| InterDigital | All the listed SBFD configurations, for evaluation purposes, can be considered by companies for their evaluations. Companies can describe their assumed SBFD configurations. |
| Samsung | Support to SBFD subband configuration #1. |
| KDDI | Considering an interference impact to the adjacent carrier, we support SBFD Subband configuration #1. |
| Moderator | It seems this proposal is agreeable. I moved it to section 6 as stable. |

#### Initial proposal 4-2-3:

For performance evaluation and comparison between legacy TDD operation and SBFD operation under SBFD Deployment Case 1 (Non-coexistence case with single SBFD subband configuration), the following is considered as the baseline for legacy TDD operation:

- Baseline: Static TDD UL/DL configuration with {DDDSU}, where S=10D:2G:2U

  - FFS: whether other TDD UL/DL configurations ((e.g., the same DL-to-UL resource ratio between SBFD and static TDD) can optionally be used as the baseline.

  - FFS: whether dynamic TDD can optionally be used as the baseline.

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| QC | Support |
| New H3C | Support |
| CATT | Support |
| Spreadtrum | Support |
| ZTE | We think dynamic TDD can also be adopted as baseline. It would be good to see the comparison between dynamic TDD and subband full duplex and figure out the target scenarios for them. Regarding the TDD configuration, it is related to the frequency. Maybe we should first decide the frequency for simulation first. |
| NEC | Agree |
| vivo | The static TDD configuration can be used as the baseline for performance comparison. Whether to compare SBFD with dynamic TDD is up to company. |
| Sharp | We support the proposal. |
| Sony | Support |
| Xiaomi | Support. |
| Huawei, HiSilicon | We think the design of baseline should also consider the fairness of comparison. The uplink resources for baseline (legacy TDD) should be the same as the uplink resources for SBFD. For example, the baseline should be DDDSU if the slot format of SBFD is XXXXX with 20% uplink subband bandwidth, and the baseline should be DDSUU if the slot format of SBFD is XXXXU with 20% (or 25% more precisely) uplink subband bandwidth. |
| Ericsson | We support the proposal. |
| Nokia/NSB | Support |
| Intel | We are OK with the proposal. |
| OPPO | We share the same view as Huawei and ZTE. The assumption of a semi-static UL-light baseline could be too favor in showing gains by running full duplex. |
| Samsung | Support to main bullet. |
| KDDI | Support. |
| LG | Support |
| Moderator | Based on ZTE’s comment, initial proposal 4-2-3a is added. Proposal 4-2-3b is updated based on companies’ comments. |

### 5.2.4 2nd Round Proposals (closed)

#### Updated proposal 4-2-1:

Consider at least one of the following TDD frame structures from gNB perspective for SBFD evaluation:

- Frame structure#1: DXXXU, where X denotes the SBFD slot with DL subband(s) and UL subband.

- Frame structure#2: XXXXU, where X denotes the SBFD slot with DL subband(s) and UL subband.

- FFS: Frame structure#3: XXXXX, where X denotes the SBFD slot with DL subband(s) and UL subband.

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| New H3C | We are fine with this proposal |
| Intel | We are OK with the proposal. |
| Samsung | We support Frame structure#1 and ok to use structure#2. But, we do not support Frame structure#3 considering additional CLI in legacy U slot. One clarification is that when legacy TDD slot use DDDSU, where S has 10D:2Gap:2U, the forth X slot means introduction of UL subband in the first D and Gap symbols and not intended to allow DL subband in the last 2U symbols. To reflect this, we suggest to the following modification. Consider at least one of the following TDD frame structures from gNB perspective for SBFD evaluation: - Frame structure#1: DXXXU, where X denotes the SBFD slot, where UL subband is configured in all DL symbols and flexible symbolswith DL subband(s) and UL subband. - Frame structure#2: XXXXU, where X denotes the SBFD slot, where UL subband is configured in all DL symbols and flexible symbolswith DL subband(s) and UL subband. - Frame structure#3: XXXXX, where X denotes the SBFD slot, where UL subband is configured in all DL symbols and flexible symbols and DL subband(s) is(are) configured in all UL symbolswith DL subband(s) and UL subband. |
| Vivo | We are fine with this proposal. Besides, it would be better to add a note that company should report their assumption about frame structure. |
| CATT | We also do not see the need for frame structure #3. |
| Sony | I think we can remove either Frame Structure #2 or Frame Structure #3. |
| Nokia, NSB | We support the proposal. |
| NEC | The proposal seems good to us |
| Huawei, HiSilicon | Fine. |
| Xiaomi | It seems frame structure#1 and frame structure#2 has significant more supports. Can we put structure#3 in FFS? |
| Spreadtrum | Support Frame structure #1 as baseline, where legacy DL slot and UL slot should be reserved. |
| Ericsson | We are fine with the proposal. |
| NTT DOCOMO | We support the proposal. |
| QC | Support frame structure #3 as baseline. In FR2, for legacy TDD, SBFD and dynamic TDD, support periodic reserved D-only and U-only slots for common control channels, e.g. 20 slots per 20ms for SSB, 20 slots per 160ms for PRACH. |
| Moderator | The proposal 4-2-1 has been merged to updated proposal 4-2-3c. |

#### Initial proposal 4-2-3a:

For SBFD simulation, consider 4GHz for FR1 and 30GHz for FR2-1.

#### Updated proposal 4-2-3b:

For performance evaluation and comparison between legacy TDD operation and SBFD operation under SBFD Deployment Case 1 (Non-coexistence case with single SBFD subband configuration), down-select from the following options:

- Option 1:

  - Legacy TDD: Static TDD UL/DL configuration with {DDDSU}, where S=10D:2G:2U

  - SBFD: Frame structure#1 (DXXXU), SBFD UL subband is about 20MHz

- Option 2:

  - Legacy TDD: Static TDD UL/DL configuration with {DDDSU}, where S=10D:2G:2U

  - SBFD: Frame structure#2 (XXXXU), SBFD UL subband is about 20MHz

- Option 3:

  - Legacy TDD: Static TDD UL/DL configuration with {DDSUU}, where S=10D:2G:2U

  - SBFD: Frame structure#2 (XXXXU), SBFD UL subband is about 20MHz

- Option 4:

  - Legacy TDD: Static TDD UL/DL configuration with {DDDSU}, where S=10D:2G:2U

  - SBFD: Frame structure#3 (XXXXX), SBFD UL subband is about 20MHz

- FFS: whether dynamic TDD can optionally be used as the baseline for legacy TDD.

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| Intel | We are OK with both Proposal 4-2-3a and 4-2-3b |
| Samsung | Support to Initial proposal 4-2-3a But, in the updated proposal 4-2-3b, we are not supportive of option 3 and option 4, which allow DL subband in UL symbols. |
| Vivo | We are fine with proposal 4-2-3a. For proposal 4-2-3b, we prefer the original proposal 4-2-3. For SBFD, we don’t think these options need to be down-selected. According proposal 4-2-1, all the frame structures can be considered. We suggest that the frame structures for SBFD evaluation can be reported by companies. Regarding the subband size for UL, it will be depending on the system bandwidth and the traffic load between DL and UL. So it can be further discussed. |
| CATT | Is the proposal for FR1 only or for both FR1 and FR2-1? What is the bandwidth of the carrier? For each frame structure, whether a D/X/U represents one slot or two slots? For option 3 and 4, is the intention to also support DL subband in UL slot? |
| ZTE | It is not clear how we get the number of 20MHz in the above proposals. More clarification is needed. |
| Sony | Support Proposal 4-2-3a For Proposal 4-2-3b, we prefer Option 1 & 2. Option 3 & 4 isn’t that much different to Option 2 and we think they are not worth consuming companies’ limited time on them. |
| NEC | The proposal seems okay to us |
| Huawei, HiSilicon | Option 3 and 4 should be supported at least considering the fairness of comparison. |
| Xiaomi | Support Initial proposal 4-2-3a. For proposal 4-2-3b, we share same view with Samsung. |
| Spreadtrum | Suggest to add bandwidth of the carrier to make 20MHz UL subband meaningful. Also agree with Samsung to deprioritize option 3 and option 4. |
| Ericsson | We are fine with the proposal. In our opinion, Option 2 is the baseline. All the D slots are replaced by SBFD slots and the U slot is common for both Static TDD and SBFD. This provides fair comparison and highlights if there are any UL coverage improvements due to the increased amount of UL resources for SBFD. However, in general, we are ok with option 1,2 and 3. |
| QC | Support Proposal 4-2-3a Support option 4 for proposal 4-2-3b. |
| Moderator | I assume 4-2-3a is stable. Updated proposal 4-2-3c has been provided to based on companies’ comments on proposal 4-2-1 and 4-2-3b. |

### 5.2.5 3rd Round Proposals (closed)

#### Updated proposal 4-2-2a:

For SBFD evaluation, consider the following for SBFD subband configurations:

- Baseline: SBFD Subband configuration#1 with {DUD} pattern, which means one SBFD slot consists of one UL subband at the center of the channel bandwidth and two DL subbands at two sides of the channel bandwidth.

- Optional: SBFD Subband configuration#2 with {DU} pattern, which means one SBFD slot consists of one UL subband at one side of the channel bandwidth and one DL subband at the other side of the channel bandwidth.

- Use the following parameters for description of SBFD subband configuration in evaluation assumptions:

  - ND: the number of RBs in one DL subband

  - NU: the number of RBs in one UL subband

  - NG: the number of RBs in one guard band between one UL subband and one DL subband

| Company | Comment |
| --- | --- |
| New H3C | We are fine with this proposal |
| ZTE | Currently, it is not clear which option is more appropriate for deployment. We are ok with these two options, but we are not ok to list the second one as optional. |
| Huawei, HiSilicon | We think it may be okay to just label these two options and option 1 and option 2. Both options can be evaluated. |
| Sony | For the case where there are 2 DL subbands, I take it we can configure 2 ND values in a real deployment but for simulation purpose we assume both DL subbands have the same bandwidth. |
| MediaTek | Fine with this proposal |
| InterDigital | Fine with the FL proposal |
| Intel | We are Ok with the proposal. |
| Nokia, NSB | We have similar views as ZTE and Huawei. |
| QC | Support |
| Fujitsu | Support |
| DOCOMO | We are fine with the proposal. |
| LG | We are fine with the proposal. |
| Vivo | We are fine with the proposal |
| CATT | We support the FL proposal. |
| Ericsson | We support the proposal. |
| Spreadtrum | We have similar views as ZTE and Huawei. |
| Xiaomi | Generally fine with the proposal. One clarification: the major difference between {DUD} and {DU} subband configuration is the suppression capability on the interference from the adjacent band. Do we need to consider the interference from adjacent band for this two cases? |
| Samsung | We are fine with ths proposal. |

#### Initial proposal 4-2-3a (No update):

For SBFD simulation, consider 4GHz for FR1 and 30GHz for FR2-1.

#### Updated proposal 4-2-3c:

For performance evaluation and comparison between legacy TDD operation and SBFD operation under SBFD Deployment Case 1 (Non-coexistence case with single SBFD subband configuration), consider the following alternatives:

- Optional: Alt 1:

  - Legacy TDD: Static TDD UL/DL configuration with {DDDSU}, where S=10D:2G:2U

  - SBFD: Frame structure#1 (DXXXU), where X denotes a SBFD slot. In time domin, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about 20% of the channel bandwidth.

- Baseline: Alt 2:

  - Legacy TDD: Static TDD UL/DL configuration with {DDDSU}, where S=10D:2G:2U

  - SBFD: Frame structure#2 (XXXXU), where X denotes a SBFD slot. In time domin, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about 20% of the channel bandwidth.

- Optional: Alt 3:

  - Legacy TDD: Static TDD UL/DL configuration with {DDSUU}, where S=10D:2G:2U

  - SBFD: Frame structure#2 (XXXXU), where X denotes a SBFD slot. In time domin, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about 20% of the channel bandwidth.

- Optional: Alt 4:

  - Legacy TDD: Static TDD UL/DL configuration with {DDDSU}, where S=10D:2G:2U

  - SBFD: Frame structure#3 (XXXXX), where X denotes a SBFD slot. In time domin, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about 20% of the channel bandwidth.

- FFS: whether dynamic TDD can optionally be used for legacy TDD for comparison.

| Company | Comment |
| --- | --- |
| ZTE | We are not sure how the percentage of UL subband is derived. If the intention is to keep the same amount of UL resource between SBFD and legacy TDD, then it seems the perctange should be modified. For Alt.1, Alt.2 and Alt.4, the perctange should be 22.8%. For Alt.3, the perctange should be 42.9%. From our perspective, Alt.1, Alt.2 and Alt.3 should be baseline. |
| Huawei, HiSilicon | There is a typo in option 2, “domain” Alt 3 and Alt 4 should be supported at least considering the fairness of compairons (under same UL resources). |
| Sony | We do not see the need for so many different SBFD Frame Structures but since most are optionals we are fine with the proposal. |
| MediaTek | We have clarification question regarding the proposal, SBFD Deployment Case 1 is meant to evaluate the “non-coexistence” scenario. Shouldn’t this be related to Deployment Case 4? |
| Intel | We are OK with the proposal. |
| Nokia, NSB | As commented by other companies, we think that either Alt. 3 or Alt. 4 should be considered as “baseline” as these two have the same amount of UL resources. |
| QC | We think that Alt 4 should be the baseline. One minor comment, on the main bullet, it will be good to add word ‘baseline’ for further clarification. For performance evaluation and comparison between baseline legacy TDD operation and SBFD operation under SBFD. |
| ZTE2 | Please ignore our previous comment in this table, we did some wrong calculation… We are ok to make Alt.3 or Alt.4 as baseline. |
| Fujitsu | Support. |
| DOCOMO | We are fine with the proposal. |
| LG | We are fine with the proposal. |
| Vivo | Alt. 1/2/4 with different SBFD frame structures should be baseline. For SBFD, since some DL resources are changed to UL resources, the DL performance is expected to be impacted. Therefore, both the improvement for UL and the impact on DL should be evaluated for SBFD. Hence, the DL-to-UL resource ratio is assumed to be the same for both legacy TDD and SBFD to have a fair comparison. For Alt.1 and Alt.2, it seems not possible to keep the same DL-to-UL resource ratio between both legacy TDD and SBFD. So, Alt.4 that can ensure the same DL-to-UL resource ratio between both legacy TDD and SBFD should also be evaluated. Besides, it is not clear about the determination of SBFD UL subband size and whether the guard band is included or not. We think it can be reported by companies. |
| CATT | We think keep one alternative between Alt 3 and Alt 4 is enough. |
| Ericsson | We support the proposal. |
| Spreadtrum | It will be good to further clarify about the percentage of UL subband. Considering the fairness of comparison, the same DL/UL resource ration with baseline legacy TDD should be adopted, only Alt 4 meet the condition. But R18 duplex is used to improve the performance of uplink, it is not reasonable to have downlink subband in UL only slot, so we’d rather not to take Alt 4 as baseline. |
| Xiaomi | boutication on alt 3 and alt 4, considering slot#3 in alt 3 and slot#4 in alt 4 are UL slots, why UL subband is needed? |
| Samsung | For Alt1 and alt 2, we still have a concern on the last 2 UL symbol in the S slot. DL subband should not configured in the last 2 UL symbols in the S slot. For Alt 3, and Alt4, is the intention to allow DL subband in the UL slot (for Alt3, the 4th UL slot, for Alt4, the last UL slot)? Unclear to us if “SBFD UL subband spans all the symbols in a SBFD slot” includes DL subband in UL slot. |
| OPPO | UL resource should be same for legacy TDD case and SBFD case. The ratio of UL subbands for SB FD need to be modified. And the UL symbols in “S” slot should also be considered. |

### 5.2.6 5th Round Proposals (open)

#### Updated proposal 4-2-2b:

For SBFD evaluation, consider the following for SBFD subband configurations:

- Baseline: SBFD Subband configuration#1 with {DUD} pattern, which means one SBFD slot consists of one UL subband at the center of the channel bandwidth and two DL subbands at two sides of the channel bandwidth.

- Optional: SBFD Subband configuration#2 with {DU} pattern, which means one SBFD slot consists of one UL subband at one side of the channel bandwidth and one DL subband at the other side of the channel bandwidth.

- Use the following parameters for description of SBFD subband configuration in evaluation assumptions:

  - ND: the number of RBs in one DL subband

  - NU: the number of RBs in one UL subband

  - NG: the number of RBs in one guard band between one UL subband and one DL subband

| Company | Comment |
| --- | --- |
| Moderator | Some companies (ZTE/Huawei/Nokia/Spreadtrum) prefer to consider both two options as baseline, so I updated the proposal that both of two configurations can be considered in the simulation. |
| ZTE | Support |
| New H3C | In general, we are fine with this proposal. I wonder whether we need consider UDU pattern. |
| MediaTek | Support. |
| Xiaomi | The major difference between {DUD} and {DU} is the capability to suppress interference from adjacent band. If these two configurations are both simulated, what is the difference for SBFD without adjacent channel interference? |
| Spreadtrum | Support. |
| KDDI | Support the proposal. |
| Huawei, HiSilicon | Support |
| Sony | Support. |
| vivo | Support |
| Ericsson | Support |
| QC | Fine with the proposal for sake of progress |
| Intel | Support |
| InterDigital | Support |
| NTT DOCOMO | We are fine with the proposal. |
| Samsung | We are ok to evaluate two SBFD subband configuration patterns. |
| CATT | We are open to consider both confiugrations but we do not think both configurations need to be considered in every simulation. |

#### Updated proposal 4-2-3d:

For performance evaluation and comparison between baseline legacy TDD operation and SBFD operation under SBFD Deployment Case 1 (Non-coexistence case with single SBFD subband configuration), consider the following alternatives:

- Optional: Alt 1 (No SBFD DL subband in the slots/symbols that correspond to UL slots/symbols in legacy TDD):

  - Legacy TDD: Static TDD UL/DL configuration with {DDDSU}, where S=[12D:2G:0U]

  - SBFD: Frame structure#1 (DXXXU), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about [20%] of the channel bandwidth.

- Baseline: Alt 2 (No SBFD DL subband in the slots/symbols that correspond to UL slots/symbols in legacy TDD):

  - Legacy TDD: Static TDD UL/DL configuration with {DDDSU}, where S=[12D:2G:0U]

  - SBFD: Frame structure#2 (XXXXU), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about [20%] of the channel bandwidth.

- Optional: Alt 3 (strive for the same UL/DL resource ratio between Legacy TDD and SBFD):

  - Legacy TDD: Static TDD UL/DL configuration with {DDSUU}, where S=[12D:2G:0U]

  - SBFD: Frame structure#2 (XXXXU), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about [20%] of the channel bandwidth.

- Baseline: Alt 4 (strive for the same UL/DL resource ratio between Legacy TDD and SBFD):

  - Legacy TDD: Static TDD UL/DL configuration with {DDDSU}, where S=[12D:2G:0U]

  - SBFD: Frame structure#3 (XXXXX), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about [20%] of the channel bandwidth.

- FFS: whether dynamic TDD can optionally be used for legacy TDD for comparison.

| Company | Comment |
| --- | --- |
| Moderator | Some companies (ZTE/Huawei/Intel/vivo/QC) prefer to support Alt3 and/or Alt4 as baseline for fair comparison, but that will result that SBFD DL subband are present in UL slots. However, some companies (Samsung/Xiaomi/Spreadtrum) do not support SBFD DL subband in UL slots. Currently, I picked Alt2 and Alt4 as baseline. To me, both Alt 1 and Alt 2 are the cases that no SBFD DL subband is present in the slots/symbols corresponding to UL of legacy TDD, and both Alt3 and Alt4 strive for the same UL/DL resource ratio between Legacy TDD and SBFD. I put the format of S slot in [] (using [12D:2G:0U] is trying to avoid SBFD DL subband in the symbols that corresponds to the UL symbols in ‘S’ slot of legacy TDD, if other format for S slot is used, we can further discuss whether/how to avoid SBFD DL subband in the symbols corresponding to the UL symbols in ‘S’ slot of legacy TDD) and also put the percentage of SBFD UL subband in []. These details can be further discussed and refined. |
| MediaTek | We support having Alt-2 or/and Alt-1 as baseline. |
| Spreadtrum | We agree with MediaTek to have Alt-2 or/and Alt-1 as baseline. |
| Huawei, HiSilicon | Support |
| Sony | Support. It will be good to put the list the baseline scenarios first and the optionals scenaiors second. |
| vivo | We support the proposal |
| Ericsson | We support the proposal. |
| QC | Support |
| Intel | In the FL’s summary above, it seems Intel has been listed amongst those preferring Alt 3 or 4 as baseline. This seems to be in error. In our view, all these options can be simply listed without identifying any baseline/optional cases. The need to consider same DL:UL ratio for legacy TDD and SBFD exists mainly for evaluations in which only UL performance/coverage may be evaluated while guaranteeing a level of DL performance (at least from resource dimensioning perspective). Note that even in this case, DL performance should be evaluated due to different impact from (cross-link) interference considerations. However, for SLS evaluations that would include DL/UL UPT as performance metrics, DL:UL ratio does not need to be maintained the same across legacy TDD and SBFD configurations. This can be part of the SBFD configuration and scheduling/resource allocation decision. As long as the same offered traffic load is considered for both legacy TDD and SBFD scheme, the trade-off between DL and UL should manifest automatically, depending on the exact frame structure, scheduling and link adaptation strategies, and SBFD operation. On the other hand, we also do not see an issue regarding the statement “…. but that will result that SBFD DL subband are present in UL slots” since the “UL slot” is in legacy TDD configuration, not in the SBFD configuration. |

## 5.3 Issue #4-3: Antenna configuration

### 5.3.1 Submitted proposal

| Company | Proposals |
| --- | --- |
| CMCC | Proposal 15: The separate-Tx/Rx antenna architecture can be described with the following parameters: - Legacy parameters: M,N,P,Mg,Ng, Mp,Np,P,Mg,Ng, dH,dV, dg,H,dg,V - New parameters: da,H,da,V wherein, there are one Tx antenna array and one Rx antenna array spaced in the horizontal direction with a spacing of da,H and in the vertical direction with a spacing of da,V. Proposal 16: For separate-Tx/Rx antenna architecture, switched connection between Tx/Rx RFFE and antenna array is preferred, as illustrated in Figure 7 I, i.e., - In DL slot, the Tx RFFE connects to the 1st half of antenna array. - In SBFD slot, the Tx RFFE connects to the 1st half of antenna array, and the Rx RFFE connects to the 2nd part of antenna array. - In UL only slot, the Rx RFFE connects to the 1st half of antenna array. Proposal 17: For evaluation and comparison between SBFD and legacy TDD, separate-Tx/Rx antenna architecture is assumed for SBFD, and the total number of antenna elements of separate-Tx/Rx antenna assumed for SBFD is two times of the total number of antenna elements of shared-Tx/Rx antenna assumed for legacy TDD. Proposal 18: Use the shared-Tx/Rx antenna configurations in Table 10 for evaluation of legacy TDD operation. Table 10 The shared-Tx/Rx antenna configuration for legacy TDD. Scenarios; Antenna configuration at TRxP. Scenario A1 in FR1 (4 GHz); (M,N,P,Mg,Ng;Mp,Np) = (2,1,2,1,1;2,1), (dH,dV) = (0.5, 0.5)λ +45°, -45° polarization. Scenario A1 in FR2 (30 GHz); (M,N,P,Mg,Ng;Mp,Np) = (4,8,2,1,1;2,2), (dH,dV) = (0.5, 0.5)λ +45°, -45° polarization. Scenario A2 in FR1 (4 GHz); (M,N,P,Mg,Ng;Mp,Np) = (8,8,2,1,1;2,8), (dH,dV) = (0.5, 0.8)λ +45°, -45° polarization. Scenario A2 in FR2 (30 GHz); (M,N,P,Mg,Ng;Mp,Np) = (4,8,2,2,2;2,8), (dH,dV) = (0.5, 0.5)λ. (dg,H,dg,V) = (4.0, 2.0)λ +45°, -45° polarization. Scenario A3/C3/D3 in FR1 (4 GHz); (M,N,P,Mg,Ng;Mp,Np) = (12,8,2,1,1;4,8), (dH,dV) = (0.5, 0.8)λ +45°, -45° polarization. Proposal 19: Use the separate-Tx/Rx antenna configurations in Table 11 for evaluation of SBFD. Table 11 The separate-Tx/Rx antenna configuration for SBFD for Phase-1 calibration. Number of antenna elements per TRxP. Scenario A1 in FR1 (4 GHz); (da,H, da,V) = (N/A, [5]) λ (Note: λ corresponds to 7.5cm) For each Tx/Rx antenna: (M,N,P,Mg,Ng;Mp,Np) = (2,1,2,1,1;2,1), (dH,dV) = (0.5, 0.5)λ +45°, -45° polarization. Scenario A1 in FR2 (30 GHz); (da,H, da,V) = (N/A, [32]) λ (Note: λ corresponds to 1cm) For each Tx/Rx antenna: (M,N,P,Mg,Ng;Mp,Np) = (4,8,2,1,1;2,2), (dH,dV) = (0.5, 0.5)λ +45°, -45° polarization. Scenario A2 in FR1 (4 GHz); (da,H, da,V) = (N/A, [10.4]) λ (Note: λ corresponds to 7.5cm) For each Tx/Rx antenna: (M,N,P,Mg,Ng;Mp,Np) = (8,8,2,1,1;2,8), (dH,dV) = (0.5, 0.8)λ +45°, -45° polarization. Scenario A2 in FR2 (30 GHz); (da,H, da,V) = (N/A, [34]) λ (Note: λ corresponds to 1cm) For each Tx/Rx antenna: (M,N,P,Mg,Ng;Mp,Np) = (4,8,2,2,2;2,8), (dH,dV) = (0.5, 0.5)λ. (dg,H,dg,V) = (4.0, 2.0)λ +45°, -45° polarization. Scenario A3/C3/D3 in FR1 (4 GHz); (da,H, da,V) = (N/A, [10.4]) λ (Note: λ corresponds to 7.5cm) For each Tx/Rx antenna: (M,N,P,Mg,Ng;Mp,Np) = (12,8,2,1,1;4,8), (dH,dV) = (0.5, 0.8)λ +45°, -45° polarization. |
| Samsung | Proposal 9: For evaluation of SBFD operation, RAN1 takes the following two cases for BS antenna configuration. - Case 1. Half number of antenna elements are used to simultaneously perform DL transmission and UL reception - Case 2. The same number of antenna elements are used to simultaneously perform DL transmission and UL reception Proposal 10: For evaluation, RAN1 takes the parameters for BS antenna configurations shown in Table 6 as a starting point. |
| Ericsson | Proposal 10: For system level evaluations, when comparing SBFD performance to static TDD, two cases should be considered: SBFD network with 1) same antenna array size (with halved transmit power) and 2) same antenna array gain (with same transmit power) as static TDD network. |
| KT | Observation 1: Using separate panel for Tx/Rx can help reducing Self-Interference Proposal 2: Consider using separate Tx/Rx panel for the default design of sub-band full duplex gNB |
| LG | Proposal 10: Discuss antenna configuration for half duplex and subband non-overlapping full duplex. |
| Qualcomm | Observation 5: Support separate panels configuration with split of the antenna elements for simultaneous downlink transmission and uplink reception for subband full duplex. Companies can report assumed physical panels separation. Observation 6: Support single panel configuration with same total number of antenna elements for downlink transmission or uplink reception for legacy TDD. Proposal 9: For subband full duplex deployment scenario, BS antenna configuration is based on two panels with split of the antenna elements for simultaneous downlink transmission and uplink reception. Proposal 10: For legacy TDD deployment scenario, BS antenna configuration is based on single panel for downlink transmission or uplink reception. Observation 7: To further optimize the performance of subband full duplex when traffic is single direction in SBFD slot, TDD-like single panel configuration could be used to improve the beamforming gain. Proposal 11: At least for FR2, optionally support adaptive antenna array configuration across slots for the subband full duplex evaluation. According to traffic conditions, separate panels configuration shall be used on the subband full duplex slots with scheduled simultaneous downlink and uplink, and TDD-like single panel configuration shall be used on the dynamic TDD slots with scheduled either downlink or uplink. |

### 5.3.2 Summary

To reduce self-interference, several companies [CMCC, Samsung, Ercisson, KT, Qualcomm] propose to use separate-Tx/Rx antenna array architecture for SBFD. CMCC suggests to use the following parameters to describe the separate-Tx/Rx antenna array model:

- Legacy parameters: M,N,P,Mg,Ng, Mp,Np,P,Mg,Ng, dH,dV, dg,H,dg,V

- New parameters: da,H,da,V

- Note: there are one Tx antenna array and one Rx antenna array spaced in the horizontal direction with a spacing of da,H and in the vertical direction with a spacing of da,V.

Moderator suggests Initial proposal 4-3-1 and 4-3-2.

Separate-Tx/Rx antenna array model.

For evaluation and comparison between SBFD and legacy TDD, when separate-Tx/Rx antenna array is assumed for SBFD, the following options are proposed by companies [Samsung, Ericsson, Qualcomm, CMCC]:

- Opt 1: The total number of antenna elements of separate-Tx/Rx antenna array for SBFD is the same as the total number of antenna elements of shared-Tx/Rx antenna array for legacy TDD.

- Opt 2: The total number of antenna elements of separate-Tx/Rx antenna array for SBFD is two times of the total number of antenna elements of shared-Tx/Rx antenna array for legacy TDD.

CMCC raises that for the shared-Tx/Rx antenna architecture in legacy TDD (as shown in part(a) of the following figure), all the antenna elements can be either connected to Tx RFFE (RF front end) or to Rx RFFE by switching. For SBFD with separate-Tx/Rx antenna architecture, two options of the connection between Tx/Rx RFFE and the antenna array can be considered:

- Candidate 1: Fixed connection. As shown in part(b) of the following figure, Tx RFFE (including DAC and PA) always connected to the Tx antenna array, and Rx RFFE (including LNA and ADC) always connected to the Rx antenna array.

  - Pros: Simple.

  - Cons: Loss of UL/DL channel reciprocity, since it is hard to perform accurate estimation of DL channel associated with TX antenna array via the SRS reception associated with a different Rx antenna array.

- Candidate 2: Switched connection. As shown in partI of the following figure, 1) In DL slot, the Tx RFFE connects to the 1st half of antenna array. 2) In SBFD slot, the Tx RFFE connects to the 1st half of antenna array, and the Rx RFFE connects to the 2nd part of antenna array. 3) In UL only slot, the Rx RFFE connects to the 1st half of antenna array.

  - Pros:

    - UL/DL channel reciprocity can be maintained. Note that, the Tx RFFE and the Rx RFFE connect to the same part of antenna array (i.e., the left half of antenna array) in TDM mode, thus, the SRS reception at the Rx RFFE in UL only slot can be used to accurately estimate the DL channel in DL slot or SBFD slots.

- shared-Tx/Rx antenna array architecture

(b) Candidate 1 of separate-Tx/Rx antenna array architecture: fixed connection

I Candidate 2 of separate-Tx/Rx antenna array architecture: switched connection

Based on the above discussion, CMCC thinks candidate 2 of separate-Tx/Rx antenna array architecture is preferred, and further made the following observations based on the candidate 2 of separate-Tx/Rx antenna array architecture:

- For Opt 1 (with the same number of antenna elements as legacy TDD), the number of TXRUs of separate-Tx/Rx antenna array for SBFD will be half of the number of TXRUs of shared-Tx/Rx antenna array for legacy TDD.

- For Opt 2 (with two times of antenna elements as legacy TDD), the number of TXRUs of separate-Tx/Rx antenna array for SBFD will be the same as the number of TXRUs of shared-Tx/Rx antenna array for legacy TDD.

Qualcomm proposes to optionally support adaptive antenna array configuration across slots for the subband full duplex evaluation at least for FR2, i.e., according to traffic conditions, separate panels configuration shall be used on the subband full duplex slots with scheduled simultaneous downlink and uplink, and TDD-like single panel configuration shall be used on the dynamic TDD slots with scheduled either downlink or uplink. Moderator understands this kind of adaptive antenna array configuration is based on the assumption that the number of antenna elements of the separate-Tx/Rx antenna array for SBFD is the same as the number of antenna elements of shared-Tx/Rx antenna array for legacy TDD, and the number of TXRUs of separate-Tx/Rx antenna for SBFD is also the same as the number of TXRUs of shared-Tx/Rx antenna array for legacy TDD.

Moderator suggests Initial proposal 4-3-3.

### 5.3.3 1st Round Proposals (closed)

#### Initial proposal 4-3-1:

For SBFD operation, BS antenna configuration is based on two separate antenna arrays with split of the antenna elements for simultaneous downlink transmission and uplink reception. Use the following parameters to describe the separate-Tx/Rx antenna array model.

- Legacy parameters M,N,P,Mg,Ng, Mp,Np,P,Mg,Ng, dH,dV, dg,H,dg,V, as defined in section 7.3 in TR.38.901, are used to describe the Tx antenna array or Rx antenna array.

- New parameters da,H,da,V are used to describe the relative location between Tx antenna array and Rx antenna array.

  - For one separate-Tx/Rx antenna array, there are one Tx antenna array and one Rx antenna array spaced in the horizontal direction with a spacing of da,H and in the vertical direction with a spacing of da,V.

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| QC | We support main bullet and first sub-bullet. Companies can report the configured panel separation if needed. |
| NTT DOCOMO | If we use the separate antenna configuration for the study, it may be a limitation for the SBFD deployment. On the other hand, it is the only solution to realize the SBFD we are fine to use the configuration. |
| CATT | We think spatial isolation is one important aspect for self-interference mitigation and the feasibility and ability needs to be discussed in RAN4. The second bullet can be discussed after RAN4 provides some input. |
| Spreadtrum | Support |
| ZTE | We feel that before receiving the response from RAN4, it is too premature to exclude the shared panel case for now. we suggest to include both of them. |
| Vivo | We think Legacy parameters as defined in TR.38.901 can be used. Some options or examples can be provided for better alignment and comparison among evaluation results from different companies. |
| Sharp | We support the proposal. |
| Xiaomi | Support. |
| Huawei, HiSilicon | Fine with legacy configuration. |
| Ericsson | We support the proposal in principle. |
| Nokia/NSB | Support in principle. Similar view with QC, companies can report the configuration used. |
| Intel | We share same view as QC, and support main bullet and first sub-bullet, while as for the panel separation, this can be reported separately from companies, if applied. |
| InterDigital | Share same view as Qualcomm. |
| Samsung | Unclear to define relative location of TX antenna array and RX antenna array. For SLS, it is sufficient to define the legacy antenna parameter for reception and for transmission. If the relative location is defined, which points are needed to be further implemented in SLS? |
| KDDI | We share the same view with DOCOMO. We should study first whether BS antenna configuration with two separate antenna arrays is needed. |
| Moderator | Updated based on comments |

#### Initial proposal 4-3-2:

For legacy TDD operation, BS antenna configuration is based on single antenna array for downlink transmission or uplink reception, which can be called as shared-Tx/Rx antenna array.

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| QC | Support |
| NTT DOCOMO | We support. |
| New H3C | Support |
| CATT | Support |
| Spreadtrum | Support |
| ZTE | Support |
| vivo | Ok with the definition |
| Sharp | We support the proposal. |
| Sony | Support |

| Xiaomi | Support. |
| --- | --- |
| Huawei, HiSilicon | Support |
| Ericsson | We support the proposal in principle. |
| Nokia/NSB | Support |
| Intel | We support this proposal. |
| InterDigital | Support |
| Samsung | Agree |
| KDDI | Support. |
| Moderator | The proposal is stable. I moved it to section 6. |

#### Initial proposal 4-3-3:

For evaluation and comparison between SBFD and legacy TDD, when separate-Tx/Rx antenna array is assumed for SBFD, consider the following options:

- Opt 1: The total number of antenna elements of separate-Tx/Rx antenna array for SBFD is the same as the total number of antenna elements of shared-Tx/Rx antenna array for legacy TDD.

- Opt 2: The total number of antenna elements of separate-Tx/Rx antenna array for SBFD is two times of the total number of antenna elements of shared-Tx/Rx antenna array for legacy TDD.

- FFS: the assumption on the relationship between the number of TXRUs of separate-Tx/Rx antenna array for SBFD and the number of TXRUs of shared-Tx/Rx antenna array for legacy TDD.

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| QC | Prefer Opt 1 for fair comparison with the same #TxRUs for SBFD and legacy TDD. At least for FR2, support adaptive antenna array configuration across slots for additional SBFD evaluation. According to traffic conditions, separate panels configuration shall be used on the subband full duplex slots with scheduled simultaneous downlink and uplink, and TDD-like single panel configuration shall be used on the dynamic TDD slots with scheduled either D or U. |
| New H3C | Prefer Option 1 |
| CATT | Opt 1 should be adopted for fair comparison. |
| Spreadtrum | We prefer Opt2. For Opt1, same number of TxRU is assumed for fair comparison but which means number of antenna elements varies between legacy DL/UL slots and SBFD slots. For Opt2, more antenna elements are used for SBFD which make the comparison not fair, but antenna isolation must be considered for duplex in order to suppress SI and separated antenna will be adopted in duplex operation in future. |
| ZTE | We think both options should be considered for now. |
| vivo | We prefer option 2. Separate-Tx/Rx antenna array is necessary for SBFD to suppress the interference. To maintain the same antenna gain, increased number of antenna elements could be adopted for SBFD. Option 1 can be optionally evaluated. |
| Sharp | Both can be considered further. |
| Sony | Option 1 for the purpose of comparison with legacy. However, we would expect an SBFD gNB to have higher hardware sophistication and so Option 2 may also be reasonable. |
| MediaTek | Both options can be considered. |
| Xiaomi | Option 2. |
| Huawei, HiSilicon | For evaluation purpose, Opt 2 is preferred for simple SLS. For Opt 1, the number of TXRUs used in the SBFD slot will be the half of that used in the UL/DL-only slot. It is hard for SLS modelling. But for Opt 2, the number of TXRUs are the same in SBFD slot and UL/DL-only slot. |
| Ericsson | We support the proposal in principle. Both options could be considered for evaluations. |
| Nokia/NSB | Support |
| Intel | To allow a fair comparison between SBFD and legacy TDD, we see the appeal of Opt.1. However, we also acknowledge that Option 2 can help avoid additional complications in the SLS evaluations as antenna configuration changes between regular DL/UL slots and SBFD slots. Also, it may not be too unfair since upgrades to gNB hardware and antenna systems would be expected anyway for supporting FD operations. In light of this, perhaps Option 2 could be considered as baseline while Option 1 could be an optional configuration. |
| InterDigital | Both options can be considered. |
| Samsung | Both options should be evaluated to guide how many antenna elements are needed for SBFD operation. |
| LG | Prefer Option 2. |
| Moderator | Observation: 3 companies prefer option 1 and 5 companies prefer option 2, and other companies support both. |

### 5.3.4 2nd Round Proposals (closed)

#### Updated proposal 4-3-1:

For SBFD operation, BS antenna configuration is based on two separate antenna arrays with split of the antenna elements for simultaneous downlink transmission and uplink reception. Use the legacy parameters M,N,P,Mg,Ng, Mp,Np,P,Mg,Ng, dH,dV, dg,H,dg,V, as defined in section 7.3 in TR.38.901, to describe the separate-Tx/Rx antenna array model.

- In SBFD slots, assume the upper half antenna elements are used for transmission and the lower half antenna elements are used for reception when gNB performs simultaneous transmission and reception.

  - FFS: other adaptive transmission and reception methods in SBFD slots.

- In DL-only or UL-only slots, assume all the antenna elements are used for transmission or reception.

- Companies to report the separation of the Tx panel and Rx panel assumed in their simulation.

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| New H3C | We are fine with this proposal |
| Intel | We can support the proposal, except that we are not sure if the new text is necessary. - In SBFD slots, assume the upper half antenna elements are used for transmission and the lower half antenna elements are used for reception when gNB performs simultaneous transmission and reception. - FFS: other adaptive transmission and reception methods in SBFD slots. |
| Samsung | We are fine with the proposal. The definition of Mp and Np is not shown in section 7.3 in TR39.901. Please check the section number or TR number. |
| Vivo | We think main bullet and the last sub-bullet are sufficient. Another sub-bullet can be added for clarification, i.e., how to assume the antenna elements used for transmission or reception can be reported by companies |
| CATT | We would like to clarify whether the proposal is option 1 of the next proposal? |
| MediaTek | We have concern on the following assumption “In DL-only or UL-only slots, assume all the antenna elements are used for transmission or reception”. As some of the slots will use all the antenna elements, and other slots use half the antenna elements, switching gap need to be considered. As an example, take the following pattern DXXXU, where X slots are SBFD slots (where upper half antenna elements are used for transmission and the lower half antenna elements are used for reception). If in the D slot all the antenna elements are used for transmission, this mean a switching gap is needed between the D slot and the first X slot because the lower half of antenna elements will be switched from transmission to reception. |
| Sony | Support |
| Nokia, NSB | Similar view as vivo, we think each company should report how antenna elements are used for transmission or reception |
| Huawei, HiSilicon | We are fine with the proposal. |
| Spreadtrum | We also have the same question with CATT. |
| Ericsson | We are fine with the proposal. |
| QC | Support in principle. We have few comments: - Given the physical downtilt of the gNB array. To reduce the self-interference between the Tx and Rx panels, the transmission panel should be the bottom and reception panel is the top panel. - At least for FR2, we support adaptive antenna array configuration across SBFD slots for additional SBFD evaluation. According to traffic conditions, separate panels configuration shall be used on the subband full duplex slots with scheduled simultaneous downlink and uplink, and TDD-like single panel configuration shall be used on the dynamic TDD slots with scheduled either DL subband or UL subband. Suggest edits in red. - In DL-only, UL-only slots or SBFD slots with one direction traffic, assume all the antenna elements are used for transmission or reception. |
| Moderator | Update proposal 4-3-1a was provided based on companies’ comments. @CATT, this proposal is just to provide a description for the separate-Tx/Rx antenna array, it can be used for both options of proposal 4-3-3. |

#### Updated proposal 4-3-3:

For evaluation and comparison between SBFD and legacy TDD, when separate-Tx/Rx antenna array is assumed for SBFD, assume the total number of TxRUs of separate-Tx/Rx antenna array for SBFD is the same as the total number of TxRUs of shared-Tx/Rx antenna array for legacy TDD. Regarding antenna elements, consider the following options:

- Opt 1: The total number of antenna elements of separate-Tx/Rx antenna array for SBFD is the same as the total number of antenna elements of shared-Tx/Rx antenna array for legacy TDD.

- Opt 2: The total number of antenna elements of separate-Tx/Rx antenna array for SBFD is two times of the total number of antenna elements of shared-Tx/Rx antenna array for legacy TDD.

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| New H3C | In order to avoid decreasing performance related to coverage, we slightly prefer Opt.2. we are open to discuss about opt.1 |
| Intel | We are Ok with the proposal. |
| Samsung | Both options can be evaluated. In the proposal, it is unclear “separate TX/Rx antenna array” and “shared Tx/Rx antenna array”, which are futher described to avoid different understanding. |
| Vivo | We prefer option 2. For option 1, we wonder how to keep the total number of TxRUs of separate-Tx/Rx antenna array for SBFD is the same as the total number of TxRUs of shared-Tx/Rx antenna array for legacy TDD. |
| CATT | See our question above. |
| Sony | It will be good to have only 1 option to reduce the number of required simulations. We are fine either option 1 or 2 but not leaving both on the table as this will make it hard to compare results since some companies may simulate option 1 and others option 2 (not all companies have to capabilities/time to simulate both). |
| Nokia, NSB | We support the proposal. |
| Huawei, HiSilicon | Fine with the proposal. |
| Xiaomi | We are OK with the proposal. |
| Spreadtrum | Prefer Opt2 to avoid change of antenna element number between all DL/UL slot and SBFD slot. |
| Ericsson | We are ok with the proposal in principle. |
| QC | Support |
| Moderator | Update proposal 4-3-3a was provided based on companies’ comments. |

### 5.3.5 3rd Round Proposals (closed)

#### Updated proposal 4-3-1a:

For evaluation of SBFD operation, BS uses separate panels for simultaneous downlink transmission and uplink reception, we can call it separate-Tx/Rx antenna array for description of evaluation assumption.

- Companies can report the separation of the Tx panel and Rx panel assumed in their simulation.

- Companies can report how the antenna elements are used for transmission or reception in a slot if BS does not perform simultaneous downlink transmission and uplink reception.

| Company | Comment |
| --- | --- |
| New H3C | We are fine with this proposal |
| Huawei, HiSilicon | Fine with the proposal. |
| Sony | Support |
| MediaTek | Fine with this proposal |
| InterDigital | Fine with the FL proposal |
| Intel | We are Ok with the proposal. |
| Nokia, NSB | Support the proposal. |
| QC | Support. However, we think it is better to align on the number of antenna elements across companies to cross check the evaluation results. |
| Fujitsu | Support |
| DOCOMO | We are fine with the proposal. |
| LG | Support |
| vivo | We are fine with this proposal |
| CATT | We are fine with FL proposal. |
| Ericsson | We support the proposal. |
| Spreadtrum | We have the same view as QC. |
| Xiaomi | We are fine with the proposal. |
| Samsung | Fine with the proposal |

#### Updated proposal 4-3-2a:

For evaluation of legacy TDD operation, BS uses the same antenna array for downlink transmission and uplink reception, we can call it shared-Tx/Rx antenna array for description of evaluation assumption.

@Qualcomm, regarding your comment in the email thread, I avoid using ‘single panel’ in proposal 4-3-2, since multiple panels can be used for FR2for legacy TDD in the simulation.

| Company | Comment |
| --- | --- |
| New H3C | We are fine with this proposal |
| Huawei, HiSilicon | Fine with the proposal. |
| Sony | Support |
| MediaTek | Fine with this proposal |
| InterDigital | Fine with the FL proposal |
| Intel | We are OK with the proposal. |
| Nokia, NSB | Support the proposal. |
| QC | Support |
| Fujitsu | Support |
| DOCOMO | We are fine with the proposal. |
| LG | Support |
| vivo | We are fine with this proposal |
| CATT | We are fine with FL proposal. |
| Ericsson | Support. |
| Spreadtrum | Support. |
| Xiaomi | Fine with the FL proposal. |
| Samsung | Fine with the proposal |

#### Updated proposal 4-3-3a:

For evaluation and comparison between SBFD and legacy TDD, assume the total number of TxRUs of the antenna array for SBFD is the same as the total number of TxRUs of the antenna array for legacy TDD. Regarding antenna elements, both of the two options can be used.

- Opt 1: The total number of antenna elements of the antenna array for SBFD is the same as the total number of antenna elements of the antenna array for legacy TDD.

- Opt 2: The total number of antenna elements of the antenna array for SBFD is two times of the total number of antenna elements of the antenna array for legacy TDD.

- Companies report which option is assumed in their simulation.

| Company | Comment |
| --- | --- |
| New H3C | We are fine with this proposal |
| ZTE | OK |
| Huawei, HiSilicon | Fine with the proposal. |
| Sony | Support |
| MediaTek | Fine with this proposal |
| InterDigital | Fine with the FL proposal |
| Intel | We are OK with the proposal. |
| Nokia, NSB | Support the proposal. |
| QC | Support |
| Fujitsu | Support |
| DOCOMO | We are fine with the proposal. |
| LG | Support |
| vivo | We are fine with this proposal |
| CATT | We are fine with FL proposal. |
| Ericsson | We support the proposal. |
| Spreadtrum | Support |
| Xiaomi | Fine with the FL proposal |
| Samsung | Fine with the proposal |

## 5.4 Issue #4-4: Channel model

### 5.4.1 Submitted proposal

| Company | Proposals |
| --- | --- |
| CMCC | Proposal 10: For TRxP-TRxP channel model: - In scenario A1, reuse the channel model for indoor-Office (Open office) in TR 38.901 as the channel model for TRxP-TRxP by setting hUE=hgNB. - In scenarios A2, reuse the channel model for Umi-Street Canyon in TR 38.901 as the channel model for TRxP-TRxP by setting hUE=hgNB. - In scenarios A3/C3/D3, reuse the channel model for Uma in TR 38.901 as the channel model for TRxP-TRxP by setting hUE=hgNB. Proposal 11: For UE-UE channel mode, Opt 1 of the following options can be used: - Opt 1: Reuse the channel model for gNB-UE in TR 38.901 as the channel model for UE-UE by setting hgNB=hUE., e.g., - In scenario A1, reuse the channel model for indoor-Office (Open office) in TR 38.901 as the channel model for UE-UE by setting hgNB=hUE. - In scenario A2/A3/C3/D3, reuse the channel model for Umi-Street canyon in TR 38.901 as the channel model for UE-UE by setting hgNB=hUE. - Opt 2: Reuse the UE-UE channel model for flexible duplex evaluation in TR 38.802. Proposal 12: For UE-UE channel model, the applicability range of the pathloss model for Umi-Street canyon in Table 7.4.1-1 in TR 38.901 is extended from 10m≤d2D≤5km to X m≤d2D≤5km, e.g., X = [3]. Proposal 13: At least for calibration, only large-scale fading needs to be modelled for UE-UE channel. Proposal 14: In scenarios A2/A3/C3/D3, the penetration loss between UEs can follows Table A.2.1-13 and A.2.1-12 in TR 38.802 for FR1 and FR2 respectively. |
| Huawei | Observation 5: The LOS probability of BS-to-BS link determined by reusing BS-to-UE channel model in TR 38.901 with modifications in TR 38.802 is much lower than that in the realistic network. It will lead to that the BS-to-BS CLI strength level in the evaluation is much lower than that in the actual network. Proposal 9: Reuse the existing BS-to-UE channel model in TR 38.901 and TR 38.802 as much as possible to determine the BS-to-BS channel and UE-to-UE channel in Rel-18 NR duplex evolution. - FFS: the parameters, e.g., LOS probability, delay spread, angle spread, etc., in the BS-to-UE channel model should be checked whether they are applicable to the BS-to-BS channel model and UE-to-UE channel model. |
| ZTE | Proposal 11: For Subband full duplex and dynamic TDD simulation, - RAN1 considers channel model with both large scale (e.g., path loss and penetration loss) and small scale (e.g., fast fading) for co-channel gNB-gNB channel model. - RAN1 considers channel model with large scale (e.g., path loss and penetration loss) for adjacent channel gNB-gNB channel model. - RAN1 considers channel model with large scale (e.g., path loss and penetration loss) for UE-UE co-channel and adjacent channel model. Proposal 12: Regarding channel model for simulation of subband duplex and dynamic TDD - gNB-gNB channel model: reusing the 38.901 channel model by replacing the UE’s antenna height with victim gNB’s antenna height; - UE-UE channel model: reusing the 38.901 channel model by replacing the gNB’s antenna height with aggressor UE’s antenna height; - FFS LOS probability |
| vivo | Proposal 11: The channel model in TR38.901 with necessary modification can be used for BS-to-BS and UE-to-UE channel modeling. |
| Xiaomi | Proposal 7: Study and determine the channel model for different links. The following assumption can be the starting point: - gNB-to-gNB, reuse the channel model defined in TR38.901 - gNB-to-UE, reuse the channel model defined in TR38.901 - UE-to-UE, reuse the channel model defined in TR38.802 |
| Nokia | Proposal 10: For the channel models for dynamic TDD and SBFD system-level evaluations, re-use the existing channel models in TR 38.901 as much as possible for gNB-to-UE, gNB-to-gNB and UE-to-UE links. - See Table 2 in this contribution for considerations on channel model and UE and BS height for Urban Macro (Uma), Dense Urban and Indoor Hotspot (InH) scenarios. - RAN1 to discuss whether changes to the LOS probability calculations in Umi and InH channel models are needed to explicitly account for the transmitter-receiver height (in both absolute and relative terms). Table 2: Channel models for gNB-UE, gNB-gNB and UE-UE links. Uma (500 m ISD); Dense Urban (200 m ISD); Indoor Hotspot (20 m ISD). gNB-UE; TR 38.901 Uma; TR 38.901 Umi; TR 38.901 InH. gNB-gNB; TR 38.901 Uma with hUE = hBS = 25m; TR 38.901 Umi with hUE = hBS = 10m; TR 38.901 InH with hUE = hBS = 3m. UE-UE; TR 38.901 Umi with hUE = hBS; TR 38.901 Umi with hUE = hBS; TR 38.901 InH with hUE = hBS = 1.5m. HetNet (500 m ISD macro layer + outdoor small cells); HetNet (500 m ISD macro layer + indoor small cells). Macro gNB-UE; TR 38.901 Uma; TR 38.901 Uma. Small cell gNB-UE; TR 38.901 Umi; TR 38.901 InH. Macro-to-Macro; TR 38.901 Uma with hUE = hBS = 25m; TR 38.901 Uma with hUE = hBS = 25m. Macro-to-small cell; TR 38.901 Uma with hUE =10m and hBS = 25m; TR 38.901 Uma with hUE = 3m and hBS = 25m, and O2I penetration loss. Small cell-to-small cell; TR 38.901 Umi with hUE = hBS = 10m; TR 38.901 InH with hUE = hBS = 3m. UE-UE; TR 38.901 Umi with hUE = hBS; TR 38.901 InH if UEs are in the same office, otherwise Umi.. |
| Intel | Proposal 2: Channel models used in Rel-16 CLI/RIM study could be used as a starting point. |
| Qualcomm | Observation 14: gNB-to-gNB channel model by leveraging gNB-to-UE channel model in TR 38.802 / 38.901 with certain changes on UE to mimic a gNB. Observation 15: UE-to-UE inter-subband CLI model can be done using similar procedure for modelling inter-gNB inter-subband CLI. |

### 5.4.2 Summary

gNB-gNB and UE-UE channel model are needed in RAN1 SLS for SBFD and dynamic/flexible TDD evaluation. ZTE proposes that:

- RAN1 considers channel model with both large scale (e.g., path loss and penetration loss) and small scale (e.g., fast fading) for co-channel gNB-gNB channel model.

- RAN1 considers channel model with large scale (e.g., path loss and penetration loss) for adjacent channel gNB-gNB channel model.

- RAN1 considers channel model with large scale (e.g., path loss and penetration loss) for UE-UE co-channel and adjacent channel model.

Moderator suggests Initial proposal 4-4-1.

Vivo raises the following observation for gNB-gNB and UE-UE channel model:

In the simulation parameters given in TR 38.828, it is recommended to use the channel model in TR38.803 for BS-to UE and BS-to-BS, while for UE-to-UE the channel model in TR 36.828 is used. The channel model in TR 38.803 does not give a model for small-scale fading, which will lead to a decrease in the accuracy of the simulation results, especially for the BS-to-UE. In addition, the channel model in TR 36.828 is commonly used in LTE evaluation. In previous NR evaluations, the channel model in TR38.901 was widely used. Therefore, For SBFD evaluation, we recommend reusing the channel model in TR38.901 for BS-to-BS and UE-to-UE channel modeling. Some necessary modifications can be considered for the BS-to-BS and UE-to-UE channel modeling.

For gNB-gNB channel model, most companies suggest to reuse gNB-to-UE channel model in TR 38.901 with necessary modification, e.g.,

- Replacing the UE’s antenna height with gNB’s antenna height

  - CMCC, Huawei, ZTE, vivo, xiaomi, Nokia, Intel, Qualcomm

- FFS: whether/how to update LOS probability.

  - Huawei, ZTE, Nokia

Moderator suggests Initial proposal 4-4-2.

For UE-UE channel model, two options are proposed by companies:

- Option 1: Reuse the gNB-UE 5GCM in TR 38.901 with necessary modification for both FR1 and FR2, similar as the UE-UE channel model for flexible duplex evaluation in TR38.802 for FR2.

  - Replacing the gNB’s antenna height with UE’s antenna height

    - CMCC, ZTE, vivo, xiaomi, Nokia, [Qualcomm]

  - Extending the applicability range of the pathloss model for Umi-Street canyon in Table 7.4.1-1 in TR 38.901 is extended from 10m≤d2D≤5km to X m≤d2D≤5km, e.g., X = [3]

    - CMCC

- Option 2: Reuse the UE-UE channel model for flexible duplex evaluation in TR 38.802 for both FR1 and FR2.

  - Huawei, Intel

Moderator suggests Initial proposal 4-4-3.

CMCC suggested that the penetration loss between UEs can follows Table A.2.1-13 and A.2.1-12 in TR 38.802 for FR1 and FR2 respectively.

### 5.4.3 1st Round Proposals (closed)

#### Initial proposal 4-4-1:

For gNB-gNB channel model and UE-UE channel model in RAN1 SLS:

- Both large scale fading (e.g., path loss, penetration loss, shadowing) and small scale fading (e.g., fast fading including antenna gain) are modeled for co-channel gNB-gNB channel model.

- Only large scale fading (e.g., path loss, penetration loss, shadowing, antenna gain) is modeled for adjacent-channel gNB-gNB channel model.

- Only large scale fading (e.g., path loss, penetration loss, shadowing, antenna gain) is modeled for co-channel and adjacent-channel UE-UE channel model.

- Note: Antenna gain is calculated based on the gNB-gNB or UE-UE LOS direction instead on the multi-path directions if fast fading is not modeled.

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| QC | This proposal is related to discussion of 3-4. - Both large scales fading and small scale fading should be considered for UE-UE co-channel - The co-channel model should be based on TR 38.802 and TR 38.901 with some adjustments on height and angular spread. - Large scale fading considered for adjacent channel UE-UE channel model and adjacent channel BS-BS channel model. |
| New H3C | Support |
| Spreadtrum | The channels of gNB-gNB/UE-UE are used for interference analysis, where large scale factors are dominant. Thus, we recommend to consider only large scale fadings for channel modelling of co-channel/adjacent-channel gNB-gNB/UE-UE. |
| ZTE | We support this proposal in principle. |
| Vivo | Depending on the CLI handling scheme, the small scale fading for co-channel BS-BS link is not always necessary. We suggest to allow company to decide whether to model small fading for co-channel BS-BS link, i.e. not mandated. |
| Sharp | We are open to discuss. Large-scale parameters may be enough for gNB-gNB channel model since gNB can be expected not to move. |
| Sony | Support |
| Xiaomi | Agree. |
| Huawei, HiSilicon | Both large scale fading and small scale fading should be modelled for co-channel channel gNB-gNB channel and UE-UE channel. The small scale fading should be considered because the potential techniques to solve the CLI will use the characteristic of small scale fading, such as advanced receiver and beamforming. If only large scale fading is considered here, the CLI is equal to white noise and losses the direction information. And therefore, the potential techniques cannot be evaluated. |
| Ericsson | We support the proposal in principle. In addition, realistic assumptions i.e., LOS must be assumed between gNBs and also for UEs that are close. |
| Nokia/NSB | This proposal does not seem aligned to proposal 3-4 (see our input there). We think they should be jointly discussed |
| Intel | We mostly agree with the proposal, with the exception that both large and small scale fading should be modelled for gNB-gNB and UE-UE co-channel cases. |
| Samsung | Small scale fading for co-channel UE-UE interference is needed. |
| Moderator | Updated based on comments |

#### Initial proposal 4-4-2:

For gNB-gNB channel model, reuse gNB-to-UE channel model in TR 38.901 with necessary modification, e.g.,

- Replacing the UE’s antenna height with gNB’s antenna height.

- FFS: whether/how to update LOS probability.

- FFS: Other details and necessary modifications

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| QC | Support in principle. In addition to antenna height adjustment, angular spread should be updated as well. |
| New H3C | Support |
| CATT | Support |
| Spreadtrum | Support |
| ZTE | Similar view as Qualcomm. |
| Vivo | agree |
| Sharp | We support the proposal. |
| Sony | Support |
| Xiaomi | Support. |
| Huawei, HiSilicon | Fine with the proposal. And the LOS probability can be 0.8 at least. For HetNet scenario,the path loss model of Macro cell (Aggressor)->indoor small cell (Victim) should use Uma+penetration loss. |
| Nokia/NSB | Support. |
| Intel | We support the proposal, and second the comment from QC regarding the angular spread. |
| Samsung | Support. LOS probability and angular spread can be discussed. |
| Moderator | Updated based on comments. |

#### Initial proposal 4-4-3:

For UE-UE channel model, consider the following two options:

- Option 1: Reuse the gNB-UE 5GCM in TR 38.901 with necessary modifications for both FR1 and FR2, similar as the UE-UE channel model for flexible duplex evaluation in TR38.802 for FR2.

  - For Indoor hotspot, reuse the gNB-UE 5GCM Indoor-office in TR38.901, and for Dense urban and Urban macro, reuse the gNB-UE 5GCM Umi-Street canyon in TR38.901 with necessary modification, e.g.,

    - Replacing the gNB’s antenna height with UE’s antenna height, updating ASD and ZSD.

    - FFS: Other details and necessary modifications.

- Option 2: Reuse the UE-UE channel model for flexible duplex evaluation in TR 38.802 for both FR1 and FR2 with necessary modifications.

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| New H3C | Prefer Option 2 |
| CATT | We prefer Option 1. |
| Spreadtrum | Support option-1. |
| ZTE | We support Option1. The channel mode for FR1 in 38.802 is referred to LTE 3D channel, which is not appropriate for subband full duplex. |
| Vivo | Option 1 is preferred. |
| Xiaomi | Prefer option 2. |
| Huawei, HiSilicon | Prefer Option 1. |
| Ericsson | It is simpler to reuse the existing models. However, one needs to consider LOS between UEs that are close to each other. This can be added in the FFS for other details. |
| Nokia/NSB | Support |
| Intel | We prefer Option 1. |
| Samsung | Prefer option 2. |
| LG | Support the proposal and prefer Option 2. |
| Moderator | Observation: 6 companies support option 1 and 4 companies support option 2. Let’s try if the companies are OK to select option 1 for simplicity. |

### 5.4.4 2nd Round Proposals (closed)

#### Updated proposal 4-4-1:

For gNB-gNB channel model and UE-UE channel model in RAN1 SLS:

- Companies to report whether large scale fading (e.g., path loss, penetration loss, shadowing) and small scale fading (e.g., fast fading including antenna gain) are modeled for co-channel gNB-gNB channel model and co-channel UE-UE channel model in their simulations.

- Only large scale fading (e.g., path loss, penetration loss, shadowing, antenna gain) is modeled for adjacent-channel gNB-gNB channel model and adjacent-channel UE-UE channel model.

- Note: Antenna gain is calculated based on the gNB-gNB or UE-UE LOS direction instead on the multi-path directions if fast fading is not modeled.

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| New H3C | We are fine with this proposal |
| Intel | Given that likely we may need to calibrate our SLSs, and channel model may play an important role in aligning/comparing the results across companies, it may be preferrable for co-channel UE-UE and gNB-gNB to converge in one option rather than leave up to companies to choose. Our preference is to consider both large and small scale fading in this case. |
| Samsung | We prefer to define large scale facing and small scale fading models for co-channel gNB-gNB channel model and co-channel UE-UE channel. If it is up to each company on whether to implement large scale fading or not, we can have diverse evaluation results, which make hard to draw a conclusion. So, large scale fading model for co-channel gNB-gNB, co-channel UE-UE should be defined. It is ok to leave small scale fading model to each company’s implementation. |
| Vivo | We are fine with the proposal |
| CATT | We think large scale fading should be modelled while small scale fading can be up to companies to report for co-channel gNB-gNB and UE-UE channel model. |
| Sony | Support |
| Nokia, NSB | We are ok with the proposal, although we are also open to agree on having fast-fading model included for both gNB-gNB and UE-UE links which would be helpful for results comparability. |
| Huawei, HiSilicon | Fine with the proposal. |
| Spreadtrum | We have similar view with Intel to not leave up the choice to companies because of the calibration between companies. Refer to CLI calculation, we think large scale fading cover most of the part and small scale fading is not necessary. |
| Ericsson | Our comment regarding using realistic LOS assumptions for both gNB-gNB and UE-UE channel model is not captured in the updated proposal. |
| QC | Support. However, It is preferred to have common assumptions of large/small scale channel model across all companies for BS-BS and UE-UE. |
| Moderator | Updated proposal 4-4-1a was provided based on companies’ comments |

#### Updated proposal 4-4-2:

For gNB-gNB channel model, reuse gNB-to-UE channel model in TR 38.901 with necessary modification, e.g.,

- Replacing the UE’s antenna height with gNB’s antenna height, updating the angular spread

- FFS: whether/how to update LOS probability.

- FFS: Other details and necessary modifications

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| New H3C | We are fine with this proposal |
| Intel | We are Ok with the updated proposal. |
| Samsung | Support |
| vivo | We are fine with the proposal |
| CATT | Fine with the proposal. |
| Sony | Support |
| Nokia, NSB | We support the proposal |
| Huawei, HiSilicon | Fine with the proposal. We suggest LOS probability to be 0.8 at least. For HetNet, the path loss model of Macro cell (Aggressor)->indoor small cell (Victim) should use Uma+penetration loss. |
| Xiaomi | Support. |
| Spreadtrum | Support |
| Ericsson | Same comment as above. We propose the following modifications : #### Updated proposal 4-4-2: For gNB-gNB channel model, reuse gNB-to-UE channel model in TR 38.901 with necessary modification, e.g., - Replacing the UE’s antenna height with gNB’s antenna height, updating the angular spread - FFS: whether/how to Realistic update of LOS probability. - FFS: Other details and necessary modifications |
| QC | Support |
| Moderator | Updated proposal 4-4-2a was provided based on companies’ comments |

#### Updated proposal 4-4-3:

For UE-UE channel model, select option 1:

- Option 1: Reuse the gNB-UE 5GCM in TR 38.901 with necessary modifications for both FR1 and FR2, similar as the UE-UE channel model for flexible duplex evaluation in TR38.802 for FR2.

  - For Indoor hotspot, reuse the gNB-UE 5GCM Indoor-office in TR38.901, and for Dense urban and Urban macro, reuse the gNB-UE 5GCM Umi-Street canyon in TR38.901 with necessary modification, e.g.,

    - Replacing the gNB’s antenna height with UE’s antenna height, updating ASD and ZSD.

    - FFS: Other details and necessary modifications.

- Option 2: Reuse the UE-UE channel model for flexible duplex evaluation in TR 38.802 for both FR1 and FR2 with necessary modifications.

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| New H3C | We prefer option 2. We can open to discuss about option .1 |
| Intel | We are OK with the FL’s recommendation. |
| Samsung | We prefer option 2. In option 1, we need to resolve FFS point which requires additional workload. |
| Vivo | We prefer option 1. |
| CATT | Support |
| Nokia, NSB | We support the proposal |
| Huawei, HiSilicon | Fine |
| Xiaomi | We prefer option 2. Agree with Samsung that option 1 is not a completed solution. The UE-UE channel model in TR38.802 is already be justified and used for NR simulation. It should be sufficient for duplex simulation. |
| Spreadtrum | We prefer Option1 to align with gNB-gNB channel from TR38.901 which is the 5G channel smodel. |
| Ericsson | We support the proposal and prefer Option 1. |
| QC | Support proposal and prefer option 1. |
| Moderator | No update. Most companies are OK with option 1. Hope we can use a unified model. The following table is from TR38.802 with an example for option 1 added. For option 1, I think the modification for FR1 is very similar as that for FR2, which may not introduce too much additional discussion compared with option 2. Please note the UE clustering or Los probability will be separately discussed regardless of option 1 and option 2. UE-UE channel model for flexible duplex evaluation in TR 38.802. Dense urban/Urban macro; Indoor hotspot. Below 6GHz; Above 6GHz; Below 6GHz; Above 6GHz. Large-scale channel parameters; Option 2: A.2.1.2 in TR36.843 Option 1: Umi-Street canyon (h_BS=1.5m ~ 22.5m) penetration loss between UEs follows Table A.2.1-13 in TR38.802; Umi-Street canyon (h_BS=1.5m ~ 22.5m), penetration loss between UEs follows Table A.2.1-12 in TR38.802; Option 2: A.2.1.2 in TR36.843 Option 1: 5GCM Indoor-office (h_BS=1.5m); 5GCM Indoor-office (h_BS=1.5m). Fast fading parameters; Option 2: InH for indoor to indoor, and 3D Umi for other cases. Option 1: Umi-Street canyon. ASD and ZSD statistics updated to be the same as ASA and ZSA.; Umi-Street canyon; ASD and ZSD statistics updated to be the same as ASA and ZSA.; Option 2: A.2.1.2 in TR36.843 (ITU InH), ASD statistics updated to be the same as ASA. Option 1: 5GCM Indoor-office (h_BS=1.5m), ASD and ZSD statistics updated to be the same as ASA and ZSA; 5GCM Indoor-office (h_BS=1.5m), ASD and ZSD statistics updated to be the same as ASA and ZSA. |

### 5.4.5 3rd Round Proposals (closed)

#### Updated proposal 4-4-1a:

For gNB-gNB channel model and UE-UE channel model in RAN1 SLS,

- Large scale fading (e.g., path loss, penetration loss, shadowing) should be modelled for co-channel gNB-gNB channel model and co-channel UE-UE channel model, and companies report whether small scale fading (e.g., fast fading including antenna gain) is also modeled in their simulation.

- Only large scale fading (e.g., path loss, penetration loss, shadowing, antenna gain) is modeled for adjacent-channel gNB-gNB channel model and adjacent-channel UE-UE channel model.

- Note: Antenna gain is calculated based on the gNB-gNB or UE-UE LOS direction instead on the multi-path directions if fast fading is not modeled.

- FFS: how to model realistic LOS probability for gNB-gNB and UE-UE channel model.

| Company | Comment |
| --- | --- |
| New H3C | We are fine with this proposal |
| ZTE | OK |
| Huawei, HiSilicon | For the second subbullet, it can be similar to the first subbulet - Only lLarge scale fading (e.g., path loss, penetration loss, shadowing, antenna gain) is modeled for adjacent-channel gNB-gNB channel model and adjacent-channel UE-UE channel model, and companies report whether small scale fading (e.g., fast fading including antenna gain) is also modeled in their simulation. |
| Sony | Support. |
| Intel | We still have same concerns as those we previously raised. Given that likely we may need to calibrate our SLSs, and channel model may play an important role in aligning/comparing the results across companies, it may be preferrable for co-channel UE-UE and gNB-gNB to converge in one option rather than leave up to companies to choose. Our preference is to consider both large and small scale fading in this case. |
| Nokia, NSB | Support the proposal. |
| QC | Support |
| Fujitsu | Support |
| LG | We are fine with the proposal with Huawei’s modification. |
| Vivo | We are fine with this proposal |
| CATT | We are fine with FL proposal. |
| Ericsson | We support the proposal in principle. |
| Spreadtrum | We have the same concerns as Intel. It is better to do the calibration based on aligned channel model. Or only large scale fading will be considered in calibration. |
| Xiaomi | Support. |
| Samsung | Fine with the proposal |

#### Updated proposal 4-4-2a:

For gNB-gNB channel model, reuse gNB-to-UE channel model in TR 38.901 with necessary modification, e.g.,

- Replacing the UE’s antenna height with gNB’s antenna height, updating the angular spread

- Realistic update LOS probability.

- FFS: Other details and necessary modifications

| Company | Comment |
| --- | --- |
| New H3C | We are fine with this proposal |
| ZTE | Regarding the “Realistic update LOS probability”, does it mean it is up to companies to report the LOS probability? |
| Huawei, HiSilicon | Fine with this proposal. Similar to our previous comment, we suggest LOS probability to be 0.8 at least. For HetNet, the path loss model of Macro cell (Aggressor)->indoor small cell (Victim) should use Uma+penetration loss. |
| Intel | We are generally fine with the proposal, but we have some concerns with the term “realistic” added in the second sub-bullet, and with what this may imply and on how “realistic” will be met when updating the LOS probability. Therefore, we would prefer to remove it and agree on the original text. |
| Nokia, NSB | We in principle support the proposal, but require further clarifications on what the bulletpoint ‘Realistic update LOS probability’ means as also commented by ZTE. |
| QC | Support in general. However, we think more details is needed on the bullet of Realistic update LOS probability. |
| Fujitsu | Support |
| LG | Support |
| vivo | We are fine with the proposal |
| CATT | We are fine with FL proposal. |
| Ericsson | We support the proposal in principle. We agree with Nokia/Huawei that clarification is needed on the LOS probability. Our comment was that it needs to be realistic. For e.g., in Macro Scenarios where gNBs are located at 25m heights, there is almost always 100% LOS between gNBs. Therefore, we think 100% LOS probability needs to be assumed as it is the worst case for most scenarios. We are also open to Huawei’s suggestion to use at least 0.8. |
| Spreadtrum | We are generally fine with this proposal. But if ‘realistic’ means that it is up to companies to report as described in ZTE, we also have a concern in the calibration. |
| Xiaomi | We are also confused bout ‘realistic update LOS probability’. From our understanding, the intention here is we need to discuss how to calculate LOS probability in a realistic way, and the calculating method should be same among companies. |
| Samsung | What is meaning of “realistic”? how does RAN1 make realistic LOS probability? “updated LOS probability. FFS: details” is acceptable. |
| OPPO | The clarification for “realistic” is needed. |

#### Updated proposal 4-4-3a:

For UE-UE channel model, select option 1:

- Option 1: Reuse the gNB-UE 5GCM in TR 38.901 with necessary modifications for both FR1 and FR2, similar as the UE-UE channel model for flexible duplex evaluation in TR38.802 for FR2.

  - For Indoor hotspot, reuse the gNB-UE 5GCM Indoor-office in TR38.901, and for Dense urban and Urban macro, reuse the gNB-UE 5GCM Umi-Street canyon in TR38.901 with necessary modification, e.g.,

    - Replacing the gNB’s antenna height with UE’s antenna height, updating ASD and ZSD.

    - FFS: Other details and necessary modifications.

- Option 2: Reuse the UE-UE channel model for flexible duplex evaluation in TR 38.802 for both FR1 and FR2 with necessary modifications.

| Company | Comment |
| --- | --- |
| New H3C | Either way is fine to us. We slightly prefer option 2. If majority views prefer option 1, we can live with option 1. |
| ZTE | Option1 is preferred. |
| Huawei, HiSilicon | Fine with the proposal. |
| Intel | We are fine with the proposal. |
| Nokia, NSB | Support the proposal. |
| Fujitsu | Support. |
| LG | We prefer option 2, which is already used for NR simulation. |
| Vivo | We are fine with the proposal |
| CATT | We are fine with FL proposal and we prefer Option 1. |
| Ericsson | We are fine with the proposal and support Option1. |
| Spreadtrum | We are fine with the proposal and support Option1. |
| Xiaomi | Fine with the proposal. |
| Samsung | We are fine with both options and ok to go with majority views. |

### 5.4.6 5th Round Proposals (open)

#### Updated proposal 4-4-1b:

For gNB-gNB co-channel/adjacent-channel channel model and UE-UE co-channel/adjacent-channel channel model in RAN1 SLS,

- Large scale fading (e.g., path loss, penetration loss, shadowing) should be modelled, and companies report whether small scale fading (e.g., fast fading including antenna gain) is also modeled in their simulation.

- Note: Antenna gain is calculated based on the gNB-gNB or UE-UE LOS direction instead on the multi-path directions if fast fading is not modeled.

- FFS: how to model realistic LOS probability for gNB-gNB and UE-UE channel model.

- FFS: How to set aligned channel model amongst companies for SLS calibration (if needed).

| Company | Comment |
| --- | --- |
| Moderator | Updated based on comments. |
| ZTE | OK |
| New H3C | OK |
| MediaTek | Fine with the proposal. |
| Xiaomi | Support. |
| Spreadtrum | OK |
| Huawei, HiSilicon | OK |
| Sony | Support. |
| Nokia, NSB | Support |
| vivo | Support |
| Ericsson | Support |
| QC | Support |
| Intel | We really fail to understand how calibration among companies will be performed if whether small scale fading is modelled or not would be up to companies. Won’t this require companies to simulate both cases or is the expectation that small scale fading won’t impact results? |
| Samsung | Fine with the proposal |
| CATT | Fine with the proposal |

#### Updated proposal 4-4-2b:

For gNB-gNB channel model, reuse gNB-to-UE channel model in TR 38.901 with necessary modification, e.g.,

- Replacing the UE’s antenna height with gNB’s antenna height, updating the angular spread

- FFS: whether/how to update LOS probability.

- FFS: Other details and necessary modifications

| Company | Comment |
| --- | --- |
| Moderator | Updated based on comments. I think current ‘FFS: whether/how to update LOS probability’ is general enough, and we can further discuss how to make the LOS probability realistic later. |
| ZTE | OK |
| Spreadtrum | Support |
| Huawei, HiSilicon | OK |
| Sony | Support. |
| Nokia, NSB | Support |
| vivo | Support |
| Ericsson | Support. |
| QC | Support |
| Intel | Support |
| Samsung | Fine with the proposal |
| CATT | Fine with the proposal |

#### Updated proposal 4-4-3b:

For UE-UE channel model, select option 1:

- Option 1: Reuse the gNB-UE 5GCM in TR 38.901 with necessary modifications for both FR1 and FR2, similar as the UE-UE channel model for flexible duplex evaluation in TR38.802 for FR2.

  - For Indoor hotspot, reuse the gNB-UE 5GCM Indoor-office in TR38.901, and for Dense urban and Urban macro, reuse the gNB-UE 5GCM Umi-Street canyon in TR38.901 with necessary modification, e.g.,

    - Replacing the gNB’s antenna height with UE’s antenna height, updating ASD and ZSD.

    - FFS: Other details and necessary modifications.

- Option 2: Reuse the UE-UE channel model for flexible duplex evaluation in TR 38.802 for both FR1 and FR2 with necessary modifications.

| Company | Comment |
| --- | --- |
| Moderator | It seems majority are OK with this proposal. Let’s have a last check. |
| ZTE | OK |
| Spreadtrum | OK |
| Huawei, HiSilicon | OK |
| Sony | Support. |
| Nokia, NSB | Support |
| vivo | Support |
| Ericsson | We support the proposal and prefer Option 1. |
| QC | We think that option 2 is more accurate for UE-UE channel modeling, especially path loss computation for couple of reasons: - The path loss table in 38.901 is based on applicability range for hBS = 25m. So, reusing this to model UE-to-UE channel by setting gNB height to 1.5 is not valid neither a good approximation. Also, for UMi, using 38.901 is not even an option as hBS is not even a parameter of the path loss equation for NLOS. - Extending 38.901 might not be a good choice as the Uma and Umi models in 38.901 considers the minimum distance between gNB-UE are 35, and 10 m respectively (Table 7.2-1 in 38.901). Those models might not hold true for a distance < 35 m. We suggest companies to have a second look and consider option 2. |
| Intel | Support. |
| Samsung | We are fine with both options and ok to go with majority views. |
| CATT | Fine with the proposal |

## 5.5 Issue #4-5: Other issues

### 5.5.1 Submitted proposal

| Company | Proposals |
| --- | --- |
| CMCC | Proposal 20: For DL transmission power, the following options can be considered for SBFD. - Opt 1: The DL power spectrum density is kept unchanged over DL only slots and SBFD slots. - Opt 2: The DL power spectrum density can be boosted in SBFD slots compared to that in DL only slots. Proposal 21: Option 1 for DL transmission power can be used for calibration of SBFD. - Opt 1: The DL power spectrum density is kept unchanged over DL only slots and SBFD slots. On Scenario C3 (FR1) Proposal 25: No system level simulation is needed for Scenario C3 (Urban Macro considering co-channel co-existence with legacy TDD operation). On Scenario D3 (FR1) Proposal 26: It is not necessary for RAN1 to perform the performance evaluation for adjacent-channel co-existence between SBFD operation and legacy TDD operation. |
| Ericsson | Proposal 13: It is important to consider UE-UE interference in the study which is a potential consequence of the clustered distribution UEs that are of indoors. Proposal 15: In the system level simulations, the scheduled bandwidth for UL transmissions (within a semi-statically configured UL subband) should be adaptive based on achieving a target S(I)NR, for both static TDD and SBFD networks. |
| LG | Proposal 5: DL Tx power control can be applied for evaluation of subband non-overlapping full duplex and dynamic/flexible TDD. |
| MediaTek | Proposal 5: For the evaluations of SBFD and DTDD schemes, RAN1 should consider clustered UEs deployments to accurately capture the impact of inter-UE CLI. |

# 6 Stable Proposals

#### Initial proposal 4-2-3a:

For SBFD simulation, consider 4GHz for FR1 and 30GHz for FR2-1.

#### Updated proposal 4-3-1a:

For evaluation of SBFD operation, BS uses separate panels for simultaneous downlink transmission and uplink reception, we can call it separate-Tx/Rx antenna array for description of evaluation assumption.

- Companies can report the separation of the Tx panel and Rx panel assumed in their simulation.

- Companies can report how the antenna elements are used for transmission or reception in a slot if BS does not perform simultaneous downlink transmission and uplink reception.

#### Updated proposal 4-3-2a:

For evaluation of legacy TDD operation, BS uses the same antenna array for downlink transmission and uplink reception, we can call it shared-Tx/Rx antenna array for description of evaluation assumption.

#### Updated proposal 4-3-3a:

For evaluation and comparison between SBFD and legacy TDD, assume the total number of TxRUs of the antenna array for SBFD is the same as the total number of TxRUs of the antenna array for legacy TDD. Regarding antenna elements, both of the two options can be used.

- Opt 1: The total number of antenna elements of the antenna array for SBFD is the same as the total number of antenna elements of the antenna array for legacy TDD.

- Opt 2: The total number of antenna elements of the antenna array for SBFD is two times of the total number of antenna elements of the antenna array for legacy TDD.

- Companies report which option is assumed in their simulation.

# 7 Proposals for GTW

Updated proposal 3-3c:

Regarding gNB self-interference modelling for system level simulation purpose, consider introducing ratio of self-interference (RSI) to represent the overall self-interference suppression capability of gNB by means of spatial isolation, subband frequency isolation, digital interference cancellation and beamform nulling/isolation, etc. RSI also takes into account the impact of Tx/Rx antenna element gain on self-interference. The RSI, denoted as αSIm,n,  can be defined as the ratio of the total power transmitted by gNB across all transmit chains on a frequency unit m (e.g., subband/RB/subcarrier m) in a SBFD carrier to the residual self-interference received by the same gNB on a single receiver chain on a different frequency unit n (e.g., another subband/RB/subcarrier n) in the same SBFD carrier.

- FFS: Model for link level simulations and relevant questions to ask RAN4

- FFS: details of gNB self-interference modelling using RSI in SLS. As one example based on per-RB-RSI, the gNB self-interference on a single receiver chain at UL RB n can be modelled as

  - ISIn=m≠nm∈DL subbandISIm,n, wherein,

    - dBmISIm,n=Ptxm-dBαSIm,n

    - ISIm,nis the gNB self-interference on a single receiver chain at UL RB n caused by DL transmission on DL RB m.

    - m is the DL RB index in DL subbands.

    - Ptxm is gNB’s DL transmission power across all transmit chains at RB m (in dBm).

    - αSIm,n is the per-RB-RSI.

  - FFS: consider a statistical clutter model based on statistics of clutter strength and AoA.

- The following should be asked to RAN4:

  - What is the value range of RSI αSI(n,m) for each frequency range, and under what assumptions on the self-interference suppression means the value range of RSI is provided?

    - RAN1 understands the RSI can be described per subband, per RB, or per subcarrier depending on the granularity of the frequency unit, and it is up to RAN4 to provide the RSI in which granularity.

  - Whether it is possible for RAN4 to provide RAN1 the respective capabilities of different self-interference suppression means? e.g., is it possible to provide the separate estimates for spatial isolation, subband frequency isolation, beamform nulling/isolation, and digital cancellation, etc., as below?

    - dB(αSI(m,n))= dB(αSI-spatial)+dB(αSI-frequency (m,n))+dB(αSI-beam)+dB(αSI-digtial) +…

      - αSI-spatial denotes the spatial isolation.

      - αSI-frequency (m,n) denotes the suband frequency isolation between the Tx frequency unit m and the Rx frequency unit n.

      - αSI-beam denotes the beamform nulling or beam isolation.

      - αSI-digtial denotes the digital cancellation capability.

  - Whether it is possible to simplify the RSI as frequency flat model, and under which condition(s) the dependency of the RSI on frequency can be ignored?

  - The feasibility of provided value range of RSI regarding factors such as blocking, AGC, etc.

  - Does RSI have any dependency with the following factors or any other factors? What are the dependencies?

    - gNB’s antenna aspects, e.g., the assumed antenna architecture, the number of transmit chains and receive chains, etc.

    - Frequency aspects, e.g., the frequency distance between the Tx frequency unit m and the Rx frequency unit n, the number of RBs allocated for DL transmission, etc.

    - Beam aspects, e.g., Tx/Rx beam-pair for FR1/FR2 especially for clutter echo, etc.

- Note: RAN1’s consideration on the frequency locations and sizes of SBFD DL subband and SBFD UL subband assumed in SBFD operation can be provided to RAN4.

### Updated proposal 3-1c(v2):

For discussion purpose, consider the following as RAN1’s common understanding:

- Co-channel interference: The interference is from the aggressor to the victim in the same carrier.

  - Co-channel intra-subband interference: The interference is caused by transmission of the aggressor on a set of RBs in a carrier to reception of the victim on the same set of RBs in the same carrier.

  - Co-channel inter-subband interference: The interference is caused by transmission of the aggressor in a first set of RBs in a carrier to reception of the victim in a second set of RBs in the same carrier, where the two RB sets are non-overlapping in frequency.

- Adjacent channel interference: The interference is from the aggressor in carrier#1 to the victim in carrier#2, where the carrier#1 and carrier#2 are adjacent carriers.

Note 1: ‘Co-channel’ here means ‘co-carrier’. ‘Adjacent-channel’ here means ‘adjacent-carrier’.

### Updated proposal 3-2c(v2):

For discussion purpose, consider the following as the common understanding in RAN1 on the definition of interference types for SBFD operation:

- gNB self-interference (SI): Interference caused by DL transmission on a set of DL RBs in a carrier to UL reception on a set of UL RBs in the same carrier at the gNB side, where the two RB sets are non-overlapping in frequency.

- gNB-UE co-channel intra-subband interference: This is the same as the legacy DL interference type in legacy TDD network with static TDD UL/DL configuration.

- UE-gNB co-channel intra-subband interference: This is the same as the legacy UL interference type in legacy TDD network with static TDD UL/DL configuration.

- (inter-cell) inter-site gNB-gNB co-channel intra-subband CLI: CLI caused by DL transmission of the aggressor gNB on a set of RBs in one carrier to UL reception of the victim gNB in a different site on the same set of RBs in the same carrier.

- (inter-cell) co-site inter-sector co-channel intra-subband CLI: CLI caused by DL transmission of the aggressor gNB on a set of RBs in one carrier to UL reception of the victim gNB in another sector of the same site on the same set of RBs in the same carrier.

- (inter-cell) UE-UE co-channel intra-subband CLI: CLI caused by UL transmission of the aggressor UE on a set of RBs in one carrier to DL reception of the victim UE on the same set of RBs in the same carrier.

- (inter-cell) inter-site gNB-gNB co-channel inter-subband CLI: CLI caused by DL transmission of the aggressor gNB on a first set of RBs in a carrier to UL reception of the victim gNB in a different site on a second set of RBs in the same carrier, where the two RB sets are non-overlapping in frequency.

- (inter-cell) co-site inter-sector co-channel inter-subband CLI: CLI caused by DL transmission of the aggressor gNB on a first set of RBs in a carrier to UL reception of the victim gNB in another sector of the same site on a second set of RBs in the same carrier, where the two RB sets are non-overlapping in frequency.

- (intra-cell/inter-cell) UE-UE co-channel inter-subband CLI: CLI caused by UL transmission of the aggressor UE on a first set of RBs in a carrier to DL reception of the victim UE on a second set of RBs in the same cell or neighboring cell in the same carrier, where the two RB sets are non-overlapping in frequency.

- gNB-gNB adjacent-channel CLI: CLI caused by DL transmission of the aggressor gNB in a carrier to UL reception of the victim gNB in another adjacent carrier.

  - This includes adjacent-channel CLI between gNBs in the same and different sectors of the same site, i.e., co-site intra and inter-sector adjacent-channel CLI.

- UE-UE adjacent-channel CLI: CLI caused by UL transmission of the aggressor UE in a carrier to DL reception of the victim UE in another adjacent carrier.

Note: Some of the interferences may not be used according to the deployment scenarios, e.g, whether the SBFD subband configurations are the same or different across gNBs.

Note: This does not imply we need to consider all the above interference types in evaluation for SBFD.

### Updated proposal 3-5d(v1):

Regarding gNB-gNB and UE-UE adjacent-channel CLI modelling for system level simulation, RAN1 understands at least the following aspects need to be considered:

- Aspect 1: The unwanted emissions due to Tx non-linearity at the transmitter of the aggressor from the allocated RBs in one carrier to the non-allocated RBs in the adjacent carrier.

- Aspect 2: The receiver selectivity at the victim to receive the desired signal in the allocated RBs in one carrier in the presence of the unwanted signals at the non-allocated RBs in the adjacent carrier. (e.g. receiver blocking at the victim, overload of the receiver dynamic range, etc)

The following questions should be asked to RAN4:

- Whether it is feasible to consider the above two aspects for gNB-gNB and UE-UE adjacent-channel CLI modelling in system level simulation? Are there any other aspects should also be taken into account?

- For a specific pair of DL frequency unit m (e.g., subband/RB m) and UL frequency unit n (e.g., subband/RB n) of gNB-gNB link, where the DL frequency unit m and UL frequency unit n are in adjacent carriers and non-overlapping in frequency, and assuming the aggressor gNB transmits on the DL frequency unit m and the victim gNB receives on the UL frequency unit n,

  - How to model the interference from DL frequency unit m to UL frequency unit n due to Aspect 1 (defined above) at the gNB transmitter?

  - How to model the interference from DL frequency unit m to UL frequency unit n due to Aspect 2 (defined above) at the gNB receiver?

  - How to model the above interferences for the following cases:

    - the two gNBs are from the same sector of the same site in adjacent carriers, i.e., co-site co-sector gNB-gNB adjacent-channel CLI

    - the two gNBs are from different sectors of the same site in adjacent carriers, i.e., co-site inter-sector gNB-gNB adjacent-channel CLI

    - the two gNBs are from different sites in adjacent carriers, i.e., inter-site gNB-gNB adjacent-channel CLI

  - Whether it is feasible to define a similar interference ratio as BS-BS ACIR in TR38.828 but in the subband of the adjacent carrier, with finer granularity (e.g., per subband or per RB), to represent the overall effect of the Aspect 1 and Aspect 2 described above?

    - For example, whether it is feasible to define gNB-gNB-adjacent-channel-per-RB/subband interference ratio as the ratio of the power transmitted by the aggressor gNB on DL frequency unit m to the interference received by the victim gNB on UL frequency unit n? If it is feasible, then what is the value range of the gNB-gNB-adjacent-channel-per-RB/subband interference ratio for each frequency range?

- For a specific pair of DL frequency unit m (e.g., subband/RB m) and UL frequency unit n (e.g., subband/RB n) of UE-UE link, where the DL frequency unit m and UL frequency unit n are in adjacent carriers and non-overlapping in frequency, and assuming the aggressor UE transmits on the UL frequency unit n and the victim UE receives on the DL frequency unit m,

  - How to model the interference from UL frequency unit n to DL frequency unit m due to Aspect 1 (defined above) at the UE transmitter?

  - How to model the interference from UL frequency unit n to DL frequency unit m due to Aspect 2 at the UE receiver?

  - Whether it is feasible to define a similar interference ratio as UE-UE ACIR in TR38.828 but in the subband of the adjacent carrier, with finer granularity (e.g., per subband or per RB), to represent the overall effect of the Aspect 1 and Aspect 2 described above?

    - For example, whether it is feasible to define UE-UE-adjacent-channel-per-RB/subband interference ratio as the ratio of the power transmitted by the aggressor UE on UL frequency unit n to the interference received by the victim UE on DL frequency unit m? If it is feasible, then what is the value range of the UE-UE-adjacent-channel-per-RB/subband interference ratio for each frequency range?

### Updated proposal 3-5d(v2):

Regarding gNB-gNB and UE-UE adjacent-channel CLI modelling for system level simulation, RAN1 understands at least the following aspects need to be considered:

- Aspect 1: The unwanted emissions due to Tx non-linearity at the transmitter of the aggressor from the allocated RBs in one carrier to the non-allocated RBs in the adjacent carrier.

- Aspect 2: The receiver selectivity at the victim to receive the desired signal in the allocated RBs in one carrier in the presence of the unwanted signals at the non-allocated RBs in the adjacent carrier. (e.g. receiver blocking at the victim, overload of the receiver dynamic range, etc)

The following questions should be asked to RAN4:

- Whether it is feasible to consider the above two aspects for gNB-gNB and UE-UE adjacent-channel CLI modelling in system level simulation? Are there any other aspects should also be taken into account?

- For a specific pair of DL frequency unit m (e.g., subband/RB m) and UL frequency unit n (e.g., subband/RB n) of gNB-gNB link, where the DL frequency unit m and UL frequency unit n are in adjacent carriers and non-overlapping in frequency, and assuming the aggressor gNB transmits on the DL frequency unit m and the victim gNB receives on the UL frequency unit n,

  - How to model the interference from DL frequency unit m to UL frequency unit n due to Aspect 1 (defined above) at the gNB transmitter?

  - How to model the interference from DL frequency unit m to UL frequency unit n due to Aspect 2 (defined above) at the gNB receiver?

  - How to model the above interferences for the following cases:

    - the two gNBs are from the same sector of the same site in adjacent carriers, i.e., co-site co-sector gNB-gNB adjacent-channel CLI

    - the two gNBs are from different sectors of the same site in adjacent carriers, i.e., co-site inter-sector gNB-gNB adjacent-channel CLI

    - the two gNBs are from different sites in adjacent carriers, i.e., inter-site gNB-gNB adjacent-channel CLI

- For a specific pair of DL frequency unit m (e.g., subband/RB m) and UL frequency unit n (e.g., subband/RB n) of UE-UE link, where the DL frequency unit m and UL frequency unit n are in adjacent carriers and non-overlapping in frequency, and assuming the aggressor UE transmits on the UL frequency unit n and the victim UE receives on the DL frequency unit m,

  - How to model the interference from UL frequency unit n to DL frequency unit m due to Aspect 1 (defined above) at the UE transmitter?

  - How to model the interference from UL frequency unit n to DL frequency unit m due to Aspect 2 at the UE receiver?

### Updated proposal 3-3d:

For the purposes of link-level evaluations, an overall "net effect" model is needed that captures key components of the gNB transmit chain including crest-factor reduction (CFR) + digital pre-distortion (DPD) + power amplifier (PA), while satisfying the -45 dBc requirement on ACLR. For link-level simulations, the model should be sufficiently detailed to allow accurate generation of the self-interference signals to be cancelled. In Rel-15, RAN1 has asked RAN4 for realistic power amplifier model for 5G waveform evaluation (R1-163890 / R4-163128) and based on RAN4 response, RAN1 has adopted modified RAPP PA model (R1-167779) that considers DPD and CFR and 45 dB ACLR.

The following should be asked to RAN4

- For accurate generation of the self-interference signals at the gNB for the purposes of link level evaluation of self-interference cancellation, recommend a model to be used for capturing the overall "net" effect of key gNB transmit chain components, i.e., CFR + DPD + PA for both FR1 and FR2.

- Whether the RAPP PA model in R1-167779 can be used for the RAN1 evaluation and whether any further model enhancements is needed.

### Updated proposal 4-2-2b:

For SBFD evaluation, consider the following for SBFD subband configurations:

- SBFD Subband configuration#1 with {DUD} pattern, which means one SBFD slot consists of one UL subband at the center of the channel bandwidth and two DL subbands at two sides of the channel bandwidth.

- SBFD Subband configuration#2 with {DU} pattern, which means one SBFD slot consists of one UL subband at one side of the channel bandwidth and one DL subband at the other side of the channel bandwidth.

- Use the following parameters for description of SBFD subband configuration in evaluation assumptions:

  - ND: the number of RBs in one DL subband

  - NU: the number of RBs in one UL subband

  - NG: the number of RBs in one guard band between one UL subband and one DL subband

### Updated proposal 4-2-3d:

For performance evaluation and comparison between baseline legacy TDD operation and SBFD operation under SBFD Deployment Case 1 (Non-coexistence case with single SBFD subband configuration), consider the following alternatives:

- Baseline:

  - Alt 2 (No SBFD DL subband in the slots/symbols that correspond to UL slots/symbols in legacy TDD):

    - Legacy TDD: Static TDD UL/DL configuration with {DDDSU}, where S=[12D:2G:0U]

    - SBFD: Frame structure#2 (XXXXU), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about [20%] of the channel bandwidth.

  - Alt 4 (strive for the same UL/DL resource ratio between Legacy TDD and SBFD):

    - Legacy TDD: Static TDD UL/DL configuration with {DDDSU}, where S=[12D:2G:0U]

    - SBFD: Frame structure#3 (XXXXX), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about [20%] of the channel bandwidth.

- Optional:

  - Alt 1 (No SBFD DL subband in the slots/symbols that correspond to UL slots/symbols in legacy TDD):

    - Legacy TDD: Static TDD UL/DL configuration with {DDDSU}, where S=[12D:2G:0U]

    - SBFD: Frame structure#1 (DXXXU), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about [20%] of the channel bandwidth.

  - Alt 3 (strive for the same UL/DL resource ratio between Legacy TDD and SBFD):

    - Legacy TDD: Static TDD UL/DL configuration with {DDSUU}, where S=[12D:2G:0U]

    - SBFD: Frame structure#2 (XXXXU), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about [20%] of the channel bandwidth.

- FFS: whether dynamic TDD can optionally be used for legacy TDD for comparison.

### Updated proposal 4-4-1b:

For gNB-gNB co-channel/adjacent-channel channel model and UE-UE co-channel/adjacent-channel channel model in RAN1 SLS,

- Large scale fading (e.g., path loss, penetration loss, shadowing) should be modelled, and companies report whether small scale fading (e.g., fast fading including antenna gain) is also modeled in their simulation.

- Note: Antenna gain is calculated based on the gNB-gNB or UE-UE LOS direction instead on the multi-path directions if fast fading is not modeled.

- FFS: how to model realistic LOS probability for gNB-gNB and UE-UE channel model.

- FFS: How to set aligned channel model amongst companies for SLS calibration (if needed).

### Updated proposal 4-4-2b:

For gNB-gNB channel model, reuse gNB-to-UE channel model in TR 38.901 with necessary modification, e.g.,

- Replacing the UE’s antenna height with gNB’s antenna height, updating the angular spread

- FFS: whether/how to update LOS probability.

- FFS: Other details and necessary modifications

### Updated proposal 4-4-3b:

For UE-UE channel model, select option 1:

- Option 1: Reuse the gNB-UE 5GCM in TR 38.901 with necessary modifications for both FR1 and FR2, similar as the UE-UE channel model for flexible duplex evaluation in TR38.802 for FR2.

  - For Indoor hotspot, reuse the gNB-UE 5GCM Indoor-office in TR38.901, and for Dense urban and Urban macro, reuse the gNB-UE 5GCM Umi-Street canyon in TR38.901 with necessary modification, e.g.,

    - Replacing the gNB’s antenna height with UE’s antenna height, updating ASD and ZSD.

    - FFS: Other details and necessary modifications.

- Option 2: Reuse the UE-UE channel model for flexible duplex evaluation in TR 38.802 for both FR1 and FR2 with necessary modifications.

### Updated proposal 1-4b:

For SBFD Deployment Case 4, at least consider the following scenarios for evaluation from RAN1 perspective:

- FR1: Urban Macro

- FR2-1: Dense Urban Macro layer

- FFS: UE outdoor/indoor proportion, clustering, etc

- FFS: the grid shift between two networks, e.g., 0%, 100%

- FFS: Indoor hotspot, Dense Urban Micro layer

### Updated proposal 1-2c:

For SBFD evaluation from RAN1 perspective, consider the following deployment cases:

- Deployment Case 1

- Deployment Case 4 (strive for SLS methodology used in RAN1 to be coordinated with RAN4)

- Deployment Case 3-2 with 2-layer, in which one layer uses legacy TDD operation and the other layer uses SBFD operation

- Deployment Case 3-1 with 1-layer

- Deployment Case 2

Note: It is up to each company to provide the results. The results can be used to draw conclusion/recommendation depending on the number of companies providing the results.

### Updated proposal 2-1c:

For evaluation of dynamic/flexible TDD, consider the following scenarios:

- FR1

  - Indoor office with dynamic TDD UL/DL assignment

  - HetNet with Urban Macro and Indoor office deployed in the same carrier, and Macro gNBs use DL dominant static TDD UL/DL configuration. Both of the following options can be considered for this scenario.

    - Option 1: Indoor gNBs use UL dominant static TDD UL/DL configuration

    - Option 2: Indoor gNBs use dynamic TDD UL/DL assignment

  - Adjacent-channel coexistence case between dynamic TDD and legacy TDD

    - FFS: detailed scenario for adjacent-channel coexistence case

    - FFS: detailed simulation assumptions that are different from the Rel-16 co-existence study.

  - Urban Macro with dynamic TDD UL/DL assignment

  - Dense Urban with two layers deployed in the same carrier, and Macro gNBs use DL dominant static TDD UL/DL configuration. Both of the following options can be considered for this scenario.

    - Option 1: Micro gNBs use UL dominant static TDD UL/DL configuration

    - Option 2: Micro gNBs use dynamic TDD UL/DL assignment

- FR2-1

  - Indoor office with dynamic TDD UL/DL assignment

  - Dense Urban Macro layer with dynamic TDD UL/DL assignment

  - Adjacent-channel coexistence case between dynamic TDD and legacy TDD

    - FFS: detailed scenario for adjacent-channel coexistence case

    - FFS: detailed simulation assumptions that are different from the Rel-16 co-existence study.

Note: It is up to each company to provide the results. The results can be used to draw conclusion/recommendation depending on the number of companies providing the results.

# References

- RP-213591, New SI: Study on evolution of NR duplex operation, CMCC

- RP-220633, Revised SID: Study on evolution of NR duplex operation, CMCC

- R1-2203156	Overview of evaluation on NR duplex evolution	Huawei, HiSilicon

- R1-2203203	Discussion of evaluation on NR duplex evolution	ZTE

- R1-2203214	Discussion for Evaluation on NR duplex evolution	New H3C Technologies Co., Ltd.

- R1-2203327	Discussion on evaluation on NR duplex evolution	Spreadtrum Communications, BUPT

- R1-2203458	Discussion on deployment scenario and evaluation methodology for duplex operation	CATT

- R1-2203557	Evaluation on NR duplex evolution	vivo

- R1-2203814	Discussion on evaluation on NR duplex evolution	xiaomi

- R1-2203903	Deployment scenario and evaluation methodology for duplex evolution	Samsung

- R1-2204021	Discussion on evaluation on NR duplex evolution	OPPO

- R1-2204053	Evaluation on NR duplex evolution	SHARP Corporation

- R1-2204068	Evaluation assumption and methodology for study on NR-duplex	InterDigital, Inc.

- R1-2204106	Evaluation of NR duplex evolution	Ericsson

- R1-2204122	Discussion on deployment scenario of NR duplex evolution	KT Corp.

- R1-2204135	Discussion on evaluation on NR duplex evolution	Panasonic

- R1-2204244	Initial evaluation on NR duplex evolution	Apple

- R1-2204303	Discussion on evaluation on NR duplex evolution	CMCC

- R1-2204379	Discussion on evaluation on NR duplex evolution	NTT DOCOMO, INC.

- R1-2204430	On the evaluation methodology for NR duplexing enhancements	Nokia, Nokia Shanghai Bell

- R1-2204529	Study on Evaluation for NR duplex evolution	LG Electronics

- R1-2204721	Deployment scenarios and evaluation methodology for NR duplex evolution	MediaTek Inc.

- R1-2204750	Discussion on evaluation on NR duplex evolution	CEWiT

- R1-2204799	On evaluations for NR duplex evolution	Intel Corporation

- R1-2205030	On Deployment scenarios and evaluation Methodology for NR duplex evolution	Qualcomm Incorporated