3GPP TSG-RAN WG1 #109-e		R1-2205583

e-Meeting, May 09 - 20, 2022

Source: 	Moderator (Ericsson)

Title:      	Summary of Email discussion [109-e-R17_DSS-01]

Agenda item:	8.13

Document for:	Discussion and Decision

# 1 Introduction

This document provides summary of email discussion [109-e-R17_DSS-01] on following issues related to maintenance of Rel17 WI on DSS discussed during preparation phase of RAN1#109-eMeeting ([109-e-Prep-AI8.13 DSS] with related summary in [11])

[109-e-R17_DSS-01] Email discussion for moderator’s proposals in the FL summary R1-2205192 for maintenance on DSS – Ravi (Ericsson)

- Discussion and decision by May 18

Discuss the following clarifications/corrections in RAN1#109-e

- Allow ‘SCell dormancy indication’ to be sent on sSCell - [1]

- Clarify that 'Minimum applicable scheduling offset indicator' field is only applied to scheduling cell carrying the corresponding PDCCH - [1]

- Clarify that n_CI for P(S)cell self-scheduling is 0 - [3]

- Clarify 38.213 text on disabling alpha when sSCell is deactivated/dormant -- [4],[9]

- DCI size alignment between PCell and sSCell when the sSCell is deactivated or dormant - [4]

- If common SS set (x_p) of P(S)Cell and SS set (x_s) of sSCell are configured with same searchSpaceId value, then x_s is not used for CCS from sSCell to P(S)Cell and used for self-carrier scheduling - [6]

- Revise 38.212 text for CIF - [6]

- Support search space sharing on sSCell for a DCI format scheduling PCell transmission and a DCI format scheduling SCell transmission - [8]

- Clarify that search space sharing applies only for the PDCCH candidates in the same scheduling cell - [10]

# 2. Discussion

### 2.1 Topic 1

Please provide your input to below questions Q1-Q2 on this topic.

#### Question 1

Q1.  Is it OK to support below proposal from R1-2203102

- When PDSCH or PUSCH on a PCell can be scheduled by PDCCH of sSCell and the PCell, SCell dormancy indication can also be sent on the sSCell, and the dormancy indication on the PCell and on the sSCell are configured separately

Companies are requested to indicate their view about the above question in the Table below.

| Company Name | Yes/No | Comments (Topic 1, Q1) |
| --- | --- | --- |
| Qualcomm |  | We are open to discuss SCell dormancy indication by a DCI format 1_1/0_1 on the sSCell that schedules P(S)Cell. However, it is not clear to us why separate configurations are necessary for dormancy indication on the P(S)Cell and on the sSCell. It is also not clear whether the proposal here intends to support DCI 2_6 on the sSCell. |
| Samsung | No | The proposal is a minor enhancement with specification impact but without a need for maintenance purposes. SCell dormancy indication is an infrequent event compared to regular scheduling and can be handled by the PCell without meaningful impact on PDCCH offloading. |
| ZTE | Yes | We are fine with SCell dormancy indication by a DCI format 1_1/0_1 on the sSCell that schedules P(S)Cell. That is to keep the intention to just offload PDCCH to sSCell without additional unnecessary configuration. For Format 2_6, it is a group common DCI which is not the main control overhead and different from non-fallback DCI used for scheduling unicast. Thus, DCI format 2_6 can be kept on PCell. |
| LG Electronics | No | SCell dormancy indication on P(S)Cell is sufficient. |
| vivo |  | We support Scell dormancy indication by DCI 1_1/0_1. And we have similar questions with Qualcomm. |
| Apple |  | We are open to discuss. But in our view, this is only useful when USS cannot be configured on SpCell |
| Intel |  | We support SCell dormancy indication by a DCI 1_1/0_1 on sSCell. However, it is not clear why it should be separate configuration from P(S)Cell |
| OPPO | No | Agree with Samsung’s comment that this is a minor enhancement. |
| Nokia, NSB |  | It is OK to support the dormancy indication in sSCell, but only if this doesn’t add a new UE capability and there are no additional RRC impacts. The need to support dormancy indication in the sSCell is more nice-to-have than providing significant opportunity for PDCCH offloading. |
| Ericsson1 |  | We would be OK to support dormancy indication in sSCell, if this it can be done without additional new RRC/capability signaling. |
| Spreadtrum |  | Same view as QC. We think there is no strong motivation to support separate SCell dormancy indication configurations on PCell and sSCell, hope get some clarification. |
| Huawei, HiSilicon | Yes | @ Qualcomm - About separate configurations Since sSCell can be set dormant by PCell, so SCell can be included in a SCell group together with other SCells corresponding to the SCell dormancy indicate on the PCell. When the SCell dormancy indicates transmission on the sSCell, the gNB may not want to set the SCell to dormant. Therefore, for the implementation flexibility, separate configurations for the dormancy indication on the PCell and on the sSCell can be considered. - About Format 2_6 There is a conclusion at RAN1#106b_e: When sSCell to PCell cross-carrier scheduling is configured, DCI format 2_6 (if configured) is monitored only on P(S)Cell. In fact, we are open to discuss it. @ Samsung If only the PCell can send the SCell dormancy indication within the activation time, the USS must be configured on both the PCell and the sSCell. The USS on PCell may be configured just because it needs to send SCell dormancy indication. However, the configuration of the USS on the PCell requires more BD/CCEs allocated on the PCell, which reduces the number of BD/CCEs allocated on the SCell for scheduling the PCell and reduces the scheduling efficiency. |
| Moderator Notes1 |  | Please continue discussion using current table considering responses from Huawei, HiSilicon. |
| Samsung |  | There is no issue that requires a correction. It is reasonable for a UE to have USS sets on the PCell at least in slots that are UL slots on the sSCell. Just for the sake of argument, a UE could even be configured with a single USS set having a single PDCCH candidate on the PCell and there still wouldn’t be a problem. |
| Intel |  | We still think SCell dormancy indication from sSCell is necessary. For PDCCH offloading, one scenario is no USS at all on PCell. In this case, DCI for SCell dormancy indication is necessary On the other hand, even some PDCCHs can be considered on PCell, if the amount of PDCCH candidate is limited (small α), the DCI for SCell dormancy switching may not be transmitted in time considering potential PDCCH blocking. |
| LG Electronics |  | We still SCell dormancy indication on P(S)Cell is sufficient and share the view from Samsung. |
| Intel |  | For Type A UE, it is agreed in UE feature session to define an additional FG 34-1a for the DCI formats 0_1,1_1,0_2, 1_2 monitoring on P(S)Cell. Consequently, a UE of the basic FG 34-1 will not support the configuration of DCI forma 0_1/1_1 on P(S)Cell. To support SCell dormancy switching, the only way is to use a DCI format on sSCell to trigger the dormancy switching. |
| Huawei, HiSilicon |  | Share Intel’s view. |
| Samsung |  | We do not support the proposal. For Type-A/B UEs (34-1 and 34-2), a UE can be configured USS sets with DCI format 0_1/1_1 on PCell that provide SCell dormancy indication. We do not support specific enhancements targeting the “further simplified” UE (34-1a). If such UE is ever deployed, it can be served by fast SCell activation/deactivation and/or using DCI format 2_6. Basically, there is no critical issue related to UE power savings that needs to be corrected for Rel-17 DSS and to justify new RRC impact. |

#### Question 2

Q2. If yes to Q1, is it OK to agree to the TPs to 38.212 and 38.213 in Proposal 2 and Proposal 3 respectively of R1-2203102?

Companies are requested to indicate their view about the above question in the Table below.

| Company Name | Yes/No | Comments (Topic 1, Q2) |
| --- | --- | --- |
| Qualcomm |  | The TPs in Proposal 2 and Proposal 3 do not show how separate configurations are used – clarification is appreciated. |
| vivo |  | Share the same view with Qualcomm |
| Apple |  | The TP seems to be incomplete, what about the following paragraph in 38.213? If a UE is provided search space sets to monitor PDCCH for detection of DCI format 1_1, and if - the CRC of DCI format 1_1 is scrambled by a C-RNTI or a MCS-C-RNTI, and if - a one-shot HARQ-ACK request field is not present or has a '0' value, and if - the UE detects a DCI format 1_1 on the primary cell that does not include a carrier indicator field, or detects a DCI format 1_1 on the primary cell that includes a carrier indicator field with value equal to 0, and if |
| Intel |  | Share the same view with Qualcomm |
| Nokia, NSB |  | The TPs seem OK and if this is all that is needed we’d be OK with it, but we did not review if that is all that is needed and were not able to follow the Apple concern. |
| Spreadtrum |  | We think it should be clarified how the separate SCell dormancy indication configurations is used. |
| Huawei, HiSilicon | Yes | The separate configurations requires modifications to the TS38.331. An example (but not necessarily to be discussed in RAN1) TP in TS 38.331 6.3.2 Radio resource control information elements […] DormancyGroupID-r16 ::= INTEGER (0..4) DormancyGroupID-r17 ::= INTEGER (0..4) DormantBWP-Config-r16::= SEQUENCE { dormantBWP-Id-r16 BWP-Id OPTIONAL, -- Need M withinActiveTimeConfig-r16 SetupRelease { WithinActiveTimeConfig-r16 } OPTIONAL, -- Need M outsideActiveTimeConfig-r16 SetupRelease { OutsideActiveTimeConfig-r16 } OPTIONAL -- Need M } WithinActiveTimeConfig-r16 ::= SEQUENCE { firstWithinActiveTimeBWP-Id-r16 BWP-Id OPTIONAL, -- Need M dormancyGroupWithinActiveTime-r16 DormancyGroupID-r16 OPTIONAL -- Need R dormancyGroupWithinActiveTime-r17 DormancyGroupID-r17 OPTIONAL -- Need R } […] DormantBWP-Config field descriptions. dormancyGroupWithinActiveTime-r16 This field contains the ID of an SCell group for Dormancy within active time on PCell, to which this SCell belongs. The use of the Dormancy within active time SCell groups is specified in TS 38.213 [13].. dormancyGroupWithinActiveTime-r17 This field contains the ID of an SCell group for Dormancy within active time on the SCell scheduling the primary cell, to which this SCell belongs. The use of the Dormancy within active time SCell groups is specified in TS 38.213 [13].. […] @Apple We think we have modified the paragraph you gave in our TP. Could you please check or am I missing. |
| Moderator Notes1 |  | Please continue discussion using current table considering responses from Huawei, HiSilicon. |
| Intel |  | If there is a majority support of the separate configuration proposed by Huawei, we are fine with it. Essentially, we think it is beneficial to enable SCell dormancy switching by a DCI on sSCell. |

### 2.2 Topic 2

Please provide your input to below question Q1 on this topic.

#### Question 1

Q1. Do you see need to clarify the following as discussed in R1-2203102?

- The 'Minimum applicable scheduling offset indicator' field in the DCI on one scheduling cell is only applied to the scheduling cell carrying the corresponding PDCCH

Companies are requested to indicate their view about the above question in the Table below.

| Company Name | Yes/No | Comments (Topic 2, Q1) |  |
| --- | --- | --- | --- |
| Qualcomm | No | It is not clear what is the meaning of “min K0/K2 offset is applied only to the scheduling cell from the two candidate scheduling cells”. Once min K0/K2 offset is configured, the UE assumes the K0/K2 value in the DCI for scheduled PDSCH/PUSCH on the scheduled cell is not smaller than the min K0/K2 offset. This should be regardless of which scheduling cell the DCI format is carried. |  |
| Samsung | No | It is clear that DSS does not introduce new behavior for the minimum scheduling offset - the minimum scheduling offset is same as the Rel-16 one for self-scheduling or for cross-scheduling. |  |
| ZTE | No | From our understanding, there is no ambiguity issue, because scheduling offset are configured based on scheduled cell (PCell), and CCS scheduling with different SCS had already been supported in current spec. We think there is only one understanding which is aligned with HW’s proposal. So no need to clarify. |  |
| LG Electronics | No | We share the view from Qualcomm that indicated min K0/K2 offset is applied to the scheduled cell. |  |
| vivo | No | In our view, minimumSchedulingOffsetK0 is configured in scheduled cell and the 'Minimum applicable scheduling offset indicator' field in the DCI applies to both scheduling cells (Pcell and sScell) according to current spec. So there is no need to update the spec. |  |
| Apple | No | Should be the scheduled cell. There is nothing new compared to the Rel-16 when minimum scheduling offset is defined for power saving |  |
| Intel | No | We prefer to clarify that the min K0/K2 offset is applied to the scheduled cell. In other words, the indicate K0/K2 offset applies to DCI formats on both PCell and sSCell for PCell scheduling |  |
| Spreadtrum | No | Same view as QC and we think the min K0/K2 offset is applied to the scheduled cell. |  |
| Huawei, HiSilicon | Yes | It seems to us clear that this field also concerns scheduling cell in addition to scheduled cell that was mentioned by others, since it is an ‘offset’ then need to know offset from who-A to who-B. The scheduled cell is who-B which is obvious. This proposal is trying to clarify what scheduling cell is being referred to and what if not. For example the DCI format 0_1/1_1 sent on both scheduling cells may include the 'Minimum applicable scheduling offset indicator' field. If the 'Minimum applicable scheduling offset indicator' field in the DCI on one scheduling cell can be applied to both of the two scheduling cells, it means that the two scheduling cells use the same Minimum K0. As shown in the figure below, if Minimum K0=1, the time duration between PDCCH reception and PDSCH reception for two scheduling cells is t1 and t2 respectively. If the time duration between PDCCH reception and PDSCH reception for two scheduling cells need to be configured more flexibly, such as t1 and t3 respectively, then a simple solution is only applying the 'Minimum applicable scheduling offset indicator' field in the DCI only on the scheduling cell carrying this DCI including the 'Minimum applicable scheduling offset indicator' field. |  |
| Moderator Notes1 |  |  | Please continue discussion using current table considering responses from Huawei, HiSilicon. |
| Samsung |  | Still don’t understand what the issue is. It seems what is suggested related to the gNB providing inappropriate indication which is something that anyway cannot be avoided with the proposal. |  |
| Huawei, HiSilicon |  | Not sure we understand where companies’ confusion is. For simplicity, is it common understanding that in the above figure in our previous response, for the same scheduled cell (PCell), the PDCCH on PCell can indicate the minimum offset of K0=1 and the PDCCH on SCell can indicate the minimum offset of K0=2? (i.e. t1 and t3 is possible) |  |

### 2.3 Topic 3

Please provide your input to questions Q1-Q2 below

#### Question 1

Q1. Do you see need to modify the specification to make it clear that n_CI for P(S)cell self-scheduling is 0, as discussed in R1-2203528?

Companies are requested to indicate their view about the above question in the Table below.

| Company Name | Yes/No | Comments (Topic 3, Q1) |
| --- | --- | --- |
| Qualcomm | Yes | Since RAN1 agreed that CIF value is not always equal to nCI value (when CCS is configured from sSCell to P(S)Cell, CIF value in the DCI format on the P(S)Cell is reserved), this clarification is necessary. |
| Samsung |  | Even in Rel-15/16, the value of n_CI = 0 appears to be un-specified for self-scheduling in the case that “cif-Presence is set to false” (e.g., when the scheduling cell – here, the PCell – is not scheduling any other cell). Cif-Presence The field is used to indicate whether carrier indicator field is present (value true) or not (value false) in PDCCH DCI formats, see TS 38.213 [13]. If cif-Presence is set to true, the CIF value indicating a grant or assignment for this cell is 0. If a clarification is needed, RAN1 may consider a general Rel-16/17 CR (skip Rel-15) to capture that for self-scheduling of a scheduling cell, n_CI=0. That would be enough (also for DSS). |
| ZTE | Yes |  |
| vivo | Yes | We don’t think there is problem on general Rel-16/17. For the case that “cif-Presence is set to false” mentioned by Samsung, UE is not configured with a CIF by CrossCarrierSchedulingConfig. According to the following text in TS 38.213, it is clear that nCI=0 in this case nCI is the carrier indicator field value if the UE is configured with a carrier indicator field by CrossCarrierSchedulingConfig for the serving cell on which PDCCH is monitored; otherwise, including for any CSS, nCI=0; However for DSS case, UE is configured with CIF for scheduling Pcell. If the above text is not changed, Pcell self-scheduling will use the configured CIF value that is not correct understanding. The spec should be updated to make this clear. |
| Apple |  | We are fine to clarify it |
| Intel | Yes |  |
| OPPO | Yes |  |
| Nokia, NSB |  | OK to clarify |
| Ericsson1 |  | OK to clarify. |
| Spreadtrum | Yes | When sScell scheduling P(S)cell is configured, CIF value in the DCI format is just reserved field and has no special meaning. We think n_CI for P(S)cell self-scheduling is 0 in this case. |
| Huawei, HiSilicon | Yes |  |
| Moderator Notes1 |  | Companies seem to agree that clarification is needed. Proposal-TP2 related to the discussion proposed on reflector. |

#### Question 2

Q2. If yes to Q1, is it OK to agree to the TP#1 in section 2.1 of  R1-2203528?

Companies are requested to indicate their view about the above question in the Table below.

| Company Name | Yes/No | Comments (Topic 3, Q2) |
| --- | --- | --- |
| Qualcomm | Yes | We are OK with the TP. |
| Samsung | No | If a clarification TP is agreed, it can be simpler and more generic – e.g. as follows “nCI is the carrier indicator field value if the UE is configured with a carrier indicator field by CrossCarrierSchedulingConfig for the serving cell on which PDCCH is monitored; otherwise, including for any CSS or for scheduling from the serving cell on the serving cell, nCI=0;” |
| ZTE | Yes |  |
| LG Electronics |  | If TP is needed for clarification, an alternative could be the following. “nCI is the carrier indicator field value if the UE is configured with a carrier indicator field by CrossCarrierSchedulingConfig for the serving cell on which PDCCH is monitored except for scheduling from the serving cell on the serving cell; otherwise, including for any CSS, nCI=0;” |
| vivo | Yes | We support the TP as proponent |
| Apple |  | LG TP looks better for us |
| Intel |  | LG TP looks better for us |
| Nokia, NSB |  | Samsung/LG approach seems less intrusive. But what is the meaning of “for scheduling from the serving cell on the serving cell”? Should it be from secondary xxx to primary? In the LG example n_cl is not the carrier indicator field when the “except” condition triggers, but then nothing else is said for n_cl in that case, leaving it undefined. The “otherwise” clause triggers only if the “except” clause did not trigger, and would need to ne changed to “in which case”, but then it would not anymore be the otherwise clause it was before. |
| Ericsson1 |  | OK in principle with compact TP like Samsung suggested, but with slight update as shown below. “nCI is the carrier indicator field value if the UE is configured with a carrier indicator field by CrossCarrierSchedulingConfig for the serving cell on which PDCCH is monitored; otherwise, including for any CSS or for scheduling of the serving cell from the same serving cell, nCI=0;” |
| Spreadtrum | Yes | Agree with Ericsson’s update |
| Moderator Notes1 |  | Proposal-TP2 related to the discussion proposed on reflector. |
| Moderator Notes2 |  | After further discussion on [109-e-R17_DSS-01] email reflector thread following was agreed Agreement - Adopt the TP of Propsal-TP2v2 in R1-2205584 for 38.213 v17.1.0 sub-clause 10.1 |

### 2.4 Topic 4

Please provide your input to questions Q1-Q2 below.

#### Question 1

Q1. Is it OK to agree to TP#1 of R1-2203876?

Companies are requested to indicate their view about the above question in the Table below.

| Company Name | Yes/No | Comments (Topic 4, Q1) |
| --- | --- | --- |
| Qualcomm | No | First of all, we prefer to have a clear text in the spec for the paragraph. TP#1 of R1-2204962 is clear. With this, it is clear that if the UE indicates support of disabling scaling factor alpha and if the sSCell is deactive or dormant, the UE does not count BD or CCE on the sSCell for P(S)Cell scheduling. Therefore, it seems TP#1 of R1-2203876 is not necessary if TP#1 of R1-2204962 is adopted. |
| Samsung | Yes | 38.213 includes a reference from clause 10.1.1 to clause 10.1 for the case of deactivated sSCell. However, Clause 10.1 includes procedures for BD/CCE counting for deactivated scheduling cells. Then, current text for PDCCH monitoring for DSS in 10.1.1 may be interpreted to imply that CCS from sSCell to PCell remains applicable and that BD/CCE limits include the sSCell as a scheduling cell for Pcell when the sSCell is deactivated. The proposed TP intends to simply clarify that CCS from sSCell to Pcell is not applicable for PDCCH monitoring procedures when sSCell is deactivated or has a dormant active DL BWP. |
| ZTE | No | Similar view as Qualcomm |
| LG Electronics | Yes | We slightly prefer Samsung’s TP to minimize the specification impact. |
| Vivo |  | We should down select one TP from Q1 and Q2. We are fine with either one. TP from Q1 should address separate UE capability issue if agreed in UE feature. |
| Intel | No | Similar view as Qualcomm |
| OPPO | Yes |  |
| Ericsson1 | No | Agree with Qualcomm. The proposed TP does not address below issue raised in our contribution According to agreements separate UE capabilities are provided for disabling scaling factor α due to sSCell deactivation (say FG 34-x) and for disabling scaling factor α when activated sSCell is switched to dormant BWP (say FG 34-y). For example, if the UE indicates FG 34-x and does not indicate FG 34-y, scaling factor α is not disabled when sSCell is switched to dormant BWP. However, according to current specification text or the proposed TP, it is not clear whether scaling factor α is disabled or not (i.e., whether text in subclause 10.1.1 should be applicable or not) for such cases where UE indicates different values for support of FG 34-x and 34-y. Also, other procedures based on SCell to Pcell scheduling being configured such as DCI size matching should continue regardless of sSCell deactivation/dormant BWP and the proposed TP seems not aligned with that. |
| Spreadtrum | No | Agree with QC and we prefer TP#1 of R1-2204962. |
| Huawei, Hisilicon |  | Similar view as Qualcomm |
| Moderator Notes1 |  | Please provide further comments (if any) using current Table. |
| Samsung |  | OK to go with the majority. |

#### Question 2

Q1. Is it OK to agree to TP#1 of R1-2204962?

Companies are requested to indicate their view about the above question in the Table below.

| Company Name | Yes/No | Comments (Topic 4, Q2) |
| --- | --- | --- |
| Qualcomm | Yes | We are OK with the TP. |
| Samsung | No | The value ‘α = 1’ was discussed in RAN1 and not agreed. A simple clarification is sufficient and there is no need to replace the current wording. The issue of separate UE capabilities for sSCell deactivation and sSCell dormancy can be reflected in the TP for (Topic 4, Q1). |
| ZTE | Yes | Fine with this TP |
| LG Electronics |  | We slightly prefer Samsung’s TP to minimize the specification impact. |
| Vivo |  | We should down select one TP from Q1 and Q2. We are fine with either one. TP from Q1 should address separate UE capability issue if agreed in UE feature. |
| Apple |  | We slightly prefer this TP |
| Intel | Yes |  |
| Ericsson1 | Yes |  |
| Spreadtrum | Yes | We are OK with the TP. |
| Huawei, HiSilicon | Yes | We are OK with the TP. |
| Moderator Notes1 |  | There seems to be common understanding that the issue of separate UE capabilities for sSCell deactivation and sSCell dormancy needs to be addressed. Most companies seem to be OK with the TP in of R1-2204962. @Samsung, considering inputs so far, is it OK to agree to this TP? |
| Samsung |  | OK in principle with the TP from R1-2204962 but not in its current form. The two paragraphs say essentially the same thing (one for SCell deactivation, the other for cell dormancy) and can be combined to avoid duplication. Also, there is no need to say “… scaling factor α provided by ccs-BlindDetectionSplit is disabled and …” – it is enough to say that “α =1 is assumed …” – i.e. the following TP would be OK. If the UE indicates capability <FG 34-x> or <FG 34-y> [18, TS 38.306], and if the secondary cell is deactivated or the active DL BWP is a dormant DL BWP, α = 1 is assumed for the procedures described in remaining of this clause. |
| LG Electronics |  | We are OK with Samsung’s modified TP above. |
| Moderator Notes2 |  | After further discussion on [109-e-R17_DSS-01] email reflector thread following was agreed Agreement - Agree the TP Proposal-TP3v2 in R1-2205584 for 38.213 v17.1.0 sub-clause 10.1.1 |

### 2.5 Topic 5

Please provide your input to question Q1 below

#### Question 1

Q1. Do you see need for additional clarification related to DCI size alignment between non-fallback DCI formats of P(S)Cell and sSCell for case when sSCell is deactivated/dormant as discussed in Proposal 2 of R1-2203876

Companies are requested to indicate their view about the above question in the Table below.

| Company Name | Yes/No | Comments (Topic 5, Q1) |
| --- | --- | --- |
| Qualcomm | No | The proposal was not part of the RAN1 agreement. We do not see an essential need of this. |
| Samsung | Yes | RAN1 agreed to disable the scaling factor α for BD/CCE handling when sSCell is deactivated/dormant. Then, it would make sense for a consistent design to not have the DCI size alignment between PCell and sSCell. In any case, it would be good to have a RAN1 conclusion for what falls back to non-DSS and what remains as for DSS when the sSCell is deactivated/dormant. |
| ZTE | No | Based on our understanding, size alignment is semi-static determined, regardless of dynamic (de)activated or dormant. |
| LG Electronics |  | In our view, regardless of whether sSCell is deactivated/dormant or activated, DCI size alignment agreement is applied. |
| Vivo |  | If DCI size alignment is disabled when sScell is deactivated/dormant, the DCI size for Pcell self-scheduling may be changed based on dynamic activation/deactivation or dormancy/non-dormancy if DCI size for Pcell scheduling is smaller than sScell scheduling Pcell. This is indeed not expected. If DCI size alignment is still enabled when sScell is deactivated/dormant, the BWP used for determination of DCI size for sScell scheduling Pcell should be discussed. Besides, the DCI size may also be changed if DCI size for Pcell scheduling is smaller than sScell scheduling Pcell. To avoid DCI size change for Pcell self-scheduling, an alternative way could be: UE doesn’t expect that DCI size for Pcell self-scheduling is smaller than that for sScell scheduling Pcell when sScell scheduling Pcell is configured. |
| Apple |  | We prefer the clarification that sSCell (de)activation and dormancy operation will not impact DCI size |
| Intel |  | We don’t think there is a real performance difference, so fine to take a simple way to maintain the DCI size |
| OPPO | No | In our understanding, the DCI size alignment is based on the cell configuration. When sSCell is deactivated/dormant, if the configuration of CCS for PCell is not changed, the DCI size alignment should be maintained. |
| Nokia, NSB | No | We don’t see it a good idea that the DCI size varies based on deactivation/dormancy. Better to clarify that the DCI size is not impacted as Apple and Intel seem to imply. |
| Ericsson1 | No | We do not prefer this as it can lead to DCI size ambiguity for primary cell scheduling during sSCell activation/deactivation/dormancy, Etc. |
| Spreadtrum | No | For the scheduled P(S)cell, there is only one scheduling cell(i.e. P(S)cell) when sSCell is deactivated/dormant, so DCI size alignment between non-fallback DCI formats of P(S)Cell and sSCell is not required. |
| Moderator Notes1 |  | Please continue any further discussion using current Table. |
| Samsung |  | OK to conclude that DCI size alignment between PCell and sSCell is maintained for sSCell deactivation/dormancy. |

### 2.6 Topic 6

Please provide your input to question Q1 below

#### Question 1

Q1. Do you see the need to clarify the following as discussed in Proposal 1 of R1-2204624?

- If a common SS set (x_p) of P(S)Cell and SS set (x_s) of sSCell are configured with same SearchSpaceId value, then x_s is not used for CCS from sSCell to P(S)Cell and used for self-carrier scheduling.

Companies are requested to indicate their view about the above question in the Table below.

| Company Name | Yes/No | Comments (Topic 6, Q1) |
| --- | --- | --- |
| Qualcomm | No | gNB has flexibility on assigning the SS set IDs. There is no critical issue with the current rule. |
| Samsung | No | The Rel-16 procedure for search space linking was clarified for DSS and agreed in RAN1#108. There is no problem with current specifications and there is no need to revert that agreement as gNB configuration can handle linking for both USS and CSS sets. |
| ZTE | No | Similar view as Samsung. |
| LG Electronics | Yes | If this clarification is not adopted, SS set index that is used for Pcell CSS set cannot be used for USS set index on sSCell, which imposes limitation on the usage for SS set index on sSCell. |
| Vivo | Yes | We share the same view as LG. |
| Apple | No |  |
| Intel | No | There are large number of candidate SS set IDs. So there is no real problem for the current agreement. |
| OPPO | No | The linkage between the SS set(x_p) on Pcell and the SS set (x_s) on sSCell is handled by gNB. It is not necessary to add the restriction in spec. |
| Ericsson1 | No | It can left to gNB implementation. |
| Spreadtrum | No | We think there is no need to clarify whether the rule agreed in RAN1#108-e meeting is applied to common SS set of P(S)Cell. |
| Moderator Notes1 |  | Please continue any further discussion using current table. |
| Samsung |  | There is no essential, or even modestly important, correction to be made. |

### 2.7 Topic 7

Please provide your input to question Q1 below

#### Question 1

Q1. Do you see need to modify CIF description in 38.212 according to Proposal 2 of  R1-2204624?

Companies are requested to indicate their view about the above question in the Table below.

| Company Name | Yes/No | Comments (Topic 7, Q1) |
| --- | --- | --- |
| Qualcomm | No | We do not see a need to modify it (although technically not a problem with either). |
| Samsung | No | No need to revert the previous RAN1 agreement on CIF. Current specifications do not have a problem. |
| ZTE | No | Seems not essential. |
| LG Electronics | Yes | For example, if TCI field is absent for Pcell PDCCH but present for sSCell PDCCH (scheduling Pcell PDSCH), - According to current specification, CIF reserved 3 bits for Pcell PDCCH and 3 padding bits for sSCell PDCCH are needed, which is totally redundant. - On the other hand, based on Proposal 2 of R1-2204624, any of CIF reserved bits or padding bits are not necessary. Therefore, the proposal would avoid unnecessary DCI overhead especially for compact DCI payload size. |
| Vivo | No | No need to revert the RAN1 agreement. |
| Apple | No |  |
| Intel | No |  |
| OPPO | No | The spec is aligned with RAN1 agreement. There is no need to modify it. |
| Ericsson1 | No |  |
| Spreadtrum | No | Revising CIF agreement does not seem to be necessary. |
| Huawei, HiSilicon | Yes | DCI size can be reduced in some configurations. |
| Moderator Notes1 |  | Please continue any further discussion using current table. |
| Samsung |  | It is surprising to see companies preferring to keep the DCI size unchanged when the sSCell is deactivated/dormant, also propose a correction with the argument that it may result sometimes to a (smaller) reduction in DCI size. There is no impact on DCI BLER and there is no need for a correction. |

### 2.8 Topic 8

Please provide your input to question Q1 below

#### Question 1

Q1. Do you see need to clarify the following as discussed in Proposal 1 of  R1-2204778?

- Support search space sharing on sSCell for a DCI format scheduling PCell transmission and a DCI format scheduling SCell transmission if the two DCI have the same DCI size.

- The search space sharing is enabled by default for a DCI scheduling PCell transmission from sSCell

Companies are requested to indicate their view about the above question in the Table below.

| Company Name | Yes/No | Comments (Topic 8, Q1) |
| --- | --- | --- |
| Qualcomm |  | The first bullet is a normal search space sharing that has been supported so far, according to our understanding. We are not sure why the search space sharing is proposed to be enabled by default for cross-carrier scheduling from sSCell to P(S)Cell. We do not think it is by default and it shall be based on the corresponding optional UE capability. |
| Samsung | No | The proposal above appears to mandate search space set sharing by default for DSS, rather than based on UE capability. That is not reasonable. |
| ZTE |  | Topic 8 and Topic 9 can be merged together. Similar view as Qualcomm. |
| LG Electronics |  | It seems that the first bullet is overlapped with Topic 9 for which TP proposed in R1-2204996 is sufficient. For the second bullet, we share the view with Qualcomm that search space sharing is not default but optional feature as before. |
| vivo |  | Agree with Qualcomm and ZTE |
| Apple |  | TP in Topic 9 is enough |
| Intel | Yes | As we discussed in our contribution, the intention is to allow more PDCCH candidate for CCS from sSCell to PCell. Otherwise, based the agreed BD/CCE limit handling option A, the number of BD/CCE on sSCell for PCell scheduling may be quite limited. Therefore, it is preferred to bind the capability of SS set sharing with the feature of CCS from sSCell to PCell. If there is a concern to enable SS set sharing for any PCell/SCells, is it OK to limit the SS set sharing between a DCI for PCell and a DCI for sSCell self-scheduling? Then, other SS set sharing is still controlled by the UE capability. |
| OPPO |  | For the first bullet, it may need a clarification for the case where CCS and search space sharing are configured simultaneously. But for the second bullet, it seems not essential. The CCS could work well no matter whether there is search space sharing or not. |
| Ericsson1 |  | We think this can be addressed by topic 9. |
| Spreadtrum | No | We prefer TP in Topic 9. |
| Huawei, HiSilicon |  | Agree with Qualcomm and ZTE |
| Moderator Notes1 |  | Considering the comments for this Topic and Topic 9, it appears discussion on this Topic can be closed if TP agreed for Topic 9. If any further discussion, please continue using current table. |

### 2.9 Topic 9

Please provide your input to question Q1 below

#### Question 1

Q1. Is it OK to agree to TP to 38.213 proposed in R1-2204996 at least for Rel17? (Note, discussion also related to Topic 8)

Companies are requested to indicate their view about the above question in the Table below.

| Company Name | Yes/No | Comments (Topic 9, Q1) |
| --- | --- | --- |
| Qualcomm | Yes | Our proposal is that the search space sharing is still optional feature (not by default), and is limited within the same scheduling cell. |
| Samsung |  | We are OK with the proposal, but that also needs at least a Rel-16 CR – if the UE behavior is clear for Rel-16, it will be clear for DSS as well. |
| ZTE | Yes |  |
| LG Electronics | Yes |  |
| vivo | Yes |  |
| Apple | Yes |  |
| Intel | Yes |  |
| OPPO | Yes |  |
| Ericsson1 | Yes | Similar comment as Samsung that this can be applied for Rel-16. |
| Spreadtrum | Yes | We are OK to TP proposed in R1-2204996. Search space sharing applies only for the PDCCH candidates in the same scheduling cell. |
| Huawei, HiSilicon | Yes |  |
| Moderator Notes1 |  | TP seems to stable. Proposal-TP1 related to the discussion proposed on reflector. |
| Moderator Notes2 |  | After further discussion on [109-e-R17_DSS-01] email reflector thread following was agreed Agreement - Adopt the TP of Propsal-TP1 in R1-2205584 for 38.213 v17.1.0 sub-clause 10.1 - Note: Whether the change is also made to previous release specification(s) can be discussed in respective maintenance A.I. |

# 3 Conclusions

Following agreements were made in RAN1#109-e based on discussions in the [109-e-R17_DSS-01] email thread.

Agreement

- Adopt the TP of Propsal-TP1 in R1-2205584 for 38.213 v17.1.0 sub-clause 10.1

  - Note: Whether the change is also made to previous release specification(s) can be discussed in respective maintenance A.I.

Agreement

- Adopt the TP of Propsal-TP2v2 in R1-2205584 for 38.213 v17.1.0 sub-clause 10.1

Agreement

- Agree the TP Proposal-TP3v2 in R1-2205584 for 38.213 v17.1.0 sub-clause 10.1.1

Note: To editors, above proposals cited by Agreements could be found in R1-2205584, or in the section of summary of R1-2205583

# 4 References

| [1] | R1-2203102 | Discussion on NR Dynamic spectrum sharing | Huawei, HiSilicon |
| --- | --- | --- | --- |
| [2] | R1-2203196 | Maintenance of DSS and MR-DC | ZTE |
| [3] | R1-2203528 | Maintenance on Scell scheduling Pcell | vivo |
| [4] | R1-2203876 | Remaining details of NR dynamic spectrum sharing | Samsung |
| [5] | R1-2204005 | Remaining issues of cross-carrier scheduling from sSCell to Pcell | OPPO |
| [6] | R1-2204624 | Remaining issues on Cross-carrier scheduling from Scell to Pcell | LG Electronics |
| [7] | R1-2204694 | On Cross-Carrier Scheduling from sSCell to P(S)Cell | MediaTek Inc. |
| [8] | R1-2204778 | On SCell scheduling PCell transmissions | Intel Corporation |
| [9] | R1-2204962 | Maintenance for Rel-17 DSS | Ericsson |
| [10] | R1-2204996 | Maintenance on NR Dynamic spectrum sharing | Qualcomm Incorporated |
| [11] | R1-2205192 | Summary of [109-e-Prep-AI8.13 DSS] | Moderator (Ericsson) |
| [12] | R1-2205584 | Agreed TPs from Email discussion [109-e-R17_DSS-01] | Moderator (Ericsson) |