3GPP TSG-RAN WG1 #109-e		R1-2205584

e-Meeting, May 09 - 20, 2022

Source: 	Moderator (Ericsson)

Title:      	Agreed TPs from Email discussion [109-e-R17_DSS-01]

Agenda item:	8.13

Document for:	Discussion and Decision

# 1 Introduction

This document captures the TPs agreed in email discussion [109-e-R17_DSS-01].

Corresponding email discussion summary is provided in [1].

Note: CR coversheet text provided below each TP is draft provided by Moderator (for Editor’s consideration) and not RAN1 agreement.

# 2. Agreed TPs

Agreement

- Adopt the TP of Propsal-TP1 in R1-2205584 for 38.213 v17.1.0 sub-clause 10.1

  - Note: Whether the change is also made to previous release specification(s) can be discussed in respective maintenance A.I.

Agreed TP of Propsal-TP1 given below.

------------------- start TP to 38.213 sub-clause 10.1 ----------------------------
<unchanged text omitted>
A UE that

-    is configured for operation with carrier aggregation, and

-    indicates support of search space sharing through searchSpaceSharingCA-UL or through searchSpaceSharingCA-DL, and

-    has a PDCCH candidate with CCE aggregation level L in CORESET p associated with search space set si of a serving cell for detection of a first DCI format, other than DCI format 0_0 or DCI format 1_0, having a first size and scheduling

-    PUSCH transmission or configured grant Type 2 PUSCH release on serving cell nCI,2, or

-    PDSCH reception or having associated HARQ-ACK information without scheduling PDSCH reception on serving cell nCI,2

can receive a corresponding PDCCH through a PDCCH candidate with CCE aggregation level L in CORESET p associated with search space set sj of the serving cell for detection of a second DCI format having a second size and associated with scheduling on serving cell nCI,1 if the first size and the second size are same and if neither of search space sets si and sj includes searchSpaceLinking.
<unchanged text omitted>

------------------- end TP ----------------------------

CR coversheet text (draft provided by Moderator, not RAN1 agreement) for agreed Proposal-TP1

Reason for change: Search space sharing should be for PDCCH candidates of same scheduling cell.  but current text in 38.213 sub-clause 10.1 is ambiguous on whether it applies to candidates of the same scheduling cell or not (at least for the case when UE is configured for SCell to P(S)Cell scheduling).

Summary of change: Clarify in 38.213 sub-clause 10.1 that search space sharing applies only for the PDCCH candidates of the same scheduling cell

Consequences if not approved: Unclear behavior for PDCCH reception for a UE supporting searchSpaceSharingCA-UL or searchSpaceSharingCA-DL at least for case when SCell to P(S)Cell scheduling is configured.

Agreement

- Adopt the TP of Propsal-TP2v2 in R1-2205584 for 38.213 v17.1.0 sub-clause 10.1

Agreed TP of Propsal-TP2v2 given below.

------------------- start TP to 38.213 sub-clause 10.1 ----------------------------
<unchanged text omitted>
For a search space set s associated with CORESET p, the CCE indexes for aggregation level L corresponding to PDCCH candidate ms,nCI(L)  of the search space set in slot ns,fμ for an active DL BWP of a serving cell corresponding to carrier indicator field value nCI are given by

L⋅Yp,ns,fμ+ms,nCI(L)⋅NCCE,pL⋅Ms,maxL+nCImodNCCE,pL+i

where

for any CSS, Yp,ns,fμ=0;

for a USS, Yp,ns,fμ=Ap⋅Yp,ns,fμ-1modD, Yp,-1=nRNTI≠0, Ap=39827 for pmod3=0, Ap=39829 for pmod3=1, Ap=39839 for pmod3=2, and D=65537;

i=0,⋯,L-1;

NCCE,p is the number of CCEs, numbered from 0 to NCCE,p-1, in CORESET p and, if any, per RB set;

nCI is the carrier indicator field value if the UE is configured with a carrier indicator field by CrossCarrierSchedulingConfig for the serving cell on which PDCCH is monitored, except for scheduling of the serving cell from the same serving cell in which case nCI=0 ; otherwise, including for any CSS, nCI=0;

ms,nCI(L)=0,⋯,Ms,nCIL-1, where Ms,nCIL is the number of PDCCH candidates the UE is configured to monitor for aggregation level L of a search space set s for a serving cell corresponding to nCI;

for any CSS, Ms,maxL=Ms,0L;

for a USS, Ms,maxL is the maximum of Ms,nCIL over all configured nCI values for a CCE aggregation level L of search space set s ;

the RNTI value used for nRNTI is the C-RNTI.

<unchanged text omitted>

------------------- end TP ----------------------------

CR coversheet text (draft provided by Moderator, not RAN1 agreement) for agreed Proposal-TP2v2

Reason for change: The n_CI value to use for P(S)Cell self-scheduling is unclear from current text in 38.213 sub-clause 10.1 for case when SCell to P(S)Cell scheduling is configured for the UE (for this case, CIF field in DCI format 0_1,1_1,0_2,1_2 of P(S)Cell is reserved)

Summary of change: Clarify in 38.213 sub-clause 10.1 that n_CI=0 should be used for self-scheduling.

Consequences if not approved: Unclear UE behavior for PDCCH reception on P(S)Cell when SCell to P(S)Cell scheduling is configured for a UE.

Agreement

- Agree the TP Proposal-TP3v2 in R1-2205584 for 38.213 v17.1.0 sub-clause 10.1.1

Agreed TP Proposal-TP3v2 given below.

------------------------------- start TP to 38.213v17.1.0 subclause 10.1.1 ---------------------------------

Note: <FG 34-3> in TP below to be replaced by agreed UE capability name for disabling scaling factor α when sSCell is deactivated. <FG 34-4> in TP below to be replaced by agreed UE capability name for disabling scaling factor α when activated sSCell is switched to dormant BWP

10.1.1    Self-carrier and cross-carrier scheduling on the primary cell

A UE can be configured for scheduling on the primary cell from the primary cell and from a secondary cell [12, TS 38.331]. The UE is either not provided monitoringCapabilityConfig or the UE is provided only monitoringCapabilityConfig = r15monitoringcapability for the primary cell and for the secondary cell. The UE is not provided coresetPoolIndex on the primary cell or on the secondary cell.

The SCS configuration μP for the active DL BWP on the primary cell is smaller than or equal to the SCS configuration μS for the active DL BWP on the secondary cell.

For the remaining of this clause, either the secondary cell is activated and the active DL BWP is not a dormant DL BWP for a UE, or the UE does not indicate a capability [16, TS 36.306] for disabling α that is provided by ccs-BlindDetectionSplit; otherwise, procedures described in the remaining of this clause are not applicable for the UE and the procedures are as described in clause 10.1.

If the UE indicates capability <FG 34-3>  [18, TS 38.306] and the secondary cell is deactivated, or if the UE indicates capability <FG 34-4> [18, TS 38.306] and the active DL BWP of the secondary cell is a dormant DL BWP for the UE,  α =1 is assumed for the procedures described in remaining of this clause.

If μP<μS, the UE determines MPDCCHtotal,slot,μP and CPDCCHtotal,slot,μP, and determines MPDCCHtotal,slot,μS and CPDCCHtotal,slot,μS, by including the primary cell only in the Ncells,0DL,μP downlink cells in j=03Ncells,0DL,j+γ∙Ncells,1DL,j, as described in clause 10.1. If μP=μS=μ, the UE determines MPDCCHtotal,slot,μ and CPDCCHtotal,slot,μ by including the primary cell once in the Ncells,0DL,μ downlink cells in j=03Ncells,0DL,j+γ∙Ncells,1DL,j, as described in clause 10.1.

For scheduling on the primary cell from the primary cell, the UE is not required to monitor more than α∙minMPDCCHmax,slot,μP,MPDCCHtotal,slot,μP  PDCCH candidates per slot or more than α∙minCPDCCHmax,slot,μP,CPDCCHtotal,slot,μP non-overlapping CCEs per slot on the active DL BWP of the primary cell, where α is provided by ccs-BlindDetectionSplit.

For scheduling on the primary cell from the secondary cell, the UE is not required to monitor on the active DL BWP of the secondary cell more than

-    MPDCCHmax,slot,μS PDCCH candidates per slot or more than CPDCCHmax,slot,μS non-overlapping CCEs per slot of the active DL BWP of the secondary cell

-    minMPDCCHmax,slot,μP,MPDCCHtotal,slot,μP-α∙minMPDCCHmax,slot,μP,MPDCCHtotal,slot,μP PDCCH candidates per slot or more than minCPDCCHmax,slot,μP,CPDCCHtotal,slot,μP-α∙minCPDCCHmax,slot,μP,CPDCCHtotal,slot,μP non-overlapping CCEs per slot of the active DL BWP of the primary cell

If μP<μS, the UE does not count PDCCH candidates and non-overlapping CCEs that the UE monitors for scheduling on the primary cell from the secondary cell towards MPDCCHtotal,slot,μS and CPDCCHtotal,slot,μS, respectively.

If μP<μS, the UE counts PDCCH candidates and non-overlapping CCEs that the UE monitors for scheduling on the primary cell from the secondary cell towards MPDCCHtotal,slot,μP and CPDCCHtotal,slot,μP, respectively.

For allocation of PDCCH candidates and non-overlapping CCEs to search space sets for scheduling on the primary cell from the primary cell, the UE applies the procedure in clause 10.1 using α∙minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ instead of minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ, and using α∙minCPDCCHmax,slot,μ,CPDCCHtotal,slot,μ instead of minCPDCCHmax,slot,μ,CPDCCHtotal,slot,μ for the primary cell.

------------------------------- end TP to 38.213v17.1.0 subclause 10.1.1 ---------------------------------

CR coversheet text (draft provided by Moderator, not RAN1 agreement) for agreed Proposal-TP3v2

Reason for change: For SCell to P(S)Cell scheduling, separate UE capabilities are agreed for disabling scaling factor α for case when SCell is deactivated (<FG 34-3>), and for case when activated SCell is switched to dormant BWP (<FG 34-4>). According to current specification text in 38.213 sub-clause 10.1.1., it is not clear whether scaling factor α is disabled or not when UE indicates different values for support of <FG 34-3> and <FG 34-4>.

Summary of change: Clarify that α =1 is assumed if the UE indicates capability <FG 34-3> and the secondary cell is deactivated, or if the UE indicates capability <FG 34-4> and the active DL BWP of the secondary cell is a dormant DL BWP for the UE.

Consequences if not approved: Unclear UE behavior for PDCCH reception on P(S)Cell when SCell to P(S)Cell scheduling is configured for a UE.

# 4 References

[1] R1-2205583, Summary of Email discussion [109-e-R17_DSS-01], Moderator (Ericsson), RAN1#109-e, May 2022.