3GPP TSG RAN WG1 Meeting #109-e														    R1- 2205588

e-Meeting, May 9th – 20th, 2022

Source:				Moderator (Fraunhofer HHI)

Title:                    FL Summary for AI 9.4.2 - Co-Channel Coexistence for LTE and NR Sidelink

Agenda item:		9.4.2

Document for:	Discussion

# Introduction

The work item for NR sidelink evolution was approved in RAN#94e and revised in RAN#95e [1], and the following objectives were identified in relation to the co-channel coexistence between LTE and NR sidelink:

4. Study and specify, if necessary, mechanism(s) for co-channel coexistence for LTE sidelink and NR sidelink including performance, necessity, feasibility, and potential specification impact if any [RAN1, RAN2, RAN4]

- Reuse the in-device coexistence framework defined in Rel-16 as much as possible

This document provides a summary of the submitted contributions, email discussion topics and outcomes during RAN1#109-e meeting for AI 9.4.2.

[109-e-R18-SL-03] Co-channel coexistence for LTE SL and NR SL by May 20 – Sarun (Fraunhofer)

- Check points: May 16, May 20

# Contact Information

In this section, delegates responding to this email discussion can enter their details, in order to be informed about who is handling each topic. If a company has several delegates handling the same sub-agenda, all delegates can provide their information (and add their topics in bracket).

| Company | Name | Email Address |
| --- | --- | --- |
| Fraunhofer | Tom Wirth | thomas.wirth@hhi.fraunhofer.de |
| NTT DOCOMO | Shohei Yoshioka | shohei.yoshioka@docomo-lab.com |
| Apple | Chunxuan Ye | Chunxuan_ye@apple.com |
| vivo | Siqi Liu | liusiqi@vivo.com |
| Intel | Kilian Roth Salvatore Talarico | kilian.roth@intel.com salvatore.talarico@intel.com |
| Qualcomm | Sourjya Dutta | sourdutt@qti.qualcomm.com |
| Lenovo | Zhennian Sun | sunzn1@lenovo.com |
| Samsung | Emad Farag | e.farag@samsung.com |
| OPPO | Kevin Lin Yi Ding | kevin.lin@oppo.com yi.ding@oppo.com |
| ZTE,Sanechips | Yuzhou Hu | hu.yuzhou@zte.com.cn |
| Transsion | Xingya Shen | xingya.shen@transsion.com |
| InterDigital | Moon-il Lee | Moonil.lee@interdigital.com |
| Spreadtrum | Haowen Liu | haowen.liu@unisoc.com |
| Sharp | Luochao | chao.luo@cn.sharp-world.com |
| Xiaomi | Zhao Qun | zhaoqun1@xiaomi.com |
| NEC | Jin Yang | yangjin@labs.nec.cn |
| Ericsson | Ricardo Blasco | name.surname at company . com |
| Sony | Xiaoxue Wang | Xiaoxue.Wang@sony.com |
| HiSilicon | Fan Yang | james.yangfan@huawei.com |
| Huawei | Xiang Mi | shawn.mixiang@huawei.com |
| Nokia, NSB | Torsten Wildschek | torsten.wildschek@nokia.com |
| Mitsubishi Electric | Cristina Ciochina | c.ciochina at fr.merce.mee.com |
| CATT | ShupengLi | lsp@catt.cn |
| LGE | Seungmin Lee Daesung Hwang | edison.lee@lge.com daesung.hwang@lge.com |
| Toyota ITC | Takayuki Shimizu | takayuki.shimizu@toyota.com |
| China Mobile | Pengyu JI | jipengyu@chinamobile.com |
| ETRI | Junghoon Lee | jh.lee@etri.re.kr |
| Panasonic | Yang KANG | yang.kang@sg.panasonic.com |
| China Telecom | Jing Guo | guojing6@chinatelecom.cn |
| MediaTek | Umut Ugurlu | umut.ugurlu@mediatek.com |
| Bosch | Khaled Hassan | khaled.hassan@bosch.com |
| WILUS | Youngjoon Yoon | eric.yoon@wilusgroup.com |
| CableLabs | Dorin Viorel | d.viorel@cablelabs.com |
| Continental Automotive GmbH | David Gonzalez | david.gonzalez.gonzalez@continental-corporation.com |

# Collection of Agreements/Conclusions in RAN1#109-e

## Agreements from Thursday’s GTW on 12th May

Agreement:

For co-channel coexistence in Rel-18, no changes in the LTE SL specifications are allowed.

Agreement:

For co-channel coexistence in Rel-18, Rel-16/17 simulation assumptions are reused for evaluation of solutions, except for the UE dropping model.

- FFS: UE dropping model

## Agreements from Direct Email Approval on 20th May

Agreement:

For the study of co-channel coexistence solutions in Rel-18, the combination of operational modes Mode 2 NR SL with Mode 4 LTE SL (Combination A) is considered with high priority.

- FFS: Whether/how to support Mode 1 NR SL + Mode 4 LTE SL (Combination B) and/or Mode 2 NR SL + Mode 3 LTE SL (Combination C).

Agreement:

For evaluation of co-channel coexistence solutions in Rel-18, support the inclusion of dual module devices with NR+LTE modules using the following UE dropping models:

- UE Dropping Model A: The distance between 1 LTE SL module and 1 NR SL module are maintained as zero to model a co-located dual module device. The inter-device distance between any two adjacent devices in the same lane, which may be either a single module or a dual module device, is modified by doubling the time in the upper limit, resulting in max{2 meter, an exponential random variable with the average of the speed * 4sec}.

- UE Dropping Model B: The distance between 1 LTE SL module and 1 NR SL module are maintained as zero to model a co-located dual module device. The inter-device distance between any two adjacent devices in the same lane, which may be either a single module or a dual module device, is maintained the same as current assumptions, i.e., max{2 meter, an exponential random variable with the average of the speed * 2sec}.

Companies should mention the UE dropping model and the distribution of each device type (single/dual module) used in their simulation assumptions.

Agreement:

Feasibility of semi-static resource pool partitioning and dynamic resource sharing as possible solutions for co-channel coexistence are to be studied.

Agreement:

For studying the feasibility of dynamic resource sharing as a possible solution for co-channel coexistence,

- For device type A, the NR SL module uses the sensing and resource reservation information shared by the LTE SL module.

  - FFS details on how the NR SL module uses this information.

  - FFS details on how the LTE SL module shares the information to the NR SL module, exact information shared, timeline etc.

- FFS: Whether/how to define other method(s) for device type A to be aware of resources being occupied by LTE SL.

- FFS: Whether/how device type B should be supported.

# Scope of Study

Background: For both LTE SL and NR SL to co-exist within the same frequency channel, it is important to limit the scope of the study based on the inputs of the companies as well as on the permitted time allocated for this topic. This includes the type of devices that are considered – devices that contain both LTE SL and NR SL modules, which was considered in the Rel-16 in-device coexistence topic and devices that contain only NR SL modules. Another aspect to be discussed are the combination of operational modes for which co-channel coexistence have to be considered. It is understood that a combination of Mode 1 NR SL + Mode 3 LTE SL would be handled by the network. Furthermore, restrictions to the LTE SL specifications are also to be discussed since LTE SL has been deployed in different parts of the world already, and changes would not be favorable. This exercise is also to limit the work done within this study, in order to finish the WID in time.

## [ACTIVE] Issue 1-1: Type of Devices to be Considered

### Company Views for 1st Round of Discussions

Question 1-1: Which of the following devices have to be considered for co-channel coexistence? If other device types are to be considered, please describe.

- Device type A: UEs with dual LTE SL and NR SL modules

- Device type B: UEs with only NR SL modules

| Company | Device Type(s) | Comments |
| --- | --- | --- |
| NTT DOCOMO | At least type A | If time is allowed, also type B should be considered to achieve better performance. |
| Apple | Type A > Type B | Type B can also be considered if time allows. |
| vivo | See comments | The function of a Type B UE equipped with only NR modules in the proposal should be clarified first. If type B UE refers to a UE with only R16/17 NR SL modules, then it can neither decode nor transmit LTE SCI and in this case, type B UE can be considered for solution A/B but should not be considered for solution C in question 2-1 is adopted as it cannot identify LTE-NR SL collision. But if it refers to a UE with R18 NR SL modules that has not been discussed, there may be some further enhancements on top of R16/17 NR SL features, for example, R18 NR SL modules might be enhanced to process LTE SCI. This question is also related to the solutions in Question 2-1. If solution A/B is adopted, both UE types can be considered as there is no LTE-NR SL collision. But if solution C is used and NR modules has the same functionality as R16/R17 SL, TypeB should be excluded. |
| Intel | Comment | In our opinion, both devices could be considered, but at least type A must be taken into consideration especially if dynamic solution is considered. In contrast, if only the semi-static solution is considered, there is no need to consider type A and type B would be sufficient. |
| Qualcomm | Device Type A | A Rel-18 NR UE operating in a shared channel with R14/15 LTE UE-s must be capable of communicating with the R14/15 LTE UE-s. This is required to support critical applications currently running over LTE like basic safety messages. For evaluations, there should also be legacy devices with only LTE SL modules to reflect the state of deployments when co-channel coex solutions would be applied. |
| Lenovo | Device Type A | In R16 sidelink it assumed that UE shall equip with both LTE module and NR module，we think that the motivation to support device type B should be clarified further. |
| Samsung | At least Type A | Type B devices can be considered through information sharing from Type A devices. |
| OPPO | Device type A | Whether device type B can detect the SCI of LTE SL ought to be clarified. |
| ZTE, Sanechips | At least device Type A | In our opinion, Rel-16/17 SL would be deployed in some regions, so UEs with only NR SL modules should be also considered in the evaluation of coexistence scenario. |
| Transsion | At least Device Type A | Device Type A is the baseline, and if time is allowed, we can investigate Device Type B. |
| InterDigital | Type A and Type B |  |
| Spreadtrum | Device Type A, acceptable with type B | Considering reuse of the Rel-16 in-device coexistence framework as much as possible, device type A should be supported at least. Device type B is acceptable for us in principle. However, if dynamic resource sharing is supported, how it can obtain the LTE transmission to avoid the conflict needs to be considered carefully. |
| Sharp | See comments | It is not fully clear what “consider” means here. For example, does it mean a co-channel coexistence enhancement in Rel-18, if any, should only be specified for a Device type A UE and not for a Device type B UE? |
| xiaomi | Type A | There are much less standardization/implementation impact for Type A than Type B. |
| NEC | Prefer type B, while type A with low priority | As mentioned by vivo and Intel, device type A is more suitable for solution C while type B is sufficient for solutions A and B. Considering the workload, we prefer to study semi-static solutions with type B first, then if time allowed, further discuss other cases. |
| Ericsson | Baseline for Device Type B + Optimization for Device Type A | The baseline solutions defined in this WI should be applicable for devices with only NR SL modules and optimizations can be considered for devices with LTE and NR SL module. |
| Sony | At least Type A | Type A should be discussed firstly |
| Fraunhofer | Type A and Type B |  |
| Huawei, HiSilicon | Only device type A | Based on the Rel-18 WID (copied as below), Rel-16 in-device coexistence frame work should be reused as much as possible, which means dual LTE SL and NR SL modules are assumed by default. Thus, Device type B is not in the scope of Rel-18 SL. - Study and specify, if necessary, mechanism(s) for co-channel coexistence for LTE sidelink and NR sidelink including performance, necessity, feasibility, and potential specification impact if any [RAN1, RAN2, RAN4] - Reuse the in-device coexistence framework defined in Rel-16 as much as possible. We doubt the existence of type B device in the real deployment especially considering the coexistence with LTE-V in ITS band. To coexist with LTE-V, NR-V needs to receive the basic safety message from LTE-V UE. If a NR UE supports NR-V module only, then it cannot receive such LTE-V safety information and should not be allowed by the regulator. Therefore, we think Rel-16 in-device coexistence framework is the only way forward, in which UE has dual LTE SL and NR SL modules. Only device type A can be considered. |
| Nokia, NSB | At least device Type A | In our view, device type A is essential for co-channel co-existence, and support for device type B could be studied if time allows as it will bring an additional level of flexibility for operators and device vendors. |
| Mitsubishi | At least type A |  |
| CATT/GOHIGH | Both device | We think both should be considered |
| LGE | Comment | From our perspective, Type A shoud be considered but it is not clear what scenarios are targeted for Type B. |
| Toyota ITC | At least Type A | At least Type A should be considered. If time allows, Type B can be considered. |
| Bosch | Type B (with comment) | The base line of this objective (in this WI) should be Type B, where the solution needs to consider no assistance form an LTE device in the module. Type A (best effort): It is also understandable that Rel-16/17/18 device will always have dual stack (LTE, NR), at least for in-device coexistence. However, the cooperation between the devices for resource selection and exclusion is limited for lower complexity. |

### Summary of 1st Round of Discussions

Summary of Preferences

The following is a summary of the preferences marked by companies on the different types of devices that can be considered for co-channel coexistence in Rel-18.

- Companies supporting only Type A: QC, Lenovo, OPPO, xiaomi, HW+HiSi. [5]

- Companies supporting Type A with high priority and Type B with low priority (at least Type A): DCM, Apple, SS, ZTE+Sanechips, Transsion, Spreadtrum, Sony, Nokia, Mitsubishi, LGE, Toyota. [11]

- Companies supporting both Type A and B: IDC, Fraunhofer, CATT. [3]

- Companies supporting Type B with high priority and Type A with low priority: NEC, Ericsson. [2]

- Companies supporting device types based on solutions used (Q2-1): Vivo, Intel. [2]

Summary of Comments

Based on the comments from companies, it is clear that device type A is of higher priority and device type B can be considered if time permits. This is because device type A was considered for the Rel-16 in-device coexistence framework, and should be reused as much as possible according to the WID. Support for device type A is also required in order to support dynamic resource sharing as a solution for co-channel coexistence, where type A devices contain LTE SL modules capable of sharing sensing and resource selection information to the NR SL module.

Regarding device type B, Vivo had raised a concern regarding whether the type B device consists of an existing Rel-16/17 module that cannot decode LTE SCIs, or a new Rel-18 module that is capable of decoding LTE SCIs. In the FL’s opinion, it can be further studied as to whether Rel-18 SL UEs are capable of decoding LTE SCIs in order to support dynamic resource sharing. In other words, for device type B to support dynamic co-channel coexistence, it should be capable to decoding LTE SCIs.

Vivo, Intel and NEC had linked the support of device types to the solutions being considered in Q2-1 for co-channel coexistence. The FL would prefer to check the number of supporting companies depending on the device type to ascertain the best possible solution to cater to co-channel coexistence.

However, NEC and Ericsson prefer to consider device type B as the baseline, with optimizations to be considered for device type A.

In response to Sharp’s concern, the question is intended to determine whether device type A, B or both are to be considered in the study to support co-channel coexistence.

FL’ Recommendation

While only 5 companies support the consideration of device type B, a majority of 19 companies support the consideration of device type A in the study of solutions for co-channel coexistence. Device type B can be considered if time permits, but device type A should be given higher priority.

Proposal 1-1:

- For the study of co-channel coexistence solutions in Rel-18, at least Type A devices that contain both LTE SL and NR SL modules are supported.

  - FFS: Whether to support Type B devices that contain only NR SL modules.

### Company Views for 2nd Round of Discussions

Question 1-1a: Based on the inputs from companies in the previous round, would the following proposal be acceptable to the companies?

Proposal 1-1:

- For the study of co-channel coexistence solutions in Rel-18, at least Type A devices that contain both LTE SL and NR SL modules are supported.

  - FFS: Whether to support Type B devices that contain only NR SL modules.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Intel | Yes | We are Ok with the proposal. |
| Apple | Yes |  |
| ETRI | Yes |  |
| Lenovo | Yes |  |
| Panasonic | Yes |  |
| China Telecom | Yes |  |
| Sony | Yes |  |
| MediaTek | Yes |  |
| CMCC | Yes |  |
| Spreadtrum | Yes |  |
| ZTE, Sanechips | Yes |  |
| OPPO | Yes |  |
| Fraunhofer | Yes |  |
| NEC | Yes | Type B should be considered for the scenarios which low capacity and cost devices are needed. |
| Transsion | Yes |  |
| Samsung | Yes | Regarding the comment from Huawei, HiSilicon that “Device type B is not in the scope of Rel-18 SL.” This is not our reading of the WID, while the WID says “reuse the in-device coexistence framework defined in Rel-16 as much as possible”, this doesn’t preclude solutions on top of in-device coexistence framework, and it doesn’t mandate that the UE has both LTE SL module and NR SL module. |
| Qualcomm | Yes with modification | While we agree with the proposal that at least Type A devices should be supported for cochannel coexistence, we also want the proposal to further capture that LTE-only devices may be present in the system for evaluations. The following re-wording is preferred by us: - For the study of co-channel coexistence solutions in Rel-18, at least Type A devices that contain both LTE SL and NR SL modules and Type C devices that contain only LTE SL module are supported. - FFS: Whether to support Type B devices that contain only NR SL modules. We strongly believe that this clarification will prevent confusion in subsequent meetings. |
| Toyota ITC | Yes |  |
| Bosch | See comments | We prefer Type B asa baseline and Type A with best effort. As we mentioned in our delayed 1st round comment (copied above), it is clear for us that all devices from Rel-16 on should have LTE and NR onboard (at least for V2X applications). This is required for LTE/NR in-device coexistence. However, our concern is if Type B means real time cooperation between the two modules, i.e., including sharing instantaneous resource allocation information. It is important to keep complexity affordable and assume that Type A does not lead to a high complexity design. |
| Nokia, NSB | Yes | Regarding Qualcomm’s proposal to add Type C (only LTE SL) – we think that should be common understanding, so we don’t see it as necessary to state it explicitly, but we aren’t opposed to it either. |
| Xiaomi | Yes |  |
| CATT/GOHIGH | See comments | Does type A ‘UEs with dual LTE SL and NR SL modules’ includes the legacy ‘UEs with dual LTE SL and NR SL modules’ that are not able to update ? |
| vivo | modifications | As there is a clear majority in favor of Type A, we are open to supporting type A for progress. But as we commented in previous round, even type B with legacy NR SL module is sufficient at least for solutions A and B, thus, we think type B should be included at least for semi-static solution. In addition, NR UL with R18 NR module may support LTE SCI decoding, which allows more efficient coordination. therefore, we suggest adding the following note: - For the study of co-channel coexistence solutions in Rel-18, at least Type A devices that contain both LTE SL and NR SL modules module are supported. - For the study of co-channel coexistence solution in Rel-18, Type B devices that contain NR SL modules only module are supported at least if semi-static (i.e., TDM or FDM based) co-channel coexistence solution is supported - FFS: Whether to support Type B devices that contain only NR SL modules if dynamic co-channel coexistence solution is supported. |
| Huawei, HiSilicon | Yes (but remove FFS) | We support the direction of the proposal, but we still think it is not necessary to further discuss Type B devices. The main problem of Type B devices is it cannot receive the message from LTE-V UEs. Especially for the coexistence scenario on ITS band, NR-V UEs shall be capable to receive the safety message from LTE-V UEs. Otherwise, it will cause serious problem (even accident). On the interpretation of the WID “reuses the in-device coexistence framework…as much as possible”. Our understanding is for the real deployment, type B devices cannot support the coexistence with LTE-V devices. Thus, “as much as possible” seems not include any other possibility apart from type A devices. For adding Type C devices commented by QC, we have not seen the necessity to add it. For a LTE SL UE, it would definitely to contain only LTE SL module. For a NR SL UE, there is no situation to support only LTE SL module, because it is NR SL UE. LTE-V only devices will be considered in the evaluations, there is no need to mention it in the proposal to avoid any confusion. Based on the above comment, we modify the proposal as below: Proposal 1-1: - For the study of co-channel coexistence solutions in Rel-18, at least Type A devices that contain both LTE SL and NR SL modules are supported. - FFS: Whether to support Type B devices that contain only NR SL modules. |
| InterDigital | Yes |  |
| DCM | Yes | With keeping the FFS. We would like to emphasize that in 1st round, our position is rather ‘Companies supporting Type A with high priority and Type B with low priority (at least Type A)’. |
| LGE | Yes |  |
| Sharp | Yes |  |
| Ericsson | No | In our view, we should focus on devices with only NR SL module (Type B) since it cannot be assumed that devices with both modules (LTE + NR) will be implemented. Even if the devices implement LTE as well, we should avoid making dependencies between them if possible. |
| WILUS | Yes |  |

### Summary of 2nd Round of Discussions

Summary of Preferences

The following is a summary of the preferences marked by companies on Proposal 1-1.

- Companies supporting the proposal: Intel, Apple, ETRI, Lenovo, Panasonic, China Telecom, Sony, MediaTek, CMCC, Spreadtrum, ZTE+Sanechips, OPPO, Fraunhofer, NEC, Transsion, Samsung, QC, Toyota, Nokia, Xiaomi, HW+HiSi, IDC, DCM, LGE, Sharp, WILUS. [26]

- Companies supporting the inclusion of type B devices: NEC, Samsung, Bosch, Vivo, DCM, Ericsson. [6]

- Companies not supporting the proposal: Bosch, Ericsson. [2]

Summary of Comments

While majority of the companies are in favor of supporting at least type A devices that contain both LTE SL and NR SL modules, there was some discussion and comments on the type B devices.

- Samsung had stated that although the WID states that Rel-16 in-device framework should be reused as much as possible, it does not preclude other solutions on top of the framework.

- Vivo also commented on device type B having the Rel-16/17 NR SL module would be capable of functioning at least for the semi-static co-channel coexistence solutions.

- Ericsson also insists that not all devices can be assumed to be dual module devices.

- On the other hand, HW states that type B devices should not be considered in the study due to their inability to receive LTE SL messages.

Based on the comments, the FL seeks further clarity on type B devices, which is elaborated in Section 4.1.5.

Regarding the comment from QC on the inclusion of type C devices that consist of only LTE SL modules, Nokia and HW have both replied stating that they should be considered for the evaluation of co-channel coexistence solutions. It mirrors the FL’s understanding that while type C devices should be considered in the evaluation, the agreement made in the previous meeting would rule out any further enhancements to these LTE SL devices.

Regarding the comment from Bosch on type A devices, the issues faced in implementing dynamic resource sharing solutions are discussed in Sections 5.4 and 5.5. For device type B, please refer to Section 4.1.5.

Regarding the comment from CATT, it is the FL’s understanding that type A devices would be similar to those from Rel-16, which can support in-device coexistence, as mentioned in the WID.

FL’ Recommendation

26 companies support the proposal and only 2 do not. Hence the following proposal can be considered for an agreement, with further discussions to be continued for type B devices.

Proposal 1-1(I): (For direct email endorsement)

- For the study of co-channel coexistence solutions in Rel-18, at least Type A devices that contain both LTE SL and NR SL modules are supported.

  - FFS: Whether to support Type B devices that contain only NR SL modules.

### Company Views for 3rd Round of Discussions

Based on the feedback from companies regarding clarity on type B devices, it is clear that these devices need to be able to determine the presence of LTE SL transmissions in a shared resource pool. This is not currently supported by Rel-16/17 NR SL devices. Hence there could be 2 flavors of type B devices – one that uses the Rel-16/17 NR modules and one that uses a new Rel-18 NR module.

- Type B1 devices based on Rel-16/17

These devices are based on the in-device coexistence framework. These devices can work with semi-static solutions in a straightforward manner, but can also use IUC messages from other type A devices to work with dynamic resource sharing solutions.

- New Type B2 devices based on Rel-18

These devices should have the ability to decode LTE SCIs, or at least measure the RSSI to detect the presence of LTE transmissions on resources in order to work with dynamic resource sharing solutions.

Question 1-1b: Regarding type B devices, which of the following versions of type B devices are to be considered for co-channel coexistence? Companies are encouraged to mention the flavor of type B device that they intend to study further for co-channel coexistence solutions, and indicate how the device can operate using the options listed.

- Type B1: Devices with Rel-16/17 NR modules that

  - Option 1-1: Work only with semi-static resource sharing solutions.

  - Option 1-2: Receive IUC messages from other type A devices.

  - Option 1-3: others (please describe).

- Type B2: Devices with new Rel-18 NR modules capable of detecting the presence of LTE transmissions by

  - Option 2-1: Decoding LTE SCIs.

  - Option 2-2: RSSI measurements.

  - Option 2-3: others (please describe).

- Option 3: Do not support Type B devices.

| Company | Device Type(s) | Option(s) | Comments |
| --- | --- | --- | --- |
| ZTE | Option 3 |  | We think the device which can be considered for co-channel coexistence with other devices is type A device. But regarding the deployment scenarios, to study and evaluate the co-channel coexistence solutions, both LTE only and NR only devices should be considered as ‘other devices’. That means Type B1 devices should be considered as part of coexistence legacy UEs with Rel-18 enhanced UEs. Not sure whether companies have aligned understanding on that . If so, we can agree to - Type B1: Devices with Rel-16/17 NR modules for evaluation purpose only that |
| Ericsson | Type B1 + Type B2 | All | We support to study devices with Rel-16/17 NR module and new Rel-18 module. For the different options we think that all should be considered at this stage. |
| Panasonic | B1 |  | R16/17 NR would be enough for NR-only modules. |
| NEC | Type B1 with modification | Option 1-1 | First, we do not think it is a common understanding that “it is clear that these devices need to be able to determine the presence of LTE SL transmissions in a shared resource pool.” According to the above definition of type B1, using IUC messages received from type A devices means that type A devices need to merge the information from both LTE module and NR module to generate IUC assistant information, and it needs to modify Rel-17 IUC schemes. Besides, if type B devices work in a dynamic resource sharing scenario, and there is no type A devices nearby, it may cause serious resource collision. Based on that, for the definition of type B1, we propose to modify it as follows: These devices are based on the in-device coexistence framework. These devices can work with semi-static solutions in a straightforward manner, but can also use IUC messages from other type A devices to work with dynamic resource sharing solutions. |
| OPPO | Type B1 | Option 1-1 | For Device Type B1, we think the semi-static resource partitioning can be used to limit the workload and scope. In the meanwhile, we do not support Option 1-2 because it is a partial solution in our view. There is no guarantee that Rel-17 UE can always find a UE A providing the IUC message and the solution of IUC is not supported by Rel-16 UE. For the new Rel-18 UE, we think the study of dual module UE is enough. |
| Xiaomi | Type B1 | Option 1-1 | We do not think the implementation cost of Type B2 is reasonable and thus we do not support to study Type B2 devices. For Type B1 device, to support dynamic resource sharing with LTE UEs, Rel-17 IUC needs to be enhanced to at least support UE to UE scheduling. Considering the standardization complexity, we do not support this option. |
| Qualcomm | Option 3 (leave as FFS) |  | At this point of the study, we do not think the support of Type B devices is a critical issue. As such, both Type B1 and B2 have limitations and complexity issues which will require careful analysis. We support to keep the previous FFS and discuss Type B after an initial design for Type A + LTE Legacy devices is in place. |
| Intel | Type B1 | Option 1-1 | We think that device type B1 is sufficient to study the semi-static resource partitioning, and there is no need to support type B2. |
| Samsung | Type B1 Type B2 | Option 1-2 Option 2-3 | Type B1 devices are Rel-17 NR devices that support IUC Type B2 devices are Rel-18 NR that devices that support IUC. It seems that Option 2-1 could be a special case of Type A devices. |
| Bosch | Type B1 and Type B2 |  | Type B1 devices Rel-16/17 NR devices supporting (at least) Option 1-1. Type B2 devices Rel-18 supporting at least Option 2-2. |
| IDCC | Type B1 | Option 1-2 | We consider Option 1-2 an incremental extension of R17 IUC framework with reasonable work load and clear benefit for Type B1 device. |
| MediaTek | Type B1 | Option 1-1 | We think that semi-static partitioning is sufficient for Type-B devices. |
| Apple | Type B1 | Option 1-1 and 1-2 | For option 1-1, there is no specification effort. For option 1-2, we may only consider R17 NR SL module since IUC is supported in R17 NR SL module. |
| Nokia, NSB | B1, B2 | 1-1, 1-2 (if definition is updated), 2-1, 2-2 | Regarding Type B1: Option 1-1: suffers from the drawbacks of the semi-static approach discussed before. Option 1-2: This would require that the UE Type B1, in its resource selection, always takes IUC information provided by a more advanced UE, e.g. UE Type A or B2, into account; it must not transmit unless it has such IUC information. But we are not aware of any Rel-17 legacy mechanism to impose such a constraint on this UE. So it is not clear how Option 1-2 can work for a Rel-17 UE. On the other hand, one could imagine a Rel-18 UE, which can be (pre)configured with such a constraint. If Option 1-2 is to be supported then it seems that “ Type B1: Devices with Rel-16/17 NR modules” needs to be updated such that Type B1 includes Rel-18 NR modules. |
| ETRI | Type B1 | Option 1-1 and 1-2 | Type B1 will be sufficient, no need to introduce new type since we already defined new type of UE, i.e., Type A. In case of option 1-1, no or little spec impact. In case of option 1-2, since Rel-17 UE supports ICU feature, the functionality can be used for coexistence. |
| Lenovo | Type B1 and Type B2 |  | On Option 1-1 for Type B1 we are wondering if device type A and device type B1 are all existing whether only semi-static resource sharing solutions are applied? Or different resource pools for different type of UEs, e.g., dynamic resource sharing for Type A and semi-static for type B1? |
| Spreadtrum | Type B1 | Option 1-1 and 1-2 | Option 1-1 should be supported at least. For Option 1-2, IUC can help Type B devices to perform resource selection. However, as other companies mentioned, if there is no UE-A that can sense LTE transmission nearby the Type B1(Option 1-2) device, how does this device perform resource selection? This issue should be considered. |
| WILUS | Type B1 Type B2 | Option 1-2 Option 2-2, Option 2-3 | Since Rel-17 devices support IUC, we think Option 1-2 is reasonable. Since LTE SL traffic (CAM, BSM) is assumed to be periodic, we think that Option 2-2 for Type B2 may be reasonable. Note that Type B2 is Rel-18 devices, hence Type B2 devices need to support IUC. We think Type B2 with Option 2-1 is equal to Type A. |
| DCM | B1 | 1-2 | Probably, Option 1-2 intends that UE-A determines preferred/non-preferred resources or collision detection based on LTE-SL signalling. This perspective should be clearly mentioned. Otherwise, we are not sure what is the intention of this question with option 1-2. As rational of option 1-2, if UE-A supports to receive LTE-SL reservation and to use it in resource selection, then why it cannot be used for IUC is quite unclear. Available information should be used as much as possible. |
| Transsion | Type B1 | Option 1-1 | We believe semi-static resource sharing is sufficient for Type B UE |
| Sony | Type B1 | Option 1-1 and 1-2 | Option 1-1 should be supported as no spec impact. Option 1-2 can get resource usage information from other UE(s) to help Type B UE select resource, option 1-2 also should be supported. |
| vivo | Type B2 Type B1 | Option 2-1 Option1-1 | Considering that NR SL devices should have the ability to receive the Basic safety Message from LTE SL devices, Option 2-1 should be considered. As we commented before, option1-1 supports semi-static coexistence. While regarding option 1-2, devices with Rel-16/17 NR modules to receive IUC messages from other type A devices still suffer resource LTE-NR collision because of the latency of IUC procedure. |
| Huawei, HiSilicon | None | Option 3 | We strongly suggest RAN1 should have a reasonable scope for further study. Based on the question 1-1b, type B devices are dived into two sub-types, and each sub-type is further divided into different options. Based on the input from companies, the preferences are quite divergent. So what will RAN1 going to study? To find solution(s) for each options? We do not think it is reasonable or even not possible within limited TU. So we share the similar views from QC that the Type B devices is not critical. Due to absence of LTE-V module, type B devices cannot receive the message from LTE-V. Even considering the IUC message from other NR SL UEs with dual modules, it is not always feasible. Since it cannot be guaranteed that there would be a dual-module UE is within its communication range. Therefore, we do not think Type B devices are applicable for Rel-18 coexistence in ITS band. Considering the inter-UE coordination could not help to resolve all problems brought by absence of LTE-V module, and will also complicate the whole design. In the end, we still think it is not necessary to have further study on Type B devices. |
| CATT/GOHIGH | Type B1 | 1-1 |  |
| LGE | FFS |  | At this moment, the strong motivation for Type B device-related enhancement is unclear. |

### Company Views for 3rd Round of Direct Email Discussions

Based on the inputs collected from companies after the 2nd round of email discussions, and with the question on type B devices separated, the following proposal was discussed directly via email.

Proposal 1-1(I):

- For the study of co-channel coexistence solutions in Rel-18, at least Type A devices that contain both LTE SL and NR SL modules are supported.

  - FFS: Whether to support Type B devices that contain only NR SL modules.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | No | We have strong concerns with the proposal as it is currently formulated. Our understanding of the service deployment model supported by the 3GPP SL specifications is that different services will use different RATs (and potentially different releases/features within a RAT). Within a RAT, coexistence is guaranteed by design by means of having a baseline functionality (reservations, sensing, etc.) even if different UEs participating in different services may not be able to process each other’s transmissions. The point is that, for a given RAT, we do not require every UE to implement every feature to ensure fair coexistence. Focusing only on devices that have both LTE SL and NR SL modules is a big step in the opposite direction. What we are saying is that coexistence between the RATs is only worked out for UEs supporting both technologies, regardless of the services that they want to operated. Our view is that having an additional LTE module just for the sake of improving coexistence is simply not justified from a cost perspective. Note that we have no problem in considering devices that have both SLs, but we think that the case that only NR SL is available has to be considered too. |
| DCM | No | Similar view with Ericsson. At least the FFS point should be maintained in consideration of existence of multiple supporters of type B device. |
| NEC | No | We have a similar view with Ericsson and DCM, the FFS should be restored and type B devices should be considered. |
| Samsung |  | We would like to keep the FFS. As this is still being discussed, we think that it is an accurate reflection of the current situation that Type B devices will be further studied whether to be supported or not. Studying “whether to support Type B devices” doesn’t mean that Type B devices will be supported, this is the subject of the study. |

Since companies were of the view that even if the discussions on type B devices are ongoing, it was accurate to retain the FFS in the proposal. Hence, the FL updated the proposal, and companies were invited to comment on the revised version.

Proposal 1-1(II):

- For the study of co-channel coexistence solutions in Rel-18, at least Type A devices that contain both LTE SL and NR SL modules are supported.

  - FFS: Whether to support Type B devices that contain only NR SL modules.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes with removing the FFS | We still think the FFS is not needed, and the Rel-18 coexistence design should be based on UE with dual LTE SL and NR SL modules, i.e. Type A devices, only. The problem of Type B devices is they cannot receive the information including safety message from LTE-V devices due to the absence of LTE SL module, and how these NR module only UEs support coexistence with LTE-V UEs are doubtful. Therefore, RAN1 should keep a reasonable scope for the further discussion, and we prefer to remove the FFS. Suggested changes are below. Proposal 1-1(II): 1. For the study of co-channel coexistence solutions in Rel-18, at least Type A devices that contain both LTE SL and NR SL modules are supported. FFS: Whether to support Type B devices that contain only NR SL modules. |
| ZTE,Sanechips | Comments | Thank you very much for the update. Our understanding, however, as elaborated in the document is that, type B UE needs to be accounted for in the evaluations while it's questionable whether specific optimization is needed. The Rel-18 NR UE shall be likely to monitor LTE sidelink and impact the overall sensing procedure and if so it may as well simply have two moduals, resorting to type A. Capturing the following would somehow alleviate our concern, though not sure how it feels for others. · For the study of co-channel coexistence solutions in Rel-18, at least Type A devices that contain both LTE SL and NR SL modules are supported. o FFS: Whether to support Type B devices that contain only NR SL modules for evaluation purpose only. |
| Bosch | Comments | We understand that Type A is required, at least, for receiving safety critical messages and in-device coexistence between LTE-V2X and NR-V2X. However, we don’t see the reason and the need that this mandates a baseline solution for R18 coexistence! We believe Type-A only solution will always trigger issues related to the level of coordination between the modules and timeline of readiness/availability of information. We also wonder, in the deployment phase, if this (Type-A) solution will add too many constraints that should be avoid at the moment (especially when there is intra and inter RAT competition and regulatory issues). Therefore, we are proposing to examine the feasibility of Type-B only and whether the complexity (vs. performance) of its solution is acceptable (vs. Type-A solution). Therefore, we propose the following Proposal as a compromise: · For the study of co-channel coexistence solutions in Rel-18, at least Type A devices that contain both LTE SL and NR SL modules are supported. o FFS: Whether Type B devices that contain only NR SL modules can support a feasible, low complexity and good performance coexistence mechanism. |
| Continental Automotive GmbH |  | We support having the FFS as NR-sidelink-only devices would need to monitor LTE-sidelink. Therefore, we believe it is necessary to consider/study type B devices within the co-channel coexistence solution in Rel. 18. We support the proposal and to maintain the FFS as formulated originally, i.e., · For the study of co-channel coexistence solutions in Rel-18, at least Type A devices that contain both LTE SL and NR SL modules are supported. o FFS: Whether to support Type B devices that contain only NR SL modules Additional considerations, such as evaluation purposes or performance/complexity are of course to be considered in the analysis, so the FFS as above would be fine. |
| Vivo | Ok with the proposal including FFS for typeB | As there is an ongoing discussion on type B, we feel it would be a bit early to preclude type B, we support Proposal 1-1(II) with the FFS. Proposal 1-1(II): - For the study of co-channel coexistence solutions in Rel-18, at least Type A devices that contain both LTE SL and NR SL modules are supported. - FFS: Whether to support Type B devices that contain only NR SL modules. |
| DCM | Yes | OK with the proposal including FFS. Same view with vivo. |
| Qualcomm | Yes with comments | We agree to supporting Rel-18 dual devices for the coexistence study and can accept the FFS on Type B device for progress. On the other hand, we would like the proposal to also state clearly that in addition to Rel-18 Type A devices, Rel-14/15 legacy LTE devices must also be considered in the same system for this study. Clarifying this in the early stages of the discussion is needed to avoid confusion stemming from assuming only R-18 dual devices in system evaluations, which will defeat the purpose of this study. We also agree to the edit made by our colleagues from ZTE. Thus, we propose the following edits. - For the study of co-channel coexistence solutions in Rel-18, at least Type A devices that contain both LTE SL and NR SL modules and legacy devices that contain only R-14/15 LTE SL module are supported. FFS: Whether to support Type B devices that contain only NR SL modules for evaluation purposes only. |
| NEC | Yes | Agree with the proposal including FFS |
| Transsion | Yes | We are fine with the proposal. |
| CMCC | OK | Whether Type B UE can be supported depends on the feasibility and impact on spec complexity for monitoring the SCI from LTE-V UE, at this initial stage, we think it is OK to keep it as FFS. |
| Apple | Yes | Due to the ongoing discussions of supporting Type B devices in FL summary, we are fine to keep the FFS in this proposal since they are related. |
| Intel | Yes | We are OK with the proposal and to keep the FFS. |
| Sony | Yes | We are ok with FL’s proposal. |
| CATT/GOHIGH | OK | With the FFS |
| OPPO | Yes | As we commented on the FTP, the Type B UE should focus on the Rel-16/Rel-17 UEs with semi-static partitioning approach. For Rel-18 UE, the study of the dual module UE is enough. |
| Lenovo | Yes | We are fine with FL’s proposal and keep the FFS. |
| LGE | Yes with comment | We are supportive of adding “for evaluation purposes only” in FFS point. |
| Ericsson |  | Our preference is to turn the FFS into a an agreement of its own. Given the situation, we would be fine to keep it as an FFS, but certainly not “for evaluation purposes only” |

### Summary of 3rd Round of Discussions

Summary of Preferences

The following is a summary of the preferences marked by companies on Proposal 1-1(I) and (II).

- Companies supporting to keep the FFS: Ericsson (opposes evaluation only), DCM, NEC, Samsung, Bosch, Continental, Vivo, QC, NEC, Transsion, CMCC, Apple, Intel, Sony, CATT, OPPO, Lenovo. [17]

- Companies supporting type B devices for evaluation only: ZTE+Sanechips, QC, LGE. [3]

- Companies supporting to remove the FFS: HW+HiSi. [1]

The following is a summary of the preferences marked by companies regarding type B devices.

- Companies supporting Type B1: Ericsson (all options), Panasonic, NEC, OPPO, Xiaomi, Intel, Samsung, Bosch, IDC, MediaTek, Apple, Nokia, ETRI, Lenovo, Spreadtrum, WILUS, DCM, Transsion, Sony, Vivo, CATT. [21]

  - Option 1-1: NEC, OPPO, Xiaomi, Intel, Bosch, MediaTek, Apple, Nokia, ETRI, Spreadtrum, Transsion, Sony, Vivo, CATT. [14]

  - Option 1-2: Samsung, IDC, Apple, Nokia (with modification), ETRI, Spreadtrum, WILUS, DCM, Sony. [9]

- Companies supporting Type B2: Ericsson (all options), Samsung, Bosch, Nokia, Lenovo, WILUS, Vivo. [7]

  - Option 2-1: Nokia, Vivo. [2]

  - Option 2-2: Bosch, Nokia, WILUS. [3]

  - Option 2-3 - supporting IUC: Samsung, Nokia. [2]

- Companies supporting Option 3: ZTE+Sanechips, QC, HW+HiSi, LGE. [4]

Summary of Comments

Regarding Proposal 1-1:

With regard to the comment from QC, the intension of the proposal is to determine the type of devices that can be capable of supporting solutions for co-channel coexistence. Since it was agreed that there would be no specification changes to LTE SL, legacy devices containing Rel-14/15 modules cannot carry these new solutions. On the other hand, when evaluating the performance of these solutions, it is the FL’s understanding that LTE SL devices have to be considered within the same system. There should not be any ambiguity in this understanding because the WID clearly states that the solutions cater to co-channel coexistence for LTE SL and NR SL.

A few companies had also stated that type B devices should be considered only for evaluation. As mentioned earlier, the intention of the proposal is to determine the type of devices that can be capable of supporting solutions for co-channel coexistence, and not for only evaluation. For this purpose, type B devices can also be included, and are being discussed.

Regarding Type B Devices:

A few companies had expressed their concern on how IUC messages can assist type B devices to determine resources that are being used by LTE SL transmissions. In the FL’s view, type B devices would require other type A devices that are capable of IUC (Rel-17 NR SL UEs) to assist type B devices. The action of type B devices in the case it does not receive such an IUC message is unclear and can be further studied.

Only a small number of companies are in favor of introducing new features in Rel-18 NR SL type B devices that are capable of being aware of LTE SL transmissions.

FL’ Recommendation

For supporting type A devices, majority of the companies are in favor. The FFS is Proposal 1-1 can be removed if both the proposals are considered together.

Proposal 1-1(II):

- For the study of co-channel coexistence solutions in Rel-18, at least Type A devices that contain both LTE SL and NR SL modules are supported.

  - FFS: Whether to support Type B devices that contain only NR SL modules.

Proposal 1-1b (I):

- For the study of co-channel coexistence solutions in Rel-18, at least Type B devices that contain only NR SL modules based on the Rel-16/17 are supported.

  - These devices support semi-static resource partitioning solutions.

  - FFS: Whether these devices can support dynamic resource sharing solutions using IUC messages.

  - FFS: Other type B devices.

### Company Views for 4th Round of Discussions

Based on the discussions that took place in Wednesday’s GTW on 18 May, Proposal 1-1(II) and Proposal 1-1b (I) were merged and the following was the outcome of the discussion.

Proposal

- For the study of co-channel coexistence solutions in Rel-18, at least Type A are considered in the coexistence with Type C devices of release earlier than Rel-18.

  - These Type A devices can be studied for both semi-static partitioning solutions and dynamic resource sharing solutions.

- FFS: Study the feasibility of Type B devices for coexistence with Type C devices of release earlier than Rel-18, and if feasible study the co-channel coexistence solutions in Rel-18.

  - These Type B devices are studied at least for semi-static resource partitioning solutions.

  - FFS: Whether these devices can support dynamic resource sharing solutions using IUC messages or other types of dynamic resource sharing solutions.

- Note:

  - Type A devices are devices that contain both LTE SL and NR SL modules

  - Type B devices are devices that contain only NR SL modules

  - Type C devices are devices that contain only LTE SL modules based on Rel-14/Rel-15

  - FFS: other types of devices

- Note: the potential Rel-18 solutions should ensure that Rel-18 NR-SL UEs and NR-SL UEs of earlier releases are able to operate in the same NR resource pools, where resource pools can support Type A and Type B devices

Evaluation of Solutions

According to the WID’s justification, the solution should consider the “V2X deployment scenario where both LTE V2X and NR V2X devices are to coexist in the same frequency channel”. The WID, under objectives, also state that “Rel-18 sidelink should be able to coexist with Rel-16/17 sidelink in the same resource pool.” to ensure backward compatibility. In the FL’s view, the WID requires new Rel-18 SL devices to be able to coexist with other Rel-14/15 LTE SL devices, in the same frequency channel, as well as with Rel-16/17 NR SL devices, in the same resource pool. Hence, evaluation of all co-channel coexistence solutions applied to any new Rel-18 SL device should be evaluated to coexist with Rel-14/15 LTE SL devices and Rel-16/17 NR SL devices.

While the Rel-14/15 LTE SL devices are mapped to the type C devices, the Rel-16/17 NR SL devices can be mapped to device type D. Based on these device types, we can have a separate bullet describing that the new Rel-18 SL devices should be able to coexist with type C and type D devices.

Intention of Proposal

From the comments from companies, the FL wants to reiterate that the intention of the proposal is to determine new Rel-18 SL device types that should be considered for the study of co-channel coexistence solutions. Since the WID states that the in-device coexistence framework defined in Rel-16 is to be reused as much as possible, the new Rel-18 SL devices should be based on the Rel-16 dual module devices with both NR SL and LTE SL modules that support in-device coexistence. However, it does not preclude the consideration of new Rel-18 SL devices that contain only NR SL modules, which are now called type B devices.

Feasibility of type B.

The WID states that the solutions should be studied based on their performance, necessity, feasibility and potential specification impact. While type A devices can assume to have lesser specification impact than type B devices, neither have been studied in terms of their performance, necessity or feasibility. In order to maintain fairness between both device types, the text “feasibility of type B devices” can be removed. The FFS for type B is retained since a number of companies had expressed concerns on whether it can be used to apply co-channel coexistence solutions.

Based on the above comments, let us consider the following proposal:

Proposal 1-1 (III)

- For the study of co-channel coexistence solutions in Rel-18, at least device type A is considered.

  - FFS: Whether type B devices are considered.

- For the study of co-channel coexistence solutions in Rel-18, the supported device types coexist with type C, type D and type E devices.

- Note:

  - Type A devices are Rel-18 devices that contain both LTE SL and NR SL modules

  - Type B devices are Rel-18 devices that contain only NR SL modules

  - Type C devices are Rel-14/Rel-15 devices that contain only LTE SL modules

  - Type D devices are Rel-16/17 devices that contain only NR SL modules

  - Type E devices are Rel-16 devices that contain both LTE SL and NR SL modules based on in-device coexistence framework

| Company | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | Comments | We note that the issue of Rel-18 NR SL UE-s inter-working with Rel-16/17 NR SL UE-s may be more accurately captured as backward compatibility. Backward compatibility is captured as general principle in the WID. However, this objective is specifically about NR and LTE coexistence. As part of the design, RAN1 can ensure that Rel-18 devices are backward compatible with older NR SL releases in the same resource pool, which cannot contain LTE SL devices. We prefer to not capture older NR sidelink devices as part of the coexistence study because that would expand the scope beyond what is in the WID and would greatly increase evaluation effort. However, if companies insist on explicitly mentioning backward compatibility, we propose to clearly separate that from coexistence: - For the study of co-channel coexistence solutions in Rel-18, at least device type A is considered. - FFS: Whether type B devices are considered. - For the study of co-channel coexistence solutions in Rel-18, the supported device types coexist with type C devices in the same channel and is backwards compatible with type D and type E devices when they operate over the same resource pool. - Note: - Type A devices are Rel-18 devices that contain both LTE SL and NR SL modules - Type B devices are Rel-18 devices that contain only NR SL modules - Type C devices are Rel-14/Rel-15 devices that contain only LTE SL modules - Type D devices are Rel-16/17 devices that contain only NR SL modules - Type E devices are Rel-16/17 devices that contain both LTE SL and NR SL modules based on in-device coexistence framework |
| CMCC | Comments | We generally share the same views with QC, i.e., we also think coexistence b/w type A and type D/E is out of scope of Rel-18, the sentence below the WID “Rel-18 sidelink should be able to coexist with Rel-16/17 sidelink in the same resource pool” is just a design guideline for NR evaluation in Rel-18, not a specific objective for the coexistence design. Besides, if type D and E devices are involved, we are really worry about the workload, so we think at least they should be FFS or determined as second priority. |
| Apple | Comments | For the second bullet, “the supported device types” should be changed to “the supported Rel-18 device types” to clarify the co-existence is between Type A/B and Type C/D/E. Regarding the coexistence between Type D and Type C, in our view, they could at least co-exist in semi-static resource partitioning solution. |
| InterDigital | Comment | We don’t see strong motivation to include Type-D UE for evaluation and agree with Qualcomm and CMCC that this is out of scope. If companies insist to include Type-D, we will be ok with Qualcomm’s modification |
| ZTE,Sanechips | Support | The FL proposal captures completely and exactly scenarios in need of investigation. Note that the recommended solution as outcome of this study should well accommodate the scenarios in the FL proposal. |
| Xiaomi | Comment | We share the understanding as QC. Backward compatibility issue is different from inter-RAT coexistence issue. R18 NR SL UE can coexist with R16/17 NR SL UE in the same resource pool does not mean that it needs to support co-channel coexistence with R16/17 NR SL UEs and LTE SL UEs altogether. We are fine with QC revision or we can agree on only the first bullet. |
| DCM | Support | Current proposal seems very good to capture the intention clearly. Of course our preference is to include type B with type A, but we can accept with the FFS as compromise. Agree with ZTE. Any device type should be considered in this stage as FL states, and we can discuss further. |
| Spreadtrum | Comment | We share the same understanding with QC and prefer not to take R16/R17 NR UE into account. |
| Panasonic | Support |  |
| Sony | Comments | We support Oualcomm’s modification. And the study of Type D and Type E services can be FFS if time is allowed. |
| Intel | Yes with comments | We are generally OK with the proposal, and share same view as Qualcomm regarding characterizing the co-existence with type D and E devices, since this will expand the scope beyond the WID intention. |
| OPPO | Comments | We share the same view with QC. According to the objective of this WID, we should focus on the co-channel coexistence between Rel-18 NR SL UE and LTE SL UE. When we study and design the Rel-18 SL UE, it is expected that the backward compatibility ought to be guaranteed. Therefore, we propose to delete the Type D and E UE in the second bullet and add a note that the coexistence with Type D and E UE belongs to the issue of backward compatibility |
| Sharp | Comments | On the 2nd bullet: - Agree with other companies that Types D and E should be removed. - Since RAN1 may end up considering only one device type => “…, the supported device type(s) coexist with …” |
| vivo | Comments | The study of Type D and Type E are out of scope, thus should be removed. We agree with OPPO, adding a note that the coexistence with Type D and E UE belongs to the issue of backward compatibility would be sufficient. |
| ETRI | Generally OK with comment | Regarding the second bullet, we share with similar view with other companies. We think the coexistence with Type D and E is out of scope of the WID, and prefer to focus the coexistence between Type A and C. |
| Fraunhofer | Support | We are supportive of the FL’s proposal. We also understand the clarifications in wording provided by QC regarding the coexistence scenarios supported, and the update from Apple. Hence we propose the following: - For the study of co-channel coexistence solutions in Rel-18, at least device type A is considered. - FFS: Whether type B devices are considered. - For the study of co-channel coexistence solutions in Rel-18, the supported Rel-18 device type(s) coexist with type C devices in the same channel. - Note: The supported Rel-18 device type(s) are backward compatible with type D and type E devices when they operate over the same resource pool. - Note: - Type A devices are Rel-18 devices that contain both LTE SL and NR SL modules - Type B devices are Rel-18 devices that contain only NR SL modules - Type C devices are Rel-14/Rel-15 devices that contain only LTE SL modules - Type D devices are Rel-16/17 devices that contain only NR SL modules - Type E devices are Rel-16/17 devices that contain both LTE SL and NR SL modules based on in-device coexistence framework |
| NEC | Yes with comments | We propose to focus on co-existence between type A and type C, and type B and type C, which are main points of the topic. Issues related to type D/E seem to be out of the study scope. |
| Samsung | Comments | Suggest the following updates - I don’t think we need so many device types. It is sufficient to have, Type A (NR+LTE), Type B (NR only) and Type C (LTE only) devices. Whether the devices are NR devices are Rel-16/17/18 we can discuss this later. LTE devices are Rel-14/15 devices. - We think that a resource pool should include Type A and Type B devices. It would be too restrictive to just consider Type A devices. Therefore, we suggest to delete the sub-bullet and add Type B to the main bullet: “For the study of co-channel coexistence solutions in Rel-18, at least device type A and device Type B are is considered”. At this stage we should not limit the scope of the study. The outcome of the study will determine how we move forward. - Suggest to update the second bullet as follows: - For the study of co-channel coexistence solutions in Rel-18, NR resource pool(s) with Type A and Type B devices co-exist with LTE resource pool(s) with Type A and Type C devices |
| Nokia, NSB | Comments | Agree with QC and others that coexistence with type D and E in the same resource pool as type C is not needed for this objective. |
| Lenovo | Comments | We share the same views as QC |
| MediaTek | Comment | Similar view as QC. Type-D and E can be removed, and backward compatibility needs to be ensured as indicated in WID. |
| Huawei, HiSilicon | Comments | First of all, it should be clarified that, based on the WID (copied as below and highlighted), the co-channel coexistence issue is studied only for the case of LTE sidelink and NR sidelink. Furthermore, the WID also requires to reuse the in-device coexistence framework as much as possible which definitely the cosexistence between LTE-V and NR-V. So the coexistence for Type A devices and Rel-16/17 NR SL UEs is not in the scope of co-channel coexistence. - Study and specify, if necessary, mechanism(s) for co-channel coexistence for LTE sidelink and NR sidelink including performance, necessity, feasibility, and potential specification impact if any [RAN1, RAN2, RAN4] - Reuse the in-device coexistence framework defined in Rel-16 as much as possible. However, how to coexist Rel-16/Rel-17 UE is more likely a backward compatibility issue, and the WID has clearly stated as well that “Rel-18 sidelink should be able to coexist with Rel-16/17 sidelink in the same resource pool”. This is a universal principle applied for all the Rel-18 sidelink objectives, not just for coexistence. Thus, we think the general description in the WID is sufficient and it is not necessary to claim it again in a dedicated agreement. Rel-18 sidelink should be able to coexist with Rel-16/17 sidelink in the same resource pool. This does not preclude the possibility of operating Rel-18 sidelink in a dedicated resource pool.. For the FFS related to Type B devices, we keep the views that type B devices would be problematic to coexist with LTE-V devices in ITS band, since it cannot receive the safety message from LTE-V UEs. For progress, we can accept to leave an FFS to further study the feasibility of type B devices in this meeting. As per the definition of each type of UE, we think whether to support dual module is related UE functionality and capability. There is no strong motivation to associate them with specific releases. So we suggest to remove the release number as well. Therefore, some suggestion on the proposal are provided as below. Proposal 1-1 (III) - For the study of co-channel coexistence solutions in Rel-18, at least device type A is considered. - FFS: Whether type B devices are considered. - For the study of co-channel coexistence solutions in Rel-18, the supported considered device type(s) coexist with type C, type D and type E devices. - Note: - Type A devices are Rel-18 devices that contain both LTE SL and NR SL modules - Type B devices are Rel-18 devices that contain only NR SL modules - Type C devices are Rel-14/Rel-15 devices that contain only LTE SL modules - Type D devices are Rel-16/17 devices that contain only NR SL modules - Type E devices are Rel-16 devices that contain both LTE SL and NR SL modules based on in-device coexistence framework |
| CATT3/GOHIGH | Comment | For older NR sidelink devices ,they should be considered for the coexistence study because the WID clearly indicate ‘Rel-18 sidelink should be able to coexist with Rel-16/17 sidelink in the same resource pool.’ Therefore we need to evaluate the impact of new scheme developed |
| Ericsson | No | We have strong concern about keeping type B devices only as FFS. In our view these devices shall be considered and the different solutions to enable co-channel coexistences should be studied. Proposal 1-1 (III) - For the study of co-channel coexistence solutions in Rel-18, at least device type A and type B are considered. - FFS: Whether type B devices are considered. - For the study of co-channel coexistence solutions in Rel-18, the supported device types coexist with type C, type D and type E devices. - Note: - Type A devices are Rel-18 devices that contain both LTE SL and NR SL modules - Type B devices are Rel-18 devices that contain only NR SL modules - Type C devices are Rel-14/Rel-15 devices that contain only LTE SL modules - Type D devices are Rel-16/17 devices that contain only NR SL modules - Type E devices are Rel-16 devices that contain both LTE SL and NR SL modules based on in-device coexistence framework |

### Company Views for 5th Round of Email Discussions

FL’s comment:

For the first bullet, only 2 companies want Type B to be moved to the main bullet. Given that the majority of companies prefer to leave Type B under FFS, the FL urges the companies that it can still be studied and supported in future meetings.

For the second bullet, companies are of the common understanding that Type D and Type E devices coexisting with Type A and Type B devices is out of the scope of the WID. It was included in the objectives to ensure backwards compatibility, which should be done by default.

Proposal 1-1 (IV)

- For the study of co-channel coexistence solutions in Rel-18, at least device type A is considered.

  - FFS: Whether type B devices are considered.

- For the study of co-channel coexistence solutions in Rel-18, the supported considered device type(s) coexist with type C devices in the same channel, type D and type E devices.

- Note:

  - Type A devices are Rel-18 devices that contain both LTE SL and NR SL modules

  - Type B devices are Rel-18 devices that contain only NR SL modules

  - Type C devices are Rel-14/Rel-15 devices that contain only LTE SL modules

  - Type D devices are Rel-16/17 devices that contain only NR SL modules

  - Type E devices are Rel-16 devices that contain both LTE SL and NR SL modules based on in-device coexistence framework

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | No | - We think that it is within the scope of the WID to consider co-existence with NR devices of earlier releases. The WID says “mechanism(s) for co-channel coexistence for LTE sidelink and NR sidelink”. There is nothing in this statement that suggest the NR SL or the LTE SL devices should be Rel-18 devices only. The WID additional says “Rel-18 sidelink should be able to coexist with Rel-16/17 sidelink in the same resource pool”. If the Rel-18 SL devices are within the same resource pool as the Rel-16/17 devices it is natural to consider that the Rel-16/17 devices would need to co-exist with the LTE SL devices. Therefore, the WID requires the co-existence of Rel-16/17 NR SL (in addition to Rel-18 NR SL) with LTE SL devices. - The devices types considered can be NR+LTE SL devices, NR only SL devices and LTE only SL devices. This is irrespective of the release of the device. No need to differentiate device types by release. - Our preference is to include both Type A and Type B in the study. There is no need to be restrictive at this point in time. In addition, the Rel-17/16 NR SL devices that co-exist in the same resource pool and should in turn co-exist with LTE SL devices might not have an LTE SL module, or if they do have an LTE SL module it would not communicate (e.g., sensing information) with the NR SL module of the respective device and hence they behave like NR-only devices. Accordingly, these types of devices should be studied. Our first preference is to remove the FFS for Type B and include in the main bullet (as reflected in the update below). If this is not agreeable by the majority of companies, we can accept keeping the FFS for now. Proposal 1-1 (IV) Suggested Update - For the study of co-channel coexistence solutions in Rel-18, at least device type A and device type B are is considered. - FFS: Whether type B devices are considered. - For the study of co-channel coexistence solutions in Rel-18, the supported considered device type(s) coexist with type C devices in the same channel, type D and type E devices. - Note: Rel-18 device type(s) can operate in the same NR SL resource pool as Rel-16/17 device types. - Note: - Type A devices are Rel-18 devices that contain both LTE SL and NR SL modules - Type B devices are Rel-18 devices that contain only NR SL modules - Type C devices are Rel-14/Rel-15 devices that contain only LTE SL modules - Type D devices are Rel-16/17 devices that contain only NR SL modules - Type E devices are Rel-16 devices that contain both LTE SL and NR SL modules based on in-device coexistence framework |
| Qualcomm | Yes |  |
| Bosch | No | Bosch: The FFS in the current status is not supported from our side. Our request is to consider both device types, Type A and Type B for SL co-channel coexistence. If we need an FFS for Type B as a compromise, then we would be fine with an FFS or “a main bullet” that allows studying the feasibility of Type B devices for co-channel coexistence in Rel-18. Therefore, we recommend changing the FFS to the follows (being also a main bullet): - Study the feasibility of Type B devices for the study of co-channel coexistence solutions in Rel-18 |
| Apple | Comment | When defining the device types (A/B/C), we prefer not to include the release number. Specifically, we do not preclude the co-channel coexistence between R16/17 NR SL UEs in the same channel as LTE SL UEs, especially in the semi-static resource pool partitioning solutions. |
| CATT | No | We still have a concern for Proposal 1-1 (IV) as it does not address the requirement of WID ‘Rel-18 sidelink should be able to coexist with Rel-16/17 sidelink in the same resource pool.’ All of new schemes need to be evaluated with type D/E device to make sure they will not negatively impact those devices. To put it in another way, yes we are developing coexistence solution for Type A ( maybe/FFS type B) devices but type D/E device cannot be ignored during the process. Therefore we need to add sub-bullet : Feasibility of the development solution need to be evaluated with Type D/E devices. |
| ZTE | No | Just would like to echo Shupeng's email below. The subbullet is needed given FL's assessment 'It was included in the objectives to ensure backwards compatibility, which should be done by default. ' Feasibility of the development solution need to be evaluated with Type D/E devices. |
| Intel | Yes |  |
| Panasonic | Yes |  |
| NEC | Yes |  |
| Xiaomi | Yes |  |
| Vivo | Yes | Also ok with the intention of the proposal from ZTE, but prefer to just have a note saying that: backward compatibility with legacy UE should be ensured |
| CMCC | Yes |  |
| Transsion | Yes |  |
| OPPO | Yes |  |

FL’s comment:

For the first bullet, based on the comments from Bosch and Samsung, plus Ericsson from the previous rounds, let us try including type B in the main bullet. We have already tried it as an FFS and tried agreeing to only type A, neither of which were successful.

For the second bullet, companies insist that the point on backward compatibility needs to be included, hence refer to the new note, which has been taken from the WID.

Proposal 1-1 (V)

- For the study of co-channel coexistence solutions in Rel-18, at least device type A and type B are is considered.

  - FFS: Whether type B devices are considered.

- For the study of co-channel coexistence solutions in Rel-18, the supported considered device type(s) coexist with type C devices in the same channel, type D and type E devices.

  - Note: The considered device type(s) are backward compatible with Rel-16/17 devices and coexist when they operate over the same resource pool.

- Note:

  - Type A devices are Rel-18 devices that contain both LTE SL and NR SL modules

  - Type B devices are Rel-18 devices that contain only NR SL modules

  - Type C devices are Rel-14/Rel-15 devices that contain only LTE SL modules

  - Type D devices are Rel-16/17 devices that contain only NR SL modules

  - Type E devices are Rel-16 devices that contain both LTE SL and NR SL modules based on in-device coexistence framework

| Company | Yes/No | Comments |
| --- | --- | --- |
| Continental | Yes |  |
| QC | No | We do not agree with the proposal . The WID clearly states that in-device coexistence framework is to be used as baseline: "Reuse the in-device coexistence framework defined in Rel-16 as much as possible". Hence, Type A devices are what the WID mentions for the coexistence study. We do not support introducing another device type that also has the technical issue of not being able to communicate safety messages with LTE SL devices in the same channel. |
| Huawei, HiSilicon | No | We do not support the proposal. Let us emphasize our concerns again on the Type B devices, which are due to the absence of LTE-V module, they cannot receive the safety message from LTE-V UEs and this is quite dangerous for V2X application in ITS band. We cannot accept to capture Type B devices in the main bullet to claim it is considered, we can compromise to give companies more time to check the feasibility until next meeting and leave it as FFS right now. As the added note, we think the description on the backward compatibility in the WID takes same effect, and moreover, the WID is a high level guidance on the design for whole Rel-18 sidelink objectives, so it is not necessary to iterate again, neither the note is needed. - For the study of co-channel coexistence solutions in Rel-18, at least device type A and type B are is considered. - FFS: Whether type B devices are considered. - For the study of co-channel coexistence solutions in Rel-18, the supported considered device type(s) coexist with type C devices in the same channel, type D and type E devices. - Note: The considered device type(s) are backward compatible with Rel-16/17 devices and coexist when they operate over the same resource pool. - Note: - Type A devices are Rel-18 devices that contain both LTE SL and NR SL modules - Type B devices are Rel-18 devices that contain only NR SL modules - Type C devices are Rel-14/Rel-15 devices that contain only LTE SL modules - Type D devices are Rel-16/17 devices that contain only NR SL modules - Type E devices are Rel-16 devices that contain both LTE SL and NR SL modules based on in-device coexistence framework We really hope RAN1 can make forward on this point, but current proposal we cannot accept. Our concerns on Type B devices can be summarized as following. - Type B devices cannot receive the safety message and would encounter serious problems coexisting with the LTE-V UEs in the actual deployment in ITS band. - Type B devices is not following the WID which requires to reuse the in-device coexistence framework defined in Rel-16 as much as possible. We can compromise to keep type B devices in the FFS for further checking, but we do not accept the current proposal to consider it for study in the main bullet. |
| ZTE | Comment | No strong views as to whether to take into account both devices, but this proposal is quite important at least for those interested in assessing aspects including which direction (semi static only or both) to pursure taking into account the scenarios designated to fulfill. Thus we would prefer to have some agreement on at least the coexistence scenario of type A with others. As suggested in earlier rounds as well as GTW, devices of earlier releases need to be accounted for in bringing up evaluations. Thus we feel previously suggested note as compromise from Shupeng is necessary. If there is no convergence on which devices to be considered coexisting with type A, maybe we can remove everything other than having type A device here. Proposal 1-1 (V)  For the study of co-channel coexistence solutions in Rel-18, at least device type A is considered and type B are is considered. o FFS: Whether type B devices are considered.  For the study of co-channel coexistence solutions in Rel-18, the supported considered device type(s) coexist with type C devices in the same channel, type D and type E devices. o Note: The considered device type(s) are backward compatible with Rel-16/17 devices and coexist when they operate over the same resource pool.  Note: o Type A devices are Rel-18 devices that contain both LTE SL and NR SL modules o Type B devices are Rel-18 devices that contain only NR SL modules o Type C devices are Rel-14/Rel-15 devices that contain only LTE SL modules o Type D devices are Rel-16/17 devices that contain only NR SL modules o Type E devices are Rel-16 devices that contain both LTE SL and NR SL modules based on in-device coexistence framework |
| Nokia, NSB | Yes | Support; we are also OK with the earlier text “FFS: Whether type B devices are considered” Regarding some of the objections to type B: - The WID does not state that Rel-16 IDC framework is the baseline, it just states “Reuse the in-device coexistence framework defined in Rel-16 as much as possible” – in the case of type B, “as much as possible” simply reduces to zero. - Regarding the claim that any ITS station needs to be able to receive LTE SL, because basic safety messages are sent over LTE SL: That is a decision to be made by road authorities, not by RAN1. Moreover, e.g. an RSU may not need to receive basic safety messages in order to offer advanced services over NR SL. |
| Ericsson | Yes |  |
| Mitsubishi | Yes | We are generally supportive of the latest proposals from the feature lead. Concerning the debate of type A vs type B in Proposal 1-1, I don’t think that the presence/absence of a colocalized LTE device should be the point to be debated, but the availability of timely information (e.g. sensing) coming from a colocalized LTE device. The main characteristic of “type B” is that the NR module does not assume availability of information from a LTE device when performing its resource selection (and thus it should be able to do something by itself to detect/avoid the LTE transmission), which is different from the absence of a LTE SL device in the vehicle. Would modifying the type B definition be an agreeable compromise for companies concerned about the absence of a LTE-V BSM module from the vehicle? Type B devices are Rel-18 devices that contain a NR SL module not assuming the availability of information from a colocalized LTE SL module |
| LGE | No | As commented in the GTW session, we have a STRONG concern in considering the many variants of Type B device which have the partial functionalities of LTE SL. As per the relevant objective sentence of “Reuse the in-device coexistence framework defined in Rel-16 as much as possible” in WID, it is clear that RAN1 should conduct the study with Type A device as the baseline (i.e., FFS: Type B device). Note that when designing the in-device coexistence framework in Rel-16, we only consider Type A device. Just for the progress, our suggestion is to update the description of Type B device in NOTE as “Type B devices are Rel-18 devices that contain only NR SL modules without any functionalities of LTE SL”. |
| Samsung | Yes with comments | We are generally fine with this proposal. However, we would like to delete “Rel-18” and “Rel-14/Rel-15” from the definition of the respective Type-A, Type-B, Type-C devices: - Note: - Type A devices are Rel-18 devices that contain both LTE SL and NR SL modules - Type B devices are Rel-18 devices that contain only NR SL modules - Type C devices are Rel-14/Rel-15 devices that contain only LTE SL modules We are also fine with the definition of Type-B devices from Cristina/Mitsubishi, but deleting Rel-18 Type B devices are Rel-18 devices that contain a NR SL module not assuming the availability of information from a colocalized LTE SL module (if any) |

FL’s comment:

For the first bullet, the FL would like to remind companies that the proposal is to study device types A and B over the course of the next 3 meetings, and does not agree to support these device types. Hence the FL would urge companies to accept the first bullet as part of this study, instead of discarding a solution without studying them.

@Yuzhou, for the 2nd bullet, the reason the different device types and bullet on coexistence was brought in was because of a difference in opinion among companies about the WID. To repeat again, According to the WID’s justification, the solution should consider the “V2X deployment scenario where both LTE V2X and NR V2X devices are to coexist in the same frequency channel”. This is reflected in the 2nd bullet. The WID, under objectives, also state that “Rel-18 sidelink should be able to coexist with Rel-16/17 sidelink in the same resource pool.” to ensure backward compatibility. This is reflected in the sub-bullet. Hence, it is the FL’s view that the 2nd bullet and sub-bullet are in-line with the WID, and includes the content of the bullet suggested by Shupeng.

@Cristina, you have pointed out one of the primary differences between type A and type B devices, where type B UEs do not have information about LTE SL transmissions from a collocated LTE SL module. I hope this is the common understanding among companies. We can see responses from others if this is explanation would help.

### Summary of 5th Round of Email Discussions

Due to opposition from some companies to consider type B devices for the study, and some companies insisting that they have to be included, this topic could not be agreed upon.

Regarding the coexistence with other device types, some companies insisted on including the coexistence between Rel-18 and Rel-16/17 devices, and although a note was included, it proved to be not enough.

## [CLOSED] Issue 1-2: Combination of Operational Modes to be Considered

### Company Views for 1st Round of Discussions

Question 1-2: Which of the following mix of operational modes have to be considered for co-channel coexistence? If other combinations are to be considered, please describe.

- Combination A : Mode 2 NR SL + Mode 4 LTE SL

- Combination B : Mode 1 NR SL + Mode 4 LTE SL

- Combination C : Mode 2 NR SL + Mode 3 LTE SL

| Company | Combinations(s) | Comments |
| --- | --- | --- |
| NTT DOCOMO | A/B/C |  |
| Apple | A > B, C | At least Combination A should be considered. Combinations B and C can be considered if time allows. |
| Vivo | A and C(1st priority) B(2nd priority) | If it is entirely relied on NR SL UE to detect the collision between LTE SL and NR SL via LTE SA decoding, there would be no difference between combination A and combination C. If the NR reservation can be detected by the LTE UE, e.g., when the NR SL UE transmits an LTE SA that can be detected by the LTE UE for reservation for future NR transmission, the mode3 UE can report its sensing result to enb, then enb can adjust the following scheduling strategy to avoid the collision. |
| Intel | Combination A | We think we should focus on combination A, which in our view is the more critical scenario, and also since for other combinations many sub-options should be also considered: for example, if the gNB can observe allocations in the SL or not. |
| Qualcomm | Combination A (with comments) | It is our opinion that the most challenging scenario for cochannel coexistence of NR SL and LTE SL is when NR SL operates in Mode 2 and LTE SL operates in Mode 4. This would also be the deployment mode in many regions. Thus, it is our view that RAN 1 focus on this combination and revisit Combinations B and C in subsequent meetings (once the details for Combination A are stable). |
| Lenovo | Combination A | We think combination A should be prioritized to others, for others we think network implementation could avoid the resource collision. |
| Samsung | At least A for dynamic co-channel co-existence. A/B/C for semi-static co-channel co-existence. | A can support through dynamic or semi-static co-channel co-existence. B and C can be supported semi-static co-channel co-existence. Semi-static co-channel co-existence is when difference resources are allocated to NR SL and LTE SL. Dynamic co-channel existence is when the same resource can be used by LTE SL or NR SL and the devices decide dynamically which RAT to use. |
| OPPO | Combination A | In LTE SL, the coexistence of mode 3 and mode 4 was supported. Therefore, if combination B and combination C are considered in this study item, we have to discuss another two combinations (i.e., Mode 1 NR SL+ Mode 3 LTE SL+ Mode 4 LTE SL, Mode 2 NR SL + Mode 3 LTE SL+ Mode 4 LTE SL). If so, the scope of this study item is too large. |
| ZTE, Sanechips | A&B&C | In terms of study scope, the support of combination C may be similar to that as combination A if the enhancements take place only on NR sidelink with some additional consideration on corresponding Uu. But we think all of the combinations A, B and C should be considered. |
| Transsion | Combination A | We believe that Combination A is the most critical case. Regarding Combination B and Combination C, gNB/eNB can schedule the resource to avoid collision between NR SL transmission and LTE SL transmission. |
| InterDigital | Combination A/B | We consider Combination A and Combination C is similar in terms of solution space, because LTE specification and thereby Mode 3 LTE operation shall not be impacted. The solution will target Mode 2 NR SL in both combinations. |
| Spreadtrum | Combination A(1st priority) and C(2nd priority) | Combination A should be considered with 1st priority. Considering that NR UE can actively avoid the conflict with LTE sidelink transmission, there looks no difference between combination A and combination C. |
| Sharp | Combination A | Other combinations are less critical in terms of co-channel coexistence. |
| Xiaomi | A | Combination A needs to be supported to enable OOC operation, and thus needs to be investigated. Combination B and C can be supported by NW implementation without spec. impact. |
| NEC | A | To start the discussion from combination A which may be more broadly deployed. |
| Ericsson | Combination A |  |
| Sony | At least A | Combination A should be discussed firstly |
| Fraunhofer | Combination A and C (1st priority) Combination B (2nd priority) |  |
| Huawei, HiSilicon | Combination A only | The topic of Rel-18 co-existence is to discuss how to support the coexistence between two RAT (NR SL and LTE SL), rather than how to coexist between two RA modes. The combination B and C is more likely to discuss RA schemes of gNB indication and UE autonomously selection on the same resources at same time, which should be avoided. In Rel-15 LTE-V, how to support RA mode-3 and mode-4 are comprehensively discussed, but only limited design specified due to high complexity and limited gain. Moreover, the Rel-16 NR-V does not allow mode-1 and mode-2 in the same resource pool. Therefore, we do not see scenario or benefit to support different RA schemes on the same resources especially for Rel-18 co-existence. RAN1 does not need to spend time discussing combination B and C considering the limited TU. We support Combination A only. |
| Nokia, NSB | Combination A with high priority | Combination A should be treated with high priority as we anticipate this combination to be the most common one. |
| Mitsubishi | A, B, C | A is the most challenging and may be discussed first |
| CATT/GOHIGH | A, B, C | All there need to be considered. FFS piriority. |
| LGE | Commment | The study of co-channal coexistence in Rel-18 can be mpact on Combination A considering its challenge and typicalness of scenario. |
| Toyota ITC | At least A | At least Combination A should be considered. If time allows, Combinations B and C can be considered. |
| Bosch | A | ONLY Combination A according to the WID motivation (coexistence issue is only for ITS band, referring to 5GAA in the WID) |

### Summary of 1st Round of Discussions

Summary of Preferences

The following is a summary of the preferences marked by companies on the mix of operational modes that can be considered for co-channel coexistence in Rel-18.

- Companies that support only Combination A: Intel, Lenovo, OPPO, Transsion, Sharp, Xiaomi, NEC, Ericsson, HW+HiSi, LGE. [10]

- Companies that support Combination A with high priority and Combination B/C with low priority: Apple, QC, Spreadtrum, Sony, Nokia. [5]

- Companies that support Combination A+B+C: DCM, ZTE+Sanechips, IDC, Mitsubishi, CATT. [5]

- Companies that support Combination A+C with high priority and Combination B with low priority: Vivo, Fraunhofer. [2]

- Companies that support Combination A for Solution C (dynamic), and Combination A+B+C for Solution A/B (semi-static): SS. [1]

- Companies that consider Combination A and C similar due to no changes expected in LTE specifications: Vivo, ZTE, IDC, Spreadtrum. [4]

Summary of Comments

Based on the feedback from the companies, since it is expected that there would be no change in the LTE SL specifications, it was mentioned that Combination A and C would essentially be the same. This is because the changes required to support both combinations would be limited to the NR SL module.

On the other hand, other companies state that in Combination B and C, Mode 1 NR SL UEs and Mode 3 LTE SL UEs could use the assistance of the eNB/gNB for scheduling assistance and collision avoidance. Hence they are not a critical combination to be included in the study of co-channel coexistence solutions, and can be deprioritized.

FL’ Recommendation

All the companies that have responded to the email discussion agree that Combination A should be considered for the study of solutions for co-channel coexistence.

Proposal 1-2:

- For the study of co-channel coexistence solutions in Rel-18, the supported combination of operational modes is Mode 2 NR SL with Mode 4 LTE SL (Combination A).

  - FFS: Whether to support Mode 1 NR SL + Mode 4 LTE SL (Combination B) and Mode 2 NR SL + Mode 3 LTE SL (Combination C).

### Company Views for 2nd Round of Discussions

Question 1-2a: Based on the inputs from companies in the previous round, would the following proposal be acceptable to the companies?

Proposal 1-2:

- For the study of co-channel coexistence solutions in Rel-18, the supported combination of operational modes is Mode 2 NR SL with Mode 4 LTE SL (Combination A).

  - FFS: Whether to support Mode 1 NR SL + Mode 4 LTE SL (Combination B) and Mode 2 NR SL + Mode 3 LTE SL (Combination C).

| Company | Yes/No | Comments |
| --- | --- | --- |
| Intel | Yes |  |
| Apple | Yes |  |
| ETRI | Yes |  |
| Lenovo | Yes |  |
| Panasonic | Yes |  |
| China Telecom | Yes |  |
| Sony | Yes |  |
| MediaTek | Yes |  |
| CMCC | Yes |  |
| Spreadtrum | Yes |  |
| ZTE, Sanechips | No | Considering NR sidelink deployment scenarios, all of the combinations should be considered. |
| OPPO | Yes |  |
| Fraunhofer | Yes | While we would also like all combinations to be considered, we are fine with handling Combination A first and are open to considering the other combinations if time permits, as covered in the FFS. |
| NEC | Yes |  |
| Transsion | Yes |  |
| Samsung | Yes |  |
| Qualcomm | Yes |  |
| Toyota ITC | Yes |  |
| Bosch | No (comment) | We do not see any need for the FFS. As we mentioned above, the main intention of this objective in the work item, is to consider NR/LTE coexistence in ITS bands /dedicated spectrum. Therefore, we see no necessity to consider combination which may not have high market value or deployment options. |
| Nokia, NSB | Yes |  |
| Xiaomi | Yes |  |
| CATT/GOHIGH | Yes |  |
| vivo | comment | As summarized by FL, combinations A and C are the same because no changes are expected to LTE spec as well as that a NR SL UE cannot distinguish whether LTE SCI comes from mode 3 UE or mode 4 UE, so a common mechanism is sufficient to handle both combinations A/C. We wonder why combination C is for FFS while combination A is supported. Our concern is that if only combination A is for study, will that restrict the final coexistence solution from being used for combination C, even if that solution is ultimately proved to be applicable for combination C. Perhaps we could just say that study the co-existence of mode 2 NR SL + LTE SL without mentioning the type of RA mode used by LTE SL? Proposal 1-2: - For the study of co-channel coexistence solutions in Rel-18, the supported combination of operational modes is Mode 2 NR SL with Mode 4 LTE SL (Combination A). - FFS: Whether to support Mode 1 NR SL + Mode 4 LTE SL (Combination B) and Mode 2 NR SL + Mode 3 LTE SL (Combination C). |
| Huawei, HiSilicon | Yes (but remove FFS) | The combination B and C seems going to re-discuss whether/how to support two different resource allocation schemes in a same resource pool, like mode 3+mode 4 for LTE SL or mode-1+mode-2 for NR SL. We do not think it is necessary or beneficial for the coexistence between NR SL and LTE SL, thus we suggest to delete the FFS. Proposal 1-2: - For the study of co-channel coexistence solutions in Rel-18, the supported combination of operational modes is Mode 2 NR SL with Mode 4 LTE SL (Combination A). - FFS: Whether to support Mode 1 NR SL + Mode 4 LTE SL (Combination B) and Mode 2 NR SL + Mode 3 LTE SL (Combination C). |
| InterDigital | Yes |  |
| DCM | Yes | With keeping FFS |
| LGE | Yes |  |
| Sharp | Yes |  |
| Ericsson | Yes, without FFS | We think we should only focus on the combination of Mode 2 in NR SL and Mode 4 in LTE SL. Therefore, we propose to delete the FFS from this proposal: Proposal 1-2: - For the study of co-channel coexistence solutions in Rel-18, the supported combination of operational modes is Mode 2 NR SL with Mode 4 LTE SL (Combination A). - FFS: Whether to support Mode 1 NR SL + Mode 4 LTE SL (Combination B) and Mode 2 NR SL + Mode 3 LTE SL (Combination C). |
| WILUS | Yes |  |

### Summary of 2nd Round of Discussions

Summary of Preferences

The following is a summary of the preferences marked by companies on Proposal 1-2.

- Companies supporting the proposal: Intel, Apple, ETRI, Lenovo, Panasonic, China Telecom, Sony, MediaTek, CMCC, Spreadtrum, OPPO, Fraunhofer, NEC, Transsion, Samsung, QC, Toyota, Nokia, Xiaomi, CATT, IDC, DCM, LGE, Sharp, WILUS. [25]

- Companies supporting the proposal but without the FFS: Bosch, HW+HiSi, Ericsson. [3]

- Companies supporting the proposal but wants to merge Combination A and C: Vivo. [1]

- Companies not supporting the proposal: ZTE+Sanechips. [1]

Summary of Comments

Majority of the companies are in favor of supporting the proposal to include the combination of operational modes Mode 2 NR SL and Mode 4 LTE SL for the study of co-channel coexistence solutions, while Combination B and C can be considered as FFS, if time permits.

While ZTE prefers to support all the combinations, the FL would like to urge the company to consider the limited time budget for this topic, which is why the remaining combinations can be considered if there is time left over.

Bosch, HW and Ericsson had stated that the FFS should be removed since the discussions should ideally focus on coexistence in the ITS band, and it would not be beneficial to consider modes assisted by the eNB/gNB.

Vivo on the other hand mentions that Combination A and C are essentially the same as the LTE SL specifications are not to be changed, and are concerned that if the proposal would rule out the support for Combination C even though the solutions could support it. While the FL understands Vivo’s view, there are more companies against even having the other combinations in FFS.

FL’ Recommendation

25 companies support the proposal, while 3 are fine with the main bullet but not the FFS. Only one company opposes the proposal. Hence the following proposal can be considered for an agreement.

Proposal 1-2(I): (For direct email endorsement)

- For the study of co-channel coexistence solutions in Rel-18, the supported combination of operational modes is Mode 2 NR SL with Mode 4 LTE SL (Combination A).

  - FFS: Whether/how to support Mode 1 NR SL + Mode 4 LTE SL (Combination B) and Mode 2 NR SL + Mode 3 LTE SL (Combination C).

### Company Views for 3rd Round of Direct Email Discussions

Based on the inputs collected from companies after the 2nd round of email discussions, the following proposal was discussed directly via email.

Proposal 1-2(I):

- For the study of co-channel coexistence solutions in Rel-18, the supported combination of operational modes is Mode 2 NR SL with Mode 4 LTE SL (Combination A).

  - FFS: Whether/how to support Mode 1 NR SL + Mode 4 LTE SL (Combination B) and Mode 2 NR SL + Mode 3 LTE SL (Combination C).

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE,Sanechips | No | We would still say the FFSed part is more relevant to us with NW control. Given companies have consensus the spec. effort to support semi static operation is limited. May we suggest the following as a compromise. · For the study of co-channel coexistence solutions in Rel-18, the supported combination of operational modes is Mode 2 NR SL with Mode 4 LTE SL (Combination A), Mode 1 NR SL + Mode 4 LTE SL (Combination B) and Mode 2 NR SL + Mode 3 LTE SL (Combination C) at least for semi static solution o FFS: Whether/how to support Mode 1 NR SL + Mode 4 LTE SL (Combination B) and Mode 2 NR SL + Mode 3 LTE SL (Combination C) for dynamic solution |
| Huawei, HiSilicon | Yes with removing the FFS | The FFS seems to study how to support different RA schemes within a resource pool, just like NR mode 1+ NR mode 2 in the same RP. However, such an issue had been discussed in Rel-16 for NR-V, and RAN2 confirms that it is not supported. Conclusion is provided below. RAN2 reconfirms that resource pool sharing between resource pool for mode 1 and mode2 is not allowed in Rel-16. No standard effort to support it in Rel-16. Therefore, we do not see any necessities and benefits to re-discuss this similar issue in Rel-18 coexistence, and suggest to remove the FFS. Proposal 1-2(I): · For the study of co-channel coexistence solutions in Rel-18, the supported combination of operational modes is Mode 2 NR SL with Mode 4 LTE SL (Combination A). o FFS: Whether/how to support Mode 1 NR SL + Mode 4 LTE SL (Combination B) and Mode 2 NR SL + Mode 3 LTE SL (Combination C). |
| Intel | Yes with comments | We agree with Huawei, and believe that combination A is the typical scenario. Therefore, we echo comments above and we do not believe that the FFS is needed. |
| Bosch | Yes, if the FFS is removed | In our understanding, LTE/NR-V2Xcoexistence was only considered to cover dedicated carriers /ITS spectrum coexistence problems (either due to regional regulatory or scarce spectrum in some regions). Other than this, we don’t expect that LTE / NR V2X would coexist between network operated and autonomous modes. Therefore, we would like to remove the FFS. We may also recommend to re-open the discuss of this objective again in TSG RAN#96 to have a clarification on the scope of NR-LTE V2X coexistence. |
| Qualcomm | Yes | We are ok with the FL’s proposal, including the FFS for progress. |
| NEC | Yes | We are fine with the proposal. |
| Samsung | Yes | We are fine with the proposal including the FFS |
| Apple | Yes | We are fine with the proposal. At this stage, we do not preclude the case where NR sidelink and LTE sidelink have different resource allocation modes in the context of co-channel coexistence. |
| Lenovo | Yes | We support FL’s proposal |
| ETRI | Yes | We are fine with FL’s proposal. |
| Transsion | Yes | We are fine with FL’s proposal including the FFS. |
| CATT/GOHIGH | OK | We share some of the view from ZTE but currently the proposal seems the only way forward. |
| Spreadtrum | Yes | We are ok with FL’s proposal. |
| Vivo | Yes | We share a similar view as ZTE, but we can live with FL’s proposal including the FFS for progress. |
| CMCC | Yes without the FFS | We are OK for the main bullet. However, for the FFS, since in Rel-16 NR sidelink we do not support such kind of coexistence b/w different mode in a same resource pool, we think this discussion should not be re-triggered during Rel-18. |
| DCM | Yes | Prefer to keep FFS. |
| Sony | Yes | We are ok with FL’s proposal. |
| OPPO | Yes | We support FL’s proposal and ok with the FFS for progress |
| LGE | Yes | We are fine with FL’s proposal for the progress. |

### Summary of 3rd Round of Discussions

Summary of Preferences

The following is a summary of the preferences marked by companies on Proposal.

- Companies supporting the proposal: QC, NEC, Samsung, Apple, Lenovo, ETRI, Transsion, CATT, Spreadtrum, Vivo, DCM, Sony, OPPO, LGE. [14]

- Companies supporting the proposal without FFS: HW+HiSi, Intel, Bosch, CMCC. [4]

- Companies not supporting the proposal: ZTE+Sanechips. [1]

Summary of Comments

Regarding HW’s comment, the FL understands that Mode 1 and Mode 2 in Rel-16 are configured in different resource pools. However, it does not say anything about sharing resource pools between Mode 1 NR and Mode 4 LTE or Mode 2 NR and Mode 3 LTE. The FL does not see why this cannot be left as FFS, especially since a significant number of companies have shown interest in studying it further.

FL’ Recommendation

Majority of the companies support combination A, and are fine with considering Combination B and C later with second priority.

Proposal 1-2(II):

- For the study of co-channel coexistence solutions in Rel-18, the supported combination of operational modes is Mode 2 NR SL with Mode 4 LTE SL (Combination A).

  - FFS: Whether/how to support Mode 1 NR SL + Mode 4 LTE SL (Combination B) and Mode 2 NR SL + Mode 3 LTE SL (Combination C) if time permits.

### Company Views for 4th Round of Discussions

Based on the inputs from companies in the previous round, would the following proposal be acceptable to the companies?

Proposal 1-2(II):

- For the study of co-channel coexistence solutions in Rel-18, the supported combination of operational modes is Mode 2 NR SL with Mode 4 LTE SL (Combination A).

  - FFS: Whether/how to support Mode 1 NR SL + Mode 4 LTE SL (Combination B) and Mode 2 NR SL + Mode 3 LTE SL (Combination C) if time permits.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | Yes | We agree that Mode 2 NR SL with Mode 4 LTE SL should be the primary focus of the study |
| CMCC | Yes | Mode 2 NR SL with Mode 4 LTE SL shall be regarded as first priority. |
| Apple | Yes | We think the different combinations should not be precluded at this stage in the context of co-channel coexistence. |
| InterDigital | Yes |  |
| ZTE,Sanechips | Comment | We are still not convinced by this proposal compared with our previous proposed compromise. To address the work load concern, we would like to suggest the following, • For the study of co-channel coexistence solutions in Rel-18, the supported combination of operational modes Mode 2 NR SL with Mode 4 LTE SL (Combination A) takes higher priority o FFS: Whether/how to support Mode 1 NR SL + Mode 4 LTE SL (Combination B) and Mode 2 NR SL + Mode 3 LTE SL (Combination C) if time permits. |
| Xiaomi | Yes with comment | We agree the principle but it may be better to say “combination A is considered” rather than it is supported at the current stage |
| DCM | OK |  |
| Spreadtrum | Yes |  |
| Panasonic | Yes |  |
| Sony | Yes |  |
| Intel | Yes | We believe that combination A is the typical scenario for which we should study co-channel coexistence solutions, and we are still not OK with the FFS. We would rather avoid to end up with solution over-optimized to accommodate for a scenarios (Combination B and C), which would not be deployed/used in practice. |
| OPPO | Yes | We think leaving the combination B and C as FFS is already a compromise. |
| Sharp | Yes |  |
| vivo | Yes |  |
| ETRI | Yes |  |
| Fraunhofer | Yes | We are supportive of the FL’s proposal, with the following changes as suggested by ZTE and Xiaomi: - For the study of co-channel coexistence solutions in Rel-18, the supported combination of operational modes is Mode 2 NR SL with Mode 4 LTE SL (Combination A) is considered with high priority. - FFS: Whether/how to support Mode 1 NR SL + Mode 4 LTE SL (Combination B) and Mode 2 NR SL + Mode 3 LTE SL (Combination C) if time permits, are considered with low priority. |
| NEC | Yes |  |
| Samsung | Yes | No need to add “if time permits”. |
| Nokia, NSB | Yes | Combination A is of practical relevance. |
| Lenovo | Yes |  |
| MediaTek | Comment | We’re fine with this proposal with two comments as below: 1) We suggest removing “if time permits”. The decision to include or preclude combination B/C should be made based on the necessity of the use scenario, not based on time permit. 2) We suggest adding “and/or” between combination-B and combination-C. We suggest following revision: - For the study of co-channel coexistence solutions in Rel-18, the supported combination of operational modes is Mode 2 NR SL with Mode 4 LTE SL (Combination A). - FFS: Whether/how to support Mode 1 NR SL + Mode 4 LTE SL (Combination B) and/or Mode 2 NR SL + Mode 3 LTE SL (Combination C) if time permits |
| Huawei, HiSilicon | Yes with removing the FFS | We prefer to study the Mode 2 NR SL with Mode 4 LTE SL (Combination A) only, since it is the most typical scenario for coexistence. For the FFS, the core problem is whether to discuss two resource allocation schemes (network scheduling and UE autonomous selection) in the same resource. As FL nicely replies, mode-1 and mode-2 is not allowed in the same resource pool because of extremely complicated design and huge implementation complexity. The combination B and C have similar problem and even worse due to it relates to another RAT. So based on the comments from companies in the last round, we do not see the reason and motivation to support these two additional combinations, neither see the necessity to have further study on this point. If companies can clarify it clearly, we are open for FFS, but currently we think the FFS should be removed. Proposal 1-2(II): - For the study of co-channel coexistence solutions in Rel-18, the supported combination of operational modes is Mode 2 NR SL with Mode 4 LTE SL (Combination A). - FFS: Whether/how to support Mode 1 NR SL + Mode 4 LTE SL (Combination B) and Mode 2 NR SL + Mode 3 LTE SL (Combination C) if time permits. |
| CATT/GOHIGH | Comments | Prefer to remove ‘if time permits.’ |
| Ericsson | Yes | We think that only Mode 2 and Mode 4 should be considered but we are OK to keep the FFS at this point of the release. |
| Toyota ITC | Yes |  |

### Company Views for 5th Round of Email Discussions

FL’s comment:

Majority of the companies support the proposal, with all companies supporting the main bullet. 2 companies are against the inclusion of the FFS, while one company insists on its inclusion. The addition of “with high priority” implies that the main bullet is the primary focus of the study.

Proposal 1-2(III):

- For the study of co-channel coexistence solutions in Rel-18, the supported combination of operational modes is Mode 2 NR SL with Mode 4 LTE SL (Combination A) is considered with high priority.

  - FFS: Whether/how to support Mode 1 NR SL + Mode 4 LTE SL (Combination B) and/or Mode 2 NR SL + Mode 3 LTE SL (Combination C) if time permits.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | Yes | But would like to delete “with high priority” from the main bullet. Other combinations are FFS, so the “high priority” is with respect to what? |
| Qualcomm | Yes |  |
| Bosch | No to FFS | we don’t support the FFS and we believe it is not within the scope of the WID as, in our understanding, it is an issue of dedicated spectra and ITS spectrum only. We may also need to clarify the scope of co-channel coexistence in Rel.-18 SL Evo in TSG RAN#96 (before getting involved in RAN1 study). As a compromise to proceed this meeting, we suggest removing “/how” from the FFS. FFS: Whether/how to support Mode 1 NR SL + Mode 4 LTE SL (Combination B) and/or Mode 2 NR SL + Mode 3 LTE SL (Combination C) |
| Apple | Yes |  |
| Intel | No to FFS | We do not support the FFS, and we would be OK with the updated text from Bosh |
| Panasonic | Yes |  |
| NEC | Yes |  |
| Xiaomi | Yes |  |
| Vivo | Yes |  |
| CMCC | Yes |  |
| Transsion | Yes |  |
| OPPO | Yes |  |

FL’s comment:

Suggestion from Bosch, and agreeable to Intel, to update FFS has been included.

Proposal 1-2(IV):

- For the study of co-channel coexistence solutions in Rel-18, the supported combination of operational modes is Mode 2 NR SL with Mode 4 LTE SL (Combination A) is considered with high priority.

  - FFS: Whether/how to support Mode 1 NR SL + Mode 4 LTE SL (Combination B) and/or Mode 2 NR SL + Mode 3 LTE SL (Combination C) if time permits.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Continental | Yes |  |
| ZTE | No | Regarding the proposal below, could anyone provide reason on dropping how? Frankly speaking, we don't think it's fair to change a typical wording just to open the door for solutions under a FFSed scenario. Proposal 1-2(IV):  For the study of co-channel coexistence solutions in Rel-18, the combination of operational modes is Mode 2 NR SL with Mode 4 LTE SL (Combination A) is considered with high priority. o FFS: Whether/how to support Mode 1 NR SL + Mode 4 LTE SL (Combination B) and/or Mode 2 NR SL + Mode 3 LTE SL (Combination C) if time permits. |
| Nokia, NSB | Yes | editorial: there is a spurious “is” in the main bullet. |
| Ericsson | Yes | We prefer the previous wording, but we can live with the current wording too. |

FL’s comment:

No concerns regarding the main bullet. At ZTE’s request, the “/how” has been reinstated in the FFS, with the editorial correction from Nokia. The FL hopes Bosch and Intel can accept the “/how”.

Proposal 1-2(V):

- For the study of co-channel coexistence solutions in Rel-18, the supported combination of operational modes is Mode 2 NR SL with Mode 4 LTE SL (Combination A) is considered with high priority.

  - FFS: Whether/how to support Mode 1 NR SL + Mode 4 LTE SL (Combination B) and/or Mode 2 NR SL + Mode 3 LTE SL (Combination C) if time permits.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | Yes |  |

### Summary of 5th Round of Email Discussions

The following version of the proposal was submitted for email approval.

Proposal 1-2(V):

- For the study of co-channel coexistence solutions in Rel-18, the supported combination of operational modes is Mode 2 NR SL with Mode 4 LTE SL (Combination A) is considered with high priority.

  - FFS: Whether/how to support Mode 1 NR SL + Mode 4 LTE SL (Combination B) and/or Mode 2 NR SL + Mode 3 LTE SL (Combination C) if time permits.

## [CLOSED] Issue 1-3: Specification Changes to LTE SL

### Company Views for 1st Round of Discussions

Question 1-3: Should co-channel coexistence bring about any changes to the LTE SL specifications? If yes, please elaborate on the changes required.

| Company | Yes/No | Comments |
| --- | --- | --- |
| NTT DOCOMO | No |  |
| Apple | No |  |
| vivo | No |  |
| Intel | No | Our view is that solutions should be made such that no impact on LTE specification. |
| Qualcomm | No | We do not think there should specification changes for LTE SL. As we demonstrate in our paper, NR SL and LTE SL cochannel coexistence is feasible without any change in specification, devices or (pre)configurations of LTE SL UE-s. |
| Lenovo | No |  |
| Samsung | No | No impact to LTE devices |
| OPPO | No |  |
| ZTE, Sanechips | No | The solutions should not have impact on LTE sidelink and its corresponding Uu specification. |
| Transsion | No |  |
| InterDigital | No | We shall ensure the co-existence with LTE V2X Ues of previous releases |
| Spreadtrum | No |  |
| Sharp | No |  |
| Xiaomi | No |  |
| NEC | No |  |
| Ericsson | No | Changes to the LTE SL specification shall not be performed. |
| Sony | No |  |
| Fraunhofer | No |  |
| Huawei, HiSilicon | No | No spec impact is allowed for LTE-V, since it is already deployed for years. |
| Nokia, NSB | No |  |
| Mitsubishi | No |  |
| CATT | NO |  |
| LGE | Comment | At this moment, it is not clear whether any chages are necessary on LTE specification in terms of mpacting the co-channal coexistence between LTE SL and NR SL. |
| Bosch | Comment | We don’t support any change in LTE lower layers (e.g., phy procedure). However, we may accept manipulating measurements if this does not change specifications. |

### Summary of 1st Round of Discussions

Summary of Comments

Companies have a unanimous view that no specification changes are to be done for LTE SL.

FL’ Recommendation

Proposal 1-3:

- For co-channel coexistence in Rel-18, no changes in the LTE SL specifications are allowed.

## [CLOSED] Issue 1-4: Simulation Assumptions to be Considered

### Company Views for 1st Round of Discussions

Question 1-4: Rel-16/17 simulation assumptions are reused for evaluation of co-channel coexistence solutions. Should any additional assumptions be considered on top of the existing simulation assumptions? If yes, please elaborate.

| Company | Yes/No | Comments |
| --- | --- | --- |
| NTT DOCOMO | No |  |
| Apple | No |  |
| vivo | No |  |
| Intel | No |  |
| Qualcomm | Yes | We are of the opinion that some of the simulation assumptions should be revisited for NR-LTE SL cochannel coexistence since the Rel-16/17 assumption were not designed for co-channel coexistence: - The inter-vehicle separation (currently 2s) should be reconsidered when NR SL – LTE SL dual devices are considered for evaluations. When both NR SL and LTE SL UE-s are in the same vehicle, the current assumption on the in-vehicle separation will over-crowd the limited resources of the system. LTE bands are generally of bandwidth less than on equal to 20 MHz. Given than previous traffic assumptions for NR evaluations assume of bandwidths up to 40 MHz, RAN 1 should revisit assumptions on the NR traffic including packet size, inter-arrival time (or periodicity) etc., as well as latency and range requirements. |
| Lenovo | No |  |
| Samsung | Comment | Simulation assumption for NR SL can follow the Rel-16/17 simulation assumptions. On top of that there should be simulation assumption for LTE SL. |
| OPPO | No | The simulation assumptions and profile for NR V2X and LTE V2X are enough. |
| ZTE, Sanechips | No |  |
| Transsion | No |  |
| Spreadtrum | No |  |
| Sharp | No |  |
| Xiaomi | No |  |
| NEC | No |  |
| Ericsson | No | Rel-16/Rel-17 evaluation assumptions can be reused |
| Fraunhofer | No |  |
| Huawei, HiSilicon | No | Rel-16/17 simulation assumptions are reused. We suggest not to discuss such evaluation methodology issues, which are very time consuming and not so relevant for this objective. If some companies have strong interest to simulate a specific scenario, it can be left to company report. |
| Nokia, NSB | No |  |
| CATT | NO |  |
| LGE | Comment | We think that the existing simulation parameters can be reused as much as possible, and if companies apply any other assumptions for their evaluations, its details can be provided. |
| Bosch | No |  |

### Summary of 1st Round of Discussions

Summary of Comments

One company feels that additional assumptions have to be taken into account, another has commented that the LTE SL simulation assumptions can be considered on top of the Rel-16/17 simulation assumptions. The remaining have stated that the Rel-16/17 simulation assumptions can be reused for Rel-18 co-channel coexistence.

FL’ Recommendation

Based on company views, the Rel-16/17 simulation assumptions can be reused for evaluation of co-channel coexistence solutions. It can be left up to individual companies to simulate using additional considerations.

Proposal 1-4:

- For co-channel coexistence in Rel-18, Rel-16/17 simulation assumptions are reused for evaluation of solutions.

Agreement in GTW

For co-channel coexistence in Rel-18, Rel-16/17 simulation assumptions are reused for evaluation of solutions, except for the UE dropping model.

- FFS: UE dropping model

### Company Views for 2nd Round of Discussions

Based on the discussions that took place in Thursday’s GTW, the following agreement was made:

Agreement:

For co-channel coexistence in Rel-18, Rel-16/17 simulation assumptions are reused for evaluation of solutions, except for the UE dropping model.

- FFS: UE dropping model

According to the comments made by Qualcomm, the current Rel-16/17 simulation assumptions did not take into account dual-mode devices with co-located LTE SL and NR SL modules. Due to this constraint, NR and LTE UEs have to be dropped separately, maintaining a distance of max{2 meter, an exponential random variable with the average of the speed * 2 sec}. This results in overcrowding of devices within the same region, leading to excessive collisions.

In order to resolve this issue, one option would be to modify the inter-device distance by doubling the time in the second variable resulting in max{2 meter, an exponential random variable with the average of the speed * 4 sec}. Another solution would be to maintain the same inter-device distance for 1 LTE SL UE and 1 NR SL UE to mimic an LTE-NR dual module device by co-locating both the LTE SL and NR SL UEs of a dual module device. Other solutions can also be considered.

Question 1-4a: Which of the following solutions can be adopted for the UE dropping model used for the evaluation of co-channel coexistence solutions? If other solutions are to be considered, please describe.

- Option A: Modify the inter-device distance by doubling the time in the upper limit, resulting in max{2 meter, an exponential random variable with the average of the speed * 4 sec}.

- Option B: Maintain the same inter-device distance for 1 LTE SL UE and 1 NR SL UE to mimic a dual module device.

- Option C: Others (please describe).

| Company | Option(s) | Comments |
| --- | --- | --- |
| Intel | Option B |  |
| Apple | Option B |  |
| ZTE, Sanechips | Option B | Please note that Option B is only applied when type A devices are evaluated. |
| OPPO | Option B |  |
| Samsung | Option C | Randomly drop Ues using the Rel-16 rule, i.e., maintaining a distance of max{2 meter, an exponential random variable with the average of the speed * 2 sec}. Randomly decide each UE whether it is an LTE only UE with probability A%, an LTE+NR UE with probability B% or an NR only UE with probability C%. Where, A+B+C = 100. If NR Ues always have an LTE module, C=0. One example can be: A = 50 and B= 50. |
| Qualcomm | Option A | We thank the feature lead for capturing our concern with the current simulation assumptions. For further clarification, we can consider the following two cases as shown in the figures below. In the top figure a 500m section of a highway with single RAT UE-s are shown while in the figure in the bottom pane, the same 500 m section has a mix of single RAT and dual RAT devices. It is straightforward from the above plots to conclude that the single RAT drop has fewer UE-s in the same area compared to the drop with both single and dual devices. (For comparison, there are 37 single RAT UE-s in the first case while there are 19 single RAT UE-s and 18 dual RAT UE in the second case. Thus the second drop has effectively 19 + 2*18 = 55 UE-s in the same region) Due to this densification of devices, we need to consider a system with a higher inter-vehicle separation to avoid very large increases in sidelink traffic density. |
| CATT/GOHIGH | B |  |
| vivo | Option A | Option B introduces more simulation efforts. Currently, LTE UE and NR Ues are dropped independently in the SLS, option B requires modifications to the dropping scheme, i.e., LTE UE and NR UE should be considered jointly so that each LTE UE has a corresponding NR UE with the same inter-device distance. Secondly, even if the inter-device distance of LTE and NR Ues is guaranteed to be the same, the density of Ues would increase, as shown in the figure from Qualcomm, thus leading to a higher congestion level. |
| Huawei, HiSilicon | Option B | Based on the explanation from company supporting option A, the intention of option A is to avoid large density of sidelink traffic. However, it is more like an issue that the total number of dropped Ues as well as what the proportion of LTE-V Ues among all Ues is allocated, rather than a dropping model issue. Considering the limited TU, we prefer to simply reuse existed sidelink dropping model, and the number/proportion of Ues can reported by companies when providing the results. |
| DCM | Option B |  |
| LGE | Comment | First of all, we are not sure how the dropping of dual module device can be implemented by using Option B. In other words, considering that different inter-vehicle distance models are provided by TR 36.885 (for LTE SL) and TR 38.885 (for NR SL), we think that it would be better for RAN1 to decide at least which one is the reference for the dropping of dual module device. |
| Ericsson | Option B |  |

### Summary of 2nd Round of Discussions

Summary of Preferences

The following is a summary of the preferences marked by companies on the different options to adopt the UE dropping model in order to incorporate dual module devices for the evaluation of co-channel coexistence solutions.

- Companies supporting Option B: Intel, Apple, ZTE+Sanechips, OPPO, CATT, HW+HiSi, DCM, Ericsson. [8]

- Companies supporting Option A: QC, Vivo. [2]

- Companies supporting Option C: Samsung. [1]

Summary of Comments

With regards to type A devices, companies are in favor of using the same inter-device distance for 1 LTE SL UE and 1 NR SL UE to mimic a dual module device for including these type of devices in the UE dropping model.

ZTE has reiterated that this UE dropping model is restricted to only Type A devices. This has been included in the updated proposal.

Samsung has suggested an alternative UE drop model, which however does not address the concern raised by Qualcomm in the GTW. The current Rel-16/17 simulation assumptions did not take into account dual-mode devices with co-located LTE SL and NR SL modules. Due to this constraint, NR and LTE UEs have to be dropped separately, maintaining a distance of max{2 meter, an exponential random variable with the average of the speed * 2 sec}, thereby not being able to emulate dual module devices. This results in overcrowding of devices within the same region, leading to excessive collisions.

LGE has raised a concern regarding the choice of either the LTE SL or NR SL dropping model. In the FL’s understanding, the agreement made in the previous GTW makes it clear that the reference evaluation assumptions are taken from Rel-16/17. The additional UE dropping model is to be applied only for the dual module device and is on top of the existing dropping model from Rel-16/17.

FL’ Recommendation

While 8 companies support Option B, only 2 support Option A. Hence the following proposal can be considered for an agreement, with a minor update.

Proposal 1-4b (I):

- For co-channel coexistence in Rel-18, in order to include dual module devices in the UE dropping model for simulation assumptions, the same inter-device distance for 1 LTE SL UE and 1 NR SL UE are maintained to mimic a dual module device. This is applicable only for dual module (Type A) devices.

### Company Views for 3rd Round of Discussions

Question 1-4b: Based on the inputs from companies in the previous round, would the following proposal be acceptable to the companies?

Proposal 1-4b (I):

- For co-channel coexistence in Rel-18, in order to include dual module devices in the UE dropping model for simulation assumptions, the same inter-device distance for 1 LTE SL UE and 1 NR SL UE are maintained to mimic a dual module device. This is applicable only for dual module (Type A) devices.

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE,Sanechips | OK |  |
| Ericsson | Yes |  |
| OPPO | Comments | Our initial understanding is that the inter-device distance for 1 LTE SL UE and 1 NR SL UE is same as the simulation assumption in Rel-16. However, after checking the comments from other companies, we realize the intention of this proposal is to consider two adjacent Ues as one dual module UE and the distance of these two Ues maintains the same value during the simulation. Is it the correct understanding? If yes, we think the distance cannot be maintained due to the moving of Ues, the wrap around model and UE turns left/right at the crossing in Urban scenario. Therefore, we begin to have the same concern for the extra simulation efforts brought by this proposal. Furthermore, if the dual module devices only have the NR traffic (e.g. the LTE module is only used for sensing), the overcrowding will not take place. Hence, this proposal should be applicable only for dual module devices with both NR and LTE traffic. |
| Qualcomm | No | Maintaining the same inter-device distance logic for dual mode devices will induce an artificial sparsity in the system. For instance, take the example of the NR highway drop model. For vehicles travelling at 140 km/hr (on average), the distribution of the inter-device distances is shown below: That is there is a > 10% chance that the NR and LTE modules of a Type-A R18 device will be separated by more than 50 m and a > 5% chance that they are separated by 100 m. This will lead to extremely incorrect system level performance as devices separated by 10-s or 100-s of meters may view the channel quite differently. Moreover, the problem will be greatly exacerbated in the urban scenario where the difference in distance will lead to UE-s (simulated to be on the same vehicle) to have very different (e.g., LTE has a NloS channel to a second UE while NR views the channel as LoS). We would also like to address the point raised by Huawei, HiSilicon in the last round of the email discussion. Changing the number of devices based on the current inter-device distance assumption will simply mean that fewer devices are clustered in a smaller region. That is, the device density will be the same (and thus the interference issue) and such a simulation will only consider a smaller segment of the network. Thus, we reiterate that using a change in the inter-vehicle separation is the only viable option to induce sparsity in the device drop without compromising the realistic nature of the simulations. |
| Intel | Yes with comment | We believe that the first thing to converge on is the type of Ues that will be considered in this study. If only type A is considered, then we support the proposal since both LTE and NR module will be collocate, and we do not see any reason to make any changes in the UE’s dropping model. However, if RAN1 intention is to additionally support type B, comments from QC seem to be valid. |
| Samsung |  | This model doubles the number of devices per unit area (there will be LTE and NR devices). The dropping of LTE and NR devices would need to be considered jointly as some devices would have NR+LTE module and should be located at the same position. |
| Nokia, NSB | Question | What does “same inter-device distance for 1 LTE SL UE and 1 NR SL UE are maintained to mimic a dual module device” mean exactly? Reading previous comments here it seems to us that there are different interpretations. |
| DCM | Yes | Similar view with Intel. |
| vivo | no | we think Qualcomm’s comment is valid. The channel condition of a LTE UE and its corresponding SL UE with same inter-device distance could be different if following option B. |
| Huawei, HiSilicon | Clarifications are needed | It is not very clear the meaning or the intention of this proposal. If the understanding is as the explanation from OPPO that to “consider two adjacent Ues as one dual module UE and the distance of these two Ues maintains the same value during the simulation”, we think it is not necessary, since a dual modules UE is just one single UE and two modules are at same location without any distance. If the proposal is going to clarify the dual modules UE should keep same distance with LTE-V UE as that with other NR UE with dual modules, we think it is also hardly to be guaranteed, since the dropping of UE is randomized within a range. As the comment from QC, if RAN1 clarifies that the dual modules of a NR device are at same location, is there still any UE density issue? Therefore, to simply the discussion and simulation, we prefer the proposal provided by FL in the first round, i.e. Proposal 1-4: - For co-channel coexistence in Rel-18, Rel-16/17 simulation assumptions are reused for evaluation of solutions. |
| CATT/GOHIGH | OK |  |
| LGE | Comment | For the simplicity, we suggest to go with the following. Define a reference dropping model (e.g., by selecting one of the two models defined in TS 37.885 and TS 36.885) which can be used for all the evaluation scenarios. Note that for the dual module device, it is assumed that LTE SL module and NR SL module are in the same location. |

### Summary of 3rd Round of Discussions

Summary of Preferences

The following is a summary of the preferences marked by companies on Proposal 1-4b (I).

- Companies supporting the proposal: ZTE+Sanechips, Ericsson, Intel, DCM, CATT. [5]

- Companies not supporting the proposal: OPPO, QC, Vivo, HW+HiSi. [5]

- Companies supporting dual module devices with NR+LTE modules dropped in the same location (co-located): OPPO, Intel, Samsung, DCM, LGE. [5]

Summary of Comments

Based on the comments from companies, there seems to be some ambiguity in a device maintaining the same inter-device distance for 1 LTE SL UE and 1 NR SL UE. The objective of this option was to co-locate an LTE SL module and an NR SL module to mimic a dual module device.

Samsung had mentioned the issue with the number of devices per unit area if both LTE SL and NR SL devices are not considered jointly. The intention is to consider them jointly and in the same location.

Repeating the same comment from the last round, LGE has raised a concern regarding the choice of either the LTE SL or NR SL dropping model. In the FL’s understanding, the agreement made in the previous GTW makes it clear that the reference evaluation assumptions are taken from Rel-16/17. The additional UE dropping model is to be applied only for the dual module device and is on top of the existing dropping model from Rel-16/17.

FL’ Recommendation

Given that there seems to be a split in which UE dropping model to use among companies, the FL suggests to agree on both, assuming that both would ensure the dropping of dual module devices with co-located NR SL and LTE SL modules.

Proposal 1-4b (II):

- For evaluation of co-channel coexistence solutions in Rel-18, support the inclusion of dual module devices with NR+LTE modules using the following UE dropping models:

  - UE Dropping Model A: The inter-device distance is modified by doubling the time in the upper limit, resulting in max{2 meter, an exponential random variable with the average of the speed * 4sec}.

  - UE Dropping Model B: The same inter-device distance for 1 LTE SL UE and 1 NR SL UE are maintained as to model a co-located dual module device.

- Companies should mention the UE dropping model used in their simulation assumptions.

### Company Views for 4th Round of Discussions

Based on the inputs from companies in the previous round, would the following proposal be acceptable to the companies?

Proposal 1-4b (II):

- For evaluation of co-channel coexistence solutions in Rel-18, support the inclusion of dual module devices with NR+LTE modules using the following UE dropping models:

  - UE Dropping Model A: The inter-device distance is modified by doubling the time in the upper limit, resulting in max{2 meter, an exponential random variable with the average of the speed * 4sec}.

  - UE Dropping Model B: The same inter-device distance for 1 LTE SL UE and 1 NR SL UE are maintained as to model a co-located dual module device.

- Companies should mention the UE dropping model used in their simulation assumptions.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | Comment | We are still not clear about Option B. Based on the feature lead’s explanation, the NR SL and LTE SL modules are co-located, but then it is not clear how this co-location is maintained if distances are considered separately for LTE SL UE and NR SL UE. We propose to explicitly capture the existing distance assumption and that it is between vehicles, which could be dual RAT or single RAT. - For evaluation of co-channel coexistence solutions in Rel-18, support the inclusion of dual module devices with NR+LTE modules using the following UE dropping models: - UE Dropping Model A: The inter-device distance is modified by doubling the time in the upper limit, resulting in max{2 meter, an exponential random variable with the average of the speed * 4sec}. - UE Dropping Model B: The same inter-device distance for 1 LTE SL UE and 1 NR SL UE are maintained as to model a co-located dual module device. The inter-device distance between any two devices, which may be either a single RAT or a dual RAT device, is maintained the same as current assumptions, i.e., max{2 meter, an exponential random variable with the average of the speed * 2sec}. - Companies should mention the UE dropping model used in their simulation assumptions. |
| ZTE,Sanechips | Yes |  |
| DCM | Yes |  |
| Intel | Yes | We are OK with the FL’s proposal |
| OPPO | Comments | We are still not convinced by Option B. In the simulation, the UE will move on the road and turn left/right at the crossing. In highway scenario, the UE may enter the scenario from one end again when it moves out of the scenario at the other end. Hence, we are not sure how to maintain the distance between 1 LTE SL UE and NR SL UE. In addition, the channel state of two UEs may be different especially in urban scenario as commented by other companies. As for the overcrowding issue proposed by QC, we can accept the Option A with less simulation efforts as a compromise. |
| vivo | comments | As pointed out by companies, channel condition of LTE UE and its corresponding NR UE could be different if following option B. But for progress, we are ok with proposal from Qualcomm |
| Fraunhofer | Yes, with comments | We are fine with the current proposal. In our understanding, the inter-device distance that is calculated based on the current inter-device distance formula (max{2 meter, an exponential random variable with the average of the speed * 2sec}). In order to mimic a dual RAT device alone, 1 LTE SL module and 1 NR SL module are dropped onto the same location, by maintaining the same inter-device distance from a reference device to each of these LTE SL and NR SL modules. The inter-device distance between such dual module devices and other devices (LTE-only, NR-only, dual module) is determined based on the existing formula, as stated by QC. If Model B needs further clarification, we can add the details as proposed by QC, and modify to: Dropping Model B: The same inter-device distance for 1 LTE SL UE and 1 NR SL UE are maintained as to model a co-located dual module device. The inter-device distance between any two devices, which may be either a single module or a dual module device, is maintained the same as current assumption, i.e., max{2 meter, an exponential random variable with the average of the speed * 2sec}. |
| Samsung | Yes | OK for progress |
| Nokia, NSB | Comment | As per our previous comment, it does not seem clear what “same inter-device distance for 1 LTE SL UE and 1 NR SL UE are maintained” means. We prefer QC’s proposed wording. |
| Huawei, HiSilicon | Seem comments | Our understanding for a dual module UE is a single UE with the capability to decode SCI from LTE-V UE (via its LTE-V module) and SCI from NR-V UE (via its NR-V module), so there would be no distance between its two modules, and no impact on the dropping model. That is why we prefer to simply reuse the dropping model of Rel-16/17. Based on FL’s explanation, the intention of this proposal is to co-locate LTE-V module and NR-V module to mimic a dual module device, which also means the distance between these two modules are zero. Thus we think it is better to explain the intention more clearly in the proposal. Some suggested changes are provided as below. Proposal 1-4b (II): - For evaluation of co-channel coexistence solutions in Rel-18, support the inclusion of dual module devices with NR+LTE modules using the following UE dropping models: - UE Dropping Model A: The inter-device distance is modified by doubling the time in the upper limit, resulting in max{2 meter, an exponential random variable with the average of the speed * 4sec}. - UE Dropping Model B: The same inter-device distance for 1 LTE SL UE module and 1 NR SL UE module within a NR SL UE are maintained as zero to model a co-located dual module device. - Companies should mention the UE dropping model used in their simulation assumptions. |
| Ericsson | Yes | We do not see the need for Model B. Companies are free to bring additional evaluations if they consider it appropriate. |

### Company Views for 5th Round of Email Discussions

FL’s comment:

The FL has taken the comments provided by QC and HW into account and reflected in the updated proposal.

Proposal 1-4b (III):

- For evaluation of co-channel coexistence solutions in Rel-18, support the inclusion of dual module devices with NR+LTE modules using the following UE dropping models:

  - UE Dropping Model A: The inter-device distance is modified by doubling the time in the upper limit, resulting in max{2 meter, an exponential random variable with the average of the speed * 4sec}.

  - UE Dropping Model B: The same inter-device distance for between 1 LTE SL UE module and 1 NR SL UE module are maintained as zero to model a co-located dual module device. The inter-device distance between any two devices, which may be either a single module or a dual module device, is maintained the same as current assumptions, i.e., max{2 meter, an exponential random variable with the average of the speed * 2sec}.

- Companies should mention the UE dropping model used in their simulation assumptions.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | No | We think that UE dropping model B is incomplete as described. The proposal says: “The inter-device distance between any two devices, which may be either a single module or a dual module device” What determines if a device has a single module or dual modules. Is this done randomly, for example, each device based on a probability can be a single module or a dual module? If so, this should be mentioned. |
| Qualcomm | Yes | We thank the FL for considering and accepting the changes we suggested. We agree with the overall proposal and the current definition for UE Dropping Model B. For the sake of clarity, we believe the wording for Model A should also contain the same wording as model B except about the inter-device distance. Hence, we propose the following minor edit to the FL’s proposal. However, if that is clear to all companies, then we are ok to not change the text. - For evaluation of co-channel coexistence solutions in Rel-18, support the inclusion of dual module devices with NR+LTE modules using the following UE dropping models: - UE Dropping Model A: The distance between 1 LTE SL module and 1 NR SL module are maintained as zero to model a co-located dual module device. The inter-device distance between any two devices, which may be either a single module or a dual module device, The inter-device distance is modified by doubling the time in the upper limit, resulting in max{2 meter, an exponential random variable with the average of the speed * 4sec}. - UE Dropping Model B: The distance between 1 LTE SL module and 1 NR SL module are maintained as zero to model a co-located dual module device. The inter-device distance between any two devices, which may be either a single module or a dual module device, is maintained the same as current assumptions, i.e., max{2 meter, an exponential random variable with the average of the speed * 2sec}. - Companies should mention the UE dropping model used in their simulation assumptions. |
| Bosch | Yes |  |
| Apple | Yes |  |
| Intel | Yes |  |
| Panasonic | Yes |  |
| NEC | Yes |  |
| Xiaomi | Yes |  |
| Vivo | Yes | Also ok with further clarification from Qualcomm |
| CMCC | Yes |  |
| Transsion | Yes |  |
| OPPO | Yes | We are ok with this proposal for progress and we think the description of “any two UEs” is not accurate because the inter-device distance is used to restrict the distance between two adjacent UEs in the same lane in TR37.885, so we suggest to use “any two adjacent devices in the same lane” for clarification. Meanwhile, we are also ok with the clarification from QC.  For evaluation of co-channel coexistence solutions in Rel-18, support the inclusion of dual module devices with NR+LTE modules using the following UE dropping models: o UE Dropping Model A: The inter-device distance is modified by doubling the time in the upper limit, resulting in max{2 meter, an exponential random variable with the average of the speed * 4sec}. o UE Dropping Model B: The same inter-device distance for between 1 LTE SL UE module and 1 NR SL UE module are maintained as zero to model a co-located dual module device. The inter-device distance between any two adjacent devices in the same lane, which may be either a single module or a dual module device, is maintained the same as current assumptions, i.e., max{2 meter, an exponential random variable with the average of the speed * 2sec}.  Companies should mention the UE dropping model used in their simulation assumptions. |

FL’s comment:

The FL has taken the comments provided by QC and OPPO into account and reflected in the updated proposal. Regarding Samsung’s comment, the distribution of each device type can be mentioned by the companies in their simulation assumptions.

Proposal 1-4b (IV):

- For evaluation of co-channel coexistence solutions in Rel-18, support the inclusion of dual module devices with NR+LTE modules using the following UE dropping models:

  - UE Dropping Model A: The distance between 1 LTE SL module and 1 NR SL module are maintained as zero to model a co-located dual module device. The inter-device distance between any two devices, which may be either a single module or a dual module device, The inter-device distance is modified by doubling the time in the upper limit, resulting in max{2 meter, an exponential random variable with the average of the speed * 4sec}.

  - UE Dropping Model B: The same inter-device distance for between 1 LTE SL UE module and 1 NR SL UE module are maintained as zero to model a co-located dual module device. The inter-device distance between any two adjacent devices in the same lane, which may be either a single module or a dual module device, is maintained the same as current assumptions, i.e., max{2 meter, an exponential random variable with the average of the speed * 2sec}.

- Companies should mention the UE dropping model used in their simulation assumptions.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Continental | Yes |  |
| QC | Yes | We suggest a minor edit to make the definition of Model A and Model B consistent and avoid future confusion: - For evaluation of co-channel coexistence solutions in Rel-18, support the inclusion of dual module devices with NR+LTE modules using the following UE dropping models: - UE Dropping Model A: The distance between 1 LTE SL module and 1 NR SL module are maintained as zero to model a co-located dual module device. The inter-device distance between any two adjacent devices in the same lane, which may be either a single module or a dual module device, The inter-device distance is modified by doubling the time in the upper limit, resulting in max{2 meter, an exponential random variable with the average of the speed * 4sec}. - UE Dropping Model B: The same inter-device distance for between 1 LTE SL UE module and 1 NR SL UE module are maintained as zero to model a co-located dual module device. The inter-device distance between any two adjacent devices in the same lane, which may be either a single module or a dual module device, is maintained the same as current assumptions, i.e., max{2 meter, an exponential random variable with the average of the speed * 2sec}. - Companies should mention the UE dropping model used in their simulation assumptions. |
| Nokia, NSB | Yes | prefer the wording proposed by QC to harmonize description of the models by adding “in the same lane” to Model A |
| Ericsson | Comment | We do not see the need for this, but we will not object if there is wide consensus to support both models. |

FL’s comment:

The minor updates from QC have been added. No serious concerns have been raised after Samsung’s comment, which the FL hopes has been addressed in previous emails.

Proposal 1-4b (V):

- For evaluation of co-channel coexistence solutions in Rel-18, support the inclusion of dual module devices with NR+LTE modules using the following UE dropping models:

  - UE Dropping Model A: The distance between 1 LTE SL module and 1 NR SL module are maintained as zero to model a co-located dual module device. The inter-device distance between any two adjacent devices in the same lane, which may be either a single module or a dual module device, The inter-device distance is modified by doubling the time in the upper limit, resulting in max{2 meter, an exponential random variable with the average of the speed * 4sec}.

  - UE Dropping Model B: The same inter-device distance for between 1 LTE SL UE module and 1 NR SL UE module are maintained as zero to model a co-located dual module device. The inter-device distance between any two adjacent devices in the same lane, which may be either a single module or a dual module device, is maintained the same as current assumptions, i.e., max{2 meter, an exponential random variable with the average of the speed * 2sec}.

- Companies should mention the UE dropping model used in their simulation assumptions.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | Yes | Following your reply to our question, we would like to add: - Companies to report on distribution of each device type |

### Summary of 5th Round of Email Discussions

The following version of the proposal was submitted for email approval.

Proposal 1-4b (VI):

- For evaluation of co-channel coexistence solutions in Rel-18, support the inclusion of dual module devices with NR+LTE modules using the following UE dropping models:

  - UE Dropping Model A: The distance between 1 LTE SL module and 1 NR SL module are maintained as zero to model a co-located dual module device. The inter-device distance between any two adjacent devices in the same lane, which may be either a single module or a dual module device, The inter-device distance is modified by doubling the time in the upper limit, resulting in max{2 meter, an exponential random variable with the average of the speed * 4sec}.

  - UE Dropping Model B: The same inter-device distance for between 1 LTE SL UE module and 1 NR SL UE module are maintained as zero to model a co-located dual module device. The inter-device distance between any two adjacent devices in the same lane, which may be either a single module or a dual module device, is maintained the same as current assumptions, i.e., max{2 meter, an exponential random variable with the average of the speed * 2sec}.

- Companies should mention the UE dropping model and the distribution of each device type (single/dual module) used in their simulation assumptions.

# Co-Channel Coexistence Solutions to be Considered

Background: In the 5GAA contribution to the 3GPP Rel-18 workshop, 5GAA had listed co-channel coexistence as #2 in their list of topics to be considered for Rel-18. Over the course of the discussions for the scope of the WID during the workshop and subsequent plenary meetings, the solutions considered were semi-static and dynamic co-channel coexistence. Based on the inputs from the contributions, 2 flavors of semi-static co-channel coexistence were expressed by companies, while dynamic co-channel coexistence was also discussed.

While semi-static solutions that use separate resource pools for LTE SL and NR SL in a TDM or FDM manner are possible within the current specifications, issues in using them for co-channel coexistence have been identified by companies. On the other hand, dynamic solutions are not currently specified and would require some restrictions to be considered for a timely completion of the WID. Hence the semi-static and dynamic co-existence solutions have to be discussed and explored.

## [CLOSED] Issue 2-1: Semi-Static and Dynamic Co-channel Coexistence

### Company Views for 1st Round of Discussions

Question 2-1: Which of the following solutions should be considered for co-channel coexistence? If other solutions are to be considered, please describe.

- Solution A: TDM based co-channel coexistence – Separate resource pools for LTE SL and NR SL in the same channel or band, configured over different time slots.

- Solution B: FDM based co-channel coexistence – Separate resource pools for LTE SL and NR SL in the same time slots, configured over different channels or bands.

- Solution C: Dynamic co-channel coexistence – Same resource pool for LTE SL and NR SL, sharing resources across time and frequency.

| Company | Solution(s) | Comments |
| --- | --- | --- |
| NTT DOCOMO | C | A and B are already considered in Rel-16/17 SL basically, and they have an issue on less resource efficiency. For this topic, C is the intended situation in our understanding. |
| Apple | C | Solutions A and B do not achieve the spectrum efficiency as Solution C. |
| vivo | Solution A and Solution B | Semi-static solution can achieve co-channel coexistence with no spec impact. What’s more, our simulation in R1-2203563 shows that Solution C causes both performance loss to LTE devices and NR devices compared with semi-static solutions. |
| Intel | Solution A + B | At least A and B should be considered, and C should be further studied to assess whether this may bring any benefits compared to A and B. |
| Qualcomm | Solution C | Dynamically sharing the same resource pool by both NR SL and LTE SL is the only way that can support cochannel coexistence under all possible deployment scenarios. For this we make the following observations: - Static TDM/FDM resource pool separation will either require network (NW) support or require legacy (LTE SL) devices to be updated. Firstly, many regions consider NW support of SL UE-s as optional, hence UE-s not implementing this feature may not be configured with the resource pool partition. Secondly, SL services should be available without NW coverage and hence reconfiguring RPs through NW control may be infeasible in these scenarios. Lastly, performing a factory update of legacy LTE UE-s may also be challenging or infeasible. - Semi-static TDM/FDM resource pool partition is not adaptive to changes in traffic and network. Changes in network and traffic may be affected by the introduction of new NR UE-s and the phasing out of older LTE UE-s over time; the UE moving from an area with high LTE penetration to an area with high NR penetration; changes in the channel (LoS to NloS) may also change the UE-s perception of the network/traffic. - Mismatch of the pool partition between the various SL UE-s in the network will be another cause of performance degradation for solution A and B. Mismatch in the RP partition will lead to different UE-s having different understanding of the logical slot index. This, while affecting both systems, will be especially deleterious for the performance of feedback-based NR transmissions. HARQ based retransmissions is a fundamental feature of NR SL. Having a semi-static FDM resource pool with NR PSFCH resource may be detrimental to the performance of LTE SL UE-s which will experience different reception power in the middle of a sub-frame leading to decoding errors. |
| Lenovo | Solution C | Considering the resource usage efficiency, we think solution C should be supported |
| Samsung | C | A and B can already be supported using Rel-16/17 mechanisms. |
| OPPO | Solution A and C | As for solution C, we think it should be discussed with some assumptions or restrictions (e.g. the same SCS between LTE and NR SL) |
| ZTE, Sanechips | At least solution A and B | Considering legacy Rel-16/17 sidelink could be on the same carrier with LTE sidelink and Rel-18 NR sidelink, at least solution A and B can be supported without much work load. Solution C should be further studied to assess the benefits and spec. impact compared to A and B. |
| Transsion | Solution C | Supporting solution A and solution B means updating the (pre-)configuration parameters of the legacy LTE SL UE, which is not expected. |
| InterDigital | Solution C | We think solution A and solution B can be considered as exiting solutions, which could be done by NW implementation. So, we don’t see a need to further study them in R18. The study of coexistence should strive for evaluating Solution C in a timely fashion to draw a conclusion. |
| Spreadtrum | Solution A and B at least | Solution A and B could be the baseline for studying the co-channel coexistence of LTE sidelink and NR sidelink. Regarding solution C, the benefits need to be further evaluated and determine whether this solution should be supported. Besides, the case of partial overlapping resource pool should also be taken into account in solution C. |
| Sharp | Solutions A, B, and C | In our view, in the study phase, RAN1 should study the pros and cons of all possible solutions, and it would be next step to consider whether any solution(s) should be adopted. |
| Xiaomi | A,B and C | From our understanding, all the solutions are included in the study phase. It is still FFS which enhancement would be necessary to be specified in R18. We agree with other company that A and B have already been supported in R16/17. But they still need to be considered at least for performance comparison purpose. |
| NEC | A and B | Separated resource pools for LTE SL and NR SL may introduce less impact on specification and require limited workload, and we prefer to discuss solutions A and B first. |
| Ericsson | All | We are fine to consider any potential solution, although it is unclear how static A or B can do much beyond what is possible in Rel-16 |
| Sony | Solution C | Solution C can solve the resource usage efficiency issue caused by solution A and B. |
| Fraunhofer | Solution C | Our understanding is that while Solutions A and B are possible with the existing specifications, their issues regarding the AGC impact and PSFCH usage are clear. Hence we prefer to study Solution C. |
| Huawei, HiSilicon | Current spec already supports Solution A and B | Current specifications already support TDMed and FDMed co-channel co-existence for LTE sidelink and NR sidelink via resource pool separation between the two RATs. Which manner is applied can be up to implementation. Solution C has many technical issues. LTE-V’s performance might be impacted by NR-V due to different SCS and PSFCH resources are configured for NR-V. Specifically, larger SCS than 15kHz supported by NR-V would result in more than one NR SL transmissions within one LTE-V subframe and possibly make AGC of LTE-V lose effectiveness. On the other hand, if PSFCH are configured on the shared resources, it may cause collision between LTE-V transmission and NR-V HARQ. In addition, our simulation results show that solution A/B/C have similar PRR performance. In summary, we suggest the following conclusion to conclude that solution A/B are already supported by current specification, and so far it is not justified to specify Solution C. Conclusion: RAN1 concludes that current specifications already support co-channel co-existence for LTE sidelink and NR sidelink via resource pool separation between the two RATs. |
| Nokia, NSB | C | In our understanding, Solution A is already supported by specification and could be deployed, provided that preconfiguration can be updated (which is doubtful). Both Solution A and Solution B suffer from the problem that update of preconfiguration is problematic and from the inefficiencies of semi-static resource sharing. |
| Mitsubishi | C, A, B | C should be supported, and further investigate how. It should be confirmed if A and B are already fully supported in the current specs or if any enhancement is needed |
| CATT/GOHIGH | A+B | C FFS |
| LGE | At least C | We are not sure what aspects (especially that have an mpact on the specification) should be further studied for Solution A and B. |
| Toyota ITC | C | Solution C should be considered for flexible deployment and efficient use of resources. |
| Bosch | Comments | A + B; However, C FFS: strive for a low complexity solution which can be reused in R16/17 (if needed). |

### Summary of 1st Round of Discussions

Summary of Preferences

The following is a summary of the preferences marked by companies on the different solutions that can be considered for co-channel coexistence in Rel-18.

- Companies that support only Solution C: Apple, QC, Lenovo, Transsion, Sony, Nokia, Toyota. [7]

- Companies that support Solution C, with the understanding that Solutions A and B are already supported: DCM, SS, IDC, Fraunhofer, LGE. [5]

- Companies that support Solution A+B+C: Sharp, Xiaomi, Ericsson, Mitsubishi. [4]

- Companies that support Solution A+B: Vivo, NEC, HW+HiSi, CATT. [4]

- Companies that support Solution A+B with high priority, Solution C with low priority: Intel, ZTE+Sanechips, Spreadtrum. [3]

- Companies that support Solution A+C (with restrictions): OPPO. [1]

Summary of Comments

Based on the comments from companies, it is clear that configuring of separate resource pools in a TDM or FDM manner is already possible based on the Rel-16/17 specifications, and would have no specification impact, as mentioned by Vivo, ZTE+Sanechips, NEC, HW+HiSi. However, using this semi-static resource sharing as a solution for co-channel coexistence has been identified to be challenging. The following issues have been identified by companies:

- Low resource usage and spectrum efficiency – DCM, Apple, Lenovo, Sony

- NW support for legacy LTE and NR UEs is optional and factory updating is challenging, making it difficult to update resource pool preconfigurations – QC, Transsion, Nokia

- Not adaptive to changes in traffic and NW, especially when phasing out older LTE UEs for NR UEs – QC

- Mismatch of pool partitioning between LTE and NR UEs – QC

- FDM resource pools with NR PSFCH will cause a performance drop due to different reception power in the middle of a sub-frame leading to decoding errors – QC, Fraunhofer

- AGC impact – Fraunhofer

On the other hand, dynamic resource sharing is a new solution that requires specification effort. The following issues have been identified by companies:

- Possible impact on LTE SL performance due to difference in SCS and presence of PSFCH configured for NR SL – HW+HiSi

- Causes loss of AGC effectiveness if a larger SCS (>15kHz) is used for NR SL – HW+HiSi

- Causes performance loss to LTE and NR devices – Vivo

- Needs to be further evaluated – Intel, Spreadtrum

It has to be noted that a few of the issues identified for FDM based resource sharing and dynamic resource sharing are common, namely the mismatching of SCS and possible restriction of the shared resource pool to only 15kHz, the impact of SL PSFCH used in NR SL and the AGC issues.

FL’ Recommendation

While 17 companies support the use of dynamic resource sharing as a solution for co-channel coexistence, 11 companies prefer using semi-static resource sharing. Since there is no clear majority, both solutions can be considered in the study for co-channel coexistence at this point.

Proposal 2-1:

- Semi-static and dynamic resource sharing are considered as possible solutions for co-channel coexistence.

  - For semi-static resource sharing, it is already possible to configure separate resource pools for LTE NR and SL NR based on Rel-16/17 specifications.

### Company Views for 2nd Round of Discussions

Question 2-1a: Based on the inputs from companies in the previous round, would the following proposal be acceptable to the companies?

Proposal 2-1:

- Semi-static and dynamic resource sharing are considered as possible solutions for co-channel coexistence.

  - For semi-static resource sharing, it is already possible to configure separate resource pools for LTE NR and SL NR based on Rel-16/17 specifications.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Intel | Yes |  |
| Apple | Yes | For clarification, we could change “separate resource pools” to “non-overlapping (in time-and-frequency) resource pools” |
| ETRI | Yes |  |
| Lenovo | Yes |  |
| Panasonic | Yes |  |
| China Telecom | Yes | We are fine with FL proposal. |
| Sony | Yes |  |
| MediaTek | Yes |  |
| CMCC | Yes |  |
| Spreadtrum | Yes |  |
| ZTE, Sanechips | Yes |  |
| OPPO | Yes |  |
| Fraunhofer | Yes |  |
| NEC | Yes |  |
| Transsion | Yes |  |
| Samsung | Yes |  |
| Qualcomm | Yes with comments | We reiterate our position that semi-static resource pool partition cannot be a general solution for NR SL and LTE SL coexistence. As we pointed out in our earlier replies, semi-static resource partion has the following severe drawbacks: - The signalling of the resource pool partition may not be possible in may regions for SL as NW support is considered an optional feature - Configuration and re-configuration of the resource pool partition will not be possible for the Mode 2 NR SL + Mode 4 LTE SL scenario due to the absence of gNB/eNB - Moreover, while the NW can update the parameters through signalling, the precise parameter to update is not up to the NW to decide. It is determined by standards bodies over a large area (e.g., country) in slow time scales (over years). Given the traffic load condition of LTE V2X/NR V2X can vary over finer geographical scale (e.g., urban-rural) and time scale (e.g., months) semistatic solution based on RRC configuration is not suitable. For example, if a new application for NR V2X is rolling out, with RRC Config. Report, the only option is to wait for the standards body to update the RRC param specification, then roll out at once over the whole county. This is slow and risky. Thus we propose the following modification to the proposal - Dynamic resource sharing is considered as possible general solution for co-channel coexistence. - Semi-static resource pool partitioning may be considered where it is possible to change the (pre)configuration. - For semi-static resource sharing, it is already possible to configure separate resource pools for LTE NR and SL NR based on Rel-16/17 specifications. Per our understanding the semi-static approaches discussed do not share resources at all but FDM/TDM the available spectrum into non-overlapping portions. Hence, we prefere not to use the term “semi-static resource sharing”. |
| Toyota ITC | Yes |  |
| Bosch | Yes (with comments) | We are aligned with Apple’s comment to replace separate with “non-overlapping resources”. Additionally, we need to agree on the complexity of dynamic coexistence solutions, as we need to agree on the complexity. |
| Nokia, NSB | Yes |  |
| Xiaomi | Yes |  |
| CATT/GOHIGH | See comments | This despends on the overall progress of WID. I believe originally this objective of WID is targeted only for two meetings. If we consider both semi-static and dynamic scheme we need to re-adjust the overall plan for the wid. |
| Vivo | Yes | For QC’s comment 2/3, we believe that for OoC Ues and IC Ues, reconfiguration can be updated via the V1 interface, which can provide fast updates to accommodate the time-varying traffic loads. While for the example ‘For example, if a new application for NR V2X is rolling out, with RRC Config. Report, the only option is to wait for the standards body to update the RRC param specification, then roll out at once over the whole county.’ Wouldn’t this also be a problem for dynamic solutions? If the existing RRC parameters are not able to carry new V2X applications, the spec will need to be updated any way regardless if a static-static or dynamic solution is used. Proposal 2-1: - Semi-static and dynamic resource sharing are considered as possible solutions for co-channel coexistence. - For semi-static resource sharing, it is already possible supported to configure separate resource pools for LTE NR and SL NR based on Rel-16/17 specifications. |
| Huawei, HiSilicon | See comments | Our preference is whether to support dynamic resource sharing should be further studied, but to performance loss caused by the AGC issue and collision with PFSCH. For progress, we can accept is can be a “possible solution” in this meeting. For the semi-static resource sharing, our contribution R1-2203148, section 2.1.1 has cleary explained how the (pre-)configuration of resource pools are updated and we copied it as below. In short, the configurations can be updated by RRC or SIB12, and pre-configuration can be update by the reconfiguration on UIUC. - The IE of SL-ResourcePool is used to configure the parameters of NR sidelink resource pool. For the vehicles in coverage and RRC_CONNECTED state, the IE can be updated by SL-ConfigDedicatedNR within RRCReconfiguration[4]. For the UEs in coverage but in RRC_INACTIVE or RRC_IDLE state, it can be further updated by SIB12 within SystemInformation [4]. For LTE-V, SL-CommResourcePoolV2X is the corresponding IE used to configure the parameters of LTE-V resource pool. For the vehicles in coverage and RRC_CONNECTED state, the IE can be updated by SL-CommConfig within RRCConnectionReconfiguration[5]. For the UEs in coverage but in RRC_IDLE state, it can be further updated by SystemInformationBlockType21 within SystemInformation [5]. - Pre-configuration can be also updated by over-the-air signaling. In 5GS (5G System), the parameters for V2X communications over PC5 are specified in Clause 5.1 in TS 23.287 [6], where the V2X parameters can be re-configured for both ME (Mobile Equipment) and UICC (Universal Integrated Circuit Card). For the parameters within ME, they cannot be modified later. However, the pre-configurations in UICC could be updated through the signaling received from the operator and those information in UICC are prioritized than those within ME [6]. Thus, the pre-configuration of resource pools in NR sidelink and LTE-V can be updated. In our understanding, for a dedicated region, the proportion of NR-V Ues over LTE-V Ues would not change sharply. Maybe the traffic for a UE is dynamic, but the total NR-V/LTE-V traffic for this region which associated with the proportion NR-V over LTE-V would vary gradually. Thus, the semi-static update on resource pools are sufficient. To reflect the current situation on the support of semi-static resource sharing, we suggest following changes on the proposal. Proposal 2-1: - Semi-static and Ddynamic resource sharing isare considered as a possible solutions for co-channel coexistence. - For the solution of semi-static resource sharing for co-channel coexistence, it is already possible allowed to configure separate resource pools for LTE NR and SL NR based on Rel-16/17 specifications. |
| InterDigital | Yes |  |
| DCM | Yes |  |
| LGE | Comment | The key point here is that at this moment it is not clear whether even the semi-static solution is really feasible (e.g., considering the regional regulation/policy already finalized, the difficulty of adaptively updating the (pre)configuration) in terms of supporting the co-channel coexistence for LTE SL and NR SL. So, we think that the sub-bullet should be removed for now before the sufficient investigation is to be done, which makes the semi-static solution misinterpreted as already feasible. Also in the main-bullet, with the similar reason, the wording of “can” should be added for the clarity of intention (e.g., semi-static and dynamic resource sharing can be considered as possible solutions for co-channel coexistence). |
| Sharp | Yes |  |
| Ericsson | Yes |  |
| WILUS | Yes |  |

### Summary of 2nd Round of Discussions

Summary of Preferences

The following is a summary of the preferences marked by companies on Proposal 2-1.

- Companies supporting the proposal: Intel, Apple, ETRI, Lenovo, Panasonic, China Telecom, Sony, MediaTek, CMCC, Spreadtrum, ZTE+Sanechips, OPPO, Fraunhofer, NEC, Transsion, Samsung, QC, Toyota, Bosch, Nokia, Xiaomi, Vivo, IDC, DCM, Sharp, Ericsson, Wilus. [27]

- Companies not supporting the proposal: CATT, HW+HiSi, LGE. [3]

Summary of Comments

Majority of the companies are in favor of supporting the proposal to study both semi-static and dynamic resource sharing as possible solutions for co-channel coexistence.

Apple had suggested to clarify that the separate resource pools used for semi-static resource sharing between LTE SL and NR SL are non-overlapping in time and frequency. This has been included in the revised proposal.

In QC’s comment, they have stressed on the challenges faced for updating semi-static resource configurations in a timely manner. LGE also has similar concerns against semi-static solutions. This topic was discussed in Q2-2 and the understanding among companies is that while it is theoretically possible to carry out the updates, its feasibility in carrying out the updates is questionable and not in the scope of RAN1’s discussion.

Regarding the comment from Bosch, the challenges faced in implementing dynamic resource sharing solutions are discussed in Sections 5.4 and 5.5.

Vivo and HW respond to QC’s concerns of updating semi-static resource configurations, and also points out how they can be updated. While the FL understands HW’s concerns against dynamic resource sharing solutions, both solutions have to be studied to identify their pros and cons.

In response to CATT’s comment, in the FL’s opinion, both the solutions have to be studied first as possible solutions for co-channel coexistence. According to the work plan submitted by the rapporteurs of the WI, the aim is to complete the AI by RAN1#111, which gives us 4 meetings, including the current one. This should give us adequate time to identify the benefits and potential issues of each solution, and work towards specifying them.

FL’ Recommendation

Since 27 companies support the proposal and only 3 are against it, consider the following proposal for an agreement, with minor updates as suggested by companies in the comments.

Proposal 2-1(I):

- Feasibility of semi-static and dynamic resource sharing are considered to be studied as possible solutions for co-channel coexistence.

  - For semi-static resource sharing, it is already possible supported to configure separate non-overlapping (in time and frequency) resource pools for LTE NR and SL NR based on Rel-16/17 specifications.

### Company Views for 3rd Round of Discussions

Question 2-1b: Based on the inputs from companies in the previous round, would the following proposal be acceptable to the companies?

Proposal 2-1(I):

- Feasibility of semi-static and dynamic resource sharing are considered to be studied as possible solutions for co-channel coexistence.

  - For semi-static resource sharing, it is already possible supported to configure separate non-overlapping (in time and frequency) resource pools for LTE NR and SL NR based on Rel-16/17 specifications.

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE,Sanechips | Comments | We don’t think it can be claimed supported given for FDMed case, there is AGC issue (actually the system may not work) due to the mismatch of LTE/NR channel structure and for TDMed case there is latency issue. We would prefer to capture the revert to possible wording and capture the FFS point below FFS: the potential spec. impact for TDM/FDM based semi-static resource sharing. |
| Ericsson | Yes | We propose to modify the wording of the proposal in order to have more clarity. Regarding the sub-bullet, why do we need it? It does not add anything it just states something already known. We propose to delete it. We propose to update the proposal as follows: Proposal 2-1(I): - Feasibility of semi-static and dynamic resource sharing as possible solutions for co-channel coexistence to be studied. |
| Panasonic | Yes |  |
| NEC | Yes with comments | The sub-bullet should be removed as the semi-static scheme also needs further discussion to determine whether the legacy resource pool configuration is sufficient and whether potential enhancement schemes are required for co-existence. |
| OPPO | Yes |  |
| xiaomi | Comment | We think at least the feasibility of TDMed semi-static resource sharing does not need to be studied. In addition, on the subbullet, we think it should be “non-overlapping (in time and/or frequency)”. |
| CableLabs | Yes with Comments | Remove the bullet, since it looks to be redundant. |
| Qualcomm | Yes with edit | We agree with the current wording as it stresses the need to assess the feasibility of both dynamic resource sharing and semi-static resource pool partition. Based on the comment made both by Apple and ourselves, we would like to stress that the use of the term “semi-static resource sharing” is incorrect. In a non-overlapping RP partition, there is no resource sharing as captured in the sub bullet. Also, the resource pool partition is not supported in all regions. We thank vivo and Huawei for the comments on how to update the (pre-)configuration. However, those replies are related to the signalling and mechanism, not how such an update would take place in practice. In many regions, there is no requirement to support configuration over Uu and such a scheme is not considered for deployments. Over the air updates of pre-configuration are not universally supported, in fact few vehicles support over the air update of any firmware. Therefore, we cannot guarantee that LTE V2X devices would be updated to use the new pre-configuration at all. Even for the vehicles that support OTA updates, there is no guarantee that SL devices would receive the updates simultaneously. Hence OTA updates cannot be relied on to ensure pre-configuration consistency and a functioning system when the change in traffic/network is as significant as changing the resource pool bitmap. Lastly, any pre-configuration change for V2X must go through the appropriate regional standards body, which is an involved process. This is not an issue for dynamic coexistence solutions since, by design, they can accommodate variable traffic. We would like to also note that traffic variability is not only in time but also may be location specific, i.e., the mix of newer/older vehicles vary within a region based on location. Hence, we recommend the following edits to the FL’s proposal: - Feasibility of semi-static resource pool partition and dynamic resource sharing are considered to be studied as possible solutions for co-channel coexistence. - For semi-static resource sharing, it is already possible supported to configure separate non-overlapping (in time and frequency) resource pools for LTE NR and SL NR based on Rel-16/17 specifications. |
| Intel | Yes | We are also OK with Ericsson proposed text. |
| Samsung | Yes |  |
| Bosch | Yes with Editing. | Even if resource partitioning can be conducted with NR Rel-16/17, we have not studied it for coexistence. Therefore, we may delete the sub-bullet as proposed by some companies: • Feasibility of semi-static and dynamic resource sharing as possible solutions for co-channel coexistence to be studied. |
| Toyota ITC | Yes |  |
| InterDigital | Yes |  |
| MediaTek | Yes with comment | As mentioned by some companies, we need to look into the feasibility of R16 semi-static TDM/FDM for R18 coexistence, so we can delete the sub-bullet. |
| Apple | Yes with edits | We share the view with Qualcomm that “semi-static resource pool partition” should be used instead of “semi-static resource sharing”. |
| Nokia, NSB | Yes |  |
| ETRI | Yes |  |
| Lenovo | Yes |  |
| Spreadtrum | Yes |  |
| WILUS | Yes |  |
| DCM | Yes |  |
| Transsion | Yes |  |
| Sony | Yes |  |
| vivo | Yes with modifications | We are generally fine with this proposal. But we think the ‘non-overlapping’ can be for either time or frequency domain. And there are some typos should be modified as following: Proposal 2-1(I): - Feasibility of semi-static and dynamic resource sharing are considered to be studied as possible solutions for co-channel coexistence. - For semi-static resource sharing, it is already possible supported to configure separate non-overlapping (in time and or frequency) resource pools for LTE SL NR and NR SL NR based on Rel-16/17 specifications. |
| Huawei, HiSilicon | Yes with modification | We do not think the feasibility of semi-static resource sharing needs to be further studied. Based on the comments from Vivo and us in the last round, we have demonstrated that how to update the RRC configuration and pre-configuration over the signalling. So it is feasible from specification perspective. For the case QC mentioned that a few UEs cannot support over-the-air update, based on our understanding, it is also feasible to update the configurations via wire for vehicles maintenance. However, whether it is feasible for every region, we think this topic is beyond 3GPP discussion. Different regions may have different regulation. For some places, one kind of way to update might be not allowed temporally, but it does mean the method is not feasible for every place. For the subbullet, we think it is necessary to be kept. It reflects current situation of the spec and help to align companies understanding, and we also think it should have same level with the first bullet. Therefore, we suggest to have following modifications in green font on the proposal. Proposal 2-1(I): - Feasibility of semi-static and dynamic resource sharing isare considered to be studied as a possible solutions for co-channel coexistence. - For the solution of semi-static resource sharing for co-channel coexistence, it is already possible supported to configure separate non-overlapping (in time and frequency) resource pools for LTE NRSL and SL NR SL based on Rel-16/17 specifications. |
| CATT/GOHIGH |  | Remove the sub-bullet |
| LGE | Comment | Again, we think that the sub-bullet is not necessary. This is because RAN1 has never investigated whether the update of (pre)configuration for the semi-static resource pool partition is feasible or not. Given with this situation, keeping the sub-bullet creates only unnecessary misinterpretation of the proposal. |

### Summary of 3rd Round of Discussions

Summary of Preferences

The following is a summary of the preferences marked by companies on Proposal.

- Companies supporting the proposal: Panasonic, OPPO, Intel, Samsung, Toyota, IDC, Apple, Nokia, ETRI, Lenovo, Spreadtrum, WILUS, DCM, Transsion, Sony, Vivo, HW+HiSi (comments). [17]

- Companies supporting the proposal without sub-bullet: ZTE+Sanechips, Ericsson, NEC, CableLabs, QC, Bosch, MediaTek, CATT, LGE. [9]

Summary of Comments

Based on the suggestions made by QC and Apple regarding the incorrectness of using the term “semi-static resource sharing”, the FL has been included in the revised proposal.

HW feels that semi-static resource sharing need not be studied further, but can be agreed as a solution for co-channel coexistence. The FL disagrees with the assessment, because although resource pools for LTE SL and NR SL can be configured in a semi-static and orthogonal manner, their feasibility as a co-channel coexistence solution has not been studied. In fact, multiple companies had raised concerns for both the TDM and FDM-based semi-static resource partitioning, and unless these concerns have been studied in detail and resolved, these solutions can only be included in the study, but not agreed as feasible solutions.

Ericsson had text correction, which has also been included in the revised proposal.

FL’ Recommendation

It seems that all the companies are in favor of studying both semi-static resource pool sharing and dynamic resource sharing solutions for co-channel coexistence.

In the FL’s opinion, with regard to the sub-bullet, it simply states that resource pools for LTE SL and NR SL can already be configured in a semi-static and non-overlapping manner according to Rel-16/17 specifications. It does not add any significant value to the proposal, apart from stating a feature that is already supported by the previous release. The FL has listed both options down, and can be discussed in the GTW.

Proposal 2-1(II):

Option 1:

- Feasibility of semi-static resource pool partitioning and dynamic resource sharing as possible solutions for co-channel coexistence are studied.

  - For semi-static resource pool partitioning, it is already supported to configure separate non-overlapping (in time and/or frequency) resource pools for LTE SL and NR SL based on Rel-16/17 specifications.

Option 2:

- Feasibility of semi-static resource pool partitioning and dynamic resource sharing as possible solutions for co-channel coexistence are studied.

### Company Views for 4th Round of Discussions

Based on the inputs from companies in the previous round, would the following proposal be acceptable to the companies?

Proposal 2-1(II):

- Feasibility of semi-static resource pool partitioning and dynamic resource sharing as possible solutions for co-channel coexistence are studied.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | Yes | We agree to the FL’s proposal |
| CMCC | Yes | We agree to the FL’s proposal |
| Apple | Yes | Since we have not yet studied feasibility of these two solutions, we suggest modifying the proposal to Feasibility of semi-static resource pool partitioning and dynamic resource sharing as possible solutions for co-channel coexistence are to be studied. |
| InterDigital | Yes |  |
| ZTE,Sanechips | Yes | Ok with Apple's edit. |
| Xiaomi |  | We are not sure how feasibility of TDMed semi-static resource pool partitioning can be further studied, since in closed issue 2-2 discussion it has been summarized that “Based on the comments received from companies, it is theoretically possible to carry out the updates, but its feasibility in carrying out the updates is questionable and not in the scope of RAN1’s discussion.” |
| DCM | Yes | We prefer to keep the sub-bullet, but can live with it for progress. |
| Spreadtrum | Yes |  |
| Panasonic | Yes |  |
| Sony | Yes | Agree with Apple’s modify. |
| Intel | Yes | We are OK with the FL’s proposal |
| OPPO | Yes |  |
| Sharp | Yes |  |
| vivo | Yes |  |
| ETRI | Yes |  |
| Fraunhofer | Yes | We are supportive of the FL’s proposal. Regarding Xiaomi’s comment, in our understanding, the aspect of updating semi-static resource configurations was decided to not be pursued further because companies had stated that from RAN1’s perspective, the tools to update configurations are specified. However, its feasibility in real life is unfortunately out of RAN1’s scope, which is why this aspect of semi-static resource partitioning need not be considered further. However, other aspects of its feasibility can be studied. We are also fine with Apple’s suggestion. Feasibility of semi-static resource pool partitioning and dynamic resource sharing as possible solutions for co-channel coexistence are to be studied. |
| NEC | Yes |  |
| Samsung | Yes | OK for progress |
| Nokia, NSB | Yes |  |
| Lenovo | Yes |  |
| MediaTek | Yes | Support FL proposal. |
| Huawei, HiSilicon | No | We disagree the assessment from FL on the feasibility of semi-static resource pool partitioning. As we clarified, the feasibility of semi-static resource sharing is already verified Rel-16/17 and the semi-static resource sharing is already supported in specification, especially the TDM manner between NR-V and LTE-V pools. The update of resource pool (pre-) configuration is also applicable as we explained through multiple methods including over-the-air or via wire locally. If companies still have concerns on the FDMed manner of semi-static resource pool partitioning, we can compromise to have this case only to further study the feasibility. On the removing sub-bullet, we strongly think it is needed. The sub-bullet clearly states the current specification and help the group to keep on the same page. It is already implemented and no further investigation is needed. Modification on the proposal is suggested using green font. Proposal 2-1(II): - Feasibility of semi-static resource pool partitioning in FDMed manner and dynamic resource sharing as possible solutions for co-channel coexistence are studied. - For semi-static resource pool partitioning, it is already supported to configure separate non-overlapping (in time and/or frequency) resource pools for LTE SL and NR SL based on Rel-16/17 specifications. |
| Ericsson | Yes | We are not supportive of considering dynamic resource sharing |
| Toyota ITC | Yes |  |

### Company Views for 5th Round of Email Discussions

FL’s comment:

Majority of the companies are in favour of the proposal, while only 1 company opposes it. In the FL’s view, the feasibility of semi-static resource partitioning as a solution for co-channel coexistence has not been studied nor determined. Although they can be implemented using the existing specifications, they still need to be investigated in terms of co-channel coexistence solutions. Changes suggested by Apple have been considered in the updated proposal.

Proposal 2-1(III):

- Feasibility of semi-static resource pool partitioning and dynamic resource sharing as possible solutions for co-channel coexistence are to be studied.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | Yes |  |
| Qualcomm | Yes |  |
| Bosch | Yes |  |
| Apple | Yes |  |
| Intel | Yes |  |
| Panasonic | Yes |  |
| NEC | Yes |  |
| Xiaomi | Yes | We can accept the proposal with the understanding that RAN1 will make conclusion on the feasibility of both TDMed and FDMed semi-static resource pool partitioning in the future |
| Vivo | Yes |  |
| CMCC | Yes |  |
| Transsion | Yes |  |
| OPPO | Yes |  |
| Continental | Yes |  |
| Huawei, HiSilicon | No | We have concerns on the proposal if the subbullet is not captured. The subbullet is simply capturing the current spec points, it does not imply the semi-static resource partitioning is already agreed or not. It is beneficial to keep companies on the same page for further study. Based on the companies comment, no one disagrees that the current spec support resource pools partitioning in time and/or frequency. Thus, what is the technical reason to not have the subbullet? We keep suggesting to add the subbullet back. - Feasibility of semi-static resource pool partitioning in FDMed manner and dynamic resource sharing as possible solutions for co-channel coexistence are studied. - For semi-static resource pool partitioning, it is already supported to configure separate non-overlapping (in time and/or frequency) resource pools for LTE SL and NR SL based on Rel-16/17 specifications. |
| Nokia, NSB | Yes |  |
| Ericsson | Yes |  |

FL’s comment:

Companies seem to be fine with the proposal. In response to Xiaomi’s comment, yes both semi-static solutions will be studied.

FL’s comment:

No concerns regarding the main bullet.

@Yangfan, the FL is unsure about the value of adding a sub-bullet that captures details of the current specification. Although resource pools for LTE SL and NR SL can be configured in a semi-static and orthogonal manner, their feasibility as a co-channel coexistence solution has not been studied. In fact, multiple companies had raised concerns for both the TDM and FDM-based semi-static resource partitioning, and unless these concerns have been studied in detail and resolved, these solutions can only be included in the study (as reflected in the proposal), but not yet agreed as feasible solutions.

### Summary of 5th Round of Email Discussions

The following version of the proposal was submitted for email approval.

Proposal 2-1(III):

- Feasibility of semi-static resource pool partitioning and dynamic resource sharing as possible solutions for co-channel coexistence are to be studied.

## [CLOSED] Issue 2-2: Issues with Semi-static Co-channel Coexistence Solutions

### Company Views for 1st Round of Discussions

Question 2-2: If semi-static coexistence solutions (Solution A and B) were selected in Q2-1, given that LTE SL has already been deployed, how would the resource pool (pre-)configurations be updated in a timely and flexible manner?

If other issues are to be considered for solution A, please describe.

| Company | Comments |
| --- | --- |
| vivo | The (pre-)configurations of LTE SL can be updated via Uu interface or PCF interface. |
| Intel | In our view, this might be an implementation or regulatory matter that should be further discussed. |
| Qualcomm | In our view, this is one of the critical issues with semi-static solutions. LTE V2X pre-configurations have already been approved, after a lengthy process, by regional standards bodies and in those cases are extremely difficult to change. Then there is the issue of different UEs having different versions of a pre-configuration, preventing those Ues from communicating with each other and causing collisions between their transmissions. Lastly, this also impacts system flexibility as more devices employ NR SL and more resource are needed for NR SL compared to LTE SL. |
| Samsung | In coverage Ues can be updated through RRC and SIB signalling. For out-of-coverage Ues, provision and update of parameters is described in the 23.287 clause 5.1 and its sub-clauses. |
| ZTE, Sanechips | Similar views as vivo and Intel. For LTE SL, up to 16 Rx resource pool per carrier in SL (pre-)configurations can provide some flexibility to handle the issue that different Ues may have different Tx resource pools. |
| Xiaomi | There are existing solutions for updating (pre-)configuration. We do not think enhancement on (pre-)configuration updating shall be included in the study scope. |
| NEC | Solution A and B are used to provide semi-static co-existence of the two RATs, as resource pool (pre-)configuration can be adapted with a certain period. A quite timely and flexible update is not a feature to be obtained by using semi-static co-existence solution. |
| Ericsson | This issue is beyond the scope of RAN1 and likely beyond the scope of 3GPP |
| Huawei, HiSilicon | Our paper R1-2203148 has provided detailed explanation how to support resource pool (pre-)configurations for both NR-V and LTE-V. In general, for LTE-V, resource pool configurations are updated by SL-CommResourcePoolV2X IE. For the vehicles in coverage and RRC_CONNECTED state, the IE can be updated by SL-CommConfig within RRCConnectionReconfiguration. For the Ues in coverage but in RRC_IDLE state, it can be further updated by SystemInformationBlockType21 within SystemInformation. Pre-configuration can be also updated by over-the-air signalling. In 5G System, the V2X parameters can be re-configured for both ME (Mobile Equipment) and UICC (Universal Integrated Circuit Card). The pre-configurations in UICC could be updated through the signalling received from the operator. And those information in UICC are prioritized than those within ME. Therefore, although LTE SL has already been deployed, its resource pool (pre-)configuration can still be updated. |
| Nokia, NSB | We think that it is firstly very doubtful whether it is feasible to reconfigure already deployed LTE devices, and secondly it is very difficult to manage a transition from one pre-configuration to another while this reconfiguration is ongoing. While TS 23.287 defines mechanisms for update of preconfiguration, it is not clear whether Ues are required to support these mechanisms and how it can be ensured in practice that all the Ues perform the update in a timely manner. |
| Mitsubishi | Although theoretically feasible, such reconfiguration might not be easily doable in practice. Answering this question might help us reach some common understanding about why we need to go towards dynamic co-channel coexistence, (not sure that we need to make any agreement on this) |
| CATT | This should depend on implementation. |
| Bosch | This is regulatory, profiling, and configuration issue which should not be discussed in 3GPP. The penetration of each device type in the network should be evaluated (by an entity) and (pre-)configuration is updated. |

### Summary of 1st Round of Discussions

Summary of Comments

Under this question, some companies have explained how the possible update and reconfiguration of resource pool (pre-)configurations are possible, while other companies have stated that though theoretically possible, it would be extremely difficult to update already preconfigured LTE resource pools.

- Possible and feasible to reconfigure (pre-)configurations: Vivo, SS, ZTE+Sanechips, HW+HiSi. [4]

- Difficult to update (pre-)configurations: QC, Nokia, Mitsubishi. [3]

- Out of scope: Intel, Xiaomi, NEC, Ericsson. [4]

FL’ Recommendation

It is clear that for semi-static resource sharing to work as a solution for co-channel coexistence, it is vital for already deployed LTE SL UEs to update and reconfigure their resource pool configurations. Based on the comments received from companies, it is theoretically possible to carry out the updates, but its feasibility in carrying out the updates is questionable and not in the scope of RAN1’s discussion.

## [ACTIVE] Issue 2-3: Issues with FDM based Co-channel Coexistence Solutions

### Company Views for 1st Round of Discussions

Question 2-3: If solution B was selected in Q2-1, the presence of PSFCH symbols in NR SL time slots and the NR SL resource pools configured with different SCSs result in AGC issues for LTE SL Ues. How would these be addressed?

If other issues are to be considered for solution B, please describe.

| Company | Comments |
| --- | --- |
| vivo | Firstly, we believe that a typical scenario for LTE-NR coexistence is the single SCS case and prefer to limit the NR SL SCS to 15kHz regardless of which solution is used. Secondly, the AGC issue can be solved by (pre-)configuring guard bands/RBs between LTE-NR frequencies |
| Intel | We agree with Vivo’s comments. |
| Qualcomm | In addition to AGC, there is the issue of dividing already narrow ITS spectrum. |
| Samsung | We can use the same SCS for LTE SL and NR SL to avoid AGC problems. PSFCH slots can be configured to be used exclusively by NR SL, alternatively HARQ-ACK can be disabled. |
| ZTE, Sanechips | If solution B is applied, NR SL SCS should be 15kHz too, and the slot with PSFCH should not be configured on the same slot for LTE SL by implementation of resource pool configuration. |
| Xiaomi | We agree with vivo comments. |
| NEC | We agree with vivo, 15kHz should be used as a common configuration for both the RATs. |
| Huawei, HiSilicon | We share the view with companies that these problems can be avoided by implementation, RAN1 does not need to discuss how to optimize such issues. |
| Bosch | As Vivo’s comments, guard RBs can be used (where we may have from both schemes leftover RBs). Regarding the limited ITS band, we believe that this should not be a permenant problem. In otherwords, once the deployment scales, band should be available. |

### Summary of 1st Round of Discussions

Summary of Comments

Based on the comments from companies, the solutions to be used in order to overcome the AGC issues caused by the presence of PSFCH symbols in NR SL time slots and the NR SL resource pools configured with different SCSs are as follows:

- For both semi-static and dynamic resource sharing, limit the SCS to 15kHz: Vivo, Intel, SS, ZTE+Sanechips, Xiaomi, NEC. [6]

- AGC issue can be solved by (pre-)configuring guard bands/RBs between LTE-NR frequencies: Vivo. [1]

- Resource pools for LTE SL should not be configured on the same slots as NR SL with PSFCH: ZTE+Sanechips. [1]

- PSFCH slots can be configured to be used exclusively by NR SL, alternatively HARQ-ACK can be disabled: SS. [1]

- Issues can be solved by implementation: HW+HiSi. [1]

FL’ Recommendation

Among the companies that support FDM based semi-static resource sharing, it is understood that the resource pools have to be limited to 15kHz SCS for the solution to overcome the AGC issues. Since this is conditional upon Proposal 2-1, it can be considered once Proposal 2-1 has been agreed/settled upon.

Proposal 2-3:

- For semi-static resource sharing as a possible solution for co-channel coexistence, the resource pools for both LTE SL and NR SL are limited to a SCS of 15kHz.

### Company Views for 2nd Round of Discussions

Question 2-3a: Based on the inputs from companies in the previous round, would the following proposal be acceptable to the companies?

Proposal 2-3:

- For semi-static resource sharing as a possible solution for co-channel coexistence, the resource pools for both LTE SL and NR SL are limited to a SCS of 15kHz.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Intel |  | While we are OK with the proposal, we are not sure what the expected spec impact would be here if we make this greement. In this sense, we agree with HW that this issue could be solved via implementation. In this sense, an additional sub-bullet (or note) could be added as follows: Proposal 2-3: - For semi-static resource sharing as a possible solution for co-channel coexistence, the resource pools for both LTE SL and NR SL are limited to a SCS of 15kHz. - This would not have any spec impact. |
| Apple |  | This proposal is not applicable to TDM-based semi-static resource sharing. Hence, we need to clarify it in the proposal, i.e., “For FDM based semi-static resource sharing….” |
| ETRI | Yes | Agree with Apple’s comments |
| Lenovo | Yes |  |
| Panasonic | Yes |  |
| China Telecom |  | We think the AGC issues caused by the presence of PSFCH symbols in NR SL time slots and the NR SL resource pools configured with different SCSs can be resolved by UE implementation. Whether it need to have the constrains on SCSs needs further discussion, not only for semi-static resource sharing but also for dynamic resource sharing. |
| Sony | Yes |  |
| MediaTek | Yes for only FDM-based sharing |  |
| CMCC |  | First, this should be only applicable for FDM case. One more clarification is that in NR SCS is configured tied to BWP, not resource pool, do we need to mention SL BWP in this proposal? |
| Spreadtrum | Yes |  |
| ZTE, Sanechips | Yes for only FDM based solution | Share the same views with CMCC, NR SL SCS is configured per SL BWP. |
| OPPO |  | We share the same view with Apple. |
| Fraunhofer | Yes, with comments | Although this could be handled by implementation, some mention of such a restriction needs to be done while studying the viable solutions for co-channel coexistence. We also agree with Apple and others that such a restriction should be limited to only FDM based solutions. |
| NEC | Yes |  |
| Transsion | Yes for only FDM-based sharing |  |
| Samsung | Proposal not needed | This would only apply to FDMed resource pools. There is no such restriction needed for TDMed resource pools. Even for FDMed resource pools this can be left for implementation. We can have a conclusion along the lines of: For semi-static resource sharing between LTE SL and NR SL, RAN1 will not persue any optimaztion when LTE SL and NR SL have different sub-carrier spacing. |
| Qualcomm | Comment | We generally agree that the SCS for the cochannel deployment should be 15 kHz for both semi-static and dynamic cochannel coexistence. However, we think that it should be clarified that the limitation is for the purpose of the study only. - For studying semi-static resource sharing pool portioning as a possible solution for co-channel coexistence, the resource pools for both LTE SL and NR SL are limited to a SCS of 15kHz. |
| Bosch | Comment | We are fine that this proposal is for FDMed only (unless we can apply guard bands/RBs). However, for TDM, different numerologies may be allowed. |
| Nokia, NSB | No | No motivation for this constraint in TDM semi-static sharing; for FDM semi-static sharing we can study additional approaches to address the AGC issue, so it is premature to impose this constraint. |
| Xiaomi | No | Firstly, this applies only to FDMed semi-staticaly resource pool sharing; secondly, although we think setting SCS of NR SL as 15KHz can be a solution by implementation, we do not intend to preclude other possible solutions in the study。 |
| CATT/GOHIGH | No | it is premature to impose this constraint. |
| Vivo | Yes |  |
| Huawei, HiSilicon | Yes for FDMed case | We share similar views with some companies that this restriction is applied to FDMed case, so suggested changes are as following. Proposal 2-3: - For FDM based semi-static resource sharing as a possible solution for co-channel coexistence, the resource pools for both LTE SL and NR SL are limited to a SCS of 15kHz. |
| DCM | Yes |  |
| LGE | No | We don’t think that this kind of conclusion should be made at this meeting (i.e., RAN1 needs to have further discussion on what kind of restrictions are really necessary for each possible solution). Also at the moment, it is not clear why this kind of restriction is necessary even for TDM based semi=stacte solution. |
| WILUS | Yes | We agree with Apple’s comment that this proposal needs to clarify only to be applicable for FDM based semi-static resource sharing. |

### Summary of 2nd Round of Discussions

Summary of Preferences

The following is a summary of the preferences marked by companies on Proposal 2-3.

- Companies supporting the proposal: Lenovo, Panasonic, Sony, Spreadtrum, NEC, QC, Vivo, DCM. [8]

- Companies supporting the proposal only for FDM: Apple, ETRI, MediaTek, CMCC, ZTE+Sanechips, OPPO, Fraunhofer, Transsion, Bosch, HW+HiSi, Wilus. [11]

- Companies supporting the proposal but does not have any spec impact: Intel. [1]

- Companies not supporting the proposal since it can be resolved by implementation: China Telecom, Samsung, Xiaomi. [3]

- Companies not supporting the proposal: Nokia, Xiaomi, CATT, LGE. [4]

Summary of Comments

Many companies have mentioned that restricting the SCS of SL BWPs containing LTE SL and NR SL resource pools to 15kHz needs to be done only if they are FDMed with each other, and should not apply for TDM-based resource pool sharing. This has been included in the revised proposal.

CMCC and ZTE stated that the SCS configuration is linked to the SL BWP and not to each resource pool, which is as per the Rel-16/17 specifications. This change has also been included in the revised proposal.

Another set of companies had expressed their view that the restriction can be carried out by network implementation, and need not be specified. In the FL’s view, although resource pool configurations are handled by the network, it has to be specified that if the resource pools are used for FDM-based resource sharing, they can be configured only in SL BWPs with an SCS of 15kHz.

The comment from QC stresses on stating that the proposal is meant to study the feasibility of semi static resource sharing. This change has been addressed in new proposal.

FL’ Recommendation

In the discussions, 19 companies support the proposal with the correction on being meant only for FDM-based solutions. 7 companies do not support it. Hence the following proposal can be considered for an agreement, with updates as suggested by companies in the comments.

Proposal 2-3(I):

- For studying the feasibility of FDM-based semi-static resource sharing as a possible solution for co-channel coexistence, the SL BWP configured with resource pools for both LTE SL and NR SL are limited to a SCS of 15 kHz.

### Company Views for 3rd Round of Discussions

Question 2-3b: Based on the inputs from companies in the previous round, would the following proposal be acceptable to the companies?

Proposal 2-3(I):

- For studying the feasibility of FDM-based semi-static resource sharing as a possible solution for co-channel coexistence, the SL BWP configured with resource pools for both LTE SL and NR SL are limited to a SCS of 15 kHz.

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE,Sanechips | Yes |  |
| Ericsson | OK |  |
| Panasonic | Yes |  |
| NEC | Yes |  |
| OPPO | Comments | There is no concept/definition of SL BWP in LTE SL. Therefore, we suggest to rephrase the last sentence “the SL BWP configured with resource pools for NR SL is limited to a SCS of 15 kHz, which is same as LTE SL”. |
| Xiaomi | Comment | We agree with OPPO, the SL BWP configuration is only for NR SL. |
| Qualcomm | Please see edit and comments | We again emphasize that the semi-static mechanism considered does not lead to any resource sharing. Hence the proposal should be re-worded as: - For studying the feasibility of FDM-based semi-static resource sharing resource pool partitioning as a possible solution for co-channel coexistence, the SL BWP configured with resource pools for both LTE SL and NR SL are limited to a SCS of 15 kHz. The same approach should be followed for both semi-static and dynamic coexistence solutions, i.e. either both list aspects to consider or neither does. For example, AGC impact due to PSFCH also exists in FDM solutions. Similarly, either the study of both is limited to 15 kHz or both need to study other SCS. FDM-based solutions would also have to drop NR or LTE transmissions. Lastly, there are the issue associated with the semi-static nature of the solution, such as the inability to adapt to changes in traffic or device mix, the loss of available spectrum, and the inability to change pre-configuration of existing devices and deployments. |
| Intel | No | Agree with other companies that this is a valuable constraint, but we are still not convinced that this could not be simply handled by implementation, and spec impact is needed. |
| Samsung | Yes | While it is fine to study the feasibility of FDM-based semi-static resource sharing assuming that the same SCS for LTE and NR SL. We think that this should not have a spec impact. |
| Bosch | Comments | In our understanding, LTE does not consider any BWP configuration, but only resource pools configured in a carrier. Additionally, the NR and LTE resources may not be always in the same device. Therefore, the NR resources may be configured with different numerology to that viable for LTE, i.e., 15 kHz always for LTE. In this case, the NR resource pool configured in an NR BWP will be orthogonal to the LTE one. Only when in-device coexistence is needed, we need to stick to 15 kHz @ both. |
| InterDigital | Yes |  |
| MediaTek | Yes with comment | We agree that SCS of NR-SL should be limited to 15 KHz for FDM-bases semi-static. As pointed out by companies, we should re-phrase the proposal as LTE-SL does not have SL-BWP definition. |
| Apple | Comments | Agree with other companies’ comments that SL BWP is not defined in LTE SL. |
| Nokia, NSB | No | We can study additional approaches to address the AGC issue, so it is premature to impose this constraint. |
| ETRI | Yes |  |
| Lenovo | Comments | We share the same comment from OPPO |
| Spreadtrum | Yes | Agree with OPPO |
| WILUS | Yes | We agree with OPPO, LTE SL does not have SL BWP definition. |
| DCM | Yes | Agree with OPPO |
| Transsion | Yes with comments | We share the similar view as some companies that SL BWP is not defined for LTE SL. |
| Sony | Yes | Agree with OPPO |
| vivo | Yes | Agree with OPPO |
| Huawei, HiSilicon | Yes with comments | LTE-V does not have the concept of SL BWP, we share the views that the proposal should be updated accordingly. The suggestion from OPPO is valid. |
| CATT/GOHIGH | Comments | Add FFS: other SCS |
| LGE | Comment | We suggest to update the proposal as follows: - For studying the feasibility of FDM-based semi-static resource sharing as a possible solution for co-channel coexistence, at least the following can be considered: The SL BWP configured with resource pools for NR SL is limited to a SCS of 15 kHz, which is same as LTE SL |

### Summary of 3rd Round of Discussions

Summary of Preferences

The following is a summary of the preferences marked by companies on Proposal.

- Companies supporting the proposal: ZTE+Sanechips, Ericsson, Panasonic, NEC, OPPO (rewording), Xiaomi (agree with OPPO), QC (rewording), Samsung, IDC, MediaTek, Apple, ETRI, Lenovo, Spreadtrum, WILUS, DCM, Transsion, Sony, Vivo, HW+HiSi, CATT (add FFS for other SCS), LGE (comment). [22]

- Companies not supporting the proposal: Intel, Nokia. [2]

Summary of Comments

OPPO had suggested a correction that the concept of SL BWPs does not exist in LTE, and hence should be updated to reflect this. Many other companies had also agreed to this change, and the FL concurs. The changes have been made in the revised proposal.

Since a few companies are hesitant to introduce such a constraint, but is in the interest of the majority, the FL recommends the inclusion of an FFS to explore other solutions to consider SCSs apart from 15kHz, as recommended by CATT.

FL’ Recommendation

Majority of the companies are in favor of the proposal to introduce a limitation of FDM-based resource pools for both NR SL and LTE SL. Hence, the following proposal can be considered, taking into account the changes suggested.

Proposal 2-3(II):

- For studying the feasibility of FDM-based semi-static resource pool partitioning as a possible solution for co-channel coexistence, the SL BWP configured with resource pools for NR SL is limited to a SCS of 15 kHz, which is the same SCS as LTE SL.

  - FFS: Whether/how to consider other SCSs

### Company Views for 4th Round of Discussions

Based on the inputs from companies in the previous round, would the following proposal be acceptable to the companies?

Proposal 2-3(II):

- For studying the feasibility of FDM-based semi-static resource pool partitioning as a possible solution for co-channel coexistence, the SL BWP configured with resource pools for NR SL is limited to a SCS of 15 kHz, which is the same SCS as LTE SL.

  - FFS: Whether/how to consider other SCSs

| Company | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | comments | In principle we support the proposal. We also note that the issues identified for dynamic coexistence in Proposal 2-5(II) also apply for this. For instance, FDM of PSFCH and LTE transmissions will lead to AGC issues. Further, due to half-duplex issues, the LTE module cannot transmit and receive when NR is transmitting and vice-versa. Thus, the issue of packet dropping is also pertinent to the FDM based semi-static pool partition. Also, we not that the list of issues is not exhaustive and newer issues may raise or the indicated ones get resolved when the actual design is considered. One such important example is in-band emissions. There may be others which will require analysis and simulations. Hence, we propose to remove the FFS or list all the issues. It is simpler to remove the FFS and that is our first preference: - For studying the feasibility of FDM-based semi-static resource pool partitioning as a possible solution for co-channel coexistence, the SL BWP configured with resource pools for NR SL is limited to a SCS of 15 kHz, which is the same SCS as LTE SL. - FFS: Whether/how to consider other SCSs Or: - For studying the feasibility of FDM-based semi-static resource pool partitioning as a possible solution for co-channel coexistence, the SL BWP configured with resource pools for NR SL is limited to a SCS of 15 kHz, which is the same SCS as LTE SL. The following aspects are to be investigated: - Handling of numerologies other than 15kHz - Configuration of overlapping time resources for LTE SL and NR SL including in slots where NR PSFCH may be transmitted, taking into account the handling of AGC. - Mechanisms to avoid dropping of NR SL transmissions impacted by LTE SL transmissions. - FFS: Other aspects. |
| CMCC | Yes |  |
| Apple | Yes | We suggest removing “which is the same SCS as LTE SL”, since it is redundant as LTE SL only supports 15 kHz SCS and no changes in LTE SL are allowed. We are also open to list other issues (e.g., NR PFSCH, dropping of NR SL transmissions as in Proposal 2-5) |
| InterDigital | Yes |  |
| ZTE,Sanechips | Yes |  |
| Xiaomi | Yes |  |
| DCM | Yes |  |
| Spreadtrum | Yes |  |
| Panasonic | Yes |  |
| Sony | Yes |  |
| Intel | Yes with comments | We are generally OK with the proposal, and we do not want to block the progress. However, we just want to make sure that this would not have any spec impact per-se. |
| OPPO | Yes | We are also open to list other FFS issues for the FDM approach. |
| Sharp | Yes |  |
| vivo | Yes | Although we support to limit the SCS to 15kHz, we are ok to have other SCS as FFS for progress. |
| ETRI | Yes |  |
| Fraunhofer | Yes | We are supportive of the FL’s proposal. If the FFS is controversial, we are also fine to drop it. |
| NEC | Yes |  |
| Samsung | Yes |  |
| Nokia, NSB | Comment | In terms of issues to be addressed, there are strong similarities between FDM-based semi-static partitioning and dynamic sharing; so we would prefer to harmonize this proposal here with Proposal 2-5 (II) and add the following (copied from Proposal 2-5 (II)): the following aspects are to be investigated: - Handling of numerologies other than 15kHz - Configuration of overlapping time resources for LTE SL and NR SL including in slots where NR PSFCH may be transmitted, taking into account the handling of AGC. - Mechanisms to avoid dropping of NR SL transmissions impacted by LTE SL transmissions. - FFS: Other aspects. |
| Lenovo | Yes |  |
| MediaTek | Comment | We are supportive. But we prefer removing the FFS point. If the FFS point is kept, we are a bit struggling to understand what the proposal is achieving. The main bullet point is suggesting that FDM with NR-SL of 15 KHz SCS is further studied. If the FFS point also keeps the scope of FDM study to other SCS values, then we are basically only agreeing to study FDM solution in general without any limitation of the SCS value. |
| Huawei, HiSilicon | Yes, can accept to remove the FFS | We are fine with the proposal in general. As companies’ analysis, support of other SCS may introduce other issues, we can accept to focus on the 15kHz SCS case primarily and remove the FFS. |
| CATT/GOHIGH | Yes | Prefer to keep the FFS, no reason to exclude the study at this stage. |
| Ericsson | Yes |  |

### Company Views for 5th Round of Email Discussions

FL’s comment:

All companies are supportive of the main bullet, the question remains only with the inclusion/exclusion of the FFS.

Proposal 2-3(III):

- For studying the feasibility of FDM-based semi-static resource pool partitioning as a possible solution for co-channel coexistence, the SL BWP configured with resource pools for NR SL is limited to a SCS of 15 kHz, which is the same SCS as LTE SL.

  - FFS: Whether/how to consider other SCSs

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | Yes with FFS | Support with FFS kept. Since, the main bullet says 15 kHz SCS is studied, the FFS should be kept. Otherwise, remove 15 kHz from the main bullet and the FFS. |
| Qualcomm | Yes |  |
| Bosch | Yes with FFS |  |
| Apple | Yes | Fine with the current version, though we still think “which is the same SCS as LTE SL” is not needed. |
| Intel | Yes | We are OK with the current version, and still think that this should not have any spec impact. |
| Panasonic | Yes |  |
| NEC | Yes |  |
| Xiaomi | Yes |  |
| Vivo | Yes |  |
| CMCC | Yes | Comments. Whether PSFCH and AGC should also mentioned here and to be studied like proposal 2-5(III). |
| Transsion | Yes |  |
| OPPO | Yes |  |
| Continental | Yes |  |
| Nokia, NSB | Yes with FFS |  |
| Ericsson | Yes |  |

FL’s comment:

Companies are fine with the main bullet. While Samsung and Bosch want to include the FFS, it was seen in previous rounds that companies opposed it. We can listen to the views of other companies before changing it again. In response to CMCC’s comment, we could mention other issues, but the FL would prefer to focus on the main bullet first.

FL’s comment:

Companies are fine with the main bullet, while 3 companies insist on including the FFS. Based on this, the FL has modified the proposal, kindly check. The update does not preclude the study of other SCSs being supported.

Proposal 2-3(IV):

- For studying the feasibility of FDM-based semi-static resource pool partitioning as a possible solution for co-channel coexistence, the SL BWP configured with NR SL resource pools for NR SL is limited to with a SCS of 15 kHz is considered, which is the same SCS as LTE SL.

  - FFS: Whether/how to consider other SCSs

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | Yes with FFS | Support with FFS. Otherwise remove 15 kHz from the main bullet |

### Summary of 5th Round of Email Discussions

Some companies were of the view that FDM-based semi-static solutions should be studied with the limitation of the SCS to 15kHz, while some insisted that the FFS should remain.

Hence this topic could not be agreed upon in this meeting.

## [CLOSED] Issue 2-4: Issues with Dynamic Co-channel Coexistence Solutions

### Company Views for 1st Round of Discussions

Question 2-4: If solution C was selected in Q2-1, how would NR SL modules/UEs be aware of the resources that are being occupied by LTE SL modules/UEs? Kindly factor in the cases where a device can have both NR SL and LTE SL modules, and a device with only an NR SL module.

If other issues are to be considered for solution C, please describe.

| Company | Comments |
| --- | --- |
| NTT DOCOMO | In device type A, LTE-SL module shares reservation information to NR-SL module, and NR-SL module excludes corresponding resources in resource allocation procedure. In device type B, the UE receives IUC message generated based on LTE-SL reservation and performs corresponding ehaviour. |
| Apple | For NR SL mode 2 operation: A device with both LTE SL module and NR SL module can perform sensing on LTE SL and passes the sensing results to its NR SL module for its resource selection. A device (UE-B) with only NR SL module can receive inter-UE coordination (either scheme 1 or scheme 2) from another device (UE-A) with LTE SL module and NR SL module. This inter-UE coordination information is used for UE-B’s resource selection. For NR SL mode 1 operation: A device with both LTE SL module and NR SL module can perform sensing on LTE SL and reports the sensing results to gNB to facilitate its resource allocation. |
| Vivo | The NR UE with dual modules can be aware of the LTE SL reservations through the information provided by LTE module. As for UE with only NR SL module, it depends the functionality of the NR module, please refer to our comments in Question 1-1 |
| Intel | While we believe that further study is needed to determine whether solution C may be beneficial, a dynamic solution does in our opinion only make sense if only type A Ues are mandated. As type B Ues cannot be aware of the LTE SL allocation we think for a dynamic solution type B Ues should not be present. |
| Qualcomm | For a device with both NR SL and LTE SL modules - The LTE SL module can provide information on its own LTE SL sensing over an in-device shared interface. We note that the Rel. 16 in-device coexistence mechanism makes the same assumption for prioritizing traffic. In fact, the dynamic pool sharing can be supported by reusing this framework. For devices with only NR SL modules - We reiterate that supporting UE-s with only NR SL modules may not be feasible due to the basic-safety concerns detailed in our reply to Q 1-1. - If UE-s with only NR SL modules is to be supported in addition to the dual-module devices, they could receive information from the dual-module devices in the system. |
| Lenovo | For device type A we think the internal UE coordination could be used to avoid the resource collision between LTE sidelink and NR sidelink, for two modules in one UE we think the coordination on TX/RX status could be exchanged to avoid the simple dropping of one module as defined in R16 sidelink, and NR module could also exclude the resources reserved by other LTE sidelink Ues based on the sensing results of its own LTE module |
| Samsung | For devices with LTE SL module and NR SL module, the LTE SL module can share sensing results with the NR SL module to use for resource exclusion. For devices without LTE SL module, they can receive inter-UE co-ordination information about conflicts or preferred/non-preferred resources from Ues that have an LTE SL module. |
| OPPO | The NR module is able to detect the SCI of LTE SL or the NR module can derive the sensing result from LTE module. |
| ZTE, Sanechips | For device type A, the LTE module may help NR module to sense the LTE reservation resources. But LTE SL UE cannot acquire the information of NR SL. The impact on LTE SL should be further studied. And for type B Ues, they cannot be aware of the LTE SL resource reservation, and the dynamic solution should not be feasible for type B Ues. |
| Transsion | For device type A, the LTE SL module can share its sensing results to NR SL module through the internal interface. For device type B, we believe that this may not be feasible for the scenario of dynamic co-channel coexistence. |
| InterDigital | Short-terms TDM is specified in R16 to enable prioritization for “not-co-channel” coexistence. The spec states a UE has the knowledge of both LTE and NR SL TX/RX T ms prior to the earliest TX/RX where T < 4 and is based on UE implementation. Therefore, it is already considered feasible a NR SL module receives resource related information from a LTE SL module residing within the same UE. More LTE SL resource information is needed for co-channel coexistence, so the type of the information and the processing time for NR SL to receive this information should be studied further. For NR SL-only UE, we think the R17 IUC framework can be applied so the NR SL-only UE can receive resource information related to LTE resources from a UE with both NR SL and LTE SL modules. |
| Spreadtrum | For device type A - The sensing results of LTE transmission can be transmitted from the LTE module to the NR module via the interface; For device type A - Whether NR SL device can sense LTE SL transmission needs to be further clarified. - If NR SL device has the capability for sensing LTE SL transmission, it can sense the LTE transmission independently without any assistance; - If NR SL device does not has the capability for sensing LTE SL transmission, it can receive some explicit signaling (including sensing results of LTE SL transmission or preferred/non-preferred resource set) from other UEs or NW. |
| Xiaomi | UE with both NR SL and LTE SL modules can share the LTE information to NR module. For UE with only NR SL module, NR SL module shall have the capability of receiving LTE SL PSCCH/PSSCH. From our point of view, the implementation cost of UE with only NR SL module would be too high. |
| NEC | - As solution C may need more discussion and introduce lots of specification impact, further assessments are needed to keep a reasonable workload. |
| Ericsson | For any device (NR SL alone or NR SL + LTE SL), the following ways of detecting the resources occupied by LTE SL UEs can be considered: - Indirect detection. That is, the device detects LTE SL transmissions without decoding the corresponding signals. For example, energy measurements such as RSSI may be used. - Explicit ignaling. In this case, the device is informed by others about the presence of LTE SL transmissions. Such ignaling could come from another UE, the NW (e.g., a gNB), or OTT. For NR SL + LTE SL devices; - Direct detection as an optimization feature. That is, the device is capable of detecting LTE SL signals. This is applicable to devices implementing both NR SL and LTE SL and should not be considered as a baseline. |
| Sony | For device Type A UE, UE’s LTE SL module can sense and transmit the resource usage information to its NR SL module, and receive other SL UE’s resource usage information through IUC scheme. For device Type B UE, it can obtain LTE and NR SL resource usage information through IUC scheme. |
| Fraunhofer | For Type A devices, the exact procedure on how the LTE SL module transfers/forwards its sensing results to the NR SL module need to be studied. For Type B devices, the UE should be either capable of decoding LTE SL SCIs, or carry out RSSI measurements to detect the presence of LTE transmissions. Another option is to use IUC messaging from a Type A device, informing other Type B devices of the presence of LTE SL transmissions. |
| Huawei, HiSilicon | As explain under Issue2-1, we think it is not justified to specify Solution C considering its complexity and similar performance compare to Solution A/B. If solution C is deemed necessary, the only possible way relies on UE being equipped with both LTE-V and NR-V modules. For device that have both modules, sensing result of LTE-V reservation from LTE-V modules can be delivered to NR-V modules internally. When NR SL UE performs resource selection procedure, the sensing results of LTE-V are used so that resources being occupied by LTE-V UEs are excluded. For the UE only supporting NR-V module, as explained for Question 1-1, it is not even in the scope and should be not considered. |
| Nokia, NSB | In the case where a device has both NR SL and LTE SL modules, then the NR SL module could take into account in its resource selection procedure (including in the resource re-evaluation/re-selection step) the sensing results from the LTE SL module. The case of a device with only NR SL module requires further study. |
| CATT/GOHIGH | FFS but normally UE with dual modules can assume this is through the information provided by LTE module |
| LGE | For Device Type A, for example, LTE SL module can provide the informaotin about the resources that are occupied as a result of LTE SL sensing operation to NR SL module. For Device Type B, we think that before diuscussing the details in supporing Solution C, its necessity to be considered is firstly clarified/decided. |
| Bosch | If type A (as a baseline): transferring LTE sensing results (instantenously) may need to be assumed. Even though, how to transfer and/or how to utilize LTE sensing may be left to the UE implementation, which may have degradation from vendor to another. In our understanding, RSSI sensing and accomplishing quasi dynamic (e.g., using detect and vacate) may be more feasible giving the workload. |

### Summary of 1st Round of Discussions

Summary of Comments

Based on the comments from companies, it is clear that for device type A, the NR SL module can use the sensing and resource reservation information shared by the LTE module to exclude the corresponding resources for its resource selection procedure. The exact method of sharing/forwarding was mentioned by a few companies, but the details have to be further studied.

For device type B, it should either be able to decode LTE SCIs or use IUC messages from other type A devices to exclude corresponding resources in the resource selection procedure. Some companies also feel that device type B should not be considered for dynamic resource sharing.

- For device type A, the LTE SL module shares resource reservation information to the NR SL module, and the NR SL module excludes corresponding resources in the resource selection procedure – DCM, Apple, Vivo, QC (using an in-device shared interface), Lenovo (using internal UE coordination), SS, OPPO, ZTE+Sanechips, Transsion (using an internal interface), IDC (similar to Rel-16 in-device procedure), Spreadtrum, Xiaomi, Ericsson, Sony, Fraunhofer, HW+HiSi, Nokia, CATT, LGE. [19]

- For device type B, solution C is not feasible since type B UEs cannot be aware of LTE SL resource allocation – Vivo, Intel, QC, ZTE+Sanechips, Transsion, HW+HiSi. [6]

- For device type B, the type B UE receives IUC messages from a type A UE to exclude corresponding resources in the resource selection procedure – DCM, Apple, SS, IDC, Sony, Fraunhofer. [6]

- For device type B, it should be capable of processing LTE SCIs in order to exclude corresponding resources in the resource selection procedure – Vivo, OPPO, Spreadtrum, Xiaomi, Fraunhofer. [5]

- For device type B, it should receive explicit signaling including sensing results in order to exclude corresponding resources in the resource selection procedure from other UEs or NW – Spreadtrum, Ericsson (for type A devices as well). [2]

- For both device types, the device should detect LTE SL transmissions indirectly e.g. using RSSI measurements – Ericsson, Fraunhofer. [2]

- For device type A, in Mode 1, the LTE SL module performs sensing and reports the sensing results to the gNB – Apple [1]

FL’ Recommendation

19 companies are in favor of supporting solution C for device type A, with the details of the exact sharing/forwarding procedure FFS. Since only 5 companies have indicated their preference to support device type B, it can be left for FFS.

Proposal 2-4:

- For dynamic resource sharing as a possible solution for co-channel coexistence,

  - For device type A, the NR SL module uses the sensing and resource reservation information shared by the LTE module to exclude the corresponding resources for its resource selection procedure.

    - FFS details on how the LTE module shares the information to the NR module, exact information shared etc.

  - FFS: Whether/how device type B should be supported.

### Company Views for 2nd Round of Discussions

Question 2-4a: Based on the inputs from companies in the previous round, would the following proposal be acceptable to the companies?

Proposal 2-4:

- For dynamic resource sharing as a possible solution for co-channel coexistence,

  - For device type A, the NR SL module uses the sensing and resource reservation information shared by the LTE module to exclude the corresponding resources for its resource selection procedure.

    - FFS details on how the LTE module shares the information to the NR module, exact information shared etc.

  - FFS: Whether/how device type B should be supported.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Intel |  | We are generally OK with the proposal, but since we have not yet agreed on whether dynamic resource sharing will be adopted, we would rather prefer the following language: Proposal 2-4: - For If dynamic resource sharing is considered, then as a possible solution for co-channel coexistence, - For device type A, the NR SL module uses the sensing and resource reservation information shared by the LTE module to exclude the corresponding resources for its resource selection procedure. - FFS details on how the LTE module shares the information to the NR module, exact information shared etc. - FFS: Whether/how device type B should be supported. |
| Apple | Yes |  |
| ETRI | Yes |  |
| Lenovo | Yes |  |
| Panasonic | Yes |  |
| China Telecom | Yes | We are fine with FL proposal. For device type B, it needs more detailed discussion and to make the final decision with less specification changes. |
| Sony | Yes |  |
| MediaTek | Comment | We prefer not to agree on the details of a solution before a detailed discussion/study. The phrasing “NR SL module uses the sensing and resource reservation information shared by the LTE module to exclude the corresponding resources” seems too detailed. We need to discuss what kind of information can be used/shared and what are the possible actions that can be taken by the NR SL module first. |
| CMCC | Yes |  |
| Spreadtrum | Yes |  |
| ZTE, Sanechips |  | We support the revised proposal by Intel. |
| OPPO | Yes |  |
| Fraunhofer | Yes |  |
| NEC | Yes |  |
| Transsion | Yes |  |
| Samsung | Yes with comments | Proposal is fine at a high-level, we suggest some updates: - Change “to exclude the corresponding resources” to “resource exclusion” as the reserved resources may or may not be excluded based on additional criteria - In the third-level sub-bullet add timeline (timeline of shared information) - For dynamic resource sharing as a possible solution for co-channel coexistence, - For device type A, the NR SL module uses the sensing and resource reservation information shared by the LTE module to exclude the corresponding resources for resource exclusion in its resource selection procedure. - FFS details on how the LTE module shares the information to the NR module, exact information shared, timeline etc. - FFS: Whether/how device type B should be supported. |
| Qualcomm | No | Although we agree with the overarching theme of this proposal, the phrase “exclude corresponding resources” is too restrictive for designing the dynamic coexistence mechanism at this stage. RAN 1 should study how the NR module uses the sensing and resource reservation information. We propose the following change in the wording: - For dynamic resource sharing as a possible solution for co-channel coexistence, - For device type A, the NR SL module uses the sensing and resource reservation information shared by the LTE module. To exclude the corresponding resources for its resource selection procedure. - FFS: How the NR module uses this information - FFS details on how the LTE module shares the information to the NR module, exact information shared etc. - FFS: Whether/how device type B should be supported. |
| Toyota ITC | No | For device type A, the current proposal is too specific and restrictive at this stage. We should study how the NR SL module uses the information shared by the LTE SL module. In addition, we suggest adding “FFS: Other method(s) for device type A” in the sub-bullets. We propose the following changes: - For dynamic resource sharing as a possible solution for co-channel coexistence, - For device type A, the NR SL module uses the sensing and resource reservation information shared by the LTE SL module to exclude the corresponding resources for its resource selection procedure. - FFS details on how the NR SL module uses the information shared by the LTE SL module. - FFS details on how the LTE module shares the information to the NR module, exact information shared, etc. - FFS: Other method(s) for device type A. - FFS: Whether/how device type B should be supported. |
| Bosch | No | We prefer to go step-by-step at this stage, e.g., first agree whether dynamic coexistence is supported, the agree which UE type is supported, and finaly agree on the details. |
| Nokia, NSB | No | Current text might require the NR SL module to exclude any resource reserved by LTE, regardless of LTE versus NR priority and of LTE PSSCH-RSRP. The solution needs to be more flexible, The updates by Qualcomm or Toyota are fine. |
| Xiaomi | Yes |  |
| CATT | No | For dynamic resource sharing as a possible solution for co-channel coexistence, - For device type A, the NR SL module may uses the sensing and resource reservation information shared by the LTE SL module. - FFS details - FFS: Other method(s) for device type A. FFS: Whether/how device type B should be supported. |
| Vivo | No | 1. We share a similar view as Qualcomm and Toyota ITC, “NR SL module uses the sensing and resource reservation information shared by the LTE module to exclude the corresponding resources” is too detailed for design. 2. the main bullet should be changed to ‘If dynamic resource sharing is considered’ Proposal 2-4: - For If dynamic resource sharing is considered, as a possible solution for co-channel coexistence, - For device type A, LTE module performs sensing of LTE SCI and helps NR SL module for the NR SL module uses the sensing and resource reservation information shared by the LTE module to exclude the corresponding resources for its resource selection procedure for NR SL transmission. - FFS details on how the LTE module shares the information to the NR module, exact information shared etc. - FFS details on how the NR SL module uses the information shared by the LTE SL module. - FFS: Whether/how device type B should be supported. |
| Huawei, HiSilicon | See comments | It has not been justified whether to specify Solution C considering its complexity and similar performance comparing to Solution A/B, thus we tend to agree with Intel’s direction that the proposal needs to be modified to ‘If dynamic resource sharing is considered, then…’. How to transport the information between NR-V and LTE-V modules is UE internal implementation, so the part “how the LTE module shares the information to the NR module” in the FFS is not needed. A general FFS for the details on the impact on resource allocation is sufficient. And as our comments on issue 1-1, the last FFS for type B device is not needed as well. Proposal 2-4: - For If dynamic resource sharing is considered, then as a possible solution for co-channel coexistence, - For device type A, the NR SL module uses the sensing and resource reservation information shared by the LTE module to exclude the corresponding resources for its resource selection procedure. - FFS details on how the LTE module shares the information to the NR module, exact information shared etc the impact on the NR sidelink resource (re-)selection. - FFS: Whether/how device type B should be supported. |
| InterDigital | No | Similar view with Qualcomm that it is premature to make decision how this information is used in NR module. Qualcomm’s suggested update looks good to us. |
| DCM | No | Probably, suggestion from QC/Toyota would be better in this stage. In addition, we would like to emphasize that in round 1, we supported ‘For device type B, the type B UE receives IUC messages from a type A UE to exclude corresponding resources in the resource selection procedure’. |
| LGE | Comment | In principle, we are fine with the intention of proposal. Also for the progress, the modified version provided by Qualcomm can be acceptable to us. |
| Sharp | No | We share the view from other companies that it is premature to agree on the details at this stage. We are fine to go with QC’s update of the proposal. |
| Ericsson | See comment | We are OK to study the above solution. - For dynamic resource study sharing as a possible solution for co-channel coexistence, - For device type A, the NR SL module uses the sensing and resource reservation information shared by the LTE module to exclude the corresponding resources for its resource selection procedure. - FFS details on how the LTE module shares the information to the NR module, exact information shared etc. - FFS: Whether/how device type B should be supported. In addition, we think that device type B has to be supported and the potential solutions regarding this device should be studied. |
| WILUS | Yes |  |

### Summary of 2nd Round of Discussions

Summary of Preferences

The following is a summary of the preferences marked by companies on Proposal 2-4.

- Companies supporting the proposal: Apple, ETRI, Lenovo, Panasonic, China Telecom, Sony, CMCC, Spreadtrum, OPPO, Fraunhofer, NEC, Transsion, Xiaomi, Ericsson, Wilus. [15]

- Companies supporting the proposal if the resource exclusion aspect is moved to FFS: Samsung, QC, Toyota, Nokia, CATT, Vivo, IDC, DCM, LGE, Sharp. [10]

- Companies supporting the proposal depending on whether dynamic resource sharing is agreed: Intel, ZTE+Sanechips, Vivo, HW+HiSi. [4]

- Companies supporting proposal with generic FFS: CATT, HW+HiSi. [2]

- Companies not supporting the proposal: MediaTek, Bosch. [2]

Summary of Comments

While majority of the companies are in favor of the proposal, some companies had stated that the exclusion of resources used by LTE SL transmissions, by NR SL UEs, should be moved to FFS. This has been reflected in the updated version of the proposal below.

Another set of companies had commented that the proposal’s wording needs to be changed since dynamic resource sharing has not been agreed. In the FL’s view, if Proposal 2-1 is agreed, dynamic resource sharing can be studied as one of the solutions for co-channel coexistence. In this case, the wording of the proposal clearly states that dynamic resource sharing is “a possible solution”, and does not imply that is has been agreed as a solution yet.

MediaTek had raised concerns on the proposal containing too much details. According to the FL, the proposal talks about only the task of the LTE SL module sharing its sensing and resource reservation information to the NR SL module. Every other detail regarding how this sharing takes place and what information is exactly shared is FFS. The FL is open to any wording changes to make this more clear.

Another set of companies

FL’ Recommendation

While 15 companies support the proposal, a further 10 support it once the exclusion aspect is moved to FFS. Due to the multiple FFS points under the first sub-bullet, the FL is also fine with wording it as “FFS: details”. Hence, consider the following proposal for an agreement.

Proposal 2-4(I):

- For studying the feasibility of dynamic resource sharing as a possible solution for co-channel coexistence,

  - For device type A, the NR SL module uses the sensing and resource reservation information shared by the LTE SL module to exclude the corresponding resources for its resource selection procedure.

    - FFS details on how the NR SL module uses this information.

    - FFS details on how the LTE SL module shares the information to the NR SL module, exact information shared, timeline etc.

  - FFS: Other method(s) for device type A to be aware of resources being occupied by LTE SL.

  - FFS: Whether/how device type B should be supported.

### Company Views for 3rd Round of Discussions

Question 2-4b: Based on the inputs from companies in the previous round, would the following proposal be acceptable to the companies?

Proposal 2-4(I):

- For studying the feasibility of dynamic resource sharing as a possible solution for co-channel coexistence,

  - For device type A, the NR SL module uses the sensing and resource reservation information shared by the LTE SL module to exclude the corresponding resources for its resource selection procedure.

    - FFS details on how the NR SL module uses this information.

    - FFS details on how the LTE SL module shares the information to the NR SL module, exact information shared, timeline etc.

  - FFS: Other method(s) for device type A to be aware of resources being occupied by LTE SL.

  - FFS: Whether/how device type B should be supported.

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE,Sanechips | No | We prefer to change the last FFS into - FFS: Whether/how the evaluation should take into account device type B |
| Ericsson | See comment | In our view, device type B shall be supported and what should be further studied are the different methods used by these UEs to detect the resources occupied by LTE SL. Therefore, we propose to modify the proposal as follows: Proposal 2-4(I): - For studying the feasibility of dynamic resource sharing as a possible solution for co-channel coexistence, - For device type A, the NR SL module uses the sensing and resource reservation information shared by the LTE SL module to exclude the corresponding resources for its resource selection procedure. - FFS details on how the NR SL module uses this information. - FFS details on how the LTE SL module shares the information to the NR SL module, exact information shared, timeline etc. - FFS: Other method(s) for device type A to be aware of resources being occupied by LTE SL. - For device type B, the NR SL module detects the presence of LTE SL transmissions by itself - FFS: Methods used by device type B to be aware of resources being occupied by LTE SL. - FFS: Support of signaling from UEs/NW |
| Panasonic | Yes |  |
| NEC | Yes with comments | According to the discussion of the definition of type B devices in 4.1.5, the last FFS may need to be removed. |
| OPPO | Yes |  |
| Xiaomi | Yes |  |
| Qualcomm | Yes | We agree with the FL’s proposal |
| Intel | Yes | We are fine with the FL’s proposal |
| Samsung | Yes with comments | We are fine with the proposal. But we would like to delete “FFS: Other method(s) for device type A to be aware of resources being occupied by LTE SL.” This is too broad for the study item |
| Bosch | No | We are still supporting Type B as a simpler solution/alternative to a complex and with harsh timeline LTE-NR modules interworking/sharing. Therefore, we propose the following modification (partially inline with E///): - For studying the feasibility of dynamic resource sharing as a possible solution for co-channel coexistence, - For device type A, the NR SL module uses the sensing and resource reservation information shared by the LTE SL module to exclude the corresponding resources for its resource selection procedure. - FFS details on how the NR SL module uses this information. - FFS details on how the LTE SL module shares the information to the NR SL module, exact information shared, timeline etc. - FFS complexity of the solution - FFS: Other method(s) for device type A to be aware of resources being occupied by LTE SL. - For device type B, the NR SL module detects the presence of LTE SL transmissions by itself - FFS: Methods used by device type B to be aware of resources being occupied by LTE SL. - FFS complexity of the solution |
| Toyota ITC | Yes |  |
| InterDigital | Yes |  |
| MediaTek | Comment | We are open to study dynamic resource sharing based on information exchange between LTE-SL and NR-SL modules. However, we are a bit concerned about making an agreement eight now by listing a set of FFS before the group discusses such a mechanism in details. We would like to suggest revising the proposal (as below), so that we don’t introduce any limitation on potential solutions. Also, we prefer removing the last bullet point. We don’t see how type-B devices can be supported in a design that requires information sharing by ‘LTE-SL’ module. - For studying the feasibility of dynamic resource sharing as a possible solution for co-channel coexistence, - For device type A, the NR SL module uses the sensing and resource reservation information shared by the LTE SL module to exclude the corresponding resources for its resource selection procedure. - FFS details on how the NR SL module uses this information. - FFS details on how what information the LTE SL module shares the information to the NR SL module and how, exact information shared, timeline etc. - FFS: Other method(s) for device type A to be aware of resources being occupied by LTE SL. - FFS: Whether/how device type B should be supported. |
| Apple | Yes |  |
| Nokia, NSB | Yes |  |
| ETRI | Yes |  |
| Lenovo | Yes |  |
| Spreadtrum | Yes |  |
| WILUS | Yes |  |
| DCM | Yes |  |
| Transsion | Yes |  |
| Sony | Yes |  |
| vivo | Yes |  |
| Huawei, HiSilicon | Yes with comments | We are fine with the direction, but following three FFS should be deleted. - FFS details on how the LTE SL module shares the information to the NR SL module, exact information shared, timeline etc. - FFS: Other method(s) for device type A to be aware of resources being occupied by LTE SL. - FFS: Whether/how device type B should be supported. For the FFS on how the LTE SL module shares the information. It is up to UE internal implementation. The FFS for NR SL module uses this information is sufficient. For the FFS on other method(s), it is not very clear since a Type A UE can obtain all the information including LTE-V sensing, reservation and even measurement results from its LTE-V module. So how to exchange the information is up to UE implementation, and it is not necessary to have further study. For the FFS on type B, as we explained, it is not necessary to have further study, and we suggest to remove the FFS. Thus, we suggest the following modification in green font. Proposal 2-4(I): - For studying the feasibility of dynamic resource sharing as a possible solution for co-channel coexistence, - For device type A, the NR SL module uses the sensing and resource reservation information shared by the LTE SL module to exclude the corresponding resources for its resource selection procedure. - FFS details on how the NR SL module uses this information. - FFS details on how the LTE SL module shares the information to the NR SL module, exact information shared, timeline etc. - FFS: Other method(s) for device type A to be aware of resources being occupied by LTE SL. - FFS: Whether/how device type B should be supported. |
| LGE | Yes with a small modification | To be clear, we suggest to add the yellow-marked part below in the proposal because at this moment all the possibilities for such further enhancement (e.g., nothing else is finally specified/adopted) should be open. Proposal 2-4(I): - For studying the feasibility of dynamic resource sharing as a possible solution for co-channel coexistence, - For device type A, the NR SL module uses the sensing and resource reservation information shared by the LTE SL module to exclude the corresponding resources for its resource selection procedure. - FFS details on how the NR SL module uses this information. - FFS details on how the LTE SL module shares the information to the NR SL module, exact information shared, timeline etc. - FFS: Whether/how to define other method(s) for device type A to be aware of resources being occupied by LTE SL. FFS: Whether/how device type B should be supported. |

### Summary of 3rd Round of Discussions

Summary of Preferences

The following is a summary of the preferences marked by companies on Proposal.

- Companies supporting the proposal: Ericsson (with additions to type B), Panasonic, NEC, OPPO, Xiaomi, QC, Intel, Samsung (delete first FFS), Bosch (additional FFS), Toyota, IDC, MediaTek (comments), Apple, Nokia, ETRI, Lenovo, Spreadtrum, WILUS, DCM, Transsion, Sony, Vivo, HW+HiSi (delete FFSs), LGE (small correction). [24]

- Companies not supporting the proposal: ZTE+Sanechips. [1]

Summary of Comments

Regarding the last FFS bullet, it can be updated depending on the outcome of the discussion on Proposal 1-1 and 1-1b. This should cater to the comments made by ZTE, Ericsson, NEC and Bosch.

With regard to the comment from MediaTek, the NR SL module requires to be aware of the resources that are being used by the LTE SL module in order to avoid using them in a dynamically shared resource pool. Since the exact details of the information shared are FFS, the FL does not see an issue here.

Based on the comments by HW, the FL’s view is that the FFSs would help in a focused discussion in later meetings, even if some of these actions would result in being up to UE implementation.

FL’ Recommendation

Almost all the companies are in favor of supporting the proposal. The FL has made the suggested changes, and the proposal can now be considered for agreement.

Proposal 2-4(II):

- For studying the feasibility of dynamic resource sharing as a possible solution for co-channel coexistence,

  - For device type A, the NR SL module uses the sensing and resource reservation information shared by the LTE SL module.

    - FFS details on how the NR SL module uses this information.

    - FFS details on how the LTE SL module shares the information to the NR SL module, exact information shared, timeline etc.

  - FFS: Whether/how to define other method(s) for device type A to be aware of resources being occupied by LTE SL.

  - FFS: Whether/how device type B should be supported.

### Company Views for 4th Round of Discussions

Based on the inputs from companies in the previous round, would the following proposal be acceptable to the companies?

Proposal 2-4(II):

- For studying the feasibility of dynamic resource sharing as a possible solution for co-channel coexistence,

  - For device type A, the NR SL module uses the sensing and resource reservation information shared by the LTE SL module.

    - FFS details on how the NR SL module uses this information.

    - FFS details on how the LTE SL module shares the information to the NR SL module, exact information shared, timeline etc.

  - FFS: Whether/how to define other method(s) for device type A to be aware of resources being occupied by LTE SL.

  - FFS: Whether/how device type B should be supported.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | Yes | We agree to the FL’s proposal. |
| CMCC | Yes | We agree to the FL’s proposal. |
| Apple | Yes |  |
| InterDigital | Yes |  |
| ZTE,Sanechips | Comments | Still prefer to change the last FFS into FFS: Whether/how the evaluation should take into account device type B |
| Xiaomi | Yes |  |
| DCM | Yes |  |
| Spreadtrum | Yes |  |
| Panasonic | Yes |  |
| Sony | Yes |  |
| Intel | Yes | We are OK with the proposal. |
| OPPO | Yes |  |
| Sharp | Yes |  |
| vivo | Yes |  |
| ETRI | Yes |  |
| Fraunhofer | Yes | We are supportive of the FL’s proposal. With regards to the last FFS, proposal 1-1 handles the case where type B is for further study. If companies have an issue with this FFS, we do not see the need to include it here at all. |
| NEC | Yes |  |
| Samsung | Comment | We think that device B should be supported. A resource pool can include device A and B, therefore we should develop methods for device Type B. Suggest to update the last bullet to: - FFS: Whether/how device type B should be supported. |
| Nokia, NSB | Yes |  |
| Lenovo | Yes |  |
| MediaTek | Yes |  |
| Huawei, HiSilicon | Yes with comments | We are generally fine with the proposal, some additional comments are below. The two FFS under the bullet “For device type A, the NR SL module uses…” can be merged to a single one, for example, as bellowing: - FFS details on how the NR SL module derives and/or uses this information. Whether to support Type B devices is already captured in Proposal 1-1 (III), it is not necessary to repeat it in every proposal. The proposal can be modified as following, changes are in the green font: Proposal 2-4(II): - For studying the feasibility of dynamic resource sharing as a possible solution for co-channel coexistence, - For device type A, the NR SL module uses the sensing and resource reservation information shared by the LTE SL module. - FFS details on how the NR SL module derives and/or uses this information. - FFS details on how the LTE SL module shares the information to the NR SL module, exact information shared, timeline etc. - FFS: Whether/how to define other method(s) for device type A to be aware of resources being occupied by LTE SL. - FFS: Whether/how device type B should be supported. |
| Ericsson | Yes with modification | We propose to include the procedures for type B devices: Proposal 2-4(II): - For studying the feasibility of dynamic resource sharing as a possible solution for co-channel coexistence, - For device type A, the NR SL module uses the sensing and resource reservation information shared by the LTE SL module. - FFS details on how the NR SL module uses this information. - FFS details on how the LTE SL module shares the information to the NR SL module, exact information shared, timeline etc. - FFS: Whether/how to define other method(s) for device type A to be aware of resources being occupied by LTE SL. - FFS: Whether/how device type B should be supported. - For device type B, - FFS: Details and method(s) for device type B to be aware of resources being occupied by LTE SL. |
| Toyota ITC | Yes |  |

### Company Views for 5th Round of Email Discussions

FL’s comment:

All companies are supportive of the main bullet and the first sub-bullet. There are a few companies opposing the inclusion of the last FFS, while one company wants to keep it.

Proposal 2-4(II):

- For studying the feasibility of dynamic resource sharing as a possible solution for co-channel coexistence,

  - For device type A, the NR SL module uses the sensing and resource reservation information shared by the LTE SL module.

    - FFS details on how the NR SL module uses this information.

    - FFS details on how the LTE SL module shares the information to the NR SL module, exact information shared, timeline etc.

  - FFS: Whether/how to define other method(s) for device type A to be aware of resources being occupied by LTE SL.

  - FFS: Whether/how device type B should be supported.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | Yes |  |
| Qualcomm | Yes |  |
| Bosch | Yes |  |
| Apple | Yes |  |
| Intel | Yes |  |
| Panasonic | Yes |  |
| NEC | Yes |  |
| Xiaomi | Yes |  |
| Vivo | Yes |  |
| CMCC | Yes with comments | Type B has mentioned in 1-1, no need to repeat it here |
| Transsion | Yes |  |
| OPPO | Yes |  |
| Continental | Yes |  |
| Nokia, NSB | Yes |  |
| Ericsson | Comment | Our preference is to consider type B without FFS. This would be aligned with Proposal 1-1 (V) above. For the sake of progress, we are fine to leave it FFS and discuss next meeting. |

FL’s comment:

In response to CMCC’s comment, companies in previous rounds insist on including the last bullet. We can revisit based on further comments.

### Summary of 5th Round of Email Discussions

The following version of the proposal was submitted for email approval.

Proposal 2-4(II):

- For studying the feasibility of dynamic resource sharing as a possible solution for co-channel coexistence,

  - For device type A, the NR SL module uses the sensing and resource reservation information shared by the LTE SL module.

    - FFS details on how the NR SL module uses this information.

    - FFS details on how the LTE SL module shares the information to the NR SL module, exact information shared, timeline etc.

  - FFS: Whether/how to define other method(s) for device type A to be aware of resources being occupied by LTE SL.

  - FFS: Whether/how device type B should be supported.

## [ACTIVE] Issue 2-5: Constraints for Dynamic Co-channel Coexistence Solutions

### Company Views for 1st Round of Discussions

Question 2-5: If solution C was selected in Q2-1, in order to limit the scope of the study on dynamic resource sharing, which of the following constraints should be considered?

- Constraint A: Possibility of not configuring PSFCH in shared resource pools

- Constraint B: Limiting the shared resource pool to a single SCS that is supported by both LTE SL and NR SL (15 KHz)

- Other constraints, please describe.

| Company | Solution(s) | Comments |
| --- | --- | --- |
| NTT DOCOMO |  | We think some solution to solve the issues can be discussed rather than agreeing the constraints. But to limit workload, we can accept at least constraint B. |
| Apple |  | Given a limited scope, we are open to both constraint A and constraint B. |
| vivo | Constraint B | It’s too complicated to support coexistence in the mixed SCS case, for example, AGC would be problematic for LTE UE as power of the multiple NR slots overlapping with a LTE subframe may change, and it would also be hard to coordinate LTE SL and NR SL to reduce the collision rate. Hence, it’s better to limit the shared resource pool to a single SCS. PSFCH is one important feature of NR SL to improve efficiency and reliability, NR SL with disabled HARQ-ACK little performance gain compare with LTE SL. |
| Intel |  | The number of scenarios for dynamic coexistence definitely need to be limited. However, we would like to postpone any decision until the benefit of solution C are clearer. |
| Qualcomm | Constraint B | We support limiting the shared resource pool to a single SCS supported by both NR SL and LTE SL (i.e., 15 kHz) as a simplification for the study. HARQ based retransmissions are a fundamental feature of NR SL and disabling PSFCH will essentially imply that only broadcast services will be supported in the cochannel deployment. This will make the NR system (and the applications it can support) extremely restricted. Hence, we do not support Constraint A. We acknowledge that the use of 15 kHz instead of 30 kHz SCS for NR SL has some negative effect on system performance. On the other hand, as we show in the simulation results presented, the effect is quite limited while it greatly simplifies system design. |
| Lenovo | Constraint B | We think that two cases may be considered for dynamic resource sharing: Case 1: All resources in a NR resource pool are shared between LTE sidelink and NR sidelink Case 2: Partial resources in a NR resource pool are shared between LTE sidelink and NR sidelink. On Constraint A we think it is only suitable for case 1, for case 2 we think the PSFCH could be configured if the slots configured with PSFCH are not shared with LTE sidelink. We support Constraint B considering the UE complexity |
| Samsung |  | At this stage, let’s not introduce constraints. We can discuss later. |
| OPPO | Both Constraint A and B | It is beneficial to limit the workload when both constraints are considered. |
| ZTE, Sanechips | At least Constraint A and B | Considering the case the slot with PSFCH is overlapped with LTE sidelink, AGC would be problematic. Besides, in LTE SL, only full slot(with 14 symbols) is supported, therefor NR SL should be restricted not to configure partial slot with less than 14 symbols. In addition, Type A UEs can only be considered. |
| Transsion | Constraint B | Considering the limited TU of this objective, we support Constraint B. |
| InterDigital |  | We prefer to start the study of dynamic solutions without the stated constraints. Both constraints might be potentially resolved in a dynamic solution, e.g., if NR SL UE performs resource selection considering PSFCH resource, then PSFCH transmission will not overlapping with any LTE SL transmission and cause any AGC issue. We can evaluate the solution later and determine whether the complexity and/or processing may be too much and thus constraints should be used to simply the solution. |
| Spreadtrum | Constraint B | Using a single SCS could avoid the complicated design. |
| Sharp | Constraint B | Constraint B can be a baseline for the study. |
| Xiaomi | Constraint B | Although the decision can be made based on the study results in a later stage, we are fine with constraint B. |
| NEC | Constraint B |  |
| Ericsson | Others | Constraint A is too broad. We believe it should be rephrased as “not configuring overlapping time resources for LTE and NR in which PSFCH may be transmitted”. Constraint B needs further study. It is not clear to us if it is necessary or not. Other constraints: partition the NR SL pool in different groups of resources. Different groups of resources may be used depending on the presence/absence of LTE SL Ues. |
| Sony | Constraint B | We support constraint B and we think we can discuss this constraint issue after clarification of solution C. |
| Fraunhofer |  | While we do not prefer having any constraints this early in the discussions, we understand the challenges in defining dynamic resource sharing with the constraints. We can accept Constraint B due to the workload and time limitations. |
| Huawei, HiSilicon | At least A and B (with some comments) | If solution C is deemed necessary, we think constraints to solution C on PSFCH and SCS configurations are necessary to avoid too much specification workload. To avoid introducing a new concept of “share resource pool”, we suggest the following red changes: == - Constraint A: Possibility of not configuring PSFCH in shared resources pools • Constraint B: Limiting the shared resource pool to a single SCS that is supported by both LTE SL and NR SL (15 KHz) . Limit NR SL resource pools to 15kHz SCS when they overlap with LTE-V resource pools. |
| Nokia, NSB | None | We think it is too early to discuss limitations of the scope of solution C. The proposed constraints are not preferable for NR SL operation as PSFCH is an essential part of NR SL, and the ability to use higher SCS than 15kHz is important for scenarios with high Doppler effect. |
| Mitsubishi | None at this point | We should strive to find a solution to those problems first (and restrict only if we are unable to find an appropriate solution) |
| CATT |  | Need to discuss the principle for these restrictions first |
| LGE | Comment | We think that it is not desirable to firstly decide what constraints/study scope limitations should be applied for Solution C before a sufficient investigation of its necessity is conducted. |
| Toyota ITC | None | At this initial stage, it is too early to put restrictions and limit the scope. We should first consider solutions without restrictions. |

### Summary of 1st Round of Discussions

Summary of Preferences

The following is a summary of the preferences marked by companies on the constraints that can be considered for dynamic resource sharing in Rel-18.

- Companies supporting Contraint B: DCM, Vivo, QC, Lenovo, Transsion, Spreadtrum, Sharp, Xiaomi, NEC, Sony. [10]

- Companies supporting no constraints at this point: Intel, SS, IDC, Fraunhofer, Nokia, Mitsubishi, CATT, LGE, Toyota. [9]

- Companies supporting Constraint A and B: Apple, OPPO. [2]

- Companies supporting at least Constraint A and B: ZTE+Sanechips, HW+HiSi. [2]

Summary of Comments

While some companies think that the decision of introducing constraints on the configuration of shared resource pools can be done later, others believe that this would greatly simplify the design and help during the study phase of coexistence in shared resource pools. Although limiting NR to only use 15kHz SCS would results in a clear performance deficit, some companies believe that a resulting simplified system design outweighs the benefit of having a flexible numerology in shared resource pools. Furthermore, some companies who support this constraint also believe that supporting HARQ operation in these pools would show a clear performance advantage over non-HARQ-based LTE systems and thus it should be included in the study. Two companies believes that further constraints should be introduced, e.g., the slot structure of an NR slot (to be not less than 14 symbols) or partitioning of resources based on presence of LTE SL UEs.

FL’ Recommendation

While 9 companies do not prefer to have a constraint for dynamic resource pool sharing at this point, 14 companies support the use of Constraint B, confining the resource pool configuration to a single SCS for the study phase. The possibility of not configuring the PSFCH in dynamic shared resource pools or on shared resources themselves is only supported by 4 companies.

Proposal 2-5:

- For dynamic resource sharing as a possible solution for co-channel coexistence, limit the shared resource pool to a single SCS of 15kHz that is supported by both LTE SL and NR SL.

  - FFS: Whether/how to configure PSFCH within shared resource pools.

### Company Views for 2nd Round of Discussions

Question 2-5a: Based on the inputs from companies in the previous round, would the following proposal be acceptable to the companies?

Proposal 2-5:

- For dynamic resource sharing as a possible solution for co-channel coexistence, limit the shared resource pool to a single SCS of 15kHz that is supported by both LTE SL and NR SL.

  - FFS: Whether/how to configure PSFCH within shared resource pools.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Intel | No | As previously mentioned, given that we just started discussion on this topic, we believe it is too early to place constraints, and we would like to not to estrict ourself at this point. |
| Apple | Yes | We are also fine to keep this open in this meeting. |
| ETRI | No | The restriction seems to be premature. |
| Lenovo | Yes |  |
| Panasonic | Yes |  |
| China Telecom | No | Firstly, we think it need to be more cautious on the limitation on SCSs of 15KHz. In addition, it may be too early to present potential constraints in FFS. The dynamic resource sharing inevitably brings some complexity, especially for Ues equipped with both LTE sidelink module and NR sidelink module. There may need to solve some issues related to PSFCH resources, sensing mechanism and so on. PSFCH is not supported in LTE-V2X but supported in NR-V2X. How to enable PSFCH in dynamic resource allocation needs more discussion. Regarding sensing information, we think they could be exchanged between LTE sidelink module and NR sidelink module for better dynamic resource allocation. NR sidelink module can decode LTE SCI to obtain LTE sidelink information. Other considerations on dynamic resource allocation can be further studied with necessary evaluation results, aiming to provide a better performance with manageable complexity. |
| Sony | Yes |  |
| MediaTek | Comment | Although 15 KHz SCS limitation seems reasonable to us, we think it’s too early to introduce this restriction. |
| CMCC | Yes with comment | As we comments for the semi-static scheme, we are wondering whether we need mention SL-BWP here since SCS is configured tied to BWP not resource pool in NR SL. |
| Spreadtrum | Yes |  |
| ZTE, Sanechips | Comment | Similar views as Intel and MediaTek. |
| OPPO | Yes with comment | We think the shared resource pool ought to be clarified. It means the resource pool of LTE SL and NR SL is fully overlapped or partial overlapped. Maybe we should use “the overlapped resource pools for LTE and NR SL” instead. |
| Fraunhofer | Comment | While we can agree with the proposal, we would prefer to not have such a restriction so early in the study. |
| NEC | Yes |  |
| Transsion | Comment | We share the similar view with some companies that it is premature to have this restriction before we conduct the study. |
| Samsung | Comment | For progress we can accept the proposal with change by saying at least 15 kHz and adding FFS for other SCS - For dynamic resource sharing as a possible solution for co-channel coexistence, limit the shared resource pool at least to a single SCS of 15kHz that is supported by both LTE SL and NR SL. - FFS: Whether/how to support other SCS for NR SL. - FFS: Whether/how to configure PSFCH within shared resource pools. |
| Toyota ITC | No | Since higher SCSs and PSFCH are important features of NR SL, we should first study issues associated with mixed SCS scenarios and PSFCH and then consider potential solutions before limiting the scope at this stage. |
| Bosch | No | We share the concerns of some companies that it may be a bit early to have such restrictions. |
| Nokia, NSB | No | Similar to Proposal 2-3, we can study additional approaches to address the AGC issue, so it is premature to impose this constraint. |
| Xiaomi | Yes with comment | Although we are fine with setting constraint B for the study, we are also open to make decisions later on. |
| CATT/GOHIGH | No | This may depend on the actual schme discussed so it is too early to make the decision. |
| Vivo | No | PSFCH is an essential feature of NR sidelink, so we prefer to keep it. We can discuss later how to enable PSFCH in dynamic resource allocation. Proposal 2-5: - For dynamic resource sharing as a possible solution for co-channel coexistence, - limit the shared resource pool to a single SCS of 15kHz that is supported by both LTE SL and NR SL. - PSFCH can be configured for NR SL within the shared resource pools - FFS: Whether/how to configure PSFCH within shared resource pools. |
| Huawei, HiSilicon | Yes | Similar comment as that for Issue 2-4, whether to support dynamic resource sharing relies on further study, so it is better to use similar wording, i.e. ‘If dynamic resource sharing is considered, then…’. The meaning of “shared resource pool” is not clear. As LTE-V and NR-V already have their own resource pools independently, it is more suitable to refer to they have ‘shared resources (i.e. resource pools are overlapping)’ instead of ‘shared resource pool’. Furthermore, the reason to limit 15kHz SCS for NR SL is the performance loss on LTE-V due to the AGC issues. Specifically, if the larger SCS is supported, e.g. 30kHz, there will be two transmission opportunities for NR-V corresponding to one LTE-V subframe. However, LTE-V UE only performs AGC adjustment once on the starting of the subframe, if there are much more, or much less NR transmissions occur on the second transmission opportunities, the AGC of LTE-V would no longer take effect and the whole TB cannot be received successfully. Therefore, the SCS constraints on the NR-V is necessary. On the other hand, if the PSFCH are configured on the shared resources, it will lead to the collision between LTE-V transmission and NR SL HARQ feedback, the performance of both RATs are reduced. So we think the PSFCH should be precluded on the shared resources At last, we suggest to modify the proposal as following: Proposal 2-5: - ForIf dynamic resource sharing is considered, then as a possible solution for co-channel coexistence, limit the NR SL shared resource pool to a single SCS of 15kHz when the NR SL resource pool overlaps with a LTE-V resource pool that is supported by both LTE SL and NR SL. - FFS: Whether/how to Do not configure PSFCH within shared resource pools. |
| InterDigital | No | As we indicated earlier in the 1st round discussion, it is too early to have such restrictions already. |
| DCM |  | Probably, further study on the constraints is the best approach in this meeting in consideration of companies’ comments. |
| LGE | No | We think that RAN1 doesn’t need to make this kind of conclusion at this meeting before having the sufficient investigation on the necessity of restrictions for each solution. |
| Sharp | Yes |  |
| Ericsson |  | It is not clear in our view that we need to limit the resource pool to a SCS of 15 kHz. We think that we need to study it first and once the issues are clearly understood we can study it. Regarding the FFS bullet, we think that the wording allows for a too broad interpretation, we propose to reword it as follows: - FFS: how to avoid configuring overlapping time resources for LTE and NR in which PSFCH may be transmitted. Moreover, we think that other solutions for the case of dynamic resource sharing should not be precluded. We propose to include the following: - Study other constraints: partition the NR SL pool in different groups of resources. Different groups of resources may be used depending on the presence/absence of LTE SL UEs. Therefore, we propose to modify the proposal as: Proposal 2-5: - For dynamic resource sharing as a possible solution for co-channel coexistence, - FFS whether it is needed limit the shared resource pool to a single SCS of 15kHz that is supported by both LTE SL and NR SL. - FFS: Whether/how to configure PSFCH within shared resource pools. FFS: how to avoid configuring overlapping time resources for LTE and NR in which PSFCH may be transmitted. - Study other constraints: partition the NR SL pool in different groups of resources. Different groups of resources may be used depending on the presence/absence of LTE SL UEs. |
| WILUS | No | We think it is too early to have such a limitation on SCS. We prefer to further study other SCSs without excluding any options at this stage. |

### Summary of 2nd Round of Discussions

Summary of Preferences

The following is a summary of the preferences marked by companies on Proposal 2-5.

- Companies supporting the proposal: Apple (can accept not having the restriction), Lenovo, Panasonic, Sony, CMCC, Spreadtrum, OPPO, NEC, Samsung (add at least), Xiaomi (can accept not having the restriction), Vivo (add sub-bullet stating no restriction for PSFCH), HW+HiSi (agree with FFS), Sharp. [13]

- Companies not supporting the proposal since it is too early to place a restriction on a solution: Intel, ETRI, China Telecom, MediaTek, ZTE+Sanechips, Fraunhofer, Transsion, Toyota, Bosch, Nokia, CATT, IDC, DCM, LGE, Ericsson, Wilus. [16+2]

Summary of Comments

Based on the comments from companies, it seems that introducing any kind of restriction or limitation on dynamic resource sharing based solutions are not favored at this early stage of the study.

Companies have rather identified the possible challenges that face dynamic resource sharing such as mixed SCS scenarios and the enabling of PSFCH in dynamically shared resources. They prefer to further study and evaluate them before applying restrictions.

HW supports both the restrictions and feels that it is one of the solutions to handle the AGC issue brought about in the case of NR SL being configured with a higher SCS as well as not result in collisions between LTE SL transmissions and NR HARQ feedback.

Ericsson on the other hand, prefers to list the restrictions as FFS. They also point to a possible solution where resource are partitioned within a resource pool, where one portion can be used exclusively by NR SL UEs, and a shared class where NR SL UEs can use these resources only if not used by LTE SL UEs. This is primarily to handle the mixed SCS case between NR SL and LTE SL.

OPPO had suggested to clarify that for dynamic resource sharing, the resource pools for LTE SL and NR SL are fully or partially overlapped. CMCC had stated that the SCS configuration is linked to the SL BWP and not to each resource pool, which is as per the Rel-16/17 specifications. The FL agrees with these aspects.

FL’ Recommendation

While 16 companies do not support the introduction of restrictions at such an early stage, 13 companies support the proposal. Hence, the FL recommends to list out the possible aspects that have to be considered by companies for the study on dynamic resource sharing solutions. Companies are encouraged to bring solutions that would include these aspects in RAN1#110.

Proposal 2-5 (I):

- For studying the feasibility of dynamic resource sharing as a possible solution for co-channel coexistence, the following aspects have been identified:

  - Handling of numerologies other than 15kHz

  - Configuration of overlapping time resources for LTE SL and NR SL in slots where NR PSFCH may be transmitted, taking into account the handling of AGC.

  - Mechanisms to avoid dropping of NR SL transmissions impacted by LTE SL transmissions.

  - FFS: Other aspects.

### Company Views for 3rd Round of Discussions

Question 2-5b: Based on the inputs from companies in the previous round, would the following proposal be acceptable to the companies?

Proposal 2-5 (I):

- For studying the feasibility of dynamic resource sharing as a possible solution for co-channel coexistence, the following aspects have been identified:

  - Handling of numerologies other than 15kHz

  - Configuration of overlapping time resources for LTE SL and NR SL in slots where NR PSFCH may be transmitted, taking into account the handling of AGC.

  - Mechanisms to avoid dropping of NR SL transmissions impacted by LTE SL transmissions.

  - FFS: Other aspects.

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE,Sanechips |  | We are not sure of the intention of this proposal Should we declare the feasibility of supporting dynamic solution after all these subbullets are resolved or not. If so, we don’t need it for now. |
| Ericsson | See comment | We are generally fine with the discussion, but it should be clear that we need to study overlaps in general, not just slots with PSFCH resources. Proposal 2-5 (I): - For studying the feasibility of dynamic resource sharing as a possible solution for co-channel coexistence, the following aspects have been identified: - Handling of numerologies other than 15kHz - Configuration of overlapping time resources for LTE SL and NR SL including in slots where NR PSFCH may be transmitted, taking into account the handling of AGC. - Mechanisms to avoid dropping of NR SL transmissions impacted by LTE SL transmissions. - FFS: Other aspects. |
| Panasonic | Yes |  |
| NEC | Yes with comments | It may be more clear to add “FFS” to each sub-bullet. |
| OPPO | Yes |  |
| Xiaomi |  | We think at least synchronization between NR SL and LTE SL shall also be taken into account. |
| CableLabs | Yes with comments | Clarify the requirements by adding FFS to each sub-bullet |
| Qualcomm | No | Similar to Issue 5.3 (semi-static FDM based pool), it would be expedient from a system design point of view to initially focus on studying dynamic resource starting with SCS set to 15 kHz. The same approach should be applied to both potential solutions. Further, the current proposal is much broader than the proposal in the last round. The previous proposal focused on SCS and its impact on AGC settling at the LTE devices. On the other hand, the new proposal attempts to list all possible issues that will need to be addressed in the dynamic resource sharing scenario. We do not believe that having an exhaustive list of issues that need to be addressed with dynamic resource sharing is feasible at this stage of the study and suggest using the proposal from the last round, which followed the same proposal format as the FDM solutions. We propose the following compromise: - For the initial study of dynamic resource sharing as a possible solution for co-channel coexistence, limit the shared resource pool to a single SCS of 15kHz that is supported by both LTE SL and NR SL. - FFS: Whether/How to configure PSFCH within shared resource pools. - FFS: Handling of numerologies other than 15kHz The last FFS on handling of numerologies other than 15 kHz is based on the comments made by Ericsson in the last round of email discussions. |
| Intel | Yes with comments | As other companies, we would rather prefer to have FFS added to each individual sub-bullet. |
| Samsung | Yes with comments | We are fine with the proposal, but we want to delete “taking into account the handling of AGC” There is no need to mention AGC as this could be just one of the examples of issues faced when PSFCH slots overlap with LTE SL slots. |
| Bosch | Comments | We suggest adding: - Complexity of the solution - Whether NR V2X Rel-16 features could be impacted, e.g., HARQ, re-evaluation, pre-emption etc. |
| Toyota ITC | Yes |  |
| InterDigital | Yes |  |
| MediaTek | Ok with comment | We are fine with the proposal, although the intention of the proposal is not clear to us. We can think of different solutions for dynamic resource sharing, hence different issues can be listed for different solutions. With ‘FFS other aspects’ listed in the last sub-bullet, we are OK with the proposal. |
| Apple | Yes |  |
| Nokia, NSB | Yes |  |
| ETRI | Yes |  |
| Lenovo | Yes |  |
| Spreadtrum | Yes |  |
| WILUS | Yes |  |
| DCM | Yes |  |
| Transsion | Yes |  |
| Sony | Yes |  |
| vivo | Yes |  |
| Huawei, HiSilicon | New proposal provided | We are not quite sure the intention of the proposal. Does it mean the listed questions in the FFS should be further studied in the future meetings? However, we think it goes too much details and lack of verification for the method of dynamic resource sharing. So we suggest a new proposal similar as Proposal 2-3(I), which to further study the feasibility of dynamic resource sharing with SL BWP limited to 15kHz SCS So new proposal can be as following. New Proposal 2-5 (I): - For studying the feasibility of dynamic resource sharing as a possible solution for co-channel coexistence, the SL BWP configured with resource pools for NR SL is limited to a SCS of 15 kHz. |
| LGE | Comment | We think that it would be better that the intention of proposal should be what aspects are to be investigated for the feasibility of dynamic resource sharing (i.e., not what points are identified) because RAN1 did not have a change to discuss any relevant details. |

### Summary of 3rd Round of Discussions

Summary of Preferences

The following is a summary of the preferences marked by companies on Proposal.

- Companies supporting the proposal: Ericsson (correction), Panasonic, NEC (add FFS), OPPO, Xiaomi (addition of sync), CableLabs (add FFS), Intel (add FFS), Samsung (delete AGC), Bosch (additions), Toyota, IDC, MediaTek, Apple, Nokia, ETRI, Lenovo, Spreadtrum, WILUS, DCM, Transsion, Sony, Vivo, LGE (comment). [23]

- Companies not supporting the proposal: ZTE+Sanechips, QC, HW+HiSi. [3]

Summary of Comments

The intention of the proposal is to give guidance on what to investigate until the next RAN1 meeting, in RAN1#110, which is supported by many of the companies. Three companies would add FFS to each point, which would be fine from the FL’s point of view. Additional points to be considered in the investigations are synchronization (Xiaomi) and the impact on NR V2X Rel-16 features (Bosch), which is only supported by these companies.

Two companies, QC and HW+HiSi oppose this proposal, and want to limit the study on dynamic resource sharing by constraining the SCS to 15kHz. Without imposing such a constraint, this can be further explored by the first bullet in the revised proposal.

FL’ Recommendation

Taking the comments into consideration, the FL would recommend modifying the proposal as follows:

Proposal 2-5 (II):

- For studying the feasibility of dynamic resource sharing as a possible solution for co-channel coexistence, the following aspects are to be investigated:

  - Handling of numerologies other than 15kHz

  - Configuration of overlapping time resources for LTE SL and NR SL including in slots where NR PSFCH may be transmitted, taking into account the handling of AGC.

  - Mechanisms to avoid dropping of NR SL transmissions impacted by LTE SL transmissions.

  - FFS: Other aspects.

### Company Views for 4th Round of Discussions

Based on the inputs from companies in the previous round, would the following proposal be acceptable to the companies?

Proposal 2-5 (II):

- For studying the feasibility of dynamic resource sharing as a possible solution for co-channel coexistence, the following aspects are to be investigated:

  - Handling of numerologies other than 15kHz

  - Configuration of overlapping time resources for LTE SL and NR SL including in slots where NR PSFCH may be transmitted, taking into account the handling of AGC.

  - Mechanisms to avoid dropping of NR SL transmissions impacted by LTE SL transmissions.

  - FFS: Other aspects.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | No | We do not agree to this proposal. We strongly believe that for progress we need to agree, like the semi-static resource pool partition case, that the initial design should focus on NR SL and LTE SL operating with the same numerology. Following the proposal by Huawei, HiSilicon, in the last round and similar to Proposal 2-3(II), we propose the following compromise: - For studying the feasibility of dynamic resource sharing as a possible solution for co-channel coexistence, the following aspects are to be investigated NR SL is limited to a SCS of 15 kHz, which is the same SCS as LTE SL To address the concerns raised by some companies in previous rounds of discussions, we want to clarify that the 15 kHz SCS is only for the initial study and design and does not preclude (like 2-3(II)) enhancements that can accommodate heterogeneous numerologies (e.g., 15 kHz for LTE and 30 kHz for NR etc.). Further, consistent with our position in Proposal 2-3(II), we propose not list out possible aspects to be investigated as more analysis and study is required to identify an exhaustive list and determine their effects on performance. Other aspects like synchronization, complexity issues etc., had been identified by companies in the last round. There may be many more that may come up during the design and evaluations. Moreover, many of these issues also apply to semi-static resource partitioning. |
| CMCC | Yes | We are generally fine with this proposal. |
| Apple | Yes |  |
| InterDigital | Yes |  |
| ZTE,Sanechips | Yes | We support the proposal |
| Xiaomi |  | We are not sure about the intention of the proposal. We are not sure whether these three bullets are the most urgent issues need to be investigated for the study. |
| DCM | Yes |  |
| Spreadtrum | Yes |  |
| Panasonic | Yes |  |
| Sony | Yes |  |
| Intel | Yes | We are Ok with the proposal |
| OPPO | Yes |  |
| Sharp | Yes |  |
| vivo | comment | In the proposal, only SCS other than 15kHz are requested for investigation. What about 15kHz? Does no further investigation means that 15kHz is already supported? |
| ETRI | Yes |  |
| Fraunhofer | Yes | We are supportive of the FL’s proposal, to serve as a guideline for further discussions in the next meeting. |
| NEC | Yes |  |
| Samsung | Comment | Direction of proposal is fine. However, suggest deleting the third sub-bullet. “Mechanisms to avoid dropping of NR SL transmissions impacted by LTE SL transmissions” can be included in other aspects. |
| Nokia, NSB | Yes |  |
| Lenovo | Yes |  |
| MediaTek | Comment | We suggest including 15KHz SCS case and no PSFCH case in the investigation. We are not sure why the simpler cases are not included in the list. We should not preclude 15KHz SCS case, or PSFCH transmission-free slots. We suggest following revision: - For studying the feasibility of dynamic resource sharing as a possible solution for co-channel coexistence, the following aspects are to be investigated: - Handling of 15 KHz numerology only vs. handling all numerologies other than 15kHz - Configuration of overlapping time resources for LTE SL and NR SL including only in slots where NR PSFCH transmission is not conducted vs. also including in slots where NR PSFCH may be transmitted, taking into account the handling of AGC. - Mechanisms to avoid dropping of NR SL transmissions impacted by LTE SL transmissions. - FFS: Other aspects. |
| Huawei, HiSilicon | No, new proposal provided. | We share similar views with QC that there is no need to list all possible issues to further discuss. Each technical issue companies interested can be proposed in their contributions, and driven by these papers, RAN1 could further study them in the future meetings. Considering the progress, we do not think it is beneficial either to have a quite long list of potential study points. Some basic designs can be agreed to guide the future discussion. As commented by companies, to support SCS other than 15kHz could result in the AGC issue for LTE-V if dynamic resource sharing is used, so we think 15kHz SCS can be agreed as baseline and other issues is up to further studied. Therefore, we re-propose our suggested new proposal in the previous round. QC’s suggestion is also fine with us New Proposal 2-5 (I): - For studying the feasibility of dynamic resource sharing as a possible solution for co-channel coexistence, the SL BWP configured with resource pools for NR SL is limited to a SCS of 15 kHz. |
| CATT/GOHIGH | Yes |  |
| Ericsson | Yes |  |
| Toyota ITC | Yes |  |

### Company Views for 5th Round of Email Discussions

FL’s comment:

While majority of the companies are supportive of the proposal, 2 companies feel that a limitation of 15kHz on the SCS for dynamic resource sharing has to be imposed. In response to MediaTek’s comment, the 2nd bullet includes the effect of overlapping resources in general, and not only for slots with NR PSFCH. The concern regarding the first bullet from MediaTek and Vivo has been taken into account in the revised version.

Proposal 2-5 (III):

- For studying the feasibility of dynamic resource sharing as a possible solution for co-channel coexistence, the following aspects are to be investigated:

  - Handling of numerologies including, and other than, 15kHz

  - Configuration of overlapping time resources for LTE SL and NR SL including in slots where NR PSFCH may be transmitted, taking into account the handling of AGC.

  - Mechanisms to avoid dropping of NR SL transmissions impacted by LTE SL transmissions.

  - FFS: Other aspects.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | Yes | Prefer to delete “Mechanisms to avoid dropping of NR SL transmissions impacted by LTE SL transmissions” or at least make FFS |
| Qualcomm | No | We do not agree to this proposal. We reiterate our stance that it is premature to list out the exact aspects to be investigated for dynamic resource sharing. More importantly, such a list of potential topics to investigate has not been proposed for the semi-static resource pool partitioning solution. As we noted in earlier rounds of discussion, these issues of mixed numerology, AGC, packet dropping, etc., are all present in the semi-static case as well. Thus, to be consistent with the proposals for the study of the semi-static resource pool portioning solution, we propose the following compromise: - For studying the feasibility of dynamic resource sharing as a possible solution for co-channel coexistence, the SL BWP configured with resource pools for NR SL is limited to a SCS of 15 kHz, which is the same SCS as LTE SL |
| Bosch | Yes |  |
| Apple | Yes |  |
| Intel | Yes |  |
| Panasonic | Yes |  |
| NEC | Yes |  |
| Xiaomi | No | We do not support this proposal. As we have stated in the email response, if the intention is to list all the open issues, the list is not complete; if the intention is only on issues related to numerology, it needs to be clarified in the main bullet. |
| Vivo | Yes |  |
| CMCC | Yes |  |
| Transsion | Yes |  |
| OPPO | Yes |  |
| Continental | Yes |  |
| Qualcomm | No | We believe that the following part of the proposal is non-controversial and may be agreed similar to the semi-static case: - For studying the feasibility of dynamic resource sharing as a possible solution for co-channel coexistence, the SL BWP configured with resource pools for NR SL is limited to a SCS of 15 kHz, which is the same SCS as LTE SL. |
| Huawei, HiSilicon | No | We share the views from QC, a formula of proposal for semi-static resource sharing can be reused. To agree the following SCS part first, we think this is less controversial. - For studying the feasibility of dynamic resource sharing as a possible solution for co-channel coexistence, the SL BWP configured with resource pools for NR SL is limited to a SCS of 15 kHz, which is the same SCS as LTE SL. |
| Nokia, NSB | Comment | Would prefer to not drop it. |
| Ericsson | Comment | Our understanding is that this proposal is dropped. We are fine with discussing the subject matter next meeting. |

FL’s comment:

QC and Xiaomi do not support the proposal, while Samsung has suggested some changes. We can stop discussions on this proposal for this meeting, with companies supporting dynamic resource sharing to focus on these aspects.

### Summary of 5th Round of Email Discussions

While Nokia would like to bring the proposal back, HW and QC insist on mirroring the limitation of SCS for dynamically shared resource pools. This was opposed by a significant number of companies in the previous rounds, which is why the proposal was to be considered as a guideline of aspects to be taken into account in the next meeting. But even this proved unsuccessful in the end.

Hence this topic could not be agreed upon in this meeting.

# Summary of Contributions

## Scope of Study and Design Principles

- Solutions to be considered for co-channel coexistence

  - TDM based semi-static co-channel coexistence – Separate resource pools for LTE SL and NR SL in the same channel or band, configured over different time slots.

    - Support: [2/Huawei], [3/Spreadtrum], [4/ZTE], [5/CATT], [6/Vivo], [7/Mitsubishi], [8/China Telecom], [10/NEC], [11/Lenovo], [12/LGE], [14/Xiaomi], [15/Samsung], [16/OPPO], [19/CMCC], [21/Sharp], [22/CAICT], [23/Fraunhofer], [25/MediaTek].

    - Not support: [1/Nokia], [17/InterDigital], [26/Ericsson], [28/QC].

  - FDM based semi static co-channel coexistence – Separate resource pools for LTE SL and NR SL in the same time slots, configured over different channels or bands.

    - Support: [2/Huawei], [4/ZTE], [6/Vivo], [8/China Telecom], [11/Lenovo], [12/LGE], [15/Samsung], [21/Sharp], [22/CAICT], [23/Fraunhofer].

    - Needs further study/Not support: [1/Nokia], [3/Spreadtrum], [5/CATT], [25/MediaTek], [26/Ericsson], [27/Bosch].

  - Dynamic co-channel coexistence – Same resource pool for LTE SL and NR SL, sharing resources across time and frequency.

    - Support: [1/Nokia], [3/Spreadtrum], [7/Mitsubishi], [8/China Telecom], [11/Lenovo], [12/LGE], [13/Sony], [14/Xiaomi], [16/OPPO], [18/Apple], [21/Sharp], [22/CAICT], [23/Fraunhofer], [24/ETRI], [26/Ericsson], [28/QC].

    - Needs further study/Not support: [2/Huawei], [4/ZTE], [5/CATT], [10/NEC], [19/CMCC], [25/MediaTek], [27/Bosch].

- Support of devices based on their capability

  - UEs with dual LTE SL and NR SL modules

    - [1/Nokia], [7/Mitsubishi], [13/Sony], [15/Samsung], [17/InterDigital], [18/Apple], [19/CMCC], [20/DCM], [24/ETRI], [25/MediaTek], [26/Ericsson], [28/QC].

  - UEs with only NR SL modules

    - [1/Nokia], [7/Mitsubishi - Further study required], [20/DCM – Discussion required], [26/Ericsson].

  - UEs with partial dual RAT (partial LTE SL support and full NR SL support)

    - [1/Nokia].

- Mix of operational modes to be considered for co-channel coexistence

  - Mode 1 NR SL + Mode 3 LTE SL: Since the network controls the scheduling and resource allocation, resource overlapping and collisions can be avoided.

    - [7/Mitsubishi], [15/Samsung], [22/CAICT].

  - Mode 1 NR SL + Mode 4 LTE SL

    - [4/ZTE], [7/Mitsubishi], [15/Samsung].

  - Mode 2 NR SL + Mode 3 LTE SL

    - [4/ZTE], [7/Mitsubishi], [15/Samsung].

  - Mode 2 NR SL + Mode 4 LTE SL

    - [4/ZTE], [7/Mitsubishi], [11/Lenovo], [15/Samsung], [18/Apple], [22/CAICT], [24/ETRI], [25/MediaTek].

- No specification changes are allowed for the LTE SL specifications.

  - [3/Spreadtrum], [5/CATT], [8/China Telecom], [10/NEC], [11/Lenovo], [18/Apple], [21/Sharp], [22/CAICT], [24/ETRI], [25/MediaTek], [26/Ericsson], [27/Bosch], [28/QC].

- Reuse the Rel-16 prioritization rules from NR SL.

  - [2/Huawei], [4/ZTE], [5/CATT], [8/China Telecom], [9/Panasonic], [25/MediaTek].

- Simulation parameters/assumptions to be considered

  - Consider periodic and aperiodic traffic scenarios for NR SL and periodic traffic scenarios for LTE SL.

    - [15/Samsung], [21/Sharp], [25/MediaTek].

  - Consider traffic model defined in TR 37.885.

    - [15/Samsung], [25/MediaTek].

  - Consider PRR and PIR as metrics for evaluation.

    - [15/Samsung], [25/MediaTek – Only PRR].

  - Consider different ratios of NR:LTE traffic loads.

    - [21/Sharp].

## Details of Semi-static Co-channel Coexistence

- TDM and FDM based co-channel coexistence

  - Can already be carried out with minimal (if any) specification impact.

    - [1/Nokia], [2/Huawei], [4/ZTE], [5/CATT], [6/Vivo], [7/Mitsubishi], [10/NEC], [11/Lenovo], [18/Apple], [19/CMCC], [25/MediaTek].

  - RRC configurations and pre-configurations of resource pools can be updated OTA or via wired manner.

    - [2/Huawei], [6/Vivo].

  - Resource pool (pre-)configurations cannot be updated in a timely and flexible manner after being first established, hence stifling the transitioning/re-farming of LTE SL resources to NR SL resources.

    - [1/Nokia], [4/ZTE], [5/CATT], [6/Vivo], [7/Mitsubishi], [8/China Telecom], [14/Xiaomi], [16/OPPO], [18/Apple], [24/ETRI], [26/Ericsson], [28/QC].

- Drawbacks of TDM based co-channel coexistence

  - Causes latency delays

    - [1/Nokia], [12/LGE], [16/OPPO], [23/Fraunhofer].

  - DFN/SFN alignment is required between LTE SL and NR SL.

    - [1/Nokia].

- Drawbacks of FDM based co-channel coexistence

  - AGC settings of LTE SL UEs are impacted by power variation issues due to the presence of PSFCH symbols in an NR SL time slot.

    - [1/Nokia], [16/OPPO].

  - Resource pools with different SCS for NR SL and LTE SL would result in AGC issues.

    - [1/Nokia], [5/CATT], [21/Sharp].

  - FDMed resource pools for NR SL and LTE SL would result in half-duplex issues.

    - [3/Spreadtrum], [12/LGE].

## Details of Dynamic Resource Sharing

- For UEs with both LTE SL and NR SL modules, the NR SL module should be made aware internally of sensing results/candidate resource sets/selected resources used by the LTE SL module.

  - [2/Huawei], [3/Spreadtrum], [4/ZTE], [7/Mitsubishi], [8/China Telecom], [11/Lenovo], [13/Sony], [14/Xiaomi], [15/Samsung], [16/OPPO], [17/InterDigital], [18/Apple], [19/CMCC], [20/DCM], [22/CAICT], [24/ETRI], [25/MediaTek], [26/Ericsson], [28/QC].

- For UEs with both LTE SL and NR SL modules, the NR SL module should avoid/exclude resources indicated to be used by the LTE SL module.

  - [2/Huawei], [4/ZTE], [11/Lenovo], [13/Sony], [15/Samsung], [16/OPPO], [17/InterDigital], [25/MediaTek], [28/QC].

- NR SL UEs should be aware of resources where LTE SL UEs are transmitting by sensing/decoding the SCIs transmitted by the LTE SL UEs.

  - [3/Spreadtrum], [8/China Telecom], [14/Xiaomi], [22/CAICT], [25/MediaTek].

- NR SL UEs should be aware of resources where LTE SL UEs are transmitting indirectly by using RSSI measurements on the resources used for the transmissions.

  - [22/CAICT], [26/Ericsson].

- NR SL module should be aware of resources where LTE SL UEs are transmitting by receiving an explicit signaling from the LTE module about the resources that are being used for the transmissions.

  - [15/Samsung].

- UEs with both NR SL and LTE SL modules should use inter-UE coordination messages to inform other UEs (that may or may not have the LTE SL module) about resources that are occupied by LTE SL transmissions.

  - [13/Sony], [15/Samsung], [17/InterDigital], [18/Apple], [20/DCM].

- For UEs with both LTE SL and NR SL modules, when NR SL modules are operating in Mode 1, they should report the resource usage by LTE SL in the shared resource pool to the gNB.

  - [12/LGE], [13/Sony], [18/Apple].

- Different classes of resources should be defined within a resource pool, where one class can be used exclusively by NR SL UEs, and a shared class where NR SL UEs can use only if LTE SL UEs are not using them.

  - [26/Ericsson].

- Effect of PSFCH resources should be studied in shared resource pools used by both LTE SL and NR SL UEs, including the possibility of not defining PSFCH in these resource pools.

  - [2/Huawei], [3/Spreadtrum], [4/ZTE], [5/CATT], [6/Vivo], [7/Mitsubishi], [8/China Telecom], [11/Lenovo], [12/LGE], [19/CMCC], [22/CAICT], [23/Fraunhofer], [25/MediaTek].

- Slot and subframe boundaries between LTE SL and NR SL should be aligned.

  - [1/Nokia], [15/Samsung], [24/ETRI].

- Shared resource pools should be restricted to the same SCS, like only 15 KHz SCS.

  - [2/Huawei], [4/ZTE], [6/Vivo], [11/Lenovo], [12/LGE], [19/CMCC], [25/MediaTek].

- Resource pools with different SCS for NR SL and LTE SL would result in half-duplex issues.

  - [2/Huawei].

- Shared resource pools should be restricted in terms of their channel bandwidth and subchannel configurations.

  - [6/Vivo].

- AGC impact should be studied for dynamic co-channel coexistence.

  - [1/Nokia], [2/Huawei], [4/ZTE], [11/Lenovo], [25/MediaTek].

- Mechanisms to avoid dropping of NR SL transmissions impacting LTE SL transmissions should be studied, beyond applying the Rel-16 in-device coexistence framework.

  - [1/Nokia], [28/QC].

- Study the necessity of synchronization procedures to be updated for supporting co-channel coexistence.

  - [1/Nokia], [6/Vivo], [14/Xiaomi].

- Study priority-based framework for each RAT (LTE SL and NR SL) for supporting co-channel coexistence.

  - [23/Fraunhofer], [29/Continental].

## Observations from Simulation Results

- Comparison between Solution A (TDM based resource pools), Solution B (FDM based resource pools) and Solution C (dynamically shared resource pools)

  - Low traffic density for LTE SL UEs and medium traffic density for NR SL UEs (1:2) in an urban scenario, for both periodic and aperiodic traffic scenarios, there is no clear performance difference for the overall system.

    - [2/Huawei].

  - Medium traffic density for LTE SL UEs and low traffic density for NR SL UEs (2:1) in an urban scenario, for both periodic and aperiodic traffic scenarios, there is no clear performance difference for the overall system.

    - [2/Huawei].

- Comparison between semi-statically configured resource pools (Solution A and B) and dynamically shared resource pools (Solution C)

  - Equal density for LTE SL UEs and NR SL UEs (1:1) in an urban scenario, for periodic traffic scenarios, Solution C has a 0.5% performance loss for LTE SL UEs.

    - [6/Vivo].

  - Low traffic density for LTE SL UEs and high traffic density for NR SL UEs (1:3) in an urban scenario, for periodic traffic scenarios, Solution C has a ~3% performance loss for LTE SL UEs.

    - [6/Vivo].

  - Equal density for LTE SL UEs and NR SL UEs (1:1) in an urban scenario, for periodic traffic scenarios, Solution C has a 5% performance gain for NR SL UEs.

    - [6/Vivo].

  - Low traffic density for LTE SL UEs and high traffic density for NR SL UEs (1:3) in an urban scenario, for periodic traffic scenarios, Solution C has negligible performance gain for NR SL UEs.

    - [6/Vivo].

- Comparison between different favors of dynamically shared resource pools (Solution C) – in option 1, only NR devices detect resources reserved by LTE SL UEs; in option 2, both LTE and NR SL UEs detect resources reserved by the other. In both cases, the UE will avoid collisions based on the knowledge of the reserved resources.

  - Option 2 has negligible performance loss for LTE SL UEs.

    - [6/Vivo].

  - Option 2 has a 5% performance gain for LTE SL UEs.

    - [6/Vivo].

- Comparison between Solution A (TDM based resource pools) and Solution C (dynamically shared resource pools)

  - Equal density for LTE SL UEs and NR SL UEs (1:1) in freeway scenario A, for periodic traffic scenarios, there is no clear performance difference for LTE and NR SL UEs.

    - [16/OPPO].

  - Equal density for LTE SL UEs and NR SL UEs (1:1) in freeway scenario A, for periodic traffic for LTE SL UEs and aperiodic for NR SL UEs, there is no clear performance difference for LTE SL UEs and Solution A has a 2% performance loss for NR SL UEs.

    - [16/OPPO].

  - Varying traffic density for LTE SL UEs and NR SL UEs in an urban scenario, for periodic traffic scenarios, Solution A has performance loss of one RAT and the performance gain for another RAT compared with Solution C. It is hard to find a proper TDM ratio between the NR and LTE resource pool in urban scenarios to avoid the performance loss of NR-based or LTE-based V2X RAT.

    - [16/OPPO].

- Comparison between Rel-16 in-device coexistence and dynamically shared resource pools, where based on the priorities of the transmissions, it drops NR SL or LTE SL transmissions and receptions.

  - Medium traffic density for LTE SL UEs and low traffic density for NR Rel-16 SL UEs (2:1) in a highway scenario, for periodic traffic for LTE SL UEs and aperiodic for NR SL UEs, when Rel-16 in-device coexistence mechanism is used for coexistence, the performance of the lower priority RAT is severely degraded by up to 50%. Dynamically shared resource pools strikes a balance between NR SL and LTE SL.

    - [28/QC].

  - Equal density for LTE SL UEs and NR SL UEs (1:1) in a highway scenario, for periodic traffic for LTE SL UEs and aperiodic for NR SL UEs, when Rel-16 in-device coexistence mechanism is used for coexistence, the performance of the lower priority RAT is severely degraded by up to 50%. Dynamically shared resource pools strikes a balance between NR SL and LTE SL.

    - [28/QC].

## Others

- Resources should be used by UEs in a detect-and-vacate manner, where transmissions from one RAT of lower priority (LTE SL or NR SL) can be dropped in favor of transmissions from the other of higher priority.

  - Support: [12/LGE – Trigger re-selection instead of dropping], [27/Bosch].

  - Not support: [26/Ericsson], [28/QC].

# References

- R1-2203124	On Co-channel Coexistence for LTE Sidelink and NR Sidelink	Nokia, Nokia Shanghai Bell

- R1-2203148	Co-channel coexistence for LTE sidelink and NR sidelink	Huawei, HiSilicon

- R1-2203333	Discussion on Co-channel coexistence for LTE sidelink and NR sidelink	Spreadtrum Communications

- R1-2203367	Study on co-channel coexistence for LTE sidelink and NR sidelink	ZTE, Sanechips

- R1-2203463	Discussion on co-channel coexistence for LTE sidelink and NR sidelink	CATT, GOHIGH

- R1-2203563	Co-channel coexistence for LTE sidelink and NR sidelink	vivo

- R1-2203642	On sidelink co-channel coexistence issues	Mitsubishi Electric RCE

- R1-2203658	Discusson on co-channel coexistence for LTE sidelink and NR sidelink	China Telecom

- R1-2203668	Discussion on Sidelink Co-channel Coexistence 	Panasonic Holdings Corporation

- R1-2203695	Co-existence between LTE and NR sidelink	NEC

- R1-2203705	Discussion on co-channel coexistence for LTE sidelink and NR sidelink	Lenovo

- R1-2203715	Discussion on co-channel coexistence for LTE sidelink and NR sidelink	LG Electronics

- R1-2203736	Discussion on co-channel coexistence for LTE sidelink and NR sidelink	Sony

- R1-2203820	Discussion on co-channel coexistence for LTE and NR sidelink	xiaomi

- R1-2203908	On co-channel coexistence for LTE sidelink and NR sidelink	Samsung

- R1-2203984	Discussion on co-channel coexistence for LTE and NR V2X	OPPO

- R1-2204049	Co-channel coexistence for LTE sidelink and NR sidelink	InterDigital, Inc.

- R1-2204249	On co-channel coexistence for LTE sidelink and NR sidelink	Apple

- R1-2204308	Discussion on co-channel coexistence for LTE sidelink and NR sidelink	CMCC

- R1-2204384	Discussions on co-channel coexistence of LTE-SL and NR-SL	NTT DOCOMO, INC.

- R1-2204429	Discussions on co-channel coexistence for LTE sidelink and NR sidelink	Sharp

- R1-2204585	Considerations on co-channel coexistence for LTE SL and NR SL	CAICT

- R1-2204609	Discussion on Co-Channel Coexistence for LTE and NR Sidelink	Fraunhofer HHI, Fraunhofer IIS

- R1-2204652	Discussion on co-channel coexistence for LTE sidelink and NR sidelink	ETRI

- R1-2204720	On co-channel coexistence between LTE sidelink and NR sidelink	MediaTek Inc.

- R1-2204740	Co-channel coexistence between LTE sidelink and NR sidelink	Ericsson

- R1-2204867	On co-channel coexistence between LTE and NR sidelink	ROBERT BOSCH GmbH

- R1-2205035	Co-channel Coexistence Between LTE SL and NR SL	Qualcomm Incorporated

- R1-2205119	General considerations on Coexistence between LTE- and NR-sidelink	Continental Automotive GmbH