3GPP TSG RAN WG1 #109-e	        		   R1-2205650

# e-Meeting, May 9th – 20th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | 0323 | rev |  | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on sidelink enhancements in NR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh-Core |  |  |  |  |  | Date: |  |  | 2022-05-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - For HARQ-ACK information, a priority value of a PSFCH transmission is denoted as i and value range of i should be from 1 to 8. For conflict information, a priority value of a PSFCH transmission is denoted as i-8, and the range of i should be from 9 to 16 because the value range of the priority value is from 1 to 8 in clause 16.2.3. - Clarify the values of reserved bits for SCI format 1-A in clause 16.4. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify the value range of i for priority values of PSFCH with HARQ-ACK information or with conflict information in clause 16.2.3. - Capture reference to TS 38.212 for values of reserved bits in SCI format 1-A in clause 16.4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete support for sidelink enhancements in NR. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.2.3, 16.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS 38.212 |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## *** Unchanged text is omitted ***

### 16.2.3	PSFCH

## *** Unchanged text is omitted ***

-	else

-	the UE autonomously selects Nmax,PSFCH PSFCH transmissions with ascending order of corresponding priority field values as described in clause 16.2.4.2

-	if PPSFCH,one+10log10Nmax,PSFCH≤PCMAX, where PCMAX is determined for the Nmax,PSFCH PSFCH transmissions according to [8-1, TS 38.101-1]

-	NTx,PSFCH=Nmax,PSFCH and PPSFCH,k(i)=PPSFCH,one [dBm]

-	else

-	the UE autonomously selects NTx,PSFCH PSFCH transmissions in ascending order of corresponding priority field values as described in clause 16.2.4.2 over the PSFCH transmissions with HARQ-ACK information, if any, and then with ascending order of priority value over the PSFCH transmissions with conflict information, if any, such that NTx,PSFCH≥max1,i=1KMi where Mi, 1≤i≤8, is a number of PSFCHs with priority value i for PSFCH with HARQ-ACK information and Mi, i>8M, is a number of PSFCHs with priority value i-8 for PSFCH with conflict information and K is defined as

-	the largest value satisfying PPSFCH,one+10log10max1,i=1KMi≤PCMAX where PCMAX is determined according to [8-1, TS 38.101-1] for transmission of all PSFCHs in i=1KMi, if any

-	zero, otherwise

and

PPSFCH,k(i)=minPCMAX-10log10(NTx,PSFCH),PPSFCH,one [dBm]

where PCMAX is determined for the  NTx,PSFCH simultaneous PSFCH transmissions according to [8-1, TS 38.101-1]

-	else

PPSFCH,k(i)=PCMAX-10log10(NTx,PSFCH) [dBm]

where the UE autonomously determines NTx,PSFCH PSFCH transmissions with ascending order of corresponding priority field values as described in clause 16.2.4.2 over the PSFCH transmissions with HARQ-ACK information, if any, and then with ascending order of priority value over the PSFCH transmissions with conflict information, if any, such that NTx,PSFCH≥1 and where PCMAX is determined for the NTx,PSFCH PSFCH transmissions according to [8-1, TS 38.101-1].

## *** Unchanged text is omitted ***

## 16.4	UE procedure for transmitting PSCCH

## *** Unchanged text is omitted ***

A UE that transmits a PSCCH with SCI format 1-A using sidelink resource allocation mode 1 [6, TS 38.214] sets

-	the values of the frequency resource assignment field and the time resource assignment field for the SCI format 1-A transmitted in the m-th resource for PSCCH/PSSCH transmission provided by a dynamic grant or by a SL configured grant, where m= 1,…,M and M is the total number of resources for PSCCH/PSSCH transmission provided by a dynamic grant or the number of resources for PSCCH/PSSCH transmission in a period provided by a SL configured grant type 1 or SL configured grant type 2, as follows:

-	the frequency resource assignment field and time resource assignment field indicate the m-th to M-th resources as described in [6, TS 38.214].

For decoding of a SCI format 1-A, a UE may assume that a number of bits provided by sl-NumReservedBits can have any value as described in [4, TS 38.212].

## *** Unchanged text is omitted ***