3GPP TSG-WG1 Meeting # 109-e	R1-2205665

# e-Meeting, May 9th - May 20th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 1419 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to NB-IoT/eMTC support for Non-Terrestrial Networks |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Motorola Mobility |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN-Core |  |  |  |  |  | Date: |  |  | 2022-05-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Corrections needed on time synchronization and timing relationship enhancement features of Rel-17 NB-IoT/eMTC support for NTN: - The formula for Delaycommont that provides how a UE interprets/uses the Common TA related parameters and computes the NTA,adjcommon is missing in clause 4.2.3 for eMTC, and clause 16.1.2 for NB-IoT. - As TDD was not treated during the IoT NTN WI, TDD clauses in the spec should not be changed because of NTN. - Clarify how to calculate indices for subframes that UE is not required to monitor for NPDCCH in NTN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add formula for Delaycommont in clause 4.2.3 and 16.1.2 - Clause 10.2, 16.4.2, 16.5.1: Remove all references to Koffset from the TDD clauses. - Clause 16.6: Separate out the description for NB-IoT NTN to avoid affecting legacy TN behavior. For NB-IoT NTN, clarify that DL subframes not required to be monitored are those that overlap with the previously described transmission UL subframes. Offset end of period when UE is not expected to monitor by Kmac to reflect NTN RTT. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect operation of Rel-17 NB-IoT/eMTC support for NTN features. - Incomplete specification on how a UE interprets/uses the Common TA related parameters and computes the NTA,adjcommon. - Release 17 IoT NTN does not support TDD so clauses would wrongly imply that it does and potentially confuse implementers. - Moderate potential for misinterpretation of which subframes in NB-IoT NTN are not required to be monitored. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 4.2.3, 10.2, 16.1.2, 16.4.2, 16.5.1, 16.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanged parts are omitted>

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

ACK	Acknowledgement

AUL	Autonomous Uplink

AUL-DFI	AUL downlink feedback information

BCH	Broadcast Channel

CCE	Control Channel Element

CDD	Cyclic Delay Diversity

CG	Cell Group

CIF	Carrier Indicator Field

CQI	Channel Quality Indicator

CRC	Cyclic Redundancy Check

CRI	CSI-RS Resource Indicator

CSI	Channel State Information

CSI-IM	CSI-interference measurement

DAI	Downlink Assignment Index

DC	Dual Connectivity

DCI	Downlink Control Information

DL	Downlink

DL-SCH	Downlink Shared Channel

DTX	Discontinuous Transmission

EDT	Early Data Transmission

EN-DC	E-UTRA NR Dual Connectivity with MCG using E-UTRA and SCG using NR

EPDCCH	Enhanced Physical Downlink Control Channel

EPRE	Energy Per Resource Element

MCG	Master Cell Group

MCS	Modulation and Coding Scheme

NACK	Negative Acknowledgement

NE-DC	NR E-UTRA Dual Connectivity with MCG using NR and SCG using E-UTRA

NPBCH	Narrowband Physical Broadcast CHannel

NPDCCH	Narrowband Physical Downlink Control CHannel

NPDSCH	Narrowband Physical Downlink Shared CHannel

NPRACH	Narrowband Physical Random Access CHannel

NPUSCH	Narrowband Physical Uplink Shared CHannel

NPSS	Narrowband Primary Synchronization Signal

NSSS	Narrowband Secondary Synchronization Signal

NRS	Narrowband Reference Signal

NTN	Non-Terrestrial Network

PBCH	Physical Broadcast Channel

PCFICH	Physical Control Format Indicator Channel

PDCCH	Physical Downlink Control Channel

PDSCH	Physical Downlink Shared Channel

PHICH	Physical Hybrid ARQ Indicator Channel

PMCH	Physical Multicast Channel

PMI	Precoding Matrix Indicator

PRACH	Physical Random Access Channel

PRS	Positioning Reference Signal

PRB	Physical Resource Block

PSBCH	Physical Sidelink Broadcast Channel

PSCCH	Physical Sidelink Control Channel

PSCell	Primary Secondary cell

PSDCH	Physical Sidelink Discovery Channel

PSSCH	Physical Sidelink Shared Channel

PSSS	Primary Sidelink Synchronisation Signal

PUCCH	Physical Uplink Control Channel

PUCCH-SCell	PUCCH SCell

PUR	Preconfigured Uplink Resource

PUSCH	Physical Uplink Shared Channel

PTI	Precoding Type Indicator

RBG	Resource Block Group

RE	Resource Element

RI	Rank Indication

RS	Reference Signal

RSS	Resynchronization Signal

SCG	Secondary Cell Group

SINR	Signal to Interference plus Noise Ratio

SPS C-RNTI	Semi-Persistent Scheduling C-RNTI

SR	Scheduling Request

SRS	Sounding Reference Symbol

SSSS	Secondary Sidelink Synchronisation Signal

TAG	Timing Advance Group

TBS	Transport Block Size

UCI	Uplink Control Information

UE	User Equipment

UL	Uplink

UL-SCH	Uplink Shared Channel

VRB	Virtual Resource Block

<Unchanged parts are omitted>

### 4.2.3	Transmission timing adjustments

Upon reception of a timing advance command or a timing adjustment indication for a TAG containing the primary cell or PSCell, the UE shall adjust uplink transmission timing for PUCCH/PUSCH/SRS of the primary cell or PSCell based on the received timing advance command or a timing adjustment indication.

The UL transmission timing for PUSCH/SRS of a secondary cell is the same as the primary cell if the secondary cell and the primary cell belong to the same TAG. If the primary cell in a TAG has a frame structure type 1 and a secondary cell in the same TAG has a frame structure type 2 or frame structure 3, UE may assume that NTA ≥ 624.

If the UE is configured with a SCG, the UL transmission timing for PUSCH/SRS of a secondary cell other than the PSCell is the same as the PSCell if the secondary cell and the PSCell belong to the same TAG.

Upon reception of a timing advance command or a timing adjustment indication for a TAG not containing the primary cell or PSCell, if all the serving cells in the TAG have the same frame structure type, the UE shall adjust uplink transmission timing for PUSCH/SRS of all the secondary cells in the TAG based on the received timing advance command or a timing adjustment indication where the UL transmission timing for PUSCH /SRS is the same for all the secondary cells in the TAG.

Upon reception of a timing advance command or a timing adjustment indication for a TAG not containing the primary cell or PSCell, if a serving cell in the TAG has a different frame structure type compared to the frame structure type of another serving cell in the same TAG, the UE shall adjust uplink transmission timing for PUSCH/SRS of all the secondary cells in the TAG by using NTAoffset = 624 regardless of the frame structure type of the serving cells and based on the received timing advance command or a timing adjustment indication where the UL transmission timing for PUSCH /SRS is the same for all the secondary cells in the TAG. NTAoffset is described in [3].

The timing adjustment indication specified in [11] indicates the initial NTA used for a TAG.The timing advance command for a TAG indicates the change of the uplink timing relative to the current uplink timing for the TAG as multiples of 16. The start timing of the random access preamble is specified in [3].

In case of random access response, an 11-bit timing advance command [8], TA, for a TAG indicates NTA values by index values of TA = 0, 1, 2, ..., 256 if the UE is configured with a SCG, and TA = 0, 1, 2, ..., 1282 otherwise, where an amount of the time alignment for the TAG is given by NTA = TA 16. NTA is defined in [3].

In other cases, a 6-bit timing advance command [8] or the Timing advance adjustment field in DCI format 6-0A/B if present [4], TA, for a TAG indicates adjustment of the current NTA value, NTA,old, to the new NTA value, NTA,new, by index values of TA = 0, 1, 2,..., 63, where NTA,new = NTA,old + (TA 31)16. Here, adjustment of NTA value by a positive or a negative amount indicates advancing or delaying the uplink transmission timing for the TAG by a given amount respectively.

For a non-BL/CE UE, for a timing advance command received on

-	subframe n, the corresponding adjustment of the uplink transmission timing shall apply from the beginning of subframe n+5 if the UE is configured with higher layer parameter shortProcessingTime and the corresponding PDCCH with CRC scrambled by C-RNTI is in the UE-specific search space, n+6 otherwise.

-	slot n, the corresponding adjustment of the uplink transmission timing shall apply from the first subframe boundary no earlier than slot [n+8].

-	subslot n, the corresponding adjustment of the uplink transmission timing shall apply from the first subframe boundary no earlier than

-	subslot [n+16] if higher layer parameter proc-TimeAdv-r15= 'nplus4set1'.

-	subslot [n+18] if higher layer parameter proc-TimeAdv-r15= 'nplus6set1'or 'nplus6set2'.

-	subslot [n+20] if higher layer parameter proc-TimeAdv-r15= 'nplus8set2'.

For serving cells in the same TAG, when the UE's uplink PUCCH/PUSCH/SRS transmissions in subframe n and subframe n+1 are overlapped due to the timing adjustment, the UE shall complete transmission of subframe n and not transmit the overlapped part of subframe n+1.

For a BL/CE UE, for a timing advance command received on subframe n, the corresponding adjustment of the uplink transmission timing shall apply for the uplink PUCCH/PUSCH/SRS transmissions in subframe n+6+Koffset. When the BL/CE UE's uplink PUCCH/PUSCH/SRS transmissions in subframe n and subframe n+1 are on the same narrowband and are overlapped due to the timing adjustment, the UE shall complete transmission of subframe n and is not required to transmit in subframe n+1 until the first available symbol that has no overlapping portion with subframe n. When the BL/CE UE's uplink PUCCH/PUSCH/SRS transmissions in subframe n and subframe n+1 are on different narrowbands, and the timing adjustment occurs in the guard period for narrowband retuning, the UE is not required to transmit in subframe n+1 until the first available symbol that has no overlapping portion with subframe n and which does not reduce the guard period. The value of Koffset is given by,

-	if the UE is configured with the higher layer parameter CellSpecificKoffset,

-	Koffset= Kcell_offset-KUE_offset where

Kcell_offset is the parameter CellSpecificKoffset provided by higher layers, and

KUE_offset is the parameter UESpecificKoffset provided by higher layers, otherwise KUE_offset=0

-	otherwise,

-	Koffset=0.

If the received downlink timing changes and is not compensated or is only partly compensated by the uplink timing adjustment without timing advance command as specified in [10], the UE changes NTA accordingly.

For a BL/CE UE in a NTN serving cell, using serving satellite higher-layer ephemeris parameters, if configured, the BL/CE UE determines NTA,adjUE (defined in [3]) using the serving satellite position and its own position to pre-compensate the two-way transmission delay on the service link. To pre-compensate the two-way transmission delay between the uplink time synchronization reference point and the serving satellite, the BL/CE UE determines NTA,adjcommon (defined in [3]) based on one-way propagation delay Delaycommont which can be obtained as:

Delaycommont=12NTAcommon+NTAcommonDrift×t-tepoch+NTAcommonDriftVariation×t-tepoch2

where NTAcommon, NTAcommonDrift, and NTAcommonDriftVariation are given by the higher layer parameters nta-Common, nta-CommonDrift, and nta-CommonDriftVariation respectively, and tepoch is the epoch time given by the higher layer parameter epochTime. Delaycommon(t) provides a distance at time t between the serving satellite and the uplink time synchronization reference point divided by the speed of light. The uplink time synchronization reference point is the point where DL and UL are frame aligned with an offset given by NTA,offset.

<Unchanged parts are omitted>

## 10.2	Uplink HARQ-ACK timing

<Unchanged parts are omitted>

For TDD, a BL/CE UE shall upon detection of a PDSCH within subframe(s) n-k-Koffset, where  and  is defined in Table 10.1.3.1-1 intended for the UE and for which HARQ-ACK response shall be provided, transmit the HARQ-ACK response using the same  derived according to Clause 10.1.3.1 in subframe(s) n+ki with i =0,1, …, N-1, where

-	subframe n-k is the last subframe in which the PDSCH is transmitted; and

-	0≤k0<k1<…,kN-1 and the value of and  is provided by higher layers parameter pucch-NumRepetitionCE-format1, if configured, otherwise it is provided by higher layer parameter pucch-NumRepetitionCE-Msg4-Level0-r13, pucch-NumRepetitionCE-Msg4-Level1-r13, pucch-NumRepetitionCE-Msg4-Level2-r13 or pucch-NumRepetitionCE-Msg4-Level3-r13 depending on whether the most recent PRACH coverage enhancement level for the UE is 0, 1, 2 or 3, respectively; and

if N>1

-	subframe(s) n+ki with i=0,1,…,N-1 are N consecutive BL/CE UL subframe(s) immediately after subframe n-1, and the set of BL/CE UL subframes are configured by higher layers;

otherwise

-	k0 =0

except if the UE is configured with higher layer parameter ce-PDSCH-MultiTB-Config and multiple TB are scheduled in the corresponding DCI.

For TDD, if a BL/CE UE is configured with higher layer parameter ce-PDSCH-MultiTB-Config and multiple TBs are scheduled in the corresponding DCI, the BL/CE UE shall upon detection of a PDSCH intended for the UE and for which HARQ-ACK response shall be provided, transmit the HARQ-ACK response using the same  derived according to Clause 10.1.3.1 in subframe(s) sb+ki+Koffset with b=0,1,…,B-1,  i =0,1, …, N-1, where

-	B is the number of TB bundles

-	if the UE is not configured with higher layer parameter harq-AckBundling in ce-PDSCH-MultiTB-Config, B=NTB with bundle b consisting of only TB b.

-	Else, the value of B and the corresponding TBs in each bundle is determined according to clause 7.3

-	is the number of scheduled TB determined in the corresponding DCI;

-	s0=n0+4, sb=max {nb+4,  sb-1+Nb-1}, b≠0

-	nb is the last subframe in which the PDSCH containing TB bundle b is transmitted;

-	Nb denotes the number of consecutive subframes including subframes that are not BL/CE UL subframes where the PUCCH with HARQ ACK for TB bundle b with repetition number of N is transmitted;

and

-	0≤k0<k1<…,kN-1 and the value of and  is provided by higher layers parameter pucch-NumRepetitionCE-format1, if configured, otherwise it is provided by higher layer parameter pucch-NumRepetitionCE-Msg4-Level0-r13, pucch-NumRepetitionCE-Msg4-Level1-r13, pucch-NumRepetitionCE-Msg4-Level2-r13 or pucch-NumRepetitionCE-Msg4-Level3-r13 depending on whether the most recent PRACH coverage enhancement level for the UE is 0, 1, 2 or 3, respectively; and

-	subframe(s) sb+ki+Koffset with i=0,1,…,N-1 for TB bundle b are N consecutive BL/CE UL subframe(s) immediately after subframe sb-1+Koffset, and the set of BL/CE UL subframes are configured by higher layers.

The uplink timing for the ACK corresponding to a detected PDCCH/EPDCCH indicating downlink SPS release shall be the same as the uplink timing for the HARQ-ACK corresponding to a detected PDSCH, as defined above.

For a BL/CE UE, the uplink timing for the ACK corresponding to a detected MPDCCH indicating downlink SPS release shall be the same as the uplink timing for the HARQ-ACK corresponding to a detected PDSCH, as defined above.

For a BL/CE UE, if a first HARQ-ACK transmission associated to a first set of PDSCH partially collides with a second HARQ-ACK transmission associated to a second set of PDSCH transmissions, the last PDSCH of the first set of PDSCH transmissions being detected before the last PDSCH of the second set of PDSCH transmissions, the UE shall drop the second HARQ-ACK transmission.

<Unchanged parts are omitted>

### 16.1.2	Timing synchronization

Upon reception of a timing advance command, the UE shall adjust uplink transmission timing for NPUSCH, and SR if configured with higher layer parameter sr-WithoutHARQ-ACK-Config, based on the received timing advance command.

The timing advance command indicates the change of the uplink timing relative to the current uplink timing as multiples of 16. The start timing of the random access preamble is specified in [3].

In case of random access response, an 11-bit timing advance command [8], TA, indicates NTA values by index values of TA = 0, 1, 2, ..., 1536, where an amount of the time alignment is given by NTA = TA 16. NTA is defined in [3].

In other cases, a 6-bit timing advance command [8] or the Timing advance adjustment field in DCI format N0 if present [4], TA, indicates adjustment of the current NTA value, NTA,old, to the new NTA value, NTA,new, by index values of TA = 0, 1, 2,..., 63, where NTA,new = NTA,old + (TA 31)16. Here, adjustment of NTA value by a positive or a negative amount indicates advancing or delaying the uplink transmission timing by a given amount respectively.

For a timing advance command reception ending in DL subframe n, the corresponding adjustment of the uplink transmission timing shall apply from the first available NB-IoT uplink slot following the end of n+12 DL subframe and the first available NB-IoT uplink slot is the first slot of a NPUSCH transmission. When the UE's uplink NPUSCH transmissions in NB-IoT uplink slot n and NB-IoT uplink slot n+1 are overlapped due to the timing adjustment, the UE shall complete transmission of NB-IoT uplink slot n and not transmit the overlapped part of NB-IoT uplink slot n+1.

If the received downlink timing changes and is not compensated or is only partly compensated by the uplink timing adjustment without timing advance command as specified in [10], the UE changes NTA accordingly.

For a UE in a NTN serving cell, using serving satellite higher-layer ephemeris parameters, if configured, the UE determines NTA,adjUE (defined in [3]) using the serving satellite position and its own position to pre-compensate the two-way transmission delay on the service link. To pre-compensate the two-way transmission delay between the uplink time synchronization reference point and the serving satellite, the UE determines NTA,adjcommon (defined in [3]) based on one-way propagation delay Delaycommont which can be obtained as:

Delaycommont=12NTAcommon+NTAcommonDrift×t-tepoch+NTAcommonDriftVariation×t-tepoch2

where NTAcommon, NTAcommonDrift, and NTAcommonDriftVariation are given by the higher layer parameters nta-Common, nta-CommonDrift, and nta-CommonDriftVariation respectively, and tepoch is the epoch time given by the higher layer parameter epochTime. Delaycommon(t) provides a distance at time t between the serving satellite and the uplink time synchronization reference point divided by the speed of light. The uplink time synchronization reference point is the point where DL and UL are frame aligned with an offset given by NTA,offset.

<Unchanged parts are omitted>

### 16.4.2	UE procedure for reporting ACK/NACK

The UE shall upon detection of a NPDSCH transmission ending in NB-IoT subframe n intended for the UE and for which an ACK/NACK shall be provided, start, after the end of

-	 DL subframe for FDD,

-	 NB-IoT UL subframes following the end of n+12Koffset subframe for TDD,

transmission of the NPUSCH carrying ACK/NACK response, and SR (if any) if the serving cell is FDD and the UE is configured with higher layer parameter sr-with-HARQ-ACK-Config, using NPUSCH format 2 in N consecutive NB-IoT UL slots, where

-	, where

-	the value of is given by the higher layer parameter ack-NACK-NumRepetitions-Msg4 configured for the associated NPRACH resource for Msg4 NPDSCH transmission, and higher layer parameter ack-NACK-NumRepetitions otherwise,

-	the value of  is the number of slots of the resource unit (defined in clause 10.1.2.3 of [3]), and

-	if the UE is configured with higher layer parameter harq-ACK-Bundling in npdsch-MultiTB-Config, then , otherwise , where the value of is determined by the Number of scheduled TB for Unicast field if present in the NPDCCH corresponding to the NPDSCH, otherwise ,

-	allocated subcarrier for ACK/NACK and value of k0 is determined by the ACK/NACK resource field in the DCI format of the corresponding NPDCCH according to Table 16.4.2-1, and Table 16.4.2-2,

-	for FDD, .

-	for TDD, .

-	For

-	if the UE is configured with higher layer parameter harq-AckBundling in npdsch-MultiTB-Config, and the NPDSCH corresponding to a NPDCCH with DCI CRC scrambled by C-RNTI,

-	the ACK/NACK response is generated by performing a logical AND operation of HARQ-ACKs corresponding to the TBr+1 ,

-	otherwise,

-	NB-IoT UL slots  with  of the NPUSCH carry ACK/NACK response for TBr+1 ,

<Unchanged parts are omitted>

### 16.5.1	UE procedure for transmitting format 1 narrowband physical uplink shared channel

NPUSCH format 1 transmission can be scheduled by a NPDCCH with DCI format N0, or the transmission can correspond to using preconfigured uplink resource configured by higher layers. Transmission using preconfigured uplink resource is initiated by higher layers as specified in [14] , while retransmission of transport blocks transmitted using preconfigured uplink resource are scheduled by a NPDCCH with DCI format N0.

A UE shall upon detection on a given serving cell of a NPDCCH with DCI format N0 ending in NB-IoT DL subframe n scheduling NPUSCH intended for the UE, perform, at the end of

-	n+k0+Koffset DL subframe for FDD,

-	k0 NB-IoT UL subframes following the end of n+8 subframe for TDD,

a corresponding NPUSCH transmission using NPUSCH format 1 in N consecutive NB-IoT UL slots ni with i = 0, 1, …, N-1 according to the NPDCCH information where

-	subframe n is the last subframe in which the NPDCCH is transmitted and is determined from the starting subframe of NPDCCH transmission and the DCI subframe repetition number field in the corresponding DCI; and

-	, where the value of is determined by the repetition number field in the corresponding DCI (see Clause 16.5.1.1), the value of is determined by the resource assignment field in the corresponding DCI (see Clause 16.5.1.1), the value of  is the number of NB-IoT UL slots of the resource unit (defined in clause 10.1.2.3 of [3]) corresponding to the  allocated number of subcarriers (as determined in Clause 16.5.1.1) in the corresponding DCI, and the value of is determined by the Number of scheduled TB for Unicast field, if present, in the corresponding DCI,  otherwise

-	n0 is the first NB-IoT UL slot starting after the end of subframe n+k0+Koffset for FDD

-	n0 is the first NB-IoT UL slot starting after k0 NB-IoT UL subframes following the end of n+8 subframe for TDD

-	value of k0 is determined by the scheduling delay field () in the corresponding DCI according to Table 16.5.1-1 for FDD and Table 16.5.1-1A for TDD

-	For ,

-	if the UE is configured with higher layer parameter npusch-MultiTB-Config set to 'interleaved', and NPUSCH corresponding to a NPDCCH with DCI CRC scrambled by C-RNTI, and  where  for ,  otherwise.

-	NB-IoT UL slots  with  are associated with TBr+1 ,

-	otherwise,

-	NB-IoT UL slots  with  are associated with TBr+1 ,

<Unchanged parts are omitted>

## 16.6	Narrowband physical downlink control channel related procedures

Throughout this clause, if a NB-IoT UE is configured with higher layer parameter k-Mac, Kmac = k-Mac otherwise, Kmac = 0.

<Unchanged parts are omitted>

For a NPDCCH UE-specific search space, if a NB-IoT UE is configured with higher layer parameter twoHARQ-ProcessesConfig or npusch-MultiTB-Config and if the NB-IoT UE detects NPDCCH with DCI Format N0 ending in subframe n, and if the corresponding NPUSCH format 1 transmission starts from n+k or in a NTN serving cell, from an uplink subframe which, after accounting for uplink transmission timing, overlaps with downlink subframe n+k,

-	if the corresponding NPDCCH with DCI format N0 with CRC scrambled by C-RNTI schedules two transport blocks as determined by the Number of scheduled TB for Unicast field if present, the UE is not required to monitor an NPDCCH candidate in any subframe starting from subframe n+1 to subframe n+k-1, otherwise the UE is not required to monitor an NPDCCH candidate in any subframe starting from subframe n+k-2 to subframe n+k-1; and

- the UE does not expect to receive a DCI Format N0 before subframe n+k-2 for which the corresponding NPUSCH format 1 transmission ends later than subframe n+k+255 if the corresponding NPDCCH with DCI format N0 schedules one transport block.

-	for TDD, and if the corresponding NPUSCH format1 transmission ends in subframe n+m, the UE is not required to monitor NPDCCH in any subframe starting from subframe n+ k to subframe n+m-1.

otherwise

-	if the NB-IoT UE detects NPDCCH with DCI Format N0 ending in subframe n or receives a NPDSCH carrying a random access response grant ending in subframe n, and if the corresponding NPUSCH format 1 transmission starts from n+k or in a NTN serving cell, from an uplink subframe which, after accounting for uplink transmission timing, overlaps with downlink subframe n+k, the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+k-1.

-	for TDD, if the NB-IoT UE detects NPDCCH with DCI Format N0 ending in subframe n or receives a NPDSCH carrying a random access response grant ending in subframe n, and if the corresponding NPUSCH format 1 transmission ends in n+k, the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+k.

For a NPDCCH UE-specific search space, if a NB-IoT UE is configured with higher layer parameter twoHARQ-ProcessesConfig or npdsch-MultiTB-Config

-	and if the NB-IoT UE detects NPDCCH with DCI Format N1 ending in subframe n, and if a NPDSCH transmission starts from n+k,

-	if the corresponding NPDCCH with DCI format N1 with CRC scrambled by C-RNTI schedules two transport blocks as determined by the Number of scheduled TB for Unicast field if present, the UE is not required to monitor an NPDCCH candidate in any subframe starting from subframe n+1 to subframe n+k-1;

-	otherwise, the UE is not required to monitor an NPDCCH candidate in any subframe starting from subframe n+k-2 to subframe n+k-1;

otherwise

-	if the NB-IoT UE detects NPDCCH with DCI Format N1 or N2 ending in subframe n, and if the corresponding NPDSCH transmission starts from n+k, the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+k-1.

If a NB-IoT UE detects NPDCCH with DCI Format N1 ending in subframe n, and if the corresponding NPDSCH transmission starts from n+k, and

-	for FDD, if the corresponding NPUSCH format 2 transmission starts from subframe n+m or in a NTN serving cell, from an uplink subframe which, after accounting for uplink transmission timing, overlaps with downlink subframe n+m, the UE is not required to monitor NPDCCH in any subframe starting from subframe n+ k to subframe n+m-1.

-	for TDD, if the corresponding NPUSCH format 2 transmission ends in subframe n+m the UE is not required to monitor NPDCCH in any subframe starting from subframe n+ k to subframe n+m-1.

If a NB-IoT UE detects NPDCCH with DCI Format N1 for "PDCCH order" ending in subframe n, and

-	for FDD, if the corresponding NPRACH transmission starts from subframe n+k or in a NTN serving cell, from an uplink subframe which, after accounting for uplink transmission timing, overlaps with downlink subframe n+k, the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+k-1.

-	for TDD, if the corresponding NPRACH transmission ends in subframe n+k, the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+k-1.

If a NB-IoT UE is configured with higher layer parameter twoHARQ-ProcessesConfig

-	and if the UE has a NPUSCH transmission ending in subframe n,

-	the UE is not required to receive transmissions in the Type B half-duplex guard periods as specified in [3]for FDD ; and

-	the UE is not expected to receive an NPDCCH with DCI format N0/N1 for the same HARQ process ID as the NPUSCH transmission in any subframe starting from subframe n+1 to subframe n+3 or in a NTN serving cell, in any downlink subframe that overlaps with uplink subframe n+1 to subframe n+Kmac+3;

else if the UE is not using higher layer parameter edt-Parameters or if the UE is using higher layer parameter edt-Parameters and

-	if the NB-IoT UE has a NPUSCH transmission ending in subframe n , the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+3 or in a NTN serving cell, in any downlink subframe that overlaps with uplink subframe n+1 to subframe n+Kmac+3.

otherwise,

-	If the NB-IoT UE has a NPUSCH transmission for Msg3 ending in subframe n'with transport block size , whereas if would have been selected the NPUSCH transmission would have ended in subframe n, the UE is not required to monitor NPDCCH in any subframe starting from subframe n'+1 to subframe n+3 or in a NTN serving cell, in any downlink subframe that overlaps with uplink subframe n'+1 to subframe n+Kmac+3.

If a NB-IoT UE receives a NPDSCH transmission ending in subframe n, and if the UE is not required to transmit a corresponding NPUSCH format 2, the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+12.

If a NB-IoT UE is configured with higher layer parameter twoHARQ-ProcessesConfig

-	the UE is not required to monitor an NPDCCH candidate of an NPDCCH search space if the candidate ends in subframe n, and if the UE is configured to monitor NPDCCH candidates of another NPDCCH search space having starting subframe k0 before subframe n+5

otherwise

-	the UE is not required to monitor NPDCCH candidates of an NPDCCH search space if an NPDCCH candidate of the NPDCCH search space ends in subframe n, and if the UE is configured to monitor NPDCCH candidates of another NPDCCH search space having starting subframe k0 before subframe n+5.

An NB-IoT UE is not required to monitor NPDCCH candidates of an NPDCCH search space during an NPUSCH UL gap.

An NB-IoT UE is not required to monitor NPDCCH candidates of a Type2A-NPDCCH common search space during the scheduling gap or the processing gap.

For an NB-IoT UE configured with higher layer parameter sr-WithoutHARQ-ACK-Config, if the transmission of a narrowband random access preamble for SR ends on subframe n,

-	in case of frame structure type 1 with NPRACH format 0 and 1 when the number of NPRACH repetitions is greater than or equal to 64, or NPRACH format 2 when the number of NPRACH repetitions is greater than or equal to 16, the UE is not required to monitor NPDCCH UE-specific search space from subframe n to subframe n+40 or in a NTN serving cell, in any downlink subframes that overlap with uplink subframe n to subframe n+Kmac+40,

-	otherwise, the UE is not required to monitor NPDCCH UE-specific search space from subframe n to subframe n+3 or in a NTN serving cell, in any downlink subframes that overlap with uplink subframe n to subframe n+Kmac+3.

<Unchanged parts are omitted>