3GPP TSG-RAN WG1 Meeting #109-e	R1-2205677

# e-Meeting, May 9th – 20th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | 0286 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on eIAB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB_enh-Core |  |  |  |  |  | Date: |  |  | 2022-05-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For clause 5.2.2.5, in RAN1#108-e meeting (email discussion [109-e-R17-eIAB-02] summarized in R1-2205515), it was agreed that the IAB-MT shall consider the provided DL TX power adjustment in addition to Pc when deriving CSI feedback: Agreement The IAB-MT shall account for the provided DL TX power adjustment (as indicated by the parent-node) in addition to Pc when deriving CSI feedback.. However, in the list of UE assumptions on CSI calculation in TS38.214 Clause 2.3.1.1, the agreed IAB-MT behavior for CSI feedback with the provided DL TX power adjustment is not captured under the ratio of PDSCH EPRE to CSI-RS EPRE. |
| Summary of change: |  | In clause 5.2.2.5, it is explicitly stated that the IAB-MT shall account for the provided DL TX power adjustment (as indicated by the parent-node) when deriving CSI feedback in TS38.214 Clause 2.3.1.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguous IAB-MT behavior when deriving CSI feedback if a DL TX power adjustment is provided from the parent node in addition to Pc when deriving CSI feedback. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<omitted text>

#### 5.2.2.5	CSI reference resource definition

The CSI reference resource for a serving cell is defined as follows:

-	In the frequency domain, the CSI reference resource is defined by the group of downlink physical resource blocks corresponding to the band to which the derived CSI relates.

-	In the time domain, the CSI reference resource for a CSI reporting in uplink slot n' is defined by a single downlink slot n-nCSI_ref-Koffset⋅2μDL2μKoffset, where Koffset is a parameter configured by higher layer as specified in clause 4.2 of [6 TS 38.213], and where μKoffsetis the subcarrier spacing configuration for Koffset with a value of 0 for frequency range 1,

-	where  +Nslot,offset,ULCA2μoffset,UL-Nslot,offset,DLCA2μoffset,DL∙2μDL and and  are the subcarrier spacing configurations for DL and UL, respectively, and Nslot, offsetCA and  are determined by higher-layer configured ca-SlotOffset for the cells transmitting the uplink and downlink, as defined in clause 4.5 of [4, TS 38.211]

-	where for periodic and semi-persistent CSI reporting

-	if a single CSI-RS/SSB resource is configured for channel measurement nCSI_ref is the smallest value greater than or equal to 4⋅2µDL, such that it corresponds to a valid downlink slot, or

-	if multiple CSI-RS/SSB resources are configured for channel measurement nCSI_ref is the smallest value greater than or equal to , such that it corresponds to a valid downlink slot.

-	where for aperiodic CSI reporting, if the UE is indicated by the DCI to report CSI in the same slot as the CSI request, nCSI_ref is such that the reference resource is in the same valid downlink slot as the corresponding CSI request, otherwise nCSI_ref is the smallest value greater than or equal to , such that slot n- nCSI_ref corresponds to a valid downlink slot, where Z' corresponds to the delay requirement as defined in Clause 5.4.

-	when periodic or semi-persistent CSI-RS/CSI-IM or SSB is used for channel/interference measurements, the UE is not expected to measure channel/interference on the CSI-RS/CSI-IM/SSB whose last OFDM symbol is received up to Z' symbols before transmission time of the first OFDM symbol of the aperiodic CSI reporting.

A slot in a serving cell shall be considered to be a valid downlink slot if:

-	it comprises at least one higher layer configured downlink or flexible symbol, and

-	it does not fall within a configured measurement gap for that UE

If there is no valid downlink slot for the CSI reference resource corresponding to a CSI Report Setting in a serving cell, CSI reporting is omitted for the serving cell in uplink slot n'.

After the CSI report (re)configuration, serving cell activation, BWP change, or activation of SP-CSI, the UE reports a CSI report only after receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement no later than CSI reference resource and drops the report otherwise.

When DRX is configured, the UE reports a CSI report only if receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement in DRX Active Time no later than CSI reference resource and drops the report otherwise. When DRX is configured and the CSI-RS Resource Set for channel measurement corresponding to a CSI report is configured with two Resource Groups and N Resource Pairs, as described in clause 5.2.1.4.1, the UE reports a CSI report only if receiving at least one CSI-RS transmission occasion for each CSI-RS resource in a Resource Pair within the same DRX Active Time no later than CSI reference resource and drops the report otherwise. When the UE is configured to monitor DCI format 2_6 and if the UE configured by higher layer parameter ps-TransmitOtherPeriodicCSI to report CSI with the higher layer parameter reportConfigType set to 'periodic' and reportQuantity set to quantities other than 'cri-RSRP', 'ssb-Index-RSRP', 'cri-RSRP-Capability[Set]Index', and 'ssb-Index-RSRP-Capability[Set]Index ' when drx-onDurationTimer is not started, the UE shall report CSI during the time duration indicated by drx-onDurationTimer in DRX-Config also outside active time according to the procedure described in Clause 5.2.1.4 if receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement during the time duration indicated by drx-onDurationTimer in DRX-Config outside DRX active time or in DRX Active Time no later than CSI reference resource and drops the report otherwise. When the UE is configured to monitor DCI format 2_6 and if the UE configured by higher layer parameter ps-TransmitPeriodicL1-RSRP to report L1-RSRP with the higher layer parameter reportConfigType set to 'periodic' and reportQuantity set to 'cri-RSRP', 'ssb-Index-RSRP', 'cri-RSRP-Capability[Set]Index', or 'ssb-Index-RSRP-Capability[Set]Index' when drx-onDurationTimer is not started, the UE shall report L1-RSRP during the time duration indicated by drx-onDurationTimer in DRX-Config also outside active time according to the procedure described in clause 5.2.1.4 and when reportQuantity set to 'cri-RSRP' or 'cri-RSRP-Capability[Set]Index' if receiving at least one CSI-RS transmission occasion for channel measurement during the time duration indicated by drx-onDurationTimer in DRX-Config outside DRX active time or in DRX Active Time no later than CSI reference resource and drops the report otherwise.

When deriving CSI feedback, the UE is not expected that a NZP CSI -RS resource for channel measurement overlaps with CSI-IM resource for interference measurement or NZP CSI -RS resource for interference measurement.

If configured to report CQI index, in the CSI reference resource, the UE shall assume the following for the purpose of deriving the CQI index, and if also configured, for deriving PMI and RI:

-	The first 2 OFDM symbols are occupied by control signaling.

-	The number of PDSCH and DM-RS symbols is equal to 12.

-	The same bandwidth part subcarrier spacing configured as for the PDSCH reception

-	The bandwidth as configured for the corresponding CQI report.

-	The reference resource uses the CP length and subcarrier spacing configured for PDSCH reception

-	No resource elements used by primary or secondary synchronization signals or PBCH.

-	Redundancy Version 0.

-	The ratio of PDSCH EPRE to CSI-RS EPRE is as given in Clause 5.2.2.3.1.

-	In addition, the IAB-MT shall account for the provided DL TX power adjustment, if indicated for the slot of the CSI reference resource by DL Tx Power Adjustment MAC CE as described in [10, TS 38.321].

-	Assume no REs allocated for NZP CSI-RS and ZP CSI-RS.

-	Assume the same number of front-loaded DM-RS symbols as the maximum front-loaded symbols configured by the higher layer parameter maxLength in DMRS-DownlinkConfig.

-	Assume the same number of additional DM-RS symbols as the additional symbols configured by the higher layer parameter dmrs-AdditionalPosition.

-	Assume the PDSCH symbols are not containing DM-RS.

-	Assume PRB bundling size of 2 PRBs.

-	The PDSCH transmission scheme where the UE may assume that PDSCH transmission would be performed with up to 8 transmission layers as defined in Clause 7.3.1.4 of [4, TS 38.211]. For CQI calculation, the UE should assume that PDSCH signals on antenna ports in the set [1000,…, 1000+ν-1] for ν layers would result in signals equivalent to corresponding symbols transmitted on antenna ports [3000,…, 3000+P-1], as given by

y3000(i)⋯y3000+P-1(i)=W(i)x0(i)⋯xν-1(i)

where  is a vector of PDSCH symbols from the layer mapping defined in Clause 7.3.1.4 of [4, TS 38.211],  is the number of CSI-RS ports. If only one CSI-RS port is configured, W(i) is 1. If the higher layer parameter reportQuantity in CSI-ReportConfig for which the CQI is reported is set to either 'cri-RI-PMI-CQI' or 'cri-RI-LI-PMI-CQI', W(i) is the precoding matrix corresponding to the reported PMI applicable to x(i). If the higher layer parameter reportQuantity in CSI-ReportConfig for which the CQI is reported is set to 'cri-RI-CQI', W(i) is the precoding matrix corresponding to the procedure described in Clause 5.2.1.4.2. If the higher layer parameter reportQuantity in CSI-ReportConfig for which the CQI is reported is set to 'cri-RI-i1-CQI', W(i) is the precoding matrix corresponding to the reported i1 according to the procedure described in Clause 5.2.1.4.2. The corresponding PDSCH signals transmitted on antenna ports [3000,…,3000 + P - 1] would have a ratio of EPRE to CSI-RS EPRE equal to the ratio given in Clause 5.2.2.3.1. If the higher layer parameter reportQuantity in CSI-ReportConfig for which the CQI is reported is set to either 'cri-RI-PMI-CQI' or 'cri-RI-LI-PMI-CQI', the corresponding CSI-RS Resource Set for channel measurement is configured with two Resource Groups and N Resource Pairs, as described in clause 5.2.1.4.1, the reported CRI corresponds to an entry of the N Resource Pairs, and the reported rank combination is {ν1,ν2}, as described in clause 5.2.1.4.2, for CQI calculation, the UE should assume that

-	PDSCH signals on antenna ports in the set [1000,…,1000+ν1-1] for ν1 layers would result in signals equivalent to corresponding symbols transmitted on antenna ports [3000,…,3000+P-1] of the Group 1 CSI-RS resource in the Resource Pair indicated by the CRI, and PDSCH signals on antenna ports in the set [1000+ν1,…,1000+ν1+ν2-1] for ν2 layers would result in signals equivalent to corresponding symbols transmitted on antenna ports [3000,…,3000+P-1] of the Group 2 CSI-RS resource in the Resource Pair indicated by the CRI, as given by

yj3000(i)⋯yj3000+P-1(i)=Wjix(j-1)⋅ν1i⋯xν1+(j-1)⋅ν2-1i

where Wj(i), j=1,2 are the two precoding matrices corresponding to the two reported PMIs applicable to x(i), as described in clause 5.2.1.4.2; and the indices j=1,2 are associated to the two Resource Groups configured in the corresponding CSI-RS Resource Set for channel measurement; that the signals yj, j=1,2, fully overlap in time and frequency, and that, for the calculation of RI, PMI and LI (if configured) of νj layers, j=1,2, the interference from the other νj mod 2+1 layers is derived from channel measurement and precoding matrix corresponding to the other νj mod 2+1 layers.

-	The UE shall assume that the corresponding PDSCH signals for νj layers transmitted on the P antenna ports of the CSI-RS resource in Group j would have a ratio of EPRE to CSI-RS EPRE equal to the powerControlOffset of the respective CSI-RS resource, for j=1,2.

<omitted text>