3GPP TSG-RAN WG1 Meeting #109-e 	R1-2205687

e-Meeting, May 9-20, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.212 | CR | 0115 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-16 editorial corrections for TS 38.212 (mirrored to Rel-17) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-Core 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2022-05-23 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Correction on higher-layer parameter per the outcome of email discussion [109-e-R16-NR-U-01]. Misalignment of parameter name between TS 38.212 and TS 38.331. - Correction on higher-layer parameter per the outcome of email discussion [109-e-R16-V2X-03]. - Corrections on some typos in clause 8.3.1.1 and clause 8.3.2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Capture the corrections as shown in R1-2203298. - Capture the correction as shown in section 2 of R1-2205197. - Corrections on some typos in clause 8.3.1.1 and clause 8.3.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification is incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.1.1.1, 7.3.1.1.2, 7.3.1.2.1, 7.3.1.2.2, 7.3.1.4.2, 8.3.1.1, 8.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 7.3.1.1.1	Format 0_0

DCI format 0_0 is used for the scheduling of PUSCH in one cell.

The following information is transmitted by means of the DCI format 0_0 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI:

-	Identifier for DCI formats – 1 bit

-	The value of this bit field is always set to 0, indicating an UL DCI format

< Unchanged parts are omitted >

-	ChannelAccess-CPext – 2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4, or Table 7.3.1.1.1-4A if channelAccessMode-r16 = "semiStatic" is provided, for operation in a cell with shared spectrum channel access; 0 bit otherwise.

-	Padding bits, if required.

< Unchanged parts are omitted >

The following information is transmitted by means of the DCI format 0_0 with CRC scrambled by TC-RNTI:

-	Identifier for DCI formats – 1 bit

-	The value of this bit field is always set to 0, indicating an UL DCI format

< Unchanged parts are omitted >

-	ChannelAccess-CPext – 2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4, or Table 7.3.1.1.1-4A if channelAccessMode-r16 = "semiStatic" is provided, for operation in a cell with shared spectrum channel access; 0 bit otherwise

-	Padding bits, if required.

< Unchanged parts are omitted >

Table 7.3.1.1.1-4A: Channel access type & CP extension if channelAccessMode-r16 = "semiStatic" is provided

| Bit field mapped to index | Channel Access Type | The CP extension T_"ext" index defined in Clause 5.3.1 of [4, TS 38.211] | Initiator of the channel occupancy associated with the UL transmission as described in Clause x.x in TS 37.213 |
| --- | --- | --- | --- |
| 0 | No sensing as defined in Clause 4.3 in TS 37.213 | 0 | gNB |
| 1 | No sensing as defined in Clause 4.3 in TS 37.213 | 2 | gNB |
| 2 | Sensing within a 25us interval as defined in Clause 4.3 in TS 37.213 | 0 | gNB |
| 3 | Sensing as defined in Clause 4.3.1.2 in TS 37.213 | 0 | UE |
| Note: Row index 3 is only applicable if ue-SemiStaticChannelAccessConfig is provided. Otherwise, the row is reserved. |  |  |  |

##### 7.3.1.1.2	Format 0_1

DCI format 0_1 is used for the scheduling of one or multiple PUSCH in one cell, or indicating CG downlink feedback information (CG-DFI) to a UE.

The following information is transmitted by means of the DCI format 0_1 with CRC scrambled by C-RNTI or CS-RNTI or SP-CSI-RNTI or MCS-C-RNTI:

-	Identifier for DCI formats – 1 bit

-	The value of this bit field is always set to 0, indicating an UL DCI format

< Unchanged parts are omitted >

-	ChannelAccess-CPext-CAPC – 0, 1, 2, 3, 4, 5 or 6 bits. The bitwidth for this field is determined as log2(I) bits, where I is the number of entries in the higher layer parameter ul-AccessConfigListDCI-0-1 or in Table 7.3.1.1.1-4A if channelAccessMode-r16 = "semiStatic" is provided, for operation in a cell with shared spectrum channel access; otherwise 0 bit. One or more entries from Table 7.3.1.1.2-35 or Table 7.3.1.1.2-35A are configured by the higher layer parameter ul-AccessConfigListDCI-0-1.

< Unchanged parts are omitted >

##### 7.3.1.2.1	Format 1_0

DCI format 1_0 is used for the scheduling of PDSCH in one DL cell.

The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI:

-	Identifier for DCI formats – 1 bits

-	The value of this bit field is always set to 1, indicating a DL DCI format

< Unchanged parts are omitted >

-	ChannelAccess-CPext – 2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4, or Table 7.3.1.1.1-4A if channelAccessMode-r16 = "semiStatic" is provided, for operation in a cell with shared spectrum channel access; 0 bits otherwise

< Unchanged parts are omitted >

The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by TC-RNTI:

-	Identifier for DCI formats – 1 bit

-	The value of this bit field is always set to 1, indicating a DL DCI format

< Unchanged parts are omitted >

-	ChannelAccess-CPext – 2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4, or Table 7.3.1.1.1-4A if channelAccessMode-r16 = "semiStatic" is provided, for operation in a cell with shared spectrum channel access; otherwise 0 bit

< Unchanged parts are omitted >

##### 7.3.1.2.2	Format 1_1

DCI format 1_1 is used for the scheduling of one or multiple PDSCH in one cell.

The following information is transmitted by means of the DCI format 1_1 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI:

-	Identifier for DCI formats – 1 bits

-	The value of this bit field is always set to 1, indicating a DL DCI format

< Unchanged parts are omitted >

-	ChannelAccess-CPext – 0, 1, 2, 3 or 4 bits. The bitwidth for this field is determined as log2(I) bits, where I is the number of entries in the higher layer parameter ul-AccessConfigListDCI-1-1 or in Table 7.3.1.1.1-4A if channelAccessMode-r16 = "semiStatic" is provided, for operation in a cell with shared spectrum channel access; otherwise 0 bit. One or more entries from Table 7.3.1.2.2-6 or Table 7.3.1.2.2-6A are configured by the higher layer parameter ul-AccessConfigListDCI-1-1.

< Unchanged parts are omitted >

##### 7.3.1.4.2	Format 3_1

DCI format 3_1 is used for scheduling of LTE PSCCH and LTE PSSCH in one cell.

The following information is transmitted by means of the DCI format 3_1 with CRC scrambled by SL Semi-Persistent Scheduling V-RNTI:

-	Timing offset – 3 bits determined by higher layer parameter sl-TimeOffsetEUTRA-List, as defined in clause 16.6 of [5, TS 38.213]

-	Carrier indicator –3 bits as defined in 5.3.3.1.9A of [11, TS 36.212].

-	Lowest index of the subchannel allocation to the initial transmission -  bits as defined in 5.3.3.1.9A of [11, TS 36.212].

-	Frequency resource location of initial transmission and retransmission, as defined in 5.3.3.1.9A of [11, TS 36.212]

-	Time gap between initial transmission and retransmission, as defined in 5.3.3.1.9A of [11, TS 36.212]

-	SL index – 2 bits as defined in 5.3.3.1.9A of [11, TS 36.212]

-	SL SPS configuration index – 3 bits as defined in clause 5.3.3.1.9A of [11, TS 36.212].

-	Activation/release indication – 1 bit as defined in clause 5.3.3.1.9A of [11, TS 36.212].

If the UE is configured to monitor DCI format 3_0 and the number of information bits in DCI format 3_1 is less than the payload of DCI format 3_0, zeros shall be appended to DCI format 3_1 until the payload size equals that of DCI format 3_0.

#### 8.3.1.1	SCI format 1-A

SCI format 1-A is used for the scheduling of PSSCH and 2nd-stage-SCI on PSSCH

The following information is transmitted by means of the SCI format 1-A:

< Unchanged parts are omitted >

-	Additional MCS table indicator – as defined in clause 8.1.3.1 of [6, TS 38.214]: 1 bit if one MCS table is configured by higher layer parameter sl-Additional-MCS-Table; 2 bits if two MCS tables are configured by higher layer parameter sl-Additional-MCS-Table; 0 bit otherwise.

-	PSFCH overhead indication – 1 bit as defined clause 8.1.3.2 of [6, TS 38.214] if higher layer parameter sl-PSFCH-Period = 2 or 4; 0 bit otherwise.

< Unchanged parts are omitted >

### 8.3.2	CRC attachment

CRC attachment is performed according to clause 7.3.2 except that scrambling is not performed.