3GPP TSG-RAN Meeting #109e	R1-2205690

e-meeting, May 9 – 20, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.211 | CR | 0098 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of PUSCH DM-RS generation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2022-05-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Unclear definition of symbol number for PUSCH DM-RS sequence generation when transform preoding is used. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarifired the definition of symbol number |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguous specificaation |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.1.1.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

6.4.1.1.1.2	Sequence generation when transform precoding is enabled

If transform precoding for PUSCH is enabled, the reference-signal sequence  shall be generated according to

where ru,vα,δ(n) with δ=1 depends on the configuration:

-	if the higher-layer parameter dmrs-UplinkTransformPrecoding is configured, π/2-BPSK modulation is used for PUSCH, and the PUSCH transmission is not a msg3 transmission, and the transmission is not scheduled using DCI format 0_0 in a common search space, ru,vα,δ(n) is given by clause 5.2.3 with cinit given by

cinit=217Nsymbslotns,fμ+l+12NIDnSCID+1+2NIDnSCID+nSCIDmod 231

where nSCID=0 unless given by the DCI according to clause 7.3.1.1.2 in  [4, TS38.212] for a transmission scheduled by DCI format 0_1, or given by the DCI according to clause 7.3.1.1.3 in  [4, TS38.212] for a transmission scheduled by DCI format 0_2 if the antenna ports field in the DCI format 0_2 is not 0 bit, or given by the higher-layer parameter antennaPort for a PUSCH transmission scheduled by a type-1 configured grant; and

-	NID0,NID1∈0,1,…,65535 are given by the higher-layer parameters pi2BPSK-ScramblingID0 and pi2BPSK-ScramblingID1, respectively, in the DMRS-UplinkConfig IE if provided and the PUSCH is scheduled by DCI format 0_1, or by DCI format 0_2 if the antenna ports field in the DCI format 0_2 is not 0 bit, or by a PUSCH transmission with a configured grant;

-	NID0∈0,1,…,65535 is given by the higher-layer parameter pi2BPSK-ScramblingID0 in the DMRS-UplinkConfig IE if provided and the PUSCH is scheduled by DCI format 0_0 with the CRC scrambled by C-RNTI, MCS-C-RNTI, or CS-RNTI, or by DCI format 0_2 if the antenna ports field in the DCI format 0_2 is  0 bit;

-	NIDnSCID=NIDcell otherwise;

-	otherwise, ru,vα,δ(n) is given by clause 5.2.2 with α=0.

The sequence group u=fgh+nIDRS mod 30, where nIDRS is given by

-	nIDRS=nIDPUSCH if nIDPUSCH is configured by the higher-layer parameter nPUSCH-Identity in the DMRS-UplinkConfig IE, and

-	the higher-layer parameter dmrs-UplinkTransformPrecoding is not configured or the higher-layer parameter dmrs-UplinkTransformPrecoding is configured and π/2-BPSK modulation is not used for PUSCH, and

-	the PUSCH is neither scheduled by RAR UL grant nor scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI according to clause 8.3 in [5, TS 38.213];

-	nIDRS=NIDnSCID if the higher-layer parameter dmrs-UplinkTransformPrecoding is configured, π/2-BPSK modulation is used for PUSCH, the PUSCH transmission is not a msg3 transmission, and the transmission is not scheduled using DCI format 0_0 in a common search space;

-	nIDRS=NIDcell otherwise

where  and the sequence number v are given by:

-	if neither group, nor sequence hopping is enabled

-	if group hopping is enabled and sequence hopping is disabled

where the pseudo-random sequence  is defined by clause 5.2.1 and shall be initialized with  at the beginning of each radio frame

-	if sequence hopping is enabled and group hopping is disabled

where the pseudo-random sequence  is defined by clause 5.2.1 and shall be initialized with  at the beginning of each radio frame.

The hopping mode is controlled by higher-layer parameters:

-	for PUSCH transmission scheduled by RAR UL grant or by DCI format 0_0 with CRC scrambled by TC-RNTI, sequence hopping is disabled and group hopping is enabled or disabled by the higher-layer parameter groupHoppingEnabledTransformPrecoding;

-	for all other transmissions, sequence hopping and group hopping are enabled or disabled by the respective higher-layer parameters sequenceHopping and sequenceGroupHopping if these parameters are provided, otherwise, the same hopping mode as for Msg3 shall be used.

The UE is not expected to handle the case of combined sequence hopping and group hopping.

The quantity l above is the OFDM symbol number in the slot except for the case of double-symbol DMRS in which case l is the OFDM symbol number in the slot of the first symbol of the double-symbol DMRS.