3GPP TSG RAN WG1 #109-e                                                                                  R1-2205697

e-Meeting, May 9th – 20th, 2022

Source:                Moderator (Intel Corporation)

Title:	Summary#5 of AI: 9.9.1 NR PDCCH reception in symbols with LTE CRS REs

Agenda item:	9.9.1

Document for:	Discussion and Decision

# Introduction

In RAN#94-e meeting a work item on eDSS support was agreed for Rel-18 [1]. The objectives of the WID include enhancements to NR PDCCH reception as shown below:

The following objectives shall be included for improvement of NR spectrum efficiency for LTE-NR co-existence (RAN1): • Study and if needed specify NR PDCCH reception in symbols with LTE CRS REs. [RAN1]

This document contains summary of the companies’ and moderator’s proposals.

# Extended email discussion

## EVM clarifications and template for results collection

In order to agree on a template we need a set of common scenarios and KPIs. I have collected the following clarifications for the evaluation assumptions and KPIs/others. Please comment on (at least)

- whether the configurations for baseline and Rel-18 enhancements are agreeable (columns 2 and 3)

- column 4 is derived from what we agreed as example KPIs/Others in SLS assumptions. pls. comment if certain KPIs can be mandated or dropped.

Proposal#1:

| Scenario | Baseline | Rel-18 enhancement | Companies to report one or more of following (examples): |
| --- | --- | --- | --- |
| #1A | s0: reserved for LTE s1: reserved for LTE s2: NR PDCCH (any NR UE) s3-13: reserved for NR PDSCH | s0: reserved for LTE s1: NR PDCCH (only Rel18 UE) s2: NR PDCCH (any NR UE) s3-13: reserved for NR PDSCH | 1. NR PDCCH capacity improvement of Rel-18 enhancement compared to baseline 2. NR PDCCH coverage/outage due to Rel-18 enhancement 3. Potential degradation of LTE due to Rel-18 enhancement compared to baseline 4. Whether and how to achieve coexistence with legacy UEs 5. Assumptions on: Fraction of LTE UEs, Receiver |
| #2 | s0: reserved for LTE s1: reserved for LTE s2: NR PDCCH (any NR UE) s3-13: reserved for NR PDSCH | s0: reserved for LTE s1+s2: NR PDCCH (only Rel18 UE) s2: NR PDCCH (any NR UE) s3-13: reserved for NR PDSCH | 1. NR PDCCH capacity improvement of Rel-18 enhancement compared to baseline 2. NR PDCCH coverage/outage due to Rel-18 enhancement 3. Potential degradation of LTE due to Rel-18 enhancement compared to baseline 4. Whether and how to achieve coexistence with legacy UEs 5. Assumptions on: Fraction of LTE UEs, Receiver |
| #3 | s0: reserved for LTE s1: reserved for LTE s2-3: NR PDCCH (NR UE FG22-12) s4-13: reserved for NR PDSCH | s0: reserved for LTE s1+s2+s3: NR PDCCH (only Rel18 UE) s2+s3: NR PDCCH (NR UE FG22-12) s4-13: reserved for NR PDSCH | 1. NR PDCCH capacity improvement of Rel-18 enhancement compared to baseline 2. NR PDCCH coverage/outage due to Rel-18 enhancement 3. Potential degradation of LTE due to Rel-18 enhancement compared to baseline 4. Whether and how to achieve coexistence with legacy UEs 5. Assumptions on: Fraction of LTE UEs, Receiver |

| Company | Comment |  |
| --- | --- | --- |
| Huawei/Hisi | For the 1st column: (i) ‘any NR UE’ should be ‘legacy NR UE’, as there is no R18 UEs for baseline. (ii) There seems to be no need for simulating PDSCH for subsequent symbols; if the intention is for clarification, then ‘s3-13’ or ‘s4-13’ should be reserved for LTE and/or NR PDSCH. (iii) s2+s3 for legacy NR PDCCH can also be optionally considered for baseline of #1A, to show the capacity gain of enabling R18 PDCCH on s1 on top of the 2OS for legacy NR. For the 2nd column: #1A: s2 should be ‘legacy NR UE’ to align with baseline. s2+s3 is optionally considered to align with the 1st column. #2: A question for ‘s2 : NR PDCCH’: is the intention to assume the legacy NR is FDMed with R18 NR and the legacy NR CORESET blocks R18 PDCCH resources? If so, the NR UE should be ‘legacy’ rather than ‘any’. #3: Same changes as #2. For the 3rd column: 1. the absolute values of PDCCH capacity for R18 and baseline (instead of the relative improvement) can be reported to better align results over companies; 2. the absolute values of PDCCH outage/coverage (e.g., percentile of UEs with BLER>1% under AL16) for R18 and baseline can be reported to better align results over companies; 5. (i) Receiver for PDCCH (e.g., process as legacy where UE is not aware of punctured PDCCH RE; or, receiver punctures) and receiver for DMRS (i.e., channel estimator) should be separately reported. (ii) The methodology for R18 PDCCH capacity calculation under two cases need to be reported: 1) LTE BW < NR BW; e.g., LTE BW=10MHz while NR BW=20MHz, the relative capacity gain over legacy is proportional to the same BW. 2) legacy NR CORESET is FDMed with R18 NR and blocks R18 NR PDCCH so the available R18 PDCCH resource/capacity is impacted. #1A; s0: reserved for LTE s1: reserved for LTE s2[+s3]: NR PDCCH (legacy any NR UE) s3[/4]-13: reserved for LTE/NR PDSCH; s0: reserved for LTE s1: NR PDCCH (only Rel18 UE) s2[+s3]: NR PDCCH (legacy any NR UE) s3[/4]-13: reserved for LTE/NR PDSCH; 1. NR PDCCH capacity improvement of Rel-18 enhancement compared to and baseline 2. NR PDCCH coverage/outage of due to Rel-18 enhancement and baseline 3. Potential degradation of LTE due to Rel-18 enhancement compared to baseline 4. Whether and how to achieve coexistence with legacy UEs 5. Assumptions on: Fraction of LTE UEs, Receiver for PDCCH and DMRS, capacity calculation methodology if LTE bandwidth<NR bandwidth or legacy NR PDCCH blocks R18 NR PDCCH.. #2; s0: reserved for LTE s1: reserved for LTE s2: NR PDCCH (legacy any NR UE) s3-13: reserved for LTE/NR PDSCH; s0: reserved for LTE s1+s2: NR PDCCH (only Rel18 UE) s2: NR PDCCH (legacy any NR UE) s3-13: reserved for LTE/NR PDSCH; 1. NR PDCCH capacity improvement of Rel-18 enhancement compared to and baseline 2. NR PDCCH coverage/outage of due to Rel-18 enhancement and baseline 3. Potential degradation of LTE due to Rel-18 enhancement compared to baseline 4. Whether and how to achieve coexistence with legacy UEs 5. Assumptions on: Fraction of LTE UEs, Receiver for PDCCH and DMRS, capacity calculation methodology if LTE bandwidth<NR bandwidth or legacy NR PDCCH blocks R18 NR PDCCH.. #3; s0: reserved for LTE s1: reserved for LTE s2-3: NR PDCCH (NR UE FG22-12) s4-13: reserved for LTE/NR PDSCH; s0: reserved for LTE s1+s2+s3: NR PDCCH (only Rel18 UE) s2+s3: NR PDCCH (legacy NR UE FG22-12) s4-13: reserved for LTE/NR PDSCH; 1. NR PDCCH capacity improvement of Rel-18 enhancement compared to and baseline 2. NR PDCCH coverage/outage of due to Rel-18 enhancement and baseline 3. Potential degradation of LTE due to Rel-18 enhancement compared to baseline 4. Whether and how to achieve coexistence with legacy UEs 5. Assumptions on: Fraction of LTE UEs, Receiver for PDCCH and DMRS, capacity calculation methodology if LTE bandwidth<NR bandwidth or legacy NR CORESET blocks R18 NR PDCCH.. |
| Samsung | OK with #1A. Agree with HW that s2 and s3 should also be considered for legacy UEs but OK to prioritize s2 as that is the only possibility for CORESET#0. #2 and #3 require more discussion. Given that the scenario is LTE-NR coexistence and that PDCCHs in frequency-overlapping CORESETs that span different and overlapping symbols cannot be assumed orthogonal, #2 and #3 may not be realistic. For #2/#3 and 10/20 MHz BW, CORESET#0 needs to be 5/10 MHz (to avoid frequency overlapping for CORESETs with different symbols) resulting to a maximum of only 4/8 CCEs (s3 cannot be used for CORESET#0), when 10/20 MHz (8/16 CCEs) is possible in Rel-17. That would penalize capacity and coverage for all UEs (and for legacy UEs after initial access). That is not something that makes sense for a NW to do but at least some discussion/clarifications are needed. Basically, for #2/#3, there needs to be description for how CORESET#0 is supported and on a corresponding impact. The same for CORESET#1 of legacy UEs. |  |
| Ericsson1 | On template format, it would be good to have a template that provides - a way to summarize link performance results from different inputs (i.e., SNR for 1%BLER for different ALs for ‘baseline’ and ‘Rel18 enhancement’ cases) - a way to summarize ‘NR PDCCH capacity’ results from different inputs (i.e., based on post-processing the link results with additional assumptions/considerations). We are generally fine with clarifications suggested in Moderator Proposal#1 for further alignment. Regarding comments from Samsung and Huawei/HiSi, To reflect Samsung comments, perhaps following can be added as a separate point in 4th column -- “Assumption on how COREST0 is handled”. On HW/HiSi comments, - symbols without CRS (i.e., s2 for #1A, #2 and s2+s3 for #3) can be used by any UE including those supporting the Rel18 enhancement (e.g., for #2, the Rel18 UE can be configured with two CORESETs one just with s2 and another with s1+s2 and PDCCH for that UE can be sent on any of these CORESETs). Current formulation “any UE” reflects this. HW proposal “legacy UE” seems to preclude this. Given this we think current formulation is accurate. - On “NR PDCCH capacity improvement of Rel-18 enhancement compared to and baseline”, following can be reported -- i) capacity with baseline ii) capacity when Rel18 enhancement is added on top of baseline in addition to iii) relative gain (agree with HW that this may help alignment between company results). Prefer following wording to reflect this “NR PDCCH capacity when supporting Rel-18 enhancement compared to and baseline, and relative gain” - On “NR PDCCH coverage/outage of due to Rel-18 enhancement and baseline”, the current formulation “NR PDCCH coverage/outage due to Rel-18 enhancement” asks to report if there is any outage/coverage impact due to the Rel18 enhancement which seems OK as is from our side. HW proposed “NR PDCCH coverage/outage of due to Rel-18 enhancement” is not clear, i.e., is intention to report e.g., “fraction X of UEs in the cell for which Rel18 enhancement can be used”? This fraction X may also be reported but ‘not being able to support Rel18 enhancement’ is not same as ‘outage’ as PDCCHs can be sent using legacy mechanism for any UE. - On “Assumptions on: Fraction of LTE UEs, Receiver for PDCCH and DMRS, capacity calculation methodology if LTE bandwidth<NR bandwidth or legacy NR CORESET blocks R18 NR PDCCH., perhaps following wording is better as it would PDCCHs that are blocked “Assumptions on: Fraction of LTE UEs, Receiver for PDCCH and DMRS, capacity calculation methodology if LTE bandwidth<NR bandwidth or legacy NR CORESET blocks R18 NR PDCCH legacy NR PDCCH blocks NR PDCCH sent using Rel18 mechanism” - On “There seems to be no need for simulating PDSCH for subsequent symbols” we have same understanding. That said we think it is reasonable to compare cases where same number of symbols are reserved for PDSCH as discussed during the meeting.” - On “s2+s3 for legacy NR PDCCH can also be optionally considered for baseline of #1A”, if companies see need for it, perhaps better to capture as below. #1A; s0: reserved for LTE s1: reserved for LTE s2: NR PDCCH (any NR UE) s3-13: reserved for NR PDSCH Following can also be optionally considered as baseline2 s0: reserved for LTE s1: reserved for LTE s2+s3: NR PDCCH (NR UE FG 22-12) with only s2 for NR UEs not supporting FG22-12 s4-13: reserved for NR PDSCH; s0: reserved for LTE s1: NR PDCCH (only Rel18 UE) s2: NR PDCCH (any NR UE) s3-13: reserved for NR PDSCH If baseline2 used s0: reserved for LTE s1: NR PDCCH (only Rel18 UE) s2+s3: NR PDCCH (NR UE FG 22-12) with only s2 for NR UEs not supporting FG22-12 s4-13: reserved for NR PDSCH; 1. NR PDCCH capacity improvement of Rel-18 enhancement compared to baseline 2. NR PDCCH coverage/outage due to Rel-18 enhancement 3. Potential degradation of LTE due to Rel-18 enhancement compared to baseline 4. Whether and how to achieve coexistence with legacy UEs 5. Assumptions on: Fraction of LTE UEs, Receiver. |
| MediaTek | Thanks to Moderator’s efforts on clarifying simulation assumption. Our comments are as follows. We also have some questions and will appreciate some corresponding clarification. - General comments for all scenarios - As mentioned by Huawei//Hisi, Samsung, and Ericsson, we are also confused about capturing PDSCH resource in the table since there is no need to simulate PDSCH performance. We suggest not capturing it. - For the 4th column - Receiver: we agree that companies should report the receiver assumptions on PDCCH and DMRS and we are ok with Huawei/Hisi ‘s suggestion. However, for each agreed options (1-1, 1-2, 2), we already have agreed baseline assumptions on receivers. Should we capture this aspect somewhere? Or we should have some wording like “Receiver (baseline receiver assumption based on RAN1#109-e agreements)”. The intention is to encourage companies at least report the simulation associated with those agreed baselines. - PDCCH capacity: we agree with Huawei/Hisi and Ericsson that the absolute capacity for each scheme should be reported. - We agree with Samsung that the coexistence between legacy UEs and Rel-18 UEs on CORESET#0 configuration should be further discussed. Although we think this aspect belongs to “whether and how to achieve coexistence with legacy UEs”, we are ok to capture this aspect explicitly. - Q:For symbol s1: NR PDCCH (only Rel18 UE), does it mean no LTE or it means s1 is overlapped with LTE UEs? That is, NR PDCCH will be punctured by LTE CRS on s1. - Comments for each scenario - #1A - We share the same view with Huawei/Hisi and Samsung that “s2+s3: NR PDCCH” should be included as one of baselines to compare with Rel-18 enhancements. - We agree with Huawei/Hisi that baseline should be “legacy UE” instead of “any UE.” - Q: we understand s2 can be used for any UE. However, what should we assume for simulation and PDCCH capacity analysis? Can the 1 symbol CORESET in s2 shared by legacy and Rel-18 UEs or the respective CORESETs should be FDMed? If FDMed is assumed, how should we compare the PDCCH capacity with the baseline where no FDMed is assumed? - #2 - As mentioned by Samsung, we also have questions on how to simulate: s1+s2: NR PDCCH (only Rel-18 UE) s2: NR PDCCH (any NR UE) That is, if s1+s2 are used for Rel-18 UEs, how could legacy UE utilize s2? FDM with limitation on the resource allocation for both Rel-18 and legacy UE CORESETs? |  |
| Qualcomm | We are generally OK with the current direction (Moderator’s Proposal with suggestions by other companies above). The concern on the following is not clear to us: s1+s2: NR PDCCH (only Rel-18 UE) s2: NR PDCCH (any NR UE), including CORESET#0 It is possible for NW to configure the two CORESETs such that they are FDMed. It is also possible for NW to configure the two CORESETs such that they are fully (or partially) overlapped in frequency-domain (but partially overlapped in time-domain). In the latter case, there could be blocking between two PDCCH candidates associated with the two CORESETs. However, this already exists in legacy releases where on s2, PDCCH monitoring using CORESET#0 and non-zero CORESET are configured. Note that frequency-domain resource allocation of CORESET#0 may not be aligned with the 6 RB grid for non-zero CORESETs anyway. And, even within the same CORESETs, PDCCHs for different UEs block each other due to different hash patterns. For the evaluation purpose, we think companies can assume either FDM or overlapping as described above. In case of FDM, there must be linear impact on the PDCCH capacity. In case of overlapping, statistically there would be no substantial impact. To be accurate, companies can evaluate PDCCH blocking probability as the number (or the ratio) of PDCCHs associated with the first and second CORESETs, instead of computing expected number of CCEs for a PDCCH transmission. |  |
| Vivo1 | One comment for editorial change for section1 introduction: the summary should be for eDSS not for enhanced MIMO. Regarding the proposal, we are generally OK with the proposal. Regarding the configuration of CORESETs in #2 and #3, it is not clear if the CORESETs in column 3 are overlapping on s2(i.e., CORESET 1 on s1+s2 for R18 UE, CORESET2 on s2 for any UE including R18 UE) or if they are spanning on different symbols (i.e., CORESET1 on s1 for R18 UE, CORESET2 on s2 for any UE including R18 UE), CORESET configuration should be clarified. - Rel-18 enhancement in scenario1A s0: reserved for LTE CORESTE1 on s1: NR PDCCH (only Rel18 UE) CORESTE2 on s2: NR PDCCH (any NR UE) s3-13: reserved for NR PDSCH - Rel-18 enhancement in scenario2 s0: reserved for LTE CORESTE1 on s1+s2: NR PDCCH (only Rel18 UE) CORESTE2 on s2: NR PDCCH (any NR UE) s3-13: reserved for NR PDSCH - Rel-18 enhancement in scenario3 s0: reserved for LTE CORESTE1 on s1+s2+s3: NR PDCCH (only Rel18 UE) CORESTE2 on s2+s3: NR PDCCH (NR UE FG22-12) s4-13: reserved for NR PDSCH Besides, we share a similar view as Qualcomm that companies can evaluate either FDM or frequency overlapping case, but for frequency overlapping case, company can assume ideal scheduling, that is, no blocking between R18 PDCCH and PDCCH for legacy UE. Regarding comments on CORESET#0, how to configure CORESET#0 is only part of coexistence, the evaluation should be more generic and the CORESET on S2 can be any CORESET shared byR18 UE and legacy UEs, and should not be limited to CORESET#0. |  |
| Spreadtrum | For the 4th column, we agree with Huawei that receiver for DMRS (i.e., channel estimator) should be separately reported. We think that the 4th column is mainly KPI related to SLS, but channel estimation performance is mainly reflected by LLS. Thus, it would be clearer to add a column to clarify the channel estimation under each scenario. In our understanding, the table is divided according to scenarios, but different scenarios may contain multiple options, and there are multiple channel estimations under each option. Thus, we suggest to add a column to clarify the baseline channel estimation and optional channel estimation in each scenario. As mentioned by Huawei//Hisi and MTK, we also think there seems to be no need for simulating PDSCH performance. In order to avoid ambiguity, we suggest to remove it. Regarding scenario #1A, we agree with Huawei that s2 and s3 for legacy NR PDCCH should be optionally considered for baseline. Agree with most companies’s comments, we think the configuration of CORESET in scenario #2 and #3 should be clarified. |  |
| ZTE | It seems above proposal#1 is more for SLS clarifications. We agree Ericsson that we also need to agree on a template for LLS results collection, e.g., the following could be a starting point. Scenario; Baseline; Rel-18 enhancement; Companies to report one or more of following. #1; PDCCH with 1 clean symbol; PDCCH with 1 overlapping symbol and 1 clean symbol; Assumptions: - Based on the agreed LLS assumptions and companies report optional assumptions if any. - Companies report the assumed options, i.e., the agreed Option 1-1, Option 1-2 or Option 2. Performance metric - SNR@1%BLER. #2; PDCCH with 2 clean symbols; PDCCH with 1 overlapping symbol and 1 clean symbol， or PDCCH with 1 overlapping symbol and 2 clean symbols. Regarding proposal#1 for SLS calibration, we are generally fine and the following are our views. - ‘any NR UE’ vs ‘legacy UE’: For baseline, it’s better to assume legacy UEs on symbols not overlapping with CRS. For Rel-18 enhancement, we agree with Ericsson that there is no need to limit to only legacy UEs. So, we suggest to adopt the proposed changes on the 1st column from HW, while not the 2nd column. - We are ok with Ericsson proposed baseline2 for scenario #1A. - We agree with QC that it’s better to clarify what’s the assumptions for coexistence. That is, companies can report one of the following assumptions. - Assumption#1: The configured CORESET for legacy UEs and new UEs are FDMed. This will lead to some of the RBs in s1 cannot be allocated. - Assumption#2: The configured CORESET for legacy UEs and new UEs can be overlapped while gNB needs to avoid scheduling both legacy UEs and new UEs in the overlapping resources. |
| Xiaomi |  | We share the same view with QC that Rel 18-any NR UE coexistence on s2 for #2 and s2+s3 for #3 needs to be clarified. If the two CORESETs are FDMed, the frequency ranges of the two CORESETs need to be specified so that the evaluation results can be compared. |

## Update-1

@HW: optional scenario added in #1A, other companies have clarified reasoning for “any NR UE” in column #3, current wording (column #4) does not preclude companies from reporting outage of Rel-18 enhancement and baseline

@Samsung: added CORESET#0 handling (companies to report)

@Ericsson, ZTE: PDCCH capacity (as part of column #4) will be summarized in template, SNR@1%BLER is not an agreed KPI yet, I am not sure that it will be an easy discussion to agree on a common set of baseline and enhancement schemes for capturing such results.

@MTK: deleted PUSCH bullet, added a note to emphasize baseline Rx assumptions from agreement, about the question on s1, my understanding is that LTE CRS ports are assumed to exist on s1, for #1A – my understanding is that using s2 for legacy NR UEs only is also allowed for Rel-18 enhancement – companies can report their assumptions

@Vivo: my understanding is aligned with your proposal but CORESET numbering may bring other unnecessary issues so I hope you understand

@Spreadtrum:  you are right that different set of assumptions (channel estimation for example) will create different results and my intention is to enable such reporting in the template. I will create a template once the discussion stabilizes.

@Xiaomi: added clarification of CORESET configuration and blocking between legacy NR and Rel-18 PDCCH as part of assumptions

Please consider the modification below for the next round of comments:

Proposal#1:

| Scenario | Baseline | Rel-18 enhancement | Companies to report one or more of following (examples): |
| --- | --- | --- | --- |
| #1A | s0: reserved for LTE s1: reserved for LTE s2: NR PDCCH (any legacy NR UE) s3-13: reserved for NR PDSCH baseline-2 (optional): s0: reserved for LTE s1: reserved for LTE s2+s3: NR PDCCH (NR UE FG 22-12), only s2 for legacy NR UEs not supporting FG22-12 | s0: reserved for LTE s1: NR PDCCH (only Rel18 UE) s2: NR PDCCH (any NR UE) s3-13: reserved for NR PDSCH for baseline-2: s0: reserved for LTE s1: NR PDCCH (only Rel18 UE) s2+s3: NR PDCCH (NR UE FG 22-12), only s2 for any NR UEs not supporting FG22-12 | 1. NR PDCCH capacity due to improvement of Rel-18 enhancement compared to and baseline, and relative gain 2. NR PDCCH coverage/outage due to Rel-18 enhancement 3. Potential degradation of LTE due to Rel-18 enhancement compared to baseline 4. Whether and how to achieve coexistence with legacy UEs 5. Assumptions on: Fraction of LTE UEs, Receiver for PDCCH and DMRS (pls. note baseline receiver assumptions from RAN1#109-e agreements), capacity calculation methodology if LTE bandwidth<NR bandwidth, CORESET configurations and blocking between legacy NR PDCCH and Rel18 enhancement PDCCH, CORESET#0 handling |
| #2 | s0: reserved for LTE s1: reserved for LTE s2: NR PDCCH (any legacy NR UE) s3-13: reserved for NR PDSCH | s0: reserved for LTE s1+s2: NR PDCCH (only Rel18 UE) s2: NR PDCCH (any NR UE) s3-13: reserved for NR PDSCH | 1. NR PDCCH capacity due to improvement of Rel-18 enhancement compared to and baseline, and relative gain 2. NR PDCCH coverage/outage due to Rel-18 enhancement 3. Potential degradation of LTE due to Rel-18 enhancement compared to baseline 4. Whether and how to achieve coexistence with legacy UEs 5. Assumptions on: Fraction of LTE UEs, Receiver for PDCCH and DMRS (pls. note baseline receiver assumptions from RAN1#109-e agreements), capacity calculation methodology if LTE bandwidth<NR bandwidth, CORESET configurations and blocking between legacy NR PDCCH and Rel18 enhancement PDCCH, CORESET#0 handling |
| #3 | s0: reserved for LTE s1: reserved for LTE s2-3: NR PDCCH (NR UE FG22-12) s4-13: reserved for NR PDSCH | s0: reserved for LTE s1+s2+s3: NR PDCCH (only Rel18 UE) s2+s3: NR PDCCH (NR UE FG22-12) s4-13: reserved for NR PDSCH | 1. NR PDCCH capacity due to improvement of Rel-18 enhancement compared to and baseline, and relative gain 2. NR PDCCH coverage/outage due to Rel-18 enhancement 3. Potential degradation of LTE due to Rel-18 enhancement compared to baseline 4. Whether and how to achieve coexistence with legacy UEs 5. Assumptions on: Fraction of LTE UEs, Receiver for PDCCH and DMRS (pls. note baseline receiver assumptions from RAN1#109-e agreements), capacity calculation methodology if LTE bandwidth<NR bandwidth, CORESET configurations and blocking between legacy NR PDCCH and Rel18 enhancement PDCCH, CORESET#0 handling |

| Company | Comments |
| --- | --- |
| Huawei/Hisi | Replies to @ Ericsson @Moderator: For the “R18 enhancement” column: - “symbols without CRS (i.e., s2 for #1A, #2 and s2+s3 for #3) can be used by any UE including those supporting the Rel18 enhancement”: as per the agreement, Scenario#2 is ‘2 symbols CORESET’ while Scenario#3 is ‘3 symbols CORESET’, so “s2 only for #2” and “s2+s3 for #3” are not supported by the agreement; as to “s2 only for #1A”, we think it there is no difference between legacy NR PDCCH and R18 PDCCH on the clean symbol only, so making s2 for legacy NR PDCCH can better align with the baseline and other scenarios. Regarding that, we still believe it should be changed as “s2: NR PDCCH (legacy any NR UE)”. For the “Companies to report” column: - “NR PDCCH capacity due to improvement of Rel-18 enhancement compared to and baseline, and relative gain”: An editorial change from ‘due to’ to ‘for’, i.e., “NR PDCCH capacity for due to improvement of Rel-18 enhancement compared to and baseline, and relative gain” - “NR PDCCH coverage/outage of due to Rel-18 enhancement and baseline”: the intention of our change in the last round is the same as the previous bullet, i.e., companies can report the absolute values of outage for R18 and baseline, separately. In our understanding, for both legacy and R18, there may be fraction of UEs which cannot achieve 1% target BLER at the highest AL=16 (i.e. outage). To be consistent with the previous bullet, it can be updated as “NR PDCCH coverage/outage for due to Rel-18 enhancement and baseline, and relative loss” In addition, for the updated table: For the #1A row: for the “baseline” column, “s2+s3: NR PDCCH (NR UE FG 22-12), only s2 for legacy NR UEs not supporting FG22-12” – for this case, we may simply assume “s2+s3: NR PDCCH (NR UE FG 22-12), only s2 for legacy NR UEs not supporting FG22-12”, rather than assuming two types of legacy UEs coexisting, which complicates the simulation. Similarly for the “R18 enhancement” column, “only s2 for any NR UEs not supporting FG22-12” should be crossed out. For the “Companies to report” column: from the views of some companies, the term ‘blocking’ seems causing some confusion; as a clarification, our intention is to assume the FDMed legacy PDCCH and R18 PDCCH (instead of the PDCCH candidate blocking), so that the capacity gain of the R18 enhancement over baseline is proportionally reduced. The minor change can be “capacity calculation methodology if LTE bandwidth<NR bandwidth, CORESET configurations and blocking/FDM between legacy NR PDCCH and Rel18 enhancement PDCCH” |
| MediaTek | Thanks to Moderator’s consolidated proposal. We have further comments as follows. - #1A - Our original proposal is to add baseline-2 as one of the baselines, not targeting for one optional baseline. The intention is to compare Rel-16 UE (FG22-12) with Rel-18 UE supporting 1 punc symbol+1 clean symbol to see how much is the performance difference and the outcome can be used as additional information to determine whether there is a need to introduce Rel-18 feature. If we were the only company wants to consider baseline-2 as mandatory baseline, we can accept baseline-2 as optional baseline. However, based on the intention above, we don’t think the evaluation should be limited to: UE supporting baseline-2 v.s. UE supporting 1 punc symbol on top of baseline-2 as shown in the current proposal. After all, baseline-2 is optional and companies should have the freedom to evaluate UE supporting baseline-2 v.s. UE supporting 1punc symbol+1 clean symbol as well, if they were interested in those comparison. Therefore, we suggest Scenario; Baseline; Rel-18 enhancement. #1A; baseline-1 s0: reserved for LTE s1: reserved for LTE s2: NR PDCCH (any legacy NR UE) s3-13: reserved for NR PDSCH baseline-2 (optional): s0: reserved for LTE s1: reserved for LTE s2+s3: NR PDCCH (NR UE FG 22-12), only s2 for legacy NR UEs not supporting FG22-12; for baseline-1 and baseline-2: s0: reserved for LTE s1: NR PDCCH (only Rel18 UE) s2: NR PDCCH (any NR UE) s3-13: reserved for NR PDSCH for baseline-2: s0: reserved for LTE s1: NR PDCCH (only Rel18 UE) s2+s3: NR PDCCH (NR UE FG 22-12), only s2 for any NR UEs not supporting FG22-12. - #2 - Based on the current table format, if our understanding is correct, the only difference between scenarios #1 and scenario#2 is the CORESET symbol length. Therefore, we also suggest to add optional baseline-2 to scenario #2. That is: #2; s0: reserved for LTE s1: reserved for LTE s2: NR PDCCH (any legacy NR UE) s3-13: reserved for NR PDSCH baseline-2 (optional): s0: reserved for LTE s1: reserved for LTE s2+s3: NR PDCCH (NR UE FG 22-12), only s2 for legacy NR UEs not supporting FG22-12; s0: reserved for LTE s1+s2: NR PDCCH (only Rel18 UE) s2: NR PDCCH (any NR UE) s3-13: reserved for NR PDSCH. |
| Ericsson2 | We are generally OK with moderator proposed updates to Proposal#1 Regarding further comment from @HW/HiSi - on ‘legacy NR UE”/” any NR UE”, there is no restriction in previous agreements/discussions on how UEs can receive PDCCH in symbols without CRS (e.g., no restriction that CORESET0 for Rel18 UE with enhancement should be in s1+s2, s1+s2+s3). The text cited by HW was about CORESET used for receiving PDCCH in symbol with CRS (i.e., with 1,2 or 3 symbol CORESET) and does not say anything about restrictions on other CORESETs. - on proposed updates for bullets #1 and #2 in last column - for #1, current text by moderator is clearer than the suggested update (if update is needed, our earlier suggestion “NR PDCCH capacity when supporting Rel-18 enhancement” is better in our view). for #2 – we prefer current formulation for reason explained earlier. - prefer to keep “only s2 for any NR UEs not supporting FG22-12” as 22-12 is Rel16 capability and handling of Rel15 UEs (and also CORESET0) should be taken into account. |
| Qualcomm | We agree with MTK that it is reasonable to consider that the Rel-16 feature (FG22-12) is part of the baselines. For performance comparison between “the baseline with FG22-12” and “the Rel-18 enhancement”, it would be fair to assume the same amount/ratio of UEs supporting FG22-12 in both cases. Companies should pick up reasonable assumption and report it with the results. The simplest assumption is that all the UEs in this cell support FG22-12. Complicated assumption is that some UEs support FG22-12 but not Rel-18 enh, some UEs support Rel-18 enh but not FG22-12, and some other UEs support both FG22-12 and Rel-18 enh. Regarding “any UEs vs legacy UEs”, we think “any UEs” makes sense, since in reality, there is no such constraint (i.e., NR PDCCH on s2 can also be configured for Rel-18 UEs by default). Regarding multiplexing PDCCHs for legacy UEs and Rel-18 UEs, as we have commented, FDM (semi-static FDM via CORESET configurations) is not the only option – rather FDM is less efficient way. For the evaluation purpose of the frequency overlapping case, we agree with vivo/Spreadtrum/ZTE that companies should report how the multiplexing is supposed to be done (ideal scheduling is an example assumption as vivo suggested). The point is that the PDCCH blocking due to frequency overlapping configuration should not be the focus/issue of the current discussion – it exists even today (even within the same CORESET). |
| Mod | The following issue seem non-controversial (I can implement this) 1. NR PDCCH capacity for due to improvement of Rel-18 enhancement compared to and baseline, and relative gain” The following issues seem controversial so pls. check if you can live with current wording: 1. @HW: change to s2: legacy NR UE 2. @MTK: the current comparison keeps the amount of system resources for PDCCH somewhat similar between baseline and enhancement - it would be good to keep the current wording as a minimum set of cases for reporting, companies can bring other cases – no problem but not sure its a good idea to expand the number of combinations 3. further clarification of FDM or overlapping CORESET configuration between legacy and Rel-18 NR PDCCH |
| Samsung | In general, a decision whether or not to introduce a new feature depends on tradeoffs of what is currently possible. CORESETs for Rel-17 UEs can be placed in s1 and s2 and span the whole BW (even when there is CRS in s1 – gNB indicates there isn’t). Proposals to restrict the BW and/or symbols where CORESETs, including CORESET#0, for Rel-17 UEs can be, and/or restrict gNB scheduling, should be compared in terms of coverage/capacity/blocking/scheduling restrictions/BLER with the Rel-17 baseline without any restrictions assuming that a NW seamlessly punctures 1/3 of DM-RS in s1 (1/6 of DM-RS in total for s1 and s2). The above issues should be addressed with detailed analysis, not abstractly, in support of any such proposals. So, we do not agree that s1 should always be assumed reserved for LTE as baseline when it does not need to, especially for scenarios #2 and #3. That would also be against the agreement to include “Option 2” for evaluation. If the current setup is to be kept, we suggest adding a “#4” that is same as ‘#2’ except that “s1+s2” are for any UE (for both baseline and “Rel-18 enhancement”). We do not think optional UE features (that also cannot be used for CORESET#0) should be part of the baseline. Also, there are only 2 CORESETs that UEs support in FR1. From a NW perspective and for the small BWs associated with LTE-NR coexistence, there is often only CORESET#0 for Rel-17 UEs as there is no reason or BW left to have CORESET#1. |
| Huawei/Hisi 2 | @Ericsson On ‘legacy NR UE”/” any NR UE”, the Scenarios in the agreement are assumed for R18 PDCCH (i.e., overlapping with CRS) as per our understanding. Do you mean the legacy PDCCH/CORESET without overlapping with CRS is also interpreted as subject to R18 UE? If so, is one R18 UE assumed to have multiple CORESET configurations (PDCCH w/ collision and PDCCH w/o collision) in the evaluation? Then what is the methodology to evaluate the hybrid CORESETs in LLS? Maybe I misunderstood something, and your further clarifications are appreciated. @Moderator: changing ‘s2: NR PDCCH (any NR UE)’ to ‘s2: legacy NR UE’ is fine to us. Agreement For evaluations consider the following list of scenarios: - Scenario#1A: 1 symbol CORESET, overlapped with CRS – Option 2 only - Scenario#2: 2 symbols CORESET, including 1 overlapping symbol and 1 clean symbol – Option 1-1/1-2/2 - Scenario#3: 3 symbols CORESET, including 1 overlapping symbol and 2 clean symbols – Option 1-1/1-2/2. On proposed updates for bullets #1 and #2 in last column, for both bullet 1 and bullet 2, ‘when supporting’ is fine to us. Try to change as follows (@Moderator I guess Ericsson’s concern is the wording ‘for’ is not accurate): - NR PDCCH capacity when supporting due to improvement of Rel-18 enhancement compared to and baseline, and relative gain - NR PDCCH coverage/outage when supporting due to Rel-18 enhancement, and baseline, and relative performance On “only s2 for any NR UEs not supporting FG22-12”, the intention of adding this sub-scenario is to simulate the UE supporting R16 capability. If we still keep “only s2 for legacy NR UEs not supporting FG22-12” in the assumption of baseline-2, then what is the difference with baselin-1 (which already reflects R15 capability)? |
| MediaTek | Regarding Huawei/Hisi’s confusion on “only s2 for legacy NR UEs not supporting FG22-12” and ‘legacy NR UE”/” any NR UE”, we originally share the same confusion on how to reflect it in LLS simulation. After Qualcomm/ vivo/Spreadtrum/ZTE clarification, maybe vivo’s suggestion on perfect scheduling can be an easy setup and companies can perform some theoretical scaling (something like Huawei proposes in section 2.3?) when coexistence is considered. Note sure this is a common understanding though. @Moderator: For the reply, “the current comparison keeps the amount of system resources for PDCCH somewhat similar between baseline and enhancement” I thought baseline-2 has much similar PDCCH resource than baseline-1 when comparing to Rel-18 enhancement: 1 puncture symbol+1 clean symbol? I might misunderstood something? Regardless of whether there is a similar PDCCH resource between baseline and enhancement, we agree with Moderator that keeping minimum set of reporting is better, which is why we can accept adding FG 22-12 UE as optional baselines and optional comparison instead of insisting on mandatory baseline. In that case, for companies which are interested in those optional cases, there are some common understanding configurations for evaluation (Based on our observation, there are at least 2 companies are interested in evaluating baseline-2 in scenarios #1A and #2.) For the companies which are not interested in those optional case, then they can simply ignore the optional one and report minimum simulation results. For the study purpose, it is important to cap the simulation efforts but it is also important to have broad enough simulation scenarios and investigation to draw the conclusion. To us, making some comparison as optional can achieve good balance. |
| Vivo2 | Regarding HW/Hisi’s comments on ‘legacy NR UE’ vs ‘any NR UE’ for ‘R18 enhancement’ column: from the system perspective, ‘any NR UE’ is more reasonable, it is up to NW to schedule R18 UE on clean symbol. But from the evaluation perspective, there is no difference between R18 UE and legacy UE on the clean symbol, it would be simpler to derive the total PDCCH capacity if only legacy UE is considered on S2. Instead, if both R18 UE and legacy UE can be scheduled on CORESET on S2, we need to further report/consider the ratios of R18 UE using CORESET on S1+S2 and R18 UE using CORESET on S2 to derive a capacity for the hybrid CORESET configuration. In that sense, we are ok with HW/HiSi’s modifications for simplicity. Regarding the evaluation of UE with FG22-12: We prefer to remove ‘only s2 for legacy NR UEs not supporting FG22-12’. In the proposal, #3 is also for evaluating the gains of R18 over legacy FG 22-12, while the differences between #1A and 3 for R18 enhancement column are the R18 CORERSET length and UE type. It is not clear why ‘only s2 for legacy NR UEs not supporting FG22-12’ is additionally considered in #1A, the assumption of UE types should be aligned. #1A; s0: reserved for LTE s1: reserved for LTE s2: NR PDCCH (any legacy NR UE) s3-13: reserved for NR PDSCH baseline-2 (optional): s0: reserved for LTE s1: reserved for LTE s2+s3: NR PDCCH (NR UE FG 22-12), only s2 for legacy NR UEs not supporting FG22-12. #3; s0: reserved for LTE s1: reserved for LTE s2-3: NR PDCCH (NR UE FG22-12) s4-13: reserved for NR PDSCH. |
| Mod2 | Dear all, thanks for the comments so far and thanks for your understanding/compromise given our constraints of just an email discussion. @HW, not sure whether you misunderstood my previous comment but since changing “any” to “legacy” and the other modification proposals are controversial, its not easy to implement it straightaway @MTK, thank you for the understanding in order to progress the discussion @Samsung, your requirement on companies to report CORESET#0 handling is reflected already. I think expanding to more scenarios is not desirable, note that current proposal on the table allows us to evaluate scheme 2 (s2 not reserved) with other schemes. @Vivo, pls. see similar comment from HW/Ericsson on removing “only s2..” so I think people understand each other’s views and making further changes will not be easy As I mentioned above, please consider a revised proposal with the following revision and pls. comment if you absolutely cannot live with it 1. NR PDCCH capacity for due to improvement of Rel-18 enhancement compared to and baseline, and relative gain” |

Proposal#2:

The following results collection template is used (4 such tables- one each for scenario #1A, scenario #1A baseline-2, scenario-2 and scenario-3)

| source# | assumptions (column#4, bullet#5 in Proposal#1) | KPI (column#4, bullet#1 in Proposal#1) | KPI (column#4, bullet#2 in Proposal#1) | KPI (column#4, bullet#3 in Proposal#1) | KPI (column#4, bullet#4 in Proposal#1) |
| --- | --- | --- | --- | --- | --- |
| source-1 |  |  |  |  |  |
| source-1 |  |  |  |  |  |
| source-2 |  |  |  |  |  |

| Company | Comments |
| --- | --- |
| Huawei/Hisi | Basically fine with the proposal. Two minor comments: 1) duplicated ‘source-1’ in the 1st column. 2) It looks “column#4, bullet#4 in Proposal#1 (Whether and how to achieve coexistence with legacy UEs)” is rather part of the Assumption than a KPI? |
| Ericsson2 | We propose following updates Proposal#2-updated: Companies are encouraged to use the following results collection template is used (e.g., 4 such tables- one each for scenario #1A, scenario #1A baseline-2, scenario-2 and scenario-3) Note: Link-level performance results (e.g., those used for estimating PDCCH capacity) can be provided in a separate table source#; assumptions (column#4, bullet#5 in Proposal#1); KPI1, e.g., column#4, bullet#1 in Proposal#1; KPI2, e.g., column#4, bullet#2 in Proposal#1); …. Baseline; Rel18 enhancement; gain. source-1. source-1. source-2. The ‘e.g.’ for KP!s is to align with column 4 of Proposal 1 (and previous agreements). Then regarding capturing of link-level results (i.e., “@Ericsson, ZTE: PDCCH capacity (as part of column #4) will be summarized in template, SNR@1%BLER is not an agreed KPI yet, I am not sure that it will be an easy discussion to agree on a common set of baseline and enhancement schemes for capturing such results.”) our intention was not to trigger discussion on additional baseline etc. ‘PDCCH capacity’ would be derived from link-level performance for different options (e.g. 1-1,1-2,2) according to agreed assumptions. Our suggestion was about summarizing such performance results. However, given moderator comments, we would be ok if following note is added for now - “Note: Link-level performance results (e.g., those used for estimating PDCCH capacity) can be provided in a separate table'”. |
| Qualcomm | Agree with Ericsson. |
| Mod | I can add the note proposed by Ericsson and add e.g. @HW: duplicated source-1 is intentional if source-1 contributes results with 2 different sets of assumptions - sorry I should have clarified, column#4, bullet#4 in Proposal#1 was agreed as KPI so I left it there. |
| Samsung | Fine with the proposal. When capturing any results, the underlying assumptions/implications on the NW operation should also be captured as the objective is to potentially benefit the NW. |
| Huawei/Hisi 2 | Thanks @Moderator and we are OK with the proposal. We are also fine with Ericsson’s updates. |
| MediaTek | We also agree with Ericsson’s proposal. Just one clarification question on the note: Note: Link-level performance results (e.g., those used for estimating PDCCH capacity) can be provided in a separate table “those used for estimating PDCCH capacity” means something like BLER information or? |
| Vivo2 | We are ok with Ericsson’s proposal with further clarifications on the note. Our understanding is that ‘Link-level performance results (e.g., those used for estimating PDCCH capacity) ‘are the SNR required for each AL value to achieve the target BLER. |
| ZTE | We support the proposal from Ericsson. Note that, the KPIs agreed are only for SLS while it should be clear that we should also do LLS and it’s better to report the related LLS results, i.e., SNR@1%BLER. A simple template table for LLS is helpful. |

## Update-2

Dear all, just for clarity pls. find the latest draft of Proposal#1 with a small change to bullet-1 of the KPIs. Pls. comment if you cannot live with it.

Proposal#1:

| Scenario | Baseline | Rel-18 enhancement | Companies to report one or more of following (examples): |
| --- | --- | --- | --- |
| #1A | s0: reserved for LTE s1: reserved for LTE s2: NR PDCCH (any legacy NR UE) s3-13: reserved for NR PDSCH baseline-2 (optional): s0: reserved for LTE s1: reserved for LTE s2+s3: NR PDCCH (NR UE FG 22-12), only s2 for legacy NR UEs not supporting FG22-12 | s0: reserved for LTE s1: NR PDCCH (only Rel18 UE) s2: NR PDCCH (any NR UE) s3-13: reserved for NR PDSCH for baseline-2: s0: reserved for LTE s1: NR PDCCH (only Rel18 UE) s2+s3: NR PDCCH (NR UE FG 22-12), only s2 for any NR UEs not supporting FG22-12 | 1. NR PDCCH capacity for due to improvement of Rel-18 enhancement compared to and baseline, and relative gain 2. NR PDCCH coverage/outage due to Rel-18 enhancement 3. Potential degradation of LTE due to Rel-18 enhancement compared to baseline 4. Whether and how to achieve coexistence with legacy UEs 5. Assumptions on: Fraction of LTE UEs, Receiver for PDCCH and DMRS (pls. note baseline receiver assumptions from RAN1#109-e agreements), capacity calculation methodology if LTE bandwidth<NR bandwidth, CORESET configurations and blocking between legacy NR PDCCH and Rel18 enhancement PDCCH, CORESET#0 handling |
| #2 | s0: reserved for LTE s1: reserved for LTE s2: NR PDCCH (any legacy NR UE) s3-13: reserved for NR PDSCH | s0: reserved for LTE s1+s2: NR PDCCH (only Rel18 UE) s2: NR PDCCH (any NR UE) s3-13: reserved for NR PDSCH | 1. NR PDCCH capacity for due to improvement of Rel-18 enhancement compared to and baseline, and relative gain 2. NR PDCCH coverage/outage due to Rel-18 enhancement 3. Potential degradation of LTE due to Rel-18 enhancement compared to baseline 4. Whether and how to achieve coexistence with legacy UEs 5. Assumptions on: Fraction of LTE UEs, Receiver for PDCCH and DMRS (pls. note baseline receiver assumptions from RAN1#109-e agreements), capacity calculation methodology if LTE bandwidth<NR bandwidth, CORESET configurations and blocking between legacy NR PDCCH and Rel18 enhancement PDCCH, CORESET#0 handling |
| #3 | s0: reserved for LTE s1: reserved for LTE s2-3: NR PDCCH (NR UE FG22-12) s4-13: reserved for NR PDSCH | s0: reserved for LTE s1+s2+s3: NR PDCCH (only Rel18 UE) s2+s3: NR PDCCH (NR UE FG22-12) s4-13: reserved for NR PDSCH | 1. NR PDCCH capacity for due to improvement of Rel-18 enhancement compared to and baseline, and relative gain 2. NR PDCCH coverage/outage due to Rel-18 enhancement 3. Potential degradation of LTE due to Rel-18 enhancement compared to baseline 4. Whether and how to achieve coexistence with legacy UEs 5. Assumptions on: Fraction of LTE UEs, Receiver for PDCCH and DMRS (pls. note baseline receiver assumptions from RAN1#109-e agreements), capacity calculation methodology if LTE bandwidth<NR bandwidth, CORESET configurations and blocking between legacy NR PDCCH and Rel18 enhancement PDCCH, CORESET#0 handling |

| Company | Comments |
| --- | --- |
| Huawei/Hisi | For ‘legacy vs any’ point, I can understand the coexisting of hybrid CORESETs in the real network, but also need to point out that the hybrid case should not be mandatory in evaluation. So try to change it to ‘any/legacy’ as a middle ground, which means companies can choose either or both in the simulations by willingness. For ‘only s2 for legacy NR UEs not supporting FG22-12’ point, we are not fully convinced on the reason to keep it there as it is redundant with baselin-1, but as this case is optional anyway, we can live with it for sake of progress. We can live with the current proposal on other points we had comments in previous rounds for sake of progress. Scenario; Rel-18 enhancement. #1A; s0: reserved for LTE s1: NR PDCCH (only Rel18 UE) s2: NR PDCCH (any/legacy NR UE) s3-13: reserved for NR PDSCH for baseline-2: s0: reserved for LTE s1: NR PDCCH (only Rel18 UE) s2+s3: NR PDCCH (NR UE FG 22-12), only s2 for any NR UEs not supporting FG22-12. #2; s0: reserved for LTE s1+s2: NR PDCCH (only Rel18 UE) s2: NR PDCCH (any/legacy NR UE) s3-13: reserved for NR PDSCH. |
| Vivo3 | We are ok with the changes from HW/Hisi to add ‘legacy’ in column3, companies can report which UE type is considered in the evaluation. For our previous comment on aligning UE types between baseline-2 of #1 and baseline of #3, we are not still clear why different UE types are considered for S2 in #1A and #3, but we can live with the latest proposal. |
| Xiaomi | Baseline-2 of “Rel-18 enhancement” column is not clear to us: - for baseline-2: - s0: reserved for LTE - s1: NR PDCCH (only Rel18 UE) - s2+s3: NR PDCCH (NR UE FG 22-12), only s2 for any NR UEs not supporting FG22-12 If s2 is also for Rel-18 UEs, which symbols occupied by the CORESET configured for Rel-18 UEs? It seems that there are two interpretations: - Interpretation#1: There are two CORESETs while one occupies s1 and another one occupies s2; - Interpretation#2: Only one CORESET occupies s1+s2; Our understanding is that interpretation#1 is the case. If the NR UEs don’t support FG22-12, interpretation#2 corresponds to the same case as scenario#2. If this is the common understanding, we can accept it. According to the simulation scenarios, although enough simulation scenarios can provide more investigations to draw the conclusion, the results comparison is not easy if too many configurations depend on company reporting. Maybe defining some basic configurations and optional configurations can help solve the above problem. Companies will report the evaluation results of basic configurations, while the evaluation results of optional configurations depend on the selection of companies. Regarding the “blocking between legacy NR PDCCH and Rel-18 enhancement PDCCH” in item#5 of last column, we have the same view with Qualcomm, i.e., the PDCCH blocking due to frequency overlapping also exists in exiting schemes, there is no need for further discussion. |
| Mod | @HW, Vivo: thanks, lets try the version from HW on “legacy vs any” issue. and check if any concerns on this. @Xiaomi: my understanding is interpretation #1 |
| Ericsson3 | Thanks for further discussion. Reserving s2 only for legacy is an unrealistic restriction. This is a key issue for evaluation and related to how CORESET0 is handled for Rel18 UEs and associated PDCCH performance for that CORESET. As discussed so far, having two CORESETs: (CORESET X: s2, CORESET Y: s1 for scenario #1A), (CORESET X: s2, CORESET Y: s1+s2 for scenario #2) is common expected configuration to be used. We do not see point of evaluating a scenario where s2 is not available for Rel18 UEs and do not agree with proposed update (“any/legacy”) from HW. The current motivation mentioned by vivo, HW for restricting s2 only for legacy UEs seems to be ‘ease of evaluation’(something like - evaluation of PDCCH capacity for set up where UE supports two CORESETs is complicated). For #1A, it is straightforward as the CORESETs are separate (capacity according to corresponding link-level performance is computed for s1 and s2 and added). For #2, #3 some scaling is required to estimate aggregate PDCCH capacity and this companies can report how it is done. To address this perhaps following Note can be added – “For computing PDCCH capacity, companies to report how aggregate PDCCH capacity across all CORESETs configured for the UE is computed including any simplifying assumptions made (e.g. assuming only single CORESET to determine aggregate PDCCH capacity even when multiple CORESETs can be used for the UE in real system)”. In summary, we are fine with current Proposal#1 (i.e., current update from in v24) and do not agree with proposed update from HW. As a compromise to address the concern on ‘ease of evaluation’ from HW/vivo we are OK to add following Note to the proposal#1 of v24 - “Note: For PDCCH capacity, companies to report how aggregate PDCCH capacity across all CORESETs configured for the UE is computed including any simplifying assumptions made (e.g., assuming single CORESET to determine aggregate PDCCH capacity even when multiple CORESETs can be used for the UE in real system)” |
| Samsung | Handling of CORESET#0 is important but does not directly relate to proposed scenarios #2 and #3. In RAN1#109-e it was extensively discussed/agreed to evaluate “Option 2”. That allows legacy operation (super-position – gNB can skip transmission of DM-RS colliding with CRS). That has been possible since Rel-15 and there are deployments with 4 CRS ports and NR PDCCH on s1 and s2. That is not reflected on any proposed scenarios for evaluation when, at least in our perspective, it is the baseline. We cannot agree with #2 and #3. At this stage, as further progress does not seem possible, companies may of course evaluate #2 and #3 but we will evaluate the Rel-18 proposals relative to what Rel-17 can do by having s1 and s2 available for NR PDCCH by seamlessly puncturing DM-RS REs that overlap with CRS (1/6 of DM-RS REs). |
| Huawei/Hisi 2 | Regarding Ericsson’s concern on CORESET#0 of R18 UE (which is also legacy PDCCH w/o overlapping with CRS symbols), we may change it to ‘legacy NR PDCCH (any NR UE)’ to be clear. From the simulation perspective, we believe the performance for the one symbol CORESET is the same irrespective it is for legacy NR UE or R18 NR UE. Under this change (‘NR PDCCH’’Legacy NR PDCCH’), we can also accept the ‘Note:’ by Ericsson. Scenario; Rel-18 enhancement. #1A; s0: reserved for LTE s1: NR PDCCH (only Rel18 UE) s2: Legacy NR PDCCH (any NR UE) s3-13: reserved for NR PDSCH for baseline-2: s0: reserved for LTE s1: NR PDCCH (only Rel18 UE) s2+s3: NR PDCCH (NR UE FG 22-12), only s2 for any NR UEs not supporting FG22-12. #2; s0: reserved for LTE s1+s2: NR PDCCH (only Rel18 UE) s2: Legacy NR PDCCH (any NR UE) s3-13: reserved for NR PDSCH. |
| MediaTek | Let us clarify our previous comment. Our intention is to have some common understanding on optional comparison, including the one we proposed as shown below Scenario; Baseline; Rel-18 enhancement. #1A; baseline-1 s0: reserved for LTE s1: reserved for LTE s2: NR PDCCH (any legacy NR UE) s3-13: reserved for NR PDSCH baseline-2 (optional): s0: reserved for LTE s1: reserved for LTE s2+s3: NR PDCCH (NR UE FG 22-12), only s2 for legacy NR UEs not supporting FG22-12; for baseline-1 and baseline-2: s0: reserved for LTE s1: NR PDCCH (only Rel18 UE) s2: NR PDCCH (any NR UE) s3-13: reserved for NR PDSCH for baseline-2: s0: reserved for LTE s1: NR PDCCH (only Rel18 UE) s2+s3: NR PDCCH (NR UE FG 22-12), only s2 for any NR UEs not supporting FG22-12. Sorry for the confusion. In that case, companies which want to simulate baseline-2 v.s. 1 punc symbol+1 clean symbol will have the common understanding and the minimum reporting simulation results is still the same. To us, it is not desirable to limit the comparison for optional baseline as shown in the Moderator’s proposal for study purpose. |

Proposal#2:

Companies are encouraged to use the following results collection template (e.g., 4 such tables- one each for scenario #1A, scenario #1A baseline-2, scenario-2 and scenario-3)

Note: Link-level performance results (e.g., those used for estimating PDCCH capacity) can be provided in a separate table

| source# | assumptions (column#4, bullet#5 in Proposal#1) | KPI-1 (e.g. column#4, bullet#1 in Proposal#1) | KPI-2 (e.g. column#4, bullet#2 in Proposal#1) | KPI-3 (e.g. column#4, bullet#3 in Proposal#1) | KPI-4 (e.g. column#4, bullet#4 in Proposal#1) |
| --- | --- | --- | --- | --- | --- |
| source-1 |  |  |  |  |  |
| source-1 |  |  |  |  |  |
| source-2 |  |  |  |  |  |

| Company | Comments |
| --- | --- |
| Huawei/Hisi | OK with it. |
| Xiaomi | We are fine with it |

## Possible implementation options:

Possible implementation options that may be agreeable- I believe the focus is primarily on the UE side (e.g., impact to channel estimation, PDCCH rate-matching etc.) but pls. feel free on share your views.

| Company | Comment |
| --- | --- |
| MediaTek | Based on the agreements made in RAN1 #109-e meeting, at least option1-1 and option 1-2 receiver assumptions are clear. For option 2 receiver assumption, it is also clear if UE assumes no puncturing on both PDCCH payload and DMRS, which is agreed as baseline. For the option 2 optional receiver, can proponents clarify on how to handle 1 symbol irregular PDCCH DMRS pattern, especially for scenario #1A? |
| Samsung | Our understanding of option 2 is that for superposition, things are as in Rel-17 (e.g. gNB indicates that CRS is only on s0 but actual CRS uses 4 ports). For option 2, that may also apply for supporting PDCCH (including for CORESET#0) in s1 and s2 for both Rel-17 UEs and Rel-18 UEs (that support PDCCH puncturing due to CRS) – i.e. no need for strict FDM. For puncturing, we understand that an irregular DM-RS pattern can be simulated - whether to have a UE capability to support it is a separate issue. We are OK to simulate option 1-2 (given the outcome from RAN1#109-e) but there is no reason for it – the empty REs from option 1-1 can be utilized through some modest (~1.7 dB) power boosting. No need to define/simulate new PDCCH rate matching and the BLER should be practically same as for option 1-1 with the power boosting. We had the same discussion/conclusion before for other issues, including for TEIs, in RAN1. |
| vivo | We are unsure of the intent of this part. Is this discussion intended to further clarify the implementation of UE in the evaluation, or to analyze potential specification changes? If the former case, we agree with MTK that the company needs to clarify how the estimation will be done based on the irregular DMRS pattern as well as the implementation of receiver supporting super-positioning. If the latter case, according to the discussion and agreements, there is at least one common dominator in all the proposed receiver assumptions that UE will use DMRS/PDCCH on the clean symbol for CE/PDCCH decoding in #2 and #3. |
| Spreadtrum | From our observation, UE receiver behavior is not mandatorily required to match gNB TX side. Thus for Option 1-1 and Option 2, it may still potentially have multiple channel estimation approaches from UE side. The Option 1-2 is very clear with only one receiver choice for UE. So far, we prefer option 1-2 for its simplicity and fixed channel estimation, and reluctant to support those options which will finally depend on the UE implementation with multiple CE choices. But we can open to other options or combined options to adapt to potentially different deployment scenarios only if UE implementation on channel estimation can be fixed from spec. based on legacy UE CE mechanism. |
| Mod | my understanding from the last GTW is that companies wanted an opportunity to further argue their support or concerns for either of the options 1-1, 1-2 or 2. UE implementation concern is one such issue. I think simulation results will play a small role in the conclusion of this WID so my intention here is to provide a forum for such discussion. If companies could succinctly share their support or concerns here it will help folks to prepare for the August meeting. |
| Qualcomm | We agree with Spreadtrum that the Option 1-2 is very clear and preferred solution. As we commented during the meeting, even though companies are now currently saying that Option 1-1 and Option 2 would allow multiple CE choices just by implementations (e.g., baseline receiver and CRS nulling receiver), substantial RAN4/RAN5 work for performance requirements and test cases for the multiple CE choices would be required in this release or in the future releases. In addition, there are a lot of flexibilities in LTE-CRS patterns – carrier frequency, bandwidth, v-shift, MBSFN subframe patterns, and multiple overlapping/non-overlapping LTE CRSs. It is not clear how a UE can implement advanced CE for all these cases. |
| Vivo2 | Thanks to moderator for clarification. As UE vender, we prefer to have a straightforward and unified solution for all cases instead of allowing multiple options for CE/PDCCH decoding especially considering that receiver implementations have a large impact on the performance. Additionally, we also have concerns on the complexity of supporting CE based on irregular DMRS patterns, as each irregular DMRS pattern may require special/specific handling. |
| Xiaomi | We also prefer Option 1-2 as Spreadtrum and Qualcomm have stated. For Samsung’s comment on “the empty REs from option 1-1 can be utilized through some modest (~1.7 dB) power boosting”, we don’t understand where the empty REs come from? Maybe we are missing something, we would appreciate it if it could be clarified. |

## Clarifications on LLS and SLS assumptions and KPIs:

Several questions have been raised that is related to details on how the simulations are to be performed e.g. how to determine PDCCH capacity, how to evaluate legacy NR UEs etc. Pls. use this table to raise questions and exchange views that may not be suitable for discussion under Proposal#1

| Company | Comments |
| --- | --- |
| Huawei/Hisi | Firstly we understand the details are reported by companies and not needed to be aligned in EVM. Then we would like to share our understandings on how to evaluate the following cases. Coexistence of legacy NR PDCCH and R18 PDCCH: For how to calculate the capacity for the FDMed legacy NR PDCCH and R18 PDCCH, an example (for Scenario#2) can be: assuming the payload ratio (i.e., ratio of occupied resources) between R18 UE and legacy UE is X:(1-X), and assuming the occupied BW between R18 UE and legacy UE as Y: (1-Y), then the relation between X and Y can be expressed as an equation 2Y/(1-Y)=X/(1-X), and Y can be calculated as Y=X/(2-X). Thus under a predefined payload ratio of X:(1-X), the capacity gain is derived as Y * capacity gain without considering co-existence with legacy NR (i.e., the capacity gain calculated in the red dashed rectangle). LTE bandwidth<NR bandwidth: Take Scenario#2 for example. For baseline, assuming two parts are configured for legacy NR: part 1 is configured as 2OS CORESET 1 for s0+s1 on non-LTE BW; part 2 is configured as 1 OS CORESET 2 for s2 on NR BW. The capacity for part 1 and part 2 are separately calculated, and the total capacity is the sum of Capacity-part 1 + Capacity-part 2. For R18, also assuming two parts are configured: part 1’ is configured as 3OS CORESET 1’ for s0+s1+s2 on non-LTE BW; part 2’ is configured as 2 OS CORESET 2’ for s1+s2 on LTE BW; the total capacity is the sum of Capacity-part 1’ + Capacity-part 2’. Note the assumption for legacy NR UEs (s0+s1 [+s2]) under LTE bandwidth<NR bandwidth is different from the baseline EVM where only s2 is assumed for legacy NR on the non-LTE BW. We can interpret that the baseline EVM only applies for the part of LTE BW. |
| MediaTek | We have one clarification question for LLS simulation assumption on antenna configurations. In current agreement, we have Correlation; Low. Number of BS antennas; 4 Tx, (M,N,P,Mg,Ng;Mp,Np)= (1,2,2,1,1;1,1), [2 Tx, (M,N,P,Mg,Ng;Mp,Np)= (1,1,2,1,1;1,1).]. However, in TS 38.101-4, low correlation antenna configuration is only defined in B.2.3.1 ULA case where no polarization is considered and there is no description on low correlation configuration in B.2.3.2 cross polarized antenna cases where only medium and high correlation configuration are defined as shown below Correlation Model; 1; 2; . Medium Correlation; 0.3; 0.3; 0.6; 0.2. High Correlation; 0.9; 0.9; 0.9; 0.3. Note 1: Value of α1 applies when more than one pair of cross-polarized antenna elements in first dimension at gNB side. Note 2: Value of α2 applies when more than one pair of cross-polarized antenna elements in second dimension at gNB side. Note 3: Value of β applies when more than one pair of cross-polarized antenna elements at UE side.. If our understanding is correct, should we just assume there is no correlation between the antennas for “low correlation”? |
| Qualcomm | Just to repeat the previous comment: Regarding multiplexing PDCCHs for legacy UEs and Rel-18 UEs, as we have commented, FDM (semi-static FDM via CORESET configurations) is not the only option – rather FDM is less efficient way. For the evaluation purpose of the frequency overlapping case, we agree with vivo/Spreadtrum/ZTE that companies should report how the multiplexing is supposed to be done (ideal scheduling is an example assumption as vivo suggested). The point is that the PDCCH blocking due to frequency overlapping configuration should not be the focus/issue of the current discussion – it exists even today (even within the same CORESET). |
| Mod | @MTK: yes – that would be my understanding |
| Samsung | Regarding the comment by Huawei and considering the spectrum that the operators have available in scenarios of LTE-NR coexistence, the assumption should be that LTE BW = NR BW. Also, if NR BW > LTE BW, the purpose of this study/evaluation is unclear. |
| Mediatek | Thanks to Moderator’s clarification. Thanks to Huawei/Hisi for sharing the understanding on how to calculate PDCCH capacity under coexistence. Based on the analysis, then should we simulate the case when no coexistence is considered (i.e., capacity gain without considering co-existence with legacy NR) then scale the PDCCH capacity with some chosen (X,Y) to reflect the PDCCH capacity with coexistence? We also agree with Samsung that simulating NR BW>LTE BW might complicate the analysis. Some approach like what Huawei/Hisi propose may be needed for evaluating case like NR BW=20MHz and LTE BW=10MHZ. This aspect should be clarified. |
| Vivo2 | As we commented in the previous round, we prefer to assume ideal scheduling for frequency overlapping case. One way to derive the total capacity for frequency overlapping case is to divide the amount of PDCCH resources by a hybrid avg CCE. Taking #2 as example, the average CCE for CORESET#A on S1+S2 and CORESET#B on S2 can be separately calculated from LLS. The ratio of R18 UE and legacy UE are a and b, if ‘only legacy UE’ on S2 as HW suggested, then the hybrid avg CCE for each scheduling for R18 enhancement is: a*avg CCE of CORESET A+b*avg CCE of CORESET B. If ‘any NR UE’ is considered for S2, then ratios of R18 UE in each CORESET should be further considered to derive the hybrid avg CCE. |
| ZTE | We would like to share our views on how to evaluate PDCCH capacity as follows. Please correct us if there is something incorrect. Step1: Perform system level simulation to get geometry results based on the agreed SLS assumptions; Step2: Perform link level simulation to get the BLER-SNR results for PDCCH based on the agreed LLS assumptions for Rel-18 UEs and legacy UEs; Step3: Calculate the probability of each aggregation level targeting 10^-2 BLER based on the simulation results in Step1 and Step2, and then calculate the average number of CCEs for Rel-18 UEs (denoting as X) and legacy UEs (denoting as Y) respectively. Step4: Assuming the configured resources for PDCCH (i.e., RBs across available symbols) for ‘R18 enhancement’ is A times of the ‘baseline’, the capacity gain is alpha*A*Y/X, where alpha is the ratio of the configured resources for Rel-18 UEs among all resources configured for ‘Rel-18 enhancement’. |

# References

[1] RP-213575, New WI: Enhancement of NR Dynamic spectrum sharing (DSS), Ericsson, 3GPP TSG RAN Meeting #94e, Electronic Meeting, Dec. 6 - 17, 2021.

[2] R1-2203137, Discussion on NR PDCCH reception in symbols with LTE CRS REs	Huawei, HiSilicon

[3] R1-2203210	Discussion on NR PDCCH reception for DSS	ZTE

[4] R1-2203344	Discussion on NR PDCCH reception in symbols with LTE CRS Res	Spreadtrum Communications

[5] R1-2203581	Discussion on PDCCH reception on CRS symbol	vivo

[6] R1-2203648	Evaluation of NR PDCCH overlapping with LTE CRS	InterDigital, Inc.

[7] R1-2203834	Discussion on NR PDCCH reception in symbols with LTE CRS REs	xiaomi

[8] R1-2203923	Considerations on PDCCH receptions in symbols with LTE CRS	Samsung

[9] R1-2204024	Discussion on NR PDCCH reception in symbols with LTE CRS REs	OPPO

[10] R1-2204260	Discussion on NR PDCCH reception in symbols with LTE CRS REs	Apple

[11] R1-2204323	Discussion on NR PDCCH reception in symbols with LTE CRS REs	CMCC

[12] R1-2204395	Discussion on NR PDCCH reception in symbols with LTE CRS REs	NTT DOCOMO, INC.

[13] R1-2204630	Discussion on NR PDCCH reception in symbols with LTE CRS REs	LG Electronics

[14] R1-2204709	Discussion on NR PDCCH reception in symbols with LTE CRS REs	MediaTek Inc.

[15] R1-2204815	Discussion on NR PDCCH reception in DSS	Intel Corporation

[16] R1-2204823	NR PDCCH overlapping with LTE CRS	Nokia, Nokia Shanghai Bell

[17] R1-2204885	NR PDCCH reception in symbols with LTE CRS REs	Ericsson

[18] R1-2205049	NR PDCCH reception in symbols with LTE CRS REs	Qualcomm Incorporated

# Appendix (Summary of the agreements)

The agreements made in RAN1#TBD meetings are provided below.