3GPP TSG RAN WG1 #109-e			R1-2203023

e-Meeting, May 9th – 20th, 2022

3GPP TSG-RAN4 WG4 Meeting #102-e	R4-2206981

# Electronic meeting, Febrary 17 – March 1, 2022

# Title:	LS reply on condition of PRS measurement outside MG

Response to:	R1-2112883 (R4-2200051)

Release:	Rel-17

Work Item:	NR_pos_enh-Core

Source:	RAN4

To:	RAN1

Cc:

# Contact Person:

# Name:	Qian Yang

# E-mail Address: qian9.yang@vivo.com

# 1. Overall Description:

RAN4 thanks RAN1 for the LS on the condition of PRS measurement outside MG.

RAN4 has discussed the threshold applied to the Rx timing difference between the serving cell PRS and PRS from a non-serving cell/TRP to determine the applicability of PRS measurements outside MG.

RAN4 has agreed to introduce a UE capability to accommodate different UE implementations of PRS measurements outside MG.

- Introduce UE capability for the maximum Rx timing difference in MG-less PRS measurement

  - Option 1: two values {CP length, 0.5 slot}

  - Other options are not precluded

The threshold corresponds to the maximum Rx timing difference that the UE supports for PRS measurement outside MG.

RAN4 has agreed that the expected Rx timing difference between the PRS from the non-serving cell/TRP and that from the serving cell is determined by the expected RSTD and expected RSTD uncertainty in the assistance data, and any difference in timing between reference cell and the serving cell. The UE is not required to determine whether the threshold condition is satisfied by performing measurements. Therefore, no requirements on the UE to calculate the Rx time difference and/or to compare it against the threshold will be defined.

The details of the UE capability for the maximum Rx timing difference in MG-less PRS measurement will be further discussed as part of Rel-17 RAN4 UE feature list.

2. Actions:

To: RAN WG1

ACTION: 	RAN4 kindly asks RAN1 to take the above RAN4 agreements into account in the future work.

3. Reference:

- R1-2112883, LS on the condition of PRS measurement outside the MG

4. Date of Next TSG RAN WG4 Meetings:

TSG RAN WG4 Meeting #103-e		May 16 – May 27, 2022			Electronic Meeting