3GPP TSG RAN WG1 #109-e			R1-2203033

e-Meeting, May 9th – 20th, 2022

3GPP TSG-RAN WG4 Meeting #102e                	    			       R4-2206788

Electronic Meeting, February 21 – March 3, 2022

Title:	LS on collision handling of concurrent MGs

Response to:

Release:	Rel-17

Work Item:	NR_MG_enh-Core

Source:	RAN4

To:	RAN2

Cc:                              RAN1

Contact Person:

#### Name:	Tsang-wei (Ato), Yu

E-mail Address:	ato.yu (at) mediaTek.com

1. Overall Description

RAN4 has discussed the handling of collisions between concurrent measurement gaps and reached the following agreements

- Introduce a priority rule to resolve collisions between measurement gap occasions

    - In each collision, the UE will perform only measurements associated with the measurement gap with the highest priority

    - The priority of the measurement gap can be RRC configurable

    - In Rel-17, define requirements for the case when different measurement gaps are configured with different priorities (i.e., do not consider equal priorities case)

  - Regarding the number of priority levels, only two levels are needed in the NR_MG_enh WI. However, considering forward compatibility on inter-working with other features (e.g., MUSIM, NTN, Positioning), RAN4 recommends 5 levels. RAN4 kindly requests that at least two priority levels are supported in Rel-17 and leaves the decision to support a higher number of priority levels to RAN2.

2. Actions:

ACTION: 	RAN4 respectfully asks RAN2 to take above information and implement the required RRC signaling for the priority level.

3. References:

4. Date of Next RAN4 Meetings:

RAN4#103e, E-Meeting, 16 May to 27 May 2022