3GPP TSG-RAN WG1 Meeting #110	R1-2205779

# Toulouse, France, Augest 22 – 26, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | xxx | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on A-CSI multiplexing on TBoMS PUSCH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_cov_enh-Core |  |  |  |  |  | Date: |  |  | 2022-08-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - Capture the agreement from RAN1#109-e below. RAN1#109-e Agreement Note: To editors, above agreement on SP/A-CSI multiplexing on a slot of TBoMS with UL-SCH is modified as follows. - SP/A-CSI multiplexing on a slot of TBoMS with UL-SCH is supported. - FFS: which slot of the single TBoMS (and which repetition, in case of TBoMS repetition) is selected for SP/A-CSI multiplexing. - SP/A-CSI multiplexing on TBoMS without UL-SCH is not supported. - A-CSI should be multiplexed on the first slot of TBoMS with UL-SCH based on the agreement from RAN1#107-e and the conclusion from RAN1#107b-e below, where A-CSI is just a type of UCI with aperiodic triggering. RAN1#107-e Agreement - For TBoMS, UCI is multiplexed on the individual overlapping slot for UL transmission in one carrier - FFS: timeline requirements - FFS: details on the calculation of the number of coded modulation symbols per layer for UCI multiplexing on a single TBoMS. - Note: no new UCI multiplexing mechanism other than existing puncturing or rate-matching is introduced for TBoMS in Rel-17. RAN1#107b-e Conclusion Existing rules can be reused for UCI multiplexing on PUSCH in case of TBoMS and UL CA scenario. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update to reflect that: - SP/A-CSI multiplexing on TBoMS without UL-SCH is not supported. - A-CSI is multiplexed on the first slot of TBoMS with UL-SCH. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification for TB processing over multiple slots is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.3, 6.1.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Isolated Impact Analysis: This CR has isolated impact on A-CSI multiplexing onto PUSCH when the PUSCH is of TB processing over multiple slot. The CR is the first CR to introduce and complete such functionality. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 5.2.3	CSI reporting using PUSCH

<Unchanged parts are omitted>

CSI reporting on PUSCH can be multiplexed with uplink data on PUSCH except that semi-persistent CSI reporting on PUSCH activated by a DCI format is not expected to be multiplexed with uplink data on the PUSCH. CSI reporting on PUSCH can also be performed without any multiplexing with uplink data from the UE except that the PUSCH transmission is of TB processing over multiple slots.

<Unchanged parts are omitted>

#### 6.1.2.1	Resource allocation in time domain

<Unchanged parts are omitted>

For TB processing over multiple slots, when a DCI format 0_1 and DCI format 0_2 schedule aperiodic CSI report(s) on PUSCH with transport block by a ‘CSI request’ field on a DCI, the CSI report(s) is transmitted only on the first transmission occaision.

##### 6.1.2.1.1	Determination of the resource allocation table to be used for PUSCH

<Unchanged parts are omitted>