3GPP TSG-RAN WG1 Meeting #110	R1-2205908

# Toulouse, France, August 22nd –26th, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR |  | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on RSTD measurement for 38.214 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-Core |  |  |  |  |  | Date: |  |  | 2022-08-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | (1) According to the agreement made in RAN1#107e meeting, the maximum number of reported RSTD measurements obtained from different DL PRS resources per UE Rx TEG per target TRP is 4. Also in RAN1#109e meeting, up to 32 measurement instances are supported in a single measurement report. In summary, if measurement instance and TEG are supported, the maximum number of reported RSTD measurements obtained from different DL PRS resources per UE Rx TEG per target TRP in a single measurement instance is 4. The maximum number of reported RSTD measurements obtained from different DL PRS resources per UE Rx TEG per target TRP in a single measurement report can be up to 4*32. There are two paragraphs describing the maximum number of reported RSTD measurements in the current TS 38.214. The motivation was for Rel-16 and Rel-17 respectively. However, the description of the first paragraph is unclear whether it is also applicable for the case of multiple measurement instances in a single report. Hence, we suggest to explicitly indicate that the maximum number is applicable for a single measurement instance in order to make it clear that the location information/elements included in one measurement instance for Rel-17 is equivalent/similar to that included in one measurement report for Rel-16. Agreement in RAN1#107-e: - The maximum number of reported RSTD measurements obtained from different DL PRS resources per UE Rx TEG per target TRP is 4. - The target TRP can be the same as the RSTD reference TRP or a neighbor TRP - Note: The number of DL PRS resources per target TRP in a measurement report is still limited to 4 as in Rel-16. - The maximum number of reported RTOA measurements obtained from different UL SRS resources for positioning per TRP Rx TEG for a UE is 4. - The maximum number of reported UE Rx-Tx time difference measurements obtained from different DL PRS resources per UE Rx TEG for a TRP is 4. - The maximum number of reported gNB Rx-Tx time difference measurements obtained from different UL SRS resources per TRP Rx TEG for a UE is 4. - The maximum number of reported UE Rx-Tx time difference measurements obtained from different DL PRS resources per UE RxTx TEG for a TRP is 4. - The maximum number of reported gNB Rx-Tx time difference measurements obtained from different UL SRS resources per TRP RxTx TEG for a UE is 4. - Signaling details are left to RAN2 and RAN3 Agreement in RAN1#109-e: - Support up to 32 measurement instances in a single measurement report. - Inform RAN2/RAN3 on RAN1’s decision (2) DL RSTD is defined as the relative timing difference between TRPs. Therefore, when it comes to “dl-PRS-ID” (TRP ID), it is better decribed as “per pair of dl-PRS-ID” for DL RSTD measurements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The maximum number of reported RSTD measurements is defined per measurement instance. - Use “per pair of dl-PRS-ID” to replace “dl-PRS-ID” for DL RSTD measurements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification is not aligned with the agreements and may cause confusions, and the usage of measurement instance is not fully captured. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.6.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: This is to correct the specification to align the previous agreement. No isolated impact is expected. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

5.1.6.5 PRS reception procedure

<Unrelated part omitted>

The UE may be configured to measure and report, subject to UE capability, up to 4 DL RSTD measurements per pair of dl-PRS-ID with each measurement between a different pair of DL PRS resources or DL PRS resource sets within the DL PRS configured for those dl-PRS-ID. The up to 4 measurements being performed on the same pair of dl-PRS-ID and all DL RSTD measurements in the same measurement instance of the same report use a single reference timing.

<Unrelated part omitted>

The UE may be configured to report one or more measurement instances, each with its own timestamp, on DL RSTD, DL PRS-RSRP, and/or UE Rx-Tx time difference measurements, in a single measurement report.

Timing Error Group(s) (TEG(s)) at UE side are defined:

-	UE Rx TEG is associated with one or more DL measurements, which have the Rx timing error difference within a certain margin.

-	UE RxTx TEG is associated with one or more UE Rx-Tx time difference measurements, which have the 'Rx timing errors+Tx timing errors' difference within a certain margin.

The UE may be configured to report, subject to UE capability, via high layer parameter [UERxTEG-ID-Request_DL-TDOA], the association information of DL RSTD measurement(s) with UE Rx TEG(s) via higher layer parameter [ueRxTEG-ID] when the UE reports the DL RSTD measurement(s). The UE may report up to 4 RSTD measurements associated with different DL PRS resources per UE Rx TEG per pair of dl-PRS-ID.